3GPP TSG-SA WG6 Meeting #62	S6-243575

Maastricht, Netherlands 19th – 23rd August 2024	(revision of S6a240299, 3344, merged 3145)

Source:	Nokia, Samsung?

Title:	A-DACM architecture update

Spec:	3GPP TR 23.700-21 v1.1.0

Agenda item:	8.4

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution provides a proposal for A-DACM architecture update.

2. Reason for Change

In TR 23.700-21 v1.1.0, clause 6.2.1, the following EN is to be resolved:

Editor's Note: How VAL client(s) access the services of A-DACM function is FFS.

The following options of access are possible:

- The VAL client can use a corresponding SEAL client to access the A-DACM service which is compliant with the architecture in TS 23.434. This limits the VAL client to access only the services/APIs exposed on SEAL-UU reference point. The services/APIs exposed over SEAL-UU reference point does not include all the services/APIs exposed over SEAL-S reference point.

- The VAL client (acting as API invoker) can directly invoke the service APIs of A-DACM (acting as AEF) which is compliant with CAPIF as specified in TS 23.222. This enables VAL client to consume the services/APIs exposed on SEAL-VC reference point. The services/APIs exposed over SEAL-VC is the same as the services/APIs exposed over SEAL-S reference point.

Both approaches are to be captured in the TR.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 v1.1.0.

* * * First Change * * * *

## 6.2	Option #2: A-DACM architecture to support Metaverse services

### 6.2.1	Application enablement architecture

Figure 6.2.1-1 illustrates the A-DACM architecture to support Metaverse services using SEAL architecture specified in 3GPP TS 23.434 [8].

Figure 6.2.1-1: A-DACM architecture to support Metaverse services using SEAL architecture

The SEAL architecture includes A-DACM client and server which supports the VAL layer. The A-DACM server interacts with the VAL server(s) over the A-DACM-S reference point. The A-DACM client interacts with the VAL client(s) over the A-DACM-C reference point. The interactions between A-DACM client and A-DACM server is ove the A-DACM-UU reference point. The A-DACM server may utilize the 5GC services over N33 reference point. The A-DACM client may be located within the VAL client(s) in which case the A-DACM-C reference point will be internal to VAL client(s). The service APIs offered by A-DACM server on A-DACM-UU and A-DACM-S to the VAL layer functions (via A-DACM client) follow the service-based architecture as shown in the figure 6.2.1-3.

Figure 6.2.1-2 illustrates the A-DACM architecture to support Metaverse services using CAPIF architecture specified in 3GPP TS 23.222 [14].

Figure 6.2.1-2: A-DACM architecture to support Metaverse services using CAPIF architecture

The SEAL architecture includes A-DACM server which supports the VAL layer. The VAL server(s) (acting as CAPIF’s API invoker) interact with the A-DACM server (acting as CAPIF’s AEF) over the A-DACM-S reference point (compliant with CAPIF-2/2e). The VAL client(s) (acting as CAPIF’s API invoker) interact with the A-DACM server (acting as CAPIF’s AEF) over the A-DACM-VC reference point (compliant with CAPIF-2/2e). The service APIs offered by A-DACM server on A-DACM-VC and A-DACM-S to the VAL layer functions follow the service-based architecture as shown in the figure 6.2.1-3. The procedures of CAPIF as specified in 3GPP TS 23.222 [14] apply for the interactions between VAL layer and A-DACM server. The A-DACM server may utilize the 5GC services over N33 reference point.

Figure 6.2.1-3 illustrates the service-based SEAL architecture to support Metaverse services.

Figure 6.2.1-3: Service-based SEAL architecture to support Metaverse services

The A-DACM server exposes the APIs over the Sadacm interface to the VAL layer. The VAL functions (VAL clients and servers) and the A-DACM client (serving a VAL client) consume the services by interacting on the Sadacm interface of the A-DACM server. The CAPIF can be used by VAL functions to discover the services of the A-DACM server.

NOTE:	The A-DACM service can be realized as a new SEAL service or part of existing SEAL service.