3GPP TSG-SA WG6 Meeting #62	S6-243641

Maastricht, The Netherlands, 19th – 23rd August 2024	(revision of S6-243302)

Source:	Ericsson

Title:	AIML Services Lifecycle Operations Control and Management

Spec:	3GPP TS 23.482 V0.1.0

Agenda item:	9.10

Document for:	Approval

Contact:	Ashish S Sharma (ashish.sanjay.sharma@ericsson.com)

1. Introduction

Adding AIML Services Lifecycle Operations Control and Management procedure.

2. Reason for Change

3GPP TR 23.700-82 Solution#11 is recommended for normative work for the AIML Enablement service lifecycle management. The procedure enables management of the service lifecycle with service operation modes like pause, continue etc. The service operation mode provides better resource optimization. For example in model training service, the training can be configured to stop automatically if it is using maximum memory or cpu. By using service operation mode, val server or aiml server can pause the aiml service and perform some action related to the service and again resume or continue the aiml service. For example, the val server can pause the model training, load new data set, and again resume the model training service to train on new dataset. There are cases where the model saturates or it does not improve the model performance, in such situations, the val server can stop the aiml service, which saves compute and associated costs. Sometimes it is important to pause the model training and do some hyperparameter optimization and then resume the model training. Similarly in FL model training, the VAL server may pause the training of devices which are in poor network coverage. There are numerous examples and above are only a few examples to describe how managing the aiml service life cycle is critical to aiml services.

Since the AIML service can be compute-heavy and due to resource constraints, it is important to consider the efficient usage of the resources. Therefore the solution provides the AIML service operation mode configurations to enable the conditional triggering and/or scheduled triggering of the AIML service operation modes. Also, the heterogeneity of AIML clients like tiny IoT devices which are low in resources(like network, compute, power) may require the network-controlled conditional triggering for efficient usage of client resources.

Also, the AIML service is dynamic. For example, the participating device in model training can go offline anytime(like the user shutdowns the application) or the device in poor network coverage. Therefore the solution provides the configuration of event-based state transition reporting of the current status of the AIML service operation. It can also be configured for periodic status reporting of the AIML service operation.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.1.0.

* * * First Change * * * *

## 8.x	AIML service lifecycle management procedure

### 8.x.1	General

The lifecycle management of the AIML services is an essential requirement for the applications to manage the AIML services like model training, inference, discovery etc. An AIML service (as defined in TR) is equivalent to an AIMLE service and assists in performing or enabling one or more AIML operations.

### 8.x.2	AIML service lifecycle management procedure

Figure 8.x.1-1: AIML service lifecycle management procedure

- The VAL server sends AIML service lifecycle management request to the AIML Enablement server as per the Table 8.x.3.1-1.

- Based on the request, If AIML service operation mode is not present then the AIMLE determines the service operation mode for the AIML service operation lifeycle. If AIML client identifier is not present then the AIMLE server uses the procedure X for AIML service operation like client discovery/selection, otherwise uses the AIML client identifiers for model training using procedure Y and sends the AIML Enablement client service operation request to the AIML Enablement client(s) as per Table 8.x.3.2-1.

If the AIML service operation mode in the request is start or continue then the AIML enablement server can, monitor the AIML client(s) QoS using NEF or NRM services, If the AIML service operation mode is pause or finish, then the AIML Enablement server can stop monitoring the QoS of AIML clients.

3.	The AIML Enablement client on receiving the AIML service operation mode performs the service operation mode for the AIML service operation and sends AIML Enablement client sends a response indicating the success or failure of the AIML Enablement client service operation.

4.	The AIML Enablement server provides the AIML service lifecycle management response to the VAL server. The message includes AIML service operation ID and the status of the AIML service operation.

5.	The VAL server sends AIML service lifecycle management update request to the AIML Enablement server with the AIML service operation ID and update information.

6.	The AIML Enablement server receives the request and performs steps 2 and step 3 The AIML Enablement server provides the AIML service lifecycle management update response to the VAL server. The message includes the result of the update and the AIML service operation ID.

Editor’s Note: More information related to the procedure, information flow will be added is FFS.

* * * Next change * * * *

### 8.x.3	Information flows

#### 8.x.3.1	AIML service lifecycle management request/update request

Table 8.x.3.1-1 shows the request sent by an AIML service consumer(e.g. VAL server) to an AIML Enablement server for the AIML service lifecycle management request /update request

Table 8.x.3.1-1: AIML service lifecycle management request/update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g. AIML service consumer). |
| VAL service identifier | O | An identifier for the VAL service associated with the requestor. |
| AIMLE client identifier | O | Indicates the AIML enablement client identity |
| AIML service operation identifier | O | Indicates the existing AIML service operation identifier like AIML model training service |
| AIML service operation mode | O See NOTE 1 | Indicates the required AIMLE service operation modes like start, continue, pause, finish.The pause operation mode defines the temporary stop of the AIML service.. The continue operation mode defines the resuming of the paused operation mode. The finish mode is defined to stop the AIML operation. |
| AIML service operation information | O | Indicates AIML service operation information(.e.g model training parameters like AIML service model container or URI of the model to fetch the model from a repository, AIML service aggregator URI to send model updates, AIML service operation optimization assistance like maximum convergence time |
| NOTE 1: This IE is applicable when VAL server is using explicitly AIMLE model training service idenitified by AIML service operation identifier. |  |  |

#### 8.x.3.2	AIML service lifecycle management response/update response

Table 8.x.3.2-1 shows the request sent by an AIML Enablement server to an AIML service consumer(e.g. VAL server) for the AIML service lifecycle management response or update response.

Table 8.x.3.1-1: AIML service lifecycle management response /update response

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g. AIML service consumer). |
| VAL service identifier | O | An identifier for the VAL service associated with the requestor. |
| AIML service operation ID | M | An identifier to identify the AIML service operation ID |
| AIML service operation mode status | M | Indicates the current state of AIMLE service operation. Possible values start, stop, continue, pause |

#### 8.x.3.3	AIML Enablement client service operation request

Table 8.x.3.3-1 shows the request sent by an AIML Enablement server to an AIML Enablement client for the AIML Enablement client service operation request.

Table 8.x.3.3-1: AIML Enablement client service operation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g. AIML service consumer). |
| VAL service identifier | O | An identifier for the VAL service associated with the requestor. |
| AIMLE client identifier | M | Indicates the AIML enablement client identity |
| AIML service operation ID | M | An identifier to identify the AIML service operation ID |
| AIML service operation mode | O | Indicates the required AIMLE service operation modes like start, continue, pause, finish. |

#### 8.x.3.4	AIML Enablement client service operation response

Table 8.x.3.4-1 shows the request sent by an AIML Enablement client to an AIML Enablement server for the AIML Enablement client service operation response or update response.

Table 8.x.3.4-1: AIML Enablement client service operation response

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g. AIML service consumer). |
| VAL service identifier | O | An identifier for the VAL service associated with the requestor. |
| AIML service operation ID | M | An identifier to identify the AIML service operation ID |
| AIML service operation mode status | M | Indicates the current state of AIMLE service operation. Possible values start, stop, continue, pause |

* * * End of changes * * * *