3GPP TSG-SA WG6 Meeting #62	S6-243653

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243564, 3248)

Source:	Huawei, Hisilicon

Title:	Solution on Satellite edge computing EES and gNB onboard

Spec:	3GPP TR 23.700-01 v1.1.0

Agenda item:	8.5

Document for:	Approval

Contact:	Yajie Hu (Huyajie1@huawei.com)

1. Introduction

This paper proposes a solution on Satellite edge computing EES and RAN onboard.

2. Reason for Change

In Key issue#5, it is considered that UPF and gNB are deployed on the NGSO, in such case the EES and EAS can also considered to be deployed on the NGSO along with the UPF and gNB.

Thus the issue may happen that, the UE serving satellite is SAT ID#1, which is the gNB related Satellite ID connected by the UE. However the EES#2 may be identified for the UE when SAT ID#1 and SAT ID#2 can both provide service to the UE based on UE current location. Thus the UE may not be able to obtain optimal service via RAN#1-UPF#1-EES#2.

Thus how to optimize the service provisioning procedure to support the EAS and EES onboard should be studied.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 v1.1.0.

* * * First Change * * * *

7.2.1.1.2	Further consideration for service provisioning

An external server responsible for providing satellite availability information for each satellite based on real-time satellite orbit information (e.g. eccentricity, inclination, true anomaly) may exist (e.g. see 3GPP TS 23.501 [5], Annex Q). During service provisioning, the EEC can provide needed satellite capability information (e.g. satellite frequency bands) to the ECS and the ECS will be responsible to monitor UE location and UE moving prediction, and query the external server(s) to get a list of satellites (each identified by satellite ID) that can service the list of waypoints of the UE, then determine a suitable EES with a matching satellite ID to serve the UE with longest service time in service provisioning. Once the ECS identifies the suitable EES, it responds/notifies the EEC with the EES information. In addition, the corresponding ECS selected satellite ID is sent to the EEC.

Without external server, the EEC sends service provisioning request message, the request message contains the UE serving Satellite ID which is the Satellite ID connected by the UE, then the ECS can determine EES based on the UE serving Satellite ID and EES Satellite ID. Then the ECS sends the service provisioning response message to the EEC contains the identified EES. Alternatively, in the service provisioning procedure, the EEC sends service provisioning request message to the ECS as defined in 3GPP TS 23.558 [10] clause 8.3.3.2.2, then the ECS sends the service provisioning response message to the EEC contains list of EES along with EES Satellite ID. Upone receiving the response message, the EEC can determine EES based on the UE serving Satellite ID and EES Satellite ID.

Editor’s Note: how can UE obtain serving Satellite ID which is the gNB related Satellite ID is FFS.