3GPP TSG-SA WG6 Meeting #62	S6-243695

Maastricht, Netherlands, 19th – 23rd August 2024	(revision of S6-243652, 3569,3299)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on Update Sol#AE4 about estimated delivery time

Spec:	3GPP TR 23.700-01 v1.1.0

Agenda item:	8.6

Document for:	Approval

Contact:	gecuili@huawei.com

- Introduction

This pCR is proposed to update the solution#AE4 to introduce the estimated delivery time.

2. Reason for Change

SA2 already made conclusion for their KI #2: Support of Store and Forward Satellite operation as described in clause 8.2 of TS 23.700-29.

In the conclusion, the estimated delivery time is indicated to the UE/external SCS/AS.

“7)	The MME may indicate to UE the estimated delivery time in NAS messages (Attach accept or TAU accept message or service accept). How UE uses this information is left for UE implementation.

NOTE 2:	The estimated delivery time is the estimated time to send data from the UE to Gateway.

8)	The core network can indicate to external SCS/AS whether UE is registered in S&F Mode and the estimated delivery time.”

The estimated delivery time can be regarded as a one-time delay for the UL/DL transmission. From the application layer, such estimated delivery time can be used by the application layer to estimate whether the data is already forwarded by the S&F satellite. Take the DL data as an example, if T=estimated delivery time has passed, it means the data sent by the SCS/AS and cached at the S&F satellite has already left the S&F. In this case, the storage in the S&F is renewed so that the SCS/AS can send more data within the allocated quota. A figure is shown as below to illustrate the concept that the SEAL server based on the estimated delivery time to control the DL data transmission including when to send and how much data to send.

This solution is proposed to resolve the above issues.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR  23.700-01 0.3.0.

* * * First Change * * * *

7.2.4.1.2	Procedure of SEAL Server exposing the S&F status to the VAL server

Pre-condition:

-	The SEAL client, the SEAL server and the VAL server all are deployed on the ground.

-	The UPF/PGW-U in the 3GPP CN is deployed on the satellite.

Figure 7.2.4.1.2-1: Procedure of SEAL Server exposing the S&F status to the VAL server

1.	The VAL server sends UE S&F status events subscription requests to the SEAL server. The subscription events may include the UE Registration status in S&F Mode, the status of Feeder Link, the expected Delivery/Delay Time, the maximum S&F data storage quota per UE, the trigger conditions, etc.

2.	The SEAL server authorizes the subscription request from the VAL server and sends the subscription request to the VAL UE and/or the 3GPP CN respectively if the authorization is successful.

3.	The SEAL client and/or 3GPP CN acknowledge the SEAL server with UE S&F status events subscription response. And report the requested UE S&F status events when the trigger conditions for subscription are met. E.g., the UE is registered in S&F mode which may reported via UE or 3GPP CN, the Feeder Link is not available, the expected Delay Time is 10s, and the present maximum S&F data storage quota on Satellite 1 per UE is 10G, estimated delivery time which are all exposed from 3GPP CN through NEF/SCEF. The estimated delivery time indicating the estimated time to send data from the UE to gateway or from gateway to UE as described in clause 8.2 of 3GPP TS 23.700-29 [15].

Editor's note:	The UE S&F status events exposed from 3GPP CN depend on the progress of SA2 Rel-19 study.

4.	The SEAL server stores or updates the received UE S&F events results.

5.	The SEAL server may notify the VAL server for the received UE S&F events received in Step 3. The VAL server may take them into consideration and adjust the traffic transmission policies when there is DL data for the VAL UE as follows.

-	The VAL server may pend the DL data transition when the UE is in S&F Mode;

-	The VAL server may pend the DL data transition when the Feeder Link is not available;

-	The VAL server may transmit the DL data volume smaller than maximum S&F data storage quota;

-	The VAL server may re-transmit the pending DL data when the Feeder Link is available.

NOTE:	The above actions of the VAL server are all out of 3GPP scope.

6.	If the VAL server doesn’t subscribe the UE S&F status events or doesn’t support the S&F actions described in Step 5, the VAL server may send the DL data to the SEAL server even when e.g., the Feeder Link is not available.

7.	The SEAL server checks the received UE S&F events status and takes the following corresponding actions based on the different status of events.

When feeder link is available, the SEAL server sends the DL data (e.g., bulk#1) towards the 3GPP network. At the same time, the NRM server starts an estimated delivery timer based on the estimated delivery time which is corresponding to the feeder link. The estimated delivery timer is greater than or equal to the estimated delivery time.

8a.	The SEAL server may send “Pending DL data” request to the VAL server with the reason (e.g. the Feeder Link is not available, the UE is in S&F Mode, and estimated delivery timer is not expired) and the pending timer concluded from the expected Delivery/Delay Time will also be included.

8b.	The SEAL server may send “Reject DL data” request to the VAL server with the failure reason (e.g. the Feeder Link is not available).

8c.	The SEAL server may send “Forward DL data with the priority” request to the VAL server with the forwarding priories (e.g. when the DL data volume exceeds the maximum S&F data storage quota, only higher priorities services will be forwarded, such as the emergency service, VIP users).

8d.	The SEAL server may send “Adjust DL data volume” request to the VAL server with the adjusted traffic volume which is smaller than the maximum S&F data storage quota (e.g. when the DL data volume exceeds the maximum S&F data storage quota, the smaller traffic volume will be suggested and adjusted). If estimated delivery timer expires, the SEAL server calculates the new DL data volume (e.g., increasing the volume by the data size (e.g., size of bulk#1) delivered with the previous estimated delivery time), and informs the VAL server the adjusted DL data volume.