3GPP TSG-SA WG6 Meeting #62	S6-243720

# Maastricht, NL, 19 - 23 August, 2024

Source:	Ericsson

Title:	EN resolution for Sol#3

Spec:	3GPP TR 23.700-22 V0.4.0

Agenda item:	8.7

Document for:	Approval

1. Introduction

This pCR introduces the ENs resolution and completion of Solution#11.

2. Reason for Change

Solution#11 contains the ENs that shall be resolved.

3. Conclusions

This pCR resolves the ENs and updates the completes the Solution#11.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-23 V0.4.0.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[3]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	IETF draft-ietf-oauth-browser-based-apps-17: "OAuth 2.0 for Browser-Based Apps".

Editor's note:	The reference to draft-ietf-oauth-browser-based-apps-17 will be revised to RFC when finalized by IETF.

[6]	IETF 6749: "The OAuth 2.0 Authorization Framework".

[x]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

* * * Next Change * * * *

### 6.4.1	Solution description

#### 6.4.1.0	General

This solution addresses the KI#1 and enables the finer granularity of the access control for service API, i.e.:

1.	Service operation level access, the API invoker may request and the CAPIF may grant the access for the given service operation(s) of the service API.

2.	Resource level access, the API invoker may request and the CAPIF may grant the access for the given resource(s) (i.e., the object or component of the API on which the operations are acted upon as defined in clause 3.1 of 3GPP TS 23.222 [2]) of the service API.

NOTE:	The feature level access can be discussed during the normative phase.

#### 6.4.1.1	Impact to existing CAPIF procedures

The CAPIF procedure in 3GPP TS 23.222 [2] can be enhanced (highlighted in bold italics) as follows.

#### 8.3.2.1 Service API publish request Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function. Table 8.3.2.1-1: Service API publish request Information element; Status; Description. API publisher information; M; The information of the API publisher may include identity, authentication and authorization information. Service API information; M; The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API type, service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), Service API context (optional), and Network Slice Info (optional).. Shareable information; O (see NOTE); Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained.. NOTE: If the shareable information is not present, the service API is not allowed to be shared.. The Service KPIs is defined as below: Table 8.3.2.1-2: Service KPIs Information element; Status; Description. Maximum Request rate; O; Maximum request rate from the API Invoker supported by the server.. Maximum Response time; O; The maximum response time advertised for the API Invoker's service requests.. Availability; O; Advertised percentage of time the server is available for the API Invoker's use.. Available Compute; O; The maximum compute resource available for the API Invoker.. Available Graphical Compute; O; The maximum graphical compute resource available for the API Invoker.. Available Memory; O; The maximum memory resource available for the API Invoker.. Available Storage; O; The maximum storage resource available for the API Invoker.. Connection Bandwidth; O; The connection bandwidth in Kbit/s advertised for the API Invoker's use.. The Service API context is defined as below: Table 8.3.2.1-3: Service API context Information element; Status; Description. Service operations; O; The list of the supported service operations.. Resources; O; The list of supported resources, i.e, the object or component of the API on which the operations are acted upon.. #### 8.7.2.1 Service API discover request Table 8.7.2.1-1 describes the information flow service API discover request from the API invoker to the CAPIF core function. Table 8.7.2.1-1: Service API discover request Information element; Status; Description. API invoker identity information; M; Identity information of the API invoker discovering service APIs. Query information; M; Criteria for discovering matching service APIs (e.g. service API type, Serving Area Information (optional), preferred AEF location (optional), required API provider name (optional), UE IP address (optional), interfaces, protocols, Service KPIs (optional), Service API context (optional), and Network Slice Info (optional)). (see NOTE). NOTE: It should be possible to discover all the service APIs.. ### 8.11.3 Procedure Figure 8.11.3-1 illustrates the procedure for obtaining authorization to access the service API. Pre-condition: 1. The API invoker is onboarded and has received an API invoker identity. Figure 8.11.3-1: Procedure for the API invoker obtaining authorization for service API access 1. The API invoker sends an obtain service API authorization request to the CAPIF core function for obtaining permission to access the service API with optional Service API context by including the API invoker identity information and any information required for authentication of the API invoker. 2. The CAPIF core function validates the authentication of the API invoker (using authentication information) and checks whether the API invoker is permitted to access the requested service API with optional Service API context. NOTE 1: The authentication process is specified in subclause 6.5.2.3 of 3GPP TS 33.122 [12]. 3. Based on the API invoker's subscription information the authorization information to access the service APIs with optional Service API context is sent to the API invoker in the obtain service API authorization response. NOTE 2: The mechanism for distribution of the authorization information for the API invoker to the API exposing function is specified in subclause 6.5.2.3 of 3GPP TS 33.122 [12].

* * * Next Change * * * *

### 6.4.4	Solution evaluation

The coordination with SA3 is needed to align the information flows and procedures in 3GPP TS 33.122 [3].

This solution addesses KI#1 and enables the finer granularity of the access control for service API.

# *** End of Changes ***