3GPP TSG-SA WG6 Meeting #64	S6-245333

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245015)

Source:	FirstNet

Title:	Pseudo-CR on Generic IOPS – Cleaning up references in section 6

Spec:	3GPP TR 23.700-09, v0.3.0

Agenda item:	8.8

Document for:	Approval

Contact:	Mark Lipford, mark.lipford@firstnet.gov

1. Introduction

In reviewing the most recent version of the IOPS technical report (3GPP TR 23.700-09, v0.3.0) there are a number of references to 3GPP TS 23.180 throughout section 6 that do not have the appropriate [x] indication afterwards.  There are also instances where multicast/broadcast are used instead of broadcast/multicast.

This pCR proposes adding the reference as needed and corrects the broadcast/multicast usage.

2. Reason for Change

To make the docment more editorially correct

3. Conclusions

Recommends editorial corrections to add the references and correct the broadcast/multicast.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-09, v0.3.0.

* * * First Change * * * *

# 6.0	Solutions

## 6.1	Solution 1: Clause 1 (Scope) of TS 23.180

### 6.1.1	General

This clause is related to KI#1 addresses the Scope section of 3GPP TS 23.180 [7]. It describes which changes are needed to make the scope of 3GPP TS 23.180 applicable for all supported 3GPP access methods.

### 6.1.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

# 1	Scope

This document specifies the functional architecture, procedures and information flows needed to support mission critical services in the IOPS mode of operation. IOPS mode of operation allows an isolated 3GPP network to maintain a level of MC services in case the backhaul connection is lost. For example, for EPS the IOPS mode of operation is defined in 3GPP TS 23.401 [2].

The corresponding service requirements are defined in 3GPP TS 22.346 [9] and 3GPP TS 22.280 [11].

The present document is applicable to support mission critical services in the IOPS mode of operation for all supported access types including E-UTRAN, 5G or 5GA.

The functional architecture to support mission critical services in the IOPS mode of operation can be used for public safety applications and also for general commercial applications, e.g. utility companies and railways, for the case of a backhaul failure.

### 6.1.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.1.4	Solution evaluation

The above proposal converts the current scope section of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.2	Solution 2: Clause 3.1 (Definitions) of TS 23.180

### 6.2.1	General

This clause is related to KI#1 addresses the Scope section of 3GPP TS 23.180 [7].

### 6.2.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

IOPS MC system: The collection of application functions and enabling capabilities required to support mission critical services in the IOPS mode of operation.

IOPS mode of operation: Provides the ability to maintain a level of communications for MC users in case the backhaul connection is lost.

For the purposes of the present document, the following terms given in 3GPP TS 23.280 [3] apply

MC service

MC service client

MC service group

MC service ID

MC service user

MC service UE

MC user

### 6.2.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.2.4	Solution evaluation

The above proposal converts the current scope section of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.3	Solution 3: Clause 4 (Introduction) of TS 23.180

### 6.3.1	General

This clause is related to KI#1 addresses clause 4 of 3GPP TS 23.180 [7]. It describes which changes are needed to make it applicable for all supported 3GPP access methods.

### 6.3.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

# 4	Introduction

For the case of a backhaul failure, mission critical (MC) services (e.g. MCPTT defined in 3GPP TS 23.379 [5] and MCData defined in 3GPP TS 23.282 [6]) can also be supported based on the availability of an isolated operation for public safety (IOPS) MC system. The IOPS MC system provides MC services support to the MC service UE for example via a single public land mobile network (PLMN) dedicated to the IOPS mode of operation, as defined in 3GPP TS 23.401 [2].

A common functional architecture, including application plane and signalling plane, to support MC services in the IOPS mode of operation is specified in this document.

### 6.3.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.3.4	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.4	Solution 4: Clause 5 (Architecture requirement) of TS 23.180

### 6.4.1	General

This clause is related to KI#1 addresses clause 5 of 3GPP TS 23.180 [7]. It describes which changes are needed to make it applicable for all supported 3GPP access methods.

### 6.4.2	Solution

### 5.1.2	Requirements

[AR-5.1.2-a] For the case of a backhaul failure between the radio access network and the macro 3GPP core network, the IOPS MC system shall provide support for MC services in the IOPS mode of operation until the failure is recovered.

[AR-5.1.2-b] MC service UEs within the coverage of the local IOPS 3GPP system  shall be able to provide MC services over the IOPS MC system.

[AR-5.1.2-c] The IOPS MC system shall provide mechanisms to verify the availability of MC service UEs within the IOPS MC system.

## 5.2	MC service requirements

### 5.2.1	Description

This clause specifies the MC service requirements in the IOPS mode of operation.

### 5.2.2	Requirements

[AR-5.2.2-a] The following MCPTT service features shall be supported in the IOPS mode of operation

-	MCPTT group call

-	MCPTT emergency group call

-	MCPTT private call

-	MCPTT emergency private call

[AR-5.2.2-b] The following MCData service features shall be supported in the IOPS mode of operation

-	MCData short data service

## 5.3	Network broadcast/multicast session requirements

### 5.3.1	Description

This clause specifies the network bearer requirements in the IOPS mode of operation.

### 5.3.2	Requirements

[AR-5.3.2-a] The IOPS MC system shall provide support for unicast  session establishment and modification to support MC services via unicast transmissions in the IOPS mode of operation.

[AR-5.3.2-b] The IOPS MC system shall provide support for broadcast/multicast session establishment and modification to support MC services via /broadcast/multicast transmissions in the IOPS mode of operation.

[AR-5.3.2-c] The IOPS MC system shall support announcement of broadcast/multicast sessions  to the MC service UEs.

## 5.4	IP connectivity functionality

### 5.4.1	Description

The support of MC services in the IOPS mode of operation can be based on the support of the IP connectivity functionality. The IP connectivity functionality enables that MC services are provided by the MC service clients via the IOPS MC connectivity function.

An IOPS MC connectivity function supporting the IP connectivity functionality does not provide MC services. Instead, the IOPS MC connectivity function enables discovering MC service users and provides IP connectivity for the communication among the discovered MC service users, i.e. it distributes the related IP traffic to the MC service UEs over IP unicast and/or multicast transmissions. A MC service UE supporting the IP connectivity functionality in the IOPS mode of operation enables transmitting the IP packets related to a MC service communication over the IOPS MC connectivity function.

This clause specifies requirements for the support of the IP connectivity functionality in the IOPS mode of operation.

### 5.4.2	General requirements

[AR-5.4.2-a] The IOPS MC connectivity function supporting the IP connectivity functionality shall support discovering MC service users within the coverage of the local IOPS 3GPP system.

[AR-5.4.2-b] The IOPS MC connectivity function supporting the IP connectivity functionality shall support announcement, subscription and notification mechanisms for discovered MC service users supporting the IP connectivity functionality.

[AR-5.4.2-c] The IOPS MC connectivity function supporting the IP connectivity functionality shall provide IP connectivity for communication among MC service users requesting the support of the IP connectivity functionality.

[AR-5.4.2-d] The MC service UEs supporting the IP connectivity functionality in the IOPS mode of operation shall enable transmitting the IP packets related to a MC service communication over the IOPS MC connectivity function.

### 5.4.3	Bearer management requirements

[AR-5.4.3-a] The MC service UEs shall use specific APNs or DNNs for the support of the IP connectivity functionality, as follows:

-	An APN/DNN for the IOPS related application signalling related to the IP connectivity functionality support, e.g. for IOPS discovery, subscription and notification.

-	An MC service APN/DNN for the communication with another MC user or group via the IOPS MC connectivity function based on the IP connectivity functionality, e.g. for the signalling and media.

[AR-5.4.3-b] The APNs or DNNs for the IP connectivity functionality support shall be made available to the MC service UEs either via UE (pre)configuration or via initial UE configuration, as described in 3GPP TS 23.280 [3], and 3GPP TS 23.289 [23.289].

### 6.4.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

#### 6.4.3.1	Clause 2

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services ".

[4]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[5]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.002: "Network Architecture".

[8]	3GPP TS 23.203: "Policy and charging control architecture".

[9]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1".

[10]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[11]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[23.289]	3GPP TS 23.289: "Mission Critical Services over 5G System; Stage 2".

#### 6.4.3.2	Clause 3

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

IOPS MC system: The collection of application functions and enabling capabilities required to support mission critical services in the IOPS mode of operation.

IOPS mode of operation: As described in 3GPP TS 23.401 Annex K [2].

Local IOPS 3GPP system: It is a group of 3GPP core entities that are either co-sited with or co-located in an IOPS capable base station.

Unicast session: A unicast session is either a unicast bearer in EPS, or a unicast PDU session in 5GS.

For the purposes of the present document, the following terms given in 3GPP TS 23.280 [3] apply

MC service

MC service client

MC service group

MC service ID

MC service user

MC service UE

MC user

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

APN	Access Point Name

DNN	Data Network Name

IOPS	Isolated Operation for Public Safety

EPC	Evolved Packet Core

EPS	Evolved Packet System

MBMS	Multimedia Broadcast and Multicast Service

MC	Mission Critical

PLMN	Public Land Mobile Network

### 6.4.4	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.5	Solution 5: Clause 6 (Involved business relationships)

### 6.5.1	General

This solution addresses key issue #1 with the focus on clause 6 to be written in a generic way to be a generic IOPS solution.

### 6.5.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

# 6	Involved business relationships

The description of the involved business relationships for the support of MC services in the IOPS mode of operation is contained in clause 6 of 3GPP TS 23.280 [3], and clause 6.4 of 3GPP TS 23.289 [23.289].

### 6.5.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.5.4	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.6	Solution 6: Clause 7 (Functional model) of TS 23.180

### 6.6.1	General

This solution addresses key issue #1 with the focus on clause 7 to be written in a generic way to be a generic IOPS solution.

### 6.6.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

# 7	Functional model

## 7.1	General

The functional model to support MC services in the IOPS mode of operation in case of a backhaul failure consists of a set of functions and reference points.

NOTE:	Terminology such as client and server are not meant to imply specific physical implementation of a functional entity.

## 7.2	Description of the planes

The functional model for the support of MC services in the IOPS mode of operation is defined as a series of planes to allow for the breakdown of the architectural description.

The application plane supports all the necessary functions to establish and provide MC services in the IOPS mode of operation. The application plane uses the services of the signalling control plane to establish the association of the users involved in an MC service. The signalling control plane also offers access to and control of services across MC services.

## 7.3	Functional model description

### 7.3.1	General

The application plane and signalling plane supporting the functional model for the IOPS mode of operation are described in this clause.

### 7.3.2	Functional model for the signalling control plane in the IOPS mode of operation

Figure 7.3.2-1 shows the common functional model for the signalling control plane in the IOPS mode of operation.

Figure 7.3.2-1 Functional model for the signalling control plane in the IOPS mode of operation

NOTE:	The functional model for the signalling plane in the IOPS mode of operation is described as a common functional model across MC services.

### 7.3.3	MCPTT functional model for the application plane in the IOPS mode of operation

Figure 7.3.3-1 shows the MCPTT functional model for the application plane in the IOPS mode of operation.

Figure 7.3.3-1 MCPTT functional model for the application plane in the IOPS mode of operation

NOTE:	MC-IOPS-1, MC-IOPS-5 and MC-IOPS-6 reference points are common across MC services.

In the model shown in figure 7.3.3-1, the following apply:

-	MCPTT-IOPS-2 carries signalling related to the support of broadcast/multicast transmissions, e.g. MBMS bearer announcement, or 5G MBS session announcement, between the IOPS MC connectivity function and the MCPTT client.

-	MCPTT-IOPS-3 carries signalling and media over IP unicast transmissions between the IOPS distribution function and the MCPTT client of the MCPTT UE.

-	MCPTT-IOPS-4 carries signalling and media on downlink over IP multicast transmissions between the IOPS distribution function and the MCPTT client of the MCPTT UE.

### 7.3.4	MCData functional model for the application plane in the IOPS mode of operation

Figure 7.3.4-1 shows the MCData functional model for the application plane in the IOPS mode of operation.

Figure 7.3.4-1 MCData functional model for the application plane in the IOPS mode of operation

NOTE:	MC-IOPS-1, MC-IOPS-5 and MC-IOPS-6 reference points are common across MC services.

In the model shown in figure 7.3.4-1, the following apply:

-	MCData-IOPS-2 carries signalling related to the support of broadcast/multicast transmissions, e.g. MBMS bearer announcement, 5G MBS session announcement, between the IOPS MC connectivity function and the MCData client.

-	MCData-IOPS-3 carries signalling and media over IP unicast transmissions between the IOPS distribution function and the MCData client of the MCData UE.

-	MCData-IOPS-4 carries signalling and media on downlink over IP multicast transmissions between the IOPS distribution function and the MCData client of the MCData UE.

## 7.4	Functional entities description

### 7.4.1	Application plane

#### 7.4.1.1	General

Entities within the application plane of an IOPS MC system provide application specific functions to support MC services in the IOPS mode of operation.

#### 7.4.1.2	IOPS connectivity client

The IOPS connectivity client functional entity provides support for enabling that a user at the MC service client is registered and discovered by the IOPS MC system in the IOPS mode of operation. This functional entity is located in the MC service UE.

#### 7.4.1.3	IOPS MC connectivity function

The IOPS MC connectivity function entity provides supporting MC services in the IOPS mode of operation. It includes registering and discovering users within the IOPS MC system. The IOPS MC connectivity functional entity represents a specific instantiation of the GCS AS described in 3GPP TS 23.468 [4] for EPS, or Application Function (AF) for 5GS to manage unicast and multicast transmissions for group communication related sessions.

The IOPS MC connectivity function entity is supported by the signalling application server functional entity of the signalling control plane.

#### 7.4.1.4	IOPS distribution function

The IOPS distribution function entity provides support for distributing the IP packets containing the MC service application data received from a MC service UE in the IOPS mode of operation.

By means of information provided by the IOPS MC connectivity function (e.g. IP addresses, transport layer ports), it will provide the following functionality:

-	receiving uplink IP transmissions from the MC service UE by means of the MCPTT-IOPS-3 reference point for MCPTT and the MCData-IOPS-3 reference point for MCData;

-	distributing the related IP packets via downlink IP unicast transmissions to MC service UEs by means of the MCPTT-IOPS-3 reference point for MCPTT and the MCData-IOPS-3 reference point for MCData;

-	distributing the related IP packets via downlink IP multicast transmissions to MC service UEs by means of the MCPTT-IOPS-4 reference point for MCPTT and the MCData-IOPS-4 reference point for MCData.

#### 7.4.1.5	MC service client

The MC service client functional entity, e.g. a MCPTT client and a MCData client, supporting the IOPS mode of operation acts as the user agent for the corresponding MC service transactions as well as all IOPS related application transactions.

### 7.4.2	Signalling control plane

#### 7.4.2.1	Signalling user agent client

The signalling user agent client functional entity supports at the MC service client all required transactions related to the IOPS discovery. This includes supporting registration, publication, subscription and notification events.

#### 7.4.2.2	Signalling application server

The signalling application server functional entity supports at the IOPS MC system all required transactions related to the IOPS discovery. This includes supporting registration, publication, subscription and notification events.

## 7.5	Reference points

### 7.5.1	Application plane

#### 7.5.1.1	Reference point MC-IOPS-1 (between the IOPS MC connectivity function and the IOPS connectivity client)

The MC-IOPS-1 reference point, which exists between the IOPS MC connectivity function and the IOPS connectivity client, is used for IOPS related application signalling (e.g. registration, publication, subscription and notification events). The MC-IOPS-1 reference point utilizes the MC-IOPS-X1 reference point for transport and routing of the IOPS related signalling.

#### 7.5.1.2	Reference point MCPTT-IOPS-2 (between the IOPS MC connectivity function and the MCPTT client)

The MCPTT-IOPS-2 reference point, which exists between the IOPS MC connectivity function and the MCPTT client, is used to communicate signalling related to the broadcast/multicast sessions  utilized for broadcast/multicast transmissions, e.g. MBMS bearer announcement or 5G MBS session announcement in the IOPS mode of operation.

#### 7.5.1.3	Reference point MCData-IOPS-2 (between the IOPS MC connectivity function and the MCData client)

The MCData-IOPS-2 reference point, which exists between the IOPS MC connectivity function and the MCData client, is used to communicate signalling related to the broadcast/multicast sessions  utilized for broadcast/multicast transmissions, e.g. MBMS bearer announcement or 5G MBS session announcement in the IOPS mode of operation.

#### 7.5.1.4	Reference point MCPTT-IOPS-3 (between the IOPS distribution function and the MCPTT client)

The MCPTT-IOPS-3 reference point, which exists between the IOPS distribution function and the MCPTT client, is used for IP unicast transmissions in uplink and downlink. The MCPTT-IOPS-3 reference point uses the SGi reference point or N6 reference point as defined in 3GPP TS 23.002 [7] or 3GPP TS 23.501[23.501], respectively.

#### 7.5.1.5	Reference point MCData-IOPS-3 (between the IOPS distribution function and the MCData client)

The MCData-IOPS-3 reference point, which exists between the IOPS distribution function and the MCData client, is used for IP unicast transmissions in uplink and downlink. The MCData-IOPS-3 reference point uses the SGi reference point or N6 reference point as defined in 3GPP TS 23.002 [7] or 3GPP TS 23.501[23.501], respectively.

#### 7.5.1.6	Reference point MCPTT-IOPS-4 (between the IOPS distribution function and the MCPTT client)

The MCPTT-IOPS-4 reference point, which exists between the IOPS distribution function and the MCPTT client, is used for IP multicast transmissions in downlink. The MCPTT-IOPS-4 reference point uses the MB2-U reference point as defined in 3GPP TS 23.468 [4], or Nmb8/ Nmb6 reference point defined in 3GPP TS 23.247 [23.247], respectively.

#### 7.5.1.7	Reference point MCData-IOPS-4 (between the IOPS distribution function and the MCData client)

The MCData-IOPS-4 reference point, which exists between the IOPS distribution function and the MCData client, is used for IP multicast transmissions in downlink. The MCData-IOPS-4 reference point uses the MB2-U reference point as defined in 3GPP TS 23.468 [4], or Nmb8/Nmb6 reference points as defined in 3GPP TS 23.247 [23.247], respectively.

#### 7.5.1.8	Reference point MC-IOPS-5 (between the IOPS MC connectivity function and the local IOPS 3GPP system)

The MC-IOPS-5 reference point, which exists between the IOPS MC connectivity function and the local IOPS 3GPP system, is used to obtain unicast bearers with appropriate QoS from the local IOPS 3GPP system. The MC-IOPS-5 reference point utilizes the Rx interface of the EPS, or N5 interface of 5G system according to 3GPP TS 23.203 [8] or 3GPP TS 23.503[23.503], respectively.

#### 7.5.1.9	Reference point MC-IOPS-6 (between the IOPS MC connectivity function and the local IOPS 3GPP system )

The MC-IOPS-6 reference point, which exists between the IOPS MC connectivity function and the local IOPS 3GPP system, is used to request the allocation and activation of broadcast/multicast transport resources, i.e. MBMS bearers or 5GMBS sessions, during the IOPS mode of operation. The MC-IOPS-6 reference point uses the MB2-C interface as defined in 3GPP TS 23.468 [4], Nmb10, Nmb13, or N33, as defined in 3GPP TS 23.247 [23.247].

### 7.5.2	Signalling control plane

#### 7.5.2.1	Reference point MC-IOPS-X1 (between the signalling application server and the signalling user agent client)

The MC-IOPS-X1 reference point, which exists between the signalling application server and the signalling user agent client, is used to support the IOPS related application signalling (e.g. registration, publication, subscription and notification events).

NOTE:	The IOPS related signalling supporting, e.g., the IOPS discovery, subscription and notification events can be SIP-based. In such a case, the signalling application server acts as a SIP application server (AS) or application function (AF) and the signalling user agent client acts as the SIP user agent for all SIP related transactions.

### 6.6.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services ".

[4]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[5]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.002: "Network Architecture".

[8]	3GPP TS 23.203: "Policy and charging control architecture".

[9]	3GPP TS 22.346: "Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1".

[10]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[11]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[23.501]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[23.247]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[23.503]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

### 6.6.4	Solution evaluation

The above proposal converts the current clause of 3GPP TS 23.180 into an access generic IOPS solution.

## 6.7	Solution 7: Converting the functional model and deployment section of TS 23.180

### 6.7.1 	General

This clause is related to KI#1 addresses the Scope section of 3GPP TS 23.180 [7]. It describes which changes are needed to make the application of functional model to deployments of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

NOTE:	in clause 6.aa.2 and 6.aa.3, the original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

### 6.7.2		Addressing the functional model and deployment section of 3GPP TS 23.180

# 9	Application of functional model to deployments

## 9.1	Overview

The IOPS architectural model includes application functions at the IOPS MC system and UEs to support MC services in the IOPS mode of operation during a backhaul failure. The IOPS MC system provides MC services support to the MC service UE via a single PLMN dedicated to the IOPS mode of operation.

The architectural model to support MC services in the IOPS mode of operation consists of a signalling control plane and an application plane. The signalling control plane provides the necessary signalling support for the related IOPS application layer transactions, e.g. the registration and discovery of UEs on the IOPS MC system. The IOPS application plane provides the necessary support for the transport of the IOPS operation related application data as well as the IP packets containing the MC service application data to be distributed via the IOPS MC system.

The MC service application data includes all signalling control data and application data (control and media) required to provide MC services between MC service clients. The IP related transmissions are established over the IOPS MC system via IP unicast and multicast transmissions.

## 9.2	IOPS architectural model diagram

Figure 9.2-1 shows the IOPS architectural model for the IOPS MC system solution in case of a backhaul failure.

Figure 9.2-1 Architectural model in the IOPS mode of operation

### 9.2.1	IOPS application services support layer

The IOPS MC system is composed of the following functional entities for the support of MC services in the IOPS mode of operation:

-	an IOPS MC connectivity function as described in clause 7.4.1.3, an IOPS distribution function as described in clause 7.4.1.4 and a signalling application server as described in clause 7.4.2.2.

### 9.2.2	Local IOPS 3GPP system

A local IOPS 3GPP system can comprise either:

-	a local core network and a single isolated IOPS-capable base station, which may be co-located or have connectivity to the local core network; or

-	a local core network and two or more IOPS-capable base station, which have connectivity to a single local core network.

NOTE:	For 4G IOPS, the implementation and deployment guidelines using a Local EPC approach is described in 3GPP TS 23.401 [2].

The local IOPS 3GPP system provides point-to-point and/or point-to-multipoint transmission services with QoS in the IOPS mode of operation, e.g., as described in 3GPP TS 23.401 [2] for EPS.

### 9.2.3	UE 1

The UE 1 is an MC service UE supporting the IOPS mode of operation. It supports transmission services and applications to provide MC services over the IOPS MC system. The UE 1 is composed of the following functional entities:

-	for MC services over the IOPS MC system, MC service client(s) as described in clause 7.4.1.5;

-	for IOPS application plane, an IOPS connectivity client as described in clause 7.4.1.2; and

-	for IOPS signalling control, a signalling user agent as described in subclause 7.4.2.1.

### 9.2.4	UE 2

## The UE 2 represents one or more UEs with the same functionality as UE 1.

## 6.8	Solution 8: Clause 10.2.2.3 (IOPS discovery request) of TS 23.180

### 6.8.1	General

This clause is related to KI#1 and addresses clause 10.2.2.3 of 3GPP TS 23.180 [7].

### 6.8.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

#### 10.2.2.3	IOPS discovery request

Table 10.2.2.3-1 describes the information flow for the IOPS discovery request from the IOPS connectivity client to the IOPS MC connectivity function.

Table 10.2.2.3-1: IOPS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| IOPS MC service ID | M | The IOPS MC service ID of the requesting MC user |
| IP connectivity information | M | It includes the requesting MC service UE's IP address (NOTE 1) |
| List of IOPS MC service group ID (NOTE 2) | M | It includes the IOPS group (IOPS MC service group ID and its associated IOPS group IP multicast address) or list of IOPS groups the requesting MC user is configured with |
| Availability of connectivity information | O | It indicates if the connectivity information of the requesting MC user can be made available to other users on the system |
| List of IOPS MC service IDs (NOTE 2 and NOTE 3) | O | The specific list of IOPS MC service IDs that the requesting MC user's connectivity information can be shared with |
| Priority state | M | Indicates whether the requesting MC user is requesting higher priority or not |
| NOTE 1: The MC service UE's IP address is assigned by the local IOPS 3GPP system based on the PDN connection or PDU session established by the MC user for the IOPS IP connectivity functionality. NOTE 2: The list of IOPS MC service IDs or IOPS MC service group IDs shall be of the same MC service as the MC user requesting to be discovered. NOTE 3: If present, the user's connectivity information availability can be only made available to the provided specific list of IOPS MC service IDs. |  |  |

### 6.8.3	Other affected clauses

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution. If this clause is empty, no other clauses are affected.

### 6.8.4	Solution evaluation

The above proposal converts the current scope section of 3GPP TS 23.180 [7] into an access generic IOPS solution.

## 6.9	Solution 9: Converting the functional model and deployment section of TS 23.180

### 6.9.1 	General

This clause is related to KI#1 addresses the MBMS section of 3GPP TS 23.180 [7]. It describes which changes are needed to make the application of functional model to deployments of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

NOTE:	in clause 6.8.2 and 6.8.3, the original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

### 6.9.2		Addressing the MBMS section of 3GPP TS 23.180

## 10.4	Use of broadcast/multicast transmissions

### 10.4.1	General

An IOPS MC connectivity function can provide support for group communications based on broadcast/multicast transmissions. Broadcast/multicast transmissions can be used for the downlink delivery of MC service application signalling and media to the MC service UEs over broadcast/multicast session (e.g., MBMS bearers in EPS, MBS session in 5GS).

The use of broadcast/multicast transmissions in the IOPS mode of operation are based on the related information flows and procedures specified in 3GPP TS 23.280 [3], 3GPP TS 23.379 [5], 3GPP TS 23.282 [6], and 3GPP TS 23.289 [23289].

### 10.4.2	Information flows

#### 10.4.2.1	MapIOPSGroupToNetworkSession (IP connectivity functionality)

Table 10.4.2.1-1 describes the information flow to associate a group session based on the IP connectivity functionality to a broadcast/multicast session. It is sent from the IOPS MC connectivity function to the MC service client.

Table 10.4.2.1-1: MapIOPSGroupToNetworkSession (IP connectivity functionality)

| Information element | Status | Description |
| --- | --- | --- |
| IOPS MC service group ID | M | This element identifies the IOPS MC service group, in which the group session is started. |
| Media stream identifier | M | This element identifies the media stream of the SDP used for the group session (e.g. MBMS subchannel). |
| Identifier of broadcast/multicast session (NOTE) | O | The identifier of the network broadcast/multicast session (e.g., TMGI) if the media of the group session is not sent on the same broadcast/multicast session as this MapIOPSGroupToNetwork message. |
| Session acknowledgement indicator | O | Indication that the group session requires acknowledgement from receiving MC service clients. |
| NOTE: “Identifier of broadcast/multicast session” IE shall be present if this message is sent over a different broadcast/multicast session than the media of the group session; This IE may be present if this message is sent over the same broadcast/multicast session as the media of the group session. |  |  |

#### 10.4.2.2	UnmapIOPSGroupFromNetworkSession (IP connectivity functionality)

Table 10.4.2.2-1 describes the information flow to disconnect a group session based on the IP connectivity functionality from an broadcast/multicast session. It is sent from the IOPS MC connectivity function to the MC service client.

Table 10.4.2.2-1: UnmapIOPSGroupFromNetworkSession

| Information element | Status | Description |
| --- | --- | --- |
| IOPS MC service group ID | M | This element identifies the IOPS MC service group, in which the group session is started. |

### 10.4.3	Use of pre-established broadcast/multicast session

The MC services in the IOPS mode of operation shall support the procedure for using pre-established broadcast/multicast session as specified 3GPP TS 23.280 [3] or 3GPP TS 23.289 [23289] with the following clarification:

-	The MC service server is the IOPS MC connectivity function.

The MCPTT service shall use the MCPTT-IOPS-2, MCPTT-IOPS-4, and MC-IOPS-6 reference points for this procedure.

The MCData service shall use the MCData-IOPS-2, MCData-IOPS-4, and MC-IOPS-6 reference points for this procedure.

The IOPS MC system may use pre-established broadcast/multicast sessions for distributing the media associated to group sessions. Depending on the capacity of the broadcast/multicast session, the broadcast/multicast sessioncan be used to broadcast one or more group sessions in parallel.

### 10.4.4	Use of dynamic broadcast/multicast session establishment

The MC services in the IOPS mode of operation shall support the procedure for using dynamic broadcast/multicast session as specified 3GPP TS 23.280 [3] or 3GPP TS 23.289 [23289] with the following clarification:

-	The MC service server is the IOPS MC connectivity function.

The MCPTT service shall use the MCPTT-IOPS-2, MCPTT-IOPS-4, and MC-IOPS-6 reference points for this procedure.

The MCData service shall use the MCData-IOPS-2, MCData-IOPS-4, and MC-IOPS-6 reference points for this procedure.

The IOPS MC system may use dynamic MBMS bearer establishment for distributing the media associated to group sessions.

### 10.4.5	Group session connect and disconnect over broadcast/multicast session (IP connectivity functionality)

#### 10.4.5.1	General

MBMS bearers can be used for group sessions based on the IP connectivity functionality. One broadcast/multicast session can be associated to one or more specific groups or group sessions. Before sending packets related to a group session, i.e. signalling and media, over an broadcast/multicast session, the IOPS MC connectivity function transmits the association information between the group session and the broadcast/multicast session. When the group session over the broadcast/multicast session is finished, this temporary association information of a group session to specific resources on a broadcast/multicast session is undone. Prior to the association of a group session to a broadcast/multicast session the broadcast/multicast session is activated and announced to the MC service clients.

#### 10.4.5.2	Procedure

##### 10.4.5.2.1	Group Session connect over broadcast/multicast session

In figure 10.4.5.2.1-1 the MC service client 1 is the client that initiates a group session, e.g. an MCPTT client initiating a group call. The MC service client 1 may be within the broadcast/multicast  service area. The MC service clients 2 … n represent MC service clients, e.g. MCPTT clients, receiving the group session packets over an MBMS bearer. There may be other receiving MC service clients both over unicast bearers and over this or other broadcast/multicast session(s), however, they are not illustrated in this figure.

Pre-conditions:

-	The MC service user profile used for the IOPS mode of operation is pre-provisioned in the MC service UEs

-	The IOPS MC service group ID and its associated IOPS group IP multicast address are pre-configured in the MC service clients

-	The MC service users have an active PDN connection to the IOPS MC connectivity function for the communication based on the IP connectivity functionality

-	The MC service users are discovered by the IOPS MC connectivity function to support MC services based on the IP connectivity functionality

Figure 10.4.5.2.1-1: Group session connect on broadcast/multicast session (IP connectivity functionality)

1.	Activation and announcement of broadcast/multicast session availability.

2.	MC user at the MC service client 1 wants to initiate an MC service group session with a specific IOPS MC service group based on the IP connectivity functionality.

3.	The MC service client 1 transmits the related group session packets via the IOPS MC connectivity function targeting the IOPS MC service group (i.e. using the corresponding IOPS group IP multicast address)

4.	The IOPS MC connectivity function determines that the received packets correspond to a group session. Therefore, the IOPS MC connectivity function transmits a MapIOPSGroupToNetwork message over a previously established broadcast/multicast session to all users monitoring the broadcast/multicast session. The MapIOPSGroupToNetwork message includes association information between the group session and the broadcast/multicast session. It includes the corresponding IOPS MC service group ID associated to the IOPS group IP multicast address related to the group session.

5.	The IOPS MC connectivity function distributes the received group session packets from the MC service client 1 to other MC service clients (e.g. MC service clients 2 … n) over the broadcast/multicast session.

NOTE:	MC service clients receiving the group session packets over the broadcast/multicast session only decode the packets that are addressed to their pre-configured IOPS group IP multicast address(es). Otherwise, the MC service clients discard the packets.

##### 10.4.5.2.2	Group session disconnect over broadcast/multicast session

Figure 10.4.5.2.2-1 shows the high-level procedure where an UnmapIOPSGroupFromNetworkSession message is transmitted from the IOPS MC connectivity function to MC service clients to indicate that a group session is being disconnected or dissociated from the broadcast/multicast session.

Figure 10.4.5.2.2-1: Group session disconnect on broadcast/multicast session (IP connectivity functionality)

1.	A group session based on the IP connectivity functionality is ongoing; the group session packets received from the MC service client 1 are distributed over broadcast/multicast session to other MC service clients within the system.

2.	The IOPS MC connectivity function has determined to disconnect the group session from the broadcast/multicast session for the MC service clients within the system.

NOTE:	The IOPS MC connectivity function may decide to disconnect a group session from a broadcast/multicast session if group session packets are not anymore received addressing the corresponding IOPS MC service group (or IOPS group IP multicast address).

3.	An UnmapGroupFromBearer message is transmitted from the IOPS MC connectivity function to the MC service clients and possibly to the MC service client 1 (if it is within the broadcast/multicast coverage area) on broadcast/multicast session(s).

### 6.9.3	Other affected clauses

#### 6.9.3.1	definitions update

The generic definitions are required.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

IOPS MC system: The collection of application functions and enabling capabilities required to support mission critical services in the IOPS mode of operation.

IOPS mode of operation: As described in 3GPP TS 23.401 Annex K [2].

Broadcast/multicast session: is either the MBMS bearer in EPS or the MBS session in 5GS.

For the purposes of the present document, the following terms given in 3GPP TS 23.280 [3] apply

MC service

MC service client

MC service group

MC service ID

MC service user

MC service UE

MC user

### 6.9.4	Solution evaluation

The above proposal converts the current MBMS section of 3GPP TS 23.180 into an access generic IOPS solution.

## 6.10	Solution 10: Clause 10.5.1.4 (IOPS emergency group call) of TS 23.180

### 6.10.1	General

This clause is related to KI#1 addresses section 10.5.1.4 of 3GPP TS 23.180 [7]. It describes which changes are needed to make the section IOPS emergency group call of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

### 6.10.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

#### 10.5.1.4	IOPS emergency group call

The procedure in figure 10.5.1.4-1 illustrates the procedure for an IOPS MCPTT emergency group call establishment based on the IP connectivity functionality. The IOPS emergency group call is a special case of the IOPS group call setup procedure described in clause 10.5.1.3, wherein the IOPS group call announcement contains an indication that the IOPS group call is an IOPS emergency group call. The group call participants can become aware of the IOPS MCPTT group's in-progress emergency state based on the emergency indicator.

When an MCPTT client intends to initiate an IOPS emergency group call, the MCPTT client can request higher priority from the IOPS MC connectivity function via the IOPS discovery request.

An IOPS group call in-progress can be upgraded to an IOPS emergency group call by including the emergency indicator within the periodic IOPS group call announcement. An IOPS group call in-progress can also be upgraded by a participating MCPTT client by sending an IOPS emergency group call upgrade to the IOPS group.

The MCPTT user who initiated an IOPS emergency group call or upgraded an IOPS group call to an emergency group call, or an authorized user can cancel the emergency state of the group call by sending an IOPS emergency group call state cancel to the IOPS MCPTT group. The emergency state of the IOPS group call remains active until the emergency group call ends or the in-progress emergency state is cancelled.

Pre-conditions:

-	MCPTT user profile used for the IOPS mode of operation is pre-provisioned in the MCPTT UEs;

-	The IOPS MCPTT group ID and its associated IOPS group IP multicast address are pre-configured in the MCPTT clients;

-	MCPTT users have an active PDN connection or PDU session to the IOPS MC connectivity function for the communication based on the IP connectivity functionality;

-	MCPTT users affiliated to the target IOPS MCPTT group are discovered by the IOPS MC connectivity function supporting the IP connectivity functionality;

-	The IOPC MC connectivity function may have established an multicast/broadcast/multicast session and announced it to the MCPTT clients;

-	MCPTT client 1 may have retrieved group connectivity information from the IOPS connectivity client related to the target IOPS MCPTT group;

-	MCPTT clients 1, 2 … n are configured within the same IOPS MCPTT group.

Figure 10.5.1.4-1: IOPS emergency group call setup based on the IP connectivity functionality

1.	The MCPTT user at MCPTT client 1 would like to initiate an IOPS emergency group call with a specific IOPS MCPTT group based on the IP connectivity functionality.

NOTE 1:	The MCPTT client 1 may have previously requested higher priority from the IOPS MC connectivity function using the IOPS discovery request.

2.	The MCPTT client 1 sends an IOPS group call announcement to the target IOPS MCPTT group. The announcement contains an indication that the call is an IOPS emergency group call. The MCPTT client 1 transmits the group session packets carrying the IOPS group call announcement to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

3.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCPTT group. The IOPS MC connectivity function decides to distribute the received group session packets to the target MCPTT clients over multicast/broadcast/multicast and/or unicast transmissions. If the MCPTT client 1 requested a priority state from the IOPS MC connectivity function, the IOPS MC connectivity function distributes the group session packets with higher priority.

4.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS group call announcement to the MCPTT clients from the target IOPS MCPTT group.

5.	The MCPTT clients receiving the IOPS group call announcement with an emergency indicator join the IOPS emergency group call and notify the target MCPTT users about the IOPS emergency group call. The IOPS MCPTT group's emergency state is defined.

NOTE 2:	Whilst the emergency state of the IOPS group call remains active, other participating MCPTT clients of the group call may also request higher priority from the IOPS MC connectivity function using the IOPS discovery request.

6.	If confirm mode indication is included in the IOPS group call announcement, the receiving MCPTT clients respond to the IOPS MCPTT group indicating the result of the establishment of the announced IOPS emergency group call. The receiving MCPTT clients transmit the group session packets carrying the IOPS group call response to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

NOTE 3:	Step 6 can also occur prior to step 5.

7.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCPTT group. The IOPS MC connectivity function decides to distribute the received group session packets to the target MCPTT clients over multicast/broadcast/multicast and/or unicast transmissions. If any participating MCPTT client of the group call requested a priority state from the IOPS MC connectivity function, the IOPS MC connectivity function distributes the group session packets with higher priority.

8.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS group call response to the MCPTT clients from the target IOPS MCPTT group. The MCPTT clients recognize the IOPS emergency group call originator through the IOPS group call announcement and can check the participants of the IOPS group call through the received response message.

9.	The MCPTT clients have successfully established the IOPS emergency group call based on the IP connectivity functionality.

NOTE 4:	Due to the movement of the participants (in and out of the local IOPS 3GPP system coverage) during the IOPS emergency group call, the IOPS group call announcement is periodically sent by the MCPTT client 1.

NOTE 5:	The participating MCPTT clients do not need to respond to the periodic IOPS group call announcement.

## 6.11	Solution 11: Clause 10.5.1.3 (IOPS group call setup) of TS 23.180

### 6.11.1	General

This clause is related to KI#1 addresses section 10.5.1.3 of 3GPP TS 23.180 [7]. It describes which changes are needed to make the section 10.5.1.3 (IOPS group call setup) of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

### 6.11.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

#### 10.5.1.3	IOPS group call setup

The procedure in figure 10.5.1.3-1 illustrates the procedure for an IOPS MCPTT group call establishment based on the IP connectivity functionality. The procedure describes how an MCPTT client initiates and establishes an IOPS MCPTT group call with other MCPTT clients.

Pre-conditions:

-	MCPTT user profile used for the IOPS mode of operation is pre-provisioned in the MCPTT UEs;

-	The IOPS MCPTT group ID and its associated IOPS group IP multicast address are pre-configured in the MCPTT clients;

-	MCPTT users have an active PDN connection or PDU session to the IOPS MC connectivity function for the communication based on the IP connectivity functionality;

-	MCPTT users affiliated to the target IOPS MCPTT group are discovered by the IOPS MC connectivity function supporting the IP connectivity functionality;

-	The IOPC MC connectivity function may have established an multicast/broadcast/multicast session and announced it to the MCPTT clients;

-	MCPTT client 1 may have retrieved group connectivity information from the IOPS connectivity client related to the target IOPS MCPTT group;

-	MCPTT clients 1, 2 … n are configured within the same IOPS MCPTT group.

Figure 10.5.1.3-1: IOPS group call setup based on the IP connectivity functionality

1.	The MCPTT user at MCPTT client 1 would like to initiate an IOPS group call with a specific IOPS MCPTT group based on the IP connectivity functionality.

2.	The MCPTT client 1 sends an IOPS group call announcement to the target IOPS MCPTT group. The MCPTT client 1 transmits the group session packets carrying the IOPS group call announcement to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

3.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCPTT group. The IOPS MC connectivity function decides distributing the received group session packets to the target MCPTT clients over multicast/broadcast/multicast and/or unicast transmissions.

4.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS group call announcement to the MCPTT clients from the target IOPS MCPTT group.

5.	The MCPTT clients receiving the IOPS group call announcement join the IOPS group call and notify the target MCPTT users about the IOPS group call.

6.	If confirm mode indication is included in the IOPS group call announcement, the receiving MCPTT clients respond to the IOPS MCPTT group indicating the result of the establishment of the announced IOPS group call. The receiving MCPTT clients transmit the group session packets carrying the IOPS group call response to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

NOTE 1:	Step 6 can also occur prior to step 5.

7.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCPTT group. The IOPS MC connectivity function decides distributing the received group session packets to the target MCPTT clients over multicast/broadcast/multicastand/or unicast transmissions.

8.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS group call response to the MCPTT clients from the target IOPS MCPTT group. The MCPTT clients recognize the IOPS group call originator through the IOPS group call announcement and can check the participants of the IOPS group call through the received response message.

9.	The MCPTT clients have successfully established the IOPS group call with floor control based on the IP connectivity functionality.

NOTE 2:	Due to the movement of the participants (in and out of the local IOPS 3GPP system coverage) during the IOPS group call, the IOPS group call announcement is periodically sent by the MCPTT client 1.

NOTE 3:	The participating MCPTT clients do not need to respond to the periodic IOPS group call announcement.

## 6.12	Solution 12: Clause 10.5.3.1 (IOPS floor control - General) of TS 23.180

### 6.12.1	General

This clause is related to KI#1 addresses section 10.5.3.1 of 3GPP TS 23.180 [7]. It describes which changes are needed to make the General section of IOPS floor control of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

### 6.12.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

#### 10.5.3.1	General

For MCPTT calls based on the IP connectivity functionality in the IOPS mode of operation, floor control is performed by using floor control messages among the MCPTT clients without a centralized MCPTT server. The MCPTT client can transmit voice packets over the IOPS MC connectivity function once it is granted the right to speak, either locally in the UE or by the reception of a floor granted message from another MCPTT client.

The MCPTT client currently speaking performs the temporary floor arbitrator during speaking since there is no centralized MCPTT floor control server. The floor arbitrator controls the floor whether or not queue is supported, and when floor is requested with override. If queue is supported, the MCPTT client performing floor arbitrator grants the right to speak to the next speaker and transfers the floor arbitrator role after completing the voice transfer and releasing the floor. For IOPS group calls, the floor arbitrator also transfers the floor control queue when granting the floor. The next MCPTT client receiving the right to speak becomes the new floor arbitrator and, for IOPS group calls, has the floor control queue.

For IOPS group calls, the group session packets carrying the floor control messages can be transmitted by the IOPS MC connectivity function over multicast/broadcast/multicast transmissions and can be monitored by all the members from the target IOPS MCPTT group.

The following clauses specify the floor control procedures and information flows for IOPS private calls and IOPS group calls based on the IP connectivity functionality in the IOPS mode of operation.

## 6.13	Solution 13: Clause 10.5.3.3 (IOPS floor control during silence) of TS 23.180

### 6.13.1	General

This clause is related to KI#1 addresses section 10.5.3.3 of 3GPP TS 23.180 [7]. It describes which changes are needed to make the section 10.5.3.3 (IOPS floor control during silence) of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

### 6.13.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

#### 10.5.3.3	IOPS floor control during silence

If a floor arbitrator does not exist, figure 10.5.3.3-1 shows the successful high level floor control procedure during periods when there is no detectable talker in an IOPS group call based on the IP connectivity functionality.

NOTE 1:	The description also applies to IOPS private calls.

Pre-conditions:

-	MCPTT user profile used for the IOPS mode of operation is pre-provisioned in the MCPTT UEs.

-	MCPTT users have an active PDN connection or PDU session to the IOPS MC connectivity function for the communication based on the IP connectivity functionality

-	The IOPS MCPTT group ID and its associated IOPS group IP multicast address are pre-configured in the MCPTT clients (for the case of an IOPS group call)

-	The IOPC MC connectivity function may have established a multicast/broadcast/multicast session and announced it to the MCPTT clients

-	The MCPTT users are discovered by the IOPS MC connectivity function supporting the IP connectivity functionality.

-	MCPTT clients has retrieved connectivity information from the target MCPTT user (for the case of an IOPS private call).

-	An IOPS private call or IOPS group call based on the IP connectivity functionality has been established. No participant is currently talking (i.e. the floor is idle) and no floor arbitrator is identified.

Figure 10.5.3.3-1: Successful floor taken flow in an IOPS group call based on the IP connectivity functionality (no floor contention)

1.	The MCPTT client 1 sends the IOPS floor request message to the target IOPS MCPTT group. The MCPTT client 1 transmits the group session packets carrying the IOPS floor request message to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

2.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCPTT group. The IOPS MC connectivity function decides distributing the received group session packets to the target MCPTT clients over multicast/broadcast/multicast and/or unicast transmissions.

3.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS floor request to the MCPTT clients from the target IOPS MCPTT group.

4.	The MCPTT client 1 does not detect any floor contention. Floor contention occurs when multiple floor requests may exist simultaneously.

NOTE 2:	The mechanism for detecting floor contention in the IOPS mode of operation is out of scope of the present document.

5.	The MCPTT client 1 sends the IOPS floor taken message to the IOPS MCPTT group. The MCPTT client 1 transmits the group session packets carrying the IOPS floor taken message to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

6.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCPTT group. The IOPS MC connectivity function decides distributing the received group session packets to the target MCPTT clients over multicast/broadcast/multicast and/or unicast transmissions.

7.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS floor taken message to the MCPTT clients from the target IOPS MCPTT group.

8.	The MC user at MCPTT client 1 gets a notification that the IOPS floor request was successful (the floor has been granted).

NOTE 3:	Step 8 can also occur prior to steps 6 and 7.

9.	The MCPTT client 1 begins voice transmission with the target IOPS MCPTT group based on the IP connectivity functionality.

## 6.14	Solution 14: Clause 10.6.1.4 (IOPS group standalone SDS using signalling control plane) of TS 23.180

### 6.14.1	General

This clause is related to KI#1 addresses section 10.6.1.4 of 3GPP TS 23.180 [7]. It describes which changes are needed to make the scope of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

### 6.14.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

#### 10.6.1.4	IOPS group standalone SDS using signalling control plane

##### 10.6.1.4.1	General

IOPS group standalone SDS using signalling control plane based on the IP connectivity functionality can use pre-configured information provided to MCData clients prior to initiating the data service. When an MCData client initiates an IOPS group standalone SDS based on the IP connectivity functionality it uses the pre-configured IOPS group IP multicast address associated to the target IOPS MCData group ID. The related group session packets, i.e. signalling messages, carrying the data are transmitted to the IOPS MC connectivity function for distribution to the corresponding discovered MC users of the target IOPS MCData group.

The IOPS MC connectivity function can distribute the group session packets to the discovered MC users over broadcast/multicast sessions as described in clause 10.4.5.

The IOPS MC connectivity function can also replicate and distribute the group session packets over unicast transmissions to MCData UEs associated to the target IOPS MCData group. MCData UEs receiving the group session packets are associated to discovered MC users that included the target IOPS MCData group ID within the IOPS discovery request, as described in clause 10.5.2.3.

##### 10.6.1.4.2	Procedure

The procedure in figure 10.6.1.4.2-1 describes the case where an MCData user is initiating an IOPS group MCData communication for sending standalone SDS data over signalling control plane to an IOPS MCData group, with or without disposition request. Standalone refers to sending unidirectional data in one transaction.

Pre-conditions:

-	MCData user profile used for the IOPS mode of operation is pre-provisioned in the MCData UEs.

-	The IOPS MCData group ID and its associated IOPS group IP multicast address are pre-configured in the MCData clients.

-	MCData users have an active PDN connection or PDU session to the IOPS MC connectivity function for the communication based on the IP connectivity functionality.

-	MCData users affiliated to the target IOPS MCData group are discovered by the IOPS MC connectivity function supporting the IP connectivity functionality.

-	The IOPS MC connectivity function may have established a broadcast/multicast session and announced it to the MCData clients.

-	MCData client 1 may have retrieved group connectivity information from the IOPS connectivity client related to the target IOPS MCData group.

-	MCData clients 1, 2 … n are configured within the same IOPS MCData group.

Figure 10.6.1.4.2-1: IOPS group standalone SDS using signalling control plane based on the IP connectivity functionality

1.	The MCData user at MCData client 1 would like to initiate an IOPS SDS data transfer with a specific IOPS MCData group based on the IP connectivity functionality. The MCData client 1 checks whether the MCData user 1 is authorized to send an IOPS MCData group standalone data request.

2.	The MCData client 1 sends an IOPS MCData group standalone data request to the target IOPS MCData group. The MCData client 1 transmits the group session packets carrying the IOPS MCData group standalone data request to the IOPS MC connectivity function for distribution to the corresponding IOPS group IP multicast address.

The IOPS MCData group standalone data request contains the data payload, i.e. the SDS content. The request also contains a conversation identifier for message thread indication and may contain a disposition request if indicated by the user at MCData client 1.

3.	The IOPS MC connectivity function determines that the received packets correspond to a group session targeting a specific IOPS MCData group. The IOPS MC connectivity function decides distributing the received group session packets to the target MCData clients over broadcast/multicast session and/or unicast transmissions.

4.	The IOPS MC connectivity function distributes the group session packets carrying the IOPS MCData group standalone data request to the discovered MCData clients from the target IOPS MCData group.

5.	The MCData clients receiving the IOPS MCData group standalone data request check whether any policy is to be asserted to limit certain types of messages or content to certain members due to, for example, location or user privilege. If the policy assertion is positive and the payload is for MCData user consumption (e.g. it is not application data, not command instructions, etc.) then the MCData clients notify the target MCData users.

The actions taken when the payload contains application data or command instructions are based on the payload content. Payload content received by an MCData client which is addressed to a known local non-MCData application that is not yet running shall cause the MCData client to start the local non-MCData application (i.e., remote start application) and shall pass the payload content to the just started application.

NOTE:	If the policy assertion was negative, the corresponding MCData client sends an appropriate notification to MCData client 1.

6.	If MCData data disposition was indicated (for delivery, read or both) within the request sent by the MCData client 1, then the receiving MCData clients initiate the corresponding IOPS MCData data disposition notification(s) towards the MCData client 1, i.e. addressing the MCData UE 1's IP address.

7.	The IOPS MC connectivity function receives the session packets addressing the MCData UE 1's IP address. The IOPS MC connectivity function checks if the MCData UE 1's IP address corresponds to a discovered MC user in order to distribute the received session packets. If it does, the IOPS MC connectivity function distributes the received session packets to the target MCData client over unicast transmissions.

8.	The IOPS MC connectivity function distributes the session packets carrying the IOPS MCData data disposition notification to the MCData client 1.

## 6.15	Solution 15: Clause 1.7 (MC IOPS notification) of TS 23.180

### 6.15.1	General

This clause is related to KI#1 addresses section 10.7 of 3GPP TS 23.180 [7]. It describes which changes are needed to make the scope of 3GPP TS 23.180 [7] applicable for all supported 3GPP access methods.

### 6.15.2	Solution

Note:	The original clause has been copied from TS 23.180 [7]. The change marks highlight the required modifications for an access generic IOPS solution.

## 10.7	MC IOPS notification

### 10.7.1	General

In the IOPS mode of operation, it is assumed that the IOPS MC system does not have connectivity to the primary MC system due to the backhaul failure. Therefore, the primary MC system cannot be aware of the initiation of the IOPS operation and the corresponding activation of an IOPS MC connectivity function within the primary MC system coverage.

When an IOPS MC system is active, MC service UEs can move around and may enter and leave the IOPS MC system coverage, i.e. the MC service users may switch from the active IOPS MC connectivity function to the MC service server of the primary MC system, and vice versa.

In order to notify the primary MC service server about the active IOPS MC connectivity function, when MC service users register to the primary MC service server after being recently registered to the IOPS MC connectivity function, the MC service users can provide information to the primary MC service server about the active IOPS MC connectivity function and optionally include associated dynamic information. The primary MC service server uses the provided information to become aware of the active IOPS MC connectivity function.

NOTE:	Dynamic information can be, e.g., information about other available MC service users or active MC service groups that the MC service user identified while on the IOPS MC connectivity function. The primary MC service server can use the dynamic information to determine that affiliated MC service users might be registered on the active IOPS MC connectivity function and might not be reachable on the system.

Upon the receipt of the notification, the primary MC service server may notify other MC service users in the proximity of the IOPS MC system coverage about the corresponding active IOPS MC connectivity function. This information can be used by the MC service users to optimize the user experience, e.g. to improve the switching time between systems and to obtain information about the potential availability of other registered MC service users or active MC service groups on the IOPS MC connectivity function.

### 10.7.2	Information flows

#### 10.7.2.1	MC IOPS notification

Table 10.7.2.1-1 describes the information flow MC IOPS notification from the MC service client to the primary MC service server.

Table 10.7.2.1-1: MC IOPS notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the MC service user providing the notification |
| IOPS MC system information | M | Information related to the identified active IOPS MC connectivity function (see NOTE) |
| List of MC service group IDs | O | The list of groups identified by the MC service user as active on the IOPS MC connectivity function |
| List of MC service IDs | O | The list of other users identified by the MC service user as available on the IOPS MC connectivity function |
| NOTE: The IOPS MC system information consists of the following elements: IOPS PLMN ID, server URI of the IOPS MC connectivity function, and location information (set of coordinates including altitude, longitude and latitude, and time of measurement and optional accuracy) related to the MC service user registration on the IOPS MC connectivity function. |  |  |

Table 10.7.2.1-2 describes the information flow MC IOPS notification from the primary MC service server to the MC service client.

Table 10.7.2.1-2: MC IOPS notification

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID (see NOTE 1) | M | The identity of the MC service user receiving the notification |
| IOPS MC system information | M | Information related to the identified active IOPS MC connectivity function (see NOTE 2) |
| List of MC service group IDs (see NOTE 3) | O | The list of MC service groups identified as active on the IOPS MC connectivity function |
| List of MC service IDs (see NOTE 3) | O | The list of MC service users identified as available on the IOPS MC connectivity function |
| NOTE 1: This information element is not included if the notification is transmitted over broadcast/multicast session. NOTE 2: The IOPS MC system information consists of the following elements: server URI of the IOPS MC connectivity function, and location information (set of coordinates including altitude, longitude and latitude) where the IOPS MC connectivity function is identified as active. NOTE 3: The MC service server may provide information about identified active MC service groups or available MC service users on the IOPS MC connectivity function. This information is only included if the MC service user receiving the notification is authorized, e.g. if the MC service user is a member of the corresponding MC service groups. This information element is not included if the notification is transmitted over broadcast/multicast session. |  |  |

### 10.7.3	MC IOPS notification procedure

Figure 10.7.3-1 describes the IOPS MC notification procedure when an MC service user has left an active IOPS MC system and enters the primary MC system.

Pre-conditions:

-	There is an active IOPS MC connectivity function and the neighbouring cells of the IOPS MC system are part of the primary MC system.

-	The MC service user 1 is initially registered to the IOPS MC connectivity function for the support of MC services in the IOPS mode of operation. The MC service user 1 is authorized to provide MC IOPS notifications to the primary MC service server.

-	The MC service user 2 is registered to the primary MC service server and is in the proximity of the IOPS MC system coverage.

-	MC server users 1 and 2 are members of the same MC service group or are authorized to have a one-to-one MC service communication.

Figure 10.7.3-1: MC IOPS notification procedure

1.	An IOPS mode of operation is active, and the MC service communication is handled by the IOPS MC connectivity function. The MC service client 1 is registered to the IOPS MC connectivity function.

2.	The MC service client 1 moves out of the coverage of the IOPS MC system and registers to the primary MC service server.

3.	The MC service client 1 sends a MC IOPS notification to the primary MC service server to provide information about an active IOPS MC connectivity function in the area. This notification includes information about the active IOPS MC connectivity function such as server URI, associated IOPS PLMN ID, and location information. Also, the MC service client 1 may indicate which MC service groups and MC service users were identified as active and available on the IOPS MC connectivity function.

4.	The primary MC service server becomes aware of the active IOPS MC connectivity function and can use the received information to determine that affiliated MC service users might be registered on the active IOPS MC connectivity function and might not be reachable on the system. If the primary MC service server determines that the IOPS MC connectivity function is active and identifies that affiliated MC service users are in the proximity of the IOPS MC system, the primary MC service server may notify the corresponding MC service users about the IOPS MC connectivity function.

NOTE 1:	The primary MC service server can use received information from different MC IOPS notifications (e.g. location information including the time of measurement) and the information obtained from location information subscriptions (as described in 3GPP TS 23.280 [3]) to determine if the IOPS MC connectivity function might be still active. For instance, if information received from location information subscriptions indicates that MC service users are located within the notified active IOPS MC system coverage, the primary MC service server can determine that the IOPS MC connectivity function is no longer active.

5.	If the primary MC service server determines that the IOPS MC connectivity function is active, it sends a MC IOPS notification to the MC service client 2 in proximity of the active IOPS MC system coverage. This information can be used by the MC service user to become aware of the active IOPS MC connectivity function. Hence, the MC service user might decide to not move into the IOPS MC system coverage or to improve the switching time between the systems. Also, the MC service user can be aware of the potential availability of MC service users or active MC service groups on the IOPS MC connectivity function.

NOTE 2:	The MC IOPS notification can be sent on a broadcast/multicast session configured within the proximity of the active IOPS MC system to target multiple MC service users. In this case, information about the active MC service IDs and MC service group IDs is not included in the MC IOPS notification.

* * * End of Changes * * * *