3GPP TSG-SA WG6 Meeting #64	S6-245430

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245066)

Source:	Apple, Telefonica, Lenovo, NCSRD, UMA

Title:	Pseudo-CR on CAPIF vendor extensibility

Spec:	3GPP TR 23.946 v0.7.0

Agenda item:	9.7

Document for:	Approval

Contact:	Walter Featherstone, w_featherstone@apple.com

1. Introduction

This contribution describes the use of the CAPIF vendor extensibility feature.

2. Reason for Change

Vendor extensibility is a CAPIF feature that enables third-party API frameworks to re-use CAPIF functionality, adapted to their requirements, by extending either CAPIF APIs or CAPIF data models. ETSI MEC is used as an example API framework.

In revision 1 changed the ETSI GS MEC 011 referenced version to v3.3.1 (which was published 2024-11). Added further references to specific clauses in the stage 3 specifications. The attribute “id” was changed to “Location” (one the ETSI MEC defined example options) within the example for “category”.  The schema in Table 6.X.4-2 and Table 6.X.4-3 is corrected. The example URIs now use “example.com”. The example values for both “supportedFeatures” and “apiProvName” are updated. Courier New is used for code examples.

4. Proposal

It is proposed to approve this pCR for 3GPP TR 23.946 v0.7.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[3]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[4]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[5]		ETSI GS MEC 011 V3.3.1: "Multi-access Edge Computing (MEC); Edge Platform Application Enablement".

[6]	3GPP TR 23.958: "Edge Application Standards in 3GPP and Alignment with External Organizations".

[7]	EVOLVED-5G ICT-41 project, https://evolved-5g.eu

[8]	NEF emulator version 2.2.3, February 2024. https://github.com/EVOLVED-5G/NEF_emulator

[9]	ETSI Software Development Group: "OpenCAPIF", https://ocf.etsi.org

[10]	ETSI Software Development Group GitLab repository: "OpenCAPIF", https://labs.etsi.org/rep/ocf/capif.

[11]		GSMA PRD OPG.02 - "Operator Platform Telco Edge Requirements Version 6.0", https://www.gsma.com/futurenetworks/wp-content/uploads/2024/02/OPG.02-v6.0-Operator-Platform-Requirements-and-Architecture.pdf.

[12]		OCF Project Postman Collection. https://ocf.etsi.org/documentation/develop/testing/postman/.

[13]		OCF Project Common Operations, https://ocf.etsi.org/documentation/develop/testing/testplan/common_operations/.

[14]		3GPP TS 29.122: "T8 reference point for Northbound APIs".

[15]	IETF RFC 6749 (October 2012): "The OAuth 2.0 Authorization Framework".

[TS29500]		3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

* * * Second Change * * * *

## 6.X	CAPIF Vendor Extensibility

### 6.X.1	General

Vendor extensibility is a CAPIF feature that enables third-party API frameworks to re-use CAPIF functionality, adapted to their requirements, by extending either CAPIF APIs or CAPIF data models. A detailed description of vendor extensibility mechanisms is provided on 3GPP TS 29.122 [14] and 3GPP TS 29.500 [TS29500]. The CAPIF APIs supporting this feature are the following:

- CAPIF_Publish_Service_API, including “VendorExt” option in the supported feature list (see Table 8.1.6-1 of 3GPP TS 29.222 [3], which lists 3 features), indicating the support for CAPIF vendor specific extensions on the ServiceAPIDescription data model.

- CAPIF_Discover_Service_API, including “VendSpecQueryParams” option in the supported feature list (see Table 8.2.6-1 of 3GPP TS 29.222 [3], which lists 9 features), indicating the support of vendor specific query parameters in API discovery requests.

### 6.X.2	Include Vendor Specific information

An API provider can include a vendor specific attribute in the ServiceAPIDescription model by enabling “VendorExt” supported feature option and adding the field in the following manner (see clause 5.2.13.2 of 3GPP TS 29.122 [14]):

"vendorSpecific-<vendor-identification>": {

...

}

where “vendor-identification” represents a unique name of the vendor publishing the API, using one of the following structures:

- IANA-assigned enterprise code (e.g., vendorSpecific-010415 for “3GPP”)

- Domain name (e.g., vendorSpecific-3gpp.org)

- URN (e.g., vendorSpecific-urn:3gpp:example)

### 6.X.3	Query for Vendor Specific information

An API invoker can discover API services including vendor specific attributes by adding the following query parameters (see clause 5.2.13.3 of 3GPP TS 29.122 [14]):

- supported-features,  enabling “VendSpecQueryParams” option

- vend-spec-<query-parameter-name> = {

"target": "...",
"value": "..."

}

where “query-parameter-name” represents the actual name of the query parameter, “target” is a JSON pointer towards the targeted attribute in the targeted resource representation and “value” is the actual value of the query parameter to be used for filtering.

### 6.X.4	ETSI MEC extensibility to CAPIF

According to ETSI GS MEC 011 [5] specification, MEC framework, leveraging CAPIF extensibility feature, can engage CAPIF and re-use its functionalities. A concrete example is presented in tables 6.X.4-1 and 6.X.4-4, describing the request that a MEC platform has to send towards CAPIF to publish an API, as well as a request of an invoker searching an API with MEC specific attributes.

ETSI MEC supports two extensibility attributes on CAPIF:

- “vendorSpecific-urn:etsi:mec:capifext:service-info” in ServiceAPIDescription and ServiceAPIDescriptionPatch data types, and

- “vendorSpecific-urn:etsi:mec:capifext:transport-info” in AefProfile data type

Both attributes are included on the following API Publish request.

Table 6.X.4-1: Publish MEC API with vendor specific attributes in CAPIF (schema)

HTTP POST {apiRoot}/published-apis/v1/{apfId}/service-apis Request body: { "apiName": "string", "aefProfiles": [ { "aefId": "string", "versions": [ { "apiVersion": "string", "expiry": "string", "resources": [ { "resourceName": "string", "commType": "string", "uri": "string", "custOpName": "string", "operations": ["string"], "description": "string" } ], "custOperations": [ { "commType": "string", "custOpName": "string", "operations": [ "string" ], "description": "string" } ] } ], "interfaceDescriptions": [ { "ipv4Addr": "string", "port": "int", "securityMethods": [ "string" ] } ], "vendorSpecific-urn:etsi:mec:capifext:transport-info": { "name": "string", "description": "string", "type": "string", "protocol": "string", "version": "string", "security": { "grantTypes": "string", "tokenEndpoint": "string" } } } ], "vendorSpecific-urn:etsi:mec:capifext:service-info": { "serializer": "string", "state": "string", "scopeOfLocality": "string", "consumedLocalOnly": "boolean", "isLocal": "boolean", "category": { "href": "string", "id": "string", "name": "string", "version": "string" } }, "description": "string", "supportedFeatures": "string", "apiProvName": "string" }

When APIs are published including Vendor Specific information, API Invokers can filter the Discovery of APIs including query parameters to filter by vendor specific information. Here we show some examples of CAPIF Discover API including vendor specific filtering.

The schema for discovering APIs using vendor specific information is shown in Tables 6.X.4-2 and 6.X.4-3:

Table 6.X.4-2: Discover APIs with vendor specific query parameters (schema for transport-info)

HTTP GET {apiRoot}/service-apis/v1/allServiceAPIs? api-invoker-id="string"& supported-features="string"& vend-spec-type={"target": "...", "value": "string"}

Table 6.X.4-3: Discover APIs with vendor specific query parameters (schema for service-info)

HTTP GET {apiRoot}/service-apis/v1/allServiceAPIs? api-invoker-id="string"& supported-features="string"& vend-spec-state={"target": "...", "value": "string"}

Based in these schemas, the following examples show how to use CAPIF Extensibility. Table 6.X.4-4 shows an API Publication that includes both transport-info and service-info as part of the API description published.

Table 6.X.4-4: Publish MEC API with vendor specific attributes in CAPIF (OCF example)

HTTP POST https://capif.mobilesandbox.cloud:37211/published-apis/v1/{apfId}/service-apis Request body: { "apiName": "api_demo_v2", "aefProfiles": [ { "aefId": "AEFb1a522967511fd6d6def0cbb3b5f05", "versions": [ { "apiVersion": "v2", "expiry": "2021-11-30T10:32:02.004Z", "resources": [ { "resourceName": "endpoint_1", "commType": "REQUEST_RESPONSE", "uri": "/endpoint", "custOpName": "string", "operations": [ "GET" ], "description": "Endpoint to receive a welcome message" } ], "custOperations": [ { "commType": "REQUEST_RESPONSE", "custOpName": "string", "operations": [ "GET" ], "description": "string" } ] } ], "interfaceDescriptions": [ { "ipv4Addr": "localhost", "port": 8089, "securityMethods": ["Oauth"] } ], "vendorSpecific-urn:etsi:mec:capifext:transport-info": { "name": "trasport1", "description": "Transport Info 1", "type": "REST_HTTP", "protocol": "HTTP", "version": "2", "security": { "grantTypes": "OAUTH2_CLIENT_CREDENTIALS", "tokenEndpoint": "https://token-endpoint.example.com/" } } } ], "vendorSpecific-urn:etsi:mec:capifext:service-info": { "serializer": "JSON", "state": "ACTIVE", "scopeOfLocality": "MEC_SYSTEM", "consumedLocalOnly": "True", "isLocal": "True", "category": { "href": "https://location.example.com", "id": "Location", "name": "Location", "version": "1.0" } }, "description": "Hello api services", "supportedFeatures": "4", "apiProvName": "OCF_MEC" }

The next example in Table 6.X.4-5 shows how to query CAPIF using transport-info to query for APIs which vend-spec-type is “REST_HTTP”.

Table 6.X.4-5: Discover APIs with vendor specific query parameters (OCF example for transport-info)

HTTP GET https://capif.mobilesandbox.cloud:37211/service-apis/v1/allServiceAPIs? api-invoker-id=INVd4d70e16c52bace2b8f40ed6f6badS& supported-features=4& vend-spec-type={"target": "/vendorSpecific-urn:etsi:mec:capifext:transport-info", "value": "REST_HTTP"}

Similarly, the next example in Table 6.X.4-6 shows how to query CAPIF for APIs that contains service-info which vend-spec-state is “ACTIVE”.

Table 6.X.4-6: Discover APIs with vendor specific query parameters (OCF example for service-info)

HTTP GET https://capif.mobilesandbox.cloud:37211/service-apis/v1/allServiceAPIs? api-invoker-id=INVd4d70e16c52bace2b8f40ed6f6badS& supported-features=4& vend-spec-state={"target": "/vendorSpecific-urn:etsi:mec:capifext:service-info", "value": "ACTIVE"}

* * * End of Changes * * * *