3GPP TSG-SA WG6 Meeting #64	S6-245514

Orlando, USA, 18th – 122nd November 2024	(revision of S6-245188, S6-244618)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0338 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add procedure for exposing Sidelink ranging information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TR 23.700-72, the conclusion for KI#5 specifies solution #7 can be considered for normative work to address exposing ranging information to 3rd parties. This contribution targets adding procedure in SEAL-LM architecture to expose ranging information to 3rd parties. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for exposing Sidelink ranging information (9.3.y) Add information flows for request/response in 9.3.2.a and 9.3.2.b API definition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ranging information is not available to 3rd parties. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.a (new), 9.3.2.b (new), 9.3.y (new), 9.4.1, 9.4.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause 9.3.x refers to procedure described in S6-244617. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.2.a	Short-Range based positioning information request

Table 9.3.2.a-1 describes the information flow from the VAL server to the SEAL LMS for the Short-Range based positioning information request.

Table 9.3.2.a-1: Short-Range based positioning information request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identity | M | Identity of the requestor (e.g. VAL user or VAL UE) |
| VAL service ID | O | Identity of the VAL service (ranging/SL positioning service) for which the ranging/SL positioning information is requested. |
| Requested Short-Range based positioning information | M | Indicates the requested Short-Range based positioning information (e.g. range, direction, relative position, relative velocity, etc.). |
| UE list | M | A list of reference UE identities that have been configured for ranging/SL positioning operation, including client, target, and reference UEs. |
| QoS requirement | O | The QoS requirement for the ranging/SL positioning operation. |
| Reporting events | O | Reporting event triggers as specified in table 9.3.2.4-1. |
| Expiration | O | The expiration for the Short-Range based positioning information request. |

#### 9.3.2.b	Short-Range based positioning information response

Table 9.3.2.b-1 describes the information flow from the SEAL LMS to the VAL server for the Short-Range based positioning information response.

Table 9.3.2.b-1: Short-Range based positioning information response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure code can be provided. |
| Short-Range based positioning information | O (NOTE) | The Short-Range based positioning information associated with locations of UEs within the field of view of the target UE. |
| NOTE: This information element is present when the status for the request is success. |  |  |

* * * Next Change * * * *

### 9.3.y	Short-Range based positioning information

Figure 9.3.y-1 describes a procedure to expose Short-Range based positioning information to third parties.

Pre-condition:

- Target and reference UEs are configured and provisioned for SL positioning management service as described in clause 9.3.x.

Figure 9.3.y-1: Short-Range based positioning information procedure

- A VAL server sends a Short-Range based positioning information request to SEAL LMS to initiate ranging/SL positioning operation for target and reference UEs. The request includes the VAL service identifier, the reference UE list (client, target and reference UEs), the requested Short-Range based positioning information (e.g. range, direction, relative position, relative velocity, etc.), reporting events, QoS requirement, and expiration of the Short-Range based positioning information request.

2.	The SEAL LMS sends a request to the Client UE to initiate the Short-Range based positioning information procedure between the target and reference UEs using the information provided in step 1.

3.	The Client UE initiates the Ranging/SL positioning operation as described in 3GPP TS 23.586 [13] or in off network location reporting as specified in clause 9.5. Either UE-only operation or Network-based operation is performed when using procedure in 3GPP TS 23.586 [13].

4.	The Client UE sends the Short-Range based positioning information results to the SEAL LMS, which uses the Short-Range based positioning informatiuon results to derive a field of view for the target UE. The SEAL LMS uses the range and direction between the target UE and reference UEs to determine UEs that are within a field of view of the target UE.

5.	The SEAL LMS sends a response to the VAL server with the requested Short-Range based positioning information and exposes location information of UEs within a field of view of the target UE.

* * * Next Change * * * *

## 9.4	SEAL APIs for location management

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting Notify_Trigger_Location_Reporting Notify_Adaptive_Configuration | VAL server | Subscribe/Notify |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Update_Location_Info_Subscription | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Update_Location_Monitoring_Subscription | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
|  | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Delete_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Change_Event | SEAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Update_Subscription_VAL_Service_Area_Change_Event | SEAL server |  |
| SS_SRPositioningInformation | SR_Positioning_Information | VAL server | Request/Response |

* * * Next Change * * * *

### 9.4.z	SS_ SRPositioningInformation API

#### 9.4.z.1	General

API description: This API enables a VAL server to obtain Short-range based positioning information from a location management server over LM-S.

#### 9.4.z.2	SR_Positioning_Information operation

API operation name: SR_Positioning_Information

Description: Request Short-range based positioning information for a target UE.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.a

Outputs: Refer subclause 9.3.2.b

See subclause 9.3.y for the details of usage of this API operation.

* * * End of Change * * *