3GPP TSG-SA6 Meeting #64	S6-245538

# Orlando, USA, 18th - 22nd November 2024	(revision of S6-245538)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0226 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement to API invoker Authorization with the Purpose of Data Processing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Apple (UK) Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The solution #14 of 3GPP TR 23.700-22 was concluded to be considered as candidate for normative work for addressing the inclusion of purpose for data processing in the mechanism for API invoker requesting/obtaining authorization information. This CR proposes implementing this solution in 3GPP TS 23.222. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the API invoker obtains resource owner authorization information, more information can be carried, including additional information about the API invoker (such as ASP id, application id) and the purpose of data processing. The CCF will include this purpose of data processing to obtain user consent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the purpose of data processing is not obtained from the resource owner authorization information request, the resource owner function may deny the request. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.31.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented after S6-245401 (218r1), where the new text should be added to step 1 in the procedure (i.e., extending the sentence “The API invoker requests to obtain RO authorization information to invoke the service API exposed by the API exposing function by sending an obtain service API authorization request to the CCF” to “The API invoker requests to obtain RO authorization information to invoke the service API exposed by the API exposing function (AEF) and access information owned by the resource owner at the AEF through the invocation by sending an obtain service API authorization request to the CCF” and adding the new sentence in step one in this CR straight after that). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In revision 1, the finer level service API access requirements and information the API invoker intends to provide to the AEF at service invocation are added in the obtain service API authorization request. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.31.3	Procedure

Figure 8.31.3-1 illustrates the procedure for API invoker obtaining authorization from resource owner.

Pre-conditions:

1.	The resource owner function can communicate with the API invoker.

2.	The service API access requires obtaining authorization from resource owner.

Figure 8.31.3-1: Procedure for API invoker obtaining authorization from resource owner

1.	The API invoker requests to obtain resource owner authorization information to invoke the service API exposed by the API exposing function (AEF) and access information owned by the resource owner at the AEF through the invocation. The authorization function provides the authorization by interacting with the resource owner via the resource owner function. The request contains API invoker information (its identity and optionally application service information such as application service provider and application identifier), the purpose for data processing and any information required for authentication of the API invoker. The request may include resource owner-related finer level service API access requirements (e.g., access per service API operation or access per API service resource) and resource owner-related information the API invoker intends to provide to the AEF at service invocation (i.e., service API data model information elements the API invoker seeks authorization to provide to the AEF).

NOTE:	The detailed procedure to obtain the resource owner's authorization information is specified in TS 33.122 [12].

Editor's Note:		Authorization for finer level service API access and information intended to be provided by the API invoker at invocation, whilst accounting for the provided purpose for data processing, is to be detailed by SA3.

2.	The API invoker sends service API invocation request to the API exposing function with the resource owner authorization information received in step 1.

3.	The API invoker receives the service API invocation response resulting from the service API invocation once the API exposing function has checked whether the API invoker is authorized to invoke that service API based on the authorization information.

* * * End of Changes * * * *