3GPP TSG-SA WG6 Meeting #64	S6-245609

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245154)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on Solution #3 update

Spec:	3GPP TR 23.700-35

Agenda item:	8.9

Document for:	Approval

Contact:	Yang Yanmei (yangyanmei@huawei.com)

1. Introduction

This P-CR is to make some editoria corrections to the TR 23.700-35 V0.1.0.

2. Reason for Change

There are some editoria issues in the clause 8 of TR 23.700-35 V0.1.0.

4. Proposal

It is proposed to agree the following changes to the clause 8 of TR 23.700-35 V0.1.0.

* * * First Change * * * *

8.3	Solution#3 SEALDD APIs improvement

8.3.1	Solution description

This solution is corresponding to Gap#4.2.2-1. In this solution, the SEAL DD services and associated service APIs exposed to VAL are reconstructed per the following principles:

- Each SEAL DD service and associated service APIs serve one interpretable-value-to-application, which serve one potential application requirement.

- The name of the SEAL DD service and the associated service APIs explicity indicate the satified application requirement. This enables Application developers know clearly why the SEAL DD service and the associated service APIs can benefit their applications.

- Each SEAL DD service APIs contains only required and necessary parameters for the services, which enables the service API being used in a simple and easy way.

The 3GPP TS 23.433 [2] should capture following enhancement as this solution proposed.

8.3.1.1 SEALDD regular connection management

8.3.1.1.1	General

The following clauses specify procedures, information flow and APIs for establishing an SEALDD enabled end-to-end connection between VAL client and VAL server. The end-to-end connection is uniquely identified in the SEALDD layer by the SEALDD-UU flow ID. The specific procedures detailed in the subsequent clauses are for cases in which the SEALDD regular connection is used respectively for application signalling, application data delivery initiated by VAL server, and application data delivery initiated based on DD policy.

As for the signalling transmission connection establishment, the SEALDD enabler layer can determine to establish different signalling transmission connection. Considering that many applications have no performance for the application signalling transmission, the best effort signalling transmission connection service is defined. There are also applications like mission critical requiring low latency and reliable performance, accordingly the reliable signalling transmission connection is defined as another SEAL DD service

* * * Next Change * * * *