3GPP TSG-SA WG6 Meeting #64	S6-245731

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245594, S6-245714)

Source:	Ericsson

Title:	Support of AIML Services for Assisting Edge Computing

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

Added descriptions and procedure for support of AIML services for assisting edge computing on execution node selection to TS 23.482.

2. Reason for Change

Edge computing is a distributed computing model that brings computation and data storage closer to the sources of data. Such design can reduce the latency compared to when an application runs on a centralized data centre. Some examples of edge use cases include self-driving cars, autonomous robots, smart equipment data and automated retail, etc. Solution#29 is recommended for normative work on support AIML services for assisting edge computing. See more details in 3GPP TR 23.700-82. This paper proposes to add descriptions and procedure for supporting of AIML services for assisting edge computing on execution node selection to TS 23.482.

3. Conclusions

This paper proposes to add descriptions and procedure for support of AIML services for assisting edge computing on execution node selection to TS 23.482.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.482 V0.3.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[6]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[7]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[8]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[9]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[10]	3GPP TS 22.261: "Service requirements for the 5G system".

[11]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[X1]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

* * * Next Change * * * *

## 8.x	Support of AIML Services for Assisting Hierarchical Computing

This clause describes procedure for supporting AIML services for assisting hierarchical computing.

### 8.x.1	General

This clause describes the procedure for assisting hierarchical computing by the AIMLE server. A entity (e.g. CAS or EAS which are defined in 3GPP TS 23.558 [X1]) can have different roles in hiearchitical computing (with one root node (e.g., CAS, EAS), the root has one or more children which are also known as sub-root node(s) (e.g., EAS), and multiple leaf nodes (e.g. EAS) with no children). Here, hierarchical computing represents a computation architecture with multiple computation entities involved and multiple levels of computations for a computation task.

### 8.x.2	Procedure for assisting hierarchical computing process

Figure 8.x.2-1 illustrates the procedures for AIMLE server to assist a hierarchical computing process.

Pre-conditions:

-	An AI/ML task be treated as a specical computing task being completed at consumer (e.g., CAS, EAS).

-	The consumer decides its role in the hierarchical computing architecture for an AI/ML task (e.g. FL training) based on its local configuration.

-	The AIMLE server can assist a hierarchical computing process by providing time window(s) recommendation for computing task distribution if the consumer is a root node in a hierarchical computing process, providing time window(s) recommendation for intermediate output delivery if the consumer is a leaf node in a hierarchical computing process, and candidate execution node list provisioning or computing preparation status provisioning, etc.

-	The consumer decides that assistance from AIMLE server to support the hierarchical computing process (AI/ML task) is needed, due to lack of capability on e.g. execution node selection.

Figure 8.x.2-1: Procedure for assist a hiearchitical computing process

Figure 8.x.2-1 illustrates the procedure for AIMLE servers assist a hiearchitical computing process. The corresponding procedure in detail is as follows:

- The VAL server (e.g. CAS, EAS) sends hiearchitical computing assistance request to the edge AIMLE server for a hiearchitical computing process (AI/ML task). The request message includes information as described in Table 8.x.3.1-1.

- The edge AIMLE server authenticates and authorizes the request from the consumer, and checks its capability to provide the requested assistance information.

If the request is authorized and the edge AIMLE server can generate the required assistance information (e.g. computing preparation status at an execution node which is registratered to it), it performs step 3 and 4.

If the request is authorized but the edge AIMLE server cannot generate the required assistance information (e.g. execution nodes are registratered to different edge AIMLE servers), it performs step 5 and skips steps 3 and 4.

- The edge AIMLE server performs operations to generate assistance information which is requested in step 1. For example, for computing preparation status at an execution node, the edge AIMLE server may subscribe/request analytics from ADAE server (e.g. edge load analytics, edge computing preparation analytics) and aggregate the information received to generate assistance information.

- The edge AIMLE server sends hiearchitical computing assistance response to the consumer (e.g. computing preparation status at the execution node). The response message contains the information as described in Table 8.x.3.2-1.

- If the edge AIMLE server cannot generate the required assistance information, it sends hiearchitical computing assistance request to central AIMLE server. The request message includes information as described in Table 8.x.3.1-1.

- The central AIMLE server authenticates and authorizes the request from the edge AIMLE server.

- If the request is authorized, the central AIMLE server performs operations to generate assistance information which is requested in step 1.

For example, for execution node selection, according to the role of the VAL server in request message in step 1, the central AIMLE server derives the requirements on the execution node (e.g. high computation capability, high communication capability, or both). Then, the central AIMLE server may subscribe/request analytics from ADAE server (e.g. edge load analytics, edge computing preparation analytics), trigger generation of split operation assistance information, retrieve FL member information from ML Repository (the FL member with EAS ID). The existing services can be reused for execution node selection, e.g., the step 3 in clause 8.12.2.

- The central AIMLE server sends hiearchitical computing assistance response with the generated assistance information (e.g. a list of selected candidate execution nodes). The response message contains the information as described in Table 8.x.3.2-1.

8a.	The central AIMLE server sends the hiearchitical computing assistance response to the edge AIMLE server.

8b.	The edge AIMLE server sends the hiearchitical computing assistance response to the consumer.

The VAL server (e.g. CAS, EAS) uses the assistance information for decision making on its computing operations.

NOTE:	If the VAL server (e.g. CAS, EAS) know the information of the central AIMLE server, it can send the hiearchitical computing assistance request to the central AIMLE server directly for the hiearchitical computing process (AI/ML task). The procedure is the same as the steps 5-8a in Figure 8.x.2-1 with replacing edge AIMLE server by consumer.

* * * Next Change * * * *

### 8.x.3	Information flows

#### 8.x.3.1	Hiearchitical computing assistance request

Table 8.x.3.1-1 shows the request sent by VAL server (e.g. CAS, EAS) to the AIMLE server for assistance of a hiearchitical computing process (AI/ML task).

Table 8.x.3.1-1: Hiearchitical computing assistance request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Original requestor identifier | O | The identifier of the original requestor, e.g. VAL server ID, EAS ID, CAS ID. |
| Role | M | Represents the role of the VAL server in a hiearchitical computing architecture (e.g. root node, sub-root node or leaf node of a hierarchical computing process). |
| Computing task type | M | The type of computing task (e.g. VFL, HFL). |
| Assistance information type | M | Represents the assistance information type, which is used to indicate the assistance information needed, e.g. candidate execution node list, computing preparation status at an execution node. |
| Execution node(s) | O | Represent one execution node or a list of candidate execution nodes. |

#### 8.x.3.2	Assist hierarchical computing response

Table 8.x.3.2-1 shows the response sent by AIMLE server to the VAL server (e.g. CAS, EAS) for assisting the hierarchical computing process.

Table 8.x.3.2-1: Hierarchical computing assistance response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE 1) | Indicates that the assist hierarchical computing request was successful. |
| >Assistance informaiton | M | The assistance informaiton for assisting hierarchical computing process, e.g. candidate execution node list, computing preparation status at an execution node. |
| >>List of candidate execution nodes | O (NOTE 2) | A list of selected candidate execution nodes. |
| >>Preparation status | O (NOTE 2) | Computing preparation status at the execution node provided in request. |
| Failure response | O (NOTE 1) | Indicates that the assist hierarchical computing request was failure. |
| >Cause | M | Reason for the failure. |
| NOTE 1: One of the IEs shall be present. NOTE 2: At least one of the IEs shall be present. |  |  |

* * * Next Change * * * *

### 9.2.Y	AIMLE Assistance of Hierarchical Computing API

### 9.2.Y.1	General

Table 9.2.Y.1-1 illustrates the API for assist hierarchical computing. This API enables the communication between VAL server (e.g. CAS, EAS) and the AIMLE server for assistance of a hierarchical computing process.

Table 9.2.Y.1-1: Aimles_HierarchicalComputingAssist API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Aimles_HierarchicalComputingAssist | Request | Request/Response | VAL Server (e.g. CAS, EAS) |

### 9.2.Y.2	Aimles_HierarchicalComputingAssist_Request operation

API operation name: Aimles_HierarchicalComputingAssist_Request

Description: The consumer requests for assistance of a hierarchical computing process.

Inputs: See clause 8.x.3.1.

Outputs: See clause 8.x.3.2.

See clause 8.x.2 for details of usage of this operation.

# *** End of Changes ***