3GPP TSG-SA WG6 Meeting #64	S6-245741

Orlando, USA, 18th – 122nd November 2024	(revision of S6-245629,S6-245230)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0049 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to edge load analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_ADAES |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specific content of the analytics output in Table 8.8.3.7-1 and Table 8.8.3.9-1 for edge load analytics is unclear. The definition of clear outputs is needed to complete the analytics documentation and avoid implementation problems. This contribution proposes to update the outputs of the edge load analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Updated the IEs in the tables for analytics outputs. - Corrected the title and description of Table 8.8.3.9-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete output definition for edge load analytics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.3.2, 8.8.3.7, 8.8.3.8, 8.8.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.8.3.2	Edge analytics subscription request

Table 8.8.3.2-1 describes information elements for the edge analytics subscription request from the VAL server / Consumer to the ADAE server.

Table 8.8.3.2-1: Edge analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, EAS). |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “Edge platform analytics”, “EES analytics”, “EAS analytics”. |
| Analytics type | M | The type of analytics for the event, e.g. statistics or predictions. |
| Destination EAS information | O (NOTE) | This identifier shows the destination EAS information including destination EAS ID and destination EAS endpoint for which the analytics subscription applies. |
| Destination EES information | O (NOTE) | This identifier shows the destination EES information including destination EES ID and destination EES endpoint for which the analytics subscription applies. |
| DNN/DNAI | O (NOTE) | DNN or DNAIs information for which the subscription applies. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the subscription request. |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |
| NOTE: At least one of these shall be present. |  |  |

* * * Next Change * * * *

#### 8.8.3.7	Edge analytics Notification

Table 8.8.3.7-1 describes information elements for the Edge analytics Notification from the ADAE server to the VAL server / Consumer.

Table 8.8.3.7-1: Edge analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| Analytics Output | M | The analytics outputs, which can be predictive or statistical parameter. |
| > Edge platform analytics | O (see NOTE) | Statistics/predictions of the performance/load of the EDN or the associated edge platform, such as the number of EAS or EES connections. |
| >> Applicable time period | O | The time period that the analytics applies to. |
| >> Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| > EES analytics | O (see NOTE) | Statistics/predictions of the performance/load of the EES, such as the EES unavailability. |
| >> Applicable time period | O | The time period that the analytics applies to. |
| >> Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| > EAS analytics | O (see NOTE) | Statistics/predictions of the performance/load of the EAS, such as the EAS unavailability. |
| >> Applicable time period | O | The time period that the analytics applies to. |
| >> Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| NOTE: At least one of these information elements shall be provided based on the analytics ID specified in the edge analytics subscription request as in Table 8.8.3.2-1. |  |  |

* * * Next Change * * * *

#### 8.8.3.8	Get analytics data request

Table 8.8.3.8-1 describes information elements for the Get analytics data request from the analytics consumer to the ADAE server.

Table 8.8.3.8-1: Get analytics data request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, EAS, EES). |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “Edge platform analytics”, “EES analytics”, “EAS analytics”. |
| Analytics type | M | The type of analytics, e.g. statistics or predictions. |
| Destination EASs information | O (NOTE) | This identifier provides the list of destination EASs information including destination EAS ID and destination EAS endpoint for which the analytics request applies. |
| Destination EESs information | O (NOTE) | This identifier provides the list of destination EESs information including destination EES ID and destination EES endpoint for which the analytics request applies. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Time duration | O | Time duration since when analytics data is required. |
| NOTE: At least one of these shall be present |  |  |

#### 8.8.3.9	Get analytics data response

Table 8.8.3.9-1 describes information elements for the Get analytics dataresponse from the ADAE server to the consumer.

Table 8.8.3.9-1: Get analytics dataresponse

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics data request (positive or negative acknowledgement). |
| Analytics ID | O | The identifier of the analytics event. |
| Analytics Output | O | The analytics outputs, which can be predictive or statistical parameter. |
| > Edge platform analytics | O (see NOTE) | Statistics/predictions of the performance/load of the EDN or the associated edge platform, such as the number of EAS or EES connections. |
| >> Applicable time period | O | The time period that the analytics applies to. |
| >> Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| > EES analytics | O (see NOTE) | Statistics/predictions of the performance/load of the EES, such as the EES unavailability. |
| >> Applicable time period | O | The time period that the analytics applies to. |
| >> Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| > EAS analytics | O (see NOTE) | Statistics/predictions of the performance/load of the EAS, such as the EAS unavailability. |
| >> Applicable time period | O | The time period that the analytics applies to. |
| >> Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| NOTE: At least one of these information elements shall be provided based on the analytics ID specified in the get analytics data request as in Table 8.8.3.8-1. |  |  |

* * * End of Changes * * * *