3GPP TSG-SA WG6 Meeting #63	S6-244693

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244632, S6-244126)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0086 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-modal SEALDD flow identifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2024-10-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Multi-modal SEALDD flow identifier is missing and is needed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for multi-modal SEALDD flow identifier. Multi-modal SEALDD flow based operations to 5G CN and between SEALDD client and server require a multi-modal SEALDD flow identifier |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification of multi-modal SEALDD flow is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8, 9.12.2.1.2, 9.12.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 8	Identities and commonly used values

## 8.1	General

The common identities for SEAL refer to 3GPP TS 23.434 [4]. The following clauses list the additional identities and commonly used values for SEALDD.

## 8.2	SEALDD server ID

The SEALDD server ID uniquely identifies the SEAL data delivery server.

## 8.3	SEALDD client ID

The SEALDD client ID is a globally unique value that identifies the SEAL data delivery client.

## 8.4	SEALDD-UU flow ID

The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different VAL application traffic, which has the same attributes, e.g. the same 5-tuple, media type, QoS requirements. The SEALDD-UU flow ID should be uniquely identified with the SEALDD-UU connection.

## 8.5	Multi-modal SEALDD-UU flow ID

The multi-modal SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to uniquely identify associated multi-modal VAL application traffic flows per VAL service.

* * * Next Change * * * *

##### 9.12.2.1.2	SEALDD enabled XR data transmission establishment

Figure 9.12.2.1.2-1 illustrate the procedure for establishing XR data transmission connection, and the SEALDD facilitates the XR application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

Figure 9.12.2.1.2-1: SEALDD enabled XR data transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for XR traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_XRTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements.

2.	Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF.

3-5.	Same as step 3-5 of clause 9.2.2.2.

6.	The SEALDD client establishes XR transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the XR transmission connection response. The response also includes a multi-modal SEALDD-UU flow ID which the SEALDD server allocates.

NOTE:	The multi-modal SEALDD-UU flow ID is used by the SEALDD client and server to minimize the exchange of multi-modal flow information (e.g., traffic descriptors) during subsequent multi-modal operations (e.g., multi-modal SEALDD measurements).

7-8.	Same as step 8-9 of clause 9.2.2.2.

The XR application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP).

* * * Next Change * * * *

#### 9.12.3.4	SEALDD XR transmission connection establishment response

Table 9.12.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the XR transmission connection establishment.

Table 9.12.3.4-1: SEALDD XR transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| Multi-modal SEALDD flow ID | M | Identity of the Multi-modal SEALDD-UU flow associated with the multi-modal XR connection. |
| SEALDD traffic descriptors | O | SEALDD traffic descriptors for multiple flows (e.g. address(s), port(s), transport layer protocol) of the SEALDD server side used to establish SEALDD multi-modal connection. |
| Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP. |
| Cause | O (See NOTE) | Indicates the reason for the failure |
| NOTE: The IE is only present if the Result is failure. |  |  |

* * * End of Changes * * * *