3GPP TSG-SA WG6 Meeting #63	S6-244702

Hyderabad, India, 14th – 18th October 2024	(revision of S6-244622, 4256)

Source:	Huawei, Hisilicon

Title:	Clarification on signalling transmission connection establishment

Spec:	3GPP TS 23.700-35 v0.0.0

Agenda item:	8.9

Document for:	Approval

Contact:	Yajie Hu (huyajie1@huawei.com)

1. Introduction

Currently, the API defined in SEALDD can be improved to expose a better service to the 3rd party, thus this contribution is proposed to provide Key issue and corresponding solution.

2. Reason for Change

Currently, the API defined in SEALDD can be improved to expose a better service to the 3rd party. The improvement can contains following aspect:

1.	The functions and values supported by APIs defined in SEALDD need to be further improved to make it easier for third parties to understand the functions and values of APIs. In this case, third parties can easily understand the functions and values of each API and select appropriate APIs.

2.	The functions supported by APIs need to be further simplified and clarified. That is, it is recommended that an API support only one function. In this way, an API does not corresponding too many optional unnecessary parameters. This makes it easier for 3rd parties to use the API.

Thus the APIs defined in SEALDD can be further improved.

3. Proposal

It is proposed to agree the following pCR to TR 23.700-35.

* * * First Change * * * *

4.x.	<Exsiting SEAL service X>

4.x.1	Analysis

Currently, the API defined in SEALDD can be improved to expose a better service to the 3rd party. The improvement can contains following aspect:

- The functions and values supported by APIs defined in SEALDD need to be further improved to make it easier for third parties to understand the functions and values of APIs. In this case, third parties can easily understand the functions and values of each API and select appropriate APIs.

- The functions supported by APIs need to be further simplified and clarified. That is, it is recommended that an API support only one function. In this way, an API does not corresponding too many optional unnecessary parameters. This makes it easier for 3rd parties to use the API.

4.x.2	Identified gaps

Currently, the API defined in SEALDD is not friendly to vertical application:

- The functions supported by APIs are should be further improved for 3rd party to understand.

- The APIs need to be further simplified and clarified to make it easier for 3rd party to use.

8.X	Solution#X.1

The 3GPP TS 23.433 should capture following enhancement as this solution proposed.

9.x	SEALDD regular connection management

9.x.1	General

The following clauses specify procedures, information flow and APIs for establishing an SEALDD enabled end-to-end connection between VAL client and VAL server. The end-to-end connection is uniquely identified in the SEALDD layer by the SEALDD-UU flow ID. The specific procedures detailed in the subsequent clauses are for cases in which the SEALDD regular connection is used respectively for application signalling, application data delivery initiated by VAL server, and application data delivery initiated based on DD policy.

As for the signalling transmission connection establishment, the SEALDD enabler layer can determine to establish different signalling transmission connection (i.e. best effort signalling transmission connection, reliable signanlling transmission connection) for the VAL server based on the signalling transmission requirement.

9.x.2	Procedure

9.x.2.1	SEALDD enabled best effort signalling transmission connection establishment procedure

Figure 9.x.2.1-1 illustrate the procedure for best effort signalling transmission connection establishment. In this clause, considering VAL server’s requirement on signalling transmission is not strict, then the VAL server can invoke best effort signalling transmission connection establishment API to enable the connection establishment.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

Figure 9.x.2.1-1: SEALDD best effort signalling transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for application signalling transfer and allocates address as SEALDD-S connection information for receiving the application signalling packets from SEALDD server. The VAL server sends SEALDD enabled best effort signalling transmission connection establishment request to the SEALDD server. The SEALDD enabled best effort signalling transmission connection establishment request includes the VAL server ID, VAL service ID to identify the VAL application traffic, the SEALDD-S Data transmission connection information of the VAL server side.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server allocates a specific address used for SEALDD traffic transfer with the incoming SEALDD client(s) for the VAL server and responds with a SEALDD enabled best effort signalling transmission connection establishment response.

Step3-step8 are same as 3GPP TS 23.433 clause 9.2.2.1 step 2-step8.

9.x.3.1	SEALDD enabled best effort signalling transmission connection establishment request

Table 9.x.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the best effort application signalling traffic transmission service.

Table 9.2.3.1-1: SEALDD enabled Best effort transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| SEALDD-S connection information | M | Address information (e.g., IP address and/or port, URL) of the VAL server to receive the signalling traffic from the SEALDD server |
| Requested 5GS Delay | O | The Requested 5GS Delay provided by the VAL server |

9.x.3.2	SEALDD enabled best effort transmission connection establishment response

Table 9.x.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the best effort application signalling traffic transmission.

Table 9.2.3.2-1: SEALDD enabled best effort transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S connection information | O | Address information (e.g., IP address and/or port, URL) of the SEALDD server to receive the signalling traffic from the VAL server |
| Cause | O See NOTE | Indicates the reason for the failure, e.g. SEALDD policy mismatch. |
| NOTE: The IE is only present if the Result is failure. |  |  |

9.x.3.3	SEALDD signalling transmission connection establishment initiated by SEALDD client request

Table 9.x.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the  SEALDD signalling transmission connection establishment.

Table 9.x.3.3-1: SEALDD signalling transmission connection establishment initiated by SEALDD client request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (SEALDD client or SEALDD server). |
| SEALDD-UU flow ID | M (See NOTE 1) | Identity of the SEALDD-UU flow. |
| VAL server ID | O | Identity of the VAL server, applicable for SEALDD client side initiated request. |
| VAL service ID | O | Identity of the VAL service |
| Selected VAL server endpoint | M | Endpoint of the selected VAL server |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD client side (for client side initiated request) |
| Identity | O | The VAL user ID of the VAL user or VAL UE ID. |
| SEALDD communication lifetime | O | Identifies the DD communication lifetime, applicable for SEALDD server side initiated request. |
| NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. |  |  |

9.x.3.4	SEALDD regular signalling transmission connection establishment initiated by SEALDD client response

Table 9.x.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responding to the SEALDD signalling transmission connection establishment nitiated by SEALDD client.

Table 9.x.3.4-1: SEALDD signalling transmission connection establishment initiated by SEALDD client response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of establishing the SEALDD connection. |
| SEALDD traffic descriptor | O | SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD server side |
| Pending timer | O (See NOTE 1) | The pending timer to trigger the re-connection from SEALDD client when bandwidth limit check is failed. |
| Cause | O (See NOTE 2) | Indicates the reason for the failure |
| NOTE 1: These IEs are used for the SEALDD enabled bandwidth control for different VAL users, applicable for client side initiated request. NOTE 2: This IE is only present if the Result is failure |  |  |

* * * END of Change * * * *