| 3GPP TS 36.133 V17.12.2 (2023-12) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	94

1	Scope	96

2	References	96

3	Definitions, symbols and abbreviations	98

3.1	Definitions	98

3.2	Symbols	100

3.3	Abbreviations	101

3.4	Test tolerances	104

3.5	Additional notation	104

3.5.1	Groups of bands	104

3.6	General	106

3.6.1	Applicability of requirements in this specification version	106

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	112

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	114

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	115

3.6.2	Applicability of requirements for EN-DC operation	116

3.6.3	Applicability of requirements for NE-DC operation	117

3.6.4	Applicability of requirements for NGEN-DC operation	118

4	E-UTRAN RRC_IDLE state mobility	118

4.1	Cell Selection	118

4.2	Cell Re-selection	118

4.2.1	Introduction	118

4.2.2	Requirements	119

4.2.2.1	Measurement and evaluation of serving cell	120

4.2.2.2	Void	121

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	121

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	123

4.2.2.5	Measurements of inter-RAT cells	125

4.2.2.5.1	Measurements of UTRAN FDD cells	125

4.2.2.5.2	Measurements of UTRAN TDD cells	127

4.2.2.5.3	Measurements of GSM cells	128

4.2.2.5.4	Measurements of HRPD cells	129

4.2.2.5.5	Measurements of cdma2000 1X	130

4.2.2.5.6	Measurements of NR cells	131

4.2.2.5.7	Measurements of NR cells subject to CCA	134

4.2.2.5.8	Measurements of NR cells for RedCap	136

4.2.2.6	Evaluation of cell re-selection criteria	138

4.2.2.7	Maximum interruption in paging reception	138

4.2.2.8	void	139

4.2.2.9	UE measurement capability	139

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	140

4.2.2.10	Reselection to CSG cells	140

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	141

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	141

4.2.2.11	Void	142

4.2.2.12	Void	142

4.2.2.13	Void	142

4.2.2.14	UE measurement capability for RedCap	142

4.3	Minimization of Drive Tests (MDT)	143

4.3.1	Introduction	143

4.3.2	Measurements	143

4.3.2.1	Requirements	143

4.3.3	Relative Time Stamp Accuracy	143

4.3.3.1	Requirements	143

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	144

4.3.4.1	 Requirements	144

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	144

4.3.5.1	Requirements for timeSinceFailure	144

4.4	MBSFN Measurements	144

4.4.1	Introduction	144

4.4.2	MBSFN RSRP measurements	144

4.4.3	MBSFN RSRQ measurements	145

4.4.4	MCH BLER measurements	145

4.5	Proximity-based Services	145

4.5.1	Introduction	145

4.5.2	Requirements	145

4.5.2.1	Interruptions with ProSe Direct Discovery	145

4.5.2.2	Interruptions with ProSe Direct Communication	145

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	146

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	146

4.6	Cell Selection and Re-selection Requirements for UE category NB1	147

4.6.1	Cell Selection	147

4.6.2	Cell Re-selection	147

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	147

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	148

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	149

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	150

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	152

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	153

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	154

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	155

4.6.2.7	Maximum interruption in paging reception in normal coverage	157

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	157

4.6.2.8	UE measurement capability	157

4.6.2.9	WUS receptions for NB1	157

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	158

4.6.3.1	Introduction	158

4.6.3.2	Requirements on UE synchronization for transmission using PUR	158

4.6.3.3	Requirements on TA validation for transmission using PUR	158

4.7	Cell Selection and Re-selection Requirements for UE category M1	159

4.7.1	Cell Selection	159

4.7.2	Cell Re-selection	159

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	159

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	159

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	160

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	161

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	163

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	164

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	165

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	165

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	165

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	166

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	167

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	170

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	171

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	172

4.7.2.3	WUS receptions for UE category M1	172

4.7.3	Channel quality report for UE Category M1 in idle mode	172

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	173

4.7.4.1	Introduction	173

4.7.4.2	Requirements on UE synchronization for transmission using PUR	173

4.7.4.3	Requirements on TA validation for transmission using PUR	173

4.8	Idle State Positioning Measurement Requirements for UE category NB1	174

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	174

4.8.1.1	RSTD Measurement Reporting Delay	176

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	176

4.8.2.1	RSTD Measurement Reporting Delay	178

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	178

4.8.3.1	RSTD Measurement Reporting Delay	179

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	180

4.8.4.1	RSTD Measurement Reporting Delay	181

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	182

4.8.5.1	Measurement Reporting Delay	183

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	183

4.8.6.1	Measurement Reporting Delay	184

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	185

4.8.7.1	Measurement Reporting Delay	186

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	186

4.8.8.1	Measurement Reporting Delay	187

4.9	Idle Mode CA Measurement	188

4.9.1	Introduction	188

4.9.2	Requirement	188

4.9.2.1	Detected cell requirement during state transition and Idle mode	188

4.9.2.2	Measurements of inter-frequency CA candidate cells	189

4.9.2.3	Measurements on serving cell	189

4A	E-UTRAN RRC_INACTIVE state mobility	190

4A.1	Cell Re-selection	190

4A.1.1	Introduction	191

4A.1.2	Requirements	191

4A.1.2.1	UE measurement capability	191

4A.1.2.2	Measurement and evaluation of serving cell	191

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	191

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	191

4A.1.2.5	Evaluation of cell re-selection criteria	191

4A.1.2.6	Maximum interruption in paging reception	191

4A.1.2.7	Measurements of inter-RAT NR cells	191

4A.1.2.8	UE measurement capability for RedCap	191

4A.1.2.9	Measurements of inter-RAT NR cells for RedCap	191

4A.2	Requirements for UE Category M1	192

4A.2.1	Introduction	192

4A.2.2	Cell Selection	192

4A.2.3	Cell Reselection	192

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	192

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	192

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	193

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	193

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	193

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	193

4A.2.4	Channel quality report for UE Category M1 in idle mode	195

5	E-UTRAN RRC_CONNECTED state mobility	195

5.1	E-UTRAN Handover	196

5.1.1	Introduction	196

5.1.2	Requirements	196

5.1.2.1	E-UTRAN FDD – FDD	196

5.1.2.1.1	Handover delay	196

5.1.2.1.2	Interruption time	196

5.1.2.2	E-UTRAN FDD – TDD	197

5.1.2.2.1	(Void)	198

5.1.2.2.2	(Void)	198

5.1.2.3	E-UTRAN TDD – FDD	198

5.1.2.3.1	(Void)	198

5.1.2.3.2	(Void)	198

5.1.2.4	E-UTRAN TDD – TDD	198

5.1.2.4.1	Handover delay	198

5.1.2.4.2	Interruption time	198

5.1.2.5	E-UTRAN HD–FDD	200

5.1.2.5.1	Handover delay	200

5.1.2.5.2	Interruption time	200

5.1.2.6	E-UTRAN FDD – FDD conditional handover	201

5.1.2.6.1	Handover delay	201

5.1.2.6.2	Measurement time	202

5.1.2.6.3	Preparation time	202

5.1.2.6.4	Interruption time	202

5.1.2.7	E-UTRAN FDD – TDD conditional handover	202

5.1.2.8	E-UTRAN TDD – FDD conditional handover	203

5.1.2.9	E-UTRAN TDD – TDD conditional handover	203

5.2	Void	203

5.3	Handover to other RATs	203

5.3.1	E-UTRAN - UTRAN FDD Handover	203

5.3.1.1	Introduction	203

5.3.1.1.1	Handover delay	203

5.3.1.1.2	Interruption time	203

5.3.2	E-UTRAN - UTRAN TDD Handover	204

5.3.2.1	Introduction	204

5.3.2.2	Requirements	204

5.3.2.2.1	Handover delay	204

5.3.2.2.2	Interruption time	204

5.3.3	E-UTRAN - GSM Handover	205

5.3.3.1	Introduction	205

5.3.3.2	Requirements	205

5.3.3.2.1	Handover delay	205

5.3.3.2.2	Interruption time	205

5.3.4	E-UTRAN - NR FR1 Handover	205

5.3.4.1	Introduction	205

5.3.4.2	Handover delay	206

5.3.4.3	Interruption time	206

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	206

5.3.4A.1	Introduction	206

5.3.4A.2	Handover delay	207

5.3.4A.3	Interruption time	207

5.3.4B	E-UTRAN - NR FR1 Handover for RedCap	208

5.3.4B.1	Introduction	208

5.3.4B.2	Requirements	208

5.3.5	E-UTRAN - NR FR2 Handover	208

5.3.5.1	Introduction	208

5.3.5.2	Handover delay	208

5.3.5.3	Interruption time	208

5.4	Handover to Non-3GPP RATs	209

5.4.1	E-UTRAN – HRPD Handover	209

5.4.1.1	Introduction	209

5.4.1.1.1	Handover delay	209

5.4.1.1.2	Interruption time	209

5.4.2	E-UTRAN – cdma2000 1X Handover	210

5.4.2.1	Introduction	210

5.4.2.1.1	Handover delay	210

5.4.2.1.2	Interruption time	210

5.5	E-UTRAN Handover for Cat-M1 UEs	210

5.5.1	Introduction	210

5.5.2	Requirements in CEModeA	211

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	211

5.5.2.1.1	Handover delay	211

5.5.2.1.2	Interruption time	211

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	211

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	212

5.5.2.3.1	Void	212

5.5.2.3.2	Void	212

5.5.3	Requirements in CEModeB	212

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	212

5.5.3.1.1	Handover delay	212

5.5.3.1.2	Interruption time	212

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	213

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	213

5.6	Void	213

5.7	E-UTRAN DAPS Handover	213

5.7.1	Introduction	213

5.7.2	Requirements	213

5.7.2.1	E-UTRAN FDD – FDD	213

5.7.2.1.1	DAPS Handover delay	213

5.7.2.1.2	Interruption time	214

5.7.2.2	E-UTRAN FDD – TDD	214

5.7.2.3	E-UTRAN TDD – FDD	215

5.7.2.4	E-UTRAN TDD – TDD	215

5.8	EN-DC Handover with PSCell	215

5.8.1	Introduction	215

5.8.1.1	Handover with PSCell Interruption time	215

5.8.1.2	Handover with PSCell - NR PSCell Changing Delay requirements	215

5.9	EN-DC Handover with PSCell using CCA	216

5.9.1	Introduction	216

5.9.1.1	Handover with PSCell – E-UTRA HO Interruption time	217

5.9.1.2	Handover with PSCell - NR PSCell Change Delay requirements	217

6	RRC Connection Mobility Control	218

6.1	RRC Re-establishment	218

6.1.1	Introduction	218

6.1.2	Requirements	218

6.1.2.1	UE Re-establishment delay requirement	218

6.2	Random Access	219

6.2.1	Introduction	219

6.2.2	Requirements	219

6.2.2.1	Contention based random access	219

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	219

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	219

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	219

6.2.2.1.4	Void	219

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	219

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	220

6.2.2.2	Non-Contention based random access	220

6.2.2.2.1	Correct behaviour when receiving Random Access Response	220

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	220

6.2.3	Requirements for Cat-M1 UEs	220

6.3	RRC Connection Release with Redirection	220

6.3.1	Introduction	220

6.3.2	Requirements	220

6.3.2.1	RRC connection release with redirection to UTRAN FDD	220

6.3.2.2	RRC connection release with redirection to GERAN	221

6.3.2.3	RRC connection release with redirection to UTRAN TDD	221

6.3.2.4	RRC connection release with redirection to NR	222

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	222

6.3.2.6	RRC connection release with redirection to NR Redcap	224

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	225

6.4.1	Introduction	225

6.4.2	Requirements	225

6.5	RRC Re-establishment for NB-IoT UEs	225

6.5.1	Introduction	225

6.5.2	Requirements	225

6.5.2.1	UE Re-establishment delay requirement in normal coverage	225

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	226

6.6	Random Access for UE category NB1	226

6.6.1	Introduction	226

6.6.2	Requirements	227

6.6.2.1	Correct behaviour when receiving Random Access Response reception	227

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	227

6.6.2.3	Correct behaviour when receiving a NACK on msg3	227

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	227

6.6.2.5	Correct behaviour when contention Resolution timer expires	227

6.6.2.6	MSG3-based channel quality report for UE Category NB1	227

6.6.3	Requirements for NPRACH configuration	228

6.7	RRC Re-establishment for Cat-M1 UEs	228

6.7.1	Introduction	228

6.7.2	Requirements	228

6.7.2.1	UE Re-establishment delay requirement for CEModeA	228

6.7.2.2	UE Re-establishment delay requirement for CEModeB	229

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	229

6.8.1	Introduction	229

6.8.2	Requirements	230

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	230

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	230

6.9.1	Introduction	230

6.9.2	Requirements	230

7	Timing and signalling characteristics	231

7.1	UE transmit timing	231

7.1.1	Introduction	231

7.1.2	Requirements	232

7.2	UE timer accuracy	233

7.2.1	Introduction	233

7.2.2	Requirements	233

7.3	Timing Advance	233

7.3.1	Introduction	233

7.3.2	Requirements	233

7.3.2.1	Timing Advance adjustment delay	233

7.3.2.2	Timing Advance adjustment accuracy	234

7.4	Cell phase synchronization accuracy (TDD)	234

7.4.1	Definition	234

7.4.2	Minimum requirements	234

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	235

7.5.1	Introduction	235

7.5.2	eNodeB Synchronization Requirements	235

7.5.2.1	Synchronized E-UTRAN	235

7.5.2.2	Non-Synchronized E-UTRAN	235

7.6	Radio Link Monitoring	235

7.6.1	Introduction	235

7.6.2	Requirements	237

7.6.2.1	Minimum requirement when no DRX is used	237

7.6.2.2	Minimum requirement when DRX is used	237

7.6.2.3	Minimum requirement at transitions	239

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	239

7.6.2.5	Minimum requirement under IDC Interference	239

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	239

7.7.1	Introduction	239

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	240

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	241

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	242

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	243

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	244

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	245

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	246

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	246

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	246

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	247

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	248

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	249

7.7.14	SCell Activation Delay Requirement for Dormant SCell	249

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	251

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	251

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	253

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	253

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	255

7.8	Interruptions with Carrier Aggregation	257

7.8.1	Introduction	257

7.8.2	Requirements	257

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	257

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	257

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	257

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	258

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	258

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	258

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	258

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	258

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	259

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	260

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	260

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	260

7.8.2.13	Interruptions at SRS carrier based switching	261

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	262

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	262

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	262

7.8.2.17	Interruptions during CQI measurement on dormant SCell	262

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	263

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	263

7.8.2.20	Interruptions at SCell hibernation	263

7.8.2.21	Interruptions at direct SCell activation and hibernation	263

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	264

7.9.1	Introduction	264

7.9.2	Minimum Requirements for Interband Carrier Aggregation	264

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	264

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	265

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	265

7.10.1	Introduction	265

7.10.2	Requirements	265

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	265

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	265

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	266

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	266

7.11	Radio Link Monitoring for UE Category 0	267

7.11.1	Introduction	267

7.11.2	Requirements for FD-FDD and TDD	268

7.11.2.1	Minimum requirement when no DRX is used	268

7.11.2.2	Minimum requirement when DRX is used	269

7.11.2.3	Minimum requirement at transitions	269

7.11.3	Requirements for HD-FDD	270

7.11.3.1	Minimum requirement when no DRX is used	270

7.11.3.2	Minimum requirement when DRX is used	270

7.11.3.3	Minimum requirement at transitions	271

7.12	Interruptions with Dual Connectivity	271

7.12.1	Introduction	271

7.12.2	Requirements	271

7.12.2.1	Interruptions at PSCell addition/release	271

7.12.2.2	Interruptions at transitions between active and non-active during DRX	271

7.12.2.3	Interruptions at transitions from non-DRX to DRX	272

7.12.2.4	Interruptions at SCell addition/release	272

7.12.2.5	Interruptions at SCell activation/deactivation	272

7.12.2.6	Interruptions during measurements on SCC	273

7.12.2.7	Interruptions at SRS carrier based switching	273

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	274

7.13.1	Definition	274

7.13.2	Minimum requirements	274

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	274

7.14.1	Introduction	274

7.14.2	PSCell Addition Delay Requirement	274

7.14.3	PSCell Release Delay Requirement	275

7.15	Maximum Receive Timing Difference in Dual Connectivity	275

7.15.1	Introduction	275

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	275

7.16	Proximity-based Services	275

7.16.1	Introduction	275

7.16.2	Requirements	275

7.16.2.1	ProSe UE transmission timing	275

7.16.2.1.1	Serving cell or PCell as timing reference	276

7.16.2.1.2	SCell or non-serving cell as timing reference	276

7.16.3	Interruptions with ProSe	276

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	276

7.16.3.2	Interruptions at ProSe Direct Communication configuration	277

7.16.3.3	Interruptions during ProSe Direct Discovery	277

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	277

7.16.3.5	Interruptions during ProSe Direct Communication	278

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	278

7.16.4.1	Measurement and evaluation of selected cell	278

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	278

7.16.5	Selection / Reselection of ProSe relay UE	279

7.16.6	ProSe operation under deactivated SCell	279

7.17	Maximum Transmission Timing Difference in Dual Connectivity	279

7.17.1	Introduction	279

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	280

7.18.1	Introduction	280

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	280

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	280

7.19	Radio Link Monitoring for UE Category M1	280

7.19.1	Introduction	280

7.19.2	Requirements for FD-FDD and TDD CE mode A	281

7.19.2.1	Minimum requirement when no DRX is used	282

7.19.2.2	Minimum requirement when DRX is used	283

7.19.2.3	Minimum requirement at transitions	284

7.19.3	Requirements for HD-FDD with CE mode A	284

7.19.3.1	Minimum requirement when no DRX is used	285

7.19.3.2	Minimum requirement when DRX is used	285

7.19.3.3	Minimum requirement at transitions	286

7.19.4	Requirements for FD-FDD and TDD with CE mode B	286

7.19.4.1	Minimum requirement when no DRX is used	288

7.19.4.2	Minimum requirement when DRX is used	289

7.19.4.3	Minimum requirement at transitions	290

7.19.5	Requirements for HD-FDD with CE mode B	290

7.19.5.1	Minimum requirement when no DRX is used	290

7.19.5.2	Minimum requirement when DRX is used	291

7.19.5.3	Minimum requirement at transitions	292

7.20	UE transmit timing for NB-IoT	292

7.20.1	Introduction	292

7.20.2	Requirements	292

7.21	 UE timer accuracy for NB-IoT	293

7.21.1	Introduction	293

7.21.2	Requirements	293

7.22	Timing Advance for NB-IoT	293

7.22.1	Introduction	293

7.22.2	Requirements	293

7.22.2.1	Timing Advance adjustment delay	293

7.22.2.2	Timing Advance adjustment accuracy	293

7.23	Radio Link Monitoring for Category NB1 UE	293

7.23.1	Introduction	293

7.23.2	Requirements for Category NB1 UE	294

7.23.2.1	Minimum requirement when no DRX is used	294

7.23.2.2	Minimum requirement when DRX is used	295

7.23.2.3	Minimum requirement at transitions	295

7.24	UE transmit timing for Category M1	295

7.24.1	Introduction	295

7.24.2	Requirements	296

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	296

7.25.1	Definition	296

7.25.2	Minimum requirements	297

7.26	UE transmit timing for Category M2	297

7.26.1	Introduction	297

7.26.2	Requirements	297

7.27	UE timer accuracy for category M1	297

7.27.1	Introduction	297

7.27.2	Requirements	297

7.28	Timing Advance for Category M1	298

7.28.1	Introduction	298

7.28.2	Requirements	298

7.29	Interruptions requirements with FeMBMS	298

7.29.1	Introduction	298

7.29.2	Requirements	298

7.30	Numerology switching delay requirements with FeMBMS	298

7.30.1	Introduction	298

7.30.2	Requirements	298

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	298

7.31.1	Introduction	298

7.31.2	NR PSCell Addition Delay Requirement	299

7.31.3	NR PSCell Release Delay Requirement	299

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	300

7.31A.1	Introduction	300

7.31A.2	NR PSCell Addition Delay Requirement	300

7.31A.3	NR PSCell Release Delay Requirement	301

7.32	Interruptions with EN-DC	301

7.32.1	Introduction	301

7.32.2	Requirements	302

7.32.2.1	Interruptions at PSCell addition/release	302

7.32.2.2	Interruptions at transitions between active and non-active during DRX	302

7.32.2.3	Interruptions at transitions from non-DRX to DRX	302

7.32.2.4	Interruptions at SCell addition/release	302

7.32.2.5	Interruptions at SCell activation/deactivation	302

7.32.2.6	Interruptions during measurements on SCC	303

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	303

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	303

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	303

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	304

7.32.2.7	Interruptions at active BWP switching	304

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	305

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	305

7.32.2.10	Interruptions at SCell hibernation	305

7.32.2.11	Interruptions at direct SCell activation and hibernation	305

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	305

7.32.2.13	Interruptions at NR SRS carrier based switching	305

7.32.2.14	Interruptions at NR SCell dormancy	306

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	306

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	307

7.32.2.15	Interruption during NR measurement with autonomous gaps	307

7.32.2.16	Interruptions at SRS carrier based switching	307

7.32.2.17	Interruptions at SCG activation/deactivation	308

7.32.2.18	Interruptions due to NR SRS antenna port switching	308

7.32.2.19	Interruptions at fast SCell activation/deactivation	308

7.32.2.20	Interruptions due to RRM measurements on deactivated NR SCG	309

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	309

7.33.1	Introduction	309

7.33.2	Requirements	309

7.34	Void	309

7.35	Interruptions with SFTD measurements	309

7.35.1	Introduction	309

7.35.2	Requirements	310

7.36	Interruptions with NE-DC	310

7.36.1	Introduction	310

7.36.2	Requirements	310

7.36.2.1	Interruptions at transitions between active and non-active during DRX	310

7.36.2.2	Interruptions at transitions from non-DRX to DRX	311

7.36.2.3	Interruptions at SCell addition/release	311

7.36.2.4	Interruptions at SCell activation/deactivation	311

7.36.2.5	Interruptions during measurements on SCC	312

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	312

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	312

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	312

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	312

7.36.2.6	Interruptions at active BWP switching	312

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	313

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	313

7.36.2.9	Interruptions at SCell hibernation	313

7.36.2.10	Interruptions at direct SCell activation and hibernation	313

7.36.2.11	Interruptions at NR SRS carrier based switching	314

7.36.2.12	Interruptions at NR SCell dormancy	314

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	314

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	315

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	315

7.36.2.14	Interruption during NR measurement with autonomous gaps	315

7.36.2.15	Interruptions at SRS carrier based switching	316

7.36.2.16	Interruptions due to NR SRS antenna port switching	316

7.37	Interruptions during NR measurement with autonomous gaps	317

7.37.1	Introduction	317

7.37.2	Requirements	317

7.38 SCG Activation and Deactivation Delay	317

7.38.1	Introduction	317

7.38.2	SCG Activation Delay Requirement	317

7.38.3	SCG Deactivation Delay Requirement	318

8	UE Measurements Procedures in RRC_CONNECTED State	319

8.1	General Measurement Requirements	319

8.1.1	Introduction	319

8.1.2	Requirements	319

8.1.2.1	UE measurement capability	319

8.1.2.1.1	Monitoring of multiple layers using gaps	327

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	328

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	329

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	331

8.1.2.1.1d	Monitoring of multiple layers using gaps (RedCap)	332

8.1.2.1.2	Network controlled small gap	332

8.1.2.2	E-UTRAN intra frequency measurements	334

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	334

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	339

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	343

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	344

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	345

8.1.2.3	E-UTRAN inter frequency measurements	345

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	346

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	351

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	358

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	358

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	358

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	359

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	361

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	362

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	363

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	370

8.1.2.4	Inter RAT measurements	370

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	370

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	375

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	375

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	379

8.1.2.4.5	E-UTRAN FDD – GSM measurements	379

8.1.2.4.6	E-UTRAN TDD – GSM measurements	384

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	384

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	386

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	386

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	386

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	387

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	387

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	387

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	387

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	389

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	389

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	389

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	389

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	390

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	390

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	392

8.1.2.4.21	E-UTRAN FDD – NR measurements	392

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	396

8.1.2.4.22	E-UTRAN TDD – NR measurements	400

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	400

8.1.2.4.23	Void	400

8.1.2.4.24	Void	400

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	400

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	402

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	402

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	403

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	403

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	403

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	405

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	407

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	409

8.1.2.6.5	Void	410

8.1.2.6.6	Void	410

8.1.2.6.7	Void	410

8.1.2.6.8	Void	410

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	410

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	411

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	412

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	414

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	416

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	418

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	419

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	421

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	423

8.1.2.7	E-UTRAN E-CID Measurements	425

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	425

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	426

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	427

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	428

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	428

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	429

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	431

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	434

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	438

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	441

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	441

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	442

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	442

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	443

8.1.2.10	Void	443

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	443

8.2.1	Introduction	443

8.2.2	Requirements	443

8.3	Measurements for E-UTRA carrier aggregation	448

8.3.1	Introduction	448

8.3.2	Measurements of the primary component carrier	448

8.3.3	Measurements of a secondary component carrier	448

8.3.3.1	Measurements of a secondary component carrier with active SCell	448

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	449

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	449

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	450

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	452

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	452

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	452

8.4.1	Introduction	452

8.4.2	Measurements on the primary component carrier	453

8.4.3	Measurements on a secondary component carrier	454

8.4.4	Measurements on both primary component carrier and a secondary component carrier	454

8.4.5	Measurements on different secondary component carriers	455

8.5	Measurements for UE category 0	456

8.5.1	Introduction	456

8.5.2 	Requirements	457

8.5.2.1 	E-UTRAN intra frequency measurements	457

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	457

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	460

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	462

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	466

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	466

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	467

8.6	Discovery signal measurements	468

8.6.1	Introduction	468

8.6.2	Requirements for CRS based discovery signal measurements	468

8.6.2.1	E-UTRAN intra frequency measurements	468

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	468

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	471

8.6.2.2	E-UTRAN inter frequency measurements	473

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	474

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	476

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	479

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	479

8.6.3	Requirements for CSI-RS based discovery signal measurements	479

8.6.3.1	E-UTRAN intra frequency measurements	479

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	480

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	482

8.6.3.2	E-UTRAN inter frequency measurements	484

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	485

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	487

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	490

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	490

8.7	Discovery signal measurements for E-UTRA carrier aggregation	490

8.7.1	Introduction	490

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	491

8.7.2.1	Measurements of the primary component carrier	491

8.7.2.2	Measurements of a secondary component carrier	491

8.7.2.3	Measurements of a secondary component carrier with active SCell	491

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	491

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	491

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	492

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	494

8.7.3.1	Measurements of the primary component carrier	494

8.7.3.2	Measurements of a secondary component carrier	494

8.7.3.3	Measurements of a secondary component carrier with active SCell	494

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	494

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	494

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	496

8.8	Measurements for E-UTRA dual connectivity	497

8.8.1	Introduction	497

8.8.2	Intra-frequency measurements requirements on PCell	497

8.8.3	Intra-frequency measurements requirements on PSCell	498

8.8.4	Inter-frequency and inter-RAT measurement requirements	498

8.8.5	Intra-frequency measurements with autonomous gaps	498

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	498

8.8.5.2	ECGI reporting delay	499

8.8.6	Inter-frequency measurements with autonomous gaps	499

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	499

8.8.6.2	ECGI reporting delay	500

8.8.7	SSTD Measurements	500

8.8.7.1	Introduction	500

8.8.7.2	SSTD Measurement requirements	500

8.8.7.3	SSTD Measurement Reporting Delay	501

8.8.8	Intra-frequency measurements requirements on SCell	501

8.9	MBSFN Measurements	501

8.9.1	Introduction	501

8.9.2	MBSFN RSRP Measurements	501

8.9.3	MBSFN RSRQ Measurements	502

8.9.4	MCH BLER Measurements	502

8.10	Proximity-based Services	502

8.10.1	Introduction	502

8.10.2	Requirements	502

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	502

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	503

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	503

8.11.1	Introduction	503

8.11.2	CRS based discovery signal measurements	504

8.11.2.1	E-UTRAN intra-frequency measurements	504

8.11.2.1.1	Requirements	504

8.11.2.1.1.1	Requirements when no DRX is used	504

8.11.2.1.1.1.1	Measurement Reporting Requirements	506

8.11.2.1.1.2	Requirements when DRX is used	507

8.11.2.1.1.2.1	Measurement Reporting Requirements	509

8.11.2.2	E-UTRAN inter-frequency measurements	510

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	510

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	514

8.11.3	CSI-RS based discovery signal measurements	514

8.11.3.1	E-UTRAN intra-frequency measurements	514

8.11.3.1.1	Requirements	514

8.11.3.1.1.1	Requirements when no DRX is used	514

8.11.3.1.1.1.1	Measurement Reporting Requirements	516

8.11.3.1.1.2	Requirements when DRX is used	516

8.11.3.1.1.2.1	Measurement Reporting Requirements	517

8.11.3.2	E-UTRAN inter-frequency measurements	518

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	518

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	522

8.11.4	RSSI measurements	522

8.11.4.1	E-UTRAN intra-frequency measurements	522

8.11.4.2	E-UTRAN inter-frequency measurements	522

8.11.5	Channel occupancy measurements	523

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	523

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	523

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	523

8.12.1	Introduction	523

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	523

8.12.2.1	Introduction	523

8.12.2.2	Measurements of a secondary component carrier	523

8.12.2.3	Measurements of a secondary component carrier with active SCell	523

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	524

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	524

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	526

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	529

8.12.3.1	Introduction	529

8.12.3.2	Measurements of a secondary component carrier	529

8.12.3.3	Measurements of a secondary component carrier with active SCell	529

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	529

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	529

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	531

8.13	Measurements for UE Category M1	533

8.13.1	Introduction	533

8.13.2	Requirements for UE category M1 with CE mode A	533

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	534

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	534

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	539

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	541

8.13.2.2	Void	545

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	545

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	545

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	548

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	550

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	551

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	551

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	553

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	555

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	556

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	556

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	556

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	557

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	557

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	557

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	558

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	558

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	559

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	560

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	560

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	561

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	565

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	567

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	571

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	571

8.13.3	Requirements for UE category M1 with CE mode B	572

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	572

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	573

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	577

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	580

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	585

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	586

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	586

8.13.3.2	Void	587

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	587

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	587

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	590

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	592

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	593

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	593

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	593

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	593

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	594

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	594

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	595

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	595

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	595

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	600

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	602

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	606

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	607

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	607

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	609

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	611

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	612

8.14	Measurements for UE category NB1	612

8.14.1	Introduction	612

8.14.2	NB-IoT intra frequency measurements under normal coverage	613

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	613

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	613

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	613

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	613

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	613

8.14.4	Connected mode channel quality report for UE Category NB1	613

8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM	614

8.14.6	NB-IoT neighbour cell measurements	614

8.14.6.1	Introduction	614

8.14.6.2	Requirements	614

8.14.6.3	Intra-frequency neighbour cell measurements	615

8.14.6.4	Inter-frequency neighbour cell measurements	615

8.14.6.5	 Requirements for monitoring multiple carriers	616

8.15	Void	617

8.16	Measurements for UE Category M2	617

8.16.1	Introduction	617

8.16.2	Requirements for UE category M2 with CE mode A	617

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	617

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	618

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	618

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	619

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	619

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	619

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	619

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	620

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	622

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	625

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	625

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	625

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	627

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	630

8.16.3	Requirements for UE category M2 with CE mode B	630

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	630

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	630

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	633

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	636

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	636

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	636

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	638

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	641

8.17	Measurements for E-UTRA – NR Dual Connectivity	641

8.17.1	Introduction	641

8.17.1.1	Measurement Gap Sharing	641

8.17.1A	Intrafrequency Measurements	642

8.17.2	SFTD Measurements	642

8.17.2.1	Introduction	642

8.17.2.2	SFTD Measurement requirements	642

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	643

8.17.2.3	SFTD Measurement Reporting Delay	644

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	644

8.17.3.1	Introduction	644

8.17.3.2	E-UTRAN FDD inter frequency measurements	645

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	645

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	646

8.17.3.3	E-UTRAN TDD inter frequency measurements	648

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	648

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	650

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	652

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	652

8.17.4.1.1	NR Inter-RAT cell identification	652

8.17.4.1.2	NR Inter-RAT measurement	654

8.17.4.1.3	NR Inter-RAT measurement reporting	654

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	655

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	655

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	655

8.17.4A.1.1	NR Inter-RAT cell identification	656

8.17.4A.1.2	NR Inter-RAT measurement	657

8.17.4A.1.3	NR Inter-RAT measurement reporting	658

8.17.4A.1.4	NR inter-RAT RSSI measurements	659

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	659

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	660

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	660

8.17.5.1	Introduction	660

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	660

8.17.5.2.1	Identification of a new UTRA FDD cell	660

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	660

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	661

8.17.5.2.4	Periodic Reporting	661

8.17.5.2.5	Event Triggered Reporting	661

8.17.5.2.6	Event-triggered Periodic Reporting	662

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	662

8.17.5.3.1	Periodic Reporting	663

8.17.5.3.2	Event Triggered Reporting	663

8.17.5.3.3	Event-triggered Periodic Reporting	664

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	664

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	664

8.17.7.1	Introduction	664

8.17.7.2	Identification of a new UTRA FDD cell for SON	664

8.17.7.2.1	Requirements when no DRX is used	664

8.17.7.2.2	Requirements when DRX is used	665

8.17.7.2.3	Reporting Delay	665

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	666

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	666

8.17.9.1	Introduction	666

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	666

8.17.9.2.1	Identification of a new UTRA FDD cell	666

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	666

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	667

8.17.9.2.4	Periodic Reporting	667

8.17.9.2.5	Event Triggered Reporting	667

8.17.9.2.6	Event-triggered Periodic Reporting	668

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	668

8.17.9.3.1	Periodic Reporting	669

8.17.9.3.2	Event Triggered Reporting	669

8.17.9.3.3	Event-triggered Periodic Reporting	669

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	670

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	670

8.17.11.1	Introduction	670

8.17.11.2	Identification of a new UTRA TDD cell for SON	670

8.17.11.2.1	Requirements when no DRX is used	670

8.17.11.2.2	Requirements when DRX is used	670

8.17.11.2.3	Reporting Delay	671

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	671

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	672

8.17.13.1	Introduction	672

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	672

8.17.13.2.1	GSM carrier RSSI	672

8.17.13.2.2	BSIC verification	672

8.17.13.2.3	Enhanced BSIC verification	674

8.17.13.2.4	Periodic Reporting	674

8.17.13.2.5	Event Triggered Reporting	674

8.17.13.2.6	Event-triggered Periodic Reporting	675

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	675

8.17.13.3.1	GSM carrier RSSI	675

8.17.13.3.2	BSIC verification	675

8.17.13.3.3	Periodic Reporting	677

8.17.13.3.4	Event Triggered Reporting	677

8.17.13.3.5	Event-triggered Periodic Reporting	677

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	677

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	677

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	677

8.17.15.1.1	RSTD Measurement Reporting Delay	678

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	678

8.17.15.2.1	RSTD Measurement Reporting Delay	678

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	678

8.17.15.3.1	RSTD Measurement Reporting Delay	679

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	679

8.17.15.4.1	RSTD Measurement Reporting Delay	679

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	680

8.17.16.1	Introduction	680

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	680

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	680

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	680

8.17.17.1	Introduction	680

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	680

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	680

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	680

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	680

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	680

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	680

8.18	Measurements for non-BL/CE UE	680

8.18.1	Introduction	680

8.18.2	Requirements for non-BL/CE UE with CE Mode B	681

8.18.2.1	E-UTRAN intra frequency measurements	681

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	681

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	681

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	682

8.19	Measurements for NR – E-UTRA Dual Connectivity	683

8.19.1	Introduction	683

8.19.2	Intra-frequency Measurements	683

8.19.3	Inter-frequency Measurements	683

8.19.4	Void	683

8.19.5	Intra-frequency E-CID Measurements	683

8.19.6	Intra-frequency measurements with autonomous gaps	684

8.19.6.1	Introduction	684

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	684

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	684

8.19.7	Inter-frequency measurements with autonomous gaps	684

8.19.7.1	Introduction	684

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	684

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	684

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	684

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	684

8.20	Inter-RAT NR Measurements for RedCap UE	684

8.20.1	Introduction	684

8.20.2	E-UTRAN FDD – NR measurements	684

8.20.2.1	Identification of a new NR cell	684

8.20.2.2	Periodic Reporting	688

8.20.2.3	Event Triggered Reporting	688

8.20.2.4	Event-triggered Periodic Reporting	688

8.20.3	E-UTRAN TDD – NR measurements	688

9	Measurements performance requirements for UE	689

9.1	E-UTRAN measurements	689

9.1.1	Introduction	689

9.1.2	Intra-frequency RSRP Accuracy Requirements	689

9.1.2.1	Absolute RSRP Accuracy	689

9.1.2.2	Relative Accuracy of RSRP	690

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	691

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	691

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	692

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	693

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	694

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	695

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	696

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	696

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	696

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	697

9.1.2B.1	Introduction	697

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	697

9.1.3	Inter-frequency RSRP Accuracy Requirements	698

9.1.3.1	Absolute RSRP Accuracy	698

9.1.3.2	Relative Accuracy of RSRP	699

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	700

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	700

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	701

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	701

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	702

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	703

9.1.3B.1	Introduction	703

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	703

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	703

9.1.4	RSRP Measurement Report Mapping	704

9.1.5	Intra-frequency RSRQ Accuracy Requirements	704

9.1.5.1	Absolute RSRQ Accuracy	704

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	705

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	706

9.1.5.4	Absolute WB-RSRQ Accuracy	707

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	708

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	709

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	709

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	709

9.1.5B.1	Introduction	709

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	709

9.1.6	Inter-frequency RSRQ Accuracy Requirements	710

9.1.6.1	Absolute RSRQ Accuracy	710

9.1.6.2	Relative Accuracy of RSRQ	711

9.1.6.3	Absolute WB-RSRQ Accuracy	712

9.1.6.4	Relative WB-RSRQ Accuracy	712

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	713

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	714

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	714

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	714

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	715

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	716

9.1.6B.1	Introduction	716

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	716

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	716

9.1.7	RSRQ Measurement Report Mapping	717

9.1.8	Power Headroom	717

9.1.8.1	Period	718

9.1.8.2	Reporting Delay	718

9.1.8.3	Void	718

9.1.8.4	Report Mapping	718

9.1.9	UE Rx – Tx time difference	718

9.1.9.1	Measurement Requirement	718

9.1.9.2	Measurement Report mapping	719

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	720

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	721

9.1.10	Reference Signal Time Difference (RSTD)	722

9.1.10.1	Intra-Frequency Accuracy Requirement	722

9.1.10.2	Inter-Frequency Accuracy Requirement	723

9.1.10.3	RSTD Measurement Report Mapping	725

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	725

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	726

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	727

9.1.11	Carrier aggregation measurement accuracy	728

9.1.11.1	Primary component carrier accuracy requirement	729

9.1.11.2	Secondary component carrier accuracy requirement	729

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	729

9.1.11.4	Secondary component carrier relative accuracy requirement	729

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	729

9.1.13	Measurement accuracy for UE category 0	730

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	730

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	730

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	731

9.1.14	Accuracy requirements for Discovery Signal Measurements	732

9.1.14.1	Introduction	732

9.1.14.2	RSRP measurements in discovery signal occasions	732

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	732

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	732

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	732

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	733

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	734

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	734

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	734

9.1.14.3.3	CSI-RSRP measurement report mapping	735

9.1.14.4	RSRQ measurements in discovery signal occasions	735

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	735

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	736

9.1.15.1.1	Primary component carrier accuracy requirement	736

9.1.15.1.2	Secondary component carrier accuracy requirement	736

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	736

9.1.15.1.4	Secondary component carrier relative accuracy requirement	736

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	736

9.1.15.2.1	Primary component carrier accuracy requirement	736

9.1.15.2.2	Secondary component carrier accuracy requirement	736

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	736

9.1.15.2.4	Secondary component carrier relative accuracy requirement	736

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	737

9.1.17	RS-SINR Measurements	737

9.1.17.1	Measurement Report Mapping	737

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	738

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	738

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	738

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	738

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	739

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	740

9.1.18.1	Introduction	740

9.1.18.2	RSRP measurements	740

9.1.18.2.1	RSRP measurement report mapping	740

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	740

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	741

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	741

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	742

9.1.18.3	RSRQ measurements	742

9.1.18.3.1	RSRQ measurement report mapping	742

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	742

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	743

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	743

9.1.18.4	CSI-RSRP measurements	744

9.1.18.4.1	CSI-RSRP measurement report mapping	744

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	744

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	744

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	745

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	745

9.1.18.5	RSSI measurements	746

9.1.18.5.1	RSSI measurement report mapping	746

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	746

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	747

9.1.18.6	Channel occupancy measurements	747

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	747

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	747

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	748

9.1.19.1	Introduction	748

9.1.19.2	Accuracy requirements for measurements on SCC	748

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	748

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	748

9.1.20	SFN and Subframe Time Difference (SSTD)	749

9.1.20.1	SSTD Accuracy Requirement	749

9.1.20.2	SSTD Measurement Report Mapping	749

9.1.21	Measurement accuracy for UE category M1	750

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	750

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	752

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	753

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	755

9.1.21.5	RSRP Measurement Report Mapping	756

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	756

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	757

9.1.21.8	RSRQ Measurement Report Mapping	758

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	758

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	759

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	760

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	761

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	762

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	763

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	764

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	765

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	766

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	767

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	768

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	769

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	772

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	775

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	775

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	778

9.1.22	Measurement accuracy for UE Category NB1	779

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	779

9.1.22.2	Void	780

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	780

9.1.22.4	Void	781

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	781

9.1.22.6	Void	782

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	782

9.1.22.8	Void	783

9.1.22.9	NRSRP Measurement Report Mapping	783

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	783

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	784

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	785

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	786

9.1.22.14	NRSRQ Measurement Report Mapping	787

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	788

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	788

9.1.22.17	Channel quality reporting for UE Category NB2 with 16-QAM	789

9.1.23	Power Headroom for UE Category NB1	789

9.1.23.1	Period	790

9.1.23.2	Reporting Delay	790

9.1.23.3	Report Mapping for UE Category NB1	790

9.1.23.3.1	Void	791

9.1.23.3.2	Void	791

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	791

9.1.24	Void	792

9.1.25	Measurement accuracy for UE category M2	792

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	792

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	793

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	794

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	795

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	797

9.1.26	Measurement Accuracy for non-BL CE UE	798

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	799

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	801

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	801

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	803

9.1.26.5	RSRP Measurement Report Mapping	804

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	804

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	804

9.1.26.8	RSRQ Measurement Report Mapping	805

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	805

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	806

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	807

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	808

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	809

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	809

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	809

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	810

9.1.27	SFN and frame Timing Difference (SFTD)	811

9.1.27.1	SFTD Accuracy Requirement	811

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	813

9.1.28.1	SFTD Accuracy Requirement under CCA	813

9.2	UTRAN FDD Measurements	814

9.2.1	UTRAN FDD CPICH RSCP	814

9.2.2	Void	815

9.2.3	UTRAN FDD CPICH Ec/No	815

9.3	UTRAN TDD Measurements	815

9.3.1	UTRAN TDD P-CCPCH RSCP	816

9.3.2	Void	816

9.3.3	Void	816

9.4	GSM Measurements	816

9.4.1	GSM carrier RSSI	816

9.5	CDMA2000 1x RTT Measurements	816

9.5.1	CDMA2000 1x RTT Pilot Strength	816

9.6	PCMAX,c	817

9.6.1	Report Mapping	817

9.6.2 	Estimation Period	817

9.6.3	Reporting Delay	817

9.7	IEEE802.11 Measurements	817

9.7.1	WLAN RSSI	817

9.7.2	WLAN RSSI Measurement Report Mapping	817

9.8	MBSFN Measurements	818

9.8.1	Introduction	818

9.8.2	MBSFN RSRP	818

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	818

9.8.2.2	MBSFN RSRP measurement report mapping	819

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	819

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	820

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	820

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	820

9.8.3	MBSFN RSRQ	821

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	821

9.8.3.2	MBSFN RSRQ measurement report mapping	821

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	822

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	822

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	822

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	823

9.8.4	MCH BLER	823

9.8.4.1	Measurement report mapping for MCH BLER	823

9.8.4.2	Measurement report mapping for MCH Block Number	824

9.9	ProSe Measurements	825

9.9.1	Introduction	825

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	825

9.9.2.1	Absolute S-RSRP Accuracy	825

9.9.2.2	Relative Accuracy of S-RSRP	826

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	827

9.9.3.1	Absolute SD-RSRP Accuracy	827

9.9.3.2	Relative Accuracy of SD-RSRP	827

9.10	V2X Measurements	828

9.10.1	Introduction	828

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	828

9.10.2.1	Absolute S-RSRP Accuracy	828

9.10.2.2	Relative Accuracy of S-RSRP	829

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	829

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	829

9.10.4	S-RSSI Measurement Accuracy Requirements	830

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	830

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	830

9.11	NR Measurements	831

9.11.1	NR SS-RSRP Measurements	831

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	831

9.11.2	NR SS-RSRQ Measurements	832

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	832

9.11.3	NR SS-SINR Measurements	832

9.11.4	NR SS-RSRP Measurements under CCA	833

9.11.5	NR SS-RSRQ Measurements under CCA	833

9.11.6	NR SS-SINR Measurements under CCA	833

9.11.7	NR RSSI Measurements under CCA	834

9.11.8	NR Channel Occupancy Measurements under CCA	834

10	Measurements Performance Requirements for E-UTRAN	834

10.1	Received Interference Power	834

10.1.1	Absolute accuracy requirement	834

10.1.2	Relative accuracy requirement	835

10.1.3	Received Interference Power measurement report mapping	835

10.2	Angle of Arrival (AOA)	835

10.2.1	Range/mapping	835

10.3	Timing Advance (TADV)	835

10.3.1	Report mapping	835

11	ProSe Requirements in Any Cell Selection state	836

11.1	Introduction	836

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	836

11.2.1	Introduction	836

11.2.2	ProSe UE transmission timing	836

11.3	Initiation/Cease of SLSS Transmissions	837

11.3.1	Introduction	837

11.3.2	Requirements	837

11.4	Measurements for ProSe in Any Cell Selection State	837

11.4.1	Introduction	837

11.4.2	Requirements	837

11.4.2.1	E-UTRA FDD	837

11.4.2.2	E-UTRA TDD	838

11.5	Selection / Reselection of ProSe Synchronization Reference	838

11.5.1	Introduction	838

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	838

11.5.2.1	Introduction	838

11.5.2.2	Requirements	838

11.6	Void	839

11.7	Selection / Reselection of ProSe relay UE	839

11.7.1	Introduction	839

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	839

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	839

12.1	Introduction	839

12.2	Transmit Timing	840

12.2.1	GNSS as timing reference	840

12.3	Interruption	840

12.4	Reliability of GNSS signal	840

13	V2X Requirements	840

13.1	Introduction	840

13.2	UE Transmit Timing	841

13.2.1	Introduction	841

13.2.2	GNSS as synchronization reference source	841

13.2.3	Serving cell/PCell as synchronization reference source	841

13.2.4	SyncRef UE as synchronization reference source	841

13.3	Initiation/Cease of SLSS Transmissions	841

13.3.1	Introduction	841

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	841

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	842

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	842

13.4	Selection / Reselection of V2X Synchronization Reference Source	843

13.5	Autonomous Resource Selection/Reselection measurements	844

13.5.1	Introduction	844

13.5.2	PSSCH-RSRP measurements	844

13.5.3	S-RSSI measurements	844

13.6	Congestion Control measurements	844

13.7	Interruption	844

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	844

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	844

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	845

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	845

13.8	Reliability of GNSS signal	845

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	846

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	846

Annex A (normative): Test Cases	853

A.1	Purpose of annex	853

A.2	Requirement classification for statistical testing	853

A.2.1	Types of requirements in TS 36.133	853

A.2.1.1	Time and delay requirements on UE higher layer actions	853

A.2.1.2	Measurements of power levels, relative powers and time	853

A.2.1.3	Implementation requirements	854

A.2.1.4	Physical layer timing requirements	854

A.3	RRM test configurations	855

A.3.1	Reference Measurement Channels	855

A.3.1.1	PDSCH	855

A.3.1.1.1	FDD	855

A.3.1.1.2	TDD	860

A.3.1.1.3	FDD for UE category 0	863

A.3.1.1.4	HD-FDD for UE category 0	864

A.3.1.1.5	TDD for UE category 0	865

A.3.1.1.6	Frame Structure 3	866

A.3.1.2	PCFICH/PDCCH/PHICH	867

A.3.1.2.1	FDD	867

A.3.1.2.2	TDD	867

A.3.1.2.3	HD-FDD for UE category 0	868

A.3.1.2.4	FS 3	868

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	868

A.3.1.3.1	FDD in CEModeA	869

A.3.1.3.2	HD-FDD in CEModeA	869

A.3.1.3.3	TDD in CEModeA	870

A.3.1.3.4	FDD in CEModeB	870

A.3.1.3.5	HD-FDD in CEModeB	871

A.3.1.3.6	TDD in CEModeB	871

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	872

A.3.1.4.1	FDD in CEModeA	872

A.3.1.4.2	HD-FDD in CEModeA	873

A.3.1.4.3	TDD in CEModeA	874

A.3.1.4.4	FDD in CEModeB	875

A.3.1.4.5	 HD-FDD in CEModeB	876

A.3.1.4.6	TDD in CEModeB	876

A.3.1.5	NPDSCH Reference Channel for UE category NB1	877

A.3.1.5.1	HD-FDD in-band operation	877

A.3.1.5.2	Void	878

A.3.1.5.3	HD-FDD standalone operation	878

A.3.1.5.4	Void	879

A.3.1.5.5	HD-FDD guard band operation	879

A.3.1.5.6	Void	880

A.3.1.5.7	TDD in-band operation	880

A.3.1.5.8	TDD standalone operation	880

A.3.1.5.9	TDD guard band operation	881

A.3.1.6	NPDCCH Reference Channel for UE category NB1	881

A.3.1.6.1	In-band operation	881

A.3.1.6.2	Void	882

A.3.1.6.3	Standalone operation	882

A.3.1.6.4	Void	882

A.3.1.6.5	Guard band operation	882

A.3.1.6.6	Void	883

A.3.2	OFDMA Channel Noise Generator (OCNG)	883

A.3.2.1	OCNG Patterns for FDD	883

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	884

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	884

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	885

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	885

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	886

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	887

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	887

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	887

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	888

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	889

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	889

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	890

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	890

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	891

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	891

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	892

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	892

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	893

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	893

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	894

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	894

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	895

A.3.2.2	OCNG Patterns for TDD	895

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	896

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	896

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	897

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	897

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	898

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	899

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	900

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	901

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	901

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	902

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	902

A.3.2.3	OCNG Patterns for Narrowband IoT	903

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	904

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	905

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	905

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	906

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	907

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	908

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	909

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	910

A.3.2.4	OCNG Patterns for V2X sidelink	911

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	911

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	912

A.3.3	Reference DRX Configurations	912

A.3.4	ABS Transmission Configurations	913

A.3.4.1	Non-MBSFN ABS Transmission Configurations	913

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	913

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	913

A.3.4.2	MBSFN ABS Transmission Configurations	914

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	914

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	915

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	915

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	915

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	916

A.3.6.1	Introduction	916

A.3.7	Test Cases with Different Channel Bandwidths	916

A.3.7.1	Introduction	916

A.3.7.2	Principle of testing	916

A.3.8	Antenna Configuration	916

A.3.8.1	Antenna connection for 4 Rx capable UEs	916

A.3.8.1.1 Introduction	916

A.3.8.1.2 Principle of testing	916

A.3.8.1.2.1 Single carrier tests	916

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	917

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	918

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	918

A.3.8.2	Antenna connection for 8 Rx capable UEs	918

A.3.8.2.1	Introduction	918

A.3.8.2.2	Principle of testing	918

A.3.8.2.2.1	Single carrier tests	918

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	919

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	919

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	919

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	919

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	919

A.3.9.1	Introduction	919

A.3.9.2	Principle of testing	919

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	920

A.3.10.1	Introduction	920

A.3.10.2	Principle of testing	920

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	920

A.3.11.1	Introduction	920

A.3.11.2	Principle of Testing	920

A.3.12	Proximity-based Services	920

A.3.12.1	Introduction	920

A.3.12.2	Reference DRX configurations for ProSe tests	920

A.3.12.3	Test Cases with Different Channel Bandwidths	921

A.3.12.3.1	Introduction	921

A.3.12.3.2	Principle of testing	921

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	921

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	928

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	931

A.3.12.6.1	FDD	931

A.3.12.7	Reference measurement channels for ProSe Direct Communication	931

A.3.12.7.1	FDD	931

A.3.12.8	ProSe Receive Traffic Generator	932

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	932

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	932

A.3.13	Time Offset between Cells	933

A.3.13.1	Introduction	933

A.3.13.2	Definition	933

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	933

A.3.14.1	Introduction	933

A.3.14.2	Principle of testing	933

A.3.15	Dual connectivity test cases with different combination of duplex mode	933

A.3.15.1	Introduction	933

A.3.15.2	Principle of testing	933

A.3.16	Reference PRACH Configurations	934

A.3.17	Listen before talk model	934

A.3.17.1	Introduction	934

A.3.17.2	Definition	934

A.3.18	Reference NPRACH Configurations	935

A.3.19	Dual connectivity test cases with different bandwidth combinations	936

A.3.19.1	Introduction	936

A.3.19.2	Principle of testing	936

A.3.20	Category M1 UE Test Cases	937

A.3.20.1	Introduction	937

A.3.20.2	Principle of Cat-M1 UE Testing	937

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	938

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	939

A.3.21.1	Introduction	939

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	939

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	940

A.3.22	Category 1bis UE Test Cases	941

A.3.22.1	Introduction	941

A.3.22.2	Principle of Category 1bis UE Testing	941

A.3.23	Category NB2 UE Test Cases	945

A.3.23.1	Introduction	945

A.3.23.2	Principle of Category NB2 UE Testing	945

A.3.24	V2X sidelink communication	948

A.3.24.1	Introduction	948

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	949

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	953

A.3.25	Category M2 UE Test Cases	954

A.3.25.1	Introduction	954

A.3.25.2	Principle of Cat-M2 UE Testing	954

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	955

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	956

A.3.26.1	Introduction	956

A.3.26.2	Principle of testing	956

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	956

A.3.27.1	Introduction	956

A.3.27.2	Principle of INACTIVE cell re-selection Testing	956

A.4	E-UTRAN RRC_IDLE state	956

A.4.2	Cell Re-Selection	956

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	956

A.4.2.1.1	Test Purpose and Environment	956

A.4.2.1.2	Test Requirements	958

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	959

A.4.2.2.1	Test Purpose and Environment	959

A.4.2.2.2	Test Requirements	960

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	961

A.4.2.3.1	Test Purpose and Environment	961

A.4.2.3.2	Test Requirements	962

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	963

A.4.2.4.1	Test Purpose and Environment	963

A.4.2.4.2	Test Requirements	964

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	965

A.4.2.5.1	Test Purpose and Environment	965

A.4.2.5.2	Test Requirements	966

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	967

A.4.2.6.1	Test Purpose and Environment	967

A.4.2.6.2	Test Requirements	968

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	969

A.4.2.7.1	Test Purpose and Environment	969

A.4.2.7.2	Test Requirements	970

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	971

A.4.2.8.1	Test Purpose and Environment	971

A.4.2.8.2	Test Requirements	972

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	973

A.4.2.9.1	Test Purpose and Environment	973

A.4.2.9.2	Test Requirements	973

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	973

A.4.2.10.1	Test Purpose and Environment	973

A.4.2.10.2	Test Requirements	977

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	977

A.4.2.11.1	Test Purpose and Environment	977

A.4.2.11.2	Test Requirements	981

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	981

A.4.2.12.1	Test Purpose and Environment	981

A.4.2.12.2	Test Requirements	983

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	984

A.4.2.13.1	Test Purpose and Environment	984

A.4.2.13.2	Test Requirements	985

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	986

A.4.2.14.1	Test Purpose and Environment	986

A.4.2.14.2	Test Requirements	987

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	988

A.4.2.15.1	Test Purpose and Environment	988

A.4.2.15.2	Test Requirements	989

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	990

A.4.2.16.1	Test Purpose and Environment	990

A.4.2.16.2	Test Requirements	991

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	992

A.4.2.17.1	Test Purpose and Environment	992

A.4.2.17.2	Test Requirements	993

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	994

A.4.2.18.1	Test Purpose and Environment	994

A.4.2.18.2	Test Requirements	996

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	997

A.4.2.19.1	Test Purpose and Environment	997

A.4.2.19.2	Test Requirements	999

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	1000

A.4.2.20.1	Test Purpose and Environment	1000

A.4.2.20.2	Test Requirements	1001

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	1002

A.4.2.21.1	Test Purpose and Environment	1002

A.4.2.21.2	Test Requirements	1003

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1004

A.4.2.22.1	Test Purpose and Environment	1004

A.4.2.22.2	Test Requirements	1005

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1006

A.4.2.23.1	Test Purpose and Environment	1006

A.4.2.23.2	Test Requirements	1007

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1008

A.4.2.24.1	Test Purpose and Environment	1008

A.4.2.24.2	Test Requirements	1011

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1012

A.4.2.25.1	Test Purpose and Environment	1012

A.4.2.25.2	Test Requirements	1013

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1014

A.4.2.26.1	Test Purpose and Environment	1014

A.4.2.26.2	Test Requirements	1015

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1016

A.4.2.27.1	Test Purpose and Environment	1016

A.4.2.27.2	Test Requirements	1017

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1018

A.4.2.28.1	Test Purpose and Environment	1018

A.4.2.28.2	Test Requirements	1019

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1020

A.4.2.29.1	Test Purpose and Environment	1020

A.4.2.29.2	Test Requirements	1021

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	1022

A.4.2.30.1	Test Purpose and Environment	1022

A.4.2.30.2	Test Requirements	1023

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	1024

A.4.2.31.1	Test Purpose and Environment	1024

A.4.2.31.2	Test Requirements	1025

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	1026

A.4.2.32.1	Test Purpose and Environment	1026

A.4.2.32.2	Test Requirements	1027

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	1028

A.4.2.33.1	Test Purpose and Environment	1028

A.4.2.33.2	Test Requirements	1029

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	1030

A.4.2.34.1	Test Purpose and Environment	1030

A.4.2.34.2	Test Requirements	1031

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1032

A.4.2.35.1	Test Purpose and Environment	1032

A.4.2.35.2	Test Requirements	1034

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1035

A.4.2.36.1	Test Purpose and Environment	1035

A.4.2.36.2	Test Requirements	1037

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1038

A.4.2.37.1	Test Purpose and Environment	1038

A.4.2.37.2	Test Requirements	1040

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1041

A.4.2.38.1	Test Purpose and Environment	1041

A.4.2.38.2	Test Requirements	1043

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1044

A.4.2.39.1	Test Purpose and Environment	1044

A.4.2.39.2	Test Requirements	1045

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1046

A.4.2.40.1	Test Purpose and Environment	1046

A.4.2.40.2	Test Requirements	1047

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1048

A.4.2.41.1	Test Purpose and Environment	1048

A.4.2.41.2	Test Requirements	1051

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1052

A.4.2.42.1	Test Purpose and Environment	1052

A.4.2.42.2	Test Requirements	1054

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1055

A.4.2.43.1	Test Purpose and Environment	1055

A.4.2.43.2	Test Requirements	1057

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1058

A.4.2.44.1	Test Purpose and Environment	1058

A.4.2.44.2	Test Requirements	1060

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1061

A.4.2.45.1	Test Purpose and Environment	1061

A.4.2.45.2	Test Requirements	1063

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1064

A.4.2.46.1	Test Purpose and Environment	1064

A.4.2.46.2	Test Requirements	1066

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1067

A.4.2.47.1	Test Purpose and Environment	1067

A.4.2.47.2	Test Requirements	1069

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1070

A.4.2.48.1	Test Purpose and Environment	1070

A.4.2.48.2	Test Requirements	1071

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1072

A.4.2.49.1	Test Purpose and Environment	1072

A.4.2.49.2	Test Requirements	1074

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1075

A.4.2.50.1	Test Purpose and Environment	1075

A.4.2.50.2	Test Requirements	1077

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1078

A.4.2.51.1	Test Purpose and Environment	1078

A.4.2.51.2	Test Requirements	1080

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1081

A.4.2.52.1	Test Purpose and Environment	1081

A.4.2.52.2	Test Requirements	1083

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1084

A.4.2.53.1	Test Purpose and Environment	1084

A.4.2.53.2	Test Requirements	1086

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1087

A.4.2.54.1	Test Purpose and Environment	1087

A.4.2.54.2	Test Requirements	1089

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1090

A.4.2.55.1	Test Purpose and Environment	1090

A.4.2.55.2	Test Requirements	1092

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1093

A.4.2.56.1	Test Purpose and Environment	1093

A.4.2.56.2	Test Requirements	1094

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1095

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1095

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1095

A.4.3.1.1.1	Test Purpose and Environment	1095

A.4.3.1.1.2	Test Requirements	1097

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1097

A.4.3.1.2.1	Test Purpose and Environment	1097

A.4.3.1.2.2	Test Requirements	1100

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1100

A.4.3.1.3.1	Test Purpose and Environment	1100

A.4.3.1.3.2	Test Requirements	1103

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1103

A.4.3.1.4.1	Test Purpose and Environment	1103

A.4.3.1.4.2	Test Requirements	1104

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1104

A.4.3.1.5.1	Test Purpose and Environment	1104

A.4.3.1.5.2	Test Requirements	1107

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1108

A.4.3.2.1	Test Purpose and Environment	1108

A.4.3.2.1.1	Void	1108

A.4.3.2.1.2	1.28Mcps TDD option	1108

A.4.3.2.1.3	Void	1110

A.4.3.2.2	Test Requirements	1110

A.4.3.2.2.1	1.28Mcps TDD option	1110

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1110

A.4.3.2A.1	Test Purpose and Environment	1110

A.4.3.2A.2	Test Requirements	1114

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1114

A.4.3.3.1	Test Purpose and Environment	1114

A.4.3.3.2	Test Requirements	1117

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1117

A.4.3.3A.1	Test Purpose and Environment	1117

A.4.3.3A.2	Test Requirements	1122

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1123

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1123

A.4.3.4.1.1	Test Purpose and Environment	1123

A.4.3.4.1.2	Test Requirements	1125

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1125

A.4.3.4.2.1	Test Purpose and Environment	1125

A.4.3.4.2.2	Test Requirements	1127

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1127

A.4.3.4.3.1	Test Purpose and Environment	1127

A.4.3.4.3.2	Test Requirements	1130

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1130

A.4.3.4.4.1	Test Purpose and Environment	1130

A.4.3.4.4.2	Test Requirements	1134

A.4.4	E-UTRAN to GSM Cell Re-Selection	1134

A.4.4.1	E-UTRAN FDD – GSM:	1134

A.4.4.1.1	Test Purpose and Environment	1134

A.4.4.1.2	Test Requirements	1136

A.4.4.2	E-UTRAN TDD – GSM:	1136

A.4.4.2.1	Test Purpose and Environment	1136

A.4.4.2.2	Test Requirements	1138

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1139

A.4.5.1	E-UTRAN FDD – HRPD	1139

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1139

A.4.5.1.1.1	Test Purpose and Environment	1139

A.4.5.1.1.2	Test Requirements	1141

A.4.5.2	E-UTRAN TDD – HRPD	1141

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1141

A.4.5.2.1.1	Test Purpose and Environment	1141

A.4.5.2.1.2	Test Requirements	1144

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1144

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1144

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1144

A.4.6.1.1.1	Test Purpose and Environment	1144

A.4.6.1.1.2	Test Requirements	1147

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1147

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1147

A.4.6.2.1.1	Test Purpose and Environment	1147

A.4.6.2.1.2	Test Requirements	1150

A.4.7	Idle State Positioning Measurement for UE category NB1	1150

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1150

A.4.7.1.1	Test Purpose and Environment	1150

A.4.7.1.2	Test Requirements	1155

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1156

A.4.7.2.1	Test Purpose and Environment	1156

A.4.7.4.2	Test Requirements	1160

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1161

A.4.7.3.1	Test Purpose and Environment	1161

A.4.7.3.2	Test Requirements	1165

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1166

A.4.7.4.1	Test Purpose and Environment	1166

A.4.7.4.2	Test Requirements	1170

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1171

A.5.1	E-UTRAN Handover	1171

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1171

A.5.1.1.1	Test Purpose and Environment	1171

A.5.1.1.2	Test Requirements	1173

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1173

A.5.1.2.1	Test Purpose and Environment	1173

A.5.1.2.2	Test Requirements	1175

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1175

A.5.1.3.1	Test Purpose and Environment	1175

A.5.1.3.2	Test Requirements	1177

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1177

A.5.1.4.1	Test Purpose and Environment	1177

A.5.1.4.2	Test Requirements	1179

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1179

A.5.1.5.1	Test Purpose and Environment	1179

A.5.1.5.2	Test Requirements	1181

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1181

A.5.1.6.1	Test Purpose and Environment	1181

A.5.1.6.2	Test Requirements	1183

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1183

A.5.1.7.1	Test Purpose and Environment	1183

A.5.1.7.2	Test Requirements	1186

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1186

A.5.1.8.1	Test Purpose and Environment	1186

A.5.1.8.2	Test Requirements	1189

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1189

A.5.1.9.1	Test Purpose and Environment	1189

A.5.1.9.2	Test Requirements	1190

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1190

A.5.1.10.1	Test Purpose and Environment	1190

A.5.1.10.2	Test Requirements	1192

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1192

A.5.1.11.1	Test Purpose and Environment	1192

A.5.1.11.2	Test Requirements	1194

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1194

A.5.1.12.1	Test Purpose and Environment	1194

A.5.1.12.2	Test Requirements	1196

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1196

A.5.1.13.1	Test Purpose and Environment	1196

A.5.1.13.2	Test Requirements	1198

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1199

A.5.1.14.1	Test Purpose and Environment	1199

A.5.1.14.2	Test Requirements	1200

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1201

A.5.1.15.1	Test Purpose and Environment	1201

A.5.1.15.2	Test Requirements	1202

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1203

A.5.1.16.1	Test Purpose and Environment	1203

A.5.1.16.2	Test Requirements	1204

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1205

A.5.1.17.1	Test Purpose and Environment	1205

A.5.1.17.2	Test Requirements	1206

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1207

A.5.1.18.1	Test Purpose and Environment	1207

A.5.1.18.2	Test Requirements	1208

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1209

A.5.1.19.1	Test Purpose and Environment	1209

A.5.1.19.2	Test Requirements	1210

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1211

A.5.1.20.1	Test Purpose and Environment	1211

A.5.1.20.2	Test Requirements	1212

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1213

A.5.1.21.1	Test Purpose and Environment	1213

A.5.1.21.2	Test Requirements	1214

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1214

A.5.1.22.1	Test Purpose and Environment	1214

A.5.1.22.2	Test Requirements	1216

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1216

A.5.1.23.1	Test Purpose and Environment	1216

A.5.1.23.2	Test Requirements	1218

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1218

A.5.1.24.1	Test Purpose and Environment	1218

A.5.1.24.2	Test Requirements	1220

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1220

A.5.1.25.1	Test Purpose and Environment	1220

A.5.1.25.2	Test Requirements	1222

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1223

A.5.1.26.1	Test Purpose and Environment	1223

A.5.1.26.2	Test Requirements	1224

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1225

A.5.1.27.1	Test Purpose and Environment	1225

A.5.1.27.2	Test Requirements	1226

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1227

A.5.1.28.1	Test Purpose and Environment	1227

A.5.1.28.2	Test Requirements	1228

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1229

A.5.1.29.1	Test Purpose and Environment	1229

A.5.1.29.2	Test Requirements	1230

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1231

A.5.1.30.1	Test Purpose and Environment	1231

A.5.1.30.2	Test Requirements	1232

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1233

A.5.1.31.1	Test Purpose and Environment	1233

A.5.1.31.2	Test Requirements	1234

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1235

A.5.1.32.1	Test Purpose and Environment	1235

A.5.1.32.2	Test Requirements	1236

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1237

A.5.1.33.1	Test Purpose and Environment	1237

A.5.1.13.2	Test Requirements	1238

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1239

A.5.1.34.1	Test Purpose and Environment	1239

A.5.1.34.2	Test Requirements	1240

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1241

A.5.1.35.1	Test Purpose and Environment	1241

A.5.1.35.2	Test Requirements	1242

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1243

A.5.1.36.1	Test Purpose and Environment	1243

A.5.1.36.2	Test Requirements	1244

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1245

A.5.1.37.1	Test Purpose and Environment	1245

A.5.1.37.2	Test Requirements	1246

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1247

A.5.1.38.1	Test Purpose and Environment	1247

A.5.1.38.2	Test Requirements	1248

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1249

A.5.1.39.1	Test Purpose and Environment	1249

A.5.1.39.2	Test Requirements	1250

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1251

A.5.1.40.1	Test Purpose and Environment	1251

A.5.1.40.2	Test Requirements	1252

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1253

A.5.1.41.1	Test Purpose and Environment	1253

A.5.1.41.2	Test Requirements	1255

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1256

A.5.1.42.1	Test Purpose and Environment	1256

A.5.1.42.2	Test Requirements	1257

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1258

A.5.1.43.1	Test Purpose and Environment	1258

A.5.1.43.2	Test Requirements	1259

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1260

A.5.1.44.1	Test Purpose and Environment	1260

A.5.1.44.2	Test Requirements	1261

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1262

A.5.1.45.1	Test Purpose and Environment	1262

A.5.1.45.1	Test Requirements	1263

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1264

A.5.1.46.1	Test Purpose and Environment	1264

A.5.1.46.2	Test Requirements	1265

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1266

A.5.1.47.1	Test Purpose and Environment	1266

A.5.1.47.2	Test Requirements	1267

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1268

A.5.1.48.1	Test Purpose and Environment	1268

A.5.1.48.2	Test Requirements	1269

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1270

A.5.1.49.1	Test Purpose and Environment	1270

A.5.1.49.2	Test Requirements	1271

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1272

A.5.1.50.1	Test Purpose and Environment	1272

A.5.1.50.2	Test Requirements	1273

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1274

A.5.1.51.1	Test Purpose and Environment	1274

A.5.1.51.2	Test Requirements	1276

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1276

A.5.152.1	Test Purpose and Environment	1276

A.5.1.52.2	Test Requirements	1279

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1280

A.5.1.53.1	Test Purpose and Environment	1280

A.5.1.53.2	Test Requirements	1282

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1283

A.5.1.54.1	Test Purpose and Environment	1283

A.5.1.54.2	Test Requirements	1285

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1286

A.5.1.55.1	Test Purpose and Environment	1286

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1290

A.5.1.56.1	Test Purpose and Environment	1290

A.5.1.56.2	Test Requirements	1293

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1294

A.5.1.57.1	Test Purpose and Environment	1294

A.5.1.57.2	Test Requirements	1296

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1296

A.5.1.58.1	Test Purpose and Environment	1296

A.5.1.58.2	Test Requirements	1299

A.5.2	E-UTRAN Handover to other RATs	1299

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1299

A.5.2.1.1	Test Purpose and Environment	1299

A.5.2.1.2	Test Requirements	1301

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1302

A.5.2.2.1	Test Purpose and Environment	1302

A.5.2.2.2	Test Requirements	1305

A.5.2.3 E-UTRAN FDD- GSM Handover	1305

A.5.2.3.1	Test Purpose and Environment	1305

A.5.2.3.2	Test Requirements	1306

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1307

A.5.2.4.1	Test Purpose and Environment	1307

A.5.2.4.1.1	Void	1307

A.5.2.4.1.2	1.28 Mcps TDD option	1307

A.5.2.4.1.3	Void	1309

A.5.2.4.2	Test Requirements	1309

A.5.2.4.2.1	Void	1309

A.5.2.4.2.2	1.28 Mcps TDD option	1309

A.5.2.4.2.3	Void	1309

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1309

A.5.2.5.1	Test Purpose and Environment	1309

A.5.2.5.1.3	Void	1312

A.5.2.5.2	Test Requirements	1312

A.5.2.5.2.1	Void	1312

A.5.2.5.2.2	1.28 Mcps TDD option	1312

A.5.2.5.2.3	Void	1312

A.5.2.6	E-UTRAN TDD - GSM Handover	1312

A.5.2.6.1	Test Purpose and Environment	1312

A.5.2.6.2	Test Requirements	1314

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1315

A.5.2.7.1	Test Purpose and Environment	1315

A.5.2.7.2	Test Requirements	1317

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1317

A.5.2.8.1	Test Purpose and Environment	1317

A.5.2.8.2	Test Requirements	1318

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1319

A.5.2.9.1	Test Purpose and Environment	1319

A.5.2.9.2	Test Requirements	1320

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1321

A.5.2.10.1	Test Purpose and Environment	1321

A.5.2.10.2	Test Requirements	1323

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1323

A.5.2.10A.1	Test Purpose and Environment	1323

A.5.2.10A.2	Test Requirements	1326

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1326

A.5.2.10B.1	Test Purpose and Environment	1326

A.5.2.10B.2	Test Requirements	1329

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1329

A.5.2.11.1	Test Purpose and Environment	1329

A.5.2.11.2	Test Requirements	1329

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1330

A.5.3.1	E-UTRAN FDD – HRPD Handover	1330

A.5.3.1.1	Test Purpose and Environment	1330

A.5.3.1.2	Test Requirements	1332

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1332

A.5.3.2.1	Test Purpose and Environment	1332

A.5.3.2.2	Test Requirements	1335

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1335

A.5.3.3.1	Test Purpose and Environment	1335

A.5.3.3.2	Test Requirements	1338

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1338

A.5.3.4.1	Test Purpose and Environment	1338

A.5.3.4.2	Test Requirements	1339

A.5.3.5	E-UTRAN TDD – HRPD Handover	1340

A.5.3.5.1	Test Purpose and Environment	1340

A.5.3.5.2	Test Requirements	1343

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1343

A.5.3.6.1	Test Purpose and Environment	1343

A.5.3.6.2	Test Requirements	1346

A.6	RRC Connection Control	1346

A.6.1	RRC Re-establishment	1346

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1346

A.6.1.1.1	Test Purpose and Environment	1346

A.6.1.1.2	Test Requirements	1347

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1348

A.6.1.2.1	Test Purpose and Environment	1348

A.6.1.2.2	Test Requirements	1349

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1350

A.6.1.3.1	Test Purpose and Environment	1350

A.6.1.3.2	Test Requirements	1351

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1352

A.6.1.4.1	Test Purpose and Environment	1352

A.6.1.4.2	Test Requirements	1353

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1354

A.6.1.5.1	Test Purpose and Environment	1354

A.6.1.5.2	Test Requirements	1354

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1354

A.6.1.6.1	Test Purpose and Environment	1354

A.6.1.6.2	Test Requirements	1356

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1357

A.6.1.7.1	Test Purpose and Environment	1357

A.6.1.7.2	Test Requirements	1358

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1359

A.6.1.8.1	Test Purpose and Environment	1359

A.6.1.8.2	Test Requirements	1360

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1361

A.6.1.9.1	Test Purpose and Environment	1361

A.6.1.9.2	Test Requirements	1362

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1363

A.6.1.10.1	Test Purpose and Environment	1363

A.6.1.10.2	Test Requirements	1364

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1365

A.6.1.11.1	Test Purpose and Environment	1365

A.6.1.11.2	Test Requirements	1366

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1367

A.6.1.12.1	Test Purpose and Environment	1367

A.6.1.12.2	Test Requirements	1368

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1369

A.6.1.13.1	Test Purpose and Environment	1369

A.6.1.13.2	Test Requirements	1370

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1371

A.6.1.14.1	Test Purpose and Environment	1371

A.6.1.14.2	Test Requirements	1372

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1373

A.6.1.15.1	Test Purpose and Environment	1373

A.6.1.15.2	Test Requirements	1375

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1376

A.6.1.16.1	Test Purpose and Environment	1376

A.6.1.16.2	Test Requirements	1378

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1379

A.6.1.17.1	Test Purpose and Environment	1379

A.6.1.17.2	Test Requirements	1380

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1381

A.6.1.18.1	Test Purpose and Environment	1381

A.6.1.18.2	Test Requirements	1382

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1383

A.6.1.19.1	Test Purpose and Environment	1383

A.6.1.19.2	Test Requirements	1384

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1385

A.6.1.20.1	Test Purpose and Environment	1385

A.6.1.20.2	Test Requirements	1386

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1387

A.6.1.21.1	Test Purpose and Environment	1387

A.6.1.21.2	Test Requirements	1388

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1389

A.6.1.22.1	Test Purpose and Environment	1389

A.6.1.22.2	Test Requirements	1390

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1391

A.6.1.23.1	Test Purpose and Environment	1391

A.6.1.23.2	Test Requirements	1393

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1394

A.6.1.24.1	Test Purpose and Environment	1394

A.6.1.24.2	Test Requirements	1396

A.6.2	Random Access	1397

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1397

A.6.2.1.1	Test Purpose and Environment	1397

A.6.2.1.2	Test Requirements	1398

A.6.2.1.2.1	Random Access Response Reception	1398

A.6.2.1.2.2	No Random Access Response Reception	1398

A.6.2.1.2.3	Receiving a NACK on msg3	1398

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1399

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1399

A.6.2.1.2.6	Contention Resolution Timer expiry	1399

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1399

A.6.2.2.1	Test Purpose and Environment	1399

A.6.2.2.2	Test Requirements	1400

A.6.2.2.2.1	Random Access Response Reception	1401

A.6.2.2.2.2	No Random Access Response Reception	1401

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1401

A.6.2.3.1	Test Purpose and Environment	1401

A.6.2.3.2	Test Requirements	1403

A.6.2.3.2.1	Random Access Response Reception	1403

A.6.2.3.2.2	No Random Access Response reception	1403

A.6.2.3.2.3	Receiving a NACK on msg3	1403

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1404

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1404

A.6.2.3.2.6	Contention Resolution Timer expiry	1404

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1404

A.6.2.4.1	Test Purpose and Environment	1404

A.6.2.4.2	Test Requirements	1406

A.6.2.4.2.1	Random Access Response Reception	1406

A.6.2.4.2.2	No Random Access Response Reception	1406

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1406

A.6.2.5.1	Test Purpose and Environment	1406

A.6.2.5.2	Test Requirements	1407

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1407

A.6.2.6.1	Test Purpose and Environment	1407

A.6.2.6.2	Test Requirements	1407

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1408

A.6.2.7.1	Test Purpose and Environment	1408

A.6.2.7.2	Test Requirements	1410

A.6.2.7.2.1	Random Access Response Reception	1410

A.6.2.7.2.2	No Random Access Response Reception	1410

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1410

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1411

A.6.2.8.1	Test Purpose and Environment	1411

A.6.2.8.2	Test Requirements	1413

A.6.2.8.2.1	Random Access Response Reception	1413

A.6.2.8.2.2	No Random Access Response Reception	1413

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1413

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1414

A.6.2.9.1	Test Purpose and Environment	1414

A.6.2.9.2	Test Requirements	1417

A.6.2.9.2.1	Random Access Response Reception	1417

A.6.2.9.2.2	No Random Access Response Reception	1418

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1418

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1419

A.6.2.10.1	Test Purpose and Environment	1419

A.6.2.10.2	Test Requirements	1421

A.6.2.10.2.1	Random Access Response Reception	1421

A.6.2.10.2.2	No Random Access Response Reception	1422

A.6.2.10.2.3	Receiving a NACK on msg3	1422

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1422

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1422

A.6.2.10.2.6	Contention Resolution Timer expiry	1422

A.6.2.10.2.7	PRACH Resource Selection	1423

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1423

A.6.2.11.1	Test Purpose and Environment	1423

A.6.2.11.2	Test Requirements	1425

A.6.2.11.2.1	Random Access Response Reception	1425

A.6.2.11.2.2	No Random Access Response Reception	1426

A.6.2.11.2.3	Receiving a NACK on msg3	1426

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1426

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1426

A.6.2.11.2.6	Contention Resolution Timer expiry	1426

A.6.2.11.2.7	PRACH Resource Selection	1427

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1427

A.6.2.12.1	Test Purpose and Environment	1427

A.6.2.12.2	Test Requirements	1429

A.6.2.12.2.1	Random Access Response Reception	1429

A.6.2.12.2.2	No Random Access Response Reception	1430

A.6.2.12.2.3	Receiving a NACK on msg3	1430

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1430

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1430

A.6.2.12.2.6	Contention Resolution Timer expiry	1430

A.6.2.12.2.7	PRACH Resource Selection	1431

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1431

A.6.2.13.1	Test Purpose and Environment	1431

A.6.2.13.2	Test Requirements	1433

A.6.2.13.2.1	Random Access Response Reception	1433

A.6.2.13.2.2	No Random Access Response Reception	1434

A.6.2.13.2.3	Receiving a NACK on msg3	1434

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1434

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1434

A.6.2.13.2.6	Contention Resolution Timer expiry	1434

A.6.2.13.2.7	PRACH Resource Selection	1435

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1435

A.6.2.14.1	Test Purpose and Environment	1435

A.6.2.14.2	Test Requirements	1437

A.6.2.14.2.1	Random Access Response Reception	1437

A.6.2.14.2.2	No Random Access Response Reception	1438

A.6.2.14.2.3	Receiving a NACK on msg3	1438

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1438

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1438

A.6.2.14.2.6	Contention Resolution Timer expiry	1438

A.6.2.14.2.7	PRACH Resource Selection	1439

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1439

A.6.2.15.1	Test Purpose and Environment	1439

A.6.2.15.2	Test Requirements	1441

A.6.2.15.2.1	Random Access Response Reception	1441

A.6.2.15.2.2	No Random Access Response Reception	1442

A.6.2.15.2.3	Receiving a NACK on msg3	1442

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1442

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1442

A.6.2.15.2.6	Contention Resolution Timer expiry	1442

A.6.2.15.2.7	PRACH Resource Selection	1443

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1443

A.6.2.16.1	Test Purpose and Environment	1443

A.6.2.16.2	Test Requirements	1446

A.6.2.16.2.1	Random Access Response Reception	1446

A.6.2.16.2.2	No Random Access Response Reception	1447

A.6.2.16.2.3	Receiving a NACK on msg3	1447

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1447

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1447

A.6.2.16.2.6	Contention Resolution Timer expiry	1447

A.6.2.16.2.7	NPRACH Resource Selection	1447

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1448

A.6.2.17.1	Test Purpose and Environment	1448

A.6.2.17.2	Test Requirements	1451

A.6.2.17.2.1	Random Access Response Reception	1451

A.6.2.17.2.2	No Random Access Response Reception	1452

A.6.2.17.2.3	Receiving a NACK on msg3	1452

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1452

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1452

A.6.2.17.2.6	Contention Resolution Timer expiry	1452

A.6.2.17.2.7	NPRACH Resource Selection	1452

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1453

A.6.2.18.1	Test Purpose and Environment	1453

A.6.2.18.2	Test Requirements	1456

A.6.2.18.2.1	Random Access Response Reception	1456

A.6.2.18.2.2	No Random Access Response Reception	1457

A.6.2.18.2.3	Receiving a NACK on msg3	1457

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1457

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1457

A.6.2.18.2.6	Contention Resolution Timer expiry	1457

A.6.2.18.2.7	NPRACH Resource Selection	1457

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1458

A.6.2.19.1	Test Purpose and Environment	1458

A.6.2.19.2	Test Requirements	1460

A.6.2.19.2.1	Random Access Response Reception	1460

A.6.2.19.2.2	No Random Access Response Reception	1461

A.6.2.19.2.3	Receiving a NACK on msg3	1461

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1461

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1461

A.6.2.19.2.6	Contention Resolution Timer expiry	1461

A.6.2.19.2.7	NPRACH Resource Selection	1461

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1462

A.6.2.20.1	Test Purpose and Environment	1462

A.6.2.20.2	Test Requirements	1464

A.6.2.20.2.1	Random Access Response Reception	1464

A.6.2.20.2.2	No Random Access Response Reception	1465

A.6.2.20.2.3	Receiving a NACK on msg3	1465

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1465

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1465

A.6.2.20.2.6	Contention Resolution Timer expiry	1465

A.6.2.20.2.7	NPRACH Resource Selection	1465

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1466

A.6.2.21.1	Test Purpose and Environment	1466

A.6.2.21.2	Test Requirements	1468

A.6.2.21.2.1	Random Access Response Reception	1468

A.6.2.21.2.2	No Random Access Response Reception	1469

A.6.2.21.2.3	Receiving a NACK on msg3	1469

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1469

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1469

A.6.2.21.2.6	Contention Resolution Timer expiry	1469

A.6.2.21.2.7	NPRACH Resource Selection	1469

A.6.3	RRC Connection Release with Redirection	1470

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1470

A.6.3.1.1	Test Purpose and Environment	1470

A.6.3.1.2	Test Requirements	1472

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1472

A.6.3.2.1	Test Purpose and Environment	1472

A.6.3.2.2	Test Requirements	1474

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1474

A.6.3.3.1	Test Purpose and Environment	1474

A.6.3.3.2	Test Requirements	1475

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1476

A.6.3.4.1	Test Purpose and Environment	1476

A.6.3.4.2	Test Requirements	1477

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1478

A.6.3.5.1	Test Purpose and Environment	1478

A.6.3.5.2	Test Requirements	1480

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1480

A.6.3.6.1	Test Purpose and Environment	1480

A. 6.3.6.2	Test Requirements	1483

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1483

A.6.3.7.1	Test Purpose and Environment	1483

A.6.3.7.2	Test Requirements	1486

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1486

A.6.3.8.1	Test Purpose and Environment	1486

A.6.3.8.2	Test Requirements	1489

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1489

A.6.3.9.1	Test Purpose and Environment	1489

A.6.3.9.2	Test Requirements	1491

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1491

A.6.3.10.1	Test Purpose and Environment	1491

A.6.3.10.2	Test Requirements	1493

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1493

A.6.3.11.1	Test Purpose and Environment	1493

A.6.3.11.2	Test Requirements	1495

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1495

A.6.3.12.1	Test Purpose and Environment	1495

A.6.3.12.2	Test Requirements	1498

A.6.3.13	Redirection from E-UTRA to NR FR1 for redcap UE	1498

A.6.3.13.1	Test Purpose and Environment	1498

A.6.3.13.2	Test Parameters	1498

A.6.3.13.3	Test Requirements	1502

A.7	Timing and Signalling Characteristics	1503

A.7.1	UE Transmit Timing	1503

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1503

A.7.1.1.1	Test Purpose and Environment	1503

A.7.1.1.2	Test Requirements	1504

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1505

A.7.1.2.1	Test Purpose and Environment	1505

A.7.1.2.2	Test Requirements	1507

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1508

A.7.1.3.1	Test Purpose and Environment	1508

A.7.1.3.2	Test Requirements	1510

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1511

A.7.1.4.1	Test Purpose and Environment	1511

A.7.1.4.2	Test Requirements	1512

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1513

A.7.1.4A.1	Test Purpose and Environment	1513

A.7.1.4A.2	Test Requirements	1513

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1513

A.7.1.5.1	Test Purpose and Environment	1513

A.7.1.5.2	Test Requirements	1514

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1514

A.7.1.6.1	Test Purpose and Environment	1514

A.7.1.6.2	Test Requirements	1516

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1517

A.7.1.7.1	Test Purpose and Environment	1517

A.7.1.7.2	Test Requirements	1518

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1519

A.7.1.7A.1	Test Purpose and Environment	1519

A.7.1.7A.2	Test Requirements	1519

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1519

A.7.1.7B.1	Test Purpose and Environment	1519

A.7.1.7B.2	Test Requirements	1519

A.7.1.8	Void	1520

A.7.1.8.1	Void	1520

A.7.1.8.2	Void	1520

A.7.1.9	Void	1520

A.7.1.9.1	Void	1520

A.7.1.9.2	Void	1520

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1520

A.7.1.10.1	Test Purpose and Environment	1520

A.7.1.10.2	Test Requirements	1522

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1522

A.7.1.11.1	Test Purpose and Environment	1522

A.7.1.11.2	Test Requirements	1524

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1524

A.7.1.12.1	Test Purpose and Environment	1524

A.7.1.12.2	Test Requirements	1526

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1526

A.7.1.13.1	Test Purpose and Environment	1526

A.7.1.13.2	Test Requirements	1528

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1529

A.7.1.14.1	Test Purpose and Environment	1529

A.7.1.14.2	Test Requirements	1530

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1530

A.7.1.15.1	Test Purpose and Environment	1530

A.7.1.15.2	Test Requirements	1531

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1532

A.7.1.16.1	Test Purpose and Environment	1532

A.7.1.16.2	Test Requirements	1533

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1534

A.7.1.17.1	Test Purpose and Environment	1534

A.7.1.17.2	Test Requirements	1536

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1537

A.7.1.18.1	Test Purpose and Environment	1537

A.7.1.18.2	Test Requirements	1539

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1540

A.7.1.19.1	Test Purpose and Environment	1540

A.7.1.19.2	Test Requirements	1541

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1541

A.7.1.20.1	Test Purpose and Environment	1541

A.7.1.20.2	Test Requirements	1543

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1543

A.7.1.21.1	Test Purpose and Environment	1543

A.7.1.21.2	Test Requirements	1545

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1545

A.7.1.22.1	Test Purpose and Environment	1545

A.7.1.22.2	Test Requirements	1547

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1547

A.7.1.23.1	Test Purpose and Environment	1547

A.7.1.23.2	Test Requirements	1549

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1549

A.7.1.24.1	Test Purpose and Environment	1549

A.7.1.24.2	Test Requirements	1550

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1551

A.7.1.25.1	Test Purpose and Environment	1551

A.7.1.25.2	Test Requirements	1552

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1553

A.7.1.26.1	Test Purpose and Environment	1553

A.7.1.26.2	Test Requirements	1554

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1555

A.7.1.27.1	Test Purpose and Environment	1555

A.7.1.27.2	Test Requirements	1557

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1557

A.7.1.28.1	Test Purpose and Environment	1557

A.7.1.28.2	Test Requirements	1559

A.7.2	UE Timing Advance	1560

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1560

A.7.2.1.1	Test Purpose and Environment	1560

A.7.2.1.2	Test Requirements	1561

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1562

A.7.2.2.1	Test Purpose and Environment	1562

A.7.2.2.2	Test Requirements	1564

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1564

A.7.2.3.1	Test Purpose and Environment	1564

A.7.2.3.2	Test Requirements	1564

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1564

A.7.2.4.1	Test Purpose and Environment	1564

A.7.2.4.2	Test Requirements	1567

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1567

A.7.2.5.1	Test Purpose and Environment	1567

A.7.2.5.2	Test Requirements	1569

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1569

A.7.2.5A.1	Test Purpose and Environment	1569

A.7.2.5A.2	Test Requirements	1570

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1570

A.7.2.5B.1	Test Purpose and Environment	1570

A.7.2.5B.2	Test Requirements	1570

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1570

A.7.2.6.1	Test Purpose and Environment	1570

A.7.2.6.2	Test Requirements	1573

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1573

A.7.2.7.1	Test Purpose and Environment	1573

A.7.2.7.2	Test Requirements	1575

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1575

A.7.2.8.1	Test Purpose and Environment	1575

A.7.2.8.2	Test Requirements	1577

A.7.2.9.2	Test Requirements	1578

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1579

A.7.2.10.1	Test Purpose and Environment	1579

A.7.2.10.2	Test Requirements	1580

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1580

A.7.2.11.1	Test Purpose and Environment	1580

A.7.2.11.2	Test Requirements	1582

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1582

A.7.2.12.1	Test Purpose and Environment	1582

A.7.2.12.2	Test Requirements	1584

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1585

A.7.2.13.1	Test Purpose and Environment	1585

A.7.2.13.2	Test Requirements	1586

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1587

A.7.2.14.1	Test Purpose and Environment	1587

A.7.2.14.2	Test Requirements	1589

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1589

A.7.2.15.1	Test Purpose and Environment	1589

A.7.2.15.2	Test Requirements	1591

A.7.3	Radio Link Monitoring	1591

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1591

A.7.3.1.1	Test Purpose and Environment	1591

A.7.3.1.2	Test Requirements	1595

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1595

A.7.3.2.1	Test Purpose and Environment	1595

A.7.3.2.2	Test Requirements	1599

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1599

A.7.3.3.1	Test Purpose and Environment	1599

A.7.3.3.2	Test Requirements	1603

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1603

A.7.3.4.1	Test Purpose and Environment	1603

A.7.3.4.2	Test Requirements	1607

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1607

A.7.3.5.1	Test Purpose and Environment	1607

A.7.3.5.2	Test Requirements	1610

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1611

A.7.3.6.1	Test Purpose and Environment	1611

A.7.3.6.2	Test Requirements	1614

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1614

A.7.3.7.1	Test Purpose and Environment	1614

A.7.3.7.2	Test Requirements	1617

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1617

A.7.3.8.1	Test Purpose and Environment	1617

A.7.3.8.2	Test Requirements	1620

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1620

A.7.3.9.2	Test Requirements	1623

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1623

A.7.3.10.1	Test Purpose and Environment	1623

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1627

A.7.3.11.1	Test Purpose and Environment	1627

A.7.3.11.2	Test Requirements	1632

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1632

A.7.3.12.1	Test Purpose and Environment	1632

A.7.3.12.2	Test Requirements	1637

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1637

A.7.3.13.1	Test Purpose and Environment	1637

A.7.3.13.2	Test Requirements	1640

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1640

A.7.3.14.1	Test Purpose and Environment	1640

A.7.3.14.2	Test Requirements	1643

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1643

A.7.3.15.1	Test Purpose and Environment	1643

A.7.3.15.2	Test Requirements	1647

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1647

A.7.3.16.1	Test Purpose and Environment	1647

A.7.3.16.2	Test Requirements	1652

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1652

A.7.3.17.1	Test Purpose and Environment	1652

A.7.3.17.2	Test Requirements	1656

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1656

A.7.3.18.1	Test Purpose and Environment	1656

A.7.3.18.2	Test Requirements	1660

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1660

A.7.3.19.1	Test Purpose and Environment	1660

A.7.3.19.2	Test Requirements	1665

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1665

A.7.3.20.1	Test Purpose and Environment	1665

A.7.3.20.2	Test Requirements	1670

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1670

A.7.3.21.1	Test Purpose and Environment	1670

A.7.3.21.2	Test Requirements	1675

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1675

A.7.3.22.1	Test Purpose and Environment	1675

A.7.3.22.2	Test Requirements	1680

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1680

A.7.3.23.1	Test Purpose and Environment	1680

A.7.3.23.2	Test Requirements	1681

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1681

A.7.3.24.1	Test Purpose and Environment	1681

A.7.3.24.2	Test Requirements	1682

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1682

A.7.3.25.1	Test Purpose and Environment	1682

A.7.3.25.2	Test Requirements	1683

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1683

A.7.3.26.1	Test Purpose and Environment	1683

A.7.3.26.2	Test Requirements	1686

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1686

A.7.3.27.1	Test Purpose and Environment	1686

A.7.3.27.2	Test Requirements	1689

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1689

A.7.3.28.1	Test Purpose and Environment	1689

A.7.3.28.2	Test Requirements	1692

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1692

A.7.3.29.1	Test Purpose and Environment	1692

A.7.3.29.2	Test Requirements	1695

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1695

A.7.3.30.1	Test Purpose and Environment	1695

A.7.3.30.2	Test Requirements	1698

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1698

A.7.3.31.1	Test Purpose and Environment	1698

A.7.3.31.2	Test Requirements	1701

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1701

A.7.3.32.1	Test Purpose and Environment	1701

A.7.3.32.2	Test Requirements	1704

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1704

A.7.3.33.1	Test Purpose and Environment	1704

A.7.3.33.2	Test Requirements	1707

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1707

A.7.3.34.1	Test Purpose and Environment	1707

A.7.3.34.2	Test Requirements	1710

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1710

A.7.3.35.1	Test Purpose and Environment	1710

A.7.3.35.2	Test Requirements	1713

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1713

A.7.3.36.1	Test Purpose and Environment	1713

A.7.3.36.2	Test Requirements	1716

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1716

A.7.3.37.1	Test Purpose and Environment	1716

A.7.3.37.2	Test Requirements	1719

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1719

A.7.3.38.1	Test Purpose and Environment	1719

A.7.3.38.2	Test Requirements	1722

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1722

A.7.3.39.1	Test Purpose and Environment	1722

A.7.3.39.2	Test Requirements	1725

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1726

A.7.3.40.1	Test Purpose and Environment	1726

A.7.3.40.2	Test Requirements	1728

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1729

A.7.3.41.1	Test Purpose and Environment	1729

A.7.3.41.2	Test Requirements	1732

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1732

A.7.3.42.1	Test Purpose and Environment	1732

A.7.3.42.2	Test Requirements	1735

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1735

A.7.3.43.1	Test Purpose and Environment	1735

A.7.3.43.2	Test Requirements	1738

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1738

A.7.3.44.1	Test Purpose and Environment	1738

A.7.3.44.2	Test Requirements	1741

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1742

A.7.3.45.1	Test Purpose and Environment	1742

A.7.3.45.2	Test Requirements	1744

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1745

A.7.3.46.1	Test Purpose and Environment	1745

A.7.3.46.2	Test Requirements	1748

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1748

A.7.3.47.1	Test Purpose and Environment	1748

A.7.3.47.2	Test Requirements	1751

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1751

A.7.3.48.1	Test Purpose and Environment	1751

A.7.3.48.2	Test Requirements	1754

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1754

A.7.3.49.1	Test Purpose and Environment	1754

A.7.3.49.2	Test Requirements	1757

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1757

A.7.3.50.1	Test Purpose and Environment	1757

A.7.3.50.2	Test Requirements	1760

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1760

A.7.3.51.1	Test Purpose and Environment	1760

A.7.3.51.2	Test Requirements	1763

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1763

A.7.3.52.1	Test Purpose and Environment	1763

A.7.3.52.2	Test Requirements	1766

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1766

A.7.3.53.1	Test Purpose and Environment	1766

A.7.3.53.2	Test Requirements	1769

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1769

A.7.3.54.1	Test Purpose and Environment	1769

A.7.3.54.2	Test Requirements	1772

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1772

A.7.3.55.1	Test Purpose and Environment	1772

A.7.3.55.2	Test Requirements	1775

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1775

A.7.3.56.1	Test Purpose and Environment	1775

A.7.3.56.2	Test Requirements	1778

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1778

A.7.3.57.1	Test Purpose and Environment	1778

A.7.3.57.2	Test Requirements	1781

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1781

A.7.3.58.1	Test Purpose and Environment	1781

A.7.3.58.2	Test Requirements	1784

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1784

A.7.3.59.1	Test Purpose and Environment	1784

A.7.3.59.2	Test Requirements	1787

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1787

A.7.3.60.1	Test Purpose and Environment	1787

A.7.3.60.2	Test Requirements	1791

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1791

A.7.3.61.1	Test Purpose and Environment	1791

A.7.3.61.2	Test Requirements	1795

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1795

A.7.3.62.1	Test Purpose and Environment	1795

A.7.3.62.2	Test Requirements	1799

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1799

A.7.3.63.1	Test Purpose and Environment	1799

A.7.3.63.2	Test Requirements	1803

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1803

A.7.3.64.1	Test Purpose and Environment	1803

A.7.3.64.2	Test Requirements	1806

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1807

A.7.3.65.1	Test Purpose and Environment	1807

A.7.3.65.2	Test Requirements	1810

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1811

A.7.3.66.1	Test Purpose and Environment	1811

A.7.3.66.2	Test Requirements	1813

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1814

A.7.3.67.1	Test Purpose and Environment	1814

A.7.3.67.2	Test Requirements	1817

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1818

A.7.3.68.1	Test Purpose and Environment	1818

A.7.3.68.2	Test Requirements	1820

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1820

A.7.3.69.1	Test Purpose and Environment	1820

A.7.3.69.2	Test Requirements	1822

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1822

A.7.3.70.1	Test Purpose and Environment	1822

A.7.3.70.2	Test Requirements	1824

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1824

A.7.3.71.1	Test Purpose and Environment	1824

A.7.3.71.2	Test Requirements	1827

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1827

A.7.3.72.1	Test Purpose and Environment	1827

A.7.3.72.2	Test Requirements	1829

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1829

A.7.3.73.1	Test Purpose and Environment	1829

A.7.3.73.2	Test Requirements	1831

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1831

A.7.3.74.1	Test Purpose and Environment	1831

A.7.3.74.2	Test Requirements	1834

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1834

A.7.3.75.1	Test Purpose and Environment	1834

A.7.3.75.2	Test Requirements	1836

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1837

A.7.3.76.1	Test Purpose and Environment	1837

A.7.3.76.2	Test Requirements	1840

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1840

A.7.3.77.1	Test Purpose and Environment	1840

A.7.3.77.2	Test Requirements	1843

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1843

A.7.3.78.1	Test Purpose and Environment	1843

A.7.3.78.2	Test Requirements	1846

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1846

A.7.3.79.1	Test Purpose and Environment	1846

A.7.3.79.2	Test Requirements	1849

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1849

A.7.3.80.1	Test Purpose and Environment	1849

A.7.3.80.2	Test Requirements	1852

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1852

A.7.3.81.1	Test Purpose and Environment	1852

A.7.3.81.2	Test Requirements	1855

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1855

A.7.3.82.1	Test Purpose and Environment	1855

A.7.3.82.2	Test Requirements	1858

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1858

A.7.3.83.1	Test Purpose and Environment	1858

A.7.3.83.2	Test Requirements	1859

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1860

A.7.3.84.1	Test Purpose and Environment	1860

A.7.3.84.2	Test Requirements	1862

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1862

A.7.3.85.1	Test Purpose and Environment	1862

A.7.3.85.2	Test Requirements	1863

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1864

A.7.3.86.1	Test Purpose and Environment	1864

A.7.3.86.2	Test Requirements	1866

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1866

A.7.3.87.1	Test Purpose and Environment	1866

A.7.3.87.2	Test Requirements	1868

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1869

A.7.3.88.1	Test Purpose and Environment	1869

A.7.3.88.2	Test Requirements	1873

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1873

A.7.3.89.1	Test Purpose and Environment	1873

A.7.3.89.2	Test Requirements	1877

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1877

A.7.3.90.1	Test Purpose and Environment	1877

A.7.3.90.2	Test Requirements	1881

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1881

A.7.3.91.1	Test Purpose and Environment	1881

A.7.3.91.2	Test Requirements	1885

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1885

A.7.3.92.1	Test Purpose and Environment	1885

A.7.3.92.2	Test Requirements	1888

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1889

A.7.3.93.1	Test Purpose and Environment	1889

A.7.3.93.2	Test Requirements	1892

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1893

A.7.3.94.1	Test Purpose and Environment	1893

A.7.3.94.2	Test Requirements	1895

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1896

A.7.3.95.1	Test Purpose and Environment	1896

A.7.3.95.2	Test Requirements	1899

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1900

A.7.3.96.1	Test Purpose and Environment	1900

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1901

A.7.3.97.1	Test Purpose and Environment	1901

A.7.3.97.2	Test Requirements	1904

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1904

A.7.3.98.1	Test Purpose and Environment	1904

A.7.3.98.2	Test Requirements	1907

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1907

A.7.3.99.1	Test Purpose and Environment	1907

A.7.3.99.2	Test Requirements	1910

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1910

A.7.3.100.1	Test Purpose and Environment	1910

A.7.3.100.2	Test Requirements	1912

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1912

A.7.3.101.1	Test Purpose and Environment	1912

A.7.3.101.2	Test Requirements	1914

A.7.4	Interruption for Dual Connectivity	1914

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1914

A.7.4.1.1	Test Purpose and Environment	1914

A.7.4.1.2	Test Requirements	1917

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1917

A.7.4.2.1	Test Purpose and Environment	1917

A.7.4.2.2	Test Requirements	1919

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1919

A.7.4.3.1	Test Purpose and Environment	1919

A.7.4.3.2	Test Requirements	1921

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1921

A.7.4.4.1	Test Purpose and Environment	1921

A.7.4.4.2	Test Requirements	1924

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1924

A.7.4.5.1	Test Purpose and Environment	1924

A.7.4.5.2	Test Requirements	1926

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1926

A.7.4.6.1	Test Purpose and Environment	1926

A.7.4.6.2	Test Requirements	1928

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1928

A.7.4.7.1	Test Purpose and Environment	1928

A.7.4.7.2	Test Requirements	1932

A.7.5	Proximity-based Services	1932

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1932

A.7.5.1.1	Test Purpose and Environment	1932

A.7.5.1.2	Test Requirements	1933

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1934

A.7.5.2.1	Test Purpose and Environment	1934

A.7.5.1.2	Test Requirements	1934

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1935

A.7.5.3.1	Test Purpose and Environment	1935

A.7.5.3.2	Test Requirements	1936

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1937

A.7.5.4.1	Test Purpose and Environment	1937

A.7.5.4.2	Test Requirements	1938

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1939

A.7.5.5.1	Test Purpose and Environment	1939

A.7.5.5.2	Test Requirements	1941

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1942

A.7.5.6.1	Test Purpose and Environment	1942

A.7.5.6.2	Test Requirements	1943

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1944

A.7.5.7.1	Test Purpose and Environment	1944

A.7.5.7.2	Test Requirements	1946

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1946

A.7.5.8.1	Test Purpose and Environment	1946

A.7.5.8.2	Test Requirements	1948

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1949

A.7.5.9.1	Test Purpose and Environment	1949

A.7.5.9.2	Test Requirements	1951

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1952

A.7.5.10.1	Test Purpose and Environment	1952

A.7.5.10.2	Test Requirements	1954

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1955

A.7.5.11.1	Test Purpose and Environment	1955

A.7.5.11.2	Test Requirements	1957

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1957

A.7.5.12.1	Test Purpose and Environment	1957

A.7.5.12.2	Test Requirements	1961

A.7.6	Interruption for carrier aggregation	1962

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1962

A.7.6.1.1	Test Purpose and Environment	1962

A.7.6.1.2	Test Requirements	1965

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1965

A.7.6.2.1	Test Purpose and Environment	1965

A.7.6.2.2	Test Requirements	1968

A.8	UE Measurements Procedures	1969

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1969

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1969

A.8.1.1.1	Test Purpose and Environment	1969

A.8.1.1.2	Test Requirements	1970

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1971

A.8.1.2.1	Test Purpose and Environment	1971

A.8.1.2.2	Test Requirements	1972

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1973

A.8.1.3.1	Test Purpose and Environment	1973

A.8.1.3.2	Test Requirements	1975

A.8.1.4	Void	1975

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1975

A.8.1.5.1	Test Purpose and Environment	1975

A.8.1.5.2	Test Requirements	1977

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1977

A.8.1.6.1	Test Purpose and Environment	1977

A.8.1.6.2	Test Requirements	1979

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1979

A.8.1.7.1	Test Purpose and Environment	1979

A.8.1.7.2	Test Requirements	1981

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1982

A.8.1.8.1	Test Purpose and Environment	1982

A.8.1.8.2	Test Requirements	1984

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1985

A.8.1.9.1	Test Purpose and Environment	1985

A.8.1.9.2	Test Requirements	1985

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1985

A.8.1.10.1	Test Purpose and Environment	1985

A.8.1.10.2	Test Requirements	1986

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1986

A.8.1.11.1	Test Purpose and Environment	1986

A.8.1.11.2	Test Requirements	1988

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1989

A.8.1.12.1	Test Purpose and Environment	1989

A.8.1.12.2	Test Requirements	1990

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1991

A.8.1.13.1	Test Purpose and Environment	1991

A.8.1.13.2	Test Requirements	1993

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1993

A.8.1.14.1	Test Purpose and Environment	1993

A.8.1.14.2	Test Requirements	1995

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1996

A.8.1.15.1	Test Purpose and Environment	1996

A.8.1.15.2	Test Requirements	1997

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1998

A.8.1.16.1	Test Purpose and Environment	1998

A.8.1.16.2	Test Requirements	2000

A.8.1.17	Void	2000

A.8.1.18	Void	2000

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2000

A.8.1.19.1	Test Purpose and Environment	2000

A.8.1.19.2	Test Requirements	2002

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2003

A.8.1.20.1	Test Purpose and Environment	2003

A.8.1.20.2	Test Requirements	2005

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2005

A.8.1.21.1	Test Purpose and Environment	2005

A.8.1.21.2	Test Requirements	2007

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2008

A.8.1.22.1	Test Purpose and Environment	2008

A.8.1.22.2	Test Requirements	2010

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2010

A.8.1.23.1	Test Purpose and Environment	2010

A.8.1.23.2	Test Requirements	2011

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2012

A.8.1.24.1	Test Purpose and Environment	2012

A.8.1.24.2	Test Requirements	2014

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2014

A.8.1.25.1	Test Purpose and Environment	2014

A.8.1.25.2	Test Requirements	2016

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2016

A.8.1.26.1	Test Purpose and Environment	2016

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2019

A.8.1.27.1	Test Purpose and Environment	2019

A.8.1.27.2	Test Requirements	2021

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2021

A.8.1.28.1	Test Purpose and Environment	2021

A.8.1.28.2	Test Requirements	2023

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2023

A.8.1.29.1	Test Purpose and Environment	2023

A.8.1.29.2	Test Requirements	2025

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2026

A.8.1.30.1	Test Purpose and Environment	2026

A.8.1.30.2	Test Requirements	2028

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2028

A.8.1.31.1	Test Purpose and Environment	2028

A.8.1.31.2	Test Requirements	2030

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2031

A.8.1.32.1	Test Purpose and Environment	2031

A.8.1.32.2	Test Requirements	2032

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2033

A.8.1.33.1	Test Purpose and Environment	2033

A.8.1.33.2	Test Requirements	2034

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2035

A.8.1.34.1	Test Purpose and Environment	2035

A.8.1.34.2	Test Requirements	2036

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2037

A.8.1.35.1	Test Purpose and Environment	2037

A.8.1.35.2	Test Requirements	2038

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2039

A.8.1.36.1	Test Purpose and Environment	2039

A.8.1.36.2	Test Requirements	2040

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2041

A.8.1.37.1	Test Purpose and Environment	2041

A.8.1.37.2	Test Requirements	2043

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2043

A.8.1.38.1	Test Purpose and Environment	2043

A.8.1.38.2	Test Requirements	2044

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2045

A.8.1.39.1	Test Purpose and Environment	2045

A.8.1.39.2	Test Requirements	2047

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2047

A.8.1.40.1	Test Purpose and Environment	2047

A.8.1.40.2	Test Requirements	2049

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2049

A.8.1.41.1	Test Purpose and Environment	2049

A.8.1.41.2	Test Requirement	2051

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2051

A.8.1.42.1	Test Purpose and Environment	2051

A.8.1.42.2	Test Requirement	2052

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2052

A.8.1.43.1	Test Purpose and Environment	2052

A.8.1.43.2	Test Requirements	2055

A.8.1.44	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2055

A.8.1.44.1	Test Purpose and Environment	2055

A.8.1.44.2	Test Requirements	2058

A.8.1.45	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2059

A.8.1.45.1	Test Purpose and Environment	2059

A.8.1.45.2	Test Requirements	2061

A.8.1.46	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2062

A.8.1.46.1	Test Purpose and Environment	2062

A.8.1.46.2	Test Requirements	2063

A.8.1.47	TDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2064

A.8.1.47.1	Test Purpose and Environment	2064

A.8.1.47.2	Test Requirements	2066

A.8.1.48	TDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2067

A.8.1.48.1	Test Purpose and Environment	2067

A.8.1.48.2	Test Requirements	2069

A.8.1.49	TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2070

A.8.1.49.1	Test Purpose and Environment	2070

A.8.1.49.2	Test Requirements	2071

A.8.150	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2072

A.8.1.50.1	Test Purpose and Environment	2072

A.8.1.50.2	Test Requirements	2074

A.8.1.51	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2075

A.8.1.51.1	Test Purpose and Environment	2075

A.8.1.51.2	Test Requirements	2077

A.8.1.52	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2078

A.8.1.52.1	Test Purpose and Environment	2078

A.8.1.52.2	Test Requirements	2079

A.8.1.53	TDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2080

A.8.1.53.1	Test Purpose and Environment	2080

A.8.1.53.2	Test Requirements	2082

A.8.1.54	TDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2083

A.8.1.54.1	Test Purpose and Environment	2083

A.8.1.54.2	Test Requirements	2085

A.8.1.55	TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2086

A.8.1.55.1	Test Purpose and Environment	2086

A.8.1.55.2	Test Requirements	2087

A.8.2	E-UTRAN TDD Intra-frequency Measurements	2088

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2088

A.8.2.1.1	Test Purpose and Environment	2088

A.8.2.1.2	Test Requirements	2089

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2090

A.8.2.2.1	Test Purpose and Environment	2090

A.8.2.2.2	Test Requirements	2092

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2092

A.8.2.3.1	Test Purpose and Environment	2092

A.8.2.3.2	Test Requirements	2094

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2095

A.8.2.4.1	Test Purpose and Environment	2095

A.8.2.4.2	Test Requirements	2097

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2097

A.8.2.5.1	Test Purpose and Environment	2097

A.8.2.5.2	Test Requirements	2099

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2100

A.8.2.6.1	Test Purpose and Environment	2100

A.8.2.6.2	Test Requirements	2103

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2104

A.8.2.7.1	Test Purpose and Environment	2104

A.8.2.7.2	Test Requirements	2105

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2106

A.8.2.8.1	Test Purpose and Environment	2106

A.8.2.8.2	Test Requirements	2108

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2108

A.8.2.9.1	Test Purpose and Environment	2108

A.8.2.9.2	Test Requirements	2110

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2111

A.8.2.10.1	Test Purpose and Environment	2111

A.8.2.10.2	Test Requirements	2113

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2113

A.8.2.11.1	Test Purpose and Environment	2113

A.8.2.11.2	Test Requirements	2115

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2115

A.8.2.12.1	Test Purpose and Environment	2115

A.8.2.12.2	Test Requirements	2117

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2118

A.8.2.13.1	Test Purpose and Environment	2118

A.8.2.13.2	Test Requirements	2120

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2120

A.8.2.14.1	Test Purpose and Environment	2120

A.8.2.14.2	Test Requirement	2122

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2122

A.8.2.15.1	Test Purpose and Environment	2122

A.8.2.15.2	Test Requirements	2124

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2124

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2124

A.8.3.1.1	Test Purpose and Environment	2124

A.8.3.1.2	Test Requirements	2126

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2126

A.8.3.2.1	Test Purpose and Environment	2126

A.8.3.2.2	Test Requirements	2129

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2129

A.8.3.3.1	Test Purpose and Environment	2129

A.8.3.3.2	Test Requirements	2132

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2132

A.8.3.4.1	Test Purpose and Environment	2132

A.8.3.4.2	Test Requirements	2134

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2134

A.8.3.5.2	Test Requirements	2136

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2136

A.8.3.6.1	Test Purpose and Environment	2136

A.8.3.6.2	Test Requirements	2138

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2139

A.8.3.7.1	Test Purpose and Environment	2139

A.8.3.7.2	Test Requirements	2142

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2142

A.8.3.8.1	Test Purpose and Environment	2142

A.8.3.8.2	Test Requirements	2145

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2145

A.8.3.9.1	Test Purpose and Environment	2145

A.8.3.9.2	Test Requirements	2149

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2150

A.8.3.10.1	Test Purpose and Environment	2150

A.8.3.10.2	Test Requirements	2151

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2151

A.8.3.11.1	Test Purpose and Environment	2151

A.8.3.11.2	Test Requirement	2153

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2154

A.8.3.12.1	Test Purpose and Environment	2154

A.8.3.12.2	Test Requirement	2155

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2156

A.8.3.13.1	Test Purpose and Environment	2156

A.8.3.13.2	Test Requirement	2157

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2158

A.8.3.14.1	Test Purpose and Environment	2158

A.8.3.14.2	Test Requirement	2159

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2160

A.8.3.15.1	Test Purpose and Environment	2160

A.8.3.15.2	Test Requirement	2161

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2162

A.8.3.16.1	Test Purpose and Environment	2162

A.8.3.16.2	Test Requirement	2164

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2164

A.8.3.17.1	Test Purpose and Environment	2164

A.8.3.17.2	Test Requirement	2167

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2167

A.8.3.18.1	Test Purpose and Environment	2167

A.8.3.18.2	Test Requirement	2170

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2170

A.8.3.19.1	Test Purpose and Environment	2170

A.8.3.19.2	Test Requirement	2173

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2173

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2173

A.8.4.1.1	Test Purpose and Environment	2173

A.8.4.1.2	Test Requirements	2175

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2175

A.8.4.2.1	Test Purpose and Environment	2175

A.8.4.2.2	Test Requirements	2178

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2178

A.8.4.3.1	Test Purpose and Environment	2178

A.8.4.3.2	Test Requirements	2180

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2181

A.8.4.4.1	Test Purpose and Environment	2181

A.8.4.4.2	Test Requirements	2183

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2184

A.8.4.5.1	Test Purpose and Environment	2184

A.8.4.5.2	Test Requirements	2186

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2186

A.8.4.6.1	Test Purpose and Environment	2186

A.8.4.6.2	Test Requirements	2187

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2187

A.8.4.7.1	Test Purpose and Environment	2187

A.8.4.7.2	Test Requirements	2191

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2191

A.8.4.8.1	Test Purpose and Environment	2191

A.8.4.8.2	Test Requirements	2194

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2194

A.8.4.9.1	Test Purpose and Environment	2194

A.8.4.9.2	Test Requirements	2199

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2199

A.8.4.10.1	Test Purpose and Environment	2199

A.8.4.10.2	Test Requirements	2201

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2201

A.8.4.11.1	Test Purpose and Environment	2201

A.8.4.11.2	Test Requirement	2203

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2203

A.8.4.12.1	Test Purpose and Environment	2203

A.8.4.12.2	Test Requirement	2205

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2206

A.8.4.13.1	Test Purpose and Environment	2206

A.8.4.13.2	Test Requirement	2207

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2208

A.8.4.14.1	Test Purpose and Environment	2208

A.8.4.14.2	Test Requirement	2210

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2210

A.8.4.15.1	Test Purpose and Environment	2210

A.8.4.15.2	Test Requirement	2213

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2213

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2213

A.8.5.1.1	Test Purpose and Environment	2213

A.8.5.1.2	Test Requirements	2215

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2215

A.8.5.2.1	Test Purpose and Environment	2215

A.8.5.2.2	Test Requirements	2217

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2217

A.8.5.3.1	Test Purpose and Environment	2217

A.8.5.3.2	Test Requirements	2220

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2220

A.8.5.4.1	Test Purpose and Environment	2220

A.8.5.4.2	Test Requirements	2222

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2222

A.8.5.5.1	Test Purpose and Environment	2222

A.8.5.5.2	Test Requirements	2225

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2225

A.8.5.6.1	Test Purpose and Environment	2225

A.8.5.6.2	Test Requirements	2226

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2227

A.8.5.7.1	Test Purpose and Environment	2227

A.8.5.7.2	Test Requirements	2227

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2227

A.8.5.8.1	Test Purpose and Environment	2227

A.8.5.8.2	Test Requirements	2230

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2230

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2230

A.8.6.1.1	Test Purpose and Environment	2230

A.8.6.1.2	Test Requirements	2232

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2232

A.8.6.2.1	Test Purpose and Environment	2232

A.8.6.2.2	Test Requirements	2235

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2235

A.8.6.3.1	Test Purpose and Environment	2235

A.8.6.3.2	Test Requirements	2238

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2238

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2238

A.8.7.1.1	Test Purpose and Environment	2238

A.8.7.1.1.1	Void	2238

A.8.7.1.1.2	1.28 Mcps TDD option	2238

A.8.7.1.1.3	Void	2240

A.8.7.1.2	Test Requirements	2240

A.8.7.1.2.1	Void	2240

A.8.7.1.2.2	1.28 Mcps TDD option	2240

A.8.7.1.2.3	Void	2240

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2240

A.8.7.2.1	Test Purpose and Environment	2240

A.8.7.2.2	Test Requirements	2243

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2244

A.8.7.3.1	Test Purpose and Environment	2244

A.8.7.3.2	Test Parameters	2244

A.8.7.3.3	Test Requirements	2245

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2246

A.8.7.4.1	Test Purpose and Environment	2246

A.8.7.4.2	Test Requirements	2248

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2248

A.8.7.5.1	Test Purpose and Environment	2248

A.8.7.5.2	Test Requirements	2250

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2250

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2250

A.8.7A.1.1	Test Purpose and Environment	2250

A.8.7A.1.2	Test Requirements	2253

A.8.8	E-UTRAN FDD – GSM Measurements	2253

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2253

A.8.8.1.1	Test Purpose and Environment	2253

A.8.8.1.2	Test Requirements	2255

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2255

A.8.8.2.1	Test Purpose and Environment	2255

A.8.8.2.2	Test Requirements	2257

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2258

A.8.8.3.1	Test Purpose and Environment	2258

A.8.8.3.2	Test Requirements	2259

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2260

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2260

A.8.9.1.1	Test Purpose and Environment	2260

A.8.9.1.2	Test Requirements	2261

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2262

A.8.9.2.1	Test Purpose and Environment	2262

A.8.9.2.2	Test Requirements	2264

A.8.10	E-UTRAN TDD – GSM Measurements	2264

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2264

A.8.10.1.1	Test Purpose and Environment	2264

A.8.10.1.2	Test Requirements	2265

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2266

A.8.10.2.1	Test Purpose and Environment	2266

A.8.10.2.2	Test Requirements	2268

A.8.11	Monitoring of Multiple Layers	2268

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2268

A.8.11.1.1	Test Purpose and Environment	2268

A.8.11.1.2	Test Requirements	2270

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2271

A.8.11.2.1	Test Purpose and Environment	2271

A.8.11.2.2	Test Requirements	2272

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2273

A.8.11.3.1	Test Purpose and Environment	2273

A.8.11.3.2	Test Requirements	2275

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2275

A.8.11.4.1	Test Purpose and Environment	2275

A.8.11.4.2	Test Requirements	2278

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2278

A.8.11.5.1	Test Purpose and Environment	2278

A.8.11.5.2	Test Requirements	2280

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2281

A.8.11.6.1	Test Purpose and Environment	2281

A.8.11.6.2	Test Requirements	2283

A.8.12	RSTD Intra-frequency Measurements	2284

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2284

A.8.12.1.1	Test Purpose and Environment	2284

A.8.12.1.2	Test Requirements	2288

A.8.12.1.2A	Test Requirements for UE Category 1bis	2288

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2288

A.8.12.2.1	Test Purpose and Environment	2288

A.8.12.2.2	Test Requirements	2293

A.8.12.2.2A	Test Requirements for UE Category 1bis	2293

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2293

A.8.12.3.1	Test Purpose and Environment	2293

A.8.12.3.2	Test Requirements	2299

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2299

A.8.12.4.1	Test Purpose and Environment	2299

A.8.12.4.2	Test Requirements	2304

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2304

A.8.12.5.1	Test Purpose and Environment	2304

A.8.12.5.2	Test Requirements	2309

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2309

A.8.12.6.1	Test Purpose and Environment	2309

A.8.12.6.2	Test Requirements	2314

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2314

A.8.12.7.1	Test Purpose and Environment	2314

A.8.12.7.2	Test Requirements	2319

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2319

A.8.12.8.1	Test Purpose and Environment	2319

A.8.12.8.2	Test Requirements	2324

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2324

A.8.12.9.1	Test Purpose and Environment	2324

A.8.12.9.2	Test Requirements	2329

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2329

A.8.12.10.1	Test Purpose and Environment	2329

A.8.12.10.2	Test Requirements	2334

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2334

A.8.12.11.1	Test Purpose and Environment	2334

A.8.12.11.2	Test Requirements	2339

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2339

A.8.12.12.1	Test Purpose and Environment	2339

A.8.12.12.2	Test Requirements	2344

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2344

A.8.12.13.1	Test Purpose and Environment	2344

A.8.12.13.2	Test Requirements	2349

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2349

A.8.12.14.1	Test Purpose and Environment	2349

A.8.12.14.2	Test Requirements	2354

A.8.13	RSTD Inter-frequency Measurements	2354

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2354

A.8.13.1.1	Test Purpose and Environment	2354

A.8.13.1.2	Test Requirements	2359

A.8.13.1.2A	Test Requirements for UE Category 1bis	2359

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2359

A.8.13.2.1	Test Purpose and Environment	2359

A.8.13.2.2	Test Requirements	2365

A.8.13.2.2A	Test Requirements for UE Category 1bis	2365

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2365

A.8.13.3.1	Test Purpose and Environment	2365

A.8.13.3.2	Test Requirements	2372

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2372

A.8.13.4.1	Test Purpose and Environment	2372

A.8.13.4.2	Test Requirements	2378

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2378

A.8.13.5.1	Test Purpose and Environment	2378

A.8.13.5.2	Test Requirements	2384

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2384

A.8.13.6.1	Test Purpose and Environment	2384

A.8.13.6.2	Test Requirements	2390

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2390

A.8.13.7.1	Test Purpose and Environment	2390

A.8.13.7.2	Test Requirements	2396

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2396

A.8.13.8.1	Test Purpose and Environment	2396

A.8.13.8.2	Test Requirements	2402

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2402

A.8.13.9.1	Test Purpose and Environment	2402

A.8.13.9.2	Test Requirements	2408

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2408

A.8.13.10.1	Test Purpose and Environment	2408

A.8.13.10.2	Test Requirements	2414

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2414

A.8.13.11.1	Test Purpose and Environment	2414

A.8.13.11.2	Test Requirements	2420

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2420

A.8.13.12.1	Test Purpose and Environment	2420

A.8.13.12.2	Test Requirements	2426

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2426

A.8.13.13.1	Test Purpose and Environment	2426

A.8.13.13.2	Test Requirements	2432

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2432

A.8.13.14.1	Test Purpose and Environment	2432

A.8.13.14.2	Test Requirements	2438

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2438

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2438

A.8.14.1.1	Test Purpose and Environment	2438

A.8.14.1.2	Test Requirements	2440

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2440

A.8.14.2.1	Test Purpose and Environment	2440

A.8.14.2.2	Test Requirements	2443

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2443

A.8.14.3.1	Test Purpose and Environment	2443

A.8.14.3.2	Test Requirements	2445

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2446

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2446

A.8.15.1.1	Test Purpose and Environment	2446

A.8.15.1.2	Test Requirements	2447

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2447

A.8.15.2.1	Test Purpose and Environment	2447

A.8.15.2.2	Test Requirements	2450

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2450

A.8.15.3.1	Test Purpose and Environment	2450

A.8.15.3.2	Test Requirements	2452

A.8.16	E-UTRAN Carrier Aggregation Measurements	2453

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2453

A.8.16.1.1	Test Purpose and Environment	2453

A.8.16.1.2	Test Requirements	2455

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2455

A.8.16.2.1	Test Purpose and Environment	2455

A.8.16.2.2	Test Requirements	2457

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2457

A.8.16.3.1	Test Purpose and Environment	2457

A.8.16.3.2	Test Requirements	2459

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2460

A.8.16.3A.1	Test Purpose and Environment	2460

A.8.16.3A.2	Test Requirements	2461

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2462

A.8.16.4.1	Test Purpose and Environment	2462

A.8.16.4.2	Test Requirements	2463

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2464

A.8.16.4A.1	Test Purpose and Environment	2464

A.8.16.4A.2	Test Requirements	2466

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2467

A.8.16.5.1	Test Purpose and Environment	2467

A.8.16.5.2	Test Requirements	2467

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2467

A.8.16.6.1	Test Purpose and Environment	2467

A.8.16.6.2	Test Requirements	2468

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2468

A.8.16.7.1	 Test Purpose and Environment	2468

A.8.16.7.2	 Test Requirements	2469

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2469

A.8.16.8.1	 Test Purpose and Environment	2469

A.8.16.8.2	 Test Requirements	2470

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2470

A.8.16.9.1	Test Purpose and Environment	2470

A.8.16.9.2	Test Requirements	2471

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2471

A.8.16.10.1	Test Purpose and Environment	2471

A.8.16.10.2	Test Requirements	2472

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2472

A.8.16.11.1	 Test Purpose and Environment	2472

A.8.16.11.2	 Test Requirements	2473

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2473

A.8.16.12.1	 Test Purpose and Environment	2473

A.8.16.12.2	 Test Requirements	2473

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2473

A.8.16.13.1	Test Purpose and Environment	2473

A.8.16.13.2	Test Requirements	2474

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2474

A.8.16.14.1	Test Purpose and Environment	2474

A.8.16.14.2	Test Requirements	2475

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2475

A.8.16.15.1	 Test Purpose and Environment	2475

A.8.16.7.2	 Test Requirements	2475

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2475

A.8.16.16.1	 Test Purpose and Environment	2475

A.8.16.16.2	 Test Requirements	2476

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2476

A.8.16.17.1	Test Purpose and Environment	2476

A.8.16.17.2	Test Requirements	2478

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2479

A.8.16.17A.1	Test Purpose and Environment	2479

A.8.16.17A.2	Test Requirements	2479

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2479

A.8.16.17B.1	Test Purpose and Environment	2479

A.8.16.17B.2	Test Requirements	2480

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2480

A.8.16.17C.1	Test Purpose and Environment	2480

A.8.16.17C.2	Test Requirements	2480

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2480

A.8.16.18.1	Test Purpose and Environment	2480

A.8.16.18.2	Test Requirements	2482

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2483

A.8.16.18A.1	Test Purpose and Environment	2483

A.8.16.18A.2	Test Requirements	2483

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2483

A.8.16.18B.1	Test Purpose and Environment	2483

A.8.16.18B.2	Test Requirements	2484

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2484

A.8.16.18C.1	Test Purpose and Environment	2484

A.8.16.18C.2	Test Requirements	2484

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2484

A.8.16.18D.1	Test Purpose and Environment	2484

A.8.16.18D.2	Test Requirements	2485

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2485

A.8.16.19.1	Test Purpose and Environment	2485

A.8.16.19.2	Test Requirements	2487

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2487

A.8.16.19A.1	Test Purpose and Environment	2487

A.8.16.19A.2	Test Requirements	2488

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2488

A.8.16.19B.1	Test Purpose and Environment	2488

A.8.16.19B.2	Test Requirements	2488

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2488

A.8.16.19C.1	Test Purpose and Environment	2488

A.8.16.19C.2	Test Requirements	2489

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2489

A.8.16.20.1	Test Purpose and Environment	2489

A.8.16.20.2	Test Requirements	2491

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2492

A.8.16.20A.1	Test Purpose and Environment	2492

A.8.16.20A.2	Test Requirements	2492

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2492

A.8.16.20B.1	Test Purpose and Environment	2492

A.8.16.20B.2	Test Requirements	2493

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2493

A.8.16.20C.1	Test Purpose and Environment	2493

A.8.16.20C.2	Test Requirements	2493

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2493

A.8.16.20D.1	Test Purpose and Environment	2493

A.8.16.20D.2	Test Requirements	2494

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2494

A.8.16.21.1	Test Purpose and Environment	2494

A.8.16.21.2	Test Requirements	2496

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2496

A.8.16.22.1	Test Purpose and Environment	2496

A.8.16.22.2	Test Requirements	2498

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2498

A.8.16.23.1	Test Purpose and Environment	2498

A.8.16.23.2	Test Requirements	2501

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2501

A.8.16.24.1	Test Purpose and Environment	2501

A.8.16.24.2	Test Requirements	2504

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2504

A.8.16.25.1	Test Purpose and Environment	2504

A.8.16.25.2	Test Requirements	2507

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2507

A.8.16.26.1	Test Purpose and Environment	2507

A.8.16.26.2	Test Requirements	2510

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2510

A.8.16.27.1	Test Purpose and Environment	2510

A.8.16.27.2	Test Requirements	2515

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2515

A.8.16.28.1	Test Purpose and Environment	2515

A.8.16.28.2	Test Requirements	2520

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2520

A.8.16.29.1	Test Purpose and Environment	2520

A.8.16.29.2	Test Requirements	2525

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2525

A.8.16.30.1	Test Purpose and Environment	2525

A.8.16.30.2	Test Requirements	2530

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2530

A.8.16.31.1	Test Purpose and Environment	2530

A.8.16.31.2	Test Requirements	2535

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2535

A.8.16.32.1	Test Purpose and Environment	2535

A.8.16.32.2	Test Requirements	2540

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2540

A.8.16.33.1	Test Purpose and Environment	2540

A.8.16.33.2	Test Requirements	2545

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2545

A.8.16.34.1	Test Purpose and Environment	2545

A.8.16.34.2	Test Requirements	2550

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2550

A.8.16.35.1	Test Purpose and Environment	2550

A.8.16.35.2	Test Requirements	2552

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2553

A.8.16.36.1	Test Purpose and Environment	2553

A.8.16.36.2	Test Requirements	2555

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2556

A.8.16.37.1	Test Purpose and Environment	2556

A.8.16.37.2	Test Requirements	2558

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2559

A.8.16.38.1	Test Purpose and Environment	2559

A.8.16.38.2	Test Requirements	2561

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2562

A.8.16.39.1	Test Purpose and Environment	2562

A.8.16.39.2	Test Requirements	2565

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2565

A.8.16.40.1	Test Purpose and Environment	2565

A.8.16.40.2	Test Requirements	2568

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2568

A.8.16.41.1	Test Purpose and Environment	2568

A.8.16.41.2	Test Requirements	2570

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2571

A.8.16.42.1	Test Purpose and Environment	2571

A.8.16.42.2	Test Requirements	2573

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2574

A.8.16.43.1	Test Purpose and Environment	2574

A.8.16.43.2	Test Requirements	2577

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2577

A.8.16.44.1	Test Purpose and Environment	2577

A.8.16.44.2	Test Requirements	2580

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2580

A.8.16.45.1	Test Purpose and Environment	2580

A.8.16.45.2	Test Requirements	2583

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2583

A.8.16.46.1	Test Purpose and Environment	2583

A.8.16.46.2	Test Requirements	2585

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2586

A.8.16.47.1	Test Purpose and Environment	2586

A.8.16.47.2	Test Requirements	2588

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2589

A.8.16.48.1	Test Purpose and Environment	2589

A.8.16.48.2	Test Requirements	2591

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2592

A.8.16.49.1	Test Purpose and Environment	2592

A.8.16.49.2	Test Requirements	2595

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2595

A.8.16.50.1	Test Purpose and Environment	2595

A.8.16.50.2	Test Requirements	2597

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2598

A.8.16.51.1	Test Purpose and Environment	2598

A.8.16.51.2	Test Requirements	2603

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2603

A.8.16.52.1	Test Purpose and Environment	2603

A.8.16.52.2	Test Requirements	2608

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2608

A.8.16.53.1	Test Purpose and Environment	2608

A.8.16.53.2	Test Requirements	2612

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2612

A.8.16.54.1	Test Purpose and Environment	2612

A.8.16.54.2	Test Requirements	2616

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2616

A.8.16.55.1	Test Purpose and Environment	2616

A.8.16.55.2	Test Requirements	2622

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2622

A.8.16.56.1	Test Purpose and Environment	2622

A.8.16.56.2	Test Requirements	2628

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2628

A.8.16.57.1	Test Purpose and Environment	2628

A.8.16.57.2	Test Requirements	2630

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2631

A.8.16.58.1	Test Purpose and Environment	2631

A.8.16.58.2	Test Requirements	2633

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2634

A.8.16.59.1	Test Purpose and Environment	2634

A.8.16.59.2	Test Requirements	2638

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2639

A.8.16.60.1	Test Purpose and Environment	2639

A.8.16.60.2	Test Requirements	2642

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2643

A.8.16.61.1	Test Purpose and Environment	2643

A.8.16.61.2	Test Requirements	2646

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2646

A.8.16.62.1	Test Purpose and Environment	2646

A.8.16.62.2	Test Requirements	2648

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2649

A.8.16.63.1	Test Purpose and Environment	2649

A.8.16.63.2	Test Requirements	2652

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2653

A.8.16.64.1	Test Purpose and Environment	2653

A.8.16.64.2	Test Requirements	2657

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2657

A.8.16.65.1	Test Purpose and Environment	2657

A.8.16.65.2	Test Requirements	2662

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2662

A.8.16.66.1	Test Purpose and Environment	2662

A.8.16.66.2	Test Requirements	2667

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2667

A.8.16.67.1	Test Purpose and Environment	2667

A.8.16.67.2	Test Requirements	2672

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2672

A.8.16.68.1	Test Purpose and Environment	2672

A.8.16.68.2	Test Requirements	2677

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2677

A.8.16.69.1	Test Purpose and Environment	2677

A.8.16.69.2	Test Requirements	2681

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2681

A.8.16.70.1	Test Purpose and Environment	2681

A.8.16.70.2	Test Requirements	2685

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2685

A.8.16.71.1	Test Purpose and Environment	2685

A.8.16.71.2	Test Requirements	2690

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2690

A.8.16.72.1	Test Purpose and Environment	2690

A.8.16.72.2	Test Requirements	2695

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2695

A.8.16.73.1	Test Purpose and Environment	2695

A.8.16.73.2	Test Requirements	2702

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2702

A.8.16.74.1	Test Purpose and Environment	2702

A.8.16.74.2	Test Requirements	2708

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2708

A.8.16.75.1	Test Purpose and Environment	2708

A.8.16.75.2	Test Requirements	2714

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2714

A.8.16.76.1	Test Purpose and Environment	2714

A.8.16.76.2	Test Requirements	2720

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2720

A.8.16.77.1	Test Purpose and Environment	2720

A.8.16.77.2	Test Requirements	2724

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2724

A.8.16.78.1	Test Purpose and Environment	2724

A.8.16.78.2	Test Requirements	2726

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2727

A.8.16.79.1	Test Purpose and Environment	2727

A.8.16.79.2	Test Requirements	2734

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2734

A.8.16.80.1	Test Purpose and Environment	2734

A.8.16.80.2	Test Requirements	2741

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2741

A.8.16.81.1	Test Purpose and Environment	2741

A.8.16.81.2	Test Requirements	2747

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2747

A.8.16.82.1	Test Purpose and Environment	2747

A.8.16.82.2	Test Requirements	2753

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2753

A.8.16.83.1	Test Purpose and Environment	2753

A.8.16.83.2	Test Requirements	2758

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2758

A.8.16.84.1	Test Purpose and Environment	2758

A.8.16.84.2	Test Requirements	2764

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2764

A.8.16.85.1	Test Purpose and Environment	2764

A.8.16.85.2	Test Requirements	2768

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2768

A.8.16.86.1	Test Purpose and Environment	2768

A.8.16.86.2	Test Requirements	2771

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2771

A.8.16.87.1	Test Purpose and Environment	2771

A.8.16.87.2	Test Requirements	2777

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2777

A.8.16.88.1	Test Purpose and Environment	2777

A.8.16.88.2	Test Requirements	2785

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2785

A.8.16.89.1	Test Purpose and Environment	2785

A.8.16.89.2	Test Requirements	2792

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2792

A.8.16.90.1	Test Purpose and Environment	2792

A.8.16.90.2	Test Requirements	2799

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2799

A.8.16.91.1	Test Purpose and Environment	2799

A.8.16.91.2	Test Requirements	2805

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2805

A.8.16.92.1	Test Purpose and Environment	2805

A.8.16.92.2	Test Requirements	2813

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2813

A.8.16.93.1	Test Purpose and Environment	2813

A.8.16.93.2	Test Requirements	2820

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2820

A.8.16.94.1	Test Purpose and Environment	2820

A.8.16.94.2	Test Requirements	2827

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2827

A.8.16.95.1	Test Purpose and Environment	2827

A.8.16.95.2	Test Requirements	2835

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2835

A.8.16.96.1	Test Purpose and Environment	2835

A.8.16.96.2	Test Requirements	2845

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2845

A.8.16.97.1	Test Purpose and Environment	2845

A.8.16.97.2	Test Requirements	2852

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2852

A.8.16.98.1	Test Purpose and Environment	2852

A.8.16.98.2	Test Requirements	2860

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2860

A.8.16.99.1	Test Purpose and Environment	2860

A.8.16.99.2	Test Requirements	2870

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2870

A.8.16.100.1	Test Purpose and Environment	2870

A.8.16.100.2	Test Requirements	2880

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2880

A.8.16.101.1	Test Purpose and Environment	2880

A.8.16.101.2	Test Requirements	2890

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2890

A.8.16.102.1	Test Purpose and Environment	2890

A.8.16.102.2	Test Requirements	2901

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2901

A.8.16.103.1	Test Purpose and Environment	2901

A.8.16.103.2	Test Requirements	2906

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2907

A.8.16.104.1	Test Purpose and Environment	2907

A.8.16.104.2	Test Requirements	2911

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2912

A.8.16.105.1	Test Purpose and Environment	2912

A.8.16.105.2	Test Requirements	2916

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2917

A.8.16.106.1	Test Purpose and Environment	2917

A.8.16.106.2	Test Requirements	2921

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2921

A.8.16.107.1	Test Purpose and Environment	2921

A.8.16.107.2	Test Requirements	2923

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2924

A.8.16.108.1	Test Purpose and Environment	2924

A.8.16.108.2	Test Requirements	2926

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2927

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2927

A.8.17.1.1	Test Purpose and Environment	2927

A.8.17.1.2	Test Requirements	2933

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2933

A.8.17.2.1	Test Purpose and Environment	2933

A.8.17.2.2	Test Requirements	2940

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2940

A.8.17.3.1	Test Purpose and Environment	2940

A.8.17.3.2	Test Requirements	2941

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2941

A.8.17.4.1	Test Purpose and Environment	2941

A.8.17.4.2	Test Requirements	2942

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2942

A.8.17.5.1	Test Purpose and Environment	2942

A.8.17.5.2	Test Requirements	2943

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2943

A.8.17.6.1	Test Purpose and Environment	2943

A.8.17.6.2	Test Requirements	2944

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2945

A.8.17.7.1	Test Purpose and Environment	2945

A.8.17.7.2	Test Requirements	2945

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2946

A.8.17.8.1	Test Purpose and Environment	2946

A.8.17.8.2	Test Requirements	2946

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2947

A.8.17.9.1	Test Purpose and Environment	2947

A.8.17.9.2	Test Requirements	2948

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2948

A.8.17.10.1	Test Purpose and Environment	2948

A.8.17.10.2	Test Requirements	2955

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2956

A.8.17.11.1	Test Purpose and Environment	2956

A.8.17.11.2	Test Requirements	2962

A.8.18	E-UTRAN TDD – HRPD Measurements	2963

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2963

A.8.18.1.1	Test Purpose and Environment	2963

A.8.18.1.2	Test Requirements	2965

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2965

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2965

A.8.19.1.1	Test Purpose and Environment	2965

A.8.19.1.2	Test Requirements	2966

A.8.20	Inter-frequency/RAT Measurements in CA mode	2967

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2967

A.8.20.1.1	Test Purpose and Environment	2967

A.8.20.1.2	Test Requirements	2968

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2968

A.8.20.2.1	Test Purpose and Environment	2969

A.8.20.2.2	Test Requirements	2970

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2971

A.8.20.2A.1	Test Purpose and Environment	2971

A.8.20.2A.2	Test Requirements	2971

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2971

A.8.20.2B.1	Test Purpose and Environment	2971

A.8.20.2B.2	Test Requirements	2974

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2974

A.8.20.3.1	Test Purpose and Environment	2974

A.8.20.3.2	Test Requirements	2976

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2976

A.8.20.4.1	Test Purpose and Environment	2976

A.8.20.4.1.1	1.28 Mcps TDD option	2976

A.8.20.4.2	Test Requirements	2978

A.8.20.4.2.1	1.28 Mcps TDD option	2978

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2978

A.8.20.4A.1	Test Purpose and Environment	2978

A.8.20.4A.1.1	1.28 Mcps TDD option	2978

A.8.20.4A.2	Test Requirements	2978

A.8.20.4A.2.1	1.28 Mcps TDD option	2978

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2979

A.8.20.4B.1	Test Purpose and Environment	2979

A.8.20.4B.1.1	1.28 Mcps TDD option	2979

A.8.20.4B.2	Test Requirements	2981

A.8.20.4B.2.1	1.28 Mcps TDD option	2981

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2981

A.8.21.1	Test Purpose and Environment	2981

A.8.21.2 Test Requirements	2985

A.8.22	E-UTRAN Discovery Signal Measurements	2985

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2985

A.8.22.1.1	Test Purpose and Environment	2985

A.8.22.1.2	Test Requirements	2988

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2988

A.8.22.2.1	Test Purpose and Environment	2988

A.8.22.2.2	Test Requirements	2991

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2991

A.8.22.3.1	Test Purpose and Environment	2991

A.8.22.3.2	Test Requirements	2994

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2994

A.8.22.4.1	Test Purpose and Environment	2994

A.8.22.4.2	Test Requirements	2997

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2997

A.8.22.5.1	Test Purpose and Environment	2997

A.8.22.5.2	Test Requirements	3001

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3001

A.8.22.6.1	Test Purpose and Environment	3001

A.8.22.6.2	Test Requirements	3005

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3005

A.8.22.7.1	Test Purpose and Environment	3005

A.8.22.7.2	Test Requirements	3009

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	3009

A.8.22.8.1	Test Purpose and Environment	3009

A.8.22.8.2	Test Requirements	3013

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3013

A.8.22.9.1	Test Purpose and Environment	3013

A.8.22.9.2	Test Requirements	3015

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3016

A.8.22.10.1	Test Purpose and Environment	3016

A.8.22.10.2	Test Requirements	3018

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3019

A.8.22.11.1	Test Purpose and Environment	3019

A.8.22.11.2	Test Requirements	3022

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3022

A.8.22.12.1	Test Purpose and Environment	3022

A.8.22.12.2	Test Requirements	3026

A.8.23	E-UTRAN Dual Connectivity Measurements	3026

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3026

A.8.23.1.1	Test Purpose and Environment	3026

A.8.23.1.2	Test Requirements	3029

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	3029

A.8.23.2.1	Test Purpose and Environment	3029

A.8.23.2.2	Test Requirements	3032

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3032

A.8.23.3.1	Test Purpose and Environment	3032

A.8.23.3.2	Test Requirements	3035

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3035

A.8.23.4.1	Test Purpose and Environment	3035

A.8.23.4.2	Test Requirements	3038

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	3038

A.8.23.5.1	Test Purpose and Environment	3038

A.8.23.5.2	Test Requirements	3041

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3041

A.8.23.6.1	Test Purpose and Environment	3041

A.8.23.6.2	Test Requirements	3044

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	3044

A.8.23.7.1	Test Purpose and Environment	3044

A.8.23.7.2	Test Requirements	3046

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	3047

A.8.23.8.1	Test Purpose and Environment	3047

A.8.23.8.2	Test Requirements	3049

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	3050

A.8.23.9.1	Test Purpose and Environment	3050

A.8.23.9.2	Test Requirements	3053

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3053

A.8.23.10.1	Test Purpose and Environment	3053

A.8.23.10.2	Test Requirements	3056

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3056

A.8.23.11.1	Test Purpose and Environment	3056

A.8.23.11.2	Test Requirements	3059

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3059

A.8.23.12.1	Test Purpose and Environment	3059

A.8.23.12.2	Test Requirements	3062

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3062

A.8.23.13.1	Test Purpose and Environment	3062

A.8.23.13.2	Test Requirements	3065

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	3065

A.8.23.14.1	Test Purpose and Environment	3065

A.8.23.14.2	Test Requirements	3068

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	3068

A.8.23.15.1	Test Purpose and Environment	3068

A.8.23.15.2	Test Requirements	3071

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	3071

A.8.23.16.1	Test Purpose and Environment	3071

A.8.23.16.2	Test Requirements	3072

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	3073

A.8.23.17.1	Test Purpose and Environment	3073

A.8.23.17.2	Test Requirements	3075

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3075

A.8.23.18.1	Test Purpose and Environment	3075

A.8.23.18.2	Test Requirements	3076

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3077

A.8.23.19.1	Test Purpose and Environment	3077

A.8.23.19.2	Test Requirements	3078

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3079

A.8.23.20.1	Test Purpose and Environment	3079

A.8.23.20.2	Test Requirements	3080

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3081

A.8.23.21.1	Test Purpose and Environment	3081

A.8.23.21.2	Test Requirements	3082

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3083

A.8.23.22.1	Test Purpose and Environment	3083

A.8.23.22.2	Test Requirements	3084

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3085

A.8.23.23.1	Test Purpose and Environment	3085

A.8.23.23.2	Test Requirements	3086

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3087

A.8.23.24.1	Test Purpose and Environment	3087

A.8.23.24.2	Test Requirements	3089

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	3090

A.8.23.25.1	Test Purpose and Environment	3090

A.8.23.25.2	Test Requirements	3092

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3093

A.8.23.26.1	Test Purpose and Environment	3093

A.8.23.26.2	Test Requirements	3095

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3096

A.8.23.27.1	Test Purpose and Environment	3096

A.8.23.27.2	Test Requirements	3099

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3099

A.8.23.28.1	Test Purpose and Environment	3099

A.8.23.28.2	Test Requirements	3103

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3103

A.8.23.29.1	Test Purpose and Environment	3103

A.8.23.29.2	Test Requirements	3107

A.8.24	Proximity-based Services	3107

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3107

A.8.24.1.1	Test Purpose and Environment	3107

A.8.24.1.2	Test Requirements	3108

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3109

A.8.24.2.1	Test Purpose and Environment	3109

A.8.24.2.2	Test Requirements	3110

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3110

A.8.24.3.1	Test Purpose and Environment	3111

A.8.24.3.2	Test Requirements	3112

A.8.25	E-UTRAN-WLAN Measurements	3113

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3113

A.8.25.1.1	Test Purpose and Environment	3113

A.8.25.1.2	Test Requirements	3115

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3115

A.8.25.2.1	Test Purpose and Environment	3115

A.8.25.2.2	Test Requirements	3117

A.8.26 Frame Structure 3 (FS3)	3117

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3117

A.8.26.1.1	Test Purpose and Environment	3117

A.8.26.1.2	Test Requirements	3119

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3120

A.8.26.2.1	Test Purpose and Environment	3120

A.8.26.2.2	Test Requirements	3122

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3123

A.8.26.3.1	Test Purpose and Environment	3123

A.8.26.3.2	Test Requirements	3126

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3126

A.8.26.3A.1	Test Purpose and Environment	3126

A.8.26.3A.2	Test Requirements	3129

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3129

A.8.26.4.1	Test Purpose and Environment	3129

A.8.26.4.2	Test Requirements	3133

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3133

A.8.26.4A.1	Test Purpose and Environment	3133

A.8.26.4A.2	Test Requirements	3137

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3137

A.8.26.5.1	Test Purpose and Environment	3137

A.8.26.5.2	Test Requirements	3140

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3140

A.8.26.5A.1	Test Purpose and Environment	3140

A.8.26.5A.2	Test Requirements	3144

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3144

A.8.26.6.1	Test Purpose and Environment	3144

A.8.26.6.2	Test Requirements	3148

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3148

A.8.26.6A.1	Test Purpose and Environment	3148

A.8.26.6A.2	Test Requirements	3152

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3152

A.8.26.7.1	Test Purpose and Environment	3152

A.8.26.7.2	Test Requirements	3155

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3155

A.8.26.8.1	Test Purpose and Environment	3155

A.8.26.8.2	Test Requirements	3159

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3159

A.8.26.9.1	Test Purpose and Environment	3159

A.8.26.9.2	Test Requirements	3162

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3162

A.8.26.10.1	Test Purpose and Environment	3162

A.8.26.10.2	Test Requirements	3165

A.9	Measurement Performance Requirements	3165

A.9.1	RSRP	3165

A.9.1.1	FDD Intra frequency case	3165

A.9.1.1.1	Test Purpose and Environment	3165

A.9.1.1.2	Test parameters	3165

A.9.1.1.3	Test Requirements	3168

A.9.1.2	TDD Intra frequency case	3168

A.9.1.2.1	Test Purpose and Environment	3168

A.9.1.2.2	Test parameters	3168

A.9.1.2.3	Test Requirements	3169

A.9.1.3	FDD—FDD Inter frequency case	3170

A.9.1.3.1	Test Purpose and Environment	3170

A.9.1.3.2	Test parameters	3170

A.9.1.3.3	Test Requirements	3173

A.9.1.4	TDD—TDD Inter frequency case	3173

A.9.1.4.1	Test Purpose and Environment	3173

A.9.1.4.2	Test parameters	3173

A.9.1.4.3	Test Requirements	3175

A.9.1.5	FDD—TDD Inter frequency case	3176

A.9.1.5.1	Test Purpose and Environment	3176

A.9.1.5.2	Test parameters	3176

A.9.1.5.3	Test Requirements	3177

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	3178

A.9.1.6.1	Test Purpose and Environment	3178

A.9.1.6.2	Test parameters	3178

A.9.1.6.3	Test Requirements	3181

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	3181

A.9.1.7.1	Test Purpose and Environment	3181

A.9.1.7.2	Test parameters	3181

A.9.1.7.3	Test Requirements	3184

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3184

A.9.1.8.1	Test Purpose and Environment	3184

A.9.1.8.2	Test parameters	3184

A.9.1.8.3	Test Requirements	3188

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3188

A.9.1.9.1	Test Purpose and Environment	3188

A.9.1.9.2	Test parameters	3188

A.9.1.9.3	Test Requirements	3191

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3191

A.9.1.10.1	Test Purpose and Environment	3191

A.9.1.10.2	Test parameters	3191

A.9.1.10.3	Test Requirements	3194

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3194

A.9.1.11.1	Test Purpose and Environment	3194

A.9.1.11.2	Test parameters	3194

A.9.1.11.3	Test Requirements	3198

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3198

A.9.1.12.1	Test Purpose and Environment	3198

A.9.1.12.2	Test parameters	3198

A.9.1.12.3	Test Requirements	3199

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3199

A.9.1.13.1	Test Purpose and Environment	3199

A.9.1.13.2	Test parameters	3199

A.9.1.13.3	Test Requirements	3200

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3200

A.9.1.14.1	Test Purpose and Environment	3200

A.9.1.14.2	Test parameters	3200

A.9.1.14.3	Test Requirements	3204

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3204

A.9.1.15.1	Test Purpose and Environment	3204

A.9.1.15.2	Test parameters	3204

A.9.1.15.3	Test Requirements	3206

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	3207

A.9.1.16.1	Test Purpose and Environment	3207

A.9.1.16.2	Test parameters	3207

A.9.1.16.3	Test Requirements	3208

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	3209

A.9.1.17.1	Test Purpose and Environment	3209

A.9.1.17.2	Test parameters	3209

A.9.1.17.3	Test Requirements	3210

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3211

A.9.1.18.1	Test Purpose and Environment	3211

A.9.1.18.2	Test parameters	3211

A.9.1.18.3	Test Requirements	3212

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3213

A.9.1.19.1	Test Purpose and Environment	3213

A.9.1.19.2	Test parameters	3213

A.9.1.19.3	Test Requirements	3213

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3213

A.9.1.20.1	Test Purpose and Environment	3213

A.9.1.20.2	Test parameters	3213

A.9.1.20.3	Test Requirements	3215

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3215

A.9.1.21.1	Test Purpose and Environment	3215

A.9.1.21.2	Test parameters	3215

A.9.1.21.3	Test Requirements	3215

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3215

A.9.1.22.1	Test Purpose and Environment	3215

A.9.1.22.2	Test parameters	3216

A.9.1.22.3	Test Requirements	3220

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3220

A.9.1.23.1	Test Purpose and Environment	3220

A.9.1.23.2	Test parameters	3220

A.9.1.23.3	Test Requirements	3224

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3224

A.9.1.24.1	Test Purpose and Environment	3224

A.9.1.24.2	Test parameters	3224

A.9.1.24.3	Test Requirements	3225

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3225

A.9.1.25.1	Test Purpose and Environment	3225

A.9.1.25.2	Test parameters	3225

A.9.1.25.3	Test Requirements	3228

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3228

A.9.1.26.1	Test Purpose and Environment	3228

A.9.1.26.2	Test parameters	3228

A.9.1.26.3	Test Requirements	3231

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3231

A.9.1.27.1	Test Purpose and Environment	3231

A.9.1.27.2	Test parameters	3231

A.9.1.27.3	Test Requirements	3234

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3234

A.9.1.28.1	Test Purpose and Environment	3234

A.9.1.28.2	Test parameters	3234

A.9.1.28.3	Test Requirements	3237

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3237

A.9.1.29.1	Test Purpose and Environment	3237

A.9.1.29.2	Test parameters	3237

A.9.1.29.3	Test Requirements	3240

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3240

A.9.1.30.1	Test Purpose and Environment	3240

A.9.1.30.2	Test parameters	3241

A.9.1.30.3	Test Requirements	3244

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3244

A.9.1.31.1	Test Purpose and Environment	3244

A.9.1.31.2	Test parameters	3244

A.9.1.31.3	Test Requirements	3247

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3248

A.9.1.32.1	Test Purpose and Environment	3248

A.9.1.32.2	Test parameters	3248

A.9.1.32.3	Test Requirements	3251

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3251

A.9.1.33.1	Test Purpose and Environment	3251

A.9.1.33.2	Test parameters	3251

A.9.1.33.3	Test Requirements	3254

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3254

A.9.1.34.1	Test Purpose and Environment	3254

A.9.1.34.2	Test parameters	3254

A.9.1.34.3	Test Requirements	3257

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3257

A.9.1.35.1	Test Purpose and Environment	3257

A.9.1.35.2	Test parameters	3257

A.9.1.35.3	Test Requirements	3260

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3261

A.9.1.36.1	Test Purpose and Environment	3261

A.9.1.36.2	Test parameters	3261

A.9.1.36.3	Test Requirements	3264

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3264

A.9.1.37.1	Test Purpose and Environment	3264

A.9.1.37.2	Test parameters	3264

A.9.1.37.3	Test Requirements	3267

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3268

A.9.1.38.1	Test Purpose and Environment	3268

A.9.1.38.2	Test parameters	3268

A.9.1.38.3	Test Requirements	3271

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3272

A.9.1.39.1	Test Purpose and Environment	3272

A.9.1.39.2	Test parameters	3272

A.9.1.39.3	Test Requirements	3278

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3278

A.9.1.40.1	Test Purpose and Environment	3278

A.9.1.40.2	Test parameters	3279

A.9.1.40.3	Test Requirements	3285

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3285

A.9.1.41.1	Test Purpose and Environment	3285

A.9.1.41.2	Test parameters	3285

A.9.1.41.3	Test Requirements	3288

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3288

A.9.1.42.1	Test Purpose and Environment	3288

A.9.1.42.2	Test parameters	3288

A.9.1.42.3	Test Requirements	3291

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3291

A.9.1.43.1	Test Purpose and Environment	3291

A.9.1.43.2	Test parameters	3291

A.9.1.43.3	Test Requirements	3292

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3293

A.9.1.44.1	Test Purpose and Environment	3293

A.9.1.44.2	Test parameters	3293

A.9.1.44.3	Test Requirements	3298

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3298

A.9.1.45.1	Test Purpose and Environment	3298

A.9.1.45.2	Test parameters	3298

A.9.1.45.3	Test Requirements	3303

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3303

A.9.1.46.1	Test Purpose and Environment	3303

A.9.1.46.2	Test parameters	3303

A.9.1.46.3	Test Requirements	3312

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3313

A.9.1.47.1	Test Purpose and Environment	3313

A.9.1.47.2	Test parameters	3313

A.9.1.47.3	Test Requirements	3322

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3322

A.9.1.48.1	Test Purpose and Environment	3322

A.9.1.48.2	Test parameters	3323

A.9.1.48.3	Test Requirements	3333

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3334

A.9.1.49.1	Test Purpose and Environment	3334

A.9.1.49.2	Test parameters	3334

A.9.1.49.3	Test Requirements	3344

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3345

A.9.1.50.1	Test Purpose and Environment	3345

A.9.1.50.2	Test parameters	3345

A.9.1.50.3	Test Requirements	3352

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3353

A.9.1.51.1	Test Purpose and Environment	3353

A.9.1.51.2	Test parameters	3353

A.9.1.51.3	Test Requirements	3359

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3360

A.9.1.52.1	Test Purpose and Environment	3360

A.9.1.52.2	Test parameters	3360

A.9.1.52.3	Test Requirements	3363

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3363

A.9.1.52A.1	Test Purpose and Environment	3363

A.9.1.52A.2	Test parameters	3363

A.9.1.52A.3	Test Requirements	3364

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3364

A.9.1.53.1	Test Purpose and Environment	3364

A.9.1.53.2	Test parameters	3365

A.9.1.53.3	Test Requirements	3368

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3368

A.9.1.53A.1	Test Purpose and Environment	3368

A.9.1.53A.2	Test parameters	3368

A.9.1.53A.3	Test Requirements	3369

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3370

A.9.1.54.1	Test Purpose and Environment	3370

A.9.1.54.2	Test parameters	3370

A.9.1.54.3	Test Requirements	3373

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	3373

A.9.1.55.1	Test Purpose and Environment	3373

A.9.1.55.2	Test parameters	3373

A.9.1.55.3	Test Requirements	3377

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	3377

A.9.1.56.1	Test Purpose and Environment	3377

A.9.1.56.2	Test parameters	3377

A.9.1.56.3	Test Requirements	3380

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3381

A.9.1.57.1	Test Purpose and Environment	3381

A.9.1.57.2	Test parameters	3381

A.9.1.57.3	Test Requirements	3384

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3384

A.9.1.57A.1	Test Purpose and Environment	3384

A.9.1.57A.2	Test parameters	3384

A.9.1.57A.3	Test Requirements	3385

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3385

A.9.1.58.1	Test Purpose and Environment	3385

A.9.1.58.2	Test parameters	3386

A.9.1.58.3	Test Requirements	3389

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3389

A.9.1.58A.1	Test Purpose and Environment	3389

A.9.1.58A.2	Test parameters	3389

A.9.1.58A.3	Test Requirements	3390

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3390

A.9.1.59.1	Test Purpose and Environment	3390

A.9.1.59.2	Test parameters	3391

A.9.1.59.3	Test Requirements	3393

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	3393

A.9.1.60.1	Test Purpose and Environment	3393

A.9.1.60.2	Test parameters	3393

A.9.1.60.3	Test Requirements	3396

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	3397

A.9.1.61.1	Test Purpose and Environment	3397

A.9.1.61.2	Test parameters	3397

A.9.1.61.3	Test Requirements	3400

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3400

A.9.1.62.1	Test Purpose and Environment	3400

A.9.1.62.2	Test parameters	3400

A.9.1.62.3	Test Requirements	3403

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3403

A.9.1.53.1	Test Purpose and Environment	3403

A.9.1.53.2	Test parameters	3403

A.9.1.63.3	Test Requirements	3406

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3406

A.9.1.64.1	Test Purpose and Environment	3406

A.9.1.64.2	Test parameters	3406

A.9.1.64.3	Test Requirements	3407

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3408

A.9.1.65.1	Test Purpose and Environment	3408

A.9.1.65.2	Test parameters	3408

A.9.1.65.3	Test Requirements	3414

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3414

A.9.1.66.1	Test Purpose and Environment	3414

A.9.1.66.2	Test parameters	3414

A.9.1.66.3	Test Requirements	3420

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3420

A.9.1.67.1	Test Purpose and Environment	3420

A.9.1.67.2	Test parameters	3420

A.9.1.67.3	Test Requirements	3421

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3422

A.9.1.68.1	Test Purpose and Environment	3422

A.9.1.68.2	Test parameters	3422

A.9.1.68.3	Test Requirements	3426

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3426

A.9.1.69.1	Test Purpose and Environment	3426

A.9.1.69.2	Test parameters	3426

A.9.1.69.3	Test Requirements	3430

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3431

A.9.1.70.1	Test Purpose and Environment	3431

A.9.1.70.2	Test parameters	3431

A.9.1.70.3	Test Requirements	3443

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3444

A.9.1.71.1	Test Purpose and Environment	3444

A.9.1.71.2	Test parameters	3444

A.9.1.71.3	Test Requirements	3456

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3457

A.9.1.72.1	Test Purpose and Environment	3457

A.9.1.72.2	Test parameters	3457

A.9.1.72.3	Test Requirements	3469

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	3470

A.9.1.73.1	Test Purpose and Environment	3470

A.9.1.73.2	Test parameters	3470

A.9.1.73.3	Test Requirements	3473

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	3473

A.9.1.74.1	Test Purpose and Environment	3473

A.9.1.74.2	Test parameters	3473

A.9.1.74.3	Test Requirements	3474

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	3474

A.9.1.75.1	Test Purpose and Environment	3474

A.9.1.75.2	Test parameters	3475

A.9.1.75.3	Test Requirements	3476

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3476

A.9.1.76.1	Test Purpose and Environment	3476

A.9.1.76.2	Test parameters	3476

A.9.1.76.3	Test Requirements	3479

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3479

A.9.1.77.1	Test Purpose and Environment	3479

A.9.1.77.2	Test parameters	3479

A.9.1.77.3	Test Requirements	3482

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3482

A.9.1.78.1	Test Purpose and Environment	3482

A.9.1.78.2	Test parameters	3482

A.9.1.78.3	Test Requirements	3485

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3485

A.9.1.79.1	Test Purpose and Environment	3485

A.9.1.79.2	Test parameters	3485

A.9.1.79.3	Test Requirements	3488

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3488

A.9.1.80.1	Test Purpose and Environment	3488

A.9.1.80.2	Test parameters	3488

A.9.1.80.3	Test Requirements	3491

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3491

A.9.1.81.1	Test Purpose and Environment	3491

A.9.1.78.2	Test parameters	3491

A.9.1.81.3	Test Requirements	3494

A.9.2	RSRQ	3494

A.9.2.1	FDD Intra frequency case	3494

A.9.2.1.1	Test Purpose and Environment	3494

A.9.2.1.2	Test parameters	3494

A.9.2.1.3	Test Requirements	3497

A.9.2.2	TDD Intra frequency case	3497

A.9.2.2.1	Test Purpose and Environment	3497

A.9.2.2.2	Test parameters	3497

A.9.2.2.3	Test Requirements	3499

A.9.2.3	FDD—FDD Inter frequency case	3499

A.9.2.3.1	Test Purpose and Environment	3499

A.9.2.3.2	Test parameters	3499

A.9.2.3.3	Test Requirements	3502

A.9.2.4	TDD—TDD Inter frequency case	3502

A.9.2.4.1	Test Purpose and Environment	3502

A.9.2.4.2	Test parameters	3502

A.9.2.4.3	Test Requirements	3504

A.9.2.4A	FDD—TDD Inter frequency case	3505

A.9.2.4A.1	Test Purpose and Environment	3505

A.9.2.4A.2	Test parameters	3505

A.9.2.4A.3	Test Requirements	3507

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	3508

A.9.2.5.1	Test Purpose and Environment	3508

A.9.2.5.2	Test parameters	3508

A.9.2.5.3	Test Requirements	3511

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	3511

A.9.2.6.1	Test Purpose and Environment	3511

A.9.2.6.2	Test parameters	3512

A.9.2.6.3	Test Requirements	3515

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3515

A.9.2.7.1	Test Purpose and Environment	3515

A.9.2.7.2	Test parameters	3515

A.9.2.7.3	Test Requirements	3519

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3519

A.9.2.8.1	Test Purpose and Environment	3519

A.9.2.8.2	Test parameters	3519

A.9.2.8.3	Test Requirements	3523

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	3523

A.9.2.9.1	Test Purpose and Environment	3523

A.9.2.9.2	Test parameters	3523

A.9.2.9.3	Test Requirements	3527

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	3527

A.9.2.10.1	Test Purpose and Environment	3527

A.9.2.10.2	Test parameters	3527

A.9.2.10.3	Test Requirements	3531

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3531

A.9.2.11.1	Test Purpose and Environment	3531

A.9.2.11.2	Test parameters	3531

A.9.2.11.3	Test Requirements	3532

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3532

A.9.2.12.1	Test Purpose and Environment	3532

A.9.2.12.2	Test parameters	3533

A.9.2.12.3	Test Requirements	3533

A.9.2.13	Void	3534

A.9.2.13.1	Void	3534

A.9.2.13.2	Void	3534

A.9.2.13.3	Void	3534

A.9.2.14	Void	3534

A.9.2.14.1	Void	3534

A.9.2.14.2	Void	3534

A.9.2.14.3	Void	3534

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3534

A.9.2.15.1	Test Purpose and Environment	3534

A.9.2.15.2	Test parameters	3534

A.9.2.15.3	Test Requirements	3538

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3538

A.9.2.16.1	Test Purpose and Environment	3538

A.9.2.16.2	Test parameters	3538

A.9.2.16.3	Test Requirements	3543

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	3543

A.9.2.17.1	Test Purpose and Environment	3543

A.9.2.17.2	Test parameters	3543

A.9.2.17.3	Test Requirements	3545

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	3545

A.9.2.18.1	Test Purpose and Environment	3545

A.9.2.18.2	Test parameters	3545

A.9.2.18.3	Test Requirements	3548

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	3548

A.9.2.19.1	Test Purpose and Environment	3548

A.9.2.19.2	Test parameters	3548

A.9.2.19.3	Test Requirements	3550

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	3550

A.9.2.20.1	Test Purpose and Environment	3550

A.9.2.20.2	Test parameters	3550

A.9.2.20.3	Test Requirements	3553

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3553

A.9.2.21.1	Test Purpose and Environment	3553

A.9.2.21.2	Test parameters	3553

A.9.2.21.3	Test Requirements	3556

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3556

A.9.2.22.1	Test Purpose and Environment	3557

A.9.2.22.2	Test parameters	3557

A.9.2.22.3	Test Requirements	3557

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3557

A.9.2.23.1	Test Purpose and Environment	3557

A.9.2.23.2	Test parameters	3557

A.9.2.23.3	Test Requirements	3560

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3560

A.9.2.24.1	Test Purpose and Environment	3560

A.9.2.24.2	Test parameters	3560

A.9.2.24.3	Test Requirements	3561

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3561

A.9.2.25.1	Test Purpose and Environment	3561

A.9.2.25.2	Test parameters	3561

A.9.2.25.3	Test Requirements	3566

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3566

A.9.2.26.1	Test Purpose and Environment	3566

A.9.2.26.2	Test parameters	3566

A.9.2.26.3	Test Requirements	3570

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	3570

A.9.2.27.1	Test Purpose and Environment	3570

A.9.2.27.2	Test parameters	3570

A.9.2.27.3	Test Requirements	3571

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3571

A.9.2.28.1	Test Purpose and Environment	3571

A.9.2.28.2	Test parameters	3571

A.9.2.28.3	Test Requirements	3574

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3574

A.9.2.29.1	Test Purpose and Environment	3574

A.9.2.29.2	Test parameters	3574

A.9.2.29.3	Test Requirements	3577

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3577

A.9.2.30.1	Test Purpose and Environment	3577

A.9.2.30.2	Test parameters	3577

A.9.2.30.3	Test Requirements	3580

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3580

A.9.2.31.1	Test Purpose and Environment	3580

A.9.2.31.2	Test parameters	3580

A.9.2.31.3	Test Requirements	3583

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3583

A.9.2.32.1	Test Purpose and Environment	3583

A.9.2.32.2	Test parameters	3583

A.9.2.32.3	Test Requirements	3586

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3586

A.9.2.33.1	Test Purpose and Environment	3586

A.9.2.33.2	Test parameters	3586

A.9.2.33.3	Test Requirements	3590

A.9.2.34	FDD—FDD Inter frequency new RSRQ	3590

A.9.2.34.1	Test Purpose and Environment	3590

A.9.2.34.2	Test parameters	3590

A.9.2.34.3	Test Requirements	3593

A.9.2.35	TDD—TDD Inter frequency new RSRQ	3593

A.9.2.35.1	Test Purpose and Environment	3593

A.9.2.35.2	Test parameters	3593

A.9.2.35.3	Test Requirements	3596

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	3596

A.9.2.36.1	Test Purpose and Environment	3596

A.9.2.36.2	Test parameters	3596

A.9.2.36.3	Test Requirements	3597

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	3598

A.9.2.37.1	Test Purpose and Environment	3598

A.9.2.37.2	Test parameters	3598

A.9.2.37.3	Test Requirements	3599

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3600

A.9.2.38.1	Test Purpose and Environment	3600

A.9.2.38.2	Test parameters	3600

A.9.2.38.3	Test Requirements	3604

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3604

A.9.2.39.1	Test Purpose and Environment	3604

A.9.2.39.2	Test parameters	3604

A.9.2.39.3	Test Requirements	3608

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3608

A.9.2.40.1	Test Purpose and Environment	3608

A.9.2.40.2	Test parameters	3608

A.9.2.40.3	Test Requirements	3611

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3612

A.9.2.41.1	Test Purpose and Environment	3612

A.9.2.41.2	Test parameters	3612

A.9.2.41.3	Test Requirements	3615

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	3615

A.9.2.42.1	Test Purpose and Environment	3615

A.9.2.42.2	Test parameters	3615

A.9.2.42.3	Test Requirements	3618

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	3618

A.9.2.43.1	Test Purpose and Environment	3618

A.9.2.43.2	Test parameters	3618

A.9.2.43.3	Test Requirements	3621

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	3621

A.9.2.44.1	Test Purpose and Environment	3621

A.9.2.44.2	Test parameters	3621

A.9.2.44.3	Test Requirements	3623

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	3623

A.9.2.45.1	Test Purpose and Environment	3623

A.9.2.45.2	Test parameters	3623

A.9.2.45.3	Test Requirements	3627

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	3628

A.9.2.46.1	Test Purpose and Environment	3628

A.9.2.46.2	Test parameters	3628

A.9.2.46.3	Test Requirements	3632

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3633

A.9.2.47.1	Test Purpose and Environment	3633

A.9.2.47.2	Test parameters	3633

A.9.2.47.3	Test Requirements	3637

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3637

A.9.2.48.1	Test Purpose and Environment	3637

A.9.2.48.2	Test parameters	3638

A.9.2.48.3	Test Requirements	3642

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3643

A.9.2.49.1	Test Purpose and Environment	3643

A.9.2.49.2	Test parameters	3643

A.9.2.49.3	Test Requirements	3649

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3650

A.9.2.50.1	Test Purpose and Environment	3650

A.9.2.50.2	Test parameters	3650

A.9.2.50.3	Test Requirements	3656

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	3656

A.9.2.51.1	Test Purpose and Environment	3656

A.9.2.51.2	Test parameters	3657

A.9.2.51.3	Test Requirements	3661

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	3661

A.9.2.52.1	Test Purpose and Environment	3661

A.9.2.52.2	Test parameters	3661

A.9.2.52.3	Test Requirements	3664

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3665

A.9.2.53.1	Test Purpose and Environment	3665

A.9.2.53.2	Test parameters	3665

A.9.2.53.3	Test Requirements	3671

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3672

A.9.2.54.1	Test Purpose and Environment	3672

A.9.2.54.2	Test parameters	3672

A.9.2.54.3	Test Requirements	3678

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3678

A.9.2.55.1	Test Purpose and Environment	3678

A.9.2.55.2	Test parameters	3679

A.9.2.55.3	Test Requirements	3684

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3684

A.9.2.56.1	Test Purpose and Environment	3684

A.9.2.56.2	Test parameters	3684

A.9.2.56.3	Test Requirements	3689

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3689

A.9.2.57.1	Test Purpose and Environment	3689

A.9.2.57.2	Test parameters	3690

A.9.2.57.3	Test Requirements	3694

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3695

A.9.2.58.1	Test Purpose and Environment	3695

A.9.2.58.2	Test parameters	3695

A.9.2.58.3	Test Requirements	3699

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3700

A.9.2.59.1	Test Purpose and Environment	3700

A.9.2.59.2	Test parameters	3700

A.9.2.59.3	Test Requirements	3705

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	3706

A.9.2.60.1	Test Purpose and Environment	3706

A.9.2.60.2	Test parameters	3706

A.9.2.60.3	Test Requirements	3709

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	3709

A.9.2.61.1	Test Purpose and Environment	3709

A.9.2.61.2	Test parameters	3709

A.9.2.61.3	Test Requirements	3712

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	3712

A.9.2.62.1	Test Purpose and Environment	3712

A.9.2.62.2	Test parameters	3712

A.9.2.62.3	Test Requirements	3715

A.9.3	UTRAN FDD CPICH RSCP	3715

A.9.3.1	E-UTRAN FDD	3715

A.9.3.1.1	Test Purpose and Environment	3715

A.9.3.1.2	Parameters	3715

A.9.3.1.3	Test Requirements	3718

A.9.3.2 E-UTRAN TDD	3718

A.9.3.2.1	Test Purpose and Environment	3718

A.9.3.2.2	Parameters	3718

A.9.3.2.3	Test Requirements	3721

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	3721

A.9.3.3.1	Test Purpose and Environment	3721

A.9.3.3.2	Parameters	3721

A.9.3.3.3	Test Requirements	3722

A.9.4	UTRAN FDD CPICH Ec/No	3723

A.9.4.1	E-UTRAN FDD	3723

A.9.4.1.1	Test Purpose and Environment	3723

A.9.4.1.2	Parameters	3723

A.9.4.1.3	Test Requirements	3725

A.9.4.2	E-UTRAN TDD	3726

A.9.4.2.1	Test Purpose and Environment	3726

A.9.4.2.2	Parameters	3726

A.9.4.2.3	Test Requirements	3728

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	3729

A.9.4.3.1	Test Purpose and Environment	3729

A.9.4.3.2	Parameters	3729

A.9.4.3.3	Test Requirements	3730

A.9.5	UTRAN TDD measurement	3730

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	3730

A.9.5.1.1	Test Purpose and Environment	3730

A.9.5.1.2	Test parameters	3730

A.9.5.1.3	Test Requirements	3732

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	3733

A.9.5.2.1	Test Purpose and Environment	3733

A.9.5.2.2	Test parameters	3733

A.9.5.2.3	Test Requirements	3734

A.9.6	GSM Carrier RSSI	3735

A.9.6.1	E-UTRAN FDD	3735

A.9.6.1.1	Test Purpose and Environment	3735

A.9.6.1.2	Test Requirements	3736

A.9.6.2	E-UTRAN TDD	3736

A.9.6.2.1	Test Purpose and Environment	3736

A.9.6.2.2	Test Requirements	3738

A.9.7	UE Rx – Tx Time Difference	3739

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	3739

A.9.7.1.1	Test Purpose and Environment	3739

A.9.7.1.2	Test parameters	3739

A.9.7.1.3	Test Requirements	3740

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	3740

A.9.7.2.1	Test Purpose and Environment	3740

A.9.7.2.2	Test parameters	3740

A.9.7.2.3	Test Requirements	3742

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3742

A.9.7.3.1	Test Purpose and Environment	3742

A.9.7.3.2	Test parameters	3742

A.9.7.3.3	Test Requirements	3745

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3745

A.9.7.4.1	Test Purpose and Environment	3745

A.9.7.4.2	Test Parameters	3745

A.9.7.4.3	Test Requirements	3748

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3748

A.9.7.5.1	Test Purpose and Environment	3748

A.9.7.5.2	Test parameters	3748

A.9.7.5.3	Test Requirements	3751

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3751

A.9.7.6.1	Test Purpose and Environment	3751

A.9.7.6.2	Test Parameters	3751

A.9.7.6.3	Test Requirements	3754

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3754

A.9.7.7.1	Test Purpose and Environment	3754

A.9.7.7.2	Test parameters	3754

A.9.7.7.3	Test Requirements	3755

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3756

A.9.7.8.1	Test Purpose and Environment	3756

A.9.7.8.2	Test parameters	3756

A.9.7.8.3	Test Requirements	3757

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3757

A.9.7.9.1	Test Purpose and Environment	3757

A.9.7.9.2	Test parameters	3757

A.9.7.9.3	Test Requirements	3758

A.9.8	RSTD	3759

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	3759

A.9.8.1.1	Test Purpose and Environment	3759

A.9.8.1.2	Test Requirements	3761

A.9.8.1.2A	Test Requirements for UE Category 1bis	3761

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	3762

A.9.8.2.1	Test Purpose and Environment	3762

A.9.8.2.2	Test Requirements	3765

A.9.8.2.2A	Test Requirements for UE Category 1bis	3765

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	3766

A.9.8.3.1	Test Purpose and Environment	3766

A.9.8.3.2	Test Requirements	3768

A.9.8.3.2A	Test Requirements for UE Category 1bis	3768

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	3769

A.9.8.4.1	Test Purpose and Environment	3769

A.9.8.4.2	Test Requirements	3771

A.9.8.4.2A	Test Requirements for UE Category 1bis	3771

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	3772

A.9.8.5.1	Test Purpose and Environment	3772

A.9.8.5.2	Test Requirements	3774

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	3774

A.9.8.6.1	Test Purpose and Environment	3774

A.9.8.6.2	Test Requirements	3777

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3777

A.9.8.7.1	Test Purpose and Environment	3777

A.9.8.7.2	Test Requirements	3778

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3778

A.9.8.8.1	Test Purpose and Environment	3778

A.9.8.8.2	Test Requirements	3779

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3779

A.9.8.9.1	Test Purpose and Environment	3779

A.9.8.9.2	Test Requirements	3780

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3780

A.9.8.10.1	Test Purpose and Environment	3780

A.9.8.10.2	Test Requirements	3781

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	3781

A.9.8.11.1	Test Purpose and Environment	3781

A.9.8.11.2	Test Requirements	3782

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	3782

A.9.8.12.1	Test Purpose and Environment	3782

A.9.8.12.2	Test Requirements	3783

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	3783

A.9.8.13.1	Test Purpose and Environment	3783

A.9.8.13.2	Test Requirements	3784

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3784

A.9.8.14.1	Test Purpose and Environment	3784

A.9.8.14.2	Test Requirements	3791

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3791

A.9.8.15.1	Test Purpose and Environment	3791

A.9.8.15.2	Test Requirements	3797

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3797

A.9.8.16.1	Test Purpose and Environment	3797

A.9.8.16.2	Test Requirements	3800

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3800

A.9.8.17.1	Test Purpose and Environment	3800

A.9.8.17.2	Test Requirements	3803

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3803

A.9.8.18.1	Test Purpose and Environment	3803

A.9.8.18.2	Test Requirements	3806

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3806

A.9.8.19.1	Test Purpose and Environment	3806

A.9.8.19.2	Test Requirements	3809

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	3809

A.9.8.20.1	Test Purpose and Environment	3809

A.9.8.20.2	Test Requirements	3812

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	3813

A.9.8.21.1	Test Purpose and Environment	3813

A.9.8.21.2	Test Requirements	3816

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	3817

A.9.8.22.1	Test Purpose and Environment	3817

A.9.8.22.2	Test Requirements	3820

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	3821

A.9.8.23.1	Test Purpose and Environment	3821

A.9.8.23.2	Test Requirements	3824

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	3824

A.9.8.24.1	Test Purpose and Environment	3824

A.9.8.24.2	Test Requirements	3828

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	3829

A.9.8.25.1	Test Purpose and Environment	3829

A.9.8.25.2	Test Requirements	3832

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3833

A.9.8.26.1	Test Purpose and Environment	3833

A.9.8.26.2	Test Requirements	3835

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3835

A.9.8.27.1	Test Purpose and Environment	3835

A.9.8.27.2	Test Requirements	3838

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	3838

A.9.8.28.1	Test Purpose and Environment	3838

A.9.8.28.2	Test Requirements	3841

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3841

A.9.8.29.1	Test Purpose and Environment	3841

A.9.8.29.2	Test Requirements	3845

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3846

A.9.8.30.1	Test Purpose and Environment	3846

A.9.8.30.2	Test Requirements	3849

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	3850

A.9.8.31.1	Test Purpose and Environment	3850

A.9.8.31.2	Test Requirements	3854

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3854

A.9.8.32.1	Test Purpose and Environment	3854

A.9.8.32.2	Test Requirements	3857

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3857

A.9.8.33.1	Test Purpose and Environment	3857

A.9.8.33.2	Test Requirements	3860

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3860

A.9.8.34.1	Test Purpose and Environment	3860

A.9.8.34.2	Test Requirements	3863

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3863

A.9.8.35.1	Test Purpose and Environment	3863

A.9.8.35.2	Test Requirements	3866

A.9.9	RSRP and RSRQ on the serving cell	3866

A.9.9.1	FDD Intra frequency serving cell case	3866

A.9.9.1.1	Test Purpose and Environment	3866

A.9.9.1.2	Test parameters	3866

A.9.9.1.3	Test Requirements	3869

A.9.9.2	TDD Intra frequency serving cell case	3869

A.9.9.2.1	Test Purpose and Environment	3869

A.9.9.2.2	Test parameters	3869

A.9.9.2.3	Test Requirements	3871

A.9.10	SSTD	3871

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	3871

A.9.10.1.1	Test Purpose and Environment	3871

A.9.10.1.2	Test parameters	3871

A.9.10.1.3	Test Requirements	3873

A.9.10.2	Void	3873

A.9.10.3	Void	3873

A.9.10.4	Void	3873

A.9.11 RSSI	3873

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	3873

A.9.11.1.1	Test Purpose and Environment	3873

A.9.11.1.2	Test parameters	3873

A.9.11.1.3	Test Requirements	3876

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	3876

A.9.11.2.1	Test Purpose and Environment	3876

A.9.11.2.2	Test parameters	3876

A.9.12	Channel occupancy	3879

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	3879

A.9.12.1.1	Test Purpose and Environment	3879

A.9.12.1.2	Test parameters	3879

A.9.12.1.3	Test Requirements	3882

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	3882

A.9.12.2.1	Test Purpose and Environment	3882

A.9.12.2.2	Test parameters	3882

A.9.12.2.3	Test Requirements	3885

A.9.13	RS-SINR	3885

A.9.13.1	FDD Intra-Frequency Case	3885

A.9.13.1.1	Test Purpose and Environment	3885

A.9.13.1.2	Test parameters	3885

A.9.13.1.3	Test Requirements	3888

A.9.13.2	TDD Intra-Frequency Case	3888

A.9.13.2.1	Test Purpose and Environment	3888

A.9.13.2.2	Test parameters	3888

A.9.13.2.3	Test Requirements	3891

A.9.13.3	FDD—FDD Inter frequency case	3891

A.9.13.3.1	Test Purpose and Environment	3891

A.9.13.3.2	Test parameters	3891

A.9.13.3.3	Test Requirements	3894

A.9.13.4	TDD—TDD Inter frequency case	3894

A.9.13.4.1	Test Purpose and Environment	3894

A.9.13.4.2	Test parameters	3894

A.9.13.4.3	Test Requirements	3898

A.9.13.5	FDD—TDD Inter frequency case	3899

A.9.13.5.1	Test Purpose and Environment	3899

A.9.13.5.2	Test parameters	3899

A.9.13.5.3	Test Requirements	3905

A.9.13.6	TDD—FDD Inter frequency case	3905

A.9.13.6.1	Test Purpose and Environment	3905

A.9.13.6.2	Test parameters	3905

A.9.13.6.3	Test Requirements	3909

A.9.14	Channel quality reporting accuracy	3909

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	3909

A.9.14.1.1	Test Purpose and Environment	3909

A.9.14.1.2	Test parameters	3910

A.9.14.1.3	Test Requirements	3910

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	3911

A.9.14.2.1	Test Purpose and Environment	3911

A.9.14.2.2	Test parameters	3911

A.9.14.2.3	Test Requirements	3912

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	3912

A.9.14.3.1	Test Purpose and Environment	3912

A.9.14.3.2	Test parameters	3912

A.9.14.3.3	Test Requirements	3913

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	3913

A.9.14.4.1	Test Purpose and Environment	3913

A.9.14.4.2	Test parameters	3913

A.9.14.4.3	Test Requirements	3914

A.10	Proximity-based Services in Any Cell Selection State	3937

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	3937

A.10.1.1	Test Purpose and Environment	3937

A.10.1.2	Test Requirements	3938

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3939

A.10.2.1	Test Purpose and Environment	3939

A.10.2.2	Test Requirements	3940

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	3940

A.10.3.1	Test Purpose and Environment	3940

A.10.3.2	Test Requirements	3942

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	3943

A.10.4.1	Test Purpose and Environment	3943

A.10.4.2	Test Requirements	3945

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	3945

A.11.1	V2V UE Transmission Timing Accuracy Test	3945

A.11.1.1	Test Purpose and Environment	3945

A.11.1.2	Test requirements	3946

A.11.2	Interruptions due to V2V sidelink communication	3946

A.11.2.1	Test Purpose and Environment	3946

A.11.2.2	Test Requirements	3948

A.12	3948

A.12.1	V2X UE Transmission Timing Accuracy Test	3948

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3948

A.12.1.1.1	Test Purpose and Environment	3948

A.12.1.1.2	Test requirements	3949

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3950

A.12.1.2.1	Test Purpose and Environment	3950

A.12.1.2.2	Test Requirements	3950

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3951

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3951

A.12.2.1.1	Test Purpose and Environment	3951

A.12.2.1.2	Test Requirements	3952

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3953

A.12.2.2.1	Test Purpose and Environment	3953

A.12.2.2.2	Test Requirements	3954

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3954

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3954

A.12.3.1.1	Test Purpose and Environment	3954

A.12.3.1.2	Test Requirements	3957

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3958

A.12.3.2.1	Test Purpose and Environment	3958

A.12.3.1.2	Test Requirements	3960

A.12.4	Congestion Control Measurement Test for V2X UE	3960

A.12.4.1	Test Purpose and Environment	3960

A.12.4.2	Test Requirements	3962

A.12.5	Interruptions due to V2X Sidelink Communication	3962

A.12.5.1	Test Purpose and Environment	3962

A.12.5.2	Test Requirements	3964

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3964

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3964

A.12.6.1.1	Test Purpose and Environment	3964

A.12.6.1.2	Test Requirements	3966

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3966

A.12.6.2.1	Test Purpose and Environment	3966

A.12.6.1.2	Test Requirements	3968

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3968

A.12.7.1	Test Purpose and Environment	3968

A.12.7.2	Test Requirements	3971

A.12.8	Interruptions due to V2X Carrier Aggregation	3972

A.12.8.1	Interruptions on a FDD PCell	3972

A.12.8.1.1	Test Purpose and Environment	3972

A.12.8.1.2	Test Requirements	3973

A.12.8.2	Interruptions on a TDD PCell	3973

A.12.8.2.1	Test Purpose and Environment	3973

A.12.8.2.2	Test Requirements	3975

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3976

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3976

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3976

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3976

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3976

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3978

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3979

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3979

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3979

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3979

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3981

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3982

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3982

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3982

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3983

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3983

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3984

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3984

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3984

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3985

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3985

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3986

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3986

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3986

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3986

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3987

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3987

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3987

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3988

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3988

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3988

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3989

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3989

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3990

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3991

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3992

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3993

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3994

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3994

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3994

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3994

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3995

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3995

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3997

B.2.24	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	3997

B.2.25	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	3997

B.3	Conditions for measurements performance requirements for UE	3997

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3997

B.3.2	Void	3998

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3998

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3998

B.3.5	Conditions for UE Rx – Tx time difference	3998

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3998

B.3.7	Conditions for inter-frequency RSTD measurements	3998

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3999

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3999

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3999

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3999

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4000

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	4000

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4000

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	4000

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	4000

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4001

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	4001

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	4001

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4001

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	4001

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	4001

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4002

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	4002

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	4002

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	4002

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	4002

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	4002

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4002

B.3.21.1	Conditions for RSRP measurements	4002

B.3.21.2	Conditions for RSRQ measurements	4003

B.3.21.3	Conditions for CSI-RSRP measurements	4003

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4003

B.3.22.1	Conditions for RSRP measurements	4003

B.3.22.2	Void	4003

B.3.22.3	Conditions for CSI-RSRP measurements	4003

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4004

B.3.23.1	Conditions for RSRP measurements	4004

B.3.23.2	Conditions for RSRQ measurements	4004

B.3.23.3	Conditions for CSI-RSRP measurements	4004

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4004

B.3.24.1	Conditions for RSRP measurements	4004

B.3.24.2	Conditions for RSRQ measurements	4004

B.3.24.3	Conditions for CSI-RSRP measurements	4004

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4005

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4005

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	4005

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	4005

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4006

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4006

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4006

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4006

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4006

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4006

B.4	RRM Requirements Exceptions	4007

B.4.1	General	4007

B.4.2	Receiver sensitivity relaxation for UE supporting CA	4007

B.4.3	Receiver sensitivity relaxation for UE configured with CA	4007

B.4.3.1	Inter-band carrier aggregation	4007

B.4.3.2	Intra-band non-contiguous carrier aggregation	4007

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	4007

B.5	Conditions for Measurement Performance Requirements for ProSe UE	4008

B.5.1	Conditions for S-RSRP Accuracy Requirements	4008

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	4008

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4008

B.5.4	Conditions for SD-RSRP Accuracy Requirements	4009

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	4009

B.6	Conditions for V2X	4010

B.6.1	Test parameters for GNSS signals	4010

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	4010

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	4010

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4010

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	4011

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	4011

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	4011

Annex C (informative): Change history:	4013