3GPP TS 29.583 V18.0.0 (2024-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Application layer support for Personal IoT Network (PINAPP);

Personal IoT Network (PIN) server services;

Stage 3

(Release 18)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Intpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	10

1	Scope	11

2	References	11

3	Definitions and abbreviations	11

3.1	Definitions	11

3.2	Abbreviations	12

4	Overview	12

5	Services offered by PIN Server	13

5.1	Introduction	13

5.2	PIN_ASRegistration Service	13

5.2.1	Service Description	13

5.2.2	Service Operations	13

5.2.2.1	Introduction	13

5.2.2.2	PIN_ASRegistration_Request	14

5.2.2.2.1	General	14

5.2.2.2.2	PAS Registration Creation	14

5.2.2.3	PIN_ASRegistration_Update	14

5.2.2.3.1	General	14

5.2.2.3.2	Updating an existing Registration	15

5.2.2.4	PIN_ASRegistration_Deregister	15

5.2.2.4.1	General	15

5.2.2.4.2	Deregistering the Registration	15

5.3	PIN_ASServiceSwitch Service	16

5.3.1	Service Description	16

5.3.2	Service Operations	16

5.3.2.1	Introduction	16

5.3.2.2	PIN_ASServiceSwitch_Subscribe	16

5.3.2.2.1	General	16

5.3.2.2.2	Service Switch Information Subscription Creation	16

5.3.2.3	PIN_ASServiceSwitch_Notify	17

5.3.2.3.1	General	17

5.3.2.3.2	Service Switch Information Notification	17

5.3.2.4	PIN_ASServiceSwitch_Update	17

5.3.2.4.1	General	17

5.3.2.4.2	Service Switch Information Update	17

5.3.2.5	PIN_ASServiceSwitch_Unsubscribe	18

5.3.2.5.1	General	18

5.3.2.5.2	Unsubscribing the Service Switch Information	18

5.4	PIN_ASServiceContinuity Service	18

5.4.1	Service Description	18

5.4.2	Service Operations	19

5.4.2.1	Introduction	19

5.4.2.2	PIN_ASServiceContinuity_Subscribe	19

5.4.2.2.1	General	19

5.4.2.2.2	Service Continuity Information Subscription Creation	19

5.4.2.3	PIN_ASServiceContinuity_Notify	19

5.4.2.3.1	General	19

5.4.2.3.2	Service Continuity Information Notification	20

5.4.2.4	PIN_ASServiceContinuity_Update	20

5.4.2.4.1	General	20

5.5.2.4.2	Service Continuity Information Update	20

5.4.2.5	PIN_ASServiceContinuity_Unsubscribe	21

5.4.2.5.1	General	21

5.4.2.5.2	Unsubscribing the Service Continuity Information	21

6	API Definitions	21

6.1	PIN_ASRegistration Service API	21

6.1.1	Introduction	21

6.1.2	Usage of HTTP	22

6.1.3	Resources	22

6.1.3.1	Overview	22

6.1.3.2	Resource: PAS Registrations	22

6.1.3.2.1	Description	22

6.1.3.2.2	Resource Definition	23

6.1.3.2.3	Resource Standard Methods	23

6.1.3.2.4	Resource Custom Operations	24

6.1.3.3	Resource: Individual PAS Registration	24

6.1.3.3.1	Description	24

6.1.3.3.2	Resource Definition	24

6.1.3.3.3	Resource Standard Methods	24

6.1.3.3.3.1	GET	24

6.1.3.3.3.2	PUT	25

6.1.3.3.3.3	DELETE	26

6.1.3.3.3.4	PATCH	27

6.1.3.3.4	Resource Custom Operations	28

6.1.4	Custom Operations without associated resources	28

6.1.5	Notifications	28

6.1.6	Data Model	28

6.1.6.1	General	28

6.1.6.2	Structured data types	29

6.1.6.2.1	Introduction	29

6.1.6.2.2	Type: PASRegistration	30

6.1.6.2.3	Type: ConnectivityInfo	30

6.1.6.2.4	Type: PASRegistrationPatch	30

6.1.6.3	Simple data types and enumerations	31

6.1.6.3.1	Introduction	31

6.1.6.3.2	Simple data types	31

6.1.7	Error Handling	31

6.1.7.1	General	31

6.1.7.2	Protocol Errors	31

6.1.7.3	Application Errors	31

6.1.8	Feature negotiation	31

6.1.9	Security	31

6.2	PIN_ASServiceSwitch API	32

6.2.1	Introduction	32

6.2.2	Usage of HTTP	32

6.2.3	Resources	32

6.2.3.1	Overview	32

6.2.3.2	Resource: Service Switch Information Subscriptions	33

6.2.3.2.1	Description	33

6.2.3.2.2	Resource Definition	33

6.2.3.2.3	Resource Standard Methods	33

6.2.3.2.3.1	POST	33

6.2.3.2.4	Resource Custom Operations	34

6.2.3.3	Resource: Individual Service Switch Information Subscription	34

6.2.3.3.1	Description	34

6.2.3.3.2	Resource Definition	34

6.2.3.3.3	Resource Standard Methods	34

6.2.3.3.3.1	GET	34

6.2.3.3.3.2	PATCH	35

6.2.3.3.3.3	PUT	36

6.2.3.3.3.4	DELETE	37

6.2.3.3.3	Resource Custom Operations	38

6.2.4	Custom Operations without associated resources	38

6.2.5	Notifications	39

6.2.5.1	General	39

6.2.5.1	Service Switch Information Notification	39

6.2.5.1.1	Description	39

6.2.5.1.2	Target URI	39

6.2.5.1.3	Standard Methods	39

6.2.5.1.3.1	POST	39

6.2.6	Data Model	40

6.2.6.1	General	40

6.2.6.2	Structured data types	41

6.2.6.2.1	Introduction	41

6.2.6.2.2	Type: ServiceSwitchInfo	41

6.2.6.2.3	Type: ServiceSwitchInfoPatch	42

6.2.6.2.4	Type: ServiceSwitchInfoNotification	42

6.2.6.2.5	Type: ServiceSwitchReportInfo	42

6.2.6.3	Simple data types and enumerations	42

6.2.6.3.1	Introduction	42

6.2.6.3.2	Enumeration: EventType	43

6.2.7	Error Handling	43

6.2.7.1	General	43

6.2.7.2	Protocol Errors	43

6.2.7.3	Application Errors	43

6.2.8	Feature negotiation	43

6.2.9	Security	43

6.3	PIN_ASServiceContinuity API	43

6.3.1	Introduction	43

6.3.2	Usage of HTTP	44

6.3.3	Resources	44

6.3.3.1	Overview	44

6.3.3.2	Resource: Service Continuity Information Subscriptions	45

6.3.3.2.1	Description	45

6.3.3.2.2	Resource Definition	45

6.3.3.2.3	Resource Standard Methods	45

6.3.3.2.3.1	POST	45

6.3.3.2.4	Resource Custom Operations	46

6.3.3.3	Resource: Individual Service Continuity Information Subscription	46

6.3.3.3.1	Description	46

6.3.3.3.2	Resource Definition	46

6.3.3.3.3	Resource Standard Methods	46

6.3.3.3.3.1	GET	46

6.3.3.3.3.2	PATCH	47

6.3.3.3.3.3	PUT	48

6.3.3.3.3.4	DELETE	49

6.3.3.3.3	Resource Custom Operations	50

6.3.4	Custom Operations without associated resources	50

6.3.5	Notifications	51

6.3.5.1	General	51

6.3.5.1	Service Continuity Information Notification	51

6.3.5.1.1	Description	51

6.3.5.1.2	Target URI	51

6.3.5.1.3	Standard Methods	51

6.3.5.1.3.1	POST	51

6.3.6	Data Model	52

6.3.6.1	General	52

6.3.6.2	Structured data types	53

6.3.6.2.1	Introduction	53

6.3.6.2.2	Type: ServiceContinuityInfo	53

6.3.6.2.3	Type: ServiceContinuityInfoPatch	54

6.3.6.2.4	Type: ServiceContinuityInfoNotification	54

6.3.6.2.5	Type: ServiceContinuityReportInfo	54

6.3.6.3	Simple data types and enumerations	55

6.3.6.3.1	Introduction	55

6.3.6.3.2	Enumeration: EventType	55

6.3.7	Error Handling	55

6.3.7.1	General	55

6.3.7.2	Protocol Errors	55

6.3.7.3	Application Errors	55

6.3.8	Feature negotiation	55

6.3.9	Security	55

7	Using Common API Framework	56

7.1	General	56

7.2	Security	56

Annex A (normative): OpenAPI specification	57

A.1	General	57

A.2	PIN_ASRegistration API	57

A.3	PIN_ASServiceSwitch API	61

A.4	PIN_ASServiceContinuity API	67

Annex B (informative): Change history	73

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the stage 3 protocol and data model for PIN-9 interface between the PIN application server and PIN server. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the PIN server over PIN-9 interface. The stage 2 functional requirements are defined in 3GPP TS 23.542 [10].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[3]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[4]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[5]	3GPP TR 21.900: "Technical Specification Group working methods".

[6]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[8]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[11]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[12]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Personal IoT Network (PIN): A configured and managed group of PIN Element(s) that are able to communicate with each other directly, communicate with each other via PIN Element(s) with Gateway Capability (i.e. PEGC(s)), or use a PEGC to communicate with devices or servers that are outside of the PIN via the 5G network. A PIN includes at least one PEGC and is managed by PIN Element(s) with Management Capability (i.e. PEMC(s)) with the support by an AF if AF is deployed.

PIN Element with Gateway Capability (PEGC): A PIN Element with the ability to provide DN connectivity via the 5G network for other PIN Elements and/or is able to provide relay functionality for communication between PIN Elements. Only a UE is able to act as a PEGC.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

PAS	PIN Application Server

PEGC	PIN Element with Gateway Capability

PIN	Personal IoT Network

PINAPP	Personal IoT Network Application

# 4	Overview

The Personal IoT Network (PIN) Server forms part of the Application layer support for Personal IoT Networks defined in 3GPP TS 23.542 [10]. It is aimed to support the server-side functionalities required for managing the PIN. The PIN Server provides the following functionalities:

-	support application server registration management procedure;

-	support application server service switch subscription procedure; and

-	support application server service continuity subscription procedure.

Figure 4-1 shows the reference point representation of the architecture for Personal IoT Network Application.

Figure 4-1: PINAPP architecture

PIN-9 reference point exists between the application server and PIN server for the interactions related to enabling PINAPP.

# 5	Services offered by PIN Server

## 5.1	Introduction

Table 5.1-1: List of PIN server APIs

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| PIN_ASRegistration | Request | Request/Response | e.g. PAS |
|  | Update | Request/Response | e.g. PAS |
|  | Deregister | Request/Response | e.g. PAS |
| PIN_ASServiceSwitch | Subscribe | Request/Response | e.g. PAS |
|  | Notify | Request | e.g. PAS |
|  | Update | Request/Response | e.g. PAS |
|  | Delete | Request/Response | e.g. PAS |
| PIN_ASServiceContinuity | Subscribe | Request/Response | e.g. PAS |
|  | Notify | Request | e.g. PAS |
|  | Update | Request/Response | e.g. PAS |
|  | Delete | Request/Response | e.g. PAS |

Table 5.1-2 summarizes the corresponding PIN server APIs defined in this specification.

Table 5.1-2: PIN server API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | API Name | Annex |
| --- | --- | --- | --- | --- | --- |
| PIN_ASRegistration | 5.2 | Service for AS registration | TS29583_PIN_ASRegistration.yaml | pin-as-registration | A.2 |
| PIN_ASServiceSwitch | 5.3 | Service for reporting service switch | TS29583_PIN_ASServiceSwitch.yaml | pin-as-serviceswitch | A.3 |
| PIN_ASServiceContinuity | 5.4 | Service for reporting service continuity | TS29583_PIN_ASServiceContinuity.yaml | pin-as-servicecontinuity | A.4 |

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (i.e. PIN Server) takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

## 5.2	PIN_ASRegistration Service

### 5.2.1	Service Description

The PIN_ASRegistration API, as defined in 3GPP TS 23.542 [10], allows a PAS to register, update its registration and deregister at a given PIN server via PIN-9 interface.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operation defined for PIN_ASRegistration API is shown in the table 5.2.2.1-1.

Table 5.2.2.1-1: Operations of the PIN_ASRegistration API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| PIN_ASRegistration_Request | This service operation is used by the service consumer to register to the PIN server. | e.g. PAS |
| PIN_ASRegistration_Update | This service operation is used by the service consumer to update the registration information to the PIN server. | e.g. PAS |
| PIN_ASRegistration_Deregister | This service operation is used by the service consumer to deregister from the PIN server. | e.g. PAS |

#### 5.2.2.2	PIN_ASRegistration_Request

##### 5.2.2.2.1	General

This service operation is used by the PAS to register itself to a PIN server.

##### 5.2.2.2.2	PAS Registration Creation

Figure 5.2.2.2.2-1: Procedure for PAS Registration Creation

1.	When the service consumer needs to register to the PIN server, the service consumer shall send the HTTP POST request message to the PIN Server targeting the "PAS Registrations" collection resource, with the request body including the PASRegistration data structure.

2a.	Upon success, if the PAS is authorized, the PIN Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual PAS Registration" resource within the PASRegistration data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.7.

#### 5.2.2.3	PIN_ASRegistration_Update

##### 5.2.2.3.1	General

This service operation is used by the PAS to update its registration information at a given PIN server.

##### 5.2.2.3.2	Updating an existing Registration

Figure 5.2.2.3.2-1: Procedure for update of a registration

1.	When the service consumer needs to update the registration information to the PIN server, the service consumer shall send the HTTP PUT/PATCH message to the PIN server targeting the "Individual PAS Registration" collection resource, with the request body including the PASRegistrationPatch data structure.

2a.	Upon success, if the PAS is authorized, the PIN server shall respond with an HTTP PUT response including "200 OK" status code with the response body containing the fully updated PAS registration information of the resource within the PASRegistration data structure or HTTP PATCH response including "204 No Content" status code.

2b.	On failure, the PIN server shall send the error response as specified in clause 6.1.7.

#### 5.2.2.4	PIN_ASRegistration_Deregister

##### 5.2.2.4.1	General

This service operation is used by the PAS to deregister itself from a given PIN server.

##### 5.2.2.4.2	Deregistering the Registration

Figure 5.2.2.4.2-1: Procedure for Deregistering the Registration

1.	When the service consumer needs to deregister from the PIN server, the service consumer shall send an HTTP DELETE request to the PIN server targeting the corresponding "Individual PAS Registration" resource.

2a.	Upon success, if the PAS is authorized, the PIN server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.1.7.

If the PIN server determines that the received HTTP DELETE request needs to be redirected, the PIN server may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative PIN server where the message should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2].

## 5.3	PIN_ASServiceSwitch Service

### 5.3.1	Service Description

The PIN_ASServiceSwitch API, as defined in 3GPP TS 23.542 [10], allows a PAS to obtain the information about the service switch at a given PIN server via PIN-9 interface.

### 5.3.2	Service Operations

#### 5.3.2.1	Introduction

The service operation defined for PIN_ASServiceSwitch API is shown in the table 5.3.2.1-1.

Table 5.3.2.1-1: Operations of the PIN_ASServiceSwitch API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| PIN_ASServiceSwitch_Subscribe | This service operation is used by the PAS to subscribe to PIN server, for reporting of service switch information. | PAS |
| PIN_ASServiceSwitch_Notify | This service operation is used by the PIN server to notify the PAS about the service switch information. | PIN server |
| PIN_ASServiceSwitch_Update | This service operation is used by the PAS to update its subscription at PIN server, for reporting of service switch information. | PAS |
| PIN_ASServiceSwitch_Unsubscribe | This service operation is used by the PAS to remove its subscription at PIN server, for reporting of service switch information. | PAS |

#### 5.3.2.2	PIN_ASServiceSwitch_Subscribe

##### 5.3.2.2.1	General

This service operation is used by the PAS to subscribe to PIN server, for reporting of service switch information.

##### 5.3.2.2.2	Service Switch Information Subscription Creation

Figure 5.3.2.2.2-1: Procedure for PAS Subscription Creation

1.	When the service consumer needs to subscribe to service switch information at the PIN server, the service consumer shall send the HTTP POST request message to the PIN Server targeting the "Service Switch Information Subscriptions" collection resource, with the request body including the ServiceSwitchInfo data structure.

2a.	Upon success, if the PAS is authorized, the PIN Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual Service Switch Information Subscription" resource within the ServiceSwitchInfo data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.2.7.

#### 5.3.2.3	PIN_ASServiceSwitch_Notify

##### 5.3.2.3.1	General

This service operation is used by the PIN server to notify the PAS about the service switch information.

##### 5.3.2.3.2	Service Switch Information Notification

Figure 5.3.2.3.2-1: Procedure for Service Switch Information Notification

1.	When the service consumer needs to notify the service switch information at the PIN server, the service consumer shall send the HTTP POST request message to the PIN Server targeting the "Service Switch Information Subscriptions" collection resource, with the request body including the ServiceSwitchInfoNotification data structure.

2a.	Upon success, if the PAS is authorized, the PAS shall process the Service Switch Information Notification and shall respond to the PIN server with "204 No Content" message.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.2.7.

#### 5.3.2.4	PIN_ASServiceSwitch_Update

##### 5.3.2.4.1	General

This service operation is used by the PAS to update a service switch subscription with the PIN Server.

##### 5.3.2.4.2	Service Switch Information Update

To update service switch information reporting subscription at the PIN server, the PAS shall send a HTTP PATCH or PUT message to the PIN server on resource URI identifying the "Individual Service Switch Information Subscription" resource representation, as specified in clause 6.2.3.3.3.2 for HTTP PATCH message and in clause 6.2.3.3.3.3 for HTTP PUT message.

The HTTP PUT message shall replace all properties in the existing resource with the service switch information subscription in the request.

The HTTP PATCH message shall modify the properties in the existing resource with the service switch information subscription information in the request.

Upon receiving the HTTP PUT or HTTP PATCH message from the PAS, the PIN server shall process the request message and verify whether the PAS is authorized to modify the requested subscription resource or not:

a)	if the PAS is authorized to modify the requested subscription resource, the PIN server shall:

1)	update the resource identified by Resource URI of the PAS service switch information subscription with the updated information received in the HTTP PATCH or PUT request message;

2)	upon successful update of PAS service switch information, return the updated PAS service switch information subscription in the "200 OK" response message or respond with the "204 No Content" message indicating to the PAS that the PAS service switch information subscription is updated successfully. In the response message, the PIN server may provide an updated expiration time to indicate to the PAS when the updated subscription will automatically expire.

b)	if the PAS is not authorized to modify the requested subscription resource, the PIN server shall:

1)	take proper error handling actions, as specified in clause 6.2.7; and

2)	send an HTTP response message with an appropriate error status code.

If the PIN server determines that the received HTTP PATCH or PUT message needs to be redirected, the PIN server may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative PIN server where the message should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2].

If the expiration time is provided, then to maintain the subscription, the PAS shall send an update subscription prior to subscription expiry time and the PIN server shall reset the expiration time in the PIN server. If the update subscription request is not sent before the expiry time, then the PIN server shall treat the PAS subscription as unsubscribed and remove the corresponding PAS service switch information subscription resource.

#### 5.3.2.5	PIN_ASServiceSwitch_Unsubscribe

##### 5.3.2.5.1	General

This service operation is used by the PAS to remove its subscription at PIN server, for reporting of service switch information.

##### 5.3.2.5.2	Unsubscribing the Service Switch Information

Figure 5.3.2.5.2-1: Procedure for Unsubscribing the Service Switch Information

1.	When the service consumer needs to unsubscribe its service switch information subscription from the PIN server, the service consumer shall send the HTTP DELETE request message to the PIN Server targeting the "Individual Service Switch Information Subscription" collection resource.

2a.	Upon success, if the PAS is authorized, the PIN Server shall respond with an HTTP "204 No content" status code to indicate the successful removal of the subscription information.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.2.7.

## 5.4	PIN_ASServiceContinuity Service

### 5.4.1	Service Description

The PIN_ASServiceContinuity API, as defined in 3GPP TS 23.542 [10], allows a PAS to obtain the information about the service continuity at a given PIN server via PIN-9 interface.

### 5.4.2	Service Operations

#### 5.4.2.1	Introduction

The service operation defined for PIN_ASServiceContinuity API is shown in the table 5.4.2.1-1.

Table 5.4.2.1-1: Operations of the PIN_ASServiceContinuity API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| PIN_ASServiceContinuity_Subscribe | This service operation is used by the PAS to subscribe to PIN server, for reporting of service continuity information. | PAS |
| PIN_ASServiceContinuity_Notify | This service operation is used by the PIN server to notify the PAS about the service continuity information. | PIN server |
| PIN_ASServiceContinuity_Update | This service operation is used by the PAS to update its subscription at PIN server, for reporting of service continuity information. | PAS |
| PIN_ASServiceContinuity_Unsubscribe | This service operation is used by the PAS to remove its subscription at PIN server, for reporting of service continuity information. | PAS |

#### 5.4.2.2	PIN_ASServiceContinuity_Subscribe

##### 5.4.2.2.1	General

This service operation is used by the PAS to subscribe to PIN server, for reporting of service continuity information.

##### 5.4.2.2.2	Service Continuity Information Subscription Creation

Figure 5.4.2.2.2-1: Procedure for Continuity Information Subscription Creation

1.	When the service consumer needs to subscribe to service continuity information at the PIN server, the service consumer shall send the HTTP POST request message to the PIN Server targeting the "Service Continuity Information Subscriptions" collection resource, with the request body including the ServiceContinuityInfo data structure.

2a.	Upon success, if the PAS is authorized, the PIN Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual Service Continuity Information Subscription" resource within the ServiceContinuityInfo data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.3.7.

#### 5.4.2.3	PIN_ASServiceContinuity_Notify

##### 5.4.2.3.1	General

This service operation is used by the PIN server to notify the PAS about the service continuity information.

##### 5.4.2.3.2	Service Continuity Information Notification

Figure 5.4.2.3.2-1: Procedure for Service Continuity Information Notification

1.	When the service consumer needs to notify the service continuity information at the PIN server, the service consumer shall send the HTTP POST request message to the PIN Server targeting the "Service Continuity Information Subscriptions" collection resource, with the request body including the ServiceContinuityInfoNotification data structure.

2a.	Upon success, if the PAS is authorized, the PAS shall process the Service Continuity Information Notification and shall respond to the PIN server with "204 No Content" message.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.3.7.

#### 5.4.2.4	PIN_ASServiceContinuity_Update

##### 5.4.2.4.1	General

This service operation is used by the PAS to update a service continuity subscription with the PIN Server.

##### 5.5.2.4.2	Service Continuity Information Update

Figure 5.4.2.4.2-1: Procedure for the Service Continuity Information Update

1.	When the service consumer needs to update service continuity information reporting subscription at the PIN server, the service consumer shall send the HTTP PATCH or HTTP PUT request message to the PIN Server targeting the "Individual Service Continuity Information Subscription" collection resource, with the request body including the ServiceContinuityInfo data structure.

2a.	Upon success, if the PAS is authorized, the PIN Server shall respond with an HTTP "204 No content" or an HTTP "200 OK" status code with the response body containing a representation of the created "Individual Service Continuity Information Subscription" resource within the ServiceContinuityInfoPatch data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.3.7.

#### 5.4.2.5	PIN_ASServiceContinuity_Unsubscribe

##### 5.4.2.5.1	General

This service operation is used by the PAS to remove its subscription at PIN server, for reporting of service continuity information.

##### 5.4.2.5.2	Unsubscribing the Service Continuity Information

Figure 5.4.2.5.2-1: Procedure for Unsubscribing the Service Continuity Information

1.	When the service consumer needs to unsubscribe its service continuity information subscription from the PIN server, the service consumer shall send the HTTP DELETE request message to the PIN Server targeting the "Individual Service Continuity Information Subscription" collection resource.

2a.	Upon success, if the PAS is authorized, the PIN Server shall respond with an HTTP "204 No content" status code to indicate the successful removal of the subscription information.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.3.7.

# 6	API Definitions

## 6.1	PIN_ASRegistration Service API

### 6.1.1	Introduction

The PIN_ASRegistration service shall use the PIN_ASRegistration service API.

The API URI of the PIN_ASRegistration service API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [2], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

-	The <apiName> shall be "pin-as-registration".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (i.e. PIN Server) takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

### 6.1.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [2] shall apply for the PIN_ASRegistration API.

### 6.1.3	Resources

#### 6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the PIN_ASRegistration API.

Figure 6.1.3.1-1: Resource URI structure of the PIN_ASRegistration API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| PAS Registrations | /registrations | POST | Register a new PAS at the PIN server. |
| Individual PAS Registration | /registrations/{registrationId} | GET | Fetch an individual PAS registration resource. |
|  |  | PUT | Fully replace an individual PAS registration resource. |
|  |  | PATCH | Partially modify an individual PAS registration resource. |
|  |  | DELETE | Remove an individual PAS registration resource. |

#### 6.1.3.2	Resource: PAS Registrations

##### 6.1.3.2.1	Description

This resource represents all the PAS that are registered to the PIN server.

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/pin-as-registration/<apiVersion>/registrations

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |

##### 6.1.3.2.3	Resource Standard Methods

6.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST Request Body on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PASRegistration | M | 1 | PAS registration request information. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PASRegistration | M | 1 | 201 Created | PAS information is successfully registered to the PIN server. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 6.1.3.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/pin-as-registration/<apiVersion>/registrations/{registrationId} |

Table 6.1.3.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 6.1.3.2.4	Resource Custom Operations

None.

#### 6.1.3.3	Resource: Individual PAS Registration

##### 6.1.3.3.1	Description

This Individual PAS Registration resource represents an individual PAS registered at a given PIN server.

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/pin-as-registration/<apiVersion>/registrations/{registrationId}

This resource shall support the resource URI variables defined in the table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |
| registrationId | string | Identifies a PAS registration. |

##### 6.1.3.3.3	Resource Standard Methods

###### 6.1.3.3.3.1	GET

This method retrieves the PAS information registered at the PIN server. This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PASResgistration | M | 1 | 200 OK | The PAS registration information at the PIN server. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

Table 6.1.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

###### 6.1.3.3.3.2	PUT

This method updates the PAS registration information at the PIN server by completely replacing the existing registration data. This method shall support the URI query parameters specified in the table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PASRegistration | M | 1 | Details of the PAS registration information to be updated. |

Table 6.1.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PASRegistration | M | 1 | 200 OK | The Individual PAS registration information updated successfully and the updated PAS registration information is returned in the response. |
| n/a |  |  | 204 No Content | The Individual PAS registration information updated successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

Table 6.1.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

###### 6.1.3.3.3.3	DELETE

This method deregisters a PAS registration from the PIN server. This method shall support the URI query parameters specified in the table 6.1.3.3.3.3-1.

Table 6.1.3.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.3.3.3.3-3.

Table 6.1.3.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The individual PAS registration information matching the registrationId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

Table 6.1.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

###### 6.1.3.3.3.4	PATCH

This method partially updates the PAS registration information at the PIN server. This method shall support the URI query parameters specified in the table 6.1.3.3.3.4-1.

Table 6.1.3.3.3.4-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.4-2 and the response data structures and response codes specified in table 6.1.3.3.3.4-3.

Table 6.1.3.3.3.4-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PASRegistrationPatch | M | 1 | Details of the PAS registration information to be updated |

Table 6.1.3.3.3.4-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PASRegistration | M | 1 | 200 OK | The Individual PAS registration information was updated successfully and the updated PAS registration information is returned in the response. |
| n/a |  |  | 204 No Content | The Individual PAS registration information was updated successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [6]. |
| NOTE: The mandatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

Table 6.1.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

##### 6.1.3.3.4	Resource Custom Operations

None.

### 6.1.4	Custom Operations without associated resources

None.

### 6.1.5	Notifications

None.

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the PIN_ASRegistration API.

Table 6.1.6.1-1: PIN_ASRegistration API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| ConnectivityInfo | 6.1.6.2.3 | Contains the connectivity information used to communicate with the PAS. |  |
| PASRegistration | 6.1.6.2.2 | Represents the PAS registration information. |  |
| PASRegistrationPatch | 6.1.6.2.4 | Represents the requested modifications to the PAS registration information. |  |

Table 6.1.6.1-2 specifies data types re-used by the PIN_ASRegistration API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the PIN_ASRegistration API.

Table 6.1.6.1-2: PIN_ASRegistration API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [2] | Used to capture the expiration time of PAS registration. |  |
| DateTimeRm | 3GPP TS 29.571 [11] | Used to capture the expiration time of PAS registration patch. |  |
| Fqdn | 3GPP TS 29.571 [11] | Used to express the Fully Qualified Domain Name of PAS end point. |  |
| Ipv4Addr | 3GPP TS 29.122 [2] | Identifies the IPv4 address of the PAS. |  |
| Ipv6Addr | 3GPP TS 29.122 [2] | Identifies the IPv6 address of the PAS. |  |
| SupportedFeatures | 3GPP TS 29.571 [11] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: PASRegistration

Table 6.1.6.2.2-1: Definition of type PASRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| conInfo | ConnectivityInfo | M | 1 | Contains the connectivity information used to communicate with the PAS. |  |
| passId | string | M | 1 | Identifies the PIN service that provided by the PAS. |  |
| expTime | DateTime | O | 0..1 | Identifies the expiration time for the PAS registration. To maintain an active registration status, a registration update is required before the expiration time. If the expiration time is not present, then it indicates that the registration of PAS never expires. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.8. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 6.1.6.2.3	Type: ConnectivityInfo

Table 6.1.6.2.3-1: Definition of type ConnectivityInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| fqdn | Fqdn | O | 0..1 | Fully Qualified Domain Name of the PAS. |  |
| ipv4Addr | Ipv4Addr | O | 0..1 | IPv4 address of the PAS. |  |
| ipv6Addr | Ipv6Addr | O | 0..1 | IPv6 address of the PAS. |  |
| uri | Uri | O | 0..1 | URI information of the PAS. |  |
| NOTE: At least one of the attributes shall be provided. |  |  |  |  |  |

##### 6.1.6.2.4	Type: PASRegistrationPatch

Table 6.1.6.2.4-1: Definition of type PASRegistrationPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| passId | string | O | 0..1 | Identifies the PIN service that provided by the PAS. |  |
| conInfo | ConnectivityInfo | O | 0..1 | Contains the connectivity information used to communicate with the PAS. |  |
| expTime | DateTimeRm | O | 0..1 | Identifies the expiration time for the PAS registration. If the expiration time is not present, then it indicates that the registration of PAS never expires. |  |
| NOTE: At least one of the attributes shall be provided. |  |  |  |  |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the PIN_ASRegistration API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [2]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [2] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [2].

In addition, the requirements in the following clauses are applicable for the PIN_ASRegistration API.

#### 6.1.7.2	Protocol Errors

No specific procedures for the PIN_ASRegistration API are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the PIN_ASRegistration API are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| n/a |  |  |

### 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the PIN_ASRegistration API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [2].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| n/a |  |  |

### 6.1.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [2] shall apply for the PIN_ASRegistration API.

## 6.2	PIN_ASServiceSwitch API

### 6.2.1	Introduction

The PIN_ASServiceSwitch service shall use the PIN_ASServiceSwitch API.

The API URI of the PIN_ASServiceSwitch API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [2], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

-	The <apiName> shall be "pin-as-serviceswitch".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (i.e. PIN Server) takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

### 6.2.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [2] shall apply for the PIN_ASServiceSwitch API.

### 6.2.3	Resources

#### 6.2.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.3.1-1 depicts the resource URIs structure for the PIN_ASServiceSwitch API.

Figure 6.2.3.1-1: Resource URI structure of the PIN_ASServiceSwitch API

Table 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Service Switch Information Subscriptions | /subscriptions | POST | Create a subscription for reporting of service switch information to the PAS. |
| Individual Service Switch Information Subscription | /subscriptions/{subscriptionId} | GET | Retrieve the Individual service switch information subscription information identified by subscriptionId. |
|  |  | PUT | Fully replace the individual service switch information subscription identified by subscriptionId. |
|  |  | PATCH | Partially update the individual service switch information subscription identified by subscriptionId. |
|  |  | DELETE | Remove the individual service switch information subscription identified by subscriptionId. |

#### 6.2.3.2	Resource: Service Switch Information Subscriptions

##### 6.2.3.2.1	Description

This resource represents all service switch information subscriptions at a given PIN server.

##### 6.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/pin-as-serviceswitch/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 6.2.3.2.2-1.

Table 6.2.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1. |

##### 6.2.3.2.3	Resource Standard Methods

###### 6.2.3.2.3.1	POST

This method creates the service switch information subscription at the PIN server for reporting of the service switch information. This method shall support the URI query parameters specified in the table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceSwitchInfo | M | 1 | Create a new service switch information subscription. |

Table 6.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceSwitchInfo | M | 1 | 201 Created | The individual service switch information subscription resource created successfully. The information about the confirmed subscription at the PIN server is provided in the response body. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/pin-as-serviceswitch/<apiVersion>/subscriptions/{subscriptionId} |

Table 6.2.3.2.3.1-5: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 6.2.3.2.4	Resource Custom Operations

None.

#### 6.2.3.3	Resource: Individual Service Switch Information Subscription

##### 6.2.3.3.1	Description

This resource represents the individual service switch information subscription of a PAS at a given PIN server.

##### 6.2.3.3.2	Resource Definition

Resource URI: {apiRoot}/pin-as-serviceswitch/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in the table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.1. |
| subscriptionId | string | Identifies an individual service switch information subscription. |

##### 6.2.3.3.3	Resource Standard Methods

###### 6.2.3.3.3.1	GET

This method retrieves the service switch information subscription information at PIN server. This method shall support the URI query parameters specified in the table 6.2.3.3.3.1-1.

Table 6.2.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.1-2 and the response data structures and response codes specified in table 6.2.3.3.3.1-3.

Table 6.2.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceSwitchInfo | M | 1 | 200 OK | The service switch information subscription information is returned by the PIN server. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

Table 6.2.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

###### 6.2.3.3.3.2	PATCH

This method partially updates the individual service switch information subscription information at the PIN server. This method shall support the URI query parameters specified in the table 6.2.3.3.3.2-1.

Table 6.2.3.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.2-2 and the response data structures and response codes specified in table 6.2.3.3.3.2-3.

Table 6.2.3.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceSwitchInfoPatch | M | 1 | Request to partially update the individual service information subscription matching the subscriptionId at the PIN server. |

Table 6.2.3.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceSwitchInfo | M | 1 | 200 OK | The individual service switch information subscription matching the subscriptionId was modified successfully and the updated service switch information subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The individual service switch information subscription matching the subscriptionId was modified successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

Table 6.2.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

###### 6.2.3.3.3.3	PUT

This method updates the service switch information subscription information at the PIN server by completely replacing the existing subscription data. This method shall support the URI query parameters specified in the table 6.2.3.3.3.3-1.

Table 6.2.3.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.3-2 and the response data structures and response codes specified in table 6.2.3.3.3.3-3.

Table 6.2.3.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceSwitchInfo | M | 1 | Details of individual service switch information subscription matching the subscriptionId to be updated at the PIN server. |

Table 6.2.3.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceSwitchInfo | M | 1 | 200 OK | The individual service switch information subscription matching the subscriptionId was modified successfully and the updated service switch subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The individual service switch information subscription matching the subscriptionId was modified successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

Table 6.2.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

###### 6.2.3.3.3.4	DELETE

This method removes the service switch information subscription information from the PIN server. This method shall support the URI query parameters specified in the table 6.2.3.3.3.4-1.

Table 6.2.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.3.3.4-2 and the response data structures and response codes specified in table 6.2.3.3.3.4-3.

Table 6.2.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a | M | 1 | 204 No Content | The individual service switch information subscription matching the subscriptionId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

Table 6.2.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

##### 6.2.3.3.3	Resource Custom Operations

None.

### 6.2.4	Custom Operations without associated resources

None.

### 6.2.5	Notifications

#### 6.2.5.1	General

Table 6.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Service Switch Information Notification | {notificationAddr} | POST | Notifies the subscriber PAS the service switch information. |

#### 6.2.5.1	Service Switch Information Notification

##### 6.2.5.1.1	Description

Service Switch Information Notification is used by the PIN server to notify a PAS with service switch information matching the filter criteria.

##### 6.2.5.1.2	Target URI

The callback URI {notificationAddr} shall be used with the callback URI variables defined in table 6.2.5.1.2-1.

Table 6.2.5.1.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationAddr | Uri | Callback reference provided by the PAS during service switch information subscription creation or update procedure. |

##### 6.2.5.1.3	Standard Methods

###### 6.2.5.1.3.1	POST

This method shall support the request data structures specified in table 6.2.5.1.3.1-1 and the response data structures and response codes specified in table 6.2.5.1.3.1-2.

Table 6.2.5.1.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceSwitchInfoNotification | M | 1 | Notification of service switch information. |

Table 6.2.5.1.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.2.5.1.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative PAS towards which the notification should be redirected. |

Table 6.2.5.1.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative PAS towards which the notification should be redirected. |

### 6.2.6	Data Model

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the PIN_ASServiceSwitch API.

Table 6.2.6.1-1: PIN_ASServiceSwitch API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EventType | 6.2.6.3.2 | Represents the event type for service switch information subscription. |  |
| ServiceSwitchInfo | 6.2.6.2.2 | Represents the service switch information subscription. |  |
| ServiceSwitchInfoPatch | 6.2.6.2.3 | Used to request the partial update of service switch information subscription. |  |
| ServiceSwitchInfoNotification | 6.2.6.2.4 | Service switch information for notification |  |
| ServiceSwitchReportInfo | 6.2.6.2.5 | List of notifications that include the information of the service switch. |  |

Table 6.2.6.1-2 specifies data types re-used by the PIN_ASServiceSwitch API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the PIN_ASServiceSwitch API.

Table 6.2.6.1-2: PIN_ASServiceSwitch API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [2] | Used to capture the expiration time of PAS subscription for service switch information reporting. |  |
| FlowInfo | 3GPP TS 29.122 [2] | Represents IP flow information. |  |
| SupportedFeatures | 3GPP TS 29.571 [11] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |

#### 6.2.6.2	Structured data types

##### 6.2.6.2.1	Introduction

##### 6.2.6.2.2	Type: ServiceSwitchInfo

Table 6.2.6.2.2-1: Definition of type ServiceSwitchInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subsEvent | EventType | M | 1 | Identifies the event type for which the subscriber is to be notified. |  |
| notificationAddr | Uri | M | 1 | URI where the notification should be delivered to. This attribute shall be present in HTTP POST message to PIN server and maybe present in HTTP PUT request. |  |
| pinId | string | M | 1 | Identifies a PIN. (see 3GPP TS 23.542 [10]). Its encoding shall comply with the UE policy part type URSP as defined in clause 5.2 of 3GPP TS 24.526 [12]. |  |
| expTime | DateTime | O | 0..1 | Indicates the expiration time of the subscription. To maintain an active registration status, a registration update is required before the expiration time. If the expiration time is not present, then it indicates that the PAS subscription never expires. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.2.8. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 6.2.6.2.3	Type: ServiceSwitchInfoPatch

Table 6.2.6.2.3-1: Definition of type ServiceSwitchInfoPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| expTime | DateTime | O | 0..1 | Indicates the proposed expiration time of the subscription. |  |
| notificationAddr | Uri | O | 0..1 | Updated URI where the service switch information notification should be delivered to. |  |
| pinId | string | O | 0..1 | Identifies a PIN. (see 3GPP TS 23.542 [10]). Its encoding shall comply with the UE policy part type URSP as defined in clause 5.2 of 3GPP TS 24.526 [12]. |  |
| subsEvent | EventType | O | 0..1 | Updated event type for which the subscriber is to be notified. |  |

##### 6.2.6.2.4	Type: ServiceSwitchInfoNotification

Table 6.2.6.2.4-1: Definition of type ServiceSwitchInfoNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subsId | string | M | 1 | Identifies the individual service switch information subscription for which the service switch information notification is delivered. |  |
| repInfo | ServiceSwitchReportInfo | M | 1 | List of service switch report information applicable to the subscription identifier. |  |

##### 6.2.6.2.5	Type: ServiceSwitchReportInfo

Table 6.2.6.2.5-1: Definition of type ServiceSwitchReportInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| acId | string | M | 1 | Identifies an application client identifier. |  |
| pinId | string | M | 1 | Identifies a PIN. (see 3GPP TS 23.542 [10]). Its encoding shall comply with the UE policy part type URSP as defined in clause 5.2 of 3GPP TS 24.526 [12]. |  |
| sessionId | string | M | 1 | Identifies an application session. |  |
| targetPineId | string | M | 1 | Identifies the target PINE that the service is switched to. |  |
| sessionDes | FlowInfo | O | 0..1 | Identifies the descriptor of application traffic flows |  |

#### 6.2.6.3	Simple data types and enumerations

##### 6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.2.6.3.2	Enumeration: EventType

The enumeration EventType represents the supported event type of service switch.

Table 6.2.6.3.2-1: Enumeration EventType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SERVICE_SWITCH_INFO | Service switch happens in the PIN |  |

### 6.2.7	Error Handling

#### 6.2.7.1	General

For the PIN_ASServiceSwitch API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [2]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [2] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [2].

In addition, the requirements in the following clauses are applicable for the PIN_ASServiceSwitch API.

#### 6.2.7.2	Protocol Errors

No specific protocol errors for the PIN_ASServiceSwitch API are specified.

#### 6.2.7.3	Application Errors

No specific application errors for the PIN_ASServiceSwitch API are specified.

### 6.2.8	Feature negotiation

The optional features in table 6.2.8-1 are defined for the PIN_ASServiceSwitch API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [2].

Table 6.2.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| n/a |  |  |

### 6.2.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [2] shall apply for the PIN_ASServiceSwitch API.

## 6.3	PIN_ASServiceContinuity API

### 6.3.1	Introduction

The PIN_ASServiceContinuity service shall use the PIN_ASServiceContinuity API.

The API URI of the PIN_ASServiceContinuity API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [2], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

-	The <apiName> shall be "pin-as-servicecontinuity".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [2].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (i.e. PIN Server) takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

### 6.3.2	Usage of HTTP

The provisions of clause 5.2.2 of 3GPP TS 29.122 [2] shall apply for the PIN_ASServiceContinuity API.

### 6.3.3	Resources

#### 6.3.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.3.3.1-1 depicts the resource URIs structure for the PIN_ASServiceContinuity API.

Figure 6.3.3.1-1: Resource URI structure of the PIN_ASServiceContinuity API

Table 6.3.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.3.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Service Continuity Information Subscriptions | /subscriptions | POST | Create a subscription for reporting of service continuity information to the PAS. |
| Individual Service Continuity Information Subscription | /subscriptions/{subscriptionId} | GET | Retrieve the Individual service continuity information subscription information identified by subscriptionId. |
|  |  | PUT | Fully replace the individual service continuity information subscription identified by subscriptionId. |
|  |  | PATCH | Partially update the individual service continuity information subscription identified by subscriptionId. |
|  |  | DELETE | Remove the individual service continuity information subscription identified by subscriptionId. |

#### 6.3.3.2	Resource: Service Continuity Information Subscriptions

##### 6.3.3.2.1	Description

This resource represents all service continuity information subscriptions at a given PIN server.

##### 6.3.3.2.2	Resource Definition

Resource URI: {apiRoot}/pin-as-servicecontinuity/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in the table 6.3.3.2.2-1.

Table 6.3.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.3.1. |

##### 6.3.3.2.3	Resource Standard Methods

###### 6.3.3.2.3.1	POST

This method creates the service continuity information subscription at the PIN server for reporting of the service continuity information. This method shall support the URI query parameters specified in the table 6.3.3.2.3.1-1.

Table 6.3.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.2.3.1-2 and the response data structures and response codes specified in table 6.3.3.2.3.1-3.

Table 6.3.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceContinuityInfo | M | 1 | Create a new service continuity information subscription. |

Table 6.3.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceContinuityInfo | M | 1 | 201 Created | The individual service continuity information subscription resource created successfully. The information about the confirmed subscription at the PIN server is provided in the response body. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.3.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/pin-as-servicecontinuity/<apiVersion>/subscriptions/{subscriptionId} |

Table 6.3.3.2.3.1-5: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 6.3.3.2.4	Resource Custom Operations

None.

#### 6.3.3.3	Resource: Individual Service Continuity Information Subscription

##### 6.3.3.3.1	Description

This resource represents the individual service continuity information subscription of a PAS at a given PIN server.

##### 6.3.3.3.2	Resource Definition

Resource URI: {apiRoot}/pin-as-servicecontinuity/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in the table 6.3.3.3.2-1.

Table 6.3.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.3.1. |
| subscriptionId | string | Identifies an individual service continuity information subscription. |

##### 6.3.3.3.3	Resource Standard Methods

###### 6.3.3.3.3.1	GET

This method retrieves the service continuity information subscription information at PIN server. This method shall support the URI query parameters specified in the table 6.3.3.3.3.1-1.

Table 6.3.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.3.3.1-2 and the response data structures and response codes specified in table 6.3.3.3.3.1-3.

Table 6.3.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.3.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceContinuityInfo | M | 1 | 200 OK | The service continuity information subscription information is returned by the PIN server. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.3.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

Table 6.3.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

###### 6.3.3.3.3.2	PATCH

This method partially updates the individual service continuity information subscription information at the PIN server. This method shall support the URI query parameters specified in the table 6.3.3.3.3.2-1.

Table 6.3.3.3.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.3.3.2-2 and the response data structures and response codes specified in table 6.3.3.3.3.2-3.

Table 6.3.3.3.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceContinuityInfoPatch | M | 1 | Request to partially update the individual service information subscription matching the subscriptionId at the PIN server. |

Table 6.3.3.3.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceContinuityInfo | M | 1 | 200 OK | The individual service continuity information subscription matching the subscriptionId was modified successfully and the updated service continuity information subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The individual service continuity information subscription matching the subscriptionId was modified successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the PATCH method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.3.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

Table 6.3.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

###### 6.3.3.3.3.3	PUT

This method updates the service continuity information subscription information at the PIN server by completely replacing the existing subscription data. This method shall support the URI query parameters specified in the table 6.3.3.3.3.3-1.

Table 6.3.3.3.3.3-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.3.3.3-2 and the response data structures and response codes specified in table 6.3.3.3.3.3-3.

Table 6.3.3.3.3.3-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceContinuityInfo | M | 1 | Details of individual service continuity information subscription matching the subscriptionId to be updated at the PIN server. |

Table 6.3.3.3.3.3-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ServiceContinuityInfo | M | 1 | 200 OK | The individual service continuity information subscription matching the subscriptionId was modified successfully and the updated service continuity subscription information is returned in the response. |
| n/a |  |  | 204 No Content | The individual service continuity information subscription matching the subscriptionId was modified successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.3.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

Table 6.3.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

###### 6.3.3.3.3.4	DELETE

This method removes the service continuity information subscription information from the PIN server. This method shall support the URI query parameters specified in the table 6.3.3.3.3.4-1.

Table 6.3.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.3.3.4-2 and the response data structures and response codes specified in table 6.3.3.3.3.4-3.

Table 6.3.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.3.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a | M | 1 | 204 No Content | The individual service continuity information subscription matching the subscriptionId is deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative PIN server. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.3.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

Table 6.3.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative PIN server. |

##### 6.3.3.3.3	Resource Custom Operations

None.

### 6.3.4	Custom Operations without associated resources

None.

### 6.3.5	Notifications

#### 6.3.5.1	General

Table 6.3.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Service Continuity Information Notification | {notificationAddr} | POST | Notifies the subscriber PAS the service continuity information. |

#### 6.3.5.1	Service Continuity Information Notification

##### 6.3.5.1.1	Description

Service Continuity Information Notification is used by the PIN server to notify a PAS with service continuity information matching the filter criteria.

##### 6.3.5.1.2	Target URI

The callback URI {notificationAddr} shall be used with the callback URI variables defined in table 6.3.5.1.2-1.

Table 6.3.5.1.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notificationAddr | Uri | Callback reference provided by the PAS during service continuity information subscription creation or update procedure. |

##### 6.3.5.1.3	Standard Methods

###### 6.3.5.1.3.1	POST

This method shall support the request data structures specified in table 6.3.5.1.3.1-1 and the response data structures and response codes specified in table 6.3.5.1.3.1-2.

Table 6.3.5.1.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ServiceContinuityInfoNotification | M | 1 | Notification of service continuity information. |

Table 6.3.5.1.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Notification is acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative EAS where the notification should be sent. Redirection handling is described in clause 5.2.10 of TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.3.5.1.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative PAS towards which the notification should be redirected. |

Table 6.3.5.1.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative PAS towards which the notification should be redirected. |

### 6.3.6	Data Model

#### 6.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the PIN_ASServiceContinuity API.

Table 6.3.6.1-1: PIN_ASServiceContinuity API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| EventType | 6.3.6.3.2 | Represents the event type for service continuity information subscription. |  |
| ServiceContinuityInfo | 6.3.6.3.2 | Represents the service continuity information subscription. |  |
| ServiceContinuityInfoPatch | 6.3.6.3.3 | Used to request the partial update of service continuity information subscription. |  |
| ServiceContinuityInfoNotification | 6.3.6.3.4 | Service continuity information for notification |  |
| ServiceContinuityReportInfo | 6.3.6.3.5 | List of notifications that include the information of the service continuity. |  |

Table 6.3.6.1-2 specifies data types re-used by the PIN_ASServiceContinuity API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the PIN_ASServiceContinuity API.

Table 6.3.6.1-2: PIN_ASServiceContinuity API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [2] | Used to capture the expiration time of PAS subscription for service continuity information reporting. |  |
| FlowInfo | 3GPP TS 29.122 [2] | Represents IP flow information. |  |
| SupportedFeatures | 3GPP TS 29.571 [11] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |

#### 6.3.6.2	Structured data types

##### 6.3.6.2.1	Introduction

##### 6.3.6.2.2	Type: ServiceContinuityInfo

Table 6.3.6.2.2-1: Definition of type ServiceContinuityInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subsEvent | EventType | M | 1 | Identifies the event type for which the subscriber is to be notified. |  |
| notificationAddr | Uri | M | 1 | URI where the notification should be delivered to. This attribute shall be present in HTTP POST message to PIN server and maybe present in HTTP PUT request. |  |
| pinId | string | M | 1 | Identifies a PIN. (see 3GPP TS 23.542 [10]). Its encoding shall comply with the UE policy part type URSP as defined in clause 5.2 of 3GPP TS 24.526 [12]. |  |
| expTime | DateTime | O | 0..1 | Indicates the expiration time of the subscription. To maintain an active registration status, a registration update is required before the expiration time. If the expiration time is not present, then it indicates that the PAS subscription never expires. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.2.8. This attribute shall be present only when feature negotiation needs to take place. |  |

##### 6.3.6.2.3	Type: ServiceContinuityInfoPatch

Table 6.3.6.2.3-1: Definition of type ServiceContinuityInfoPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| expTime | DateTime | O | 0..1 | Indicates the proposed expiration time of the subscription. |  |
| notificationAddr | Uri | O | 0..1 | Updated URI where the service continuity information notification should be delivered to. |  |
| pinId | string | O | 0..1 | Identifies a PIN. (see 3GPP TS 23.542 [10]). Its encoding shall comply with the UE policy part type URSP as defined in clause 5.2 of 3GPP TS 24.526 [12]. |  |
| subsEvent | EventType | O | 0..1 | Updated event type for which the subscriber is to be notified. |  |

##### 6.3.6.2.4	Type: ServiceContinuityInfoNotification

Table 6.3.6.2.4-1: Definition of type ServiceContinuityInfoNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subsId | string | M | 1 | Identifies the individual service continuity information subscription for which the service continuity information notification is delivered. |  |
| repInfo | ServiceContinuityReportInfo | M | 1 | List of service continuity report information applicable to the subscription identifier. |  |

##### 6.3.6.2.5	Type: ServiceContinuityReportInfo

Table 6.3.6.2.5-1: Definition of type ServiceContinuityReportInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| acId | string | M | 1 | Identifies an application client identifier. |  |
| pegcId | string | M | 1 | Identifies a PEGC. |  |
| pinId | string | M | 1 | Identifies a PIN. (see 3GPP TS 23.542 [10]). Its encoding shall comply with the UE policy part type URSP as defined in clause 5.2 of 3GPP TS 24.526 [12]. |  |
| serviceId | string | M | 1 | Identifies a PIN service. |  |
| sessionId | string | M | 1 | Identifies an application session. |  |
| targetPineId | string | M | 1 | Identifies the target PINE. |  |
| sessionDes | FlowInfo | O | 0..1 | Identifies the descriptor of application traffic flows |  |

#### 6.3.6.3	Simple data types and enumerations

##### 6.3.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.3.6.3.2	Enumeration: EventType

The enumeration EventType represents the supported event type of service continuity.

Table 6.3.6.3.2-1: Enumeration EventType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SERVICE_CONTINUITY_INFO | Service continuity happens in the PIN |  |

### 6.3.7	Error Handling

#### 6.3.7.1	General

For the PIN_ASServiceContinuity API, HTTP error responses shall be supported as specified in clause 5.2.6 of 3GPP TS 29.122 [2]. Protocol errors and application errors specified in clause 5.2.6 of 3GPP TS 29.122 [2] shall be supported for the HTTP status codes specified in table 5.2.6-1 of 3GPP TS 29.122 [2].

In addition, the requirements in the following clauses are applicable for the PIN_ASServiceContinuity API.

#### 6.3.7.2	Protocol Errors

No specific protocol errors for the PIN_ASServiceContinuity API are specified.

#### 6.3.7.3	Application Errors

No specific application errors for the PIN_ASServiceContinuity API are specified.

### 6.3.8	Feature negotiation

The optional features in table 6.3.8-1 are defined for the PIN_ASServiceContinuity API. They shall be negotiated using the extensibility mechanism defined in clause 5.2.7 of 3GPP TS 29.122 [2].

Table 6.3.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| n/a |  |  |

### 6.3.9	Security

The provisions of clause 6 of 3GPP TS 29.122 [2] shall apply for the PIN_ASServiceContinuity API.

# 7	Using Common API Framework

## 7.1	General

When CAPIF is used with a PIN Server service, the PIN Server shall support the following functionalities as defined in 3GPP TS 29.222 [7]:

-	the API exposing function and the related APIs over CAPIF-2/2e and CAPIF-3/3e reference points;

-	the API publishing function and the related APIs over CAPIF-4/4e reference point;

-	the API management function and the related APIs over CAPIF-5/5e reference point; and

-	at least one of the security methods for authentication and authorization, and the related security mechanisms.

In a centralized deployment as defined in 3GPP TS 23.222 [6], where the CAPIF core function and the API provider domain functions are co-located, the interactions between the CAPIF core function and the API provider domain functions may be independent of the CAPIF-3/3e, CAPIF-4/4e and CAPIF-5/5e reference points.

When CAPIF is used with a PIN Server service, the PIN Server shall register all the northbound APIs features in the CAPIF Core Function.

## 7.2	Security

When CAPIF is used for external exposure, before invoking an API exposed by the PIN Server, the service API consumer acting as an API invoker shall negotiate the security method (PKI, TLS-PSK or OAuth 2.0) with the CAPIF core function and ensure that the PIN Server has enough credentials to authenticate the service API consumer, as defined in clauses 5.6.2.2 and 6.2.2.2 of 3GPP TS 29.222 [7].

If PKI or TLS-PSK is selected as the security method to be used between the service API consumer and the PIN Server, upon API invocation, the PIN Server shall retrieve the authorization information from the CAPIF core function as described in clause 5.6.2.4 of 3GPP TS 29.222 [7].

As indicated in 3GPP TS 33.122 [8], the access to the PIN Server APIs may be authorized by means of the OAuth 2.0 protocol (see IETF RFC 6749 [9]), using the "Client Credentials" authorization grant, where the CAPIF core function (see 3GPP TS 29.222 [7]) plays the role of the authorization server.

NOTE 1:	In this release, only "Client Credentials" authorization grant is supported.

If OAuth 2.0 is selected as the security method to be used between the service API consumer and the PIN Server, the service API consumer shall, prior to consuming the services offered by the PIN Server APIs, obtain a "token" from the authorization server, by invoking the Obtain_Authorization service operation as described in clause 5.6.2.3.2 of 3GPP TS 29.222 [7].

The PIN Server APIs do not define any scopes for OAuth 2.0 authorization. It is the PIN Server responsibility to check whether the service API consumer is authorized to use an API based on the provided "token". Once the PIN Server verifies the "token", it shall check whether the PIN Server identifier in the "token" matches its own published identifier, and whether the API name in the "token" matches its own published API name. If those checks are passed, the service API consumer has full authority to access any resource or operation provided by the invoked API.

NOTE 2:	For the aforementioned security methods, the PIN Server needs to apply admission control according to access control policies after performing the authorization checks.

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [5] and clause 5B of 3GPP TR 21.900 [7]).

# A.2	PIN_ASRegistration API

openapi: 3.0.0

info:

title: PINServer PAS Registration_API

version: 1.0.0-alpha.7

description: |

API for PAS registration to PIN server.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.583 V18.0.0; Application layer support for Personal IoT Network (PINAPP);

Personal IoT Network (PIN) Server Services; Stage 3.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.583/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/pin-as-registration/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.3 of 3GPP TS 29.583.

paths:

/registrations:

post:

summary: Create a new PAS Registration

operationId: CreatePASRegistration

tags:

- PAS Registrations (Collection)

description: Register a new PAS at the PIN Server.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PASRegistration'

responses:

'201':

description: PAS information is registered successfully at PIN server.

content:

application/json:

schema:

$ref: '#/components/schemas/PASRegistration'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/registrations/{registrationId}:

get:

summary: Get an Individual PAS Registration

operationId: GetIndPASReg

tags:

- Individual PAS Registration (Document)

description: Retrieve an Individual PAS registration resource.

parameters:

- name: registrationId

in: path

description: Registration Id.

required: true

schema:

type: string

responses:

'200':

description: OK (The PAS registration information at the PIN Server).

content:

application/json:

schema:

$ref: '#/components/schemas/PASRegistration'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an Individual PAS Registration

operationId: UpdateIndPASReg

tags:

- Individual PAS Registration (Document)

description: Fully replace an existing PAS Registration resource.

parameters:

- name: registrationId

in: path

description: PAS Registration Id.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PASRegistration'

responses:

'200':

description: OK (The PAS registration information is updated successfully).

content:

application/json:

schema:

$ref: '#/components/schemas/PASRegistration'

'204':

description: No Content. The PAS registration information is updated successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an Individual PAS Registration

operationId: ModifyIndPASReg

tags:

- Individual PAS Registration (Document)

description: Partially update an existing PAS Registration resource.

parameters:

- name: registrationId

in: path

description: PAS registration Id.

required: true

schema:

type: string

requestBody:

description: Partial update an existing PAS registration resource.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/PASRegistrationPatch'

responses:

'200':

description: >

The Individual PAS registration is successfully modified and

the updated registration information is returned in the response.

content:

application/json:

schema:

$ref: '#/components/schemas/PASRegistration'

'204':

description: No Content. The Individual PAS registration is successfully modified.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an Individual PAS Registration

operationId: DeleteIndPASReg

tags:

- Individual PAS Registration (Document)

description: Delete an existing PAS registration at PIN server.

parameters:

- name: registrationId

in: path

description: Registration Id.

required: true

schema:

type: string

responses:

'204':

description: The individual PAS registration is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

PASRegistration:

type: object

description: Represents an PAS registration information.

properties:

conInfo:

$ref: '#/components/schemas/ConnectivityInfo'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

passId:

type: string

description: Indentifies a PIN service

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- conInfo

- passId

ConnectivityInfo:

type: object

description: Represents a connection information of PAS.

properties:

fqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

ipv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

uri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

anyOf:

- required: [uri]

- required: [fqdn]

- required: [ipv4Addr]

- required: [ipv6Addr]

PASRegistrationPatch:

type: object

description: Represents partial update request of individual PAS registration information.

properties:

conInfo:

$ref: '#/components/schemas/ConnectivityInfo'

expTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTimeRm'

passId:

type: string

description: Identifies a PIN service provided by PAS.

anyOf:

- required: [conInfo]

- required: [expTime]

- required: [passId]

# A.3	PIN_ASServiceSwitch API

openapi: 3.0.0

info:

title: PINServer Service Switch Information_API

version: 1.0.0-alpha.6

description: |

API for PINServer Service Switch Information Reporting.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.583 V18.0.0; Application layer support for Personal IoT Network (PINAPP);

Personal IoT Network (PIN) Server Services; Stage 3.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.583/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/pin-as-serviceswitch/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.2.1 of 3GPP TS 29.583.

paths:

/subscriptions:

post:

summary: Creates a new Individual Service Switch Information Subscriptions resource

operationId: CreateServiceSwitchInfo

tags:

- Service Switch Information Subscriptions (Collection)

description: Create a Subscription for reporting service switch information to PAS.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSwitchInfo'

responses:

'201':

description: >

Created (The individual Service Switch information subscription resource is created successfully)

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSwitchInfo'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

ServiceSwitchInfoNotification:

'{request.body#/notificationAddr}':

post:

requestBody: # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSwitchInfoNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: Read an Individual Service Switch Information Subscriptions resource

operationId: ReadIndServiceSwitchInfo

tags:

- Individual Service Switch Information Subscription (Document)

description: Retrieve an Individual Service Switch information subscription information.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

responses:

'200':

description: OK (Successfully get the Service Switch information subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSwitchInfo'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an Individual Service Switch Information Subscriptions resource

operationId: UpdateIndServiceSwitchInfo

tags:

- Individual Service Switch Information Subscription (Document)

description: Fully replace an existing Individual Service Switch information Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSwitchInfo'

responses:

'200':

description: OK (Successfully modified The individual Service Switch information).

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSwitchInfo'

'204':

description: No Content.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an Individual Service Switch Information Subscriptions resource

operationId: ModifyIndServiceSwitchInfo

tags:

- Individual Service Switch Information Subscription (Document)

description: Partially update an existing Individual ServiceSwitch information.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

description: Partial update an existing Individual ServiceSwitch information.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/ServiceSwitchInfoPatch'

responses:

'200':

description: >

OK (The Individual ServiceSwitch information Subscription is successfully modified

and the updated subscription information is returned in the response).

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSwitchInfo'

'204':

description: >

No Content (The individual Service Switch information subscription was modified

successfully).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an Individual Service Switch Information Subscriptions resource

operationId: DeleteIndServiceSwitchInfo

tags:

- Individual Service Switch Information Subscription (Document)

description: Delete an existing Individual ServiceSwitchinformation Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

responses:

'204':

description: The individual subscription is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Components

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ServiceSwitchInfo:

type: object

description: Represents an Individual Service Switch Information Subscription.

properties:

subsEvent:

$ref: '#/components/schemas/EventType'

notificationAddr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

pinId:

type: string

description: Identifies a PIN.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- subsEvent

- notificationAddr

- pinId

ServiceSwitchInfoPatch:

type: object

description: Represents the partial update of Individual Service Switch Information.

properties:

subsEvent:

$ref: '#/components/schemas/EventType'

notificationAddr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

pinId:

type: string

description: Identifies a PIN.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

ServiceSwitchInfoNotification:

type: object

description: Represent the service switch information for notification.

properties:

subsId:

type: string

description: >

Identifies the individual service switch information subscription for which

the service switch information notification is delivered.

repInfo:

$ref: '#/components/schemas/ServiceSwitchReportInfo'

required:

- subsId

- repInfo

ServiceSwitchReportInfo:

type: object

description: List of notifications that include the information of the service switch.

properties:

acId:

type: string

description: Identifies an application client identifier.

pinId:

type: string

description: Identifies a PIN.

sessionId:

type: string

description: Identifies an application session.

targetPineId:

type: string

description: Identifies the PINE that the service is switched to.

sessionDes:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

required:

- acId

- pinId

- sessionId

- targetPineId

EventType:

anyOf:

- type: string

enum:

- SERVICE_SWITCH_INFO

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Indicates service switch happens in a PIN.

Possible values are:

- SERVICE_SWITCH_INFO: Indicates service switch happens in a PIN.

# A.4	PIN_ASServiceContinuity API

openapi: 3.0.0

info:

title: PINServer Service Continuity Information_API

version: 1.0.0-alpha.6

description: |

API for PINServer Service Continuity Information Reporting.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.583 V18.0.0; Application layer support for Personal IoT Network (PINAPP);

Personal IoT Network (PIN) Server Services; Stage 3.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.583/'

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/pin-as-servicecontinuity/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.3.1 of 3GPP TS 29.583.

paths:

/subscriptions:

post:

summary: Creates a new Individual Service Continuity Information Subscriptions resource

operationId: CreateServiceContinuityInfo

tags:

- Service Continuity Information Subscriptions (Collection)

description: Create a Subscription for reporting service continuity information to PAS.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceContinuityInfo'

responses:

'201':

description: >

Created (The individual Service Continuity information subscription resource is created successfully)

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceContinuityInfo'

headers:

Location:

description: 'Contains the URI of the newly created resource'

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

ServiceContinuityInfoNotification:

'{request.body#/notificationAddr}':

post:

requestBody: # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceContinuityInfoNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: Read an Individual Service Continuity Information Subscriptions resource

operationId: ReadIndServiceContinuityInfo

tags:

- Individual Service Continuity Information Subscription (Document)

description: Retrieve an Individual Service Continuity information subscription information.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

responses:

'200':

description: OK (Successfully get the Service Continuity information subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceContinuityInfo'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an Individual Service Continuity Information Subscriptions resource

operationId: UpdateIndServiceContinuityInfo

tags:

- Individual Service Continuity Information Subscription (Document)

description: Fully replace an existing Individual Service Continuity information Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceContinuityInfo'

responses:

'200':

description: OK (Successfully modified The individual Service Continuity information).

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceContinuityInfo'

'204':

description: No Content.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an Individual Service Continuity Information Subscriptions resource

operationId: ModifyIndServiceContinuityInfo

tags:

- Individual Service Continuity Information Subscription (Document)

description: Partially update an existing Individual ServiceContinuity information.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

requestBody:

description: Partial update an existing Individual ServiceContinuity information.

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/ServiceContinuityInfoPatch'

responses:

'200':

description: >

OK (The Individual ServiceContinuity information Subscription is successfully modified

and the updated subscription information is returned in the response).

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceContinuityInfo'

'204':

description: >

No Content (The individual Service Continuity information subscription was modified

successfully).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an Individual Service Continuity Information Subscriptions resource

operationId: DeleteIndServiceContinuityInfo

tags:

- Individual Service Continuity Information Subscription (Document)

description: Delete an existing Individual Service Continuity information Subscription.

parameters:

- name: subscriptionId

in: path

description: Subscription Id.

required: true

schema:

type: string

responses:

'204':

description: The individual subscription is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Components

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ServiceContinuityInfo:

type: object

description: Represents an Individual Service Continuity Information Subscription.

properties:

subsEvent:

$ref: '#/components/schemas/EventType'

notificationAddr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

pinId:

type: string

description: Identifies a PIN.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- subsEvent

- notificationAddr

- pinId

ServiceContinuityInfoPatch:

type: object

description: Represents the partial update of Individual Service Continuity Information.

properties:

subsEvent:

$ref: '#/components/schemas/EventType'

notificationAddr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

pinId:

type: string

description: Identifies a PIN.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

ServiceContinuityInfoNotification:

type: object

description: Represent the service continuity information for notification.

properties:

subsId:

type: string

description: >

Identifies the individual service continuity information subscription for which

the service continuity information notification is delivered.

repInfo:

$ref: '#/components/schemas/ServiceContinuityReportInfo'

required:

- subsId

- repInfo

ServiceContinuityReportInfo:

type: object

description: List of notifications that include the information of the service continuity.

properties:

acId:

type: string

description: Identifies an application client identifier.

pinId:

type: string

description: Identifies a PIN.

pegcId:

type: string

description: Identifies a PEGC.

serviceId:

type: string

description: Identifies a PIN service.

sessionId:

type: string

description: Identifies an application session.

targetPineId:

type: string

description: Identifies the PINE.

sessionDes:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

required:

- acId

- pinId

- pegcId

- serviceId

- sessionId

- targetPineId

EventType:

anyOf:

- type: string

enum:

- SERVICE_CONTINUITY_INFO

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Indicates service continuity happens in a PIN.

Possible values are:

- SERVICE_CONTINUITY_INFO: Indicates service continuity happens in a PIN.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-03 | CT3#126 |  |  |  |  | TS skeleton for Application layer support for Personal IoT Network (PINAPP); Personal IoT Network (PIN) Server Services. | 0.0.0 |
| 2023-04 | CT3#127e | C3-231511 |  |  |  | Inclusion of C3-231265 | 0.1.0 |
| 2023-09 | CT3#129 | C3-233739 |  |  |  | Inclusion of C3-233576, C3-233688, C3-233689, C3-233690 Editorial correction from the rapporteur. | 0.2.0 |
| 2023-10 | CT3#130 | C3-234662 |  |  |  | Inclusion of C3-234511, C3-234512, C3-234514, C3-234602, C3-234603 Editorial correction from the rapporteur. | 0.3.0 |
| 2023-11 | CT3#131 | C3-235465 |  |  |  | Inclusion of C3-235521, C3-235522, C3-235546, C3-235552 Editorial correction from the rapporteur. | 0.4.0 |
| 2023-12 | CT#102 | CP-233291 |  |  |  | Presentation to TSG CT for information. | 1.0.0 |
| 2024-03 | CT3#133 | C3-241655 |  |  |  | Inclusion of C3-241545, C3-241546, C3-241547, C3-241575, C3-241592 Editorial correction from the rapporteur. | 1.1.0 |
| 2024-03 | CT#103 | CP-240216 |  |  |  | Presentation to TSG CT for approval. | 2.0.0 |
| 2024-03 | CT#103 | CP-240216 |  |  |  | Approved by TSG CT. | 18.0.0 |