| 3GPP TS 31.121 V17.3.0 (2024-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; UICC-terminal interface; Universal Subscriber Identity Module (USIM) application test specification (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	31

Introduction	32

1	Scope	33

2	References	33

3	Definitions and abbreviations	36

3.1	Definitions	36

3.2	Symbols	37

3.3	Abbreviations	37

3.4	Coding Conventions	39

3.5	Generic procedures for E-UTRAN/UTRAN/GERAN/IMS/NB-IoT/5G-NR	39

3.6	Applicability	41

3.6.1	Applicability of the present document	41

3.6.2	Applicability to terminal equipment	41

3.6.3	Applicability of the individual tests	41

3.7	Table of optional features	42

3.8	Applicability table	46

4	Default values for UICC	62

4.1	Definition of default values for USIM-Terminal interface testing (Default UICC)	62

4.1.1	Values of the EF's (Default UICC)	62

4.1.1.1	EFIMSI (IMSI)	62

4.1.1.3	EFLOCI (Location Information)	63

4.1.1.4	EFKeys (Ciphering and Integrity Keys)	63

4.1.1.5	EFKeysPS (Ciphering and Integrity Keys for Packet Switched domain)	63

4.1.1.6	EFACC (Access Control Class)	63

4.1.1.7	EFFPLMN (Forbidden PLMNs)	63

4.1.1.8	EFUST (USIM Service Table)	63

4.1.1.9	EFEST (Enable Service Table)	64

4.1.1.10	EFADN (Abbreviated Dialling Number)	64

4.1.1.11	EFPLMNwACT (User Controlled PLMN Selector with Access Technology)	64

4.1.1.12	EFOPLMNwACT (Operator Controlled PLMN Selector with Access Technology)	65

4.1.1.13	Void	66

4.1.1.14	PIN	66

4.1.1.15	PIN2	66

4.1.1.16	Unblock PIN	66

4.1.1.17	Unblock PIN2	66

4.1.1.18	Other Values of the USIM	66

4.1.1.19	EFPSLOCI (Packet Switch Location Information)	66

4.1.1.20	Universal PIN	67

4.1.1.21	Unblock Universal PIN	67

4.2	Definition of FDN UICC	67

4.2.1	Values of the EF's (FDN UICC)	67

4.2.1.3	EFFDN (Fixed Dialling Numbers)	67

4.2.1.4	EFECC (Emergency Call Codes)	68

4.2.1.5	Other Values of the USIM	68

4.3	Void	69

4.4	Definition of E-UTRAN/EPC UICC	69

4.4.1	EFUST (USIM Service Table)	69

4.4.2	EFEPSLOCI (EPS Information)	69

4.4.3	EFPLMNwACT (User Controlled PLMN Selector with Access Technology)	69

4.4.4	EFOPLMNwACT (Operator Controlled PLMN Selector with Access Technology)	70

4.4.5	EFACSGL (Allowed CSG Lists)	70

4.4.6	EFCSGT (CSG Type)	71

4.4.7	EFHNBN (Home (e)NodeB Name)	73

4.4.8	EFEPSNSC (EPS NAS Security Context)	75

4.5	Definition of E-UTRAN/EPC ISIM-UICC	75

4.5.1	Applications on the E-UTRAN/EPC ISIM-UICC	75

4.5.2	Default USIM values on E-UTRAN/EPC ISIM-UICC	75

4.5.3	Default ISIM values on E-UTRAN/EPC ISIM-UICC	75

4.5.3.1	EFAD (Administrative Data)	75

4.5.3.3	EFIMPI (IMS private user identity)	76

4.5.3.4	EFDOMAIN (Home Network Domain Name)	76

4.5.3.5	EFIMPU (IMS public user identity)	76

4.5.3.6	EFP-CSCF (P-CSCF ADDRESS)	77

4.5.3.7	EFSMS (Short Message Service)	77

4.5.3.8	EFSMSR (Short message status reports)	78

4.5.3.9	EFSMSP (Short message service parameters)	78

4.5.3.10	EFSMSS (SMS Status)	78

4.5.4	Default values at DF_TELECOM	78

4.5.4.1	EFPSISMSC (Public Service Identity of the SM-SC)	78

4.6	Definition of ACSGL/OCSGL E-UTRAN/EPC UICC	79

4.6.1	EFUST (USIM Service Table)	79

4.6.2	EFAD (Administrative Data)	79

4.6.3	EFOCSGL (Operator CSG Lists)	79

4.6.4	EFOCSGT (Operator CSG Type)	80

4.6.5	EFOHNBN (Operator Home (e)NodeB Name)	82

4.7	Definition of Non Access Stratum Configuration UICC	83

4.7.1	Values of the EFs	83

4.7.1.2	EFNASCONFIG (Non Access Stratum Configuration)	83

4.8	Definition of Non Access Stratum Configuration of E-UTRAN/EPC UICC	84

4.8.1	EFUST (USIM Service Table)	84

4.8.2	EFEPSLOCI (EPS Information)	84

4.8.3	EFPLMNwACT (User Controlled PLMN Selector with Access Technology)	84

4.8.4	EFOPLMNwACT (Operator Controlled PLMN Selector with Access Technology)	85

4.8.5	EFACSGL (Allowed CSG Lists)	85

4.8.6	EFCSGT (CSG Type)	86

4.8.7	EFHNBN (Home (e)NodeB Name)	88

4.8.8	EFEPSNSC (EPS NAS Security Context)	89

4.8.9	EFNASCONFIG (Non Access Stratum Configuration)	89

4.9	Definition of 5G-NR UICC	90

4.9.0	Introduction	90

4.9.2	EFIMSI (IMSI)	90

4.9.3	EF5GS3GPPLOCI (5GS 3GPP location information)	90

4.9.4	EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)	91

4.9.5	EFRouting_Indicator (Routing Indicator EF)	92

4.9.6	EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)	92

4.9.7	EF5GSN3GPPNSC (5GS non-3GPP Access NAS Security Context)	92

4.9.8	EFPLMNwACT (User Controlled PLMN Selector with Access Technology)	93

4.10.1	EFUST (USIM Service Table)	94

4.10.2	EFIMSI (IMSI)	94

4.10.3	EFSUPI_NAI (SUPI as Network Access Identifier)	94

4.10.4	EF5GS3GPPLOCI (5GS 3GPP location information)	94

4.10.5	EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)	95

4.10.6	EFRouting_Indicator (Routing Indicator EF)	96

4.10.7	EFAD (Administrative Data)	96

4.11	5G-NR UICC – support of Rel-16 features	96

4.11.1	Introduction	96

4.11.2	EFUST (USIM Service Table)	96

4.11.3	EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)	96

4.11.4	EF5GSN3GPPNSC (5GS non-3GPP Access NAS Security Context)	97

4.11.5	EF5GAUTHKEYS (5G Authentication Keys)	98

4.12	Definition of CAG 5G-NR UICC	98

4.12.1	EFUST (USIM Service Table)	98

4.12.2	EFCAG (Pre-configured CAG information list)	99

4.13	Definition of 5G-NR ISIM-UICC	99

4.13.1	Applications on the 5G-NR ISIM-UICC	99

4.13.2	Default ISIM values on 5G-NR ISIM-UICC	99

4.14	Default WLAN AP and UE configuration for non-3GPP access	99

4.14.1	Default NG-SS WLAN AP configuration	99

4.14.1.1	WLAN AP frequency	99

4.14.1.2	WLAN AP default parameters	99

4.14.2	Default WLAN configuration for the UE	100

4.14.2.1	N3IWF Identifier configuration	100

4.14.2.2	Non-3GPP access node selection information	100

4.15	Definition of 5G-NR UICC supporting Rel-17 features	100

4.15.0	Introduction	100

4.15.2	EFMCHPPLMN (Multiplier Coefficient for Higher Priority PLMN search)	100

4.15.3	EFHPLMNwACT (HPLMN selector with Access Technology)	101

4A	Default values for NG-SS and UE configurations	101

4A.1	Default NG-SS and UE configuration for different usage settings	101

4A.2	Default NG-SS configuration for 3GPP access with IMS service	101

4A.2.1	Default NG-SS configuration for IMS Registration support	101

4A.2.2	Default NG-SS configuration for PS voice support	101

4A.3	Default NG-SS procedure for 5G Registration on 3GPP access with IMS service	101

4A.4	Default UE configuration for IMS service	102

5	Subscription related tests	103

5.1	IMSI / TMSI handling	103

5.1.1	UE identification by short IMSI	103

5.1.1.1	Definition and applicability	103

5.1.1.2	Conformance requirement	103

5.1.1.3	Test purpose	103

5.1.1.4	Method of test	103

5.1.1.4.1	Initial conditions	103

5.1.1.4.2	Procedure	103

5.1.1.5	Acceptance criteria	104

5.1.2	UE identification by short IMSI using a 2 digit MNC	104

5.1.2.1	Definition and applicability	104

5.1.2.2	Conformance requirement	104

5.1.2.3	Test purpose	104

5.1.2.4	Method of test	104

5.1.2.4.1	Initial conditions	104

5.1.2.4.2	Procedure	105

5.1.2.5	Acceptance criteria	105

5.1.3	UE identification by "short" TMSI	106

5.1.3.1	Definition and applicability	106

5.1.3.2	Conformance requirement	106

5.1.3.3	Test purpose	106

5.1.3.4	Method of test	106

5.1.3.4.1	Initial conditions	106

5.1.3.4.2	Procedure	107

5.1.3.5	Acceptance criteria	107

5.1.4	UE identification by "long" TMSI	107

5.1.4.1	Definition and applicability	107

5.1.4.2	Conformance requirement	107

5.1.4.3	Test purpose	107

5.1.4.4	Method of test	108

5.1.4.4.1	Initial conditions	108

5.1.4.4.2	Procedure	108

5.1.4.5	Acceptance criteria	108

5.1.5	UE identification by long IMSI, TMSI updating and key set identifier assignment	109

5.1.5.1	Definition and applicability	109

5.1.5.2	Conformance requirement	109

5.1.5.3	Test purpose	109

5.1.5.4	Method of test	110

5.1.5.4.1	Initial conditions	110

5.1.5.4.2	Procedure	110

5.1.5.5	Acceptance criteria	111

5.1.6	UE identification by short IMSI when accessing E-UTRAN/EPC	112

5.1.6.1	Definition and applicability	112

5.1.6.2	Conformance requirement	112

5.1.6.3	Test purpose	112

5.1.6.4	Method of test	112

5.1.6.4.1	Initial conditions	112

5.1.6.4.2	Procedure	112

5.1.6.5	Acceptance criteria	113

5.1.7	UE identification by short IMSI using a 2 digit MNC when accessing E-UTRAN/EPC	113

5.1.7.1	Definition and applicability	113

5.1.7.2	Conformance requirement	113

5.1.7.3	Test purpose	113

5.1.7.4	Method of test	113

5.1.7.4.1	Initial conditions	113

5.1.7.4.2	Procedure	114

5.1.7.5	Acceptance criteria	114

5.1.8	UE identification after changed IMSI with service "EMM Information" not available	114

5.1.8.1	Definition and applicability	114

5.1.8.2	Conformance requirement	115

5.1.8.3	Test purpose	115

5.1.8.4	Method of test	115

5.1.8.4.1	Initial conditions	115

5.1.8.4.2	Procedure	115

5.1.8.5	Acceptance criteria	116

5.1.9	UE identification by GUTI when using USIM with service "EMM Information" not available	116

5.1.9.2	Conformance requirement	116

5.1.9.3	Test purpose	116

5.1.9.4	Method of test	116

5.1.9.4.1	Initial conditions	116

5.1.9.4.2	Procedure	117

5.1.9.5	Acceptance criteria	117

5.1.10	UE identification by GUTI when using USIM with service "EMM Information" available	117

5.1.10.1	Definition and applicability	117

5.1.10.2	Conformance requirement	117

5.1.10.3	Test purpose	118

5.1.10.4	Method of test	118

5.1.10.4.1	Initial conditions	118

5.1.10.4.2	Procedure	118

5.1.10.5	Acceptance criteria	118

5.2	Access Control handling	119

5.2.1	Access Control information handling	119

5.2.1.1	Definition and applicability	119

5.2.1.2	Conformance requirement	119

5.2.1.3	Test purpose	120

5.2.1.4	Method of test	120

5.2.1.4.1	Initial conditions	120

5.2.1.4.2	Coding details	120

5.2.1.4.3	Procedure	121

5.2.1.5	Acceptance criteria	121

5.2.2	Access Control information handling for E-UTRAN/EPC	133

5.2.2.1	Definition and applicability	133

5.2.2.2	Conformance requirement	133

5.2.2.3	Test purpose	133

5.2.2.4	Method of test	134

5.2.2.4.1	Initial conditions	134

5.2.2.4.2	Coding details	134

5.2.2.4.3	Procedure	134

5.2.2.5	Acceptance criteria	134

5.2.3	Access Control information handling for NB-IoT	145

5.2.3.1	Definition and applicability	145

5.2.3.2	Conformance requirement	145

5.2.3.3	Test purpose	145

5.2.3.4	Method of test	146

5.2.3.4.1	Initial conditions	146

5.2.3.4.2	Coding details	146

5.2.3.4.3	Procedure	146

5.2.3.5	Acceptance criteria	146

5.3	Handling subscription identifier privacy for 5G	158

5.3.1	SUCI calculation by ME using null scheme	158

5.3.1.1	Definition and applicability	158

5.3.1.2	Conformance requirement	158

5.3.1.3	Test purpose	158

5.3.1.4	Method of test	159

5.3.1.4.1	Initial conditions	159

5.3.1.4.2	Procedure	160

5.3.1.5	Acceptance criteria	160

5.3.2	SUCI calculation by ME using Profile B	161

5.3.2.1	Definition and applicability	161

5.3.2.2	Conformance requirement	161

5.3.2.3	Test purpose	161

5.3.2.4	Method of test	161

5.3.2.4.1	Initial conditions	161

5.3.2.4.2	Procedure	162

5.3.2.5	Acceptance criteria	162

5.3.3	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B	162

5.3.3.1	Definition and applicability	162

5.3.3.2	Conformance requirement	162

5.3.3.3	Test purpose	162

5.3.3.4	Method of test	163

5.3.3.4.1	Initial conditions	163

5.3.3.4.2	Procedure	164

5.3.3.5	Acceptance criteria	164

5.3.4	UE identification by SUCI in response to IDENTITY REQUEST message	164

5.3.4.1	Definition and applicability	164

5.3.4.2	Conformance requirement	165

5.3.4.3	Test purpose	165

5.3.4.4	Method of test	165

5.3.4.4.1	Initial conditions	165

5.3.4.4.2	Procedure	166

5.3.4.5	Acceptance criteria	166

5.3.5	UE identification by SUCI in response to IDENTITY REQUEST message with T3519 timer expiry	167

5.3.5.1	Definition and applicability	167

5.3.5.2	Conformance requirement	167

5.3.5.3	Test purpose	167

5.3.5.4	Method of test	168

5.3.5.4.1	Initial conditions	168

5.3.5.4.2	Procedure	168

5.3.5.5	Acceptance criteria	169

5.3.6	UE identification by SUCI in response to IDENTITY REQUEST message and AUTHENTICATION REJECT	169

5.3.6.1	Definition and applicability	169

5.3.6.2	Conformance requirement	169

5.3.6.3	Test purpose	170

5.3.6.4	Method of test	170

5.3.6.4.1	Initial conditions	170

5.3.6.4.2	Procedure	170

5.3.6.5	Acceptance criteria	171

5.3.7	SUCI calculation by the ME using null scheme – missing parameters for subscription identifier privacy support by the USIM	171

5.3.7.1	Definition and applicability	171

5.3.7.2	Conformance requirement	171

5.3.7.3	Test purpose	172

5.3.7.4	Method of test	172

5.3.7.4.1	Initial conditions	172

5.3.7.4.2	Procedure	172

5.3.7.5	Acceptance criteria	172

5.3.8	UE identification by 5G-GUTI – Last Registered TAI stored on USIM	173

5.3.8.1	Definition and applicability	173

5.3.8.2	Conformance requirement	173

5.3.8.3	Test purpose	173

5.3.8.4	Method of test	173

5.3.8.4.2	Procedure	174

5.3.8.5	Acceptance criteria	174

5.3.9	UE identification by 5G-GUTI – Last Registered TAI stored by ME	174

5.3.9.1	Definition and applicability	174

5.3.9.3	Test purpose	174

5.3.9.4	Method of test	175

5.3.9.4.1	Initial conditions	175

5.3.9.4.2	Procedure	175

5.3.9.5	Acceptance criteria	175

5.3.10	UE identification after SUPI is changed	176

5.3.10.1	Definition and applicability	176

5.3.10.2	Conformance requirement	176

5.3.10.3	Test purpose	176

5.3.10.4	Method of test	176

5.3.10.4.1	Initial conditions	176

5.3.10.4.2	Procedure	176

5.3.10.5	Acceptance criteria	177

5.3.11	SUCI calculation by ME using Profile A	177

5.3.11.1	Definition and applicability	177

5.3.11.2	Conformance requirement	177

5.3.11.3	Test purpose	177

5.3.11.4	Method of test	177

5.3.11.4.1	Initial conditions	178

5.3.11.4.2	Procedure	178

5.3.11.5	Acceptance criteria	179

5.3.12	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile A	179

5.3.12.1	Definition and applicability	179

5.3.12.2	Conformance requirement	179

5.3.12.3	Test purpose	179

5.3.12.4	Method of test	180

5.3.12.4.1	Initial conditions	180

5.3.12.4.2	Procedure	181

5.3.12.5	Acceptance criteria	181

5.3.13	SUCI calculation by ME using null scheme– no Protection Scheme Identifier provisioned in the USIM	181

5.3.13.1	Definition and applicability	181

5.3.13.2	Conformance requirement	181

5.3.13.3	Test purpose	182

5.3.13.4	Method of test	182

5.3.13.4.1	Initial conditions	182

5.3.13.4.2	Procedure	182

5.3.13.5	Acceptance criteria	182

5.3.14	SUCI calculation by ME using null scheme – no Home Network Public Key for supported protection scheme provisioned in the USIM	183

5.3.14.1	Definition and applicability	183

5.3.14.2	Conformance requirement	183

5.3.14.3	Test purpose	183

5.3.14.4	Method of test	183

5.3.14.4.1	Initial conditions	183

5.3.14.4.2	Procedure	184

5.3.14.5	Acceptance criteria	184

5.3.15	SUCI calculation by ME using null scheme with the E-UTRAN/EPC UICC	184

5.3.15.1	Definition and applicability	184

5.3.15.2	Conformance requirement	184

5.3.15.3	Test purpose	184

5.3.15.4	Method of test	185

5.3.15.4.1	Initial conditions	185

5.3.15.4.2	Procedure	185

5.3.15.5	Acceptance criteria	185

5.3.16	SUCI calculation by ME using the lower priority protection scheme when the higher priority protection scheme is not supported by the ME	186

5.3.16.1	Definition and applicability	186

5.3.16.2	Conformance requirement	186

5.3.16.3	Test purpose	186

5.3.16.4	Method of test	186

5.3.16.4.1	Initial conditions	186

5.3.16.4.2	Procedure	187

5.3.16.5	Acceptance criteria	188

5.3.17	SUCI calculation by ME using Profile B with compressed Home Network Public Key	188

5.3.17.1	Definition and applicability	188

5.3.17.2	Conformance requirement	188

5.3.17.3	Test purpose	189

5.3.17.4	Method of test	189

5.3.17.4.1	Initial conditions	189

5.3.17.4.2	Procedure	190

5.3.17.5	Acceptance criteria	190

5.4	Unified Access Control information handling for 5G-NR	191

5.4.1	Unified Access Control – Access identity 0, no access identities indicated by USIM	191

5.4.1.1	Definition and applicability	191

5.4.1.2	Conformance requirement	191

5.4.1.3	Test purpose	191

5.4.1.4	Method of test	192

5.4.1.4.1	Initial conditions	192

5.4.1.4.2	Procedure	192

5.4.1.4.3	Acceptance criteria	193

5.4.1A	Unified Access Control – Access identity 0, no access identities indicated by USIM, Access Category 8	193

5.4.1A.1	Definition and applicability	193

5.4.1A.2	Conformance requirement	194

5.4.1A.3	Test purpose	194

5.4.1A.4	Method of test	194

5.4.1A.4.1	Initial conditions	194

5.4.1.4.2	Procedure	196

5.4.1A.4.3	Acceptance criteria	196

5.4.2	Unified Access Control – Access Identity 1 – MPS indicated by USIM	197

5.4.2.1	Definition and applicability	197

5.4.2.2	Conformance requirement	197

5.4.2.3	Test purpose	197

5.4.2.4	Method of test	198

5.4.2.4.1	Initial conditions	198

5.4.2.4.2	Procedure	198

5.4.2.4.3	Acceptance criteria	199

5.4.2A	Unified Access Control – Access Identity 1 – MPS indicated by USIM, Access Category 8	200

5.4.2A.1	Definition and applicability	200

5.4.2A.2	Conformance requirement	200

5.4.2A.3	Test purpose	201

5.4.2A.4	Method of test	201

5.4.2A.4.1	Initial conditions	201

5.4.2A.4.2	Procedure	202

5.4.2A.4.3	Acceptance criteria	202

5.4.3	Unified Access Control – Access Identity 1 – no MPS indication by USIM and SUPI not changed	203

5.4.3.1	Definition and applicability	203

5.4.3.2	Conformance requirement	203

5.4.3.3	Test purpose	204

5.4.3.4	Method of test	204

5.4.3.4.1	Initial conditions	204

5.4.3.4.2	Procedure	206

5.4.3.4.3	Acceptance criteria	206

5.4.4	Unified Access Control – Access Identity 1 – no MPS indication by USIM and SUPI is changed	207

5.4.4.1	Definition and applicability	207

5.4.4.2	Conformance requirement	207

5.4.4.3	Test purpose	208

5.4.4.4	Method of test	208

5.4.4.4.1	Initial conditions	208

5.4.4.4.2	Procedure	209

5.4.4.4.3	Acceptance criteria	210

5.4.5	Unified Access Control – Access Identity 2 – MCS indicated by USIM	210

5.4.5.1	Definition and applicability	210

5.4.5.2	Conformance requirement	211

5.4.5.3	Test purpose	211

5.4.5.4	Method of test	211

5.4.5.4.1	Initial conditions	211

5.4.5.4.2	Procedure	212

5.4.5.4.3	Acceptance criteria	212

5.4.5A	Unified Access Control – Access Identity 2 – MCS indicated by USIM	213

5.4.5A.1	Definition and applicability	213

5.4.5A.2	Conformance requirement	214

5.4.5A.3	Test purpose	214

5.4.5A.4	Method of test	215

5.4.5A.4.1	Initial conditions	215

5.4.5A.4.2	Procedure	216

5.4.5A.4.3	Acceptance criteria	216

5.4.6	Unified Access Control – Access Identity 2 – no MCS indication by USIM and SUPI is not changed	217

5.4.6.1	Definition and applicability	217

5.4.6.2	Conformance requirement	217

5.4.6.3	Test purpose	218

5.4.6.4	Method of test	218

5.4.6.4.1	Initial conditions	218

5.4.6.4.2	Procedure	219

5.4.6.4.3	Acceptance criteria	220

5.4.7	Unified Access Control – Access Identity 2 – no MCS indication by USIM and SUPI is changed	220

5.4.7.1	Definition and applicability	220

5.4.7.2	Conformance requirement	221

5.4.7.3	Test purpose	222

5.4.7.4	Method of test	222

5.4.7.4.1	Initial conditions	222

5.4.7.4.2	Procedure	223

5.4.7.4.3	Acceptance criteria	224

5.4.8	Unified Access Control – Access Identities 11 and 15 indicated by USIM	224

5.4.8.1	Definition and applicability	224

5.4.8.2	Conformance requirement	225

5.4.8.3	Test purpose	225

5.4.8.4	Method of test	225

5.4.8.4.1	Initial conditions	225

5.4.8.4.2	Procedure	226

5.4.8.4.3	Acceptance criteria	226

5.4.8A	Unified Access Control – Access Identities 11 and 15 indicated by USIM, Access Category 8	227

5.4.8A.1	Definition and applicability	227

5.4.8A.2	Conformance requirement	228

5.4.8A.3	Test purpose	228

5.4.8A.4	Method of test	228

5.4.8A.4.1	Initial conditions	228

5.4.8A.4.2	Procedure	229

5.4.8.4.3	Acceptance criteria	230

5.4.9	Unified Access Control – Access Identities 12, 13 and 14 indicated by USIM	231

5.4.9.1	Definition and applicability	231

5.4.9.2	Conformance requirement	231

5.4.9.3	Test purpose	231

5.4.9.4	Method of test	232

5.4.9.4.1	Initial conditions	232

5.4.9.4.2	Procedure	232

5.4.9.4.3	Acceptance criteria	233

5.4.9A	Unified Access Control – Access Identities 12, 13 and 14 indicated by USIM, Access Category 8	234

5.4.9A.1	Definition and applicability	234

5.4.9A.2	Conformance requirement	234

5.4.9A.3	Test purpose	235

5.4.9A.4	Method of test	235

5.4.9A.4.1	Initial conditions	235

5.4.9A.4.2	Procedure	236

5.4.9A.4.3	Acceptance criteria	237

5.4.10	Unified Access Control – Operator-Defined Access Category	237

5.4.10.1	Definition and applicability	237

5.4.10.2	Conformance requirement	238

5.4.10.3	Test purpose	239

5.4.10.4	Method of test	239

5.4.10.4.1	Initial conditions	239

5.4.10.4.2	Procedure	241

5.4.10.5	Acceptance criteria	241

5.4.11	Unified Access Control – Operator-Defined Access Categories, no change in SUPI	242

5.4.11.1	Definition and applicability	242

5.4.11.2	Conformance requirement	243

5.4.11.3	Test purpose	244

5.4.11.4	Method of test	244

5.4.11.4.1	Initial conditions	244

5.4.11.4.2	Procedure	246

5.4.11.4.3	Acceptance criteria	247

5.4.12	Unified Access Control – Operator-Defined Access Categories, SUPI change	247

5.4.12.1	Definition and applicability	247

5.4.12.2	Conformance requirement	248

5.4.12.3	Test purpose	249

5.4.12.4	Method of test	249

5.4.12.4.1	Initial conditions	249

5.4.12.4.2	Procedure	251

5.4.12.4.3	Acceptance criteria	252

5.5	Handling of operator controlled features	253

5.5.1	Display of registered 5G PLMN name from USIM	253

5.5.1.1	Definition and applicability	253

5.5.1.2	Conformance requirement	253

5.5.1.3	Test purpose	253

5.5.1.4	Method of tests	253

5.5.1.4.1	Initial conditions	253

5.5.1.4.2	Procedure	255

5.5.1.4.3	Acceptance criteria	256

5.5.2	Display of registered 5G PLMN name from ME	256

5.5.2.1	Definition and applicability	256

5.5.2.2	Conformance requirement	256

5.5.2.3	Test purpose	256

5.5.2.4	Method of tests	256

5.5.2.4.1	Initial conditions	256

5.5.2.4.2	Procedure	258

5.5.2.4.3	Acceptance criteria	259

5.6	Handling subscription identifier privacy for 5G - SUPI type in NAI format	259

5.6.1	SUCI calculation by ME using null scheme	259

5.6.1.1	Definition and applicability	259

5.6.1.2	Conformance requirement	259

5.6.1.3	Test purpose	260

5.6.1.4	Method of test	260

5.6.1.4.1	Initial conditions	260

5.6.1.4.2	Procedure	261

5.6.1.5	Acceptance criteria	261

5.6.2	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile A	262

5.6.2.1	Definition and applicability	262

5.6.2.2	Conformance requirement	262

5.6.2.3	Test purpose	262

5.6.2.4	Method of test	263

5.6.2.4.1	Initial conditions	263

5.6.2.4.2	Procedure	264

5.6.2.5	Acceptance criteria	264

5.6.3	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B	264

5.6.3.1	Definition and applicability	264

5.6.3.2	Conformance requirement	265

5.6.3.3	Test purpose	265

5.6.3.4	Method of test	265

5.6.3.4.1	Initial conditions	265

5.6.3.4.2	Procedure	267

5.6.3.5	Acceptance criteria	267

5.6.4	UE identification after SUPI is changed	267

5.6.4.1	Definition and applicability	267

5.6.4.2	Conformance requirement	268

5.6.4.3	Test purpose	268

5.6.4.4	Method of test	268

5.6.4.4.1	Initial conditions	268

5.6.4.4.2	Procedure	269

5.6.4.5	Acceptance criteria	269

5.6.5	UE identification by SUCI during initial registration – SUCI calculation by ME using profile A	269

5.6.5.1	Definition and applicability	269

5.6.5.2	Conformance requirement	270

5.6.5.3	Test purpose	270

5.6.5.4	Method of test	270

5.6.5.4.1	Initial conditions	270

5.6.5.4.2	Procedure	271

5.6.5.5	Acceptance criteria	272

5.6.6	UE identification by SUCI during initial registration – SUCI calculation by USIM using profile B	272

5.6.6.1	Definition and applicability	272

5.6.6.2	Conformance requirement	272

5.6.6.3	Test purpose	273

5.6.6.4	Method of test	273

5.6.6.4.1	Initial conditions	273

5.6.6.4.2	Procedure	274

5.6.6.5	Acceptance criteria	274

6	Security related Tests	275

6.1	PIN handling	275

6.1.1	Entry of PIN	275

6.1.1.1	Definition and applicability	275

6.1.1.2	Conformance requirement	275

6.1.1.3	Test purpose	275

6.1.1.4	Method of test	275

6.1.1.4.1	Initial conditions	275

6.1.1.4.2	Procedure	275

6.1.1.5	Acceptance criteria	275

6.1.2	Change of PIN	276

6.1.2.1	Definition and applicability	276

6.1.2.2	Conformance requirement	276

6.1.2.3	Test purpose	276

6.1.2.4	Method of test	276

6.1.2.4.1	Initial conditions	276

6.1.2.4.2	Procedure	276

6.1.2.5	Acceptance criteria	276

6.1.3	Unblock PIN	277

6.1.3.1	Definition and applicability	277

6.1.3.2	Conformance requirement	277

6.1.3.3	Test purpose	277

6.1.3.4	Method of test	277

6.1.3.4.1	Initial conditions	277

6.1.3.4.2	Procedure	277

6.1.3.5	Acceptance criteria	278

6.1.4	Entry of PIN2	278

6.1.4.1	Definition and applicability	278

6.1.4.2	Conformance requirement	278

6.1.4.3	Test purpose	278

6.1.4.4	Method of test	278

6.1.4.4.1	Initial conditions	278

6.1.4.4.2	Procedure	279

6.1.4.5	Acceptance criteria	279

6.1.5	Change of PIN2	279

6.1.5.1	Definition and applicability	279

6.1.5.2	Conformance requirement	279

6.1.5.3	Test purpose	279

6.1.5.4	Method of test	279

6.1.5.4.1	Initial conditions	279

6.1.5.4.2	Procedure	279

6.1.5.5	Acceptance criteria	280

6.1.6	Unblock PIN2	280

6.1.6.1	Definition and applicability	280

6.1.6.2	Conformance requirement	280

6.1.6.3	Test purpose	280

6.1.6.4	Method of test	280

6.1.6.4.1	Initial conditions	280

6.1.6.4.2	Procedure	280

6.1.6.5	Acceptance criterias	281

6.1.7	Replacement of PIN	282

6.1.7.1	Definition and applicability	282

6.1.7.2	Conformance requirement	282

6.1.7.3	Test purpose	282

6.1.7.4	Method of test	282

6.1.7.4.1	Initial conditions	282

6.1.7.4.2	Procedure	282

6.1.7.5	Acceptance criteria	282

6.1.8	Change of Universal PIN	283

6.1.8.1	Definition and applicability	283

6.1.8.2	Conformance requirement	283

6.1.8.3	Test purpose	283

6.1.8.4	Method of test	283

6.1.8.4.1	Initial conditions	283

6.1.8.4.2	Procedure	283

6.1.8.5	Acceptance criteria	284

6.1.9	Unblock Universal PIN	284

6.1.9.1	Definition and applicability	284

6.1.9.2	Conformance requirement	284

6.1.9.3	Test purpose	284

6.1.9.4	Method of test	284

6.1.9.4.1	Initial conditions	284

6.1.9.4.2	Procedure	284

6.1.9.5	Acceptance criteria	285

6.1.10	Entry of PIN on multi-verification capable UICCs	285

6.1.10.1	Definition and applicability	285

6.1.10.2	Conformance requirement	285

6.1.10.4	Method of test	285

6.1.10.4.1	Initial conditions	285

6.1.10.4.2	Procedure	286

6.1.10.5	Acceptance criteria	286

6.1.11	Change of PIN on multi-verification capable UICCs	286

6.1.11.1	Definition and applicability	286

6.1.11.2	Conformance requirement	286

6.1.11.3	Test purpose	287

6.1.11.4	Method of test	287

6.1.11.4.1	Initial conditions	287

6.1.11.4.2	Procedure	287

6.1.11.5	Acceptance criteria	288

6.1.12	Unblock PIN on multi-verification capable UICCs	288

6.1.12.1	Definition and applicability	288

6.1.12.2	Conformance requirement	288

6.1.12.3	Test purpose	288

6.1.12.4	Method of test	288

6.1.12.4.1	Initial conditions	288

6.1.12.4.2	Procedure	289

6.1.12.5	Acceptance criteria	290

6.1.13	Entry of PIN2 on multi-verification capable UICCs	290

6.1.13.1	Definition and applicability	290

6.1.13.2	Conformance requirement	290

6.1.13.3	Test purpose	290

6.1.13.4	Method of test	290

6.1.13.4.1	Initial conditions	290

6.1.13.4.2	Procedure	291

6.1.13.5	Acceptance criteria	291

6.1.14	Change of PIN2 on multi-verification capable UICCs	292

6.1.14.1	Definition and applicability	292

6.1.14.2	Conformance requirement	292

6.1.14.3	Test purpose	292

6.1.14.4	Method of test	292

6.1.14.4.1	Initial conditions	292

6.1.14.4.2	Procedure	293

6.1.14.5	Acceptance criteria	293

6.1.15	Unblock PIN2 on multi-verification capable UICCs	293

6.1.15.1	Definition and applicability	293

6.1.15.2	Conformance requirement	293

6.1.15.3	Test purpose	294

6.1.15.4	Method of test	294

6.1.15.4.1	Initial conditions	294

6.1.15.4.2	Procedure	295

6.1.15.5	Acceptance criterias	295

6.1.16	Replacement of PIN with key reference "07"	296

6.1.16.1	Definition and applicability	296

6.1.16.2	Conformance requirement	296

6.1.16.3	Test purpose	296

6.1.16.4	Method of test	296

6.1.16.4.1	Initial conditions	296

6.1.16.4.2	Procedure	297

6.1.16.5	Acceptance criteria	297

6.2	Fixed Dialling Numbers (FDN) handling	298

6.2.1	Terminal and USIM with FDN enabled, EFADN readable and updateable	298

6.2.1.1	Definition and applicability	298

6.2.1.2	Conformance requirement	298

6.2.1.3	Test purpose	298

6.2.1.4	Method of test	299

6.2.1.4.1	Initial conditions	299

6.2.1.4.2	Procedure	299

6.2.1.5	Acceptance criteria	299

6.2.2	Terminal and USIM with FDN disabled	300

6.2.2.1	Definition and applicability	300

6.2.2.2	Conformance requirement	300

6.2.2.3	Test purpose	300

6.2.2.4	Method of test	300

6.2.2.4.1	Initial conditions	300

6.2.2.4.2	Procedure	301

6.2.2.5	Acceptance criteria	301

6.2.3	Enabling, disabling and updating of FDN	301

6.2.3.1	Definition and applicability	301

6.2.3.2	Conformance requirement	301

6.2.3.3	Test purpose	301

6.2.3.4	Method of test	302

6.2.3.4.1	Initial conditions	302

6.2.3.4.2	Procedure	302

6.2.3.5	Acceptance criteria	302

6.2.4	Terminal and USIM with FDN enabled, EFADN readable and updateable (Rel-4 and onwards)	302

6.2.4.1	Definition and applicability	302

6.2.4.2	Conformance requirement	302

6.2.4.3	Test purpose	303

6.2.4.4	Method of test	303

6.2.4.4.1	Initial conditions	303

6.2.4.4.2	Procedure	303

6.2.4.5	Acceptance criteria	304

6.3	Void	304

6.4	Advice of charge (AoC) handling	304

6.4.1	AoC not supported by USIM	304

6.4.1.1	Definition and applicability	304

6.4.1.2	Conformance requirement	304

6.4.1.3	Test purpose	305

6.4.1.4	Method of test	305

6.4.1.4.1	Initial conditions	305

6.4.1.4.2	Procedure	305

6.4.1.5	Acceptance criteria	306

6.4.2	Maximum frequency of ACM updating	306

6.4.2.1	Definition and applicability	306

6.4.2.2	Conformance requirement	306

6.4.2.3	Test purpose	306

6.4.2.4	Method of test	306

6.4.2.4.1	Initial conditions	306

6.4.2.4.2	Procedure	307

6.4.2.5	Acceptance criteria	309

6.4.3	Call terminated when ACM greater than ACMmax	309

6.4.3.1	Definition and applicability	309

6.4.3.2	Conformance requirement	309

6.4.3.3	Test purpose	309

6.4.3.4	Method of test	310

6.4.3.4.1	Initial conditions	310

6.4.3.4.2	Procedure	310

6.4.3.5	Acceptance criteria	312

6.4.4	Response codes of increase command of ACM	312

6.4.4.1	Definition and applicability	312

6.4.4.2	Conformance requirement	313

6.4.4.3	Test purpose	313

6.4.4.4	Method of test	313

6.4.4.4.1	Initial conditions	313

6.4.4.4.2	Procedure	314

6.4.4.5	Acceptance criteria	316

7	PLMN related tests	317

7.1	FPLMN handling	317

7.1.1	Adding FPLMN to the Forbidden PLMN list	317

7.1.1.1	Definition and applicability	317

7.1.1.2	Conformance requirement	317

7.1.1.3	Test purpose	319

7.1.1.4	Method of test	319

7.1.1.4.1	Initial conditions	319

7.1.1.4.2	Procedure	320

7.1.1.5	Acceptance criteria	323

7.1.2	UE updating forbidden PLMNs	325

7.1.2.1	Definition and applicability	325

7.1.2.2	Conformance requirement	326

7.1.2.3	Test purpose	326

7.1.2.4	Method of test	326

7.1.2.4.1	Initial conditions	326

7.1.2.4.2	Procedure	327

7.1.2.5	Acceptance criteria	328

7.1.3	UE deleting forbidden PLMNs	328

7.1.3.1	Definition and applicability	328

7.1.3.2	Conformance requirement	329

7.1.3.3	Test purpose	329

7.1.3.4	Method of test	330

7.1.3.4.1	Initial conditions	330

7.1.3.4.2	Procedure	330

7.1.3.5	Acceptance criteria	331

7.1.4	Adding FPLMN to the forbidden PLMN list when accessing E-UTRAN	332

7.1.4.1	Definition and applicability	332

7.1.4.2	Conformance requirement	333

7.1.4.3	Test purpose	333

7.1.4.4	Method of test	333

7.1.4.4.1	Initial conditions	333

7.1.4.4.2	Procedure	334

7.1.4.5	Acceptance criteria	335

7.1.5	UE updating forbidden PLMNs when accessing E-UTRAN	335

7.1.5.1	Definition and applicability	335

7.1.5.2	Conformance requirement	335

7.1.5.3	Test purpose	336

7.1.5.4	Method of test	336

7.1.5.4.1	Initial conditions	336

7.1.5.4.2	Procedure	336

7.1.5.5	Acceptance criteria	336

7.1.6	UE deleting forbidden PLMNs when accessing E-UTRAN	337

7.1.6.1	Definition and applicability	337

7.1.6.2	Conformance requirement	337

7.1.6.3	Test purpose	337

7.1.6.4	Method of test	338

7.1.6.4.1	Initial conditions	338

7.1.6.4.2	Procedure	338

7.1.6.5	Acceptance criteria	338

7.1.7	Updating the Forbidden PLMN list  after receiving non-integrity protected reject message – UTRAN	339

7.1.7.1	Definition and applicability	339

7.1.7.2	Conformance requirement	339

7.1.7.3	Test purpose	340

7.1.7.4	Method of test	340

7.1.7.4.1	Initial conditions	340

7.1.7.4.2	Procedure	340

7.1.7.5	Acceptance criteria	341

7.1.8	Updating the Forbidden PLMN list after receiving non-integrity protected reject message – E-UTRAN	341

7.1.8.1	Definition and applicability	341

7.1.8.2	Conformance requirement	341

7.1.8.3	Test purpose	342

7.1.8.4	Method of test	342

7.1.8.4.1	Initial conditions	342

7.1.8.4.2	Procedure	342

7.1.8.5	Acceptance criteria	343

7.1.9	Adding FPLMN to the forbidden PLMN list when accessing satellite NG-RAN	343

7.1.9.1	Definition and applicability	343

7.1.9.2	Conformance requirement	344

7.1.9.3	Test purpose	344

7.1.9.4	Method of test	344

7.1.9.4.1	Initial conditions	344

7.1.9.4.2	Procedure	345

7.1.9.5	Acceptance criteria	345

7.1.10	UE updating forbidden PLMNs when accessing satellite NG-RAN	346

7.1.10.1	Definition and applicability	346

7.1.10.2	Conformance requirement	346

7.1.10.3	Test purpose	346

7.1.10.4	Method of test	347

7.1.10.4.1	Initial conditions	347

7.1.10.4.2	Procedure	347

7.1.10.5	Acceptance criteria	347

7.1.11	UE deleting forbidden PLMNs when accessing satellite NG-RAN in manual mode	348

7.1.11.1	Definition and applicability	348

7.1.11.2	Conformance requirement	348

7.1.11.3	Test purpose	348

7.1.11.4	Method of test	348

7.1.11.4.1	Initial conditions	348

7.1.11.4.2	Procedure	349

7.1.11.5	Acceptance criteria	349

7.2	User controlled PLMN selector handling	350

7.2.1	UE updating the User controlled PLMN selector list	350

7.2.1.1	Definition and applicability	350

7.2.1.2	Conformance requirement	350

7.2.1.3	Test purpose	350

7.2.1.4	Method of test	350

7.2.1.4.1	Initial conditions	350

7.2.1.4.2	Procedure	350

7.2.1.5	Acceptance criteria	350

7.2.2	UE recognising the priority order of the User controlled PLMN selector list with the same access technology.	351

7.2.2.1	Definition and applicability	351

7.2.2.2	Conformance requirement	351

7.2.2.3	Test purpose	352

7.2.2.4	Method of test	352

7.2.2.4.1	Initial conditions	352

7.2.2.4.2	Procedure	353

7.2.2.5	Acceptance criteria	354

7.2.3	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference.	355

7.2.3.1	Definition and applicability	355

7.2.3.2	Conformance requirement	355

7.2.3.3	Test purpose	355

7.2.3.4	Method of test	355

7.2.3.4.1	Initial conditions	355

7.2.3.5	Acceptance criteria	356

7.2.4	Void	357

7.2.5	UE updating the User controlled PLMN selector list for E-UTRAN	357

7.2.5.1	Definition and applicability	357

7.2.5.2	Conformance requirement	357

7.2.5.3	Test purpose	357

7.2.5.4	Method of test	357

7.2.5.4.1	Initial conditions	357

7.2.5.4.2	Procedure	357

7.2.5.5	Acceptance criteria	357

7.2.6	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference- UTRAN/E-UTRAN	358

7.2.6.1	Definition and applicability	358

7.2.6.2	Conformance requirement	358

7.2.6.3	Test purpose	359

7.2.6.4	Method of test	359

7.2.6.4.1	Initial conditions	359

7.2.6.4.2	Procedure	359

7.2.6.5	Acceptance criteria	359

7.2.7	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference- GSM/E-UTRAN	360

7.2.7.1	Definition and applicability	360

7.2.7.2	Conformance requirement	360

7.2.7.3	Test purpose	360

7.2.7.4	Method of test	360

7.2.7.4.1	Initial conditions	360

7.2.7.4.2	Procedure	361

7.2.7.5	Acceptance criteria	361

7.2.8	UE recognising the priority order of the User controlled PLMN selector list with the same access technology – E-UTRAN in NB-S1 mode	362

7.2.8.1	Definition and applicability	362

7.2.8.2	Conformance requirement	362

7.2.8.3	Test purpose	362

7.2.8.4	Method of test	362

7.2.8.4.1	Initial conditions	362

7.2.8.2	Procedure	363

7.2.8.5	Acceptance criteria	363

7.2.9	UE recognising the priority order of the User controlled PLMN selector list using the ACT preference – E-UTRAN in WB-S1/E-UTRAN in NB-S1	364

7.2.9.1	Definition and applicability	364

7.2.9.2	Conformance requirement	364

7.2.9.3	Test purpose	364

7.2.9.4	Method of test	364

7.2.9.4.1	Initial conditions	364

7.2.9.4.2	Procedure	365

7.2.9.5	Acceptance criteria	365

7.2.10	UE updating the User controlled PLMN selector list for satellite-NG-RAN	366

7.2.10.1	Definition and applicability	366

7.2.10.2	Conformance requirement	366

7.2.10.3	Test purpose	366

7.2.10.4	Method of test	366

7.2.10.4.1	Initial conditions	366

7.2.10.4.2	Procedure	366

7.2.10.5	Acceptance criteria	366

7.2.11.1	Definition and applicability	367

7.2.11.2	Conformance requirement	367

7.2.11.3	Test purpose	367

7.2.11.4	Method of test	368

7.2.11.4.1	Initial conditions	368

7.2.11.4.2	Procedure	368

7.2.11.5	Acceptance criteria	368

7.2.12	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference UTRAN/satellite NG-RAN	369

7.2.12.1	Definition and applicability	369

7.2.12.2	Conformance requirement	369

7.2.12.3	Test purpose	369

7.2.12.4	Method of test	369

7.2.12.4.1	Initial conditions	369

7.2.12.4.2	Procedure	369

7.2.12.5	Acceptance criteria	370

7.2.13	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference E-UTRAN/satellite NG-RAN	370

7.2.13.1	Definition and applicability	370

7.2.13.2	Conformance requirement	370

7.2.13.3	Test purpose	371

7.2.13.4	Method of test	371

7.2.13.4.1	Initial conditions	371

7.2.13.4.2	Procedure	371

7.2.13.5	Acceptance criteria	371

7.2.14	UE recognising the priority order of the User controlled PLMN selector list using an ACT preference NG-RAN/satellite NG-RAN	372

7.2.14.1	Definition and applicability	372

7.2.14.2	Conformance requirement	372

7.2.14.3	Test purpose	372

7.2.14.4	Method of test	372

7.2.14.4.1	Initial conditions	372

7.2.14.4.2	Procedure	372

7.2.14.5	Acceptance criteria	373

7.3	Operator controlled PLMN selector handling	373

7.3.1	UE recognising the priority order of the Operator controlled PLMN selector list.	373

7.3.1.1	Definition and applicability	373

7.3.1.2	Conformance requirement	374

7.3.1.3	Test purpose	374

7.3.1.4	Method of test	374

7.3.1.4.1	Initial conditions	374

7.3.1.4.2	Procedure	375

7.3.1.5	Acceptance criteria	376

7.3.2	UE recognising the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list.	377

7.3.2.1	Definition and applicability	377

7.3.2.2	Conformance requirement	377

7.3.2.3	Test purpose	377

7.3.2.4	Method of test	378

7.3.2.4.1	Initial conditions	378

7.3.2.4.2	Procedure	379

7.3.2.5	Acceptance criteria	380

7.3.3	UE recognising the priority order of the Operator controlled PLMN selector list when accessing E-UTRAN	381

7.3.3.1	Definition and applicability	381

7.3.3.2	Conformance requirement	381

7.3.3.3	Test purpose	381

7.3.3.4	Method of test	381

7.3.3.4.1	Initial conditions	381

7.3.3.4.2	Procedure	383

7.3.3.5	Acceptance criteria	383

7.3.4	UE recognising the priority order of the User controlled PLMN selector over the Operator controlled PLMN selector list – E-UTRAN	384

7.3.4.1	Definition and applicability	384

7.3.4.2	Conformance requirement	384

7.3.4.3	Test purpose	384

7.3.4.4	Method of test	384

7.3.4.4.1	Initial conditions	384

7.3.4.4.2	Procedure	385

7.3.4.5	Acceptance criteria	385

7.4	Higher priority PLMN search handling	397

7.4.1	UE recognising the search period of the Higher priority PLMN	397

7.4.1.1	Definition and applicability	397

7.4.1.2	Conformance requirement	397

7.4.1.3	Test purpose	398

7.4.1.4	Method of test	398

7.4.1.4.1	Initial conditions	398

7.4.1.4.2	Procedure	398

7.4.1.5	Acceptance criteria	401

7.4.2	GSM/UmTS dual mode UEs recognising the search period of the Higher priority PLMN	402

7.4.2.1	Definition and applicability	402

7.4.2.2	Conformance requirement	402

7.4.2.3	Test purpose	402

7.4.2.4	Method of test	402

7.4.2.4.1	Initial conditions	402

7.4.2.4.2	Procedure	403

7.4.2.5	Acceptance criteria	405

7.4.3	UE recognising the search period of the Higher priority PLMN – E-UTRAN	405

7.4.3.1	Definition and applicability	405

7.4.3.2	Conformance requirement	406

7.4.3.3	Test purpose	406

7.4.3.4	Method of test	406

7.4.3.4.1	Initial conditions	406

7.4.3.4.2	Procedure	407

7.4.3.5	Acceptance criteria	407

7.4.4	E-UTRAn/EPC capable UEs recognising the search period of the Higher priority PLMN – GSM/E-UTRAN	408

7.4.4.1	Definition and applicability	408

7.4.4.2	Conformance requirement	408

7.4.4.3	Test purpose	408

7.4.4.4	Method of test	408

7.4.4.4.1	Initial conditions	408

7.4.4.4.2	Procedure	409

7.4.4.5	Acceptance criteria	410

7.4.5	E-UTRAn/EPC capable UEs recognising the search period of the Higher priority PLMN – UTRAN/E-UTRAN	410

7.4.5.1	Definition and applicability	410

7.4.5.2	Conformance requirement	410

7.4.5.3	Test purpose	410

7.4.5.4	Method of test	411

7.4.5.4.1	Initial conditions	411

7.4.5.4.2	Procedure	412

7.4.5.5	Acceptance criteria	412

7.4.6	UE recognising Multiplier Coefficient for Higher Priority PLMN search - satellite-NG-RAN	413

7.4.6.1	Definition and applicability	413

7.4.6.2	Conformance requirement	413

7.4.6.3	Test purpose	413

7.4.6.4	Method of test	413

7.4.6.4.1	Initial conditions	413

7.4.6.4.2	Procedure	414

7.4.6.5	Acceptance criteria	415

7.4.7	UE recognising the search period of the Higher priority PLMN – NG-RAN	415

7.4.7.1	Definition and applicability	415

7.4.7.2	Conformance requirement	415

7.4.7.3	Test purpose	416

7.4.7.4	Method of test	416

7.4.7.4.1	Initial conditions	416

7.4.7.4.2	Procedure	416

7.4.7.5	Acceptance criteria	417

7.5	Void	417

8	Subscription independent tests	418

8.1	Phone book procedures	418

8.1.1	Recognition of a previously changed phonebook	418

8.1.1.1	Definition and applicability	418

8.1.1.2	Conformance requirement	418

8.1.1.3	Test purpose	418

8.1.1.4	Method of test	418

8.1.1.4.1	Initial conditions	418

8.1.1.4.2	Procedure	419

8.1.1.5	Acceptance criteria	419

8.1.2	Update of the Phonebook Synchronisation Counter (PSC)	419

8.1.2.1	Definition and applicability	419

8.1.2.2	Conformance requirement	419

8.1.2.3	Test purpose	420

8.1.2.4	Method of test	420

8.1.2.4.1	Initial conditions	420

8.1.2.4.2	Procedure	420

8.1.2.5	Acceptance criteria	421

8.1.3	Phonebook content handling	421

8.1.3.1	Handling of BCD number/ SSC content extension	421

8.1.3.1.1	Definition and applicability	421

8.1.3.1.2	Conformance requirement	421

8.1.3.1.3	Test purpose	421

8.1.3.1.4	Method of test	421

8.1.3.1.5	Acceptance criteria	424

8.1.4	Phonebook selection	425

8.1.4.1	Definition and applicability	425

8.1.4.2	Conformance requirement	425

8.1.4.3	Test purpose	425

8.1.4.4	Method of test	426

8.1.4.4.1	Initial conditions	426

8.1.4.4.2	Procedure	428

8.1.4.5	Acceptance criteria	429

8.1.5	Local Phonebook handling	429

8.1.5.1	Definition and applicability	429

8.1.5.2	Conformance requirement	430

8.1.5.3	Test purpose	430

8.1.5.4	Method of test	430

8.1.5.4.1	Initial conditions	430

8.1.5.4.2	Procedure	431

8.1.5.5	Acceptance criteria	431

8.2	Short message handling report	432

8.2.1	Correct storage of a SM on the USIM	432

8.2.1.1	Definition and applicability	432

8.2.1.2	Conformance requirement	432

8.2.1.3	Test purpose	432

8.2.1.4	Method of test	432

8.2.1.4.1	Initial conditions	432

8.2.1.4.2	Procedure	433

8.2.1.5	Acceptance criteria	433

8.2.2	Correct reading of a SM on the USIM	433

8.2.2.1	Definition and applicability	433

8.2.2.2	Conformance requirement	433

8.2.2.3	Test purpose	433

8.2.2.4	Method of test	434

8.2.2.4.1	Initial conditions	434

8.2.2.4.2	Procedure	435

8.2.2.5	Acceptance criteria	435

8.2.3	SM memory capacity exceeded handling	435

8.2.3.1	Definition and applicability	435

8.2.3.2	Conformance requirement	435

8.2.3.3	Test purpose	435

8.2.3.4	Method of test	436

8.2.3.4.1	Initial conditions	436

8.2.3.4.2	Procedure	438

8.2.3.5	Acceptance criteria	438

8.2.4	Correct storage of an SM on the UICC	440

8.2.4.1	Definition and applicability	440

8.2.4.2	Conformance requirement	440

8.2.4.3	Test purpose	440

8.2.4.4	Method of test	440

8.2.4.4.1	Initial conditions	440

8.2.4.4.2	Procedure	442

8.2.4.5	Acceptance criteria	443

8.2.5	Correct reading of a SM on the USIM if USIM and ISIM are present	444

8.2.5.1	Definition and applicability	444

8.2.5.2	Conformance requirement	444

8.2.5.3	Test purpose	444

8.2.5.4	Method of test	445

8.2.5.4.1	Initial conditions	445

8.2.5.4.2	Procedure	446

8.2.5.5	Acceptance criteria	446

8.2.6	Correct reading of a SM on the ISIM if USIM and ISIM are present	446

8.2.6.1	Definition and applicability	446

8.2.6.2	Conformance requirement	446

8.2.6.3	Test purpose	447

8.2.6.4	Method of test	447

8.2.6.4.1	Initial conditions	447

8.2.6.4.2	Procedure	448

8.2.6.5	Acceptance criteria	448

8.2.7	Correct storage of an SM on the UICC	448

8.2.7.1	Definition and applicability	448

8.2.7.2	Conformance requirement	449

8.2.7.3	Test purpose	449

8.2.7.4	Method of test	449

8.2.7.4.1	Initial conditions	449

8.2.7.4.2	Procedure	449

8.2.7.5	Acceptance criteria	449

8.3	MMS related tests	449

8.3.1	UE recognising the priority order of MMS Issuer Connectivity Parameters	449

8.3.1.1	Definition and applicability	449

8.3.1.2	Conformance requirement	450

8.3.1.3	Test purpose	450

8.3.1.4	Method of test	450

8.3.1.4.1	Initial conditions	450

8.3.1.4.2	Procedure	455

8.3.1.5	Acceptance criteria	455

8.3.2	UE recognising the priority order of MMS User Connectivity Parameters	455

8.3.2.1	Definition and applicability	455

8.3.2.2	Conformance requirement	455

8.3.2.3	Test purpose	456

8.3.2.4	Method of test	456

8.3.2.4.1	Initial conditions	456

8.3.2.4.2	Procedure	461

8.3.2.5	Acceptance criteria	461

8.3.3	UE recognising the priority order of MMS Issuer Connectivity Parameters over the MMS User Connectivity Parameters	461

8.3.3.1	Definition and applicability	461

8.3.3.2	Conformance requirement	462

8.3.3.3	Test purpose	462

8.3.3.4	Method of test	462

8.3.3.4.1	Initial conditions	462

8.3.3.4.2	Procedure	468

8.3.3.5	Acceptance criteria	468

8.3.4	Usage of MMS notification	468

8.3.4.1	Definition and applicability	468

8.3.4.2	Conformance requirement	468

8.3.4.3	Test purpose	469

8.3.4.4	Method of test	469

8.3.4.4.1	Initial conditions	469

8.3.4.4.2	Procedure	473

8.3.4.5	Acceptance criteria	473

8.4	UICC presence detection	474

8.4.1	Definition and applicability	474

8.4.2	Conformance requirement	474

8.4.3	Test purpose	474

8.4.4	Method of test	475

8.4.4.1	Initial conditions	475

8.4.4.2	Procedure	475

8.4.5	Acceptance criteria	475

8.5	UICC presence detection when connected to E-UTRAN/EPC	475

8.5.1	Definition and applicability	475

8.5.2	Conformance requirement	475

8.5.3	Test purpose	476

8.5.4	Method of test	476

8.5.4.1	Initial conditions	476

8.5.4.2	Procedure	476

8.5.5	Acceptance criteria	476

9	USIM service handling	477

9.1	Access Point Name Control List handling	477

9.1.1	Access Point Name Control List handling for terminals supporting ACL	477

9.1.1.1	Definition and applicability	477

9.1.1.2	Conformance requirement	477

9.1.1.3	Test purpose	477

9.1.1.4	Method of test	477

9.1.1.4.1	Initial conditions	477

9.1.1.4.2	Procedure	478

9.1.1.5	Acceptance criteria	478

9.1.2	Network provided APN handling for terminals supporting ACL	479

9.1.2.1	Definition and applicability	479

9.1.2.2	Conformance requirement	479

9.1.2.3	Test purpose	479

9.1.2.4	Method of test	479

9.1.2.4.1	Initial conditions	479

9.1.2.4.2	Procedure	480

9.1.2.5	Acceptance criteria	480

9.1.3	Access Point Name Control List handling for terminals not supporting ACL	481

9.1.3.1	Definition and applicability	481

9.1.3.2	Conformance requirement	481

9.1.3.3	Test purpose	481

9.1.3.4	Method of test	481

9.1.3.4.1	Initial conditions	481

9.1.3.4.2	Procedure	482

9.1.3.5	Acceptance criteria	482

9.1.4	Access Point Name Control List handling for terminals supporting ACL connected to E-UTRAN/EPC	482

9.1.4.1	Definition and applicability	482

9.1.4.2	Conformance requirement	482

9.1.4.3	Test purpose	482

9.1.4.4	Method of test	482

9.1.4.4.1	Initial conditions	482

9.1.4.4.2	Procedure	483

9.1.4.5	Acceptance criteria	484

9.1.5	Void9.1.6 Void	484

9.2	Service Dialling Numbers handling	484

9.2.1	Definition and applicability	484

9.2.2	Conformance requirement	484

9.2.3	Test purpose	484

9.2.4	Method of test	485

9.2.4.1	Initial conditions	485

9.2.4.2	Procedure	487

9.2.5	Acceptance criteria	487

10	CSG list handling	488

10.1	CSG list handling for E-UTRA	488

10.1.1	Automatic CSG selection in E-UTRA with CSG list on USIM, success	488

10.1.1.1	Definition and applicability	488

10.1.1.2	Conformance requirement	488

10.1.1.3	Test purpose	488

10.1.1.4	Method of test	488

10.1.1.4.1	Initial conditions	488

10.1.1.4.2	Procedure	489

10.1.1.5	Acceptance criteria	489

10.1.2	Automatic CSG selection in E-UTRA with CSG list on USIM, removal of CSG ID from the USIM	490

10.1.2.1	Definition and applicability	490

10.1.2.2	Conformance requirement	490

10.1.2.3	Test purpose	490

10.1.2.4	Method of test	490

10.1.2.4.1	Initial conditions	490

10.1.2.4.2	Procedure	491

10.1.2.5	Acceptance criteria	491

10.1.3	Manual CSG selection in E-UTRA with CSG list on USIM, success	492

10.1.3.1	Definition and applicability	492

10.1.3.2	Conformance requirement	492

10.1.3.3	Test purpose	492

10.1.3.4	Method of test	492

10.1.3.4.1	Initial conditions	492

10.1.3.4.2	Procedure	493

10.1.3.5	Acceptance criteria	493

10.1.4	Manual CSG selection in E-UTRA with CSG list on USIM, rejected	495

10.1.4.1	Definition and applicability	495

10.1.4.2	Conformance requirement	495

10.1.4.3	Test purpose	495

10.1.4.4	Method of test	495

10.1.4.4.1	Initial conditions	495

10.1.4.4.2	Procedure	496

10.1.4.5	Acceptance criteria	496

10.1.5	CSG selection in E-UTRA with no CSG list on USIM, no IMSI change	497

10.1.5.1	Definition and applicability	497

10.1.5.2	Conformance requirement	497

10.1.5.3	Test purpose	497

10.1.5.4	Method of test	498

10.1.5.4.1	Initial conditions	498

10.1.5.4.2	Procedure	498

10.1.5.5	Acceptance criteria	499

10.1.6	CSG selection in E-UTRA with no CSG list on USIM, with IMSI change	500

10.1.6.1	Definition and applicability	500

10.1.6.2	Conformance requirement	500

10.1.6.3	Test purpose	500

10.1.6.4	Method of test	500

10.1.6.4.1	Initial conditions	500

10.1.6.4.2	Procedure	501

10.1.6.5	Acceptance criteria	502

10.1.7	Manual CSG selection without display restrictions in E-UTRA with ACSG list and OCSG list on USIM	502

10.1.7.1	Definition and applicability	502

10.1.7.2	Conformance requirement	502

10.1.7.3	Test purpose	503

10.1.7.4	Method of test	503

10.1.7.4.1	Initial conditions	503

10.1.7.4.2	Procedure	503

10.1.7.5	Acceptance criteria	504

10.1.8	Manual CSG selection with display restrictions in E-UTRA with ACSG list and OCSG list on USIM	505

10.1.8.1	Definition and applicability	505

10.1.8.2	Conformance requirement	505

10.1.8.3	Test purpose	506

10.1.8.4	Method of test	506

10.1.8.4.1	Initial conditions	506

10.1.8.4.2	Procedure	507

10.1.8.5	Acceptance criteria	507

10.2	CSG list handling for UTRA	509

10.2.1	Manual CSG selection without display restrictions in UTRA with ACSG list and OCSG list on USIM	509

10.2.1.1	Definition and applicability	509

10.2.1.2	Conformance requirement	509

10.2.1.3	Test purpose	509

10.2.1.4	Method of test	509

10.2.1.4.1	Initial conditions	509

10.2.1.4.2	Procedure	510

10.2.1.5	Acceptance criteria	510

10.2.2	Manual CSG selection with display restrictions in UTRA with ACSG list and OCSG list on USIM	512

10.2.2.1	Definition and applicability	512

10.2.2.2	Conformance requirement	512

10.2.2.3	Test purpose	512

10.2.2.4	Method of test	512

10.2.2.4.1	Initial conditions	512

10.2.2.4.2	Procedure	513

10.2.2.5	Acceptance criteria	514

10.2.3	Manual CSG selection in UTRA with CSG list on USIM, success	515

10.2.3.1	Definition and applicability	515

10.2.3.2	Conformance requirement	515

10.2.3.3	Test purpose	516

10.2.3.4	Method of test	516

10.2.3.4.1	Initial conditions	516

10.2.3.4.2	Procedure	516

10.2.3.5	Acceptance criteria	517

11	NAS security context parameter handling	518

11.1	NAS security context parameter handling when service "EMM Information" is available	518

11.1.1	Definition and applicability	518

11.1.2	Conformance requirement	518

11.1.3	Test purpose	519

11.1.4	Method of test	519

11.1.4.1	Initial conditions	519

11.1.4.2	Procedure	519

11.1.5	Acceptance criteria	520

11.2	NAS security context parameter handling when service "EMM Information" is not available, no IMSI change	520

11.2.1	Definition and applicability	520

11.2.2	Conformance requirement	520

11.2.3	Test purpose	521

11.2.4	Method of test	521

11.2.4.1	Initial conditions	521

11.2.4.2	Procedure	521

11.2.5	Acceptance criteria	522

11.3	NAS security context parameter handling when service "EMM Information" is not available, IMSI changed	522

11.3.1	Definition and applicability	522

11.3.2	Conformance requirement	522

11.3.3	Test purpose	523

11.3.4	Method of test	523

11.3.4.1	Initial conditions	523

11.3.4.2	Procedure	523

11.3.5	Acceptance criteria	524

11.4	EPS NAS Security Context Storage	524

11.4.1	Definition and applicability	524

11.4.2	Conformance requirement	524

11.4.3	Test purpose	524

11.4.4	Method of test	525

11.4.4.1	Initial conditions	525

11.4.4.2	Procedure	525

11.4.5	Acceptance criteria	526

12	Non Access Stratum (NAS) Configuration parameter handling	526

12.1	EFNASCONFIG – NAS signaling priority handling	526

12.1.1	Definition and applicability	526

12.1.2	Void	526

12.1.3	Void	526

12.1.4	Void	527

12.1.5	Void	527

12.2	EFNASCONFIG – NMO I Network Mode of Operation I handling	527

12.2.1	Definition and applicability	527

12.2.2	Void	527

12.2.3	Void	527

12.2.4	Void	527

12.2.5	Void	527

12.3	EFNASCONFIG – Attach with IMSI handling	527

12.3.1	Definition and applicability	527

12.3.2	Void	527

12.3.3	Void	527

12.3.4	Void	527

12.3.5	Void	527

12.4	EFNASCONFIG – Verifying Minimum Periodic Search Timer	527

12.4.1	Definition and applicability	527

12.4.2	Void	528

12.4.3	Void	528

12.4.4	Void	528

12.4.5	Void	528

12.5	EFNASCONFIG – Extended access barring handling	528

12.5.1	Definition and applicability	528

12.5.2	Void	528

12.5.3	Void	528

12.5.4	Void	528

12.5.5	Void	528

12.6	EFNASCONFIG – Verifying Timer T3245 Behaviour	528

12.6.1	Definition and applicability	528

12.6.2	Conformance requirement	529

12.6.3	Test purpose	529

12.6.4	Method of test	529

12.6.4.1	Initial conditions	529

12.6.4.2	Procedure	530

12.6.5	Acceptance criteria	530

12.7	EFNASCONFIG – Override NAS signalling low priority	531

12.7.1	Definition and applicability	531

12.7.2	Conformance requirement	531

12.7.3	Test purpose	531

12.7.4	Method of test	531

12.7.4.1	Initial conditions	531

12.7.4.2	Procedure	532

12.7.5	Acceptance criteria	532

12.8	EFNASCONFIG – Override Extended access barring	532

12.8.1	Definition and applicability	532

12.8.2	Conformance requirement	532

12.8.3	Test purpose	533

12.8.4	Method of test	533

12.8.4.1	Initial conditions	533

12.8.4.2	Procedure	533

12.8.5	Acceptance criteria	534

12.9	EFNASCONFIG – Fast First Higher Priority PLMN Search	534

12.9.1	Definition and applicability	534

12.9.2	Conformance requirement	534

12.9.3	Test purpose	534

12.9.4	Method of test	534

12.9.4.1	Initial conditions	534

12.9.4.2	Procedure	535

12.9.5	Acceptance criteria	535

12.10	EFNASCONFIG – E-UTRA Disabling Allowed for EMM cause #15	536

12.10.1	Definition and applicability	536

12.10.2	Conformance requirement	536

12.10.3	Test purpose	536

12.10.4	Method of test	536

12.10.4.1	Initial conditions	536

12.10.4.2	Procedure	537

12.10.5	Acceptance criteria	537

12.11	EFNASCONFIG – SM_RetryWaitTime	538

12.11.1	Definition and applicability	538

12.11.2	Conformance requirement	538

12.11.3	Test purpose	538

12.11.4	Method of test	538

12.11.4.1	Initial conditions	538

12.11.4.2	Procedure	539

12.11.5	Acceptance criteria	539

12.12	EFNASCONFIG – SM_RetryAtRATChange	539

12.12.1	Definition and applicability	539

12.12.2	Conformance requirement	539

12.12.3	Test purpose	539

12.12.4	Method of test	540

12.12.5	Acceptance criteria	540

13	UICC interface during PSM	540

13.1	UICC interface in PSM handling for E-UTRAN – No UICC deactivation in PSM	540

13.1.1	Definition and applicability	540

13.1.2	Conformance requirement	540

13.1.3	Test purpose	540

13.1.4	Method of test	540

13.1.4.1	Initial conditions	540

13.1.4.2	Procedure	541

13.1.5	Acceptance criteria	541

13.2	UICC interface in PSM handling for E-UTRAN – PSM not accepted by E-USS/NB-SS	541

13.2.1	Definition and applicability	541

13.2.2	Conformance requirement	541

13.2.3	Test purpose	542

13.2.4	Method of test	542

13.2.4.1	Initial conditions	542

13.2.4.2	Procedure	542

13.2.5	Acceptance criteria	542

13.3	UICC interface in PSM handling for E-UTRAN – UICC deactivation in PSM	543

13.3.1	Definition and applicability	543

13.3.2	Conformance requirement	543

13.3.3	Test purpose	543

13.3.4	Method of test	543

13.3.4.1	Initial conditions	543

13.3.4.2	Procedure	544

13.3.5	Acceptance criteria	544

13.4	UICC interface in PSM for E-UTRAN – SUSPEND UICC	544

13.4.1	Definition and applicability	544

13.4.2	Conformance requirement	544

13.4.4	Method of test	545

13.4.4.1	Initial conditions	545

13.4.4.2	Procedure	545

13.4.5	Acceptance criteria	546

14	UICC interface during eDRX	546

14.1	UICC interface during eDRX for E-UTRAN – eDRX is not supported by the UICC	546

14.1.1	Definition and applicability	546

14.1.2	Conformance requirement	546

14.1.3	Test purpose	546

14.1.4	Method of test	546

14.1.4.1	Initial conditions	546

14.1.4.2	Procedure	547

14.1.5	Acceptance criteria	547

14.2	UICC interface during eDRX for E-UTRAN – eDRX is not accepted by E-USS/NB-SS	548

14.2.1	Definition and applicability	548

14.2.2	Conformance requirement	548

14.2.3	Test purpose	548

14.2.4	Method of test	548

14.2.4.1	Initial conditions	548

14.2.5	Acceptance criteria	549

14.3	UICC interface during eDRX for E-UTRAN – UICC deactivation during eDRX	549

14.3.1	Definition and applicability	549

14.3.2	Conformance requirement	549

14.3.3	Test purpose	550

14.3.4	Method of test	550

14.3.4.1	Initial conditions	550

14.3.4.2	Procedure	551

14.3.5	Acceptance criteria	551

14.4	UICC interface during eDRX for E-UTRAN– SUSPEND UICC	551

14.4.1	Definition and applicability	551

14.4.2	Conformance requirement	551

14.4.3	Test purpose	552

14.4.4	Method of test	552

14.4.4.1	Initial conditions	552

14.4.4.2	Procedure	553

14.4.5	Acceptance criteria	553

15	Authentication procedure and NAS security context handling for 5G	553

15.1	Authentication procedure for EAP-AKA'	553

15.1.1	Authentication procedure for EAP-AKA' - Authentication is successful	553

15.1.1.1	Definition and applicability	553

15.1.1.2	Conformance requirement	554

15.1.1.3	Test purpose	554

15.1.1.4	Method of test	554

15.1.1.4.1	Initial conditions	554

15.1.1.4.2	Procedure	555

15.1.1.5	Acceptance criteria	555

15.1A.1	Authentication procedure for EAP-AKA' - Authentication is Successful	556

15.1A.1.1	Definition and applicability	556

15.1A.1.2	Conformance requirement	557

15.1A.1.3	Test purpose	557

15.1A.1.4	Method of test	557

15.1A.1.4.1	Initial conditions	557

15.1A.1.4.2	Procedure	557

15.1A.1.5	Acceptance criteria	558

15.1.2	Authentication procedure for EAP-AKA' – Authentication is successful - GSM UICC	559

15.1.2.1	Definition and applicability	559

15.1.2.2	Conformance requirement	559

15.1.2.3	Test purpose	560

15.1.2.4	Method of test	560

15.1.2.4.1	Initial conditions	560

15.1.2.4.2	Procedure	560

15.1.2.5	Acceptance criteria	561

15.1.3	Authentication procedure for EAP-AKA' – AUTN fails on the USIM	561

15.1.3.1	Definition and applicability	561

15.1.3.2	Conformance requirement	561

15.1.3.3	Test purpose	562

15.1.3.4	Method of test	562

15.1.3.4.1	Initial conditions	562

15.1.3.4.2	Procedure	562

15.1.3.5	Acceptance criteria	563

15.1.4	Authentication procedure for EAP-AKA' - after SUPI is changed	563

15.1.4.1	Definition and applicability	563

15.1.4.2	Conformance requirement	563

15.1.4.3	Test purpose	563

15.1.4.4	Method of test	563

15.1.4.4.2	Procedure	564

15.2	Authentication procedure for 5G AKA	565

15.2.1.4	Method of test	566

15.2.1.4.1	Initial conditions	566

15.2A.1.4	Method of test	568

15.2A.1.4.1	Initial conditions	568

15.2A.2	Authentication procedure for 5G AKA – Authentication is successful - GSM UICC	570

15.2A.2.1	Definition and applicability	570

15.2A.2.2	Conformance requirement	570

15.2A.2.3	Test purpose	571

15.2A.2.4	Method of test	571

15.2A.2.4.1	Initial conditions	571

15.2A.2.4.2	Procedure	571

15.2A.2.5	Acceptance criteria	572

15.2.2	Authentication procedure for 5G AKA – Authentication is successful - GSM UICC	572

15.2.2.1	Definition and applicability	572

15.2.2.2	Conformance requirement	572

15.2.2.3	Test purpose	573

15.2.2.4	Method of test	573

15.2.2.4.1	Initial conditions	573

15.2.2.4.2	Procedure	573

15.2.2.5	Acceptance criteria	574

15.2.3	Authentication procedure 5G AKA – AUTN fails on the USIM	574

15.2.3.1	Definition and applicability	574

15.2.3.2	Conformance requirement	574

15.2.3.3	Test purpose	575

15.2.3.4	Method of test	575

15.2.3.4.1	Initial conditions	575

15.2.3.4.2	Procedure	575

15.2.3.5	Acceptance criteria	576

15.2.4	Authentication procedure for 5G AKA - after SUPI is changed	576

15.2.4.1	Definition and applicability	576

15.2.4.2	Conformance requirement	576

15.2.4.3	Test purpose	576

15.2.4.4	Method of test	577

15.2.4.4.1	Initial conditions	577

15.2.4.4.2	Procedure	577

15.2.4.5	Acceptance criteria	578

15.2.5.2	Conformance requirement	578

15.2.5.4	Method of test	579

15.2.5.4.1	Initial conditions	579

15.2.5.4.2	Procedure	580

15.2.5.5	Acceptance criteria	581

15.3	Authentication procedure for 5G AKA over non-3GPP access via N3IWF	584

15.3.1.4	Method of test	585

15.3.1.4.1	Initial conditions	585

15.3.1.4.2	Procedure	585

15.4	5G AKA Authentication procedure for Multiple registration over 3GPP access and non-3GPP access	588

15.4.1.4	Method of test	589

15.4.1.4.1	Initial conditions	589

15.4.1.4.2	Procedure	589

15.4.2.4	Method of test	595

15.4.2.4.1	Initial conditions	595

15.4.2.4.2	Procedure	595

16	UE Route Selection Policy (URSP) procedure	599

16.1	Pre-configured URSP rules	599

16.1.1	Support for URSP by USIM	599

16.1.1.1	Definition and applicability	599

16.1.1.2	Conformance requirement	599

16.1.1.3	Test purpose	600

16.1.1.4	Method of test	600

16.1.1.4.1	Initial conditions	600

16.1.1.4.2	Procedure	602

16.1.1.5	Acceptance criteria	602

16.1.2	Support for URSP by ME	602

16.1.2.1	Definition and applicability	602

16.1.2.2	Conformance requirement	602

16.1.2.3	Test purpose	602

16.1.2.4	Method of test	602

16.1.2.4.1	Initial conditions	602

16.1.2.4.2	Procedure	603

16.1.2.5	Acceptance criteria	603

16.1.3	Support of Signalled URSP	604

16.1.3.1	Definition and applicability	604

16.1.3.2	Conformance requirement	604

16.1.3.3	Test purpose	604

16.1.3.4	Method of test	604

16.1.3.4.1	Initial conditions	604

16.1.3.4.2	Procedure	606

16.1.3.5	Acceptance criteria	606

17	CAG list handling	607

17.1	CAG list handling for 5G	607

17.1.1	Automatic CAG selection with preconfigured CAG list on USIM	607

17.1.1.1	Definition and applicability	607

17.1.1.2	Conformance requirement	607

17.1.1.3	Test purpose	607

17.1.1.4	Method of tests	607

17.1.1.4.1	Initial conditions	607

17.1.1.4.2	Procedure	608

17.1.1.4.3	Acceptance criteria	608

Annex A (informative):	Structure for uac-BarringInfo and Operator-defined access category	609

Annex B (informative):	 5G-NR generic procedure for 5G Registration on 3GPP access with IMS service	614

Annex C (informative):	Change history	615

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document defines the application test specification.

The aim of the present document is to ensure interoperability between an UICC and a terminal independently of the respective manufacturer, card issuer or operator. The present document does not define any aspects related to the administrative management phase of the UICC. Any internal technical realisation of either the UICC or the Terminal is only specified where these are reflected over the interface.

Application specific details for applications residing on an UICC are specified in the respective application specific documents. The logical and physical Characteristics of the UICC Terminal interface is specified in document ETSI TS 102 221 [5]. The Universal Subscriber Identity Module (USIM)-application for 3G telecommunication networks is specified in document TS 31.102 [4].