3GPP TR 37.717-11-21 V17.0.0 (2022-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Networks;

Dual Connectivity (DC) of x bands (x=1, 2, 3, 4) LTE inter-band

CA (x DL/1 UL) and 2 bands NR Inter-band CA (2 DL/1 UL)

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	47

1	Scope	49

2	References	49

3	Definitions, symbols and abbreviations	49

3.1	Definitions	49

3.2	Symbols	50

3.3	Abbreviations	50

4	Background	50

4.1	TR Maintenance	50

5	Dual Connectivity band combinations of LTE x bands DL/1UL(x=1,2,3,4) + NR 2 bands DL/1UL: General Part	51

5.1	UE RF architectures	51

5.2	General treatment of ∆TIB and ∆RIB values	51

5.3	Summary of self-interference analysis	51

6	Dual Connectivity band combinations of LTE 1 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	110

6.1	DC_3_n1-n257, DC_3-3_n1-n257	110

6.1.1	Operating bands for DC	110

6.1.2	Channel bandwidths per operating band for DC	111

6.1.3	Co-existence studies	112

6.1.4	∆TIB and ∆RIB values	112

6.1.5	MSD	112

6.2	DC_7_n1-n257, DC_7-7_n1-n257	112

6.2.1	Operating bands for DC	112

6.2.2	Channel bandwidths per operating band for DC	113

6.2.3	Co-existence studies	113

6.2.4	∆TIB and ∆RIB values	113

6.2.5	MSD	113

6.3	DC_11_n3-n28	114

6.3.1	Operating bands for DC	114

6.3.2	Channel bandwidths per operating band for DC	114

6.3.3	Co-existence studies	114

6.3.4	∆TIB and ∆RIB values	115

6.3.5	MSD	115

6.4	DC_1_n28-n41	116

6.4.1	Operating bands for DC	116

6.4.2	Channel bandwidths per operating band for DC	116

6.4.3	Co-existence studies	116

6.4.4	∆TIB and ∆RIB values	118

6.4.5	MSD	119

6.5	DC_3_n28-n41	120

6.5.1	Operating bands for DC	120

6.5.2	Channel bandwidths per operating band for DC	120

6.5.3	Co-existence studies	120

6.5.4	∆TIB and ∆RIB values	122

6.5.5	MSD	123

6.6	DC_2_n48-n66	124

6.6.1	Operating bands for DC	124

6.6.2	Configuration for DC	125

6.6.3	Co-existence studies	125

6.6.4	∆TIB and ∆RIB values	125

6.6.5	MSD	126

6.7	DC_48_n25-n48	126

6.7.1	Operating bands for DC	126

6.7.2	Configuration for DC	127

6.7.3	Co-existence studies	127

6.7.4	∆TIB and ∆RIB values	127

6.7.5	MSD	127

6.8	DC_48_n48-n66	128

6.8.1	Operating bands for DC	128

6.8.2	Configuration for DC	128

6.8.3	Co-existence studies	128

6.8.4	∆TIB and ∆RIB values	128

6.8.5	MSD	129

6.9	DC_66_n25-n48	129

6.9.1	Operating bands for DC	129

6.9.2	Configuration for DC	129

6.9.3	Co-existence studies	130

6.9.4	∆TIB and ∆RIB values	130

6.9.5	MSD	130

6.10	DC_7_n40-n78	131

6.10.1	Operating bands for DC	131

6.10.2	Configuration for DC	131

6.10.3	Co-existence studies	131

6.10.4	∆TIB and ∆RIB values	131

6.10.5	MSD	132

6.11	DC_8_n40-n78	133

6.11.1	Operating bands for DC	133

6.11.2	Configuration for DC	133

6.11.3	Co-existence studies	133

6.11.4	∆TIB and ∆RIB values	133

6.11.5	MSD	134

6.12	DC_28_n1-n40	135

6.12.1	Operating bands for DC	135

6.12.2	Configuration for DC	135

6.12.3	Co-existence studies	135

6.12.4	∆TIB and ∆RIB values	135

6.12.5	MSD	136

6.13	DC_1_n3-n77	137

6.13.1	Operating bands for DC	137

6.13.2	Channel bandwidths per operating band for DC	138

6.13.3	Co-existence studies	138

6.13.4	∆TIB and ∆RIB values	139

6.13.5	MSD	139

6.14	DC_8_n3-n77	140

6.14.1	Operating bands for DC	140

6.14.2	Channel bandwidths per operating band for DC	141

6.14.3	Co-existence studies	141

6.14.4	∆TIB and ∆RIB values	142

6.14.5	MSD	142

6.15	DC_1_n3-n41	143

6.15.1	Operating bands for DC	143

6.15.2	Channel bandwidths per operating band for DC	144

6.15.3	Co-existence studies	144

6.15.4	∆ TIB and ∆RIB values	144

6.15.5	MSD	145

6.16	DC_1_n41-n77	146

6.16.1	Operating bands for DC	146

6.16.2	Channel bandwidths per operating band for DC	146

6.16.3	Co-existence studies	146

6.16.4	∆TIB and ∆RIB values	147

6.16.5	MSD	147

6.17	DC_3_n3-n41	148

6.17.1	Operating bands for DC	148

6.17.2	Channel bandwidths per operating band for DC	148

6.17.3	Co-existence studies	148

6.17.4	∆TIB and ∆RIB values	149

6.17.5	MSD	149

6.18	DC_3_n41-n77	150

6.18.1	Operating bands for DC	150

6.18.2	Channel bandwidths per operating band for DC	150

6.18.3	Co-existence studies	150

6.18.4	∆TIB and ∆RIB values	151

6.18.5	MSD	151

6.19	DC_41_n3-n41	152

6.19.1	Operating bands for DC	152

6.19.2	Channel bandwidths per operating band for DC	152

6.19.3	Co-existence studies	153

6.19.4	∆TIB and ∆RIB values	153

6.19.5	MSD	154

6.20	DC_41_n41-n77	154

6.20.1	Operating bands for DC	154

6.20.2	Channel bandwidths per operating band for DC	154

6.20.3	Co-existence studies	155

6.20.4	∆TIB and ∆RIB values	155

6.20.5	MSD	155

6.21	DC_41_n41-n78	156

6.21.1	Operating bands for DC	156

6.21.2	Channel bandwidths per operating band for DC	156

6.21.3	Co-existence studies	156

6.21.4	∆TIB and ∆RIB values	157

6.21.5	MSD	157

6.22	DC_2_n7-n66	157

6.22.1	Operating bands for DC	157

6.22.2	Channel bandwidths per operating band for DC	158

6.22.3	Co-existence studies	158

6.22.4	∆TIB and ∆RIB values	158

6.22.5	MSD	159

6.23	DC_2_n38-n66	159

6.23.1	Operating bands for DC	159

6.23.2	Channel bandwidths per operating band for DC	160

6.23.3	Co-existence studies	160

6.23.4	∆TIB and ∆RIB values	160

6.23.5	MSD	161

6.24	DC_2_n66-n78	161

6.24.1	Operating bands for DC	161

6.24.2	Channel bandwidths per operating band for DC	162

6.24.3	Co-existence studies	162

6.24.4	∆TIB and ∆RIB values	162

6.24.5	MSD	163

6.25	DC_12_n7-n66	163

6.25.1	Operating bands for DC	163

6.25.2	Channel bandwidths per operating band for DC	164

6.25.3	Co-existence studies	164

6.25.4	∆TIB and ∆RIB values	164

6.25.5	MSD	165

6.26	DC_66_n38-n66	165

6.26.1	Operating bands for DC	165

6.26.2	Channel bandwidths per operating band for DC	166

6.26.3	Co-existence studies	166

6.26.4	∆TIB and ∆RIB values	166

6.26.5	MSD	167

6.27	DC_19_n1-n77	167

6.27.1	Operating bands for DC	167

6.27.2	Configuration for DC	168

6.27.3	Co-existence studies	168

6.27.4	∆TIB and ∆RIB values	168

6.27.5	MSD	169

6.28	DC_19_n1-n78	169

6.28.1	Operating bands for DC	169

6.28.2	Configuration for DC	170

6.28.3	Co-existence studies	170

6.28.4	∆TIB and ∆RIB values	170

6.28.5	MSD	170

6.29	DC_11_n3-n77	171

6.29.1	Operating bands for DC	171

6.29.2	Channel bandwidths per operating band for DC	172

6.29.3	Co-existence studies	172

6.29.4	∆TIB and ∆RIB values	173

6.29.5	MSD	173

6.30	DC_11_n28-n77	175

6.30.1	Operating bands for DC	175

6.30.2	Channel bandwidths per operating band for DC	176

6.30.3	Co-existence studies	176

6.30.4	∆TIB and ∆RIB values	177

6.30.5	MSD	177

6.31	DC_42_n3-n28	178

6.31.1	Operating bands for DC	178

6.31.2	Channel bandwidths per operating band for DC	179

6.31.3	Co-existence studies	179

6.31.4	∆TIB and ∆RIB values	180

6.31.5	MSD	180

6.32	DC_42_n3-n77	180

6.32.1	Operating bands for DC	180

6.32.2	Channel bandwidths per operating band for DC	181

6.32.3	Co-existence studies	183

6.32.4	∆TIB and ∆RIB values	183

6.32.5	MSD	183

6.33	DC_41_n28-n41	184

6.33.1	Operating bands for DC	184

6.33.2	Channel bandwidths per operating band for DC	184

6.33.3	Co-existence studies	184

6.33.4	∆TIB and ∆RIB values	187

6.33.5	MSD	187

6.34	DC_18_n28-n41	188

6.34.1	Operating bands for DC	188

6.34.2	Channel bandwidths per operating band for DC	188

6.34.3	Co-existence studies	188

6.34.4	∆TIB and ∆RIB values	189

6.34.5	MSD	189

6.35	DC_18_n28-n77	190

6.35.1	Operating bands for DC	190

6.35.2	Channel bandwidths per operating band for DC	191

6.35.3	Co-existence studies	191

6.35.4	∆TIB and ∆RIB values	191

6.35.5	MSD	192

6.36	DC_18_n28-n78	193

6.36.1	Operating bands for DC	193

6.36.2	Channel bandwidths per operating band for DC	193

6.36.3	Co-existence studies	193

6.36.4	∆TIB and ∆RIB values	194

6.36.5	MSD	194

6.37	DC_18_n3-n41	195

6.37.1	Operating bands for DC	195

6.37.2	Channel bandwidths per operating band for DC	195

6.37.3	Co-existence studies	195

6.37.4	∆TIB and ∆RIB values	196

6.37.5	MSD	196

6.38	DC_18_n41-n77	197

6.38.1	Operating bands for DC	197

6.38.2	Channel bandwidths per operating band for DC	198

6.38.3	Co-existence studies	198

6.38.4	∆TIB and ∆RIB values	198

6.38.5	MSD	199

6.39	DC_18_n41-n78	200

6.39.1	Operating bands for DC	200

6.39.2	Channel bandwidths per operating band for DC	200

6.39.3	Co-existence studies	201

6.39.4	∆TIB and ∆RIB values	201

6.39.5	MSD	201

6.40	DC_19_n1-n79	202

6.40.1	Operating bands for DC	202

6.40.2	Configuration for DC	202

6.40.3	Co-existence studies	202

6.40.4	∆TIB and ∆RIB values	203

6.40.5	MSD	203

6.41	DC_21_n1-n77	204

6.41.1	Operating bands for DC	204

6.41.2	Configuration for DC	204

6.41.3	Co-existence studies	204

6.41.4	∆TIB and ∆RIB values	205

6.41.5	MSD	205

6.42	DC_21_n1-n78	206

6.42.1	Operating bands for DC	206

6.42.2	Configuration for DC	206

6.42.3	Co-existence studies	206

6.42.4	∆TIB and ∆RIB values	206

6.42.5	MSD	207

6.43	DC_21_n1-n79	208

6.43.1	Operating bands for DC	208

6.43.2	Configuration for DC	208

6.43.3	Co-existence studies	208

6.43.4	∆TIB and ∆RIB values	208

6.43.5	MSD	209

6.44	DC_42_n1-n77	209

6.44.1	Operating bands for DC	209

6.44.2	Configuration for DC	210

6.44.3	Co-existence studies	210

6.44.4	∆TIB and ∆RIB values	210

6.44.5	MSD	210

6.45	DC_42_n1-n78	211

6.45.1	Operating bands for DC	211

6.45.2	Configuration for DC	211

6.45.3	Co-existence studies	211

6.45.4	∆TIB and ∆RIB values	211

6.45.5	MSD	212

6.46	DC_42_n1-n79	212

6.46.1	Operating bands for DC	212

6.46.2	Configuration for DC	213

6.46.3	Co-existence studies	213

6.46.4	∆TIB and ∆RIB values	213

6.46.5	MSD	213

6.47	DC_28_n1-n78	214

6.47.1	Operating bands for DC	214

6.47.2	Channel bandwidths per operating band for DC	214

6.47.3	Co-existence studies	214

6.47.4	∆TIB and ∆RIB values	215

6.47.5	MSD	215

6.48	DC_2_n5-n77	216

6.48.1	Operating bands for DC	216

6.48.2	Channel bandwidths per operating band for DC	216

6.48.3	Co-existence studies	216

6.48.4	∆TIB and ∆RIB values	217

6.48.5	MSD	217

6.49	DC_2_n66-n77	218

6.49.1	Operating bands for DC	218

6.49.2	Channel bandwidths per operating band for DC	219

6.49.3	Co-existence studies	219

6.49.4	∆TIB and ∆RIB values	219

6.49.5	MSD	220

6.50	DC_13_n2-n77	221

6.50.1	Operating bands for DC	221

6.50.2	Channel bandwidths per operating band for DC	221

6.50.3	Co-existence studies	222

6.50.4	∆TIB and ∆RIB values	222

6.50.5	MSD	222

6.51	DC_13_n5-n48	223

6.51.1	Operating bands for DC	223

6.51.2	Channel bandwidths per operating band for DC	224

6.51.3	Co-existence studies	224

6.51.4	∆TIB and ∆RIB values	224

6.51.5	MSD	225

6.52	DC_13_n48-n66	225

6.52.1	Operating bands for DC	225

6.52.2	Channel bandwidths per operating band for DC	226

6.52.3	Co-existence studies	226

6.52.4	∆TIB and ∆RIB values	226

6.52.5	MSD	227

6.53	DC_13_n66-n77	228

6.53.1	Operating bands for DC	228

6.53.2	Channel bandwidths per operating band for DC	228

6.53.3	Co-existence studies	228

6.53.4	∆TIB and ∆RIB values	229

6.53.5	MSD	229

6.54	DC_66_n2-n77	230

6.54.1	Operating bands for DC	230

6.54.2	Channel bandwidths per operating band for DC	231

6.54.3	Co-existence studies	231

6.54.4	∆TIB and ∆RIB values	231

6.54.5	MSD	232

6.55	DC_66_n5-n48	233

6.55.1	Operating bands for DC	233

6.55.2	Channel bandwidths per operating band for DC	233

6.55.3	Co-existence studies	234

6.55.4	∆TIB and ∆RIB values	234

6.55.5	MSD	234

6.56	DC_66_n5-n77	235

6.56.1	Operating bands for DC	235

6.56.2	Channel bandwidths per operating band for DC	235

6.56.3	Co-existence studies	236

6.56.4	∆TIB and ∆RIB values	236

6.56.5	MSD	236

6.57	DC_66_n66-n77	237

6.57.1	Operating bands for DC	237

6.57.2	Channel bandwidths per operating band for DC	237

6.57.3	Co-existence studies	238

6.57.4	∆TIB and ∆RIB values	238

6.57.5	MSD	238

6.58	DC_40_n41-n258	239

6.58.1	 Operating bands for DC	239

6.58.2	 Configuration for DC	239

6.58.3	 Co-existence studies	239

6.58.4	∆TIB and ∆RIB values	239

6.58.5	MSD	239

6.59	DC_40_n79-n258	240

6.59.1	 Operating bands for DC	240

6.59.2	 Configuration for DC	240

6.59.3	 Co-existence studies	240

6.59.4	∆TIB and ∆RIB values	240

6.59.5	MSD	240

6.60	 DC_41_n79-n258	241

6.60.1	 Operating bands for DC	241

6.60.2	Configuration for DC	241

6.60.3	Co-existence studies	241

6.60.4	∆TIB and ∆RIB values	241

6.60.5	MSD	241

6.61	DC_8_n78-n258	242

6.61.1	Operating bands for DC	242

6.61.2	Channel bandwidths per operating band for DC	242

6.61.3	Co-existence studies	242

6.61.4	∆TIB and ∆RIB values	242

6.61.5	MSD	243

6.62	DC_8_n40-n258	243

6.62.1	Operating bands for DC	243

6.62.2	Channel bandwidths per operating band for DC	243

6.62.3	Co-existence studies	244

6.62.4	∆TIB and ∆RIB values	244

6.62.5	MSD	244

6.63	DC_7_n78-n258	244

6.63.1	Operating bands for DC	244

6.63.2	Channel bandwidths per operating band for DC	245

6.63.3	Co-existence studies	245

6.63.4	∆TIB and ∆RIB values	245

6.63.5	MSD	246

6.64	DC_3_n78-n258	246

6.64.1	Operating bands for DC	246

6.64.2	Channel bandwidths per operating band for DC	247

6.64.3	Co-existence studies	247

6.64.4	∆TIB and ∆RIB values	247

6.64.5	MSD	248

6.65	DC_28_n78-n258	248

6.65.1	Operating bands for DC	248

6.65.2	Channel bandwidths per operating band for DC	248

6.65.3	Co-existence studies	249

6.65.4	∆TIB and ∆RIB values	249

6.65.5	MSD	249

6.66	DC_40_n1-n78	249

6.67	DC_12_n5-n77	255

6.67.5	MSD	255

6.68	DC_14_n5-n77	256

6.68.5	MSD	256

6.69	DC_2_n71-n78	256

6.69.1	Operating bands for DC	256

6.69.2	Channel bandwidths per operating band for DC	257

6.69.3	Co-existence studies	257

6.69.4	∆TIB and ∆RIB values	257

6.69.5	MSD	258

6.70	DC_7_n71-n78	258

6.70.1	Operating bands for DC	258

6.70.2	Channel bandwidths per operating band for DC	259

6.70.3	Co-existence studies	259

6.70.4	∆TIB and ∆RIB values	259

6.70.5	MSD	260

6.71	DC_71_n38-n78	260

6.71.1	Operating bands for DC	260

6.71.2	Channel bandwidths per operating band for DC	261

6.71.3	Co-existence studies	261

6.71.4	∆TIB and ∆RIB values	261

6.71.5	MSD	262

6.72	DC_5_n66-n78	263

6.72.1	Operating bands for DC	263

6.72.2	Channel bandwidths per operating band for DC	263

6.72.3	Co-existence studies	263

6.72.4	∆TIB and ∆RIB values	264

6.72.5	MSD	264

6.73	DC_71_n66-n78	265

6.73.1	Operating bands for DC	265

6.73.2	Channel bandwidths per operating band for DC	265

6.73.3	Co-existence studies	265

6.73.4	∆TIB and ∆RIB values	266

6.73.5	MSD	266

6.74	DC_5_n38-n66	267

6.74.1	Operating bands for DC	267

6.74.2	Channel bandwidths per operating band for DC	267

6.74.3	Co-existence studies	267

6.74.4	∆TIB and ∆RIB values	268

6.74.5	MSD	268

6.75	DC_12_n2-n38	269

6.75.1	Operating bands for DC	269

6.75.2	Channel bandwidths per operating band for DC	269

6.75.3	Co-existence studies	269

6.75.4	∆TIB and ∆RIB values	270

6.75.5	MSD	270

6.76	DC_12_n2-n41	271

6.76.1	Operating bands for DC	271

6.76.2	Channel bandwidths per operating band for DC	271

6.76.3	Co-existence studies	271

6.76.4	∆TIB and ∆RIB values	272

6.76.5	MSD	272

6.77	DC_71_n2-n78	273

6.77.1	Operating bands for DC	273

6.77.2	Channel bandwidths per operating band for DC	273

6.77.3	Co-existence studies	273

6.77.4	∆TIB and ∆RIB values	274

6.77.5	MSD	274

6.78	DC_1_n28-n79	275

6.78.1	Operating bands for DC	275

6.78.2	Configuration for DC	275

6.78.3	Co-existence studies	275

6.78.4	∆TIB and ∆RIB values	275

6.78.5	MSD	276

6.79	DC_3_n28-n79	277

6.79.1	Operating bands for DC	277

6.79.2	Configuration for DC	277

6.79.3	Co-existence studies	277

6.79.4	∆TIB and ∆RIB values	277

6.79.5	MSD	278

6.80	DC_21_n28-n77	279

6.80.1	Operating bands for DC	279

6.80.2	Configuration for DC	279

6.80.3	Co-existence studies	279

6.80.4	∆TIB and ∆RIB values	280

6.80.5	MSD	280

6.81	DC_21_n28-n78	281

6.81.1	Operating bands for DC	281

6.81.2	Configuration for DC	281

6.81.3	Co-existence studies	282

6.81.4	∆TIB and ∆RIB values	282

6.81.5	MSD	282

6.82	DC_66_n71-n78	283

6.82.1	Operating bands for DC	283

6.82.2	Channel bandwidths per operating band for DC	284

6.82.3	Co-existence studies	284

6.82.4	∆TIB and ∆RIB values	284

6.82.5	MSD	285

6.83	DC_2_n38-n71	285

6.83.1	Operating bands for DC	285

6.83.2	Channel bandwidths per operating band for DC	286

6.83.3	Co-existence studies	286

6.83.4	∆TIB and ∆RIB values	286

6.83.5	MSD	287

6.84	DC_66_n38-n71	287

6.84.1	Operating bands for DC	287

6.84.2	Channel bandwidths per operating band for DC	288

6.84.3	Co-existence studies	288

6.84.4	∆TIB and ∆RIB values	288

6.84.5	MSD	289

6.85	DC_71_n38-n66	289

6.85.1	Operating bands for DC	289

6.85.2	Channel bandwidths per operating band for DC	290

6.85.3	Co-existence studies	290

6.85.4	∆TIB and ∆RIB values	290

6.85.5	MSD	291

6.86	DC_2_n2-n38	291

6.86.1	Operating bands for DC	291

6.86.2	Channel bandwidths per operating band for DC	292

6.86.3	Co-existence studies	292

6.86.4	∆TIB and ∆RIB values	292

6.86.5	MSD	293

6.87	DC_66_n2-n38	293

6.87.1	Operating bands for DC	293

6.87.2	Channel bandwidths per operating band for DC	294

6.87.3	Co-existence studies	294

6.87.4	∆TIB and ∆RIB values	294

6.87.5	MSD	295

6.88	DC_2_n2-n41	295

6.88.1	Operating bands for DC	295

6.88.2	Channel bandwidths per operating band for DC	296

6.88.3	Co-existence studies	296

6.88.4	∆TIB and ∆RIB values	296

6.88.5	MSD	297

6.89	DC_71_n2-n41	297

6.89.1	Operating bands for DC	297

6.89.2	Channel bandwidths per operating band for DC	298

6.89.3	Co-existence studies	298

6.89.4	∆TIB and ∆RIB values	298

6.89.5	MSD	299

6.90	DC_7_n2-n78	300

6.90.1	Operating bands for DC	300

6.90.2	Channel bandwidths per operating band for DC	300

6.90.3	Co-existence studies	300

6.90.4	∆TIB and ∆RIB values	301

6.90.5	MSD	301

6.91	DC_2_n66-n71	302

6.91.1	Operating bands for DC	302

6.91.2	Channel bandwidths per operating band for DC	302

6.91.3	Co-existence studies	302

6.91.4	∆TIB and ∆RIB values	303

6.91.5	MSD	303

6.92	DC_7_n66-n71	303

6.92.1	Operating bands for DC	303

6.92.2	Channel bandwidths per operating band for DC	304

6.92.3	Co-existence studies	304

6.92.4	∆TIB and ∆RIB values	304

6.92.5	MSD	305

6.93	DC_66_n66-n71	305

6.93.1	Operating bands for DC	305

6.93.2	Channel bandwidths per operating band for DC	306

6.93.3	Co-existence studies	306

6.93.4	∆TIB and ∆RIB values	306

6.93.5	MSD	307

6.94	DC_2_n2-n71	307

6.94.1	Operating bands for DC	307

6.94.2	Channel bandwidths per operating band for DC	308

6.94.3	Co-existence studies	308

6.94.4	∆TIB and ∆RIB values	308

6.94.5	MSD	309

6.95	DC_7_n2-n71	309

6.95.1	Operating bands for DC	309

6.95.2	Channel bandwidths per operating band for DC	310

6.95.3	Co-existence studies	310

6.95.4	∆TIB and ∆RIB values	310

6.95.5	MSD	311

6.96	DC_66_n2-n71	311

6.96.1	Operating bands for DC	311

6.96.2	Channel bandwidths per operating band for DC	312

6.96.3	Co-existence studies	312

6.96.4	∆TIB and ∆RIB values	312

6.96.5	MSD	313

6.97	DC_2_n2-n66	313

6.97.1	Operating bands for DC	313

6.97.2	Channel bandwidths per operating band for DC	314

6.97.3	Co-existence studies	314

6.97.4	∆TIB and ∆RIB values	314

6.97.5	MSD	315

6.98	DC_7_n2-n66	315

6.98.1	Operating bands for DC	315

6.98.2	Channel bandwidths per operating band for DC	316

6.98.3	Co-existence studies	316

6.98.4	∆TIB and ∆RIB values	316

6.98.5	MSD	317

6.99	DC_71_n2-n66	317

6.99.1	Operating bands for DC	317

6.99.2	Channel bandwidths per operating band for DC	318

6.99.3	Co-existence studies	318

6.99.4	∆TIB and ∆RIB values	318

6.99.5	MSD	319

6.100	DC_21_n28-n79	319

6.100.1	Operating bands for DC	319

6.100.2	Configuration for DC	320

6.100.3	Co-existence studies	320

6.100.4	∆TIB and ∆RIB values	320

6.100.5	MSD	321

6.101	DC_8_n28-n78	321

6.101.1	 Operating bands for DC	321

6.101.2 	Configuration for DC	322

6.101.3	 Co-existence studies	322

6.101.4	 ∆TIB and ∆RIB values	326

6.101.5	MSD	326

6.102	DC_2_n2-n78	327

6.102.1	Operating bands for DC	327

6.102.2	Channel bandwidths per operating band for DC	328

6.102.3	Co-existence studies	328

6.102.4	∆TIB and ∆RIB values	328

6.102.5	MSD	329

6.103	DC_66_n2-n66	330

6.103.1	Operating bands for DC	330

6.103.2	Channel bandwidths per operating band for DC	330

6.103.3	Co-existence studies	330

6.103.4	∆TIB and ∆RIB values	331

6.103.5	MSD	331

6.104	DC_39_n40-n258	332

6.104.1	 Operating bands for DC	332

6.104.2	 Configuration for DC	332

6.104.3	 Co-existence studies	332

6.104.4	∆TIB and ∆RIB values	332

6.104.5	MSD	332

6.105	DC_39_n41-n258	333

6.105.1	 Operating bands for DC	333

6.105.2	 Configuration for DC	333

6.105.3	 Co-existence studies	333

6.105.4	∆TIB and ∆RIB values	333

6.105.5	MSD	333

6.106	DC_39_n79-n258	334

6.106.1	 Operating bands for DC	334

6.106.2	 Configuration for DC	334

6.106.3	 Co-existence studies	334

6.106.4	∆TIB and ∆RIB values	334

6.106.5	MSD	334

6.107	DC_13_n5-n77	335

6.107.1	Operating bands for DC	335

6.107.2	Configuratoin for DC	335

6.107.3	Co-existence studies	335

6.107.4	∆TIB and ∆RIB values	335

6.107.5	MSD	336

6.108 DC_28_n7-n258	337

6.108.1	Operating bands for DC	337

6.108.2	Channel bandwidths per operating band for DC	338

6.108.3	Co-existence studies	338

6.108.4	∆TIB and ∆RIB values	338

6.108.5	MSD	339

6.109	DC_7_n25-n66	339

6.109.1	Operating bands for DC	339

6.109.2	Channel bandwidths per operating band for DC	340

6.109.3	Co-existence studies	340

6.109.4	∆TIB and ∆RIB values	340

6.109.5	MSD	341

6.110	DC_66_n25-n66	341

6.110.1	Operating bands for DC	341

6.110.2	Channel bandwidths per operating band for DC	342

6.110.3	Co-existence studies	342

6.110.4	∆TIB and ∆RIB values	342

6.110.5	MSD	343

6.111	DC_30_n5-n77	343

6.111.5	MSD	343

6.112	DC_13_n25-n66	344

6.112.1	Operating bands for DC	344

6.112.2	Channel bandwidths per operating band for DC	345

6.112.3	Co-existence studies	345

6.112.4	∆TIB and ∆RIB values	345

6.112.5	MSD	346

6.113	 DC_8_n39-n40	346

6.113.1	Operating bands for DC	346

6.113.2	Configuration for DC	347

6.113.3	Co-existence studies	347

6.113.4	∆TIB and ∆RIB values	347

6.113.5	MSD	347

6.114	DC_3_n1-n8, DC_3-3_n1-n8	348

6.114.1	Operating bands for DC	348

6.114.2	Configuration for DC	348

6.114.3	Co-existence studies	348

6.114.4	∆TIB and ∆RIB values	348

6.114.5	MSD	349

6.115	DC_7_n1-n8, DC_7-7_n1-n8	349

6.115.1	Operating bands for DC	349

6.115.2	Configuration for DC	349

6.115.3	Co-existence studies	350

6.115.4	∆TIB and ∆RIB values	350

6.115.5	MSD	350

6.116	 DC_8_n39-n79	350

6.116.1	Operating bands for DC	350

6.116.2	Configuration for DC	351

6.116.3	Co-existence studies	351

6.116.4	∆TIB and ∆RIB values	351

6.116.5	MSD	352

6.117	DC_5_n66-n77	352

6.117.1	Operating bands for DC	352

6.117.2	Configuratoin for DC	353

6.117.3	Co-existence studies	353

6.117.4	∆TIB and ∆RIB values	353

6.117.5	MSD	354

6.118	DC_5_n5-n77	355

6.118.1	Operating bands for DC	355

6.118.2	Configuration for DC	355

6.118.3	Co-existence studies	355

6.118.4	∆TIB and ∆RIB values	355

6.118.5	MSD	356

6.119	DC_5_n2-n77	357

6.119.1	Operating bands for DC	357

6.119.2	Configuration for DC	357

6.119.3	Co-existence studies	357

6.119.4	∆TIB and ∆RIB values	357

6.119.5	MSD	358

6.120	DC_2_n2-n77	359

6.120.1	Operating bands for DC	359

6.120.2	Configuration for DC	359

6.120.3	     Co-existence studies	359

6.120.4	∆TIB and ∆RIB values	359

6.120.5	MSD	360

6.121	DC_3-3_n8-n78	361

6.121.1	Operating bands for DC	361

6.121.2	Configuration for DC	361

6.121.3	Co-existence studies	361

6.121.4	∆TIB and ∆RIB values	362

6.121.5	MSD	362

6.122	DC_7-7_n8-n78	362

6.122.1	Operating bands for DC	362

6.122.2	Configuration for DC	363

6.122.3	Co-existence studies	363

6.122.4	∆TIB and ∆RIB values	363

6.122.5	MSD	364

6.123.2	Configuration for DC	364

6.124	DC_20_n8-n78	366

6.124.1	 Operating bands for DC	366

6.124.2 	Configuration for DC	366

6.124.3	 Co-existence studies	366

6.124.4	 ∆TIB and ∆RIB values	370

6.124.5	MSD	371

6.125	DC_3_n1-n38	371

6.125.1	Operating bands for DC	371

6.125.2	Channel bandwidths per operating band for DC	372

6.125.3	Co-existence studies	372

6.125.4	∆TIB and ∆RIB values	372

6.125.5	MSD	372

6.126	DC_8_n1-n40	373

6.126.1	Operating bands for DC	373

6.126.2	Configuration for DC	373

6.126.3	Co-existence studies	373

6.126.4	∆TIB and ∆RIB values	373

6.126.5	MSD	374

6.127	DC_8_n3-n79	374

6.127.1	Operating bands for DC	374

6.127.2	Channel bandwidths per operating band for DC	375

6.127.3	Co-existence studies	375

6.127.4	∆TIB and ∆RIB values	375

6.127.5	MSD	376

6.128	DC_8_n77-n79	377

6.128.1	Operating bands for DC	377

6.128.2	Channel bandwidths per operating band for DC	377

6.128.3	Co-existence studies	378

6.128.4	∆TIB and ∆RIB values	378

6.128.5	MSD	378

6.129	DC_1_n3-n79	378

6.129.1	Operating bands for DC	378

6.129.2	Channel bandwidths per operating band for DC	379

6.129.3	Co-existence studies	379

6.129.4	∆TIB and ∆RIB values	379

6.129.5	MSD	380

6.130	DC_8_n28-n79	381

6.130.1	Operating bands for DC	381

6.130.2	Channel bandwidths per operating band for DC	381

6.130.3	Co-existence studies	382

6.130.4	∆TIB and ∆RIB values	382

6.130.5	MSD	382

6.131	DC_12_n66-n78	383

6.131.1	Operating bands for DC	383

6.131.2	Configuration for DC	384

6.131.3	Co-existence studies	384

6.131.4	∆TIB and ∆RIB values	384

6.131.5	MSD	385

6.132	DC_13_n7-n78	385

6.132.1	Operating bands for DC	385

6.132.2	Configuration for DC	386

6.132.3	Co-existence studies	386

6.132.4	∆TIB and ∆RIB values	386

6.132.5	MSD	387

6.133	DC_3_n1-n41	387

6.133.1	Operating bands for DC	387

6.133.2	Channel bandwidths per operating band for DC	388

6.133.3	Co-existence studies	388

6.133.4	∆TIB and ∆RIB values	388

6.133.5	MSD	388

6.134	DC_8_n1-n77	389

6.134.1	Operating bands for DC	389

6.134.2	Channel bandwidths per operating band for DC	390

6.134.3	Co-existence studies	390

6.134.4	∆TIB and ∆RIB values	391

6.134.5	MSD	391

6.135	DC_42_n1-n3	392

6.135.1	Operating bands for DC	392

6.135.2	Channel bandwidths per operating band for DC	393

6.135.3	Co-existence studies	393

6.135.4	∆TIB and ∆RIB values	394

6.135.5	MSD	394

6.136	DC_7_n12-n77	394

6.136.5	MSD	394

6.137	DC_7_n71-n77	395

6.137.5	MSD	395

6.138	DC_5_n2-n41	395

6.138.5	MSD	395

6.139	DC_3_n28-n75	396

6.139.5	MSD	399

6.140	DC_1_n38-n78	399

6.140.1	Operating bands for DC	399

6.140.2	Configuration for DC	400

6.140.3	Co-existence studies	400

6.140.4	∆TIB and ∆RIB values	400

6.140.5	MSD	401

6.141	DC_3_n38-n78	401

6.141.1	Operating bands for DC	401

6.141.2	Configuration for DC	401

6.141.3	Co-existence studies	401

6.141.4	∆TIB and ∆RIB values	402

6.141.5	MSD	402

6.142	DC_7_n38-n78	402

6.142.1	Operating bands for DC	402

6.142.2	Configuration for DC	403

6.142.3	Co-existence studies	403

6.142.4	∆TIB and ∆RIB values	403

6.142.5	MSD	403

6.143	DC_20_n38-n78	404

6.143.1	Operating bands for DC	404

6.143.2	Configuration for DC	404

6.143.3	Co-existence studies	404

6.143.4	∆TIB and ∆RIB values	404

6.143.5	MSD	405

6.144	DC_1_n3-n8	405

6.144.1	 Operating bands for DC	405

6.144.2 	Configuration for DC	406

6.144.3	 Co-existence studies	406

6.144.4	 ∆TIB and ∆RIB values	410

6.144.5	MSD	411

6.145	DC_8_n1-n28	411

6.145.1	 Operating bands for DC	411

6.145.2 	Configuration for DC	411

6.145.3	 Co-existence studies	412

6.145.4	 ∆TIB and ∆RIB values	412

6.145.5	MSD	412

6.146	DC_11_n77-n79	413

6.146.1	Operating bands for DC	413

6.146.2	Channel bandwidths per operating band for DC	414

6.146.3	Co-existence studies	414

6.146.4	∆TIB and ∆RIB values	415

6.146.5	MSD	415

6.147	DC_7_n78-n79	415

6.147.1	 Operating bands for DC	415

6.147.2 	Configuration for DC	416

6.147.3	 Co-existence studies	416

6.147.4	 ∆TIB and ∆RIB values	420

6.147.5	MSD	420

6.148	DC_8_n1-n3	421

6.148.1	Operating bands for DC	421

6.148.2	Channel bandwidths per operating band for DC	422

6.148.3	Co-existence studies	422

6.148.4	∆TIB and ∆RIB values	422

6.148.5	MSD	423

6.149	DC_41_n1-n77	423

6.149.1	Operating bands for DC	423

6.149.2	Channel bandwidths per operating band for DC	424

6.149.3	Co-existence studies	424

6.149.4	∆TIB and ∆RIB values	425

6.149.5	MSD	425

6.150	DC_42_n1-n3	426

6.150.1	Operating bands for DC	426

6.150.2	Channel bandwidths per operating band for DC	427

6.150.3	Co-existence studies	427

6.150.4	∆TIB and ∆RIB values	428

6.150.5	MSD	428

6.151	DC_11_n3-n79	428

6.151.1	Operating bands for DC	428

6.151.2	Channel bandwidths per operating band for DC	429

6.151.3	Co-existence studies	429

6.151.4	∆TIB and ∆RIB values	429

6.151.5	MSD	430

6.152	DC_11_n1-n77	431

6.152.5	MSD	431

6.153	DC_3_n20-n67	431

6.153.5	MSD	431

6.154	DC_11_n3-n79	432

6.154.5	MSD	432

6.155	DC_2_n25-n66	432

6.155.1	Operating bands for DC	432

6.155.2	Channel bandwidths per operating band for DC	433

6.155.3	Co-existence studies	433

6.155.4	∆TIB and ∆RIB values	433

6.155.5	MSD	433

6.156	DC_5_n2-n78	434

6.156.1	Operating bands for DC	434

6.156.2	Channel bandwidths per operating band for DC	434

6.156.3	Co-existence studies	435

6.156.4	∆TIB and ∆RIB values	435

6.156.5	MSD	435

6.157	DC_66_n2-n78	436

6.157.1	Operating bands for DC	436

6.157.2	Channel bandwidths per operating band for DC	436

6.157.3	Co-existence studies	437

6.157.4	∆TIB and ∆RIB values	437

6.157.5	MSD	437

6.158	DC_12_n2-n78	438

6.158.1	Operating bands for DC	438

6.158.2	Channel bandwidths per operating band for DC	439

6.158.3	Co-existence studies	439

6.158.4	∆TIB and ∆RIB values	439

6.158.5	MSD	440

6.159	DC_1_n28-n75	440

6.159.1	 Operating bands for DC	440

6.159.2 	Configuration for DC	440

6.159.3	 Co-existence studies	441

6.159.4	 ∆TIB and ∆RIB values	443

6.160	DC_38_n3-n78	443

6.160.1	Operating bands for DC	443

6.160.2	Configuration for DC	444

6.160.3	Co-existence studies	444

6.160.4	∆TIB and ∆RIB values	444

6.160.5	MSD	445

6.161	DC_3_n20-n67	445

6.161.1	Operating bands for DC	445

6.161.2	Channel bandwidths per operating band for DC	446

6.161.3	Co-existence studies	446

6.161.4	∆TIB and ∆RIB values	446

6.161.5	MSD	447

6.162	DC_11_n1-n77	447

6.162.1	Operating bands for DC	447

6.162.2	Channel bandwidths per operating band for DC	448

6.162.3	Co-existence studies	448

6.162.4	∆TIB and ∆RIB values	448

6.162.5	MSD	449

6.163	DC_5_n1-n78	450

6.163.1	Operating bands for DC	450

6.163.2	Channel bandwidths per operating band for DC	450

6.163.3	Co-existence studies	450

6.163.4	∆TIB and ∆RIB values	450

6.163.5	MSD	451

6.164	DC_5_n3-n78	452

6.164.1	Operating bands for DC	452

6.164.2	Channel bandwidths per operating band for DC	452

6.164.3	Co-existence studies	452

6.164.4	∆TIB and ∆RIB values	452

6.164.5	MSD	453

6.165	DC_8_n3-n78	454

6.165.1	Operating bands for DC	454

6.165.2	Channel bandwidths per operating band for DC	454

6.165.3	Co-existence studies	454

6.165.4	∆TIB and ∆RIB values	454

6.165.5	MSD	455

6.166	DC_20_n1-n67	456

6.166.1	Operating bands for DC	456

6.166.2	Channel bandwidths per operating band for DC	456

6.166.3	Co-existence studies	456

6.166.4	∆TIB and ∆RIB values	456

6.166.5	MSD	457

6.167	DC_20_n3-n67	457

6.167.1	Operating bands for DC	457

6.167.2	Channel bandwidths per operating band for DC	458

6.167.3	Co-existence studies	458

6.167.4	∆TIB and ∆RIB values	458

6.167.5	MSD	458

6.168	DC_3_n41-n258	459

6.168.1	Operating bands for DC	459

6.168.2	Configuration for DC	459

6.168.3	Co-existence studies	459

6.168.4	∆TIB and ∆RIB values	460

6.168.5	MSD	460

6.169	DC_8_n41-n258	460

6.169.1	Operating bands for DC	460

6.169.2	Configuration for DC	460

6.169.3	Co-existence studies	460

6.169.4	∆TIB and ∆RIB values	461

6.169.5	MSD	461

6.170	DC_41_n1-n3	461

6.170.1	Operating bands for DC	461

6.170.2	Channel bandwidths per operating band for DC	462

6.170.3	Co-existence studies	462

6.170.4	∆TIB and ∆RIB values	462

6.170.5	MSD	463

6.171	DC_1_n3-n75	463

6.171.1	Operating bands for DC	463

6.171.2	Channel bandwidths per operating band for DC	464

6.171.3	Co-existence studies	464

6.171.4	∆TIB and ∆RIB values	466

6.171.5	MSD	466

6.172	DC_20_n7-n78	467

6.172.1	Operating bands for DC	467

6.172.2	Channel bandwidths per operating band for DC	467

6.172.3	Co-existence studies	467

6.172.4	∆TIB and ∆RIB values	470

6.172.5	Reference sensitivity exceptions	471

6.173	DC_3_n5-n40	471

6.173.1	Operating bands for DC	471

6.173.2	Inter-band DC Configurations	472

6.173.3	Co-existence studies	472

6.173.4	∆TIB and ∆RIB values	472

6.173.5	MSD	472

6.174	DC_8_n79-n258	473

6.174.1	Operating bands for DC	473

6.174.2	Configuration for DC	473

6.174.3	Co-existence studies	473

6.174.4	∆TIB and ∆RIB values	473

6.174.5	MSD	473

6.175	DC_n8-n77_1	474

6.175.1	 Operating bands for DC	474

6.175.2	 Channel bandwidths per operating band for DC	474

6.175.3	 Co-existence studies	475

6.175.4	∆TIB and ∆RIB values	475

6.175.5	MSD	475

6.176	 DC_1_n8-n77	475

6.176.1	 Operating bands for DC	475

6.176.2	 Channel bandwidths per operating band for DC	476

6.176.3	 Co-existence studies	476

6.176.4	∆TIB and ∆RIB values	480

6.176.5	MSD	480

6.177	DC_n77-n257_1	481

6.177.1	 Operating bands for DC	481

6.177.2	 Channel bandwidths per operating band for DC	482

6.177.3	 Co-existence studies	482

6.177.4	∆TIB and ∆RIB values	482

6.177.5	MSD	482

6.178	DC_n77-n257_3	483

6.178.1	 Operating bands for DC	483

6.178.2	 Channel bandwidths per operating band for DC	483

6.178.3	 Co-existence studies	484

6.178.4	∆TIB and ∆RIB values	484

6.178.5	MSD	484

7	Dual Connectivity band combinations of LTE 2 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	485

7.1	DC_3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-7-7_n1-n257, DC_3-3-7-7_n1-n257	485

7.1.1	Operating bands for DC	485

7.1.2	Channel bandwidths per operating band for DC	486

7.1.3	Co-existence studies	488

7.1.4	∆TIB and ∆RIB values	488

7.1.5	MSD	488

7.2	DC_1-42_n28-n77	488

7.2.1	Operating bands for DC	488

7.2.2	Channel bandwidths per operating band for DC	488

7.2.3	Co-existence studies	490

7.2.4	∆TIB and ∆RIB values	490

7.2.5	MSD	490

7.3	DC_3-42_n28-n77	491

7.3.1	Operating bands for DC	491

7.3.2	Channel bandwidths per operating band for DC	491

7.3.3	Co-existence studies	492

7.3.4	∆TIB and ∆RIB values	492

7.3.5	MSD	493

7.4	DC_8-42_n28-n77	493

7.4.1	Operating bands for DC	493

7.4.2	Channel bandwidths per operating band for DC	493

7.4.3	Co-existence studies	494

7.4.4	∆TIB and ∆RIB values	494

7.4.5	MSD	495

7.5	DC_1-3_n28-n41	495

7.5.1	Operating bands for DC	495

7.5.2	Channel bandwidths per operating band for DC	496

7.5.3	Co-existence studies	496

7.5.4	∆TIB and ∆RIB values	496

7.5.5	MSD	497

7.6	DC_2-48_n48-n66	497

7.6.1	Operating bands for DC	497

7.6.2	Configuration for DC	497

7.6.3	Co-existence studies	498

7.6.4	∆TIB and ∆RIB values	498

7.6.5	MSD	498

7.7	DC_48-66_n25-n48	498

7.7.1	Operating bands for DC	498

7.7.2	Configuration for DC	499

7.7.3	Co-existence studies	499

7.7.4	∆TIB and ∆RIB values	499

7.7.5	MSD	500

7.8	DC_1-8_n40-n78	500

7.8.1	Operating bands for DC	500

7.8.2	Configuration for DC	500

7.8.3	Co-existence studies	500

7.8.4	∆TIB and ∆RIB values	500

7.8.5	MSD	501

7.9	DC_3-7_n1-n40	501

7.9.1	Operating bands for DC	501

7.9.2	Configuration for DC	502

7.9.3	Co-existence studies	502

7.9.4	∆TIB and ∆RIB values	502

7.9.5	MSD	502

7.10	DC_3-8_n40-n78	503

7.10.1	Operating bands for DC	503

7.10.2	Configuration for DC	503

7.10.3	Co-existence studies	503

7.10.4	∆TIB and ∆RIB values	503

7.10.5	MSD	504

7.11	DC_3-28_n1-n40	504

7.11.1	Operating bands for DC	504

7.11.2	Configuration for DC	505

7.11.3	Co-existence studies	505

7.11.4	∆TIB and ∆RIB values	505

7.11.5	MSD	505

7.12	DC_7-8_n40-n78	506

7.12.1	Operating bands for DC	506

7.12.2	Configuration for DC	506

7.12.3	Co-existence studies	506

7.12.4	∆TIB and ∆RIB values	506

7.12.5	MSD	507

7.13	DC_7-28_n1-n40	507

7.13.1	Operating bands for DC	507

7.13.2	Configuration for DC	508

7.13.3	Co-existence studies	508

7.13.4	∆TIB and ∆RIB values	508

7.13.5	MSD	508

7.14	DC_1-3_n41-n77	509

7.14.1	Operating bands for DC	509

7.14.2	Channel bandwidths per operating band for DC	509

7.14.3	Co-existence studies	509

7.14.4	∆TIB and ∆RIB values	509

7.14.5	MSD	510

7.15	DC_1-3_n3-n41	510

7.15.1	Operating bands for DC	510

7.15.2	Channel bandwidths per operating band for DC	511

7.15.3	Co-existence studies	511

7.15.4	∆TIB and ∆RIB values	511

7.15.5	MSD	512

7.16	DC_1-3_n3-n77	512

7.16.1	Operating bands for DC	512

7.16.2	Channel bandwidths per operating band for DC	512

7.16.3	Co-existence studies	513

7.16.4	∆TIB and ∆RIB values	513

7.16.5	MSD	513

7.17	DC_1-3_n3-n78	513

7.17.1	Operating bands for DC	513

7.17.2	Channel bandwidths per operating band for DC	514

7.17.3	Co-existence studies	514

7.17.4	∆TIB and ∆RIB values	514

7.17.5	MSD	515

7.18	DC_1-18_n3-n77	515

7.18.1	Operating bands for DC	515

7.18.2	Channel bandwidths per operating band for DC	515

7.18.3	Co-existence studies	516

7.18.4	∆TIB and ∆RIB values	516

7.18.5	MSD	516

7.19	DC_1-41_n3-n41	516

7.19.1	Operating bands for DC	516

7.19.2	Channel bandwidths per operating band for DC	516

7.19.3	Co-existence studies	516

7.19.4	∆TIB and ∆RIB values	517

7.19.5	MSD	517

7.20	DC_1-41_n3-n77	517

7.20.1	Operating bands for DC	517

7.20.2	Channel bandwidths per operating band for DC	518

7.20.3	Co-existence studies	518

7.20.4	∆TIB and ∆RIB values	518

7.20.5	MSD	518

7.21	DC_1-41_n3-n78	518

7.21.1	Operating bands for DC	518

7.21.2	Channel bandwidths per operating band for DC	519

7.21.3	Co-existence studies	519

7.21.4	∆TIB and ∆RIB values	519

7.21.5	MSD	519

7.22	DC_1-41_n41-n77	519

7.22.1	Operating bands for DC	519

7.22.2	Channel bandwidths per operating band for DC	520

7.22.3	Co-existence studies	520

7.22.4	∆TIB and ∆RIB values	520

7.22.5	MSD	521

7.23	DC_1-41_n41-n78	521

7.23.1	Operating bands for DC	521

7.23.2	Channel bandwidths per operating band for DC	521

7.23.3	Co-existence studies	521

7.23.4	∆TIB and ∆RIB values	521

7.23.5	MSD	522

7.24	DC_3-18_n3-n77	522

7.24.1	Operating bands for DC	522

7.24.2	Channel bandwidths per operating band for DC	523

7.24.3	Co-existence studies	523

7.24.4	∆TIB and ∆RIB values	523

7.24.5	MSD	524

7.25	DC_3-18_n3-n78	524

7.25.1	Operating bands for DC	524

7.25.2	Channel bandwidths per operating band for DC	524

7.25.3	Co-existence studies	524

7.25.4	∆TIB and ∆RIB values	524

7.25.5	MSD	525

7.26	DC_3-41_n3-n41	525

7.26.1	Operating bands for DC	525

7.26.2	Channel bandwidths per operating band for DC	526

7.26.3	Co-existence studies	526

7.26.4	∆TIB and ∆RIB values	526

7.26.5	MSD	527

7.27	DC_3-41_n3-n77	527

7.27.1	Operating bands for DC	527

7.27.2	Channel bandwidths per operating band for DC	528

7.27.3	Co-existence studies	528

7.27.4	∆TIB and ∆RIB values	528

7.27.5	MSD	529

7.28	DC_3-41_n3-n78	529

7.28.1	Operating bands for DC	529

7.28.2	Channel bandwidths per operating band for DC	530

7.28.3	Co-existence studies	530

7.28.4	∆TIB and ∆RIB values	530

7.28.5	MSD	531

7.29	DC_3-41_n41-n77	531

7.29.1	Operating bands for DC	531

7.29.2	Channel bandwidths per operating band for DC	531

7.29.3	Co-existence studies	532

7.29.4	∆TIB and ∆RIB values	532

7.29.5	MSD	532

7.30	DC_3-41_n41-n78	533

7.30.1	Operating bands for DC	533

7.30.2	Channel bandwidths per operating band for DC	533

7.30.3	Co-existence studies	533

7.30.4	∆TIB and ∆RIB values	533

7.30.5	MSD	534

7.31	DC_2-7_n38-n66	534

7.31.1	Operating bands for DC	534

7.31.2	Channel bandwidths per operating band for DC	535

7.31.3	Co-existence studies	535

7.31.4	∆TIB and ∆RIB values	535

7.31.5	MSD	536

7.32	DC_7-66_n38-n78 and DC_7-7-66_n38-n78	536

7.32.1	Operating bands for DC	536

7.32.2	Channel bandwidths per operating band for DC	537

7.32.3	Co-existence studies	537

7.32.4	∆TIB and ∆RIB values	537

7.32.5	MSD	538

7.33	DC_1-8_n3-n77	538

7.33.1	Operating bands for DC	538

7.33.2	Channel bandwidths per operating band for DC	539

7.33.3	Co-existence studies	540

7.33.4	∆TIB and ∆RIB values	540

7.33.5	MSD	540

7.34	DC_1-11_n3-n28	541

7.34.1	Operating bands for DC	541

7.34.2	Channel bandwidths per operating band for DC	542

7.34.3	Co-existence studies	542

7.34.4	∆TIB and ∆RIB values	542

7.34.5	MSD	543

7.35	DC_8-11_n3-n28	543

7.35.1	Operating bands for DC	543

7.35.2	Channel bandwidths per operating band for DC	544

7.35.3	Co-existence studies	544

7.35.4	∆TIB and ∆RIB values	544

7.35.5	MSD	545

7.36	DC_1-18_n28-n41	545

7.36.1	Operating bands for DC	545

7.36.2	Channel bandwidths per operating band for DC	546

7.36.3	Co-existence studies	546

7.36.4	∆TIB and ∆RIB values	546

7.36.5	MSD	547

7.37	DC_1-18_n28-n77	547

7.37.1	Operating bands for DC	547

7.37.2	Channel bandwidths per operating band for DC	548

7.37.3	Co-existence studies	548

7.37.4	∆TIB and ∆RIB values	548

7.37.5	MSD	549

7.38	DC_1-18_n28-n78	549

7.38.1	Operating bands for DC	549

7.38.2	Channel bandwidths per operating band for DC	550

7.38.3	Co-existence studies	550

7.38.4	∆TIB and ∆RIB values	550

7.38.5	MSD	551

7.39	DC_1-18_n3-n41	551

7.39.1	Operating bands for DC	551

7.39.2	Channel bandwidths per operating band for DC	552

7.39.3	Co-existence studies	552

7.39.4	∆TIB and ∆RIB values	552

7.39.5	MSD	553

7.40	DC_1-18_n41-n77	553

7.40.1	Operating bands for DC	553

7.40.2	Channel bandwidths per operating band for DC	554

7.40.3	Co-existence studies	554

7.40.4	∆TIB and ∆RIB values	554

7.40.5	MSD	555

7.41	DC_1-18_n41-n78	555

7.41.1	Operating bands for DC	555

7.41.2	Channel bandwidths per operating band for DC	556

7.41.3	Co-existence studies	556

7.41.4	∆TIB and ∆RIB values	556

7.41.5	MSD	557

7.42	DC_1-41_n28-n41	557

7.42.1	Operating bands for DC	557

7.42.2	Channel bandwidths per operating band for DC	558

7.42.3	Co-existence studies	558

7.42.4	∆TIB and ∆RIB values	558

7.42.5	MSD	559

7.43	DC_3-18_n28-n41	559

7.43.1	Operating bands for DC	559

7.43.2	Channel bandwidths per operating band for DC	560

7.43.3	Co-existence studies	560

7.43.4	∆TIB and ∆RIB values	560

7.43.5	MSD	561

7.44	DC_3-18_n28-n77	561

7.44.1	Operating bands for DC	561

7.44.2	Channel bandwidths per operating band for DC	562

7.44.3	Co-existence studies	562

7.44.4	∆TIB and ∆RIB values	562

7.44.5	MSD	563

7.45	DC_3-18_n28-n78	563

7.45.1	Operating bands for DC	563

7.45.2	Channel bandwidths per operating band for DC	564

7.45.3	Co-existence studies	564

7.45.4	∆TIB and ∆RIB values	564

7.45.5	MSD	565

7.46	DC_3-18_n3-n41	565

7.46.1	Operating bands for DC	565

7.46.2	Channel bandwidths per operating band for DC	566

7.46.3	Co-existence studies	566

7.46.4	∆TIB and ∆RIB values	566

7.46.5	MSD	567

7.47	DC_3-18_n41-n77	567

7.47.1	Operating bands for DC	567

7.47.2	Channel bandwidths per operating band for DC	568

7.47.3	Co-existence studies	568

7.47.4	∆TIB and ∆RIB values	568

7.47.5	MSD	569

7.48	DC_3-18_n41-n78	569

7.48.1	Operating bands for DC	569

7.48.2	Channel bandwidths per operating band for DC	570

7.48.3	Co-existence studies	570

7.48.4	∆TIB and ∆RIB values	570

7.48.5	MSD	571

7.49	DC_3-41_n28-n41	571

7.49.1	Operating bands for DC	571

7.49.2	Channel bandwidths per operating band for DC	572

7.49.3	Co-existence studies	572

7.49.4	∆TIB and ∆RIB values	572

7.49.5	MSD	573

7.50	DC_3-19_n1-n77	573

7.50.1	Operating bands for DC	573

7.50.2	Configuration for DC	574

7.50.3	Co-existence studies	574

7.50.4	∆TIB and ∆RIB values	574

7.50.5	MSD	575

7.51	DC_3-19_n1-n78	575

7.51.1	Operating bands for DC	575

7.51.2	Configuration for DC	575

7.51.3	Co-existence studies	575

7.51.4	∆TIB and ∆RIB values	575

7.51.5	MSD	576

7.52	DC_3-19_n1-n79	576

7.52.1	Operating bands for DC	576

7.52.2	Configuration for DC	577

7.52.3	Co-existence studies	577

7.52.4	∆TIB and ∆RIB values	577

7.52.5	MSD	578

7.53	DC_3-21_n1-n77	578

7.53.1	Operating bands for DC	578

7.53.2	Configuration for DC	578

7.53.3	Co-existence studies	578

7.53.4	∆TIB and ∆RIB values	578

7.53.5	MSD	579

7.54	DC_3-21_n1-n78	579

7.54.1	Operating bands for DC	579

7.54.2	Configuration for DC	580

7.54.3	Co-existence studies	580

7.54.4	∆TIB and ∆RIB values	580

7.54.5	MSD	581

7.55	DC_3-21_n1-n79	581

7.55.1	Operating bands for DC	581

7.55.2	Configuration for DC	581

7.55.3	Co-existence studies	581

7.55.4	∆TIB and ∆RIB values	581

7.55.5	MSD	582

7.56	DC_3-42_n1-n77	582

7.56.1	Operating bands for DC	582

7.56.2	Configuration for DC	583

7.56.3	Co-existence studies	583

7.56.4	∆TIB and ∆RIB values	583

7.56.5	MSD	584

7.57	DC_3-42_n1-n78	584

7.57.1	Operating bands for DC	584

7.57.2	Configuration for DC	584

7.57.3	Co-existence studies	584

7.57.4	∆TIB and ∆RIB values	584

7.57.5	MSD	585

7.58	DC_3-42_n1-n79	585

7.58.1	Operating bands for DC	585

7.58.2	Configuration for DC	586

7.58.3	Co-existence studies	586

7.58.4	∆TIB and ∆RIB values	586

7.58.5	MSD	587

7.59	DC_19-21_n1-n77	587

7.59.1	Operating bands for DC	587

7.59.2	Configuration for DC	587

7.59.3	Co-existence studies	587

7.59.4	∆TIB and ∆RIB values	587

7.59.5	MSD	588

7.60	DC_19-21_n1-n78	588

7.60.1	Operating bands for DC	588

7.60.2	Configuration for DC	589

7.60.3	Co-existence studies	589

7.60.4	∆TIB and ∆RIB values	589

7.60.5	MSD	590

7.61	DC_19-21_n1-n79	590

7.61.1	Operating bands for DC	590

7.61.2	Configuration for DC	590

7.61.3	Co-existence studies	590

7.61.4	∆TIB and ∆RIB values	590

7.61.5	MSD	591

7.62	DC_19-42_n1-n77	591

7.62.1	Operating bands for DC	591

7.62.2	Configuration for DC	592

7.62.3	Co-existence studies	592

7.62.4	∆TIB and ∆RIB values	592

7.62.5	MSD	593

7.63	DC_19-42_n1-n78	593

7.63.1	Operating bands for DC	593

7.63.2	Configuration for DC	593

7.63.3	Co-existence studies	593

7.63.4	∆TIB and ∆RIB values	593

7.63.5	MSD	594

7.64	DC_19-42_n1-n79	594

7.64.1	Operating bands for DC	594

7.64.2	Configuration for DC	595

7.64.3	Co-existence studies	595

7.64.4	∆TIB and ∆RIB values	595

7.64.5	MSD	596

7.65	DC_21-42_n1-n77	596

7.65.1	Operating bands for DC	596

7.65.2	Configuration for DC	596

7.65.3	Co-existence studies	596

7.65.4	∆TIB and ∆RIB values	596

7.65.5	MSD	597

7.66	DC_21-42_n1-n78	597

7.66.1	Operating bands for DC	597

7.66.2	Configuration for DC	598

7.66.3	Co-existence studies	598

7.66.4	∆TIB and ∆RIB values	598

7.66.5	MSD	599

7.67	DC_21-42_n1-n79	599

7.67.1	Operating bands for DC	599

7.67.2	Configuration for DC	599

7.67.3	Co-existence studies	599

7.67.4	∆TIB and ∆RIB values	599

7.67.5	MSD	600

7.68	DC_3-20_n1-n78	600

7.68.1	Operating bands for DC	600

7.68.2	Channel bandwidths per operating band for DC	601

7.68.3	Co-existence studies	601

7.68.4	∆TIB and ∆RIB values	601

7.68.5	MSD	602

7.69	DC_7-20_n1-n78	602

7.69.1	Operating bands for DC	602

7.69.2	Channel bandwidths per operating band for DC	603

7.69.3	Co-existence studies	603

7.69.4	∆TIB and ∆RIB values	603

7.69.5	MSD	604

7.70	DC_3-7_n40-n78	604

7.70.1	Operating bands for DC	604

7.70.2	Channel bandwidths per operating band for DC	604

7.70.3	Co-existence studies	605

7.70.4	∆TIB and ∆RIB values	605

7.70.5	MSD	605

7.71	DC_1-7_n40-n78	606

7.71.1	Operating bands for DC	606

7.71.2	Channel bandwidths per operating band for DC	606

7.71.3	Co-existence studies	607

7.71.4	∆TIB and ∆RIB values	607

7.71.5	MSD	607

7.72	DC_7-28_n40-n78	608

7.72.1	Operating bands for DC	608

7.72.2	Channel bandwidths per operating band for DC	608

7.72.3	Co-existence studies	609

7.72.4	∆TIB and ∆RIB values	609

7.72.5	MSD	609

7.73	DC_2-13_n66-n77	610

7.73.1	Operating bands for DC	610

7.73.2	Channel bandwidths per operating band for DC	611

7.73.3	Co-existence studies	611

7.73.4	∆TIB and ∆RIB values	611

7.73.5	MSD	612

7.74	DC_2-66_n66-n77	612

7.74.1	Operating bands for DC	612

7.74.2	Channel bandwidths per operating band for DC	613

7.74.3	Co-existence studies	613

7.74.4	∆TIB and ∆RIB values	613

7.74.5	MSD	614

7.75	DC_2-66_n5-n77	614

7.75.1	Operating bands for DC	614

7.75.2	Channel bandwidths per operating band for DC	615

7.75.3	Co-existence studies	615

7.75.4	∆TIB and ∆RIB values	615

7.75.5	MSD	616

7.76	DC_13-66_n66-n77	616

7.76.1	Operating bands for DC	616

7.76.2	Channel bandwidths per operating band for DC	617

7.76.3	Co-existence studies	617

7.76.4	∆TIB and ∆RIB values	617

7.76.5	MSD	618

7.77	DC_13-66_n2-n77	618

7.77.1	Operating bands for DC	618

7.77.2	Channel bandwidths per operating band for DC	619

7.77.3	Co-existence studies	619

7.77.4	∆TIB and ∆RIB values	619

7.77.5	MSD	620

7.78	DC_13-66_n5-n48	620

7.78.1	Operating bands for DC	620

7.78.2	Channel bandwidths per operating band for DC	621

7.78.3	Co-existence studies	621

7.78.4	∆TIB and ∆RIB values	621

7.78.5	MSD	622

7.79	DC_1-8_n78-n258	622

7.79.1	Operating bands for DC	622

7.79.2	Channel bandwidths per operating band for DC	623

7.79.3	Co-existence studies	623

7.79.4	∆TIB and ∆RIB values	623

7.79.5	MSD	623

7.80	DC_3-8_n78-n258	624

7.80.1	Operating bands for DC	624

7.80.2	Channel bandwidths per operating band for DC	624

7.80.3	Co-existence studies	624

7.80.4	∆TIB and ∆RIB values	625

7.80.5	MSD	625

7.81	DC_7-8_n78-n258	625

7.81.1	Operating bands for DC	625

7.81.2	Channel bandwidths per operating band for DC	626

7.81.3	Co-existence studies	627

7.81.4	∆TIB and ∆RIB values	627

7.81.5	MSD	627

7.82	DC_1-8_n40-n258	627

7.82.1	Operating bands for DC	627

7.82.2	Channel bandwidths per operating band for DC	628

7.82.3	Co-existence studies	628

7.82.4	∆TIB and ∆RIB values	628

7.82.5	MSD	628

7.83	DC_3-8_n40-n258	628

7.83.1	Operating bands for DC	628

7.83.2	Channel bandwidths per operating band for DC	629

7.83.3	Co-existence studies	630

7.83.4	∆TIB and ∆RIB values	630

7.83.5	MSD	630

7.84	DC_7-8_n40-n258	630

7.84.1	Operating bands for DC	630

7.84.2	Channel bandwidths per operating band for DC	631

7.84.3	Co-existence studies	632

7.84.4	∆TIB and ∆RIB values	632

7.84.5	MSD	632

7.85	DC_3-11_n28-n77	632

7.85.1	Operating bands for DC	632

7.85.2	Channel bandwidths per operating band for DC	633

7.85.3	Co-existence studies	633

7.85.4	∆TIB and ∆RIB values	634

7.85.5	MSD	634

7.86	DC_8-11_n28-n77	634

7.86.1	Operating bands for DC	634

7.86.2	Channel bandwidths per operating band for DC	635

7.86.3	Co-existence studies	636

7.86.4	∆TIB and ∆RIB values	636

7.86.5	MSD	636

7.87	DC_1-11_n3-n77	637

7.87.1	Operating bands for DC	637

7.87.2	Channel bandwidths per operating band for DC	638

7.87.3	Co-existence studies	639

7.87.4	∆TIB and ∆RIB values	639

7.87.5	MSD	639

7.88	DC_8-11_n3-n77	640

7.88.1	Operating bands for DC	640

7.88.2	Channel bandwidths per operating band for DC	641

7.88.3	Co-existence studies	642

7.88.4	∆TIB and ∆RIB values	642

7.88.5	MSD	642

7.89	DC_1-11_n28-n77	643

7.89.1	Operating bands for DC	643

7.89.2	Channel bandwidths per operating band for DC	644

7.89.3	Co-existence studies	645

7.89.4	∆TIB and ∆RIB values	645

7.89.5	MSD	645

7.90	DC_1-42_n3-n28	646

7.90.1	Operating bands for DC	646

7.90.2	Channel bandwidths per operating band for DC	647

7.90.3	Co-existence studies	648

7.90.4	∆TIB and ∆RIB values	648

7.90.5	MSD	648

7.91	DC_1-42_n3-n77	649

7.91.1	Operating bands for DC	649

7.91.2	Channel bandwidths per operating band for DC	650

7.91.3	Co-existence studies	652

7.91.4	∆TIB and ∆RIB values	652

7.91.5	MSD	653

7.92	DC_8-42_n3-n28	653

7.92.1	Operating bands for DC	653

7.92.2	Channel bandwidths per operating band for DC	654

7.92.3	Co-existence studies	655

7.92.4	∆TIB and ∆RIB values	655

7.92.5	MSD	655

7.93	DC_8-42_n3-n77	656

7.93.1	Operating bands for DC	656

7.93.2	Channel bandwidths per operating band for DC	657

7.93.3	Co-existence studies	659

7.93.4	∆TIB and ∆RIB values	659

7.93.5	MSD	660

7.94	DC_1-21_n28-n77	660

7.94.1	Operating bands for DC	660

7.94.2	Configuration for DC	660

7.94.3	Co-existence studies	661

7.94.4	∆TIB and ∆RIB values	661

7.94.5	MSD	661

7.95	DC_1-21_n28-n78	661

7.95.1	Operating bands for DC	661

7.95.2	Configuration for DC	662

7.95.3	Co-existence studies	662

7.95.4	∆TIB and ∆RIB values	662

7.95.5	MSD	663

7.96	DC_1-21_n28-n79	663

7.96.1	Operating bands for DC	663

7.96.2	Configuration for DC	663

7.96.3	Co-existence studies	664

7.96.4	∆TIB and ∆RIB values	664

7.96.5	MSD	664

7.97	DC_1-3_n28-n79	664

7.97.1	Operating bands for DC	664

7.97.2	Configuration for DC	665

7.97.3	Co-existence studies	665

7.97.4	∆TIB and ∆RIB values	665

7.97.5	MSD	666

7.98	DC_3-21_n28-n77	666

7.98.1	Operating bands for DC	666

7.98.2	Configuration for DC	666

7.98.3	Co-existence studies	667

7.98.4	∆TIB and ∆RIB values	667

7.98.5	MSD	667

7.99	DC_3-21_n28-n78	667

7.99.1	Operating bands for DC	667

7.99.2	Configuration for DC	668

7.99.3	Co-existence studies	668

7.99.4	∆TIB and ∆RIB values	668

7.99.5	MSD	669

7.100	DC_3-21_n28-n79	669

7.100.1	Operating bands for DC	669

7.100.2	Configuration for DC	669

7.100.3	Co-existence studies	670

7.100.4	∆TIB and ∆RIB values	670

7.100.5	MSD	670

7.101	DC_1-8_n28-n78	670

7.101.1	 Operating bands for DC	670

7.101.2 	Configuration for DC	671

7.101.3	 Co-existence studies	671

7.101.4	 ∆TIB and ∆RIB values	672

7.101.5	MSD	672

7.102	DC_3-8_n28-n78	672

7.102.1	 Operating bands for DC	672

7.102.2 	Configuration for DC	673

7.102.3	 Co-existence studies	673

7.102.4	 ∆TIB and ∆RIB values	673

7.102.5	MSD	674

7.103	DC_7-8_n28-n78	674

7.103.1	 Operating bands for DC	674

7.103.2 	Configuration for DC	674

7.103.3	 Co-existence studies	675

7.103.4	 ∆TIB and ∆RIB values	675

7.103.5	MSD	675

7.104	DC_3-28_n1-n78	675

7.104.1	Operating bands for DC	675

7.104.2	Channel bandwidths per operating band for DC	676

7.104.3	Co-existence studies	677

7.104.4	∆TIB and ∆RIB values	677

7.104.5	MSD	677

7.105	DC_7-28_n1-n78	678

7.105.1	Operating bands for DC	678

7.105.2	Channel bandwidths per operating band for DC	678

7.105.3	Co-existence studies	679

7.105.4	∆TIB and ∆RIB values	679

7.105.5	MSD	679

7.106	DC_3-40_n1-n78	680

7.106.1	Operating bands for DC	680

7.106.2	Channel bandwidths per operating band for DC	680

7.106.3	Co-existence studies	681

7.106.4	∆TIB and ∆RIB values	681

7.106.5	MSD	681

7.107	DC_7-40_n1-n78	682

7.107.1	Operating bands for DC	682

7.107.2	Channel bandwidths per operating band for DC	682

7.107.3	Co-existence studies	683

7.107.4	∆TIB and ∆RIB values	683

7.107.5	MSD	683

7.108	DC_8-40_n1-n78	684

7.108.1	Operating bands for DC	684

7.108.2	Channel bandwidths per operating band for DC	684

7.108.3	Co-existence studies	685

7.108.4	∆TIB and ∆RIB values	685

7.108.5	MSD	685

7.109	DC_2-7_n25-n66	686

7.109.1	Operating bands for DC	686

7.109.2	Channel bandwidths per operating band for DC	687

7.109.3	Co-existence studies	688

7.109.4	∆TIB and ∆RIB values	688

7.109.5	MSD	688

7.110	DC_2-13_n25-n66	689

7.110.1	Operating bands for DC	689

7.110.2	Channel bandwidths per operating band for DC	689

7.110.3	Co-existence studies	690

7.110.4	∆TIB and ∆RIB values	690

7.110.5	MSD	690

7.111	DC_2-66_n25-n66	691

7.111.1	Operating bands for DC	691

7.111.2	Channel bandwidths per operating band for DC	691

7.111.3	Co-existence studies	692

7.111.4	∆TIB and ∆RIB values	692

7.111.5	MSD	692

7.112	DC_7-13_n25-n66	693

7.112.1	Operating bands for DC	693

7.112.2	Channel bandwidths per operating band for DC	694

7.112.3	Co-existence studies	694

7.112.4	∆TIB and ∆RIB values	694

7.112.5	MSD	695

7.113	DC_7-66_n25-n66	695

7.113.1	Operating bands for DC	695

7.113.2	Channel bandwidths per operating band for DC	696

7.113.3	Co-existence studies	696

7.113.4	∆TIB and ∆RIB values	697

7.113.5	MSD	697

7.114	DC_3-7_n3-n78	697

7.114.1	Operating bands for DC	697

7.114.2	Channel bandwidths per operating band for DC	698

7.114.3	Co-existence studies	698

7.114.4	∆TIB and ∆RIB values	698

7.114.5	MSD	699

7.115	DC_3-28_n3-n78	699

7.115.1	Operating bands for DC	699

7.115.2	Channel bandwidths per operating band for DC	700

7.115.3	Co-existence studies	700

7.115.4	∆TIB and ∆RIB values	700

7.115.5	MSD	701

7.116	DC_3-7_n1-n8, DC_3-3-7_n1-n8, DC_3-7-7_n1-n8, DC_3-3-7-7_n1-n8	702

7.116.1	Operating bands for DC	702

7.116.2	Configuration for DC	702

7.116.3	Co-existence studies	702

7.116.4	∆TIB and ∆RIB values	702

7.116.5	MSD	703

7.117	DC_3-7_n8-n78, DC_3-3-7_n8-n78, DC_3-7-7_n8-n78, DC_3-3-7-7_n8-n78	703

7.117.1	Operating bands for DC	703

7.117.2	Configuration for DC	703

7.117.3	Co-existence studies	704

7.117.4	∆TIB and ∆RIB values	704

7.117.5	MSD	704

7.118	DC_1-7_n8-n78	704

7.118.1	 Operating bands for DC	704

7.118.2 	Configuration for DC	705

7.118.3	 Co-existence studies	705

7.118.4	 ∆TIB and ∆RIB values	705

7.118.5	MSD	706

7.119	DC_1-20_n8-n78	706

7.119.1	 Operating bands for DC	706

7.119.2 	Configuration for DC	706

7.119.3	 Co-existence studies	707

7.119.4	 ∆TIB and ∆RIB values	707

7.119.5	MSD	707

7.120	DC_3-20_n8-n78	707

7.120.1	 Operating bands for DC	707

7.120.2 	Configuration for DC	708

7.120.3	 Co-existence studies	708

7.120.4	 ∆TIB and ∆RIB values	709

7.120.5	MSD	709

7.121	DC_7-20_n8-n78	709

7.121.1	 Operating bands for DC	709

7.121.2 	Configuration for DC	710

7.121.3	 Co-existence studies	710

7.121.4	 ∆TIB and ∆RIB values	710

7.121.5	MSD	711

7.122	DC_7-8_n1-n40	711

7.122.1	Operating bands for DC	711

7.122.2	Channel bandwidths per operating band for DC	712

7.122.3	Co-existence studies	712

7.122.4	∆TIB and ∆RIB values	712

7.122.5	MSD	713

7.123	DC_1-3_n8-n78	713

7.123.1	 Operating bands for DC	713

7.123.2 	Configuration for DC	713

7.123.3	 Co-existence studies	714

7.123.4	 ∆TIB and ∆RIB values	714

7.123.5	MSD	714

7.124	DC_3-8_n1-n40	715

7.124.1	Operating bands for DC	715

7.124.2	Channel bandwidths per operating band for DC	716

7.124.3	Co-existence studies	716

7.124.4	∆TIB and ∆RIB values	716

7.124.5	MSD	717

7.125	DC_1-8_n3-n79	718

7.125.1	Operating bands for DC	718

7.125.2	Channel bandwidths per operating band for DC	718

7.125.3	Co-existence studies	718

7.125.4	∆TIB and ∆RIB values	718

7.125.5	MSD	719

7.126	DC_1-8_n77-n79	719

7.126.1	Operating bands for DC	719

7.126.2	Channel bandwidths per operating band for DC	720

7.126.3	Co-existence studies	720

7.126.4	∆TIB and ∆RIB values	720

7.126.5	MSD	720

7.128.1	Operating bands for DC	722

7.128.2	Channel bandwidths per operating band for DC	722

7.128.3	Co-existence studies	722

7.128.4	∆TIB and ∆RIB values	722

7.128.5	MSD	722

7.129	DC_2-5_n2-n41	723

7.129.5	MSD	723

7.130 	DC_2-5_n2-n77	723

7.130.1	Operating bands for DC	723

7.130.2	Configuration for DC	723

7.130.3	Co-existence studies	723

7.130.4	∆TIB and ∆RIB values	723

7.130.5	MSD	724

7.131 	DC_2-5_n5-n77	724

7.131.1	Operating bands for DC	724

7.131.2	Configuration for DC	724

7.131.3	Co-existence studies	724

7.131.4	∆TIB and ∆RIB values	724

7.131.5	MSD	725

7.132 	DC_5-66_n66-n77	725

7.132.1	Operating bands for DC	725

7.132.2	Configuration for DC	725

7.132.3	Co-existence studies	725

7.132.4	∆TIB and ∆RIB values	725

7.132.5	MSD	726

7.133 	DC_2-5_n66-n77	726

7.133.1	Operating bands for DC	726

7.133.2	Configuration for DC	726

7.133.3	Co-existence studies	726

7.133.4	∆TIB and ∆RIB values	727

7.133.5	MSD	727

7.134 	DC_13-66_n5-n77	727

7.134.1	Operating bands for DC	727

7.134.2	Configuration for DC	727

7.134.3	Co-existence studies	728

7.134.4	∆TIB and ∆RIB values	728

7.134.5	MSD	728

7.135 	DC_2-13_n2-n77	728

7.135.1	Operating bands for DC	728

7.135.2	Configuration for DC	728

7.135.3	Co-existence studies	729

7.135.4	∆TIB and ∆RIB values	729

7.135.5	MSD	729

7.136  DC_2-13_n5-n77	729

7.136.1	Operating bands for DC	729

7.136.2	Configuration for DC	730

7.136.3	Co-existence studies	730

7.136.4	∆TIB and ∆RIB values	730

7.136.5	MSD	730

7.137  DC_2-66_n2-n77	730

7.137.1	Operating bands for DC	730

7.137.2	Configuration for DC	731

7.137.3	Co-existence studies	731

7.137.4	∆TIB and ∆RIB values	731

7.137.5	MSD	731

7.138  DC_5-66_n2-n77	732

7.138.1	Operating bands for DC	732

7.138.2	Configuration for DC	732

7.138.3	Co-existence studies	732

7.138.4	∆TIB and ∆RIB values	732

7.138.5	MSD	733

7.139  DC_5-66_n5-n77	733

7.139.1	Operating bands for DC	733

7.139.2	Configuration for DC	733

7.139.3	Co-existence studies	733

7.139.4	∆TIB and ∆RIB values	733

7.139.5	MSD	734

7.140  DC_3-8_n1-n28	734

7.140.1	 Operating bands for DC	734

7.140.2 	Configuration for DC	734

7.140.3	 Co-existence studies	734

7.140.4	 ∆TIB and ∆RIB values	735

7.140.5	MSD	735

7.141	DC_1-8_n28-n79	735

7.141.1	Operating bands for DC	735

7.141.2	Channel bandwidths per operating band for DC	736

7.141.3	Co-existence studies	736

7.141.4	∆TIB and ∆RIB values	736

7.141.5	MSD	736

7.142	DC_1-8_n77-n79	737

7.142.1	Operating bands for DC	737

7.142.2	Channel bandwidths per operating band for DC	737

7.142.3	Co-existence studies	737

7.142.4	∆TIB and ∆RIB values	737

7.142.5	MSD	738

7.143	DC_3-8_n77-n79	738

7.143.1	Operating bands for DC	738

7.143.2	Channel bandwidths per operating band for DC	738

7.143.3	Co-existence studies	739

7.143.4	∆TIB and ∆RIB values	739

7.143.5	MSD	739

7.144	DC_8-41_n3-n77	739

7.144.1	Operating bands for DC	739

7.144.2	Channel bandwidths per operating band for DC	740

7.144.3	Co-existence studies	740

7.144.4	∆TIB and ∆RIB values	740

7.144.5	MSD	741

7.145	DC_8-42_n1-n77	741

7.145.1	Operating bands for DC	741

7.145.2	Channel bandwidths per operating band for DC	742

7.145.3	Co-existence studies	742

7.145.4	∆TIB and ∆RIB values	742

7.145.5	MSD	742

7.146	DC_13-66_n5-n77	743

7.146.1	Operating bands for DC	743

7.146.2	Channel bandwidths per operating band for DC	743

7.146.3	Co-existence studies	743

7.146.4	∆TIB and ∆RIB values	743

7.146.5	MSD	743

7.147	DC_2-66_n5-n77	744

7.147.1	Operating bands for DC	744

7.147.2	Channel bandwidths per operating band for DC	744

7.147.3	Co-existence studies	744

7.147.4	∆TIB and ∆RIB values	744

7.147.5	MSD	744

7.148	DC_2-13_n66-n77	745

7.148.1	Operating bands for DC	745

7.148.2	Channel bandwidths per operating band for DC	745

7.148.3	Co-existence studies	745

7.148.4	∆TIB and ∆RIB values	745

7.148.5	MSD	745

7.149	DC_2-66_n66-n77	746

7.149.1	Operating bands for DC	746

7.149.2	Channel bandwidths per operating band for DC	746

7.149.3	Co-existence studies	746

7.149.4	∆TIB and ∆RIB values	746

7.149.5	MSD	746

7.150	 DC_3-32_n1-n28	747

7.150.1	 Operating bands for DC	747

7.150.2 	Configuration for DC	748

7.150.3	 Co-existence studies	749

7.150.4	 ∆TIB and ∆RIB values	749

7.151	 DC_20-32_n1-n28	749

7.151.1	 Operating bands for DC	749

7.151.2 	Configuration for DC	750

7.151.3	 Co-existence studies	751

7.151.4	 ∆TIB and ∆RIB values	751

7.152	 DC_1-38_n3-n78	751

7.152.1	Operating bands for DC	751

7.152.2	Configuration for DC	751

7.152.3	Co-existence studies	751

7.152.4	∆TIB and ∆RIB values	751

7.152.5	MSD	752

7.153	 DC_7-38_n3-n78	752

7.153.1	Operating bands for DC	752

7.153.2	Configuration for DC	752

7.153.3	Co-existence studies	752

7.153.4	∆TIB and ∆RIB values	752

7.153.5	MSD	753

7.154	 DC_20-38_n3-n78	753

7.154.1	Operating bands for DC	753

7.154.2	Configuration for DC	753

7.154.3	Co-existence studies	753

7.154.4	∆TIB and ∆RIB values	753

7.154.5	MSD	754

7.155	DC_2-66_n25-n66	754

7.155.1	Operating bands for DC	754

7.155.2	Channel bandwidths per operating band for DC	754

7.155.3	Co-existence studies	754

7.155.4	∆TIB and ∆RIB values	754

7.155.5	MSD	755

7.156	DC_2-5_n66-n78	755

7.156.1	Operating bands for DC	755

7.156.2	Channel bandwidths per operating band for DC	755

7.156.3	Co-existence studies	755

7.156.4	∆TIB and ∆RIB values	755

7.156.5	MSD	756

7.157	DC_2-5_ n2-n78	756

7.157.1	Operating bands for DC	756

7.157.2	Channel bandwidths per operating band for DC	756

7.157.3	Co-existence studies	756

7.157.4	∆TIB and ∆RIB values	756

7.157.5	MSD	757

7.158	DC_2-66_ n2-n78	757

7.158.1	Operating bands for DC	757

7.158.2	Channel bandwidths per operating band for DC	757

7.158.3	Co-existence studies	757

7.158.4	∆TIB and ∆RIB values	757

7.158.5	MSD	758

7.159	DC_5-66_ n2-n78	758

7.159.1	Operating bands for DC	758

7.159.2	Channel bandwidths per operating band for DC	758

7.159.3	Co-existence studies	758

7.159.4	∆TIB and ∆RIB values	758

7.159.5	MSD	759

7.160	DC_2-7_n2-n78	759

7.160.1	Operating bands for DC	759

7.160.2	Channel bandwidths per operating band for DC	759

7.160.3	Co-existence studies	759

7.160.4	∆TIB and ∆RIB values	759

7.160.5	MSD	760

7.161	DC_2-7_n71-n78	760

7.161.1	Operating bands for DC	760

7.161.2	Channel bandwidths per operating band for DC	760

7.161.3	Co-existence studies	760

7.161.4	∆TIB and ∆RIB values	760

7.161.5	MSD	761

7.162	DC_2-12_n2-n78	761

7.162.1	Operating bands for DC	761

7.162.2	Channel bandwidths per operating band for DC	761

7.162.3	Co-existence studies	761

7.162.4	∆TIB and ∆RIB values	761

7.162.5	MSD	762

7.163	DC_2-12_ n66-n78	762

7.163.1	Operating bands for DC	762

7.163.2	Channel bandwidths per operating band for DC	762

7.163.3	Co-existence studies	762

7.163.4	∆TIB and ∆RIB values	762

7.163.5	MSD	763

7.164	DC_2-66_n71-n78	763

7.164.1	Operating bands for DC	763

7.164.2	Channel bandwidths per operating band for DC	763

7.164.3	Co-existence studies	763

7.164.4	∆TIB and ∆RIB values	763

7.164.5	MSD	764

7.165	DC_2-71_n2-n78	764

7.165.1	Operating bands for DC	764

7.165.2	Channel bandwidths per operating band for DC	764

7.165.3	Co-existence studies	764

7.165.4	∆TIB and ∆RIB values	764

7.165.5	MSD	765

7.166	DC_2-71_n66-n78	765

7.166.1	Operating bands for DC	765

7.166.2	Channel bandwidths per operating band for DC	765

7.166.3	Co-existence studies	765

7.166.4	∆TIB and ∆RIB values	765

7.166.5	MSD	766

7.167	DC_5-7_n2-n78	766

7.167.1	Operating bands for DC	766

7.167.2	Channel bandwidths per operating band for DC	766

7.167.3	Co-existence studies	766

7.167.4	∆TIB and ∆RIB values	766

7.167.5	MSD	767

7.168	DC_5-7_n66-n78	767

7.168.1	Operating bands for DC	767

7.168.2	Channel bandwidths per operating band for DC	767

7.168.3	Co-existence studies	767

7.168.4	∆TIB and ∆RIB values	767

7.168.5	MSD	768

7.169	DC_7-12_n2-n78	768

7.169.1	Operating bands for DC	768

7.169.2	Channel bandwidths per operating band for DC	768

7.169.3	Co-existence studies	768

7.169.4	∆TIB and ∆RIB values	768

7.169.5	MSD	769

7.170	DC_7-12_ n66-n78	769

7.170.1	Operating bands for DC	769

7.170.2	Channel bandwidths per operating band for DC	769

7.170.3	Co-existence studies	769

7.170.4	∆TIB and ∆RIB values	769

7.170.5	MSD	770

7.171	DC_7-66_n2-n78	770

7.171.1	Operating bands for DC	770

7.171.2	Channel bandwidths per operating band for DC	770

7.171.3	Co-existence studies	770

7.171.4	∆TIB and ∆RIB values	770

7.171.5	MSD	771

7.172	DC_7-66_n71-n78	771

7.172.1	Operating bands for DC	771

7.172.2	Channel bandwidths per operating band for DC	771

7.172.3	Co-existence studies	771

7.172.4	∆TIB and ∆RIB values	771

7.172.5	MSD	772

7.173	DC_72-71_n2-n78	772

7.173.1	Operating bands for DC	772

7.173.2	Channel bandwidths per operating band for DC	772

7.173.3	Co-existence studies	772

7.173.4	∆TIB and ∆RIB values	772

7.173.5	MSD	773

7.174	DC_7-71_n66-n78	773

7.174.1	Operating bands for DC	773

7.174.2	Channel bandwidths per operating band for DC	773

7.174.3	Co-existence studies	773

7.174.4	∆TIB and ∆RIB values	773

7.174.5	MSD	774

7.175	DC_12-66_ n2-n78	774

7.175.1	Operating bands for DC	774

7.175.2	Channel bandwidths per operating band for DC	774

7.175.3	Co-existence studies	774

7.175.4	∆TIB and ∆RIB values	774

7.175.5	MSD	775

7.176	DC_66-71_n2-n78	775

7.176.1	Operating bands for DC	775

7.176.2	Channel bandwidths per operating band for DC	775

7.176.3	Co-existence studies	775

7.176.4	∆TIB and ∆RIB values	775

7.176.5	MSD	776

7.177	DC_1-3_n28-n75	776

7.177.1	 Operating bands for DC	776

7.177.2 	Configuration for DC	777

7.177.3	 Co-existence studies	778

7.177.4	 ∆TIB and ∆RIB values	778

7.178	DC_3-20_n28-n75	778

7.178.1	 Operating bands for DC	778

7.178.2 	Configuration for DC	779

7.178.3	 Co-existence studies	780

7.178.4	 ∆TIB and ∆RIB values	780

7.179	DC_1-20_n28-n75	780

7.179.1	 Operating bands for DC	780

7.179.2 	Configuration for DC	781

7.179.3	 Co-existence studies	781

7.179.4	 ∆TIB and ∆RIB values	781

7.180	DC_1-7_n3-n38	782

7.180.1	Operating bands for DC	782

7.180.2	Channel bandwidths per operating band for DC	782

7.180.3	Co-existence studies	783

7.180.4	∆TIB and ∆RIB values	783

7.180.5	MSD	783

7.181	DC_7-20_n3-n38	784

7.181.1	Operating bands for DC	784

7.181.2	Channel bandwidths per operating band for DC	784

7.181.3	Co-existence studies	784

7.181.4	∆TIB and ∆RIB values	785

7.181.5	MSD	785

7.182	DC_1-11_n3-n79	785

7.182.1	Operating bands for DC	785

7.182.2	Channel bandwidths per operating band for DC	786

7.182.3	Co-existence studies	786

7.182.4	∆TIB and ∆RIB values	786

7.182.5	MSD	787

7.183	DC_1-11_n77-n79	787

7.183.1	Operating bands for DC	787

7.183.2	Channel bandwidths per operating band for DC	788

7.183.3	Co-existence studies	788

7.183.4	∆TIB and ∆RIB values	788

7.183.5	MSD	789

7.184	DC_8-11_n3-n79	789

7.184.1	Operating bands for DC	789

7.184.2	Channel bandwidths per operating band for DC	790

7.184.3	Co-existence studies	790

7.184.4	∆TIB and ∆RIB values	790

7.184.5	MSD	791

7.185	DC_8-11_n77-n79	791

7.185.1	Operating bands for DC	791

7.185.2	Channel bandwidths per operating band for DC	792

7.185.3	Co-existence studies	792

7.185.4	∆TIB and ∆RIB values	792

7.185.5	MSD	793

7.186	DC_8-41_n1-n77	793

7.186.1	Operating bands for DC	793

7.186.2	Channel bandwidths per operating band for DC	794

7.186.3	Co-existence studies	794

7.186.4	∆TIB and ∆RIB values	794

7.186.5	MSD	795

7.187	DC_8-42_n1-n3	795

7.187.1	Operating bands for DC	795

7.187.2	Channel bandwidths per operating band for DC	796

7.187.3	Co-existence studies	796

7.187.4	∆TIB and ∆RIB values	796

7.187.5	MSD	797

7.188	DC_8-11_n1-n77	797

7.188.1	Operating bands for DC	797

7.188.2	Channel bandwidths per operating band for DC	798

7.188.3	Co-existence studies	798

7.188.4	∆TIB and ∆RIB values	798

7.188.5	MSD	799

7.189	DC_8-41_n1-n3	799

7.189.1	Operating bands for DC	799

7.189.2	Channel bandwidths per operating band for DC	800

7.189.3	Co-existence studies	800

7.189.4	∆TIB and ∆RIB values	800

7.189.5	MSD	801

7.190	DC_1-20_n7-n78	801

7.190.1	Configurations for DC	801

7.190.2	Channel bandwidths per operating band for DC	802

7.190.3	Coexistence Studies	802

7.190.4	∆TIB and ∆RIB values	802

7.190.5	Reference sensitivity exceptions	803

7.191	DC_3-8_n40-n79	803

7.191.1	Operating bands for DC	803

7.191.2	Configuration for DC	803

7.191.3	Co-existence studies	803

7.191.4	∆TIB and ∆RIB values	803

7.191.5	MSD	804

7.192	DC_n77-n257_1-3	804

7.192.1	 Operating bands for DC	804

7.192.2	 Channel bandwidths per operating band for DC	805

7.192.3	 Co-existence studies	805

7.192.4	∆TIB and ∆RIB values	806

7.192.5	MSD	806

8	Dual Connectivity band combinations of LTE 3 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	807

8.1	DC_1-3-8_n28-n77	807

8.1.1	Operating bands for DC	807

8.1.2	Channel bandwidths per operating band for DC	808

8.1.3	Co-existence studies	808

8.1.4	∆TIB and ∆RIB values	808

8.1.5	MSD	809

8.2	DC_3-7-8_n40-n78	809

8.2.1	Operating bands for DC	809

8.2.2	Configuration for DC	810

8.2.3	Co-existence studies	810

8.2.4	∆TIB and ∆RIB values	810

8.2.5	MSD	811

8.3	DC_3-7-28_n1-n40	811

8.3.1	Operating bands for DC	811

8.3.2	Configuration for DC	811

8.3.3	Co-existence studies	811

8.3.4	∆TIB and ∆RIB values	812

8.3.5	MSD	812

8.4	DC_1-3-18_n3-n77	812

8.4.1	Operating bands for DC	812

8.4.2	Channel bandwidths per operating band for DC	813

8.4.3	Co-existence studies	813

8.4.4	∆TIB and ∆RIB values	813

8.4.5	MSD	814

8.5	DC_1-3-18_n3-n78	814

8.5.1	Operating bands for DC	814

8.5.2	Channel bandwidths per operating band for DC	814

8.5.3	Co-existence studies	815

8.5.4	∆TIB and ∆RIB values	815

8.5.5	MSD	815

8.6	DC_1-3-41_n3-n41	816

8.6.1	Operating bands for DC	816

8.6.2	Channel bandwidths per operating band for DC	816

8.6.3	Co-existence studies	816

8.6.4	∆TIB and ∆RIB values	816

8.6.5	MSD	817

8.7	DC_1-3-41_n3-n77	817

8.7.1	Operating bands for DC	817

8.7.2	Channel bandwidths per operating band for DC	818

8.7.3	Co-existence studies	818

8.7.4	∆TIB and ∆RIB values	818

8.7.5	MSD	819

8.8	DC_1-3-41_n3-n78	819

8.8.1	Operating bands for DC	819

8.8.2	Channel bandwidths per operating band for DC	820

8.8.3	Co-existence studies	820

8.8.4	∆TIB and ∆RIB values	820

8.8.5	MSD	821

8.9	DC_1-3-41_n41-n77	821

8.9.1	Operating bands for DC	821

8.9.2	Channel bandwidths per operating band for DC	821

8.9.3	Co-existence studies	822

8.9.4	∆TIB and ∆RIB values	822

8.9.5	MSD	822

8.10	DC_1-3-41_n41-n78	823

8.10.1	Operating bands for DC	823

8.10.2	Channel bandwidths per operating band for DC	823

8.10.3	Co-existence studies	823

8.10.4	∆TIB and ∆RIB values	823

8.10.5	MSD	824

8.11	DC_1-8-42_n28-n77	824

8.11.1	Operating bands for DC	824

8.11.2	Channel bandwidths per operating band for DC	825

8.11.3	Co-existence studies	827

8.11.4	∆TIB and ∆RIB values	827

8.11.5	MSD	828

8.12	DC_1-3-18_n28-n77	828

8.12.1	Operating bands for DC	828

8.12.2	Channel bandwidths per operating band for DC	829

8.12.3	Co-existence studies	830

8.12.4	∆TIB and ∆RIB values	830

8.12.5	MSD	830

8.13	DC_1-3-18_n28-n78	831

8.13.1	Operating bands for DC	831

8.13.2	Channel bandwidths per operating band for DC	832

8.13.3	Co-existence studies	833

8.13.4	∆TIB and ∆RIB values	833

8.13.5	MSD	833

8.14	DC_1-3-41_n28-n41	834

8.14.1	Operating bands for DC	834

8.14.2	Channel bandwidths per operating band for DC	835

8.14.3	Co-existence studies	835

8.14.4	∆TIB and ∆RIB values	835

8.14.5	MSD	836

8.15	DC_3-19-42_n1-n77	836

8.15.1	Operating bands for DC	836

8.15.2	Configuration for DC	837

8.15.3	Co-existence studies	837

8.15.4	∆TIB and ∆RIB values	837

8.15.5	MSD	838

8.16	DC_3-19-42_n1-n78	838

8.16.1	Operating bands for DC	838

8.16.2	Configuration for DC	839

8.16.3	Co-existence studies	839

8.16.4	∆TIB and ∆RIB values	839

8.16.5	MSD	840

8.17	DC_3-19-42_n1-n79	840

8.17.1	Operating bands for DC	840

8.17.2	Configuration for DC	841

8.17.3	Co-existence studies	841

8.17.4	∆TIB and ∆RIB values	841

8.17.5	MSD	842

8.18	DC_3-21-42_n1-n77	842

8.18.1	Operating bands for DC	842

8.18.2	Configuration for DC	843

8.18.3	Co-existence studies	843

8.18.4	∆TIB and ∆RIB values	843

8.18.5	MSD	844

8.19	DC_3-21-42_n1-n78	844

8.19.1	Operating bands for DC	844

8.19.2	Configuration for DC	845

8.19.3	Co-existence studies	845

8.19.4	∆TIB and ∆RIB values	845

8.19.5	MSD	846

8.20	DC_3-21-42_n1-n79	846

8.20.1	Operating bands for DC	846

8.20.2	Configuration for DC	847

8.20.3	Co-existence studies	847

8.20.4	∆TIB and ∆RIB values	847

8.20.5	MSD	848

8.21	DC_19-21-42_n1-n77	848

8.21.1	Operating bands for DC	848

8.21.2	Configuration for DC	849

8.21.3	Co-existence studies	849

8.21.4	∆TIB and ∆RIB values	849

8.21.5	MSD	850

8.22	DC_19-21-42_n1-n78	850

8.22.1	Operating bands for DC	850

8.22.2	Configuration for DC	851

8.22.3	Co-existence studies	851

8.22.4	∆TIB and ∆RIB values	851

8.22.5	MSD	852

8.23	DC_19-21-42_n1-n79	852

8.23.1	Operating bands for DC	852

8.23.2	Configuration for DC	853

8.23.3	Co-existence studies	853

8.23.4	∆TIB and ∆RIB values	853

8.23.5	MSD	854

8.24	DC_3-7-20_n1-n78	854

8.24.1	Operating bands for DC	854

8.24.2	Channel bandwidths per operating band for DC	855

8.24.3	Co-existence studies	855

8.24.4	∆TIB and ∆RIB values	855

8.24.5	MSD	856

8.25	DC_3-7-28_n40-n78	856

8.25.1	Operating bands for DC	856

8.25.2	Channel bandwidths per operating band for DC	857

8.25.3	Co-existence studies	858

8.25.4	∆TIB and ∆RIB values	858

8.25.5	MSD	859

8.26	DC_1-3-7_n40-n78	859

8.26.1	Operating bands for DC	859

8.26.2	Channel bandwidths per operating band for DC	860

8.26.3	Co-existence studies	861

8.26.4	∆TIB and ∆RIB values	861

8.26.5	MSD	862

8.27	DC_1-7-28_n40-n78	862

8.27.1	Operating bands for DC	862

8.27.2	Channel bandwidths per operating band for DC	863

8.27.3	Co-existence studies	864

8.27.4	∆TIB and ∆RIB values	864

8.27.5	MSD	865

8.28	DC_3-7-8_n78-n258	865

8.28.1	Operating bands for DC	865

8.28.2	Channel bandwidths per operating band for DC	866

8.28.3	Co-existence studies	866

8.28.4	∆TIB and ∆RIB values	866

8.28.5	MSD	866

8.29	DC_3-7-8_n40A-n258	867

8.29.1	Operating bands for DC	867

8.29.2	Channel bandwidths per operating band for DC	868

8.29.3	Co-existence studies	868

8.29.4	∆TIB and ∆RIB values	868

8.29.5	MSD	868

8.30	DC_1-8-11_n3-n28	869

8.30.1	Operating bands for DC	869

8.30.2	Channel bandwidths per operating band for DC	870

8.30.3	Co-existence studies	870

8.30.4	∆TIB and ∆RIB values	870

8.30.5	MSD	871

8.31	DC_1-8-11_n3-n77	871

8.31.1	Operating bands for DC	871

8.31.2	Channel bandwidths per operating band for DC	872

8.31.3	Co-existence studies	873

8.31.4	∆TIB and ∆RIB values	873

8.31.5	MSD	873

8.32	DC_1-3-11_n28-n77	874

8.32.1	Operating bands for DC	874

8.32.2	Channel bandwidths per operating band for DC	875

8.32.3	Co-existence studies	876

8.32.4	∆TIB and ∆RIB values	876

8.32.5	MSD	876

8.33	DC_1-8-11_n28-n77	877

8.33.1	Operating bands for DC	877

8.33.2	Channel bandwidths per operating band for DC	878

8.33.3	Co-existence studies	879

8.33.4	∆TIB and ∆RIB values	879

8.33.5	MSD	879

8.34	DC_1-8-42_n3-n28	880

8.34.1	Operating bands for DC	880

8.34.2	Channel bandwidths per operating band for DC	881

8.34.3	Co-existence studies	883

8.34.4	∆TIB and ∆RIB values	883

8.34.5	MSD	883

8.35	DC_1-8-42_n3-n77	884

8.35.1	Operating bands for DC	884

8.35.2	Channel bandwidths per operating band for DC	885

8.35.3	Co-existence studies	887

8.35.4	∆TIB and ∆RIB values	887

8.35.5	MSD	888

8.36	DC_3-8-11_n28-n77	888

8.36.1	Operating bands for DC	888

8.36.2	Channel bandwidths per operating band for DC	889

8.36.3	Co-existence studies	890

8.36.4	∆TIB and ∆RIB values	890

8.36.5	MSD	890

8.37	DC_1-3-42_n28-n77	891

8.37.1	Operating bands for DC	891

8.37.2	Channel bandwidths per operating band for DC	892

8.37.3	Co-existence studies	894

8.37.4	∆TIB and ∆RIB values	894

8.37.5	MSD	895

8.38	 DC_1-7-8_n28-n78	895

8.38.1	 Operating bands for DC	895

8.38.2 	Configuration for DC	896

8.38.3	 Co-existence studies	896

8.38.4	 ∆TIB and ∆RIB values	896

8.38.5	MSD	897

8.39	 DC_3-7-8_n28-n78	897

8.39.1	 Operating bands for DC	897

8.39.2 	Configuration for DC	897

8.39.3	 Co-existence studies	898

8.39.4	 ∆TIB and ∆RIB values	898

8.39.5	MSD	898

8.40	DC_3-7-28_n1-n78	899

8.40.1	Operating bands for DC	899

8.40.2	Channel bandwidths per operating band for DC	899

8.40.3	Co-existence studies	900

8.40.4	∆TIB and ∆RIB values	900

8.40.5	MSD	901

8.41	DC_1-7-28_n3-n78	901

8.41.1	Operating bands for DC	901

8.41.2	Channel bandwidths per operating band for DC	902

8.41.3	Co-existence studies	903

8.41.4	∆TIB and ∆RIB values	903

8.41.5	MSD	904

8.42	DC_3-7-40_n1-n78	904

8.42.1	Operating bands for DC	904

8.42.2	Channel bandwidths per operating band for DC	905

8.42.3	Co-existence studies	905

8.42.4	∆TIB and ∆RIB values	905

8.42.5	MSD	906

8.43	DC_3-8-40_n1-n78	906

8.43.1	Operating bands for DC	906

8.43.2	Channel bandwidths per operating band for DC	907

8.43.3	Co-existence studies	907

8.43.4	∆TIB and ∆RIB values	907

8.43.5	MSD	908

8.44	DC_7-8-40_n1-n78	908

8.44.1	Operating bands for DC	908

8.44.2	Channel bandwidths per operating band for DC	909

8.44.3	Co-existence studies	909

8.44.4	∆TIB and ∆RIB values	910

8.44.5	MSD	910

8.45	DC_1-3-18_n3-n41	911

8.45.1	Operating bands for DC	911

8.45.2	Channel bandwidths per operating band for DC	912

8.45.3	Co-existence studies	912

8.45.4	∆TIB and ∆RIB values	912

8.45.5	MSD	913

8.46	DC_1-3-18_n28-n41	914

8.46.1	Operating bands for DC	914

8.46.2	Channel bandwidths per operating band for DC	915

8.46.3	Co-existence studies	915

8.46.4	∆TIB and ∆RIB values	915

8.46.5	MSD	916

8.47	DC_1-3-18_n41-n77	917

8.47.1	Operating bands for DC	917

8.47.2	Channel bandwidths per operating band for DC	918

8.47.3	Co-existence studies	919

8.47.4	∆TIB and ∆RIB values	919

8.47.5	MSD	919

8.48	DC_1-3-18_n41-n78	920

8.48.1	Operating bands for DC	920

8.48.2	Channel bandwidths per operating band for DC	921

8.48.3	Co-existence studies	922

8.48.4	∆TIB and ∆RIB values	922

8.48.5	MSD	922

8.49	DC_1-3-8_n28-n78	923

8.49.1	 Operating bands for DC	923

8.49.2 	Configuration for DC	923

8.49.3	 Co-existence studies	923

8.49.4	 ∆TIB and ∆RIB values	924

8.49.5	MSD	924

8.50	DC_1-3-20_n7-n78	924

8.50.1	Operating bands for DC	924

8.50.2	Channel bandwidths per operating band for DC	925

8.50.3	Co-existence studies	925

8.50.4	∆TIB and ∆RIB values	925

8.50.5	MSD	926

8.51	DC_2-7-13_n25-n66	926

8.51.1	Operating bands for DC	926

8.51.2	Channel bandwidths per operating band for DC	927

8.51.3	Co-existence studies	928

8.51.4	∆TIB and ∆RIB values	928

8.51.5	MSD	928

8.52	DC_2-7-66_n25-n66	929

8.52.1	Operating bands for DC	929

8.52.2	Channel bandwidths per operating band for DC	930

8.52.3	Co-existence studies	931

8.52.4	∆TIB and ∆RIB values	931

8.52.5	MSD	931

8.53	DC_1-3-7_n3-n78	932

8.53.1	Operating bands for DC	932

8.53.2	Channel bandwidths per operating band for DC	933

8.53.3	Co-existence studies	933

8.53.4	∆TIB and ∆RIB values	933

8.53.5	MSD	934

8.54	DC_1-3-28_n3-n78	934

8.54.1	Operating bands for DC	934

8.54.2	Channel bandwidths per operating band for DC	935

8.54.3	Co-existence studies	935

8.54.4	∆TIB and ∆RIB values	935

8.54.5	MSD	936

8.55	DC_3-7-28_n3-n78	936

8.55.1	Operating bands for DC	936

8.55.2	Channel bandwidths per operating band for DC	937

8.55.3	Co-existence studies	937

8.55.4	∆TIB and ∆RIB values	937

8.55.5	MSD	938

8.56	DC_1-3-7_n8-n78	938

8.56.1	 Operating bands for DC	938

8.56.2 	Configuration for DC	939

8.56.3	 Co-existence studies	939

8.56.4	 ∆TIB and ∆RIB values	939

8.56.5	MSD	940

8.57	DC_1-3-20_n8-n78	940

8.57.1	 Operating bands for DC	940

8.57.2 	Configuration for DC	940

8.57.3	 Co-existence studies	941

8.57.4	 ∆TIB and ∆RIB values	941

8.57.5	MSD	941

8.58	DC_1-7-20_n8-n78	942

8.58.1	 Operating bands for DC	942

8.58.2 	Configuration for DC	942

8.58.3	 Co-existence studies	942

8.58.4	 ∆TIB and ∆RIB values	943

8.58.5	MSD	943

8.59	DC_3-7-20_n8-n78	943

8.59.1	 Operating bands for DC	943

8.59.2 	Configuration for DC	944

8.59.3	 Co-existence studies	944

8.59.4	 ∆TIB and ∆RIB values	944

8.59.5	MSD	945

8.60	DC_3-7-8_n1-n40	945

8.60.1	Operating bands for DC	945

8.60.2	Channel bandwidths per operating band for DC	946

8.60.3	Co-existence studies	946

8.60.4	∆TIB and ∆RIB values	946

8.60.5	MSD	947

8.61	DC_2-13-66_n5-n77	948

8.61.1	Operating bands for DC	948

8.61.2	Configuration for DC	948

8.61.3	Co-existence studies	948

8.61.4	∆TIB and ∆RIB values	948

8.61.5	MSD	949

8.62	DC_2-13-66_n66-n77	949

8.62.1	Operating bands for DC	949

8.62.2	Configuration for DC	949

8.62.3	Co-existence studies	949

8.62.4	∆TIB and ∆RIB values	949

8.62.5	MSD	950

8.63	DC_2-13-66_n2-n77	950

8.63.1	Operating bands for DC	950

8.63.2	Configuration for DC	950

8.63.3	Co-existence studies	950

8.63.4	∆TIB and ∆RIB values	951

8.63.5	MSD	951

8.64	DC_2-5-66_n5-n77	951

8.64.1	Operating bands for DC	951

8.64.2	Configuration for DC	951

8.64.3	Co-existence studies	952

8.64.4	∆TIB and ∆RIB values	952

8.64.5	MSD	952

8.65	DC_2-5-66_n2-n77	952

8.65.1	Operating bands for DC	952

8.65.2	Configuration for DC	953

8.65.3	Co-existence studies	953

8.65.4	∆TIB and ∆RIB values	953

8.65.5	MSD	953

8.66.1	Operating bands for DC	954

8.66.2	Channel bandwidths per operating band for DC	954

8.66.3	Co-existence studies	954

8.66.4	∆TIB and ∆RIB values	954

8.66.5	MSD	955

8.67	DC_3-20-32_n1-n28	955

8.67.1	 Operating bands for DC	955

8.67.2 	Configuration for DC	956

8.67.3	 Co-existence studies	957

8.67.4	 ∆TIB and ∆RIB values	957

8.68	DC_1-3-20_n28-n75	957

8.68.1	 Operating bands for DC	957

8.68.2 	Configuration for DC	958

8.68.3	 Co-existence studies	958

8.68.4	 ∆TIB and ∆RIB values	959

8.69	DC_2-5-66_n66-n77	959

8.69.1	Operating bands for DC	959

8.69.2	Channel bandwidths per operating band for DC	960

8.69.3	Co-existence studies	960

8.69.4	∆TIB and ∆RIB values	960

8.69.5	MSD	961

8.70	DC_2-2-13-66_n66-n77	961

8.70.1	Operating bands for DC	961

8.70.2	Channel bandwidths per operating band for DC	961

8.70.3	Co-existence studies	962

8.70.4	∆TIB and ∆RIB values	962

8.70.5	MSD	962

8.71	DC_1-7-20_n3-n38	963

8.71.1	Operating bands for DC	963

8.71.2	Channel bandwidths per operating band for DC	963

8.71.3	Co-existence studies	964

8.71.4	∆TIB and ∆RIB values	964

8.71.5	MSD	964

8.72	DC_1-3-8_n77-n79	965

8.72.1	Operating bands for DC	965

8.72.2	Channel bandwidths per operating band for DC	965

8.72.3	Co-existence studies	966

8.72.4	∆TIB and ∆RIB values	966

8.72.5	MSD	966

8.73	DC_7-20-38_n3-n78	966

8.73.1	Operating bands for DC	966

8.73.2	Configuration for DC	967

8.73.3	Co-existence studies	967

8.73.4	∆TIB and ∆RIB values	967

8.73.5	MSD	968

8.74	DC_1-7-38_n3-n78	968

8.74.1	Operating bands for DC	968

8.74.2	Configuration for DC	968

8.74.3	Co-existence studies	969

8.74.4	∆TIB and ∆RIB values	969

8.74.5	MSD	969

8.75	DC_1-7-20_n3-n78	969

8.75.1	Operating bands for DC	969

8.75.2	Configuration for DC	970

8.75.3	Co-existence studies	970

8.75.4	∆TIB and ∆RIB values	970

8.75.5	MSD	971

8.76	DC_1-8-11_n3-n79	971

8.76.1	Operating bands for DC	971

8.76.2	Channel bandwidths per operating band for DC	972

8.76.3	Co-existence studies	972

8.76.4	∆TIB and ∆RIB values	972

8.76.5	MSD	973

8.77	DC_1-8-11_n77-n79	973

8.77.1	Operating bands for DC	973

8.77.2	Channel bandwidths per operating band for DC	974

8.77.3	Co-existence studies	974

8.77.4	∆TIB and ∆RIB values	974

8.77.5	MSD	975

9	Dual Connectivity band combinations of LTE 4 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	976

9.1	DC_1-3-7-28_n40-n78	976

9.1.1	Operating bands for DC	976

9.1.2	Channel bandwidths per operating band for DC	976

9.1.3	Co-existence studies	977

9.1.4	∆TIB and ∆RIB values	977

9.1.5	MSD	978

9.2	DC_1-3-8-11_n28-n77	978

9.2.1	Operating bands for DC	978

9.2.2	Channel bandwidths per operating band for DC	979

9.2.3	Co-existence studies	981

9.2.4	∆TIB and ∆RIB values	981

9.2.5	MSD	981

9.3	DC_1-3-7-8_n28-n78	982

9.3.1	 Operating bands for DC	982

9.3.2 	Configuration for DC	982

9.3.3	 Co-existence studies	983

9.3.4	 ∆TIB and ∆RIB values	983

9.3.5	MSD	983

9.4	DC_3-7-8-40_n1-n78	983

9.4.1	Operating bands for DC	983

9.4.2	Channel bandwidths per operating band for DC	984

9.4.3	Co-existence studies	985

9.4.4	∆TIB and ∆RIB values	985

9.4.5	MSD	985

9.5	DC_1-3-7-28_n78-n257	986

9.5.1	Operating bands for DC	986

9.5.2	Channel bandwidths per operating band for DC	987

9.5.3	Co-existence studies	987

9.5.4	∆TIB and ∆RIB values	987

9.5.5	MSD	987

9.6	DC_1-3-7-28_n3-n78	988

9.6.1	Operating bands for DC	988

9.6.2	Channel bandwidths per operating band for DC	989

9.6.3	Co-existence studies	989

9.6.4	∆TIB and ∆RIB values	989

9.6.5	MSD	990

9.7	 DC_1-3-7-20_n8-n78	990

9.7.1	 Operating bands for DC	990

9.7.2 	Configuration for DC	991

9.7.3	 Co-existence studies	991

9.7.4	 ∆TIB and ∆RIB values	991

9.7.5	MSD	992

9.8	 DC_1-3-7-20_n38-n78	992

9.8.1 	Operating bands for DC	992

9.8.2 	Configuration for DC	993

9.8.3	 Co-existence studies	993

9.8.4	 ∆TIB and ∆RIB values	993

9.8.5	MSD	993

9.9	 DC_1-7-20-38_n3-n78	994

9.9.1 	Configuration for DC	994

9.9.2	 Co-existence studies	994

9.9.3	 ∆TIB and ∆RIB values	994

9.9.4	MSD	995

Annex A: Change history	996

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report for Dual connectivity (DC) band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL in Rel-17 as shown in the WID for Rel-17.

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[4]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[5]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Background

The present document is a technical report for DC band combinations of LTE x bands (x=1,2,3,4) inter-band CA (x DL/1UL) + inter-band NR 2 bands (2DL/1UL) under Rel-17 time frame. It covers both the UE and BS side. The document is divided in two different parts:

- 	General part: this part covers BS and UE specific which is band combination independent.

- 	Specific band combination part:  this part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs.

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	Dual Connectivity band combinations of LTE x bands DL/1UL(x=1,2,3,4) + NR 2 bands DL/1UL: General Part

## 5.1	UE RF architectures

RAN4 consider same example RF architecture which are described in section 5.1 in TR37.863-01-01.  One different point is that LTE and NR modemchip can be used one chip solutions from rel-16.

To support multiple component carriers in LTE and NR, RAN4 consider specific RF components such as Diplexer, Triplexer, Quadplexer and Hexaplexer to derive self-interference problems according to the LTE x bands DL/1UL(x=1,2,3,4) + NR 2 band DL/1UL DC band combinations.

## 5.2	General treatment of ∆TIB and ∆RIB values

For the LTE x bands DL/1UL(x=1,2,3,4) + NR 2 band DL/1UL DC band combinations, RAN4 should consider to reuse agreed additional insertion losses for all DC band combinations in rel-16 when new RF components are not introduced to support this basket WI. If the new RF components are introduced, then more detail decription will be captured in some specific DC band combinations.

## 5.3	Summary of self-interference analysis

Table 5.3-1 summarizes the EN- DC band combinations with self-interference problems for 3DL/2UL DC operation.

Table 5.3-1: Summary of Self-interference analysis for LTE 1band & NR 2 bands DL/1 band /UL DC operation

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_8_n28-n78 | DC_8A-n28 | 4th harmonic into n78 5th harmonic into n78 | 4th & 5th IMDs into n78 | - | - 4th harmonic problem will be solved in DC_8A-n78A - 5th harmonic problem will be solved in DC_28A-n78A - Same as DC_8_n28-n77A with 2UL_DC_8A_n28A |
|  | DC_8A_n78A | - | 4th IMD into n28 | - | - Same as DC_8_n28-n77A with 2UL_DC_8A_n77A |
| DC_(n)41AA-n78 DC_(n)41CA-n78 DC_(n)41DA-n78 | DC_41A_n78A | - | 4th IMD into n41 | - | B41 and n41 will be operated by synchronous operation. So there is no IMD problem |
| DC_40A_n1-n78 DC_40C_n1-n78 | DC_40A_n1A DC_40C_n1A |  | 4th IMD & 5th IMD into n78 | - | - Same as DC_1A_n40A-n78A with 2UL_DC_1A_n40A |
|  | DC_40A_n78A DC_40C_n78A |  | 4th IMD into n1 | - | -7.6dB by IMD4 |
| DC_3C_n1-n78 | DC_3A_n1A | 2nd harmonic from 3 into n78 | 2nd & 4th IMDs into n78 | Yes | - Harmonic problems was already covered in DC_3A_n78A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_3A_n78A | - | 5th IMD into n1 | - | - Same as DC_3A_n1A-n78A with 2UL_DC_3A_n78A |
| DC_1_n28-n41 | DC_1A_n28A | - | 2nd & 4th IMDs into n41 |  | - Same as DC_1A-41A_n28A with 2UL_DC_1A_n28A |
|  | DC_1A_n41A |  | 2nd & 5th IMDs into n28 |  | - 29.3 dB by IMD2 - 4.5dB by IMD5 |
| DC_3_n28-n41 | DC_3A_n28A |  | 2nd & 3rd IMDs into n41 |  | - Same as DC_3A-41A_n28A with 2UL_DC_3A_n28A |
|  | DC_3A_n41A |  | 2nd & 3rd IMDs into n28 |  | - Same as DC_3A-28A_n41A with 2UL_DC_3A_n41A |
| DC_28_n1-n40 | DC_28A_n1A | - | 4th IMD into n40 | - | - Same as DC_1A_n28A-n40A with 2UL_DC_1A_n28A |
|  | DC_28A_n40A | 3rd harmonic from B28 into n1 | - | - | 3rd harmonic problem was covered in lower DC or CA band combinations |
| DC_7_n40-n78 | DC_7A_n40A | - | - | - | No issue |
|  | DC_7A_n78A |  | 4th IMD into n40 |  | - 9.2 dB by 4th IMD |
| DC_8_n40-n78 | DC_8A_n40A | 4th harmonic from B8 into n78 | 2nd & 3rd IMDs into n78 |  | - 28.7dB by IMD2 |
|  | DC_8A_n78A |  | 2nd IMD into n40 |  | - 27.3 dB by IMD2 |
| DC_1_n3-n77 DC_1_n3-n77(2A) DC_n3-n77_1 DC_n3-n77(2A)_1 | DC_1A_n3A or DC_n3A_1A | 2nd harmonic from B1 or n3 into n77 | 2nd & 4th IMDs into n77 | Yes | - Harmonic problems already covered in DC_n3A-n78A. - Same as IMD issues in DC_1A_n3A-n78A - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_1A_n77A or DC_n77A_1A | - | 2nd , 4th & 5th IMDs into n3 | - | - Same as IMD issues in DC_1A_n3A-n78A |
| DC_8_n3-n77A DC_8_n3-n77(2A) DC_n3-n77A(2A)_8 | DC_8A_n3A DC_n3A_8A | 2nd harmonic from n3 into n77 4th harmonic from B8 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems already covered in CA_3A_n77A and DC_8A_n77A - Same as DC_3A_n8A-n78A with 2UL_DC_3A_n8A |
|  | DC_8A_n77A DC_n77A_8A | 2nd harmonic from B8 into n3 | 3rd IMD into n3 |  | - Same as DC_3A-8A-n77A with 2UL_DC_8A_n77A |
| DC_11_n28-n77A DC_11_n28-n77(2A) | DC_11A_n28A | 5th harmonic from n28 into n77 | 3rd & 4th IMDs into n77 |  | - Harmonic problems already covered in CA_n28A_n77A - 17.5 dB by IMD3 - 11.2dB by IMD4 |
|  | DC_11A_n77A | - | 3rd & 4th IMDs into n28 |  | - 15.8 dB by IMD3 - 11.0 dB by IMD4 |
| DC_42A_n3-n28 DC_42C_n3-n28 | DC_42A_n3A DC_42C_n3A | - | - |  | - No issue |
|  | DC_42A_n28A DC_42C_n28A | - | - |  | - No issue |
| DC_42A_n3-n77A DC_42A_n3-n77(2A) DC_42C_n3-n77A DC_42C_n3-n77(2A) | DC_42A_n3A DC_42C_n3A | 2nd harmonic from n3 into n77 | 4th & 5th IMDs into n77 |  | - Harmonic problems already covered in CA_n3A_n77A - These IMD problem was not impact to n77 due to synchronous TDD operation |
| DC_2_n5-n77 or DC_2A-2A_n5A-n77A | DC_2A_n5A | 2nd harmonic from B2 into n77 5th harmonic from n5 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems already covered in DC_2A_n77A or CA_n5A-n77A. - 16.0 dB by IMD3 - 4.1 dB by IMD5 |
|  | DC_2A_n77A | - | 5th IMD into n5 |  | - 3.8 dB by IMD5 |
| DC_13_n2-n77 | DC_13A_n2A | 2nd harmonic from n2 into n77 5th harmonic from B13 into n77 | 3rd, 4th & 5th IMDs into n77 |  | - Harmonic problems already covered in CA_n2A-n77A or DC_13A_n77A. - 17.3 dB by IMD3 - 12.4 dB by IMD4 - 4.5 dB by IMD5 |
|  | DC_13A_n77A | - | 3rd IMD into n2 |  | - 16.0 dB by IMD3 |
| DC_66_n2-n77 | DC_66A_n2A | 2nd harmonic from B66 or n2 into n77 | 2nd & 4th IMDs into n77 | - | - Same as DC_2A_n66A-n78A with 2UL_DC_2A_n66A |
|  | DC_66A_n77A | - | 2nd , 4th & 5th IMDs into n2 | - | - Same as DC_2A-66A_n78A with 2UL_DC_66A_n78A |
| DC_66_n5-n77 DC_66-66_n5-n77 | DC_66A_n5A | 2nd harmonic from B66 into n77 4th & 5th harmonics from n5 into n77 | 3rd, 4th & 5th IMDs into n77 | - | - These harmonic issue already solved in DC_66A_n77A or DC_5A_n77A - 16.6 dB by IMD3 - 12.1 dB by IMD4 - 3.6 dB by IMD5 |
|  | DC_66A_n77A | - |  | - | - No issue |
| DC_28_n1-n78 | DC_28A_n1A | 5th harmonic from 28 into n78 | 3rd IMD into n78 | - | - Harmonic problems already solved in CA_28A-n78A - Same as DC_1A_n28A-n78A with 2UL_DC_1A_n28A |
|  | DC_28A_n78A | 3rd harmonic from B28 into n1 | 3rd IMD into n1 | - | - 3rd harmonic will be covered in CA_1A-28A - Same as DC_1A-28A_n78A with 2UL_DC_28A_n78A |
| DC_2_n48-n66 | DC_2A_n48A | - | 4th IMD into n66 | - | - Same as DC_2A-66A_n48A with 2UL_DC_2A_n48A |
|  | DC_2A_n66A | 2nd harmonic from n66 into n48 | 2nd IMD into n48 | - | - Same as DC_2A-48A_n66A with 2UL_DC_2A_n66A |
| DC_48_n25-n48 | DC_48A_n25A | - | 4th IMD into n48 |  | There was no IMD problem due to TDD n48 |
| DC_48_n48-n66 | DC_48A_n66A | 2nd harmonic from n66 into n48 | 4th IMD into n48 |  | There was no IMD problem due to TDD n48 |
| DC_66_n25-n48 | DC_66A_n25A | 2nd harmonic from n66 into n48 | 2nd IMD into n48 | - | - Same as DC_2A-48A_n66A with 2UL_DC_2A_n66A |
|  | DC_66A_n48A | - | 2nd IMD into n25 | - | - Same as DC_2A-48A_n66A with 2UL_DC_48A_n66A |
| DC_3_n40-n41C | DC_3A_n40A | 3rd harmonic from B3 into n41 | - | - | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_3A_n41A | - | - | Yes | CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_3_n40-n79C | DC_3A_n40A | 2nd harmonic form n40 into n79 | 5th IMD into n79 | - | - Same as DC_3A_n40A-n79A with 2UL_DC_3A_n40A |
|  | DC_3A_n79A | - | 5th IMD into n40 | - | - Same as DC_3A_n40A-n79A with 2UL_DC_3A_n79A |
| DC_3_n41A-n79C DC_3_n41C-n79A DC_3_n41C-n79C | DC_3A_n41A | 2nd harmonic form n41 into n79 | 2nd & 5th IMDs into n79 | Yes | - The 2nd harmonic problem will be studied in CA_n41-n79. - Same as DC_3A_n41A-n79A with 2UL_DC_3A_n41A - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
|  | DC_3A_n79A | - | 2nd & 5th IMDs into n41 | - | - Same as DC_3A_n41A-n79A with 2UL_DC_3A_n79A |
| DC_8_n40-n41C | DC_8A_n40A | 3rd harmonic from B8 into n41 | - | - | Harmonic problem will be studied in DC_8A_n41A. CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_8A_n41A | - | 5th IMD into n40 | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_8_n40-n79C | DC_8A_n40A | 2nd harmonic from n40 into n79 5th harmonic from B8 into n79 | 4th IMD into n79 | - | - These harmonic problem will be treated lower order DC or CA band combos. - Same as DC_8A_n40A-n79A with 2UL_DC_8A_n40A |
|  | DC_8A_n79A | - | 4th IMD into n40 | - | - Same as DC_8A_n40A-n79A with 2UL_DC_8A_n79A |
| DC_8_n41A-n79C DC_8_n41C-n79A DC_8_n41C-n79C | DC_8A_n41A | 2nd harmonic from n41 into n79 5th harmonic from B8 into n79 | 3rd IMD into n79 | - | - These harmonic problem will be treated lower order DC or CA band combos. - Same as DC_8A_n41A-n79A with 2UL_DC_8A_n41A |
|  | DC_8A_n79A | 3rd harmonic from B8 into n41 | 3rd IMD into n41 | - | - These harmonic problem will be treated lower order DC or CA band combos. - Same as DC_8A_n41A-n79A with 2UL_DC_8A_n79A |
| DC_39_n40-n41C | DC_39A_n40A | - | 3rd IMD into n41 | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
|  | DC_39A_n41A | - | - | - | - CA_n40-n41 operation should consider synchronous operation. So No self desense problem to 3rd NR Band |
| DC_39_n40-n79C | DC_39A_n40A | 2nd harmonic from n40 into n79 | 4th IMD into n79 | - | - The harmonic problem will be treated CA_n40-n79 band combos. - Same as DC_39A_n40A-n79A with 2UL_DC_39A_n40A |
|  | DC_39A_n79A | - | - | - | No issue |
| DC_39_n41A-n79C DC_39_n41C-n79A DC_39_n41C-n79C | DC_39A_n41A | 2nd harmonic from n41 into n79 | 2nd & 5th IMDs into n79 | - | - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - Same as DC_39A_n41A-n79A with 2UL_DC_39A_n41A |
|  | DC_39A_n79A | - | 2nd & 5th IMDs into n41 | - | - Same as DC_39A_n41A-n79A with 2UL_DC_39A_n79A |
| DC_13_n5-n48 | DC_13A_n48A | - | - | - | No issue |
| DC_13_n48-n66 | DC_13A_n48A | - | 3rd IMD into n66 | - | Follow 17.1dB MSD for CA_13A-48A-66A in TS36.101 |
|  | DC_13A_n66A | 2nd harmonic from n66 into n48 | 5th IMD into n48 | - | - 2nd harmonic issue will be solved in CA_n48-n66 - In rel-16 TR, define as 2.8 dB |
| DC_66_n5-n48 | DC_66A_n5A | 2nd harmonic from n66 into n48 | 5th IMD into n48 | - | - 2nd harmonic issue will be solved in CA_n48-n66 - In rel-16 TR, define as 3.3 dB |
|  | DC_66A_n48A | - | - | - | No issue |
| DC_11_n3-n28 | DC_11A_n3A | - | 5th IMD into n28 |  | - In rel-16 TR, define as 2.5 dB |
|  | DC_11A_n28A | - | - | - | No issue |
| DC_3_n1-n257 DC_3-3_n1-n257 | DC_3A_n1A | - | - | Yes | No issue |
|  | DC_3A_n257A | - | - | - | No issue |
| DC_7_n1-n257 DC_7-7_n1-n257 | DC_7A_n1A | High order harmonics from B7 or n1 | - | - | No issue |
|  | DC_7A_n257A | - | - | - | No issue |
| DC_3_n78-n258 | DC_3A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_3A_n258A | 2nd harmonic from B3 into B78 | - | - | - Harmonic problem already covered in DC_3A_n78A |
| DC_7A_n78-n258 DC_7C_n78-n258 | DC_7A_n78A DC_7C_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_7A_n258A DC_7C_n258A | - | - | - | - No issue |
| DC_28_n78-n258 | DC_28A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_28A_n258A | 5th harmonic from B28 into n78 | - | - | - Harmonic problem already covered in DC_28A_n78A |
| DC_8_n78-n258 | DC_8A_n78A | 7th & 8th harmonics from n78 into n258 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_8A_n258A | 4th harmonic from B8 into n78 | - | - | - Harmonic problem already covered in DC_8A_n78A |
| DC_8_n40-n258 | DC_8A_n40A | 11th harmonics from n40 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_8A_n258A | - | - | - | - No issue |
| DC_3_n34-n258 | DC_3A_n34A | 14th & 15th harmonics from B3 into n258 12th & 13th harmonics from n34 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_3A_n258A | - | - | - | - No issue |
| DC_3_n41-n258 | DC_3A_n41A | 10th harmonic from n41 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_3A_n258A | - | - | - | - No issue |
| DC_8_n34-n258 | DC_8A_n34A | 12th & 13th harmonics from n34 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_8A_n258A | - | - | - | - No issue |
| DC_8_n41-n258 | DC_8A_n41A | 10th harmonic from n41 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_8A_n258A | 3rd harmonic from B8 into n41 | - | - | - Harmonic problem already covered in DC_8A_n41A |
| DC_39_n40-n258 | DC_39A_n40A | 11th harmonics from n40 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_39A_n258A | - | - | - | - No issue |
| DC_39_n41A-n258 DC_39_n41C-n258 | DC_39A_n41A | 10th harmonic from n41 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_39A_n258A | - | - | - | - No issue |
| DC_8_n79-n258 | DC_8A_n79A | 5th & 6th harmonic from n79 into n258 | - |  | - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_8A_n258A | 5th harmonic from B8 into n79 | - | - | - Harmonic problem already covered in DC_8A_n79A |
| DC_39_n79A-n258 DC_39_n79C-n258 | DC_39A_n79A | 5th & 6th harmonic from n79 into n258 | - |  | - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_39A_n258A | - | - | - | - No issue |
| DC_40_n41A-n258 DC_40_n41C-n258 | DC_40A_n41A | 10th harmonic from n41 into n258 11th harmonics from B40 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 |
|  | DC_40A_n258A | - | - | - | - No issue |
| DC_40_n79A-n258 DC_40_n79C-n258 | DC_40A_n79A | 11th harmonics from B40 into n258 5th & 6th harmonic from n79 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_40A_n258A | 2nd harmonic from B40 into n79 |  |  | - These harmonic problem will be covered in DC_40A_n79A |
| DC_41_n79A-n258 DC_41_n79C-n258 | DC_41A_n79A | 10th harmonic from n41 into n258 5th & 6th harmonic from n79 into n258 |  |  | - No harmonic problems higher than 7th order between FR1 and FR2 - These harmonic problem will be covered in CA_n79A_n258A |
|  | DC_41A_n258A | 2nd harmonic from n41 into n79 |  |  | - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used |
| DC_8_n77(2A)-n257 | DC_8A_n77A | 7th & 8th Harmonics from n77 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_8A_n257D DC_8A_n257G DC_8A_n257H DC_8A_n257I | 4th harmonic from B8 into n77 | - | - | - Harmonic problem already covered in DC_8A_n77A |
| DC_11_n77(2A)-n257 | DC_11A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | No harmonic problem to n257 |
|  | DC_11A_n257D DC_11A_n257G DC_11A_n257H DC_11A_n257I | - | - | - | No issue |
| DC_28_n77(2A)-n257D | DC_28A_n77A | 7th & 8th harmonics from n77 into n257 | - | - | No harmonic problem to n257 |
|  | DC_28A_n257D | 5th harmonic from B28 into n77 | - | - | 5th harmonic problem was solved in DC_28A_n77A |
| DC_2_n7A-n66A DC_2_n7(2A)-n66A | DC_2A_n7A | - | - | - | No issue |
|  | DC_2A_n66A | - | - | - | No issue |
| DC_12_n7A-n66A DC_12_n7(2A)-n66A | DC_12A_n7A | 3rd harmonic into n66 from B12 | - | - | The 3rd harmonic issue already covered in DC_12A_n66A |
|  | DC_12A_n66A | - | - | - | No issue |
| DC_2_n66A-n78(2A) DC_2_n66(2A)-n78A DC_2_n66(2A)-n78(2A) | DC_2A_n66A | 2nd harmonics into n78 from B2 and n66 | 2nd & 4th IMDs into n78 | - | The 2nd harmonic issue already covered in DC_66A_n78A - Follow same as DC_2A_n66A-n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_2A_n78A | - | 4th IMD into n66 | - | - For 4th IMD problem, follow same as DC_2A-66A_n78A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_12_n66A-n78A DC_12_n66A-n78(2A) DC_12_n66(2A)-n78A DC_12_n66(2A)-n78(2A) | DC_12A_n66A | 2nd harmonic into n78 from n66 5th harmonic into n78 from B12 | 5th IMD into n78 | - | These harmonic issues are already covered in DC_66A_n78A or DC_12A_n78A - 4.1dB by 5th IMD in TR37.716-21-21 |
|  | DC_12A_n78A | 3rd harmonic into n66 from B12 | 3rd IMD into n66 | - | 3rd harmonic issue already covered in DC_12A_n66A - 16.5dB by 3rd IMD in TR37.716-21-21 |
| DC_13_n7-n78 | DC_13A_n7A | - | 2nd & 4th IMDs into n78 | - | - 29.0 dB by 2nd IMD already analyzed in TR37.716-21-21 |
|  | DC_13A_n78A |  | 2nd IMD into n7 | - | - 27.9 dB by 2nd IMD already analyzed in TR37.716-21-21 |
| DC_2_n38-n66 | DC_2A_n38A | - | - | - | No issue |
|  | DC_2A_n66A | - | - | - | No issue |
| DC_66_n38-n66 | DC_66A_n38A | - | - | - | No issue |
|  | DC_66A_n66A | - | - | - | DC_66A_n66A only allowed single UL switched mode |
| DC_1-3_n3-n77, DC_1-3_n3-n77(2A) or DC_1-3_n3-n78 | DC_1A_n3A | 2nd harmonic from n3 into n77/n78 | 2nd & 4th IMDs into n77/n78 | Yes | - Harmonic problems was already covered in DC_3A_n77A. - 2nd &4th IMDs problems same as DC_1A_n3A-n78A. |
|  | DC_1A_n77A or DC_1A_n78A |  | 2nd, 4th & 5th IMDs into B3 | Yes | These MSD issues are same as DC_1A-3A_n77A. - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_3A_n3A DC_(n)3AA | 2nd harmonic from n3 into n77/n78 | 2nd & 4th IMD into n77/n78 | - | - Harmonic problems was already covered in DC_3A_n77A. - Due to apply MPR, no need to define MSD |
|  | DC_3A_n77A or DC_3A_n78A | - | 2nd & 5th IMD into B1 2nd, 4th &5th IMD into n3 | - | - 31.0 dB by 2nd IMD that follow DC_1A-3A_n77A in TR37.863-02-01 in rel-15. - Follow DC_3A_n1A-n78A for 5th IMD problem. - MSD problems in n3 were already solved in DC_3A_n77A |
| DC_18_n28-n77 or DC_18_n28-n78 | DC_18A_n28A | 4th & 5th harmonic from B18 and n28 fall into n77/n78 | 5th IMD into n77/n78 | - | - Harmonic problems was already covered in DC_18A_n77A or CA_n28-n77 - 4.0 dB by IMD5 |
|  | DC_18A_n77A or DC_18A_n78A | - | 5th IMD into n77 | - | - 5.7 dB by IMD5 |
| DC_1_n3-n41 | DC_1A_n3A | - | 5th IMD into n41 | - | - 5.0 dB by 5th IMD that follow DC_1A-41A_n3A in TS38.101-3 |
|  | DC_1A_n41A | - | - | Yes | Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
| DC_3_n3-n41 | DC_3A_n3A | - | - | - | No issue |
|  | DC_3A_n41A | - | 4th IMD into n3 | - | 4th IMD issue already covered by DC_3_n41 in Table 7.3B.2.3.5.1-1 in TS38.101-13 |
| DC_18_n3-n41 | DC_18A_n3A | - | 2nd & 3rd IMDs into B41 | - | 3rd IMD problem already covered in DC_18A-41A_n3A - 29.4 dB for 2nd IMD |
|  | DC_18A_n41A | - | 2nd IMD into n3 |  | - 28.2 dB by IMD2 |
| DC_1-3_n3-n41 | DC_1A_n3A | - | 5th IMD into n41 |  | 5th IMD problem already covered in CA_1A-3A-41A in TS36.101 |
|  | DC_1A_n41A | - | - | - | No issue |
|  | DC_3A_n3A | - | - | - | No issue |
|  | DC_3A_n41A | - | - | - | No issue |
| DC_18_n28-n41 | DC_18A_n28A |  | 5th IMD into n41 | - | - 5.1 dB by IMD5 |
|  | DC_18A_n41A |  | 5th IMD into n28 | - | - 4.7 dB by IMD5 |
| DC_1_n41-n77 | DC_1A_n41A | 2nd harmonic from B1 into n77 | 3rd , 4th & 5th IMDs into n77 | - | The harmonic problem already covered in DC_1_n77 in TS38.101-3 These IMDs problem follow DC_1_n41-n78 in TS38.101-3 |
|  | DC_1A_n77A | - | 4th & 5th IMDs into n41 | - | The IMD problem follow DC_1-41_n77 in TS38.101-3 |
| DC_3_n41-n77 | DC_3A_n41A | 2nd harmonic from B3 into n77 | 3rd & 5th IMDs into n77 | - | The harmonic problem already covered in DC_3_n77 in TS38.101-3 - These IMDs problem follow DC_3_n41-n78 in TS38.101-3 |
|  | DC_3A_n77A |  | 5th IMD into n41 |  | - The IMD problem follow DC_3-41_n77 in TS38.101-3 |
| DC_18_n41-n77 or DC_18_n41-n78 | DC_18A_n41A | 4th harmonic from B18 into n78 | 2nd & 4th IMDs into n77/n78 3rd IMD into n77 | - | - 30.1 dB by IMD2 -16.4 dB by IMD3 - 9.1 dB by IMD4 |
|  | DC_18A_n77A or DC_18A_n78A |  | 2nd IMD into n4177/n78 3rd IMD into n41 in DC_18A_n77A | - | - 28.5 dB by IMD2 |
| DC_41_n3-n41 | DC_41A_n3A | - | 4th IMD into n41 | Yes | For TDD band, the IMD problem is not impact to the 3rd Rx band |
|  | DC_41A_n41A | - | - | Yes | No issue |
| DC_41_n28-n41 | DC_41A_n28A | - | - | - | No issue |
|  | DC_41A_n41A | - | - | - | No issue |
| DC_41_n41-n77 or DC_41_n41-n78 | DC_41A_n41A | - | - | - | No issue |
|  | DC_41A_n77A or DC_41A_n78A | - | 4th IMD into n41 |  | For TDD band, the IMD problem is not impact to the 3rd Rx band |
| DC_(n)5AA-n77A | DC_(n)5AA | 4th & 5th harmonics from n5 into n77 | 4th & 5th IMD into n77 |  | 4th & 5th harmonic issues were covered in DC_5A_n77A The 4th & 5th IMD problem will not be impacted due to only single UL switching operation. |
|  | DC_5A_n77A |  | 4th & 5th IMDs |  | The 4th & 5th IMD problem will be covered in DC_5A_n77A |
| DC_12_n5-n77 | DC_12A_n5A | 4th & 5th harmonics from n5 into n77 | 5th IMD |  | The 4th & 5th IMD problem will be covered in DC_5A_n77A - 5.4 dB by IMD5 |
|  | DC_12A_n77A |  | 5th IMD |  | - 4.8 dB by IMD5 |
| DC_14_n5-n77 | DC_14A_n5A | 4th & 5th harmonics from n5 into n77 | 4th & 5th IMD |  | The 4th & 5th IMD problem will be covered in DC_5A_n77A - 10.6 dB by IMD4 -5.1 dB by IMD5 |
|  | DC_14A_n77A |  | 5th IMD |  | - 4.4 dB by IMD5 |
| DC_2_n71-n78 DC_2-2_n71-n78 | DC_2A_n71A | 2nd harmonic from B2 into n78 5th harmonic from n71 into n78 | 3rd IMD into n78 | - | These harmonics issues were already solved in DC_2_n78 or CA_n71_n78 - 8.0 dB by IMD3 |
|  | DC_2A_n78A | - | - | - | No issue |
| DC_7_n71-n78 | DC_7A_n71A | 5th harmonic from n71 into n78 | 4th IMD into n78 | - | - The harmonic issue was already solved in CA_n71_n78 - 9.7 dB by IMD4 |
|  | DC_7A_n78A | - | 5th IMD into n71 | - | - 3.9 dB by IMD5 |
| DC_66_n71-n78 | DC_66A_n71A | -2nd harmonic from B66 into n78 5th harmonic from n71 into n78 | 4th & 5th IMDs into n78 | - | Harmonic issues were already solved in DC_66_n78 or CA_n71_n78 - MSD of 4th IMD can be reused in DC_48A-66A_n71A in TS38.101-3 |
|  | DC_66A_n78A | - | - | - | No issue |
| DC_2_n38-n71 | DC_2A_n38A | - | - | - | No issue |
|  | DC_2A_n71A | - | 2nd IMD into n38 | - | 29.2 dB of DC_2A_n41A-n71A can be reused in TS38.101-3 |
| DC_66_n38-n71 | DC_66A_n38A | - | - | - | No issue |
|  | DC_66A_n71A | - | - | - | No issue |
| DC_2_n38-n78 | DC_2A_n38A | 2nd harmonic from B2 | 3rd IMD into n78 |  | Harmonic issue were covered in DC_2A_n78A 14.8 dB was captured in TR37.716-21-21 in Rel-16 |
|  | DC_2A_n78A | - | - | - | No issue |
| DC_66_n38-n78 | DC_66A_n38A | 2nd harmonic from B66 | 3rd IMD into n78 | - | Harmonic issue were covered in DC_66A_n78A 15.0 dB was captured in TR37.716-21-21 in Rel-16 |
|  | DC_66A_n78A | - | - | - | No issue |
| DC_71_n38-n78 | DC_71A_n38A | 5th harmonic from B71 into n78 | 2nd & 4th IMDs into n78 |  | Harmonic issue were covered in DC_71A_n78A - 29.1 dB by IMD2 -10.3 dB by IMD4 |
|  | DC_71A_n78A |  | 2nd IMD into n38 |  | - 28.7 dB by IMD2 |
| DC_5_n66-n77 | DC_5A_n66A | -2nd harmonic from B66 into n77 4th & 5th harmonics from B5 into n77 | 3rd , 4th & 5th IMDs into n77 |  | - Harmonics issue will be solved in DC_66A_n77A or DC_5A_n77A - Reuse 16.6dB MSD of DC_5A_n66A-n78A with dual uplink DC_5A_n66A in TS38.101-3 |
|  | DC_5A_n77A | - | 3rd IMDs into n66 |  | - Reuse 13.2dB MSD of DC_5A_n66A-n77A with dual uplink DC_5A_n77A in TS38.101-3 |
| DC_5_n66-n78 | DC_5A_n66A | -2nd harmonic from B66 into n78 4th harmonic from B5 into n78 | 3rd & 5th IMDs into n78 |  | Harmonic issue were covered in DC_66A_n78A or DC_5A_n78A - Same MSD levels can be reused in DC_66_n5-n77 in this TR |
|  | DC_5A_n78A | - | 3rd IMD into n66 |  | 13.2 dB by DC_5A-66A_n78A can be reused in TS38.101-3 |
| DC_71_n66-n78 | DC_71A_n66A | -2nd harmonic from n66 into n78 5th harmonic from B71 into n78 | 4th & 5th IMDs into n78 | - | Harmonic issues were already solved in DC_2_n78 or CA_n71_n78 - MSD of 4th IMD can be reused in DC_48A-66A_n71A in TS38.101-3 |
|  | DC_71A_n78A |  | 3rd IMD into n66 | - | - 15.5 dB by IMD3 |
| DC_5_n38-n66 | DC_5A_n38A | - | - | - | No issue |
|  | DC_5A_n66A |  | 2nd & 3rd IMD into n38 |  | - 28.9 dB by IMD2 - 16.4 dB by IMD3 |
| DC_71_n38-n66 | DC_71A_n38A | - | - | - | No issue |
|  | DC_71A_n66A | - | - | - | No issue |
| DC_2_n2-n38 or DC_2_n2-n41 | DC_2A_n2A | - | - | - | DC_2A_n2A only allowed single UL switched mode |
|  | DC_2A_n38A or DC_2A_n41A | - | - | - | No issue |
| DC_66_n2-n38 | DC_66A_n2A | - | - | - | No issue |
|  | DC_66A_n38A | - | - | - | No issue |
| DC_12_n2-n38 or DC_12_n2-n41 | DC_12A_n2A |  | 2nd IMD into n38/n41 | - | - 28.7 dB by IMD2 |
|  | DC_12A_n38A or DC_12A_n41A | - | - | - | No issue |
| DC_71_n2-n41 | DC_71A_n2A | 4th harmonic from B71 into n41 | 2nd IMD into n41 | - | Harmonic issue already covered in DC_71_n41 - 29.2dB MSD by DC_2A_n41A-n71A can be reused in TS38.101-3 |
|  | DC_71A_n41A | 3rd harmonic from B71 into n2 | 2nd IMD into n2 |  | Harmonic issue already covered in DC_71_n2 - 26.0dB MSD by DC_2A-71A_n38A can be reused in TS38.101-3 |
| DC_2_n2-n78 | DC_2A_n2A | 2nd harmonic from B2 into n78 | 2nd & 4th IMDs into n78 |  | DC_2A_n2A only allowed single UL switched mode. |
|  | DC_2A_n78A |  | 2nd,4th & 5th IMDs into n2 |  | The IMD problem already solved in DC_2A_n78A in TS38.101-3 |
| DC_7_n2-n78 | DC_7A_n2A | 2nd harmonic from n2 into n78 | 5th IMD into n78 |  | Harmonic issue already covered in DC_2_n78 4.2dB MSD of DC_2A_n7A-n78A can be reused in TS38.101-3 |
|  | DC_7A_n78A | - | 4th IMD into n2 |  | 8.6dB MSD of DC_2A-7A_n78A can be reused in TS38.101-3 |
| DC_71_n2-n78 | DC_71A_n2A | 2nd harmonic from n2 into n78 5th harmonic from B71 into n78 | 3rd IMD into n78 |  | Harmonic issue already covered in DC_2_n78 or DC_71_n78 - 3rd IMD can be reused in DC_2_n71-n78 in this TR |
|  | DC_71A_n78A | 3rd harmonic from B71 into n2 | 3rd IMD into n2 |  | Harmonic issue already covered in DC_71_n2 - 16.5dB MSD of DC_2A-71A_n78A can be reused in TS38.101-3 |
| DC_2_n66-n71 DC_2-2_n66-n71 | DC_2A_n66A | - | - | - | No issue |
|  | DC_2A_n71A | - | - | - | No issue |
| DC_7_n66-n71 | DC_7A_n66A | - | - | - | No issue |
|  | DC_7A_n71A | - | - | - | No issue |
| DC_66_n66-n71 | DC_66A_n66A | - | - | - | DC_66A_n66A only allowed single UL switched mode |
|  | DC_66A_n71A |  | 4th IMD into n66 |  | 5.0dB MSD for DC_66A_n71A can be reused in TS38.101-3 |
| DC_2_n2-n71 | DC_2A_n2A | - | - | - | DC_2A_n2A only allowed single UL switched mode |
|  | DC_2A_n71A | 3rd harmonic from n71 into B2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_7_n2-n71 | DC_7A_n2A |  | 2nd & 5th IMDs into n71 |  | 28.7dB MSD for 2nd IMD of DC_2A_n41A-n71A can be reused in TS38.101-3 |
|  | DC_7A_n71A | 3rd harmonic from n71 into n2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_66_n2-n71 | DC_66A_n2A | - | - | - | No issue |
|  | DC_66A_n71A | 3rd harmonic from n71 into n2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_2_n2-n66 | DC_2A_n2A | - | - | - | DC_2A_n2A only allowed single UL switched mode |
|  | DC_2A_n66A |  | 3rd & 5th IMDs into n2 |  | 20.0dB MSD of for 3rd IMD DC_2A_n66A can be reused in TS38.101-3 |
| DC_7_n2-n66 | DC_7A_n2A | - | - | - | No issue |
|  | DC_7A_n66A | - | - | - | No issue |
| DC_66_n2-n66 | DC_66A_n2A |  | 5th IMD into n66 |  | 4.0dB MSD of for 5th IMD DC_2A_n66A can be reused in TS38.101-3 |
|  | DC_66A_n66A | - | - | - | DC_66A_n66A only allowed single UL switched mode |
| DC_71_n2-n66 | DC_71A_n2A | - | - | - | No issue |
|  | DC_71A_n66A | 3rd harmonic from B71 into n2 | - | - | Harmonic issue already covered in DC_2_n71 |
| DC_2_n66-n77 or DC_2-2_n66-n77 | DC_2A_n77A | - | 2nd , 4th & 5th IMDs into n66 | - | - 29.2 dB for 2nd IMD - 10.3 dB MSD by 4th IMD in DC_2A-66A_n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_2_n66-n77 DC_2-2_n66-n77 | DC_2A_n66A | 2nd harmonic from B2 or n66 | 2nd & 4th IMDs into n77 |  | - Harmonic issue already covered in DC_2_n77 or CA_n66-n77 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_13_n66-n77 | DC_13A_n77A |  | 3rd IMD into n66 |  | - 17.1dB MSD for DC_13-66A_n48A can be reused in TS38.101-3 |
|  | DC_13A_n66A | 2nd harmonic from n66 into n77 5th harmonic from B13 into n77 | 3rd, 4th & 5th IMDs into n77 | - | -Harmonic issue will be solved in CA_n66-n77 or DC_13_n77 - 16.4 dB by IMD3 - 11.7 dB by IMD4 - 4.4 dB by IMD5 |
| DC_66_n66-n77 | DC_66A_n77A | - | 2nd & 5th IMDs into B66 | - | - These IMDs problems already covered in DC_66A_n77A |
| DC_11_n3-n77 or DC_11_n3-n77(2A) | DC_11A_n3A | 2nd harmonic from n3 into n77 | 4th & 5th IMDs into n77 | - | Harmonic issues already solved in DC_3_n77 - 10.8 dB by IMD4 |
|  | DC_11A_n77A |  | 2nd & 5th IMDs into n3 |  | - 29.0 dB by IMD2 |
| DC_2_n25-n66 | DC_2A_n25A | - | 5th IMD into n66 |  | - Reuse 4.0 dB MSD in DC_2A-66A_n25A in TS38.101-3 |
|  | DC_2A_n66A |  | 3rd & 5th IMDs into n25 |  | Reuse 20.0 dB MSD in DC_2A_n66A for 3rd IMD in TS38.101-3 |
| DC_7_n25-n66 DC_7-7_n25-n66 | DC_7A_n25A | - | - | - | No issue |
|  | DC_7A_n66A | - | - | - | No issue |
| DC_13_n25-n66 | DC_13A_n25A | - | 4th IMD into n66 | - | - Reuse 7.2 dB MSD in DC_13A-66A_n2A in TS38.101-3 |
|  | DC_13A_n66A | - | 4th IMD into n25 | - | - Reuse 6.2 dB MSD in DC_2A-13A_n66A in TS38.101-3 |
| DC_66_n25-n66 | DC_66A_n25A | - | 3rd & 5th IMDs into n66 | - | - Reuse 23.0 dB & 4.0 dB MSD in DC_66A_n25A in TS38.101-3 |
| DC_7_n66-n77 DC_7-7_n66-n77 | DC_7A_n66A | 2nd harmonic from n66 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic issue already covered in CA_n66-n77 - Reuse 16.0dB MSD for 3rd IMD in DC_7A_n66A-n78A in TS38.101-3 |
|  | DC_7A_n77A | - | 4th IMD into n66 | - | - Reuse 8.7dB MSD for 4th IMD in DC_7A-66A_n78A in TS38.101-3 |
| DC_21_n28-n77 | DC_21A_n28A | 5th harmonic from n28 into n77 | 3rd & 4th IMDs into n77 | - | - 17.3 dB in n77 by IMD3 |
|  | DC_21A_n77A | - | 3rd & 4th IMDs into n28 | - | - Reuse 16.9 dB MSD for 3rd IMD in DC_21A-28A_n77A in TS38.101-3 |
| DC_1_n28-n79 | DC_1A_n28A | 6th & 7th harmonic from n28 into n79 | 3rd & 5th IMDs into n79 |  | - No harmonic issue by 6th & 7th harmonic order - 14.9dB in n79 by IMD3 |
|  | DC_1A_n79A | - | 3rd & 4th IMDs into n28 |  | - Reuse 15.2 dB/ 10.0 dB MSD for 3rd /4th IMD in DC_1A-28A_n79A in TS38.101-3 |
| DC_3_n28-n79 | DC_3A_n28A | 6th & 7th harmonic from n28 into n79 | 4th & 5th IMDs into n79 |  | - No harmonic issue by 6th & 7th harmonic order - 9.4dB in n79 by IMD4 |
|  | DC_3A_n79A | - | 4th IMD into n28 | - | - Reuse 10.3 dB MSD for 4th IMD in DC_3A-28A_n79A in TS38.101-3 |
| DC_21_n28-n79 | DC_21A_n28A | 6th & 7th harmonic from n28 into n79 | 4th & 5th IMDs into n79 |  | - No harmonic issue by 6th & 7th harmonic order - 6.3 dB in n79 by IMD4 |
|  | DC_21A_n79A | - | 5th IMD into n28 | - | - 2.8 dB in n28 by IMD5 |
| DC_21_n28-n78 | DC_21A_n28A | 5th harmonic from n28 into n78 | 3rd & 4th IMDs into n78 |  | - Harmonic issue already covered in CA_n28-n78 - same MSD value and test configuration of DC_21A_n28A-n77A in this TR |
|  | DC_21A_n78A | - | 3rd IMD into n28 | - | - Reuse 16.9 dB MSD for 3rd IMD in DC_21A-28A_n77A in TS38.101-3 |
| DC_2_n2-n77 | DC_2A_n77A | - | 2nd & 4th IMD IMDs into n2 | - | - Reuse 26 dB/ 8dB MSD for 2nd / 4th IMD in DC_2A_n78A in TS38.101-3 |
| DC_5_n2-n77 | DC_5A_n2A | 2nd harmonic from n2 and 4th & 5th harmonics from B5 into n77 | 3rd & 5th IMDs into n77 | - | - Harmonic issue already covered in CA_n2-n77 or DC_5_n77 - 16.0dB MSD by 3rd IMD of DC_2A_n5A-n77A in this TR can be used |
|  | DC_5A_n77A | - | - | - | No issue |
| DC_13_n5-n77 | DC_13A_n77A | - | 5th IMD into n5 | - | - 4.5 dB in n5 by IMD5 |
| DC_66-66_n2-n77 | DC_66A_n2A | 2nd harmonic from B66 or n2 into n77 | 2nd & 4th IMDs into n77 |  | - Harmonic issue already covered in CA_n2-n77 or DC_66_n77 - Reuse 29.4 dB/ 8.9dB MSD for 2nd / 4th IMD in DC_2A_n66A-n78A in TS38.101-3 |
|  | DC_66A_n77A | - | 2nd , 4th & 5th IMDs into n2 |  | - Reuse 32.1 dB/ 9.1dB/ 2.1dB MSD for 2nd / 4th / 5th IMD in DC_2A-66A_n78A in TS38.101-3 |
| DC_5_n5-n77 | DC_5A_n77A | - | 4th & 5th IMDs into n5 |  | - Reuse 8.3dB MSD for 4th IMD in DC_5A_n78A in TS38.101-3 |
| DC_2-66_n66-n77 | DC_2A_n66A | 2nd harmonic from B2 or n66 into n77 | 2nd & 4th IMDs into n77 |  | - Harmonic issue already covered in DC_2_n77 or CA_n66-n77 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_2A_n77A | - | 2nd , 4th & 5th IMDs into n66 | - | - 29.2 dB for 2nd IMD - 10.3 dB MSD by 4th IMD in DC_2A-66A_n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n77A | - | 2nd , 4th & 5th IMDs into n2 |  | - Reuse 32.1 dB/ 9.1dB/ 2.1dB MSD for 2nd / 4th / 5th IMD in DC_2A-66A_n78A in TS38.101-3 |
| DC_5-66_n66-n77 | DC_5A_n66A | -2nd harmonic from B66 into n77 4th & 5th harmonic from B5 into n77 | 3rd , 4th & 5th IMDs into n77 |  | Harmonic issue were covered in DC_66A_n77A or DC_5A_n77A - Same MSD levels can be reused in DC_66_n5-n77 in this TR |
|  | DC_5A_n77A |  | 3rd IMD into B66 or n66 |  | - Reuse 13.2 dB MSD for 3rd IMD in DC_5A-66A_n78A in TS38.101-3 |
|  | DC_66A_n77A | - | 2nd & 5th IMDs into n66 | - | - These IMD problem already solved in DC_66A_n77A |
| DC_5-66_n5-n77 | DC_5A_n77A |  | 3rd IMD into B66 or n66 4th & 5th IMDs into n5 |  | - Reuse 13.2 dB MSD for 3rd IMD in DC_5A-66A_n78A in TS38.101-3 - Reuse 8.3 dB MSD for 4th IMD in DC_5A_n78A in TS38.101-3 |
|  | DC_66A_n5A | -2nd harmonic from B66 into n77 4th & 5th harmonic from B5 into n77 | 3rd , 4th & 5th IMDs into n77 2nd & 5th IMD into B5 |  | Harmonic issue were covered in DC_66A_n77A or DC_5A_n77A - Same MSD levels can be reused in DC_66_n5-n77 in this TR - 30dB MSD for 2nd IMD in DC_5A_n66A in TS38.101-3 can be reused |
|  | DC_66A_n77A | - | - | - | No issue |
| DC_28_n7-n258 | DC_28A_n7A DC_28A_n7B | 10th harmonic from n7 into n258 | - | - | No issue |
|  | DC_28A_n258A/G/H/I | - | - | - | No issue |
| DC_30_n5-n77 | DC_30_n5 | 4th & 5th harmonic from n5 into n77 | 3rd IMD into n77 | - | 4th & 5th Harmonic issues already covered in DC_5A_n77A. -16.1 dBby 3rd IMD |
|  | DC_30_n77 | - | 3rd & 5th IMDs into n5 | - | - 15.3 dB by 3rd IMD |
| DC_3-3_n8-n78 | DC_3_n8 | 2nd harmonic from B3 into n78 4th harmonic from n8 into n78 | 3rd & 5th IMDs into n78 4th & 5th IMDs into B3 | - | The 3rd IMD problem, follow MSD (16.3dB) for DC_3A_n8A-n78A. The 4th &5th IMDs problem already covered in DC_3A_n8A. |
|  | DC_3_n78 | - | - | - | No issue |
| DC_7-7_n8-n78 | DC_7_n8 | 4th harmonic from n8 into n78 | 2nd & 4th IMDs into n78 |  | 4th Harmonic issues already covered in DC_8A_n78A. The 2nd IMD problem, follow MSD (28.5dB) for DC_7A_n8A-n78A. |
|  | DC_7_n78 | - | 2nd IMD into n8 |  | The 2nd IMDs problem, follow MSD (29.7dB) for DC_7A_n8A-n78A. |
| DC_3_n1-n8 DC_3-3_n1-n8 | DC_3_n1 | - | - | Yes | - Small freq. gap was covered in Table 7.3.1A-0bA in TS38.101-3 |
|  | DC_3_n8 | - | - | - | No issue |
| DC_7_n1-n8 DC_7-7_n1-n8 | DC_7_n1 | - | 5th IMD into n8 | - | - Reuse 4.5dB by 5th IMD from DC_1A-28A_n7A |
|  | DC_7_n8 | 3rd harmonic from n8 into B7 | - | - | The harmonic issue already covered in DC_7A_n8A. |
| DC_n1-n28_3 | DC_n1A_3A | - | - | - | No issue |
|  | DC_n28A_3A | 3rd harmonic from n28 into n1 | 4th IMD | - | - Harmonic problem already covered in DC_1A_n28A 11.0dB by 4th IMD problem same as DC_1A-3A_28A in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_3_n28-n78 | DC_3C_n78A | - | - | - | No issue |
| DC_3_n7-n78 | DC_3C_n78A | - | - | - | No issues |
| DC_8_n1-n40 | DC_8A_n1A | - | - | - | No issues |
|  | DC_8A_n40A | - | 5th IMD into n1 | - | - FFS |
| DC_3_n1-n38 | DC_3A_n1A | - | - | - | No issues |
|  | DC_3A_n38A | - | - | - | No issues |
| DC_3_n1-n41 | DC_3A_n1A | - | 5th IMD into n41 | - | - 5.0 dB by 5th IMD that follow DC_1A-41A_n3A in TS38.101-3 |
|  | DC_3A_n41A | - | - | - | No issues |
| DC_1_n3-n79 | DC_1A_n3A |  | 5th IMD | Yes | - 4.7dB by 5th IMD that follow DC_3A_n1A-n79A in TR37.716-21-21 |
|  | DC_1A_n79A |  |  | Yes | Cross isolation issue already solved. |
| DC_8_n3-n79 | DC_8A_n3A |  | 3rd & 5th IMDs | - | - 15.7dB by 3rd IMD |
|  | DC_8A_n79A |  | 4th IMD | - | -8.8dB by 4th IMD that follow DC_3A-8A_n79A in TS38.101-3 |
| DC_8_n28-n79 | DC_8A_n28A | 5th harmonic from B8 into n79 | 5th IMD | - | - Harmonic problem already covered in DC_8A_n79A - 0dB by 5th IMD |
|  | DC_8A_n79A |  | 5th IMD | - | - 3.9dB by 5th IMD |
| DC_8_n77-n79 DC_8_n77(2A)-n79 | DC_8A_n77A | 5th harmonic from B8 into n79 | 2nd , 4th & 5th IMDs |  | - Harmonic problem already covered in DC_8A_n79A - RAN4 agreed not to consider simultaneous Rx/Tx capability for the CA band combinations with n77 and n79. |
|  | DC_8A_n79A | - | 2nd IMD |  | - RAN4 agreed not to consider simultaneous Rx/Tx capability for the CA band combinations with n77 and n79. |
| DC_8_n77-n257D/G/H/I DC_n77-n257A/G/H/I/J/K/L/M_8 DC_n77(2A)-n257A/G/H/I/J/K/L/M_8 | DC_8A_n77A DC_n77A_8A | - | - |  | No issues |
|  | DC_8A_n257D/G/H/I DC_n257A_8A | - | - |  | No issues |
| DC_11_n77-n257D/G/H/I | DC_11A_n77A | - | - |  | No issues |
|  | DC_11A_n257D/G/H/I | - | - |  | No issues |
| DC_2_n12-n77 DC_2-2_n12-n77 | DC_2A_n12A | 2nd harmonic from B2 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_2A_n77A - 16.0 dB MSD by 3rd IMD from CA_n2-n12-n77 can be reused |
|  | DC_2A_n77A | - | - | - | No issues |
| DC_7_n12-n77 | DC_7A_n12A | - | 3rd & 4th IMDs | - | - 15.2 dB by 3rd IMD |
|  | DC_7A_n77A | - | 2nd IMD | - | - 30.8dB MSD by 2nd IMD from DC_7A-12A_n78A can be reused |
| DC_66_n12-n77 | DC_66A_n12A | 2nd harmonic from B66 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_66A_n77A - 16.0 dB MSD by 3rd IMD from CA_n12-n66-n77 can be reused |
|  | DC_66A_n77A |  | 3rd IMD | - | - 15.2dB MSD by 3rd IMD from DC_12A-66A_n77A can be reused |
| DC_12_n41-n66 | DC_12A_n41A | 3rd harmonic from B12 into n66 | - | - | - Harmonic problem already covered in DC_12A_n66A |
|  | DC_12A_n66A | - | 2nd IMD | - | Edge IMD2 regions will be impact in n41 - 28.7dB MSD by 2nd IMD from DC_12A_n2A-n41A can be reused |
| DC_12_n66-n77 | DC_12A_n66A | 2nd harmonic from n66 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_66A_n77A - 16.0 dB MSD by 3rd IMD from CA_n12-n66-n77 can be reused |
|  | DC_12A_n77A | 3rd harmonic from B12 into n66 | 3rd & 4th IMDs | - | - Harmonic problem already covered in DC_12A_n66A - 13.2dB MSD by 3rd IMD from DC_12A-66A_n77A can be reused |
| DC_71_n66-n77 | DC_71A_n66A | 2nd harmonic from n66 into n77 5th harmonic from B71 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_66A_n77A or DC_71A_n77A - 15.9dB MSD by 3rd IMD from CA_n66-n71-n77 can be reused |
|  | DC_71A_n77A | - | 3rd & 4th IMDs | - | - 15.5dB MSD by 3rd IMD from DC_71A_n66A-n78A can be reused |
| DC_2_n71-n77 DC_2-2_n71-n77 | DC_2A_n71A | 2nd harmonic from B2 into n77 5th harmonic from n71 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_2A_n77A or DC_71A_n77A - 8.0dB MSD by 3rd IMD from DC_2A_n71A-n78A can be reused |
|  | DC_2A_n77A | - | - | - | No issues |
| DC_7_n71-n77 | DC_7A_n71A | 5th harmonic from n71 into n77 | 3rd & 4th IMDs | - | - Harmonic problem already covered in DC_71A_n77A - 15.2dB by 3rd IMD |
|  | DC_7A_n77A | - | - | - | No issues |
| DC_66_n71-n77 | DC_66A_n71A | 2nd harmonic from B66 into n77 5th harmonic from n71 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_66A_n77A or DC_71A_n77A - 15.9dB MSD by 3rd IMD from CA_n66-n71-n77 can be reused |
|  | DC_66A_n77A | - | 3rd IMD | - | - 15.2dB MSD by 3rd IMD from DC_12A-66A_n77A can be reused |
| DC_5_n2-n41 | DC_5A_n2A | 3rd harmonic from B5 into n41 | 2nd IMD | - | - Harmonic problem already covered in DC_5A_n41A - 29.7dB by 2nd IMD |
|  | DC_5A_n41A | - | - | - | No issues |
| DC_5_n2-n66 | DC_5A_n2A | - | 4th IMD | - | - 7.2dB MSD by 4th IMD from CA_n2-n5-n66 can be reused |
|  | DC_5A_n66A | - | - | - | No issues |
| DC_12_n2-n66 | DC_12A_n2A | 3rd harmonic from B12 into n66 | - | - | - Harmonic problem already covered in DC_12A_n66A |
|  | DC_12A_n66A | - | 4th IMD | - | - 0dB MSD in DC_2A-12A_n66A in TS38.101-3 |
| DC_7_n2-n77 | DC_7A_n2A | 2nd harmonic from n2 into n77 | 5th IMD |  | - Harmonic problem already covered in DC_2A_n77A - 4.2dB MSD by 5th IMD from DC_2A_n7A-n78A can be reused |
|  | DC_7A_n77A | - | 4th IMD | - | - 8.6dB MSD by 4th IMD from DC_2A-7A_n78A can be reused |
| DC_12_n2-n77 | DC_12A_n2A | 2nd harmonic from n2 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_2A_n77A - 16.0dB MSD by 3rd IMD from CA_n2-n12-n77 can be reused |
|  | DC_12A_n77A | - | 3rd & 4th IMDs | - | - 16.5dB MSD by 3rd IMD from DC_2A-12A_n77A can be reused |
| DC_71_n2-n77 | DC_71A_n2A | 2nd harmonic from n2 into n77 5th harmonic from B71 into n77 | 3rd, 4th & 5th IMDs | - | - Harmonic problem already covered in DC_2A_n77A or DC_71A_n77A - 8.0 dB MSD by 3rd IMD from DC_2A_n71A-n78A |
|  | DC_71A_n77A | - | 3rd & 4th IMDs | - | - 16.5dB MSD by 3rd IMD from DC_2A-71A_n78A can be reused |
| DC_5_n2-n78 | DC_5A_n2A | 2nd harmonic from n2 into n78 4th harmonic from B5 into n78 | 3rd IMD | - | - Harmonic problem already covered in DC_2A_n78A or DC_5A_n78A - 16.0dB MSD by 3rd IMD from DC_2A_n5A-n77A can be reused |
|  | DC_5A_n78A |  | 3rd IMD |  | - 16.5dB MSD by 3rd IMD from DC_2A-5A_n77A can be reused |
| DC_12_n2-n78 | DC_12A_n2A | 2nd harmonic from n2 into n78 | 3rd IMD | - | - Harmonic problem already covered in DC_2A_n77A - MSD will follow from DC_12_n2-n77 |
|  | DC_12A_n78A | - | 3rd IMD | - | - MSD will follow from DC_12_n2-n77 |
| DC_2-2_n41-n66 | DC_2A_n41A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
| DC_2-2_n41-n71 | DC_2A_n41A | - | 2nd & 5th IMDs | - | - 28.7dB MSD by 2nd IMD from DC_2A_n41A-n71A can be reused |
|  | DC_2A_n71A | 4th harmonic from n71 into n41 | - | - | - Harmonic problem already covered in DC_71A_n41A |
| DC_2-2_n66-n78 | DC_2A_n66A | 2nd harmonic from B2 or n66 into n78 | 2nd & 4th IMDs into n78 |  | - Harmonic issue already covered in DC_2_n78 or CA_n66-n78 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_2A_n78A | - | 4th IMD into n66 | - | - 10.3 dB MSD by 4th IMD in DC_2A-66A_n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
| DC_2-5_n2-n41 | DC_2A_n2A | - | - | - | No issues when RAN4 only consider the single switched uplink transmission |
|  | DC_2A_n41A | - | - | - | No issue |
|  | DC_5A_n2A | 3rd harmonics from n2 into n41 | 2nd IMD into n41 |  | - Harmonic issue already covered in DC_2_n41 - 29.7dB by 2nd IMD |
|  | DC_5A_n41A | - | - | - | No issues |
| DC_2-12_n2-n41 | DC_2A_n2A | - | - | - | No issues when RAN4 only consider the single switched uplink transmission |
|  | DC_2A_n41A | - | 2nd & 5th IMDs into B12 | - | - 28.7dB MSD by 2nd IMD from DC_2A-12A_n41A can be reused |
|  | DC_12A_n2A | 3rd harmonics from n2 into n41 | 2nd IMD into n41 |  | - Harmonic issue already covered in DC_2_n41 - 28.7 dB by IMD2 from DC_12A_n2A-n41A can be reused |
|  | DC_12A_n41A | - | 2nd IMD into B2 and n2 | - | - 26.0dB MSD by 2nd IMD from DC_2A-12A_n41A can be reused |
| DC_2-66_n2-n41 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n41A | - | - | - | No issues |
|  | DC_66A_n2A | - | - | - | No issues |
|  | DC_66A_n41A |  | 4th IMD into B2 and n2 | - | - 11.0dB MSD by 2nd IMD from DC_2A-66A_n41A can be reused |
| DC_2-71_n2-n41 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n41A | - | 2nd & 5th IMDs into B71 | - | - 28.7dB MSD by 2nd IMD from DC_2A_n41A-n71A can be reused |
|  | DC_71A_n2A | 4th harmonic from B71 into n41 | - | - | - Harmonic problem already covered in DC_71A_n41A |
|  | DC_71A_n41A | 3rd harmonic from B71 into B2/n2 | 2nd IMD into B2 and n2 | - | - Harmonic issue already covered in DC_2_n71 - 26.0dB MSD by DC_71A_n2A-n41A can be reused in TS38.101-3 |
| DC_2-5_n2-n66 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
|  | DC_5A_n2A | - | 4th IMD into n66 | - | - 7.2dB MSD by 4th IMD from CA_n2-n5-n66 can be reused |
|  | DC_5A_n66A | - | - | - | No issues |
| DC_2-7_n2-n66 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
|  | DC_7A_n2A | - | - | - | No issues |
|  | DC_7A_n66A | - | - | - | No issues |
| DC_2-12_n2-n66 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
|  | DC_12A_n2A | 3rd harmonics from B12 into n66 | - | - | - Harmonic problem already covered in DC_12A_n66A |
|  | DC_12A_n66A | - | 4th IMD into B2 and n2 |  | - 0dB MSD in DC_2A-12A_n66A in TS38.101-3 |
| DC_2-71_n2-n66 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n66A | - | - | - | No issues |
|  | DC_71A_n2A | - | - | - | No issues |
|  | DC_71A_n66A | 3rd harmonics from B71 into B2 and n2 | - | - | - Harmonic problem already covered in DC_2A_n71A |
| DC_2-7_n2-n71 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n71A | - | - | - | No issues |
|  | DC_7A_n2A | - | 2nd & 5th IMDs into n71 | - | - 28.7dB MSD for 2nd IMD of DC_7A_n2A-n71A can be reused in TS38.101-3 |
|  | DC_7A_n71A | 3rd harmonics from n71 into B2 and n2 | - | - | - Harmonic problem already covered in DC_2A_n71A |
| DC_2-66_n2-n71 | DC_2A_n2A | - | - | - | No issues |
|  | DC_2A_n71A | - | - | - | No issues |
|  | DC_66A_n2A | - | - | - | No issues |
|  | DC_66A_n71A | 3rd harmonics from B71 into B2 and n2 | - | - | - Harmonic problem already covered in DC_2A_n71A |
| DC_2-7_n2-n77 | DC_2A_n2A | 2nd harmonic from B2 into n77 | 2nd & 4th IMDs into n77 | - | - Harmonic problem already covered in DC_2A_n77A - Only switched uplink transmission is allowed |
|  | DC_2A_n77A | - | 5th IMD into B7 |  | - 3.4dB MSD for 5th IMD of DC_2A-7A_n77A can be reused in TS38.101-3 |
|  | DC_7A_n2A | 2nd harmonic from n2 into n77 | 5th IMD into n77 |  | - Harmonic problem already covered in DC_2A_n77A - 4.2dB MSD for 5th IMD of DC_2A_n7A-n78A can be reused in TS38.101-3 |
|  | DC_7A_n77A | - | 4th IMD into B2 and n2 | - | - 8.6dB MSD for 4th IMD of DC_2A-7A_n77A can be reused in TS38.101-3 |
| DC_2-12_n2-n77 | DC_2A_n2A | 2nd harmonic from B2 into n77 | 2nd & 4th IMDs into n77 | - | - Harmonic problem already covered in DC_2A_n77A - Only switched uplink transmission is allowed |
|  | DC_2A_n77A | - |  |  | No issues |
|  | DC_12A_n2A | 2nd harmonic from n2 into n77 | 3rd , 4th & 5th IMDs into n77 |  | - Harmonic problem already covered in DC_2A_n77A - 16.0dB MSD by 3rd IMD from CA_n2-n12-n77 can be reused |
|  | DC_12A_n77A | - | 3rd & 4th IMDs into B2 and n2 | - | - 16.5dB MSD by 3rd IMD from DC_2A-12A_n77A can be reused |
| DC_2-71_n2-n77 | DC_2A_n2A | 2nd harmonic from B2 into n77 | 2nd & 4th IMDs into n77 | - | - Harmonic problem already covered in DC_2A_n77A - Only switched uplink transmission is allowed |
|  | DC_2A_n77A |  |  |  | No issues |
|  | DC_71A_n2A | 2nd harmonic from n2 into n77 5th harmonic from B71 into n77 | 3rd, 4th & 5th IMDs into n77 | - | - Harmonic problem already covered in DC_2A_n77A or DC_71A_n77A - 8.0dB MSD |
|  | DC_71A_n77A | - | 3rd & 4th IMDs into B2and n2 | - | - 16.5dB MSD by 3rd IMD from DC_2A-71A_n78A can be reused |
| DC_2-5_n2-n78 | DC_2A_n2A | 2nd harmonic from B2 into n78 | 2nd & 4th IMDs into n78 | - | - Harmonic problem already covered in DC_2A_n78A - Only switched uplink transmission is allowed |
|  | DC_2A_n78A |  | 5th IMD into B5 | - | - 3.8 dB by 5th IMD from DC_2A-5A_n77A can be reused |
|  | DC_5A_n2A | 2nd harmonic from n2 into n78 4th harmonic from B5 into n78 | 3rd IMD into n78 | - | - Harmonic problem already covered in DC_2A_n78A or DC_5A_n78A - 16.0dB MSD by 3rd IMD from DC_2A_n5A-n77A can be reused |
|  | DC_5A_n78A |  | 3rd IMD into B2 and n2 |  | - 16.5dB MSD by 3rd IMD from DC_2A-5A_n77A can be reused |
| DC_2-66_n2-n78 | DC_2A_n2A | 2nd harmonic from B2 into n78 | 2nd & 4th IMDs into n78 | - | - Harmonic problem already covered in DC_2A_n78A - Only switched uplink transmission is allowed |
|  | DC_2A_n78A | - | 4th IMD into n66 | - | - 10.3 dB MSD by 4th IMD in DC_2A-66A_n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n2A | 2nd harmonic from n2 into n78 | 2nd & 4th IMDs into n78 |  | - Harmonic issue already covered in DC_2_n78 or CA_n66-n78 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n78A |  | 2nd, 4th & 5th IMDs into B2 and n2 |  | - 32.1dB MSD by 2nd IMD from DC_2A-66A_n78A can be reused |
| DC_66_n2-n78 | DC_66A_n2A | 2nd harmonic from n2 into n78 | 2nd & 4th IMDs into n78 |  | - Harmonic issue already covered in DC_2_n78 or CA_n66-n78 - 29.4 dB and 8.9 dB MSD by 2nd & 4th IMDs in DC_2A_n66A-n78A can be reused in Table 7.3B.2.3.5.2-1 in TS38.101-3 |
|  | DC_66A_n78A |  | 2nd, 4th & 5th IMDs into B2 and n2 |  | - 32.1dB MSD by 2nd IMD from DC_2A-66A_n78A can be reused |
| DC_38_n3-n78 | DC_38A_n3A | 2nd harmonic from n3 into n78 | 3rd IMD into n78 |  | - Harmonic issue already covered in DC_3_n78 - 16.1dB MSD by 3rd IMD from DC_3A_n7A-n78A can be reused |
|  | DC_38A_n78A |  | 3rd & 4th IMDs into n3 |  | - 17.6dB MSD by 3rd IMD from DC_3A-7A_n78A can be reused |
| DC_1_n38-n78 | DC_1A_n78A | - | 4th IMD into n38 |  | - 9.1dB MSD by 4th IMD from DC_1A-7A_n78A can be reused |
| DC_3_n38-n78 | DC_3A_n78A | - | - |  | No issues |
| DC_7_n38-n78 | - | - | - |  | No issues |
| DC_20_n38-n78 | DC_20A_n78A | 3rd harmonic from B20 into n38 | 2nd IMD into n38 | - | - Harmonic issue already covered in DC_20_n38 - 30.8dB MSD by 2nd IMD from DC_7A-20A_n78A can be reused |
| DC_1_n28-n75 | DC_1A_n28A | 2nd harmonic from n28 into n75 | - | - | - Harmonic issue already covered in DC_28_n75 |
| DC_3_n28-n75 | DC_3A_n28A | 2nd harmonic from n28 into n75 | 5th IMD into n75 | - | - Harmonic issue already covered in DC_28_n75 - 3.3dB by 5th IMD |
| DC_20_n8-n78 | DC_20A_n8A | 4th harmonic from B20 or n8 into n78 | 4th IMD into n78 |  | - Harmonic issue already covered in DC_20_n78 or DC_8_n78 - 10.3 dB reuse by 4th IMD of DC_8A_n28A-n78A |
|  | DC_20A_n78A | - | 4th IMD into n8 |  | - 12.1 dB reuse by 4th IMD of DC_8A-20A_n78A |
| DC_1_n3-n8 | DC_1A_n3A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_1A_n8A | 2nd harmonic from n8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
| DC_1_n8-n77 DC_1_n8-n77(2A) DC_n8-n77_1 DC_n8-n77(2A)_1 | DC_1A_n8A or DC_n8A_1A |  | 3rd IMD into n77 | - | - 14.9 dB MSD by 3rd IMD of DC_1A_n8A-n78A can be reused. |
|  | DC_1A_n77A or DC_n77A_1A | - | 5th IMD into n8 | - | - 3.3dB MSD by 5th IMD of DC_1A-8A_n77A can be reused. |
| DC_1-3_n3-n8 | DC_1A_n3A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_1A_n8A | 2nd harmonic from n8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n8A | - | - | - | No issues |
| DC_3_n3-n77 DC_3_n3-n77(2A) DC_n3-n77_3 DC_n3-n77(2A)_3 | DC_(n)3AA | 2nd harmonic from n3 into n77 | 2nd & 4th IMD into n77 | - | - Harmonic problems was already covered in DC_3A_n77A. - Due to apply MPR, no need to define MSD |
|  | DC_3A_n77A DC_n77A_3A | - | 2nd, 4th &5th IMD into n3 | - | - MSD problems in n3 were already solved in DC_3A_n77A |
| DC_3_n3-n8 DC_n3-n8_3 | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n8A or DC_n8A_3A | - | - | - | No issues |
| DC_3_n8-n77 DC_3_n8-n77(2A) DC_n8-n77_3 DC_n8-n77(2A)_3 | DC_3A_n8A or DC_n8A_3A | 2nd harmonic from B3 into n77 4th harmonic from n8 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems was already covered in DC_3A_n77A or DC_8A_n77A. - 16.3dB MSD by 3rd IMD in DC_3A_n8A-n78A can be reused |
|  | DC_3A_n77A or DC_n77A_3A | - | 4th IMD into n8 |  | - 9.7 dB MSD by 4th IMD in DC_3A-8A_n77A can be reused |
| DC_8-3_n3-n77 DC_8-3_n3-n77(2A) | DC_(n)3AA | 2nd harmonic from B3 into n77 | 2nd & 4th IMDs into n77 |  | - Harmonic problems was already covered in DC_3A_n77A - Due to apply MPR, no need to define MSD |
|  | DC_3A_n77A | - | 4th IMD into n8 |  | - 9.7 dB MSD by 4th IMD in DC_3A-8A_n77A can be reused |
|  | DC_8A_n3A | 2nd harmonic from B3 into n77 4th harmonic from n8 into n77 | 3rd & 5th IMDs into n77 |  | - Harmonic problems was already covered in DC_3A_n77A or DC_8A_n77A. - 16.3dB MSD by 3rd IMD in DC_3A_n8A-n78A can be reused |
|  | DC_8A_n77A | 2nd harmonic from B8 into B3 | 3rd IMD into B3 and n3 |  | - Harmonic problems was already covered in DC_3A_n8A - 16.5dB MSD by 3rd IMD in DC_3A-8A_n77A can be reused |
| DC_1-3_n3-n257A/G/H/I/J/K/L/M | DC_1A_n3A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_1A_n257A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n257A | - | - | - | No issues |
| DC_1_n77-n257J/K/L/M DC_1_n77(2A)-n257J/K/L/M DC_n77-n257_1 DC_n77(2A)-n257_1 | DC_1A_n77A or DC_n77A_1A | - | - | - | No issues |
|  | DC_1A_n257A or DC_n257A_1A | - | - | - | No issues |
| DC_1_n8-n257A/G/H/I/J/K/L/M DC_n8-n257_1 | DC_1A_n8A or DC_n8A_1A | - | - | - | No issues |
|  | DC_1A_n257A or DC_n257A_1A | - | - | - | No issues |
| DC_3_n3-n257A/G/H/I/J/K/L/M DC_n3-n257_3 | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n257A or DC_n257A_3A | - | - | - | No issues |
| DC_3_n77-n257J/K/L/M, DC_3_n77(2A)-n257J/K/L/M DC_n77-n257_3 DC_n77(2A)-n257_3 | DC_3A_n77A or DC_n77A_3A | - | - | - | No issues |
|  | DC_3A_n257A or DC_n257A_3A | - | - | - | No issues |
| DC_3_n8-n257A/G/H/I/J/K/L/M DC_n8-n257_3 | DC_3A_n8A or DC_n8A_3A | - | - | - | No issues |
|  | DC_3A_n257A or DC_n257A_3A | - | - | - | No issues |
| DC_8_n3-n257A/G/H/I/J/K/L/M DC_n3-n257G/H/I/J/K/L/M_8 | DC_8A_n3A DC_n3A_8A | - | - | - | No issues |
|  | DC_8A_n257A DC_n257A_8A | 2nd harmonic from B8 into n3 | - | - | - Harmonic problems was already covered in DC_3A_n8A |
| DC_8-3_n3-n257A/G/H/I/J/K/L/M | DC_8A_n3A | - | - | - | No issues |
|  | DC_8A_n257A | 2nd harmonic from B8 into n3 | - | - | - Harmonic problems was already covered in DC_3A_n8A |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_3A_n257A | - | - | - | No issues |
| DC_n3-n257A/G/H/I/J/K/L/M_1-3 | DC_n3A_1A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_n257A_1A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_n257A_3A | - | - | - | No issues |
| DC_n3-n77_1-3 DC_n3-n77(2A)_1-3 | DC_n3A_1A | 2nd harmonic from B1 or n3 into n77 | 2nd & 4th IMDs into n77 | Yes | - Harmonic problems already covered in DC_n3A-n78A. - Same as IMD issues in DC_1A_n3A-n78A - Small freq. gap was covered in Table 7.3.1A-0bA in TS36.101 |
|  | DC_(n)3AA | 2nd harmonic from B3 into n77 | 2nd & 4th IMDs into n77 |  | - Harmonic problems was already covered in DC_3A_n77A - Due to apply MPR, no need to define MSD |
|  | DC_n77A_1A | - | 2nd , 4th & 5th IMDs into B3/n3 | - | - Same as IMD issues in DC_1A_n3A-n78A |
|  | DC_n77A_3A | - | 2nd & 5th IMD into B1 2nd, 4th &5th IMD into n3 | - | - 31.0 dB by 2nd IMD that follow DC_1A-3A_n77A in TR37.863-02-01 in rel-15. - Follow DC_3A_n1A-n78A for 5th IMD problem. - MSD problems in n3 were already solved in DC_3A_n77A |
| DC_n3-n8_1 | DC_n3A_1A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_n8A_1A | 2nd harmonic from n8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
| DC_n3-n8_1-3 | DC_n3A_1A | - | - | Yes | - Cross band isolation issues already solved |
|  | DC_(n)3AA | - | - | - | No issues |
|  | DC_n8A_1A | 2nd harmonic from n8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
|  | DC_n8A_3A | - | - | - | No issues |
| DC_7_n79-n257A/G/H/I/J/K/L/M | DC_7A_n79A | 6th harmonic from n79 into n257 | - | - | - No harmonic problems by 6th order between FR1 and FR2 The DC_7A_n79A is still open in TS38.101-3 |
|  | DC_7A_n257A/G/H/I/J/K/L/M | 2nd harmonic from B7 into n79 | - | - | DC_7A_n79A will be solved the harmonic problem. |
| DC_7_n79-n258A/G/H/I/J/K/L/M | DC_7A_n79A | 5th & 6th harmonics from n79 into n258 | - | - | - No harmonic problems by 5th order between FR1 and FR2 The DC_7A_n79A is still open in TS38.101-3 |
|  | DC_7A_n258A/G/H/I/J/K/L/M | 2nd harmonic from B7 into n79 | - | - | DC_7A_n79A will be solved the harmonic problem. |
| DC_3_n78-n79C | DC_3A_n78A |  | 3rd IMD into n79 |  | - Reuse 16.3dB of DC_3A_n78A-n79A by dual uplink DC_3A_n78A in TS38.101-3 |
|  | DC_3A_n79A | 2nd harmonic from B3 into n78 | 5th IMD into n78 |  | - 2nd harmonic issue already solved in DC_3A_n78A. - Reuse 4.2dB of DC_3A_n78A-n79A by dual uplink DC_3A_n79A in TS38.101-3 |
| DC_7_n78-n79 | DC_7A_n78A | - | 3rd, 4th & 5th IMDs into n79 | - | - 16.7 by 3rd IMD |
|  | DC_7A_n79A | - | 4th IMDs into n78 | - | - 10.4 by 4th IMD |
| DC_8_n1-n28 | DC_8A_n1A | - | 4th IMD into n28 | - | - 11.6dB by 4th IMD |
|  | DC_8A_n28A | 3 harmonic from n28 into n1 | 5th IMD into n1 | - | - Harmonic problem already covered in DC_1A_n28A - When consider the 5MHz UL CBW for DC_8_n28, the 5th IMD does not fall into the B1 own Rx band. This was decision made for DC_1-8_n28 |
| DC_3_n78-n257J/K/L/M DC_3-3_n78-n257G/H/I/J/K/L/M | DC_3A_n78A | 7th & 8th harmonics from n78 into n257 | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_3A_n257A | 2nd harmonic from B3 into B78 | - | - | - Harmonic problem already covered in DC_3A_n78A |
| DC_7_n78-n257G/H/I/J/K/L/M DC_7-7_n78-n257G/H/I/J/K/L/M | DC_7A_n78A | 7th & 8th harmonic from n78 into n257 |  |  | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_7A_n257A | - | - | - | - No issue |
| DC_8_n1-n257 | DC_8A_n1A | - | - | - | - No harmonic problems by 7th & 8th order between FR1 and FR2 |
|  | DC_8A_n257A | - | - | - | - No issue |
| DC_42_n78-n257 | DC_42A_n257A | - | - | - | - No IMD problem due to synchronous operation between B42 and n78 |
| DC_42_n77-n257 | DC_42A_n257A | - | - | - | - No IMD problem due to synchronous operation between B42 and n77 |
| DC_20_n3-n78 | DC_20A_n3A | 4th harmonic from B20 into n78 | 3rd & 5th IMDs into n78 |  | - Reuse 16.8dB of DC_3A_n20A-n78A by 3rd IMD in TS38.101-3 |
|  | DC_20A_n78A |  | 3rd IMD into n3 |  | - Reuse 17.3dB of DC_3A-20A_n78A by 3rd IMD in TS38.101-3 |
| DC_20_n3-n67 | DC_20A_n3A | 4th harmonic from B20 into n78 | 3rd & 5th IMDs into n78 |  | - Reuse 16.8dB of DC_3A_n20A-n78A by 3rd IMD in TS38.101-3 |
| DC_2_n12-n78 DC_2-2_n12-n78 | DC_2A_n12A | 2nd harmonic from B2 and 5th harmonic from n12 fall into n78 | 3rd IMD into n78 | - | - Harmonic problem already covered in DC_2A_n78A or DC_12A_n78A - Reuse 16.0dB of CA_n2A-n12A-n77A by 3rd IMD in TS38.101-1 |
|  | DC_2A_n78A | - | - | - | - No issue |
| DC_7_n12-n78 | DC_7A_n12A | 5th harmonic from n12 fall into n78 | 4th IMD into n78 |  | - Harmonic problem already covered in DC_12A_n78A - Reuse 9.7dB of CA_n7A-n28A-n78A in TS38.101-1. |
|  | DC_7A_n78A | - | 2nd IMD into n78 | - | - Harmonic problem already covered in DC_2A_n78A or DC_12A_n78A - Reuse 30.8dB of DC_7A-12A_n78A by 2nd IMD in TS38.101-3 |
| DC_66_n12-n78 | DC_66A_n12A | 2nd harmonic from B66 and 5th harmonic from n12 fall into n78 | 5th IMD into n78 |  | - Harmonic problem already covered in DC_66A_n78A or DC_12A_n78A - Reuse 4.1dB of DC_12A_n66A-n78A by 5th IMD in TS38.101-3 |
|  | DC_66A_n78A | - | - | - | - No issue |
| DC_1_n77-n79 | DC_1A_n77A | - | 3rd,4th & 5th IMD fall into n79 |  | - Reuse 15.9dB of DC_1A_n78A-n79A by 3rd IMD in TS38.101-3 |
|  | DC_1A_n79A | 2nd harmonic from B1 fall into n77 | 5th IMD fall into n77 |  | - Harmonic problem already covered in DC_1A_n77A - Reuse 4.6dB of DC_1A_n78A-n79A by 5th IMD in TS38.101-3 |
| DC_8_n1-n3 | DC_8A_n1A | 2nd harmonic from B8 into n3 | - | Yes | - Harmonic issue already covered in DC_8_n3A - Cross band isolation issues already solved |
|  | DC_8A_n3A | - | - | - | - No issue |
| DC_8_n1-n77 DC_8_n1-n77(2A) | DC_8A_n1A |  | 3rd IMD into n77 | - | - 14.9 dB MSD by 3rd IMD of DC_1A_n8A-n78A can be reused. |
|  | DC_8A_n77A | - | 3rd IMD into n1 |  | - 14.4dB MSD by 3rd IMD in DC_1A-8A_n77A can be reused |
| DC_11_n1-n77 DC_11_n1-n77(2A) | DC_11A_n1A | 2nd harmonic from B11 into n77 | 2nd & 5th IMDs into n77 |  | - Harmonic issue already covered in DC_11_n77A - 29.6 dB by 2nd IMD |
|  | DC_11A_n77A | - | 2nd IMD into n1 |  | - Reuse 30.8dB MSD by 2nd IMD in DC_1A-11A_n77A in TS38.101-3 |
| DC_11_n77-n79 DC_11_n77(2A)-n79 | DC_11A_n77A | - | 2nd & 4th IMDs into n79 | Yes | - If simultaneous Rx/Tx capability is supported, need to define MSD requirements. Else then, not need to define MSD |
|  | DC_11A_n79A | - | 2nd IMD into n77 | Yes | - If simultaneous Rx/Tx capability is supported, need to define MSD requirements. Else then, not need to define MSD |
| DC_41_n1-n3 | DC_41A_n1A | - | - | Yes | The cross band isolation issue already covered in TS38.101-1 |
|  | DC_41A_n3A | - | - | - | - No issue |
| DC_41_n1-n77 | DC_41A_n1A | 2nd harmonic from n1 into n77 | 3rd , 4th & 5th IMDs into n77 | - | - The harmonic problem already covered in DC_1_n77 in TS38.101-3 - Reuse 19.6dB by IMD3 in DC_1A_n41A-n78A in TS38.101-3 |
|  | DC_41A_n77A | - | 4th IMD into n1 | - | - Reuse 9.3dB by 4th IMD of DC_1A-41A_n77A in TS38.101-3 |
| DC_42_n1-n3 | DC_42A_n1A | - | - | Yes | The cross band isolation issue already covered in TS38.101-1 |
|  | DC_42A_n3A | - | - | - | - No issue |
| DC_42_n1-n77 | DC_42A_n1A | 2nd harmonic from n1 into n77 | 4th & 5th IMDs into n77 | Yes | - Harmonic issue already covered in DC_1A_n77A - RAN4 do not allow the simultaneous Rx/Tx for DC_42A_n77A. So there was no self desense issue. |
| DC_3_n20-n67 | DC_3A_n20A |  | - 4th IMD into n67 |  | - 10.1 dB by 4th IMD |
| DC_11_n3-n79 | DC_11A_n3A | - | 3rd IMD into n79 | - | - 16.2 dB by 3rd IMD |
|  | DC_11A_n79A |  | 3rd IMD into n3 |  | - Reuse 17.8dB by 3rd IMD in DC_3A-21A_n79A in TS38.101-3 |

For the high order xDL/2UL (x=4,5,6) DC combinations, the self-interference problems will be covered in 3DL/2UL or lower order DC operation. Hence, RAN4 do not make summary tables for high order xDL/2UL DC band combinations.

Based on these assumptions, we proposed the MSD test configuration and required MSD levels by self-interference as below.

Table 5.3-2: Proposed MSD levels and MSD test configuration for 3DL/2UL DC UE

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_40A_n1A-n78A DC_40C_n1A-n78A | B40 | IMD4 | ¦2*fB40 -2*fn78¦ | 2320 | 5 | 25 | 2320 | 5 | N/A |
|  | n78 |  |  | 3380 | 10 | 50 | 3380 | 10 |  |
|  | n1 |  |  | 1930 | 5 | 25 | 2120 | 5 | 7.6 |
| DC_7A_n40A-n78A | B7 | IMD4 | ¦2*fB7 -2*fn78¦ | 2520 | 10 | 50 | 2640 | 10 | N/A |
|  | n78 |  |  | 3700 | 10 | 50 | 3700 | 10 |  |
|  | n40 |  |  | 2360 | 5 | 25 | 2360 | 5 | 9.2 |
| DC_8A_n40A-n78A | 8 | IMD2 | ¦fB8 +fn40¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n40 |  |  | 2395 | 5 | 25 | 2395 | 5 |  |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 | 28.7 |
|  | 8 | IMD3 | ¦fB8 -2*fn40¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n40 |  |  | 2340 | 5 | 25 | 2340 | 5 |  |
|  | n78 |  |  | 3370 | 10 | 50 | 3370 | 10 | 16.9 |
|  | 8 | IMD2 | ¦fB8 -fn78¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 |  |
|  | n40 |  |  | 2395 | 5 | 25 | 2395 | 5 | 27.3 |
| DC_11A_n28A-n77A DC_11A_n28A-n77(2A) | 11 | IMD3 | ¦2*fB11 +fn28¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77 |  |  | 3629 | 10 | 50 | 3629 | 10 | 17.5 |
|  | 11 | IMD4 | ¦3*fB11 -fn28¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77 |  |  | 3586 | 10 | 50 | 3586 | 10 | 11.2 |
|  | 11 | IMD3 | ¦2*fB11 - fn77¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n77 |  |  | 3684 | 10 | 50 | 3684 | 10 |  |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 | 15.8 |
|  | 11 | IMD4 | ¦3*fB11 -fn77¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n77 |  |  | 3564 | 10 | 50 | 3564 | 10 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 11.0 |
| DC_2A_n5A-n77A DC_2A-2A_n5A-n77A | 2 | IMD3 | ¦fB2 +2*fn5¦ | 1880 | 5 | 25 | 1960 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3540 | 10 | 50 | 3540 | 10 | 16.0 |
|  | 2 | IMD5 | ¦3*fB2 -2*fn5¦ | 1880 | 5 | 25 | 1960 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3980 | 10 | 50 | 3980 | 10 | 4.1 |
|  | 2 | IMD5 | ¦3*fB2 -2*fn77¦ | 1907 | 5 | 25 | 1987 | 5 | N/A |
|  | n77 |  |  | 3305 | 10 | 50 | 3305 | 10 |  |
|  | n5 |  |  | 844 | 5 | 25 | 889 | 5 | 3.8 |
| DC_13A_n2A-n77A | 13 | IMD3 | ¦2*fB13 +fn2¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n2 |  |  | 1880 | 5 | 25 | 1960 | 5 |  |
|  | n77 |  |  | 3444 | 10 | 50 | 3444 | 10 | 17.3 |
|  | 13 | IMD4 | ¦3*fB13 +fn2¦ | 780 | 5 | 25 | 749 | 5 | N/A |
|  | n2 |  |  | 1855 | 5 | 25 | 1935 | 5 |  |
|  | n77 |  |  | 4195 | 10 | 50 | 4195 | 10 | 12.4 |
|  | 13 | IMD5 | ¦2*fB13 -3*fn2¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n2 |  |  | 1880 | 5 | 25 | 1960 | 5 |  |
|  | n77 |  |  | 4076 | 10 | 50 | 4076 | 10 | 4.5 |
|  | 13 | IMD3 | ¦2*fB13 -fn77¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n77 |  |  | 3524 | 10 | 50 | 3524 | 10 |  |
|  | n2 |  |  | 1880 | 5 | 25 | 1960 | 5 | 16.0 |
| DC_66A_n5A-n77A DC_66A-66A_n5A-n77A | 66 | IMD3 | ¦fB66 +2*fn5¦ | 1760 | 5 | 25 | 2160 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3420 | 10 | 50 | 3420 | 10 | 16.6 |
|  | 66 | IMD4 | ¦fB66 +3*fn5¦ | 1714 | 5 | 25 | 2114 | 5 | N/A |
|  | n5 |  |  | 827 | 5 | 25 | 872 | 5 |  |
|  | n77 |  |  | 4195 | 10 | 50 | 4195 | 10 | 12.1 |
|  | 66 | IMD5 | ¦3*fB66 -2*fn5¦ | 1760 | 5 | 25 | 2160 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3620 | 10 | 50 | 3620 | 10 | 3.6 |
| DC_18A_n28A-n77A DC_18A_n28A-n78A | 18 | IMD5 | ¦2*fB18 +3*fn28¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 |  |
|  | n77/n78 |  |  | 3770 | 10 | 50 | 3770 | 10 | 4.0 |
| DC_18A_n28A-n77A | 18 | IMD5 | ¦4*fB18 -fn77¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n77 |  |  | 4045 | 10 | 50 | 4045 | 10 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 5.7 |
| DC_18A_n3A-n41A | 18 | IMD2 | ¦fB18 +fn3¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n41 |  |  | 2540 | 5 | 25 | 2540 | 5 | 29.4 |
|  | 18 | IMD2 | ¦fB18 -fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2655 | 5 | 25 | 2655 | 5 |  |
|  | n3 |  |  | 1740 | 5 | 25 | 1835 | 5 | 28.2 |
| DC_18A_n28A-n41A | 18 | IMD5 | ¦4*fB18 -fn28¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 |  |
|  | n41 |  |  | 2570 | 5 | 25 | 2570 | 5 | 5.1 |
|  | 18 | IMD5 | ¦4*fB18 -fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 4.7 |
| DC_18A_n41A-n77A DC_18A_n41A-n78A | 18 | IMD2 | ¦fB18 +fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2570 | 5 | 25 | 2570 | 5 |  |
|  | n77/n78 |  |  | 3390 | 10 | 50 | 3390 | 10 | 30.1 |
|  | 18 | IMD3 | ¦2*fB18 +fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n77 |  |  | 4155 | 10 | 50 | 4155 | 10 | 16.4 |
|  | 18 | IMD4 | ¦2*fB18 -2*fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n77/n78 |  |  | 3390 | 10 | 50 | 3390 | 10 | 9.1 |
|  | 18 | IMD2 | ¦fB18 -fn78¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n77/n78 |  |  | 3450 | 10 | 50 | 3450 | 10 |  |
|  | n41 |  |  | 2630 | 5 | 25 | 2630 | 5 | 28.5 |
| DC_12A_n5A-n77A | 12 | IMD5 | ¦4*fB12 +fn5¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3662 | 10 | 50 | 3662 | 10 | 5.4 |
|  | 12 | IMD5 | ¦4*fB12 -fn77¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n77 |  |  | 3707 | 10 | 50 | 3707 | 10 |  |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 | 4.8 |
| DC_14A_n5A-n77A | 14 | IMD4 | ¦fB14 +3*fn5¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n5 |  |  | 845 | 5 | 25 | 890 | 5 |  |
|  | n77 |  |  | 3328 | 10 | 50 | 3328 | 10 | 10.6 |
|  | 14 | IMD5 | ¦fB14 +4*fn5¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 4113 | 10 | 50 | 4113 | 10 | 5.1 |
|  | 14 | IMD5 | ¦4*fB14 -fn77¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n77 |  |  | 4047 | 10 | 50 | 4047 | 10 |  |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 | 4.4 |
| DC_2A_n71A-n78A or DC_71A_n2A-n78A | 2/n2 | IMD3 | ¦fB2 +2*fn71¦ | 1907.5 | 5 | 25 | 1987.5 | 5 | N/A |
|  | 71/n71 |  |  | 695.5 | 5 | 25 | 649.5 | 5 |  |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 | 8.0 |
| DC_7A_n71A-n78A | 7 | IMD4 | ¦2*fB7 -2*fn71¦ | 2550 | 5 | 25 | 2670 | 5 | N/A |
|  | n71 |  |  | 693 | 5 | 25 | 647 | 5 |  |
|  | n78 |  |  | 3714 | 10 | 50 | 3714 | 10 | 9.7 |
|  | 7 | IMD5 | ¦3*fB7 -2*fn78¦ | 2555 | 5 | 25 | 2675 | 5 | N/A |
|  | n78 |  |  | 3520 | 10 | 50 | 3520 | 10 |  |
|  | n71 |  |  | 671 | 5 | 25 | 625 | 5 | 3.9 |
| DC_71A_n38A-n78A | 71 | IMD2 | ¦fB71 +fn38¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n38 |  |  | 2615 | 5 | 25 | 2615 | 5 |  |
|  | n78 |  |  | 3308 | 10 | 50 | 3308 | 10 | 29.1 |
|  | 71 | IMD4 | ¦2*fB71 -2*fn38¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n38 |  |  | 2580 | 5 | 25 | 2580 | 5 |  |
|  | n78 |  |  | 3774 | 10 | 50 | 3774 | 10 | 10.3 |
|  | 71 | IMD2 | ¦fB71 -fn78¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n78 |  |  | 3308 | 10 | 50 | 3308 | 10 |  |
|  | n38 |  |  | 2615 | 5 | 25 | 2615 | 5 | 28.7 |
| DC_5A_n66A-n78A | 5 | IMD3 | ¦fn66 +2*fB5¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n78 |  |  | 3420 | 10 | 50 | 3420 | 10 | 16.6 |
|  | 5 | IMD5 | ¦3*fn66 -2*fB5¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n78 |  |  | 3620 | 10 | 50 | 3620 | 10 | 3.6 |
| DC_71A_n66A-n78A | 71 | IMD3 | ¦2*fB71 -fn78¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n78 |  |  | 3546 | 10 | 50 | 3546 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 15.5 |
| DC_5A_n38A-n66A | 5 | IMD2 | ¦fB5 +fn66¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n38 |  |  | 2590 | 5 | 25 | 2590 | 5 | 28.9 |
|  | 5 | IMD3 | ¦fB5 -2*fn66¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1720 | 5 | 25 | 2120 | 5 |  |
|  | n38 |  |  | 2610 | 5 | 25 | 2610 | 5 | 16.4 |
| DC_12A_n2A-n38A or DC_12A_n2A-n41A | 12 | IMD2 | ¦fn2 +fB12¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 |  |
|  | n38/n41 |  |  | 2608 | 5 | 25 | 2608 | 5 | 28.7 |
| DC_2A_n66A-n77A | 2 | IMD2 | ¦fB2 -fn77¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n77 |  |  | 4060 | 10 | 50 | 4060 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 29.2 |
|  | 2 | IMD4 | ¦3*fB2 -fn77¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n77 |  |  | 3540 | 10 | 50 | 3540 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 10.4 |
|  | 2 | IMD5 | ¦3*fB2 -2*fn77¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n77 |  |  | 3930 | 10 | 50 | 3930 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 4.0 |
| DC_13A_n66A-n77A | 13 | IMD3 | ¦2*fB13 +fn66¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n77 |  |  | 3324 | 10 | 50 | 3324 | 10 | 16.4 |
|  | 13 | IMD4 | ¦3*fB13 +fn66¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n77 |  |  | 4106 | 10 | 50 | 4106 | 10 | 11.7 |
|  | 13 | IMD5 | ¦2*fB13 -3*fn66¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n77 |  |  | 3716 | 10 | 50 | 3716 | 10 | 4.4 |
| DC_11A_n3A-n77A | 11 | IMD4 | ¦fB11 -3*fn3¦ | 1440 | 5 | 25 | 1488 | 5 | N/A |
|  | n3 |  |  | 1740 | 5 | 25 | 1835 | 5 |  |
|  | n77 |  |  | 3780 | 10 | 50 | 3780 | 10 | 10.8 |
|  | 11 | IMD5 | ¦4*fB11 -fn3¦ | 1440 | 5 | 25 | 1488 | 5 | N/A |
|  | n3 |  |  | 1740 | 5 | 25 | 1835 | 5 |  |
|  | n77 |  |  | 4020 | 10 | 50 | 4020 | 10 | 4.1 |
|  | 11 | IMD2 | ¦fB11 -fn77¦ | 1440 | 5 | 25 | 1488 | 5 | N/A |
|  | n77 |  |  | 3310 | 10 | 50 | 3310 | 10 |  |
|  | n3 |  |  | 1775 | 5 | 25 | 1870 | 5 | 29.0 |
|  | 11 | IMD5 | ¦4*fB11 -fn77¦ | 1440 | 5 | 25 | 1488 | 5 | N/A |
|  | n77 |  |  | 3925 | 10 | 50 | 3925 | 10 |  |
|  | n3 |  |  | 1740 | 5 | 25 | 1835 | 5 | 3.7 |
| DC_1A_n28A-n79A | 1 | IMD3 | ¦2*fB1 -fn79¦ | 1930 | 5 | 25 | 2120 | 5 | N/A |
|  | n28 |  |  | 733 | 5 | 25 | 788 | 5 | 15.2 |
|  | n79 |  |  | 4648 | 40 | 216 | 4648 | 40 | N/A |
|  | 1 | IMD3 | ¦2*fB1 +fn28¦ | 1950 | 5 | 25 | 2140 | 5 | N/A |
|  | n28 |  |  | 730 | 5 | 25 | 785 | 5 |  |
|  | n79 |  |  | 4630 | 40 | 216 | 4630 | 40 | 14.9 |
| DC_3A_n28A-n79A | 3 | IMD4 | ¦3*fB3 -fn79¦ | 1770 | 5 | 25 | 1865 | 5 | N/A |
|  | n28 |  |  | 725 | 5 | 25 | 780 | 5 | 10.3 |
|  | n79 |  |  | 4530 | 40 | 216 | 4530 | 40 | N/A |
|  | 3 | IMD4 | ¦3*fB3 -fn28¦ | 1770 | 5 | 25 | 1865 | 5 | N/A |
|  | n28 |  |  | 725 | 5 | 25 | 780 | 5 |  |
|  | n79 |  |  | 4585 | 40 | 216 | 4585 | 40 | 9.4 |
| DC_21A_n28A-n77A DC_21A_n28A-n78A | 21 | IMD3 | ¦2*fB21 -fn78¦ | 1452 | 5 | 25 | 1500 | 5 | N/A |
|  | n28 |  |  | 730.5 | 5 | 25 | 785.5 | 5 | 16.9 |
|  | n77/n78 |  |  | 3689.5 | 10 | 50 | 3689.5 | 10 | N/A |
|  | 21 | IMD3 | ¦2*fB21 +fn28¦ | 1452 | 5 | 25 | 1500 | 5 | N/A |
|  | n28 |  |  | 730.5 | 5 | 25 | 785.5 | 5 |  |
|  | n77/n78 |  |  | 3634.5 | 10 | 50 | 3634.5 | 10 | 17.3 |
| DC_21A_n28A-n79A | 21 | IMD5 | ¦4*fB21 -fn79¦ | 1450.4 | 5 | 25 | 1498.4 | 5 | N/A |
|  | n28 |  |  | 735.5 | 5 | 25 | 790.5 | 5 | 2.8 |
|  | n79 |  |  | 4980 | 40 | 216 | 4980 | 40 | N/A |
|  | 21 | IMD4 | ¦2*fB21 +2*fn28¦ | 1460.4 | 5 | 25 | 1508.4 | 5 | N/A |
|  | n28 |  |  | 745 | 5 | 25 | 800 | 5 |  |
|  | n79 |  |  | 4420 | 40 | 216 | 4420 | 40 | [6.3] |
| DC_13A_n5A-n77A | 13 | IMD5 | ¦4*fB13 -fn77¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n77 |  |  | 4013 | 10 | 50 | 4013 | 10 |  |
|  | n5 |  |  | 840 | 5 | 25 | 885 | 5 | 4.5 |
| DC_8A_n28A-n79A | 8 | IMD5 | ¦4*fB8 + fn28¦ | 912.5 | 5 | 25 | 957.5 | 5 | N/A |
|  | n28 |  |  | 745.5 | 5 | 25 | 800.5 | 5 |  |
|  | n79 |  |  | 4420 | 40 | 216 | 4420 | 40 | 0.0 |
|  | 8 | IMD5 | ¦4*fB8 - fn79¦ | 905 | 5 | 25 | 950 | 5 | N/A |
|  | n79 |  |  | 4420 | 40 | 216 | 4420 | 5 |  |
|  | n28 |  |  | 745 | 5 | 25 | 800 | 40 | 3.9 |
| DC_8A_n1A-n28A | 8 | IMD4 | ¦3*fB8 -fn1¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n1 |  |  | 1965 | 5 | 25 | 2155 | 5 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 11.6 |
| DC_7A_n78A-n79A | 7 | IMD3 | ¦2*fn78 -fB7¦ | 2520 | 10 | 50 | 2640 | 10 | N/A |
|  | n78 |  |  | 3550 | 10 | 50 | 3550 | 10 |  |
|  | n79 |  |  | 4580 | 40 | 216 | 4580 | 40 | 16.71 |
|  | 7 | IMD4 | ¦2*fn79 -2*fB7¦ | 2565 | 10 | 50 | 2685 | 10 | N/A |
|  | n79 |  |  | 4450 | 40 | 216 | 4450 | 40 |  |
|  | n78 |  |  | 3770 | 10 | 50 | 3770 | 10 | 10.4 |
| DC_11A_n1A-n77A | B11 | IMD2 | ¦fn1 +fB11¦ | 1435 | 5 | 25 | 1483 | 5 | N/A |
|  | n1 |  |  | 1940 | 5 | 25 | 2130 | 5 |  |
|  | n77 |  |  | 3375 | 10 | 50 | 3375 | 10 | 29.62 |
| DC_3A_n20A-n67A | B3 | IMD4 | ¦3*fn20 -fB3¦ | 1765 | 5 | 25 | 1860 | 5 | N/A |
|  | n20 |  |  | 837 | 5 | 25 | 796 | 5 |  |
|  | n67 |  |  | - | - | - | 746 | 5 | 10.1 |
| DC_11A_n3A-n79A | B11 | IMD3 | ¦fn3 +2*fB11¦ | 1435 | 5 | 25 | 1483 | 5 | N/A |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 5 |  |
|  | n79 |  |  | 4640 | 40 | 216 | 4640 | 40 | 16.2 |

# 6	Dual Connectivity band combinations of LTE 1 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

## 6.1	DC_3_n1-n257, DC_3-3_n1-n257

### 6.1.1	Operating bands for DC

Table 6.1.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n257 DC_3-3_n1-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.1.2	Channel bandwidths per operating band for DC

Table 6.1.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A_n1A-n257D DC_3A_n1A-n257E DC_3A_n1A-n257F DC_3A_n1A-n257G DC_3A_n1A-n257H DC_3A_n1A-n257I DC_3A_n1A-n257J DC_3A_n1A-n257K DC_3A_n1A-n257L DC_3A_n1A-n257M | DC_3A_n1A DC_3A_n257A | 3 | -- | 440 640 840 240 340 440 540 640 740 840 |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_3A-3A_n1A-n257D DC_3A-3A_n1A-n257E DC_3A-3A_n1A-n257F DC_3A-3A_n1A-n257G DC_3A-3A_n1A-n257H DC_3A-3A_n1A-n257I DC_3A-3A_n1A-n257J DC_3A-3A_n1A-n257K DC_3A-3A_n1A-n257L DC_3A-3A_n1A-n257M | DC_3A_n1A DC_3A_n257A | 3 | See CA_3A-3A in TS 36.101 | 460 660 860 260 360 460 560 660 760 860 |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |

### 6.1.3	Co-existence studies

Based on co-existence studies of DC_3A_n1A and DC_3A_n257A specified in TR 37.716-11-11, TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 6.1.4	∆TIB and ∆RIB values

For DC_3_n1-n257, DC_3-3_n1-n257, TIB,c and RIB,c values are already mentioned in 37.716-21-21 under the same band combinations with n257A.

### 6.1.5	MSD

No additional MSD requirement is needed.

## 6.2	DC_7_n1-n257, DC_7-7_n1-n257

### 6.2.1	Operating bands for DC

Table 6.2.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n1-n257 DC_7-7_n1-n257 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 6.2.2	Channel bandwidths per operating band for DC

Table 6.2.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_7A_n1A-n257D DC_7A_n1A-n257E DC_7A_n1A-n257F DC_7A_n1A-n257G DC_7A_n1A-n257H DC_7A_n1A-n257I DC_7A_n1A-n257J DC_7A_n1A-n257K DC_7A_n1A-n257L DC_7A_n1A-n257M | DC_7A_n1A DC_7A_n257A | 7 | -- | 440 640 840 240 340 440 540 640 740 840 |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_7A-7A_n1A-n257D DC_7A-7A_n1A-n257E DC_7A-7A_n1A-n257F DC_7A-7A_n1A-n257G DC_7A-7A_n1A-n257H DC_7A-7A_n1A-n257I DC_7A-7A_n1A-n257J DC_7A-7A_n1A-n257K DC_7A-7A_n1A-n257L DC_7A-7A_n1A-n257M | DC_7A_n1A DC_7A_n257A | 3 | See CA_7A-7A in TS 36.101 | 460 660 860 260 360 460 560 660 760 860 |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |

### 6.2.3	Co-existence studies

Based on co-existence studies of DC_7A_n1A and DC_7A_n257A specified in TR 37.716-11-11, TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 6.2.4	∆TIB and ∆RIB values

For DC_7_n1-n257, DC_7-7_n1-n257, TIB,c and RIB,c values are already mentioned in 37.716-21-21 under the same band combinations with n257A.

### 6.2.5	MSD

No additional MSD requirement is needed.

## 6.3	DC_11_n3-n28

### 6.3.1	Operating bands for DC

Table 6.3.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n3-n28 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 6.3.2	Channel bandwidths per operating band for DC

Table 6.3.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_11A_n3A-n28A | DC_11A_n3A DC_11A_n28A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 50 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.3.3	Co-existence studies

Based on the co-existence studies of DC_11A-n3A and DC_11A-n28A captured in TR 37.716-11-11, 5th order IMD generated by dual uplink of Band 11 + Band n3 may also fall into own Rx of band n28.

### 6.3.4	∆TIB and ∆RIB values

For DC_11_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n3-n28 | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |

Table 6.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n3-n28 | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |

### 6.3.5	MSD

As mentioned in 6.3.3, IMD5 of B11 and n3 to Band n28 Rx need to be addressed for REFSENS relaxation. Based on the relaxation value of CA_3-21-28, The following value is proposed:

Table 6.3.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_11A-n3A_n28A | 11 | 1435 | 5 | 25 | 1483 | N/A | FDD | N/A |
|  | n3 | 1753 | 5 | 25 | 1848 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 3.0 | FDD | IMD5 |

## 6.4	DC_1_n28-n41

### 6.4.1	Operating bands for DC

Table 6.4.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n28A-n41A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.4.2	Channel bandwidths per operating band for DC

Table 6.4.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A_n28A-n41A | DC_1A_n28A DC_1A_n41A | 1 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 6.4.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_1A_n28A-n41A with dual uplink on DC_1A_n28A and DC_1A_n41A were calculated and listed in Table 6.4.3-1 and 6.4.3-2 respectively.

Table 6.4.3-1: Harmonic and IMD analysis for DC_1_n28-n41 with UL on DC_1_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1172 | 1277 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 | 3257 | 424 | 574 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 | 4708 | 3326 | 3476 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5012 | 5237 | 129 | 324 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6463 | 6688 | 4029 | 4224 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2344 | 2554 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 832 | 1072 | 6932 | 7217 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 | 4972 | 8383 | 8668 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 | 1851 | 4264 | 4534 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 | 6204 | 7166 | 7436 |

Table 6.4.3-2: Harmonic and IMD analysis for DC_1_n28-n41 with UL on DC_1_n41

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7488 | 8070 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 | 770 | 4416 | 4670 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1150 | 1464 | 3012 | 3460 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6336 | 6650 | 6912 | 7360 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3070 | 3444 | 5508 | 6150 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8256 | 8630 | 9408 | 10050 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1032 | 1540 | 8832 | 9340 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8004 | 8840 | 4990 | 5424 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11904 | 12740 | 10176 | 10610 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3528 | 4230 | 380 | 948 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11328 | 12030 | 10752 | 11320 |

Based on Table 6.4.3-1 and Table 6.4.3-2:

- 2nd and 4th order IMD generated by dual uplink of Band 1 + Band n28 may also fall into own Rx of band n41, it could reuse the MSD value of DC_1A-41A_n28A

- 2nd and 5th order IMD generated by dual uplink of Band 1 + Band n41 may also fall into own Rx of band n28.

### 6.4.4	∆TIB and ∆RIB values

For DC_1A_n28A-n41A, the TIB,c and RIB,c values can reuse the values of DC_1A-41A_n28A as given in the tables below.

Table 6.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n41 | 1 | 0.5 |
|  | n28 | 0.5 |
|  | n41 | 0.5 |

Table 6.4.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n41 | 1 | 0 |
|  | n28 | 0.2 |
|  | n41 | 0 |

### 6.4.5	MSD

Table 6.4.5-1 shows the required MSD:

Table 6.4.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_1A_n28A-n41A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n28A-n41A | 1 | IMD2 | ¦fB1+fBn28¦ | 1935 | 5 | 25 | 2125 | N/A |
|  | n28 |  |  | 718 | 5 | 25 | 773 | N/A |
|  | n41 |  |  | 2653 | 10 | 50 | 2653 | 30.1 |
|  | 1 | IMD2 | ¦fBn41 -fB1¦ | 1923 | 5 | 25 | 2113 | N/A |
|  | n41 |  |  | 2685 | 10 | 50 | 2685 |  |
|  | n28 |  |  | 707 | 5 | 25 | 762 | 29.3 |
|  | 1 | IMD5 | ¦2*fBn41 -3*fB1¦ | 1935 | 5 | 25 | 2125 | N/A |
|  | n41 |  |  | 2510 | 10 | 50 | 2510 |  |
|  | n28 |  |  | 730 | 10 | 50 | 785 | 4.5 |
| NOTE 1: This band is subject to IMD4 also which MSD is not specified |  |  |  |  |  |  |  |  |

## 6.5	DC_3_n28-n41

### 6.5.1	Operating bands for DC

Table 6.5.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A_n28A-n41A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.5.2	Channel bandwidths per operating band for DC

Table 6.5.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A_n28A-n41A | DC_3A_n28A DC_3A_n41A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 6.5.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_3A_n28A-n41A with dual uplink on DC_3A_n28A and DC_3A_n41A were calculated and listed in Table 6.5.3-1 and 6.5.3-2 respectively.

Table 6.5.3-1: Harmonic and IMD analysis for DC_3_n28-n41 with UL on DC_3_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2244 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1082 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2672 | 2867 | 214 | 379 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4318 | 3116 | 3281 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4382 | 4652 | 324 | 534 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6103 | 3819 | 4029 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1924 | 2164 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1027 | 1282 | 6092 | 6437 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4777 | 7543 | 7888 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1176 | 1461 | 3634 | 3949 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5814 | 6536 | 6851 |

Table 6.5.3-2: Harmonic and IMD analysis for DC_3_n28-n41 with UL on DC_3_n41

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7488 | 8070 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 711 | 980 | 4206 | 4475 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 730 | 1074 | 3207 | 3670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5916 | 6260 | 6702 | 7165 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 2859 | 5703 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7626 | 8045 | 9198 | 9855 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1422 | 1960 | 8412 | 8950 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8199 | 9050 | 4150 | 4644 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11694 | 12545 | 9336 | 9830 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3918 | 4650 | 250 | 363 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10908 | 11640 | 10122 | 10735 |

Based on Table 6.5.3-1 and Table 6.5.3-2:

- 2nd and 3rd order IMD generated by dual uplink of Band 3 + Band n28 may also fall into own Rx of band n41, it could reuse the MSD value of DC_3A-41A_n28A

- 2nd and 3rd order IMD generated by dual uplink of Band 3 + Band n41 may also fall into own Rx of band n28.

### 6.5.4	∆TIB and ∆RIB values

For DC_3A_n28A-n41A, the TIB,c and RIB,c values can reuse the values of DC_3A-41A_n28A as given in the tables below.

Table 6.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n41 | 3 | 0.5 |
|  | n28 | 0.3 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.5.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n41 | 3 | 0 |
|  | n28 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 6.5.5	MSD

Table 6.5.5-1 shows the required MSD:

Table 6.5.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n28A-n41A

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3A_n28A-n41A | 3 | 1720 | 5 | 25 | 1815 | N/A | TDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 261 | FDD | IMD2 ¦fn41-fB3¦ |
|  | n41 | 2510 | 5 | 25 | 2510 | N/A | FDD | N/A |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | FDD | N/A |
|  | n41 | 2518 | 5 | 25 | 2518 | 27.4 | TDD | IMD2 ¦fB3+fn28¦ |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | FDD | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | FDD | N/A |
|  | n41 | 2687 | 5 | 25 | 2687 | 15.9 | TDD | IMD3 ¦2*fB3-fn28¦ |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified |  |  |  |  |  |  |  |  |

## 6.6	DC_2_n48-n66

### 6.6.1	Operating bands for DC

Table 6.6.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n48-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.6.2	Configuration for DC

Table 6.6.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A_n48A-n66A | DC_2A_n48A DC_2A_n66A | 2A | CA_n48A-n66A |

### 6.6.3	Co-existence studies

Co-existence analysis for DC_2_n48 UL shows no impact to NR Band n66 DL.

Co-existence analysis for DC_2_n66 UL shows that 2nd HAM of band 66 may fall into the lowest 10MHz of NR Band n48 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further 2nd IMD might also fall in n48 DL.

### 6.6.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below. Values from DC_2-48_n66 already defined in 38.101-3 is reused.

Table 6.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n48-n66 | 2 | 0.6 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |

Table 6.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n48-n66 | 2 | 0.3 |
|  | n48 | 0.4 |
|  | n66 | 0.3 |

### 6.6.5	MSD

Based on co-existence studies additional MSD is needed to be defined due to 2nd IMD. Similar values as for DC_2A-48A_n66A already defined in 38101-3 is proposed.

Table 6.6.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n48A-n66A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |

## 6.7	DC_48_n25-n48

### 6.7.1	Operating bands for DC

Table 6.7.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_48_n25-n48 | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

### 6.7.2	Configuration for DC

Table 6.7.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_48A_n25A-n48A | DC_48A_n25A | 48A | CA_n25A-n48A |

### 6.7.3	Co-existence studies

Co-existence analysis for DC_48_n25 UL shows that 4th IMD may fall into NR Band n48 DL. However, since 48 and n48 will be operated as synchronous in same UL/DL configuration there is no IMD problem.

### 6.7.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48_n25-n48 | 48 | 0.8 |
|  | n25 | 0.6 |
|  | n48 | 0.8 |

Table 6.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48_n25-n48 | 48 | 0.4 |
|  | n25 | 0.3 |
|  | n48 | 0.4 |

### 6.7.5	MSD

Based on co-existence studies no additional MSD is needed.

## 6.8	DC_48_n48-n66

### 6.8.1	Operating bands for DC

Table 6.8.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_48_n48-n66 | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.8.2	Configuration for DC

Table 6.8.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_48A_n48A-n66A | DC_48A_n66A | 48A | CA_n48A-n66A |

### 6.8.3	Co-existence studies

Co-existence analysis for DC_48_n66 UL shows that 2nd HAM of band 66 may fall into the lowest 10MHz of NR Band n48 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further 4th order IMD might fall in 48 DL However, since 48 and n48 will be operated as synchronous in same UL/DL configuration no IMD issue.

### 6.8.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48_n48-n66 | 48 | 0.8 |
|  | n48 |  |
|  | n66 | 0.6 |

Table 6.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48_n48-n66 | 48 | 0.4 |
|  | n48 |  |
|  | n66 | 0.3 |

### 6.8.5	MSD

Based on co-existence studies no additional MSD is needed.

## 6.9	DC_66_n25-n48

### 6.9.1	Operating bands for DC

Table 6.9.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n25-n48 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

### 6.9.2	Configuration for DC

Table 6.9.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_66A_n25A-n48A | DC_66A_n25A DC_66A_n48A | 66A | CA_n25A-n48A |

### 6.9.3	Co-existence studies

Co-existence analysis for DC_66_n48 UL shows that 2nd HAM of band 66 may fall into the lowest 10MHz of NR Band n48 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further 2nd IMD might also fall in n25 and 5th order IMD might fall in 66 DL. But the 5th IMD already covered in Table 7.3B.2.3.5.1-1 for DC_66A_n48A

Co-existence analysis for DC_66_n25 UL shows that 2nd HAM of band 66 may fall into the lowest 10MHz of NR Band n48 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further 2nd IMD might also fall in n48 DL.

### 6.9.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n25-n48 | 66 | 0.6 |
|  | n25 | 0.6 |
|  | n48 | 0.8 |

Table 6.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n25-n48 | 66 | 0.3 |
|  | n25 | 0.3 |
|  | n48 | 0.4 |

### 6.9.5	MSD

Based on co-existence studies additional MSD is needed to be defined. Similar values as for DC_2A-48A_n66A already defined in 38101-3 is proposed due to similarity in frequency range.

Table 6.9.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n25-n48 | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n48 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 66 | 1735 | 5 | 25 | 2135 | N/A | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | 28.3 | IMD2 |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |

## 6.10	DC_7_n40-n78

### 6.10.1	Operating bands for DC

Table 6.10.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n40-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.10.2	Configuration for DC

Table 6.10.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A_n40A-n78A | DC_7A_n40A DC_7A_n78A | 7A | CA_n40A-n78A |

### 6.10.3	Co-existence studies

Co-existence analysis for DC_7_n40 UL shows no impact to NR Band n78 DL.

Co-existence analysis for DC_7_n78 UL shows 4th IMD might fall into NR Band n40 DL.

### 6.10.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n40-n78 | 7 | 0.5 |
|  | n40 | 0.6 |
|  | n78 | 0.8 |

Table 6.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n40-n78 | 7 | 0 |
|  | n40 | 0.5 |
|  | n78 | 0.5 |

### 6.10.5	MSD

Based on co-existence studies additional MSD is needed to be defined due to 4nd IMD.

Table 6.10.5-1: Proposed MSD levels from interested companies for DC_7_n40-n78

| DC bands | UL DC | IMD |  | Nokia | LG | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n40A-n78A | 7 | IMD4 | ¦2*fB7 -2*fn78¦ | 9.3 | 9.1 | 9.2 |
|  | n78 |  |  |  |  |  |
|  | n40 |  |  |  |  |  |

Table 6.10.5-2: MSD test points for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n40A-n78A | 7 | 2520 | 10 | 50 | 2640 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 9.2 | IMD4 |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |

## 6.11	DC_8_n40-n78

### 6.11.1	Operating bands for DC

Table 6.11.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n40-n78 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.11.2	Configuration for DC

Table 6.11.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_8A_n40A-n78A | DC_8A_n40A DC_8A_n78A | 8A | CA_n40A-n78A |

### 6.11.3	Co-existence studies

Co-existence analysis for DC_8_n40 UL shows that 4th HAM of band 8 may fall into NR Band n78 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3. Further, it shows that 2nd and 3rd IMD may fall in NR Band n78 DL.

Co-existence analysis for DC_8_n78 UL shows that 2nd IMD may fall into NR Band n40 DL.

### 6.11.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n40-n78 | 8 | 0.6 |
|  | n40 | 0.3 |
|  | n78 | 0.8 |

Table 6.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n40-n78 | 8 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

### 6.11.5	MSD

Based on co-existence studies additional MSD is needed.

Table 6.10.5-1: Proposed MSD levels from interested companies for DC_7_n40-n78

| DC bands | UL DC | IMD |  | Nokia | LG | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n40A-n78A | 8 | IMD2 | ¦fB8 + fn40¦ | 28.2 | 29.2 | 28.7 |
|  | n40 |  |  |  |  |  |
|  | n78 |  |  |  |  |  |
|  | 8 | IMD2 | ¦fB8 - fn78¦ | 26.0 | 28.6 | 27.3 |
|  | n78 |  |  |  |  |  |
|  | n40 |  |  |  |  |  |

Table 6.11.5-2: MSD test points for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n40A-n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
|  | n78 | 3305 | 10 | 50 | 3307.5 | 28.7 | IMD21 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | 27.3 | IMD2 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |

## 6.12	DC_28_n1-n40

### 6.12.1	Operating bands for DC

Table 6.12.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n1-n40 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 6.12.2	Configuration for DC

Table 6.12.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_28A_n1A-n40A | DC_28A_n1A DC_28A_n40A | 28A | CA_n1A-n40A |

### 6.12.3	Co-existence studies

Co-existence analysis for DC_28_n1 UL shows that 4th IMD may fall in NR Band n40 DL.

Co-existence analysis for DC_28_n40 UL shows that 3rd HAM of band 28 may fall into NR Band n1 DL. This is already covered in Table 7.3B.2.3.1-1 and Table 7.3B.2.3.1-2 in 38.101-3.

### 6.12.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n1-n40 | 28 | 0.6 |
|  | n1 | 0.3 |
|  | n40 | 0.5 |

Table 6.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n1-n40 | 28 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0 |

### 6.12.5	MSD

Based on co-existence studies additional MSD is needed.

Table 6.12.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n1A-n40A | 28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2374 | 5 | 25 | 2374 | 10.1 | IMD4 |

## 6.13	DC_1_n3-n77

### 6.13.1	Operating bands for DC

Table 6.13.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n3-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.13.2	Channel bandwidths per operating band for DC

Table 6.13.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A_n3A-n77A | DC_1A_n3A DC_1A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 250 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.13.3	Co-existence studies

Based on the co-existence studies of DC_1A-n3A and DC_1A-n77A captured in TR 37.716-11-11 and 37.863-01-01, 2nd and 4th order IMD generated by dual uplink of Band 1 + Band n3 may also fall into own Rx of band n77 and 2nd and 4th order IMD generated by dual uplink of Band 1 + Band n77 may also fall into own Rx of band n3.

### 6.13.4	∆TIB and ∆RIB values

For DC_1_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n77 | 1 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 6.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n77 | 1 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 6.13.5	MSD

As mentioned in 6.13.3, IMD2 and IMD4 of B1 and n3 to Band n77 Rx and those of B1 and n77 to Band n3 need to be addressed for REFSENS relaxation. Based on the exceptions of  DC_1-3_n77 and DC_1_n3-n78, the following values are proposed:

Table 6.13.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A_n3A-n77A DC_1A_n3A-n77(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A | FDD | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | 28.4 | TDD | IMD2 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n77 | 3360 | 10 | 50 | 3360 | 11.2 | TDD | IMD4 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | TDD | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | FDD | IMD2 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | TDD | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 8.5 | FDD | IMD4 |

## 6.14	DC_8_n3-n77

### 6.14.1	Operating bands for DC

Table 6.14.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n3-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.14.2	Channel bandwidths per operating band for DC

Table 6.14.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n3A-n77A | DC_8A_n3A DC_8A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 240 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.14.3	Co-existence studies

Based on the co-existence studies of DC_8A-n3A and DC_8A-n77A captured in TR 37.716-11-11 and 37.863-01-01, 3rd and 5th order IMD generated by dual uplink of Band 8 + Band n3 may also fall into own Rx of band n77 and 3rd order IMD generated by dual uplink of Band 8 + Band n77 may also fall into own Rx of band n3.

### 6.14.4	∆TIB and ∆RIB values

For DC_8_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n3-n77 | 8 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 6.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n3-n77 | 8 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 6.14.5	MSD

As mentioned in 6.14.3, IMD3 of B8 and n3 to n77 Rx and IMD3 of B8 and n77 to n3 Rx need to be addressed for REFSENS relaxation. Based on REFSENS relaxation of DC_3-8_n78 and DC_3_n8-n78, the following values are proposed:

Table 6.14.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A-n3A_n77A DC_8A-n3A_n77(2A) | 8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | TDD | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.3 | FDD | IMD3 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 16.5 | FDD | IMD3 |

## 6.15	DC_1_n3-n41

### 6.15.1	Operating bands for DC

Table 6.15.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1_n3-n41 | 1 | n3, n41 |

### 6.15.2	Channel bandwidths per operating band for DC

Table 6.15.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A_n3A-n41A | DC_1A_n3A DC_1A_n41A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.15.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.15.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_1_n3-n41

Table 6.15.3-1: Summary of Self-interference analysis for DC_1_n3-n41 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_1_n3-n41 | DC_1A_n3A | No | IMD5 | Yes | Yes |
|  | DC_1A_n41A | No | No | Yes | Yes |

The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_1_n3 and DC_1_n41.

### 6.15.4	∆ TIB and ∆RIB values

For DC_1_n3-n41, the TIB,c and RIB,c values are reuse the values of DC_1-3_n41 as given in the tables below.

Table 6.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n41 | 1 | 0.5 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.15.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n41 | 1 | 0 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 6.15.5	MSD

Table 6.15.5-1 shows the required MSD, its value reused the value of CA_1A-3A-41:

Table 6.15.5-1: MSD exception for Scell due to dual uplink operation for DC_1_n3-n41

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n3A-n41A | 1 | IMD5 | ¦3*fB1-2*fn3¦ | 1977.5 | 5 | 25 | 2167.5 | N/A |
|  | n3 |  |  | 1712.5 | 5 | 25 | 1807.5 | N/A |
|  | n41 |  |  | 2507.5 | 5 | 25 | 2507.5 | 5.0 |

## 6.16	DC_1_n41-n77

### 6.16.1	Operating bands for DC

Table 6.16.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1_n41-n77 | 1 | n41, n77 |

### 6.16.2	Channel bandwidths per operating band for DC

Table 6.16.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A_n41A-n77A | DC_1A_n41A DC_1A_n77A | 1 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.16.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.16.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_1_n41-n77

Table 6.16.3-1: Summary of Self-interference analysis for DC_1_n41-n77 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_1_n41-n77 | DC_1A_n41A | H2 of B1 UL impacts to n77 DL | IMD3, IMD4, IMD5 | Yes | Yes |
|  | DC_1A_n77A | H2 of B1 UL impacts to n77 DL | IMD4, IMD5 | Yes | Yes |

The MSD due to harmonic issue has been defined in the constituent fall-back modes DC_1_n77.

The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_1_n41.

### 6.16.4	∆TIB and ∆RIB values

For DC_1_n41-n77, the TIB,c and RIB,c values are reuse the values of DC_1-41_n77 as given in the tables below.

Table 6.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n41-n77 | 1 | 0.5 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |

Table 6.16.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1_n41-n77 | 1 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

### 6.16.5	MSD

Table 6.16.5-1 shows the required MSD, its values can reuse the values for DC_1A_n41A-n78A:

Table 6.16.5-1: MSD exception for Scell due to dual uplink operation for DC_1_n41-n77

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n41A-n77A | 1 | IMD3 | ¦2*fn41- fB1¦ | 1970 | 5 | 25 | 2160 | N/A |
|  | n41 |  |  | 2650 | 10 | 25 | 2650 | N/A |
|  | n77 |  |  | 3330 | 10 | 50 | 3330 | 19.6 |
|  | 1 | IMD4 | ¦3*fB1 –fn41¦ | 1970 | 5 | 25 | 2160 | N/A |
|  | n41 |  |  | 2520 | 5 | 25 | 2640 | N/A |
|  | n77 |  |  | 3390 | 10 | 50 | 3390 | 10.11 |
|  | 1 | IMD4 | ¦2*fn77 –2*fB1¦ | 1970 | 5 | 25 | 2160 | N/A |
|  | n41 |  |  | 2680 | 5 | 25 | 2680 | 11.51 |
|  | n77 |  |  | 3310 | 10 | 50 | 3310 | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |  |

## 6.17	DC_3_n3-n41

### 6.17.1	Operating bands for DC

Table 6.17.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3_n3-n41 | 3 | n3, n41 |

### 6.17.2	Channel bandwidths per operating band for DC

Table 6.17.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A_n3A-n41A | DC_3A_n3A1 DC_3A_n41A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.17.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.17.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_3_n3-n41

Table 6.17.3-1: Summary of Self-interference analysis for DC_3_n3-n41 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_3_n3-n41 | DC_3A_n3A | No | No | No | No |
|  | DC_3A_n41A | No | IMD4 | Yes | Yes |

The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_3_n41.

### 6.17.4	∆TIB and ∆RIB values

For DC_3_n3-n41, the TIB,c and RIB,c values are reuse the values of DC_3_n41 as given in the tables below.

Table 6.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n3-n41 | 3 | 0.5 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.17.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3_n3-n41 | 3 | 0 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 6.17.5	MSD

Table 6.17.5-1 shows the required MSD, its value can reuse the value for DC_3A_n41A:

Table 6.17.5-1: MSD exception for Scell due to dual uplink operation for DC_3_n3-n41

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n3A-n41A | 3 | IMD4 | ¦2*fn41-2*fB3¦ | 1725 | 5 | 25 | 1820 | N/A |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 8.2 |
|  | n41 |  |  | 2657.5 | 5 | 25 | 2657.5 | N/A |

## 6.18	DC_3_n41-n77

### 6.18.1	Operating bands for DC

Table 6.18.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3_n41-n77 | 3 | n41, n77 |

### 6.18.2	Channel bandwidths per operating band for DC

Table 6.18.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A_n41A-n77A | DC_3A_n41A DC_3A_n77A | 3 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.18.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.18.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_3_n41-n77

Table 6.18.3-1: Summary of Self-interference analysis for DC_3_n41-n77 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_3_n41-n77 | DC_3A_n41A | H2 of B3 UL impacts to n77 DL | IMD3, IMD5 | Yes | Yes |
|  | DC_3A_n77A | H2 of B3 UL impacts to n77 DL | IMD5 | No | Yes |

The MSD due to harmonic issue has been defined in the constituent fall-back modes DC_3_n77.The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_3_n41.

### 6.18.4	∆TIB and ∆RIB values

For DC_3_n41-n77, the TIB,c and RIB,c values are reuse the values of DC_3-41_n77 as given in the tables below.

Table 6.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n41-n77 | 3 | 0.6 |
|  | n41 | 0.31/0.82 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.18.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3_n41-n77 | 3 | 0.2 |
|  | n41 | 010.52 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 6.18.5	MSD

Table 6.18.5-1 shows the required MSD, its value can reuse the value for DC_3A_n41A-n78A:

Table 6.18.5-1: MSD exception for Scell due to dual uplink operation for DC_3_n41-n77

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n41A-n77A | 3 | IMD3 | ¦2*fn41- fB3¦ | 1730 | 5 | 25 | 1825 | N/A |
|  | n41 |  |  | 2560 | 10 | 50 | 2560 | N/A |
|  | n77 |  |  | 3390 | 10 | 50 | 3390 | 16.41 |
|  | 3 | IMD5 | ¦2*fn77 –3*fB3¦ | 1720 | 5 | 25 | 1815 | N/A |
|  | n41 |  |  | 2640 | 5 | 25 | 2640 | 5.3 |
|  | n77 |  |  | 3900 | 10 | 50 | 3900 | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified |  |  |  |  |  |  |  |  |

## 6.19	DC_41_n3-n41

### 6.19.1	Operating bands for DC

Table 6.19.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_41_n3-n41 | 41 | n3, n41 |

### 6.19.2	Channel bandwidths per operating band for DC

Table 6.19.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_41A_n3A-n41A | DC_41A_n3A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.19.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.19.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_41_n3-n41

Table 6.19.3-1: Summary of Self-interference analysis for DC_41_n3-n41 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_41_n3-n41 | DC_41_n3 | No | No | Yes | Yes |
|  | DC_41_n41 | No | No | No | No |

The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_41_n3.

### 6.19.4	∆TIB and ∆RIB values

For DC_41_n3-n41, the TIB,c and RIB,c values are reuse the values of DC_41_n3 as given in the tables below.

Table 6.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n3-n41 | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 6.19.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n3-n41 | 41 | 01/0.52 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 6.19.5	MSD

There are no additional MSD requirements for this band combination.

## 6.20	DC_41_n41-n77

### 6.20.1	Operating bands for DC

Table 6.20.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_41_n41-n77 | 41 | n41, n77 |

### 6.20.2	Channel bandwidths per operating band for DC

Table 6.20.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_41A_n41A-n77A | DC_41A_n77A | 41 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.20.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.20.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_41_n41-n77

Table 6.20.3-1: Summary of Self-interference analysis for DC_41_n41-n77 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_41_n41-n77 | DC_41A_n77A | No | IMD4 | Yes | No |

The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_41_n77.

Since DC_41_n41 only apply for non-simultaneous Tx/Rx between all carriers, there is no IMD4 issue for DC_41_n41-n77 with UL DC_41_n77.

### 6.20.4	∆TIB and ∆RIB values

For DC_41_n41-n77, the TIB,c and RIB,c values are reuse the values of DC_41_n77 as given in the tables below.

Table 6.20.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n41-n77 | 41 | 0.3 |
|  | n41 | 0.3 |
|  | n77 | 0.8 |

Table 6.20.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n41-n77 | 41 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

### 6.20.5	MSD

There is no additional MSD requirement for this band combination.

## 6.21	DC_41_n41-n78

### 6.21.1	Operating bands for DC

Table 6.21.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_41_n41-n78 | 41 | n41, n78 |

### 6.21.2	Channel bandwidths per operating band for DC

Table 6.21.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_41A_n41A-n78A | DC_41A_n78A | 41 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.21.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.21.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_41_n41-n78

Table 6.21.3-1: Summary of Self-interference analysis for DC_41_n41-n78 with 2 bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_41_n41-n78 | DC_41_n78 | No | IMD4 | Yes | No |

The MSD due to small frequency separation has been defined in the constituent fall-back modes DC_41_n78.

Since DC_41_n41 only apply for non-simultaneous Tx/Rx between all carriers, there is no IMD4 issue for DC_41_n41-n78 with UL DC_41_n78.

### 6.21.4	∆TIB and ∆RIB values

For DC_41_n41-n78, the TIB,c and RIB,c values are reuse the values of DC_41_n78 as given in the tables below.

Table 6.21.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n41-n78 | 41 | 0.3 |
|  | n41 | 0.3 |
|  | n78 | 0.8 |

Table 6.21.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n41-n78 | 41 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

### 6.21.5	MSD

There is no additional MSD requirement for this band combination.

## 6.22	DC_2_n7-n66

### 6.22.1	Operating bands for DC

Table 6.22.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2_n7-n66 | 2 | n7, n66 |

### 6.22.2	Channel bandwidths per operating band for DC

Table 6.22.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 |  | 20 | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  | Maximum aggregated bandwidth [MHz] |  |
| DC_2A_n7A-n66A | DC_2A_n7A DC_2A_n66A | 2 |  | 15 | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 |  |
|  |  | n7 |  | See CA_n7A-n66A BCS configurations in Table 5.5A.3.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A_n7(2A)-n66A | DC_2A_n7A DC_2A_n66A | 2 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n7 |  | See CA_n7(2A)-n66A BCS configurations in Table 5.5A.3.1-1 in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.22.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.22.3-1 summarizes the EN- DC band combinations with self-interference problems for DC_2_n7-n66

Table 6.22.3-1: Summary of Self-interference analysis for DC_2_n7-n66 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_2_n7-n66 | DC_2A_n7A | No | No | No | No |
|  | DC_2A_n66A | No | No | No | No |

### 6.22.4	∆TIB and ∆RIB values

For DC_2_n77-n66, the TIB,c and RIB,c values can reuse the values of DC_2A-7A_n66A as given in the tables below.

Table 6.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n7-n66 | 2 | 0.5 |
|  | n7 | 0.5 |
|  | n66 | 0.5 |

Table 6.22.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2_n7-n66 | 2 | 0.3 |
|  | n7 | 0.5 |
|  | n66 | 0.5 |

### 6.22.5	MSD

There are no additional MSD requirements for this band combination.

## 6.23	DC_2_n38-n66

### 6.23.1	Operating bands for DC

Table 6.23.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2_n38-n66 | 2 | n38, n66 |

### 6.23.2	Channel bandwidths per operating band for DC

Table 6.23.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_2A_n38A-n66A | DC_2A_n38A DC_2A_n66A | 2 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  |  | n38 |  | See CA_n38A-n66A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.23.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.23.3-1 summarizes the EN- DC band combinations with self-interference problems for DC_2_n38-n66

Table 6.23.3-1: Summary of Self-interference analysis for DC_2_n38-n66 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_2_n38-n66 | DC_2A_n38A | No | No | No | No |
|  | DC_2A_n66A | No | No | No | No |

### 6.23.4	∆TIB and ∆RIB values

For DC_2_n38-n66, the TIB,c and RIB,c values are given in the tables below.

Table 6.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n38-n66 | 2 | 0.5 |
|  | n38 | 0.9 |
|  | n66 | 0.5 |

Table 6.23.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2_n38-n66 | 2 | 0.3 |
|  | n38 | 0.5 |
|  | n66 | 0.5 |

### 6.23.5	MSD

There are no additional MSD requirements for this band combination.

## 6.24	DC_2_n66-n78

### 6.24.1	Operating bands for DC

Table 6.24.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2_n66-n78 | 2 | n66, n78 |

### 6.24.2	Channel bandwidths per operating band for DC

Table 6.24.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_2A_n66A-n78(2A) DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A) | DC_2A_n66A DC_2A_n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66A-n78(2A) BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 See CA_n66(2A)-n78A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 See CA_n66(2A)-n78(2A) BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.24.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.24.3-1 summarizes the EN- DC band combinations with self-interference problems for DC_2_n66-n78

Table 6.24.3-1: Summary of Self-interference analysis for DC_2_n66-n78 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_2_n66-n78 | DC_2A_n66A | H2 of B2 UL impacts to n78 DL | IMD2 and IMD4 impacts to n78 | No | Yes |
|  | DC_2A_n78A | H2 of B2 UL impacts to n78 DL | IMD4 impacts to n66 | No | Yes |

The MSD due to harmonic issue has been defined in the constituent fall-back modes DC_2A_n78A in TS 38.101-3.

### 6.24.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values have been defined in the constituent fall-back modes DC_2A_n66A-n78A in TS 38.101-3.

### 6.24.5	MSD

The MSD due to intermodulation have been defined in the constituent fall-back modes DC_2A_n66A-n78A in TS 38.101-3. Table 6.24.5-1 shows the required MSD:

Table 6.24.5-1: MSD exception for Scell due to dual uplink operation for DC_2_n66-n78

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n66A-n78(2A) DC_2A_n66(2A)-n78A DC_2A_n66(2A)-n78(2A) | 2 | IMD4 | ¦3*fB2-fn78¦ | 1880 | 5 | 25 | 1960 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 10.3 |
|  | n78 |  |  | 3480 | 10 | 50 | 3480 | N/A |
|  | 2 | IMD2 | ¦fB2 +fn66¦ | 1880 | 5 | 25 | 1960 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 | N/A |
|  | n78 |  |  | 3620 | 10 | 50 | 3620 | 29.4 |
|  | 2 | IMD4 | ¦3*fn66 -fB2¦ ¦3*fB2 –fn66¦ | 1880 | 5 | 25 | 1960 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 | N/A |
|  | n78 |  |  | 3340 | 10 | 50 | 3340 | 8.9 |

## 6.25	DC_12_n7-n66

### 6.25.1	Operating bands for DC

Table 6.25.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_12_n7-n66 | 12 | n7, n66 |

### 6.25.2	Channel bandwidths per operating band for DC

Table 6.25.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 |  | 20 | 25 |  | 30 |  | 40 |  | 50 |  | 60 |  | 70 |  | 80 |  | 90 |  | 100 |  | Maximum aggregated bandwidth [MHz] |  |
| DC_12A_n7A-n66A | DC_12A_n7A DC_12A_n66A | 12 |  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 |  |
|  |  | n7 |  | See CA_n7A-n66A BCS configurations in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_12A_n7(2A)-n66A | DC_12A_n7A DC_12A_n66A | 12 |  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 |
|  |  | n7 |  | See CA_n7(2A)-n66A BCS configurations in TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.25.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.25.3-1 summarizes the EN-DC band combinations with self-interference problems for DC_12_n7-n66

Table 6.25.3-1: Summary of Self-interference analysis for DC_12_n7-n66 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_12_n7-n66 | DC_12A_n7A | H3 of B12 UL impacts to n66 DL | No | No | Yes |
|  | DC_12A_n66A | H3 of B12 UL impacts to n66 DL | No | No | Yes |

The MSD due to harmonic issue has been defined in the constituent fall-back modes DC_12_n66.

### 6.25.4	∆TIB and ∆RIB values

For DC_12_n7-n66, the TIB,c and RIB,c values can reuse the values of CA_7A-12A_66A as given in the tables below.

Table 6.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n7-n66 | 12 | 0.8 |
|  | n7 | 0.5 |
|  | n66 | 0.5 |

Table 6.25.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_12_n7-n66 | 12 | 0.5 |
|  | n7 | 0.5 |
|  | n66 | 0.5 |

### 6.25.5	MSD

There are no additional MSD requirements for this band combination.

## 6.26	DC_66_n38-n66

### 6.26.1	Operating bands for DC

Table 6.26.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_66_n38-n66 | 66 | n38, n66 |

### 6.26.2	Channel bandwidths per operating band for DC

Table 6.26.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_66A_n38A-n66A | DC_66A_n38A DC_66A_n66A1 | 66 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  |  | n38 |  | See CA_n38A-n66A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.26.3	Co-existence studies

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back mode.Table 6.26.3-1 summarizes the EN- DC band combinations with self-interference problems for DC_66_n38-n66

Table 6.26.3-1: Summary of Self-interference analysis for DC_66_n38-n66 with 2bands UL

| Downlink band configuration | Uplink DC Configuration | Harmonic relation issues | intermodulation to own rx band | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_66_n38-n66 | DC_66A_n38A | No | No | No | No |
|  | DC_66A_n66A | No | No | No | No |

### 6.26.4	∆TIB and ∆RIB values

For DC_66_n38-n66, the TIB,c and RIB,c values are given in the tables below.

Table 6.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n38-n66 | 66 | 0.5 |
|  | n38 | 0.5 |
|  | n66 | 0.5 |

Table 6.26.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66_n38-n66 | 66 | 0.5 |
|  | n38 | 0.5 |
|  | n66 | 0.5 |

### 6.26.5	MSD

There are no additional MSD requirements for this band combination.

## 6.27	DC_19_n1-n77

### 6.27.1	Operating bands for DC

Table 6.27.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19_n1-n77 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.27.2	Configuration for DC

Table 6.27.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A_n1A-n77A | DC_19A_n1A DC_19A_n77A | 19A | CA_n1A-n77A |

### 6.27.3	Co-existence studies

Based on the co-existence studies of DC_19_n1 and DC_19_n77, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_19_n1 uplink may fall into own Rx of band n77.

-	5th order IMD products generated by DC_19_n1 uplink may fall into own Rx of band n77.

-	3rd order IMD products generated by DC_19_n77 uplink may fall into own Rx of band n1.

### 6.27.4	∆TIB and ∆RIB values

For DC_19_n1-n77, the TIB,c and RIB,c values are reused from DC_1-19_n77, and are given in the tables below.

Table 6.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19_n1-n77 | 19 | 0.3 |
|  | n1 | 0.3 |
|  | n77 | 0.8 |

Table 6.27.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19_n1-n77 | 19 | 0 |
|  | n1 | 0 |
|  | n77 | 0.5 |

### 6.27.5	MSD

The MSD values for own Rx impact of the 3rd band stated in 6.27.3 are shown in the following table [1].  (We will provide MSD result with [ ] because other companies wants to further study.)

Table 6.27.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_19A_n1A-n77A | 19 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n77 | 3655 | 10 | 50 | 3655 | [21.4] | IMD34 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.28	DC_19_n1-n78

### 6.28.1	Operating bands for DC

Table 6.28.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19_n1-n78 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.28.2	Configuration for DC

Table 6.28.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A_n1A-n78A | DC_19A_n1A DC_19A_n78A | 19A | CA_n1A-n78A |

### 6.28.3	Co-existence studies

Based on the co-existence studies of DC_19_n1 and DC_19_n78, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_19_n1 uplink may fall into own Rx of band n78.

-	3rd order IMD products generated by DC_19_n78 uplink may fall into own Rx of band n1.

### 6.28.4	∆TIB and ∆RIB values

For DC_19_n1-n78, the TIB,c and RIB,c values are reused from DC_1-19_n78, and are given in the tables below.

Table 6.28.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19_n1-n78 | 19 | 0.3 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 6.28.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19_n1-n78 | 19 | 0 |
|  | n1 | 0 |
|  | n78 | 0.5 |

### 6.28.5	MSD

The MSD values for own Rx impact of the 3rd band stated in 6.28.3 are shown in the following table [1]. (We will provide MSD result with [ ] because other companies wants to consider further.)

Table 6.28.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_19A_n1A-n78A | 19 | 840 | 5 | 25 | 885 | N/A | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n78 | 3655 | 10 | 50 | 3655 | [21.4] | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |

## 6.29	DC_11_n3-n77

### 6.29.1	Operating bands for DC

Table 6.29.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n3-n77 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.29.2	Channel bandwidths per operating band for DC

Table 6.29.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_11A_n3A-n77A | DC_11A_n3A DC_11A_n77A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_11A_n3A-n77(2A) | DC_11A_n3A DC_11A_n77A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 240 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.29.3	Co-existence studies

Based on the co-existence studies of DC_11A-n3A and DC_11A-n77A captured in TR 37.716-11-11 and 37.863-01-01,

- 4th and 5th order IMD generated by dual uplink of Band 11 + Band n3 may also fall into own Rx of Band n77

- 2nd and 5th order IMD generated by dual uplink of Band 11 + Band n77 may also fall into own Rx of Band n3.

### 6.29.4	∆TIB and ∆RIB values

For DC_11_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n3-n77 | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n77 | 0.8 |

Table 6.29.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n3-n77 | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n77 | 0.5 |

### 6.29.5	MSD

As mentioned in 6.29.3, IMD4 of B11 and n3 to n77 Rx and IMD2 of B11 and n77 to n3 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 6.29.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_11A-n3A_n77A DC_11A-n3A_n77(2A) | 11 | 1440 | 5 | 25 | 1488 | N/A | FDD | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | FDD | N/A |
|  | n77 | 3780 | 10 | 50 | 3780 | 10.8 | TDD | IMD4 |
|  | 11 | 1440 | 5 | 25 | 1488 | N/A | FDD | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | N/A | FDD | N/A |
|  | n77 | 4020 | 10 | 50 | 4020 | 4.1 | TDD | IMD5 |
|  | 11 | 1440 | 5 | 25 | 1488 | N/A | FDD | N/A |
|  | n77 | 3310 | 10 | 50 | 3310 | N/A | TDD | N/A |
|  | n3 | 1775 | 5 | 25 | 1870 | 29.0 | FDD | IMD2 |
|  | 11 | 1440 | 5 | 25 | 1488 | N/A | FDD | N/A |
|  | n77 | 3925 | 10 | 50 | 3925 | N/A | TDD | N/A |
|  | n3 | 1740 | 5 | 25 | 1835 | 3.7 | FDD | IMD5 |

## 6.30	DC_11_n28-n77

### 6.30.1	Operating bands for DC

Table 6.30.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n28-n77 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.30.2	Channel bandwidths per operating band for DC

Table 6.30.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_11A_n28A-n77A | DC_11A_n28A DC_11A_n77A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_11A_n28A-n77(2A) | DC_11A_n28A DC_11A_n77A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Co-existence studies

Based on the co-existence studies of DC_11A-n28A and DC_11A-n77A captured in TR 37.716-11-11 and 37.863-01-01,

- 3rd and 4th order IMD generated by dual uplink of Band 11 + Band n28 may also fall into own Rx of Band n77

3rd and 4th order IMD generated by dual uplink of Band 11 + Band n77 may also fall into own Rx of Band n28.

### 6.30.4	∆TIB and ∆RIB values

For DC_11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n28-n77 | 11 | 0.4 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 6.30.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n28-n77 | 11 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 6.30.5	MSD

As mentioned in 6.30.3, IMD3 of B11 and n28 to n77 Rx and IMD3 of B11 and n77 to n28 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_11A_n28A-n77A DC_11A_n28A-n77(2A) | 11 | IMD3 | ¦2*fB11 +fn28¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77 |  |  | 3629 | 10 | 50 | 3629 | 10 | 17.5 |
|  | 11 | IMD4 | ¦3*fB11 -fn28¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n77 |  |  | 3586 | 10 | 50 | 3586 | 10 | 11.2 |
|  | 11 | IMD3 | ¦2*fB11 - fn77¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n77 |  |  | 3684 | 10 | 50 | 3684 | 10 |  |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 | 15.8 |
|  | 11 | IMD4 | ¦3*fB11 -fn77¦ | 1443 | 5 | 25 | 1491 | 5 | N/A |
|  | n77 |  |  | 3564 | 10 | 50 | 3564 | 10 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 11.0 |

## 6.31	DC_42_n3-n28

### 6.31.1	Operating bands for DC

Table 6.31.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n3-n28 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 6.31.2	Channel bandwidths per operating band for DC

Table 6.31.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_42A_n3A-n28A | DC_42A_n3A DC_42A_n28A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 70 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n3A-n28A | DC_42A_n3A DC_42A_n28A DC_42C_n28A | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.31.3	Co-existence studies

Based on the co-existence studies of DC_42A-n3A and DC_42A-n28A, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 6.31.4	∆TIB and ∆RIB values

For DC_42_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n3-n28 | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n28 | 0.8 |

Table 6.31.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n3-n28 | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n28 | 0.5 |

### 6.31.5	MSD

As mentioned in 6.31.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.32	DC_42_n3-n77

### 6.32.1	Operating bands for DC

Table 6.32.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n3-n77 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.32.2	Channel bandwidths per operating band for DC

Table 6.32.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_42A_n3A-n77A | DC_42A_n3A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_42A_n3A-n77(2A) | DC_42A_n3A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 250 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n3A-n77A | DC_42A_n3A DC_42C_n3A | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_42C_n3A-n77(2A) | DC_42A_n3A DC_42C_n3A | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 270 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.32.3	Co-existence studies

Based on the co-existence studies of DC_42A-n3A and DC_42A-n77A, 4th and 5th order IMD generated by dual uplink of Band 42 + Band n3 may also fall into own Rx of band n77. But due to the TDD synchronous operation of B42 and n77, it is not impacted.

### 6.32.4	∆TIB and ∆RIB values

For DC_42_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n3-n77 | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 6.32.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n3-n77 | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 6.32.5	MSD

As mentioned in 6.32.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.33	DC_41_n28-n41

### 6.33.1	Operating bands for DC

Table 6.33.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41A_n28A-n41A | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.33.2	Channel bandwidths per operating band for DC

Table 6.33.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_41A_n28A-n41A | DC_41A_n28A | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 6.33.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_41A_n28A-n41A with uplink on DC_41A_n28A was calculated and listed in Table 6.33.3-1.

Table 6.33.3-1: Harmonic and IMD analysis for DC_41_n28-n41 with UL on DC_41_n28

| UE UL carriers | Fx_low | Fx_high | Fy_low | Fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2244 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1082 | 962 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2672 | 2867 | 379 | 214 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4318 | 3116 | 3281 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4382 | 4652 | 324 | 534 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1924 | 2164 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6103 | 3819 | 4029 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4826 | 5066 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1282 | 1027 | 6437 | 6092 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1176 | 1461 | 3949 | 3634 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4777 | 7543 | 7888 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5814 | 6536 | 6851 |

No IMD issue for this combination.

### 6.33.4	∆TIB and ∆RIB values

For DC_41A_n28A-n41A, the TIB,c and RIB,c values can reuse the values of DC_3A-41A_n28A as given in the tables below.

Table 6.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n28-n41 | 41 | 0.31/0.82 |
|  | n28 | 0.3 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

Table 6.33.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_41_n28-n41 | 41 | 01/0.52 |
|  | n28 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

### 6.33.5	MSD

There is no additional MSD requirement for this configuration.

## 6.34	DC_18_n28-n41

### 6.34.1	Operating bands for DC

Table 6.34.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n28A-n41A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.34.2	Channel bandwidths per operating band for DC

Table 6.34.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n28A-n41A | DC_18A_n28A DC_18A_n41A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 145 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 6.34.3	Co-existence studies

Based on the co-existence studies of DC_18A-n28A and DC_18A-n41, impact on own Rx of 3rd band is as follows.

- 5th order IMD generated by dual uplink of Band 18 + Band n28 may also fall into own Rx of band n41.

- 5th order IMD generated by dual uplink of Band 18 + Band n41 may also fall into own Rx of band n28. Since band 18 is only used in Japan, there will be no IMD issue for this dual uplink configuration considering Japanese spectrum allocation.

### 6.34.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n28A-n41A are shown in the following tables.

Table 6.34.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n28-n41 | 18 | 0.5 |
|  | n28 | 0.5 |
|  | n41 | 0.3 |

Table 6.34.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n28-n41 | 18 | 0 |
|  | n28 | 0 |
|  | n41 | 0 |

### 6.34.5	MSD

Table 6.34.5-1 shows the required MSD:

Table 6.34.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n28A-n41A

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_18A_n28A-n41A | 18 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n41 | 2510 | 5 | 25 | 2510 | 5.1 | TDD | IMD5 ¦4*fB18 -fn28¦ |
|  | 18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |
|  | n41 | 2515 | 5 | 25 | 2515 | N/A | TDD | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 4.7 | FDD | IMD5 ¦4*fB18 -fn41¦ |

## 6.35	DC_18_n28-n77

### 6.35.1	Operating bands for DC

Table 6.35.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n28A-n77A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.35.2	Channel bandwidths per operating band for DC

Table 6.35.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n28A-n77A | DC_18A_n28A DC_18A_n77A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 145 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.35.3	Co-existence studies

Based on the co-existence studies of DC_18A-n28A and DC_18A-n77, impact on own Rx of 3rd band is as follows.

- 5th order IMD generated by dual uplink of Band 18 + Band n28 may also fall into own Rx of band n77.

- 5th order IMD generated by dual uplink of Band 18 + Band n77 may also fall into own Rx of band n28.

### 6.35.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n28A-n77A are shown in the following tables.

Table 6.35.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n28-n77 | 18 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 6.35.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n28-n77 | 18 | 0 |
|  | n28 | 0 |
|  | n77 | 0.5 |

### 6.35.5	MSD

Table 6.35.5-1 shows the required MSD:

Table 6.35.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n28A-n77A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n28A-n77A | 18 | IMD5 | ¦2*fB18 +3*fn28¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 |  |
|  | n77 |  |  | 3770 | 10 | 50 | 3770 | 10 | 4.0 |
|  | 18 | IMD5 | ¦4*fB18 -fn77¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n77 |  |  | 4045 | 10 | 50 | 4045 | 10 |  |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 | 5.7 |

## 6.36	DC_18_n28-n78

### 6.36.1	Operating bands for DC

Table 6.36.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n28A-n78A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.36.2	Channel bandwidths per operating band for DC

Table 6.36.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n28A-n78A | DC_18A_n28A DC_18A_n78A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 145 |
|  |  | n28 |  | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.36.3	Co-existence studies

Based on the co-existence studies of DC_18A-n28A and DC_18A-n78, impact on own Rx of 3rd band is as follows.

- 5th order IMD generated by dual uplink of Band 18 + Band n28 may also fall into own Rx of band n78.

### 6.36.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n28A-n78A are shown in the following tables.

Table 6.36.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n28-n78 | 18 | 0.5 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 6.36.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n28-n78 | 18 | 0 |
|  | n28 | 0 |
|  | n78 | 0.5 |

### 6.36.5	MSD

Table 6.36.5-1 shows the required MSD:

Table 6.36.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n28A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n28A-n78A | 18 | IMD5 | ¦2*fB18 +3*fn28¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n28 |  |  | 710 | 5 | 25 | 765 | 5 |  |
|  | n78 |  |  | 3770 | 10 | 50 | 3770 | 10 | 4.0 |

## 6.37	DC_18_n3-n41

### 6.37.1	Operating bands for DC

Table 6.37.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n3A-n41A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.37.2	Channel bandwidths per operating band for DC

Table 6.37.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n3A-n41A | DC_18A_n3A DC_18A_n41A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  | n3 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 6.37.3	Co-existence studies

Based on the co-existence studies of DC_18A-n3A and DC_18A-n41, impact on own Rx of 3rd band is as follows.

- 2nd and 3rd order IMD generated by dual uplink of Band 18 + Band n3 may also fall into own Rx of band n41.

- 2nd order IMD generated by dual uplink of Band 18 + Band n41 may also fall into own Rx of band n3.

### 6.37.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n3A-n41A are shown in the following tables.

Table 6.37.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n3-n41 | 18 | 0.3 |
|  | n3 | 0.5 |
|  | n41 | 0.3 |

Table 6.37.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n3-n41 | 18 | 0 |
|  | n3 | 0 |
|  | n41 | 0 |

### 6.37.5	MSD

Table 6.37.5-1 shows the required MSD:

Table 6.37.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n3A-n41A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n3A-n41A | 18 | IMD2 | ¦fB18 +fn3¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n3 |  |  | 1720 | 5 | 25 | 1815 | 5 |  |
|  | n41 |  |  | 2540 | 10 | 50 | 2540 | 10 | 29.4 |
|  | 18 | IMD2 | ¦fB18 -fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2670 | 10 | 50 | 2670 | 10 |  |
|  | n3 |  |  | 1755 | 5 | 25 | 1850 | 5 | 28.2 |
|  | 18 | IMD3 | ¦2fn3-fB18 ¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n3 |  |  | 1725 | 5 | 25 | 1820 | 5 |  |
|  | n41 |  |  | 2630 | 10 | 50 | 2630 | 10 | 16.1 |

## 6.38	DC_18_n41-n77

### 6.38.1	Operating bands for DC

Table 6.38.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n41A-n77A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.38.2	Channel bandwidths per operating band for DC

Table 6.38.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n41A-n77A | DC_18A_n41A DC_18A_n77A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 215 |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n77 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.38.3	Co-existence studies

Based on the co-existence studies of DC_18A-n41A and DC_18A-n77, impact on own Rx of 3rd band is as follows.

- 2nd, 3rd and 4th order IMD generated by dual uplink of Band 18 + Band n41 may also fall into own Rx of band n77.

- 2nd and 3rd order IMD generated by dual uplink of Band 18 + Band n77 may also fall into own Rx of band n41.

### 6.38.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n41A-n77A are shown in the following tables.

Table 6.38.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n41-n77 | 18 | 0.3 |
|  | n41 | 0.3 |
|  | n77 | 0.8 |

Table 6.38.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n41-n77 | 18 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

### 6.38.5	MSD

Table 6.38.5-1 shows the required MSD:

Table 6.38.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n41A-n77A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n41A-n77A | 18 | IMD2 | ¦fB18 +fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2570 | 5 | 25 | 2570 | 5 |  |
|  | n77 |  |  | 3390 | 10 | 50 | 3390 | 10 | 30.1 |
|  | 18 | IMD3 | ¦2*fB18 +fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n77 |  |  | 4155 | 10 | 50 | 4155 | 10 | 16.4 |
|  | 18 | IMD4 | ¦2*fB18 -2*fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n77 |  |  | 3390 | 10 | 50 | 3390 | 10 | 9.1 |
|  | 18 | IMD2 | ¦fB18 -fn77¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n77 |  |  | 3450 | 10 | 50 | 3450 | 10 |  |
|  | n41 |  |  | 2630 | 5 | 25 | 2630 | 5 | 28.5 |

## 6.39	DC_18_n41-n78

### 6.39.1	Operating bands for DC

Table 6.39.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_18A_n41A-n78A | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.39.2	Channel bandwidths per operating band for DC

Table 6.39.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_18A_n41A-n78A | DC_18A_n41A DC_18A_n78A | 18 |  | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 215 |
|  |  | n41 |  | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.39.3	Co-existence studies

Based on the co-existence studies of DC_18A-n41A and DC_18A-n78, impact on own Rx of 3rd band is as follows.

- 2nd and 4th order IMD generated by dual uplink of Band 18 + Band n41 may also fall into own Rx of band n78.

- 2nd order IMD generated by dual uplink of Band 18 + Band n78 may also fall into own Rx of band n41.

### 6.39.4	∆TIB and ∆RIB values

TIB,c and RIB,c values for DC_18A_n41A-n78A are shown in the following tables.

Table 6.39.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_18_n41-n78 | 18 | 0.3 |
|  | n41 | 0.3 |
|  | n78 | 0.8 |

Table 6.39.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_18_n41-n78 | 18 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

### 6.39.5	MSD

Table 6.39.5-1 shows the required MSD:

Table 6.39.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n41A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n41A-n78A | 18 | IMD2 | ¦fB18 +fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2570 | 5 | 25 | 2570 | 5 |  |
|  | n78 |  |  | 3390 | 10 | 50 | 3390 | 10 | 30.1 |
|  | 18 | IMD4 | ¦2*fB18 -2*fn41¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n41 |  |  | 2515 | 5 | 25 | 2515 | 5 |  |
|  | n78 |  |  | 3390 | 10 | 50 | 3390 | 10 | 9.1 |
|  | 18 | IMD2 | ¦fB18 -fn78¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n78 |  |  | 3450 | 10 | 50 | 3450 | 10 |  |
|  | n41 |  |  | 2630 | 5 | 25 | 2630 | 5 | 28.5 |

## 6.40	DC_19_n1-n79

### 6.40.1	Operating bands for DC

Table 6.40.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19_n1-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.40.2	Configuration for DC

Table 6.40.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A_n1A-n79 AX1,X2 | DC_19A_n1A DC_19A_n79A | 19A | CA_n1A-n79A |
| NOTE X1: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE X2: The frequency range in band n79 is restricted for this band combination to 4500 - 4600 MHz. |  |  |  |

### 6.40.3	Co-existence studies

Based on the co-existence studies of DC_19_n1 and DC_19_n79, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_19_n1 uplink may fall into own Rx of band n79.

-	4th order IMD products generated by DC_19_n1 uplink may fall into own Rx of band n79.

-	4th order IMD products generated by DC_19_n79 uplink may fall into own Rx of band n1.

### 6.40.4	∆TIB and ∆RIB values

For DC_19_n1-n79, the TIB,c and RIB,c values are reused from DC_1-19_n79, and are given in the tables below.

Table 6.40.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19_n1-n79 | 19 | 0.3 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 6.40.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19_n1-n79 | 19 | 0.3 |
|  | n1 | 0.3 |
|  | n79 | 0 |

### 6.40.5	MSD

When IMD was calculated based on the frequency range that the operator actually owned, IMD3 and IMD4 don’t fall into own Rx of band n79 and IMD4 doesn’t fall into own Rx of band n1. Therefore, there is no need to have MSD added. NOTE X1 and NOTE X2 in Table 6.40.2-1 indicate these frequency ranges.

## 6.41	DC_21_n1-n77

### 6.41.1	Operating bands for DC

Table 6.41.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21_n1-n77 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.41.2	Configuration for DC

Table 6.41.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A_n1A-n77AX1,X2 | DC_21A_n1A DC_21A_n77A | 21A | CA_n1A-n77A |
| NOTE X1: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE X2: The frequency range in band n77 is restricted for this band combination to 3440 – 3520 MHz and 3600 - 3700 MHz. |  |  |  |

### 6.41.3	Co-existence studies

Based on the co-existence studies of DC_21_n1 and DC_21_n77, own Rx impact of the 3rd band is the followings.

-	2nd order IMD products generated by DC_21_n1 uplink may fall into own Rx of band n77.

-	5th order IMD products generated by DC_21_n1 uplink may fall into own Rx of band n77.

-	2nd order IMD products generated by DC_21_n77 uplink may fall into own Rx of band n1.

-	5th order IMD products generated by DC_21_n77 uplink may fall into own Rx of band n1.

### 6.41.4	∆TIB and ∆RIB values

For DC_21_n1-n77, the TIB,c and RIB,c values are reused from DC_1-21_n77, and are given in the tables below.

Table 6.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n1-n77 | 21 | 0.3 |
|  | n1 | 0.3 |
|  | n77 | 0.8 |

Table 6.41.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21_n1-n77 | 21 | 0 |
|  | n1 | 0 |
|  | n77 | 0.5 |

### 6.41.5	MSD

IMD2 of band 21 and band n77 to band n1 Rx need to be addressed for REFSENS relaxation. For DC_21_n1-n77, REFSENS exceptions are reused from DC_1-21_n77, already defined in TS 38.101-3. On the other hand, IMD2 and IMD5 don’t fall into own Rx of band n77 when IMD was calculated based on the frequency range that the operator actually owned. Therefore, there is no need to have MSD added about these. NOTE X1 and NOTE X2 in Table 6.41.2-1 indicate these frequency ranges.

Table 6.41.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A_n1A-n77A | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD24 |
|  | n77 | 3605 | 10 | 50 | 3605 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.42	DC_21_n1-n78

### 6.42.1	Operating bands for DC

Table 6.42.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21_n1-n78 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.42.2	Configuration for DC

Table 6.42.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A_n1A-n78AX1,X2 | DC_21A_n1A DC_21A_n78A | 21A | CA_n1A-n78A |
| NOTE X1: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE X2: The frequency range in band n78 is restricted for this band combination to 3440 – 3520 MHz and 3600 - 3700 MHz. |  |  |  |

### 6.42.3	Co-existence studies

Based on the co-existence studies of DC_21_n1 and DC_21_n78, own Rx impact of the 3rd band is the followings.

-	2nd order IMD products generated by DC_21_n1 uplink may fall into own Rx of band n78.

-	2nd order IMD products generated by DC_21_n78 uplink may fall into own Rx of band n1.

-	5th order IMD products generated by DC_21_n78 uplink may fall into own Rx of band n1.

### 6.42.4	∆TIB and ∆RIB values

For DC_21_n1-n78, the TIB,c and RIB,c values are reused from DC_1-21_n78, and are given in the tables below.

Table 6.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n1-n78 | 21 | 0.4 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 6.42.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21_n1-n78 | 21 | 0 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 6.42.5	MSD

IMD2 of band 21 and band n78 to band n1 Rx need to be addressed for REFSENS relaxation. For DC_21_n1-n78, REFSENS exceptions are reused from DC_1-21_n78, already defined in TS 38.101-3. On the other hand, IMD2 doesn’t fall into own Rx of band n78 when IMD was calculated based on the frequency range that the operator actually owned. Therefore, there is no need to have MSD added about this. NOTE X1 and NOTE X2 in Table 6.42.2-1 indicate these frequency ranges.

Table 6.42.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A_n1A-n78A | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD24 |
|  | n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.43	DC_21_n1-n79

### 6.43.1	Operating bands for DC

Table 6.43.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21_n1-n79 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.43.2	Configuration for DC

Table 6.43.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A_n1A-n79A | DC_21A_n1A DC_21A_n79A | 21A | CA_n1A-n79A |
| NOTE X1: The frequency range in band n1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130-2150 MHz for the DL. NOTE X2: The frequency range in band n79 is restricted for this band combination to 4500 - 4600 MHz. |  |  |  |

### 6.43.3	Co-existence studies

Based on the co-existence studies of DC_21_n1 and DC_21_n79, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_21_n1 uplink may fall into own Rx of band n79.

-	4th order IMD products generated by DC_21_n1 uplink may fall into own Rx of band n79.

### 6.43.4	∆TIB and ∆RIB values

For DC_21_n1-n79, the TIB,c and RIB,c values are reused from DC_1-21_n79, and are given in the tables below.

Table 6.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n1-n79 | 21 | 0.3 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 6.43.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21_n1-n79 | 21 | 0 |
|  | n1 | 0 |
|  | n79 | 0 |

### 6.43.5	MSD

When IMD was calculated based on the frequency range that the operator actually owned, IMD3 and IMD4 don’t fall into own Rx of band n79. Therefore, there is no need to have MSD added. NOTE X1 and NOTE X2 in Table 6.43.2-1 indicate these frequency ranges.

## 6.44	DC_42_n1-n77

### 6.44.1	Operating bands for DC

Table 6.44.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n1-n77 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.44.2	Configuration for DC

Table 6.44.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_42A_n1A-n77A | DC_42A_n1A | 42A | CA_n1A-n77A |
| DC_42C_n1A-n77A | DC_42A_n1A DC_42C_n1A | CA_42C | CA_n1A-n77A |

### 6.44.3	Co-existence studies

Based on the co-existence studies of DC_42A-n1A, 4th and 5th order IMD generated by dual uplink of Band 42 + Band n1 may also fall into own Rx of band n77. But due to the TDD synchronous operation of B42 and n77, it is not impacted.

### 6.44.4	∆TIB and ∆RIB values

For DC_42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-42_n77, and are given in the tables below.

Table 6.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n1-n77 | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 6.44.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n1-n77 | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

### 6.44.5	MSD

MSD are not needed considering the co-existence studies in 6.44.3.

## 6.45	DC_42_n1-n78

### 6.45.1	Operating bands for DC

Table 6.45.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n1-n78 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.45.2	Configuration for DC

Table 6.45.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_42A_n1A-n78AX1 | N/A | 42A | CA_n1A-n78A |
| DC_42C_n1A-n78AX1 | N/A | CA_42C | CA_n1A-n78A |
| NOTE X1: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. |  |  |  |

### 6.45.3	Co-existence studies

Co-existence studies are not needed because Uplink EN-DC configuration is N/A for DC_42_n1-n78.

### 6.45.4	∆TIB and ∆RIB values

For DC_42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-42_n78, and are given in the tables below.

Table 6.45.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n1-n78 | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 6.45.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n1-n78 | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 6.45.5	MSD

MSD are not needed because Uplink EN-DC configuration is N/A for DC_42_n1-n78.

## 6.46	DC_42_n1-n79

### 6.46.1	Operating bands for DC

Table 6.46.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n1-n79 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.46.2	Configuration for DC

Table 6.46.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_42A_n1A-n79AX1 | N/A | 42A | CA_n1A-n79A |
| DC_42C_n1A-n79 AX1 | N/A | CA_42C | CA_n1A-n79A |
| NOTE X1: The combination is not used alone as fall back mode of other band combinations in which UL in Band 42 is not used. |  |  |  |

### 6.46.3	Co-existence studies

Co-existence studies are not needed because Uplink EN-DC configuration is N/A for DC_42_n1-n79.

### 6.46.4	∆TIB and ∆RIB values

For DC_42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-42_n79, and are given in the tables below.

Table 6.46.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n1-n79 | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 6.46.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n1-n79 | 42 | 0.5 |
|  | n1 | 0 |
|  | n79 | 0 |

### 6.46.5	MSD

MSD are not needed because Uplink EN-DC configuration is N/A for DC_42_n1-n79.

## 6.47	DC_28_n1-n78

### 6.47.1	Operating bands for DC

Table 6.47.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n1-n78 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.47.2	Channel bandwidths per operating band for DC

Table 6.47.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_28A_n1A-n78A | DC_28A_ n1A DC_28A_ n78A | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.47.3	Co-existence studies

Based on co-existence studies, 3rd order IMD generated by dual uplink of 28_n1 may fall into own Rx of band n78.

Based on co-existence studies, 3rd order IMD generated by dual uplink of 28_n78 may fall into own Rx of band n1.

### 6.47.4	∆TIB and ∆RIB values

For DC_28_n1-n78, the TIB,c and RIB,c values are derived from DC_1-28_n78 and are given in the tables below.

Table 6.47.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n1-n78 | 28 | 0.6 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 6.47.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n1-n78 | 28 | 0.2 |
|  | n1 | 0 |
|  | n78 | 0.5 |

### 6.47.5	MSD

As mentioned in 6.47.3, 3rd order IMD generated by dual uplink of 28_n1 may fall into own Rx of band n78. Relaxation is derived from DC_1A_n28A-n78A and proposed in table below.

As mentioned in 6.47.3, 3rd order IMD generated by dual uplink of 28_n78 may fall into own Rx of band n1. Relaxation is derived from DC_1A-28A_n78A and proposed in table below.

Table 6.47.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n1A-n78A | 28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |

## 6.48	DC_2_n5-n77

### 6.48.1	Operating bands for DC

Table 6.48.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n5-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.48.2	Channel bandwidths per operating band for DC

Table 6.48.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n5A-n77A | DC_2A_n5A DC_2A_ n77A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.48.3	Co-existence studies

Based on co-existence studies, 3rd and 5th order IMD generated by dual uplink of 2_n5 may fall into own Rx of band n77.

Based on co-existence studies, 5th order IMD generated by dual uplink of 2_n77 may fall into own Rx of band n5.

### 6.48.4	∆TIB and ∆RIB values

For DC_2_n5-n77, the TIB,c and RIB,c values are derived from CA_2-13-48 and are given in the tables below.

Table 6.48.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n5-n77 | 2 | 0.6 |
|  | n5 | 0.3 |
|  | n77 | 0.8 |

Table 6.48.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n5-n77 | 2 | 0.2 |
|  | n5 | 0 |
|  | n77 | 0.5 |

### 6.48.5	MSD

As mentioned in 6.48.3, 3rd and 5th order IMD generated by dual uplink of 2_n5 may fall into own Rx of band n77.

As mentioned in 6.48.3, 5th order IMD generated by dual uplink of 2_n77 may fall into own Rx of band n5.

Table 6.48.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n5A-n77A | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3540 | 10 | 50 | 3540 | 16.0 | IMD3 |
|  | 2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3980 | 10 | 50 | 3980 | 4.1 | IMD5 |
|  | 2 | 1907 | 5 | 25 | 1987 | N/A | N/A |
|  | n5 | 844 | 5 | 25 | 889 | 3.8 | IMD5 |
|  | n77 | 3305 | 10 | 50 | 3305 | N/A | N/A |

## 6.49	DC_2_n66-n77

### 6.49.1	Operating bands for DC

Table 6.49.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.49.2	Channel bandwidths per operating band for DC

Table 6.49.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n66A-n77A DC_2A_n66A-n77C | DC_2A_ n77A DC_2A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| DC_2A-2A_n66A-n77A DC_2A-2A_n66A-n77C | DC_2A_ n77A DC_2A_n66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 280 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.49.3	Co-existence studies

Based on co-existence studies, 2nd, 4th and 5th order IMD generated by dual uplink of 2_n77 may fall into own Rx of band n66.

Based on co-existence studies, 2nd and 4th order IMD generated by dual uplink of 2_n66 may fall into own Rx of band n77

### 6.49.4	∆TIB and ∆RIB values

For DC_2_n66-n77, the TIB,c and RIB,c values are derived from CA_n25-n66-n78 and are given in the tables below.

Table 6.49.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n66-n77 | 2 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.49.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n66-n77 | 2 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 6.49.5	MSD

As mentioned in 6.49.3, 2nd 4th and 5th order IMD generated by dual uplink of 2_n77 may fall into own Rx of band n66.

Table 6.49.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n66A-n77A11 DC_2A-2A_n66A-n77A11 | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 29.2 | IMD2 |
|  | n77 | 4060 | 10 | 50 | 4060 | N/A | N/A |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 10.3 | IMD4 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | N/A |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A | 2 | 1900 | 5 | 25 | 1980 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | 4.0 | IMD5 |
|  | n77 | 3930 | 10 | 50 | 3930 | N/A | N/A |
| DC_2A_n66A-n77A11 DC_2A-2A_n66A-n77A11 | 2 | 1853 | 5 | 25 | 1933 | N/A | N/A |
|  | n66 | 1713 | 5 | 25 | 2113 | N/A | N/A |
|  | n77 | 3566 | 10 | 50 | 3566 | 29.4 | IMD2 |
| DC_2A_n66A-n77A DC_2A-2A_n66A-n77A | 2 | 1853 | 5 | 25 | 1933 | N/A | N/A |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | N/A |
|  | n77 | 3784 | 10 | 50 | 3784 | 8.9 | IMD4 |
| NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |

## 6.50	DC_13_n2-n77

### 6.50.1	Operating bands for DC

Table 6.50.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n2-n77 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.50.2	Channel bandwidths per operating band for DC

Table 6.50.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A_n2A-n77A | DC_13A_n2A DC_13A_ n77A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.50.3	Co-existence studies

Based on co-existence studies, 3rd order IMD generated by dual uplink of 13_n77 may fall into own Rx of band n2.

Based on co-existence studies, 3rd 4th and 5th order IMD generated by dual uplink of 13_n2 may fall into own Rx of band n77.

### 6.50.4	∆TIB and ∆RIB values

For DC_13_n2-n77, the TIB,c and RIB,c values are derived from DC_13-48_n2 and are given in the tables below.

Table 6.50.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n2-n77 | 13 | 0.3 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 6.50.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n2-n77 | 13 | 0 |
|  | n2 | 0.2 |
|  | n77 | 0.5 |

### 6.50.5	MSD

As mentioned in 6.50.3, 3rd order IMD generated by dual uplink of 13_n77 may fall into own Rx of band n2.

As mentioned in 6.50.3, 3rd 4th and 5th order IMD generated by dual uplink of 13_n2 may fall into own Rx of band n77.

Table 6.50.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n2A-n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n77 | 3444 | 10 | 50 | 3444 | 17.3 | IMD3 |
|  | 13 | 780 | 5 | 25 | 749 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | 12.4 | IMD4 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n77 | 4076 | 10 | 50 | 4076 | 4.5 | IMD5 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 16.0 | IMD3 |
|  | n77 | 3524 | 10 | 50 | 3524 | N/A | N/A |

## 6.51	DC_13_n5-n48

### 6.51.1	Operating bands for DC

Table 6.51.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n5-n48 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n48 | 3550 MHz | – | 3800 MHz | 3550 MHz | – | 3800 MHz | TDD |

### 6.51.2	Channel bandwidths per operating band for DC

Table 6.51.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A_n5A-n48A | DC_13A_ n48A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | 15 | Yes5 | Yes | Yes | Yes |  |  | Yes | Yes6 |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.51.3	Co-existence studies

Based on co-existence studies, there are no IMD generated by dual uplink of 13_n48 that fall into own Rx of band n5.

### 6.51.4	∆TIB and ∆RIB values

For DC_13_n5-n48, the TIB,c and RIB,c values are derived from CA_5-12-48 and are given in the tables below.

Table 6.51.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n5-n48 | 13 | 0.4 |
|  | n5 | 0.8 |
|  | n48 | 0.3 |

Table 6.51.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n5-n48 | 13 | 0.3 |
|  | n5 | 0.5 |
|  | n48 | 0 |

### 6.51.5	MSD

There is no need for MSD to be defined.

## 6.52	DC_13_n48-n66

### 6.52.1	Operating bands for DC

Table 6.52.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n48-n66 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n48 | 3550 MHz | – | 3800 MHz | 3550 MHz | – | 3800 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.52.2	Channel bandwidths per operating band for DC

Table 6.52.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A_n48A-n66A | DC_13A_n48A DC_13A_ n66A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n48 | 15 | Yes5 | Yes | Yes | Yes |  |  | Yes | Yes6 |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.52.3	Co-existence studies

Based on co-existence studies, 5th order IMD generated by dual uplink of 13_n66 may fall into own Rx of band n48.

Based on co-existence studies, 3rd order IMD generated by dual uplink of 13_n48 may fall into own Rx of band n66.

### 6.52.4	∆TIB and ∆RIB values

For DC_13_n48-n66, the TIB,c and RIB,c values are derived from DC_13-48_n66 and are given in the tables below.

Table 6.52.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n48-n66 | 13 | 0.3 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |

Table 6.52.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n48-n66 | 13 | 0 |
|  | n48 | 0.5 |
|  | n66 | 0.2 |

### 6.52.5	MSD

As mentioned in 6.52.3, 5th order IMD generated by dual uplink of 13_n66 may fall into own Rx of band n48. Relaxation is given in the TR [2] and proposed in table below.

As mentioned in 6.52.3, 3rd order IMD generated by dual uplink of 13_n48 may fall into own Rx of band n66. Relaxation is derived from CA_13A-48A-66A and DC_13A-66A_n48A and proposed in table below.

Table 6.52.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n48A-n66A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n48 | 3584 | 5 | 25 | 3584 | 2.8 | IMD5 |
|  | n66 | 1716 | 5 | 25 | 2116 | N/A | N/A |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n48 | 3695 | 5 | 25 | 3695 | N/A | N/A |
|  | n66 | 1731 | 5 | 25 | 2131 | 17.1 | IMD3 |

## 6.53	DC_13_n66-n77

### 6.53.1	Operating bands for DC

Table 6.53.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n66-n77 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.53.2	Channel bandwidths per operating band for DC

Table 6.53.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A_n66A-n77A | DC_13A_n66A DC_13A_ n77A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.53.3	Co-existence studies

Based on co-existence studies, 3rd order IMD generated by dual uplink of 13_n77 may fall into own Rx of band n66.

Based on co-existence studies, 3rd 4th and 5th order IMD generated by dual uplink of 13_n66 may fall into own Rx of band n77.

### 6.53.4	∆TIB and ∆RIB values

For DC_13_n66-n77, the TIB,c and RIB,c values are derived from DC_13-48_n66 and are given in the tables below.

Table 6.53.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n66-n77 | 13 | 0.3 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.53.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n66-n77 | 13 | 0 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |

### 6.53.5	MSD

As mentioned in 6.53.3, 3rd order IMD generated by dual uplink of 13_n77 may fall into own Rx of band n66.

As mentioned in 6.53.3, 3rd 4th and 5th order IMD generated by dual uplink of 13_n66 may fall into own Rx of band n77.

Table 6.53.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n66A-n77A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3324 | 10 | 50 | 3324 | 16.4 | IMD3 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 4106 | 10 | 50 | 4106 | 11.7 | IMD4 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3716 | 10 | 50 | 3716 | 4.4 | IMD5 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n66 | 1731 | 5 | 25 | 2131 | 17.1 | IMD3 |
|  | n77 | 3695 | 10 | 50 | 3695 | N/A | N/A |

## 6.54	DC_66_n2-n77

### 6.54.1	Operating bands for DC

Table 6.54.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n2-n77 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.54.2	Channel bandwidths per operating band for DC

Table 6.54.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n2A-n77A | DC_66A_n2A DC_66A_ n77A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.54.3	Co-existence studies

Based on co-existence studies, 2nd, 4th and 5th order IMD generated by dual uplink of 66_n77 may fall into own Rx of band n2.

Based on co-existence studies, 2nd and 4th order IMD generated by dual uplink of 66_n2 may fall into own Rx of band n77.

### 6.54.4	∆TIB and ∆RIB values

For DC_66_n2-n77, the TIB,c and RIB,c values are derived from CA_n25-n66-n78 and are given in the tables below.

Table 6.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n2-n77 | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 6.54.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n2-n77 | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n77 | 0.5 |

### 6.54.5	MSD

As mentioned in 6.54.3, 2nd 4th and 5th order IMD generated by dual uplink of 66_n77 may fall into own Rx of band n2. Relaxation are derived from DC_2A-66A_n78A and proposed in table below.

As mentioned in 6.54.3, 2nd and 4th order IMD generated by dual uplink of 66_n2 may fall into own Rx of band n77. Relaxation are derived from DC_2A_n66A-n78A and proposed in table below.

Table 6.54.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n2A-n77A | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 29.4 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | N/A |
|  | n77 | 3340 | 10 | 50 | 3340 | 8.9 | IMD4 |
|  | 2 | 1880 | 5 | 25 | 1960 | 32.1 | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3700 | 10 | 50 | 3700 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 9.1 | IMD4 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 2 | 1880 | 5 | 25 | 1960 | 2.1 | IMD5 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | N/A |

## 6.55	DC_66_n5-n48

### 6.55.1	Operating bands for DC

Table 6.55.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n5-n48 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n48 | 3350 MHz | – | 3700 MHz | 3350 MHz | – | 3700 MHz | TDD |

### 6.55.2	Channel bandwidths per operating band for DC

Table 6.55.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n5A-n48A | DC_66A_n5A DC_66A_ n48A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | 15 | Yes5 | Yes | Yes | Yes |  |  | Yes | Yes6 |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.55.3	Co-existence studies

Based on co-existence studies, 5th order IMD generated by dual uplink of 66_n5 may fall into own Rx of band n48.

### 6.55.4	∆TIB and ∆RIB values

For DC_66_n5-n48, the TIB,c and RIB,c values are derived from DC_48-66_n5 and are given in the tables below.

Table 6.55.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n5-n48 | 66 | 0.6 |
|  | n5 | 0.3 |
|  | n48 | 0.8 |

Table 6.55.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n5-n48 | 66 | 0.2 |
|  | n5 | 0 |
|  | n48 | 0.5 |

### 6.55.5	MSD

As mentioned in 6.55.3, 5th order IMD generated by dual uplink of 66_n5 may fall into own Rx of band n48. Relaxation are defined in Rel-16 TR [2] and proposed in table below.

Table 6.55.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n5A-n48A | 66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | N/A | N/A |
|  | n48 | 3582 | 5 | 25 | 3582 | 3.3 | IMD5 |

## 6.56	DC_66_n5-n77

### 6.56.1	Operating bands for DC

Table 6.56.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n5-n77 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.56.2	Channel bandwidths per operating band for DC

Table 6.56.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n5A-n77A | DC_66A_n5A DC_66A_ n77A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| DC_66A-66A_n5A-n77A | DC_66A_n5A DC_66A_ n77A | 66 | See CA_66A-66A Bandwidth Combination Set 0 in TS 36.101 Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.56.3	Co-existence studies

Based on co-existence studies, 3rd 4th and 5th order IMD generated by dual uplink of 66_n5 may fall into own Rx of band n77.

### 6.56.4	∆TIB and ∆RIB values

For DC_66_n5-n77, the TIB,c and RIB,c values are derived from DC_48-66_n5 and are given in the tables below.

Table 6.56.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n5-n77 | 66 | 0.6 |
|  | n5 | 0.3 |
|  | n77 | 0.8 |

Table 6.56.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n5-n77 | 66 | 0.2 |
|  | n5 | 0 |
|  | n77 | 0.5 |

### 6.56.5	MSD

As mentioned in 6.56.3, 3rd 4th and 5th order IMD generated by dual uplink of 66_n5 may fall into own Rx of band n77.

Table 6.56.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n5A-n77A DC_66A-66A_n5A-n77A | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3420 | 10 | 50 | 3420 | 16.6 | IMD3 |
|  | 66 | 1714 | 5 | 25 | 2114 | N/A | N/A |
|  | n5 | 827 | 5 | 25 | 872 | N/A | N/A |
|  | n77 | 4195 | 10 | 50 | 4195 | 12.1 | IMD4 |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n5 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | 3.6 | IMD5 |

## 6.57	DC_66_n66-n77

### 6.57.1	Operating bands for DC

Table 6.57.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n66-n77 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.57.2	Channel bandwidths per operating band for DC

Table 6.57.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n66A-n77A | DC_66A_ n77A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.57.3	Co-existence studies

Based on co-existence studies, 2nd and 5th order IMD generated by dual uplink of 66_n77 may fall into own Rx of band n66.

### 6.57.4	∆TIB and ∆RIB values

For DC_66_n66-n77, the TIB,c and RIB,c values are derived from CA_n66-n77 and are given in the tables below.

Table 6.57.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n66-n77 | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.57.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n66-n77 | 66 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |

### 6.57.5	MSD

As mentioned in 6.57.3, 2nd and 5th order IMD generated by dual uplink of 66_n77 may fall into own Rx of band n66. Relaxation are derived from CA_n66A-n77A and proposed in table below.

Table 6.57.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n66A-n77A | 66 | 1730 | 5 | 25 | 2140 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2170 | 31 | IMD2 |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
|  | 66 | 1730 | 5 | 25 | 2140 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2170 | 5.0 | IMD5 |
|  | n77 | 3680 | 10 | 50 | 3680 | N/A | N/A |

## 6.58	DC_40_n41-n258

### 6.58.1		Operating bands for DC

Table 6.58.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_40_n41-n258 | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n257 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.58.2		Configuration for DC

Table 6.58.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_40A_n41A-n258A DC_40A_n41C-n258A | DC_40A_n41A DC_40A_n258A |

### 6.58.3		Co-existence studies

- For combination of DC_40_n41, the co-existence studies have been covered in TR37.716-11-11, where band 40 and band n41 are synchronized operaiton.

- For combination of DC_40_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

### 6.58.4	∆TIB and ∆RIB values

For the DC conbination of DC_40_n41-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 40 and n79 are the same with the DC_40_n41.

### 6.58.5	MSD

No additional MSD requirement is needed.

## 6.59	DC_40_n79-n258

### 6.59.1		Operating bands for DC

Table 6.59.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_40_n79-n258 | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.59.2		Configuration for DC

Table 6.59.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_40A_n79A-n258A DC_40A_n79C-n258A | DC_40A_n79A DC_40A_n258A |

### 6.59.3		Co-existence studies

- For combination of DC_40_n79, the co-existence studies have been covered in TR37.716-11-11, where 2nd harmonic from B40 into n79

- For combination of DC_40_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

### 6.59.4	∆TIB and ∆RIB values

For the DC conbination of DC_40_n79-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 40 and n79 are the same with the DC_40_n79.

### 6.59.5	MSD

No additional MSD requirement is needed.

## 6.60		DC_41_n79-n258

### 6.60.1		Operating bands for DC

Table 6.60.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41_n79-n258 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n257 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.60.2	Configuration for DC

Table 6.60.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_41A_n79A-n258A DC_41A_n79C-n258A | DC_41A_n79A DC_41A_n258A |

### 6.60.3	Co-existence studies

- For combination of DC_41_n79, the co-existence studies have been covered in TR37.716-11-11, where over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used.

- For combination of DC_41_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

### 6.60.4	∆TIB and ∆RIB values

For the DC conbination of DC_41_n79-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 41 and n79 are the same with the DC_41_n79.

### 6.60.5	MSD

No additional MSD requirement is needed.

## 6.61	DC_8_n78-n258

### 6.61.1	Operating bands for DC

Table 6.61.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n78-n258 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

### 6.61.2	Channel bandwidths per operating band for DC

Table 6.61.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_8A_n78A-n258A DC_8A_n78A-n258D DC_8A_n78A-n258E DC_8A_n78A-n258F DC_8A_n78A-n258G DC_8A_n78A-n258H DC_8A_n78A-n258I DC_8A_n78A-n258J DC_8A_n78A-n258K DC_8A_n78A-n258L DC_8A_n78A-n258M | DC_8A_n78A DC_8A_n258A | 8 | - | 310 410 510 610 710 810 910 |
|  |  | n78 | CA_n78A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

### 6.61.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures this combination. There is no additional harmonic and/or intermodulation impact for this combination.

### 6.61.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

### 6.61.5	MSD

No additional MSD requirement is needed.

## 6.62	DC_8_n40-n258

### 6.62.1	Operating bands for DC

Table 6.62.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n40-n258 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

### 6.62.2	Channel bandwidths per operating band for DC

Table 6.62.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_8A_n40A-n258A DC_8A_n40A-n258D DC_8A_n40A-n258E DC_8A_n40A-n258F DC_8A_n40A-n258G DC_8A_n40A-n258H DC_8A_n40A-n258I DC_8A_n40A-n258J DC_8A_n40A-n258K DC_8A_n40A-n258L DC_8A_n40A-n258M | DC_8A_n40A DC_8A_n258A | 8 | - | 290 390 490 590 690 790 890 |
|  |  | n40 | CA_n40A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

### 6.62.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures this combination. There is no additional harmonic and/or intermodulation impact for this combination.

### 6.62.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

### 6.62.5	MSD

No additional MSD requirement is needed.

## 6.63	DC_7_n78-n258

### 6.63.1	Operating bands for DC

Table 6.63.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n78-n258 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.63.2	Channel bandwidths per operating band for DC

Table 6.63.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_7A_n78A-n258A DC_7A_n78A-n258G DC_7A_n78A-n258H DC_7A_n78A-n258I DC_7A_n78A-n258J DC_7A_n78A-n258K DC_7A_n78A-n258L DC_7A_n78A-n258M DC_7C_n78A-n258A DC_7C_n78A-n258G DC_7C_n78A-n258H DC_7C_n78A-n258I DC_7C_n78A-n258J DC_7C_n78A-n258K DC_7C_n78A-n258L DC_7C_n78A-n258M | DC_7A_n78A DC_7A_n258A DC_7C_n78A DC_7C_n258A | 7 | - | 920 |
|  |  | n78 | - |  |
|  |  | n258 | - |  |

### 6.63.3	Co-existence studies

Based on co-existence studies of DC_7_n78 and DC_7_n258 specified in TR 37.717-11-21, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 6.63.4	∆TIB and ∆RIB values

For DC_7_n78-n258, the TIB,c and RIB values are given in the tables below.

Table 6.63.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n78-n258 | 7 | 0.5 |
|  | n78 | 0.8 |
|  | n258 | 0 |

Table 6.63.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n78-n258 | 7 | 0 |
|  | n78 | 0.5 |
|  | n258 | 0 |

### 6.63.5	MSD

There is no additional MSD issue for DC_7_n78-n258.

## 6.64	DC_3_n78-n258

### 6.64.1	Operating bands for DC

Table 6.64.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n78-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.64.2	Channel bandwidths per operating band for DC

Table 6.64.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A_n78A-n258A DC_3A_n78A-n258G DC_3A_n78A-n258H DC_3A_n78A-n258I DC_3A_n78A-n258J DC_3A_n78A-n258K DC_3A_n78A-n258L DC_3A_n78A-n258M | DC_3A_n78A DC_3A_n258A | 3 | - | 920 |
|  |  | n78 | - |  |
|  |  | n258 | - |  |

### 6.64.3	Co-existence studies

Based on co-existence studies of DC_3_n78 and DC_3_n258 specified in TR 37.717-11-21, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 6.64.4	∆TIB and ∆RIB values

For DC_3_n78-n258, the TIB,c and RIB values are given in the tables below.

Table 6.64.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n78-n258 | 3 | 0.6 |
|  | n78 | 0.8 |
|  | n258 | 0 |

Table 6.64.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n78-n258 | 3 | 0.2 |
|  | n78 | 0.5 |
|  | n258 | 0 |

### 6.64.5	MSD

There is no additional MSD issue for DC_3_n78-n258.

## 6.65	DC_28_n78-n258

### 6.65.1	Operating bands for DC

Table 6.65.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n78-n258 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.65.2	Channel bandwidths per operating band for DC

Table 6.65.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_28A_n78A-n258A DC_28A_n78A-n258G DC_28A_n78A-n258H DC_28A_n78A-n258I DC_28A_n78A-n258J DC_28A_n78A-n258K DC_28A_n78A-n258L DC_28A_n78A-n258M | DC_28A_n78A DC_28A_n258A | 28 | - | 920 |
|  |  | n78 | - |  |
|  |  | n258 | - |  |

### 6.65.3	Co-existence studies

Based on co-existence studies of DC_28_n78 and DC_28_n258 specified in TR 37.717-11-21, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 6.65.4	∆TIB and ∆RIB values

For DC_28_n78-n258, the TIB,c and RIB values are given in the tables below.

Table 6.65.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n78-n258 | 28 | 0.5 |
|  | n78 | 0.8 |
|  | n258 | 0 |

Table 6.65.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n78-n258 | 28 | 0.2 |
|  | n78 | 0.5 |
|  | n258 | 0 |

### 6.65.5	MSD

There is no additional MSD issue for DC_28_n78-n258.

## 6.66	DC_40_n1-n78

### 6.66.1	 Operating bands for DC

Table 6.66.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_40A_n1A-n78A DC_40C_n1A-n78A | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n1 | 1920 MHz |  | 1980 MHz | 1920 MHz |  | 1980 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.66.2 	Configuration for DC

Table 6.66.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_40A_n1A-n78A DC_40C_n1A-n78A |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.66.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_40A_n1A and DC_40A_n78A are shown below.

Table 6.66.3-1: The harmonic and IMD products caused by UL DC_40A_n1A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2300 | 2400 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4600 | 4800 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6900 | 7200 | 5760 | 5940 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9200 | 9600 | 7680 | 7920 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 11500 | 12000 | 9600 | 9900 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 480 | 320 | 4220 | 4380 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2620 | 2880 | 1440 | 1660 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6520 | 6780 | 6140 | 6360 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4920 | 5280 | 3360 | 3640 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8820 | 9180 | 8060 | 8340 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 640 | 960 | 8440 | 8760 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5620 | 5280 | 7680 | 7220 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1340 | 960 | 3360 | 2940 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9980 | 10320 | 11120 | 11580 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10360 | 10740 | 10740 | 11160 |

Table 6.66.3-2: The harmonic and IMD products caused by UL DC_40A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2300 | 2400 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4600 | 4800 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6900 | 7200 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9200 | 9600 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 11500 | 12000 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 900 | 1500 | 5600 | 6200 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 | 1500 | 4200 | 5300 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7900 | 8600 | 8900 | 10000 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3100 | 3900 | 7500 | 9100 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10200 | 11000 | 12200 | 13800 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3000 | 1800 | 11200 | 12400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12900 | 10800 | 6300 | 5400 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6800 | 5100 | 600 | 700 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15500 | 17600 | 12500 | 13400 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14500 | 16200 | 13500 | 14800 |

IMD4 and IMD5 caused by UL DC_40_n1 may fall into Rx band n78.

IMD4 caused by UL DC_40_n78 may fall into Rx band n1.

### 6.66.4	 ∆TIB and ∆RIB values

Table 6.66.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_40_n1-n78 | 40 | 0.5 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 6.66.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_40_n1-n78 | 40 | 0.4 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 6.66.5	MSD

All MSD requirements for DC_40A_n1A-n78A can reuse the requriements of DC_1A-40A_n78A and DC_1A_n40A-n78A. The MSD exceptions are shown below.

Table 6.66.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_40A_n1A-n78A DC_40C_n1A-n78A | 40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n78 | 3450 | 10 | 50 | 3450 | 9.8 | IMD4 |
|  | 40 | 2360 | 5 | 25 | 2360 | N/A | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 9.1 | IMD4 |

## 6.67	DC_12_n5-n77

### 6.67.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 produced by Band 12 and NR band n5 that impact the reference sensitivity of NR band n77. Also, there is IMD5 produced by Band 12 and NR band n77 that impact the reference sensitivity of NR band n5.

The required MSD levels and test configuration are shown in the following Table.

Table 6.67.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n5A-n77A | 12 | IMD5 | ¦4*fB12 +fn5¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 3662 | 10 | 50 | 3662 | 10 | 5.4 |
|  | 12 | IMD5 | ¦4*fB12 -fn77¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n77 |  |  | 3707 | 10 | 50 | 3707 | 10 |  |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 | 4.8 |

## 6.68	DC_14_n5-n77

### 6.68.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD4 and IMD5 produced by Band 14 and NR band n5 that impact the reference sensitivity of NR band n77. Also, there is IMD5 produced by Band 14 and NR band n77 that impact the reference sensitivity of NR band n5.

The required MSD levels and test configuration are shown in the following Table.

Table 6.68.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_14A_n5A-n77A | 14 | IMD4 | ¦fB14 +3*fn5¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n5 |  |  | 845 | 5 | 25 | 890 | 5 |  |
|  | n77 |  |  | 3328 | 10 | 50 | 3328 | 10 | 10.6 |
|  | 14 | IMD5 | ¦fB14 +4*fn5¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 |  |
|  | n77 |  |  | 4113 | 10 | 50 | 4113 | 10 | 5.1 |
|  | 14 | IMD5 | ¦4*fB14 -fn77¦ | 793 | 5 | 25 | 763 | 5 | N/A |
|  | n77 |  |  | 4047 | 10 | 50 | 4047 | 10 |  |
|  | n5 |  |  | 830 | 5 | 25 | 875 | 5 | 4.4 |

## 6.69	DC_2_n71-n78

### 6.69.1	Operating bands for DC

Table 6.69.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n71-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.69.2	Channel bandwidths per operating band for DC

Table 6.69.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n71A-n78A | DC_2A_n71A DC_2A_ n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.69.3	Co-existence studies

Based on co-existence studies, IMD3 generated by dual uplink of 2_n71 may fall into own Rx of band n78.

Based on co-existence studies, there are no IMD generated by dual uplink of 2_n78 into own Rx of band n71.

### 6.69.4	∆TIB and ∆RIB values

For DC_2_n71-n78, the TIB,c and RIB,c values are derived from DC_2-71_n78 and are given in the tables below.

Table 6.69.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n71-n78 | 2 | 0.6 |
|  | n71 | 0.6 |
|  | n78 | 0.8 |

Table 6.69.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n71-n78 | 2 | 0.2 |
|  | n71 | 0.2 |
|  | n78 | 0.5 |

### 6.69.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD3 produced by Band 2 and NR band n71 that impact the reference sensitivity of NR band n78.

The required MSD levels and test configuration are shown in the following Table.

Table 6.69.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n71A-n78A | 2 | IMD3 | ¦fB2 +2*fn71¦ | 1907.5 | 5 | 25 | 1987.5 | 5 | N/A |
|  | n71 |  |  | 695.5 | 5 | 25 | 649.5 | 5 |  |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 | 8.0 |

## 6.70	DC_7_n71-n78

### 6.70.1	Operating bands for DC

Table 6.70.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n71-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.70.2	Channel bandwidths per operating band for DC

Table 6.70.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A_n71A-n78A | DC_7A_n71A DC_7A_ n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.70.3	Co-existence studies

Based on co-existence studies, IMD4 generated by dual uplink of 7_n71 may fall into own Rx of band n78.

Based on co-existence studies, IMD5 generated by dual uplink of 7_n78 may fall into own Rx of band n71.

### 6.70.4	∆TIB and ∆RIB values

For DC_7_n71-n78, the TIB,c and RIB,c values are derived from DC_41_n28-n78 and are given in the tables below.

Table 6.70.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n71-n78 | 7 | 0.3 |
|  | n71 | 0.5 |
|  | n78 | 0.8 |

Table 6.70.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n71-n78 | 7 | 0 |
|  | n71 | 0.2 |
|  | n78 | 0.5 |

### 6.70.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD4 produced by Band 7 and NR band n71 that impact the reference sensitivity of NR band n78. Also there is IMD5 produced by Band 7 and NR band n78 that impact the reference sensitivity of NR band n71.

The required MSD levels and test configuration are shown in the following Table.

Table 6.70.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n71A-n78A | 7 | IMD4 | ¦2*fB7 -2*fn71¦ | 2550 | 5 | 25 | 2670 | 5 | N/A |
|  | n71 |  |  | 693 | 5 | 25 | 647 | 5 |  |
|  | n78 |  |  | 3714 | 10 | 50 | 3714 | 10 | 9.7 |
|  | 7 | IMD5 | ¦3*fB7 -2*fn78¦ | 2555 | 5 | 25 | 2675 | 5 | N/A |
|  | n78 |  |  | 3520 | 10 | 50 | 3520 | 10 |  |
|  | n71 |  |  | 671 | 5 | 25 | 625 | 5 | 3.9 |

## 6.71	DC_71_n38-n78

### 6.71.1	Operating bands for DC

Table 6.71.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n38-n78 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.71.2	Channel bandwidths per operating band for DC

Table 6.71.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_71A_n38A-n78A | DC_71A_n38A DC_71A_ n78A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.71.3	Co-existence studies

Based on co-existence studies, IMD2 and IMD4 generated by dual uplink of 71_n38 may fall into own Rx of band n78.

Based on co-existence studies, IMD2 generated by dual uplink of 71_n78 may fall into own Rx of band n38.

### 6.71.4	∆TIB and ∆RIB values

For DC_71_n38-n78, the TIB,c and RIB,c values are derived from DC_41_n28-n78 and are given in the tables below.

Table 6.71.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n38-n78 | 71 | 0.5 |
|  | n38 | 0.3 |
|  | n78 | 0.8 |

Table 6.71.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_71_n38-n78 | 71 | 0.2 |
|  | n38 | 0 |
|  | n78 | 0.5 |

### 6.71.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD2 and IMD4 produced by Band 71 and NR band n38 that impact the reference sensitivity of NR band n78. Also there is IMD2 produced by Band 71 and NR band n78 that impact the reference sensitivity of NR band n38.

The required MSD levels and test configuration are shown in the following Table.

Table 6.71.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_71A_n38A-n78A | 71 | IMD2 | ¦fB71 +fn38¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n38 |  |  | 2615 | 5 | 25 | 2615 | 5 |  |
|  | n78 |  |  | 3308 | 10 | 50 | 3308 | 10 | 29.1 |
|  | 71 | IMD4 | ¦2*fB71 -2*fn38¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n38 |  |  | 2580 | 5 | 25 | 2580 | 5 |  |
|  | n78 |  |  | 3774 | 10 | 50 | 3774 | 10 | 10.3 |
|  | 71 | IMD2 | ¦fB71 -fn78¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n78 |  |  | 3308 | 10 | 50 | 3308 | 10 |  |
|  | n38 |  |  | 2615 | 5 | 25 | 2615 | 5 | 28.7 |

## 6.72	DC_5_n66-n78

### 6.72.1	Operating bands for DC

Table 6.72.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n66-n78 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.72.2	Channel bandwidths per operating band for DC

Table 6.72.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_5A_n66A-n78A | DC_5A_n66A DC_5A_ n78A | 5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.72.3	Co-existence studies

Based on co-existence studies, IMD3 and IMD5 generated by dual uplink of 5_n66 may fall into own Rx of band n78.

Based on co-existence studies, IMD3 generated by dual uplink of 5_n78 may fall into own Rx of band n66.

### 6.72.4	∆TIB and ∆RIB values

For DC_5_n66-n78, the TIB,c and RIB,c values are derived from DC_5-66_n78 and are given in the tables below.

Table 6.72.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n66-n78 | 71 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 6.72.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n66-n78 | 71 | 0.2 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |

### 6.72.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD3 and IMD5 produced by Band 5 and NR band n66 that impact the reference sensitivity of NR band n78.

The required MSD levels and test configuration are shown in the following Table.

Table 6.72.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n66A-n78A | 5 | IMD3 | ¦fn66 +2*fB5¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n78 |  |  | 3420 | 10 | 50 | 3420 | 10 | 16.6 |
|  | 5 | IMD5 | ¦3*fn66 -2*fB5¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n78 |  |  | 3620 | 10 | 50 | 3620 | 10 | 3.6 |

## 6.73	DC_71_n66-n78

### 6.73.1	Operating bands for DC

Table 6.73.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n66-n78 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.73.2	Channel bandwidths per operating band for DC

Table 6.73.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_71A_n66A-n78A | DC_71A_n66A DC_71A_ n78A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.73.3	Co-existence studies

Based on co-existence studies, IMD4 generated by dual uplink of 71_n66 may fall into own Rx of band n78.

Based on co-existence studies, IMD3 generated by dual uplink of 71_n78 may fall into own Rx of band n66.

### 6.73.4	∆TIB and ∆RIB values

For DC_71_n66-n78, the TIB,c and RIB,c values are derived from DC_66-71_n78 and are given in the tables below.

Table 6.73.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n66-n78 | 71 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 6.73.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_71_n66-n78 | 71 | 0.2 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |

### 6.73.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD3 produced by Band 71 and NR band n78 that impact the reference sensitivity of NR band n66.

The required MSD levels and test configuration are shown in the following Table.

Table 6.73.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_71A_n66A-n78A | 71 | IMD3 | ¦2*fB71 -fn78¦ | 693 | 5 | 25 | 647 | 5 | N/A |
|  | n78 |  |  | 3546 | 10 | 50 | 3546 | 10 |  |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 | 15.5 |

## 6.74	DC_5_n38-n66

### 6.74.1	Operating bands for DC

Table 6.74.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n38-n66 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.74.2	Channel bandwidths per operating band for DC

Table 6.74.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_5A_n38A-n66A | DC_5A_n38A DC_5A_ n66A | 5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 6.74.3	Co-existence studies

Based on co-existence studies, there are no issues with dual uplink of 5_n38 falling into own Rx of band n66.

Based on co-existence studies, IMD2 and IMD3 generated by dual uplink of 5_n66 may fall into own Rx of band n38.

### 6.74.4	∆TIB and ∆RIB values

For DC_5_n38-n66, the TIB,c and RIB,c values are derived from DC_5-7_n71 and are given in the tables below.

Table 6.74.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n38-n66 | 5 | 0.5 |
|  | n38 | 0.8 |
|  | n66 | 0.5 |

Table 6.74.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n38-n66 | 5 | 0.2 |
|  | n38 | 0 |
|  | n66 | 0 |

### 6.74.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD2 and IMD3 produced by Band 5 and NR band n66 that impact the reference sensitivity of NR band n38.

The required MSD levels and test configuration are shown in the following Table.

Table 6.74.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n38A-n66A | 5 | IMD2 | ¦fB5 +fn66¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1760 | 5 | 25 | 2160 | 5 |  |
|  | n38 |  |  | 2590 | 5 | 25 | 2590 | 5 | 28.9 |
|  | 5 | IMD3 | ¦fB5 -2*fn66¦ | 830 | 5 | 25 | 875 | 5 | N/A |
|  | n66 |  |  | 1720 | 5 | 25 | 2120 | 5 |  |
|  | n38 |  |  | 2610 | 5 | 25 | 2610 | 5 | 16.4 |

## 6.75	DC_12_n2-n38

### 6.75.1	Operating bands for DC

Table 6.75.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n2-n38 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

### 6.75.2	Channel bandwidths per operating band for DC

Table 6.75.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_12A_n2A-n38A | DC_12A_n2A DC_12A_ n38A | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 70 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 6.75.3	Co-existence studies

Based on co-existence studies, there are no IMD generated by dual uplink of 12_n38 into own Rx of band n2.

Based on co-existence studies, IMD2 generated by dual uplink of 12_n2 may fall into own Rx of band n38.

### 6.75.4	∆TIB and ∆RIB values

For DC_12_n2-n38, the TIB,c and RIB,c values are derived from DC_2_n41-n71 and are given in the tables below.

Table 6.75.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n2-n38 | 12 | 0.3 |
|  | n2 | 0.5 |
|  | n38 | 0.5 |

Table 6.75.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12_n2-n38 | 12 | 0 |
|  | n2 | 0 |
|  | n38 | 0 |

### 6.75.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 12 and NR band n2 that impact the reference sensitivity of NR band n38.

The required MSD levels and test configuration are shown in the following Table.

Table 6.75.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n2A-n38A | 12 | IMD2 | ¦fn2 +fB12¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 |  |
|  | n38 |  |  | 2608 | 5 | 25 | 2608 | 5 | 28.7 |

## 6.76	DC_12_n2-n41

### 6.76.1	Operating bands for DC

Table 6.76.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n2-n41 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.76.2	Channel bandwidths per operating band for DC

Table 6.76.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_12A_n2A-n41A | DC_12A_n2A DC_12A_ n41A | 12 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 6.76.3	Co-existence studies

Based on co-existence studies, there are no IMD generated by dual uplink of 12_n41 into own Rx of band n2.

Based on co-existence studies, IMD2 generated by dual uplink of 12_n2 may fall into own Rx of band n41.

### 6.76.4	∆TIB and ∆RIB values

For DC_12_n2-n41, the TIB,c and RIB,c values are derived from DC_2_n41-n71 and are given in the tables below.

Table 6.76.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n2-n41 | 12 | 0.3 |
|  | n2 | 0.5 |
|  | n41 | 0.5 |

Table 6.76.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12_n2-n41 | 12 | 0 |
|  | n2 | 0 |
|  | n41 | 0 |

### 6.76.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 12 and NR band n2 that impact the reference sensitivity of NR band n41.

The required MSD levels and test configuration are shown in the following Table.

Table 6.76.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n2A-n41A | 12 | IMD2 | ¦fn2 +fB12¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 |  |
|  | n41 |  |  | 2608 | 5 | 25 | 2608 | 5 | 28.7 |

## 6.77	DC_71_n2-n78

### 6.77.1	Operating bands for DC

Table 6.77.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n2-n78 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.77.2	Channel bandwidths per operating band for DC

Table 6.77.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_71A_n2A-n78A | DC_71A_n2A DC_71A_ n78A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.77.3	Co-existence studies

Based on co-existence studies, IMD3 generated by dual uplink of 71_n2 may fall into own Rx of band n78.

Based on co-existence studies, IMD3 generated by dual uplink of 71_n78 may fall into own Rx of band n2.

### 6.77.4	∆TIB and ∆RIB values

For DC_71_n2-n78, the TIB,c and RIB,c values are derived from DC_2-71_n78 and are given in the tables below.

Table 6.77.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n2-n78 | 71 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 6.77.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_71_n2-n78 | 71 | 0.2 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

### 6.77.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD3 produced by Band 71 and NR band n2 that impact the reference sensitivity of NR band n78.

The required MSD levels and test configuration are shown in the following Table.

Table 6.77.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_71A_n2A-n78A | n2 | IMD3 | ¦fn2 +2*fB71¦ | 1907.5 | 5 | 25 | 1987.5 | 5 | N/A |
|  | B71 |  |  | 695.5 | 5 | 25 | 649.5 | 5 |  |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 | 8.0 |

## 6.78	DC_1_n28-n79

### 6.78.1	Operating bands for DC

Table 6.78.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n28-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.78.2	Configuration for DC

Table 6.78.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A_n28A-n79A | DC_1A_n28A DC_1A_n79A | 1A | CA_n28A-n79A |

### 6.78.3	Co-existence studies

Based on the co-existence studies of DC_1_n28 and DC_1_n79, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_1_n28 uplink may fall into own Rx of band n79.

-	5th order IMD products generated by DC_1_n28 uplink may fall into own Rx of band n79.

-	3rd order IMD products generated by DC_1_n79 uplink may fall into own Rx of band n28.

-	4th order IMD products generated by DC_1_n79 uplink may fall into own Rx of band n28.

### 6.78.4	∆TIB and ∆RIB values

For DC_1_n28-n79, the TIB,c and RIB,c values are reused from DC_1-28_n79, and are given in the tables below.

Table 6.78.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n79 | 1 | 0.3 |
|  | n28 | 0.6 |
|  | n79 | 0 |

Table 6.78.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n79 | 1 | 0 |
|  | n28 | 0.2 |
|  | n79 | 0 |

### 6.78.5	MSD

-	3rd and 5th order IMD products generated by DC_1_n28 uplink may fall into own Rx of band n79.

- The MSD values are shown in the following table. These values are the average of the analysis results of the two companies. [1][2]

-	3rd and 4th order IMD products generated by DC_1_n79 uplink may fall into own Rx of band n28.

- For DC_1_n28-n79, REFSENS exceptions are reused from DC_1-28_n79, already defined in TS 38.101-3. The MSD values are shown in the following table.

Table 6.78.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n28A-n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | 15.2 | IMD3X1 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n79 | 4630 | 40 | 216 | 4630 | 14.9 | IMD34 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE X1: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.79	DC_3_n28-n79

### 6.79.1	Operating bands for DC

Table 6.79.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n28-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.79.2	Configuration for DC

Table 6.79.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A_n28A-n79A | DC_3A_n28A DC_3A_n79A | 3A | CA_n28A-n79A |

### 6.79.3	Co-existence studies

Based on the co-existence studies of DC_3_n28 and DC_3_n79, own Rx impact of the 3rd band is the followings.

-	4th order IMD products generated by DC_3_n28 uplink may fall into own Rx of band n79.

-	5th order IMD products generated by DC_3_n28 uplink may fall into own Rx of band n79.

-	4th order IMD products generated by DC_3_n79 uplink may fall into own Rx of band n28.

### 6.79.4	∆TIB and ∆RIB values

For DC_3_n28-n79, the TIB,c and RIB,c values are reused from DC_3-28_n79, and are given in the tables below.

Table 6.79.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n79 | 3 | 0.3 |
|  | n28 | 0.3 |
|  | n79 | 0 |

Table 6.79.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n79 | 3 | 0 |
|  | n28 | 0 |
|  | n79 | 0 |

### 6.79.5	MSD

-	4th and 5th order IMD products generated by DC_3_n28 uplink may fall into own Rx of band n79.

- The MSD values are shown in the following table. These values are the average of the analysis results of the two companies. [1][2]

-	4th order IMD products generated by DC_3_n79 uplink may fall into own Rx of band n28.

- For DC_3_n28-n79, REFSENS exceptions are reused from DC_3-28_n79, already defined in TS 38.101-3. The MSD values are shown in the following table.

Table 6.79.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n28A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n79 | 4585 | 40 | 216 | 4585 | 9.4 | IMD44 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.80	DC_21_n28-n77

### 6.80.1	Operating bands for DC

The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783-793 MHz for the DL because only a certain operator uses band 21. This restriction is mentioned in TP for DC_21_n28 (R4-2100352).

Table 6.80.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21_n28-n77 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.80.2	Configuration for DC

Table 6.80.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A_n28A-n77A | DC_21A_n28A DC_21A_n77A | 21A | CA_n28A-n77A |

### 6.80.3	Co-existence studies

Based on the co-existence studies of DC_21_n28 and DC_21_n77, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n77.

-	4th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n77.

-	3rd order IMD products generated by DC_21_n77 uplink may fall into own Rx of band n28.

-	4th order IMD products generated by DC_21_n77 uplink may fall into own Rx of band n28.

### 6.80.4	∆TIB and ∆RIB values

For DC_21_n28-n77, the TIB,c and RIB,c values are reused from DC_21-28_n77, and are given in the tables below.

Table 6.80.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n28-n77 | 21 | 0.4 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 6.80.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21_n28-n77 | 21 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 6.80.5	MSD

-	3rd and 4th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n77.

- The MSD values are shown in the following table. These values are the average of the analysis results of the two companies. [1][2]

-	3rd and 4th order IMD products generated by DC_21_n77 uplink may fall into own Rx of band n28.

- For DC_21_n28-n77, REFSENS exceptions are reused from DC_21-28_n77, already defined in TS 38.101-3. The MSD values are shown in the following table.

Table 6.80.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A_n28A-n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | n28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD3X1 |
|  | n77 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | n28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n77 | 3634.5 | 10 | 50 | 3634.5 | 17.3 | IMD3X1 |
| NOTE X1: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.81	DC_21_n28-n78

### 6.81.1	Operating bands for DC

The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783-793 MHz for the DL because only a certain operator uses band 21. This restriction is mentioned in TP for DC_21_n28 (R4-2100352).

Table 6.81.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21_n28-n78 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.81.2	Configuration for DC

Table 6.81.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A_n28A-n78A | DC_21A_n28A DC_21A_n78A | 21A | CA_n28A-n78A |

### 6.81.3	Co-existence studies

Based on the co-existence studies of DC_21_n28 and DC_21_n78, own Rx impact of the 3rd band is the followings.

-	3rd order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n78.

-	4th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n78.

-	3rd order IMD products generated by DC_21_n78 uplink may fall into own Rx of band n28.

-	4th order IMD products generated by DC_21_n78 uplink may fall into own Rx of band n28.

### 6.81.4	∆TIB and ∆RIB values

For DC_21_n28-n78, the TIB,c and RIB,c values are reused from DC_21-28_n78, and are given in the tables below.

Table 6.81.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n28-n78 | 21 | 0.4 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 6.81.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21_n28-n78 | 21 | 0.5 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 6.81.5	MSD

-	3rd and 4th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n78.

- The MSD values are shown in the following table. These values are the average of the analysis results of the two companies. [1][2]

-	3rd and 4th order IMD products generated by DC_21_n78 uplink may fall into own Rx of band n28.

- For DC_21_n28-n78, REFSENS exceptions are reused from DC_21-28_n78, already defined in TS 38.101-3. The MSD values are shown in the following table.

Table 6.81.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A_n28A-n78A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | n28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD3X1 |
|  | n78 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | n28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n78 | 3634.5 | 10 | 50 | 3634.5 | 17.3 | IMD3X1 |
| NOTE X1: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.82	DC_66_n71-n78

### 6.82.1	Operating bands for DC

Table 6.82.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n71-n78 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.82.2	Channel bandwidths per operating band for DC

Table 6.82.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n71A-n78A | DC_66A_n71A DC_66A_ n78A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.82.3	Co-existence studies

Based on co-existence studies, IMD4 and IMD5 generated by dual uplink of 66_n71 may fall into own Rx of band n78.

### 6.82.4	∆TIB and ∆RIB values

For DC_66_n71-n78, the TIB,c and RIB,c values are derived from DC_66-71_n78 and are given in the tables below.

Table 6.82.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n71-n78 | 66 | 0.6 |
|  | n71 | 0.6 |
|  | n78 | 0.8 |

Table 6.82.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n71-n78 | 66 | 0.2 |
|  | n71 | 0.2 |
|  | n78 | 0.5 |

### 6.82.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD4 and IMD5 produced by Band 66 and NR band n71 that impact the reference sensitivity of NR band n78.

The required MSD levels and test configuration are shown in the following Table. IMD4 as in DC_48-66_n71 and IMD5 as in DC_3_n28-n78.

Table 6.82.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n71A-n78A | n71 | IMD4 | ¦3*fB71 + fn66¦ | 665.5 | 5 | 25 | 619.5 | 5 | N/A |
|  | n78 |  |  | 3709 | 5 | 25 | 3709 | 10 | 13.0 |
|  | 66 |  |  | 1712.5 | 5 | 25 | 2112.5 | 5 | N/A |
|  | n71 | IMD5 | ¦2*fn71 -3*fB66¦ | 695 | 5 | 25 | 649 | 5 | N/A |
|  | n78 |  |  | 3755 | 10 | 50 | 3755 | 10 | 4.5 |
|  | 66 |  |  | 1715 | 5 | 25 | 2115 | 5 | N/A |

## 6.83	DC_2_n38-n71

### 6.83.1	Operating bands for DC

Table 6.83.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n38-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.83.2	Channel bandwidths per operating band for DC

Table 6.83.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n38A-n71A | DC_2A_n38A DC_2A_ n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.83.3	Co-existence studies

Based on co-existence studies, there are no IMD generated by dual uplink of 2_n38 into own Rx of band n71.

Based on co-existence studies, IMD2 generated by dual uplink of 2_n71 may fall into own Rx of band n38.

### 6.83.4	∆TIB and ∆RIB values

For DC_2_n38-n71, the TIB,c and RIB,c values are derived from DC_2_n41-n71 and are given in the tables below.

Table 6.83.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n38-n71 | 2 | 0.5 |
|  | n38 | 0.5 |
|  | n71 | 0.3 |

Table 6.83.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n38-n71 | 2 | 0 |
|  | n38 | 0 |
|  | n71 | 0 |

### 6.83.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 2 and NR band n71 that impact the reference sensitivity of NR band n38.

The required MSD levels and test configuration are shown in the following Table. IMD2 as in DC_2_n41-n71.

Table 6.83.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n38A-n71A | 2 | IMD2 | ¦fB2 +fn71¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n38 |  |  | 2586 | 5 | 25 | 2586 | 5 | 29.2 |
|  | n71 |  |  | 686 | 5 | 25 | 640 | 5 | N/A |

## 6.84	DC_66_n38-n71

### 6.84.1	Operating bands for DC

Table 6.84.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n38-n71 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.84.2	Channel bandwidths per operating band for DC

Table 6.84.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n38A-n71A | DC_66A_n38A DC_66A_ n71A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.84.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

### 6.84.4	∆TIB and ∆RIB values

For DC_66_n38-n71, the TIB,c and RIB,c values are derived from DC_66-71_n38 and are given in the tables below.

Table 6.84.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n38-n71 | 66 | 0.5 |
|  | n38 | 0.8 |
|  | n71 | 0.5 |

Table 6.84.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n38-n71 | 66 | 0.5 |
|  | n38 | 0.5 |
|  | n71 | 0.5 |

### 6.84.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

## 6.85	DC_71_n38-n66

### 6.85.1	Operating bands for DC

Table 6.85.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n38-n66 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.85.2	Channel bandwidths per operating band for DC

Table 6.85.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_71A_n38A-n66A | DC_71A_n38A DC_71A_ n66A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 6.85.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

### 6.85.4	∆TIB and ∆RIB values

For DC_71_n38-n66, the TIB,c and RIB,c values are derived from DC_66-71_n38 and are given in the tables below.

Table 6.85.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n38-n66 | 71 | 0.5 |
|  | n38 | 0.8 |
|  | n66 | 0.5 |

Table 6.85.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_71_n38-n66 | 71 | 0.5 |
|  | n38 | 0.5 |
|  | n66 | 0.5 |

### 6.85.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

## 6.86	DC_2_n2-n38

### 6.86.1	Operating bands for DC

Table 6.86.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n2-n38 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

### 6.86.2	Channel bandwidths per operating band for DC

Table 6.86.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n2A-n38A | DC_2A_ n38A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 6.86.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

### 6.86.4	∆TIB and ∆RIB values

For DC_2_n2-n38, the TIB,c and RIB,c values are derived from DC_2_n38 and are given in the tables below.

Table 6.86.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n2-n38 | 2 | 0.5 |
|  | n2 | 0.5 |
|  | n38 | 0.9 |

Table 6.86.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n2-n38 | 2 | 0 |
|  | n2 | 0 |
|  | n38 | 0 |

### 6.86.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

## 6.87	DC_66_n2-n38

### 6.87.1	Operating bands for DC

Table 6.87.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n2-n38 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

### 6.87.2	Channel bandwidths per operating band for DC

Table 6.87.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n2A-n38A | DC_66A_n2A DC_66A_ n38A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 6.87.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

### 6.87.4	∆TIB and ∆RIB values

For DC_66_n2-n38, the TIB,c and RIB,c values are derived from DC_2-66_n38 and are given in the tables below.

Table 6.87.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n2-n38 | 66 | 0.5 |
|  | n2 | 0.5 |
|  | n38 | 0.9 |

Table 6.87.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n2-n38 | 66 | 0.5 |
|  | n2 | 0.3 |
|  | n38 | 0.5 |

### 6.87.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

## 6.88	DC_2_n2-n41

### 6.88.1	Operating bands for DC

Table 6.88.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n2-n41 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.88.2	Channel bandwidths per operating band for DC

Table 6.88.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n2A-n41A | DC_2A_ n41A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 6.88.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

### 6.88.4	∆TIB and ∆RIB values

For DC_2_n2-n41, the TIB,c and RIB,c values are derived from DC_2_n38 and are given in the tables below.

Table 6.88.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n2-n41 | 2 | 0.5 |
|  | n2 | 0.5 |
|  | n41 | 0.5 |

Table 6.88.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n2-n41 | 2 | 0 |
|  | n2 | 0 |
|  | n41 | 0 |

### 6.88.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

## 6.89	DC_71_n2-n41

### 6.89.1	Operating bands for DC

Table 6.89.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n2-n41 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.89.2	Channel bandwidths per operating band for DC

Table 6.89.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_71A_n2A-n41A | DC_71A_n2A DC_71A_ n41A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 6.89.3	Co-existence studies

Based on co-existence studies, IMD2 generated by dual uplink of 71_n2 may fall into own Rx of band n41.

Based on co-existence studies, IMD2 generated by dual uplink of 71_n41 may fall into own Rx of band n2.

### 6.89.4	∆TIB and ∆RIB values

For DC_71_n2-n41, the TIB,c and RIB,c values are derived from DC_2_n41-n71 and are given in the tables below.

Table 6.89.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n2-n41 | 71 | 0.3 |
|  | n2 | 0.5 |
|  | n41 | 0.5 |

Table 6.89.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_71_n2-n41 | 71 | 0 |
|  | n2 | 0 |
|  | n41 | 0 |

### 6.89.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 71 and NR band n2 that impact the reference sensitivity of NR band n41. The required MSD levels and test configuration are shown in the following Table and with values as in DC_2_n41-n71.

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 71 and NR band n41 that impact the reference sensitivity of NR band n2. The required MSD levels and test configuration are shown in the following Table and with values as in DC_2-71_n38.

Table 6.89.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_71A_n2A-n41A | n2 | IMD2 | ¦fB71 +fn2¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n41 |  |  | 2586 | 5 | 25 | 2586 | 10 | 29.2 |
|  | 71 |  |  | 686 | 5 | 50 | 640 | 5 | N/A |
|  | n2 | IMD2 | ¦ fn41 - fB71¦ | 1862 | 5 | 25 | 1942 | 5 | 26 |
|  | n41 |  |  | 2610 | 5 | 25 | 2610 | 10 | N/A |
|  | 71 |  |  | 668 | 5 | 25 | 622 | 5 | N/A |

## 6.90	DC_7_n2-n78

### 6.90.1	Operating bands for DC

Table 6.90.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n2-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.90.2	Channel bandwidths per operating band for DC

Table 6.90.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A_n2A-n78A | DC_7A_n2A DC_7A_ n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.90.3	Co-existence studies

Based on co-existence studies, IMD5 generated by dual uplink of 7_n2 may fall into own Rx of band n78.

Based on co-existence studies, IMD4 generated by dual uplink of 7_n78 may fall into own Rx of band n2.

### 6.90.4	∆TIB and ∆RIB values

For DC_7_n2-n78, the TIB,c and RIB,c values are derived from DC_2_n7-n78 and are given in the tables below.

Table 6.90.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n2-n78 | 7 | 0.5 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 6.90.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n2-n78 | 7 | 0.5 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

### 6.90.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD4 generated by dual uplink of 7_n78 may fall into own Rx of band n2. MSD of DC_2A-7A_n78A is reused

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 generated by dual uplink of 7_n2 may fall into own Rx of band n78. MSD of DC_2A_n7A-n78A is reused.

The required MSD levels and test configuration are shown in the following Table.

Table 6.90.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n2A-n78A | B7 | IMD4 | ¦2*fB7 -2*fn78¦ | 2550 | 5 | 25 | 2685 | 5 | N/A |
|  | n2 |  |  | 1870 | 5 | 25 | 1950 | 5 | 8.6 |
|  | n78 |  |  | 3525 | 10 | 50 | 3525 | 10 | N/A |
|  | B7 | IMD5 | ¦2*fn2 -3*fB7¦ | 2525 | 5 | 25 | 2645 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n78 |  |  | 3775 | 10 | 50 | 3775 | 10 | 4.2 |

## 6.91	DC_2_n66-n71

### 6.91.1	Operating bands for DC

Table 6.91.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n66-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.91.2	Channel bandwidths per operating band for DC

Table 6.91.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n66A-n71A | DC_2A_n66A DC_2A_ n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.91.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

### 6.91.4	∆TIB and ∆RIB values

For DC_2_n66-n71, the TIB,c and RIB,c values are derived from DC_2-66_n71 and are given in the tables below.

Table 6.91.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n66-n71 | 2 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.3 |

Table 6.91.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n66-n71 | 2 | 0.3 |
|  | n66 | 0.3 |
|  | n71 | 0 |

### 6.91.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

## 6.92	DC_7_n66-n71

### 6.92.1	Operating bands for DC

Table 6.92.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n66-n71 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.92.2	Channel bandwidths per operating band for DC

Table 6.92.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A_n66A-n71A | DC_7A_n66A DC_7A_ n71A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.92.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

### 6.92.4	∆TIB and ∆RIB values

For DC_7_n66-n71, the TIB,c and RIB,c values are derived from DC_7-66_n71 and are given in the tables below.

Table 6.92.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n66-n71 | 7 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.5 |

Table 6.92.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n66-n71 | 7 | 0.5 |
|  | n66 | 0.5 |
|  | n71 | 0.1 |

### 6.92.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

## 6.93	DC_66_n66-n71

### 6.93.1	Operating bands for DC

Table 6.93.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n66-n71 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.93.2	Channel bandwidths per operating band for DC

Table 6.93.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n66A-n71A | DC_66A_n66A DC_66A_ n71A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.93.3	Co-existence studies

Based on co-existence studies, IMD4 generated by dual uplink of 66_n71 may fall into own Rx of band n66.

### 6.93.4	∆TIB and ∆RIB values

For DC_66_n66-n71, the TIB,c and RIB,c values are derived from DC_66_n71 and are given in the tables below.

Table 6.93.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n66-n71 | 66 | 0.3 |
|  | n66 | 0.3 |
|  | n71 | 0.3 |

Table 6.93.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n66-n71 | 66 | 0 |
|  | n66 | 0 |
|  | n71 | 0 |

### 6.93.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD4 generated by dual uplink of 66_n71 may fall into own Rx of band n66. MSD of DC_66A_n71A is reused.

The required MSD levels and test configuration are shown in the following Table.

Table 6.93.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n66A-n71A | B66 | IMD4 | ¦2*fn71 -2*fB66¦ | 1752 | 5 | 25 | 2152 | 5 | N/A |
|  | n66 |  |  | 1718 | 5 | 25 | 2118 | 5 | 5.0 |
|  | n71 |  |  | 693 | 5 | 25 | 647 | 5 | N/A |

## 6.94	DC_2_n2-n71

### 6.94.1	Operating bands for DC

Table 6.94.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n2-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.94.2	Channel bandwidths per operating band for DC

Table 6.94.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n2A-n71A | DC_2A_ n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.94.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

### 6.94.4	∆TIB and ∆RIB values

For DC_2_n2-n71, the TIB,c and RIB,c values are derived from DC_2_n71 and are given in the tables below.

Table 6.94.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n2-n71 | 2 | 0.3 |
|  | n2 | 0.3 |
|  | n71 | 0.3 |

Table 6.94.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n2-n71 | 2 | 0 |
|  | n2 | 0 |
|  | n71 | 0 |

### 6.94.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

## 6.95	DC_7_n2-n71

### 6.95.1	Operating bands for DC

Table 6.95.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n2-n71 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.95.2	Channel bandwidths per operating band for DC

Table 6.95.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A_n2A-n71A | DC_7A_n2A DC_7A_ n71A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.95.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are IMD2 and IMD5 generated by dual uplink of 7_n2 that may fall into own Rx of band n71.

### 6.95.4	∆TIB and ∆RIB values

For DC_7_n2-n71, the TIB,c and RIB,c values are derived from DC_2-66_n71 and are given in the tables below.

Table 6.95.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n2-n71 | 7 | 0.5 |
|  | n2 | 0.5 |
|  | n71 | 0.3 |

Table 6.95.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n2-n71 | 7 | 0.3 |
|  | n2 | 0.3 |
|  | n71 | 0 |

### 6.95.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are IMD2 and IMD5 generated by dual uplink of 7_n2 that may fall into own Rx of band n71. MSD IMD2 of DC_2A_n41A-n71A and MSD IMD5 of CA_n1A-n7A-n28A are reused.

The required MSD levels and test configuration are shown in the following Table.

Table 6.95.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n2A-n71A | B7 | IMD2 | ¦fB7 -fn2¦ | 2530 | 5 | 25 | 2530 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n71 |  |  | 676 | 5 | 25 | 630 | 5 | 28.7 |
|  | B7 | IMD5 | ¦2*f B7 -3*fn2¦ | 2530 | 5 | 25 | 2530 | 5 | N/A |
|  | n2 |  |  | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n71 |  |  | 686 | 5 | 25 | 640 | 5 | 4.5 |

## 6.96	DC_66_n2-n71

### 6.96.1	Operating bands for DC

Table 6.96.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n2-n71 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.96.2	Channel bandwidths per operating band for DC

Table 6.96.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n2A-n71A | DC_66A_n2A DC_66A_ n71A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.96.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

### 6.96.4	∆TIB and ∆RIB values

For DC_66_n2-n71, the TIB,c and RIB,c values are derived from DC_2-66_n71 and are given in the tables below.

Table 6.96.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n2-n71 | 66 | 0.5 |
|  | n2 | 0.5 |
|  | n71 | 0.3 |

Table 6.96.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n2-n71 | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n71 | 0 |

### 6.96.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

## 6.97	DC_2_n2-n66

### 6.97.1	Operating bands for DC

Table 6.97.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n2-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.97.2	Channel bandwidths per operating band for DC

Table 6.97.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n2A-n66A | DC_2A_ n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 6.97.3	Co-existence studies

Based on co-existence studies, IMD3 and IMD5 generated by dual uplink of 2_n66 may fall into own Rx of band n2.

### 6.97.4	∆TIB and ∆RIB values

For DC_2_n2-n66, the TIB,c and RIB,c values are derived from DC_2_n66 and are given in the tables below.

Table 6.97.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n2-n66 | 2 | 0.5 |
|  | n2 | 0.5 |
|  | n66 | 0.5 |

Table 6.97.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n2-n66 | 2 | 0.3 |
|  | n2 | 0.3 |
|  | n66 | 0.3 |

### 6.97.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD3 and IMD5 generated by dual uplink of 2_n66 may fall into own Rx of band n2.

For IMD3 MSD of DC_2A_n66A is reused.

IMD5 from dual uplink of 2_n66 can only fall into own Rx of band n2 using the band edge frequencies 1780 (n66 UL) and 1850 (B2 UL). Therefore, test points for MSD for IMD5 cannot be defined.

The required MSD levels and test configuration are shown in the following Table.

Table 6.97.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n2A-n66A | B2 | IMD3 | ¦2*fB2 -fn66¦ | 1875 | 5 | 25 | 1955 | 5 | N/A |
|  | n2 |  |  | 1895 | 5 | 25 | 1975 | 5 | 20 |
|  | n66 |  |  | 1775 | 5 | 25 | 2175 | 5 | N/A |

## 6.98	DC_7_n2-n66

### 6.98.1	Operating bands for DC

Table 6.98.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n2-n66 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.98.2	Channel bandwidths per operating band for DC

Table 6.98.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A_n2A-n66A | DC_7A_n2A DC_7A_ n66A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 6.98.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

### 6.98.4	∆TIB and ∆RIB values

For DC_7_n2-n66, the TIB,c and RIB,c values are derived from DC_2-7_n66 and are given in the tables below.

Table 6.98.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n2-n66 | 7 | 0.5 |
|  | n2 | 0.5 |
|  | n66 | 0.5 |

Table 6.98.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n2-n66 | 7 | 0.5 |
|  | n2 | 0.3 |
|  | n66 | 0.5 |

### 6.98.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

## 6.99	DC_71_n2-n66

### 6.99.1	Operating bands for DC

Table 6.99.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_71_n2-n66 | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.99.2	Channel bandwidths per operating band for DC

Table 6.99.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_71A_n2A-n66A | DC_71A_n2A DC_71A_ n66A | 71 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 6.99.3	Co-existence studies

Based on Table 5.3-1 in TR 37.717-11-21 [1], there are no co-existence issues.

### 6.99.4	∆TIB and ∆RIB values

For DC_71_n2-n66, the TIB,c and RIB,c values are derived from DC_2-66_n71 and are given in the tables below.

Table 6.99.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_71_n2-n66 | 71 | 0.3 |
|  | n2 | 0.5 |
|  | n66 | 0.5 |

Table 6.99.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_71_n2-n66 | 71 | 0 |
|  | n2 | 0.3 |
|  | n66 | 0.3 |

### 6.99.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21 [1], there is no need to define MSD.

## 6.100	DC_21_n28-n79

### 6.100.1	Operating bands for DC

The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783-793 MHz for the DL because only a certain operator uses band 21. This restriction is mentioned in TP for DC_21_n28 (R4-2100352).

Table 6.100.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21_n28-n79 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.100.2	Configuration for DC

Table 6.100.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A_n28A-n79A | DC_21A_n28A DC_21A_n79A | 21A | CA_n28A-n79A |

### 6.100.3	Co-existence studies

Based on the co-existence studies of DC_21_n28 and DC_21_n79, own Rx impact of the 3rd band is the followings.

-	4th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n79.

-	5th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n79.

-	5th order IMD products generated by DC_21_n79 uplink may fall into own Rx of band n28.

### 6.100.4	∆TIB and ∆RIB values

For DC_21_n28-n79, the TIB,c and RIB,c values are reused from DC_21-28_n79, and are given in the tables below.

Table 6.100.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21_n28-n79 | 21 | 0.4 |
|  | n28 | 0.3 |
|  | n79 | 0 |

Table 6.100.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_21_n28-n79 | 21 | 0 |
|  | n28 | 0 |
|  | n79 | 0 |

### 6.100.5	MSD

-	4th and 5th order IMD products generated by DC_21_n28 uplink may fall into own Rx of band n79.

- The MSD values are shown in the following table. These values are the average of the analysis results of the two companies. [1][2]

-	5th order IMD products generated by DC_21_n79 uplink may fall into own Rx of band n28.

- The MSD values are shown in the following table. These values are the average of the analysis results of the two companies. [1][2]

Table 6.100.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A_n28A-n79A | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n28 | 735.5 | 5 | 25 | 790.5 | 2.8 | IMD5 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
|  | 21 | 1460.4 | 5 | 25 | 1508.4 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | [6.3] | IMD44 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.101	DC_8_n28-n78

### 6.101.1	 Operating bands for DC

Table 6.101.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A_n28A-n78A | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz |  | 748 MHz | 758 MHz |  | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.101.2 	Configuration for DC

Table 6.101.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A_n28A-n78A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.101.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_8A_n28A and DC_8A_n78A are shown below.

Table 6.101.3-1: The harmonic and IMD products caused by UL DC_8A_n28A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 212 | 132 | 1583 | 1663 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1012 | 1127 | 491 | 616 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2463 | 2578 | 2286 | 2411 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1892 | 2042 | 1194 | 1364 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3343 | 3493 | 2989 | 3159 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 264 | 424 | 3166 | 3326 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2112 | 1897 | 2957 | 2772 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 484 | 279 | 1339 | 1144 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3692 | 3907 | 4223 | 4408 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3869 | 4074 | 4046 | 4241 |

Table 6.101.3-2: The harmonic and IMD products caused by UL DC_8A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385 | 2920 | 4180 | 4715 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2040 | 1470 | 5685 | 6720 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 | 5630 | 7480 | 8515 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1160 | 555 | 8985 | 10520 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5940 | 6545 | 10780 | 12315 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5840 | 4770 | 8360 | 9430 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14320 | 12285 | 360 | 280 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9640 | 8070 | 3855 | 4960 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 | 16115 | 6820 | 7460 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 | 13230 | 9240 | 10345 |

IMD4 and IMD5 caused by UL DC_8_n28 may fall into Rx band n78.

IMD4 caused by UL DC_8_n78 may fall into Rx band n28.

### 6.101.4	 ∆TIB and ∆RIB values

Table 6.101.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A_n28A-n78A | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 6.101.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8A_n28A-n78A | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 6.101.5	MSD

All MSD requirements for DC_8A_n28A-n78A can reuse the requriements of DC_8A_n28A-n77A. The MSD exceptions are shown below.

Table 6.101.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A_n28A-n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n78 | 3455 | 10 | 50 | 3455 | 10.3 | IMD4 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 11.6 | IMD4 |
|  | n78 | 3495 | 10 | 50 | 3495 | N/A | N/A |

## 6.102	DC_2_n2-n78

### 6.102.1	Operating bands for DC

Table 6.102.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n2-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.102.2	Channel bandwidths per operating band for DC

Table 6.102.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A_n2A-n78A | DC_2A_ n78A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.102.3	Co-existence studies

Based on co-existence studies, IMD2, IMD4 and IMD5 generated by dual uplink of 2_n78 may fall into own Rx of band n2.

### 6.102.4	∆TIB and ∆RIB values

For DC_2_n2-n78, the TIB,c and RIB,c values are derived from DC_2_n78 and are given in the tables below.

Table 6.102.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n2-n78 | 2 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 6.102.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n2-n78 | 2 | 0.2 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

### 6.102.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD2, IMD3 and IMD5 generated by dual uplink of 2_n78 may fall into own Rx of band n2. MSD of DC_2A_n78A is reused.

The required MSD levels and test configuration are shown in the following Table.

Table 6.102.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n2A-n78A | B2 | IMD23 | ¦2*fn78 -fB2¦ | 1852.5 | 5 | 25 | 1932.5 | 5 | N/A |
|  | n2 |  |  | 1862.5 | 5 | 25 | 1942.5 | 5 | 26 |
|  | n78 |  |  | 3795 | 10 | 50 | 3795 | 10 | N/A |
| DC_2A_n2A-n78A | B2 | IMD43 | ¦3*fB2 -fn78¦ | 1885 | 5 | 25 | 1965 | 5 | N/A |
|  | n2 |  |  | 1865 | 5 | 25 | 1945 | 5 | 8.0 |
|  | n78 |  |  | 3710 | 10 | 50 | 3710 | 10 | N/A |
| NOTE 3: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

## 6.103	DC_66_n2-n66

### 6.103.1	Operating bands for DC

Table 6.103.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n2-n66 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.103.2	Channel bandwidths per operating band for DC

Table 6.103.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n2A-n66A | DC_66A_ n2A | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### Co-existence studies

Based on co-existence studies

- IMD5 generated by dual uplink of 66_n2 may fall into own Rx of band n66

- IMD3 generated by dual uplink of 66_n2 may fall into own Rx of band n2.

### 6.103.4	∆TIB and ∆RIB values

For DC_66_n2-n66, the TIB,c and RIB,c values are derived from DC_66_n66 and are given in the tables below.

Table 6.103.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n2-n66 | 66 | 0.5 |
|  | n2 | 0.5 |
|  | n66 | 0.5 |

Table 6.103.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n2-n66 | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n66 | 0.3 |

### 6.103.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD5 generated by dual uplink of 66_n2 that may fall into own Rx of band n66 and IMD3 generated bv dual uplink of 66_n2 that may fall into own Rx of band n2. MSD of DC_2A_n66A is reused.

The required MSD levels and test configuration are shown in the following Table.

Table 6.103.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n2A-n66A | B66 | IMD3 | ¦2*fn2 -fB66¦ | 1775 | 5 | 25 | 2175 | 5 | N/A |
|  | n2 |  |  | 1855 | 5 | 25 | 1935 | 5 | 20 |
|  | n66 |  |  | 1720 | 5 | 25 | 2120 | 5 | N/A |
|  | B66 | IMD5 | ¦2*fB66 -3*fn2¦ | 1720 | 5 | 25 | 2120 | 5 | N/A |
|  | n2 |  |  | 1870 | 5 | 25 | 1950 | 5 | N/A |
|  | n66 |  |  | 1770 | 5 | 25 | 2170 | 5 | 4.0 |

## 6.104	DC_39_n40-n258

### 6.104.1		Operating bands for DC

Table 6.104.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_39_n40-n258 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n257 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.104.2		Configuration for DC

Table 6.104.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_39A_n40A-n258A | DC_39A_n40A DC_39A_n258A |

### 6.104.3		Co-existence studies

- For combination of DC_39_n40, the co-existence studies have been covered in TR37.716-11-11, where band 39 and band n40 are synchronized operaiton.

- For combination of DC_39_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

### 6.104.4	∆TIB and ∆RIB values

For the DC conbination of DC_39_n40-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 39 and n40 are the same with the values of the constiture DC_39_n40 defined in TS38.101-3.

### 6.104.5	MSD

No additional MSD requirement is needed.

## 6.105	DC_39_n41-n258

### 6.105.1		Operating bands for DC

Table 6.105.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_39_n41-n258 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.105.2		Configuration for DC

Table 6.105.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_39A_n41A-n258A DC_39A_n41C-n258A | DC_39A_n41A DC_39A_n258A |

### 6.105.3		Co-existence studies

- For combination of DC_39_n41, the co-existence studies have been covered in TR37.716-11-11, where band 39 and band n41 are synchronized operaiton.

- For combination of DC_39_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

### 6.105.4	∆TIB and ∆RIB values

For the DC conbination of DC_39_n41-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 39 and n41 are the same with the values of the constiture DC_39_n41 defined in TS38.101-3.

### 6.105.5	MSD

No additional MSD requirement is needed.

## 6.106	DC_39_n79-n258

### 6.106.1		Operating bands for DC

Table 6.106.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_39_n79-n258 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.106.2		Configuration for DC

Table 6.106.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_39A_n79A-n258A DC_39A_n79C-n258A | DC_39A_n79A DC_39A_n258A |

### 6.106.3		Co-existence studies

- For combination of DC_39_n79, the co-existence studies have been covered in TR37.716-11-11, where no harmonic issues for this combination.

- For combination of DC_39_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

### 6.106.4	∆TIB and ∆RIB values

For the DC combination of DC_39_n79-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 39 and n79 are the same with the values of the constitute DC_39_n79 defined in TS38.101-3.

### 6.106.5	MSD

No additional MSD requirement is needed.

## 6.107	DC_13_n5-n77

### 6.107.1	Operating bands for DC

Table 6.107.1-1: DC band combination of one LTE + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n5-n77 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.107.2	Configuration for DC

Table 6.107.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_13A_n5A-n77A | DC_13A_ n77A | 13A | CA_n5A-n77A |

### 6.107.3	Co-existence studies

According to the studies in 37.717-11-11, the Rx impacts are identified as below,

- For UL DC_13A_n77A, IMD5 of the 2UL may fall into band n5 Rx

Thus additional MSD should be considered to mitigate the impact of the interference for DC_13A_n5A-n77A combination.

### 6.107.4	∆TIB and ∆RIB values

For DC_13_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.107.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n5-n77 | 13 | 0.5 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 6.107.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n5-n77 | 13 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

### 6.107.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 produced by Band 13 and NR band n77 that impact the reference sensitivity of NR band n5.

The required MSD level and test configuration are are specified in Table 6.107.5-1 for this dual connectivity configuration.

Table 6.107.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_13A_n5A-n77A11 | 13 | IMD5 | ¦4*fB13 -fn77¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n77 |  |  | 4013 | 10 | 50 | 4013 | 10 |  |
|  | n5 |  |  | 840 | 5 | 25 | 885 | 5 | 4.5 |
| NOTE 11:The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |

## 6.108 DC_28_n7-n258

### 6.108.1	Operating bands for DC

Table 6.108.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n7-n258 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.108.2	Channel bandwidths per operating band for DC

Table 6.108.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_28A_n7A-n258A DC_28A_n7A-n258B DC_28A_n7A-n258C DC_28A_n7A-n258D DC_28A_n7A-n258E DC_28A_n7A-n258F DC_28A_n7A-n258G DC_28A_n7A-n258H DC_28A_n7A-n258I DC_28A_n7A-n258J DC_28A_n7A-n258K DC_28A_n7A-n258L DC_28A_n7A-n258M DC_28A_n7B-n258A DC_28A_n7B-n258B DC_28A_n7B-n258C DC_28A_n7B-n258D DC_28A_n7B-n258E DC_28A_n7B-n258F DC_28A_n7B-n258G DC_28A_n7B-n258H DC_28A_n7B-n258I DC_28A_n7B-n258J DC_28A_n7B-n258K DC_28A_n7B-n258L DC_28A_n7B-n258M | DC_28A_n7A DC_28A_n258A DC_28A_n258G DC_28A_n258H DC_28A_n258I | 28 | - | 880 |
|  |  | n7 | - |  |
|  |  | n258 | - |  |

### 6.108.3	Co-existence studies

Based on co-existence studies of DC_28_n7 and DC_28_n258 specified in TR 37.717-11-21, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 6.108.4	∆TIB and ∆RIB values

For DC_28_n7-n258, the TIB,c and RIB values are derived from CA_n7-n28 and are given in the tables below.

Table 6.108.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n7-n258 | 28 | 0.3 |
|  | n7 | 0.3 |
|  | n258 | 0 |

Table 6.108.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n7-n258 | 28 | 0 |
|  | n7 | 0 |
|  | n258 | 0 |

### 6.108.5	MSD

There is no additional MSD issue for DC_28_n7-n258.

## 6.109	DC_7_n25-n66

### 6.109.1	Operating bands for DC

Table 6.109.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n25-n66 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.109.2	Channel bandwidths per operating band for DC

Table 6.109.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A_n25A-n66A | DC_7A_n25A DC_7A_n66A | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n25 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| DC_7A-7A_n25A-n66A | DC_7A_n25A DC_7A_n66A | 7 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 |
|  |  | n25 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| DC_7C_n25A-n66A | DC_7A_n25A DC_7A_n66A | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 |
|  |  | n25 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

### 6.109.3	Co-existence studies

Based on co-existence studies there are no IMD issues into own Rx of bands.

### 6.109.4	∆TIB and ∆RIB values

For DC_7_n25-n66, the TIB,c and RIB,c values are derived from DC_2-7_n66 and are given in the tables below.

Table 6.109.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n25-n66 | 7 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 6.109.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n25-n66 | 7 | 0.5 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

### 6.109.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is no IMD issues.

## 6.110	DC_66_n25-n66

### 6.110.1	Operating bands for DC

Table 6.110.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n25-n66 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.110.2	Channel bandwidths per operating band for DC

Table 6.110.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_66A_n25A-n66A | DC_66A_n25A DC_66A_n66A2 | 66 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n25 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
| NOTE 2: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.110.3	Co-existence studies

Based on co-existence studies, IMD3 and IMD5 generated by dual uplink of 66_n25 may fall into own Rx of band n66.

### 6.110.4	∆TIB and ∆RIB values

For DC_66_n25-n66, the TIB,c and RIB,c values are derived from DC_2-66_n66 and are given in the tables below.

Table 6.110.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n25-n66 | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 6.110.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n25-n66 | 66 | 0.3 |
|  | n25 | 0.3 |
|  | n66 | 0.3 |

### 6.110.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD3 and IMD5 produced by UL 66_n25 that impact DL n66. MSD value as in DC_66_n25.

Table 6.110.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n25A-n66A | 66 | 1712.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 | N/A | N/A |
|  | n66 | 1717.5 | 5 | 25 | 2117.5 | 23 | IMD3 |
|  | 66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n25 | 1873 | 5 | 25 | 1953 | N/A | N/A |
|  | n66 | 1719 | 5 | 25 | 2119 | 4 | IMD5 |

## 6.111	DC_30_n5-n77

### 6.111.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD3 produced by Band 30 and NR band n5 that impact the reference sensitivity of NR band n77. Also there are IMD3 & IMD5 produced by Band 30 and NR band n77 that impact the reference sensitivity of NR band n5.

The required MSD levels and test configuration are shown in the following Table.

Table 6.111.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_30A_n5A-n77A | B30 | IMD3 | ¦2*fB30 –fn5¦ | 2310 | 5 | 25 | 2355 | 5 | N/A |
|  | n5 |  |  | 834 | 5 | 25 | 879 | 5 |  |
|  | n77 |  |  | 3786 | 10 | 50 | 3786 | 10 | 16.1 |
|  | B30 | IMD3 | ¦2*fB30 -fn77¦ | 2310 | 5 | 25 | 2355 | 5 | N/A |
|  | n77 |  |  | 3741 | 10 | 50 | 3741 | 10 |  |
|  | n5 |  |  | 834 | 5 | 25 | 879 | 5 | 15.3 |
|  | B30 | IMD5 | ¦3*fB30 -2*fn77¦ | 2310 | 5 | 25 | 2355 | 5 | N/A |
|  | n77 |  |  | 3905 | 10 | 50 | 3905 | 10 |  |
|  | n5 |  |  | 835 | 5 | 25 | 880 | 5 | 4.4 |

## 6.112	DC_13_n25-n66

### 6.112.1	Operating bands for DC

Table 6.112.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n25-n66 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.112.2	Channel bandwidths per operating band for DC

Table 6.112.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A_n25A-n66A | DC_13A_n25A DC_13A_n66A | 13 | 15 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 90 |
|  |  | n25 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

### 6.112.3	Co-existence studies

Based on co-existence studies, IMD4 generated by dual uplink of 13_n25 may fall into own Rx of band n66.

Based on co-existence studies, IMD4 generated by dual uplink of 13_n66 may fall into own Rx of band n25.

### 6.112.4	∆TIB and ∆RIB values

For DC_13_n25-n66, the TIB,c and RIB,c values are derived from DC_2-13_n66 and are given in the tables below.

Table 6.112.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n25-n66 | 13 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 6.112.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n25-n66 | 13 | 0 |
|  | n25 | 0.3 |
|  | n66 | 0.3 |

### 6.112.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD4 produced by UL 13_n25 that impact DL n66. MSD value as in DC_13-66_n2.

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD4 produced by UL 13_n66 that impact DL n25. MSD value as in DC_2-13_n66.

Table 6.112.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n25A-n66A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | N/A | N/A |
|  | n66 | 1736 | 5 | 25 | 2156 | 7.2 | IMD4 |
| DC_13A_n25A-n66A | 13 | 780 | 5 | 25 | 749 | N/A | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | 6.2 | IMD4 |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | N/A |

## 6.113		DC_8_n39-n40

### 6.113.1	Operating bands for DC

Table 6.113.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n39-n40 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 6.113.2	Configuration for DC

Table 6.113.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_8A_n39A-n40A | DC_8A_n39A DC_8A_n40A | 8A | CA_n39A-n40A |

### 6.113.3	Co-existence studies

Co-existence analysis for DC_8_n39 UL in TR 37.716-11-11 shows no IMD produce impact to NR Band n40 DL.

Co-existence analysis for DC_8_n40 UL in TR TR 37.863-01-01 shows IMD5 produce impact to NR Band n39 DL.

### 6.113.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.113.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n39-n40 | 8 | 0.3 |
|  | n39 | 0.3 |
|  | n40 | 0.3 |

Table 6.113.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n39-n40 | 8 | 0 |
|  | n39 | 0.3 |
|  | n40 | 0.3 |

### 6.113.5	MSD

Based on co-existence studies, there is IMD5 products cause by DC_8_n40 will falls into band n39. However, simultaneous Rx/Tx is not supported for CA_n39-n40, which means synchronization between band n39 and n40. Therefore no additional MSD is needed to be defined

## 6.114	DC_3_n1-n8, DC_3-3_n1-n8

### 6.114.1	Operating bands for DC

Table 6.114.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n8 DC_3-3_n1-n8 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |

### 6.114.2	Configuration for DC

Table 6.114.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A_n1A-n8A DC_3A-3A_n1A-n8A | DC_3A_n1A DC_3A_n8A | 3A CA_3A-3A | CA_n1A-n8A |

### 6.114.3	Co-existence studies

Based on co-existence studies of DC_3A_n1A and DC_3A_n8A specified in TR 37.716-11-11, there is no additional  intermodulation impact for the additional band receiver.

### 6.114.4	∆TIB and ∆RIB values

For DC_3_n1-n8 and DC_3-3_n1-n8, the TIB,c and RIB,c values are given in the tables below.

Table 6.114.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n1-n8, DC_3-3_n1-n8 | 3 | 0.3 |
|  | n1 | 0.3 |
|  | n8 | 0.3 |

Table 6.114.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n1-n8, DC_3-3_n1-n8 | 3 | 0 |
|  | n1 | 0 |
|  | n8 | 0 |

### 6.114.5	MSD

No additional MSD requirement is needed.

## 6.115	DC_7_n1-n8, DC_7-7_n1-n8

### 6.115.1	Operating bands for DC

Table 6.115.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n1-n8 DC_7-7_n1-n8 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |

### 6.115.2	Configuration for DC

Table 6.115.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A_n1A-n8A DC_7A-7A_n1A-n8A | DC_7A_n1A DC_7A_n8A | 7A CA_7A-7A | CA_n1A-n8A |

### 6.115.3	Co-existence studies

Based on co-existence studies of DC_7A_n1A and DC_7A_n8A specified in TR 37.716-11-11:

- 5th order of IMD from band 7 and band n1 will fall into the own Rx frequency of Band n8. But in the studies for LTE DL CA_1A-7A-8A with UL CA_1A-7A in TS 36.714-00-02, it is mentioned that the IMD5 is fairly small and the overlapped region is quite small portion, which concluded that no MSD is needed.

### 6.115.4	∆TIB and ∆RIB values

For DC_7_n1-n8 and DC_7-7_n1-n8, the TIB,c and RIB,c values are given in the tables below.

Table 6.115.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n1-n8, DC_7-7_n1-n8 | 7 | 0.6 |
|  | n1 | 0.5 |
|  | n8 | 0.5 |

Table 6.115.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n1-n8, DC_7-7_n1-n8 | 7 | 0 |
|  | n1 | 0 |
|  | n8 | 0.2 |

### 6.115.5	MSD

No additional MSD requirement is needed.

## 6.116		DC_8_n39-n79

### 6.116.1	Operating bands for DC

Table 6.116.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n39-n79 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.116.2	Configuration for DC

Table 6.116.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_8A_n39A-n79A | DC_8A_n39A DC_8A_n79A | 8A | CA_n39A-n79A |

### 6.116.3	Co-existence studies

Co-existence analysis for DC_8_n39 UL in TR 37.716-11-11 shows 3rd IMD and 4th IMD produce impact to NR Band n79 DL.

Co-existence analysis for DC_8_n79 UL in TR TR 37.863-01-01 shows 4th IMD produce impact to NR Band n39 DL.

### 6.116.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.116.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n39-n79 | 8 | 0.3 |
|  | n39 | 0.3 |
|  | n79 | 0 |

Table 6.116.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n39-n79 | 8 | 0 |
|  | n39 | 0 |
|  | n79 | 0 |

### 6.116.5	MSD

Based on co-existence studies additional MSD is needed to be defined, shown in table 6.116.5-1.

Table 6.116.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n39A-n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |
|  | n79 | 4680 | 40 | 216 | 4680 | 15.9 | IMD3 |
| DC_8A_n39A-n79A | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |
|  | n79 | 4560 | 40 | 216 | 4560 | 12.1 | IMD4 |
| DC_8A_n39A-n79A | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n39 | 1907.5 | 10 | 50 | 1907.5 | 13.8 | IMD4 |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | N/A |

## 6.117	DC_5_n66-n77

### 6.117.1	Operating bands for DC

Table 6.117.1-1: DC band combination of one LTE + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n66-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.117.2	Configuration for DC

Table 6.117.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A_n66A-n77A DC_5A_n66A-n77C | DC_5A_n66A DC_5A_ n77A | 5A | CA_n66A-n77A |

### 6.117.3	Co-existence studies

According to the studies in in 37.717-21-11, the Rx impacts are identified as below,

- For UL DC_5A_n66A, IMD3, IMD4 and IMD5 may fall into own Rx of band n77

- For UL DC_5A_n77A, IMD3 of the 2UL may fall into own Rx of band n66.

Thus additional MSD should be considered to mitigate the impact of the interference for DC_5A_n66A-n77A combination.

### 6.117.4	∆TIB and ∆RIB values

For DC_5_n66-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.117.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n66-n77 | 5 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.117.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n66-n77 | 5 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |

### 6.117.5	MSD

Based on co-existence studies, additional MSD is specified in Table 6.117.5-1 for this dual connectivity configuration.

Table 6.117.5-1: MSD test points for SCell due to uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n66A-n77A | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | n66 | 1742 | 5 | 25 | 2142 | 13.2 | IMD3 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 5 | 845 | 5 | 25 | 890 | N/A | N/A |
|  | n66 | 1785 | 5 | 25 | 2185 | N/A | N/A |
|  | n77 | 3475 | 10 | 50 | 3475 | 16.1 | IMD3 |
|  | 5 | 827 | 5 | 25 | 872 | N/A | N/A |
|  | n66 | 1713 | 5 | 25 | 2113 | N/A | N/A |
|  | n77 | 4194 | 10 | 50 | 4194 | 8.2 | IMD411 |
|  | 5 | 827 | 5 | 25 | 872 | N/A | N/A |
|  | n66 | 1713 | 5 | 25 | 2113 | N/A | N/A |
|  | n77 | 3485 | 10 | 50 | 3485 | 3.3 | IMD5 |
| NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction |  |  |  |  |  |  |  |

## 6.118	DC_5_n5-n77

### 6.118.1	Operating bands for DC

Table 6.118.1-1: DC band combination of one LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n5-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.118.2	Configuration for DC

Table 6.118.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A_n5A-n77A | DC_5A_ n77A | 5A | CA_n5A-n77A |

### 6.118.3	Co-existence studies

According to lower order coexistence study, the Rx impacts are identified as below,

For UL DC_5A_n77A, IMD4 and IMD5 of 2UL may fall into band n5 Rx frequency range

Thus additional MSD should be considered to mitigate the impact of the interference for DC_5A_n5A-n77A combination.

### 6.118.4	∆TIB and ∆RIB values

For DC_5_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.118.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n5-n77 | 5 | 0.6 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 6.118.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n5-n77 | 5 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

### 6.118.5	MSD

Based on co-existence studies, additional MSD is specified in Table 6.118.5-1 for this dual connectivity configuration.

Table 6.118.5-1: MSD test points for SCell due to uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC CA Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) |  | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n5A-n77A | 5 | 844 |  | 5 | 25 | 889 | N/A | N/A |
|  | n5 | 844 |  | 5 | 25 | 889 | 8.3 | IMD4 |
|  | n77 | 3421 |  | 10 | 50 | 3421 | N/A | N/A |
|  | 5 | 826.5 | 5 |  | 25 | 871.5 | N/A | N/A |
|  | n5 | 827 | 5 |  | 25 | 872 | 5.5 | IMD511 |
|  | n77 | 4178 | 10 |  | 50 | 4178 | N/A | N/A |
| NOTE 11:The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |

## 6.119	DC_5_n2-n77

### 6.119.1	Operating bands for DC

Table 6.119.1-1: DC band combination of one LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n2-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.119.2	Configuration for DC

Table 6.119.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A_n2A-n77A | DC_5A_ n77A | 5A | CA_n2A-n77A |

### 6.119.3	Co-existence studies

According to the studies in 37.717-21-11, the Rx impacts are identified as below

- For UL DC_5A_n77A, IMD3 of 2UL may fall into own Rx of band n2.

Thus additional MSD should be considered to mitigate the impact of the interference for DC_5A_n2A-n77A combination.

### 6.119.4	∆TIB and ∆RIB values

For DC_5_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.119.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n2-n77 | 5 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 6.119.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n2-n77 | 5 | 0.2 |
|  | n2 | 0.2 |
|  | n77 | 0.5 |

### 6.119.5	MSD

Based on co-existence studies, additional MSD is specified in Table 6.119.5-1 for this dual connectivity configuration.

Table 6.119.5-1: MSD test points for SCell due to uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_5A_n2A-n77A | n2 | 1907 | 5 | 25 | 1987 | 16.5 | IMD3 |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | N/A |
|  | n77 | 3680 | 5 | 25 | 3680 | N/A | N/A |

## 6.120	DC_2_n2-n77

### 6.120.1	Operating bands for DC

Table 6.120.1-1: DC band combination of one LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n2-n77 | 2 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n2 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.120.2	Configuration for DC

Table 6.120.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A_n2A-n77A | DC_2A_ n77A | 2A | CA_n2A-n77A |

### 6.120.3	     Co-existence studies

According to lower order coexistence study, the Rx impacts are identified as below,

- For UL DC_2A_n77A, IMD2, IMD4 and IMD5 of 2UL may fall into band n2 Rx frequency range

Thus additional MSD should be considered to mitigate the impact of the interference for DC_2A_n2A-n77A combination.

### 6.120.4	∆TIB and ∆RIB values

For DC_2_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.120.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n2-n77 | 2 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 6.120.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n2-n77 | 2 | 0.2 |
|  | n2 | 0.2 |
|  | n77 | 0.5 |

### 6.120.5	MSD

Based on co-existence studies, additional MSD is specified in Table 6.120.5-1 for this dual connectivity configuration.

Table 6.120.5-1: MSD test points for SCell due to uplink operation for EN-DC in NR FR1 (three bands)

|  | Operating band/ Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_2A_n2A-n77A | 2 | 1855 | 5 | 25 | 1935 | N/A | N/A |
|  | n2 | 1855 | 5 | 25 | 1935 | 26 | IMD2 |
|  |  |  |  |  |  | 28.75 |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1885 | 5 | 25 | 1965 | N/A | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 | 8.0 | IMD44 |
|  |  |  |  |  |  | 10.75 |  |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |

## 6.121	DC_3-3_n8-n78

### 6.121.1	Operating bands for DC

Table 6.121.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-3_n8-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.121.2	Configuration for DC

Table 6.121.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-3A_n8A-n78A5 | DC_3A_n8A DC_3A_n78A | 3A CA_3A-3A | CA_n8A-n78A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

### 6.121.3	Co-existence studies

Based on co-existence studies of DC_3A_n8A and DC_3A_n78A specified in TR 37.716-11-11, the impact on the third bands includes:

- 3rd and 5th order IMD from band 3 and band n8 will fall into the own Rx frequency of Band n78.

### 6.121.4	∆TIB and ∆RIB values

For DC_3-3_n8-n78, the TIB,c and RIB,c values are given in the tables below.

Table 6.121.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-3_n8-n78 | 3 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.121.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-3_n8-n78 | 3 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 6.121.5	MSD

Since the MSD can be covered by DC_3_n8-n78, no additional requirement is needed.

## 6.122	DC_7-7_n8-n78

### 6.122.1	Operating bands for DC

Table 6.122.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-7_n8-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.122.2	Configuration for DC

Table 6.122.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-7A_n8A-n78A5 | DC_7A_n8A DC_7A_n78A | 7A CA_7A-7A | CA_n8A-n78A |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

### 6.122.3	Co-existence studies

Based on co-existence studies of DC_7A_n8A and DC_7A_n78A specified in TR 37.716-11-11, the impact on the third bands includes:

- 2nd and 4th order IMD from band 7 and band n8 will fall into the own Rx frequency of Band n78.

- 2nd order IMD from band 7 and band n78 will fall into the own Rx frequency of Band n8.

### 6.122.4	∆TIB and ∆RIB values

For DC_7-7_n8-n78, the TIB,c and RIB,c values are given in the tables below.

Table 6.122.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-7_n8-n78 | 7 | 0.5 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.122.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-7_n8-n78 | 7 | 0 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 6.122.5	MSD

Since the MSD can be covered by DC_7_n8-n78, no additional requirement is needed.

## 6.123	DC_7_n66-n77

### 6.123.1	Operating bands for DC

Table 6.123.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n66-n77 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.123.2	Configuration for DC

Table 6.123.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | DC_7A_n66A DC_7A_n77A |

### 6.123.3	Co-existence studies

Based on co-existence studies,

- 3rd and 5th order IMD generated by dual uplink of 7_n66 may fall into own Rx of band n77

- 4th order IMD generated by dual uplink of 7_n77 may fall into own Rx of band n66

### 6.123.4	∆TIB and ∆RIB values

For DC_7_n66-n77, the TIB,c and RIB,c values are derived from DC_7_n66-n78 and are given in the tables below.

Table 6.123.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n66-n77 | 7 | 0.5 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 6.123.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n66-n77 | 7 | 0.5 |
|  | n66 | 0.5 |
|  | n77 | 0.5 |

### 6.123.5	MSD

As mentioned in 6.123.3, 3rd and 5th order IMD generated by dual uplink of 7_n66 may fall into own Rx of band n77, and 4th order IMD generated by dual uplink of 7_n77 may fall into own Rx of band n66. The MSD due to 3rd and 4rd order IMD can be derived from DC_7_n66-n78. The MSD due to 5th order IMD generated by DC_7_n66 can be derived from DC_2A_n7A-n78A.

Table 6.123.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2550 | 5 | 25 | 2685 | N/A | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 | 8.7 | IMD4 |
|  | n77 | 3625 | 10 | 50 | 3625 | N/A | N/A |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2542 | 5 | 25 | 2662 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3344 | 10 | 50 | 3344 | 16.0 | IMD3 |
| DC_7A_n66A-n77A DC_7A-7A_n66A-n77A DC_7C_n66A-n77A | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 4040 | 10 | 50 | 4040 | 4.2 | IMD5 |

## 6.124	DC_20_n8-n78

### 6.124.1	 Operating bands for DC

Table 6.124.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20A_n8A-n78A | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.124.2 	Configuration for DC

Table 6.124.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20A_n8A-n78A |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.124.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_20A_n78A and DC_20A_n8A are shown below.

Table 6.124.3-1: The harmonic and IMD products caused by UL DC_20A_n8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 18 | 83 | 1712 | 1777 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 749 | 844 | 898 | 998 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2544 | 2639 | 2592 | 2692 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1581 | 1706 | 1778 | 1913 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3376 | 3501 | 3472 | 3607 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 166 | 36 | 3424 | 3554 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2828 | 2658 | 2568 | 2413 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1081 | 916 | 826 | 666 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4352 | 4522 | 4208 | 4363 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4304 | 4469 | 4256 | 4416 |

Table 6.124.3-2: The harmonic and IMD products caused by UL DC_20A_n78A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3328 | 3448 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4160 | 4310 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2438 | 2968 | 4132 | 4662 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2136 | 1576 | 5738 | 6768 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4964 | 5524 | 7432 | 8462 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1304 | 714 | 9038 | 10568 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 | 6386 | 10732 | 12262 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5936 | 4876 | 8264 | 9324 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14368 | 12338 | 148 | 472 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9736 | 8176 | 4014 | 5104 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 | 16062 | 6628 | 7248 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 | 13124 | 9096 | 10186 |

IMD4 caused by UL DC_20_n8 may fall into Rx band n78.

IMD4 caused by UL DC_20_n78 may fall into Rx band n8.

### 6.124.4	 ∆TIB and ∆RIB values

Table 6.124.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A_n8A-n78A | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 6.124.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20A_n8A-n78A | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 6.124.5	MSD

All MSD requirements for DC_20A_n8A-n78A can reuse the requriements of DC_8A-20A_n78A and DC_8A_n28A-n78A. The MSD exceptions are shown below.

Table 6.124.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A_n8A-n78A | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n78 | 3567 | 10 | 50 | 3567 | 10.3 | IMD4 |
|  | n8 | 895 | 5 | 25 | 940 | 12.1 | IMD4 |
|  | n78 | 3481 | 10 | 50 | 3481 | N/A | N/A |
|  | 20 | 847 | 5 | 25 | 806 | N/A | N/A |

## 6.125	DC_3_n1-n38

### 6.125.1	Operating bands for DC

Table 6.125.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n38 | 3 | 1710MHz | – | 1785MHz | 1805MHz | – | 1880MHz | FDD |
|  | n1 | 1920MHz | – | 1980MHz | 2110MHz | – | 2170MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

### 6.125.2	Channel bandwidths per operating band for DC

Table 6.125.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n1A-n38A | DC_3A_n1A DC_3A_n38A | 3 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 110 |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n38 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

### 6.125.3	Co-existence studies

Based on co-existence studies there are no IMD issues into own Rx of bands.

### 6.125.4	∆TIB and ∆RIB values

For DC_3A_n1A-n38, the TIB,c and RIB,c values are derived from DC_1-3_n38 and are given in the tables below.

Table 6.125.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n1-n38 | 3 | 0.5 |
|  | n1 | 0.5 |
|  | n38 | 0.5 |

Table 6.125.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n1-n38 | 3 | 0 |
|  | n1 | 0 |
|  | n38 | 0 |

### 6.125.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is no IMD issues.

## 6.126	DC_8_n1-n40

### 6.126.1	Operating bands for DC

Table 6.126.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n1-n40 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 6.126.2	Configuration for DC

Table 6.126.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_8A_n1A-n40A | DC_8A_n1A DC_8A_n40A | 8A | CA_n1A-n40A |

### 6.126.3	Co-existence studies

Co-existence analysis for DC_8_n1 UL shows no impact to NR Band n40 DL.

Co-existence analysis for DC_8_n40 UL shows 5th IMD might fall in NR Band n1 DL.

### 6.126.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.126.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n1-n40 | 8 | 0.3 |
|  | n1 | 0.3 |
|  | n40 | 0.5 |

Table 6.126.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n1-n40 | 8 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0.5 |

### 6.126.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 produced by Band 8 and NR band n40 that impact the reference sensitivity of NR band n1.

The required MSD levels and test configuration are shown in the following Table.

Table 6.126.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n1A-n40A | B8 | IMD5 | ¦3*fB8 -2*fn40¦ | 885 | 5 | 25 | 930 | 5 | N/A |
|  | n40 |  |  | 2390 | 5 | 25 | 2390 | 5 |  |
|  | n1 |  |  | 1935 | 5 | 25 | 2125 | 5 | 3.3 |

## 6.127	DC_8_n3-n79

### 6.127.1	Operating bands for DC

Table 6.127.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n3-n79 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.127.2	Channel bandwidths per operating band for DC

Table 6.127.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n3A-n79A | DC_8A_n3A DC_8A_n79A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 6.127.3	Co-existence studies

Based on the co-existence studies of DC_8A-n3A and DC_8A-n79A, 3rd and 4th order IMD generated by dual uplink of Band 8 + Band n3 may also fall into own Rx of band n79 and 4th order IMD generated by dual uplink of Band 8 + Band n79 may also fall into own Rx of band n3.

### 6.127.4	∆TIB and ∆RIB values

For DC_8_n3-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.127.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n3-n79 | 8 | 0.3 |
|  | n3 | 0.3 |
|  | n79 | 0.8 |

Table 6.127.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n3-n79 | 8 | 0 |
|  | n3 | 0 |
|  | n79 | 0.5 |

### 6.127.5	MSD

As mentioned in 6.127.3, IMD3 and IMD4 of B8 and n3 to Band n79 Rx and IMD4 of B8 and n79 to Band n3 need to be addressed for REFSENS relaxation. The following values are proposed:

Table 6.127.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A_n3A-n79A | 8 | 885 | 5 | 25 | 930 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n79 | 4425 | 40 | 216 | 4425 | 15.7 | TDD | IMD31 |
| DC_8A_n3A-n79A | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | TDD | N/A |
|  | n3 | 1755 | 5 | 25 | 1850 | 8.8 | FDD | IMD4 |
| NOTE 1: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

## 6.128	DC_8_n77-n79

### 6.128.1	Operating bands for DC

Table 6.128.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n77-n79 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.128.2	Channel bandwidths per operating band for DC

Table 6.128.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n77A-n79A | DC_8A_n77A DC_8A_n79A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 6.128.3	Co-existence studies

Based on the co-existence studies of DC_8A-n77A and DC_8A-n79A, 2nd, 4th and 5th order IMD generated by dual uplink of Band 8 + Band n77 may also fall into own Rx of band n79 and 2nd order IMD generated by dual uplink of Band 8 + Band n79 may also fall into own Rx of band n77. But those IMDs are not occurred since the non-simultaneous Rx/Tx operation is considered for CA_n77-n79.

### 6.128.4	∆TIB and ∆RIB values

For DC_8_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.128.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n77-n79 | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 6.128.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n77-n79 | 8 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |

### 6.128.5	MSD

As mentioned in 6.128.3, there is no need to specify additional MSD requirement for this UL DC configuration

## 6.129	DC_1_n3-n79

### 6.129.1	Operating bands for DC

Table 6.129.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n3-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.129.2	Channel bandwidths per operating band for DC

Table 6.129.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A_n3A-n79A | DC_1A_n3A DC_1A_n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 6.129.3	Co-existence studies

Based on the co-existence studies of DC_1A-n3A and DC_1A-n79A, 5th order IMD generated by dual uplink of Band 1 + Band n3 may also fall into own Rx of band n79.

### 6.129.4	∆TIB and ∆RIB values

For DC_1_n3-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.129.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n79 | 1 | 0.3 |
|  | n3 | 0.3 |
|  | n79 | 0.8 |

Table 6.129.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n79 | 1 | 0 |
|  | n3 | 0 |
|  | n79 | 0.5 |

### 6.129.5	MSD

As mentioned in 6.129.3, IMD5 of B1 and n3 to Band n79 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A_n3A-n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n79 | 4950 | 40 | 216 | 4950 | 4.7 | TDD | IMD5 |

## 6.130	DC_8_n28-n79

### 6.130.1	Operating bands for DC

Table 6.130.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n28-n79 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.130.2	Channel bandwidths per operating band for DC

Table 6.130.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n28A-n79A | DC_8A_n28A DC_8A_n79A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 6.130.3	Co-existence studies

Based on the co-existence studies of DC_8A-n28A and DC_8A-n79A, 5th order IMD generated by dual uplink of Band 8 + Band n28 may also fall into own Rx of band n79 and 5th order IMD generated by dual uplink of Band 8 + Band n79 may also fall into own Rx of band n28.

### 6.130.4	∆TIB and ∆RIB values

For DC_8_n28-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.130.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n28-n79 | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n79 | 0.8 |

Table 6.130.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n28-n79 | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n79 | 0.5 |

### 6.130.5	MSD

As mentioned in 6.130.3, IMD5 of B8 and n28 to Band n79 Rx and IMD5 of B8 and n79 to Band n28 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A_n28A-n79A | 8 | 912.5 | 5 | 25 | 957.5 | N/A | FDD | N/A |
|  | n28 | 745.5 | 5 | 25 | 800.5 | N/A | FDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | 0.0 | FDD | IMD5 |
| DC_8A_n28A-n79A | 8 | 905 | 5 | 25 | 950 | N/A | FDD | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | TDD | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 3.9 | FDD | IMD5 |

## 6.131	DC_12_n66-n78

### 6.131.1	Operating bands for DC

Table 6.131.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n66-n78 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.131.2	Configuration for DC

Table 6.131.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | DC_12A_n66A DC_12A_n78A |

### 6.131.3	Co-existence studies

Based on co-existence studies,

- 5th order IMD generated by dual uplink of 12_n66 may fall into own Rx of band n78

- 3rd order IMD generated by dual uplink of 12_n78 may fall into own Rx of band n66

### 6.131.4	∆TIB and ∆RIB values

For DC_12_n66-n78, the TIB,c and RIB,c values are derived from DC_12-66_n78 and are given in the tables below.

Table 6.131.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n66-n78 | 12 | 0.6 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 6.131.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12_n66-n78 | 12 | 0.2 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |

### 6.131.5	MSD

As mentioned in 6.131.3, 3rd order IMD generated by dual uplink of 12_n78 may fall into own Rx of band n66, and 5th order IMD generated by dual uplink of 12_n66 may fall into own Rx of band n78. As agreed in TR 37.716-21-21, the MSD is defined in Table 6.131.5-1.

Table 6.131.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | 12 | 703 | 5 | 25 | 733 | N/A | N/A |
|  | n66 | 1740 | 5 | 25 | 2140 | 16.5 | IMD3 |
|  | n78 | 3546 | 10 | 50 | 3546 | N/A | N/A |
| DC_12A_n66A-n78A DC_12A_n66(2A)-n78A DC_12A_n66A-n78(2A) DC_12A_n66(2A)-n78(2A) | 12 | 703 | 5 | 25 | 733 | N/A | N/A |
|  | n66 | 1720 | 5 | 25 | 2120 | N/A | N/A |
|  | n78 | 3754 | 10 | 50 | 3754 | 4.1 | IMD5 |

## 6.132	DC_13_n7-n78

### 6.132.1	Operating bands for DC

Table 6.132.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13_n7-n78 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.132.2	Configuration for DC

Table 6.132.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_13A_n7A-n78A | DC_13A_n7A DC_13A_n78A |

### 6.132.3	Co-existence studies

Based on co-existence studies,

- 2nd and 4th order IMD generated by dual uplink of 13_n7 may fall into own Rx of band n78

- 2nd order IMD generated by dual uplink of 13_n78 may fall into own Rx of band n7

### 6.132.4	∆TIB and ∆RIB values

For DC_13_n7-n78, the TIB,c and RIB,c values are derived from DC_12_n7-n78 and are given in the tables below.

Table 6.132.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n7-n78 | 13 | 0.5 |
|  | n7 | 0.5 |
|  | n78 | 0.8 |

Table 6.132.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13_n7-n78 | 13 | 0.2 |
|  | n7 | 0.5 |
|  | n78 | 0.5 |

### 6.132.5	MSD

As mentioned in 6.132.3, -	2nd and 4th order IMD generated by dual uplink of 13_n7 may fall into own Rx of band n78, and 2nd order IMD generated by dual uplink of 13_n78 may fall into own Rx of band n7. As agreed in TR 37.716-21-21, the MSD is defined in Table 6.132.5-1.

Table 6.132.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n7A-n78A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n78 | 3432 | 10 | 50 | 3432 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 27.9 | IMD2 |
| DC_13A_n7A-n78A | 13 | 749 | 5 | 25 | 780 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3622 | 10 | 50 | 3622 | 9 | IMD4 |
| DC_13A_n7A-n78A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n78 | 3312 | 10 | 50 | 3312 | 29.0 | IMD2 |

## 6.133	DC_3_n1-n41

### 6.133.1	Operating bands for DC

Table 6.133.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n41 | 3 | 1710MHz | – | 1785MHz | 1805MHz | – | 1880MHz | FDD |
|  | n1 | 1920MHz | – | 1980MHz | 2110MHz | – | 2170MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 6.133.2	Channel bandwidths per operating band for DC

Table 6.133.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n1A-n41A | DC_3A_n1A DC_3A_n41A | 3 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.133.3	Co-existence studies

Based on co-existence studies there are IMD5 impact from UL 3_n1 into DL n41.

### 6.133.4	∆TIB and ∆RIB values

For DC_3A_n1A-n41, the TIB,c and RIB,c values are derived from DC_1-3_n41 and are given in the tables below.

Table 6.133.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n1-n41 | 3 | 0.5 |
|  | n1 | 0.5 |
|  | n41 | 0.5 |

Table 6.133.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n1-n41 | 3 | 0 |
|  | n1 | 0 |
|  | n41 | 0 |

### 6.133.5	MSD

Based on co-existence studies MSD need to be defined. MSD value in table below is same as for DC_1A_n3A-n41A.

Table 6.133.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n1A-n41A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n41 | 2507.5 | 5 | 25 | 2507.5 | 5.0 | IMD5 |

## 6.134	DC_8_n1-n77

### 6.134.1	Operating bands for DC

Table 6.134.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n1-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.134.2	Channel bandwidths per operating band for DC

Table 6.134.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n1A-n77A5 | DC_8A_n1A DC_8A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A_n1A-n77(2A)5 | DC_8A_n1A DC_8A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.134.3	Co-existence studies

Based on the co-existence studies of DC_8A-n1A and DC_8A-n77A, 3rd and 5th order IMD generated by dual uplink of Band 8 + Band n1 may also fall into own Rx of band n77 and 3rd order IMD generated by dual uplink of Band 8 + Band n77 may also fall into own Rx of band n1.

### 6.134.4	∆TIB and ∆RIB values

For DC_8_n1-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.134.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n1-n77 | 8 | 0.6 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 6.134.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n1-n77 | 8 | 0.2 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

### 6.134.5	MSD

As mentioned in 6.134.3, IMD3 and IMD5 of B 8 and n1 to Band n77 Rx and IMD3 of B 8 and n77 to Band n1 Rx need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_8A_n1A-n77A | 8 | 900 | 5 | 25 | 945 | N/A | FDD | N/A |
|  | n1 | 1945 | 5 | 25 | 2135 | N/A | FDD | N/A |
|  | n77 | 3745 | 10 | 50 | 3745 | 14.9 | TDD | IMD31 |
| DC_8A_n1A-n77A | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n77 | 3960 | 10 | 50 | 3960 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 14.4 | FDD | IMD3 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

## 6.135	DC_42_n1-n3

### 6.135.1	Operating bands for DC

Table 6.135.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n1-n3 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |

### 6.135.2	Channel bandwidths per operating band for DC

Table 6.135.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_42A_n1A-n3A | DC_42A_n1A DC_42A_n3A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 60 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
| DC_42C_n1A-n3A | DC_42A_n1A DC_42C_n1A DC_42A_n3A DC_42C_n3A | 42 | CA_42C |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |

### 6.135.3	Co-existence studies

Based on the co-existence studies of DC_42A-n1A and DC_42A-n3A, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 6.135.4	∆TIB and ∆RIB values

For DC_42_n1-n3, the TIB,c and RIB,c values are given in the tables below.

Table 6.135.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n1-n3 | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n3 | 0.6 |

Table 6.135.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n1-n3 | 42 | 0.5 |
|  | n1 | 0 |
|  | n3 | 0.2 |

### 6.135.5	MSD

As mentioned in 6.135.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.136	DC_7_n12-n77

### 6.136.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD3 produced by Band 7 and NR band n12 that impact the reference sensitivity of NR band n77. Also IMD4 product impacted to the n77. But the MSD by IMD4 is not specified.

The required MSD level and test configuration are shown in the following Table.

Table 6.136.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n12A-n77A | B7 | IMD3 | ¦2*fn12 +fB7¦ | 2520 | 10 | 50 | 2640 | 10 | N/A |
|  | n12 |  |  | 708 | 5 | 25 | 738 | 5 |  |
|  | n77 |  |  | 3936 | 10 | 50 | 3936 | 10 | 15.2 |

## 6.137	DC_7_n71-n77

### 6.137.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD3 produced by Band 7 and NR band n71 that impact the reference sensitivity of NR band n77. Also IMD4 product impacted to the n77. But the MSD by IMD4 is not specified.

The required MSD level and test configuration are shown in the following Table.

Table 6.137.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n71A-n77A | B7 | IMD3 | ¦2*fn71 +fB7¦ | 2520 | 10 | 50 | 2640 | 10 | N/A |
|  | n71 |  |  | 693 | 5 | 25 | 647 | 5 |  |
|  | n77 |  |  | 3906 | 10 | 50 | 3906 | 10 | 15.2 |

## 6.138	DC_5_n2-n41

### 6.138.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 5 and NR band n2 that impact the reference sensitivity of NR band n41.

The required MSD levels and test configuration are shown in the following Table.

Table 6.138.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n2A-n41A | B5 | IMD2 | ¦fB5 +fn2¦ | 829 | 5 | 25 | 874 | 5 | N/A |
|  | n2 |  |  | 1855 | 5 | 25 | 1935 | 5 |  |
|  | n41 |  |  | 2684 | 5 | 25 | 2684 | 5 | 29.7 |

## 6.139	DC_3_n28-n75

### 6.139.1	 Operating bands for DC

Table 6.139.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n28-n75 | 1 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n75 | N/A | – | N/A MHz | 1432 MHz | – | 1517 MHz | SDL |

### 6.139.2 	Configuration for DC

Table 6.139.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n28A-n75A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 90 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| DC_3C_n28A-n75A |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 110 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

### 6.139.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_3_n28 are shown below.

Table 6.139.3-1: The harmonic and IMD products caused by UL DC_3_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1082 | 962 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2672 | 2867 | 379 | 214 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4123 | 4318 | 3116 | 3281 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4382 | 4652 | 324 | 534 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5833 | 6103 | 3819 | 4029 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1924 | 2164 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1282 | 1027 | 6437 | 6092 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1176 | 1461 | 3949 | 3634 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4522 | 4777 | 7543 | 7888 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5529 | 5814 | 6536 | 6851 |

The second harmonic interference of band n28 may fall into band n75.

IMD5 may fall into Rx of band n75 when band 3 and n28 transmit.

### 6.139.4	 ∆TIB and ∆RIB values

Table 6.139.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n28-n75 | 3 | 0.3 |
|  | n28 | 0.3 |

Table 6.139.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n28-n75 | 3 | 0.5 |
|  | n28 | 0.5 |
|  | n75 | 0 |

### 6.139.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD5 produced by Band 3 and NR band n28 that impact the reference sensitivity of NR band n75.

The required MSD levels and test configuration are shown in the following Table.

Table 6.139.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n28A-n75A DC_3C_n28A-n75A | B3 | IMD5 | ¦3*fn28 -2*fB3¦ | 1780 | 5 | 25 | 1875 | 5 | N/A |
|  | n28 |  |  | 708 | 5 | 25 | 763 | 5 |  |
|  | n75 |  |  | - | - | - | 1436 | 5 | 3.3 |

## 6.140	DC_1_n38-n78

### 6.140.1	Operating bands for DC

Table 6.140.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n38-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.140.2	Configuration for DC

Table 6.140.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A_n38A-n78A | DC_1A_n78A | 1A | CA_n38A-n78A |

### 6.140.3	Co-existence studies

Co-existence analysis for DC_1_n78 UL in TR 37.863-01-01 shows IMD4 produce may fall into NR Band n38 DL.

### 6.140.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.140.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n38-n78 | 1 | 0.5 |
|  | n38 | 0.5 |
|  | n78 | 0.8 |

Table 6.140.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n38-n78 | 1 | 0 |
|  | n38 | 0 |
|  | n78 | 0.5 |

### 6.140.5	MSD

Based on co-existence studies additional MSD is needed to be defined. The MSD value is given in table 6.140.5-1.

Table 6.140.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n38A-n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n38 | 2590 | 5 | 25 | 2590 | 12.7 | IMD4 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |

## 6.141	DC_3_n38-n78

### 6.141.1	Operating bands for DC

Table 6.141.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n38-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.141.2	Configuration for DC

Table 6.141.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_3A_n38A-n78A | DC_3A_n78A | 3A | CA_n38A-n78A |

### 6.141.3	Co-existence studies

Co-existence analysis for DC_3_n78 UL have already been captured in TR 37.863-01-01, where there are no IMD produces may fall into NR Band n38 DL.

### 6.141.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.141.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n38-n78 | 3 | 0.6 |
|  | n38 | 0.5 |
|  | n78 | 0.8 |

Table 6.141.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n38-n78 | 3 | 0.5 |
|  | n38 | 0 |
|  | n78 | 0.5 |

### 6.141.5	MSD

According to the co-existence studies, there are no additional MSD will be defined.

## 6.142	DC_7_n38-n78

### 6.142.1	Operating bands for DC

Table 6.142.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n38-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.142.2	Configuration for DC

Table 6.142.2-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_7A_n38A-n78A17,18 | N/A | NA | n78A |
| NOTE 17: The combination is not used alone as fall back mode of other band combinations. NOTE 18: Power imbalance between downlink carriers on Band 7 and Band 38 or band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

To avoid duplicated NOTEs in the TS38.101-3 specification, it is proposed to add ‘or band n38’ on top of the existing NOTE 18 in table 5.5B.4.2 in the TS38.101-3, shown as above.

### 6.142.3	Co-existence studies

No need to analysis the co-existence studies for this configuration.

### 6.142.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.142.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n38-n78 | n78 | 0.8 |

Table 6.142.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n38-n78 | 7 | 0 |
|  | n38 | 0 |
|  | n78 | 0.5 |

### 6.142.5	MSD

There are no additional MSD is needed to be defined.

## 6.143	DC_20_n38-n78

### 6.143.1	Operating bands for DC

Table 6.143.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n38-n78 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.143.2	Configuration for DC

Table 6.143.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_20A_n38A-n78A | DC_20A_n78A | 20A | CA_n38A-n78A |

### 6.143.3	Co-existence studies

Co-existence analysis for DC_20_n78 UL in TR 37.863-01-01 shows IMD2 produce may fall into NR Band n38 DL.

### 6.143.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 6.143.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n38-n78 | 20 | 0.6 |
|  | n38 | 0.3 |
|  | n78 | 0.8 |

Table 6.143.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n38-n78 | 20 | 0.2 |
|  | n38 | 0 |
|  | n78 | 0.5 |

### 6.143.5	MSD

Based on co-existence studies additional MSD is needed to be defined. The MSD value is given in table 6.143.5-1.

Table 6.143.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n38A-n78A | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | n38 | 2600 | 5 | 25 | 2600 | 30.9 | IMD2 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |

## 6.144	DC_1_n3-n8

### 6.144.1	 Operating bands for DC

Table 6.144.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n3A-n8A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

### 6.144.2 	Configuration for DC

Table 6.144.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A_n3A-n8A | DC_1A_n3A DC_1A_n8A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 50 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.144.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_1A_n3A and DC_1A_n8A are shown below.

Table 6.144.3-1: The harmonic and IMD products caused by UL DC_1A_n3A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 270 | 135 | 3630 | 3765 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2055 | 2250 | 1440 | 1650 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5550 | 5745 | 5340 | 5550 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 4230 | 3150 | 3435 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7470 | 7725 | 7050 | 7335 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 270 | 540 | 7260 | 7530 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5220 | 4860 | 6210 | 5895 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1515 | 1170 | 2520 | 2190 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 | 9120 | 9390 | 9705 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9315 | 9180 | 9510 |

Table 6.144.3-2: The harmonic and IMD products caused by UL DC_1A_n8A

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

### 6.144.4	 ∆TIB and ∆RIB values

Table 6.144.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A_n3A-n8A | 1 | 0.3 |
|  | n3 | 0.3 |
|  | n8 | 0.3 |

Table 6.144.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A_n3A-n8A | 1 | 0 |
|  | n3 | 0 |
|  | n8 | 0 |

### 6.144.5	MSD

The second harmonic of n8 may occur on the Rx of n3. The corresponding MSD values are already mentionned in 38.101-3 Table 7.3B.2.3.1-1

## 6.145	DC_8_n1-n28

### 6.145.1	 Operating bands for DC

Table 6.145.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n1-n28 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 6.145.2 	Configuration for DC

Table 6.145.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A_n1A-n28A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n28 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |

### 6.145.3	 Co-existence studies

When Uplink EN-DC configuration is DC_8_n28, 5th IMD will fall into Rx band of Band 1. However, when consider the 5MHz UL channel BW for DC_8_n28, the IMD5 does not fall into the B1 own Rx band. This was decision made for DC_1-8_n28 in 37.716-21-11.

IMD4 caused by UL DC_8_n1 may fall into Rx band n28.

### 6.145.4	 ∆TIB and ∆RIB values

Relaxation values of DC_1A-8A_n28A are re-used

Table 6.145.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n1-n28 | 8 | 0.6 |
|  | n1 | 0.3 |
|  | n28 | 0.6 |

Table 6.145.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n1-n28 | 8 | 0.2 |
|  | n28 | 0.2 |

### 6.145.5	MSD

IMD4 MSD is copied from DC_8A_n28A-n78A and IMD5 treatment from DC_2A-5A_n12A is reused

Table 6.145.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A_n1A-n28A1 | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n1 | 1965 | 5 | 25 | 2155 | N/A | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 11.6 | IMD4 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.146	DC_11_n77-n79

### 6.146.1	Operating bands for DC

Table 6.146.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n77-n79 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.146.2	Channel bandwidths per operating band for DC

Table 6.146.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_11A_n77A-n79A | DC_11A_n77A DC_11A_n79A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
| DC_11A_n77(2A)-n79A | DC_11A_n77A DC_11A_n79A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 310 |
|  |  | n77 | CA_n77(2A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 6.146.3	Co-existence studies

Based on the co-existence studies of DC_11A-n77A and DC_11A-n79A, 2nd and 4th order IMD generated by dual uplink of Band 11 + Band n77 may also fall into own Rx of band n79 and 2nd order IMD generated by dual uplink of Band 11 + Band n79 may also fall into own Rx of band n77. But due to the TDD synchronous operation of n77 and n79, they are not impacted.

### 6.146.4	∆TIB and ∆RIB values

For DC_11_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.146.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n77-n79 | 11 | 0.4 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 6.146.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n77-n79 | 11 | 0 |
|  | n77 | 0.5 |
|  | n79 | 0 |

### 6.146.5	MSD

As mentioned in 6.146.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.147	DC_7_n78-n79

### 6.147.1	 Operating bands for DC

Table 6.147.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n78-n79 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.147.2 	Configuration for DC

Table 6.147.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n78A-n79A |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_7A_n78A-n79C |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 320 |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n79 | See CA_n79C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.147.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_7_n78 and DC_7_n79 are shown below.

Table 6.147.3-1: The harmonic and IMD products caused by UL DC_7_n79

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 4400 | 5000 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 8800 | 10000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 13200 | 15000 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 17600 | 20000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 22000 | 25000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1830 | 2500 | 6900 | 7570 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 0 | 740 | 6230 | 7500 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9400 | 10140 | 11300 | 12570 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2500 | 3310 | 10630 | 12500 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11900 | 12710 | 15700 | 17570 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5000 | 3660 | 13800 | 15140 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 17500 | 15030 | 5880 | 5000 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 10000 | 8060 | 1090 | 2500 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 20100 | 22570 | 14400 | 15280 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 18200 | 20140 | 16300 | 17710 |

Table 6.147.3-2: The harmonic and IMD products caused by UL DC_7_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 730 | 1300 | 5800 | 6370 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13970 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2600 | 1460 | 11600 | 12740 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12700 | 10630 | 6980 | 6200 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6400 | 4760 | 1110 | 100 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 17770 | 13300 | 14080 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

IMD4 caused by UL DC_7_n79 may fall into Rx band n78.

IMD3, IMD4 and IMD5 caused by UL DC_7_n78 may fall into Rx band n79.

### 6.147.4	 ∆TIB and ∆RIB values

Table 6.147.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7_n78-n79 | 7 | 0.5 |
|  | n78 | 0.8 |
|  | n79 | 0.8 |

Table 6.147.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7_n78-n79 | 7 | 0.5 |
|  | n78 | 0.5 |
|  | n79 | 0.5 |

### 6.147.5	MSD

Based on co-existence studies additional MSD is needed to be defined due to 3rd and 4th IMD.

Table 6.147.5-1: Proposed MSD levels from interested companies for DC_7_n78-n79

| DC bands | UL DC | IMD |  | HW | LG | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n78A-n79A DC_7A_n78A-n79C | 7 | IMD3 | ¦2*fn78 -fB7¦ | 24.5 | 16.7 | 20.6 |
|  | n78 |  |  |  |  |  |
|  | n79 |  |  |  |  |  |
|  | 7 | IMD4 | ¦2*fB7 -2*fn79¦ | 2.4 | 10.4 | 6.4 |
|  | n78 |  |  |  |  |  |
|  | n79 |  |  |  |  |  |

Table 6.147.5-2: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n78A-n79A DC_7A_n78A-n79C | 7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3600 | 10 | 50 | 3600 | N/A | N/A |
|  | n79 | 4680 | 40 | 216 | 4680 | 20.6 | IMD31, 2 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3770 | 10 | 50 | 3770 | 6.4 | IMD4 |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: For the DC band combination, simultaneous Rx/Tx capability is allowed between n78 and n79. |  |  |  |  |  |  |  |

## 6.148	DC_8_n1-n3

### 6.148.1	Operating bands for DC

Table 6.148.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n1-n3 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |

### 6.148.2	Channel bandwidths per operating band for DC

Table 6.148.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A_n1A-n3A | DC_8A_n1A DC_8A_n3A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 60 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |

### 6.148.3	Co-existence studies

Based on the co-existence studies of DC_8A-n1A and DC_8A-n3A, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 6.148.4	∆TIB and ∆RIB values

For DC_8_n1-n3, the TIB,c and RIB,c values are given in the tables below.

Table 6.148.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n1-n3 | 8 | 0.3 |
|  | n1 | 0.3 |
|  | n3 | 0.3 |

Table 6.148.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n1-n3 | 8 | 0 |
|  | n1 | 0 |
|  | n3 | 0 |

### 6.148.5	MSD

As mentioned in 6.148.3, there is no need to specify additional MSD requirement for this UL DC configuration..

## 6.149	DC_41_n1-n77

### 6.149.1	Operating bands for DC

Table 6.149.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41_n1-n77 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.149.2	Channel bandwidths per operating band for DC

Table 6.149.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_41A_n1A-n77A | DC_41A_n1A DC_41A_n77A | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_41C_n1A-n77A | DC_41A_n1A DC_41A_n77A DC_41C_n77A | 41 | See CA_41C |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 6.149.3	Co-existence studies

Based on the co-existence studies of DC_41A-n1A and DC_41A-n77A, 3rd, 4th and 5th order IMD generated by dual uplink of Band 41 + Band n1 may fall into own Rx of band n77 and 4th order IMD generated by dual uplink of Band 41 + Band n77 may also fall into own Rx of band n1.

### 6.149.4	∆TIB and ∆RIB values

For DC_41_n1-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.149.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n1-n77 | 41 | 0.5 |
|  | n1 | 0.5 |
|  | n77 | 0.8 |

Table 6.149.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41_n1-n77 | 41 | 0 |
|  | n1 | 0 |
|  | n77 | 0.5 |

### 6.149.5	MSD

As mentioned in 6.149.3, IMD3, IMD4 and IMD5 of B41 and n1 to Band n77 Rx and IMD4 of B41 and n77 to Band n1 need to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_41A_n1A-n77A DC_41C_n1A-n77A | 41 | 2650 | 5 | 25 | 2650 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3330 | 10 | 50 | 3330 | 19.6 | TDD | IMD34, 9 |
|  | 41 | 2510 | 5 | 25 | 2510 | N/A | TDD | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A | TDD | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | 11.0 | FDD | IMD4 |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

## 6.150	DC_42_n1-n3

### 6.150.1	Operating bands for DC

Table 6.150.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_42_n1-n3 | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |

### 6.150.2	Channel bandwidths per operating band for DC

Table 6.150.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_42A_n1A-n3A5 | DC_42A_n1A DC_42A_n3A | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 90 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| DC_42C_n1A-n3A5 | DC_42A_n1A DC_42A_n3A DC_42C_n1A DC_42C_n3A | 42 | CA_42C |  |  |  |  |  |  |  |  |  |  |  |  |  | 110 |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.150.3	Co-existence studies

Based on the co-existence studies of DC_42A-n1A and DC_42A-n3A, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 6.150.4	∆TIB and ∆RIB values

For DC_42_n1-n3, the TIB,c and RIB,c values are given in the tables below.

Table 6.150.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_42_n1-n3 | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n3 | 0.6 |

Table 6.150.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_42_n1-n3 | 42 | 0.5 |
|  | n1 | 0 |
|  | n3 | 0.2 |

### 6.150.5	MSD

As mentioned in 6.150.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.151	DC_11_n3-n79

### 6.151.1	Operating bands for DC

Table 6.151.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n3-n79 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 6.151.2	Channel bandwidths per operating band for DC

Table 6.151.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_11A_n3A-n79A5 | DC_11A_n3A DC_11A_n79A | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
| Note 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.151.3	Co-existence studies

Based on the co-existence studies of DC_11A-n3A and DC_11A-n79A, 3rd order IMD generated by dual uplink of Band 11 + Band n3 may also fall into own Rx of band n79 and 3rd order IMD generated by dual uplink of Band 11 + Band n79 may also fall into own Rx of band n3.

### 6.151.4	∆TIB and ∆RIB values

For DC_11_n3-n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.151.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n3-n79 | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n79 | 0.8 |

Table 6.151.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n3-n79 | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n79 | 0.5 |

### 6.151.5	MSD

As mentioned in 6.151.3, IMD3 of B11 and n3 to Band n79 Rx and IMD3 of B11 and n79 to Band n3 Rx need to be addressed for REFSENS relaxation. Based on the MSD analysis results in [2][3], the following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_11A_n3A-n79A | 11 | 1435 | 5 | 25 | 1483 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A | FDD | N/A |
|  | n79 | 4640 | 40 | 216 | 4640 | 16.2 | TDD | IMD3 |
|  | 11 | 1435 | 5 | 25 | 1483 | N/A | FDD | N/A |
|  | n79 | 4735 | 40 | 216 | 4735 | N/A | TDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | 17.8 | FDD | IMD3 |

## 6.152	DC_11_n1-n77

### 6.152.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there are IMD2 produced by Band 11 and NR band n1 that impact the reference sensitivity of NR band n77. Also IMD5 product impacted to the n77. But the MSD by IMD5 is not specified.

The required MSD level and test configuration are shown in the following Table.

Table 6.152.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_11A_n1A-n77A | B11 | IMD2 | ¦fn1 +fB11¦ | 1435 | 5 | 25 | 1483 | 5 | N/A |
|  | n1 |  |  | 1940 | 5 | 25 | 2130 | 5 |  |
|  | n77 |  |  | 3375 | 10 | 50 | 3375 | 10 | 29.62 |
| NOTE 2: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |

## 6.153	DC_3_n20-n67

### 6.153.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD4 produced by Band 3 and NR band n20 that impact the reference sensitivity of NR band n67.

The required MSD level and test configuration are shown in the following Table.

Table 6.153.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n20A-n67A | B3 | IMD4 | ¦3*fn20 -fB3¦ | 1765 | 5 | 25 | 1860 | 5 | N/A |
|  | n20 |  |  | 837 | 5 | 25 | 796 | 5 |  |
|  | n67 |  |  | - | - | - | 746 | 5 | 10.1 |

## 6.154	DC_11_n3-n79

### 6.154.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD3 produced by Band 11 and NR band n3 that impact the reference sensitivity of NR band n79.

The required MSD level and test configuration are shown in the following Table.

Table 6.154.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_11A_n3A-n79A | B11 | IMD3 | ¦fn3 +2*fB11¦ | 1435 | 5 | 25 | 1483 | 5 | N/A |
|  | n3 |  |  | 1770 | 5 | 25 | 1865 | 5 |  |
|  | n79 |  |  | 4640 | 40 | 216 | 4640 | 40 | 16.2 |

## 6.155	DC_2_n25-n66

### 6.155.1	Operating bands for DC

Table 6.155.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2_n25-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 6.155.2	Channel bandwidths per operating band for DC

Table 6.155.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n25A-n66A | DC_2A_n66A | 2 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |

### 6.155.3	Co-existence studies

Based on co-existence studies IMD3 and IMD5 from UL 2_n66 may impact DL n25.

### 6.155.4	∆TIB and ∆RIB values

For DC_2_n25-n66, the TIB,c and RIB,c values are derived from DC_2_n66 and are given in the tables below.

Table 6.155.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2_n25-n66 | 2 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 6.155.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2_n25-n66 | 2 | 0.3 |
|  | n25 | 0.3 |
|  | n66 | 0.3 |

### 6.155.5	MSD

Based on co-existence studies MSD need to be defined. MSD values are reused from DC_2A_n66A and DC_2A-66A_n25A.

Table 6.155.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_2_n25-n66 | 2 | 1855 | 5 | 25 | 1935 | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 | N/A | FDD | N/A |
|  | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 | 4 | FDD | IMD5 |
|  | n66 | 1750 | 5 | 25 | 2150 | N/A | FDD | N/A |

## 6.156	DC_5_n2-n78

### 6.156.1	Operating bands for DC

Table 6.156.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n2-n78 | 5 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.156.2	Channel bandwidths per operating band for DC

Table 6.156.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n2A-n78A | DC_5A_n2A DC_5A_n78A | 5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n2 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.156.3	Co-existence studies

Based on co-existence studies IMD3 from UL 5_n2 may impact DL n78.

Based on co-existence studies IMD3 from UL 5_n78 may impact DL n2.

### 6.156.4	∆TIB and ∆RIB values

For DC_5_n2-n78, the TIB,c and RIB,c values are derived from CA_n5-n25-n78 and are given in the tables below.

Table 6.156.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n2-n78 | 5 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 6.156.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n2-n78 | 5 | 0.2 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

### 6.156.5	MSD

Based on co-existence studies MSD need to be defined. MSD values are reused from CA_n5-n25-n77.

Table 6.156.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_5_n2-n78 | n5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | 16.0 | TDD | IMD3 |
|  | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n2 | 1907 | 5 | 25 | 1987 | 16.5 | FDD | IMD3 |
|  | n78 | 3680 | 10 | 25 | 3680 | N/A | TDD | N/A |

## 6.157	DC_66_n2-n78

### 6.157.1	Operating bands for DC

Table 6.157.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66_n2-n78 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.157.2	Channel bandwidths per operating band for DC

Table 6.157.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n2A-n78A | DC_66A_n2A DC_66A_n78A | 66 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 160 |
|  |  | n2 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.157.3	Co-existence studies

Based on co-existence studies IMD2 and IMD4 from UL 66_n2 may impact DL n78.

Based on co-existence studies IMD2, IMD4 and IMD5 from UL 66_n78 may impact DL n2.

### 6.157.4	∆TIB and ∆RIB values

For DC_66_n2-n78, the TIB,c and RIB,c values are derived from DC_66_n2-n77 and are given in the tables below.

Table 6.157.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n2-n78 | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 6.157.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n2-n78 | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n78 | 0.5 |

### 6.157.5	MSD

Based on co-existence studies MSD need to be defined. MSD values are reused from DC_66_n2-n77 in TS 38.101-3 and TR 37.717-11-21.

Table 6.157.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_66_n2-n78 | 66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 32.1 | FDD | IMD2 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | TDD | N/A |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 34.9 | TDD | IMD2 |
|  | 66 | 1740 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | 20.9 | TDD | IMD4 |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | FDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 21.1 | FDD | IMD4 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | TDD | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5 |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |

## 6.158	DC_12_n2-n78

### 6.158.1	Operating bands for DC

Table 6.158.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12_n2-n78 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.158.2	Channel bandwidths per operating band for DC

Table 6.158.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n2A-n78A | DC_12A_n2A DC_12A_n78A | 12 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n2 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.158.3	Co-existence studies

Based on co-existence studies IMD3 from UL 12_n2 may impact DL n78.

Based on co-existence studies IMD3 from UL 12_n78 may impact DL n2.

### 6.158.4	∆TIB and ∆RIB values

For DC_12_n2-n78, the TIB,c and RIB,c values are derived from CA_n12-n2-n77 and are given in the tables below.

Table 6.158.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12_n2-n78 | 12 | 0.3 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 6.158.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12_n2-n78 | 12 | 0.2 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

### 6.158.5	MSD

Based on co-existence studies MSD need to be defined. MSD values are reused from CA_n2-n12-n77.

Table 6.158.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_12_n2-n78 | 12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n2 | 1880 | 5 | 25 | 1960 | 16.5 | FDD | IMD3 |
|  | n78 | 3375 | 10 | 50 | 3375 | N/A | TDD | N/A |
|  | 12 | 707.5 | 5 | 25 | 737.5 | N/A | FDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 | N/A | FDD | N/A |
|  | n78 | 3315 | 10 | 50 | 3315 | 16.0 | TDD | IMD3 |

## 6.159	DC_1_n28-n75

### 6.159.1	 Operating bands for DC

Table 6.159.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n28-n75 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n75 | N/A | – | N/A MHz | 1432 MHz | – | 1517 MHz | SDL |

### 6.159.2 	Configuration for DC

Table 6.159.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A_n28A-n75A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 90 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

### 6.159.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_1_n28 are shown below.

Table 6.159.3-1: The harmonic and IMD products caused by UL DC_1_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1277 | 1172 | 2623 | 2728 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3092 | 3257 | 574 | 424 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4543 | 4708 | 3326 | 3476 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5012 | 5237 | 129 | 324 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6463 | 6688 | 4029 | 4224 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2344 | 2554 | 5246 | 5456 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1072 | 832 | 7217 | 6932 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1596 | 1851 | 4534 | 4264 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4732 | 4972 | 8383 | 8668 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5949 | 6204 | 7166 | 7436 |

There is no IMD issue for DC_1_n28-n75.

The 2nd harmonic interference from band n28 may fall into Rx band of band n75, but the MSD requirements have been specified into the TS 38.101-1.

### 6.159.4	 ∆TIB and ∆RIB values

Table 6.159.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n28-n75 | 1 | 0.3 |
|  | n28 | 0.7 |

Table 6.159.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n28-n75 | 1 | 0.2 |
|  | n28 | 0 |
|  | n75 | 0 |

## 6.160	DC_38_n3-n78

### 6.160.1	Operating bands for DC

Table 6.160.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_38_n3-n78 | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1710 MHz | – | 1785 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.160.2	Configuration for DC

Table 6.160.2-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_38A_n3A-n78A | DC_38A_n3A DC_38A_n78A | 38A | CA_n3A-n78A |

### 6.160.3	Co-existence studies

In terms of the co-existence analysis of DC_38_n3 UL and DC_38_n78 in TR 37.863-01-01 and TR 37.717-11-21

- IMD3 produces caused by DC_38_n3 UL may fall into NR Band n78 DL.

- IMD3 and IMD4 produces caused by DC_38_n78 UL fall into NR Band n3 DL.

### 6.160.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are proposed to be the same with DC_3A_n38A-n78A, which are given in the tables below.

Table 6.160.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_38_n3-n78 | 38 | 0.5 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 6.160.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_38_n3-n78 | 38 | 0 |
|  | n3 | 0.5 |
|  | n78 | 0.5 |

### 6.160.5	MSD

Based on co-existence studies additional MSD is needed to be defined. The MSD value is given in table 6.160.5-1.

Table 6.160.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_38A_n3A-n78A | 38 | 2560 | 10 | 50 | 2560 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 |
|  | 38 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n3 | 1745 | 5 | 25 | 1840 | 17.6 | IMD39 |
|  | n78 | 3400 | 10 | 52 | 3400 | N/A | N/A |
| NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.161	DC_3_n20-n67

### 6.161.1	Operating bands for DC

Table 6.161.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n20-n67 | 3 | 1805 MHz | – | 1880 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n67 | N/A |  |  | 738 MHz |  | 758 MHz | SDL |

### 6.161.2	Channel bandwidths per operating band for DC

Table 6.161.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n20A-n67A | DC_3A_n20A | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 80 |
|  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n67 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| DC_3C_n20A-n67A | DC_3A_n20A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  | n20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n67 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

### 6.161.3	Co-existence studies

Based on co-existence studies there is IMD4 impact from UL 3_n20 into DL n67.

### 6.161.4	∆TIB and ∆RIB values

For DC_3_n20-n67, the TIB,c and RIB,c values are derived from DC_3-20_n28 and are given in the tables below.

Table 6.161.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n20-n67 | 3 | 0.3 |
|  | n20 | 0.5 |
|  | n67 | 0.5 |

Table 6.161.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n20-n67 | 3 | 0 |
|  | n20 | 0.1 |
|  | n67 | 0.1 |

### 6.161.5	MSD

As can be seen in the co-existence studies there is a need to define MSD. MSD value from R4-2200794.

Table 6.161.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_3_n20-n67 | 3 | 1765 | 5 | 25 | 1860 | N/A | FDD | N/A |
|  | n20 | 837 | 5 | 25 | 796 | N/A | FDD | N/A |
|  | n67 | N/A | 5 | 25 | 746 | 10.1 | FDD | IMD4 |

## 6.162	DC_11_n1-n77

### 6.162.1	Operating bands for DC

Table 6.162.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_11_n1-n77 | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.162.2	Channel bandwidths per operating band for DC

Table 6.162.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_11A_n1A-n77A5 | DC_11A_n1A DC_11A_n77A | 11 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| DC_11A_n1A-n77(2A)5 | DC_11A_n1A DC_11A_n77A | 11 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.162.3	Co-existence studies

Based on the co-existence studies of DC_11A-n1A and DC_11A-n77A, 2nd and 5th order IMD generated by dual uplink of Band 11 + Band n1 may also fall into own Rx of band n77 and 2nd and 5th order IMD generated by dual uplink of Band 11 + Band n77 may also fall into own Rx of band n1.

### 6.162.4	∆TIB and ∆RIB values

For DC_11_n1-n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.162.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_11_n1-n77 | 11 | 0.4 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 6.162.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_11_n1-n77 | 11 | 0 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

### 6.162.5	MSD

As mentioned in 6.162.3, IMD2 and IMD5 of B11 and n1 to Band n77 Rx and IMD2 and IMD5 of B11 and n77 to Band n1 Rx need to be addressed for REFSENS relaxation. Thanks to [2] and [3], the following values are proposed:

Table 5.1.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_11A_n1A-n77A DC_11A_n1A-n77(2A) | 11 | 1435 | 5 | 25 | 1483 | N/A | FDD | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | n77 | 3375 | 10 | 50 | 3375 | 29.6 | TDD | IMD21 |
|  | 11 | 1438 | 5 | 25 | 1486 | N/A | FDD | N/A |
|  | n77 | 3578 | 10 | 50 | 3578 | N/A | TDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | 30.8 | FDD | IMD21 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

## 6.163	DC_5_n1-n78

### 6.163.1	Operating bands for DC

Table 6.163.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n1-n78 | 5 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.163.2	Channel bandwidths per operating band for DC

Table 6.163.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n1A-n78A | DC_5A_n1A DC_5A_n78A | 5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.163.3	Co-existence studies

Based on co-existence studies IMD3 from UL 5_n1 may impact DL n78.

Based on co-existence studies IMD3 from UL 5_n78 may impact DL n1.

### 6.163.4	∆TIB and ∆RIB values

For DC_5_n1-n78, the TIB,c and RIB,c values are derived from CA_n1-n5-n78 and are given in the tables below.

Table 6.163.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n1-n78 | 5 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 6.163.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n1-n78 | 5 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 6.163.5	MSD

Based on co-existence studies MSD need to be defined. Values from CA_n1-n5-n78 are reused.

Table 6.163.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_5_n1-n78 | 5 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n1 | 1932 | 5 | 25 | 2122 | 18.1 | FDD | IMD3 |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | 5 | 830 | 5 | 25 | 875 | N/A | FDD | N/A |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n78 | 3610 | 10 | 50 | 3610 | 15.7 | TDD | IMD3 |

## 6.164	DC_5_n3-n78

### 6.164.1	Operating bands for DC

Table 6.164.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5_n3-n78 | 5 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.164.2	Channel bandwidths per operating band for DC

Table 6.164.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A_n3A-n78A | DC_5A_n3A DC_5A_n78A | 5 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.164.3	Co-existence studies

Based on co-existence studies IMD3 and IMD5 from UL 5_n3 may impact DL n78.

Based on co-existence studies IMD3 from UL 5_n78 may impact DL n3.

### 6.164.4	∆TIB and ∆RIB values

For DC_5_n3-n78, the TIB,c and RIB,c values are derived from CA_n3-n5-n78 and are given in the tables below.

Table 6.164.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n3-n78 | 5 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 6.164.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5_n3-n78 | 5 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 6.164.5	MSD

Based on co-existence studies MSD need to be defined. Values from CA_n3-n5-n78 are reused.

Table 6.164.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_5_n3-n78 | 5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n78 | 3408 | 10 | 50 | 3408 | 16.1 | TDD | IMD3 |
|  | 5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n78 | 3512 | 10 | 50 | 3512 | 4.5 | TDD | IMD5 |
|  | 5 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n3 | 1767 | 5 | 25 | 1862 | 15.7 | FDD | IMD3 |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |

## 6.165	DC_8_n3-n78

### 6.165.1	Operating bands for DC

Table 6.165.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n3-n78 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.165.2	Channel bandwidths per operating band for DC

Table 6.165.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n3A-n78A | DC_8A_n3A DC_8A_n78A | 8 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.165.3	Co-existence studies

Based on co-existence studies IMD3 and IMD5 from UL 8_n3 may impact DL n78.

Based on co-existence studies IMD3 from UL 8_n78 may impact DL n3.

### 6.165.4	∆TIB and ∆RIB values

For DC_8_n3-n78, the TIB,c and RIB,c values are derived from CA_n3-n8-n78 and are given in the tables below.

Table 6.165.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8_n3-n78 | 8 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 6.165.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8_n3-n78 | 8 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 6.165.5	MSD

Based on co-existence studies MSD need to be defined. Values from CA_n3-n8-n78 are reused.

Table 6.165.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_8_n3-n78 | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n78 | 3550 | 10 | 50 | 3550 | 16.1 | TDD | IMD3 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n78 | 3370 | 10 | 50 | 3370 | 4.5 | TDD | IMD5 |
|  | 8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | 15.7 | FDD | IMD3 |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | TDD | N/A |

## 6.166	DC_20_n1-n67

### 6.166.1	Operating bands for DC

Table 6.166.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n1-n67 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n67 | N/A |  |  | 738 MHz |  | 758 MHz | SDL |

### 6.166.2	Channel bandwidths per operating band for DC

Table 6.166.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n1A-n67A | DC_20A_n1A | 20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 90 |
|  |  | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  | n67 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

### 6.166.3	Co-existence studies

Based on co-existence studies there is no IMD impact from UL 20_n1 into DL n67.

### 6.166.4	∆TIB and ∆RIB values

For DC_20_n1-n67, the TIB,c and RIB,c values are derived from DC_1-20_n28 and are given in the tables below.

Table 6.166.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n1-n67 | 20 | 0.6 |
|  | n1 | 0.5 |
|  | n67 | 0.6 |

Table 6.166.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n1-n67 | 20 | 0 |
|  | n1 | 0.2 |
|  | n67 | 0.2 |

### 6.166.5	MSD

Based on co-existence studies no MSD need to be defined.

## 6.167	DC_20_n3-n67

### 6.167.1	Operating bands for DC

Table 6.167.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n3-n67 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n3 | 1805 MHz | – | 1880 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n67 | N/A |  |  | 738 MHz |  | 758 MHz | SDL |

### 6.167.2	Channel bandwidths per operating band for DC

Table 6.167.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n3A-n67A | DC_20A_n3A | 20 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 80 |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  |  |
|  |  | n67 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

### 6.167.3	Co-existence studies

Based on co-existence studies there is IMD4 impact from UL 20_n3 into DL n67.

### 6.167.4	∆TIB and ∆RIB values

For DC_20_n3-n67, the TIB,c and RIB,c values are derived from DC_3-20_n28 and are given in the tables below.

Table 6.167.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n3-n67 | 20 | 0.5 |
|  | n3 | 0.3 |
|  | n67 | 0.5 |

Table 6.167.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n3-n67 | 20 | 0.1 |
|  | n3 | 0 |
|  | n67 | 0.1 |

### 6.167.5	MSD

As can be seen in the co-existence studies there is a need to define MSD. MSD value from R4-2200794.

Table 6.167.5-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_20_n3-n67 | 20 | 837 | 5 | 25 | 796 | N/A | FDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | FDD | N/A |
|  | n67 | N/A | 5 | 25 | 746 | 9.4 | FDD | IMD4 |

## 6.168	DC_3_n41-n258

### 6.168.1	Operating bands for DC

Table 6.168.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n41-n258 | 3 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.168.2	Configuration for DC

Table 6.168.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n41A-n258A | DC_3A_n41A DC_3A_n258A |

### 6.168.3	Co-existence studies

- For combination of DC_3_n41, the co-existence studies have been covered in TR37.716-11-11, where no impact on band n258.

- For combination of DC_3_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

### 6.168.4	∆TIB and ∆RIB values

For the DC conbination of DC_3_n41-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 3 and n41 are the same with the values of the constiture DC_3_n41 defined in TS38.101-3.

### 6.168.5	MSD

No additional MSD requirement is needed.

## 6.169	DC_8_n41-n258

### 6.169.1	Operating bands for DC

Table 6.169.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n41-n258 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.169.2	Configuration for DC

Table 6.169.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A_n41A-n258A | DC_8A_n41A DC_8A_n258A |

### 6.169.3	Co-existence studies

- For combination of DC_8_n41, the co-existence studies have been covered in TR37.716-11-11, where no impact on band n258.

- For combination of DC_8_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency seperation.

### 6.169.4	∆TIB and ∆RIB values

For the DC conbination of DC_8_n41-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 8 and n41 are the same with the values of the constiture DC_8_n41 defined in TS38.101-3.

### 6.169.5	MSD

No additional MSD requirement is needed.

## 6.170	DC_41_n1-n3

### 6.170.1	Operating bands for DC

Table 6.170.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41_n1-n3 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |

### 6.170.2	Channel bandwidths per operating band for DC

Table 6.170.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Channel bandwidth [MHz] | Maximum aggregated bandwidth For DL [MHz] |
| DC_41A_n1A-n3A | DC_41A_n1A DC_41A_n3A | 41 | 5, 10, 15, 20 | 70 |
|  |  | n1 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
| DC_41C_n1A-n3A | DC_41A_n1A DC_41A_n3A | 41 | CA_41C | 90 |
|  |  | n1 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |

### 6.170.3	Co-existence studies

Based on the co-existence studies of DC_41A-n1A and DC_41A-n3A, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 6.170.4	∆TIB and ∆RIB values

For DC_41_n1-n3, the TIB,c and RIB,c values are given in the tables below.

Table 6.170.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_41_n1-n3 | 41 | 0.53 |
|  |  | 0.84 |
|  | n1 | 0.5 |
|  | n3 | 0.5 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

Table 6.170.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_41_n1-n3 | 41 | 03 |
|  |  | 0.54 |
|  | n1 | 0 |
|  | n3 | 0 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

### 6.170.5	MSD

As mentioned in 6.170.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 6.171	DC_1_n3-n75

### 6.171.1	Operating bands for DC

Table 6.171.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A_n3A-n75A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

### 6.171.2	Channel bandwidths per operating band for DC

Table 6.171.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A_n3A-n75A | DC_1A_n3A | 1 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | n3 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

### 6.171.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_1A_n3A-n75A with dual uplink on DC_1A_n3A were calculated and listed in Table 6.171.3-1.

Table 6.171.3-1: Harmonic and IMD analysis for DC_1_n3-n75 with UL on DC_1_n3

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 1710 | 1785 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 3840 – 3960 |  |  | 3420 – 3570 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 5760 – 5940 |  |  | 5130 – 5355 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 135 – 270 |  |  | 3630 – 3765 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2055 – 2250 |  |  | 1440 – 1650 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 5550 – 5745 |  |  | 5340 – 5550 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 1890 – 2010 |  |  | 1690 – 1805 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975 – 4230 |  |  | 3150 – 3435 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 270 – 540 |  |  | 7260 – 7530 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7470 – 7725 |  |  | 7050 – 7335 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4860 – 5220 |  |  | 5895 – 6210 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1170 – 1515 |  |  | 2190 – 2520 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8760 – 9120 |  |  | 9390 – 9705 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 – 9315 |  |  | - – 9510 |  |  |

Based on Table 6.171.3-1:

- 3rd and 5th order IMD generated by dual uplink of Band 1 + Band n3 may fall into own Rx of band n75 – it could reuse the MSD value of DC_1A-32A_n3A

- 3rd order IMD generated by dual uplink of Band 1 + Band n3 may fall into own Rx of band n1

### 6.171.4	∆TIB and ∆RIB values

For DC_1A_n3A-n75A, the TIB,c and RIB,c values can reuse the values of DC_1A-32A_n3A as given in the tables below.

Table 6.171.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n3-n75 | 1 | 0.5 |
|  | n3 | 0.5 |

Table 6.171.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n3-n75 | 1 | 0 |
|  | n3 | 0 |
|  | n75 | 0 |

### 6.171.5	MSD

Table 6.171.5-1 shows the required MSD.

| Table 6.1715-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n3A-n75A | n75 | N/A | 5 | 25 | 1480 | 15.2 | IMD34 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.172	DC_20_n7-n78

### 6.172.1	Operating bands for DC

Table 6.172.1-1: DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20A_n7A-n78A | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 6.172.2	Channel bandwidths per operating band for DC

Table 6.172.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_20A_n7A-n78A | DC_20A_n7A DC_20A_n78A | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n7 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.172.3	Co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_20A_n7A-n78A with dual uplink on DC_20A_n7A and DC_20A_n78A were calculated and listed in Table 6.172.3-1 and 6.172.3-2 respectively.

Table 6.172.3-1: Harmonic and IMD analysis for DC_20_n7-n78 with UL on DC_20_n7

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 2500 | 2570 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 5000 – 5140 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 7500 – 7710 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1638 – 1738 |  |  | 3332 – 3432 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 776 – 906 |  |  | 4138 – 4308 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4164 – 4294 |  |  | 5832 – 6002 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 782 – 912 |  |  | 2480 – 2590 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 – 86 |  |  | 6638 – 6878 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 3276 – 3476 |  |  | 6664 – 6864 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4996 – 5156 |  |  | 8332 – 8572 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9138 – 9448 |  |  | 758 – 948 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5776 – 6046 |  |  | 2414 – 2644 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10832 – 11142 |  |  | 5828 – 6018 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9164 – 9434 |  |  | 7496 – 7726 |  |  |

Table 6.172.3-2: Harmonic and IMD analysis for DC_20_n7-n78 with UL on DC_20_n78

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 |  | 3300 | 3800 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1664 – 1724 |  |  | 6600 – 7600 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2496 – 2586 |  |  | 9900 – 11400 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2438 – 2968 |  |  | 4132 – 4662 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 1576 – 2136 |  |  | 5738 – 6768 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4964 – 5524 |  |  | 7432 – 8462 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 732 – 962 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 714 – 1304 |  |  | 9038 – 10568 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 4876 – 5936 |  |  | 8264 – 9324 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5796 – 6386 |  |  | 10732 – 12262 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12338 – 14368 |  |  | 148 – 472 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8176 – 9736 |  |  | 4014 – 5104 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14032 – 16062 |  |  | 6628 – 7248 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11564 – 13124 |  |  | 9096 – 10186 |  |  |

Based on Table 6.172.3-1 and Table 6.172.3-2:

- 2nd order IMD generated by dual uplink of Band 20 + Band n7 may fall into own Rx of band n78 – it could reuse the MSD value of DC_20A_n41A-n78A

- 4th order IMD generated by dual uplink of Band 20 + Band n7 may fall into own Rx of band n78

- 3rd and 5th order IMD generated by dual uplink of Band 20 + Band n7 may fall into own Rx of band 20

- 5th order IMD generated by dual uplink of Band 20 + Band n7 may fall into own Rx of band n7

- 2nd order IMD generated by dual uplink of Band 20 + Band n78 may fall into own Rx of band n7 – it could reuse the MSD value of DC_7A-20A_n78A

- 2nd order IMD generated by dual uplink of Band 20 + Band n78 may fall into own Rx of band 20

- 4th order IMD generated by dual uplink of Band 20 + Band n78 may fall into own Rx of band n7 and band 20

### 6.172.4	∆TIB and ∆RIB values

For DC_20A_n7A-n78A, the TIB,c and RIB,c values can reuse the values of DC_7A-20A_n78A and DC_20A_n41A-n78A as given in the tables below

Table 6.172.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n7-n78 | 20 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.8 |

Table 6.172.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20_n7-n78 | n78 | 0.5 |

### 6.172.5	Reference sensitivity exceptions

Table 6.172.5-1 shows the required MSD.

| Table 6.172.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A_n7A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n7 | 2555 | 5 | 25 | 2675 | 30.8 | IMD2 |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
|  | 20 | 850 | 5 | 25 | 809 | N/A | N/A |
|  | n7 | 2550 | 10 | 50 | 2675 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | 28.8 | IMD21 |
| NOTE 1: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

## 6.173	DC_3_n5-n40

### 6.173.1	Operating bands for DC

Table 6.173.1-1: EN-DC band combination (three bands)

| EN-DC Band | E-UTRA Band | NR CA Band |
| --- | --- | --- |
| DC_3_n5-n40 | 3 | CA_n5-n40 |

### 6.173.2	Inter-band DC Configurations

Table 6.173.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A_n5A-n40A | DC_3A_n5A DC_3A_n40A |

### 6.173.3	Co-existence studies

Based on the co-existense studies, No IMD of dual Tx falls into the third band Rx.

### 6.173.4	∆TIB and ∆RIB values

For DC_3_n5-n40, the TIB,c and RIB,c values are given in the tables below, reuse from CA_3-5-40..

Table 6.173.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3_n5-n40 | 3 | 0.5 |
|  | n5 | 0.3 |
|  | n40 | 0.5 |

Table 6.173.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3_n5-n40 | 3 | 0 |
|  | n5 | 0 |
|  | n40 | 0 |

### 6.173.5	MSD

As analysed in 6.173.3, there is no need to specify additional MSD requirement.

## 6.174	DC_8_n79-n258

### 6.174.1	Operating bands for DC

Table 6.174.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n79-n258 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

### 6.174.2	Configuration for DC

Table 6.174.2-1:  Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A_n79A-n258A | DC_8A_n79A DC_8A_n258A |

### 6.174.3	Co-existence studies

- For combination of DC_8_n79, the co-existence studies have been covered in TR37.716-11-11, where no impact on band n258.

- For combination of DC_8_n258, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency separation.

### 6.174.4	∆TIB and ∆RIB values

For the DC conbination of DC_8_n79-n258, the TIB,c and RIB,c for band n258 equals to 0, and the TIB,c and RIB,c for band 8 and n79 are the same with the values of the constiture DC_8_n79 defined in TS38.101-3.

### 6.174.5	MSD

No additional MSD requirement is needed.

## 6.175	DC_n8-n77_1

### 6.175.1		Operating bands for DC

Table 6.175.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_n8-n77_1 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |

### 6.175.2		Channel bandwidths per operating band for DC

Table 6.175.2-1: Supported bandwidths per DC inter-band NR 2DL/1UL + one LTE band

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_n8A-n77A_1A |  |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  |  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_n8A-n77(2A)_1A |  |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 6.175.3		Co-existence studies

- Similar to DC_1_n8-n77 in TR 37.717-11-21

### 6.175.4	∆TIB and ∆RIB values

Similar to DC_1_n8-n77 in TR 37.717-11-21

### 6.175.5	MSD

Similar to DC_1_n8-n77 in TR 37.717-11-21

## 6.176	 DC_1_n8-n77

### 6.176.1		Operating bands for DC

Table 6.176.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n8-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 6.176.2		Channel bandwidths per operating band for DC

Table 6.176.2-1: Supported bandwidths per DC inter-band one LTE + NR 2DL/1UL bands

| NR DC and E-UTRA Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_1A_n8A-n77A | DC_1A_n8A DC_1A_n77A | 1 | - | 170 |
|  |  | n8 | CA_n8A-n77A in TS 38.101-1 |  |
|  |  | n7 |  |  |
| DC_1A_n8A-n77(2A) |  | 1 | - | 220 |
|  |  | n8 | CA_n8A-n77(2A) in TS 38.101-1 |  |
|  |  | n7 |  |  |

### 6.176.3		Co-existence studies

The harmonic and IMD products caused by UL DC_1A_n8A are shown below.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1100 | 1005 | 2800 | 2895 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2925 | 3080 | 220 | 90 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4720 | 4875 | 3680 | 3810 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4845 | 5060 | 660 | 825 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6640 | 6855 | 4560 | 4725 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2010 | 2200 | 5600 | 5790 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1740 | 1540 | 7040 | 6765 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1095 | 1320 | 4180 | 3930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5440 | 5640 | 8560 | 8835 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6480 | 6705 | 7520 | 7770 |

- 2nd  harmonic of n1 and 4th harmonic of n8 could fall on n77 receiver band.

- IM3 and IM5 could fall on n77 receiver band.

The harmonic and IMD products caused by UL DC_1A_n77A are shown below.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 16500 | 21000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1320 | 2280 | 5220 | 6180 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 360 | 660 | 4620 | 6480 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 | 8160 | 8520 | 10380 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1560 | 2640 | 7920 | 10680 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 | 10140 | 11820 | 14580 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4560 | 2640 | 10440 | 12360 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14880 | 11220 | 4620 | 3480 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8760 | 5940 | 660 | 2640 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 | 18780 | 10980 | 12120 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 | 16560 | 12360 | 14340 |

- IM5 could fall on n8 receiver band

### 6.176.4	∆TIB and ∆RIB values

Table 6.176.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1_n8-n77 | B1 | 0.3 |
|  | n8 | 0.6 |
|  | n77 | 0.8 |

Table 6.176.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1_n8-n77 | B1 | 0 |
|  | n8 | 0.2 |
|  | n77 | 0.5 |

### 6.176.5	MSD

As mentioned above, IMD3 and IMD5 of B1 and n8 to n77 Rx need to be addressed for REFSENS relaxation. The proposed values are given in the following table

IMD5 of B1 and n77 to n8 Rx is addressed at TR 37.716-21-11 and it is repeated below.

Table 6.x.5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order | Single UL allowed |
| DC_1A_n8A-n77A DC_1A_n8A-n77(2A) | 1 | 1955 | 5 | 25 | 2145 | N/A | FDD | N/A | No |
|  | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A |  |
|  | n77 | 3410 | 10 | 50 | 3410 | 1.5 | TDD | IMD5 |  |
| DC_1A_n8A-n77A DC_1A_n8A-n77(2A) | n8 | 910 | 5 | 25 | 955 | N/A | FDD | N/A | No |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |  |
|  | n77 | 3960 | 10 | 50 | 3960 | 8.8 | TDD | IMD3 |  |
| DC_1A_n8A-n77A DC_1A_n8A-n77(2A) | 1 | 1955 | 5 | 25 | 2145 | N/A | FDD | N/A | No |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | TDD | N/A |  |
|  | n8 | 910 | 5 | 25 | 955 | 3.3 | FDD | IMD5 |  |

For the MSD due to the 2nd harmonic of B1 to n77 Rx, please refer to DC_1_n77

For the MSD due to the 4th harmonic of n8 to n77 Rx, please refer to DC_n77_8 in TR 37.717-11-11

## 6.177	DC_n77-n257_1

### 6.177.1		Operating bands for DC

Table 6.177.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_n77-n257_1 | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
|  | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |

### 6.177.2		Channel bandwidths per operating band for DC

Table 6.177.2-1: Supported bandwidths per DC inter-band NR 2DL/1UL + one LTE band

| NR DC and E-UTRA Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_n77A-n257A_1A DC_n77A-n257G_1A DC_n77A-n257H_1A DC_n77A-n257I_1A DC_n77A-n257J_1A DC_n77A-n257K_1A DC_n77A-n257L_1A DC_n77A-n257M_1A | DC_n77A_1A DC_n257A_1A | 1 | - | 250 350 450 550 650 750 850 |
|  |  | n77 | CA_n77A-n257A/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_n77(2A)-n257A_1A DC_n77(2A)-n257G_1A DC_n77(2A)-n257H_1A DC_n77(2A)-n257I_1A DC_n77(2A)-n257J_1A DC_n77(2A)-n257K_1A DC_n77(2A)-n257L_1A DC_n77(2A)-n257M_1A |  | 1 | - | 550 350 450 550 650 750 850 950 |
|  |  | n77 | CA_n77(2A)-n257A/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |

### 6.177.3		Co-existence studies

- For combination of DC_n77A_1A, the co-existence studies have been covered in TR 37.863-01-01 as DC_1A_n77A.

- For combination of DC_n257A_1A, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency separation.

### 6.177.4	∆TIB and ∆RIB values

Similar to DC_1_n77-n258 in TR 37.716-21-21

### 6.177.5	MSD

No additional MSD requirement is needed.

## 6.178	DC_n77-n257_3

### 6.178.1		Operating bands for DC

Table 6.178.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_n77-n257_3 | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |

### 6.178.2		Channel bandwidths per operating band for DC

Table 6.178.2-1: Supported bandwidths per DC inter-band NR 2DL/1UL + one LTE band

| NR DC and E-UTRA Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_n77A-n257A_3A DC_n77A-n257G_3A DC_n77A-n257H_3A DC_n77A-n257I_3A DC_n77A-n257J_3A DC_n77A-n257K_3A DC_n77A-n257L_3A DC_n77A-n257M_3A | DC_n77A_3A DC_n257A_3A | 3 | - | 550 250 350 450 550 650 750 850 |
|  |  | n77 | CA_n77A-n257A/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_n77(2A)-n257A_3A DC_n77(2A)-n257G_3A DC_n77(2A)-n257H_3A DC_n77(2A)-n257I_3A DC_n77(2A)-n257J_3A DC_n77 (2A) -n257K_3A DC_n77(2A)-n257L_3A DC_n77(2A)-n257M_3A |  | 3 | - | 650 350 450 550 650 750 850 950 |
|  |  | n77 | CA_n77(2A)-n257A/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |

### 6.178.3		Co-existence studies

- For combination of DC_n77A_3A, the co-existence studies have been covered in TR 37.863-01-01 as DC_3A_n77A.

- For combination of DC_n257A_3A, it is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency separation.

### 6.178.4	∆TIB and ∆RIB values

Similar to DC_3_n77-n258 in TR 37.716-21-21

### 6.178.5	MSD

No additional MSD requirement is needed.

# 7	Dual Connectivity band combinations of LTE 2 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

## 7.1	DC_3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-7-7_n1-n257, DC_3-3-7-7_n1-n257

### 7.1.1	Operating bands for DC

Table 7.1.1-1: DC band combination of one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n1-n257 DC_3-3-7_n1-n257 DC_3-7-7_n1-n257 DC_3-3-7-7_n1-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 7.1.2	Channel bandwidths per operating band for DC

Table 7.1.2-1: Supported bandwidths per DC one LTE band + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A-7A_n1A-n257D DC_3A-7A_n1A-n257E DC_3A-7A_n1A-n257F DC_3A-7A_n1A-n257G DC_3A-7A_n1A-n257H DC_3A-7A_n1A-n257I DC_3A-7A_n1A-n257J DC_3A-7A_n1A-n257K DC_3A-7A_n1A-n257L DC_3A-7A_n1A-n257M | DC_3A_n1A DC_3A_n257A DC_7A_n1A DC_7A_n257A | 3 | See CA_3A-7A in TS 36.101 | 460 660 860 260 360 460 560 660 760 860 |
|  |  | 7 |  |  |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_3A-3A-7A_n1A-n257D DC_3A-3A-7A_n1A-n257E DC_3A-3A-7A_n1A-n257F DC_3A-3A-7A_n1A-n257G DC_3A-3A-7A_n1A-n257H DC_3A-3A-7A_n1A-n257I DC_3A-3A-7A_n1A-n257J DC_3A-3A-7A_n1A-n257K DC_3A-3A-7A_n1A-n257L DC_3A-3A-7A_n1A-n257M | DC_3A_n1A DC_3A_n257A DC_7A_n1A DC_7A_n257A | 3 | See CA_3A-3A-7A in TS 36.101 | 480 680 880 280 380 480 580 680 780 880 |
|  |  | 7 |  |  |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_3A-7A-7A_n1A-n257D DC_3A-7A-7A_n1A-n257E DC_3A-7A-7A_n1A-n257F DC_3A-7A-7A_n1A-n257G DC_3A-7A-7A_n1A-n257H DC_3A-7A-7A_n1A-n257I DC_3A-7A-7A_n1A-n257J DC_3A-7A-7A_n1A-n257K DC_3A-7A-7A_n1A-n257L DC_3A-7A-7A_n1A-n257M | DC_3A_n1A DC_3A_n257A DC_7A_n1A DC_7A_n257A | 3 | See CA_3A-7A-7A in TS 36.101 | 480 680 880 280 380 480 580 680 780 880 |
|  |  | 7 |  |  |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_3A-3A-7A-7A_n1A-n257D DC_3A-3A-7A-7A_n1A-n257E DC_3A-3A-7A-7A_n1A-n257F DC_3A-3A-7A-7A_n1A-n257G DC_3A-3A-7A-7A_n1A-n257H DC_3A-3A-7A-7A_n1A-n257I DC_3A-3A-7A-7A_n1A-n257J DC_3A-3A-7A-7A_n1A-n257K DC_3A-3A-7A-7A_n1A-n257L DC_3A-3A-7A-7A_n1A-n257M | DC_3A_n1A DC_3A_n257A DC_7A_n1A DC_7A_n257A | 3 | See CA_3A-3A-7A-7A in TS 36.101 | 500 700 900 300 400 500 600 700 800 900 |
|  |  | 7 |  |  |
|  |  | n1 | See CA_n1A-n257D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |

### 7.1.3	Co-existence studies

Based on co-existence studies of DC_3A_n1A and DC_3A_n257A specified in TR 37.716-11-11, TR 37.863-01-01, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.1.4	∆TIB and ∆RIB values

For DC_3-7_n1-n257, DC_3-3-7_n1-n257, DC_3-7-7_n1-n257, DC_3-3-7-7_n1-n257, TIB,c and RIB,c values are already mentioned in 37.716-21-21 under the same band combinations with n257A.

### 7.1.5	MSD

No additional MSD requirement is needed.

## 7.2	DC_1-42_n28-n77

### 7.2.1	Operating bands for DC

Table 7.2.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-42_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.2.2	Channel bandwidths per operating band for DC

Table 7.2.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-42A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-42A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-42C_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.2.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-42_n28, DC_1-42_n77, DC_1_n28-n77 and DC_42_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.2.4	∆TIB and ∆RIB values

For DC_1-42_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n28-n77 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 7.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-42_n28-n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

### 7.2.5	MSD

As mentioned in 7.2.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.3	DC_3-42_n28-n77

### 7.3.1	Operating bands for DC

Table 7.3.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n28-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.3.2	Channel bandwidths per operating band for DC

Table 7.3.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_3A-42A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_42A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_3A-42A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_42A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-42C_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_3A-42C_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.3.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_3-42_n28, DC_3-42_n77, DC_3_n28-n77 and DC_42_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.3.4	∆TIB and ∆RIB values

For DC_3-42_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n28-n77 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 7.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n28-n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

### 7.3.5	MSD

As mentioned in 7.3.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.4	DC_8-42_n28-n77

### 7.4.1	Operating bands for DC

Table 7.4.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-42_n28-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.4.2	Channel bandwidths per operating band for DC

Table 7.4.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-42A_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A-42A_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 250 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A-42C_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A-42C_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.4.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-42_n28, DC_8-42_n77, DC_8_n28-n77 and DC_42_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.4.4	∆TIB and ∆RIB values

For DC_8-42_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-42_n28-n77 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 7.4.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-42_n28-n77 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

### 7.4.5	MSD

As mentioned in 7.4.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.5	DC_1-3_n28-n41

### 7.5.1	Operating bands for DC

Table 7.5.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A_n28A-n41A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 7.5.2	Channel bandwidths per operating band for DC

Table 7.5.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.5.3	Co-existence studies

Co-existence studies of DC_1A-3A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

### 7.5.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_1-3_n28-n41 are given in the tables below reusing the values for DC_ DC_1-3-41_n28.

Table 7.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n28-n41 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n28 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.5.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n28-n41 | 1 | 0 |
|  | 3 | 0 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.5.5	MSD

All MSD requirements forDC_1-3_n28-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.6	DC_2-48_n48-n66

### 7.6.1	Operating bands for DC

Table 7.6.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-48_n48-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 7.6.2	Configuration for DC

Table 7.6.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-48A_n48A-n66A | DC_2A_n48A DC_2A_n66A DC_48A_n66A | CA_2A-48A | CA_n48A-n66A |

### 7.6.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 7.6.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48_n48-n66 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n48 | 0.8 |
|  | n66 | 0.6 |

Table 7.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-48_n48-n66 | 2 | 0.3 |
|  | 48 | 0.4 |
|  | n48 | 0.4 |
|  | n66 | 0.3 |

### 7.6.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.7	DC_48-66_n25-n48

### 7.7.1	Operating bands for DC

Table 7.7.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_48-66_n25-n48 | 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

### 7.7.2	Configuration for DC

Table 7.7.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_48A-66A_n25A-n48A | DC_48A_n25A DC_66A_n25A DC_66A_n48A | CA_48A-66A | CA_n25A-n48A |

### 7.7.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 7.7.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_48-66_n25-n48 | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n25 | 0.6 |
|  | n48 | 0.8 |

Table 7.7.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_48-66_n25-n48 | 48 | 0.4 |
|  | 66 | 0.3 |
|  | n25 | 0.3 |
|  | n48 | 0.4 |

### 7.7.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.8	DC_1-8_n40-n78

### 7.8.1	Operating bands for DC

Table 7.8.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n40-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.8.2	Configuration for DC

Table 7.8.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-8A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_8A_n40A DC_8A_n78A | CA_1A-8A | CA_n40A-n78A |

### 7.8.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 7.8.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n40-n78 | 1 | 0.5 |
|  | 8 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 7.8.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n40-n78 | 1 | 0 |
|  | 8 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

### 7.8.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.9	DC_3-7_n1-n40

### 7.9.1	Operating bands for DC

Table 7.9.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n1-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 7.9.2	Configuration for DC

Table 7.9.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_7A_n1A DC_7A_n40A | CA_3A-7A | CA_n1A-n40A |

### 7.9.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 7.9.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n1-n40 | 3 | 0.6 |
|  | 7 | 0.8 |
|  | n1 | 0.6 |
|  | n40 | 0.9 |

Table 7.9.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n1-n40 | 3 | 0 |
|  | 7 | 0.3 |
|  | n1 | 0 |
|  | n40 | 0.8 |

### 7.9.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.10	DC_3-8_n40-n78

### 7.10.1	Operating bands for DC

Table 7.10.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n40-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.10.2	Configuration for DC

Table 7.10.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-8A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_8A_n40A DC_8A_n78A | CA_3A-8A | CA_n40A-n78A |

### 7.10.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 7.10.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n40-n78 | 3 | 0.6 |
|  | 8 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 7.10.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8_n40-n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

### 7.10.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.11	DC_3-28_n1-n40

### 7.11.1	Operating bands for DC

Table 7.11.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28_n1-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 7.11.2	Configuration for DC

Table 7.11.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_28A_n1A DC_28A_n40A | CA_3A-28A | CA_n1A-n40A |

### 7.11.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 7.11.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n1-n40 | 3 | 0.5 |
|  | 28 | 0.6 |
|  | n1 | 0.5 |
|  | n40 | 0.5 |

Table 7.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n1-n40 | 3 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0 |

### 7.11.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.12	DC_7-8_n40-n78

### 7.12.1	Operating bands for DC

Table 7.12.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-8_n40-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.12.2	Configuration for DC

Table 7.12.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-8A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_8A_n40A DC_8A_n78A | CA_7A-8A | CA_n40A-n78A |

### 7.12.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 7.12.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n40-n78 | 7 | 0.5 |
|  | 8 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 7.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n40-n78 | 7 | 0 |
|  | 8 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

### 7.12.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.13	DC_7-28_n1-n40

### 7.13.1	Operating bands for DC

Table 7.13.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-28_n1-n40 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 7.13.2	Configuration for DC

Table 7.13.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-28A_n1A-n40A | DC_7A_n1A DC_7A_n40A DC_28A_n1A DC_28A_n40A | CA_7A-28A | CA_n1A-n40A |

### 7.13.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 7.13.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n1-n40 | 7 | 0.8 |
|  | 28 | 0.6 |
|  | n1 | 0.6 |
|  | n40 | 0.9 |

Table 7.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n1-n40 | 7 | 0.3 |
|  | 28 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0.8 |

### 7.13.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.14	DC_1-3_n41-n77

### 7.14.1	Operating bands for DC

Table 7.14.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3_n41-n77 | 1, 3 | n41, n77 |

### 7.14.2	Channel bandwidths per operating band for DC

Table 7.14.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A | 1 | See CA_1A-3A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.14.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.14.4	∆TIB and ∆RIB values

For DC_1-3_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n41-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |

Table 7.14.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n41-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n41 | 0 |
|  | n77 | 0.5 |

### 7.14.5	MSD

There are no additional MSD requirements for this band combination.

## 7.15	DC_1-3_n3-n41

### 7.15.1	Operating bands for DC

Table 7.15.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3_n3-n41 | 1, 3 | n3, n41 |

### 7.15.2	Channel bandwidths per operating band for DC

Table 7.15.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A1 DC_3A_n41A | 1 | See CA_1A-3A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.15.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.15.4	∆TIB and ∆RIB values

For DC_1-3_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3-n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.15.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3-n41 | 1 | 0 |
|  | 3 | 0 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.15.5	MSD

There are no additional MSD requirements for this band combination.

## 7.16	DC_1-3_n3-n77

### 7.16.1	Operating bands for DC

Table 7.16.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3_n3-n77 | 1, 3 | n3, n77 |

### 7.16.2	Channel bandwidths per operating band for DC

Table 7.16.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A1 DC_3A_n77A | 1 | See CA_1A-3A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.16.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.16.4	∆TIB and ∆RIB values

For DC_1-3_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 7.16.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 7.16.5	MSD

There are no additional MSD requirements for this band combination.

## 7.17	DC_1-3_n3-n78

### 7.17.1	Operating bands for DC

Table 7.17.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3_n3-n78 | 1, 3 | n3, n78 |

### 7.17.2	Channel bandwidths per operating band for DC

Table 7.17.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A1 DC_3A_n78A | 1 | See CA_1A-3A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.17.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.17.4	∆TIB and ∆RIB values

For DC_1-3_n3-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 7.17.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n3-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 7.17.5	MSD

There are no additional MSD requirements for this band combination.

## 7.18	DC_1-18_n3-n77

### 7.18.1	Operating bands for DC

Table 7.18.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-18_n3-n77 | 1, 18 | n3, n77 |

### 7.18.2	Channel bandwidths per operating band for DC

Table 7.18.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-18A_n3A-n77A | DC_1A_n3A DC_1A_n77A | 1 | See CA_1A-18A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.18.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.18.4	∆TIB and ∆RIB values

For DC_1-18_n3-n77, the TIB,c and RIB,c values have existed in TS 38.101-3.

### 7.18.5	MSD

There are no additional MSD requirements for this band combination.

## 7.19	DC_1-41_n3-n41

### 7.19.1	Operating bands for DC

Table 7.19.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-41_n3-n41 | 1, 41 | n3, n41 |

### 7.19.2	Channel bandwidths per operating band for DC

Table 7.19.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_41A_n3A | 1 | See CA_1A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.19.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.19.4	∆TIB and ∆RIB values

For DC_1-41_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3-n41 | 1 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.19.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n3-n41 | 1 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.19.5	MSD

There are no additional MSD requirements for this band combination.

## 7.20	DC_1-41_n3-n77

### 7.20.1	Operating bands for DC

Table 7.20.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-41_n3-n77 | 1, 41 | n3, n77 |

### 7.20.2	Channel bandwidths per operating band for DC

Table 7.20.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-41A_n3A-n77A | DC_1A_n3A DC_1A_n77A | 1 | See CA_1A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.20.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.20.4	∆TIB and ∆RIB values

For DC_1-41_n3-n77, the TIB,c and RIB,c values are already defined in TS 38.101-3.

### 7.20.5	MSD

There are no additional MSD requirements for this band combination.

## 7.21	DC_1-41_n3-n78

### 7.21.1	Operating bands for DC

Table 7.21.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-41_n3-n78 | 1, 41 | n3, n78 |

### 7.21.2	Channel bandwidths per operating band for DC

Table 7.21.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-41A_n3A-n78A | DC_1A_n3A DC_1A_n78A | 1 | See CA_1A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.21.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.21.4	∆TIB and ∆RIB values

For DC_1-41_n3-n78, the TIB,c and RIB,c values are already defined in TS 38.101-3.

### 7.21.5	MSD

There are no additional MSD requirements for this band combination.

## 7.22	DC_1-41_n41-n77

### 7.22.1	Operating bands for DC

Table 7.22.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-41_n41-n77 | 1, 41 | n41, n77 |

### 7.22.2	Channel bandwidths per operating band for DC

Table 7.22.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-41A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_41A_n77A | 1 | See CA_1A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.22.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.22.4	∆TIB and ∆RIB values

For DC_1-41_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n41-n77 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |

Table 7.22.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n41-n77 | 1 | 0 |
|  | 41 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

### 7.22.5	MSD

There are no additional MSD requirements for this band combination.

## 7.23	DC_1-41_n41-n78

### 7.23.1	Operating bands for DC

Table 7.23.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-41_n41-n78 | 1, 41 | n41, n78 |

### 7.23.2	Channel bandwidths per operating band for DC

Table 7.23.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-41A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_41A_n78A | 1 | See CA_1A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.23.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.23.4	∆TIB and ∆RIB values

For DC_1-41_n41-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n41-n78 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |

Table 7.23.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n41-n78 | 1 | 0 |
|  | 41 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

### 7.23.5	MSD

There are no additional MSD requirements for this band combination.

## 7.24	DC_3-18_n3-n77

### 7.24.1	Operating bands for DC

Table 7.24.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-18_n3-n77 | 3, 18 | n3, n77 |

### 7.24.2	Channel bandwidths per operating band for DC

Table 7.24.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-18A_n3A-n77A | DC_3A_n3A1 DC_3A_n77A DC_18A_n3A DC_18A_n77A | 3 | See CA_3A-18A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.24.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.24.4	∆TIB and ∆RIB values

For DC_3-18_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.24.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n3-n77 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 7.24.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n3-n77 | 3 | 0.2 |
|  | 18 | 0 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 7.24.5	MSD

There are no additional MSD requirements for this band combination.

## 7.25	DC_3-18_n3-n78

### 7.25.1	Operating bands for DC

Table 7.25.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-18_n3-n78 | 3, 18 | n3, n78 |

### 7.25.2	Channel bandwidths per operating band for DC

Table 7.25.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-18A_n3A-n78A | DC_3A_n3A DC_3A_n78A DC_18A_n3A DC_18A_n78A | 3 | See CA_3A-18A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.25.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.25.4	∆TIB and ∆RIB values

For DC_3-18_n3-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n3-n78 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 7.25.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n3-n78 | 3 | 0.2 |
|  | 18 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 7.25.5	MSD

There are no additional MSD requirements for this band combination.

## 7.26	DC_3-41_n3-n41

### 7.26.1	Operating bands for DC

Table 7.26.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-41_n3-n41 | 3, 41 | n3, n41 |

### 7.26.2	Channel bandwidths per operating band for DC

Table 7.26.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-41A_n3A-n41A | DC_3A_n3A1 DC_3A_n41A DC_41A_n3A | 3 | See CA_3A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.26.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.26.4	∆TIB and ∆RIB values

For DC_3-41_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3-n41 | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.26.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3-n41 | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.26.5	MSD

There are no additional MSD requirements for this band combination.

## 7.27	DC_3-41_n3-n77

### 7.27.1	Operating bands for DC

Table 7.27.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-41_n3-n77 | 3, 41 | n3, n77 |

### 7.27.2	Channel bandwidths per operating band for DC

Table 7.27.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-41A_n3A-n77A | DC_3A_n3A1 DC_3A_n77A DC_41A_n3A DC_41A_n77A | 3 | See CA_3A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41C_n3A-n77A | DC_3A_n3A1 DC_3A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A | 3 | See CA_3A-41C BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.27.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.27.4	∆TIB and ∆RIB values

For DC_3-41_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3-n77 | 3 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.27.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3-n77 | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.27.5	MSD

There are no additional MSD requirements for this band combination.

## 7.28	DC_3-41_n3-n78

### 7.28.1	Operating bands for DC

Table 7.28.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-41_n3-n78 | 3, 41 | n3, n78 |

### 7.28.2	Channel bandwidths per operating band for DC

Table 7.28.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-41A_n3A-n78A | DC_3A_n3A1 DC_3A_n78A DC_41A_n3A DC_41A_n78A | 3 | See CA_3A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3A-41C_n3A-n78A | DC_3A_n3A1 DC_3A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A | 3 | See CA_3A-41C BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.28.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.28.4	∆TIB and ∆RIB values

For DC_3-41_n3-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.28.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3-n78 | 3 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.28.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n3-n78 | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.28.5	MSD

There are no additional MSD requirements for this band combination.

## 7.29	DC_3-41_n41-n77

### 7.29.1	Operating bands for DC

Table 7.29.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-41_n41-n77 | 3, 41 | n41, n77 |

### 7.29.2	Channel bandwidths per operating band for DC

Table 7.29.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-41A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_41A_n77A | 3 | See CA_3A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.29.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.29.4	∆TIB and ∆RIB values

For DC_3-41_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.29.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n41-n77 | 3 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n41 | 0.31/0.82 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.29.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n41-n77 | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n41 | 01/0.52 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.29.5	MSD

There are no additional MSD requirements for this band combination.

## 7.30	DC_3-41_n41-n78

### 7.30.1	Operating bands for DC

Table 7.30.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_3-41_n41-n78 | 3, 41 | n41, n78 |

### 7.30.2	Channel bandwidths per operating band for DC

Table 7.30.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_3A-41A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_41A_n78A | 3 | See CA_3A-41A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.30.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.30.4	∆TIB and ∆RIB values

For DC_3-41_n41-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n41-n78 | 3 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n41 | 0.31/0.82 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 7.30.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n41-n78 | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n41 | 01/0.52 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 7.30.5	MSD

There are no additional MSD requirements for this band combination.

## 7.31	DC_2-7_n38-n66

### 7.31.1	Operating bands for DC

Table 7.31.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2-7_n38-n66 DC_2-7-7_n38-n66 | 2, 7 | n38, n66 |

### 7.31.2	Channel bandwidths per operating band for DC

Table 7.31.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_2A-7A_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A | 2 | See CA_2A-7A BCS configurations in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | See CA_n38A-n66A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-7C_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A | 2 | See CA_2A-7C BCS configurations in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | See CA_n38A-n66A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_2A-7A-7A_n38A-n66A | DC_2A_n38A DC_2A_n66A DC_7A_n66A | 2 | See CA_2A-7A-7A BCS configurations in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | See CA_n38A-n66A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.31.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.31.4	∆TIB and ∆RIB values

For DC_2-7_n38-n66, the TIB,c and RIB,c values are given in the tables below.

Table 7.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n38-n66 DC_2-7-7_n38-n66 | 2 | 0.5 |
|  | n66 | 0.5 |

Table 7.31.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n38-n66 DC_2-7-7_n38-n66 | 2 | 0.3 |
|  | n66 | 0.5 |

### 7.31.5	MSD

There are no additional MSD requirements for this band combination.

## 7.32	DC_7-66_n38-n78 and DC_7-7-66_n38-n78

### 7.32.1	Operating bands for DC

Table 7.32.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_7-66_n38-n78 DC_7-7-66_n38-n78 | 7, 66 | n38, n78 |

### 7.32.2	Channel bandwidths per operating band for DC

Table 7.32.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_7A-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A | 7 | See CA_7A-66A BCS configurations in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | See CA_n38A-n78A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7A-7A-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A | 7 | See CA_7A-7A-66A BCS configurations in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | See CA_n38A-n78A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_7C-66A_n38A-n78A | DC_66A_n38A DC_66A_n78A | 7 | See CA_7C-66A BCS configurations in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n38 | See CA_n38A-n78A BCS configurations in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.32.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.32.4	∆TIB and ∆RIB values

For DC_7-66_n38-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n38-n78 DC_7-7-66_n38-n78 | 66 | 0.6 |
|  | n78 | 0.8 |

Table 7.32.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n38-n78 DC_7-7-66_n38-n78 | 66 | 0.2 |
|  | n78 | 0.5 |

### 7.32.5	MSD

There are no additional MSD requirements for this band combination.

## 7.33	DC_1-8_n3-n77

### 7.33.1	Operating bands for DC

Table 7.33.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n3-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.33.2	Channel bandwidths per operating band for DC

Table 7.33.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.33.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8_n3, DC_1-8_n77, DC_1_n3-n77 and DC_8_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.33.4	∆TIB and ∆RIB values

For DC_1-8_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n3-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n3 | 0.8 |
|  | n77 | 0.8 |

Table 7.33.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n3-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 7.33.5	MSD

As mentioned in 7.33.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.34	DC_1-11_n3-n28

### 7.34.1	Operating bands for DC

Table 7.34.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-11_n3-n28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 7.34.2	Channel bandwidths per operating band for DC

Table 7.34.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_11A_n3A DC_11A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 80 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.34.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-11_n3, DC_1-11_n28, DC_1_n3-n28 and DC_11_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.34.4	∆TIB and ∆RIB values

For DC_1-11_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 7.34.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n3-n28 | 1 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |

Table 7.34.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n3-n28 | 1 | 0 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |

### 7.34.5	MSD

As mentioned in 7.34.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.35	DC_8-11_n3-n28

### 7.35.1	Operating bands for DC

Table 7.35.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-11_n3-n28 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 7.35.2	Channel bandwidths per operating band for DC

Table 7.35.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-11A_n3A-n28A | DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 70 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.35.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-11_n3, DC_8-11_n28, DC_8_n3-n28 and DC_11_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.35.4	∆TIB and ∆RIB values

For DC_8-11_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 7.35.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-11_n3-n28 | 8 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |

Table 7.35.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-11_n3-n28 | 8 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |

### 7.35.5	MSD

As mentioned in 7.35.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.36	DC_1-18_n28-n41

### 7.36.1	Operating bands for DC

Table 7.36.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n28A-n41A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 7.36.2	Channel bandwidths per operating band for DC

Table 7.36.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_18A_n28A DC_18A_n41A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.36.3	Co-existence studies

Co-existence studies of DC_1A-18A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

### 7.36.4	∆TIB and ∆RIB values

For DC_1-18_n28-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.36.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28-n41 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n28 | 0.5 |
|  | n41 | 0.31 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

Table 7.36.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28-n41 | 1 | 0 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n41 | 01 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

### 7.36.5	MSD

All MSD requirements forDC_1-18_n28-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.37	DC_1-18_n28-n77

### 7.37.1	Operating bands for DC

Table 7.37.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n28A-n77A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.37.2	Channel bandwidths per operating band for DC

Table 7.37.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_18A_n28A DC_18A_n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.37.3	Co-existence studies

Co-existence studies of DC_1A-18A_n28A-n77A and protected bands are already covered in the constituent fall-back modes.

### 7.37.4	∆TIB and ∆RIB values

For DC_1-18_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.37.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28-n77 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 7.37.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28-n77 | 1 | 0 |
|  | 18 | 0 |
|  | n28 | 0 |
|  | n77 | 0.5 |

### 7.37.5	MSD

All MSD requirements forDC_1-18_n28-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.38	DC_1-18_n28-n78

### 7.38.1	Operating bands for DC

Table 7.38.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.38.2	Channel bandwidths per operating band for DC

Table 7.38.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_18A_n28A DC_18A_n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.38.3	Co-existence studies

Co-existence studies of DC_1A-18A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.38.4	∆TIB and ∆RIB values

For DC_1-18_n28-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.38.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28-n78 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 7.38.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n28-n78 | 1 | 0 |
|  | 18 | 0 |
|  | n28 | 0 |
|  | n78 | 0.5 |

### 7.38.5	MSD

All MSD requirements forDC_1-18_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.39	DC_1-18_n3-n41

### 7.39.1	Operating bands for DC

Table 7.39.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n3A-n41A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 7.39.2	Channel bandwidths per operating band for DC

Table 7.39.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_18A_n3A DC_18A_n41A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 175 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.39.3	Co-existence studies

Co-existence studies of DC_1A-18A_n3A-n41A and protected bands are already covered in the constituent fall-back modes.

### 7.39.4	∆TIB and ∆RIB values

For DC_1-18_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.39.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n3-n41 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | n3 | 0.3 |
|  | n41 | 0.31 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

Table 7.39.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n3-n41 | 1 | 0 |
|  | 18 | 0 |
|  | n3 | 0 |
|  | n41 | 01 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

### 7.39.5	MSD

All MSD requirements forDC_1-18_n3-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.40	DC_1-18_n41-n77

### 7.40.1	Operating bands for DC

Table 7.40.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n41A-n77A | 1 | 1920MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.40.2	Channel bandwidths per operating band for DC

Table 7.40.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_18A_n41A DC_18A_n77A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 235 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.40.3	Co-existence studies

Co-existence studies of DC_1A-18A_n41A-n77A and protected bands are already covered in the constituent fall-back modes.

### 7.40.4	∆TIB and ∆RIB values

For DC_1-18_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.40.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n41-n77 | 1 | 0.6 |
|  | 18 | 0.3 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |

Table 7.40.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n41-n77 | 1 | 0.2 |
|  | 18 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

### 7.40.5	MSD

All MSD requirements forDC_1-18_n41-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.41	DC_1-18_n41-n78

### 7.41.1	Operating bands for DC

Table 7.41.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-18A_n41A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.41.2	Channel bandwidths per operating band for DC

Table 7.41.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-18A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_18A_n41A DC_18A_n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 235 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.41.3	Co-existence studies

Co-existence studies of DC_1A-18A_n41A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.41.4	∆TIB and ∆RIB values

For DC_1-18_n41-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n41-n78 | 1 | 0.5 |
|  | 18 | 0.3 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

Table 7.41.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-18_n41-n78 | 1 | 0 |
|  | 18 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

### 7.41.5	MSD

All MSD requirements forDC_1-18_n41-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.42	DC_1-41_n28-n41

### 7.42.1	Operating bands for DC

Table 7.42.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-41A_n28A-n41A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 7.42.2	Channel bandwidths per operating band for DC

Table 7.42.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-41A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_41A_n28A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.42.3	Co-existence studies

Co-existence studies of DC_1A-41A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

### 7.42.4	∆TIB and ∆RIB values

For DC_1-41_n28-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n28-n41 | 1 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

Table 7.42.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-41_n28-n41 | 1 | 0 |
|  | 41 | 01/0.52 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

### 7.42.5	MSD

All MSD requirements forDC_1-41_n28-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.43	DC_3-18_n28-n41

### 7.43.1	Operating bands for DC

Table 7.43.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-18A_n28A-n41A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 7.43.2	Channel bandwidths per operating band for DC

Table 7.43.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-18A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_18A_n28A DC_18A_n41A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.43.3	Co-existence studies

Co-existence studies of DC_3A-18A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

### 7.43.4	∆TIB and ∆RIB values

For DC_3-18_n28-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n28-n41 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n28 | 0.5 |
|  | n41 | 0.31 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

Table 7.43.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n28-n41 | 3 | 0.2 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n41 | 01 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

### 7.43.5	MSD

All MSD requirements forDC_3-18_n28-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.44	DC_3-18_n28-n77

### 7.44.1	Operating bands for DC

Table 7.44.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-18A_n28A-n77A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.44.2	Channel bandwidths per operating band for DC

Table 7.44.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-18A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.44.3	Co-existence studies

Co-existence studies of DC_3A-18A_n28A-n77A and protected bands are already covered in the constituent fall-back modes.

### 7.44.4	∆TIB and ∆RIB values

For DC_3-18_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n28-n77 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 7.44.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n28-n77 | 3 | 0.2 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 7.44.5	MSD

All MSD requirements forDC_3-18_n28-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.45	DC_3-18_n28-n78

### 7.45.1	Operating bands for DC

Table 7.45.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-18A_n28A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.45.2	Channel bandwidths per operating band for DC

Table 7.45.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-18A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_18A_n28A DC_18A_n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 165 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.45.3	Co-existence studies

Co-existence studies of DC_3A-18A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.45.4	∆TIB and ∆RIB values

For DC_3-18_n28-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.45.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n28-n78 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 7.45.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n28-n78 | 3 | 0.2 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 7.45.5	MSD

All MSD requirements forDC_3-18_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.46	DC_3-18_n3-n41

### 7.46.1	Operating bands for DC

Table 7.46.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-18A_n3A-n41A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 7.46.2	Channel bandwidths per operating band for DC

Table 7.46.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-18A_n3A-n41A | DC_3A_n3A1 DC_3A_n41A DC_18A_n3A DC_18A_n41A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 175 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.46.3	Co-existence studies

Co-existence studies of DC_3A-18A_n3A-n41A and protected bands are already covered in the constituent fall-back modes.

### 7.46.4	∆TIB and ∆RIB values

For DC_3-18_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.46.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n3-n41 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n41 | 0.31 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

Table 7.46.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n3-n41 | 3 | 0.2 |
|  | 18 | 0 |
|  | n3 | 0.2 |
|  | n41 | 01 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

### 7.46.5	MSD

All MSD requirements forDC_3-18_n3-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.47	DC_3-18_n41-n77

### 7.47.1	Operating bands for DC

Table 7.47.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-18A_n41A-n77A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.47.2	Channel bandwidths per operating band for DC

Table 7.47.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-18A_n41A-n77A | DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 235 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.47.3	Co-existence studies

Co-existence studies of DC_3A-18A_n41A-n77A and protected bands are already covered in the constituent fall-back modes.

### 7.47.4	∆TIB and ∆RIB values

For DC_3-18_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.47.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n41-n77 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |

Table 7.47.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n41-n77 | 3 | 0.2 |
|  | 18 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

### 7.47.5	MSD

All MSD requirements forDC_3-18_n41-n77 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.48	DC_3-18_n41-n78

### 7.48.1	Operating bands for DC

Table 7.48.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-18A_n41A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.48.2	Channel bandwidths per operating band for DC

Table 7.48.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-18A_n41A-n78A | DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 235 |
|  |  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.48.3	Co-existence studies

Co-existence studies of DC_3A-18A_n41A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.48.4	∆TIB and ∆RIB values

For DC_3-18_n41-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.48.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n41-n78 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n41 | 0.6 |
|  | n78 | 0.8 |

Table 7.48.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-18_n41-n78 | 3 | 0.2 |
|  | 18 | 0 |
|  | n41 | 01 |
|  | n78 | 0.5 |

### 7.48.5	MSD

All MSD requirements forDC_3-18_n41-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.49	DC_3-41_n28-n41

### 7.49.1	Operating bands for DC

Table 7.49.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-41A_n28A-n41A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 7.49.2	Channel bandwidths per operating band for DC

Table 7.49.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-41A_n28A-n41A | DC_3A_n28A DC_3A_n41A DC_41A_n28A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.49.3	Co-existence studies

Co-existence studies of DC_3A-41A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

### 7.49.4	∆TIB and ∆RIB values

For DC_3-41_n28-n41, the TIB,c and RIB,c values are given in the tables below.

Table 7.49.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n28-n41 | 3 | 0.6 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

Table 7.49.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-41_n28-n41 | 3 | 0.2 |
|  | 41 | 01/0.52 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

### 7.49.5	MSD

All MSD requirements forDC_3-41_n28-n41 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.50	DC_3-19_n1-n77

### 7.50.1	Operating bands for DC

Table 7.50.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19_n1-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.50.2	Configuration for DC

Table 7.50.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-19A_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A | CA_3A-19A | CA_n1A-n77A |

### 7.50.3	Co-existence studies

Co-existence studies of DC_3-19_n1-n77 are already covered by the fallback configurations, DC_3-19_n1, DC_3-19_n77, DC_3_n1-n77 and DC_19_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.50.4	∆TIB and ∆RIB values

For DC_3-19_n1-n77, the TIB,c and RIB,c values are reused from DC_1-3-19_n77, and are given in the tables below.

Table 7.50.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n1-n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.50.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19_n1-n77 | 3 | 0.2 |
|  | 19 | 0 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

### 7.50.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.51	DC_3-19_n1-n78

### 7.51.1	Operating bands for DC

Table 7.51.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.51.2	Configuration for DC

Table 7.51.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-19A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A | CA_3A-19A | CA_n1A-n78A |

### 7.51.3	Co-existence studies

Co-existence studies of DC_3-19_n1-n78 are already covered by the fallback configurations, DC_3-19_n1, DC_3-19_n78, DC_3_n1-n78 and DC_19_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.51.4	∆TIB and ∆RIB values

For DC_3-19_n1-n78, the TIB,c and RIB,c values are reused from DC_1-3-19_n78, and are given in the tables below.

Table 7.51.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n1-n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.51.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19_n1-n78 | 3 | 0.2 |
|  | 19 | 0 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 7.51.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.52	DC_3-19_n1-n79

### 7.52.1	Operating bands for DC

Table 7.52.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19_n1-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.52.2	Configuration for DC

Table 7.52.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-19A_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A | CA_3A-19A | CA_n1A-n79A |

### 7.52.3	Co-existence studies

Co-existence studies of DC_3-19_n1-n79 are already covered by the fallback configurations, DC_3-19_n1, DC_3-19_n79, DC_3_n1-n79 and DC_19_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.52.4	∆TIB and ∆RIB values

For DC_3-19_n1-n79, the TIB,c and RIB,c values are reused from DC_1-3-19_n79, and are given in the tables below.

Table 7.52.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19_n1-n79 | 3 | 0.3 |
|  | 19 | 0.3 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 7.52.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19_n1-n79 | 3 | 0 |
|  | 19 | 0 |
|  | n1 | 0 |
|  | n79 | 0 |

### 7.52.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.53	DC_3-21_n1-n77

### 7.53.1	Operating bands for DC

Table 7.53.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21_n1-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.53.2	Configuration for DC

Table 7.53.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A | CA_3A-21A | CA_n1A-n77A |

### 7.53.3	Co-existence studies

Co-existence studies of DC_3-21_n1-n77 are already covered by the fallback configurations, DC_3-21_n1, DC_3-21_n77, DC_3_n1-n77 and DC_21_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.53.4	∆TIB and ∆RIB values

For DC_3-21_n1-n77, the TIB,c and RIB,c values are reused from DC_1-3-21_n77, and are given in the tables below.

Table 7.53.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n1-n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.53.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n1-n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

### 7.53.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.54	DC_3-21_n1-n78

### 7.54.1	Operating bands for DC

Table 7.54.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.54.2	Configuration for DC

Table 7.54.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A | CA_3A-21A | CA_n1A-n78A |

### 7.54.3	Co-existence studies

Co-existence studies of DC_3-21_n1-n78 are already covered by the fallback configurations, DC_3-21_n1, DC_3-21_n78, DC_3_n1-n78 and DC_21_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.54.4	∆TIB and ∆RIB values

For DC_3-21_n1-n78, the TIB,c and RIB,c values are reused from DC_1-3-21_n78, and are given in the tables below.

Table 7.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n1-n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.54.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n1-n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 7.54.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.55	DC_3-21_n1-n79

### 7.55.1	Operating bands for DC

Table 7.55.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21_n1-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.55.2	Configuration for DC

Table 7.55.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A | CA_3A-21A | CA_n1A-n79A |

### 7.55.3	Co-existence studies

Co-existence studies of DC_3-21_n1-n79 are already covered by the fallback configurations, DC_3-21_n1, DC_3-21_n79, DC_3_n1-n79 and DC_21_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.55.4	∆TIB and ∆RIB values

For DC_3-21_n1-n79, the TIB,c and RIB,c values are reused from DC_1-3-21_n79, and are given in the tables below.

Table 7.55.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n1-n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 7.55.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n1-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n1 | 0 |
|  | n79 | 0 |

### 7.55.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.56	DC_3-42_n1-n77

### 7.56.1	Operating bands for DC

Table 7.56.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n1-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.56.2	Configuration for DC

Table 7.56.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-42A_n1A-n77A | DC_3A_n1A DC_3A_n77A | CA_3A-42A | CA_n1A-n77A |
| DC_3A-42C_n1A-n77A | DC_3A_n1A DC_3A_n77A | CA_3A-42C | CA_n1A-n77A |

### 7.56.3	Co-existence studies

Co-existence studies of DC_3-42_n1-n77 are already covered by the fallback configurations, DC_3-42_n1, DC_3-42_n77 and DC_3_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.56.4	∆TIB and ∆RIB values

For DC_3-42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-3-42_n77, and are given in the tables below.

Table 7.56.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n1-n77 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.56.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n1-n77 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

### 7.56.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.57	DC_3-42_n1-n78

### 7.57.1	Operating bands for DC

Table 7.57.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.57.2	Configuration for DC

Table 7.57.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-42A_n1A-n78A | DC_3A_n1A DC_3A_n78A | CA_3A-42A | CA_n1A-n78A |
| DC_3A-42C_n1A-n78A | DC_3A_n1A DC_3A_n78A | CA_3A-42C | CA_n1A-n78A |

### 7.57.3	Co-existence studies

Co-existence studies of DC_3-42_n1-n78 are already covered by the fallback configurations, DC_3-42_n1, DC_3-42_n78 and DC_3_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.57.4	∆TIB and ∆RIB values

For DC_3-42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-3-42_n78, and are given in the tables below.

Table 7.57.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n1-n78 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.57.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n1-n78 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 7.57.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.58	DC_3-42_n1-n79

### 7.58.1	Operating bands for DC

Table 7.58.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-42_n1-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.58.2	Configuration for DC

Table 7.58.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-42A_n1A-n79A | DC_3A_n1A DC_3A_n79A | CA_3A-42A | CA_n1A-n79A |
| DC_3A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A | CA_3A-42C | CA_n1A-n79A |

### 7.58.3	Co-existence studies

Co-existence studies of DC_3-42_n1-n79 are already covered by the fallback configurations, DC_3-42_n1, DC_3-42_n79 and DC_3_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.58.4	∆TIB and ∆RIB values

For DC_3-42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-3-42_n79, and are given in the tables below.

Table 7.58.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-42_n1-n79 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n79 | 0 |

Table 7.58.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-42_n1-n79 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n79 | 0 |

### 7.58.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.59	DC_19-21_n1-n77

### 7.59.1	Operating bands for DC

Table 7.59.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21_n1-n77 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.59.2	Configuration for DC

Table 7.59.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-21A_n1A-n77A | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A | CA_19A-21A | CA_n1A-n77A |

### 7.59.3	Co-existence studies

Co-existence studies of DC_19-21_n1-n77 are already covered by the fallback configurations, DC_19-21_n1, DC_19-21_n77, DC_19_n1-n77 and DC_21_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.59.4	∆TIB and ∆RIB values

For DC_19-21_n1-n77, the TIB,c and RIB,c values are reused from DC_1-19-21_n77, and are given in the tables below.

Table 7.59.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n1-n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n1 | 0.3 |
|  | n77 | 0.8 |

Table 7.59.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21_n1-n77 | 19 | 0 |
|  | 21 | 0 |
|  | n1 | 0 |
|  | n77 | 0.5 |

### 7.59.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.60	DC_19-21_n1-n78

### 7.60.1	Operating bands for DC

Table 7.60.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21_n1-n78 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.60.2	Configuration for DC

Table 7.60.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-21A_n1A-n78A | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A | CA_19A-21A | CA_n1A-n78A |

### 7.60.3	Co-existence studies

Co-existence studies of DC_19-21_n1-n78 are already covered by the fallback configurations, DC_19-21_n1, DC_19-21_n78, DC_19_n1-n78 and DC_21_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.60.4	∆TIB and ∆RIB values

For DC_19-21_n1-n78, the TIB,c and RIB,c values are reused from DC_1-19-21_n78, and are given in the tables below.

Table 7.60.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n1-n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.60.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21_n1-n78 | 19 | 0 |
|  | 21 | 0 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 7.60.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.61	DC_19-21_n1-n79

### 7.61.1	Operating bands for DC

Table 7.61.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21_n1-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.61.2	Configuration for DC

Table 7.61.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-21A_n1A-n79A | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A | CA_19A-21A | CA_n1A-n79A |

### 7.61.3	Co-existence studies

Co-existence studies of DC_19-21_n1-n79 are already covered by the fallback configurations, DC_19-21_n1, DC_19-21_n79, DC_19_n1-n79 and DC_21_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.61.4	∆TIB and ∆RIB values

For DC_19-21_n1-n79, the TIB,c and RIB,c values are reused from DC_1-19-21_n79, and are given in the tables below.

Table 7.61.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21_n1-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 7.61.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21_n1-n79 | 19 | 0 |
|  | 21 | 0 |
|  | n1 | 0 |
|  | n79 | 0 |

### 7.61.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.62	DC_19-42_n1-n77

### 7.62.1	Operating bands for DC

Table 7.62.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-42_n1-n77 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.62.2	Configuration for DC

Table 7.62.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-42A_n1A-n77A | DC_19A_n1A DC_19A_n77A | CA_19A-42A | CA_n1A-n77A |
| DC_19A-42C_n1A-n77A | DC_19A_n1A DC_19A_n77A | CA_19A-42C | CA_n1A-n77A |

### 7.62.3	Co-existence studies

Co-existence studies of DC_19-42_n1-n77 are already covered by the fallback configurations, DC_19-42_n1, DC_19-42_n77 and DC_19_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.62.4	∆TIB and ∆RIB values

For DC_19-42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-19-42_n77, and are given in the tables below.

Table 7.62.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n1-n77 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.62.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-42_n1-n77 | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

### 7.62.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.63	DC_19-42_n1-n78

### 7.63.1	Operating bands for DC

Table 7.63.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-42_n1-n78 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.63.2	Configuration for DC

Table 7.63.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-42A_n1A-n78A | DC_19A_n1A DC_19A_n78A | CA_19A-42A | CA_n1A-n78A |
| DC_19A-42C_n1A-n78A | DC_19A_n1A DC_19A_n78A | CA_19A-42C | CA_n1A-n78A |

### 7.63.3	Co-existence studies

Co-existence studies of DC_19-42_n1-n78 are already covered by the fallback configurations, DC_19-42_n1, DC_19-42_n78 and DC_19_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.63.4	∆TIB and ∆RIB values

For DC_19-42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-19-42_n78, and are given in the tables below.

Table 7.63.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n1-n78 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 7.63.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-42_n1-n78 | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |
|  | n78 | 0.5 |

### 7.63.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.64	DC_19-42_n1-n79

### 7.64.1	Operating bands for DC

Table 7.64.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-42_n1-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.64.2	Configuration for DC

Table 7.64.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-42A_n1A-n79A | DC_19A_n1A DC_19A_n79A | CA_19A-42A | CA_n1A-n79A |
| DC_19A-42C_n1A-n79A | DC_19A_n1A DC_19A_n79A | CA_19A-42C | CA_n1A-n79A |

### 7.64.3	Co-existence studies

Co-existence studies of DC_19-42_n1-n79 are already covered by the fallback configurations, DC_19-42_n1, DC_19-42_n79 and DC_19_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.64.4	∆TIB and ∆RIB values

For DC_19-42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-19-42_n79, and are given in the tables below.

Table 7.64.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-42_n1-n79 | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 7.64.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-42_n1-n79 | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |
|  | n79 | 0 |

### 7.64.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.65	DC_21-42_n1-n77

### 7.65.1	Operating bands for DC

Table 7.65.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21-42_n1-n77 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.65.2	Configuration for DC

Table 7.65.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A-42A_n1A-n77A | DC_21A_n1A DC_21A_n77A | CA_21A-42A | CA_n1A-n77A |
| DC_21A-42C_n1A-n77A | DC_21A_n1A DC_21A_n77A | CA_21A-42C | CA_n1A-n77A |

### 7.65.3	Co-existence studies

Co-existence studies of DC_21-42_n1-n77 are already covered by the fallback configurations, DC_21-42_n1, DC_21-42_n77 and DC_21_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.65.4	∆TIB and ∆RIB values

For DC_21-42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-21-42_n77, and are given in the tables below.

Table 7.65.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n1-n77 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.65.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21-42_n1-n77 | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

### 7.65.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.66	DC_21-42_n1-n78

### 7.66.1	Operating bands for DC

Table 7.66.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21-42_n1-n78 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.66.2	Configuration for DC

Table 7.66.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A-42A_n1A-n78A | DC_21A_n1A DC_21A_n78A | CA_21A-42A | CA_n1A-n78A |
| DC_21A-42C_n1A-n78A | DC_21A_n1A DC_21A_n78A | CA_21A-42C | CA_n1A-n78A |

### 7.66.3	Co-existence studies

Co-existence studies of DC_21-42_n1-n78 are already covered by the fallback configurations, DC_21-42_n1, DC_21-42_n78 and DC_21_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.66.4	∆TIB and ∆RIB values

For DC_21-42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-21-42_n78, and are given in the tables below.

Table 7.66.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n1-n78 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 7.66.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21-42_n1-n78 | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |
|  | n78 | 0.5 |

### 7.66.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.67	DC_21-42_n1-n79

### 7.67.1	Operating bands for DC

Table 7.67.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_21-42_n1-n79 | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.67.2	Configuration for DC

Table 7.67.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_21A-42A_n1A-n79A | DC_21A_n1A DC_21A_n79A | CA_21A-42A | CA_n1A-n79A |
| DC_21A-42C_n1A-n79A | DC_21A_n1A DC_21A_n79A | CA_21A-42C | CA_n1A-n79A |

### 7.67.3	Co-existence studies

Co-existence studies of DC_21-42_n1-n79 are already covered by the fallback configurations, DC_21-42_n1, DC_21-42_n79 and DC_21_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.67.4	∆TIB and ∆RIB values

For DC_21-42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-21-42_n79, and are given in the tables below.

Table 7.67.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_21-42_n1-n79 | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 7.67.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_21-42_n1-n79 | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |
|  | n79 | 0 |

### 7.67.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.68	DC_3-20_n1-n78

### 7.68.1	Operating bands for DC

Table 7.68.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-20A_n1A-n78A DC_3C-20A_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.68.2	Channel bandwidths per operating band for DC

Table 7.68.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_3C_n1A DC_3C_n78A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.68.3	Co-existence studies

Co-existence studies of DC_3-20_n1-n78 and protected bands are already covered in the constituent fall-back modes.

### 7.68.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_3-20_n1-n78 are given in the tables below reusing the values for DC_1-3-20_n78.

Table 7.68.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n1-n78 | n1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |

Table 7.68.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n1-n78 | n1 | 0.2 |
|  | 3 | 0.2 |
|  | 20 | 0 |
|  | n78 | 0.5 |

### 7.68.5	MSD

All MSD requirements for DC_3-20_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.69	DC_7-20_n1-n78

### 7.69.1	Operating bands for DC

Table 7.69.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-20A_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.69.2	Channel bandwidths per operating band for DC

Table 7.69.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A-20A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.69.3	Co-existence studies

Co-existence studies of DC_7-20_n1-n78 and protected bands are already covered in the constituent fall-back modes.

### 7.69.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_7-20_n1-n78 are given in the tables below reusing the values for DC_1-7-20_n78.

Table 7.69.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20_n1-n78 | n1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.4 |
|  | n78 | 0.8 |

Table 7.69.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-20_n1-n78 | n1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n78 | 0.5 |

### 7.69.5	MSD

All MSD requirements for DC_7-20_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.70	DC_3-7_n40-n78

### 7.70.1	Operating bands for DC

Table 7.70.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n40-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.70.2	Channel bandwidths per operating band for DC

Table 7.70.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_3A-7A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.70.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.70.4	∆TIB and ∆RIB values

For DC_3-7_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.70.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n40-n78 | 3 | 0.6 |
|  | 7 | 0.5 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 7.70.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n40-n78 | 3 | 0.2 |
|  | 7 | 0 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

### 7.70.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.71	DC_1-7_n40-n78

### 7.71.1	Operating bands for DC

Table 7.71.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7_n40-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.71.2	Channel bandwidths per operating band for DC

Table 7.71.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_1A-7A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.71.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.71.4	∆TIB and ∆RIB values

For DC_1-7_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.71.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n40-n78 | 1 | 0.6 |
|  | 7 | 0.5 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 7.71.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n40-n78 | 1 | 0.2 |
|  | 7 | 0 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

### 7.71.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.72	DC_7-28_n40-n78

### 7.72.1	Operating bands for DC

Table 7.72.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-28_n40-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.72.2	Channel bandwidths per operating band for DC

Table 7.72.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_7A-28A_n40A-n78A | DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.72.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.72.4	∆TIB and ∆RIB values

For DC_7-28_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 7.72.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n40-n78 | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 7.72.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n40-n78 | 7 | 0 |
|  | 28 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

### 7.72.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.73	DC_2-13_n66-n77

### 7.73.1	Operating bands for DC

Table 7.73.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.73.2	Channel bandwidths per operating band for DC

Table 7.73.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-13A_n66A-n77A DC_2A-13A_n66A-n77C | DC_2A_n66A DC_2A_n77A DC_13A_n66A DC_13A_n77A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| DC_2A-2A-13A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_13A_n66A DC_13A_n77A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.73.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.73.4	∆TIB and ∆RIB values

For DC_2-13_n66-n77, the TIB,c and RIB,c values are derived from DC_2-13-66_n48 and are given in the tables below.

Table 7.73.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n66-n77 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.73.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13_n66-n77 | 2 | 0.3 |
|  | 13 | 0 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 7.73.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.74	DC_2-66_n66-n77

### 7.74.1	Operating bands for DC

Table 7.74.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.74.2	Channel bandwidths per operating band for DC

Table 7.74.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-66A_n66A-n77A DC_2A-66A_n66A-n77C | DC_2A_n66A DC_2A_n77A DC_66A_n77A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| DC_2A-2A-66A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_66A_n77A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.74.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.74.4	∆TIB and ∆RIB values

For DC_2-66_n66-n77, the TIB,c and RIB,c values are derived from DC_2_n66-n78 and are given in the tables below.

Table 7.74.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n66-n77 DC_2-2-66_n66-n77 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.74.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n66-n77 DC_2-2-66_n66-n77 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 7.74.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.75	DC_2-66_n5-n77

### 7.75.1	Operating bands for DC

Table 7.75.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n5-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.75.2	Channel bandwidths per operating band for DC

Table 7.75.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-66A_n5A-n77A | DC_66A_n5A DC_66A_n77A DC_2A_n5A DC_2A_n77A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.75.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.75.4	∆TIB and ∆RIB values

For DC_2-66_n5-n77, the TIB,c and RIB,c values are derived from DC_2-13-66_n48 and are given in the tables below.

Table 7.75.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n5-n77 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n5 | 0.3 |
|  | n77 | 0.8 |

Table 7.75.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n5-n77 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n5 | 0 |
|  | n77 | 0.5 |

### 7.75.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.76	DC_13-66_n66-n77

### 7.76.1	Operating bands for DC

Table 7.76.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13-66_n66-n77 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.76.2	Channel bandwidths per operating band for DC

Table 7.76.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A-66A_n66A-n77A | DC_13A_n66A DC_13A_n77A DC_66A_n77A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.76.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.76.4	∆TIB and ∆RIB values

For DC_13-66_n66-n77, the TIB,c and RIB,c values are derived from DC_13-66_n48 and are given in the tables below.

Table 7.76.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n66-n77 | 13 | 0.3 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.76.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-66_n66-n77 | 13 | 0 |
|  | 66 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |

### 7.76.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.77	DC_13-66_n2-n77

### 7.77.1	Operating bands for DC

Table 7.77.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13-66_n2-n77 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.77.2	Channel bandwidths per operating band for DC

Table 7.77.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A-66A_n2A-n77A | DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.77.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.77.4	∆TIB and ∆RIB values

For DC_13-66_n2-n77, the TIB,c and RIB,c values are derived from DC_2-13-66_n48 and are given in the tables below.

Table 7.77.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n2-n77 | 13 | 0.3 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 7.77.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-66_n2-n77 | 13 | 0 |
|  | 66 | 0.2 |
|  | n2 | 0.2 |
|  | n77 | 0.5 |

### 7.77.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.78	DC_13-66_n5-n48

### 7.78.1	Operating bands for DC

Table 7.78.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13-66_n5-n48 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n48 | 3550 MHz | – | 3800 MHz | 3550 MHz | – | 3800 MHz | TDD |

### 7.78.2	Channel bandwidths per operating band for DC

Table 7.78.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_13A-66A_n5A-n48A | DC_13A_n48A DC_66A_n5A DC_66A_n48A | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n48 | 15 | Yes5 | Yes | Yes | Yes |  |  | Yes | Yes6 |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes6 | Yes6 |  | Yes6 | Yes6,4 | Yes6 |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. NOTE 5: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an downlink SCell part of CA configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.78.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.78.4	∆TIB and ∆RIB values

For DC_13-66_n5-n48, the TIB,c and RIB,c values are derived from DC_5-48-66_n12 and are given in the tables below.

Table 7.78.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n5-n48 | 13 | 0.4 |
|  | 66 | 0.6 |
|  | n5 | 0.8 |
|  | n48 | 0.8 |

Table 7.78.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-66_n5-n48 | 13 | 0.3 |
|  | 66 | 0.2 |
|  | n5 | 0.5 |
|  | n48 | 0.5 |

### 7.78.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.79	DC_1-8_n78-n258

### 7.79.1	Operating bands for DC

Table 7.79.1-1: DC band combination LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n78-n258 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

### 7.79.2	Channel bandwidths per operating band for DC

Table 7.79.2-1: Supported bandwidths per DC LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_1A-8A_n78A-n258A DC_1A-8A_n78A-n258D DC_1A-8A_n78A-n258E DC_1A-8A_n78A-n258F DC_1A-8A_n78A-n258G DC_1A-8A_n78A-n258H DC_1A-8A_n78A-n258I DC_1A-8A_n78A-n258J DC_1A-8A_n78A-n258K DC_1A-8A_n78A-n258L DC_1A-8A_n78A-n258M | DC_1A_n78A DC_1A_n258A DC_8A_n78A DC_8A_n258A | 1 | See CA_1A-8A in TS 36.101 | 330 430 530 630 730 830 930 |
|  |  | 8 |  |  |
|  |  | n78 | CA_n78A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

### 7.79.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

### 7.79.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

### 7.79.5	MSD

No additional MSD requirement is needed.

## 7.80	DC_3-8_n78-n258

### 7.80.1	Operating bands for DC

Table 7.80.1-1: DC band combination of LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n78-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

### 7.80.2	Channel bandwidths per operating band for DC

Table 7.80.2-1: Supported bandwidths per DC LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A-8A_n78A-n258A DC_3A-8A_n78A-n258D DC_3A-8A_n78A-n258E DC_3A-8A_n78A-n258F DC_3A-8A_n78A-n258G DC_3A-8A_n78A-n258H DC_3A-8A_n78A-n258I DC_3A-8A_n78A-n258J DC_3A-8A_n78A-n258K DC_3A-8A_n78A-n258L DC_3A-8A_n78A-n258M | DC_3A_n78A DC_3A_n258A DC_8A_n78A DC_8A_n258A | 3 | See CA_3A-8A in TS 36.101 | 330 430 530 630 730 830 930 |
|  |  | 8 |  |  |
|  |  | n78 | CA_n78A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

### 7.80.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

### 7.80.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

### 7.80.5	MSD

No additional MSD requirement is needed.

## 7.81	DC_7-8_n78-n258

### 7.81.1	Operating bands for DC

Table 7.81.1-1: DC band combination of LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-8_n78-n258 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

### 7.81.2	Channel bandwidths per operating band for DC

Table 7.81.2-1: Supported bandwidths per DC LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_7A-8A_n78A-n258A DC_7A-8A_n78A-n258D DC_7A-8A_n78A-n258E DC_7A-8A_n78A-n258F DC_7A-8A_n78A-n258G DC_7A-8A_n78A-n258H DC_7A-8A_n78A-n258I DC_7A-8A_n78A-n258J DC_7A-8A_n78A-n258K DC_7A-8A_n78A-n258L DC_7A-8A_n78A-n258M | DC_7A_n78A DC_7A_n258A DC_8A_n78A DC_8A_n258A | 7 | See CA_7A-8A in TS 36.101 | 330 430 530 630 730 830 930 |
|  |  | 8 |  |  |
|  |  | n78 | CA_n78A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

### 7.81.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

### 7.81.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

### 7.81.5	MSD

No additional MSD requirement is needed.

## 7.82	DC_1-8_n40-n258

### 7.82.1	Operating bands for DC

Table 7.82.1-1: DC band combination of LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n40-n258 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

### 7.82.2	Channel bandwidths per operating band for DC

Table 7.82.2-1: Supported bandwidths per DC LTE 2bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_1A-8A_n40A-n258A DC_1A-8A_n40A-n258D DC_1A-8A_n40A-n258E DC_1A-8A_n40A-n258F DC_1A-8A_n40A-n258G DC_1A-8A_n40A-n258H DC_1A-8A_n40A-n258I DC_1A-8A_n40A-n258J DC_1A-8A_n40A-n258K DC_1A-8A_n40A-n258L DC_1A-8A_n40A-n258M | DC_1A_n40A DC_1A_n258A DC_8A_n40A DC_8A_n258A | 1 | See CA_1A-8A in TS 36.101 | 310 410 510 610 710 810 910 |
|  |  | 8 |  |  |
|  |  | n40 | CA_n40A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

### 7.82.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

### 7.82.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

### 7.82.5	MSD

No additional MSD requirement is needed.

## 7.83	DC_3-8_n40-n258

### 7.83.1	Operating bands for DC

Table 7.83.1-1: DC band combination of LTE 2 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n40-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

### 7.83.2	Channel bandwidths per operating band for DC

Table 7.83.2-1: Supported bandwidths per DC LTE 2 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A-8A_n40A-n258A DC_3A-8A_n40A-n258D DC_3A-8A_n40A-n258E DC_3A-8A_n40A-n258F DC_3A-8A_n40A-n258G DC_3A-8A_n40A-n258H DC_3A-8A_n40A-n258I DC_3A-8A_n40A-n258J DC_3A-8A_n40A-n258K DC_3A-8A_n40A-n258L DC_3A-8A_n40A-n258M | DC_3A_n40A DC_3A_n258A DC_8A_n40A DC_8A_n258A | 3 | See CA_3A-8A in TS 36.101 | 310 410 510 610 710 810 910 |
|  |  | 8 |  |  |
|  |  | n40 | CA_n40A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

### 7.83.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

### 7.83.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

### 7.83.5	MSD

No additional MSD requirement is needed.

## 7.84	DC_7-8_n40-n258

### 7.84.1	Operating bands for DC

Table 7.84.1-1: DC band combination of LTE 2 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-8_n40-n258 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

### 7.84.2	Channel bandwidths per operating band for DC

Table 7.84.2-1: Supported bandwidths per DC LTE 2 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_7A-8A_n40A-n258A DC_7A-8A_n40A-n258D DC_7A-8A_n40A-n258E DC_7A-8A_n40A-n258F DC_7A-8A_n40A-n258G DC_7A-8A_n40A-n258H DC_7A-8A_n40A-n258I DC_7A-8A_n40A-n258J DC_7A-8A_n40A-n258K DC_7A-8A_n40A-n258L DC_7A-8A_n40A-n258M | DC_7A_n40A DC_7A_n258A DC_8A_n40A DC_8A_n258A | 7 | See CA_7A-8A in TS 36.101 | 310 410 510 610 710 810 910 |
|  |  | 8 |  |  |
|  |  | n40 | CA_n40A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

### 7.84.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

### 7.84.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

### 7.84.5	MSD

No additional MSD requirement is needed.

## 7.85	DC_3-11_n28-n77

### 7.85.1	Operating bands for DC

Table 7.85.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-11_n28-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.85.2	Channel bandwidths per operating band for DC

Table 7.85.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_3A-11A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_3A-11A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 250 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.85.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_3-11_n28, DC_3-11_n77, DC_3_n28-n77 and DC_11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.85.4	∆TIB and ∆RIB values

For DC_3-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.85.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-11_n28-n77 | 3 | 0.8 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 7.85.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-11_n28-n77 | 3 | 0.3 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 7.85.5	MSD

As mentioned in 7.85.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.86	DC_8-11_n28-n77

### 7.86.1	Operating bands for DC

Table 7.86.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-11_n28-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.86.2	Channel bandwidths per operating band for DC

Table 7.86.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-11A_n28A-n77A | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
| DC_8A-11A_n28A-n77(2A) | DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  | 240 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.86.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-11_n28, DC_8-11_n77, DC_8_n28-n77 and DC_11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.86.4	∆TIB and ∆RIB values

For DC_8-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.86.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-11_n28-n77 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 7.86.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-11_n28-n77 | 8 | 0.2 |
|  | 11 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 7.86.5	MSD

As mentioned in 7.86.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.87	DC_1-11_n3-n77

### 7.87.1	Operating bands for DC

Table 7.87.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-11_n3-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.87.2	Channel bandwidths per operating band for DC

Table 7.87.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-11A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-11A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_11A_n3A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.87.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-11_n3, DC_1-11_n77, DC_1_n3-n77 and DC_11_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.87.4	∆TIB and ∆RIB values

For DC_1-11_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.87.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n3-n77 | 1 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n77 | 0.8 |

Table 7.87.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n3-n77 | 1 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n77 | 0.5 |

### 7.87.5	MSD

As mentioned in 7.87.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.88	DC_8-11_n3-n77

### 7.88.1	Operating bands for DC

Table 7.88.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-11_n3-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.88.2	Channel bandwidths per operating band for DC

Table 7.88.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-11A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A-11A_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 250 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.88.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-11_n3, DC_8-11_n77, DC_8_n3-n77 and DC_11_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.88.4	∆TIB and ∆RIB values

For DC_8-11_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.88.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-11_n3-n77 | 8 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n77 | 0.8 |

Table 7.88.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-11_n3-n77 | 8 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n77 | 0.5 |

### 7.88.5	MSD

As mentioned in 7.88.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.89	DC_1-11_n28-n77

### 7.89.1	Operating bands for DC

Table 7.89.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-11_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.89.2	Channel bandwidths per operating band for DC

Table 7.89.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-11A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-11A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 250 |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.89.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-11_n28, DC_1-11_n77, DC_1_n28-n77 and DC_11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.89.4	∆TIB and ∆RIB values

For DC_1-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.89.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n28-n77 | 1 | 0.6 |
|  | 11 | 0.4 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 7.89.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n28-n77 | 1 | 0.2 |
|  | 11 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 7.89.5	MSD

As mentioned in 7.89.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.90	DC_1-42_n3-n28

### 7.90.1	Operating bands for DC

Table 7.90.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-42_n3-n28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 7.90.2	Channel bandwidths per operating band for DC

Table 7.90.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-42A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 90 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_42A_n3A DC_42A_n28A DC_42C_n3A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 110 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.90.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-42_n3, DC_1-42_n28, DC_1_n3-n28 and DC_42_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.90.4	∆TIB and ∆RIB values

For DC_1-42_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 7.90.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n3-n28 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n28 | 0.8 |

Table 7.90.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-42_n3-n28 | 1 | 0 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n28 | 0.5 |

### 7.90.5	MSD

As mentioned in 7.90.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.91	DC_1-42_n3-n77

### 7.91.1	Operating bands for DC

Table 7.91.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-42_n3-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.91.2	Channel bandwidths per operating band for DC

Table 7.91.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-42A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_42A_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-42A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_42A_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-42C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-42C_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_42A_n3A DC_42C_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 290 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.91.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-42_n3, DC_1-42_n77, DC_1_n3-n77 and DC_42_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.91.4	∆TIB and ∆RIB values

For DC_1-42_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.91.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-42_n3-n77 | 1 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 7.91.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-42_n3-n77 | 1 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 7.91.5	MSD

As mentioned in 7.91.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.92	DC_8-42_n3-n28

### 7.92.1	Operating bands for DC

Table 7.92.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-42_n3-n28 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 7.92.2	Channel bandwidths per operating band for DC

Table 7.92.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-42A_n3A-n28A | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42A_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 80 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A-42C_n3A-n28A | DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.92.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-42_n3, DC_8-42_n28, DC_8_n3-n28 and DC_42_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.92.4	∆TIB and ∆RIB values

For DC_8-42_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 7.92.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-42_n3-n28 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n28 | 0.8 |

Table 7.92.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-42_n3-n28 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n28 | 0.5 |

### 7.92.5	MSD

As mentioned in 7.92.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.93	DC_8-42_n3-n77

### 7.93.1	Operating bands for DC

Table 7.93.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-42_n3-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.93.2	Channel bandwidths per operating band for DC

Table 7.93.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-42A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A-42A_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_8A-42C_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A-42C_n3A-n77(2A) | DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A DC_42A_n77A DC_42C_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.93.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-42_n3, DC_8-42_n77, DC_8_n3-n77 and DC_42_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.93.4	∆TIB and ∆RIB values

For DC_8-42_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.93.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-42_n3-n77 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 7.93.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-42_n3-n77 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 7.93.5	MSD

As mentioned in 7.93.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.94	DC_1-21_n28-n77

### 7.94.1	Operating bands for DC

Table 7.94.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_1-21_n28-n77 | 1 | 1920 MHz | – | 1980 MHz |  | 2110 MHz | – | 2170 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n77 | 3300 MHz | – | 4200 MHz |  | 3300 MHz | – | 4200 MHz |  | TDD |  |

### 7.94.2	Configuration for DC

Table 7.94.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-21A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_21A_n28A DC_21A_n77A | CA_1A-21A | CA_n28A-n77A |

### 7.94.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

### 7.94.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.94.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n28-n77 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 7.94.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21_n28-n77 | 1 | 0.2 |
|  | 21 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 7.94.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.95	DC_1-21_n28-n78

### 7.95.1	Operating bands for DC

Table 7.95.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_1-21_n28-n78 | 1 | 1920 MHz | – | 1980 MHz |  | 2110 MHz | – | 2170 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n78 | 3300 MHz | – | 3800 MHz |  | 3300 MHz | – | 3800 MHz |  | TDD |  |

### 7.95.2	Configuration for DC

Table 7.95.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-21A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_21A_n28A DC_21A_n78A | CA_1A-21A | CA_n28A-n78A |

### 7.95.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

### 7.95.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.95.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n28-n78 | 1 | 0.6 |
|  | 21 | 0.4 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 7.95.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21_n28-n78 | 1 | 0.2 |
|  | 21 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 7.95.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.96	DC_1-21_n28-n79

### 7.96.1	Operating bands for DC

Table 7.96.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_1-21_n28-n79 | 1 | 1920 MHz | – | 1980 MHz |  | 2110 MHz | – | 2170 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n79 | 4400 MHz | – | 5000 MHz |  | 4400 MHz | – | 5000 MHz |  | TDD |  |

### 7.96.2	Configuration for DC

Table 7.96.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-21A_n28A-n79A | DC_1A_n28A DC_1A_n79A DC_21A_n28A DC_21A_n79A | CA_1A-21A | CA_n28A-n79A |

### 7.96.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

### 7.96.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.96.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-21_n28-n79 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | n28 | 0.6 |
|  | n79 | 0 |

Table 7.96.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-21_n28-n79 | 1 | 0.3 |
|  | 21 | 0 |
|  | n28 | 0.3 |
|  | n79 | 0 |

### 7.96.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.97	DC_1-3_n28-n79

### 7.97.1	Operating bands for DC

Table 7.97.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_1-3_n28-n79 | 1 | 1920 MHz | – | 1980 MHz |  | 2110 MHz | – | 2170 MHz |  | FDD |  |
|  | 3 | 1710 MHz | – | 1785 MHz |  | 1805 MHz | – | 1880 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n79 | 4400 MHz | – | 5000 MHz |  | 4400 MHz | – | 5000 MHz |  | TDD |  |

### 7.97.2	Configuration for DC

Table 7.97.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n28A-n79A | DC_1A_n28A DC_1A_n79A DC_3A_n28A DC_3A_n79A | CA_3A-21A | CA_n28A-n79A |

### 7.97.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

### 7.97.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.97.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n28-n79 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n28 | 0.6 |
|  | n79 | 0 |

Table 7.97.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n28-n79 | 1 | 0 |
|  | 3 | 0 |
|  | n28 | 0.2 |
|  | n79 | 0 |

### 7.97.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.98	DC_3-21_n28-n77

### 7.98.1	Operating bands for DC

Table 7.98.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_3-21_n28-n77 | 3 | 1710 MHz | – | 1785 MHz |  | 1805 MHz | – | 1880 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n77 | 3300 MHz | – | 4200 MHz |  | 3300 MHz | – | 4200 MHz |  | TDD |  |

### 7.98.2	Configuration for DC

Table 7.98.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_21A_n28A DC_21A_n77A | CA_3A-21A | CA_n28A-n77A |

### 7.98.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

### 7.98.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.98.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n28-n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n28 | 0.5 |
|  | n77 | 0.8 |

Table 7.98.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n28-n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 7.98.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.99	DC_3-21_n28-n78

### 7.99.1	Operating bands for DC

Table 7.99.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_3-21_n28-n78 | 3 | 1710 MHz | – | 1785 MHz |  | 1805 MHz | – | 1880 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n78 | 3300 MHz | – | 3800 MHz |  | 3300 MHz | – | 3800 MHz |  | TDD |  |

### 7.99.2	Configuration for DC

Table 7.99.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_21A_n28A DC_21A_n78A | CA_3A-21A | CA_n28A-n78A |

### 7.99.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

### 7.99.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.99.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n28-n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 7.99.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n28-n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 7.99.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.100	DC_3-21_n28-n79

### 7.100.1	Operating bands for DC

Table 7.100.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |  |
| DC_3-21_n28-n79 | 3 | 1710 MHz | – | 1785 MHz |  | 1805 MHz | – | 1880 MHz |  | FDD |  |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz |  | 1495.9 MHz | – | 1510.9 MHz |  | FDD |  |
|  | n28 | 703 MHz | – | 748 MHz |  | 758 MHz | – | 803 MHz |  | FDD |  |
|  | n79 | 4400 MHz | – | 5000 MHz |  | 4400 MHz | – | 5000 MHz |  | TDD |  |

### 7.100.2	Configuration for DC

Table 7.100.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A_n28A-n79A | DC_3A_n28A DC_3A_n79A DC_21A_n28A DC_21A_n79A | CA_3A-21A | CA_n28A-n79A |

### 7.100.3	Co-existence studies

Co-existence studies are omitted since those are covered by the co-existence studies for the lower order of fallback band combinations such as DC of LTE 1 band + NR 1band, LTE 2 bands + NR 1band, and LTE 1band + NR 2bands.

### 7.100.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.100.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21_n28-n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | n28 | 0.3 |
|  | n79 | 0 |

Table 7.100.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21_n28-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | n28 | 0.3 |
|  | n79 | 0 |

### 7.100.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.101	DC_1-8_n28-n78

### 7.101.1	 Operating bands for DC

Table 7.101.1-1: LTE 2 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-8A_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.101.2 	Configuration for DC

Table 7.101.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-8A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.101.3	 Co-existence studies

Co-existence studies of DC_1A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.101.4	 ∆TIB and ∆RIB values

Table 7.101.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n28-n78 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 7.101.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n28-n78 | 1 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 7.101.5	MSD

All MSD requirements for DC_1-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.102	DC_3-8_n28-n78

### 7.102.1	 Operating bands for DC

Table 7.102.1-1: LTE 2 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-8A_n28A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.102.2 	Configuration for DC

Table 7.102.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-8A_n28A-n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.102.3	 Co-existence studies

Co-existence studies of DC_3A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.102.4	 ∆TIB and ∆RIB values

Table 7.102.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n28-n78 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 7.102.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8_n28-n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 7.102.5	MSD

All MSD requirements for DC_3-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.103	DC_7-8_n28-n78

### 7.103.1	 Operating bands for DC

Table 7.103.1-1: LTE 2 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-8A_n28A-n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.103.2 	Configuration for DC

Table 7.103.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-8A_n28A-n78A |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.103.3	 Co-existence studies

Co-existence studies of DC_7A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.103.4	 ∆TIB and ∆RIB values

Table 7.103.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n28-n78 | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |
|  | n78 | 0.8 |

Table 7.103.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n28-n78 | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 7.103.5	MSD

All MSD requirements for DC_7-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.104	DC_3-28_n1-n78

### 7.104.1	Operating bands for DC

Table 7.104.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.104.2	Channel bandwidths per operating band for DC

Table 7.104.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_3A-28A_n1A-n78A | DC_3A_n1A DC_28A_n1A DC_3A_n78A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.104.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.104.4	∆TIB and ∆RIB values

For DC_3-28_n1-n78, the TIB,c and RIB,c values are derived from DC_1-28_n3-n78 and given in the tables below.

Table 7.104.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n1-n78 | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.104.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n1-n78 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 7.104.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.105	DC_7-28_n1-n78

### 7.105.1	Operating bands for DC

Table 7.105.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-28_n1-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.105.2	Channel bandwidths per operating band for DC

Table 7.105.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_7A-28A_n1A-n78A | DC_7A_n1A DC_28A_n1A DC_7A_n78A DC_28A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.105.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.105.4	∆TIB and ∆RIB values

For DC_7-28_n1-n78, the TIB,c and RIB,c values are derived from DC_1-7-28_n78 and given in the tables below.

Table 7.105.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28_n1-n78 | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 7.105.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28_n1-n78 | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 7.105.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.106	DC_3-40_n1-n78

### 7.106.1	Operating bands for DC

Table 7.106.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-40A_n1A-n78A DC_3A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.106.2	Channel bandwidths per operating band for DC

Table 7.106.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.106.3	Co-existence studies

Co-existence studies of DC_3-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

### 7.106.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_3-40_n1-n78 are given in the tables below reusing the values for DC_1-3_n40-n78.

Table 7.106.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-40_n1-n78 | n1 | 0.5 |
|  | 3 | 0.6 |
|  | 40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 7.106.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-40_n1-n78 | n1 | 0 |
|  | 3 | 0.2 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 7.106.5	MSD

All MSD requirements for DC_3-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.107	DC_7-40_n1-n78

### 7.107.1	Operating bands for DC

Table 7.107.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-40A_n1A-n78A DC_7A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.107.2	Channel bandwidths per operating band for DC

Table 7.107.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A-40A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_7A-40C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.107.3	Co-existence studies

Co-existence studies of DC_7-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

### 7.107.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_7-40_n1-n78 are given in the tables below reusing the values for DC_1-7-40_n78.

Table 7.107.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-40_n1-n78 | n1 | 0.6 |
|  | 7 | 0.5 |
|  | 40 | 0.51 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 7.107.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-40_n1-n78 | n1 | 0.2 |
|  | 7 | 0 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 7.107.5	MSD

All MSD requirements for DC_7-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.108	DC_8-40_n1-n78

### 7.108.1	Operating bands for DC

Table 7.108.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8A-40A_n1A-n78A DC_8A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.108.2	Channel bandwidths per operating band for DC

Table 7.108.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_8A-40A_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_8A-40C_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.108.3	Co-existence studies

Co-existence studies of DC_8-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

### 7.108.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_8-40_n1-n78 are given in the tables below reusing the values for DC_1-8_n40-n78.

Table 7.108.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-40_n1-n78 | n1 | 0.5 |
|  | 8 | 0.3 |
|  | 40 | 0.51 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 7.108.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_8-40_n1-n78 | n1 | 0 |
|  | 8 | 0.2 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 7.108.5	MSD

All MSD requirements for DC_8-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.109	DC_2-7_n25-n66

### 7.109.1	Operating bands for DC

Table 7.109.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-7_n25-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 7.109.2	Channel bandwidths per operating band for DC

Table 7.109.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-7A_n25A-n66A6,7 | DC_2A_n66A DC_7A_n25A DC_7A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_2A-7A-7A_n25A-n66A6,7 | DC_2A_n66A DC_7A_n25A DC_7A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 7 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_2A-7C_n25A-n66A6,7 | DC_2A_n66A DC_7A_n25A DC_7A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply for the Band 42 and Band n77/n78 combination when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.109.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.109.4	∆TIB and ∆RIB values

For DC_2-7_n25-n66, the TIB,c and RIB,c values are derived from DC_2-7_n66 and given in the tables below.

Table 7.109.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n25-n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 7.109.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n25-n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

### 7.109.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.110	DC_2-13_n25-n66

### 7.110.1	Operating bands for DC

Table 7.110.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13_n25-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 7.110.2	Channel bandwidths per operating band for DC

Table 7.110.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-13A_n25A-n66A6,7 | DC_2A_n66A DC_13A_n25A DC_13A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 110 |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply for the Band 42 and Band n77/n78 combination when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.110.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.110.4	∆TIB and ∆RIB values

For DC_2-13_n25-n66, the TIB,c and RIB,c values are derived from DC_2-13_n66 and given in the tables below.

Table 7.110.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n25-n66 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 7.110.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13_n25-n66 | 2 | 0.3 |
|  | 13 | 0 |
|  | n25 | 0.3 |
|  | n66 | 0.3 |

### 7.110.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.111	DC_2-66_n25-n66

### 7.111.1	Operating bands for DC

Table 7.111.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n25-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 7.111.2	Channel bandwidths per operating band for DC

Table 7.111.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-66A_n25A-n66A6,7 | DC_2A_n66A DC_66A_n25A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply for the Band 42 and Band n77/n78 combination when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.111.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.111.4	∆TIB and ∆RIB values

For DC_2-66_n25-n66, the TIB,c and RIB,c values are derived from DC_2_n66 and given in the tables below.

Table 7.111.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n25-n66 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 7.111.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n25-n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n25 | 0.3 |
|  | n66 | 0.3 |

### 7.111.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.112	DC_7-13_n25-n66

### 7.112.1	Operating bands for DC

Table 7.112.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-13_n25-n66 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 7.112.2	Channel bandwidths per operating band for DC

Table 7.112.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 110 |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_7A-7A-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A | 7 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_7C-13A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 7.112.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.112.4	∆TIB and ∆RIB values

For DC_7-13_n25-n66, the TIB,c and RIB,c values are derived from DC_2-7-13_n66 and given in the tables below.

Table 7.112.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-13_n25-n66 | 7 | 0.5 |
|  | 13 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 7.112.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-13_n25-n66 | 7 | 0.5 |
|  | 13 | 0 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

### 7.112.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.113	DC_7-66_n25-n66

### 7.113.1	Operating bands for DC

Table 7.113.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-66_n25-n66 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 7.113.2	Channel bandwidths per operating band for DC

Table 7.113.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_7A-7A-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A | 7 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_7C-66A_n25A-n66A | DC_7A_n25A DC_7A_n66A DC_66A_n25A | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |

### 7.113.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.113.4	∆TIB and ∆RIB values

For DC_7-66_n25-n66, the TIB,c and RIB,c values are derived from DC_2-7_n66 and given in the tables below.

Table 7.113.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n25-n66 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 7.113.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n25-n66 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

### 7.113.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.114	DC_3-7_n3-n78

### 7.114.1	Operating bands for DC

Table 7.114.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n3-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.114.2	Channel bandwidths per operating band for DC

Table 7.114.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_3A-7A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_3A_n78A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7C_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_3A_n78A DC_7C_n78A DC_7A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.114.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.114.4	∆TIB and ∆RIB values

For DC_3-7_n3-n78, the TIB,c and RIB,c values are derived from CA_n3-n7-n78 and given in the tables below.

Table 7.114.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n3-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 7.114.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n3-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 7.114.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.115	DC_3-28_n3-n78

### 7.115.1	Operating bands for DC

Table 7.115.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-28_n3-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.115.2	Channel bandwidths per operating band for DC

Table 7.115.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_3A-28A_n3A-n78A | DC_3A_n3A4 DC_28A_n3A DC_3A_n78A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.115.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.115.4	∆TIB and ∆RIB values

For DC_3-28_n3-n78, the TIB,c and RIB,c values are derived from CA_n3-n28-n78 and given in the tables below.

Table 7.115.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28_n3-n78 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | n3 | 0.5 |
|  | n78 | 0.8 |

Table 7.115.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28_n3-n78 | 3 | 0 |
|  | 28 | 0.2 |
|  | n3 | 0 |
|  | n78 | 0.5 |

### 7.115.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.116	DC_3-7_n1-n8, DC_3-3-7_n1-n8, DC_3-7-7_n1-n8, DC_3-3-7-7_n1-n8

### 7.116.1	Operating bands for DC

Table 7.116.1-1: DC band combination of two LTE bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n1-n8, DC_3-3-7_n1-n8, DC_3-7-7_n1-n8, DC_3-3-7-7_n1-n8 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz |  |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |

### 7.116.2	Configuration for DC

Table 7.116.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n1A-n8A DC_3A-3A-7A_n1A-n8A DC_3A-7A-7A_n1A-n8A DC_3A-3A-7A-7A_n1A-n8A | DC_3A_n1A DC_3A_n8A DC_7A_n1A DC_7A_n8A | CA_3A-7A CA_3A-3A-7A CA_3A-7A-7A CA_3A-3A-7A-7A | CA_n1A-n8A |

### 7.116.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.116.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below.

Table 7.116.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n1-n8, DC_3-3-7_n1-n8, DC_3-7-7_n1-n8, DC_3-3-7-7_n1-n8 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n1 | 0.6 |
|  | n8 | 0.6 |

Table 7.116.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n1-n8, DC_3-3-7_n1-n8, DC_3-7-7_n1-n8, DC_3-3-7-7_n1-n8 | 3 | 0 |
|  | 7 | 0 |
|  | n1 | 0 |
|  | n8 | 0.2 |

### 7.116.5	MSD

No additional MSD requirement is needed.

## 7.117	DC_3-7_n8-n78, DC_3-3-7_n8-n78, DC_3-7-7_n8-n78, DC_3-3-7-7_n8-n78

### 7.117.1	Operating bands for DC

Table 7.117.1-1: DC band combination of two LTE bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n8-n78, DC_3-3-7_n8-n78, DC_3-7-7_n8-n78, DC_3-3-7-7_n8-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.117.2	Configuration for DC

Table 7.117.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A_n8A-n78A2 DC_3A-3A-7A_n8A-n78A2 DC_3A-7A-7A_n8A-n78A2 DC_3A-3A-7A-7A_n8A-n78A2 | DC_3A_n8A DC_3A_n78A DC_7A_n8A DC_7A_n78A | CA_3A-7A CA_3A-3A-7A CA_3A-7A-7A CA_3A-3A-7A-7A | CA_n8A-n78A |
| NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |

### 7.117.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.117.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values are given in the tables below, which is the same as the values for DC_3-7-8_n78.

Table 7.117.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7_n8-n78, DC_3-3-7_n8-n78, DC_3-7-7_n8-n78, DC_3-3-7-7_n8-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 7.117.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7_n8-n78, DC_3-3-7_n8-n78, DC_3-7-7_n8-n78, DC_3-3-7-7_n8-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 7.117.5	MSD

No additional MSD requirement is needed.

## 7.118	DC_1-7_n8-n78

### 7.118.1	 Operating bands for DC

Table 7.118.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.118.2 	Configuration for DC

Table 7.118.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.118.3	 Co-existence studies

Co-existence studies of DC_1A-7A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.118.4	 ∆TIB and ∆RIB values

Table 7.118.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A_n8A-n78A | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 7.118.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-7A_n8A-n78A | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 7.118.5	MSD

All MSD requirements for DC_1A-7A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.119	DC_1-20_n8-n78

### 7.119.1	 Operating bands for DC

Table 7.119.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-20A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.119.2 	Configuration for DC

Table 7.119.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-20A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.119.3	 Co-existence studies

Co-existence studies of DC_1A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.119.4	 ∆TIB and ∆RIB values

Table 7.119.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-20A_n8A-n78A | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 7.119.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-20A_n8A-n78A | 1 | 0.2 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 7.119.5	MSD

All MSD requirements for DC_1A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.120	DC_3-20_n8-n78

### 7.120.1	 Operating bands for DC

Table 7.120.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-20A_n8A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.120.2 	Configuration for DC

Table 7.120.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A_n8A-n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.120.3	 Co-existence studies

Co-existence studies of DC_3A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.120.4	 ∆TIB and ∆RIB values

Table 7.120.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-20A_n8A-n78A | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 7.120.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-20A_n8A-n78A | 3 | 0.2 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 7.120.5	MSD

All MSD requirements for DC_3A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.121	DC_7-20_n8-n78

### 7.121.1	 Operating bands for DC

Table 7.121.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-20A_n8A-n78A | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.121.2 	Configuration for DC

Table 7.121.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A-20A_n8A-n78A |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.121.3	 Co-existence studies

Co-existence studies of DC_7A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.121.4	 ∆TIB and ∆RIB values

Table 7.121.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A_n8A-n78A | 7 | 0.5 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 7.121.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7A-20A_n8A-n78A | 7 | 0 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 7.121.5	MSD

All MSD requirements for DC_7A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.122	DC_7-8_n1-n40

### 7.122.1	Operating bands for DC

Table 7.122.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n1-n40 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 7.122.1-2: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-8A_n1A-n40A | DC_7A_n1A DC_8A_n1A DC_7A_n40A DC_8A_n40A | CA_7A-8A | CA_n1A-n40A |

### 7.122.2	Channel bandwidths per operating band for DC

Table 7.122.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_7A-8A_n1A-n40A | DC_7A_n1A DC_8A_n1A DC_7A_n40A DC_8A_n40A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |

### 7.122.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back modes.

### 7.122.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.122.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n1-n40 | 7 | 0.8 |
|  | 8 | 0.6 |
|  | n1 | 0.6 |
|  | n40 | 0.9 |

Table 7.122.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n1-n40 | 7 | 0.3 |
|  | 8 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0.8 |

### 7.122.5	MSD

Based on co-existence studies no additional MSD is needed.

## 7.123	DC_1-3_n8-n78

### 7.123.1	 Operating bands for DC

Table 7.123.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.123.2 	Configuration for DC

Table 7.123.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.123.3	 Co-existence studies

Co-existence studies of DC_1A-3A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.123.4	 ∆TIB and ∆RIB values

Table 7.123.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A_n8A-n78A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 7.123.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A_n8A-n78A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 7.123.5	MSD

All MSD requirements for DC_1A-3A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.124	DC_3-8_n1-n40

### 7.124.1	Operating bands for DC

Table 7.124.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n1-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 7.124.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-8A_n1A-n40A | DC_3A_n1A DC_8A_n1A DC_3A_n40A DC_8A_n40A | CA_3A-8A | CA_n1A-n40A |

### 7.124.2	Channel bandwidths per operating band for DC

Table 7.124.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_3A-8A_n1A-n40A | DC_3A_n1A DC_8A_n1A DC_3A_n40A DC_8A_n40A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |

### 7.124.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back modes.

### 7.124.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 7.124.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n1-n40 | 3 | 0.5 |
|  | 8 | 0.5 |
|  | n1 | 0.5 |
|  | n40 | 0.6 |

Table 7.124.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8_n1-n40 | 3 | 0 |
|  | 8 | 0 |
|  | n1 | 0.1 |
|  | n40 | 0.2 |

### 7.124.5	MSD

Based on co-existence studies no additional MSD is needed.

## 7.125	DC_1-8_n3-n79

### 7.125.1	Operating bands for DC

Table 7.125.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n3-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.125.2	Channel bandwidths per operating band for DC

Table 7.125.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_8A_n3A DC_8A_n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 7.125.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8_n3, DC_1-8_n79, DC_1_n3-n79 and DC_8_n3-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.125.4	∆TIB and ∆RIB values

For DC_1-8_n3-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.125.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n3-n79 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n3 | 0.3 |
|  | n79 | 0.8 |

Table 7.125.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n3-n79 | 1 | 0 |
|  | 8 | 0 |
|  | n3 | 0 |
|  | n79 | 0.5 |

### 7.125.5	MSD

As mentioned in 7.125.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.126	DC_1-8_n77-n79

### 7.126.1	Operating bands for DC

Table 7.126.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.126.2	Channel bandwidths per operating band for DC

Table 7.126.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 7.126.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8_n77, DC_1-8_n79, DC_1_n77-n79 and DC_8_n77-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.126.4	∆TIB and ∆RIB values

For DC_1-8_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.126.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n77-n79 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 7.126.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n77-n79 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | n77 | 0.5 |
|  | n79 | 0 |

### 7.126.5	MSD

As mentioned in 7.126.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.127	DC_2-7_n66-n77

### 7.127.1	Operating bands for DC

Table 7.127.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-7_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.127.2	Channel bandwidths per operating band for DC

Table 7.127.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-7A_n66A-n77A DC_2A-7C_n66A-n77A DC_2A-7A-7A_n66A-n77A | DC_2A_n66A DC_7A_n66A DC_2A_n77A DC_7A_n77A |

### 7.127.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.127.4	∆TIB and ∆RIB values

For DC_2-7_n66-n77, the TIB,c and RIB,c values are derived from DC_2-7_n66-n78 and are given in the tables below.

Table 7.127.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n66-n77 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.127.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n66-n77 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |
|  | n77 | 0.5 |

### 7.127.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.128	DC_7-66_n66-n77

### 7.128.1	Operating bands for DC

Table 7.128.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-66_n66-n77 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.128.2	Channel bandwidths per operating band for DC

Table 7.128.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A-66A_n66A-n77A DC_7C-66A_n66A-n77A DC_7A-7A-66A_n66A-n77A | DC_7A_n66A DC_7A_n77A DC_66A_n77A |

### 7.128.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.128.4	∆TIB and ∆RIB values

For DC_7-66_n66-n77, the TIB,c and RIB,c values are derived from DC_7-66_n66-n78 and are given in the tables below.

Table 7.128.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n66-n77 | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.128.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n66-n77 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
|  | n77 | 0.5 |

### 7.128.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.129	DC_2-5_n2-n41

### 7.129.5	MSD

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD2 produced by Band 5 and NR band n2 that impact the reference sensitivity of NR band n41.

The required MSD levels and test configuration are shown in the following Table.

Table 7.129.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| NR CA inter-bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-5A_n2A-n41A | B5 | IMD2 | ¦fB5 +fn2¦ | 829 | 5 | 25 | 874 | 5 | N/A |
|  | n2 |  |  | 1855 | 5 | 25 | 1935 | 5 |  |
|  | n41 |  |  | 2684 | 5 | 25 | 2684 | 5 | 29.7 |

## 7.130 	DC_2-5_n2-n77

### 7.130.1	Operating bands for DC

Table 7.130.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-5_n2-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.130.2	Configuration for DC

Table 7.130.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A_n2A-n77A | DC_2A_n77A DC_5A_n2A DC_5A_n77A | CA_2A-5A | CA_n2A-n77A |

### 7.130.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 7.130.4	∆TIB and ∆RIB values

For DC_2-5_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.130.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n2-n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 7.130.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n2-n77 | 2 | 0.2 |
|  | 5 | 0.2 |
|  | n2 | 0.2 |
|  | n77 | 0.5 |

### 7.130.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.131 	DC_2-5_n5-n77

### 7.131.1	Operating bands for DC

Table 7.131.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-5_n5-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.131.2	Configuration for DC

Table 7.131.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A | CA_2A-5A | CA_n5A-n77A |

### 7.131.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 7.131.4	∆TIB and ∆RIB values

For DC_2-5_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.131.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n5-n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 7.131.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n5-n77 | 2 | 0.2 |
|  | 5 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

### 7.131.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.132 	DC_5-66_n66-n77

### 7.132.1	Operating bands for DC

Table 7.132.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5-66_n66-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.132.2	Configuration for DC

Table 7.132.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-66A_n66A-n77A | DC_5A_n66A DC_5A_n77A DC_66A_n77A | CA_5A-66A | CA_n66A-n77A |

### 7.132.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 7.132.4	∆TIB and ∆RIB values

For DC_5-66_n66-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.132.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n66-n77 | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.132.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66_n66-n77 | 5 | 0.2 |
|  | 66 | 0.2 |
|  | n66 | 0.2 |
|  | n77 | 0.5 |

### 7.132.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.133 	DC_2-5_n66-n77

### 7.133.1	Operating bands for DC

Table 7.133.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-5_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.133.2	Configuration for DC

Table 7.133.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_5A_n66A DC_5A_n77A | CA_2A-5A | CA_n66A-n77A |

### 7.133.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 7.133.4	∆TIB and ∆RIB values

For DC_2-5_n66-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.133.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n66-n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.133.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n66-n77 | 2 | 0.3 |
|  | 5 | 0.2 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 7.133.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.134 	DC_13-66_n5-n77

### 7.134.1	Operating bands for DC

Table 7.134.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_13-66_n5-n77 | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.134.2	Configuration for DC

Table 7.134.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_13A-66A_n5A-n77 | DC_13A_n77A DC_66A_n5A DC_66A_n77A | CA_13A-66A | CA_ n5A-n77 |
| DC_13A-66A-66A_n5A-n77A | DC_13A_n77A DC_66A_n5A DC_66A_n77A | CA_13A-66A-66A | CA_ n5A-n77 |

### 7.134.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 7.134.4	∆TIB and ∆RIB values

For DC_13-66_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.134.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n5-n77 DC_13-66-66_n5-n77 | 13 | 0.5 |
|  | 66 | 0.6 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 7.134.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_13-66_n5-n77 DC_13-66-66_n5-n77 | 13 | 0.2 |
|  | 66 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

### 7.134.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.135 	DC_2-13_n2-n77

### 7.135.1	Operating bands for DC

Table 7.135.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13_n2-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.135.2	Configuration for DC

Table 7.135.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-13A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A | CA_2A-13A | CA_n2A-n77A |

### 7.135.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 7.135.4	∆TIB and ∆RIB values

For DC_2-13_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.135.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n2-n77 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 7.135.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13_n2-n77 | 2 | 0.2 |
|  | 13 | 0 |
|  | n2 | 0.2 |
|  | n77 | 0.5 |

### 7.135.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.136  DC_2-13_n5-n77

### 7.136.1	Operating bands for DC

Table 7.136.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13_n5-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.136.2	Configuration for DC

Table 7.136.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-13A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A | CA_2A-13A | CA_n5A-n77A |
| DC_2A-2A-13A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A | CA_2A-2A-13A | CA_n5A-n77A |

### 7.136.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 7.136.4	∆TIB and ∆RIB values

For DC_2-13_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.136.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n5-n77 DC_2-2-13_n5-n77 | 2 | 0.6 |
|  | 13 | 0.5 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 7.136.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13_n5-n77 DC_2-2-13_n5-n77 | 2 | 0.2 |
|  | 13 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

### 7.136.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.137  DC_2-66_n2-n77

### 7.137.1	Operating bands for DC

Table 7.137.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n2-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.137.2	Configuration for DC

Table 7.137.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-66A_n2A-n77A | DC_2A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-66A | CA_n2A-n77A |
| DC_2A-66A-66A_n2A-n77A | DC_2A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-66A-66A | CA_n2A-n77A |

### 7.137.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 7.137.4	∆TIB and ∆RIB values

For DC_2-66_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.137.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n2-n77 DC_2-66-66_n2-n77 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 7.137.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n2-n77 DC_2-66-66_n2-n77 | 2 | 0.2 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n77 | 0.5 |

### 7.137.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.138  DC_5-66_n2-n77

### 7.138.1	Operating bands for DC

Table 7.138.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5-66_n2-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 66 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.138.2	Configuration for DC

Table 7.138.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-66A_n2A-n77A | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A | CA_5A-66A | CA_n2A-n77A |
| DC_5A-66A-66A_n2A-n77A | DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A | CA_5A-66A-66A | CA_n2A-n77A |

### 7.138.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 7.138.4	∆TIB and ∆RIB values

For DC_5-66_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.138.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n2-n77 DC_5-66-66_n2-n77 | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 7.138.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66_n2-n77 DC_5-66-66_n2-n77 | 5 | 0.2 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n77 | 0.5 |

### 7.138.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.139  DC_5-66_n5-n77

### 7.139.1	Operating bands for DC

Table 7.139.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5-66_n5-n77 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.139.2	Configuration for DC

Table 7.139.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-66A_n5A-n77A | DC_5A_n77A DC_66A_n5A DC_66A_n77A | CA_5A-66A | CA_n5A-n77A |
| DC_5A-66A-66A_n5A-n77A | DC_5A_n77A DC_66A_n5A DC_66A_n77A | CA_5A-66A-66A | CA_n5A-n77A |

### 7.139.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 7.139.4	∆TIB and ∆RIB values

For DC_5-66_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.139.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n5-n77 DC_5-66-66_n5-n77 | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 7.139.4-2: ΔRIB.c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66_n5-n77 DC_5-66-66_n5-n77 | 5 | 0.2 |
|  | 66 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

### 7.139.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.140  DC_3-8_n1-n28

### 7.140.1	 Operating bands for DC

Table 7.140.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n1-n28 | 3 | 1710 MHz | – | 1755 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 7.140.2 	Configuration for DC

Table 7.140.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-8A_n1A-n28A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n28 | 15 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |

### 7.140.3	 Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.140.4	 ∆TIB and ∆RIB values

Relaxation values of DC_1-3-8_n28 are re-used

Table 7.140.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n1-n28 | 3 | 0.3 |
|  | 8 | 0.6 |
|  | n1 | 0.3 |
|  | n28 | 0.6 |

Table 7.140.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8_n1-n28 | 8 | 0.2 |
|  | n28 | 0.2 |

### 7.140.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.141	DC_1-8_n28-n79

### 7.141.1	Operating bands for DC

Table 7.141.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n28-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.141.2	Channel bandwidths per operating band for DC

Table 7.141.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A_n28A-n79A2 | DC_1A_n28A DC_1A_n79A DC_8A_n28A DC_8A_n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
| NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.141.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8_n28, DC_1-8_n79, DC_1_n28-n79 and DC_8_n28-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.141.4	∆TIB and ∆RIB values

For DC_1-8_n28-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.141.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n28-n79 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n79 | 0.8 |

Table 7.141.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n28-n79 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n28 | 0.6 |
|  | n79 | 0.5 |

### 7.141.5	MSD

As mentioned in 7.141.3, there is no need to specify additional MSD requirement for this UL DC configuration

## 7.142	DC_1-8_n77-n79

### 7.142.1	Operating bands for DC

Table 7.142.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 945 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.142.2	Channel bandwidths per operating band for DC

Table 7.142.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
| DC_1A-8A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 330 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | CA_n77(2A) BCS1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 7.142.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8_n77, DC_1-8_n79, DC_1_n77-n79 and DC_8_n77-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.142.4	∆TIB and ∆RIB values

For DC_1-8_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.142.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n77-n79 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 7.142.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8_n77-n79 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | n77 | 0.5 |
|  | n79 | 0 |

### 7.142.5	MSD

As mentioned in 7.142.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.143	DC_3-8_n77-n79

### 7.143.1	Operating bands for DC

Table 7.143.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n77-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 945 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.143.2	Channel bandwidths per operating band for DC

Table 7.143.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_3A-8A_n77A-n79A | DC_3A_n77A DC_3A_n79A DC_8A_n77A DC_8A_n79A | 3 | 15 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

### 7.143.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_3-8_n77, DC_3-8_n79, DC_3_n77-n79 and DC_8_n77-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.143.4	∆TIB and ∆RIB values

For DC_3-8_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.143.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n77-n79 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 7.143.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8_n77-n79 | 3 | 0.6 |
|  | 8 | 0.3 |
|  | n77 | 0.8 |
|  | n79 | 0 |

### 7.143.5	MSD

As mentioned in 7.143.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.144	DC_8-41_n3-n77

### 7.144.1	Operating bands for DC

Table 7.144.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-41_n3-n77 | 8 | 880 MHz | – | 915 MHz | 945 MHz | – | 960 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.144.2	Channel bandwidths per operating band for DC

Table 7.144.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-41A_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_41A_n3A DC_41A_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 41 | 15 | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A-41C_n3A-n77A | DC_8A_n3A DC_8A_n77A DC_41A_n3A DC_41C_n3A DC_41A_n77A DC_41C_n77A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 41 | CA_41C BCS3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.144.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-41_n3, DC_8-41_n77, DC_8_n3-n77 and DC_41_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.144.4	∆TIB and ∆RIB values

For DC_8-41_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.144.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-41_n3-n77 | 8 | 0.6 |
|  | 41 | 0.3X |
|  |  | 0.8Y |
|  | n3 | 0.6 |
|  | n77 | 0.8 |
| NOTE X: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE Y: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

Table 7.144.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-41_n3-n77 | 8 | 0.2 |
|  | 41 | 0X |
|  |  | 0.5Y |
|  | n3 | 0.2 |
|  | n77 | 0.5 |
| NOTE X: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz. NOTE Y: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

### 7.144.5	MSD

As mentioned in 7.144.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.145	DC_8-42_n1-n77

### 7.145.1	Operating bands for DC

Table 7.145.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-42_n1-n77 | 8 | 880 MHz | – | 915 MHz | 945 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.145.2	Channel bandwidths per operating band for DC

Table 7.145.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-42A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_42A_n1A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_8A-42C_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_42A_n1A DC_42C_n1A | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 42 | CA_42C BCS0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

### 7.145.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-42_n1, DC_8-42_n77, DC_8_n1-n77 and DC_42_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.145.4	∆TIB and ∆RIB values

For DC_8-42_n1-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.145.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-42_n1-n77 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.145.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-42_n1-n77 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

### 7.145.5	MSD

As mentioned in 7.145.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.146	DC_13-66_n5-n77

### 7.146.1	Operating bands for DC

Table 7.146.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_13-66_n5-n77 | 13, 66 | n5, n77 |

### 7.146.2	Channel bandwidths per operating band for DC

Table 7.146.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_13A-66A_n5A-n77A | DC_13A_n77A DC_66A_n5A DC_66A_n77A | 13 | See CA_13A-66A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | See CA_n5A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.146.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.146.4	∆TIB and ∆RIB values

Relaxation values are copied from DC_5-13_n66.

Table 7.146.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n5-n77 | 13 | 0.3 |
|  | 66 | 0.3 |
|  | n5 | 0.3 |
|  | n77 | 0.8 |

Table 7.146.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13-66_n5-n77 | 13 | 0 |
|  | 66 | 0 |
|  | n5 | 0 |
|  | n77 | 0.5 |

### 7.146.5	MSD

There are no additional MSD requirements for this band combination.

## 7.147	DC_2-66_n5-n77

### 7.147.1	Operating bands for DC

Table 7.147.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2-66_n5-n77 | 2, 66 | n5, n77 |

### 7.147.2	Channel bandwidths per operating band for DC

Table 7.147.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_2A-66A_n5A-n77A | DC_66A_n5A DC_66A_n77A DC_5A_n77A | 2 | See CA_2A-66A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n5 | See CA_n5A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.147.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.147.4	∆TIB and ∆RIB values

Relaxation values are copied from DC_2-5_n66

Table 7.147.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n5-n77 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |
|  | n77 | 0.8 |

Table 7.147.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n5-n77 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n5 | 0 |
|  | n77 | 0.5 |

### 7.147.5	MSD

There are no additional MSD requirements for this band combination.

## 7.148	DC_2-13_n66-n77

### 7.148.1	Operating bands for DC

Table 7.148.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2-13_n66-n77 | 2, 13 | n66, n77 |

### 7.148.2	Channel bandwidths per operating band for DC

Table 7.148.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_2A-13A_n66A-n77A | DC_2A_n66A | 2 | See CA_2A-13A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.148.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.148.4	∆TIB and ∆RIB values

Relaxation values are copied from DC_2-13_n66

Table 7.148.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n66-n77 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | n66 | 0.5 |
|  | n77 | 0.8 |

Table 7.148.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-13_n66-n77 | 2 | 0.3 |
|  | 13 | 0 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 7.148.5	MSD

There are no additional MSD requirements for this band combination.

## 7.149	DC_2-66_n66-n77

### 7.149.1	Operating bands for DC

Table 7.149.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2-66_n66-n77 | 2, 66 | n66, n77 |

### 7.149.2	Channel bandwidths per operating band for DC

Table 7.149.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_2A-66A_n66A-n77A | DC_2A_n66A | 2 | See CA_2A-66A BCS0 in Table 5.6A.1-2 of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.149.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 7.149.4	∆TIB and ∆RIB values

Relaxation values are copied from DC_2_n66-n77

Table 7.149.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n66-n77 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 7.149.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n66-n77 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 7.149.5	MSD

There are no additional MSD requirements for this band combination.

## 7.150		DC_3-32_n1-n28

### 7.150.1	 Operating bands for DC

Table 7.150.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-32_n1-n28 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 32 | N/A | – | N/A | 1452 MHz | – | 1496 MHz | SDL |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 7.150.2 	Configuration for DC

Table 7.150.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-32A_n1A-n28A | DC_3A_n1A DC_3A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | 32 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C-32A_n1A-n28A | DC_3A_n1A DC_3A_n28A DC_3C_n1A DC_3C_n28A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 32 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.150.3	 Co-existence studies

Co-existence studies of DC_3-32_n1-n28 are already covered by the fallback configurations.

### 7.150.4	 ∆TIB and ∆RIB values

Table 7.150.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-32_n1-n28 | n1 | 0.3 |
|  | 3 | 0.3 |
|  | n28 | 0.6 |

Table 7.150.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-32_n1-n28 | n1 | 0.2 |
|  | 3 | 0 |
|  | n28 | 0.2 |
|  | 32 | 0 |

## 7.151		DC_20-32_n1-n28

### 7.151.1	 Operating bands for DC

Table 7.151.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20-32_n1-n28 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 32 | N/A | – | N/A | 1452 MHz | – | 1496 MHz | SDL |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 7.151.2 	Configuration for DC

Table 7.151.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_20A-32A_n1A-n28A | DC_20A_n1A DC_20A_n28A | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | 32 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.151.3	 Co-existence studies

Co-existence studies of DC_20-32_n1-n28 are already covered by the fallback configurations.

### 7.151.4	 ∆TIB and ∆RIB values

Table 7.151.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-32_n1-n28 | n1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.7 |

Table 7.151.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_20-32_n1-n28 | n1 | 0 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | 32 | 0 |

## 7.152		DC_1-38_n3-n78

### 7.152.1	Operating bands for DC

Table 7.152.1-1: LTE 2 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-38_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.152.2	Configuration for DC

Table 7.152.2-1: Inter-band DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-38A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_38A_n3A DC_38A_n78A | CA_1A-38A | CA_n3A-n78A |

### 7.152.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back modes.

### 7.152.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below, which are the same with DC_1-3_n38-n78.

Table 7.152.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-38_n3-n78 | 1 | 0.5 |
|  | 38 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 7.152.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-38_n3-n78 | 1 | 0 |
|  | 38 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 7.152.5	MSD

Based on co-existence studies no additional MSD is needed.

## 7.153		DC_7-38_n3-n78

### 7.153.1	Operating bands for DC

Table 7.153.1-1: LTE 2 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-38_n3-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.153.2	Configuration for DC

Table 7.153.2-1: Inter-band DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_7A-38A_n3A-n78A10 | N/A | N/A | CA_n3A-n78A |
| NOTE 10: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |  |  |

### 7.153.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back modes.

### 7.153.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below, which are the same with DC_1-3_n38-n78.

Table 7.153.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-38_n3-n78 | n3 | 0.6 |
|  | n78 | 0.8 |

Table 7.153.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-38_n3-n78 | 7 | 0.5 |
|  | 38 | 0.5 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 7.153.5	MSD

Based on co-existence studies no additional MSD is needed.

## 7.154		DC_20-38_n3-n78

### 7.154.1	Operating bands for DC

Table 7.154.1-1: LTE 2 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20-38_n3-n78 | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.154.2	Configuration for DC

Table 7.154.2-1: Inter-band DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_20A-38A_n3A-n78A | DC_20A_n3A DC_20A_n78A DC_38A_n3A DC_38A_n78A | CA_20A-38A | CA_n3A-n78A |

### 7.154.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back modes.

### 7.154.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below, which are the same with DC_3-20_n38-n78.

Table 7.154.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20-38_n3-n78 | 20 | 0.6 |
|  | 38 | 0.5 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 7.154.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_20-38_n3-n78 | 20 | 0.2 |
|  | 38 | 0.4 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 7.154.5	MSD

Based on co-existence studies no additional MSD is needed.

## 7.155	DC_2-66_n25-n66

### 7.155.1	Operating bands for DC

Table 7.155.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n25-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 7.155.2	Channel bandwidths per operating band for DC

Table 7.155.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-66A_n25A-n66A | DC_2A_n66A DC_66A_n25A | CA_2A-66A | CA_n25A-n66A |

### 7.155.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.155.4	∆TIB and ∆RIB values

For DC_2-66_n25-n66, the TIB,c and RIB,c values are derived from DC_2_n66 and given in the tables below.

Table 7.155.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n25-n66 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 7.155.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n25-n66 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

### 7.155.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.156	DC_2-5_n66-n78

### 7.156.1	Operating bands for DC

Table 7.156.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-5_n66-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.156.2	Channel bandwidths per operating band for DC

Table 7.156.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A_n66A-n78A | DC_2A_n66A DC_5A_n66A DC_2A_n78A DC_5A_n78A | CA_2A-5A | CA_n66A-n78A |

### 7.156.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.156.4	∆TIB and ∆RIB values

For DC_2-5_n66-n78, the TIB,c and RIB,c values are derived from DC_2-5-66_n77 and given in the tables below.

Table 7.156.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n66-n78 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | n66 | 0.5 |
|  | n78 | 0.8 |

Table 7.156.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n66-n78 | 2 | 0.3 |
|  | 5 | 0 |
|  | n66 | 0.3 |
|  | n78 | 0.5 |

### 7.156.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.157	DC_2-5_ n2-n78

### 7.157.1	Operating bands for DC

Table 7.157.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-5_n2-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.157.2	Channel bandwidths per operating band for DC

Table 7.157.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A_n2A-n78A | DC_5A_n2A DC_2A_n78A DC_5A_n78A | CA_2A-5A | CA_n2A-n78A |

### 7.157.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.157.4	∆TIB and ∆RIB values

For DC_2-5_n2-n78, the TIB,c and RIB,c values are derived from DC_2-5_n78 and given in the tables below.

Table 7.157.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5_n2-n78 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 7.157.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5_n2-n78 | 2 | 0.2 |
|  | 5 | 0.2 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

### 7.157.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.158	DC_2-66_ n2-n78

### 7.158.1	Operating bands for DC

Table 7.158.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n2-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.158.2	Channel bandwidths per operating band for DC

Table 7.158.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-66A_n2A-n78A | DC_66A_n2A DC_2A_n78A DC_66A_n78A | CA_2A-66A | CA_n2A-n78A |

### 7.158.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.158.4	∆TIB and ∆RIB values

For DC_2-66_n2-n78, the TIB,c and RIB,c values are derived from DC_2-66_n78 and given in the tables below.

Table 7.158.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n2-n78 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 7.158.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n2-n78 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n78 | 0.5 |

### 7.158.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.159	DC_5-66_ n2-n78

### 7.159.1	Operating bands for DC

Table 7.159.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5-66_n2-n78 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.159.2	Channel bandwidths per operating band for DC

Table 7.159.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-66A_n2A-n78A | DC_5A_n2A DC_66A_n2A DC_5A_n78A DC_66A_n78A | CA_5A-66A | CA_n2A-n78A |

### 7.159.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.159.4	∆TIB and ∆RIB values

For DC_5-66_n2-n78, the TIB,c and RIB,c values are derived from DC_2-5-66_n77 and given in the tables below.

Table 7.159.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-66_n2-n78 | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |
|  | n78 | 0.8 |

Table 7.159.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-66_n2-n78 | 5 | 0 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n78 | 0.5 |

### 7.159.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.160	DC_2-7_n2-n78

### 7.160.1	Operating bands for DC

Table 7.160.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-7_n2-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.160.2	Channel bandwidths per operating band for DC

Table 7.160.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A_n2A-n78A | DC_7A_n2A DC_2A_n78A DC_7A_n78A | CA_2A-7A | CA_n2A-n78A |

### 7.160.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.160.4	∆TIB and ∆RIB values

For DC_2-7_n2-n78, the TIB,c and RIB,c values are derived from DC_2_n7-n78 and given in the tables below.

Table 7.160.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n2-n78 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 7.160.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n2-n78 | 2 | 0.2 |
|  | 7 | 0.5 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

### 7.160.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.161	DC_2-7_n71-n78

### 7.161.1	Operating bands for DC

Table 7.161.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-7_n71-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.161.2	Channel bandwidths per operating band for DC

Table 7.161.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A_n71A-n78A | DC_2A_n71A DC_7A_n71A DC_2A_n78A DC_7A_n78A | CA_2A-7A | CA_n71A-n78A |

### 7.161.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.161.4	∆TIB and ∆RIB values

For DC_2-7_n71-n78, the TIB,c and RIB,c values are derived from DC_2-7-71_n78 and given in the tables below.

Table 7.161.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7_n71-n78 | 2 | 0.6 |
|  | 7 | 0.6 |
|  | n71 | 0.6 |
|  | n78 | 0.8 |

Table 7.161.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7_n71-n78 | 2 | 0.2 |
|  | 7 | 0.2 |
|  | n71 | 0.2 |
|  | n78 | 0.5 |

### 7.161.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.162	DC_2-12_n2-n78

### 7.162.1	Operating bands for DC

Table 7.162.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-12_n2-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.162.2	Channel bandwidths per operating band for DC

Table 7.162.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A_n2A-n78A | DC_12A_n2A DC_2A_n78A DC_7A_n78A | CA_2A-12A | CA_n2A-n78A |

### 7.162.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.162.4	∆TIB and ∆RIB values

For DC_2-12_n2-n78, the TIB,c and RIB,c values are derived from DC_2-12_n78 and given in the tables below.

Table 7.162.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n2-n78 | 2 | 0.6 |
|  | 12 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 7.162.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12_n2-n78 | 2 | 0.2 |
|  | 12 | 0.2 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

### 7.162.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.163	DC_2-12_ n66-n78

### 7.163.1	Operating bands for DC

Table 7.163.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-12_n66-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.163.2	Channel bandwidths per operating band for DC

Table 7.163.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A_n66A-n78A | DC_2A_n66A DC_12A_n66A DC_2A_n78A DC_12A_n78A | CA_2A-12A | CA_n66A-n78A |

### 7.163.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.163.4	∆TIB and ∆RIB values

For DC_2-12_n66-n78, the TIB,c and RIB,c values are derived from DC_2-12-66_n78 and given in the tables below.

Table 7.163.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12_n66-n78 | 2 | 0.5 |
|  | 12 | 0.3 |
|  | n66 | 0.5 |
|  | n78 | 0.8 |

Table 7.163.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12_n66-n78 | 2 | 0.3 |
|  | 12 | 0 |
|  | n66 | 0.3 |
|  | n78 | 0.5 |

### 7.163.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.164	DC_2-66_n71-n78

### 7.164.1	Operating bands for DC

Table 7.164.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n71-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.164.2	Channel bandwidths per operating band for DC

Table 7.164.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-66A_n71A-n78A | DC_2A_n71A DC_66A_n71A DC_2A_n78A DC_66A_n78A | CA_2A-66A | CA_n71A-n78A |

### 7.164.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.164.4	∆TIB and ∆RIB values

For DC_2-66_n71-n78, the TIB,c and RIB,c values are derived from DC_2-66-71_n78 and given in the tables below.

Table 7.164.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n71-n78 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n71 | 0.3 |
|  | n78 | 0.5 |

Table 7.164.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n71-n78 | 2 | 0.3 |
|  | 66 | 0.5 |
|  | n71 | 0 |
|  | n78 | 0.5 |

### 7.164.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.165	DC_2-71_n2-n78

### 7.165.1	Operating bands for DC

Table 7.165.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-71_n2-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.165.2	Channel bandwidths per operating band for DC

Table 7.165.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-71A_n2A-n78A | DC_71A_n2A DC_2A_n78A DC_7A_n78A | CA_2A-71A | CA_n2A-n78A |

### 7.165.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.165.4	∆TIB and ∆RIB values

For DC_2-71_n2-n78, the TIB,c and RIB,c values are derived from DC_2-71_n78 and given in the tables below.

Table 7.165.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-71_n2-n78 | 2 | 0.6 |
|  | 71 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 7.165.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-71_n2-n78 | 2 | 0.2 |
|  | 71 | 0.2 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

### 7.165.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.166	DC_2-71_n66-n78

### 7.166.1	Operating bands for DC

Table 7.166.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-71_n66-n78 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.166.2	Channel bandwidths per operating band for DC

Table 7.166.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-71A_n66A-n78A | DC_2A_n66A DC_71A_n66A DC_2A_n78A DC_71A_n78A | CA_2A-71A | CA_n66A-n78A |

### 7.166.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.166.4	∆TIB and ∆RIB values

For DC_2-71_n66-n78, the TIB,c and RIB,c values are derived from DC_2-66-71_n78 and given in the tables below.

Table 7.166.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-71_n66-n78 | 2 | 0.5 |
|  | 71 | 0.3 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |

Table 7.166.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-71_n66-n78 | 2 | 0.3 |
|  | 71 | 0 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |

### 7.166.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.167	DC_5-7_n2-n78

### 7.167.1	Operating bands for DC

Table 7.167.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5-7_n2-n78 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.167.2	Channel bandwidths per operating band for DC

Table 7.167.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-7A_n2A-n78A | DC_5A_n2A DC_7A_n2A DC_5A_n78A DC_7A_n78A | CA_5A-7A | CA_n2A-n78A |

### 7.167.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.167.4	∆TIB and ∆RIB values

For DC_5-7_n2-n78, the TIB,c and RIB,c values are derived from DC_3-5-7_n78 and given in the tables below.

Table 7.167.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-7_n2-n78 | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 7.167.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-7_n2-n78 | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

### 7.167.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.168	DC_5-7_n66-n78

### 7.168.1	Operating bands for DC

Table 7.168.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_5-7_n66-n78 | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.168.2	Channel bandwidths per operating band for DC

Table 7.168.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-7A_n66A-n78A | DC_5A_n66A DC_7A_n66A DC_5A_n78A DC_7A_n78A | CA_5A-7A | CA_n66A-n78A |

### 7.168.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.168.4	∆TIB and ∆RIB values

For DC_5-7_n66-n78, the TIB,c and RIB,c values are derived from DC_7-12-66_n78 and given in the tables below.

Table 7.168.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-7_n66-n78 | 5 | 0.5 |
|  | 7 | 0.8 |
|  | n66 | 1 |
|  | n78 | 0.8 |

Table 7.168.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-7_n66-n78 | 5 | 0.2 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |

### 7.168.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.169	DC_7-12_n2-n78

### 7.169.1	Operating bands for DC

Table 7.169.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-12_n2-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.169.2	Channel bandwidths per operating band for DC

Table 7.169.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-12A_n2A-n78A | DC_7A_n2A DC_12A_n2A DC_7A_n78A DC_12A_n78A | CA_7A-12A | CA_n2A-n78A |

### 7.169.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.169.4	∆TIB and ∆RIB values

For DC_7-12_n2-n78, the TIB,c and RIB,c values are derived from DC_2-7-12_n78 and given in the tables below.

Table 7.169.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-12_n2-n78 | 7 | 0.6 |
|  | 12 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 7.169.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-12_n2-n78 | 7 | 0.2 |
|  | 12 | 0.2 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

### 7.169.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.170	DC_7-12_ n66-n78

### 7.170.1	Operating bands for DC

Table 7.170.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-12_n66-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.170.2	Channel bandwidths per operating band for DC

Table 7.170.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-12A_n66A-n78A | DC_7A_n66A DC_12A_n66A DC_7A_n78A DC_12A_n78A | CA_7A-12A | CA_n66A-n78A |

### 7.170.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.170.4	∆TIB and ∆RIB values

For DC_7-12_n66-n78, the TIB,c and RIB,c values are derived from DC_7-12-66_n78 and given in the tables below.

Table 7.170.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-12_n66-n78 | 7 | 0.8 |
|  | 12 | 0.5 |
|  | n66 | 1 |
|  | n78 | 0.8 |

Table 7.170.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-12_n66-n78 | 7 | 0.5 |
|  | 12 | 0.2 |
|  | n66 | 0.5 |
|  | n78 | 0.5 |

### 7.170.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.171	DC_7-66_n2-n78

### 7.171.1	Operating bands for DC

Table 7.171.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-66_n2-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.171.2	Channel bandwidths per operating band for DC

Table 7.171.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-66A_n2A-n78A | DC_7A_n2A DC_66A_n2A DC_7A_n78A DC_66A_n78A | CA_7A-66A | CA_n2A-n78A |

### 7.171.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.171.4	∆TIB and ∆RIB values

For DC_7-66_n2-n78, the TIB,c and RIB,c values are derived from DC_2-7-66_n78 and given in the tables below.

Table 7.171.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n2-n78 | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 7.171.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n2-n78 | 7 | 0 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n78 | 0.5 |

### 7.171.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.172	DC_7-66_n71-n78

### 7.172.1	Operating bands for DC

Table 7.172.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-66_n71-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.172.2	Channel bandwidths per operating band for DC

Table 7.172.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-66A_n71A-n78A | DC_7A_n71A DC_66A_n71A DC_7A_n78A DC_66A_n78A | CA_7A-66A | CA_n71A-n78A |

### 7.172.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.172.4	∆TIB and ∆RIB values

For DC_7-66_n71-n78, the TIB,c and RIB,c values are derived from DC_7-66-71_n78 and given in the tables below.

Table 7.172.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66_n71-n78 | 7 | 0.6 |
|  | 66 | 0.6 |
|  | n71 | 0.3 |
|  | n78 | 0.8 |

Table 7.172.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66_n71-n78 | 7 | 0.2 |
|  | 66 | 0.2 |
|  | n71 | 0 |
|  | n78 | 0.5 |

### 7.172.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.173	DC_72-71_n2-n78

### 7.173.1	Operating bands for DC

Table 7.173.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-71_n2-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.173.2	Channel bandwidths per operating band for DC

Table 7.173.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-71A_n2A-n78A | DC_7A_n2A DC_71A_n2A DC_7A_n78A DC_71A_n78A | CA_7A-71A | CA_n2A-n78A |

### 7.173.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.173.4	∆TIB and ∆RIB values

For DC_7-71_n2-n78, the TIB,c and RIB,c values are derived from DC_2-7-71_n78 and given in the tables below.

Table 7.173.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-71_n2-n78 | 7 | 0.6 |
|  | 71 | 0.6 |
|  | n2 | 0.6 |
|  | n78 | 0.8 |

Table 7.173.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-71_n2-n78 | 7 | 0.2 |
|  | 71 | 0.2 |
|  | n2 | 0.2 |
|  | n78 | 0.5 |

### 7.173.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.174	DC_7-71_n66-n78

### 7.174.1	Operating bands for DC

Table 7.174.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-71_n66-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.174.2	Channel bandwidths per operating band for DC

Table 7.174.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-71A_n66A-n78A | DC_7A_n66A DC_71A_n66A DC_7A_n78A DC_71A_n78A | CA_7A-71A | CA_n66A-n78A |

### 7.174.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.174.4	∆TIB and ∆RIB values

For DC_7-71_n66-n78, the TIB,c and RIB,c values are derived from DC_7-66-71_n78 and given in the tables below.

Table 7.174.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-71_n66-n78 | 7 | 0.6 |
|  | 71 | 0.3 |
|  | n66 | 0.6 |
|  | n78 | 0.8 |

Table 7.174.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-71_n66-n78 | 7 | 0.2 |
|  | 71 | 0 |
|  | n66 | 0.2 |
|  | n78 | 0.5 |

### 7.174.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.175	DC_12-66_ n2-n78

### 7.175.1	Operating bands for DC

Table 7.175.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_12-66_n2-n78 | 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.175.2	Channel bandwidths per operating band for DC

Table 7.175.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_12A-66A_n2A-n78A | DC_12A_n2A DC_66A_n2A DC_12A_n78A DC_66A_n78A | CA_12A-66A | CA_n2A-n78A |

### 7.175.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.175.4	∆TIB and ∆RIB values

For DC_12-66_n2-n78, the TIB,c and RIB,c values are derived from DC_2-12-66_n78 and given in the tables below.

Table 7.175.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_12-66_n2-n78 | 12 | 0.3 |
|  | 66 | 0.5 |
|  | n 2 | 0.5 |
|  | n78 | 0.8 |

Table 7.175.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_12-66_n2-n78 | 12 | 0 |
|  | 66 | 0.3 |
|  | n 2 | 0.3 |
|  | n78 | 0.5 |

### 7.175.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.176	DC_66-71_n2-n78

### 7.176.1	Operating bands for DC

Table 7.176.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_66-71_n2-n78 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 7.176.2	Channel bandwidths per operating band for DC

Table 7.176.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_66A-71A_n2A-n78A | DC_66A_n2A DC_71A_n2A DC_66A_n78A DC_71A_n78A | CA_66A-71A | CA_n71A-n78A |

### 7.176.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 7.176.4	∆TIB and ∆RIB values

For DC_66-71_n2-n78, the TIB,c and RIB,c values are derived from DC_2-66-71_n78 and given in the tables below.

Table 7.176.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66-71_n2-n78 | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n2 | 0.5 |
|  | n78 | 0.5 |

Table 7.176.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66-71_n2-n78 | n66 | 0.5 |
|  | 71 | 0 |
|  | 2 | 0.3 |
|  | n78 | 0.5 |

### 7.176.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 7.177	DC_1-3_n28-n75

### 7.177.1	 Operating bands for DC

Table 7.177.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n28-n75 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n75 | N/A | – | N/A MHz | 1432 MHz | – | 1517 MHz | SDL |

### 7.177.2 	Configuration for DC

Table 7.177.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n28A-n75A | DC_1A_n28A DC_3A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 110 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| DC_1A-3C_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_3C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 130 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

### 7.177.3	 Co-existence studies

Co-existence studies of DC_1-3_n28-n75 are already covered by the fallback configurations.

### 7.177.4	 ∆TIB and ∆RIB values

Table 7.177.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n28-n75 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n28 | 0.6 |

Table 7.177.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n28-n75 | 1 | 0.2 |
|  | 3 | 0 |
|  | n28 | 0.2 |
|  | n75 | 0 |

## 7.178	DC_3-20_n28-n75

### 7.178.1	 Operating bands for DC

Table 7.178.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-20_n28-n75 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n75 | N/A | – | N/A MHz | 1432 MHz | – | 1517 MHz | SDL |

### 7.178.2 	Configuration for DC

Table 7.178.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A_n28A-n75A | DC_3A_n28A DC_20A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 110 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| DC_3C-20A_n28A-n75A | DC_20A_n28A DC_3A_n28A DC_3C_n28A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 130 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

### 7.178.3	 Co-existence studies

Co-existence studies of DC_3-20_n28-n75 are already covered by the fallback configurations.

### 7.178.4	 ∆TIB and ∆RIB values

Table 7.178.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20_n28-n75 | 3 | 0.3 |
|  | 20 | 0.5 |
|  | n28 | 0.5 |

Table 7.178.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20_n28-n75 | 3 | 0.5 |
|  | 20 | 0 |
|  | n28 | 0.5 |
|  | n75 | 0 |

## 7.179	DC_1-20_n28-n75

### 7.179.1	 Operating bands for DC

Table 7.179.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-20_n28-n75 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n75 | N/A | – | N/A MHz | 1432 MHz | – | 1517 MHz | SDL |

### 7.179.2 	Configuration for DC

Table 7.179.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-20A_n28A-n75A | DC_1A_n28A DC_20A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 110 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

### 7.179.3	 Co-existence studies

Co-existence studies of DC_1-20_n28-n75 are already covered by the fallback configurations.

### 7.179.4	 ∆TIB and ∆RIB values

Table 7.179.4-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20_n28-n75 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.7 |

Table 7.179.4-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20_n28-n75 | 1 | 0 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |
|  | n75 | 0 |

## 7.180	DC_1-7_n3-n38

### 7.180.1	Operating bands for DC

Table 7.180.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7_n3-n38 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

### 7.180.2	Channel bandwidths per operating band for DC

Table 7.180.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-7A_n3A-n38A | DC_1A_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 110 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |

### 7.180.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.180.4	∆TIB and ∆RIB values

For DC_1-7_n3-n38, the TIB,c and RIB,c values are given in the tables below.

Table 7.180.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7_n3-n38 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n3 | 0.6 |
|  | n38 | 0.5 |

Table 7.180.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7_n3-n38 | 1 | 0 |
|  | 7 | 0 |
|  | n3 | 0 |
|  | n38 | 0.2 |

### 7.180.5	MSD

All MSD requirements for DC_1-7_n3-n38 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.181	DC_7-20_n3-n38

### 7.181.1	Operating bands for DC

Table 7.181.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-20_n3-n38 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

### 7.181.2	Channel bandwidths per operating band for DC

Table 7.181.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_7A-20A_n3A-n38A | DC_20A_n3A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 110 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |

### 7.181.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.181.4	∆TIB and ∆RIB values

For DC_7-20_n3-n38, the TIB,c and RIB,c values are given in the tables below.

Table 7.181.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20_n3-n38 | 7 | 0.5 |
|  | 20 | 0.3 |
|  | n3 | 0.5 |
|  | n38 | 0.5 |

Table 7.181.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-20_n3-n38 | 7 | 0 |
|  | 20 | 0.2 |
|  | n3 | 0 |
|  | n38 | 0.2 |

### 7.181.5	MSD

All MSD requirements for DC_7-20_n3-n38 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.182	DC_1-11_n3-n79

### 7.182.1	Operating bands for DC

Table 7.182.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-11_n3-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.182.2	Channel bandwidths per operating band for DC

Table 7.182.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-11A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_11A_n3A DC_11A_n79A | 1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 11 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |

### 7.182.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-11_n3, DC_1-11_n79, DC_1_n3-n79 and DC_11_n3-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.182.4	∆TIB and ∆RIB values

For DC_1-11_n3-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.182.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n3-n79 | 1 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n79 | 0.8 |

Table 7.182.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n3-n79 | 1 | 0 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n79 | 0.5 |

### 7.182.5	MSD

As mentioned in 7.182.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.183	DC_1-11_n77-n79

### 7.183.1	Operating bands for DC

Table 7.183.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-11_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.183.2	Channel bandwidths per operating band for DC

Table 7.183.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-11A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_11A_n77A DC_11A_n79A | 1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 11 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| DC_1A-11A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_11A_n77A DC_11A_n79A | 1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 330 |
|  |  | 11 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | CA_n77(2A) BCS1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |

### 7.183.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-11_n77, DC_1-11_n79, DC_1_n77-n79 and DC_11_n77-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.183.4	∆TIB and ∆RIB values

For DC_1-11_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.183.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11_n77-n79 | 1 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |
|  | n79 | 0 |

Table 7.183.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-11_n77-n79 | 1 | 0.2 |
|  | 11 | 0 |
|  | n77 | 0.5 |
|  | n79 | 0 |

### 7.183.5	MSD

As mentioned in 7.183.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.184	DC_8-11_n3-n79

### 7.184.1	Operating bands for DC

Table 7.184.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-11_n3-n79 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.184.2	Channel bandwidths per operating band for DC

Table 7.184.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-11A_n3A-n79A | DC_8A_n3A DC_8A_n79A DC_11A_n3A DC_11A_n79A | 8 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 11 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |

### 7.184.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-11_n3, DC_8-11_n79, DC_8_n3-n79 and DC_11_n3-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.184.4	∆TIB and ∆RIB values

For DC_8-11_n3-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.184.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-11_n3-n79 | 8 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n79 | 0.8 |

Table 7.184.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-11_n3-n79 | 8 | 0 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n79 | 0.5 |

### 7.184.5	MSD

As mentioned in 7.184.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.185	DC_8-11_n77-n79

### 7.185.1	Operating bands for DC

Table 7.185.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-11_n77-n79 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.185.2	Channel bandwidths per operating band for DC

Table 7.185.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-11A_n77A-n79A | DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A | 8 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 11 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |
| DC_8A-11A_n77(2A)-n79A | DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A | 8 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 320 |
|  |  | 11 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |

### 7.185.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-11_n77, DC_8-11_n79, DC_8_n77-n79 and DC_11_n77-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.185.4	∆TIB and ∆RIB values

For DC_8-11_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.185.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-11_n77-n79 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 7.185.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-11_n77-n79 | 8 | 0.2 |
|  | 11 | 0 |
|  | n77 | 0.5 |
|  | n79 | 0 |

### 7.185.5	MSD

As mentioned in 7.185.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.186	DC_8-41_n1-n77

### 7.186.1	Operating bands for DC

Table 7.186.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-41_n1-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.186.2	Channel bandwidths per operating band for DC

Table 7.186.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-41A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_41A_n1A DC_41A_n77A | 8 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 41 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
| DC_8A-41C_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_41A_n1A DC_41A_n77A | 8 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 41 | See CA_41C bandwidth combination set 3 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |

### 7.186.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-41_n1, DC_8-41_n77, DC_8_n1-n77 and DC_41_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.186.4	∆TIB and ∆RIB values

For DC_8-41_n1-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.186.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-41_n1-n77 | 8 | 0.6 |
|  | 41 | 0.5 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.186.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-41_n1-n77 | 8 | 0.2 |
|  | 41 | 0 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

### 7.186.5	MSD

As mentioned in 7.186.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.187	DC_8-42_n1-n3

### 7.187.1	Operating bands for DC

Table 7.187.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-42_n1-n3 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |

### 7.187.2	Channel bandwidths per operating band for DC

Table 7.187.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-42A_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_42A_n1A DC_42A_n3A | 8 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 80 |
|  |  | 42 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |
| DC_8A-42C_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_42A_n1A DC_42C_n1A DC_42A_n3A DC_42C_n3A | 8 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  | 100 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |

### 7.187.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-42_n1, DC_8-42_n3, DC_8_n1-n3 and DC_42_n1-n3. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.187.4	∆TIB and ∆RIB values

For DC_8-42_n1-n3, the TIB,c and RIB,c values are given in the tables below.

Table 7.187.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-42_n1-n3 | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n3 | 0.6 |

Table 7.187.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-42_n1-n3 | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n1 | 0 |
|  | n3 | 0.2 |

### 7.187.5	MSD

As mentioned in 7.187.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.188	DC_8-11_n1-n77

### 7.188.1	Operating bands for DC

Table 7.188.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-11_n1-n77 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 7.188.2	Channel bandwidths per operating band for DC

Table 7.188.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Channel bandwidth [MHz] | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-11A_n1A-n77A | DC_8A_n1A DC_8A_n77A DC_11A_n1A DC_11A_n77A | 8 | 5, 10 | 140 |
|  |  | 11 | 5, 10 |  |
|  |  | n1 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
| DC_8A-11A_n1A-n77(2A) | DC_8A_n1A DC_8A_n77A DC_11A_n1A DC_11A_n77A | 8 | 5, 10 | 240 |
|  |  | 11 | 5, 10 |  |
|  |  | n1 | 5, 10, 15, 20 |  |
|  |  | n77 | n77(2A) |  |

### 7.188.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-11_n1, DC_8-11_n77, DC_8_n1-n77 and DC_11_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.188.4	∆TIB and ∆RIB values

For DC_8-11_n1-n77, the TIB,c and RIB,c values are given in the tables below.

Table 7.188.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-11_n1-n77 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 7.188.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-11_n1-n77 | 8 | 0.2 |
|  | 11 | 0 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

### 7.188.5	MSD

As mentioned in 7.188.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.189	DC_8-41_n1-n3

### 7.189.1	Operating bands for DC

Table 7.189.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-41_n1-n3 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |

_

### 7.189.2	Channel bandwidths per operating band for DC

Table 7.189.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Channel bandwidth [MHz] | Maximum aggregated bandwidth For DL [MHz] |
| DC_8A-41A_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_41A_n1A DC_41A_n3A | 8 | 5, 10 | 80 |
|  |  | 41 | 5, 10, 15, 20 |  |
|  |  | n1 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
| DC_8A-41C_n1A-n3A | DC_8A_n1A DC_8A_n3A DC_41A_n1A DC_41A_n3A | 8 | 5, 10 | 100 |
|  |  | 41 | CA_41C |  |
|  |  | n1 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |

### 7.189.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_8-41_n1, DC_8-41_n3, DC_8_n1-n3 and DC_41_n1-n3. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.189.4	∆TIB and ∆RIB values

For DC_8-41_n1-n3, the TIB,c and RIB,c values are given in the tables below.

Table 7.189.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8-41_n1-n3 | 8 | 0.3 |
|  | 41 | 0.54 |
|  |  | 0.85 |
|  | n1 | 0.5 |
|  | n3 | 0.5 |
| NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

Table 7.189.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_8-41_n1-n3 | 8 | 0 |
|  | 41 | 03 |
|  |  | 0.54 |
|  | n1 | 0 |
|  | n3 | 0 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 - 2690 MHz NOTE 4: The requirement is applied for UE transmitting on the frequency range of 2496 – 2515 MHz. |  |  |

### 7.189.5	MSD

As mentioned in 7.189.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 7.190	DC_1-20_n7-n78

### 7.190.1	Configurations for DC

Table 7.190.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-20A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_20A_n7A DC_20A_n78A |

### 7.190.2	Channel bandwidths per operating band for DC

Table 7.190.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-20A_n7A-n78A | DC_1A_n7A DC_1A_n78A DC_20A_n7A DC_20A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 7.190.3	Coexistence Studies

Co-existence studies of DC_1A-20A_n7A-n78A and protected bands are already covered in the constituent fall-back modes.

### 7.190.4	∆TIB and ∆RIB values

Table 7.190.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20_n7-n78 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n7 | 0.3 |
|  | n78 | 0.8 |

Table 7.190.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20_n7-n78 | n78 | 0.5 |

### 7.190.5	Reference sensitivity exceptions

All MSD requirements for DC_1-20_n7-n78 are covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.191	DC_3-8_n40-n79

### 7.191.1	Operating bands for DC

Table 7.191.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8_n40-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 7.191.2	Configuration for DC

Table 7.191.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | Uplink NR band CA configuration |
| --- | --- | --- | --- |
| DC_3A-8A_n40A-n79A | DC_3A_n40A DC_3A_n79A DC_8A_n40A DC_8A_n79A | CA_3A-8A | CA_n40A-n79A |

### 7.191.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 7.191.4	∆TIB and ∆RIB values

For DC_3-8_n40-n79, the TIB,c and RIB,c values are given in the tables below.

Table 7.191.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n40-n79 | 3 | 0.5 |
|  | 8 | 0.3 |
|  | n40 | 0.5 |
|  | n79 | 0 |

Table 7.191.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-8_n40-n79 | 3 | 0 |
|  | 8 | 0 |
|  | n40 | 0 |
|  | n79 | 0 |

### 7.191.5	MSD

All MSD requirements for DC_3-8_n40-n79 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 7.192	DC_n77-n257_1-3

### 7.192.1		Operating bands for DC

Table 7.192.1-1: DC band combination of NR 2DL/1UL + 2 LTE bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_n77-n257_1-3 | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
|  | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |

### 7.192.2		Channel bandwidths per operating band for DC

Table 7.192.2-1: Supported bandwidths per DC inter-band NR 2DL/1UL + two LTE band

| NR DC and E-UTRA Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_n77A-n257A_1A-3A DC_n77A-n257G_1A-3A DC_n77A-n257H_1A-3A DC_n77A-n257I_1A-3A DC_n77A-n257J_1A-3A DC_n77A-n257K_1A-3A DC_n77A-n257L_1A-3A DC_n77A-n257M_1A-3A | DC_n77A_1A DC_n257A_1A DC_n77A_3A DC_n257A-3A | 3 | - | 550 250 350 450 550 650 750 850 |
|  |  | n77 | CA_n77A-n257A/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |
| DC_n77(2A)-n257A_1A-3A DC_n77(2A)-n257G_1A-3A DC_n77(2A)-n257H_1A-3A DC_n77(2A)-n257I_1A-3A DC_n77(2A)-n257J_1A-3A DC_n77 (2A) -n257K_1A-3A DC_n77(2A)-n257L_1A-3A DC_n77(2A)-n257M_1A-3A |  | 3 | - | 650 350 450 550 650 750 850 950 |
|  |  | n77 | CA_n77(2A)-n257A/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n257 |  |  |

### 7.192.3		Co-existence studies

- For combination of DC_n77A_3A and DC_n77A_1A, the co-existence studies have been covered in TR 37.863-01-01 as DC_3A_n77A and DC_1A_n77A.

- For combination of DC_n257A_3A and DC_n257A_1A, there is no need to consider the harmonic and intermodulation co-existence studies due to the large frequency separation.

### 7.192.4	∆TIB and ∆RIB values

Table 7.192.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_n77-n257_1-3 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | n77 | 0.8 |
|  | n258 | 0 |

Table 7.192.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_n77-n257_1-3 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | n77 | 0.5 |
|  | n258 | 0 |

### 7.192.5	MSD

No additional MSD requirement is needed.

# 8	Dual Connectivity band combinations of LTE 3 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

## 8.1	DC_1-3-8_n28-n77

### 8.1.1	Operating bands for DC

Table 8.1.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-8_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.1.2	Channel bandwidths per operating band for DC

Table 8.1.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |  | 30 MHz |  | 40 MHz |  | 50 MHz |  | 60 MHz |  | 70 MHz |  | 80 MHz |  | 90 MHz |  | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |  |
| DC_1A-3A-8A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  | Yes |  | Yes |  | Yes |  |  |  |
| DC_1A-3A-8A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.1.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-3-8_n28, DC_1-3-8_n77, DC_1-3_n28-n77, DC_1-8_n28-n77 and DC_3-8_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.1.4	∆TIB and ∆RIB values

For DC_1-3-8_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8_n28-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-8_n28-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 8.1.5	MSD

As mentioned in 8.1.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.2	DC_3-7-8_n40-n78

### 8.2.1	Operating bands for DC

Table 8.2.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-8_n40-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.2.2	Configuration for DC

Table 8.2.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A-8A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_8A_n40A DC_8A_n78A | CA_3A-7A-8A | CA_n40A-n78A |

### 8.2.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 8.2.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 8.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8_n40-n78 | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 8 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 8.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8_n40-n78 | 3 | 0.2 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

### 8.2.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.3	DC_3-7-28_n1-n40

### 8.3.1	Operating bands for DC

Table 8.3.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-28_n1-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

### 8.3.2	Configuration for DC

Table 8.3.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A-28A_n1A-n40A | DC_3A_n1A DC_3A_n40A DC_7A_n1A DC_7A_n40A DC_28A_n1A DC_28A_n40A | CA_3A-7A-28A | CA_n1A-n40A |

### 8.3.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 8.3.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 8.3.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n1-n40 | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 28 | 0.6 |
|  | n1 | 0.6 |
|  | n40 | 0.9 |

Table 8.3.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n1-n40 | 3 | 0 |
|  | 7 | 0.3 |
|  | 28 | 0.2 |
|  | n1 | 0 |
|  | n40 | 0.8 |

### 8.3.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.4	DC_1-3-18_n3-n77

### 8.4.1	Operating bands for DC

Table 8.4.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-18_n3-n77 | 1, 3, 18 | n3, n77 |

### 8.4.2	Channel bandwidths per operating band for DC

Table 8.4.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-18A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A1 DC_3A_n77A DC_18A_n3A DC_18A_n77A | 1 | See CA_1A-3A-18A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.4.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 8.4.4	∆TIB and ∆RIB values

For DC_1-3-18_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 8.4.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 18 | 0 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 8.4.5	MSD

There are no additional MSD requirements for this band combination.

## 8.5	DC_1-3-18_n3-n78

### 8.5.1	Operating bands for DC

Table 8.5.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-18_n3-n78 | 1, 3, 18 | n3, n78 |

### 8.5.2	Channel bandwidths per operating band for DC

Table 8.5.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-18A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A DC_3A_n78A DC_18A_n3A DC_18A_n78A | 1 | See CA_1A-3A-18A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.5.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 8.5.4	∆TIB and ∆RIB values

For DC_1-3-18_n3-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.5.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 8.5.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 18 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 8.5.5	MSD

There are no additional MSD requirements for this band combination.

## 8.6	DC_1-3-41_n3-n41

### 8.6.1	Operating bands for DC

Table 8.6.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-41_n3-n41 | 1, 3, 41 | n3, n41 |

### 8.6.2	Channel bandwidths per operating band for DC

Table 8.6.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A1 DC_3A_n41A DC_41A_n3A | 1 | See CA_1A-3A-41A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n41A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.6.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 8.6.4	∆TIB and ∆RIB values

For DC_1-3-41_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 8.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3-n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.6.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3-n41 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 8.6.5	MSD

There are no additional MSD requirements for this band combination.

## 8.7	DC_1-3-41_n3-n77

### 8.7.1	Operating bands for DC

Table 8.7.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-41_n3-n77 | 1, 3, 41 | n3, n77 |

### 8.7.2	Channel bandwidths per operating band for DC

Table 8.7.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-41A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A1 DC_3A_n77A DC_41A_n3A DC_41A_n77A | 1 | See CA_1A-3A-41A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_3A_n3A1 DC_3A_n77A DC_41A_n3A DC_41A_n77A DC_41C_n3A DC_41C_n77A | 1 | See CA_1A-3A-41C BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.7.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 8.7.4	∆TIB and ∆RIB values

For DC_1-3-41_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.7.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | n3 | 0.6 |
|  | n77 | 0.8 |

Table 8.7.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 8.7.5	MSD

There are no additional MSD requirements for this band combination.

## 8.8	DC_1-3-41_n3-n78

### 8.8.1	Operating bands for DC

Table 8.8.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-41_n3-n78 | 1, 3, 41 | n3, n78 |

### 8.8.2	Channel bandwidths per operating band for DC

Table 8.8.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-41A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A1 DC_3A_n78A DC_41A_n3A DC_41A_n78A | 1 | See CA_1A-3A-41A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-41C_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_3A_n3A1 DC_3A_n78A DC_41A_n3A DC_41A_n78A DC_41C_n3A DC_41C_n78A | 1 | See CA_1A-3A-41C BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | See CA_n3A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.8.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 8.8.4	∆TIB and ∆RIB values

For DC_1-3-41_n3-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.8.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 8.8.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 8.8.5	MSD

There are no additional MSD requirements for this band combination.

## 8.9	DC_1-3-41_n41-n77

### 8.9.1	Operating bands for DC

Table 8.9.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-41_n41-n77 | 1, 3, 41 | n41, n77 |

### 8.9.2	Channel bandwidths per operating band for DC

Table 8.9.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-41A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_41A_n77A | 1 | See CA_1A-3A-41A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.9.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 8.9.4	∆TIB and ∆RIB values

For DC_1-3-41_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.9.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n41-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
|  | n77 | 0.8 |

Table 8.9.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n41-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | n41 | 0 |
|  | n77 | 0.5 |

### 8.9.5	MSD

There are no additional MSD requirements for this band combination.

## 8.10	DC_1-3-41_n41-n78

### 8.10.1	Operating bands for DC

Table 8.10.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_1-3-41_n41-n78 | 1, 3, 41 | n41, n78 |

### 8.10.2	Channel bandwidths per operating band for DC

Table 8.10.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_1A-3A-41A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_41A_n78A | 1 | See CA_1A-3A-41A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | See CA_n41A-n78A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.10.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 8.10.4	∆TIB and ∆RIB values

For DC_1-3-41_n41-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.10.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n41-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | n41 | 0.5 |
|  | n78 | 0.8 |

Table 8.10.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n41-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | n41 | 0 |
|  | n78 | 0.5 |

### 8.10.5	MSD

There are no additional MSD requirements for this band combination.

## 8.11	DC_1-8-42_n28-n77

### 8.11.1	Operating bands for DC

Table 8.11.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-42_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.11.2	Channel bandwidths per operating band for DC

Table 8.11.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-42A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A-42A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A-42C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A-42C_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 290 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.11.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-42_n28, DC_1-8-42_n77, DC_1-8_n28-n77, DC_1-42_n28-n77 and DC_8-42_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.11.4	∆TIB and ∆RIB values

For DC_1-8-42_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.11.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n28-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 8.11.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-42_n28-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

### 8.11.5	MSD

As mentioned in 8.11.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.12	DC_1-3-18_n28-n77

### 8.12.1	Operating bands for DC

Table 8.12.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.12.2	Channel bandwidths per operating band for DC

Table 8.12.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-18A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 185 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
| DC_1A-3A-18A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 285 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.12.3	Co-existence studies

Co-existence studies of DC_1A-3A-18A_n28A-n77A and protected bands are already covered in the constituent fall-back modes.

### 8.12.4	∆TIB and ∆RIB values

For DC_1-3-18_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.12.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n28-n77 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.12.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n28-n77 | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 8.12.5	MSD

As mentioned in 8.12.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.13	DC_1-3-18_n28-n78

### 8.13.1	Operating bands for DC

Table 8.13.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.13.2	Channel bandwidths per operating band for DC

Table 8.13.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-18A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n77A DC_18A_n28A DC_18A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 185 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
| DC_1A-3A-18A_n28A-n78(2A) | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_18A_n28A DC_18A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 285 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.13.3	Co-existence studies

Co-existence studies of DC_1A-3A-18A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

### 8.13.4	∆TIB and ∆RIB values

For DC_1-3-18_n28-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.13.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n28-n78 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 8.13.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n28-n78 | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 8.13.5	MSD

As mentioned in 8.13.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.14	DC_1-3-41_n28-n41

### 8.14.1	Operating bands for DC

Table 8.14.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-41_n28-n41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 8.14.2	Channel bandwidths per operating band for DC

Table 8.14.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-41A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A DC_41A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |

### 8.14.3	Co-existence studies

Co-existence studies of DC_1A-3A-41A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

### 8.14.4	∆TIB and ∆RIB values

For DC_1-3-41_n28-n41, the TIB,c and RIB,c values are given in the tables below.

Table 8.14.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n28-n41 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.6 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

Table 8.14.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-41_n28-n41 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n28 | 0.2 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz |  |  |

### 8.14.5	MSD

As mentioned in 8.14.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.15	DC_3-19-42_n1-n77

### 8.15.1	Operating bands for DC

Table 8.15.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19-42_n1-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.15.2	Configuration for DC

Table 8.15.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-19A-42A_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A | CA_3A-19A-42A | CA_n1A-n77A |
| DC_3A-19A-42C_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_19A_n1A DC_19A_n77A | CA_3A-19A-42C | CA_n1A-n77A |

### 8.15.3	Co-existence studies

Co-existence studies of DC_3-19-42_n1-n77 are already covered by the fallback configurations, DC_3-19-42_n1, DC_3-19-42_n77, DC_3-19_n1-n77, DC_3-42_n1-n77 and DC_19-42_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.15.4	∆TIB and ∆RIB values

For DC_3-19-42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-3-19-42_n77, and are given in the tables below.

Table 8.15.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-42_n1-n77 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.8 |

Table 8.15.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19-42_n1-n77 | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

### 8.15.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.16	DC_3-19-42_n1-n78

### 8.16.1	Operating bands for DC

Table 8.16.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19-42_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.16.2	Configuration for DC

Table 8.16.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-19A-42A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A | CA_3A-19A-42A | CA_n1A-n78A |
| DC_3A-19A-42C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_19A_n1A DC_19A_n78A | CA_3A-19A-42C | CA_n1A-n78A |

### 8.16.3	Co-existence studies

Co-existence studies of DC_3-19-42_n1-n78 are already covered by the fallback configurations, DC_3-19-42_n1, DC_3-19-42_n78, DC_3-19_n1-n78, DC_3-42_n1-n78 and DC_19-42_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.16.4	∆TIB and ∆RIB values

For DC_3-19-42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-3-19-42_n78, and are given in the tables below.

Table 8.16.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-42_n1-n78 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

Table 8.16.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19-42_n1-n78 | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 8.16.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.17	DC_3-19-42_n1-n79

### 8.17.1	Operating bands for DC

Table 8.17.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-19-42_n1-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 8.17.2	Configuration for DC

Table 8.17.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-19A-42A_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A | CA_3A-19A-42A | CA_n1A-n79A |
| DC_3A-19A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_19A_n1A DC_19A_n79A | CA_3A-19A-42C | CA_n1A-n79A |

### 8.17.3	Co-existence studies

Co-existence studies of DC_3-19-42_n1-n79 are already covered by the fallback configurations, DC_3-19-42_n1, DC_3-19-42_n79, DC_3-19_n1-n79, DC_3-42_n1-n79 and DC_19-42_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.17.4	∆TIB and ∆RIB values

For DC_3-19-42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-3-19-42_n79, and are given in the tables below.

Table 8.17.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-42_n1-n79 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n79 | 0 |

Table 8.17.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-19-42_n1-n79 | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n79 | 0 |

### 8.17.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.18	DC_3-21-42_n1-n77

### 8.18.1	Operating bands for DC

Table 8.18.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21-42_n1-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.18.2	Configuration for DC

Table 8.18.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A-42A_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A | CA_3A-21A-42A | CA_n1A-n77A |
| DC_3A-21A-42C_n1A-n77A | DC_3A_n1A DC_3A_n77A DC_21A_n1A DC_21A_n77A | CA_3A-21A-42C | CA_n1A-n77A |

### 8.18.3	Co-existence studies

Co-existence studies of DC_3-21-42_n1-n77 are already covered by the fallback configurations, DC_3-21-42_n1, DC_3-21-42_n77, DC_3-21_n1-n77, DC_3-42_n1-n77 and DC_21-42_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.18.4	∆TIB and ∆RIB values

For DC_3-21-42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-3-21-42_n77, and are given in the tables below.

Table 8.18.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21-42_n1-n77 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n77 | 0.6 |

Table 8.18.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21-42_n1-n77 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.2 |

### 8.18.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.19	DC_3-21-42_n1-n78

### 8.19.1	Operating bands for DC

Table 8.19.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21-42_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.19.2	Configuration for DC

Table 8.19.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A-42A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A | CA_3A-21A-42A | CA_n1A-n78A |
| DC_3A-21A-42C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_21A_n1A DC_21A_n78A | CA_3A-21A-42C | CA_n1A-n78A |

### 8.19.3	Co-existence studies

Co-existence studies of DC_3-21-42_n1-n78 are already covered by the fallback configurations, DC_3-21-42_n1, DC_3-21-42_n78, DC_3-21_n1-n78, DC_3-42_n1-n78 and DC_21-42_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.19.4	∆TIB and ∆RIB values

For DC_3-21-42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-3-21-42_n78, and are given in the tables below.

Table 8.19.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21-42_n1-n78 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n78 | 0.6 |

Table 8.19.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21-42_n1-n78 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n78 | 0.2 |

### 8.19.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.20	DC_3-21-42_n1-n79

### 8.20.1	Operating bands for DC

Table 8.20.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-21-42_n1-n79 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 8.20.2	Configuration for DC

Table 8.20.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-21A-42A_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A | CA_3A-21A-42A | CA_n1A-n79A |
| DC_3A-21A-42C_n1A-n79A | DC_3A_n1A DC_3A_n79A DC_21A_n1A DC_21A_n79A | CA_3A-21A-42C | CA_n1A-n79A |

### 8.20.3	Co-existence studies

Co-existence studies of DC_3-21-42_n1-n79 are already covered by the fallback configurations, DC_3-21-42_n1, DC_3-21-42_n79, DC_3-21_n1-n79, DC_3-42_n1-n79 and DC_21-42_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.20.4	∆TIB and ∆RIB values

For DC_3-21-42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-3-21-42_n79, and are given in the tables below.

Table 8.20.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21-42_n1-n79 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |
|  | n79 | 0 |

Table 8.20.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-21-42_n1-n79 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n79 | 0 |

### 8.20.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.21	DC_19-21-42_n1-n77

### 8.21.1	Operating bands for DC

Table 8.21.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21-42_n1-n77 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.21.2	Configuration for DC

Table 8.21.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-21A-42A_n1A-n77A | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A | CA_19A-21A-42A | CA_n1A-n77A |
| DC_19A-21A-42C_n1A-n77A | DC_19A_n1A DC_19A_n77A DC_21A_n1A DC_21A_n77A | CA_19A-21A-42C | CA_n1A-n77A |

### 8.21.3	Co-existence studies

Co-existence studies of DC_19-21-42_n1-n77 are already covered by the fallback configurations, DC_19-21-42_n1, DC_19-21-42_n77, DC_19-21_n1-n77, DC_19-42_n1-n77 and DC_21-42_n1-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.21.4	∆TIB and ∆RIB values

For DC_19-21-42_n1-n77, the TIB,c and RIB,c values are reused from DC_1-19-21-42_n77, and are given in the tables below.

Table 8.21.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n1-n77 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n77 | 0.8 |

Table 8.21.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21-42_n1-n77 | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |
|  | n77 | 0.5 |

### 8.21.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.22	DC_19-21-42_n1-n78

### 8.22.1	Operating bands for DC

Table 8.22.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21-42_n1-n78 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.22.2	Configuration for DC

Table 8.22.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-21A-42A_n1A-n78A | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A | CA_19A-21A-42A | CA_n1A-n78A |
| DC_19A-21A-42C_n1A-n78A | DC_19A_n1A DC_19A_n78A DC_21A_n1A DC_21A_n78A | CA_19A-21A-42C | CA_n1A-n78A |

### 8.22.3	Co-existence studies

Co-existence studies of DC_19-21-42_n1-n78 are already covered by the fallback configurations, DC_19-21-42_n1, DC_19-21-42_n78, DC_19-21_n1-n78, DC_19-42_n1-n78 and DC_21-42_n1-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.22.4	∆TIB and ∆RIB values

For DC_19-21-42_n1-n78, the TIB,c and RIB,c values are reused from DC_1-19-21-42_n78, and are given in the tables below.

Table 8.22.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n1-n78 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n78 | 0.8 |

Table 8.22.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21-42_n1-n78 | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |
|  | n78 | 0.5 |

### 8.22.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.23	DC_19-21-42_n1-n79

### 8.23.1	Operating bands for DC

Table 8.23.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_19-21-42_n1-n79 | 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
|  | 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 8.23.2	Configuration for DC

Table 8.23.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_19A-21A-42A_n1A-n79A | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A | CA_19A-21A-42A | CA_n1A-n79A |
| DC_19A-21A-42C_n1A-n79A | DC_19A_n1A DC_19A_n79A DC_21A_n1A DC_21A_n79A | CA_19A-21A-42C | CA_n1A-n79A |

### 8.23.3	Co-existence studies

Co-existence studies of DC_19-21-42_n1-n79 are already covered by the fallback configurations, DC_19-21-42_n1, DC_19-21-42_n79, DC_19-21_n1-n79, DC_19-42_n1-n79 and DC_21-42_n1-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.23.4	∆TIB and ∆RIB values

For DC_19-21-42_n1-n79, the TIB,c and RIB,c values are reused from DC_1-19-21-42_n79, and are given in the tables below.

Table 8.23.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n1-n79 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |
|  | n79 | 0 |

Table 8.23.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_19-21-42_n1-n79 | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |
|  | n79 | 0 |

### 8.23.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.24	DC_3-7-20_n1-n78

### 8.24.1	Operating bands for DC

Table 8.24.1-1: DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-20A_n1A-n78A DC_3C-7A-20A_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.24.2	Channel bandwidths per operating band for DC

Table 8.24.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-7A-20A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_3C_n1A DC_3C_n78A DC_7A_n1A DC_7A_n78A DC_20A_n1A DC_20A_n78A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.24.3	Co-existence studies

Co-existence studies of DC_3-7-20_n1-n78 and protected bands are already covered in the constituent fall-back modes.

### 8.24.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_3-7-20_n1-n78 are given in the tables below reusing the values for DC_1-3-7-20_n78.

Table 8.24.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-20_n1-n78 | n1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | n78 | 0.8 |

Table 8.24.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-7-20_n1-n78 | n1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n78 | 0.5 |

### 8.24.5	MSD

All MSD requirements for DC_3-7-20_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 8.25	DC_3-7-28_n40-n78

### 8.25.1	Operating bands for DC

Table 8.25.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-28_n40-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.25.2	Channel bandwidths per operating band for DC

Table 8.25.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_3A-7A-28A_n40A-n78A | DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.25.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 8.25.4	∆TIB and ∆RIB values

For DC_3-7-28_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.25.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n40-n78 | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 8.25.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n40-n78 | 3 | 0.2 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

### 8.25.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.26	DC_1-3-7_n40-n78

### 8.26.1	Operating bands for DC

Table 8.26.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n40-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.26.2	Channel bandwidths per operating band for DC

Table 8.26.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_1A-3A-7_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.26.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 8.26.4	∆TIB and ∆RIB values

For DC_1-3-7_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.26.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n40-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.8 |
|  | n40 | 0.9 |
|  | n78 | 0.8 |

Table 8.26.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7_n40-n78 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0.3 |
|  | n40 | 0.8 |
|  | n78 | 0.5 |

### 8.26.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.27	DC_1-7-28_n40-n78

### 8.27.1	Operating bands for DC

Table 8.27.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7-28_n40-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.27.2	Channel bandwidths per operating band for DC

Table 8.27.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_1A-7A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.27.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 8.27.4	∆TIB and ∆RIB values

For DC_1-7-28_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.27.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-28_n40-n78 | 1 | 0.6 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n40 | 0.5 |
|  | n78 | 0.8 |

Table 8.27.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-28_n40-n78 | 1 | 0.2 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | n40 | 0.4 |
|  | n78 | 0.5 |

### 8.27.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.28	DC_3-7-8_n78-n258

### 8.28.1	Operating bands for DC

Table 8.28.1-1: DC band combination of LTE 3 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-8_n78-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

### 8.28.2	Channel bandwidths per operating band for DC

Table 8.28.2-1: Supported bandwidths per DC  LTE 3 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A-7A-8A_n78A-n258A DC_3A-7A-8A_n78A-n258D DC_3A-7A-8A_n78A-n258E DC_3A-7A-8A_n78A-n258F DC_3A-7A-8A_n78A-n258G DC_3A-7A-8A_n78A-n258H DC_3A-7A-8A_n78A-n258I DC_3A-7A-8A_n78A-n258J DC_3A-7A-8A_n78A-n258K DC_3A-7A-8A_n78A-n258L DC_3A-7A-8A_n78A-n258M | DC_3A_n78A DC_3A_n258A DC_7A_n78A DC_7A_n258A DC_8A_n78A DC_8A_n258A | 3 | See CA_3A-7A-8A in TS 36.101 | 350 450 550 650 750 850 950 |
|  |  | 7 |  |  |
|  |  | 8 |  |  |
|  |  | n78 | CA_n78A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

### 8.28.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

### 8.28.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

### 8.28.5	MSD

No additional MSD requirement is needed.

## 8.29	DC_3-7-8_n40A-n258

### 8.29.1	Operating bands for DC

Table 8.29.1-1: DC band combination of LTE 3 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-3-7-8_n40-n258 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz |  |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz |  |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz |  |

### 8.29.2	Channel bandwidths per operating band for DC

Table 8.29.2-1: Supported bandwidths per DC LTE 3 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_3A-7A-8A_n40A-n258A DC_3A-7A-8A_n40A-n258D DC_3A-7A-8A_n40A-n258E DC_3A-7A-8A_n40A-n258F DC_3A-7A-8A_n40A-n258G DC_3A-7A-8A_n40A-n258H DC_3A-7A-8A_n40A-n258I DC_3A-7A-8A_n40A-n258J DC_3A-7A-8A_n40A-n258K DC_3A-7A-8A_n40A-n258L DC_3A-7A-8A_n40A-n258M | DC_3A_n40A DC_3A_n258A DC_7A_n40A DC_7A_n258A DC_8A_n40A DC_8A_n258A | 3 | See CA_3A-7A-8A in TS 36.101 | 330 430 530 630 730 830 930 |
|  |  | 7 |  |  |
|  |  | 8 |  |  |
|  |  | n40 | CA_n40A-n258A/D/E/F/G/H/I/J/K/L/M in TS 38.101-3 |  |
|  |  | n258 |  |  |

### 8.29.3	Co-existence studies

The co-existence summary in section 5.3 of 37.717-11-21 already captures the needed studies. There is no additional harmonic and/or intermodulation impact for this combination.

### 8.29.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

### 8.29.5	MSD

No additional MSD requirement is needed.

## 8.30	DC_1-8-11_n3-n28

### 8.30.1	Operating bands for DC

Table 8.30.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-11_n3-n28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 8.30.2	Channel bandwidths per operating band for DC

Table 8.30.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-11A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_11A_n3A DC_11A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 90 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.30.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-11_n3, DC_1-8-11_n28, DC_1-8_n3-n28, DC_1-11_n3-n28 and DC_8-11_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.30.4	∆TIB and ∆RIB values

For DC_1-8-11_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 8.30.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n28 | 0.6 |

Table 8.30.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n28 | 0.2 |

### 8.30.5	MSD

As mentioned in 8.30.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.31	DC_1-8-11_n3-n77

### 8.31.1	Operating bands for DC

Table 8.31.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-11_n3-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.31.2	Channel bandwidths per operating band for DC

Table 8.31.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-11A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A-11A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_11A_n3A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.31.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-11_n3, DC_1-8-11_n77, DC_1-8_n3-n77, DC_1-11_n3-n77 and DC_8-11_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.31.4	∆TIB and ∆RIB values

For DC_1-8-11_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.31.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n77 | 0.8 |

Table 8.31.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n77 | 0.5 |

### 8.31.5	MSD

As mentioned in 8.31.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.32	DC_1-3-11_n28-n77

### 8.32.1	Operating bands for DC

Table 8.32.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-11_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.32.2	Channel bandwidths per operating band for DC

Table 8.32.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-11A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-3A-11A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 270 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.32.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-3-11_n28, DC_1-3-11_n77, DC_1-3_n28-n77, DC_1-11_n28-n77 and DC_3-11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.32.4	∆TIB and ∆RIB values

For DC_1-3-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.32.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-11_n28-n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.32.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-11_n28-n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 8.32.5	MSD

As mentioned in 8.32.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.33	DC_1-8-11_n28-n77

### 8.33.1	Operating bands for DC

Table 8.33.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-11_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.33.2	Channel bandwidths per operating band for DC

Table 8.33.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-11A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A-11A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.33.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-11_n28, DC_1-8-11_n77, DC_1-8_n28-n77, DC_1-11_n28-n77 and DC_8-11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.33.4	∆TIB and ∆RIB values

For DC_1-8-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.33.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n28-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.33.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-11_n28-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 11 | 0 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 8.33.5	MSD

As mentioned in 8.33.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.34	DC_1-8-42_n3-n28

### 8.34.1	Operating bands for DC

Table 8.34.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-42_n3-n28 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |

### 8.34.2	Channel bandwidths per operating band for DC

Table 8.34.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-42A_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 100 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A-42C_n3A-n28A | DC_1A_n3A DC_1A_n28A DC_8A_n3A DC_8A_n28A DC_42A_n3A DC_42C_n3A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.34.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-42_n3, DC_1-8-42_n28, DC_1-8_n3-n28, DC_1-42_n3-n28 and DC_8-42_n3-n28. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.34.4	∆TIB and ∆RIB values

For DC_1-8-42_n3-n28, the TIB,c and RIB,c values are given in the tables below.

Table 8.34.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n3-n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |
|  | n28 | 0.8 |

Table 8.34.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-42_n3-n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n28 | 0.5 |

### 8.34.5	MSD

As mentioned in 8.34.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.35	DC_1-8-42_n3-n77

### 8.35.1	Operating bands for DC

Table 8.35.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-42_n3-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.35.2	Channel bandwidths per operating band for DC

Table 8.35.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-42A_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A-42A_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A-42C_n3A-n77A | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-8A-42C_n3A-n77(2A) | DC_1A_n3A DC_1A_n77A DC_8A_n3A DC_8A_n77A DC_42A_n3A DC_42C_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 300 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.35.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-42_n3, DC_1-8-42_n77, DC_1-8_n3-n77, DC_1-42_n3-n77 and DC_8-42_n3-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.35.4	∆TIB and ∆RIB values

For DC_1-8-42_n3-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.35.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n3-n77 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.8 |
|  | n77 | 0.8 |

Table 8.35.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-42_n3-n77 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |
|  | n77 | 0.5 |

### 8.35.5	MSD

As mentioned in 8.35.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.36	DC_3-8-11_n28-n77

### 8.36.1	Operating bands for DC

Table 8.36.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8-11_n28-n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.36.2	Channel bandwidths per operating band for DC

Table 8.36.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_3A-8A-11A_n28A-n77A | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_3A-8A-11A_n28A-n77(2A) | DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.36.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_3-8-11_n28, DC_3-8-11_n77, DC_3-8_n28-n77, DC_3-11_n28-n77 and DC_8-11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.36.4	∆TIB and ∆RIB values

For DC_3-8-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.36.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-11_n28-n77 | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 8.36.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8-11_n28-n77 | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 8.36.5	MSD

As mentioned in 8.36.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.37	DC_1-3-42_n28-n77

### 8.37.1	Operating bands for DC

Table 8.37.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-42_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.37.2	Channel bandwidths per operating band for DC

Table 8.37.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-42A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-3A-42A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-3A-42C_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-3A-42C_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_42A_n28A DC_42C_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 300 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.37.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-3-42_n28, DC_1-3-42_n77, DC_1-3_n28-n77, DC_1-42_n28-n77 and DC_3-42_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.37.4	∆TIB and ∆RIB values

For DC_1-3-42_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.37.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-42_n28-n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |
|  | n77 | 0.8 |

Table 8.37.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-42_n28-n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |
|  | n77 | 0.5 |

### 8.37.5	MSD

As mentioned in 8.37.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.38		DC_1-7-8_n28-n78

### 8.38.1	 Operating bands for DC

Table 8.38.1-1: LTE 3 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A-8A_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.38.2 	Configuration for DC

Table 8.38.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A-8A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.38.3	 Co-existence studies

Co-existence studies of DC_1A-7A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

### 8.38.4	 ∆TIB and ∆RIB values

Table 8.38.4-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-8_n28-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 8.38.4-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-8_n28-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 8.38.5	MSD

All MSD requirements for DC_1-7-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 8.39		DC_3-7-8_n28-n78

### 8.39.1	 Operating bands for DC

Table 8.39.1-1: LTE 3 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-8A_n28A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.39.2 	Configuration for DC

Table 8.39.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A-8A_n28A-n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.39.3	 Co-existence studies

Co-existence studies of DC_3A-7A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

### 8.39.4	 ∆TIB and ∆RIB values

Table 8.39.4-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8_n28-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 8.39.4-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-8_n28-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 8.39.5	MSD

All MSD requirements for DC_3-7-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 8.40	DC_3-7-28_n1-n78

### 8.40.1	Operating bands for DC

Table 8.40.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-28_n1-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.40.2	Channel bandwidths per operating band for DC

Table 8.40.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_3A-7A-28A_n1A-n78A | DC_3A_n1A DC_7A_n1A DC_28A_n1A DC_3A_n78A DC_7A_n78A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.40.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 8.40.4	∆TIB and ∆RIB values

For DC_3-7-28_n1-n78, the TIB,c and RIB,c values are derived from DC_1-3-7-28_n78 and are given in the tables below.

Table 8.40.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n1-n78 | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n1 | 0.7 |
|  | n78 | 0.6 |

Table 8.40.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n1-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n1 | 0.2 |
|  | n78 | 0.5 |

### 8.40.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.41	DC_1-7-28_n3-n78

### 8.41.1	Operating bands for DC

Table 8.41.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7-28_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.41.2	Channel bandwidths per operating band for DC

Table 8.41.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_1A-7A-28A_n3A-n78A | DC_1A_n3A DC_7A_n3A DC_28A_n3A DC_1A_n78A DC_7A_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| DC_1A-7C-28A_n3A-n78A | DC_1A_n3A DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_1A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.41.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 8.41.4	∆TIB and ∆RIB values

For DC_1-7-28_n3-n78, the TIB,c and RIB,c values are derived from DC_1-3-7-28_n78 and are given in the tables below.

Table 8.41.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-28_n3-n78 | 1 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n3 | 0.7 |
|  | n78 | 0.8 |

Table 8.41.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-28_n3-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 8.41.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.42	DC_3-7-40_n1-n78

### 8.42.1	Operating bands for DC

Table 8.42.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-40A_n1A-n78A DC_3A-7A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.42.2	Channel bandwidths per operating band for DC

Table 8.42.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.42.3	Co-existence studies

Co-existence studies of DC_3-7-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

### 8.42.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_3-7-40_n1-n78 are given in the tables below reusing the values for DC_1-3-7-40_n78.

Table 8.42.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-40_n1-n78 | n1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 8.42.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-7-40_n1-n78 | n1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 8.42.5	MSD

All MSD requirements for DC_3-7-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 8.43	DC_3-8-40_n1-n78

### 8.43.1	Operating bands for DC

Table 8.43.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-8A-40A_n1A-n78A DC_3A-8A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.43.2	Channel bandwidths per operating band for DC

Table 8.43.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-8A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-8A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.43.3	Co-existence studies

Co-existence studies of DC_3-8-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

### 8.43.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_3-8-40_n1-n78 are given in the tables below reusing the values for DC_1-3-8-40_n78.

Table 8.43.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-40_n1-n78 | n1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | 40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 8.43.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-8-40_n1-n78 | n1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 8.43.5	MSD

All MSD requirements for DC_3-8-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 8.44	DC_7-8-40_n1-n78

### 8.44.1	Operating bands for DC

Table 8.44.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7A-8A-40A_n1A-n78A DC_7A-8A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.44.2	Channel bandwidths per operating band for DC

Table 8.44.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A-8A-40A_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_7A-8A-40C_n1A-n78A | DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.44.3	Co-existence studies

Co-existence studies of DC_7-8-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

### 8.44.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_7-8-40_n1-n78 are given in the tables below reusing the values for DC_1-7-8-40_n78.

Table 8.44.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8-40_n1-n78 | n1 | 0.6 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 8.44.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-8-40_n1-n78 | n1 | 0.2 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 8.44.5	MSD

All MSD requirements for DC_7-8-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 8.45	DC_1-3-18_n3-n41

### 8.45.1	Operating bands for DC

Table 8.45.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n3-n41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 8.45.2	Channel bandwidths per operating band for DC

Table 8.45.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-18A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A DC_3A_n41A DC_18A_n3A DC_18A_n41A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 175 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |

### 8.45.3	Co-existence studies

Co-existence studies of DC_1A-3A-18A_n3A-n41A and protected bands are already covered in the constituent fall-back modes.

### 8.45.4	∆TIB and ∆RIB values

For DC_1-3-18_n3-n41, the TIB,c and RIB,c values are given in the tables below.

Table 8.45.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3-n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 18 | 0.3 |
|  | n3 | 0.5 |
|  | n41 | 0.31 |
|  |  | 0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.45.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n3-n41 | 1 | 0.0 |
|  | 3 | 0.5 |
|  | 18 | 0 |
|  | n3 | 0.5 |
|  | n41 | 01 |
|  |  | 0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 8.45.5	MSD

As mentioned in 8.45.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.46	DC_1-3-18_n28-n41

### 8.46.1	Operating bands for DC

Table 8.46.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n28-n41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

### 8.46.2	Channel bandwidths per operating band for DC

Table 8.46.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-18A_n28A-n41A | DC_1A_n28A DC_1A_n41A DC_3A_n28A DC_3A_n41A DC_18A_n28A DC_18A_n41A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 175 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |

### 8.46.3	Co-existence studies

Co-existence studies of DC_1A-3A-18A_n28A-n41A and protected bands are already covered in the constituent fall-back modes.

### 8.46.4	∆TIB and ∆RIB values

For DC_1-3-18_n28-n41, the TIB,c and RIB,c values are given in the tables below.

Table 8.46.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n28-n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 18 | 0.3 |
|  | n28 | 0.6 |
|  | n41 | 0.31 |
|  |  | 0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.46.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n28-n41 | 1 | 0.0 |
|  | 3 | 0.5 |
|  | 18 | 0 |
|  | n28 | 0.2 |
|  | n41 | 01 |
|  |  | 0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 8.46.5	MSD

As mentioned in 8.46.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.47	DC_1-3-18_n41-n77

### 8.47.1	Operating bands for DC

Table 8.47.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n41-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.47.2	Channel bandwidths per operating band for DC

Table 8.47.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-18A_n41A-n77A | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
| DC_1A-3A-18A_n41A-n77(2A) | DC_1A_n41A DC_1A_n77A DC_3A_n41A DC_3A_n77A DC_18A_n41A DC_18A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 355 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.47.3	Co-existence studies

Co-existence studies of DC_1A-3A-18A_n41A-n77A and protected bands are already covered in the constituent fall-back modes.

### 8.47.4	∆TIB and ∆RIB values

For DC_1-3-18_n41-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.47.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n41-n77 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 18 | 0.3 |
|  | n41 | 0.31 |
|  |  | 0.82 |
|  | n77 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.47.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n41-n77 | 1 | 0.0 |
|  | 3 | 0.5 |
|  | 18 | 0 |
|  | n41 | 01 |
|  |  | 0.52 |
|  | n77 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 8.47.5	MSD

As mentioned in 8.47.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.48	DC_1-3-18_n41-n78

### 8.48.1	Operating bands for DC

Table 8.48.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-18_n41-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.48.2	Channel bandwidths per operating band for DC

Table 8.48.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-18A_n41A-n78A | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 255 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
| DC_1A-3A-18A_n41A-n78(2A) | DC_1A_n41A DC_1A_n78A DC_3A_n41A DC_3A_n78A DC_18A_n41A DC_18A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 355 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 | 15 | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.48.3	Co-existence studies

Co-existence studies of DC_1A-3A-18A_n41A-n78A and protected bands are already covered in the constituent fall-back modes.

### 8.48.4	∆TIB and ∆RIB values

For DC_1-3-18_n41-n78, the TIB,c and RIB,c values are given in the tables below.

Table 8.48.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n41-n77 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 18 | 0.3 |
|  | n41 | 0.31 |
|  |  | 0.82 |
|  | n78 | 0.8 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 8.48.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-18_n41-n77 | 1 | 0.0 |
|  | 3 | 0.5 |
|  | 18 | 0 |
|  | n41 | 01 |
|  |  | 0.52 |
|  | n78 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz NOTE 2 Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 8.48.5	MSD

As mentioned in 8.48.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.49	DC_1-3-8_n28-n78

### 8.49.1	 Operating bands for DC

Table 8.49.1-1: LTE 3 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-8A_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.49.2 	Configuration for DC

Table 8.49.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-8A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.49.3	 Co-existence studies

Co-existence studies of DC_1A-3A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

### 8.49.4	 ∆TIB and ∆RIB values

Table 8.49.4-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 8.49.4-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-8_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 8.49.5	MSD

All MSD requirements for DC_1-3-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 8.50	DC_1-3-20_n7-n78

### 8.50.1	Operating bands for DC

Table 8.50.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-20_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.50.2	Channel bandwidths per operating band for DC

Table 8.50.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |  | 30 MHz |  | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-20A_n7A-n78A | DC_1A_n7A DC_3A_n7A DC_20A_n7A DC_1A_n78A DC_3A_n78A DC_20A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.50.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-3-20_n7, DC_1-3-20_n78, DC_1-3_n7-n78, DC_3-20_n7-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.50.4	∆TIB and ∆RIB values

For DC_1-3-8_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.50.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-20_n7-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.3 |
|  | n7 | 0.6 |
|  | n78 | 0.8 |

Table 8.50.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-20_n7-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 20 | 0 |
|  | n7 | 0 |
|  | n78 | 0.5 |

### 8.50.5	MSD

As mentioned in 8.50.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.51	DC_2-7-13_n25-n66

### 8.51.1	Operating bands for DC

Table 8.51.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-7-13_n25-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 8.51.2	Channel bandwidths per operating band for DC

Table 8.51.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-7A-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 130 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_2A-7A-7A-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 7 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_2A-7C-13A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_13A_n25A DC_13A_n66A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply for the Band 42 and Band n77/n78 combination when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.51.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 8.51.4	∆TIB and ∆RIB values

For DC_2-7-13_n25-n66, the TIB,c and RIB,c values are derived from DC_2-7-13_n66 and are given in the tables below.

Table 8.51.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-13_n25-n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 13 | 0.3 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 8.51.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-13_n25-n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 13 | 0 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

### 8.51.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.52	DC_2-7-66_n25-n66

### 8.52.1	Operating bands for DC

Table 8.52.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-7-66_n25-n66 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

### 8.52.2	Channel bandwidths per operating band for DC

Table 8.52.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_2A-7A-66A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_2A-7A-7A-66A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | See CA_7A-7A in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| DC_2A-7C-66A_n25A-n66A5,6 | DC_2A_n66A DC_7A_n25A DC_7A_n66A DC_66A_n25A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n25 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n66 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
| NOTE 5: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. NOTE 6: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply for the Band 42 and Band n77/n78 combination when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.52.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 8.52.4	∆TIB and ∆RIB values

For DC_2-7-66_n25-n66, the TIB,c and RIB,c values are derived from DC_2-7_n66 and are given in the tables below.

Table 8.52.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-66_n25-n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |
|  | n66 | 0.5 |

Table 8.52.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-66_n25-n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.3 |
|  | n66 | 0.5 |

### 8.52.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.53	DC_1-3-7_n3-n78

### 8.53.1	Operating bands for DC

Table 8.53.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.53.2	Channel bandwidths per operating band for DC

Table 8.53.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_1A-3A-7A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3A-7C_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_7C_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.53.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 8.53.4	∆TIB and ∆RIB values

For DC_1-3-7_n3-n78, the TIB,c and RIB,c values are derived from DC_1-3-7_n78 and are given in the tables below.

Table 8.53.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n3-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | n3 | 0.7 |
|  | n78 | 0.8 |

Table 8.53.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7_n3-n78 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | n3 | 0.3 |
|  | n78 | 0.5 |

### 8.53.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.54	DC_1-3-28_n3-n78

### 8.54.1	Operating bands for DC

Table 8.54.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-28_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.54.2	Channel bandwidths per operating band for DC

Table 8.54.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_1A-3A-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.54.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 8.54.4	∆TIB and ∆RIB values

For DC_1-3-28_n3-n78, the TIB,c and RIB,c values are derived from DC_1-3-28_n78 and are given in the tables below.

Table 8.54.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-28_n3-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 8.54.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-28_n3-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 8.54.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.55	DC_3-7-28_n3-n78

### 8.55.1	Operating bands for DC

Table 8.55.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-28_n3-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.55.2	Channel bandwidths per operating band for DC

Table 8.55.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_3A-7A-28A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_28A_n3A DC_3A_n78A DC_7A_n78A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7C-28A_n3A-n78A | DC_3A_n3A4 DC_7A_n3A DC_7C_n3A DC_28A_n3A DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.55.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 8.55.4	∆TIB and ∆RIB values

For DC_3-7-28_n3-n78, the TIB,c and RIB,c values are derived from DC_3-7-28_n78 and are given in the tables below.

Table 8.55.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n3-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 8.55.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n3-n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 8.55.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.56	DC_1-3-7_n8-n78

### 8.56.1	 Operating bands for DC

Table 8.56.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-7A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.56.2 	Configuration for DC

Table 8.56.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-7A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.56.3	 Co-existence studies

Co-existence studies of DC_1A-3A-7A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 8.56.4	 ∆TIB and ∆RIB values

Table 8.56.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-7A_n8A-n78A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 8.56.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-7A_n8A-n78A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 8.56.5	MSD

All MSD requirements for DC_1A-3A-7A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 8.57	DC_1-3-20_n8-n78

### 8.57.1	 Operating bands for DC

Table 8.57.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-20A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.57.2 	Configuration for DC

Table 8.57.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-20A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.57.3	 Co-existence studies

Co-existence studies of DC_1A-3A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 8.57.4	 ∆TIB and ∆RIB values

Table 8.57.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-20A_n8A-n78A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 8.57.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-20A_n8A-n78A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 8.57.5	MSD

All MSD requirements for DC_1A-3A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 8.58	DC_1-7-20_n8-n78

### 8.58.1	 Operating bands for DC

Table 8.58.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-7A-20A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.58.2 	Configuration for DC

Table 8.58.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A-20A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.58.3	 Co-existence studies

Co-existence studies of DC_1A-7A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 8.58.4	 ∆TIB and ∆RIB values

Table 8.58.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-20A_n8A-n78A | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 8.58.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-7A-20A_n8A-n78A | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 8.58.5	MSD

All MSD requirements for DC_1A-7A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 8.59	DC_3-7-20_n8-n78

### 8.59.1	 Operating bands for DC

Table 8.59.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-20A_n8A-n78A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.59.2 	Configuration for DC

Table 8.59.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A-20A_n8A-n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 8.59.3	 Co-existence studies

Co-existence studies of DC_3A-7A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 8.59.4	 ∆TIB and ∆RIB values

Table 8.59.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A-20A_n8A-n78A | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 8.59.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3A-7A-20A_n8A-n78A | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 8.59.5	MSD

All MSD requirements for DC_3A-7A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 8.60	DC_3-7-8_n1-n40

### 8.60.1	Operating bands for DC

Table 8.60.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-8_n1-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 8.60.1-2: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A-8A_n1A-n40A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_3A_n40A DC_7A_n40A DC_8A_n40A | CA_3A-7A-8A | CA_n1A-n40A |

### 8.60.2	Channel bandwidths per operating band for DC

Table 8.60.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_3A-7A-8A_n1A-n40A | DC_3A_n1A DC_7A_n1A DC_8A_n1A DC_3A_n40A DC_7A_n40A DC_8A_n40A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |

### 8.60.3	Co-existence studies

Co-existence studies of this 5DL/2UL DC configuration are already covered in the constituent fall-back modes.

### 8.60.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 8.60.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8_n1-n40 | 3 | 0.5 |
|  | 7 | 0.8 |
|  | 8 | 0.6 |
|  | n1 | 0.6 |
|  | n40 | 0.9 |

Table 8.60.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-8_n1-n40 | 3 | 0 |
|  | 7 | 0.3 |
|  | 8 | 0.2 |
|  | n1 | 0.1 |
|  | n40 | 0.8 |

### 8.60.5	MSD

Based on co-existence studies no additional MSD is needed.

## 8.61	DC_2-13-66_n5-n77

### 8.61.1	Operating bands for DC

Table 8.61.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13-66_n5-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.61.2	Configuration for DC

Table 8.61.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-13A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A | CA_2A-13A-66A | CA_n5A-n77A |
| DC_2A-2A-13A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A | CA_2A-2A-13A-66A | CA_n5A-n77A |
| DC_2A-13A-66A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_13A_n77A DC_66A_n5A DC_66A_n77A | CA_2A-13A-66-66A | CA_n5A-n77A |

### 8.61.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 8.61.4	∆TIB and ∆RIB values

For DC_2-13-66_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.61.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13-66_n5-n77 DC_2-2-13-66_n5-n77 DC_2-13-66-66_n5-n77 | 2 | 0.6 |
|  | 13 | 0.5 |
|  | 66 | 0.6 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 8.61.4-2: ΔRIB

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13-66_n5-n77 DC_2-2-13-66_n5-n77 DC_2-13-66-66_n5-n77 | 2 | 0.3 |
|  | 13 | 0.2 |
|  | 66 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

### 8.61.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.62	DC_2-13-66_n66-n77

### 8.62.1	Operating bands for DC

Table 8.62.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13-66_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.62.2	Configuration for DC

Table 8.62.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-13A-66A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_13A_n66A DC_13A_n77A DC_66A_n77A | CA_2A-13A-66A | CA_n66A-n77A |
| DC_2A-2A-13A-66A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_13A_n66A DC_13A_n77A DC_66A_n77A | CA_2A-2A-13A-66A | CA_n66A-n77A |

### 8.62.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 8.62.4	∆TIB and ∆RIB values

For DC_2-13-66_n66-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.62.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13-66_n66-n77 DC_2-2-13-66_n66-n77 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 8.62.4-2: ΔRIB

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13-66_n66-n77 DC_2-2-13-66_n66-n77 | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 8.62.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.63	DC_2-13-66_n2-n77

### 8.63.1	Operating bands for DC

Table 8.63.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-13-66_n2-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.63.2	Configuration for DC

Table 8.63.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-13A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-13A-66A | CA_n2A-n77A |
| DC_2A-13A-66A-66A_n2A-n77A | DC_2A_n77A DC_13A_n2A DC_13A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-13A-66A-66A | CA_n2A-n77A |

### 8.63.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 8.63.4	∆TIB and ∆RIB values

For DC_2-13-66_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.63.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13-66_n2-n77, DC_2-13-66-66_n2-n77 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 8.63.4-2: ΔRIB

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-13-66_n2-n77 DC_2-13-66-66_n2-n77 | 2 | 0.2 |
|  | 13 | 0 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n77 | 0.5 |

### 8.63.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.64	DC_2-5-66_n5-n77

### 8.64.1	Operating bands for DC

Table 8.64.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-5-66_n5-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.64.2	Configuration for DC

Table 8.64.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A DC_66A_n5A DC_66A_n77A | CA_2A-5A-66A | CA_n5A-n77A |
| DC_2A-5A-66A-66A_n5A-n77A | DC_2A_n5A DC_2A_n77A DC_5A_n77A DC_66A_n5A DC_66A_n77A | CA_2A-5A-66A-66A | CA_n5A-n77A |

### 8.64.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 8.64.4	∆TIB and ∆RIB values

For DC_2-5-66_n5-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.64.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n5-n77 DC_2-5-66-66_n5-n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n5 | 0.6 |
|  | n77 | 0.8 |

Table 8.64.4-2: ΔRIB

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-66_n5-n77 DC_2-5-66-66_n5-n77 | 2 | 0.3 |
|  | 5 | 0.2 |
|  | 66 | 0.2 |
|  | n5 | 0.2 |
|  | n77 | 0.5 |

### 8.64.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.65	DC_2-5-66_n2-n77

### 8.65.1	Operating bands for DC

Table 8.65.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-5-66A_n2-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.65.2	Configuration for DC

Table 8.65.2-1: Inter-band EN-DC configurations (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A-66A_n2A-n77A | DC_2A_n77A DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-5A-66A | CA_n2A-n77A |
| DC_2A-5A-66A-66A_n2A-n77A | DC_2A_n77A DC_5A_n2A DC_5A_n77A DC_66A_n2A DC_66A_n77A | CA_2A-5A-66A-66A | CA_n2A-n77A |

### 8.65.3	Co-existence studies

Co-existence studies are captured in lower order combinations.

### 8.65.4	∆TIB and ∆RIB values

For DC_2-5-66_n2-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.65.4-1: ΔTIB,c

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n2-n77 DC_2-5-66-66_n2-n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |
|  | n77 | 0.8 |

Table 8.65.4-2: ΔRIB

| Inter-band EN-DC Configuration | E-UTRA or NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-66_n2-n77 DC_2-5-66-66_n2-n77 | 2 | 0.2 |
|  | 5 | 0.2 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |
|  | n77 | 0.5 |

### 8.65.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.66	DC_2-7-66_n66-n77

### 8.66.1	Operating bands for DC

Table 8.66.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-7-66_n66-n77 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 8.66.2	Channel bandwidths per operating band for DC

Table 8.66.2-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-7A-66A_n66A-n77A DC_2A-7A-7A-66A_n66A-n77A DC_2A-7C-66A_n66A-n77A | DC_2A_n66A DC_7A_n66A DC_2A_n77A DC_7A_n77A DC_66A_n77A |

### 8.66.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 8.66.4	∆TIB and ∆RIB values

For DC_2-7-66_n66-n77, the TIB,c and RIB,c values are given in the tables below.

Table 8.66.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-66_n66-n77 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |
|  | n77 | 0.8 |

Table 8.66.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-66_n66-n77 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
|  | n77 | 0.5 |

### 8.66.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.67	DC_3-20-32_n1-n28

### 8.67.1	 Operating bands for DC

Table 8.67.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-20-32_n1-n28 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 32 | N/A | – | N/A MHz | 1452 MHz | – | 1496 MHz | SDL |

### 8.67.2 	Configuration for DC

Table 8.67.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A-32A_n1A-n28A | DC_3A_n1A DC_20A_n1A DC_3A_n28A DC_20A_n28A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 32 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_3C-20A-32A_n1A-n28A | DC_3A_n1A DC_3C_n1A DC_20A_n1A DC_3A_n28A DC_3C_n28A DC_20A_n28A | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 32 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.67.3	 Co-existence studies

Co-existence studies of DC_3-20-32_n1-n28 are already covered by the fallback configurations.

### 8.67.4	 ∆TIB and ∆RIB values

Table 8.67.4-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20-32_n1-n28 | n1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |

Table 8.67.4-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20-32_n1-n28 | n1 | 0.2 |
|  | 3 | 0.5 |
|  | 20 | 0.2 |
|  | n28 | 0.5 |
|  | 32 | 0 |

## 8.68	DC_1-3-20_n28-n75

### 8.68.1	 Operating bands for DC

Table 8.68.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-20_n28-n75 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n75 | N/A | – | N/A MHz | 1432 MHz | – | 1517 MHz | SDL |

### 8.68.2 	Configuration for DC

Table 8.68.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL configurations | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-20A_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_20A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 130 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| DC_1A-3C-20A_n28A-n75A | DC_1A_n28A DC_3A_n28A DC_3C_n28A DC_20A_n28A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

### 8.68.3	 Co-existence studies

Co-existence studies of DC_1-3-20_n28-n75 are already covered by the fallback configurations.

### 8.68.4	 ∆TIB and ∆RIB values

Table 8.68.4-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-20_n28-n75 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |

Table 8.68.4-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-20_n28-n75 | 1 | 0.2 |
|  | 3 | 0.5 |
|  | 20 | 0.2 |
|  | n28 | 0.5 |
|  | n75 | 0 |

## 8.69	DC_2-5-66_n66-n77

### 8.69.1	Operating bands for DC

Table 8.69.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2-5-66_n66-n77 | 2, 5, 66 | n66, n77 |

### 8.69.2	Channel bandwidths per operating band for DC

Table 8.69.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_2A-5A-66A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_5A_n66A DC_5A_n77A DC_66A_n77 | 2 | See CA_2A-5A-66A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.69.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 8.69.4	∆TIB and ∆RIB values

Relaxation values are copied from DC_2-5-66_n77

Table 8.69.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n66-n77 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
|  | n77 | 0.8 |

Table 8.69.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n66-n77 | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 8.69.5	MSD

There are no additional MSD requirements for this band combination.

## 8.70	DC_2-2-13-66_n66-n77

### 8.70.1	Operating bands for DC

Table 8.70.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC Band | LTE Band (Table 5.5-1 in TS36.101[4]) | NR Band (Table 5.2-1 in TS38.101-1[2] and TS38.101-2[3]) |
| --- | --- | --- |
| DC_2-2-13-66_n66-n77 | 2, 13, 66 | n66, n77 |

### 8.70.2	Channel bandwidths per operating band for DC

Table 8.70.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| DC_2A-2A-13A-66A_n66A-n77A | DC_2A_n66A DC_2A_n77A DC_13A_n66A DC_13A_n77A DC_66A_n77A | 2 | See CA_2A-2A-13A-66A BCS0 in Table 5.6A.1-2a of TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n66 | See CA_n66A-n77A BCS0 in Table 5.5A.3.1-1 of TS 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 8.70.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered in the constituent fall-back mode.

### 8.70.4	∆TIB and ∆RIB values

Relaxation values are copied from DC_2-13_n66

Table 8.70.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-2-13-66_n66-n77 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |
|  | n77 | 0.8 |

Table 8.70.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-2-13-66_n66-n77 | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |
|  | n77 | 0.5 |

### 8.70.5	MSD

There are no additional MSD requirements for this band combination.

## 8.71	DC_1-7-20_n3-n38

### 8.71.1	Operating bands for DC

Table 8.71.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7-20_n3-n38 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |

### 8.71.2	Channel bandwidths per operating band for DC

Table 8.71.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-7A-20A_n3A-n38A | DC_1A-n3A DC_20A_n3A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 130 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes |  |  |  |  |  |  |  |

### 8.71.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.71.4	∆TIB and ∆RIB values

For DC_1-7-20_n3-n38, the TIB,c and RIB,c values are given in the tables below.

Table 8.71.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-20_n3-n38 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.5 |
|  | n3 | 0.6 |
|  | n38 | 0.5 |

Table 8.71.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-20_n3-n38 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0.2 |
|  | n3 | 0 |
|  | n38 | 0.2 |

### 8.71.5	MSD

All MSD requirements for DC_1-7-20_n3-n38 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 8.72	DC_1-3-8_n77-n79

### 8.72.1	Operating bands for DC

Table 8.72.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-8_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 8.72.2	Channel bandwidths per operating band for DC

Table 8.72.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-8A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_3A_n77A DC_3A_n79A DC_8A_n77A DC_8A_n79A | 1 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 250 |
|  |  | 3 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 |  | 10 | 15 | 20 |  |  | 40 | 50 | 60 |  | 80 | 90 | 100 |  |
|  |  | n79 |  |  |  |  |  |  | 40 | 50 | 60 |  | 80 |  | 100 |  |

### 8.72.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-3-8_n77, DC_1-3-8_n79, DC_1-3_n77-n79, DC_1-8_n77-n79 and DC_3-8_n77-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.72.4	∆TIB and ∆RIB values

For DC_1-3-8_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.72.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8_n77-n79 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 8.72.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-8_n77-n79 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 8 | 0.3 |
|  | n77 | 0.5 |
|  | n79 | 0 |

### 8.72.5	MSD

As mentioned in 8.72.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.73	DC_7-20-38_n3-n78

### 8.73.1	Operating bands for DC

Table 8.73.1-1: LTE 3 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-20-38_n3-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.73.2	Configuration for DC

Table 8.73.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-20A-38A_n3A-n78A | DC_20A_n3A DC_20A_n78A | CA_7A-20A-38A | CA_n3A-n78A |

### 8.73.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 8.73.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 8.73.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-38_n3-n78 | 7 | 0.5 |
|  | 20 | 0.6 |
|  | 38 | 0.5 |
|  | n3 | 0.5 |
|  | n78 | 0.8 |

Table 8.73.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-20-38_n3-n78 | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0.4 |
|  | n3 | 0 |
|  | n78 | 0.6 |

### 8.73.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.74	DC_1-7-38_n3-n78

### 8.74.1	Operating bands for DC

Table 8.74.1-1: LTE 3 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7-38_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.74.2	Configuration for DC

Table 8.74.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-7A-38A_n3A-n78A | DC_1A_n3A DC_1A_n78A | CA_1A-7A-38A | CA_n3A-n78A |

### 8.74.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 8.74.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 8.74.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-38_n3-n78 | 1 | 0.6 |
|  | 7 | 0 |
|  | 38 | 0 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 8.74.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-38_n3-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 38 | 0 |
|  | n3 | 0 |
|  | n78 | 0.8 |

### 8.74.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.75	DC_1-7-20_n3-n78

### 8.75.1	Operating bands for DC

Table 8.75.1-1: LTE 3 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7-20_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 8.75.2	Configuration for DC

Table 8.75.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-7A-20A_n3A-n78A | DC_1A_n3A DC_1A_n78A DC_20A_n3A DC_20A_n78A | CA_1A-7A-20A | CA_n3A-n78A |

### 8.75.3	Co-existence studies

Co-existence studies are captured in TR 37.716-11-11 as well as for lower order combinations in TR 37.716-21-21.

### 8.75.4	∆TIB and ∆RIB values

TIB,c and RIB,c values are given in the tables below.

Table 8.75.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-20_n3-n78 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.4 |
|  | n3 | 0.5 |
|  | n78 | 0.8 |

Table 8.75.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-20_n3-n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n3 | 0 |
|  | n78 | 0.5 |

### 8.75.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 8.76	DC_1-8-11_n3-n79

### 8.76.1	Operating bands for DC

Table 8.76.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-11_n3-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 8.76.2	Channel bandwidths per operating band for DC

Table 8.76.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Channel bandwidth [MHz] | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-11A_n3A-n79A | DC_1A_n3A DC_1A_n79A DC_8A_n3A DC_8A_n79A DC_11A_n3A DC_11A_n79A | 1 | 5, 10, 15, 20 | 170 |
|  |  | 8 | 5, 10 |  |
|  |  | 11 | 5, 10 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

### 8.76.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-11_n3, DC_1-8-11_n79, DC_1-8_n3-n79, DC_1-11_n3-n79 and DC_8-11_n3-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.76.4	∆TIB and ∆RIB values

For DC_1-8-11_n3-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.76.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n79 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |
|  | n79 | 0.8 |

Table 8.76.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-11_n3-n79 | 1 | 0 |
|  | 8 | 0 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |
|  | n79 | 0.5 |

### 8.76.5	MSD

As mentioned in 8.76.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 8.77	DC_1-8-11_n77-n79

### 8.77.1	Operating bands for DC

Table 8.77.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8-11_n77-n79 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

### 8.77.2	Channel bandwidths per operating band for DC

Table 8.77.2-1: Supported bandwidths per DC LTE 3DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Channel bandwidth [MHz] | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-8A-11A_n77A-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A | 1 | 5, 10, 15, 20 | 240 |
|  |  | 8 | 5, 10 |  |
|  |  | 11 | 5, 10 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
| DC_1A-8A-11A_n77(2A)-n79A | DC_1A_n77A DC_1A_n79A DC_8A_n77A DC_8A_n79A DC_11A_n77A DC_11A_n79A | 1 | 5, 10, 15, 20 | 340 |
|  |  | 8 | 5, 10 |  |
|  |  | 11 | 5, 10 |  |
|  |  | n77 | CA_n77(2A) |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |

### 8.77.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 3DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-8-11_n77, DC_1-8-11_n79, DC_1-8_n77-n79, DC_1-11_n77-n79 and DC_8-11_n77-n79. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 8.77.4	∆TIB and ∆RIB values

For DC_1-8-11_n77-n79, the TIB,c and RIB,c values are given in the tables below.

Table 8.77.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n77-n79 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n77 | 0.8 |
|  | n79 | 0.5 |

Table 8.77.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-8-11_n77-n79 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 11 | 0 |
|  | n77 | 0.5 |
|  | n79 | 0 |

### 8.77.5	MSD

As mentioned in 8.77.3, there is no need to specify additional MSD requirement for this UL DC configuration.

# 9	Dual Connectivity band combinations of LTE 4 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

## 9.1	DC_1-3-7-28_n40-n78

### 9.1.1	Operating bands for DC

Table 9.1.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7-28_n40-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 9.1.2	Channel bandwidths per operating band for DC

Table 9.1.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |

| DC_1A-3A-7A-28A_n40A-n78A | DC_1A_n40A DC_1A_n78A DC_3A_n40A DC_3A_n78A DC_7A_n40A DC_7A_n78A DC_28A_n40A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 260 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes9 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes | Yes |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 9.1.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 9.1.4	∆TIB and ∆RIB values

For DC_1-3-7-28_n40-n78, the TIB,c and RIB,c values are given in the tables below.

Table 9.1.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n40-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 28 | 0.3 |
|  | n40 | 0.9 |
|  | n78 | 0.8 |

Table 9.1.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n40-n78 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0.3 |
|  | 28 | 0.2 |
|  | n40 | 0.8 |
|  | n78 | 0.5 |

### 9.1.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 9.2	DC_1-3-8-11_n28-n77

### 9.2.1	Operating bands for DC

Table 9.2.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-8-11_n28-n77 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

### 9.2.2	Channel bandwidths per operating band for DC

Table 9.2.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | Subcarrier Spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth For DL [MHz] |
| DC_1A-3A-8A-11A_n28A-n77A | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_1A-3A-8A-11A_n28A-n77(2A) | DC_1A_n28A DC_1A_n77A DC_3A_n28A DC_3A_n77A DC_8A_n28A DC_8A_n77A DC_11A_n28A DC_11A_n77A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 280 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 9.2.3	Co-existence studies

Co-existence studies of this DC LTE inter-band DL/1UL + inter-band NR 2DL/1UL are already covered by the fallback configurations, DC_1-3-8-11_n28, DC_1-3-8-11_n77, DC_1-3-8_n28-n77, DC_1-3-11_n28-n77, DC_1-8-11_n28-n77 and DC_3-8-11_n28-n77. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

### 9.2.4	∆TIB and ∆RIB values

For DC_1-3-8-11_n28-n77, the TIB,c and RIB,c values are given in the tables below.

Table 9.2.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-8-11_n28-n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |
|  | n77 | 0.8 |

Table 9.2.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-8-11_n28-n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |
|  | n77 | 0.5 |

### 9.2.5	MSD

As mentioned in 9.2.3, there is no need to specify additional MSD requirement for this UL DC configuration.

## 9.3	DC_1-3-7-8_n28-n78

### 9.3.1	 Operating bands for DC

Table 9.3.1-1: LTE 4 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-7A-8A_n28A-n78A | 1 | 1920 MHz | – | 1980 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 9.3.2 	Configuration for DC

Table 9.3.2-1: Supported bandwidths per DC band combination of LTE 4DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-7A-8A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 200 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 9.3.3	 Co-existence studies

Co-existence studies of DC_1A-3A-7A-8A_n28A-n78A and protected bands are already covered in the constituent fall-back modes.

### 9.3.4	 ∆TIB and ∆RIB values

Table 9.3.4-1: ΔTIB,c due to EN-DC(six bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-8_n28-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |
|  | n78 | 0.8 |

Table 9.3.4-2: ΔRIB,c due to EN-DC (six bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-8_n28-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |
|  | n78 | 0.5 |

### 9.3.5	MSD

All MSD requirements for DC_1-3-7-8_n28-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 9.4	DC_3-7-8-40_n1-n78

### 9.4.1	Operating bands for DC

Table 9.4.1-1: DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3A-7A-8A-40A_n1A-n78A DC_3A-7A-8A-40C_n1A-n78A | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 9.4.2	Channel bandwidths per operating band for DC

Table 9.4.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A-8A-40A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3A-7A-8A-40C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_7A_n1A DC_7A_n78A DC_8A_n1A DC_8A_n78A DC_40A_n1A DC_40A_n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 9.4.3	Co-existence studies

Co-existence studies of DC_3-7-8-40_n1-n78 and protected bands are already covered in the constituent fall-back modes.

### 9.4.4	∆TIB and ∆RIB values

The TIB,c and RIB,c values of DC_3-7-8-40_n1-n78 are given in the tables below reusing the values for DC_1-3-7-8-40_n78.

Table 9.4.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8-40_n1-n78 | n1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 40 | 0.31 |
|  | n78 | 0.81 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 9.4.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8-40_n1-n78 | n1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 40 | 0.41 |
|  | n78 | 0.51 |
| NOTE 1: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 9.4.5	MSD

All MSD requirements for DC_3-7-8-40_n1-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 9.5	DC_1-3-7-28_n78-n257

### 9.5.1	Operating bands for DC

Table 9.5.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7-28_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

### 9.5.2	Channel bandwidths per operating band for DC

Table 9.5.2-1: Supported bandwidths per DC LTE 4 bands + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band | Channel bandwidth set | Maximum aggregated bandwidth [MHz] |
| --- | --- | --- | --- | --- |
| DC_1A-3A-7A-28A_n78A-n257A DC_1A-3A-7A-28A_n78A-n257G DC_1A-3A-7A-28A_n78A-n257H DC_1A-3A-7A-28A_n78A-n257I | DC_1A_n78A DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_3A_n78A DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_7A_n78A DC_7A_n257A DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_28A_n78A DC_28A_n257A DC_28A_n257G DC_28A_n257H DC_28A_n257I | 1 | See CA_1A-3A-7A-28A in TS 36.101 | 580 380 480 580 |
|  |  | 3 |  |  |
|  |  | 7 |  |  |
|  |  | 28 |  |  |
|  |  | n78 | CA_n78A-n257A/G/H/I in TS 38.101-3 |  |
|  |  | n257 |  |  |

### 9.5.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 9.5.4	∆TIB and ∆RIB values

No further TIB,c and RIB,c values are needed defined for this combination including FR2 as the FR1 combinations is already completed.

### 9.5.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 9.6	DC_1-3-7-28_n3-n78

### 9.6.1	Operating bands for DC

Table 9.6.1-1: DC band combination of LTE 4DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7-28_n3-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 9.6.2	Channel bandwidths per operating band for DC

Table 9.6.2-1: Supported bandwidths per DC LTE 4DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Max aggreg. BW DL [MHz] |
| DC_1A-3A-7A-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3A DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 220 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3A-7C-28A_n3A-n78A | DC_1A_n3A DC_3A_n3A4 DC_7A_n3ADC_7C_n3A DC_28A_n3A DC_1A_n78A DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 240 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| NOTE 4: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 9.6.3	Co-existence studies

Co-existence studies are captured for lower order combinations.

### 9.6.4	∆TIB and ∆RIB values

For DC_1-3-7-28_n3-n78, the TIB,c and RIB,c values are derived from DC_1-3-7-28_n78 and are given in the tables below.

Table 9.6.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n3-n78 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n3 | 0.7 |
|  | n78 | 0.8 |

Table 9.6.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-28_n3-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0.2 |
|  | n78 | 0.5 |

### 9.6.5	MSD

MSD was studied in lower order band combinations. No need for additional MSD analysis.

## 9.7		DC_1-3-7-20_n8-n78

### 9.7.1	 Operating bands for DC

Table 9.7.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1A-3A-7A-20A_n8A-n78A | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 9.7.2 	Configuration for DC

Table 9.7.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-7A-20A_n8A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 180 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

### 9.7.3	 Co-existence studies

Co-existence studies of DC_1A-3A-7A-20A_n8A-n78A and protected bands are already covered in the constituent fall-back modes.

### 9.7.4	 ∆TIB and ∆RIB values

Table 9.7.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-3A-7A-20A_n8A-n78A | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |
|  | n78 | 0.8 |

Table 9.7.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1A-3A-7A-20A_n8A-n78A | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n8 | 0.2 |
|  | n78 | 0.5 |

### 9.7.5	MSD

All MSD requirements for DC_1A-3A-7A-20A_n8A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 9.8		DC_1-3-7-20_n38-n78

### 9.8.1 	Operating bands for DC

Table 9.8.1-1: DC band combination of LTE 3DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7-20_n38-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2500 MHz | – | 2570 MHz | FDD |
|  | 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

### 9.8.2 	Configuration for DC

Table 9.8.2-1: Inter-band EN-DC configurations (six bands)

| EN-DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-7A-20A_n38A-n78A | DC_1A_n78A DC_3A_n78A DC_20A_n78A | CA_1A-3A-20A | n78A |

### 9.8.3	 Co-existence studies

Co-existence studies of  DC_1A-3A-7A-20A_n38A-n78A with the UL ENDC configurations of DC_1A_n78A

, DC_3A_n78A and DC_20A_n78A are already covered in the constituent fall-back modes.

### 9.8.4	 ∆TIB and ∆RIB values

Table 9.8.4-1: ΔTIB,c due to EN-DC(six bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7-20_n38-n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.6 |
|  | n78 | 0.8 |

Table 9.8.4-2: ΔRIB,c due to EN-DC (six bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7-20_n38-n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | n38 | 0.2 |
|  | n78 | 0.5 |

### 9.8.5	MSD

All MSD requirements for DC_1A-3A-7A-20A_n38A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 9.9		DC_1-7-20-38_n3-n78

### 9.9.1 	Configuration for DC

Table 9.9.1-1: Inter-band EN-DC configurations (six bands)

| EN-DC configuration | Uplink EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_1A-7A-20A-38A_n3A-n78A | DC_1A_n3A DC_20A_n3A DC_1A_n78A DC_20A_n78A | CA_1A-7A-20A-38A | CA_n3A-n78A |

### 9.9.2	 Co-existence studies

Co-existence studies of  DC_1A-7A-20A-38A_n3A-n78A with the UL ENDC configurations of  DC_1A_n3A,

DC_1A_n78A, DC_20A_n3A and DC_20A_n78A are already covered in the constituent fall-back modes.

### 9.9.3	 ∆TIB and ∆RIB values

Table 9.9.3-1: ΔTIB,c due to EN-DC(six bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-20-38_n3-n78 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.6 |
|  | 38 | 0.5 |
|  | n3 | 0.6 |
|  | n78 | 0.8 |

Table 9.9.3-2: ΔRIB,c due to EN-DC (six bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-20-38_n3-n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.2 |
|  | 38 | 0.4 |
|  | n3 | 0 |
|  | n78 | 0.8 |

### 9.9.4	MSD

All MSD requirements for DC_1A-7A-20A-38A_n3A-n78A are already covered by constituent fallback modes. No additional MSD requirement is needed.

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Subject/Comment | Old | New |
| 2020-08 | RAN4 #96-e | R4-2010130 |  |  | TR37.717-11-21 Skeleton for DC band combinations of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL | N/A | 0.0.1 |
| 2020-09 | RAN4 #96-e | R4-2011885 |  |  | TR 37.717-11-21 v0.1.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 | 0.0.1 | 0.1.0 |
| 2020-11 | RAN4 #97-e | R4-2014304 |  |  | TR 37.717-11-21 v0.2.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 10.7 in RAN4 97e meeting report | 0.1.0 | 0.2.0 |
| 2021-02 | RAN4 #98-e | R4-2100272 |  |  | TR 37.717-11-21 v0.3.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 9.7 in RAN4 98e meeting report | 0.2.0 | 0.3.0 |
| 2021-04 | RAN4 #98-e-bis | R4-2104965 |  |  | TR 37.717-11-21 v0.4.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 7.7 in RAN4 98bis-e meeting report | 0.3.0 | 0.4.0 |
| 2021-05 | RAN4 #99-e | R4-2109841 |  |  | TR 37.717-11-21 v0.5.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 8.21 in RAN4 99-e meeting report | 0.4.0 | 0.5.0 |
| 2021-08 | RAN4 #100-e | R4-2112735 |  |  | TR 37.717-11-21 v0.6.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 8.19 in RAN4 100-e meeting report | 0.5.0 | 0.6.0 |
| 2021-11 | RAN4 #101-e | R4-2118065 |  |  | TR 37.717-11-21 v0.7.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 7.18 in RAN4 101-e meeting report | 0.6.0 | 0.7.0 |
| 2022-03 | RAN4 #102-e | R4-2206645 |  |  | TR 37.717-11-21 v0.8.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 5.19 in RAN4 101-e-bis meeting report and TPs in section 9.19 in RAN4 102-e meeting report | 0.7.0 | 0.8.0 |
| 2022-05 | RAN4 #103-e | R4-2207632 |  |  | TR 37.717-11-21 v0.9.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Included all approved TPs in section 8.15 in RAN4 103-e meeting report | 0.8.0 | 0.9.0 |
| 2022-06 | RAN #96 | RP-221184 |  |  | TR 37.717-11-21 v1.1.0 for LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-17 - Presented for approval at RAN Plenary | 0.9.0 | 1.1.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-06 | RAN#96 |  |  |  |  | Approved by plenary – Rel-17 spec under change control | 17.0.0 |