| 3GPP TS 38.133 V17.14.0 (2024-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	120

1	Scope	122

2	References	122

3	Definitions, symbols and abbreviations	123

3.1	Definitions	123

3.2	Symbols	125

3.3	Abbreviations	126

3.4	Test tolerances	128

3.5	Frequency bands grouping	128

3.5.1	Introduction	128

3.5.2	NR operating bands in FR1	128

3.5.2A	NR operating bands for satellite access in FR1	129

3.5.3	NR operating bands in FR2	129

3.6	Applicability of requirements in this specification version	130

3.6.1	RRC connected state requirements in DRX	131

3.6.2	Number of serving carriers	131

3.6.2.1	Number of serving carriers for SA	131

3.6.2.2	Number of serving carriers for EN-DC	131

3.6.2.3	Number of serving carriers for NE-DC	131

3.6.2.4	Number of serving carriers for NR-DC	132

3.6.3	Applicability for intra-band FR2	132

3.6.4	Applicability for FR2 UE power classes	132

3.6.5	Applicability for SDL bands	132

3.6.6	Applicability of requirements for NGEN-DC operation	132

3.6.7	Applicability of QCL	132

3.6.9	Applicability of requirements for scheduling availability	133

3.6.10	Applicability of requirements for measurement restrictions	133

3.6.11	Applicability of requirements for Redcap UEs	133

3.6.11.1	RRC connected state requirements in DRX	133

3.6.11.2	Applicability for FR2 Redcap UE power classes	133

3.6.11.3	Applicability of QCL	133

3.6.12	Applicability of requirements for Satellite Access	134

3.6.13	Applicability of requirements for FR2	134

3.6.14	Applicability of requirements for FR2 Power Class 6	134

3.6.15	Applicability of requirements for per-FR gap	134

4	SA: RRC_IDLE state mobility	134

4.1	Cell Selection	134

4.2	Cell Re-selection	135

4.2.1	Introduction	135

4.2.2	Requirements	135

4.2.2.1	UE measurement capability	135

4.2.2.2	Measurement and evaluation of serving cell	135

4.2.2.3	Measurements of intra-frequency NR cells	138

4.2.2.4	Measurements of inter-frequency NR cells	141

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	146

4.2.2.6	Maximum interruption in paging reception	149

4.2.2.7	General requirements	150

4.2.2.8	Minimum requirement at transitions	150

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	150

4.2.2.9.1	Introduction	150

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	151

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	153

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	155

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	156

4.2.2.10.1	Introduction	156

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	156

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	159

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	162

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	162

4.2.2.11.1	Introduction	162

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	163

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	164

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	166

4.2A	Cell Re-selection when subject to CCA	167

4.2A.1	Introduction	167

4.2A.2	Requirements	167

4.2A.2.1	UE measurement capability	167

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	168

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	168

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	170

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	172

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	172

4.2A.2.7	General requirements	172

4.2B	Cell Re-selection for RedCap	173

4.2B.1	Introduction	173

4.2B.2	Requirements	173

4.2B.2.1	UE measurement capability for RedCap	173

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	173

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	173

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	173

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	175

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	178

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	181

4.2B.2.6	Maximum interruption in paging reception for RedCap	183

4.2B.2.7	General requirements for RedCap	183

4.2B.2.8	Minimum requirement at transitions	183

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	184

4.2B.2.9.1	Introduction	184

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	185

4.2B.2.9.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	188

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criteria	188

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	188

4.2B.2.9.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	189

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	189

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	189

4.2B.2.9.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criterion	189

4.2B.2.9.9	Measurements for UE fulfilling low mobility criterion	190

4.2B.2.9.10	Measurements for UE fulfilling not-at-cell edge criterion	193

4.2B.2.9.11	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	195

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	195

4.2B.2.10.1	Introduction	195

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	196

4.2B.2.10.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	198

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge-criterion	199

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	199

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	200

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	200

4.2B.2.10.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	200

4.2B.2.10.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criteria	200

4.2B.2.10.9	Measurements for UE fulfilling low mobility criterion	201

4.2B.2.10.10	Measurements for UE fulfilling not-at-cell edge criterion	203

4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	206

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	206

4.2B.2.11.1	Introduction	206

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	207

4.2B.2.11.3	Measurements for a UE not-at-cell edge while stationary criterion	208

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	208

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	209

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and stationary not-at-cell-edge while stationary criteria	209

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	209

4.2B.2.11.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	210

4.2B.2.11.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criteria	210

4.2B.2.11.9	Measurements for UE fulfilling low mobility criterion	210

4.2B.2.11.10	Measurements for UE fulfilling with not-at-cell edge criterion	212

4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	213

4.2C	Cell Re-selection for NR UE for Satellite Access	213

4.2C.1	Introduction	213

4.2C.2	Requirements	214

4.2C.2.1	UE measurement capability	214

4.2C.2.2	Measurement and evaluation of serving cell	214

4.2C.2.3	Measurements of intra-frequency NR cells	215

4.2C.2.4	Measurements of inter-frequency NR cells	217

4.2C.2.5	Maximum interruption in paging reception	220

4.2C.2.6	Minimum requirement at transitions	221

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	221

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	221

4.2C.2.9	General requirements	221

4.3	Minimization of Drive Tests (MDT)	221

4.3.1	Introduction	221

4.3.2	Measurement Requirements	222

4.3.3	Requirements for Relative Time Stamp Accuracy	222

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	222

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	223

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	223

4.3C.1	Introduction	223

4.3C.2	Measurement Requirements	223

4.3C.3	Requirements for Relative Time Stamp Accuracy	224

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	224

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	224

4.4	Idle Mode CA/DC Measurements	225

4.4.1	Introduction	225

4.4.2	Measurement Requirements	225

4.4.2.1	Detected cell requirement during state transition and Idle mode	225

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	225

4.4.2.3	Measurements on serving cell	227

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	227

5	SA: RRC_INACTIVE state mobility	228

5.1	Cell Re-selection	228

5.1.1	Introduction	228

5.1.2	Requirements	228

5.1.2.1	UE measurement capability	228

5.1.2.2	Measurement and evaluation of serving cell	228

5.1.2.3	Measurements of intra-frequency NR cells	229

5.1.2.4	Measurements of inter-frequency NR cells	230

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	231

5.1.2.6	Maximum interruption in paging reception	232

5.1.2.7	General requirements	232

5.1.2.8	Minimum requirement at transitions	232

5.1.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	232

5.1.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	233

5.1.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	234

5.1A	Cell Re-selection with CCA	235

5.1A.1	Introduction	235

5.1A.2	Requirements	235

5.1A.2.1	UE measurement capability	235

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	235

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	235

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	235

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	235

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	235

5.1A.2.7	General requirements	235

5.1B	Cell Re-selection for RedCap	235

5.1B.1	Introduction	235

5.1B.2	Requirements	236

5.1B.2.1	UE measurement capability	236

5.1B.2.2	Measurement and evaluation of serving cell	236

5.1B.2.3	Measurements of intra-frequency NR cells	237

5.1B.2.4	Measurements of inter-frequency NR cells	238

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	238

5.1B.2.6	Maximum interruption in paging reception	239

5.1B.2.7	General requirements	239

5.1B.2.8	Minimum requirement at transitions	239

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	239

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	240

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	241

5.1C	Cell Re-selection	242

5.1C.1	Introduction	242

5.1C.2	Requirements	242

5.1C.2.1	UE measurement capability	242

5.1C.2.2	Measurement and evaluation of serving cell	242

5.1C.2.3	Measurements of intra-frequency NR cells	242

5.1C.2.4	Measurements of inter-frequency NR cells	243

5.1C.2.5	Maximum interruption in paging reception	243

5.1C.2.6	General requirements	243

5.2	Void	243

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	243

5.2B.1	Introduction	243

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	243

5.2B.2.1	Void	243

5.2B.3	TA validation requirements for RedCap	243

5.2B.2.2	Void	244

5.2B.2.3	Void	245

5.2B.4	Scheduling restriction	245

5.2B.5	Applicability conditions for CG-SDT for RedCap	245

5.3	Minimization of Drive Tests (MDT)	245

5.3.1	Introduction	245

5.3.2	Measurement Requirements	245

5.3.3	Requirements for Relative Time Stamp Accuracy	245

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	245

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	246

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	246

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	246

5.3C.1	Introduction	246

5.3C.2	Measurement Requirements	246

5.3C.3	Requirements for Relative Time Stamp Accuracy	246

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	246

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	247

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	247

5.4	Inactive Mode CA/DC Measurements	247

5.4.1	Introduction	247

5.4.2	Measurement Requirements	247

5.4.2.1	Detected cell requirement during state transition and inactive mode	247

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	247

5.4.2.3	Measurements on serving cell	247

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	247

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	247

5.5.1	Introduction	247

5.5.2	Requirements on UE synchronization for small data transmissions	248

5.5.3	TA validation requirements	248

5.5.4	Scheduling restriction	249

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	249

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	249

5.5.4.3	Scheduling availability of UE performing measurements on FR2	249

5.5.5	Applicability conditions for SDT	250

5.6	NR measurements for positioning	250

5.6.1	Introduction	250

5.6.2	RSTD measurements	251

5.6.2.1	Introduction	251

5.6.2.2	Requirements Applicability	251

5.6.2.3	Measurement Capability	251

5.6.2.5	Measurements Period Requirements	252

5.6.3	PRS-RSRP measurements	254

5.6.3.1	Introduction	254

5.6.3.2	Requirements applicability	254

5.6.3.3	Measurement Capability	254

5.6.3.4	Measurement Reporting Requirements	254

5.6.3.5	Measurement Period Requirements	255

5.6.4	UE Rx-Tx time difference measurements	257

5.6.4.1	Introduction	257

5.6.4.2	Requirements Applicability	257

5.6.4.3	Measurement Capability	257

5.6.4.4	Measurement Reporting Requirements	258

5.6.4.5	Measurement Period Requirements	258

5.6.5	PRS-RSRPP measurements	261

5.6.5.1	Introduction	261

5.6.5.2	Requirements Applicability	261

5.6.5.3	Measurement Capability	261

5.6.5.4	Measurement Reporting Requirements	261

5.6.5.5	Measurement Period Requirements	261

5.7	Random access based Small Data Transmissions (RA-SDT)	263

5.7.1	Introduction	263

5.7.2	Requirements for small data transmissions based on 2-step RA	263

5.7.3	Requirements for small data transmissions based on 4-step RA	263

5.7.4	Applicability conditions for SDT	263

5.7B	Random access based Small Data Transmissions (RA-SDT) for RedCap	263

5.7B.1	Introduction	263

5.7B.2	Requirements for small data transmissions based on 2-step RA	263

5.7B.3	Requirements for small data transmissions based on 4-step RA	263

5.7B.4	Applicability conditions for RA-SDT for RedCap	263

6	RRC_CONNECTED state mobility	264

6.1	Handover	264

6.1.1	NR Handover	264

6.1.1.1	Introduction	264

6.1.1.2	NR FR1 - NR FR1 Handover	264

6.1.1.2.1	Handover delay	264

6.1.1.2.2	Interruption time	264

6.1.1.3	NR FR2- NR FR1 Handover	265

6.1.1.3.1	Handover delay	265

6.1.1.3.2	Interruption time	265

6.1.1.4	NR FR2- NR FR2 Handover	266

6.1.1.4.1	Handover delay	266

6.1.1.4.2	Interruption time	266

6.1.1.5	NR FR1- NR FR2 Handover	267

6.1.1.5.1	Handover delay	267

6.1.1.5.2	Interruption time	267

6.1.2	NR Handover to other RATs	268

6.1.2.1	NR – E-UTRAN Handover	268

6.1.2.1.1	Introduction	268

6.1.2.1.2	Handover delay	268

6.1.2.1.3	Interruption time	268

6.1.2.2	NR – UTRAN Handover	269

6.1.2.2.1	Introduction	269

6.1.2.2.2	Handover delay	269

6.1.2.2.3	Interruption time	269

6.1.3	NR DAPS Handover	270

6.1.3.1	Introduction	270

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	270

6.1.3.2.1	DAPS handover delay	270

6.1.3.2.2	Interruption time	271

6.1.3.3	NR FR2- NR FR1 DAPS Handover	273

6.1.3.3.1	DAPS handover delay	273

6.1.3.3.2	Interruption time	273

6.1.3.4	NR FR1- NR FR2 DAPS Handover	274

6.1.3.4.1	DAPS handover delay	274

6.1.3.4.2	Interruption time	274

6.1.4	NR Conditional Handover	275

6.1.4.1	Introduction	275

6.1.4.2	NR FR1 – NR FR1 conditional handover	275

6.1.4.3	NR FR2 – NR FR1 conditional handover	276

6.1.4.4	NR FR2 – NR FR2 conditional handover	277

6.1.4.4.1	Handover delay	277

6.1.4.4.2	Measurement time	277

6.1.4.4.3	Preparation time	277

6.1.4.4.4	Interruption time	277

6.1.4.5	NR FR1 – NR FR2 conditional handover	278

6.1.5	NR Handover with PSCell	278

6.1.5.1	Introduction	278

6.1.5.2	Handover with PSCell from NR SA to EN-DC	278

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	279

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	279

6.1.5.3	HO with PSCell from NE-DC to NE-DC	279

6.1.5.3.1	Handover delay	280

6.1.5.3.2	HO with PSCell - PCell Interruption time	280

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	280

6.1.5.4	HO with PSCell from NR-DC to NR-DC	280

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	282

6.1.5.5.1	Introduction	282

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	282

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	282

6.1A	Void	283

6.1A.1	Void	283

6.1A.1.1	Void	283

6.1A.1.2	Void	283

6.1A.1.2.1	Void	283

6.1A.1.2.2	Void	283

6.1B	Handover to target cell using CCA	284

6.1B.1	NR Handover	284

6.1B.1.1	Introduction	284

6.1B.1.2	NR FR1 - NR FR1 Handover	284

6.1B.1.2.1	Handover delay	284

6.1B.1.2.2	Interruption time	284

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	285

6.1B.1.3.1	Handover delay	285

6.1B.1.3.2	Interruption time	285

6.1B.1.4	NR FR1- NR FR2-2 Handover	286

6.1B.1.4.1	Handover delay	286

6.1B.1.4.2	Interruption time	286

6.1C	Handover for SAN	287

6.1C.1	NR SAN Handover	287

6.1C.1.1	Introduction	287

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	287

6.1C.1.2.1	Handover delay	288

6.1C.1.2.2	Interruption time	288

6.1C.2	NR SAN Conditional Handover	289

6.1C.2.1	Introduction	289

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	289

6.1C.2.2.1	Handover delay	289

6.1C.2.2.2	Measurement time	289

6.1C.2.2.3	Preparation time	290

6.1C.2.2.4	Interruption time	290

6.1D	Handover for RedCap	291

6.1D.1	NR Handover	291

6.1D.1.1	Introduction	291

6.1D.1.2	NR FR1 - NR FR1 Handover	291

6.1D.1.2.1	Handover delay	291

6.1D.1.2.2	Interruption time	292

6.1D.1.3	NR FR2- NR FR2 Handover	293

6.1D.1.3.1	Handover delay	293

6.1D.1.3.2	Interruption time	293

6.1D.2	NR Handover to other RATs	295

6.1D.2.1	NR – E-UTRAN Handover	295

6.2	RRC Connection Mobility Control	295

6.2.1	SA: RRC Re-establishment	295

6.2.1.1	Introduction	295

6.2.1.2	Requirements	295

6.2.1.2.1	UE Re-establishment delay requirement	295

6.2.1A	RRC Re-establishment with CCA	297

6.2.1A.1	Introduction	297

6.2.1A.2	Requirements	297

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	297

6.2.1B	SA: RRC Re-establishment for RedCap	299

6.2.1B.1	Introduction	299

6.2.1B.2	Requirements	299

6.2.2	Random access	300

6.2.2.1	Introduction	300

6.2.2.2	Requirements for 4-step RA type	300

6.2.2.2.1	Contention based random access	300

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	300

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	301

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	301

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	301

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	301

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	301

6.2.2.2.2	Non-Contention based random access	301

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	301

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	302

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	302

6.2.2.2.3	UE behaviour when configured with supplementary UL	302

6.2.2.3	Requirements for 2-step RA type	302

6.2.2.3.1	Contention based random access	303

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	303

6.2.2.3.1.2	Correct behaviour when receiving MsgB	303

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	303

6.2.2.3.2	Non-Contention based random access	303

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	303

6.2.2.3.2.2	Correct behaviour when receiving MsgB	304

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	304

6.2.2.3.3	UE behaviour when configured with supplementary UL	304

6.2.2A	Random access when CCA is used on target frequency	304

6.2.2A.1	Introduction	304

6.2.2A.2	Requirements for 4-step RA type	304

6.2.2A.2.1	Contention based random access	305

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	305

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	305

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	305

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	305

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	306

6.2.2A.2.2	Non-Contention based random access	306

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	306

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	306

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	307

6.2.2A.3	Requirements for 2-step RA type	307

6.2.2A.3.1	Contention based random access	307

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	307

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	308

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	308

6.2.2A.3.2	Non-Contention based random access	308

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	308

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	309

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	309

6.2.2B	Random access for RedCap	309

6.2.2B.1	Introduction	309

6.2.2B.2	Requirements	309

6.2.3	SA: RRC Connection Release with Redirection	310

6.2.3.1	Introduction	310

6.2.3.2	Requirements	310

6.2.3.2.1	RRC connection release with redirection to NR	310

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	311

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	311

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	313

6.2.3A.1	Introduction	313

6.2.3A.2	Requirements	313

6.2.3A.2.1	RRC connection release with redirection to NR	313

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	313

6.2C	RRC Connection Mobility Control for Satellite Access	313

6.2C.1	SA: RRC Re-establishment for Satellite Access	313

6.2C.1.1	Introduction	313

6.2C.1.2	Requirements	314

6.2C.1.2.1	UE Re-establishment delay requirement	314

6.2C.2	Random access for satellite access	315

6.2C.2.1	Introduction	315

6.2C.2.2	Requirements for 4-step RA type	315

6.2C.2.2.1	Contention based random access	316

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	316

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	316

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	316

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	316

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	316

6.2C.2.2.1.6	Correct behaviour when contention Resolution timer expires	316

6.2C.2.2.2	Non-Contention based random access	316

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	316

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	317

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	317

6.2C.2.3	Requirements for 2-step RA type	317

6.2C.2.3.1	Contention based random access	318

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	318

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	318

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	318

6.2C.2.3.2	Non-Contention based random access	318

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	318

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	319

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	319

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	319

6.2C.3.1	Introduction	319

6.2C.3.2	Requirements	319

6.2C.3.2.1	RRC connection release with redirection to NR	319

7	Timing	320

7.1	UE transmit timing	320

7.1.1	Introduction	320

7.1.2	Requirements	320

7.1.2.1	Gradual timing adjustment	322

7.1.2.2	Void	323

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	323

7.1A	UE transmit timing for RedCap	323

7.1A.1	Introduction	323

7.1A.2	Requirements	324

7.1A.2.1	Gradual timing adjustment	325

7.1C	UE transmit timing for Satellite Access	325

7.1C.1	Introduction	325

7.1C.2	Requirements	325

7.1C.2.1	Gradual timing adjustment	326

7.2	UE timer accuracy	327

7.2.1	Introduction	327

7.2.2	Requirements	327

7.2A	UE timer accuracy for RedCap	327

7.2A.1	Introduction	327

7.2A.2	Requirements	327

7.2C	UE timer accuracy for satellite access	327

7.2C.1	Introduction	327

7.2C.2	Requirements	328

7.3	Timing advance	328

7.3.1	Introduction	328

7.3.2	Requirements	328

7.3.2.1	Timing Advance adjustment delay	328

7.3.2.2	Timing Advance adjustment accuracy	328

7.3A	Timing Advance for RedCap	328

7.3A.1	Introduction	328

7.3A.2	Requirements	329

7.3A.2.1	Timing Advance adjustment delay	329

7.3A.2.2	Timing Advance adjustment accuracy	329

7.3C	Timing advance for satellite access	329

7.3C.1	Introduction	329

7.3C.2	Requirements	329

7.3C.2.1	Timing Advance adjustment delay	329

7.3C.2.2	Timing Advance adjustment accuracy	329

7.4	Cell phase synchronization accuracy	330

7.4.1	Definition	330

7.4.2	Minimum requirements	330

7.5	Maximum Transmission Timing Difference	330

7.5.1	Introduction	330

7.5.2	Minimum Requirements for inter-band EN-DC	330

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	331

7.5.3	Minimum Requirements for intra-band EN-DC	331

7.5.4	Minimum Requirements for NR Carrier Aggregation	332

7.5.5	Minimum Requirements for inter-band NE-DC	332

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	333

7.5.6	Minimum Requirements for inter-band NR DC	333

7.6	Maximum Receive Timing Difference	334

7.6.1	Introduction	334

7.6.2	Minimum Requirements for inter-band EN-DC	334

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	335

7.6.3	Minimum Requirements for intra-band EN-DC	335

7.6.4	Minimum Requirements for NR Carrier Aggregation	336

7.6.5	Minimum Requirements for inter-band NE-DC	337

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	337

7.6.6	Minimum Requirements for inter-band NR DC	337

7.7	deriveSSB-IndexFromCell tolerance	338

7.7.1	Minimum requirements	338

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	339

7.7A.1	Minimum requirements	339

7.8	Void	339

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	339

7.9.1	Minimum requirements	339

8	Signalling characteristics	340

8.1	Radio Link Monitoring	340

8.1.1	Introduction	340

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	341

8.1.2	Requirements for SSB based radio link monitoring	341

8.1.2.1	Introduction	342

8.1.2.2	Minimum requirement	342

8.1.2.3	Measurement restrictions for SSB based RLM	346

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	346

8.1.3	Requirements for CSI-RS based radio link monitoring	347

8.1.3.1	Introduction	347

8.1.3.2	Minimum requirement	348

8.1.3.3	Measurement restrictions for CSI-RS based RLM	351

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	352

8.1.4	Minimum requirement at transitions	353

8.1.5	Minimum requirement for UE turning off the transmitter	353

8.1.6	Minimum requirement for L1 indication	353

8.1.7	Scheduling availability of UE during radio link monitoring	354

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	354

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	354

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	354

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	355

8.1A	Radio Link Monitoring with CCA on Target Frequency	355

8.1A.1	Introduction	355

8.1A.2	Requirements for SSB Based Radio Link Monitoring	356

8.1A.2.1	Introduction	356

8.1A.2.2	Minimum Requirement	357

8.1A.3	Minimum requirement at transitions	360

8.1A.4	Minimum requirement for UE turning off the transmitter	361

8.1A.5	Minimum requirement for L1 indication	361

8.1A.6	Scheduling availability of UE during radio link monitoring	361

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	361

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	362

8.1B	Radio Link Monitoring for RedCap	362

8.1B.1	Introduction	362

8.1B.2	Requirements for SSB based radio link monitoring	363

8.1B.2.1	Introduction	363

8.1B.2.2	Minimum requirement	364

8.1B.2.3	Measurement restrictions for SSB based RLM	366

8.1B.3	Requirements for CSI-RS based radio link monitoring	367

8.1B.3.1	Introduction	367

8.1B.3.2	Minimum requirement	367

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	370

8.1B.4	Minimum requirement at transitions	371

8.1B.5	Minimum requirement for UE turning off the transmitter	371

8.1B.6	Minimum requirement for L1 indication	371

8.1B.7	Scheduling availability of UE during radio link monitoring	371

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	372

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	372

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	372

8.1C	Radio Link Monitoring for Satellite Access	372

8.1C.1	Introduction	372

8.1C.2	Requirements for SSB based radio link monitoring	373

8.1C.2.1	Introduction	373

8.1C.2.2	Minimum requirement	374

8.1C.2.3	Measurement restrictions for SSB based RLM	375

8.1C.3	Requirements for CSI-RS based radio link monitoring	375

8.1C.3.1	Introduction	375

8.1C.3.2	Minimum requirement	376

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	377

8.1C.4	Minimum requirement at transitions	378

8.1C.5	Minimum requirement for UE turning off the transmitter	378

8.1C.6	Minimum requirement for L1 indication	378

8.1C.7	Scheduling availability of UE during radio link monitoring	378

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	379

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	379

8.2	Interruption	379

8.2.1	EN-DC Interruption	379

8.2.1.1	Introduction	379

8.2.1.2	Requirements	380

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	380

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	380

8.2.1.2.3	Interruptions at SCell addition/release	380

8.2.1.2.4	Interruptions at SCell activation/deactivation	381

8.2.1.2.5	Interruptions during measurements on SCC	383

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC	383

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	383

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	383

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	384

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	384

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	385

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	386

8.2.1.2.8.1	Interruptions during direct SCell activation and hibernation of E-UTRA SCell	386

8.2.1.2.8.2	Interruptions during direct SCell activation	386

8.2.1.2.9	Interruptions at SCell hibernation	386

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	387

8.2.1.2.11	Interruptions due to UE-specific CBW change	387

8.2.1.2.12	Interruptions at NR SRS carrier based switching	387

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	389

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	390

8.2.1.2.15	Interruptions due to SCell dormancy	390

8.2.1.2.15.1	Interruptions due to SCell dormancy switch	390

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy	390

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy	391

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	391

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	391

8.2.1.2.18	Interruptions at NR SRS antenna port switching	392

8.2.1.2.19	Interruptions at fast SCell activation	393

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	394

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	394

8.2.2.1	Introduction	394

8.2.2.2	Requirements	395

8.2.2.2.1	Interruptions at SCell addition/release	395

8.2.2.2.2	Interruptions at SCell activation/deactivation	396

8.2.2.2.3	Interruptions during measurements on deactivated SCC	397

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	398

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	398

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	399

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	401

8.2.2.2.8	Interruptions due to UE-specific CBW change	401

8.2.2.2.9	Interruptions at NR SRS carrier based switching	401

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	403

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	403

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	404

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	404

8.2.2.2.11	Interruptions at direct SCell activation	404

8.2.2.2.12	Interruptions due to SCell dormancy	405

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	405

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	405

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	405

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	405

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	405

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	406

8.2.2.2.16	Interruptions at NR SRS antenna port switching	406

8.2.2.2.17	Interruptions at fast SCell activation	407

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	408

8.2.3	NE-DC Interruptions	408

8.2.3.1	Introduction	408

8.2.3.2	Requirements	409

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	409

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	409

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	409

8.2.3.2.4	Interruptions at SCell activation/deactivation	411

8.2.3.2.5	Interruptions during measurements on SCC	412

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	412

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	412

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	413

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	413

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	413

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	414

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	414

8.2.3.2.9	Interruptions at SCell hibernation	414

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	415

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	415

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	416

8.2.3.2.13	Interruptions due to SCell dormancy	417

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	418

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	418

8.2.3.2.17	Interruptions at fast SCell activation	420

8.2.3.2.18	Interruptions due to UE-specific CBW change	420

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	421

8.2.4	NR-DC: Interruptions	421

8.2.4.1	Introduction	421

8.2.4.2	Requirements	422

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	422

8.2.4.2.2	Interruptions at SCell activation/deactivation	423

8.2.4.2.3	Interruptions during measurements on SCC	424

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	424

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	425

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	425

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	425

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	426

8.2.4.2.9	Interruptions at NR SRS carrier based switching	426

8.2.4.2.10	Interruptions at direct SCell activation	427

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	428

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	428

8.2.4.2.13	 Interruptions due to SCell dormancy	429

8.2.4.2.14	Interruptions at NR SRS antenna port switching	429

8.2.4.2.15	Interruptions at fast SCell activation	430

8.2.4.2.16	Interruptions at SCG activation/deactivation	431

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	431

8.2.4.2.18 Interruptions during RLM/BFD measurements on deactivated PSCell	432

8.2.4.2.19	Interruptions due to UE-specific CBW change	432

8.2.4.2A	Void	432

8.2.4.2A.1	Void	432

8.2.4.2A.2	Void	432

8.2.4.2A.3	Void	432

8.3	SCell Activation and Deactivation Delay	432

8.3.1	Introduction	432

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	432

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	437

8.3.4	Direct SCell Activation at SCell addition	437

8.3.5	Direct SCell Activation at Handover	439

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	440

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	444

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	445

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	446

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	447

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	450

8.3.13.1	Introduction	450

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	452

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	452

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	453

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	455

8.3A.1	Introduction	455

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	456

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	460

8.4	UE UL carrier RRC reconfiguration delay	460

8.4.1	Introduction	460

8.4.2	UE UL carrier configuration delay requirement	461

8.4.3	UE UL carrier deconfiguration delay requirement	461

8.5	Link Recovery Procedures	461

8.5.1	Introduction	461

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	462

8.5.2	Requirements for SSB based beam failure detection	463

8.5.2.1	Introduction	463

8.5.2.2	Minimum requirement	463

8.5.2.3	Measurement restriction for SSB based beam failure detection	466

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	467

8.5.3	Requirements for CSI-RS based beam failure detection	468

8.5.3.1	Introduction	468

8.5.3.2	Minimum requirement	468

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	472

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	473

8.5.4	Minimum requirement for L1 indication	473

8.5.5	Requirements for SSB based candidate beam detection	474

8.5.5.1	Introduction	474

8.5.5.2	Minimum requirement	474

8.5.5.3	Measurement restriction for SSB based candidate beam detection	477

8.5.6	Requirements for CSI-RS based candidate beam detection	477

8.5.6.1	Introduction	477

8.5.6.2	Minimum requirement	478

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	481

8.5.7	Scheduling availability of UE during beam failure detection	481

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	481

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	482

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	482

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	483

8.5.8	Scheduling availability of UE during candidate beam detection	483

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	483

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	483

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	483

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	484

8.5.9	Requirements for Beam Failure Recovery in SCell	484

8.5.9.1	Introduction	484

8.5.9.2	Requirement	484

8.5.10	Minimum requirement at transitions for beam failure detection	484

8.5A	Link Recovery Procedures when CCA is used on target frequency	485

8.5A.1	Introduction	485

8.5A.2	Requirements for SSB based beam failure detection	486

8.5A.2.1	Introduction	486

8.5A.2.2	Minimum requirement	486

8.5A.2.3	Measurement restriction for SSB based beam failure detection	488

8.5A.3	Void	488

8.5A.4	Minimum requirement for L1 indication	488

8.5A.5	Requirements for SSB based candidate beam detection	489

8.5A.5.1	Introduction	489

8.5A.5.2	Minimum requirement	489

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	491

8.5A.6	Void	492

8.5A.7	Scheduling availability of UE during beam failure detection	492

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	492

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	492

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	492

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR DC	492

8.5A.8	Scheduling availability of UE during candidate beam detection	492

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	492

8.5A.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	493

8.5B	Link Recovery Procedures for Redcap	493

8.5B.1	Introduction	493

8.5B.2	Requirements for SSB based beam failure detection for Redcap	493

8.5B.2.1	Introduction	493

8.5B.2.2	Minimum requirement	494

8.5B.2.3	Measurement restriction for SSB based beam failure detection	495

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	496

8.5B.3.1	Introduction	496

8.5B.3.2	Minimum requirement	496

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	498

8.5B.4	Minimum requirement for L1 indication for Redcap	499

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	500

8.5B.5.1	Introduction	500

8.5B.5.2	Minimum requirement	500

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	502

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	502

8.5B.6.1	Introduction	502

8.5B.6.2	Minimum requirement	502

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	504

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	505

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	505

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	505

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	505

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	506

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	506

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	506

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	506

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	506

8.5C	Link Recovery Procedures for Satellite Access	507

8.5C.1	Introduction	507

8.5C.2	Requirements for SSB based beam failure detection	507

8.5C.2.1	Introduction	507

8.5C.2.2	Minimum requirement	508

8.5C.2.3	Measurement restriction for SSB based beam failure detection	508

8.5C.3	Requirements for CSI-RS based beam failure detection	509

8.5C.3.1	Introduction	509

8.5C.3.2	Minimum requirement	509

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	510

8.5C.4	Minimum requirement for L1 indication	511

8.5C.5	Requirements for SSB based candidate beam detection	511

8.5C.5.1	Introduction	511

8.5C.5.2	Minimum requirement	511

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	512

8.5C.6	Requirements for CSI-RS based candidate beam detection	513

8.5C.6.1	Introduction	513

8.5C.6.2	Minimum requirement	513

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	514

8.5C.7	Scheduling availability of UE during beam failure detection	514

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	514

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	514

8.5C.8	Scheduling availability of UE during candidate beam detection	515

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	515

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	515

8.5C.9	Minimum requirement at transitions for beam failure detection	515

8.6	Active BWP switch delay	515

8.6.1	Introduction	515

8.6.2	DCI and timer based BWP switch delay on a single CC	516

8.6.2A	DCI based BWP switch delay on multiple CCs	517

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	517

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	519

8.6.2B	Timer based BWP switch delay on multiple CCs	519

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	519

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	519

8.6.3	RRC based BWP switch delay on a single CC	520

8.6.3A	RRC based BWP switch delay on multiple CCs	521

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	521

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	521

8.6.4	BWP switch delay on Consistent UL CCA recovery	522

8.6A	Active BWP switch delay for RedCap	522

8.6A.1	Introduction	522

8.6A.2	DCI and timer based BWP switch delay on a single CC	522

8.6A.3	RRC based BWP switch delay on a single CC	524

8.6C	Active BWP switch delay for satellite access	524

8.6C.1	Introduction	524

8.6C.2	DCI and timer based BWP switch delay on a single CC	524

8.6C.3	RRC based BWP switch delay on a single CC	526

8.7	Void	526

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	526

8.8.1	Introduction	526

8.8.2	E-UTRAN PSCell Addition Delay Requirement	526

8.8.3	E-UTRAN PSCell Release Delay Requirement	527

8.9	NR-DC: PSCell Addition and Release Delay	527

8.9.1	Introduction	527

8.9.2	PSCell Addition Delay Requirement	527

8.9.3	PSCell Release Delay Requirement	528

8.9A	Conditional PSCell Addition Delay	528

8.9A.1	Introduction	528

8.9A.2	Conditional PSCell Addition Delay Requirement	528

8.9A.2.1	Measurement time	529

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	529

8.9B.1	Introduction	529

8.9B.2	PSCell Addition Delay Requirement	530

8.9B.3	PSCell Release Delay Requirement	530

8.10	Active TCI state switching delay	531

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	532

8.10.4	DCI based TCI state switch delay	532

8.10.5	RRC based TCI state switch delay	533

8.10.6	Active TCI state list update delay	533

8.10A	Active TCI state switching delay with CCA	533

8.10A.1	Introduction	533

8.10A.2	Known conditions for TCI state	534

8.10A.3	MAC-CE based TCI state switch delay	534

8.10A.4	DCI based TCI state switch delay	535

8.10A.5	RRC based TCI state switch delay	535

8.10A.6	Active TCI state list update delay	536

8.10B	Active TCI state switching delay for RedCap	536

8.10B.1	Introduction	536

8.10B.2	Known conditions for TCI state	536

8.10B.3	MAC-CE based TCI state switch delay	537

8.10B.4	DCI based TCI state switch delay	538

8.10B.5	RRC based TCI state switch delay	538

8.10B.6	Active TCI state list update delay	538

8.10C	Active TCI state switching delay for satellite access	539

8.10C.1	Introduction	539

8.10C.2	MAC-CE based TCI state switch delay	539

8.10C.4	DCI based TCI state switch delay	539

8.10C.5	RRC based TCI state switch delay	539

8.10C.6	Active TCI state list update delay	539

8.11	PSCell Change	540

8.11A	PSCell Change in Carriers with CCA	540

8.11B	Conditional PSCell Change	540

8.11B.1	Introduction	540

8.11B.2	Conditoinal PSCell Change delay	541

8.11B.2.1	Measurement time	541

8.11D	Conditional PSCell Change in Carriers with CCA	542

8.11D.1	Introduction	542

8.11D.2	Conditional PSCell Change delay	542

8.11D.2.1	Measurement time	543

8.12	Uplink spatial relation switch delay	543

8.12.1	Introduction	543

8.12.2	Known conditions for spatial relation when associated with DL-RS	543

8.12.3	MAC-CE based spatial relation switch delay	544

8.12.4	DCI based spatial relation switch delay	544

8.12.5	RRC based spatial relation switch delay	545

8.12A	Uplink spatial relation switch delay for RedCap	545

8.12A.1	Introduction	545

8.12A.2	Known conditions for spatial relation when associated with DL-RS	545

8.12A.3	MAC-CE based spatial relation switch delay	546

8.12A.4	DCI based spatial relation switch delay	546

8.12A.5	RRC based spatial relation switch delay	547

8.12C	Uplink spatial relation switch delay for satellite access	547

8.12C.1	Void	548

8.12C.2	Void	548

8.12C.3	Void	548

8.12C.4	Void	548

8.12C.5	Void	548

8.13	UE-specific CBW change	548

8.13.1	Introduction	548

8.13.2	UE-specific CBW change delay	548

8.13A	UE-specific CBW change for RedCap	548

8.13A.1	Introduction	548

8.13A.2	UE-specific CBW change delay	548

8.13C	UE-specific CBW change for satellite access	549

8.13C.1	Introduction	549

8.13C.2	UE-specific CBW change delay	549

8.14	Pathloss reference signal switching delay	549

8.14.1	Introduction	549

8.14.2	Known conditions for pathloss reference signal	550

8.14.3	MAC-CE based pathloss reference signal switch delay	550

8.14C	Pathloss reference signal switching delay for satellite access	551

8.14C.1	Introduction	551

8.14C.2	Known conditions for pathloss reference signal	551

8.14C.3	MAC-CE based pathloss reference signal switch delay	551

8.15	Active downlink TCI state switching delay for unified TCI	552

8.15.1	Introduction	552

8.15.4	DCI based downlink TCI state switch delay	554

8.15.5	Active Downlink TCI state list update delay	554

8.16	Active uplink TCI state switching delay for unified TCI	555

8.16.1	Introduction	555

8.16.4	DCI based uplink TCI state switch delay	557

8.16.5	Active Uplink TCI state list update delay	558

8.17	SCG Activation and Deactivation Delay	559

8.17.1	Introduction	559

8.17.2	SCG Activation Delay Requirement	559

8.17.3	SCG Deactivation Delay Requirement	560

8.18	TRP specific Link Recovery Procedures	561

8.18.1	Introduction	561

8.18.2	Requirements for TRP specific SSB based beam failure detection	561

8.18.2.1	Introduction	561

8.18.2.2	Minimum requirement	562

8.18.2.3	Measurement restriction for SSB based beam failure detection	564

8.18.3	Requirements for CSI-RS based beam failure detection	564

8.18.3.1	Introduction	564

8.18.3.2	Minimum requirement	565

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	567

8.18.4	Minimum requirement for L1 indication	568

8.18.5	Requirements for SSB based candidate beam detection	568

8.18.5.1	Introduction	568

8.18.5.2	Minimum requirement	568

8.18.5.3	Measurement restriction for SSB based candidate beam detection	570

8.18.6	Requirements for CSI-RS based candidate beam detection	571

8.18.6.1	Introduction	571

8.18.6.2	Minimum requirement	571

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	573

8.18.7	Requirements for TRP specific Beam Failure Recovery	574

8.18.7.1	Introduction	574

8.18.7.2	Requirement	574

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	574

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	574

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	575

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	575

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	575

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	576

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	576

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	576

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	576

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	577

8.19	Pre-configured measurement gap activation/deactivation delay	577

8.19.1	Introduction	577

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	577

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	577

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	577

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	578

9	Measurement Procedure	579

9.1	General measurement requirement	579

9.1.1	Introduction	579

9.1.2	Measurement gap	579

9.1.2.1	EN-DC: Measurement Gap Sharing	591

9.1.2.1a	SA: Measurement Gap Sharing	592

9.1.2.1b	NE-DC: Measurement Gap Sharing	593

9.1.2.1c	NR-DC: Measurement Gap Sharing	594

9.1.3	UE Measurement capability	595

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	595

9.1.3.1a	SA: Monitoring of multiple layers using gaps	595

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	596

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	596

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	597

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	598

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	598

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	599

9.1A.3.2	Void	600

9.1.3A	UE Measurement capability under operation mode with CCA	600

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	600

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	600

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	600

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	601

9.1.3C	UE Measurement capability under operation mode with satellite access	601

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	601

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	602

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	602

9.1.4.1	Introduction	602

9.1.4.2	Requirements	602

9.1.5	Carrier-specific scaling factor	606

9.1.5.1	Monitoring of multiple layers outside gaps	607

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	609

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	610

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	611

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	611

9.1.5.2	Monitoring of multiple layers within gaps	612

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	614

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	616

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	618

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	620

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	621

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	622

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	622

9.1.5.3	Monitoring of multiple layers within NCSG	622

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	623

9.1.6	Minimum requirement at transitions	623

9.1.7	Pre-configured measurement gap	624

9.1.7.1	Introduction	624

9.1.7.2	Requirements applicability	624

9.1.7.3	Requirements	624

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	625

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	625

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	626

9.1.8	Concurrent measurement gaps	626

9.1.8.1	Introduction	626

9.1.8.2	Requirements	626

9.1.8.3	Collision between concurrent measurement gaps	628

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	628

9.1.9	Network controlled small gap	628

9.1.9.1	Introduction	628

9.1.9.2	Requirements applicability	629

9.1.10	MUSIM gaps	632

9.1.11	UL gap for Tx power management	633

9.1A	General measurement requirement for RedCap	634

9.1A.1	Introduction	634

9.1A.2	Measurement gap	634

9.1A.2.1	SA: Measurement Gap Sharing	638

9.1A.3	UE Measurement capability	639

9.1A.3.1	SA: Monitoring of multiple layers using gaps	639

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	639

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	639

9.1A.4.1	Introduction	639

9.1A.4.2	Requirements	640

9.1A.5	Carrier-specific scaling factor	640

9.1A.5.1	Monitoring of multiple layers outside gaps	640

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	641

9.1A.5.2	Monitoring of multiple layers within gaps	641

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	641

9.1A.6	Minimum requirement at transitions	642

9.1C	General measurement requirement for SAN	643

9.1C.1	Introduction	643

9.1C.2	Measurement gap	643

9.1C.8	Concurrent measurement gaps for SAN	645

9.1C.8.1	Introduction	645

9.1C.8.2	Requirements	645

9.1C.8.3	Collision between concurrent measurement gaps	645

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	646

9.1C.9	Collision between SMTC and measurement gap for SAN	646

9.1C.9.1	Introduction	646

9.1C.9.2	Collision between SMTCs and measurement gap	646

9.1C.9.3	Collision between multiple SMTCs on a SAN carrier	646

9.2	NR intra-frequency measurements	647

9.2.1	Introduction	647

9.2.2	Requirements applicability	649

9.2.3	Number of cells and number of SSB	649

9.2.3.1	Requirements for FR1	649

9.2.3.2	Requirements for FR2	649

9.2.4	Measurement Reporting Requirements	649

9.2.4.1	Periodic Reporting	649

9.2.4.2	Event-triggered Periodic Reporting	650

9.2.4.3	Event Triggered Reporting	650

9.2.5	Intrafrequency measurements without measurement gaps	650

9.2.5.1	Intrafrequency cell identification	650

9.2.5.2	Measurement period	655

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	658

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	658

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	658

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	659

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	660

9.2.5.4	SFTD Measurements between PCell and PSCell	661

9.2.5.4.1	Introduction	661

9.2.5.4.2	SFTD Measurement delay	661

9.2.5.4.3	SFTD Measurement Reporting Delay	661

9.2.6	Intra-frequency measurements with measurement gaps	662

9.2.6.1	Void	662

9.2.6.2	Intra-frequency cell identification	662

9.2.6.3	Intrafrequency Measurement Period	664

9.2.7	Intra-frequency measurements with NCSG	666

9.2.7.1	Intra-frequency cell identification	666

9.2.7.2	Measurement period	668

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	669

9.2A	NR intra-frequency measurements with CCA	669

9.2A.1	Introduction	669

9.2A.2	Requirements applicability	670

9.2A.3	Number of cells and number of SSB	670

9.2A.3.1	Requirements for FR1	670

9.2A.3.2	Requirements for FR2-2	671

9.2A.4	Measurement Reporting Requirements	671

9.2A.5	Intra-frequency measurements without measurement gaps	672

9.2A.5.2	Measurement period	676

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	678

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	679

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	679

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	679

9.2A.6	Intra-frequency measurements with measurement gaps	679

9.2A.6.1	Intra-frequency cell identification	679

9.2A.6.2	Intra-frequency Measurement Period	682

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	683

9.2A.7.1	Intra-frequency RSSI measurements	683

9.2A.7.2	Intra-frequency Channel occupancy measurements	685

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	686

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	686

9.2B	NR intra-frequency measurements for RedCap	687

9.2B.1	Introduction	687

9.2B.2	Requirements applicability	687

9.2B.3	Number of cells and number of SSB	688

9.2B.3.1	Requirements for FR1	688

9.2B.3.2	Requirements for FR2	688

9.2B.4	Measurement Reporting Requirements	688

9.2B.4.1	Periodic Reporting	688

9.2B.4.2	Event-triggered Periodic Reporting	688

9.2B.4.3	Event Triggered Reporting	688

9.2B.5	Intra-frequency measurements without measurement gaps	689

9.2B.5.1	Intra-frequency cell identification	689

9.2B.5.2	Measurement period	691

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	692

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	692

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	692

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	693

9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	693

9.2B.6	Intra-frequency measurements with measurement gaps	694

9.2B.6.1	Intra-frequency cell identification	694

9.2B.6.2	Intra-frequency Measurement Period	695

9.2C	NR intra-frequency measurements for SAN	696

9.2C.1	Introduction	696

9.2C.2	Requirements applicability	697

9.2C.3	Number of cells and number of SSB	697

9.2C.3.1	Requirements for FR1	697

9.2C.4	Measurement Reporting Requirements	697

9.2C.4.1	Periodic Reporting	697

9.2C.4.2	Event-triggered Periodic Reporting	698

9.2C.4.3	Event Triggered Reporting	698

9.2C.5	Intra frequency measurements without measurement gaps	698

9.2C.5.1	Intra frequency cell identification	698

9.2C.5.2	Measurement period	700

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	701

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	701

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	701

9.2C.6	Intra-frequency measurements with measurement gaps	702

9.2C.6.1	Intra-frequency cell identification	702

9.2C.6.3	Intrafrequency Measurement Period	703

9.3	NR inter-frequency measurements	703

9.3.1	Introduction	703

9.3.2	Requirements applicability	705

9.3.2.1	Void	706

9.3.2.2	Void	706

9.3.3	Number of cells and number of SSB	706

9.3.3.1	Requirements for FR1	706

9.3.3.2	Requirements for FR2	706

9.3.4	Inter-frequency measurement with measurement gaps	706

9.3.4.1	Void	709

9.3.4.2	Void	709

9.3.5	Inter-frequency measurements	709

9.3.5.1	Void	711

9.3.5.2	Void	711

9.3.5.3	Void	711

9.3.6	Inter-frequency measurements reporting requirements	711

9.3.6.1	Periodic Reporting	711

9.3.6.2	Event-triggered Periodic Reporting	711

9.3.6.3	Event-triggered Reporting	711

9.3.7	Void	712

9.3.8	Inter-frequency SFTD measurement requirements	712

9.3.8.1	Introduction	712

9.3.8.2	SFTD Measurement delay	712

9.3.8.3	SFTD Measurement reporting delay	713

9.3.9	Inter frequency measurements without measurement gaps	713

9.3.9.1	Inter frequency Cell identification	713

9.3.9.2	Measurement period	716

9.3.9.3	Scheduling availability of UE during inter-frequency measurements when the SSB is completely contained in the active BWP of the UE	717

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	717

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	718

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	718

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	719

9.3.9.4	Scheduling availability of UE during inter-frequency measurements when the SSB is not completely contained in the active BWP of the UE	719

9.3.9.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	719

9.3.9.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	720

9.3.9.4.3	Scheduling availability of UE performing measurements on FR2	720

9.3.9.4.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	722

9.3.10	Inter-frequency measurement with NCSG	722

9.3.10.1	Inter-frequency cell identification	722

9.3.10.2	Measurement period	724

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	724

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	724

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	725

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	725

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	727

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	727

9.3A.1	Introduction	727

9.3A.2	Requirements applicability	728

9.3A.3	Number of cells and number of SSB	728

9.3A.3.1	Requirements for FR1	728

9.3A.3.2	Requirements for FR2-2	729

9.3A.4	Inter-frequency cell identification	729

9.3A.5	Inter-frequency measurements	731

9.3A.6	NR Inter-frequency measurements reporting requirements	733

9.3A.6.1	Periodic Reporting	733

9.3A.6.2	Event-triggered Periodic Reporting	733

9.3A.6.3	Event-triggered Reporting	733

9.3A.8	Inter-frequency RSSI measurements	733

9.3A.9	Inter-frequency channel occupancy measurements	734

9.3B	NR inter-frequency measurements for RedCap	735

9.3B.1	Introduction	735

9.3B.2	Requirements applicability	735

9.3B.3	Number of cells and number of SSB	736

9.3B.3.1	Requirements for FR1	736

9.3B.3.2	Requirements for FR2	736

9.3B.4	Inter-frequency measurement with measurement gaps	736

9.3B.5	Inter-frequency measurements	737

9.3B.6	Inter-frequency measurements reporting requirements	738

9.3B.6.1	Periodic Reporting	738

9.3B.6.2	Event-triggered Periodic Reporting	738

9.3B.6.3	Event-triggered Reporting	738

9.3B.7	Inter frequency measurements without measurement gaps	739

9.3B.7.1	Inter frequency Cell identification	739

9.3B.7.2	Measurement period	741

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	742

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	742

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	743

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	743

9.3B.7.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	743

9.3C	NR inter-frequency measurements for SAN	744

9.3C.1	Introduction	744

9.3C.2	Requirements applicability	744

9.3C.3	Number of cells and number of SSB	744

9.3C.3.1	Requirements for FR1	744

9.3C.4	Inter-frequency measurement with measurement gaps	745

9.3C.5	Inter-frequency measurements	746

9.3C.6	Inter-frequency measurements reporting requirements	746

9.3C.6.1	Periodic Reporting	746

9.3C.6.2	Event-triggered Periodic Reporting	746

9.3C.6.3	Event-triggered Reporting	747

9.3C.7	Inter frequency measurements without measurement gaps	747

9.3C.7.1	Inter frequency Cell identification	747

9.3C.7.2	Measurement period	749

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	749

9.3C.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	749

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	750

9.4	Inter-RAT measurements	750

9.4.1	Introduction	750

9.4.2	NR − E-UTRAN FDD measurements	752

9.4.2.1	Introduction	752

9.4.2.2	Requirements when no DRX is used	752

9.4.2.3	Requirements when DRX is used	753

9.4.2.4	Measurement reporting requirements	755

9.4.2.4.1	Periodic Reporting	755

9.4.2.4.2	Event-Triggered Periodic Reporting	755

9.4.2.4.3	Event-Triggered Reporting	756

9.4.2.5	Scheduling Availability During NR − E-UTRAN FDD measurements with NCSG	756

9.4.3	NR − E-UTRAN TDD measurements	756

9.4.3.1	Introduction	756

9.4.3.2	Requirements when no DRX is used	756

9.4.3.3	Requirements when DRX is used	758

9.4.3.4	Measurement reporting requirements	759

9.4.3.4.1	Periodic Reporting	759

9.4.3.4.2	Event-Triggered Periodic Reporting	760

9.4.3.4.3	Event-Triggered Reporting	760

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	760

9.4.4	Inter-RAT RSTD measurements	760

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	760

9.4.4.1.1	Introduction	760

9.4.4.1.2	Requirements	761

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	764

9.4.4.2.1	Introduction	764

9.4.4.2.2	Requirements	765

9.4.5	Inter-RAT E-CID measurements	768

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	768

9.4.5.1.1	Introduction	768

9.4.5.1.2	Requirements	768

9.4.5.1.3	Measurement Reporting Delay	768

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	769

9.4.5.2.1	Introduction	769

9.4.5.2.2	Requirements	769

9.4.5.2.3	Measurement Reporting Delay	769

9.4.6	NR − UTRAN FDD measurements	769

9.4.6.1	Introduction	769

9.4.6.2	Requirements when no DRX is used	769

9.4.6.3	Requirements when DRX is used	770

9.4.7	NR – E-UTRAN measurements with autonomous gaps	772

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	772

9.4.7.2	CGI reporting delay	772

9.4A	Inter-RAT measurements for RedCap	773

9.4A.1	Introduction	773

9.4A.2	NR − E-UTRAN FDD measurements	774

9.4A.2.1	Introduction	774

9.4A.2.2	Requirements when no DRX is used	774

9.4A.2.3	Requirements when DRX is used	775

9.4A.2.4	Measurement reporting requirements	776

9.4A.2.4.1	Periodic Reporting	776

9.4A.2.4.2	Event-Triggered Periodic Reporting	776

9.4A.2.4.3	Event-Triggered Reporting	776

9.4A.3	NR − E-UTRAN TDD measurements	777

9.4A.3.1	Introduction	777

9.4A.3.2	Requirements when no DRX is used	777

9.4A.3.3	Requirements when DRX is used	779

9.4A.3.4	Measurement reporting requirements	780

9.4A.3.4.1	Periodic Reporting	780

9.4A.3.4.2	Event-Triggered Periodic Reporting	780

9.4A.3.4.3	Event-Triggered Reporting	780

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	781

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	781

9.4A.4.2	CGI reporting delay	781

9.4A.4.3	CGI reporting scheduling restriction	781

9.5	L1-RSRP measurements for Reporting	782

9.5.1	Introduction	782

9.5.2	Requirements applicability	782

9.5.3	Measurement Reporting Requirements	783

9.5.3.1	Periodic Reporting	783

9.5.3.2	Semi-Persistent Reporting	783

9.5.3.3	Aperiodic Reporting	783

9.5.4	L1-RSRP measurement requirements	784

9.5.4.1	SSB based L1-RSRP Reporting	784

9.5.4.2	CSI-RS based L1-RSRP Reporting	787

9.5.4A	Void	790

9.5.4A.1	Void	790

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	790

9.5.5.1	Measurement restriction for SSB based L1-RSRP	790

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	791

9.5.6	Scheduling availability of UE during L1-RSRP measurement	792

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	792

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	792

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	792

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	794

9.5A	L1-RSRP measurements for Reporting under CCA	794

9.5A.1	Introduction	794

9.5A.2	Requirements applicability	794

9.5A.3	Measurement Reporting Requirements	794

9.5A.3.1	Periodic Reporting	795

9.5A.3.2	Semi-Persistent Reporting	795

9.5A.3.3	Aperiodic Reporting	795

9.5A.4	L1-RSRP measurement requirements	795

9.5A.4.1	SSB based L1-RSRP Reporting	795

9.5A.5	Measurement restriction for L1-RSRP measurement	798

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	798

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	798

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	798

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	799

9.5A.6.3	(Void)	799

9.5A.6.3A	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	799

9.5A.6.3B	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	799

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	800

9.5B	L1-RSRP measurements for Reporting for RedCap	800

9.5B.1	Introduction	800

9.5B.2	Requirements applicability	800

9.5B.3	Measurement Reporting Requirements	801

9.5B.3.1	Periodic Reporting	801

9.5B.3.2	Semi-Persistent Reporting	801

9.5B.3.3	Aperiodic Reporting	801

9.5B.4	L1-RSRP measurement requirements	802

9.5B.4.1	SSB based L1-RSRP Reporting	802

9.5B.4.2	CSI-RS based L1-RSRP Reporting	803

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	806

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	806

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	807

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	807

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	807

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	807

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	808

9.5C	L1-RSRP measurements for Reporting for satellite access	808

9.5C.1	Introduction	808

9.5C.3	Measurement Reporting Requirements	809

9.5C.3.1	Periodic Reporting	809

9.5C.3.2	Semi-Persistent Reporting	809

9.5C.3.3	Aperiodic Reporting	810

9.5C.4	L1-RSRP measurement requirements	810

9.5C.4.1	SSB based L1-RSRP Reporting	810

9.5C.5	Measurement restriction for L1-RSRP measurement	812

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	812

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	812

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	813

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	813

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	813

9.6	NE-DC: Measurements	813

9.6.1	Introduction	813

9.6.2	SFTD Measurements	813

9.6.2.1	Introduction	813

9.6.2.2	SFTD Measurement requirements	813

9.7	Cross Link Interference measurements	814

9.7.1	Introduction	814

9.7.2	SRS-RSRP measurements	814

9.7.2.1	Introduction	814

9.7.2.2	Requirements applicability	815

9.7.2.3	Measurement Reporting Requirements	815

9.7.2.3.1	Periodic Reporting	815

9.7.2.3.2	Event-triggered Periodic Reporting	815

9.7.2.3.3	Event Triggered Reporting	815

9.7.2.4	Measurement capability	815

9.7.2.5	SRS-RSRP measurement period	816

9.7.3	CLI-RSSI measurements	816

9.7.3.1	Introduction	816

9.7.3.2	Requirements applicability	816

9.7.3.3	Measurement Reporting Requirements	816

9.7.3.3.1	Periodic Reporting	816

9.7.3.3.2	Event-triggered Periodic Reporting	816

9.7.3.3.3	Event Triggered Reporting	817

9.7.3.4	Measurement capability	817

9.7.3.5	CLI-RSSI measurement period	817

9.7.4	Scheduling availability of UE during CLI measurements	817

9.7.4.1	Scheduling availability of UE performing measurement on FR1	817

9.7.4.2	Scheduling availability of UE performing measurement on FR2	818

9.8	L1-SINR measurements for Reporting	818

9.8.1	Introduction	818

9.8.2	Requirements applicability	819

9.8.3	Measurement Reporting Requirements	819

9.8.3.1	Periodic Reporting	819

9.8.3.2	Semi-Persistent Reporting	820

9.8.4	L1-SINR measurement requirements	820

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	820

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	825

9.8.5	Measurement restriction for L1-SINR measurement	826

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	827

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	827

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	828

9.8.6	Scheduling availability of UE during L1-SINR measurement	828

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	828

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	828

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	829

9.9	 NR measurements for positioning	830

9.9.1	Introduction	830

9.9.1.1	General Aspects of Gap-based Measurement	830

9.9.1.2	General Aspects of Gapless Measurement	830

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	831

9.9.2	RSTD measurements	832

9.9.2.1	Introduction	832

9.9.2.2	Requirements Applicability	832

9.9.2.3	Measurement Capability	832

9.9.2.4	Measurement Reporting Requirements	832

9.9.2.4.1	Void	833

9.9.2.4.2	Void	833

9.9.2.4.3	Void	833

9.9.2.5	Measurements Period Requirements	833

9.9.2.6	Void	836

9.9.2.7	Measurements Period Requirements without Measurement Gaps	836

9.9.2.8	Void	839

9.9.2.9	Measurements Period Requirements with both MG and PPW	839

9.9.3	PRS-RSRP measurements	839

9.9.3.1	Introduction	839

9.9.3.2	Requirements applicability	839

9.9.3.3	Measurement Capability	840

9.9.3.4	Measurement Reporting Requirements	840

9.9.3.5	Measurement Period Requirements	840

9.9.3.6	Measurement Period Requirements without Measurement Gaps	842

9.9.3.7	Void	845

9.9.3.8	Measurements Period Requirements with both MG and PPW	845

9.9.4	UE Rx-Tx time difference measurements	846

9.9.4.1 Introduction	846

9.9.4.2 Requirements Applicability	846

9.9.4.3 Measurement Capability	846

9.9.4.4 Measurement Reporting Requirements	846

9.9.4.5	Measurement Period Requirements	846

9.9.4.6	Measurement Period Requirements without Measurement Gaps	850

9.9.4.7	Void	853

9.9.4.8	Measurements Period Requirements with both MG and PPW	853

9.9.5	E-CID measurements	854

9.9.5.1	Introduction	854

9.9.5.2	Measurement Requirements	854

9.9.5.2.1	Intra-frequency Measurement Requirements	854

9.9.5.2.2	Inter-frequency Measurement Requirements	854

9.9.5.2.3	Measurement Reporting Delay	855

9.9.6	PRS-RSRPP measurements	855

9.9.6.1	Introduction	855

9.9.6.2	Requirements applicability	855

9.9.6.3	Measurement capability	855

9.9.6.4	Measurement reporting requirements	855

9.9.6.5	Measurement period requirements	855

9.9.6.6	Measurement Period Requirements without Measurement Gaps	856

9.9.6.7	Void	856

9.9.6.8	Measurements Period Requirements with both MG and PPW	856

9.10	CSI-RS based L3 measurements	856

9.10.1	Introduction	856

9.10.2	CSI-RS based intra-frequency measurements	856

9.10.2.1	Introduction	856

9.10.2.2	Requirements applicability	857

9.10.2.3	Number of cells and number of CSI-RS	858

9.10.2.3.1	Requirements for FR1	858

9.10.2.3.2	Requirements for FR2	858

9.10.2.4	Measurement Reporting Requirements	858

9.10.2.4.1	Periodic Reporting	858

9.10.2.4.2	Event-triggered Periodic Reporting	859

9.10.2.4.3	Event Triggered Reporting	859

9.10.2.5	Intra-frequency measurements without measurement gaps	859

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	860

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	861

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	861

9.10.3	CSI-RS based Inter-frequency measurements	861

9.10.3.1	Introduction	861

9.10.3.2	Requirements applicability	861

9.10.3.3	Number of cells and number of CSI-RS resources	862

9.10.3.3.1	Requirements for FR1	862

9.10.3.3.2	Requirements for FR2	862

9.10.3.4	Measurements reporting requirements	862

9.10.3.4.1	Periodic Reporting	863

9.10.3.4.2	Event-triggered Periodic Reporting	863

9.10.3.4.3	Event-triggered Reporting	863

9.10.3.5	Inter frequency measurements with measurement gaps	863

9.11	NR measurements with autonomous gaps	865

9.11.1	Introduction	865

9.11.2	CGI identification of an NR cell with autonomous gaps	865

9.11.3	CGI reporting delay	866

9.11A	NR measurements with autonomous gaps for RedCap	866

9.11A.1	Introduction	866

9.11A.2	CGI identification of an NR cell with autonomous gaps	866

9.11A.3	CGI reporting delay	867

9.11A.4	CGI reporting scheduling restriction	868

9.12	Measurement for Propagation Delay Compensation	868

9.12.1	Introduction	868

9.12.2	Requirements Applicability	868

9.12.3	Measurement Capability	869

9.12.4	Measurement period requirements	869

9.12.4.1	PRS Measurement Period	869

9.12.4.2	TRS Measurement Period	870

9.12.5	Measurement Reporting Requirements	871

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	871

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	871

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	871

9.13.1	Introduction	871

9.13.2	Requirements Applicability	872

9.13.3	Measurement Reporting Requirements	872

9.13.3.1	Periodic Reporting	872

9.13.3.2	Semi-Persistent Reporting	873

9.13.3.3	Aperiodic Reporting	873

9.13.4	L1-RSRP measurement requirements	873

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	873

9.13.5	Measurement restriction for L1-RSRP measurement	875

9.13.5.1	Measurement restriction for SSB based L1-RSRP	875

9.13.6	Scheduling availability of UE during L1-RSRP measurement	876

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	876

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	876

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	877

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	877

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	877

10	Measurement Performance requirements	879

10.1	NR measurements	879

10.1.1	Introduction	879

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	879

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	879

10.1.2.1.1	Absolute SS-RSRP Accuracy	879

10.1.2.1.2	Relative SS-RSRP Accuracy	880

10.1.2.2	Void	881

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	881

10.1.2.3.1	Absolute CSI-RSRP Accuracy	881

10.1.2.3.2	Relative CSI-RSRP Accuracy	882

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	883

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	883

10.1.2B.1.1	Absolute SS-RSRP Accuracy	883

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	884

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	884

10.1.2C.1.1	Absolute SS-RSRP Accuracy	884

10.1.2C.1.2	Relative SS-RSRP Accuracy	885

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	885

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	885

10.1.3.1.1	Absolute SS-RSRP Accuracy	885

10.1.3.1.2	Relative SS-RSRP Accuracy	886

10.1.3.2	Void	887

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	887

10.1.3.3.1	Absolute CSI-RSRP Accuracy	887

10.1.3.3.2	Relative CSI-RSRP Accuracy	888

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	889

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	889

10.1.3B.1.1	Absolute SS-RSRP Accuracy	889

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	890

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	890

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	890

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	891

10.1.4.2	Void	892

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	892

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	892

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	893

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	894

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	894

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	894

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	895

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	895

10.1.4C.1.1	Absolute Accuracy of SS-RSRP in FR1	895

10.1.4C.1.2	Relative Accuracy of SS-RSRP in FR1	896

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	896

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	896

10.1.5.1.1	Absolute SS-RSRP Accuracy	896

10.1.5.1.2	Relative SS-RSRP Accuracy	897

10.1.5.2	Void	898

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	898

10.1.5.3.1	Absolute CSI-RSRP Accuracy	898

10.1.5.3.2	Relative CSI-RSRP Accuracy	899

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	900

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	900

10.1.5B.1.1	Absolute SS-RSRP Accuracy	900

10.1.6	RSRP Measurement Report Mapping	901

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	903

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	903

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	903

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	904

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	904

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	905

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	905

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	905

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	906

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	906

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	906

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	907

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	907

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	907

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	908

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	908

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	908

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	908

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	909

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	909

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	909

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	909

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	910

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	911

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	911

10.1.9.2.2	Relative CSI-RSRQ Accuracy	912

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	913

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	913

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	913

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	914

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	914

10.1.9C.1.1	Absolute Accuracy of SS-RSRQ in FR1	914

10.1.9C.1.2	Relative Accuracy of SS-RSRQ in FR1	915

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	916

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	916

10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2	916

10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2	916

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	917

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	917

10.1.10.2.2	Relative CSI-RSRQ Accuracy	918

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	919

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	919

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	919

10.1.11	RSRQ report mapping	920

10.1.12	Intra-frequency SINR accuracy requirements for FR1	920

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	920

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	920

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	921

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	921

10.1.12C	Intra-frequency SINR accuracy requirements for FR1 SAN	922

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	922

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	922

10.1.13	Intra-frequency SINR accuracy requirements for FR2	923

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	923

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	923

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	924

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	924

10.1.14	Inter-frequency SINR accuracy requirements for FR1	924

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	924

10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1	924

10.1.14.1.2	Relative Accuracy of SS-SINR in FR1	925

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	926

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	926

10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1	927

10.1.14C	Inter-frequency SINR accuracy requirements for FR1 SAN	928

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	928

10.1.14C.1.1	Aboslute Accuracy of SS-SINR in FR1	928

10.1.14C.1.2	Relative Accuracy of SS-SINR in FR1	929

10.1.15	Inter-frequency SINR accuracy requirements for FR2	930

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	930

10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2	930

10.1.15.1.2	Relative Accuracy of SS-SINR in FR2	930

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	931

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	931

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	932

10.1.16	SINR report mapping	933

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	933

10.1.17	Power Headroom	934

10.1.17.1	Power Headroom Report	934

10.1.17.1.1	Power Headroom Report Mapping	934

10.1.18	PCMAX,c,f	935

10.1.18.1	Report Mapping	935

10.1.19	L1-RSRP accuracy requirements for FR1	935

10.1.19.1	SSB based L1-RSRP accuracy requirements	935

10.1.19.1.1	Absolute Accuracy	935

10.1.19.1.2	Relative Accuracy	936

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	937

10.1.19.2.1	Absolute Accuracy	937

10.1.19.2.2	Relative Accuracy	938

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	939

10.1.19C.1	SSB based L1-RSRP accuracy requirements	939

10.1.19C.1.1	Absolute Accuracy	939

10.1.19C.1.2	Relative Accuracy	940

10.1.20	L1-RSRP accuracy requirements for FR2	940

10.1.20.1	SSB based L1-RSRP accuracy requirements	940

10.1.20.1.1	Absolute Accuracy	940

10.1.20.1.2	Relative Accuracy	941

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	942

10.1.20.2.1	Absolute Accuracy	942

10.1.20.2.2	Relative Accuracy	943

10.1.21	SFTD accuracy requirements	944

10.1.21.1	SFTD acuracy requirements for NE-DC	944

10.1.21.2	SFTD acuracy requirements for NR-DC	946

10.1.21.3	Inter frequency SFTD acuracy requirements	947

10.1.22	CLI measurement accuracy requirements	948

10.1.22.1	SRS-RSRP	948

10.1.22.1.1	SRS-RSRP Accuracy	948

10.1.22.1.2	SRS-RSRP report mapping	950

10.1.22.2	CLI-RSSI	950

10.1.22.2.1	CLI-RSSI Accuracy	950

10.1.22.2.2	CLI-RSSI report mapping	951

10.1.23	RSTD Measurements	952

10.1.23.1	Introduction	952

10.1.23.2	Measurement Accuracy Requirements	952

10.1.23.3	Report mapping	962

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	962

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	964

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	966

10.1.24	PRS-RSRP Measurements	968

10.1.24.1	Introduction	968

10.1.24.2	Measurement Accuracy Requirements	968

10.1.24.2.1	Absolute PRS RSRP accuracy	968

10.1.24.2.2	Relative PRS RSRP accuracy	972

10.1.24.3	Report mapping	976

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	976

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	977

10.1.25	UE Rx-Tx Time Difference Measurements	979

10.1.25.1	Introduction	979

10.1.25.2	Measurement Accuracy Requirements	979

10.1.25.3	Report mapping	992

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	992

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	994

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	995

10.1.26	FR2 P-MPR report	997

10.1.26.1	Report mapping	997

10.1.27	L1-SINR accuracy requirements for FR1	997

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	997

10.1.27.1.1	Absolute Accuracy	997

10.1.27.1.2	Relative Accuracy	998

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	999

10.1.27.2.1	Absolute Accuracy	999

10.1.27.2.2	Relative Accuracy	1000

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1002

10.1.27.3.1	Absolute Accuracy	1002

10.1.27.3.2	Relative Accuracy	1004

10.1.28	L1-SINR accuracy requirements for FR2	1005

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	1012

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1012

10.1.29.1.1	Absolute SS-RSRQ Accuracy	1012

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	1012

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1012

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	1012

10.1.30.1.2	Relative Accuracy of SS-RSRQ	1013

10.1.31	Intra-frequency SINR accuracy requirements under CCA	1014

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	1014

10.1.31.1.1	Absolute SS-SINR Accuracy	1014

10.1.32	Inter-frequency SINR accuracy requirements under CCA	1014

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	1014

10.1.32.1.1	Aboslute Accuracy of SS-SINR	1014

10.1.32.1.2	Relative Accuracy of SS-SINR	1015

10.1.33	L1-RSRP accuracy requirements under CCA	1016

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	1016

10.1.33.1.1	Absolute Accuracy	1016

10.1.33.1.2	Relative Accuracy	1016

10.1.34	RSSI measurements under CCA	1017

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	1017

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	1017

10.1.34.3	RSSI measurement report mapping	1017

10.1.35	Channel occupancy measurements under CCA	1018

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	1018

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	1018

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	1018

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	1018

10.1.36.1.1	Absolute SS-RSRP Accuracy	1018

10.1.36.1.2	Relative SS-RSRP Accuracy	1019

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	1020

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	1020

10.1.37.1.1	Absolute Accuracy of SS-RSRP	1020

10.1.37.1.2	Relative Accuracy of SS-RSRP	1020

10.1.38	PRS-RSRPP Measurements	1021

10.1.38.1	Introduction	1021

10.1.38.2	Measurement Accuracy Requirements	1021

10.1.38.2.1	Absolute PRS RSRPP accuracy	1021

10.1.38.3	Report mapping	1025

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1025

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1026

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	1027

10.1.39.1	 Void	1027

10.1.39.2	 Measurement Accuracy Requirements for PRS	1027

10.1.39.3	 Measurement Accuracy Requirements for TRS	1031

10.1.40	Void	1034

10.1A	NR measurements for RedCap	1034

10.1A.1	Introduction	1034

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	1034

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	1034

10.1A.2.1.1	Absolute SS-RSRP Accuracy	1034

10.1A.2.1.2	Relative SS-RSRP Accuracy	1035

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	1036

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	1036

10.1A.3.1.1	Absolute SS-RSRP Accuracy	1036

10.1A.3.1.2	Relative SS-RSRP Accuracy	1036

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	1036

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	1036

10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1	1036

10.1A.4.1.2	Relative Accuracy of SS-RSRP in FR1	1037

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	1038

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	1038

10.1A.5.1.1	Absolute SS-RSRP Accuracy	1038

10.1A.5.1.2	Relative SS-RSRP Accuracy	1038

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	1038

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1038

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	1038

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	1039

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1039

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	1039

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	1039

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1039

10.1A.8.1.1	Absolute Accuracy of SS-RSRQ in FR1	1039

10.1A.8.1.2	Relative Accuracy of SS-RSRQ in FR1	1040

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	1041

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1041

10.1A.9.1.1	Absolute Accuracy of SS-RSRQ in FR2	1041

10.1A.9.1.2	Relative Accuracy of SS-RSRQ in FR2	1041

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	1041

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	1041

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	1041

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	1042

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	1042

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	1042

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	1042

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	1042

10.1A.12.1.1	Aboslute Accuracy of SS-SINR in FR1	1042

10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1	1043

10.1A.13	 Inter-frequency SINR accuracy requirements for FR2	1044

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	1044

10.1A.13.1.1	Aboslute Accuracy of SS-SINR in FR2	1044

10.1A.13.1.2	Relative Accuracy of SS-SINR in FR2	1044

10.1A.14	 L1-RSRP accuracy requirements for FR1	1044

10.1A.14.1	SSB based L1-RSRP accuracy requirements	1044

10.1A.14.1.1	Absolute Accuracy	1044

10.1A.14.1.2	Relative Accuracy	1045

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	1046

10.1A.14.2.1	Absolute Accuracy	1046

10.1A.14.2.2	Relative Accuracy	1047

10.1A.15	 L1-RSRP accuracy requirements for FR2	1048

10.1A.15.1	SSB based L1-RSRP accuracy requirements	1048

10.1A.15.1.1	Absolute Accuracy	1048

10.1A.15.1.2	Relative Accuracy	1048

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	1048

10.1A.15.2.1	Absolute Accuracy	1048

10.1A.15.2.2	Relative Accuracy	1048

10.2	E-UTRAN measurements	1049

10.2.1	Introduction	1049

10.2.2	E-UTRAN RSRP measurements	1049

10.2.3	E-UTRAN RSRQ measurements	1049

10.2.4	E-UTRAN RSTD measurements	1049

10.2.5	E-UTRAN RS-SINR measurements	1050

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	1050

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	1050

10.2A	E-UTRAN measurements for RedCap	1051

10.2A.1	Introduction	1051

10.2A.2	E-UTRAN RSRP measurements	1051

10.2A.3	E-UTRAN RSRQ measurements	1051

10.2A.4	E-UTRAN RS-SINR measurements	1052

10.3	UTRAN FDD Measurements	1052

10.3.1	UTRAN FDD CPICH RSCP	1052

10.3.2	UTRAN FDD CPICH Ec/No	1053

10.4	V2X measurements	1053

10.4.1	Introduction	1053

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	1053

10.4.2.1	PSBCH-RSRP Absolute Accuracy	1053

10.4.2.2	PSBCH-RSRP Relative Accuracy	1054

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	1055

10.4.3.1	Absolute SL-RSSI Accuracy	1055

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	1055

10.4.4.1	Absolute L1 SL-RSRP Accuracy	1055

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	1056

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	1056

11	Void	1057

12	V2X Requirements	1058

12.1	Introduction	1058

12.2	UE Transmit Timing	1058

12.2.1	Introduction	1058

12.2.2	GNSS as synchronization reference source	1058

12.2.3	NR Cell as synchronization reference source	1059

12.2.4	E-URTAN Cell as synchronization reference source	1059

12.2.5	SyncRef UE as synchronization reference source	1060

12.3	Initiation/Cease of SLSS Transmissions	1060

12.3.1	Introduction	1060

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1060

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1061

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1062

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1062

12.4	Selection / Reselection of V2X Synchronization Reference Source	1063

12.5	L1 SL-RSRP measurements	1064

12.5.1	Introduction	1064

12.5.2	SL-RSRP measurements	1064

12.6	Congestion Control measurements	1065

12.7	Interruption	1065

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	1065

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	1066

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	1067

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	1068

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	1068

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	1069

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	1069

12.7.8	Interruptions at NR sidelink discovery configuration	1069

12.8	Reliability of GNSS signal	1070

12.9	Scheduling availability	1070

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	1070

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	1070

12.10	Selection / Reselection of relay UE	1071

12.10.1	Introduction	1071

12.10.2	Selection / Reselection of relay UE	1071

13	Measurement Performance Requirements for NR gNB	1071

13.1	UL-RTOA	1071

13.1.1	Report mapping	1071

13.1.1A	Additional Path Report Mapping for UL-RTOA	1073

13.2	gNB Rx-Tx time difference	1075

13.2.1	Report mapping	1075

13.2.1A	 Additional Path Report Mapping for gNB Rx-Tx	1077

13.2.2	Measurement Accuracy Requirements	1079

13.2.2.1	Introduction	1079

13.3	UL SRS RSRP measurement	1080

13.3.1	Report mapping	1080

13.3.2	Measurement accuracy requirements	1081

13.3.2.1	Introduction	1081

13.3.2.2	Requirements	1081

13.4	AoA/ZoA	1082

13.4.1	Report mapping	1082

13.5	Timing advance (TADV)	1083

13.5.1	Report mapping	1083

13.6	UL SRS RSRPP measurement	1084

13.6.1	Report mapping	1084

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	1086

13.7.1	Report mapping	1086

13.7.2	Measurement Accuracy Requirements	1086

13.7.2.1	Introduction	1086

13.7.2.2	Requirements	1086

Annex A (normative): Test Cases	1088

A.1	Purpose of annex	1088

A.2	Requirement classification for statistical testing	1088

A.2.1	Types of requirements in TS 38.133	1088

A.2.1.1	Time and delay requirements on UE higher layer actions	1088

A.2.1.2	Measurements of power levels, relative powers and time	1088

A.2.1.3	Implementation requirements	1089

A.2.1.4	Physical layer timing requirements	1089

A.2.1.5	Requirements under CCA	1089

A.3	RRM test configurations	1090

A.3.1	Reference measurement channels	1090

A.3.1.1	PDSCH	1090

A.3.1.1.1	FDD	1090

A.3.1.1.2	TDD	1091

A.3.1.2	CORESET for RMSI scheduling	1094

A.3.1.2.1	FDD	1094

A.3.1.2.2	TDD	1095

A.3.1.3	CORESET for RMC scheduling	1098

A.3.1.3.1	FDD	1098

A.3.1.3.2	TDD	1099

A.3.1.4	TDD UL/DL configuration	1102

A.3.1A	Reference measurement channels under CCA	1104

A.3.1A.1	PDSCH	1104

A.3.1A.1.1	TDD	1104

A.3.1A.2	CORESET for RMSI scheduling	1105

A.3.1A.2.1	TDD	1105

A.3.1A.3	CORESET for RMC scheduling	1106

A.3.1A.3.1	TDD	1106

A.3.1A.4	TDD UL/DL configuration	1107

A.3.1A.5	RMC burst transmission model	1107

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	1107

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	1108

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	1108

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	1109

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	1109

A.3.2.2	Void	1110

A.3.3	Reference DRX configurations	1110

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	1110

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	1111

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	1111

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	1111

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	1112

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	1112

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	1112

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	1113

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	1113

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	1113

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	1114

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	1114

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	1114

A.3.3.14	DRX Configuration 14: DRX cycle = 160 ms and TAT = Infinity	1115

A.3.4	Test Cases with Different Channel Bandwidths	1115

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	1115

A.3.4.1.1	Introduction	1115

A.3.4.1.2	Principle of testing	1115

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	1115

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	1115

A.3.5.1.1	Introduction	1115

A.3.5.1.2	Principle of Testing	1115

A.3.6	Antenna configurations	1116

A.3.6.1	Antenna configurations for FR1	1116

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	1116

A.3.6.1.1.1	Introduction	1116

A.3.6.1.1.2	Principle of testing	1116

A.3.6.2	Antenna configurations for FR2	1119

A.3.6A	Antenna configurations with unlicensed bands	1119

A.3.6A.1	Antenna configurations for FR1	1119

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	1119

A.3.6A.1.1.1	Introduction	1119

A.3.6A.1.1.2	Principle of testing	1119

A.3.7	EN-DC test setup	1121

A.3.7.1	Introduction	1121

A.3.7.2	E-UTRAN Serving Cell Parameters	1121

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1121

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1122

A.3.7A	NR FR1-FR2 test setup	1123

A.3.7B	EN-DC test setup with unlicensed bands	1124

A.3.7B.1	Introduction	1124

A.3.7B.2	E-UTRAN Serving Cell Parameters	1124

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1124

A.3.7C	LTE-FR1/FR2 test setup	1125

A.3.7D	NE-DC test setup	1125

A.3.7D.1	Introduction	1125

A.3.7D.2	E-UTRAN Serving Cell Parameters	1125

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1125

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1125

A.3.8	PRACH configurations	1125

A.3.8.1	Introduction	1125

A.3.8.2	PRACH configurations in FR1	1126

A.3.8.2.1	FR1 PRACH configuration 1	1126

A.3.8.2.2	FR1 PRACH configuration 2	1126

A.3.8.2.3	FR1 PRACH configuration 3	1127

A.3.8.2.4	FR1 PRACH configuration 4	1128

A.3.8.3	PRACH configurations in FR2	1129

A.3.8.3.1	FR2 PRACH configuration 1	1129

A.3.8.3.2	FR2 PRACH configuration 2	1130

A.3.8.3.3	FR2 PRACH configuration 3	1131

A.3.8.3.4	FR2 PRACH configuration 4	1132

A.3.8A	PRACH configurations under CCA	1133

A.3.8A.1	Introduction	1133

A.3.8A.2	PRACH configurations in FR1	1133

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1133

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1134

A.3.9	BWP configurations	1135

A.3.9.1	Introduction	1135

A.3.9.2	Downlink BWP configurations	1136

A.3.9.2.1	Initial BWP	1136

A.3.9.2.2	Dedicated BWP	1136

A.3.9.3	Uplink BWP configurations	1137

A.3.9.3.1	Initial BWP	1137

A.3.9.3.2	Dedicated BWP	1137

A.3.9A	BWP configurations for RedCap	1137

A.3.9A.1	Introduction	1137

A.3.9A.2	Downlink BWP configurations	1138

A.3.9A.2.1	Dedicated BWP	1138

A.3.9A.3	Uplink BWP configurations	1138

A.3.9A.3.1	Dedicated BWP	1138

A.3.10	SSB Configurations	1139

A.3.10.1	SSB Configurations for FR1	1139

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1139

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1141

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1141

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1142

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1142

A.3.10.2	SSB Configurations for FR2	1143

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1143

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1143

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1144

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1144

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1145

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1145

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1146

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1146

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1147

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1147

A.3.10A	SSB Configurations under CCA	1152

A.3.10A.1	SSB Configurations under CCA for FR1	1152

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1152

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1152

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1153

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1153

A.3.10B	SSB Configurations for RedCap	1154

A.3.10B.1	SSB Configurations for FR1	1154

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1154

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1154

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1155

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1155

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1156

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1156

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1157

A.3.10B.2	SSB Configurations for FR2	1157

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1157

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1158

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1158

A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1159

A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1159

A.3.11	SMTC Configurations	1160

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1160

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1160

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1160

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1160

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1161

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1161

A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms	1161

A.3.11.8	SMTC pattern 8: SMTC period = 10 ms with SMTC duration = 1 ms	1161

A.3.11.9	SMTC pattern 9: SMTC period = 20 ms with SMTC duration = 1 ms	1161

A.3.11A	SMTC Configurations for RedCap	1162

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1162

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1162

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1162

A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms	1162

A.3.12	Test Cases with Different CC Configurations	1162

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1162

A.3.12.1.1	Introduction	1162

A.3.12.1.2	Principle of testing	1163

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1163

A.3.12.2.1	Introduction	1163

A.3.12.2.2	Principle of testing	1163

A.3.13	Test Cases in SA and EN-DC Operations	1163

A.3.13.1	Introduction	1163

A.3.13.2	Principle of Testing	1163

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	1164

A.3.13A.1	Introduction	1164

A.3.13A.2	Principle of Testing in EN-DC	1164

A.3.13A.3	Principle of Testing in SA	1164

A.3.13A.4	Principle of Testing in E-UTRA	1165

A.3.13B	Test Cases for EN-DC and NE-DC Operations	1166

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1166

A.3.13B.1.1	Introduction	1166

A.3.13B.1.2	Principle of Testing	1166

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1166

A.3.13B.2.1	Introduction	1166

A.3.13B.2.2	Principle of Testing	1166

A.3.14	CSI-RS configurations	1167

A.3.14.1	FDD	1167

A.3.14.2	TDD	1169

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1175

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1175

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1175

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1175

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1176

A.3.15.3	Setup 3: 2 AoAs	1176

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1176

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1176

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1176

A.3.16	TCI State Configuration	1177

A.3.16.1	Introduction	1177

A.3.16.2	TCI states	1177

A.3.16A	Unified TCI State Configuration	1177

A.3.16A.1	Introduction	1177

A.3.16A.2	DLorJoint TCI states	1178

A.3.16A.3	UL TCI states	1178

A.3.17	Configurations of CSI-RS for tracking	1179

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1179

A.3.17.1.1	FDD	1179

A.3.17.1.2	TDD	1181

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1183

A.3.17.2.1	TDD	1183

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1184

A.3.18.1	Introduction	1184

A.3.18.2	PRACH Power Measurement	1184

A.3.19	Test applicability for DAPS handover	1184

A.3.19.1	Introduction	1184

A.3.19.2	Principle of testing	1184

A.3.20	MsgA configurations	1185

A.3.20.1	Introduction	1185

A.3.20.2	MsgA configurations in FR1	1185

A.3.20.2.1	FR1 MsgA configuration 1	1185

A.3.20.2.2	FR1 MsgA configuration 2	1186

A.3.20.3	MsgA configurations in FR2	1188

A.3.20.3.1	FR2 MsgA configuration 1	1188

A.3.20.3.2	FR2 MsgA configuration 2	1189

A.3.20A	MsgA configurations under CCA	1191

A.3.20A.1	Introduction	1191

A.3.20A.2	MsgA configurations in FR1	1191

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1191

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1192

A.3.21	V2X sidelink communication	1195

A.3.21.1	Introduction	1195

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1195

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1199

A.3.21.4	Reference SL-DRX configurations	1200

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1200

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1200

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1200

A.3.22	CSI-IM configurations	1200

A.3.22.1	FDD	1200

A.3.22.2	TDD	1201

A.3.23	Spatial Relation Configuration	1202

A.3.23.1	Introduction	1202

A.3.23.2	Spatial Relation	1203

A.3.24	SRS configuration	1204

A.3.25	Channel bandwidth (CBW) configurations	1206

A.3.25.1	DL UE specific CBW	1206

A.3.25.2	UL UE specific CBW	1207

A.3.26	CCA model	1207

A.3.26.1	Introduction	1207

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1207

A.3.26.2.1	DL CCA model	1207

A.3.26.2.2	UL CCA model	1209

A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2	1209

A.3.26.3.1	DL CCA model	1209

A.3.26.3.2	UL CCA model	1210

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	1210

A.3.27.1	Introduction	1210

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	1210

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	1211

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	1211

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	1211

A.3.28	Discovery Burst Transmission Window configuration under CCA	1211

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1211

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1211

A.3.29.1	Introduction	1211

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1211

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1212

A.3.30	CSI-RS configurations for RRM	1213

A.3.30.1	FDD	1213

A.3.30.2	TDD	1214

A.3.31	PRS Configurations	1216

A.3.31.1.	PRS Configurations for FR1	1216

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	1216

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	1217

A.3.31.2.	PRS Configurations for FR2	1217

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	1217

A.3.32	NR sidelink discovery	1217

A.3.32.1	Introduction	1217

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1217

A.3.32.3	Principle of Testing	1218

A.3.33	PRS Processing Window (PPW) configurations	1218

A.3.34	Testing principles for test cases related to PRS measurements	1218

A.3.34.1	Introduction	1218

A.3.34.2	Test cases in RRC_INACTIVE state	1219

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1219

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1219

A.3.35	Testing principle for RedCap UE	1220

A.3.35.1	Introduction	1220

A.3.35.2	Principle of testing for FR1	1220

A.3.35.3	Principle of testing for FR2	1220

A.3.36	Testing related to Satellite access	1220

A.3.36.1	Introduction	1220

A.3.36.2	Principle of testing GSO and NGSO scenarios	1220

A.3.36.2	Principle of testing different RRM requirements	1221

A.3.36.3	Principle of testing different ephemeris formats	1221

A.3.36.4	General setup for SIB19	1223

A.3.36.5	Satellite specific parameters configuration	1224

A.3.36.5.1	Satellite specific configuration for serving cell	1224

A.3.36.5.2	Satellite specific configuration for neighbour cell	1225

A.4	EN-DC tests with all NR cells in FR1	1225

A.4.1	Void	1225

A.4.2	Void	1225

A.4.3	RRC_CONNECTED state mobility	1225

A.4.3.1	Void	1225

A.4.3.2	RRC Connection Mobility Control	1225

A.4.3.2.1	Void	1225

A.4.3.2.2	Random Access	1225

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1225

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1229

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1233

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1237

A.4.3.2.3	Void	1241

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1241

A.4.3.3.1	Test Purpose and Environment	1241

A.4.3.3.2	Test Requirements	1247

A.4.4	Timing	1248

A.4.4.1	UE transmit timing	1248

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1248

A.4.4.1.1.1	Test Purpose and environment	1248

A.4.4.1.1.2	Test requirements	1252

A.4.4.2	UE timer accuracy	1252

A.4.4.3	Timing advance	1252

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1252

A.4.4.3.1.1	Test Purpose and Environment	1252

A.4.4.3.1.2	Test Parameters	1252

A.4.4.3.1.3	Test Requirements	1256

A.4.5	Signaling characteristics	1256

A.4.5.1	Radio link Monitoring	1256

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1257

A.4.5.1.1.1	Test Purpose and Environment	1257

A.4.5.1.1.2	Test Requirements	1261

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1261

A.4.5.1.2.1	Test Purpose and Environment	1261

A.4.5.1.2.2	Test Requirements	1267

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1267

A.4.5.1.3.1	Test Purpose and Environment	1267

A.4.5.1.3.2	Test Requirements	1272

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1272

A.4.5.1.4.1	Test Purpose and Environment	1272

A.4.5.1.4.2	Test Requirements	1278

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1278

A.4.5.1.5.1	Test Purpose and Environment	1278

A.4.5.1.5.2	Test Requirements	1283

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1283

A.4.5.1.6.1	Test Purpose and Environment	1283

A.4.5.1.6.2	Test Requirements	1289

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1289

A.4.5.1.7.1	Test Purpose and Environment	1289

A.4.5.1.7.2	Test Requirements	1294

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1295

A.4.5.1.8.1	Test Purpose and Environment	1295

A.4.5.1.8.2	Test Requirements	1300

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1300

A.4.5.1.9.1	Test Purpose and Environment	1300

A.4.5.2	Interruption	1306

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1306

A.4.5.2.1.1	Test Purpose and Environment	1306

A.4.5.2.1.2	Test Requirements	1310

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1310

A.4.5.2.2.1	Test Purpose and Environment	1310

A.4.5.2.2.2	Test Requirements	1314

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1314

A.4.5.2.3.1	Test Purpose and Environment	1314

A.4.5.2.3.2	Test Requirements	1321

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1322

A.4.5.2.4.1	Test Purpose and Environment	1322

A.4.5.2.4.2	Test Requirements	1326

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1327

A.4.5.2.5.1	Test Purpose and Environment	1327

A.4.5.2.5.2	Test Requirements	1330

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1330

A.4.5.2.6.1	Test Purpose and Environment	1330

A.4.5.2.6.2	Test Requirements	1334

A.4.5.2.7	Void	1334

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1334

A.4.5.2.8.1	Test Purpose and Environment	1334

A.4.5.2.8.2	Test Requirements	1337

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1338

A.4.5.2.9.1	Test Purpose and Environment	1338

A.4.5.2.9.2	Test Requirements	1341

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	1342

A.4.5.2.10.1	Test Purpose and Environment	1342

A.4.5.2.10.2	Test Requirements	1345

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	1345

A.4.5.2.11.1	Test Purpose and Environment	1345

A.4.5.2.11.2	Test Requirements	1350

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	1351

A.4.5.2.12.1	Test Purpose and Environment	1351

A.4.5.3	SCell Activation and Deactivation Delay	1358

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1358

A.4.5.3.1.1	Test Purpose and Environment	1358

A.4.5.3.1.2	Test Requirements	1364

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	1365

A.4.5.3.2.1	Test Purpose and Environment	1365

A.4.5.3.2.2	Test Requirements	1365

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1365

A.4.5.3.3.1	Test Purpose and Environment	1365

A.4.5.3.3.2	Test Requirements	1366

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1367

A.4.5.3.4.1	Test Purpose and Environment	1367

A.4.5.3.3.2	Test Requirements	1369

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1369

A.4.5.3.5.1	Test Purpose and Environment	1369

A.4.5.3.5.2	Test Requirements	1377

A.4.5.3.6	Fast SCell Activation of known SCell in FR1 for 160ms SCell measurement cycle	1378

A.4.5.3.6.1	Test Purpose and Environment	1378

A.4.5.3.6.2	Test Requirements	1382

A.4.5.3.7	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	1383

A.4.5.3.7.1	Test Purpose and Environment	1383

A.4.5.3.7.2	Test Requirements	1383

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1384

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1384

A.4.5.4.1.1	Test Purpose and Environment	1384

A.4.5.4.1.2	Test Requirements	1392

A.4.5.5	Beam Failure Detection and Link recovery procedures	1392

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1392

A.4.5.5.1.1	Test Purpose and Environment	1392

A.4.5.5.1.2	Test Requirements	1398

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1399

A.4.5.5.2.1	Test Purpose and Environment	1399

A.4.5.5.2.2	Test Requirements	1404

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1405

A.4.5.5.3.1	Test Purpose and Environment	1405

A.4.5.5.3.2	Test Requirements	1410

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1411

A.4.5.5.4.1	Test Purpose and Environment	1411

A.4.5.5.4.2	Test Requirements	1416

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1417

A.4.5.5.5.1	Test Purpose and Environment	1417

A.4.5.5.5.2	Test Requirements	1422

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1423

A.4.5.5.6.1	Test Purpose and Environment	1423

A.4.5.5.6.2	Test Requirements	1429

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1430

A.4.5.5.7.1	Test Purpose and Environment	1430

A.4.5.5.7.2	Test Requirements	1435

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1436

A.4.5.5.8.1	Test Purpose and Environment	1436

A.4.5.5.8.2	Test Requirements	1442

A.4.5.6	Active BWP switch	1443

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1443

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1443

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1448

A.4.5.6.2	RRC-based Active BWP Switch	1454

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1458

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1458

A.4.5.6.3.1.1	Test Purpose and Environment	1458

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1464

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1464

A.4.5.6.4.1.1	Test Purpose and Environment	1464

A.4.5.6.4.1.2	Test Requirements	1471

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1471

A.4.5.6.4.2.1	Test Purpose and Environment	1471

A.4.5.6.4.2.2	Test Requirements	1481

A.4.5.6.5	SCell dormancy switch	1482

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1482

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1487

A.4.5.6.5.2.1	Test Purpose and Environment	1487

A.4.5.6.5.2.2	Test Requirements	1493

A.4.5.7	PSCell addition and release delay	1493

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1493

A.4.5.7.1.1	Test purpose and environment	1493

A.4.5.7.1.2	Test Requirements	1498

A.4.5.8	DL Interruptions at switching between two uplink carriers	1499

A.4.5.8.1	Test Purpose and Environment	1499

A.4.5.8.2	Test Requirements	1502

A.4.5.9	UE specific CBW change	1503

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	1503

A.4.5.9.1.1	Test Purpose and Environment	1503

A.4.5.9.1.2	Test Requirements	1507

A.4.5.10	PSCell activation and deactivation delay	1507

A.4.5.10.1	PSCell activation and deactivation delay	1507

A.4.5.10.1.1	Test purpose and environment	1507

A.4.5.10.1.2	Test Requirements	1511

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)	1512

A.4.5.11.1	Conditional PSCell Addition and Release Delay	1512

A.4.5.11.1.1	Test purpose and environment	1512

A.4.5.11.1.2	Test Parameters	1512

A.4.5.11.1.3	Test Requirements	1516

A.4.6	Measurement procedure	1516

A.4.6.1	Intra-frequency Measurements	1516

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1516

A.4.6.1.1.1	Test purpose and Environment	1516

A.4.6.1.1.2	Test parameters	1516

A.4.6.1.1.3	Test Requirements	1520

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1520

A.4.6.1.2.1	Test purpose and Environment	1520

A.4.6.1.2.2	Test parameters	1520

A.4.6.1.2.2	Test Requirements	1524

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1524

A.4.6.1.3.1	Test purpose and Environment	1524

A.4.6.1.3.2	Test parameters	1524

A.4.6.1.3.3	Test Requirements	1528

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1528

A.4.6.1.4.1	Test purpose and Environment	1528

A.4.6.1.4.2	Test parameters	1528

A.4.6.1.4.3	Test Requirements	1532

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1532

A.4.6.1.5.1	Test purpose and Environment	1532

A.4.6.1.5.2	Test parameters	1532

A.4.6.1.5.3	Test Requirements	1534

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1535

A.4.6.1.6.1	Test purpose and Environment	1535

A.4.6.1.6.2	Test parameters	1535

A.4.6.1.6.3	Test Requirements	1537

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1538

A.4.6.1.7.1	Test purpose and Environment	1538

A.4.6.1.7.2	Test parameters	1538

A.4.6.1.7.3	Test Requirements	1542

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	1542

A.4.6.1.8.1	Test Purpose and Environment	1542

A.4.6.1.8.2	Test Requirements	1547

A.4.6.2	Inter-frequency Measurements	1548

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1548

A.4.6.2.1.1	Test Purpose and Environment	1548

A.4.6.2.1.2	Test Requirements	1552

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1552

A.4.6.2.2.1	Test Purpose and Environment	1552

A.4.6.2.2.2	Test Requirements	1556

A.4.6.2.3	Void	1556

A.4.6.2.4	Void	1556

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1556

A.4.6.2.5.1	Test Purpose and Environment	1556

A.4.6.2.5.2	Test Requirements	1561

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1561

A.4.6.2.6.1	Test Purpose and Environment	1561

A.4.6.2.6.2	Test Requirements	1566

A.4.6.2.7	Void	1566

A.4.6.2.8	Void	1566

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	1567

A.4.6.2.9.1	Test Purpose and Environment	1567

A.4.6.2.9.2	Test Requirements	1570

A.4.6.3	Void	1570

A.4.6.4	L1-RSRP measurement for beam reporting	1570

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1570

A.4.6.4.1.1	Test Purpose and Environment	1570

A.4.6.4.1.2	Test parameters	1571

A.4.6.4.1.3	Test Requirements	1574

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	1574

A.4.6.4.2.1	Test Purpose and Environment	1574

A.4.6.4.2.2	Test parameters	1575

A.4.6.4.2.3	Test Requirements	1578

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1578

A.4.6.4.3.1	Test Purpose and Environment	1578

A.4.6.4.3.2	Test parameters	1579

A.4.6.4.3.3	Test Requirements	1582

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1582

A.4.6.4.4.1	Test Purpose and Environment	1582

A.4.6.4.4.2	Test parameters	1583

A.4.6.4.4.3	Test Requirements	1585

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1585

A.4.6.4.5.1	Test Purpose and Environment	1585

A.4.6.4.5.2	Test parameters	1586

A.4.6.4.5.3	Test Requirements	1589

A.4.6.5	CLI measurements	1589

A.4.6.5.1	SRS-RSRP measurement with non-DRX	1589

A.4.6.5.1.1	Test Purpose and Environment	1589

A.4.6.5.1.2	Test Parameters	1590

A.4.6.5.1.3	Test Requirements	1593

A.4.6.5.2	CLI-RSSI measurement with non-DRX	1593

A.4.6.5.2.1	Test Purpose and Environment	1593

A.4.6.5.2.2	Test Parameters	1594

A.4.6.5.2.3	Test Requirements	1595

A.4.6.6.1.2	Test Requirements	1600

A.4.6.7	L1-SINR measurement for beam reporting	1600

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1603

A.4.6.7.2.1	Test Purpose and Environment	1603

A.4.6.7.2.2	Test parameters	1603

A.4.6.7.2.3	Test Requirements	1606

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1606

A.4.6.7.3.1	Test Purpose and Environment	1606

A.4.6.7.3.2	Test parameters	1606

A.4.6.7.3.3	Test Requirements	1608

A.4.6.8	CSI-RS based intra-frequency Measurement	1609

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	1609

A.4.6.8.1.1	Test purpose and Environment	1609

A.4.6.8.1.2	Test Requirements	1613

A.4.6.9	CSI-RS based inter-frequency Measurement	1613

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	1613

A.4.6.9.1.1	Test Purpose and Environment	1613

A.4.6.9.1.2	Test Requirements	1617

A.4.7	Measurement Performance requirements	1618

A.4.7.1	SS-RSRP	1618

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1618

A.4.7.1.1.1	Test Purpose and Environment	1618

A.4.7.1.1.2	Test parameters	1618

A.4.7.1.1.3	Test Requirements	1622

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1622

A.4.7.1.2.1	Test Purpose and Environment	1622

A.4.7.1.2.2	Test parameters	1623

A.4.7.1.2.3	Test Requirements	1627

A.4.7.1.3	Void	1627

A.4.7.2	SS-RSRQ	1627

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1627

A.4.7.2.1.1	Test Purpose and Environment	1627

A.4.7.2.1.2	Test Parameters	1628

A.4.7.2.1.3	Test Requirements	1632

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1632

A.4.7.2.2.1	Test Purpose and Environment	1632

A.4.7.2.2.2	Test Parameters	1632

A.4.7.2.2.3	Test Requirements	1637

A.4.7.3	SS-SINR	1637

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1637

A.4.7.3.1.1	Test Purpose and Environment	1637

A.4.7.3.1.2	Test Parameters	1637

A.4.7.3.1.3	Test Requirements	1640

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1640

A.4.7.3.2.1	Test Purpose and Environment	1640

A.4.7.3.2.2	Test Parameters	1640

A.4.7.3.2.3	Test Requirements	1645

A.4.7.4	L1-RSRP measurement for beam reporting	1645

A.4.7.4.1	SSB based L1-RSRP measurement	1645

A.4.7.4.1.1	Test Purpose and Environment	1645

A.4.7.4.1.2	Test parameters	1646

A.4.7.4.1.3	Test Requirements	1649

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1649

A.4.7.4.2.1	Test Purpose and Environment	1649

A.4.7.4.2.2	Test parameters	1650

A.4.7.4.2.3	Test Requirements	1653

A.4.7.5	SFTD accuracy	1653

A.4.7.5.1	SFTD accuracy	1653

A.4.7.5.1.1	Test Purpose and Environment	1653

A.4.7.5.1.2	Test Parameters	1653

A.4.7.5.1.3	Test Requirements	1658

A.4.7.5.2	Void	1658

A.4.7.5.3	Void	1658

A.4.7.6	CLI measurements	1658

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1658

A.4.7.6.1.1	Test Purpose and Environment	1658

A.4.7.6.1.2	Test parameters	1659

A.4.7.6.1.3	Test Requirements	1664

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1664

A.4.7.6.2.1	Test Purpose and Environment	1664

A.4.7.6.2.2	Test parameters	1665

A.4.7.6.2.3	Test Requirements	1668

A.4.7.7	L1-SINR measurement for beam reporting	1668

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1672

A.4.7.7.2.1	Test Purpose and Environment	1672

A.4.7.7.2.2	Test parameters	1672

A.4.7.7.2.3	Test Requirements	1676

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1676

A.4.7.7.3.1	Test Purpose and Environment	1676

A.4.7.7.3.2	Test parameters	1676

A.4.7.7.3.3	Test Requirements	1679

A.4.7.8	CSI-RSRP	1679

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1679

A.4.7.8.1.1	Test Purpose and Environment	1679

A.4.7.8.1.2	Test parameters	1680

A.4.7.8.1.3	Test Requirements	1685

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1685

A.4.7.8.2.1	Test Purpose and Environment	1685

A.4.7.8.2.2	Test parameters	1685

A.4.7.8.2.3	Test Requirements	1690

A.4.7.9	CSI-RSRQ	1690

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1690

A.4.7.9.1.1	Test Purpose and Environment	1690

A.4.7.9.1.2	Test Parameters	1690

A.4.7.9.1.3	Test Requirements	1695

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1695

A.4.7.9.2.1	Test Purpose and Environment	1695

A.4.7.9.2.2	Test Parameters	1695

A.4.7.9.2.3	Test Requirements	1700

A.4.7.10	CSI-SINR	1700

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1700

A.4.7.10.1.1	Test Purpose and Environment	1700

A.4.7.10.1.2	Test Parameters	1701

A.4.7.10.1.3	Test Requirements	1705

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1705

A.4.7.10.2.1	Test Purpose and Environment	1705

A.4.7.10.2.2	Test Parameters	1706

A.4.7.10.2.3	Test Requirements	1710

A.4.8	Void	1710

A.4A	NE-DC test with all NR cells in FR1	1711

A.4A.1	Signaling characteristics	1711

A.4A.1.1	E-UTRAN PSCell addition	1711

A.4A.1.1.1	Test purpose and environment	1711

A.4A.1.1.2	Test Requirements	1715

A.4A.1.2	Active BWP switch	1716

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1716

A.4A.1.2.1.1	Test Purpose and Environment	1716

A.4A.1.2.1.2	Test Requirements	1720

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	1721

A.4A.1.3.1	Test purpose and environment	1721

A.4A.1.3.2	Test Requirements	1725

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	1726

A.4A.1.4.1	Test Purpose and Environment	1726

A.4A.1.4.2	Test Parameters	1726

A.4A.1.4.3	Test Requirements	1731

A.4A.1.4.3.1	Test Requirements for NR HO	1731

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	1731

A.4A.2	Measurement performance	1732

A.4A.2.1	SFTD accuracy	1732

A.4A.2.1.1	SFTD accuracy	1732

A.4A.2.1.1.1	Test Purpose	1732

A.4A.2.1.1.2	Test Environment	1732

A.4A.2.1.1.3	Test Requirements	1735

A.5	EN-DC tests with one or more NR cells in FR2	1736

A.5.1	Void	1736

A.5.2	Void	1736

A.5.3	RRC_CONNECTED state mobility	1736

A.5.3.1	Void	1736

A.5.3.2	RRC Connection Mobility Control	1736

A.5.3.2.1	Void	1736

A.5.3.2.2	Random Access	1736

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	1736

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1739

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1745

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1748

A.5.3.2.3	Void	1752

A.5.3.3	Handover with PSCell with known FR2 target PSCell	1752

A.5.3.3.1	Test purpose and environment	1752

A.5.3.3.2	Test Requirements	1757

A.5.5.3.3	void	1758

A.5.5.3.4	void	1758

A.5.5.3.5	void	1758

A.5.5.3.6	void	1758

A.5.4	Timing	1758

A.5.4.1	UE transmit timing	1758

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1758

A.5.4.1.1.1	Test Purpose and environment	1758

A.5.4.1.1.2	Test requirements	1761

A.5.4.2	UE timer accuracy	1762

A.5.4.3	Timing advance	1762

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	1762

A.5.4.3.1.1 Test Purpose and Environment	1762

A.5.4.3.1.2 Test Parameters	1762

A.5.4.3.1.3	Test Requirements	1766

A.5.5	Signaling characteristics	1766

A.5.5.1	Radio link Monitoring	1766

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1766

A.5.5.1.1.1	Test Purpose and Environment	1766

A.5.5.1.1.2	Test Requirements	1770

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1771

A.5.5.1.2.1	Test Purpose and Environment	1771

A.5.5.1.2.2	Test Requirements	1774

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1775

A.5.5.1.3.1	Test Purpose and Environment	1775

A.5.5.1.3.2	Test Requirements	1779

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1779

A.5.5.1.4.1	Test Purpose and Environment	1779

A.5.5.1.4.2	Test Requirements	1783

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1783

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1787

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1791

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1796

A.5.5.1.8.2	Test Requirements	1800

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1801

A.5.5.1.9.1	Test Purpose and Environment	1801

A.5.5.1.9.2	Test Requirements	1803

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1803

A.5.5.1.10.1	Test Purpose and Environment	1803

A.5.5.1.10.2	Test Requirements	1806

A.5.5.2	Interruption	1806

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1806

A.5.5.2.1.1	Test Purpose and Environment	1806

A.5.5.2.1.2	Test Requirements	1809

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1809

A.5.5.2.2.1	Test Purpose and Environment	1809

A.5.5.2.2.2	Test Requirements	1812

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1812

A.5.5.2.3.1	Test Purpose and Environment	1812

A.5.5.2.3.2	Test Requirements	1816

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1816

A.5.5.2.4.1	Test Purpose and Environment	1816

A.5.5.2.4.2	Test Requirements	1819

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1820

A.5.5.2.5.1	Test Purpose and Environment	1820

A.5.5.2.5.2	Test Requirements	1823

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1823

A.5.5.2.6.1	Test Purpose and Environment	1823

A.5.5.2.6.2	Test Requirements	1826

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1826

A.5.5.2.7.1	Test Purpose and Environment	1826

A.5.5.2.7.2	Test Requirements	1830

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1830

A.5.5.2.8.1 Test Purpose and Environment	1830

A.5.5.2.8.3	Test Requirements	1832

A.5.5.3	SCell Activation and Deactivation Delay	1832

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1832

A.5.5.3.1.1	Test Purpose and Environment	1832

A.5.5.3.1.2	Test Requirements	1834

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1834

A.5.5.3.2.1	Test Purpose and Environment	1834

A.5.5.3.2.2	Test Requirements	1838

A.5.5.3.3	Void	1838

A.5.5.3.4	Void	1838

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1838

A.5.5.3.5.1	Test Purpose and Environment	1838

A.5.5.3.5.2	Test Requirements	1841

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1842

A.5.5.3.6.1	Test Purpose and Environment	1842

A.5.5.3.6.2	Test Requirements	1846

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1847

A.5.5.3.7.1	Test Purpose and Environment	1847

A.5.5.3.7.2	Test Requirements	1850

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	1850

A.5.5.3.8.1	Test Purpose and Environment	1850

A.5.5.3.8.2	Test Requirements	1854

A.5.5.3.9	PUCCH SCell Activation and deactivation of known SCell in FR2	1854

A.5.5.3.9.1	Test Purpose and Environment	1854

A.5.5.3.9.2	Test Requirements	1858

A.5.5.3.10	PUCCH SCell Activation and deactivation of unknown SCell in FR2	1858

A.5.5.3.10.1	Test Purpose and Environment	1858

A.5.5.3.10.2	Test Requirements	1861

A.5.5.3.11	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	1862

A.5.5.3.11.1	Test Purpose and Environment	1862

A.5.5.3.11.2	Test Requirements	1865

A.5.5.3.12	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	1866

A.5.5.3.12.1	Test Purpose and Environment	1866

A.5.5.3.12.2	Test Requirements	1869

A.5.5.4	Void	1870

A.5.5.5	Beam Failure Detection and Link recovery procedures	1870

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1870

A.5.5.5.1.1	Test Purpose and Environment	1870

A.5.5.5.1.2	Test Requirements	1874

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1875

A.5.5.5.2.1	Test Purpose and Environment	1875

A.5.5.5.2.2	Test Requirements	1879

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1880

A.5.5.5.3.1	Test Purpose and Environment	1880

A.5.5.5.3.2	Test Requirements	1884

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1885

A.5.5.5.4.1	Test Purpose and Environment	1885

A.5.5.5.4.2	Test Requirements	1889

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1890

A.5.5.5.5.1	Test Purpose and Environment	1890

A.5.5.5.5.2	Test Requirements	1894

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1895

A.5.5.5.6.1	Test Purpose and Environment	1895

A.5.5.5.6.2	Test Requirements	1899

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1900

A.5.5.5.7.1	Test Purpose and Environment	1900

A.5.5.5.7.2	Test Requirements	1904

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1905

A.5.5.5.8.1	Test Purpose and Environment	1905

A.5.5.5.8.2	Test Requirements	1910

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	1910

A.5.5.5.9.1	Test Purpose and Environment	1910

A.5.5.5.9.2	Test Requirements	1914

A.5.5.6	Active BWP switch	1914

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1914

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1914

A.5.5.6.1.1.1	Test Purpose and Environment	1914

A.5.5.6.1.1.2	Test Requirements	1918

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1918

A.5.5.6.2	RRC-based Active BWP Switch	1923

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1923

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1927

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1927

A.5.5.6.4	SCell dormancy switch	1931

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1931

A.5.5.6.4.1.1	Test Purpose and Environment	1931

A.5.5.6.4.1.2	Test Requirements	1935

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1935

A.5.5.6.4.2.1	Test Purpose and Environment	1935

A.5.5.6.4.2.2	Test Requirements	1942

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1942

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1942

A.5.5.7	PSCell addition and release delay	1945

A.5.5.7.1	Addition and Release Delay of NR PSCell	1945

A.5.5.7.1.1	Test purpose and environment	1945

A.5.5.7.1.2	Test Requirements	1948

A.5.5.8	Active TCI state switch delay	1949

A.5.5.8.1	MAC-CE based active TCI state switch	1949

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1949

A.5.5.8.1.1.1	Test Purpose and Environment	1949

A.5.5.8.1.1.2	Test Requirements	1953

A.5.5.8.2	RRC based active TCI state switch	1953

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1953

A.5.5.8.2.1.1	Test Purpose and Environment	1953

A.5.5.8.2.1.2	Test Requirements	1957

A.5.5.9	Uplink spatial relation switch delay	1957

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1957

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1957

A.5.5.9.1.1.1	Test Purpose and Environment	1957

A.5.5.9.1.1.2	Test Requirements	1960

A.5.5.9.2	RRC based spatial relation switch	1960

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1960

A.5.5.9.2.1.1	Test Purpose and Environment	1960

A.5.5.9.2.1.2	Test Requirements	1963

A.5.5.10	UE specific CBW change	1963

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1963

A.5.5.10.1.1	Test Purpose and Environment	1963

A.5.5.10.1.2	Test Requirements	1966

A.5.5.11	Unified TCI state switch delay	1967

A.5.5.11.1	MAC-CE based active joint TCI state switch	1967

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	1967

A.5.5.11.1.1.1	Test Purpose and Environment	1967

A.5.5.11.1.1.2	Test parameters	1967

A.5.5.11.1.1.3	Test Requirements	1970

A.5.5.11.2	MAC-CE based active uplink TCI state switch	1970

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	1970

A.5.5.11.2.1.1	Test Purpose and Environment	1970

A.5.5.11.2.1.2	Test parameters	1971

A.5.5.11.2.1.3	Test Requirements	1973

A.5.5.11.3	MAC-CE based active downlink TCI state switch	1973

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	1973

A.5.5.11.3.1.1	Test Purpose and Environment	1973

A.5.5.11.3.1.2	Test Parameters	1974

A.5.5.11.3.1.3	Test Requirements	1978

A.5.5.12	PSCell activation and deactivation delay	1978

A.5.5.12.1	PSCell activation and deactivation delay	1978

A.5.5.12.1.1	Test purpose and environment	1978

A.5.5.12.1.2	Test Requirements	1981

A.5.5.13	Conditional PSCell addition and release delay	1982

A.5.5.13.1	Addition and Release Delay of NR PSCell	1982

A.5.5.13.1.1	Test purpose and environment	1982

A.5.5.13.1.2	Test Requirements	1985

A.5.5.13.2	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR PSCell	1986

A.5.5.13.2.1	Test Purpose and Environment	1986

A.5.5.13.2.2	Test Requirements	1989

A.5.6	Measurement procedure	1989

A.5.6.1	Intra-frequency Measurements	1989

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1989

A.5.6.1.1.1	Test purpose and Environment	1989

A.5.6.1.1.2	Test Requirements	1993

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1993

A.5.6.1.2.1	Test purpose and Environment	1993

A.5.6.1.2.2	Test Requirements	1995

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1996

A.5.6.1.3.1	Test purpose and Environment	1996

A.5.6.1.3.2	Test Requirements	2000

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	2000

A.5.6.1.4.1	Test purpose and Environment	2000

A.5.6.1.4.2	Test Requirements	2003

A.5.6.2	Inter-frequency Measurements	2004

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2004

A.5.6.2.1.1	Test Purpose and Environment	2004

A.5.6.2.1.2	Test Requirements	2007

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2007

A.5.6.2.2.1	Test Purpose and Environment	2007

A.5.6.2.2.2	Test Requirements	2011

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2011

A.5.6.2.3.1	Test Purpose and Environment	2011

A.5.6.2.3.2	Test Requirements	2015

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2015

A.5.6.2.4.1	Test Purpose and Environment	2015

A.5.6.2.4.2	Test Requirements	2019

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2019

A.5.6.2.5.1	Test Purpose and Environment	2019

A.5.6.2.5.2	Test Requirements	2024

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2024

A.5.6.2.6.1	Test Purpose and Environment	2024

A.5.6.2.6.2	Test Requirements	2028

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2028

A.5.6.2.7.1	Test Purpose and Environment	2028

A.5.6.2.7.2	Test Requirements	2033

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2033

A.5.6.2.8.1	Test Purpose and Environment	2033

A.5.6.2.8.2	Test Requirements	2038

A.5.6.3	L1-RSRP measurement for beam reporting	2039

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2039

A.5.6.3.1.1	Test Purpose and Environment	2039

A.5.6.3.1.2	Test parameters	2039

A.5.6.3.1.3	Test Requirements	2041

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	2041

A.5.6.3.2.1	Test Purpose and Environment	2041

A.5.6.3.2.2	Test parameters	2042

A.5.6.3.2.3	Test Requirements	2044

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	2044

A.5.6.3.3.1	Test Purpose and Environment	2044

A.5.6.3.3.2	Test parameters	2045

A.5.6.3.3.3	Test Requirements	2047

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	2047

A.5.6.3.4.1	Test Purpose and Environment	2047

A.5.6.3.4.2	Test parameters	2048

A.5.6.3.4.3	Test Requirements	2050

A.5.6.4	CLI measurements	2051

A.5.6.4.1	SRS-RSRP measurement with DRX	2051

A.5.6.4.1.1	Test Purpose and Environment	2051

A.5.6.4.1.2	Test Parameters	2051

A.5.6.4.1.3	Test Requirements	2053

A.5.6.4.2	CLI-RSSI measurement with DRX	2053

A.5.6.4.2.1	Test Purpose and Environment	2053

A.5.6.4.2.2	Test Parameters	2054

A.5.6.4.2.3	Test Requirements	2056

A.5.6.5	Measurements with autonomous gaps	2056

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	2056

A.5.6.5.1.1	Test Purpose and Environment	2056

A.5.6.5.1.2	Test Requirements	2059

A.5.6.6	L1-SINR measurement for beam reporting	2060

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2062

A.5.6.6.2.1	Test Purpose and Environment	2062

A.5.6.6.2.2	Test parameters	2063

A.5.6.6.2.3	Test Requirements	2066

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2066

A.5.6.6.3.1	Test Purpose and Environment	2066

A.5.6.6.3.2	Test parameters	2067

A.5.6.6.3.3	Test Requirements	2069

A.5.6.7	CSI-RS based Intra-frequency Measurements	2069

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	2069

A.5.6.7.1.1	Test purpose and Environment	2069

A.5.6.7.1.2	Test Requirements	2072

A.5.6.8	CSI-RS based Inter-frequency Measurements	2072

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	2072

A.5.6.8.1.1	Test Purpose and Environment	2072

A.5.6.8.1.2	Test Requirements	2077

A.5.7	Measurement Performance requirements	2077

A.5.7.1	SS-RSRP	2077

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2077

A.5.7.1.1.1	Test Purpose and Environment	2077

A.5.7.1.1.2	Test parameters	2078

A.5.7.1.1.3	Test Requirements	2080

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2081

A.5.7.1.2.1	Test Purpose and Environment	2081

A.5.7.1.2.2	Test parameters	2081

A.5.7.1.2.3	Test Requirements	2085

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2086

A.5.7.1.3.1	Test Purpose and Environment	2086

A.5.7.1.3.2	Test parameters	2086

A.5.7.1.3.3	Test Requirements	2089

A.5.7.2	SS-RSRQ	2090

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2090

A.5.7.2.1.1	Test Purpose and Environment	2090

A.5.7.2.1.2	Test Parameters	2090

A.5.7.2.1.3	Test Requirements	2093

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2093

A.5.7.2.2.1	Test Purpose and Environment	2093

A.5.7.2.2.2	Test Parameters	2093

A.5.7.2.2.3	Test Requirements	2095

A.5.7.3	SS-SINR	2096

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2096

A.5.7.3.1.1	Test Purpose and Environment	2096

A.5.7.3.1.2	Test Parameters	2096

A.5.7.3.1.3	Test Requirements	2099

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2099

A.5.7.3.2.1	Test Purpose and Environment	2099

A.5.7.3.2.2	Test Parameters	2099

A.5.7.3.2.3	Test Requirements	2101

A.5.7.4	L1-RSRP measurement for beam reporting	2101

A.5.7.4.1	SSB based L1-RSRP measurement	2101

A.5.7.4.1.1	Test Purpose and Environment	2101

A.5.7.4.1.2	Test parameters	2102

A.5.7.4.1.3	Test Requirements	2104

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2105

A.5.7.4.2.1	Test Purpose and Environment	2105

A.5.7.4.2.2	Test parameters	2105

A.5.7.4.2.3	Test Requirements	2107

A.5.7.5	CLI measurements	2108

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	2108

A.5.7.5.1.1	Test Purpose and Environment	2108

A.5.7.5.1.2	Test parameters	2108

A.5.7.5.1.3	Test Requirements	2111

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	2112

A.5.7.5.2.1	Test Purpose and Environment	2112

A.5.7.5.2.2	Test parameters	2112

A.5.7.5.2.3	Test Requirements	2114

A.5.7.6	L1-SINR measurement for beam reporting	2115

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2118

A.5.7.6.2.1	Test Purpose and Environment	2118

A.5.7.6.2.2	Test parameters	2118

A.5.7.6.2.3	Test Requirements	2120

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2121

A.5.7.6.3.1	Test Purpose and Environment	2121

A.5.7.6.3.2	Test parameters	2121

A.5.7.6.3.3	Test Requirements	2123

A.5.7.7	CSI-RSRP	2124

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2124

A.5.7.7.1.1	Test Purpose and Environment	2124

A.5.7.7.1.2	Test parameters	2124

A.5.7.7.1.3	Test Requirements	2126

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2127

A.5.7.7.2.1	Test Purpose and Environment	2127

A.5.7.7.2.2	Test parameters	2127

A.5.7.7.2.3	Test Requirements	2131

A.5.7.8	CSI-RSRQ	2132

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2132

A.5.7.8.1.1	Test Purpose and Environment	2132

A.5.7.8.1.2	Test Parameters	2132

A.5.7.8.1.3	Test Requirements	2134

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2134

A.5.7.8.2.1	Test Purpose and Environment	2134

A.5.7.8.2.2	Test Parameters	2134

A.5.7.8.2.3	Test Requirements	2136

A.5.7.9	CSI-SINR	2136

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2136

A.5.7.9.1.1	Test Purpose and Environment	2136

A.5.7.9.1.2	Test Parameters	2136

A.5.7.9.1.3	Test Requirements	2139

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2139

A.5.7.9.2.1	Test Purpose and Environment	2139

A.5.7.9.2.2	Test Parameters	2139

A.5.7.9.2.3	Test Requirements	2142

A.5.8	Void	2142

A.6	NR standalone tests with all NR cells in FR1	2144

A.6.1	SA: RRC_IDLE state mobility	2144

A.6.1.1	Cell re-selection to NR	2144

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	2144

A.6.1.1.1.1	Test Purpose and Environment	2144

A.6.1.1.1.2	Test Parameters	2144

A.6.1.1.1.3	Test Requirements	2148

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	2148

A.6.1.1.2.1	Test Purpose and Environment	2148

A.6.1.1.2.2	Test Parameters	2148

A.6.1.1.2.3	Test Requirements	2152

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2152

A.6.1.1.3.1	Test Purpose and Environment	2152

A.6.1.1.3.2	Test Parameters	2152

A.6.1.1.3.3	Test Requirements	2157

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2157

A.6.1.1.4.1	Test Purpose and Environment	2157

A.6.1.1.4.2	Test Parameters	2157

A.6.1.1.4.3	Test Requirements	2160

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2160

A.6.1.1.5.1	Test Purpose and Environment	2160

A.6.1.1.5.2	Test Parameters	2160

A.6.1.1.5.3	Test Requirements	2165

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2166

A.6.1.1.6.1	Test Purpose and Environment	2166

A.6.1.1.6.2	Test Parameters	2166

A.6.1.1.6.3	Test Requirements	2170

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2171

A.6.1.1.7.1	Test Purpose and Environment	2171

A.6.1.1.7.2	Test Parameters	2171

A.6.1.1.7.3	Test Requirements	2174

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2174

A.6.1.1.8.1	Test Purpose and Environment	2174

A.6.1.1.8.2	Test Parameters	2174

A.6.1.1.8.3	Test Requirements	2178

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2178

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2178

A.6.1.2.1.1	Test Purpose and Environment	2178

A.6.1.2.1.2	Test Parameters	2178

A.6.1.2.1.3	Test Requirements	2181

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2182

A.6.1.2.2.1	Test Purpose and Environment	2182

A.6.1.2.2.2	Test Parameters	2182

A.6.1.2.2.3	Test Requirements	2185

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2186

A.6.1.2.3.1	Test Purpose and Environment	2186

A.6.1.2.3.2	Test Parameters	2186

A.6.1.2.3.3	Test Requirements	2189

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2190

A.6.1.2.4.1	Test Purpose and Environment	2190

A.6.1.2.4.2	Test Parameters	2190

A.6.1.2.4.3	Test Requirements	2193

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2194

A.6.1.2.5.1	Test Purpose and Environment	2194

A.6.1.2.5.2	Test Parameters	2194

A.6.1.2.5.3	Test Requirements	2198

A.6.1.1.7	Void	2199

A.6.2	SA: RRC_INACTIVE state mobility	2199

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2199

A.6.2.1.1	Test purpose and Environment	2199

A.6.2.1.2	Test Parameters	2201

A.6.2.1.3	Test requirements	2203

A.6.3	RRC_CONNECTED state mobility	2203

A.6.3.1	Handover	2203

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2203

A.6.3.1.1.1	Test Purpose and Environment	2203

A.6.3.1.1.2	Test Parameters	2203

A.6.3.1.1.3 Test Requirements	2207

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2207

A.6.3.1.2.1	Test Purpose and Environment	2207

A.6.3.1.2.2	Test Parameters	2207

A.6.3.1.2.3	Test Requirements	2211

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2211

A.6.3.1.3.1	Test Purpose and Environment	2211

A.6.3.1.3.2	Test Parameters	2211

A.6.3.1.3.3	Test Requirements	2215

A.6.3.1.4	SA NR - E-UTRAN handover	2215

A.6.3.1.4.1	Test Purpose and Environment	2215

A.6.3.1.4.2	Test Requirements	2221

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2221

A.6.3.1.5.1	Test Purpose and Environment	2221

A.6.3.1.5.2	Test Requirements	2227

A.6.3.1.6	 SA NR - UTRAN FDD handover	2227

A.6.3.1.6.1	Test Purpose and Environment	2227

A.6.3.1.6.2	Test Requirements	2231

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2231

A.6.3.1.7.1	Test Purpose and Environment	2231

A.6.3.1.7.2	Test Parameters	2231

A.6.3.1.7.3	Test Requirements	2235

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2236

A.6.3.1.8.1	Test Purpose and Environment	2236

A.6.3.1.8.2	Test Parameters	2236

A.6.3.1.8.3	Test Requirements	2240

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2241

A.6.3.1.9.1	Test Purpose and Environment	2241

A.6.3.1.9.2	Test Parameters	2241

A.6.3.1.9.3	Test Requirements	2245

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2245

A.6.3.1.10.1	Test Purpose and Environment	2245

A.6.3.1.10.2	Test Parameters	2245

A.6.3.1.10.3	Test Requirements	2248

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2248

A.6.3.1.11.1	Test Purpose and Environment	2248

A.6.3.1.11.2	Test Parameters	2248

A.6.3.1.11.3 Test Requirements	2255

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2255

A.6.3.1.12.1	Test Purpose and Environment	2255

A.6.3.1.12.2	Test Parameters	2255

A.6.3.1.12.3 Test Requirements	2263

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2263

A.6.3.1.13.1	Test Purpose and Environment	2263

A.6.3.1.13.2	Test Requirements	2272

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PScell addition	2272

A.6.3.1.14.1	Test Purpose and Environment	2272

A.6.3.1.14.2	Test Requirements	2281

A.6.3.2	RRC Connection Mobility Control	2282

A.6.3.2.1	SA: RRC Re-establishment	2282

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2282

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2286

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2290

A.6.3.2.2	Random Access	2293

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2293

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2298

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2302

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2307

A.6.3.2.3	SA: RRC Connection Release with Redirection	2311

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2311

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2315

A.6.3.3 Conditional handover	2322

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2322

A.6.3.3.1.1	Test Purpose and Environment	2322

A.6.3.3.1.2	Test Parameters	2322

A.6.3.3.1.3 Test Requirements	2326

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2326

A.6.3.3.2.1	Test Purpose and Environment	2326

A.6.3.3.2.2	Test Parameters	2326

A.6.3.3.2.3 Test Requirements	2330

A.6.4	Timing	2330

A.6.4.1	UE transmit timing	2330

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2330

A.6.4.1.1.1	Test Purpose and environment	2330

A.6.4.1.1.2	Test requirements	2334

A.6.4.2	UE timer accuracy	2334

A.6.4.3	Timing advance	2334

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2334

A.6.4.3.1.1	Test Purpose and Environment	2334

A.6.4.3.1.2	Test Parameters	2334

A.6.4.3.1.3	Test Requirements	2338

A.6.5	Signalling characteristics	2338

A.6.5.1	Radio link Monitoring	2338

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2339

A.6.5.1.1.1	Test Purpose and Environment	2339

A.6.5.1.1.2	Test Requirements	2344

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2344

A.6.5.1.2.1	Test Purpose and Environment	2344

A.6.5.1.2.2	Test Requirements	2350

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2350

A.6.5.1.3.1	Test Purpose and Environment	2350

A.6.5.1.3.2	Test Requirements	2356

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2356

A.6.5.1.4.1	Test Purpose and Environment	2356

A.6.5.1.4.2	Test Requirements	2362

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2362

A.6.5.1.5.1	Test Purpose and Environment	2362

A.6.5.1.5.2	Test Requirements	2368

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2368

A.6.5.1.6.1	Test Purpose and Environment	2368

A.6.5.1.6.2	Test Requirements	2373

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2373

A.6.5.1.7.1	Test Purpose and Environment	2373

A.6.5.1.7.2	Test Requirements	2378

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2378

A.6.5.1.8.1	Test Purpose and Environment	2378

A.6.5.1.8.2	Test Requirements	2383

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2383

A.6.5.1.9.1	Test Purpose and Environment	2383

A.6.5.1.9.2	Test Requirements	2389

A.6.5.2	Interruption	2389

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2389

A.6.5.2.1.2	Test Requirements	2393

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2394

A.6.5.2.2.1	Test Purpose and Environment	2394

A.6.5.2.2.2	Test Parameters	2394

A.6.5.2.2.3	Test Requirements	2396

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2397

A.6.5.2.3.1	Test Purpose and Environment	2397

A.6.5.2.3.2	Test Parameters	2397

A.6.5.2.3.3	Test Requirements	2399

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2400

A.6.5.2.4.1	Test Purpose and Environment	2400

A.6.5.2.4.2	Test Parameters	2400

A.6.5.2.4.3	Test Requirements	2402

A.6.5.3	SCell Activation and Deactivation Delay	2403

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2403

A.6.5.3.1.1	Test Purpose and Environment	2403

A.6.5.3.1.2	Test Requirements	2408

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2409

A.6.5.3.2.1	Test Purpose and Environment	2409

A.6.5.3.2.2	Test Requirements	2409

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2409

A.6.5.3.3.1	Test Purpose and Environment	2409

A.6.5.3.3.2	Test Requirements	2410

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2410

A.6.5.3.4.1	Test Purpose and Environment	2410

A.6.5.3.4.2	Test Requirements	2417

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2417

A.6.5.3.5.1	Test Purpose and Environment	2417

A.6.5.3.5.2	Test Requirements	2422

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2422

A.6.5.3.6.1	Test Purpose and Environment	2422

A.6.5.3.6.2	Test Requirements	2426

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2426

A.6.5.3.7.1	Test Purpose and Environment	2426

A.6.5.3.7.2	Test Requirements	2430

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2431

A.6.5.3.8.1	Test Purpose and Environment	2431

A.6.5.3.8.2	Test Requirements	2435

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2436

A.6.5.3.9.1	Test Purpose and Environment	2436

A.6.5.3.9.2	Test Requirements	2440

A.6.5.3.10	Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2441

A.6.5.3.10.1	Test Purpose and Environment	2441

A.6.5.3.10.2	Test Requirements	2444

A.6.5.3.11	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2444

A.6.5.3.11.1	Test Purpose and Environment	2444

A.6.5.3.11.2	Test Requirements	2445

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2445

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2445

A.6.5.4.1.1	Test Purpose and Environment	2445

A.6.5.4.1.2	Test Requirements	2454

A.6.5.4.2	Void	2455

A.6.5.5	Beam Failure Detection and Link recovery procedures	2455

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2455

A.6.5.5.1.1	Test Purpose and Environment	2455

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2461

A.6.5.5.2.1	Test Purpose and Environment	2461

A.6.5.5.2.2	Test Requirements	2467

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2468

A.6.5.5.3.1	Test Purpose and Environment	2468

A.6.5.5.3.2	Test Requirements	2473

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2474

A.6.5.5.4.1	Test Purpose and Environment	2474

A.6.5.5.4.2	Test Requirements	2479

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2480

A.6.5.5.5.1	Test Purpose and Environment	2480

A.6.5.5.5.2	Test Requirements	2484

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2484

A.6.5.5.6.1	Test Purpose and Environment	2484

A.6.5.5.6.2	Test Requirements	2489

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2489

A.6.5.5.7.1	Test Purpose and Environment	2489

A.6.5.5.7.2	Test Requirements	2495

A.6.5.6	Active BWP switch	2496

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2496

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2496

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2502

A.6.5.6.2	RRC-based Active BWP Switch	2507

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2507

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2511

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2511

A.6.5.6.4	SCell dormancy switch	2518

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2518

A.6.5.6.4.1.1	Test Purpose and Environment	2518

A.6.5.6.4.1.2	Test Requirements	2524

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2524

A.6.5.6.4.2.1	 Test Purpose and Environment	2524

A.6.5.6.4.2.2	Test Requirements	2531

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2531

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2531

A.6.5.7	DL interruptions at switching between two uplink carriers	2536

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2536

A.6.5.7.1.1	Test Purpose and Environment	2536

A.6.5.7.1.2	Test Requirements	2540

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2540

A.6.5.7.2.1	Test Purpose and Environment	2540

A.6.5.7.2.2	Test Requirements	2544

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2544

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2544

A.6.5.7A.1.1	Test Purpose and Environment	2544

A.6.5.7A.1.2	Test Requirements	2548

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2548

A.6.5.7A.2.1	Test Purpose and Environment	2548

A.6.5.7A.2.2	Test Requirements	2552

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2552

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2552

A.6.5.7B.1.1	Test Purpose and Environment	2552

A.6.5.7B.1.2	Test Requirements	2556

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2556

A.6.5.7B.2.1	Test Purpose and Environment	2556

A.6.5.7B.2.2	Test Requirements	2560

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2560

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2560

A.6.5.7C.1.1	Test Purpose and Environment	2560

A.6.5.7C.1.2	Test Requirements	2564

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2565

A.6.5.7C.2.1	Test Purpose and Environment	2565

A.6.5.7C.2.2	Test Requirements	2569

A.6.5.8	UE specific CBW change	2570

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2570

A.6.5.8.1.1	Test Purpose and Environment	2570

A.6.5.8.1.2	Test Requirements	2574

A.6.5.9	Pathloss reference signal switching delay	2574

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2574

A.6.5.9.1.1	Test Purpose and Environment	2574

A.6.5.9.1.2	Test Requirements	2577

A.6.6	Measurement procedure	2578

A.6.6.1	Intra-frequency Measurements	2578

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2578

A.6.6.1.1.1	Test purpose and Environment	2578

A.6.6.1.1.2	Test parameters	2578

A.6.6.1.1.3	Test Requirements	2582

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2582

A.6.6.1.2.1	Test purpose and Environment	2582

A.6.6.1.2.2	Test parameters	2582

A.6.6.1.2.3	Test Requirements	2586

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2586

A.6.6.1.3.1	Test purpose and Environment	2586

A.6.6.1.3.2	Test parameters	2586

A.6.6.1.3.3	Test Requirements	2590

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2590

A.6.6.1.4.1	Test purpose and Environment	2590

A.6.6.1.4.2	Test parameters	2590

A.6.6.1.4.3	Test Requirements	2594

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2594

A.6.6.1.5.1	Test purpose and Environment	2594

A.6.6.1.5.2	Test parameters	2594

A.6.6.1.5.3	Test Requirements	2596

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2597

A.6.6.1.6.1	Test purpose and Environment	2597

A.6.6.1.6.2	Test parameters	2597

A.6.6.1.6.3	Test Requirements	2598

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2599

A.6.6.1.7.1	Test purpose and Environment	2599

A.6.6.1.7.2	Test parameters	2599

A.6.6.1.7.3	Test Requirements	2603

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	2603

A.6.6.1.8.1	Test purpose and Environment	2603

A.6.6.1.8.2	Test parameters	2603

A.6.6.1.8.3	Test Requirements	2608

A.6.6.2	Inter-frequency Measurements	2608

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2608

A.6.6.2.1.1	Test Purpose and Environment	2608

A.6.6.2.1.2	Test Requirements	2612

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2612

A.6.6.2.2.1	Test Purpose and Environment	2612

A.6.6.2.2.2	Test Requirements	2616

A.6.6.2.3	Void	2617

A.6.6.2.4	Void	2617

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2617

A.6.6.2.5.1	Test Purpose and Environment	2617

A.6.6.2.5.2	Test Requirements	2621

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2621

A.6.6.2.6.1	Test Purpose and Environment	2621

A.6.6.2.6.2	Test Requirements	2625

A.6.6.2.7	Void	2625

A.6.6.2.8	Void	2625

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2625

A.6.6.2.9.1	Test Purpose and Environment	2625

A.6.6.2.9.2	Test Requirements	2629

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2629

A.6.6.2.10.1	Test Purpose and Environment	2629

A.6.6.2.10.2	Test Requirements	2633

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2637

A.6.6.2.12.1	Test Purpose and Environment	2637

A.6.6.2.12.2	Test Requirements	2641

A.6.6.3	Inter-RAT Measurements	2641

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2641

A.6.6.3.1.1	Test Purpose and Environment	2641

A.6.6.3.1.2	Test Requirements	2647

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2647

A.6.6.3.2.1	Test Purpose and Environment	2647

A.6.6.3.2.2	Test Requirements	2654

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2654

A.6.6.3.3.1	Test Purpose and Environment	2654

A.6.6.3.3.2	Test Requirements	2659

A.6.6.4	L1-RSRP measurement for beam reporting	2659

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2659

A.6.6.4.1.1	Test Purpose and Environment	2659

A.6.6.4.1.2	Test parameters	2660

A.6.6.4.1.3	Test Requirements	2663

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2663

A.6.6.4.2.1	Test Purpose and Environment	2663

A.6.6.4.2.2	Test parameters	2664

A.6.6.4.2.3	Test Requirements	2667

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2667

A.6.6.4.3.1	Test Purpose and Environment	2667

A.6.6.4.3.2	Test parameters	2668

A.6.6.4.3.3	Test Requirements	2671

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2671

A.6.6.4.4.1	Test Purpose and Environment	2671

A.6.6.4.4.2	Test parameters	2672

A.6.6.4.4.3	Test Requirements	2675

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2675

A.6.6.4.5.1	Test Purpose and Environment	2675

A.6.6.4.5.2	Test parameters	2676

A.6.6.4.5.3	Test Requirements	2679

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	2679

A.6.6.4.6.1	Test Purpose and Environment	2679

A.6.6.4.6.2	Test parameters	2680

A.6.6.4.6.3	Test Requirements	2683

A.6.6.5	Inter-RAT UTRAN FDD measurements	2684

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2684

A.6.6.5.1.1	Test Purpose and Environment	2684

A.6.6.5.1.2	Test Requirements	2687

A.6.6.6	CLI measurements	2687

A.6.6.6.1	SRS-RSRP measurement with DRX	2687

A.6.6.6.1.1	Test Purpose and Environment	2687

A.6.6.6.1.2	Test Parameters	2688

A.6.6.6.1.3	Test Requirements	2691

A.6.6.6.2	CLI-RSSI measurement with DRX	2691

A.6.6.6.2.1	Test Purpose and Environment	2691

A.6.6.6.2.2	Test Parameters	2692

A.6.6.6.2.3	Test Requirements	2694

A.6.6.7	NR measurements with autonomous gaps	2694

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2694

A.6.6.7.1.1	Test Purpose and Environment	2694

A.6.6.7.1.2	Test Parameters	2694

A.6.6.7.1.3	Test Requirements	2697

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2697

A.6.6.7.2.1	Test Purpose and Environment	2697

A.6.6.7.2.2	Test Requirements	2700

A.6.6.8	L1-SINR measurement for beam reporting	2701

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2703

A.6.6.8.2.1	Test Purpose and Environment	2703

A.6.6.8.2.2	Test parameters	2703

A.6.6.8.2.3	Test Requirements	2707

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2707

A.6.6.8.3.1	Test Purpose and Environment	2707

A.6.6.8.3.2	Test parameters	2708

A.6.6.8.3.3	Test Requirements	2710

A.6.6.9	Idle Mode CA/DC Measurements	2710

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2710

A.6.6.9.1.1	Test Purpose and Environment	2710

A.6.6.9.1.2	Test Requirements	2717

A.6.6.10	CSI-RS based intra-frequency Measurements	2717

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2717

A.6.6.10.1.1	Test purpose and Environment	2717

A.6.6.10.1.2	Test Requirements	2720

A.6.6.11	CSI-RS based inter-frequency Measurements	2720

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	2720

A.6.6.11.1.1	Test Purpose and Environment	2720

A.6.6.11.1.2	Test Requirements	2724

A.6.6.12	RSTD measurements	2724

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2724

A.6.6.12.1.1	Test Purpose and Environment	2724

A.6.6.12.1.2	Test Requirements	2731

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2732

A.6.6.12.2.1	Test Purpose and Environment	2732

A.6.6.12.2.2	Test Requirements	2738

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2738

A.6.6.12.3.1	Test Purpose and Environment	2738

A.6.6.12.3.2	Test Requirements	2744

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	2745

A.6.6.12.4.1	Test Purpose and Environment	2745

A.6.6.12.4.2	Test Requirements	2748

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	2749

A.6.6.12.5.1	Test Purpose and Environment	2749

A.6.6.12.5.2	Test Requirements	2753

A.6.6.13 PRS-RSRP measurements	2753

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	2753

A.6.6.13.1.1	Test purpose and Environment	2753

A.6.6.13.1.2	Test Requirements	2757

The rate of correct events observed during repeated tests shall be at least 90%A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2757

A.6.6.13.2.1	Test purpose and Environment	2757

A.6.6.13.2.2	Test Requirements	2761

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	2761

A.6.6.13.3.1	Test purpose and Environment	2761

A.6.6.13.3.2	Test Requirements	2764

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	2764

A.6.6.13.4.1	Test purpose and Environment	2764

A.6.6.14	UE Rx-Tx time difference measurements	2769

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2769

A.6.6.14.1.1	Test purpose and environment	2769

A.6.6.14.1.2	Test requirements	2773

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	2773

A.6.6.14.2.1	Test purpose and environment	2773

A.6.6.14.2.2	Test requirements	2777

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	2777

A.6.6.14.3.1	Test purpose and environment	2777

A.6.6.14.3.2	Test requirements	2780

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	2780

A.6.6.14.4.1	Test purpose and environment	2780

A.6.6.14.4.2	Test requirements	2783

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	2783

A.6.6.14.4.1	Test purpose and environment	2783

A.6.6.14.4.2	Test requirements	2787

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	2787

A.6.6.15.1	Test Purpose and Environment	2787

A.6.6.15.2	Test Requirements	2794

A.6.6.16	PRS-RSRPP measurements	2795

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	2795

A.6.6.16.1.1	Test purpose and Environment	2795

A.6.6.16.1.2	Test Requirements	2797

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2797

A.6.6.16.2.1	Test purpose and Environment	2797

A.6.6.16.2.2	Test Requirements	2800

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	2800

A.6.6.16.3.1	Test purpose and Environment	2800

A.6.6.16.3.2	Test Requirements	2803

A.6.6.17	SA event triggered reporting tests with Pre-MG	2803

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	2803

A.6.6.17.1.1	Test purpose and Environment	2803

A.6.6.17.1.2	Test parameters	2803

A.6.6.17.1.3	Test Requirements	2807

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2807

A.6.6.17.2.1	Test purpose and Environment	2807

A.6.6.17.2.2	Test parameters	2807

A.6.6.17.2.3	Test Requirements	2812

A.6.6.17.3	Void	2812

A.6.6.17.3.1	Void	2812

A.6.6.17.3.2	Void	2812

A.6.6.17.3.3	Void	2812

A.6.6.18	SA event triggered reporting tests with concurrent gaps	2812

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	2812

A.6.6.18.1.1	Test Purpose and Environment	2812

A.6.6.18.1.2	Test Requirements	2816

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	2816

A.6.6.18.2.1	Test Purpose and Environment	2816

A.6.6.18.2.2	Test Requirements	2820

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2820

A.6.6.18.3.1	Test Purpose and Environment	2820

A.6.6.18.3.2	Test Requirements	2827

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2827

A.6.6.18.4.1	Test Purpose and Environment	2827

A.6.6.18.4.2	Test Requirements	2831

A.6.6.19	SA event triggered reporting tests with NCSG	2832

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	2832

A.6.6.19.1.1	Test purpose and Environment	2832

A.6.6.19.1.2	Test parameters	2832

A.6.6.19.1.3	Test Requirements	2836

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2836

A.6.6.19.2.1	Test Purpose and Environment	2836

A.6.6.19.2.2	Test parameters	2836

A.6.6.19.2.3	Test Requirements	2840

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2840

A.6.6.19.3.1	Test Purpose and Environment	2840

A.6.6.19.3.2	Test parameters	2841

A.6.6.19.3.3	Test Requirements	2846

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2846

A.6.6.19.4.1	Test purpose and Environment	2846

A.6.6.19.4.2	Test parameters	2846

A.6.6.19.4.3	Test Requirements	2849

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2849

A.6.6.20.1	Test purpose and environment	2849

A.6.6.20.2	Test requirements	2853

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	2853

A.6.6.21.1	Test purpose and environment	2853

A.6.6.21.2	Test requirements	2855

A.6.7	Measurement Performance requirements	2856

A.6.7.1	SS-RSRP	2856

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2856

A.6.7.1.1.1	Test Purpose and Environment	2856

A.6.7.1.1.2	Test parameters	2856

A.6.7.1.1.3	Test Requirements	2861

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2861

A.6.7.1.2.1	Test Purpose and Environment	2861

A.6.7.1.2.2	Test parameters	2861

A.6.7.1.2.3	Test Requirements	2865

A.6.7.1.3	Void	2865

A.6.7.2	SS-RSRQ	2865

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2865

A.6.7.2.1.1	Test Purpose and Environment	2865

A.6.7.2.1.2	Test Parameters	2866

A.6.7.2.1.3	Test Requirements	2870

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2870

A.6.7.2.2.1	Test Purpose and Environment	2870

A.6.7.2.2.2	Test Parameters	2870

A.6.7.2.2.3	Test Requirements	2875

A.6.7.3	SS-SINR	2875

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2875

A.6.7.3.1.1	Test Purpose and Environment	2875

A.6.7.3.1.2	Test Parameters	2876

A.6.7.3.1.3	Test Requirements	2880

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2881

A.6.7.3.2.1	Test Purpose and Environment	2881

A.6.7.3.2.2	Test Parameters	2881

A.6.7.3.2.3	Test Requirements	2885

A.6.7.4	L1-RSRP measurement for beam reporting	2886

A.6.7.4.1	SSB based L1-RSRP measurement	2886

A.6.7.4.1.1	Test Purpose and Environment	2886

A.6.7.4.1.2	Test parameters	2886

A.6.7.4.1.3	Test Requirements	2890

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2890

A.6.7.4.2.1	Test Purpose and Environment	2890

A.6.7.4.2.2	Test parameters	2891

A.6.7.4.2.3	Test Requirements	2895

A.6.7.5	E-UTRAN RSRP	2895

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2895

A.6.7.5.1.1	Test Purpose and Environment	2895

A.6.7.5.1.2	Test parameters	2896

A.6.7.5.1.3	Test Requirements	2902

A.6.7.6	E-UTRAN RSRQ	2903

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2903

A.6.7.6.1.1	Test Purpose and Environment	2903

A.6.7.6.1.2	Test parameters	2903

A.6.7.6.1.3	Test Requirements	2908

A.6.7.7	E-UTRAN RS-SINR	2909

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2909

A.6.7.7.1.1	Test Purpose and Environment	2909

A.6.7.7.1.2	Test parameters	2909

A.6.7.7.1.3	Test Requirements	2915

A.6.7.8	CLI measurements	2916

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	2916

A.6.7.8.1.1	Test Purpose and Environment	2916

A.6.7.8.1.2	Test parameters	2916

A.6.7.8.1.3	Test Requirements	2922

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	2922

A.6.7.8.2.1	Test Purpose and Environment	2922

A.6.7.8.2.2	Test parameters	2923

A.6.7.8.2.3	Test Requirements	2926

A.6.7.9	L1-SINR measurement for beam reporting	2927

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2931

A.6.7.9.2.1	Test Purpose and Environment	2931

A.6.7.9.2.2	Test parameters	2932

A.6.7.9.2.3	Test Requirements	2937

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2937

A.6.7.9.3.1	Test Purpose and Environment	2937

A.6.7.9.3.2	Test parameters	2937

A.6.7.9.3.3	Test Requirements	2942

A.6.7.10	CSI-RSRP	2942

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2942

A.6.7.9.10.1	Test Purpose and Environment	2942

A.6.7.9.10.2	Test parameters	2943

A.6.7.10.1.3	Test Requirements	2948

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2948

A.6.7.9.10.1	Test Purpose and Environment	2948

A.6.7.10.2.2	Test parameters	2948

A.6.7.10.2.3	Test Requirements	2953

A.6.7.11	CSI-RSRQ	2953

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2953

A.6.7.11.1.1	Test Purpose and Environment	2953

A.6.7.11.1.2	Test Parameters	2953

A.6.7.11.1.3	Test Requirements	2958

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2958

A.6.7.11.2.1	Test Purpose and Environment	2958

A.6.7.11.2.2	Test Parameters	2958

A.6.7.11.2.3	Test Requirements	2964

A.6.7.12	CSI-SINR	2965

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2965

A.6.7.12.1.1	Test Purpose and Environment	2965

A.6.7.12.1.2	Test Parameters	2965

A.6.7.12.1.3	Test Requirements	2969

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2970

A.6.7.12.2.1	Test Purpose and Environment	2970

A.6.7.12.2.2	Test Parameters	2970

A.6.7.12.2.3	Test Requirements	2976

A.6.7.13	RSTD measurements	2976

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	2976

A.6.7.13.1.1	Test purpose and Environment	2976

A.6.7.13.1.2	Test Requirements	2979

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	2979

A.6.7.13.2.1	Test purpose and Environment	2979

A.6.7.13.2.2	Test Requirements	2983

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2983

A.6.7.13.3.1	Test purpose and Environment	2983

A.6.7.13.3.2	Test Requirements	2985

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	2986

A.6.7.14	PRS-RSRP measurements	2988

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	2988

A.6.7.14.1.1	Test Purpose and Environment	2988

A.6.7.14.1.2	Test parameters	2988

A.6.7.14.1.3	Test Requirements	2992

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	2992

A.6.7.14.2.1	Test Purpose and Environment	2992

A.6.7.14.2.2	Test parameters	2992

A.6.7.14.2.3	Test Requirements	2995

A.6.7.14.3	Void	2995

A.6.7.14.3.1	Void	2995

A.6.7.14.3.2	Void	2995

A.6.7.14.3.3	Void	2995

A.6.7.15	UE Rx-Tx time difference measurements	2995

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	2995

A.6.7.15.1.1	Test purpose and environment	2995

A.6.7.15.1.2	Test parameters	2996

A.6.7.15.1.3	Test requirements	2999

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	3000

A.6.7.15.2.1	Test purpose and environment	3000

A.6.7.15.2.2	Test parameters	3000

A.6.7.15.2.3	Test requirements	3002

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3002

A.6.7.15.3.1	Test purpose and environment	3002

A.6.7.15.3.2	Test parameters	3003

A.6.7.15.3.3	Test requirements	3006

A.6.7.16	PRS-RSRPP measurements	3006

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	3006

A.6.7.16.1.1	Test Purpose and Environment	3006

A.6.7.16.1.2	Test parameters	3007

A.6.7.16.1.3	Test Requirements	3011

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	3011

A.6.7.16.2.1	Test Purpose and Environment	3011

A.6.7.16.2.2	Test parameters	3011

A.6.8	Measurement procedure in RRC_INACTIVE	3015

A.6.8.1	RSTD measurements	3015

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	3015

A.6.8.1.1.1	Test Purpose and Environment	3015

A.6.8.1.1.2	Test Requirements	3018

A.6.8.1.2	 NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3018

A.6.8.1.2.1	Test Purpose and Environment	3018

A.6.8.1.2.2	Test Requirements	3023

A.6.8.2	PRS-RSRP measurements	3023

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3023

A.6.8.2.1.1	Test purpose and Environment	3023

A.6.8.2.1.2	Test Requirements	3025

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3026

A.6.8.2.2.1	Test purpose and Environment	3026

A.6.8.2.2.2	Test Requirements	3030

A.6.8.3	UE Rx-Tx time difference measurements	3030

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3030

A.6.8.3.1.1	Test purpose and environment	3030

A.6.8.3.1.2	Test requirements	3034

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	3034

A.6.8.3.2.1	Test purpose and environment	3034

A.6.8.3.2.2	Test requirements	3037

A.6.8.4	PRS-RSRPP measurements	3037

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3037

A.6.8.4.1.1	Test purpose and Environment	3037

A.6.8.4.1.2	Test Requirements	3039

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	3040

A.6.8.4.2.1	Test purpose and Environment	3040

A.6.8.4.2.2	Test Requirements	3042

A.6.9	Measurement performance requirements in RRC_INACTIVE	3042

A.6.9.1	RSTD measurements	3042

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3042

A.6.9.1.1.1	Test purpose and Environment	3042

A.6.9.1.1.2	Test Requirements	3044

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	3045

A.6.9.1.2.1	Test purpose and Environment	3045

A.6.9.1.2.2	Test Requirements	3047

A.6.9.2	PRS-RSRP measurements	3047

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	3047

A.6.9.2.1.1	Test Purpose and Environment	3047

A.6.9.2.1.2	Test parameters	3047

A.6.9.2.1.3	Test Requirements	3050

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	3050

A.6.9.2.2.1	Test Purpose and Environment	3050

A.6.9.2.2.2	Test parameters	3050

A.6.9.2.2.3	Test Requirements	3052

A.6.9.3	UE Rx-Tx time difference measurements	3053

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	3053

A.6.9.3.1.1.1	Test purpose and environment	3053

A.6.9.3.1.1.2 Test parameters	3053

A.6.9.3.1.1.3	Test requirements	3054

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	3055

A.6.9.3.2.1	Test purpose and environment	3055

A.6.9.3.2.2	Test parameters	3055

A.6.9.3.2.3	Test requirements	3058

A.6.9.4	PRS-RSRPP measurements	3058

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	3058

A.6.9.4.1.1	Test Purpose and Environment	3058

A.6.9.4.1.2	Test parameters	3058

A.6.9.4.1.3	Test Requirements	3062

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	3062

A.6.9.4.2.1	Test Purpose and Environment	3062

A.6.9.4.2.2	Test parameters	3062

A.6.9.4.2.3	Test Requirements	3064

A.7	NR standalone tests with one or more NR cells in FR2	3067

A.7.1	SA: RRC_IDLE state mobility	3067

A.7.1.1	Cell re-selection to NR	3067

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	3067

A.7.1.1.1.1	Test Purpose and Environment	3067

A.7.1.1.1.2	Test Parameters	3067

A.7.1.1.1.3	Test Requirements	3070

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	3070

A.7.1.1.2.1	Test Purpose and Environment	3070

A.7.1.1.2.2	Test Parameters	3071

A.7.1.1.2.3	Test Requirements	3073

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3073

A.7.1.1.3.1	Test Purpose and Environment	3073

A.7.1.1.3.2	Test Parameters	3073

A.7.1.1.3.3	Test Requirements	3076

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3076

A.7.1.1.4.1	Test Purpose and Environment	3076

A.7.1.1.4.2	Test Parameters	3076

A.7.1.1.4.3	Test Requirements	3079

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3079

A.7.1.1.5.1	Test Purpose and Environment	3079

A.7.1.1.5.2	Test Parameters	3079

A.7.1.1.5.3	Test Requirements	3082

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3082

A.7.1.1.6.1	Test Purpose and Environment	3082

A.7.1.1.6.2	Test Parameters	3082

A.7.1.1.6.3	Test Requirements	3085

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3085

A.7.1.1.7.1	Test Purpose and Environment	3085

A.7.1.1.7.2	Test Parameters	3085

A.7.1.1.7.3	Test Requirements	3089

A.7.2	SA: RRC_INACTIVE state mobility	3089

A.7.2.1	Small Data Transmission	3089

A.7.2.1.1	TA validation for CG-SDT in FR2	3089

A.7.2.1.1.1	Test Purpose and Environment	3089

A.7.2.1.1.2	Test Requirements	3093

A.7.3	RRC_CONNECTED state mobility	3093

A.7.3.1	Handover	3093

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	3093

A.7.3.1.1.1	Test Purpose and Environment	3093

A.7.3.1.1.2	Test Parameters	3093

A.7.3.1.1.3	Test Requirements	3097

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	3097

A.7.3.1.2.1	Test Purpose and Environment	3097

A.7.3.1.2.2	Test Parameters	3097

A.7.3.1.2.3	Test Requirements	3100

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	3100

A.7.3.1.3.1	Test Purpose and Environment	3100

A.7.3.1.3.2	Test Parameters	3100

A.7.3.1.3.3	Test Requirements	3101

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	3102

A.7.3.1.4.1	Test Purpose and Environment	3102

A.7.3.1.4.2	Test Parameters	3102

A.7.3.1.4.3 Test Requirements	3109

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	3109

A.7.3.1.5.1	Test Purpose and Environment	3109

A.7.3.1.5.2	Test Parameters	3109

A.7.3.1.5.3 Test Requirements	3116

A.7.3.1.6	Handover with PSCell from SA to EN-DC with; unknown FR2 target PScell	3116

A.7.3.1.6.1	Test Purpose and Environment	3116

A.7.3.1.6.2	Test Parameters	3116

A.7.3.1.6.3	Test Requirements	3124

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	3125

A.7.3.1.7.1	Test purpose and environment	3125

A.7.3.1.7.2	Test Requirements	3130

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	3131

A.7.3.1.8.1	Test Purpose and Environment	3131

A.7.3.1.8.2	Test Requirements	3136

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	3136

A.7.3.1.9.1	Test Purpose and Environment	3136

A.7.3.1.9.2	Test Parameters	3136

A.7.3.1.9.3	Test Requirements	3139

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3139

A.7.3.1.10.1	Test Purpose and Environment	3139

A.7.3.1.10.2	Test Parameters	3139

A.7.3.1.10.3	Test Requirements	3143

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3143

A.7.3.1.11.1	Test Purpose and Environment	3143

A.7.3.1.11.2	Test Parameters	3143

A.7.3.1.11.3	Test Requirements	3147

A.7.3.2	RRC Connection Mobility Control	3147

A.7.3.2.1	SA: RRC Re-establishment	3147

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3147

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3150

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3153

A.7.3.2.1.3.1	Test Purpose and Environment	3153

A.7.3.2.1.3.2	Test Requirements	3155

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	3156

A.7.3.2.1.4.1	Test Purpose and Environment	3156

A.7.3.2.1.4.2	Test Requirements	3158

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	3159

A.7.3.2.1.5.1	Test Purpose and Environment	3159

A.7.3.2.1.5.2	Test Requirements	3162

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	3162

A.7.3.2.1.6.1	Test Purpose and Environment	3162

A.7.3.2.1.6.2	Test Requirements	3164

A.7.3.2.2	Random Access	3165

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3165

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3169

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3173

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3176

A.7.3.2.3	SA: RRC Connection Release with Redirection	3179

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3179

A.7.3.3 Conditional Handover	3183

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3183

A.7.3.3.1.1	Test Purpose and Environment	3183

A.7.3.3.1.2	Test Parameters	3183

A.7.3.3.1.2.3	Test Requirements	3186

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3186

A.7.3.3.2.1	Test Purpose and Environment	3186

A.7.3.3.2.2	Test Parameters	3186

A.7.3.3.2.3 Test Requirements	3188

A.7.4	Timing	3188

A.7.4.1	UE transmit timing	3188

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3188

A.7.4.1.1.1	Test Purpose and environment	3188

A.7.4.1.1.2	Test requirements	3191

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3192

A.7.4.1.2.1	Test Purpose and environment	3192

A.7.4.1.2.2	Test requirements	3196

A.7.4.2	UE timer accuracy	3197

A.7.4.3	Timing advance	3197

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3197

A.7.4.3.1.1	Test Purpose and Environment	3197

A.7.4.3.1.2	Test Parameters	3197

A.7.4.3.1.3 Test Requirements	3200

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3201

A.7.4.3.2.1	Test Purpose and Environment	3201

A.7.4.3.2.2	Test Parameters	3201

A.7.4.3.2.3	Test Requirements	3205

A.7.5	Signaling characteristics	3205

A.7.5.1	Radio link Monitoring	3205

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3205

A.7.5.1.1.1	Test Purpose and Environment	3205

A.7.5.1.1.2	Test Requirements	3208

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3209

A.7.5.1.2.1	Test Purpose and Environment	3209

A.7.5.1.2.2	Test Requirements	3213

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3214

A.7.5.1.3.1	Test Purpose and Environment	3214

A.7.5.1.3.2	Test Requirements	3218

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3218

A.7.5.1.4.1	Test Purpose and Environment	3218

A.7.5.1.4.2	Test Requirements	3223

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3223

A.7.5.1.5.1	Test Purpose and Environment	3223

A.7.5.1.5.2	Test Requirements	3228

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3228

A.7.5.1.6.1	Test Purpose and Environment	3228

A.7.5.1.6.2	Test Requirements	3232

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3232

A.7.5.1.7.1	Test Purpose and Environment	3232

A.7.5.1.7.2	Test Requirements	3236

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3236

A.7.5.1.8.1	Test Purpose and Environment	3236

A.7.5.1.8.2	Test Requirements	3241

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3241

A.7.5.1.9.1	Test Purpose and Environment	3241

A.7.5.1.9.2	Test Requirements	3244

A.7.5.2	Interruption	3244

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3244

A.7.5.2.1.1	Test Purpose and Environment	3244

A.7.5.2.1.2	Test Requirements	3247

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3248

A.7.5.2.2.1	Test Purpose and Environment	3248

A.7.5.2.2.2	Test Parameters	3248

A.7.5.2.2.3	Test Requirements	3250

A.7.5.3	SCell Activation and Deactivation Delay	3250

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3250

A.7.5.3.1.1	Test Purpose and Environment	3250

A.7.5.3.1.2	Test Requirements	3252

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3252

A.7.5.3.2.1	Test Purpose and Environment	3252

A.7.5.3.2.2	Test Requirements	3256

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3257

A.7.5.3.3.1	Test Purpose and Environment	3257

A.7.5.3.3.2	Test Requirements	3260

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3261

A.7.5.3.4.1	Test Purpose and Environment	3261

A.7.5.3.4.2	Test Requirements	3264

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3265

A.7.5.3.5.1	Test Purpose and Environment	3265

A.7.5.3.5.2	Test Requirements	3268

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3269

A.7.5.3.6.1	Test Purpose and Environment	3269

A.7.5.3.6.2	Test Requirements	3273

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3274

A.7.5.3.7.1	Test Purpose and Environment	3274

A.7.5.3.7.2	Test Requirements	3278

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3279

A.7.5.3.8.1	Test Purpose and Environment	3279

A.7.5.3.8.2	Test Requirements	3283

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3284

A.7.5.3.9.1	Test Purpose and Environment	3284

A.7.5.3.9.2	Test Requirements	3287

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3288

A.7.5.3.10.1	Test Purpose and Environment	3288

A.7.5.3.10.2	Test Requirements	3292

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3293

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3293

A.7.5.3.11.1.1	Test Purpose and Environment	3293

A.7.5.3.11.1.2	Test Requirements	3297

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3298

A.7.5.3.11.2.1	Test Purpose and Environment	3298

A.7.5.3.11.2.2	Test Requirements	3302

A.7.5.3.12	Void	3303

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3303

A.7.5.3.13.1	Test Purpose and Environment	3303

A.7.5.3.13.2	Test Requirements	3305

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3306

A.7.5.3.14.1	Test Purpose and Environment	3306

A.7.5.3.14.2	Test Requirements	3309

A.7.5.15	Void	3310

A.7.5.4	Void	3310

A.7.5.5	Beam Failure Detection and Link recovery procedures	3310

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3310

A.7.5.5.1.1	Test Purpose and Environment	3310

A.7.5.5.1.2	Test Requirements	3314

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3315

A.7.5.5.2.1	Test Purpose and Environment	3315

A.7.5.5.2.2	Test Requirements	3318

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3319

A.7.5.5.3.1	Test Purpose and Environment	3319

A.7.5.5.3.2	Test Requirements	3323

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3324

A.7.5.5.4.1	Test Purpose and Environment	3324

A.7.5.5.4.2	Test Requirements	3328

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3329

A.7.5.5.5.1	Test Purpose and Environment	3329

A.7.5.5.5.2	Test Requirements	3332

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3333

A.7.5.5.6.1	Test Purpose and Environment	3333

A.7.5.5.6.2	Test Requirements	3337

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3338

A.7.5.5.7.1	Test Purpose and Environment	3338

A.7.5.5.7.2	Test Requirements	3342

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3343

A.7.5.5.8.1	Test Purpose and Environment	3343

A.7.5.5.8.2	Test Requirements	3347

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3347

A.7.5.5.9.1	Test Purpose and Environment	3347

A.7.5.5.9.2	Test Requirements	3353

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3353

A.7.5.5.10.1	Test Purpose and Environment	3353

A.7.5.5.10.2	Test Requirements	3359

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3359

A.7.5.5.11.1	Test Purpose and Environment	3359

A.7.5.5.11.2	Test Requirements	3364

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3364

A.7.5.5.12.1	Test Purpose and Environment	3364

A.7.5.5.12.2	Test Requirements	3369

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	3369

A.7.5.5.13.1	Test Purpose and Environment	3369

A.7.5.5.13.2	Test Requirements	3373

A.7.5.6	Active BWP switch	3373

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3373

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3373

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3378

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3383

A.7.5.6.1.3.1	Test Purpose and Environment	3383

A.7.5.6.1.3.2	Test Requirements	3386

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3386

A.7.5.6.1.4.1	Test Purpose and Environment	3386

A.7.5.6.1.4.2	Test Requirements	3391

A.7.5.6.2	RRC-based Active BWP Switch	3392

A.7.5.6.2.1.1	Test Purpose and Environment	3392

A.7.5.6.2.1.2	Test Requirements	3395

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3396

A.7.5.6.2.2.1	Test Purpose and Environment	3396

A.7.5.6.2.2.2	Test Requirements	3400

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3401

A.7.5.6.3.1.1	Test Purpose and Environment	3401

A.7.5.6.3.1.2	Test Requirements	3404

A.7.5.6.4	SCell dormancy switch	3404

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3404

A.7.5.6.4.1.1	Test Purpose and Environment	3404

A.7.5.6.4.1.2	Test Requirements	3409

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3409

A.7.5.6.4.2.1	 Test Purpose and Environment	3409

A.7.5.6.4.2.2	 Test Requirements	3414

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3414

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3414

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3417

A.7.5.6.5.2.1	Test Purpose and Environment	3417

A.7.5.6.5.2.2	Test Requirements	3421

A.7.5.7	PSCell addition and release delay	3422

A.7.5.7.1	Addition and Release Delay of known NR PSCell	3422

A.7.5.7.1.1	Test Purpose and Environment	3422

A.7.5.7.1.2	Test Requirements	3425

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in	3425

A.7.5.7.2.1	Test Purpose and Environment	3425

A.7.5.7.2.2	Test Requirements	3428

A.7.5.7.3	Addition and Release Delay of known NR PSCell in FR2-2	3428

A.7.5.7.3.1	Test Purpose and Environment	3428

A.7.5.7.3.2	Test Requirements	3431

A.7.5.7.4	Addition and Release Delay of unknown NR PSCell in FR2-2	3431

A.7.5.7.4.1	Test Purpose and Environment	3431

A.7.5.7.4.2	Test Requirements	3434

A.7.5.8	Active TCI state switch delay	3434

A.7.5.8.1	MAC-CE based active TCI state switch	3434

A.7.5.8.2	RRC based active TCI state switch	3438

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3442

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3442

A.7.5.8.3.1.1	Test Purpose and Environment	3442

A.7.5.8.3.1.2	Test Requirements	3446

A.7.5.9	Uplink spatial relation switch delay	3447

A.7.5.9.1.1.1	Test Purpose and Environment	3447

A.7.5.9.1.1.2	Test Requirements	3450

A.7.5.9.2	RRC based spatial relation switch	3450

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3450

A.7.5.9.2.1.1	Test Purpose and Environment	3450

A.7.5.9.2.1.2	Test Requirements	3453

A.7.5.10	UE specific CBW change	3453

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3453

A.7.5.10.1.1	Test Purpose and Environment	3453

A.7.5.10.1.2	Test Requirements	3456

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3457

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3457

A.7.5.11.1.1	Test Purpose and Environment	3457

A.7.5.11.1.2	Test Requirements	3460

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3460

A.7.5.12.1	Addition and Release Delay of PSCell	3460

A.7.5.12.1.1	Test purpose and environment	3460

A.7.5.12.1.2	Test Parameters	3460

A.7.5.12.1.3	Test Requirements	3463

A.7.5.13	 Unified TCI state switching delay	3463

A.7.5.13.1	MAC-CE based active joint TCI state switching	3463

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3463

A.7.5.13.1.1.1	Test Purpose and Environment	3463

A.7.5.13.1.1.2	Test parameters	3464

A.7.5.13.1.1.3	Test Requirements	3466

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3466

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3466

A.7.5.13.2.1.1	Test Purpose and Environment	3466

A.7.5.13.2.1.2	Test parameters	3467

A.7.5.13.2.1.3	Test Requirements	3469

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3469

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3469

A.7.5.13.3.1.1	Test Purpose and Environment	3469

A.7.5.13.3.1.2	Test Parameters	3470

A.7.5.13.3.1.3	Test Requirements	3473

A.7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3473

A.7.5.14.1	Test Purpose and Environment	3473

A.7.5.14.2	Test Requirements	3476

A.7.6	Measurement procedure	3477

A.7.6.1	Intra-frequency Measurements	3477

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	3477

A.7.6.1.1.1	Test purpose and Environment	3477

A.7.6.1.1.2	Test Requirements	3480

A.7.6.1.2	SA event triggered reporting test without gap under DRX	3480

A.7.6.1.2.1	Test purpose and Environment	3480

A.7.6.1.2.2	Test Requirements	3483

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	3484

A.7.6.1.3.1	Test purpose and Environment	3484

A.7.6.1.3.2	Test Requirements	3487

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	3487

A.7.6.1.4.1	Test purpose and Environment	3487

A.7.6.1.4.2	Test Requirements	3490

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasFlagFR2-r17	3491

A.7.6.1.5.1	Test purpose and Environment	3491

A.7.6.1.5.2	Test Requirements	3494

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	3494

A.7.6.1.6.1	Test purpose and Environment	3494

A.7.6.1.6.2	Test Requirements	3497

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	3498

A.7.6.1.7.1	Test purpose and Environment	3498

A.7.6.1.7.2	Test Requirements	3500

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	3501

A.7.6.1.8.1	Test purpose and Environment	3501

A.7.6.1.8.2	Test Requirements	3504

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	3505

A.7.6.1.9.1	Test purpose and Environment	3505

A.7.6.1.9.2	Test Requirements	3507

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	3508

A.7.6.1.10.1	Test purpose and Environment	3508

A.7.6.1.10.2	Test Requirements	3512

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	3512

A.7.6.1.11.1	Test purpose and Environment	3512

A.7.6.1.11.2	Test Requirements	3515

A.7.6.2	Inter-frequency Measurements	3515

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	3515

A.7.6.2.1.1	Test Purpose and Environment	3515

A.7.6.2.1.2	Test Requirements	3519

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR2)	3519

A.7.6.2.2.1	Test Purpose and Environment	3519

A.7.6.2.2.2	Test Requirements	3523

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	3523

A.7.6.2.3.1	Test Purpose and Environment	3523

A.7.6.2.3.2	Test Requirements	3527

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (Pcell in FR2)	3527

A.7.6.2.4.1	Test Purpose and Environment	3527

A.7.6.2.4.2	Test Requirements	3531

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3531

A.7.6.2.5.1	Test Purpose and Environment	3531

A.7.6.2.5.2	Test Requirements	3535

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR1)	3536

A.7.6.2.6.1	Test Purpose and Environment	3536

A.7.6.2.6.2	Test Requirements	3540

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3541

A.7.6.2.7.1	Test Purpose and Environment	3541

A.7.6.2.7.2	Test Requirements	3545

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3546

A.7.6.2.8.1	Test Purpose and Environment	3546

A.7.6.2.8.2	Test Requirements	3550

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3551

A.7.6.2.9.1	Test Purpose and Environment	3551

A.7.6.2.9.2	Test Requirements	3555

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	3555

A.7.6.2.10.1	Test Purpose and Environment	3555

A.7.6.2.10.2	Test Requirements	3557

A.7.6.2.11	SA event triggered reporting test without gap under DRX	3558

A.7.6.2.11.1	Test Purpose and Environment	3558

A.7.6.2.11.2	Test Requirements	3560

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	3561

A.7.6.2.12.1	Test Purpose and Environment	3561

A.7.6.2.12.2	Test Requirements	3564

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	3565

A.7.6.2.13.1	Test Purpose and Environment	3565

A.7.6.2.13.2	Test Requirements	3569

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	3570

A.7.6.2.14.1	Test Purpose and Environment	3570

A.7.6.2.14.2	Test Requirements	3574

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	3575

A.7.6.2.15.1	Test Purpose and Environment	3575

A.7.6.2.15.2	Test Requirements	3579

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	3580

A.7.6.2.16.1	Test Purpose and Environment	3580

A.7.6.2.16.2	Test Requirements	3586

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	3586

A.7.6.2.17.1	Test Purpose and Environment	3586

A.7.6.2.17.2	Test Requirements	3593

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	3594

A.7.6.2.18.1	Test Purpose and Environment	3594

A.7.6.2.18.2	Test Requirements	3600

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	3600

A.7.6.2.19.1	Test Purpose and Environment	3600

A.7.6.2.19.2	Test Requirements	3608

A.7.6.3	L1-RSRP measurement for beam reporting	3609

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3609

A.7.6.3.1.1	Test Purpose and Environment	3609

A.7.6.3.1.2	Test parameters	3609

A.7.6.3.1.3	Test Requirements	3611

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	3611

A.7.6.3.2.1	Test Purpose and Environment	3611

A.7.6.3.2.2	Test parameters	3612

A.7.6.3.2.3	Test Requirements	3614

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	3614

A.7.6.3.3.1	Test Purpose and Environment	3614

A.7.6.3.3.2	Test parameters	3615

A.7.6.3.3.3	Test Requirements	3617

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	3618

A.7.6.3.4.1	Test Purpose and Environment	3618

A.7.6.3.4.2	Test parameters	3618

A.7.6.3.3.3	Test Requirements	3620

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3621

A.7.6.3.5.1	Test Purpose and Environment	3621

A.7.6.3.5.2	Test parameters	3621

A.7.6.3.5.3	Test Requirements	3623

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3623

A.7.6.3.6.1	Test Purpose and Environment	3623

A.7.6.3.6.2	Test parameters	3624

A.7.6.3.6.3	Test Requirements	3627

A.7.6.3.7	SSB based L1-RSRP measurement for FR2-2 when DRX is used	3627

A.7.6.3.7.1	Test Purpose and Environment	3627

A.7.6.3.7.2	Test parameters	3628

A.7.6.3.7.3	Test Requirements	3631

A.7.6.4	CLI measurements	3632

A.7.6.4.1	SRS-RSRP measurement with non-DRX	3632

A.7.6.4.1.1	Test Purpose and Environment	3632

A.7.6.4.1.2	Test Parameters	3632

A.7.6.4.1.3	Test Requirements	3634

A.7.6.4.2	CLI-RSSI measurement with non-DRX	3634

A.7.6.4.2.1	Test Purpose and Environment	3634

A.7.6.4.2.2	Test Parameters	3635

A.7.6.4.2.3	Test Requirements	3636

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	3637

A.7.6.5.1.1	Test Purpose and Environment	3637

A.7.6.5.1.2	Test Requirements	3640

A.7.6.6	L1-SINR measurement for beam reporting	3640

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	3642

A.7.6.6.2.1	Test Purpose and Environment	3642

A.7.6.6.2.2	Test parameters	3643

A.7.6.6.2.3	Test Requirements	3645

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	3645

A.7.6.6.3.1	Test Purpose and Environment	3645

A.7.6.6.3.2	Test parameters	3646

A.7.6.6.3.3	Test Requirements	3648

A.7.6.7	CSI-RS based intra-frequency Measurements	3648

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	3648

A.7.6.7.1.1	Test purpose and Environment	3648

A.7.6.7.1.2	Test Requirements	3651

A.7.6.8	CSI-RS based inter-frequency Measurements	3651

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	3651

A.7.6.8.1.1	Test Purpose and Environment	3651

A.7.6.8.1.2	Test Requirements	3654

A.7.6.9	RSTD measurements	3654

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	3654

A.7.6.9.1.1	Test Purpose and Environment	3654

A.7.6.9.1.2	Test Requirements	3662

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	3662

A.7.6.9.2.1	Test Purpose and Environment	3662

A.7.6.9.2.2	Test Requirements	3670

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	3670

A.7.6.9.3.1	Test Purpose and Environment	3670

A.7.6.9.3.2	Test Requirements	3676

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	3677

A.7.6.9.4.1	Test Purpose and Environment	3677

A.7.6.9.4.2	Test Requirements	3680

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	3681

A.7.6.9.5.1	Test Purpose and Environment	3681

A.7.6.9.5.2	Test Requirements	3684

A.7.6.10 PRS-RSRP measurements	3685

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	3685

A.7.6.10.1.1	Test Purpose and Environment	3685

A.7.6.10.1.2	Test Requirements	3689

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3689

A.7.6.10.2.1	Test Purpose and Environment	3689

A.7.6.10.2.2	Test Requirements	3693

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	3693

A.7.6.10.3.1	Test Purpose and Environment	3693

A.7.6.10.3.2	Test Requirements	3697

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3697

A.7.6.10.4.1	Test Purpose and Environment	3697

A.7.6.10.4.2	Test Requirements	3701

A.7.6.11	UE Rx-Tx time difference measurements	3701

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3701

A.7.6.11.1.1	Test purpose and environment	3701

A.7.6.11.1.2	Test requirements	3705

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	3705

A.7.6.11.2.1	Test purpose and environment	3705

A.7.6.11.2.2	Test requirements	3709

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	3709

A.7.6.11.3.1	Test purpose and environment	3709

A.7.6.11.3.2	Test requirements	3711

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	3712

A.7.6.11.4.1	Test purpose and environment	3712

A.7.6.11.4.2	Test requirements	3714

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with RxTx TEG	3715

A.7.6.11.5.1	Test purpose and environment	3715

A.7.6.11.5.2	Test requirements	3717

A.7.6.12	PRS-RSRPP measurements	3718

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3718

A.7.6.12.1.1	Test Purpose and Environment	3718

A.7.6.12.1.2	Test Requirements	3721

A.7.6.12.2	PRS-RSRPP reporting delay test case for reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3721

A.7.6.12.2.1	Test Purpose and Environment	3721

A.7.6.12.2.2	Test Requirements	3724

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	3724

A.7.6.12.3.1	Test Purpose and Environment	3724

A.7.6.12.3.2	Test Requirements	3727

A.7.6.13	UE Rx-Tx time difference measurements for PDC	3728

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	3728

A.7.6.13.1.1	Test purpose and environment	3728

A.7.6.13.1.2	Test requirements	3729

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	3730

A.7.6.13.2.1	Test purpose and environment	3730

A.7.6.13.2.2	Test requirements	3731

A.7.6.14	SA event triggered reporting tests with Pre-MG	3732

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	3732

A.7.6.14.1.1	Test purpose and Environment	3732

A.7.6.14.1.2	Test parameters	3732

A.7.6.14.1.3	Test Requirements	3735

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	3735

A.7.6.14.2.1	Test purpose and Environment	3735

A.7.6.14.2.2	Test parameters	3735

A.7.6.14.2.3	Test Requirements	3738

A.7.6.15	SA event triggered reporting tests with concurrent gaps	3738

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3738

A.7.6.15.1.1	Test Purpose and Environment	3738

A.7.6.15.1.2	Test Requirements	3741

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3741

A.7.6.15.2.1	Test Purpose and Environment	3741

A.7.6.15.2.2	Test Requirements	3744

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement	3744

A.7.6.15.3.1	Test Purpose and Environment	3744

A.7.6.15.3.2	Test Requirements	3748

A.7.6.16	SA event triggered reporting tests with NCSG	3748

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	3748

A.7.6.16.1.1	Test purpose and Environment	3748

A.7.6.16.1.2	Test Requirements	3752

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	3752

A.7.6.16.2.1	Test Purpose and Environment	3752

A.7.6.16.2.2	Test Requirements	3755

A.7.6.16.3	Event triggered reporting test on deactivated Scell measurement via NCSG in FR2 in non-DRX	3755

A.7.6.16.3.1	Test Purpose and Environment	3755

A.7.6.16.3.2	Test Requirements	3758

A.7.7	Measurement Performance requirements	3758

A.7.7.1	SS-RSRP	3759

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3759

A.7.7.1.1.1	Test Purpose and Environment	3759

A.7.7.1.1.2	Test parameters	3759

A.7.7.1.1.3	Test Requirements	3763

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3764

A.7.7.1.2.1	Test Purpose and Environment	3764

A.7.7.1.2.2	Test parameters	3764

A.7.7.1.2.3	Test Requirements	3768

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	3769

A.7.7.1.3.1	Test Purpose and Environment	3769

A.7.7.1.3.2	Test parameters	3769

A.7.7.1.3.3	Test Requirements	3773

A.7.7.2	SS-RSRQ	3773

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3773

A.7.7.2.1.1	Test Purpose and Environment	3773

A.7.7.2.1.2	Test Parameters	3773

A.7.7.2.1.3	Test Requirements	3775

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3775

A.7.7.2.2.1	Test Purpose and Environment	3775

A.7.7.2.2.2	Test Parameters	3775

A.7.7.2.2.3	Test Requirements	3777

A.7.7.3	SS-SINR	3777

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3777

A.7.7.3.1.1	Test Purpose and Environment	3777

A.7.7.3.1.2	Test Parameters	3777

A.7.7.3.1.3	Test Requirements	3779

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3779

A.7.7.3.2.1	Test Purpose and Environment	3779

A.7.7.3.2.2	Test Parameters	3779

A.7.7.3.2.3	Test Requirements	3781

A.7.7.4	L1-RSRP measurement for beam reporting	3781

A.7.7.4.1	SSB based L1-RSRP measurement	3781

A.7.7.4.1.1	Test Purpose and Environment	3781

A.7.7.4.1.2	Test parameters	3782

A.7.7.4.1.3	Test Requirements	3784

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3785

A.7.7.4.2.1	Test Purpose and Environment	3785

A.7.7.4.2.2	Test parameters	3785

A.7.7.4.2.3	Test Requirements	3787

A.7.7.5	CLI measurements	3788

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	3788

A.7.7.5.1.1	Test Purpose and Environment	3788

A.7.7.5.1.2	Test parameters	3788

A.7.7.5.1.3	Test Requirements	3791

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	3792

A.7.7.5.2.1	Test Purpose and Environment	3792

A.7.7.5.2.2	Test parameters	3792

A.7.7.5.2.3	Test Requirements	3794

A.7.7.6	L1-SINR measurement for beam reporting	3795

A.7.7.6.1.1	Test Purpose and Environment	3795

A.7.7.6.1.2	Test parameters	3795

A.7.7.6.1.3	Test Requirements	3797

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3798

A.7.7.6.2.1	Test Purpose and Environment	3798

A.7.7.6.2.2	Test parameters	3798

A.7.7.6.2.3	Test Requirements	3800

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3801

A.7.7.6.3.1	Test Purpose and Environment	3801

A.7.7.6.3.2	Test parameters	3801

A.7.7.6.3.3	Test Requirements	3803

A.7.7.7	CSI-RSRP	3804

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3804

A.7.7.7.1.1	Test Purpose and Environment	3804

A.7.7.7.1.2	Test parameters	3804

A.7.7.7.1.3	Test Requirements	3807

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3808

A.7.7.7.2.1	Test Purpose and Environment	3808

A.7.7.7.2.2	Test parameters	3808

A.7.7.7.2.3	Test Requirements	3811

A.7.7.8	CSI-RSRQ	3812

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3812

A.7.7.8.1.1	Test Purpose and Environment	3812

A.7.7.8.1.2	Test Parameters	3812

A.7.7.8.1.3	Test Requirements	3814

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3814

A.7.7.8.2.1	Test Purpose and Environment	3814

A.7.7.8.2.2	Test Parameters	3814

A.7.7.8.2.3	Test Requirements	3816

A.7.7.9	CSI-SINR	3816

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3816

A.7.7.9.1.1	Test Purpose and Environment	3816

A.7.7.9.1.2	Test Parameters	3816

A.7.7.9.1.3	Test Requirements	3818

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3818

A.7.7.9.2.1	Test Purpose and Environment	3818

A.7.7.9.2.2	Test Parameters	3819

A.7.7.9.2.3	Test Requirements	3821

A.7.7.10	RSTD measurements	3821

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	3821

A.7.7.10.1.1	Test purpose and Environment	3821

A.7.7.10.1.2	Test Requirements	3824

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	3824

A.7.7.10.2.1	Test purpose and Environment	3824

A.7.7.10.2.2	Test Requirements	3828

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3829

A.7.7.10.3.1	Test purpose and Environment	3829

A.7.7.10.3.2	Test Requirements	3830

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	3831

A.7.7.10.4.1	Test purpose and Environment	3831

A.7.7.10.4.2	Test Requirements	3833

A.7.7.11	PRS-RSRP measurements	3833

A.7.7.11.1	SA measurement accuracy with PRS in FR2	3833

A.7.7.11.1.1	Test Purpose and Environment	3833

A.7.7.11.1.2	Test parameters	3833

A.7.7.11.1.3	Test Requirements	3836

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	3837

A.7.7.11.2.1	Test Purpose and Environment	3837

A.7.7.11.2.2	Test parameters	3837

A.7.7.11.2.3	Test Requirements	3839

A.7.7.12	UE Rx-Tx time difference measurements	3840

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	3840

A.7.7.12.1.1	Test purpose and environment	3840

A.7.7.12.1.2	Test parameters	3840

A.7.7.12.1.3	Test requirements	3843

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3844

A.7.7.12.2.1	Test purpose and environment	3844

A.7.7.12.2.2	Test parameters	3844

A.7.7.12.2.3	Test requirements	3847

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3847

A.7.7.12.3.1	Test purpose and environment	3847

A.7.7.12.3.2	Test parameters	3847

A.7.7.12.3.3	Test requirements	3850

A.7.7.13	PRS-RSRPP measurements	3850

A.7.7.13.1	SA measurement accuracy with PRS in FR2	3850

A.7.7.13.1.1	Test Purpose and Environment	3850

A.7.7.13.1.2	Test parameters	3851

A.7.7.13.1.3	Test Requirements	3853

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	3854

A.7.7.13.2.1	Test Purpose and Environment	3854

A.7.7.13.2.2	Test parameters	3854

A.7.7.13.2.3	Test Requirements	3856

A.7.8	Measurement procedure in RRC_INACTIVE	3857

A.7.8.1	RSTD measurements	3857

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	3857

A.7.8.1.1.1	Test Purpose and Environment	3857

A.7.8.1.1.2	Test Requirements	3860

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3860

A.7.8.1.2.1	Test Purpose and Environment	3860

A.7.8.1.2.2	Test Requirements	3866

A.7.8.2	PRS-RSRP measurements	3867

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3867

A.7.8.2.1.1	Test Purpose and Environment	3867

A.7.8.2.1.2	Test Requirements	3871

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3871

A.7.8.2.2.1	Test purpose and Environment	3871

A.7.8.2.2.2	Test Requirements	3875

A.7.8.3	UE Rx-Tx time difference measurements	3875

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3875

A.7.8.3.1.1	Test purpose and environment	3875

A.7.8.3.1.2	Test requirements	3879

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	3879

A.7.8.3.2.1	Test purpose and environment	3879

A.7.8.3.2.2	Test requirements	3881

A.7.8.4	PRS-RSRPP measurements	3882

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3882

A.7.8.4.1.1	Test Purpose and Environment	3882

A.7.8.4.1.2	Test Requirements	3884

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3885

A.7.8.4.2.1	Test Purpose and Environment	3885

A.7.8.4.2.2	Test Requirements	3887

A.7.9	Measurement performance requirements in RRC_INACTIVE	3888

A.7.9.1	RSTD measurements	3888

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3888

A.7.9.1.1.1	Test purpose and Environment	3888

A.7.9.1.1.2	Test Requirements	3890

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3890

A.7.9.1.2.1	Test purpose and Environment	3890

A.7.9.1.2.2	Test Requirements	3892

A.7.9.2	PRS-RSRP measurements	3892

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	3892

A.7.9.2.1.1	Test Purpose and Environment	3892

A.7.9.2.1.2	Test parameters	3892

A.7.9.2.1.3	Test Requirements	3894

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	3895

A.7.9.2.2.1	Test Purpose and Environment	3895

A.7.9.2.2.2	Test parameters	3895

A.7.9.2.2.3	Test Requirements	3897

A.7.9.3	UE Rx-Tx time difference measurements	3898

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	3898

A.7.9.3.1.1	Test purpose and environment	3898

A.7.9.3.1.2	Test parameters	3898

A.7.9.3.1.3	Test requirements	3901

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3901

A.7.9.3.2.1	Test purpose and environment	3901

A.7.9.3.2.2	Test parameters	3901

A.7.9.3.2.3	Test requirements	3904

A.7.9.4	PRS-RSRPP measurements	3904

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	3904

A.7.9.4.1.1	Test Purpose and Environment	3904

A.7.9.4.1.2	Test parameters	3904

A.7.9.4.1.3	Test Requirements	3906

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	3907

A.7.9.4.2.1	Test Purpose and Environment	3907

A.7.9.4.2.2	Test parameters	3907

A.7.9.4.2.3	Test Requirements	3909

A.8	E-UTRA standalone tests for NR RRM	3910

A.8.1	Void	3910

A.8.2	RRC_IDLE state mobility	3910

A.8.2.1	Inter-RAT NR Cell re-selection	3910

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	3910

A.8.2.1.1.1	Test Purpose and Environment	3910

A.8.2.1.1.2	Test Requirements	3915

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	3916

A.8.2.1.2.1	Test Purpose and Environment	3916

A.8.2.1.2.2	Test Requirements	3920

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	3921

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	3921

A.8.2.2.1.1	Test Purpose and Environment	3921

A.8.2.2.1.2	Test Requirements	3925

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	3925

A.8.2.2.2.1	Test Purpose and Environment	3925

A.8.2.2.2.2	Test Requirements	3928

A.8.3	RRC_CONNECTED state mobility	3929

A.8.3.1	Handover	3929

A.8.3.1.1	E-UTRAN - NR handover in FR1	3929

A.8.3.1.1.1	Test Purpose and Environment	3929

A.8.3.1.1.2	Test Requirements	3934

A.8.4	Measurement procedure	3934

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	3934

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	3934

A.8.4.1.1.1	Test Purpose and Environment	3934

A.8.4.1.1.2	Test Requirements	3936

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	3937

A.8.4.1.2.1	Test Purpose and Environment	3937

A.8.4.1.2.2	Test Requirements	3938

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	3938

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3938

A.8.4.2.1.1	Test Purpose and Environment	3938

A.8.4.2.1.2	Test Requirements	3943

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3943

A.8.4.2.2.1	Test Purpose and Environment	3943

A.8.4.2.2.2	Test Requirements	3947

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3947

A.8.4.2.3.1	Test Purpose and Environment	3947

A.8.4.2.3.2	Test Requirements	3951

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3951

A.8.4.2.4.1	Test Purpose and Environment	3951

A.8.4.2.4.2	Test Requirements	3955

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	3955

A.8.4.2.5.1	Test Purpose and Environment	3955

A.8.4.2.5.2	Test Requirements	3957

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	3958

A.8.4.2.6.1	Test Purpose and Environment	3958

A.8.4.2.6.2	Test Requirements	3960

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	3961

A.8.4.2.7.1	Test Purpose and Environment	3961

A.8.4.2.7.2	Test Requirements	3963

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	3964

A.8.4.2.8.1	Test Purpose and Environment	3964

A.8.4.2.8.2	Test Requirements	3966

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	3967

A.8.4.2.9.1	Test Purpose and Environment	3967

A.8.4.2.9.2	Test Requirements	3972

A.8.5	Measurement performance	3972

A.8.5.1	SFTD accuracy	3972

A.8.5.1.1	SFTD accuracy	3972

A.8.5.1.1.1	Test Purpose	3972

A.8.5.1.1.2	Test Environment	3972

A.8.5.1.1.3	Test Requirements	3978

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	3978

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3978

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3983

A.8.5.2.1.2.1	Test Purpose and Environment	3983

A.8.5.2.1.2.2	Test Parameters	3983

A.8.5.2.1.2.3	Test Requirements	3985

A.8.5.2.2	SS-RSRQ	3985

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3985

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3990

A.8.5.2.2.2.1	Test Purpose and Environment	3990

A.8.5.2.2.2.2	Test Parameters	3990

A.8.5.2.2.2.3	Test Requirements	3992

A.8.5.2.3	SS-SINR	3992

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	3992

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	3996

A.8.5.2.3.2.1	Test Purpose and Environment	3996

A.8.5.2.3.2.2	Test Parameters	3996

A.8.5.2.3.2.3	Test Requirements	3998

A.9	V2X Tests	3999

A.9.1	V2X Tests in FR1	3999

A.9.1.1	Test for V2X UE Transmit Timing	3999

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	3999

A.9.1.1.1.1	Test Purpose and Environment	3999

A.9.1.1.1.2	Test requirements	3999

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	3999

A.9.1.1.2.1	Test Purpose and Environment	3999

A.9.1.1.2.2	Test requirements	4000

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	4000

A.9.1.1.3.1	Test Purpose and Environment	4000

A.9.1.1.3.2	Test requirements	4003

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	4003

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	4003

A.9.1.2.1.1	Test Purpose and Environment	4003

A.9.1.2.1.2	Test Requirements	4006

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	4006

A.9.1.2.2.1	Test Purpose and Environment	4006

A.9.1.2.2.2	Test Requirements	4007

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	4008

A.9.1.2.3.1	Test Purpose and Environment	4008

A.9.1.2.3.2	Test Requirements	4010

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	4010

A.9.1.3.1	 Test for GNSS configured as the highest priority	4010

A.9.1.3.1.1	Test Purpose and Environment	4010

A.9.1.3.1.2	Test Requirements	4012

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	4013

A.9.1.3.2.1	Test Purpose and Environment	4013

A.9.1.3.2.2	Test Requirements	4015

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	4016

A.9.1.3.3.1	Test Purpose and Environment	4016

A.9.1.3.3.2	Test Requirements	4018

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	4019

A.9.1.3.4.1	Test Purpose and Environment	4019

A.9.1.3.4.2	Test Requirements	4021

A.9.1.4	Test for L1 SL-RSRP Measurement	4022

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	4022

A.9.1.4.1.1	Test Purpose and Environment	4022

A.9.1.4.1.2	Test Requirements	4025

A.9.1.4.2	Test for V2X UE Resource Pre-emption	4025

A.9.1.4.2.1	Test Purpose and Environment	4025

A.9.1.4.2.2	Test Requirements	4029

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	4029

A.9.1.4.3.1	Test Purpose and Environment	4029

A.9.1.4.3.2	Test Requirements	4036

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	4036

A.9.1.4.4.1	Test Purpose and Environment	4036

A.9.1.4.4.2	Test Requirements	4040

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	4040

A.9.1.4.5.1	Test Purpose and Environment	4040

A.9.1.4.5.2	Test Requirements	4043

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	4043

A.9.1.4.6.1	Test Purpose and Environment	4043

A.9.1.4.6.2	Test Requirements	4047

A.9.1.5	Test for Congestion Control Measurement	4047

A.9.1.5.1	Test Purpose and Environment	4047

A.9.1.5.2	Test Requirements	4053

A.9.1.6	Test for Interruption	4053

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	4053

A.9.1.6.1.1	Test Purpose and Environment	4053

A.9.1.6.1.2	Test Requirements	4057

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	4057

A.9.1.6.2.1	Test Purpose and Environment	4057

A.9.1.6.2.2	Test Requirements	4061

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	4061

A.9.1.6.3.1	Test Purpose and Environment	4061

A.9.1.6.3.2	Test Requirements	4064

A.9.1.7	Selection / Reselection of relay UE	4064

A.9.1.7.1	Test Purpose and Environment	4064

A.9.1.7.2	Test Requirements	4070

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	4071

A.10.1	RRC_CONNECTED state mobility	4071

A.10.1.1	RRC connection mobility control	4071

A.10.1.1.1	Random Access	4071

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	4071

A.10.1.1.1.1.1	Test Purpose and Environment	4071

A.10.1.1.1.1.2	Test Requirements	4074

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	4074

A.10.1.1.1.1.2.2	Random Access Response Reception	4074

A.10.1.1.1.1.2.3	No Random Access Response Reception	4075

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	4075

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	4075

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	4076

A.10.1.1.1.2.1	Test Purpose and Environment	4076

A.10.1.1.1.2.2	Test Requirements	4079

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	4079

A.10.1.1.1.2.2.2	Random Access Response Reception	4080

A.10.1.1.1.2.2.3	No Random Access Response Reception	4080

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	4080

A.10.1.1.1.3.1	Test Purpose and Environment	4080

A.10.1.1.1.3.2	Test Requirements	4084

A.10.1.1.1.3.2.1	MsgA Transmission	4084

A.10.1.1.1.3.2.2	MsgB Reception	4084

A.10.1.1.1.3.2.3	No MsgB Reception	4085

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	4085

A.10.1.1.1.4.1	Test Purpose and Environment	4085

A.10.1.1.1.4.2	Test Requirements	4089

A.10.1.1.1.4.2.1	MsgA Transmission	4089

A.10.1.1.1.4.2.2	MsgB Reception	4090

A.10.1.1.1.4.2.3	No MsgB Reception	4090

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	4090

A.10.1.2.1	Test Purpose and Environment	4090

A.10.1.2.2	Test Requirements	4096

A.10.2	Timing	4097

A.10.2.1	UE transmit timing	4097

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	4097

A.10.2.1.1.1	Test Purpose and environment	4097

A.10.2.1.1.2	Test requirements	4100

A.10.2.2	UE timing advance	4101

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	4101

A.10.2.2.1.1	Test Purpose and Environment	4101

A.10.2.2.1.2	Test Parameters	4101

A.10.2.2.1.3	Test Requirements	4105

A.10.3	Signalling characteristics	4105

A.10.3.1	Radio link monitoring	4105

A.10.3.1.1	Introduction	4105

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4106

A.10.3.1.2.1	Test purpose and environment	4106

A.10.3.1.2.2	Test requirements	4110

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4111

A.10.3.1.3.1	Test purpose and environment	4111

A.10.3.1.3.2	Test requirements	4116

A.10.3.1.4	Void	4116

A.10.3.1.4.1	Void	4116

A.10.3.1.4.2	Void	4116

A.10.3.1.5	Void	4116

A.10.3.1.5.1	Void	4116

A.10.3.1.5.2	Void	4116

A.10.3.2	Void	4116

A.10.3.3	SCell activation and deactivation delay	4116

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	4116

A.10.3.3.1.1	Test Purpose and Environment	4116

A.10.3.3.1.2	Test Requirements	4121

A.10.3.3.2	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	4121

A.10.3.3.2.1	Test Purpose and Environment	4121

A.10.3.3.2.2	Test Requirements	4122

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	4122

A.10.3.3.3.1	Test Purpose and Environment	4122

A.10.3.3.3.2	Test Requirements	4123

A.10.3.4	Beam failure detection and link recovery procedures	4123

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	4123

A.10.3.4.1.1	Test Purpose and Environment	4123

A.10.3.4.1.2	Test Requirements	4128

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	4128

A.10.3.4.2.1	Test Purpose and Environment	4128

A.10.3.4.2.2	Test Requirements	4134

A.10.3.5	Active BWP switching	4134

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	4134

A.10.3.5.1.2	Test Requirements	4139

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	4140

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4140

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	4143

A.10.3.5.3	RRC-based Active BWP Switch	4147

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4147

A.10.3.6	PSCell addition and release delay	4151

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	4151

A.10.3.6.1.1	Test purpose and environment	4151

A.10.3.6.1.2	Test Requirements	4156

A.10.3.7	Void	4157

A.10.4	Measurement procedure	4157

A.10.4.1	Intra-frequency measurements	4157

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	4157

A.10.4.1.1.1	Test purpose and environment	4157

A.10.4.1.1.2	Test parameters	4157

A.10.4.1.1.3	Test Requirements	4160

A.10.4.1.2	Void A.10.4.1.3 Void A.10.4.1.4 Event-triggered reporting tests on PSCC with per-UE gaps under DRX	4160

A.10.4.1.4.1	Test purpose and environment	4160

A.10.4.1.4.2	Test parameters	4160

A.10.4.1.4.3	Test Requirements	4165

A.10.4.1.7	Void	4166

A.10.4.1.8	Void	4166

A.10.4.1.9	Void	4166

A.10.4.1.10	Void	4166

A.10.4.1.11	Void	4166

A.10.4.1.12	Void	4166

A.10.4.2	Inter-frequency measurements	4166

A.10.4.2.1	Void	4166

A.10.4.2.2	Void	4166

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	4166

A.10.4.2.3.1	Test Purpose and Environment	4166

A.10.4.2.3.2	Test Requirements	4170

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	4171

A.10.4.2.4.1	Test Purpose and Environment	4171

A.10.4.2.4.2	Test Requirements	4175

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	4176

A.10.4.2.5.1	Test Purpose and Environment	4176

A.10.4.2.5.2	Test Requirements	4180

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	4181

A.10.4.2.6.1	Test Purpose and Environment	4181

A.10.4.2.6.2	Test Requirements	4185

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	4186

A.10.4.2.7.1	Test Purpose and Environment	4186

A.10.4.2.7.2	Test Requirements	4192

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4192

A.10.4.2.8.1	Test Purpose and Environment	4192

A.10.4.2.8.2	Test Requirements	4198

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4199

A.10.4.2.9.1	Test Purpose and Environment	4199

A.10.4.2.9.2	Test Requirements	4204

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4204

A.10.4.2.10.1	Test Purpose and Environment	4204

A.10.4.2.10.2	Test Requirements	4210

A.10.4.3	L1-RSRP measurements for beam reporting	4211

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4211

A.10.4.3.1.1	Test Purpose and Environment	4211

A.10.4.3.1.2	Test parameters	4211

A.10.4.3.1.3	Test Requirements	4213

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4214

A.10.4.3.2.1	Test Purpose and Environment	4214

A.10.4.3.2.2	Test parameters	4214

A.10.4.3.2.3	Test Requirements	4216

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4217

A.10.4.3.3.1	Test Purpose and Environment	4217

A.10.4.3.3.2	Test parameters	4217

A.10.4.3.3.3	Test Requirements	4220

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4221

A.10.4.3.4.1	Test Purpose and Environment	4221

A.10.4.3.4.2	Test parameters	4221

A.10.4.3.4.3	Test Requirements	4224

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4225

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4225

A.10.4.4.1.1	Test Purpose and Environment	4225

A.10.4.4.1.2	Test Requirements	4229

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4230

A.10.4.4.2.1	Test Purpose and Environment	4230

A.10.4.4.2.2	Test Requirements	4235

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4236

A.10.4.4.3.1	Test Purpose and Environment	4236

A.10.4.4.3.2	Test Requirements	4240

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4241

A.10.4.4.4.1	Test Purpose and Environment	4241

A.10.4.4.4.2	Test Requirements	4246

A.10.5	Measurement performance	4247

A.10.5.1	SS-RSRP	4247

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4247

A.10.5.1.1.1	Test Purpose and Environment	4247

A.10.5.1.1.2	Test parameters	4247

A.10.5.1.1.3	Test Requirements	4250

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4250

A.10.5.1.2.1	Test Purpose and Environment	4250

A.10.5.1.2.2	Test parameters	4250

A.10.5.1.2.3	Test Requirements	4253

A.10.5.2	SS-RSRQ	4253

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4253

A.10.5.2.1.1	Test Purpose and Environment	4253

A.10.5.2.1.2	Test Parameters	4253

A.10.5.2.1.3	Test Requirements	4256

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4256

A.10.5.2.2.1	Test Purpose and Environment	4256

A.10.5.2.2.2	Test Parameters	4256

A.10.5.2.2.3	Test Requirements	4259

A.10.5.3	SS-SINR	4259

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4259

A.10.5.3.1.1	Test Purpose and Environment	4259

A.10.5.3.1.2	Test Parameters	4259

A.10.5.3.1.3	Test Requirements	4262

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4262

A.10.5.3.2.1	Test Purpose and Environment	4262

A.10.5.3.2.2	Test Parameters	4262

A.10.5.3.2.3	Test Requirements	4265

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4265

A.10.5.3.3.1	Test Purpose and Environment	4265

A.10.5.3.3.2	Test Parameters	4265

A.10.5.3.3.3	Test Requirements	4268

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4268

A.10.5.4.1	SSB based L1-RSRP measurement	4268

A.10.5.4.1.1	Test Purpose and Environment	4268

A.10.5.4.1.2	Test parameters	4269

A.10.5.4.1.3	Test Requirements	4271

A.10.5.5	RSSI	4271

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4271

A.10.5.5.1.1	Test Purpose and Environment	4271

A.10.5.5.1.2	Test parameters	4271

A.10.5.5.1.3	Test Requirements	4274

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	4274

A.10.5.5.2.1	Test Purpose and Environment	4274

A.10.5.5.2.2	Test parameters	4274

A.10.5.5.2.3	Test Requirements	4277

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4277

A.10.5.5.3.1	Test Purpose and Environment	4277

A.10.5.5.3.2	Test parameters	4277

A.10.5.5.3.3	Test Requirements	4281

A.10.5.6	Channel occupancy	4281

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4281

A.10.5.6.1.1	Test Purpose and Environment	4281

A.10.5.6.1.2	Test parameters	4281

A.10.5.6.1.3	Test Requirements	4285

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4285

A.10.5.6.2.1	Test Purpose and Environment	4285

A.10.5.6.2.2	Test parameters	4285

A.10.5.6.2.3	Test Requirements	4288

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4288

A.10.5.6.3.1	Test Purpose and Environment	4288

A.10.5.6.3.2	Test parameters	4288

A.10.5.6.3.3	Test Requirements	4292

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4294

A.11.1	RRC_IDLE state mobility	4294

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4294

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4294

A.11.1.1.1.1	Test Purpose and Environment	4294

A.11.1.1.1.2	Test Parameters	4294

A.11.1.1.1.3	Test Requirements	4297

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4297

A.11.1.1.2.1	Test Purpose and Environment	4297

A.11.1.1.2.2	Test Parameters	4297

A.11.1.1.2.3	Test Requirements	4301

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4301

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4301

A.11.1.2.1.1	Test Purpose and Environment	4301

A.11.1.2.1.2	Test Parameters	4302

A.11.1.2.1.3	Test Requirements	4308

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4309

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4309

A.11.1.3.1.1	Test Purpose and Environment	4309

A.11.1.3.1.2	Test Parameters	4309

A.11.1.3.1.3	Test Requirements	4315

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4316

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4316

A.11.1.4.1.1	Test Purpose and Environment	4316

A.11.1.4.1.2	Test Parameters	4316

A.11.1.4.1.3	Test Requirements	4319

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4320

A.11.1.4.2.1	Test Purpose and Environment	4320

A.11.1.4.2.2	Test Requirements	4323

A.11.2	RRC_CONNECTED state mobility	4324

A.11.2.1	Handover	4324

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4324

A.11.2.1.1.1	Test Purpose and Environment	4324

A.11.2.1.1.2	Test Parameters	4324

A.11.2.1.1.3 Test Requirements	4327

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4327

A.11.2.1.2.1	Test Purpose and Environment	4327

A.11.2.1.2.2	Test Parameters	4327

A.11.2.1.2.3	Test Requirements	4330

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4330

A.11.2.1.3.1	Test Purpose and Environment	4330

A.11.2.1.3.2	Test Parameters	4330

A.11.2.1.3.3 Test Requirements	4333

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4333

A.11.2.1.4.1	Test Purpose and Environment	4333

A.11.2.1.4.2	Test Parameters	4333

A.11.2.1.4.3 Test Requirements	4337

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4338

A.11.2.1.5.1	Test Purpose and Environment	4338

A.11.2.1.5.2	Test Parameters	4338

A.11.2.1.5.3 Test Requirements	4341

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4342

A.11.2.1.6.1	Test Purpose and Environment	4342

A.11.2.1.6.2	Test Parameters	4342

A.11.2.1.6.3 Test Requirements	4345

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4346

A.11.2.1.7.1	Test Purpose and Environment	4346

A.11.2.1.7.2	Test Requirements	4352

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4352

A.11.2.1.8.1	Test Purpose and Environment	4352

A.11.2.1.8.2	Test Requirements	4357

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4357

A.11.2.1.9.1	Test Purpose and Environment	4357

A.11.2.1.9.2	Test Requirements	4366

A.11.2.2	RRC connection mobility control	4367

A.11.2.2.1	RRC re-establishment	4367

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4367

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4370

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4377

A.11.2.2.2	Random Access	4382

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4382

A.11.2.2.2.1.1	Test Purpose and Environment	4382

A.11.2.2.2.1.2	Test Requirements	4385

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4385

A.11.2.2.2.1.2.2	Random Access Response Reception	4385

A.11.2.2.2.1.2.3	No Random Access Response Reception	4386

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4386

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4386

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4387

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4387

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4387

A.11.2.2.2.2.1	Test Purpose and Environment	4387

A.11.2.2.2.2.2	Test Requirements	4390

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4390

A.11.2.2.2.2.2.2	Random Access Response Reception	4391

A.11.2.2.2.2.2.3	No Random Access Response Reception	4391

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4391

A.11.2.2.2.3.1	Test Purpose and Environment	4391

A.11.2.2.2.3.2	Test Requirements	4395

A.11.2.2.2.3.2.1	MsgA Transmission	4395

A.11.2.2.2.3.2.2	MsgB Reception	4395

A.11.2.2.2.3.2.3	No MsgB Reception	4396

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4396

A.11.2.2.2.4.1	Test Purpose and Environment	4396

A.11.2.2.2.4.2	Test Requirements	4400

A.11.2.2.2.4.2.1	MsgA Transmission	4400

A.11.2.2.2.4.2.2	MsgB Reception	4401

A.11.2.2.2.4.2.3	No MsgB Reception	4401

A.11.2.2.3	RRC connection release with redirection	4402

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4402

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4406

A.11.3	Timing	4411

A.11.3.1	UE transmit timing	4411

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4411

A.11.3.1.1.1	Test Purpose and environment	4411

A.11.3.1.1.2	Test requirements	4414

A.11.3.2	UE timing advance	4415

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4415

A.11.3.2.1.1	Test Purpose and Environment	4415

A.11.3.2.1.2	Test Parameters	4415

A.11.3.2.1.3	Test Requirements	4419

A.11.4	Signalling characteristics	4419

A.11.4.1	Radio link monitoring	4419

A.11.4.1.1	Introduction	4419

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4420

A.11.4.1.2.1	Test purpose and environment	4420

A.11.4.1.2.2	Test requirements	4424

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4424

A.11.4.1.3.1	Test purpose and environment	4424

A.11.4.1.3.2	Test requirements	4430

A.11.4.1.4	Void	4431

A.11.4.1.4.1	Void	4431

A.11.4.1.4.2	Void	4431

A.11.4.1.5	Void	4431

A.11.4.1.5.1	Void	4431

A.11.4.1.5.2	Void	4431

A.11.4.2	Void	4431

A.11.4.3	SCell activation and deactivation delay	4431

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4431

A.11.4.3.1.1	Test Purpose and Environment	4431

A.11.4.3.1.2	Test Requirements	4435

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4435

A.11.4.3.2.1	Test Purpose and Environment	4435

A.11.4.3.2.2	Test Requirements	4436

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4436

A.11.4.3.3.1	Test Purpose and Environment	4436

A.11.4.3.3.2	Test Requirements	4437

A.11.4.4	Beam failure detection and link recovery procedures	4437

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4437

A.11.4.4.1.1	Test Purpose and Environment	4437

A.11.4.4.1.2	Test Requirements	4442

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4442

A.11.4.4.2.1	Test Purpose and Environment	4442

A.11.4.4.2.2	Test Requirements	4448

A.11.4.5	Active BWP switching	4448

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4448

A.11.4.5.1.1	Test Purpose and Environment	4448

A.11.4.5.1.2	Test Requirements	4452

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4453

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4453

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4456

A.11.4.5.3	RRC-based Active BWP Switch	4459

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4459

A.11.4.6	Void	4462

A.11.5	Measurement procedure	4462

A.11.5.1	Intra-frequency measurements	4462

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4462

A.11.5.1.1.1	Test purpose and environment	4462

A.11.5.1.1.2	Test parameters	4462

A.11.5.1.1.3	Test Requirements	4465

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4465

A.11.5.1.2.1	Test purpose and environment	4465

A.11.5.1.2.2	Test parameters	4465

A.11.5.1.2.3	Test Requirements	4468

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	4468

A.11.5.1.3.1	Test purpose and environment	4468

A.11.5.1.3.2	Test parameters	4468

A.11.5.1.3.3	Test Requirements	4468

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	4469

A.11.5.1.4.1	Test purpose and environment	4469

A.11.5.1.4.2	Test parameters	4469

A.11.5.1.4.3	Test Requirements	4469

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4469

A.11.5.1.5.1	Test purpose and environment	4469

A.11.5.1.5.2	Test parameters	4469

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4470

A.11.5.1.6.1	Test purpose and environment	4470

A.11.5.1.6.2	Test parameters	4470

A.11.5.1.6.3	Test Requirements	4470

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4470

A.11.5.1.7.1	Test purpose and environment	4470

A.11.5.1.7.2	Test parameters	4470

A.11.5.1.7.3	Test Requirements	4470

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4471

A.11.5.1.8.1	Test purpose and environment	4471

A.11.5.1.8.2	Test parameters	4471

A.11.5.1.8.3	Test Requirements	4471

A.11.5.1.9	Void	4471

A.11.5.1.10	Void	4471

A.11.5.1.11	Void	4471

A.11.5.1.12	Void	4471

A.11.5.2	Inter-frequency measurements	4471

A.11.5.2.1	Void	4471

A.11.5.2.2	Void	4471

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4471

A.11.5.2.3.1	Test Purpose and Environment	4471

A.11.5.2.3.2	Test Requirements	4475

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4476

A.11.5.2.4.1	Test Purpose and Environment	4476

A.11.5.2.4.2	Test Requirements	4481

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4481

A.11.5.2.5.1	Test Purpose and Environment	4481

A.11.5.2.5.2	Test Requirements	4485

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4486

A.11.5.2.6.1	Test Purpose and Environment	4486

A.11.5.2.6.2	Test Requirements	4491

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4491

A.11.5.2.7.1	Test Purpose and Environment	4491

A.11.5.2.7.2	Test Requirements	4495

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4496

A.11.5.2.8.1	Test Purpose and Environment	4496

A.11.5.2.8.2	Test Requirements	4501

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4501

A.11.5.2.9.1	Test Purpose and Environment	4501

A.11.5.2.9.2	Test Requirements	4506

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4507

A.11.5.2.10.1	Test Purpose and Environment	4507

A.11.5.2.10.2	Test Requirements	4512

A.11.5.3	Inter-RAT E-UTRAN measurements	4513

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	4513

A.11.5.3.1.1	Test Purpose and Environment	4513

A.11.5.3.1.2	Test Requirements	4517

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	4517

A.11.5.3.2.1	Test Purpose and Environment	4517

A.11.5.3.2.2	Test Requirements	4522

A.11.5.4	L1-RSRP measurements for beam reporting	4522

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4522

A.11.5.4.1.1	Test Purpose and Environment	4522

A.11.5.4.1.2	Test parameters	4522

A.11.5.4.1.3	Test Requirements	4524

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4524

A.11.5.4.2.1	Test Purpose and Environment	4524

A.11.5.4.2.2	Test parameters	4525

A.11.5.4.2.3	Test Requirements	4527

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4527

A.11.5.4.3.1	Test Purpose and Environment	4527

A.11.5.4.3.2	Test parameters	4528

A.11.5.4.3.3	Test Requirements	4531

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4532

A.11.5.4.4.1	Test Purpose and Environment	4532

A.11.5.4.4.2	Test parameters	4532

A.11.5.4.4.3	Test Requirements	4535

A.11.6	Measurement performance	4536

A.11.6.1	SS-RSRP	4536

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4536

A.11.6.1.1.1	Test Purpose and Environment	4536

A.11.6.1.1.2	Test parameters	4536

A.11.6.1.1.3	Test Requirements	4538

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4538

A.11.6.1.2.1	Test Purpose and Environment	4538

A.11.6.1.2.2	Test parameters	4538

A.11.6.1.2.3	Test Requirements	4540

A.11.6.2	SS-RSRQ	4540

A.11.6.2.1	Intra-frequency measurement accuracy	4540

A.11.6.2.1.1	Test Purpose and Environment	4540

A.11.6.2.1.2	Test Parameters	4540

A.11.6.2.1.3	Test Requirements	4543

A.11.6.2.2	Inter-frequency measurement accuracy	4543

A.11.6.2.2.1	Test Purpose and Environment	4543

A.11.6.2.2.2	Test Parameters	4543

A.11.6.2.2.3	Test Requirements	4546

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4546

A.11.6.2.3.1	Test Purpose and Environment	4546

A.11.6.2.3.2	Test Parameters	4546

A.11.6.2.3.3	Test Requirements	4549

A.11.6.2.4	Inter-frequency measurement accuracy	4549

A.11.6.2.4.1	Test Purpose and Environment	4549

A.11.6.2.4.2	Test Parameters	4549

A.11.6.2.4.3	Test Requirements	4558

A.11.6.3	SS-SINR	4558

A.11.6.3.1	Intra-frequency measurement accuracy	4558

A.11.6.3.1.1	Test Purpose and Environment	4558

A.11.6.3.1.2	Test Parameters	4558

A.11.6.3.1.3	Test Requirements	4561

A.11.6.3.2	Inter-frequency measurement accuracy	4561

A.11.6.3.2.1	Test Purpose and Environment	4561

A.11.6.3.2.2	Test Parameters	4561

A.11.6.3.2.3	Test Requirements	4564

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4564

A.11.6.3.3.1	Test Purpose and Environment	4564

A.11.6.3.3.2	Test Parameters	4564

A.11.6.3.3.3	Test Requirements	4567

A.11.6.3.4	Inter-frequency measurement accuracy	4567

A.11.6.3.4.1	Test Purpose and Environment	4567

A.11.6.3.4.2	Test Parameters	4567

A.11.6.3.4.3	Test Requirements	4575

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4575

A.11.6.4.1	SSB based L1-RSRP measurement	4575

A.11.6.4.1.1	Test Purpose and Environment	4575

A.11.6.4.1.2	Test parameters	4576

A.11.6.4.1.3	Test Requirements	4579

A.11.6.5	RSSI	4579

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	4579

A.11.6.5.1.1	Test Purpose and Environment	4579

A.11.6.5.1.2	Test parameters	4579

A.11.6.5.1.3	Test Requirements	4582

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	4582

A.11.6.5.2.1	Test Purpose and Environment	4582

A.11.6.5.2.2	Test parameters	4582

A.11.6.5.2.3	Test Requirements	4585

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4585

A.11.6.5.3.1	Test Purpose and Environment	4585

A.11.6.5.3.2	Test parameters	4585

A.11.6.5.3.3	Test Requirements	4588

A.11.6.6	Channel occupancy	4588

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4588

A.11.6.6.1.1	Test Purpose and Environment	4588

A.11.6.6.1.2	Test parameters	4588

A.11.6.6.1.3	Test Requirements	4591

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4591

A.11.6.6.2.1	Test Purpose and Environment	4591

A.11.6.6.2.2	Test parameters	4591

A.11.6.6.2.3	Test Requirements	4594

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4594

A.11.6.6.3.1	Test Purpose and Environment	4594

A.11.6.6.3.2	Test parameters	4594

A.11.6.6.3.3	Test Requirements	4597

A.11.6.7	E-UTRAN RSRP	4597

A.11.6.8	E-UTRAN RSRQ	4597

A.11.6.9	E-UTRAN SINR	4597

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4597

A.12.1	RRC_IDLE state mobility	4597

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4597

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4597

A.12.1.1.1.1	Test Purpose and Environment	4597

A.12.1.1.1.2	Test Requirements	4602

A.12.2	RRC_CONNECTED state mobility	4603

A.12.2.1	Handover	4603

A.12.2.1.1	E-UTRAN - NR with CCA handover	4603

A.12.2.1.1.1	Test Purpose and Environment	4603

A.12.2.1.1.2	Test Requirements	4608

A.12.3	Void	4609

A.12.4	Measurement procedure	4609

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4609

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4609

A.12.4.1.1.1	Test Purpose and Environment	4609

A.12.4.1.1.2	Test Requirements	4613

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4613

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4613

A.12.4.2.1.1	Test Purpose and Environment	4613

A.12.4.2.1.2	Test Requirements	4617

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4618

A.12.4.2.2.1	Test Purpose and Environment	4618

A.12.4.2.2.2	Test Requirements	4622

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4623

A.12.4.2.3.1	Test Purpose and Environment	4623

A.12.4.2.3.2	Test Requirements	4627

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4628

A.12.4.2.4.1	Test Purpose and Environment	4628

A.12.4.2.4.2	Test Requirements	4632

A.12.4.2.5	RSSI measurement reporting	4633

A.12.4.2.5.1	Test purpose and environment	4633

A.12.4.2.5.2	Test parameters	4633

A.12.4.2.5.3	Test Requirements	4636

A.12.4.2.6	Channel occupancy measurement reporting	4636

A.12.4.2.6.1	Test purpose and environment	4636

A.12.4.2.6.2	Test parameters	4636

A.12.5	Measurement performance	4639

A.12.5.1	E-UTRANNR SFTD	4639

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4639

A.12.5.1.1.1	Test Purpose	4639

A.12.5.1.1.2	Test Environment	4639

A.12.5.1.1.3	Test Requirements	4644

A.12.5.2	E-UTRANNR SS-RSRP	4644

A.12.5.3	E-UTRANNR SS-RSRQ	4644

A.12.5.4	E-UTRANNR SS-SINR	4644

A.12.5.5	E-UTRANNR RSSI	4644

A.12.5.6	E-UTRANNR channel occupancy	4644

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4660

A.13.1	Void	4660

A.13.2	Signalling characteristics	4660

A.13.2.1	Void	4660

A.13.2.2	SCell activation and deactivation delay	4660

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4660

A.13.2.2.1.1	Test Purpose and Environment	4660

A.13.2.2.1.2	Test Requirements	4664

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4664

A.13.2.2.2.1	Test Purpose and Environment	4664

A.13.2.2.2.2	Test Requirements	4665

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4665

A.13.2.2.3.1	Test Purpose and Environment	4665

A.13.2.2.3.2	Test Requirements	4666

A.13.2.3	Void	4666

A.13.3	Measurement procedure	4666

A.13.3.1	Intra-frequency measurements	4666

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4666

A.13.3.1.1.1	Test purpose and environment	4666

A.13.3.1.1.2	Test parameters	4666

A.13.3.1.1.3	Test Requirements	4671

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4671

A.13.3.1.2.1	Test purpose and environment	4671

A.13.3.1.2.2	Test parameters	4671

A.13.3.1.2.3	Test Requirements	4676

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4676

A.13.3.1.3.1	Test purpose and environment	4676

A.13.3.1.3.2	Test parameters	4676

A.13.3.1.3.3	Test Requirements	4681

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4681

A.13.3.1.4.1	Test purpose and environment	4681

A.13.3.1.4.2	Test parameters	4681

A.13.3.1.4.3	Test Requirements	4686

A.13.3.1.5	Void	4686

A.13.3.1.6	Void	4686

A.13.3.2	Inter-frequency measurements	4686

A.13.3.2.1	Void	4686

A.13.3.2.2	Void	4686

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4686

A.13.3.2.3.1	Test Purpose and Environment	4686

A.13.3.2.3.2	Test Requirements	4691

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4691

A.13.3.2.4.1	Test Purpose and Environment	4691

A.13.3.2.4.2	Test Requirements	4696

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4697

A.13.3.2.5.1	Test Purpose and Environment	4697

A.13.3.2.5.2	Test Requirements	4702

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4702

A.13.3.2.6.1	Test Purpose and Environment	4702

A.13.3.2.6.2	Test Requirements	4708

A.13.3.3	L1-RSRP measurements for beam reporting	4709

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	4709

A.13.3.3.1.1	Test Purpose and Environment	4709

A.13.3.3.1.2	Test parameters	4709

A.13.3.3.1.3	Test Requirements	4713

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	4713

A.13.3.3.2.1	Test Purpose and Environment	4713

A.13.3.3.2.2	Test parameters	4713

A.13.3.3.2.3	Test Requirements	4717

A.13.4	Measurement performance	4717

A.13.4.1	SS-RSRP	4717

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4717

A.13.4.1.1.1	Test Purpose and Environment	4717

A.13.4.1.1.2	Test parameters	4717

A.13.4.1.1.3	Test Requirements	4719

A.13.4.2	SS-RSRQ	4719

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	4719

A.13.4.2.1.1	Test Purpose and Environment	4719

A.13.4.2.1.2	Test Parameters	4719

A.13.4.2.1.3	Test Requirements	4726

A.13.4.3	SS-SINR	4727

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	4727

A.13.4.3.1.1	Test Purpose and Environment	4727

A.13.4.3.1.2	Test Parameters	4727

A.13.4.3.1.3	Test Requirements	4734

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4734

A.13.4.4.1	SSB based L1-RSRP measurement	4734

A.13.4.4.1.1	Test Purpose and Environment	4734

A.13.4.4.1.2	Test parameters	4734

A.13.4.4.1.3	Test Requirements	4738

A.13.4.5	RSSI	4738

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	4738

A.13.4.5.1.1	Test Purpose and Environment	4738

A.13.4.5.1.2	Test parameters	4738

A.13.4.5.1.3	Test Requirements	4741

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4741

A.13.4.5.2.1	Test Purpose and Environment	4741

A.13.4.5.2.2	Test parameters	4741

A.13.4.5.2.3	Test Requirements	4745

A.13.4.6	Channel occupancy	4745

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4745

A.13.4.6.1.1	Test Purpose and Environment	4745

A.13.4.6.1.2	Test parameters	4745

A.13.4.6.1.3	Test Requirements	4749

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4749

A.13.4.6.2.1	Test Purpose and Environment	4749

A.13.4.6.2.2	Test parameters	4749

A.13.4.6.2.3	Test Requirements	4753

A.14	NR standalone tests for Satellite access	4753

A.14.1	RRC_IDLE state mobility	4753

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	4753

A.14.1.1.1	Test Purpose and Environment	4753

A.14.1.1.2	Test Parameters	4753

A.14.1.1.3	Test Requirements	4755

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements	4756

A.14.1.2.1	Test Purpose and Environment	4756

A.14.1.2.2	Test Parameters	4756

A.14.1.2.3	Test Requirements	4760

A.14.1.3	Time-based measurement initiation to FR1 intra-frequency NR cell reselection	4761

A.14.1.3.1	Test Purpose and Environment	4761

A.14.1.3.2	Test Parameters	4761

A.14.1.3.3	Test Requirements	4762

A.14.1.4	Location-based cell measurement initiation to FR1 intra-frequency NR cell reselection	4763

A.14.1.4.1	Test Purpose and Environment	4763

A.14.1.4.2	Test Parameters	4763

A.14.1.4.3	Test Requirements	4765

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	4766

A.14.1.5.1	Test Purpose and Environment	4766

A.14.1.5.2	Test Parameters	4766

A.14.1.5.3	Test Requirements	4768

A.14.1.6	Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements]	4769

A.14.1.6.1	Test Purpose and Environment	4769

A.14.1.6.2	Test Parameters	4769

A.14.1.6.3	Test Requirements	4771

A.14.1.7	Time-based Cell measurement initiation to FR1 inter-frequency cell reselection	4772

A.14.1.7.1	Test Purpose and Environment	4772

A.14.1.7.2	Test Parameters	4772

A.14.1.7.3	Test Requirements	4773

A.14.1.8	Location-based measurement initiation to FR1 inter-frequency NR cell reselection	4774

A.14.1.8.1	Test Purpose and Environment	4774

A.14.1.8.2	Test Parameters	4774

A.14.1.8.3	Test Requirements	4776

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4777

A.14.1.9.1	Test Purpose and Environment	4777

A.14.1.9.2	Test Parameters	4777

A.14.1.9.3	Test Requirements	4780

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4780

A.14.1.10.1	Test Purpose and Environment	4780

A.14.1.10.2	Test Parameters	4780

A.14.1.10.3	Test Requirements	4782

A.14.2	RRC_CONNECTED state mobility	4783

A.14.2.1	Handover	4783

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	4783

A.14.2.1.1.1	Test Purpose and Environment	4783

A.14.2.1.1.2	Test Parameters	4783

A.14.2.1.1.3	Test Requirements	4785

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	4786

A.14.2.1.2.1	Test Purpose and Environment	4786

A.14.2.1.2.2	Test Parameters	4786

A.14.2.1.2.3	Test Requirements	4788

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	4788

A.14.2.1.3.1	Test Purpose and Environment	4788

A.14.2.1.3.2	Test Parameters	4788

A.14.2.1.3.3	Test Requirements	4791

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	4791

A.14.2.1.4.1	Test Purpose and Environment	4791

A.14.2.1.4.2	Test Parameters	4791

A.14.2.1.4.3	Test Requirements	4793

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	4793

A.14.2.1.5.1	Test Purpose and Environment	4793

A.14.2.1.5.2	Test Parameters	4794

A.14.2.1.5.3	Test Requirements	4797

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	4797

A.14.2.1.6.1	Test Purpose and Environment	4797

A.14.2.1.6.2	Test Parameters	4797

A.14.2.1.6.3	Test Requirements	4801

A.14.2.2	RRC Connection Mobility Control	4801

A.14.2.2.1	SA: RRC Re-establishment for SAN	4801

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	4801

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	4804

A.14.2.2.2	Random Access	4807

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	4807

A.14.2.2.2.1.1	Test Purpose and Environment	4807

A.14.2.2.2.1.2	Test Requirements	4810

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	4811

A.14.2.2.2.2.1	Test Purpose and Environment	4811

A.14.2.2.2.2.2	Test Requirements	4815

A.14.2.2.3	RRC Connection Release with Redirection	4816

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	4816

A.14.2.2.3.1.1	Test Purpose and Environment	4816

A.14.2.2.3.1.2	Test Parameters	4816

A.14.2.2.3.1.3	Test Requirements	4819

A.14.3	Timing for Satellite Access	4819

A.14.3.1	UE transmit timing for Satellite Access	4819

A.14.3.1.1	NR UE Transmit Timing Test for FR1	4819

A.14.3.1.1.1	Test Purpose and environment	4819

A.14.3.1.1.2	Test requirements	4821

A.14.3.2	Timing advance for satellite access	4822

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	4822

A.14.3.2.1.1	Test Purpose and Environment	4822

A.14.3.2.1.2	Test Parameters	4822

A.14.3.2.1.3	Test Requirements	4825

A.14.4	Signalling characteristics	4825

A.14.4.1	Radio link Monitoring	4825

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4825

A.14.4.1.1.1	Test Purpose and Environment	4825

A.14.4.1.1.2	Test Requirements	4830

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4830

A.14.4.1.2.1	Test Purpose and Environment	4830

A.14.4.1.2.2	Test Requirements	4836

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4836

A.14.4.1.3.1	Test Purpose and Environment	4836

A.14.4.1.3.2	Test Requirements	4840

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4840

A.14.4.1.4.1	Test Purpose and Environment	4840

A.14.4.1.4.2	Test Requirements	4845

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4845

A.14.4.1.5.1	Test Purpose and Environment	4845

A.14.4.1.5.2	Test Requirements	4850

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4850

A.14.4.1.6.1	Test Purpose and Environment	4850

A.14.4.1.6.2	Test Requirements	4854

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4854

A.14.4.1.7.1	Test Purpose and Environment	4854

A.14.4.1.7.2	Test Requirements	4859

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4859

A.14.4.1.8.1	Test Purpose and Environment	4859

A.14.4.1.8.2	Test Requirements	4864

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	4864

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	4864

A.14.4.2.1.1	Test Purpose and Environment	4864

A.14.4.2.1.2	Test Requirements	4870

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	4870

A.14.4.2.2.1	Test Purpose and Environment	4870

A.14.4.2.2.2	Test Requirements	4875

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	4875

A.14.4.2.3.1	Test Purpose and Environment	4875

A.14.4.2.3.2	Test Requirements	4881

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	4881

A.14.4.2.4.1	Test Purpose and Environment	4881

A.14.4.2.4.2	Test Requirements	4886

A.14.4.2.5	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	4886

A.14.4.2.5.1	Test Purpose and Environment	4886

A.14.4.2.5.2	Test Requirements	4891

A.14.4.2.6	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in DRX mode	4891

A.14.4.2.6.1	Test Purpose and Environment	4891

A.14.4.2.6.2	Test Requirements	4896

A.14.4.3	Active BWP switch for satellite access	4896

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	4896

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	4896

A.14.4.3.1.1.1	Test Purpose and Environment	4896

A.14.4.3.1.1.2	Test Requirements	4901

A.14.4.3.2	RRC-based Active BWP Switch	4901

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4901

A.14.4.3.2.1.1	Test Purpose and Environment	4901

A.14.4.3.2.1.2	Test Requirements	4905

A.14.4.4	UE specific CBW change for satellite access	4905

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	4905

A.14.4.4.1.1	Test Purpose and Environment	4905

A.14.4.4.1.2	Test Requirements	4909

A.14.4.5	Pathloss reference signal switching delay	4909

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	4909

A.14.4.5.1.1	Test Purpose and Environment	4909

A.14.4.5.1.2	Test Requirements	4911

A.14.5	Measurement procedure	4912

A.14.5.1	Intra-frequency Measurements	4912

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	4912

A.14.5.1.1.1	Test purpose and Environment	4912

A.14.5.1.1.2	Test parameters	4912

A.14.5.1.1.3	Test Requirements	4914

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	4915

A.14.5.1.2.1	Test purpose and Environment	4915

A.14.5.1.2.2	Test parameters	4915

A.14.5.1.2.3	Test Requirements	4916

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	4917

A.14.5.1.3.1	Test purpose and Environment	4917

A.14.5.1.3.2	Test parameters	4917

A.14.5.1.3.3	Test Requirements	4919

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	4920

A.14.5.1.4.1	Test purpose and Environment	4920

A.14.5.1.4.2	Test parameters	4920

A.14.5.1.4.3	Test Requirements	4922

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	4923

A.14.5.1.5.1	Test purpose and Environment	4923

A.14.5.1.5.2	Test parameters	4923

A.15.5.1.5.3	Test Requirements	4925

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	4926

A.14.5.1.6.1	Test purpose and Environment	4926

A.14.5.1.6.2	Test parameters	4926

A.14.5.1.6.3	Test Requirements	4928

A.14.5.2	Inter-frequency Measurements	4929

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	4929

A.14.5.2.1.1	Test Purpose and Environment	4929

A.14.5.2.1.2	Test Requirements	4933

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	4933

A.14.5.2.2.1	Test Purpose and Environment	4933

A.14.5.2.2.2	Test Requirements	4937

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	4937

A.14.5.2.3.1	Test Purpose and Environment	4937

A.14.5.2.3.2	Test Requirements	4941

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	4941

A.14.5.2.4.1	Test Purpose and Environment	4941

A.14.5.2.4.2	Test Requirements	4945

A.14.5.2.5	void	4945

A.14.5.2.5.1	void	4945

A.14.5.2.5.2	void	4945

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	4945

A.14.5.2.6.1	Test Purpose and Environment	4945

A.14.5.2.6.2	Test Requirements	4949

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	4949

A.14.5.2.7.1	Test purpose and Environment	4949

A.14.5.2.7.2	Test parameters	4949

A.14.5.2.7.3	Test Requirements	4951

A.14.5.2.8	Event triggered reporting tests without gap under DRX	4952

A.14.5.2.8.1	Test purpose and Environment	4952

A.14.5.2.8.2	Test parameters	4952

A.14.5.2.8.3	Test Requirements	4953

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	4954

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	4954

A.14.5.3.1.1	Test Purpose and Environment	4954

A.14.5.3.1.2	Test parameters	4954

A.14.5.3.1.3	Test Requirements	4957

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	4957

A.14.5.3.2.1	Test Purpose and Environment	4957

A.14.5.3.2.2	Test parameters	4957

A.14.5.3.2.3	Test Requirements	4961

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	4961

A.14.5.3.3.1	Test Purpose and Environment	4961

A.14.5.3.3.2	Test parameters	4961

A.14.5.3.3.3	Test Requirements	4965

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	4965

A.14.5.3.4.1	Test Purpose and Environment	4965

A.14.5.3.4.2	Test parameters	4966

A.14.5.3.4.3	Test Requirements	4969

A.14.6	Measurement Performance requirements	4969

A.14.6.1	 SS-RSRP for SAN	4969

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4969

A.14.6.1.1.1	Test Purpose and Environment	4969

A.14.6.1.1.2	Test parameters	4969

A.14.6.1.1.3	Test Requirements	4973

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	4973

A.14.6.1.2.1	Test Purpose and Environment	4973

A.14.6.1.2.2	Test parameters	4973

A.14.6.1.2.3	Test Requirements	4976

A.14.6.2	SS-RSRQ	4976

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4976

A.14.6.2.1.1	Test Purpose and Environment	4976

A.14.6.2.1.2	Test Parameters	4976

A.14.6.2.1.3	Test Requirements	4979

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	4979

A.14.6.2.2.1	Test Purpose and Environment	4979

A.14.6.2.2.2	Test Parameters	4979

A.14.6.2.2.3	Test Requirements	4982

A.14.6.3	SS-SINR	4982

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4982

A.14.6.3.1.1	Test Purpose and Environment	4982

A.14.6.3.1.2	Test Parameters	4982

A.14.6.3.1.3	Test Requirements	4986

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	4986

A.14.6.3.2.1	Test Purpose and Environment	4986

A.14.6.3.2.2	Test Parameters	4986

A.14.6.3.2.3	Test Requirements	4990

A.14.6.4	L1-RSRP measurement for beam reporting	4990

A.14.6.4.1	SSB based L1-RSRP measurement	4990

A.14.6.4.1.1	Test Purpose and Environment	4990

A.14.6.4.1.2	Test parameters	4990

A.14.6.4.1.3	Test Requirements	4993

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	4994

A.14.6.4.2.1	Test Purpose and Environment	4994

A.14.6.4.2.2	Test parameters	4994

A.14.6.4.2.3	Test Requirements	4997

A.15	NR standalone tests with one or more NR cells in FR2-2	4999

A.15.1	SA: RRC_IDLE state mobility	4999

A.15.1.1	Cell re-selection to NR	4999

A.15.1.1.1	Cell reselection to FR2-2 intra-frequency NR case	4999

A.15.1.1.1.1	Test Purpose and Environment	4999

A.15.1.1.1.2	Test Parameters	4999

A.15.1.1.1.3	Test Requirements	5003

A.15.1.2	Cell reselection to FR2-2 inter-frequency NR case	5003

A.15.1.2.1	Test Purpose and Environment	5003

A.15.1.2.2	Test Parameters	5003

A.15.1.2.3	Test Requirements	5007

A.15.1.3	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5007

A.15.1.3.1	Test Purpose and Environment	5007

A.15.1.3.2	Test Parameters	5007

A.15.1.3.3	Test Requirements	5010

A.15.1.4	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5010

A.15.1.4.1	Test Purpose and Environment	5010

A.15.1.4.2	Test Parameters	5010

A.15.1.4.3	Test Requirements	5013

A.15.1.5	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5013

A.15.1.5.1	Test Purpose and Environment	5013

A.15.1.5.2	Test Parameters	5013

A.15.1.5.3	Test Requirements	5017

A.15.1.6	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5017

A.15.1.6.1	Test Purpose and Environment	5017

A.15.1.6.2	Test Parameters	5017

A.15.1.6.3	Test Requirements	5020

A.15.2	Signaling characteristics	5020

A.15.2.1	SCell Activation and Deactivation Delay	5020

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	5020

A.15.2.1.1.1	Test Purpose and Environment	5020

A.15.2.1.1.2	Test Requirements	5023

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	5023

A.15.2.1.2.1	Test Purpose and Environment	5023

A.15.2.1.2.2	Test Requirements	5028

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	5029

A.15.2.1.3.1	Test Purpose and Environment	5029

A.15.2.1.3.2	Test Requirements	5032

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	5033

A.15.2.1.4.1	Test Purpose and Environment	5033

A.15.2.1.4.2	Test Requirements	5036

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	5037

A.15.2.1.5.1	Test Purpose and Environment	5037

A.15.2.1.5.2	Test Requirements	5041

A.15.3	RRC_CONNECTED state mobility	5042

A.15.3.1	Handover	5042

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	5042

A.15.3.1.1.1	Test Purpose and Environment	5042

A.15.3.1.1.2	Test Parameters	5042

A.15.3.1.1.3	Test Requirements	5046

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	5046

A.15.3.1.2.1	Test Purpose and Environment	5046

A.15.3.1.2.2	Test Parameters	5046

A.15.3.1.2.3	Test Requirements	5050

A.15.4	Measurement procedure	5051

A.15.4.1	Intra-frequency Measurements	5051

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	5051

A.15.4.1.1.1	Test purpose and Environment	5051

A.15.4.1.1.2	Test Requirements	5053

A.15.4.2	Inter-frequency Measurements	5054

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	5054

A.15.4.2.1.1	Test Purpose and Environment	5054

A.15.4.2.1.2	Test Requirements	5057

A.16	NR standalone tests with all NR cells in FR1 for RedCap	5058

A.16.1	SA: RRC_IDLE state mobility for RedCap	5058

A.16.1.1	Cell re-selection to NR	5058

A.16.1.1.1	Cell reselection to FR1 intra-frequency NR case for 1 Rx UE	5058

A.16.1.1.1.1	Test Purpose and Environment	5058

A.16.1.1.1.2	Test Parameters	5058

A.16.1.1.1.3	Test Requirements	5062

A.16.1.1.2	Cell reselection to FR1 intra-frequency NR case for 2 Rx UE	5062

A.16.1.1.2.1	Test Purpose and Environment	5062

A.16.1.1.2.2	Test Parameters	5062

A.16.1.1.2.3	Test Requirements	5066

A.16.1.1.3	Cell reselection to FR1 inter-frequency NR case for 1 Rx UE	5066

A.16.1.1.3.1	Test Purpose and Environment	5066

A.16.1.1.3.2	Test Parameters	5066

A.16.1.1.3.3	Test Requirements	5071

A.16.1.1.4	Cell reselection to FR1 inter-frequency NR case for 2 Rx UE	5071

A.16.1.1.4.1	Test Purpose and Environment	5071

A.16.1.1.4.2	Test Parameters	5071

A.16.1.1.4.3	Test Requirements	5076

A.16.1.1.5	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5076

A.16.1.1.5.1	Test Purpose and Environment	5076

A.16.1.1.5.2	Test Parameters	5076

A.16.1.1.5.3	Test Requirements	5081

A.16.1.1.6	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5081

A.16.1.1.6.1	Test Purpose and Environment	5081

A.16.1.1.6.2	Test Parameters	5081

A.16.1.1.6.3	Test Requirements	5086

A.16.1.1.7	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5086

A.16.1.1.7.1	Test Purpose and Environment	5086

A.16.1.1.7.2	Test Parameters	5086

A.16.1.1.7.3	Test Requirements	5091

A.16.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5091

A.16.1.1.8.1	Test Purpose and Environment	5091

A.16.1.1.8.2	Test Parameters	5092

A.16.1.1.8.3	Test Requirements	5096

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	5097

A.16.1.2.1	Cell reselection to higher priority E-UTRAN for 1RX	5097

A.16.1.2.1.1	Test Purpose and Environment	5097

A.16.1.2.1.2	Test Parameters	5097

A.16.1.2.1.3	Test Requirements	5100

A.16.1.2.2	Cell reselection to higher priority E-UTRAN for 2RX	5101

A.16.1.2.2.1	Test Purpose and Environment	5101

A.16.1.2.2.2	Test Parameters	5101

A.16.1.2.2.3	Test Requirements	5104

A.16.1.2.3.1	Test Purpose and Environment	5105

A. 16.1.2.3.2	Test Parameters	5105

A.16.1.2.3.3	Test Requirements	5108

A.16.1.2.4.1	Test Purpose and Environment	5109

A.16.1.2.4.2	Test Parameters	5109

A.16.1.3.1.3	Test Requirements	5112

A.16.1.2.5	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5113

A.16.1.2.5.1	Test Purpose and Environment	5113

A.16.1.2.5.2	Test Parameters	5113

A.16.1.2.5.3	Test Requirements	5116

A.16.1.2.6	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5117

A.16.1.2.6.1	Test Purpose and Environment	5117

A.16.1.2.6.2	Test Parameters	5117

A.16.1.2.6.3	Test Requirements	5120

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	5121

A.16.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5121

A.16.2.1.1	NR UE CG-SDT Test in FR1 for 1Rx RedCap UE	5121

A.16.2.1.1.1	Test purpose and Environment	5121

A.16.2.1.1.2	Test Parameters	5122

A.16.2.1.1.3	Test requirements	5126

A.16.2.1.2	NR UE CG-SDT Test in FR1 for 2Rx RedCap UE	5126

A.16.2.1.2.1	Test purpose and Environment	5126

A.16.2.1.2.2	Test Parameters	5128

A.16.2.1.2.3	Test requirements	5131

A.16.3	RRC_CONNECTED state mobility for RedCap	5131

A.16.3.1	Handover	5131

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	5131

A.16.3.1.1.1	Test Purpose and Environment	5131

A.16.3.1.1.2	Test Parameters	5132

A.16.3.1.1.3	Test Requirements	5135

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	5135

A.16.3.1.2.1	Test Purpose and Environment	5135

A.16.3.1.2.2	Test Parameters	5135

A.16.3.1.2.3	Test Requirements	5139

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5139

A.16.3.1.3.1	Test Purpose and Environment	5139

A.16.3.1.3.2	Test Parameters	5139

A.16.3.1.3.3	Test Requirements	5143

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5143

A.16.3.1.4.1	Test Purpose and Environment	5143

A.16.3.1.4.2	Test Parameters	5143

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5147

A.16.3.1.5.1	Test Purpose and Environment	5147

A.16.3.1.5.2	Test Parameters	5147

A.16.3.1.5.3	Test Requirements	5151

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5151

A.16.3.1.6.1	Test Purpose and Environment	5151

A.16.3.1.6.2	Test Parameters	5151

A.16.3.1.6.3	Test Requirements	5155

A.16.3.1.7	SA NR - E-UTRAN handover for 1Rx UE	5155

A.16.3.1.7.1	Test Purpose and Environment	5155

A.16.3.1.7.2	Test Requirements	5161

A.16.3.1.8	 SA NR - E-UTRAN handover for 2Rx UE	5161

A.16.3.1.8.1	Test Purpose and Environment	5161

A.16.3.1.8.2	Test Requirements	5167

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	5167

A.16.3.1.9.1	Test Purpose and Environment	5167

A.16.3.1.9.2	Test Requirements	5174

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	5174

A.16.3.1.10.1	Test Purpose and Environment	5174

A.16.3.1.10.2	Test Requirements	5180

A.16.3.2	RRC Connection Mobility Control	5180

A.16.3.2.1	SA: RRC Re-establishment	5180

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	5180

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	5184

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	5188

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	5193

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	5198

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	5202

A.16.3.2.2	Random Access	5206

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5206

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5212

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5217

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5227

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5231

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5235

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5239

A.16.3.2.3	SA: RRC Connection Release with Redirection	5243

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	5243

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	5247

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	5251

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	5258

A.16.4	Timing for RedCap	5265

A.16.4.1	UE transmit timing	5265

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	5265

A.16.4.1.1.1	Test Purpose and environment	5265

A.16.4.1.1.2	Test requirements	5268

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	5269

A.16.4.1.2.1	Test Purpose and environment	5269

A.16.4.1.2.2	Test requirements	5271

A.16.4.2	UE timer accuracy	5271

A.16.4.3	Timing advance	5271

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	5271

A.16.4.3.1.1	Test Purpose and Environment	5271

A.16.4.3.1.2	Test Parameters	5271

A.16.4.3.1.3	Test Requirements	5275

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	5275

A.16.4.3.2.1	Test Purpose and Environment	5275

A.16.4.3.2.2	Test Parameters	5276

A.16.4.3.2.3	Test Requirements	5279

A.16.5	Signalling characteristics for RedCap	5280

A.16.5.1	Radio link Monitoring	5280

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5280

A.16.5.1.1.1	Test Purpose and Environment	5280

A.16.5.1.1.2	Test Requirements	5285

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5285

A.16.5.1.2.1	Test Purpose and Environment	5285

A.16.5.1.2.2	Test Requirements	5291

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5291

A.16.5.1.3.1	Test Purpose and Environment	5291

A.16.5.1.3.2	Test Requirements	5297

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5297

A.16.5.1.4.1	Test Purpose and Environment	5297

A.16.5.1.4.2	Test Requirements	5303

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5303

A.16.5.1.5.1	Test Purpose and Environment	5303

A.16.5.1.5.2	Test Requirements	5309

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5309

A.16.5.1.6.1	Test Purpose and Environment	5309

A.16.5.1.6.2	Test Requirements	5314

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5314

A.16.5.1.7.1	Test Purpose and Environment	5314

A.16.5.1.7.2	Test Requirements	5319

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5319

A.16.5.1.8.1	Test Purpose and Environment	5319

A.16.5.1.8.2	Test Requirements	5325

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5325

A.16.5.1.9.1	Test Purpose and Environment	5325

A.16.5.1.9.2	Test Requirements	5331

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5331

A.16.5.1.10.1	Test Purpose and Environment	5331

A.16.5.1.10.2	Test Requirements	5337

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5337

A.16.5.1.11.1	Test Purpose and Environment	5337

A.16.5.1.11.2	Test Requirements	5343

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5343

A.16.5.1.12.1	Test Purpose and Environment	5343

A.16.5.1.12.2	Test Requirements	5349

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5349

A.16.5.1.13.1	Test Purpose and Environment	5349

A.16.5.1.13.2	Test Requirements	5354

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5354

A.16.5.1.14.1	Test Purpose and Environment	5354

A.16.5.1.14.2	Test Requirements	5360

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5360

A.16.5.1.15.1	Test Purpose and Environment	5360

A.16.5.1.15.2	Test Requirements	5366

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5366

A.16.5.1.16.1	Test Purpose and Environment	5366

A.16.5.1.16.2	Test Requirements	5372

A.16.5.2	Beam Failure Detection and Link recovery procedures	5372

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5372

A.16.5.2.1.1	Test Purpose and Environment	5372

A.16.5.2.1.2	Test Requirements	5378

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5378

A.16.5.2.2.1	Test Purpose and Environment	5378

A.16.5.2.2.2	Test Requirements	5384

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5384

A.16.5.2.3.1	Test Purpose and Environment	5384

A.16.5.2.3.2	Test Requirements	5391

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5391

A.16.5.2.4.1	Test Purpose and Environment	5391

A.16.5.2.4.2	Test Requirements	5397

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5397

A.16.5.2.5.1	Test Purpose and Environment	5397

A.16.5.2.5.2	Test Requirements	5403

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5403

A.16.5.2.6.1	Test Purpose and Environment	5403

A.16.5.2.6.2	Test Requirements	5409

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5409

A.16.5.2.7.1	Test Purpose and Environment	5409

A.16.5.2.7.2	Test Requirements	5415

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5415

A.16.5.2.8.1	Test Purpose and Environment	5415

A.16.5.2.8.2	Test Requirements	5421

A.16.5.3	Active BWP switch	5421

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	5421

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	5421

A.16.5.3.1.1.1	Test Purpose and Environment	5421

A.16.5.3.1.2.1	Test Requirements	5426

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	5426

A.16.5.3.1.2.1	Test Purpose and Environment	5426

A.16.5.3.1.2.2	Test Requirements	5431

A.16.5.3.2	RRC-based Active BWP Switch	5431

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	5431

A.16.5.3.2.1.1	Test Purpose and Environment	5431

A.16.5.3.2.1.2	Test Requirements	5435

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	5435

A.16.5.3.2.2.1	Test Purpose and Environment	5435

A.16.5.3.2.2.2	Test Requirements	5439

A.16.5.4	UE specific CBW change	5439

A.16.5.4.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	5439

A.16.5.4.1.1	Test Purpose and Environment	5439

A.16.5.4.1.2	Test Requirements	5443

A.16.5.4.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	5443

A.16.5.4.2.1	Test Purpose and Environment	5443

A.16.5.4.2.2	Test Requirements	5447

A.16.6	Measurement procedure for RedCap	5447

A.16.6.1	Intra-frequency Measurements	5447

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	5447

A.16.6.1.1.1	Test purpose and Environment	5447

A.16.6.1.1.2	Test parameters	5447

A.16.6.1.1.3	Test Requirements	5451

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	5451

A.16.6.1.2.1	Test purpose and Environment	5451

A.16.6.1.2.2	Test parameters	5451

A.16.6.1.2.3	Test Requirements	5455

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	5455

A.16.6.1.3.1	Test purpose and Environment	5455

A.16.6.1.3.2	Test parameters	5455

A.16.6.1.3.3	Test Requirements	5459

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	5459

A.16.6.1.4.1	Test purpose and Environment	5459

A.16.6.1.4.2	Test parameters	5459

A.16.6.1.4.3	Test Requirements	5463

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5463

A.16.6.1.5.1	Test purpose and Environment	5463

A.16.6.1.5.2	Test parameters	5463

A.16.6.1.5.3	Test Requirements	5467

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5467

A.16.6.1.6.1	Test purpose and Environment	5467

A.16.6.1.6.2	Test parameters	5467

A.16.6.1.6.3	Test Requirements	5471

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5471

A.16.6.1.7.1	Test purpose and Environment	5471

A.16.6.1.7.2	Test parameters	5471

A.16.6.1.7.3	Test Requirements	5475

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5475

A.16.6.1.8.1	Test purpose and Environment	5475

A.16.6.1.8.2	Test parameters	5475

A.16.6.1.8.3	Test Requirements	5479

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5479

A.16.6.1.9.1	Test purpose and Environment	5479

A.16.6.1.9.2	Test parameters	5479

A.16.6.1.9.3	Test Requirements	5481

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5482

A.16.6.1.10.1	Test purpose and Environment	5482

A.16.6.1.10.2	Test parameters	5482

A.16.6.1.10.3	Test Requirements	5483

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5484

A.16.6.1.11.1	Test purpose and Environment	5484

A.16.6.1.11.2	Test parameters	5484

A.16.6.1.11.3	Test Requirements	5486

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5487

A.16.6.1.12.1	Test purpose and Environment	5487

A.16.6.1.12.2	Test parameters	5487

A.16.6.1.12.3	Test Requirements	5489

A.16.6.2	Inter-frequency Measurements	5490

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5490

A.16.6.2.1.1	Test Purpose and Environment	5490

A.16.6.2.1.2	Test Requirements	5495

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5495

A.16.6.2.2.1	Test Purpose and Environment	5495

A.16.6.2.2.2	Test Requirements	5500

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	5500

A.16.6.2.3.1	Test Purpose and Environment	5500

A.16.6.2.3.2	Test Requirements	5503

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	5504

A.16.6.2.4.1	Test Purpose and Environment	5504

A.16.6.2.4.2	Test Requirements	5507

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5508

A.16.6.2.5.1	Test Purpose and Environment	5508

A.16.6.2.5.2	Test Requirements	5511

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5512

A.16.6.2.6.1	Test Purpose and Environment	5512

A.16.6.2.6.2	Test Requirements	5515

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5516

A.16.6.2.7.1	Test Purpose and Environment	5516

A.16.6.2.7.2	Test Requirements	5520

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5521

A.16.6.2.8.1	Test Purpose and Environment	5521

A.16.6.2.8.2	Test Requirements	5524

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5525

A.16.6.2.9.1	Test Purpose and Environment	5525

A.16.6.2.9.2	Test Requirements	5528

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5528

A.16.6.2.10.1	Test Purpose and Environment	5528

A.16.6.2.10.2	Test Requirements	5531

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5531

A.16.6.2.11.1	Test Purpose and Environment	5531

A.16.6.2.11.2	Test Requirements	5534

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5534

A.16.6.2.12.1	Test Purpose and Environment	5534

A.16.6.2.12.2	Test Requirements	5538

A.16.6.3	Inter-RAT Measurements	5539

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5539

A.16.6.3.1.1	Test purpose and Environment	5539

A.16.6.3.1.2	Test Requirements	5544

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5544

A.16.6.3.2.1	Test purpose and Environment	5544

A.16.6.3.2.2	Test Requirements	5549

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5549

A.16.6.3.3.1	Test purpose and Environment	5549

A.16.6.3.3.2	Test Requirements	5554

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5554

A.16.6.3.4.1	Test purpose and Environment	5554

A.16.6.3.4.2	Test Requirements	5559

A.16.6.4	L1-RSRP measurement for beam reporting	5559

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5559

A.16.6.4.1.1	Test Purpose and Environment	5559

A.16.6.4.1.2	Test parameters	5559

A.16.6.4.1.3	Test Requirements	5563

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5563

A.16.6.4.2.1	Test Purpose and Environment	5563

A.16.6.4.2.2	Test parameters	5564

A.16.6.4.2.3	Test Requirements	5567

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5567

A.16.6.4.3.1	Test Purpose and Environment	5567

A.16.6.4.3.2	Test parameters	5568

A.16.6.4.3.3	Test Requirements	5571

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5571

A.16.6.4.4.1	Test Purpose and Environment	5571

A.16.6.4.4.2	Test parameters	5572

A.16.6.4.4.3	Test Requirements	5575

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5575

A.16.6.4.5.1	Test Purpose and Environment	5575

A.16.6.4.5.2	Test parameters	5576

A.16.6.4.5.3	Test Requirements	5579

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5579

A.16.6.4.6.1	Test Purpose and Environment	5579

A.16.6.4.6.2	Test parameters	5580

A.16.6.4.6.3	Test Requirements	5583

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5583

A.16.6.4.7.1	Test Purpose and Environment	5583

A.16.6.4.7.2	Test parameters	5584

A.16.6.4.7.3	Test Requirements	5587

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5587

A.16.6.4.8.1	Test Purpose and Environment	5587

A.16.6.4.8.2	Test parameters	5588

A.16.6.4.8.3	Test Requirements	5591

A.16.6.5	NR measurements with autonomous gaps	5591

A.16.6.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5591

A.16.6.5.1.1	Test Purpose and Environment	5591

A.16.6.5.1.2	Test Parameters	5591

A.16.6.5.1.3	Test Requirements	5594

A.16.6.5.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5594

A.16.6.5.2.1	Test Purpose and Environment	5594

A.16.6.5.2.2	Test Parameters	5594

A.16.6.5.2.3	Test Requirements	5597

A.16.6.5.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5597

A.16.6.5.3.1	Test Purpose and Environment	5597

A.16.6.5.3.2	Test Requirements	5602

A.16.6.5.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5603

A.16.6.5.4.1	Test Purpose and Environment	5603

A.16.6.5.4.2	Test Requirements	5608

A.16.7	Measurement Performance requirements for RedCap	5609

A.16.7.1	SS-RSRP	5609

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5609

A.16.7.1.1.1	Test Purpose and Environment	5609

A.16.7.1.1.2	Test parameters	5609

A.16.7.1.1.3	Test Requirements	5614

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2Rx UE	5614

A.16.7.1.2.1	Test Purpose and Environment	5614

A.16.7.1.2.2	Test parameters	5614

A.16.7.1.2.3	Test Requirements	5619

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5619

A.16.7.1.3.1	Test Purpose and Environment	5619

A.16.7.1.3.2	Test parameters	5619

A.16.7.1.3.3	Test Requirements	5623

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5623

A.16.7.1.4.1	Test Purpose and Environment	5623

A.16.7.1.4.2	Test parameters	5624

A.16.7.1.4.3	Test Requirements	5628

A.16.7.2	SS-RSRQ	5628

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5628

A.16.7.2.1.1	Test Purpose and Environment	5628

A.16.7.2.1.2	Test Parameters	5629

A.16.7.2.1.3	Test Requirements	5634

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5634

A.16.7.2.2.1	Test Purpose and Environment	5634

A.16.7.2.2.2	Test Parameters	5634

A.16.7.2.2.3	Test Requirements	5639

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5639

A.16.7.2.3.1	Test Purpose and Environment	5639

A.16.7.2.3.2	Test parameters	5639

A.16.7.2.3.3	Test Requirements	5644

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5644

A.16.7.2.4.1	Test Purpose and Environment	5644

A.16.7.2.4.2	Test parameters	5644

A.16.7.2.4.3	Test Requirements	5650

A.16.7.3	SS-SINR	5650

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5650

A.16.7.3.1.1	Test Purpose and Environment	5650

A.16.7.3.1.2	Test parameters	5651

A.16.7.3.1.3	Test Requirements	5655

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5655

A.16.7.3.2.1	Test Purpose and Environment	5655

A.16.7.3.2.2	Test parameters	5656

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5660

A.16.7.3.3.1	Test Purpose and Environment	5660

A.16.7.3.3.2	Test parameters	5660

A.16.7.3.3.3	Test Requirements	5665

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5666

A.16.7.3.4.1	Test Purpose and Environment	5666

A.16.7.3.4.2	Test parameters	5666

A.16.7.3.4.3	Test Requirements	5670

A.16.7.4	L1-RSRP measurement for beam reporting	5671

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5671

A.16.7.4.1.1	Test Purpose and Environment	5671

A.16.7.4.1.2	Test parameters	5671

A.16.7.4.1.3	Test Requirements	5675

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5675

A.16.7.4.2.1	Test Purpose and Environment	5675

A.16.7.4.2.2	Test parameters	5675

A.16.7.4.2.3	Test Requirements	5676

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5676

A.16.7.4.3.1	Test Purpose and Environment	5676

A.16.7.4.3.2	Test parameters	5676

A.16.7.4.3.3	Test Requirements	5680

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5680

A.16.7.4.4.1	Test Purpose and Environment	5680

A.16.7.4.4.2	Test parameters	5681

A.16.7.4.4.3	Test Requirements	5681

A.16.7.5	E-UTRAN RSRP	5681

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5681

A.16.7.5.1.1	Test Purpose and Environment	5681

A.16.7.5.1.2	Test parameters	5681

A.16.7.5.1.3	Test Requirements	5687

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5688

A.16.7.5.2.1	Test Purpose and Environment	5688

A.16.7.5.2.2	Test parameters	5688

A.16.7.5.2.3	Test Requirements	5694

A.16.7.6	E-UTRAN RSRQ	5695

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5695

A.16.7.6.1.1	Test Purpose and Environment	5695

A.16.7.6.1.2	Test parameters	5695

A.16.7.6.1.3	Test Requirements	5701

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5702

A.16.7.6.2.1	Test Purpose and Environment	5702

A.16.7.6.2.2	Test parameters	5702

A.16.7.6.2.3	Test Requirements	5708

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5710

A.17.1	SA: RRC_IDLE state mobility for RedCap	5710

A.17.1.1	Cell re-selection to NR	5710

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5710

A.17.1.1.1.1	Test Purpose and Environment	5710

A.17.1.1.1.2	Test Parameters	5710

A.17.1.1.1.3	Test Requirements	5714

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5714

A.17.1.1.2.1	Test Purpose and Environment	5714

A.17.1.1.2.2	Test Parameters	5714

A.17.1.1.2.3	Test Requirements	5718

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5718

A.17.1.1.3.1	Test Purpose and Environment	5718

A.17.1.1.3.2	Test Parameters	5718

A.17.1.1.3.3	Test Requirements	5721

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5721

A.17.1.1.4.1	Test Purpose and Environment	5721

A.17.1.1.4.2	Test Parameters	5721

A.17.1.1.4.3	Test Requirements	5725

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5725

A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5725

A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap	5725

A.17.2.1.1.1	Test Purpose and Environment	5725

A.17.2.1.1.2	Test Requirements	5729

A.17.3	RRC_CONNECTED state mobility for RedCap	5730

A.17.3.1	Handover for RedCap	5730

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5730

A.17.3.1.1.1	Test Purpose and Environment	5730

A.17.3.1.1.2	Test Parameters	5730

A.17.3.1.1.3	Test Requirements	5732

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5732

A.17.3.1.2.1	Test Purpose and Environment	5732

A.17.3.1.2.2	Test Parameters	5732

A.17.3.1.2.3 Test Requirements	5735

A.17.3.2	RRC Connection Mobility Control for RedCap	5735

A.17.3.2.1	SA: RRC Re-establishment	5735

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5735

A.17.3.2.1.1.1	Test Purpose and Environment	5735

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5738

A.17.3.2.1.2.1	Test Purpose and Environment	5738

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5741

A.17.3.2.1.3.1	Test Purpose and Environment	5741

A.17.3.2.1.3.2	Test Requirements	5743

A.17.3.2.2	Random Access	5744

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5744

A.17.3.2.2.1.1	Test Purpose and Environment	5744

A.17.3.2.2.1.2	Test Requirements	5746

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5748

A.17.3.2.2.2.1	Test Purpose and Environment	5748

A.17.3.2.2.2.2	Test Requirements	5750

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5752

A.17.3.2.2.3.1	Test Purpose and Environment	5752

A.17.3.2.2.3.2	Test Requirements	5754

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5755

A.17.3.2.2.4.1	Test Purpose and Environment	5755

A.17.3.2.2.4.2	Test Requirements	5757

A.17.3.2.3	SA: RRC Connection Release with Redirection	5758

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5758

A.17.3.2.3.1.1	Test Purpose and Environment	5758

A.17.3.2.3.1.2	Test Parameters	5758

A.17.3.2.3.1.3	Test Requirements	5761

A.17.4	Timing	5761

A.17.4.1	UE transmit timing	5761

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5761

A.17.4.1.1.1	Test Purpose and environment	5761

A.17.4.1.1.2	Test requirements	5764

A.17.4.2	UE timer accuracy	5765

A.17.4.3	Timing advance	5765

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5765

A.17.4.3.1.1	Test Purpose and Environment	5765

A.17.4.3.1.2	Test Parameters	5765

A.17.4.3.1.3 Test Requirements	5768

A.17.5	Signaling characteristics for RedCap	5768

A.17.5.1	Radio link Monitoring for RedCap	5768

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5768

A.17.5.1.1.1	Test Purpose and Environment	5768

A.17.5.1.1.2	Test Requirements	5771

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5772

A.17.5.1.2.1	Test Purpose and Environment	5772

A.17.5.1.2.2	Test Requirements	5776

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5777

A.17.5.1.3.1	Test Purpose and Environment	5777

A.17.5.1.3.2	Test Requirements	5781

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5781

A.17.5.1.4.1	Test Purpose and Environment	5781

A.17.5.1.4.2	Test Requirements	5786

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5786

A.17.5.1.5.1	Test Purpose and Environment	5786

A.17.5.1.5.2	Test Requirements	5791

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5791

A.17.5.1.6.1	Test Purpose and Environment	5791

A.17.5.1.6.2	Test Requirements	5795

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5795

A.17.5.1.7.1	Test Purpose and Environment	5795

A.17.5.1.7.2	Test Requirements	5799

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5799

A.17.5.1.8.1	Test Purpose and Environment	5799

A.17.5.1.8.2	Test Requirements	5804

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5804

A.17.5.1.9.1	Test Purpose and Environment	5804

A.17.5.1.9.2	Test Requirements	5807

A.17.5.2	Beam Failure Detection and Link recovery procedures	5807

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5807

A.17.5.2.1.1	Test Purpose and Environment	5807

A.17.5.2.1.2	Test Requirements	5811

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5811

A.17.5.2.2.1	Test Purpose and Environment	5811

A.17.5.2.2.2	Test Requirements	5815

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5815

A.17.5.2.3.1	Test Purpose and Environment	5815

A.17.5.2.3.2	Test Requirements	5819

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5819

A.17.5.2.4.1	Test Purpose and Environment	5819

A.17.5.2.4.2	Test Requirements	5823

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5823

A.17.5.2.5.1	Test Purpose and Environment	5823

A.17.5.2.5.2	Test Requirements	5827

A.17.5.3	Active BWP switch for RedCap	5827

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5827

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5827

A.17.5.3.1.1.1	Test Purpose and Environment	5827

A.17.5.3.1.1.2	Test Requirements	5830

A.17.5.3.2	RRC-based Active BWP Switch	5830

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5830

A.17.5.3.2.1.1	Test Purpose and Environment	5830

A.17.5.3.2.1.2	Test Requirements	5833

A.17.5.4	Active TCI state switch delay	5834

A.17.5.4.1	MAC-CE based active TCI state switch	5834

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5834

A.17.5.4.1.1.1	Test Purpose and Environment	5834

A.17.5.4.1.1.2	Test Requirements	5837

A.17.5.4.2	RRC based active TCI state switch	5837

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5837

A.17.5.4.2.1.1	Test Purpose and Environment	5837

A.17.5.4.2.1.2	Test Requirements	5841

A.17.5.5	Uplink spatial relation switch delay	5841

A.17.5.5.1	MAC-CE based Spatial Relation switch	5841

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5841

A.17.5.5.1.1.1	Test Purpose and Environment	5841

A.17.5.5.1.1.2	Test Requirements	5844

A.17.5.5.2	RRC based spatial relation switch	5844

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5844

A.17.5.5.2.1.2	Test Requirements	5847

A.17.5.6	UE specific CBW change	5847

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5847

A.17.5.6.1.1	Test Purpose and Environment	5847

A.17.5.6.1.2	Test Requirements	5850

A.17.6	Measurement procedure for RedCap	5851

A.17.6.1	Intra-frequency Measurements	5851

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5851

A.17.6.1.1.1	Test purpose and Environment	5851

A.17.6.1.1.2	Test Requirements	5854

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5854

A.17.6.1.2.1	Test purpose and Environment	5854

A.7.6.1.2.2	Test Requirements	5854

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5855

A.17.6.1.3.1	Test purpose and Environment	5855

A.17.6.1.3.2	Test Requirements	5859

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5859

A.17.6.1.4.1	Test purpose and Environment	5859

A.17.6.1.4.2	Test Requirements	5862

A.17.6.2	Inter-frequency Measurements	5863

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5863

A.17.6.2.1.1	Test Purpose and Environment	5863

A.17.6.2.1.2	Test Requirements	5866

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5866

A.17.6.2.2.1	Test Purpose and Environment	5866

A.17.6.2.2.2	Test Requirements	5870

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	5870

A.17.6.2.3.1	Test Purpose and Environment	5870

A.17.6.2.3.2	Test Requirements	5874

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	5874

A.17.6.2.4.1	Test Purpose and Environment	5874

A.17.6.2.4.2	Test Requirements	5878

A.17.6.3	L1-RSRP measurement for beam reporting	5879

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	5879

A.17.6.3.1.1	Test Purpose and Environment	5879

A.17.6.3.1.2	Test parameters	5879

A.17.6.3.1.3	Test Requirements	5879

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	5879

A.17.6.3.2.1	Test Purpose and Environment	5879

A.17.6.3.2.2	Test parameters	5879

A.17.6.3.2.3	Test Requirements	5881

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	5881

A.17.6.3.3.1	Test Purpose and Environment	5881

A.17.6.3.3.2	Test parameters	5882

A.17.6.3.3.3	Test Requirements	5884

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	5885

A.17.6.3.4.1	Test Purpose and Environment	5885

A.17.6.3.4.2	Test parameters	5885

A.7.6.3.3.3	Test Requirements	5887

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	5888

A.17.6.4.1.1	Test Purpose and Environment	5888

A.17.6.4.1.2	Test Requirements	5891

A.17.7	Measurement Performance requirements	5891

A.17.7.1	SS-RSRP	5891

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5891

A.17.7.1.1.1	Test Purpose and Environment	5891

A.17.7.1.1.2	Test parameters	5891

A.17.7.1.1.3	Test Requirements	5895

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5896

A.17.7.1.2.1	Test Purpose and Environment	5896

A.17.7.1.2.2	Test parameters	5896

A.17.7.1.2.3	Test Requirements	5900

A.17.7.2	SS-RSRQ	5901

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5901

A.17.7.2.1.1	Test Purpose and Environment	5901

A.17.7.2.1.2	Test Parameters	5901

A.17.7.2.1.3	Test Requirements	5903

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	5903

A.17.7.2.2.1	Test Purpose and Environment	5903

A.17.7.2.2.2	Test parameters	5903

A.17.7.2.2.3	Test Requirements	5905

A.17.7.3	L1-RSRP measurement for beam reporting	5905

A.17.7.3.1	SSB based L1-RSRP measurement	5905

A.17.7.3.1.1	Test Purpose and Environment	5905

A.17.7.3.1.2	Test parameters	5906

A.17.7.3.1.3	Test Requirements	5906

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5906

A.17.7.3.2.1	Test Purpose and Environment	5906

A.17.7.3.2.2	Test parameters	5906

A.17.7.3.2.3	Test Requirements	5906

A.17.7.4	SS-SINR	5907

A.17.7.4.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	5907

A.17.7.4.1.1	Test Purpose and Environment	5907

A.17.7.4.1.2	Test parameters	5907

A.17.7.4.1.3	Test Requirements	5909

A.17.7.4.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	5909

A.17.7.4.2.1	Test Purpose and Environment	5909

A.17.7.4.2.2	Test Parameters	5909

A.17.7.4.2.3	Test Requirements	5911

A.18	E-UTRA standalone tests for NR RRM for RedCap	5911

A.18.1	RRC_IDLE state mobility	5911

A.18.1.1	Inter-RAT NR Cell re-selection	5911

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	5911

A.18.1.1.1.1	Test Purpose and Environment	5911

A.18.2.1.1.2	Test Requirements	5916

A.18.2	RRC_CONNECTED state mobility	5916

A.18.2.1	Handover	5916

A.18.2.1.1	E-UTRAN - NR handover in FR1	5916

A.18.2.1.1.1	Test Purpose and Environment	5916

A.18.2.1.1.2	Test Requirements	5921

A.18.2.2	RRC connection release with redirection	5921

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	5921

A.18.2.2.1.1	Test Purpose and Environment	5921

A.18.2.2.1.2	Test Parameters	5921

A.18.2.2.1.3	Test Requirements	5928

A.18.3	Measurement procedure	5928

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	5928

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	5928

A.18.3.1.1.1	Test Purpose and Environment	5928

A.18.3.1.1.2	Test Requirements	5933

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	5933

A.18.3.1.2.1	Test Purpose and Environment	5933

A.18.3.1.2.2	Test Requirements	5939

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	5939

A.18.3.1.3.1	Test Purpose and Environment	5939

A.18.3.1.3.2	Test Requirements	5945

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	5945

A.18.3.1.4.1	Test Purpose and Environment	5945

A.18.3.1.4.2	Test Requirements	5951

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	5951

A.18.3.1.5.1	Test Purpose and Environment	5951

A.18.3.1.5.2	Test Requirements	5953

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	5954

A.18.3.1.6.1	Test Purpose and Environment	5954

A.18.3.1.6.2	Test Requirements	5956

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	5957

A.18.3.1.7.1	Test Purpose and Environment	5957

A.18.3.1.7.2	Test Requirements	5959

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	5960

A.18.3.1.8.1	Test Purpose and Environment	5960

A.18.3.1.8.2	Test Requirements	5962

Annex B (normative): Conditions for RRM requirements applicability for operating bands	5966

B.1	Conditions for NR RRC_IDLE state mobility	5966

B.1.1	Introduction	5966

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	5966

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	5967

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	5967

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	5969

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	5969

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	5970

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	5970

B.2.1	Introduction	5970

B.2.1.1	General	5970

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	5970

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	5970

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	5970

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	5971

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	5972

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	5972

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	5972

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	5973

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	5973

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	5973

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	5973

B.2.2	Conditions for NR intra-frequency measurements	5974

B.2.3	Conditions for NR inter-frequency measurements	5975

B.2.4	Conditions for NR L1-RSRP reporting	5976

B.2.4.1	Conditions for SSB based L1-RSRP reporting	5976

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	5978

B.2.5	Conditions for RRC connection release with redirection to NR	5979

B.2.6	Void	5980

B.2.6.1	Void	5980

B.2.6.2	Void	5980

B.2.7	Conditions for SRS-RSRP measurements	5980

B.2.8	Conditions for NR L1-SINR reporting	5981

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	5981

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	5982

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	5982

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	5983

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	5985

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	5985

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	5986

B.2.9	Conditions for NR intra-frequency measurements under CCA	5987

B.2.10	Conditions for NR inter-frequency measurements under CCA	5987

B.2.11	Conditions for NR L1-RSRP reporting under CCA	5987

B.2.11.1	Conditions for SSB based L1-RSRP reporting	5987

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	5988

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	5989

B.2.14	Conditions for NR PRS-based measurements	5991

B.2.15	Conditions for NR intra-frequency measurements for RedCap	5993

B.2.16	Conditions for NR inter-frequency measurements for RedCap	5994

B.2.17	Conditions for NR intra-frequency measurements for satellite access	5996

B.2.18	Conditions for NR inter-frequency measurements for satellite access	5997

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	5997

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	5997

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	5997

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	5998

B.3	RRM Requirements Exceptions	5998

B.3.1	Introduction	5998

B.3.2	Receiver sensitivity relaxation for CA	5998

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	5998

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	5998

B.3.2.2.1	Inter-band carrier aggregation	5998

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	5998

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	5999

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	5999

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	5999

B.3.2.4.1	Intra-band contiguous carrier aggregation	5999

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	5999

B.3.3	Receiver sensitivity relaxation for DC	5999

B.3.3.1	Receiver sensitivity relaxation for EN-DC	5999

B.3.3.2	Receiver sensitivity relaxation for NE-DC	5999

B.3.4	Receiver sensitivity relaxation for SUL	5999

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	5999

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	6000

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	6000

B.4	Conditions for V2X	6000

B.4.1	Test parameters for GNSS signals	6000

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	6000

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	6001

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	6001

B.5	High level test procedure for SAN RRM tests	6001

Annex C (informative): Change history	6003