3GPP TS 26.445 V15.4.2 (2022-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Codec for Enhanced Voice Services (EVS);

Detailed Algorithmic Description

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

UMTS, LTE, EVS, telephony

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	14

1	Scope	15

2	References	15

3	Definitions, abbreviations and mathematical expressions	17

3.1	Definitions	17

3.2	Abbreviations	17

3.3	Mathematical Expressions	19

4	General description of the coder	20

4.1	Introduction	20

4.2	Input/output sampling rate	20

4.3	Codec delay	20

4.4	Coder overview	20

4.4.1	Encoder overview	21

4.4.1.1	Linear Prediction Based Operation	21

4.4.1.2	Frequency Domain Operation	22

4.4.1.3	Inactive Signal coding	23

4.4.1.4	Source Controlled VBR Coding	23

4.4.2	Decoder overview	23

4.4.2.1	Parametric Signal Representation Decoding (Bandwidth Extension)	23

4.4.2.2	Frame loss concealment	23

4.4.3	DTX/CNG operation	24

4.4.3.1	Inactive Signal coding	24

4.4.4	AMR-WB-interoperable option	24

4.4.5	Channel-Aware Mode	24

4.5	Organization of the rest of the Technical Standard	24

5	Functional description of the encoder	25

5.1	Common processing	25

5.1.1	High-pass Filtering	25

5.1.2	Complex low-delay filter bank analysis	25

5.1.2.1	Sub-band analysis	25

5.1.2.2	Sub-band energy estimation	26

5.1.3	Sample rate conversion to 12.8 kHz	27

5.1.3.1	Conversion of 16, 32 and 48 kHz signals to 12.8 kHz	27

5.1.3.2	Conversion of 8 kHz signals to 12.8 kHz	27

5.1.3.3	Conversion of input signals to 16, 25.6 and 32 kHz	29

5.1.4	Pre-emphasis	29

5.1.5	Spectral analysis	30

5.1.5.1	Windowing and DFT	30

5.1.5.2	Energy calculations	31

5.1.6	Bandwidth detection	32

5.1.6.1	Mean and maximum energy values per band	32

5.1.7	Bandwidth decision	34

5.1.8	Time-domain transient detection	37

5.1.9	Linear prediction analysis	38

5.1.9.1	LP analysis window	38

5.1.9.2	Autocorrelation computation	38

5.1.9.3	Adaptive lag windowing	39

5.1.9.4	Levinson-Durbin algorithm	39

5.1.9.5	Conversion of LP coefficients to LSP parameters	40

5.1.9.6	LSP interpolation	41

5.1.9.7	Conversion of LSP parameters to LP coefficients	41

5.1.9.8	LP analysis at 16kHz	42

5.1.10	Open-loop pitch analysis	43

5.1.10.1	Perceptual weighting	43

5.1.10.2	Correlation function computation	44

5.1.10.3	Correlation reinforcement with past pitch values	45

5.1.10.4	Normalized correlation computation	46

5.1.10.5	Correlation reinforcement with pitch lag multiples	46

5.1.10.6	Initial pitch lag determination and reinforcement based on pitch coherence with other half-frames	47

5.1.10.7	Pitch lag determination and parameter update	48

5.1.10.8	Correction of very short and stable open-loop pitch estimates	49

5.1.10.9	Fractional open-loop pitch estimate for each subframe	51

5.1.11	Background noise energy estimation	52

5.1.11.1	First stage of noise energy update	52

5.1.11.2	Second stage of noise energy update	54

5.1.11.2.1	Basic parameters for noise energy update	54

5.1.11.2.2	Spectral diversity	55

5.1.11.2.3	Complementary non-stationarity	55

5.1.11.2.4	HF energy content	56

5.1.11.2.5	Tonal stability	56

5.1.11.2.6	High frequency dynamic range	60

5.1.11.2.7	Combined decision for background noise energy update	60

5.1.11.3	Energy-based parameters for noise energy update	62

5.1.11.3.1	Closeness to current background estimate	62

5.1.11.3.2	Features related to last correlation or harmonic event	62

5.1.11.3.3	Energy-based pause detection	63

5.1.11.3.4	Long-term linear prediction efficiency	63

5.1.11.3.5	Additional long-term parameters used for noise estimation	64

5.1.11.4	Decision logic for noise energy update	65

5.1.12	Signal activity detection	68

5.1.12.1	SAD1 module	69

5.1.12.1.1	SNR outlier filtering	71

5.1.12.2	SAD2 module	72

5.1.12.3	Combined decision of SAD1 and SAD2 modules for WB and SWB signals	75

5.1.12.4	Final decision of the SAD1 module for NB signals	75

5.1.12.5	Post-decision parameter update	76

5.1.12.6	SAD3 module	77

5.1.12.6.1	Sub-band FFT	77

5.1.12.6.2	Computation of signal features	78

5.1.12.6.3	Computation of SNR parameters	81

5.1.12.6.4	Decision of background music	83

5.1.12.6.5	Decision of background update flag	83

5.1.12.6.6	SAD3 Pre-decision	84

5.1.12.6.7	SAD3 Hangover	86

5.1.12.7	Final SAD decision	86

5.1.12.8	DTX hangover addition	88

5.1.13	Coding mode determination	90

5.1.13.1	Unvoiced signal classification	92

5.1.13.1.1	Voicing measure	92

5.1.13.1.2	Spectral tilt	92

5.1.13.1.3	Sudden energy increase from a low energy level	93

5.1.13.1.4	Total frame energy difference	94

5.1.13.1.5	Energy decrease after spike	94

5.1.13.1.6	Decision about UC mode	95

5.1.13.2	Stable voiced signal classification	96

5.1.13.3	Signal classification for FEC	96

5.1.13.3.1	Signal classes for FEC	97

5.1.13.3.2	Signal classification parameters	97

5.1.13.3.3	Classification procedure	98

5.1.13.4	Transient signal classification	99

5.1.13.5	Modification of coding mode in special cases	100

5.1.13.6	Speech/music classification	101

5.1.13.6.1	First stage of the speech/music classifier	101

5.1.13.6.2	Scaling of features in the first stage of the speech/music classifier	103

5.1.13.6.3	Log-probability and decision smoothing	104

5.1.13.6.4	State machine and final speech/music decision	105

5.1.13.6.5	Improvement of the classification for mixed and music content	108

5.1.13.6.6	Second stage of the speech/music classifier	112

5.1.13.6.7	Context-based improvement of the classification for stable tonal signals	114

5.1.13.6.8	Detection of sparse spectral content	118

5.1.13.6.9	Decision about AC mode	120

5.1.13.6.10	Decision about IC mode	120

5.1.14	Coder technology selection	120

5.1.14.1	ACELP/MDCT-based technology selection at 9.6kbps, 16.4 and 24.4 kbps	121

5.1.14.1.1	Segmental SNR estimation of the MDCT-based technology	121

5.1.14.1.2	Segmental SNR estimation of the ACELP technology	127

5.1.14.1.3	Hysteresis and final decision	128

5.1.14.2	TCX/HQ MDCT technology selection at 13.2 and 16.4 kbps	129

5.1.14.3	TCX/HQ MDCT technology selection at 24.4 and 32 kbps	131

5.1.14.4	TD/Multi-mode FD BWE technology selection at 13.2 kbps and 32 kbps	134

5.2	LP-based Coding	135

5.2.1	Perceptual weighting	135

5.2.2	LP filter coding and interpolation	136

5.2.2.1	LSF quantization	136

5.2.2.1.1	LSF weighting function	136

5.2.2.1.2	Bit allocation	139

5.2.2.1.3	Predictor allocation	140

5.2.2.1.4	LSF quantizer structure	140

5.2.2.1.5	LSFQ for voiced coding mode at 16 kHz internal sampling frequency : BC-TCVQ	145

5.2.2.1.6	Mid-frame LSF quantizer	152

5.2.3	Excitation coding	153

5.2.3.1	Excitation coding in the GC, VC and high rate IC/UC modes	153

5.2.3.1.1	Computation of the LP residual signal	154

5.2.3.1.2	Target signal computation	155

5.2.3.1.3	Impulse response computation	155

5.2.3.1.4	Adaptive codebook	155

5.2.3.1.5	Algebraic codebook	158

5.2.3.1.6	Combined algebraic codebook	168

5.2.3.1.7	Gain quantization	182

5.2.3.2	Excitation coding in TC mode	187

5.2.3.2.1	Glottal pulse codebook search	187

5.2.3.2.2	TC frame configurations	191

5.2.3.3	Excitation coding in UC mode at low rates	196

5.2.3.3.1	Structure of the Gaussian codebook	196

5.2.3.3.2	Correction of the Gaussian codebook spectral tilt	197

5.2.3.3.3	Search of the Gaussian codebook	198

5.2.3.3.4	Quantization of the Gaussian codevector gain	199

5.2.3.3.5	Other parameters in UC mode	200

5.2.3.3.6	Update of filter memories	200

5.2.3.4	Excitation coding in IC and UC modes at 9.6 kbps	200

5.2.3.4.1	Algebraic codebook	201

5.2.3.4.2	Gaussian noise generation	202

5.2.3.4.3	Gain coding	202

5.2.3.4.4	Memory update	204

5.2.3.5	Excitation coding in GSC mode	204

5.2.3.5.1	Determining the subframe length	205

5.2.3.5.2	Computing time-domain excitation contribution	205

5.2.3.5.3	Frequency transform of residual and time-domain excitation contribution	206

5.2.3.5.4	Computing energy dynamics of transformed residual and quantization of noise level	207

5.2.3.5.6	Find and encode the cut-off frequency	207

5.2.3.5.7	Band energy computation and quantization	209

5.2.3.5.8	PVQ Bit allocation	209

5.2.3.5.9	Quantization of difference signal	210

5.2.3.5.10	Spectral dynamic and noise filling	210

5.2.3.5.11	Quantized gain addition, temporal and frequency contributions combination	210

5.2.3.5.12	Specifics for wideband 8kbps	211

5.2.3.5.13	Inverse DCT	212

5.2.3.5.14	Remove pre-echo in case of onset detection	212

5.2.4	Bass post-filter gain quantization	213

5.2.5	Source Controlled VBR Coding	213

5.2.5.1	Principles of VBR Coding	213

5.2.5.2	EVS VBR Encoder Coding Modes and Bit-Rates	214

5.2.5.3	Prototype-Pitch-Period (PPP) Encoding	214

5.2.5.3.1	PPP Algorithm	214

5.2.5.3.2	Amplitude Quantization	216

5.2.5.3.3	Phase Quantization	216

5.2.5.4	Noise-Excited-Linear-Prediction (NELP) Encoding	216

5.2.5.5	Average Data Rate (ADR) Control for the EVS VBR mode	216

5.2.6	Coding of upper band for LP-based Coding Modes	220

5.2.6.1	Bandwidth extension in time domain	220

5.2.6.1.1	High band target signal generation	221

5.2.6.1.2	TBE LP analysis	222

5.2.6.1.3	Quantization of linear prediction parameters	224

5.2.6.1.4	Interpolation of LSF coefficients	227

5.2.6.1.5	Target and residual energy calculation and quantization	229

5.2.6.1.6	Generation of the upsampled version of the lowband excitation	229

5.2.6.1.7	Non-Linear Excitation Generation	230

5.2.6.1.8	Spectral flip of non-linear excitation in time domain	231

5.2.6.1.9	Down-sample using all-pass filters	231

5.2.6.1.10	Adaptive spectral whitening	232

5.2.6.1.11	Envelope modulated noise mixing	232

5.2.6.1.12	Spectral shaping of the noise added excitation	234

5.2.6.1.13	Post processing of the shaped excitation	235

5.2.6.1.14	Estimation of temporal gain shape parameters	236

5.2.6.1.15	Estimation of frame gain parameters	239

5.2.6.1.16	Estimation of TEC/TFA envelope parameters	241

5.2.6.1.17	Estimation of full-band frame energy parameters	244

5.2.6.2	Multi-mode FD Bandwidth Extension Coding	246

5.2.6.2.1	SWB/FB Multi-mode FD Bandwidth Extension	246

5.2.6.2.2	WB Multi-mode FD Bandwidth Extension	257

5.2.6.3	Coding of upper band at 64 kb/s	261

5.2.6.3.1	Coding in normal mode	261

5.2.6.3.2	Coding in transient mode	265

5.3	MDCT Coding Mode	268

5.3.1	General description	268

5.3.2	Time-to-frequency transformations	268

5.3.2.1	Transform sizes and MDCT configurations	268

5.3.2.2	Long block transformation (ALDO window)	268

5.3.2.2.1	Folding and on-the-fly window decimation	270

5.3.2.2.2	eDCT	272

5.3.2.3	Transient location dependent overlap and transform length	274

5.3.2.4	Short block transformation	275

5.3.2.4.2	Short window transform for MDCT based TCX	279

5.3.2.5	Special window transitions	280

5.3.2.5.1	ALDO to short transition	280

5.3.2.5.2	Short to ALDO transition	280

5.3.2.6	Modified Discrete Sine Transform	280

5.3.3	MDCT based TCX	281

5.3.3.1	General description	281

5.3.3.1.1	High level overview	281

5.3.3.1.2	Rate dependent configuration	281

5.3.3.2	General encoding procedure	282

5.3.3.2.1	LPC parameter calculation	282

5.3.3.2.2	Temporal Noise Shaping	286

5.3.3.2.3	LPC shaping in MDCT domain	288

5.3.3.2.4	Adaptive low frequency emphasis	291

5.3.3.2.5	Spectrum noise measure in power spectrum	292

5.3.3.2.6	Low pass factor detector	293

5.3.3.2.7	Uniform quantizer with adaptive dead-zone	293

5.3.3.2.8	Arithmetic coder	293

5.3.3.2.9	Global gain coding	305

5.3.3.2.10	Noise Filling	306

5.3.3.2.11	Intelligent Gap Filling	309

5.3.3.2.12	Memory updates	324

5.3.3.2.13	Global Gain Adjuster	324

5.3.4	High Quality MDCT coder (HQ)	325

5.3.4.1	Low-rate HQ coder	325

5.3.4.1.1	Tonality Estimation	326

5.3.4.1.2	Grouping of spectral coefficients	326

5.3.4.1.3	Energy Envelope coding	331

5.3.4.1.4	MDCT coefficients quantization	338

5.3.4.2	High-rate HQ coder	374

5.3.4.2.1	Normal Mode	376

5.3.4.2.2	Transient Mode	386

5.3.4.2.3	Generic, Harmonic and HVQ mode detector	387

5.3.4.2.4	Harmonic Mode	388

5.3.4.2.5	HVQ	389

5.3.4.2.6	Generic Mode	392

5.3.4.2.7	Pyramid Vector Quantization (PVQ) and indexing	400

5.4	Switching of Coding Modes	410

5.4.1	General description	410

5.4.2	MDCT coding mode to CELP coding mode	410

5.4.2.1	MDCT to CELP transition 1 (MC1)	411

5.4.2.2	MDCT to CELP transition 2 (MC2)	411

5.4.2.3	MDCT to CELP transition 3 (MC3)	411

5.4.3	CELP coding mode to MDCT coding mode	411

5.4.3.1	CELP coding mode to MDCT based TCX coding mode	411

5.4.3.2	CELP coding mode to HQ MDCT coding mode	412

5.4.3.2.1	Constrained CELP coding and simplified BWE coding	412

5.4.3.2.2	HQ MDCT coding with a modified analysis window	413

5.4.4	Internal sampling rate switching	414

5.4.4.1	Reset of LPC memory	414

5.4.4.2	Conversion of LP filter between 12.8 and 16 kHz internal sampling rates	414

5.5.4.1.1	Modification of the Power Spectrum	414

5.5.4.1.2	Computation of the Power Spectrum	415

5.5.4.1.3	Computation of the Autocorrelation	416

5.4.4.3	Extrapolation of LP filter	417

5.4.4.4	Buffer resampling with linear interpolation	417

5.4.4.5	Update of CELP input signal memories	417

5.4.4.6	Update of MDCT-based TCX input signal memories	418

5.4.4.7	Update of CELP synthesis memories	418

5.4.5	EVS primary and AMR-WB IO	418

5.4.5.1	Switching from primary modes to AMR-WB IO	418

5.4.5.2	Switching from AMR-WB IO mode to primary modes	419

5.4.6	Rate switching	419

5.4.6.1	Rate switching along with internal sampling rate switching	419

5.4.6.2	Rate switching along with coding mode switching	419

5.5	Frame erasure concealment side information	420

5.5.1	Signal classification parameter	420

5.5.2	Energy information	420

5.5.3	Phase control information	421

5.5.4	Pitch lag information	422

5.5.5	Spectral envelope diffuser	422

5.5.6	Tonality flag information	423

5.6	DTX/CNG operation	424

5.6.1	Overview	424

5.6.1.1	SID update	425

5.6.1.2	Spectral tilt based SID transmission	426

5.6.1.3	CNG selector	427

5.6.2	Encoding for LP-CNG	427

5.6.2.1	LP-CNG CN parameters estimation	428

5.6.2.1.1	LP-CNG Hangover analysis period determination	428

5.6.2.1.2	LP-CNG filter parameters evaluation for low-band signal	428

5.6.2.1.3	LP-CNG CNG-LSF quantization for low-band signal	429

5.6.2.1.4	LP-CNG synthesis filter computation for local CNG synthesis	430

5.6.2.1.5	LP-CNG energy calculation and quantization	431

5.6.2.1.6	LP-CNG energy smoothing for local CNG synthesis	432

5.6.2.1.7	LP-CNG LF-BOOST determination and quantization	432

5.6.2.1.8	LP-CNG high band analysis and quantization	433

5.6.2.2	LP-CNG local CNG synthesis	435

5.6.2.3	LP-CNG CNG Memory update	435

5.6.3	Encoding for FD-CNG	435

5.6.3.1	Spectral partition energies	436

5.6.3.1.1	Computation of the FFT partition energies	436

5.6.3.1.2	Computation of the CLDFB partition energies	436

5.6.3.1.3	FD-CNG configurations	436

5.6.3.2	FD-CNG noise estimation	437

5.6.3.2.1	Dynamic range compression for the input energies	437

5.6.3.2.2	Noise tracking	438

5.6.3.2.3	Dynamic range expansion for the estimated noise energies	442

5.6.3.3	Adjusting the first SID frame in FD-CNG	442

5.6.3.4	FD-CNG resetting mechanism	442

5.6.3.5	Encoding SID frames in FD-CNG	442

5.6.3.6	FD-CNG local CNG synthesis	444

5.6.3.6.1	SID parameters interpolation	445

5.6.3.6.2	LPC estimation from the interpolated SID parameters	445

5.6.3.6.3	FD-CNG encoder comfort noise generation	445

5.6.3.6.4	FD-CNG encoder memory update	445

5.7	AMR-WB-interoperable modes	446

5.7.1	Pre-processing	446

5.7.2	Linear prediction analysis and quantization	446

5.7.2.1	Windowing and auto-correlation computation	446

5.7.2.2	Levinson-Durbin algorithm	446

5.7.2.3	LP to ISP conversion	446

5.7.2.4	ISP to LP conversion	447

5.7.2.5	Quantization of the ISP coefficients	447

5.7.2.6	Interpolation of the ISPs	447

5.7.3	Perceptual weighting	447

5.7.4	Open-loop pitch analysis	447

5.7.5	Impulse response computation	447

5.7.6	Target signal computation	447

5.7.7	Adaptive codebook search	448

5.7.8	Algebraic codebook search	448

5.7.9	Quantization of the adaptive and fixed codebook gains	448

5.7.10	Memory update	448

5.7.11	High-band gain generation	448

5.7.12	CNG coding	448

5.8	Channel Aware Coding	448

5.8.1	Introduction	448

5.8.2	Principles of Channel Aware Coding	449

5.8.3	Bit-Rate Allocation for Primary and Partial Redundant Frame Coding	450

5.8.3.1	Primary frame bit-rate reduction	450

5.8.3.2	Partial Redundant Frame Coding	450

5.8.3.2.1	Construction of partial redundant frame for Generic and Voiced Coding modes	450

5.8.3.2.2	Construction of partial redundant frame for Unvoiced Coding mode	450

5.8.3.2.3	Construction of partial redundant frame for TCX frame	451

5.8.3.2.4	RF_NO_DATA partial redundant frame type	451

5.8.3.3	Decoding	451

5.8.4	Channel aware mode encoder configurable parameters	451

6	Functional description of the Decoder	453

6.1	LP-based Decoding	453

6.1.1	General LP-based decoding	453

6.1.1.1	LSF decoding	453

6.1.1.1.1	General LSF decoding	453

6.1.1.1.2	LSF decoding for voiced coding mode at 16 kHz internal sampling frequency	456

6.1.1.2	Reconstruction of the excitation	458

6.1.1.2.1	Reconstruction of the excitation in GC and VC modes and high rate IC/UC modes	458

6.1.1.2.2	Reconstruction of the excitation in TC mode	465

6.1.1.2.3	Reconstruction of the excitation in UC mode at low rates	465

6.1.1.2.4	Reconstruction of the excitation in IC/UC mode at 9.6 kbps	467

6.1.1.2.5	Reconstruction of the excitation in GSC	468

6.1.1.3	Excitation post-processing	469

6.1.1.3.1	Anti-sparseness processing	469

6.1.1.3.2	Gain smoothing for noise enhancement	469

6.1.1.3.3	Pitch enhancer	470

6.1.1.3.4	Music post processing	471

6.1.2	Source Controlled VBR decoding	478

6.1.3	Synthesis	478

6.1.4	Post-processing	479

6.1.4.1.1	Long-term post-filter	479

6.1.4.1.2	Short-term post-filter	480

6.1.4.1.3	Post-filter NB parameters	481

6.1.4.1.4	Post-filter WB and SWB parameters	481

6.1.4.1.5	Tilt compensation	482

6.1.4.1.6	Adaptive gain control	482

6.1.4.2	Bass post-filter	483

6.1.5	Decoding of upper band for LP-based Coding Modes	485

6.1.5.1	Decoding Time-domain Bandwidth Extension	485

6.1.5.1.1	Generation of the upsampled version of the lowband excitation	485

6.1.5.1.2	Non-Linear Excitation Generation	486

6.1.5.1.3	De-quantization of high band parameters	487

6.1.5.1.4	LSP interpolation	487

6.1.5.1.5	Spectral flip in time domain	487

6.1.5.1.6	Down-sample using all-pass filters	488

6.1.5.1.7	Adaptive spectral whitening	488

6.1.5.1.8	Envelope modulated noise mixing	489

6.1.5.1.9	Spectral shaping of the noise added excitation	490

6.1.5.1.10	Post processing of the shaped excitation	490

6.1.5.1.11	Gain shape update	491

6.1.5.1.12	SHB synthesis	491

6.1.5.1.13	Core-switching and high-band memory updates	493

6.1.5.1.14	TEC/TFA post processing	493

6.1.5.1.15	Full-band synthesis	495

6.1.5.2	Multi-mode FD Bandwidth Extension decoding	496

6.1.5.2.1	SWB multi-mode FD BWE decoding	496

6.1.5.2.2	WB multi-mode FD BWE decoding	504

6.1.5.3	Decoding of upper band at 64 kb/s	512

6.1.5.3.1	Decoding in normal mode	512

6.1.5.3.2	Decoding in transient mode	517

6.1.5.3.3	Windowing and frequency-to-time transformation	519

6.1.5.3.4	Post-processing in temporal domain	519

6.2	MDCT Coding mode decoding	521

6.2.1	General MDCT decoding	521

6.2.2	MDCT based TCX	521

6.2.2.1	Rate dependent configurations	521

6.2.2.2	Init module parameters	521

6.2.2.2.1	TCX block configuration	521

6.2.2.2.2	LPC parameter	521

6.2.2.2.3	PLC Wavefrom adjustment	522

6.2.2.2.4	Global Gain	522

6.2.2.2.5	Noise fill parameter	523

6.2.2.2.6	LTP	523

6.2.2.2.7	TNS parameter	523

6.2.2.2.8	Harmonic model	523

6.2.2.2.9	IGF bit stream reader	523

6.2.2.2.10	Spectral data	526

6.2.2.2.11	Residual bits	526

6.2.2.3	Decoding process	527

6.2.2.3.1	Arithmetic decoder	527

6.2.2.3.2	Adaptive low frequency de-emphasis	528

6.2.2.3.3	Global gain decoding	529

6.2.2.3.4	Residual bits decoding	529

6.2.2.3.5	TCX formant enhancement	530

6.2.2.3.6	Noise Filling	531

6.2.2.3.7	Apply global gain and LPC shaping in MDCT domain	532

6.2.2.3.8	IGF apply	533

6.2.2.3.9	Inverse window grouping (TCX5 separation)	538

6.2.2.3.10	Temporal Noise Shaping	539

6.2.2.3.11	IGF temporal flattening	539

6.2.3	High Quality MDCT decoder (HQ)	540

6.2.3.1	Low-rate HQ decoder	540

6.2.3.1.1	Mode decoding	540

6.2.3.1.2	Energy Envelope decoding	540

6.2.3.1.3	Spectral coefficients decoding	543

6.2.3.2	High-rate HQ decoder	553

6.2.3.2.1	Normal Mode	553

6.2.3.2.2	Transient Mode	561

6.2.3.2.3	Harmonic Mode	562

6.2.3.2.4	HVQ	564

6.2.3.2.5	Generic Mode	565

6.2.3.2.6	PVQ decoding and de-indexing	570

6.2.4	Frequency-to-time transformation	572

6.2.4.1	Long block transformation (ALDO window)	572

6.2.4.1.1	eDCT	572

6.2.4.1.2	Unfolding and windowing	572

6.2.4.1.3	Overlap-add	573

6.2.4.1.4	Pre-echo attenuation	574

6.2.4.2	Transient location dependent overlap and transform length	580

6.2.4.3	Short block transformation	580

6.2.4.3.1	Short window transform in TDA domain	580

6.2.4.3.2	Short window transform for MDCT based TCX	581

6.2.4.4	Special window transitions	581

6.2.4.4.1	ALDO to short transition	581

6.2.4.4.2	Short to ALDO transition	582

6.2.4.5	Low Rate MDCT Synthesis	582

6.3	Switching coding modes in decoding	582

6.3.1	General description	582

6.3.2	MDCT coding mode to CELP coding mode	583

6.3.2.1	MDCT to CELP transition 1 (MC1)	583

6.3.2.2	MDCT to CELP transition 2 (MC2)	584

6.3.2.3	MDCT to CELP transition 3 (MC3)	584

6.3.3	CELP coding mode to MDCT coding mode	584

6.3.3.1	CELP coding mode to MDCT based TCX coding mode	585

6.3.3.2	CELP coding mode to HQ MDCT coding mode	586

6.3.3.2.1	Constrained CELP decoding and simplified BWE decoding	587

6.3.3.2.1.1	Optimized cubic interpolation	587

6.3.3.2.2	HQ MDCT decoding with a modified synthesis window	588

6.3.3.2.3	Cross-fading	588

6.3.4	Internal sampling rate switching	588

6.3.4.1	Reset of LPC memory	588

6.3.4.2	Conversion of LP filter between 12.8 and 16 kHz internal sampling rates	589

6.3.4.3	Extrapolation of LP filter	589

6.3.4.4	Update of CELP synthesis memories	589

6.3.4.5	Update of CELP decoded past signal	589

6.3.4.6	Post-processing	589

6.3.4.6.1	Adaptive post-filtering	589

6.3.4.6.2	Bass post filter	589

6.3.4.7	CLDFB	589

6.3.5	EVS primary modes and AMR-WB IO	590

6.3.5.1	Switching from primary modes to AMR-WB IO	590

6.3.5.2	Switching from AMR-WB IO mode to primary modes	590

6.3.6	Rate switching	590

6.3.6.1	Rate switching along with internal sampling rate switching	590

6.3.6.2	Rate switching along with coding mode switching	590

6.3.6.3	Adaptive post-filter reset and smoothing	590

6.3.7	Bandwidth switching	590

6.3.7.1	Bandwidth switching detector	590

6.3.7.2	Super wideband switching to wideband	591

6.3.7.2.1	TBE mode	591

6.3.7.2.2	Multi-mode FD BWE mode	593

6.3.7.2.3	MDCT core	594

6.3.7.3	Wideband switching to super wideband	594

6.4	De-emphasis	594

6.5	Resampling to the output sampling frequency	594

6.6	Decoding of frame erasure concealment side information	594

6.7	Decoding in DTX/CNG operation	595

6.7.1	Overview	595

6.7.2	Decoding for LP-CNG	595

6.7.2.1	LP-CNG decoding Overview	595

6.7.2.1.1	CNG parameter updates in active periods	595

6.7.2.1.2	DTX-hangover based parameter analysis in LP-CNG mode	596

6.7.2.1.3	LP-CNG low-band energy decoding	598

6.7.2.1.4	LP-CNG low-band filter parameters decoding	598

6.7.2.1.5	LP-CNG low-band excitation generation	598

6.7.2.1.6	LP-CNG low-band synthesis	600

6.7.2.1.7	LP-CNG high-band decoding and synthesis	600

6.7.2.2	Memory update	602

6.7.3	Decoding for FD-CNG	602

6.7.3.1	Decoding SID frames in FD-CNG	602

6.7.3.1.1	SID parameters decoding	603

6.7.3.1.2	SID parameters interpolation	603

6.7.3.1.3	LPC estimation from the interpolated SID parameters	604

6.7.3.2	Noise tracking during active frames in FD-CNG	604

6.7.3.2.1	Spectral partition energies	604

6.7.3.2.2	FD-CNG noise estimation	605

6.7.3.2.3	Noise shaping in FD-CNG	605

6.7.3.3	Noise generation for SID or zero frames in FD-CNG	607

6.7.3.3.1	Update of the noise levels for FD-CNG	607

6.7.3.3.2	Comfort noise generation in the frequency domain	607

6.7.3.3.3	Comfort noise generation in the time domain	607

6.7.3.3.4	FD-CNG decoder memory update	609

6.8	AMR-WB-interoperable modes	610

6.8.1	Decoding and speech synthesis	610

6.8.1.1	Excitation decoding	610

6.8.1.2	Excitation post-processing	611

6.8.1.2.1	Anti-sparseness processing	611

6.8.1.2.2	Gain smoothing for noise enhancement	611

6.8.1.2.3	Pitch enhancer	611

6.8.1.3	Synthesis filtering	611

6.8.1.4	Music and Unvoiced/inactive Post-processing	611

6.8.1.4.1	Music post processing	611

6.8.1.4.2	Unvoiced and inactive post processing	612

6.8.1.5	Synthesis filtering and overwriting the current CELP synthesis	615

6.8.1.6	Formant post-filter	615

6.8.1.7	Comfort noise addition	615

6.8.1.8	Bass post-filter	615

6.8.2	Resampling	616

6.8.3	High frequency band	616

6.8.3.1	Preliminary estimation steps	616

6.8.3.1.1	Estimation of tilt, figure of merit and voice factors	617

6.8.3.1.2	Estimation of sub-frame gains based on LP spectral envelopes	618

6.8.3.2	Generation of high-band excitation	621

6.8.3.2.1	DCT	621

6.8.3.2.2	High band generation	621

6.8.3.2.3	Extraction of tonal and ambiance components	623

6.8.3.2.4	Recombination	624

6.8.3.2.5	Filtering in DCT domain	625

6.8.3.2.6	Inverse DCT	626

6.8.3.2.7	Gain computation and scaling of excitation	626

6.8.3.3	LP filter for the high frequency band	628

6.8.3.4	High band synthesis	628

6.8.4	CNG decoding	629

6.9	Common post-processing	629

6.9.1	Comfort noise addition	629

6.9.1.1	Noisy speech detection	629

6.9.1.2	Noise estimation for CNA	630

6.9.1.2.1	CNA noise estimation in DTX-on mode when FD-CNG is triggered	630

6.9.1.2.2	CNA noise estimation in DTX-on mode when LP-CNG is triggered	630

6.9.1.2.3	CNA noise estimation in DTX-off mode	630

6.9.1.3	Noise generation in the FFT domain and addition in the time domain	630

6.9.1.4	Noise generation and addition in the MDCT domain	631

6.9.2	Long term prediction processing	631

6.9.2.1	Decoding LTP parameters	631

6.9.2.2	LTP post filtering	632

6.9.3	Complex low delay filter bank synthesis	634

6.9.4	High pass filtering	635

7	Description of the transmitted parameter indices	635

7.1	Bit allocation for the default option	635

7.1.1	Bit allocation at VBR 5.9, 7.2 – 9.6 kbps	635

7.1.2	Bit allocation at 13.2 kbps	636

7.1.3	Bit allocation at 16.4 and 24.4 kbps	636

7.1.4	Bit allocation at 32 kbps	637

7.1.5	Bit allocation at 48, 64, 96 and 128 kbps	638

7.2	Bit allocation for SID frames in the DTX operation	639

7.3	Bit allocation for the AMR-WB-interoperable option	640

7.4	Bit Allocation for the Channel-Aware Mode	640

Annex A (normative): RTP Payload Format and SDP Parameters	642

A.0	General	642

A.1	RTP Header Usage	642

A.2	EVS RTP Payload Format	642

A.2.1	EVS codec Compact Format	642

A.2.1.1	Compact format for EVS Primary mode	643

A.2.1.2	Compact format for EVS AMR-WB IO mode (except SID)	643

A.2.1.2.1	Representation of Codec Mode Request (CMR) in Compact format for EVS AMR-WB IO mode	643

A.2.1.2.2	Payload structure of Compact EVS AMR-WB IO mode frame	644

A.2.1.3	Special case for 56 bit payload size (EVS Primary or EVS AMR-WB IO SID)	645

A.2.2	EVS codec Header-Full format	645

A.2.2.1	EVS RTP payload structure	645

A.2.2.1.1	CMR byte	646

A.2.2.1.2	ToC byte	649

A.2.2.1.3	Speech Data	651

A.2.2.1.4	Zero padding	651

A.2.2.1.4.1	Zero padding for octet alignment of speech data (EVS AMR-WB IO)	651

A.2.2.1.4.2	Zero padding for size collision avoidance	651

A.2.2.1.4.3	Additional zero padding	651

A.2.3	Header-Full/Compact format handling	651

A.2.3.1	Default format handling	652

A.2.3.2	Header-Full-only format handling	652

A.2.4	AMR-WB backward compatible EVS AMR-WB IO mode format	652

A.2.5	Sessions with multiple mono channels	652

A.2.5.1	Encoding of multiple mono channels	652

A.2.5.2	RTP header usage	652

A.2.5.3	Construction of the RTP payload	652

A.2.6	Storage Format	653

A.2.6.1	Header	653

A.2.6.2	Speech Frames	654

A.3	Payload Format Parameters	654

A.3.1	EVS Media Type Registration	654

A.3.2	Mapping Media Type Parameters into SDP	657

A.3.3	Detailed Description of Usage of SDP Parameters	657

A.3.3.1	Offer-Answer Model Considerations	657

A.3.3.2	Examples	659

A.3.3.3	Interactions of the dtx and dtx-recv parameters	660

Annex B (informative): Change history	661

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a detailed description of the signal processing algorithms of the Enhanced Voice Services coder.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.441: "Codec for Enhanced Voice Services (EVS); General Overview".

[3]	3GPP TS 26.442: "Codec for Enhanced Voice Services (EVS); ANSI C code (fixed-point)".

[4]	3GPP TS 26.444: "Codec for Enhanced Voice Services (EVS); Test Sequences".

[5]	3GPP TS 26.446: "Codec for Enhanced Voice Services (EVS); AMR-WB Backward Compatible Functions".

[6]	3GPP TS 26.447: "Codec for Enhanced Voice Services (EVS); Error Concealment of Lost Packets".

[7]	3GPP TS 26.448: "Codec for Enhanced Voice Services (EVS); Jitter Buffer Management".

[8]	3GPP TS 26.173: "ANSI-C code for the Adaptive Multi Rate - Wideband (AMR-WB) speech codec".

[9]	3GPP TS 26.190: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Transcoding functions".

[10]	3GPP TS 26.103: "Speech codec list for GSM and UMTS".

[11]	3GPP TS 26.290: "Extended AMR Wideband codec; Transcoding functions".

[12]	3GPP TS 26.401: "General audio codec audio processing functions; Enhanced aacPlus general audio codec; General description".

[13]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[14]	3GPP TS 22.105: "Services and Service capabilities".

[15]	IETF RFC 4867 (2007): "RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs", J. Sjoberg, M. Westerlund, A. Lakaniemi and Q. Xie.

[16]	IEEE Transactions on Acoustic, Speech and Signals Processing, Vol. 2, April, pp. 9-12 (1993): “Immittance spectral pairs (ISP) for speech encoding”,Bistritz, Y., and Peller, S.

[17]	IEEE Journal on Selected Areas in Communications, Vol. 6, No. 2, February, pp. 314-323 (1988): "Transform Coding of Audio Signals Using Perceptual Noise Criteria", Johnston, J.D.

[18]	IEEE Transactions on Acoustic, Speech and Signals Processing, Vol. 34, No. 6, December, pp. 1419 1426 (1986): "The computation of line spectral frequencies using Chebyshev polynomials", Kabal, P., and Ramachandran, R.P.

[19]	IEEE Proceedings of International Conference on Acoustics, Speech and Signal Processing, March-April, pp. 4001-4004 (2008): "Transition mode coding for source controlled CELP codecs”, Eksler, V., and Jelínek, M.

[20]	3GPP2 C.S0014-B, (2006) “Enhanced Variable Rate Codec, Speech Service 3 Option 3 and 68 for Wideband Spread Spectrum 4 Digital Systems."

[21]	Speech and Audio Processing for Coding, Enhancement and Recognition, Springer (2014): Recent Speech Coding Technologies and Standards", Sinder, D., Varga, I., Krishnan, V. Rajendran, V, and Villette, S.

[22]	Proceedings of EUSIPCO (2002): "LSF quantization with multiple scale lattice VQ for transmission over noisy channels", Vasilache, A. and Tabus, I.

[23]	Proceedings of INTERSPEECH (2013): “Computationally efficient objective function for algebraic codebook optimization in ACELP”, Bäckström, T.

[24]	Proceedings of INTERSPEECH (2012): "Enumerative algebraic coding for ACELP", Bäckström, T.

[25]	ITU-T Recommendation G.718 Amendment 2 (2010): "New Annex B on Superwideband scalable extension for ITU-T G.718 and corrections to main body fixed-point C-code and description text."

[26]	Signal Processing, no. 82, pp 563-586 (2002): "Multiple-scale leader-lattice VQ with application to LSF quantization", Vasilache, A. and Tabus, I.

[27]	IETF RFC 4566 (2006): "SDP: Session Description Protocol", M. Handley, V. Jacobson and C. Perkins.

[28]	IETF RFC 3264 (2002): "An Offer/Answer Model with the Session Description Protocol (SDP)", J. Rosenberg and H. Schulzrinne.

[29]	IETF RFC 5939 (2010): "Session Description Protocol (SDP) Capability Negotiation", F. Andreasen.

[30]	IETF RFC 3550 (2003): "RTP: A Transport Protocol for Real-Time Applications", H. Schulzrinne, S. Casner, R. Frederick and V. Jacobson.

[31]	IETF RFC 0768 (1980): "User Datagram Protocol", J. Postel.

[32]	ITU-T Recommendation G.722 Amendment 1 (2010): "7 kHz audio-coding within 64 kbit/s: New Annex B with superwideband embedded extension."

[33]	ITU-T Recommendation G.720.1 (2010): "Generic sound activity detector."

[34]	American Mathematical Society, (2002): "Numerical Analysis: Mathematics of Scientific Computing", Kincaid, D. and Cheney, W.

[35]	ITU-T Recommendation G.722.2 (2003): "Wideband coding of speech at around 16 kbit/s using Adaptive Multi-Rate Wideband (AMR-WB)."

[36]	3GPP TS 26.201: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; Frame structure".

[37]	3GPP TS 26.171: "Speech codec speech processing functions; Adaptive Multi-Rate - Wideband (AMR-WB) speech codec; General description".

[38]	IETF RFC 3551 (2003): "RTP Profile for Audio and Video Conferences with Minimal Control", H. Schulzrinne and S. Casner.

[39]	3GPP TS 26.103: "Speech codec list for GSM and UMTS"

[40]	3GPP TS 26.244: "3GPP file format (3GP)".

[41]	IETF RFC 3839 (2004): "MIME Type Registrations for 3rd Generation Partnership Project (3GPP) Multimedia files", R. Castagno and D. Singer.

[42]	IEEE Transactions on Information Theory, Vol. 37, no. 6, Nov., pp. 1551-1566 (1991): "Trellis Coded Vector Quantization", T.R. Fischer, M.W. Marcellin, and M. Wang.

[43]	3GPP TS 26.443: "Codec for Enhanced Voice Services (EVS); ANSI C code (floating-point)"

# 3	Definitions, abbreviations and mathematical expressions

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

frame: an array of audio samples spanning 20ms time duration.

EVS codec: The EVS codec includes two operational modes: EVS Primary operational mode (EVS Primary mode) and EVS AMR-WB Inter-Operable modes (EVS AMR-WB IO mode). When using the EVS AMR-WB IO mode the speech frames are bitstream interoperable with the AMR-WB codec [9]. Frames generated by an EVS AMR-WB IO mode encoder can be decoded by an AMR-WB decoder, without the need for transcoding. Likewise, frames generated by an AMR-WB encoder can be decoded by an EVS AMR-WB IO mode decoder, without the need for transcoding.

EVS Primary mode: Includes 11 bit-rates for fixed-rate or multi-rate operation; 1 average bit-rate for variable bit-rate operation; and 1 bit-rate for SID (3GPP TS 26.441 [2]). The EVS Primary mode can encode narrowband, wideband, super-wideband and fullband signals. None of these bit-rates are interoperable with the AMR-WB codec.

EVS AMR-WB IO mode: Includes 9 codec modes and SID. All are bitstream interoperable with the AMR-WB codec (3GPP TS 26.171 ‎[37‎]).

Operational mode: Used for the EVS codec to distinguish between EVS Primary mode and EVS AMR-WB IO mode.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACELP	Algebraic Code-Excited Linear Prediction

AMR	Adaptive Multi Rate (codec)

AMR-NB	Adaptive Multi Rate Narrowband (codec) – Also referred to as AMR

AMR-WB	Adaptive Multi Rate Wideband (codec)

AR	Auto-Regressive

AVQ	Algebraic Vector Quantization

CELP	Code-Excited Linear Prediction

CLDFB	Complex Low Delay Filter Bank

CMR	Codec Mode Request

CNG	Comfort Noise Generator

DCT	Discrete Cosine Transformation

DFT	Discrete Fourier Transform

DTX	Discontinuous Transmission

EFR	Enhanced Full Rate (codec)

EVS	Enhanced Voice Services

FB	Fullband

FCB	Fixed Codebook

FEC	Frame Erasure Concealment

FFT	Fast Fourier Transform

FIR	Finite Impulse Response

FT	Frame Type

GC	Generic Coding

HF	High Frequency

HP	High Pass

IIR	Infinite Impulse Response

IP	Internet Protocol

ISF	Immittance Spectral Frequency

ISP	Immittance Spectral Pair

ISPP	Interleaved Single-Pulse Permutation

JBM	Jitter Buffer Management

LD	Low Delay

LP	Linear Prediction

LPF	Low Pass Filter

LSB	Least Significant Bit

LSF	Line Spectral Frequency

LSP	Line Spectral Pair

LTP	Long-Term Prediction

MA	Moving Average

MDCT	Modified Discrete Cosine Transform

MRLVQ	Multi-Rate Lattice Vector Quantization

MSB	Most Significant Bit

MSVQ	Multi-Stage Vector Quantization

MTSI	Multimedia Telephony Service for IMS

NB	Narrowband

OL	Open-Loop

PCPRS	Path-Choose Pulse Replacement Search

PS	Packet Switched

PSTN	Public Switched Telephone Network

QMF	Quadrature Mirror Filter (See also CLDFB)

SAD	Signal Activity Detection

SDP	Session Description Protocol

SID	Silence Insertion Descriptor

SNR	Signal-to-Noise Ratio

SWB	Super Wideband

TC	Transition Coding

ToC	Table of Contents

UC	Unvoiced Coding

VC	Voiced Coding

VMR-WB	Variable Rate Multimode Wideband

VQ	Vector Quantization

WB	Wideband

WMOPS	Weighted Millions of Operations Per Second

## 3.3	Mathematical Expressions

For the purposes of the present document, the following conventions apply to mathematical expressions:

indicates the largest integer less than or equal to x: ,  and ;

indicates the smallest integer greater than or equal to x: ,  and

|x|	indicates the absolute value of x: |17| = 17, |–17| = 17;

min(x0,…xN–1)	indicates the minimum of x0,…, xN–1, N being the number of components;

max(x0,…xN–1)	indicates the maximum of x0, …, xN–1;

AT	indicates the transpose of matrix A;

indicates the remainder after dividing x by y:  ;

round(x)	is traditional rounding: round(x) =  ;

exp(x)	is equivalent to ex where e is the base of the natural algorithm;

indicates summation;

indicates product;

Unless otherwise specified, log(x) denotes logarithm at base 10 throughout this Recommendation.

# 4	General description of the coder

## 4.1	Introduction

The present document is a detailed description of the signal processing algorithms of the Enhanced Voice Services coder. The detailed mapping from 20ms input blocks of audio samples in 16 bit uniform PCM format to encoded blocks of bits and from encoded blocks of bits to output blocks of reconstructed audio samples is explained. Four sampling rates are supported; 8 000, 16 000, 32 000 and 48 000 samples/s and the bit rates for the encoded bit stream of may be 5.9, 7.2, 8.0, 9.6, 13.2, 16.4, 24.4, 32.0, 48.0, 64.0, 96.0 or 128.0 kbit/s. An AMR-WB Interoperable mode is also provided which operates at bit rates for the encoded bit stream of 6.6, 8.85, 12.65, 14.25, 15.85, 18.25, 19.85, 23.05 or 23.85 kbit/s.

The procedure of this document is mandatory for implementation in all network entities and User Equipment (UE)s supporting the EVS coder.

The present document does not describe the ANSI-C code of this procedure. For a description of the reference fixed-point ANSI C code specifications, see [3]; for a description of the reference floating-point ANSI C code specification see [43].

In the case of discrepancy between the procedure described in the present document and its ANSI-C code specifications contained in [3] the procedure defined by the [3] prevails. In the case of discrepancy between the procedure described in the present document and its ANSI-C code specifications contained in [43] the procedure defined by [43] prevails.

## 4.2	Input/output sampling rate

The encoder can accept fullband (FB), superwideband (SWB), wideband (WB) or narrow-band (NB) signals sampled at 48, 32, 16 or 8 kHz. Similarly, the decoder output can be 48, 32, 16 or 8 kHz, FB, SWB, WB or NB.

## 4.3	Codec delay

The input signal is processed using 20 ms frames. The codec delay depends on the output sampling rate. For WB, SWB and FB output (i.e. output sampling rate > 8 kHz) the overall algorithmic delay is 32 ms. It consists of one 20 ms frame, 0.9375 ms delay of input resampling filters on the encoder-side, 8.75 ms for the encoder look-ahead, and 2.3125 ms delay of time-domain bandwidth extension on the decoder-side. For 8 kHz decoder output the decoder delay is reduced to 1.25 ms needed for resampling using a complex low-delay filterbank, resulting in 30.9375 ms overall algorithmic delay.

## 4.4	Coder overview

The EVS codec employs a hybrid coding scheme combining linear predictive (LP) coding techniques based upon ACELP (Algebraic Code Excited Linear Prediction), predominantly for speech signals, with a transform coding method, for generic content, as well as inactive signal coding in conjunction with VAD/DTX/CNG (Voice Activity Detection/Discontinuous Transmission/ Comfort Noise Generation) operation. The EVS codec is capable of switching between these different coding modes without artefacts.

The EVS codec supports 5.9kbps narrowband and wideband variable bit rate (VBR) operation based upon the ACELP coding paradigm which also provides the AMR-WB interoperable encoding and decoding. In addition to perceptually optimized waveform matching, the codec utilizes parametric representations of certain frequency ranges. These parametric representations constitute coded bandwidth extensions or noise filling strategies.

The decoder generates the signal parameters represented by the indices transmitted in the bit-stream. For the bandwidth extension and noise fill regions estimates from the coded regions are used in addition to the decoded parametric data to generate the signals for these frequency regions.

The description of the coding algorithm of this Specification is made in terms of bit-exact, fixed-point mathematical operations. The ANSI C code described in [3], which constitutes an integral part of this Specification, reflects this bit-exact, fixed-point descriptive approach. The mathematical descriptions of the encoder (clause 5), and decoder (clause 6), can be implemented in different ways, possibly leading to a codec implementation not complying with this Specification. Therefore, the algorithm description of the ANSI C code of [3] shall take precedence over the mathematical descriptions of clauses 5 and 6 whenever discrepancies are found. A non-exhaustive set of test signals that can be used with the ANSI C code are available described in [4].

### 4.4.1	Encoder overview

Figure 1 represents a high-level overview of the encoder. The signal resampling block corrects mismatches between the sampling frequency and the signal bandwidth command line parameter that is specified on the command line or through a file containing a bandwidth switching profile as explained in TS 26.442 and TS 26.443. For the case that the signal bandwidth is lower than half the input sampling frequency, the signal is decimated to a the lowest possible sampling rate out of the set of (8, 16, 32 kHz)  that is larger than twice the signal bandwidth.

Figure 1: Encoder overview

The signal analysis determines which of three possible encoder strategies to employ: LP based coding (ACELP), frequency domain encoding and inactive coding (CNG). In some operational modes the signal analysis step includes a closed loop decision to determine which encoding method will result in the lowest distortion. Further parameters derived in the signal analysis aid the operation of these coding blocks and some of the analysis parameters, such as the coding strategy to be employed, are encoded into the bit-stream. In each of the coding blocks the signal analysis is further refined to obtain parameters relevant for the particular coding block.

The signal analysis and sub-sequent decision of the coding mode is performed independently for each 20ms frame and the switching between different modes is possible on a frame-by-frame basis. In the switching instance parameters are exchanged between the coding modes to ensure that the switching is as seamless as possible and closed-loop methods are sometimes employed in this case. In addition, switching between different bandwidths and/or bit-rates (including both the EVS Primary mode and the EVS AMR-WB IO mode) is possible on frame boundaries.

The signal analysis and all other blocks have full access to the command line parameters such as bit-rate, sampling rate, signal bandwidth, DTX activation as signalling information.

#### 4.4.1.1	Linear Prediction Based Operation

The input signal is split into high frequency band and low frequency band paths; where the cut-off frequency between these two bands is determined from the operational mode (bandwidth and bit-rate) of the codec.

The linear-prediction coefficient estimation is performed for every 20ms frame. Within a frame, several interpolation points are established depending upon the bitrate and the optimum interpolation is transmitted to the decoder. The linear-prediction residual is further analysed and quantized using different quantization schemes dependent upon the nature of the residual. For the 5.9kbps VBR operation additional low-rate coding modes at rates conforming to the design constraints are employed.

The high-frequency portion of the signal is represented with several different parametric representations. The parameters used for this representation vary as a function of the bit-rate and the residual quantization strategy. The transmitted parameters include some or all of spectral envelope, energy information and temporal evolution information.

The LP based core can be configured so that both the linear prediction coefficients and the residual quantization are interoperable with the AMR-WB decoder. For this purpose the configuration of the LP coefficient estimation, parametric HF representation and the residual quantization is similar to those of AMR-WB. For the AMR-WB interoperable operation modes, identical codebooks to the AMR-WB quantizers are used.

Figure 2: Linear prediction based operation including parametric HF representation

#### 4.4.1.2	Frequency Domain Operation

For the frequency domain coding the encoding block can be envisaged as being separated into a control layer and a signal processing layer. The control layer performs signal analysis to derive several control and configuration parameters for the signal processing layer. The time-to-frequency transformation is based on the Modified Discrete Cosine Transform (MDCT) and provides adaptive time-frequency resolution. The control layer derives measures of the time distribution of the signal energy in a frame and controls the transform.

The MDCT coefficients are quantized using a variety of direct and parametric representations depending upon bit rate signal type and operating mode.

Figure 3: Frequency domain encoder

#### 4.4.1.3	Inactive Signal coding

When the codec is operated in DTX on mode the signal classifier depicted in Figure 1 selects the discontinuous transmission (DTX) mode for frames that are determined to consist of background noise. For these frames a low-rate parametric representation of the signal is transmitted no more frequently than every 8 frames (160ms).

The low-rate parametric representation is used in the decoder for comfort noise generation (CNG) and includes parameters describing the frequency envelope of the background signal, energy parameters describing the overall energy and its time evolution.

#### 4.4.1.4	Source Controlled VBR Coding

VBR coding describes a method that assigns different number of bits to a speech frame in the coded domain depending on the characteristics of the input speech signal [20] [21]. This method is often called source-controlled coding as well. Typically, a source-controlled coder encodes speech at different bit rates depending on how the current frame is classified, e.g., voiced, unvoiced, transient, or silence. Note that DTX operation can be combined with VBR coders in the same way as with Fixed Rate (FR) coders; the VBR operation is related to active speech segments.

The VBR solution provides narrowband and wideband coding using the bit rates 2.8, 7.2 and 8.0 kbps and produces an average bit rate at 5.9 kbps.

Due to the finer bit allocation, in comparison to Fixed Rate (FR) coding, VBR offers the advantage of a better speech quality at the same average active bit rate than FR coding at the given bit rate. The benefits of VBR can be exploited if the transmission network supports the transmission of speech frames (packets) of variable size, such as in LTE and UMTS networks.

### 4.4.2	Decoder overview

The decoder receives all quantized parameters and generates a synthesized signal. Thus, for the majority of encoder operations it represents the inverse of the quantized value to index operations.

For the AMR-WB interoperable operation the index lookup is performed using the AMR-WB codebooks and the decoder is configured to generate an improved synthesized signal from the AMR-WB bitstream.

#### 4.4.2.1	Parametric Signal Representation Decoding (Bandwidth Extension)

In addition to the generation of signal components specifically represented by the transmitted indices, the decoder performs estimates of signal regions where the transmitted signal representation is incomplete, i.e. for the parametric signal representations and noise fill as well as blind bandwidth extension in some cases.

#### 4.4.2.2	Frame loss concealment

The EVS codec includes frame loss concealment algorithms [21]. For all coding modes an extrapolation algorithm is in place that estimates the signal in a lost frame. For the LP based core this estimation operates on the last received residual and LP coefficients. For the frequency domain core in some cases the last received MDCT coefficients are extrapolated and in addition the resulting time domain signal is guaranteed to give a smooth time evolution from the last received frame into the missing frames.

Once the frame loss is recovered, i.e., the first good frame is received the codec memory is updated and frame boundary mismatches towards the last lost frame are minimized.

For situations of sustained frame loss the signal is either faded to background noise or its energy is reduced and finally muted when no reasonable extrapolation can be assumed.

The EVS codec also includes the “channel aware” mode, which may be employed for improved performance under packet loss conditions in a VoIP system. In the channel aware mode, partial copies (secondary frames) of the current speech frame are piggybacked on future speech frames (primary frames), without any increase in the total bit rate for the primary and secondary frames. If the current frame is lost, then its partial copy can be retrieved by polling the de-jitter buffer to enable faster and improved recovery from the packet loss.

### 4.4.3	DTX/CNG operation

The codec is equipped with a signal activity detection (SAD) algorithm for classifying each input frame as active or inactive. It supports a discontinuous transmission (DTX) operation in which the comfort noise generation (CNG) module is used to approximate and update the statistics of the background noise at a variable bit rate. Thus, the transmission rate during inactive signal periods is variable and depends on the estimated level of the background noise. By default in the command line the transmission rate of CNG update is fixed to 8 frames. However, the CNG update rate can also be set to another fixed value or a variable rate by means of a command line parameter; when the transmission rate during inactive signal periods is variable, it depends on the estimated level of the background noise.

#### 4.4.3.1	Inactive Signal coding

When the codec is operated in DTX on mode the signal classifier depicted in Figure 1 selects the discontinuous transmission (DTX) mode for frames that are determined to consist of background noise. For these frames a low-rate parametric representation of the signal is transmitted no more frequently than every 8 frames (160ms).

The low-rate parametric representation is used in the decoder for comfort noise generation (CNG) and includes parameters describing the frequency envelope of the background signal, energy parameters describing the overall energy and its time evolution.

### 4.4.4	AMR-WB-interoperable option

As mentioned previously, EVS can operate in a mode which is fully interoperable with the AMR-WB codec bitstream.

### 4.4.5	Channel-Aware Mode

EVS offers partial redundancy [21] based error robust channel aware mode at 13.2 kbps for both wideband and super-wideband audio bandwidths.

In a VoIP system, packets arrive at the decoder with random jitters in their arrival time. Packets may also arrive out of order at the decoder. Since the decoder expects to be fed a speech packet every 20 ms to output speech samples in periodic blocks, a de-jitter buffer is required to absorb the jitter in the packet arrival time. The channel aware mode combines the presence of a de-jitter buffer with partial redundancy coding of a current frame which gets piggy backed onto a future frame. At the receiver, the de-jitter buffer is polled to check if a partial redundant copy of the current lost frame is available in any of the future frames. If present, the partial redundant information is used to synthesize the lost frame which offers significant quality improvements under low to high FER conditions. Source control is used to determine which frames of input can best be coded at a reduced frame rate (called primary frames) to accommodate the attachment of redundancy without altering the total packet size. In this way, the channel aware mode includes redundancy in a constant-bit-rate channel (13.2 kbps).

## 4.5	Organization of the rest of the Technical Standard

In clauses 5 and 6, detailed descriptions of the encoder and the decoder are given. Bit allocation is summarized in clause 7.