3GPP TS 38.473 V17.11.0 (2024-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NG-RAN;

F1 application protocol (F1AP)

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

NG-RAN, Radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	17

1	Scope	18

2	References	18

3	Definitions and abbreviations	20

3.1	Definitions	20

3.2	Abbreviations	21

4	General	22

4.1	Procedure specification principles	22

4.2	Forwards and backwards compatibility	23

4.3	Specification notations	23

5	F1AP services	23

6	Services expected from signalling transport	24

7	Functions of F1AP	24

8	F1AP procedures	24

8.1	List of F1AP Elementary procedures	24

8.2	Interface Management procedures	27

8.2.1	Reset	27

8.2.1.1	General	27

8.2.1.2	Successful Operation	27

8.2.1.2.1	Reset Procedure Initiated from the gNB-CU	27

8.2.1.2.2	Reset Procedure Initiated from the gNB-DU	28

8.2.1.3	Abnormal Conditions	29

8.2.2	Error Indication	29

8.2.2.1	General	29

8.2.2.2	Successful Operation	29

8.2.2.3	Abnormal Conditions	30

8.2.3	F1 Setup	30

8.2.3.1	General	30

8.2.3.2	Successful Operation	30

8.2.3.3	Unsuccessful Operation	32

8.2.3.4	Abnormal Conditions	32

8.2.4	gNB-DU Configuration Update	32

8.2.4.1	General	32

8.2.4.2	Successful Operation	33

8.2.4.3	Unsuccessful Operation	35

8.2.4.4	Abnormal Conditions	35

8.2.5	gNB-CU Configuration Update	36

8.2.5.1	General	36

8.2.5.2	Successful Operation	36

8.2.5.3	Unsuccessful Operation	38

8.2.5.4	Abnormal Conditions	39

8.2.6	gNB-DU Resource Coordination	39

8.2.6.1	General	39

8.2.6.2	Successful Operation	39

8.2.7	gNB-DU Status Indication	39

8.2.7.1	General	39

8.2.7.2	Successful Operation	40

8.2.7.3	Abnormal Conditions	40

8.2.8	F1 Removal	40

8.2.8.1	General	40

8.2.8.2	Successful Operation	41

8.2.8.3	Unsuccessful Operation	41

8.2.8.4	Abnormal Conditions	42

8.2.9	Network Access Rate Reduction	42

8.2.9.1	General	42

8.2.9.2	Successful operation	42

8.2.9.3	Abnormal Conditions	43

8.2.10	Resource Status Reporting Initiation	43

8.2.10.1	General	43

8.2.10.2	Successful Operation	43

8.2.10.3	Unsuccessful Operation	44

8.2.10.4	Abnormal Conditions	44

8.2.11	Resource Status Reporting	45

8.2.11.1	General	45

8.2.11.2	Successful Operation	45

8.2.11.3	Unsuccessful Operation	45

8.2.11.4	Abnormal Conditions	45

8.3	UE Context Management procedures	45

8.3.1	UE Context Setup	45

8.3.1.1	General	45

8.3.1.2	Successful Operation	46

8.3.1.3	Unsuccessful Operation	53

8.3.1.4	Abnormal Conditions	53

8.3.2	UE Context Release Request (gNB-DU initiated)	54

8.3.2.1	General	54

8.3.2.2	Successful Operation	54

8.3.2.3	Abnormal Conditions	54

8.3.3	UE Context Release (gNB-CU initiated)	55

8.3.3.1	General	55

8.3.3.2	Successful Operation	55

8.3.3.4	Abnormal Conditions	55

8.3.4	UE Context Modification (gNB-CU initiated)	56

8.3.4.1	General	56

8.3.4.2	Successful Operation	56

8.3.4.3	Unsuccessful Operation	66

8.3.4.4	Abnormal Conditions	67

8.3.5	UE Context Modification Required (gNB-DU initiated)	67

8.3.5.1	General	67

8.3.5.2	Successful Operation	67

8.3.5.2A	Unsuccessful Operation	69

8.3.5.3	Abnormal Conditions	69

8.3.6	UE Inactivity Notification	69

8.3.6.1	General	69

8.3.6.2	Successful Operation	70

8.3.6.3	Abnormal Conditions	70

8.3.7	Notify	70

8.3.7.1	General	70

8.3.7.2	Successful Operation	70

8.3.7.3	Abnormal Conditions	71

8.3.8	Access Success	71

8.3.8.1	General	71

8.3.8.2	Successful Operation	71

8.3.8.3	Abnormal Conditions	71

8.4	RRC Message Transfer procedures	71

8.4.1	Initial UL RRC Message Transfer	71

8.4.1.1	General	71

8.4.1.2	Successful operation	72

8.4.1.3	Abnormal Conditions	72

8.4.2	DL RRC Message Transfer	72

8.4.2.1	General	72

8.4.2.2	Successful operation	73

8.4.2.3	Abnormal Conditions	73

8.4.3	UL RRC Message Transfer	74

8.4.3.1	General	74

8.4.3.2	Successful operation	74

8.4.3.3	Abnormal Conditions	74

8.4.4	RRC Delivery Report	74

8.4.4.1	General	74

8.4.4.2	Successful operation	75

8.4.4.3	Abnormal Conditions	75

8.5	Warning Message Transmission Procedures	75

8.5.1	Write-Replace Warning	75

8.5.1.1	General	75

8.5.1.2	Successful Operation	75

8.5.1.3	Unsuccessful Operation	76

8.5.1.4	Abnormal Conditions	76

8.5.2	PWS Cancel	76

8.5.2.1	General	76

8.5.2.2	Successful Operation	76

8.5.2.3	Unsuccessful Operation	77

8.5.2.4	Abnormal Conditions	77

8.5.3	PWS Restart Indication	77

8.5.3.1	General	77

8.5.3.2	Successful Operation	77

8.5.3.3	Abnormal Conditions	78

8.5.4	PWS Failure Indication	78

8.5.4.1	General	78

8.5.4.2	Successful Operation	78

8.5.4.3	Abnormal Conditions	78

8.6	System Information Procedures	78

8.6.1	System Information Delivery	78

8.6.1.1	General	78

8.6.1.2	Successful Operation	78

8.6.1.3	Abnormal Conditions	79

8.7	Paging procedures	79

8.7.1	Paging	79

8.7.1.1	General	79

8.7.1.2	Successful Operation	79

8.7.1.3	Abnormal Conditions	80

8.8	Trace Procedures	80

8.8.1	Trace Start	80

8.8.1.1	General	80

8.8.1.2	Successful Operation	80

8.8.1.3	Abnormal Conditions	81

8.8.2	Deactivate Trace	81

8.8.2.1	General	81

8.8.2.2	Successful Operation	81

8.8.2.3	Abnormal Conditions	81

8.8.3	Cell Traffic Trace	81

8.8.3.1	General	81

8.8.3.2	Successful Operation	82

8.8.3.3	Abnormal Conditions	82

8.9	Radio Information Transfer procedures	82

8.9.1	DU-CU Radio Information Transfer	82

8.9.1.1	General	82

8.9.1.2	Successful operation	82

8.9.1.3	Abnormal Conditions	82

8.9.2	CU-DU Radio Information Transfer	83

8.9.2.1	General	83

8.9.2.2	Successful operation	83

8.9.2.3	Abnormal Conditions	83

8.10	IAB Procedures	83

8.10.0	General	83

8.10.1	BAP Mapping Configuration	83

8.10.1.1	General	83

8.10.1.2	Successful Operation	83

8.10.1.A	Unsuccessful Operation	85

8.10.1.3	Abnormal Conditions	85

8.10.2	gNB-DU Resource Configuration	85

8.10.2.1	General	85

8.10.2.2	Successful Operation	85

8.10.2.B	Unsuccessful Operation	86

8.10.2.3	Abnormal Conditions	86

8.10.3	IAB TNL Address Allocation	86

8.10.3.1	General	86

8.10.3.2	Successful Operation	87

8.10.3.C	Unsuccessful Operation	87

8.10.3.3	Abnormal Conditions	88

8.10.4	IAB UP Configuration Update	88

8.10.4.1	General	88

8.10.4.2	Successful Operation	88

8.10.4.3	Unsuccessful Operation	89

8.10.4.4	Abnormal Conditions	89

8.11	Self Optimisation Support procedures	89

8.11.1	Access and Mobility Indication	89

8.11.1.1	General	89

8.11.1.2	Successful Operation	89

8.11.1.3	Abnormal Conditions	90

8.12	Reference Time Information Reporting procedures	90

8.12.1	Reference Time Information Reporting Control	90

8.12.1.1	General	90

8.12.1.2	Successful Operation	90

8.12.1.3	Abnormal Conditions	90

8.12.2	Reference Time Information Report	90

8.12.2.1	General	90

8.12.2.2	Successful Operation	91

8.12.2.3	Abnormal Conditions	91

8.13	Positioning Procedures	91

8.13.1	Positioning Assistance Information Control	91

8.13.1.1	General	91

8.13.1.2	Successful Operation	91

8.13.1.3	Abnormal Conditions	92

8.13.2	Positioning Assistance Information Feedback	92

8.13.2.1	General	92

8.13.2.2	Successful Operation	92

8.13.2.3	Abnormal Conditions	92

8.13.3	Positioning Measurement	93

8.13.3.1	General	93

8.13.3.2	Successful Operation	93

8.13.3.3	Unsuccessful Operation	94

8.13.3.4	Abnormal Conditions	94

8.13.4	Positioning Measurement Report	94

8.13.4.1	General	94

8.13.4.2	Successful Operation	94

8.13.4.3	Unsuccessful Operation	95

8.13.4.4	Abnormal Conditions	95

8.13.5	Positioning Measurement Abort	95

8.13.5.1	General	95

8.13.5.2	Successful Operation	95

8.13.5.3	Unsuccessful Operation	95

8.13.5.4	Abnormal Conditions	95

8.13.6	Positioning Measurement Failure Indication	95

8.13.6.1	General	95

8.13.6.2	Successful Operation	96

8.13.6.3	Unsuccessful Operation	96

8.13.6.4	Abnormal Conditions	96

8.13.7	Positioning Measurement Update	96

8.13.7.1	General	96

8.13.7.2	Successful Operation	96

8.13.7.3	Unsuccessful Operation	97

8.13.7.4	Abnormal Conditions	97

8.13.8	TRP Information Exchange	97

8.13.8.1	General	97

8.13.8.2	Successful Operation	97

8.13.8.3	Unsuccessful Operation	98

8.13.9	Positioning Information Exchange	98

8.13.9.1	General	98

8.13.9.2	Successful Operation	98

8.13.9.3	Unsuccessful Operation	99

8.13.10	Positioning Activation	99

8.13.10.1	General	99

8.13.10.2	Successful Operation	99

8.13.10.3	Unsuccessful Operation	100

8.13.10.4	Abnormal Conditions	100

8.13.11	Positioning Deactivation	100

8.13.11.1	General	100

8.13.11.2	Successful Operation	100

8.13.11.3	Unsuccessful Operation	101

8.13.11.4	Abnormal Conditions	101

8.13.12	E-CID Measurement Initiation	101

8.13.12.1	General	101

8.13.12.2	Successful Operation	101

8.13.12.3	Unsuccessful Operation	102

8.13.13	E-CID Measurement Failure Indication	102

8.13.13.1	General	102

8.13.13.2	Successful Operation	102

8.13.13.3	Unsuccessful Operation	102

8.13.14	E-CID Measurement Report	102

8.13.14.1	General	102

8.13.14.2	Successful Operation	103

8.13.14.3	Unsuccessful Operation	103

8.13.15	E-CID Measurement Termination	103

8.13.15.1	General	103

8.13.15.2	Successful Operation	103

8.13.15.3	Unsuccessful Operation	103

8.13.16	Positioning Information Update	104

8.13.16.1	General	104

8.13.16.2	Successful Operation	104

8.13.16.3	Unsuccessful Operation	104

8.13.16.4	Abnormal Conditions	104

8.13.17	PRS Configuration Exchange	104

8.13.17.1	General	104

8.13.17.2	Successful Operation	105

8.13.17.3	Unsuccessful Operation	105

8.13.17.4	Abnormal Conditions	105

8.13.18	Measurement Preconfiguration	105

8.13.18.1	General	105

8.13.18.2	Successful Operation	106

8.13.18.3	Unsuccessful Operation	106

8.13.19	Measurement Activation	106

8.13.19.1	General	106

8.13.19.2	Successful Operation	107

8.13.19.3	Unsuccessful Operation	107

8.13.20	Positioning System Information Delivery	107

8.13.20.1	General	107

8.13.20.2	Successful Operation	107

8.13.20.3	Abnormal Conditions	108

8.14	NR MBS Procedures	108

8.14.1	Broadcast Context Setup	108

8.14.1.1	General	108

8.14.1.2	Successful Operation	108

8.14.1.3	Unsuccessful Operation	109

8.14.1.4	Abnormal Conditions	109

8.14.2	Broadcast Context Release	109

8.14.2.1	General	109

8.14.2.2	Successful Operation	109

8.14.2.3	Unsuccessful Operation	110

8.14.2.4	Abnormal Conditions	110

8.14.3	Broadcast Context Release Request	110

8.14.3.1	General	110

8.14.3.2	Successful Operation	110

8.14.3.3	Unsuccessful Operation	110

8.14.3.4	Abnormal Conditions	110

8.14.4	Broadcast Context Modification	110

8.14.4.1	General	110

8.14.4.2	Successful Operation	111

8.14.4.3	Unsuccessful Operation	112

8.14.4.4	Abnormal Conditions	112

8.14.5	Multicast Group Paging	112

8.14.5.1	General	112

8.14.5.2	Successful Operation	112

8.14.5.3	Abnormal Conditions	113

8.14.6	Multicast Context Setup	113

8.14.6.1	General	113

8.14.6.2	Successful Operation	113

8.14.6.3	Unsuccessful Operation	114

8.14.6.4	Abnormal Conditions	114

8.14.7	Multicast Context Release	114

8.14.7.1	General	114

8.14.7.2	Successful Operation	114

8.14.7.3	Unsuccessful Operation	115

8.14.7.4	Abnormal Conditions	115

8.14.8	Multicast Context Release Request	115

8.14.8.1	General	115

8.14.8.2	Successful Operation	115

8.14.8.3	Unsuccessful Operation	115

8.14.8.4	Abnormal Conditions	115

8.14.9	Multicast Context Modification	115

8.14.9.1	General	115

8.14.9.2	Successful Operation	116

8.14.9.3	Unsuccessful Operation	117

8.14.9.4	Abnormal Conditions	117

8.14.10	Multicast Distribution Setup	117

8.14.10.1	General	117

8.14.10.2	Successful Operation	117

8.14.10.3	Unsuccessful Operation	118

8.14.10.4	Abnormal Conditions	118

8.14.11	Multicast Distribution Release	118

8.14.11.1	General	118

8.14.11.2	Successful Operation	118

8.14.11.3	Unsuccessful Operation	119

8.14.11.4	Abnormal Conditions	119

8.15	PDC Measurement Reporting procedures	119

8.15.1	PDC Measurement Initiation	119

8.15.1.1	General	119

8.15.1.2	Successful Operation	119

8.15.1.3	Unsuccessful Operation	120

8.15.2	PDC Measurement Report	120

8.15.2.1	General	120

8.15.2.2	Successful Operation	120

8.15.2.3	Unsuccessful Operation	120

8.15.3	PDC Measurement Termination	120

8.15.3.1	General	120

8.15.3.2	Successful Operation	121

8.15.3.3	Unsuccessful Operation	121

8.15.3.4	Abnormal Conditions	121

8.15.4	PDC Measurement Failure Indication	121

8.15.4.1	General	121

8.15.4.2	Successful Operation	121

8.15.4.3	Unsuccessful Operation	121

8.15.4.4	Abnormal Conditions	121

8.16	QMC Procedures	122

8.16.1	QoE Information Transfer	122

8.16.1.1	General	122

8.16.1.2	Successful operation	122

8.16.1.3	Abnormal Conditions	122

9	Elements for F1AP Communication	122

9.1	General	122

9.2	Message Functional Definition and Content	123

9.2.1	Interface Management messages	123

9.2.1.1	RESET	123

9.2.1.2	RESET ACKNOWLEDGE	123

9.2.1.3	ERROR INDICATION	124

9.2.1.4	F1 SETUP REQUEST	124

9.2.1.5	F1 SETUP RESPONSE	125

9.2.1.6	F1 SETUP FAILURE	126

9.2.1.7	GNB-DU CONFIGURATION UPDATE	126

9.2.1.8	GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE	128

9.2.1.9	GNB-DU CONFIGURATION UPDATE FAILURE	129

9.2.1.10	GNB-CU CONFIGURATION UPDATE	129

9.2.1.11	GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE	132

9.2.1.12	GNB-CU CONFIGURATION UPDATE FAILURE	133

9.2.1.13	GNB-DU RESOURCE COORDINATION REQUEST	133

9.2.1.14	GNB-DU RESOURCE COORDINATION RESPONSE	134

9.2.1.15	GNB-DU STATUS INDICATION	135

9.2.1.16	F1 REMOVAL REQUEST	135

9.2.1.17	F1 REMOVAL RESPONSE	135

9.2.1.18	F1 REMOVAL FAILURE	135

9.2.1.19	NETWORK ACCESS RATE REDUCTION	136

9.2.1.20	RESOURCE STATUS REQUEST	136

9.2.1.21	RESOURCE STATUS RESPONSE	137

9.2.1.22	RESOURCE STATUS FAILURE	138

9.2.1.23	RESOURCE STATUS UPDATE	138

9.2.2	UE Context Management messages	139

9.2.2.1	UE CONTEXT SETUP REQUEST	139

9.2.2.2	UE CONTEXT SETUP RESPONSE	147

9.2.2.3	UE CONTEXT SETUP FAILURE	151

9.2.2.4	UE CONTEXT RELEASE REQUEST	151

9.2.2.5	UE CONTEXT RELEASE COMMAND	152

9.2.2.6	UE CONTEXT RELEASE COMPLETE	153

9.2.2.7	UE CONTEXT MODIFICATION REQUEST	153

9.2.2.8	UE CONTEXT MODIFICATION RESPONSE	166

9.2.2.9	UE CONTEXT MODIFICATION FAILURE	172

9.2.2.10	UE CONTEXT MODIFICATION REQUIRED	173

9.2.2.11	UE CONTEXT MODIFICATION CONFIRM	176

9.2.2.11A	UE CONTEXT MODIFICATION REFUSE	178

9.2.2.12	UE INACTIVITY NOTIFICATION	178

9.2.2.13	 NOTIFY	179

9.2.2.14	ACCESS SUCCESS	179

9.2.3	RRC Message Transfer messages	179

9.2.3.1	INITIAL UL RRC MESSAGE TRANSFER	179

9.2.3.2	DL RRC MESSAGE TRANSFER	180

9.2.3.3	UL RRC MESSAGE TRANSFER	181

9.2.3.4	RRC DELIVERY REPORT	182

9.2.4	Warning Message Transmission Messages	182

9.2.4.1	WRITE-REPLACE WARNING REQUEST	182

9.2.4.2	WRITE-REPLACE WARNING RESPONSE	183

9.2.4.3	PWS CANCEL REQUEST	183

9.2.4.4	PWS CANCEL RESPONSE	184

9.2.4.5	PWS RESTART INDICATION	185

9.2.4.6	PWS FAILURE INDICATION	185

9.2.5	System Information messages	185

9.2.5.1	SYSTEM INFORMATION DELIVERY COMMAND	185

9.2.6	Paging messages	186

9.2.6.1	PAGING	186

9.2.7	Trace Messages	187

9.2.7.1	TRACE START	187

9.2.7.2	DEACTIVATE TRACE	187

9.2.7.3	CELL TRAFFIC TRACE	187

9.2.8	Radio Information Transfer messages	188

9.2.8.1	DU-CU RADIO INFORMATION TRANSFER	188

9.2.8.2	CU-DU RADIO INFORMATION TRANSFER	188

9.2.9	IAB messages	189

9.2.9.1	BAP MAPPING CONFIGURATION	189

9.2.9.2	BAP MAPPING CONFIGURATION ACKNOWLEDGE	190

9.2.9.2A	BAP MAPPING CONFIGURATION FAILURE	190

9.2.9.3	GNB-DU RESOURCE CONFIGURATION	191

9.2.9.4	GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE	195

9.2.9.4A	GNB-DU RESOURCE CONFIGURATION FAILURE	195

9.2.9.5	IAB TNL ADDRESS REQUEST	195

9.2.9.6	IAB TNL ADDRESS RESPONSE	196

9.2.9.6A	IAB TNL ADDRESS FAILURE	197

9.2.9.7	IAB UP CONFIGURATION UPDATE REQUEST	197

9.2.9.8	IAB UP CONFIGURATION UPDATE RESPONSE	198

9.2.9.9	IAB UP CONFIGURATION UPDATE FAILURE	198

9.2.10	Self Optimisation Support Messages	199

9.2.10.1	ACCESS AND MOBILITY INDICATION	199

9.2.11	Reference Time Information Reporting messages	199

9.2.11.1	REFERENCE TIME INFORMATION REPORTING CONTROL	200

9.2.11.2	REFERENCE TIME INFORMATION REPORT	200

9.2.12	Messages for Positioning Procedures	200

9.2.12.1	POSITIONING ASSISTANCE INFORMATION CONTROL	200

9.2.12.2	POSITIONING ASSISTANCE INFORMATION FEEDBACK	200

9.2.12.3	POSITIONING MEASUREMENT REQUEST	201

9.2.12.4	POSITIONING MEASUREMENT RESPONSE	203

9.2.12.5	POSITIONING MEASUREMENT FAILURE	203

9.2.12.6	POSITIONING MEASUREMENT REPORT	204

9.2.12.7	POSITIONING MEASUREMENT ABORT	204

9.2.12.8	POSITIONING MEASUREMENT FAILURE INDICATION	204

9.2.12.9	POSITIONING MEASUREMENT UPDATE	205

9.2.12.10	TRP INFORMATION REQUEST	205

9.2.12.11	TRP INFORMATION RESPONSE	206

9.2.12.12	TRP INFORMATION FAILURE	206

9.2.12.13	POSITIONING INFORMATION REQUEST	207

9.2.12.14	POSITIONING INFORMATION RESPONSE	207

9.2.12.15	POSITIONING INFORMATION FAILURE	207

9.2.12.16	POSITIONING ACTIVATION REQUEST	208

9.2.12.17	POSITIONING ACTIVATION RESPONSE	208

9.2.12.18	POSITIONING ACTIVATION FAILURE	208

9.2.12.19	POSITIONING DEACTIVATION	209

9.2.12.20	E-CID MEASUREMENT INITIATION REQUEST	209

9.2.12.21	E-CID MEASUREMENT INITIATION RESPONSE	210

9.2.12.22	E-CID MEASUREMENT INITIATION FAILURE	210

9.2.12.23	E-CID MEASUREMENT FAILURE INDICATION	211

9.2.12.24	E-CID MEASUREMENT REPORT	211

9.2.12.25	E-CID MEASUREMENT TERMINATION COMMAND	211

9.2.12.26	POSITIONING INFORMATION UPDATE	212

9.2.12.27	PRS CONFIGURATION REQUEST	212

9.2.12.28	PRS CONFIGURATION RESPONSE	212

9.2.12.29	PRS CONFIGURATION FAILURE	213

9.2.12.30	MEASUREMENT PRECONFIGURATION REQUIRED	213

9.2.12.31	MEASUREMENT PRECONFIGURATION CONFIRM	213

9.2.12.32	MEASUREMENT PRECONFIGURATION REFUSE	214

9.2.12.33	MEASUREMENT ACTIVATION	214

9.2.12.34	POSITIONING SYSTEM INFORMATION DELIVERY COMMAND	215

9.2.13	Broadcast Context Management messages	215

9.2.13.1	BROADCAST CONTEXT SETUP REQUEST	215

9.2.13.2	BROADCAST CONTEXT SETUP RESPONSE	216

9.2.13.3	BROADCAST CONTEXT SETUP FAILURE	216

9.2.13.4	BROADCAST CONTEXT RELEASE COMMAND	217

9.2.13.5	BROADCAST CONTEXT RELEASE COMPLETE	217

9.2.13.5a	BROADCAST CONTEXT RELEASE REQUEST	217

9.2.13.6	BROADCAST CONTEXT MODIFICATION REQUEST	217

9.2.13.7	BROADCAST CONTEXT MODIFICATION RESPONSE	219

9.2.13.8	BROADCAST CONTEXT MODIFICATION FAILURE	220

9.2.14	Multicast Context Management messages	220

9.2.14.1	MULTICAST GROUP PAGING	220

9.2.14.2	MULTICAST CONTEXT SETUP REQUEST	221

9.2.14.3	MULTICAST CONTEXT SETUP RESPONSE	221

9.2.14.4	MULTICAST CONTEXT SETUP FAILURE	222

9.2.14.5	MULTICAST CONTEXT RELEASE COMMAND	222

9.2.14.6	MULTICAST CONTEXT RELEASE COMPLETE	222

9.2.14.6a	MULTICAST CONTEXT RELEASE REQUEST	223

9.2.14.7	MULTICAST CONTEXT MODIFICATION REQUEST	223

9.2.14.8	MULTICAST CONTEXT MODIFICATION RESPONSE	224

9.2.14.9	MULTICAST CONTEXT MODIFICATION FAILURE	225

9.2.14.10	MULTICAST DISTRIBUTION SETUP REQUEST	225

9.2.14.11	MULTICAST DISTRIBUTION SETUP RESPONSE	225

9.2.14.12	MULTICAST DISTRIBUTION SETUP FAILURE	226

9.2.14.13	MULTICAST DISTRIBUTION RELEASE COMMAND	226

9.2.14.14	MULTICAST DISTRIBUTION RELEASE COMPLETE	227

9.2.15	PDC Measurement Reporting messages	227

9.2.15.1	PDC MEASUREMENT INITIATION REQUEST	227

9.2.15.2	PDC MEASUREMENT INITIATION RESPONSE	228

9.2.15.3	PDC MEASUREMENT INITIATION FAILURE	228

9.2.15.4	PDC MEASUREMENT REPORT	228

9.2.15.5	PDC MEASUREMENT TERMINATION COMMAND	228

9.2.15.6	PDC MEASUREMENT FAILURE INDICATION	229

9.2.16	QMC messages	229

9.2.16.1	QOE INFORMATION TRANSFER	229

9.3	Information Element Definitions	229

9.3.1	Radio Network Layer Related IEs	229

9.3.1.1	Message Type	229

9.3.1.2	Cause	230

9.3.1.3	Criticality Diagnostics	234

9.3.1.4	gNB-CU UE F1AP ID	234

9.3.1.5	gNB-DU UE F1AP ID	234

9.3.1.6	RRC-Container	235

9.3.1.7	SRB ID	235

9.3.1.8	DRB ID	235

9.3.1.9	gNB-DU ID	235

9.3.1.10	Served Cell Information	235

9.3.1.11	Transmission Action Indicator	240

9.3.1.12	NR CGI	240

9.3.1.13	Time To wait	241

9.3.1.14	PLMN Identity	241

9.3.1.15	Transmission Bandwidth	241

9.3.1.16	Void	242

9.3.1.17	NR Frequency Info	242

9.3.1.18	gNB-DU System Information	243

9.3.1.19	E-UTRAN QoS	244

9.3.1.20	Allocation and Retention Priority	244

9.3.1.21	GBR QoS Information	245

9.3.1.22	Bit Rate	245

9.3.1.23	Transaction ID	246

9.3.1.24	DRX Cycle	246

9.3.1.25	CU to DU RRC Information	246

9.3.1.26	DU to CU RRC Information	248

9.3.1.27	RLC Mode	253

9.3.1.28	SUL Information	253

9.3.1.29	5GS TAC	254

9.3.1.29a	Configured EPS TAC	254

9.3.1.30	RRC Reconfiguration Complete Indicator	254

9.3.1.31	UL Configuration	254

9.3.1.32	C-RNTI	254

9.3.1.33	Cell UL Configured	254

9.3.1.34	RAT-Frequency Priority Information	255

9.3.1.35	LCID	255

9.3.1.36	Duplication activation	255

9.3.1.37	Slice Support List	255

9.3.1.38	S-NSSAI	256

9.3.1.39	UE Identity Index value	256

9.3.1.40	Paging DRX	256

9.3.1.41	Paging Priority	256

9.3.1.42	gNB-CU System Information	256

9.3.1.43	RAN UE Paging identity	257

9.3.1.44	CN UE Paging Identity	257

9.3.1.45	QoS Flow Level QoS Parameters	257

9.3.1.46	GBR QoS Flow Information	258

9.3.1.47	Dynamic 5QI Descriptor	259

9.3.1.48	NG-RAN Allocation and Retention Priority	261

9.3.1.49	Non Dynamic 5QI Descriptor	261

9.3.1.50	Maximum Packet Loss Rate	262

9.3.1.51	Packet Delay Budget	262

9.3.1.52	Packet Error Rate	263

9.3.1.53	Averaging Window	263

9.3.1.54	Maximum Data Burst Volume	263

9.3.1.55	Masked IMEISV	263

9.3.1.56	Notification Control	263

9.3.1.57	RAN Area Code	264

9.3.1.58	PWS System Information	264

9.3.1.59	Repetition Period	264

9.3.1.60	Number of Broadcasts Requested	264

9.3.1.61	Void	264

9.3.1.62	SIType List	264

9.3.1.63	QoS Flow Identifier	265

9.3.1.64	Served E-UTRA Cell Information	265

9.3.1.65	Available PLMN List	265

9.3.1.66	RLC Failure Indication	266

9.3.1.67	Uplink TxDirectCurrentList Information	266

9.3.1.68	Service Status	266

9.3.1.69	RLC Status	266

9.3.1.70	RRC Version	266

9.3.1.71	RRC Delivery Status	267

9.3.1.72	QoS Flow Mapping Indication	267

9.3.1.73	Resource Coordination Transfer Information	267

9.3.1.74	E-UTRA PRACH Configuration	267

9.3.1.75	Resource Coordination E-UTRA Cell Information	268

9.3.1.76	Extended Available PLMN List	269

9.3.1.77	Associated SCell List	269

9.3.1.78	Cell Direction	270

9.3.1.79	Paging Origin	270

9.3.1.80	E-UTRA Transmission Bandwidth	270

9.3.1.81	Message Identifier	270

9.3.1.82	Serial Number	270

9.3.1.83	UAC Assistance Information	270

9.3.1.84	UAC Action	271

9.3.1.85	 UAC reduction Indication	272

9.3.1.86	Additional SIB Message List	272

9.3.1.87	Cell Type	272

9.3.1.87a	Configured TAC Indication	272

9.3.1.88	Trace Activation	273

9.3.1.89	Intended TDD DL-UL Configuration	274

9.3.1.90	Additional RRM Policy Index	275

9.3.1.91	DU-CU RIM Information	275

9.3.1.92	CU-DU RIM Information	275

9.3.1.93	gNB Set ID	276

9.3.1.94	Lower Layer Presence Status Change	276

9.3.1.95	Traffic Mapping Information	276

9.3.1.96	IP-to-layer-2 traffic mapping Information List	276

9.3.1.97	IP Header Information	277

9.3.1.98	BAP layer BH RLC channel mapping Information List	277

9.3.1.99	Mapping Information to Remove	278

9.3.1.100	Mapping Information Index	278

9.3.1.101	IAB TNL Addresses Requested	278

9.3.1.102	IAB TNL Address	279

9.3.1.103	Uplink BH Non-UP Traffic Mapping	279

9.3.1.104	Non-UP Traffic Type	279

9.3.1.105	IAB Info IAB-donor-CU	280

9.3.1.106	IAB Info IAB-DU	280

9.3.1.107	gNB-DU Cell Resource Configuration	280

9.3.1.108	Multiplexing Info	283

9.3.1.109	IAB STC Info	284

9.3.1.110	BAP Routing ID	285

9.3.1.111	BAP Address	285

9.3.1.112	BAP Path ID	285

9.3.1.113	BH RLC Channel ID	286

9.3.1.114	BH Information	286

9.3.1.115	Control Plane Traffic Type	287

9.3.1.116	NR V2X Services Authorized	287

9.3.1.117	LTE V2X Services Authorized	287

9.3.1.118	LTE UE Sidelink Aggregate Maximum Bit Rate	287

9.3.1.119	NR UE Sidelink Aggregate Maximum Bit Rate	288

9.3.1.120	SL DRB ID	288

9.3.1.121	PC5 QoS Flow Identifier	288

9.3.1.122	PC5 QoS Parameters	288

9.3.1.123	Alternative QoS Parameters Set Index	289

9.3.1.124	Alternative QoS Parameters Set Notify Index	289

9.3.1.125	Alternative QoS Parameters Set List	289

9.3.1.126	Non Dynamic PQI Descriptor	289

9.3.1.127	Dynamic PQI Descriptor	290

9.3.1.128	TNL Capacity Indicator	290

9.3.1.129	Radio Resource Status	291

9.3.1.130	Composite Available Capacity Group	293

9.3.1.131	Composite Available Capacity	293

9.3.1.132	Cell Capacity Class Value	293

9.3.1.133	Capacity Value	294

9.3.1.134	Slice Available Capacity	294

9.3.1.135	Number of Active UEs	295

9.3.1.136	Hardware Load Indicator	295

9.3.1.137	NR Carrier List	295

9.3.1.138	SSB Positions In Burst	296

9.3.1.139	NR PRACH Configuration	296

9.3.1.140	NR PRACH Configuration List	296

9.3.1.141	TSC Traffic Characteristics	298

9.3.1.142	TSC Assistance Information	299

9.3.1.143	Periodicity	299

9.3.1.144	Burst Arrival Time	299

9.3.1.145	Extended Packet Delay Budget	299

9.3.1.146	RLC Duplication Information	299

9.3.1.147	Reporting Request Type	300

9.3.1.148	Time Reference Information	300

9.3.1.149	Reference Time	300

9.3.1.150	MDT Configuration	301

9.3.1.151	MDT PLMN List	301

9.3.1.152	M5 Configuration	302

9.3.1.153	M6 Configuration	302

9.3.1.154	M7 Configuration	302

9.3.1.155	NID	303

9.3.1.156	NPN Support Information	303

9.3.1.157	NPN Broadcast Information	303

9.3.1.158	Broadcast SNPN ID List	303

9.3.1.159	Broadcast NID List	304

9.3.1.160	Broadcast CAG-Identifier List	304

9.3.1.161	CAG ID	304

9.3.1.162	Broadcast PNI-NPN ID Information	304

9.3.1.163	Available SNPN ID List	304

9.3.1.164	Void	305

9.3.1.165	Extended Slice Support List	305

9.3.1.166	Positioning Measurement Result	305

9.3.1.167	UL Angle of Arrival	306

9.3.1.168	UL RTOA Measurement	306

9.3.1.169	Additional Path List	307

9.3.1.170	gNB Rx-Tx Time Difference	307

9.3.1.171	Time Stamp	308

9.3.1.172	TRP Measurement Quality	308

9.3.1.173	Measurement Beam Information	309

9.3.1.174	NG-RAN Access Point Position	309

9.3.1.175	Requested SRS Transmission Characteristics	309

9.3.1.176	TRP Information	311

9.3.1.177	PRS Configuration	312

9.3.1.178	DL-PRS Muting Pattern	313

9.3.1.179	Spatial Direction Information	314

9.3.1.180	SRS Resource Set ID	314

9.3.1.181	Spatial Relation Information	314

9.3.1.182	SRS Resource Trigger	314

9.3.1.183	Relative Time 1900	315

9.3.1.184	Geographical Coordinates	315

9.3.1.185	DL-PRS Resource Coordinates	316

9.3.1.186	Relative Geodetic Location	317

9.3.1.187	Relative Cartesian Location	317

9.3.1.188	Reference Point	317

9.3.1.189	Location Uncertainty	318

9.3.1.190	NG-RAN High Accuracy Access Point Position	318

9.3.1.191	Positioning Broadcast Cells	318

9.3.1.192	 SRS Configuration	319

9.3.1.193	SRS Resource	320

9.3.1.194	Positioning SRS Resource	321

9.3.1.195	SRS Resource Set	322

9.3.1.196	Positioning SRS Resource Set	323

9.3.1.197	TRP ID	323

9.3.1.198	NR-PRS Beam Information	324

9.3.1.199	E-CID Measurement Result	325

9.3.1.200	Cell Portion ID	325

9.3.1.201	Pathloss Reference Information	325

9.3.1.202	SSB Information	326

9.3.1.203	SSB Time/Frequency Configuration	326

9.3.1.204	Search Window Information	326

9.3.1.205	Extended gNB-DU Name	327

9.3.1.206	Extended gNB-CU Name	327

9.3.1.207	F1-C Transfer Path	327

9.3.1.208	SFN Offset	327

9.3.1.209	Transmission Stop Indicator	328

9.3.1.210	Spatial Relation Information per SRS Resource	328

9.3.1.211	CCO Assistance Information	328

9.3.1.212	Affected Cells and Beams	329

9.3.1.213	Coverage Modification Notification	329

9.3.1.214	Cells for SON List	330

9.3.1.215	Neighbour NR Cells for SON List	330

9.3.1.216	NR Mode Info Rel16	330

9.3.1.217	Frequency Info Rel16	331

9.3.1.218	MBS Session ID	331

9.3.1.219	gNB-CU MBS F1AP ID	331

9.3.1.220	gNB-DU MBS F1AP ID	332

9.3.1.221	MBS Area Session ID	332

9.3.1.222	MBS Service Area	332

9.3.1.223	MBS Service Area information	332

9.3.1.224	MRB ID	333

9.3.1.225	MBS CU to DU RRC Information	333

9.3.1.226	MBS Broadcast Neighbour Cell List	333

9.3.1.227	IAB Congestion Indication	334

9.3.1.228	F1-C Transfer Path NRDC	334

9.3.1.229	IAB TNL Address Exception	334

9.3.1.230	RB Set Configuration	334

9.3.1.231	Survival Time	335

9.3.1.232	PDC Measurement Result	335

9.3.1.233	SCG Activation Request	336

9.3.1.234	SCG Activation Status	336

9.3.1.235	Requested DL PRS Transmission Characteristics	336

9.3.1.236	Start Time and Duration	337

9.3.1.237	PRS Transmission Off Information	337

9.3.1.238	UL-AoA Assistance Information	337

9.3.1.239	Zenith Angle of Arrival Information	338

9.3.1.240	On-demand PRS TRP Information	338

9.3.1.241	LCS to GCS Translation	340

9.3.1.242	Response Time	340

9.3.1.243	ARP Location Information	340

9.3.1.244	ARP ID	341

9.3.1.245	Multiple UL AoA	341

9.3.1.246	UL SRS-RSRPP	341

9.3.1.247	SRS Resource type	341

9.3.1.248	Extended Additional Path List	342

9.3.1.249	LoS/NLoS Information	342

9.3.1.250	Requested DL-PRS Resource List	343

9.3.1.251	Void	343

9.3.1.252	TRP Tx TEG Association	343

9.3.1.253	TRP TEG Information	343

9.3.1.254	Measurement Characteristics Request Indicator	344

9.3.1.255	UE Reporting Information	344

9.3.1.256	TRP Beam Antenna Information	345

9.3.1.257	TRP Beam Antenna Angles	345

9.3.1.258	NR Paging eDRX Information	346

9.3.1.259	NR Paging eDRX Information for RRC INACTIVE	347

9.3.1.260	QoE Metrics	347

9.3.1.261	CG-SDT Session Info	347

9.3.1.262	SDT Information	347

9.3.1.263	Path Switch Configuration	347

9.3.1.264	Sidelink Relay Configuration	348

9.3.1.265	PC5 RLC Channel ID	348

9.3.1.266	Uu RLC Channel ID	348

9.3.1.267	Remote UE Local ID	348

9.3.1.268	5G ProSe Authorized	349

9.3.1.269	PEIPS Assistance Information	349

9.3.1.270	UE Paging Capability	349

9.3.1.271	gNB-DU UE Slice Maximum Bit Rate List	349

9.3.1.272	Multicast MBS Session List	350

9.3.1.273	TAI NSAG Support List	350

9.3.1.274	MDT PLMN Modification List	350

9.3.1.275	MRB RLC Configuration	350

9.3.1.276	Timing Error Margin	351

9.3.1.277	SDT Bearer Configuration Info	351

9.3.1.278	PosSIType List	352

9.3.1.279	IAB-DU Cell Resource Configuration-Mode-Info	352

9.3.1.280	TRP Rx TEG Information	353

9.3.1.281	TRP Tx TEG Information	353

9.3.1.282	TRP RxTx TEG Information	353

9.3.1.283	Uplink TxDirectCurrentTwoCarrierList Information	354

9.3.1.284	Uplink TxDirectCurrentMoreCarrierList Information	354

9.3.1.285	Extended UE Identity Index Value	354

9.3.1.286	Hashed UE Identity Index Value	354

9.3.1.287	Broadcast Area Scope	354

9.3.2	Transport Network Layer Related IEs	355

9.3.2.1	UP Transport Layer Information	355

9.3.2.2	GTP-TEID	355

9.3.2.3	Transport Layer Address	355

9.3.2.4	CP Transport Layer Information	356

9.3.2.5	Transport Layer Address Info	356

9.3.2.6	URI	357

9.3.2.7	BC Bearer Context F1-U TNL Info	357

9.3.2.8	MBS Multicast F1-U Context Descriptor	357

9.3.2.9	Void	358

9.3.2.10	MBS PTP Retransmission Tunnel Required	358

9.3.2.11	Multicast F1-U Context Reference F1	358

9.3.2.12	MRB Progress Information	358

9.3.2.13	Multicast F1-U Context Reference CU	358

9.4	Message and Information Element Abstract Syntax (with ASN.1)	359

9.4.1	General	359

9.4.2	Usage of private message mechanism for non-standard use	359

9.4.3	Elementary Procedure Definitions	360

9.4.4	PDU Definitions	377

9.4.5	Information Element Definitions	477

9.4.6	Common Definitions	634

9.4.7	Constant Definitions	635

9.4.8	Container Definitions	653

9.5	Message Transfer Syntax	657

9.6	Timers	657

10	Handling of unknown, unforeseen and erroneous protocol data	657

Annex A (informative):	Change History	658

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the 5G radio network layer signalling protocol for the F1 interface. The F1 interface provides means for interconnecting a gNB-CU and a gNB-DU of a gNB within an NG-RAN, or for interconnecting a gNB-CU and a gNB-DU of an en-gNB within an E-UTRAN. The F1 Application Protocol (F1AP) supports the functions of F1 interface by signalling procedures defined in the present document. F1AP is developed in accordance to the general principles stated in TS 38.401 [4] and TS 38.470 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.470: "NG-RAN; F1 general aspects and principles".

[3]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[4]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[5]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER)".

[6]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[7]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[8]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[9]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)".

[10]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[11]	3GPP TS 23.203: "Policy and charging control architecture".

[12]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[13]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[14]	3GPP TR 25.921: (version.7.0.0): "Guidelines and principles for protocol description and error".

[15]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[16]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[17]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[18]	3GPP TS 29.281: "General Packet Radio System (GPRS); Tunnelling Protocol User Plane (GTPv1-U) ".

[19]	3GPP TS 38.414: "NG-RAN; NG data transport".

[20]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[21]	3GPP TS 23.501: "System Architecture for the 5G System".

[22]	3GPP TS 38.472: "NG-RAN; F1 signalling transport".

[23]	3GPP TS 23.003: "Numbering, addressing and identification".

[24]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state ".

[25]	3GPP TS 36.104: "Base Station (BS) radio transmission and reception".

[26]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[27]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[28]	3GPP TS 38.423: "NG-RAN; Xn application protocol (XnAP)".

[29]	3GPP TS 32.422: "Trace control and configuration management".

[30]	3GPP TS 38.340: "NR; Backhaul Adaptation Protocol (BAP) specification".

[31]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[32]	3GPP TS 38.314: "	NR; Layer 2 measurements".

[33]	3GPP TS 38.211: "NR; Physical channels and modulation".

[34]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[35]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT)".

[36]	3GPP TS 23.032:"Technical Specification Group Services and System Aspects; Universal Geographical Area Description (GAD)".

[37]	3GPP TS 38.455: "NG-RAN; NR Positioning protocol A (NRPPa)".

[38]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[39]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[40]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[41]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[42]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[43]	3GPP TS 38.215: "NR; Physical layer (PHY); Measurements".

[44]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[45]	Void

[46]	3GPP TS 37.213: "NR; Physical layer procedures for shared spectrum channel access".

[47]	3GPP TS 37.483: "E1 Application Protocol (E1AP)".

# 3	Definitions and abbreviations

## 3.1	Definitions

elementary procedure: F1AP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between gNB-CU and gNB-DU. These Elementary Procedures are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as standalone procedures, which can be active in parallel. The usage of several F1AP EPs together is specified in stage 2 specifications (e.g., TS 38.470 [2]).

An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success and/or failure).

-	Class 2: Elementary Procedures without response.

For Class 1 EPs, the types of responses can be as follows:

Successful:

-	A signalling message explicitly indicates that the elementary procedure successfully completed with the receipt of the response.

Unsuccessful:

-	A signalling message explicitly indicates that the EP failed.

-	On time supervision expiry (i.e., absence of expected response).

Successful and Unsuccessful:

-	One signalling message reports both successful and unsuccessful outcome for the different included requests. The response message used is the one defined for successful outcome.

Class 2 EPs are considered always successful.

BH RLC channel: as defined in TS 38.300 [6].

Conditional handover: as defined in TS 38.300 [6].

Conditional PSCell Addition: as defined in TS 37.340 [7].

Conditional PSCell Change: as defined in TS 37.340 [7].

DAPS Handover: as defined in TS 38.300 [6].

EN-DC operation: Used in this specification when the F1AP is applied for gNB-CU and gNB-DU in E-UTRAN.

gNB: as defined in TS 38.300 [6].

gNB-CU: as defined in TS 38.401 [4].

gNB-CU UE F1AP ID: as defined in TS 38.401 [4].

gNB-DU: as defined in TS 38.401 [4].

gNB-DU UE F1AP ID: as defined in TS 38.401 [4].

en-gNB: as defined in TS 37.340 [7].

IAB-MT: as defined in TS 38.300 [6].

IAB-DU: as defined in TS 38.300 [6].

IAB-node: as defined in TS 38.300 [6].

IAB-donor: as defined in TS 38.300 [6].

IAB-donor-CU: as defined in TS 38.401 [4].

IAB-donor-DU: as defined in TS 38.401 [4].

MBS session resource: as defined in TS 38.401 [4].

MBS-associated signalling: When F1AP messages associated to one MBS session uses the MBS-associated logical F1-connection for association of the message to the MBS session in gNB-DU and gNB-CU.

MBS-associated logical F1-connection: The MBS-associated logical F1-connection uses the identities GNB-CU MBS F1AP ID and GNB-DU MBS F1AP ID according to the definition in TS 38.401 [4]. For a received MBS-associated F1AP message the gNB-CU identifies the associated MBS session based on the GNB-CU MBS F1AP ID IE and the gNB-DU identifies the associated MBS session based on the GNB-DU MBS F1AP ID IE.

MBS Session context in a gNB-DU: as defined in TS 38.401 [4].

Multicast F1-U Context: as defined in TS 38.401 [4].

Other SI: as defined in TS 38.300 [6].

Public network integrated NPN: as defined in TS 23.501 [21].

Stand-alone Non-Public Network: as defined in TS 23.501 [21].

UE-associated signalling: When F1AP messages associated to one UE uses the UE-associated logical F1-connection for association of the message to the UE in gNB-DU and gNB-CU.

UE-associated logical F1-connection: The UE-associated logical F1-connection uses the identities GNB-CU UE F1AP ID and GNB-DU UE F1AP ID according to the definition in TS 38.401 [4]. For a received UE associated F1AP message the gNB-CU identifies the associated UE based on the GNB-CU UE F1AP ID IE and the gNB-DU identifies the associated UE based on the GNB-DU UE F1AP ID IE. The UE-associated logical F1-connection may exist before the F1 UE context is setup in gNB-DU.

U2N Relay UE: a UE that provides functionality to support connectivity to the network for U2N Remote UE(s).

U2N Remote UE: a UE that communicates with the network via a U2N Relay UE.

Uu Relay RLC channel: as defined in TS 38.300 [6].

PC5 Relay RLC channel: as defined in TS 38.300 [6].

SRAP: Sidelink relay adaptation protocol, as defined in TS 38.300 [6].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

ARP	Antenna Reference Point

ARPI	Additional RRM Policy Index

BH	Backhaul

CAG	Closed Access Group

CN	Core Network

CG	Cell Group

CG-SDT	Configured Grant-Small Data Transmission

CGI	Cell Global Identifier

CHO	Conditional Handover

CP	Control Plane

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

DL-PRS	Downlink Positioning Reference Signal

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

FSA ID	MBS Frequency Selection Area (FSA) ID

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

NID	Network Identifier

NPN	Non-Public Network

NSAG	Network Slice AS Group

NSSAI	Network Slice Selection Assistance Information

PDC	Propagation Delay Compensation

PEIPS	Paging Early Indication with Paging Subgrouping

posSIB	Positioning SIB

PNI-NPN	Public Network Integrated NPN

PTP	Point to Point

PTM	Point to Multipoint

QMC	QoE Measurement Collection

QoE	Quality of Experience

RANAC	RAN Area Code

RedCap	Reduced Capability

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RRC	Radio Resource Control

RSRP	Reference Signal Received Power

SDT	Small Data Transmission

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

TAC	Tracking Area Code

TAI	Tracking Area Identity

TEG	Timing Error Group

TRP	Transmission-Reception Point

U2N	UE-to-Network

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

Z-AoA	Zenith Angles of Arrival

# 4	General

## 4.1	Procedure specification principles

The principle for specifying the procedure logic is to specify the functional behaviour of the terminating node exactly and completely. Any rule that specifies the behaviour of the originating node shall be possible to be verified with information that is visible within the system.

The following specification principles have been applied for the procedure text in clause 8:

-	The procedure text discriminates between:

1)	Functionality which "shall" be executed.

The procedure text indicates that the receiving node "shall" perform a certain function Y under a certain condition. If the receiving node supports procedure X but cannot perform functionality Y requested in the REQUEST message of a Class 1 EP, the receiving node shall respond with the message used to report unsuccessful outcome for this procedure, containing an appropriate cause value.

2)	Functionality which "shall, if supported" be executed.

The procedure text indicates that the receiving node "shall, if supported," perform a certain function Y under a certain condition. If the receiving node supports procedure X, but does not support functionality Y, the receiving node shall proceed with the execution of the EP, possibly informing the requesting node about the not supported functionality.

-	Any required inclusion of an optional IE in a response message is explicitly indicated in the procedure text. If the procedure text does not explicitly indicate that an optional IE shall be included in a response message, the optional IE shall not be included. For requirements on including Criticality Diagnostics IE, see clause 10.

## 4.2	Forwards and backwards compatibility

The forwards and backwards compatibility of the protocol is assured by mechanism where all current and future messages, and IEs or groups of related IEs, include ID and criticality fields that are coded in a standard format that will not be changed in the future. These parts can always be decoded regardless of the standard version.

## 4.3	Specification notations

For the purposes of the present document, the following notations apply:

Procedure	When referring to an elementary procedure in the specification the Procedure Name is written with the first letters in each word in upper case characters followed by the word "procedure", e.g. Handover Preparation procedure.

Message	When referring to a message in the specification the MESSAGE NAME is written with all letters in upper case characters followed by the word "message", e.g. HANDOVER REQUEST message.

IE	When referring to an information element (IE) in the specification the Information Element Name is written with the first letters in each word in upper case characters and all letters in Italic font followed by the abbreviation "IE", e.g. E-RAB ID IE.

Value of an IE	When referring to the value of an information element (IE) in the specification the "Value" is written as it is specified in the specification enclosed by quotation marks, e.g. "Value".

# 5	F1AP services

F1AP provides the signalling service between gNB-DU and the gNB-CU that is required to fulfil the F1AP functions described in clause 7. F1AP services are divided into two groups:

Non UE-associated services:	They are related to the whole F1 interface instance between the gNB-DU and gNB-CU utilising a non UE-associated signalling connection.

UE-associated services:	They are related to one UE. F1AP functions that provide these services are associated with a UE-associated signalling connection that is maintained for the UE in question.

MBS-associated services:	They are related to one MBS service. F1AP functions that provide these services are associated with a MBS-associated signalling connection that is maintained for the MBS service in question.

Unless explicitly indicated in the procedure specification, at any instance in time one protocol endpoint shall have a maximum of one ongoing F1AP procedure related to a certain UE.

Unless explicitly indicated in the procedure specification, at any instance in time one protocol endpoint shall have a maximum of one ongoing F1AP procedure related to a certain MBS session.

All considerations of gNB-DU in this specification also apply to the IAB-DU and IAB-donor-DU, unless stated otherwise. All considerations of gNB-CU in this specification apply to the IAB-donor-CU as well, unless stated otherwise.

# 6	Services expected from signalling transport

The signalling connection shall provide in sequence delivery of F1AP messages. F1AP shall be notified if the signalling connection breaks.

# 7	Functions of F1AP

The functions of F1AP are described in TS 38.470 [2].

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| gNB-DU Resource Coordination | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| F1 Removal | F1 REMOVAL REQUEST | F1 REMOVAL RESPONSE | F1 REMOVAL FAILURE |
| BAP Mapping Configuration | BAP MAPPING CONFIGURATION | BAP MAPPING CONFIGURATION ACKNOWLEDGE | BAP MAPPING CONFIGURATION FAILURE |
| GNB-DU Resource Configuration | GNB-DU RESOURCE CONFIGURATION | GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE | GNB-DU RESOURCE CONFIGURATION FAILURE |
| IAB TNL Address Allocation | IAB TNL ADDRESS REQUEST | IAB TNL ADDRESS RESPONSE | IAB TNL ADDRESS FAILURE |
| IAB UP Configuration Update | IAB UP CONFIGURATION UPDATE REQUEST | IAB UP CONFIGURATION UPDATE RESPONSE | IAB UP CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Positioning Measurement | POSITIONING MEASUREMENT REQUEST | POSITIONING MEASUREMENT RESPONSE | POSITIONING MEASUREMENT FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| Broadcast Context Setup | BROADCAST CONTEXT SETUP REQUEST | BROADCAST CONTEXT SETUP RESPONSE | BROADCAST CONTEXT SETUP FAILURE |
| Broadcast Context Release | BROADCAST CONTEXT RELEASE COMMAND | BROADCAST CONTEXT RELEASE COMPLETE |  |
| Broadcast Context Modification | BROADCAST CONTEXT MODIFICATION REQUEST | BROADCAST CONTEXT MODIFICATION RESPONSE | BROADCAST CONTEXT MODIFICATION FAILURE |
| Multicast Context Setup | MULTICAST CONTEXT SETUP REQUEST | MULTICAST CONTEXT SETUP RESPONSE | MULTICAST CONTEXT SETUP FAILURE |
| Multicast Context Release | MULTICAST CONTEXT RELEASE COMMAND | MULTICAST CONTEXT RELEASE COMPLETE |  |
| Multicast Context Modification | MULTICAST CONTEXT MODIFICATION REQUEST | MULTICAST CONTEXT MODIFICATION RESPONSE | MULTICAST CONTEXT MODIFICATION FAILURE |
| Multicast Distribution Setup | MULTICAST DISTRIBUTION SETUP REQUEST | MULTICAST DISTRIBUTION SETUP RESPONSE | MULTICAST DISTRIBUTION SETUP FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE COMMAND | MULTICAST DISTRIBUTION RELEASE COMPLETE |  |
| PDC Measurement Initiation | PDC MEASUREMENT INITIATION REQUEST | PDC MEASUREMENT INITIATION RESPONSE | PDC MEASUREMENT INITIATION FAILURE |
| PRS Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |
| Measurement Preconfiguration | MEASUREMENT PRECONFIGURATION REQUIRED | MEASUREMENT PRECONFIGURATION CONFIRM | MEASUREMENT PRECONFIGURATION REFUSE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Context Release Request | BROADCAST CONTEXT RELEASE REQUEST |
| Multicast Context Release Request | MULTICAST CONTEXT RELEASE REQUEST |
| PDC Measurement Report | PDC MEASUREMENT REPORT |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| PDC Measurement Failure Indication | PDC MEASUREMENT FAILURE INDICATION |
| PDC Measurement Termination | PDC MEASUREMENT TERMINATION COMMAND |
| Measurement Activation | MEASUREMENT ACTIVATION |
| QoE Information Transfer | QOE INFORMATION TRANSFER |
| Positioning System Information Delivery | POSITIONING SYSTEM INFORMATION DELIVERY COMMAND |

## 8.2	Interface Management procedures

### 8.2.1	Reset

#### 8.2.1.1	General

The purpose of the Reset procedure is to initialise or re-initialise the F1AP UE-related contexts, in the event of a failure in the gNB-CU or gNB-DU. This procedure does not affect the application level configuration data exchanged during, e.g., the F1 Setup procedure.

The procedure uses non-UE associated signalling.

#### 8.2.1.2	Successful Operation

##### 8.2.1.2.1	Reset Procedure Initiated from the gNB-CU

Figure 8.2.1.2.1-1: Reset procedure initiated from the gNB-CU. Successful operation

In the event of a failure at the gNB-CU, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the gNB-DU.

At reception of the RESET message the gNB-DU shall release all allocated resources on F1 and radio resources related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the indicated UE contexts including F1AP ID.

After the gNB-DU has released all assigned F1 resources and the UE F1AP IDs for all indicated UE associations which can be used for new UE-associated logical F1-connections over the F1 interface, the gNB-DU shall respond with the RESET ACKNOWLEDGE message. The gNB-DU does not need to wait for the release of radio resources to be completed before returning the RESET ACKNOWLEDGE message.

If the RESET message contains the UE-associated logical F1-connection list IE, then:

-	The gNB-DU shall use the gNB-CU UE F1AP ID IE and/or the gNB-DU UE F1AP ID IE to explicitly identify the UE association(s) to be reset.

-	The gNB-DU shall include in the RESET ACKNOWLEDGE message, for each UE association to be reset, the UE-associated logical F1-connection Item IE in the UE-associated logical F1-connection list IE. The UE-associated logical F1-connection Item IEs shall be in the same order as received in the RESET message and shall include also unknown UE-associated logical F1-connections. Empty UE-associated logical F1-connection Item IEs, received in the RESET message, may be omitted in the RESET ACKNOWLEDGE message.

-	If the gNB-CU UE F1AP ID IE is included in the UE-associated logical F1-connection Item IE for a UE association, the gNB-DU shall include the gNB-CU UE F1AP ID IE in the corresponding UE-associated logical F1-connection Item IE in the RESET ACKNOWLEDGE message.

-	If the gNB-DU UE F1AP ID IE is included in the UE-associated logical F1-connection Item IE for a UE association, the gNB-DU shall include the gNB-DU UE F1AP ID IE in the corresponding UE-associated logical F1-connection Item IE in the RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the RESET message is received, any other ongoing procedure (except for another Reset procedure) on the same F1 interface related to a UE association, indicated explicitly or implicitly in the RESET message, shall be aborted.

##### 8.2.1.2.2	Reset Procedure Initiated from the gNB-DU

Figure 8.2.1.2.2-1: Reset procedure initiated from the gNB-DU. Successful operation

In the event of a failure at the gNB-DU, which has resulted in the loss of some or all transaction reference information, a RESET message shall be sent to the gNB-CU.

At reception of the RESET message the gNB-CU shall release all allocated resources on F1 related to the UE association(s) indicated explicitly or implicitly in the RESET message and remove the F1AP ID for the indicated UE associations.

After the gNB-CU has released all assigned F1 resources and the UE F1AP IDs for all indicated UE associations which can be used for new UE-associated logical F1-connections over the F1 interface, the gNB-CU shall respond with the RESET ACKNOWLEDGE message.

If the RESET message contains the UE-associated logical F1-connection list IE, then:

-	The gNB-CU shall use the gNB-CU UE F1AP ID IE and/or the gNB-DU UE F1AP ID IE to explicitly identify the UE association(s) to be reset.

-	The gNB-CU shall in the RESET ACKNOWLEDGE message include, for each UE association to be reset, the UE-associated logical F1-connection Item IE in the UE-associated logical F1-connection list IE. The UE-associated logical F1-connection Item IEs shall be in the same order as received in the RESET message and shall include also unknown UE-associated logical F1-connections. Empty UE-associated logical F1-connection Item IEs, received in the RESET message, may be omitted in the RESET ACKNOWLEDGE message.

-	If the gNB-CU UE F1AP ID IE is included in the UE-associated logical F1-connection Item IE for a UE association, the gNB-CU shall include the gNB-CU UE F1AP ID IE in the corresponding UE-associated logical F1-connection Item IE in the RESET ACKNOWLEDGE message.

-	If the gNB-DU UE F1AP ID IE is included in a UE-associated logical F1-connection Item IE for a UE association, the gNB-CU shall include the gNB-DU UE F1AP ID IE in the corresponding UE-associated logical F1-connection Item IE in the RESET ACKNOWLEDGE message.

Interactions with other procedures:

If the RESET message is received, any other ongoing procedure (except for another Reset procedure) on the same F1 interface related to a UE association, indicated explicitly or implicitly in the RESET message, shall be aborted.

#### 8.2.1.3	Abnormal Conditions

Not applicable.

### 8.2.2	Error Indication

#### 8.2.2.1	General

The Error Indication procedure is initiated by a node in order to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

If the error situation arises due to reception of a message utilising UE associated signalling, then the Error Indication procedure uses UE associated signalling. Otherwise the procedure uses non-UE associated signalling.

#### 8.2.2.2	Successful Operation

Figure 8.2.2.2-1: Error Indication procedure, gNB-CU originated. Successful operation

Figure 8.2.2.2-2: Error Indication procedure, gNB-DU originated. Successful operation

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the receiving node.

The ERROR INDICATION message shall contain at least either the Cause IE or the Criticality Diagnostics IE. In case the Error Indication procedure is triggered by utilising UE associated signalling the gNB-CU UE F1AP ID IE and gNB-DU UE F1AP ID IE shall be included in the ERROR INDICATION message. If one or both of the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE are not correct, the cause shall be set to appropriate value, e.g., "Unknown or already allocated gNB-CU UE F1AP ID", "Unknown or already allocated gNB-DU UE F1AP ID" or "Unknown or inconsistent pair of UE F1AP ID".

#### 8.2.2.3	Abnormal Conditions

Not applicable.

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

The gNB-DU initiates the procedure by sending a F1 SETUP REQUEST message including the appropriate data to the gNB-CU. The gNB-CU responds with a F1 SETUP RESPONSE message including the appropriate data.

The exchanged data shall be stored in respective node and used as long as there is an operational TNL association. When this procedure is finished, the F1 interface is operational and other F1 messages may be exchanged.

If the F1 SETUP REQUEST message contains the gNB-DU Name IE, the gNB-CU may use this IE as a human readable name of the gNB-DU. If the F1 SETUP REQUEST message contains the Extended gNB-DU Name IE, the gNB-CU may use this IE as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if included.

If the F1 SETUP RESPONSE message contains the gNB-CU Name IE, the gNB-DU may use this IE as a human readable name of the gNB-CU. If the F1 SETUP RESPONSE message contains the Extended gNB-CU Name IE, the gNB-DU may use this IE as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if included.

If the F1 SETUP REQUEST message contains the gNB-DU Served Cells List IE, the gNB-CU shall take into account as specified in TS 38.401 [4].

For NG-RAN, the gNB-DU shall include the gNB-DU System Information IE and the TAI Slice Support List IE in the F1 SETUP REQUEST message.

The gNB-CU may include the Cells to be Activated List IE in the F1 SETUP RESPONSE message. The Cells to be Activated List IE includes a list of cells that the gNB-CU requests the gNB-DU to activate. The gNB-DU shall activate the cells included in the Cells to be Activated List IE and reconfigure the physical cell identity for cells for which the NR PCI IE is included.

If Cells to be Activated List Item IE is included in the F1 SETUP RESPONSE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the F1 SETUP RESPONSE message.

For NG-RAN, the gNB-DU may include the RAN Area Code IE in the F1 SETUP REQUEST message. The gNB-CU may use it according to TS 38.300 [6].

For NG-RAN, the gNB-DU may include Supported MBS FSA ID List IE in the Served Cell Information IE in the F1 SETUP REQUEST message. The gNB-CU may use it according to TS 38.300 [6].

For NG-RAN, the gNB-CU may include Available PLMN List IE, and optionally also Extended Available PLMN List IE in the F1 SETUP RESPONSE message, if the available PLMN(s) are different from what gNB-DU has provided in F1 SETUP REQUEST message, gNB-DU shall take this into account and only broadcast the PLMN(s) included in the received Available PLMN list(s).

For NG-RAN, the gNB-CU may include Available SNPN ID List IE in the F1 SETUP RESPONSE message. If the available SNPN(s) are different from what gNB-DU has provided in F1 SETUP REQUEST message, gNB-DU shall take this into account and only broadcast the SNPN(s) included in the received Available SNPN ID list.

The Latest RRC Version Enhanced IE shall be included in the F1 SETUP REQUEST message and in the F1 SETUP RESPONSE message.

If in F1 SETUP REQUEST message, the Cell Direction IE is present, the gNB-CU should use it to understand whether the cell is for UL or DL only. If in F1 SETUP REQUEST message, the Cell Direction IE is omitted in the Served Cell Information IE it shall be interpreted as that the Cell Direction is Bi-directional.

If the Intended TDD DL-UL Configuration IE is present in the F1 SETUP REQUEST message, the receiving gNB-CU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-CU may merge the Intended TDD DL-UL Configuration information received from two or more gNB-DUs. The gNB-CU shall consider the received Intended TDD DL-UL Configuration content valid until reception of an update of the IE for the same cell(s).

If the Aggressor gNB Set ID IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, take it into account.

If the Victim gNB Set ID IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, take it into account.

If the F1 SETUP REQUEST message contains the Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the SFN Offset IE is contained in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information to deduce the SFN0 offset of the reported cell.

If the F1 SETUP RESPONSE message contains the Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the F1 SETUP RESPONSE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the BAP Address IE is included in the F1 SETUP REQUEST, the receiving gNB-CU shall, if supported, consider the information therein for discovering the collocation of an IAB-DU and an IAB-MT.

If the F1 SETUP REQUEST message is received from an IAB-donor-DU, the gNB-CU shall, if supported, include the BAP Address IE in the F1 SETUP RESPONSE message.

NOTE:	How to identify the IAB-donor-DU is up to gNB-CU implementation.

If the F1 SETUP RESPONSE message contains the BAP Address IE, the gNB-DU shall, if supported, store the received BAP address and use it as specified in TS 38.340 [30].

If the NR PRACH Configuration List IE is included in the Served Cell Information IE contained in the F1 SETUP REQUEST message, the gNB-CU may store the information, and forward it to other RAN nodes for RACH optimisation. If the L139 Info IE included in the NR PRACH Configuration List IE is present, it shall contain the Root Sequence Index IE.

If the RedCap Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

#### 8.2.3.3	Unsuccessful Operation

Figure 8.2.3.3-1: F1 Setup procedure: Unsuccessful Operation

If the gNB-CU cannot accept the setup, it should respond with a F1 SETUP FAILURE and appropriate cause value.

If the F1 SETUP FAILURE message includes the Time To Wait IE, the gNB-DU shall wait at least for the indicated time before reinitiating the F1 setup towards the same gNB-CU.

#### 8.2.3.4	Abnormal Conditions

Not applicable.

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [7]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

The gNB-DU initiates the procedure by sending a GNB-DU CONFIGURATION UPDATE message to the gNB-CU including an appropriate set of updated configuration data that it has just taken into operational use. The gNB-CU responds with GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in both nodes and used as long as there is an operational TNL association or until any further update is performed.

If gNB-DU ID IE is contained in the GNB-DU CONFIGURATION UPDATE message for a newly established SCTP association, the gNB-CU will associate this association with the related gNB-DU.

If Served Cells To Add Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall add cell information according to the information in the Served Cell Information IE. For NG-RAN, the gNB-DU shall include the gNB-DU System Information IE.

If Served Cells To Modify Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall modify information of cell indicated by Old NR CGI IE according to the information in the Served Cell Information IE and overwrite the served cell information for the affected served cell. Further, if the gNB-DU System Information IE is present the gNB-CU shall store and replace any previous information received.

If Served Cells To Delete Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall delete information of cell indicated by Old NR CGI IE.

If Cells Status Item IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall update the information about the cells, as described in TS 38.401 [4]. If if the Switching Off Ongoing IE is present in the Cells Status Item IE, contained in the GNB-DU CONFIGURATION UPDATE message, and the corresponding Service State IE is set to "Out-of-Service", the gNB-CU shall ignore the Switching Off Ongoing IE.

If Cells to be Activated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for cells for which the NR PCI IE is included.

If Cells to be Activated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If Cells to be Deactivated List Item IE is contained in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall deactivate all the cells with NR CGI listed in the IE.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

For NG-RAN, the gNB-DU may include the RAN Area Code IE in the GNB-DU CONFIGURATION UPDATE message. The gNB-CU shall store and replace any previously provided RAN Area Code IE by the received RAN Area Code IE.

For NG-RAN, the gNB-DU may include the Supported MBS FSA ID List IE in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message. The gNB-CU shall store and replace any previously provided MBS FSA ID list IE by the received MBS FSA ID list IE.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

If Available SNPN ID List IE is contained in GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU shall overwrite the whole available SNPN ID list and update the corresponding system information.

If in GNB-DU CONFIGURATION UPDATE message, the Cell Direction IE is present, the gNB-CU should use it to understand whether the cell is for UL or DL only. If in GNB-DU CONFIGURATION UPDATE message, the Cell Direction IE is omitted in the Served Cell Information IE it shall be interpreted as that the Cell Direction is Bi-directional.

If the GNB-DU CONFIGURATION UPDATE message includes gNB-DU TNL Association To Remove List IE, the gNB-CU shall, if supported, initiate removal of the TNL association(s) indicated by gNB-DU TNL endpoint(s) and gNB-CU TNL endpoint(s) if the TNL Association Transport Layer Address gNB-CU IE is present, or the TNL association(s) indicated by gNB-DU TNL endpoint(s) if the TNL Association Transport Layer Address gNB-CU IE is absent:

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the gNB-DU TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-DU TNL endpoints correspond to all gNB-DU TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

- 	if the received TNL Association Transport Layer Address gNB-CU IE includes the Port Number IE, the gNB-CU TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-CU TNL endpoints correspond to all gNB-CU TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If the Intended TDD DL-UL Configuration IE is present in the GNB-DU CONFIGURATION UPDATE message, the receiving gNB-CU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-CU may merge the Intended TDD DL-UL Configuration information received from two or more gNB-DUs. The gNB-CU shall consider the received Intended TDD DL-UL Configuration IE content valid until reception of an update of the IE for the same cell(s).

If the Aggressor gNB Set ID IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account.

If the Victim gNB Set ID IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account.

If the GNB-DU CONFIGURATION UPDATE message includes Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message includes Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the SFN Offset IE is contained in the Served Cell Information IE in GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information to deduce the SFN0 offset of the reported cell.

If the NR PRACH Configuration List IE is included in the Served Cell Information IE contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store the information, and forward it to other RAN nodes for RACH optimisation. If the L139 Info IE included in the NR PRACH Configuration List IE is present, it shall contain the Root Sequence Index IE.

If the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message contains the BAP Address IE, the gNB-DU shall, if supported, store the received BAP address and use it as specified in TS 38.340 [30].

If the Coverage Modification Notification IE is contained in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, take it into account for Coverage and Capacity Optimization.

If the Cells for SON IE is present in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU may store or update this information and behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the RedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

If the gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU. If the Extended gNB-DU Name IE is included in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-DU and shall ignore the gNB-DU Name IE if also included.

#### 8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: gNB-DU Configuration Update procedure: Unsuccessful Operation

If the gNB-CU cannot accept the update, it shall respond with a GNB-DU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-DU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-DU shall wait at least for the indicated time before reinitiating the GNB-DU CONFIGURATION UPDATE message towards the same gNB-CU.

#### 8.2.4.4	Abnormal Conditions

Not applicable.

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

If the gNB-CU TNL Association To Add List IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, use it to establish the TNL association(s) with the gNB-CU. If the gNB-CU TNL Association To Add List is included in the GNB-CU CONFIGURATION UPDATE message, and if the TNL Association Transport Layer Information IE does not include the Port Number IE, the gNB-DU shall assume that port number value 38472 is used for the endpoint. The gNB-DU shall report to the gNB-CU, in the gNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the successful establishment of the TNL association(s) with the gNB-CU as follows:

-	A list of TNL address(es) with which the gNB-DU successfully established the TNL association shall be included in the gNB-CU TNL Association Setup List IE;

-	A list of TNL address(es) with which the gNB-DU failed to establish the TNL association shall be included in the gNB-CU TNL Association Failed To Setup List IE.

If the GNB-CU CONFIGURATION UPDATE message includes gNB-CU TNL Association To Remove List IE, the gNB-DU shall, if supported, initiate removal of the TNL association(s) indicated by gNB-CU TNL endpoint(s) and gNB-DU TNL endpoint(s) if the TNL Association Transport Layer Address gNB-DU IE is present, or the TNL association(s) indicated by gNB-CU TNL endpoint(s) if the TNL Association Transport Layer Address gNB-DU IE is absent:

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the gNB-CU TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-CU TNL endpoints correspond to all gNB-CU TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

- 	if the received TNL Association Transport Layer Address gNB-DU IE includes the Port Number IE, the gNB-DU TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-DU TNL endpoints correspond to all gNB-DU node TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If the gNB-CU TNL Association To Update List IE is contained in the gNB-CU CONFIGURATION UPDATE message the gNB-DU shall, if supported, overwrite the previously stored information for the related TNL Association(s).

- 	if the received TNL Association Transport Layer Address IE includes the Port Number IE, the gNB-CU TNL endpoint is identified by the Endpoint IP Address IE and the Port Number IE. Otherwise, the gNB-CU TNL endpoints correspond to all gNB-CU TNL endpoints identified by the Endpoint IP Address IE and any port number(s).

If in the gNB-CU CONFIGURATION UPDATE message the TNL Association usage IE is included in the gNB-CU TNL Association To Add List IE or the gNB-CU TNL Association To Update List IE, the gNB-DU node shall, if supported, use it as described in TS 38.472 [22].

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

If Protected E-UTRA Resources List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall protect the corresponding resource of the cells indicated by E-UTRA Cells List IE for spectrum sharing between E-UTRA and NR.

If the GNB-CU CONFIGURATION UPDATE message contains the Protected E-UTRA Resource Indication IE, the receiving gNB-DU should forward it to lower layers and use it for cell-level resource coordination. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE when expressing its desired resource allocation during gNB-DU Resource Coordination procedure. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE content valid until reception of a new update of the IE for the same gNB-DU.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

If Available SNPN ID List IE is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole available SNPN ID list and update the corresponding system information.

If Cells Failed to be Activated Item IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU shall consider that the indicated cells are out-of-service as defined in TS 38.401 [4].

If the Neighbour Cell Information List IE is present in the GNB-CU CONFIGURATION UPDATE message, the receiving gNB-DU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-DU shall consider the received Neighbour Cell Information List IE content valid until reception of an update of the IE for the same cell(s). If the Intended TDD DL-UL Configuration NR IE is absent from the Neighbour Cell Information List IE, whereas the corresponding NR CGI IE is present, the receiving gNB-DU shall remove the previously stored Neighbour Cell Information IE corresponding to the NR CGI.

If the GNB-CU CONFIGURATION UPDATE message includes Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message includes Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-CU CONFIGURATION UPDATE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows IAB-node access or not.

If the BAP Address IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, store the received BAP address and use it as specified in TS 38.340 [30].

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a new cell and/or beam configuration.

If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust coverage of its cells.

If the Cells for SON IE is present in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store or update this information and it behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU. If the Extended gNB-CU Name IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU may store it or update this IE value if already stored, and use it as a human readable name of the gNB-CU and shall ignore the gNB-CU Name IE if also included.

#### 8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: gNB-CU Configuration Update: Unsuccessful Operation

If the gNB-DU cannot accept the update, it shall respond with a GNB-CU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU shall wait at least for the indicated time before reinitiating the GNB-CU CONFIGURATION UPDATE message towards the same gNB-DU.

#### 8.2.5.4	Abnormal Conditions

Not applicable.

### 8.2.6	gNB-DU Resource Coordination

#### 8.2.6.1	General

The purpose of the gNB-DU Resource Coordination procedure is to enable coordination of radio resource allocation between a gNB-CU and a gNB-DU for the purpose of spectrum sharing between E-UTRA and NR. This procedure is to be used only for the purpose of spectrum sharing between E-UTRA and NR.

The procedure uses non-UE-associated signalling.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: gNB-DU Resource Coordination, successful operation

A gNB-CU initiates the procedure by sending the GNB-DU RESOURCE COORDINATION REQUEST message to a gNB-DU over the F1 interface.

The gNB-DU extracts the E-UTRA – NR Cell Resource Coordination Request Container IE and it replies by sending the GNB-DU RESOURCE COORDINATION RESPONSE message.

In case of NR-initiated gNB-DU Resource Coordination procedure, the Ignore Coordination Request Container IE shall be present and set to "yes" and the E-UTRA – NR Cell Resource Coordination Request Container IE in the GNB-DU RESOURCE COORDINATION REQUEST message shall be ignored.

### 8.2.7	gNB-DU Status Indication

#### 8.2.7.1	General

The purpose of the gNB-DU Status Indication procedure is informing the gNB-CU that the gNB-DU is overloaded so that overload reduction actions can be applied. This procedure is also used to inform the IAB-donor-CU about a downlink congestion at an IAB-DU or an IAB-donor-DU. The procedure uses non-UE associated signalling.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: gNB-DU Status Indication procedure

If the gNB-DU Overload Information IE in the GNB-DU STATUS INDICATION message indicates that the gNB-DU is overloaded, the gNB-CU shall apply overload reduction actions until informed, with a new GNB-DU STATUS INDICATION message, that the overload situation has ceased.

The detailed overload reduction policy is up to gNB-CU implementation.

If the gNB-DU is an IAB-DU or an IAB-donor-DU, and if the IAB Congestion Indication IE is present in the GNB-DU STATUS INDICATION message and only includes the Child Node Identifier IE, the gNB-CU shall, if supported, consider that the backhaul link between the gNB-DU and the node identified by the Child Node Identifier IE is congested. If the IAB Congestion Indication IE is present in the GNB-DU STATUS INDICATION message and includes both the Child Node Identifier IE and the BH RLC CH ID IE, the gNB-CU shall, if supported, consider that congestion occurs on the corresponding BH RLC channel(s) over the link towards the node identified by the Child Node Identifier IE.

#### 8.2.7.3	Abnormal Conditions

Void.

### 8.2.8	F1 Removal

#### 8.2.8.1	General

The purpose of the F1 Removal procedure is to remove the interface instance and all related resources between the gNB-DU and the gNB-CU in a controlled manner. If successful, this procedure erases any existing application level configuration data in the two nodes.

NOTE:	In case the signalling transport is shared among several F1-C interface instances, and the TNL association is still used by one or several F1-C interface instances, the initiating node should not initiate the removal of the TNL association.

The procedure uses non-UE-associated signaling.

#### 8.2.8.2	Successful Operation

Figure 8.2.8-1: F1 Removal, gNB-DU initiated, successful operation

Figure 8.2.8.2-2: F1 Removal, gNB-CU initiated, successful operation

Successful F1 Removal, gNB-DU initiated

The gNB-DU initiates the procedure by sending the F1 REMOVAL REQUEST message to the gNB-CU. Upon reception of the F1 REMOVAL REQUEST message the gNB-CU shall reply with the F1 REMOVAL RESPONSE message. After receiving the F1 REMOVAL RESPONSE message, the gNB-DU may initiate removal of the TNL association towards the gNB-CU, if applicable, and may remove all resources associated with that interface instance. The gNB-CU may then remove all resources associated with that interface instance.

Successful F1 Removal, gNB-CU initiated

The gNB-CU initiates the procedure by sending the F1 REMOVAL REQUEST message to the gNB-DU. Upon reception of the F1 REMOVAL REQUEST message the gNB-DU shall reply with the F1 REMOVAL RESPONSE message. After receiving the F1 REMOVAL RESPONSE message, the gNB-CU may initiate removal of the TNL association towards the gNB-DU, if applicable, and may remove all resources associated with that interface instance. The gNB-DU may then remove all resources associated with that interface instance.

#### 8.2.8.3	Unsuccessful Operation

Figure 8.2.8.3-1: F1 Removal, gNB-DU initiated, unsuccessful operation

Figure 8.2.8.3-2: F1 Removal, gNB-CU initiated, unsuccessful operation

Unsuccessful F1 Removal, gNB-DU initiated

If the gNB-CU cannot accept to remove the interface instance with the gNB-DU it shall respond with an F1 REMOVAL FAILURE message with an appropriate cause value.

Unsuccessful F1 Removal, gNB-CU initiated

If the gNB-DU cannot accept to remove the interface instance with the gNB-CU it shall respond with an F1 REMOVAL FAILURE message with an appropriate cause value.

#### 8.2.8.4	Abnormal Conditions

Not applicable.

### 8.2.9	Network Access Rate Reduction

#### 8.2.9.1	General

The purpose of the Network Access Rate Reduction procedure is to indicate to the gNB-DU that the rate at which UEs are accessing the network need to be reduced from its current level.

The procedure uses non-UE associated signalling.

#### 8.2.9.2	Successful operation

gNB

-

DU

NETWORK ACCESS RATE REDUCTION

gNB

-

CU

Figure 8.2.9.2-1: Network Access Rate Reduction, Successful operation

The gNB-CU initiates the procedure by sending a NETWORK ACCESS RATE REDUCTION message to the gNB-DU. When receiving the NETWORK ACCESS RATE REDUCTION message the gNB-DU should take into account the information contained in the UAC assistance information to set the parameters for Unified Access Barring.

If the NID IE is contained in the NETWORK ACCESS RATE REDUCTION message, the gNB-DU should take it into account and combine the NID IE with the PLMN Identity IE to identify the SNPN.

#### 8.2.9.3	Abnormal Conditions

Not applicable

### 8.2.10	Resource Status Reporting Initiation

#### 8.2.10.1	General

This procedure is used by an gNB-CU to request the reporting of load measurements to gNB-DU.

The procedure uses non UE-associated signalling.

#### 8.2.10.2	Successful Operation

Figure 8.2.10.2-1: Resource Status Reporting Initiation, successful operation

gNB-CU initiates the procedure by sending the RESOURCE STATUS REQUEST message to gNB-DU to start a measurement, stop a measurement, or add cells to report for a measurement. Upon receipt, gNB-DU:

-	shall initiate the requested measurement according to the parameters given in the request in case the Registration Request IE set to "start"; or

-	shall stop all cells measurements and terminate the reporting in case the Registration Request IE is set to "stop"; or

-	shall add cells indicated in the Cell To Report List IE to the measurements initiated before for the given measurement IDs, in case the Registration Request IE is set to "add". If measurements are already initiated for a cell indicated in the Cell To Report List IE, this information shall be ignored.

If the Registration Request IE is set to "start" in the RESOURCE STATUS REQUEST message and the Report Characteristics IE indicates cell specific measurements, the Cell To Report List IE shall be included.

If Registration Request IE is set to "add" in the RESOURCE STATUS REQUEST message, the Cell To Report List IE shall be included.

If gNB-DU is capable to provide all requested resource status information, it shall initiate the measurement as requested by gNB-CU, and respond with the RESOURCE STATUS RESPONSE message.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects gNB-DU shall perform measurements on. For each cell, gNB-DU shall include in the RESOURCE STATUS UPDATE message:

-	the Radio Resource Status IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If the cell for which Radio Resource Status IE is requested to be reported supports more than one SSB, the Radio Resource Status IE for such cell shall include the SSB Area Radio Resource Status Item IE for all SSB areas supported by the cell. If the SSB To Report List IE is included for a cell, the Radio Resource IE for such cell shall only include the SSB Area Radio Resource Status List IE; If the cell for which Radio Resource Status IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Radio Resource Status IE for such cell shall, if supported, include the requested Slice Radio Resource Status Item IE if the cell for which Radio Resource Status IE is requested to be reported supports MIMO the Radio Resource Status IE for such cell shall include the MIMO PRB usasge Information IE;

-	the TNL Capacity Indicator IE, if the second bit, "TNL Capacity Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the third bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one SSB the Composite Available Capacity Group IE for such cell shall include the SSB Area Capacity Value List IE for all SSB areas supported by the cell, providing the SSB area capacity with respect to the Cell Capacity Class Value IE. If the SSB To Report List IE is included for a cell, the Composite Available Capacity Group IE for such cell shall include the requested SSB Area Capacity Value List IE providing the SSB area capacity with respect to the Cell Capacity Class Value. If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Slice Available Capacity IE for such cell shall include the requested Slice Available Capacity Value Downlink IE and Slice Available Capacity Value Uplink IE, providing the slice capacity with respect to the Cell Capacity Class Value.

-	the Hardware Load Indicator IE, if the fourth bit, " HW LoadInd Periodic " of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Number of Active UEs IE, if the fifth bit, "Number of Active UEs Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the NR-U Channel List IE, if the sixth bit, " NR-U Channel List Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1".

If the Reporting Periodicity IE in the RESOURCE STATUS REQUEST is present, this indicates the periodicity for the reporting of periodic measurements. The gNB-DU shall report once, unless otherwise requested within the Reporting Periodicity IE.

#### 8.2.10.3	Unsuccessful Operation

Figure 8.2.10.3-1: Resource Status Reporting Initiation, unsuccessful operation

If any of the requested measurements cannot be initiated, gNB-DU shall send the RESOURCE STATUS FAILURE message with an appropriate cause value.

#### 8.2.10.4	Abnormal Conditions

If the initiating gNB-CU does not receive either RESOURCE STATUS RESPONSE message or RESOURCE STATUS FAILURE message, the gNB-CU may reinitiate the Resource Status Reporting Initiation procedure towards the same gNB-DU, provided that the content of the new RESOURCE STATUS REQUEST message is identical to the content of the previously unacknowledged RESOURCE STATUS REQUEST message with the same Transaction ID.

If the Report Characteristics IE bitmap is set to "0" (all bits are set to "0") in the RESOURCE STATUS REQUEST message then gNB-DU shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

If the gNB-DU receives a RESOURCE STATUS REQUEST message which includes the Registration Request IE set to "start" and the gNB-CU Measurement ID IE corresponding to an existing on-going load measurement reporting, for which a different Transaction ID is used, then gNB-DU shall initiate a RESOURCE STATUS FAILURE message with an appropriate cause value.

### 8.2.11	Resource Status Reporting

#### 8.2.11.1	General

This procedure is initiated by gNB-DU to report the result of measurements admitted by gNB-DU following a successful Resource Status Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.2.11.2	Successful Operation

Figure 8.2.11.2-1: Resource Status Reporting, successful operation

The gNB-DU shall report the results of the admitted measurements in RESOURCE STATUS UPDATE message. The admitted measurements are the measurements that were successfully initiated during the preceding Resource Status Reporting Initiation procedure.

If some results of the admitted measurements in RESOURCE STATUS UPDATE message are missing, the gNB-CU shall consider that these results were not available at the gNB-DU.

#### 8.2.11.3	Unsuccessful Operation

Not applicable.

#### 8.2.11.4	Abnormal Conditions

Void.

## 8.3	UE Context Management procedures

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB,DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly.

If the servingCellMO List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, select servingCellMO after determining the list of BWPs for the UE and include the list of servingCellMOs that have been encoded in CellGroupConfig IE as ServingCellMO-encoded-in-CGC List IE in the UE CONTEXT SETUP RESPONSE message.

If the Configured BWP List IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, take it into account when requesting the gNB-DU for generating preconfigured measurement GAP for the indicated BWPs.

If the SpCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly.

If the SCell To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider it as a list of candidate SCells to be set up. If the SCell UL Configured IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the DRX Cycle IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall use the provided value from the gNB-CU.

If the UL Configuration IE in DRB to Be Setup Item IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the SRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB. If the Additional Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup the indicated RLC entities for the indicated SRB. If the SDT RLC Bearer Configuration IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT SRB indicated by the SRB ID IE. If the SRB Mapping Info IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the SRB Mapping Info IE for the SRB identified by the SRB ID IE and the Uu Relay RLC channel identified by the SRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of SRB data to Uu Relay RLC channel.

If the DRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the QoS Flow Mapping Indication IE is included in the DRB To Be Setup List IE for a QoS flow, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the indicated DRB. If the SDT RLC Bearer Configuration IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, use it for packet transmission belonging to the SDT DRB indicated by the DRB ID IE. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info IE. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT SETUP REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

If the BH RLC Channel To Be Setup List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the Traffic Mapping Information IE is included in the BH RLC Channel To Be Setup Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, process the Traffic Mapping Information IE as follows:

-	if the IP to layer2 Traffic Mapping Info IE is included, the gNB-DU shall store the mapping information contained in the IP to layer2 Traffic Mapping Info To Add IE, if present, for the egress BH RLC channel identified by the BH RLC CH ID IE, and shall remove the previously stored mapping information as indicated by the IP to layer2 Mapping Traffic Info To Remove IE, if present. The gNB-DU shall use the mapping information stored for the mapping of IP traffic to layer 2, as specified in TS 38.340 [30].

-	if the BAP layer BH RLC channel Mapping Info IE is included, the gNB-DU shall store the mapping information contained in the BAP layer BH RLC channel Mapping Info To Add IE, if present, for the egress or ingress BH RLC channel identified by the BH RLC CH ID IE, and shall remove the previously stored mapping information as indicated by the BAP layer BH RLC channel Mapping Info To Remove IE, if present. The gNB-DU shall use the mapping information stored when forwarding traffic on BAP sublayer, as specified in TS 38.340 [30].

If two UL UP TNL Information IEs are included in UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT SETUP RESPONSE message and setup two RLC entities for the indicated DRB. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

If one or two Additional PDCP Duplication UP TNL Information IEs are included in the UE CONTEXT SETUP REQUEST message for a DRB, the gNB-DU shall, if supported, include one or two Additional PDCP Duplication UP TNL Information IEs in the UE CONTEXT SETUP RESPONSE message and setup one or two additional RLC entities for the indicated DRB. The gNB-CU and the gNB-DU use the Additional PDCP Duplication UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2].

If Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activating/deactivating CA based PDCP duplication for the DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when activating/deactivating CA based PDCP duplication for the DRB with more than two RLC entities.

If DC Based Duplication Configured IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall regard that DC based PDCP duplication is configured for this DRB if the value is set to be "true" and it should take the responsibility of PDCP duplication activation/deactivation. If DC Based Duplication Activation IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU should take it into account when activating/deactivating DC based PDCP duplication for this DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT SETUP REQUEST message for a DRB, the gNB-DU shall, if supported, take it into account when activating/deactivating DC based PDCP duplication for the DRB with more than two RLC entities. If the Primary Path Indication IE is included in the RLC Duplication Information IE, the gNB-DU shall, if supported, take it into account when performing DC based PDCP duplication for the DRB with more than two RLC entities.

If UL PDCP SN length IE is included in the UE CONTEXT SETUP REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

For EN-DC operation, and if the Subscriber Profile ID for RAT/Frequency priority IE is received from an MeNB, the UE CONTEXT SETUP REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE. If the Additional RRM Policy Index IE is received from an MeNB, the UE CONTEXT SETUP REQUEST message shall, if supported, contain the Additional RRM Policy Index IE. The gNB-DU shall store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [20]. The gNB-DU shall, if supported, store the received Additional RRM Policy Index in the UE context and use it as defined in TS 36.300 [20].

If the Index to RAT/Frequency Selection Priority IE is available at the gNB-CU, the Index to RAT/Frequency Selection Priority IE shall be included in the UE CONTEXT SETUP REQUEST. The gNB-DU may use it for RRM purposes.

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT SETUP RESPONSE message, the result for all the requested DRBs, SRBs, BH RLC channels, Uu RLC channels, PC5 Relay RLC channels, and SL DRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to Setup List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of BH RLC channels which are successfully established shall be included in the BH RLC Channel Setup List IE;

-	A list of BH RLC channels which failed to be established shall be included in the BH RLC Channel Failed to be Setup List IE;

-	A list of SL DRBs which are successfully established shall be included in the SL DRB Setup List IE;

-	A list of SL DRBs which failed to be established shall be included in the SL DRB Failed to Setup List IE.

-	A list of Uu Relay RLC channels which are successfully established shall be included in the Uu RLC Channel Setup List IE;

-	A list of Uu Relay RLC channels which failed to be established shall be included in the Uu RLC Channel Failed to be Setup List IE;

-	A list of PC5 Relay RLC channels which are successfully established shall be included in the PC5 RLC Channel Setup List IE;

-	A list of PC5 Relay RLC channels which failed to be established shall be included in the PC5 RLC Channel Failed to be Setup List IE;

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB or SL DRB or a BH RLC channel or a Uu RLC channel or a PC5 Relay RLC channel, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

For EN-DC operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [15].

For NG-RAN operation, the gNB-CU shall include in the UE CONTEXT SETUP REQUEST the DRB Information IE.

For DC operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE at the gNB acting as secondary node. If the CG-ConfigInfo IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

For sidelink operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE if the gNB-CU receives sidelink related UE information from UE. If the CG-ConfigInfo IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information or NR sidelink information as defined in TS 38.331 [8].

If the HandoverPreparationInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU of the gNB acting as master node shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. The gNB-CU shall only initiate the UE Context Setup procedure for handover or secondary node addition when at least one DRB is setup for the UE, or at least one BH RLC channel is set up for IAB-MT. If the HandoverPreparationInformation IE containing the sidelink related UE information is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information or NR sidelink information as defined in TS 38.331 [8].

If the received CU to DU RRC Information IE does not include source cell group configuration, the gNB-DU shall generate the cell group configuration using full configuration. Otherwise, delta configuration is allowed.

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message.

If the MeasConfig IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall deduce that changes to the measurements configuration need to be applied. If the measObjectToAddModList IE is included in the MeasConfig IE, then the frequencies added in such IE are to be activated. Then the gNB-DU shall decide if measurement gaps are needed or not and, if needed, the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message. If the measObjectToRemoveList IE is included in the MeasConfig IE, the gNB-DU shall ignore it.

If the NeedForGapsInfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForGapNCSG-InfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForGapNCSG-InfoEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8].

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT SETUP REQUEST message shall be ignored. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Transfer Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], it shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT SETUP REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

The UEAssistanceInformationEUTRA IE shall be included in CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformationEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring LTE sidelink resources for the UE.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

If the Masked IMEISV IE is contained in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) failed to be set up with an appropriate cause value for each SCell failed to setup.

If the Inactivity Monitoring Request IE is contained in the UE CONTEXT SETUP REQUEST message, gNB-DU may consider that the gNB-CU has requested the gNB-DU to perform UE inactivity monitoring. If the Inactivity Monitoring Response IE is contained in the UE CONTEXT SETUP RESPONSE message and set to "Not-supported", the gNB-CU shall consider that the gNB-DU does not support UE inactivity monitoring for the UE.

If the ServCellInfoList IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall take it into account to generate the content of inter-node RRC message, i.e., CG-Config or CG-ConfigInfo, as described in TS 38.331 [8].

If the Full Configuration IE is contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

If the C-RNTI IE is included in the UE CONTEXT SETUP RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.

The UE Context Setup Procedure is not used to configure SRB0.

If the UE CONTEXT SETUP REQUEST message contains the RRC-Container IE, the gNB-DU shall send the corresponding RRC message to the UE via SRB1.

If the Notification Control IE is included in the DRB to Be Setup List IE contained in the UE CONTEXT SETUP REQUEST message and it is set to active, the gNB-DU shall, if supported, monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store the received UL PDU Session Aggregate Maximum Bit Rate and use it when enforcing uplink traffic policing for non-GBR Bearers for the concerned UE as specified in TS 23.501 [21].

The gNB-DU shall store the received gNB-DU UE Aggregate Maximum Bit Rate Uplink and use it for non-GBR Bearers for the concerned UE.

If the UE CONTEXT SETUP REQUEST message contains the QoS Flow Mapping Indication IE, the gNB-DU may take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the UE CONTEXT SETUP REQUEST message contains the New gNB-CU UE F1AP ID IE, the gNB-DU shall, if supported, replace the value received in the gNB-CU UE F1AP ID IE by the value of the New gNB-CU UE F1AP ID and use it for further signalling.

If the RAN UE ID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store and replace any previous information received.

If the Trace Activation IE is included in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, initiate the requested trace function as described in TS 32.422 [29].

In particular, the gNB-DU shall, if supported:

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT and Trace", initiate the requested trace session and MDT session as described in TS 32.422 [29];

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT Only", initiate the requested MDT session as described in TS 32.422 [29] and the gNB-DU shall ignore Interfaces To Trace IE, and Trace Depth IE. If the Management Based MDT PLMN List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store the received information in the UE context, and use this information to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [29].

For each QoS flow whose DRB has been successfully established and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [21].

If the UE CONTEXT SETUP REQUEST message contains the Configured BAP Address IE, the gNB-DU shall, if supported, store this BAP address configured for the corresponding child IAB-node and use it as specified in TS 38.340 [30].

If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Setup List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30].

If the F1-C Transfer Path IE is included in UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account.

If the NR V2X Services Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it contains one or more IEs set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized for the relevant service(s).

If the LTE V2X Services Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it contains one or more IEs set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized for the relevant service(s).

If the NR UE Sidelink Aggregate Maximum Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for NR V2X services.

If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for LTE V2X services.

If the PC5 Link Aggregate Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for NR V2X services as defined in TS 23.287 [40].

If the TSC Traffic Characteristics IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE.

If the Conditional Inter-DU Mobility Information IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall consider that the request concerns a conditional handover or conditional PSCell addition or conditional PSCell change for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Target gNB-DU UE F1AP ID IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU shall replace the existing prepared conditional handover or conditional PSCell addition or conditional PSCell change identified by the Target gNB-DU UE F1AP ID IE and the SpCell ID IE.

If the Serving NID IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall combine the Serving NID IE with the Serving PLMN IE to identify the serving NPN, and may take it into account for UE context establishment.

If the Estimated Arrival Probability IE is contained in the Conditional Inter-DU Mobility Information IE included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU may use the information to allocate necessary resources for the UE.

If for a given E-RAB for EN-DC operation the ENB DL Transport Layer Address IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If for a given Qos flow for NG-RAN operation the PDCP Terminating Node DL Transport Layer Address IE is included in the UE CONTEXT SETUP REQUEST message, then the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the F1-C Transfer Path NRDC IE is included in UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account.

If the MDT Polluted Measurement Indicator IE is included in the UE CONTEXT SETUP REQUEST, the gNB-DU shall take this information into account as specified in TS 38.401 [4].

If the SCG Activation Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU may use it to configure SCG resources as specified in TS 37.340 [7] , and if supported, shall include the SCG Activation Status IE in the UE CONTEXT SETUP RESPONSE message. If the SCG Activation Request IE in the UE CONTEXT SETUP REQUEST message is set to “Activate SCG”, the gNB-DU shall activate the SCG resources and set the SCG Activation Status IE in the UE CONTEXT SETUP RESPONSE message to “SCG Activated”.

If the Old CG-SDT Session Info IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, retrieve the old CG-SDT resource configuration and old UE context based on the indicated gNB-CU F1AP UE ID and gNB-DU F1AP UE ID.

If the 5G ProSe Authorized IE is contained in the UE CONTEXT SETUP REQUEST message and it contains one or more IEs set to "authorized", the gNB-DU node shall, if supported, consider that the UE is authorized for the relevant service(s).

If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for 5G ProSe services.

If the 5G ProSe PC5 Link Aggregate Bit Rate IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it for the concerned UE's sidelink communication in network scheduled mode for 5G ProSe services as defined in TS 23.304 [44].

If the Uu RLC Channel To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4]. gNB-DU generates the Uu Relay RLC channel configurations for a L2 U2N Relay UE.

If the PC5 RLC Channel To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4]. gNB-DU generates the PC5 Relay RLC channel configurations for a L2 U2N Remote UE.

If the Path Switch Configuration IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to configure the path switch from direct path to indirect path as specified in TS 38.401 [4].

If the MUSIM-GapConfig IE is contained in the CU to DU RRC Information IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, decide to use this IE for MUSIM gap configuration or select another one based on the received UEAssistanceInformation IE. If gNB-DU selects a different MUSIM gap configuration from received UEAssistanceInformation IE, then it shall include the selected MUSIM gap information to the gNB-CU in the MUSIM-GapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message.

If MUSIM-GapConfig IE is not contained in the CU to DU RRC Information IE, then gNB-DU shall, if supported, send the selected MUSIM gap configuration based on the received UEAssistanceInformation IE, to the gNB-CU in the MUSIM-GapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message. When MUSIM-GapConfig IE is received, the gNB-CU should use this value.

If the gNB-DU UE Slice Maximum Bit Rate List IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store and use the information for the uplink traffic policing for each concerned slice as specified in TS 23.501 [21].

If the Multicast MBS Session Setup List IE is contained in the UE CONTEXT SETUP REQUEST message the gNB-DU shall, if supported, store and use the information for configuring MBS Session Resources, if applicable.

If the UE Multicast MRB To Be Setup List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for configuring MBS Session Resources, if applicable. And if the MBS PTP Retransmission Tunnel Required IE is included in the UE Multicast MRB to Be Setup Item IEs IE, the gNB-DU shall, if supported trigger the establishment of the MBS PTP Retransmission F1-U tunnel. If the MBS PTP Forwarding Tunnel Required Information IE is included in the UE Multicast MRB to Be Setup Item IEs IE, the gNB-DU shall, if supported trigger the establishment of the MBS PTP Forwarding F1-U tunnel. If the Source MRB ID IE is included in the UE Multicast MRB to Be Setup Item IEs IE, the DU shall, if supported, use it to identify the MRB configuration as provided to the UE in the source cell and take it into account for configuring MBS Session Resources.

If the Dedicated SI Delivery Indication IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, take it into account for the system information delivery to the UE as described in TS 38.331 [8].

If the InterFrequencyConfig-NoGap IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the ul-GapFR2-Config IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the TwoPHRModeMCG IE or the TwoPHRModeSCG IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use this value as described in TS 38.331 [8].

If the MBSInterestIndication IE is included in the CU to DU RRC Information IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

If the ncd-SSB-RedCapInitialBWP-SDT IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: UE Context Setup Request procedure: unsuccessful Operation

If the gNB-DU is not able to establish an F1 UE context, or cannot even establish one bearer it shall consider the procedure as failed and reply with the UE CONTEXT SETUP FAILURE message. If the Conditional Inter-DU Mobility Information IE was included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall include the received SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT SETUP FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT SETUP REQUEST message, it shall reply with the UE CONTEXT SETUP FAILURE message with an appropriate cause value. Further, if the Candidate SpCell List IE is included in the UE CONTEXT SETUP REQUEST message and the gNB-DU is not able to accept the SpCell ID IE, the gNB-DU shall, if supported, include the Potential SpCell List IE in the UE CONTEXT SETUP FAILURE message and the gNB-CU should take this into account for selection of an opportune SpCell. The gNB-DU shall include the cells in the Potential SpCell List IE in a priority order, where the first cell in the list is the one most desired and the last one is the one least desired (e.g., based on load conditions). If the Potential SpCell List IE is present but no Potential SpCell Item IE is present, the gNB-CU should assume that none of the cells in the Candidate SpCell List IE are acceptable for the gNB-DU.

#### 8.3.1.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value. If the gNB-DU receives a UE CONTEXT SETUP REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT SETUP REQUEST message and is set to the value "delay critical" but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT SETUP RESPONSE message with an appropriate cause value.

In case of "CHO-replace" when the Target gNB-DU UE F1AP ID IE is included, if the candidate cell in the SpCell ID IE included in the UE CONTEXT SETUP REQUEST message was not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore this candidate cell.

### 8.3.2	UE Context Release Request (gNB-DU initiated)

#### 8.3.2.1	General

The purpose of the UE Context Release Request procedure is to enable the gNB-DU to request the gNB-CU to release the UE-associated logical F1-connection or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change. The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: UE Context Release (gNB-DU initiated) procedure. Successful operation

The gNB-DU controlling a UE-associated logical F1-connection initiates the procedure by generating a UE CONTEXT RELEASE REQUEST message towards the affected gNB-CU node.

The UE CONTEXT RELEASE REQUEST message shall indicate the appropriate cause value.

If the Candidate Cells To Be Cancelled List IE is included in the UE CONTEXT RELEASE REQUEST message, the gNB-CU shall consider that the only the resources reserved for the candidate cells identified by the included NR CGIs and associated to the UE-associated signaling identified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE are about to be released by the gNB-DU.

Interactions with UE Context Release procedure:

The UE Context Release procedure may be initiated upon reception of a UE CONTEXT RELEASE REQUEST message.

Interactions with UE Context Setup procedure:

The UE Context Release Request procedure may be performed before the UE Context Setup procedure to request the release of an existing UE-associated logical F1-connection and related resources in the gNB-DU.

#### 8.3.2.3	Abnormal Conditions

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the UE CONTEXT RELEASE REQUEST message were not prepared using the same UE-associated signaling connection, the gNB-CU shall ignore those non-associated candidate cells.

### 8.3.3	UE Context Release (gNB-CU initiated)

#### 8.3.3.1	General

The purpose of the UE Context Release procedure is to enable the gNB-CU to order the release of the UE-associated logical connection or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change. The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: UE Context Release (gNB-CU initiated) procedure. Successful operation

The gNB-CU initiates the procedure by sending the UE CONTEXT RELEASE COMMAND message to the gNB-DU.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall release all related signalling and user data transport resources and reply with the UE CONTEXT RELEASE COMPLETE message. If the CG-SDT Kept Indicator IE is contained in the UE CONTEXT RELEASE COMMAND message and set to "true", the gNB-DU shall, if supported, consider that the UE is sent to RRC_INACTIVE state with CG-SDT configuration and store the configured CG-SDT resources, C-RNTI, CS-RNTI, the CG-SDT related RLC configurations and F1-U connections associated with the SDT bearers while releasing the UE context.

If the old gNB-DU UE F1AP ID IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall additionally release the UE context associated with the old gNB-DU UE F1AP ID.

If the UE CONTEXT RELEASE COMMAND message contains the RRC-Container IE, the gNB-DU shall send the RRC container to the UE via the SRB indicated by the SRB ID IE.

If the UE CONTEXT RELEASE COMMAND message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication, if configured, for the SRB for the included RRC-Container IE.

If the Candidate Cells To Be Cancelled List IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall consider that the gNB-CU is cancelling only the conditional handover or conditional PSCell addition or conditional PSCell change associated to the cells identified by the included NR CGIs and associated to the UE-associated signaling identified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE.

If the Positioning Context Reservation Indication IE is included in the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall not release the positioning context including the SRS configuration for the UE.

Interactions with UE Context Setup procedure:

The UE Context Release procedure may be performed before the UE Context Setup procedure to release an existing UE-associated logical F1-connection and related resources in the gNB-DU, e.g. when gNB-CU rejects UE access it shall trigger UE Context Release procedure with the cause value of UE rejection.

#### 8.3.3.4	Abnormal Conditions

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the UE CONTEXT RELEASE COMMAND message were not prepared using the same UE-associated signalling connection, the gNB-DU shall ignore those non-associated candidate cells.

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

If the SpCell ID IE is included in the UE CONTEXT MODIFICATION REQUEST message and the Conditional Intra-DU Mobility Information IE is not present, the gNB-DU shall replace any previously received value and regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the ServCellIndex IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take this into account for the indicated SpCell. If the SpCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SpCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SpCell accordingly. If the servingCellMO List IE is included in the UE CONTEXT SETUP MODIFICATION REQUEST message, the gNB-DU shall, if supported, configure servingCellMO after determining the list of BWPs for the UE and include the list of servingCellMOs that have been encoded in CellGroupConfig IE as ServingCellMO-encoded-in-CGC List IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the Configured BWP List IE is included in the UE CONTEXT MODIFICATION RESPONSE message the gNB-CU shall, if supported, take it into account when requesting the gNB-DU for generating preconfigured measurement GAP for the indicated BWPs.

If the Preconfigured Measurement GAP Request IE is present in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATON REQUEST message, the gNB-DU shall, if supported, consider that the content of the previous CellGroupConfig IE was not sent to the UE and generate the pre-configured measurement GAP for the indicated BWPs in the MeasConfig IE. If the gNB-DU successfully generates pre-configured measurement GAP for the indicated BWPs, the gNB-DU shall update the CellGroupConfig IE with the content of the previous CellGroupConfig IE and the preconfigured measurement GAP configuration in the UE CONTEXT MODIFICATION RESPONSE message.

If the SCell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a list of candidate SCells to be set up. If the SCell To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message and the indicated SCell(s) are already setup, the gNB-DU shall replace any previously received value. If the SCell UL Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure UL for the indicated SCell accordingly. If the servingCellMO IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall configure servingCellMO for the indicated SCell accordingly.

If the SCell To Be Removed List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a list of SCells to be removed.

If the DRX Cycle IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall use the provided value from the gNB-CU. If the DRX configuration indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and set to "release", the gNB-DU shall release DRX configuration.

If the SL DRX Cycle list IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the provided value from the gNB-CU for the indicated RX UE of this UE. If the SL DRX configuration indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and set to "release", the gNB-DU shall, if supported, release SL DRX configuration for the indicated RX UE of this UE.

If the SRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4], and replace any previously received value. If Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup two RLC entities for the indicated SRB if the value is set to be "true", or delete the RLC entity of secondary path if the value is set to be "false". If the Additional Duplication Indication IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, setup the indicated RLC entities for the indicated SRB. If the SRB Mapping Info IE is contained in the SRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the SRB Mapping Info IE for the SRB identified by the SRB ID IE and the Uu Relay RLC channel identified by the SRB Mapping Info. The gNB-DU shall use the mapping information stored for the mapping of SRB data to Uu Relay RLC channel.

If the DRB To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in the TS 38.401 [4]. If the DRB Mapping Info IE is contained in the DRB To Be Setup List IE, the gNB-DU shall, if supported, store the mapping information indicated in the DRB Mapping Info IE, if present, for the DRB identified by the DRB ID IE and the Uu Relay RLC channel identified by the DRB Mapping Info. The gNB-DU shall use the mapping information stored for the mapping of DRB data to Uu Relay RLC channel.

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

If the BH RLC Channel To Be Setup List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the Traffic Mapping Information IE is included in the BH RLC Channel To Be Setup Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, process the Traffic Mapping Information IE following the behaviour described for the UE Context Setup procedure.

If the BH RLC Channel To Be Modified List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall act as specified in TS 38.401 [4]. If the Traffic Mapping Information IE is included in the BH RLC Channel To Be Modified Item IEs IE for a BH RLC Channel, the gNB-DU shall, if supported, process the Traffic Mapping Information IE following the behaviour described for the UE Context Setup procedure.

If the BH RLC Channel To Be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall release the BH RLC channels in the list.

If two UL UP TNL Information IEs are included in UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall include two DL UP TNL Information IEs in UE CONTEXT MODIFICATION RESPONSE message and setup two RLC entities for the indicated DRB. gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2]. The first UP TNL Information IE of the two UP TNL Information IEs is for the primary path.

If one or two Additional PDCP Duplication UP TNL Information IEs are included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall, if supported, include one or two Additional PDCP Duplication UP TNL Information IEs in the UE CONTEXT MODIFICATION RESPONSE message and setup one or two additional RLC entities for the indicated DRB. The gNB-CU and the gNB-DU use the Additional PDCP Duplication UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2].

If Duplication Activation IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU should take it into account when activating/deactivating CA based PDCP duplication for the DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for the DRB with more than two RLC entities.

If DC Based Duplication Configured IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall regard that DC based PDCP duplication is configured for this DRB if the value is set to be "true" and it should take the responsibility of PDCP duplication activation/deactivation. Otherwise, the gNB-DU shall regard that DC based PDCP duplication is de-configured for this DRB id the value is set to be "false", and it should stop PDCP duplication activation/deactivation by MAC CE. If DC Based Duplication Activation IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU should take it into account when activating/deactivating DC based PDCP duplication for this DRB. If the RLC Duplication State List IE is included in the RLC Duplication Information IE contained in the UE CONTEXT MODIFICATION REQUEST message for a DRB, the gNB-DU shall, if supported, take it into account when activating/deactivating DC based PDCP duplication for the DRB with more than two RLC entities. If the Primary Path Indication IE is included in the RLC Duplication Information IE, the gNB-DU shall, if supported, take it into account when performing DC based PDCP duplication for the DRB with more than two RLC entities.

For a certain DRB which was allocated with two GTP-U tunnels, if such DRB is modified and given one GTP-U tunnel via the UE Context Modification procedure, the gNB-DU shall consider that the CA based PDCP duplication for the concerned DRB is de-configured. If such UE Context Modification procedure occurs, the Duplication Activation IE shall not be included for the concerned DRB.

If the UL Configuration IE in DRB to Be Setup Item IE or DRB to Be Modified Item IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall take it into account for UL scheduling.

If the RRC Reconfiguration Complete Indicator IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider the ongoing reconfiguration procedure involving changes of the L1/L2 configuration at the gNB-DU signalled to the gNB-CU via the CellGroupConfig IE for MR-DC operation or standalone operation has been successfully performed when such IE is set to ‘true’; otherwise (when such IE is set to ‘failure’), the gNB-DU shall consider the ongoing reconfiguration procedure has been failed and it shall continue to use the old L1/L2 configuration.

If DL PDCP SN length IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

If UL PDCP SN length IE is included in the UE CONTEXT MODIFICATION REQUEST message for a DRB, gNB-DU shall, if supported, store this information and use it for lower layer configuration.

If the RLC Failure Indication IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU should consider that the RLC entity indicated by such IE needs to be re-established when the CA-based packet duplication is active, and the gNB-DU may include the Associated SCell List IE in UE CONTEXT MODIFICATION RESPONSE by containing a list of SCell(s) associated with the RLC entity indicated by the RLC Failure Indication IE.

If the UE CONTEXT MODIFICATION REQUEST message contains the RRC-Container IE, the gNB-DU shall send the corresponding RRC message to the UE. If the UE CONTEXT MODIFICATION REQUEST message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication, if configured, for the SRB for the included RRC-Container IE.

If the UE CONTEXT MODIFICATION REQUEST message contains the Transmission Action Indicator IE, the gNB-DU shall stop or restart (if already stopped) data transmission for the UE, according to the value of this IE. It is up to gNB-DU implementation when to stop or restart the UE scheduling.

For EN-DC operation, if the DRB to Be Setup List IE is present in the UE CONTEXT MODIFICATION REQUEST message the gNB-CU shall include the E-UTRAN QoS IE. The allocation of resources according to the values of the Allocation and Retention Priority IE included in the E-UTRAN QoS IE shall follow the principles described for the E-RAB Setup procedure in TS 36.413 [15]. For NG-RAN operation, the gNB-CU shall include the DRB Information IE in the UE CONTEXT MODIFICATION REQUEST message.

If the gNB-CU includes the SMTC information of the measured frequency(ies) in the MeasurementTimingConfiguration IE of the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate the measurement gaps based on the received SMTC information. Then the gNB-DU shall send the measurement gaps information to the gNB-CU in the MeasGapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message.

If the MeasConfig IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall deduce that changes to the measurements’ configuration need to be applied. The gNB-DU shall take the received info, e.g. the measObjectToAddModList IE, and/or the measObjectToRemoveList IE into account, when generating measurement gap and when deciding if a measurement gap is needed or not.

If the NeedForGapsInfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForGapNCSG-InfoNR IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8]. If the NeedForGapNCSG-InfoEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as described in TS 38.331 [8].

For DC operation, if the gNB-CU includes the CG-Config IE in the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU may initiate low layer parameters coordination taking this information into account.

For sidelink operation, the CG-ConfigInfo IE shall be included in the CU to DU RRC Information IE if the gNB-CU receives sidelink related UE information from UE. If the CG-ConfigInfo IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall regard it as an indication of V2X sidelink information or NR sidelink information as defined in TS 38.331 [8].

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], after completion of UE Context Setup procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Transfer Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT MODIFICATION REQUEST message shall be ignored.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], after completion of UE Context Setup procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION REQUEST message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

For EN-DC operation, and if the Subscriber Profile ID for RAT/Frequency priority IE is received from an MeNB, the UE CONTEXT MODIFICTION REQUEST message shall contain the Subscriber Profile ID for RAT/Frequency priority IE. If the Additional RRM Policy Index IE is received from an MeNB, the UE CONTEXT MODIFICATION REQUEST message shall , if supported, contain the Additional RRM Policy Index IE. The gNB-DU shall store the received Subscriber Profile ID for RAT/Frequency priority in the UE context and use it as defined in TS 36.300 [20]. The gNB-DU shall, if supported, store the received Additional RRM Policy Index in the UE context and use it as defined in TS 36.300 [20].

If the Index to RAT/Frequency Selection Priority IE is modified at the gNB-CU, the Index to RAT/Frequency Selection Priority IE shall be included in the UE CONTEXT MODIFICATION REQUEST. The gNB-DU may use it for RRM purposes.

Only one of the following IEs shall be contained in the UE CONTEXT MODIFICATION REQUEST message: the Uplink TxDirectCurrentList Information IE or the Uplink TxDirectCurrentTwoCarrierList Information IE or the Uplink TxDirectCurrentMoreCarrierList Information IE. If the UE CONTEXT MODIFICATION REQUEST message contains one of the Uplink TxDirectCurrentList Information IE or the Uplink TxDirectCurrentTwoCarrierList Information IE or the Uplink TxDirectCurrentMoreCarrierList Information IE, the gNB-DU may take that into account when selecting L1 configuration.

The UEAssistanceInformation IE shall be included in CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformation IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

The UEAssistanceInformationEUTRA IE shall be included in CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message if the gNB-CU received this IE from the UE; if the UEAssistanceInformationEUTRA IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring LTE sidelink resources for the UE.

The gNB-DU shall report to the gNB-CU, in the UE CONTEXT MODIFICATION RESPONSE message, the result for all the requested or modified DRBs, SRBs, BH RLC Channels, Uu Relay RLC channels, PC5 Relay RLC channels, and SL DRBs in the following way:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed to be Setup List IE;

-	A list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be modified shall be included in the DRB Failed to be Modified List IE;

-	A list of SRBs which failed to be established shall be included in the SRB Failed to be Setup List IE.

-	A list of successfully established SRBs with logical channel identities for primary path shall be included in the SRB Setup List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of successfully modified SRBs with logical channel identities for primary path shall be included in the SRB Modified List IE only if CA based PDCP duplication is initiated for the concerned SRBs.

-	A list of BH RLC channels which are successfully established shall be included in the BH RLC Channel Setup List IE;

-	A list of BH RLC channels which failed to be established shall be included in the BH RLC Channel Failed to be Setup List IE;

-	A list of BH RLC channels which are successfully modified shall be included in the BH RLC Channel Modified List IE;

-	A list of BH RLC channels which failed to be modified shall be included in the BH RLC Channel Failed to be Modified List IE;

-	A list of Uu Relay RLC channels which are successfully established shall be included in the Uu RLC Channel Setup List IE;

-	A list of Uu Relay RLC channels which failed to be established shall be included in the Uu RLC Channel Failed to be Setup List IE;

-	A list of Uu Relay RLC channels which are successfully modified shall be included in the Uu RLC Channel Modified List IE;

-	A list of Uu Relay RLC channels which are failed to be modified shall be included in the Uu RLC Channel Failed to be Modified List IE;

-	A list of PC5 Relay RLC channels which are successfully established shall be included in the PC5 RLC Channel Setup List IE;

-	A list of PC5 Relay RLC channels which failed to be established shall be included in the PC5 RLC Channel Failed to be Setup List IE;

-	A list of PC5 Relay RLC channels which are successfully modified shall be included in the PC5 RLC Channel Modified List IE;

-	A list of PC5 Relay RLC channels which failed to be modified shall be included in the PC5 RLC Channel Failed to be Modified List IE;

-	A list of SL DRBs which are successfully established shall be included in the SL DRB Setup List IE;

-	A list of SL DRBs which failed to be established shall be included in the SL DRB Failed to be Setup List IE;

-	A list of SL DRBs which are successfully modified shall be included in the SL DRB Modified List IE;

-	A list of SL DRBs which failed to be modified shall be included in the SL DRB Failed to be Modified List IE.

For each GBR DRB, if the Alternative QoS Parameters Sets IE is included in the GBR QoS Flow Information IE in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU shall, if supported, behave the same as the NG-RAN node in the PDU Session Resource Setup procedure, specified in TS 38.413 [3].

If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Setup List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30].

If the BAP Control PDU Channel IE is included in the BH RLC Channel to be Modified List IE, the gNB-DU shall, if supported, consider that the configured BH RLC channel can be used to transmit BAP Control PDUs, and use this BH RLC channel as specified in TS 38.340 [30]. Otherwise, if the BAP Control PDU Channel IE is not present for any BH RLC channel, any available BH RLC channel can be used to transmit BAP Control PDUs as specified in TS 38.340 [30].

If the F1-C Transfer Path IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account.

When the gNB-DU reports the unsuccessful establishment of a DRB or SRB or SL DRB or a BH RLC channel or a Uu Relay RLC channel or a PC5 Relay RLC channel, the cause value should be precise enough to enable the gNB-CU to know the reason for the unsuccessful establishment.

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT MODIFICATION RESPONSE, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

If the DU to CU RRC Information IE is included in the UE CONTEXT MODIFICATION RESPONSE message, except for the CG-SDT procedure and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of CG-SDT, and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

If the ServCellInfoList IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall take it into account to generate the content of inter-node message, i.e., CG-Config or CG-ConfigInfo, as described in TS 38.331 [8].

If the UE-CapabilityRAT-ContainerList IE is included in the UE CONTEXT SETUP MODIFICATION REQUEST, the gNB-DU shall take this information into account for UE specific configurations.

If the SCell Failed To Setup List IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the corresponding SCell(s) failed to be set up with an appropriate cause value for each SCell failed to setup.

If the C-RNTI IE is included in the UE CONTEXT MODIFICATION RESPONSE, the gNB-CU shall consider that the C-RNTI has been allocated by the gNB-DU for this UE context.

If the Inactivity Monitoring Request IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU may consider that the gNB-CU has requested the gNB-DU to perform UE inactivity monitoring. If the Inactivity Monitoring Response IE is contained in the UE CONTEXT MODIFICATION RESPONSE message and set to "Not-supported", the gNB-CU shall consider that the gNB-DU does not support UE inactivity monitoring for the UE.

The UE Context Modify Procedure is not used to configure SRB0.

If in the UE CONTEXT MODIFICATION REQUEST, the Notification Control IE is included in the DRB to Be Setup List IE or the DRB to Be Modified List IE and it is set to active, the gNB-DU shall, if supported, monitor the QoS of the DRB and notify the gNB-CU if the QoS cannot be fulfilled any longer or if the QoS can be fulfilled again. The Notification Control IE can only be applied to GBR bearers.

If the UL PDU Session Aggregate Maximum Bit Rate IE is included in the QoS Flow Level QoS Parameters IE containded in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall replace the received UL PDU Session Aggregate Maximum Bit Rate and use it as specified in TS 23.501 [21].

If the gNB-DU UE Aggregate Maximum Bit Rate Uplink IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall:

-	replace the previously provided gNB-DU UE Aggregate Maximum Bit Rate Uplink with the new received gNB-DU UE Aggregate Maximum Bit Rate Uplink;

-	use the received gNB-DU UE Aggregate Maximum Bit Rate Uplink for non-GBR Bearers for the concerned UE.

The gNB-DU UE Aggregate Maximum Bit Rate Uplink IE shall be sent in the UE CONTEXT MODIFICATION REQUEST if DRB to Be Setup List IE is included and the gNB-CU has not previously sent it. The gNB-DU shall store and use the received gNB-DU UE Aggregate Maximum Bit Rate Uplink IE.

If the RLC Status IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall assume that RLC has been reestablished at the gNB-DU and may trigger PDCP data recovery.

If the GNB-DU Configuration Query IE is contained in the UE CONTEXT MODIFICATION REQUEST message, gNB-DU shall include the DU To CU RRC Information IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the Bearer Type Change IE is included in DRB to Be Modified List IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall either reset the lower layers or generate a new LCID for the affected bearer as specified in TS 37.340 [7].

For NE-DC operation, if NeedforGap IE is included in the UE CONTEXT MODIFICATION REQUEST message,the gNB-DU shall generate measurement gap for the SeNB.

If the QoS Flow Mapping Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, replace any previously received value and take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the Lower Layer presence status change IE set to "suspend lower layers" is included in the UE CONTEXT MODIFICATION REQUEST, the gNB-DU shall keep all lower layer configuration for UEs, and not transmit or receive data from UE.

If the Lower Layer presence status change IE set to "resume lower layers" is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall use the previously stored lower layer configuration for the UE.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate a CellGroupConfig IE using full configuration and include it in the UE CONTEXT MODIFICATION RESPONSE.

If the Full Configuration IE is contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall consider that the gNB-DU has generated the CellGroupConfig IE using full configuration.

For each QoS flow whose DRB has been successfully established or modified and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [21].

If the NR V2X Services Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its V2X services authorization information for the UE accordingly. If the NR V2X Services Authorized IE includes one or more IEs set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the LTE V2X Services Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its V2X services authorization information for the UE accordingly. If the LTE V2X Services Authorized IE includes one or more IEs set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the LTE UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE LTE Sidelink Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for LTE V2X services.

If the NR UE Sidelink Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE NR Sidelink Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services.

If the PC5 Link Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided UE PC5 Link Aggregate Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for NR V2X services as defined in TS 23.287 [40].

If the TSC Traffic Characteristics IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE.

If the CPAC MCG Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CPAC Trigger is set to "CPAC-preparation", the gNB-DU shall, if supported, consider that the request concerns a conditional PSCell addition or conditional PSCell change. The gNB-DU takes the included CG-Config and/or CG-ConfigInfo IE into account, and may provide a corresponding CellGroupConfig IE for MCG configuration preparation in the UE CONTEXT MODIFICATION RESPONSE message. The UE CONTEXT MODIFICATION RESPONSE message also includes a Requested Target Cell ID IE corresponding to the PSCell ID IE in the UE CONTEXT MODIFICATION REQUEST message.

If the CPAC MCG Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CPAC Trigger is set to "CPAC-executed", the gNB-DU shall, if supported, consider that, for the included PSCell ID IE corresponding to the selected PSCell, the UE has successfully executed the CPAC preparation. The gNB-DU shall apply the corresponding CellGroupConfig IE for MCG configuration.

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-initiation", the gNB-DU shall consider that the request concerns a conditional handover or conditional PSCell addition or conditional PSCell change for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-replace", the gNB-DU shall replace the existing prepared conditional mobility identified by the gNB-DU UE F1AP ID IE and the SpCell ID IE.

If the Conditional Intra-DU Mobility Information IE is included in the UE CONTEXT MODIFICATION REQUEST message and the CHO Trigger is set to "CHO-cancel", the gNB-DU shall consider that the gNB-CU is about to remove any reference to, and release any resources previously reserved for the candidate cells associated to the UE-associated signalling identified by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE. If the Candidate Cells To Be Cancelled List IE is also included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider that only the resources reserved for the cells identified by the included NR CGIs are about to be released by the gNB-CU.

If the Transmission Stop Indicator IE is included within the DRB to Be Modified Item IE in the UE CONTEXT MODIFICATION REQUEST message and set to “true”, the gNB-DU shall, if supported, stop the data transmission for the DRB. It is up to gNB-DU implementation when to stop the UE scheduling for that DRB.

If the SCG Indicator IE is contained in the UE CONTEXT MODIFICATION REQUEST message and it is set to “released”, the gNB-DU shall, if supported, deduce that an SCG is removed.

If the Estimated Arrival Probability IE is contained in the Conditional Intra-DU Mobility Information IE included in the UE CONTEXT MODIFICATION REQUEST message, then the gNB-DU may use the information to allocate necessary resources for the UE.

If the Location Measurement Information IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring measurement gaps for the UE.

If the F1-C Transfer Path NRDC IE is included in UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account.

If for a given E-RAB for EN-DC operation the ENB DL Transport Layer Address IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If for a given Qos flow for NG-RAN operation the PDCP Terminating Node DL Transport Layer Address IE is included in the UE CONTEXT MODIFICATION REQUEST message, then the gNB-DU shall, if supported, use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the gNB-DU is an IAB-DU, and if the IAB Conditional RRC Message Delivery Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message together with the RRC-Container IE, and if its value is set to “true”, and if the RRC-Container IE is for a child IAB-MT of the gNB-DU, the gNB-DU shall, if supported, withhold the RRC message until one of the following conditions is met:

If the gNB-DU belongs to a migrating IAB-node, whose co-located IAB-MT has successfully performed the random-access procedure to the target parent node, and if the migrating IAB-node has one or more routing entries for the target path.

The gNB-DU receives a subsequent F1AP message including an RRC-Container IE for the same child node.

If the gNB-DU belongs to a descendant node of the migrating IAB-node, whose co-located IAB-MT has received an RRCReconfiguration message including the intra-donor migration configurations, e.g., new TNL address(es) and the new default UL BAP routing ID.

If the gNB-DU belongs to a migrating IAB-node, whose co-located IAB-MT has successfully performed RLF recovery after handover failure, and if the migrating IAB-node has one or more routing entries for the target path.

If the MDT Polluted Measurement Indicator IE is included in the UE CONTEXT MODIFICATION REQUEST, the gNB-DU shall take this information into account as specified in TS 38.401 [4].

If the SCG Activation Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU may use it to configure SCG resources as specified in TS 37.340 [7] , and if supported, shall include the SCG Activation Status IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the CG-SDT Query Indication IE is included in the UE CONTEXT MODIFICATION REQUEST message and set to ‘true’, the gNB-DU shall, if supported, provide the CG-SDT related resource configuration for the bearers indicated as SDT bearers in the SDT-MAC-PHY-CG-Config IE within the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU. If the SDT-MAC-PHY-CG-Config IE is also included in the UE CONTEXT MODIFICATION REQUEST message within the CU to DU RRC Information IE, the gNB-DU may provide the delta signalling version of the SDT-MAC-PHY-CG-Config IE within the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message to the gNB-CU.

If the 5G ProSe Authorized IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its 5G ProSe services authorization information for the UE accordingly. If the 5G ProSe Authorized IE includes one or more IEs set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the relevant service(s).

If the SDT Bearer Configuration Query Indication IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, provide the RLC bearer configuration in the SDT Bearer Configuration Info IE in the UE CONTEXT MODIFICATION RESPONSE message for each bearer indicated as SDT bearer.

If the 5G ProSe UE PC5 Aggregate Maximum Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided 5G ProSe UE PC5 Aggregate Maximum Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services.

If the 5G ProSe PC5 Link Aggregate Bit Rate IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported:

-	replace the previously provided 5G ProSe PC5 Link Aggregate Bit Rate, if available in the UE context, with the received value;

-	use the received value for the concerned UE’s sidelink communication in network scheduled mode for 5G ProSe services as defined in TS 23.304 [44].

If the Updated Remote UE Local ID IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, replace the previously provided Remote UE Local ID, if available in the UE context, with the received value.

If the Uu RLC Channel To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4].

If the Uu RLC Channel To Be Modified List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4].

If the Uu RLC Channel To Be Release List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the Uu Relay RLC channels in the list.

If the PC5 RLC Channel To Be Setup List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4]. gNB-DU generates the PC5 Relay RLC channel configurations for a L2 U2N Remote UE or U2N Relay UE. If the F1AP-IDs are associated with a U2N Relay UE, the PC5 RLC Channel to be Setup Item IEs IE shall include the Remote UE Local ID and correspondingly, the PC5 RLC Channel Setup Item IEs IE and the PC5 RLC Channel Failed to be Setup Item IE in the UE CONTEXT MODIFICATION RESPONSE message shall include the Remote UE Local ID IE.

If the PC5 RLC Channel To Be Modified List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, act as specified in TS 38.401 [4]. gNB-DU generates the PC5 Relay RLC channel configurations for a L2 U2N Remote UE or U2N Relay UE. If the F1AP-IDs are associated with a U2N Relay UE, the PC5 RLC Channel to be Modified Item IEs IE shall include the Remote UE Local ID IE and correspondingly, the PC5 RLC Channel Modified Item IEs IE and the PC5 RLC Channel Failed to be Modified Item IEs IE in the UE CONTEXT MODIFICATION RESPONSE message shall include the Remote UE Local ID IE.

If the PC5 RLC Channel To Be Release List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the PC5 Relay RLC channels in the list. If the F1AP-IDs are associated with a U2N Relay UE, the PC5 RLC Channel to be Released Item IEs IE shall include the Remote UE Local ID IE.

If the Path Switch Configuration IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it to configure the path switch from direct path to indirect path as specified in TS 38.401 [4].

If the MUSIM-GapConfig IE is contained in the CU to DU RRC Information IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, decide to use this IE for MUSIM gap configuration or select another one based on the received UEAssistanceInformation IE. If gNB-DU selects a different MUSIM gap configuration from received UEAssistanceInformation IE, then it shall include the selected MUSIM gap information to the gNB-CU in the MUSIM-GapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message.

If MUSIM-GapConfig IE is not contained in the CU to DU RRC Information IE, then gNB-DU shall, if supported, send the selected MUSIM gap configuration based on the received UEAssistanceInformation IE, to the gNB-CU in the MUSIM-GapConfig IE of the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message. When MUSIM-GapConfig IE is received, the gNB-CU should use this value.

If the gNB-DU UE Slice Maximum Bit Rate List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported,

-	store and replace the previously provided gNB-DU UE Slice Maximum Bit Rate List, if any, with the new received gNB-DU UE Slice Maximum Bit Rate List;

-	use the received gNB-DU UE Slice Maximum Bit Rate List for the uplink traffic policing for each concerned slice as specified in TS 23.501 [21].

If the Multicast MBS Session Setup List IE or the Multicast MBS Session Remove List IE or both IEs are contained in the UE CONTEXT MODIFICATION REQUEST message the gNB-DU shall, if supported, store and use the information for configuring MBS Session Resources, if applicable.

If the UE Multicast MRB To Be Setup at Modify List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for configuring MBS Session Resources, if applicable, and shall include the Multicast F1-U Context Reference CU IE, if available, in the UE CONTEXT MODIFICATION RESPONSE message. And if the MBS PTP Retransmission Tunnel Required IE is included in the UE Multicast MRB to Be Setup at Modify Item IEs IE, the gNB-DU shall, if supported trigger the establishment of the MBS PTP Retransmission F1-U tunnel.

If the MBS PTP Forwarding Tunnel Required Information IE is included in the UE Multicast MRB to Be Setup at Modify Item IEs IE, the gNB-DU shall, if supported trigger the establishment of the MBS PTP Forwarding F1-U tunnel.

If the Management Based MDT PLMN Modification List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, overwrite any previously stored Management Based MDT PLMN List information in the UE context and use the received information to determine subsequent selection of the UE for management based MDT defined in TS 32.422 [29].

If the Dedicated SI Delivery Indication IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, take it into account for the system information delivery to the UE as described in TS 38.331 [8].

If the InterFrequencyConfig-NoGap IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the ul-GapFR2-Config IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

If the TwoPHRModeMCG IE or the TwoPHRModeSCG IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use this value as described in TS 38.331 [8].

If the MBSInterestIndication IE is included in the CU to DU RRC Information IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account when configuring resources for the UE.

If the ncd-SSB-RedCapInitialBWP-SDT IE is contained in the DU to CU RRC Information IE that is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, use it as described in TS 38.331 [8].

#### 8.3.4.3	Unsuccessful Operation

Figure 8.3.4.3-1: UE Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the UE context can be successfully performed, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value. If the Conditional Intra-DU Mobility Information IE was included in the UE CONTEXT MODIFICATION REQUEST message and set to "CHO-initiation", the gNB-DU shall include the received SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION FAILURE message.

If the gNB-DU is not able to accept the SpCell ID IE in UE CONTEXT MODIFICATION REQUEST message, it shall reply with the UE CONTEXT MODIFICATION FAILURE message.

If the Conditional Intra-DU Mobility Information IE was included and set to "CHO-initiation" or "CHO-replace" but the SpCell ID IE was not included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall respond with the UE CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

#### 8.3.4.4	Abnormal Conditions

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a E-UTRAN QoS IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the gNB-DU receives a UE CONTEXT MODIFICATION REQUEST message containing a DRB QoS IE for a GBR QoS DRB but where the GBR QoS Flow Information IE is not present, the gNB-DU shall report the establishment of the corresponding DRBs as failed in the DRB Failed to Setup List IE of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the Delay Critical IE is included in the Dynamic 5QI Descriptor IE within the DRB QoS IE in the UE CONTEXT MODIFICATION REQUEST message and is set to the value "delay critical" but the Maximum Data Burst Volume IE is not present, the gNB-DU shall report the establishment of the corresponding DRB as failed in the DRB Failed to Setup List IE of the of the UE CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the UE CONTEXT MODIFICATION REQUEST message were not prepared using the same UE-associated signaling connection, the gNB-DU shall ignore those non-associated candidate cells.

If more than one of the following IEs, i.e., the Uplink TxDirectCurrentList Information IE or the Uplink TxDirectCurrentTwoCarrierList Information IE or the Uplink TxDirectCurrentMoreCarrierList Information IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall consider it as a logical error.

### 8.3.5	UE Context Modification Required (gNB-DU initiated)

#### 8.3.5.1	General

The purpose of the UE Context Modification Required procedure is to modify the established UE Context, e.g., modifying and releasing radio bearer resources, or sidelink radio bearer resources or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change. The procedure uses UE-associated signalling.

#### 8.3.5.2	Successful Operation

Figure 8.3.5.2-1: UE Context Modification Required procedure. Successful operation

The F1AP UE CONTEXT MODIFICATION REQUIRED message is initiated by the gNB-DU.

The gNB-CU reports the successful update of the UE context in the UE CONTEXT MODIFICATION CONFIRM message.

For a given bearer for which PDCP CA duplication was already configured, if two DL UP TNL Information IEs are included in UE CONTEXT MODIFICATION REQUIRED message for a DRB, the gNB-CU shall include two UL UP TNL Information IEs in UE CONTEXT MODIFICATION CONFIRM message. The gNB-CU and gNB-DU use the UL UP TNL Information IEs and DL UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2], and the first UP TNL Information IE is still for the primary path.

For a given bearer for which PDCP CA duplication was already configured, if one or two Additional PDCP Duplication UP TNL Information IEs are included in the UE CONTEXT MODIFICATION REQUIRED message for a DRB, the gNB-CU shall, if supported, include one or two Additional PDCP Duplication UP TNL Information IEs in the UE CONTEXT MODIFICATION CONFIRM message. The gNB-CU and gNB-DU use the Additional PDCP Duplication UP TNL Information IEs to support packet duplication for intra-gNB-DU CA as defined in TS 38.470 [2].

If the BH Information IE is included in the UL UP TNL Information to be setup List IE or the Additional PDCP Duplication TNL List IE for a DRB, the gNB-DU shall, if supported, use the indicated BAP Routing ID and BH RLC channel for transmission of the corresponding GTP-U packets to the IAB-donor, as specified in TS 38.340 [30].

If the Resource Coordination Transfer Container IE is included in the UE CONTEXT MODIFICATION REQUIRED, the gNB-CU shall transparently transfer this information for the purpose of resource coordination as described in TS 36.423 [9], TS 38.423 [28].

For EN-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION CONFIRM message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MeNB Resource Coordination Information as defined in TS 36.423 [9], after completion of UE Context Modification Required procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION CONFIRM message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MeNB Resource Coordination Information at the gNB acting as secondary node as described in TS 36.423 [9]. If the Resource Coordination E-UTRA Cell Information IE is included in the Resource Coordination Transfer Information IE, the gNB-DU shall store the information replacing previously received information for the same E-UTRA cell, and use the stored information for the purpose of resource coordination. If the Ignore PRACH Configuration IE is present and set to "true" the E-UTRA PRACH Configuration IE in the UE CONTEXT MODIFICATION CONFIRM message shall be ignored.

For NGEN-DC or NE-DC operation, if the gNB-CU includes the Resource Coordination Transfer Information IE in the UE CONTEXT MODIFICATION CONFIRM message, the gNB-DU shall, if supported, use it for the purpose of resource coordination. If the gNB-CU received the MR-DC Resource Coordination Information as defined in TS 38.423 [28], after completion of UE Context Modification Required procedures, the gNB-CU shall transparently transfer it to the gNB-DU via the Resource Coordination Transfer Container IE in the UE CONTEXT MODIFICATION CONFIRM message. The gNB-DU shall use the information received in the Resource Coordination Transfer Container IE for reception of MR-DC Resource Coordination Information at the gNB as described in TS 38.423 [28].

If the DU to CU RRC Information IE is included in the UE CONTEXT MODIFICATION REQUIRED message, the gNB-CU shall perform RRC Reconfiguration as described in TS 38.331 [8]. The CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8].

If the ServCellInfoList IE is included in the DU to CU RRC Information IE contained in the UE CONTEXT MODIFICATION REQUIRED message, the gNB-CU shall take it into account to generate the content of inter-node message, i.e., CG-Config or CG-ConfigInfo, as described in TS 38.331 [8].

If the UE CONTEXT MODIFICATION CONFIRM message includes the Execute Duplication IE, the gNB-DU shall perform CA based duplication, if configured, for the SRB for the included RRC-Container IE.

If the UE CONTEXT MODIFICATION REQUIRED message contains the RLC Status IE, the gNB-CU shall assume that RLC has been reestablished at the gNB-DU and may trigger PDCP data recovery.

If the Candidate Cells To Be Cancelled List IE is included in the UE CONTEXT MODIFICATION REQUIRED message, the gNB-CU shall consider that only the resources reserved for the candidate cells identified by the included NR CGIs and associated to the UE-associated signaling identified by the gNB-CU UE F1AP ID IE and the gNB-CU UE F1AP ID IE are about to be released by the gNB-DU.

If the PC5 RLC Channel Required to be Modified List IE or the PC5 RLC Channel Required to be Released List IE is included in the UE CONTEXT MODIFICATION REQUIRED message and the F1AP-IDs is associated with a U2N Relay UE, the PC5 RLC Channel Required to be Modified List IE or the PC5 RLC Channel Required to be Released List shall include the Remote UE Local ID and correspondingly, the PC5 RLC Channel Modified Item IEs in the UE CONTEXT MODIFICATION CONFIRM message shall include the Remote UE Local ID IE.

If the UE Multicast MRB Required to Be Modified List IE is included in the UE CONTEXT MODIFICATION REQUIRED message

-	containing for an MRB the MRB type reconfiguration IE set to "true" the gNB-CU shall take the MRB Reconfigured RLC mode IE into account to reconfigure the UE and to decide whether to request a PDCP status report as specified in TS 38.300 [6] and include the MBS PTP Retransmission Tunnel Required IE in the UE Multicast MRB Confirmed to Be Modified Item IEs IE.

-	containing for an MRB the Multicast F1-U Context Reference CU IE the gNB-CU shall, if supported, replace previously provided information by the newly received and take it into account when retrieving MRB progress information.

Interaction with the Multicast Distribution Setup procedure:

If the UE CONTEXT MODIFICATION CONFIRM message contains for an MRB the MBS PTP Retransmission Tunnel Required IE in the UE Multicast MRB Confirmed to Be Modified Item IEs IE the gNB-DU shall, if supported, trigger the Multicast Distribution Setup procedure to setup requested F1-U resources, if applicable.

#### 8.3.5.2A	Unsuccessful Operation

Figure 8.3.5.2A-1: UE Context Modification Required procedure. Unsuccessful operation.

In case none of the requested modifications of the UE context can be successfully performed, the gNB-CU shall respond with the UE CONTEXT MODIFICATION REFUSE message with an appropriate cause value.

#### 8.3.5.3	Abnormal Conditions

If one or more candidate cells in the Candidate Cells To Be Cancelled List IE included in the UE CONTEXT MODIFICATION REQUIRED message were not prepared using the same UE-associated signaling connection, the gNB-CU shall ignore those non-associated candidate cells.

### 8.3.6	UE Inactivity Notification

#### 8.3.6.1	General

This procedure is initiated by the gNB-DU to indicate the UE activity event.

The procedure is also used to request the termination of SDT session.

The procedure uses UE-associated signalling.

#### 8.3.6.2	Successful Operation

Figure 8.3.6.2-1: UE Inactivity Notification procedure.

The gNB-DU initiates the procedure by sending the UE INACTIVITY NOTIFICATION message to the gNB-CU.

If the DRB ID IE is included in the DRB Activity Item IE in the UE INACTIVITY NOTIFICATION message, the DRB Activity IE shall also be included

If the gNB-CU receives the SDT Termination Request IE in the UE INACTIVITY NOTIFICATION message, the gNB-CU shall, if supported, consider that the termination of the ongoing SDT transaction is requested from the gNB-DU for this UE and act as specified in TS 38.300 [6].

#### 8.3.6.3	Abnormal Conditions

Not applicable.

### 8.3.7	Notify

#### 8.3.7.1	General

The purpose of the Notify procedure is to enable the gNB-DU to inform the gNB-CU that the QoS of an already established GBR DRB cannot be fulfilled any longer or that it can be fulfilled again. The procedure uses UE-associated signalling.

#### 8.3.7.2	Successful Operation

Figure 8.3.7.2-1: Notify procedure. Successful operation.

The gNB-DU initiates the procedure by sending a NOTIFY message.

The NOTIFY message shall contain the list of the GBR DRBs associated with notification control for which the QoS is not fulfilled anymore or for which the QoS is fulfilled again by the gNB-DU. The gNB-DU may also indicate an alternative QoS parameters set which it can currently fulfil in the Current QoS Parameters Set Index IE.

Upon reception of the NOTIFY message, the gNB-CU may identify which are the affected PDU sessions and QoS flows. The gNB-CU may inform the 5GC that the QoS for these PDU sessions or QoS flows is not fulfilled any longer or it is fulfilled again.

#### 8.3.7.3	Abnormal Conditions

Not applicable.

### 8.3.8	Access Success

#### 8.3.8.1	General

The purpose of the Access Success procedure is to enable the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover or conditional PSCell addition or conditional PSCell change. The procedure uses UE-associated signalling.

#### 8.3.8.2	Successful Operation

Figure 8.3.8.2-1: Access Success procedure. Successful operation.

The gNB-DU initiates the procedure by sending a ACCESS SUCCESS message.

Upon reception of the ACCESS SUCCESS message, the gNB-CU shall consider that the UE successfully accessed the cell indicated by the included NR CGI IE in this gNB-DU and consider all the other CHO preparations or conditional PSCell addition or conditional PSCell change preparations accepted for this UE under the same UE-associated signaling connection in this gNB-DU as cancelled.

Interaction with other procedure:

The gNB-CU may initiate UE Context Release procedure toward the other signalling connections or other candidate gNB-DUs for this UE, if any.

#### 8.3.8.3	Abnormal Conditions

If the ACCESS SUCCESS message refers to a context that does not exist, the gNB-CU shall ignore the message.

## 8.4	RRC Message Transfer procedures

### 8.4.1	Initial UL RRC Message Transfer

#### 8.4.1.1	General

The purpose of the Initial UL RRC Message Transfer procedure is to transfer the initial RRC message to the gNB-CU. The procedure uses non-UE-associated signaling.

#### 8.4.1.2	Successful operation

Figure 8.4.1.2-1: Initial UL RRC Message Transfer procedure.

The gNB-DU initiates the procedure by sending an INITIAL UL RRC MESSAGE TRANSFER. The establishment of the UE-associated logical F1-connection shall be initiated as part of the procedure.

If neither the DU to CU RRC Container IE nor the Sidelink Configuration Container IE in the Sidelink Relay Configuration IE is included in the INITIAL UL RRC MESSAGE TRANSFER, the gNB-CU should reject the UE under the assumption that the gNB-DU is not able to serve such UE. If the gNB-DU is able to serve the UE, the gNB-DU shall include the DU to CU RRC Container IE or the Sidelink Configuration Container IE in the Sidelink Relay Configuration IE and the gNB-CU shall configure the UE as specified in TS 38.331 [8]. The gNB-DU shall not include the ReconfigurationWithSync field in the CellGroupConfig IE as defined in TS 38.331 [8] of the DU to CU RRC Container IE.

If the SUL Access Indication IE is included in the INITIAL UL RRC MESSAGE TRANSFER, the gNB-CU shall consider that the UE has performed access on SUL carrier.

If the RAN UE ID IE is contained in the INITIAL UL RRC MESSAGE TRANSFER message, the gNB-CU shall, if supported, store it and use it as specified in TS 38.401 [4].

If the RRC-Container-RRCSetupComplete IE is included in the INITIAL UL RRC MESSAGE TRANSFER, the gNB-CU shall take it into account as specified in TS 38.401 [4].

If the NR RedCap UE Indication IE is included in the INITIAL UL RRC MESSAGE TRANSFER message, the gNB-CU shall, if supported, consider that the accessing UE is a RedCap UE.

If the SDT Information IE is included in the INITIAL UL RRC MESSAGE TRANSFER, the gNB-CU shall, if supported, consider that the UE is accessing for SDT as defined in TS 38.300 [6], and may use the information contained in the SDT Assistant Information IE, if any, for context retrieval.

If the Sidelink Relay Configuration IE is included in the INITIAL UL RRC MESSAGE TRANSFER, the gNB-CU shall, if supported, consider that the UE is a NR ProSe Layer-2 U2N Remote UE identified by the Remote UE Local ID IE, and it is connected to the U2N Relay UE indicated by the gNB-DU UE F1AP ID of Relay UE IE.

#### 8.4.1.3	Abnormal Conditions

Not applicable.

### 8.4.2	DL RRC Message Transfer

#### 8.4.2.1	General

The purpose of the DL RRC Message Transfer procedure is to transfer an RRC message The procedure uses UE-associated signalling.

#### 8.4.2.2	Successful operation

Figure 8.4.2.2-1: DL RRC Message Transfer procedure

The gNB-CU initiates the procedure by sending a DL RRC MESSAGE TRANSFER message. If a UE-associated logical F1-connection exists, the DL RRC MESSAGE TRANSFER message shall contain the gNB-DU UE F1AP ID IE, which should be used by gNB-DU to lookup the stored UE context.If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established at reception of the DL RRC MESSAGE TRANSFER message.

If the Index to RAT/Frequency Selection Priority IE is included in the DL RRC MESSAGE TRANSFER, the gNB-DU may use it for RRM purposes. If the Additional RRM Policy Index IE is included in the DL RRC MESSAGE TRANSFER, the gNB-DU may use it for RRM purposes.

The DL RRC MESSAGE TRANSFER message shall include, if available, the old gNB-DU UE F1AP ID IE so that the gNB-DU can retrieve the existing UE context in RRC connection reestablishment procedure, as defined in TS 38.401 [4].

The DL RRC MESSAGE TRANSFER message shall include, if SRB duplication is activated, the Execute Duplication IE, so that the gNB-DU can perform CA based duplication for the SRB.

If the gNB-DU identifies the UE-associated logical F1-connection by the gNB-DU UE F1AP ID IE in the DL RRC MESSAGE TRANSFER message and the old gNB-DU UE F1AP ID IE is included, it shall release the old gNB-DU UE F1AP ID and the related configurations associated with the old gNB-DU UE F1AP ID.

If the UE Context not retrievable IE set to "true" is included in the DL RRC MESSAGE TRANSFER, the DL RRC MESSAGE TRANSFER may contain the Redirected RRC message IE and use it as specified in TS 38.401 [4].

If the UE Context not retrievable IE set to "true" is included in the DL RRC MESSAGE TRANSFER, the DL RRC MESSAGE TRANSFER may contain the PLMN Assistance Info for Network Sharing IE, if available at the gNB-CU and may use it as specified in TS 38.401 [4].

If the DL RRC MESSAGE TRANSFER message contains the New gNB-CU UE F1AP ID IE, the gNB-DU shall, if supported, replace the value received in the gNB-CU UE F1AP ID IE by the value of the New gNB-CU UE F1AP ID and use it for further signalling.

If the DL RRC MESSAGE TRANSFER contains the SRB Mapping Info IE, the gNB-DU shall, if supported, use it for the Remote UE’s SRB0 or SRB1 transfer.

Interactions with UE Context Release Request procedure:

If the UE Context not retrievable IE set to "true" is included in the DL RRC MESSAGE TRANSFER, the gNB-DU may trigger the UE Context Release Request procedure, as specified in TS 38.401 [4].

#### 8.4.2.3	Abnormal Conditions

Not applicable.

### 8.4.3	UL RRC Message Transfer

#### 8.4.3.1	General

The purpose of the UL RRC Message Transfer procedure is to transfer an RRC message as an UL PDCP-PDU to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.4.3.2	Successful operation

Figure 8.4.3.2-1: UL RRC Message Transfer procedure

The gNB-DU initiates the procedure by sending a UL RRC MESSAGE TRANSFER message. When the gNB-DU has received from the radio interface an RRC message to which a UE-associated logical F1-connection for the UE exists, the gNB-DU shall send the UL RRC MESSAGE TRANSFER message to the gNB-CU including the RRC message as a RRC-Container IE.

If the Selected PLMN ID IE is contained in the UL RRC MESSAGE TRANSFER message, the gNB-CU may use it as specified in TS 38.401 [4].

If the UL RRC MESSAGE TRANSFER message contains the New gNB-DU UE F1AP ID IE, the gNB-CU shall, if supported, replace the value received in the gNB-DU UE F1AP ID IE by the value of the New gNB-DU UE F1AP ID and use it for further signalling.

#### 8.4.3.3	Abnormal Conditions

Not applicable.

### 8.4.4	RRC Delivery Report

#### 8.4.4.1	General

The purpose of the RRC Delivery Report procedure is to transfer to the gNB-CU information about successful delivery of DL PDCP-PDUs including RRC messages. The procedure uses UE-associated signalling.

#### 8.4.4.2	Successful operation

gNB

-

DU

RRC DELIVERY REPORT

gNB

-

CU

Figure 8.4.4.2-1: RRC Delivery Report procedure.

The gNB-DU initiates the procedure by sending an RRC DELIVERY REPORT message. When the gNB-DU has successfully delivered an RRC message to the UE for which the gNB-CU has requested a delivery report, the gNB-DU shall send the RRC DELIVERY REPORT message to the gNB-CU containng the RRC Delivery Status IE and the SRB ID IE.

#### 8.4.4.3	Abnormal Conditions

Not applicable.

## 8.5	Warning Message Transmission Procedures

### 8.5.1	Write-Replace Warning

#### 8.5.1.1	General

The purpose of Write-Replace Warning procedure is to start or overwrite the broadcasting of warning messages. The procedure uses non UE-associated signalling.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2-1: Write-Replace Warning procedure: successful operation

The gNB-CU initiates the procedure by sending a WRITE-REPLACE WARNING REQUEST message to the gNB-DU.

Upon receipt of the WRITE-REPLACE WARNING REQUEST message, the gNB-DU shall prioritise its resources to process the warning message.

The gNB-DU acknowledges the WRITE-REPLACE WARNING REQUEST message by sending a WRITE-REPLACE WARNING RESPONSE message to the gNB-CU.

Upon receipt of the WRITE-REPLACE WARNING REQUEST message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the WRITE-REPLACE WARNING RESPONSE message for UEs that are unable to receive system information from broadcast.

If Dedicated SI Delivery Needed UE List IE is contained in the WRITE-REPLACE WARNING RESPONSE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

Upon reception of the Notification Information IE in the PWS System Information IE in the WRITE-REPLACE WARNING REQUEST message, the gNB-DU shall use this information to avoid that duplications trigger new broadcast or replace existing broadcast.

If the gNB-DU receives a WRITE-REPLACE WARNING REQUEST message with the Notification Information IE in the PWS System Information IE which are different from those of ongoing broadcast warning messages, and if the SIB Type IE is set to "8", the gNB-DU shall broadcast the received warning message concurrently with other ongoing messages.

If the gNB-DU receives a WRITE-REPLACE WARNING REQUEST message with the Notification Information IE in the PWS System Information IE which are different from those of ongoing broadcast warning messages, and if the SIB Type IE is set to the value other than ‘8’ , the gNB-DU shall use the newly received one to replace the ongoing broadcast warning message with the same value of SIB Type IE.

If the SIB Type IE in the PWS System Information IE in the WRITE-REPLACE WARNING REQUEST message is set to "8" and if a value "0" is received in the Number of Broadcast Requested IE and if the Repetition Period IE is different from "0", the gNB-DU shall broadcast the received warning message indefinitely.

If Additional SIB Message List IE is included in PWS System Information IE, the gNB-DU shall store all SIB message(s) in PWS System Information IE, and consider that the first segment of public warning message is included in SIB message IE, and the remaining segments are listed in Additional SIB Message List IE in segmentation sequence order.

#### 8.5.1.3	Unsuccessful Operation

Not applicable.

#### 8.5.1.4	Abnormal Conditions

If the gNB-DU receives a WRITE-REPLACE WARNING REQUEST message which does not include the Notification Information IE in the PWS System Information IE, the gNB-DU shall consider it as a logical error.

### 8.5.2	PWS Cancel

#### 8.5.2.1	General

The purpose of the PWS Cancel procedure is to cancel an already ongoing broadcast of a warning message. The procedure uses non UE-associated signalling.

#### 8.5.2.2	Successful Operation

Figure 8.5.2.2-1: PWS Cancel procedure: successful operation

The gNB-CU initiates the procedure by sending a PWS CANCEL REQUEST message to the gNB-DU.

The gNB-DU shall acknowledge the PWS CANCEL REQUEST message by sending the PWS CANCEL RESPONSE message.

If the Cancel-All Warning Messages Indicator IE is present in the PWS CANCEL REQUEST message, then the gNB-DU shall stop broadcasting and discard all warning messages for the area as indicated in the Cell Broadcast To Be Cancelled List IE or in all the cells of the gNB-DU if the Cell Broadcast To Be Cancelled List IE is not included. The gNB-DU shall acknowledge the PWS CANCEL REQUEST message by sending the PWS CANCEL RESPONSE message, and shall, if there is area to report where an ongoing broadcast was stopped successfully, include the Cell Broadcast Cancelled List IE with the Number of Broadcasts IE set to 0.

If the Cell Broadcast To Be Cancelled List IE is not included in the PWS CANCEL REQUEST message, the gNB-DU shall stop broadcasting and discard the warning message identified by the Message Identifier IE and the Serial Number IE in the Notification Information IE in all of the cells in the gNB-DU.

If the Notification Information IE is included in the PWS CANCEL REQUEST, the gNB-DU shall cancel broadcast of the public warning message identified by the Notification Information IE.

If an area included in the Cell Broadcast To Be Cancelled List IE in the PWS CANCEL REQUEST message does not appear in the Cell Broadcast Cancelled List IE in the PWS CANCEL RESPONSE, the gNB-CU shall consider that the gNB-DU had no ongoing broadcast to stop for the public warning message identified, if present, by the Notification Information IE in that area.

If the Cell Broadcast Cancelled List IE is not included in the PWS CANCEL RESPONSE message, the gNB-CU shall consider that the gNB-DU had no ongoing broadcast to stop for the public warning message identified, if present, by the Notification Information IE.

#### 8.5.2.3	Unsuccessful Operation

If the gNB-DU receives a PWS CANCEL REQUEST message which contains neither the Cancel-all Warning Messages Indicator IE nor the Notification Information IE, the gNB-DU shall consider it as a logical error.

#### 8.5.2.4	Abnormal Conditions

Not applicable.

### 8.5.3	PWS Restart Indication

#### 8.5.3.1	General

The purpose of PWS Restart Indication procedure is to inform the gNB-CU that PWS information for some or all cells of the gNB-DU are available for reloading from the CBC if needed. The procedure uses non UE-associated signalling.

#### 8.5.3.2	Successful Operation

Figure 8.5.3.2-1: PWS restart indication

The gNB-DU initiates the procedure by sending a PWS RESTART INDICATION message to the gNB-CU.

#### 8.5.3.3	Abnormal Conditions

Not applicable.

### 8.5.4	PWS Failure Indication

#### 8.5.4.1	General

The purpose of the PWS Failure Indication procedure is to inform the gNB-CU that ongoing PWS operation for one or more cells of the gNB-DU has failed. The procedure uses non UE-associated signalling.

#### 8.5.4.2	Successful Operation

Figure 8.5.4.2-1: PWS failure indication

The gNB-DU initiates the procedure by sending a PWS FAILURE INDICATION message to the gNB-CU.

#### 8.5.4.3	Abnormal Conditions

Not applicable.

## 8.6	System Information Procedures

### 8.6.1	System Information Delivery

#### 8.6.1.1	General

The purpose of the System Information Delivery procedure is to command the gNB-DU to broadcast the requested one or several SystemInformation messages including the Other SI as requested by the gNB-CU. The procedure uses non-UE associated signalling.

#### 8.6.1.2	Successful Operation

Figure 8.6.1.2-1: System Information Delivery procedure. Successful operation.

The gNB-CU initiates the procedure by sending a SYSTEM INFORMATION DELIVERY COMMAND message to the gNB-DU.

Upon reception of the SYSTEM INFORMATION DELIVERY COMMAND message, the gNB-DU shall broadcast the requested one or several SystemInformation messages, including the Other SI, indicated by the SIType List IE, and if the UE corresponding to the confirmed UE ID IE is not in RRC connected state, delete the UE context, if any.

Interactions with gNB-DU Configuration Update procedure:

Upon reception of SYSTEM INFORMATION DELIVERY COMMAND message, the gNB-DU Configuration Update procedure may be performed , and as part of such procedure the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in GNB-DU CONFIGURATION UPDATE message for UEs that are unable to receive system information from broadcast.

#### 8.6.1.3	Abnormal Conditions

Not applicable.

## 8.7	Paging procedures

### 8.7.1	Paging

#### 8.7.1.1	General

The purpose of the Paging procedure is used to provide the paging information to enable the gNB-DU to page a UE. The procedure uses non-UE associated signalling.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: Paging procedure. Successful operation.

The gNB-CU initiates the procedure by sending a PAGING message.

The Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it to determine the final paging cycle for the UE.

The Paging Priority IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 23.501 [21].

At the reception of the PAGING message, the gNB-DU shall perform paging of the UE in cells which belong to cells as indicated in the Paging Cell List IE.

The Paging Origin IE may be included in the PAGING message, and if present the gNB-DU shall transfer it to the UE.

The RAN UE Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The CN UE Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The NR Paging eDRX Information IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The NR Paging eDRX Information for RRC INACTIVE IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The Paging Cause IE may be included in the PAGING message. If present the gNB-DU shall, if supported, send it to UE according to TS 38.331 [8].

The PEIPS Assistance Information IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [6].

The RedCap Indication IE may be included in the UE Paging Capability IE in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging of RedCap UEs.

The Last Used Cell Indication IE may be included in the Paging Cell Item IEs IE of the PAGING message, and if present the gNB-DU shall, if supported, consider the cell identified by the NR CGI IE as the last used cell of the paged UE, and use it as specified in TS 38.331 [8].

The PEI Subgrouping Support Indication IE may be included in the Paging Cell Item IEs IE in the PAGING message, and if present the gNB-DU shall, if supported, consider that the cell identified by the NR CGI IE is supported by the UE to receive the paging early indication as described in TS 38.300 [6] and TS 38.304 [24].

The UE Paging Capability IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, take it into account when paging the UE.

The Extended UE Identity Index Value IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The Hashed UE Identity Index Value IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

#### 8.7.1.3	Abnormal Conditions

Not applicable.

## 8.8	Trace Procedures

### 8.8.1	Trace Start

#### 8.8.1.1	General

The purpose of the Trace Start procedure is to allow the gNB-CU to request the gNB-DU to initiate a trace session for a UE. The procedure uses UE-associated signalling.

#### 8.8.1.2	Successful Operation

Figure 8.8.1.2-1: Trace start procedure: Successful Operation.

The gNB-CU initiates the procedure by sending a TRACE START message. Upon reception of the TRACE START message, the gNB-DU shall initiate the requested trace session for the requested UE, as described in TS 32.422 [29]. In particular, the gNB-DU shall, if supported:

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT and Trace" initiate the requested trace session and MDT session as described in TS 32.422 [29];

-	if the Trace Activation IE includes the MDT Activation IE set to "Immediate MDT Only" initiate the requested MDT session as described in TS 32.422 [29] and the gNB-DU shall ignore Interfaces To Trace IE, and Trace Depth IE;

#### 8.8.1.3	Abnormal Conditions

Void.

### 8.8.2	Deactivate Trace

#### 8.8.2.1	General

The purpose of the Deactivate Trace procedure is to allow the gNB-CU to request the gNB-DU to stop the trace session for the indicated trace reference. The procedure uses UE-associated signalling.

#### 8.8.2.2	Successful Operation

Figure 8.8.2.2-1: Deactivate trace procedure: Successful Operation

The gNB-CU initiates the procedure by sending a DEACTIVATE TRACE message. Upon reception of the DEACTIVATE TRACE message, the gNB-DU shall stop the trace session for the indicated trace reference contained in the Trace ID IE, as described in TS 32.422 [29].

#### 8.8.2.3	Abnormal Conditions

Void.

### 8.8.3	Cell Traffic Trace

#### 8.8.3.1	General

The purpose of the Cell Traffic Trace procedure is to send the allocated Trace Recording Session Reference and the Trace Reference to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.8.3.2	Successful Operation

Figure 8.8.3.2-1: Cell Traffic Trace procedure. Successful operation.

The procedure is initiated with a CELL TRAFFIC TRACE message sent from the gNB-DU to the gNB-CU.

If the Privacy Indicator IE is included in the message, the gNB-CU shall store the information so that it can be transferred towards the AMF.

#### 8.8.3.3	Abnormal Conditions

Void.

## 8.9	Radio Information Transfer procedures

### 8.9.1	DU-CU Radio Information Transfer

#### 8.9.1.1	General

The purpose of the DU-CU Radio Information Transfer procedure is to transfer radio-related information from the gNB-DU to the gNB-CU. The procedure uses non-UE-associated signalling.

#### 8.9.1.2	Successful operation

Figure 8.9.1.2-1: DU-CU Radio Information Transfer procedure.

The gNB-DU initiates the procedure by sending the DU-CU RADIO INFORMATION TRANSFER message to the gNB-CU.

The gNB-CU considers that the RIM-RS Detection Status IE indicates the RIM-RS detection status of the cell identified by Aggressor Cell ID IE.

#### 8.9.1.3	Abnormal Conditions

Not applicable.

### 8.9.2	CU-DU Radio Information Transfer

#### 8.9.2.1	General

The purpose of the CU-DU Radio Information Transfer procedure is to transfer radio-related information from the gNB-CU to the gNB-DU. The procedure uses non-UE-associated signalling.

#### 8.9.2.2	Successful operation

Figure 8.9.2.2-1: CU-DU Radio Information Transfer procedure.

The gNB-CU initiates the procedure by sending the CU-DU RADIO INFORMATION TRANSFER message to the gNB-DU. The gNB-DU considers that the RIM-RS Detection Status IE indicates the detection status of RIM-RS associated with Victim gNB Set ID IE.

#### 8.9.2.3	Abnormal Conditions

Not applicable.

## 8.10	IAB Procedures

### 8.10.0	General

In this version of the specification, the IAB procedures are used to configure IAB-donor-DU or IAB-DU.

NOTE: 	The IAB procedures are applicable for IAB-nodes and IAB-donor-DU, where the term "gNB-DU" applies to IAB-DU and IAB-donor-DU, and the term "gNB-CU" applies to IAB-donor-CU, unless otherwise specified.

### 8.10.1	BAP Mapping Configuration

#### 8.10.1.1	General

The BAP Mapping Configuration Procedure is initiated by the gNB-CU to configure the DL/UL routing information and/or traffic mapping information needed for the gNB-DU. The procedure uses non-UE associated signalling.

NOTE:	Implementation shall ensure the avoidance of potential race conditions, i.e. it shall ensure that conflicting traffic mapping configurations are not concurrently performed using the non-UE-associated BAP Mapping Configuration procedure and the UE-associated UE Context Management procedures.

#### 8.10.1.2	Successful Operation

Figure 8.10.1.2-1: BAP Mapping Configuration procedure: Successful Operation

The gNB-CU initiates the procedure by sending BAP MAPPING CONFIGURATION message to the gNB-DU. The gNB-DU replies to the gNB-CU with BAP MAPPING CONFIGURATION ACKNOWLEDGE.

If BH Routing Information Added List IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, store the BH routing information from this IE and use it for DL/UL traffic forwarding as specified in TS 38.340 [30]. If BH Routing Information Added List IE contains information for an existing BAP Routing ID, the gNB-DU shall, if supported, replace the previously stored routing information for this BAP Routing ID with the corresponding information in the BH Routing Information Added List IE.

If BH Routing Information Removed List IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, remove the BH routing information according to such IE.

If the Traffic Mapping Information IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, process the Traffic Mapping Information IE as follows:

-	if the IP to layer2 Traffic Mapping Info IE is included, the gNB-DU shall store the mapping information contained in the IP to layer2 Traffic Mapping Info To Add IE, if present, and remove the previously stored mapping information as indicated by the IP to layer2 Traffic Mapping Info To Remove IE, if present. The gNB-DU shall use the mapping information stored for the mapping of IP traffic to layer 2, as specified in TS 38.340 [30].

-	if the BAP layer BH RLC channel Mapping Info IE is included, the gNB-DU shall store the mapping information contained in the BAP layer BH RLC channel Mapping Info To Add IE, if present, and remove the previously stored mapping information as indicated by the BAP layer BH RLC channel Mapping Info To Remove IE, if present. The gNB-DU shall use the mapping information stored when forwarding traffic on BAP sublayer, as specified in TS 38.340 [30].

If the Buffer Size Threshold IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, use it to determine the DL congestion based on the flow control feedback from child IAB-nodes as specified in TS 38.340 [30].

If BAP Header Rewriting Added List IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, store the BAP header rewriting configuration from this IE, and use it as specified in TS 38.340 [30]. If BAP Header Rewriting Added List IE contains information for an existing ingress BAP Routing ID, the gNB-DU shall, if supported, replace the previously stored BAP header rewriting configuration for this ingress BAP Routing ID with the corresponding information in the BAP Header Rewriting Added List IE.

If BAP Header Rewriting Removed List IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, remove the BAP header rewriting configuration according to such IE.

If the Re-routing Enable Indicator IE is included in the BAP MAPPING CONFIGURATION message, and the value is set as “false”, the gNB-DU shall, if supported, disable the inter-donor-DU re-routing. If the Re-routing Enable Indicator IE is included in the BAP MAPPING CONFIGURATION message, and the value is set as “true”, the gNB-DU shall, if supported, enable the inter-donor-DU re-routing, as specified in TS 38.340 [30].

#### 8.10.1.A	Unsuccessful Operation

Figure 8.10.1.3-1: BAP Mapping Configuration procedure: Unsuccessful Operation

If the gNB-DU cannot accept the configuration, it shall respond with a BAP MAPPING CONFIGURATION FAILURE and appropriate cause value.

If the BAP MAPPING CONFIGURATION FAILURE message includes the Time To Wait IE, the gNB-CU shall wait at least for the indicated time before reinitiating the BAP MAPPING CONFIGURATION message towards the same gNB-DU.

#### 8.10.1.3	Abnormal Conditions

Not applicable.

### 8.10.2	gNB-DU Resource Configuration

#### 8.10.2.1	General

The gNB-DU Resource Configuration procedure is initiated by the gNB-CU in order to configure the resource usage for a gNB-DU. The procedure uses non-UE associated signalling.

In this version of the specification, this procedure is used to configure IAB resources.

#### 8.10.2.2	Successful Operation

Figure 8.10.2.2-1: gNB-DU Resource Configuration procedure: Successful Operation

The gNB-CU initiates the procedure by sending the GNB-DU RESOURCE CONFIGURATION message to gNB-DU. The gNB-DU replies to the gNB-CU with the GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE message.

For each cell in the Activated Cells to Be Updated List IE of the GNB-DU RESOURCE CONFIGURATION message, the gNB-DU shall store the resource configuration contained in the IAB-DU Cell Resource Configuration-Mode-Info IE and use it when performing scheduling in compliance with TS 38.213 [31].

If the Child-Node List IE is included in the GNB-DU RESOURCE CONFIGURATION message, for each child-node indicated by the gNB-CU UE F1AP ID IE and gNB-DU UE F1AP ID IE, and for each cell served by this child node indicated by the NR CGI IE in the Child-Node Cells List IE, the gNB-DU shall store the received information and use this information for scheduling, in compliance with TS 38.213 [31], clause 14.

If the Neighbour-Node Cells List IE is included in the GNB-DU RESOURCE CONFIGURATION message, for each neighbour-node cell indicated by the NR CGI IE in the Neighbour-Node Cells List IE, the gNB-DU shall store the received information and use this information for cross-link interference management and/or semi-static resource coordination. If the Peer Parent-Node Indicator IE is included in the GNB-DU RESOURCE CONFIGURATION message and the value is set as “true”, the gNB-DU shall, consider the cell indicated by the NR CGI IE is served by the peer parent node of the IAB-node indicated by the gNB-CU UE F1AP ID IE and the gNB-DU UE F1AP ID IE.

If the Serving Cells List IE is included in the GNB-DU RESOURCE CONFIGURATION message, the gNB-DU shall store the received information and use this information for scheduling, in compliance with TS 38.213 [31], clause 14.

#### 8.10.2.B	Unsuccessful Operation

Figure 8.10.2.3-1: gNB-DU Resource Configuration procedure: Unsuccessful Operation

If the gNB-DU cannot accept the configuration, it shall respond with a GNB-DU RESOURCE CONFIGURATION FAILURE and appropriate cause value.

If the GNB-DU RESOURCE CONFIGURATION FAILURE message includes the Time To Wait IE, the gNB-CU shall wait at least for the indicated time before reinitiating the GNB-DU RESOURCE CONFIGURATION message towards the same gNB-DU.

#### 8.10.2.3	Abnormal Conditions

Not applicable.

### 8.10.3	IAB TNL Address Allocation

#### 8.10.3.1	General

The purpose of the IAB TNL Address Allocation procedure is to allocate TNL addresses to be used by the IAB-node(s). This procedure uses non-UE associated signalling.

NOTE:	This procedure is applicable for IAB-donor-DU, where the term "gNB-DU" applies to IAB-donor-DU, and the term "gNB-CU" applies to IAB-donor-CU.

#### 8.10.3.2	Successful Operation

Figure 8.10.3.2-1: IAB TNL Address Allocation procedure: Successful Operation

The gNB-CU initiates the procedure by sending the IAB TNL ADDRESS REQUEST message to the gNB-DU.

If the IAB TNL ADDRESS REQUEST message contains the IAB IPv4 Addresses Requested IE, the gNB-DU shall allocate the individual TNL address(es) accordingly and include these IPv4 address(es) in the IAB TNL ADDRESS RESPONSE message.

If the IAB TNL ADDRESS REQUEST message contains the IAB IPv6 Request Type IE, the gNB-DU shall allocate the individual IPv6 address(es) or IPv6 address prefix(es) accordingly and include these IPv6 address(es) or IPv6 address prefix(es) in the IAB TNL ADDRESS RESPONSE message.

If the IAB TNL ADDRESS REQUEST message contains the IAB TNL Addresses To Remove List IE, the gNB-DU shall consider that the TNL address(es) and/or TNL address prefix(es) therein are no longer used by the IAB-node(s). In addition, if the IAB TNL ADDRESS REQUEST message only contains the IAB TNL Addresses to Remove List IE, the gNB-CU shall ignore the IAB Allocated TNL Address List IE in the IAB TNL ADDRESS RESPONSE message.

If the IAB TNL ADDRESS RESPONSE message contains the IAB TNL Address Usage IE in the IAB Allocated TNL Address Item IE, the gNB-CU shall consider the indicated TNL address usage when allocating a TNL address to an IAB-node. Otherwise, the gNB-CU shall consider that the TNL address can be used for all traffic when allocating the TNL address to an IAB-node.

If the IAB TNL Address Exception IE is included in the IAB TNL ADDRESS REQUEST message and the gNB-DU is an IAB-donor-DU, the gNB-DU shall, if supported, consider the IP address(es) therein as exempt from TNL address filtering, and forward the packets with the address(es) indicated by this IE, as specified in TS 38.401 [4].

#### 8.10.3.C	Unsuccessful Operation

Figure 8.10.3.3-1: IAB TNL Address Allocation procedure: Unsuccessful Operation

If the gNB-DU cannot accept the request, it shall respond with an IAB TNL ADDRESS FAILURE and appropriate cause value.

If the IAB TNL ADDRESS FAILURE message includes the Time To Wait IE, the gNB-CU shall wait at least for the indicated time before reinitiating the IAB TNL ADDRESS REQUEST message towards the same gNB-DU.

#### 8.10.3.3	Abnormal Conditions

Not applicable.

### 8.10.4	IAB UP Configuration Update

#### 8.10.4.1	General

The purpose of the IAB UP Configuration Update procedure is to update the UP parameters including UL mapping configuration and the UL/DL UP TNL information between IAB-donor-CU and IAB-node. This procedure uses non-UE associated signalling.

NOTE:	This procedure is applicable for IAB-nodes, where the term "gNB-DU" applies to IAB-DU, and the term "gNB-CU" applies to IAB-donor-CU.

NOTE:	Implementation shall ensure the avoidance of potential race conditions, i.e. it shall ensure that the update of UP configuration (e.g. the UL/DL UP TNL information, UL mapping information) is not concurrently performed using the non-UE-associated IAB UP Configuration Update procedure and the UE-associated procedures for UE Context Management.

#### 8.10.4.2	Successful Operation

Figure 8.10.4.2-1: IAB UP Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending the IAB UP CONFIGURATION UPDATE REQUEST message to the gNB-DU. The gNB-DU replies to the gNB-CU with the IAB UP CONFIGURATION UPDATE RESPONSE message.

If the UL UP TNL Information to Update List IE is included in the IAB UP CONFIGURATION UPDATE REQUEST message, the gNB-DU shall perform the mapping according to the new received BH Information IE for each F1-U GTP tunnel indicated by the UL UP TNL Information IE. If the New UL UP TNL Information IE is included in UL UP TNL Information to Update List IE, the gNB-DU shall use it to replace the information of UL F1-U GTP tunnel indicated by the UL UP TNL Information IE.

If the UL UP TNL Address to Update List IE is included in the IAB UP CONFIGURATION UPDATE REQUEST message, the gNB-DU shall replace the old TNL address with the new TNL address for all the maintained UL F1-U GTP tunnels corresponding to the old TNL address.

If the DL UP TNL Address to Update List IE is included in the IAB UP CONFIGURATION UPDATE RESPONSE message, the gNB-CU shall replace the old TNL address with the new TNL address for all the maintained DL F1-U GTP tunnels corresponding to the old TNL address.

#### 8.10.4.3	Unsuccessful Operation

Figure 8.10.4.3-1: IAB UP Configuration Update procedure: Unsuccessful Operation

If the gNB-DU receives an IAB UP CONFIGURATION UPDATE REQUEST message and cannot perform any update accordingly, it shall consider the update procedure as failed and respond with an IAB UP CONFIGURATION UPDATE FAILURE message and an appropriate cause value.

If the IAB UP CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU shall wait at least for the indicated time before reinitiating the IAB UP CONFIGURATION UPDATE REQUEST message towards the same gNB-DU.

#### 8.10.4.4	Abnormal Conditions

Not applicable.

## 8.11	Self Optimisation Support procedures

### 8.11.1	Access and Mobility Indication

#### 8.11.1.1	General

This procedure is initiated by gNB-CU to send the Access and Mobility related Information to gNB-DU.

The procedure uses non-UE-associated signalling.

#### 8.11.1.2	Successful Operation

Figure 8.11.1.2-1: Access and Mobility Indication procedure. Successful operation

The Access and Mobility Indication procedure is initiated by ACCESS AND MOBILITY INDICATION message sent from gNB-CU to gNB-DU.

If the ACCESS AND MOBILITY INDICATION message contains the RACH Report Information List IE the gNB-DU shall take it into account for optimisation of RACH access procedures.

If the ACCESS AND MOBILITY INDICATION message contains the RLF Report Information List IE the gNB-DU shall take it into account for optimisation of mobility parameters.

If the ACCESS AND MOBILITY INDICATION message contains the Successful HO Report Information List IE the gNB-DU may take it into account for optimisation of mobility parameters.

#### 8.11.1.3	Abnormal Conditions

Not applicable.

## 8.12	Reference Time Information Reporting procedures

### 8.12.1	Reference Time Information Reporting Control

#### 8.12.1.1	General

The purpose of the Reference Time Information Reporting Control procedure is to command the gNB-DU to send the requested accurate reference time information to the gNB-CU. The procedure uses non-UE associated signalling.

#### 8.12.1.2	Successful Operation

Figure 8.12.1.2-1: Reference Time Information Reporting Control

The gNB-CU initiates the procedure by sending REFERENCE TIME INFORMATION REPORTING CONTROL message to the gNB-DU. Upon reception of the REFERENCE TIME INFORMATION REPORTING CONTROL message, the gNB-DU shall, if supported, perform the requested reference time information reporting action.

The Report Type IE indicates to the gNB-DU whether:

-	to report on demand;

-	to report periodic, with a frequency as specified by the Report Periodicity IE;

-	to stop periodic reporting.

#### 8.12.1.3	Abnormal Conditions

Not applicable.

### 8.12.2	Reference Time Information Report

#### 8.12.2.1	General

The purpose of the Reference Time Information Report procedure is to report the accurate reference time information from the gNB-DU to the gNB-CU. The procedure uses non-UE associated signalling.

#### 8.12.2.2	Successful Operation

Figure 8.12.2-2-1: Reference Time Information Report

The gNB-DU initiates the procedure by sending a REFERENCE TIME INFORMATION REPORT message to the gNB-CU. The REFERENCE TIME INFORMATION REPORT message may be used as a response to the REFERENCE TIME INFORMATION REPORTING CONTROL message.

#### 8.12.2.3	Abnormal Conditions

Not applicable.

## 8.13	Positioning Procedures

### 8.13.1	Positioning Assistance Information Control

#### 8.13.1.1	General

The purpose of the Positioning Assistance Information Control procedure is to allow the gNB-CU to signal positioning assistance information to the gNB-DU for positioning assistance information broadcasting. The procedure uses non-UE-associated signalling.

#### 8.13.1.2	Successful Operation

Figure 8.13.1.2-1: Positioning Assistance Information Control procedure

The gNB-CU initiates the procedure by sending a POSITIONING ASSISTANCE INFORMATION CONTROL message.

If the Positioning Assistance Information IE is included in the POSITIONING ASSISTANCE INFORMATION CONTROL message, the gNB-DU shall, if supported, replace any previously stored positioning assistance information and use the received information to configure positioning assistance information broadcasting as specified in TS 38.455 [37].

If the Broadcast IE is included in the POSITIONING ASSISTANCE INFORMATION CONTROL message and set to "start", the gNB-DU may start broadcasting the positioning assistance information. If the Broadcast IE is included in the POSITIONING ASSISTANCE INFORMATION CONTROL message and set to "stop", the gNB-DU may stop broadcasting the positioning assistance information.

If the Positioning Broadcast Cells IE is included in the POSITIONING ASSISTANCE INFORMATION CONTROL message, the gNB-DU shall, if supported, consider that the received assistance information is applicable to the cells in this IE.

Interaction with the Positioning Assistance Information Feedback procedure:

If the Routing ID IE is included in the POSITIONING ASSISTANCE INFORMATION CONTROL message, the gNB-DU shall, if supported, store this information and include it in any future POSITIONING ASSISTANCE INFORMATION FEEDBACK messages associated to the requested positioning assistance information broadcasting.

#### 8.13.1.3	Abnormal Conditions

If the Broadcast IE is included in the POSITIONING ASSISTANCE INFORMATION CONTROL message and set to "start", and no positioning assistance information is available, the gNB-DU shall consider the procedure as failed.

If neither the Positioning Assistance Information IE nor the Broadcast IE are included in the POSITIONING ASSISTANCE INFORMATION CONTROL message, the gNB-DU shall consider the procedure as failed.

### 8.13.2	Positioning Assistance Information Feedback

#### 8.13.2.1	General

The purpose of the Positioning Assistance Information Feedback procedure is to allow the gNB-DU to give feedback to the gNB-CU on positioning assistance information broadcasting. The procedure uses non-UE-associated signalling.

#### 8.13.2.2	Successful Operation

Figure 8.13.2.2-1: Positioning Assistance Information Feedback procedure

The gNB-DU initiates the procedure by sending a POSITIONING ASSISTANCE INFORMATION FEEDBACK message. If the Positioning Assistance Information Failure List IE is included in the POSITIONING ASSISTANCE INFORMATION FEEDBACK message, the gNB-CU shall consider that positioning assistance information broadcasting could not be configured for the relevant information.

If the Positioning Broadcast Cells IE is included in the POSITIONING ASSISTANCE INFORMATION FEEDBACK message, the gNB-CU shall consider that the feedback provided is applicable to the cells in this IE.

If the Routing ID IE is included in the POSITIONING ASSISTANCE INFORMATION FEEDBACK message, the gNB-CU may use this information to identify the positioning assistance information broadcasting for which feedback is provided.

#### 8.13.2.3	Abnormal Conditions

Void.

### 8.13.3	Positioning Measurement

#### 8.13.3.1	General

The purpose of the Positioning Measurement procedure is to allow the gNB-CU to request one or more TRPs in the gNB-DU to perform and report positioning measurements. The procedure uses non-UE-associated signalling.

#### 8.13.3.2	Successful Operation

Figure 8.13.3.2-1: Positioning Measurement procedure: successful operation

The gNB-CU initiates the procedure by sending a POSITIONING MEASUREMENT REQUEST message to the gNB-DU, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The gNB-DU node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the gNB-DU shall reply with the POSITIONING MEASUREMENT RESPONSE message including the Positioning Measurement Response List IE.

If the Positioning Report Characteristics IE is set to "OnDemand", the gNB-DU shall return the corresponding measurement results in the Positioning Measurement Result List IE in the POSITIONING MEASUREMENT RESPONSE message, and the gNB-CU shall consider that this reporting has been terminated by the gNB-DU.

If the Measurement Beam Information Request IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU node shall, if supported, include the Measurement Beam Information IE in the Positioning Measurement Result IE of the POSITIONING MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the Measurement Result IE in the POSITIONING MEASUREMENT RESPONSE message, the gNB-CU may use it for further signalling. If the Measurement Quality IE includes the Zenith Quality IE, the gNB-CU may use it for further signalling.

If the System Frame Number IE and/or the Slot Number IE are included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Measurement Characteristics Request Indicator IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, take the requested measurement characteristics into account when configuring measurements, and include the requested information, if available, in the POSITIONING MEASUREMENT RESPONSE message.

If the Number of TRP Rx TEGs IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, use it to measure the same SRS resource with different TRP Rx TEGs for the indicated TRP, and report the corresponding UL-RTOA and/or gNB Rx-Tx time difference measurements.

If the Number of TRP RxTx TEGs IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, use it to measure the same SRS resource with different TRP RxTx TEGs with the same TRP Tx TEG for the indicated TRP, and report the corresponding gNB Rx-Tx time difference measurements.

If the Measurement Time Occasion IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU may take it into account as the number of SRS measurement time occasions for a measurement instance.

Interaction with the Positioning Measurement Report procedure:

If the Positioning Report Characteristics IE is set to "Periodic", the gNB-DU shall initiate the corresponding measurements, and it shall reply with the POSITIONING MEASUREMENT RESPONSE message without including any measurement results in the message. The gNB-DU shall then periodically initiate the Positioning Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, return the corresponding measurement results in the POSITIONING MEASUREMENT RESPONSE message within the indicated time.

If the Positioning Report Characteristics IE is set to "Periodic" and the Positioning Measurement Amount IE is included in the POSITIONING MEASUREMENT REQUEST message, the gNB-DU shall, if supported, take it into account for sending the POSITIONING MEASUREMENT REPORT message.

#### 8.13.3.3	Unsuccessful Operation

Figure 8.13.3.3-1: Positioning Measurement procedure: unsuccessful operation

If the gNB-DU is unable to configure any of the requested positioning measurements for any of the TRPs in the TRP Measurement Request List IE of the POSITIONING MEASUREMENT REQUEST message, it shall respond with a POSITIONING MEASUREMENT FAILURE message.

#### 8.13.3.4	Abnormal Conditions

If the gNB-DU receives a POSITIONING MEASUREMENT REQUEST message containing an LMF Measurement ID corresponding to an ongoing positioning measurement, it shall consider the procedure as failed and initiate local error handling.

### 8.13.4	Positioning Measurement Report

#### 8.13.4.1	General

The purpose of the Positioning Measurement Report procedure is for the gNB-DU to report positioning measurements to the gNB-CU. The procedure uses non-UE-associated signalling.

#### 8.13.4.2	Successful Operation

Figure 8.13.4.2-1: Positioning Measurement Report procedure: successful operation

The gNB-DU initiates the procedure by sending a POSITIONING MEASUREMENT REPORT message. The POSITIONING MEASUREMENT REPORT message contains the positioning measurement results according to the associated measurement configuration.

#### 8.13.4.3	Unsuccessful Operation

Not applicable.

#### 8.13.4.4	Abnormal Conditions

Not applicable.

### 8.13.5	Positioning Measurement Abort

#### 8.13.5.1	General

The purpose of the Positioning Measurement Abort procedure is to enable the gNB-CU to abort an on-going measurement. The procedure uses non-UE-associated signalling.

#### 8.13.5.2	Successful Operation

Figure 8.13.5.2-1: Positioning Measurement Abort procedure: successful operation

The gNB-CU initiates the procedure by generating a POSITIONING MEASUREMENT ABORT message. Upon receiving this message, the gNB-DU shall terminate the on-going measurement identified by the RAN Measurement ID IE and may release any resources previously allocated for the same measurement.

#### 8.13.5.3	Unsuccessful Operation

Not applicable.

#### 8.13.5.4	Abnormal Conditions

If the gNB-DU cannot identify the previously requested measurement to be aborted, it shall ignore the POSITIONING MEASUREMENT ABORT message.

### 8.13.6	Positioning Measurement Failure Indication

#### 8.13.6.1	General

The purpose of the Positioning Measurement Failure Indication procedure is for the gNB-DU to notify the gNB-CU that the positioning measurements previously requested with the Positioning Measurement procedure can no longer be reported. The procedure uses non-UE-associated signalling.

#### 8.13.6.2	Successful Operation

Figure 8.13.6.2-1: Positioning Measurement Failure Indication procedure: successful operation

The gNB-DU initiates the procedure by sending a POSITIONING MEASUREMENT FAILURE INDICATION message. Upon reception of the POSITIONING MEASUREMENT FAILURE INDICATION message, the gNB-CU shall consider that the indicated positioning measurements have been terminated by the gNB-DU.

#### 8.13.6.3	Unsuccessful Operation

Not applicable.

#### 8.13.6.4	Abnormal Conditions

Not applicable.

### 8.13.7	Positioning Measurement Update

#### 8.13.7.1	General

The purpose of the Positioning Measurement Update procedure is to modify one or more periodic positioning measurements performed by the gNB-DU. The procedure uses non-UE-associated signalling.

#### 8.13.7.2	Successful Operation

Figure 8.13.7.2-1: Positioning Measurement Update procedure: successful operation

The gNB-CU initiates the procedure by generating a POSITIONING MEASUREMENT UPDATE message. Upon receiving the message, the gNB-DU shall overwrite the previously received measurement configuration for the corresponding measurements.

If the Number of TRP Rx TEGs IE is included in the TRP Measurement Update List IE in the POSITIONING MEASUREMENT UPDATE message, the gNB-DU shall clear any previously stored information and store the newly received information.

If the Number of TRP RxTx TEGs IE is included in the TRP Measurement Update List IE in the POSITIONING MEASUREMENT UPDATE message, the gNB-DU shall clear any previously stored information and store the newly received information.

If the Measurement Characteristics Request Indicator IE is included in the POSITIONING MEASUREMENT UPDATE message, the gNB-DU shall clear any previously stored information and store the newly received information.

If the Measurement Time Occasion IE is included in the POSITIONING MEASUREMENT UPDATE message, the gNB-DU shall clear any previously stored information and store the newly received information.

#### 8.13.7.3	Unsuccessful Operation

Not applicable.

#### 8.13.7.4	Abnormal Conditions

If the gNB-DU cannot identify the given positioning measurements, it shall regard the procedure as failed and initiate local error handling.

### 8.13.8	TRP Information Exchange

#### 8.13.8.1	General

The purpose of the TRP Information Exchange procedure is to allow the gNB-CU to request the gNB-DU to provide detailed information for TRPs hosted by the gNB-DU. The procedure uses non-UE-associated signalling.

#### 8.13.8.2	Successful Operation

Figure 8.13.8.2-1: TRP Information Exchange procedure, successful operation

The gNB-CU initiates the procedure by sending a TRP INFORMATION REQUEST message. The gNB-DU responds with a TRP INFORMATION RESPONSE message that contains the requested TRP information.

If the TRP List IE is included in the TRP INFORMATION REQUEST message, the gNB-DU should include in the TRP INFORMATION RESPONSE message, the requested information for all TRPs included in the TRP List IE.

If the TRP List IE is not included in the TRP INFORMATION REQUEST message, the gNB-DU should include the requested information for all TRPs hosted by the gNB-DU in the TRP INFORMATION RESPONSE message.

If the PRS Muting IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the gNB-CU may use it for further signaling.

If the QCL Info IE is included in the PRS Configuration IE in the TRP INFORMATION RESPONSE message, the gNB-CU may use it for further signaling.

If the DL-PRS Resource Coordinates IE is included in the Geographical Coordinates IE in the TRP Information IE in the TRP INFORMATION RESPONSE message, the gNB-CU may use it for further signaling.

#### 8.13.8.3	Unsuccessful Operation

Figure 8.13.8.3-1: TRP Information Exchange procedure, unsuccessful operation

If the gNB-DU cannot provide any of the requested information, the gNB-DU shall respond with a TRP INFORMATION FAILURE message.

### 8.13.9	Positioning Information Exchange

#### 8.13.9.1	General

The Positioning Information Exchange procedure is initiated by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals and to retrieve the SRS configuration from the gNB-DU. The procedure uses UE-associated signalling.

#### 8.13.9.2	Successful Operation

Figure 8.13.9.2-1: Positioning Information Exchange procedure, successful operation

The gNB-CU initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the gNB-DU.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the gNB-DU may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message. If the SRS Positioning INACTIVE Query Indication IE is also included in the POSITIONING INFORMATION REQUEST message and set to ‘true’, the gNB-DU shall, if supported, include the SRS-PosRRC-InactiveConfig IE in the POSITIONING INFORMATION RESPONSE message.

If the Spatial Relation Information per SRS Resource IE and the Periodicity List IE are both included in the Requested SRS Transmission Characteristics IE, the gNB-DU shall consider that the Spatial Relation per SRS Resource Item IE and the Periodicity List Item IE have one-to-one mapping relation.

If the UE Reporting Information IE is included in the POSITIONING INFORMATION REQUEST message, the gNB-DU may take this information into account for allocating proper CG-SDT resources when positioning a UE.

Interaction with the UE Context Modification Required (gNB-DU initiated) procedure:

The UE Context Modification Required (gNB-DU initiated) procedure may be performed before the POSITIONING INFORMATION RESPONSE message.

#### 8.13.9.3	Unsuccessful Operation

Figure 8.13.9.3-1: Positioning Information Exchange procedure, unsuccessful operation

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message and the gNB-DU is unable to configure any SRS transmissions for the UE, the gNB-DU shall respond with a POSITIONING INFORMATION FAILURE message with an appropriate cause value.

If the gNB-DU is unable to provide any of the requested information, the gNB-DU shall respond with a POSITIONING INFORMATION FAILURE message with an appropriate cause value.

### 8.13.10	Positioning Activation

#### 8.13.10.1	General

The Positioning Activation procedure is initiated by the gNB-CU to request the gNB-DU to activate semi-persistent or trigger aperiodic UL SRS transmission by the UE. The procedure uses UE-associated signalling.

#### 8.13.10.2	Successful Operation

Figure 8.13.10.2-1: Positioning Activation procedure, successful operation

The gNB-CU initiates the procedure by sending a POSITIONING ACTIVATION REQUEST message to the gNB-DU.

For semi-persistent UL SRS, the POSITIONING ACTIVATION REQUEST message includes an indication of the UL SRS resource set to be activated, and may include the spatial relation for the semi-persistent UL SRS resource to be activated. For aperiodic UL SRS, if the SRS Resource Trigger IE is included in the POSITIONING ACTIVATION REQUEST message, the gNB-DU shall take the value of this IE into account when triggering aperiodic SRS transmission by the UE.

If the Activation Time IE is included in the POSITIONING ACTIVATION REQUEST message, the gNB-DU shall take the indicated value as the requested time for activation of the UE’s SRS transmission.

Following successful activation of UL SRS transmission in the UE, the gNB-DU shall respond with a POSITIONING ACTIVATION RESPONSE message. If the POSITIONING ACTIVATION RESPONSE message includes the System Frame Number and/or the Slot Number IEs, the gNB-CU shall consider that the respective information indicates the activation time of SRS transmission by the UE.

#### 8.13.10.3	Unsuccessful Operation

Figure 8.13.10.3-1: Positioning Activation procedure, unsuccessful operation

If the gNB-DU is unable to activate UL SRS transmission in the UE, it shall respond with a POSITIONING ACTIVATION FAILURE message.

If the gNB-DU is unable to trigger the aperiodic SRS transmission with the indicated SRS Resource Trigger IE, it shall respond with a POSITIONING ACTIVATION FAILURE message with an appropriate cause value

#### 8.13.10.4	Abnormal Conditions

Void.

### 8.13.11	Positioning Deactivation

#### 8.13.11.1	General

The Positioning Deactivation procedure is initiated by the gNB-CU to indicate to the gNB-DU node that UL SRS transmission should be deactivated in the UE. The procedure uses UE-associated signalling.

#### 8.13.11.2	Successful Operation

Figure 8.13.11.2-1: Positioning Deactivation procedure, successful operation

The gNB-CU initiates the procedure by sending a POSITIONING DEACTIVATION message to the gNB-DU, including an indication of the UL SRS resources to be deactivated.

#### 8.13.11.3	Unsuccessful Operation

Not Applicable.

#### 8.13.11.4	Abnormal Conditions

Void.

### 8.13.12	E-CID Measurement Initiation

#### 8.13.12.1	General

The purpose of E-CID Measurement Initiation procedure is to allow the gNB-CU to request the gNB-DU to report E-CID measurements used by LMF to compute the location of the UE. The procedure uses UE-associated signalling.

#### 8.13.12.2	Successful Operation

Figure 8.13.12.2-1: E-CID Measurement Initiation procedure, successful operation

The gNB-CU initiates the procedure by sending an E-CID MEASUREMENT INITIATION REQUEST message. If the gNB-DU is able to initiate the requested E-CID measurements, it shall reply with the E-CID MEASUREMENT INITIATION RESPONSE message.

If the E-CID Report Characteristics IE is set to "OnDemand", the gNB-DU shall return the result of the measurement in the E-CID MEASUREMENT INITIATION RESPONSE message including, if available, the Geographical Coordinates IE in the E-CID Measurement Result IE and the Cell Portion ID IE, and the gNB-CU shall consider that the E-CID measurements for the UE have been terminated by the gNB-DU. The Measured Results List IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT INITIATION RESPONSE message when measurement quantities other than "Default" have been requested.

Interaction with the E-CID Measurement Report procedure:

If the E-CID Report Characteristics IE is set to "Periodic", the gNB-DU shall initiate the requested measurements and shall reply with the E-CID MEASUREMENT INITIATION RESPONSE message without including either the E-CID Measurement Result IE or the Cell Portion ID IE in this message. The gNB-DU shall then periodically initiate the E-CID Measurement Report procedure for the measurements, with the requested reporting periodicity.

#### 8.13.12.3	Unsuccessful Operation

Figure 8.13.12.3-1: E-CID Measurement Initiation procedure, unsuccessful operation

If the gNB-DU is not able to initiate at least one of the requested E-CID measurements, the gNB-DU shall respond with an E-CID MEASUREMENT INITIATION FAILURE message.

### 8.13.13	E-CID Measurement Failure Indication

#### 8.13.13.1	General

The purpose of the E-CID Measurement Failure Indication procedure is for the gNB-DU to notify the gNB-CU that the E-CID measurements previously requested with the E-CID Measurement Initiation procedure can no longer be reported. The procedure uses UE-associated signalling.

#### 8.13.13.2	Successful Operation

Figure 8.13.13.2-1: E-CID Measurement Failure Indication, successful operation

The gNB-DU initiates the procedure by sending an E-CID MEASUREMENT FAILURE INDICATION message. Upon reception of the E-CID MEASUREMENT FAILURE INDICATION message, the gNB-CU shall consider that the E-CID measurements for the UE have been terminated by the gNB-DU.

#### 8.13.13.3	Unsuccessful Operation

Not applicable.

### 8.13.14	E-CID Measurement Report

#### 8.13.14.1	General

The purpose of E-CID Measurement Report procedure is for the gNB-DU to provide the E-CID measurements for the UE to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.13.14.2	Successful Operation

Figure 8.13.14.2-1: E-CID Measurement Report procedure, successful operation

The gNB-DU initiates the procedure by sending an E-CID MEASUREMENT REPORT message. The E-CID MEASUREMENT REPORT message contains the E-CID measurement results according to the measurement configuration in the respective E-CID MEASUREMENT INITIATION REQUEST message.

The Measured Results List IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT REPORT message when measurement quantities other than "Default" have been requested.

If available, the gNB-DU shall include the Geographical Coordinates IE in the E-CID Measurement Result IE in the E-CID MEASUREMENT REPORT message.

If available, the gNB-DU shall include the Cell Portion ID IE in the E-CID MEASUREMENT REPORT message.

#### 8.13.14.3	Unsuccessful Operation

Not applicable.

### 8.13.15	E-CID Measurement Termination

#### 8.13.15.1	General

The purpose of E-CID Measurement Termination procedure is to terminate periodical E-CID measurements for the UE performed by the gNB-DU. The procedure uses UE-associated signalling.

#### 8.13.15.2	Successful Operation

Figure 8.13.15.2-1: E-CID Measurement Termination procedure, successful operation

The gNB-CU initiates the procedure by generating an E-CID MEASUREMENT TERMINATION COMMAND message.

#### 8.13.15.3	Unsuccessful Operation

Not applicable.

### 8.13.16	Positioning Information Update

#### 8.13.16.1	General

The Positioning Information Update procedure is initiated by the gNB-DU to indicate to the gNB-CU that a change has occurred in the SRS configuration. The procedure uses UE-associated signalling.

#### 8.13.16.2	Successful Operation

Figure 8.13.16.2-1: Positioning Information Update procedure, successful operation

The gNB-DU initiates the procedure by sending a POSITIONING INFORMATION UPDATE message to the gNB-CU.

If the SRS Configuration IE is included in the POSITIONING INFORMATION UPDATE message, the gNB-CU shall consider this information as the updated SRS Configuration for the UE. If the SFN Initialisation Time IE is included in the POSITIONING INFORMATION UPDATE message, the gNB-CU shall consider this information as the SFN Initialisation Time associated to the SRS Configuration.

#### 8.13.16.3	Unsuccessful Operation

Not Applicable.

#### 8.13.16.4	Abnormal Conditions

Void.

### 8.13.17	PRS Configuration Exchange

#### 8.13.17.1	General

The PRS Configuration Exchange procedure is initiated by the gNB-CU to request the gNB-DU to configure or update (i.e., turn off) the PRS transmissions.

The procedure uses non-UE-associated signalling.

#### 8.13.17.2	Successful Operation

Figure 8.13.17.2-1: PRS Configuration Exchange procedure, successful operation

The gNB-CU initiates the procedure by sending a PRS CONFIGURATION REQUEST message to the gNB-DU.

If the PRS Configuration Request Type IE is set to “configure”, the gNB-DU should use the information in the Requested DL PRS Transmission Characteristics IE to configure DL-PRS transmission by the indicated TRP(s).

If the PRS Configuration Request Type IE is set to “off”, the gNB-DU should, if supported, use the information in the PRS Transmission Off Information IE to turn off the DL-PRS transmission for the indicated TRP(s), PRS Resource Set(s), or PRS Resource(s).

If DL-PRS transmission is successfully configured or updated for at least one of the TRPs, the gNB-DU shall respond with the PRS CONFIGURATION RESPONSE message.

#### 8.13.17.3	Unsuccessful Operation

Figure 8.13.17.3-1: PRS Configuration Exchange procedure, unsuccessful operation

If the gNB-DU cannot configure or update DL-PRS transmission for any of the TRPs in the PRS TRP List IE of the PRS CONFIGURATION REQUEST message, it shall respond with a PRS CONFIGURATION FAILURE message with an appropriate cause value.

#### 8.13.17.4	Abnormal Conditions

Void.

### 8.13.18	Measurement Preconfiguration

#### 8.13.18.1	General

The Measurement Preconfiguration procedure allows the gNB-CU to provide necessary information to the serving gNB-DU and request the gNB-DU to preconfigure measurement gap and/or PRS processing window of the UE.

#### 8.13.18.2	Successful Operation

Figure 8.13.18.2-1: Measurement Preconfiguration procedure, successful operation

The gNB-CU initiates the procedure by sending a MEASUREMENT PRECONFIGURATION REQUIRED message.

If the gNB-DU is able to configure measurement gap or PRS processing window, it shall reply with the MEASUREMENT PRECONFIGURATION CONFIRM message.

If the PosMeasGapPreConfigList IE is included in the MEASUREMENT PRECONFIGURATION CONFIRM message, the gNB-CU shall, if supported, take the preconfigured measurement gaps information into account.

#### 8.13.18.3	Unsuccessful Operation

Figure 8.13.18.3-1: Measurement Preconfiguration procedure, unsuccessful operation

If the gNB-DU cannot configure any of the measurement gap or PRS processing window, the gNB-DU shall respond with a MEASUREMENT PRECONFIGURATION REFUSE message.

### 8.13.19	Measurement Activation

#### 8.13.19.1	General

The Measurement Activation procedure is initiated by the gNB-CU to request the gNB-DU to activate or deactivate the preconfigured measurement gap or PRS processing window for the UE.

#### 8.13.19.2	Successful Operation

Figure 8.13.19.2-1: Measurement Activation procedure, successful operation

The gNB-CU initiates the procedure by sending a MEASUREMENT ACTIVATION message.

If the PRS Measurement Info List IE is included in the MEASUREMENT ACTIVATION message, the gNB-DU may take it into account when activating pre-configured measurement gap in the UE.

#### 8.13.19.3	Unsuccessful Operation

Not Applicable.

### 8.13.20	Positioning System Information Delivery

#### 8.13.20.1	General

The purpose of the Positioning System Information Delivery procedure is to command the gNB-DU to broadcast the requested one or several Positioning SI messages indicated by the gNB-CU. The procedure uses non-UE associated signalling.

#### 8.13.20.2	Successful Operation

Figure 8.13.20.2-1: Positioning System Information Delivery procedure. Successful operation.

The gNB-CU initiates the procedure by sending a POSITIONING SYSTEM INFORMATION DELIVERY COMMAND message to the gNB-DU.

Upon reception of the POSITIONING SYSTEM INFORMATION DELIVERY COMMAND message, the gNB-DU shall broadcast the requested one or several Positioning SI messages, indicated by the PosSITypeList IE, and if the UE corresponding to the Confirmed UE ID IE is not in the RRC connected state, delete the UE context, if any.

Interactions with gNB-DU Configuration Update procedure:

Upon reception of POSITIONING SYSTEM INFORMATION DELIVERY COMMAND message, the gNB-DU Configuration Update procedure may be performed, and as part of such procedure the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in GNB-DU CONFIGURATION UPDATE message for UEs that are unable to receive system information from broadcast.

#### 8.13.20.3	Abnormal Conditions

Not applicable.

## 8.14	NR MBS Procedures

### 8.14.1	Broadcast Context Setup

#### 8.14.1.1	General

The purpose of the Broadcast Context Setup procedure is to establish an MBS Session context for a broadcast session in the gNB-DU.

The procedure uses MBS-associated signalling.

#### 8.14.1.2	Successful Operation

Figure 8.14.1.2-1: Broadcast Context Setup procedure: Successful Operation

The gNB-CU initiates the procedure by sending BROADCAST CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the broadcast MBS Session context, it replies to the gNB-CU with BROADCAST CONTEXT SETUP RESPONSE.

If the MBS Service Area IE is included in the BROADCAST CONTEXT SETUP REQUEST message, the gNB-DU shall take this information into account for shared F1-U tunnel assignment.

The gNB-DU shall report to the gNB-CU, in the BROADCAST CONTEXT SETUP RESPONSE message, the result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Broadcast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Broadcast MRB Failed To Be Setup List IE;

If the Broadcast MRB Failed To Setup List IE is contained in the BROADCAST CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the Broadcast MRB(s) failed to be setup with an appropriate cause value for each Broadcast MRB failed to setup.

If

-	either the MBS Service Area IE was included in the BROADCAST CONTEXT SETUP REQUEST message,

-	or the the MBS Service Area IE was not included in the BROADCAST CONTEXT SETUP REQUEST message and the gNB-DU was not able to establish MBS Session Resources in all cells served by the gNB-DU,

the Broadcast Area Scope IE shall be included in the BROADCAST CONTEXT SETUP RESPONSE message to indicate the cells where MBS Session resources have been successfully established in the gNB-DU.

#### 8.14.1.3	Unsuccessful Operation

Figure 8.14.1.3-1: Broadcast Context Setup procedure: unsuccessful Operation

If the gNB-DU is not able to establish the requested MBS session context for all the MRBs in any of its cells it shall consider the procedure as failed and reply with the BROADCAST CONTEXT SETUP FAILURE message.

#### 8.14.1.4	Abnormal Conditions

Not applicable.

### 8.14.2	Broadcast Context Release

#### 8.14.2.1	General

The purpose of the Broadcast Context Release procedure is to enable the gNB-CU to order the release of an established MBS Session context for a broadcast session in the gNB-DU.

The procedure uses MBS-associated signalling.

#### 8.14.2.2	Successful Operation

Figure 8.14.2.2-1: Broadcast Context Release procedure. Successful operation

The gNB-CU initiates the procedure by sending the BROADCAST CONTEXT RELEASE COMMAND message to the gNB-DU.

Upon reception of the BROADCAST CONTEXT RELEASE COMMAND message, the gNB-DU shall release all signalling and user data transport resources associated with the context and reply with the BROADCAST CONTEXT RELEASE COMPLETE message.

#### 8.14.2.3	Unsuccessful Operation

Not applicable.

#### 8.14.2.4	Abnormal Conditions

Not applicable.

### 8.14.3	Broadcast Context Release Request

#### 8.14.3.1	General

The purpose of the Broadcast Context Release Request procedure is to request the gNB-CU to trigger the Broadcast Context Release procedure.

The procedure uses MBS-associated signalling.

#### 8.14.3.2	Successful Operation

Figure 8.14.3.2-1: Broadcast Context Release Request procedure. Successful operation

The gNB-DU initiates the procedure by sending the BROADCAST CONTEXT RELEASE REQUEST message to the gNB-CU.

Interaction with the Broadcast Context Release procedure:

Upon reception of the BROADCAST CONTEXT RELEASE REQUEST message, the gNB-CU should trigger the Broadcast Context Release procedure.

#### 8.14.3.3	Unsuccessful Operation

Not applicable.

#### 8.14.3.4	Abnormal Conditions

Not applicable.

### 8.14.4	Broadcast Context Modification

#### 8.14.4.1	General

The purpose of the Broadcast Context Modification procedure is to modify an established MBS Session context for a broadcast session in the gNB-DU.

The procedure uses MBS-associated signalling.

#### 8.14.4.2	Successful Operation

Figure 8.14.4.2-1: Broadcast Context Modification procedure. Successful operation

The BROADCAST CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and, if successful, report the update in the BROADCAST CONTEXT MODIFICATION RESPONSE message.

If the Broadcast MRB To Be Setup List IE is contained in the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall setup the corresponding resources for the requested MRB(s), and report to the gNB-CU, in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Broadcast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Broadcast MRB Failed To Be Setup List IE;

If the Broadcast MRB Failed To Be Setup List IE is contained in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the setup of the indicated MRB(s) as failed and indicate the reason for the failure with an appropriate cause value for each MRB failed to be setup.

If the Broadcast MRB To Be Modified List IE is contained in the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall update the corresponding context and resources for the requested MRB(s), and report to the gNB-DU, in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the modification result of all the requested Broadcast MRBs in the following way:

-	A list of MRBs which have been successfully modified shall be included in the Broadcast MRB Modified List IE;

-	A list of MRBs which failed to be modified shall be included in the Broadcast MRB Failed To Be Modified List IE;

If the Broadcast MRB Failed To Be Modified List IE is contained in the BROADCAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the Broadcast MRB(s) failed to be modified with an appropriate cause value for each Broadcast MRB failed to modify.

If the MBS Service Area IE is included in the BROADCAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall

-	release MBS Session Resources within cells not contained in the MBS Service Area IE, if any;

-	establish MBS Session Resources within cells which have not been contained in MBS Service Area information previously received;

-	replace MBS Service Area information previously received with information received in the MBS Service Area IE included in the BROADCAST CONTEXT MODIFICATION REQUEST message;

-	include the Broadcast Area Scope IE in the BROADCAST CONTEXT MODIFICATION RESPONSE message to indicate the cells where MBS Session resources are currently established in the gNB-DU.

If the the MBS Service Area IE was not included in the BROADCAST CONTEXT MODIFICATION REQUEST message and the gNB-DU has released MBS Session Resources within at least one cell or has established MBS Session Resources within at least one cell the gNB-DU shall include the Broadcast Area Scope IE in the BROADCAST CONTEXT MODIFICATION RESPONSE message to indicate the cells where MBS Session resources are currently established in the gNB-DU.

#### 8.14.4.3	Unsuccessful Operation

Figure 8.14.4.3-1: Broadcast Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the broadcast context can be successfully performed, the gNB-DU shall respond with the BROADCAST CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

#### 8.14.4.4	Abnormal Conditions

Not applicable.

### 8.14.5	Multicast Group Paging

#### 8.14.5.1	General

The purpose of the Multicast Group Paging procedure is used to provide the paging information to enable the gNB-DU to multicast group page UEs which have joined an MBS Session and notify them about its activation. The procedure uses non-UE associated signalling.

#### 8.14.5.2	Successful Operation

Figure 8.14.5.2-1: Multicast Group Paging

The gNB-CU initiates the Multicast Group Paging procedure by sending the MULTICAST GROUP PAGING message to the gNB-DU.

At the reception of the MULTICAST GROUP PAGING message, the gNB-DU shall perform paging of the MBS Session identified by the MBS Session ID IE.

If the Paging DRX IE is included in the MULTICAST GROUP PAGING message gNB-DU shall use it according to TS 38.304 [24].

If the UE Identity List for Paging IE is included in the MULTICAST GROUP PAGING message, the gNB-DU shall, if supported, use it according to TS 38.304 [24]. If absent, the gNB-DU shall perform multicast group paging of the MBS session in all paging occasions within at least one default paging cycle, as specified in TS 38.304 [24].

At the reception of the MULTICAST GROUP PAGING message, the gNB-DU shall perform paging of the MBS Session in all the served cells of the PLMN indicated in the MBS Session ID IE except if the MC Paging Cell List IE is included in which case the gNB-DU shall, if supported, perform multicast group paging of the MBS session in the indicated cells only.

#### 8.14.5.3	Abnormal Conditions

Void.

### 8.14.6	Multicast Context Setup

#### 8.14.6.1	General

The purpose of the Multicast Context Setup procedure is to establish an MBS Session context in the gNB-DU for a multicast session.

The procedure uses MBS-associated signalling.

#### 8.14.6.2	Successful Operation

Figure 8.14.6.2-1: Multicast Context Setup procedure: Successful Operation

The gNB-CU initiates the procedure by sending MULTICAST CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the multicast MBS Session context, it replies to the gNB-CU with MULTICAST CONTEXT SETUP RESPONSE.

If the MBS Service Area IE is included in the MULTICAST CONTEXT SETUP REQUEST message, the gNB-DU shall take this information into account for shared F1-U tunnel assignment.

The gNB-DU shall report to the gNB-CU, in the MULTICAST CONTEXT SETUP RSPONSE message, the result of all the requested Multicast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Multicast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Multicast MRB Failed To Be Setup List IE;

If the Multicast MRB Failed To Setup List IE is contained in the MULTICAST CONTEXT SETUP RESPONSE message, the gNB-CU shall regard the Multicast MRB(s) failed to be setup with an appropriate cause value for each Multicast MRB failed to setup.

Interaction with the Multicast Distribution Context Setup procedure:

Upon reception of the MULTICAST CONTEXT SETUP REQUEST procedure, the gNB-DU shall trigger either per cell or per MBS Area Session ID or for the whole gNB-DU the Multicast Distribution Context Setup procedure to establish per cell or per MBS Area Session ID or the the whole gNB DU per accepted MRB a shared F1-U tunnel.

#### 8.14.6.3	Unsuccessful Operation

Figure 8.14.6.3-1: Multicast Context Setup procedure: unsuccessful Operation

If the gNB-DU is not able to establish the MBS session context it shall consider the procedure as failed and reply with the MULTICAST CONTEXT SETUP FAILURE message.

#### 8.14.6.4	Abnormal Conditions

Not applicable.

### 8.14.7	Multicast Context Release

#### 8.14.7.1	General

The purpose of the Multicast Context Release procedure is to enable the gNB-CU to order the release of an established MBS session context in the gNB-DU for a multicast session.

The procedure uses MBS-associated signalling.

#### 8.14.7.2	Successful Operation

Figure 8.14.7.2-1: Multicast Context Release procedure. Successful operation

The gNB-CU initiates the procedure by sending the MULTICAST CONTEXT RELEASE COMMAND message to the gNB-DU.

Upon reception of the MULTICAST CONTEXT RELEASE COMMAND message, the gNB-DU shall release all signalling and user data transport resources associated with the context and reply with the MULTICAST CONTEXT RELEASE COMPLETE message.

#### 8.14.7.3	Unsuccessful Operation

Not applicable.

#### 8.14.7.4	Abnormal Conditions

Not applicable.

### 8.14.8	Multicast Context Release Request

#### 8.14.8.1	General

The purpose of the Multicast Context Release Request procedure is to request the gNB-CU to trigger the Multicast Context Release procedure.

The procedure uses MBS-associated signalling.

#### 8.14.8.2	Successful Operation

Figure 8.14.8.2-1: Multicast Context Release Request procedure. Successful operation

The gNB-DU initiates the procedure by sending the MULTICAST CONTEXT RELEASE REQUEST message to the gNB-CU.

Interaction with the Multicast Context Release procedure:

Upon reception of the MULTICAST CONTEXT RELEASE REQUEST message, the gNB-CU should trigger the Multicast Context Release procedure.

#### 8.14.8.3	Unsuccessful Operation

Not applicable.

#### 8.14.8.4	Abnormal Conditions

Not applicable.

### 8.14.9	Multicast Context Modification

#### 8.14.9.1	General

The purpose of the Multicast Context Modification procedure is to modify an established MBS session context in the gNB-DU for a multicast session.

The procedure uses MBS-associated signalling.

#### 8.14.9.2	Successful Operation

Figure 8.14.9.2-1: Multicast Context Modification procedure. Successful operation

The MULTICAST CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the MULTICAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and, if successful, report the update in the MULTICAST CONTEXT MODIFICATION RESPONSE message.

If the Multicast MRB To Be Setup List IE is contained in the MULTICAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall setup the corresponding resources for the requested MRB(s), and report to the gNB-CU, in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the result of all the requested Multicast MRBs in the following way:

-	A list of MRBs which have been successfully established shall be included in the Multicast MRB Setup List IE;

-	A list of MRBs which failed to be established shall be included in the Multicast MRB Failed To Be Setup List IE;

If the Multicast MRB Failed To Be Setup List IE is contained in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the setup of the indicated MRB(s) as failed and indicated the resource for the failure with an appropriate cause value for each MRB failed to be setup.

If the Multicast MRB To Be Modified List IE is contained in the MULTICAST CONTEXT MODIFICATION REQUEST message, the gNB-DU shall update the corresponding context and resources for the requested MRB(s), and report to the gNB-CU, in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the modification result of all the requested Multicast MRBs in the following way:

-	A list of MRBs which have been successfully modified shall be included in the Multicast MRB Modified List IE;

-	A list of MRBs which failed to be modified shall be included in the Multicast MRB Failed To Be Modified List IE;

If the Multicast MRB Failed To Be Modified List IE is contained in the MULTICAST CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall regard the Multicast MRB(s) failed to be modified with an appropriate cause value for each Multicast MRB failed to modify.

#### 8.14.9.3	Unsuccessful Operation

Figure 8.14.9.3-1: Multicast Context Modification procedure. Unsuccessful operation

In case none of the requested modifications of the multicast context can be successfully performed, the gNB-DU shall respond with the MULTICAST CONTEXT MODIFICATION FAILURE message with an appropriate cause value.

#### 8.14.9.4	Abnormal Conditions

Not applicable.

### 8.14.10	Multicast Distribution Setup

#### 8.14.10.1	General

The purpose of the Multicast Distribution Setup procedure is to establish F1-U bearers for the multicast MBS session.

The procedure uses MBS-associated signalling.

#### 8.14.10.2	Successful Operation

Figure 8.14.10.2-1: Multicast Distribution Setup procedure: Successful Operation

The gNB-DU initiates the procedure by sending MULTICAST DISTRIBUTION SETUP REQUEST message to the gNB-CU. If the gNB-CU succeeds to establish the multicast context, it replies to the gNB-DU with MULTICAST DISTRIBUTION SETUP RESPONSE.

The MULTICAST DISTRIBUTION SETUP REQUEST message shall contain F1-U TNL information for the MRBs accepted for the MBS Session by the gNB-DU and indicate in the MBS Multicast F1-U Context Descriptor IE, if the shared F1-U tunnel(s) for the MRB(s) are established on a per NR CGI or per MBS Area Session ID basis or for a ptp MRB leg.

Upon reception of the MULTICAST DISTRIBUTION SETUP REQUEST message the gNB-CU shall allocate F1-U resources and reply accordingly to the gNB-DU in the MULTICAST DISTRIBUTION SETUP RESPONSE message.

If the MC F1-U Context usage IE in the MBS Multicast F1-U Context Descriptor IE is set to "ptp forwarding" the gNB-CU shall, if supported, use the MRB Progress Information IE to determine at which PDCP SN to start transmitting multicast data to the gNB-DU.

#### 8.14.10.3	Unsuccessful Operation

Figure 8.14.10.3-1: Multicast Distribution Setup procedure: unsuccessful Operation

If the gNB-CU is not able to provide the requested resources it shall consider the procedure as failed and reply with the MULTICAST CONTEXT SETUP FAILURE message.

#### 8.14.10.4	Abnormal Conditions

Not applicable.

### 8.14.11	Multicast Distribution Release

#### 8.14.11.1	General

The purpose of the Multicast Distribution Release procedure is to enable the gNB-DU to order the release of F1-U tunnels previously established using the Multicast Distribution Setup procedure.

The procedure uses MBS-associated signalling.

#### 8.14.11.2	Successful Operation

Figure 8.14.11.2-1: Multicast Distribution Release procedure. Successful operation

The gNB-DU initiates the procedure by sending the MULTICAST DISTRIBUTION RELEASE COMMAND message to the gNB-CU.

Upon reception of the MULTICAST DISTRIBUTION RELEASE COMMAND message, the gNB-CU shall release all signalling and user data transport resources associated with the context and reply with the MULTICAST DISTRIBUTION RELEASE COMPLETE message.

#### 8.14.11.3	Unsuccessful Operation

Not applicable.

#### 8.14.11.4	Abnormal Conditions

Not applicable.

## 8.15	PDC Measurement Reporting procedures

### 8.15.1	PDC Measurement Initiation

#### 8.15.1.1	General

The purpose of the PDC Measurement Initiation procedure is to enable the gNB-CU to request the gNB-DU to report measurements used for propagation delay compensation at the gNB-CU or UE. The procedure uses UE-associated signalling.

#### 8.15.1.2	Successful Operation

Figure 8.15.1.2-1: PDC Measurement Initiation procedure, successful operation

The gNB-CU initiates the procedure by sending a PDC MEASUREMENT INITIATION REQUEST message. If the gNB-DU is able to initiate the requested PDC measurements, it shall reply with the PDC MEASUREMENT INITIATION RESPONSE message.

If the PDC Report Type IE is set to "OnDemand", the gNB-DU shall return the result of the measurement in the PDC MEASUREMENT INITIATION RESPONSE message including the PDC Measurement Result IE, and the gNB-CU shall consider that the PDC measurements for the UE have been terminated by the gNB-DU.

Interaction with the PDC Measurement Report procedure:

If the PDC Report Type IE is set to "Periodic", the gNB-DU shall initiate the requested measurements and shall reply with the PDC MEASUREMENT INITIATION RESPONSE message without including the PDC Measurement Result IE in this message. The gNB-DU shall then periodically initiate the PDC Measurement Report procedure for the measurements, with the requested reporting periodicity.

#### 8.15.1.3	Unsuccessful Operation

Figure 8.15.1.3-1: PDC Measurement Initiation procedure, unsuccessful operation

If the gNB-DU is not able to initiate at least one of the requested PDC measurements, the gNB-DU shall respond with a PDC MEASUREMENT INITIATION FAILURE message.

### 8.15.2	PDC Measurement Report

#### 8.15.2.1	General

The purpose of the PDC Measurement Report procedure is for the gNB-DU to provide the PDC measurements for the UE to the gNB-CU. The procedure uses UE-associated signalling.

#### 8.15.2.2	Successful Operation

Figure 8.15.2.2-1: PDC Measurement Report procedure, successful operation

The gNB-DU initiates the procedure by sending a PDC MEASUREMENT REPORT message. The PDC MEASUREMENT REPORT message contains the PDC measurement results according to the measurement configuration in the respective PDC MEASUREMENT INITIATION REQUEST message.

#### 8.15.2.3	Unsuccessful Operation

Not applicable.

### 8.15.3	PDC Measurement Termination

#### 8.15.3.1	General

The purpose of the PDC Measurement Termination procedure is to enable the gNB-CU to terminate an on-going periodical PDC measurement. The procedure uses UE-associated signalling.

#### 8.15.3.2	Successful Operation

Figure 8.15.3.2-1: PDC Measurement Termination procedure: successful operation

The gNB-CU initiates the procedure by sending a PDC MEASUREMENT TERMINATION COMMAND message. Upon receiving this message, the gNB-DU shall terminate the ongoing PDC measurement and may release any resources previously allocated for the same measurement.

#### 8.15.3.3	Unsuccessful Operation

Not applicable.

#### 8.15.3.4	Abnormal Conditions

If the gNB-DU cannot identify the previously requested measurement to be terminated, it shall ignore the PDC MEASUREMENT TERMINATION COMMAND message.

### 8.15.4	PDC Measurement Failure Indication

#### 8.15.4.1	General

The purpose of the PDC Measurement Failure Indication procedure is for the gNB-DU to notify the gNB-CU that the PDC measurements previously requested with the PDC Measurement Initiation procedure can no longer be reported. The procedure uses UE-associated signalling.

#### 8.15.4.2	Successful Operation

Figure 8.15.4.2-1: PDC Measurement Failure Indication procedure: successful operation

The gNB-DU initiates the procedure by sending a PDC MEASUREMENT FAILURE INDICATION message. Upon reception of the PDC MEASUREMENT FAILURE INDICATION message, the gNB-CU shall consider that the indicated PDC measurements have been terminated by the gNB-DU.

#### 8.15.4.3	Unsuccessful Operation

Not applicable.

#### 8.15.4.4	Abnormal Conditions

Void.

## 8.16	QMC Procedures

### 8.16.1	QoE Information Transfer

#### 8.16.1.1	General

The purpose of the QoE Information Transfer procedure is to transfer RAN visible QoE information from the gNB-CU to the gNB-DU. The procedure uses UE-associated signalling.

#### 8.16.1.2	Successful operation

Figure 8.16.1.2-1: QoE Information Transfer procedure

The gNB-CU initiates the procedure by sending the QOE INFORMATION TRANSFER message to the gNB-DU.

If the QoE Information List IE is included in QOE INFORMATION TRANSFER message, the gNB-DU may take it into account according to TS 38.300 [6].

#### 8.16.1.3	Abnormal Conditions

Not applicable.

# 9	Elements for F1AP Communication

## 9.1	General

Subclauses 9.2 and 9.3 present the F1AP message and IE definitions in tabular format. The corresponding ASN.1 definition is presented in subclause 9.4. In case there is contradiction between the tabular format and the ASN.1 definition, the ASN.1 shall take precedence, except for the definition of conditions for the presence of conditional IEs, where the tabular format shall take precedence.

The messages have been defined in accordance to the guidelines specified in TR 25.921 [14].

When specifying IEs which are to be represented by bitstrings, if not otherwise specifically stated in the semantics description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

-	The first bit (leftmost bit) contains the most significant bit (MSB);

-	The last bit (rightmost bit) contains the least significant bit (LSB);

-	When importing bitstrings from other specifications, the first bit of the bitstring contains the first bit of the concerned information;

The following attributes are used for the tabular description of the messages and information elements: Presence, Range Criticality and Assigned Criticality. Their definition and use can be found in TS 38.413 [3].

## 9.2	Message Functional Definition and Content

### 9.2.1	Interface Management messages

#### 9.2.1.1	RESET

This message is sent by both the gNB-CU and the gNB-DU and is used to request that the F1 interface, or parts of the F1 interface, to be reset.

Direction: gNB-CU  gNB-DU and gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| CHOICE Reset Type | M |  |  |  | YES | reject |
| >F1 interface |  |  |  |  |  |  |
| >>Reset All | M |  | ENUMERATED (Reset all,...) |  | - |  |
| >Part of F1 interface |  |  |  |  |  |  |
| >>UE-associated logical F1-connection list |  | 1 |  |  | - |  |
| >>>UE-associated logical F1-connection Item |  | 1 .. <maxnoofIndividualF1ConnectionsToReset> |  |  | EACH | reject |
| >>>>gNB-CU UE F1AP ID | O |  | 9.3.1.4 |  | - |  |
| >>>>gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualF1ConnectionsToReset | Maximum no. of UE-associated logical F1-connections allowed to reset in one message. Value is 65536. |

#### 9.2.1.2	RESET ACKNOWLEDGE

This message is sent by both the gNB-CU and the gNB-DU as a response to a RESET message.

Direction: gNB-DU  gNB-CU and gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| UE-associated logical F1-connection list |  | 0..1 |  |  | YES | ignore |
| >UE-associated logical F1-connection Item |  | 1 .. <maxnoofIndividualF1ConnectionsToReset> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | O |  | 9.3.1.4 |  | - |  |
| >>gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofIndividualF1ConnectionsToReset | Maximum no. of UE-associated logical F1-connections allowed to reset in one message. Value is 65536. |

#### 9.2.1.3	ERROR INDICATION

This message is sent by both the gNB-CU and the gNB-DU and is used to indicate that some error has been detected in the node.

Direction: gNB-CU  gNB-DU and gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 | This IE is ignored if received in UE associated signalling message. | YES | reject |
| gNB-CU UE F1AP ID | O |  | 9.3.1.4 |  | YES | ignore |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| Cause | O |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.4	F1 SETUP REQUEST

This message is sent by the gNB-DU to transfer information associated to an F1-C interface instance.

NOTE:	If a TNL association is shared among several F1-C interface instances, several F1 Setup procedures are issued via the same TNL association after that TNL association has become operational.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-DU ID | M |  | 9.3.1.9 |  | YES | reject |
| gNB-DU Name | O |  | PrintableString(SIZE(1..150,...)) |  | YES | ignore |
| gNB-DU Served Cells List |  | 0.. 1 |  | List of cells configured in the gNB-DU | YES | reject |
| >gNB-DU Served Cells Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| gNB-DU RRC version | M |  | RRC version 9.3.1.70 |  | YES | reject |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-node. | YES | ignore |
| Extended gNB-DU Name | O |  | 9.3.1.205 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

#### 9.2.1.5	F1 SETUP RESPONSE

This message is sent by the gNB-CU to transfer information associated to an F1-C interface instance.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Cells to be Activated List |  | 0.. 1 |  |  | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  | List of cells to be activated | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| gNB-CU RRC version | M |  | RRC version 9.3.1.70 |  | YES | reject |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

#### 9.2.1.6	F1 SETUP FAILURE

This message is sent by the gNB-CU to indicate F1 Setup failure.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to wait | O |  | 9.3.1.13 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.7	GNB-DU CONFIGURATION UPDATE

This message is sent by the gNB-DU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Served Cells To Add List |  | 0..1 |  | Complete list of added cells served by the gNB-DU | YES | reject |
| >Served Cells To Add Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| Served Cells To Modify List |  | 0..1 |  | Complete list of modified cells served by the gNB-DU | YES | reject |
| >Served Cells To Modify Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Served Cell Information | M |  | 9.3.1.10 | Information about the cells configured in the gNB-DU | - |  |
| >>gNB-DU System Information | O |  | 9.3.1.18 | RRC container with system information owned by gNB-DU | - |  |
| Served Cells To Delete List |  | 0..1 |  | Complete list of deleted cells served by the gNB-DU | YES | reject |
| >Served Cells To Delete Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>Old NR CGI | M |  | NR CGI 9.3.1.12 |  | - |  |
| Cells Status List |  | 0..1 |  | Complete list of active cells | YES | reject |
| >Cells Status Item |  | 0 .. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Service Status | M |  | 9.3.1.68 |  | - |  |
| Dedicated SI Delivery Needed UE List |  | 0..1 |  | List of UEs unable to receive system information from broadcast | YES | ignore |
| >Dedicated SI Delivery Needed UE Item |  | 1 .. <maxnoofUEIDs> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-DU ID | O |  | 9.3.1.9 |  | YES | reject |
| gNB-DU TNL Association To Remove List |  | 0..1 |  |  | YES | reject |
| >gNB-DU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | reject |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-DU. | - | - |
| >>TNL Association Transport Layer Address gNB-CU | O |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU | - | - |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Coverage Modification Notification | O |  | 9.3.1.213 |  | YES | Ignore |
| gNB-DU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-DU. | YES | ignore |
| Extended gNB-DU Name | O |  | 9.3.1.205 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |

#### 9.2.1.8	GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-CU to a gNB-DU to acknowledge update of information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0.. 1 |  | List of cells to be activated | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >> gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Cells to be Deactivated List |  | 0.. 1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - | - |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

#### 9.2.1.9	GNB-DU CONFIGURATION UPDATE FAILURE

This message is sent by the gNB-CU to indicate gNB-DU Configuration Update failure.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to wait | O |  | 9.3.1.13 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Transport Layer Address gNB-DU | O |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-DU. | YES | reject |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Barred | M |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| >>IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) | Corresponds to information provided in the iab-Support contained in the PLMN-IdentityInfo IE or contained in the NPN-IdentityInfo IE as defined in TS 38.331 [8]. The codepoint value “barred” indicates that the iab-Support is not sent in SIB1, and the codepoint value “not-barred” indicates that the iab-Support is sent in SIB1. | - |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 1.. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | - |  |
| >>E-UTRA Cells List |  | 1 |  | List of applicable E-UTRA cells. | - |  |
| >>>E-UTRA Cells List Item |  | 1 .. <maxCellineNB> |  |  | - |  |
| >>>>EUTRA Cell ID | M |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Identifier IE contained in the ECGI as defined in subclause 9.2.14 in TS 36.423 [9]. | - |  |
| >>>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | - |  |
| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| BAP Address | O |  | 9.3.1.111 | Indicates a BAP address assigned to the IAB-donor-DU. | YES | ignore |
| CCO Assistance Information | O |  | 9.3.1.211 | Indicates CCO Assistance Information for cells and beams served by the gNB-DU of the same NG-RAN node or for cells and beams not served by the gNB-DU. | YES | Ignore |
| Cells for SON List | O |  | 9.3.1.214 |  | YES | ignore |
| gNB-CU Name | O |  | PrintableString(SIZE(1..150,...)) | Human readable name of the gNB-CU. | YES | ignore |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

#### 9.2.1.11	GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-DU to a gNB-CU to acknowledge update of information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instance, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells Failed to be Activated List |  | 0..1 |  | List of cells which are failed to be activated | YES | reject |
| >Cells Failed to be Activated Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cause | M |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| gNB-CU TNL Association Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU | - |  |
| gNB-CU TNL Association Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association Failed To Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU | - |  |
| >>Cause | M |  | 9.3.1.2 |  | - |  |
| Dedicated SI Delivery Needed UE List |  | 0..1 |  | List of UEs unable to receive system information from broadcast | YES | ignore |
| >Dedicated SI Delivery Needed UE List |  | 1 .. <maxnoofUEIDs> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - | - |
| >>NR CGI | M |  | 9.3.1.12 |  | - | - |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |

#### 9.2.1.12	GNB-CU CONFIGURATION UPDATE FAILURE

This message is sent by the gNB-DU to indicate gNB-CU Configuration Update failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to wait | O |  | 9.3.1.13 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.13	GNB-DU RESOURCE COORDINATION REQUEST

This message is sent by a gNB-CU to a gNB-DU, to express the desired resource allocation for data traffic, for the sake of resource coordination. The message triggers gNB-DU resource coordination (for NR-initiated resource coordination), to indicate an initial resource offer by the E-UTRA node (for E-UTRA-initiated gNB-DU Resource Coordination), or to indicate the agreed resource allocation that is to be executed.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Request type | M |  | ENUMERATED (offer, execution, ...) |  | YES | reject |
| E-UTRA – NR Cell Resource Coordination Request Container | M |  | OCTET STRING | In EN-DC case, includes the X2AP E-UTRA – NR CELL RESOURCE COORDINATION REQUEST message as defined in subclause 9.1.4.24 in TS 36.423 [9]. In NG-RAN cases, includes the XnAP E-UTRA – NR CELL RESOURCE COORDINATION REQUEST message as defined in subclause 9.1.2.23 in TS 38.423 [28]. | YES | reject |
| Ignore Coordination Request Container | O |  | ENUMERATED (yes, ...) |  | YES | reject |

#### 9.2.1.14	GNB-DU RESOURCE COORDINATION RESPONSE

This message is sent by a gNB-DU to a gNB-CU, to express the desired resource allocation for data traffic, as a response to the GNB-DU RESOURCE COORDINATION REQUEST.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| E-UTRA – NR Cell Resource Coordination Response Container | M |  | OCTET STRING | In EN-DC case, includes the X2AP E-UTRA – NR CELL RESOURCE COORDINATION RESPONSE message as defined in subclause 9.1.4.25 in TS 36.423 [9]. In NG-RAN cases, includes the XnAP E-UTRA – NR CELL RESOURCE COORDINATION RESPONSE message as defined in subclause 9.1.2.24 in TS 38.423 [28]. | YES | reject |

#### 9.2.1.15	GNB-DU STATUS INDICATION

This message is sent by the gNB-DU to indicate to the gNB-CU its status of overload.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-DU Overload Information | M |  | ENUMERATED (overloaded, not-overloaded) |  | YES | reject |
| IAB Congestion Indication | O |  | 9.3.1.227 |  | YES | ignore |

#### 9.2.1.16	F1 REMOVAL REQUEST

This message is sent by either the gNB-DU or the gNB-CU to intiate the removal of the interface instance and the related resources.

Direction: gNB-DU  gNB-CU, gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |

#### 9.2.1.17	F1 REMOVAL RESPONSE

This message is sent by either the gNB-DU or the gNB-CU to acknowledge the initiation of removal of the interface instance and the related resources.

Direction: gNB-CU  gNB-DU, gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.18	F1 REMOVAL FAILURE

This message is sent by either the gNB-DU or the gNB-CU to indicate that removing the interface instance and the related resources cannot be accepted.

Direction: gNB-CU  gNB-DU, gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.19	NETWORK ACCESS RATE REDUCTION

This message is sent by the gNB-CU to indicate to the gNB-DU a need to reduce the rate at which UEs access the network.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| UAC Assistance Information | M |  | 9.3.1.83 |  | YES | reject |

#### 9.2.1.20	RESOURCE STATUS REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestStoporAdd |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, add, …) | Type of request for which the resource status is required. | YES | ignore |
| Report Characteristics | C-ifRegistrationRequestStart |  | BIT STRING (SIZE(32)) | Each position in the bitmap indicates measurement object the gNB-DU is requested to report. First Bit = PRB Periodic, Second Bit = TNL Capacity Ind Periodic, Third Bit = Composite Available Capacity Periodic, Fourth Bit = HW LoadInd Periodic, Fifth Bit = Number of Active UEs Periodic, Sixth Bit = NR-U Channel List Periodic. Other bits shall be ignored by the gNB-DU. | YES | ignore |
| Cell To Report List |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>SSB To Report List |  | 0..1 |  | SSB list to which the request applies. | - |  |
| >>>SSB To Report Item |  | 1 .. < maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB index | M |  | INTEGER (0..63) |  | - |  |
| >>Slice To Report List |  | 0..1 |  | S-NSSAI list to which the request applies. | - |  |
| >>>Slice To Report Item |  | 1..< maxnoofBPLMNsNR> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.1.14 | Broadcast PLMN | - |  |
| >>>>S-NSSAI List |  | 1 |  |  | - |  |
| >>>>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  | - |  |
| >>>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| Reporting Periodicity | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms,10000ms, …) | Periodicity that can be used for reporting of indicated measurements. Also used as the averaging window length for all measurement object if supported. This IE is ignored if the Registration Request IE is set to “add”. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStoporAdd | This IE shall be present if the Registration Request IE is set to the value "stop" or "add". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a gNB node cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

#### 9.2.1.21	RESOURCE STATUS RESPONSE

This message is sent by gNB-DU to gNB-CU to indicate that the requested measurement is successfully initiated.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.22	RESOURCE STATUS FAILURE

This message is sent by gNB-DU to gNB-CU to indicate that for any of the requested measurement objects the measurement cannot be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.1.23	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Hardware Load Indicator | O |  | 9.3.1.136 |  | YES | ignore |
| TNL Capacity Indicator | O |  | 9.3.1.128 |  | YES | ignore |
| Cell Measurement Result |  | 0..1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellingNBDU > |  |  | - |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Radio Resource Status | O |  | 9.3.1.129 |  | - |  |
| >>Composite Available Capacity Group | O |  | 9.3.1.130 |  | - |  |
| >>Slice Available Capacity | O |  | 9.3.1.134 |  | - |  |
| >>Number of Active UEs | O |  | 9.3.1.135 |  | - |  |
| >>NR-U Channel List |  | 0..1 |  |  | YES | ignore |
| >>>NR-U Channel Item |  | 1..<maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Identifies a portion of the NR-U Channel Bandwidth on which channel access procedure in shared spectrum has been performed in the last reporting period. | - |  |
| >>>>Channel occupancy time percentage DL | M |  | INTEGER (0..100) | The percentage of time for which the channel resources have been utilised for DL traffic served by the corresponding NR-U Channel of the serving cell. Value 100 indicates that the channel resources have been utilized for DL traffic served by the corresponding NR-U Channel of the serving cell for the whole duration between consecutive reporting. | - |  |
| >>>>Energy Detection Threshold DL | M |  | INTEGER (-100..-50,…) | Average ED Threshold used for DL channel sensing at the gNB. Value is in dBm. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |

### 9.2.2	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| >>SDT RLC Bearer Configuration | O |  | OCTET STRING | Includes the RLC-BearerConfig IE defined in subclause 6.3.2 of TS 38.331 [8] | YES | ignore |
| >>SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the SRB | YES | ignore |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID of the DL tunnel corresponding to such UL tunnel | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuraiton 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC basedUL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | M |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>SDT RLC Bearer Configuration | O |  | OCTET STRING | RLC-BearerConfig IE defined in subclause 6.3.2 of TS 38.331 [8] | YES | ignore |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.55 |  | YES | ignore |
| Serving PLMN | O |  | PLMN ID 9.3.1.14 | Indicates the PLMN serving the UE. | YES | ignore |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | C-ifDRBSetup |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE (8)) |  | YES | ignore |
| Trace Activation | O |  | 9.3.1.88 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS Information | M |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. | - |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. | - |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| Configured BAP Address | O |  | 9.3.1.111 | The BAP address configured for the corresponding child IAB-node. | YES | reject |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier |  |  | 9.3.1.121 |  | - |  |
| >>RLC mode | M |  | 9.3.1.27 |  | - |  |
| Conditional Inter-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | - | - |
| >Target gNB-DU UE F1AP ID | C-ifCHOmod |  | 9.3.1.5 | Allocated at the target gNB-DU | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.151 |  | YES | ignore |
| Serving NID | O |  | 9.3.1.155 |  | YES | reject |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| F1-C Transfer Path NRDC | O |  | 9.3.1.228 |  | YES | reject |
| MDT Polluted Measurement Indicator | O |  | ENUMERATED (IDC,no-IDC, …) | Indication on whether MDT Measurement affect (e.g. IDC) is undertaken or not. | YES | ignore |
| SCG Activation Request | O |  | 9.3.1.233 |  | YES | ignore |
| Old CG-SDT Session Info | O |  | CG-SDT Session Info 9.3.1.261 |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.268 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | This IE applies only if the UE is authorized for 5G ProSe services, and only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| Uu RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | M |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| PC5 RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 | This IE is not used in this version of the specification. | - |  |
| >>CHOICE PC5 RLC Channel QoS Information | M |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| Path Switch Configuration | O |  | 9.3.1.263 |  | YES | ignore |
| gNB-DU UE Slice Maximum Bit Rate List | O |  | 9.3.1.271 | The Slice Maximum Bit Rate List is the maximum aggregate UL bit rate per slice, to be enforced by the gNB-DU, if feasible. This IE is ignored if the DRB to Be Setup List IE is not present. | YES | ignore |
| Multicast MBS Session Setup List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has joined. | YES | reject |
| UE Multicast MRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>MBS PTP Retransmission Tunnel Required | O |  | 9.3.2.10 |  | - |  |
| >>MBS PTP Forwarding Tunnel Required Information | O |  | MRB Progress Information 9.3.2.12 |  | - |  |
| >>Source MRB ID | O |  | 9.3.1.224 MRB ID | In case of inter-DU handover, indicates the MRB ID provided to the UE in the source cell. | - | ignore |
| ServingCellMO List |  | 0..1 |  | For NCD-SSBs | YES | ignore |
| >ServingCellMO Item IEs |  | 1 .. <maxnoofServingCellMOs> |  |  | EACH | ignore |
| >>servingCellMO | M |  | INTEGER (1..64) |  | - |  |
| >>SSB frequency | M |  | INTEGER (0..3279165) | ARFCN | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

#### 9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DU To CU RRC Information | M |  | 9.3.1.26 |  | YES | reject |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item Iist |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used in this version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| SRB Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Failed to Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >DRB Failed to Setup Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SCell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell Failed to Setup Item |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Inactivity Monitoring Response | O |  | ENUMERATED (not-supported, ...) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| SRB Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| BH RLC Channel Setup List |  | 0..1 |  | The list of BH RLC channels which are successfully established. | YES | ignore |
| >BH RLC Channel Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Setup List |  | 0..1 |  | The list of BH RLC channels whose setup has failed. | YES | ignore |
| >BH RLC Channel Failed to be Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Setup List |  | 0..1 |  | The List of SL DRBs which are successfully established. | YES | ignore |
| >SL DRB Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Failed To Setup List |  | 0..1 |  |  | EACH | ignore |
| >SL DRB Failed To Setup Item IE |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Requested Target Cell ID | O |  | NR CGI 9.3.1.12 | Special Cell indicated in the UE CONTEXT SETUP REQUEST message. | YES | reject |
| SCG Activation Status | O |  | 9.3.1.234 |  | YES | ignore |
| Uu RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| Uu RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 | This IE is not used in this version of the specification. |  |  |
| PC5 RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 | This IE is not used in this version of the specification. | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| ServingCellMO-encoded-in-CGC List |  | 0..1 |  |  | YES | ignore |
| >ServingCellMO-encoded-in-CGC Item IEs |  | 1 .. <maxNrofBWPs> |  | The servingCellMO which has been encoded in CellGroupConfig IE. | EACH | ignore |
| >>servingCellMO | M |  | INTEGER (1..64) |  | - |  |
| >>BWP ID | M |  | INTEGER (0..4) |  | YES | ignore |
| UE Multicast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>Multicast F1-U Context Reference CU | M |  | 9.3.2.13 |  | - |  |
| Dedicated SI Delivery Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| Configured BWP List |  | 0..1 |  | This IE is present when the gNB-DU configures at least one BWP with NCD-SSB or without SSB. | YES | ignore |
| >Configured BWP Item IEs |  | 1 .. <maxNrofBWPs> |  |  | EACH | ignore |
| >>BWP-Id | M |  | INTEGER (0..4) | The IE is used to refer to one BWP. | - |  |
| >>BWP Location And Bandwidth | M |  | INTEGER (0..37949) | The IE type range is the same as the locationAndBandwidth IE in BWP IE as specified in TS 38.331 [8]. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxNrofBWPs | Maximum number of BWPs per serving cell, the maximum value is 8. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |

#### 9.2.2.3	UE CONTEXT SETUP FAILURE

This message is sent by the gNB-DU to indicate that the setup of the UE context was unsuccessful.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Potential SpCell List |  | 0..1 |  |  | YES | ignore |
| >Potential SpCell Item IEs |  | 0 .. <maxnoofPotentialSpCells> |  |  | EACH | ignore |
| >>Potential SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| Requested Target Cell ID | O |  | NR CGI 9.3.1.12 | Special Cell indicated in the UE CONTEXT SETUP REQUEST message. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPotentialSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |

#### 9.2.2.4	UE CONTEXT RELEASE REQUEST

This message is sent by the gNB-DU to request the gNB-CU to release the UE-associated logical F1 connection or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |

#### 9.2.2.5	UE CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU to request the gNB-DU to release the UE-associated logical F1 connection or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8] encapsulated in a PDCP PDU, or the DL-CCCH-Message message as defined in subclause 6.2 of TS 38.331 [8]. | YES | ignore |
| SRB ID | C- ifRRCContainer |  | 9.3.1.7 | The gNB-DU sends the RRC message on the indicated SRB. | YES | ignore |
| old gNB-DU UE F1AP ID | O |  | 9.3.1.5 | Include it if RRCReestablishmentRequest is not accepted | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| Positioning Context Reservation Indication | O |  | ENUMERATED (True,...) |  | YES | ignore |
| CG-SDT Kept Indicator | O |  | ENUMERATED (true, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |

| Condition | Explanation |
| --- | --- |
| ifRRCContainer | This IE shall be present if the RRC container IE is present. |

#### 9.2.2.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the gNB-DU to confirm the release of the UE-associated logical F1 connection or candidate cells in conditional handover or conditional PSCell addition or conditional PSCell change.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| >>SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the SRB | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT SRB. | YES | reject |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID of the DL tunnel corresponding to such UL tunnel | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates SDT DRB. | YES | reject |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS |  |  |  |  |  |  |
| >>>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  |  |  |  |  |  |
| >>>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 |  | YES | ignore |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>DL PDCP SN length | O |  | ENUMERATED(12bits,18bits , ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Bearer Type Change | O |  | ENUMERATED (true, …) |  | YES | ignore |
| >>RLC Mode | O |  | 9.3.1.27 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, …, false) | Indication on whether DC based PDCP duplication is configured or not. | YES | reject |
| >>DC Based Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>Transmission Stop Indicator | O |  | 9.3.1.209 |  | YES | ignore |
| >>SDT Indicator Modify | O |  | ENUMTERATED (true, false, …) | Indicates SDT DRB or not. | YES | reject |
| SRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >SRB To Be Released Item IEs |  | 1.. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| DRX configuration indicator | O |  | ENUMERATED(release,...) |  | YES | ignore |
| RLC Failure Indication | O |  | 9.3.1.66 |  | YES | ignore |
| Uplink TxDirectCurrentList Information | O |  | 9.3.1.67 |  | YES | ignore |
| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| Need for Gap | O |  | ENUMERATED (true, …) | Indicate gap for SeNB configured measurement is requested.It only applied to NE DC scenario. | Yes | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| Lower Layer Presence Status Change | O |  | 9.3.1.94 |  | Yes | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | M |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Modified List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | O |  |  |  |  |  |
| >>>BH RLC CH QoS |  |  |  |  |  |  |
| >>>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. | - |  |
| >>>E-UTRAN BH RLC CH QoS |  |  |  |  |  |  |
| >>>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. | - |  |
| >>>Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Control Plane Traffic Type | M |  | 9.3.1.115 |  | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Released List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Released Item IEs |  | 1 .. <maxnoofBHRLCChannels > |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| SL DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| SL DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Released Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| Conditional Intra-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, CHO-cancel, …) |  | - | - |
| >Candidate Cells To Be Cancelled List | C-ifCHOcancel | 0 .. <maxnoofCellsinCHO> |  |  | - | - |
| >>Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| SCG Indicator | O |  | ENUMERATED(released,...) | This IE is used at the MN in NR-DC and NE-DC and it indicates the release of an SCG | YES | ignore |
| Uplink TxDirectCurrentTwoCarrierList Information | O |  | 9.3.1.283 |  | YES | ignore |
| IAB Conditional RRC Message Delivery Indication | O |  | ENUMERATED (true, …) | Indicates whether the RRC message within should be withheld. This IE is only applicable if the UE is an IAB-MT, and the gNB-DU is an IAB-DU. | YES | reject |
| F1-C Transfer Path NRDC | O |  | 9.3.1.228 | This IE is only applicable if the UE is an IAB-MT. | YES | reject |
| MDT Polluted Measurement Indicator | O |  | ENUMERATED (IDC,no-IDC, …) | Indication on whether MDT Measurement affect (e.g. IDC) is undertaken or not. | YES | ignore |
| SCG Activation Request | O |  | 9.3.1.233 |  | YES | ignore |
| CG-SDT Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| 5G ProSe Authorized | O |  | 9.3.1.268 |  | YES | ignore |
| 5G ProSe UE PC5 Aggregate Maximum Bit Rate | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for 5G ProSe services. | YES | ignore |
| 5G ProSe PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | This IE applies only if the UE is authorized for 5G ProSe services, and only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| Updated Remote UE Local ID | O |  | Remote UE Local ID 9.3.1.267 | This IE indicates the updated Remote UE Local ID for the U2N Remote UE associated with the F1AP-IDs | YES | ignore |
| Uu RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | M |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| Uu RLC Channel to Be Modified List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>CHOICE Uu RLC Channel QoS Information | O |  |  |  | - |  |
| >>>Uu RLC Channel QoS |  |  |  |  |  |  |
| >>>>Uu RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>Uu Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>Uu Control Plane Traffic Type | M |  | ENUMERATED(SRB0, SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the Uu Relay RLC Channel. | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| Uu RLC Channel to Be Released List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel to Be Released Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC channel ID | M |  | 9.3.1.266 |  | - |  |
| PC5 RLC Channel to Be Setup List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>CHOICE PC5 RLC Channel QoS Information | M |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …) | This IE indicates the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| PC5 RLC Channel to Be Modified List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  |  |  |
| >>CHOICE PC5 RLC Channel QoS Information | O |  |  |  | - |  |
| >>>PC5 RLC Channel QoS |  |  |  |  |  |  |
| >>>>PC5 RLC Channel QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>>PC5 Control Plane Traffic Type |  |  |  |  |  |  |
| >>>>PC5 Control Plane Traffic Type | M |  | ENUMERATED(SRB1, SRB2, …) | This IE indicate the type of SRB conveyed via the PC5 Relay RLC Channel. | - |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| PC5 RLC Channel to Be Released List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel to be Released Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| Path Switch Configuration | O |  | 9.3.1.263 |  | YES | ignore |
| gNB-DU UE Slice Maximum Bit Rate List | O |  | 9.3.1.271 | The Slice Maximum Bit Rate List is the maximum aggregate UL bit rate per slice, to be enforced by the gNB-DU, if feasible. | YES | ignore |
| Multicast MBS Session Setup List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has joined. | YES | reject |
| Multicast MBS Session Remove List | O |  | Multicast MBS Session List 9.3.1.272 | The list of MBS Session ID that UE has left. | YES | reject |
| UE Multicast MRB to Be Setup at Modify List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Setup at Modify Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>MBS PTP Retransmission Tunnel Required | O |  | 9.3.2.10 |  | - |  |
| >>MBS PTP Forwarding Tunnel Required Information | O |  | MRB Progress Information 9.3.2.12 |  | - |  |
| UE Multicast MRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| SL DRX Cycle List |  | 0..1 |  |  | YES | ignore |
| >SL DRX Cycle Item IEs |  | 1 .. <maxnoofSLdestinations > |  |  | EACH | ignore |
| >>RX UE ID | M |  | BIT STRING (SIZE(24)) | Indicates the destination L2 ID of RX UE associated to this UE. | - |  |
| >>CHOICE SL DRX Information | M |  |  |  | - |  |
| >>>SL DRX Cycle |  |  |  |  |  |  |
| >>>>SL DRX Cycle Length | M |  | ENUMERATED (ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, ...) | Indicates the desired SL DRX cycle for RX UE associated to this UE. | - |  |
| >>>No SL DRX |  |  |  |  | - |  |
| >>>>SL DRX configuration indicator | M |  | ENUMERATED(release,...) |  | - |  |
| Management Based MDT PLMN Modification List | O |  | MDT PLMN Modification List 9.3.1.274 |  | YES | ignore |
| SDT Bearer Configuration Query Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| DAPS HO status | O |  | ENUMERATED(initiation, …) | This IE is used if DAPS HO is initiated. | YES | ignore |
| ServingCellMO List |  | 0..1 |  | For NCD-SSBs | YES | ignore |
| >ServingCellMO Item IEs |  | 1 .. <maxnoofServingCellMOs> |  |  | EACH | ignore |
| >>servingCellMO | M |  | INTEGER (1..64) |  | - |  |
| >>SSB frequency | M |  | INTEGER (0..3279165) | ARFCN | - |  |
| Uplink TxDirectCurrentMoreCarrierList Information | O |  | 9.3.1.284 |  | YES | ignore |
| CPAC MCG Information |  | 0..1 |  | This IE is used at the MN for MCG configuration as specified in TS 37.340 [7] for CPAC. | YES | ignore |
| >CPAC Trigger | M |  | ENUMERATED (CPAC-preparation, CPAC-executed, …) |  | - | - |
| >PSCell ID | M |  | NR CGI 9.3.1.12 | The PSCell corresponding to the included CG-Config IE at CPAC-preparation or the selected PSCell by the UE at CPAC-executed. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofServingCellMOs | Maximum number of ServingCellMOs for NCD-SSB per cell. Maximum value is 16 |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channel allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 64. |
| maxnoofSLdestinations | Maximum number of destination for NR sidelink communication, the maximum value is 32 |

| Condition | Explanation |
| --- | --- |
| ifCHOcancel | This IE may be present if the CHO Trigger IE is present and set to "CHO-cancel". |

#### 9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| DU To CU RRC Information | O |  | 9.3.1.26 |  | YES | reject |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used in this version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| DRB Modified List |  | 0..1 |  | The List of DRBs which are successfully modified. | YES | ignore |
| >DRB Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>LCID | O |  | 9.3.1.35 | LCID for the primary path or for the split secondary path for fallback to split bearer if PDCP duplication is applied. | - |  |
| >>DL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>RLC Status | O |  | 9.3.1.69 | Indicates the RLC has been re-established at the gNB-DU. | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used in this version of the specification. | YES | ignore |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.3.1.123 | Index to the currently fulfilled alternative QoS parameters set. | YES | ignore |
| SRB Failed to be Setup List |  | 0..1 |  | The List of SRBs which are failed to be established. | YES | ignore |
| >SRB Failed to be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Setup List |  | 0..1 |  | The List of DRBs which are failed to be setup. | YES | ignore |
| >DRB Failed to be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SCell Failed To Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell Failed to Setup Item |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| DRB Failed to be Modified List |  | 0..1 |  | The List of DRBs which are failed to be modified. | YES | ignore |
| >DRB Failed to be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Inactivity Monitoring Response | O |  | ENUMERATED (Not-supported, ...) |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | ignore |
| Associated SCell List | O |  | 9.3.1.77 |  | YES | ignore |
| SRB Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Setup Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| SRB Modified List |  | 0..1 |  |  | YES | ignore |
| >SRB Modified Item |  | 1 .. <maxnoofSRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>LCID | M |  | 9.3.1.35 | LCID for the primary path if PDCP duplication is applied | - |  |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| BH RLC Channel Setup List |  | 0..1 |  | The list of BH RLC channels which are successfully established. | YES | ignore |
| >BH RLC Channel Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Setup List |  | 0..1 |  | The list of BH RLC channels whose setup has failed. | YES | ignore |
| >BH RLC Channel Failed to be Setup Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| BH RLC Channel Modified List |  | 0..1 |  | The list of BH RLC channels which are successfully modified. | YES | ignore |
| >BH RLC Channel Modified Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| BH RLC Channel Failed to be Modified List |  | 0..1 |  | The list of BH RLC channels whose modification has failed. | YES | ignore |
| >BH RLC Channel Failed to be Modified Item |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | ignore |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Setup List |  | 0..1 |  | The List of SL DRBs which are successfully established. | YES | ignore |
| >SL DRB Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Modified List |  | 0..1 |  | The List of SL DRBs which are successfully modified. | YES | ignore |
| >SL DRB Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Failed To Setup List |  | 0..1 |  | The List of SL DRBs which are failed to be setup. | YES | ignore |
| >SL DRB Failed To Setup Item |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SL DRB Failed To be Modified List |  | 0..1 |  | The List of SL DRBs which are failed to be modified. | YES | ignore |
| >SL DRB Failed To be Modified Item |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | ignore |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>cause | O |  | 9.3.1.2 |  | - |  |
| Requested Target Cell ID | O |  | NR CGI 9.3.1.12 | Special Cell or PSCell ID in the CPAC MCG Information IE indicated in the UE CONTEXT MODIFICATION REQUEST message. | YES | reject |
| SCG Activation Status | O |  | 9.3.1.234 |  | YES | ignore |
| Uu RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  |  |  |
| Uu RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Uu RLC Channel Modified List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| Uu RLC Channel Failed to be Modified List |  | 0..1 |  |  | YES | ignore |
| >Uu RLC Channel Failed to be Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Failed to be Setup List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Setup Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| PC5 RLC Channel Modified List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Failed to be Modified List |  | 0..1 |  |  | YES | ignore |
| >PC5 RLC Channel Failed to be Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| SDT Bearer Configuration Info | O |  | 9.3.1.277 |  | YES | ignore |
| UE Multicast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>Multicast F1-U Context Reference CU | M |  | 9.3.2.13 |  | - |  |
| ServingCellMO-encoded-in-CGC List |  | 0..1 |  |  | YES | ignore |
| >ServingCellMO-encoded-in-CGC Item IEs |  | 1 .. <maxNrofBWPs> |  | The servingCellMO which has been encoded in CellGroupConfig IE. | - |  |
| >>servingCellMO | M |  | INTEGER (1..64) |  | - |  |
| >>BWP ID | M |  | INTEGER (0..4) |  | YES | ignore |
| Dedicated SI Delivery Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| Configured BWP List |  | 0..1 |  | This IE is present when the gNB-DU configures at least one BWP with NCD-SSB or without SSB. | YES | ignore |
| >Configured BWP Item IEs |  | 1 .. <maxNrofBWPs> |  |  | EACH | ignore |
| >>BWP-Id | M |  | INTEGER (0..4) | The IE is used to refer to one BWP. | - |  |
| >>BWP Location And Bandwidth | M |  | INTEGER (0..37949) | The IE type range is the same as the locationAndBandwidth IE in BWP IE as specified in TS 38.331 [8]. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxNrofBWPs | Maximum number of BWPs per serving cell, the maximum value is 8. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 32. |

#### 9.2.2.9	UE CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-DU to indicate a context modification failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Requested Target Cell ID | O |  | NR CGI 9.3.1.12 | Special Cell indicated in the UE CONTEXT MODIFICATION REQUEST message. | YES | reject |

#### 9.2.2.10	UE CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-DU to request the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| DU To CU RRC Information | O |  | 9.3.1.26 |  | YES | reject |
| DRB Required to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB Required to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DL UP TNL Information to be setup List |  | 0..1 |  |  | - |  |
| >>>DL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofDLUPTNLInformation> |  |  | - |  |
| >>>>DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>RLC Status | O |  | 9.3.1.69 | Indicates the RLC has been re-established at the gNB-DU. | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 | This IE is not used in this version of the specification. | YES | ignore |
| SRB Required to be Released List |  | 0..1 |  |  | YES | reject |
| >SRB Required to be Released List Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| DRB Required to be Released List |  | 0..1 |  |  | YES | reject |
| >DRB Required to be Released List Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| BH RLC Channel Required to be Released List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel Required to be Released Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| SL DRB Required to Be Modified List |  | 0..1 |  |  | YES | reject |
| >SL DRB Required to Be Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| SL DRB Required to be Released List |  | 0..1 |  |  | YES | reject |
| >SL DRB Required to be Release Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| Candidate Cells To Be Cancelled List |  | 0 .. <maxnoofCellsinCHO> |  |  | YES | reject |
| >Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| Uu RLC Channel Required to be Modified List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel Required to be Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| Uu RLC Channel Required to be Released List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel Required to be Released Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| PC5 RLC Channel Required to be Modified List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel Required to be Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| PC5 RLC Channel Required to be Released List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel Required to be Released Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - |  |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| UE Multicast MRB Required to Be Modified List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Required to Be Modified Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>MRB type reconfiguration | O |  | ENUMERATED (true, ...) |  | - |  |
| >>MRB Reconfigured RLC mode | C-ifMRBTypeReconf |  | MRB RLC Configuration 9.3.1.275 |  | - |  |
| >>Multicast F1-U Context Reference CU | O |  | 9.3.2.13 |  | YES | reject |
| UE Multicast MRB Required to Be Released List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Required to Be Released Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLUPTNLInformation | Maximum no. of DL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCellsinCHO | Maximum no. cells that can be prepared for a conditional mobility. Value is 8. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |
| maxnoofMRBsforUE | Maximum no. of multicast MRB allowed towards one UE, the maximum value is 32. |

| Condition | Explanation |
| --- | --- |
| ifMRBTypeReconf | This IE shall be present if the MRB Type Reconfiguration IE is present. |

#### 9.2.2.11	UE CONTEXT MODIFICATION CONFIRM

This message is sent by the gNB-CU to inform the gNB-DU the successful modification.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| DRB Modified List |  | 0..1 |  | The List of DRBs which are successfully modified. | YES | ignore |
| >DRB Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>>>DRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE is not used in this version of the specification. | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| SL DRB Modified List |  | 0..1 |  |  | YES | reject |
| >SL DRB Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| Uu RLC Channel Modified List |  | 0..1 |  |  | YES | reject |
| >Uu RLC Channel Modified Item IEs |  | 1 .. <maxnoofUuRLCChannels> |  |  | - |  |
| >>Uu RLC Channel ID | M |  | 9.3.1.266 |  | - |  |
| PC5 RLC Channel Modified List |  | 0..1 |  |  | YES | reject |
| >PC5 RLC Channel Modified Item IEs |  | 1 .. <maxnoofPC5RLCChannels> |  |  | - | - |
| >>PC5 RLC Channel ID | M |  | 9.3.1.265 |  | - |  |
| >>Remote UE Local ID | O |  | 9.3.1.267 |  | - |  |
| UE Multicast MRB Confirmed to Be Modified List |  | 0..1 |  |  | YES | reject |
| >UE Multicast MRB Confirmed to Be Modified Item IEs |  | 1 .. <maxnoofMRBsforUE> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 | MRB ID for the UE. | - |  |
| >>MBS PTP Retransmission Tunnel Required | O |  | 9.3.2.10 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of UL UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofUuRLCChannels | Maximum no. of Uu Relay RLC channels for L2 U2N relaying per Relay UE, the maximum value is 32. |
| maxnoofPC5RLCChannels | Maximum no. of PC5 Relay RLC channels allowed for L2 U2N relaying per Remote UE or Relay UE, the maximum value is 512. |

#### 9.2.2.11A	UE CONTEXT MODIFICATION REFUSE

This message is sent by the gNB-CU to indicate the UE context modification was unsuccessful.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.2.12	UE INACTIVITY NOTIFICATION

This message is sent by the gNB-DU to provide information about the UE activity to the gNB-CU.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DRB Activity List |  | 1 |  |  | YES | reject |
| >DRB Activity Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DRB Activity | O |  | ENUMERATED (Active, Not active) |  | - |  |
| SDT Termination Request | O |  | ENUMERATED (radio link problem, normal, …) | Indicate the reason of request for termination of the ongoing SDT. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |

#### 9.2.2.13		NOTIFY

This message is sent by the gNB-DU to notify the gNB-CU that the QoS for already established DRBs associated with notification control is not fulfilled any longer or it is fulfilled again.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DRB Notify List |  | 1 |  |  | YES | reject |
| >DRB Notify Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Notification Cause | M |  | ENUMERATED(Fulfilled, Not-Fulfilled, ...) |  | - |  |
| >>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters set Notify Index 9.3.1.124 | Index to the currently fulfilled alternative QoS parameters set. Value 0 indicates that NG-RAN cannot even fulfil the lowest alternative parameter set. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |

#### 9.2.2.14	ACCESS SUCCESS

This message is sent by the gNB-DU to inform the gNB-CU of which cell the UE has successfully accessed during conditional handover or conditional PSCell addition or conditional PSCell change.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 |  | YES | reject |

### 9.2.3	RRC Message Transfer messages

#### 9.2.3.1	INITIAL UL RRC MESSAGE TRANSFER

This message is sent by the gNB-DU to transfer the initial layer 3 message to the gNB-CU over the F1 interface.

Direction: gNB-DU gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 | NG-RAN Cell Global Identifier (NR CGI) | YES | reject |
| C-RNTI | M |  | 9.3.1.32 | C-RNTI allocated at the gNB-DU | YES | reject |
| RRC-Container | M |  | 9.3.1.6 | Includes the UL-CCCH-Message message or UL-CCCH1-Message message as defined in subclause 6.2 of TS 38.331 [8]. | YES | reject |
| DU to CU RRC Container | O |  | OCTET STRING | Includes the CellGroupConfig IE as defined in subclause 6.3.2 in TS 38.331 [8]. Required at least to carry SRB1 configuration. The ReconfigurationWithSync field is not included in the CellGroupConfig IE. | YES | reject |
| SUL Access Indication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | ignore |
| RAN UE ID | O |  | OCTET STRING (SIZE (8)) |  | YES | ignore |
| RRC-Container-RRCSetupComplete | O |  | 9.3.1.6 | Includes the UL-DCCH-Message message including the RRCSetupComplete message, as defined in subclause 6.2 of TS 38.331 [8]. | YES | ignore |
| NR RedCap UE Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| SDT Information | O |  | 9.3.1.262 |  | YES | ignore |
| Sidelink Relay Configuration | O |  | 9.3.1.264 |  | YES | ignore |

#### 9.2.3.2	DL RRC MESSAGE TRANSFER

This message is sent by the gNB-CU to transfer the layer 3 message to the gNB-DU over the F1 interface.

Direction: gNB-CU gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| old gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | reject |
| SRB ID | M |  | 9.3.1.7 |  | YES | reject |
| Execute Duplication | O |  | ENUMERATED (true, ...) |  | YES | ignore |
| RRC-Container | M |  | 9.3.1.6 | Includes the DL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8] encapsulated in a PDCP PDU, or the DL-CCCH-Message message as defined in subclause 6.2 of TS 38.331 [8]. | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| UE Context not retrievable | O |  | ENUMERATED (true, ...) |  | YES | reject |
| Redirected RRC message | O |  | RRC Container 9.3.1.6 | Includes the UL-CCCH-Message message as defined in subclause 6.2 of TS 38.331 [8]. | YES | reject |
| PLMN Assistance Info for Network Sharing | O |  | PLMN Identity 9.3.1.14 |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| SRB Mapping Info | O |  | Uu RLC Channel ID 9.3.1.266 | This IE contains the mapped Uu Relay RLC CH ID for the Remote UE’s SRB0 or SRB1. | YES | ignore |

#### 9.2.3.3	UL RRC MESSAGE TRANSFER

This message is sent by the gNB-DU to transfer the layer 3 message to the gNB-CU over the F1 interface.

Direction: gNB-DU gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SRB ID | M |  | 9.3.1.7 |  | YES | reject |
| RRC-Container | M |  | 9.3.1.6 | Includes the UL-DCCH-Message message as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. In case of CG-SDT, may include the UL-CCCH-Message message or UL-CCCH1-Message message as defined in subclause 6.2 of TS 38.331 [8]. | YES | reject |
| Selected PLMN ID | O |  | PLMN Identity 9.3.1.14 |  | YES | reject |
| New gNB-DU UE F1AP ID | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | YES | reject |

#### 9.2.3.4	RRC DELIVERY REPORT

This message is sent by the gNB-DU to inform the gNB-CU about the delivery status of DL RRC messages.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RRC Delivery Status | M |  | 9.3.1.71 |  | YES | ignore |
| SRB ID | M |  | 9.3.1.7 |  | YES | ignore |

### 9.2.4	Warning Message Transmission Messages

#### 9.2.4.1	WRITE-REPLACE WARNING REQUEST

This message is sent by the gNB-CU to request the start or overwrite of the broadcast of a warning message.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| PWS System Information | M |  | 9.3.1.58 | This IE includes the system information for public warning, as defined in TS 38.331 [8]. | YES | reject |
| Repetition Period | M |  | 9.3.1.59 |  | YES | reject |
| Number of Broadcasts Requested | M |  | 9.3.1.60 |  | YES | reject |
| Cell To Be Broadcast List |  | 0..1 |  |  | YES | reject |
| >Cell to Be Broadcast Item IEs |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

#### 9.2.4.2	WRITE-REPLACE WARNING RESPONSE

This message is sent by the gNB-DU to acknowledge the gNB-CU on the start or overwrite request of a warning message.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cell Broadcast Completed List |  | 0..1 |  |  | YES | reject |
| >Cell Broadcast Completed Item IEs |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Dedicated SI Delivery Needed UE List |  | 0..1 |  | List of UEs unable to receive system information from broadcast | YES | ignore |
| >Dedicated SI Delivery Needed UE Item |  | 1 .. <maxnoofUEIDs> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |

#### 9.2.4.3	PWS CANCEL REQUEST

This message is forwarded by the gNB-CU to gNB-DU to cancel an already ongoing broadcast of a warning message

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Number of Broadcasts Requested | M |  | 9.3.1.60 | This IE is not used in this version of the specification | YES | reject |
| Cell Broadcast To Be Cancelled List |  | 0..1 |  |  | YES | reject |
| >Cell Broadcast to Be Cancelled Item IEs |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Cancel-all Warning Messages Indicator | O |  |  | ENUMERATED (true, ...) | YES | reject |
| Notification Information | O |  |  | This IE is ignored If the Cancel-all Warning Messages Indicator IE is included. | YES | reject |
| >Message Identifier | M |  | 9.3.1.81 |  | - |  |
| >Serial Number | M |  | 9.3.1.82 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

#### 9.2.4.4	PWS CANCEL RESPONSE

This message is sent by the gNB-DU to indicate the list of warning areas where cancellation of the broadcast of the identified message was successful and unsuccessful.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cell Broadcast Cancelled List |  | 0..1 |  |  | YES | reject |
| >Cell Broadcast Cancelled Item IEs |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Number of Broadcasts | M |  | INTEGER (0..65535) | This IE is set to ‘0’ if valid results are not known or not available. It is set to 65535 if the counter results have overflowed. | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. of cells that can be served by a gNB-DU. Value is 512. |

#### 9.2.4.5	PWS RESTART INDICATION

This message is sent by the gNB-DU to inform the gNB-CU that PWS information for some or all cells of the gNB-DU are available if needed.

Direction: gNB-DU gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| NR CGI List for Restart List |  | 1 |  |  | YES | reject |
| >NR CGI List for Restart Item IEs |  | 1..<maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. of cells that can be served by a gNB-DU. Value is 512. |

#### 9.2.4.6	PWS FAILURE INDICATION

This message is sent by the gNB-DU to inform the gNB-CU that ongoing PWS operation for one or more cells of the gNB-DU has failed.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| PWS failed NR CGI List |  | 0..1 |  |  | YES | reject |
| >PWS failed NR CGI Item IEs |  | 1..<maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Number of Broadcasts | M |  | INTEGER (0..65535) | This IE is not used in the specification and is ignored. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. of cells that can be served by a gNB-DU. Value is 512. |

### 9.2.5	System Information messages

#### 9.2.5.1	SYSTEM INFORMATION DELIVERY COMMAND

This message is sent by the gNB-CU and is used to request the gNB-DU to broadcast the requested SystemInformation messages including the Other SI.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 | NR cell identifier | YES | reject |
| SIType List | M |  | 9.3.1.62 |  | YES | reject |
| Confirmed UE ID | M |  | gNB-DU UE F1AP ID 9.3.1.5 |  | YES | reject |

### 9.2.6	Paging messages

#### 9.2.6.1	PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.3.1.39 |  | YES | reject |
| CHOICE Paging Identity | M |  |  |  | YES | reject |
| >RAN UE Paging identity |  |  |  |  |  |  |
| >>RAN UE Paging identity | M |  | 9.3.1.43 |  | - |  |
| >CN UE paging identity |  |  |  |  |  |  |
| >>CN UE paging identity | M |  | 9.3.1.44 |  | - |  |
| Paging DRX | O |  | 9.3.1.40 | It is defined as the minimum between the RAN UE Paging DRX and CN UE Paging DRX | YES | ignore |
| Paging Priority | O |  | 9.3.1.41 |  | YES | ignore |
| Paging Cell List |  | 1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Last Used Cell Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >>PEI Subgrouping Support Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| Paging Origin | O |  | 9.3.1.79 |  | YES | ignore |
| RAN UE Paging DRX | O |  | Paging DRX 9.3.1.40 | This IE indicates the RAN paging cycle as defined in TS 38.304 [24]. | YES | ignore |
| CN UE Paging DRX | O |  | Paging DRX 9.3.1.40 | This IE indicates the UE specific paging cycle as defined in TS 38.304 [24]. | YES | ignore |
| NR Paging eDRX Information | O |  | 9.3.1.258 |  | YES | ignore |
| NR Paging eDRX Information for RRC INACTIVE | O |  | 9.3.1.259 |  | YES | ignore |
| Paging Cause | O |  | ENUMERATED(voice, …) | This IE indicates the paging cause is IMS voice, refer to TS 23.501[21]. | YES | ignore |
| PEIPS Assistance Information | O |  | 9.3.1.269 |  | YES | ignore |
| UE Paging Capability | O |  | 9.3.1.270 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.3.1.285 |  | YES | ignore |
| Hashed UE Identity Index Value | O |  | 9.3.1.286 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

### 9.2.7	Trace Messages

#### 9.2.7.1	TRACE START

This message is sent by the gNB-CU to initiate a trace session for a UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Trace Activation | M |  | 9.3.1.88 |  | YES | ignore |

#### 9.2.7.2	DEACTIVATE TRACE

This message is sent by the gNB-CU to deactivate a trace session.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Trace ID | M |  | OCTET STRING (SIZE(8)) | As per Trace ID in Trace Activation IE | YES | ignore |

#### 9.2.7.3	CELL TRAFFIC TRACE

This message is sent by the gNB-DU to to transfer trace specific information.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Trace ID | M |  | OCTET STRING (SIZE(8)) | This IE is composed of the following: Trace Reference defined in TS 32.422 [29] (leftmost 6 octets, with PLMN information encoded as in 9.3.1.14), and Trace Recording Session Reference defined in TS 32.422 [29] (last 2 octets). | YES | ignore |
| Trace Collection Entity IP Address | M |  | Transport Layer Address 9.3.2.3 | For File based Reporting. Defined in TS 32.422 [29]. Should be ignored if URI is present | YES | ignore |
| Privacy Indicator | O |  | ENUMERATED (Immediate MDT, Logged MDT, ...) |  | YES | ignore |
| Trace Collection Entity URI | O |  | URI 9.3.2.6 | For Streaming based Reporting. Defined in TS 32.422 [29] Replaces Trace Collection Entity IP Address if present | YES | ignore |

### 9.2.8	Radio Information Transfer messages

#### 9.2.8.1	DU-CU RADIO INFORMATION TRANSFER

This message is sent by a gNB-DU to a gNB-CU, to convey radio-related information.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| CHOICE DU-CU Radio Information Type | M |  |  |  | YES | ignore |
| >RIM |  |  |  |  |  |  |
| >>DU-CU RIM Information | M |  | 9.3.1.91 |  | - | - |

#### 9.2.8.2	CU-DU RADIO INFORMATION TRANSFER

This message is sent by a gNB-CU to a gNB-DU, to convey radio-related information.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| CHOICE CU-DU Radio Information Type | M |  |  |  | YES | ignore |
| >RIM |  |  |  |  |  |  |
| >>CU-DU RIM Information | M |  | 9.3.1.92 |  | - | - |

### 9.2.9	IAB messages

#### 9.2.9.1	BAP MAPPING CONFIGURATION

This message is sent by the gNB-CU to provide the backhaul routing information and/or traffic mapping information to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| BH Routing Information Added List |  | 0...1 |  |  | YES | ignore |
| >BH Routing Information Added List Item |  | 1.. <maxnoofRoutingEntries> |  |  | EACH | ignore |
| >>BAP Routing ID | M |  | 9.3.1.110 |  | - |  |
| >>Next-Hop BAP Address | M |  | 9.3.1.111 | Indicates the BAP address of the next hop IAB-node or IAB-donor-DU. | - |  |
| >>Non-F1-Terminating IAB-donor Topology Indicator | O |  | ENUMERATED(true, ...) | If present, indicates that the routing entry applies to the non-F1-terminating IAB-donor topology of the boundary IAB-node. | YES | ignore |
| BH Routing Information Removed List |  | 0...1 |  |  | YES | ignore |
| >BH Routing Information Removed List Item |  | 1.. <maxnoofRoutingEntries> |  |  | EACH | ignore |
| >>BAP Routing ID | M |  | 9.3.1.110 |  | - |  |
| Traffic Mapping Information | O |  | 9.3.1.95 |  | YES | ignore |
| Buffer Size Threshold | O |  | INTEGER (0..224-1) | The buffer size threshold (in kilobytes) for DL local rerouting, triggered by hop-by-hop flow control feedback. | YES | ignore |
| BAP Header Rewriting Added List |  | 0...1 |  |  | YES | ignore |
| >BAP Header Rewriting Added List Item |  | 1.. <maxnoofRoutingEntries> |  |  | EACH | ignore |
| >>Ingress BAP Routing ID | M |  | 9.3.1.110 |  | - |  |
| >>Egress BAP Routing ID | M |  | 9.3.1.110 |  | - |  |
| >>Non-F1-terminating IAB-donor Topology Indicator | O |  | ENUMERATED (true, …) | If present, indicates that the egress BAP Routing ID in the present BAP header rewriting entry pertains to the non-F1-terminating IAB-donor topology of the boundary IAB-node. | - |  |
| Re-routing Enable Indicator | O |  | ENUMERATED (true, false, …) |  | YES | ignore |
| BAP Header Rewriting Removed List |  | 0...1 |  |  | YES | ignore |
| >BAP Header Rewriting Removed List Item |  | 1.. <maxnoofRoutingEntries> |  |  | EACH | ignore |
| >>Ingress BAP Routing ID | M |  | 9.3.1.110 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRoutingEntries | Maximum no. of routing entries, the maximum value is 1024. |

#### 9.2.9.2	BAP MAPPING CONFIGURATION ACKNOWLEDGE

This message is sent by the gNB-DU as a response to a BAP MAPPING CONFIGURATION message.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.9.2A	BAP MAPPING CONFIGURATION FAILURE

This message is sent by the gNB-DU to indicate a BAP Mapping Configuration Update failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to wait | O |  | 9.3.1.13 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.9.3	GNB-DU RESOURCE CONFIGURATION

This message is sent by the gNB-CU to provide the resource configuration for an gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Activated Cells to Be Updated List |  | 0..1 |  | List of activated cells served by the IAB-DU or the IAB-donor-DU whose resource configuration is updated | YES | reject |
| >Activated Cells To Be Updated List Item |  | 1 .. <maxnoofServedCellsIAB> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>IAB-DU Cell Resource Configuration-Mode-Info | M |  | 9.3.1.279 | In the current version of this specification, for FDD, this IE only contains the gNB-DU Cell Resource Configuration-FDD-UL IE and the gNB-DU Cell Resource Configuration-FDD-DL IE, for TDD, this IE only contains the gNB-DU Cell Resource Configuration-TDD IE | - |  |
| Child-Nodes List |  | 0..1 |  | List of child IAB-nodes served by the IAB-DU or IAB-donor-DU. | YES | reject |
| >Child-Nodes List Item |  | 1 .. <maxnoofChildIABNodes> |  |  | EACH | reject |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 | Identifier of a descendant node IAB-MT at the IAB-donor-CU. | YES | reject |
| >>gNB-DU UE F1AP ID | M |  | 9.3.1.5 | Identifier of a child-node IAB-MT at an IAB-DU or IAB-donor-DU. | YES | reject |
| >>Child-Node Cells List |  | 0..1 |  | List of cells served by the child-node IAB-DU whose resource configuration is updated. | YES | reject |
| >>>Child-Node Cells List Item |  | 1 .. <maxnoofServedCellsIAB > |  |  | EACH | reject |
| >>>>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>>>IAB-DU Cell Resource Configuration-Mode-Info | O |  | 9.3.1.279 |  | - |  |
| >>>>IAB STC Info | O |  | 9.3.1.109 | STC configuration of child-node IAB-DU’s cell. | - |  |
| >>>>RACH Config Common | O |  | OCTET STRING | Includes the rach-ConfigCommon contained in the BWP-UplinkCommon IE as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>>>RACH Config Common IAB | O |  | OCTET STRING | Includes the IAB-specific rach-ConfigCommonIAB contained in the BWP-UplinkCommon IE as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>>>CSI-RS Configuration | O |  | OCTET STRING | Includes the NZP-CSI-RS-Resource IE as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>>>SR Configuration | O |  | OCTET STRING | Includes the SchedulingRequestResourceConfig IE as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>>>PDCCH Configuration SIB1 | O |  | OCTET STRING | Includes the PDCCH-ConfigSIB1 IE as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>>>SCS Common | O |  | OCTET STRING | Includes the subCarrierSpacingCommon contained in the MIB message as defined in subclause 6.2.2 of TS 38.331 [8]. | - |  |
| >>>>Multiplexing Info | O |  | 9.3.1.108 | Contains information on multiplexing with cells configured for co-located IAB-MT. | - |  |
| Neighbour-Node Cells List |  | 0..1 |  | List of neighbor node cells. | YES | reject |
| >Neighbour-Node Cells List Item |  | 1 .. < maxnoofNeighbourNodeCellsIAB> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>gNB-CU UE F1AP ID | O |  | 9.3.1.4 | Identifier of a child-node IAB-MT at an IAB-donor-CU. | - |  |
| >>gNB-DU UE F1AP ID | O |  | 9.3.1.5 | Identifier of a child-node IAB-MT at an IAB-DU or IAB-donor-DU. | - |  |
| >>Peer Parent-Node Indicator | O |  | ENUMERATED (true, …) | Indicates if the cell belongs to the peer parent IAB-node of the dual connected IAB-node. | - |  |
| >>IAB-DU Cell Resource Configuration-Mode-Info | O |  | 9.3.1.279 |  | - |  |
| >>IAB STC Info | O |  | 9.3.1.109 | STC configuration of peer parent-node IAB-DU’s cell. | - |  |
| >>RACH Config Common | O |  | OCTET STRING | Common RACH Configuration of peer parent node IAB-DU’s cell. Includes the rach-ConfigCommon contained in the BWP-UplinkCommon IE as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>RACH Config Common IAB | O |  | OCTET STRING | IAB specific common RACH Configuration of peer parent node IAB-DU’s cell. Includes the IAB-specific rach-ConfigCommonIAB contained in the BWP-UplinkCommon IE as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>CSI-RS Configuration | O |  | OCTET STRING | CSI-RS configuration of peer parent node IAB-DU’s cell. Includes the NZP-CSI-RS-Resource as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>SR Configuration | O |  | OCTET STRING | SR configuration of peer parent node IAB-DU’s cell. Includes the SchedulingRequestResourceConfig IE as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>PDCCH Configuration SIB1 | O |  | OCTET STRING | PDCCH configuration SIB1 of peer parent node IAB-DU’s cell. Includes the PDCCH-ConfigSIB1 IE as defined in subclause 6.3.2 of TS 38.331 [8]. | - |  |
| >>SCS Common | O |  | OCTET STRING | SCS Common of peer parent node IAB-DU’s cell. Includes the subCarrierSpacingCommon contained in the MIB message as defined in subclause 6.2.2 of TS 38.331 [8]. | - |  |
| Serving Cells List |  | 0..1 |  | List of serving cells of the co-located IAB-MT. | YES | reject |
| >Serving Cells List Item |  | 1 .. < maxnoofServingCells > |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>CHOICE IAB-MT Cell NA Resource Configuration-Mode-Info | O |  |  |  | - |  |
| >>>FDD |  |  |  |  |  |  |
| >>>>FDD Info |  | 1 |  |  | - |  |
| >>>>>gNB-DU Cell NA Resource Configuration-FDD-UL | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains FDD UL NA resource configuration of parent IAB-node’s cell for the co-located IAB-MT. | - |  |
| >>>>>gNB-DU Cell NA Resource Configuration-FDD-DL | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains FDD DL NA resource configuration of parent IAB-node’s cell for the co-located IAB-MT. | - |  |
| >>>>>UL Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>>>UL Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| >>>>>UL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the UL Transmission Bandwidth IE shall be ignored. | - |  |
| >>>>>DL Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>>>DL Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| >>>>>DL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the DL Transmission Bandwidth IE shall be ignored. | - |  |
| >>>TDD |  |  |  |  | - |  |
| >>>>TDD Info |  | 1 |  |  | - |  |
| >>>>>gNB-DU Cell NA Resource Configuration-TDD | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains TDD NA resource configuration of parent IAB-node’s cell for the co-located IAB-MT. | - |  |
| >>>>>NR Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>>>Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| >>>>>Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the Transmission Bandwidth IE shall be ignored. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofChildIABNodes | Maximum number of child nodes served by an IAB-DU or IAB-donor-DU. Value is 1024. |
| maxnoofServedCellsIAB | Maximum number of cells served by an IAB-DU or IAB-donor-DU. Value is 512. |
| maxnoofNeighbourNodeCellsIAB | Maximum no. of neighbour cells. Value is 1024. |
| MaxnoofServingCells | Maximum no. of serving cells for IAB-MT. Value is 32 |

#### 9.2.9.4	GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE

This message is sent by the gNB-DU to acknowledge the reception of an GNB-DU RESOURCE CONFIGURATION message.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.9.4A	GNB-DU RESOURCE CONFIGURATION FAILURE

This message is sent by the gNB-DU to indicate a gNB-DU Resource Configuration Update failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to wait | O |  | 9.3.1.13 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.9.5	IAB TNL ADDRESS REQUEST

This message is sent by the gNB-CU to request the allocation of IP addresses for IAB-node(s).

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| IAB IPv4 Addresses Requested | O |  | IAB TNL Addresses Requested 9.3.1.101 |  | YES | reject |
| CHOICE IAB IPv6 Request Type | O |  |  |  | YES | reject |
| >IPv6 Address |  |  |  |  | - |  |
| >>IAB IPv6 Addresses Requested | M |  | IAB TNL Addresses Requested 9.3.1.101 |  | - |  |
| >IPv6 Prefix |  |  |  |  | - |  |
| >>IAB IPv6 Address Prefixes Requested | M |  | IAB TNL Addresses Requested 9.3.1.101 |  | - |  |
| IAB TNL Addresses To Remove List |  | 0..1 |  |  | YES | reject |
| >IAB TNL Addresses To Remove Item |  | 1..<maxnoofTLAsIAB> |  |  | EACH | reject |
| >>IAB TNL Address | M |  | 9.3.1.102 |  | - |  |
| IAB TNL Address Exception | O |  | 9.3.1.229 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofTLAsIAB | Maximum no. of individual IPv4/IPv6 addresses or IPv6 address prefixes that can be allocated in one procedure execution. The value is 1024. |

#### 9.2.9.6	IAB TNL ADDRESS RESPONSE

This message is sent by the gNB-DU to indicate the TNL addresses allocated to IAB-node(s).

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| IAB Allocated TNL Address List |  | 1 |  |  | YES | reject |
| >IAB Allocated TNL Address Item |  | 1..<maxnoofTLAsIAB> |  |  | EACH | reject |
| >>IAB TNL Address | M |  | 9.3.1.102 |  | - |  |
| >>IAB TNL Address Usage | O |  | ENUMERATED (F1-C, F1-U, Non-F1, …) | The usage of the allocated IPv4 or IPv6 address or IPv6 address prefix. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTLAsIAB | Maximum no. of IPv6 addresses or IPv6 address prefixes and/or individual IPv4 addresses that can be allocated in one procedure execution. The value is 1024. |

#### 9.2.9.6A	IAB TNL ADDRESS FAILURE

This message is sent by the gNB-DU to indicate an IAB TNL Address Allocation failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to wait | O |  | 9.3.1.13 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.9.7	IAB UP CONFIGURATION UPDATE REQUEST

This message is sent by the gNB-CU to provide the updated UL BH Information or the updated UL UP TNL Information/Address to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| UL UP TNL Information to Update List |  | 0..1 |  |  | YES | ignore |
| >UL UP TNL Information to Update List Item IEs |  | 1.. < maxnoofULUPTNLInformationforIAB> |  |  | EACH | ignore |
| >>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | This field indicates the UL UP TNL Information used before configuration update. | - |  |
| >>New UL UP TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | If present, this field indicates the new UL UP TNL Information used after configuration update. | - |  |
| >>BH Information | M |  | 9.3.1.114 |  | - |  |
| UL UP TNL Address to Update List |  | 0..1 |  |  | YES | ignore |
| >UL UP TNL Address to Update List Item IEs |  | 1.. < maxnoofUPTNLAddresses> |  |  | EACH | ignore |
| >>Old TNL Address | M |  | Transport Layer Address 9.3.2.3 | The old UL UP Transport Layer Address of gNB-CU used for UL F1-U GTP Tunnel before the configuration update. | - |  |
| >>New TNL Address | M |  | Transport Layer Address 9.3.2.3 | The corresponding new UL UP Transport Layer Address that replaces the old one. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofULUPTNLInformationforIAB | Maximum no. of UL UP TNL Information allowed towards one IAB node, the maximum value is 32768. |
| maxnoofUPTNLAddresses | Maximum no. of TNL addresses for F1-U. Value is 8. |

#### 9.2.9.8	IAB UP CONFIGURATION UPDATE RESPONSE

This message is sent by the gNB-DU to provide the updated TNL address(es) of the DL F1-U GTP tunnels to the gNB-CU.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| DL UP TNL Address to Update List |  | 0..1 |  |  | YES | ignore |
| >DL UP TNL Address to Update List Item IEs |  | 1.. < maxnoofUPTNLAddresses> |  |  | EACH | ignore |
| >>Old TNL Address | M |  | Transport Layer Address 9.3.2.3 | The old DL UP Transport Layer Address of gNB-DU used for DL F1-U GTP tunnel before the configuration update. | - |  |
| >>New TNL Address | M |  | Transport Layer Address 9.3.2.3 | The corresponding new Transport Layer Address used to replace the old one. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUPTNLAddresses | Maximum no. of TNL addresses for F1-U. Value is 8. |

#### 9.2.9.9	IAB UP CONFIGURATION UPDATE FAILURE

This message is sent by the gNB-DU to indicate an IAB UP Configuration Update failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Time to wait | O |  | 9.3.1.13 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

### 9.2.10	Self Optimisation Support Messages

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RACH Report Information List |  | 0..1 |  |  | YES | ignore |
| >RACH Report Information Item |  | 1 .. <maxnoofRACHReports> |  |  | - |  |
| >>RACH Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |
| maxnoofRLFReports | Maximum no. of RLF Reports, the maximum value is 64. |
| maxnoofSuccessfulHOReports | Maximum no. of Successful HO Reports, the maximum value is 64. |

### 9.2.11	Reference Time Information Reporting messages

#### 9.2.11.1	REFERENCE TIME INFORMATION REPORTING CONTROL

This message is sent by the gNB-CU and is used to request the gNB-DU to deliver the accurate reference time information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Reporting Request Type | M |  | 9.3.1.147 |  | YES | reject |

#### 9.2.11.2	REFERENCE TIME INFORMATION REPORT

This message is sent by the gNB-DU and is used to report the accurate reference time information to the gNB-CU.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | ignore |
| Time Reference Information | M |  | 9.3.1.148 |  | YES | ignore |

### 9.2.12	Messages for Positioning Procedures

#### 9.2.12.1	POSITIONING ASSISTANCE INFORMATION CONTROL

This message is sent by the gNB-CU to transfer positioning assistance information.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Positioning Assistance Information | O |  | OCTET STRING | Contains the Assistance Information IE as defined in TS 38.455 [37]. | YES | reject |
| Broadcast | O |  | ENUMERATED (start, stop, ...) |  | YES | reject |
| Positioning Broadcast Cells | O |  | 9.3.1.191 | The cell(s) that are requested to broadcast posSIB(s) according to the Positioning Assistance Information IE. | YES | reject |
| Routing ID | O |  | OCTET STRING |  | YES | reject |

#### 9.2.12.2	POSITIONING ASSISTANCE INFORMATION FEEDBACK

This message is sent by the gNB-DU to give feedback on positioning assistance information broadcasting.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Positioning Assistance Information Failure List | O |  | OCTET STRING | Contains the Assistance Information IE as defined in TS 38.455 [37]. | YES | reject |
| Positioning Broadcast Cells | O |  | 9.3.1.191 | The cells associated to the feedback provided in the Positioning Assistance Information Failure List IE. | YES | reject |
| Routing ID | O |  | OCTET STRING |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.12.3	POSITIONING MEASUREMENT REQUEST

This message is sent by the gNB-CU to request the gNB-DU to configure a positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | - |  |
| >>TRP ID | M |  | 9.3.1.197 |  | - |  |
| >>Search Window Information | O |  | 9.3.1.204 |  | - |  |
| >>NR CGI | O |  | 9.3.1.12 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.3.1.238 |  | YES | ignore |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Positioning Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, …) |  | YES | reject |
| Positioning Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, …, 20480ms, 40960ms, extended) | The codepoint 120ms, 240ms, 480ms, 1024ms, 2048ms, 1min, 6min, 12min, and 30min are not applicable. | YES | reject |
| Positioning Measurement Quantities |  | 1 |  |  | YES | reject |
| >Positioning Measurement Quantities Item |  | 1..<maxnoofPosMeas> |  |  | EACH |  |
| >>Positioning Measurement Type | M |  | ENUMERATED (gNB RX-TX, UL-SRS-RSRP, UL AoA, UL RTOA, …, Multiple UL AoA, UL SRS-RSRPP) |  | - | - |
| >>Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | TS 38.133 [38] | - |  |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.3.1.192 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true, ...) | This IE is ignored when the Measurement characteristics Request Indicator IE is included. | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Measurement Periodicity Extended | C-ifMeasPerExt |  | ENUMERATED (160ms, 320ms, 1280ms, 2560ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |
| Response Time | O |  | 9.3.1.242 | This IE is ignored when the Positioning Report Characteristics IE is set to “Periodic”. | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.3.1.254 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4,…) |  | YES | ignore |
| Positioning Measurement Amount | O |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | This IE is ignored if the Positioning Report Characteristics IE is set to ‘OnDemand’. Value 0 represents an infinite number of periodic reporting. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPosMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one measurement message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Positioning Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Positioning Measurement Periodicity IE is set to the value "extended". |

#### 9.2.12.4	POSITIONING MEASUREMENT RESPONSE

This message is sent by the gNB-DU to report positioning measurements for the target UE.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Positioning Measurement Result List |  | 0..1 |  |  | YES | reject |
| >Positioning Measurement Result List Item |  | 1..< maxnoofMeasTRPs> |  |  | - |  |
| >>Positioning Measurement Result | M |  | 9.3.1.166 |  | - | - |
| >>TRP ID | M |  | 9.3.1.197 |  | - |  |
| >>NR CGI | O |  | 9.3.1.12 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRP measurements that can be included within one message. Value is 64. |

#### 9.2.12.5	POSITIONING MEASUREMENT FAILURE

This message is sent by the gNB-DU to report measurement failure.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.12.6	POSITIONING MEASUREMENT REPORT

This message is sent by the gNB-DU to report positioning measurements for the target UE.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Positioning Measurement Result List |  | 1 |  |  | YES | reject |
| >Positioning Measurement Result List Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH |  |
| >>Positioning Measurement Result | M |  | 9.3.1.166 |  | - | - |
| >>TRP ID | M |  | 9.3.1.197 |  | - | - |
| >>NR CGI | O |  | 9.3.1.12 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRP measurements that can be included within one message. Value is 64. |

#### 9.2.12.7	POSITIONING MEASUREMENT ABORT

This message is sent by the gNB-CU to request the gNB-DU to abort a positioning measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1.. 65536,…) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536,…) |  | YES | reject |

#### 9.2.12.8	POSITIONING MEASUREMENT FAILURE INDICATION

This message is sent by the gNB-DU to indicate that the previously requested positioning measurements can no longer be reported.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1.. 65536,…) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536,…) |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.12.9	POSITIONING MEASUREMENT UPDATE

This message is sent by the gNB-CU to update a previously configured measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| LMF Measurement ID | M |  | INTEGER (1..65536,…) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536,…) |  | YES | reject |
| SRS Configuration | O |  | 9.3.1.192 |  | YES | ignore |
| TRP Measurement Update List |  | 0..1 |  |  | YES | reject |
| >TRP Measurement Update Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.3.1.197 |  | - |  |
| >>AoA Search Window Information | O |  | UL-AoA Assistance Information 9.3.1.238 |  | - |  |
| >>Number of TRP Rx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| >>Number of TRP RxTx TEGs | O |  | ENUMERATED (2, 3, 4, 6, 8, …) |  | YES | ignore |
| Measurement Characteristics Request Indicator | O |  | 9.3.1.254 |  | YES | ignore |
| Measurement Time Occasion | O |  | ENUMERATED (o1, o4, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 64. |

#### 9.2.12.10	TRP INFORMATION REQUEST

This message is sent by a gNB-CU to request information for TRPs hosted by a gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| TRP list |  | 0..1 |  |  | YES | ignore |
| >TRP list Item |  | 1..<maxnoofTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.3.1.197 |  | - |  |
| TRP Information Type List |  | 1 |  |  | YES | reject |
| >TRP Information Type Item |  | 1 .. <maxnoofTRPInfoTypes> |  |  | EACH | reject |
| >>TRP Information Type Item | M |  | ENUMERATED (nr pci, ng-ran cgi, nr arfcn, prs config, ssb config, sfn init time, spatial direction info, geo-coordinates, …, trp type, on-demand prs, trp Tx teg, beam antenna info) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |
| maxnoofTRPs | Maximum no. of TRPs in a gNB. Value is 65535. |

#### 9.2.12.11	TRP INFORMATION RESPONSE

This message is sent by a gNB-DU to convey TRP information to a gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| TRP Information List |  | 1 |  |  | YES | ignore |
| >TRP Information Item |  | 1 .. <maxnoofTRPs> |  |  | EACH | ignore |
| >>TRP Information | M |  | 9.3.1.176 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPs | Maximum no. of TRPs in a gNB-DU. Value is 65535. |

#### 9.2.12.12	TRP INFORMATION FAILURE

This message is sent by a gNB-DU node to indicate that the requested TRP information cannot be provided to a gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.12.13	POSITIONING INFORMATION REQUEST

This message is sent by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals for uplink positioning measurement and also to retrieve the SRS configuration from the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Requested SRS Transmission Characteristics | O |  | 9.3.1.175 |  | YES | ignore |
| UE Reporting Information | O |  | 9.3.1.255 |  | YES | ignore |
| SRS Positioning INACTIVE Query Indication | O |  | ENUMERATED (true, ...) | Applicable only if the Requested SRS Transmission Characteristics IE is present | YES | ignore |

#### 9.2.12.14	POSITIONING INFORMATION RESPONSE

This message is sent by the gNB-DU to provide the configured SRS information to the gNB-CU.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SRS Configuration | O |  | 9.3.1.192 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| SRS-PosRRC-InactiveConfig | O |  | OCTET STRING | Includes the SRS-PosRRC-InactiveConfig IE as defined in TS 38.331 [8] | YES | ignore |

#### 9.2.12.15	POSITIONING INFORMATION FAILURE

This message is sent by the gNB-DU to indicate that no SRS transmissions could be configured for the UE for uplink positioning measurement.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.12.16	POSITIONING ACTIVATION REQUEST

This message is sent by the gNB-CU to cause the gNB-DU to activate/trigger UL SRS transmission by the UE.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE SRS type | M |  |  |  | YES | reject |
| >Semi-persistent |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.3.1.180 |  | - | - |
| >>SRS Spatial Relation | O |  | Spatial Relation Information 9.3.1.181 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | - | - |
| >>Spatial Relation Information per SRS Resource | O |  | 9.3.1.210 |  | YES | ignore |
| >Aperiodic |  |  |  |  |  |  |
| >>Aperiodic | M |  | ENUMERATED (true, …) |  | - | - |
| >>SRS Resource Trigger | O |  | 9.3.1.182 |  | - | - |
| Activation Time | O |  | Relative Time 1900 9.3.1.183 | Indicates the start time when the SRS activation is requested | YES | ignore |

#### 9.2.12.17	POSITIONING ACTIVATION RESPONSE

This message is sent by the gNB-DU to confirm successful UL SRS activation in the UE.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.12.18	POSITIONING ACTIVATION FAILURE

This message is sent by the gNB-DU to indicate that activation of UL SRS transmission in the UE was unsuccessful.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.12.19	POSITIONING DEACTIVATION

This message is sent by the gNB-CU to cause the NG RAN node to deactivate UL SRS transmission or release all the transmission by the UE.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE Abort Transmission | M |  |  |  | YES | ignore |
| >SRS Resource Set ID deactivation |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.3.1.180 |  | - |  |
| >Release ALL |  |  | NULL |  |  |  |

#### 9.2.12.20	E-CID MEASUREMENT INITIATION REQUEST

This message is sent by gNB-CU to initiate E-CID measurements.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| LMF UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| E-CID Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, …) |  | YES | reject |
| E-CID Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, …, 20480ms, 40960ms, extended) | The codepoint “extended” is not applicable. This IE is not applicable to NR Angle of Arrival. | YES | reject |
| E-CID Measurement Quantities |  | 1 .. <maxnoofMeasE-CID> |  |  | EACH | reject |
| >E-CID Measurement Quantities Item | M |  | ENUMERATED (Default, NR Angle of Arrival, …, NR Timing Advance) | If “Default” is the only requested measurement quantity, it indicates that the Measured Results List IE need not be included in response or reporting messages. | - |  |
| Measurement Periodicity NR-AoA | C- ifReportCharacteristicsPeriodicAndMeasQuantityItemAoA |  | ENUMERATED (160ms, 320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 40960ms, 61440ms, 81920ms, 368640ms, 737280ms, 1843200ms, …) |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasE-CID | Maximum no. of E-CID measured quantities that can be configured and reported with one message. Value is 64. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the E-CID Report Characteristics IE is set to the value "Periodic". |
| ifReportCharacteristicsPeriodicAndMeasQuantityItemAoA | This IE shall be present if the E-CID Report Characteristics IE is set to the value "Periodic" and the E-CID Measurement Quantities Item IE is set to the value "NR Angle of Arrival". |

#### 9.2.12.21	E-CID MEASUREMENT INITIATION RESPONSE

This message is sent by gNB-DU to indicate that the requested E-CID measurement is successfully initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| LMF UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| E-CID Measurement Result | O |  | 9.3.1.199 |  | YES | ignore |
| Cell Portion ID | O |  | 9.3.1.200 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.12.22	E-CID MEASUREMENT INITIATION FAILURE

This message is sent by gNB-DU to indicate that the requested E-CID measurement cannot be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| LMF UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.12.23	E-CID MEASUREMENT FAILURE INDICATION

This message is sent by gNB-DU to indicate that the previously requested E-CID measurement can no longer be reported.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| LMF UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.12.24	E-CID MEASUREMENT REPORT

This message is sent by gNB-DU to report the results of the requested E-CID measurement.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| LMF UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| E-CID Measurement Result | M |  | 9.3.1.199 |  | YES | ignore |
| Cell Portion ID | O |  | 9.3.1.200 |  | YES | ignore |

#### 9.2.12.25	E-CID MEASUREMENT TERMINATION COMMAND

This message is sent by the gNB-CU to terminate the requested E-CID measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| LMF UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |
| RAN UE Measurement ID | M |  | INTEGER (1.. 256, …) |  | YES | reject |

#### 9.2.12.26	POSITIONING INFORMATION UPDATE

This message is sent by the gNB-DU to indicate that a change in the SRS configuration has occurred.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SRS configuration | O |  | 9.3.1.192 |  | YES | ignore |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 |  | YES | ignore |

#### 9.2.12.27	PRS CONFIGURATION REQUEST

This message is sent by a gNB-CU to request a gNB-DU to configure or update PRS transmissions.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| PRS Configuration Request Type | M |  | ENUMERATED (configure, off, …) |  | YES | reject |
| PRS TRP List |  | 1 |  |  | YES | ignore |
| >PRS TRP Item |  | 1 .. <maxnoofTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.3.1.197 |  | - |  |
| >>Requested DL PRS Transmission Characteristics | C-ifConf |  | 9.3.1.235 |  | - |  |
| >>PRS Transmission Off Information | C-ifOff |  | 9.3.1.237 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPs | Maximum no. of TRPs in a gNB-DU Value is 65535 |

| Condition | Explanation |
| --- | --- |
| ifConf | This IE shall be present if the PRS Configuration Request Type IE is set to the value "configure". |
| ifOff | This IE shall be present if the PRS Configuration Request Type IE is set to the value "off". |

#### 9.2.12.28	PRS CONFIGURATION RESPONSE

This message is sent by a gNB-DU to acknowledge configuring or updating the PRS transmissions.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| PRS Transmission TRP List |  | 0..1 |  |  | YES | ignore |
| >PRS Transmission TRP Item |  | 1 .. <maxnoofTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.3.1.197 |  | - |  |
| >>PRS Configuration | M |  | 9.3.1.117 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPs | Maximum no. of TRPs in a gNB-DU Value is 65535. |

#### 9.2.12.29	PRS CONFIGURATION FAILURE

This message is sent by the gNB-DU to indicate that it cannot configure any PRS transmission.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.12.30	MEASUREMENT PRECONFIGURATION REQUIRED

This message is sent by a gNB-CU to provide the PRS configuration information of multiple TRPs to a gNB-DU and request to configure measurement gap or PRS processing window of the UE.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| TRP PRS Information List |  | 1 |  |  | YES | ignore |
| >TRP PRS Information Item |  | 1 .. <maxnoTRPs> |  |  | EACH | ignore |
| >>TRP ID | M |  | 9.3.1.197 |  | - |  |
| >>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>NR CGI | O |  | 9.3.1.12 |  | - |  |
| >>PRS Configuration | M |  | 9.3.1.177 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPs | Maximum no. of TRPs for on-demand PRS in a gNB-DU Value is 256 |

#### 9.2.12.31	MEASUREMENT PRECONFIGURATION CONFIRM

This message is sent by an gNB-DU to gNB-CU to confirm successful configuration of measurement gap or PRS processing window of the UE.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| PosMeasGapPreConfigList |  | 0..1 |  |  | YES | ignore |
| >PosMeasGapPreConfigToAddModList | O |  | OCTET STRING | Includes the PosMeasGapPreConfigToAddModList IE as defined in TS 38.331 [8] | YES | Ignore |
| >PosMeasGapPreConfigToReleaseList | O |  | OCTET STRING | Includes the PosMeasGapPreConfigToReleaseList IE as defined in TS 38.331 [8] | YES | Ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.12.32	MEASUREMENT PRECONFIGURATION REFUSE

This message is sent by gNB-DU to indicate configuration of measurement gap or PRS processing window of the UE was unsuccessful.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.12.33	MEASUREMENT ACTIVATION

This message is sent by the gNB-CU to request the gNB-DU to activate or deactivate the preconfigured measurement gap or PRS processing window for the UE.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Request Type | M |  | ENUMERATED (activate, deactivate, …) |  | YES | reject |
| PRS Measurement Info List |  | 0..1 |  |  | YES | ignore |
| >PRS Measurement Info Item |  | 1 .. < maxFreqLayers> |  |  | - |  |
| >>Point A | M |  | INTEGER (0..3279165) |  | - |  |
| >>MeasPRS Periodicity | M |  | ENUMERATED (ms20, ms40, ms80, ms160, …) | Measurement gap periodicity in units of ms | - |  |
| >>MeasPRS Offset | M |  | INTEGER (0..159, …) | Measurement gap offset in units of subframes | - |  |
| >>Measurement PRS Length | M |  | ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20} |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxFreqLayers | Maximum no. of frequency layers. Value is 4 |

#### 9.2.12.34	POSITIONING SYSTEM INFORMATION DELIVERY COMMAND

This message is sent by the gNB-CU and is used to request the gNB-DU to broadcast the indicated positioning SI message.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 | NR cell identifier | YES | reject |
| PosSIType List | M |  | 9.3.1.278 |  | YES | reject |
| Confirmed UE ID | M |  | gNB-DU UE F1AP ID 9.3.1.5 |  | YES | reject |

### 9.2.13	Broadcast Context Management messages

#### 9.2.13.1	BROADCAST CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of an MBS session context for a broadcast session, and establish an MBS-associated logical F1-connection.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| MBS CU to DU RRC Information | M |  | 9.3.1.225 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.38 |  | YES | reject |
| Broadcast MRB To Be Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

#### 9.2.13.2	BROADCAST CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a broadcast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast MRB Setup List |  | 1 |  |  | YES | reject |
| >Broadcast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| Broadcast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Broadcast Area Scope | O |  | 9.3.1.287 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

#### 9.2.13.3	BROADCAST CONTEXT SETUP FAILURE

This message is sent by the gNB-DU to indicate that the setup of the broadcast context was unsuccessful.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | O |  | 9.3.1.220 |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.13.4	BROADCAST CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU to request the gNB-DU to release the broadcast context for a given broadcast service.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | gNB-CU MBS F1AP ID 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | gNB-DU MBS F1AP ID 9.3.1.220 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.13.5	BROADCAST CONTEXT RELEASE COMPLETE

This message is sent by the gNB-DU to confirm the release of the broadcast context for a given broadcast service.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.13.5a	BROADCAST CONTEXT RELEASE REQUEST

This message is sent by the gNB-DU to request the gNB-CU to trigger the Broadcast Context Release procedure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.13.6	BROADCAST CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to request the gNB-DU to modify broadcast context information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 | Overwrites any previously received MBS Service Area information | YES | reject |
| MBS CU to DU RRC Information | M |  | 9.3.1.225 |  | YES | reject |
| Broadcast MRB To Be Setup List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| Broadcast MRB To Be Modified List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB to Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | O |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 0 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at CU | O |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | Updated gNB-CU endpoint(s) of the F1 transport bearer(s). For delivery of F1-U PDU Type 1. | - |  |
| Broadcast MRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBs> |  |  | YES | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

#### 9.2.13.7	BROADCAST CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a broadcast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Broadcast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | M |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| Broadcast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Broadcast MRB Modified List |  | 0..1 |  |  | YES | reject |
| >Broadcast MRB Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>BC Bearer Context F1-U TNL Info at DU | O |  | BC Bearer Context F1-U TNL Info 9.3.2.7 | Updated gNB-DU endpoint(s) of the F1-U transport bearer(s). For delivery of DL PDUs. | - |  |
| Broadcast MRB Failed To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >Broadcast MRB Failed To Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Broadcast Area Scope | O |  | 9.3.1.287 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

#### 9.2.13.8	BROADCAST CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-DU to indicate a broadcast context modification failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

### 9.2.14	Multicast Context Management messages

#### 9.2.14.1	MULTICAST GROUP PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to multicast group page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| UE Identity List for Paging |  | 0..1 |  |  | YES | ignore |
| >UE Identity for Paging Item |  | 1..<maxnoofUEIDforPaging> |  |  | - |  |
| >>UE Identity Index value | M |  | 9.3.1.39 |  | - |  |
| >>Paging DRX | O |  | 9.3.1.40 |  | - |  |
| MC Paging Cell List |  | 0..1 |  |  | YES | ignore |
| >MC Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEIDforPaging | Maximum no. of UE ID for multicast group paging. Value is 4096. |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

#### 9.2.14.2	MULTICAST CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of an MBS session context for a multicast session, and establish an MBS-associated logical F1-connection.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.218 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| S-NSSAI | M |  | 9.3.1.38 |  | YES | reject |
| Multicast MRBs To Be Setup List |  | 1 |  |  | YES | reject |
| >Multicast MRBs to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | M |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

#### 9.2.14.3	MULTICAST CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a multicast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Multicast MRB Setup List |  | 1 |  |  | YES | reject |
| >Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | Reject |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | MRB ID 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

#### 9.2.14.4	MULTICAST CONTEXT SETUP FAILURE

This message is sent by the gNB-DU to indicate that the setup of the multicast context was unsuccessful.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | O |  | 9.3.1.220 |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.14.5	MULTICAST CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU to request the gNB-DU to release the multicast context for a given multicast service.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.14.6	MULTICAST CONTEXT RELEASE COMPLETE

This message is sent by the gNB-DU to confirm the release of the multicast context for a given multicast service.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.14.6a	MULTICAST CONTEXT RELEASE REQUEST

This message is sent by the gNB-DU to request the gNB-CU to trigger the Multicast Context Release procedure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.14.7	MULTICAST CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to request the gNB-DU to modify multicast context information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.222 |  | YES | reject |
| Multicast MRB To Be Setup List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>MRB QoS Information | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 1 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | M |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| Multicast MRB To Be Modified List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>MRB QoS Information | O |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>MBS QoS Flows Mapped to MRB Item |  | 0 .. <maxnoofMBSQoSFlows> |  |  | - |  |
| >>>MBS QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.63 |  | - |  |
| >>>MBS QoS Flow Level QoS Parameters | M |  | QoS Flow Level QoS Parameters 9.3.1.45 |  | - |  |
| >>DL PDCP SN Length | O |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| Multicast MRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB to Be Released Item IEs |  | 1 .. <maxnoofMRBs> |  |  | YES | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

#### 9.2.14.8	MULTICAST CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a multicast context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Multicast MRB Setup List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Multicast MRB Modified List |  | 0..1 |  |  | YES | reject |
| >Multicast MRB Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| Multicast MRB Failed To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >Multicast MRB Failed To Be Modified Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

#### 9.2.14.9	MULTICAST CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-DU to indicate a multicast context modification failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.14.10	MULTICAST DISTRIBUTION SETUP REQUEST

This message is sent by the gNB-DU to request the setup of a Multicast F1-U Context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Multicast F1-U Context Descriptor | M |  | 9.3.2.8 |  | YES | reject |
| Multicast F1-U Context To Be Setup List |  | 1 |  |  | YES | reject |
| >Multicast F1-U Context To Be Setup Item |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>MRB F1-U TNL Info at DU | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1-U transport bearer. | - |  |
| >>MRB Progress Information | C-ifPTPForwarding |  | 9.3.2.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |
| maxnoofMBSQoSFlows | Maximum no. of flows allowed to be mapped to one MRB, the maximum value is 64. |

| Condition | Explanation |
| --- | --- |
| ifPTPForwarding | This IE shall be present if the MC F1-U Context usage IE in the MBS Multicast F1-U Context Descriptor IE is set to "ptp forwarding". |

#### 9.2.14.11	MULTICAST DISTRIBUTION SETUP RESPONSE

This message is sent by the gNB-CU to confirm the setup of a Multicast F1-U Context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Multicast F1-U Context Descriptor | M |  | 9.3.2.8 |  | YES | reject |
| Multicast F1-U Context Setup List |  | 1 |  |  | YES | reject |
| >Multicast F1-U Context Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | reject |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>MRB F1-U TNL Info at CU | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1-U transport bearer. | - |  |
| Multicast F1-U Context Failed To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >Multicast F1-U Context Failed To Be Setup Item IEs |  | 1 .. <maxnoofMRBs> |  |  | EACH | ignore |
| >>MRB ID | M |  | 9.3.1.224 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Multicast F1-U Context Reference CU | M |  | 9.3.2.13 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRB allowed to be setup for one MBS Session, the maximum value is 32. |

#### 9.2.14.12	MULTICAST DISTRIBUTION SETUP FAILURE

This message is sent by the gNB-DU to indicate that the setup of the Multicast F1-U Context was was unsuccessful.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | O |  | 9.3.1.220 |  | YES | ignore |
| MBS Multicast F1-U Context Descriptor | M |  | 9.3.2.8 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.14.13	MULTICAST DISTRIBUTION RELEASE COMMAND

This message is sent by the gNB-DU to request the gNB-CU to release the Multicast F1-U Context for a given multicast MBS Session.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Multicast F1-U Context Descriptor | M |  | 9.3.2.8 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.14.14	MULTICAST DISTRIBUTION RELEASE COMPLETE

This message is sent by the gNB-CU to confirm the release of the Multicast F1-U Context.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | 9.3.1.219 |  | YES | reject |
| gNB-DU MBS F1AP ID | M |  | 9.3.1.220 |  | YES | reject |
| MBS Multicast F1-U Context Descriptor | M |  | 9.3.2.8 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

### 9.2.15	PDC Measurement Reporting messages

#### 9.2.15.1	PDC MEASUREMENT INITIATION REQUEST

This message is sent by gNB-CU to initiate PDC measurements.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN UE PDC Measurement ID | M |  | INTEGER (1..16, …) |  | YES | reject |
| PDC Report Type | M |  | ENUMERATED (OnDemand, Periodic, …) |  | YES | reject |
| PDC Measurement Periodicity | C-ifReportTypePeriodic |  | ENUMERATED (80ms, 120ms, 160ms, 240ms, 320ms, 480ms, 640ms, 1024ms, 1280ms, 2048ms, 2560ms, 5120ms, …) |  | YES | reject |
| PDC Measurement Quantities |  | 1 .. <maxnoMeasPDC> |  |  | EACH | reject |
| >PDC Measurement Quantities Item | M |  | ENUMERATED (NR PDC TADV, gNB RX-TX, …) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeasPDC | Maximum no. of PDC measured quantities that can be configured and reported with one message. Value is 16. Maximum is 1 in this release. |

| Condition | Explanation |
| --- | --- |
| ifReportTypePeriodic | This IE shall be present if the PDC Report Type IE is set to the value "Periodic". |

#### 9.2.15.2	PDC MEASUREMENT INITIATION RESPONSE

This message is sent by gNB-DU to indicate that the requested PDC measurement is successfully initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN UE PDC Measurement ID | M |  | INTEGER (1..16, …) |  | YES | reject |
| PDC Measurement Result | O |  | 9.3.1.232 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.15.3	PDC MEASUREMENT INITIATION FAILURE

This message is sent by gNB-DU to indicate that the requested PDC measurement cannot be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN UE PDC Measurement ID | M |  | INTEGER (1..16, …) |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.15.4	PDC MEASUREMENT REPORT

This message is sent by gNB-DU to report the results of the requested PDC measurement.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN UE PDC Measurement ID | M |  | INTEGER (1..16, …) |  | YES | reject |
| PDC Measurement Result | M |  | 9.3.1.232 |  | YES | ignore |

#### 9.2.15.5	PDC MEASUREMENT TERMINATION COMMAND

This message is sent by the gNB-CU to request the gNB-DU to terminate an ongoing periodical PDC measurement.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN UE PDC Measurement ID | M |  | INTEGER (1..16, …) |  | YES | ignore |

#### 9.2.15.6	PDC MEASUREMENT FAILURE INDICATION

This message is sent by the gNB-DU to indicate that the previously requested PDC measurements can no longer be reported.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN UE PDC Measurement ID | M |  | INTEGER (1..16, …) |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

### 9.2.16	QMC messages

#### 9.2.16.1	QOE INFORMATION TRANSFER

This message is sent by a gNB-CU to a gNB-DU, to indicate information related to RAN visible QoE.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| QoE Information List |  |  | 0..1 |  | YES | ignore |
| >QoE Information Item |  |  | 1..<maxnoofQoEInformation> |  | EACH | ignore |
| >>QoE Metrics | O |  | 9.3.1.260 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoEInformation | Maximum no. of QoE information for one UE, the maximum value is 16. |

## 9.3	Information Element Definitions

### 9.3.1	Radio Network Layer Related IEs

#### 9.3.1.1	Message Type

The Message Type IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type |  |  |  |  |
| >Procedure Code | M |  | INTEGER (0..255) |  |
| >Type of Message | M |  | CHOICE (Initiating Message, Successful Outcome, Unsuccessful Outcome, ...) |  |

#### 9.3.1.2	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the F1AP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, RL failure-RLC, Unknown or already allocated gNB-CU UE F1AP ID, Unknown or already allocated gNB-DU UE F1AP ID, Unknown or inconsistent pair of UE F1AP ID, Interaction with other procedure, Not supported QCI Value, Action Desirable for Radio Reasons, No Radio Resources Available, Procedure cancelled, Normal Release, ..., Cell not available, RL failure-others, UE rejection, Resources not available for the slice(s), AMF initiated abnormal release, Release due to Pre-Emption, PLMN not served by the gNB-CU, Multiple DRB ID Instances, Unknown DRB ID, Multiple BH RLC CH ID Instances, Unknown BH RLC CH ID, CHO-CPC resources to be changed, NPN not supported, NPN access denied, gNB-CU Cell Capacity Exceeded, Report Characteristics Empty, Existing Measurement ID, Measurement Temporarily not Available, Measurement not Supported For The Object, Unknown BAP address, Unknown BAP routing ID, Insufficient UE Capabilities, SCG activation deactivation failure, SCG deactivation failure due to data transmission, Requested Item not Supported on Time, Unknown or already allocated gNB-CU MBS F1AP ID, Unknown or already allocated gNB-DU MBS F1AP ID, Unknown or inconsistent pair of MBS F1AP ID, Unknown or inconsistent MRB ID, TAT-SDT expiry) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Unspecified, Transport Resource Unavailable, ... , Unknown TNL address for IAB, Unknown UP TNL information for IAB) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, ...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Not enough User Plane Processing Resources, Hardware Failure, O&M Intervention, Unspecified, ...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the related capability is missing. On the other hand, "not available" cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| RL Failure-RLC | The action is due to an RL failure caused by exceeding the maximum number of ARQ retransmissions. |
| Unknown or already allocated gNB-CU UE F1AP ID | The action failed because the gNB-CU UE F1AP ID is either unknown, or (for a first message received at the gNB-CU) is known and already allocated to an existing context. |
| Unknown or already allocated gNB-DU UE F1AP ID | The action failed because the gNB-DU UE F1AP ID is either unknown, or (for a first message received at the gNB-DU) is known and already allocated to an existing context. |
| Unknown or inconsistent pair of UE F1AP ID | The action failed because both UE F1AP IDs are unknown, or are known but do not define a single UE context. |
| Interaction with other procedure | The action is due to an ongoing interaction with another procedure. |
| Not supported QCI Value | The action failed because the requested QCI is not supported. |
| Action Desirable for Radio Reasons | The reason for requesting the action is radio related. |
| No Radio Resources Available | The cell(s) in the requested node don’t have sufficient radio resources available. |
| Procedure cancelled | The sending node cancelled the procedure due to other urgent actions to be performed. |
| Normal Release | The action is due to a normal release of the UE (e.g. because of mobility) and does not indicate an error. |
| Cell Not Available | The action failed due to no cell available in the requested node. |
| RL Failure-others | The action is due to an RL failure caused by other radio link failures than exceeding the maximum number of ARQ retransmissions. |
| UE rejection | The action is due to gNB-CU’s rejection of a UE access request. |
| Resources not available for the slice(s) | The requested resources are not available for the slice(s). |
| AMF initiated abnormal release | The release is triggered by an error in the AMF or in the NAS layer. |
| Release due to Pre-Emption | Release is initiated due to pre-emption. |
| PLMN not served by the gNB-CU | The PLMN indicated by the UE is not served by the gNB-CU. |
| Multiple DRB ID Instances | The action failed because multiple instances of the same DRB had been provided. |
| Unknown DRB ID | The action failed because the DRB ID is unknow. |
| Multiple BH RLC CH ID Instances | The action failed because multiple instances of the same BH RLC CH ID had been provided. This cause value is only applicable to IAB. |
| Unknown BH RLC CH ID | The action failed because the BH RLC CH ID is unknown. This cause value is only applicable to IAB. |
| CHO-CPC resources to be changed | The gNB-DU requires gNB-CU to replace, i.e. overwrite the configuration of indicated candidate target cell. |
| NPN not supported | The action fails because the indicated SNPN is not supported in the node. |
| NPN access denied | The action is due to rejection of a UE access request for NPN. |
| gNB-CU Cell Capacity Exceeded | The number of cells requested to be added was exceeding maximum cell capacity in the gNB-CU. |
| Report Characteristics Empty | The action failed because there is no measurement object in the report characteristics. |
| Existing Measurement ID | The action failed because the measurement ID is already used. |
| Measurement Temporarily not Available | The gNB-DU can temporarily not provide the requested measurement object. |
| Measurement not Supported For The Object | At least one of the concerned object(s) does not support the requested measurement. |
| Unknown BAP address | The action failed because the BAP address is unknown. This cause value is only applicable to IAB. |
| Unknown BAP routing ID | The action failed because the BAP routing ID is unknown. This cause value is only applicable to IAB. |
| Insufficient UE Capabilities | The setup can’t proceed due to insufficient UE capabilities. |
| SCG activation deactivation failure | The action failed due to rejection of the SCG activation deactivation request. |
| SCG deactivation failure due to data transmission | The SCG deactivation failure due to ongoing or arriving data transmission. |
| Requested Item not Supported on Time | The gNB-DU is unable to provide the measurement results on time. |
| Unknown or already allocated gNB-CU MBS F1AP ID | The action failed because the gNB-CU MBS F1AP ID is either unknown, or (for a first message received at the gNB-CU) is known and already allocated to an existing context. |
| Unknown or already allocated gNB-DU MBS F1AP ID | The action failed because the gNB-DU MBS F1AP ID is either unknown, or (for a first message received at the gNB-DU) is known and already allocated to an existing context. |
| Unknown or inconsistent pair of MBS F1AP ID | The action failed because both MBS F1AP IDs are unknown, or are known but do not define a single MBS context. |
| Unknown or inconsistent MRB ID | The action failed because the MRB ID is unknown or inconsistent. |
| TAT-SDT expiry | The UE context release is requested from the gNB-DU due to the expiry of the Timing Alignment timer for CG-SDT. |

| Transport Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |
| Transport Resource Unavailable | The required transport resources are not available. |
| Unknown TNL address for IAB | The action failed because the TNL address is unknown. This cause value is only applicable to IAB. |
| Unknown UP TNL information for IAB | The action failed because the UP TNL information is unknown. This cause value is only applicable to IAB. |

| Protocol cause | Meaning |
| --- | --- |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated "reject". |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated "ignore and notify". |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |

| Miscellaneous cause | Meaning |
| --- | --- |
| Control Processing Overload | Control processing overload. |
| Not Enough User Plane Processing Resources Available | No enough resources are available related to user plane processing. |
| Hardware Failure | Action related to hardware failure. |
| O&M Intervention | The action is due to O&M intervention. |
| Unspecified Failure | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer or Protocol. |

#### 9.3.1.3	Criticality Diagnostics

The Criticality Diagnostics IE is sent by the gNB-DU or the gNB-CU when parts of a received message have not been comprehended or were missing, or if the message contained logical errors. When applicable, it contains information about which IEs were not comprehended or were missing.

For further details on how to use the Criticality Diagnostics IE, (see clause 10). The conditions for inclusion of the Transaction ID IE are described in clause 10.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | O |  | INTEGER (0..255) | Procedure Code is to be used if Criticality Diagnostics is part of Error Indication procedure, and not within the response message of the same procedure that caused the error. |
| Triggering Message | O |  | ENUMERATED(initiating message, successful outcome, unsuccessful outcome) | The Triggering Message is used only if the Criticality Diagnostics is part of Error Indication procedure. |
| Procedure Criticality | O |  | ENUMERATED(reject, ignore, notify) | This Procedure Criticality is used for reporting the Criticality of the Triggering message (Procedure). |
| Transaction ID | O |  | 9.3.1.23 |  |
| Information Element Criticality Diagnostics |  | 0 .. <maxnoof Errors> |  |  |
| >IE Criticality | M |  | ENUMERATED(reject, ignore, notify) | The IE Criticality is used for reporting the criticality of the triggering IE. The value 'ignore' is not applicable. |
| >IE ID | M |  | INTEGER (0..65535) | The IE ID of the not understood or missing IE. |
| >Type of Error | M |  | ENUMERATED(not understood, missing, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofErrors | Maximum no. of IE errors allowed to be reported with a single message. The value for maxnoofErrors is 256. |

#### 9.3.1.4	gNB-CU UE F1AP ID

The gNB-CU UE F1AP ID uniquely identifies the UE association over the F1 interface within the gNB-CU.

NOTE: 	If F1-C signalling transport is shared among multiple interface instances, the value of the gNB-CU UE F1AP ID is allocated so that it can be associated with the corresponding F1-C interface instance.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-CU UE F1AP ID | M |  | INTEGER (0 .. 232 -1) |  |

#### 9.3.1.5	gNB-DU UE F1AP ID

The gNB-DU UE F1AP ID uniquely identifies the UE association over the F1 interface within the gNB-DU.

NOTE: 	If F1-C signalling transport is shared among multiple interface instances, the value of the gNB-DU UE F1AP ID is allocated so that it can be associated with the corresponding F1-C interface instance.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-DU UE F1AP ID | M |  | INTEGER (0 .. 232 -1) |  |

#### 9.3.1.6	RRC-Container

This information element contains a gNB-CUUE or a UE  gNB-CU message that is transferred without interpretation in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RRC-Container | M |  | OCTET STRING |  |

#### 9.3.1.7	SRB ID

This IE uniquely identifies a SRB for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SRB ID | M |  | INTEGER (0..3, ..., 4) | Corresponds to the identities of SRB as defined in TS 38.331 [8]. Value 0 indicates SRB0, value 1 indicates SRB1, etc. |

#### 9.3.1.8	DRB ID

This IE uniquely identifies a DRB for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DRB ID | M |  | INTEGER (1.. 32, ...) | Corresponds to the DRB-Identity defined in TS 38.331 [8]. |

#### 9.3.1.9	gNB-DU ID

The gNB-DU ID uniquely identifies the gNB-DU at least within a gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-DU ID | M |  | INTEGER (0 .. 236-1) | The gNB-DU ID is independently configured from cell identifiers, i.e. no connection between gNB-DU ID and cell identifiers. |

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| CHOICE NR-Mode-Info | M |  |  |  | - |  |
| >FDD |  |  |  |  | - |  |
| >>FDD Info |  | 1 |  |  | - |  |
| >>>UL FreqInfo | M |  | NR Frequency Info 9.3.1.17 | This IE is ignored if the Cell Direction IE is included and set to “dl-only”. | - |  |
| >>>DL FreqInfo | M |  | NR Frequency Info 9.3.1.17 | This IE is ignored if the Cell Direction IE is included and set to “ul-only”. | - |  |
| >>>UL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 | This IE is ignored if the Cell Direction IE is included and set to “dl-only”. | - |  |
| >>>DL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 | This IE is ignored if the Cell Direction IE is included and set to “ul-only”. | - |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the UL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>DL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the DL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >TDD |  |  |  |  | - |  |
| >>TDD Info |  | 1 |  |  | - |  |
| >>>NR FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 | This IE is ignored if the Transmission Bandwidth asymmetric IE is present. | - |  |
| >>>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | YES | ignore |
| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon contained in the ServingCellConfigCommon IE as defined in TS 38.331 [8] | YES | ignore |
| >>>Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>Transmission Bandwidth asymmetric |  | 0..1 |  | Indicates the asymmetric UL and DL transmission bandwidth. | YES | ignore |
| >>>>UL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 | . | – |  |
| >>>>DL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | – |  |
| >NR-U |  |  |  |  | YES | ignore |
| >>NR-U Channel Info List |  | 1..< maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>NR-U Channel Info Item |  |  |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Index to uniquely identify the part of the NR-U Channel Bandwidth consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum. Value 1 represents the first part of the NR-U Channel Bandwidth on which a channel access procedure is performed. Value 2 represents the second part of the NR-U Channel Bandwidth on which a channel access procedure is performed, and so on. | - |  |
| >>>>NR-U ARFCN | M |  | INTEGER (0.. maxNRARFCN) | It represents the centre frequency of the NR-U Channel Bandwidth for NR bands restricted to operation with shared spectrum channel access, as defined in TS 37.213 [46]. Allowed values are specified in TS 38.101-1 [26] in Table 5.4.2.3-2, Table 5.4.2.3-3 and Table 5.4.2.3-4. | - |  |
| >>>>NR-U Channel Bandwidth | M |  | ENUMERATED (10MHz, 20MHz, 40MHz, 60 MHz, 80 MHz,. …, 100MHz) |  | - |  |
| Measurement Timing Configuration | M |  | OCTET STRING | Includes the MeasurementTimingConfiguration inter-node message defined in TS 38.331 [8]. | - |  |
| RANAC | O |  | RAN Area Code 9.3.1.57 |  | YES | ignore |
| Extended Served PLMNs List |  | 0..1 |  | This is included if more than 6 Served PLMNs is to be signalled. | YES | ignore |
| >Extended Served PLMNs Item |  | 1 ..<maxnoofExtendedBPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >>TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | - |  |
| >>NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.273 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| Cell Direction | O |  | 9.3.1.78 |  | YES | ignore |
| Broadcast PLMN Identity Info List |  | 0..<maxnoofBPLMNsNR> |  | This IE corresponds to the PLMN-IdentityInfoList IE and the NPN-IdentityInfoList IE (if available) in SIB1 as specified in TS 38.331 [8]. All PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE and NPN identities and associated information contained in the NPN-IdentityInfoList IE (if available) are included and provided in the same order as broadcast in SIB1. NOTE: In case of NPN-only cell, the PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE are not included. | YES | ignore |
| Cell Type | O |  | 9.3.1.87 |  | YES | ignore |
| >PLMN Identity List | M |  | Available PLMN List 9.3.1.65 | Broadcast PLMN IDs in SIB1 associated to the NR Cell Identity IE | - |  |
| >Extended PLMN Identity List | O |  | Extended Available PLMN List 9.3.1.76 |  | - |  |
| >5GS-TAC | O |  | OCTET STRING (3) |  | - |  |
| >NR Cell Identity | M |  | BIT STRING (36) |  | - |  |
| >RANAC | O |  | RAN Area Code 9.3.1.57 |  | - |  |
| >Configured TAC Indication | O |  | 9.3.1.87a | NOTE: This IE is associated with the 5GS TAC in the Broadcast PLMN Identity Info List IE | YES | ignore |
| >NPN Broadcast Information | O |  | 9.3.1.157 | If this IE is included the content of the PLMN Identity List IE and Extended PLMN Identity List IE if present in the Broadcast PLMN Identity Info List IE is ignored. | YES | reject |
| Configured TAC Indication | O |  | 9.3.1.87a | NOTE: This IE is associated with the 5GS TAC on top-level of the Served Cell Information IE | YES | ignore |
| Aggressor gNB Set ID | O |  | 9.3.1.93 | This IE indicates the associated aggressor gNB Set ID of the cell | YES | ignore |
| Victim gNB Set ID | O |  | 9.3.1.93 | This IE indicates the associated Victim gNB Set ID of the cell | YES | ignore |
| IAB Info IAB-DU | O |  | 9.3.1.106 |  | YES | ignore |
| SSB Positions In Burst | O |  | 9.3.1.138 |  | YES | ignore |
| NR PRACH Configuration | O |  | 9.3.1.139 |  | YES | ignore |
| SFN Offset | O |  | 9.3.1.208 |  | YES | ignore |
| NPN Broadcast Information | O |  | 9.3.1.157 |  | YES | reject |
| Supported MBS FSA ID List |  | 0..<maxnoofMBSFSAs> |  | Shall contain all MBS Frequency Selection Area Identities associated with the NR CGI. | YES | ignore |
| >MBS Frequency Selection Area Identity | M |  | OCTET STRING(3) |  | – |  |
| RedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselectionRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [8]. Each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of RedCap barring indicators in SIB1, see TS 38.331 [8]. First bit = 1Rx, second bit = 2Rx, third bit = halfDuplex, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed”. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 16. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

#### 9.3.1.11	Transmission Action Indicator

This IE indicates actions for the gNB-DU for the data transmission to the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transmission Action Indicator | M |  | ENUMERATED (stop, ..., restart) |  |

#### 9.3.1.12	NR CGI

The NR Cell Global Identifier (NR CGI) is used to globally identify a cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.1.14 |  |
| NR Cell Identity | M |  | BIT STRING (SIZE(36)) |  |

#### 9.3.1.13	Time To wait

This IE defines the minimum allowed waiting times.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time to wait | M |  | ENUMERATED(1s, 2s, 5s, 10s, 20s, 60s) |  |

#### 9.3.1.14	PLMN Identity

This information element indicates the PLMN Identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | OCTET STRING (SIZE(3)) | - digits 0 to 9, encoded 0000 to 1001, - 1111 used as filler digit, two digits per octet, - bits 4 to 1 of octet n encoding digit 2n-1 - bits 8 to 5 of octet n encoding digit 2n -The PLMN identity consists of 3 digits from MCC followed by either -a filler digit plus 2 digits from MNC (in case of 2 digit MNC) or -3 digits from MNC (in case of a 3 digit MNC). |

#### 9.3.1.15	Transmission Bandwidth

The Transmission Bandwidth IE is used to indicate the UL or DL transmission bandwidth.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, ... , scs480, scs960) | The values scs15, scs30, scs60, scs120, scs480 and scs960 corresponds to the sub carrier spacing in TS 38.104 [17]. |
| NRB | M |  | ENUMERATED (nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ..., nrb33, nrb62, nrb124, nrb148, nrb248, nrb44, nrb58, nrb92, nrb119, nrb188, nrb242) | This IE is used to indicate the UL or DL transmission bandwidth expressed in units of resource blocks "NRB" (TS 38.104 [17]). The values nrb11, nrb18, etc. correspond to the number of resource blocks "NRB" 11, 18, etc. |

#### 9.3.1.16	Void

Reserved for future use.

#### 9.3.1.17	NR Frequency Info

The NR Frequency Info defines the carrier frequency used in a cell for a given direction (UL or DL) in FDD or for both UL and DL directions in TDD or for an SUL carrier.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR ARFCN | M |  | INTEGER (0.. maxNRARFCN) | RF Reference Frequency as defined in TS 38.104 [17] section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. | – |  |
| SUL Information | O |  | 9.3.1.28 |  | – |  |
| Frequency Band List |  | 1 |  |  | – |  |
| >Frequency Band Item |  | 1..<maxnoofNrCellBands> |  |  | – |  |
| >>NR Frequency Band | M |  | INTEGER (1.. 1024, ...) | Operating Band as defined in TS 38.104 [17] section 5.4.2.3. The value 1 corresponds to NR operating band n1, value 2 corresponds to NR operating band n2, etc. | – |  |
| >>Supported SUL band List |  | 0..<maxnoofNrCellBands> |  |  | – |  |
| >>>Supported SUL band Item | M |  | INTEGER (1.. 1024, ...) | Supplementary NR Operating Band as defined in TS 38.104 [17] section 5.4.2.3 that can be used for SUL duplex mode as per TS 38.101-1 [26] table 5.2.-1. The value 80 corresponds to NR operating band n80, value 81 corresponds to NR operating band n81, etc. | – |  |
| Frequency Shift 7p5khz | O |  | ENUMERATED (false, true, ...) | Indicate whether the value of Δshift is 0kHz or 7.5kHz when calculating FREF,shift as defined in Section 5.4.2.1 of TS 38.104 [17]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxNRARFCN | Maximum value of NR ARFCNs. Value is 3279165. |
| maxnoofNrCellBands | Maximum no. of frequency bands supported for a NR cell. Value is 32. |

#### 9.3.1.18	gNB-DU System Information

This IE contains the system information generated by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MIB message | M |  | OCTET STRING | Includes the MIB message, as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| SIB1 message | M |  | OCTET STRING | Includes the SIB1 message, as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| SIB12 message | O |  | OCTET STRING | Includes the SIB12 IE, as defined in subclause 6.3.1 in TS 38.331 [8]. | YES | ignore |
| SIB13 message | O |  | OCTET STRING | Includes the SIB13 IE, as defined in subclause 6.3.1 in TS 38.331 [8]. | YES | ignore |
| SIB14 message | O |  | OCTET STRING | Includes the SIB14 IE, as defined in subclause 6.3.1 in TS 38.331 [8]. | YES | ignore |
| SIB10 message | O |  | OCTET STRING | Includes the SIB10 IE, as defined in subclause 6.3.1 in TS 38.331 [8]. | YES | ignore |
| SIB17 message | O |  | OCTET STRING | Includes the SIB17 IE, as defined in subclause 6.3.1 in TS 38.331 [8] | YES | ignore |
| SIB20 message | O |  | OCTET STRING | Includes the SIB20 IE, as defined in subclause 6.3.1 in TS 38.331 [8] | YES | ignore |
| SIB15 message | O |  | OCTET STRING | Includes the SIB15 IE, as defined in subclause 6.3.1 in TS 38.331 [8]. | YES | ignore |

#### 9.3.1.19	E-UTRAN QoS

This IE defines the QoS to be applied to a DRB or to a BH RLC channel for EN-DC case.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QCI | M |  | INTEGER (0..255) | QoS Class Identifier defined in TS 23.401 [10]. Logical range and coding specified in TS 23.203 [11]. For a BH RLC channel, the Packet Delay Budget included in QCI defines the upper bound for the time that a packet may be delayed between the gNB-DU and its child IAB-MT. | - |  |
| Allocation and Retention Priority | M |  | 9.3.1.20 |  | - |  |
| GBR QoS Information | O |  | 9.3.1.21 | This IE shall be present for GBR bearers only and is ignored otherwise. | - |  |
| ENB DL Transport Layer Address | O |  | Transport Layer Address 9.3.2.3 | DL Transport Layer Address of node terminating PDCP. Included for MN-terminated SCG bearers. | YES | ignore |

#### 9.3.1.20	Allocation and Retention Priority

This IE specifies the relative importance compared to other E-RABs for allocation and retention of the E-UTRAN Radio Access Bearer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Priority Level | M |  | INTEGER (0..15) | Desc.: This IE should be understood as "priority of allocation and retention" (see TS 23.401 [10]). Usage: Value 15 means "no priority". Values between 1 and 14 are ordered in decreasing order of priority, i.e. 1 is the highest and 14 the lowest. Value 0 shall be treated as a logical error if received. |
| Pre-emption Capability | M |  | ENUMERATED(shall not trigger pre-emption, may trigger pre-emption) | Desc.: This IE indicates the pre-emption capability of the request on other E-RABs (see TS 23.401 [10]). Usage: The E-RAB shall not pre-empt other E-RABs or, the E-RAB may pre-empt other E-RABs The Pre-emption Capability indicator applies to the allocation of resources for an E-RAB and as such it provides the trigger to the pre-emption procedures/processes of the eNB. |
| Pre-emption Vulnerability | M |  | ENUMERATED(not pre-emptable, pre-emptable) | Desc.: This IE indicates the vulnerability of the E-RAB to pre-emption of other E-RABs (see TS 23.401 [10]). Usage: The E-RAB shall not be pre-empted by other E-RABs or the E-RAB may be pre-empted by other RABs. Pre-emption Vulnerability indicator applies for the entire duration of the E-RAB, unless modified, and as such indicates whether the E-RAB is a target of the pre-emption procedures/processes of the eNB. |

#### 9.3.1.21	GBR QoS Information

This IE indicates the maximum and guaranteed bit rates of a GBR E-RAB for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| E-RAB Maximum Bit Rate Downlink | M |  | Bit Rate 9.3.1.22 | Maximum Bit Rate in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [10]. |
| E-RAB Maximum Bit Rate Uplink | M |  | Bit Rate 9.3.1.22 | Maximum Bit Rate in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [10]. |
| E-RAB Guaranteed Bit Rate Downlink | M |  | Bit Rate 9.3.1.22 | Guaranteed Bit Rate (provided that there is data to deliver) in DL (i.e. from EPC to E-UTRAN) for the bearer. Details in TS 23.401 [10]. |
| E-RAB Guaranteed Bit Rate Uplink | M |  | Bit Rate 9.3.1.22 | Guaranteed Bit Rate (provided that there is data to deliver) in UL (i.e. from E-UTRAN to EPC) for the bearer. Details in TS 23.401 [10]. |

#### 9.3.1.22	Bit Rate

This IE indicates the number of bits delivered by NG-RAN in UL or to NG-RAN in DL within a period of time, divided by the duration of the period. It is used, for example, to indicate the maximum or guaranteed bit rate for a GBR QoS flow, or an aggregated maximum bit rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Bit Rate | M |  | INTEGER (0.. 4,000,000,000,000,...) | The unit is: bit/s |

#### 9.3.1.23	Transaction ID

The Transaction ID IE uniquely identifies a procedure among all ongoing parallel procedures of the same type initiated by the same protocol peer. Messages belonging to the same procedure use the same Transaction ID. The Transaction ID is determined by the initiating peer of a procedure.

NOTE:	If F1-C signalling transport is shared among multiple interface instances, the Transaction ID is allocated so that it can be associated with an F1-C interface instance. The Transaction ID may identify more than one interface instance.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transaction ID | M |  | INTEGER (0..255, ...) |  |

#### 9.3.1.24	DRX Cycle

The DRX Cycle IE is to indicate the desired DRX cycle.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Long DRX Cycle Length | M |  | ENUMERATED (ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, ...) | Corresponds to the preferredDRX-LongCycle contained in the UEAssistanceInformation message defined in TS 38.331 [8] |
| Short DRX Cycle Length | O |  | ENUMERATED (ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32, ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ...) | Corresponds to the preferredDRX-ShortCycle contained in the UEAssistanceInformation message defined in TS 38.331 [8] |
| Short DRX Cycle Timer | O |  | INTEGER (1..16) | Corresponds to the preferredDRX-ShortCycleTimer contained in the UEAssistanceInformation message defined in TS 38.331 [8] |

#### 9.3.1.25	CU to DU RRC Information

This IE contains the RRC Information that are sent from gNB-CU to gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CG-ConfigInfo | O |  | OCTET STRING | Includes the CG-ConfigInfo message, as defined in TS 38.331 [8]. | - |  |
| UE-CapabilityRAT-ContainerList | O |  | OCTET STRING | This IE is used in the NG-RAN and it includes the UE-CapabilityRAT-ContainerList IE, as defined in TS 38.331 [8]. | - |  |
| MeasConfig | O |  | OCTET STRING | Includes the MeasConfig IE, as defined in TS 38.331 [8] (without the MeasGapConfig IE). For EN-DC/NGEN-DC operation, includes the list of FR2 frequencies for which the gNB-CU requests the gNB-DU to generate gaps. For NG-RAN, NE-DC and MN for NR-NR DC, includes the list of FR1 and/or FR2 frequencies, for which the gNB-CU requests the gNB-DU to generate gaps and the gap type (per-UE or per-FR). | - |  |
| Handover Preparation Information | O |  | OCTET STRING | Includes the HandoverPreparationInformation message, as defined in TS 38.331 [8]. | YES | ignore |
| CellGroupConfig | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | YES | ignore |
| Measurement Timing Configuration | O |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message defined in TS 38.331 [8]. In EN-DC/NGEN-DC, it is included when the gaps for FR2 are requested to be configured by the MeNB. For MN in NR-NR DC,it is included when the gaps for FR2 and/or FR1 are requested by the SgNB | YES | ignore |
| UEAssistanceInformation | O |  | OCTET STRING | Includes the UEAssistanceInformation message, as defined in TS 38.331 [8]. | YES | ignore |
| CG-Config | O |  | OCTET STRING | Includes the CG-Config message, as defined in TS 38.331 [8]. | YES | ignore |
| UEAssistanceInformationEUTRA | O |  | OCTET STRING | Includes the UEAssistanceInformation message, as defined in TS 36.331 [41]. | YES | ignore |
| Location Measurement Information | O |  | OCTET STRING | Includes the LocationMeasurementInfo IE, as defined in TS 38.331[8] | YES | ignore |
| MUSIM-GapConfig | O |  | OCTET STRING | Includes the MUSIM-GapConfig IE as defined in TS 38.331 [8]. | YES | reject |
| SDT-MAC-PHY-CG-Config | O |  | OCTET STRING | Includes the SDT-MAC-PHY-CG-Config IE, as defined in TS 38.331 [8]. | YES | ignore |
| MBSInterestIndication | O |  | OCTET STRING | Includes the MBSInterestIndication message as defined in TS 38.331 [8]. | YES | ignore |
| NeedForGapsInfoNR | O |  | OCTET STRING | Includes the NeedForGapsInfoNR IE, as defined in TS 38.331 [8]. | YES | ignore |
| NeedForGapNCSG-InfoNR | O |  | OCTET STRING | Includes the NeedForGapNCSG-InfoNR IE, as defined in TS 38.331 [8]. | YES | ignore |
| NeedForGapNCSG-InfoEUTRA | O |  | OCTET STRING | Includes the NeedForGapNCSG-InfoEUTRA IE, as defined in TS 38.331 [8]. | YES | ignore |
| ConfigRestrictInfoDAPS | O |  | OCTET STRING | Includes the ConfigRestrictInfoDAPS-r16 IE as defined in TS 38.331 [8]. This IE is used at the source node if DAPS HO is configured. | YES | ignore |
| Preconfigured measurement GAP Request | O |  | ENUMERATED(true, …) |  | YES | ignore |

#### 9.3.1.26	DU to CU RRC Information

This IE contains the RRC Information that are sent from the gNB-DU to the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CellGroupConfig | M |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| MeasGapConfig | O |  | OCTET STRING | Includes the MeasGapConfig IE as defined in TS 38.331 [8]. For EN-DC/NGEN-DC operation, includes the gap for FR2, as requested by the gNB-CU via MeasConfig IE. For NG-RAN, NE-DC and MN for NR-NR DC, includes the gap(s) for FR1 and/or FR2, as requested by the gNB-CU via MeasConfig IE. For pre-configured measurement GAP scenario, it includes the gapToAddModList and/or gapToReleaseList as defined in TS 38.331 [8]. | - |  |
| Requested P-MaxFR1 | O |  | OCTET STRING | Includes the requestedP-MaxFR1 contained in the CG-Config message, as defined in TS 38.331 [8]. For EN-DC, NGEN-DC and NR-DC operation, this IE should be included. | - |  |
| DRX Long Cycle Start Offset | O |  | INTEGER (0..10239) | Corresponds to the drx-LongCycleStartOffset IE contained in the DRX-Config IE as defined in TS 38.331 [8]. This field is not used in NR-DC. | - |  |
| Selected BandCombinationIndex | O |  | OCTET STRING | Includes the BandCombinationIndex IE, as defined in TS 38.331 [8]. For (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected Band Combination; if this IE is included, the gNB-CU uses this information to deduce the selected band. | YES | ignore |
| Selected FeatureSetEntryIndex | O |  | OCTET STRING | Includes the FeatureSetEntryIndex IE, as defined in TS 38.331 [8]. For (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected FeatureSet. | YES | ignore |
| Ph-InfoSCG | O |  | OCTET STRING | Includes the PH-TypeListSCG IE, as defined in TS 38.331 [8].For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in SN. | Yes | ignore |
| Requested BandCombinationIndex | O |  | OCTET STRING | Includes the BandCombinationIndex IE, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Band Combination. | YES | ignore |
| Requested FeatureSetEntryIndex | O |  | OCTET STRING | Includes the FeatureSetEntryIndex IE, as defined in TS 38.331 [8]. This IE is used for the gNB-DU to request a new Feature Set. | YES | ignore |
| DRX Config | O |  | OCTET STRING | Includes the DRX-Config IE, as defined in TS 38.331 [8]. This field is only used in NR-DC. | YES | ignore |
| PDCCH BlindDetectionSCG | O |  | OCTET STRING | Includes the pdcch-BlindDetectionSCG contained in the CG-ConfigInfo message, as defined in TS 38.331 [8]. This IE is used between the MgNB-DU and the MgNB-CU. | YES | ignore |
| Requested PDCCH BlindDetectionSCG | O |  | OCTET STRING | Includes the requestedPDCCH-BlindDetectionSCG contained in the CG-Config message, as defined in TS 38.331 [8]. This IE is used between the SgNB-DU and the SgNB-CU. | YES | ignore |
| Ph-InfoMCG | O |  | OCTET STRING | Includes the PH-TypeListMCG IE, as defined in TS 38.331 [8]. For MR-DC, this IE should be included so that gNB-CU is informed of the Power Headroom type for each serving cell in MCG. | YES | ignore |
| MeasGapSharingConfig | O |  | OCTET STRING | Includes the MeasGapSharingConfig IE as defined in TS 38.331 [8]. | YES | ignore |
| SL-PHY-MAC-RLC-Config | O |  | OCTET STRING | Includes the SL-PHY-MAC-RLC-Config-r16 IE as defined in TS 38.331 [8]. | YES | ignore |
| SL-ConfigDedicatedEUTRA-Info | O |  | OCTET STRING | Includes the SL-ConfigDedicatedEUTRA-Info IE as defined in TS 38.331 [8]. | YES | ignore |
| Requested P-MaxFR2 | O |  | OCTET STRING | Includes the requestedP-MaxFR2 contained in the CG-Config message, as defined in TS 38.331 [8]. For NR-DC operation, this IE should be included. | YES | ignore |
| SDT-MAC-PHY-CG-Config | O |  | OCTET STRING | Includes the SDT-MAC-PHY-CG-Config IE, as defined in TS 38.331 [8]. | YES | ignore |
| MUSIM-GapConfig | O |  | OCTET STRING | Includes the MUSIM-GapConfig IE as defined in TS 38.331 [8]. | YES | ignore |
| SL-RLC-ChannelToAddModList | O |  | OCTET STRING | Includes the sl-RLC-ChannelToAddModList-r17 contained in the SL-ConfigDedicatedNR IE, as defined in TS 38.331 [8] | YES | ignore |
| InterFrequencyConfig-NoGap | O |  | ENUMERATED (true, …) | Corresponds to the interFrequencyConfig-NoGap-r16 contained in the MeasConfig IE, as defined in TS 38.331 [8]. | YES | ignore |
| ul-GapFR2-Config | O |  | OCTET STRING | Includes the ul-GapFR2-Config contained in the RRCReconfiguration message, as specifed in TS 38.331 [8]. | YES | ignore |
| TwoPHRModeMCG | O |  | ENUMERATED (enabled, ...) | Corresponds to the twoPHRModeMCG contained in the CG-ConfigInfo message, as defined in TS 38.331 [8]. For NR-DC, this IE should be included so that gNB-CU is informed of the two PHR mode in the MN. | YES | ignore |
| TwoPHRModeSCG | O |  | ENUMERATED (enabled, ...) | Corresponds to the twoPHRModeSCG contained in the CG-Config message, as defined in TS 38.331 [8]. For NR-DC, this IE should be included so that gNB-CU is informed of the two PHR mode in the SN. | YES | ignore |
| ncd-SSB-RedCapInitialBWP-SDT | O |  | OCTET STRING | Includes the NonCellDefiningSSB contained in the RRCRelease message, as specified in TS 38.331 [8]. | YES | ignore |
| ServCellInfoList | O |  | OCTET STRING | Includes the ServCellInfoListSCG-NR IE or the ServCellInfoListMCG-NR IE, as defined in TS 38.331 [8]. This IE is used for inter-node message for MN and SN in case of split gNB architecture. | YES | ignore |
| Extended SL-PHY-MAC-RLC-Config | O |  | OCTET STRING | Includes the SL-PHY-MAC-RLC-Config-v1700 IE as defined in TS 38.331 [8]. If this IE is present, the SL-RLC-ChannelToAddModList IE is ignored. | YES | ignore |

#### 9.3.1.27	RLC Mode

The RLC Mode IE indicates the RLC Mode used for a DRB or a BH RLC channel, or a Uu Relay RLC channel, or a PC5 Relay RLC channel.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| RLC Mode | M |  | ENUMERATED ( RLC-AM, RLC-UM-Bidirectional, RLC-UM-Unidirectional-UL, RLC-UM-Unidirectional-DL, ...) |  |

#### 9.3.1.28	SUL Information

This IE provides information about the SUL carrier.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SUL ARFCN | M |  | INTEGER (0.. maxNRARFCN) | RF Reference Frequency as defined in TS 38.104 [17] section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the SUL carrier. Its lowest subcarrier is also known as Point A. | – |  |
| SUL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | – |  |
| Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the SUL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| Frequency Shift 7p5khz | O |  | ENUMERATED (false, true, ...) | Indicate whether the value of Δshift is 0kHz or 7.5kHz when calculating FREF,shift as defined in Section 5.4.2.1 of TS 38.104 [17]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxNRARFCN | Maximum value of NR ARFCNs. Value is 3279165. |

#### 9.3.1.29	5GS TAC

This information element is used to identify Tracking Area Code.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5GS TAC | M |  | OCTET STRING (SIZE (3)) |  |

#### 9.3.1.29a	Configured EPS TAC

This information element is used to identify a configured EPS Tracking Area Code in order to enable application of Roaming and Access Restrictions for EN-DC as specified in TS 37.340 [7]. This IE is configured for the cell, but not broadcast.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Configured EPS TAC | M |  | OCTET STRING (SIZE (2)) |  |

#### 9.3.1.30	RRC Reconfiguration Complete Indicator

This IE indicates the result of the reconfiguration performed towards the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RRC Reconfiguration Complete Indicator | M |  | ENUMERATED (true, ... , failure) |  |

#### 9.3.1.31	UL Configuration

This IE indicates how the UL scheduling is configured at gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UL UE Configuration | M |  | ENUMERATED (no-data, shared, only, ...) | Indicates how the UE uses the UL at gNB-DU, for which "no-data" indicates that the UL scheduling is not performed at gNB-DU, "shared" indicates that the UL scheduling is performed at both gNB-DU and another node, and "only" indicates that the UL scheduling is only performed at the gNB-DU. |

#### 9.3.1.32	C-RNTI

This IE contains the C-RNTI information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| C-RNTI | M |  | INTEGER (0..65535, ...) | C-RNTI as defined in TS 38.331 [8]. |

#### 9.3.1.33	Cell UL Configured

This IE indicates whether the gNB-CU requests the gNB-DU to configure the uplink as no UL, UL, SUL or UL+SUL for the indicated cell for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell UL Configured | M |  | ENUMERATED (none, UL, SUL, UL and SUL, ...) | Further details are defined in TS 38.331 [8] |

#### 9.3.1.34	RAT-Frequency Priority Information

The RAT-Frequency Priority Information contains either the Subscriber Profile ID for RAT/Frequency priority IE or the Index to RAT/Frequency Selection Priority IE. These parameters are used to define local configuration for RRM strategies.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE RAT-Frequency Priority Information | M |  |  |  |
| >EN-DC |  |  |  |  |
| >>Subscriber Profile ID for RAT/Frequency priority | M |  | INTEGER (1.. 256, ...) |  |
| >NG-RAN |  |  |  |  |
| >>Index to RAT/Frequency Selection Priority | M |  | INTEGER (1.. 256, ...) |  |

#### 9.3.1.35	LCID

This IE uniquely identifies a LCID for the associated SRB or DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LCID | M |  | INTEGER (1..32, ...) | Corresponds to the LogicalChannelIdentity defined in TS 38.331 [8]. |

#### 9.3.1.36	Duplication activation

The Duplication Activation IE indicates whether UL PDCP Duplication is activated or not.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Duplication Activation | M |  | ENUMERATED ( Active, Inactive, ...) |  |

#### 9.3.1.37	Slice Support List

This IE indicates the list of supported slices.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Support Item IEs |  | 1..<maxnoofSliceItems> |  |  |
| >S-NSSAI | M |  | 9.3.1.38 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

#### 9.3.1.38	S-NSSAI

This IE indicates the S-NSSAI as defined in TS 23.003 [23].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SST | M |  | OCTET STRING (SIZE(1)) |  |
| SD | O |  | OCTET STRING (SIZE(3)) |  |

#### 9.3.1.39	UE Identity Index value

This IE is used by the gNB-DU to calculate the Paging Frame.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE UE Identity Index Value | M |  |  |  |
| >Length-10 |  |  |  |  |
| >>Index Length 10 | M |  | BIT STRING (SIZE(10)) | Coded as specified in TS 38.304 [24]. |

#### 9.3.1.40	Paging DRX

This IE indicates the Paging DRX as defined in TS 38.304 [24].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging DRX | M |  | ENUMERATED(32, 64, 128, 256, ...) | Unit in radio frame. |

#### 9.3.1.41	Paging Priority

This IE indicates the paging priority for paging a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Priority | M |  | ENUMERATED (PrioLevel1, PrioLevel2, PrioLevel3, PrioLevel4, PrioLevel5, PrioLevel6, PrioLevel7, PrioLevel8, ...) | Lower value codepoint indicates higher priority. |

#### 9.3.1.42	gNB-CU System Information

This IE contains the system information encoded by the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SIB type to Be Updated List |  | 1 |  |  | - |  |
| >SIB type to Be Updated Item IEs |  | 1... <maxnoofSIBTypes> |  |  | - |  |
| >>SIB type | M |  | INTEGER (2..32, ...) | Indicates a certain SIB block, e.g. 2 means sibType2, 3 for sibType3, etc. Values for SIBs generated by the gNB-DU as defined subclause 5.2.2 in TS 38.470 [2], values 6, 7, 8 and values corresponding to not defined SIBs in TS 38.331 [8] are not applicable in this version of the specifications. | - |  |
| >>SIB message | M |  | OCTET STRING | SIB as defined in subclause 6.3.1 in TS 38.331 [8]. | - |  |
| >>Value Tag | M |  | INTEGER (0..31, ...) |  | - |  |
| >>areaScope | O |  | ENUMERATED (true, …) | Indicates that a SIB is area specific. If the field is not present, the SIB is cell specific. | YES | ignore |
| SystemInformationAreaID | O |  | BIT STRING (SIZE (24)) | Indicates the system information area that the cell belongs to, if any. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSIBTypes | Maximum no. of SIB types, the maximum value is 32. |

#### 9.3.1.43	RAN UE Paging identity

This IE indicates the RAN UE Paging identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| I-RNTI | M |  | BIT STRING (SIZE(40)) |  |

#### 9.3.1.44	CN UE Paging Identity

The 5G-S-TMSI is used as UE identifier for CN paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE CN UE paging identity | M |  |  |  |
| >5G-S-TMSI |  |  |  |  |
| >>5G-S-TMSI | M |  | BIT STRING (SIZE(48)) | Details defined in TS 38.413 [3] |

#### 9.3.1.45	QoS Flow Level QoS Parameters

This IE defines the QoS to be applied to a QoS flow, or to a DRB, or to a BH RLC channel, or to a Uu Relay RLC channel, or to a PC5 Relay RLC channel.

NOTE:	For a BH RLC channel, the listed mandatory IEs and the GBR QoS Flow Information IE are applicable, where GBR QoS Flow Information IE may be present if BH RLC channel conveys the traffic belonging to a GBR QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  | - |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.49 |  | - |  |
| >Dynamic 5QI |  |  |  |  | - |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.47 |  | - |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.48 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.46 | This IE shall be present for GBR QoS Flows only and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | Details in TS 23.501 [21]. This IE applies to non-GBR flows only and is ignored otherwise. | - |  |
| PDU Session ID | O |  | INTEGER (0 ..255) | As specified in TS 23.501 [21]. | YES | ignore |
| UL PDU Session Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.22 | The PDU session Aggregate Maximum Bit Rate Uplink which is associated with the involved PDU session. | YES | ignore |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …, stop) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring. | YES | ignore |
| PDCP Terminating Node DL Transport Layer Address | O |  | Transport Layer Address 9.3.2.3 | DL Transport Layer Address of node terminating PDCP. Included for MN-terminated SCG bearers and SN-terminated MCG bearers. | YES | ignore |

#### 9.3.1.46	GBR QoS Flow Information

This IE indicates QoS parameters for a GBR QoS flow or GBR bearer for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.22 | Maximum Bit Rate in DL. Details in TS 23.501 [21]. | - |  |
| Maximum Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.22 | Maximum Bit Rate in UL. Details in TS 23.501 [21]. | - |  |
| Guaranteed Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.22 | Guaranteed Bit Rate (provided there is data to deliver) in DL. Details in TS 23.501 [21]. | - |  |
| Guaranteed Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.22 | Guaranteed Bit Rate (provided there is data to deliver). Details in TS 23.501 [21]. | - |  |
| Maximum Packet Loss Rate Downlink | O |  | Maximum Packet Loss Rate 9.3.1.50 | Indicates the maximum rate for lost packets that can be tolerated in the downlink direction. Details in TS 23.501 [21]. | - |  |
| Maximum Packet Loss Rate Uplink | O |  | Maximum Packet Loss Rate 9.3.1.50 | Indicates the maximum rate for lost packets that can be tolerated in the uplink direction. Details in TS 23.501 [21]. | - |  |
| Alternative QoS Parameters Set List | O |  | 9.3.1.125 | Indicates alternative sets of QoS Parameters for the QoS flow. | YES | ignore |

#### 9.3.1.47	Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a Non-standardised or not pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Priority Level | M |  | INTEGER (1..127) | For details see TS 23.501 [21]. | - |  |
| Packet Delay Budget | M |  | 9.3.1.51 | For details see TS 23.501 [21]. For IAB, the Packet Delay Budget defines the upper bound for the time that a packet may be delayed between the IAB-DU/IAB-donor-DU and its child IAB-MT, or between the IAB-DU and its served UE. For a PC5 Relay RLC channel, the Packet Delay Budget defines the upper bound for the time that a packet may be delayed between the L2 U2N relay UE and L2 U2N remote UE. For a Uu Relay RLC channel, the Packet Delay Budget defines the upper bound for the time that a packet may be delayed between the gNB-DU and L2 U2N relay UE. This IE is ignored if the Extended Packet Delay Budget IE is present. | - |  |
| Packet Error Rate | M |  | 9.3.1.52 | For details see TS 23.501 [21]. | - |  |
| 5QI | O |  | INTEGER (0..255,...) | This IE contains the dynamically assigned 5QI as specified in TS 23.501 [21]. | - |  |
| Delay Critical | C-ifGBRflow |  | ENUMERATED (delay critical, non-delay critical) | For details see TS 23.501 [21]. | - |  |
| Averaging Window | C-ifGBRflow |  | 9.3.1.53 | For details see TS 23.501 [21]. | - |  |
| Maximum Data Burst Volume | O |  | 9.3.1.54 | For details see TS 23.501 [21]. This IE shall be included if the Delay Critical IE is set to "delay critical" and is ignored otherwise. | - |  |
| Extended Packet Delay Budget | O |  | 9.3.1.145 | Packet Delay Budget is specified in TS 23.501 [21]. | YES | ignore |
| CN Packet Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.145 | Core Network Packet Delay Budget is specified in TS 23.501 [21]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| CN Packet Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.145 | Core Network Packet Delay Budget is specified in TS 23.501 [21]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE. |

#### 9.3.1.48	NG-RAN Allocation and Retention Priority

This IE specifies the relative importance of a QoS flow or a DRB compared to other QoS flows or DRBs for allocation and retention of NG-RAN resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Priority Level | M |  | INTEGER (0..15) | Desc.: This IE defines the relative importance of a resource request (see TS 23.501 [21]). Usage: Values are ordered in decreasing order of priority, i.e., with 1 as the highest priority and 15 as the lowest priority. Further usage is defined in TS 23.501 [21]. |
| Pre-emption Capability | M |  | ENUMERATED (shall not trigger pre-emption, may trigger pre-emption) | Desc.: This IE indicates the pre-emption capability of the request on other QoS flows (see TS 23.501 [21]). Usage: The QoS flow shall not pre-empt other QoS flows or, the QoS flow may pre-empt other QoS flows. Note: The Pre-emption Capability indicator applies to the allocation of resources for a QoS flow and as such it provides the trigger to the pre-emption procedures/processes of the gNB. |
| Pre-emption Vulnerability | M |  | ENUMERATED (not pre-emptable, pre-emptable) | Desc.: This IE indicates the vulnerability of the QoS flow to pre-emption of other QoS flows (see TS 23.501 [21]). Usage: The QoS flow shall not be pre-empted by other QoS flows or the QoS flow may be pre-empted by other QoS flows. Note: The Pre-emption Vulnerability indicator applies for the entire duration of the QoS flow, unless modified and as such indicates whether the QoS flow is a target of the pre-emption procedures/processes of thegNB. |

#### 9.3.1.49	Non Dynamic 5QI Descriptor

This IE indicates the QoS Characteristics for a standardized or pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0..255,...) | This IE contains the standardized or pre-configured 5QI as specified in TS 23.501 [21]. For a BH RLC channel, the Packet Delay Budget included in 5QI defines the upper bound for the time that a packet may be delayed between the gNB-DU and its child IAB-MT. | - |  |
| Priority Level | O |  | INTEGER (1..127) | For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. | - |  |
| Averaging Window | O |  | 9.3.1.53 | For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. | - |  |
| Maximum Data Burst Volume | O |  | 9.3.1.54 | For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. | - |  |
| CN Packet Delay Budget Downlink | O |  | Extended Packet Delay Budget 9.3.1.145 | Core Network Packet Delay Budget is specified in TS 23.501 [21]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| CN Packet Delay Budget Uplink | O |  | Extended Packet Delay Budget 9.3.1.145 | Core Network Packet Delay Budget is specified in TS 23.501 [21]. This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |

#### 9.3.1.50	Maximum Packet Loss Rate

This IE indicates the Maximum Packet Loss Rate.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum Packet Loss Rate | M |  | INTEGER(0..1000) | Ratio of lost packets per number of packets sent, expressed in tenth of percent. |

#### 9.3.1.51	Packet Delay Budget

This IE indicates the Packet Delay Budget for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Packet Delay Budget | M |  | INTEGER (0..1023, ...) | Upper bound value for the delay that a packet may experience expressed in unit of 0.5ms. |

#### 9.3.1.52	Packet Error Rate

This IE indicates the Packet Error Rate for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Scalar | M |  | INTEGER (0..9, ...) | The packet error rate is expressed as Scalar x 10-k where k is the Exponent. |
| Exponent | M |  | INTEGER (0..9, ...) |  |

#### 9.3.1.53	Averaging Window

This IE indicates the Averaging Window for a QoS flow, and applies to GBR QoS Flows only.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Averaging Window | M |  | INTEGER (0..4095, ...) | Unit: ms. The default value is 2000ms. |

#### 9.3.1.54	Maximum Data Burst Volume

This IE indicates the Maximum Data Burst Volume for a QoS flow, and applies to delay critical GBR QoS flows only.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Maximum Data Burst Volume | M |  | INTEGER (0..4095, ..., 4096.. 2000000) | Unit: byte. |

#### 9.3.1.55	Masked IMEISV

This information element contains the IMEISV value with a mask, to identify a terminal model without identifying an individual Mobile Equipment.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Masked IMEISV | M |  | BIT STRING (SIZE (64)) | Coded as the International Mobile station Equipment Identity and Software Version Number (IMEISV) defined in TS 23.003 [23] with the last 4 digits of the SNR masked by setting the corresponding bits to 1. The first to fourth bits correspond to the first digit of the IMEISV, the fifth to eighth bits correspond to the second digit of the IMEISV, and so on. |

#### 9.3.1.56	Notification Control

The Notification Control IE indicates whether the notification control for a given DRB is active or not-active.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Notification Control | M |  | ENUMERATED(Active, Not-Active, ...) |  |

#### 9.3.1.57	RAN Area Code

This information element is used to uniquely identify a RAN Area Code.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RANAC | M |  | INTEGER (0..255) | RAN Area Code |

#### 9.3.1.58	PWS System Information

This IE contains the system information used for public warning.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SIB type | M |  | INTEGER (6..8, …) | Indicates a certain SIB block for public warning message, e.g. 6 means sibType6, 7 for sibType7, etc. | - |  |
| SIB message | M |  | OCTET STRING | SIB message for public warning, as defined in TS 38.331 [8]. | - |  |
| Notification Information | O |  |  |  | YES | ignore |
| >Message Identifier | M |  | 9.3.1.81 |  | - |  |
| >Serial Number | M |  | 9.3.1.82 |  | - |  |
| Additional SIB Message List | O |  | 9.3.1.86 | Additional SIB messages containing different segments of a public warning message if segmentation is applied, as defined in TS 38.331 [8]. | YES | reject |

#### 9.3.1.59	Repetition Period

This IE indicates the periodicity of the warning message to be broadcast.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Repetition Period | M |  | INTEGER (0..217-1) | The unit of value 1 to 217-1 is [second]. |

#### 9.3.1.60	Number of Broadcasts Requested

This IE indicates the number of times a message is to be broadcast.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Number of Broadcasts Requested | M |  | INTEGER (0..65535) |  |

#### 9.3.1.61	Void

#### 9.3.1.62	SIType List

This IE is used by the gNB-CU to indicate to the gNB-DU to broadcast one or several SystemInformation messages including the Other SI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SI type item IEs |  | 1.. <maxnoofSITypes> |  |  |
| >SI Type | M |  | INTEGER (1..32, ...) | Value “1” corresponds to the SI message identified by the first SI message indicated in the SI-SchedulingInfo IE in the SIB1 message, value "2" to the SI message identified by the second SI message indicated in the SI-SchedulingInfo IE in the SIB1 message, and so on, as defined in TS 38.331 [8]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSITypes | Maximum no. of SI types, the maximum value is 32. |

#### 9.3.1.63	QoS Flow Identifier

This IE identifies a QoS Flow within a PDU Session. The definition and use of the QoS Flow Identifieris specified in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flow Identifier | M |  | INTEGER (0 ..63) |  |

#### 9.3.1.64	Served E-UTRA Cell Information

This IE contains served cell information of an E-UTRA cell for spectrum sharing between E-UTRA and NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE EUTRA-Mode-Info | M |  |  |  |
| >FDD |  |  |  |  |
| >>FDD Info |  | 1 |  |  |
| >>>UL Offset to Point A | M |  | INTEGER (0..2199,...) | Indicates the offset to the center of the NR carrier for UL. |
| >>>DL Offset to Point A | M |  | INTEGER (0..2199,...) | Indicates the offset to the center of the NR carrier for DL. |
| >TDD |  |  |  |  |
| >>TDD Info |  | 1 |  |  |
| >>>Offset to Point A | M |  | INTEGER (0..2199,...) | Indicates the offset to the center of the NR carrier. |
| Protected E-UTRA Resource Indication | O |  | OCTET STRING | Indicates the Protected E-UTRA Resource Indication as defined in subclause 9.2.125 of TS 36.423 [9]. |

#### 9.3.1.65	Available PLMN List

This IE indicates the list of available PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available PLMN Item IEs |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.3.1.14 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |

#### 9.3.1.66	RLC Failure Indication

This IE indicates the LCID associated with the RLC entity needing re-establishment.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Associated LCID | M |  | LCID 9.3.1.35 |  |

#### 9.3.1.67	Uplink TxDirectCurrentList Information

This IE contains the Uplink TxDirectCurrentList information that is configured by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Uplink TxDirectCurrentList Information | M |  | OCTET STRING | Includes the UplinkTxDirectCurrentList IE as defined in TS 38.331 [8]. |

#### 9.3.1.68	Service Status

This IE is used to indicate the service status of a cell by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Service State | M |  | ENUMERATED (In-Service, Out-Of-Service, ...) | Indicates the Service State of the cell. In-Service and Out-of-Service Service States are defined in TS 38.401 [4]. |
| Switching Off Ongoing | O |  | ENUMERATED (True, ...) | This IE indicates that the gNB-DU will delete the cell after some time using a new gNB-DU Configuration Update procedure. |

#### 9.3.1.69	RLC Status

This IE indicates about the RLC configuration change included in the container towards the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reestablishment Indication | O |  | ENUMERATED (reestablished, ...) | Indicates that following a change in the radio status, the RLC has been re-established. |

#### 9.3.1.70	RRC Version

This information element is used to identify RRC version corresponding to TS 38.331 [8].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Latest RRC Version | M |  | BIT STRING (SIZE (3)) | This IE is not used in this release. | - |  |
| Latest RRC Version Enhanced | O |  | OCTET STRING (SIZE (3)) | Latest supported RRC version in the release corresponding to TS 38.331 [8]. For a 3GPP specification version x.y.z, x is encoded by the leftmost byte, y by the middle byte, and z by the rightmost byte. If the RRC protocol is not supported in the gNB-DU, this IE is set to all ‘0’s. | YES | ignore |

#### 9.3.1.71	RRC Delivery Status

This IE provides information about the delivery status of RRC messages to the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Delivery Status | M |  | INTEGER (0..212-1) | Highest NR PDCP SN successfully delivered in sequence to the UE. |
| Triggering Message | M |  | INTEGER (0..212-1) | NR PDCP SN for the RRC message that triggered the report. |

#### 9.3.1.72	QoS Flow Mapping Indication

This IE is used to indicate only the uplink or downlink QoS flow is mapped to the DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| QoS Flow Mapping Indication | M |  | ENUMERATED(ul, dl,…) | Indicates that only the uplink or downlink QoS flow is mapped to the DRB |

#### 9.3.1.73	Resource Coordination Transfer Information

This IE contains information for UE-associated E-UTRA – NR resource coordination.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MeNB Cell ID | M |  | BIT STRING (SIZE(28)) | E-UTRAN Cell Identifier IE contained in the ECGI as defined in TS 36.423 [9] clause 9.2.14 |
| Resource Coordination E-UTRA Cell Information | O |  | 9.3.1.75 |  |

#### 9.3.1.74	E-UTRA PRACH Configuration

This IE indicates the PRACH resources used in E-UTRA cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RootSequenceIndex | M |  | INTEGER (0..837) | See section 5.7.2. in TS 36.211 [27] |
| ZeroCorrelationZoneConfiguration | M |  | INTEGER (0..15) | See section 5.7.2. in TS 36.211 [27] |
| HighSpeedFlag | M |  | BOOLEAN | TRUE corresponds to Restricted set and FALSE to Unrestricted set. See section 5.7.2 in TS 36.211 [27] |
| PRACH-FrequencyOffset | M |  | INTEGER (0..94) | See section 5.7.1 of TS 36.211 [27] |
| PRACH-ConfigurationIndex | C-ifTDD |  | INTEGER (0..63) | See section 5.7.1. in TS 36.211 [27] |

| Condition | Explanation |
| --- | --- |
| ifTDD | This IE shall be present if the EUTRA-Mode-Info IE in the Resource Coordination E-UTRA Cell Information IE is set to the value "TDD". |

#### 9.3.1.75	Resource Coordination E-UTRA Cell Information

This IE contains E-UTRA cell information for UE-associated E-UTRA – NR resource coordination.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE EUTRA-Mode-Info | M |  |  |  | - |  |
| >FDD |  |  |  |  | - |  |
| >>FDD Info |  | 1 |  |  | - |  |
| >>>UL EARFCN | O |  | INTEGER (0 .. maxExtendedEARFCN, ...) | The relation between EARFCN and carrier frequency (in MHz) is defined in TS 36.104 [25]. | - |  |
| >>>DL EARFCN | M |  | INTEGER (0 .. maxExtendedEARFCN, ...) | The relation between EARFCN and carrier frequency (in MHz) is defined in TS 36.104 [25]. | - |  |
| >>>UL Transmission Bandwidth | O |  | E-UTRA Transmission Bandwidth 9.3.1.80 | Present if UL EARFCN IE is present. | - |  |
| >>>DL Transmission Bandwidth | M |  | E-UTRA Transmission Bandwidth 9.3.1.80 |  | - |  |
| >TDD |  |  |  |  | - |  |
| >>TDD Info |  | 1 |  |  | - |  |
| >>>EARFCN | M |  | INTEGER (0 .. maxExtendedEARFCN, ...) | The relation between EARFCN and carrier frequency (in MHz) is defined in TS 36.104 [25]. | - |  |
| >>>Transmission Bandwidth | M |  | E-UTRA Transmission Bandwidth 9.3.1.80 |  | - |  |
| >>>Subframe Assignment | M |  | ENUMERATED(sa0, sa1, sa2, sa3, sa4, sa5, sa6,…) | Uplink-downlink subframe configuration information defined in TS 36.211 [27]. In NB-IOT, sa0 and sa6 are not applicable. | - |  |
| >>>Special Subframe Info |  | 1 |  | Special subframe configuration information defined in TS 36.211 [27] | - |  |
| >>>>Special Subframe Patterns | M |  | ENUMERATED(ssp0, ssp1, ssp2, ssp3, ssp4, ssp5, ssp6, ssp7, ssp8, ssp9, ssp10, …) |  | - |  |
| >>>>Cyclic Prefix DL | M |  | ENUMERATED(Normal, Extended,…) |  | - |  |
| >>>>Cyclic Prefix UL | M |  | ENUMERATED(Normal, Extended,…) |  | - |  |
| E-UTRA PRACH Configuration | M |  | 9.3.1.74 |  | - |  |
| Ignore PRACH Configuration | O |  | ENUMERATED (true,...) |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxExtendedEARFCN | Maximum value of extended EARFCN. Value is 262143. |

#### 9.3.1.76	Extended Available PLMN List

This IE indicates the list of available PLMN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended Available PLMN Item IEs |  | 1..< maxnoofExtendedBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.3.1.14 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |

#### 9.3.1.77	Associated SCell List

This IE indicates the list of SCells associated with the RLC entity indicated by the RLC Failure Indication IE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Associated SCell Item IEs |  | 1..< maxnoofSCells > |  |  | - | - |
| >SCell ID | M |  | NR CGI 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |

#### 9.3.1.78	Cell Direction

This IE indicates if the cell is either bidirectional or only DL or only UL.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Direction | M |  | ENUMERATED (dl-only, ul-only) |  |

#### 9.3.1.79	Paging Origin

This IE indicates whether Paging is originated due to the PDU sessions from the non-3GPP access.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Origin | M |  | ENUMERATED (non-3GPP, …) |  |

#### 9.3.1.80	E-UTRA Transmission Bandwidth

This IE is used to indicate the E-UTRA UL or DL transmission bandwidth expressed in units of resource blocks " NRB " (TS 36.104 [25]). The values bw1, bw6, bw15, bw25, bw50, bw75, bw100 correspond to the number of resource blocks "NRB" 6, 15, 25, 50, 75, 100.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| E-UTRA Transmission Bandwidth | M |  | ENUMERATED (bw6, bw15, bw25, bw50, bw75, bw100,... ) |  |

#### 9.3.1.81	Message Identifier

This IE identifies the warning message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Identifier | M |  | BIT STRING (SIZE(16)) | This IE is set by the 5GC, transferred to the UE by the gNB node. |

#### 9.3.1.82	Serial Number

This IE identifies a particular message from the source and type indicated by the Message Identifier and is altered every time the message with a given Message Identifier is changed.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Serial Number | M |  | BIT STRING (SIZE(16)) |  |

#### 9.3.1.83	UAC Assistance Information

This information element contains assistance information helping the gNB-DU to set parameters for Unified Access Class barring.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UAC PLMN List |  | 1 |  |  | - |  |
| >UAC PLMN Item |  | 1..<maxnoofUACPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >>UAC Type List |  | 1 |  |  | - |  |
| >>>UAC Type Item |  | 1..<maxnoofUACperPLMN> |  |  | - |  |
| >>>>UAC Reduction Indication | M |  | 9.3.1.85 |  | - |  |
| >>>>CHOICE UAC Category Type | M |  |  |  | - |  |
| >>>>>UAC Standardized |  |  |  |  | - |  |
| >>>>>>UAC Action | M |  | 9.3.1.84 |  | - |  |
| >>>>>UAC Operator Defined |  |  |  |  | - |  |
| >>>>>>Access Category | M |  | INTEGER (32..63, …) | Indicates the operator defined Access Category as defined in subclause 6.3.2 in TS 38.331 [8]. | - |  |
| >>>>>>Access Identity | M |  | BIT STRING (SIZE(7)) | Indicates whether access attempt is allowed for each Access Identity as defined in subclause 6.3.2 in TS 38.331 [8]. | - |  |
| >>NID | O |  | 9.3.1.155 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofUACPLMNs | Maximum no. of UAC PLMN Ids. Value is 12. |
| maxnoofUACperPLMN | Maximum no. of signalled categories per PLMN. Value is 64. |

#### 9.3.1.84	UAC Action

This IE indicates which signalling traffic is expected to be reduced by the gNB-CU, as defined in clause 8.7.7 of TS 38.413 [3]

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UAC Action | M |  | ENUMERATED (Reject RRC connection establishments for non-emergency MO DT, Reject RRC connection establishments for Signalling, Permit Emergency Sessions and mobile terminated services only, Permit High Priority Sessions and mobile terminated services only,…) |  |

#### 9.3.1.85		UAC reduction Indication

This IE indicates the percentage of signalling traffic expected to be reduced by the gNB-CU, relative to the instantaneous incoming rate from the gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UAC reduction Indication | M |  | INTEGER (0..100) | Value 0 indicates that no access rate reduction is desired. In this version of specification, value 99 indicates the highest desired rate reduction. |

#### 9.3.1.86	Additional SIB Message List

This IE indicates the list of additional SIB messages containing all the remaining segments of a public warning message if segmentation is applied to such message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Additional SIB Message List Item IEs |  | 1.. <maxnoofAdditionalSIBs > |  |  |
| >Additional SIB | M |  | OCTET STRING | SIB message containing one segment of a public warning message, as defined in TS 38.331 [8]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofAdditionalSIBs | Maximum no. of additional segments of a public warning message. Value is 63. |

#### 9.3.1.87	Cell Type

This IE provides the cell coverage area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Size | M |  | ENUMERATED (verysmall, small, medium, large, …) |  |

#### 9.3.1.87a	Configured TAC Indication

This IE indicates that the TAC with which this IE is associated, is only configured for the cell, but not broadcast.

NOTE:	This IE is defined in accordance to the possibility foreseen in TS 38.331 [8] to not broadcast the TAC if the NR cell only supports PSCell/SCell functionality.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Configured TAC Indication | M |  | ENUMERATED (true, ...) |  |

#### 9.3.1.88	Trace Activation

This IE defines parameters related to a trace session activation.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Trace ID | M |  | OCTET STRING (SIZE(8)) | This IE is composed of the following: Trace Reference defined in TS 32.422 [29] (leftmost 6 octets, with PLMN information encoded as in 9.3.1.14), and Trace Recording Session Reference defined in TS 32.422 [29] (last 2 octets). | - | - |
| Interfaces To Trace | M |  | BIT STRING (SIZE(8)) | Each position in the bitmap represents an NG-RAN node interface: first bit = NG-C, second bit = Xn-C, third bit = Uu, fourth bit = F1-C, fifth bit = E1: other bits reserved for future use. Value '1' indicates 'should be traced'. Value '0' indicates 'should not be traced'. | - | - |
| Trace Depth | M |  | ENUMERATED (minimum, medium, maximum, minimumWithoutVendorSpecificExtension, mediumWithoutVendorSpecificExtension, maximumWithoutVendorSpecificExtension, …) | Defined in TS 32.422 [29]. | - | - |
| Trace Collection Entity IP Address | M |  | Transport Layer Address 9.3.2.3 | For File based Reporting. Defined in TS 32.422 [29]. Should be ignored if URI is present. | - | - |
| MDT Configuration | O |  | 9.3.1.150 |  | YES | ignore |
| Trace Collection Entity URI | O |  | URI 9.3.2.6 | For Streaming based Reporting. Defined in TS 32.422 [29] Replaces Trace Collection Entity IP Address if present | YES | ignore |

#### 9.3.1.89	Intended TDD DL-UL Configuration

This IE contains the subcarrier spacing, cyclic prefix and TDD DL-UL slot configuration of an NR cell that the receiving gNB needs to take into account for cross-link interference mitigation, and/or for NR-DC power coordination, when operating its own cells.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …, scs 480, scs960) | The values scs15, scs30, scs60, scs120, scs480 and scs960 corresponds to the sub carrier spacing in TS 38.104 [17]. | - |  |
| NR Cyclic Prefix | M |  | ENUMERATED (Normal, Extended, …) | The type of cyclic prefix, which determines the number of symbols in a slot. | - |  |
| NR DL-UL Transmission Periodicity | M |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms3, ms4, ms5, ms10, ms20, ms40, ms60, ms80, ms100, ms120, ms140, ms160, …) | The periodicity is expressed in the format msXpYZ, and equals X.YZ milliseconds. | - |  |
| Slot Configuration List |  | 1 |  |  | - |  |
| >Slot Configuration List Item |  | 1..<maxnoofslots> |  |  | - |  |
| >>Slot Index | M |  | INTEGER (0..5119) |  | - |  |
| >>CHOICE Symbol Allocation in Slot | M |  |  |  | - |  |
| >>>All DL |  |  | NULL | This choice implies that all symbols in the slot are DL symbols. | - |  |
| >>>All UL |  |  | NULL | This choice implies that all symbols in the slot are UL symbols. | - |  |
| >>>Both DL and UL |  |  |  |  | - |  |
| >>>>Number of DL Symbols | M |  | INTEGER (0..13) | Number of consecutive DL symbols in the slot identified by Slot Index IE. If extended cyclic prefix is used, the maximum value is 11. The Permutation IE indicates the location of DL symbols in the slot. | - |  |
| >>>>Number of UL Symbols | M |  | INTEGER (0..13) | Number of consecutive UL symbols in the slot identified by Slot Index IE. If extended cyclic prefix is used, the maximum value is 11. The Permutation IE indicates the location of UL symbols in the slot. | - |  |
| >>>>Permutation | O |  | ENUMERATED (DFU, UFD, …) | If not present, the default value is DFU. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofslots | Maximum length of number of slots in a 10-ms period. Value is 5120. |

#### 9.3.1.90	Additional RRM Policy Index

The Additional RRM Policy Index IE is used to provide additional information independent from the Subscriber Profile ID for RAT/Frequency priority as specified in TS 36.300 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Additional RRM Policy Index | M |  | BIT STRING (32) |  |

#### 9.3.1.91	DU-CU RIM Information

This IE conveys the Remote Interference Management message from the gNB-DU to the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Victim gNB Set ID | M |  | 9.3.1.93 |  |
| RIM-RS Detection Status | M |  | ENUMERATED(RS detected, RS disappeared) | This IE indicates detection status of RIM-RS in gNB-DU |
| Aggressor Cell List |  | 1 |  |  |
| >Aggressor Cell List Item |  | 1..< maxCellingNBDU > |  |  |
| >>Aggressor Cell ID | M |  | NR CGI 9.3.1.12 |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |

#### 9.3.1.92	CU-DU RIM Information

This IE conveys the Remote Interference Management message from the gNB-CU to the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Victim gNB Set ID | M |  | 9.3.1.93 |  |
| RIM-RS Detection Status | M |  | ENUMERATED(RS detected, RS disappeared) | This IE indicates detection status of RIM-RS in remote gNB(s). |

#### 9.3.1.93	gNB Set ID

The gNB Set ID IE is used to identify a group of gNBs which transmit the same RIM-RS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB Set ID | M |  | BIT STRING (SIZE(22)) |  |

#### 9.3.1.94	Lower Layer Presence Status Change

This IE indicates lower layer resources’ presence status shall be changed.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Lower Layer Presence Status Change | M |  | ENUMERATED (suspend lower layers, resume lower layers ...) | "suspend lower layers" will store CellGroupConfig. From the parameters received within the ReconfigurationWithSync, only the sPCellConfigCommon is stored. "resume lower layers" shall restore SCG and it is set only after "suspend lower layers" has been indicated. |

#### 9.3.1.95	Traffic Mapping Information

This IE includes the information used by the gNB-DU to perform traffic mapping.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Traffic Mapping Information Type | M |  |  |  |
| >IP to layer2 Traffic Mapping Info |  |  |  |  |
| >>IP to layer2 Traffic Mapping Info To Add | O |  | IP-to-layer-2 traffic mapping Information List 9.3.1.96 | This IE indicates the mapping information for forwarding of IP traffic to layer-2 to be added. |
| >>IP to layer2 Traffic Mapping Info To Remove | O |  | Mapping Information to Remove 9.3.1.99 | This IE indicates the mapping information for forwarding of IP traffic to layer 2 to be removed. |
| >BAP layer BH RLC channel Mapping Info |  |  |  |  |
| >>BAP layer BH RLC channel Mapping Info To Add | O |  | BAP layer BH RLC channel mapping Information List 9.3.1.98 | This IE indicates the mapping information for forwarding of traffic on BAP layer to be added. |
| >>BAP layer BH RLC channel Mapping Info To Remove | O |  | Mapping Information to Remove 9.3.1.99 | This IE indicates the mapping information for forwarding of traffic on BAP layer to be removed. |

#### 9.3.1.96	IP-to-layer-2 traffic mapping Information List

This IE includes the information used by the IAB-donor-DU to perform the mapping from IP layer to layer-2. If this IE appears in the UE-associated F1AP signalling, the BH Information IE should only contain the BAP Routing ID IE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IP-to-layer-2 mapping information Item |  | 1.. <maxnoofMappingEntries> |  |  |
| >Mapping Information Index | M |  | 9.3.1.100 |  |
| >IP header information | M |  | 9.3.1.97 |  |
| >BH Information | M |  | 9.3.1.114 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMappingEntries | Maximum no. of mapping entries, the maximum value is 67108864 (i.e. 2^26). |

#### 9.3.1.97	IP Header Information

This IE indicates the IP header information included in the Traffic Mapping Information IE for DL traffic.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Destination IAB TNL Address | M |  | 9.3.1.102 | This IE indicates the destination IPv4 address, or IPv6 address or IPv6 prefix of a DL packet. |
| DS Information List |  | 0.. <maxnoofDSInfo> |  |  |
| >DSCP | M |  | BIT STRING (SIZE(6)) | This IE indicates the DS information of DL traffic. |
| IPv6 Flow Label | O |  | BIT STRING (SIZE(20)) | This IE indicates the IPv6 Flow Label of DL traffic. |

| Range bound | Explanation |
| --- | --- |
| maxnoofDSInfo | Maximum no. of DSCP values related to a destination IP address that can be mapped to one BH RLC channel, the maximum value is 64. |

#### 9.3.1.98	BAP layer BH RLC channel mapping Information List

This IE includes the information used by the IAB-DU to perform the BH RLC channel mapping when forwarding traffic on BAP sublayer.

When this IE is included in the UE-associated F1AP signalling for setting up or modifying a BH RLC channel, it contains either the Prior-Hop BAP Address IE and the Ingress BH RLC CH ID IE to configure a mapping in downlink direction, or the Next-Hop BAP address IE and the Egress BH RLC CH ID IE to configure a mapping in uplink direction. This IE indicates the BH RLC channel served by the collocated IAB-MT.

When this IE is included in the non-UE-associated F1AP signalling, it shall contain the Prior-Hop BAP Address IE, the Ingress BH RLC CH ID IE, the Next-Hop BAP address IE and the Egress BH RLC CH ID IE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| BAP layer BH RLC channel mapping info Item |  | 1.. <maxnoofMappingEntries> |  |  | - |  |
| >Mapping Information Index | M |  | 9.3.1.100 |  | - |  |
| >Prior-Hop BAP Address | O |  | 9.3.1.111 |  | - |  |
| >Ingress BH RLC CH ID | O |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >Next-Hop BAP Address | O |  | 9.3.1.111 |  | - |  |
| >Egress BH RLC CH ID | O |  | BH RLC Channel ID 9.3.1.113 |  | - |  |
| >Ingress Non-F1-terminating IAB-donor Topology Indicator | O |  | ENUMERATED (true, …) | If present, indicates that the ingress topology for this entry is the non-F1-terminating IAB-donor topology of the boundary IAB-node. | YES | ignore |
| >Egress Non-F1-terminating IAB-donor Topology Indicator | O |  | ENUMERATED (true, …) | If present, indicates that the egress topology for this entry is the non-F1-terminating IAB-donor topology of the boundary IAB-node. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMappingEntries | Maximum no. of mapping entries, the maximum value is 67108864 (i.e. 2^26). |

#### 9.3.1.99	Mapping Information to Remove

This IE includes a list of mapping information indexes corresponding to the mapping configuration which is to be removed.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Mapping Information to Remove List Item |  | 1.. <maxnoofMappingEntries> |  |  |
| >Mapping Information Index | M |  | 9.3.1.100 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMappingEntries | Maximum no. of mapping entries, the maximum value is 67108864 (i.e. 2^26). |

#### 9.3.1.100	Mapping Information Index

This IE includes an index of one mapping information entry at the IAB-donor-DU or an IAB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Mapping Information Index | M |  | BIT STRING (SIZE(26)) |  |

#### 9.3.1.101	IAB TNL Addresses Requested

The IAB TNL Addresses Requested IE indicates the number of IPv4 or IPv6 addresses or IPv6 address prefixes requested for the indicated usage.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TNL Addresses or Prefixes Requested - All Traffic | O |  | INTEGER (1..256) | The number of TNL addresses/ IPv6 prefixes requested for all traffic. |
| TNL Addresses or Prefixes Requested - F1-C traffic | O |  | INTEGER (1..256) | The number of TNL addresses/IPv6 prefixes requested for F1-C traffic. |
| TNL Addresses or Prefixes Requested - F1-U traffic | O |  | INTEGER (1..256) | The number of TNL addresses/ IPv6 prefixes requested for F1-U traffic. |
| TNL Addresses or Prefixes Requested - Non-F1 traffic | O |  | INTEGER (1..256) | The number of TNL addresses/ IPv6 prefixes requested for non-F1 traffic. |

#### 9.3.1.102	IAB TNL Address

The IAB TNL Address IE indicates an IPv4 or IPv6 address or an IPv6 address prefix assigned to an IAB-node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE IAB TNL Address | M |  |  |  |
| >IPv4 Address |  |  |  |  |
| >>IPv4 Address | M |  | BIT STRING (SIZE(32)) | The IPv4 address allocated to an IAB-node. |
| >IPv6 Address |  |  |  |  |
| >>IPv6 Address | M |  | BIT STRING (SIZE(128)) | The IPv6 address allocated to an IAB-node. |
| >IPv6 Prefix |  |  |  |  |
| >>IPv6 Prefix | M |  | BIT STRING (SIZE(64)) | The IPv6 address prefix allocated to an IAB-node. |

#### 9.3.1.103	Uplink BH Non-UP Traffic Mapping

This IE indicates the mapping of uplink non-UP traffic to a BH RLC channel and BAP Routing ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Uplink Non-UP Traffic Mapping List |  | 1 |  |  |
| >Uplink Non-UP Traffic Mapping List Item IEs |  | 1 .. <maxnoofNonUPTrafficMappings> |  |  |
| >>Non-UP Traffic Type | M |  | 9.3.1.104 |  |
| >>BH Information | M |  | 9.3.1.114 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNonUPTrafficMappings | Maximum no. of non-UP traffic mappings. Value is 32. |

#### 9.3.1.104	Non-UP Traffic Type

This IE indicates the type of non-UP traffic.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Non-UP Traffic Type | M |  | ENUMERATED(UE-associated F1AP, non-UE-associated F1AP, non-F1, BAP control PDU, ...) |  |

#### 9.3.1.105	IAB Info IAB-donor-CU

This IE contains cell-specific IAB-related information sent by an IAB-donor-CU to an IAB-DU or IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB STC Info | O |  | 9.3.1.109 | Contains STC configuration of IAB-DU or IAB-donor-DU. |

#### 9.3.1.106	IAB Info IAB-DU

This IE contains cell-specific IAB-related information sent by an IAB-DU or IAB-donor-DU to an IAB-donor-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Multiplexing Info | O |  | 9.3.1.108 | Contains the information about multiplexing with cells configured for a collocated IAB-MT. Applicable for an IAB-DU. |
| IAB STC Info | O |  | 9.3.1.109 | Contains the information about STC configuration of IAB-DU or IAB-donor-DU. |

#### 9.3.1.107	gNB-DU Cell Resource Configuration

This IE contains the resource configuration of the cells served by a gNB-DU, i.e. the TDD/FDD resource parameters for each activated cell (TS 38.213 [31], clause 11.1.1).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the TDD/FDD slot configuration. | - |  |
| DUF Transmission Periodicity | O |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, …) |  | - |  |
| DUF Slot Configuration List |  | 0..1 |  |  | - |  |
| >DUF Slot Configuration Item |  | 1..<maxnoofDUFSlots> |  | The maxNrofSlots in TS 38.331 [8]. | - |  |
| >>CHOICE DUF Slot Configuration | M |  |  |  | - |  |
| >>>Explicit Format |  |  |  |  |  |  |
| >>>>Permutation | M |  | ENUMERATED (DFU, UFD, …) |  | - |  |
| >>>>Number of Downlink Symbols | O |  | INTEGER (0..14) |  | - |  |
| >>>>Number of Uplink Symbols | O |  | INTEGER (0..14) |  | - |  |
| >>>Implicit Format |  |  |  |  |  |  |
| >>>>DUF Slot Format Index | M |  | INTEGER (0..254) | Index into Table 11.1.1-1 and Table 14-2 in TS 38.213 [31], excluding the last row in Table 14-2. | - |  |
| HSNA Transmission Periodicity | M |  | ENUMERATED (ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160, …) |  | - |  |
| HSNA Slot Configuration List |  | 0..1 |  |  | - |  |
| >HSNA Slot Configuration Item |  | 1..<maxnoofHSNASlots> |  |  | - |  |
| >>HSNA Downlink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot. | - |  |
| >>HSNA Uplink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot. | - |  |
| >>HSNA Flexible | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot. | - |  |
| RB Set Configuration | O |  | 9.3.1.230 |  | YES | reject |
| Frequency-Domain HSNA Configuration List |  | 0..1 |  |  | YES | reject |
| >Frequency-Domain HSNA Configuration Item |  | 1..<maxnoofRBsetsPerCell> |  |  | EACH | reject |
| >>RB Set Index | M |  | INTEGER (0.. maxnoofRBsetsPerCell-1, …) | Refers to an RB set defined by RB Set Configuration. The RB set indices are consecutive (and increasing) starting at 0. | - |  |
| >>Frequency-Domain HSNA Slot Configuration List |  | 1 |  |  | - |  |
| >>>Frequency-Domain HSNA Slot Configuration Item |  | 1..<maxnoofHSNASlots> |  |  | - |  |
| >>>>Slot Index | O |  | INTEGER (0..5119) | Indicates an index to a slot within the HSNA Transmission Periodicity. | - |  |
| >>>>HSNA Downlink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for downlink symbols in a slot, for an RB set. | - |  |
| >>>>HSNA Uplink | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for uplink symbols in a slot, for an RB set. | - |  |
| >>>>HSNA Flexible | O |  | ENUMERATED (HARD, SOFT, NOTAVAILABLE) | HSNA value for flexible symbols in a slot, for an RB set. | - |  |
| Child IAB-Nodes NA Resource List |  | 0..1 |  | List of child IAB-nodes served by the IAB-DU or IAB-donor-DU. | YES | reject |
| >Child IAB-Nodes NA Resource List Item |  | 1 .. <maxnoofChildIABNodes> |  |  | EACH | reject |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 | Identifier of a child-node IAB-MT at the IAB-donor-CU. | - |  |
| >>gNB-DU UE F1AP ID | M |  | 9.3.1.5 | Identifier of a child-node IAB-MT at an IAB-DU or IAB-donor-DU. | - |  |
| >>NA Resource Configuration List |  | 0..1 |  | List of not-available resources of this cell for this child IAB-node | - |  |
| >>>NA Resource Configuration Item |  | 1..<maxnoofHSNASlots> |  |  | - |  |
| >>>>NA Downlink | O |  | ENUMERATED (true, false, …) | Indicates whether downlink symbols, in a slot, are available to serve the child IAB-node. | - |  |
| >>>>NA Uplink | O |  | ENUMERATED (true, false, …) | Indicates whether uplink symbols, in a slot, are available to serve the child IAB-node. | - |  |
| >>>>NA Flexible | O |  | ENUMERATED (true, false, …) | Indicates whether flexible symbols, in a slot, are available to serve the child IAB-node. | - |  |
| Parent IAB Nodes NA Resource Configuration List |  | 0..1 |  | List of unavailable resources of this cell for this IAB-node. | YES | reject |
| >Parent IAB Nodes NA Resource Configuration Item |  | 1..<maxnoofHSNASlots> |  |  | EACH | reject |
| >>NA Downlink | O |  | ENUMERATED (true, false, …) | Indicates whether downlink symbols, in a slot, are unavailable to serve the IAB-node. | - |  |
| >>NA Uplink | O |  | ENUMERATED (true, false, …) | Indicates whether uplink symbols, in a slot, are unavailable to serve the IAB-node. | - |  |
| >>NA Flexible | O |  | ENUMERATED (true, false, …) | Indicates whether flexible symbols, in a slot, are unavailable to serve the IAB-node. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDUFSlots | Maximum no. of slots in 10ms. Value is 320. |
| maxnoofSymbols | Maximum no. of symbols in a slot. Value is 14. |
| maxnoofHSNASlots | Maximum no of "Hard", "Soft" or "Not available" slots in 160ms. Value is 5120. |
| maxnoofRBsetsPerCell | Maximum no. of RB sets per IAB-DU cell. Value is 8 |
| maxnoofRBsetsPerCell-1 | Maximum no. of RB sets per IAB-DU cell minus 1. Value is 7 |
| maxnoofChildIABNodes | Maximum number of child nodes served by an IAB-DU or an IAB-donor-DU. Value is 1024. |

#### 9.3.1.108	Multiplexing Info

This IE contains information about the multiplexing capabilities between the gNB-DU’s cell and the cells configured on the co-located IAB-MT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| IAB-MT Cell List |  | 1 |  |  | - |  |
| >IAB-MT Cell Item |  | 1 .. <maxnoofServingCells> |  |  | - |  |
| >>NR Cell Identity | M |  | BIT STRING (SIZE(36)) | Cell identity of a serving cell configured for a co-located IAB-MT. | - |  |
| >>DU_RX/MT_RX | M |  | ENUMERATED (supported, not supported) | An indication of whether the IAB-node supports simultaneous reception at its DU and MT side. | - |  |
| >>DU_TX/MT_TX | M |  | ENUMERATED (supported, not supported) | An indication of whether the IAB-node supports simultaneous transmission at its DU and MT side. | - |  |
| >>DU_RX/MT_TX | M |  | ENUMERATED (supported, not supported) | An indication of whether the IAB-node supports simultaneous reception at its DU and transmission at its MT side. | - |  |
| >>DU_TX/MT_RX | M |  | ENUMERATED (supported, not supported) | An indication of whether the IAB-node supports simultaneous transmission at its DU and reception at its MT side. | - |  |
| >>DU_RX/MT_RX_extend | MO |  | ENUMERATED (supported, not supported, supported and FDM required) | An indication of whether the IAB-node supports simultaneous reception at its DU and MT side. If present, the DU_RX/MT_RX IE shall be ignored. | YES | ignore |
| >>DU_TX/MT_TX_extend | O |  | ENUMERATED (supported, not supported, supported and FDM required) | An indication of whether the IAB-node supports simultaneous transmission at its DU and MT side. If present, the DU_TX/MT_TX IE shall be ignored. | YES | ignore |
| >>DU_RX/MT_TX_extend | O |  | ENUMERATED (supported, not supported, supported and FDM required) | An indication of whether the IAB-node supports simultaneous reception at its DU and transmission at its MT side. If present, the DU_RX/MT_TX IE shall be ignored. | YES | ignore |
| >>DU_TX/MT_RX_extend | O |  | ENUMERATED (supported, not supported, supported and FDM required) | An indication of whether the IAB-node supports simultaneous transmission at its DU and reception at its MT side. If present, the DU_TX/MT_RX IE shall be ignored. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofServingCells | Maximum no. of serving cells for IAB-MT. Value is 32, as defined by the maxNrofServingCells in TS 38.331 [8]. |

#### 9.3.1.109	IAB STC Info

This IE contains cell SSB Transmission Configuration (STC) information of an IAB-DU or IAB-donor-DU. The information is used by neighbour IAB-MTs for discovery and measurements of this IAB-DU or IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB STC-Info List |  | 1 |  |  |
| >IAB STC-Info Item |  | 1 ..<maxnoofIABSTCInfo> |  |  |
| >>SSB Frequency Info | M |  | INTEGER (0.. maxNRARFCN) | The SSB central frequency. |
| >>SSB Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, …) | The SSB subcarrier spacing. |
| >>SSB Transmission Periodicity | M |  | ENUMERATED (sf10, sf20, sf40, sf80, sf160, sf320, sf640, ..., sf5) |  |
| >>SSB Transmission Timing Offset | M |  | INTEGER (0.. 127, …) | SSB transmission timing offset in number of half-frames. |
| >>CHOICE SSB Transmission Bitmap | M |  |  | The SSB-ToMeasure IE defined in TS 38.331 [8]. |
| >>>Short Bitmap |  |  |  |  |
| >>>>Short Bitmap | M |  | BIT STRING (SIZE (4)) |  |
| >>>Medium Bitmap |  |  |  |  |
| >>>>Medium Bitmap | M |  | BIT STRING (SIZE (8)) |  |
| >>>Long Bitmap |  |  |  |  |
| >>>>Long Bitmap | M |  | BIT STRING (SIZE (64)) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofIABSTCInfo | Maximum no. of STC configurations. Value is 5. This includes 1 STC configuration for access and 4 STC configurations for backhaul. |
| maxNRARFCN | Maximum value of NR ARFCNs. Value is 3279165. |

#### 9.3.1.110	BAP Routing ID

This IE indicates the BAP Routing ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BAP Address | M |  | 9.3.1.111 |  |
| Path ID | M |  | BAP Path ID 9.3.1.112 |  |

#### 9.3.1.111	BAP Address

This IE indicates the BAP address of an IAB-node or of an IAB-donor-DU, and it is part of the BAP Routing ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BAP Address | M |  | BIT STRING (SIZE(10)) | Corresponds to the bap-Address contained in the RRCReconfiguration message or contained in the BAP-RoutingID IE, or the iab-donor-DU-BAP-Address contained in the RRCReconfiguration message defined TS 38.331[8]. |

#### 9.3.1.112	BAP Path ID

This IE indicates the BAP path ID, which is part of the BAP Routing ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BAP Path ID | M |  | BIT STRING (SIZE(10)) | Corresponds to the bap-Pathid contained in the BAP-RoutingID IE defined in subclause 6.3.2 of TS 38.331 [8]. |

#### 9.3.1.113	BH RLC Channel ID

This IE uniquely identifies a BH RLC channel in the link between IAB-MT of the IAB-node and IAB-DU of the parent IAB-node or IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BH RLC CH ID | M |  | BIT STRING (SIZE(16)) |  |

#### 9.3.1.114	BH Information

This IE includes the backhaul information for UL or DL.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| BAP Routing ID | O |  | 9.3.1.110 | This IE is not needed for the BAP control PDU. For UL F1-U traffic, the BAP address included in this IE also indicates the IAB-donor-DU via which the DL traffic is transmitted. | - |  |
| Egress BH RLC CH List |  | 0..1 |  |  | - |  |
| >Egress BH RLC CH List Item |  | 1.. <maxnoofEgressLinks> |  |  | - |  |
| >>Next-Hop BAP Address | M |  | 9.3.1.111 | This IE identifies the next-hop node on the backhaul path to receive the packet. The value of this IE should be unique in the whole list. | - |  |
| >>Egress BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 | This IE identifies the BH RLC channel in the link between the IAB node/IAB-donor-DU and the node identified by the Next-Hop BAP Address IE. | - |  |
| Non-F1-Terminating IAB-donor Topology Indicator | O |  | ENUMERATED (true, …) | If present, indicates that the Next-Hop BAP Address and Egress BH RLC CH ID contained in this IE pertain to the non-F1-terminating IAB-donor topology of the boundary IAB-node. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofEgressLinks | Maximum no. of egress links. Value is 2. |

#### 9.3.1.115	Control Plane Traffic Type

This IE indicates the control plane traffic type carried over a BH RLC channel.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Control Plane Traffic Type | M |  | INTEGER (1..3, ...) | Control plane traffic types with different priorities are identified by the different codepoints in this IE, where 1 has the highest priority. |

#### 9.3.1.116	NR V2X Services Authorized

This IE provides information on the authorization status of the UE to use the NR sidelink for V2X services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Vehicle UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as Vehicle UE. |
| Pedestrian UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as Pedestrian UE. |

#### 9.3.1.117	LTE V2X Services Authorized

This IE provides information on the authorization status of the UE to use the LTE sidelink for V2X services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Vehicle UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as Vehicle UE. |
| Pedestrian UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized as Pedestrian UE. |

#### 9.3.1.118	LTE UE Sidelink Aggregate Maximum Bit Rate

This IE provides information on the Aggregate Maximum Bitrate of the UE’s communication over LTE sidelink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LTE UE Sidelink Aggregate Maximum Bit Rate | M |  | Bit Rate 9.3.1.4 | Value 0 shall be considered as a logical error by the receiving gNB-DU. |

#### 9.3.1.119	NR UE Sidelink Aggregate Maximum Bit Rate

This IE provides information on the Aggregate Maximum Bitrate of the UE’s communication over NR sidelink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR UE Sidelink Aggregate Maximum Bit Rate | M |  | Bit Rate 9.3.1.4 | Value 0 shall be considered as a logical error by the receiving gNB-DU. |

#### 9.3.1.120	SL DRB ID

This IE uniquely identifies a SL DRB for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SL DRB ID | M |  | INTEGER (1.. 512, ...) | Corresponds to the SLRB-Uu-ConfigIndex IE defined in TS 38.331 [8]. |

#### 9.3.1.121	PC5 QoS Flow Identifier

This IE uniquely identifies one sidelink QoS flow between the UE and the network in the scope of UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PC5 QoS Flow Identifier | M |  | INTEGER (1.. 2048) | Corresponds to the SL-QoS-FlowIdentity IE defined in TS 38.331 [8]. |

#### 9.3.1.122	PC5 QoS Parameters

This IE defines the QoS to be applied to a SL DRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE PC5 QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic PQI |  |  |  |  | - |  |
| >>Non Dynamic PQI Descriptor | M |  | 9.3.1.126 |  | - |  |
| >Dynamic PQI |  |  |  |  | - |  |
| >>Dynamic PQI Descriptor | M |  | 9.3.1.127 |  | - |  |
| PC5 QoS Flow Bit Rates | O |  |  | Only applies for GBR QoS Flows. | - |  |
| >Guaranteed Flow Bit Rate | M |  | Bit Rate 9.3.1.22 | Guaranteed Bit Rate for the PC5 QoS flow. Details in TS 23.287 [40]. | - |  |
| >Maximum Flow Bit Rate | M |  | Bit Rate 9.3.1.22 | Maximum Bit Rate for the PC5 QoS flow. Details in TS 23.287 [40]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flows allowed towards one UE for NR sidelink communication, the maximum value is 2048. |

#### 9.3.1.123	Alternative QoS Parameters Set Index

This IE indicates the QoS parameters set which can currently be fulfilled.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Index | M |  | INTEGER (1..8, ...) | Indicates the index of the item within the Alternative QoS Parameters Set List IE corresponding to the currently fulfilled alternative QoS parameters set. |

#### 9.3.1.124	Alternative QoS Parameters Set Notify Index

This IE indicates the QoS parameters set which can currently be fulfilled.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Notify Index | M |  | INTEGER (0..8, ...) | Indicates the index of the item within the the Alternative QoS Parameters Set List IE corresponding to the currently fulfilled alternative QoS parameters set. Value 0 indicates that NG-RAN cannot even fulfil the lowest alternative parameter set. |

#### 9.3.1.125	Alternative QoS Parameters Set List

This IE contains alternative sets of QoS parameters which the gNB can indicate to be fulfilled when notification control is enabled and it cannot fulfil the requested list of QoS parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Item |  | 1..<maxnoofQoSParaSets> |  |  |
| >Alternative QoS Parameters Set Index | M |  | 9.3.1.123 |  |
| >Guaranteed Flow Bit Rate Downlink | O |  | Bit Rate 9.3.1.22 |  |
| >Guaranteed Flow Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 |  |
| >Packet Delay Budget | O |  | 9.3.1.51 |  |
| >Packet Error Rate | O |  | 9.3.1.52 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSParaSets | Maximum no. of alternative sets of QoS Parameters allowed for the QoS profile. Value is 8. |

#### 9.3.1.126	Non Dynamic PQI Descriptor

This IE indicates the QoS Characteristics for a standardized or pre-configured PQI for sidelink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0..255,...) | This IE contains the standardized or pre-configured PQI as specified in TS 23.287 [40] |
| QoS Priority Level | O |  | INTEGER (1..8,…) | For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. |
| Averaging Window | O |  | 9.3.1.53 | For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. |
| Maximum Data Burst Volume | O |  | 9.3.1.54 | For details see TS 23.501 [21]. When included overrides standardized or pre-configured value. |

#### 9.3.1.127	Dynamic PQI Descriptor

This IE indicates the QoS Characteristics for a Non-standardised or not pre-configured PQI for sidelink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Resource Type | O |  | ENUMERATED (GBR, non-GBR, delay critical GBR, …) |  |
| QoS Priority Level | M |  | INTEGER (1..8, …) | For details see TS 23.501 [21]. |
| Packet Delay Budget | M |  | 9.3.1.51 | For details see TS 23.501 [21]. |
| Packet Error Rate | M |  | 9.3.1.52 | For details see TS 23.501 [21]. |
| Averaging Window | C-ifGBRflow |  | 9.3.1.53 | For details see TS 23.501 [21]. |
| Maximum Data Burst Volume | O |  | 9.3.1.54 | For details see TS 23.501 [21]. This IE shall be included if the Delay Critical IE is set to "delay critical" and is ignored otherwise. |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the PC5 QoS Flow Bit Rates IE is present in the PC5 QoS parameters IE. |

#### 9.3.1.128	TNL Capacity Indicator

The TNL Capacity Indicator IE indicates the offered and available capacity of the Transport Network experienced by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL TNL Offered Capacity | M |  | INTEGER (1.. 16777216,...) | Maximum capacity offered by the transport portion of the gNB-DU – gNB-CU in kbps |
| DL TNL Available Capacity | M |  | INTEGER (0.. 100,...) | Available capacity over the transport portion serving the node in percentage. Value 100 corresponds to the offered capacity |
| UL TNL Offered Capacity | M |  | INTEGER (1.. 16777216,...) | Maximum capacity offered by the transport portion of the gNB-DU – gNB-CU in kbps |
| UL TNL Available Capacity | M |  | INTEGER (0.. 100,...) | Available capacity over the transport portion serving the node in percentage. Value 100 corresponds to the offered capacity |

#### 9.3.1.129	Radio Resource Status

The Radio Resource Status IE indicates the usage of the PRBs per cell for MIMO, per SSB area and per slice for all traffic in Downlink and Uplink and the usage of PDCCH CCEs for Downlink and Uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Area Radio Resource Status List |  | 1 |  |  | - |  |
| >SSB Area Radio Resource Status Item |  | 1..<maxnoofSSBAreas> |  |  | - |  |
| >>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>SSB Area DL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL GBR PRB usage in percentage of the cell total PRB number. | - |  |
| >>SSB Area UL GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL GBR PRB usage in percentage of the cell total PRB number. | - |  |
| >>SSB Area DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area DL non-GBR PRB usage in percentage of the cell total PRB number. | - |  |
| >>SSB Area UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per SSB area UL non-GBR PRB usage in percentage of the cell total PRB number. | - |  |
| >>SSB Area DL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area DL Total PRB usage in percentage of the cell total PRB number. | - |  |
| >>SSB Area UL Total PRB usage | M |  | INTEGER (0..100) | Per SSB area UL Total PRB usage in percentage of the cell total PRB number. | - |  |
| >>DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  | - |  |
| >>UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  | - |  |
| Slice Radio Resource List |  | 0..1 |  |  | YES | ignore |
| >Slice Radio Resource Item |  | 1..< maxnoofBPLMNsNR > |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.1.14 | Broadcast PLMN | - |  |
| >>S-NSSAI Radio Resource Status List |  | 1 |  |  | - |  |
| >>>S-NSSAI Radio Resource Status Item |  | 1..<maxnoofSliceItems> |  |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>S-NSSAI DL GBR PRB usage | M |  | INTEGER (0..100) | Per slice DL GBR PRB usage in percentage of the cell total PRB number. | - |  |
| >>>>S-NSSAI UL GBR PRB usage | M |  | INTEGER (0..100) | Per slice UL GBR PRB usage for this slice in percentage of the cell total PRB number. | - |  |
| >>>>S-NSSAI DL non-GBR PRB usage | M |  | INTEGER (0..100) | Per slice DL non-GBR PRB usage for this slice in percentage of the cell total PRB number. | - |  |
| >>>>S-NSSAI UL non-GBR PRB usage | M |  | INTEGER (0..100) | Per slice UL non-GBR PRB usage for this slice in percentage of the cell total PRB number. | - |  |
| >>>>Slice DL Total PRB allocation | M |  | INTEGER (0..100) | Total amount of DL PRBs available per cell for this slice if all the resources the slice could access were usable. | - |  |
| >>>>Slice UL Total PRB allocation | M |  | INTEGER (0..100) | Total amount of UL PRBs available per cell for this slice if all the resources the slice could access were usable. | - |  |
| MIMO PRB usage Information | O |  |  |  | YES | ignore |
| >DL GBR PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell DL GBR PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [32]. | - |  |
| >UL GBR PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell UL GBR PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [32]. | - |  |
| >DL non-GBR PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell DL non-GBR PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [32]. | - |  |
| >UL non-GBR PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell UL non-GBR PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [32]. | - |  |
| >DL Total PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell DL Total PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [32]. | - |  |
| >UL Total PRB usage for MIMO | M |  | INTEGER (0..100) | Per cell UL Total PRB usage for MIMO in percentage of the cell total PRB number as defined in TS 38.314 [32]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

#### 9.3.1.130	Composite Available Capacity Group

The Composite Available Capacity Group IE indicates the overall available resource level per cell and per SSB area in the cell in Downlink and Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Composite Available Capacity Downlink | M |  | Composite Available Capacity 9.3.1.131 | For the Downlink | - |  |
| Composite Available Capacity Uplink | M |  | Composite Available Capacity 9.3.1.131 | For the Uplink, including both NUL and SUL (if available) | - |  |
| Composite Available Capacity Supplementary Uplink | O |  | Composite Available Capacity 9.3.1.131 | For the SUL | YES | ignore |

#### 9.3.1.131	Composite Available Capacity

The Composite Available Capacity IE indicates the overall available resource level in the cell in either Downlink or Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Capacity Class Value | O |  | 9.3.1.132 |  |
| Capacity Value | M |  | 9.3.1.133 | ‘0’ indicates no resource is available, Measured on a linear scale. |

#### 9.3.1.132	Cell Capacity Class Value

The Cell Capacity Class Value IE indicates the value that classifies the cell capacity with regards to the other cells. The Cell Capacity Class Value IE only indicates resources that are configured for traffic purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Capacity Class Value | M |  | INTEGER (1..100,...) | Value 1 shall indicate the minimum cell capacity, and 100 shall indicate the maximum cell capacity. There should be a linear relation between cell capacity and Cell Capacity Class Value. |

#### 9.3.1.133	Capacity Value

The Capacity Value IE indicates the amount of resources per cell and per SSB area that are available relative to the total gNB-DU resources. The capacity value should be measured and reported so that the minimum gNB-DU resource usage of existing services is reserved according to implementation. The Capacity Value IE can be weighted according to the ratio of cell capacity class values, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity with respect to the whole cell. Capacity Value should be measured on a linear scale. |
| SSB Area Capacity Value List |  | 0..1 |  |  |
| >SSB Area Capacity Value Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>SSB Index | M |  | INTEGER (0..63) |  |
| >>SSB Area Capacity Value | M |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity . SSB Area Capacity Value should be measured on a linear scale. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

#### 9.3.1.134	Slice Available Capacity

The Slice Available Capacity IE indicates the amount of resources per network slice that are available per cell relative to the total gNB-DU resources per cell. The Slice Available Capacity Value Downlink IE and the Slice Available Capacity Value Uplink IE can be weighted according to the ratio of the corresponding cell capacity class values contained in the Composite Available Capacity Group IE, if available.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Available Capacity List |  | 1 |  |  |
| >Slice Available Capacity Item |  | 1..< maxnoofBPLMNsNR > |  |  |
| >>PLMN Identity | M |  | 9.3.1.14 | Broadcast PLMN |
| >>S-NSSAI Available Capacity List |  | 1 |  |  |
| >>>S-NSSAI Available Capacity Item | M | 1 .. < maxnoofSliceItems> |  |  |
| >>>>S-NSSAI |  |  | 9.3.1.38 |  |
| >>>>Slice Available Capacity Value Downlink | O |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity . Slice Available Capacity Value Downlink should be measured on a linear scale. |
| >>>>Slice Available Capacity Value Uplink | O |  | INTEGER (0..100) | Value 0 shall indicate no available capacity, and 100 shall indicate maximum available capacity . Slice Available Capacity Value Uplink should be measured on a linear scale. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in a cell. Value is 12. |

#### 9.3.1.135	Number of Active UEs

The Number of Active UEs IE indicates the mean number of active UEs as defined in TS 38.314 [32].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Mean number of Active UEs | M |  | INTEGER (0..16777215, ...) | As defined in TS 38.314 [32] and where value "1" is equivalent to 0.1 Active UEs, value "2" is equivalent to 0.2 Active UEs, value n is equivalent to n/10 Active UEs. |

#### 9.3.1.136	Hardware Load Indicator

The Hardware Load Indicator IE indicates the status of the Hardware Load.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL Hardware Load Indicator | M |  | INTEGER (0..100) | This indicates the load in percent |
| UL Hardware Load Indicator | M |  | INTEGER (0..100) | This indicates the load in percent |

#### 9.3.1.137	NR Carrier List

This IE indicates the SCS-specific carriers per TDD, per DL, per UL or per SUL of an NR cell.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR Carrier Item |  | 1..<maxnoofNRSCSs> |  |  |
| >NR SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …, scs480, scs960) | SCS for the corresponding carrier. |
| >Offset to Carrier | M |  | INTEGER (0.. 2199, ...) | Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the NR SCS IE defined for this carrier). The maximum value corresponds to 275×8−1. See TS 38.211 [33], clause 4.4.2. |
| >Carrier Bandwidth | M |  | INTEGER (1.. maxnoofPhysicalResourceBlocks, ...) | Width of this carrier in number of PRBs (using the NR SCS IE defined for this carrier). See TS 38.211 [33], clause 4.4.2. |

| Range bound | Explanation |
| --- | --- |
| maxnoofNRSCSs | Maximum no. of SCS-specific carriers per TDD, per DL, per UL or per SUL of an NR cell. Value is 5. |
| maxnoofPhysicalResourceBlocks | Maximum no. of Physical Resource Blocks. Value is 275. |

#### 9.3.1.138	SSB Positions In Burst

Indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [31], clause 4.1.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE ssb-PositionsInBurst | M |  |  | The first/ leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted. |
| >ShortBitmap |  |  |  |  |
| >>ShortBitmap | M |  | BIT STRING (SIZE(4)) |  |
| >MediumBitmap |  |  |  |  |
| >>MediumBitmap | M |  | BIT STRING (SIZE(8)) |  |
| >LongBitmap |  |  |  |  |
| >>LongBitmap | M |  | BIT STRING (SIZE(64)) |  |

#### 9.3.1.139	NR PRACH Configuration

This IE indicates the PRACH resources by a NR cell.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL PRACH Configuration | M |  | NR PRACH Configuration List 9.3.1.140 |  |
| SUL PRACH Configuration | O |  | NR PRACH Configuration List 9.3.1.140 |  |

#### 9.3.1.140	NR PRACH Configuration List

This IE indicates the PRACH resources used or reserved in the UL carrier(s) or SUL carrier(s) of the current NR cell.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR PRACH Configuration Item |  | 0..< maxnoofPrachConfiguration > |  | Length=0 means releasing of all NR PRACH Configuration Items for this UL or SUL. | - |  |
| >NR SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …, scs 480, scs960) | The SCS of the carrier to which this PRACH Configuration Item relates, i.e. Δf in Section 5.3.2 in TS 38.211 [33]. The values scs15, scs30, scs60, scs120, scs 480, and scs960 corresponds to the sub carrier spacing in TS 38.104 [17]. NOTE: Its value may not be identical to the SCS of PRACH. | - |  |
| >PRACH Frequency Start from Carrier | M |  | INTEGER (0.. maxNrofPhysicalResourceBlocks-1, …) | Lowest number of resource blocks which can be used to deliver MSG1 or the preamble part of MSGA, counting from the start number of the corresponding carrier. Identical to in Section 5.1.2.2.2 in TS 38.214 [34] plus msg1-FrequencyStart or msgA-RO-FrequencyStart-r16 in TS 38.331 [8]. | - |  |
| >PRACH-FDM | M |  | ENUMERATED (one, two, four, eight, …) | M in Section 6.3.3.2 in TS 38.211 [33]. | - |  |
| >PRACH Configuration Index | M |  | INTEGER (0.. 255, …, 256..262) | See Section 6.3.3.2 in TS 38.211 [33]. | - |  |
| >SSB per RACH Occasion | M |  | ENUMERATED (oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen, ...) | Number of SSBs per RACH occasion. Value oneEight corresponds to one SSB associated with 8 RACH occasions, value oneFourth corresponds to one SSB associated with 4 RACH occasions, and so on. | - |  |
| >CHOICE FreqDomainLength | M |  |  | For the case of PRACH resources reserved for BFR or MSG1-based SI Request, L139 is always used. | - |  |
| >>L839 |  |  |  |  |  |  |
| >>>L839 Info |  | 1 |  |  | - |  |
| >>>>Root Sequence Index | M |  | INTEGER (0..837) | See Section 6.3.3.1 in TS 38.211 [33]. | - |  |
| >>>>Restricted Set Config | M |  | ENUMERATED (unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB, …) | See Section 6.3.3.1 in TS 38.211 [33]. | - |  |
| >>L139 |  |  |  |  |  |  |
| >>>L139 Info |  | 1 |  |  | - |  |
| >>>>PRACH SCS | M |  | ENUMERATED (scs15, scs30, scs60, scs120, …, scs480, scs960) | Subcarrier Spacing of PRACH, i.e. ΔfRA in Section 5.3.2 in TS 38.211 [33]. | - |  |
| >>>>Root Sequence Index | O |  | INTEGER (0..137) | See Section 6.3.3.1 in TS 38.211 [33]. | - |  |
| >>L571 |  |  |  |  | YES | reject |
| >>>L571 Info |  | 1 |  |  | - |  |
| >>>>PRACH SCS for L571 | M |  | ENUMERATED (scs30, scs120, … , scs480) | Subcarrier Spacing of PRACH, i.e. ΔfRA in Section 5.3.2 in TS 38.211 [33]. | - |  |
| >>>>Root Sequence Index | M |  | INTEGER (0..569) | See Section 6.3.3.1 in TS 38.211 [33]. | - |  |
| >>L1151 |  |  |  |  | YES | reject |
| >>>L1151 Info |  | 1 |  |  | - |  |
| >>>>PRACH SCS for L1151 | M |  | ENUMERATED (scs15, scs120, …) | Subcarrier Spacing of PRACH, i.e. ΔfRA in Section 5.3.2 in TS 38.211 [33]. | - |  |
| >>>>Root Sequence Index | M |  | INTEGER (0..1149) | See Section 6.3.3.1 in TS 38.211 [33]. | - |  |
| >Zero Correlation Zone Config | M |  | INTEGER (0..15) | See Section 6.3.3.1 in TS 38.211 [33]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPhysicalResourceBlocks-1 | Maximum no. of Physical Resource Blocks minus 1. Value is 274. |
| maxnoofPrachConfiguration | Maximum no. of PRACH Configuration. Value is 16. |

#### 9.3.1.141	TSC Traffic Characteristics

This IE provides the traffic characteristics of TSC QoS flows.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TSC Assistance Information Downlink | O |  | TSC Assistance Information 9.3.1.142 |  |
| TSC Assistance Information Uplink | O |  | TSC Assistance Information 9.3.1.142 |  |

#### 9.3.1.142	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [21]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.143 | Periodicity as specified in TS 23.501 [21]. | - |  |
| Burst Arrival Time | O |  | 9.3.1.144 | Burst Arrival Time as specified in TS 23.501 [21]. | - |  |
| Survival Time | O |  | 9.3.1.231 |  | YES | ignore |

#### 9.3.1.143	Periodicity

This IE indicates the Periodicity as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | INTEGER (0..640000, …) | Periodicity expressed in units of 1 us. |

#### 9.3.1.144	Burst Arrival Time

This IE indicates the Burst Arrival Time as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Burst Arrival Time | M |  | OCTET STRING | Encoded in the same format as the ReferenceTime IE as defined in TS 38.331 [8]. The value is provided with 1 us accuracy. |

#### 9.3.1.145	Extended Packet Delay Budget

This IE indicates the Packet Delay Budget for a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended Packet Delay Budget | M |  | INTEGER (0..65535, …, 65536..109999) | Upper bound value for the delay that a packet may experience expressed in unit of 0.01ms. |

#### 9.3.1.146	RLC Duplication Information

The IE contains the RLC duplication information in case that the indicated DRB is configured with more than two RLC entities as specified in TS 38.331 [8].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RLC Duplication State List |  | 1 |  |  |
| >RLC Duplication State Items |  | 1 .. <maxnoofRLCDuplicationState> |  | Each position in the list represents a secondary RLC entity in ascending order by the logical channel ID in the order of MCG and SCG. |
| >>Duplication State | M |  | ENUMERATED (Active, Inactive, ...) |  |
| Primary Path Indication | O |  | ENUMERATED (True, False,...) | Indicates whether the primary path is located at the gNB-DU for DC based PDCP duplication. |

| Range bound | Explanation |
| --- | --- |
| maxnoofRLCDuplicationState | Maximum no of Secondary RLC entities. Value is 3. |

#### 9.3.1.147	Reporting Request Type

This IE indicates the type of accurate reference time information reporting to be handled by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Event Type | M |  | ENUMERATED (on demand, periodic, stop, …) |  |
| Report Periodicity Value | C-ifEventTypeisPeriodic |  | INTEGER (0..512, …) | Indicates the periodicity of accurate reference time information report, Unit in radio frame. |

| Condition | Explanation |
| --- | --- |
| ifEventTypeisPeriodic | This IE shall be present if the Event Type IE is set to "periodic". |

#### 9.3.1.148	Time Reference Information

This IE contains the time reference information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reference Time | M |  | 9.3.1.149 |  |
| Reference SFN | M |  | INTEGER (0..1023) | Corresponds to the referenceSFN contained in the ReferenceTimeInfo IE, as defined in TS 38.331 [8] |
| Uncertainty | O |  | INTEGER (0..32767, …) | Corresponds to the uncertainty contained in the ReferenceTimeInfo IE, as defined in TS 38.331 [8]. |
| Time Information Type | O |  | ENUMERATED (localClock) | Corresponds to the timeInfoType contained in the ReferenceTimeInfo IE, as defined in TS 38.331 [8]. |

#### 9.3.1.149	Reference Time

This IE provides the accurate Reference Time information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reference Time | M |  | OCTET STRING | Includes the ReferenceTime IE contained in the ReferenceTimeInfo IE as defined in TS 38.331 [8]. |

#### 9.3.1.150	MDT Configuration

The IE defines the MDT configuration parameters.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED(Immediate MDT only, Immediate MDT and Trace, …) |  |
| Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [35]. Second Bit = M2, Fifth Bit = M5, Seventh Bit = M6, Eighth Bit = M7. Value "1" indicates "activate" and value "0" indicates "do not activate". This version of the specification does not use bits 1, bit 3, bit 4 and bit 6. |
| M2 Configuration | C-ifM2 |  | ENUMERATED (true, …) |  |
| M5 Configuration | C-ifM5 |  | 9.3.1.152 |  |
| M6 Configuration | C-ifM6 |  | 9.3.1.153 |  |
| M7 Configuration | C-ifM7 |  | 9.3.1.154 |  |

| Condition | Explanation |
| --- | --- |
| ifM2 | This IE shall be present if the Measurements to Activate IE has the second bit set to "1". |
| ifM5 | This IE shall be present if the Measurements to Activate IE has the fifth bit set to "1". |
| ifM6 | This IE shall be present if the Measurements to Activate IE has the seventh bit set to "1". |
| ifM7 | This IE shall be present if the Measurements to Activate IE has the eighth bit set to "1". |

#### 9.3.1.151	MDT PLMN List

The purpose of the MDT PLMN List IE is to provide the list of PLMN allowed for MDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT PLMN List |  | 1..<maxnoofMDTPLMNs> |  |  |
| >PLMN Identity | M |  | PLMN ID 9.3.1.14 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | Maximum no. of PLMNs in the MDT PLMN list. Value is 16. |

#### 9.3.1.152	M5 Configuration

This IE defines the parameters for M5 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| M5 Collection Period | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, min1, …) |  | - |  |
| M5 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  | - |  |
| M5 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity…) | Number of reports. | YES | ignore |

#### 9.3.1.153	M6 Configuration

This IE defines the parameters for M6 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | ENUMERATED (ms120, ms240, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30,…, ms480) |  | - |  |
| M6 Links to log | M |  | ENUMERATED(uplink, downlink, both-uplink-and-downlink, …) |  | - |  |
| M6 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity…) | Number of reports. | YES | ignore |

#### 9.3.1.154	M7 Configuration

This IE defines the parameters for M7 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| M7 Collection Period | M |  | INTEGER (1..60, …) | Unit: minutes | - |  |
| M7 Links to log | M |  | ENUMERATED(downlink, …) |  | - |  |
| M7 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity…) | Number of reports. | YES | ignore |

#### 9.3.1.155	NID

This IE is used to identify (together with a PLMN identifier) a Stand-alone Non-Public Network. The NID is specified in TS 23.003 [23].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NID | M |  | BIT STRING (SIZE(44)) |  |

#### 9.3.1.156	NPN Support Information

This IE contains NPN related information associated with Network Slicing information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NPN Support Information | M |  |  |  |
| >SNPN Information |  |  |  |  |
| >>NID | M |  | 9.3.1.155 |  |

#### 9.3.1.157	NPN Broadcast Information

This IE contains NPN related broadcast information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NPN Broadcast Information per PLMN | M |  |  |  |
| >SNPN Information |  |  |  |  |
| >>Broadcast SNPN ID List | M |  | 9.3.1.158 |  |
| >PNI-NPN Information |  |  |  |  |
| >>Broadcast PNI-NPN ID List | M |  | 9.3.1.162 |  |

#### 9.3.1.158	Broadcast SNPN ID List

This IE contains SNPN related broadcast information associated with a set of PLMNs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Broadcast SNPN ID List |  | 1..<maxnoofNIDs> |  |  |
| >PLMN Identity | M |  | 9.3.1.14 |  |
| >Broadcast NID List | M |  | 9.3.1.159 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNIDs | Maximum no. of NIDs broadcast in a cell. Value is 12. |

#### 9.3.1.159	Broadcast NID List

This IE contains a list of NIDs.

| IE/Group Name | Presence | RangeNIDsupported | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Broadcast NID |  | 1..<maxnoofNIDsupported |  |  |
| >NID | M |  | 9.3.1.155 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNIDsupported | Maximum no. of NIDs broadcast in a cell. Value is 12. |

#### 9.3.1.160	Broadcast CAG-Identifier List

This IE contains a list of CAG-Identifiers.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Broadcast CAG-Identifier List |  | 1..<maxnoofCAGsupported> |  |  |
| >CAG ID | M |  | 9.3.1.161 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCAGsupported | Maximum no. of CAG-Identifiers broadcast in a cell. Value is 12. |

#### 9.3.1.161	CAG ID

This IE is used to identify (together with a PLMN identifier) a Public Network Integrated NPN, as defined in TS 23.003 [23].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CAG ID | M |  | BIT STRING (SIZE (32)) | Closed Access Group ID used in NR. |

#### 9.3.1.162	Broadcast PNI-NPN ID Information

This IE contains a list of PNI-NPN IDs.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Broadcast PNI-NPN ID Information |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs |
| >PLMN Identity | M |  | 9.3.1.14 |  |
| >Broadcast CAG-Identifier List | M |  | 9.3.1.160 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |

#### 9.3.1.163	Available SNPN ID List

This IE indicates the list of available SNPN ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Available SNPN ID List |  | 1..<maxnoofNIDs> |  |  |
| >PLMN Identity | M |  | 9.3.1.14 |  |
| >Available NID List | M |  | Broadcast NID List 9.3.1.159 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNIDs | Maximum no. of NIDs broadcast in a cell. Value is 12. |

#### 9.3.1.164	Void

#### 9.3.1.165	Extended Slice Support List

This IE indicates a list of supported slices.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Support Item IEs |  | 1..<maxnoofExtSliceItems> |  |  |
| >S-NSSAI | M |  | 9.3.1.38 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofExtSliceItems | Maximum no. of signalled slice support items. Value is 65535. |

#### 9.3.1.166	Positioning Measurement Result

The purpose of this information element is to provide the measurement result(s).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Positioning Measured Result Item |  | 1 .. <maxnoofPosMeas> |  |  | - |  |
| >CHOICE Measured Results Value | M |  |  |  | - |  |
| >>UL Angle of Arrival |  |  |  |  |  |  |
| >>>UL Angle of Arrival | M |  | 9.3.1.167 |  | - |  |
| >>UL SRS-RSRP |  |  |  |  |  |  |
| >>>UL SRS-RSRP | M |  | INTEGER (0..126) |  | - |  |
| >>UL RTOA |  |  |  |  |  |  |
| >>>UL RTOA | M |  | UL RTOA Measurement 9.3.1.168 |  | - |  |
| >>gNB Rx-Tx Time Difference |  |  |  |  |  |  |
| >>>gNB Rx-Tx Time Difference | M |  | 9.3.1.170 |  | - |  |
| >>Zenith Angle of Arrival Information |  |  |  |  | YES | reject |
| >>>Zenith Angle of Arrival Information | M |  | 9.3.1.239 |  | - |  |
| >>Multiple UL AoA |  |  |  |  | YES | reject |
| >>>Multiple UL AoA | M |  | 9.3.1.245 |  | - |  |
| >>UL SRS-RSRPP |  |  |  |  | YES | reject |
| >>>UL SRS-RSRPP | M |  | 9.3.1.246 |  | - |  |
| >Time Stamp | M |  | 9.3.1.171 |  | - |  |
| >Measurement Quality | O |  | TRP Measurement Quality 9.3.1.172 |  | - |  |
| >Measurement Beam Information | O |  | 9.3.1.173 |  | - |  |
| >ARP ID | O |  | 9.3.1.244 |  | YES | ignore |
| >SRS Resource type | O |  | 9.3.1.247 |  | YES | ignore |
| >LoS/NLoS Information | O |  | 9.3.1.249 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPosMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 16384. |

#### 9.3.1.167	UL Angle of Arrival

This information element contains the uplink Angle of Arrival measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Azimuth Angle of Arrival | M |  | INTEGER(0..3599) | TS 38.133 [38] |
| Zenith Angle of Arrival | O |  | INTEGER(0..1799) | TS 38.133 [38] |
| LCS to GCS Translation AoA | O |  | 9.3.1.241 | If absent, the azimuth and zenith are provided in GCS. |

#### 9.3.1.168	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE UL RTOA Measurement | M |  |  |  | - |  |
| >k0 |  |  |  |  |  |  |
| >>k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [38] | - |  |
| >k1 |  |  |  |  |  |  |
| >>k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [38] | - |  |
| >k2 |  |  |  |  |  |  |
| >>k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [38] | - |  |
| >k3 |  |  |  |  |  |  |
| >>k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [38] | - |  |
| >k4 |  |  |  |  |  |  |
| >>k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [38] | - |  |
| >k5 |  |  |  |  |  |  |
| >>k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [38] | - |  |
| Additional Path List | O |  | 9.3.1.169 | This IE is ignored if the Extended Additional Path List IE is included | - |  |
| Extended Additional Path List | O |  | 9.3.1.248 |  | YES | ignore |
| TRP Rx TEG Information | O |  | 9.3.1.280 |  | YES | ignore |

#### 9.3.1.169	Additional Path List

This information element contains the additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxnoofPath> |  |  | - |  |
| >CHOICE Relative Path Delay | M |  |  |  | - |  |
| >>k0 |  |  |  |  |  |  |
| >>>k0 | M |  | INTEGER(0..16351) |  | - |  |
| >>k1 |  |  |  |  |  |  |
| >>>k1 | M |  | INTEGER(0..8176) |  | - |  |
| >>k2 |  |  |  |  |  |  |
| >>>k2 | M |  | INTEGER(0..4088) |  | - |  |
| >>k3 |  |  |  |  |  |  |
| >>>k3 | M |  | INTEGER(0..2044) |  | - |  |
| >>k4 |  |  |  |  |  |  |
| >>>k4 | M |  | INTEGER(0..1022) |  | - |  |
| >>k5 |  |  |  |  |  |  |
| >>>k5 | M |  | INTEGER(0..511) |  | - |  |
| >Path Quality | O |  | TRP Measurement Quality 9.3.1.172 |  | - |  |
| >Multiple UL AoA | O |  | 9.3.1.245 |  | YES | ignore |
| >Path Power | O |  | UL SRS-RSRPP 9.3.1.246 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPath | Maximum no. of additional path measurements. Value is 2. |

#### 9.3.1.170	gNB Rx-Tx Time Difference

This information element contains the gNB Rx-Tx Time Difference measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE gNB Rx-Tx Time Difference Measurement | M |  |  |  | - |  |
| >k0 |  |  |  |  |  |  |
| >>k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [38] | - |  |
| >k1 |  |  |  |  |  |  |
| >>k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [38] | - |  |
| >k2 |  |  |  |  |  |  |
| >>k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [38] | - |  |
| >k3 |  |  |  |  |  |  |
| >>k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [38] | - |  |
| >k4 |  |  |  |  |  |  |
| >>k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [38] | - |  |
| >k5 |  |  |  |  |  |  |
| >>k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [38] | - |  |
| Additional Path List | O |  | 9.3.1.169 |  | - |  |
| Extended Additional Path List | O |  | 9.3.1.248 |  | YES | ignore |
| TRP TEG Information | O |  | 9.3.1.253 |  | YES | ignore |

#### 9.3.1.171	Time Stamp

This information element contains the time stamp associated with the measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| System Frame Number | M |  | INTEGER(0..1023) |  | - |  |
| CHOICE Slot Index | M |  |  |  | - |  |
| >SCS-15 |  |  |  |  |  |  |
| >>SCS-15 | M |  | INTEGER(0..9) |  | - |  |
| >SCS-30 |  |  |  |  |  |  |
| >>SCS-30 | M |  | INTEGER(0..19) |  | - |  |
| >SCS-60 |  |  |  |  |  |  |
| >>SCS-60 | M |  | INTEGER(0..39) |  | - |  |
| >SCS-120 |  |  |  |  |  |  |
| >>SCS-120 | M |  | INTEGER(0..79) |  | - |  |
| >SCS-480 |  |  |  |  |  |  |
| >>SCS-480 | M |  | INTEGER(0..319) |  | YES | reject |
| >SCS-960 |  |  |  |  | - |  |
| >>SCS-960 | M |  | INTEGER(0..639) |  | YES | reject |
| Measurement Time | O |  | Relative Time 1900 9.3.1.183 |  | - |  |

#### 9.3.1.172	TRP Measurement Quality

This information element contains the TRP’s best estimate of the quality of the measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE TRP Measurement Quality | M |  |  |  |
| >Timing Measurement Quality |  |  |  |  |
| >>Measurement Quality | M |  | INTEGER(0..31) | TS 37.355 [39] |
| >>Resolution | M |  | ENUMERATED(0.1m, 1m, 10m, 30m, …) | TS 37.355 [39] |
| >Angle Measurement Quality |  |  |  |  |
| >>Azimuth Quality | M |  | INTEGER(0..255) |  |
| >>Zenith Quality | O |  | INTEGER(0..255) |  |
| >>Resolution | M |  | ENUMERATED (0.1deg, …) |  |

#### 9.3.1.173	Measurement Beam Information

This information element contains the receiving beam information when measuring UL signals.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PRS Resource ID | O |  | INTEGER(0..63) |  |
| PRS Resource Set ID | O |  | INTEGER(0..7) |  |
| SSB Index | O |  | INTEGER(0..63) |  |

#### 9.3.1.174	NG-RAN Access Point Position

This IE is used to identify the geographical position of an NG-RAN Access Point / TRP / TRP Antenna Reference Points. It is expressed as ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032 [36].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Latitude Sign | M |  | ENUMERATED (North, South) |  |
| Degrees Of Latitude | M |  | INTEGER (0..223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degrees (0°.. 90°). |
| Degrees Of Longitude | M |  | INTEGER (-223..223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degrees (-180°..+180°). |
| Direction of Altitude | M |  | ENUMERATED (Height, Depth) |  |
| Altitude | M |  | INTEGER (0..215-1) | The relation between the value (N) and the altitude (a) in meters it describes is N a < N+1, except for N=215-1 for which the range is extended to include all greater values of (a). |
| Uncertainty semi-major | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Uncertainty semi-minor | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Orientation of major axis | M |  | INTEGER (0..179) |  |
| Uncertainty Altitude | M |  | INTEGER (0..127) | The uncertainty altitude "h" expressed in metres is derived from the "uncertainty code" k, by: h=45x(1.025k-1). |
| Confidence | M |  | INTEGER (0..100) | In percentage |

#### 9.3.1.175	Requested SRS Transmission Characteristics

This IE contains the requested SRS configuration for the UE for positioning purposes.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Number Of Periodic Transmissions | C-ifResourceTypePeriodic |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of SRS transmissions. | - |  |
| Resource Type | M |  | ENUMERATED (periodic, semi-persistent, aperiodic, …) |  | - |  |
| CHOICE Bandwidth SRS | M |  |  |  | - |  |
| >FR1 |  |  |  |  |  |  |
| >>FR1 Bandwidth | M |  | ENUMERATED (5, 10, 20, 40, 50, 80, 100, ... , 15, 25, 30, 60, 35, 45, 70, 90) |  | - |  |
| >FR2 |  |  |  |  |  |  |
| >>FR2 Bandwidth | M |  | ENUMERATED (50, 100, 200, 400,…,800,1600, 2000) |  | - |  |
| SRS Resource Set List |  | 0.. 1 |  |  | - |  |
| >SRS Resource Set Item |  | 1..< maxnoSRS-ResourceSets> |  |  | - |  |
| >>Number of SRS Resources Per Set | O |  | INTEGER (1..16,...) | The number of SRS Resources per resource set for SRS transmission. | - |  |
| >>Periodicity List |  | 0.. 1 |  |  | - |  |
| >>>Periodicity List Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  | - |  |
| >>>>PeriodicitySRS | M |  | ENUMERATED (0.125, 0.25, 0.5, 0.625, 1, 1.25, 2, 2.5, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, …) | Milli-seconds | - |  |
| >>Spatial Relation Information | O |  | 9.3.1.181 | This IE is ignored if the Spatial Relation Information per SRS Resource IE is present. | - |  |
| >>Pathloss Reference Information | O |  | 9.3.1.201 |  | - |  |
| >>Spatial Relation Information per SRS Resource | O |  | 9.3.1.210 |  | YES | ignore |
| SSB Information | O |  | 9.3.1.202 |  | - |  |
| SRS Frequency | O |  | INTEGER(0..3279165) | NR ARFCN The carrier frequency of SRS transmission bandwidth. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifResourceTypePeriodic | This IE shall be present if the Resource Type IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourceSets | Maximum no of requested SRS Resource Sets for SRS transmission. Value is 16. |
| maxnoSRS-ResourcePerSet | Maximum no of SRS Resources per set. Value is 16. |

#### 9.3.1.176	TRP Information

The TRP Information IE contains information for one TRP within a gNB-DU.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TRP ID | M |  | 9.3.1.197 |  | - |  |
| TRP Information Type Response List |  | 1 |  |  | - |  |
| >TRP Information Type Response Item |  | 1 .. <maxnoofTRPInfoTypes> |  |  | - |  |
| >>CHOICE TRP Information Type Response Item | M |  |  |  | - |  |
| >>>NR PCI |  |  |  |  |  |  |
| >>>>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID | - |  |
| >>>NR CGI |  |  |  |  |  |  |
| >>>>NR CGI |  |  | 9.3.1.12 |  | - |  |
| >>>NR ARFCN |  |  |  |  |  |  |
| >>>>NR ARFCN | M |  | INTEGER (0..3279165) |  | - |  |
| >>>PRS Configuration |  |  |  |  |  |  |
| >>>>PRS Configuration | M |  | 9.3.1.177 |  | - |  |
| >>>SSB Information |  |  |  |  |  |  |
| >>>>SSB Information | M |  | 9.3.1.202 |  | - |  |
| >>>SFN Initialisation Time |  |  |  |  |  |  |
| >>>>SFN Initialisation Time | M |  | Relative Time 1900 9.3.1.183 |  | - |  |
| >>>Spatial Direction Information |  |  |  |  |  |  |
| >>>>Spatial Direction Information | M |  | 9.3.1.179 |  | - |  |
| >>>Geographical Coordinates |  |  |  |  |  |  |
| >>>>Geographical Coordinates | M |  | 9.3.1.184 |  | - |  |
| >>>TRP Type |  |  |  |  | YES | reject |
| >>>>TRP Type | M |  | ENUMERATED (prs-only-tp, srs-only-rp, tp, rp, trp…) | TS 38.305 [42] | - |  |
| >>>On-demand PRS TRP Information |  |  |  |  | YES | reject |
| >>>>On-demand PRS TRP Information | M |  | 9.3.1.240 |  | - |  |
| >>>TRP Tx TEG Association |  |  |  |  | YES | reject |
| >>>>TRP Tx TEG Association | M |  | 9.3.1.252 |  | - |  |
| >>>TRP Beam Antenna |  |  |  |  | YES | reject |
| >>>>TRP Beam Antenna Information | M |  | 9.3.1.256 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTRPInfoTypes | Maximum no of TRP information types that can be requested and reported with one message. Value is 64. |

#### 9.3.1.177	PRS Configuration

This information element contains the DL PRS configuration for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PRS Resource Set List | M | 1..<maxnoofPRSresourceSets> |  |  |
| >PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …) |  |
| >PRS bandwidth | M |  | INTEGER(1..63) | 24,28,…,272 PRBs |
| >Start PRB | M |  | INTEGER(0..2176) | Starting PRB to Point A |
| >Point A | M |  | INTEGER (0..3279165) | NR ARFCN |
| >Comb Size | M |  | ENUMERATED(2, 4, 6, 12, …) |  |
| >CP Type | M |  | ENUMERATED(normal, extended, …) |  |
| >Resource Set Periodicity | M |  | ENUMERATED(4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120,10240,20480,40960,81920, …, 128, 256, 512) | Slots |
| >Resource Set Slot Offset | M |  | INTEGER(0..81919,…) |  |
| >Resource Repetition Factor | M |  | ENUMERATED(rf1,rf2,rf4,rf6,rf8,rf16,rf32,…) |  |
| >Resource Time Gap | M |  | ENUMERATED(tg1,tg2,tg4,tg8,tg16,tg32,…) |  |
| >Resource Number of Symbols | M |  | ENUMERATED(n2,n4,n6,n12,…) |  |
| >PRS Muting | O |  |  |  |
| >>Option1 | O |  |  |  |
| >>>Muting Pattern | M |  | DL-PRS Muting Pattern 9.3.1.178 | Muting pattern option 1 is used to mute the whole PRS resource set (within a period) |
| >>>Muting Bit Repetition Factor | M |  | ENUMERATED(rf1,rf2,rf4,rf8,…) |  |
| >>Option2 | O |  |  |  |
| >>>Muting Pattern | M |  | DL-PRS Muting Pattern 9.3.1.178 | Muting pattern option 2 is used to mute the selected repetition of the resource set (within the period) |
| >PRS Resource Transmit Power | M |  | INTEGER(-60..50) |  |
| >PRS Resource List | M | 1..<maxnoofPRSresources> |  | NR-DL-PRS-Resource-r16 as defined in TS 37.355 [39] |
| >>PRS Resource ID | M |  | INTEGER(0..63) |  |
| >>Sequence ID | M |  | INTEGER(0..4095) |  |
| >>RE Offset | M |  | INTEGER(0..11, …) |  |
| >>Resource Slot Offset | M |  | INTEGER(0..511) |  |
| >>Resource Symbol Offset | M |  | INTEGER(0..12) |  |
| >>CHOICE QCL Info | O |  |  |  |
| >>>SSB |  |  |  |  |
| >>>>PCI | M |  | INTEGER (0..1007) |  |
| >>>>SSB Index | O |  | INTEGER(0..63) |  |
| >>>DL-PRS | O |  |  |  |
| >>>>QCL Source PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>>>QCL Source PRS Resource ID | O |  | INTEGER(0..63) | If absent, the QCL source PRS resource ID is the same as the PRS resource ID |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSets | Maximum no of PRS resource sets. Value is 8. |
| maxnoofPRSresources | Maximum no of PRS resources per PRS resource set. Value is 64. |

#### 9.3.1.178	DL-PRS Muting Pattern

This information element contains the DL-PRS muting pattern.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE DL-PRS Muting Pattern | M |  |  |  |
| >Two |  |  |  |  |
| >>Two | M |  | BIT STRING (SIZE(2)) |  |
| >Four |  |  |  |  |
| >>Four | M |  | BIT STRING (SIZE(4)) |  |
| >Six |  |  |  |  |
| >>Six | M |  | BIT STRING (SIZE(6)) |  |
| >Eight |  |  |  |  |
| >>Eight | M |  | BIT STRING (SIZE(8)) |  |
| >Sixteen |  |  |  |  |
| >>Sixteen | M |  | BIT STRING (SIZE(16)) |  |
| >Thirty-two |  |  |  |  |
| >>Thirty-two | M |  | BIT STRING (SIZE(32)) |  |

#### 9.3.1.179	Spatial Direction Information

This information element contains the spatial direction information of the DL PRS resources for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| NR-PRS Beam Information | M |  | 9.3.1.198 | The spatial directions of DL-PRS Resources for TRP |

#### 9.3.1.180	SRS Resource Set ID

This information element indicates a resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Resource Set ID | M |  | INTEGER (0..15) | According to TS 38.331 [8] |

#### 9.3.1.181	Spatial Relation Information

This information element indicates a spatial relation for transmission of UL SRS by a UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Spatial Relation for Resource ID |  | 1 |  | According to TS 38.321 [16] and and TS 38.331 [8] |
| >Spatial Relation for Resource ID Item |  | 1..<maxnoSpatialRelations> |  |  |
| >>CHOICE Reference Signal | M |  |  |  |
| >>>NZP CSI-RS |  |  |  |  |
| >>>>NZP CSI-RS Resource ID | M |  | INTEGER (0..191) |  |
| >>>SSB |  |  |  |  |
| >>>>PCI | M |  | INTEGER (0..1007) |  |
| >>>>SSB Index | O |  | INTEGER (0..63) |  |
| >>>SRS |  |  |  |  |
| >>>>SRS Resource ID | M |  | INTEGER (0..63) |  |
| >>>Positioning SRS |  |  |  |  |
| >>>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |
| >>>DL-PRS |  |  |  |  |
| >>>>DL-PRS ID | M |  | INTEGER (0..255) |  |
| >>>>DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >>>>DL PRS Resource ID | O |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSpatialRelations | Maximum no. of Spatial Relations that can be configured. Value is 64. |

#### 9.3.1.182	SRS Resource Trigger

This information element indicates a DCI code point according to a SRS resource set configuration.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Aperiodic SRS Resource Trigger List |  | 1..<maxnoSRS-TriggerStates> |  | According to TS 38.331 [8] |
| >Aperiodic SRS Resource Trigger |  |  | INTEGER (1..3) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-TriggerStates | Maximum no. of SRS trigger states. Value is 3. |

#### 9.3.1.183	Relative Time 1900

This information element indicates the initialisation time (e.g. SFN Initalisation Time for a cell, requested time for an action, etc).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Relative Time 1900 | M |  | BIT STRING (SIZE(64)) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second |

#### 9.3.1.184	Geographical Coordinates

This information element contains the geographical coordinates for the TRP and any associated ARP(s).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE TRP Position Definition Type | M |  |  |  | - |  |
| >Direct |  |  |  |  |  |  |
| >>CHOICE Accuracy | M |  |  |  | - |  |
| >>>normal accuracy |  |  |  |  |  |  |
| >>>>TRP Position | M |  | NG-RAN Access Point Position 9.3.1.174 | The configured estimated geographical position of the antenna of the cell/TRP. | - |  |
| >>>high accuracy |  |  |  |  |  |  |
| >>>>TRP High Accuracy Access Position | M |  | NG-RAN High Accuracy Access Point Position 9.3.1.190 | The configured estimated geographical high accuracy position of the antenna of the cell/TRP. | - |  |
| >Referenced |  |  |  |  |  |  |
| >>Reference Point | M |  | 9.3.1.188 | The reference point is used to derive the TRP position | - |  |
| >>CHOICE Type | M |  |  |  | - |  |
| >>>Geodetic |  |  |  |  |  |  |
| >>>>TRP Position Relative Geodetic | M |  | Relative Geodetic Location 9.3.1.186 | The configured estimated relative geodetic coordinate of the antenna of the cell/TRP | - |  |
| >>>Cartesian |  |  |  |  |  |  |
| >>>>TRP Position Relative Cartesian | M |  | Relative Cartesian Location 9.3.1.187 | The configured estimated relative Cartesian coordinate of the antenna of the cell/TRP | - |  |
| DL-PRS Resource Coordinates | O |  | 9.3.1.185 | DL-PRS Resource Coordinates relative to the TRP coordinate | - |  |
| ARP Location Information | O |  | 9.3.1.243 |  | YES | ignore |

#### 9.3.1.185	DL-PRS Resource Coordinates

This information element contains the geographical coordinates of the antenna reference points (ARP) for the DL-PRS Resources of a TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| DL-PRS Resource Set ARP List | M | 1..<maxnoofPRS-ResourceSets> |  |  |
| >DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >CHOICE DL-PRS Resource Set ARP Location | M |  |  | Relative to the geographical coordinates for the TRP. If this IE is absent, the Relative Location is zero for the indicated DL-PRS Resource Set ID. |
| >>Geodetic |  |  |  |  |
| >>>Relative Geodetic Location | M |  | Relative Geodetic Location 9.3.1.186 |  |
| >>Cartesian |  |  |  |  |
| >>>Relative Cartesian Location | M |  | Relative Cartesian Location 9.3.1.187 |  |
| >DL-PRS Resource ARP List | M | 1..<maxnoofPRS-ResourcesPerSet> |  |  |
| >>DL-PRS Resource ID | M |  | INTEGER (0..63) |  |
| >>CHOICE DL-PRS Resource ARP Location | M |  |  | Relative to the DL-PRS Resource Set ARP Location. If this IE is absent, the Relative Location is zero for the indicated DL-PRS Resource ID. |
| >>Geodetic |  |  |  |  |
| >>>Relative Geodetic Location | O |  | Relative Geodetic Location 9.3.1.186 |  |
| >>Cartesian |  |  |  |  |
| >>>Relative Cartesian Location | O |  | Relative Cartesian Location 9.3.1.187 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRS-ResourceSets | Maximum no of DL-PRS resource sets per TRP. Value is 2. |
| maxnoofPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |

#### 9.3.1.186	Relative Geodetic Location

This information element provides a location relative to some known reference location in a relative geodetic coordinate system.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Milli-Arc-Second Units | M |  | ENUMERATED (0.03, 0.3, 3, ...) | Units and scale factor for the delta-latitude and delta-longitude fields, TS 37.355 [39]. |
| Height Units | M |  | ENUMERATED (mm, cm, m, ...) | Units and scale factor for the delta-height field, TS 37.355 [39]. |
| Delta Latitude | M |  | INTEGER (-1024..1023) | Delta value in latitude in the unit provided in Milli-Arc-Second Units, TS 37.355 [39]. |
| Delta Longitude | M |  | INTEGER (-1024..1023) | Delta value in longitude in the unit provided in Milli-Arc-Second Units, TS 37.355 [39]. |
| Delta Height | M |  | INTEGER (-1024..1023) | Delta value in ellipsoidal height in the unit provided in Height Units, TS 37.355 [39]. |
| Location uncertainty | M |  | 9.3.1.189 |  |

#### 9.3.1.187	Relative Cartesian Location

This information element provides a location relative to some known reference location in a relative Cartesian coordinate.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| XYZ unit | M |  | ENUMERATED (mm, cm, dm,..) |  |
| X value | M |  | INTEGER (-216.. 216-1) | Positive value represents easting from reference point, in units of XYZ Unit IE. |
| Y value | M |  | INTEGER (-216.. 216-1) | Positive value represents northing from reference point in units of XYZ Unit IE. |
| Z value | M |  | INTEGER (-215.. 215-1) | Height with respect to reference point in units of XYZ Unit IE, where the XY-plane is horizontal and the Z-axis points up. |
| Location uncertainty | M |  | 9.3.1.189 |  |

#### 9.3.1.188	Reference Point

This information element provides a reference point location information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE ReferencePoint | M |  |  | Reference point to which relative location information is related to |
| >Coordinate ID |  |  |  |  |
| >>Coordinate ID | M |  | INTEGER(0.. 29-1,..) | Referential ID mapped via OAM |
| >Reference Point Coordinates |  |  |  |  |
| >>Reference Point Position | M |  | NG-RAN Access Point Position 9.3.1.174 |  |
| >Reference Point Coordinates High Accuracy |  |  |  |  |
| >>Reference Point High Accuracy Access Position | M |  | NG-RAN High Accuracy Access Point Position 9.3.1.190 |  |

#### 9.3.1.189	Location Uncertainty

This information element provides the location uncertainty information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Horizontal Uncertainty | M |  | INTEGER (0..255) | Horizontal uncertainty of the ARP latitude/longitude. Corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [36] |
| Horizontal Confidence | M |  | INTEGER (0..100) | Corresponds to confidence as defined in TS 23.032 [36]. |
| Vertical Uncertainty | M |  | INTEGER (0..255) | Vertical uncertainty of the ARP altitude. Corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [36] |
| Vertical Confidence | M |  | INTEGER (0..100) | Corresponds to confidence as defined in TS 23.032 [36]. |

#### 9.3.1.190	NG-RAN High Accuracy Access Point Position

The NG-RAN High Accuracy Access Point Position IE is used to identify the geographical position of an NG-RAN Access Point. It is expressed as High Accuracy Ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032 [36].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Degrees of Latitude | M |  | INTEGER(-2147483648..2147483647) |  |
| Degrees of Longitude | M |  | INTEGER(-2147483648..2147483647) |  |
| Altitude | M |  | INTEGER(-64000..1280000) |  |
| Uncertainty Semi Major | M |  | INTEGER (0..255) |  |
| Uncertainty Semi Minor | M |  | INTEGER (0..255) |  |
| Orientation Major Axis | M |  | INTEGER (0..179) |  |
| Horizontal Confidence | M |  | INTEGER (0..100) |  |
| Uncertainty Altitude | M |  | INTEGER (0..255) |  |
| Vertical Confidence | M |  | INTEGER (0..100) |  |

#### 9.3.1.191	Positioning Broadcast Cells

This IE is used to indicate the cells that are requested to broadcast, or failed to broadcast, the associated posSIB(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Positioning Broadcast Cells |  | 1 .. <maxnoBcastCell> |  |  |
| >NR CGI | M |  | 9.3.1.12 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoBcastCells | Maximum no. of cells broadcasting a posSIB in a NB-DU. Value is 16384. |

#### 9.3.1.192		SRS Configuration

This information element contains the SRS configuration configured by the gNB-CU for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Carrier List |  | 1..<maxnoSRS-Carriers> |  |  |
| >Point A | M |  | INTEGER (0..3279165) | NR ARFCN |
| >Uplink Channel BW-PerSCS-List |  | 1..<maxnoSCSs> |  | SCS-SpecificCarrier TS 38.331 [8] |
| >>Offset To Carrier | M |  | INTEGER(0..2199,…) | First usable RB to Point A in the number of PRBs |
| >>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…, kHz480, kHz960) |  |
| >>Carrier Bandwidth | M |  | INTEGER(1..275,…) |  |
| >Active UL BWP | M |  |  | Only the configuration in the active UL BWP is needed. |
| >>Location And Bandwidth | M |  | INTEGER(0..37949,…) | BWP TS 38.331 [8] |
| >>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…, kHz480, kHz960) |  |
| >>Cyclic Prefix | M |  | ENUMERATED(Normal, Extended) |  |
| >>Tx Direct Current Location | M |  | INTEGER(0..3301,…) |  |
| >>Shift7dot5kHz | O |  | ENUMERATED(true,…) |  |
| >>SRS Config | M |  |  | SRS-Config as defined in TS 38.331 [8] |
| >>>SRS Resource List |  | 0..<maxnoSRS-Resources> |  |  |
| >>>>SRS Resource | M |  | 9.3.1.193 | SRS-Resource as defined in TS 38.331 [8] |
| >>>Positioning SRS Resource List |  | 0..<maxnoSRS-PosResources> |  |  |
| >>>>Positioning SRS Resource | M |  | 9.3.1.194 | SRS-PosResource-r16 as defined in TS 38.331 [8] |
| >>>SRS Resource Set List |  | 0..<maxnoSRS-ResourceSets> |  |  |
| >>>>SRS Resource Set | M |  | 9.3.1.195 | SRS-ResourceSet as defined in TS 38.331 [8] |
| >>>Positioning SRS Resource Set List |  | 0..<maxnoSRS-PosResourceSets> |  |  |
| >>>>Positioning SRS Resource Set | M |  | 9.3.1.196 | SRS-PosResourceSet-r16 as defined in TS 38.331 [8] |
| >PCI | O |  | INTEGER (0..1007) | Physical Cell ID of the cell that contains the SRS carrier |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-Carriers | Maximum no of carriers for SRS. Value is 32. |
| maxnoSCSs | Maximum no of SCS spacings for a carrier. Value is 5. |
| maxnoSRS-Resources | Maximum no of SRS resources per UL BWP. Value is 64. |
| maxnoSRS-PosResources | Maximum no of positioning SRS resources per UL BWP. Value is 64. |
| maxnoSRS-ResourceSets | Maximum no of SRS resource sets. Value is 16. |
| maxnoSRS-PosResourceSets | Maximum no of positioning SRS resource sets per UL BWP. Value is 16. |

#### 9.3.1.193	SRS Resource

This information element contains the SRS resource.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SRS Resource ID | M |  | INTEGER (0..63, ...) |  | - |  |
| Number of Ports | M |  | ENUMERATED(ports1, ports2, ports4) |  | - |  |
| CHOICE Transmission Comb | M |  |  |  | - |  |
| >Comb Two |  |  |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..1) |  | - |  |
| >>Cyclic Shift | M |  | INTEGER(0..7) |  | - |  |
| >Comb Four |  |  |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..3) |  | - |  |
| >>Cyclic Shift | M |  | INTEGER(0..11) |  | - |  |
| >Comb Eight | M |  |  |  | YES | reject |
| >>Comb Offset | M |  | INTEGER(0..7) |  | - | - |
| >>Cyclic Shift | M |  | INTEGER(0..5) |  | - | - |
| Start Position | M |  | INTEGER(0..13) |  | - |  |
| Number of Symbols | M |  | ENUMERATED(1,2,4) | This IE is ignored if the Number of Symbols Extended IE is included. | - |  |
| Repetition Factor | M |  | ENUMERATED(1,2,4) | This IE is ignored if the Repetition Factor Extended IE is included. | - |  |
| Frequency Domain Position | M |  | INTEGER(0..67) |  | - |  |
| Frequency Domain Shift | M |  | INTEGER(0..268) |  | - |  |
| C-SRS | M |  | INTEGER(0..63) |  | - |  |
| B-SRS | M |  | INTEGER(0..3) |  | - |  |
| B-Hop | M |  | INTEGER(0..3) |  | - |  |
| Group or Sequence Hopping | M |  | ENUMERATED(Neither, groupHopping, sequenceHopping) |  | - |  |
| CHOICE Resource Type | M |  |  |  | - |  |
| >Periodic |  |  |  |  |  |  |
| >>Periodicity | M |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, …) |  | - |  |
| >>Offset | M |  | INTEGER(0..2559, …) |  | - |  |
| >Semi-persistent |  |  |  |  |  |  |
| >>Periodicity | M |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, …) |  | - |  |
| >>Offset | M |  | INTEGER(0..2559, …) |  | - |  |
| >Aperiodic |  |  |  |  |  |  |
| >>Aperiodic Resource Type | M |  | ENUMERATED(true,…) |  | - |  |
| Sequence ID | M |  | INTEGER(0..1023) |  | - |  |
| Number of Symbols Extended | O |  | ENUMERATED(n8,n10,n12, n14, …) |  | YES | ignore |
| Repetition Factor Extended | O |  | ENUMERATED(r3, r5, r6, r7, r8, r10, r12, r14, …) |  | YES | ignore |
| Start RB Hopping | O |  | ENUMERATED(enable) |  | YES | ignore |
| CHOICE Start RB Index | O |  |  |  | YES | ignore |
| >FreqScalingFactor2 |  |  | INTEGER (0..1) |  | - | - |
| >FreqScalingFactor4 |  |  | INTEGER (0..3) |  | - | - |

#### 9.3.1.194	Positioning SRS Resource

This information element contains the SRS resource for positioning.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |
| CHOICE Transmission Comb Positioning | M |  |  |  |
| >Comb Two |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..1) |  |
| >>Cyclic Shift | M |  | INTEGER(0..7) |  |
| >Comb Four |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..3) |  |
| >>Cyclic Shift | M |  | INTEGER(0..11) |  |
| >Comb Eight |  |  |  |  |
| >>Comb Offset | M |  | INTEGER(0..7) |  |
| >>Cyclic Shift | M |  | INTEGER(0..5) |  |
| Start Position | M |  | INTEGER(0..13) |  |
| Number of Symbols | M |  | ENUMERATED(1,2,4,8,12) |  |
| Frequency Domain Shift | M |  | INTEGER(0..268) |  |
| C-SRS | M |  | INTEGER(0..63) |  |
| Group or Sequence Hopping | M |  | ENUMERATED(Neither, groupHopping, sequenceHopping) |  |
| CHOICE Resource Type Positioning | M |  |  |  |
| >Periodic |  |  |  |  |
| >>Periodicity | M |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot40960, slot81920,…, slot128, slot256, slot512, slot20480) |  |
| >>Offset | M |  | INTEGER(0..81919,…) |  |
| >Semi-persistent |  |  |  |  |
| >>Periodicity | M |  | ENUMERATED(slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot20480, slot40960, slot81920,…, slot128, slot256, slot512, slot20480) |  |
| >>Offset | M |  | INTEGER(0..81919,…) |  |
| >Aperiodic |  |  |  |  |
| >>Slot offset | M |  | INTEGER(0..32) |  |
| Sequence ID | M |  | INTEGER(0..65535) |  |
| CHOICE Spatial Relation Positioning | O |  |  |  |
| >SSB |  |  |  |  |
| >>PCI | M |  | INTEGER (0..1007) |  |
| >>SSB index | O |  | INTEGER(0..63) |  |
| >PRS |  |  |  |  |
| >>PRS ID | M |  | INTEGER(0..255) |  |
| >>PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>PRS Resource ID | O |  | INTEGER(0..63) |  |

#### 9.3.1.195	SRS Resource Set

This information element indicates a SRS resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Resource Set ID | M |  | INTEGER(0..15) |  |
| SRS Resource ID List |  | 1..<maxnoSRS-ResourcePerSet> |  |  |
| >SRS Resource ID | M |  | INTEGER (0..63, ...) |  |
| CHOICE Resource Set Type | M |  |  |  |
| >Periodic |  |  |  |  |
| >>PeriodicSet | M |  | ENUMERATED(true,…) |  |
| >Semi-persistent |  |  |  |  |
| >>Semi-persistentSet | M |  | ENUMERATED(true,…) |  |
| >Aperiodic |  |  |  |  |
| >>SRS Resource Trigger List | M |  | INTEGER(1..3) |  |
| >>Slot offset | M |  | INTEGER(0..32) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourcePerSet | Maximum no of SRS resources per SRS resource set. Value is 16. |

#### 9.3.1.196	Positioning SRS Resource Set

This information element indicates a positioning SRS resource set in the UE for UL SRS transmission.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Positioning SRS Resource Set ID | M |  | INTEGER(0..15) |  |
| Positioning SRS Resource ID List |  | 1..<maxnoSRS-PosResourcePerSet> |  |  |
| >Positioning SRS Resource ID | M |  | INTEGER (0..63, ...) |  |
| CHOICE Resource Type | M |  |  |  |
| >Periodic |  |  |  |  |
| >>PosperiodicSet | M |  | ENUMERATED(true,…) |  |
| >Semi-persistent |  |  |  |  |
| >>Possemi-persistentSet | M |  | ENUMERATED(true,…) |  |
| >Aperiodic |  |  |  |  |
| >>SRS Resource Trigger List | M |  | INTEGER(1..3) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-PosResourcePerSet | Maximum no of positioning SRS resources per positioning SRS resource set. Value is 16. |

#### 9.3.1.197	TRP ID

The TRP ID IE is used to identify a TRP uniquely within a gNB-CU.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TRP Identifier | M |  | INTEGER (1..65535,…) | Identifies a TRP within an gNB-CU |

#### 9.3.1.198	NR-PRS Beam Information

This IE contains spatial direction information of the DL-PRS Resources.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PRS Beam Information List |  | 1 |  |  | - |  |
| >NR-PRS Beam Information Item |  | 1 .. < maxnoofPRS-ResourceSets > |  |  | - |  |
| >>PRS Resource Set ID | M |  | INTEGER (0..7) | The resource set in which the resources are associated with the angle. | - |  |
| >>PRS Angle List |  | 1 |  |  | - |  |
| >>>PRS Angle Item |  | 1..< maxnoofPRS-ResourcesPerSet > |  |  | - |  |
| >>>>NR PRS Azimuth | M |  | INTEGER (0..359) |  | - |  |
| >>>>NR PRS Azimuth fine | O |  | INTEGER (0..9) | Fine angles | - |  |
| >>>>NR PRS Elevation | O |  | INTEGER (0..180) |  | - |  |
| >>>>NR PRS Elevation fine | O |  | INTEGER (0..9) | Fine angles | - |  |
| >>>>PRS Resource ID | O |  | INTEGER(0..63) |  | YES | ignore |
| LCS to GCS Translation List |  | 0..1 |  | If absent, the azimuth and elevation are provided in GCS. | - |  |
| >LCS to GCS Translation |  | 1 .. <maxnooflcs-gcs-translation> |  |  | - |  |
| >>Alpha | M |  | INTEGER (0..359) |  | - |  |
| >>Alpha-fine | O |  | INTEGER (0..9) | Fine angles | - |  |
| >>Beta | M |  | INTEGER (0..359) |  | - |  |
| >>Beta-fine | O |  | INTEGER (0..9) | Fine angles | - |  |
| >>Gamma | M |  | INTEGER (0..359) |  | - |  |
| >>Gamma-fine | O |  | INTEGER (0..9) | Fine angles | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRS-ResourceSets | Maximum no of DL-PRS resource sets per TRP. Value is 2. |
| maxnoofPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |
| maxnooflcs-gcs-translation | Maximum no. of LCS-GS-Translation-Parameters that can reported with one message. Value is 3. The current version of the specification supports 1. |

#### 9.3.1.199	E-CID Measurement Result

The purpose of this IE is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Geographical Coordinates | O |  | 9.3.1.184 | The configured estimated geographical position of the antenna of the cell. | - |  |
| Measured Results List |  | 0..1 |  |  |  |  |
| >E-CID Measured Results Item |  | 1 .. <maxnoMeasE-CID> |  |  |  |  |
| >>CHOICE Measured Results Value | M |  |  |  |  |  |
| >>>Value Angle of Arrival NR |  |  |  |  |  |  |
| >>>>Value Angle of Arrival NR | M |  | UL Angle of Arrival 9.3.1.167 |  | - |  |
| >>>Value Timing Advance NR |  |  |  |  |  |  |
| >>>>Value Timing Advance NR | M |  | INTEGER (0.. 7690) | As defined in TS 38.215 [43] | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoMeasE-CID | Maximum no. of measured quantities that can be configured and reported with one message. Value is 64. |

#### 9.3.1.200	Cell Portion ID

This IE gives the current Cell Portion associated with the target UE. The Cell Portion ID is the unique identifier for a cell portion within a cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Portion ID | M |  | INTEGER (0..4095, …) |  |

#### 9.3.1.201	Pathloss Reference Information

This information element indicates a pathloss reference for transmission of UL SRS by a UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Pathloss Reference Signal | M |  |  |  |
| >SSB |  |  |  |  |
| >>PCI | M |  | INTEGER (0..1007) |  |
| >>SSB Index | O |  | INTEGER (0..63) |  |
| >DL-PRS |  |  |  |  |
| >>DL-PRS ID | M |  | INTEGER (0..255) |  |
| >>DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >>DL PRS Resource ID | O |  | INTEGER (0..63) |  |

#### 9.3.1.202	SSB Information

This information element contains the SSB time/frequency information for the TRPs.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SSB Information List |  | 1 |  |  |
| >SSB Information Item |  | 1…<maxNoSSBs> |  |  |
| >>SSB Configuration | M |  | SSB Time/Frequency Configuration 9.3.1.203 |  |
| >>PCI | M |  | INTEGER (0..1007) |  |

| Range bound | Explanation |
| --- | --- |
| maxNoSSBs | Maximum no of SSBs for which the configuration can be provided. Value is 255. |

#### 9.3.1.203	SSB Time/Frequency Configuration

This information element contains the time and frequency configuration of an SSB.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SSB frequency | M |  | INTEGER (0..3279165) | ARFCN |
| SSB subcarrier spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, kHz240,... , kHz480, kHz960) | The value 60kHz is not supported in this version of the specification. |
| SSB Transmit power | M |  | INTEGER (-60..50) | EPRE of SSS |
| SSB periodicity | M |  | ENUMERATED(ms5, ms10, ms20, ms40, ms80, ms160, …) |  |
| SSB half frame index | M |  | INTEGER(0..1) |  |
| SSB SFN offset | M |  | INTEGER(0..15) |  |
| CHOICE SSB Position in Burst | O |  |  |  |
| >Short |  |  |  |  |
| >>Short Bitmap |  |  | BIT STRING (SIZE(4)) |  |
| >Medium |  |  |  |  |
| >>Medium Bitmap |  |  | BIT STRING (SIZE(8)) |  |
| >Long |  |  |  |  |
| >>Long Bitmap |  |  | BIT STRING (SIZE(64)) |  |
| SFN Initialisation Time | O |  | Relative Time 1900 9.3.1.183 |  |

#### 9.3.1.204	Search Window Information

This information element contains search window information for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Expected Propagation Delay | M |  | INTEGER (-3841..3841,…) | Indicates when the SRS is expected to arrive in time at the TRP relative to the UL RTOA Reference Time. The UL RTOA Reference Time for a target SRS is defined as T0+tSRS, where - T0 is the SFN Initialisation Time - tSRS=10nf+nsf×10-3, where nf and nsf are the system frame number and the subframe number of the SRS, respectively. Granularity 4Ts, where Ts=1/(15103 2048) seconds. Centre of the search window. |
| Delay Uncertainty | M |  | INTEGER (1..246,…) | Indicates the uncertainty of the expected SRS arrival time at the TRP Granularity 4Ts, where Ts=1/(151032048) seconds. Single-sided search window. |

#### 9.3.1.205	Extended gNB-DU Name

This IE provides extended human readable name of the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-DU Name Visible | O |  | VisibleString (SIZE(1..150, …)) |  |
| gNB-DU Name UTF8 | O |  | UTF8String (SIZE(1..150, …)) |  |

#### 9.3.1.206	Extended gNB-CU Name

This IE provides extended human readable name of the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-CU Name Visible | O |  | VisibleString (SIZE(1..150, …)) |  |
| gNB-CU Name UTF8 | O |  | UTF8String (SIZE(1..150, …)) |  |

#### 9.3.1.207	F1-C Transfer Path

This IE indicates the transmission path of the F1-C traffic.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| F1-C Path NSA | M |  | ENUMERATED (lte, nr, both) | This IE indicates the transmission path of the F1-C traffic in EN-DC. |

#### 9.3.1.208	SFN Offset

This IE contains the time offset between an absolute time reference and the SFN0 start. The IE is calculated assuming that the SFN transmission started at the absolute time reference. The absolute time reference chosen is the 1980-01-06 T00:00:19 International Atomic Time (TAI).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SFN Time Offset | M |  | BIT STRING (SIZE(24)) | Time offset in microseconds between the absolute time reference "1980-01-06 T00:00:19 International Atomic Time (TAI)” and the SFN0 start. The maximum usable value is (1024*10^4-1). Values higher than the maximum are discarded. |

#### 9.3.1.209	Transmission Stop Indicator

This IE indicates to stop the data transmission at gNB-DU side for an DRB not subject to DAPS Handover.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transmission Stop Indicator | M |  | ENUMERATED (true, …) |  |

#### 9.3.1.210	Spatial Relation Information per SRS Resource

This information element indicates a spatial relation for transmission of each UL SRS resource recommened by LMF.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Spatial Relation per SRS Resource List |  | 1 |  |  |
| >Spatial Relation per SRS Resource Item |  | 1..<maxnoSRS-ResourcePerSet> |  |  |
| >CHOICE Reference Signal | M |  |  |  |
| >>NZP CSI-RS |  |  |  |  |
| >>>NZP CSI-RS Resource ID | M |  | INTEGER (0..191) |  |
| >>SSB |  |  |  |  |
| >>>NR PCI | M |  | INTEGER (0..1007) |  |
| >>>SSB Index | O |  | INTEGER (0..63) |  |
| >>SRS |  |  |  |  |
| >>>SRS Resource ID | M |  | INTEGER (0..63) |  |
| >>Positioning SRS |  |  |  |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER (0..63) |  |
| >>DL-PRS |  |  |  |  |
| >>>DL-PRS ID | M |  | INTEGER (0..255) |  |
| >>>DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >>>DL-PRS Resource ID | O |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS-ResourcePerSet | Maximum no of SRS resources per SRS resource set. Value is 16. |

#### 9.3.1.211	CCO Assistance Information

This IE indicates the Capacity and Coverage (CCO) actions for specific CCO issues detected.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CCO issue detection | O |  | ENUMERATED (coverage, cell edge capacity ...) | Indicates the type of CCO issue detected |
| Affected Cells and Beams | O |  | 9.3.1.212 |  |

#### 9.3.1.212	Affected Cells and Beams

This IE includes a list of cells and/or SS/PBCH block indexes affected by the detected CCO issue.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Affected Cell List |  | 1 .. < maxAffectedCells> |  |  |
| >NR CGI | M |  | 9.3.1.12 |  |
| >Affected SSB List |  | 0..<maxnoofSSBAreas> |  |  |
| >>SSB Index | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxAffectedCells | Maximum numbers of cells affected by a CCO issue. Value is 32. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

#### 9.3.1.213	Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Coverage Modification List |  | 1 |  |  |
| >Coverage Modification Item |  | 1 .. <maxCellingNBDU> |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. |
| >>SSB Coverage Modification List |  | 0..1 |  |  |
| >>>SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block is inactive. Other values Indicates that the SS/PBCH block is active and also indicates the coverage configuration of the concerned SS/PBCH block. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

#### 9.3.1.214	Cells for SON List

This IE contains a list of served cells in potential PRACH conflict and it may contain neighbour cell PRACH configuration.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Cells for SON Item |  | 1 .. <maxServedCellforSON> |  |  |
| >NR CGI | M |  | 9.3.1.12 |  |
| >Neighbour NR Cells for SON List | O |  | 9.3.1.215 |  |

| Range bound | Explanation |
| --- | --- |
| maxServedCellforSON | Maximum numbers of served cells where PRACH conflict is possible. Value is 256. |

#### 9.3.1.215	Neighbour NR Cells for SON List

This IE contains the configuration of NR neighbour cells which the gNB-DU may take into consideration for SON purposes.

NOTE:	If multiple served cells share a common neighbour cell and thus multiple copies of Neighbour NR Cells for SON Item IE for the neighbour cell are needed to be contained in an F1AP message, IEs other than the NR CGI IE may be omitted in the copies other than the first one present in the message.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Neighbour NR Cells for SON Item |  | 1 .. <maxNeighbourCellforSON> |  |  |
| >NR CGI | M |  | 9.3.1.12 |  |
| >NR Mode Info Rel16 | O |  | 9.3.1.216 |  |
| >SSB Positions In Burst | O |  | 9.3.1.138 |  |
| >NR Cell PRACH Configuration | O |  | 9.3.1.139 |  |

| Range bound | Explanation |
| --- | --- |
| maxNeighbourCellforSON | Maximum numbers of neighbour cells which the gNB-DU may take into consideration for SON purposes on a given served cell. Value is 32. |

#### 9.3.1.216	NR Mode Info Rel16

This IE contains the information of a NR cell which needs to be encoded differently for FDD and TDD.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NR-Mode-Info-Rel16 | M |  |  |  |
| >FDD |  |  |  |  |
| >>FDD Info Rel16 |  | 1 |  |  |
| >>>UL Frequency Info | O |  | Frequency Info Rel16 9.3.1.217 |  |
| >>>SUL Frequency Info | O |  | Frequency Info Rel16 9.3.1.217 |  |
| >TDD |  |  |  |  |
| >>TDD Info Rel16 |  | 1 |  |  |
| >>>TDD Frequency Info | O |  | Frequency Info Rel16 9.3.1.217 |  |
| >>>SUL Frequency Info | O |  | Frequency Info Rel16 9.3.1.217 |  |
| >>>TDD DL-UL Configuration Common NR | O |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon contained in the ServingCellConfigCommon IE as defined in TS 38.331 [8] |

#### 9.3.1.217	Frequency Info Rel16

This IE contains the information of a NR cell which should be encoded separately among FDD NDL, FDD NUL, TDD NDL+NUL and SUL.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR ARFCN | O |  | INTEGER (0.. maxNRARFCN) | RF Reference Frequency as defined in TS 38.104 [17] section 5.4.2.1. The frequency provided in this IE identifies the absolute frequency position of the reference resource block (Common RB 0) of the carrier. Its lowest subcarrier is also known as Point A. |
| Frequency Shift 7p5khz | O |  | ENUMERATED (false, true, ...) | Indicate whether the value of Δshift is 0kHz or 7.5kHz when calculating FREF,shift as defined in Section 5.4.2.1 of TS 38.104 [17]. |
| Carrier List | O |  | NR Carrier List 9.3.1.137 |  |

| Range bound | Explanation |
| --- | --- |
| maxNRARFCN | Maximum value of NR ARFCNs. Value is 3279165. |

#### 9.3.1.218	MBS Session ID

This IE indicates the MBS Session ID uniquely identifies an MBS session.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TMGI | M |  | OCTET STRING (SIZE(6)) | Coded as Temporary Mobile Group Identity (TMGI) defined in TS 23.003 [23]. |
| NID | O |  | 9.3.1.155 |  |

#### 9.3.1.219	gNB-CU MBS F1AP ID

The gNB-CU MBS F1AP ID uniquely identifies the MBS association over the F1 interface within the gNB-CU.

NOTE: 	If F1-C signalling transport is shared among multiple interface instances, the value of the gNB-CU MBS F1AP ID is allocated so that it can be associated with the corresponding F1-C interface instance.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-CU MBS F1AP ID | M |  | INTEGER (0 .. 232 -1) |  |

#### 9.3.1.220	gNB-DU MBS F1AP ID

The gNB-DU MBS F1AP ID uniquely identifies the MBS association over the F1 interface within the gNB-DU.

NOTE: 	If F1-C signalling transport is shared among multiple interface instances, the value of the gNB-DU MBS F1AP ID is allocated so that it can be associated with the corresponding F1-C interface instance.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-DU MBS F1AP ID | M |  | INTEGER (0 .. 232 -1) |  |

#### 9.3.1.221	MBS Area Session ID

This IE indicates the Area Session ID for MBS Session with location dependent context.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Area Session ID | M |  | INTEGER (0 .. 65535, …) |  |

#### 9.3.1.222	MBS Service Area

This IE contains the MBS service area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS Service Area Information | M |  | 9.3.1.223 |  |
| >location dependent |  |  |  |  |
| >>MBS Service Area Information Location Dependent List |  | 1..maxnoofMBSServiceArea Information |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.221 |  |
| >>>MBS Service Area Information | M |  | 9.3.1.223 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSServiceAreaInformation | Maximum no. of MBS Service Area Information elements in the MBS Service Area Information Location Dependent List IE. Value is 256 |

#### 9.3.1.223	MBS Service Area information

This IE contains MBS service area information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Service Area Cell List |  | 0..<maxnoofCellsforMBS> |  |  |
| >NR CGI | M |  | 9.3.1.12 |  |
| MBS Service Area TAI List |  | 0..<maxnoofTAIforMBS> |  |  |
| >PLMN-Identity | M |  | 9.3.1.14 |  |
| >5GS TAC | M |  | 9.3.1.29 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsforMBS | Maximum no. of cells allowed within one MBS Service Area. Value is 512. |
| maxnoofTAIforMBS | Maximum no. of TAs allowed within one MBS Service Area. Value is 512. |

#### 9.3.1.224	MRB ID

This IE indicates the MRB ID as specified in TS 38.401 [4].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MRB ID | M |  | INTEGER (1.. 512, ...) |  |

#### 9.3.1.225	MBS CU to DU RRC Information

This IE indicates the MBS CU to DU RRC Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Broadcast Cell List | M |  |  |  |
| >MBS Broadcast Cell Item |  | 1 .. <maxCellingNBDU> |  |  |
| >>NR CGI | M |  | NR CGI 9.3.1.12 |  |
| >>mtch-neighbourCell | O |  | OCTET STRING | Includes the mtch-neighbourCell contained in the MBS-SessionInfoList IE, as defined in TS 38.331[8] |
| MBS Broadcast MRB List |  | 1 |  |  |
| >MBS Broadcast MRB Item |  | 1 .. <maxnoofMRBs> |  |  |
| >>MRB ID | M |  | 9.3.1.224 |  |
| >>MRB PDCP Config Broadcast | M |  | OCTET STRING | Includes the MRB-PDCP-ConfigBroadcast IE, as defined in TS 38.331[8]. |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofMRBs | Maximum no. MRBs allowed to be setup for one MBS session, the maximum value is 32. |

#### 9.3.1.226	MBS Broadcast Neighbour Cell List

This IE indicates a list of neighbour cells where ongoing MBS sessions provided via broadcast MRB in the current cells are also provided.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Broadcast Neighbour Cell List | M |  | OCTET STRING | Includes the MBS-NeighbourCellList IE, as defined in TS 38.331[8] |

#### 9.3.1.227	IAB Congestion Indication

This IE contains the IAB downlink congestion indication. This IE is only applicable if the gNB-DU is an IAB-DU or IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB Congestion Indication List |  | 1 |  |  |
| >IAB Congestion Indication Item |  | 1.. <maxnoofIABCongInd> |  |  |
| >>Child Node Identifier | M |  | BAP Address 9.3.1.111 | This IE identifies the child node, the link to which is congested. |
| >>BH RLC CH List |  | 0..1 |  |  |
| >>>BH RLC CH Item |  | 1.. <maxnoofBHRLCChannels> |  |  |
| >>>>BH RLC CH ID | M |  | BH RLC Channel ID 9.3.1.113 | This IE identifies the congested BH RLC channel over the link towards the node identified by the Child Node Identifier IE. |

| Range bound | Explanation |
| --- | --- |
| maxnoofIABCongInd | Maximum no. of congestion indications, the maximum value is 1024. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |

#### 9.3.1.228	F1-C Transfer Path NRDC

This IE indicates the transmission path of the F1-C traffic in NR-DC. This IE is only applicable if the UE is an IAB-MT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| F1-C Path NRDC | M |  | ENUMERATED (mcg, scg, both) | This IE indicates the transmission path of the F1-C traffic in NR-DC. |

#### 9.3.1.229	IAB TNL Address Exception

This IE indicates the list of TNL addresses, pertaining to the packets to be forwarded via the inter-IAB-donor-DU tunnel by the IAB-donor-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB TNL Address List |  | 1 |  |  |
| >IAB IAB TNL Address Item |  | 1.. <maxnoofTLAsIAB> |  |  |
| >>IAB TNL Address | M |  | 9.3.1.102 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTLAsIAB | Maximum no. of individual IPv4/IPv6 addresses or IPv6 address prefixes in one procedure execution. The value is 1024. |

#### 9.3.1.230	RB Set Configuration

This IE contains the RB Set Configuration. The IE is only applicable if the gNB-DU is an IAB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Subcarrier Spacing | M |  | ENUMERATED (kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, …) | Subcarrier spacing used as reference for the RB set configuration. |
| RB Set Size | M |  | ENUMERATED (rb2, rb4, rb8, rb16, rb32, rb64) | Number of PRBs in each RB set. If the RB sets of IAB-DU H/S/NA resource configuration do not cover the entire carrier bandwidth, the remaining RBs not part of an RB set configuration are considered as included in the last RB set. |
| Number of RB Sets | M |  | INTEGER(1.. maxnoofRBsetsPerCell) | Number of configured RB sets. The RB sets are contiguous and non-overlapping. If NR Carrier List IE(9.3.1.137) is provided, the start RB index of the first RB set is the RB index of the lowest common RB with the SCS provided by RB Set Configuration IE, which overlaps with the lowest usable RB across all SCS-specific carriers provided by the NR Carrier List IE for the IAB-DU cell. Otherwise, the start RB of the first RB set is aligned with point A for the IAB-DU cell. |

| Range bound | Explanation |
| --- | --- |
| maxnoofRBsetsPerCell | Maximum no. of RB sets per IAB-DU cell. Value is 8. |

#### 9.3.1.231	Survival Time

This IE indicates the Survival Time of the TSC QoS flow as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0.. 1920000, …) | Expressed in units of 1 us. |

#### 9.3.1.232	PDC Measurement Result

The purpose of this IE is to provide the PDC measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PDC Measured Results List |  | 1 |  |  |
| >PDC Measured Results Item |  | 1 .. <maxnoMeasPDC> |  |  |
| >>CHOICE Measured Results Value | M |  |  |  |
| >>>NR PDC Timing Advance |  |  |  |  |
| >>>>NR PDC Timing Advance | M |  | INTEGER (0..62500, …) | Value is expressed in unit of [64*] ns |
| >>>PDC gNB Rx-Tx Time Difference |  |  |  |  |
| >>>>PDC gNB Rx-Tx Time Difference | M |  | INTEGER (0.. 61565, …) | Report mapping as defined in TS 38.133 [38] |

| Range bound | Explanation |
| --- | --- |
| maxnoMeasPDC | Maximum no. of measured quantities that can be configured and reported with one message. Value is 16. Maximum is 1 in this release. |

#### 9.3.1.233	SCG Activation Request

This IE indicates whether the SCG resources are required to be activated or deactivated.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SCG Activation Request | M |  | ENUMERATED (activate SCG, deactivate SCG, …) |  |

#### 9.3.1.234	SCG Activation Status

This IE indicates the status of SCG resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SCG Activation Status | M |  | ENUMERATED (SCG activated, SCG deactivated, …) |  |

#### 9.3.1.235	Requested DL PRS Transmission Characteristics

This IE contains the requested PRS configuration for transmission by the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Requested DL-PRS Resource Set List |  | 1 |  |  |
| >Requested DL-PRS Resource Set Item |  | 1..<maxnoofPRSresourceSets> |  |  |
| >>PRS bandwidth | O |  | INTEGER(1..63) | 24,28,…,272 PRBs |
| >>Comb Size | O |  | ENUMERATED(2, 4, 6, 12, …) |  |
| >>Resource Set Periodicity | O |  | ENUMERATED(4,5,8,10,16,20,32,40,64,80,160,320,640,1280,2560,5120,10240,20480,40960,81920,…, 128, 256, 512) | Slots |
| >>Resource Repetition Factor | O |  | ENUMERATED(rf1,rf2,rf4,rf6,rf8,rf16,rf32,…) |  |
| >>Resource Number of Symbols | O |  | ENUMERATED(n2,n4,n6,n12,…) |  |
| >>Requested DL-PRS Resource List | O |  | 9.3.1.250 |  |
| >>Resource Set Start Time and Duration | O |  | Start Time and Duration 9.3.1.236 | This IE is ignored if the Start Time and Duration IE is present |
| Number of Frequency Layers | O |  | INTEGER(1..4) |  |
| Start Time and Duration | O |  | 9.3.1.236 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSets | Maximum no of PRS resources set. Value is 8. |

#### 9.3.1.236	Start Time and Duration

This IE contains the start time and/or duration for the on-demand DL-PRS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Start Time | O |  | Relative Time 1900 9.3.1.183 |  |
| Duration | O |  | INTEGER (0..90060, …) | Unit: seconds |

#### 9.3.1.237	PRS Transmission Off Information

This IE contains the information to turn off particular PRS transmissions.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE level | M |  |  |  |
| >TRP level |  |  | NULL |  |
| >PRS resource set level |  |  |  |  |
| >>PRS Resource Set List |  | 1 |  |  |
| >>>PRS Resource Set Item |  | 1..<maxnoofPRSresourceSet> |  |  |
| >>>>PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >PRS resource level |  |  |  |  |
| >>PRS Resource Set List |  | 1 |  |  |
| >>>PRS Resource Set Item |  | 1..<maxnoofPRSresourceSet> |  |  |
| >>>>PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>>>PRS Resource List |  | 1 |  |  |
| >>>>>PRS Resource Item |  | 1..<maxnoofPRSresource> |  |  |
| >>>>>>PRS Resource ID | M |  | INTEGER(0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSet | Maximum no of PRS resources set. Value is 8. |
| maxnoofPRSresource | Maximum no of PRS resources per PRS resource set. Value is 64. |

#### 9.3.1.238	UL-AoA Assistance Information

This information element contains the expected uplink Angle of Arrival and uncertainty range.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE AngleMeasurement | M |  |  |  |
| >Expected UL Angle of Arrival |  |  |  |  |
| >>Expected Azimuth AoA |  | 1 |  | Defined as (φAOA - ΔφAOA/2, φAOA + ΔφAOA/2) |
| >>Expected Azimuth AoA Value | M |  | INTEGER(0..3599) | φAOA component of Expected Azimuth AoA |
| >>Expected Azimuth AoA Uncertainty Range | M |  | INTEGER(0..3599) | ΔφAOA component of Expected Azimuth AoA |
| >>Expected Zenith AoA |  | 0..1 |  | Defined as (θZOA – ΔθZOA/2, θZOA + ΔθZOA/2) |
| >>Expected Zenith AoA Value | M |  | INTEGER(0..1799) | θZOA component of Expected Zenith AoA |
| >>Expected Zenith AoA Uncertainty Range | M |  | INTEGER(0..1799) | ΔθZOA component of Expected Zenith AoA |
| >Expected UL Angle of Arrival Zenith Only |  |  |  | Defined as (θZOA – ΔθZOA/2, θZOA + ΔθZOA/2) |
| >>Expected Zenith AoA Value | M |  | INTEGER(0..1799) | θZOA component of Expected Zenith AoA |
| >>Expected Zenith AoA Uncertainty Range | M |  | INTEGER(0..1799) | ΔθZOA component of Expected Zenith AoA |
| LCS to GCS Translation | O |  | 9.3.1.241 | If absent, the azimuth and zenith are provided in GCS. In case of zenith only, the z-axis of LCS is defined along the linear array axis. |

#### 9.3.1.239	Zenith Angle of Arrival Information

This information element contains the Zenith Angle of Arrival, which can correspond to linear array measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Zenith Angle of Arrival | M |  | INTEGER(0..1799) | TS 38.133 [38] |
| LCS to GCS Translation | O |  | 9.3.1.241 | If absent, the zenith is provided in GCS. the z-axis of LCS is defined along the linear array axis |

#### 9.3.1.240	On-demand PRS TRP Information

This IE contains on-demand PRS information for the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| On-demand PRS Request Allowed | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents an on-demand PRS transmission parameter: first bit: Resource Set Periodicity second bit: PRS Bandwidth third bit: Resource Repetition Factor fourth bit: Resource Number of Symbols fifth bit: Comb Size sixth bit: Number of Frequency Layers seventh bit: Start Time and Duration eighth bit: Off Indication ninth bit: QCL Information Other bits reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. |
| Allowed Resource Set Periodicity Values | O |  | BIT STRING (SIZE(24)) | This IE applies only if the first bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the Resource Set Periodicity IE defined in subclause 9.2.x1, first bit = 4 and so on. Bit 24 is reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all Resource Set Periodicity values are allowed to be requested. |
| Allowed PRS Bandwidth Values | O |  | BIT STRING (SIZE(64)) | This IE applies only if the second bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the PRS Bandwidth IE defined in subclause 9.2.x1, first bit = 1 and so on. Bit 64 is reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all PRS Bandwidth values are allowed to be requested. |
| Allowed Resource Repetition Factor Values | O |  | BIT STRING (SIZE(8)) | This IE applies only if the third bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the Resource Repetition Factor IE defined in subclause 9.2.x1, first bit = rf1 and so on. Bit 8 is reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all Resource Repetition Factor values are allowed to be requested. |
| Allowed Resource Number of Symbols Values | O |  | BIT STRING (SIZE(8)) | This IE applies only if the fourth bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the Resource Number of Symbols IE defined in subclause 9.2.x1, first bit = n2 and so on. Bits 5-8 are reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all Resource Number of Symbols values are allowed to be requested. |
| Allowed Comb Size Values | O |  | BIT STRING (SIZE(8)) | This IE applies only if the fifth bit of the On-demand PRS Request Allowed IE is set to ‘1’. Each position in the bitmap represents a value of the Comb Size IE defined in subclause 9.2.x1, first bit = 2 and so on. Bits 5-8 are reserved for future use. Value ‘1’ indicates ‘request allowed’, Value ‘0’ indicates ‘request not allowed’. If this IE is absent, all Comb Size values are allowed to be requested. |

#### 9.3.1.241	LCS to GCS Translation

This IE contains the LCS to GCS Translation information.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Alpha | M |  | INTEGER (0..3599) |  |
| Beta | M |  | INTEGER (0..3599) |  |
| Gamma | M |  | INTEGER (0..3599) |  |

#### 9.3.1.242	Response Time

This information element contains the response time of the measurement results reporting.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Time | M |  | INTEGER(1..128, …) |  |
| Time Unit | M |  | ENUMERATED(second, ten-seconds, ten-milliseconds, …) |  |

#### 9.3.1.243	ARP Location Information

This IE contains the relative position of ARP(s) to the TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| ARP Location Information |  | 1 |  |  |
| >ARP Location Information Item |  | 1..<maxnoARPs> |  |  |
| >>ARP ID | M |  | 9.3.1.244 |  |
| >>CHOICE ARP Location Type | M |  |  |  |
| >>>geodetic |  |  |  |  |
| >>>>ARP Position Relative Geodetic | M |  | Relative Geodetic Location 9.3.1.186 |  |
| >>>cartesian |  |  |  |  |
| >>>>ARP Position Relative Cartesian | M |  | Relative Cartesian Location 9.3.1.187 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoARPs | Maximum no. of ARPs associated with a TRP. Value is 16. |

#### 9.3.1.244	ARP ID

This IE is used to uniquely identify an ARP associated with a TRP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| ARP Identifier | M |  | INTEGER (1..16, …) |  |

#### 9.3.1.245	Multiple UL AoA

This information element contains the list of the multiple UL AOAs values.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL AoA List |  | 1 |  |  |
| >UL AoA item |  | 1..<maxnoofULAoAs > |  |  |
| >>CHOICE AngleMeasurement | M |  |  |  |
| >>>UL Angle of Arrival |  |  |  |  |
| >>>>UL Angle of Arrival | M |  | 9.3.1.167 |  |
| >>>UL Zenith Angle of Arrival |  |  |  |  |
| >>>>Zenith Angle of Arrival Information | M |  | 9.3.1.239 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofULAoAs | Maximum no of UL-AOAs values (pair of AOA & ZOA values) that can be reported. Value is 8 |

#### 9.3.1.246	UL SRS-RSRPP

This information element contains the UL SRS RSRPP measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| First Path RSRP Power | M |  | INTEGER (0..126) |  |

#### 9.3.1.247	SRS Resource type

This IE contains the SRS resource type.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE Reference Signal | M |  |  |  | - |  |
| >SRS |  |  |  |  |  |  |
| >>SRS Resource ID | M |  | INTEGER(0..63) |  | - |  |
| >Positioning SRS |  |  |  |  |  |  |
| >>Positioning SRS Resource ID | M |  | INTEGER(0..63) |  | - |  |
| SRS Port Index | O |  | ENUMERATED (id1000, id1001, id1002, id1003, …) | This IE may be present if the SRS Resource ID IE is present, and is ignored otherwise. | YES | ignore |

#### 9.3.1.248	Extended Additional Path List

This IE contains the extended additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..< maxNoPathExtended> |  |  |
| >CHOICE Relative Path Delay | M |  |  |  |
| >>k0 |  |  |  |  |
| >>>k0 | M |  | INTEGER(0..16351) |  |
| >>k1 |  |  |  |  |
| >>>k1 | M |  | INTEGER(0..8176) |  |
| >>k2 |  |  |  |  |
| >>>k2 | M |  | INTEGER(0..4088) |  |
| >>k3 |  |  |  |  |
| >>>k3 | M |  | INTEGER(0..2044) |  |
| >>k4 |  |  |  |  |
| >>>k4 | M |  | INTEGER(0..1022) |  |
| >>k5 |  |  |  |  |
| >>>k5 | M |  | INTEGER(0..511) |  |
| >Path Quality | O |  | TRP Measurement Quality 9.3.1.172 |  |
| >Path Power | O |  | UL SRS-RSRPP 9.3.1.246 |  |
| >Multiple UL AoA | O |  | 9.3.1.245 |  |

| Range bound | Explanation |
| --- | --- |
| maxNoPathExtended | Maximum no. of additional path measurement. Value is 8. |

#### 9.3.1.249	LoS/NLoS Information

This IE contains the LoS/NLoS information for UL measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE LoS/NLoS Indicator | M |  |  |  |
| >Soft Indicator |  |  |  |  |
| >>LoS/NLoS Indicator Soft | M |  | INTEGER (0..10) | Values provide the likelihood of a LOS propagation path in the range between 0 and 1 with 0.1 steps resolution. Value '0' indicates NLOS and value '1' indicates LOS. |
| >Hard Indicator |  |  |  |  |
| >>LoS/NLoS Indicator Hard | M |  | ENUMERATED (NLoS, LoS) |  |

#### 9.3.1.250	Requested DL-PRS Resource List

This IE contains the requested DL-PRS resource list.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Requested DL-PRS Resource List |  | 1 |  | NR-DL-PRS-Resource-r16 as defined in TS 37.355 [39] |
| >Requested DL-PRS Resource Item |  | 1..<maxnoofPRSresource> |  |  |
| >>CHOICE QCL Info | O |  |  |  |
| >>>SSB |  |  |  |  |
| >>>>NR PCI | M |  | INTEGER(0..1007) |  |
| >>>>SSB Index | O |  | INTEGER(0..63) |  |
| >>>DL-PRS |  |  |  |  |
| >>>>QCL Source PRS Resource Set ID | M |  | INTEGER(0..7) |  |
| >>>>QCL Source PRS Resource ID | O |  | INTEGER(0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresource | Maximum no of PRS resources per PRS resource set. Value is 64. |

#### 9.3.1.251	Void

#### 9.3.1.252	TRP Tx TEG Association

This information element contains the TRP Tx TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP TEG item |  | 1 .. <maxnoTRPTEGs> |  |  |
| >TRP Tx TEG Information | M |  | 9.3.1.281 |  |
| >DL-PRS Resource Set ID | M |  | INTEGER (0..7) |  |
| >DL-PRS Resource ID List |  | 0..1 |  |  |
| >>DL-PRS Resource ID Item |  | 1..< maxnoofPRS-ResourcesPerSet > |  |  |
| >>>DL-PRS Resource ID | M |  | INTEGER (0..63) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoTRPTEGs | Maximum no of reported TRP Tx TEG association. Value is 8. |
| maxnoofPRS-ResourcesPerSet | Maximum no of DL-PRS resources of the DL-PRS resource set of the TRP. Value is 64. |

#### 9.3.1.253	TRP TEG Information

This information element contains the TRP TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE TRP TEG | M |  |  |  |
| >RxTx TEG |  |  |  |  |
| >>TRP RxTx TEG Information | M |  | 9.3.1.282 |  |
| >>TRP Tx TEG Information | O |  | 9.3.1.281 |  |
| >Rx TEG |  |  |  |  |
| >>TRP Rx TEG Information | M |  | 9.3.1.280 |  |
| >>TRP Tx TEG Information | M |  | 9.3.1.281 |  |

#### 9.3.1.254	Measurement Characteristics Request Indicator

This IE contains the measurement characteristic information requested by the gNB-CU.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Measurement characteristic request indicator | M |  | BIT STRING (SIZE(16)) | Each position in the bitmap represents a requested measurement characteristic: first bit: Measurement Beam Information Second bit: Extended Additional Path List Third bit: Additional Path Power Fourth Bit: Multiple UL AoA of Additional Path Fifth bit: LoS/NLoS Information Sixth bit: TRP Rx TEG association for UL-TDOA Seventh bit: TRP RxTxTEG-ID information for DL+UL positioning. Eighth bit: SRS Resource Type Ninth bit: Multiple Measurement Instances Other bits reserved for future use. Value ‘1’ indicates ‘requested measurement characteristic’, Value ‘0’ indicates ‘not requested’. |

#### 9.3.1.255	UE Reporting Information

This IE contains the UE Reporting Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reporting Amount | M |  | ENUMERATED (0, 1, 2, 4, 8, 16, 32, 64) | Value 0 represents an infinite number of periodic reporting |
| Reporting Interval | M |  | ENUMERATED (none, 1, 2, 4, 8, 10, 16, 20, 32, 64) | Unit: seconds |

#### 9.3.1.256	TRP Beam Antenna Information

The IE provides the beam antenna information of the TRP. It includes either the explicit beam antenna information, or a reference to another TRP’s signalled configuration, or the indication that no change has occurred with respect to previously signalled configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE TRP Beam Antenna Info Item | M |  |  |  |
| >Reference |  |  |  |  |
| >>Associated TRP ID | M |  | TRP ID 9.3.1.197 | This IE specifies the TRP ID of the associated TRP from which the beam information parameters are adopted in Local Coordinate System (LCS). |
| >Explicit |  |  |  |  |
| >>TRP Beam Antenna Angles | M |  | 9.3.1.257 |  |
| >>LCS to GCS Translation | O |  | 9.3.1.241 | Included if the azimuth and elevation are not provided in GCS. |
| >No Change |  |  | NULL | No change compared to previously signalled configuration for this TRP. |

#### 9.3.1.257	TRP Beam Antenna Angles

The IE provides the beam antenna information of the TRP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP Beam Antenna Angles Item |  | 1..< maxnoAzimuthAngles> |  |  |
| >TRP Azimuth Angle | M |  | INTEGER (0..3599) | For GCS, the azimuth angle is measured counter-clockwise from geographical North. For LCS, the azimuth angle is measured counter-clockwise from the x-axis of the LCS. |
| >TRP Azimuth Angle fine | O |  | INTEGER (0..9) | Fine angle |
| >TRP Elevation Angle List |  | 1 |  |  |
| >>TRP Elevation Angle Item |  | 1..<maxnoElevationAngles> |  |  |
| >>>TRP Elevation Angle | M |  | INTEGER (0..1800) | For GCS, the elevation angle is measured relative to zenith and positive to the horizontal direction (elevation 0 deg. points to zenith, 90 deg to the horizon). For LCS, the elevation angle is measured relative to the z-axis of the LCS (elevation 0 deg. points to the z-axis, 90 deg to the x-y plane). |
| >>>TRP Elevation Angle fine | O |  | INTEGER (0..9) | Fine angle |
| >>>TRP Beam Power List |  | 1 |  | Relative power between DL-PRS Resources for the given Azimuth and Elevation Angle. The first Relative Power element in this list provides the peak power for this Azimuth/Elevation angle and is defined as 0dB power. All the remaining Relative Power Element's in this list provide the relative DL-PRS Resource power relative to this first element in the list. |
| >>>>TRP Beam Power Item |  | 2..< maxNumResourcesPerAngle> |  |  |
| >>>>>PRS Resource Set ID | O |  | INTEGER (0..7) | DL-PRS Resource Set ID of the DL-PRS Resource for which the Relative Power is provided. If this field is absent, the DL-PRS Resource Set ID for this instance of the Beam Power List is the same as the DL-PRS Resource Set ID of the previous instance in the Beam Power List. This field shall be included at least in the first instance of the Beam Power List. |
| >>>>>PRS Resource ID | M |  | INTEGER (0..63) | DL-PRS Resource for which the Relative Power is provided. |
| >>>>>TRP Beam Relative Power | M |  | INTEGER (0..30) | The power values span from -30 to 0dB |
| >>>>>TRP Beam Relative Power "fine" | O |  | INTEGER (0..9) | Relative Power with 0.1dB resolution. The power spans from -0.9 to 0dB |

| Range bound | Explanation |
| --- | --- |
| maxNumResourcesPerAngle | Maximum number of DL-PRS Resources per angle per TRP. Value is 24. |
| maxnoAzimuthAngles | Maximum number of azimuth angles per TRP. Value is 3600. |
| maxnoElevationAngles | Maximum number of elevation angles per azimuth angle/TRP. Value is 1801. |

#### 9.3.1.258	NR Paging eDRX Information

This IE indicates the NR Paging eDRX parameters for RRC_IDLE as defined in TS 38.304 [24].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR Paging eDRX Cycle Idle | M |  | ENUMERATED(hfquarter, hfhalf, hf1, hf2, hf4, hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …) | TeDRX,CN defined in TS 38.304 [24]. Unit: [number of hyperframes]. |
| NR Paging Time Window | O |  | ENUMERATED(s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, …, s17, s18, s19, s20, s21, s22, s23, s24, s25, s26, s27, s28, s29, s30, s31, s32) | Unit: [1.28 second]. |

#### 9.3.1.259	NR Paging eDRX Information for RRC INACTIVE

This IE indicates the NR Paging eDRX parameters for RRC_INACTIVE as defined in TS 38.304 [24].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR Paging eDRX Cycle Inactive | M |  | ENUMERATED (hfquarter, hfhalf, hf1, …) | TeDRX,RAN defined in TS 38.304 [24]. Unit: [number of hyperframes]. |

#### 9.3.1.260	QoE Metrics

This IE provides the RAN visible QoE measurement report to gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Application Layer Buffer Level List | O |  | OCTET STRING | As defined in TS 38.331 [8]. |
| Playout Delay for Media Startup | O |  | OCTET STRING | As defined in TS 38.331 [8]. |

#### 9.3.1.261	CG-SDT Session Info

This IE identifies an CG-SDT session for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  |

#### 9.3.1.262	SDT Information

This IE is used to indicate an SDT transaction and to provide the assistant information from the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SDT Indicator | M |  | ENUMERATED (true,…) |  |
| SDT Assistant Information | O |  | ENUMERATED (single packet, multiple packets, …) | “single packet” indicates no subsequent SDT transmission is expected. “multiple packets” indicates subsequent SDT transmission is expected. |

#### 9.3.1.263	Path Switch Configuration

This IE provides information for switching to an indirect path from a direct path.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Target Relay UE ID | M |  | BIT STRING (SIZE(24)) | Corresponds to the targetRelayUE-Identity contained in the CellGroupConfig IE, defined in TS 38.331 [8] |
| Remote UE Local ID | M |  | 9.3.1.267 |  |
| T420 | M |  | ENUMERATED (ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000) | Corresponds to the t420 contained in the CellGroupConfig IE, defined in TS 38.331 [8] |

#### 9.3.1.264	Sidelink Relay Configuration

This IE provides information of a U2N Remote UE when accessing the network via a U2N Relay UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| gNB-DU UE F1AP ID of Relay UE | M |  | gNB-DU UE F1AP ID 9.3.1.5 |  |
| Remote UE Local ID | M |  | 9.3.1.267 |  |
| Sidelink Configuration Container | O |  | OCTET STRING | Includes the SL-ConfigDedicatedNR IE as defined in subclause 6.3.5 in TS 38.331 [8] to carry PC5 Relay RLC channel configuration for Remote UE’s SRB1. |

#### 9.3.1.265	PC5 RLC Channel ID

This IE uniquely identifies a PC5 Relay RLC channel for a L2 U2N Remote UE or a L2 U2N Relay UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PC5 RLC Channel ID | M |  | INTEGER (1.. 512, ...) |  |

#### 9.3.1.266	Uu RLC Channel ID

This IE uniquely identifies a Uu Relay RLC channel for a L2 U2N Relay UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Uu RLC Channel ID | M |  | INTEGER (1..32) | Corresponds to information provided in the Uu-RelayRLC-ChannelID IE defined in 38.331 [8]. |

#### 9.3.1.267	Remote UE Local ID

This IE uniquely identifies a L2 U2N Remote UE within the connected Relay UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Remote UE Local ID | M |  | INTEGER (0..255, ...) | Corresponds to the sl-LocalIdentity contained in the SL-SRAP-Config IE defined in TS 38.331 [8]. |

#### 9.3.1.268	5G ProSe Authorized

This IE provides information on the authorization status of the UE for NR ProSe services.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5G ProSe Direct Discovery | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Discovery |
| 5G ProSe Direct Communication | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Direct Communication |
| 5G ProSe Layer-2 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 UE-to-Network Relay |
| 5G ProSe Layer-3 UE-to-Network Relay | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-3 UE-to-Network Relay |
| 5G ProSe Layer-2 Remote UE | O |  | ENUMERATED (authorized, not authorized, ...) | Indicates whether the UE is authorized for 5G ProSe Layer-2 Remote UE |

#### 9.3.1.269	PEIPS Assistance Information

This IE provides the information related to CN paging subgrouping for a particular UE, as specified in TS 38.304 [24].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN Subgroup ID | M |  | INTEGER (0..7, …) |  |

#### 9.3.1.270	UE Paging Capability

This IE provides the UE Paging Capability information needed for paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| INACTIVE State PO-Determination | O |  | ENUMERATED(supported,…) | Corresponds to the inactiveStatePO-Determination contained in the UERadioPagingInformation IE defined in TS 38.331 [8]. |
| RedCap Indication | O |  | ENUMERATED(true,…) |  |

#### 9.3.1.271	gNB-DU UE Slice Maximum Bit Rate List

This IE contains the UE Slice Maximum Bit Rate List as specified in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Slice Maximum Bit Rate Item |  | 1..<maxnoofSMBRValues> |  |  |
| >S-NSSAI | M |  | 9.3.1.38 |  |
| >UE Slice Maximum Bit Rate Uplink | M |  | Bit Rate 9.3.1.22 | This IE indicates the UE-Slice-MBR as specified in TS 23.501 [21] in the uplink direction. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSMBRValues | Maximum no. of SLICE MAXIMUM BIT RATE values for a UE. Value is 8. |

#### 9.3.1.272	Multicast MBS Session List

This IE indicates the Multicast MBS Sessions the UE has joined.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Multicast MBS Session Item |  | 1..<maxnoofMBSSessionsofUE> |  |  |
| >MBS Session ID | M |  | 9.3.1.218 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessionsofUE | Maximum no. of MBS sessions allowed towards one UE. Value is 256. |

#### 9.3.1.273	TAI NSAG Support List

This IE indicates the list of NSAGs configured at the gNB-DU and their associated S-NSSAIs as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NSAG Support Item |  | 1..<maxnoofNSAGs> |  |  |
| >NSAG ID | M |  | INTEGER (0.. 255, …) |  |
| >NSAG Slice Support List | M |  | Extended Slice Support List 9.3.1.165 | Indicates the list of slices which belong to the NSAG. |

| Range bound | Explanation |
| --- | --- |
| maxnoofNSAGs | Maximum no. of signalled NSAGs. Value is 256. |

#### 9.3.1.274	MDT PLMN Modification List

The purpose of the MDT PLMN List Modification IE is to provide the modified list of PLMN allowed for MDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MDT PLMN Modification List |  | 0..<maxnoofMDTPLMNs> |  | An empty list indicates there is no PLMN allowed for MDT. |
| >PLMN Identity | M |  | 9.3.1.14 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMDTPLMNs | Maximum no. of PLMNs in the MDT PLMN list. Value is 16. |

#### 9.3.1.275	MRB RLC Configuration

This ID provides MRB RLC Configuration Information and is provided by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MRB RLC Configuration | M |  | ENUMERATED ( rlc-um-ptp, rlc-am-ptp, rlc-um-dl-ptm, two-rlc-um-dl-ptp-and-dl-ptm, three-rlc-um-dl-ptp-ul-ptp-dl-ptm, two-rlc-am-ptp-um-dl-ptm, ...) | The various codepoints correspond to MRB configurations specified in TS 38.300 [6] as follows: "rlc-um-ptp ": Multicast MRB with DL only RLC-UM or bidirectional RLC-UM configuration for PTP transmission; "rlc-am-ptp " Multicast MRB with RLC-AM entity configuration for PTP transmission; " rlc-um-dl-ptm " Multicast MRB with DL only RLC-UM entity for PTM transmission; "two-rlc-um-dl-ptp-and-dl-ptm ": Multicast MRB with two RLC-UM entities, one DL only RLC-UM entity for PTP transmission and the other DL only RLC-UM entity for PTM transmission; "three-rlc-um-dl-ptp-ul-ptp-dl-ptm ": Multicast MRB with three RLC-UM entities, one DL RLC-UM entity and one UL RLC-UM entity for PTP transmission and the other DL only RLC-UM entity for PTM transmission; "two-rlc-am-ptp-um-dl-ptm "; Multicast MRB with two RLC entities, one RLC-AM entity for PTP transmission and the other DL only RLC-UM entity for PTM transmission; |

#### 9.3.1.276	Timing Error Margin

This information element contains the Timing error margin for the TRP Rx TEG, or TRP Tx TEG.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Timing Error Margin | M |  | ENUMERATED(Tc0, Tc2, Tc4, Tc6, Tc8, Tc12, Tc16, Tc20, Tc24, Tc32, Tc40, Tc48, Tc56, Tc64, Tc72, Tc80,…) |  |

#### 9.3.1.277	SDT Bearer Configuration Info

This IE contains RLC bearer configuration of each SDT bearer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SDT Bearer Config List |  | 1 |  |  |
| >SDT Bearer Config Item IEs |  | 1 .. <maxnoofSDTBearers> |  |  |
| >>CHOICE SDT Bearer Type | M |  |  |  |
| >>>SRB |  |  |  |  |
| >>>>SRB ID | M |  | 9.3.1.7 |  |
| >>>DRB |  |  |  |  |
| >>>>DRB ID | M |  | 9.3.1.8 |  |
| >>SDT RLC Bearer Configuration | M |  | OCTET STRING | Includes the RLC-BearerConfig IE defined in subclause 6.3.2 of TS 38.331 [8] |

| Range bound | Explanation |
| --- | --- |
| maxnoofSDTBearers | Maximum no. of SDT bearers. Value is the summation of maximum numbers of DRBs and SRBs, i.e., 72. |

#### 9.3.1.278	PosSIType List

This IE is used to indicate the list of positioning SI message to be broadcast.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PosSI type item IEs |  | 1.. <maxnoofPosSITypes> |  |  |
| >PosSI Type | M |  | INTEGER (1..32, ...) | Value “1” corresponds to the positioning SI message identified by the first SI message indicated in the posSI-SchedulingInfo IE in the SIB1 message, value "2" to the positioning SI message identified by the second SI message indicated in the posSI-SchedulingInfo IE in the SIB1 message, and so on, as defined in TS 38.331 [8]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofPosSITypes | Maximum no. of positioning SI types, the maximum value is 32. |

#### 9.3.1.279	IAB-DU Cell Resource Configuration-Mode-Info

This IE contains the IAB-DU Cell Resource Configuration-Mode-Info.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE IAB-DU Cell Resource Configuration-Mode-Info | M |  |  |  | - |  |
| >FDD |  |  |  |  |  |  |
| >>FDD Info |  | 1 |  |  | - |  |
| >>>gNB-DU Cell Resource Configuration-FDD-UL | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains FDD UL resource configuration of the gNB-DU cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | - |  |
| >>>gNB-DU Cell Resource Configuration-FDD-DL | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains FDD DL resource configuration of the gNB-DU cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | - |  |
| >>>UL Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  | YES | reject |
| >>>UL Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  | YES | reject |
| >>>UL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the UL Transmission Bandwidth IE shall be ignored. | YES | reject |
| >>>DL Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  | YES | reject |
| >>>DL Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  | YES | reject |
| >>>DL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the DLTransmission Bandwidth IE shall be ignored. | YES | reject |
| >TDD |  |  |  |  |  |  |
| >>TDD Info |  | 1 |  |  | - |  |
| >>>gNB-DU Cell Resource Configuration-TDD | M |  | gNB-DU Cell Resource Configuration 9.3.1.107 | Contains TDD resource configuration of the gNB-DU cell. Only applicable if the gNB-DU is an IAB-DU or an IAB-donor-DU. | - |  |
| >>>NR Frequency Info | O |  | NR Frequency Info 9.3.1.17 |  | YES | reject |
| >>>Transmission Bandwidth | O |  | Transmission Bandwidth 9.3.1.15 |  | YES | reject |
| >>>Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the Transmission Bandwidth IE shall be ignored. | YES | reject |

#### 9.3.1.280	TRP Rx TEG Information

This information element contains the TRP Rx TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP Rx TEG ID | M |  | INTEGER (0..31) |  |
| TRP Rx Timing Error Margin | M |  | Timing Error Margin 9.3.1.276 | Timing error margin associated to the TRP Rx TEG ID. |

#### 9.3.1.281	TRP Tx TEG Information

This information element contains the TRP Tx TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP Tx TEG ID | M |  | INTEGER (0..7) |  |
| TRP Tx Timing Error Margin | M |  | Timing Error Margin 9.3.1.276 | Timing error margin associated to the TRP Tx TEG ID. |

#### 9.3.1.282	TRP RxTx TEG Information

This information element contains the TRP RxTx TEG information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TRP RxTx TEG ID | M |  | INTEGER (0..255) |  |
| TRP RxTx Timing Error Margin | M |  | ENUMERATED(Tc0dot5, Tc1, Tc2, Tc4, Tc8, Tc12, Tc16, Tc20, Tc24, Tc32, Tc40, Tc48, Tc64, Tc80, Tc96, Tc128, …) | Timing error margin associated to the TRP RxTx TEG ID. |

#### 9.3.1.283	Uplink TxDirectCurrentTwoCarrierList Information

This IE contains the Uplink TxDirectCurrentTwoCarrierList information that is configured by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Uplink TxDirectCurrentTwoCarrierList Information | M |  | OCTET STRING | Includes the UplinkTxDirectCurrentTwoCarrierList IE as defined in TS 38.331 [8]. |

#### 9.3.1.284	Uplink TxDirectCurrentMoreCarrierList Information

This IE contains the Uplink TxDirectCurrentMoreCarrierList information that is configured by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Uplink TxDirectCurrentMoreCarrierList Information | M |  | OCTET STRING | Includes the UplinkTxDirectCurrentMoreCarrierList IE as defined in TS 38.331 [8]. |

#### 9.3.1.285	Extended UE Identity Index Value

This IE is used by the gNB-DU to calculate the Paging Frame and Paging Occasion for eDRX, and the UE_ID based subgroup ID as specified in TS 38.304 [24].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Extended UE Identity Index Value | M |  | BIT STRING (SIZE(16)) |  |

#### 9.3.1.286	Hashed UE Identity Index Value

This IE is the 13 Most Significant Bits (MSBs) of the Hashed ID defined in TS 38.304 [24].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Hashed UE Identity Index Value | M |  | BIT STRING (SIZE(13, …)) |  |

#### 9.3.1.287	Broadcast Area Scope

This IE contains the Broadcast Area where the broadcast session is delivered.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE BroadcastAreaScope | M |  |  |  |
| >CompleteSuccess |  |  | NULL |  |
| >PartialSuccess |  |  |  |  |
| >>Broadcast Cell List |  | 1 |  |  |
| >>>Broadcast Cell Item |  | 1 .. <maxcellingNBDU> |  |  |
| >>>>Cell ID | M |  | NR CGI 9.3.1.12 | Identifier of the cells that establish the broadcast service successfully. |

| Range bound | Explanation |
| --- | --- |
| maxcellingNBDU | Maximum no. of cells which establish the MRBs successfully in one DU, the maximum value is 512. |

### 9.3.2	Transport Network Layer Related IEs

#### 9.3.2.1	UP Transport Layer Information

The UP Transport Layer Information IE identifies an F1 transport bearer associated to a DRB. It contains a Transport Layer Address and a GTP Tunnel Endpoint Identifier. The Transport Layer Address is an IP address to be used for the F1 user plane transport. The GTP Tunnel Endpoint Identifier is to be used for the user plane transport between gNB-CU and gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Transport Layer Information | M |  |  |  |
| >GTP Tunnel |  |  |  |  |
| >>Transport Layer Address | M |  | 9.3.2.3 |  |
| >>GTP-TEID | M |  | 9.3.2.2 |  |

#### 9.3.2.2	GTP-TEID

The GTP-TEID IE is the GTP Tunnel Endpoint Identifier to be used for the user plane transport between the gNB-CU and gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| GTP-TEID | M |  | OCTET STRING (SIZE(4)) | For details and range, see TS 29.281 [18]. |

#### 9.3.2.3	Transport Layer Address

This Transport Layer Address IE is an IP address.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport Layer Address | M |  | BIT STRING (SIZE(1..160, ...)) | The Radio Network Layer is not supposed to interpret the address information. It should pass it to the Transport Layer for interpretation. For details, see TS 38.414 [19]. |

#### 9.3.2.4	CP Transport Layer Information

This IE is used to provide the F1 control plane transport layer information associated with a gNB-CU – gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE CP Transport Layer Information |  |  |  |  | - |  |
| >Endpoint-IP-address |  |  |  |  |  |  |
| >>Endpoint IP address | M |  | Transport Layer Address 9.3.2.3 |  | - |  |
| >Endpoint-IP-address-and-port |  |  |  |  |  |  |
| >>Endpoint IP address | M |  | Transport Layer Address 9.3.2.3 |  | - |  |
| >>Port Number | M |  | BIT STRING (SIZE(16)) |  | YES | reject |

#### 9.3.2.5	Transport Layer Address Info

This IE is used for signalling TNL Configuration information for IPSec tunnel over which GTP traffic is transmitted.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport UP Layer Address Info to Add List |  | 0..1 |  |  |
| >Transport UP Layer Address Info to Add Item |  | 1..<maxnoofTLAs> |  |  |
| >>IP-Sec Transport Layer Address | M |  | Transport Layer Address 9.3.2.3 | Transport Layer Address for IP-Sec endpoint. |
| >>GTP Transport Layer Address To Add List |  | 0..1 |  |  |
| >>>GTP Transport Layer Address To Add Item |  | 1..<maxnoofGTPTLAs> |  |  |
| >>>>GTP Transport Layer Address Info | M |  | Transport Layer Address 9.3.2.3 | GTP Transport Layer Address for GTP end-points. |
| Transport UP Layer Address Info to Remove List |  | 0..1 |  |  |
| >Transport UP Layer Address Info to Remove Item |  | 1..<maxnoofTLAs> |  |  |
| >>IP-Sec Transport Layer Address | M |  | Transport Layer Address 9.3.2.3 | Transport Layer Address for IP-Sec endpoint. |
| >>GTP Transport Layer Address To Remove List |  | 0..1 |  |  |
| >>>GTP Transport Layer Address To Remove Item |  | 1..<maxnoofGTPTLAs> |  |  |
| >>>>GTP Transport Layer Address Info | M |  | Transport Layer Address 9.3.2.3 | GTP Transport Layer Address for GTP end-points. |

| Range bound | Explanation |
| --- | --- |
| maxnoofTLAs | Maximum no. of F1 Transport Layer Address in the message. Value is 16. |
| maxnoofGTPTLAs | Maximum no. of F1 GTP Transport Layer Address for a GTP end-point in the message. Value is 16. |

#### 9.3.2.6	URI

This IE is an URI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| URI | M |  | VisibleString | String representing URI (Uniform Resource Identifier) |

#### 9.3.2.7	BC Bearer Context F1-U TNL Info

This IE contains F1-U TNL information for an MBS Session. In case of locaction dependent MBS sessions, it also contains per Area Session ID F1-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS F1-U Information | M |  | UP Transport Layer Information 9.3.2.1 |  |
| >location dependent |  |  |  |  |
| >>Location dependent MBS F1-U Information |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.221 |  |
| >>>MBS F1-U Information | M |  | UP Transport Layer Information 9.3.2.1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

#### 9.3.2.8	MBS Multicast F1-U Context Descriptor

This IE contains a reference to a Multicast F1-U Context, information about the usage of the MBS Multicast F1-U Context and may contain an MBS Area Session ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Multicast F1-U Context Reference F1 | M |  | 9.3.2.11 |  |
| MC F1-U Context usage | M |  | ENUMERATED (ptm, ptp, ptp retransmission, ptp forwarding, ...) | "ptm" indicates that the Multicast F1-U Context is setup for ptm transmissions; decided by the DU. "ptp" indicates that the Multicast F1-U Context is setup for ptp transmissions; decided by the DU. "ptp retransmission" indicates that the Multicast F1-U Context is setup for ptp retransmissions (based on PDCP Status Report); requested by the CU "ptp forwarding" indicates that the Multicast F1-U Context is setup for transmitting from a defined MBS Progress Information status onwards; requested by the CU. |
| MBS Area Session ID | O |  | 9.3.1.221 |  |

#### 9.3.2.9	Void

Void

#### 9.3.2.10	MBS PTP Retransmission Tunnel Required

This IE indicates the request to establishment of a PTP Retransmission F1-U Tunnel for retransmitting user data for a multicast MBS Session.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS PTP Retransmission Tunnel Required | M |  | ENUMERATED (true, ...) |  |

#### 9.3.2.11	Multicast F1-U Context Reference F1

This IE contains a reference to a Multicast F1-U Context associated with an MBS Session context in a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Multicast F1-U Bearer Context Reference | M |  | OCTET STRING (SIZE(4)) | This value is allocated to uniquely denote an Multicast F1-U Context within an MBS-associated logical F1-connection. |

#### 9.3.2.12	MRB Progress Information

This IE contains the MRB progress Information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MRB Progress Information | M |  |  |  |
| >12bits |  |  |  |  |
| >>PDCP SN Length 12 | M |  | INTEGER (0..4095) |  |
| >18bits |  |  |  |  |
| >>PDCP SN Length 18 | M |  | INTEGER (0..262143) |  |

#### 9.3.2.13	Multicast F1-U Context Reference CU

This IE contains a reference to a Multicast F1-U Context associated with MBS session resources allocated in the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Multicast F1-U Bearer Context Reference CU | M |  | OCTET STRING (SIZE(4)) | This value is allocated to uniquely denote a Multicast F1-U Context associated with multicast MBS session resources allocated in the gNB-CU. NOTE: If E1 is deployed, the Multicast F1-U Bearer Context Reference CU IE refers to the Multicast F1-U Context ReferenceE1 IE as specified in TS 37.483 [47]. |

## 9.4	Message and Information Element Abstract Syntax (with ASN.1)

### 9.4.1	General

F1AP ASN.1 definition conforms to ITU-T Recommendation X.691 [5], ITU-T Recommendation X.680 [12] and ITU-T Recommendation X.681 [13].

The ASN.1 definition specifies the structure and content of F1AP messages. F1AP messages can contain any IEs specified in the object set definitions for that message without the order or number of occurrence being restricted by ASN.1. However, for this version of the standard, a sending entity shall construct an F1AP message according to the PDU definitions module and with the following additional rules:

-	IEs shall be ordered (in an IE container) in the order they appear in object set definitions.

-	Object set definitions specify how many times IEs may appear. An IE shall appear exactly once if the presence field in an object has value "mandatory". An IE may appear at most once if the presence field in an object has value "optional" or "conditional". If in a tabular format there is multiplicity specified for an IE (i.e., an IE list) then in the corresponding ASN.1 definition the list definition is separated into two parts. The first part defines an IE container list where the list elements reside. The second part defines list elements. The IE container list appears as an IE of its own. For this version of the standard an IE container list may contain only one kind of list elements.

NOTE:	In the above "IE" means an IE in the object set with an explicit ID. If one IE needs to appear more than once in one object set, then the different occurrences will have different IE IDs.

If an F1AP message that is not constructed as defined above is received, this shall be considered as Abstract Syntax Error, and the message shall be handled as defined for Abstract Syntax Error in clause 10.

### 9.4.2	Usage of private message mechanism for non-standard use

The private message mechanism for non-standard use may be used:

-	for special operator- (and/or vendor) specific features considered not to be part of the basic functionality, i.e., the functionality required for a complete and high-quality specification in order to guarantee multivendor interoperability;

-	by vendors for research purposes, e.g., to implement and evaluate new algorithms/features before such features are proposed for standardisation.

The private message mechanism shall not be used for basic functionality. Such functionality shall be standardised.

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

F1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM F1AP-CommonDataTypes

Reset,

ResetAcknowledge,

F1SetupRequest,

F1SetupResponse,

F1SetupFailure,

GNBDUConfigurationUpdate,

GNBDUConfigurationUpdateAcknowledge,

GNBDUConfigurationUpdateFailure,

GNBCUConfigurationUpdate,

GNBCUConfigurationUpdateAcknowledge,

GNBCUConfigurationUpdateFailure,

UEContextSetupRequest,

UEContextSetupResponse,

UEContextSetupFailure,

UEContextReleaseCommand,

UEContextReleaseComplete,

UEContextModificationRequest,

UEContextModificationResponse,

UEContextModificationFailure,

UEContextModificationRequired,

UEContextModificationConfirm,

ErrorIndication,

UEContextReleaseRequest,

DLRRCMessageTransfer,

ULRRCMessageTransfer,

GNBDUResourceCoordinationRequest,

GNBDUResourceCoordinationResponse,

PrivateMessage,

UEInactivityNotification,

InitialULRRCMessageTransfer,

SystemInformationDeliveryCommand,

Paging,

Notify,

WriteReplaceWarningRequest,

WriteReplaceWarningResponse,

PWSCancelRequest,

PWSCancelResponse,

PWSRestartIndication,

PWSFailureIndication,

GNBDUStatusIndication,

RRCDeliveryReport,

UEContextModificationRefuse,

F1RemovalRequest,

F1RemovalResponse,

F1RemovalFailure,

NetworkAccessRateReduction,

TraceStart,

DeactivateTrace,

DUCURadioInformationTransfer,

CUDURadioInformationTransfer,

BAPMappingConfiguration,

BAPMappingConfigurationAcknowledge,

BAPMappingConfigurationFailure,

GNBDUResourceConfiguration,

GNBDUResourceConfigurationAcknowledge,

GNBDUResourceConfigurationFailure,

IABTNLAddressRequest,

IABTNLAddressResponse,

IABTNLAddressFailure,

IABUPConfigurationUpdateRequest,

IABUPConfigurationUpdateResponse,

IABUPConfigurationUpdateFailure,

ResourceStatusRequest,

ResourceStatusResponse,

ResourceStatusFailure,

ResourceStatusUpdate,

AccessAndMobilityIndication,

ReferenceTimeInformationReportingControl,

ReferenceTimeInformationReport,

AccessSuccess,

CellTrafficTrace,

PositioningMeasurementRequest,

PositioningMeasurementResponse,

PositioningMeasurementFailure,

PositioningAssistanceInformationControl,

PositioningAssistanceInformationFeedback,

PositioningMeasurementReport,

PositioningMeasurementAbort,

PositioningMeasurementFailureIndication,

PositioningMeasurementUpdate,

TRPInformationRequest,

TRPInformationResponse,

TRPInformationFailure,

PositioningInformationRequest,

PositioningInformationResponse,

PositioningInformationFailure,

PositioningActivationRequest,

PositioningActivationResponse,

PositioningActivationFailure,

PositioningDeactivation,

PositioningInformationUpdate,

E-CIDMeasurementInitiationRequest,

E-CIDMeasurementInitiationResponse,

E-CIDMeasurementInitiationFailure,

E-CIDMeasurementFailureIndication,

E-CIDMeasurementReport,

E-CIDMeasurementTerminationCommand,

BroadcastContextSetupRequest,

BroadcastContextSetupResponse,

BroadcastContextSetupFailure,

BroadcastContextReleaseCommand,

BroadcastContextReleaseComplete,

BroadcastContextReleaseRequest,

BroadcastContextModificationRequest,

BroadcastContextModificationResponse,

BroadcastContextModificationFailure,

MulticastGroupPaging,

MulticastContextSetupRequest,

MulticastContextSetupResponse,

MulticastContextSetupFailure,

MulticastContextReleaseCommand,

MulticastContextReleaseComplete,

MulticastContextReleaseRequest,

MulticastContextModificationRequest,

MulticastContextModificationResponse,

MulticastContextModificationFailure,

MulticastDistributionSetupRequest,

MulticastDistributionSetupResponse,

MulticastDistributionSetupFailure,

MulticastDistributionReleaseCommand,

MulticastDistributionReleaseComplete,

PDCMeasurementInitiationRequest,

PDCMeasurementInitiationResponse,

PDCMeasurementInitiationFailure,

PDCMeasurementReport,

PDCMeasurementTerminationCommand,

PDCMeasurementFailureIndication,

PRSConfigurationRequest,

PRSConfigurationResponse,

PRSConfigurationFailure,

MeasurementPreconfigurationRequired,

MeasurementPreconfigurationConfirm,

MeasurementPreconfigurationRefuse,

MeasurementActivation,

QoEInformationTransfer,

PosSystemInformationDeliveryCommand

FROM F1AP-PDU-Contents

id-Reset,

id-F1Setup,

id-gNBDUConfigurationUpdate,

id-gNBCUConfigurationUpdate,

id-UEContextSetup,

id-UEContextRelease,

id-UEContextModification,

id-UEContextModificationRequired,

id-ErrorIndication,

id-UEContextReleaseRequest,

id-DLRRCMessageTransfer,

id-ULRRCMessageTransfer,

id-GNBDUResourceCoordination,

id-privateMessage,

id-UEInactivityNotification,

id-InitialULRRCMessageTransfer,

id-SystemInformationDeliveryCommand,

id-Paging,

id-Notify,

id-WriteReplaceWarning,

id-PWSCancel,

id-PWSRestartIndication,

id-PWSFailureIndication,

id-GNBDUStatusIndication,

id-RRCDeliveryReport,

id-F1Removal,

id-NetworkAccessRateReduction,

id-TraceStart,

id-DeactivateTrace,

id-DUCURadioInformationTransfer,

id-CUDURadioInformationTransfer,

id-BAPMappingConfiguration,

id-GNBDUResourceConfiguration,

id-IABTNLAddressAllocation,

id-IABUPConfigurationUpdate,

id-resourceStatusReportingInitiation,

id-resourceStatusReporting,

id-accessAndMobilityIndication,

id-ReferenceTimeInformationReportingControl,

id-ReferenceTimeInformationReport,

id-accessSuccess,

id-cellTrafficTrace,

id-PositioningMeasurementExchange,

id-PositioningAssistanceInformationControl,

id-PositioningAssistanceInformationFeedback,

id-PositioningMeasurementReport,

id-PositioningMeasurementAbort,

id-PositioningMeasurementFailureIndication,

id-PositioningMeasurementUpdate,

id-TRPInformationExchange,

id-PositioningInformationExchange,

id-PositioningActivation,

id-PositioningDeactivation,

id-PositioningInformationUpdate,

id-E-CIDMeasurementInitiation,

id-E-CIDMeasurementFailureIndication,

id-E-CIDMeasurementReport,

id-E-CIDMeasurementTermination,

id-BroadcastContextSetup,

id-BroadcastContextRelease,

id-BroadcastContextReleaseRequest,

id-BroadcastContextModification,

id-MulticastGroupPaging,

id-MulticastContextSetup,

id-MulticastContextRelease,

id-MulticastContextReleaseRequest,

id-MulticastContextModification,

id-MulticastDistributionSetup,

id-MulticastDistributionRelease,

id-PDCMeasurementInitiation,

id-PDCMeasurementInitiationRequest,

id-PDCMeasurementInitiationResponse,

id-PDCMeasurementInitiationFailure,

id-PDCMeasurementTerminationCommand,

id-PDCMeasurementFailureIndication,

id-PDCMeasurementReport,

id-pRSConfigurationExchange,

id-measurementPreconfiguration,

id-measurementActivation,

id-QoEInformationTransfer,

id-PosSystemInformationDeliveryCommand

FROM F1AP-Constants

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

F1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

F1AP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

choice-extension	ProtocolIE-SingleContainer { { F1AP-PDU-ExtIEs} }

}

F1AP-PDU-ExtIEs F1AP-PROTOCOL-IES ::= { -- this extension is not used

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			F1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

F1AP-ELEMENTARY-PROCEDURES F1AP-ELEMENTARY-PROCEDURE ::= {

F1AP-ELEMENTARY-PROCEDURES-CLASS-1			|

F1AP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-1 F1AP-ELEMENTARY-PROCEDURE ::= {

reset							|

f1Setup							|

gNBDUConfigurationUpdate		|

gNBCUConfigurationUpdate		|

uEContextSetup					|

uEContextRelease				|

uEContextModification			|

uEContextModificationRequired	|

writeReplaceWarning				|

pWSCancel						|

gNBDUResourceCoordination		|

f1Removal						|

bAPMappingConfiguration			|

gNBDUResourceConfiguration		|

iABTNLAddressAllocation			|

iABUPConfigurationUpdate		|

resourceStatusReportingInitiation	|

positioningMeasurementExchange	|

tRPInformationExchange			|

positioningInformationExchange	|

positioningActivation			|

e-CIDMeasurementInitiation		|

broadcastContextSetup			|

broadcastContextRelease			|

broadcastContextModification	|

multicastContextSetup			|

multicastContextRelease			|

multicastContextModification	|

multicastDistributionSetup		|

multicastDistributionRelease	|

pDCMeasurementInitiation		|

pRSConfigurationExchange		|

measurementPreconfiguration	,

...

}

F1AP-ELEMENTARY-PROCEDURES-CLASS-2 F1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

uEContextReleaseRequest					|

dLRRCMessageTransfer					|

uLRRCMessageTransfer					|

uEInactivityNotification				|

privateMessage							|

initialULRRCMessageTransfer				|

systemInformationDelivery				|

paging									|

notify									|

pWSRestartIndication					|

pWSFailureIndication					|

gNBDUStatusIndication					|

rRCDeliveryReport						|

networkAccessRateReduction				|

traceStart								|

deactivateTrace							|

dUCURadioInformationTransfer			|

cUDURadioInformationTransfer			|

resourceStatusReporting					|

accessAndMobilityIndication				|

referenceTimeInformationReportingControl|

referenceTimeInformationReport			|

accessSuccess							|

cellTrafficTrace						|

positioningAssistanceInformationControl		|

positioningAssistanceInformationFeedback	|

positioningMeasurementReport				|

positioningMeasurementAbort					|

positioningMeasurementFailureIndication		|

positioningMeasurementUpdate				|

positioningDeactivation						|

e-CIDMeasurementFailureIndication			|

e-CIDMeasurementReport						|

e-CIDMeasurementTermination					|

positioningInformationUpdate				|

multicastGroupPaging						|

broadcastContextReleaseRequest				|

multicastContextReleaseRequest				|

pDCMeasurementReport						|

pDCMeasurementTerminationCommand			|

pDCMeasurementFailureIndication				|

measurementActivation						|

qoEInformationTransfer						|

posSystemInformationDelivery,

...

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

reset F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Reset

SUCCESSFUL OUTCOME		ResetAcknowledge

PROCEDURE CODE			id-Reset

CRITICALITY				reject

}

f1Setup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		F1SetupRequest

SUCCESSFUL OUTCOME		F1SetupResponse

UNSUCCESSFUL OUTCOME	F1SetupFailure

PROCEDURE CODE			id-F1Setup

CRITICALITY				reject

}

gNBDUConfigurationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUConfigurationUpdate

SUCCESSFUL OUTCOME		GNBDUConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNBDUConfigurationUpdateFailure

PROCEDURE CODE			id-gNBDUConfigurationUpdate

CRITICALITY				reject

}

gNBCUConfigurationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBCUConfigurationUpdate

SUCCESSFUL OUTCOME		GNBCUConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	GNBCUConfigurationUpdateFailure

PROCEDURE CODE			id-gNBCUConfigurationUpdate

CRITICALITY				reject

}

uEContextSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextSetupRequest

SUCCESSFUL OUTCOME		UEContextSetupResponse

UNSUCCESSFUL OUTCOME	UEContextSetupFailure

PROCEDURE CODE			id-UEContextSetup

CRITICALITY				reject

}

uEContextRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseCommand

SUCCESSFUL OUTCOME		UEContextReleaseComplete

PROCEDURE CODE			id-UEContextRelease

CRITICALITY				reject

}

uEContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationRequest

SUCCESSFUL OUTCOME		UEContextModificationResponse

UNSUCCESSFUL OUTCOME	UEContextModificationFailure

PROCEDURE CODE			id-UEContextModification

CRITICALITY				reject

}

uEContextModificationRequired F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextModificationRequired

SUCCESSFUL OUTCOME		UEContextModificationConfirm

UNSUCCESSFUL OUTCOME	UEContextModificationRefuse

PROCEDURE CODE			id-UEContextModificationRequired

CRITICALITY				reject

}

writeReplaceWarning F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		WriteReplaceWarningRequest

SUCCESSFUL OUTCOME		WriteReplaceWarningResponse

PROCEDURE CODE			id-WriteReplaceWarning

CRITICALITY				reject

}

pWSCancel F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSCancelRequest

SUCCESSFUL OUTCOME		PWSCancelResponse

PROCEDURE CODE			id-PWSCancel

CRITICALITY				reject

}

errorIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-ErrorIndication

CRITICALITY				ignore

}

uEContextReleaseRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEContextReleaseRequest

PROCEDURE CODE			id-UEContextReleaseRequest

CRITICALITY				ignore

}

initialULRRCMessageTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		InitialULRRCMessageTransfer

PROCEDURE CODE			id-InitialULRRCMessageTransfer

CRITICALITY				ignore

}

dLRRCMessageTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DLRRCMessageTransfer

PROCEDURE CODE			id-DLRRCMessageTransfer

CRITICALITY				ignore

}

uLRRCMessageTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ULRRCMessageTransfer

PROCEDURE CODE			id-ULRRCMessageTransfer

CRITICALITY				ignore

}

uEInactivityNotification  F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UEInactivityNotification

PROCEDURE CODE			id-UEInactivityNotification

CRITICALITY				ignore

}

gNBDUResourceCoordination F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUResourceCoordinationRequest

SUCCESSFUL OUTCOME		GNBDUResourceCoordinationResponse

PROCEDURE CODE			id-GNBDUResourceCoordination

CRITICALITY				reject

}

privateMessage F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

systemInformationDelivery F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		SystemInformationDeliveryCommand

PROCEDURE CODE			id-SystemInformationDeliveryCommand

CRITICALITY				ignore

}

paging F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Paging

PROCEDURE CODE			id-Paging

CRITICALITY				ignore

}

notify F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		Notify

PROCEDURE CODE			id-Notify

CRITICALITY				ignore

}

networkAccessRateReduction F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		NetworkAccessRateReduction

PROCEDURE CODE			id-NetworkAccessRateReduction

CRITICALITY				ignore

}

pWSRestartIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSRestartIndication

PROCEDURE CODE			id-PWSRestartIndication

CRITICALITY				ignore

}

pWSFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PWSFailureIndication

PROCEDURE CODE			id-PWSFailureIndication

CRITICALITY				ignore

}

gNBDUStatusIndication 	F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUStatusIndication

PROCEDURE CODE			id-GNBDUStatusIndication

CRITICALITY				ignore

}

rRCDeliveryReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		RRCDeliveryReport

PROCEDURE CODE			id-RRCDeliveryReport

CRITICALITY				ignore

}

f1Removal F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		F1RemovalRequest

SUCCESSFUL OUTCOME		F1RemovalResponse

UNSUCCESSFUL OUTCOME	F1RemovalFailure

PROCEDURE CODE			id-F1Removal

CRITICALITY				reject

}

traceStart F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TraceStart

PROCEDURE CODE			id-TraceStart

CRITICALITY				ignore

}

deactivateTrace F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DeactivateTrace

PROCEDURE CODE			id-DeactivateTrace

CRITICALITY				ignore

}

dUCURadioInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		DUCURadioInformationTransfer

PROCEDURE CODE			id-DUCURadioInformationTransfer

CRITICALITY				ignore

}

cUDURadioInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CUDURadioInformationTransfer

PROCEDURE CODE			id-CUDURadioInformationTransfer

CRITICALITY				ignore

}

bAPMappingConfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BAPMappingConfiguration

SUCCESSFUL OUTCOME		BAPMappingConfigurationAcknowledge

UNSUCCESSFUL OUTCOME	BAPMappingConfigurationFailure

PROCEDURE CODE			id-BAPMappingConfiguration

CRITICALITY				reject

}

gNBDUResourceConfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		GNBDUResourceConfiguration

SUCCESSFUL OUTCOME		GNBDUResourceConfigurationAcknowledge

UNSUCCESSFUL OUTCOME	GNBDUResourceConfigurationFailure

PROCEDURE CODE			id-GNBDUResourceConfiguration

CRITICALITY				reject

}

iABTNLAddressAllocation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		IABTNLAddressRequest

SUCCESSFUL OUTCOME		IABTNLAddressResponse

UNSUCCESSFUL OUTCOME	IABTNLAddressFailure

PROCEDURE CODE			id-IABTNLAddressAllocation

CRITICALITY				reject

}

iABUPConfigurationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		IABUPConfigurationUpdateRequest

SUCCESSFUL OUTCOME		IABUPConfigurationUpdateResponse

UNSUCCESSFUL OUTCOME	IABUPConfigurationUpdateFailure

PROCEDURE CODE			id-IABUPConfigurationUpdate

CRITICALITY				reject

}

resourceStatusReportingInitiation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusRequest

SUCCESSFUL OUTCOME		ResourceStatusResponse

UNSUCCESSFUL OUTCOME	ResourceStatusFailure

PROCEDURE CODE			id-resourceStatusReportingInitiation

CRITICALITY				reject

}

resourceStatusReporting F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ResourceStatusUpdate

PROCEDURE CODE			id-resourceStatusReporting

CRITICALITY				ignore

}

accessAndMobilityIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AccessAndMobilityIndication

PROCEDURE CODE			id-accessAndMobilityIndication

CRITICALITY				ignore

}

referenceTimeInformationReportingControl F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ReferenceTimeInformationReportingControl

PROCEDURE CODE			id-ReferenceTimeInformationReportingControl

CRITICALITY				ignore

}

referenceTimeInformationReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ReferenceTimeInformationReport

PROCEDURE CODE			id-ReferenceTimeInformationReport

CRITICALITY				ignore

}

accessSuccess F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AccessSuccess

PROCEDURE CODE			id-accessSuccess

CRITICALITY				ignore

}

cellTrafficTrace F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		CellTrafficTrace

PROCEDURE CODE			id-cellTrafficTrace

CRITICALITY				ignore

}

positioningAssistanceInformationControl F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningAssistanceInformationControl

PROCEDURE CODE			id-PositioningAssistanceInformationControl

CRITICALITY				ignore

}

positioningAssistanceInformationFeedback F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningAssistanceInformationFeedback

PROCEDURE CODE			id-PositioningAssistanceInformationFeedback

CRITICALITY				ignore

}

positioningMeasurementExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementRequest

SUCCESSFUL OUTCOME		PositioningMeasurementResponse

UNSUCCESSFUL OUTCOME	PositioningMeasurementFailure

PROCEDURE CODE			id-PositioningMeasurementExchange

CRITICALITY				reject

}

positioningMeasurementReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementReport

PROCEDURE CODE			id-PositioningMeasurementReport

CRITICALITY				ignore

}

positioningMeasurementAbort F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementAbort

PROCEDURE CODE			id-PositioningMeasurementAbort

CRITICALITY				ignore

}

positioningMeasurementFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementFailureIndication

PROCEDURE CODE			id-PositioningMeasurementFailureIndication

CRITICALITY				ignore

}

positioningMeasurementUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningMeasurementUpdate

PROCEDURE CODE			id-PositioningMeasurementUpdate

CRITICALITY				ignore

}

tRPInformationExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		TRPInformationRequest

SUCCESSFUL OUTCOME		TRPInformationResponse

UNSUCCESSFUL OUTCOME	TRPInformationFailure

PROCEDURE CODE			id-TRPInformationExchange

CRITICALITY				reject

}

positioningInformationExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationRequest

SUCCESSFUL OUTCOME		PositioningInformationResponse

UNSUCCESSFUL OUTCOME	PositioningInformationFailure

PROCEDURE CODE			id-PositioningInformationExchange

CRITICALITY				reject

}

positioningActivation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningActivationRequest

SUCCESSFUL OUTCOME		PositioningActivationResponse

UNSUCCESSFUL OUTCOME	PositioningActivationFailure

PROCEDURE CODE			id-PositioningActivation

CRITICALITY				reject

}

positioningDeactivation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningDeactivation

PROCEDURE CODE			id-PositioningDeactivation

CRITICALITY				ignore

}

e-CIDMeasurementInitiation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementInitiationRequest

SUCCESSFUL OUTCOME		E-CIDMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	E-CIDMeasurementInitiationFailure

PROCEDURE CODE			id-E-CIDMeasurementInitiation

CRITICALITY				reject

}

e-CIDMeasurementFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementFailureIndication

PROCEDURE CODE			id-E-CIDMeasurementFailureIndication

CRITICALITY				ignore

}

e-CIDMeasurementReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementReport

PROCEDURE CODE			id-E-CIDMeasurementReport

CRITICALITY				ignore

}

e-CIDMeasurementTermination F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementTerminationCommand

PROCEDURE CODE			id-E-CIDMeasurementTermination

CRITICALITY				ignore

}

positioningInformationUpdate F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PositioningInformationUpdate

PROCEDURE CODE			id-PositioningInformationUpdate

CRITICALITY				ignore

}

broadcastContextSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextSetupRequest

SUCCESSFUL OUTCOME		BroadcastContextSetupResponse

UNSUCCESSFUL OUTCOME	BroadcastContextSetupFailure

PROCEDURE CODE			id-BroadcastContextSetup

CRITICALITY				reject

}

broadcastContextRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextReleaseCommand

SUCCESSFUL OUTCOME		BroadcastContextReleaseComplete

PROCEDURE CODE			id-BroadcastContextRelease

CRITICALITY				reject

}

broadcastContextReleaseRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextReleaseRequest

PROCEDURE CODE			id-BroadcastContextReleaseRequest

CRITICALITY				reject

}

broadcastContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastContextModificationRequest

SUCCESSFUL OUTCOME		BroadcastContextModificationResponse

UNSUCCESSFUL OUTCOME	BroadcastContextModificationFailure

PROCEDURE CODE			id-BroadcastContextModification

CRITICALITY				reject

}

multicastGroupPaging F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastGroupPaging

PROCEDURE CODE			id-MulticastGroupPaging

CRITICALITY				ignore

}

multicastContextSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextSetupRequest

SUCCESSFUL OUTCOME		MulticastContextSetupResponse

UNSUCCESSFUL OUTCOME	MulticastContextSetupFailure

PROCEDURE CODE			id-MulticastContextSetup

CRITICALITY				reject

}

multicastContextRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextReleaseCommand

SUCCESSFUL OUTCOME		MulticastContextReleaseComplete

PROCEDURE CODE			id-MulticastContextRelease

CRITICALITY				reject

}

multicastContextReleaseRequest F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextReleaseRequest

PROCEDURE CODE			id-MulticastContextReleaseRequest

CRITICALITY				reject

}

multicastContextModification F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastContextModificationRequest

SUCCESSFUL OUTCOME		MulticastContextModificationResponse

UNSUCCESSFUL OUTCOME	MulticastContextModificationFailure

PROCEDURE CODE			id-MulticastContextModification

CRITICALITY				reject

}

multicastDistributionSetup F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastDistributionSetupRequest

SUCCESSFUL OUTCOME		MulticastDistributionSetupResponse

UNSUCCESSFUL OUTCOME	MulticastDistributionSetupFailure

PROCEDURE CODE			id-MulticastDistributionSetup

CRITICALITY				reject

}

multicastDistributionRelease F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MulticastDistributionReleaseCommand

SUCCESSFUL OUTCOME		MulticastDistributionReleaseComplete

PROCEDURE CODE			id-MulticastDistributionRelease

CRITICALITY				reject

}

pDCMeasurementInitiation F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementInitiationRequest

SUCCESSFUL OUTCOME		PDCMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	PDCMeasurementInitiationFailure

PROCEDURE CODE			id-PDCMeasurementInitiation

CRITICALITY				reject

}

pDCMeasurementReport F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementReport

PROCEDURE CODE			id-PDCMeasurementReport

CRITICALITY				ignore

}

pDCMeasurementTerminationCommand F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementTerminationCommand

PROCEDURE CODE			id-PDCMeasurementTerminationCommand

CRITICALITY				ignore

}

pDCMeasurementFailureIndication F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PDCMeasurementFailureIndication

PROCEDURE CODE			id-PDCMeasurementFailureIndication

CRITICALITY				ignore

}

pRSConfigurationExchange F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PRSConfigurationRequest

SUCCESSFUL OUTCOME		PRSConfigurationResponse

UNSUCCESSFUL OUTCOME	PRSConfigurationFailure

PROCEDURE CODE			id-pRSConfigurationExchange

CRITICALITY				reject

}

measurementPreconfiguration F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementPreconfigurationRequired

SUCCESSFUL OUTCOME		MeasurementPreconfigurationConfirm

UNSUCCESSFUL OUTCOME	MeasurementPreconfigurationRefuse

PROCEDURE CODE			id-measurementPreconfiguration

CRITICALITY				reject

}

measurementActivation		F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementActivation

PROCEDURE CODE			id-measurementActivation

CRITICALITY				ignore

}

qoEInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		QoEInformationTransfer

PROCEDURE CODE			id-QoEInformationTransfer

CRITICALITY				ignore

}

posSystemInformationDelivery F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PosSystemInformationDeliveryCommand

PROCEDURE CODE			id-PosSystemInformationDeliveryCommand

CRITICALITY				ignore

}

END

-- ASN1STOP

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

BroadcastMRBs-FailedToBeModified-Item,

BroadcastMRBs-FailedToBeSetup-Item,

BroadcastMRBs-FailedToBeSetupMod-Item,

BroadcastMRBs-Modified-Item,

BroadcastMRBs-Setup-Item,

BroadcastMRBs-SetupMod-Item,

BroadcastMRBs-ToBeModified-Item,

BroadcastMRBs-ToBeReleased-Item,

BroadcastMRBs-ToBeSetup-Item,

BroadcastMRBs-ToBeSetupMod-Item,

Candidate-SpCell-Item,

Cause,

Cells-Failed-to-be-Activated-List-Item,

Cells-Status-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

CellULConfigured,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRB-Activity-Item,

DRBID,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRB-Notify-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DRXConfigurationIndicator,

DUtoCURRCInformation,

EUTRANQoS,

ExecuteDuplication,

FullConfiguration,

GNB-CU-MBS-F1AP-ID,

GNB-CU-UE-F1AP-ID,

GNB-DU-MBS-F1AP-ID,

GNB-DU-UE-F1AP-ID,

GNB-DU-ID,

GNB-DU-Served-Cells-Item,

GNB-DU-System-Information,

GNB-CU-Name,

GNB-DU-Name,

InactivityMonitoringRequest,

InactivityMonitoringResponse,

LowerLayerPresenceStatusChange,

MBS-Area-Session-ID,

MBS-CUtoDURRCInformation,

MBSMulticastF1UContextDescriptor,

MBS-Session-ID,

MBS-ServiceArea,

MulticastF1UContextReferenceCU,

MulticastF1UContext-ToBeSetup-Item,

MulticastF1UContext-Setup-Item,

MulticastF1UContext-FailedToBeSetup-Item,

MulticastMBSSessionList,

MulticastMRBs-ToBeSetup-Item,

MulticastMRBs-Setup-Item,

MulticastMRBs-FailedToBeSetup-Item,

MulticastMRBs-ToBeSetupMod-Item,

MulticastMRBs-ToBeModified-Item,

MulticastMRBs-ToBeReleased-Item,

MulticastMRBs-SetupMod-Item,

MulticastMRBs-FailedToBeSetupMod-Item,

MulticastMRBs-Modified-Item,

MulticastMRBs-FailedToBeModified-Item,

BroadcastAreaScope,

NotificationControl,

NRCGI,

NRPCI,

UEContextNotRetrievable,

Potential-SpCell-Item,

RAT-FrequencyPriorityInformation,

RequestedSRSTransmissionCharacteristics,

ResourceCoordinationTransferContainer,

RRCContainer,

RRCContainer-RRCSetupComplete,

RRCReconfigurationCompleteIndicator,

SCellIndex,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

SCell-FailedtoSetup-Item,

SCell-FailedtoSetupMod-Item,

ServCellIndex,

Served-Cell-Information,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

ServingCellMO,

SNSSAI,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

SRBs-Modified-Item,

SRBs-Setup-Item,

SRBs-SetupMod-Item,

TimeToWait,

TransactionID,

TransmissionActionIndicator,

UE-associatedLogicalF1-ConnectionItem,

UEIdentity-List-For-Paging-Item,

DUtoCURRCContainer,

PagingCell-Item,

SItype-List,

UEIdentityIndexValue,

GNB-CU-TNL-Association-Setup-Item,

GNB-CU-TNL-Association-Failed-To-Setup-Item,

GNB-CU-TNL-Association-To-Add-Item,

GNB-CU-TNL-Association-To-Remove-Item,

GNB-CU-TNL-Association-To-Update-Item,

MaskedIMEISV,

PagingDRX,

PagingPriority,

PagingIdentity,

Cells-to-be-Barred-Item,

PWSSystemInformation,

Broadcast-To-Be-Cancelled-Item,

Cells-Broadcast-Cancelled-Item,

NR-CGI-List-For-Restart-Item,

PWS-Failed-NR-CGI-Item,

RepetitionPeriod,

NumberofBroadcastRequest,

Cells-To-Be-Broadcast-Item,

Cells-Broadcast-Completed-Item,

Cancel-all-Warning-Messages-Indicator,

EUTRA-NR-CellResourceCoordinationReq-Container,

EUTRA-NR-CellResourceCoordinationReqAck-Container,

RequestType,

PLMN-Identity,

RLCFailureIndication,

UplinkTxDirectCurrentListInformation,

SULAccessIndication,

Protected-EUTRA-Resources-Item,

GNB-DUConfigurationQuery,

BitRate,

RRC-Version,

GNBDUOverloadInformation,

RRCDeliveryStatusRequest,

NeedforGap,

RRCDeliveryStatus,

ResourceCoordinationTransferInformation,

Dedicated-SIDelivery-NeededUE-Item,

Associated-SCell-Item,

IgnoreResourceCoordinationContainer,

PagingOrigin,

UAC-Assistance-Info,

RANUEID,

GNB-DU-TNL-Association-To-Remove-Item,

NotificationInformation,

TraceActivation,

TraceID,

Neighbour-Cell-Information-Item,

SymbolAllocInSlot,

NumDLULSymbols,

AdditionalRRMPriorityIndex,

DUCURadioInformationType,

CUDURadioInformationType,

Transport-Layer-Address-Info,

BHChannels-ToBeSetup-Item,

BHChannels-Setup-Item,

BHChannels-FailedToBeSetup-Item,

BHChannels-ToBeModified-Item,

BHChannels-ToBeReleased-Item,

BHChannels-ToBeSetupMod-Item,

BHChannels-FailedToBeModified-Item,

BHChannels-FailedToBeSetupMod-Item,

BHChannels-Modified-Item,

BHChannels-SetupMod-Item,

BHChannels-Required-ToBeReleased-Item,

BAPAddress,

BAPPathID,

BAPRoutingID,

BH-Routing-Information-Added-List-Item,

BH-Routing-Information-Removed-List-Item,

Child-Nodes-List,

Child-Nodes-List-Item,

Child-Node-Cells-List,

Child-Node-Cells-List-Item,

Activated-Cells-to-be-Updated-List,

Activated-Cells-to-be-Updated-List-Item,

UL-BH-Non-UP-Traffic-Mapping,

IABTNLAddressesRequested,

IABIPv6RequestType,

IAB-TNL-Addresses-To-Remove-Item,

IABTNLAddress,

IAB-Allocated-TNL-Address-Item,

IABv4AddressesRequested,

TrafficMappingInfo,

UL-UP-TNL-Information-to-Update-List-Item,

UL-UP-TNL-Address-to-Update-List-Item,

DL-UP-TNL-Address-to-Update-List-Item,

NRV2XServicesAuthorized,

LTEV2XServicesAuthorized,

NRUESidelinkAggregateMaximumBitrate,

LTEUESidelinkAggregateMaximumBitrate,

SLDRBs-SetupMod-Item,

SLDRBs-ModifiedConf-Item,

SLDRBID,

SLDRBs-FailedToBeModified-Item,

SLDRBs-FailedToBeSetup-Item,

SLDRBs-FailedToBeSetupMod-Item,

SLDRBs-Modified-Item,

SLDRBs-Required-ToBeModified-Item,

SLDRBs-Required-ToBeReleased-Item,

SLDRBs-Setup-Item,

SLDRBs-ToBeModified-Item,

SLDRBs-ToBeReleased-Item,

SLDRBs-ToBeSetup-Item,

SLDRBs-ToBeSetupMod-Item,

GNBCUMeasurementID,

GNBDUMeasurementID,

RegistrationRequest,

ReportCharacteristics,

CellToReportList,

HardwareLoadIndicator,

CellMeasurementResultList,

ReportingPeriodicity,

TNLCapacityIndicator,

RACHReportInformationList,

RLFReportInformationList,

ReportingRequestType,

TimeReferenceInformation,

ConditionalInterDUMobilityInformation,

ConditionalIntraDUMobilityInformation,

TargetCellList,

MDTPLMNList,

PrivacyIndicator,

TransportLayerAddress,

URI-address,

NID,

PosAssistance-Information,

PosBroadcast,

PositioningBroadcastCells,

RoutingID,

PosAssistanceInformationFailureList,

PosMeasurementQuantities,

PosMeasurementResultList,

PosReportCharacteristics,

TRPInformationTypeItem,

TRPInformationItem,

LMF-MeasurementID,

RAN-MeasurementID,

SDT-Termination-Request,

SRSResourceSetID,

SpatialRelationInfo,

SRSResourceTrigger,

SRSConfiguration,

TRPList,

E-CID-MeasurementQuantities,

MeasurementPeriodicity,

E-CID-MeasurementResult,

Cell-Portion-ID,

LMF-UE-MeasurementID,

RAN-UE-MeasurementID,

RelativeTime1900,

SystemFrameNumber,

SlotNumber,

AbortTransmission,

TRP-MeasurementRequestList,

MeasurementBeamInfoRequest,

E-CID-ReportCharacteristics,

Extended-GNB-CU-Name,

Extended-GNB-DU-Name,

F1CTransferPath,

SCGIndicator,

SpatialRelationPerSRSResource,

MeasurementPeriodicityExtended,

SuccessfulHOReportInformationList,

Coverage-Modification-Notification,

CCO-Assistance-Information,

CellsForSON-List,

IABCongestionIndication,

IABConditionalRRCMessageDeliveryIndication,

F1CTransferPathNRDC,

BufferSizeThresh,

IAB-TNL-Addresses-Exception,

BAP-Header-Rewriting-Added-List-Item,

Re-routingEnableIndicator,

NonF1terminatingTopologyIndicator,

EgressNonF1terminatingTopologyIndicator,

IngressNonF1terminatingTopologyIndicator,

Neighbour-Node-Cells-List,

Neighbour-Node-Cells-List-Item,

NA-Resource-Configuration-List,

NA-Resource-Configuration-Item,

Serving-Cells-List,

Serving-Cells-List-Item,

RBSetConfiguration,

PDCMeasurementPeriodicity,

PDCMeasurementQuantities,

PDCMeasurementResult,

PDCReportType,

RAN-UE-PDC-MeasID,

SCGActivationRequest,

SCGActivationStatus,

TRP-MeasurementUpdateList,

PRSTRPList,

PRSTransmissionTRPList,

ResponseTime,

TRP-PRS-Info-List,

PRS-Measurement-Info-List,

PRSConfigRequestType,

MeasurementCharacteristicsRequestIndicator,

MeasurementTimeOccasion,

UEReportingInformation,

PosConextRevIndication,

NRRedCapUEIndication,

NRPagingeDRXInformation,

NRPagingeDRXInformationforRRCINACTIVE,

QoEInformation,

CG-SDTQueryIndication,

CG-SDTKeptIndicator,

CG-SDTSessionInfo,

SDTInformation,

FiveG-ProSeAuthorized,

UuRLCChannelToBeSetupList,

UuRLCChannelToBeModifiedList,

UuRLCChannelToBeReleasedList,

UuRLCChannelSetupList,

UuRLCChannelFailedToBeSetupList,

UuRLCChannelModifiedList,

UuRLCChannelFailedToBeModifiedList,

UuRLCChannelRequiredToBeModifiedList,

UuRLCChannelRequiredToBeReleasedList,

PC5RLCChannelToBeSetupList,

PC5RLCChannelToBeModifiedList,

PC5RLCChannelToBeReleasedList,

PC5RLCChannelSetupList,

PC5RLCChannelFailedToBeSetupList,

PC5RLCChannelFailedToBeModifiedList,

PC5RLCChannelRequiredToBeModifiedList,

PC5RLCChannelRequiredToBeReleasedList,

PC5RLCChannelModifiedList,

RemoteUELocalID,

PathSwitchConfiguration,

SidelinkRelayConfiguration,

PagingCause,

PEIPSAssistanceInfo,

UEPagingCapability,

GNBDUUESliceMaximumBitRateList,

MDTPollutedMeasurementIndicator,

UE-MulticastMRBs-ConfirmedToBeModified-Item,

UE-MulticastMRBs-RequiredToBeModified-Item,

UE-MulticastMRBs-RequiredToBeReleased-Item,

UE-MulticastMRBs-Setup-Item,

UE-MulticastMRBs-Setupnew-Item,

UE-MulticastMRBs-ToBeReleased-Item,

UE-MulticastMRBs-ToBeSetup-Item,

UE-MulticastMRBs-ToBeSetup-atModify-Item,

PosMeasurementAmount,

BAP-Header-Rewriting-Removed-List-Item,

SLDRXCycleList,

MDTPLMNModificationList,

ActivationRequestType,

PosMeasGapPreConfigList,

PosMeasurementPeriodicityNR-AoA,

SRSPosRRCInactiveConfig,

SDTBearerConfigurationQueryIndication,

SDTBearerConfigurationInfo,

ServingCellMO-List-Item,

ServingCellMO-encoded-in-CGC-List,

PosSItypeList,

DAPS-HO-Status,

UuRLCChannelID,

UplinkTxDirectCurrentTwoCarrierListInfo,

SRSPosRRCInactiveQueryIndication,

MC-PagingCell-Item,

UlTxDirectCurrentMoreCarrierInformation,

CPACMCGInformation,

ExtendedUEIdentityIndexValue,

HashedUEIdentityIndexValue,

DedicatedSIDeliveryIndication,

Configured-BWP-List

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-BroadcastMRBs-FailedToBeModified-List,

id-BroadcastMRBs-FailedToBeModified-Item,

id-BroadcastMRBs-FailedToBeSetup-List,

id-BroadcastMRBs-FailedToBeSetup-Item,

id-BroadcastMRBs-FailedToBeSetupMod-List,

id-BroadcastMRBs-FailedToBeSetupMod-Item,

id-BroadcastMRBs-Modified-List,

id-BroadcastMRBs-Modified-Item,

id-BroadcastMRBs-Setup-List,

id-BroadcastMRBs-Setup-Item,

id-BroadcastMRBs-SetupMod-List,

id-BroadcastMRBs-SetupMod-Item,

id-BroadcastMRBs-ToBeModified-List,

id-BroadcastMRBs-ToBeModified-Item,

id-BroadcastMRBs-ToBeReleased-List,

id-BroadcastMRBs-ToBeReleased-Item,

id-BroadcastMRBs-ToBeSetup-List,

id-BroadcastMRBs-ToBeSetup-Item,

id-BroadcastMRBs-ToBeSetupMod-List,

id-BroadcastMRBs-ToBeSetupMod-Item,

id-Candidate-SpCell-Item,

id-Candidate-SpCell-List,

id-Cause,

id-Cancel-all-Warning-Messages-Indicator,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-Status-Item,

id-Cells-Status-List,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-ConfirmedUEID,

id-CriticalityDiagnostics,

id-C-RNTI,

id-CUtoDURRCInformation,

id-DRB-Activity-Item,

id-DRB-Activity-List,

id-DRBs-FailedToBeModified-Item,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-Item,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-Item,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-Item,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-Item,

id-DRBs-Modified-List,

id-DRB-Notify-Item,

id-DRB-Notify-List,

id-DRBs-Required-ToBeModified-Item,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-Item,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-Item,

id-DRBs-Setup-List,

id-DRBs-SetupMod-Item,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-Item,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-Item,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-Item,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-Item,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-ExecuteDuplication,

id-FullConfiguration,

id-gNB-CU-MBS-F1AP-ID,

id-gNB-CU-UE-F1AP-ID,

id-gNB-DU-MBS-F1AP-ID,

id-gNB-DU-UE-F1AP-ID,

id-gNB-DU-ID,

id-GNB-DU-Served-Cells-Item,

id-gNB-DU-Served-Cells-List,

id-gNB-CU-Name,

id-gNB-DU-Name,

id-Extended-GNB-CU-Name,

id-Extended-GNB-DU-Name,

id-InactivityMonitoringRequest,

id-InactivityMonitoringResponse,

id-MBS-Area-Session-ID,

id-MBS-CUtoDURRCInformation,

id-MBS-Session-ID,

id-MBS-ServiceArea,

id-MBSMulticastF1UContextDescriptor,

id-MC-PagingCell-Item,

id-MC-PagingCell-List,

id-MulticastF1UContextReferenceCU,

id-MulticastMBSSessionSetupList,

id-MulticastMBSSessionRemoveList,

id-MulticastMRBs-FailedToBeModified-List,

id-MulticastMRBs-FailedToBeModified-Item,

id-MulticastMRBs-FailedToBeSetup-List,

id-MulticastMRBs-FailedToBeSetup-Item,

id-MulticastMRBs-FailedToBeSetupMod-List,

id-MulticastMRBs-FailedToBeSetupMod-Item,

id-MulticastMRBs-Modified-List,

id-MulticastMRBs-Modified-Item,

id-MulticastMRBs-Setup-List,

id-MulticastMRBs-Setup-Item,

id-MulticastMRBs-SetupMod-List,

id-MulticastMRBs-SetupMod-Item,

id-MulticastMRBs-ToBeModified-List,

id-MulticastMRBs-ToBeModified-Item,

id-MulticastMRBs-ToBeReleased-List,

id-MulticastMRBs-ToBeReleased-Item,

id-MulticastMRBs-ToBeSetup-List,

id-MulticastMRBs-ToBeSetup-Item,

id-MulticastMRBs-ToBeSetupMod-List,

id-MulticastMRBs-ToBeSetupMod-Item,

id-MulticastF1UContext-ToBeSetup-List,

id-MulticastF1UContext-ToBeSetup-Item,

id-MulticastF1UContext-Setup-List,

id-MulticastF1UContext-Setup-Item,

id-MulticastF1UContext-FailedToBeSetup-List,

id-MulticastF1UContext-FailedToBeSetup-Item,

id-BroadcastAreaScope,

id-new-gNB-CU-UE-F1AP-ID,

id-new-gNB-DU-UE-F1AP-ID,

id-oldgNB-DU-UE-F1AP-ID,

id-PLMNAssistanceInfoForNetShar,

id-Potential-SpCell-Item,

id-Potential-SpCell-List,

id-RAT-FrequencyPriorityInformation,

id-RedirectedRRCmessage,

id-ResetType,

id-RequestedSRSTransmissionCharacteristics,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-RRCContainer-RRCSetupComplete,

id-RRCReconfigurationCompleteIndicator,

id-SCell-FailedtoSetup-List,

id-SCell-FailedtoSetup-Item,

id-SCell-FailedtoSetupMod-List,

id-SCell-FailedtoSetupMod-Item,

id-SCell-ToBeRemoved-Item,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-Item,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-Item,

id-SCell-ToBeSetupMod-List,

id-SDT-Termination-Request,

id-SelectedPLMNID,

id-Served-Cells-To-Add-Item,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-Item,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-Item,

id-Served-Cells-To-Modify-List,

id-ServCellIndex,

id-ServingCellMO,

id-SNSSAI,

id-SpCell-ID,

id-SpCellULConfigured,

id-SRBID,

id-SRBs-FailedToBeSetup-Item,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-Item,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-Item,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-ToBeReleased-Item,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-Item,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-Item,

id-SRBs-ToBeSetupMod-List,

id-SRBs-Modified-Item,

id-SRBs-Modified-List,

id-SRBs-Setup-Item,

id-SRBs-Setup-List,

id-SRBs-SetupMod-Item,

id-SRBs-SetupMod-List,

id-TimeToWait,

id-TransactionID,

id-TransmissionActionIndicator,

id-UEContextNotRetrievable,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

id-UEIdentity-List-For-Paging-List,

id-UEIdentity-List-For-Paging-Item,

id-UE-MulticastMRBs-ConfirmedToBeModified-List,

id-UE-MulticastMRBs-ConfirmedToBeModified-Item,

id-UE-MulticastMRBs-RequiredToBeModified-List,

id-UE-MulticastMRBs-RequiredToBeModified-Item,

id-UE-MulticastMRBs-RequiredToBeReleased-List,

id-UE-MulticastMRBs-RequiredToBeReleased-Item,

id-UE-MulticastMRBs-Setup-List,

id-UE-MulticastMRBs-Setup-Item,

id-UE-MulticastMRBs-Setupnew-List,

id-UE-MulticastMRBs-Setupnew-Item,

id-UE-MulticastMRBs-ToBeReleased-List,

id-UE-MulticastMRBs-ToBeReleased-Item,

id-UE-MulticastMRBs-ToBeSetup-atModify-List,

id-UE-MulticastMRBs-ToBeSetup-atModify-Item,

id-UE-MulticastMRBs-ToBeSetup-List,

id-UE-MulticastMRBs-ToBeSetup-Item,

id-DUtoCURRCContainer,

id-NRCGI,

id-PagingCell-Item,

id-PagingCell-List,

id-PagingDRX,

id-PagingPriority,

id-SItype-List,

id-UEIdentityIndexValue,

id-GNB-CU-TNL-Association-Setup-List,

id-GNB-CU-TNL-Association-Setup-Item,

id-GNB-CU-TNL-Association-Failed-To-Setup-List,

id-GNB-CU-TNL-Association-Failed-To-Setup-Item,

id-GNB-CU-TNL-Association-To-Add-Item,

id-GNB-CU-TNL-Association-To-Add-List,

id-GNB-CU-TNL-Association-To-Remove-Item,

id-GNB-CU-TNL-Association-To-Remove-List,

id-GNB-CU-TNL-Association-To-Update-Item,

id-GNB-CU-TNL-Association-To-Update-List,

id-MaskedIMEISV,

id-PagingIdentity,

id-Cells-to-be-Barred-List,

id-Cells-to-be-Barred-Item,

id-PWSSystemInformation,

id-RepetitionPeriod,

id-NumberofBroadcastRequest,

id-Cells-To-Be-Broadcast-List,

id-Cells-To-Be-Broadcast-Item,

id-Cells-Broadcast-Completed-List,

id-Cells-Broadcast-Completed-Item,

id-Broadcast-To-Be-Cancelled-List,

id-Broadcast-To-Be-Cancelled-Item,

id-Cells-Broadcast-Cancelled-List,

id-Cells-Broadcast-Cancelled-Item,

id-NR-CGI-List-For-Restart-List,

id-NR-CGI-List-For-Restart-Item,

id-PWS-Failed-NR-CGI-List,

id-PWS-Failed-NR-CGI-Item,

id-EUTRA-NR-CellResourceCoordinationReq-Container,

id-EUTRA-NR-CellResourceCoordinationReqAck-Container,

id-Protected-EUTRA-Resources-List,

id-RequestType,

id-ServingPLMN,

id-DRXConfigurationIndicator,

id-RLCFailureIndication,

id-UplinkTxDirectCurrentListInformation,

id-SULAccessIndication,

id-Protected-EUTRA-Resources-Item,

id-GNB-DUConfigurationQuery,

id-GNB-DU-UE-AMBR-UL,

id-GNB-CU-RRC-Version,

id-GNB-DU-RRC-Version,

id-GNBDUOverloadInformation,

id-NeedforGap,

id-RRCDeliveryStatusRequest,

id-RRCDeliveryStatus,

id-Dedicated-SIDelivery-NeededUE-List,

id-Dedicated-SIDelivery-NeededUE-Item,

id-ResourceCoordinationTransferInformation,

id-Associated-SCell-List,

id-Associated-SCell-Item,

id-IgnoreResourceCoordinationContainer,

id-UAC-Assistance-Info,

id-RANUEID,

id-PagingOrigin,

id-GNB-DU-TNL-Association-To-Remove-Item,

id-GNB-DU-TNL-Association-To-Remove-List,

id-NotificationInformation,

id-TraceActivation,

id-TraceID,

id-Neighbour-Cell-Information-List,

id-Neighbour-Cell-Information-Item,

id-SymbolAllocInSlot,

id-NumDLULSymbols,

id-AdditionalRRMPriorityIndex,

id-DUCURadioInformationType,

id-CUDURadioInformationType,

id-LowerLayerPresenceStatusChange,

id-Transport-Layer-Address-Info,

id-BHChannels-ToBeSetup-List,

id-BHChannels-ToBeSetup-Item,

id-BHChannels-Setup-List,

id-BHChannels-Setup-Item,

id-BHChannels-ToBeModified-Item,

id-BHChannels-ToBeModified-List,

id-BHChannels-ToBeReleased-Item,

id-BHChannels-ToBeReleased-List,

id-BHChannels-ToBeSetupMod-Item,

id-BHChannels-ToBeSetupMod-List,

id-BHChannels-FailedToBeSetup-Item,

id-BHChannels-FailedToBeSetup-List,

id-BHChannels-FailedToBeModified-Item,

id-BHChannels-FailedToBeModified-List,

id-BHChannels-FailedToBeSetupMod-Item,

id-BHChannels-FailedToBeSetupMod-List,

id-BHChannels-Modified-Item,

id-BHChannels-Modified-List,

id-BHChannels-SetupMod-Item,

id-BHChannels-SetupMod-List,

id-BHChannels-Required-ToBeReleased-Item,

id-BHChannels-Required-ToBeReleased-List,

id-BAPAddress,

id-ConfiguredBAPAddress,

id-BH-Routing-Information-Added-List,

id-BH-Routing-Information-Added-List-Item,

id-BH-Routing-Information-Removed-List,

id-BH-Routing-Information-Removed-List-Item,

id-UL-BH-Non-UP-Traffic-Mapping,

id-Child-Nodes-List,

id-Activated-Cells-to-be-Updated-List,

id-IABIPv6RequestType,

id-IAB-TNL-Addresses-To-Remove-List,

id-IAB-TNL-Addresses-To-Remove-Item,

id-IAB-Allocated-TNL-Address-List,

id-IAB-Allocated-TNL-Address-Item,

id-IABv4AddressesRequested,

id-TrafficMappingInformation,

id-UL-UP-TNL-Information-to-Update-List,

id-UL-UP-TNL-Information-to-Update-List-Item,

id-UL-UP-TNL-Address-to-Update-List,

id-UL-UP-TNL-Address-to-Update-List-Item,

id-DL-UP-TNL-Address-to-Update-List,

id-DL-UP-TNL-Address-to-Update-List-Item,

id-NRV2XServicesAuthorized,

id-LTEV2XServicesAuthorized,

id-NRUESidelinkAggregateMaximumBitrate,

id-LTEUESidelinkAggregateMaximumBitrate,

id-PC5LinkAMBR,

id-SLDRBs-FailedToBeModified-Item,

id-SLDRBs-FailedToBeModified-List,

id-SLDRBs-FailedToBeSetup-Item,

id-SLDRBs-FailedToBeSetup-List,

id-SLDRBs-Modified-Item,

id-SLDRBs-Modified-List,

id-SLDRBs-Required-ToBeModified-Item,

id-SLDRBs-Required-ToBeModified-List,

id-SLDRBs-Required-ToBeReleased-Item,

id-SLDRBs-Required-ToBeReleased-List,

id-SLDRBs-Setup-Item,

id-SLDRBs-Setup-List,

id-SLDRBs-ToBeModified-Item,

id-SLDRBs-ToBeModified-List,

id-SLDRBs-ToBeReleased-Item,

id-SLDRBs-ToBeReleased-List,

id-SLDRBs-ToBeSetup-Item,

id-SLDRBs-ToBeSetup-List,

id-SLDRBs-ToBeSetupMod-Item,

id-SLDRBs-ToBeSetupMod-List,

id-SLDRBs-SetupMod-List,

id-SLDRBs-FailedToBeSetupMod-List,

id-SLDRBs-SetupMod-Item,

id-SLDRBs-FailedToBeSetupMod-Item,

id-SLDRBs-ModifiedConf-List,

id-SLDRBs-ModifiedConf-Item,

id-gNBCUMeasurementID,

id-gNBDUMeasurementID,

id-RegistrationRequest,

id-ReportCharacteristics,

id-CellToReportList,

id-CellMeasurementResultList,

id-HardwareLoadIndicator,

id-ReportingPeriodicity,

id-TNLCapacityIndicator,

id-RACHReportInformationList,

id-RLFReportInformationList,

id-ReportingRequestType,

id-TimeReferenceInformation,

id-ConditionalInterDUMobilityInformation,

id-ConditionalIntraDUMobilityInformation,

id-targetCellsToCancel,

id-requestedTargetCellGlobalID,

id-TraceCollectionEntityIPAddress,

id-ManagementBasedMDTPLMNList,

id-PrivacyIndicator,

id-TraceCollectionEntityURI,

id-ServingNID,

id-PosAssistance-Information,

id-PosBroadcast,

id-PositioningBroadcastCells,

id-RoutingID,

id-PosAssistanceInformationFailureList,

id-PosMeasurementQuantities,

id-PosMeasurementResultList,

id-PosMeasurementPeriodicity,

id-PosReportCharacteristics,

id-TRPInformationTypeListTRPReq,

id-TRPInformationTypeItem,

id-TRPInformationListTRPResp,

id-TRPInformationItem,

id-LMF-MeasurementID,

id-RAN-MeasurementID,

id-SRSType,

id-ActivationTime,

id-AbortTransmission,

id-SRSConfiguration,

id-TRPList,

id-E-CID-MeasurementQuantities,

id-E-CID-MeasurementPeriodicity,

id-E-CID-MeasurementResult,

id-Cell-Portion-ID,

id-LMF-UE-MeasurementID,

id-RAN-UE-MeasurementID,

id-SFNInitialisationTime,

id-SystemFrameNumber,

id-SlotNumber,

id-TRP-MeasurementRequestList,

id-MeasurementBeamInfoRequest,

id-E-CID-ReportCharacteristics,

id-F1CTransferPath,

id-SCGIndicator,

id-SRSSpatialRelationPerSRSResource,

id-PosMeasurementPeriodicityExtended,

id-SuccessfulHOReportInformationList,

id-Coverage-Modification-Notification,

id-CCO-Assistance-Information,

id-CellsForSON-List,

id-IABCongestionIndication,

id-IABConditionalRRCMessageDeliveryIndication,

id-F1CTransferPathNRDC,

id-BufferSizeThresh,

id-IAB-TNL-Addresses-Exception,

id-BAP-Header-Rewriting-Added-List,

id-BAP-Header-Rewriting-Added-List-Item,

id-Re-routingEnableIndicator,

id-NonF1terminatingTopologyIndicator,

id-EgressNonF1terminatingTopologyIndicator,

id-IngressNonF1terminatingTopologyIndicator,

id-Neighbour-Node-Cells-List,

id-Serving-Cells-List,

id-MDTPollutedMeasurementIndicator,

id-PDCMeasurementPeriodicity,

id-PDCMeasurementQuantities,

id-PDCMeasurementResult,

id-PDCReportType,

id-RAN-UE-PDC-MeasID,

id-SCGActivationRequest,

id-SCGActivationStatus,

id-TRP-MeasurementUpdateList,

id-PRSTRPList,

id-PRSTransmissionTRPList,

id-ResponseTime,

id-TRP-PRS-Info-List,

id-PRS-Measurement-Info-List,

id-PRSConfigRequestType,

id-MeasurementCharacteristicsRequestIndicator,

id-MeasurementTimeOccasion,

id-UEReportingInformation,

id-PosConextRevIndication,

id-NRRedCapUEIndication,

id-RANUEPagingDRX,

id-CNUEPagingDRX,

id-NRPagingeDRXInformation,

id-NRPagingeDRXInformationforRRCINACTIVE,

id-QoEInformation,

id-CG-SDTQueryIndication,

id-CG-SDTKeptIndicator,

id-CG-SDTSessionInfoOld,

id-SDTInformation,

id-FiveG-ProSeAuthorized,

id-FiveG-ProSePC5LinkAMBR,

id-FiveG-ProSeUEPC5AggregateMaximumBitrate,

id-UuRLCChannelToBeSetupList,

id-UuRLCChannelToBeModifiedList,

id-UuRLCChannelToBeReleasedList,

id-UuRLCChannelSetupList,

id-UuRLCChannelFailedToBeSetupList,

id-UuRLCChannelModifiedList,

id-UuRLCChannelFailedToBeModifiedList,

id-UuRLCChannelRequiredToBeModifiedList,

id-UuRLCChannelRequiredToBeReleasedList,

id-PC5RLCChannelToBeSetupList,

id-PC5RLCChannelToBeModifiedList,

id-PC5RLCChannelToBeReleasedList,

id-PC5RLCChannelSetupList,

id-PC5RLCChannelFailedToBeSetupList,

id-PC5RLCChannelModifiedList,

id-PC5RLCChannelFailedToBeModifiedList,

id-PC5RLCChannelRequiredToBeModifiedList,

id-PC5RLCChannelRequiredToBeReleasedList,

id-SidelinkRelayConfiguration,

id-UpdatedRemoteUELocalID,

id-PathSwitchConfiguration,

id-PagingCause,

id-PEIPSAssistanceInfo,

id-UEPagingCapability,

id-GNBDUUESliceMaximumBitRateList,

id-PosMeasurementAmount,

id-BAP-Header-Rewriting-Removed-List,

id-BAP-Header-Rewriting-Removed-List-Item,

id-SLDRXCycleList,

id-ManagementBasedMDTPLMNModificationList,

id-ActivationRequestType,

id-PosMeasGapPreConfigList,

id-PosMeasurementPeriodicityNR-AoA,

id-SRSPosRRCInactiveConfig,

id-SDTBearerConfigurationQueryIndication,

id-SDTBearerConfigurationInfo,

id-ServingCellMO-List,

id-ServingCellMO-List-Item,

id-ServingCellMO-encoded-in-CGC-List,

id-PosSItypeList,

id-DAPS-HO-Status,

id-SRBMappingInfo,

id-UplinkTxDirectCurrentTwoCarrierListInfo,

id-SRSPosRRCInactiveQueryIndication,

id-UlTxDirectCurrentMoreCarrierInformation,

id-CPACMCGInformation,

id-ExtendedUEIdentityIndexValue,

id-HashedUEIdentityIndexValue,

id-DedicatedSIDeliveryIndication,

id-Configured-BWP-List,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofBHRLCChannels,

maxnoofRoutingEntries,

maxnoofChildIABNodes,

maxnoofServedCellsIAB,

maxnoofTLAsIAB,

maxnoofULUPTNLInformationforIAB,

maxnoofUPTNLAddresses,

maxnoofSLDRBs,

maxnoofTRPInfoTypes,

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofNeighbourNodeCellsIAB,

maxnoofMRBsforUE,

maxnoofServingCellMOs

FROM F1AP-Constants;

-- **************************************************************

--

#### -- RESET ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Reset

--

-- **************************************************************

Reset ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetIEs} },

...

}

ResetIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-ResetType					CRITICALITY reject	TYPE ResetType					PRESENCE mandatory	},

...

}

ResetType ::= CHOICE {

f1-Interface					ResetAll,

partOfF1-Interface				UE-associatedLogicalF1-ConnectionListRes,

choice-extension				ProtocolIE-SingleContainer { { ResetType-ExtIEs} }

}

ResetType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ResetAll ::= ENUMERATED {

reset-all,

...

}

UE-associatedLogicalF1-ConnectionListRes ::= SEQUENCE (SIZE(1.. maxnoofIndividualF1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalF1-ConnectionItemRes } }

UE-associatedLogicalF1-ConnectionItemRes F1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalF1-ConnectionItem	CRITICALITY reject	TYPE UE-associatedLogicalF1-ConnectionItem	PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- Reset Acknowledge

--

-- **************************************************************

ResetAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetAcknowledgeIEs} },

...

}

ResetAcknowledgeIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID									CRITICALITY reject	TYPE TransactionID													PRESENCE mandatory	}|

{ ID id-UE-associatedLogicalF1-ConnectionListResAck		CRITICALITY ignore	TYPE UE-associatedLogicalF1-ConnectionListResAck			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

UE-associatedLogicalF1-ConnectionListResAck ::= SEQUENCE (SIZE(1.. maxnoofIndividualF1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalF1-ConnectionItemResAck } }

UE-associatedLogicalF1-ConnectionItemResAck 	F1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalF1-ConnectionItem	 CRITICALITY ignore 	TYPE UE-associatedLogicalF1-ConnectionItem  	PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- ERROR INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Error Indication

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ErrorIndicationIEs}},

...

}

ErrorIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory}|

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY ignore	TYPE GNB-CU-UE-F1AP-ID			PRESENCE optional	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID			PRESENCE optional	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- F1 SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- F1 Setup Request

--

-- **************************************************************

F1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupRequestIEs} },

...

}

F1SetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-DU-ID						CRITICALITY reject	TYPE GNB-DU-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-Name						CRITICALITY ignore	TYPE GNB-DU-Name						PRESENCE optional	}|

{ ID id-gNB-DU-Served-Cells-List		CRITICALITY reject	TYPE GNB-DU-Served-Cells-List			PRESENCE optional	}|

{ ID id-GNB-DU-RRC-Version				CRITICALITY reject	TYPE RRC-Version						PRESENCE mandatory	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info		PRESENCE optional	}|

{ ID id-BAPAddress						CRITICALITY ignore	TYPE BAPAddress							PRESENCE optional	}|

{ ID id-Extended-GNB-DU-Name			CRITICALITY ignore	TYPE Extended-GNB-DU-Name				PRESENCE optional	},

...

}

GNB-DU-Served-Cells-List 	::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { GNB-DU-Served-Cells-ItemIEs } }

GNB-DU-Served-Cells-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-GNB-DU-Served-Cells-Item		CRITICALITY reject	TYPE		GNB-DU-Served-Cells-Item	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- F1 Setup Response

--

-- **************************************************************

F1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupResponseIEs} },

...

}

F1SetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-gNB-CU-Name						CRITICALITY ignore	TYPE GNB-CU-Name						PRESENCE optional	}|

{ ID id-Cells-to-be-Activated-List		CRITICALITY reject	TYPE Cells-to-be-Activated-List			PRESENCE optional	}|

{ ID id-GNB-CU-RRC-Version				CRITICALITY reject	TYPE RRC-Version						PRESENCE mandatory	}|

{ ID id-Transport-Layer-Address-Info	CRITICALITY ignore	TYPE Transport-Layer-Address-Info		PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping	CRITICALITY reject	TYPE UL-BH-Non-UP-Traffic-Mapping		PRESENCE optional	}|

{ ID id-BAPAddress						CRITICALITY ignore	TYPE BAPAddress							PRESENCE optional	}|

{ ID id-Extended-GNB-CU-Name			CRITICALITY ignore	TYPE Extended-GNB-CU-Name				PRESENCE optional	},

...

}

Cells-to-be-Activated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-Activated-List-ItemIEs } }

Cells-to-be-Activated-List-ItemIEs	F1AP-PROTOCOL-IES::= {

{ ID id-Cells-to-be-Activated-List-Item				CRITICALITY reject	TYPE Cells-to-be-Activated-List-Item						PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- F1 Setup Failure

--

-- **************************************************************

F1SetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupFailureIEs} },

...

}

F1SetupFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-DU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-DU CONFIGURATION UPDATE

--

-- **************************************************************

GNBDUConfigurationUpdate::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateIEs} },

...

}

GNBDUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID												PRESENCE mandatory	}|

{ ID id-Served-Cells-To-Add-List				CRITICALITY reject	TYPE Served-Cells-To-Add-List								PRESENCE optional	}|

{ ID id-Served-Cells-To-Modify-List				CRITICALITY reject	TYPE Served-Cells-To-Modify-List							PRESENCE optional	}|

{ ID id-Served-Cells-To-Delete-List				CRITICALITY reject	TYPE Served-Cells-To-Delete-List							PRESENCE optional	}|

{ ID id-Cells-Status-List						CRITICALITY reject	TYPE Cells-Status-List											PRESENCE optional	}|

{ ID id-Dedicated-SIDelivery-NeededUE-List		CRITICALITY ignore	TYPE Dedicated-SIDelivery-NeededUE-List					PRESENCE optional	}|

{ ID id-gNB-DU-ID								CRITICALITY reject	TYPE GNB-DU-ID													PRESENCE optional	}|

{ ID id-GNB-DU-TNL-Association-To-Remove-List	CRITICALITY reject	TYPE GNB-DU-TNL-Association-To-Remove-List				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE Transport-Layer-Address-Info							PRESENCE optional	}|

{ ID id-Coverage-Modification-Notification		CRITICALITY ignore	TYPE Coverage-Modification-Notification					PRESENCE optional	}|

{ ID id-gNB-DU-Name								CRITICALITY ignore	TYPE GNB-DU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-DU-Name					CRITICALITY ignore	TYPE Extended-GNB-DU-Name										PRESENCE optional	},

...

}

Served-Cells-To-Add-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Add-ItemIEs } }

Served-Cells-To-Modify-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Modify-ItemIEs } }

Served-Cells-To-Delete-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Delete-ItemIEs } }

Cells-Status-List	::= SEQUENCE (SIZE(0.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Status-ItemIEs } }

Dedicated-SIDelivery-NeededUE-List::= SEQUENCE (SIZE(1.. maxnoofUEIDs))	OF ProtocolIE-SingleContainer { { Dedicated-SIDelivery-NeededUE-ItemIEs } }

GNB-DU-TNL-Association-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-DU-TNL-Association-To-Remove-ItemIEs } }

Served-Cells-To-Add-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Add-Item		CRITICALITY reject	TYPE	Served-Cells-To-Add-Item				PRESENCE mandatory	},

...

}

Served-Cells-To-Modify-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Modify-Item			CRITICALITY reject	TYPE		Served-Cells-To-Modify-Item							PRESENCE mandatory	},

...

}

Served-Cells-To-Delete-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Delete-Item				CRITICALITY reject	TYPE		Served-Cells-To-Delete-Item					PRESENCE mandatory	},

...

}

Cells-Status-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Status-Item				CRITICALITY reject	TYPE		Cells-Status-Item					PRESENCE mandatory	},

...

}

Dedicated-SIDelivery-NeededUE-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Dedicated-SIDelivery-NeededUE-Item		CRITICALITY ignore	TYPE	Dedicated-SIDelivery-NeededUE-Item				PRESENCE mandatory	},

...

}

GNB-DU-TNL-Association-To-Remove-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-DU-TNL-Association-To-Remove-Item		CRITICALITY reject	TYPE	 GNB-DU-TNL-Association-To-Remove-Item			PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

GNBDUConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateAcknowledgeIEs} },

...

}

GNBDUConfigurationUpdateAcknowledgeIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List			CRITICALITY reject	TYPE Cells-to-be-Activated-List				PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List		CRITICALITY reject	TYPE Cells-to-be-Deactivated-List			PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info		CRITICALITY ignore	TYPE Transport-Layer-Address-Info			PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping		CRITICALITY reject	TYPE UL-BH-Non-UP-Traffic-Mapping			PRESENCE optional	}|

{ ID id-BAPAddress							CRITICALITY ignore  TYPE BAPAddress									PRESENCE optional }|

{ ID id-CellsForSON-List					CRITICALITY ignore  TYPE CellsForSON-List							PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- GNB-DU CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

GNBDUConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateFailureIEs} },

...

}

GNBDUConfigurationUpdateFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-CU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE

--

-- **************************************************************

GNBCUConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateIEs} },

...

}

GNBCUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE	TransactionID											PRESENCE mandatory	}|

{ ID id-Cells-to-be-Activated-List				CRITICALITY reject	TYPE	Cells-to-be-Activated-List							PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List			CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List						PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Add-List		CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Add-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Remove-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Remove-List			PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-To-Update-List	CRITICALITY ignore	TYPE	GNB-CU-TNL-Association-To-Update-List			PRESENCE optional	}|

{ ID id-Cells-to-be-Barred-List					CRITICALITY ignore	TYPE	Cells-to-be-Barred-List								PRESENCE optional	}|

{ ID id-Protected-EUTRA-Resources-List			CRITICALITY reject	TYPE	Protected-EUTRA-Resources-List					PRESENCE optional	}|

{ ID id-Neighbour-Cell-Information-List			CRITICALITY ignore	TYPE	Neighbour-Cell-Information-List					PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info			CRITICALITY ignore	TYPE	Transport-Layer-Address-Info						PRESENCE optional	}|

{ ID id-UL-BH-Non-UP-Traffic-Mapping			CRITICALITY reject	TYPE	UL-BH-Non-UP-Traffic-Mapping						PRESENCE optional	}|

{ ID id-BAPAddress								CRITICALITY ignore  TYPE 	BAPAddress												PRESENCE optional }|

{ ID id-CCO-Assistance-Information				CRITICALITY ignore	TYPE 	CCO-Assistance-Information							PRESENCE optional	}|

{ ID id-CellsForSON-List						CRITICALITY ignore	TYPE	CellsForSON-List										PRESENCE optional }|

{ ID id-gNB-CU-Name								CRITICALITY ignore	TYPE	GNB-CU-Name												PRESENCE optional	}|

{ ID id-Extended-GNB-CU-Name					CRITICALITY ignore	TYPE	Extended-GNB-CU-Name									PRESENCE optional	},

...

}

Cells-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-Deactivated-List-ItemIEs } }

GNB-CU-TNL-Association-To-Add-List		::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Add-ItemIEs } }

GNB-CU-TNL-Association-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Remove-ItemIEs } }

GNB-CU-TNL-Association-To-Update-List	::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-To-Update-ItemIEs } }

Cells-to-be-Barred-List			::= SEQUENCE(SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { Cells-to-be-Barred-ItemIEs } }

Cells-to-be-Deactivated-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-to-be-Deactivated-List-Item						CRITICALITY reject	TYPE	Cells-to-be-Deactivated-List-Item					PRESENCE mandatory	},

...

}

GNB-CU-TNL-Association-To-Add-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-To-Add-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Add-Item			PRESENCE mandatory	},

...

}

GNB-CU-TNL-Association-To-Remove-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-To-Remove-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Remove-Item			PRESENCE mandatory	},

...

}

GNB-CU-TNL-Association-To-Update-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-To-Update-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-To-Update-Item			PRESENCE mandatory	},

...

}

Cells-to-be-Barred-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-to-be-Barred-Item		CRITICALITY ignore	TYPE	 Cells-to-be-Barred-Item				PRESENCE mandatory	},

...

}

Protected-EUTRA-Resources-List ::= SEQUENCE (SIZE(1.. maxCellineNB))	OF ProtocolIE-SingleContainer { { Protected-EUTRA-Resources-ItemIEs } }

Protected-EUTRA-Resources-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Protected-EUTRA-Resources-Item 					CRITICALITY reject 	TYPE Protected-EUTRA-Resources-Item							PRESENCE mandatory},

...

}

Neighbour-Cell-Information-List ::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Neighbour-Cell-Information-ItemIEs } }

Neighbour-Cell-Information-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Neighbour-Cell-Information-Item 					CRITICALITY ignore 	TYPE Neighbour-Cell-Information-Item							PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

GNBCUConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateAcknowledgeIEs} },

...

}

GNBCUConfigurationUpdateAcknowledgeIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID								CRITICALITY reject	TYPE TransactionID											PRESENCE mandatory	}|

{ ID id-Cells-Failed-to-be-Activated-List			CRITICALITY reject	TYPE Cells-Failed-to-be-Activated-List				PRESENCE optional}|

{ ID id-CriticalityDiagnostics							CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-Setup-List			CRITICALITY ignore	TYPE GNB-CU-TNL-Association-Setup-List				PRESENCE optional	}|

{ ID id-GNB-CU-TNL-Association-Failed-To-Setup-List	CRITICALITY ignore	TYPE GNB-CU-TNL-Association-Failed-To-Setup-List	PRESENCE optional	}|

{ ID id-Dedicated-SIDelivery-NeededUE-List				CRITICALITY ignore	TYPE Dedicated-SIDelivery-NeededUE-List				PRESENCE optional	}|

{ ID id-Transport-Layer-Address-Info				CRITICALITY ignore	TYPE Transport-Layer-Address-Info						PRESENCE optional	},

...

}

Cells-Failed-to-be-Activated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Failed-to-be-Activated-List-ItemIEs } }

GNB-CU-TNL-Association-Setup-List ::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-Setup-ItemIEs } }

GNB-CU-TNL-Association-Failed-To-Setup-List ::= SEQUENCE (SIZE(1.. maxnoofTNLAssociations))	OF ProtocolIE-SingleContainer { { GNB-CU-TNL-Association-Failed-To-Setup-ItemIEs } }

Cells-Failed-to-be-Activated-List-ItemIEs F1AP-PROTOCOL-IES		::= {

{ ID id-Cells-Failed-to-be-Activated-List-Item		CRITICALITY reject	TYPE Cells-Failed-to-be-Activated-List-Item		PRESENCE mandatory	},

...

}

GNB-CU-TNL-Association-Setup-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-Setup-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-Setup-Item			PRESENCE mandatory	},

...

}

GNB-CU-TNL-Association-Failed-To-Setup-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-GNB-CU-TNL-Association-Failed-To-Setup-Item		CRITICALITY ignore	TYPE	 GNB-CU-TNL-Association-Failed-To-Setup-Item			PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- GNB-CU CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

GNBCUConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateFailureIEs} },

...

}

GNBCUConfigurationUpdateFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- GNB-DU RESOURCE COORDINATION REQUEST

--

-- **************************************************************

GNBDUResourceCoordinationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{GNBDUResourceCoordinationRequest-IEs}},

...

}

GNBDUResourceCoordinationRequest-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID									CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory	}|

{ ID id-RequestType										CRITICALITY reject	TYPE RequestType										PRESENCE mandatory	}|

{ ID id-EUTRA-NR-CellResourceCoordinationReq-Container	CRITICALITY reject	TYPE EUTRA-NR-CellResourceCoordinationReq-Container	PRESENCE mandatory}|

{ ID id-IgnoreResourceCoordinationContainer				CRITICALITY reject	TYPE IgnoreResourceCoordinationContainer		PRESENCE optional },

...

}

-- **************************************************************

--

##### -- GNB-DU RESOURCE COORDINATION RESPONSE

--

-- **************************************************************

GNBDUResourceCoordinationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{{GNBDUResourceCoordinationResponse-IEs}},

...

}

GNBDUResourceCoordinationResponse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID										CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-EUTRA-NR-CellResourceCoordinationReqAck-Container	CRITICALITY reject	TYPE EUTRA-NR-CellResourceCoordinationReqAck-Container		PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- UE Context Setup ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT SETUP REQUEST

--

-- **************************************************************

UEContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupRequestIEs} },

...

}

UEContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional 	}|

{ ID id-SpCell-ID								CRITICALITY reject	TYPE NRCGI												PRESENCE mandatory	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE mandatory	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE mandatory}|

{ ID id-Candidate-SpCell-List					CRITICALITY ignore	TYPE Candidate-SpCell-List							PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-SCell-ToBeSetup-List					CRITICALITY ignore	TYPE SCell-ToBeSetup-List								PRESENCE optional	}|

{ ID id-SRBs-ToBeSetup-List						CRITICALITY reject	TYPE SRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-DRBs-ToBeSetup-List						CRITICALITY reject	TYPE DRBs-ToBeSetup-List								PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY ignore	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-MaskedIMEISV							CRITICALITY ignore	TYPE MaskedIMEISV										PRESENCE optional	}|

{ ID id-ServingPLMN								CRITICALITY ignore	TYPE PLMN-Identity										PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE conditional }|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest						PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-new-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE optional }|

{ ID id-RANUEID									CRITICALITY ignore	TYPE RANUEID											PRESENCE optional	}|

{ ID id-TraceActivation							CRITICALITY ignore	TYPE TraceActivation									PRESENCE optional	}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex						PRESENCE optional }|

{ ID id-BHChannels-ToBeSetup-List				CRITICALITY reject	TYPE BHChannels-ToBeSetup-List						PRESENCE optional	}|

{ ID id-ConfiguredBAPAddress					CRITICALITY reject	TYPE BAPAddress											PRESENCE optional	}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized						PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-PC5LinkAMBR								CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-SLDRBs-ToBeSetup-List					CRITICALITY reject	TYPE SLDRBs-ToBeSetup-List							PRESENCE optional	}|

{ ID id-ConditionalInterDUMobilityInformation	CRITICALITY reject	TYPE ConditionalInterDUMobilityInformation		PRESENCE optional}|

{ ID id-ManagementBasedMDTPLMNList				CRITICALITY ignore	TYPE 		MDTPLMNList									PRESENCE optional }|

{ ID id-ServingNID								CRITICALITY reject	TYPE NID												PRESENCE optional }|

{ ID id-F1CTransferPath							CRITICALITY reject	TYPE F1CTransferPath									PRESENCE optional }|

{ ID id-F1CTransferPathNRDC						CRITICALITY reject	TYPE F1CTransferPathNRDC								PRESENCE optional }|

{ ID id-MDTPollutedMeasurementIndicator			CRITICALITY ignore	TYPE MDTPollutedMeasurementIndicator				PRESENCE optional }|

{ ID id-SCGActivationRequest					CRITICALITY ignore	TYPE SCGActivationRequest								PRESENCE optional }|

{ ID id-CG-SDTSessionInfoOld					CRITICALITY ignore	TYPE CG-SDTSessionInfo									PRESENCE optional }|

{ ID id-FiveG-ProSeAuthorized					CRITICALITY ignore	TYPE FiveG-ProSeAuthorized							PRESENCE optional }|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitrate	CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-FiveG-ProSePC5LinkAMBR					CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-UuRLCChannelToBeSetupList				CRITICALITY reject	TYPE UuRLCChannelToBeSetupList						PRESENCE optional}|

{ ID id-PC5RLCChannelToBeSetupList				CRITICALITY reject	TYPE PC5RLCChannelToBeSetupList						PRESENCE optional}|

{ ID id-PathSwitchConfiguration					CRITICALITY ignore	TYPE PathSwitchConfiguration				 			PRESENCE optional	}|

{ ID id-GNBDUUESliceMaximumBitRateList			CRITICALITY ignore  TYPE GNBDUUESliceMaximumBitRateList 				PRESENCE optional }|

{ ID id-MulticastMBSSessionSetupList			CRITICALITY reject	TYPE MulticastMBSSessionList							PRESENCE optional }|

{ ID id-UE-MulticastMRBs-ToBeSetup-List			CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeSetup-List				PRESENCE optional	}|

{ ID id-ServingCellMO-List						CRITICALITY ignore	TYPE ServingCellMO-List									PRESENCE optional	},

...

}

Candidate-SpCell-List::= SEQUENCE (SIZE(1..maxnoofCandidateSpCells)) OF ProtocolIE-SingleContainer { { Candidate-SpCell-ItemIEs} }

SCell-ToBeSetup-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeSetup-ItemIEs} }

SRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeSetup-ItemIEs} }

DRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeSetup-ItemIEs} }

BHChannels-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeSetup-ItemIEs} }

SLDRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeSetup-ItemIEs} }

UE-MulticastMRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF ProtocolIE-SingleContainer { { UE-MulticastMRBs-ToBeSetup-ItemIEs} }

ServingCellMO-List ::= SEQUENCE (SIZE(1..maxnoofServingCellMOs)) OF ProtocolIE-SingleContainer { { ServingCellMO-List-ItemIEs} }

Candidate-SpCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Candidate-SpCell-Item					CRITICALITY ignore	TYPE Candidate-SpCell-Item						PRESENCE mandatory	},

...

}

SCell-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeSetup-Item						CRITICALITY ignore	TYPE SCell-ToBeSetup-Item					PRESENCE mandatory	},

...

}

SRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeSetup-Item		CRITICALITY reject		TYPE SRBs-ToBeSetup-Item		PRESENCE mandatory},

...

}

DRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeSetup-Item					CRITICALITY reject	TYPE DRBs-ToBeSetup-Item					PRESENCE mandatory},

...

}

BHChannels-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeSetup-Item					CRITICALITY reject	TYPE BHChannels-ToBeSetup-Item					PRESENCE mandatory},

...

}

SLDRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeSetup-Item					CRITICALITY reject	TYPE SLDRBs-ToBeSetup-Item					PRESENCE mandatory},

...

}

UE-MulticastMRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-ToBeSetup-Item			CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeSetup-Item			PRESENCE mandatory},

...

}

ServingCellMO-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-ServingCellMO-List-Item			CRITICALITY reject	TYPE ServingCellMO-List-Item	PRESENCE mandatory},

...

}

-- **************************************************************

--

-- UE CONTEXT SETUP RESPONSE

--

-- **************************************************************

UEContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupResponseIEs} },

...

}

UEContextSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID								PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID								PRESENCE mandatory	}|

{ ID id-DUtoCURRCInformation					CRITICALITY reject	TYPE DUtoCURRCInformation							PRESENCE mandatory }|

{ ID id-C-RNTI									CRITICALITY ignore	TYPE C-RNTI											PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer	PRESENCE optional	}|

{ ID id-FullConfiguration						CRITICALITY reject	TYPE FullConfiguration								PRESENCE optional	}|

{ ID id-DRBs-Setup-List							CRITICALITY ignore	TYPE DRBs-Setup-List								PRESENCE optional	}|

{ ID id-SRBs-FailedToBeSetup-List				CRITICALITY ignore	TYPE SRBs-FailedToBeSetup-List					PRESENCE optional	}|

{ ID id-DRBs-FailedToBeSetup-List				CRITICALITY ignore	TYPE DRBs-FailedToBeSetup-List					PRESENCE optional	}|

{ ID id-SCell-FailedtoSetup-List				CRITICALITY ignore	TYPE SCell-FailedtoSetup-List					PRESENCE optional	}|

{ ID id-InactivityMonitoringResponse			CRITICALITY reject	TYPE InactivityMonitoringResponse				PRESENCE optional	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	}|

{ ID id-SRBs-Setup-List							CRITICALITY ignore	TYPE SRBs-Setup-List								PRESENCE optional	}|

{ ID id-BHChannels-Setup-List					CRITICALITY ignore	TYPE BHChannels-Setup-List						PRESENCE optional	}|

{ ID id-BHChannels-FailedToBeSetup-List			CRITICALITY ignore	TYPE BHChannels-FailedToBeSetup-List			PRESENCE optional	}|

{ ID id-SLDRBs-Setup-List						CRITICALITY ignore	TYPE SLDRBs-Setup-List								PRESENCE optional	}|

{ ID id-SLDRBs-FailedToBeSetup-List				CRITICALITY ignore	TYPE SLDRBs-FailedToBeSetup-List				PRESENCE optional	}|

{ ID id-requestedTargetCellGlobalID				CRITICALITY reject	TYPE NRCGI											PRESENCE optional}|

{ ID id-SCGActivationStatus						CRITICALITY ignore	TYPE SCGActivationStatus							PRESENCE optional }|

{ ID id-UuRLCChannelSetupList					CRITICALITY ignore	TYPE UuRLCChannelSetupList						PRESENCE optional}|

{ ID id-UuRLCChannelFailedToBeSetupList			CRITICALITY ignore	TYPE UuRLCChannelFailedToBeSetupList			PRESENCE optional}|

{ ID id-PC5RLCChannelSetupList					CRITICALITY ignore	TYPE PC5RLCChannelSetupList						PRESENCE optional}|

{ ID id-PC5RLCChannelFailedToBeSetupList		CRITICALITY ignore	TYPE PC5RLCChannelFailedToBeSetupList			PRESENCE optional}|

{ ID id-ServingCellMO-encoded-in-CGC-List		CRITICALITY ignore	TYPE ServingCellMO-encoded-in-CGC-List			PRESENCE optional}|

{ ID id-UE-MulticastMRBs-Setupnew-List			CRITICALITY reject	TYPE UE-MulticastMRBs-Setupnew-List				PRESENCE optional}|

{ ID id-DedicatedSIDeliveryIndication			CRITICALITY ignore	TYPE DedicatedSIDeliveryIndication				PRESENCE optional}|

{ ID id-Configured-BWP-List						CRITICALITY ignore	TYPE Configured-BWP-List								PRESENCE optional},

...

}

DRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-Setup-ItemIEs} }

SRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-FailedToBeSetup-ItemIEs} }

DRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-FailedToBeSetup-ItemIEs} }

SCell-FailedtoSetup-List ::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-FailedtoSetup-ItemIEs} }

SRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-Setup-ItemIEs} }

BHChannels-Setup-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-Setup-ItemIEs} }

BHChannels-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-FailedToBeSetup-ItemIEs} }

DRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Setup-Item						CRITICALITY ignore	TYPE DRBs-Setup-Item						PRESENCE mandatory},

...

}

SRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-Setup-Item						CRITICALITY ignore	TYPE SRBs-Setup-Item						PRESENCE mandatory},

...

}

SRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-FailedToBeSetup-Item		CRITICALITY ignore		TYPE SRBs-FailedToBeSetup-Item		PRESENCE mandatory},

...

}

DRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-FailedToBeSetup-Item		CRITICALITY ignore	TYPE DRBs-FailedToBeSetup-Item			PRESENCE mandatory},

...

}

SCell-FailedtoSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-FailedtoSetup-Item			CRITICALITY ignore	TYPE SCell-FailedtoSetup-Item			PRESENCE mandatory},

...

}

BHChannels-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-Setup-Item						CRITICALITY ignore	TYPE BHChannels-Setup-Item						PRESENCE mandatory},

...

}

BHChannels-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-FailedToBeSetup-Item						CRITICALITY ignore	TYPE BHChannels-FailedToBeSetup-Item		PRESENCE mandatory},

...

}

SLDRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-Setup-ItemIEs} }

SLDRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-FailedToBeSetup-ItemIEs} }

SLDRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-Setup-Item						CRITICALITY ignore	TYPE SLDRBs-Setup-Item						PRESENCE mandatory},

...

}

SLDRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-FailedToBeSetup-Item		CRITICALITY ignore	TYPE SLDRBs-FailedToBeSetup-Item			PRESENCE mandatory},

...

}

UE-MulticastMRBs-Setupnew-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF ProtocolIE-SingleContainer { { UE-MulticastMRBs-Setupnew-ItemIEs } }

UE-MulticastMRBs-Setupnew-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-Setupnew-Item		CRITICALITY reject	TYPE UE-MulticastMRBs-Setupnew-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- UE CONTEXT SETUP FAILURE

--

-- **************************************************************

UEContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextSetupFailureIEs} },

...

}

UEContextSetupFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID			PRESENCE optional	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	}|

{ ID id-Potential-SpCell-List		CRITICALITY ignore	TYPE Potential-SpCell-List		PRESENCE optional	}|

{ ID id-requestedTargetCellGlobalID	CRITICALITY reject	TYPE NRCGI						PRESENCE optional},

...

}

Potential-SpCell-List::= SEQUENCE (SIZE(0..maxnoofPotentialSpCells)) OF ProtocolIE-SingleContainer { { Potential-SpCell-ItemIEs} }

Potential-SpCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Potential-SpCell-Item				CRITICALITY ignore	TYPE Potential-SpCell-Item					PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- UE Context Release Request ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE Context Release Request

--

-- **************************************************************

UEContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ UEContextReleaseRequestIEs}},

...

}

UEContextReleaseRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-targetCellsToCancel				CRITICALITY reject	TYPE TargetCellList					PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- UE Context Release (gNB-CU initiated) ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT RELEASE COMMAND

--

-- **************************************************************

UEContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextReleaseCommandIEs} },

...

}

UEContextReleaseCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-RRCContainer					CRITICALITY ignore	TYPE RRCContainer					PRESENCE optional	}|

{ ID id-SRBID							CRITICALITY ignore	TYPE SRBID							PRESENCE conditional	}|

{ ID id-oldgNB-DU-UE-F1AP-ID			CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID				PRESENCE optional	}|

{ ID id-ExecuteDuplication				CRITICALITY ignore	TYPE ExecuteDuplication				PRESENCE optional}|

{ ID id-RRCDeliveryStatusRequest		CRITICALITY ignore	TYPE RRCDeliveryStatusRequest		PRESENCE optional }|

{ ID id-targetCellsToCancel				CRITICALITY reject	TYPE TargetCellList					PRESENCE optional}|

{ ID id-PosConextRevIndication			CRITICALITY ignore	TYPE PosConextRevIndication			PRESENCE optional}|

{ ID id-CG-SDTKeptIndicator				CRITICALITY ignore	TYPE CG-SDTKeptIndicator			PRESENCE optional},

...

}

-- **************************************************************

--

##### -- UE CONTEXT RELEASE COMPLETE

--

-- **************************************************************

UEContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextReleaseCompleteIEs} },

...

}

UEContextReleaseCompleteIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- UE Context Modification ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

UEContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequestIEs} },

...

}

UEContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-SpCell-ID								CRITICALITY ignore	TYPE NRCGI												PRESENCE optional	}|

{ ID id-ServCellIndex							CRITICALITY reject	TYPE ServCellIndex										PRESENCE optional	}|

{ ID id-SpCellULConfigured						CRITICALITY ignore	TYPE CellULConfigured									PRESENCE optional	}|

{ ID id-DRXCycle								CRITICALITY ignore	TYPE DRXCycle											PRESENCE optional	}|

{ ID id-CUtoDURRCInformation					CRITICALITY reject	TYPE CUtoDURRCInformation								PRESENCE optional	}|

{ ID id-TransmissionActionIndicator				CRITICALITY ignore	TYPE TransmissionActionIndicator					PRESENCE optional	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer		PRESENCE optional	}|

{ ID id-RRCReconfigurationCompleteIndicator		CRITICALITY ignore	TYPE RRCReconfigurationCompleteIndicator			PRESENCE optional	}|

{ ID id-RRCContainer							CRITICALITY reject	TYPE RRCContainer										PRESENCE optional	}|

{ ID id-SCell-ToBeSetupMod-List					CRITICALITY ignore	TYPE SCell-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-SCell-ToBeRemoved-List					CRITICALITY ignore	TYPE SCell-ToBeRemoved-List 							PRESENCE optional }|

{ ID id-SRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE SRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeSetupMod-List					CRITICALITY reject	TYPE DRBs-ToBeSetupMod-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeModified-List					CRITICALITY reject	TYPE DRBs-ToBeModified-List							PRESENCE optional	}|

{ ID id-SRBs-ToBeReleased-List					CRITICALITY reject	TYPE SRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-DRBs-ToBeReleased-List					CRITICALITY reject	TYPE DRBs-ToBeReleased-List							PRESENCE optional	}|

{ ID id-InactivityMonitoringRequest				CRITICALITY reject	TYPE InactivityMonitoringRequest					PRESENCE optional	}|

{ ID id-RAT-FrequencyPriorityInformation		CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation				PRESENCE optional	}|

{ ID id-DRXConfigurationIndicator				CRITICALITY ignore	TYPE DRXConfigurationIndicator						PRESENCE optional	}|

{ ID id-RLCFailureIndication					CRITICALITY ignore	TYPE RLCFailureIndication								PRESENCE optional	}|

{ ID id-UplinkTxDirectCurrentListInformation	CRITICALITY ignore	TYPE UplinkTxDirectCurrentListInformation		PRESENCE optional	}|

{ ID id-GNB-DUConfigurationQuery				CRITICALITY reject	TYPE GNB-DUConfigurationQuery						PRESENCE optional	}|

{ ID id-GNB-DU-UE-AMBR-UL						CRITICALITY ignore	TYPE BitRate											PRESENCE optional	}|

{ ID id-ExecuteDuplication						CRITICALITY ignore	TYPE ExecuteDuplication									PRESENCE optional}|

{ ID id-RRCDeliveryStatusRequest				CRITICALITY ignore	TYPE RRCDeliveryStatusRequest						PRESENCE optional }|

{ ID id-ResourceCoordinationTransferInformation	CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation	PRESENCE optional	}|

{ ID id-ServingCellMO							CRITICALITY ignore	TYPE ServingCellMO										PRESENCE optional	}|

{ ID id-NeedforGap								CRITICALITY ignore	TYPE NeedforGap											PRESENCE optional	}|

{ ID id-FullConfiguration						CRITICALITY reject	TYPE FullConfiguration									PRESENCE optional	}|

{ ID id-AdditionalRRMPriorityIndex				CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex						PRESENCE optional }|

{ ID id-LowerLayerPresenceStatusChange			CRITICALITY ignore	TYPE LowerLayerPresenceStatusChange				PRESENCE optional	}|

{ ID id-BHChannels-ToBeSetupMod-List			CRITICALITY reject	TYPE BHChannels-ToBeSetupMod-List					PRESENCE optional	}|

{ ID id-BHChannels-ToBeModified-List			CRITICALITY reject	TYPE BHChannels-ToBeModified-List					PRESENCE optional	}|

{ ID id-BHChannels-ToBeReleased-List			CRITICALITY reject	TYPE BHChannels-ToBeReleased-List					PRESENCE optional	}|

{ ID id-NRV2XServicesAuthorized					CRITICALITY ignore	TYPE NRV2XServicesAuthorized							PRESENCE optional }|

{ ID id-LTEV2XServicesAuthorized				CRITICALITY ignore	TYPE LTEV2XServicesAuthorized						PRESENCE optional }|

{ ID id-NRUESidelinkAggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate			PRESENCE optional }|

{ ID id-LTEUESidelinkAggregateMaximumBitrate	CRITICALITY ignore	TYPE LTEUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-PC5LinkAMBR								CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-SLDRBs-ToBeSetupMod-List				CRITICALITY reject	TYPE SLDRBs-ToBeSetupMod-List						PRESENCE optional	}|

{ ID id-SLDRBs-ToBeModified-List				CRITICALITY reject	TYPE SLDRBs-ToBeModified-List						PRESENCE optional	}|

{ ID id-SLDRBs-ToBeReleased-List				CRITICALITY reject	TYPE SLDRBs-ToBeReleased-List						PRESENCE optional	}|

{ ID id-ConditionalIntraDUMobilityInformation	CRITICALITY reject	TYPE ConditionalIntraDUMobilityInformation		PRESENCE optional}|

{ ID id-F1CTransferPath							CRITICALITY reject	TYPE F1CTransferPath									PRESENCE optional }|

{ ID id-SCGIndicator							CRITICALITY ignore	TYPE SCGIndicator										PRESENCE optional	}|

{ ID id-UplinkTxDirectCurrentTwoCarrierListInfo		CRITICALITY ignore	TYPE UplinkTxDirectCurrentTwoCarrierListInfo	PRESENCE optional	}|

{ ID id-IABConditionalRRCMessageDeliveryIndication				CRITICALITY reject	TYPE IABConditionalRRCMessageDeliveryIndication							PRESENCE optional	}|

{ ID id-F1CTransferPathNRDC							CRITICALITY reject	TYPE F1CTransferPathNRDC								PRESENCE optional }|

{ ID id-MDTPollutedMeasurementIndicator				CRITICALITY ignore	TYPE MDTPollutedMeasurementIndicator				PRESENCE optional }|

{ ID id-SCGActivationRequest						CRITICALITY ignore	TYPE SCGActivationRequest							PRESENCE optional }|

{ ID id-CG-SDTQueryIndication						CRITICALITY ignore	TYPE CG-SDTQueryIndication							PRESENCE optional	}|

{ ID id-FiveG-ProSeAuthorized						CRITICALITY ignore	TYPE FiveG-ProSeAuthorized							PRESENCE optional }|

{ ID id-FiveG-ProSeUEPC5AggregateMaximumBitrate		CRITICALITY ignore	TYPE NRUESidelinkAggregateMaximumBitrate		PRESENCE optional }|

{ ID id-FiveG-ProSePC5LinkAMBR						CRITICALITY ignore	TYPE BitRate											PRESENCE optional}|

{ ID id-UpdatedRemoteUELocalID						CRITICALITY ignore	TYPE RemoteUELocalID				 					PRESENCE optional	}|

{ ID id-UuRLCChannelToBeSetupList					CRITICALITY reject	TYPE UuRLCChannelToBeSetupList						PRESENCE optional}|

{ ID id-UuRLCChannelToBeModifiedList				CRITICALITY reject	TYPE UuRLCChannelToBeModifiedList					PRESENCE optional}|

{ ID id-UuRLCChannelToBeReleasedList				CRITICALITY reject	TYPE UuRLCChannelToBeReleasedList					PRESENCE optional}|

{ ID id-PC5RLCChannelToBeSetupList					CRITICALITY reject	TYPE PC5RLCChannelToBeSetupList					PRESENCE optional}|

{ ID id-PC5RLCChannelToBeModifiedList				CRITICALITY reject	TYPE PC5RLCChannelToBeModifiedList				PRESENCE optional}|

{ ID id-PC5RLCChannelToBeReleasedList				CRITICALITY reject	TYPE PC5RLCChannelToBeReleasedList				PRESENCE optional}|

{ ID id-PathSwitchConfiguration						CRITICALITY ignore	TYPE PathSwitchConfiguration				 		PRESENCE optional	}|

{ ID id-GNBDUUESliceMaximumBitRateList				CRITICALITY ignore	TYPE GNBDUUESliceMaximumBitRateList				PRESENCE optional }|

{ ID id-MulticastMBSSessionSetupList				CRITICALITY reject	TYPE MulticastMBSSessionList						PRESENCE optional }|

{ ID id-MulticastMBSSessionRemoveList				CRITICALITY reject	TYPE MulticastMBSSessionList						PRESENCE optional }|

{ ID id-UE-MulticastMRBs-ToBeSetup-atModify-List	CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeSetup-atModify-List	PRESENCE optional	}|

{ ID id-UE-MulticastMRBs-ToBeReleased-List			CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeReleased-List			PRESENCE optional	}|

{ ID id-SLDRXCycleList								CRITICALITY ignore  TYPE SLDRXCycleList 										PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNModificationList		CRITICALITY ignore	TYPE MDTPLMNModificationList						PRESENCE optional }|

{ ID id-SDTBearerConfigurationQueryIndication		CRITICALITY ignore	TYPE SDTBearerConfigurationQueryIndication		PRESENCE optional }|

{ ID id-DAPS-HO-Status								CRITICALITY ignore	TYPE DAPS-HO-Status										PRESENCE optional }|

{ ID id-ServingCellMO-List							CRITICALITY ignore	TYPE ServingCellMO-List								PRESENCE optional	}|

{ ID id-UlTxDirectCurrentMoreCarrierInformation	    CRITICALITY ignore	TYPE UlTxDirectCurrentMoreCarrierInformation	PRESENCE optional	}|

{ ID id-CPACMCGInformation						CRITICALITY ignore	TYPE CPACMCGInformation									PRESENCE optional },

...

}

SCell-ToBeSetupMod-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeSetupMod-ItemIEs} }

SCell-ToBeRemoved-List::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-ToBeRemoved-ItemIEs} }

SRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeSetupMod-ItemIEs} }

DRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeSetupMod-ItemIEs} }

BHChannels-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeSetupMod-ItemIEs} }

DRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeModified-ItemIEs} }

BHChannels-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeModified-ItemIEs} }

SRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-ToBeReleased-ItemIEs} }

DRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ToBeReleased-ItemIEs} }

BHChannels-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-ToBeReleased-ItemIEs} }

UE-MulticastMRBs-ToBeSetup-atModify-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF

ProtocolIE-SingleContainer { { UE-MulticastMRBs-ToBeSetup-atModify-ItemIEs} }

UE-MulticastMRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF ProtocolIE-SingleContainer { { UE-MulticastMRBs-ToBeReleased-ItemIEs} }

SCell-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeSetupMod-Item			CRITICALITY ignore	TYPE SCell-ToBeSetupMod-Item			PRESENCE mandatory	},

...

}

SCell-ToBeRemoved-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-ToBeRemoved-Item			CRITICALITY ignore	TYPE SCell-ToBeRemoved-Item			PRESENCE mandatory	},

...

}

SRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE SRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE DRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeModified-Item		CRITICALITY reject	TYPE DRBs-ToBeModified-Item			PRESENCE mandatory},

...

}

SRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-ToBeReleased-Item	CRITICALITY reject	TYPE SRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

DRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ToBeReleased-Item		CRITICALITY reject	TYPE DRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

BHChannels-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeSetupMod-Item		CRITICALITY reject	TYPE BHChannels-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

BHChannels-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeModified-Item		CRITICALITY reject	TYPE BHChannels-ToBeModified-Item		PRESENCE mandatory},

...

}

BHChannels-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-ToBeReleased-Item		CRITICALITY reject	TYPE BHChannels-ToBeReleased-Item		PRESENCE mandatory},

...

}

SLDRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeSetupMod-ItemIEs} }

SLDRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeModified-ItemIEs} }

SLDRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ToBeReleased-ItemIEs} }

SLDRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE SLDRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

SLDRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeModified-Item		CRITICALITY reject	TYPE SLDRBs-ToBeModified-Item		PRESENCE mandatory},

...

}

SLDRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ToBeReleased-Item		CRITICALITY reject	TYPE SLDRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

UE-MulticastMRBs-ToBeSetup-atModify-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-ToBeSetup-atModify-Item	CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeSetup-atModify-Item			PRESENCE mandatory},

...

}

UE-MulticastMRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-ToBeReleased-Item		CRITICALITY reject	TYPE UE-MulticastMRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

UEContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationResponseIEs} },

...

}

UEContextModificationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID								PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID								PRESENCE mandatory	}|

{ ID id-ResourceCoordinationTransferContainer	CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer	PRESENCE optional	}|

{ ID id-DUtoCURRCInformation					CRITICALITY reject	TYPE DUtoCURRCInformation							PRESENCE optional}|

{ ID id-DRBs-SetupMod-List						CRITICALITY ignore	TYPE DRBs-SetupMod-List								PRESENCE optional}|

{ ID id-DRBs-Modified-List						CRITICALITY ignore	TYPE DRBs-Modified-List								PRESENCE optional}|

{ ID id-SRBs-FailedToBeSetupMod-List			CRITICALITY ignore	TYPE SRBs-FailedToBeSetupMod-List				PRESENCE optional	}|

{ ID id-DRBs-FailedToBeSetupMod-List			CRITICALITY ignore	TYPE DRBs-FailedToBeSetupMod-List				PRESENCE optional	}|

{ ID id-SCell-FailedtoSetupMod-List				CRITICALITY ignore	TYPE SCell-FailedtoSetupMod-List				PRESENCE optional	}|

{ ID id-DRBs-FailedToBeModified-List			CRITICALITY ignore	TYPE DRBs-FailedToBeModified-List				PRESENCE optional	}|

{ ID id-InactivityMonitoringResponse			CRITICALITY reject	TYPE InactivityMonitoringResponse				PRESENCE optional	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	}|

{ ID id-C-RNTI									CRITICALITY ignore	TYPE C-RNTI											PRESENCE optional	}|

{ ID id-Associated-SCell-List					CRITICALITY ignore  TYPE Associated-SCell-List						PRESENCE optional	}|

{ ID id-SRBs-SetupMod-List						CRITICALITY ignore	TYPE SRBs-SetupMod-List								PRESENCE optional	}|

{ ID id-SRBs-Modified-List						CRITICALITY ignore	TYPE SRBs-Modified-List								PRESENCE optional	}|

{ ID id-FullConfiguration						CRITICALITY reject	TYPE FullConfiguration								PRESENCE optional	}|

{ ID id-BHChannels-SetupMod-List				CRITICALITY ignore	TYPE BHChannels-SetupMod-List					PRESENCE optional}|

{ ID id-BHChannels-Modified-List				CRITICALITY ignore	TYPE BHChannels-Modified-List					PRESENCE optional}|

{ ID id-BHChannels-FailedToBeSetupMod-List		CRITICALITY ignore	TYPE BHChannels-FailedToBeSetupMod-List		PRESENCE optional	}|

{ ID id-BHChannels-FailedToBeModified-List		CRITICALITY ignore	TYPE BHChannels-FailedToBeModified-List		PRESENCE optional	}|

{ ID id-SLDRBs-SetupMod-List					CRITICALITY ignore	TYPE SLDRBs-SetupMod-List							PRESENCE optional	}|

{ ID id-SLDRBs-Modified-List					CRITICALITY ignore	TYPE SLDRBs-Modified-List							PRESENCE optional	}|

{ ID id-SLDRBs-FailedToBeSetupMod-List			CRITICALITY ignore	TYPE SLDRBs-FailedToBeSetupMod-List			PRESENCE optional	}|

{ ID id-SLDRBs-FailedToBeModified-List			CRITICALITY ignore	TYPE SLDRBs-FailedToBeModified-List			PRESENCE optional	}|

{ ID id-requestedTargetCellGlobalID				CRITICALITY reject	TYPE NRCGI											PRESENCE optional}|

{ ID id-SCGActivationStatus						CRITICALITY ignore	TYPE SCGActivationStatus							PRESENCE optional }|

{ ID id-UuRLCChannelSetupList					CRITICALITY ignore	TYPE UuRLCChannelSetupList						PRESENCE optional}|

{ ID id-UuRLCChannelFailedToBeSetupList			CRITICALITY ignore	TYPE UuRLCChannelFailedToBeSetupList			PRESENCE optional}|

{ ID id-UuRLCChannelModifiedList				CRITICALITY ignore	TYPE UuRLCChannelModifiedList					PRESENCE optional}|

{ ID id-UuRLCChannelFailedToBeModifiedList		CRITICALITY ignore	TYPE UuRLCChannelFailedToBeModifiedList		PRESENCE optional}|

{ ID id-PC5RLCChannelSetupList					CRITICALITY ignore	TYPE PC5RLCChannelSetupList						PRESENCE optional}|

{ ID id-PC5RLCChannelFailedToBeSetupList		CRITICALITY ignore	TYPE PC5RLCChannelFailedToBeSetupList			PRESENCE optional}|

{ ID id-PC5RLCChannelModifiedList				CRITICALITY ignore	TYPE PC5RLCChannelModifiedList					PRESENCE optional}|

{ ID id-PC5RLCChannelFailedToBeModifiedList		CRITICALITY ignore	TYPE PC5RLCChannelFailedToBeModifiedList		PRESENCE optional}|

{ ID id-SDTBearerConfigurationInfo				CRITICALITY ignore	TYPE SDTBearerConfigurationInfo					PRESENCE optional}|

{ ID id-UE-MulticastMRBs-Setup-List				CRITICALITY reject	TYPE UE-MulticastMRBs-Setup-List				PRESENCE optional}|

{ ID id-ServingCellMO-encoded-in-CGC-List		CRITICALITY ignore	TYPE ServingCellMO-encoded-in-CGC-List		PRESENCE optional}|

{ ID id-DedicatedSIDeliveryIndication			CRITICALITY ignore	TYPE DedicatedSIDeliveryIndication			PRESENCE optional}|	{ ID id-Configured-BWP-List						CRITICALITY ignore	TYPE Configured-BWP-List							PRESENCE optional},

...

}

DRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-SetupMod-ItemIEs} }

DRBs-Modified-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-Modified-ItemIEs } }

SRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-SetupMod-ItemIEs} }

SRBs-Modified-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-Modified-ItemIEs } }

DRBs-FailedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-FailedToBeModified-ItemIEs} }

SRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-FailedToBeSetupMod-ItemIEs} }

DRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-FailedToBeSetupMod-ItemIEs} }

SCell-FailedtoSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofSCells)) OF ProtocolIE-SingleContainer { { SCell-FailedtoSetupMod-ItemIEs} }

BHChannels-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-SetupMod-ItemIEs} }

BHChannels-Modified-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-Modified-ItemIEs } }

BHChannels-FailedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-FailedToBeModified-ItemIEs} }

BHChannels-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-FailedToBeSetupMod-ItemIEs} }

Associated-SCell-List ::= SEQUENCE (SIZE(1.. maxnoofSCells)) OF ProtocolIE-SingleContainer { { Associated-SCell-ItemIEs} }

DRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-SetupMod-Item		CRITICALITY ignore		TYPE DRBs-SetupMod-Item		PRESENCE mandatory},

...

}

DRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Modified-Item			CRITICALITY ignore	TYPE DRBs-Modified-Item		PRESENCE mandatory},

...

}

SRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-SetupMod-Item		CRITICALITY ignore		TYPE SRBs-SetupMod-Item		PRESENCE mandatory},

...

}

SRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-Modified-Item			CRITICALITY ignore	TYPE SRBs-Modified-Item		PRESENCE mandatory},

...

}

SRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-FailedToBeSetupMod-Item		CRITICALITY ignore	TYPE SRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-FailedToBeSetupMod-Item		CRITICALITY ignore	TYPE DRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

DRBs-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-FailedToBeModified-Item		CRITICALITY ignore	TYPE DRBs-FailedToBeModified-Item		PRESENCE mandatory},

...

}

SCell-FailedtoSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCell-FailedtoSetupMod-Item			CRITICALITY ignore	TYPE SCell-FailedtoSetupMod-Item			PRESENCE mandatory},

...

}

Associated-SCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Associated-SCell-Item			CRITICALITY ignore	TYPE Associated-SCell-Item			PRESENCE mandatory},

...

}

BHChannels-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-SetupMod-Item		CRITICALITY ignore		TYPE BHChannels-SetupMod-Item		PRESENCE mandatory},

...

}

BHChannels-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-Modified-Item		CRITICALITY ignore	TYPE BHChannels-Modified-Item		PRESENCE mandatory},

...

}

BHChannels-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-FailedToBeSetupMod-Item		CRITICALITY ignore	TYPE BHChannels-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

BHChannels-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-FailedToBeModified-Item		CRITICALITY ignore	TYPE BHChannels-FailedToBeModified-Item		PRESENCE mandatory},

...

}

SLDRBs-SetupMod-List 			::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-SetupMod-ItemIEs} }

SLDRBs-Modified-List				::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-Modified-ItemIEs } }

SLDRBs-FailedToBeModified-List 	::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-FailedToBeModified-ItemIEs} }

SLDRBs-FailedToBeSetupMod-List 	::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-FailedToBeSetupMod-ItemIEs} }

SLDRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-SetupMod-Item		CRITICALITY ignore		TYPE SLDRBs-SetupMod-Item		PRESENCE mandatory},

...

}

SLDRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-Modified-Item			CRITICALITY ignore	TYPE SLDRBs-Modified-Item		PRESENCE mandatory},

...

}

SLDRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-FailedToBeSetupMod-Item		CRITICALITY ignore	TYPE SLDRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

SLDRBs-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-FailedToBeModified-Item		CRITICALITY ignore	TYPE SLDRBs-FailedToBeModified-Item		PRESENCE mandatory},

...

}

UE-MulticastMRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF ProtocolIE-SingleContainer { { UE-MulticastMRBs-Setup-ItemIEs } }

UE-MulticastMRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-Setup-Item		CRITICALITY reject	TYPE UE-MulticastMRBs-Setup-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

UEContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationFailureIEs} },

...

}

UEContextModificationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	}|

{ ID id-requestedTargetCellGlobalID		CRITICALITY reject	TYPE NRCGI							PRESENCE optional},

...

}

-- **************************************************************

--

#### -- UE Context Modification Required (gNB-DU initiated) ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

UEContextModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRequiredIEs} },

...

}

UEContextModificationRequiredIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-ResourceCoordinationTransferContainer		CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer			PRESENCE optional	}|

{ ID id-DUtoCURRCInformation						CRITICALITY reject	TYPE DUtoCURRCInformation								PRESENCE optional}|

{ ID id-DRBs-Required-ToBeModified-List				CRITICALITY reject	TYPE DRBs-Required-ToBeModified-List					PRESENCE optional}|

{ ID id-SRBs-Required-ToBeReleased-List				CRITICALITY reject	TYPE SRBs-Required-ToBeReleased-List					PRESENCE optional}|

{ ID id-DRBs-Required-ToBeReleased-List				CRITICALITY reject	TYPE DRBs-Required-ToBeReleased-List					PRESENCE optional}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause													PRESENCE mandatory	}|

{ ID id-BHChannels-Required-ToBeReleased-List		CRITICALITY reject	TYPE BHChannels-Required-ToBeReleased-List			PRESENCE optional}|

{ ID id-SLDRBs-Required-ToBeModified-List			CRITICALITY reject	TYPE SLDRBs-Required-ToBeModified-List				PRESENCE optional}|

{ ID id-SLDRBs-Required-ToBeReleased-List			CRITICALITY reject	TYPE SLDRBs-Required-ToBeReleased-List				PRESENCE optional}|

{ ID id-targetCellsToCancel							CRITICALITY reject	TYPE TargetCellList											PRESENCE optional}|

{ ID id-UuRLCChannelRequiredToBeModifiedList		CRITICALITY reject	TYPE UuRLCChannelRequiredToBeModifiedList			PRESENCE optional}|

{ ID id-UuRLCChannelRequiredToBeReleasedList		CRITICALITY reject	TYPE UuRLCChannelRequiredToBeReleasedList			PRESENCE optional}|

{ ID id-PC5RLCChannelRequiredToBeModifiedList		CRITICALITY reject	TYPE PC5RLCChannelRequiredToBeModifiedList			PRESENCE optional}|

{ ID id-PC5RLCChannelRequiredToBeReleasedList		CRITICALITY reject	TYPE PC5RLCChannelRequiredToBeReleasedList			PRESENCE optional}|

{ ID id-UE-MulticastMRBs-RequiredToBeModified-List	CRITICALITY reject	TYPE UE-MulticastMRBs-RequiredToBeModified-List	PRESENCE optional  }|

{ ID id-UE-MulticastMRBs-RequiredToBeReleased-List	CRITICALITY reject	TYPE UE-MulticastMRBs-RequiredToBeReleased-List	PRESENCE optional  },

...

}

DRBs-Required-ToBeModified-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-Required-ToBeModified-ItemIEs } }

DRBs-Required-ToBeReleased-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-Required-ToBeReleased-ItemIEs } }

SRBs-Required-ToBeReleased-List::= SEQUENCE (SIZE(1..maxnoofSRBs)) OF ProtocolIE-SingleContainer { { SRBs-Required-ToBeReleased-ItemIEs } }

BHChannels-Required-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF ProtocolIE-SingleContainer { { BHChannels-Required-ToBeReleased-ItemIEs } }

DRBs-Required-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Required-ToBeModified-Item			CRITICALITY reject	TYPE DRBs-Required-ToBeModified-Item		PRESENCE mandatory},

...

}

DRBs-Required-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-Required-ToBeReleased-Item			CRITICALITY reject	TYPE DRBs-Required-ToBeReleased-Item		PRESENCE mandatory},

...

}

SRBs-Required-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SRBs-Required-ToBeReleased-Item			CRITICALITY reject	TYPE SRBs-Required-ToBeReleased-Item			PRESENCE mandatory},

...

}

BHChannels-Required-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BHChannels-Required-ToBeReleased-Item			CRITICALITY reject	TYPE BHChannels-Required-ToBeReleased-Item		PRESENCE mandatory},

...

}

SLDRBs-Required-ToBeModified-List::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-Required-ToBeModified-ItemIEs } }

SLDRBs-Required-ToBeReleased-List::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-Required-ToBeReleased-ItemIEs } }

SLDRBs-Required-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-Required-ToBeModified-Item			CRITICALITY reject	TYPE SLDRBs-Required-ToBeModified-Item		PRESENCE mandatory},

...

}

SLDRBs-Required-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-Required-ToBeReleased-Item			CRITICALITY reject	TYPE SLDRBs-Required-ToBeReleased-Item		PRESENCE mandatory},

...

}

UE-MulticastMRBs-RequiredToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF

ProtocolIE-SingleContainer { { UE-MulticastMRBs-RequiredToBeModified-ItemIEs} }

UE-MulticastMRBs-RequiredToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-RequiredToBeModified-Item	CRITICALITY reject	TYPE UE-MulticastMRBs-RequiredToBeModified-Item		PRESENCE mandatory},

...

}

UE-MulticastMRBs-RequiredToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF

ProtocolIE-SingleContainer { { UE-MulticastMRBs-RequiredToBeReleased-ItemIEs} }

UE-MulticastMRBs-RequiredToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-RequiredToBeReleased-Item		CRITICALITY reject	TYPE UE-MulticastMRBs-RequiredToBeReleased-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- UE CONTEXT MODIFICATION CONFIRM

--

-- **************************************************************

UEContextModificationConfirm::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationConfirmIEs} },

...

}

UEContextModificationConfirmIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-ResourceCoordinationTransferContainer		CRITICALITY ignore	TYPE ResourceCoordinationTransferContainer			PRESENCE optional		}|

{ ID id-DRBs-ModifiedConf-List						CRITICALITY ignore	TYPE DRBs-ModifiedConf-List								PRESENCE optional		}|

{ ID id-RRCContainer								CRITICALITY ignore	TYPE RRCContainer											PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional		}|

{ ID id-ExecuteDuplication							CRITICALITY ignore	TYPE ExecuteDuplication									PRESENCE optional		}|

{ ID id-ResourceCoordinationTransferInformation		CRITICALITY ignore	TYPE ResourceCoordinationTransferInformation		PRESENCE optional		}|

{ ID id-SLDRBs-ModifiedConf-List					CRITICALITY ignore	TYPE SLDRBs-ModifiedConf-List							PRESENCE optional		}|

{ ID id-UuRLCChannelModifiedList					CRITICALITY reject	TYPE UuRLCChannelModifiedList							PRESENCE optional		}|

{ ID id-PC5RLCChannelModifiedList					CRITICALITY reject	TYPE PC5RLCChannelModifiedList							PRESENCE optional		}|

{ ID id-UE-MulticastMRBs-ConfirmedToBeModified-List	CRITICALITY reject	TYPE UE-MulticastMRBs-ConfirmedToBeModified-List	PRESENCE optional		},

...

}

DRBs-ModifiedConf-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRBs-ModifiedConf-ItemIEs } }

DRBs-ModifiedConf-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRBs-ModifiedConf-Item		CRITICALITY ignore	TYPE DRBs-ModifiedConf-Item			PRESENCE mandatory},

...

}

SLDRBs-ModifiedConf-List::= SEQUENCE (SIZE(1..maxnoofSLDRBs)) OF ProtocolIE-SingleContainer { { SLDRBs-ModifiedConf-ItemIEs } }

SLDRBs-ModifiedConf-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SLDRBs-ModifiedConf-Item		CRITICALITY ignore	TYPE SLDRBs-ModifiedConf-Item			PRESENCE mandatory},

...

}

UE-MulticastMRBs-ConfirmedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBsforUE)) OF

ProtocolIE-SingleContainer { { UE-MulticastMRBs-ConfirmedToBeModified-ItemIEs} }

UE-MulticastMRBs-ConfirmedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-MulticastMRBs-ConfirmedToBeModified-Item	CRITICALITY reject	TYPE UE-MulticastMRBs-ConfirmedToBeModified-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

-- UE CONTEXT MODIFICATION REFUSE

--

-- **************************************************************

UEContextModificationRefuse::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { UEContextModificationRefuseIEs} },

...

}

UEContextModificationRefuseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- WRITE-REPLACE WARNING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Write-Replace Warning Request

--

-- **************************************************************

WriteReplaceWarningRequest ::= SEQUENCE {

protocolIEs ProtocolIE-Container { {WriteReplaceWarningRequestIEs} },

...

}

WriteReplaceWarningRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-PWSSystemInformation 				CRITICALITY reject	TYPE PWSSystemInformation 						PRESENCE mandatory }|

{ ID id-RepetitionPeriod 					CRITICALITY reject	TYPE RepetitionPeriod 							PRESENCE mandatory }|

{ ID id-NumberofBroadcastRequest 			CRITICALITY reject	TYPE NumberofBroadcastRequest 				PRESENCE mandatory }|

{ ID id-Cells-To-Be-Broadcast-List			CRITICALITY reject	TYPE Cells-To-Be-Broadcast-List				PRESENCE optional	},

...

}

Cells-To-Be-Broadcast-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-To-Be-Broadcast-List-ItemIEs } }

Cells-To-Be-Broadcast-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-To-Be-Broadcast-Item		CRITICALITY reject	TYPE	Cells-To-Be-Broadcast-Item		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- Write-Replace Warning Response

--

-- **************************************************************

WriteReplaceWarningResponse ::= SEQUENCE {

protocolIEs ProtocolIE-Container { {WriteReplaceWarningResponseIEs} },

...

}

WriteReplaceWarningResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID										PRESENCE mandatory	}|

{ ID id-Cells-Broadcast-Completed-List			CRITICALITY reject	TYPE Cells-Broadcast-Completed-List				PRESENCE optional	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics							PRESENCE optional	}|

{ ID id-Dedicated-SIDelivery-NeededUE-List		CRITICALITY ignore	TYPE Dedicated-SIDelivery-NeededUE-List			PRESENCE optional	},

...

}

Cells-Broadcast-Completed-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Broadcast-Completed-List-ItemIEs } }

Cells-Broadcast-Completed-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Broadcast-Completed-Item		CRITICALITY reject	TYPE	Cells-Broadcast-Completed-Item		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PWS CANCEL ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PWS Cancel Request

--

-- **************************************************************

PWSCancelRequest ::= SEQUENCE {

protocolIEs ProtocolIE-Container { {PWSCancelRequestIEs} },

...

}

PWSCancelRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject TYPE TransactionID									PRESENCE mandatory	}|

{ ID id-NumberofBroadcastRequest 				CRITICALITY reject TYPE NumberofBroadcastRequest						PRESENCE mandatory }|

{ ID id-Broadcast-To-Be-Cancelled-List			CRITICALITY reject TYPE Broadcast-To-Be-Cancelled-List				PRESENCE optional	}|

{ ID id-Cancel-all-Warning-Messages-Indicator	CRITICALITY reject TYPE Cancel-all-Warning-Messages-Indicator	PRESENCE optional	}|

{ ID id-NotificationInformation					CRITICALITY reject TYPE NotificationInformation						PRESENCE optional},

...

}

Broadcast-To-Be-Cancelled-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Broadcast-To-Be-Cancelled-List-ItemIEs } }

Broadcast-To-Be-Cancelled-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Broadcast-To-Be-Cancelled-Item		CRITICALITY reject	TYPE	Broadcast-To-Be-Cancelled-Item		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- PWS Cancel Response

--

-- **************************************************************

PWSCancelResponse ::= SEQUENCE {

protocolIEs ProtocolIE-Container { {PWSCancelResponseIEs} },

...

}

PWSCancelResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-Cells-Broadcast-Cancelled-List	CRITICALITY reject	TYPE Cells-Broadcast-Cancelled-List	PRESENCE optional	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

Cells-Broadcast-Cancelled-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Broadcast-Cancelled-List-ItemIEs } }

Cells-Broadcast-Cancelled-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-Broadcast-Cancelled-Item		CRITICALITY reject	TYPE	Cells-Broadcast-Cancelled-Item		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- UE Inactivity Notification ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE Inactivity Notification

--

-- **************************************************************

UEInactivityNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ UEInactivityNotificationIEs}},

...

}

UEInactivityNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-DRB-Activity-List							CRITICALITY reject	TYPE DRB-Activity-List							PRESENCE mandatory	}|

{ ID id-SDT-Termination-Request						CRITICALITY ignore	TYPE SDT-Termination-Request					PRESENCE optional 	},

...

}

DRB-Activity-List::= SEQUENCE (SIZE(1..maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRB-Activity-ItemIEs } }

DRB-Activity-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Activity-Item			CRITICALITY reject	TYPE DRB-Activity-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- Initial UL RRC Message Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- INITIAL UL RRC Message Transfer

--

-- **************************************************************

InitialULRRCMessageTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ InitialULRRCMessageTransferIEs}},

...

}

InitialULRRCMessageTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-DU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-NRCGI								CRITICALITY reject	TYPE NRCGI									PRESENCE mandatory	}|

{ ID id-C-RNTI								CRITICALITY reject	TYPE C-RNTI									PRESENCE mandatory	}|

{ ID id-RRCContainer						CRITICALITY reject	TYPE RRCContainer							PRESENCE mandatory	}|

{ ID id-DUtoCURRCContainer					CRITICALITY reject	TYPE DUtoCURRCContainer						PRESENCE optional	}|

{ ID id-SULAccessIndication					CRITICALITY ignore	TYPE SULAccessIndication					PRESENCE optional	}|

{ ID id-TransactionID						CRITICALITY ignore	TYPE TransactionID							PRESENCE mandatory	}|

{ ID id-RANUEID								CRITICALITY ignore	TYPE RANUEID								PRESENCE optional	}|

{ ID id-RRCContainer-RRCSetupComplete		CRITICALITY ignore	TYPE RRCContainer-RRCSetupComplete 			PRESENCE optional	}|

{ ID id-NRRedCapUEIndication				CRITICALITY ignore	TYPE NRRedCapUEIndication  					PRESENCE optional	}|

{ ID id-SDTInformation						CRITICALITY ignore	TYPE SDTInformation							PRESENCE optional	}|

{ ID id-SidelinkRelayConfiguration			CRITICALITY ignore	TYPE SidelinkRelayConfiguration			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- DL RRC Message Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DL RRC Message Transfer

--

-- **************************************************************

DLRRCMessageTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ DLRRCMessageTransferIEs}},

...

}

DLRRCMessageTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID							PRESENCE mandatory	}|

{ ID id-oldgNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID							PRESENCE optional	}|

{ ID id-SRBID										CRITICALITY reject	TYPE SRBID											PRESENCE mandatory	}|

{ ID id-ExecuteDuplication							CRITICALITY ignore	TYPE ExecuteDuplication							PRESENCE optional}|

{ ID id-RRCContainer								CRITICALITY reject	TYPE RRCContainer									PRESENCE mandatory	}|

{ ID id-RAT-FrequencyPriorityInformation			CRITICALITY reject	TYPE RAT-FrequencyPriorityInformation		PRESENCE optional	}|

{ ID id-RRCDeliveryStatusRequest					CRITICALITY ignore	TYPE RRCDeliveryStatusRequest					PRESENCE optional }|

{ ID id-UEContextNotRetrievable						CRITICALITY reject	TYPE UEContextNotRetrievable					PRESENCE optional }|

{ ID id-RedirectedRRCmessage						CRITICALITY reject	TYPE OCTET STRING									PRESENCE optional }|

{ ID id-PLMNAssistanceInfoForNetShar				CRITICALITY ignore	TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-new-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID							PRESENCE optional }|

{ ID id-AdditionalRRMPriorityIndex					CRITICALITY ignore	TYPE AdditionalRRMPriorityIndex				PRESENCE optional }|

{ ID id-SRBMappingInfo								CRITICALITY ignore	TYPE UuRLCChannelID					 				PRESENCE optional },

...

}

-- **************************************************************

--

#### -- UL RRC Message Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UL RRC Message Transfer

--

-- **************************************************************

ULRRCMessageTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ ULRRCMessageTransferIEs}},

...

}

ULRRCMessageTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-SRBID							CRITICALITY reject	TYPE SRBID							PRESENCE mandatory	}|

{ ID id-RRCContainer					CRITICALITY reject	TYPE RRCContainer					PRESENCE mandatory	}|

{ ID id-SelectedPLMNID					CRITICALITY reject	TYPE PLMN-Identity					PRESENCE optional		}|

{ ID id-new-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs F1AP-PRIVATE-IES ::= {

...

}

-- **************************************************************

--

#### -- System Information ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- System information Delivery Command

--

-- **************************************************************

SystemInformationDeliveryCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ SystemInformationDeliveryCommandIEs}},

...

}

SystemInformationDeliveryCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-SItype-List				CRITICALITY reject	TYPE SItype-List					PRESENCE mandatory	}|

{ ID id-ConfirmedUEID 			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- Paging PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Paging

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PagingIEs}},

...

}

PagingIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue	CRITICALITY reject	TYPE UEIdentityIndexValue		PRESENCE mandatory	}|

{ ID id-PagingIdentity			CRITICALITY reject	TYPE PagingIdentity				PRESENCE mandatory	}|

{ ID id-PagingDRX				CRITICALITY ignore	TYPE PagingDRX					PRESENCE optional	}|

{ ID id-PagingPriority			CRITICALITY ignore	TYPE PagingPriority				PRESENCE optional	}|

{ ID id-PagingCell-List			CRITICALITY ignore	TYPE PagingCell-list			PRESENCE mandatory	}|

{ ID id-PagingOrigin			CRITICALITY ignore	TYPE PagingOrigin				PRESENCE optional	}|

{ ID id-RANUEPagingDRX			CRITICALITY ignore	TYPE PagingDRX					PRESENCE optional	}|

{ ID id-CNUEPagingDRX			CRITICALITY ignore	TYPE PagingDRX					PRESENCE optional	}|

{ ID id-NRPagingeDRXInformation	CRITICALITY ignore	TYPE NRPagingeDRXInformation	PRESENCE optional	}|

{ ID id-NRPagingeDRXInformationforRRCINACTIVE	CRITICALITY ignore	TYPE NRPagingeDRXInformationforRRCINACTIVE	PRESENCE optional	}|

{ ID id-PagingCause			CRITICALITY ignore	TYPE PagingCause				PRESENCE optional	}|

{ ID id-PEIPSAssistanceInfo		CRITICALITY ignore	TYPE PEIPSAssistanceInfo		PRESENCE optional	}|

{ ID id-UEPagingCapability		CRITICALITY ignore	TYPE UEPagingCapability			PRESENCE optional	}|

{ ID id-ExtendedUEIdentityIndexValue	CRITICALITY ignore	TYPE ExtendedUEIdentityIndexValue		PRESENCE optional}|

{ ID id-HashedUEIdentityIndexValue		CRITICALITY ignore	TYPE HashedUEIdentityIndexValue			PRESENCE optional},

...

}

PagingCell-list::= SEQUENCE (SIZE(1.. maxnoofPagingCells)) OF ProtocolIE-SingleContainer { { PagingCell-ItemIEs } }

PagingCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-PagingCell-Item		CRITICALITY ignore	TYPE PagingCell-Item			PRESENCE mandatory}	,

...

}

-- **************************************************************

--

#### -- Notify

--

-- **************************************************************

Notify ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ NotifyIEs}},

...

}

NotifyIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-DRB-Notify-List						CRITICALITY reject	TYPE DRB-Notify-List						PRESENCE mandatory	},

...

}

DRB-Notify-List::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF ProtocolIE-SingleContainer { { DRB-Notify-ItemIEs } }

DRB-Notify-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Notify-Item			CRITICALITY reject	TYPE DRB-Notify-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- NETWORK ACCESS RATE REDUCTION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Network Access Rate Reduction

--

-- **************************************************************

NetworkAccessRateReduction ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ NetworkAccessRateReductionIEs }},

...

}

NetworkAccessRateReductionIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID 					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-UAC-Assistance-Info				CRITICALITY reject	TYPE UAC-Assistance-Info			PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PWS RESTART INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PWS Restart Indication

--

-- **************************************************************

PWSRestartIndication ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { PWSRestartIndicationIEs} },

...

}

PWSRestartIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-NR-CGI-List-For-Restart-List	CRITICALITY reject	TYPE NR-CGI-List-For-Restart-List	PRESENCE mandatory	},

...

}

NR-CGI-List-For-Restart-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { NR-CGI-List-For-Restart-List-ItemIEs } }

NR-CGI-List-For-Restart-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-NR-CGI-List-For-Restart-Item		CRITICALITY reject	TYPE	NR-CGI-List-For-Restart-Item		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PWS FAILURE INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PWS Failure Indication

--

-- **************************************************************

PWSFailureIndication ::= SEQUENCE {

protocolIEs ProtocolIE-Container { { PWSFailureIndicationIEs} },

...

}

PWSFailureIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-PWS-Failed-NR-CGI-List	CRITICALITY reject	TYPE PWS-Failed-NR-CGI-List		PRESENCE optional	},

...

}

PWS-Failed-NR-CGI-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { PWS-Failed-NR-CGI-List-ItemIEs } }

PWS-Failed-NR-CGI-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-PWS-Failed-NR-CGI-Item		CRITICALITY reject	TYPE	PWS-Failed-NR-CGI-Item		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- gNB-DU STATUS INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- gNB-DU Status Indication

--

-- **************************************************************

GNBDUStatusIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUStatusIndicationIEs} },

...

}

GNBDUStatusIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-GNBDUOverloadInformation		CRITICALITY reject	TYPE GNBDUOverloadInformation		PRESENCE mandatory	}|

{ ID id-IABCongestionIndication			CRITICALITY ignore	TYPE IABCongestionIndication		PRESENCE optional		},

...

}

-- **************************************************************

--

#### -- RRC Delivery Report ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- RRC Delivery Report

--

-- **************************************************************

RRCDeliveryReport ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ RRCDeliveryReportIEs}},

...

}

RRCDeliveryReportIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID	CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID	PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID	CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID	PRESENCE mandatory	}|

{ ID id-RRCDeliveryStatus	CRITICALITY ignore	TYPE RRCDeliveryStatus	PRESENCE mandatory	}|

{ ID id-SRBID				CRITICALITY ignore	TYPE SRBID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- F1 Removal ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- F1 Removal Request

--

-- **************************************************************

F1RemovalRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ F1RemovalRequestIEs }},

...

}

F1RemovalRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- F1 Removal Response

--

-- **************************************************************

F1RemovalResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ F1RemovalResponseIEs }},

...

}

F1RemovalResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- F1 Removal Failure

--

-- **************************************************************

F1RemovalFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ F1RemovalFailureIEs }},

...

}

F1RemovalFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- TRACE ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- TRACE START

--

-- **************************************************************

TraceStart ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {TraceStartIEs} },

...

}

TraceStartIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-TraceActivation				CRITICALITY ignore	TYPE TraceActivation				PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- DEACTIVATE TRACE

--

-- **************************************************************

DeactivateTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {DeactivateTraceIEs} },

...

}

DeactivateTraceIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-TraceID						CRITICALITY ignore	TYPE TraceID						PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- CELL TRAFFIC TRACE

--

-- **************************************************************

CellTrafficTrace ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {CellTrafficTraceIEs} },

...

}

CellTrafficTraceIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ID id-TraceID							CRITICALITY ignore	TYPE TraceID						PRESENCE mandatory	}|

{ID id-TraceCollectionEntityIPAddress	CRITICALITY ignore	TYPE TransportLayerAddress			PRESENCE mandatory	}|

{ID id-PrivacyIndicator					CRITICALITY ignore	TYPE PrivacyIndicator				PRESENCE optional	}|

{ID id-TraceCollectionEntityURI	CRITICALITY ignore	TYPE URI-address		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- DU-CU Radio Information Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- DU-CU Radio Information Transfer

--

-- **************************************************************

DUCURadioInformationTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ DUCURadioInformationTransferIEs}},

...

}

DUCURadioInformationTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-DUCURadioInformationType		CRITICALITY ignore	TYPE DUCURadioInformationType				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- CU-DU Radio Information Transfer ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- CU-DU Radio Information Transfer

--

-- **************************************************************

CUDURadioInformationTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDURadioInformationTransferIEs}},

...

}

CUDURadioInformationTransferIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory	}|

{ ID id-CUDURadioInformationType	CRITICALITY ignore	TYPE CUDURadioInformationType				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- IAB PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

#### -- BAP Mapping Configuration ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BAP MAPPING CONFIGURATION

--

-- **************************************************************

BAPMappingConfiguration ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{ {BAPMappingConfiguration-IEs} },

...

}

BAPMappingConfiguration-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE	TransactionID								PRESENCE mandatory}|

{ ID id-BH-Routing-Information-Added-List		CRITICALITY ignore	TYPE	BH-Routing-Information-Added-List	PRESENCE optional}|

{ ID id-BH-Routing-Information-Removed-List		CRITICALITY ignore	TYPE	BH-Routing-Information-Removed-List	PRESENCE optional}|

{ ID id-TrafficMappingInformation				CRITICALITY ignore	TYPE	TrafficMappingInfo						PRESENCE optional}|

{ ID id-BufferSizeThresh						CRITICALITY ignore	TYPE	BufferSizeThresh							PRESENCE optional}|

{ ID id-BAP-Header-Rewriting-Added-List			CRITICALITY ignore	TYPE	BAP-Header-Rewriting-Added-List		PRESENCE optional}|

{ ID id-Re-routingEnableIndicator				CRITICALITY ignore	TYPE	Re-routingEnableIndicator				PRESENCE optional}|

{ ID id-BAP-Header-Rewriting-Removed-List		CRITICALITY ignore	TYPE	BAP-Header-Rewriting-Removed-List	PRESENCE optional},

...

}

BH-Routing-Information-Added-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries))	OF ProtocolIE-SingleContainer { { BH-Routing-Information-Added-List-ItemIEs } }

BH-Routing-Information-Removed-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries))	OF ProtocolIE-SingleContainer { { BH-Routing-Information-Removed-List-ItemIEs } }

BH-Routing-Information-Added-List-ItemIEs	F1AP-PROTOCOL-IES ::= {

{ ID id-BH-Routing-Information-Added-List-Item				CRITICALITY ignore	TYPE BH-Routing-Information-Added-List-Item						PRESENCE mandatory},

...

}

BH-Routing-Information-Removed-List-ItemIEs	F1AP-PROTOCOL-IES ::= {

{ ID id-BH-Routing-Information-Removed-List-Item				CRITICALITY ignore	TYPE BH-Routing-Information-Removed-List-Item						PRESENCE mandatory},

...

}

BAP-Header-Rewriting-Added-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries))	OF ProtocolIE-SingleContainer { { BAP-Header-Rewriting-Added-List-ItemIEs } }

BAP-Header-Rewriting-Added-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BAP-Header-Rewriting-Added-List-Item	CRITICALITY ignore	TYPE BAP-Header-Rewriting-Added-List-Item PRESENCE mandatory},

...

}

BAP-Header-Rewriting-Removed-List ::= SEQUENCE (SIZE(1.. maxnoofRoutingEntries))	OF ProtocolIE-SingleContainer { { BAP-Header-Rewriting-Removed-List-ItemIEs } }

BAP-Header-Rewriting-Removed-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BAP-Header-Rewriting-Removed-List-Item	CRITICALITY ignore	TYPE BAP-Header-Rewriting-Removed-List-Item PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- BAP MAPPING CONFIGURATION ACKNOWLEDGE

--

-- **************************************************************

BAPMappingConfigurationAcknowledge ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BAPMappingConfigurationAcknowledge-IEs} },

...

}

BAPMappingConfigurationAcknowledge-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE	TransactionID			PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE	CriticalityDiagnostics	PRESENCE optional},

...

}

-- **************************************************************

--

##### -- BAP MAPPING CONFIGURATION FAILURE

--

-- **************************************************************

BAPMappingConfigurationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { BAPMappingConfigurationFailureIEs} },

...

}

BAPMappingConfigurationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-DU Configuration ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- GNB-DU RESOURCE CONFIGURATION

--

-- **************************************************************

GNBDUResourceConfiguration ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{{ GNBDUResourceConfigurationIEs}},

...

}

GNBDUResourceConfigurationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-Activated-Cells-to-be-Updated-List		CRITICALITY reject	TYPE Activated-Cells-to-be-Updated-List	PRESENCE optional}|

{ ID id-Child-Nodes-List						CRITICALITY reject	TYPE Child-Nodes-List							PRESENCE optional}|

{ ID id-Neighbour-Node-Cells-List				CRITICALITY reject	TYPE Neighbour-Node-Cells-List				PRESENCE optional}|

{ ID id-Serving-Cells-List						CRITICALITY reject	TYPE Serving-Cells-List							PRESENCE optional},

...

}

-- **************************************************************

--

##### -- GNB-DU RESOURCE CONFIGURATION ACKNOWLEDGE

--

-- **************************************************************

GNBDUResourceConfigurationAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { GNBDUResourceConfigurationAcknowledgeIEs} },

...

}

GNBDUResourceConfigurationAcknowledgeIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-DU RESOURCE CONFIGURATION FAILURE

--

-- **************************************************************

GNBDUResourceConfigurationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { GNBDUResourceConfigurationFailureIEs} },

...

}

GNBDUResourceConfigurationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- IAB TNL Address Allocation ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- IAB TNL ADDRESS REQUEST

--

-- **************************************************************

IABTNLAddressRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {IABTNLAddressRequestIEs} },

...

}

IABTNLAddressRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-IABv4AddressesRequested				CRITICALITY reject	TYPE IABv4AddressesRequested					PRESENCE optional	}|

{ ID id-IABIPv6RequestType					CRITICALITY reject	TYPE IABIPv6RequestType							PRESENCE optional	}|

{ ID id-IAB-TNL-Addresses-To-Remove-List	CRITICALITY reject	TYPE IAB-TNL-Addresses-To-Remove-List		PRESENCE optional	}|

{ ID id-IAB-TNL-Addresses-Exception			CRITICALITY reject	TYPE IAB-TNL-Addresses-Exception				PRESENCE optional	},

...

}

IAB-TNL-Addresses-To-Remove-List	::= SEQUENCE (SIZE(1..maxnoofTLAsIAB))	OF ProtocolIE-SingleContainer { { IAB-TNL-Addresses-To-Remove-ItemIEs } }

IAB-TNL-Addresses-To-Remove-ItemIEs	F1AP-PROTOCOL-IES::= {

{ ID id-IAB-TNL-Addresses-To-Remove-Item			CRITICALITY reject	TYPE IAB-TNL-Addresses-To-Remove-Item					PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- IAB TNL ADDRESS RESPONSE

--

-- **************************************************************

IABTNLAddressResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {IABTNLAddressResponseIEs} },

...

}

IABTNLAddressResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID								CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory	}|

{ ID id-IAB-Allocated-TNL-Address-List				CRITICALITY reject	TYPE IAB-Allocated-TNL-Address-List			PRESENCE mandatory	},

...

}

IAB-Allocated-TNL-Address-List ::= SEQUENCE (SIZE(1.. maxnoofTLAsIAB))	OF ProtocolIE-SingleContainer { { IAB-Allocated-TNL-Address-List-ItemIEs } }

IAB-Allocated-TNL-Address-List-ItemIEs	F1AP-PROTOCOL-IES::= {

{ ID id-IAB-Allocated-TNL-Address-Item			CRITICALITY reject	TYPE IAB-Allocated-TNL-Address-Item					PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- IAB TNL ADDRESS FAILURE

--

-- **************************************************************

IABTNLAddressFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { IABTNLAddressFailureIEs} },

...

}

IABTNLAddressFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- IAB UP Configuration Update ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- IAB UP Configuration Update Request

--

-- **************************************************************

IABUPConfigurationUpdateRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { IABUPConfigurationUpdateRequestIEs} },

...

}

IABUPConfigurationUpdateRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID										PRESENCE mandatory  }|

{ ID id-UL-UP-TNL-Information-to-Update-List	CRITICALITY ignore	TYPE UL-UP-TNL-Information-to-Update-List		PRESENCE optional	}|

{ ID id-UL-UP-TNL-Address-to-Update-List		CRITICALITY ignore	TYPE UL-UP-TNL-Address-to-Update-List				PRESENCE optional	},

...

}

UL-UP-TNL-Information-to-Update-List ::= SEQUENCE (SIZE(1.. maxnoofULUPTNLInformationforIAB))	OF ProtocolIE-SingleContainer { { UL-UP-TNL-Information-to-Update-List-ItemIEs } }

UL-UP-TNL-Information-to-Update-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UL-UP-TNL-Information-to-Update-List-Item	CRITICALITY ignore	TYPE UL-UP-TNL-Information-to-Update-List-Item PRESENCE mandatory },

...

}

UL-UP-TNL-Address-to-Update-List ::= SEQUENCE (SIZE(1.. maxnoofUPTNLAddresses))	OF ProtocolIE-SingleContainer { { UL-UP-TNL-Address-to-Update-List-ItemIEs } }

UL-UP-TNL-Address-to-Update-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UL-UP-TNL-Address-to-Update-List-Item	CRITICALITY ignore	TYPE UL-UP-TNL-Address-to-Update-List-Item PRESENCE mandatory },

...

}

-- **************************************************************

--

-- IAB UP Configuration Update Response

--

-- **************************************************************

IABUPConfigurationUpdateResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { IABUPConfigurationUpdateResponseIEs} },

...

}

IABUPConfigurationUpdateResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional	}|

{ ID id-DL-UP-TNL-Address-to-Update-List	CRITICALITY reject	TYPE DL-UP-TNL-Address-to-Update-List	PRESENCE optional	},

...

}

DL-UP-TNL-Address-to-Update-List ::= SEQUENCE (SIZE(1.. maxnoofUPTNLAddresses))	OF ProtocolIE-SingleContainer { { DL-UP-TNL-Address-to-Update-List-ItemIEs } }

DL-UP-TNL-Address-to-Update-List-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-DL-UP-TNL-Address-to-Update-List-Item	CRITICALITY ignore	TYPE DL-UP-TNL-Address-to-Update-List-Item	PRESENCE mandatory },

...

}

-- **************************************************************

--

##### -- IAB UP Configuration Update Failure

--

-- **************************************************************

IABUPConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ { IABUPConfigurationUpdateFailureIEs} },

...

}

IABUPConfigurationUpdateFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- Resource Status Reporting Initiation ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Resource Status Request

--

-- **************************************************************

ResourceStatusRequest::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResourceStatusRequestIEs} },

...

}

ResourceStatusRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID		CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID		CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE conditional	}|

{ ID id-RegistrationRequest		CRITICALITY ignore	TYPE RegistrationRequest	PRESENCE mandatory	}|

{ ID id-ReportCharacteristics	CRITICALITY ignore	TYPE ReportCharacteristics	PRESENCE conditional	}|

{ ID id-CellToReportList		CRITICALITY ignore	TYPE CellToReportList		PRESENCE optional	}|

{ ID id-ReportingPeriodicity	CRITICALITY ignore	TYPE ReportingPeriodicity	PRESENCE  optional	},

...

}

-- **************************************************************

--

##### -- Resource Status Response

--

-- **************************************************************

ResourceStatusResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { ResourceStatusResponseIEs} },

...

}

ResourceStatusResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID		CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID		CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- Resource Status Failure

--

-- **************************************************************

ResourceStatusFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { ResourceStatusFailureIEs} },

...

}

ResourceStatusFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID		CRITICALITY reject	TYPE GNBCUMeasurementID		PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID		CRITICALITY ignore	TYPE GNBDUMeasurementID		PRESENCE mandatory	}|

{ ID id-Cause					CRITICALITY ignore	TYPE Cause					PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics	PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- Resource Status Reporting ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Resource Status Update

--

-- **************************************************************

ResourceStatusUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ ResourceStatusUpdateIEs}},

...

}

ResourceStatusUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID			PRESENCE mandatory	}|

{ ID id-gNBCUMeasurementID			CRITICALITY reject	TYPE GNBCUMeasurementID				PRESENCE mandatory	}|

{ ID id-gNBDUMeasurementID			CRITICALITY ignore	TYPE GNBDUMeasurementID			PRESENCE mandatory	}|

{ ID id-HardwareLoadIndicator			CRITICALITY ignore	TYPE HardwareLoadIndicator			PRESENCE optional	}|

{ ID id-TNLCapacityIndicator			CRITICALITY ignore	TYPE TNLCapacityIndicator		PRESENCE optional	}|

{ ID id-CellMeasurementResultList		CRITICALITY ignore	TYPE CellMeasurementResultList	PRESENCE optional	},

...

}

-- **************************************************************

--

#### --  Access And Mobility Indication ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Access And Mobility Indication

--

-- **************************************************************

AccessAndMobilityIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { AccessAndMobilityIndicationIEs} },

...

}

AccessAndMobilityIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-RACHReportInformationList				CRITICALITY ignore	TYPE RACHReportInformationList			PRESENCE optional }|

{ ID id-RLFReportInformationList				CRITICALITY ignore	TYPE RLFReportInformationList				PRESENCE optional }|

{ ID id-SuccessfulHOReportInformationList		CRITICALITY ignore	TYPE SuccessfulHOReportInformationList	PRESENCE optional },

...

}

-- **************************************************************

--

##### -- REFERENCE TIME INFORMATION REPORTING CONTROL

--

-- **************************************************************

ReferenceTimeInformationReportingControl::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { ReferenceTimeInformationReportingControlIEs} },

...

}

ReferenceTimeInformationReportingControlIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-ReportingRequestType		CRITICALITY reject	TYPE ReportingRequestType		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- REFERENCE TIME INFORMATION REPORT

--

-- **************************************************************

ReferenceTimeInformationReport::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { ReferenceTimeInformationReportIEs} },

...

}

ReferenceTimeInformationReportIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY ignore	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-TimeReferenceInformation	CRITICALITY ignore	TYPE TimeReferenceInformation		PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- Access Success

--

-- **************************************************************

AccessSuccess ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ AccessSuccessIEs}},

...

}

AccessSuccessIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-NRCGI								CRITICALITY reject	TYPE NRCGI									PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- POSITIONING ASSISTANCE INFORMATION CONTROL ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Assistance Information Control

--

-- **************************************************************

PositioningAssistanceInformationControl ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PositioningAssistanceInformationControlIEs}},

...

}

PositioningAssistanceInformationControlIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-PosAssistance-Information		CRITICALITY reject	TYPE PosAssistance-Information		PRESENCE optional}|

{ ID id-PosBroadcast					CRITICALITY reject	TYPE PosBroadcast				PRESENCE optional}|

{ ID id-PositioningBroadcastCells		CRITICALITY reject	TYPE PositioningBroadcastCells		PRESENCE optional}|

{ ID id-RoutingID						CRITICALITY reject	TYPE RoutingID				PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITIONING ASSISTANCE INFORMATION FEEDBACK ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Assistance Information Feedback

--

-- **************************************************************

PositioningAssistanceInformationFeedback ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PositioningAssistanceInformationFeedbackIEs}},

...

}

PositioningAssistanceInformationFeedbackIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID							PRESENCE mandatory	}|

{ ID id-PosAssistanceInformationFailureList	CRITICALITY reject	TYPE PosAssistanceInformationFailureList	PRESENCE optional}|

{ ID id-PositioningBroadcastCells				CRITICALITY reject	TYPE PositioningBroadcastCells				PRESENCE optional}|

{ ID id-RoutingID								CRITICALITY reject	TYPE RoutingID									PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional},

...

}

-- **************************************************************

--

#### -- POSITONING MEASUREMENT EXCHANGE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Measurement Request

--

-- **************************************************************

PositioningMeasurementRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningMeasurementRequestIEs} },

...

}

PositioningMeasurementRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory}|

{ ID id-LMF-MeasurementID						CRITICALITY reject	TYPE LMF-MeasurementID								PRESENCE mandatory}|

{ ID id-RAN-MeasurementID						CRITICALITY reject	TYPE RAN-MeasurementID								PRESENCE mandatory}|

{ ID id-TRP-MeasurementRequestList				CRITICALITY reject	TYPE TRP-MeasurementRequestList					PRESENCE mandatory}|

{ ID id-PosReportCharacteristics				CRITICALITY reject	TYPE PosReportCharacteristics					PRESENCE mandatory}|

{ ID id-PosMeasurementPeriodicity				CRITICALITY reject	TYPE MeasurementPeriodicity						PRESENCE conditional }|

-- The above IE shall be present if the PosReportCharacteristics IE is set to “periodic” --

{ ID id-PosMeasurementQuantities				CRITICALITY reject	TYPE PosMeasurementQuantities					PRESENCE mandatory}|

{ ID id-SFNInitialisationTime					CRITICALITY ignore	TYPE RelativeTime1900	PRESENCE optional	}|

{ ID id-SRSConfiguration						CRITICALITY ignore	TYPE SRSConfiguration								PRESENCE optional}|

{ ID id-MeasurementBeamInfoRequest				CRITICALITY ignore	TYPE MeasurementBeamInfoRequest	PRESENCE optional	}|

{ ID id-SystemFrameNumber						CRITICALITY ignore	TYPE SystemFrameNumber		PRESENCE optional}|

{ ID id-SlotNumber								CRITICALITY ignore	TYPE SlotNumber				PRESENCE optional}|

{ ID id-PosMeasurementPeriodicityExtended		CRITICALITY reject	TYPE MeasurementPeriodicityExtended			PRESENCE conditional }|

-- The IE shall be present the MeasurementPeriodicity IE is set to the value "extended"

{ ID id-ResponseTime							CRITICALITY ignore	TYPE ResponseTime									PRESENCE optional}|

{ ID id-MeasurementCharacteristicsRequestIndicator			CRITICALITY ignore	TYPE MeasurementCharacteristicsRequestIndicator	PRESENCE optional}|

{ ID id-MeasurementTimeOccasion					CRITICALITY ignore	TYPE MeasurementTimeOccasion	PRESENCE optional	}|

{ ID id-PosMeasurementAmount					CRITICALITY ignore	TYPE PosMeasurementAmount	PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- Positioning Measurement Response

--

-- **************************************************************

PositioningMeasurementResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningMeasurementResponseIEs} },

...

}

PositioningMeasurementResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID							CRITICALITY reject	TYPE TransactionID									PRESENCE mandatory}|

{ ID id-LMF-MeasurementID						CRITICALITY reject	TYPE LMF-MeasurementID								PRESENCE mandatory}|

{ ID id-RAN-MeasurementID						CRITICALITY reject	TYPE RAN-MeasurementID								PRESENCE mandatory}|

{ ID id-PosMeasurementResultList				CRITICALITY reject	TYPE PosMeasurementResultList					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- Positioning Measurement Failure

--

-- **************************************************************

PositioningMeasurementFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningMeasurementFailureIEs} },

...

}

PositioningMeasurementFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-LMF-MeasurementID				CRITICALITY reject	TYPE LMF-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-MeasurementID				CRITICALITY reject	TYPE RAN-MeasurementID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- POSITIONING MEASUREMENT REPORT ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Measurement Report

--

-- **************************************************************

PositioningMeasurementReport ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PositioningMeasurementReportIEs} },

...

}

PositioningMeasurementReportIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-LMF-MeasurementID			CRITICALITY reject	TYPE LMF-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-MeasurementID			CRITICALITY reject	TYPE RAN-MeasurementID			PRESENCE mandatory	}|

{ ID id-PosMeasurementResultList	CRITICALITY reject	TYPE PosMeasurementResultList	PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- POSITIONING MEASUREMENT ABORT ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Measurement Abort

--

-- **************************************************************

PositioningMeasurementAbort ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PositioningMeasurementAbortIEs} },

...

}

PositioningMeasurementAbortIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-LMF-MeasurementID				CRITICALITY reject	TYPE LMF-MeasurementID				PRESENCE mandatory	}|

{ ID id-RAN-MeasurementID				CRITICALITY reject	TYPE RAN-MeasurementID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- POSITIONING MEASUREMENT FAILURE INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Measurement Failure Indication

--

-- **************************************************************

PositioningMeasurementFailureIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PositioningMeasurementFailureIndicationIEs} },

...

}

PositioningMeasurementFailureIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-LMF-MeasurementID			CRITICALITY reject	TYPE LMF-MeasurementID				PRESENCE mandatory	}|

{ ID id-RAN-MeasurementID			CRITICALITY reject	TYPE RAN-MeasurementID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- POSITIONING MEASUREMENT UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Measurement Update

--

-- **************************************************************

PositioningMeasurementUpdate ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PositioningMeasurementUpdateIEs} },

...

}

PositioningMeasurementUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-LMF-MeasurementID			CRITICALITY reject	TYPE LMF-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-MeasurementID			CRITICALITY reject	TYPE RAN-MeasurementID			PRESENCE mandatory	}|

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-TRP-MeasurementUpdateList	CRITICALITY reject	TYPE TRP-MeasurementUpdateList 	PRESENCE optional}|

{ ID id-MeasurementCharacteristicsRequestIndicator	CRITICALITY ignore	TYPE	MeasurementCharacteristicsRequestIndicator	PRESENCE optional}|

{ ID id-MeasurementTimeOccasion		CRITICALITY ignore	TYPE MeasurementTimeOccasion	PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- TRP INFORMATION EXCHANGE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- TRP Information Request

--

-- **************************************************************

TRPInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { TRPInformationRequestIEs} },

...

}

TRPInformationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID						CRITICALITY reject	TYPE TransactionID								PRESENCE mandatory	}|

{ ID id-TRPList								CRITICALITY ignore	TYPE TRPList									PRESENCE optional	}|

{ ID id-TRPInformationTypeListTRPReq		CRITICALITY reject	TYPE TRPInformationTypeListTRPReq			PRESENCE mandatory	},

...

}

TRPInformationTypeListTRPReq ::= SEQUENCE (SIZE(1.. maxnoofTRPInfoTypes)) OF ProtocolIE-SingleContainer { { TRPInformationTypeItemTRPReq } }

TRPInformationTypeItemTRPReq 	F1AP-PROTOCOL-IES ::= {

{ ID id-TRPInformationTypeItem	 CRITICALITY reject		TYPE TRPInformationTypeItem  	PRESENCE mandatory },

...

}

-- **************************************************************

--

##### -- TRP Information Response

--

-- **************************************************************

TRPInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { TRPInformationResponseIEs} },

...

}

TRPInformationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID					CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-TRPInformationListTRPResp		CRITICALITY ignore	TYPE TRPInformationListTRPResp		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

TRPInformationListTRPResp ::= SEQUENCE (SIZE(1.. maxnoofTRPs)) OF ProtocolIE-SingleContainer { { TRPInformationItemTRPResp } }

TRPInformationItemTRPResp 	F1AP-PROTOCOL-IES ::= {

{ ID id-TRPInformationItem	 CRITICALITY ignore		TYPE TRPInformationItem  	PRESENCE mandatory },

...

}

-- **************************************************************

--

##### -- TRP Information Failure

--

-- **************************************************************

TRPInformationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { TRPInformationFailureIEs} },

...

}

TRPInformationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID				CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION EXCHANGE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Information Request

--

-- **************************************************************

PositioningInformationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningInformationRequestIEs} },

...

}

PositioningInformationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID									PRESENCE mandatory	}|

{ ID id-RequestedSRSTransmissionCharacteristics	CRITICALITY ignore	TYPE RequestedSRSTransmissionCharacteristics	PRESENCE optional}|

{ ID id-UEReportingInformation					CRITICALITY ignore	TYPE UEReportingInformation							PRESENCE optional}|

{ ID id-SRSPosRRCInactiveQueryIndication		CRITICALITY ignore	TYPE SRSPosRRCInactiveQueryIndication				PRESENCE optional},

...

}

-- **************************************************************

--

##### -- Positioning Information Response

--

-- **************************************************************

PositioningInformationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningInformationResponseIEs} },

...

}

PositioningInformationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-SRSConfiguration			CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-SFNInitialisationTime		CRITICALITY ignore	TYPE RelativeTime1900			PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional }|

{ ID id-SRSPosRRCInactiveConfig		CRITICALITY ignore	TYPE SRSPosRRCInactiveConfig	PRESENCE optional},

...

}

-- **************************************************************

--

##### -- Positioning Information Failure

--

-- **************************************************************

PositioningInformationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningInformationFailureIEs} },

...

}

PositioningInformationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- POSITIONING ACTIVATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Activation Request

--

-- **************************************************************

PositioningActivationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationRequestIEs} },

...

}

PositioningActivationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-SRSType					CRITICALITY reject	TYPE SRSType					PRESENCE mandatory	}|

{ ID id-ActivationTime			CRITICALITY ignore	TYPE RelativeTime1900		PRESENCE optional	},

...

}

SRSType ::= CHOICE {

semipersistentSRS				SemipersistentSRS,

aperiodicSRS					AperiodicSRS,

choice-extension				ProtocolIE-SingleContainer { { SRSType-ExtIEs} }

}

SRSType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SemipersistentSRS ::= SEQUENCE {

sRSResourceSetID			SRSResourceSetID,

sRSSpatialRelation			SpatialRelationInfo	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {SemipersistentSRS-ExtIEs} } OPTIONAL,

...

}

SemipersistentSRS-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SRSSpatialRelationPerSRSResource	CRITICALITY ignore	EXTENSION SpatialRelationPerSRSResource PRESENCE optional},

...

}

AperiodicSRS ::= SEQUENCE {

aperiodic					ENUMERATED {true, ...},

sRSResourceTrigger			SRSResourceTrigger		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {AperiodicSRS-ExtIEs} } OPTIONAL,

...

}

AperiodicSRS-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

##### -- Positioning Activation Response

--

-- **************************************************************

PositioningActivationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationResponseIEs} },

...

}

PositioningActivationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-SystemFrameNumber		CRITICALITY ignore	TYPE SystemFrameNumber			PRESENCE optional }|

{ ID id-SlotNumber				CRITICALITY ignore	TYPE SlotNumber					PRESENCE optional }|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

##### -- Positioning Activation Failure

--

-- **************************************************************

PositioningActivationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningActivationFailureIEs} },

...

}

PositioningActivationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- POSITIONING DEACTIVATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Deactivation

--

-- **************************************************************

PositioningDeactivation ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningDeactivationIEs} },

...

}

PositioningDeactivationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-AbortTransmission		CRITICALITY ignore	TYPE AbortTransmission			PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- POSITIONING INFORMATION UPDATE PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Information Update

--

-- **************************************************************

PositioningInformationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { PositioningInformationUpdateIEs} },

...

}

PositioningInformationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-SRSConfiguration		CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-SFNInitialisationTime	CRITICALITY ignore	TYPE RelativeTime1900			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- E-CID Measurement Initiation Request

--

-- **************************************************************

E-CIDMeasurementInitiationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationRequest-IEs}},

...

}

E-CIDMeasurementInitiationRequest-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID			CRITICALITY reject	TYPE LMF-UE-MeasurementID				PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID			CRITICALITY reject	TYPE RAN-UE-MeasurementID				PRESENCE mandatory	}|

{ ID id-E-CID-ReportCharacteristics		CRITICALITY reject	TYPE E-CID-ReportCharacteristics		PRESENCE mandatory	}|

{ ID id-E-CID-MeasurementPeriodicity	CRITICALITY reject	TYPE MeasurementPeriodicity		PRESENCE conditional	}|

-- The above IE shall be present if the E-CID-ReportCharacteristics IE is set to “periodic” –-

{ ID id-E-CID-MeasurementQuantities		CRITICALITY reject	TYPE E-CID-MeasurementQuantities	PRESENCE mandatory}|

{ ID id-PosMeasurementPeriodicityNR-AoA	CRITICALITY reject	TYPE PosMeasurementPeriodicityNR-AoA		PRESENCE conditional},

-- The IE shall be present if the E-CID-ReportCharacteristics IE is set to “periodic” and the E-CID-MeasurementQuantities-Item IE in the E-CID-MeasurementQuantities IE is set to the value "angleOfArrivalNR"--

...

}

-- **************************************************************

--

##### -- E-CID Measurement Initiation Response

--

-- **************************************************************

E-CIDMeasurementInitiationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationResponse-IEs}},

...

}

E-CIDMeasurementInitiationResponse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID		CRITICALITY reject	TYPE LMF-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID		CRITICALITY reject	TYPE RAN-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult		PRESENCE optional}|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID				PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

##### -- E-CID Measurement Initiation Failure

--

-- **************************************************************

E-CIDMeasurementInitiationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementInitiationFailure-IEs}},

...

}

E-CIDMeasurementInitiationFailure-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID		CRITICALITY reject	TYPE LMF-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID		CRITICALITY reject	TYPE RAN-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT FAILURE INDICATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- E-CID Measurement Failure Indication

--

-- **************************************************************

E-CIDMeasurementFailureIndication ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementFailureIndication-IEs}},

...

}

E-CIDMeasurementFailureIndication-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID		CRITICALITY reject	TYPE LMF-UE-MeasurementID		PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID		CRITICALITY reject	TYPE RAN-UE-MeasurementID		PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT REPORT PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- E-CID Measurement Report

--

-- **************************************************************

E-CIDMeasurementReport ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementReport-IEs}},

...

}

E-CIDMeasurementReport-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID		CRITICALITY reject	TYPE LMF-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID		CRITICALITY reject	TYPE RAN-UE-MeasurementID			PRESENCE mandatory	}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult		PRESENCE mandatory }|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID				PRESENCE optional},

...

}

-- **************************************************************

--

#### -- E-CID MEASUREMENT TERMINATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- E-CID Measurement Termination Command

--

-- **************************************************************

E-CIDMeasurementTerminationCommand ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementTerminationCommand-IEs}},

...

}

E-CIDMeasurementTerminationCommand-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-LMF-UE-MeasurementID		CRITICALITY reject	TYPE LMF-UE-MeasurementID				PRESENCE mandatory	}|

{ ID id-RAN-UE-MeasurementID		CRITICALITY reject	TYPE RAN-UE-MeasurementID				PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- BROADCAST CONTEXT SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT SETUP REQUEST

--

-- **************************************************************

BroadcastContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextSetupRequestIEs} },

...

}

BroadcastContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE	GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-MBS-Session-ID					CRITICALITY reject 	TYPE	MBS-Session-ID					PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea					CRITICALITY reject 	TYPE	MBS-ServiceArea					PRESENCE optional   }|

{ ID id-MBS-CUtoDURRCInformation		CRITICALITY reject	TYPE	MBS-CUtoDURRCInformation		PRESENCE mandatory	}|

{ ID id-SNSSAI							CRITICALITY reject	TYPE	SNSSAI							PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-ToBeSetup-List	CRITICALITY reject	TYPE	BroadcastMRBs-ToBeSetup-List	PRESENCE mandatory	},

...

}

BroadcastMRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeSetup-ItemIEs} }

BroadcastMRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeSetup-Item	CRITICALITY reject	TYPE 	BroadcastMRBs-ToBeSetup-Item	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT SETUP RESPONSE

--

-- **************************************************************

BroadcastContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextSetupResponseIEs} },

...

}

BroadcastContextSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-Setup-List			CRITICALITY reject TYPE BroadcastMRBs-Setup-List				PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-FailedToBeSetup-List	CRITICALITY ignore TYPE BroadcastMRBs-FailedToBeSetup-List	PRESENCE optional	}|

{ ID id-BroadcastAreaScope					CRITICALITY ignore TYPE BroadcastAreaScope						PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore TYPE CriticalityDiagnostics					PRESENCE optional	},

...

}

BroadcastMRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-Setup-ItemIEs} }

BroadcastMRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-FailedToBeSetup-ItemIEs} }

BroadcastMRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-Setup-Item			CRITICALITY reject	TYPE BroadcastMRBs-Setup-Item			PRESENCE mandatory},

...

}

BroadcastMRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-FailedToBeSetup-Item	CRITICALITY ignore	TYPE BroadcastMRBs-FailedToBeSetup-Item	PRESENCE mandatory},	...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT SETUP FAILURE

--

-- **************************************************************

BroadcastContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextSetupFailureIEs} },

...

}

BroadcastContextSetupFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID			CRITICALITY ignore	TYPE GNB-DU-UE-F1AP-ID			PRESENCE optional	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BROADCAST CONTEXT RELEASE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT RELEASE COMMAND

--

-- **************************************************************

BroadcastContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextReleaseCommandIEs} },

...

}

BroadcastContextReleaseCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT RELEASE COMPLETE

--

-- **************************************************************

BroadcastContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextReleaseCompleteIEs} },

...

}

BroadcastContextReleaseCompleteIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BROADCAST CONTEXT RELEASE REQUEST ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT RELEASE REQUEST

--

-- **************************************************************

BroadcastContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ BroadcastContextReleaseRequestIEs}},

...

}

BroadcastContextReleaseRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- BROADCAST CONTEXT MODIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

BroadcastContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextModificationRequestIEs} },

...

}

BroadcastContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE MBS-ServiceArea						PRESENCE optional		}|

{ ID id-MBS-CUtoDURRCInformation			CRITICALITY reject	TYPE MBS-CUtoDURRCInformation				PRESENCE mandatory	}|

{ ID id-BroadcastMRBs-ToBeSetupMod-List		CRITICALITY reject	TYPE BroadcastMRBs-ToBeSetupMod-List	PRESENCE optional	}|

{ ID id-BroadcastMRBs-ToBeModified-List		CRITICALITY reject	TYPE BroadcastMRBs-ToBeModified-List	PRESENCE optional	}|

{ ID id-BroadcastMRBs-ToBeReleased-List		CRITICALITY reject	TYPE BroadcastMRBs-ToBeReleased-List	PRESENCE optional	},

...

}

BroadcastMRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeSetupMod-ItemIEs} }

BroadcastMRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeModified-ItemIEs} }

BroadcastMRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-ToBeReleased-ItemIEs} }

BroadcastMRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE BroadcastMRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

BroadcastMRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeModified-Item		CRITICALITY reject	TYPE BroadcastMRBs-ToBeModified-Item		PRESENCE mandatory},

...

}

BroadcastMRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-ToBeReleased-Item		CRITICALITY reject	TYPE BroadcastMRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

BroadcastContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextModificationResponseIEs} },

...

}

BroadcastContextModificationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID						CRITICALITY reject TYPE GNB-CU-MBS-F1AP-ID								PRESENCE mandatory}|

{ ID id-gNB-DU-MBS-F1AP-ID						CRITICALITY reject TYPE GNB-DU-MBS-F1AP-ID								PRESENCE mandatory}|

{ ID id-BroadcastMRBs-SetupMod-List				CRITICALITY reject TYPE BroadcastMRBs-SetupMod-List					PRESENCE optional}|

{ ID id-BroadcastMRBs-FailedToBeSetupMod-List	CRITICALITY ignore TYPE BroadcastMRBs-FailedToBeSetupMod-List	PRESENCE optional}|

{ ID id-BroadcastMRBs-Modified-List				CRITICALITY reject TYPE BroadcastMRBs-Modified-List					PRESENCE optional}|

{ ID id-BroadcastMRBs-FailedToBeModified-List	CRITICALITY ignore TYPE BroadcastMRBs-FailedToBeModified-List	PRESENCE optional}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional}|

{ ID id-BroadcastAreaScope						CRITICALITY ignore TYPE BroadcastAreaScope								PRESENCE optional},

...

}

BroadcastMRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-SetupMod-ItemIEs} }

BroadcastMRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-FailedToBeSetupMod-ItemIEs} }

BroadcastMRBs-Modified-List::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-Modified-ItemIEs } }

BroadcastMRBs-FailedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { BroadcastMRBs-FailedToBeModified-ItemIEs} }

BroadcastMRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-SetupMod-Item				CRITICALITY		reject	TYPE BroadcastMRBs-SetupMod-Item			PRESENCE mandatory},

...

}

BroadcastMRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-FailedToBeSetupMod-Item	CRITICALITY		ignore	TYPE BroadcastMRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

BroadcastMRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-Modified-Item				CRITICALITY		reject	TYPE BroadcastMRBs-Modified-Item			PRESENCE mandatory},

...

}

BroadcastMRBs-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-BroadcastMRBs-FailedToBeModified-Item	CRITICALITY 	ignore	TYPE BroadcastMRBs-FailedToBeModified-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- BROADCAST CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

BroadcastContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BroadcastContextModificationFailureIEs} },

...

}

BroadcastContextModificationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- Multicast Group Paging PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Multicast Group Paging

--

-- **************************************************************

MulticastGroupPaging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastGroupPagingIEs}},

...

}

MulticastGroupPagingIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MBS-Session-ID					CRITICALITY reject	TYPE MBS-Session-ID							PRESENCE mandatory	}|

{ ID id-UEIdentity-List-For-Paging-List	CRITICALITY ignore	TYPE UEIdentity-List-For-Paging-List		PRESENCE optional		}|

{ ID id-MC-PagingCell-List				CRITICALITY ignore	TYPE MC-PagingCell-list						PRESENCE optional		},

...

}

UEIdentity-List-For-Paging-List	 ::= SEQUENCE (SIZE(1.. maxnoofUEIDforPaging)) OF ProtocolIE-SingleContainer { { UEIdentity-List-For-Paging-ItemIEs } }

UEIdentity-List-For-Paging-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UEIdentity-List-For-Paging-Item	CRITICALITY ignore	TYPE UEIdentity-List-For-Paging-Item 			PRESENCE mandatory }	,

...

}

MC-PagingCell-list::= SEQUENCE (SIZE(1.. maxnoofPagingCells)) OF ProtocolIE-SingleContainer { { MC-PagingCell-ItemIEs } }

MC-PagingCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MC-PagingCell-Item		CRITICALITY ignore	TYPE MC-PagingCell-Item			PRESENCE mandatory}	,

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP REQUEST

--

-- **************************************************************

MulticastContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupRequestIEs}},

...

}

MulticastContextSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory  }|

{ ID id-MBS-Session-ID					CRITICALITY reject 	TYPE	MBS-Session-ID						PRESENCE mandatory  }|

{ ID id-MBS-ServiceArea					CRITICALITY reject TYPE	MBS-ServiceArea							PRESENCE optional   }|

{ ID id-SNSSAI							CRITICALITY reject	TYPE	SNSSAI								PRESENCE mandatory  }|

{ ID id-MulticastMRBs-ToBeSetup-List	CRITICALITY reject	TYPE	MulticastMRBs-ToBeSetup-List		PRESENCE mandatory  },

...

}

MulticastMRBs-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeSetup-ItemIEs} }

MulticastMRBs-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeSetup-Item	CRITICALITY reject	TYPE 	MulticastMRBs-ToBeSetup-Item	PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP RESPONSE

--

-- **************************************************************

MulticastContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupResponseIEs}},

...

}

MulticastContextSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-MulticastMRBs-Setup-List			CRITICALITY reject TYPE MulticastMRBs-Setup-List					PRESENCE mandatory	}|

{ ID id-MulticastMRBs-FailedToBeSetup-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeSetup-List 	PRESENCE optional	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore TYPE CriticalityDiagnostics						PRESENCE optional	},

...

}

MulticastMRBs-Setup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-Setup-ItemIEs} }

MulticastMRBs-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeSetup-ItemIEs} }

MulticastMRBs-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-Setup-Item			CRITICALITY reject	TYPE MulticastMRBs-Setup-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeSetup-Item		CRITICALITY ignore	TYPE MulticastMRBs-FailedToBeSetup-Item	PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT SETUP FAILURE

--

-- **************************************************************

MulticastContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextSetupFailureIEs}},

...

}

MulticastContextSetupFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY ignore	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE optional	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT RELEASE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT RELEASE COMMAND

--

-- **************************************************************

MulticastContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextReleaseCommandIEs}},

...

}

MulticastContextReleaseCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT RELEASE COMPLETE

--

-- **************************************************************

MulticastContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextReleaseCompleteIEs}},

...

}

MulticastContextReleaseCompleteIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT RELEASE REQUEST ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT RELEASE REQUEST

--

-- **************************************************************

MulticastContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextReleaseRequestIEs}},

...

}

MulticastContextReleaseRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- MULTICAST CONTEXT MODIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

MulticastContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationRequestIEs}},

...

}

MulticastContextModificationRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID						PRESENCE mandatory }|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE MBS-ServiceArea						PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeSetupMod-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeSetupMod-List	PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeModified-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeModified-List	PRESENCE optional  }|

{ ID id-MulticastMRBs-ToBeReleased-List		CRITICALITY reject	TYPE MulticastMRBs-ToBeReleased-List	PRESENCE optional  },

...

}

MulticastMRBs-ToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeSetupMod-ItemIEs} }

MulticastMRBs-ToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeSetupMod-Item		CRITICALITY reject	TYPE MulticastMRBs-ToBeSetupMod-Item		PRESENCE mandatory},

...

}

MulticastMRBs-ToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeModified-ItemIEs} }

MulticastMRBs-ToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeModified-Item		CRITICALITY reject	TYPE MulticastMRBs-ToBeModified-Item		PRESENCE mandatory},

...

}

MulticastMRBs-ToBeReleased-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-ToBeReleased-ItemIEs} }

MulticastMRBs-ToBeReleased-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-ToBeReleased-Item		CRITICALITY reject	TYPE MulticastMRBs-ToBeReleased-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

MulticastContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationResponseIEs}},

...

}

MulticastContextModificationResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID						CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID								PRESENCE mandatory }|

{ ID id-gNB-DU-MBS-F1AP-ID						CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID								PRESENCE mandatory }|

{ ID id-MulticastMRBs-SetupMod-List			CRITICALITY reject TYPE MulticastMRBs-SetupMod-List						PRESENCE optional  }|

{ ID id-MulticastMRBs-FailedToBeSetupMod-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeSetupMod-List	PRESENCE optional  }|

{ ID id-MulticastMRBs-Modified-List			CRITICALITY reject TYPE MulticastMRBs-Modified-List						PRESENCE optional  }|

{ ID id-MulticastMRBs-FailedToBeModified-List	CRITICALITY ignore TYPE MulticastMRBs-FailedToBeModified-List	PRESENCE optional  }|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional  },

...

}

MulticastMRBs-SetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-SetupMod-ItemIEs} }

MulticastMRBs-SetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-SetupMod-Item				CRITICALITY		reject	TYPE MulticastMRBs-SetupMod-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeSetupMod-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeSetupMod-ItemIEs} }

MulticastMRBs-FailedToBeSetupMod-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeSetupMod-Item	CRITICALITY		ignore	TYPE MulticastMRBs-FailedToBeSetupMod-Item		PRESENCE mandatory},

...

}

MulticastMRBs-Modified-List::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-Modified-ItemIEs } }

MulticastMRBs-Modified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-Modified-Item				CRITICALITY		reject	TYPE MulticastMRBs-Modified-Item			PRESENCE mandatory},

...

}

MulticastMRBs-FailedToBeModified-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastMRBs-FailedToBeModified-ItemIEs} }

MulticastMRBs-FailedToBeModified-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastMRBs-FailedToBeModified-Item	CRITICALITY 	ignore	TYPE MulticastMRBs-FailedToBeModified-Item		PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

MulticastContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastContextModificationFailureIEs}},

...

}

MulticastContextModificationFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID				PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MULTICAST DISTRIBUTION SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST DISTRIBUTION SETUP REQUEST

--

-- **************************************************************

MulticastDistributionSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastDistributionSetupRequestIEs}},

...

}

MulticastDistributionSetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-MBSMulticastF1UContextDescriptor	CRITICALITY reject	TYPE MBSMulticastF1UContextDescriptor		PRESENCE mandatory	}|

{ ID id-MulticastF1UContext-ToBeSetup-List	CRITICALITY reject	TYPE MulticastF1UContext-ToBeSetup-List	PRESENCE mandatory	},

...

}

MulticastF1UContext-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF

ProtocolIE-SingleContainer { { MulticastF1UContext-ToBeSetup-ItemIEs} }

MulticastF1UContext-ToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastF1UContext-ToBeSetup-Item				CRITICALITY		reject	TYPE MulticastF1UContext-ToBeSetup-Item			PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST DISTRIBUTION SETUP RESPONSE

--

-- **************************************************************

MulticastDistributionSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastDistributionSetupResponseIEs}},

...

}

MulticastDistributionSetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID							CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID									PRESENCE mandatory}|

{ ID id-gNB-DU-MBS-F1AP-ID							CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID									PRESENCE mandatory}|

{ ID id-MBSMulticastF1UContextDescriptor			CRITICALITY reject	TYPE MBSMulticastF1UContextDescriptor				PRESENCE mandatory}|

{ ID id-MulticastF1UContext-Setup-List				CRITICALITY reject	TYPE MulticastF1UContext-Setup-List					PRESENCE mandatory}|

{ ID id-MulticastF1UContext-FailedToBeSetup-List	CRITICALITY ignore	TYPE MulticastF1UContext-FailedToBeSetup-List		PRESENCE optional}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics								PRESENCE optional }|

{ ID id-MulticastF1UContextReferenceCU				CRITICALITY reject	TYPE MulticastF1UContextReferenceCU					PRESENCE mandatory},

...

}

MulticastF1UContext-Setup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF ProtocolIE-SingleContainer { { MulticastF1UContext-Setup-ItemIEs} }

MulticastF1UContext-Setup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastF1UContext-Setup-Item				CRITICALITY		reject	TYPE MulticastF1UContext-Setup-Item			PRESENCE mandatory},

...

}

MulticastF1UContext-FailedToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF 
																ProtocolIE-SingleContainer { { MulticastF1UContext-FailedToBeSetup-ItemIEs} }

MulticastF1UContext-FailedToBeSetup-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-MulticastF1UContext-FailedToBeSetup-Item	CRITICALITY	 ignore	TYPE MulticastF1UContext-FailedToBeSetup-Item	 PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- MULTICAST DISTRIBUTION SETUP FAILURE

--

-- **************************************************************

MulticastDistributionSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastDistributionSetupFailureIEs}},

...

}

MulticastDistributionSetupFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY ignore	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE optional	}|

{ ID id-MBSMulticastF1UContextDescriptor	CRITICALITY reject	TYPE MBSMulticastF1UContextDescriptor		PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause										PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MULTICAST DISTRIBUTION RELEASE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MULTICAST DISTRIBUTION RELEASE COMMAND

--

-- **************************************************************

MulticastDistributionReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastDistributionReleaseCommandIEs}},

...

}

MulticastDistributionReleaseCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-MBSMulticastF1UContextDescriptor	CRITICALITY reject	TYPE MBSMulticastF1UContextDescriptor		PRESENCE mandatory	}|

{ ID id-Cause								CRITICALITY ignore	TYPE Cause										PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- MULTICAST DISTRIBUTION RELEASE COMPLETE

--

-- **************************************************************

MulticastDistributionReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MulticastDistributionReleaseCompleteIEs}},

...

}

MulticastDistributionReleaseCompleteIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-gNB-DU-MBS-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-MBS-F1AP-ID							PRESENCE mandatory	}|

{ ID id-MBSMulticastF1UContextDescriptor	CRITICALITY reject	TYPE MBSMulticastF1UContextDescriptor		PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- PDC MEASUREMENT PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PDC Measurement Initiation Request

--

-- **************************************************************

PDCMeasurementInitiationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PDCMeasurementInitiationRequest-IEs}},

...

}

PDCMeasurementInitiationRequest-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID				CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-PDC-MeasID				CRITICALITY reject	TYPE RAN-UE-PDC-MeasID					PRESENCE mandatory	}|

{ ID id-PDCReportType					CRITICALITY reject	TYPE PDCReportType						PRESENCE mandatory	}|

{ ID id-PDCMeasurementPeriodicity		CRITICALITY reject	TYPE PDCMeasurementPeriodicity			PRESENCE conditional	}|

-- The above IE shall be present if the PDCReportType IE is set to “periodic” –-

{ ID id-PDCMeasurementQuantities		CRITICALITY reject	TYPE PDCMeasurementQuantities			PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- PDC Measurement Initiation Response

--

-- **************************************************************

PDCMeasurementInitiationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PDCMeasurementInitiationResponse-IEs}},

...

}

PDCMeasurementInitiationResponse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-RAN-UE-PDC-MeasID			CRITICALITY reject	TYPE RAN-UE-PDC-MeasID			PRESENCE mandatory	}|

{ ID id-PDCMeasurementResult		CRITICALITY ignore	TYPE PDCMeasurementResult		PRESENCE optional		}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- PDC Measurement Initiation Failure

--

-- **************************************************************

PDCMeasurementInitiationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{PDCMeasurementInitiationFailure-IEs}},

...

}

PDCMeasurementInitiationFailure-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-RAN-UE-PDC-MeasID			CRITICALITY ignore	TYPE RAN-UE-PDC-MeasID				PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- PDC MEASUREMENT REPORT PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PDC Measurement Report

--

-- **************************************************************

PDCMeasurementReport ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{PDCMeasurementReport-IEs}},

...

}

PDCMeasurementReport-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-RAN-UE-PDC-MeasID			CRITICALITY reject	TYPE RAN-UE-PDC-MeasID			PRESENCE mandatory	}|

{ ID id-PDCMeasurementResult		CRITICALITY ignore	TYPE PDCMeasurementResult		PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- PDC MEASUREMENT TERMINATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PDC Measurement Termination

--

-- **************************************************************

PDCMeasurementTerminationCommand ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PDCMeasurementTerminationCommand-IEs} },

...

}

PDCMeasurementTerminationCommand-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-RAN-UE-PDC-MeasID			CRITICALITY	ignore	TYPE RAN-UE-PDC-MeasID			PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PDC MEASUREMENT FAILURE INDICATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PDC Measurement Failure Indication

--

-- **************************************************************

PDCMeasurementFailureIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { PDCMeasurementFailureIndication-IEs} },

...

}

PDCMeasurementFailureIndication-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-RAN-UE-PDC-MeasID			CRITICALITY ignore	TYPE RAN-UE-PDC-MeasID			PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- PPS CONFIGURATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- PRS CONFIGURATION REQUEST

--

-- **************************************************************

PRSConfigurationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{PRSConfigurationRequest-IEs}},

...

}

PRSConfigurationRequest-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID						PRESENCE mandatory }|

{ ID id-PRSConfigRequestType	CRITICALITY reject	TYPE PRSConfigRequestType				PRESENCE mandatory }|

{ ID id-PRSTRPList				CRITICALITY ignore	TYPE PRSTRPList							PRESENCE mandatory },

...

}

-- **************************************************************

--

##### -- PRS CONFIGURATION RESPONSE

--

-- **************************************************************

PRSConfigurationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ PRSConfigurationResponse-IEs}},

...

}

PRSConfigurationResponse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory}|

{ ID id-PRSTransmissionTRPList	CRITICALITY ignore	TYPE PRSTransmissionTRPList			PRESENCE optional}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional},

...

}

-- **************************************************************

--

##### -- PRS CONFIGURATION FAILURE

--

-- **************************************************************

PRSConfigurationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ PRSConfigurationFailure-IEs}},

...

}

PRSConfigurationFailure-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID				PRESENCE mandatory}|

{ ID id-Cause	CRITICALITY ignore	TYPE Cause	PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- MEASUREMENT PRECONFIGURATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Positioning Preconfiguration Required

--

-- **************************************************************

MeasurementPreconfigurationRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ MeasurementPreconfigurationRequired-IEs}},

...

}

MeasurementPreconfigurationRequired-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID	CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID	PRESENCE mandatory}|

{ ID id-gNB-DU-UE-F1AP-ID	CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID	PRESENCE mandatory}|

{ ID id-TRP-PRS-Info-List	CRITICALITY ignore	TYPE TRP-PRS-Info-List	PRESENCE mandatory},

...

}

-- **************************************************************

--

##### -- Positioning Preconfiguration Confirm

--

-- **************************************************************

MeasurementPreconfigurationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MeasurementPreconfigurationConfirm-IEs} },

...

}

MeasurementPreconfigurationConfirm-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-PosMeasGapPreConfigList		CRITICALITY ignore	TYPE PosMeasGapPreConfigList		PRESENCE optional }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional },

...

}

-- **************************************************************

--

##### -- Positioning Preconfiguration Refuse

--

-- **************************************************************

MeasurementPreconfigurationRefuse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MeasurementPreconfigurationRefuse-IEs} },

...

}

MeasurementPreconfigurationRefuse-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional },

...

}

-- **************************************************************

--

#### -- MEASUREMENT ACTIVATION PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- Measurement Activation

--

-- **************************************************************

MeasurementActivation ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MeasurementActivation-IEs} },

...

}

MeasurementActivation-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-ActivationRequestType		CRITICALITY reject	TYPE ActivationRequestType 			PRESENCE mandatory}|

{ ID id-PRS-Measurement-Info-List	CRITICALITY ignore	TYPE PRS-Measurement-Info-List		PRESENCE optional},

...

}

-- **************************************************************

--

#### -- QOE INFORMATION TRANSFER

--

-- **************************************************************

-- **************************************************************

--

##### -- QoE Information Transfer

--

-- **************************************************************

QoEInformationTransfer ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{QoEInformationTransfer-IEs}},

...

}

QoEInformationTransfer-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID					PRESENCE mandatory	}|

{ ID id-QoEInformation				CRITICALITY ignore	TYPE QoEInformation						PRESENCE optional	},

...

}

-- **************************************************************

--

##### -- Positioning System information Delivery Command

--

-- **************************************************************

PosSystemInformationDeliveryCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PosSystemInformationDeliveryCommandIEs}},

...

}

PosSystemInformationDeliveryCommandIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TransactionID			CRITICALITY reject	TYPE TransactionID					PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-PosSItypeList			CRITICALITY reject	TYPE PosSItypeList					PRESENCE mandatory	}|

{ ID id-ConfirmedUEID 			CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	},

...

}

END

-- ASN1STOP

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-gNB-CUSystemInformation,

id-HandoverPreparationInformation,

id-TAISliceSupportList,

id-RANAC,

id-BearerTypeChange,

id-Cell-Direction,

id-Cell-Type,

id-CellGroupConfig,

id-AvailablePLMNList,

id-PDUSessionID,

id-ULPDUSessionAggregateMaximumBitRate,

id-DC-Based-Duplication-Configured,

id-DC-Based-Duplication-Activation,

id-Duplication-Activation,

id-DLPDCPSNLength,

id-ULPDCPSNLength,

id-RLC-Status,

id-MeasurementTimingConfiguration,

id-DRB-Information,

id-QoSFlowMappingIndication,

id-ServingCellMO,

id-RLCMode,

id-ExtendedServedPLMNs-List,

id-ExtendedAvailablePLMN-List,

id-DRX-LongCycleStartOffset,

id-SelectedBandCombinationIndex,

id-SelectedFeatureSetEntryIndex,

id-Ph-InfoSCG,

id-latest-RRC-Version-Enhanced,

id-RequestedBandCombinationIndex,

id-RequestedFeatureSetEntryIndex,

id-DRX-Config,

id-UEAssistanceInformation,

id-PDCCH-BlindDetectionSCG,

id-Requested-PDCCH-BlindDetectionSCG,

id-BPLMN-ID-Info-List,

id-NotificationInformation,

id-TNLAssociationTransportLayerAddressgNBDU,

id-portNumber,

id-AdditionalSIBMessageList,

id-IgnorePRACHConfiguration,

id-CG-Config,

id-Ph-InfoMCG,

id-AggressorgNBSetID,

id-VictimgNBSetID,

id-MeasGapSharingConfig,

id-systemInformationAreaID,

id-areaScope,

id-IntendedTDD-DL-ULConfig,

id-QosMonitoringRequest,

id-BHInfo,

id-IAB-Info-IAB-DU,

id-IAB-Info-IAB-donor-CU,

id-IAB-Barred,

id-SIB12-message,

id-SIB13-message,

id-SIB14-message,

id-UEAssistanceInformationEUTRA,

id-SL-PHY-MAC-RLC-Config,

id-SL-ConfigDedicatedEUTRA-Info,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRPRACHConfig,

id-TDD-UL-DLConfigCommonNR,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-TSCTrafficCharacteristics,

id-AdditionalPDCPDuplicationTNL-List,

id-RLCDuplicationInformation,

id-AdditionalDuplicationIndication,

id-mdtConfiguration,

id-TraceCollectionEntityURI,

id-NID,

id-NPNSupportInfo,

id-NPNBroadcastInformation,

id-AvailableSNPN-ID-List,

id-SIB10-message,

id-RequestedP-MaxFR2,

id-DLCarrierList,

id-ExtendedTAISliceSupportList,

id-E-CID-MeasurementQuantities-Item,

id-ConfiguredTACIndication,

id-NRCGI,

id-SFN-Offset,

id-TransmissionStopIndicator,

id-SrsFrequency,

id-EstimatedArrivalProbability,

id-Supported-MBS-FSA-ID-List,

id-TRPType,

id-SRSSpatialRelationPerSRSResource,

id-MBS-Broadcast-NeighbourCellList,

id-PDCPTerminatingNodeDLTNLAddrInfo,

id-ENBDLTNLAddress,

id-PRS-Resource-ID,

id-LocationMeasurementInformation,

id-SliceRadioResourceStatus,

id-CompositeAvailableCapacity-SUL,

id-NR-U,

id-NR-U-Channel-List,

id-MIMOPRBusageInformation,

id-IngressNonF1terminatingTopologyIndicator,

id-NonF1terminatingTopologyIndicator,

id-EgressNonF1terminatingTopologyIndicator,

id-rBSetConfiguration,

id-frequency-Domain-HSNA-Configuration-List,

id-child-IAB-Nodes-NA-Resource-List,

id-Parent-IAB-Nodes-NA-Resource-Configuration-List,

id-uL-FreqInfo,

id-uL-Transmission-Bandwidth,

id-dL-FreqInfo,

id-dL-Transmission-Bandwidth,

id-uL-NR-Carrier-List,

id-dL-NR-Carrier-List,

id-nRFreqInfo,

id-transmission-Bandwidth,

id-nR-Carrier-List,

id-permutation,

id-M5ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-SurvivalTime,

id-PDCMeasurementQuantities-Item,

id-OnDemandPRS,

id-AoA-SearchWindow,

id-ZoAInformation,

id-ARPLocationInfo,

id-ARP-ID,

id-MultipleULAoA,

id-UL-SRS-RSRPP,

id-SRSResourcetype,

id-ExtendedAdditionalPathList,

id-LoS-NLoSInformation,

id-NumberOfTRPRxTEG,

id-NumberOfTRPRxTxTEG,

id-TRPTxTEGAssociation,

id-TRPTEGInformation,

id-TRPRx-TEGInformation,

id-TRPBeamAntennaInformation,

id-Redcap-Bcast-Information,

id-NR-TADV,

id-SDT-MAC-PHY-CG-Config,

id-CG-SDTindicatorSetup,

id-CG-SDTindicatorMod,

id-SDTRLCBearerConfiguration,

id-SRBMappingInfo,

id-DRBMappingInfo,

id-LastUsedCellIndication,

id-SIB17-message,

id-MUSIM-GapConfig,

id-SIB20-message,

id-pathPower,

id-DU-RX-MT-RX-Extend,

id-DU-TX-MT-TX-Extend,

id-DU-RX-MT-TX-Extend,

id-DU-TX-MT-RX-Extend,

id-TAINSAGSupportList,

id-SL-RLC-ChannelToAddModList,

id-SIB15-message,

id-InterFrequencyConfig-NoGap,

id-MBSInterestIndication,

id-L571Info,

id-L1151Info,

id-SCS-480,

id-SCS-960,

id-SRSPortIndex,

id-PEISubgroupingSupportIndication,

id-NeedForGapsInfoNR,

id-NeedForGapNCSGInfoNR,

id-NeedForGapNCSGInfoEUTRA,

id-Source-MRB-ID,

id-RedCapIndication,

id-UL-GapFR2-Config,

id-ConfigRestrictInfoDAPS,

id-MulticastF1UContextReferenceCU,

id-TwoPHRModeMCG,

id-TwoPHRModeSCG,

id-ncd-SSB-RedCapInitialBWP-SDT,

id-nrofSymbolsExtended,

id-repetitionFactorExtended,

id-startRBHopping,

id-startRBIndex,

id-transmissionCombn8,

id-ServCellInfoList,

id-Preconfigured-measurement-GAP-Request,

id-BWP-Id,

id-SL-PHY-MAC-RLC-ConfigExt,

id-Transmission-Bandwidth-asymmetric,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

maxnoofAdditionalSIBs,

maxnoofUACPLMNs,

maxnoofUACperPLMN,

maxCellingNBDU,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofslots,

maxnoofNonUPTrafficMappings,

maxnoofServingCells,

maxnoofServedCellsIAB,

maxnoofChildIABNodes,

maxnoofIABSTCInfo,

maxnoofSymbols,

maxnoofDUFSlots,

maxnoofHSNASlots,

maxnoofEgressLinks,

maxnoofMappingEntries,

maxnoofDSInfo,

maxnoofQoSParaSets,

maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofPhysicalResourceBlocks,

maxnoofPhysicalResourceBlocks-1,

maxnoofPRACHconfigs,

maxnoofRACHReports,

maxnoofRLFReports,

maxnoofAdditionalPDCPDuplicationTNL,

maxnoofRLCDuplicationState,

maxnoofCHOcells,

maxnoofMDTPLMNs,

maxnoofCAGsupported,

maxnoofNIDsupported,

maxnoofExtSliceItems,

maxnoofPosMeas,

maxnoofTRPInfoTypes,

maxnoofSRSTriggerStates,

maxnoofSpatialRelations,

maxnoBcastCell,

maxnoofTRPs,

maxnoofAngleInfo,

maxnooflcs-gcs-translation,

maxnoofPath,

maxnoofMeasE-CID,

maxnoofSSBs,

maxnoSRS-ResourceSets,

maxnoSRS-ResourcePerSet,

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxnoofPRS-ResourceSets,

maxnoofPRS-ResourcesPerSet,

maxNoOfMeasTRPs,

maxnoofPRSresourceSets,

maxnoofPRSresources,

maxnoofSuccessfulHOReports,

maxnoofNR-UChannelIDs,

maxServedCellforSON,

maxNeighbourCellforSON,

maxAffectedCells,

maxnoofMBSQoSFlows,

maxnoofMBSFSAs,

maxnoofMBSAreaSessionIDs,

maxnoofMBSServiceAreaInformation,

maxnoofTAIforMBS,

maxnoofCellsforMBS,

maxnoofIABCongInd,

maxnoofBHRLCChannels,

maxnoofTLAsIAB,

maxnoofRBsetsPerCell,

maxnoofRBsetsPerCell-1,

maxnoofNeighbourNodeCellsIAB,

maxnoofMeasPDC,

maxnoARPs,

maxnoofULAoAs,

maxNoPathExtended,

maxnoTRPTEGs,

maxFreqLayers,

maxNumResourcesPerAngle,

maxnoAzimuthAngles,

maxnoElevationAngles,

maxnoofPRSTRPs,

maxnoofQoEInformation,

maxnoofUuRLCChannels,

maxnoofPC5RLCChannels,

maxnoofSMBRValues,

maxnoofMBSSessionsofUE,

maxnoofSLdestinations,

maxnoofNSAGs,

maxnoofSDTBearers,

maxnoofPosSITypes,

maxnoofMRBs,

maxNrofBWPs

FROM F1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM F1AP-CommonDataTypes

ProtocolExtensionContainer{},

F1AP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;

#### -- A

AbortTransmission ::= CHOICE {

sRSResourceSetID		SRSResourceSetID,

releaseALL				NULL,

choice-extension		ProtocolIE-SingleContainer { { AbortTransmission-ExtIEs } }

}

AbortTransmission-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

AccessPointPosition ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

latitude					INTEGER (0..8388607),

longitude					INTEGER (-8388608..8388607),

directionOfAltitude			ENUMERATED {height, depth},

altitude					INTEGER (0..32767),

uncertaintySemi-major		INTEGER (0..127),

uncertaintySemi-minor		INTEGER (0..127),

orientationOfMajorAxis		INTEGER (0..179),

uncertaintyAltitude			INTEGER (0..127),

confidence					INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { AccessPointPosition-ExtIEs} } OPTIONAL

}

AccessPointPosition-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Activated-Cells-to-be-Updated-List ::= SEQUENCE (SIZE(1..maxnoofServedCellsIAB)) OF Activated-Cells-to-be-Updated-List-Item

Activated-Cells-to-be-Updated-List-Item ::=	SEQUENCE{

nRCGI								NRCGI,

iAB-DU-Cell-Resource-Configuration-Mode-Info	IAB-DU-Cell-Resource-Configuration-Mode-Info,

iE-Extensions						ProtocolExtensionContainer { { Activated-Cells-to-be-Updated-List-Item-ExtIEs} } OPTIONAL

}

Activated-Cells-to-be-Updated-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ActivationRequestType ::= ENUMERATED {activate, deactivate, ...}

ActiveULBWP  ::= SEQUENCE {

locationAndBandwidth		INTEGER (0..37949,...),

subcarrierSpacing           ENUMERATED {kHz15, kHz30, kHz60, kHz120,..., kHz480, kHz960},

cyclicPrefix				ENUMERATED {normal, extended},

txDirectCurrentLocation		INTEGER (0..3301,...),

shift7dot5kHz				ENUMERATED {true, ...} OPTIONAL,

sRSConfig					SRSConfig,

iE-Extensions					ProtocolExtensionContainer { { ActiveULBWP-ExtIEs} } OPTIONAL

}

ActiveULBWP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalDuplicationIndication ::= ENUMERATED {

three,

four,

...

}

AdditionalPath-List::= SEQUENCE (SIZE(1..maxnoofPath)) OF AdditionalPath-Item

AdditionalPath-Item ::=SEQUENCE {

relativePathDelay	RelativePathDelay,

pathQuality			TRPMeasurementQuality 	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AdditionalPath-Item-ExtIEs } }	OPTIONAL

}

AdditionalPath-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MultipleULAoA	CRITICALITY ignore EXTENSION MultipleULAoA		PRESENCE optional}|

{ ID id-pathPower		CRITICALITY ignore EXTENSION UL-SRS-RSRPP		PRESENCE optional},

...

}

ExtendedAdditionalPathList ::= SEQUENCE (SIZE (1.. maxNoPathExtended)) OF ExtendedAdditionalPathList-Item

ExtendedAdditionalPathList-Item ::= SEQUENCE {

relativeTimeOfPath	RelativePathDelay,

pathQuality			TRPMeasurementQuality	OPTIONAL,

multipleULAoA		MultipleULAoA  			OPTIONAL,

pathPower			UL-SRS-RSRPP			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ExtendedAdditionalPathList-Item-ExtIEs} } OPTIONAL,

...

}

ExtendedAdditionalPathList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalPDCPDuplicationTNL-List ::= SEQUENCE (SIZE(1..maxnoofAdditionalPDCPDuplicationTNL)) OF AdditionalPDCPDuplicationTNL-Item

AdditionalPDCPDuplicationTNL-Item ::=SEQUENCE {

additionalPDCPDuplicationUPTNLInformation		UPTransportLayerInformation,

iE-Extensions	ProtocolExtensionContainer { { AdditionalPDCPDuplicationTNL-ItemExtIEs } }	OPTIONAL,

...

}

AdditionalPDCPDuplicationTNL-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-BHInfo		CRITICALITY ignore	EXTENSION BHInfo		PRESENCE optional	},

...

}

AdditionalSIBMessageList ::= SEQUENCE (SIZE(1..maxnoofAdditionalSIBs)) OF AdditionalSIBMessageList-Item

AdditionalSIBMessageList-Item ::= SEQUENCE {

additionalSIB			OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { { AdditionalSIBMessageList-Item-ExtIEs} } OPTIONAL

}

AdditionalSIBMessageList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AdditionalRRMPriorityIndex ::= BIT STRING (SIZE(32))

AffectedCellsAndBeams-List ::= SEQUENCE (SIZE (1.. maxAffectedCells)) OF AffectedCellsAndBeams-Item

AffectedCellsAndBeams-Item::= SEQUENCE {

nRCGI					NRCGI,

affectedSSB-List		AffectedSSB-List OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AffectedCellsAndBeams-Item-ExtIEs} } OPTIONAL,

...

}

AffectedCellsAndBeams-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AffectedSSB-List::= SEQUENCE (SIZE (1..maxnoofSSBAreas)) OF AffectedSSB-Item

AffectedSSB-Item::= SEQUENCE {

sSB-Index	INTEGER(0..63),

iE-Extensions		ProtocolExtensionContainer { { AffectedSSB-Item-ExtIEs} } OPTIONAL,

...

}

AffectedSSB-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AggressorCellList ::= SEQUENCE (SIZE(1..maxCellingNBDU)) OF AggressorCellList-Item

AggressorCellList-Item ::= SEQUENCE {

aggressorCell-ID		NRCGI,

iE-Extensions	ProtocolExtensionContainer { { AggressorCellList-Item-ExtIEs } }		OPTIONAL

}

AggressorCellList-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AggressorgNBSetID ::= SEQUENCE {

aggressorgNBSetID		GNBSetID,

iE-Extensions	ProtocolExtensionContainer { { AggressorgNBSetID-ExtIEs } }	OPTIONAL

}

AggressorgNBSetID-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {AllocationAndRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationAndRetentionPriority-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AlternativeQoSParaSetList ::= SEQUENCE (SIZE(1..maxnoofQoSParaSets)) OF AlternativeQoSParaSetItem

AlternativeQoSParaSetItem ::= SEQUENCE {

alternativeQoSParaSetIndex			QoSParaSetIndex,

guaranteedFlowBitRateDL				BitRate					OPTIONAL,

guaranteedFlowBitRateUL				BitRate					OPTIONAL,

packetDelayBudget					PacketDelayBudget		OPTIONAL,

packetErrorRate						PacketErrorRate			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {AlternativeQoSParaSetItem-ExtIEs} }	OPTIONAL,

...

}

AlternativeQoSParaSetItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AngleMeasurementQuality ::= SEQUENCE {

azimuthQuality	INTEGER(0..255),

zenithQuality	INTEGER(0..255) OPTIONAL,

resolution		ENUMERATED{deg0dot1,...},

iE-Extensions	ProtocolExtensionContainer { { AngleMeasurementQuality-ExtIEs } }	OPTIONAL

}

AngleMeasurementQuality-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AperiodicSRSResourceTriggerList ::= SEQUENCE (SIZE(1..maxnoofSRSTriggerStates)) OF AperiodicSRSResourceTrigger

AperiodicSRSResourceTrigger ::= INTEGER (1..3)

Associated-SCell-Item ::= SEQUENCE {

sCell-ID		NRCGI,

iE-Extensions	ProtocolExtensionContainer { { Associated-SCell-ItemExtIEs } }	OPTIONAL

}

Associated-SCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

AvailablePLMNList ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF AvailablePLMNList-Item

AvailablePLMNList-Item ::= SEQUENCE {

pLMNIdentity			PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { { AvailablePLMNList-Item-ExtIEs} } OPTIONAL

}

AvailablePLMNList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AvailableSNPN-ID-List ::= SEQUENCE (SIZE(1..maxnoofNIDsupported)) OF AvailableSNPN-ID-List-Item

AvailableSNPN-ID-List-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

availableNIDList			BroadcastNIDList,

iE-Extensions				ProtocolExtensionContainer { { AvailableSNPN-ID-List-ItemExtIEs} } OPTIONAL,

...

}

AvailableSNPN-ID-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AveragingWindow  ::= INTEGER (0..4095, ...)

AreaScope ::= ENUMERATED {true, ...}

AoA-AssistanceInfo ::= SEQUENCE {

angleMeasurement			AngleMeasurementType,

lCS-to-GCS-Translation	LCS-to-GCS-Translation		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { AoA-AssistanceInfo-ExtIEs } } OPTIONAL,

...

}

AoA-AssistanceInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

AngleMeasurementType ::= CHOICE {

expected-ULAoA		Expected-UL-AoA,

expected-ZoA		Expected-ZoA-only,

choice-extension ProtocolIE-SingleContainer { { AngleMeasurementType-ExtIEs } }

}

AngleMeasurementType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

AppLayerBufferLevelList ::= OCTET STRING

ARP-ID ::= INTEGER (1..16, ...)

ARPLocationInformation ::= SEQUENCE (SIZE (1..maxnoARPs)) OF ARPLocationInformation-Item

ARPLocationInformation-Item ::= SEQUENCE {

aRP-ID				ARP-ID,

aRPLocationType		ARPLocationType,

iE-Extensions		ProtocolExtensionContainer { { ARPLocationInformation-ExtIEs} } OPTIONAL,

...

}

ARPLocationInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ARPLocationType ::= CHOICE {

aRPPositionRelativeGeodetic			RelativeGeodeticLocation,

aRPPositionRelativeCartesian		RelativeCartesianLocation,

choice-extension					ProtocolIE-SingleContainer { { ARPLocationType-ExtIEs } }

}

ARPLocationType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

#### -- B

BAP-Header-Rewriting-Added-List-Item::= SEQUENCE {

ingressBAPRoutingID		BAPRoutingID,

egressBAPRoutingID		BAPRoutingID,

nonF1terminatingTopologyIndicator		NonF1terminatingTopologyIndicator		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { BAP-Header-Rewriting-Added-List-Item-ExtIEs} }	OPTIONAL

}

BAP-Header-Rewriting-Added-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BAP-Header-Rewriting-Removed-List-Item::= SEQUENCE {

ingressBAPRoutingID		BAPRoutingID,

iE-Extensions			ProtocolExtensionContainer { { BAP-Header-Rewriting-Removed-List-Item-ExtIEs} } OPTIONAL

}

BAP-Header-Rewriting-Removed-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BandwidthSRS ::= CHOICE {

fR1								FR1-Bandwidth,

fR2								FR2-Bandwidth,

choice-extension				ProtocolIE-SingleContainer {{ BandwidthSRS-ExtIEs }}

}

BandwidthSRS-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

BAPAddress ::= BIT STRING (SIZE(10))

BAPCtrlPDUChannel ::= ENUMERATED {true, ...}

BAPlayerBHRLCchannelMappingInfo ::= SEQUENCE {

bAPlayerBHRLCchannelMappingInfoToAdd			BAPlayerBHRLCchannelMappingInfoList			OPTIONAL,

bAPlayerBHRLCchannelMappingInfoToRemove			MappingInformationtoRemove					OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { BAPlayerBHRLCchannelMappingInfo-ExtIEs} } OPTIONAL,

...

}

BAPlayerBHRLCchannelMappingInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BAPlayerBHRLCchannelMappingInfoList ::= SEQUENCE (SIZE(1..maxnoofMappingEntries)) OF BAPlayerBHRLCchannelMappingInfo-Item

BAPlayerBHRLCchannelMappingInfo-Item ::= SEQUENCE {

mappingInformationIndex			MappingInformationIndex,

priorHopBAPAddress				BAPAddress		OPTIONAL,

ingressbHRLCChannelID			BHRLCChannelID		OPTIONAL,

nextHopBAPAddress				BAPAddress		OPTIONAL,

egressbHRLCChannelID			BHRLCChannelID		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { BAPlayerBHRLCchannelMappingInfo-ItemExtIEs} } OPTIONAL,

...

}

BAPlayerBHRLCchannelMappingInfo-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-IngressNonF1terminatingTopologyIndicator	CRITICALITY ignore	EXTENSION IngressNonF1terminatingTopologyIndicator		PRESENCE optional}|

{ ID id-EgressNonF1terminatingTopologyIndicator	CRITICALITY ignore	EXTENSION EgressNonF1terminatingTopologyIndicator		PRESENCE optional},

...

}

BAPPathID ::= BIT STRING (SIZE(10))

BAPRoutingID ::= SEQUENCE {

bAPAddress		BAPAddress,

bAPPathID		BAPPathID,

iE-Extensions	ProtocolExtensionContainer { { BAPRoutingIDExtIEs } }	OPTIONAL

}

BAPRoutingIDExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BCBearerContextF1U-TNLInfo ::= CHOICE {

locationindpendent				MBSF1UInformation,

locationdependent				LocationDependentMBSF1UInformation,

choice-extension	ProtocolIE-SingleContainer	{{BCBearerContextF1U-TNLInfo-ExtIEs}}

}

BCBearerContextF1U-TNLInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

BitRate ::= INTEGER (0..4000000000000,...)

BearerTypeChange ::= ENUMERATED {true, ...}

BHRLCChannelID ::= BIT STRING (SIZE(16))

BHChannels-FailedToBeModified-Item ::= SEQUENCE {

bHRLCChannelID		BHRLCChannelID,

cause		Cause		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-FailedToBeModified-ItemExtIEs } }	OPTIONAL

}

BHChannels-FailedToBeModified-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-FailedToBeSetup-Item ::= SEQUENCE {

bHRLCChannelID		BHRLCChannelID,

cause	Cause	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-FailedToBeSetup-ItemExtIEs } }	OPTIONAL

}

BHChannels-FailedToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-FailedToBeSetupMod-Item ::= SEQUENCE {

bHRLCChannelID		BHRLCChannelID,

cause		Cause			OPTIONAL ,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-FailedToBeSetupMod-ItemExtIEs } }	OPTIONAL

}

BHChannels-FailedToBeSetupMod-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-Modified-Item ::= SEQUENCE {

bHRLCChannelID			BHRLCChannelID,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-Modified-ItemExtIEs } }	OPTIONAL

}

BHChannels-Modified-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-Required-ToBeReleased-Item ::= SEQUENCE {

bHRLCChannelID		BHRLCChannelID,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-Required-ToBeReleased-ItemExtIEs } }	OPTIONAL

}

BHChannels-Required-ToBeReleased-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-Setup-Item ::= SEQUENCE {

bHRLCChannelID							BHRLCChannelID,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-Setup-ItemExtIEs } }	OPTIONAL

}

BHChannels-Setup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-SetupMod-Item ::= SEQUENCE {

bHRLCChannelID							BHRLCChannelID,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-SetupMod-ItemExtIEs } }	OPTIONAL

}

BHChannels-SetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-ToBeModified-Item ::= SEQUENCE {

bHRLCChannelID					BHRLCChannelID,

bHQoSInformation				BHQoSInformation,

rLCmode				RLCMode	OPTIONAL,

bAPCtrlPDUChannel	BAPCtrlPDUChannel		OPTIONAL,

trafficMappingInfo	TrafficMappingInfo		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-ToBeModified-ItemExtIEs } }	OPTIONAL

}

BHChannels-ToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-ToBeReleased-Item ::= SEQUENCE {

bHRLCChannelID		BHRLCChannelID,

iE-Extensions	ProtocolExtensionContainer { { BHChannels-ToBeReleased-ItemExtIEs } }	OPTIONAL

}

BHChannels-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-ToBeSetup-Item ::= SEQUENCE	{

bHRLCChannelID						BHRLCChannelID,

bHQoSInformation					BHQoSInformation,

rLCmode								RLCMode,

bAPCtrlPDUChannel					BAPCtrlPDUChannel		OPTIONAL,

trafficMappingInfo					TrafficMappingInfo		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { BHChannels-ToBeSetup-ItemExtIEs } }	OPTIONAL

}

BHChannels-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHChannels-ToBeSetupMod-Item ::= SEQUENCE {

bHRLCChannelID				BHRLCChannelID,

bHQoSInformation			BHQoSInformation,

rLCmode				RLCMode,

bAPCtrlPDUChannel	BAPCtrlPDUChannel		OPTIONAL,

trafficMappingInfo	TrafficMappingInfo		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { BHChannels-ToBeSetupMod-ItemExtIEs } }	OPTIONAL

}

BHChannels-ToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BHInfo ::= SEQUENCE {

bAProutingID			BAPRoutingID 	OPTIONAL,

egressBHRLCCHList		EgressBHRLCCHList	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { BHInfo-ExtIEs} } OPTIONAL

}

BHInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NonF1terminatingTopologyIndicator	CRITICALITY ignore	EXTENSION NonF1terminatingTopologyIndicator		PRESENCE optional	},

...

}

BHQoSInformation ::= CHOICE {

bHRLCCHQoS					QoSFlowLevelQoSParameters,

eUTRANBHRLCCHQoS			EUTRANQoS,

cPTrafficType				CPTrafficType,

choice-extension			ProtocolIE-SingleContainer { { BHQoSInformation-ExtIEs} }

}

BHQoSInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

BHRLCCHList ::= SEQUENCE (SIZE(1..maxnoofBHRLCChannels)) OF BHRLCCHItem

BHRLCCHItem ::= SEQUENCE {

bHRLCChannelID			BHRLCChannelID,

iE-Extensions			ProtocolExtensionContainer {{BHRLCCHItemExtIEs }}	 OPTIONAL

}

BHRLCCHItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BH-Routing-Information-Added-List-Item ::= SEQUENCE {

bAPRoutingID				BAPRoutingID,

nextHopBAPAddress			BAPAddress,

iE-Extensions				ProtocolExtensionContainer { { BH-Routing-Information-Added-List-ItemExtIEs} }	OPTIONAL

}

BH-Routing-Information-Added-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-NonF1terminatingTopologyIndicator  CRITICALITY ignore EXTENSION   NonF1terminatingTopologyIndicator		PRESENCE optional},

...

}

BH-Routing-Information-Removed-List-Item ::= SEQUENCE {

bAPRoutingID				BAPRoutingID,

iE-Extensions				ProtocolExtensionContainer { { BH-Routing-Information-Removed-List-ItemExtIEs} }	OPTIONAL

}

BH-Routing-Information-Removed-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BPLMN-ID-Info-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNsNR)) OF BPLMN-ID-Info-Item

BPLMN-ID-Info-Item ::= SEQUENCE {

pLMN-Identity-List			AvailablePLMNList,

extended-PLMN-Identity-List	ExtendedAvailablePLMN-List	OPTIONAL,

fiveGS-TAC					FiveGS-TAC					OPTIONAL,

nr-cell-ID					NRCellIdentity,

ranac						RANAC						OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BPLMN-ID-Info-ItemExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication		PRESENCE optional }|

{	ID id-NPNBroadcastInformation		CRITICALITY reject EXTENSION NPNBroadcastInformation		PRESENCE optional},

...

}

ServedPLMNs-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF ServedPLMNs-Item

ServedPLMNs-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

iE-Extensions				ProtocolExtensionContainer { { ServedPLMNs-ItemExtIEs} } OPTIONAL,

...

}

ServedPLMNs-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-TAISliceSupportList			CRITICALITY ignore	EXTENSION SliceSupportList				PRESENCE optional	}|

{ ID id-NPNSupportInfo				CRITICALITY reject	EXTENSION NPNSupportInfo				PRESENCE optional	}|

{ ID id-ExtendedTAISliceSupportList	CRITICALITY reject	EXTENSION ExtendedSliceSupportList		PRESENCE optional	}|

{ ID id-TAINSAGSupportList			CRITICALITY ignore	EXTENSION NSAGSupportList				PRESENCE optional},

...

}

BroadcastCAGList ::= SEQUENCE (SIZE(1..maxnoofCAGsupported)) OF CAGID

BroadcastMRBs-FailedToBeModified-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-FailedtoBeModified-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-FailedtoBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-FailedToBeSetup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-FailedToBeSetup-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-FailedToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-FailedToBeSetupMod-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-FailedToBeSetupMod-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-FailedToBeSetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-Modified-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

bcBearerCtxtF1U-TNLInfoatDU	BCBearerContextF1U-TNLInfo		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-Modified-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-Modified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-Setup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

bcBearerCtxtF1U-TNLInfoatDU	BCBearerContextF1U-TNLInfo,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-Setup-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-SetupMod-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

bcBearerCtxtF1U-TNLInfoatDU	BCBearerContextF1U-TNLInfo,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-SetupMod-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-SetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-ToBeModified-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters		OPTIONAL,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List	OPTIONAL,

bcBearerCtxtF1U-TNLInfoatCU		BCBearerContextF1U-TNLInfo		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { BroadcastMRBs-ToBeModified-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-ToBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-ToBeReleased-Item	::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { BroadcastMRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

BroadcastMRBs-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-ToBeSetup-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List,

bcBearerCtxtF1U-TNLInfoatCU		BCBearerContextF1U-TNLInfo	,

iE-Extensions					ProtocolExtensionContainer { { BroadcastMRBs-ToBeSetup-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-ToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastMRBs-ToBeSetupMod-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List,

bcBearerCtxtF1U-TNLInfoatCU		BCBearerContextF1U-TNLInfo,

iE-Extensions					ProtocolExtensionContainer { { BroadcastMRBs-ToBeSetupMod-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastMRBs-ToBeSetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastNIDList ::= SEQUENCE (SIZE(1..maxnoofNIDsupported)) OF NID

BroadcastSNPN-ID-List ::= SEQUENCE (SIZE(1..maxnoofNIDsupported)) OF BroadcastSNPN-ID-List-Item

BroadcastSNPN-ID-List-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

broadcastNIDList			BroadcastNIDList,

iE-Extensions				ProtocolExtensionContainer { { BroadcastSNPN-ID-List-ItemExtIEs} } OPTIONAL,

...

}

BroadcastSNPN-ID-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastPNI-NPN-ID-List ::= SEQUENCE (SIZE(1..maxnoofCAGsupported)) OF BroadcastPNI-NPN-ID-List-Item

BroadcastPNI-NPN-ID-List-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

broadcastCAGList			BroadcastCAGList,

iE-Extensions				ProtocolExtensionContainer { { BroadcastPNI-NPN-ID-List-ItemExtIEs} } OPTIONAL,

...

}

BroadcastPNI-NPN-ID-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastAreaScope ::= CHOICE {

completeSuccess			NULL,

partialSuccess			PartialSuccessCell,

choice-extension		ProtocolIE-SingleContainer { { BroadcastAreaScope-ExtIEs } }

}

BroadcastAreaScope-ExtIEs F1AP-PROTOCOL-IES::={

...

}

BroadcastCellList ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF Broadcast-Cell-List-Item

Broadcast-Cell-List-Item ::= SEQUENCE {

cellID				NRCGI,

iE-Extensions				ProtocolExtensionContainer { { Broadcast-Cell-List-ItemExtIEs} } OPTIONAL,

...

}

Broadcast-Cell-List-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

BufferSizeThresh ::= INTEGER(0..16777215)

BurstArrivalTime ::= OCTET STRING

BWP-Id ::= INTEGER (0..4)

-- C

CAGID ::= BIT STRING (SIZE(32))

Cancel-all-Warning-Messages-Indicator ::= ENUMERATED {true, ...}

Candidate-SpCell-Item ::= SEQUENCE {

candidate-SpCell-ID			NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { Candidate-SpCell-ItemExtIEs } }	OPTIONAL,

...

}

Candidate-SpCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CapacityValue::= SEQUENCE {

capacityValue				INTEGER (0..100),

sSBAreaCapacityValueList	SSBAreaCapacityValueList		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { CapacityValue-ExtIEs} } OPTIONAL

}

CapacityValue-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

protocol			CauseProtocol,

misc				CauseMisc,

choice-extension	ProtocolIE-SingleContainer { { Cause-ExtIEs} }

}

Cause-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

not-enough-user-plane-processing-resources,

hardware-failure,

om-intervention,

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

rl-failure-rlc,

unknown-or-already-allocated-gnb-cu-ue-f1ap-id,

unknown-or-already-allocated-gnb-du-ue-f1ap-id,

unknown-or-inconsistent-pair-of-ue-f1ap-id,

interaction-with-other-procedure,

not-supported-qci-Value,

action-desirable-for-radio-reasons,

no-radio-resources-available,

procedure-cancelled,

normal-release,

...,

cell-not-available,

rl-failure-others,

ue-rejection,

resources-not-available-for-the-slice,

amf-initiated-abnormal-release,

release-due-to-pre-emption,

plmn-not-served-by-the-gNB-CU,

multiple-drb-id-instances,

unknown-drb-id,

multiple-bh-rlc-ch-id-instances,

unknown-bh-rlc-ch-id,

cho-cpc-resources-tobechanged,

nPN-not-supported,

nPN-access-denied,

gNB-CU-Cell-Capacity-Exceeded,

report-characteristics-empty,

existing-measurement-ID,

measurement-temporarily-not-available,

measurement-not-supported-for-the-object,

unknown-bh-address,

unknown-bap-routing-id,

insufficient-ue-capabilities,

scg-activation-deactivation-failure,

scg-deactivation-failure-due-to-data-transmission,

requested-item-not-supported-on-time,

unknown-or-already-allocated-gNB-CU-MBS-F1AP-ID,

unknown-or-already-allocated-gNB-DU-MBS-F1AP-ID,

unknown-or-inconsistent-pair-of-MBS-F1AP-ID,

unknown-or-inconsistent-MRB-ID,

tat-sdt-expiry

}

CauseTransport ::= ENUMERATED {

unspecified,

transport-resource-unavailable,

...,

unknown-TNL-address-for-IAB,

unknown-UP-TNL-information-for-IAB

}

CellGroupConfig ::= OCTET STRING

CellCapacityClassValue ::= INTEGER (1..100,...)

Cell-Direction ::= ENUMERATED {dl-only, ul-only}

CellMeasurementResultList ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF CellMeasurementResultItem

CellMeasurementResultItem ::= SEQUENCE {

cellID							NRCGI,

radioResourceStatus				RadioResourceStatus 			OPTIONAL,

compositeAvailableCapacityGroup	CompositeAvailableCapacityGroup	OPTIONAL,

sliceAvailableCapacity			SliceAvailableCapacity 			OPTIONAL,

numberofActiveUEs 				NumberofActiveUEs				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CellMeasurementResultItem-ExtIEs} } OPTIONAL

}

CellMeasurementResultItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NR-U-Channel-List	CRITICALITY ignore	EXTENSION NR-U-Channel-List PRESENCE optional },

...

}

Cell-Portion-ID ::= INTEGER (0..4095,...)

CellsForSON-List ::= SEQUENCE (SIZE(1.. maxServedCellforSON)) OF CellsForSON-Item

CellsForSON-Item ::= SEQUENCE {

nRCGI											NRCGI,

neighbourNR-CellsForSON-List					NeighbourNR-CellsForSON-List									OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { CellsForSON-Item-ExtIEs} }	OPTIONAL,

...

}

CellsForSON-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Failed-to-be-Activated-List-Item ::= SEQUENCE {

nRCGI				NRCGI,

cause				Cause,

iE-Extensions		ProtocolExtensionContainer { { Cells-Failed-to-be-Activated-List-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Failed-to-be-Activated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Status-Item ::= SEQUENCE {

nRCGI			NRCGI,

service-status		Service-Status,

iE-Extensions				ProtocolExtensionContainer { { Cells-Status-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Status-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-To-Be-Broadcast-Item ::= SEQUENCE {

nRCGI				NRCGI,

iE-Extensions		ProtocolExtensionContainer { { Cells-To-Be-Broadcast-ItemExtIEs } }	OPTIONAL,

...

}

Cells-To-Be-Broadcast-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Broadcast-Completed-Item ::= SEQUENCE {

nRCGI				NRCGI,

iE-Extensions		ProtocolExtensionContainer { { Cells-Broadcast-Completed-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Broadcast-Completed-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Broadcast-To-Be-Cancelled-Item ::= SEQUENCE {

nRCGI				NRCGI,

iE-Extensions		ProtocolExtensionContainer { { Broadcast-To-Be-Cancelled-ItemExtIEs } }	OPTIONAL,

...

}

Broadcast-To-Be-Cancelled-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-Broadcast-Cancelled-Item ::= SEQUENCE {

nRCGI				NRCGI,

numberOfBroadcasts	NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { { Cells-Broadcast-Cancelled-ItemExtIEs } }	OPTIONAL,

...

}

Cells-Broadcast-Cancelled-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Activated-List-Item ::= SEQUENCE {

nRCGI		NRCGI,

nRPCI		NRPCI		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Activated-List-ItemExtIEs} }	OPTIONAL,

...

}

Cells-to-be-Activated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-gNB-CUSystemInformation			CRITICALITY reject	EXTENSION GNB-CUSystemInformation				PRESENCE optional }|

{ ID id-AvailablePLMNList				CRITICALITY ignore	EXTENSION AvailablePLMNList						PRESENCE optional }|

{ ID id-ExtendedAvailablePLMN-List		CRITICALITY ignore	EXTENSION ExtendedAvailablePLMN-List			PRESENCE optional }|

{ ID id-IAB-Info-IAB-donor-CU			CRITICALITY ignore	EXTENSION IAB-Info-IAB-donor-CU					PRESENCE optional}|

{ ID id-AvailableSNPN-ID-List			CRITICALITY ignore	EXTENSION AvailableSNPN-ID-List					PRESENCE optional }|

{ ID id-MBS-Broadcast-NeighbourCellList	CRITICALITY ignore	EXTENSION MBS-Broadcast-NeighbourCellList	PRESENCE optional },

...

}

Cells-to-be-Deactivated-List-Item ::= SEQUENCE {

nRCGI			NRCGI	,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Deactivated-List-ItemExtIEs } }	OPTIONAL,

...

}

Cells-to-be-Deactivated-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Cells-to-be-Barred-Item::= SEQUENCE {

nRCGI			NRCGI	,

cellBarred		CellBarred,

iE-Extensions				ProtocolExtensionContainer { { Cells-to-be-Barred-Item-ExtIEs } }	OPTIONAL

}

Cells-to-be-Barred-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-IAB-Barred	CRITICALITY ignore	EXTENSION IAB-Barred		PRESENCE optional },

...

}

CellBarred	::=	ENUMERATED {barred, not-barred, ...}

CellSize ::= ENUMERATED {verysmall, small, medium, large, ...}

CellToReportList ::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF CellToReportItem

CellToReportItem ::= SEQUENCE {

cellID		NRCGI,

sSBToReportList		SSBToReportList		 OPTIONAL,

sliceToReportList	SliceToReportList	 OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { CellToReportItem-ExtIEs} } OPTIONAL

}

CellToReportItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellType ::= SEQUENCE {

cellSize		CellSize,

iE-Extensions		ProtocolExtensionContainer { {CellType-ExtIEs} }	OPTIONAL,

...

}

CellType-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellULConfigured ::=  ENUMERATED {none, ul, sul, ul-and-sul, ...}

CG-SDTQueryIndication ::=  ENUMERATED {true, ...}

CG-SDTKeptIndicator ::= ENUMERATED {true, ...}

CG-SDTindicatorSetup ::= ENUMERATED {true, ...}

CG-SDTindicatorMod ::= ENUMERATED {true, false, ...}

CG-SDTSessionInfo ::= SEQUENCE {

gNB-CU-UE-F1AP-ID					GNB-CU-UE-F1AP-ID,

gNB-DU-UE-F1AP-ID					GNB-DU-UE-F1AP-ID,

iE-Extensions						ProtocolExtensionContainer {{CG-SDTSessionInfo-ExtIEs}}		OPTIONAL,

...

}

CG-SDTSessionInfo-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

ChannelOccupancyTimePercentage ::= INTEGER (0..100,...)

Child-IAB-Nodes-NA-Resource-List ::= SEQUENCE (SIZE(1..maxnoofChildIABNodes)) OF Child-IAB-Nodes-NA-Resource-List-Item

Child-IAB-Nodes-NA-Resource-List-Item::= SEQUENCE {

gNB-CU-UE-F1AP-ID	GNB-CU-UE-F1AP-ID,

gNB-DU-UE-F1AP-ID	GNB-DU-UE-F1AP-ID,

nA-Resource-Configuration-List		NA-Resource-Configuration-List 		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { Child-IAB-Nodes-NA-Resource-List-Item-ExtIEs} } OPTIONAL

}

Child-IAB-Nodes-NA-Resource-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Child-Node-Cells-List ::= SEQUENCE (SIZE(1..maxnoofChildIABNodes)) OF Child-Node-Cells-List-Item

Child-Node-Cells-List-Item ::=	SEQUENCE{

nRCGI 								NRCGI,

iAB-DU-Cell-Resource-Configuration-Mode-Info 	IAB-DU-Cell-Resource-Configuration-Mode-Info	OPTIONAL,

iAB-STC-Info						IAB-STC-Info			OPTIONAL,

rACH-Config-Common					RACH-Config-Common		OPTIONAL,

rACH-Config-Common-IAB				RACH-Config-Common-IAB	OPTIONAL,

cSI-RS-Configuration				OCTET STRING			OPTIONAL,

sR-Configuration					OCTET STRING			OPTIONAL,

pDCCH-ConfigSIB1					OCTET STRING			OPTIONAL,

sCS-Common							OCTET STRING	OPTIONAL,

multiplexingInfo					MultiplexingInfo	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer {{Child-Node-Cells-List-Item-ExtIEs}}		OPTIONAL

}

Child-Node-Cells-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Child-Nodes-List ::= SEQUENCE (SIZE(1..maxnoofChildIABNodes)) OF Child-Nodes-List-Item

Child-Nodes-List-Item ::= SEQUENCE{

gNB-CU-UE-F1AP-ID	GNB-CU-UE-F1AP-ID,

gNB-DU-UE-F1AP-ID	GNB-DU-UE-F1AP-ID,

child-Node-Cells-List 	Child-Node-Cells-List	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{Child-Nodes-List-Item-ExtIEs}}		OPTIONAL

}

Child-Nodes-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CHOtrigger-InterDU ::= ENUMERATED {

cho-initiation,

cho-replace,

...

}

CHOtrigger-IntraDU ::= ENUMERATED {

cho-initiation,

cho-replace,

cho-cancel,

...

}

CNSubgroupID ::= INTEGER (0..7, ...)

CNUEPagingIdentity ::= CHOICE {

fiveG-S-TMSI			BIT STRING (SIZE(48)),

choice-extension			ProtocolIE-SingleContainer { { CNUEPagingIdentity-ExtIEs } }

}

CNUEPagingIdentity-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

CompositeAvailableCapacityGroup ::= SEQUENCE {

compositeAvailableCapacityDownlink	CompositeAvailableCapacity,

compositeAvailableCapacityUplink 	CompositeAvailableCapacity,

iE-Extensions	ProtocolExtensionContainer { { CompositeAvailableCapacityGroup-ExtIEs} } OPTIONAL

}

CompositeAvailableCapacityGroup-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CompositeAvailableCapacity-SUL		CRITICALITY ignore	EXTENSION CompositeAvailableCapacity		PRESENCE optional	},

...

}

CompositeAvailableCapacity ::= SEQUENCE {

cellCapacityClassValue 	CellCapacityClassValue		OPTIONAL,

capacityValue			CapacityValue,

iE-Extensions	ProtocolExtensionContainer { { CompositeAvailableCapacity-ExtIEs} } OPTIONAL

}

CompositeAvailableCapacity-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CHO-Probability ::= INTEGER (1..100)

ConditionalInterDUMobilityInformation ::= SEQUENCE {

cho-trigger						CHOtrigger-InterDU,

targetgNB-DUUEF1APID			GNB-DU-UE-F1AP-ID							OPTIONAL

-- This IE shall be present if the cho-trigger IE is present and set to "cho-replace" --,

iE-Extensions					ProtocolExtensionContainer { { ConditionalInterDUMobilityInformation-ExtIEs} }	OPTIONAL,

...

}

ConditionalInterDUMobilityInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

{ ID id-EstimatedArrivalProbability		CRITICALITY ignore		EXTENSION CHO-Probability		PRESENCE optional	},

...

}

ConditionalIntraDUMobilityInformation ::= SEQUENCE {

cho-trigger						CHOtrigger-IntraDU,

targetCellsTocancel				TargetCellList								OPTIONAL,

-- This IE may be present if the cho-trigger IE is present and set to "cho-cancel"

iE-Extensions					ProtocolExtensionContainer { { ConditionalIntraDUMobilityInformation-ExtIEs} }	OPTIONAL,

...

}

ConditionalIntraDUMobilityInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

{ ID id-EstimatedArrivalProbability		CRITICALITY ignore		EXTENSION CHO-Probability		PRESENCE optional	},

...

}

ConfigRestrictInfoDAPS ::= OCTET STRING

ConfiguredTACIndication ::= ENUMERATED {

true,

...

}

Configured-BWP-List ::= SEQUENCE (SIZE(1.. maxNrofBWPs)) OF Configured-BWP-Item

Configured-BWP-Item ::= SEQUENCE {

bWP-Id									BWP-Id,

bWP-Location-and-bandwidth				INTEGER (0..37949),

iE-Extensions					ProtocolExtensionContainer { { Configured-BWP-Item-ExtIEs } }	OPTIONAL,

...

}

Configured-BWP-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

CoordinateID ::= INTEGER (0..511, ...)

Coverage-Modification-Notification ::= SEQUENCE {

coverage-Modification-List				Coverage-Modification-List,

iE-Extensions								ProtocolExtensionContainer { { Coverage-Modification-Notification-ExtIEs} }	OPTIONAL,

...

}

Coverage-Modification-Notification-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

Coverage-Modification-List ::= SEQUENCE (SIZE (1..maxCellingNBDU)) OF Coverage-Modification-Item

Coverage-Modification-Item ::= SEQUENCE {

nRCGI								NRCGI,

cellCoverageState					CellCoverageState,

sSBCoverageModificationList			SSBCoverageModification-List OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { Coverage-Modification-Item-ExtIEs} } 				OPTIONAL,

...

}

Coverage-Modification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CellCoverageState ::= INTEGER (0..63, ...)

CCO-Assistance-Information ::= SEQUENCE {

cCO-issue-detection				CCO-issue-detection		OPTIONAL,

affectedCellsAndBeams-List		AffectedCellsAndBeams-List 		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { CCO-Assistance-Information-ExtIEs} }	OPTIONAL,

...

}

CCO-Assistance-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::={

...

}

CCO-issue-detection	::=	ENUMERATED {

coverage,

cell-edge-capacity,

...}

CP-TransportLayerAddress ::= CHOICE {

endpoint-IP-address				TransportLayerAddress,

endpoint-IP-address-and-port	Endpoint-IP-address-and-port,

choice-extension				ProtocolIE-SingleContainer { { CP-TransportLayerAddress-ExtIEs } }

}

CP-TransportLayerAddress-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

CPACMCGInformation ::= SEQUENCE {

cpac-trigger					CPAC-trigger,

pscellid						NRCGI,

iE-Extensions					ProtocolExtensionContainer { { CPACMCGInformation-ExtIEs} }	OPTIONAL,

...

}

CPACMCGInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CPAC-trigger ::= ENUMERATED {

cpac-preparation,

cpac-executed,

...

}

CPTrafficType ::= INTEGER (1..3,...)

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode														OPTIONAL,

triggeringMessage				TriggeringMessage													OPTIONAL,

procedureCriticality			Criticality															OPTIONAL,

transactionID					TransactionID														OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List										OPTIONAL,

iE-Extensions					ProtocolExtensionContainer {{CriticalityDiagnostics-ExtIEs}}		OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1.. maxnoofErrors)) OF CriticalityDiagnostics-IE-Item

CriticalityDiagnostics-IE-Item ::= SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError 			TypeOfError,

iE-Extensions			ProtocolExtensionContainer {{CriticalityDiagnostics-IE-Item-ExtIEs}}	OPTIONAL,

...

}

CriticalityDiagnostics-IE-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

C-RNTI ::= INTEGER (0..65535, ...)

CUDURadioInformationType ::= CHOICE {

rIM								CUDURIMInformation,

choice-extension				ProtocolIE-SingleContainer { { CUDURadioInformationType-ExtIEs} }

}

CUDURadioInformationType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

CUDURIMInformation ::= SEQUENCE {

victimgNBSetID			GNBSetID,

rIMRSDetectionStatus	RIMRSDetectionStatus,

iE-Extensions			ProtocolExtensionContainer { { CUDURIMInformation-ExtIEs} }	OPTIONAL

}

CUDURIMInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

CUtoDURRCInformation ::= SEQUENCE {

cG-ConfigInfo						CG-ConfigInfo						OPTIONAL,

uE-CapabilityRAT-ContainerList		UE-CapabilityRAT-ContainerList		OPTIONAL,

measConfig							MeasConfig							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { CUtoDURRCInformation-ExtIEs} } OPTIONAL,

...

}

CUtoDURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-HandoverPreparationInformation	CRITICALITY ignore	EXTENSION HandoverPreparationInformation		PRESENCE optional }|

{ ID id-CellGroupConfig					CRITICALITY ignore	EXTENSION CellGroupConfig							PRESENCE optional }|

{ ID id-MeasurementTimingConfiguration	CRITICALITY ignore	EXTENSION MeasurementTimingConfiguration		PRESENCE optional }|

{ ID id-UEAssistanceInformation			CRITICALITY ignore	EXTENSION UEAssistanceInformation					PRESENCE optional }|

{ ID id-CG-Config						CRITICALITY ignore	EXTENSION CG-Config									PRESENCE optional }|

{ ID id-UEAssistanceInformationEUTRA	CRITICALITY ignore	EXTENSION UEAssistanceInformationEUTRA			PRESENCE optional }|

{ ID id-LocationMeasurementInformation	CRITICALITY ignore	EXTENSION LocationMeasurementInformation		PRESENCE optional }|

{ ID id-MUSIM-GapConfig					CRITICALITY reject	EXTENSION MUSIM-GapConfig							PRESENCE optional }|

{ ID id-SDT-MAC-PHY-CG-Config			CRITICALITY ignore	EXTENSION SDT-MAC-PHY-CG-Config						PRESENCE optional }|

{ ID id-MBSInterestIndication			CRITICALITY ignore	EXTENSION MBSInterestIndication						PRESENCE optional }|

{ ID id-NeedForGapsInfoNR				CRITICALITY ignore	EXTENSION NeedForGapsInfoNR							PRESENCE optional }|

{ ID id-NeedForGapNCSGInfoNR			CRITICALITY ignore	EXTENSION NeedForGapNCSGInfoNR						PRESENCE optional }|

{ ID id-NeedForGapNCSGInfoEUTRA			CRITICALITY ignore	EXTENSION NeedForGapNCSGInfoEUTRA					PRESENCE optional }|

{ ID id-ConfigRestrictInfoDAPS			CRITICALITY ignore	EXTENSION ConfigRestrictInfoDAPS					PRESENCE optional }|

{ ID id-Preconfigured-measurement-GAP-Request			CRITICALITY ignore	EXTENSION Preconfigured-measurement-GAP-Request					PRESENCE optional },

...

}

#### -- D

DAPS-HO-Status::= ENUMERATED{initiation,... }

DCBasedDuplicationConfigured::= ENUMERATED{true,..., false}

Dedicated-SIDelivery-NeededUE-Item ::= SEQUENCE {

gNB-CU-UE-F1AP-ID						GNB-CU-UE-F1AP-ID,

nRCGI									NRCGI,

iE-Extensions							ProtocolExtensionContainer { { DedicatedSIDeliveryNeededUE-Item-ExtIEs} } OPTIONAL,

...

}

DedicatedSIDeliveryNeededUE-Item-ExtIEs F1AP-PROTOCOL-EXTENSION::={

...

}

DedicatedSIDeliveryIndication::= ENUMERATED{true, ...}

DL-PRS ::= SEQUENCE {

prsid 					INTEGER (0..255),

dl-PRSResourceSetID		PRS-Resource-Set-ID,

dl-PRSResourceID		PRS-Resource-ID	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {DL-PRS-ExtIEs} }	OPTIONAL

}

DL-PRS-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DL-PRSMutingPattern ::= CHOICE {

two					BIT STRING (SIZE(2)),

four				BIT STRING (SIZE(4)),

six					BIT STRING (SIZE(6)),

eight				BIT STRING (SIZE(8)),

sixteen				BIT STRING (SIZE(16)),

thirty-two			BIT STRING (SIZE(32)),

choice-extension							ProtocolIE-SingleContainer { { DL-PRSMutingPattern-ExtIEs } }

}

DL-PRSMutingPattern-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

DLPRSResourceCoordinates ::= SEQUENCE {

listofDL-PRSResourceSetARP		SEQUENCE (SIZE(1.. maxnoofPRS-ResourceSets)) OF DLPRSResourceSetARP,

iE-Extensions					ProtocolExtensionContainer { { DLPRSResourceCoordinates-ExtIEs } } OPTIONAL

}

DLPRSResourceCoordinates-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DLPRSResourceSetARP ::= SEQUENCE {

dl-PRSResourceSetID				PRS-Resource-Set-ID,

dL-PRSResourceSetARPLocation	DL-PRSResourceSetARPLocation,

listofDL-PRSResourceARP			SEQUENCE (SIZE(1.. maxnoofPRS-ResourcesPerSet)) OF DLPRSResourceARP,

iE-Extensions					ProtocolExtensionContainer { { DLPRSResourceSetARP-ExtIEs } } OPTIONAL

}

DLPRSResourceSetARP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DL-PRSResourceSetARPLocation ::= CHOICE {

relativeGeodeticLocation			RelativeGeodeticLocation,

relativeCartesianLocation			RelativeCartesianLocation,

choice-Extension					ProtocolIE-SingleContainer { { DL-PRSResourceSetARPLocation-ExtIEs } }

}

DL-PRSResourceSetARPLocation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

DLPRSResourceARP ::= SEQUENCE {

dl-PRSResourceID			PRS-Resource-ID,

dL-PRSResourceARPLocation	DL-PRSResourceARPLocation,

iE-Extensions				ProtocolExtensionContainer { { DLPRSResourceARP-ExtIEs } } OPTIONAL

}

DLPRSResourceARP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DL-PRSResourceARPLocation ::= CHOICE {

relativeGeodeticLocation			RelativeGeodeticLocation,

relativeCartesianLocation			RelativeCartesianLocation,

choice-Extension					ProtocolIE-SingleContainer { { DL-PRSResourceARPLocation-ExtIEs } }

}

DL-PRSResourceARPLocation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

DL-UP-TNL-Address-to-Update-List-Item	::= SEQUENCE {

oldIPAdress						TransportLayerAddress,

newIPAdress						TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { DL-UP-TNL-Address-to-Update-List-ItemExtIEs } }	OPTIONAL,

...

}

DL-UP-TNL-Address-to-Update-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DLUPTNLInformation-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofDLUPTNLInformation)) OF DLUPTNLInformation-ToBeSetup-Item

DLUPTNLInformation-ToBeSetup-Item ::= SEQUENCE {

dLUPTNLInformation	UPTransportLayerInformation	,

iE-Extensions	ProtocolExtensionContainer { { DLUPTNLInformation-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

DLUPTNLInformation-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Activity-Item ::= SEQUENCE {

dRBID			DRBID,

dRB-Activity	DRB-Activity		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRB-Activity-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Activity-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Activity ::= ENUMERATED {active, not-active}

DRBID ::= INTEGER (1..32, ...)

DRBs-FailedToBeModified-Item	::= SEQUENCE {

dRBID		DRBID		,

cause		Cause		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-FailedToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-FailedToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-FailedToBeSetup-Item	::= SEQUENCE {

dRBID	DRBID,

cause	Cause	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-FailedToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-FailedToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-FailedToBeSetupMod-Item	::= SEQUENCE {

dRBID		DRBID	,

cause		Cause			OPTIONAL ,

iE-Extensions	ProtocolExtensionContainer { { DRBs-FailedToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-FailedToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRB-Information	::=	SEQUENCE {

dRB-QoS		QoSFlowLevelQoSParameters,

sNSSAI		SNSSAI,

notificationControl		NotificationControl		OPTIONAL,

flows-Mapped-To-DRB-List	Flows-Mapped-To-DRB-List,

iE-Extensions	ProtocolExtensionContainer { { DRB-Information-ItemExtIEs } }	OPTIONAL

}

DRB-Information-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Modified-Item	::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Modified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Modified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLC-Status							CRITICALITY ignore	EXTENSION RLC-Status									PRESENCE optional }|

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List		PRESENCE optional }|

{ ID id-CurrentQoSParaSetIndex				CRITICALITY ignore	EXTENSION QoSParaSetIndex								PRESENCE optional },

...

}

DRBs-ModifiedConf-Item	::= SEQUENCE {

dRBID							DRBID,

uLUPTNLInformation-ToBeSetup-List		ULUPTNLInformation-ToBeSetup-List	,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ModifiedConf-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ModifiedConf-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional },

...

}

DRB-Notify-Item ::= SEQUENCE {

dRBID			DRBID,

notification-Cause	Notification-Cause,

iE-Extensions	ProtocolExtensionContainer { { DRB-Notify-ItemExtIEs } }	OPTIONAL,

...

}

DRB-Notify-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CurrentQoSParaSetIndex	CRITICALITY ignore	EXTENSION QoSParaSetNotifyIndex	PRESENCE optional	},

...

}

DRBs-Required-ToBeModified-Item	::= SEQUENCE {

dRBID							DRBID,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List	,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Required-ToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Required-ToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-RLC-Status			CRITICALITY ignore			EXTENSION RLC-Status				PRESENCE optional }|

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional },

...

}

DRBs-Required-ToBeReleased-Item	::= SEQUENCE {

dRBID		DRBID,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Required-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Required-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-Setup-Item ::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List	,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Setup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-Setup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional }|

{ ID id-CurrentQoSParaSetIndex				CRITICALITY ignore	EXTENSION QoSParaSetIndex									PRESENCE optional },

...

}

DRBs-SetupMod-Item	::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List	,

iE-Extensions	ProtocolExtensionContainer { { DRBs-SetupMod-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-SetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional }|

{ ID id-CurrentQoSParaSetIndex				CRITICALITY ignore	EXTENSION QoSParaSetIndex									PRESENCE optional },

...

}

DRBs-ToBeModified-Item	::= SEQUENCE {

dRBID						DRBID,

qoSInformation				QoSInformation	OPTIONAL,

uLUPTNLInformation-ToBeSetup-List	ULUPTNLInformation-ToBeSetup-List	,

uLConfiguration				ULConfiguration	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeModified-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-DLPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength										PRESENCE optional }|

{ ID id-ULPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength										PRESENCE optional }|

{ID id-BearerTypeChange						CRITICALITY ignore	EXTENSION BearerTypeChange									PRESENCE optional}|

{ ID id-RLCMode								CRITICALITY ignore	EXTENSION RLCMode											PRESENCE optional }|

{ ID id-Duplication-Activation				CRITICALITY reject	EXTENSION DuplicationActivation							PRESENCE optional }|

{ ID id-DC-Based-Duplication-Configured		CRITICALITY reject	EXTENSION DCBasedDuplicationConfigured					PRESENCE optional }|

{ ID id-DC-Based-Duplication-Activation		CRITICALITY reject	EXTENSION DuplicationActivation							PRESENCE optional }|

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List			PRESENCE optional }|

{ ID id-RLCDuplicationInformation			CRITICALITY ignore	EXTENSION RLCDuplicationInformation						PRESENCE optional}|

{ ID id-TransmissionStopIndicator			CRITICALITY ignore	EXTENSION TransmissionStopIndicator						PRESENCE optional}|

{ ID id-CG-SDTindicatorMod					CRITICALITY reject	EXTENSION CG-SDTindicatorMod								PRESENCE optional },

...

}

DRBs-ToBeReleased-Item	::= SEQUENCE {

dRBID	DRBID,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRBs-ToBeSetup-Item ::= SEQUENCE	{

dRBID						DRBID,

qoSInformation				QoSInformation,

uLUPTNLInformation-ToBeSetup-List	ULUPTNLInformation-ToBeSetup-List	,

rLCMode						RLCMode,

uLConfiguration				ULConfiguration	OPTIONAL,

duplicationActivation		DuplicationActivation	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-DC-Based-Duplication-Configured		CRITICALITY reject	EXTENSION DCBasedDuplicationConfigured				PRESENCE optional }|

{ ID id-DC-Based-Duplication-Activation		CRITICALITY reject	EXTENSION DuplicationActivation						PRESENCE optional }|

{ ID id-DLPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength									PRESENCE mandatory }|

{ ID id-ULPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength									PRESENCE optional }|

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List		PRESENCE optional }|

{ ID id-RLCDuplicationInformation			CRITICALITY ignore	EXTENSION RLCDuplicationInformation					PRESENCE optional}|

{ ID id-SDTRLCBearerConfiguration			CRITICALITY ignore	EXTENSION SDTRLCBearerConfiguration					PRESENCE optional },

...

}

DRBs-ToBeSetupMod-Item	::= SEQUENCE {

dRBID						DRBID,

qoSInformation				QoSInformation,

uLUPTNLInformation-ToBeSetup-List		ULUPTNLInformation-ToBeSetup-List,

rLCMode						RLCMode,

uLConfiguration				ULConfiguration	OPTIONAL,

duplicationActivation		DuplicationActivation	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { DRBs-ToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

DRBs-ToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-DC-Based-Duplication-Configured		CRITICALITY reject	EXTENSION DCBasedDuplicationConfigured				PRESENCE optional }|

{ ID id-DC-Based-Duplication-Activation		CRITICALITY reject	EXTENSION DuplicationActivation						PRESENCE optional }|

{ ID id-DLPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength									PRESENCE optional }|

{ ID id-ULPDCPSNLength						CRITICALITY ignore	EXTENSION PDCPSNLength									PRESENCE optional }|

{ ID id-AdditionalPDCPDuplicationTNL-List	CRITICALITY ignore	EXTENSION AdditionalPDCPDuplicationTNL-List		PRESENCE optional }|

{ ID id-RLCDuplicationInformation			CRITICALITY ignore	EXTENSION RLCDuplicationInformation					PRESENCE optional}|

{ ID id-CG-SDTindicatorSetup				CRITICALITY reject	EXTENSION CG-SDTindicatorSetup						PRESENCE optional },

...

}

DRXCycle	::= SEQUENCE {

longDRXCycleLength	LongDRXCycleLength,

shortDRXCycleLength		ShortDRXCycleLength	OPTIONAL,

shortDRXCycleTimer	ShortDRXCycleTimer OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { DRXCycle-ExtIEs} } OPTIONAL,

...

}

DRXCycle-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DRX-Config ::= OCTET STRING

DRXConfigurationIndicator	::=	ENUMERATED{	release, ...}

DRX-LongCycleStartOffset ::= INTEGER (0..10239)

DSInformationList ::= SEQUENCE (SIZE(0..maxnoofDSInfo)) OF DSCP

DSCP ::= BIT STRING (SIZE (6))

DUtoCURRCContainer ::= OCTET STRING

DUCURadioInformationType ::= CHOICE {

rIM								DUCURIMInformation,

choice-extension				ProtocolIE-SingleContainer { { DUCURadioInformationType-ExtIEs} }

}

DUCURadioInformationType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

DUCURIMInformation ::= SEQUENCE {

victimgNBSetID				GNBSetID,

rIMRSDetectionStatus		RIMRSDetectionStatus,

aggressorCellList			AggressorCellList,

iE-Extensions				ProtocolExtensionContainer { { DUCURIMInformation-ExtIEs} }		OPTIONAL

}

DUCURIMInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DUF-Slot-Config-Item 	::=	CHOICE {

explicitFormat				ExplicitFormat,

implicitFormat				ImplicitFormat,

choice-extension				ProtocolIE-SingleContainer { { DUF-Slot-Config-Item-ExtIEs} }

}

DUF-Slot-Config-Item-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

DUF-Slot-Config-List	::= SEQUENCE (SIZE(1..maxnoofDUFSlots)) OF DUF-Slot-Config-Item

DUFSlotformatIndex ::= INTEGER(0..254)

DUFTransmissionPeriodicity ::= ENUMERATED { ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ...}

DU-RX-MT-RX ::= ENUMERATED {supported, not-supported }

DU-TX-MT-TX ::= ENUMERATED {supported, not-supported }

DU-RX-MT-TX ::= ENUMERATED {supported, not-supported }

DU-TX-MT-RX ::= ENUMERATED {supported, not-supported }

DU-RX-MT-RX-Extend ::= ENUMERATED {supported, not-supported, supported-and-FDM-required, ...}

DU-TX-MT-TX-Extend ::= ENUMERATED {supported, not-supported, supported-and-FDM-required, ...}

DU-RX-MT-TX-Extend ::= ENUMERATED {supported, not-supported, supported-and-FDM-required, ...}

DU-TX-MT-RX-Extend ::= ENUMERATED {supported, not-supported, supported-and-FDM-required, ...}

DUtoCURRCInformation ::= SEQUENCE {

cellGroupConfig		CellGroupConfig,

measGapConfig			MeasGapConfig	OPTIONAL,

requestedP-MaxFR1				OCTET STRING				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { DUtoCURRCInformation-ExtIEs} } OPTIONAL,

...

}

DUtoCURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-DRX-LongCycleStartOffset			CRITICALITY ignore	EXTENSION DRX-LongCycleStartOffset				PRESENCE optional }|

{ ID id-SelectedBandCombinationIndex		CRITICALITY ignore	EXTENSION SelectedBandCombinationIndex			PRESENCE optional }|

{ ID id-SelectedFeatureSetEntryIndex		CRITICALITY ignore	EXTENSION SelectedFeatureSetEntryIndex			PRESENCE optional }|

{ ID id-Ph-InfoSCG							CRITICALITY ignore	EXTENSION Ph-InfoSCG								PRESENCE optional }|

{ ID id-RequestedBandCombinationIndex		CRITICALITY ignore	EXTENSION RequestedBandCombinationIndex		PRESENCE optional }|

{ ID id-RequestedFeatureSetEntryIndex		CRITICALITY ignore	EXTENSION RequestedFeatureSetEntryIndex		PRESENCE optional }|

{ ID id-DRX-Config							CRITICALITY ignore	EXTENSION DRX-Config								PRESENCE optional }|

{ ID id-PDCCH-BlindDetectionSCG				CRITICALITY ignore	EXTENSION PDCCH-BlindDetectionSCG				PRESENCE optional }|

{ ID id-Requested-PDCCH-BlindDetectionSCG	CRITICALITY ignore	EXTENSION Requested-PDCCH-BlindDetectionSCG	PRESENCE optional }|

{ ID id-Ph-InfoMCG							CRITICALITY ignore	EXTENSION Ph-InfoMCG								PRESENCE optional }|

{ ID id-MeasGapSharingConfig				CRITICALITY ignore	EXTENSION MeasGapSharingConfig					PRESENCE optional }|

{ ID id-SL-PHY-MAC-RLC-Config				CRITICALITY ignore	EXTENSION SL-PHY-MAC-RLC-Config					PRESENCE optional }|

{ ID id-SL-ConfigDedicatedEUTRA-Info		CRITICALITY ignore	EXTENSION SL-ConfigDedicatedEUTRA-Info			PRESENCE optional }|

{ ID id-RequestedP-MaxFR2					CRITICALITY ignore	EXTENSION RequestedP-MaxFR2							PRESENCE optional }|

{ ID id-SDT-MAC-PHY-CG-Config				CRITICALITY ignore	EXTENSION SDT-MAC-PHY-CG-Config							PRESENCE optional }|

{ ID id-MUSIM-GapConfig						CRITICALITY ignore	EXTENSION MUSIM-GapConfig							PRESENCE optional }|

{ ID id-SL-RLC-ChannelToAddModList			CRITICALITY ignore	EXTENSION SL-RLC-ChannelToAddModList			PRESENCE optional }|

{ ID id-InterFrequencyConfig-NoGap			CRITICALITY ignore	EXTENSION InterFrequencyConfig-NoGap			PRESENCE optional }|

{ ID id-UL-GapFR2-Config					CRITICALITY ignore	EXTENSION UL-GapFR2-Config							PRESENCE optional }|

{ ID id-TwoPHRModeMCG						CRITICALITY ignore	EXTENSION TwoPHRModeMCG								PRESENCE optional }|

{ ID id-TwoPHRModeSCG						CRITICALITY ignore	EXTENSION TwoPHRModeSCG								PRESENCE optional }|

{ ID id-ncd-SSB-RedCapInitialBWP-SDT		CRITICALITY ignore	EXTENSION Ncd-SSB-RedCapInitialBWP-SDT			PRESENCE optional }|

{ ID id-ServCellInfoList					CRITICALITY ignore	EXTENSION ServCellInfoList							PRESENCE optional }|

{ ID id-SL-PHY-MAC-RLC-ConfigExt			CRITICALITY ignore	EXTENSION SL-PHY-MAC-RLC-ConfigExt				PRESENCE optional },

...

}

DuplicationActivation ::= ENUMERATED{active,inactive,... }

DuplicationIndication ::= ENUMERATED {true, ... , false }

DuplicationState ::= ENUMERATED {

active,

inactive,

...

}

Dynamic5QIDescriptor	::= SEQUENCE {

qoSPriorityLevel					INTEGER (1..127),

packetDelayBudget					PacketDelayBudget,

packetErrorRate						PacketErrorRate,

fiveQI								INTEGER (0..255, ...)								OPTIONAL,

delayCritical						ENUMERATED {delay-critical, non-delay-critical}		OPTIONAL,

-- C-ifGBRflow: This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

averagingWindow 					AveragingWindow										OPTIONAL,

-- C-ifGBRflow: This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

maxDataBurstVolume					MaxDataBurstVolume									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Dynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

Dynamic5QIDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedPacketDelayBudget			CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-CNPacketDelayBudgetDownlink			CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		}|

{ ID id-CNPacketDelayBudgetUplink			CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional		},

...

}

DynamicPQIDescriptor	::= SEQUENCE {

resourceType						ENUMERATED {gbr, non-gbr, delay-critical-grb, ...}	OPTIONAL,

qoSPriorityLevel					INTEGER (1..8, ...),

packetDelayBudget					PacketDelayBudget,

packetErrorRate						PacketErrorRate,

averagingWindow 					AveragingWindow										OPTIONAL,

-- C-ifGBRflow: This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE.

maxDataBurstVolume					MaxDataBurstVolume									OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { DynamicPQIDescriptor-ExtIEs } } OPTIONAL

}

DynamicPQIDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- E

E-CID-MeasurementQuantities ::= SEQUENCE (SIZE (1.. maxnoofMeasE-CID)) OF ProtocolIE-SingleContainer { {E-CID-MeasurementQuantities-ItemIEs} }

E-CID-MeasurementQuantities-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-E-CID-MeasurementQuantities-Item	CRITICALITY reject	TYPE E-CID-MeasurementQuantities-Item		PRESENCE mandatory}

}

E-CID-MeasurementQuantities-Item ::= SEQUENCE {

e-CIDmeasurementQuantitiesValue				E-CID-MeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { E-CID-MeasurementQuantitiesValue-ExtIEs} } OPTIONAL

}

E-CID-MeasurementQuantitiesValue-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

E-CID-MeasurementQuantitiesValue ::= ENUMERATED {

default,

angleOfArrivalNR,

... ,

timingAdvanceNR

}

E-CID-MeasurementResult ::= SEQUENCE {

geographicalCoordinates		GeographicalCoordinates 	OPTIONAL,

measuredResults-List		E-CID-MeasuredResults-List 	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { E-CID-MeasurementResult-ExtIEs} } OPTIONAL

}

E-CID-MeasurementResult-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

E-CID-MeasuredResults-List ::= SEQUENCE (SIZE(1..maxnoofMeasE-CID)) OF E-CID-MeasuredResults-Item

E-CID-MeasuredResults-Item ::= SEQUENCE {

e-CID-MeasuredResults-Value 	E-CID-MeasuredResults-Value,

iE-Extensions			ProtocolExtensionContainer {{ E-CID-MeasuredResults-Item-ExtIEs }}	 OPTIONAL

}

E-CID-MeasuredResults-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

E-CID-MeasuredResults-Value ::= CHOICE {

valueAngleofArrivalNR	UL-AoA,

choice-extension		ProtocolIE-SingleContainer { { E-CID-MeasuredResults-Value-ExtIEs} }

}

E-CID-MeasuredResults-Value-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-NR-TADV			CRITICALITY ignore	TYPE NR-TADV	PRESENCE mandatory	},

...

}

E-CID-ReportCharacteristics ::= ENUMERATED {

onDemand,

periodic,

...

}

EgressBHRLCCHList ::= SEQUENCE (SIZE(1..maxnoofEgressLinks)) OF EgressBHRLCCHItem

EgressBHRLCCHItem ::= SEQUENCE {

nextHopBAPAddress 		BAPAddress,

bHRLCChannelID			BHRLCChannelID,

iE-Extensions			ProtocolExtensionContainer {{EgressBHRLCCHItemExtIEs }}	 OPTIONAL

}

EgressBHRLCCHItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EgressNonF1terminatingTopologyIndicator ::= ENUMERATED {true, ...}

Endpoint-IP-address-and-port ::=SEQUENCE {

endpointIPAddress TransportLayerAddress,

iE-Extensions					ProtocolExtensionContainer { { Endpoint-IP-address-and-port-ExtIEs} } OPTIONAL

}

Endpoint-IP-address-and-port-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-portNumber	CRITICALITY reject	EXTENSION PortNumber		PRESENCE optional},

...

}

EnergyDetectionThreshold ::= INTEGER (-100..-50, ...)

ExtendedAvailablePLMN-List ::= SEQUENCE (SIZE(1..maxnoofExtendedBPLMNs)) OF ExtendedAvailablePLMN-Item

ExtendedAvailablePLMN-Item ::= SEQUENCE {

pLMNIdentity			PLMN-Identity,

iE-Extensions		ProtocolExtensionContainer { { ExtendedAvailablePLMN-Item-ExtIEs} } OPTIONAL

}

ExplicitFormat ::=	SEQUENCE {

permutation			Permutation,

noofDownlinkSymbols	NoofDownlinkSymbols		OPTIONAL,

noofUplinkSymbols	NoofUplinkSymbols		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ExplicitFormat-ExtIEs} } OPTIONAL

}

ExplicitFormat-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedAvailablePLMN-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedServedPLMNs-List ::= SEQUENCE (SIZE(1.. maxnoofExtendedBPLMNs)) OF ExtendedServedPLMNs-Item

ExtendedServedPLMNs-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

tAISliceSupportList 		SliceSupportList	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ExtendedServedPLMNs-ItemExtIEs} } OPTIONAL,

...

}

ExtendedServedPLMNs-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NPNSupportInfo				CRITICALITY reject	EXTENSION NPNSupportInfo				PRESENCE optional	}|

{ ID id-ExtendedTAISliceSupportList	CRITICALITY reject	EXTENSION ExtendedSliceSupportList		PRESENCE optional	}|

{ ID id-TAINSAGSupportList			CRITICALITY ignore	EXTENSION NSAGSupportList				PRESENCE optional},

...

}

ExtendedSliceSupportList ::= SEQUENCE (SIZE(1.. maxnoofExtSliceItems)) OF SliceSupportItem

ExtendedUEIdentityIndexValue ::= BIT STRING (SIZE(16))

EUTRACells-List  ::= SEQUENCE (SIZE (1.. maxCellineNB)) OF EUTRACells-List-item

EUTRACells-List-item ::= SEQUENCE {

eUTRA-Cell-ID					EUTRA-Cell-ID,

served-EUTRA-Cells-Information	Served-EUTRA-Cells-Information,

iE-Extensions ProtocolExtensionContainer { { EUTRACells-List-itemExtIEs } }    OPTIONAL

}

EUTRACells-List-itemExtIEs    F1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-Cell-ID ::= BIT STRING (SIZE(28))

EUTRA-Coex-FDD-Info ::= SEQUENCE {

uL-EARFCN						ExtendedEARFCN					OPTIONAL,

dL-EARFCN						ExtendedEARFCN,

uL-Transmission-Bandwidth		EUTRA-Transmission-Bandwidth	OPTIONAL,

dL-Transmission-Bandwidth		EUTRA-Transmission-Bandwidth,

iE-Extensions					ProtocolExtensionContainer { {EUTRA-Coex-FDD-Info-ExtIEs} } OPTIONAL,

...

}

EUTRA-Coex-FDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-Coex-Mode-Info ::= CHOICE {

fDD		EUTRA-Coex-FDD-Info,

tDD		EUTRA-Coex-TDD-Info,

...

}

EUTRA-Coex-TDD-Info ::= SEQUENCE {

eARFCN							ExtendedEARFCN,

transmission-Bandwidth			EUTRA-Transmission-Bandwidth,

subframeAssignment				EUTRA-SubframeAssignment,

specialSubframe-Info			EUTRA-SpecialSubframe-Info,

iE-Extensions					ProtocolExtensionContainer { {EUTRA-Coex-TDD-Info-ExtIEs} } OPTIONAL,

...

}

EUTRA-Coex-TDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-CyclicPrefixDL ::= ENUMERATED {

normal,

extended,

...

}

EUTRA-CyclicPrefixUL ::= ENUMERATED {

normal,

extended,

...

}

EUTRA-PRACH-Configuration ::= SEQUENCE {

rootSequenceIndex						INTEGER (0..837),

zeroCorrelationIndex					INTEGER (0..15),

highSpeedFlag							BOOLEAN,

prach-FreqOffset						INTEGER (0..94),

prach-ConfigIndex						INTEGER (0..63)		OPTIONAL,

-- C-ifTDD: This IE shall be present if the EUTRA-Mode-Info IE in the Resource Coordination E-UTRA Cell Information IE is set to the value "TDD"

iE-Extensions							ProtocolExtensionContainer { {EUTRA-PRACH-Configuration-ExtIEs} }	OPTIONAL,

...

}

EUTRA-PRACH-Configuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-SpecialSubframe-Info ::= SEQUENCE {

specialSubframePatterns		EUTRA-SpecialSubframePatterns,

cyclicPrefixDL				EUTRA-CyclicPrefixDL,

cyclicPrefixUL				EUTRA-CyclicPrefixUL,

iE-Extensions				ProtocolExtensionContainer { { EUTRA-SpecialSubframe-Info-ExtIEs} } OPTIONAL,

...

}

EUTRA-SpecialSubframe-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-SpecialSubframePatterns ::= ENUMERATED {

ssp0,

ssp1,

ssp2,

ssp3,

ssp4,

ssp5,

ssp6,

ssp7,

ssp8,

ssp9,

ssp10,

...

}

EUTRA-SubframeAssignment ::= ENUMERATED {

sa0,

sa1,

sa2,

sa3,

sa4,

sa5,

sa6,

...

}

EUTRA-Transmission-Bandwidth ::= ENUMERATED {

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...

}

EUTRANQoS	::= SEQUENCE {

qCI								QCI,

allocationAndRetentionPriority	AllocationAndRetentionPriority,

gbrQosInformation				GBR-QosInformation									OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { EUTRANQoS-ExtIEs} }	OPTIONAL,

...

}

EUTRANQoS-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ENBDLTNLAddress		CRITICALITY ignore	EXTENSION TransportLayerAddress	PRESENCE optional	},

...

}

ExecuteDuplication ::= ENUMERATED{true,...}

ExtendedEARFCN ::= INTEGER (0..262143)

EUTRA-Mode-Info ::= CHOICE {

eUTRAFDD		EUTRA-FDD-Info,

eUTRATDD		EUTRA-TDD-Info,

choice-extension	ProtocolIE-SingleContainer { { EUTRA-Mode-Info-ExtIEs} }

}

EUTRA-Mode-Info-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

EUTRA-NR-CellResourceCoordinationReq-Container	::= OCTET STRING

EUTRA-NR-CellResourceCoordinationReqAck-Container	::= OCTET STRING

EUTRA-FDD-Info ::= SEQUENCE {

uL-offsetToPointA				OffsetToPointA,

dL-offsetToPointA				OffsetToPointA,

iE-Extensions					ProtocolExtensionContainer { {EUTRA-FDD-Info-ExtIEs} } OPTIONAL,

...

}

EUTRA-FDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EUTRA-TDD-Info ::= SEQUENCE {

offsetToPointA					OffsetToPointA,

iE-Extensions					ProtocolExtensionContainer { {EUTRA-TDD-Info-ExtIEs} } OPTIONAL,

...

}

EUTRA-TDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

EventType ::= ENUMERATED {

on-demand,

periodic,

stop,

...

}

ExtendedPacketDelayBudget ::= INTEGER (1..65535, ..., 65536..109999)

Expected-UL-AoA ::= SEQUENCE {

expected-Azimuth-AoA	Expected-Azimuth-AoA,

expected-Zenith-AoA		Expected-Zenith-AoA		OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { Expected-UL-AoA-ExtIEs } }	OPTIONAL,

...

}

Expected-UL-AoA-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Expected-ZoA-only ::= SEQUENCE {

expected-ZoA-only	Expected-Zenith-AoA,

iE-extensions		ProtocolExtensionContainer { { Expected-ZoA-only-ExtIEs } }	OPTIONAL,

...

}

Expected-ZoA-only-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Expected-Azimuth-AoA ::= SEQUENCE {

expected-Azimuth-AoA-value			Expected-Value-AoA,

expected-Azimuth-AoA-uncertainty	Uncertainty-range-AoA,

iE-Extensions						ProtocolExtensionContainer { { Expected-Azimuth-AoA-ExtIEs } }		OPTIONAL,

...

}

Expected-Azimuth-AoA-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Expected-Zenith-AoA ::= SEQUENCE {

expected-Zenith-AoA-value			Expected-Value-ZoA,

expected-Zenith-AoA-uncertainty		Uncertainty-range-ZoA,

iE-Extensions						ProtocolExtensionContainer { { Expected-Zenith-AoA-ExtIEs } }		OPTIONAL,

...

}

Expected-Zenith-AoA-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Expected-Value-AoA ::= INTEGER (0..3599)

Expected-Value-ZoA ::= INTEGER (0..1799)

#### -- F

F1CPathNSA ::= ENUMERATED {lte, nr, both}

F1CTransferPath ::= SEQUENCE {

f1CPathNSA						F1CPathNSA,

iE-Extensions					ProtocolExtensionContainer { { F1CTransferPath-ExtIEs} } OPTIONAL,

...

}

F1CTransferPath-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

F1CPathNRDC ::= ENUMERATED {mcg, scg, both}

F1CTransferPathNRDC ::= SEQUENCE {

f1CPathNRDC						F1CPathNRDC,

iE-Extensions					ProtocolExtensionContainer { { F1CTransferPathNRDC-ExtIEs} } OPTIONAL,

...

}

F1CTransferPathNRDC-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

FDD-Info ::= SEQUENCE {

uL-NRFreqInfo						NRFreqInfo,

dL-NRFreqInfo						NRFreqInfo,

uL-Transmission-Bandwidth			Transmission-Bandwidth,

dL-Transmission-Bandwidth			Transmission-Bandwidth,

iE-Extensions						ProtocolExtensionContainer { {FDD-Info-ExtIEs} } OPTIONAL,

...

}

FDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ULCarrierList				CRITICALITY ignore	EXTENSION NRCarrierList				PRESENCE optional }|

{ ID id-DLCarrierList				CRITICALITY ignore EXTENSION NRCarrierList				PRESENCE optional },

...

}

FDD-InfoRel16 ::= SEQUENCE {

uL-FreqInfo										FreqInfoRel16													OPTIONAL,

sUL-FreqInfo									FreqInfoRel16													OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {FDD-InfoRel16-ExtIEs} }		OPTIONAL,

...

}

FDD-InfoRel16-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

FiveG-ProSeAuthorized ::= SEQUENCE {

fiveG-proSeDirectDiscovery					FiveG-ProSeDirectDiscovery									OPTIONAL,

fiveG-proSeDirectCommunication				FiveG-ProSeDirectCommunication								OPTIONAL,

fiveG-ProSeLayer2UEtoNetworkRelay			FiveG-ProSeLayer2UEtoNetworkRelay							OPTIONAL,

fiveG-ProSeLayer3UEtoNetworkRelay			FiveG-ProSeLayer3UEtoNetworkRelay							OPTIONAL,

fiveG-ProSeLayer2RemoteUE					FiveG-ProSeLayer2RemoteUE									OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { {FiveG-ProSeAuthorized-ExtIEs} }	OPTIONAL,

...

}

FiveG-ProSeAuthorized-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

FiveG-ProSeDirectDiscovery ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeDirectCommunication ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer3UEtoNetworkRelay ::= ENUMERATED {

authorized,

not-authorized,

...

}

FiveG-ProSeLayer2RemoteUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

Flows-Mapped-To-DRB-List	::=	SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF Flows-Mapped-To-DRB-Item

Flows-Mapped-To-DRB-Item 	::= SEQUENCE {

qoSFlowIdentifier							QoSFlowIdentifier,

qoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

iE-Extensions							ProtocolExtensionContainer { { Flows-Mapped-To-DRB-ItemExtIEs} } OPTIONAL

}

Flows-Mapped-To-DRB-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoSFlowMappingIndication		CRITICALITY ignore	EXTENSION QoSFlowMappingIndication			PRESENCE optional}|

{ID id-TSCTrafficCharacteristics	CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics			PRESENCE optional},

...

}

FR1-Bandwidth ::= ENUMERATED {bw5, bw10, bw20, bw40, bw50, bw80, bw100, ..., bw15, bw25, bw30, bw60, bw35, bw45, bw70, bw90}

FR2-Bandwidth ::= ENUMERATED {bw50, bw100, bw200, bw400, ..., bw800, bw1600, bw2000}

FreqBandNrItem ::= SEQUENCE {

freqBandIndicatorNr 		INTEGER (1..1024,...),

supportedSULBandList		SEQUENCE (SIZE(0..maxnoofNrCellBands)) OF SupportedSULFreqBandItem,

iE-Extensions				ProtocolExtensionContainer { {FreqBandNrItem-ExtIEs} } OPTIONAL,

...

}

FreqBandNrItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

FreqDomainLength ::= CHOICE {

l839							L839Info,

l139							L139Info,

choice-extension				ProtocolIE-SingleContainer { {FreqDomainLength-ExtIEs} }

}

FreqDomainLength-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-L571Info	CRITICALITY reject	TYPE L571Info PRESENCE mandatory}|

{ ID id-L1151Info	CRITICALITY reject	TYPE L1151Info PRESENCE mandatory},

...

}

FreqInfoRel16 ::=  SEQUENCE {

nRARFCN											INTEGER (0..maxNRARFCN)											OPTIONAL,

frequencyShift7p5khz							FrequencyShift7p5khz											OPTIONAL,

carrierList										NRCarrierList													OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { FreqInfoRel16-ExtIEs} }		OPTIONAL,

...

}

FreqInfoRel16-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

FrequencyShift7p5khz ::= ENUMERATED {false, true, ...}

Frequency-Domain-HSNA-Configuration-List ::= SEQUENCE (SIZE(1..maxnoofRBsetsPerCell)) OF Frequency-Domain-HSNA-Configuration-Item

Frequency-Domain-HSNA-Configuration-Item::= SEQUENCE {

rBSetIndex 		    INTEGER (0..maxnoofRBsetsPerCell-1, ...),

frequency-Domain-HSNA-Slot-Configuration-List				Frequency-Domain-HSNA-Slot-Configuration-List,

iE-Extensions		ProtocolExtensionContainer { { Frequency-Domain-HSNA-Configuration-Item-ExtIEs} } OPTIONAL

}

Frequency-Domain-HSNA-Configuration-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Frequency-Domain-HSNA-Slot-Configuration-List ::= SEQUENCE (SIZE(1..maxnoofHSNASlots)) OF Frequency-Domain-HSNA-Slot-Configuration-Item

Frequency-Domain-HSNA-Slot-Configuration-Item::= SEQUENCE {

slotIndex				INTEGER (0..5119)		OPTIONAL,

hSNADownlink			HSNADownlink 			OPTIONAL,

hSNAUplink				HSNAUplink 				OPTIONAL,

hSNAFlexible			HSNAFlexible 			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { Frequency-Domain-HSNA-Slot-Configuration-Item-ExtIEs } } OPTIONAL

}

Frequency-Domain-HSNA-Slot-Configuration-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

FullConfiguration ::= ENUMERATED {full, ...}

FlowsMappedToSLDRB-List ::= SEQUENCE (SIZE(1.. maxnoofPC5QoSFlows)) OF FlowsMappedToSLDRB-Item

FlowsMappedToSLDRB-Item ::= SEQUENCE {

pc5QoSFlowIdentifier			PC5QoSFlowIdentifier,

iE-Extensions					ProtocolExtensionContainer { {FlowsMappedToSLDRB-Item-ExtIEs} } OPTIONAL,

...

}

FlowsMappedToSLDRB-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- G

GBR-QosInformation ::= SEQUENCE {

e-RAB-MaximumBitrateDL			BitRate,

e-RAB-MaximumBitrateUL			BitRate,

e-RAB-GuaranteedBitrateDL		BitRate,

e-RAB-GuaranteedBitrateUL		BitRate,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GBR-QoSFlowInformation::= SEQUENCE {

maxFlowBitRateDownlink			BitRate,

maxFlowBitRateUplink			BitRate,

guaranteedFlowBitRateDownlink	BitRate,

guaranteedFlowBitRateUplink		BitRate,

maxPacketLossRateDownlink		MaxPacketLossRate		OPTIONAL,

maxPacketLossRateUplink			MaxPacketLossRate		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GBR-QosFlowInformation-ExtIEs} } OPTIONAL,

...

}

GBR-QosFlowInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ 	ID id-AlternativeQoSParaSetList	CRITICALITY ignore	EXTENSION AlternativeQoSParaSetList	PRESENCE optional	},

...

}

CG-Config ::= OCTET STRING

GeographicalCoordinates ::= SEQUENCE {

tRPPositionDefinitionType	TRPPositionDefinitionType,

dLPRSResourceCoordinates	DLPRSResourceCoordinates	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { GeographicalCoordinates-ExtIEs } } OPTIONAL

}

GeographicalCoordinates-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ARPLocationInfo		CRITICALITY ignore EXTENSION ARPLocationInformation	PRESENCE optional},

...

}

GNB-CU-MBS-F1AP-ID		::= INTEGER (0..4294967295)

GNBCUMeasurementID ::= INTEGER (0.. 4095, ...)

GNBDUMeasurementID ::= INTEGER (0.. 4095, ...)

GNB-CUSystemInformation::= SEQUENCE {

sibtypetobeupdatedlist	SEQUENCE (SIZE(1.. maxnoofSIBTypes)) OF SibtypetobeupdatedListItem,

iE-Extensions					ProtocolExtensionContainer { { GNB-CUSystemInformation-ExtIEs} } OPTIONAL,

...

}

GNB-CUSystemInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-systemInformationAreaID  CRITICALITY ignore	EXTENSION SystemInformationAreaID PRESENCE optional},

...

}

GNB-CU-TNL-Association-Setup-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TransportLayerAddress	,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-TNL-Association-Setup-Item-ExtIEs} } OPTIONAL

}

GNB-CU-TNL-Association-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-TNL-Association-Failed-To-Setup-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TransportLayerAddress	,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-TNL-Association-Failed-To-Setup-Item-ExtIEs} } OPTIONAL

}

GNB-CU-TNL-Association-Failed-To-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-TNL-Association-To-Add-Item ::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TransportLayerAddress	,

tNLAssociationUsage						TNLAssociationUsage,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-TNL-Association-To-Add-Item-ExtIEs} } OPTIONAL

}

GNB-CU-TNL-Association-To-Add-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-TNL-Association-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TransportLayerAddress	,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-TNL-Association-To-Remove-Item-ExtIEs} } OPTIONAL

}

GNB-CU-TNL-Association-To-Remove-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-TNLAssociationTransportLayerAddressgNBDU	CRITICALITY reject	EXTENSION CP-TransportLayerAddress	PRESENCE optional},

...

}

GNB-CU-TNL-Association-To-Update-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress		CP-TransportLayerAddress	,

tNLAssociationUsage						TNLAssociationUsage OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { GNB-CU-TNL-Association-To-Update-Item-ExtIEs} } OPTIONAL

}

GNB-CU-TNL-Association-To-Update-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-UE-F1AP-ID		::= INTEGER (0..4294967295)

GNB-DU-Cell-Resource-Configuration	::= SEQUENCE {

subcarrierSpacing				SubcarrierSpacing,

dUFTransmissionPeriodicity		DUFTransmissionPeriodicity	OPTIONAL,

dUF-Slot-Config-List			DUF-Slot-Config-List	OPTIONAL,

hSNATransmissionPeriodicity		HSNATransmissionPeriodicity,

hsNSASlotConfigList				HSNASlotConfigList	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { GNB-DU-Cell-Resource-Configuration-ExtIEs } } OPTIONAL

}

GNB-DU-Cell-Resource-Configuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-rBSetConfiguration       CRITICALITY reject	EXTENSION       RBSetConfiguration	PRESENCE optional}|

{ID id-frequency-Domain-HSNA-Configuration-List	 CRITICALITY reject	EXTENSION    Frequency-Domain-HSNA-Configuration-List   PRESENCE optional}|

{ID id-child-IAB-Nodes-NA-Resource-List	CRITICALITY reject	EXTENSION Child-IAB-Nodes-NA-Resource-List    PRESENCE optional}|

{ID id-Parent-IAB-Nodes-NA-Resource-Configuration-List   CRITICALITY reject	EXTENSION  Parent-IAB-Nodes-NA-Resource-Configuration-List  PRESENCE optional},

...

}

GNB-DU-MBS-F1AP-ID		::= INTEGER (0..4294967295)

GNB-DU-UE-F1AP-ID		::= INTEGER (0..4294967295)

GNB-DU-ID			::= INTEGER (0..68719476735)

GNB-CU-Name ::= PrintableString(SIZE(1..150,...))

GNB-DU-Name ::= PrintableString(SIZE(1..150,...))

Extended-GNB-CU-Name	 ::= SEQUENCE {

gNB-CU-NameVisibleString		GNB-CU-NameVisibleString					OPTIONAL,

gNB-CU-NameUTF8String			GNB-CU-NameUTF8String						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Extended-GNB-CU-Name-ExtIEs } } OPTIONAL,

...

}

Extended-GNB-CU-Name-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-CU-NameVisibleString ::= VisibleString(SIZE(1..150,...))

GNB-CU-NameUTF8String ::= UTF8String(SIZE(1..150,...))

Extended-GNB-DU-Name	 ::= SEQUENCE {

gNB-DU-NameVisibleString		GNB-DU-NameVisibleString					OPTIONAL,

gNB-DU-NameUTF8String			GNB-DU-NameUTF8String						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Extended-GNB-DU-Name-ExtIEs } } OPTIONAL,

...

}

Extended-GNB-DU-Name-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-DU-NameVisibleString ::= VisibleString(SIZE(1..150,...))

GNB-DU-NameUTF8String ::= UTF8String(SIZE(1..150,...))

GNB-DU-Served-Cells-Item ::= SEQUENCE {

served-Cell-Information		Served-Cell-Information,

gNB-DU-System-Information	GNB-DU-System-Information	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { GNB-DU-Served-Cells-ItemExtIEs} }	OPTIONAL,

...

}

GNB-DU-Served-Cells-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-DU-System-Information ::= SEQUENCE {

mIB-message		MIB-message,

sIB1-message		SIB1-message,

iE-Extensions					ProtocolExtensionContainer { { GNB-DU-System-Information-ExtIEs } } OPTIONAL,

...

}

GNB-DU-System-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SIB12-message		CRITICALITY ignore	EXTENSION SIB12-message		PRESENCE optional}|

{ ID id-SIB13-message		CRITICALITY ignore	EXTENSION SIB13-message		PRESENCE optional}|

{ ID id-SIB14-message		CRITICALITY ignore	EXTENSION SIB14-message		PRESENCE optional}|

{ ID id-SIB10-message		CRITICALITY ignore	EXTENSION SIB10-message		PRESENCE optional}|

{ ID id-SIB17-message		CRITICALITY ignore	EXTENSION SIB17-message		PRESENCE optional}|

{ ID id-SIB20-message		CRITICALITY ignore	EXTENSION SIB20-message		PRESENCE optional}|

{ ID id-SIB15-message		CRITICALITY ignore	EXTENSION SIB15-message		PRESENCE optional},

...

}

GNB-DUConfigurationQuery ::= ENUMERATED {true, ...}

GNBDUOverloadInformation ::= ENUMERATED {overloaded, not-overloaded}

GNB-DU-TNL-Association-To-Remove-Item::= SEQUENCE {

tNLAssociationTransportLayerAddress				CP-TransportLayerAddress	,

tNLAssociationTransportLayerAddressgNBCU		CP-TransportLayerAddress		OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { { GNB-DU-TNL-Association-To-Remove-Item-ExtIEs} } OPTIONAL

}

GNB-DU-TNL-Association-To-Remove-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNBDUUESliceMaximumBitRateList::= SEQUENCE (SIZE(1.. maxnoofSMBRValues)) OF GNBDUUESliceMaximumBitRateItem

GNBDUUESliceMaximumBitRateItem::= SEQUENCE {

sNSSAI		SNSSAI,

uESliceMaximumBitRateUL		BitRate,

iE-Extensions				ProtocolExtensionContainer { { GNBDUUESliceMaximumBitRateItem-ExtIEs} } OPTIONAL,

...

}

GNBDUUESliceMaximumBitRateItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GNB-RxTxTimeDiff ::= SEQUENCE {

rxTxTimeDiff			GNBRxTxTimeDiffMeas,

additionalPath-List		AdditionalPath-List		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { GNB-RxTxTimeDiff-ExtIEs} }  OPTIONAL

}

GNB-RxTxTimeDiff-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedAdditionalPathList		CRITICALITY ignore EXTENSION ExtendedAdditionalPathList 	PRESENCE optional}|

{ ID id-TRPTEGInformation				CRITICALITY ignore EXTENSION TRPTEGInformation			PRESENCE optional },

...

}

GNBRxTxTimeDiffMeas ::= CHOICE {

k0			INTEGER (0.. 1970049),

k1			INTEGER (0.. 985025),

k2			INTEGER (0.. 492513),

k3			INTEGER (0.. 246257),

k4			INTEGER (0.. 123129),

k5			INTEGER (0.. 61565),

choice-extension		ProtocolIE-SingleContainer { { GNBRxTxTimeDiffMeas-ExtIEs } }

}

GNBRxTxTimeDiffMeas-ExtIEs		F1AP-PROTOCOL-IES ::= {

...

}

GNBSetID ::= BIT STRING (SIZE(22))

GTP-TEID				::= OCTET STRING (SIZE (4))

GTPTLAs	::= SEQUENCE (SIZE(1.. maxnoofGTPTLAs)) OF	GTPTLA-Item

GTPTLA-Item	::= SEQUENCE {

gTPTransportLayerAddress				TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { GTPTLA-Item-ExtIEs } }			OPTIONAL

}

GTPTLA-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

GTPTunnel				::= SEQUENCE {

transportLayerAddress		TransportLayerAddress,

gTP-TEID		GTP-TEID,

iE-Extensions				ProtocolExtensionContainer { { GTPTunnel-ExtIEs } } OPTIONAL,

...

}

GTPTunnel-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- H

HandoverPreparationInformation ::= OCTET STRING

HardwareLoadIndicator ::= SEQUENCE {

dLHardwareLoadIndicator			INTEGER (0..100, ...),

uLHardwareLoadIndicator			INTEGER (0..100, ...),

iE-Extensions					ProtocolExtensionContainer { { HardwareLoadIndicator-ExtIEs } } 	OPTIONAL,

...

}

HardwareLoadIndicator-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

HSNASlotConfigList ::= SEQUENCE (SIZE(1..maxnoofHSNASlots)) OF HSNASlotConfigItem

HSNASlotConfigItem 	::=	SEQUENCE {

hSNADownlink			HSNADownlink 		OPTIONAL,

hSNAUplink				HSNAUplink 			OPTIONAL,

hSNAFlexible			HSNAFlexible 		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { HSNASlotConfigItem-ExtIEs } } OPTIONAL

}

HSNASlotConfigItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

HSNADownlink ::= ENUMERATED { hard, soft, notavailable }

HSNAFlexible ::= ENUMERATED { hard, soft, notavailable }

HSNAUplink ::= ENUMERATED { hard, soft, notavailable }

HSNATransmissionPeriodicity ::=	ENUMERATED { ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms5, ms10, ms20, ms40, ms80, ms160, ...}

HashedUEIdentityIndexValue ::= BIT STRING (SIZE(13, ...))

#### -- I

IAB-Barred	::=	ENUMERATED {barred, not-barred, ...}

IABConditionalRRCMessageDeliveryIndication ::= ENUMERATED {true, ...}

IABCongestionIndication ::= SEQUENCE {

iAB-Congestion-Indication-List			IAB-Congestion-Indication-List,

iE-Extensions	ProtocolExtensionContainer { { IAB-Congestion-Indication-List-ExtIEs } } OPTIONAL

}

IAB-Congestion-Indication-List-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-Congestion-Indication-List ::= SEQUENCE (SIZE(1..maxnoofIABCongInd)) OF IAB-Congestion-Indication-Item

IAB-Congestion-Indication-Item ::= SEQUENCE {

childNodeIdentifier				BAPAddress,

bHRLCCHList		                BHRLCCHList	    OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { IAB-Congestion-Indication-ItemExtIEs } }	OPTIONAL

}

IAB-Congestion-Indication-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-Info-IAB-donor-CU ::=	SEQUENCE{

iAB-STC-Info	IAB-STC-Info	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { IAB-Info-IAB-donor-CU-ExtIEs } } OPTIONAL

}

IAB-Info-IAB-donor-CU-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-Info-IAB-DU ::=	SEQUENCE{

multiplexingInfo		MultiplexingInfo	OPTIONAL,

iAB-STC-Info		IAB-STC-Info	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { IAB-Info-IAB-DU-ExtIEs } } OPTIONAL

}

IAB-Info-IAB-DU-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-MT-Cell-List ::= SEQUENCE (SIZE(1..maxnoofServingCells)) OF IAB-MT-Cell-List-Item

IAB-MT-Cell-List-Item ::= 	SEQUENCE {

nRCellIdentity				NRCellIdentity,

dU-RX-MT-RX					DU-RX-MT-RX,

dU-TX-MT-TX					DU-TX-MT-TX,

dU-RX-MT-TX					DU-RX-MT-TX,

dU-TX-MT-RX					DU-TX-MT-RX,

iE-Extensions				ProtocolExtensionContainer { { IAB-MT-Cell-List-Item-ExtIEs } } OPTIONAL

}

IAB-MT-Cell-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-DU-RX-MT-RX-Extend			CRITICALITY ignore	EXTENSION DU-RX-MT-RX-Extend	PRESENCE optional	}|

{ ID id-DU-TX-MT-TX-Extend			CRITICALITY ignore	EXTENSION DU-TX-MT-TX-Extend	PRESENCE optional	}|

{ ID id-DU-RX-MT-TX-Extend			CRITICALITY ignore	EXTENSION DU-RX-MT-TX-Extend	PRESENCE optional	}|

{ ID id-DU-TX-MT-RX-Extend			CRITICALITY ignore	EXTENSION DU-TX-MT-RX-Extend	PRESENCE optional	},

...

}

IAB-MT-Cell-NA-Resource-Configuration-Mode-Info	::=	CHOICE {

fDD		IAB-MT-Cell-NA-Resource-Configuration-FDD-Info,

tDD		IAB-MT-Cell-NA-Resource-Configuration-TDD-Info,

choice-extension			ProtocolIE-SingleContainer { { IAB-MT-Cell-NA-Resource-Configuration-Mode-Info-ExtIEs} }

}

IAB-MT-Cell-NA-Resource-Configuration-Mode-Info-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

IAB-MT-Cell-NA-Resource-Configuration-FDD-Info ::= SEQUENCE {

gNB-DU-Cell-NA-Resource-Configuration-FDD-UL				GNB-DU-Cell-Resource-Configuration,

gNB-DU-Cell-NA-Resource-Configuration-FDD-DL				GNB-DU-Cell-Resource-Configuration,

uL-FreqInfo							NRFreqInfo					OPTIONAL,

uL-Transmission-Bandwidth			Transmission-Bandwidth		OPTIONAL,

uL-NR-Carrier-List  				NRCarrierList				OPTIONAL,

dL-FreqInfo							NRFreqInfo					OPTIONAL,

dL-Transmission-Bandwidth			Transmission-Bandwidth		OPTIONAL,

dL-NR-Carrier-List  				NRCarrierList				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { {IAB-MT-Cell-NA-Resource-Configuration-FDD-Info-ExtIEs} } OPTIONAL,

...

}

IAB-MT-Cell-NA-Resource-Configuration-FDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-MT-Cell-NA-Resource-Configuration-TDD-Info ::= SEQUENCE {

gNB-DU-Cell-NA-Resourc-Configuration-TDD				GNB-DU-Cell-Resource-Configuration,

nRFreqInfo							NRFreqInfo  	OPTIONAL,

transmission-Bandwidth			    Transmission-Bandwidth  	  OPTIONAL,

nR-Carrier-List   		  	        NRCarrierList  	OPTIONAL,

iE-Extensions					    ProtocolExtensionContainer { {IAB-MT-Cell-NA-Resource-Configuration-TDD-Info-ExtIEs} } OPTIONAL,

...

}

IAB-MT-Cell-NA-Resource-Configuration-TDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-STC-Info	::=	SEQUENCE{

iAB-STC-Info-List	IAB-STC-Info-List,

iE-Extensions		ProtocolExtensionContainer { { IAB-STC-Info-ExtIEs } } OPTIONAL

}

IAB-STC-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-STC-Info-List ::= 	SEQUENCE (SIZE(1..maxnoofIABSTCInfo)) OF IAB-STC-Info-Item

IAB-STC-Info-Item::=	SEQUENCE {

sSB-freqInfo						SSB-freqInfo,

sSB-subcarrierSpacing				SSB-subcarrierSpacing,

sSB-transmissionPeriodicity			SSB-transmissionPeriodicity,

sSB-transmissionTimingOffset		SSB-transmissionTimingOffset,

sSB-transmissionBitmap				SSB-transmissionBitmap,

iE-Extensions		ProtocolExtensionContainer { { IAB-STC-Info-Item-ExtIEs } } OPTIONAL

}

IAB-STC-Info-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-Allocated-TNL-Address-Item	::= SEQUENCE {

iABTNLAddress				IABTNLAddress,

iABTNLAddressUsage			IABTNLAddressUsage	 	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { IAB-Allocated-TNL-Address-Item-ExtIEs } } OPTIONAL

}

IAB-Allocated-TNL-Address-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-DU-Cell-Resource-Configuration-Mode-Info	::=	CHOICE {

fDD		IAB-DU-Cell-Resource-Configuration-FDD-Info,

tDD		IAB-DU-Cell-Resource-Configuration-TDD-Info,

choice-extension			ProtocolIE-SingleContainer { { IAB-DU-Cell-Resource-Configuration-Mode-Info-ExtIEs} }

}

IAB-DU-Cell-Resource-Configuration-Mode-Info-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

IAB-DU-Cell-Resource-Configuration-FDD-Info ::= SEQUENCE {

gNB-DU-Cell-Resource-Configuration-FDD-UL				GNB-DU-Cell-Resource-Configuration,

gNB-DU-Cell-Resource-Configuration-FDD-DL				GNB-DU-Cell-Resource-Configuration,

iE-Extensions					ProtocolExtensionContainer { {IAB-DU-Cell-Resource-Configuration-FDD-Info-ExtIEs} } OPTIONAL,

...

}

IAB-DU-Cell-Resource-Configuration-FDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-uL-FreqInfo					CRITICALITY reject	EXTENSION	NRFreqInfo					PRESENCE optional}|

{ID id-uL-Transmission-Bandwidth	CRITICALITY reject	EXTENSION	Transmission-Bandwidth		PRESENCE optional}|

{ID id-uL-NR-Carrier-List			CRITICALITY reject	EXTENSION	NRCarrierList				PRESENCE optional}|

{ID id-dL-FreqInfo					CRITICALITY reject	EXTENSION	NRFreqInfo  				PRESENCE optional}|

{ID id-dL-Transmission-Bandwidth	CRITICALITY reject	EXTENSION	Transmission-Bandwidth		PRESENCE optional}|

{ID id-dL-NR-Carrier-List			CRITICALITY reject	EXTENSION	NRCarrierList				PRESENCE optional},

...

}

IAB-DU-Cell-Resource-Configuration-TDD-Info ::= SEQUENCE {

gNB-DU-Cell-Resourc-Configuration-TDD				GNB-DU-Cell-Resource-Configuration,

iE-Extensions					ProtocolExtensionContainer { {IAB-DU-Cell-Resource-Configuration-TDD-Info-ExtIEs} } OPTIONAL,

...

}

IAB-DU-Cell-Resource-Configuration-TDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-nRFreqInfo				CRITICALITY reject	EXTENSION  NRFreqInfo  				PRESENCE optional}|

{ID id-transmission-Bandwidth	CRITICALITY reject	EXTENSION  Transmission-Bandwidth  	PRESENCE optional}|

{ID id-nR-Carrier-List			CRITICALITY reject	EXTENSION  NRCarrierList  			PRESENCE optional},

...

}

IABIPv6RequestType	 ::= CHOICE {

iPv6Address						IABTNLAddressesRequested,

iPv6Prefix						IABTNLAddressesRequested,

choice-extension				ProtocolIE-SingleContainer { { IABIPv6RequestType-ExtIEs} }

}

IABIPv6RequestType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

IABTNLAddress ::= CHOICE {

iPv4Address						BIT STRING (SIZE(32)),

iPv6Address						BIT STRING (SIZE(128)),

iPv6Prefix						BIT STRING (SIZE(64)),

choice-extension				ProtocolIE-SingleContainer { { IABTNLAddress-ExtIEs} }

}

IABTNLAddress-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

IABTNLAddressesRequested ::= SEQUENCE {

tNLAddressesOrPrefixesRequestedAllTraffic	INTEGER (1..256) 	OPTIONAL,

tNLAddressesOrPrefixesRequestedF1-C			INTEGER (1..256) 	OPTIONAL,

tNLAddressesOrPrefixesRequestedF1-U			INTEGER (1..256) 	OPTIONAL,

tNLAddressesOrPrefixesRequestedNoNF1		INTEGER (1..256) 	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { IABTNLAddressesRequested-ExtIEs } } OPTIONAL

}

IABTNLAddressesRequested-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-TNL-Addresses-To-Remove-Item ::= SEQUENCE {

iABTNLAddress			IABTNLAddress,

iE-Extensions			ProtocolExtensionContainer { { IAB-TNL-Addresses-To-Remove-Item-ExtIEs} } OPTIONAL

}

IAB-TNL-Addresses-To-Remove-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-TNL-Addresses-Exception ::= 	SEQUENCE {

iABTNLAddressList			IABTNLAddressList,

iE-Extensions				ProtocolExtensionContainer { { IAB-TNL-Addresses-Exception-ExtIEs} } OPTIONAL

}

IAB-TNL-Addresses-Exception-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IABTNLAddressList ::= SEQUENCE (SIZE(1.. maxnoofTLAsIAB)) OF IABTNLAddress-Item

IABTNLAddress-Item ::= SEQUENCE {

iABTNLAddress		IABTNLAddress	,

iE-Extensions	ProtocolExtensionContainer { { IABTNLAddress-ItemExtIEs } }	OPTIONAL

}

IABTNLAddress-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

IABTNLAddressUsage ::= ENUMERATED {

f1-c,

f1-u,

non-f1,

...

}

IABv4AddressesRequested ::= SEQUENCE {

iABv4AddressesRequested			IABTNLAddressesRequested,

iE-Extensions		ProtocolExtensionContainer { { IABv4AddressesRequested-ExtIEs} } OPTIONAL

}

IABv4AddressesRequested-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ImplicitFormat	::= SEQUENCE	{

dUFSlotformatIndex 			DUFSlotformatIndex,

iE-Extensions		ProtocolExtensionContainer { { ImplicitFormat-ExtIEs } } OPTIONAL

}

ImplicitFormat-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IgnorePRACHConfiguration::= ENUMERATED { true,...}

IgnoreResourceCoordinationContainer ::= ENUMERATED { yes,...}

InactivityMonitoringRequest ::= ENUMERATED { true,...}

InactivityMonitoringResponse ::= ENUMERATED { not-supported,...}

InterfacesToTrace ::= BIT STRING (SIZE(8))

IntendedTDD-DL-ULConfig ::= SEQUENCE {

nRSCS						ENUMERATED { scs15, scs30, scs60, scs120,..., scs480, scs960},

nRCP						ENUMERATED { normal, extended,...},

nRDLULTxPeriodicity			ENUMERATED { ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms3, ms4, ms5, ms10, ms20, ms40, ms60, ms80, ms100, ms120, ms140, ms160, ...},

slot-Configuration-List 	Slot-Configuration-List,

iE-Extensions						ProtocolExtensionContainer { {IntendedTDD-DL-ULConfig-ExtIEs} } OPTIONAL

}

InterFrequencyConfig-NoGap ::= ENUMERATED {

true,

...

}

IngressNonF1terminatingTopologyIndicator ::= ENUMERATED {true, ...}

IntendedTDD-DL-ULConfig-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

IPHeaderInformation ::= SEQUENCE {

destinationIABTNLAddress			IABTNLAddress,

dsInformationList					DSInformationList	OPTIONAL,

iPv6FlowLabel						BIT STRING (SIZE (20))	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { IPHeaderInformation-ItemExtIEs} } OPTIONAL,

...

}

IPHeaderInformation-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

IPtolayer2TrafficMappingInfo ::= SEQUENCE {

iPtolayer2TrafficMappingInfoToAdd					IPtolayer2TrafficMappingInfoList		OPTIONAL,

iPtolayer2TrafficMappingInfoToRemove				MappingInformationtoRemove				OPTIONAL,

iE-Extensions										ProtocolExtensionContainer { { IPtolayer2TrafficMappingInfo-ItemExtIEs} } OPTIONAL,

...

}

IPtolayer2TrafficMappingInfoList ::= SEQUENCE (SIZE(1..maxnoofMappingEntries)) OF IPtolayer2TrafficMappingInfo-Item

IPtolayer2TrafficMappingInfo-Item ::= SEQUENCE {

mappingInformationIndex		MappingInformationIndex,

iPHeaderInformation			IPHeaderInformation,

bHInfo	 					BHInfo,	iE-Extensions				ProtocolExtensionContainer { { IPtolayer2TrafficMappingInfo-ItemExtIEs} } OPTIONAL,

...

}

IPtolayer2TrafficMappingInfo-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- J

#### -- K

#### -- L

L139Info ::= SEQUENCE {

prachSCS						ENUMERATED {scs15, scs30, scs60, scs120, ..., scs480, scs960},

rootSequenceIndex			INTEGER (0..137)								OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {L139Info-ExtIEs} } 		OPTIONAL,

...

}

L139Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

L839Info ::= SEQUENCE {

rootSequenceIndex			INTEGER (0..837),

restrictedSetConfig			ENUMERATED {unrestrictedSet, restrictedSetTypeA,

restrictedSetTypeB, ...},

iE-Extension		ProtocolExtensionContainer { {L839Info-ExtIEs} } 		OPTIONAL,

...

}

L839Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

L571Info ::= SEQUENCE {

prachSCSForL571				ENUMERATED { scs30, scs120, ... , scs480},

rootSequenceIndex			INTEGER (0..569),

iE-Extension				ProtocolExtensionContainer { {L571Info-ExtIEs} } 		OPTIONAL,

...

}

L571Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

L1151Info ::= SEQUENCE {

prachSCSForL1151				ENUMERATED { scs15, scs120,...},

rootSequenceIndex				INTEGER (0..1149),

iE-Extension					ProtocolExtensionContainer { {L1151Info-ExtIEs} } 		OPTIONAL,

...

}

L1151Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LastUsedCellIndication ::= ENUMERATED {true, ...}

LCID ::= INTEGER (1..32, ...)

LCS-to-GCS-Translation::= SEQUENCE {

alpha				INTEGER (0..3599),

beta				INTEGER (0..3599),

gamma				INTEGER (0..3599),

iE-Extensions		ProtocolExtensionContainer { { LCS-to-GCS-Translation-ExtIEs} } OPTIONAL,

...

}

LCS-to-GCS-Translation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LCStoGCSTranslationList ::= SEQUENCE (SIZE (1.. maxnooflcs-gcs-translation)) OF LCStoGCSTranslation

LCStoGCSTranslation ::= SEQUENCE {

alpha			INTEGER (0..359),

alpha-fine		INTEGER (0..9)		OPTIONAL,

beta			INTEGER (0..359),

beta-fine		INTEGER (0..9)		OPTIONAL,

gamma			INTEGER (0..359),

gamma-fine		INTEGER (0..9)		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {LCStoGCSTranslation-ExtIEs} } OPTIONAL

}

LCStoGCSTranslation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LMF-MeasurementID ::= INTEGER (1.. 65536, ...)

LMF-UE-MeasurementID ::= INTEGER (1.. 256, ...)

LocationDependentMBSF1UInformation ::= SEQUENCE (SIZE(1..maxnoofMBSAreaSessionIDs)) OF LocationDependentMBSF1UInformation-Item

LocationDependentMBSF1UInformation-Item ::= SEQUENCE {

mbsAreaSession-ID				MBS-Area-Session-ID,

mbs-f1u-info-at-CU				UPTransportLayerInformation,

iE-Extensions					ProtocolExtensionContainer	{ { LocationDependentMBSF1UInformation-Item-ExtIEs } }	OPTIONAL,

...

}

LocationDependentMBSF1UInformation-Item-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

LocationMeasurementInformation ::= OCTET STRING

LocationUncertainty ::= SEQUENCE {

horizontalUncertainty		INTEGER (0..255),

horizontalConfidence		INTEGER (0..100),

verticalUncertainty			INTEGER (0..255),

verticalConfidence			INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { LocationUncertainty-ExtIEs} } OPTIONAL

}

LocationUncertainty-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LongDRXCycleLength ::= 	ENUMERATED

{ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, ...}

LowerLayerPresenceStatusChange ::= ENUMERATED {

suspend-lower-layers,

resume-lower-layers,

...

}

LoS-NLoSIndicatorHard ::= ENUMERATED {nLoS, loS}

LoS-NLoSIndicatorSoft ::= INTEGER (0..10)

LoS-NLoSInformation ::= CHOICE {

loS-NLoSIndicatorSoft		LoS-NLoSIndicatorSoft,

loS-NLoSIndicatorHard		LoS-NLoSIndicatorHard,

choice-Extension			ProtocolIE-SingleContainer {{ LoS-NLoSInformation-ExtIEs}}

}

LoS-NLoSInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

LTEUESidelinkAggregateMaximumBitrate ::= SEQUENCE {

uELTESidelinkAggregateMaximumBitrate		BitRate,

iE-Extensions					ProtocolExtensionContainer { {LTEUESidelinkAggregateMaximumBitrate-ExtIEs} } OPTIONAL

}

LTEUESidelinkAggregateMaximumBitrate-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTEV2XServicesAuthorized ::= SEQUENCE {

vehicleUE			VehicleUE														OPTIONAL,

pedestrianUE 		PedestrianUE													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LTEV2XServicesAuthorized-ExtIEs} }		OPTIONAL

}

LTEV2XServicesAuthorized-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- M

MappingInformationIndex	::= BIT STRING (SIZE (26))

MappingInformationtoRemove	::= SEQUENCE (SIZE(1..maxnoofMappingEntries)) OF MappingInformationIndex

MaskedIMEISV ::= 	BIT STRING (SIZE (64))

MaxDataBurstVolume  ::= INTEGER (0..4095, ..., 4096.. 2000000)

MaxPacketLossRate ::= INTEGER (0..1000)

MBS-Broadcast-NeighbourCellList ::= OCTET STRING

MBS-Flows-Mapped-To-MRB-List	::=	SEQUENCE (SIZE(1.. maxnoofMBSQoSFlows)) OF MBS-Flows-Mapped-To-MRB-Item

MBS-Flows-Mapped-To-MRB-Item 	::= SEQUENCE {

mBS-QoSFlowIdentifier						QoSFlowIdentifier,

mbs-QoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

iE-Extensions								ProtocolExtensionContainer { { MBS-Flows-Mapped-To-MRB-Item-ExtIEs} } OPTIONAL

}

MBS-Flows-Mapped-To-MRB-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSF1UInformation ::= SEQUENCE {

mbs-f1u-info				UPTransportLayerInformation,

iE-Extensions					ProtocolExtensionContainer	{ { MBSF1UInformation-ExtIEs } }	OPTIONAL,

...

}

MBSF1UInformation-ExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSInterestIndication ::= OCTET STRING

MBS-Session-ID ::= SEQUENCE {

tMGI						TMGI,

nID							NID					OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Session-ID-ExtIEs} } OPTIONAL,

...

}

MBS-Session-ID-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Area-Session-ID  ::= INTEGER (0..65535, ...)

MBS-CUtoDURRCInformation		::= SEQUENCE {

mBS-Broadcast-Cell-List		MBS-Broadcast-Cell-List,

mBS-Broadcast-MRB-List		MBS-Broadcast-MRB-List,

iE-Extensions				ProtocolExtensionContainer { { MBS-CUtoDURRCInformation-ExtIEs } } OPTIONAL,

...

}

MBS-CUtoDURRCInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Broadcast-Cell-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF  MBS-Broadcast-Cell-Item

MBS-Broadcast-Cell-Item ::= SEQUENCE {

nRCGI						NRCGI,

mtch-neighbourCell			OCTET STRING		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MBS-Broadcast-Cell-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Broadcast-Cell-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-Broadcast-MRB-List	::= SEQUENCE (SIZE(1.. maxnoofMRBs))	OF  MBS-Broadcast-MRB-Item

MBS-Broadcast-MRB-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

mRB-PDCP-Config-Broadcast	OCTET STRING,

iE-Extensions				ProtocolExtensionContainer { { MBS-Broadcast-MRB-Item-ExtIEs} } OPTIONAL,

...

}

MBS-Broadcast-MRB-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastF1UContextDescriptor ::= SEQUENCE {

multicastF1UContextReferenceF1		MulticastF1UContextReferenceF1,

mc-F1UCtxtusage 	ENUMERATED {ptm, ptp, ptp-retransmission, ptp-forwarding, ...},

mbsAreaSession						MBS-Area-Session-ID				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer	{{MBSMulticastF1UContextDescriptor-ExtIEs}} OPTIONAL,

...

}

MBSMulticastF1UContextDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastF1UContext-ToBeSetup-Item ::= SEQUENCE {

mRB-ID                  MRB-ID,

mbs-f1u-info-at-DU      UPTransportLayerInformation,

mbsProgressInformation		MRB-ProgressInformation						OPTIONAL,

iE-Extensions           ProtocolExtensionContainer { {MulticastF1UContext-ToBeSetup-Item-ExtIEs} }	OPTIONAL,

...

}

MulticastF1UContext-ToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastF1UContext-Setup-Item ::= SEQUENCE {

mRB-ID                  MRB-ID,

mbs-f1u-info-at-CU      UPTransportLayerInformation,

iE-Extensions           ProtocolExtensionContainer { {MulticastF1UContext-Setup-Item-ExtIEs} }	OPTIONAL,

...

}

MulticastF1UContext-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastF1UContext-FailedToBeSetup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions           ProtocolExtensionContainer { {MulticastF1UContext-FailedToBeSetup-Item-ExtIEs} }	OPTIONAL,

...

}

MulticastF1UContext-FailedToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSPTPRetransmissionTunnelRequired ::= ENUMERATED {true,	...}

MBS-ServiceArea ::= CHOICE {

locationindependent		MBS-ServiceAreaInformation,

locationdependent		MBS-ServiceAreaInformationList,

choice-Extensions		ProtocolIE-SingleContainer { {MBSServiceArea-ExtIEs} }

}

MBSServiceArea-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MBS-ServiceAreaInformation ::= SEQUENCE {

mBS-ServiceAreaCellList		MBS-ServiceAreaCellList																OPTIONAL,

mBS-ServiceAreaTAIList		MBS-ServiceAreaTAIList																OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaInformation-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaCellList ::= SEQUENCE (SIZE(1.. maxnoofCellsforMBS)) OF NRCGI

MBS-ServiceAreaTAIList ::= SEQUENCE (SIZE(1.. maxnoofTAIforMBS)) OF MBS-ServiceAreaTAIList-Item

MBS-ServiceAreaTAIList-Item ::= SEQUENCE {

plmn-ID							PLMN-Identity,

five5-TAC						FiveGS-TAC,

iE-Extensions					ProtocolExtensionContainer { {MBS-ServiceAreaTAIList-Item-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaTAIList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MBS-ServiceAreaInformationList ::= SEQUENCE (SIZE(1..maxnoofMBSServiceAreaInformation)) OF MBS-ServiceAreaInformationItem

MBS-ServiceAreaInformationItem ::= SEQUENCE {

mBS-AreaSessionID					MBS-Area-Session-ID,

mBS-ServiceAreaInformation 			MBS-ServiceAreaInformation,

iE-Extensions						ProtocolExtensionContainer { { MBS-ServiceAreaInformationItem-ExtIEs} }	OPTIONAL,

...

}

MBS-ServiceAreaInformationItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MC-PagingCell-Item ::= SEQUENCE {

nRCGI			NRCGI,

iE-Extensions	ProtocolExtensionContainer { { MC-PagingCell-ItemExtIEs } }	OPTIONAL

}

MC-PagingCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MIB-message ::= OCTET STRING

MeasConfig ::= OCTET STRING

MeasGapConfig ::= OCTET STRING

MeasGapSharingConfig ::= OCTET STRING

PosMeasurementAmount ::= ENUMERATED {ma0, ma1, ma2, ma4, ma8, ma16, ma32, ma64}

MeasurementBeamInfoRequest ::= ENUMERATED {true, ...}

MeasurementBeamInfo	 ::= SEQUENCE {

pRS-Resource-ID				PRS-Resource-ID		OPTIONAL,

pRS-Resource-Set-ID			PRS-Resource-Set-ID	OPTIONAL,

sSB-Index					SSB-Index			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MeasurementBeamInfo-ExtIEs} } OPTIONAL

}

MeasurementBeamInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MeasurementTimingConfiguration ::= OCTET STRING

MessageIdentifier ::= BIT STRING (SIZE (16))

MeasurementTimeOccasion ::= ENUMERATED {o1, o4, ...}

MeasurementCharacteristicsRequestIndicator ::= BIT STRING (SIZE (16))

MRB-ProgressInformation ::= CHOICE {

pdcp-SN12			INTEGER (0..4095),

pdcp-SN18			INTEGER (0..262143),

choice-extension		ProtocolIE-SingleContainer { { MRB-ProgressInformation-ExtIEs} }

}

MRB-ProgressInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MulticastF1UContextReferenceF1 ::= OCTET STRING (SIZE(4))

MulticastF1UContextReferenceCU ::= OCTET STRING (SIZE(4))

MultipleULAoA ::= SEQUENCE {

multipleULAoA					MultipleULAoA-List,

iE-Extensions					ProtocolExtensionContainer { { MultipleULAoA-ExtIEs} } OPTIONAL,

...

}

MultipleULAoA-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MultipleULAoA-List ::= SEQUENCE (SIZE(1.. maxnoofULAoAs)) OF MultipleULAoA-Item

MultipleULAoA-Item ::= CHOICE {

uL-AoA		UL-AoA,

ul-ZoA		ZoAInformation,

choice-extension ProtocolIE-SingleContainer { { MultipleULAoA-Item-ExtIEs } }

}

MultipleULAoA-Item-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

MDTPollutedMeasurementIndicator	::= ENUMERATED {iDC,no-IDC, ...}

MRB-ID ::= INTEGER (1..512, ...)

MulticastMBSSessionList ::= SEQUENCE (SIZE(1..maxnoofMBSSessionsofUE)) OF MulticastMBSSessionList-Item

MulticastMBSSessionList-Item ::= SEQUENCE {

mbsSessionId				MBS-Session-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMBSSessionList-Item-ExtIEs } } OPTIONAL,

...

}

MulticastMBSSessionList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-FailedToBeModified-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-FailedtoBeModified-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-FailedtoBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-FailedToBeSetup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-FailedToBeSetup-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-FailedToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-FailedToBeSetupMod-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

cause						Cause							OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-FailedToBeSetupMod-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-FailedToBeSetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-Modified-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-Modified-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-Modified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-Setup-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-Setup-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-SetupMod-Item ::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-SetupMod-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-SetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-ToBeModified-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters		OPTIONAL,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List	OPTIONAL,

mBS-DL-PDCP-SN-Length			PDCPSNLength 					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { MulticastMRBs-ToBeModified-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-ToBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-ToBeReleased-Item	::= SEQUENCE {

mRB-ID						MRB-ID,

iE-Extensions				ProtocolExtensionContainer { { MulticastMRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

MulticastMRBs-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-ToBeSetup-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List,

mBS-DL-PDCP-SN-Length			PDCPSNLength,

iE-Extensions					ProtocolExtensionContainer { { MulticastMRBs-ToBeSetup-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-ToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MulticastMRBs-ToBeSetupMod-Item ::= SEQUENCE {

mRB-ID							MRB-ID,

mRB-QoSInformation				QoSFlowLevelQoSParameters,

mBS-Flows-Mapped-To-MRB-List	MBS-Flows-Mapped-To-MRB-List,

mBS-DL-PDCP-SN-Length			PDCPSNLength,

iE-Extensions					ProtocolExtensionContainer { { MulticastMRBs-ToBeSetupMod-Item-ExtIEs} } OPTIONAL,

...

}

MulticastMRBs-ToBeSetupMod-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MultiplexingInfo 	::=	SEQUENCE{

iAB-MT-Cell-List 	IAB-MT-Cell-List,

iE-Extensions		ProtocolExtensionContainer { {MultiplexingInfo-ExtIEs} } OPTIONAL

}

MultiplexingInfo-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

M2Configuration ::= ENUMERATED {true, ...}

M5Configuration ::= SEQUENCE {

m5period			M5period,

m5-links-to-log		M5-Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M5Configuration-ExtIEs} } OPTIONAL,

...

}

M5Configuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-M5ReportAmount	CRITICALITY ignore	EXTENSION M5ReportAmount PRESENCE optional },

...

}

M5period ::= ENUMERATED { ms1024, ms2048, ms5120, ms10240, min1, ... }

M5ReportAmount	::= ENUMERATED { r1, r2, r4, r8, r16, r32, r64, infinity, ... }

M5-Links-to-log	::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}

M6Configuration ::= SEQUENCE {

m6report-Interval	M6report-Interval,

m6-links-to-log		M6-Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M6Configuration-ExtIEs} } OPTIONAL,

...

}

M6Configuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-M6ReportAmount	CRITICALITY ignore	EXTENSION M6ReportAmount PRESENCE optional },

...

}

M6report-Interval ::= ENUMERATED { ms120, ms240, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, ..., ms480}

M6ReportAmount	::= ENUMERATED { r1, r2, r4, r8, r16, r32, r64, infinity, ... }

M6-Links-to-log	::= ENUMERATED {uplink, downlink, both-uplink-and-downlink, ...}

M7Configuration ::= SEQUENCE {

m7period			M7period,

m7-links-to-log		M7-Links-to-log,

iE-Extensions		ProtocolExtensionContainer { { M7Configuration-ExtIEs} } OPTIONAL,

...

}

M7Configuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-M7ReportAmount	CRITICALITY ignore	EXTENSION M7ReportAmount PRESENCE optional},

...

}

M7period	::= INTEGER(1..60, ...)

M7ReportAmount	::= ENUMERATED { r1, r2, r4, r8, r16, r32, r64, infinity, ... }

M7-Links-to-log	::= ENUMERATED {downlink, ...}

MDT-Activation ::= ENUMERATED {

immediate-MDT-only,

immediate-MDT-and-Trace,

...

}

MDTConfiguration ::= SEQUENCE {

mdt-Activation				MDT-Activation,

measurementsToActivate		MeasurementsToActivate,

m2Configuration				M2Configuration		OPTIONAL,

--  C-ifM2: This IE shall be present if the Measurements to Activate IE has the second bit set to "1".

m5Configuration				M5Configuration		OPTIONAL,

--  C-ifM5: This IE shall be present if the Measurements to Activate IE has the fifth bit set to "1".

m6Configuration				M6Configuration		OPTIONAL,

--  C-ifM6: This IE shall be present if the Measurements to Activate IE has the seventh bit set to "1".

m7Configuration				M7Configuration		OPTIONAL,

--  C-ifM7: This IE shall be present if the Measurements to Activate IE has the eighth bit set to "1".

iE-Extensions				ProtocolExtensionContainer { { MDTConfiguration-ExtIEs} } OPTIONAL,

...

}

MDTConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MDTPLMNList ::= SEQUENCE (SIZE(1..maxnoofMDTPLMNs)) OF PLMN-Identity

MDTPLMNModificationList ::= SEQUENCE (SIZE(0..maxnoofMDTPLMNs)) OF PLMN-Identity

MeasuredResultsValue ::= CHOICE {

uL-AngleOfArrival	UL-AoA,

uL-SRS-RSRP			UL-SRS-RSRP,

uL-RTOA				UL-RTOA-Measurement,

gNB-RxTxTimeDiff	GNB-RxTxTimeDiff,

choice-extension	ProtocolIE-SingleContainer { { MeasuredResultsValue-ExtIEs } }

}

MeasuredResultsValue-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-ZoAInformation	CRITICALITY reject TYPE ZoAInformation	PRESENCE mandatory}|

{ ID id-MultipleULAoA	CRITICALITY reject TYPE MultipleULAoA	PRESENCE mandatory}|

{ ID id-UL-SRS-RSRPP	CRITICALITY reject TYPE UL-SRS-RSRPP	PRESENCE mandatory},

...

}

MeasurementsToActivate ::= BIT STRING (SIZE (8))

MUSIM-GapConfig ::= OCTET STRING

#### -- N

NA-Resource-Configuration-List ::= SEQUENCE (SIZE(1.. maxnoofHSNASlots)) OF NA-Resource-Configuration-Item

NA-Resource-Configuration-Item ::= SEQUENCE {

nADownlink					NADownlink 	    OPTIONAL,

nAUplink					NAUplink 	    OPTIONAL,

nAFlexible					NAFlexible 	    OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { NA-Resource-Configuration-Item-ExtIEs} } OPTIONAL

}

NA-Resource-Configuration-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NADownlink ::= ENUMERATED { true, false, ...}

NAFlexible ::= ENUMERATED { true, false, ...}

NAUplink ::= ENUMERATED { true, false, ...}

Ncd-SSB-RedCapInitialBWP-SDT ::= OCTET STRING

Neighbour-Node-Cells-List ::= SEQUENCE (SIZE(1..maxnoofNeighbourNodeCellsIAB)) OF Neighbour-Node-Cells-List-Item

Neighbour-Node-Cells-List-Item ::= SEQUENCE{

nRCGI			 								NRCGI,

gNB-CU-UE-F1AP-ID	GNB-CU-UE-F1AP-ID 			OPTIONAL,

gNB-DU-UE-F1AP-ID	GNB-DU-UE-F1AP-ID 			OPTIONAL,

peer-Parent-Node-Indicator						ENUMERATED {true, ...} 	OPTIONAL,

iAB-DU-Cell-Resource-Configuration-Mode-Info	IAB-DU-Cell-Resource-Configuration-Mode-Info 	OPTIONAL,

iAB-STC-Info									IAB-STC-Info	OPTIONAL,

rACH-Config-Common								RACH-Config-Common	OPTIONAL,

rACH-Config-Common-IAB							RACH-Config-Common-IAB	OPTIONAL,

cSI-RS-Configuration							OCTET STRING	OPTIONAL,

sR-Configuration								OCTET STRING	OPTIONAL,

pDCCH-ConfigSIB1								OCTET STRING	OPTIONAL,

sCS-Common										OCTET STRING	OPTIONAL,

iE-Extensions									ProtocolExtensionContainer {{Neighbour-Node-Cells-List-Item-ExtIEs}}		OPTIONAL

}

Neighbour-Node-Cells-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NeedforGap::= ENUMERATED {true, ...}

NeedForGapsInfoNR ::= OCTET STRING

NeedForGapNCSGInfoNR ::= OCTET STRING

NeedForGapNCSGInfoEUTRA ::= OCTET STRING

Neighbour-Cell-Information-Item ::= SEQUENCE {

nRCGI				NRCGI,

intendedTDD-DL-ULConfig		IntendedTDD-DL-ULConfig OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { Neighbour-Cell-Information-ItemExtIEs } }	OPTIONAL

}

Neighbour-Cell-Information-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NeighbourNR-CellsForSON-List ::= SEQUENCE (SIZE(1.. maxNeighbourCellforSON)) OF NeighbourNR-CellsForSON-Item

NeighbourNR-CellsForSON-Item ::= SEQUENCE {

nRCGI											NRCGI,

nR-ModeInfoRel16								NR-ModeInfoRel16												OPTIONAL,

sSB-PositionsInBurst							SSB-PositionsInBurst											OPTIONAL,

nRPRACHConfig									NRPRACHConfig													OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { NeighbourNR-CellsForSON-Item-ExtIEs} }	OPTIONAL,

...

}

NeighbourNR-CellsForSON-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NGRANAllocationAndRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions				ProtocolExtensionContainer { {NGRANAllocationAndRetentionPriority-ExtIEs} } OPTIONAL

}

NGRANAllocationAndRetentionPriority-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NGRANHighAccuracyAccessPointPosition ::= SEQUENCE {

latitude					INTEGER (-2147483648.. 2147483647),

longitude					INTEGER (-2147483648.. 2147483647),

altitude					INTEGER (-64000..1280000),

uncertaintySemi-major		INTEGER (0..255),

uncertaintySemi-minor		INTEGER (0..255),

orientationOfMajorAxis		INTEGER (0..179),

horizontalConfidence		INTEGER (0..100),

uncertaintyAltitude			INTEGER (0..255),

verticalConfidence			INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { NGRANHighAccuracyAccessPointPosition-ExtIEs} } OPTIONAL

}

NGRANHighAccuracyAccessPointPosition-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NID ::= BIT STRING (SIZE(44))

NonF1terminatingTopologyIndicator ::= ENUMERATED {

true,

...

}

NR-CGI-List-For-Restart-Item ::= SEQUENCE {

nRCGI				NRCGI,

iE-Extensions		ProtocolExtensionContainer { { NR-CGI-List-For-Restart-ItemExtIEs } }	OPTIONAL,

...

}

NR-CGI-List-For-Restart-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NrofSymbolsExtended ::=  ENUMERATED {n8, n10, n12, n14, ...}

NR-PRSBeamInformation ::= SEQUENCE {

nR-PRSBeamInformationList		NR-PRSBeamInformationList,

lCStoGCSTranslationList 		LCStoGCSTranslationList		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { NR-PRSBeamInformation-ExtIEs } } OPTIONAL

}

NR-PRSBeamInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-PRSBeamInformationList ::= SEQUENCE (SIZE(1.. maxnoofPRS-ResourceSets)) OF NR-PRSBeamInformationItem

NR-PRSBeamInformationItem ::= SEQUENCE {

pRSResourceSetID	PRS-Resource-Set-ID,

pRSAngleList		PRSAngleList,

iE-Extensions		ProtocolExtensionContainer { { NR-PRSBeamInformationItem-ExtIEs } } OPTIONAL

}

NR-PRSBeamInformationItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-TADV ::= INTEGER (0.. 7690)

NRRedCapUEIndication ::= ENUMERATED {true, ...}

NRPagingeDRXInformation ::= SEQUENCE {

nrpaging-eDRX-Cycle-Idle		NRPaging-eDRX-Cycle-Idle,

nrpaging-Time-Window			NRPaging-Time-Window												OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {NRPagingeDRXInformation-ExtIEs} }	OPTIONAL,

...

}

NRPagingeDRXInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRPaging-eDRX-Cycle-Idle ::= ENUMERATED {

hfquarter, hfhalf, hf1, hf2, hf4,

hf8, hf16, hf32, hf64, hf128, hf256, hf512, hf1024,

...

}

NRPaging-Time-Window ::= ENUMERATED {

s1, s2, s3, s4, s5,

s6, s7, s8, s9, s10,

s11, s12, s13, s14, s15, s16,

...,

s17, s18, s19, s20, s21,

s22, s23, s24, s25, s26,

s27, s28, s29, s30, s31, s32

}

NRPagingeDRXInformationforRRCINACTIVE ::= SEQUENCE {

nrpaging-eDRX-Cycle-Inactive		NRPaging-eDRX-Cycle-Inactive,

iE-Extensions			ProtocolExtensionContainer { { NRPagingeDRXInformationforRRCINACTIVE-ExtIEs} }	OPTIONAL,

...

}

NRPagingeDRXInformationforRRCINACTIVE-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRPaging-eDRX-Cycle-Inactive ::= ENUMERATED {

hfquarter, hfhalf, hf1,

...

}

NonDynamic5QIDescriptor	::= SEQUENCE {

fiveQI						INTEGER (0..255, ...),

qoSPriorityLevel			INTEGER (1..127)				OPTIONAL,

averagingWindow 			AveragingWindow					OPTIONAL,

maxDataBurstVolume			MaxDataBurstVolume				OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { NonDynamic5QIDescriptor-ExtIEs } } OPTIONAL

}

NonDynamic5QIDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CNPacketDelayBudgetDownlink	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional	}|

{ ID id-CNPacketDelayBudgetUplink	CRITICALITY ignore	EXTENSION ExtendedPacketDelayBudget		PRESENCE optional	},

...

}

NonDynamicPQIDescriptor	::= SEQUENCE {

fiveQI						INTEGER (0..255, ...),

qoSPriorityLevel			INTEGER (1..8, ...)				OPTIONAL,

averagingWindow 			AveragingWindow					OPTIONAL,

maxDataBurstVolume			MaxDataBurstVolume				OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { NonDynamicPQIDescriptor-ExtIEs } } OPTIONAL

}

NonDynamicPQIDescriptor-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NonUPTrafficType ::=	ENUMERATED {ue-associated, non-ue-associated, non-f1, bap-control-pdu,...}

NoofDownlinkSymbols	::= INTEGER (0..14)

NoofUplinkSymbols	::= INTEGER (0..14)

Notification-Cause ::= ENUMERATED {fulfilled, not-fulfilled, ...}

NotificationControl ::= ENUMERATED {active, not-active, ...}

NotificationInformation ::= SEQUENCE {

message-Identifier	MessageIdentifier,

serialNumber		SerialNumber,

iE-Extensions	ProtocolExtensionContainer { { NotificationInformationExtIEs} } OPTIONAL,

...

}

NotificationInformationExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

...

}

NPNBroadcastInformation ::= CHOICE {

sNPN-Broadcast-Information					NPN-Broadcast-Information-SNPN,

pNI-NPN-Broadcast-Information				NPN-Broadcast-Information-PNI-NPN,

choice-extension					ProtocolIE-SingleContainer { {NPNBroadcastInformation-ExtIEs} }

}

NPNBroadcastInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

NPN-Broadcast-Information-SNPN ::= SEQUENCE {

broadcastSNPNID-List		BroadcastSNPN-ID-List,

iE-Extension				ProtocolExtensionContainer { {NPN-Broadcast-Information-SNPN-ExtIEs} }	OPTIONAL,

...

}

NPN-Broadcast-Information-SNPN-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NPN-Broadcast-Information-PNI-NPN ::= SEQUENCE {

broadcastPNI-NPN-ID-Information		BroadcastPNI-NPN-ID-List,

iE-Extension							ProtocolExtensionContainer { {NPN-Broadcast-Information-PNI-NPN-ExtIEs} }	OPTIONAL,

...

}

NPN-Broadcast-Information-PNI-NPN-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NPNSupportInfo ::= CHOICE {

sNPN-Information		NID,

choice-extension		ProtocolIE-SingleContainer { { NPNSupportInfo-ExtIEs } }

}

NPNSupportInfo-ExtIEs		F1AP-PROTOCOL-IES ::= {

...

}

NRCarrierList ::= SEQUENCE (SIZE(1..maxnoofNRSCSs)) OF NRCarrierItem

NRCarrierItem ::= SEQUENCE {

carrierSCS						NRSCS,

offsetToCarrier					INTEGER (0..2199, ...),

carrierBandwidth				INTEGER (0..maxnoofPhysicalResourceBlocks, ...),

iE-Extension					ProtocolExtensionContainer { {NRCarrierItem-ExtIEs} } 				OPTIONAL,

...

}

NRCarrierItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRFreqInfo ::=  SEQUENCE {

nRARFCN			INTEGER (0..maxNRARFCN),

sul-Information	SUL-Information		OPTIONAL,

freqBandListNr	SEQUENCE (SIZE(1..maxnoofNrCellBands)) OF FreqBandNrItem,

iE-Extensions	ProtocolExtensionContainer { { NRFreqInfoExtIEs} } OPTIONAL,

...

}

NRFreqInfoExtIEs		F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-FrequencyShift7p5khz	CRITICALITY ignore	EXTENSION FrequencyShift7p5khz	PRESENCE optional },

...

}

NRCGI ::= SEQUENCE {

pLMN-Identity			PLMN-Identity,

nRCellIdentity			NRCellIdentity,

iE-Extensions			ProtocolExtensionContainer { {NRCGI-ExtIEs} } OPTIONAL,

...

}

NRCGI-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-Mode-Info ::= CHOICE {

fDD		FDD-Info,

tDD		TDD-Info,

choice-extension			ProtocolIE-SingleContainer { { NR-Mode-Info-ExtIEs} }

}

NR-Mode-Info-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-NR-U		CRITICALITY ignore	TYPE NR-U-Channel-Info-List PRESENCE mandatory},

...

}

NR-ModeInfoRel16 ::= CHOICE {

fDD												FDD-InfoRel16,

tDD												TDD-InfoRel16,

choice-extension								ProtocolIE-SingleContainer { { NR-ModeInfoRel16-ExtIEs} }

}

NR-ModeInfoRel16-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

NRPRACHConfig ::= SEQUENCE {

ulPRACHConfigList			NRPRACHConfigList									OPTIONAL,

sulPRACHConfigList			NRPRACHConfigList									OPTIONAL,

iE-Extension				ProtocolExtensionContainer { {NRPRACHConfig-ExtIEs} } 	OPTIONAL,

...

}

NRPRACHConfig-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRCellIdentity ::= BIT STRING (SIZE(36))

NRNRB ::= ENUMERATED { nrb11, nrb18, nrb24, nrb25, nrb31, nrb32, nrb38, nrb51, nrb52, nrb65, nrb66, nrb78, nrb79, nrb93, nrb106, nrb107, nrb121, nrb132, nrb133, nrb135, nrb160, nrb162, nrb189, nrb216, nrb217, nrb245, nrb264, nrb270, nrb273, ..., nrb33, nrb62, nrb124, nrb148, nrb248, nrb44, nrb58, nrb92, nrb119, nrb188, nrb242}

NRPCI ::= INTEGER(0..1007)

NRPRACHConfigList ::= SEQUENCE (SIZE(0..maxnoofPRACHconfigs)) OF NRPRACHConfigItem

NRPRACHConfigItem ::= SEQUENCE {

nRSCS					NRSCS,

prachFreqStartfromCarrier	INTEGER (0..maxnoofPhysicalResourceBlocks-1, ...),

prachFDM					ENUMERATED {one, two, four, eight, ...},

prachConfigIndex			INTEGER (0..255, ..., 256..262),

ssb-perRACH-Occasion		ENUMERATED {oneEighth, oneFourth, oneHalf, one,

two, four, eight, sixteen, ...},

freqDomainLength			FreqDomainLength,

zeroCorrelZoneConfig		INTEGER (0..15),

iE-Extension		ProtocolExtensionContainer { { NRPRACHConfigItem-ExtIEs} } 		OPTIONAL,

...

}

NRPRACHConfigItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRSCS ::= ENUMERATED { scs15, scs30, scs60, scs120, ..., scs480, scs960}

NRUERLFReportContainer ::= OCTET STRING

NR-U-Channel-Info-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Info-Item

NR-U-Channel-Info-Item ::= SEQUENCE {

nr-U-channel-ID			INTEGER(1.. maxnoofNR-UChannelIDs,...),

nR-ARFCN				INTEGER (0..maxNRARFCN),

bandwidth				ENUMERATED{mHz-10,mHz-20,mHz-40, mHz-60, mHz-80,...,mHz-100},

iE-Extensions			ProtocolExtensionContainer { { NR-U-Channel-Info-List-ExtIEs } }		OPTIONAL,

...

}

NR-U-Channel-Info-List-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

NR-U-Channel-List ::= SEQUENCE (SIZE (1..maxnoofNR-UChannelIDs)) OF NR-U-Channel-Item

NR-U-Channel-Item ::= SEQUENCE {

nR-U-ChannelID						INTEGER(1..maxnoofNR-UChannelIDs),

channelOccupancyTimePercentageDL		ChannelOccupancyTimePercentage,

energyDetectionThreshold				EnergyDetectionThreshold,

iE-Extensions			ProtocolExtensionContainer { { NR-U-Channel-Item-ExtIEs} } OPTIONAL,

...

}

NR-U-Channel-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NumberofActiveUEs ::= INTEGER(0..16777215, ...)

NumberOfBroadcasts ::= INTEGER (0..65535)

NumberofBroadcastRequest ::= INTEGER (0..65535)

NumberOfTRPRxTEG ::= ENUMERATED {two, three, four, six, eight, ...}

NumberOfTRPRxTxTEG ::= ENUMERATED {wo, three, four, six, eight, ...}

NumDLULSymbols ::= SEQUENCE {

numDLSymbols	INTEGER (0..13, ...),

numULSymbols	INTEGER (0..13, ...),

iE-Extensions			ProtocolExtensionContainer { { NumDLULSymbols-ExtIEs} } OPTIONAL

}

NumDLULSymbols-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-permutation		CRITICALITY ignore	EXTENSION Permutation	    PRESENCE optional },

...

}

NRV2XServicesAuthorized ::= SEQUENCE {

vehicleUE			VehicleUE														OPTIONAL,

pedestrianUE 		PedestrianUE													OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NRV2XServicesAuthorized-ExtIEs} }	OPTIONAL

}

NRV2XServicesAuthorized-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NRUESidelinkAggregateMaximumBitrate ::= SEQUENCE {

uENRSidelinkAggregateMaximumBitrate		BitRate,

iE-Extensions					ProtocolExtensionContainer { {NRUESidelinkAggregateMaximumBitrate-ExtIEs} } OPTIONAL

}

NRUESidelinkAggregateMaximumBitrate-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NZP-CSI-RS-ResourceID::= INTEGER  (0..191)

#### -- O

OffsetToPointA	::= INTEGER (0..2199,...)

OnDemandPRS-Info ::= SEQUENCE {

onDemandPRSRequestAllowed				BIT STRING (SIZE (16)),

allowedResourceSetPeriodicityValues		BIT STRING (SIZE (24))	OPTIONAL,

allowedPRSBandwidthValues				BIT STRING (SIZE (64))	OPTIONAL,

allowedResourceRepetitionFactorValues	BIT STRING (SIZE (8))	OPTIONAL,

allowedResourceNumberOfSymbolsValues	BIT STRING (SIZE (8))	OPTIONAL,

allowedCombSizeValues					BIT STRING (SIZE (8))	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { OnDemandPRS-Info-ExtIEs} } OPTIONAL,

...

}

OnDemandPRS-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- P

PacketDelayBudget ::= INTEGER (0..1023, ...)

PacketErrorRate ::= SEQUENCE {

pER-Scalar			PER-Scalar,

pER-Exponent		PER-Exponent,

iE-Extensions		ProtocolExtensionContainer { {PacketErrorRate-ExtIEs} }	OPTIONAL,

...

}

PacketErrorRate-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PagingCell-Item ::= SEQUENCE {

nRCGI		NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { PagingCell-ItemExtIEs } }	OPTIONAL

}

PagingCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-LastUsedCellIndication					CRITICALITY ignore 	EXTENSION LastUsedCellIndication						PRESENCE optional }|

{	ID id-PEISubgroupingSupportIndication			CRITICALITY ignore	EXTENSION PEISubgroupingSupportIndication			PRESENCE optional },

...

}

PagingDRX ::= ENUMERATED {

v32,

v64,

v128,

v256,

...

}

PagingIdentity ::=	CHOICE {

rANUEPagingIdentity	RANUEPagingIdentity,

cNUEPagingIdentity	CNUEPagingIdentity,

choice-extension			ProtocolIE-SingleContainer { { PagingIdentity-ExtIEs } }

}

PagingCause ::= ENUMERATED { voice,	...}

PagingIdentity-ExtIEs F1AP-PROTOCOL-IES::= {

...

}

PagingOrigin ::= ENUMERATED { non-3gpp,	...}

PagingPriority ::= ENUMERATED { priolevel1, priolevel2, priolevel3, priolevel4, priolevel5, priolevel6, priolevel7, priolevel8,...}

PEIPSAssistanceInfo ::= SEQUENCE {

cNSubgroupID		CNSubgroupID,

iE-Extensions	ProtocolExtensionContainer { { PEIPSAssistanceInfo-ExtIEs } }	OPTIONAL

}

PEIPSAssistanceInfo-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RelativePathDelay ::= CHOICE {

k0					INTEGER (0..16351),

k1					INTEGER (0..8176),

k2					INTEGER (0..4088),

k3					INTEGER (0..2044),

k4					INTEGER (0..1022),

k5					INTEGER (0..511),

choice-extension			ProtocolIE-SingleContainer { { RelativePathDelay-ExtIEs } }

}

RelativePathDelay-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

Parent-IAB-Nodes-NA-Resource-Configuration-List ::= SEQUENCE (SIZE(1..maxnoofHSNASlots)) OF Parent-IAB-Nodes-NA-Resource-Configuration-Item

Parent-IAB-Nodes-NA-Resource-Configuration-Item::= SEQUENCE {

nADownlink					NADownlink 	    OPTIONAL,

nAUplink					NAUplink 	    OPTIONAL,

nAFlexible					NAFlexible 	    OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { Parent-IAB-Nodes-NA-Resource-Configuration-Item-ExtIEs} } OPTIONAL

}

Parent-IAB-Nodes-NA-Resource-Configuration-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PartialSuccessCell ::= SEQUENCE {

broadcastCellList		BroadcastCellList,

iE-Extensions				ProtocolExtensionContainer { { PartialSuccessCell-ExtIEs} } OPTIONAL,

...

}

PartialSuccessCell-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PathlossReferenceInfo ::= SEQUENCE {

pathlossReferenceSignal			PathlossReferenceSignal,

iE-Extensions					ProtocolExtensionContainer { {PathlossReferenceInfo-ExtIEs} }	OPTIONAL

}

PathlossReferenceInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PathlossReferenceSignal ::= CHOICE {

sSB										SSB,

dL-PRS									DL-PRS,

choice-extension						ProtocolIE-SingleContainer {{PathlossReferenceSignal-ExtIEs }}

}

PathlossReferenceSignal-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PathSwitchConfiguration ::= SEQUENCE {

targetRelayUEID			BIT STRING(SIZE(24)),

remoteUELocalID			RemoteUELocalID,

t420					ENUMERATED {ms50, ms100, ms150, ms200, ms500, ms1000, ms2000, ms10000},

iE-Extensions			ProtocolExtensionContainer { { PathSwitchConfiguration-ExtIEs } }		OPTIONAL,

...

}

PathSwitchConfiguration-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5QoSFlowIdentifier ::= INTEGER (1..2048)

PC5-QoS-Characteristics ::= CHOICE {

non-Dynamic-PQI				NonDynamicPQIDescriptor,

dynamic-PQI					DynamicPQIDescriptor,

choice-extension			ProtocolIE-SingleContainer { { PC5-QoS-Characteristics-ExtIEs } }

}

PC5-QoS-Characteristics-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PC5QoSParameters	::= SEQUENCE {

pC5-QoS-Characteristics				PC5-QoS-Characteristics,

pC5-QoS-Flow-Bit-Rates				PC5FlowBitRates				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { PC5QoSParameters-ExtIEs } }	OPTIONAL,

...

}

PC5QoSParameters-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5FlowBitRates ::= SEQUENCE {

guaranteedFlowBitRate		BitRate,

maximumFlowBitRate			BitRate,

iE-Extensions				ProtocolExtensionContainer { { PC5FlowBitRates-ExtIEs } }	OPTIONAL,

...

}

PC5FlowBitRates-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelID ::= INTEGER (1..512, ...)

PC5RLCChannelQoSInformation ::= CHOICE {

pC5RLCChannelQoS				QoSFlowLevelQoSParameters,

pC5ControlPlaneTrafficType		ENUMERATED {srb1,srb2,...},

choice-extension				ProtocolIE-SingleContainer { { PC5RLCChannelQoSInformation-ExtIEs} }

}

PC5RLCChannelQoSInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PC5RLCChannelToBeSetupList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelToBeSetupItem

PC5RLCChannelToBeSetupItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

pC5RLCChannelQoSInformation		PC5RLCChannelQoSInformation,

rLCMode							RLCMode,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelToBeSetupItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelToBeSetupItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelToBeModifiedList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelToBeModifiedItem

PC5RLCChannelToBeModifiedItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

pC5RLCChannelQoSInformation		PC5RLCChannelQoSInformation			OPTIONAL,

rLCMode							RLCMode			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelToBeModifiedItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelToBeModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelToBeReleasedList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelToBeReleasedItem

PC5RLCChannelToBeReleasedItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelToBeReleasedItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelToBeReleasedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelSetupList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelSetupItem

PC5RLCChannelSetupItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelSetupItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelSetupItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelFailedToBeSetupList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelFailedToBeSetupItem

PC5RLCChannelFailedToBeSetupItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

cause							Cause					OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelFailedToBeSetupItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelFailedToBeSetupItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelModifiedList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelModifiedItem

PC5RLCChannelModifiedItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelModifiedItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelFailedToBeModifiedList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelFailedToBeModifiedItem

PC5RLCChannelFailedToBeModifiedItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

cause							Cause	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelFailedToBeModifiedItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelFailedToBeModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelRequiredToBeModifiedList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelRequiredToBeModifiedItem

PC5RLCChannelRequiredToBeModifiedItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelRequiredToBeModifiedItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelRequiredToBeModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PC5RLCChannelRequiredToBeReleasedList ::= SEQUENCE (SIZE(1.. maxnoofPC5RLCChannels)) OF PC5RLCChannelRequiredToBeReleasedItem

PC5RLCChannelRequiredToBeReleasedItem ::= SEQUENCE {

pC5RLCChannelID					PC5RLCChannelID,

remoteUELocalID					RemoteUELocalID			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PC5RLCChannelRequiredToBeReleasedItem-ExtIEs } }	OPTIONAL,

...

}

PC5RLCChannelRequiredToBeReleasedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCCH-BlindDetectionSCG ::= OCTET STRING

PDCMeasurementPeriodicity ::= ENUMERATED

{ms80, ms120, ms160, ms240, ms320, ms480, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ...}

PDCMeasurementQuantities ::= SEQUENCE (SIZE (1.. maxnoofMeasPDC)) OF ProtocolIE-SingleContainer { {PDCMeasurementQuantities-ItemIEs} }

PDCMeasurementQuantities-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-PDCMeasurementQuantities-Item	CRITICALITY reject	TYPE PDCMeasurementQuantities-Item		PRESENCE mandatory}

}

PDCMeasurementQuantities-Item ::= SEQUENCE {

pDCmeasurementQuantitiesValue				PDCMeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { PDCMeasurementQuantitiesValue-ExtIEs} } OPTIONAL

}

PDCMeasurementQuantitiesValue-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCMeasurementQuantitiesValue ::= ENUMERATED {

nr-pdc-tadv,

gNB-rx-tx,

...

}

PDCMeasurementResult ::= SEQUENCE {

pDCMeasuredResultsList			PDCMeasuredResultsList,

iE-Extensions					ProtocolExtensionContainer { { PDCMeasurementResult-ExtIEs} } OPTIONAL

}

PDCMeasurementResult-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCMeasuredResultsList ::= SEQUENCE (SIZE(1..maxnoofMeasPDC)) OF PDCMeasuredResults-Item

PDCMeasuredResults-Item ::= SEQUENCE {

pDCMeasuredResults-Value	PDCMeasuredResults-Value,

iE-Extensions				ProtocolExtensionContainer {{ PDCMeasuredResults-Item-ExtIEs }}	 OPTIONAL

}

PDCMeasuredResults-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PDCMeasuredResults-Value ::= CHOICE {

pDC-TADV-NR				PDC-TADV-NR,

pDC-RxTxTimeDiff		PDC-RxTxTimeDiff,

choice-extension		ProtocolIE-SingleContainer { { PDCMeasuredResults-Value-ExtIEs} }

}

PDCMeasuredResults-Value-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PDCReportType ::= ENUMERATED {

onDemand,

periodic,

...

}

PDC-RxTxTimeDiff ::= INTEGER (0..61565, ...)

PDC-TADV-NR ::= INTEGER (0..62500, ...)

PDCP-SN ::= INTEGER (0..4095)

PDCPSNLength	::= ENUMERATED { twelve-bits,eighteen-bits,...}

PDUSessionID ::= INTEGER (0..255)

PEISubgroupingSupportIndication ::= ENUMERATED {true, ...}

ReportingPeriodicityValue ::= INTEGER (0..512, ...)

Periodicity ::= INTEGER (0..640000, ...)

PeriodicitySRS ::= ENUMERATED { ms0p125, ms0p25, ms0p5, ms0p625, ms1, ms1p25, ms2, ms2p5, ms4, ms5, ms8, ms10, ms16, ms20, ms32, ms40, ms64, ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ms10240, ...}

PeriodicityList ::= SEQUENCE (SIZE(1.. maxnoSRS-ResourcePerSet)) OF PeriodicityList-Item

PeriodicityList-Item ::= SEQUENCE {

periodicitySRS				PeriodicitySRS,

iE-Extensions				ProtocolExtensionContainer { { PeriodicityList-ItemExtIEs} } OPTIONAL

}

PeriodicityList-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Permutation ::= ENUMERATED {dfu, ufd, ...}

Ph-InfoMCG  ::= OCTET STRING

Ph-InfoSCG  ::= OCTET STRING

PLMN-Identity ::= OCTET STRING (SIZE(3))

PlayoutDelayForMediaStartup ::= OCTET STRING

PortNumber ::= BIT STRING (SIZE (16))

PosAssistance-Information ::= OCTET STRING

PosAssistanceInformationFailureList ::= OCTET STRING

PosBroadcast ::= ENUMERATED {

start,

stop,

...

}

PosConextRevIndication ::= ENUMERATED {true, ...}

PositioningBroadcastCells ::= SEQUENCE (SIZE (1..maxnoBcastCell)) OF NRCGI

PosMeasGapPreConfigList ::= SEQUENCE {

posMeasGapPreConfigToAddModList				OCTET STRING					OPTIONAL,

posMeasGapPreConfigToReleaseList			OCTET STRING					OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { PosMeasGapPreConfigList-ExtIEs} } OPTIONAL

}

PosMeasGapPreConfigList-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

MeasurementPeriodicity ::= ENUMERATED

{ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, min1, min6, min12, min30, ..., ms20480, ms40960, extended }

MeasurementPeriodicityExtended ::= ENUMERATED {ms160, ms320, ms1280, ms2560, ms61440, ms81920,	ms368640, ms737280, ms1843200,	...}

PosMeasurementPeriodicityNR-AoA ::= ENUMERATED {

ms160,

ms320,

ms640,

ms1280,

ms2560,

ms5120,

ms10240,

ms20480,

ms40960,

ms61440,

ms81920,

ms368640,

ms737280,

ms1843200,

...

}

PosMeasurementQuantities ::= SEQUENCE (SIZE(1.. maxnoofPosMeas)) OF PosMeasurementQuantities-Item

PosMeasurementQuantities-Item ::= SEQUENCE {

posMeasurementType					PosMeasurementType,

timingReportingGranularityFactor	INTEGER (0..5) OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { PosMeasurementQuantities-ItemExtIEs} } OPTIONAL

}

PosMeasurementQuantities-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PosMeasurementResult ::= SEQUENCE (SIZE (1.. maxnoofPosMeas)) OF PosMeasurementResultItem

PosMeasurementResultItem ::= SEQUENCE {

measuredResultsValue				MeasuredResultsValue,

timeStamp							TimeStamp,

measurementQuality					TRPMeasurementQuality	OPTIONAL,

measurementBeamInfo					MeasurementBeamInfo		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { PosMeasurementResultItemExtIEs } }	OPTIONAL

}

PosMeasurementResultItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ARP-ID				CRITICALITY ignore EXTENSION ARP-ID 					PRESENCE optional}|

{ ID id-SRSResourcetype		CRITICALITY ignore EXTENSION SRSResourcetype 			PRESENCE optional}|

{ ID id-LoS-NLoSInformation	CRITICALITY ignore EXTENSION LoS-NLoSInformation		PRESENCE optional },

...

}

PosMeasurementResultList ::= SEQUENCE (SIZE(1.. maxNoOfMeasTRPs)) OF PosMeasurementResultList-Item

PosMeasurementResultList-Item ::= SEQUENCE {

posMeasurementResult			PosMeasurementResult,

tRPID							TRPID,

iE-Extensions					ProtocolExtensionContainer { { PosMeasurementResultList-ItemExtIEs} } OPTIONAL

}

PosMeasurementResultList-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRCGI	CRITICALITY ignore EXTENSION NRCGI		PRESENCE optional },

...

}

PosMeasurementType ::= ENUMERATED {

gnb-rx-tx,

ul-srs-rsrp,

ul-aoa,

ul-rtoa,

... ,

multiple-ul-aoa,

ul-srs-rsrpp

}

PosReportCharacteristics ::= ENUMERATED {

ondemand,

periodic,

...

}

PosResourceSetType  ::= CHOICE {

periodic			PosResourceSetTypePR,

semi-persistent		PosResourceSetTypeSP,

aperiodic			PosResourceSetTypeAP,

choice-extension	ProtocolIE-SingleContainer {{ PosResourceSetType-ExtIEs }}

}

PosResourceSetType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PosResourceSetTypePR ::= SEQUENCE {

posperiodicSet		ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { PosResourceSetTypePR-ExtIEs} }	OPTIONAL

}

PosResourceSetTypePR-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PosResourceSetTypeSP ::= SEQUENCE {

possemi-persistentSet		ENUMERATED{true, ...},

iE-Extensions				ProtocolExtensionContainer { { PosResourceSetTypeSP-ExtIEs} }	OPTIONAL

}

PosResourceSetTypeSP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PosResourceSetTypeAP ::= SEQUENCE {

sRSResourceTrigger-List		INTEGER(1..3),

iE-Extensions				ProtocolExtensionContainer { { PosResourceSetTypeAP-ExtIEs} }	OPTIONAL

}

PosResourceSetTypeAP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PosSItypeList ::= SEQUENCE (SIZE(1.. maxnoofPosSITypes)) OF PosSItype-Item

PosSItype-Item ::= SEQUENCE {

posItype			PosSItype   ,

iE-Extensions	ProtocolExtensionContainer { { PosSItype-ItemExtIEs } } OPTIONAL

}

PosSItype-ItemExtIEs    F1AP-PROTOCOL-EXTENSION ::= {

...

}

PosSItype ::= INTEGER (1..32, ...)

PosSRSResourceID-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResourcePerSet)) OF SRSPosResourceID

PosSRSResource-Item ::= SEQUENCE {

srs-PosResourceId				SRSPosResourceID,

transmissionCombPos				TransmissionCombPos,

startPosition					INTEGER (0..13),

nrofSymbols						ENUMERATED {n1, n2, n4, n8, n12},

freqDomainShift					INTEGER (0..268),

c-SRS							INTEGER (0..63),

groupOrSequenceHopping			ENUMERATED { neither, groupHopping, sequenceHopping },

resourceTypePos					ResourceTypePos,

sequenceId						INTEGER (0.. 65535),

spatialRelationPos				SpatialRelationPos 	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PosSRSResource-Item-ExtIEs} }	OPTIONAL

}

PosSRSResource-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PosSRSResource-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResources)) OF PosSRSResource-Item

PosSRSResourceSet-Item ::= SEQUENCE {

possrsResourceSetID				INTEGER(0..15),

possRSResourceID-List			PosSRSResourceID-List,

posresourceSetType				PosResourceSetType,

iE-Extensions		ProtocolExtensionContainer { { PosSRSResourceSet-Item-ExtIEs} }	OPTIONAL

}

PosSRSResourceSet-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PosSRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-PosResourceSets)) OF PosSRSResourceSet-Item

PrimaryPathIndication ::= ENUMERATED {

true,

false,

...

}

Pre-emptionCapability ::= ENUMERATED {

shall-not-trigger-pre-emption,

may-trigger-pre-emption

}

Pre-emptionVulnerability ::= ENUMERATED {

not-pre-emptable,

pre-emptable

}

Preconfigured-measurement-GAP-Request ::= ENUMERATED {true, ...}

PriorityLevel	::= INTEGER { spare (0), highest (1), lowest (14), no-priority (15) } (0..15)

ProtectedEUTRAResourceIndication		::= OCTET STRING

Protected-EUTRA-Resources-Item ::= SEQUENCE {

spectrumSharingGroupID					SpectrumSharingGroupID,

eUTRACells-List		EUTRACells-List,

iE-Extensions		ProtocolExtensionContainer { { Protected-EUTRA-Resources-ItemExtIEs } }	OPTIONAL

}

Protected-EUTRA-Resources-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSConfiguration ::= SEQUENCE {

pRSResourceSet-List			PRSResourceSet-List,

iE-Extensions				ProtocolExtensionContainer { { PRSConfiguration-ExtIEs } }	OPTIONAL

}

PRSConfiguration-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSInformationPos  ::= SEQUENCE {

pRS-IDPos					INTEGER(0..255),

pRS-Resource-Set-IDPos		INTEGER(0..7),

pRS-Resource-IDPos			INTEGER(0..63)	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { PRSInformationPos-ExtIEs} } OPTIONAL

}

PRSInformationPos-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRS-Measurement-Info-List ::= SEQUENCE (SIZE(1..maxFreqLayers)) OF PRS-Measurement-Info-List-Item

PRS-Measurement-Info-List-Item ::= SEQUENCE {

pointA						INTEGER (0..3279165),

measPRSPeriodicity			ENUMERATED {ms20, ms40, ms80, ms160, ...},

measPRSOffset				INTEGER (0..159, ...),

measurementPRSLength		ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6, ms10, ms20},

iE-Extensions				ProtocolExtensionContainer { { PRS-Measurement-Info-List-Item-ExtIEs} } OPTIONAL,

...

}

PRS-Measurement-Info-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Potential-SpCell-Item ::= SEQUENCE {

potential-SpCell-ID			NRCGI	,

iE-Extensions				ProtocolExtensionContainer { { Potential-SpCell-ItemExtIEs } }	OPTIONAL,

...

}

Potential-SpCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSAngleList ::= SEQUENCE (SIZE(1.. maxnoofPRS-ResourcesPerSet)) OF PRSAngleItem

PRSAngleItem ::= SEQUENCE {

nR-PRS-Azimuth			INTEGER (0..359),

nR-PRS-Azimuth-fine		INTEGER (0..9) OPTIONAL,

nR-PRS-Elevation		INTEGER (0..180) OPTIONAL,

nR-PRS-Elevation-fine	INTEGER (0..9) OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { PRSAngleItem-ItemExtIEs } }	OPTIONAL

}

PRSAngleItem-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-PRS-Resource-ID		CRITICALITY ignore EXTENSION PRS-Resource-ID		PRESENCE optional },

...

}

PRSConfigRequestType ::= ENUMERATED {configure, off, ...}

PRSMuting::= SEQUENCE {

pRSMutingOption1			PRSMutingOption1		OPTIONAL,

pRSMutingOption2			PRSMutingOption2		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { PRSMuting-ExtIEs} } OPTIONAL

}

PRSMuting-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSMutingOption1 ::= SEQUENCE {

mutingPattern					DL-PRSMutingPattern,

mutingBitRepetitionFactor		ENUMERATED{rf1,rf2,rf4,rf8,...},

iE-Extensions					ProtocolExtensionContainer { { PRSMutingOption1-ExtIEs} } OPTIONAL

}

PRSMutingOption1-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSMutingOption2 ::= SEQUENCE {

mutingPattern					DL-PRSMutingPattern,

iE-Extensions					ProtocolExtensionContainer { { PRSMutingOption2-ExtIEs} } OPTIONAL

}

PRSMutingOption2-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRS-Resource-ID ::= INTEGER (0..63)

PRSResource-List::= SEQUENCE (SIZE (1..maxnoofPRSresources)) OF PRSResource-Item

PRSResource-Item  ::= SEQUENCE {

pRSResourceID			PRS-Resource-ID,

sequenceID				INTEGER(0..4095),

rEOffset				INTEGER(0..11,...),

resourceSlotOffset		INTEGER(0..511),

resourceSymbolOffset	INTEGER(0..12),

qCLInfo					PRSResource-QCLInfo		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { PRSResource-Item-ExtIEs} } OPTIONAL

}

PRSResource-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSResource-QCLInfo  ::= CHOICE {

qCLSourceSSB		PRSResource-QCLSourceSSB,

qCLSourcePRS		PRSResource-QCLSourcePRS,

choice-extension	ProtocolIE-SingleContainer { { PRSResource-QCLInfo-ExtIEs } }

}

PRSResource-QCLInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PRSResource-QCLSourceSSB ::= SEQUENCE {

pCI-NR				INTEGER(0..1007),

sSB-Index 			SSB-Index OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { PRSResource-QCLSourceSSB-ExtIEs} } OPTIONAL,

...

}

PRSResource-QCLSourceSSB-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSResource-QCLSourcePRS ::= SEQUENCE {

qCLSourcePRSResourceSetID		PRS-Resource-Set-ID,

qCLSourcePRSResourceID 			PRS-Resource-ID OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PRSResource-QCLSourcePRS-ExtIEs} } OPTIONAL

}

PRSResource-QCLSourcePRS-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRS-Resource-Set-ID ::= INTEGER(0..7)

PRSResourceSet-List ::= SEQUENCE (SIZE (1.. maxnoofPRSresourceSets)) OF PRSResourceSet-Item

PRSResourceSet-Item ::= SEQUENCE {

pRSResourceSetID				PRS-Resource-Set-ID,

subcarrierSpacing				ENUMERATED{kHz15, kHz30, kHz60, kHz120, ...},

pRSbandwidth					INTEGER(1..63),

startPRB						INTEGER(0..2176),

pointA							INTEGER (0..3279165),

combSize						ENUMERATED{n2, n4, n6, n12, ...},

cPType							ENUMERATED{normal, extended, ...},

resourceSetPeriodicity			ENUMERATED{n4,n5,n8,n10,n16,n20,n32,n40,n64,n80,n160,n320,n640,n1280,n2560,n5120,n10240,n20480,n40960, n81920,..., n128, n256, n512},

resourceSetSlotOffset			INTEGER(0..81919,...),

resourceRepetitionFactor		ENUMERATED{rf1,rf2,rf4,rf6,rf8,rf16,rf32,...},

resourceTimeGap					ENUMERATED{tg1,tg2,tg4,tg8,tg16,tg32,...},

resourceNumberofSymbols			ENUMERATED{n2,n4,n6,n12,...},

pRSMuting						PRSMuting 		OPTIONAL,

pRSResourceTransmitPower		INTEGER(-60..50),

pRSResource-List				PRSResource-List,

iE-Extensions					ProtocolExtensionContainer { { PRSResourceSet-Item-ExtIEs} } OPTIONAL

}

PRSResourceSet-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionOffIndication ::= CHOICE {

pRSTransmissionOffPerTRP				NULL,

pRSTransmissionOffPerResourceSet		PRSTransmissionOffPerResourceSet,

pRSTransmissionOffPerResource			PRSTransmissionOffPerResource,

choice-extension		ProtocolIE-SingleContainer { { PRSTransmissionOffIndication-ExtIEs } }

}

PRSTransmissionOffIndication-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

PRSTransmissionOffPerResource ::= SEQUENCE (SIZE (1..maxnoofPRSresourceSets)) OF PRSTransmissionOffPerResource-Item

PRSTransmissionOffPerResource-Item  ::= SEQUENCE {

pRSResourceSetID								PRS-Resource-Set-ID,

pRSTransmissionOffIndicationPerResourceList		SEQUENCE (SIZE(1.. maxnoofPRSresources)) OF PRSTransmissionOffIndicationPerResource-Item,

iE-Extensions		ProtocolExtensionContainer { { PRSTransmissionOffPerResource-Item-ExtIEs } } OPTIONAL,

...

}

PRSTransmissionOffPerResource-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionOffIndicationPerResource-Item  ::= SEQUENCE {

pRSResourceID		PRS-Resource-ID,

iE-Extensions		ProtocolExtensionContainer { { PRSTransmissionOffIndicationPerResource-Item-ExtIEs } } OPTIONAL,

...

}

PRSTransmissionOffIndicationPerResource-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionOffInformation ::= SEQUENCE {

pRSTransmissionOffIndication	PRSTransmissionOffIndication,

iE-Extensions					ProtocolExtensionContainer { { PRSTransmissionOffInformation-ExtIEs } } OPTIONAL,

...

}

PRSTransmissionOffInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionOffPerResourceSet ::= SEQUENCE (SIZE (1..maxnoofPRSresourceSets)) OF PRSTransmissionOffPerResourceSet-Item

PRSTransmissionOffPerResourceSet-Item  ::= SEQUENCE {

pRSResourceSetID		PRS-Resource-Set-ID,

iE-Extensions			ProtocolExtensionContainer { { PRSTransmissionOffPerResourceSet-Item-ExtIEs } } OPTIONAL,

...

}

PRSTransmissionOffPerResourceSet-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PWS-Failed-NR-CGI-Item ::= SEQUENCE {

nRCGI				NRCGI,

numberOfBroadcasts	NumberOfBroadcasts,

iE-Extensions		ProtocolExtensionContainer { { PWS-Failed-NR-CGI-ItemExtIEs } }	OPTIONAL,

...

}

PWS-Failed-NR-CGI-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

PWSSystemInformation ::= SEQUENCE {

sIBtype 				SIBType-PWS,

sIBmessage				OCTET STRING,

iE-Extensions			ProtocolExtensionContainer { { PWSSystemInformationExtIEs } }	OPTIONAL,

...

}

PWSSystemInformationExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-NotificationInformation		CRITICALITY ignore	EXTENSION NotificationInformation		PRESENCE optional}|

{ ID id-AdditionalSIBMessageList	CRITICALITY reject	EXTENSION AdditionalSIBMessageList		PRESENCE optional},

...

}

PrivacyIndicator ::= ENUMERATED {immediate-MDT,	logged-MDT,	...}

PRS-ID ::= INTEGER(0..255)

PRSTRPList ::= SEQUENCE (SIZE(1.. maxnoofTRPs)) OF PRSTRPItem

PRSTRPItem ::= SEQUENCE {

tRP-ID		TRPID,

requestedDLPRSTransmissionCharacteristics	RequestedDLPRSTransmissionCharacteristics 		OPTIONAL,

-- The IE shall be present if the PRS Configuration Request Type IE is set to “configure” --

pRSTransmissionOffInformation		PRSTransmissionOffInformation				OPTIONAL,

-- The IE shall be present if the PRS Configuration Request Type IE is set to “off” --

iE-Extensions	ProtocolExtensionContainer { { PRSTRPItem-ExtIEs} } OPTIONAL,

...

}

PRSTRPItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RequestedDLPRSTransmissionCharacteristics ::= SEQUENCE {

requestedDLPRSResourceSet-List		RequestedDLPRSResourceSet-List,

numberofFrequencyLayers			INTEGER(1..4)										OPTIONAL,

startTimeAndDuration			StartTimeAndDuration								OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { RequestedDLPRSTransmissionCharacteristics-ExtIEs} } OPTIONAL,

...

}

RequestedDLPRSTransmissionCharacteristics-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RequestedDLPRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoofPRSresourceSets)) OF RequestedDLPRSResourceSet-Item

RequestedDLPRSResourceSet-Item ::= SEQUENCE {

pRSbandwidth					INTEGER(1..63) OPTIONAL,

combSize						ENUMERATED{n2, n4, n6, n12, ...}		OPTIONAL,

resourceSetPeriodicity			ENUMERATED{n4,n5,n8,n10,n16,n20,n32,n40,n64,n80,n160,n320,n640,n1280,n2560,n5120,n10240,n20480,n40960, n81920,..., n128, n256, n512}	OPTIONAL,

resourceRepetitionFactor		ENUMERATED{rf1,rf2,rf4,rf6,rf8,rf16,rf32,...}		OPTIONAL,

resourceNumberofSymbols			ENUMERATED{n2,n4,n6,n12,...}						OPTIONAL,

requestedDLPRSResource-List		RequestedDLPRSResource-List							OPTIONAL,

resourceSetStartTimeAndDuration	StartTimeAndDuration								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { RequestedDLPRSResourceSet-Item-ExtIEs} } OPTIONAL,

...

}

RequestedDLPRSResourceSet-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RequestedDLPRSResource-List::= SEQUENCE (SIZE (1..maxnoofPRSresources)) OF RequestedDLPRSResource-Item

RequestedDLPRSResource-Item  ::= SEQUENCE {

qCLInfo					PRSResource-QCLInfo		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RequestedDLPRSResource-Item-ExtIEs} } OPTIONAL,

...

}

RequestedDLPRSResource-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

PRSTransmissionTRPList ::= SEQUENCE (SIZE(1.. maxnoofTRPs)) OF PRSTransmissionTRPItem

PRSTransmissionTRPItem ::= SEQUENCE {

tRP-ID				TRPID,

pRSConfiguration	PRSConfiguration,

iE-Extensions			ProtocolExtensionContainer { { PRSTransmissionTRPItem-ExtIEs} } OPTIONAL,

...

}

PRSTransmissionTRPItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- Q

QCI ::= INTEGER (0..255)

QoEInformation ::= SEQUENCE {

qoEInformationList		QoEInformationList,

iE-Extensions			ProtocolExtensionContainer { { QoEInformation-ExtIEs} } OPTIONAL

}

QoEInformation-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

QoEInformationList ::= SEQUENCE (SIZE(1.. maxnoofQoEInformation)) OF QoEInformationList-Item

QoEInformationList-Item ::= SEQUENCE {

qoEMetrics			QoEMetrics	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { QoEInformationList-Item-ExtIEs} } 	OPTIONAL

}

QoEInformationList-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

QoEMetrics ::= SEQUENCE {

appLayerBufferLevelList				AppLayerBufferLevelList  OPTIONAL,

playoutDelayForMediaStartup			PlayoutDelayForMediaStartup OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { QoEMetrics-ExtIEs} } OPTIONAL,

...

}

QoEMetrics-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Characteristics ::= CHOICE {

non-Dynamic-5QI				NonDynamic5QIDescriptor,

dynamic-5QI					Dynamic5QIDescriptor,

choice-extension			ProtocolIE-SingleContainer { { QoS-Characteristics-ExtIEs } }

}

QoS-Characteristics-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

QoSFlowIdentifier ::= INTEGER (0..63)

QoSFlowLevelQoSParameters	::= SEQUENCE {

qoS-Characteristics						QoS-Characteristics,

nGRANallocationRetentionPriority		NGRANAllocationAndRetentionPriority,

gBR-QoS-Flow-Information				GBR-QoSFlowInformation				OPTIONAL,

reflective-QoS-Attribute				ENUMERATED {subject-to, ...}				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoSFlowLevelQoSParameters-ExtIEs } }	OPTIONAL

}

QoSFlowLevelQoSParameters-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionID								CRITICALITY ignore	EXTENSION PDUSessionID				PRESENCE optional}|

{ ID id-ULPDUSessionAggregateMaximumBitRate			CRITICALITY ignore	EXTENSION BitRate						PRESENCE optional}|

{ ID id-QosMonitoringRequest						CRITICALITY ignore	EXTENSION QosMonitoringRequest		PRESENCE optional}|

{ ID id-PDCPTerminatingNodeDLTNLAddrInfo			CRITICALITY ignore	EXTENSION TransportLayerAddress	PRESENCE optional	},

...

}

QoSFlowMappingIndication ::= ENUMERATED {ul,dl,...}

QoSInformation	::=	CHOICE {

eUTRANQoS					EUTRANQoS,

choice-extension			ProtocolIE-SingleContainer { { QoSInformation-ExtIEs} }

}

QoSInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

{	ID id-DRB-Information		CRITICALITY ignore TYPE DRB-Information		PRESENCE mandatory},

...

}

QosMonitoringRequest ::= ENUMERATED {ul, dl, both, ..., stop}

QoSParaSetIndex ::= INTEGER (1..8, ...)

QoSParaSetNotifyIndex ::= INTEGER (0..8, ...)

#### -- R

RACH-Config-Common	::= OCTET STRING

RACH-Config-Common-IAB	::= OCTET STRING

RACHReportContainer::= OCTET STRING

RACHReportInformationList	::= SEQUENCE (SIZE(1.. maxnoofRACHReports)) OF RACHReportInformationItem

RACHReportInformationItem	::= SEQUENCE {

rACHReportContainer				RACHReportContainer,

uEAssitantIdentifier			GNB-DU-UE-F1AP-ID		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { RACHReportInformationItem-ExtIEs} }	OPTIONAL,

...

}

RACHReportInformationItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RadioResourceStatus ::= SEQUENCE {

sSBAreaRadioResourceStatusList		SSBAreaRadioResourceStatusList,

iE-Extensions	ProtocolExtensionContainer { { RadioResourceStatus-ExtIEs} } OPTIONAL

}

RadioResourceStatus-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SliceRadioResourceStatus		CRITICALITY ignore	EXTENSION SliceRadioResourceStatus		PRESENCE optional	}|

{ ID id-MIMOPRBusageInformation			CRITICALITY ignore	EXTENSION MIMOPRBusageInformation			PRESENCE optional	},

...

}

MIMOPRBusageInformation ::= SEQUENCE {

dl-GBR-PRB-usage-for-MIMO		INTEGER (0..100),

ul-GBR-PRB-usage-for-MIMO		INTEGER (0..100),

dl-non-GBR-PRB-usage-for-MIMO	INTEGER (0..100),

ul-non-GBR-PRB-usage-for-MIMO 	INTEGER (0..100),

dl-Total-PRB-usage-for-MIMO 	INTEGER (0..100),

ul-Total-PRB-usage-for-MIMO 	INTEGER (0..100),

iE-Extensions					ProtocolExtensionContainer { { MIMOPRBusageInformation-ExtIEs} }	OPTIONAL,

...

}

MIMOPRBusageInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RANAC ::= INTEGER (0..255)

RAN-MeasurementID ::= INTEGER (1.. 65536, ...)

RAN-UE-MeasurementID ::= INTEGER (1.. 256, ...)

RAN-UE-PDC-MeasID ::= INTEGER (1..16, ...)

RANUEID ::= OCTET STRING (SIZE (8))

RANUEPagingIdentity ::= SEQUENCE	{

iRNTI						BIT STRING (SIZE(40)),

iE-Extensions				ProtocolExtensionContainer { { RANUEPagingIdentity-ExtIEs } }	OPTIONAL}

RANUEPagingIdentity-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RAT-FrequencyPriorityInformation::= CHOICE {

eNDC		SubscriberProfileIDforRFP,

nGRAN		RAT-FrequencySelectionPriority,

choice-extension			ProtocolIE-SingleContainer { { RAT-FrequencyPriorityInformation-ExtIEs} }

}

RAT-FrequencyPriorityInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

RAT-FrequencySelectionPriority::= INTEGER (1.. 256, ...)

RBSetConfiguration ::= SEQUENCE {

subcarrierSpacing				SubcarrierSpacing,

rBSetSize						RBSetSize,

nUmberRBsets					INTEGER(1..maxnoofRBsetsPerCell),

iE-Extensions					ProtocolExtensionContainer { { RBSetConfiguration-ExtIEs} } OPTIONAL

}

RBSetConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RBSetSize ::=	ENUMERATED { rb2, rb4, rb8, rb16, rb32, rb64}

Re-routingEnableIndicator ::= ENUMERATED {

true,

false,

...

}

Redcap-Bcast-Information ::= BIT STRING(SIZE(8))

RedCapIndication ::= ENUMERATED {true, ...}

Reestablishment-Indication	::=	ENUMERATED  {

reestablished,

...

}

ReferencePoint ::= CHOICE {

coordinateID					CoordinateID,

referencePointCoordinate		AccessPointPosition,

referencePointCoordinateHA		NGRANHighAccuracyAccessPointPosition,

choice-Extension				ProtocolIE-SingleContainer { { ReferencePoint-ExtIEs} }

}

ReferencePoint-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ReferenceSFN ::= INTEGER (0..1023)

ReferenceSignal ::= CHOICE {

nZP-CSI-RS								NZP-CSI-RS-ResourceID,

sSB										SSB,

sRS										SRSResourceID,

positioningSRS							SRSPosResourceID,

dL-PRS									DL-PRS,

choice-extension						ProtocolIE-SingleContainer {{ReferenceSignal-ExtIEs }}

}

ReferenceSignal-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

RelativeCartesianLocation ::= SEQUENCE {

xYZunit						ENUMERATED {mm, cm, dm, ...},

xvalue						INTEGER (-65536..65535),

yvalue						INTEGER (-65536..65535),

zvalue						INTEGER (-32768..32767),

locationUncertainty			LocationUncertainty,

iE-Extensions				ProtocolExtensionContainer { { RelativeCartesianLocation-ExtIEs} } OPTIONAL

}

RelativeCartesianLocation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RelativeGeodeticLocation ::= SEQUENCE {

milli-Arc-SecondUnits		ENUMERATED {zerodot03, zerodot3, three, ...},

heightUnits					ENUMERATED {mm, cm, m, ...},

deltaLatitude				INTEGER (-1024.. 1023),

deltaLongitude				INTEGER (-1024.. 1023),

deltaHeight					INTEGER (-1024.. 1023),

locationUncertainty			LocationUncertainty,

iE-extensions				ProtocolExtensionContainer {{RelativeGeodeticLocation-ExtIEs }} OPTIONAL

}

RelativeGeodeticLocation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RemoteUELocalID ::= INTEGER (0..255, ...)

ReferenceTime ::= OCTET STRING

RegistrationRequest ::= ENUMERATED{start, stop, add, ...}

ReportCharacteristics ::= BIT STRING (SIZE(32))

ReportingPeriodicity ::= ENUMERATED{ms500, ms1000, ms2000, ms5000, ms10000, ...}

RequestedBandCombinationIndex ::= OCTET STRING

RequestedFeatureSetEntryIndex ::= OCTET STRING

RequestedP-MaxFR2 ::= OCTET STRING

Requested-PDCCH-BlindDetectionSCG ::= OCTET STRING

RequestedSRSTransmissionCharacteristics ::= SEQUENCE {

numberOfTransmissions		INTEGER (0..500, ...)		OPTIONAL,

-- The IE shall be present if the Resource Type IE is set to “periodic” --

resourceType				ENUMERATED  {periodic, semi-persistent, aperiodic,...},

bandwidthSRS				BandwidthSRS,

sRSResourceSetList 			SRSResourceSetList				OPTIONAL,

sSBInformation				SSBInformation				OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { RequestedSRSTransmissionCharacteristics-ExtIEs} } OPTIONAL

}

RequestedSRSTransmissionCharacteristics-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SrsFrequency		CRITICALITY ignore EXTENSION SrsFrequency		PRESENCE optional },

...

}

RequestType	::= ENUMERATED {offer, execution, ...}

ResourceCoordinationEUTRACellInfo ::= SEQUENCE {

eUTRA-Mode-Info 						EUTRA-Coex-Mode-Info,

eUTRA-PRACH-Configuration 				EUTRA-PRACH-Configuration,

iE-Extensions	ProtocolExtensionContainer { { ResourceCoordinationEUTRACellInfo-ExtIEs } }	OPTIONAL,

...

}

ResourceCoordinationEUTRACellInfo-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-IgnorePRACHConfiguration		CRITICALITY reject EXTENSION IgnorePRACHConfiguration		PRESENCE optional },

...

}

ResourceCoordinationTransferInformation ::= SEQUENCE {

meNB-Cell-ID								EUTRA-Cell-ID,

resourceCoordinationEUTRACellInfo		ResourceCoordinationEUTRACellInfo	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { ResourceCoordinationTransferInformation-ExtIEs } }	OPTIONAL,

...

}

ResourceCoordinationTransferInformation-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceCoordinationTransferContainer ::= OCTET STRING

ResourceSetType  ::= CHOICE {

periodic			ResourceSetTypePeriodic,

semi-persistent		ResourceSetTypeSemi-persistent,

aperiodic			ResourceSetTypeAperiodic,

choice-extension				ProtocolIE-SingleContainer {{ ResourceSetType-ExtIEs }}

}

ResourceSetType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ResourceSetTypePeriodic ::= SEQUENCE {

periodicSet			ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { ResourceSetTypePeriodic-ExtIEs} }	OPTIONAL

}

ResourceSetTypePeriodic-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceSetTypeSemi-persistent ::= SEQUENCE {

semi-persistentSet	ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { ResourceSetTypeSemi-persistent-ExtIEs} }	OPTIONAL

}

ResourceSetTypeSemi-persistent-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceSetTypeAperiodic ::= SEQUENCE {

sRSResourceTrigger-List 	INTEGER(1..3),

slotoffset					INTEGER(0..32),

iE-Extensions				ProtocolExtensionContainer { { ResourceSetTypeAperiodic-ExtIEs} }	OPTIONAL

}

ResourceSetTypeAperiodic-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RepetitionFactorExtended ::=  ENUMERATED {n3, n5, n6, n7, n8, n10, n12, n14, ...}

RepetitionPeriod ::= INTEGER (0..131071, ...)

ReportingRequestType ::= SEQUENCE {

eventType						EventType,

reportingPeriodicityValue						ReportingPeriodicityValue		OPTIONAL,

-- C-ifEventTypeisPeriodic: This IE shall be present if the Event Type IE is set to "periodic" in the Event Type IE.

iE-Extensions					ProtocolExtensionContainer { {ReportingRequestType-ExtIEs} }	OPTIONAL

}

ReportingRequestType-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceType ::= CHOICE {

periodic			ResourceTypePeriodic,

semi-persistent		ResourceTypeSemi-persistent,

aperiodic			ResourceTypeAperiodic,

choice-extension				ProtocolIE-SingleContainer {{ ResourceType-ExtIEs }}

}

ResourceType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ResourceTypePeriodic ::= SEQUENCE {

periodicity		   ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, ...},

offset				INTEGER(0..2559, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypePeriodic-ExtIEs} }	OPTIONAL

}

ResourceTypePeriodic-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeSemi-persistent ::= SEQUENCE {

periodicity		   ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, ...},

offset				INTEGER(0..2559, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeSemi-persistent-ExtIEs} }	OPTIONAL

}

ResourceTypeSemi-persistent-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeAperiodic ::= SEQUENCE {

aperiodicResourceType	   ENUMERATED{true, ...},

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeAperiodic-ExtIEs} }	OPTIONAL

}

ResourceTypeAperiodic-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypePos ::= CHOICE {

periodic			ResourceTypePeriodicPos,

semi-persistent		ResourceTypeSemi-persistentPos,

aperiodic			ResourceTypeAperiodicPos,

choice-extension	ProtocolIE-SingleContainer {{ ResourceTypePos-ExtIEs }}

}

ResourceTypePos-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

ResourceTypePeriodicPos ::= SEQUENCE {

periodicity			ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot40960, slot81920, ..., slot128, slot256, slot512, slot20480},

offset				INTEGER(0..81919, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypePeriodicPos-ExtIEs} }	OPTIONAL

}

ResourceTypePeriodicPos-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeSemi-persistentPos ::= SEQUENCE {

periodicity			ENUMERATED{slot1, slot2, slot4, slot5, slot8, slot10, slot16, slot20, slot32, slot40, slot64, slot80, slot160, slot320, slot640, slot1280, slot2560, slot5120, slot10240, slot40960, slot81920, ..., slot128, slot256, slot512, slot20480},

offset				INTEGER(0..81919, ...),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeSemi-persistentPos-ExtIEs} }	OPTIONAL

}

ResourceTypeSemi-persistentPos-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ResourceTypeAperiodicPos ::= SEQUENCE {

slotOffset			INTEGER (0..32),

iE-Extensions		ProtocolExtensionContainer { { ResourceTypeAperiodicPos-ExtIEs} }	OPTIONAL

}

ResourceTypeAperiodicPos-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RLCDuplicationInformation ::= SEQUENCE {

rLCDuplicationStateList 			RLCDuplicationStateList,

primaryPathIndication			PrimaryPathIndication	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {RLCDuplicationInformation-ExtIEs} }	OPTIONAL

}

RLCDuplicationInformation-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RLCDuplicationStateList	::= SEQUENCE (SIZE(1..maxnoofRLCDuplicationState)) OF RLCDuplicationState-Item

RLCDuplicationState-Item ::=SEQUENCE {

duplicationState		DuplicationState,

iE-Extensions	ProtocolExtensionContainer { {RLCDuplicationState-Item-ExtIEs } }	OPTIONAL,

...

}

RLCDuplicationState-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RLCFailureIndication ::= SEQUENCE {

assocatedLCID				LCID,

iE-Extensions				ProtocolExtensionContainer { {RLCFailureIndication-ExtIEs} } OPTIONAL

}

RLCFailureIndication-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RLCMode ::= ENUMERATED {

rlc-am,

rlc-um-bidirectional,

rlc-um-unidirectional-ul,

rlc-um-unidirectional-dl,

...

}

RLC-Status ::= SEQUENCE {

reestablishment-Indication 	Reestablishment-Indication,

iE-Extensions				ProtocolExtensionContainer { { RLC-Status-ExtIEs } } OPTIONAL,

...

}

RLC-Status-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RLFReportInformationList	::= SEQUENCE (SIZE(1.. maxnoofRLFReports)) OF RLFReportInformationItem

RLFReportInformationItem	::= SEQUENCE {

nRUERLFReportContainer		NRUERLFReportContainer,

uEAssitantIdentifier		GNB-DU-UE-F1AP-ID		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { RLFReportInformationItem-ExtIEs} }	OPTIONAL,

...

}

RLFReportInformationItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RIMRSDetectionStatus ::= ENUMERATED {rs-detected, rs-disappeared, ...}

RRCContainer ::= OCTET STRING

RRCContainer-RRCSetupComplete ::= OCTET STRING

RRCDeliveryStatus ::= SEQUENCE	{

delivery-status 			PDCP-SN,

triggering-message			PDCP-SN,

iE-Extensions				ProtocolExtensionContainer { { RRCDeliveryStatus-ExtIEs } }	OPTIONAL}

RRCDeliveryStatus-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RRCDeliveryStatusRequest ::= ENUMERATED {true, ...}

RRCReconfigurationCompleteIndicator	::= ENUMERATED {

true,

...,

failure

}

RRC-Version ::= SEQUENCE	{

latest-RRC-Version			BIT STRING (SIZE(3)),

iE-Extensions				ProtocolExtensionContainer { { RRC-Version-ExtIEs } }	OPTIONAL}

RRC-Version-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID id-latest-RRC-Version-Enhanced		CRITICALITY ignore EXTENSION OCTET STRING (SIZE(3))		PRESENCE optional },

...

}

RoutingID ::= OCTET STRING

ResponseTime ::= SEQUENCE {

time      			INTEGER (1..128,...),

timeUnit	ENUMERATED {second, ten-seconds, ten-milliseconds,...},

iE-Extensions		ProtocolExtensionContainer { { ResponseTime-ExtIEs} }	OPTIONAL,

...

}

ResponseTime-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

RxTxTimingErrorMargin ::= ENUMERATED {tc0dot5, tc1, tc2, tc4, tc8, tc12, tc16, tc20, tc24, tc32, tc40, tc48, tc64, tc80, tc96, tc128, ...}

#### -- S

SCell-FailedtoSetup-Item	::= SEQUENCE {

sCell-ID			NRCGI	,

cause		Cause			OPTIONAL ,

iE-Extensions	ProtocolExtensionContainer { { SCell-FailedtoSetup-ItemExtIEs } }	OPTIONAL,

...

}

SCell-FailedtoSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SCell-FailedtoSetupMod-Item	::= SEQUENCE {

sCell-ID			NRCGI	,

cause		Cause			OPTIONAL ,

iE-Extensions	ProtocolExtensionContainer { { SCell-FailedtoSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SCell-FailedtoSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SCell-ToBeRemoved-Item	::= SEQUENCE {

sCell-ID			NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { SCell-ToBeRemoved-ItemExtIEs } }	OPTIONAL,

...

}

SCell-ToBeRemoved-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SCell-ToBeSetup-Item ::= SEQUENCE {

sCell-ID			NRCGI	,

sCellIndex			SCellIndex,

sCellULConfigured		CellULConfigured 	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SCell-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

SCell-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ServingCellMO		CRITICALITY ignore	EXTENSION ServingCellMO		PRESENCE optional	},

...

}

SCell-ToBeSetupMod-Item	::= SEQUENCE {

sCell-ID			NRCGI	,

sCellIndex			SCellIndex,

sCellULConfigured		CellULConfigured 	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SCell-ToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SCell-ToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ServingCellMO		CRITICALITY ignore	EXTENSION ServingCellMO		PRESENCE optional	},

...

}

SCellIndex ::=INTEGER (1..31, ...)

SCGActivationRequest ::= ENUMERATED {activate-scg, deactivate-scg, ...}

SCGActivationStatus ::= ENUMERATED {scg-activated, scg-deactivated, ...}

SCGIndicator	::=	ENUMERATED{released, ...}

SCS-480	::= INTEGER(0..319)

SCS-960	::= INTEGER(0..639)

SCS-SpecificCarrier ::=					SEQUENCE {

offsetToCarrier						INTEGER (0..2199,...),

subcarrierSpacing					ENUMERATED {kHz15, kHz30, kHz60, kHz120,..., kHz480, kHz960},

carrierBandwidth					INTEGER (1..275,...),

iE-Extensions						ProtocolExtensionContainer { { SCS-SpecificCarrier-ExtIEs } } OPTIONAL

}

SCS-SpecificCarrier-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTBearerConfigurationQueryIndication ::=	ENUMERATED {true, ...}

SDTBearerConfigurationInfo ::= SEQUENCE {

sDTBearerConfig-List			SDTBearerConfig-List,

iE-Extensions					ProtocolExtensionContainer { { SDTBearerConfigurationInfo-ExtIEs } } OPTIONAL

}

SDTBearerConfigurationInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTBearerConfig-List ::= SEQUENCE (SIZE(1..maxnoofSDTBearers)) OF SDTBearerConfig-List-Item

SDTBearerConfig-List-Item ::= SEQUENCE{

sDTBearerType 							SDTBearerType,

sDTRLCBearerConfiguration				SDTRLCBearerConfiguration,

iE-Extensions			ProtocolExtensionContainer {{ SDTBearerConfig-List-Item-ExtIEs}}		OPTIONAL

}

SDTBearerConfig-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTBearerType ::= CHOICE {

sRB					SRBID,

dRB					DRBID,

choice-extension				ProtocolIE-SingleContainer {{ SDTBearerType-ExtIEs }}

}

SDTBearerType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SDT-MAC-PHY-CG-Config ::= OCTET STRING

SDTInformation ::= SEQUENCE {

sdtIndicator					ENUMERATED {true,...},

sdtAssistantInformation			ENUMERATED {singlepacket, multiplepackets,...}	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SDTInformation-ExtIEs } } OPTIONAL

}

SDTInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SDTRLCBearerConfiguration ::= OCTET STRING

SDT-Termination-Request	::= ENUMERATED {radio-link-problem, normal, ...}

Search-window-information ::= SEQUENCE {

expectedPropagationDelay		INTEGER (-3841..3841,...),

delayUncertainty				INTEGER (1..246,...),

iE-Extensions					ProtocolExtensionContainer { { Search-window-information-ExtIEs } } OPTIONAL

}

Search-window-information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SerialNumber ::= BIT STRING (SIZE (16))

SIBType-PWS ::=INTEGER (6..8, ...)

SelectedBandCombinationIndex ::= OCTET STRING

SelectedFeatureSetEntryIndex ::= OCTET STRING

CG-ConfigInfo ::= OCTET STRING

ServCellInfoList ::= OCTET STRING

ServCellIndex ::= INTEGER (0..31, ...)

ServingCellMO ::= INTEGER (1..64, ...)

ServingCellMO-List-Item ::= SEQUENCE {

servingCellMO					ServingCellMO,

sSB-Frequency					INTEGER (0..3279165),

iE-Extensions					ProtocolExtensionContainer { { ServingCellMO-List-Item-ExtIEs } } OPTIONAL

}

ServingCellMO-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ServingCellMO-encoded-in-CGC-List ::= SEQUENCE (SIZE(1.. maxNrofBWPs)) OF ServingCellMO-encoded-in-CGC-Item

ServingCellMO-encoded-in-CGC-Item ::= SEQUENCE {

servingCellMO					ServingCellMO,

iE-Extensions					ProtocolExtensionContainer { { ServingCellMO-encoded-in-CGC-Item-ExtIEs } }	OPTIONAL,

...

}

ServingCellMO-encoded-in-CGC-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-BWP-Id	CRITICALITY ignore	EXTENSION BWP-Id	PRESENCE optional },

...

}

Served-Cell-Information ::= SEQUENCE {

nRCGI							NRCGI,

nRPCI							NRPCI,

fiveGS-TAC						FiveGS-TAC			OPTIONAL,

configured-EPS-TAC				Configured-EPS-TAC 		OPTIONAL,

servedPLMNs						ServedPLMNs-List,

nR-Mode-Info					NR-Mode-Info,

measurementTimingConfiguration	OCTET STRING,

iE-Extensions		ProtocolExtensionContainer { {Served-Cell-Information-ExtIEs} } OPTIONAL,

...

}

Served-Cell-Information-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RANAC							CRITICALITY ignore	EXTENSION RANAC							PRESENCE optional }|

{	ID id-ExtendedServedPLMNs-List		CRITICALITY ignore	EXTENSION ExtendedServedPLMNs-List	PRESENCE optional }|

{	ID id-Cell-Direction				CRITICALITY ignore	EXTENSION Cell-Direction				PRESENCE optional }|

{	ID id-BPLMN-ID-Info-List			CRITICALITY ignore	EXTENSION BPLMN-ID-Info-List			PRESENCE optional }|

{	ID id-Cell-Type						CRITICALITY ignore	EXTENSION CellType						PRESENCE optional}|

{	ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication		PRESENCE optional }|

{	ID id-AggressorgNBSetID				CRITICALITY ignore	EXTENSION AggressorgNBSetID				PRESENCE optional}|

{	ID id-VictimgNBSetID				CRITICALITY ignore	EXTENSION VictimgNBSetID				PRESENCE optional}|

{	ID id-IAB-Info-IAB-DU				CRITICALITY ignore	EXTENSION IAB-Info-IAB-DU				PRESENCE optional}|

{	ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst			PRESENCE optional }|

{	ID id-NRPRACHConfig					CRITICALITY ignore	EXTENSION NRPRACHConfig					PRESENCE optional }|

{	ID id-SFN-Offset					CRITICALITY ignore	EXTENSION SFN-Offset					PRESENCE optional }|

{	ID id-NPNBroadcastInformation		CRITICALITY reject 	EXTENSION NPNBroadcastInformation		PRESENCE optional }|

{	ID id-Supported-MBS-FSA-ID-List				CRITICALITY ignore	EXTENSION Supported-MBS-FSA-ID-List						PRESENCE optional }|

{	ID id-Redcap-Bcast-Information		CRITICALITY ignore	EXTENSION Redcap-Bcast-Information	PRESENCE optional },

...

}

Serving-Cells-List ::= SEQUENCE (SIZE(1..maxnoofServingCells)) OF Serving-Cells-List-Item

Serving-Cells-List-Item ::= SEQUENCE{

nRCGI 								NRCGI,

iAB-MT-Cell-NA-Resource-Configuration-Mode-Info       IAB-MT-Cell-NA-Resource-Configuration-Mode-Info		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{Serving-Cells-List-Item-ExtIEs}}		OPTIONAL

}

Serving-Cells-List-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Supported-MBS-FSA-ID-List::= SEQUENCE (SIZE(1.. maxnoofMBSFSAs)) OF MBS-FrequencySelectionArea-Identity

MBS-FrequencySelectionArea-Identity::= OCTET STRING (SIZE(3))

SFN-Offset ::= SEQUENCE {

sFN-Time-Offset					BIT STRING (SIZE(24)),

iE-Extensions		ProtocolExtensionContainer { {SFN-Offset-ExtIEs} } OPTIONAL,

...

}

SFN-Offset-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-Cells-To-Add-Item ::= SEQUENCE {

served-Cell-Information		Served-Cell-Information,

gNB-DU-System-Information	GNB-DU-System-Information	 OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Served-Cells-To-Add-ItemExtIEs} }	OPTIONAL,

...

}

Served-Cells-To-Add-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-Cells-To-Delete-Item ::= SEQUENCE {

oldNRCGI					NRCGI	,

iE-Extensions				ProtocolExtensionContainer { { Served-Cells-To-Delete-ItemExtIEs } }	OPTIONAL,

...

}

Served-Cells-To-Delete-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-Cells-To-Modify-Item ::= SEQUENCE {

oldNRCGI					NRCGI							,

served-Cell-Information		Served-Cell-Information		,

gNB-DU-System-Information	GNB-DU-System-Information 	OPTIONAL	,

iE-Extensions				ProtocolExtensionContainer { { Served-Cells-To-Modify-ItemExtIEs } }	OPTIONAL,

...

}

Served-Cells-To-Modify-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Served-EUTRA-Cells-Information::= SEQUENCE {

eUTRA-Mode-Info						EUTRA-Mode-Info,

protectedEUTRAResourceIndication	ProtectedEUTRAResourceIndication,

iE-Extensions						ProtocolExtensionContainer { {Served-EUTRA-Cell-Information-ExtIEs} } OPTIONAL,

...

}

Served-EUTRA-Cell-Information-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Service-State ::= ENUMERATED {

in-service,

out-of-service,

...

}

Service-Status ::= SEQUENCE {

service-state				Service-State,

switchingOffOngoing			ENUMERATED {true, ...}	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Service-Status-ExtIEs } }	OPTIONAL,

...

}

Service-Status-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RelativeTime1900 ::= 	BIT STRING (SIZE (64))

ShortDRXCycleLength ::=  ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32, ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ...}

ShortDRXCycleTimer ::= INTEGER (1..16)

SIB1-message ::= OCTET STRING

SIB10-message ::= OCTET STRING

SIB12-message ::= OCTET STRING

SIB13-message ::= OCTET STRING

SIB14-message ::= OCTET STRING

SIB15-message ::= OCTET STRING

SIB17-message ::= OCTET STRING

SIB20-message ::= OCTET STRING

SItype ::= INTEGER (1..32, ...)

SItype-List ::= SEQUENCE (SIZE(1.. maxnoofSITypes)) OF SItype-Item

SItype-Item ::= SEQUENCE {

sItype		SItype	,

iE-Extensions	ProtocolExtensionContainer { { SItype-ItemExtIEs } }	OPTIONAL

}

SItype-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SibtypetobeupdatedListItem ::= SEQUENCE {

sIBtype 			INTEGER (2..32,...),

sIBmessage			OCTET STRING,

valueTag			INTEGER (0..31,...),

iE-Extensions		ProtocolExtensionContainer { { SibtypetobeupdatedListItem-ExtIEs } }	OPTIONAL,

...

}

SibtypetobeupdatedListItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-areaScope	CRITICALITY ignore	EXTENSION	AreaScope	PRESENCE optional},

...

}

SidelinkRelayConfiguration ::= SEQUENCE {

gNB-DU-UE-F1APIDofRelayUE			GNB-DU-UE-F1AP-ID,

remoteUELocalID						RemoteUELocalID,

sidelinkConfigurationContainer		SidelinkConfigurationContainer		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { SidelinkRelayConfiguration-ExtIEs } }		OPTIONAL,

...

}

SidelinkRelayConfiguration-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SidelinkConfigurationContainer ::= OCTET STRING

SLDRBID ::= INTEGER (1..512, ...)

SLDRBInformation ::= SEQUENCE {

sLDRB-QoS				PC5QoSParameters,

flowsMappedToSLDRB-List	FlowsMappedToSLDRB-List,

...

}

SLDRBs-FailedToBeModified-Item	::= SEQUENCE {

sLDRBID		SLDRBID		,

cause		Cause		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-FailedToBeModified-ItemExtIEs } }	OPTIONAL

}

SLDRBs-FailedToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-FailedToBeSetup-Item	::= SEQUENCE {

sLDRBID	SLDRBID,

cause	Cause	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-FailedToBeSetup-ItemExtIEs } }		OPTIONAL

}

SLDRBs-FailedToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-FailedToBeSetupMod-Item	::= SEQUENCE {

sLDRBID			SLDRBID	,

cause			Cause			OPTIONAL ,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-FailedToBeSetupMod-ItemExtIEs } }	OPTIONAL

}

SLDRBs-FailedToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-Modified-Item	::= SEQUENCE {

sLDRBID							SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-Modified-ItemExtIEs } }	OPTIONAL

}

SLDRBs-Modified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-ModifiedConf-Item	::= SEQUENCE {

sLDRBID							SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-ModifiedConf-ItemExtIEs } }	OPTIONAL

}

SLDRBs-ModifiedConf-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-Required-ToBeModified-Item	::= SEQUENCE {

sLDRBID							SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-Required-ToBeModified-ItemExtIEs } }	OPTIONAL

}

SLDRBs-Required-ToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-Required-ToBeReleased-Item	::= SEQUENCE {

sLDRBID		SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-Required-ToBeReleased-ItemExtIEs } }	OPTIONAL

}

SLDRBs-Required-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-Setup-Item ::= SEQUENCE {

sLDRBID							SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-Setup-ItemExtIEs } }	OPTIONAL

}

SLDRBs-Setup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-SetupMod-Item	::= SEQUENCE {

sLDRBID							SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-SetupMod-ItemExtIEs } }	OPTIONAL

}

SLDRBs-SetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-ToBeModified-Item	::= SEQUENCE {

sLDRBID						SLDRBID,

sLDRBInformation			SLDRBInformation		OPTIONAL,

rLCMode						RLCMode					OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-ToBeModified-ItemExtIEs } }	OPTIONAL

}

SLDRBs-ToBeModified-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-ToBeReleased-Item	::= SEQUENCE {

sLDRBID			SLDRBID,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL

}

SLDRBs-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-ToBeSetup-Item ::= SEQUENCE	{

sLDRBID						SLDRBID,

sLDRBInformation			SLDRBInformation,

rLCMode						RLCMode,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-ToBeSetup-ItemExtIEs } }	OPTIONAL

}

SLDRBs-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRBs-ToBeSetupMod-Item	::= SEQUENCE {

sLDRBID						SLDRBID,

sLDRBInformation			SLDRBInformation,

rLCMode						RLCMode			OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SLDRBs-ToBeSetupMod-ItemExtIEs } }	OPTIONAL

}

SLDRBs-ToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRXCycleList ::= SEQUENCE (SIZE(1.. maxnoofSLdestinations)) OF SLDRXCycleItem

SLDRXCycleItem ::= SEQUENCE {

rXUEID					BIT STRING (SIZE(24)),

sLDRXInformation		SLDRXInformation,

iE-Extensions			ProtocolExtensionContainer { { SLDRXCycleItem-ExtIEs } }	OPTIONAL,

...

}

SLDRXCycleItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SLDRXInformation    ::= CHOICE {

sLDRXCycle			SLDRXCycleLength,

nosLDRX				SLDRXConfigurationIndicator,

choice-extension	ProtocolIE-SingleContainer { { SLDRXInformation-ExtIEs} }

}

SLDRXCycleLength ::= ENUMERATED{ms10, ms20, ms32, ms40, ms60, ms64, ms70, ms80, ms128, ms160, ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240, ...}

SLDRXConfigurationIndicator ::= ENUMERATED{ release, ...}

SLDRXInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SL-PHY-MAC-RLC-Config ::= OCTET STRING

SL-PHY-MAC-RLC-ConfigExt ::= OCTET STRING

SL-RLC-ChannelToAddModList::= OCTET STRING

SL-ConfigDedicatedEUTRA-Info ::= OCTET STRING

SliceAvailableCapacity ::= SEQUENCE {

sliceAvailableCapacityList	SliceAvailableCapacityList,

iE-Extensions				ProtocolExtensionContainer { { SliceAvailableCapacity-ExtIEs} } OPTIONAL

}

SliceAvailableCapacity-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SliceAvailableCapacityList ::= SEQUENCE (SIZE(1.. maxnoofBPLMNsNR)) OF SliceAvailableCapacityItem

SliceAvailableCapacityItem ::= SEQUENCE {

pLMNIdentity					PLMN-Identity,

sNSSAIAvailableCapacity-List	SNSSAIAvailableCapacity-List,

iE-Extensions	ProtocolExtensionContainer { { SliceAvailableCapacityItem-ExtIEs} } OPTIONAL

}

SliceAvailableCapacityItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIAvailableCapacity-List ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAIAvailableCapacity-Item

SNSSAIAvailableCapacity-Item ::= SEQUENCE {

sNSSAI		SNSSAI,

sliceAvailableCapacityValueDownlink	INTEGER (0..100)	OPTIONAL,

sliceAvailableCapacityValueUplink	INTEGER (0..100)	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SNSSAIAvailableCapacity-Item-ExtIEs } }	OPTIONAL

}

SNSSAIAvailableCapacity-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SliceRadioResourceStatus ::= SEQUENCE {

sliceRadioResourceStatus	SliceRadioResourceStatus-List,

iE-Extensions				ProtocolExtensionContainer { { SliceRadioResourceStatus-ExtIEs} } OPTIONAL

}

SliceRadioResourceStatus-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SliceRadioResourceStatus-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNsNR)) OF SliceRadioResourceStatus-Item

SliceRadioResourceStatus-Item::= SEQUENCE {

pLMNIdentity					PLMN-Identity,

sNSSAIRadioResourceStatus-List	SNSSAIRadioResourceStatus-List,

iE-Extensions					ProtocolExtensionContainer { { SliceRadioResourceStatus-Item-ExtIEs} } OPTIONAL

}

SliceRadioResourceStatus-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAIRadioResourceStatus-List ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAIRadioResourceStatus-Item

SNSSAIRadioResourceStatus-Item ::= SEQUENCE {

sNSSAI		SNSSAI,

sNSSAIdlGBRPRBusage			INTEGER (0..100),

sNSSAIulGBRPRBusage			INTEGER (0..100),

sNSSAIdlNonGBRPRBusage		INTEGER (0..100),

sNSSAIulNonGBRPRBusage		INTEGER (0..100),

sNSSAIdlTotalPRBallocation	INTEGER (0..100),

sNSSAIulTotalPRBallocation	INTEGER (0..100),

iE-Extensions				ProtocolExtensionContainer { { SNSSAIRadioResourceStatus-Item-ExtIEs } }	OPTIONAL

}

SNSSAIRadioResourceStatus-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SliceSupportList ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SliceSupportItem

SliceSupportItem ::= SEQUENCE {

sNSSAI	SNSSAI,

iE-Extensions				ProtocolExtensionContainer { { SliceSupportItem-ExtIEs } }	OPTIONAL

}

SliceSupportItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SliceToReportList ::= SEQUENCE (SIZE(1.. maxnoofBPLMNsNR)) OF SliceToReportItem

SliceToReportItem ::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

sNSSAIlist					SNSSAI-list,

iE-Extensions				ProtocolExtensionContainer { { SliceToReportItem-ExtIEs} } OPTIONAL

}

SliceToReportItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SlotNumber ::= INTEGER (0..79)

SNSSAI-list ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SNSSAI-Item

SNSSAI-Item ::= SEQUENCE {

sNSSAI		SNSSAI,

iE-Extensions				ProtocolExtensionContainer { { SNSSAI-Item-ExtIEs } }	OPTIONAL

}

SNSSAI-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

Slot-Configuration-List ::= SEQUENCE (SIZE(1.. maxnoofslots)) OF Slot-Configuration-Item

Slot-Configuration-Item ::= SEQUENCE {

slotIndex				INTEGER (0..5119, ...),

symbolAllocInSlot		SymbolAllocInSlot,

iE-Extensions			ProtocolExtensionContainer { { Slot-Configuration-ItemExtIEs } }	OPTIONAL

}

Slot-Configuration-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SNSSAI ::= SEQUENCE {

sST			OCTET STRING (SIZE(1)),

sD			OCTET STRING (SIZE(3)) 	OPTIONAL	,

iE-Extensions				ProtocolExtensionContainer { { SNSSAI-ExtIEs } }	OPTIONAL

}

SNSSAI-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SpatialDirectionInformation ::= SEQUENCE {

nR-PRSBeamInformation			NR-PRSBeamInformation,

iE-Extensions					ProtocolExtensionContainer { { SpatialDirectionInformation-ExtIEs } } OPTIONAL

}

SpatialDirectionInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationInfo ::= SEQUENCE {

spatialRelationforResourceID					SpatialRelationforResourceID,

iE-Extensions		ProtocolExtensionContainer { {SpatialRelationInfo-ExtIEs} }	OPTIONAL

}

SpatialRelationInfo-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationforResourceID ::= SEQUENCE (SIZE(1..maxnoofSpatialRelations)) OF SpatialRelationforResourceIDItem

SpatialRelationforResourceIDItem ::= SEQUENCE {

referenceSignal		ReferenceSignal,

iE-Extensions		ProtocolExtensionContainer { {SpatialRelationforResourceIDItem-ExtIEs} }	OPTIONAL

}

SpatialRelationforResourceIDItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationPerSRSResource ::= SEQUENCE {

spatialRelationPerSRSResource-List	SpatialRelationPerSRSResource-List,

iE-Extensions		ProtocolExtensionContainer { { SpatialRelationPerSRSResource-ExtIEs} }	OPTIONAL,

...

}

SpatialRelationPerSRSResource-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationPerSRSResource-List::= SEQUENCE(SIZE (1.. maxnoSRS-ResourcePerSet)) OF SpatialRelationPerSRSResourceItem

SpatialRelationPerSRSResourceItem ::= SEQUENCE {

referenceSignal		ReferenceSignal,

iE-Extensions		ProtocolExtensionContainer { { SpatialRelationPerSRSResourceItem-ExtIEs} }	OPTIONAL,

...

}

SpatialRelationPerSRSResourceItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SpatialRelationPos ::= CHOICE {

sSBPos					SSB,

pRSInformationPos		PRSInformationPos,

choice-extension		ProtocolIE-SingleContainer {{ SpatialInformationPos-ExtIEs }}

}

SpatialInformationPos-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SpectrumSharingGroupID ::= INTEGER (1..maxCellineNB)

SRBID ::= INTEGER (0..3, ..., 4)

SRBs-FailedToBeSetup-Item	::= SEQUENCE {

sRBID		SRBID	,

cause		Cause	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SRBs-FailedToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-FailedToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-FailedToBeSetupMod-Item	::= SEQUENCE {

sRBID		SRBID		,

cause		Cause		OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SRBs-FailedToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-FailedToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-Modified-Item ::= SEQUENCE {

sRBID							SRBID,

lCID							LCID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-Modified-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-Modified-ItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-Required-ToBeReleased-Item	::= SEQUENCE {

sRBID	SRBID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-Required-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-Required-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-Setup-Item ::= SEQUENCE {

sRBID							SRBID,

lCID								LCID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-Setup-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-Setup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-SetupMod-Item ::= SEQUENCE {

sRBID						SRBID,

lCID							LCID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-SetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-SetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-ToBeReleased-Item	::= SEQUENCE {

sRBID		SRBID,

iE-Extensions	ProtocolExtensionContainer { { SRBs-ToBeReleased-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-ToBeReleased-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRBs-ToBeSetup-Item ::= SEQUENCE {

sRBID	 SRBID	,

duplicationIndication	DuplicationIndication	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SRBs-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalDuplicationIndication	CRITICALITY ignore	EXTENSION AdditionalDuplicationIndication		PRESENCE optional	}|

{ ID id-SDTRLCBearerConfiguration		CRITICALITY ignore	EXTENSION SDTRLCBearerConfiguration				PRESENCE optional }|

{ ID id-SRBMappingInfo					CRITICALITY ignore	EXTENSION UuRLCChannelID							PRESENCE optional	},

...

}

SRBs-ToBeSetupMod-Item	::= SEQUENCE {

sRBID	SRBID,

duplicationIndication	DuplicationIndication	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SRBs-ToBeSetupMod-ItemExtIEs } }	OPTIONAL,

...

}

SRBs-ToBeSetupMod-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-AdditionalDuplicationIndication	CRITICALITY ignore	EXTENSION AdditionalDuplicationIndication		PRESENCE optional	}|

{ ID id-SRBMappingInfo					CRITICALITY ignore	EXTENSION UuRLCChannelID							PRESENCE optional	}|

{ ID id-CG-SDTindicatorSetup			CRITICALITY reject	EXTENSION CG-SDTindicatorSetup						PRESENCE optional },

...

}

SRSCarrier-List ::= SEQUENCE (SIZE(1.. maxnoSRS-Carriers)) OF SRSCarrier-List-Item

SRSCarrier-List-Item ::= SEQUENCE {

pointA							INTEGER (0..3279165),

uplinkChannelBW-PerSCS-List		UplinkChannelBW-PerSCS-List,

activeULBWP						ActiveULBWP,

pci								NRPCI		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SRSCarrier-List-Item-ExtIEs } } OPTIONAL

}

SRSCarrier-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRSConfig  ::= SEQUENCE {

sRSResource-List			SRSResource-List 		OPTIONAL,

posSRSResource-List			PosSRSResource-List 	OPTIONAL,

sRSResourceSet-List			SRSResourceSet-List 	OPTIONAL,

posSRSResourceSet-List		PosSRSResourceSet-List 	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SRSConfig-ExtIEs } } OPTIONAL

}

SRSConfig-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRSConfiguration ::= SEQUENCE {

sRSCarrier-List		SRSCarrier-List,

iE-Extensions		ProtocolExtensionContainer { { SRSConfiguration-ExtIEs } } OPTIONAL

}

SRSConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SrsFrequency ::= INTEGER (0..3279165)

SRSPortIndex ::= ENUMERATED {id1000, id1001, id1002, id1003,...}

SRSPosResourceID ::= INTEGER (0..63)

SRSResource::= SEQUENCE {

sRSResourceID					SRSResourceID,

nrofSRS-Ports					ENUMERATED {port1, ports2, ports4},

transmissionComb				TransmissionComb,

startPosition					INTEGER (0..13),

nrofSymbols						ENUMERATED {n1, n2, n4},

repetitionFactor				ENUMERATED {n1, n2, n4},

freqDomainPosition				INTEGER (0..67),

freqDomainShift					INTEGER (0..268),

c-SRS							INTEGER (0..63),

b-SRS							INTEGER (0..3),

b-hop							INTEGER (0..3),

groupOrSequenceHopping			ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType					ResourceType,

sequenceId						INTEGER (0..1023),

iE-Extensions					ProtocolExtensionContainer { { SRSResource-ExtIEs } } OPTIONAL

}

SRSResource-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-nrofSymbolsExtended			CRITICALITY ignore EXTENSION NrofSymbolsExtended			PRESENCE optional}|

{ ID id-repetitionFactorExtended		CRITICALITY ignore EXTENSION RepetitionFactorExtended 	PRESENCE optional}|

{ ID id-startRBHopping			CRITICALITY ignore EXTENSION StartRBHopping 			PRESENCE optional}|

{ ID id-startRBIndex			CRITICALITY ignore EXTENSION StartRBIndex 			PRESENCE optional},

...

}

SRSResourceID ::= INTEGER (0..63)

SRSResourceID-List::= SEQUENCE (SIZE (1..maxnoSRS-ResourcePerSet)) OF SRSResourceID

SRSResource-List ::= SEQUENCE (SIZE (1..maxnoSRS-Resources)) OF SRSResource

SRSResourceSet::= SEQUENCE {

sRSResourceSetID				SRSResourceSetID,

sRSResourceID-List				SRSResourceID-List,

resourceSetType					ResourceSetType,

iE-Extensions					ProtocolExtensionContainer { { SRSResourceSet-ExtIEs } } OPTIONAL

}

SRSResourceSet-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRSResourceSetID ::= INTEGER (0..15, ...)

SRSResourceSetList ::= SEQUENCE (SIZE(1.. maxnoSRS-ResourceSets)) OF SRSResourceSetItem

SRSResourceSetItem ::= SEQUENCE {

numSRSresourcesperset		INTEGER (1..16, ...)	OPTIONAL,

periodicityList				PeriodicityList			OPTIONAL,

spatialRelationInfo			SpatialRelationInfo		OPTIONAL,

pathlossReferenceInfo		PathlossReferenceInfo	OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { SRSResourceSetItemExtIEs } }	OPTIONAL

}

SRSResourceSetItemExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SRSSpatialRelationPerSRSResource	CRITICALITY ignore	EXTENSION SpatialRelationPerSRSResource PRESENCE optional},

...

}

SRSResourceSet-List ::= SEQUENCE (SIZE (1..maxnoSRS-ResourceSets)) OF SRSResourceSet

SRSResourceTrigger ::= SEQUENCE {

aperiodicSRSResourceTriggerList					AperiodicSRSResourceTriggerList,

iE-Extensions		ProtocolExtensionContainer { {SRSResourceTrigger-ExtIEs} }	OPTIONAL

}

SRSResourceTrigger-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SRSResourcetype ::= SEQUENCE {

sRSResourceTypeChoice					SRSResourceTypeChoice,

iE-Extensions		ProtocolExtensionContainer { { SRSResourcetype-ExtIEs} }	OPTIONAL,

...

}

SRSResourcetype-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SRSPortIndex			CRITICALITY ignore EXTENSION SRSPortIndex	PRESENCE optional },

...

}

SRSResourceTypeChoice ::= CHOICE {

sRSResourceInfo				SRSInfo,

posSRSResourceInfo			PosSRSInfo,

choice-extension			ProtocolIE-SingleContainer { { SRSResourceTypeChoice-ExtIEs} }

}

SRSResourceTypeChoice-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SRSInfo ::= SEQUENCE {

sRSResource			SRSResourceID,

...

}

SRSPosRRCInactiveConfig ::= OCTET STRING

SRSPosRRCInactiveQueryIndication ::= ENUMERATED {true, ...}

PosSRSInfo ::= SEQUENCE {

posSRSResourceID		SRSPosResourceID,

...

}

SSB ::= SEQUENCE {

pCI-NR				NRPCI,

ssb-index			SSB-Index	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {SSB-ExtIEs} }	OPTIONAL

}

SSBCoverageModification-List ::= SEQUENCE (SIZE (1..maxnoofSSBAreas)) OF SSBCoverageModification-Item

SSBCoverageModification-Item::= SEQUENCE {

sSBIndex						INTEGER(0..63),

sSBCoverageState				SSBCoverageState,

iE-Extensions		ProtocolExtensionContainer { { SSBCoverageModification-Item-ExtIEs} }	OPTIONAL,

...

}

SSBCoverageModification-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBCoverageState ::= INTEGER (0..15, ...)

SSB-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSB-freqInfo ::= INTEGER (0..maxNRARFCN)

SSB-Index ::= INTEGER(0..63)

SSB-subcarrierSpacing ::=  ENUMERATED {kHz15, kHz30, kHz120, kHz240, spare3, spare2, spare1, ...}

SSB-transmissionPeriodicity	::= ENUMERATED {sf10, sf20, sf40, sf80, sf160, sf320, sf640, ..., sf5}

SSB-transmissionTimingOffset ::= INTEGER (0..127, ...)

SSB-transmissionBitmap ::= CHOICE {

shortBitmap			BIT STRING (SIZE (4)),

mediumBitmap		BIT STRING (SIZE (8)),

longBitmap			BIT STRING (SIZE (64)),

choice-extension	ProtocolIE-SingleContainer { { SSB-transmisisonBitmap-ExtIEs} }

}

SSB-transmisisonBitmap-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SSBAreaCapacityValueList ::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF	SSBAreaCapacityValueItem

SSBAreaCapacityValueItem ::= SEQUENCE {

sSBIndex				INTEGER(0..63),

sSBAreaCapacityValue	INTEGER (0..100),

iE-Extensions			ProtocolExtensionContainer { { SSBAreaCapacityValueItem-ExtIEs} } OPTIONAL

}

SSBAreaCapacityValueItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBAreaRadioResourceStatusList::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF	SSBAreaRadioResourceStatusItem

SSBAreaRadioResourceStatusItem::= SEQUENCE {

sSBIndex					INTEGER(0..63),

sSBAreaDLGBRPRBusage		INTEGER (0..100),

sSBAreaULGBRPRBusage		INTEGER (0..100),

sSBAreaDLnon-GBRPRBusage	INTEGER (0..100),

sSBAreaULnon-GBRPRBusage	INTEGER (0..100),

sSBAreaDLTotalPRBusage		INTEGER (0..100),

sSBAreaULTotalPRBusage		INTEGER (0..100),

dLschedulingPDCCHCCEusage	INTEGER (0..100)		OPTIONAL,

uLschedulingPDCCHCCEusage	INTEGER (0..100) 		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SSBAreaRadioResourceStatusItem-ExtIEs} } OPTIONAL

}

SSBAreaRadioResourceStatusItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBInformation ::= SEQUENCE {

sSBInformationList	SSBInformationList,

iE-Extensions		ProtocolExtensionContainer { { SSBInformation-ExtIEs } }	OPTIONAL

}

SSBInformation-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBInformationList ::= SEQUENCE (SIZE(1.. maxnoofSSBs)) OF SSBInformationItem

SSBInformationItem ::= SEQUENCE {

sSB-Configuration	SSB-TF-Configuration,

pCI-NR				NRPCI,

iE-Extensions		ProtocolExtensionContainer { { SSBInformationItem-ExtIEs } }	OPTIONAL

}

SSBInformationItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSB-PositionsInBurst ::= CHOICE {

shortBitmap						BIT STRING (SIZE (4)),

mediumBitmap					BIT STRING (SIZE (8)),

longBitmap						BIT STRING (SIZE (64)),

choice-extension				ProtocolIE-SingleContainer { {SSB-PositionsInBurst-ExtIEs} }

}

SSB-PositionsInBurst-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SSB-TF-Configuration ::= SEQUENCE {

sSB-frequency				INTEGER (0..3279165),

sSB-subcarrier-spacing		ENUMERATED {kHz15, kHz30, kHz60, kHz120, kHz240, ..., kHz480, kHz960},

-- The value kHz60 is not supported in this version of the specification.

sSB-Transmit-power			INTEGER (-60..50),

sSB-periodicity				ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, ...},

sSB-half-frame-offset		INTEGER(0..1),

sSB-SFN-offset				INTEGER(0..15),

sSB-position-in-burst		SSB-PositionsInBurst		OPTIONAL,

sFNInitialisationTime		RelativeTime1900		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { SSB-TF-Configuration-ExtIEs} } OPTIONAL

}

SSB-TF-Configuration-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SSBToReportList ::= SEQUENCE (SIZE(1.. maxnoofSSBAreas)) OF SSBToReportItem

SSBToReportItem ::= SEQUENCE {

sSBIndex					INTEGER(0..63),

iE-Extensions				ProtocolExtensionContainer { { SSBToReportItem-ExtIEs} } OPTIONAL

}

SSBToReportItem-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

StartRBIndex  ::= CHOICE{

freqScalingFactor2   INTEGER(0..1),

freqScalingFactor4   INTEGER(0..3),

choice-extension	 ProtocolIE-SingleContainer { { StartRBIndex-ExtIEs} }

}

StartRBIndex-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

StartRBHopping  ::= ENUMERATED {enable}

StartTimeAndDuration ::= SEQUENCE {

startTime			RelativeTime1900				OPTIONAL,

duration			INTEGER (0..90060, ...)			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { StartTimeAndDuration-ExtIEs } }	OPTIONAL,

...

}

StartTimeAndDuration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SUL-Information ::= SEQUENCE {

sUL-NRARFCN							INTEGER (0..maxNRARFCN),

sUL-transmission-Bandwidth			Transmission-Bandwidth,

iE-Extensions						ProtocolExtensionContainer { { SUL-InformationExtIEs} } OPTIONAL,

...

}

SUL-InformationExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-CarrierList				CRITICALITY ignore	EXTENSION NRCarrierList			PRESENCE optional }|

{ ID id-FrequencyShift7p5khz	CRITICALITY ignore	EXTENSION FrequencyShift7p5khz	PRESENCE optional },

...

}

SubcarrierSpacing ::=	ENUMERATED { kHz15, kHz30, kHz60, kHz120, kHz240, spare3, spare2, spare1, ...}

SubscriberProfileIDforRFP ::= INTEGER (1..256, ...)

SuccessfulHOReportInformationList::= SEQUENCE (SIZE(1.. maxnoofSuccessfulHOReports)) OF SuccessfulHOReportInformation-Item

SuccessfulHOReportInformation-Item ::= SEQUENCE {

successfulHOReportContainer				OCTET STRING,

iE-Extensions	ProtocolExtensionContainer { { SuccessfulHOReportInformation-Item-ExtIEs } }	OPTIONAL

}

SuccessfulHOReportInformation-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SULAccessIndication ::= ENUMERATED {true,...}

SupportedSULFreqBandItem ::= SEQUENCE {

freqBandIndicatorNr 			INTEGER (1..1024,...),

iE-Extensions					ProtocolExtensionContainer { { SupportedSULFreqBandItem-ExtIEs} } OPTIONAL,

...

}

SupportedSULFreqBandItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

SurvivalTime ::= INTEGER (0.. 1920000,...)

SymbolAllocInSlot ::= CHOICE {

all-DL					NULL,

all-UL					NULL,

both-DL-and-UL			NumDLULSymbols,

choice-extension		ProtocolIE-SingleContainer { { SymbolAllocInSlot-ExtIEs } }

}

SymbolAllocInSlot-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

SystemFrameNumber ::= INTEGER (0..1023)

SystemInformationAreaID ::=BIT STRING (SIZE (24))

#### -- T

FiveGS-TAC ::= OCTET STRING (SIZE(3))

Configured-EPS-TAC ::= OCTET STRING (SIZE(2))

TargetCellList ::= SEQUENCE (SIZE(1..maxnoofCHOcells)) OF TargetCellList-Item

TargetCellList-Item ::= SEQUENCE {

target-cell								NRCGI,

iE-Extensions							ProtocolExtensionContainer { { TargetCellList-Item-ExtIEs} } OPTIONAL

}

TargetCellList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NSAGSupportList ::= SEQUENCE (SIZE(1.. maxnoofNSAGs)) OF NSAGSupportItem

NSAGSupportItem ::= SEQUENCE {

nSAG-ID					NSAG-ID,

nSAGSliceSupport		ExtendedSliceSupportList,

iE-Extensions			ProtocolExtensionContainer { {NSAGSupportItem-ExtIEs} }	OPTIONAL,

...

}

NSAGSupportItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NSAG-ID ::= INTEGER (0..255, ...)

TDD-Info ::= SEQUENCE {

nRFreqInfo						NRFreqInfo,

transmission-Bandwidth			Transmission-Bandwidth,

iE-Extensions					ProtocolExtensionContainer { {TDD-Info-ExtIEs} } OPTIONAL,

...

}

TDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-IntendedTDD-DL-ULConfig	CRITICALITY ignore	EXTENSION	IntendedTDD-DL-ULConfig	PRESENCE optional}|

{ID id-TDD-UL-DLConfigCommonNR	CRITICALITY ignore	EXTENSION TDD-UL-DLConfigCommonNR	PRESENCE optional }|

{ID id-CarrierList				CRITICALITY ignore	EXTENSION NRCarrierList				PRESENCE optional }|

{ID id-Transmission-Bandwidth-asymmetric	CRITICALITY ignore	EXTENSION Transmission-Bandwidth-asymmetric		PRESENCE optional },

...

}

TDD-InfoRel16 ::= SEQUENCE {

tDD-FreqInfo									FreqInfoRel16													OPTIONAL,

sUL-FreqInfo									FreqInfoRel16													OPTIONAL,

tDD-UL-DLConfigCommonNR							TDD-UL-DLConfigCommonNR											OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {TDD-InfoRel16-ExtIEs} }		OPTIONAL,

...

}

TDD-InfoRel16-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TDD-UL-DLConfigCommonNR ::= OCTET STRING

TRPTEGInformation ::= CHOICE {

rxTx-TEG			RxTxTEG,

rx-TEG				RxTEG,

choice-extension		ProtocolIE-SingleContainer { { TRPTEGInformation-ExtIEs} }

}

TRPTEGInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

RxTxTEG ::= SEQUENCE {

tRP-RxTx-TEGInformation			TRP-RxTx-TEGInformation,

tRP-Tx-TEGInformation			TRP-Tx-TEGInformation		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { RxTxTEG-ExtIEs } }		OPTIONAL,

...

}

RxTxTEG-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

RxTEG ::= SEQUENCE {

tRP-Rx-TEGInformation		TRP-Rx-TEGInformation,

tRP-Tx-TEGInformation		TRP-Tx-TEGInformation,

iE-Extensions			ProtocolExtensionContainer { { RxTEG-ExtIEs } }		OPTIONAL,

...

}

RxTEG-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TimeReferenceInformation ::= SEQUENCE {

referenceTime					ReferenceTime,

referenceSFN					ReferenceSFN,

uncertainty						Uncertainty				OPTIONAL,

timeInformationType				TimeInformationType		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TimeReferenceInformation-ExtIEs} }	OPTIONAL

}

TimeReferenceInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TimeInformationType ::= ENUMERATED {localClock}

TimeStamp ::= SEQUENCE {

systemFrameNumber		SystemFrameNumber,

slotIndex				TimeStampSlotIndex,

measurementTime			RelativeTime1900	OPTIONAL,

iE-Extension			ProtocolExtensionContainer { { TimeStamp-ExtIEs} }	OPTIONAL

}

TimeStamp-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TimeStampSlotIndex ::= CHOICE {

sCS-15			INTEGER(0..9),

sCS-30			INTEGER(0..19),

sCS-60			INTEGER(0..39),

sCS-120			INTEGER(0..79),

choice-extension		ProtocolIE-SingleContainer { { TimeStampSlotIndex-ExtIEs} }

}

TimeStampSlotIndex-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-SCS-480		CRITICALITY reject	TYPE SCS-480 PRESENCE mandatory}|

{ ID id-SCS-960		CRITICALITY reject	TYPE SCS-960 PRESENCE mandatory},

...

}

TimeToWait ::= ENUMERATED {v1s, v2s, v5s, v10s, v20s, v60s, ...}

TimingErrorMargin ::= ENUMERATED {m0Tc, m2Tc, m4Tc, m6Tc, m8Tc, m12Tc, m16Tc, m20Tc, m24Tc, m32Tc, m40Tc, m48Tc, m56Tc, m64Tc, m72Tc, m80Tc, ...}

TimingMeasurementQuality ::= SEQUENCE {

measurementQuality		INTEGER(0..31),

resolution				ENUMERATED{m0dot1, m1, m10, m30, ...},

iE-Extensions			ProtocolExtensionContainer { { TimingMeasurementQuality-ExtIEs} }	OPTIONAL

}

TimingMeasurementQuality-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TMGI ::=  OCTET STRING (SIZE(6))

TNLAssociationUsage ::= ENUMERATED {

ue,

non-ue,

both,

...

}

TNLCapacityIndicator::= SEQUENCE {

dLTNLOfferedCapacity		INTEGER (1.. 16777216,...),

dLTNLAvailableCapacity		INTEGER (0.. 100,...),

uLTNLOfferedCapacity		INTEGER (1.. 16777216,...),

uLTNLAvailableCapacity		INTEGER (0.. 100,...),

iE-Extensions	ProtocolExtensionContainer { { TNLCapacityIndicator-ExtIEs} } OPTIONAL

}

TNLCapacityIndicator-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TraceActivation ::= SEQUENCE {

traceID								TraceID,

interfacesToTrace					InterfacesToTrace,

traceDepth							TraceDepth,

traceCollectionEntityIPAddress		TransportLayerAddress,

iE-Extensions		ProtocolExtensionContainer { {TraceActivation-ExtIEs} }	OPTIONAL

}

TraceActivation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID id-mdtConfiguration			CRITICALITY ignore	EXTENSION	MDTConfiguration		PRESENCE optional}|

{ID id-TraceCollectionEntityURI	CRITICALITY ignore	EXTENSION URI-address				PRESENCE optional	},

...

}

TraceDepth ::= ENUMERATED {

minimum,

medium,

maximum,

minimumWithoutVendorSpecificExtension,

mediumWithoutVendorSpecificExtension,

maximumWithoutVendorSpecificExtension,

...

}

TraceID ::= OCTET STRING (SIZE(8))

TrafficMappingInfo	::= CHOICE {

iPtolayer2TrafficMappingInfo					IPtolayer2TrafficMappingInfo,

bAPlayerBHRLCchannelMappingInfo					BAPlayerBHRLCchannelMappingInfo,

choice-extension								ProtocolIE-SingleContainer { { TrafficMappingInfo-ExtIEs} }

}

TrafficMappingInfo-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TransportLayerAddress		::= BIT STRING (SIZE(1..160, ...))

TransactionID				::= INTEGER (0..255, ...)

Transmission-Bandwidth ::= SEQUENCE {

nRSCS	NRSCS,

nRNRB	NRNRB,

iE-Extensions				ProtocolExtensionContainer { { Transmission-Bandwidth-ExtIEs} } OPTIONAL,

...

}

Transmission-Bandwidth-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Transmission-Bandwidth-asymmetric ::= SEQUENCE {

ul-Transmission-Bandwidth	Transmission-Bandwidth,

dl-Transmission-Bandwidth	Transmission-Bandwidth,

iE-Extensions				ProtocolExtensionContainer { { Transmission-Bandwidth-asymmetric-ExtIEs} } OPTIONAL,

...

}

Transmission-Bandwidth-asymmetric-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TransmissionComb ::= CHOICE {

n2    SEQUENCE {

combOffset-n2              INTEGER (0..1),

cyclicShift-n2             INTEGER (0..7)

},

n4    SEQUENCE {

combOffset-n4              INTEGER (0..3),

cyclicShift-n4             INTEGER (0..11)

},

choice-extension				ProtocolIE-SingleContainer { { TransmissionComb-ExtIEs} }

}

TransmissionComb-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-transmissionCombn8	CRITICALITY reject TYPE TransmissionCombn8 PRESENCE mandatory},

...

}

TransmissionCombn8 ::= SEQUENCE {

combOffset-n8              INTEGER (0..7),

cyclicShift-n8             INTEGER (0..5),

iE-Extensions			   ProtocolExtensionContainer { { TransmissionCombn8-ExtIEs} } OPTIONAL

}

TransmissionCombn8-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TransmissionCombPos ::= CHOICE {

n2    SEQUENCE {

combOffset-n2              INTEGER (0..1),

cyclicShift-n2             INTEGER (0..7)

},

n4    SEQUENCE {

combOffset-n4              INTEGER (0..3),

cyclicShift-n4             INTEGER (0..11)

},

n8    SEQUENCE {

combOffset-n8              INTEGER (0..7),

cyclicShift-n8             INTEGER (0..5)

},

choice-extension				ProtocolIE-SingleContainer { { TransmissionCombPos-ExtIEs} }

}

TransmissionCombPos-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TransmissionStopIndicator ::= ENUMERATED {true, ... }

Transport-UP-Layer-Address-Info-To-Add-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Address-Info-To-Add-Item

Transport-UP-Layer-Address-Info-To-Add-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressToAdd			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Address-Info-To-Add-ItemExtIEs } }	OPTIONAL

}

Transport-UP-Layer-Address-Info-To-Add-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Transport-UP-Layer-Address-Info-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofTLAs)) OF Transport-UP-Layer-Address-Info-To-Remove-Item

Transport-UP-Layer-Address-Info-To-Remove-Item ::= SEQUENCE {

iP-SecTransportLayerAddress		TransportLayerAddress,

gTPTransportLayerAddressToRemove			GTPTLAs							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Transport-UP-Layer-Address-Info-To-Remove-ItemExtIEs } }	OPTIONAL

}

Transport-UP-Layer-Address-Info-To-Remove-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TransmissionActionIndicator ::= ENUMERATED {stop, ..., restart }

TRPBeamAntennaInformation ::= SEQUENCE {

choice-TRP-Beam-Antenna-Info-Item		Choice-TRP-Beam-Antenna-Info-Item								,

iE-Extensions			ProtocolExtensionContainer {{ TRPBeamAntennaInformation-ExtIEs}}				OPTIONAL,

...

}

TRPBeamAntennaInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

Choice-TRP-Beam-Antenna-Info-Item ::= CHOICE {

reference					TRPID,

explicit					TRP-BeamAntennaExplicitInformation,

noChange					NULL,

choice-extension			ProtocolIE-SingleContainer { { Choice-TRP-Beam-Info-Item-ExtIEs } }

}

Choice-TRP-Beam-Info-Item-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TRP-BeamAntennaExplicitInformation ::= SEQUENCE {

trp-BeamAntennaAngles					TRP-BeamAntennaAngles,

lcs-to-gcs-translation	 				LCS-to-GCS-Translation										OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRP-BeamAntennaExplicitInformation-ExtIEs}}				OPTIONAL,

...

}

TRP-BeamAntennaExplicitInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRP-BeamAntennaAngles ::= SEQUENCE (SIZE (1.. maxnoAzimuthAngles)) OF TRP-BeamAntennaAnglesList-Item

TRP-BeamAntennaAnglesList-Item ::= SEQUENCE {

trp-azimuth-angle					INTEGER (0..359),

trp-azimuth-angle-fine				INTEGER (0..9)	OPTIONAL,

trp-elevation-angle-list			SEQUENCE (SIZE (1.. maxnoElevationAngles)) OF TRP-ElevationAngleList-Item,

iE-Extensions			ProtocolExtensionContainer {{ TRP-BeamAntennaAnglesList-Item-ExtIEs}}			OPTIONAL,

...

}

TRP-BeamAntennaAnglesList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRP-ElevationAngleList-Item ::= SEQUENCE {

trp-elevation-angle					INTEGER (0..180),

trp-elevation-angle-fine			INTEGER (0..9)	OPTIONAL,

trp-beam-power-list					SEQUENCE (SIZE (2..maxNumResourcesPerAngle)) OF TRP-Beam-Power-Item,

iE-Extensions			ProtocolExtensionContainer {{ TRP-ElevationAngleList-Item-ExtIEs}}			OPTIONAL,

...

}

TRP-ElevationAngleList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRP-Beam-Power-Item ::= SEQUENCE {

pRSResourceSetID				PRS-Resource-Set-ID													OPTIONAL,

pRSResourceID					PRS-Resource-ID,

relativePower					INTEGER (0..30), --negative value

relativePowerFine				INTEGER (0..9)			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ TRP-Beam-Power-Item-ExtIEs}}					OPTIONAL,

...

}

TRP-Beam-Power-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPID ::= INTEGER (0.. maxnoofTRPs, ...)

TRPInformation ::= SEQUENCE {

tRPID							TRPID,

tRPInformationTypeResponseList	TRPInformationTypeResponseList,

iE-Extensions					ProtocolExtensionContainer { { TRPInformation-ExtIEs } }		OPTIONAL

}

TRPInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPInformationItem ::= SEQUENCE {

tRPInformation					TRPInformation,

iE-Extensions					ProtocolExtensionContainer { { TRPInformationItem-ExtIEs } }	OPTIONAL

}

TRPInformationItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPInformationTypeItem ::= ENUMERATED {

nrPCI,

nG-RAN-CGI,

arfcn,

pRSConfig,

sSBConfig,

sFNInitTime,

spatialDirectInfo,

geoCoord,

...,

trp-type,

ondemandPRS,

trpTxTeg,

beam-antenna-info

}

TRPInformationTypeResponseList ::= SEQUENCE (SIZE(1.. maxnoofTRPInfoTypes)) OF TRPInformationTypeResponseItem

TRPInformationTypeResponseItem ::= CHOICE {

pCI-NR								NRPCI,

nG-RAN-CGI							NRCGI,

nRARFCN								INTEGER (0..maxNRARFCN),

pRSConfiguration					PRSConfiguration,

sSBinformation						SSBInformation,

sFNInitialisationTime				RelativeTime1900,

spatialDirectionInformation			SpatialDirectionInformation,

geographicalCoordinates				GeographicalCoordinates,

choice-extension					ProtocolIE-SingleContainer { { TRPInformationTypeResponseItem-ExtIEs} }

}

TRPInformationTypeResponseItem-ExtIEs F1AP-PROTOCOL-IES ::= {

{ ID id-TRPType						CRITICALITY reject		TYPE TRPType					PRESENCE mandatory }|

{ ID id-OnDemandPRS	 				CRITICALITY reject		TYPE OnDemandPRS-Info			PRESENCE mandatory}|

{ ID id-TRPTxTEGAssociation			CRITICALITY reject		TYPE TRPTxTEGAssociation		PRESENCE mandatory}|

{ ID id-TRPBeamAntennaInformation	CRITICALITY reject		TYPE TRPBeamAntennaInformation	PRESENCE mandatory },

...

}

TRPList ::= SEQUENCE (SIZE(1.. maxnoofTRPs)) OF TRPListItem

TRPListItem ::= SEQUENCE {

tRPID							TRPID,

iE-Extensions					ProtocolExtensionContainer { { TRPListItem-ExtIEs } }	OPTIONAL

}

TRPListItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPMeasurementQuality ::= SEQUENCE {

tRPmeasurementQuality-Item 	TRPMeasurementQuality-Item,

iE-Extensions				ProtocolExtensionContainer { {TRPMeasurementQuality-ExtIEs} } OPTIONAL

}

TRPMeasurementQuality-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPMeasurementQuality-Item ::= CHOICE {

timingMeasurementQuality	TimingMeasurementQuality,

angleMeasurementQuality		AngleMeasurementQuality,

choice-extension			ProtocolIE-SingleContainer { { TRPMeasurementQuality-Item-ExtIEs } }

}

TRPMeasurementQuality-Item-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TRP-MeasurementRequestList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementRequestItem

TRP-MeasurementRequestItem ::= SEQUENCE {

tRPID							TRPID,

search-window-information		Search-window-information	OPTIONAL,

iE-extensions		ProtocolExtensionContainer { { TRP-MeasurementRequestItem-ExtIEs } } OPTIONAL

}

TRP-MeasurementRequestItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NRCGI					CRITICALITY ignore EXTENSION NRCGI					PRESENCE optional }|

{ ID id-AoA-SearchWindow		CRITICALITY ignore EXTENSION AoA-AssistanceInfo		PRESENCE optional }|

{ ID id-NumberOfTRPRxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTEG		PRESENCE optional }|

{ ID id-NumberOfTRPRxTxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTxTEG		PRESENCE optional },

...

}

TRP-PRS-Info-List ::= SEQUENCE (SIZE(1.. maxnoofPRSTRPs)) OF TRP-PRS-Info-List-Item

TRP-PRS-Info-List-Item ::= SEQUENCE {

tRP-ID				TRPID,

nR-PCI				NRPCI,

cGI-NR				NRCGI					OPTIONAL,

pRSConfiguration							PRSConfiguration,

iE-Extensions	ProtocolExtensionContainer { { TRP-PRS-Info-List-Item-ExtIEs} } OPTIONAL,

...

}

TRP-PRS-Info-List-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPPositionDefinitionType ::= CHOICE {

direct		TRPPositionDirect,

referenced	TRPPositionReferenced,

choice-extension							ProtocolIE-SingleContainer { { TRPPositionDefinitionType-ExtIEs } }

}

TRPPositionDefinitionType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TRPPositionDirect ::= SEQUENCE {

accuracy	TRPPositionDirectAccuracy,

iE-extensions		ProtocolExtensionContainer { { TRPPositionDirect-ExtIEs } }	OPTIONAL

}

TRPPositionDirect-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPPositionDirectAccuracy ::= CHOICE {

tRPPosition				AccessPointPosition,

tRPHAposition			NGRANHighAccuracyAccessPointPosition,

choice-extension		ProtocolIE-SingleContainer { { TRPPositionDirectAccuracy-ExtIEs } }

}

TRPPositionDirectAccuracy-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TRPPositionReferenced ::= SEQUENCE {

referencePoint					ReferencePoint,

referencePointType				TRPReferencePointType,

iE-extensions					ProtocolExtensionContainer { { TRPPositionReferenced-ExtIEs } } 	OPTIONAL

}

TRPPositionReferenced-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPReferencePointType ::= CHOICE {

tRPPositionRelativeGeodetic			RelativeGeodeticLocation,

tRPPositionRelativeCartesian		RelativeCartesianLocation,

choice-extension					ProtocolIE-SingleContainer { { TRPReferencePointType-ExtIEs } }

}

TRPReferencePointType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

TRP-Rx-TEGInformation ::= SEQUENCE {

tRP-Rx-TEGID					INTEGER (0..31),

tRP-Rx-TimingErrorMargin		TimingErrorMargin,

iE-Extensions		ProtocolExtensionContainer { { TRP-Rx-TEGInformation-ExtIEs } } OPTIONAL,

...

}

TRP-Rx-TEGInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRP-RxTx-TEGInformation ::= SEQUENCE {

tRP-RxTx-TEGID					INTEGER (0..255),

tRP-RxTx-TimingErrorMargin		RxTxTimingErrorMargin,

iE-Extensions		ProtocolExtensionContainer { { TRP-RxTx-TEGInformation-ExtIEs } } OPTIONAL,

...

}

TRP-RxTx-TEGInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRP-Tx-TEGInformation ::= SEQUENCE {

tRP-Tx-TEGID					INTEGER (0..7),

tRP-Tx-TimingErrorMargin		TimingErrorMargin,

iE-Extensions		ProtocolExtensionContainer { { TRP-Tx-TEGInformation-ExtIEs } } OPTIONAL,

...

}

TRP-Tx-TEGInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPTxTEGAssociation ::= SEQUENCE (SIZE(1.. maxnoTRPTEGs)) OF TRPTEG-Item

TRPTEG-Item ::= SEQUENCE {

tRP-Tx-TEGInformation		TRP-Tx-TEGInformation,

dl-PRSResourceSetID			PRS-Resource-Set-ID,

dl-PRSResourceID-List	SEQUENCE (SIZE(1.. maxnoofPRS-ResourcesPerSet)) OF DLPRSResourceID-Item OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { TRPTEGItem-ExtIEs } } OPTIONAL,

...

}

TRPTEGItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

DLPRSResourceID-Item ::= SEQUENCE {

dl-PRSResourceID		PRS-Resource-ID,

iE-Extensions			ProtocolExtensionContainer { { DLPRSResource-Item-ExtIEs} }	OPTIONAL,

...

}

DLPRSResource-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

Transport-Layer-Address-Info ::= SEQUENCE {

transport-UP-Layer-Address-Info-To-Add-List		Transport-UP-Layer-Address-Info-To-Add-List							OPTIONAL,

transport-UP-Layer-Address-Info-To-Remove-List	Transport-UP-Layer-Address-Info-To-Remove-List					OPTIONAL,

iE-Extensions	ProtocolExtensionContainer { { Transport-Layer-Address-Info-ExtIEs } }								OPTIONAL

}

Transport-Layer-Address-Info-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRPType ::= ENUMERATED {

prsOnlyTP,

srsOnlyRP,

tp,

rp,

trp,

...

}

TSCAssistanceInformation ::= SEQUENCE {

periodicity				Periodicity,

burstArrivalTime		BurstArrivalTime													OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {TSCAssistanceInformation-ExtIEs} }	OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-SurvivalTime	CRITICALITY ignore	EXTENSION SurvivalTime	PRESENCE optional },

...

}

TSCTrafficCharacteristics ::= SEQUENCE {

tSCAssistanceInformationDL		TSCAssistanceInformation								OPTIONAL,

tSCAssistanceInformationUL		TSCAssistanceInformation								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TSCTrafficCharacteristics-ExtIEs} }	OPTIONAL,

...

}

TSCTrafficCharacteristics-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TRP-MeasurementUpdateList ::= SEQUENCE (SIZE (1..maxNoOfMeasTRPs)) OF TRP-MeasurementUpdateItem

TRP-MeasurementUpdateItem ::= SEQUENCE {

tRP-ID						TRPID,

aoA-window-information		AoA-AssistanceInfo	OPTIONAL,

iE-extensions				ProtocolExtensionContainer { { TRP-MeasurementUpdateItem-ExtIEs } } OPTIONAL,

...

}

TRP-MeasurementUpdateItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NumberOfTRPRxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTEG		PRESENCE optional }|

{ ID id-NumberOfTRPRxTxTEG		CRITICALITY ignore EXTENSION NumberOfTRPRxTxTEG		PRESENCE optional },

...

}

TwoPHRModeMCG ::= ENUMERATED {enabled, ...}

TwoPHRModeSCG ::= ENUMERATED {enabled, ...}

#### -- U

UAC-Assistance-Info ::= SEQUENCE {

uACPLMN-List		UACPLMN-List,

iE-Extensions		ProtocolExtensionContainer { { UAC-Assistance-InfoExtIEs} } OPTIONAL

}

UAC-Assistance-InfoExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UACPLMN-List ::= SEQUENCE (SIZE(1..maxnoofUACPLMNs)) OF UACPLMN-Item

UACPLMN-Item::= SEQUENCE {

pLMNIdentity				PLMN-Identity,

uACType-List				UACType-List,	iE-Extensions		ProtocolExtensionContainer { { UACPLMN-Item-ExtIEs} } OPTIONAL

}

UACPLMN-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NID	CRITICALITY ignore	EXTENSION NID	PRESENCE optional },

...

}

UACType-List ::= SEQUENCE (SIZE(1..maxnoofUACperPLMN)) OF UACType-Item

UACType-Item::= SEQUENCE {

uACReductionIndication 		UACReductionIndication,

uACCategoryType				UACCategoryType,

iE-Extensions		ProtocolExtensionContainer { { UACType-Item-ExtIEs } } OPTIONAL

}

UACType-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UACCategoryType ::= CHOICE {

uACstandardized				UACAction,

uACOperatorDefined			UACOperatorDefined,

choice-extension			ProtocolIE-SingleContainer { { UACCategoryType-ExtIEs } }

}

UACCategoryType-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UACOperatorDefined ::= SEQUENCE {

accessCategory					INTEGER (32..63,...),

accessIdentity					BIT STRING (SIZE(7)),

iE-Extensions		ProtocolExtensionContainer { { UACOperatorDefined-ExtIEs} } OPTIONAL

}

UACOperatorDefined-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UACAction ::= ENUMERATED {

reject-non-emergency-mo-dt,

reject-rrc-cr-signalling,

permit-emergency-sessions-and-mobile-terminated-services-only,

permit-high-priority-sessions-and-mobile-terminated-services-only,

...

}

UACReductionIndication ::= INTEGER (0..100)

UE-associatedLogicalF1-ConnectionItem ::= SEQUENCE {

gNB-CU-UE-F1AP-ID		GNB-CU-UE-F1AP-ID	 OPTIONAL,

gNB-DU-UE-F1AP-ID		GNB-DU-UE-F1AP-ID	 OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { UE-associatedLogicalF1-ConnectionItemExtIEs} } OPTIONAL,

...

}

UEAssistanceInformation ::= OCTET STRING

UEAssistanceInformationEUTRA ::= OCTET STRING

UE-associatedLogicalF1-ConnectionItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-CapabilityRAT-ContainerList::= OCTET STRING

UEContextNotRetrievable ::= ENUMERATED {true, ...}

UEIdentityIndexValue ::= CHOICE {

indexLength10			BIT STRING (SIZE (10)),

choice-extension		ProtocolIE-SingleContainer { {UEIdentityIndexValueChoice-ExtIEs} }

}

UEIdentityIndexValueChoice-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UEIdentity-List-For-Paging-Item		::= SEQUENCE {

uEIdentityIndexValue						UEIdentityIndexValue,

pagingDRX									PagingDRX		OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UEIdentity-List-For-Paging-Item-ExtIEs} } OPTIONAL

}

UEIdentity-List-For-Paging-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-MulticastMRBs-ConfirmedToBeModified-Item::= SEQUENCE {

mRB-ID						MRB-ID,

mrb-type-reconfiguration	MBSPTPRetransmissionTunnelRequired			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UE-MulticastMRBs-ConfirmedToBeModified-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-ConfirmedToBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-MulticastMRBs-RequiredToBeModified-Item::= SEQUENCE {

mRB-ID							MRB-ID,

mrb-type-reconfiguration		ENUMERATED {true, ...}							OPTIONAL,

mrb-reconfigured-RLCtype		ENUMERATED {

rlc-um-ptp,

rlc-am-ptp,

rlc-um-dl-ptm,

two-rlc-um-dl-ptp-and-dl-ptm,

three-rlc-um-dl-ptp-ul-ptp-dl-ptm,

two-rlc-am-ptp-um-dl-ptm,

...}								OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UE-MulticastMRBs-RequiredToBeModified-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-RequiredToBeModified-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-MulticastF1UContextReferenceCU				CRITICALITY reject	EXTENSION MulticastF1UContextReferenceCU					PRESENCE optional},

...

}

UE-MulticastMRBs-RequiredToBeReleased-Item::= SEQUENCE {

mRB-ID					MRB-ID,

iE-Extensions			ProtocolExtensionContainer { { UE-MulticastMRBs-RequiredToBeReleased-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-RequiredToBeReleased-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-MulticastMRBs-Setup-Item	::= SEQUENCE {

mRB-ID							MRB-ID,

multicastF1UContextReferenceCU	MulticastF1UContextReferenceCU,

iE-Extensions					ProtocolExtensionContainer { { UE-MulticastMRBs-Setup-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-Setup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-MulticastMRBs-Setupnew-Item	::= SEQUENCE {

mRB-ID							MRB-ID,

multicastF1UContextReferenceCU	MulticastF1UContextReferenceCU,

iE-Extensions					ProtocolExtensionContainer { { UE-MulticastMRBs-Setupnew-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-Setupnew-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-MulticastMRBs-ToBeReleased-Item	::= SEQUENCE {

mRB-ID					MRB-ID,

iE-Extensions			ProtocolExtensionContainer { { UE-MulticastMRBs-ToBeReleased-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-ToBeReleased-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UE-MulticastMRBs-ToBeSetup-Item	::= SEQUENCE {

mRB-ID									MRB-ID,

mbsPTPRetransmissionTunnelRequired		MBSPTPRetransmissionTunnelRequired			OPTIONAL,

mbsPTPForwardingRequiredInformation		MRB-ProgressInformation						OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { UE-MulticastMRBs-ToBeSetup-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-ToBeSetup-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-Source-MRB-ID		CRITICALITY ignore EXTENSION MRB-ID		PRESENCE optional },

...

}

UE-MulticastMRBs-ToBeSetup-atModify-Item	::= SEQUENCE {

mRB-ID									MRB-ID,

mbsPTPRetransmissionTunnelRequired		MBSPTPRetransmissionTunnelRequired			OPTIONAL,

mbsPTPForwardingRequiredInformation		MRB-ProgressInformation						OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { UE-MulticastMRBs-ToBeSetup-atModify-Item-ExtIEs } } OPTIONAL

}

UE-MulticastMRBs-ToBeSetup-atModify-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UEPagingCapability ::= SEQUENCE {

iNACTIVEStatePODetermination						ENUMERATED {supported, ...} 	OPTIONAL,

iE-Extension				ProtocolExtensionContainer { { UEPagingCapability-ExtIEs} } 		OPTIONAL,

...

}

UEPagingCapability-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-RedCapIndication		CRITICALITY ignore 	EXTENSION RedCapIndication		PRESENCE optional },

...

}

UEReportingInformation::= SEQUENCE {

reportingAmount				ENUMERATED {ma0, ma1, ma2, ma4, ma8, ma16, ma32, ma64},

reportingInterval			ENUMERATED {none, one, two, four, eight, ten, sixteen, twenty, thirty-two, sixty-four, ...},

iE-extensions			ProtocolExtensionContainer { { UEReportingInformation-ExtIEs } }	OPTIONAL,

...

}

UEReportingInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UlTxDirectCurrentMoreCarrierInformation::= OCTET STRING

UL-AoA ::= SEQUENCE {

azimuthAoA					INTEGER (0..3599),

zenithAoA					INTEGER (0..1799)		OPTIONAL,

lCS-to-GCS-Translation	LCS-to-GCS-Translation		OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { UL-AoA-ExtIEs } }	OPTIONAL,

...

}

UL-AoA-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-BH-Non-UP-Traffic-Mapping ::= SEQUENCE {

uL-BH-Non-UP-Traffic-Mapping-List			UL-BH-Non-UP-Traffic-Mapping-List,

iE-Extensions	ProtocolExtensionContainer { { UL-BH-Non-UP-Traffic-Mapping-ExtIEs } } OPTIONAL

}

UL-BH-Non-UP-Traffic-Mapping-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-BH-Non-UP-Traffic-Mapping-List ::= SEQUENCE (SIZE(1..maxnoofNonUPTrafficMappings)) OF UL-BH-Non-UP-Traffic-Mapping-Item

UL-BH-Non-UP-Traffic-Mapping-Item ::= SEQUENCE {

nonUPTrafficType				NonUPTrafficType,

bHInfo						BHInfo,

iE-Extensions					ProtocolExtensionContainer { { UL-BH-Non-UP-Traffic-Mapping-ItemExtIEs } }	OPTIONAL

}

UL-BH-Non-UP-Traffic-Mapping-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ULConfiguration ::= SEQUENCE	{

uLUEConfiguration		ULUEConfiguration,

iE-Extensions	ProtocolExtensionContainer { { ULConfigurationExtIEs } }	OPTIONAL,

...

}

ULConfigurationExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-GapFR2-Config ::= OCTET STRING

UL-RTOA-Measurement ::= SEQUENCE {

uL-RTOA-MeasurementItem		UL-RTOA-MeasurementItem,

additionalPath-List			AdditionalPath-List OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { UL-RTOA-Measurement-ExtIEs } }	OPTIONAL

}

UL-RTOA-Measurement-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-ExtendedAdditionalPathList	CRITICALITY ignore EXTENSION ExtendedAdditionalPathList	PRESENCE optional}|

{ ID id-TRPRx-TEGInformation		CRITICALITY ignore EXTENSION TRP-Rx-TEGInformation		PRESENCE optional},

...

}

UL-RTOA-MeasurementItem ::= CHOICE {

k0					INTEGER (0..1970049),

k1					INTEGER (0..985025),

k2					INTEGER (0..492513),

k3					INTEGER (0..246257),

k4					INTEGER (0..123129),

k5					INTEGER (0..61565),

choice-extension			ProtocolIE-SingleContainer { { UL-RTOA-MeasurementItem-ExtIEs } }

}

UL-RTOA-MeasurementItem-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UL-SRS-RSRP ::= INTEGER (0..126)

UL-SRS-RSRPP ::= SEQUENCE {

firstPathRSRPP				INTEGER (0..126),

iE-extensions			ProtocolExtensionContainer { { UL-SRS-RSRPP-ExtIEs } }	OPTIONAL,

...

}

UL-SRS-RSRPP-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ULUEConfiguration ::= ENUMERATED {no-data, shared, only, ...}

UL-UP-TNL-Information-to-Update-List-Item	::= SEQUENCE {

uLUPTNLInformation		UPTransportLayerInformation,

newULUPTNLInformation	UPTransportLayerInformation		OPTIONAL,

bHInfo	BHInfo,

iE-Extensions	ProtocolExtensionContainer { { UL-UP-TNL-Information-to-Update-List-ItemExtIEs } }	OPTIONAL,

...

}

UL-UP-TNL-Information-to-Update-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UL-UP-TNL-Address-to-Update-List-Item	::= SEQUENCE {

oldIPAdress						TransportLayerAddress,

newIPAdress						TransportLayerAddress,

iE-Extensions	ProtocolExtensionContainer { { UL-UP-TNL-Address-to-Update-List-ItemExtIEs } }	OPTIONAL,

...

}

UL-UP-TNL-Address-to-Update-List-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

ULUPTNLInformation-ToBeSetup-List ::= SEQUENCE (SIZE(1..maxnoofULUPTNLInformation)) OF ULUPTNLInformation-ToBeSetup-Item

ULUPTNLInformation-ToBeSetup-Item ::=SEQUENCE {

uLUPTNLInformation		UPTransportLayerInformation,

iE-Extensions	ProtocolExtensionContainer { { ULUPTNLInformation-ToBeSetup-ItemExtIEs } }	OPTIONAL,

...

}

ULUPTNLInformation-ToBeSetup-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-BHInfo			CRITICALITY ignore	EXTENSION BHInfo			PRESENCE optional	}|

{ ID id-DRBMappingInfo	CRITICALITY ignore	EXTENSION UuRLCChannelID	PRESENCE optional	},

...

}

Uncertainty ::= INTEGER (0..32767, ...)

UplinkChannelBW-PerSCS-List ::= SEQUENCE (SIZE (1..maxnoSCSs)) OF SCS-SpecificCarrier

UplinkTxDirectCurrentListInformation ::= OCTET STRING

UplinkTxDirectCurrentTwoCarrierListInfo ::= OCTET STRING

UPTransportLayerInformation		::= CHOICE {

gTPTunnel		GTPTunnel,

choice-extension			ProtocolIE-SingleContainer { { UPTransportLayerInformation-ExtIEs} }

}

UPTransportLayerInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

URI-address ::= VisibleString

Uncertainty-range-AoA ::= INTEGER (0..3599)

Uncertainty-range-ZoA ::= INTEGER (0..1799)

UuRLCChannelID ::= INTEGER (1..32)

UuRLCChannelQoSInformation ::= CHOICE {

uuRLCChannelQoS					QoSFlowLevelQoSParameters,

uuControlPlaneTrafficType		ENUMERATED {srb0,srb1,srb2,...},

choice-extension		ProtocolIE-SingleContainer { { UuRLCChannelQoSInformation-ExtIEs} }

}

UuRLCChannelQoSInformation-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UuRLCChannelToBeSetupList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelToBeSetupItem

UuRLCChannelToBeSetupItem ::= SEQUENCE {

uuRLCChannelID					UuRLCChannelID,

uuRLCChannelQoSInformation		UuRLCChannelQoSInformation,

rLCMode							RLCMode,

iE-Extensions					ProtocolExtensionContainer { { UuRLCChannelToBeSetupItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelToBeSetupItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelToBeModifiedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelToBeModifiedItem

UuRLCChannelToBeModifiedItem ::= SEQUENCE {

uuRLCChannelID					UuRLCChannelID,

uuRLCChannelQoSInformation		UuRLCChannelQoSInformation			OPTIONAL,

rLCMode							RLCMode			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { UuRLCChannelToBeModifiedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelToBeModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelToBeReleasedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelToBeReleasedItem

UuRLCChannelToBeReleasedItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelToBeReleasedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelToBeReleasedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelSetupList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelSetupItem

UuRLCChannelSetupItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelSetupItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelSetupItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelFailedToBeSetupList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelFailedToBeSetupItem

UuRLCChannelFailedToBeSetupItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

cause					Cause	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelFailedToBeSetupItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelFailedToBeSetupItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelModifiedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelModifiedItem

UuRLCChannelModifiedItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelModifiedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelFailedToBeModifiedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelFailedToBeModifiedItem

UuRLCChannelFailedToBeModifiedItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

cause					Cause	OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelFailedToBeModifiedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelFailedToBeModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelRequiredToBeModifiedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelRequiredToBeModifiedItem

UuRLCChannelRequiredToBeModifiedItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelRequiredToBeModifiedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelRequiredToBeModifiedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

UuRLCChannelRequiredToBeReleasedList ::= SEQUENCE (SIZE(1.. maxnoofUuRLCChannels)) OF UuRLCChannelRequiredToBeReleasedItem

UuRLCChannelRequiredToBeReleasedItem ::= SEQUENCE {

uuRLCChannelID			UuRLCChannelID,

iE-Extensions			ProtocolExtensionContainer { { UuRLCChannelRequiredToBeReleasedItem-ExtIEs } }	OPTIONAL,

...

}

UuRLCChannelRequiredToBeReleasedItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

#### -- V

VictimgNBSetID ::= SEQUENCE {

victimgNBSetID		GNBSetID,

iE-Extensions		ProtocolExtensionContainer { { VictimgNBSetID-ExtIEs } }		OPTIONAL

}

VictimgNBSetID-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

VehicleUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

PedestrianUE ::= ENUMERATED {

authorized,

not-authorized,

...

}

#### -- W

#### -- X

#### -- Y

#### -- Z

ZoAInformation ::= SEQUENCE {

zenithAoA					INTEGER (0..1799),

lCS-to-GCS-Translation	LCS-to-GCS-Translation		OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { ZoAInformation-ExtIEs } }	OPTIONAL,

...

}

ZoAInformation-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

END

-- ASN1STOP

### 9.4.6	Common Definitions

-- ASN1START

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

F1AP-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-CommonDataTypes (3) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

Criticality		::= ENUMERATED { reject, ignore, notify }

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0..65535),

global				OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolExtensionID	::= INTEGER (0..65535)

ProtocolIE-ID		::= INTEGER (0..65535)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome }

END

-- ASN1STOP

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

id-F1Setup									ProcedureCode ::= 1

id-ErrorIndication							ProcedureCode ::= 2

id-gNBDUConfigurationUpdate					ProcedureCode ::= 3

id-gNBCUConfigurationUpdate					ProcedureCode ::= 4

id-UEContextSetup							ProcedureCode ::= 5

id-UEContextRelease							ProcedureCode ::= 6

id-UEContextModification					ProcedureCode ::= 7

id-UEContextModificationRequired			ProcedureCode ::= 8

id-UEMobilityCommand						ProcedureCode ::= 9

id-UEContextReleaseRequest					ProcedureCode ::= 10

id-InitialULRRCMessageTransfer				ProcedureCode ::= 11

id-DLRRCMessageTransfer						ProcedureCode ::= 12

id-ULRRCMessageTransfer						ProcedureCode ::= 13

id-privateMessage							ProcedureCode ::= 14

id-UEInactivityNotification					ProcedureCode ::= 15

id-GNBDUResourceCoordination				ProcedureCode ::= 16

id-SystemInformationDeliveryCommand			ProcedureCode ::= 17

id-Paging									ProcedureCode ::= 18

id-Notify									ProcedureCode ::= 19

id-WriteReplaceWarning						ProcedureCode ::= 20

id-PWSCancel								ProcedureCode ::= 21

id-PWSRestartIndication						ProcedureCode ::= 22

id-PWSFailureIndication						ProcedureCode ::= 23

id-GNBDUStatusIndication 					ProcedureCode ::= 24

id-RRCDeliveryReport		 				ProcedureCode ::= 25

id-F1Removal								ProcedureCode ::= 26

id-NetworkAccessRateReduction				ProcedureCode ::= 27

id-TraceStart								ProcedureCode ::= 28

id-DeactivateTrace							ProcedureCode ::= 29

id-DUCURadioInformationTransfer				ProcedureCode ::= 30

id-CUDURadioInformationTransfer				ProcedureCode ::= 31

id-BAPMappingConfiguration					ProcedureCode ::= 32

id-GNBDUResourceConfiguration				ProcedureCode ::= 33

id-IABTNLAddressAllocation					ProcedureCode ::= 34

id-IABUPConfigurationUpdate					ProcedureCode ::= 35

id-resourceStatusReportingInitiation		ProcedureCode ::= 36

id-resourceStatusReporting					ProcedureCode ::= 37

id-accessAndMobilityIndication				ProcedureCode ::= 38

id-accessSuccess							ProcedureCode ::= 39

id-cellTrafficTrace 						ProcedureCode ::= 40

id-PositioningMeasurementExchange			ProcedureCode ::= 41

id-PositioningAssistanceInformationControl	ProcedureCode ::= 42

id-PositioningAssistanceInformationFeedback	ProcedureCode ::= 43

id-PositioningMeasurementReport				ProcedureCode ::= 44

id-PositioningMeasurementAbort				ProcedureCode ::= 45

id-PositioningMeasurementFailureIndication	ProcedureCode ::= 46

id-PositioningMeasurementUpdate				ProcedureCode ::= 47

id-TRPInformationExchange					ProcedureCode ::= 48

id-PositioningInformationExchange			ProcedureCode ::= 49

id-PositioningActivation					ProcedureCode ::= 50

id-PositioningDeactivation					ProcedureCode ::= 51

id-E-CIDMeasurementInitiation				ProcedureCode ::= 52

id-E-CIDMeasurementFailureIndication		ProcedureCode ::= 53

id-E-CIDMeasurementReport					ProcedureCode ::= 54

id-E-CIDMeasurementTermination				ProcedureCode ::= 55

id-PositioningInformationUpdate				ProcedureCode ::= 56

id-ReferenceTimeInformationReport			ProcedureCode ::= 57

id-ReferenceTimeInformationReportingControl	ProcedureCode ::= 58

id-BroadcastContextSetup					ProcedureCode ::= 59

id-BroadcastContextRelease					ProcedureCode ::= 60

id-BroadcastContextReleaseRequest			ProcedureCode ::= 61

id-BroadcastContextModification				ProcedureCode ::= 62

id-MulticastGroupPaging						ProcedureCode ::= 63

id-MulticastContextSetup					ProcedureCode ::= 64

id-MulticastContextRelease					ProcedureCode ::= 65

id-MulticastContextReleaseRequest			ProcedureCode ::= 66

id-MulticastContextModification				ProcedureCode ::= 67

id-MulticastDistributionSetup				ProcedureCode ::= 68

id-MulticastDistributionRelease				ProcedureCode ::= 69

id-PDCMeasurementInitiation					ProcedureCode ::= 70

id-PDCMeasurementReport						ProcedureCode ::= 71

id-PDCMeasurementInitiationRequest			ProcedureCode ::= 72

id-PDCMeasurementInitiationResponse			ProcedureCode ::= 73

id-PDCMeasurementInitiationFailure			ProcedureCode ::= 74

id-pRSConfigurationExchange					ProcedureCode ::= 75

id-measurementPreconfiguration				ProcedureCode ::= 76

id-measurementActivation					ProcedureCode ::= 77

id-QoEInformationTransfer					ProcedureCode ::= 78

id-PDCMeasurementTerminationCommand			ProcedureCode ::= 79

id-PDCMeasurementFailureIndication 			ProcedureCode ::= 80

id-PosSystemInformationDeliveryCommand		ProcedureCode ::= 81

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 65536

maxCellingNBDU							INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofNrCellBands						INTEGER ::= 32

maxnoofSIBTypes							INTEGER ::= 32

maxnoofSITypes							INTEGER ::= 32

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB							INTEGER ::= 256

maxnoofExtendedBPLMNs					INTEGER ::= 6

maxnoofUEIDs							INTEGER ::= 65536

maxnoofBPLMNsNR							INTEGER ::= 12

maxnoofUACPLMNs							INTEGER ::= 12

maxnoofUACperPLMN						INTEGER ::= 64

maxnoofAdditionalSIBs					INTEGER ::= 63

maxnoofslots							INTEGER ::= 5120

maxnoofTLAs								INTEGER ::=	16

maxnoofGTPTLAs							INTEGER ::=	16

maxnoofBHRLCChannels					INTEGER ::= 65536

maxnoofRoutingEntries					INTEGER ::= 1024

maxnoofIABSTCInfo						INTEGER ::= 45

maxnoofSymbols							INTEGER ::= 14

maxnoofServingCells						INTEGER ::= 32

maxnoofDUFSlots							INTEGER ::= 320

maxnoofHSNASlots						INTEGER ::= 5120

maxnoofServedCellsIAB					INTEGER ::= 512

maxnoofChildIABNodes					INTEGER ::= 1024

maxnoofNonUPTrafficMappings				INTEGER ::= 32

maxnoofTLAsIAB							INTEGER ::= 1024

maxnoofMappingEntries					INTEGER ::= 67108864

maxnoofDSInfo							INTEGER ::= 64

maxnoofEgressLinks						INTEGER ::= 2

maxnoofULUPTNLInformationforIAB			INTEGER ::= 32678

maxnoofUPTNLAddresses					INTEGER ::= 8

maxnoofSLDRBs							INTEGER ::= 512

maxnoofQoSParaSets						INTEGER ::= 8

maxnoofPC5QoSFlows						INTEGER ::= 2048

maxnoofSSBAreas							INTEGER ::=	64

maxnoofPhysicalResourceBlocks			INTEGER ::= 275

maxnoofPhysicalResourceBlocks-1			INTEGER ::= 274

maxnoofPRACHconfigs						INTEGER ::= 16

maxnoofRACHReports						INTEGER ::= 64

maxnoofRLFReports						INTEGER ::= 64

maxnoofAdditionalPDCPDuplicationTNL		INTEGER ::=	2

maxnoofRLCDuplicationState				INTEGER ::=	3

maxnoofCHOcells							INTEGER ::= 8

maxnoofMDTPLMNs							INTEGER ::=	16

maxnoofCAGsupported						INTEGER ::= 12

maxnoofNIDsupported						INTEGER ::= 12

maxnoofNRSCSs							INTEGER ::= 5

maxnoofExtSliceItems					INTEGER ::= 65535

maxnoofPosMeas							INTEGER ::=	16384

maxnoofTRPInfoTypes						INTEGER ::=	64

maxnoofTRPs								INTEGER ::=	65535

maxnoofSRSTriggerStates					INTEGER ::= 3

maxnoofSpatialRelations					INTEGER ::= 64

maxnoBcastCell							INTEGER ::= 16384

maxnoofAngleInfo						INTEGER ::= 65535

maxnooflcs-gcs-translation				INTEGER ::= 3

maxnoofPath								INTEGER ::= 2

maxnoofMeasE-CID						INTEGER ::= 64

maxnoofSSBs								INTEGER ::= 255

maxnoSRS-ResourceSets					INTEGER ::= 16

maxnoSRS-ResourcePerSet					INTEGER ::= 16

maxnoSRS-Carriers						INTEGER ::= 32

maxnoSCSs								INTEGER ::= 5

maxnoSRS-Resources						INTEGER ::= 64

maxnoSRS-PosResources					INTEGER ::= 64

maxnoSRS-PosResourceSets				INTEGER ::= 16

maxnoSRS-PosResourcePerSet				INTEGER ::= 16

maxnoofPRS-ResourceSets					INTEGER ::= 2

maxnoofPRS-ResourcesPerSet				INTEGER ::= 64

maxNoOfMeasTRPs							INTEGER ::= 64

maxnoofPRSresourceSets					INTEGER ::= 8

maxnoofPRSresources						INTEGER ::= 64

maxnoofSuccessfulHOReports				INTEGER ::= 64

maxnoofNR-UChannelIDs					INTEGER ::= 16

maxServedCellforSON						INTEGER ::= 256

maxNeighbourCellforSON					INTEGER ::= 32

maxAffectedCells						INTEGER ::= 32

maxnoofMRBs								INTEGER ::= 32

maxnoofMBSQoSFlows						INTEGER ::= 64

maxnoofMBSFSAs 							INTEGER ::= 256

maxnoofUEIDforPaging 					INTEGER ::= 4096

maxnoofCellsforMBS						INTEGER ::= 512

maxnoofTAIforMBS						INTEGER ::= 512

maxnoofMBSAreaSessionIDs				INTEGER ::= 256

maxnoofMBSServiceAreaInformation		INTEGER ::= 256

maxnoofIABCongInd						INTEGER ::= 1024

maxnoofNeighbourNodeCellsIAB			INTEGER ::= 1024

maxnoofRBsetsPerCell					INTEGER ::= 8

maxnoofRBsetsPerCell-1					INTEGER ::= 7

maxnoofMeasPDC							INTEGER ::= 16

maxnoARPs								INTEGER ::=	16

maxnoofULAoAs							INTEGER ::= 8

maxNoPathExtended						INTEGER ::= 8

maxnoTRPTEGs							INTEGER ::= 8

maxFreqLayers							INTEGER ::= 4

maxNumResourcesPerAngle					INTEGER ::= 24

maxnoAzimuthAngles						INTEGER ::= 3600

maxnoElevationAngles					INTEGER ::= 1801

maxnoofPRSTRPs							INTEGER ::= 256

maxnoofQoEInformation					INTEGER ::= 16

maxnoofUuRLCChannels					INTEGER ::= 32

maxnoofPC5RLCChannels					INTEGER ::= 512

maxnoofSMBRValues						INTEGER ::= 8

maxnoofMRBsforUE						INTEGER ::= 64

maxnoofMBSSessionsofUE					INTEGER ::= 256

maxnoofSLdestinations					INTEGER ::= 32

maxnoofNSAGs							INTEGER ::= 256

maxnoofSDTBearers						INTEGER ::= 72

maxnoofServingCellMOs					INTEGER ::= 16

maxNrofBWPs								INTEGER ::= 8

maxnoofPosSITypes						INTEGER ::= 32

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause											ProtocolIE-ID ::= 0

id-Cells-Failed-to-be-Activated-List				ProtocolIE-ID ::= 1

id-Cells-Failed-to-be-Activated-List-Item			ProtocolIE-ID ::= 2

id-Cells-to-be-Activated-List						ProtocolIE-ID ::= 3

id-Cells-to-be-Activated-List-Item					ProtocolIE-ID ::= 4

id-Cells-to-be-Deactivated-List						ProtocolIE-ID ::= 5

id-Cells-to-be-Deactivated-List-Item				ProtocolIE-ID ::= 6

id-CriticalityDiagnostics							ProtocolIE-ID ::= 7

id-CUtoDURRCInformation								ProtocolIE-ID ::= 9

id-DRBs-FailedToBeModified-Item						ProtocolIE-ID ::= 12

id-DRBs-FailedToBeModified-List						ProtocolIE-ID ::= 13

id-DRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 14

id-DRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 15

id-DRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 16

id-DRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 17

id-DRBs-ModifiedConf-Item							ProtocolIE-ID ::= 18

id-DRBs-ModifiedConf-List							ProtocolIE-ID ::= 19

id-DRBs-Modified-Item								ProtocolIE-ID ::= 20

id-DRBs-Modified-List								ProtocolIE-ID ::= 21

id-DRBs-Required-ToBeModified-Item					ProtocolIE-ID ::= 22

id-DRBs-Required-ToBeModified-List					ProtocolIE-ID ::= 23

id-DRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 24

id-DRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 25

id-DRBs-Setup-Item									ProtocolIE-ID ::= 26

id-DRBs-Setup-List									ProtocolIE-ID ::= 27

id-DRBs-SetupMod-Item								ProtocolIE-ID ::= 28

id-DRBs-SetupMod-List								ProtocolIE-ID ::= 29

id-DRBs-ToBeModified-Item							ProtocolIE-ID ::= 30

id-DRBs-ToBeModified-List							ProtocolIE-ID ::= 31

id-DRBs-ToBeReleased-Item							ProtocolIE-ID ::= 32

id-DRBs-ToBeReleased-List							ProtocolIE-ID ::= 33

id-DRBs-ToBeSetup-Item								ProtocolIE-ID ::= 34

id-DRBs-ToBeSetup-List								ProtocolIE-ID ::= 35

id-DRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 36

id-DRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 37

id-DRXCycle											ProtocolIE-ID ::= 38

id-DUtoCURRCInformation								ProtocolIE-ID ::= 39

id-gNB-CU-UE-F1AP-ID								ProtocolIE-ID ::= 40

id-gNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 41

id-gNB-DU-ID										ProtocolIE-ID ::= 42

id-GNB-DU-Served-Cells-Item							ProtocolIE-ID ::= 43

id-gNB-DU-Served-Cells-List							ProtocolIE-ID ::= 44

id-gNB-DU-Name										ProtocolIE-ID ::= 45

id-NRCellID											ProtocolIE-ID ::= 46

id-oldgNB-DU-UE-F1AP-ID								ProtocolIE-ID ::= 47

id-ResetType										ProtocolIE-ID ::= 48

id-ResourceCoordinationTransferContainer			ProtocolIE-ID ::= 49

id-RRCContainer										ProtocolIE-ID ::= 50

id-SCell-ToBeRemoved-Item							ProtocolIE-ID ::= 51

id-SCell-ToBeRemoved-List							ProtocolIE-ID ::= 52

id-SCell-ToBeSetup-Item								ProtocolIE-ID ::= 53

id-SCell-ToBeSetup-List								ProtocolIE-ID ::= 54

id-SCell-ToBeSetupMod-Item							ProtocolIE-ID ::= 55

id-SCell-ToBeSetupMod-List							ProtocolIE-ID ::= 56

id-Served-Cells-To-Add-Item							ProtocolIE-ID ::= 57

id-Served-Cells-To-Add-List							ProtocolIE-ID ::= 58

id-Served-Cells-To-Delete-Item						ProtocolIE-ID ::= 59

id-Served-Cells-To-Delete-List						ProtocolIE-ID ::= 60

id-Served-Cells-To-Modify-Item						ProtocolIE-ID ::= 61

id-Served-Cells-To-Modify-List						ProtocolIE-ID ::= 62

id-SpCell-ID										ProtocolIE-ID ::= 63

id-SRBID											ProtocolIE-ID ::= 64

id-SRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 65

id-SRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 66

id-SRBs-FailedToBeSetupMod-Item						ProtocolIE-ID ::= 67

id-SRBs-FailedToBeSetupMod-List						ProtocolIE-ID ::= 68

id-SRBs-Required-ToBeReleased-Item					ProtocolIE-ID ::= 69

id-SRBs-Required-ToBeReleased-List					ProtocolIE-ID ::= 70

id-SRBs-ToBeReleased-Item							ProtocolIE-ID ::= 71

id-SRBs-ToBeReleased-List							ProtocolIE-ID ::= 72

id-SRBs-ToBeSetup-Item								ProtocolIE-ID ::= 73

id-SRBs-ToBeSetup-List								ProtocolIE-ID ::= 74

id-SRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 75

id-SRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 76

id-TimeToWait										ProtocolIE-ID ::= 77

id-TransactionID									ProtocolIE-ID ::= 78

id-TransmissionActionIndicator						ProtocolIE-ID ::= 79

id-UE-associatedLogicalF1-ConnectionItem 			ProtocolIE-ID ::= 80

id-UE-associatedLogicalF1-ConnectionListResAck		ProtocolIE-ID ::= 81

id-gNB-CU-Name										ProtocolIE-ID ::= 82

id-SCell-FailedtoSetup-List							ProtocolIE-ID ::= 83

id-SCell-FailedtoSetup-Item							ProtocolIE-ID ::= 84

id-SCell-FailedtoSetupMod-List						ProtocolIE-ID ::= 85

id-SCell-FailedtoSetupMod-Item						ProtocolIE-ID ::= 86

id-RRCReconfigurationCompleteIndicator 				ProtocolIE-ID ::= 87

id-Cells-Status-Item								ProtocolIE-ID ::= 88

id-Cells-Status-List								ProtocolIE-ID ::= 89

id-Candidate-SpCell-List							ProtocolIE-ID ::= 90

id-Candidate-SpCell-Item							ProtocolIE-ID ::= 91

id-Potential-SpCell-List							ProtocolIE-ID ::= 92

id-Potential-SpCell-Item							ProtocolIE-ID ::= 93

id-FullConfiguration								ProtocolIE-ID ::= 94

id-C-RNTI											ProtocolIE-ID ::= 95

id-SpCellULConfigured								ProtocolIE-ID ::= 96

id-InactivityMonitoringRequest						ProtocolIE-ID ::= 97

id-InactivityMonitoringResponse						ProtocolIE-ID ::= 98

id-DRB-Activity-Item								ProtocolIE-ID ::= 99

id-DRB-Activity-List								ProtocolIE-ID ::= 100

id-EUTRA-NR-CellResourceCoordinationReq-Container	ProtocolIE-ID ::= 101

id-EUTRA-NR-CellResourceCoordinationReqAck-Container	ProtocolIE-ID ::= 102

id-Protected-EUTRA-Resources-List					ProtocolIE-ID ::= 105

id-RequestType 										ProtocolIE-ID ::= 106

id-ServCellIndex									ProtocolIE-ID ::= 107

id-RAT-FrequencyPriorityInformation					ProtocolIE-ID ::= 108

id-ExecuteDuplication								ProtocolIE-ID ::= 109

id-NRCGI											ProtocolIE-ID ::= 111

id-PagingCell-Item									ProtocolIE-ID ::= 112

id-PagingCell-List									ProtocolIE-ID ::= 113

id-PagingDRX										ProtocolIE-ID ::= 114

id-PagingPriority 									ProtocolIE-ID ::= 115

id-SItype-List										ProtocolIE-ID ::= 116

id-UEIdentityIndexValue								ProtocolIE-ID ::= 117

id-gNB-CUSystemInformation							ProtocolIE-ID ::= 118

id-HandoverPreparationInformation					ProtocolIE-ID ::= 119

id-GNB-CU-TNL-Association-To-Add-Item				ProtocolIE-ID ::= 120

id-GNB-CU-TNL-Association-To-Add-List				ProtocolIE-ID ::= 121

id-GNB-CU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 122

id-GNB-CU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 123

id-GNB-CU-TNL-Association-To-Update-Item			ProtocolIE-ID ::= 124

id-GNB-CU-TNL-Association-To-Update-List			ProtocolIE-ID ::= 125

id-MaskedIMEISV										ProtocolIE-ID ::= 126

id-PagingIdentity									ProtocolIE-ID ::= 127

id-DUtoCURRCContainer								ProtocolIE-ID ::= 128

id-Cells-to-be-Barred-List							ProtocolIE-ID ::= 129

id-Cells-to-be-Barred-Item							ProtocolIE-ID ::= 130

id-TAISliceSupportList								ProtocolIE-ID ::= 131

id-GNB-CU-TNL-Association-Setup-List				ProtocolIE-ID ::= 132

id-GNB-CU-TNL-Association-Setup-Item				ProtocolIE-ID ::= 133

id-GNB-CU-TNL-Association-Failed-To-Setup-List		ProtocolIE-ID ::= 134

id-GNB-CU-TNL-Association-Failed-To-Setup-Item		ProtocolIE-ID ::= 135

id-DRB-Notify-Item									ProtocolIE-ID ::= 136

id-DRB-Notify-List									ProtocolIE-ID ::= 137

id-NotficationControl								ProtocolIE-ID ::= 138

id-RANAC											ProtocolIE-ID ::= 139

id-PWSSystemInformation								ProtocolIE-ID ::= 140

id-RepetitionPeriod									ProtocolIE-ID ::= 141

id-NumberofBroadcastRequest							ProtocolIE-ID ::= 142

id-Cells-To-Be-Broadcast-List						ProtocolIE-ID ::= 144

id-Cells-To-Be-Broadcast-Item						ProtocolIE-ID ::= 145

id-Cells-Broadcast-Completed-List 					ProtocolIE-ID ::= 146

id-Cells-Broadcast-Completed-Item 					ProtocolIE-ID ::= 147

id-Broadcast-To-Be-Cancelled-List 					ProtocolIE-ID ::= 148

id-Broadcast-To-Be-Cancelled-Item 					ProtocolIE-ID ::= 149

id-Cells-Broadcast-Cancelled-List 					ProtocolIE-ID ::= 150

id-Cells-Broadcast-Cancelled-Item 					ProtocolIE-ID ::= 151

id-NR-CGI-List-For-Restart-List 					ProtocolIE-ID ::= 152

id-NR-CGI-List-For-Restart-Item 					ProtocolIE-ID ::= 153

id-PWS-Failed-NR-CGI-List 							ProtocolIE-ID ::= 154

id-PWS-Failed-NR-CGI-Item 							ProtocolIE-ID ::= 155

id-ConfirmedUEID									ProtocolIE-ID ::= 156

id-Cancel-all-Warning-Messages-Indicator			ProtocolIE-ID ::= 157

id-GNB-DU-UE-AMBR-UL								ProtocolIE-ID ::= 158

id-DRXConfigurationIndicator						ProtocolIE-ID ::= 159

id-RLC-Status										ProtocolIE-ID ::= 160

id-DLPDCPSNLength									ProtocolIE-ID ::= 161

id-GNB-DUConfigurationQuery							ProtocolIE-ID ::= 162

id-MeasurementTimingConfiguration					ProtocolIE-ID ::= 163

id-DRB-Information									ProtocolIE-ID ::= 164

id-ServingPLMN										ProtocolIE-ID ::= 165

id-Protected-EUTRA-Resources-Item					ProtocolIE-ID ::= 168

id-GNB-CU-RRC-Version								ProtocolIE-ID ::= 170

id-GNB-DU-RRC-Version								ProtocolIE-ID ::= 171

id-GNBDUOverloadInformation							ProtocolIE-ID ::= 172

id-CellGroupConfig									ProtocolIE-ID ::= 173

id-RLCFailureIndication								ProtocolIE-ID ::= 174

id-UplinkTxDirectCurrentListInformation				ProtocolIE-ID ::= 175

id-DC-Based-Duplication-Configured					ProtocolIE-ID ::= 176

id-DC-Based-Duplication-Activation					ProtocolIE-ID ::= 177

id-SULAccessIndication								ProtocolIE-ID ::= 178

id-AvailablePLMNList								ProtocolIE-ID ::= 179

id-PDUSessionID										ProtocolIE-ID ::= 180

id-ULPDUSessionAggregateMaximumBitRate				ProtocolIE-ID ::= 181

id-ServingCellMO									ProtocolIE-ID ::= 182

id-QoSFlowMappingIndication							ProtocolIE-ID ::= 183

id-RRCDeliveryStatusRequest							ProtocolIE-ID ::= 184

id-RRCDeliveryStatus								ProtocolIE-ID ::= 185

id-BearerTypeChange									ProtocolIE-ID ::= 186

id-RLCMode											ProtocolIE-ID ::= 187

id-Duplication-Activation							ProtocolIE-ID ::= 188

id-Dedicated-SIDelivery-NeededUE-List				ProtocolIE-ID ::= 189

id-Dedicated-SIDelivery-NeededUE-Item				ProtocolIE-ID ::= 190

id-DRX-LongCycleStartOffset							ProtocolIE-ID ::= 191

id-ULPDCPSNLength									ProtocolIE-ID ::= 192

id-SelectedBandCombinationIndex						ProtocolIE-ID ::= 193

id-SelectedFeatureSetEntryIndex						ProtocolIE-ID ::= 194

id-ResourceCoordinationTransferInformation			ProtocolIE-ID ::= 195

id-ExtendedServedPLMNs-List							ProtocolIE-ID ::= 196

id-ExtendedAvailablePLMN-List						ProtocolIE-ID ::= 197

id-Associated-SCell-List							ProtocolIE-ID ::= 198

id-latest-RRC-Version-Enhanced						ProtocolIE-ID ::= 199

id-Associated-SCell-Item							ProtocolIE-ID ::= 200

id-Cell-Direction									ProtocolIE-ID ::= 201

id-SRBs-Setup-List									ProtocolIE-ID ::= 202

id-SRBs-Setup-Item									ProtocolIE-ID ::= 203

id-SRBs-SetupMod-List								ProtocolIE-ID ::= 204

id-SRBs-SetupMod-Item								ProtocolIE-ID ::= 205

id-SRBs-Modified-List								ProtocolIE-ID ::= 206

id-SRBs-Modified-Item								ProtocolIE-ID ::= 207

id-Ph-InfoSCG										ProtocolIE-ID ::= 208

id-RequestedBandCombinationIndex					ProtocolIE-ID ::= 209

id-RequestedFeatureSetEntryIndex					ProtocolIE-ID ::= 210

id-RequestedP-MaxFR2								ProtocolIE-ID ::= 211

id-DRX-Config										ProtocolIE-ID ::= 212

id-IgnoreResourceCoordinationContainer				ProtocolIE-ID ::= 213

id-UEAssistanceInformation							ProtocolIE-ID ::= 214

id-NeedforGap										ProtocolIE-ID ::= 215

id-PagingOrigin										ProtocolIE-ID ::= 216

id-new-gNB-CU-UE-F1AP-ID							ProtocolIE-ID ::= 217

id-RedirectedRRCmessage								ProtocolIE-ID ::= 218

id-new-gNB-DU-UE-F1AP-ID							ProtocolIE-ID ::= 219

id-NotificationInformation							ProtocolIE-ID ::= 220

id-PLMNAssistanceInfoForNetShar						ProtocolIE-ID ::= 221

id-UEContextNotRetrievable							ProtocolIE-ID ::= 222

id-BPLMN-ID-Info-List								ProtocolIE-ID ::= 223

id-SelectedPLMNID									ProtocolIE-ID ::= 224

id-UAC-Assistance-Info								ProtocolIE-ID ::= 225

id-RANUEID											ProtocolIE-ID ::= 226

id-GNB-DU-TNL-Association-To-Remove-Item			ProtocolIE-ID ::= 227

id-GNB-DU-TNL-Association-To-Remove-List			ProtocolIE-ID ::= 228

id-TNLAssociationTransportLayerAddressgNBDU			ProtocolIE-ID ::= 229

id-portNumber										ProtocolIE-ID ::= 230

id-AdditionalSIBMessageList							ProtocolIE-ID ::= 231

id-Cell-Type										ProtocolIE-ID ::= 232

id-IgnorePRACHConfiguration							ProtocolIE-ID ::= 233

id-CG-Config										ProtocolIE-ID ::= 234

id-PDCCH-BlindDetectionSCG							ProtocolIE-ID ::= 235

id-Requested-PDCCH-BlindDetectionSCG				ProtocolIE-ID ::= 236

id-Ph-InfoMCG										ProtocolIE-ID ::= 237

id-MeasGapSharingConfig								ProtocolIE-ID ::= 238

id-systemInformationAreaID							ProtocolIE-ID ::= 239

id-areaScope										ProtocolIE-ID ::= 240

id-RRCContainer-RRCSetupComplete					ProtocolIE-ID ::= 241

id-TraceActivation									ProtocolIE-ID ::= 242

id-TraceID											ProtocolIE-ID ::= 243

id-Neighbour-Cell-Information-List					ProtocolIE-ID ::= 244

id-SymbolAllocInSlot								ProtocolIE-ID ::= 246

id-NumDLULSymbols									ProtocolIE-ID ::= 247

id-AdditionalRRMPriorityIndex						ProtocolIE-ID ::= 248

id-DUCURadioInformationType							ProtocolIE-ID ::= 249

id-CUDURadioInformationType 						ProtocolIE-ID ::= 250

id-AggressorgNBSetID								ProtocolIE-ID ::= 251

id-VictimgNBSetID									ProtocolIE-ID ::= 252

id-LowerLayerPresenceStatusChange					ProtocolIE-ID ::= 253

id-Transport-Layer-Address-Info						ProtocolIE-ID ::= 254

id-Neighbour-Cell-Information-Item					ProtocolIE-ID ::= 255

id-IntendedTDD-DL-ULConfig							ProtocolIE-ID ::= 256

id-QosMonitoringRequest								ProtocolIE-ID ::= 257

id-BHChannels-ToBeSetup-List						ProtocolIE-ID ::= 258

id-BHChannels-ToBeSetup-Item						ProtocolIE-ID ::= 259

id-BHChannels-Setup-List							ProtocolIE-ID ::= 260

id-BHChannels-Setup-Item							ProtocolIE-ID ::= 261

id-BHChannels-ToBeModified-Item						ProtocolIE-ID ::= 262

id-BHChannels-ToBeModified-List						ProtocolIE-ID ::= 263

id-BHChannels-ToBeReleased-Item						ProtocolIE-ID ::= 264

id-BHChannels-ToBeReleased-List						ProtocolIE-ID ::= 265

id-BHChannels-ToBeSetupMod-Item						ProtocolIE-ID ::= 266

id-BHChannels-ToBeSetupMod-List						ProtocolIE-ID ::= 267

id-BHChannels-FailedToBeModified-Item				ProtocolIE-ID ::= 268

id-BHChannels-FailedToBeModified-List				ProtocolIE-ID ::= 269

id-BHChannels-FailedToBeSetupMod-Item				ProtocolIE-ID ::= 270

id-BHChannels-FailedToBeSetupMod-List				ProtocolIE-ID ::= 271

id-BHChannels-Modified-Item							ProtocolIE-ID ::= 272

id-BHChannels-Modified-List							ProtocolIE-ID ::= 273

id-BHChannels-SetupMod-Item							ProtocolIE-ID ::= 274

id-BHChannels-SetupMod-List							ProtocolIE-ID ::= 275

id-BHChannels-Required-ToBeReleased-Item			ProtocolIE-ID ::= 276

id-BHChannels-Required-ToBeReleased-List			ProtocolIE-ID ::= 277

id-BHChannels-FailedToBeSetup-Item					ProtocolIE-ID ::= 278

id-BHChannels-FailedToBeSetup-List					ProtocolIE-ID ::= 279

id-BHInfo											ProtocolIE-ID ::= 280

id-BAPAddress										ProtocolIE-ID ::= 281

id-ConfiguredBAPAddress								ProtocolIE-ID ::= 282

id-BH-Routing-Information-Added-List				ProtocolIE-ID ::= 283

id-BH-Routing-Information-Added-List-Item			ProtocolIE-ID ::= 284

id-BH-Routing-Information-Removed-List				ProtocolIE-ID ::= 285

id-BH-Routing-Information-Removed-List-Item			ProtocolIE-ID ::= 286

id-UL-BH-Non-UP-Traffic-Mapping						ProtocolIE-ID ::= 287

id-Activated-Cells-to-be-Updated-List				ProtocolIE-ID ::= 288

id-Child-Nodes-List									ProtocolIE-ID ::= 289

id-IAB-Info-IAB-DU									ProtocolIE-ID ::= 290

id-IAB-Info-IAB-donor-CU							ProtocolIE-ID ::= 291

id-IAB-TNL-Addresses-To-Remove-List					ProtocolIE-ID ::= 292

id-IAB-TNL-Addresses-To-Remove-Item					ProtocolIE-ID ::= 293

id-IAB-Allocated-TNL-Address-List					ProtocolIE-ID ::= 294

id-IAB-Allocated-TNL-Address-Item					ProtocolIE-ID ::= 295

id-IABIPv6RequestType								ProtocolIE-ID ::= 296

id-IABv4AddressesRequested							ProtocolIE-ID ::= 297

id-IAB-Barred										ProtocolIE-ID ::= 298

id-TrafficMappingInformation						ProtocolIE-ID ::= 299

id-UL-UP-TNL-Information-to-Update-List				ProtocolIE-ID ::= 300

id-UL-UP-TNL-Information-to-Update-List-Item		ProtocolIE-ID ::= 301

id-UL-UP-TNL-Address-to-Update-List					ProtocolIE-ID ::= 302

id-UL-UP-TNL-Address-to-Update-List-Item			ProtocolIE-ID ::= 303

id-DL-UP-TNL-Address-to-Update-List					ProtocolIE-ID ::= 304

id-DL-UP-TNL-Address-to-Update-List-Item			ProtocolIE-ID ::= 305

id-NRV2XServicesAuthorized							ProtocolIE-ID ::= 306

id-LTEV2XServicesAuthorized							ProtocolIE-ID ::= 307

id-NRUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= 308

id-LTEUESidelinkAggregateMaximumBitrate				ProtocolIE-ID ::= 309

id-SIB12-message									ProtocolIE-ID ::= 310

id-SIB13-message									ProtocolIE-ID ::= 311

id-SIB14-message									ProtocolIE-ID ::= 312

id-SLDRBs-FailedToBeModified-Item					ProtocolIE-ID ::= 313

id-SLDRBs-FailedToBeModified-List					ProtocolIE-ID ::= 314

id-SLDRBs-FailedToBeSetup-Item						ProtocolIE-ID ::= 315

id-SLDRBs-FailedToBeSetup-List						ProtocolIE-ID ::= 316

id-SLDRBs-Modified-Item								ProtocolIE-ID ::= 317

id-SLDRBs-Modified-List								ProtocolIE-ID ::= 318

id-SLDRBs-Required-ToBeModified-Item				ProtocolIE-ID ::= 319

id-SLDRBs-Required-ToBeModified-List				ProtocolIE-ID ::= 320

id-SLDRBs-Required-ToBeReleased-Item				ProtocolIE-ID ::= 321

id-SLDRBs-Required-ToBeReleased-List				ProtocolIE-ID ::= 322

id-SLDRBs-Setup-Item								ProtocolIE-ID ::= 323

id-SLDRBs-Setup-List								ProtocolIE-ID ::= 324

id-SLDRBs-ToBeModified-Item							ProtocolIE-ID ::= 325

id-SLDRBs-ToBeModified-List							ProtocolIE-ID ::= 326

id-SLDRBs-ToBeReleased-Item							ProtocolIE-ID ::= 327

id-SLDRBs-ToBeReleased-List							ProtocolIE-ID ::= 328

id-SLDRBs-ToBeSetup-Item							ProtocolIE-ID ::= 329

id-SLDRBs-ToBeSetup-List							ProtocolIE-ID ::= 330

id-SLDRBs-ToBeSetupMod-Item							ProtocolIE-ID ::= 331

id-SLDRBs-ToBeSetupMod-List							ProtocolIE-ID ::= 332

id-SLDRBs-SetupMod-List								ProtocolIE-ID ::= 333

id-SLDRBs-FailedToBeSetupMod-List					ProtocolIE-ID ::= 334

id-SLDRBs-SetupMod-Item								ProtocolIE-ID ::= 335

id-SLDRBs-FailedToBeSetupMod-Item					ProtocolIE-ID ::= 336

id-SLDRBs-ModifiedConf-List							ProtocolIE-ID ::= 337

id-SLDRBs-ModifiedConf-Item							ProtocolIE-ID ::= 338

id-UEAssistanceInformationEUTRA						ProtocolIE-ID ::= 339

id-PC5LinkAMBR										ProtocolIE-ID ::= 340

id-SL-PHY-MAC-RLC-Config							ProtocolIE-ID ::= 341

id-SL-ConfigDedicatedEUTRA-Info						ProtocolIE-ID ::= 342

id-AlternativeQoSParaSetList						ProtocolIE-ID ::= 343

id-CurrentQoSParaSetIndex							ProtocolIE-ID ::= 344

id-gNBCUMeasurementID								ProtocolIE-ID ::= 345

id-gNBDUMeasurementID								ProtocolIE-ID ::= 346

id-RegistrationRequest								ProtocolIE-ID ::= 347

id-ReportCharacteristics							ProtocolIE-ID ::= 348

id-CellToReportList									ProtocolIE-ID ::= 349

id-CellMeasurementResultList						ProtocolIE-ID ::= 350

id-HardwareLoadIndicator							ProtocolIE-ID ::= 351

id-ReportingPeriodicity								ProtocolIE-ID ::= 352

id-TNLCapacityIndicator								ProtocolIE-ID ::= 353

id-CarrierList										ProtocolIE-ID ::= 354

id-ULCarrierList									ProtocolIE-ID ::= 355

id-FrequencyShift7p5khz								ProtocolIE-ID ::= 356

id-SSB-PositionsInBurst								ProtocolIE-ID ::= 357

id-NRPRACHConfig									ProtocolIE-ID ::= 358

id-RACHReportInformationList						ProtocolIE-ID ::= 359

id-RLFReportInformationList							ProtocolIE-ID ::= 360

id-TDD-UL-DLConfigCommonNR							ProtocolIE-ID ::= 361

id-CNPacketDelayBudgetDownlink						ProtocolIE-ID ::= 362

id-ExtendedPacketDelayBudget						ProtocolIE-ID ::= 363

id-TSCTrafficCharacteristics						ProtocolIE-ID ::= 364

id-ReportingRequestType								ProtocolIE-ID ::= 365

id-TimeReferenceInformation							ProtocolIE-ID ::= 366

id-CNPacketDelayBudgetUplink						ProtocolIE-ID ::= 369

id-AdditionalPDCPDuplicationTNL-List				ProtocolIE-ID ::= 370

id-RLCDuplicationInformation						ProtocolIE-ID ::= 371

id-AdditionalDuplicationIndication					ProtocolIE-ID ::= 372

id-ConditionalInterDUMobilityInformation			ProtocolIE-ID ::= 373

id-ConditionalIntraDUMobilityInformation			ProtocolIE-ID ::= 374

id-targetCellsToCancel								ProtocolIE-ID ::= 375

id-requestedTargetCellGlobalID						ProtocolIE-ID ::= 376

id-ManagementBasedMDTPLMNList						ProtocolIE-ID ::= 377

id-TraceCollectionEntityIPAddress 					ProtocolIE-ID ::= 378

id-PrivacyIndicator									ProtocolIE-ID ::= 379

id-TraceCollectionEntityURI							ProtocolIE-ID ::= 380

id-mdtConfiguration									ProtocolIE-ID ::= 381

id-ServingNID										ProtocolIE-ID ::= 382

id-NPNBroadcastInformation							ProtocolIE-ID ::= 383

id-NPNSupportInfo									ProtocolIE-ID ::= 384

id-NID												ProtocolIE-ID ::= 385

id-AvailableSNPN-ID-List							ProtocolIE-ID ::= 386

id-SIB10-message									ProtocolIE-ID ::= 387

id-DLCarrierList									ProtocolIE-ID ::= 389

id-ExtendedTAISliceSupportList					ProtocolIE-ID ::= 390

id-RequestedSRSTransmissionCharacteristics			ProtocolIE-ID ::= 391

id-PosAssistance-Information						ProtocolIE-ID ::= 392

id-PosBroadcast										ProtocolIE-ID ::= 393

id-RoutingID										ProtocolIE-ID ::= 394

id-PosAssistanceInformationFailureList				ProtocolIE-ID ::= 395

id-PosMeasurementQuantities							ProtocolIE-ID ::= 396

id-PosMeasurementResultList							ProtocolIE-ID ::= 397

id-TRPInformationTypeListTRPReq						ProtocolIE-ID ::= 398

id-TRPInformationTypeItem							ProtocolIE-ID ::= 399

id-TRPInformationListTRPResp						ProtocolIE-ID ::= 400

id-TRPInformationItem								ProtocolIE-ID ::= 401

id-LMF-MeasurementID								ProtocolIE-ID ::= 402

id-SRSType											ProtocolIE-ID ::= 403

id-ActivationTime									ProtocolIE-ID ::= 404

id-AbortTransmission								ProtocolIE-ID ::= 405

id-PositioningBroadcastCells						ProtocolIE-ID ::= 406

id-SRSConfiguration									ProtocolIE-ID ::= 407

id-PosReportCharacteristics							ProtocolIE-ID ::= 408

id-PosMeasurementPeriodicity						ProtocolIE-ID ::= 409

id-TRPList											ProtocolIE-ID ::= 410

id-RAN-MeasurementID								ProtocolIE-ID ::= 411

id-LMF-UE-MeasurementID								ProtocolIE-ID ::= 412

id-RAN-UE-MeasurementID								ProtocolIE-ID ::= 413

id-E-CID-MeasurementQuantities						ProtocolIE-ID ::= 414

id-E-CID-MeasurementQuantities-Item					ProtocolIE-ID ::= 415

id-E-CID-MeasurementPeriodicity						ProtocolIE-ID ::= 416

id-E-CID-MeasurementResult							ProtocolIE-ID ::= 417

id-Cell-Portion-ID									ProtocolIE-ID ::= 418

id-SFNInitialisationTime							ProtocolIE-ID ::= 419

id-SystemFrameNumber								ProtocolIE-ID ::= 420

id-SlotNumber										ProtocolIE-ID ::= 421

id-TRP-MeasurementRequestList						ProtocolIE-ID ::= 422

id-MeasurementBeamInfoRequest						ProtocolIE-ID ::= 423

id-E-CID-ReportCharacteristics						ProtocolIE-ID ::= 424

id-ConfiguredTACIndication							ProtocolIE-ID ::= 425

id-Extended-GNB-CU-Name								ProtocolIE-ID ::= 426

id-Extended-GNB-DU-Name								ProtocolIE-ID ::= 427

id-F1CTransferPath									ProtocolIE-ID ::= 428

id-SFN-Offset										ProtocolIE-ID ::= 429

id-TransmissionStopIndicator						ProtocolIE-ID ::= 430

id-SrsFrequency										ProtocolIE-ID ::= 431

id-SCGIndicator										ProtocolIE-ID ::= 432

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-TRPType											ProtocolIE-ID ::= 434

id-SRSSpatialRelationPerSRSResource					ProtocolIE-ID ::= 435

id-PDCPTerminatingNodeDLTNLAddrInfo					ProtocolIE-ID ::= 436

id-ENBDLTNLAddress									ProtocolIE-ID ::= 437

id-PosMeasurementPeriodicityExtended				ProtocolIE-ID ::= 438

id-PRS-Resource-ID									ProtocolIE-ID ::= 439

id-LocationMeasurementInformation					ProtocolIE-ID ::= 440

id-SliceRadioResourceStatus							ProtocolIE-ID ::= 441

id-CompositeAvailableCapacity-SUL					ProtocolIE-ID ::= 442

id-SuccessfulHOReportInformationList				ProtocolIE-ID ::= 443

id-NR-U-Channel-List								ProtocolIE-ID ::= 444

id-NR-U												ProtocolIE-ID ::= 445

id-Coverage-Modification-Notification				ProtocolIE-ID ::= 446

id-CCO-Assistance-Information						ProtocolIE-ID ::= 447

id-Neighbor-node-CCO-Assistance-Information-List	ProtocolIE-ID ::= 448

id-CellsForSON-List									ProtocolIE-ID ::= 449

id-MIMOPRBusageInformation							ProtocolIE-ID ::= 450

id-gNB-CU-MBS-F1AP-ID								ProtocolIE-ID ::= 451

id-gNB-DU-MBS-F1AP-ID								ProtocolIE-ID ::= 452

id-MBS-Area-Session-ID								ProtocolIE-ID ::= 453

id-MBS-CUtoDURRCInformation							ProtocolIE-ID ::= 454

id-MBS-Session-ID									ProtocolIE-ID ::= 455

id-SNSSAI											ProtocolIE-ID ::= 456

id-MBS-Broadcast-NeighbourCellList					ProtocolIE-ID ::= 457

id-BroadcastMRBs-FailedToBeModified-List			ProtocolIE-ID ::= 458

id-BroadcastMRBs-FailedToBeModified-Item			ProtocolIE-ID ::= 459

id-BroadcastMRBs-FailedToBeSetup-List				ProtocolIE-ID ::= 460

id-BroadcastMRBs-FailedToBeSetup-Item				ProtocolIE-ID ::= 461

id-BroadcastMRBs-FailedToBeSetupMod-List			ProtocolIE-ID ::= 462

id-BroadcastMRBs-FailedToBeSetupMod-Item			ProtocolIE-ID ::= 463

id-BroadcastMRBs-Modified-List						ProtocolIE-ID ::= 464

id-BroadcastMRBs-Modified-Item						ProtocolIE-ID ::= 465

id-BroadcastMRBs-Setup-List							ProtocolIE-ID ::= 466

id-BroadcastMRBs-Setup-Item							ProtocolIE-ID ::= 467

id-BroadcastMRBs-SetupMod-List						ProtocolIE-ID ::= 468

id-BroadcastMRBs-SetupMod-Item						ProtocolIE-ID ::= 469

id-BroadcastMRBs-ToBeModified-List					ProtocolIE-ID ::= 470

id-BroadcastMRBs-ToBeModified-Item					ProtocolIE-ID ::= 471

id-BroadcastMRBs-ToBeReleased-List					ProtocolIE-ID ::= 472

id-BroadcastMRBs-ToBeReleased-Item					ProtocolIE-ID ::= 473

id-BroadcastMRBs-ToBeSetup-List						ProtocolIE-ID ::= 474

id-BroadcastMRBs-ToBeSetup-Item						ProtocolIE-ID ::= 475

id-BroadcastMRBs-ToBeSetupMod-List					ProtocolIE-ID ::= 476

id-BroadcastMRBs-ToBeSetupMod-Item					ProtocolIE-ID ::= 477

id-Supported-MBS-FSA-ID-List						ProtocolIE-ID ::= 478

id-UEIdentity-List-For-Paging-List 					ProtocolIE-ID ::= 479

id-UEIdentity-List-For-Paging-Item 					ProtocolIE-ID ::= 480

id-MBS-ServiceArea									ProtocolIE-ID ::= 481

id-MulticastMRBs-FailedToBeModified-List			ProtocolIE-ID ::= 482

id-MulticastMRBs-FailedToBeModified-Item			ProtocolIE-ID ::= 483

id-MulticastMRBs-FailedToBeSetup-List				ProtocolIE-ID ::= 484

id-MulticastMRBs-FailedToBeSetup-Item				ProtocolIE-ID ::= 485

id-MulticastMRBs-FailedToBeSetupMod-List			ProtocolIE-ID ::= 486

id-MulticastMRBs-FailedToBeSetupMod-Item			ProtocolIE-ID ::= 487

id-MulticastMRBs-Modified-List						ProtocolIE-ID ::= 488

id-MulticastMRBs-Modified-Item						ProtocolIE-ID ::= 489

id-MulticastMRBs-Setup-List							ProtocolIE-ID ::= 490

id-MulticastMRBs-Setup-Item							ProtocolIE-ID ::= 491

id-MulticastMRBs-SetupMod-List						ProtocolIE-ID ::= 492

id-MulticastMRBs-SetupMod-Item						ProtocolIE-ID ::= 493

id-MulticastMRBs-ToBeModified-List					ProtocolIE-ID ::= 494

id-MulticastMRBs-ToBeModified-Item					ProtocolIE-ID ::= 495

id-MulticastMRBs-ToBeReleased-List					ProtocolIE-ID ::= 496

id-MulticastMRBs-ToBeReleased-Item					ProtocolIE-ID ::= 497

id-MulticastMRBs-ToBeSetup-List						ProtocolIE-ID ::= 498

id-MulticastMRBs-ToBeSetup-Item						ProtocolIE-ID ::= 499

id-MulticastMRBs-ToBeSetupMod-List					ProtocolIE-ID ::= 500

id-MulticastMRBs-ToBeSetupMod-Item					ProtocolIE-ID ::= 501

id-MBSMulticastF1UContextDescriptor					ProtocolIE-ID ::= 502

id-MulticastF1UContext-ToBeSetup-List				ProtocolIE-ID ::= 503

id-MulticastF1UContext-ToBeSetup-Item				ProtocolIE-ID ::= 504

id-MulticastF1UContext-Setup-List					ProtocolIE-ID ::= 505

id-MulticastF1UContext-Setup-Item					ProtocolIE-ID ::= 506

id-MulticastF1UContext-FailedToBeSetup-List			ProtocolIE-ID ::= 507

id-MulticastF1UContext-FailedToBeSetup-Item			ProtocolIE-ID ::= 508

id-IABCongestionIndication							ProtocolIE-ID ::= 509

id-IABConditionalRRCMessageDeliveryIndication		ProtocolIE-ID ::= 510

id-F1CTransferPathNRDC								ProtocolIE-ID ::= 511

id-BufferSizeThresh 								ProtocolIE-ID ::= 512

id-IAB-TNL-Addresses-Exception						ProtocolIE-ID ::= 513

id-BAP-Header-Rewriting-Added-List					ProtocolIE-ID ::= 514

id-BAP-Header-Rewriting-Added-List-Item				ProtocolIE-ID ::= 515

id-Re-routingEnableIndicator						ProtocolIE-ID ::= 516

id-NonF1terminatingTopologyIndicator				ProtocolIE-ID ::= 517

id-EgressNonF1terminatingTopologyIndicator			ProtocolIE-ID ::= 518

id-IngressNonF1terminatingTopologyIndicator			ProtocolIE-ID ::= 519

id-rBSetConfiguration 								ProtocolIE-ID ::= 520

id-frequency-Domain-HSNA-Configuration-List			ProtocolIE-ID ::= 521

id-child-IAB-Nodes-NA-Resource-List					ProtocolIE-ID ::= 522

id-Parent-IAB-Nodes-NA-Resource-Configuration-List	ProtocolIE-ID ::= 523

id-uL-FreqInfo										ProtocolIE-ID ::= 524

id-uL-Transmission-Bandwidth						ProtocolIE-ID ::= 525

id-dL-FreqInfo										ProtocolIE-ID ::= 526

id-dL-Transmission-Bandwidth						ProtocolIE-ID ::= 527

id-uL-NR-Carrier-List								ProtocolIE-ID ::= 528

id-dL-NR-Carrier-List								ProtocolIE-ID ::= 529

id-nRFreqInfo										ProtocolIE-ID ::= 530

id-transmission-Bandwidth							ProtocolIE-ID ::= 531

id-nR-Carrier-List									ProtocolIE-ID ::= 532

id-Neighbour-Node-Cells-List						ProtocolIE-ID ::= 533

id-Serving-Cells-List								ProtocolIE-ID ::= 534

id-permutation										ProtocolIE-ID ::= 535

id-MDTPollutedMeasurementIndicator					ProtocolIE-ID ::= 536

id-M5ReportAmount 									ProtocolIE-ID ::= 537

id-M6ReportAmount 									ProtocolIE-ID ::= 538

id-M7ReportAmount 									ProtocolIE-ID ::= 539

id-SurvivalTime										ProtocolIE-ID ::= 540

id-PDCMeasurementPeriodicity						ProtocolIE-ID ::= 541

id-PDCMeasurementQuantities							ProtocolIE-ID ::= 542

id-PDCMeasurementQuantities-Item					ProtocolIE-ID ::= 543

id-PDCMeasurementResult								ProtocolIE-ID ::= 544

id-PDCReportType									ProtocolIE-ID ::= 545

id-RAN-UE-PDC-MeasID								ProtocolIE-ID ::= 546

id-SCGActivationRequest								ProtocolIE-ID ::= 547

id-SCGActivationStatus								ProtocolIE-ID ::= 548

id-PRSTRPList										ProtocolIE-ID ::= 549

id-PRSTransmissionTRPList							ProtocolIE-ID ::= 550

id-OnDemandPRS										ProtocolIE-ID ::= 551

id-AoA-SearchWindow									ProtocolIE-ID ::= 552

id-TRP-MeasurementUpdateList						ProtocolIE-ID ::= 553

id-ZoAInformation									ProtocolIE-ID ::= 554

id-ResponseTime										ProtocolIE-ID ::= 555

id-ARPLocationInfo									ProtocolIE-ID ::= 556

id-ARP-ID											ProtocolIE-ID ::= 557

id-MultipleULAoA									ProtocolIE-ID ::= 558

id-UL-SRS-RSRPP										ProtocolIE-ID ::= 559

id-SRSResourcetype									ProtocolIE-ID ::= 560

id-ExtendedAdditionalPathList						ProtocolIE-ID ::= 561

id-LoS-NLoSInformation								ProtocolIE-ID ::= 562

id-NumberOfTRPRxTEG									ProtocolIE-ID ::= 564

id-NumberOfTRPRxTxTEG								ProtocolIE-ID ::= 565

id-TRPTxTEGAssociation								ProtocolIE-ID ::= 566

id-TRPTEGInformation								ProtocolIE-ID ::= 567

id-TRPRx-TEGInformation								ProtocolIE-ID ::= 568

id-TRP-PRS-Info-List								ProtocolIE-ID ::= 569

id-PRS-Measurement-Info-List						ProtocolIE-ID ::= 570

id-PRSConfigRequestType								ProtocolIE-ID ::= 571

id-MeasurementTimeOccasion							ProtocolIE-ID ::= 573

id-MeasurementCharacteristicsRequestIndicator		ProtocolIE-ID ::= 574

id-UEReportingInformation							ProtocolIE-ID ::= 575

id-PosConextRevIndication							ProtocolIE-ID ::= 576

id-TRPBeamAntennaInformation						ProtocolIE-ID ::= 577

id-NRRedCapUEIndication 							ProtocolIE-ID ::= 578

id-Redcap-Bcast-Information							ProtocolIE-ID ::= 579

id-RANUEPagingDRX									ProtocolIE-ID ::= 580

id-CNUEPagingDRX									ProtocolIE-ID ::= 581

id-NRPagingeDRXInformation							ProtocolIE-ID ::= 582

id-NRPagingeDRXInformationforRRCINACTIVE			ProtocolIE-ID ::= 583

id-NR-TADV											ProtocolIE-ID ::= 584

id-QoEInformation									ProtocolIE-ID ::= 585

id-CG-SDTQueryIndication							ProtocolIE-ID ::= 586

id-SDT-MAC-PHY-CG-Config							ProtocolIE-ID ::= 587

id-CG-SDTKeptIndicator								ProtocolIE-ID ::= 588

id-CG-SDTindicatorSetup								ProtocolIE-ID ::= 589

id-CG-SDTindicatorMod								ProtocolIE-ID ::= 590

id-CG-SDTSessionInfoOld								ProtocolIE-ID ::= 591

id-SDTInformation									ProtocolIE-ID ::= 592

id-SDTRLCBearerConfiguration						ProtocolIE-ID ::= 593

id-FiveG-ProSeAuthorized							ProtocolIE-ID ::= 594

id-FiveG-ProSeUEPC5AggregateMaximumBitrate			ProtocolIE-ID ::= 595

id-FiveG-ProSePC5LinkAMBR	 						ProtocolIE-ID ::= 596

id-SRBMappingInfo									ProtocolIE-ID ::= 597

id-DRBMappingInfo									ProtocolIE-ID ::= 598

id-UuRLCChannelToBeSetupList						ProtocolIE-ID ::= 599

id-UuRLCChannelToBeModifiedList						ProtocolIE-ID ::= 600

id-UuRLCChannelToBeReleasedList						ProtocolIE-ID ::= 601

id-UuRLCChannelSetupList							ProtocolIE-ID ::= 602

id-UuRLCChannelFailedToBeSetupList					ProtocolIE-ID ::= 603

id-UuRLCChannelModifiedList							ProtocolIE-ID ::= 604

id-UuRLCChannelFailedToBeModifiedList				ProtocolIE-ID ::= 605

id-UuRLCChannelRequiredToBeModifiedList				ProtocolIE-ID ::= 606

id-UuRLCChannelRequiredToBeReleasedList				ProtocolIE-ID ::= 607

id-PC5RLCChannelToBeSetupList						ProtocolIE-ID ::= 608

id-PC5RLCChannelToBeModifiedList					ProtocolIE-ID ::= 609

id-PC5RLCChannelToBeReleasedList					ProtocolIE-ID ::= 610

id-PC5RLCChannelSetupList							ProtocolIE-ID ::= 611

id-PC5RLCChannelFailedToBeSetupList					ProtocolIE-ID ::= 612

id-PC5RLCChannelFailedToBeModifiedList				ProtocolIE-ID ::= 613

id-PC5RLCChannelRequiredToBeModifiedList			ProtocolIE-ID ::= 614

id-PC5RLCChannelRequiredToBeReleasedList			ProtocolIE-ID ::= 615

id-PC5RLCChannelModifiedList						ProtocolIE-ID ::= 616

id-SidelinkRelayConfiguration						ProtocolIE-ID ::= 617

id-UpdatedRemoteUELocalID							ProtocolIE-ID ::= 618

id-PathSwitchConfiguration							ProtocolIE-ID ::= 619

id-PagingCause										ProtocolIE-ID ::= 620

id-MUSIM-GapConfig									ProtocolIE-ID ::= 621

id-PEIPSAssistanceInfo								ProtocolIE-ID ::= 622

id-UEPagingCapability								ProtocolIE-ID ::= 623

id-LastUsedCellIndication							ProtocolIE-ID ::= 624

id-SIB17-message									ProtocolIE-ID ::= 625

id-GNBDUUESliceMaximumBitRateList					ProtocolIE-ID ::= 626

id-SIB20-message									ProtocolIE-ID ::= 627

id-UE-MulticastMRBs-ToBeReleased-List				ProtocolIE-ID ::= 628

id-UE-MulticastMRBs-ToBeReleased-Item				ProtocolIE-ID ::= 629

id-UE-MulticastMRBs-ToBeSetup-List					ProtocolIE-ID ::= 630

id-UE-MulticastMRBs-ToBeSetup-Item					ProtocolIE-ID ::= 631

id-MulticastMBSSessionSetupList						ProtocolIE-ID ::= 632

id-MulticastMBSSessionRemoveList					ProtocolIE-ID ::= 633

id-PosMeasurementAmount								ProtocolIE-ID ::= 634

id-SDT-Termination-Request							ProtocolIE-ID ::= 635

id-pathPower										ProtocolIE-ID ::= 636

id-DU-RX-MT-RX-Extend								ProtocolIE-ID ::= 637

id-DU-TX-MT-TX-Extend								ProtocolIE-ID ::= 638

id-DU-RX-MT-TX-Extend								ProtocolIE-ID ::= 639

id-DU-TX-MT-RX-Extend								ProtocolIE-ID ::= 640

id-BAP-Header-Rewriting-Removed-List				ProtocolIE-ID ::= 641

id-BAP-Header-Rewriting-Removed-List-Item			ProtocolIE-ID ::= 642

id-SLDRXCycleList									ProtocolIE-ID ::= 643

id-TAINSAGSupportList								ProtocolIE-ID ::= 644

id-SL-RLC-ChannelToAddModList						ProtocolIE-ID ::= 645

id-BroadcastAreaScope								ProtocolIE-ID ::= 646

id-ManagementBasedMDTPLMNModificationList 			ProtocolIE-ID ::= 647

id-SIB15-message									ProtocolIE-ID ::= 648

id-ActivationRequestType							ProtocolIE-ID ::= 649

id-PosMeasGapPreConfigList							ProtocolIE-ID ::= 650

id-InterFrequencyConfig-NoGap						ProtocolIE-ID ::= 651

id-MBSInterestIndication							ProtocolIE-ID ::= 652

id-UE-MulticastMRBs-ConfirmedToBeModified-List		ProtocolIE-ID ::= 653

id-UE-MulticastMRBs-ConfirmedToBeModified-Item		ProtocolIE-ID ::= 654

id-UE-MulticastMRBs-RequiredToBeModified-List		ProtocolIE-ID ::= 655

id-UE-MulticastMRBs-RequiredToBeModified-Item		ProtocolIE-ID ::= 656

id-UE-MulticastMRBs-RequiredToBeReleased-List		ProtocolIE-ID ::= 657

id-UE-MulticastMRBs-RequiredToBeReleased-Item		ProtocolIE-ID ::= 658

id-L571Info											ProtocolIE-ID ::= 659

id-L1151Info										ProtocolIE-ID ::= 660

id-SCS-480											ProtocolIE-ID ::= 661

id-SCS-960											ProtocolIE-ID ::= 662

id-SRSPortIndex										ProtocolIE-ID ::= 663

id-PEISubgroupingSupportIndication					ProtocolIE-ID ::= 664

id-NeedForGapsInfoNR								ProtocolIE-ID ::= 665

id-NeedForGapNCSGInfoNR								ProtocolIE-ID ::= 666

id-NeedForGapNCSGInfoEUTRA							ProtocolIE-ID ::= 667

id-procedure-code-668-not-to-be-used				ProtocolIE-ID ::= 668

id-procedure-code-669-not-to-be-used				ProtocolIE-ID ::= 669

id-procedure-code-670-not-to-be-used				ProtocolIE-ID ::= 670

id-Source-MRB-ID									ProtocolIE-ID ::= 671

id-PosMeasurementPeriodicityNR-AoA					ProtocolIE-ID ::= 672

id-RedCapIndication									ProtocolIE-ID ::= 673

id-SRSPosRRCInactiveConfig							ProtocolIE-ID ::= 674

id-SDTBearerConfigurationQueryIndication			ProtocolIE-ID ::= 675

id-SDTBearerConfigurationInfo						ProtocolIE-ID ::= 676

id-UL-GapFR2-Config									ProtocolIE-ID ::= 677

id-ConfigRestrictInfoDAPS							ProtocolIE-ID ::= 678

id-UE-MulticastMRBs-Setup-List						ProtocolIE-ID ::= 679

id-UE-MulticastMRBs-Setup-Item						ProtocolIE-ID ::= 680

id-MulticastF1UContextReferenceCU					ProtocolIE-ID ::= 681

id-PosSItypeList									ProtocolIE-ID ::= 682

id-DAPS-HO-Status									ProtocolIE-ID ::= 683

id-UplinkTxDirectCurrentTwoCarrierListInfo			ProtocolIE-ID ::= 684

id-UE-MulticastMRBs-ToBeSetup-atModify-List			ProtocolIE-ID ::= 685

id-UE-MulticastMRBs-ToBeSetup-atModify-Item			ProtocolIE-ID ::= 686

id-MC-PagingCell-List								ProtocolIE-ID ::= 687

id-MC-PagingCell-Item								ProtocolIE-ID ::= 688

id-SRSPosRRCInactiveQueryIndication					ProtocolIE-ID ::= 689

id-UlTxDirectCurrentMoreCarrierInformation	        ProtocolIE-ID ::= 690

id-CPACMCGInformation								ProtocolIE-ID ::= 691

id-TwoPHRModeMCG									ProtocolIE-ID ::= 692

id-TwoPHRModeSCG									ProtocolIE-ID ::= 693

id-ExtendedUEIdentityIndexValue						ProtocolIE-ID ::= 694

id-ServingCellMO-List								ProtocolIE-ID ::= 695

id-ServingCellMO-List-Item							ProtocolIE-ID ::= 696

id-ServingCellMO-encoded-in-CGC-List				ProtocolIE-ID ::= 697

id-HashedUEIdentityIndexValue						ProtocolIE-ID ::= 698

id-UE-MulticastMRBs-Setupnew-List					ProtocolIE-ID ::= 699

id-UE-MulticastMRBs-Setupnew-Item					ProtocolIE-ID ::= 700

id-ncd-SSB-RedCapInitialBWP-SDT						ProtocolIE-ID ::= 701

id-nrofSymbolsExtended								ProtocolIE-ID ::= 702

id-repetitionFactorExtended							ProtocolIE-ID ::= 703

id-startRBHopping									ProtocolIE-ID ::= 704

id-startRBIndex										ProtocolIE-ID ::= 705

id-transmissionCombn8								ProtocolIE-ID ::= 706

id-ServCellInfoList									ProtocolIE-ID ::= 707

id-DedicatedSIDeliveryIndication					ProtocolIE-ID ::= 708

id-Configured-BWP-List								ProtocolIE-ID ::= 709

id-Preconfigured-measurement-GAP-Request			ProtocolIE-ID ::= 710

id-BWP-Id											ProtocolIE-ID ::= 711

id-SL-PHY-MAC-RLC-ConfigExt							ProtocolIE-ID ::= 800

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

END

-- ASN1STOP

### 9.4.8	Container Definitions

-- ASN1START

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

F1AP-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Containers (5) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

Presence,

PrivateIE-ID,

ProtocolExtensionID,

ProtocolIE-ID

FROM F1AP-CommonDataTypes

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs

FROM F1AP-Constants;

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

F1AP-PROTOCOL-IES ::= CLASS {

&id				ProtocolIE-ID 					UNIQUE,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

F1AP-PROTOCOL-IES-PAIR ::= CLASS {

&id					ProtocolIE-ID 				UNIQUE,

&firstCriticality	Criticality,

&FirstValue,

&secondCriticality	Criticality,

&SecondValue,

&presence			Presence

}

WITH SYNTAX {

ID				&id

FIRST CRITICALITY		&firstCriticality

FIRST TYPE				&FirstValue

SECOND CRITICALITY		&secondCriticality

SECOND TYPE				&SecondValue

PRESENCE				&presence

}

-- **************************************************************

--

-- Class Definition for Protocol Extensions

--

-- **************************************************************

F1AP-PROTOCOL-EXTENSION ::= CLASS {

&id				ProtocolExtensionID			UNIQUE,

&criticality	Criticality,

&Extension,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

EXTENSION		&Extension

PRESENCE		&presence

}

-- **************************************************************

--

-- Class Definition for Private IEs

--

-- **************************************************************

F1AP-PRIVATE-IES ::= CLASS {

&id				PrivateIE-ID,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

-- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container {F1AP-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-SingleContainer {F1AP-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field {F1AP-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				F1AP-PROTOCOL-IES.&id				({IEsSetParam}),

criticality		F1AP-PROTOCOL-IES.&criticality		({IEsSetParam}{@id}),

value			F1AP-PROTOCOL-IES.&Value			({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container for Protocol IE Pairs

--

-- **************************************************************

ProtocolIE-ContainerPair {F1AP-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair {F1AP-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {

id					F1AP-PROTOCOL-IES-PAIR.&id					({IEsSetParam}),

firstCriticality	F1AP-PROTOCOL-IES-PAIR.&firstCriticality	({IEsSetParam}{@id}),

firstValue			F1AP-PROTOCOL-IES-PAIR.&FirstValue			({IEsSetParam}{@id}),

secondCriticality	F1AP-PROTOCOL-IES-PAIR.&secondCriticality	({IEsSetParam}{@id}),

secondValue			F1AP-PROTOCOL-IES-PAIR.&SecondValue			({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer {F1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField {F1AP-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					F1AP-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			F1AP-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		F1AP-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

-- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container {F1AP-PRIVATE-IES : IEsSetParam } ::=

SEQUENCE (SIZE (1.. maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field {F1AP-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id					F1AP-PRIVATE-IES.&id				({IEsSetParam}),

criticality			F1AP-PRIVATE-IES.&criticality		({IEsSetParam}{@id}),

value				F1AP-PRIVATE-IES.&Value				({IEsSetParam}{@id})

}

END

-- ASN1STOP

## 9.5	Message Transfer Syntax

F1AP shall use the ASN.1 Basic Packed Encoding Rules (BASIC-PER) Aligned Variant as transfer syntax, as specified in ITU-T Recommendation X.691 [5].

## 9.6	Timers

# 10	Handling of unknown, unforeseen and erroneous protocol data

Clause 10 of TS 38.413 [3] is applicable for the purposes of the present document, with the following additions for non-UE-associated procedures:

-	In case of Abstract Syntax Error, when reporting the Criticality Diagnostics IE for not comprehended IE/IEgroups or missing IE/IE groups, the Transaction ID IE shall also be included;

-	In case of Logical Error, when reporting the Criticality Diagnostics IE, the Transaction ID IE shall also be included;

- 	In case of Logical Error in a response message of a Class 1 procedure, or failure to comprehend Transaction ID IE from a received message, the procedure shall be considered as unsuccessfully terminated or not terminated (e.g., transaction ID unknown in response message), and local error handling shall be initiated.

Annex A (informative):
Change History

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-06 | R3 NR#2 | R3-172493 | - | - | - | First version | 0.1.0 |
| 2017-07 | R3 NR#2 | R3-172640 | - | - | - | Incorporated agreed TPs from R3 NR#2 Adhoc | 0.2.0 |
| 2017-08 | R3#97 | R3-173451 | - | - | - | Incorporated agreed TPs from R3#97 | 0.3.0 |
| 2017-10 | R3#97b | R3-174247 | - | - | - | Incorporated agreed TPs from R3#97b | 0.4.0 |
| 2017-12 | R3#98 | R3-175062 | - | - | - | Incorporated agreed TPs from R3#98 | 0.5.0 |
| 2017-12 | RAN#78 | RP-172287 |  |  |  | Submitted for approval to RAN | 1.0.0 |
| 2017-12 | RAN#78 |  |  |  |  | TR approved by RAN plenary | 15.0.0 |
| 2018-03 | RP-79 | RP-180468 | 0001 | 2 | B | Baseline CR for March version of TS 38.473 covering agreements of RAN3#99 | 15.1.0 |
| 2018-04 |  |  |  |  |  | Editorial correction to ASN.1 (correction to id-TimeToWait ProtocolIE-ID) | 15.1.1 |
| 2018-06 | RP-80 | RP-181237 | 0011 | 6 | B | Introduction of SA NR (38.473 Baseline CR covering RAN3 agreements) | 15.2.0 |
| 2018-06 | RP-80 | RP-181239 | 0043 | 3 | F | Essential corrections of EN-DC for NSA NR (38.473 Baseline CR covering RAN3 agreements) | 15.2.0 |
| 2018-06 | RP-80 | RP-181237 | 0045 | - | B | F1 support for LTE - NR coexistence | 15.2.0 |
| 2018-06 | RP-80 |  |  |  |  | Correction to ASN.1 and to Change History table | 15.2.1 |
| 2018-09 | RP-81 | RP-181920 | 0055 | 2 | F | Introduction of DU Configuration Query | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0056 | 4 | F | CR to 38.473 on further clarifications on System information transfer over F1 | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0058 | 4 | F | CR to 38.473 on corrections to System information delivery | 15.3.0 |
| 2018-09 | RP-81 | RP-181920 | 0059 | 1 | F | CR to 38.473 on corrections to PWS transfer over F1 | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0063 | 3 | F | CR to 38.473 on PDCP SN over F1 interface | 15.3.0 |
| 2018-09 | RP-81 | RP-181922 | 0064 | 3 | F | NR Corrections (38.473 Baseline CR covering RAN3-101 agreements) | 15.3.0 |
| 2018-09 | RP-81 | RP-181997 | 0068 | - | F | Introduction of UL AMBR on F1 | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0072 | 3 | F | Correction on cell management | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0073 | 2 | F | RLC Mode Indication over F1 | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0076 | 3 | F | CR to 38.473 on UE Identity Index value | 15.3.0 |
| 2018-09 | RP-81 | RP-181920 | 0077 | 1 | F | Correction for UE Context Modification on presence of ServCellIndex IE | 15.3.0 |
| 2018-09 | RP-81 | RP-181920 | 0078 | - | F | Executing duplication for RRC-container | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0079 | 1 | F | Indication of RLC re-establishment at the gNB-DU | 15.3.0 |
| 2018-09 | RP-81 | RP-181920 | 0080 | - | F | Exchange of SMTC over F1 | 15.3.0 |
| 2018-09 | RP-81 | RP-181920 | 0081 | - | F | Solving remaining issues with QoS parameters – TS 38.473 | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0090 |  | F | Correction of 5GS TAC | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0095 | 1 | F | Extend the RANAC size to 8bits | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0097 | - | F | Corrections of Choice | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0098 | 1 | F | Correction of TNL criticality | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0099 | 1 | F | Corrections of usage of single container | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0105 | 2 | B | RRC version handling | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0106 | 1 | B | Introduction of Overload Handling in F1-C | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0113 | - | F | CR to 38.473 on presence of QoS information | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0114 | 1 | F | Correction C-RNTI format | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0115 | - | F | Correction of QoS Parameters | 15.3.0 |
| 2018-09 | RP-81 | RP-181921 | 0116 | 1 | F | Correction on F1 Setup Request | 15.3.0 |
| 2018-12 | RP-82 | RP-182446 | 0070 | 3 | F | RRC Delivery Indication | 15.4.0 |
| 2018-12 | RP-82 | RP-182446 | 0117 | 1 | F | Correction of AMBR Enforcement | 15.4.0 |
| 2018-12 | RP-82 | RP-182446 | 0138 | - | F | CR for correction on Initial UL RRC message transfer | 15.4.0 |
| 2018-12 | RP-82 | RP-182446 | 0140 | 1 | F | CR to 38.473 on bearer type change indication | 15.4.0 |
| 2018-12 | RP-82 | RP-182446 | 0142 | 1 | F | CR to 38.473 on correction to PWS System Information | 15.4.0 |
| 2018-12 | RP-82 | RP-182446 | 0144 | 2 | F | CR to 38.473 on asymmetric mapping for UL and DL QoS flow | 15.4.0 |
| 2018-12 | RP-82 | RP-182447 | 0145 | 4 | F | Corrections on UE-associated LTE/NR resource coordination | 15.4.0 |
| 2018-12 | RP-82 | RP-182446 | 0147 | 2 | F | CR for F1 Cell Management | 15.4.0 |
| 2018-12 | RP-82 | RP-182447 | 0150 | 1 | F | Missing Transaction ID in non-UE-associated procedures | 15.4.0 |
| 2018-12 | RP-82 | RP-182446 | 0157 | 1 | F | CR to 38.473 on mapping of servingCellMO and Serving Cell | 15.4.0 |
| 2018-12 | RP-82 | RP-182446 | 0160 | 1 | F | CR to 38.473 on UE context modification required procedure | 15.4.0 |
| 2018-12 | RP-82 | RP-182447 | 0165 | 1 | F | Addition of the RLC Mode information for bearer modification | 15.4.0 |
| 2018-12 | RP-82 | RP-182448 | 0167 | 2 | F | Rapporteur CR to align tabular | 15.4.0 |
| 2018-12 | RP-82 | RP-182448 | 0168 | 2 | F | Rapporteur CR to align ASN.1 | 15.4.0 |
| 2018-12 | RP-82 | RP-182447 | 0169 | 2 | F | Correction of MaxnoofBPLMNs | 15.4.0 |
| 2018-12 | RP-82 | RP-182351 | 0174 | 2 | F | Correction on PDCP SN length on F1 | 15.4.0 |
| 2018-12 | RP-82 | RP-182447 | 0178 | 2 | F | CR for TS 38.473 for MR-DC coordination | 15.4.0 |
| 2018-12 | RP-82 | RP-182447 | 0179 | 2 | F | Support of system information update for active UE without CSS | 15.4.0 |
| 2018-12 | RP-82 | RP-182447 | 0187 | 1 | F | CR to 38.473 on clarification to the presence of UE AMBR | 15.4.0 |
| 2018-12 | RP-82 | RP-182506 | 0195 | 2 | F | CR on Scell release for RLC failure | 15.4.0 |
| 2018-12 | RP-82 | RP-182447 | 0205 | 1 | F | About bandcombinationindex and featureSetEntryIndex | 15.4.0 |
| 2018-12 | RP-82 | RP-182447 | 0211 | 1 | F | CR to 38.473 on DRB PDCP duplication | 15.4.0 |
| 2018-12 | RP-82 | RP-182447 | 0216 | 1 | F | CR to 38.473 on clarifications on system information update over F1 | 15.4.0 |
| 2018-12 | RP-82 | RP-182448 | 0219 | - | F | Correction of RRC version handling and UE inactivity notification | 15.4.0 |
| 2019-01 | RP-82 |  |  |  |  | - correction to ASN.1: addiming a missing change to "WriteReplaceWarningResponseIEs F1AP-PROTOCOL-IES ::= {" | 15.4.1 |
| 2019-03 | RP-83 | RP-190555 | 0202 | 2 | F | Indication that cells are only UL or DL on F1 | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0204 | 1 | F | AMF intitiated UE Context Release failure cause | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0220 | 1 | F | Correction to reconfiguration with sync for gNB-DU | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0225 | 1 | F | Introduction of PH-InforSCG in DU to CU RRC Information | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0226 | 1 | F | CR to 38.473 on Measurement gap coordination | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0228 | 1 | F | CR for TS 38.473 for MR-DC coordination | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0229 | 2 | F | Condition for inclusion of the Dedicated SI Delivery Needed UE List IE | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0230 | 1 | F | Correction of the Transmission stop/restart indication | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0231 | - | F | Corrections on gNB-CU/gNB-DU Configuration Update | 15.5.0 |
| 2019-03 | RP-83 | RP-190556 | 0236 | 2 | F | Correction of QoS Flow Mapping Indication | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0244 | - | F | Release due to pre-emption | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0245 | 2 | F | CR on RRC container in UE context modification request message | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0246 | 2 | F | CR on UE context modification refuse | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0247 | - | F | Transaction ID in Error Indication procedure | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0249 | 2 | F | Cells to be deactivated over F1 | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0251 | 1 | F | CR to 38.473 on SRB duplication and LCID | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0258 | - | F | CR to 38.473 on corrections for removal of PDCP duplication for SRB | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0263 | 1 | F | CR to 38.473 on transfering UEAssistanceInformation over F1 | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0265 | - | F | Rapporteur updates | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0266 | 1 | F | Correction on gNB-DU Resource Coordination | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0267 | 1 | F | Endpoint IP address and port | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0268 | 1 | F | Correction to add paging origin IE | 15.5.0 |
| 2019-03 | RP-83 | RP-190555 | 0269 | 2 | F | Multiple SCTP associations over F1AP | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0272 | 1 | F | About Cells Failed to be Activated IE in gNB-CU Configuration Update Ack | 15.5.0 |
| 2019-03 | RP-83 | RP-190556 | 0273 | 1 | F | gNB-DU UE Aggregate Maximum Bit Rate Uplink correction | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0276 | 1 | F | RRC Reconfiguration failure | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0278 | 1 | F | Node behaviour at reception of DU to CU RRC Information | 15.5.0 |
| 2019-03 | RP-83 | RP-190554 | 0281 | - | F | Addition of Transaction ID to Initial UL RRC Message Transfer | 15.5.0 |
| 2019-07 | RP-84 | RP-191397 | 0200 | 5 | F | RAN sharing with multiple Cell ID broadcast | 15.6.0 |
| 2019-07 | RP-84 | RP-191397 | 0270 | 5 | F | Addition of Network Access Rate Reduction message | 15.6.0 |
| 2019-07 | RP-84 | RP-191397 | 0271 | 3 | F | RAN UE ID for F1 | 15.6.0 |
| 2019-07 | RP-84 | RP-191396 | 0283 | 2 | F | MR-DC resource coordination in F1 | 15.6.0 |
| 2019-07 | RP-84 | RP-191396 | 0316 | 2 | F | Full configuration indication from gNB-CU to gNB-DU. | 15.6.0 |
| 2019-07 | RP-84 | RP-191396 | 0322 | 2 | F | CR to 38.473 on clarification to RRC reconfigure complete indicator | 15.6.0 |
| 2019-07 | RP-84 | RP-191394 | 0326 | 2 | F | CR to 38.473 on deconfiguring CA based PDCP duplication for DRB | 15.6.0 |
| 2019-07 | RP-84 | RP-191395 | 0330 | 3 | F | CR to 38.473 on Removal of Multiple TNLAs | 15.6.0 |
| 2019-07 | RP-84 | RP-191396 | 0348 | - | F | Full configuration in UE Context Setup | 15.6.0 |
| 2019-07 | RP-84 | RP-191396 | 0351 | 2 | F | CR on PWS segmentation over F1 | 15.6.0 |
| 2019-07 | RP-84 | RP-191396 | 0352 | 1 | F | CR on cell type over F1 | 15.6.0 |
| 2019-07 | RP-84 | RP-191396 | 0357 | - | F | Rapporteur updates: Alignment and editorials | 15.5.0 |
| 2019-07 | RP-84 | RP-191396 | 0358 | - | F | Rapporteur update: Correction of Presence for DRB information | 15.6.0 |
| 2019-07 | RP-84 | RP-191396 | 0359 | - | F | Rapporteur updates: Correction of Presence for E-UTRA PRACH Configuration | 15.6.0 |
| 2019-07 | RP-84 | RP-191396 | 0370 | - | F | Full configuration IE included in the UE Context Modification Response. | 15.6.0 |
| 2019-07 | RP-84 | RP-191396 | 0376 |  | F | CR to 38.473 on clarification for UP TNL Information IE over F1 | 15.6.0 |
| 2019-07 | RP-84 | RP-191396 | 0377 | 2 | F | Procedure description on optional IEs in CU to DU RRC information IE. | 15.6.0 |
| 2019-09 | RP-85 | RP-192166 | 0343 | 3 | F | CR on MR-DC low layer coordination with an MgNB-DU | 15.7.0 |
| 2019-09 | RP-85 | RP-192166 | 0344 | 2 | F | CR on MCG PHR format in MgNB-DU | 15.7.0 |
| 2019-09 | RP-85 | RP-192166 | 0388 |  | F | CR on DC Coordination for PDCCH Blind Detection | 15.7.0 |
| 2019-09 | RP-85 | RP-192167 | 0393 | 1 | F | Rapporteur update - clarification of semantics | 15.7.0 |
| 2019-09 | RP-85 | RP-192166 | 0399 | 1 | F | Clarification for TNLA removal | 15.7.0 |
| 2019-12 | RP-86 | RP-192915 | 0318 | 5 | F | Correction about gNB-CU System Information IE | 15.8.0 |
| 2019-12 | RP-86 | RP-192915 | 0447 | 1 | F | On CellGroupConfig handling | 15.8.0 |
| 2019-12 | RP-86 | RP-192915 | 0458 | 1 | F | Correction of S-NSSAI coding | 15.8.0 |
| 2019-12 | RP-86 | RP-192915 | 0459 | 1 | F | Removal of Requested P-MaxFR2 | 15.8.0 |
| 2019-12 | RP-86 | RP-192915 | 0479 | 2 | F | Addition of Message Identifier and Serial Number to PWS Cancel Request | 15.8.0 |
| 2019-12 | RP-86 | RP-192916 | 0482 | 2 | F | Clarifications on SCell lists | 15.8.0 |
| 2019-12 | RP-86 | RP-192916 | 0494 | - | F | RRC Container in Modification Procedure | 15.8.0 |
| 2019-12 | RP-86 | RP-192916 | 0508 | 0 | F | CR to 38.473 on applicability of the IE Selected BandCombinationIndex and Selected FeatureSetEntryIndex | 15.8.0 |
| 2019-12 | RP-86 | RP-192916 | 0509 | 1 | F | CR to 38.473 on MeasGapSharingConfig and gNB-CU System Information | 15.8.0 |
| 2019-12 | RP-86 | RP-192916 | 0510 | 1 | F | CR to 38.473 on cause values over F1 | 15.8.0 |
| 2019-12 | RP-86 | RP-192916 | 0515 | 2 | F | Clarification on Initial UL RRC Message Transfer procedure | 15.8.0 |
| 2019-12 | RP-86 | RP-192913 | 0280 | 7 | F | Trace function support for F1AP | 16.0.0 |
| 2019-12 | RP-86 | RP-192908 | 0287 | 7 | B | Support for CLI | 16.0.0 |
| 2019-12 | RP-86 | RP-192913 | 0314 | 5 | B | Introduction of Additional RRM Policy Index (ARPI) | 16.0.0 |
| 2019-12 | RP-86 | RP-192908 | 0339 | 6 | B | CR to F1-AP for RIM new message | 16.0.0 |
| 2019-12 | RP-86 | RP-192915 | 0460 |  | F | Removal of unused IEs | 16.0.0 |
| 2019-12 | RP-86 | RP-192913 | 0463 | 1 | C | Extending the MDBV Range | 16.0.0 |
| 2019-12 | RP-86 | RP-192910 | 0514 | 3 | B | CR for TS38.473 on supporting SN Resume during the RRCResume procedure | 16.0.0 |
| 2019-12 | RP-86 | RP-192914 | 0518 | 2 | F | Support for setting up IPSec a priori in F1 | 16.0.0 |
| 2020-03 | RP-87-e | RP-200428 | 0522 | 1 | A | Correction of PWS Failure Indication | 16.1.0 |
| 2020-03 | RP-87-e | RP-200428 | 0525 | - | A | Correction of the presence of UL UP TNL Information to be setup List IE in tabular | 16.1.0 |
| 2020-03 | RP-87-e | RP-200425 | 0527 | 2 | F | Corrections to CLI | 16.1.0 |
| 2020-03 | RP-87-e | RP-200425 | 0528 | 1 | D | Rapporteur: Editorial updates | 16.1.0 |
| 2020-03 | RP-87-e | RP-200425 | 0530 | 2 | B | E2E delay measurement for Qos monitoring for URLLC | 16.1.0 |
| 2020-03 | RP-87-e | RP-200428 | 0534 | 1 | A | Correction relating to Initial UL RRC Message Transfer procedure CR 38.473 | 16.1.0 |
| 2020-07 | RP-88-e | RP-201077 | 0285 | 17 | B | BL CR to 38.473: Support for IAB | 16.2.0 |
| 2020-07 | RP-88-e | RP-201074 | 0432 | 12 | B | Support of NR V2X over F1 | 16.2.0 |
| 2020-07 | RP-88-e | RP-201082 | 0441 | 12 | B | Addition of SON features | 16.2.0 |
| 2020-07 | RP-88-e | RP-201079 | 0477 | 8 | B | Introduction of NR_IIOT support to TS 38.473 | 16.2.0 |
| 2020-07 | RP-88-e | RP-201075 | 0481 | 10 | B | Baseline CR for introducing Rel-16 NR mobility enhancement | 16.2.0 |
| 2020-07 | RP-88-e | RP-201082 | 0492 | 6 | B | Addition of MDT features | 16.2.0 |
| 2020-07 | RP-88-e | RP-201080 | 0502 | 7 | B | Introduction of NPN | 16.2.0 |
| 2020-07 | RP-88-e | RP-201076 | 0537 | 1 | B | CR38.473 on TDD pattern for NR-DC power control cordination for sol1 | 16.2.0 |
| 2020-07 | RP-88-e | RP-201085 | 0539 | - | F | Rapporteur: Corrections after implementation | 16.2.0 |
| 2020-07 | RP-88-e | RP-201090 | 0543 | 2 | A | Encoding PLMNs in served cell information NR | 16.2.0 |
| 2020-07 | RP-88-e | RP-201091 | 0545 | 1 | A | Correction for usage of Cell Broadcast Cancelled List | 16.2.0 |
| 2020-07 | RP-88-e | RP-201091 | 0548 | 1 | A | Correction on UE CONTEXT MODIFICATION REQUIRED message | 16.2.0 |
| 2020-07 | RP-88-e | RP-201085 | 0561 | 1 | F | Correction on CLI | 16.2.0 |
| 2020-07 | RP-88-e | RP-201090 | 0567 | - | A | Encoding PLMNs in served cell information IEs - semantics corrections | 16.2.0 |
| 2020-07 | RP-88-e | RP-201092 | 0570 | 1 | A | Correction for UL UP TNL Information | 16.2.0 |
| 2020-07 | RP-88-e | RP-201092 | 0572 | - | A | Correction on RRC Container in Initial UL RRC Messag Transfer | 16.2.0 |
| 2020-07 | RP-88-e | RP-201092 | 0576 | 1 | A | Correction on RRC Connection Reconfiguration Complete Indicator | 16.2.0 |
| 2020-07 | RP-88-e | RP-201092 | 0581 | 2 | F | Corrections of Inactive UE Context stored at gNB-DU | 16.2.0 |
| 2020-07 | RP-88-e | RP-201085 | 0600 | 2 | F | Correction on RF parameters in NR cell information | 16.2.0 |
| 2020-07 | RP-88-e | RP-201090 | 0601 | 4 | F | Correction of S-NSSAI range | 16.2.0 |
| 2020-07 | RP-88-e | RP-201092 | 0603 | 2 | A | Correction for Handover Preparation Information | 16.2.0 |
| 2020-07 | RP-88-e | RP-201092 | 0607 | 1 | A | CR on Concurrent Warning Message Indicator over F1 (Rel-16) | 16.2.0 |
| 2020-07 | RP-88-e | RP-201092 | 0615 | - | A | Section renumbering for PWS cancel | 16.2.0 |
| 2020-07 | RP-88-e | RP-201092 | 0616 | - | A | Correction on DL RRC MESSAGE TRANSFER | 16.2.0 |
| 2020-07 | RP-88-e | RP-201092 | 0618 |  | A | Addition of abnormal conditions in PWS Cancel procedure | 16.2.0 |
| 2020-09 | RP-89-e | RP-201850 | 0495 | 10 | B | Introduction of positioning support over F1AP | 16.3.0 |
| 2020-09 | RP-89-e | RP-201956 | 0557 | 2 | A | Support of PSCell/SCell-only operation mode | 16.3.0 |
| 2020-09 | RP-89-e | RP-201956 | 0583 | 5 | F | Cell Creation Rejection when max number of supported cells is exceeded at CU CR 38.473 | 16.3.0 |
| 2020-09 | RP-89-e | RP-201956 | 0587 | 5 | A | Measurement gap deactivation over F1AP CR 38.473 | 16.3.0 |
| 2020-09 | RP-89-e | RP-201949 | 0619 | 2 | F | Slot list length correction in TDD UL-DL Configuration | 16.3.0 |
| 2020-09 | RP-89-e | RP-201956 | 0625 | 1 | F | Addition of abnormal conditions in Write-Replace Warning procedure | 16.3.0 |
| 2020-09 | RP-89-e | RP-201956 | 0628 | 2 | A | Correction of PSCell/SCell-only mode | 16.3.0 |
| 2020-09 | RP-89-e | RP-201956 | 0634 | 1 | A | Correction on UE Context Modification Procedure | 16.3.0 |
| 2020-09 | RP-89-e | RP-201956 | 0639 | 1 | F | Rapporteur Corrections | 16.3.0 |
| 2020-09 | RP-89-e | RP-201949 | 0640 | - | F | Correction of procedure ID | 16.3.0 |
| 2020-09 | RP-89-e | RP-201956 | 0642 | - | A | Correction of PWS cancel | 16.3.0 |
| 2020-09 | RP-89-e | RP-201949 | 0643 | 1 | F | Corrections on PC5 Link Aggregated Bit Rate | 16.3.0 |
| 2020-09 | RP-89-e | RP-201949 | 0660 | - | F | Correction on the Maximum Number of CHO Preparations in F1AP | 16.3.0 |
| 2020-09 | RP-89-e | RP-201956 | 0663 | 1 | F | Corrections to 38.473 on node name type | 16.3.0 |
| 2020-09 | RP-89-e | RP-201947 | 0664 | 1 | F | Correction on IAB-DU configuration | 16.3.0 |
| 2020-09 | RP-89-e | RP-201982 | 0671 |  | F | Correction on IAB-DU configuration | 16.3.0 |
| 2020-09 | RP-89-e |  |  |  |  | Correct wrong numbering of protocolIE-ID in clause 9.4.7 | 16.3.1 |
| 2020-12 | RP-90-e | RP-202310 | 0645 | 2 | F | Uniqueness of BH RLC channel ID | 16.4.0 |
| 2020-12 | RP-90-e | RP-202310 | 0658 | 3 | F | Correction on V2X related information | 16.4.0 |
| 2020-12 | RP-90-e | RP-202310 | 0665 | 2 | F | Correction on unsuccessful operations of IAB procedures | 16.4.0 |
| 2020-12 | RP-90-e | RP-202310 | 0666 | 1 | F | Correction on the identification of IAB-donor-DU | 16.4.0 |
| 2020-12 | RP-90-e | RP-202310 | 0667 | 2 | F | Correction on the Context Setup procedure for IAB node | 16.4.0 |
| 2020-12 | RP-90-e | RP-202310 | 0668 | 1 | F | Correction on BAP address | 16.4.0 |
| 2020-12 | RP-90-e | RP-202310 | 0672 | 1 | F | CR on F1-C transfer for Rel-16 IAB | 16.4.0 |
| 2020-12 | RP-90-e | RP-202311 | 0677 | - | F | Correction of F1AP positioning procedures | 16.4.0 |
| 2020-12 | RP-90-e | RP-202311 | 0678 | 1 | F | Corrections to tabular and asn.1 for NR positioning (F1AP) | 16.4.0 |
| 2020-12 | RP-90-e | RP-202310 | 0681 | 1 | F | Correction of alternative QoS profile | 16.4.0 |
| 2020-12 | RP-90-e | RP-202313 | 0683 | - | F | Removal of duplicated imports | 16.4.0 |
| 2020-12 | RP-90-e | RP-202312 | 0684 | 2 | F | Corrections of UL and DL carrier list | 16.4.0 |
| 2020-12 | RP-90-e | RP-202311 | 0689 | 1 | F | RRC alignement and various correction including ASN.1 | 16.4.0 |
| 2020-12 | RP-90-e | RP-202311 | 0691 | 1 | F | Correction of RLC Duplication Information over F1 | 16.4.0 |
| 2020-12 | RP-90-e | RP-202288 | 0695 | 3 | A | Correction on value range of UAC reduction Indication | 16.4.0 |
| 2020-12 | RP-90-e | RP-202311 | 0709 | 1 | F | Coupling TRP ID and Cell ID in Measurement procedures | 16.4.0 |
| 2021-03 | RP-91-e | RP-210123 | 0431 | 7 | B | Introduction of SFN Offset per cell over F1 | 16.5.0 |
| 2021-03 | RP-91-e | RP-210240 | 0632 | 6 | A | Correction on Overlapping Band Handling over F1 | 16.5.0 |
| 2021-03 | RP-91-e | RP-210235 | 0676 | 2 | F | Correction on PRACH coordination | 16.5.0 |
| 2021-03 | RP-91-e | RP-210239 | 0702 | 3 | F | Cause value on F1 for insufficient UE capabilities CR 38.473 | 16.5.0 |
| 2021-03 | RP-91-e | RP-210239 | 0711 | 1 | F | Update on QoS monitoring control | 16.5.0 |
| 2021-03 | RP-91-e | RP-210233 | 0715 | 2 | F | Stage-3 CR on transmission stop for Rel-16 DAPS handover | 16.5.0 |
| 2021-03 | RP-91-e | RP-210232 | 0720 | 1 | F | Correction of NPN related Cell Information | 16.5.0 |
| 2021-03 | RP-91-e | RP-210231 | 0721 | - | F | Correction on IAB configuration | 16.5.0 |
| 2021-03 | RP-91-e | RP-210231 | 0722 | - | F | Correction on BAP address configuration for IAB-donor-DU | 16.5.0 |
| 2021-03 | RP-91-e | RP-210230 | 0725 | 1 | F | Including SRS frequency information in Positioning Information Request | 16.5.0 |
| 2021-03 | RP-91-e | RP-210231 | 0728 | 2 | F | CR to 38.473: Correction on IAB related definitions and unsuccessful establishment of a BH RLC channel | 16.5.0 |
| 2021-03 | RP-91-e | RP-210230 | 0736 | - | F | Correction of the PCI IE presence in the ASN.1 for the SRS Configuration | 16.5.0 |
| 2021-06 | RP-92-e | RP-211334 | 0704 | 4 | A | How to release SCG configuration between MN-CU and MN-DU CR 38.473 | 16.6.0 |
| 2021-06 | RP-92-e | RP-211315 | 0712 | 2 | F | Clarification on TAI Slice Support List | 16.6.0 |
| 2021-06 | RP-92-e | RP-211323 | 0740 | 2 | F | Enabling CHO with SCG configuration | 16.6.0 |
| 2021-06 | RP-92-e | RP-211327 | 0743 | - | F | Correction of Spatial Relation Information | 16.6.0 |
| 2021-06 | RP-92-e | RP-211317 | 0744 | - | F | Correction on reference to RACH-Report | 16.6.0 |
| 2021-06 | RP-92-e | RP-211330 | 0753 |  | F | Stage-3 CR on system information message over F1 (Rel-16) | 16.6.0 |
| 2021-06 | RP-92-e | RP-211333 | 0760 | - | A | Correction on SRB ID | 16.6.0 |
| 2021-06 | RP-92-e | RP-211334 | 0762 | 3 | A | gNB-DU UE Aggregate Maximum Bit Rate Uplink correction | 16.6.0 |
| 2021-06 | RP-92-e | RP-211322 | 0763 | - | F | Miscellaneous corrections on IAB in TS 38.473 | 16.6.0 |
| 2021-06 | RP-92-e | RP-211327 | 0765 | 1 | F | Correction on SFN Initialisation Time | 16.6.0 |
| 2021-06 | RP-92-e | RP-211327 | 0766 | - | F | Correction on relative cartesian coordinate | 16.6.0 |
| 2021-06 | RP-92-e | RP-211322 | 0770 | 1 | F | Correction on BH RLC CH configured for BAP control PDU | 16.6.0 |
| 2021-06 | RP-92-e | RP-211322 | 0771 | - | F | Correction on gNB-DU Resource Configuration | 16.6.0 |
| 2021-06 | RP-92-e | RP-211322 | 0772 | 1 | F | Correction on UL BH information configuration for DRBs support CA based duplication | 16.6.0 |
| 2021-06 | RP-92-e | RP-211317 | 0776 | 1 | F | Correction on MLB for TS 38.473 | 16.6.0 |
| 2021-09 | RP-93-e | RP-211876 | 0790 | 1 | F | Correction of served cell information for NPN | 16.7.0 |
| 2021-09 | RP-93-e | RP-211880 | 0792 | 1 | F | Correction of wrong CR implementation for Stage-3 CR on transmission stop for Rel-16 DAPS handover | 16.7.0 |
| 2021-09 | RP-93-e | RP-211883 | 0796 | 1 | F | Adding procedural text for System Frame Number and Slot Number | 16.7.0 |
| 2021-09 | RP-93-e | RP-211881 | 0800 | - | A | Correction of the IE related to E-UTRA resource coordination in F1AP | 16.7.0 |
| 2021-12 | RP-94-e | RP-212864 | 0804 | 1 | A | Correction on F1 Removal for RAN Sharing in Rel-16 | 16.8.0 |
| 2021-12 | RP-94-e | RP-212864 | 0811 | 4 | F | Incorrect Node Name IE in ASN.1 | 16.8.0 |
| 2021-12 | RP-94-e | RP-213174 | 0822 | 3 | F | Correction on PRS-only TRP | 16.8.0 |
| 2021-12 | RP-94-e | RP-212867 | 0827 | 1 | F | Support of providing spatial relation per SRS resource from gNB-CU to gNB-DU | 16.8.0 |
| 2022-03 | RP-95-e | RP-220279 | 0778 | 4 | F | Support of dynamic ACL during dual connectivity | 16.9.0 |
| 2022-03 | RP-95-e | RP-220276 | 0837 | 1 | F | Correction on packet delay budget for IAB access link in TS 38.473 | 16.9.0 |
| 2022-03 | RP-95-e | RP-220276 | 0838 | - | F | CR to 38.473: Correction on IAB TNL Address Allocation procedure | 16.9.0 |
| 2022-03 | RP-95-e | RP-220242 | 0844 | 2 | F | CR to TS38.473: Correction on PC5 QoS parameters for NR V2X | 16.9.0 |
| 2022-03 | RP-95-e | RP-220281 | 0847 | 1 | F | Correction on positioning information configuration | 16.9.0 |
| 2022-03 | RP-95-e | RP-220281 | 0848 | 1 | F | Correction on Measurement Periodicity | 16.9.0 |
| 2022-03 | RP-95-e | RP-220281 | 0849 | 1 | F | Correction on PRS Beam Information | 16.9.0 |
| 2022-03 | RP-95-e | RP-220281 | 0850 |  | F | CR for the correction on measurement gap configuration for position | 16.9.0 |
| 2022-03 | RP-95-e | RP-220278 | 0854 | 1 | F | Correction of frequency information for DL only or UL only cell | 16.9.0 |
| 2022-03 | RP-95-e | RP-220276 | 0860 |  | F | (Stage-3) Clarification on IAB Address Remove | 16.9.0 |
| 2022-03 | RP-95-e | RP-220221 | 0710 | 9 | B | Addition of SON features enhancement | 17.0.0 |
| 2022-03 | RP-95-e | RP-220224 | 0716 | 6 | B | Introduction of NR MBS | 17.0.0 |
| 2022-03 | RP-95-e | RP-220222 | 0737 | 13 | B | CP-based Congestion Indication for IAB Networks | 17.0.0 |
| 2022-03 | RP-95-e | RP-220221 | 0738 | 7 | B | BLCR to 38.473: Support of MDT enhancement | 17.0.0 |
| 2022-03 | RP-95-e | RP-220223 | 0751 | 7 | B | Introduction of Enhanced IIoT support over F1 | 17.0.0 |
| 2022-03 | RP-95-e | RP-220218 | 0777 | 10 | B | SCG BL CR to TS 38.473 | 17.0.0 |
| 2022-03 | RP-95-e | RP-220218 | 0795 | 5 | B | BLCR to TS 38.473 for Conditional PScell Change/Addition | 17.0.0 |
| 2022-03 | RP-95-e | RP-220228 | 0803 | 6 | B | Introduction of NR Positioning enhancements | 17.0.0 |
| 2022-03 | RP-95-e | RP-220230 | 0806 | 6 | B | BL CR to F1AP on Rel-17 RedCap | 17.0.0 |
| 2022-03 | RP-95-e | RP-220236 | 0817 | 2 | B | Addition of NR Timing Advance reporting for NR UL E-CID [NRTADV] | 17.0.0 |
| 2022-03 | RP-95-e | RP-220229 | 0826 | 7 | B | Support of QoE information transfer | 17.0.0 |
| 2022-03 | RP-95-e | RP-220233 | 0833 | 3 | B | CG-SDT BLCR to TS38.473 | 17.0.0 |
| 2022-03 | RP-95-e | RP-220233 | 0834 | 3 | B | Support of RACH-based SDT | 17.0.0 |
| 2022-03 | RP-95-e | RP-220231 | 0842 | 3 | B | Introduction of SideLink Relay | 17.0.0 |
| 2022-03 | RP-95-e | RP-220219 | 0852 | 4 | B | Introduction of MultiSIM support over F1 | 17.0.0 |
| 2022-03 | RP-95-e | RP-220235 | 0855 | 4 | B | Support for UE Power Saving Enhancements | 17.0.0 |
| 2022-03 | RP-95-e | RP-220232 | 0856 | 1 | B | (BLCR to TS 38.473) Supporting network slicing enhancement | 17.0.0 |
| 2022-03 | RP-95-e | RP-220236 | 0858 |  | D | Editorial corrections | 17.0.0 |
| 2022-06 | RP-96 | RP-221143 | 0862 | 2 | F | QoE Rel-17 Corrections | 17.1.0 |
| 2022-06 | RP-96 | RP-221132 | 0863 | - | F | Correction of PDC Measurement Periodicity values | 17.1.0 |
| 2022-06 | RP-96 | RP-221141 | 0864 | 1 | F | Correction of R17 SON features enhancement | 17.1.0 |
| 2022-06 | RP-96 | RP-221134 | 0865 | 1 | F | Corrections on NR MBS in F1AP | 17.1.0 |
| 2022-06 | RP-96 | RP-221134 | 0866 | - | F | NR MBS F1AP asn.1 correction | 17.1.0 |
| 2022-06 | RP-96 | RP-221136 | 0868 | 2 | F | Correction on CG based SDT | 17.1.0 |
| 2022-06 | RP-96 | RP-221150 | 0879 | 1 | A | F1AP CR for ACL remaining issues | 17.1.0 |
| 2022-06 | RP-96 | RP-221145 | 0880 | 4 | F | CR to 38.473 on Measurement Amount | 17.1.0 |
| 2022-06 | RP-96 | RP-221139 | 0884 | - | F | ASN.1 corrections on NR SL relay for 38.473 | 17.1.0 |
| 2022-06 | RP-96 | RP-221137 | 0887 | 2 | F | Correction to MUSIM | 17.1.0 |
| 2022-06 | RP-96 | RP-221139 | 0892 | 1 | F | Corrections for SL_relay (F1AP) | 17.1.0 |
| 2022-06 | RP-96 | RP-221126 | 0894 | 1 | F | Correction on RedCap Broadcast Information for TS38.473 | 17.1.0 |
| 2022-06 | RP-96 | RP-221131 | 0896 | 1 | F | F1AP ASN.1 review for NR Positioning Enhancements | 17.1.0 |
| 2022-06 | RP-96 | RP-221134 | 0897 |  | F | Remove the editor's notes | 17.1.0 |
| 2022-06 | RP-96 | RP-221136 | 0900 | 1 | F | Correction on SRB SDT indication | 17.1.0 |
| 2022-06 | RP-96 | RP-221136 | 0901 | 1 | F | Correction on SDT termination request in F1 | 17.1.0 |
| 2022-06 | RP-96 | RP-221141 | 0902 | 1 | F | Correction on SON feature enhancements - F1AP | 17.1.0 |
| 2022-06 | RP-96 | RP-221131 | 0905 | 3 | F | Positioning corrections for F1AP | 17.1.0 |
| 2022-06 | RP-96 | RP-221132 | 0908 | 2 | F | Correction of PDC Measurement Initiation Failure message | 17.1.0 |
| 2022-06 | RP-96 | RP-221805 | 0910 | 2 | F | Correction for IAB inter-donor DU re-routing and resource multiplexing | 17.1.0 |
| 2022-06 | RP-96 | RP-221149 | 0913 | 1 | A | Correction on IAB-DU cell resource configuration | 17.1.0 |
| 2022-06 | RP-96 | RP-221130 | 0916 | 2 | F | CR to TS38.473: Correction on PC5 DRX parameters for NR V2X | 17.1.0 |
| 2022-06 | RP-96 | RP-221139 | 0920 |  | F | Corrections on Remote UE Local ID | 17.1.0 |
| 2022-06 | RP-96 | RP-221141 | 0921 | 2 | F | F1AP corrections for NR-U | 17.1.0 |
| 2022-06 | RP-96 | RP-221152 | 0923 | 2 | A | Correction for PRS Muting | 17.1.0 |
| 2022-06 | RP-96 | RP-221132 | 0924 | 2 | F | NR-IIoT F1AP correction | 17.1.0 |
| 2022-06 | RP-96 | RP-221129 | 0927 | 2 | F | Supporting network slice AS group | 17.1.0 |
| 2022-06 | RP-96 | RP-221129 | 0928 | 1 | F | Correction of the presence of UE-Slice-MBR | 17.1.0 |
| 2022-06 | RP-96 | RP-221141 | 0929 | - | F | Corrections to Load Balancing Enhancements | 17.1.0 |
| 2022-06 | RP-96 | RP-221139 | 0930 | 2 | F | Corrections for SL relay | 17.1.0 |
| 2022-06 | RP-96 | RP-221145 | 0932 | 2 | D | Editorial corrections | 17.1.0 |
| 2022-06 | RP-96 | RP-221134 | 0938 | 1 | F | Correction on MBS features | 17.1.0 |
| 2022-06 | RP-96 | RP-221136 | 0939 | 1 | F | Correction on SDT in F1AP | 17.1.0 |
| 2022-06 | RP-96 | RP-221136 | 0940 | 1 | F | Correction on Rel-17 SDT (F1AP) | 17.1.0 |
| 2022-06 | RP-96 | RP-221141 | 0945 | 1 | F | Correction on update management based MDT user consent | 17.1.0 |
| 2022-06 | RP-96 | RP-221131 | 0948 | 2 | F | Support of multiple measurement instances | 17.1.0 |
| 2022-06 | RP-96 | RP-221145 | 0949 | 1 | F | Supporting the disaster roaming information [MINT] | 17.1.0 |
| 2022-06 | RP-96 | RP-221150 | 0951 | 2 | A | SIB Issues Rel-17 | 17.1.0 |
| 2022-06 | RP-96 | RP-221150 | 0953 | 1 | A | gNB-CU and gNB-DU Name in Configuration Update Procedures | 17.1.0 |
| 2022-06 | RP-96 | RP-221137 | 0957 | - | F | Clarification on the paging cause | 17.1.0 |
| 2022-06 | RP-96 | RP-221143 | 0958 | 1 | F | CR to 38.473 on ASN.1 corrections of QoE measurement | 17.1.0 |
| 2022-06 | RP-96 | RP-221152 | 0964 |  | A | ASN.1 correction for UL-AoA | 17.1.0 |
| 2022-06 | RP-96 | RP-221141 | 0965 | 1 | F | ASN.1 corrections | 17.1.0 |
| 2022-06 | RP-96 | RP-221141 | 0968 | 1 | F | CCO corrections | 17.1.0 |
| 2022-06 | RP-96 | RP-221131 | 0969 | - | F | Corrections to Measurement Pre-configuration Information Transfer | 17.1.0 |
| 2022-06 | RP-96 | RP-221628 | 0971 | - | F | Removal of UE Tx TEG Association from F1AP | 17.1.0 |
| 2022-09 | RP-97-e | RP-222201 | 0889 | 2 | A | Correction on interFrequencyConfig-NoGap | 17.2.0 |
| 2022-09 | RP-97-e | RP-222189 | 0890 | 2 | F | Correction to SDT for supporting delta signaling | 17.2.0 |
| 2022-09 | RP-97-e | RP-222188 | 0978 | - | F | Correction on Broadcast and Unicast co-existence | 17.2.0 |
| 2022-09 | RP-97-e | RP-222183 | 0981 | 1 | F | Miscellaneous Correction on IAB | 17.2.0 |
| 2022-09 | RP-97-e | RP-222188 | 0984 | 1 | F | Further Corrections for NR MBS | 17.2.0 |
| 2022-09 | RP-97-e | RP-222188 | 0985 | 2 | F | Corrections for the establishment of F1-U ptp retransmission tunnels | 17.2.0 |
| 2022-09 | RP-97-e | RP-222185 | 0986 | 3 | B | CR for TS38.473 on Extending NR Operation to 71GHz | 17.2.0 |
| 2022-09 | RP-97-e | RP-222186 | 0988 |  | F | Addition of SRS port index | 17.2.0 |
| 2022-09 | RP-97-e | RP-222190 | 0989 | 1 | F | SL relay corrections | 17.2.0 |
| 2022-09 | RP-97-e | RP-222187 | 0994 | 1 | F | Correction of UE Paging Capability | 17.2.0 |
| 2022-09 | RP-97-e | RP-222201 | 0997 | 1 | F | Correction on measurement gap configuration over F1 in Rel-17 | 17.2.0 |
| 2022-09 | RP-97-e | RP-222186 | 0998 | 2 | F | Support of timing error margins for TEGs in F1AP | 17.2.0 |
| 2022-09 | RP-97-e | RP-222188 | 1000 | 1 | F | Introduction of MBS specific cause values | 17.2.0 |
| 2022-09 | RP-97-e | RP-222188 | 1001 | 1 | F | Correction on Multicast Group Paging | 17.2.0 |
| 2022-09 | RP-97-e | RP-222188 | 1002 | 1 | F | Correction on MRB ID Change | 17.2.0 |
| 2022-09 | RP-97-e | RP-222603 | 1004 | 4 | A | CR to 38.473 on E-CID measurement periodicity | 17.2.0 |
| 2022-09 | RP-97-e | RP-222088 | 1013 | 3 | F | Correction on RedCap paging capability | 17.2.0 |
| 2022-09 | RP-97-e | RP-222186 | 1016 | 1 | F | Rel-17 ePos correction for the missing support of SRS-PosRRC-InactiveConfig-r17 configuration | 17.2.0 |
| 2022-09 | RP-97-e | RP-222189 | 1017 | 1 | F | Transferring CG-SDT configuration and SRS positioning Inactive configuration from DU to CU | 17.2.0 |
| 2022-09 | RP-97-e | RP-222088 | 1019 | 1 | F | Correction of the maximum PTW length of IDLE eDRX | 17.2.0 |
| 2022-09 | RP-97-e | RP-222189 | 1021 | 1 | F | Correction on Rel-17 SDT | 17.2.0 |
| 2022-09 | RP-97-e | RP-222638 | 1025 | 3 | F | Introduction of uplink GapFR2 [NR_RF_FR2_req_enh2-Core] | 17.2.0 |
| 2022-09 | RP-97-e | RP-222186 | 1027 | 2 | F | Correction to positioning gap configuration | 17.2.0 |
| 2022-09 | RP-97-e | RP-222191 | 1031 | - | F | Correction to Report Caracteristics | 17.2.0 |
| 2022-09 | RP-97-e | RP-222186 | 1034 | - | F | Correction on Measurement Time Occasion | 17.2.0 |
| 2022-09 | RP-97-e | RP-222191 | 1035 | 1 | F | Correction on NR-U MLB | 17.2.0 |
| 2022-12 | RP-98-e | RP-222883 | 0975 | 4 | A | R17CR for DAPS over F1 to TS38.473 | 17.3.0 |
| 2022-12 | RP-98-e | RP-222884 | 1012 | 1 | A | Correction of on-demand SI for connected UE | 17.3.0 |
| 2022-12 | RP-98-e | RP-222879 | 1039 | 1 | F | Further correction to Report Caracteristics | 17.3.0 |
| 2022-12 | RP-98-e | RP-222882 | 1043 | 3 | F | Provision of MBS Multicast F1-U references to UE Context in gNB-CU enabling retrieval of data forwarding progress information | 17.3.0 |
| 2022-12 | RP-98-e | RP-222886 | 1046 | 2 | F | Correction of TRP TEG | 17.3.0 |
| 2022-12 | RP-98-e | RP-222884 | 1052 | 2 | A | Correction on generation of gap type over F1 in Rel-17 | 17.3.0 |
| 2022-12 | RP-98-e | RP-222886 | 1054 | 1 | F | Correction of Timing Error Margin | 17.3.0 |
| 2022-12 | RP-98-e | RP-222887 | 1057 | 1 | A | CR to 38.473 on SRS periodicity | 17.3.0 |
| 2022-12 | RP-98-e | RP-222886 | 1058 | 1 | F | Correction of ASN.1 for UL RTOA Measurement | 17.3.0 |
| 2022-12 | RP-98-e | RP-222887 | 1060 | 3 | A | Correction on positioning SI delivery over F1AP | 17.3.0 |
| 2022-12 | RP-98-e | RP-222881 | 1061 | 1 | F | Correction on resource configuration for IAB | 17.3.0 |
| 2022-12 | RP-98-e | RP-222886 | 1072 | - | F | Correction on presence of timing error margin for TRP TEGs | 17.3.0 |
| 2022-12 | RP-98-e | RP-222883 | 1075 | 1 | A | CR for DAPS state transfer in case of split gNB deployment to Rel-17 38.473 | 17.3.0 |
| 2022-12 | RP-98-e | RP-222888 | 1076 | 1 | F | SL relay corrections | 17.3.0 |
| 2022-12 | RP-98-e | RP-222888 | 1077 | 4 | F | Correction to 38.473 for SL relay (R17) | 17.3.0 |
| 2022-12 | RP-98-e | RP-222957 | 1081 | 5 | A | Support of DC Location for two UL CCs in Split architecture | 17.3.0 |
| 2022-12 | RP-98-e | RP-222882 | 1083 | 1 | F | Correction on NR MBS over F1AP | 17.3.0 |
| 2022-12 | RP-98-e | RP-222882 | 1084 | 1 | F | Correction on MRB QoS Information | 17.3.0 |
| 2022-12 | RP-98-e | RP-222879 | 1091 | 2 | F | Correction on Resource Status Reporting procedure over F1 | 17.3.0 |
| 2022-12 | RP-98-e | RP-222885 | 1094 | 2 | F | Support of DC Location for more carriers in Split architecture (The CR is not implemented. The CR is postponed to next plenary meeting due to the fact that it is impossible to modify a paragraph via 2 separate CRs without CR clash) | 17.3.0 |
| 2022-12 | RP-98-e | RP-222886 | 1100 | 1 | F | Support of DC Location for more carriers in Split architecture | 17.3.0 |
| 2023-03 | RAN#99 | RP-230591 | 1029 | 4 | B | Introduction of two PHR mode [NR_feMIMO-Core] | 17.4.0 |
| 2023-03 | RAN#99 | RP-230582 | 1090 | 4 | F | Correction to conditional MCG configuration in CPAC | 17.4.0 |
| 2023-03 | RAN#99 | RP-230589 | 1093 | 4 | F | Correction to support NCD-SSB RedCap requirements in F1AP | 17.4.0 |
| 2023-03 | RAN#99 | RP-230593 | 1094 | 3 | F | Correction for supporting DC Location for more carriers in Split architecture | 17.4.0 |
| 2023-03 | RAN#99 | RP-230585 | 1097 | 2 | F | Correction of NR PRACH Configuration List for FR2-2 | 17.4.0 |
| 2023-03 | RAN#99 | RP-230581 | 1113 | 1 | F | Correction on missing cause value for CG-SDT | 17.4.0 |
| 2023-03 | RAN#99 | RP-230589 | 1114 | 2 | F | Correction on the UE identity index for paging RedCap UE to TS38.473 | 17.4.0 |
| 2023-03 | RAN#99 | RP-230596 | 1116 | - | A | Correction on IAB UP configuration update | 17.4.0 |
| 2023-03 | RAN#99 | RP-230583 | 1117 | 1 | F | Correction on NR MBS Broadcast aspects | 17.4.0 |
| 2023-03 | RAN#99 | RP-230587 | 1118 | 1 | F | Correction of gNB Rx-Tx Time Difference | 17.4.0 |
| 2023-03 | RAN#99 | RP-230590 | 1120 | 1 | F | Correction of RRC references for SLrelay | 17.4.0 |
| 2023-03 | RAN#99 | RP-230593 | 1121 | - | F | Correction of RRC references for DRX | 17.4.0 |
| 2023-03 | RAN#99 | RP-230593 | 1122 | - | F | Correction of RRC references | 17.4.0 |
| 2023-03 | RAN#99 | RP-230591 | 1124 | 2 | B | Missing transmission bandwidth configurations in F1AP | 17.4.0 |
| 2023-03 | RAN#99 | RP-230583 | 1125 | 1 | F | Correction of F1 Broadcast Setup | 17.4.0 |
| 2023-03 | RAN#99 | RP-230588 | 1126 | - | F | PRS configuration procedure correction | 17.4.0 |
| 2023-03 | RAN#99 | RP-230581 | 1127 | 1 | F | Correction to CellGroupConfig handling for SDT | 17.4.0 |
| 2023-03 | RAN#99 | RP-230584 | 1131 | 2 | F | Correction on NR-U Channel ID | 17.4.0 |
| 2023-03 | RAN#99 | RP-230595 | 1137 | 2 | A | ASN.1 Correction of M6 Configuration | 17.4.0 |
| 2023-03 | RAN#99 | RP-230586 | 1138 | 1 | F | Correction to TS 38.473 on Re-routing Enable Indicator | 17.4.0 |
| 2023-04 | RAN#99 | - | - | - | - | Editorial Changes(font, style, line break) | 17.4.1 |
| 2023-06 | RAN#100 | RP-231073 | 1119 | 2 | F | Correction of Burst Arrival Time semantics description | 17.5.0 |
| 2023-06 | RAN#100 | RP-231075 | 1135 | 3 | A | Correction of SIType List | 17.5.0 |
| 2023-06 | RAN#100 | RP-231075 | 1144 | 2 | A | Corrections on TNL association addition, update and removal (F1AP) | 17.5.0 |
| 2023-06 | RAN#100 | RP-231072 | 1145 | 2 | F | Correction to TS 38.473 on RB Set Configuration | 17.5.0 |
| 2023-06 | RAN#100 | RP-231079 | 1146 | 2 | F | Introduction of the UE hashed ID to 38.473 | 17.5.0 |
| 2023-06 | RAN#100 | RP-231074 | 1147 | 2 | F | Correction on Broadcast Partial Success | 17.5.0 |
| 2023-06 | RAN#100 | RP-231081 | 1150 | 3 | A | ASN.1 Correction of PRACH Configuration | 17.5.0 |
| 2023-06 | RAN#100 | RP-231071 | 1158 | 2 | F | F1AP Rel-17 correction for NR-U metrics | 17.5.0 |
| 2023-06 | RAN#100 | RP-231080 | 1159 | 2 | F | Correction on F1AP for L2 U2N Relay | 17.5.0 |
| 2023-06 | RAN#100 | RP-231084 | 1162 | 2 | F | Correction of Extended Packet Delay Budget | 17.5.0 |
| 2023-06 | RAN#100 | RP-231075 | 1165 | 2 | A | Correction on E-UTRA - NR Cell Resource Coordination | 17.5.0 |
| 2023-06 | RAN#100 | RP-231074 | 1166 | 2 | F | Transfer of MBSInterestIndication from CU to DU | 17.5.0 |
| 2023-06 | RAN#100 | RP-231074 | 1170 | 1 | F | Correction of MRB Setup | 17.5.0 |
| 2023-06 | RAN#100 | RP-231079 | 1172 | 0 | F | Correction of RedCap-specific initial DL BWP without CD-SSB for SDT | 17.5.0 |
| 2023-06 | RAN#100 | RP-231077 | 1177 | 1 | F | SRS Resource correction on Comb 8, Number of Symbols and Repetition Factor | 17.5.0 |
| 2023-06 | RAN#100 | RP-231077 | 1179 | 1 | F | Subcarrier Spacing correction | 17.5.0 |
| 2023-09 | RAN#101 | RP-231896 | 1175 | 3 | A | Correction on IAB bar configuration | 17.6.0 |
| 2023-09 | RAN#101 | RP-231897 | 1191 | 1 | F | Correction of Distribution procedure | 17.6.0 |
| 2023-09 | RAN#101 | RP-231901 | 1193 | 1 | F | Mapping of SRB1 for the remote UE | 17.6.0 |
| 2023-09 | RAN#101 | RP-231899 | 1197 | - | A | Correction to TS 38.473 on inter-node message for CU-DU split scenario | 17.6.0 |
| 2023-09 | RAN#101 | RP-231896 | 1203 | - | A | Configuration of BH information for DRBs support CA based duplication | 17.6.0 |
| 2023-09 | RAN#101 | RP-231897 | 1205 | 1 | F | Correction on condition of successful MBS Broadcast Context Setup | 17.6.0 |
| 2023-09 | RAN#101 | RP-231900 | 1206 | 1 | F | PRS CONFIGURATION REQUEST Correction | 17.6.0 |
| 2023-09 | RAN#101 | RP-231900 | 1211 | - | A | Correction of Positioning SIType List | 17.6.0 |
| 2023-09 | RAN#101 | RP-231898 | 1215 | 1 | F | Rel-17 Correction in the UE Context Modification procedure abnormal description for conditional mobility modification | 17.6.0 |
| 2023-12 | RAN#102 | RP-233851 | 1220 | 3 | F | Correction on SI delivery to RedCap UE | 17.7.0 |
| 2023-12 | RAN#102 | RP-233851 | 1223 | 4 | F | Support of preconfigured Measurement GAP | 17.7.0 |
| 2023-12 | RAN#102 | RP-233848 | 1230 | 1 | F | Clarification on gNB-DU Cell Resource Configuration for IAB | 17.7.0 |
| 2023-12 | RAN#102 | RP-233849 | 1233 | 1 | F | Correction of F1-U context Reference for PTM | 17.7.0 |
| 2023-12 | RAN#102 | RP-233850 | 1237 | - | F | Correction to F1AP for the misalignment on DL PRS | 17.7.0 |
| 2023-12 | RAN#102 | RP-233850 | 1246 | - | F | Correction on TRP Information Type Response Item IE of Positioning | 17.7.0 |
| 2023-12 | RAN#102 | RP-233847 | 1247 | 1 | F | Correction on NR-Mode-Info IE of SON | 17.7.0 |
| 2024-03 | RAN#103 | RP-240644 | 1266 | 1 | F | Correction of SRB ID for QoE | 17.8.0 |
| 2024-03 | RAN#103 | RP-240645 | 1328 | - | A | PRS Angle Item ASN Correction | 17.8.0 |
| 2024-03 | RAN#103 | RP-240645 | 1330 | 1 | F | PRS bandwidth ASN Correction | 17.8.0 |
| 2024-03 | RAN#103 | RP-240646 | 1351 | 1 | F | Missing procedure text in F1AP | 17.8.0 |
| 2024-03 | RAN#103 | RP-240642 | 1358 | 1 | F | F1 correction for NR-U | 17.8.0 |
| 2024-06 | RAN#104 | RP-241118 | 1368 | 2 | F | Introduction of SL related information transmission over F1 interface | 17.9.0 |
| 2024-06 | RAN#104 | RP-241115 | 1376 | 1 | A | Correction of Time Reference Information | 17.9.0 |
| 2024-06 | RAN#104 | RP-241120 | 1396 | 2 | F | Correction on missing IE for sidelink DRX reference-option1 | 17.9.0 |
| 2024-06 | RAN#104 | RP-241114 | 1423 | 1 | F | UE Context Release Command ASN.1 correction | 17.9.0 |
| 2024-09 | RAN#105 | RP-241871 | 1447 | 1 | F | Correction on Multicast Group Paging | 17.10.0 |
| 2024-09 | RAN#105 | RP-241873 | 1451 | 2 | A | Correction on asymmetric UL and DL for TDD Carrier | 17.10.0 |
| 2024-09 | RAN#105 | RP-241871 | 1463 | 1 | F | Changing presence of IE extensions for MBS related IEs | 17.10.0 |
| 2024-09 | RAN#105 | RP-241870 | 1475 | 1 | A | Correction on SSB transmission periodicity for IAB | 17.10.0 |
| 2024-09 | RAN#105 | RP-241876 | 1482 | 1 | F | Corrections on PEI subgrouping | 17.10.0 |
| 2024-12 | RAN#106 | RP-243104 | 1488 | 1 | A | Correction of Positioning SI for connected UE | 17.11.0 |
| 2024-12 | RAN#106 | RP-243100 | 1490 | 2 | F | Correction for the semantics description | 17.11.0 |
| 2024-12 | RAN#106 | RP-243104 | 1498 | 2 | A | Positioning measurement correction related to "shall, if supported" | 17.11.0 |
| 2024-12 | RAN#106 | RP-243100 | 1501 | 3 | F | Correction on BAP mapping configuration for IAB | 17.11.0 |
| 2024-12 | RAN#106 | RP-243104 | 1513 | 2 | A | Correction on FR1 SRS Bandwidth in Requested SRS Transmission Characteristics | 17.11.0 |
| 2024-12 | RAN#106 | RP-243104 | 1515 | 3 | F | introduction of Missing Bandwidths in FR1 SRS Bandwidth | 17.11.0 |
| 2024-12 | RAN#106 | RP-243102 | 1518 | 1 | A | Clarification on SpCell ID replacement | 17.11.0 |