ETSI STC SMG1	Tdoc SMG1 038/96

Rovaniermi, 26 Feb - 01 March 1996

ETSI/STC/SMG1	Tdoc SMG1 WPA _19___/96

Sophia Antipolis, France

22-23 January 1996

Title:		Support of Optimal Routing

Source:	SMG1 Rapporteur

Support of Optimal Routing (SOR)


Service description, Stage 1

Proposed Version 0.9.0

22-23 January 1996

INTRODUCTION

Editor's note on the remaining work:

This version is a proposition following the discussion at the SMG1 ad hoc on Optimal Routing 21st/22nd December in Düsseldorf.

The stage 1 text has been reduced to the most frequent cases of tromboning in the routing of calls in a mobile environment :

Error: Reference source not found	OR is performed within a country (e.g. Mobile to mobile call where both mobiles are located in the same country)

Error: Reference source not found	OR is performed towards the country where the call would have normally been routed (e.g. Conditional Call Forwarding towards the Home Country of the called party)

All further call scenarios, including multiple call forwarding, are left to subsequent phases of SOR. Subsequent phases of SOR shall be backwards compatible with this first phase.

Table of contents

µ1 Scope	3

2 References	3

3 Definitions and abbreviations	3

4 Description of Optimal Routing	5

5 Functional requirements	5

5.1 General	5

5.2 Normal operation	5

5.2.1 : A is a fixed subscriber or a mobile subscriber who may not benefit from OR	6

5.2.2 : A is a mobile subscriber who may benefit from OR	7

5.3 Exceptional procedures or unsuccessful outcome	12

5.3.1 Non-support of SOR in the VPLMN of the caller	12

5.3.2 Non-support of SOR in the HPLMN of the B subscriber	12

6 Interactions of Optimal routing with features and supplementary services	12

6.1 Operator Determined Barring (GSM 02.41)	12

6.2 Supplementary Services	12

6.2.1 Call Barring (GSM 02.88)	12

6.2.2 Call Transfer (GSM 02.91)	12

6.2.3. Call Deflection	12

6.3 CAMEL (Customised Application for Mobile Network Enhanced Logic, 02.78, ...)	13

7. National Roaming	13

# 1	Scope

This Stage 1 description of the first phase of SOR

Error: Reference source not found	compiles the basic service requirements for SOR;

Error: Reference source not found	describes the interactions for Supplementary Services (SS) in order to cater for SOR;

Error: Reference source not found	refers to modifications to network features required by SOR.

This specification does not address the following:

Error: Reference source not found	There is no need for optimisation of the routing of calls originally directed to a fixed network subscriber, because the physical address of a fixed network terminating line cannot differ from its logical address.

Error: Reference source not found	SOR in non-GSM mobile networks is not a subject of this ETS, but might be possible by bilateral arrangement between GSM network operators and those non-GSM mobile network operators.

The purpose of SOR is to reduce the number of unnecessary inter-PLMN call legs.

The first phase of SOR applies to :

Error: Reference source not found	OR performed within a country (e.g. Mobile to mobile calls where both mobile subscribers are located in the same country)

Error: Reference source not found	OR performed towards the country where the call would have normally been routed wihtout (e.g. Conditional Call Forwarding towards the Home Country of the called party)

The complete set of scenarios included in the phase 1 of OR are presented in the paragraph for normal procedures. All other scenarios are excluded of the Phase 1 of OR.

All further call scenarios, including multiple call forwarding, are left to subsequent phases of SOR. Subsequent phases of SOR shall be backwards compatible with this first phase.

# 2	References

<to be inserted>

# 3	Definitions and abbreviations

Error: Reference source not found	Direct route:

A call uses the direct route if it is routed from the serving network of the original calling party to the serving network of the ultimate called party without any intermediate PLMN.

Error: Reference source not found	HPLMN route:

A call uses the HPLMN route if the destination is deduced from the MSISDN of the called party. This forces the call to be routed via the HPLMN of the called party.

If the called party is registered in her HPLMN, the direct route and the HPLMN route are the same.

Error: Reference source not found	HPLMN leg:

The HPLMN leg is that part of the HPLMN route from the IPLMN to the HPLMN of the called party.

Error: Reference source not found	IPLMN:

A IPLMN is a PLMN which interrogates the HPLMN of a called or forwarded-to party in order to determine the whereabouts of that party.

·	Early Call Forwarding :

Call Forwarding performed from the IPLMN before the call has been extended to the VPLMN of the forwarding subscriber (i.e. Call Forwarding Unconditional and Conditional Call Forwarding on Not Reachable known at the IPLMN before extension of the call)

·	Late Conditional Call Forwarding :

Call Forwarding performed after the call has been extended to the VPLMN of the forwarding subscriber (i.e. Conditional Call Forwarding on Busy, Conditional Call Forwarding on No Reply and Conditional Call Forwarding on Not Reachable detected in the VPLMN of the forwarding subscriber). Late Call Forwarding may be invoked in the IPLMN or in the VPLMN of the forwarding subscriber.

# 4	Description of Optimal Routing

Support of Optimal Routing (SOR) is a network feature, which enables the calls directed to a mobile subscriber, to be routed directly to the mobile subscriber's actual location, or to her forwarded-to destination (instead of via the HPLMN or in the case of Late Conditional Call Forwarding via the VPLMN) .

The IPLMN handling the call shall decide whether to optimise the routing of the call or not taking into account information provided by the called mobile subscriber's HPLMN. The  HPLMN may permit or deny OR as a network option on a subscriber basis.

NOTE:	References to the provision of Data Privacy supplementary services in case of OR have been deleted from MoU reference documents.

# 5	Functional requirements

## 5.1	General

[Hidden text :

If any party involved in a call is a fixed network party, the equivalent of the VPLMN and the HPLMN is the fixed network serving that party.]

SOR shall be provided for all circuit switched Telecommunication Services, except emergency calls, Dedicated PAD and Dedicated Packet Access (GPRS is for further study).

The network feature shall be applied automatically for all home and visiting subscribers except for the subscribers for whom the HPLMN denies the optimal routing of calls directed to them.

## 5.2	Normal operation

In the first phase of SOR, the routes of calls will be optimised in case of OR being performed within a country or towards the country where the call would have been routed normally..

Under all other circumstances, the GSM basic call routing  is applied :

·	If the IPLMN detects that basic OR cannot be applied, the IPLMN routes the call according to the dialled number.

·	If the IPLMN detects that OR for Late Conditional Call Forwarding cannot be applied, the Call Forwarding will be performed in the VPLMN of the forwarding subscriber.

If the serving network of the ultimate destination supports SOR, the serving network of the caller shall be able to indicate to the network visited by the ultimate destination that the call has been optimally routed and to indicate the originating PLMN of the optimally routed leg.

The complete set of scenarios included in the phase 1 of SOR are described below.

If one of those scenarios occurs, SOR shal be invoked.

The assumptions taken are the following :

- A subscriber A sets up a call to a mobile subscriber B, who may have forwarded her calls to a subscriber C.

- The C subscriber may be either a fixed subscriber or a mobile subscriber.

In case of C being a mobile subscriber, the location of C taken into account in the following paragraphs is the location of the HPLMN(C).

### 5.2.1 : A is a fixed subscriber or a mobile subscriber who may not benefit from OR

As the originating network does not have the ability to interrogate the HPLMN of the B subscriber, the normal GSM call handling is applied.

Once the HPLMN of the B subscriber has the control of the call, the call may be optimised in the case of a forwarded call towards a party located in the Home Country of the B subscriber or in the country visited by the B subscriber.

Note :

- A may be in any country.

- In the two following cases, the IPLMN-B is the HPLMN-B.

SCENARIO 1 : OR for Late Conditional Call Forwarding

C is in the same country as A

µ §

SCENARIO 2 : OR for Late Conditional Call Forwarding

C is in the same country as HPLMN(B)

µ §

### 5.2.2 : A is a mobile subscriber who may benefit from OR

·	Description of the call routing

If A sets up a call to B, then the originating PLMN shall interrogate the HPLMN of the mobile subscriber B in order to know how to route the call.

n	If B is registered in the same country as A, then the call shall be routed directly to B as described above.

n	If B has activated a Early Call Forward to a destination located in the Home Country of B or in the country where A is registered, then the call shall be routed directly from A to the forwarded-to-party.

n	In any other cases, the call shall be routed to the mobile subscriber B via the HPLMN of the B subscriber.

If the call cannot be completed, the originating PLMN shall receive an indication of the reason of the failure of the call completion.

Thereafter, if the mobile subscriber B has activated the corresponding Conditional Call Forward to a destination located in his Home Country or to a destination in the country where A is registered, then the originating PLMN shall route directly the call to the forwarded-to-party.

The remaining leg of the call from the intermediate point to the ultimate destination may be optimally routed.

·	Call scenarios handled in OR Phase 1

Note : In the following scenarios, the IPLMN-B is the VPLMN-A.

1) The call from the A subscriber to the B subscriber is completed (Basic OR)

a) B is located in the same country as A

SCENARIO 3 : BASIC OR - B in the same country as A

µ §

b) B is located in her Home Country (Network usage optimization)

SCENARIO 4 : BASIC OR - B in her home country

µ §

2) The call set up by the subscriber A is being forwarded to a C party

2.1) Early Call Forward

[Ed Note : for the two following scenarios, the location of B is not relevant.]

a)	the C party is in the same country as the A subscriber

SCENARIO 5  : Early CF - C in the same country as A

µ §

b)	the C party is in the same country as HPLMN(B)

SCENARIO 6 : Early CF - C in the same country as HPLMN(B)

µ §

2.2) Late Conditional Call Forward

a)	B and C are in the same country as A

SCENARIO 7 : BASIC OR - OR for Late Conditional Call Forwarding

B in the same country as A - C in the same country as A

µ §

b)	B is in the same country as A and C is in the same country as HPLMN(B)

SCENARIO 8 : BASIC OR - OR for Late Conditional Call Forwarding

B in the same country as A - C in the same country as HPLMN(B)

µ §

c)	B is in her home country and C is in the same country as A

SCENARIO 9 : BASIC OR - OR for Late Conditional Call Forwarding

B in the same country as HPLMN(B) - C in the same country as A

µ §

d)	B and C are in the country of HPLMN(B)

SCENARIO 10 : BASIC OR - OR for Late Conditional Call Forwarding

B in the same country as HPLMN(B) - C in the same country as HPLMN(B)

µ §

## 5.3	Exceptional procedures or unsuccessful outcome

### 5.3.1	Non-support of SOR in the VPLMN of the caller

If subscriber A roams into a network not supporting OR, she may not benefit from OR and only scenario 1 and scenario 2 of SOR are supported.

### 5.3.2	Non-support of SOR in the HPLMN of the B subscriber

If the HPLMN of the called party does not support SOR, SOR cannot be invoked and the route of the call as far as the called party’s VPLMN cannot be optimised (the call would normally be routed via the HPLMN of the called party).

The same procedure shall be followed if the HPLMN operator denies OR on a subscriber basis.

5.3.3. Non support of SOR in the VPLMN of the B subscriber

If a subscriber whose HPLMN supports SOR registers in a VPLMN which does not support SOR, only the cases of Early Forwarded calls may be optimised.

# 6	Interactions of Optimal routing with features and supplementary services

## 6.1	Operator Determined Barring (GSM 02.41)

The principles for the interaction between operator determined barring and SOR are the same as those for the interaction between Supplementary Service Call Barring and SOR.

## 6.2	Supplementary Services

No Interaction unless it is stated differently below.

[Editor´s note: This general rule does not apply to Call Forwarding as there is a close interaction between CF and SOR during the call set-up phase]

### 6.2.1	Call Barring (GSM 02.88)

Call Barring services are applied on the dialled number.

Note : The existing interactions between Call Forwardings and Call Barrings are not changed by the introduction of OR.

### 6.2.2	Call Transfer (GSM 02.91)

A transferred call is considered as a set  of two separate calls which may be seperately optimally routed.

Invocation of Call Transfer does not affect the routing of the calls involved.

### 6.2.3.	Call Deflection

SMG1 WPB's answer is expected.

## 6.3	CAMEL (Customised Application for Mobile Network Enhanced Logic, 02.78, ...)

If CAMEL has to be applied to any part of the call and if CAMEL modifies the destination of the call, the OR applies to any new destination introduced by CAMEL. This is applicable on all parts of the call, i.e. originating, terminating or forwarded leg.

If Barring services are also applicable to the call then they have to be handled first together with CAMEL before SOR is applied.

# 7. National Roaming

The principles of SOR may be applied to National Roaming situations.