ETSI STC SMG1	Tdoc SMG1    139    /96

Rovaniemi, 26 February - 01 March 1996

DRAFT

ETS 300 ###

(GSM 02.79)

Version:	Proposed 1.0.1

Date:	21/03/96

Key words:

European digital cellular telecommunications system;

Support of Optimal Routeing (SOR);

Service definition (Stage 1)

ETSI

European Telecommunications Standards Institute

ETSI Secretariat

Postal address: F-06921 Sophia Antipolis CEDEX - FRANCE

Office address: 650 Route des Lucioles - Sophia Antipolis - Valbonne - FRANCE

X.400: c=fr, a=atlas, p=etsi, s=secretariat - Internet: secretariat@etsi.fr

Tel.: +33 92 94 42 00 - Fax: +33 93 65 47 16

Copyright Notification: Reproduction is only permitted for the purpose of standardization work undertaken within ETSI. The copyright and the foregoing restrictions on reproduction extend to all media.

© European Telecommunications Standards Institute 1996. All rights reserved.

Blank page

Contents

Foreword	5

INTRODUCTION	5

1	Scope	7

2	References	7

3	Definitions and abbreviations	7

4	Description of Optimal Routing	9

5	Functional requirements	9

5.1	General	9

5.2	Normal operation	9

5.2.1	A is a fixed subscriber or a mobile subscriber who may not benefit from OR	10

5.2.2	A is a mobile subscriber who may benefit from OR	11

5.2.2.1	Description of the call routing	11

5.2.2.2	Call scenarios handled in OR Phase 1	12

5.3	Exceptional procedures or unsuccessful outcome	17

5.3.1	Non-support of SOR in the VPLMN of the caller	17

5.3.2	Non-support of SOR in the HPLMN of the B subscriber	17

5.3.3	Non support of SOR in the VPLMN of the B subscriber	17

6	Interactions of Optimal routing with features and supplementary services	17

6.1	Operator Determined Barring (GSM 02.41)	17

6.2	Supplementary Services	17

6.2.1	Call Barring (GSM 02.88)	17

6.2.2	Call Transfer (GSM 02.91)	17

6.2.3.	Call Deflection	17

6.3	CAMEL (Customised Application for Mobile Network Enhanced Logic, 02.78, ...)	18

Blank page

# Foreword (first clause, unnumbered)

To be drafted by the ETSI Secretariat.

# INTRODUCTION

Blank page

# 1	Scope

This Stage 1 description of the first phase of SOR

-	compiles the basic service requirements for SOR;

-	describes the interactions for Supplementary Services (SS) in order to cater for SOR;

-	refers to modifications to network features required by SOR.

This specification does not address the following:

-	There is no need for optimisation of the routing of calls originally directed to a fixed network subscriber, because the physical address of a fixed network terminating line cannot differ from its logical address.

-	SOR in non-GSM mobile networks is not a subject of this ETS, but might be possible by bilateral arrangement between GSM network operators and those non-GSM mobile network operators.

The purpose of SOR is to reduce the number of unnecessary inter-PLMN call legs.

The first phase of SOR applies to :

-	OR performed within a country (i.e. Mobile to mobile calls where both mobile subscribers are located in the same country);

-	OR performed towards the country where the call would have normally been routed without OR (e.g. Conditional Call Forwarding towards the Home Country of the called party).

The complete set of scenarios included in the phase 1 of OR is presented in the paragraph for normal procedures. All other scenarios are excluded from Phase 1 of OR.

Note that Optimal Routing is applicable to national roaming situations, that is to calls directed to a mobile subscriber roaming in her home country, but registered in a PLMN different from her HPLMN.

All further call scenarios, including multiple call forwarding, are left to subsequent phases of SOR. Subsequent phases of SOR shall be backwards compatible with this first phase.

# 2	References

<to be inserted>

# 3	Definitions and abbreviations

Administrative PLMN: The complete set of all functional entities normally regarded as a singe PLMN.

Basic OR : Routeing of a call by the direct route when no forwarding occurs.

Direct route: A call uses the direct route if it is routed from the serving network of the original calling party to the serving network of the ultimate called party without any intermediate PLMN.

HPLMN route: A call uses the HPLMN route if the destination is deduced from the MSISDN of the called party. This forces the call to be routed via the HPLMN of the called party.

HPLMN leg: The HPLMN leg is that part of the HPLMN route from the IPLMN to the HPLMN of the called party.

IPLMN: An IPLMN is a PLMN which interrogates the HPLMN of a called party in order to determine the whereabouts of that party.

Early Call Forwarding : Early Call Forwarding is Call Forwarding performed from the IPLMN before the call has been extended to the VPLMN of the forwarding subscriber (i.e. Call Forwarding Unconditional and Conditional Call Forwarding on Not Reachable known at the IPLMN before extension of the call)

Functional PLMN: For the purposes of this description the Administrative PLMN is divided into functional PLMNs that represent different aspects of the Optimal Routeing functionality.

Late Call Forwarding: Late Call Forwarding is Call Forwarding performed after the call has been extended to the VPLMN of the forwarding subscriber (i.e. Conditional Call Forwarding on Busy, Conditional Call Forwarding on No Reply and Conditional Call Forwarding on Not Reachable detected in the VPLMN of the forwarding subscriber). Late Call Forwarding may be invoked in the IPLMN or in the VPLMN of the forwarding subscriber.

# 4	Description of Optimal Routing

Support of Optimal Routing (SOR) is a network feature which enables the calls directed to a mobile subscriber to be routed directly to the mobile subscriber's actual location, or to her forwarded-to destination (instead of via the HPLMN or in the case of Late Call Forwarding via the VPLMN).

The IPLMN handling the call shall decide whether or not to optimise the routing of the call taking into account information provided by the called mobile subscriber's HPLMN. For given subscribers, as a network option, the  HPLMN may permit or deny OR on a per call basis.

NOTE:	References to the provision of Data Privacy supplementary services in case of OR have been deleted from MoU reference documents.

# 5	Functional requirements

## 5.1	General

[Hidden text :

If any party involved in a call is a fixed network party, the equivalent of the VPLMN and the HPLMN is the fixed network serving that party.]

SOR shall be provided for all circuit switched Telecommunication Services, except emergency calls, Dedicated PAD and Dedicated Packet Access (GPRS is for further study).

The network feature shall be applied automatically for all calls except for those calls for which the HPLMN of the called party denies the optimal routing.

## 5.2	Normal operation

In the first phase of SOR, the routes of calls will be optimised for the cases of OR being performed within a country or towards the country where the call would have been routed normally..

Under all other circumstances, the GSM basic call routing  is applied:

-	If the IPLMN detects that basic OR cannot be applied, the IPLMN routes the call according to the dialled number.

-	If the IPLMN detects that OR for Late Call Forwarding cannot be applied, the Call Forwarding will be performed in the VPLMN of the forwarding subscriber.

If the serving network of the ultimate destination supports SOR, the serving network of the caller shall be able to indicate to the network visited by the ultimate destination that the call has been optimally routed and to indicate the originating PLMN of the optimally routed leg.

The complete set of scenarios included in phase 1 of SOR is described below.

The notation used for the scenario diagrams is defined in figure 1.

Figure 1: Notation used in scenario diagrams

If one of those scenarios occurs, SOR shall be invoked.

The assumptions taken are the following :

-	A subscriber A sets up a call to a mobile subscriber B, who may have forwarded her calls to a subscriber C.

-	The C subscriber may be either a fixed subscriber or a mobile subscriber.

If C is a mobile subscriber, the location of C taken into account in the following paragraphs is the location of HPLMN-C.

### 5.2.1	A is a fixed subscriber or a mobile subscriber who may not benefit from OR

As the originating network does not have the ability to interrogate the HPLMN of the B subscriber, the normal GSM call handling is applied.

Once the HPLMN of the B subscriber has the control of the call, the call may be optimised in the case of a forwarded call towards a party located in the Home Country of the B subscriber or in the country visited by the B subscriber.

Note that:

-	A may be in any country.

-	In the two following cases, the IPLMN is HPLMN-B.

SCENARIO 1: OR for Late Call Forwarding,
C is in the same country as HPLMN-B

NOTE:	Call records shown in the figure are only for information

SCENARIO 2: OR for Late Call Forwarding,
C is in the same country as VPLMN-B

NOTE:	Call records shown in the figure are only for information

### 5.2.2	A is a mobile subscriber who may benefit from OR

#### 5.2.2.1	Description of the call routing

If A sets up a call to B, then the originating PLMN shall interrogate the HPLMN of the mobile subscriber B in order to know how to route the call.

If B is registered in the same country as A, then the call shall be routed directly to B as described below.

If B has activated a Call Forward to a destination located in the Home Country of B or in the country where A is registered and this leads to the invocation of Early Call Forwarding, then the call shall be routed directly from A to the forwarded-to-party.

In any other cases, the call shall be routed to the mobile subscriber B via HPLMN(B).

If the call cannot be completed, the originating PLMN shall receive an indication of the reason of the failure of the call completion.

Thereafter, if the mobile subscriber B has activated a Conditional Call Forward to a destination located in her Home Country or to a destination in the country where A is registered and this leads to the invocation of Late Call Forwarding, then the originating PLMN shall route the call directly to the forwarded-to-party.

The remaining leg of the call from the intermediate point to the ultimate destination may be optimally routed.

#### 5.2.2.2	Call scenarios handled in OR Phase 1

In the following scenarios, the IPLMN  is VPLMN-A.

1) The call from the A subscriber to the B subscriber is completed (Basic OR)

a) B is located in the same country as A

SCENARIO 3: BASIC OR, B in the same country as A

NOTE:	Call records shown in the figure are only for information

b) B is located in her Home Country (Network usage optimization)

SCENARIO 4: BASIC OR, B in her home country

NOTE:	Call records shown in the figure are only for information

2) The call set up by the subscriber A is being forwarded to a C party

2.1) Early Call Forward

Note: 	for the two following scenarios, the location of B is not relevant.

a)	the C party is in the same country as the A subscriber, but different from HPLMN B country.

SCENARIO 5: Early CF, C in the same country as A

NOTE:	Call records shown in the figure are only for information

b)	the C party is in the same country as HPLMN-B

SCENARIO 6: Early CF, C in the same country as HPLMN-B

NOTE:	Call records shown in the figure are only for information

2.2) Late Call Forward

a)	B and C are in the same country as A

SCENARIO 7: BASIC OR + OR for Late Call Forwarding,
B in the same country as A, C in the same country as A

NOTE:	Call records shown in the figure are only for information

b)	B is in the same country as A and C is in the same country as HPLMN-B

SCENARIO 8: BASIC OR + OR for Late Call Forwarding,
B in the same country as A, C in the same country as HPLMN-B

NOTE:	Call records shown in the figure are only for information

c)	B is in her home country and C is in the same country as A

SCENARIO 9: BASIC OR + OR for Late Call Forwarding,
B in the same country as HPLMN-B, C in the same country as A

NOTE:	Call records shown in the figure are only for information

d)	B and C are in the country of HPLMN-B

SCENARIO 10: BASIC OR + OR for Late Conditional Call Forwarding,
B in the same country as HPLMN-B, C in the same country as HPLMN-B

NOTE:	Call records shown in the figure are only for information

## 5.3	Exceptional procedures or unsuccessful outcome

### 5.3.1	Non-support of SOR in the VPLMN of the caller

If subscriber A roams into a network not supporting OR, she may not benefit from OR and only scenario 1 and scenario 2 of SOR are supported.

### 5.3.2	Non-support of SOR in the HPLMN of the B subscriber

If the HPLMN of the called party does not support SOR, SOR cannot be invoked and the route of the call as far as the called party’s VPLMN cannot be optimised (the call would normally be routed via the HPLMN of the called party).

The same procedure shall be followed if the HPLMN operator denies OR on a subscriber basis.

### 5.3.3	Non support of SOR in the VPLMN of the B subscriber

If a subscriber whose HPLMN supports SOR registers in a VPLMN which does not support SOR, only the cases of Early Forwarded calls may be optimised.

# 6	Interactions of Optimal routing supplementary services

## No Interaction unless it is stated differently below.

## 6.2	Call Forwarding

Interactions between SOR and Call Forwarding are dealt with in section 5.0.

6.2	Call Barring

Outgoing Call Barring services are applied according to the dialled number.

Barring of Incoming calls when Roaming outside the HPLMN country will prevent calls to a mobile subscriber who has roamed outside her HPLMN country even if OR would result in no chargeable roaming leg.

The existing interactions between Call Forwardings and Call Barrings are not changed by the introduction of OR.

## 6.3	Call Transfer

A transferred call is considered as a set  of two separate calls which may be seperately optimally routed.

Optimal Routing shall not be invoked as a result of the invocation of Call Transfer.

## 6.4	Call Deflection

A deflected call is considered as a late forwarded call and as such may be optimally routed.

# 7 with Operator Determined Barring (ODBaction between operator determind barring and SOR are the same as those for the interaction between Supplementary Service Call Barring and SOR.

# IArt of the call and if CAMEL modifies the dd by CAMEL. This is applicable on all parts of the call, i.e. originating, terminating or forwarded leg.

If Barring services are also applicable to the call then they have to be handled first together with CAMEL before SOR is applied.