| 3GPP TR 26.942 V0.3.1 (2024-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Media Energy Consumption Exposure and Evaluation Framework; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	11

3.1	Terms	11

3.2	Symbols	11

3.3	Abbreviations	11

4	Introduction to energy efficiency for media	12

4.1	General	12

4.1.1	Motivation	12

4.1.2	Energy and power in mobile networks	12

4.1.3	Energy and power in mobile device	13

4.1.4	Greenhouse gas emissions reporting and energy measurement	13

4.1.5	Legislative frameworks for greenhouse gas reporting	13

4.2	Related work	13

4.2.1	Introduction	14

4.2.2	3GPP	14

4.2.2.1	Introduction	14

4.2.2.2	Collection and exposure of energy consumption information at OAM	14

4.2.2.1.1	Introduction	14

4.2.2.1.2	Collection of network energy information by OAM	14

4.2.2.1.3	Exposure of network energy information by OAM	15

4.2.2.3	 Collection and exposure of energy consumption information at NF	15

4.2.2.4	UE data collection, reporting and event exposure	17

4.2.2.4.1	UE data collection, reporting and event exposure architecture	17

4.2.2.4.2	UE data collection, reporting and event exposure for 5G Media Streaming	18

4.2.2.4.3	 QoE Measurement Collection (QMC) functionality	19

4.2.2.4.4	Potential use of UE data collection, reporting and event exposure architecture for collection and exposure of energy consumption information	19

4.2.3	Other Standards Development Organisations	20

4.2.3.1	ITU-T	20

4.2.3.2	ITU-R	20

4.2.3.3	MPEG	21

4.2.3.4	DVB	21

4.2.3.5	ATSC	21

4.2.4	Industry Fora	21

4.2.4.1	Greening of Streaming	21

4.2.4.2	DIMPACT	21

4.2.4.3	Ultra HD Forum	21

4.2.5	Greenhouse Gas Protocol	21

4.2.5.1	Scope 1	22

4.2.5.2	Scope 2	22

4.2.5.3	Scope 3	22

5	Use cases	23

5.1	Baseline use cases defined by SA1	23

5.2	Refinement of relevant use cases defined by 3GPP SA1 in 3GPP SA4 context	24

5.3	Additional use cases defined by 3GPP SA4	24

6	Key issues	24

6.1	Key Issue #1: Information exposure	24

6.1.1 	Description	24

6.1.2	Potential requirements	25

6.2	Key Issue #2: Monitoring and measurement	26

6.2.1 	Description	26

6.2.2	Potential requirements	27

6.3	Key Issue #3: Evaluation framework	27

6.3.1 	Description	27

6.3.2	Potential requirements	28

7	Potential Solutions	28

7.1 	Mapping of Solutions to Key issues	28

7.X	Solution #<X>: <Solution Title>	28

7.X.1	Key issue mapping	28

7.X.2	Functional Description	28

7.X.3	Procedures	28

7.X.4	Impacts on existing services, entities and interfaces	28

8	Conclusions and proposed next steps	28

Annex <A>: <Informative annex title for a Technical Report>	30

Annex <B> (informative): Change history	31

# Foreword

This clause is mandatory; do not alter the text in any way other than to choose between "Specification" and "Report".

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

This clause shall start on a new page.

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Lunden2022]		D Lundén, J Malmodin, P Bergmark and N Lövehagen, "Electricity Consumption and Operational Carbon Emissions of European Telecom Network Operators", Sustainability 14(5):2637, 2022.

[Malmodin2020]		J Malmodin, "The power consumption of mobile and fixed network data services-The case of streaming video and downloading large files." Electronics Goes Green, 2020.

[Malmodin2024]		J Malmodin, N Lövehagen, P Bergmark and D Lundén, "ICT sector electricity consumption and greenhouse gas emissions – 2020 outcome", Telecommunications Policy (2024): 102701.

[GGPweb]	The Greenhouse Gas Protocol, https://ghgprotocol.org

[GGPacc]	The Greenhouse Gas Protocol, “A Corporate Accounting and Reporting Standard, Revised Edition”

[GGPsc3]	The Greenhouse Gas Protocol: "Corporate Value Chain (Scope 3) Accounting and Reporting Standard: Supplement to the GHG Protocol Corporate Accounting and Reporting Standard".

[EUreg]	The European Commission: "Commission Delegated Regulation (EU) 2023/2772", The Official Journal of the European Union, 23-7-2023

[Ember]	Ember Climate: "Global Electricity Review 2023",
https://ember-climate.org/insights/research/global-electricity-review-2023/

[28913]	3GPP TR 28.913: "Study on new aspects of Energy Efficiency (EE) for 5G phase 2"

[28554]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[28552]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[28533]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[28622]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[28532]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[23501]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[23288]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[23700-66]	3GPP TR 23.700-66: "Study on Energy Efficiency and Energy Saving".

[28.310]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[26531]	3GPP TS 26.531: "Data collection and reporting; General description and architecture".

[26532]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[26501]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[26510]	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling".

[26247]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[38300]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2”.

[26114]	3GPP TS 26.114: “IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction”

[26118]	3GPP TS 26.118: “Virtual Reality (VR) profiles for streaming applications”

[28.405]	3GPP TS 28.405: "Telecommunication management; Quality of Experience (QoE) measurement collection; Control and configuration”

[CTA-5005-A]	Consumer Technology Association CTA-5005-A: "Web Application Video Ecosystem – DASH-HLS Interoperability Specification".

[L.1210]	International Telecommunication Union, Recommendation ITU-T L.1210, "Sustainable power-feeding solutions for 5G networks", December 2019

[L.1220]	International Telecommunication Union, Recommendation ITU-T L.1220, "Innovative energy storage technology for stationary use – Part 1: Overview of energy storage", 8/2017

[L.1221]	International Telecommunication Union, Recommendation ITU-T L.1221, "Innovative energy storage technology for stationary use – Part 2: Battery", 11/2018

[L.1222]	International Telecommunication Union, Recommendation ITU-T L.1222, "Innovative energy storage technology for stationary use – Part 3: Supercapacitor technology", 5/2018

[L.1331]	International Telecommunication Union, Recommendation ITU-T L.1331, "Assessment of mobile network energy efficiency", 1/2022

[L.1350]	International Telecommunication Union, Recommendation ITU-T L.1350, "Energy efficiency metrics of a base station site", 10 /2016

[L.1351]	International Telecommunication Union, Recommendation ITU-T L.1351, "Energy efficiency measurement methodology for base station sites", 8/2018

[L.1380]	International Telecommunication Union, Recommendation ITU-T L.1380, "Smart energy solution for telecom sites", 11/2019

[L.1381]	International Telecommunication Union, Recommendation ITU-T L.1381, "Smart energy solutions for data centres", 6/2020

[L.1382]	International Telecommunication Union, Recommendation ITU-T L.1382, "Smart energy solution for telecommunication rooms", 6/2020

[L.1383]	International Telecommunication Union, Recommendation ITU-T L.1383, "Smart energy solutions for city and home applications", 10/2021

[L.1310]	International Telecommunication Union, Series L Supplement 36, "ITU-T L.1310 – Study on methods and metrics to evaluate energy efficiency for future 5G systems", 11/2017

[L.sup43]	International Telecommunication Union, Series L Supplement 43, "Smart energy saving of 5G base stations: Traffic forecasting and strategy optimization of 5G wireless network energy consumption based on artificial intelligence and other emerging technologies", 5/2021

[L.1450]	International Telecommunication Union, Recommendation ITU-T L.1450, "Methodologies for the assessment of the environmental impact of the information and communication technology sector", 9/2018

[ICT]	Jens Malmodin, Nina Lövehagen, Pernilla Bergmark, and Dag Lundén. "ICT sector electricity consumption and greenhouse gas emissions–2020 outcome." Telecommunications Policy (2024): 102701

[BT2385]	International Telecommunication Union, Report ITU-R BT.2385-1, "Reducing the environmental impact of terrestrial broadcasting systems", 03/2022

[OP104]	International Telecommunication Union, Opinion ITU-R OP.104, "Advice for sustainability strategies incorporating carbon offsetting policies", 2022

[BT2521]	International Telecommunication Union, Report ITU-R BT.2521-0, "Practical examples of actions to realize energy aware broadcasting", 3/2023

[BT2540]		International Telecommunication Union, Report ITU-R BT.2540-0, "Display energy reduction through image signal processing", 03/2024

[23001-11]	ISO/IEC 23001-11:2023 Information technology, MPEG systems technologies,  Part 11: Energy-efficient media consumption (green metadata)

[S100]	DVB, "Study Mission report on Energy Aware service Delivery and Consumption", DVB Document S100, 11/2023

[PT9]	ATSC Planning Team 9, https://www.atsc.org/subcommittees/planning-team-9-sustainability/

[GoS]	Greening of Streaming, https://www.greeningofstreaming.org/

[DIMPACT]	DIMPACT, https://dimpact.org/

[DTMeth]	DIMPACT, "Methodology: Estimating the carbon impacts of serving digital media and entertainment products", version 1.0, October 2022

[DTPaper]	DIMPACT, Draft paper "Literature review and policy principles for streaming and digital media carbon footprinting", March 2023

[UHDF]	Ultra HD Forum, https://ultrahdforum.org/ibc2023-press-release-ultra-hd-forum-to-showcase-efficient-hdr-sdr-sustainability-demos/

[22282]	3GPP TR 22.882: "Study on Energy Efficiency as a service criteria".

[ARCEP]	ARCEP, "Arcep publishes a draft decision for public consultation, with a view to enhancing its annual “Achieving digital sustainability” survey",
https://en.arcep.fr/fileadmin/cru-1714402758/user_upload/41-24-english-version.pdf

[26501]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[26502]	3GPP TS 26.502: "5G multicast-broadcast services; User service architecture".

[26506]	3GPP TS 26.506: "5G Real-time Media Communication Architecture".

[EmpCo]	Directive (EU) 2024/825: "amending Directives 2005/29/EC and 2011/83/EU as regards empowering consumers for the green transition through better protection against unfair practices and better information", 28th February 2024.
https://eur-lex.europa.eu/legal-content/EN/TXT/?uri=CELEX:32024L0825

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to TR 21.905 be the first in the list.

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core

5GMS	5G Media Streaming

AF	Application Function

AMF	Access and Mobility Management Function

ANBR	Access Network Bitrate Recommendation

AS	Application Server

ATSC	Advanced Television Systems Committee

DVB	Digital Video Broadcasting

IBC	International Broadcasting Convention

ICT	Information and Communications Technology

ITU-T	The Telecommunication Sector of the ITU

MnS	Management Service

NAB	National Association of Broadcasters

NEF	Network Exposure Function

NG-RAN	Next Generation RAN

NSACF	Network Slice Admission Control Function

NWDAF	Network Data Analytics Function

OAM	Operations, Administration and Maintenance

PCF	Policy Control Function

PEE	Power, Energy, Environmental

PNF	Physical Network Function

QMC	QoE Measurement Collection

QoE	Quality of Experience

RAN	Radio Access Network

SBMA	Service-Based Management Architecture

SMF	Session Management Function

UDM	Unified Data Management

UE	User Equipement

UPF		User Plane Function

VNF	Virtualized Network Function

# 4	Introduction to energy efficiency for media

## 4.1	General

[Editor’s note: Introduction to the concepts of energy efficiency for media]

### 4.1.1	Motivation

The reason for studying energy consumption in media delivery stems from a concern for the current state of the climate, and the need to mitigate the effects of human-induced climate change. These effects are due to greenhouse gas emissions associated with human activity, including the production of energy. In this regard, mitigation strategies revolve around 1) producing cleaner energy, and 2) using less energy. The latter is relevant for any sector, system or device not directly involved in producing energy, including those defined by 3GPP. However, with 70–80% of network traffic being media, media data centres, the transmission of media data and media consumption on UEs contribute significantly to the total energy consumed by mobile networks. Considering the 5G System, energy efficiency of each of its components as well as the system as a whole is required.In order to achieve increased energy efficiency – both at the component level and at the system level – the system needs first to be characterised. Such characterisation additionally enables reporting, thereby informing the various stakeholders of the system's energy performance. High-level measurements illustrated in clauses 4.1.2 and 4.1.3 are too coarse to allow system performance improvements, nor does it allow individual stakeholders, including Application Service Providers, network operators, and end users to know their own instantaneous energy use. Having access to fine-grained information on energy use, for instance on streaming an individual content asset, would allow the identification of potential energy hot spots, and it would facilitate government-mandated reporting which is increasingly prevalent in certain markets.

This feasibility study therefore focuses on the possibility of putting infrastructure in place that would enable the measurement and reporting of energy consumption across the media delivery eco-system of 5G networks.

### 4.1.2	Energy and power in mobile networks

The terms power and energy are closely related, with power P being the rate at which work is done. It is measured in Watts or equivalently Joules per second (symbol W), or in derived quantities such as kW, MW or TW. Energy E is power integrated over time, measured in Joules (J), or equivalently Watt-seconds (Ws). Larger quantities are often measured in kilo-Watt-hours kWh, mega-Watt-hours MWh or tera-Watt-hours (TWh). One kWh represents 3.6 ×106 J.

For the year 2020, the global annual electricity consumption (AEC) of mobile networks (including 2G up to 5G, as well as satellite communication) is estimated to have been 161 TWh, of which 146 TWh is spent by access networks, 6 TWh by the core network, and 9 TWh by support activities [Malmodin2024]. This represents 20 kWh per subscription per year [Malmodin2024]. In the period 2015–2018 this figure was estimated at 17 kWh per subscription per year [Lunden2022].

To characterize the energy used to transmit data in a more fine-grained manner, energy-per-data figures are often reported, for example in kWh/GB. This suggests that a given network expends energy directly proportional to the amount of data communicated. This, however, has been shown to be an inaccurate measure due to the presence of significant fixed overheads. As an example, the servers in a data centre need to be cooled, irrespective of whether data passes through them or not.

For this, and other reasons, the transmission of data incurs a base load which is related to the presence and maintenance of the infrastructure itself, plus a mark-up that depends on the amount of data being transmitted.

Examples of power usage for 4G systems and use cases, taking into account such base load, are given in table 4.1.2-1 [Malmodin 2020].

| System / Use case | Bit rate (Mb/s) | a (W) | b (W/Mb/s) | P (W) |
| --- | --- | --- | --- | --- |
| 4G RAN |  | 0.5 – 2 | 1 – 2 |  |
| 4G data transmission and IP core network |  | 0.05 – 0.5 | 0.03 |  |
| No data (inactive) | 0 | 1.2 | 1.53 | 1.2 |
| "YouTube" application service | 1.5 | 1.2 | 1.53 | 3.4 |
| "Netflix" application service | 4 | 1.2 | 1.53 | 7.3 |
| File download | 40 | 1.2 | 1.53 | 62 |

Table 4.1.2-1. example of power usage in 4G mobile transmission systems (after [Malmodin2020])

Where for a Bit rate (in megabits per second, Mb/s), a is the base load (in W), b is the data-dependent term (in W/Mb/s), and P is the power consumped (in W).

As can be seen in this table 4.1.2-1, the fixed overhead a is relatively important at low bit rates. For larger bit rates (e.g. the file download example) the transmission rate dominates the power consumption.

### 4.1.3	Energy and power in mobile device

According to [Malmodin 2024] the global annual electricity consumption of smartphones and feature phones in 2020 is estimated to have been around 17 and 2 TWh respectively.

### 4.1.4	Greenhouse gas emissions reporting and energy measurement

A study from Ember Climate [Ember] emphasises that electricity use may be mapped onto greenhouse gas via a conversion factor known as the carbon intensity measured in grams of Cos equivalent per kilowatt hour (g CO2-e/kWh). The carbon intensity depends strongly on the method used to produce electricity, and therefore on the natural resources available in a given geographic location. Currently, the carbon intensity ranges from under 100 CO2-e/kWh to over 700 CO2-e/kWh, with a global average of 436 CO2-e/kWh (data from [Ember]).

The measurement of greenhouse gas emissions is difficult if not impossible to perform directly, but through the locally and globally known carbon intensities, energy consumption measurements may be converted to estimates of greenhouse gas emissions. The energy consumption of a 5G network and its components could therefore be used as a proxy for greenhouse gas emissions.

### 4.1.5	Legislative frameworks for greenhouse gas reporting

Larger companies in European member states are subject to corporate sustainability reporting under the Corporate Sustainability Reporting Directive (CSRD), and following European Sustainability Reporting Standards which are available under [EUreg] supplementing Directive 2013/34/EU of the European Parliament and of the Council as regards sustainability reporting standards. This reporting law follows the Scopes defined by the Greenhouse Gas Protocol [GGPacc].

Other regions in the world may be subject to other and/or additional reporting requirements.

## 4.2	Related work

[Editor’s note: list the energy efficiency-related activities in 3GPP and elsewhere, e.g. DVB, MPEG…]

[Editor’s note: list of 3GPP TR/TS that could be considered:

SA1:

- TR 22.882: Study on Energy Efficiency as a service criteria (Release 19)

- TS 22.261: Service requirements for the 5G system (Release 19)

SA2:

- TS 23.288: Architecture enhancements for 5G System (5GS) to support network data analytics services (Release 18)

- TR 23.700-66: Study on Energy Efficiency and Energy Saving (Release 19)

SA4:

- TS 26.531: Data Collection and Reporting; General Description and Architecture (Release 18)

- TS 26.532: Data Collection and Reporting; Protocols and Formats; (Release 18)

SA5:

- TS 28.310: Energy efficiency of 5G (Release 18). SA5 umbrella TS for energy efficiency and energy saving aspects.

- TS 28.552: Management and orchestration; 5G performance measurements. Clause 5.1.1.19 defines Power, Energy and Environment (PEE) measurements for 5G Physical Network Functions (PNF)

- TS 28.554: 5G end to end Key Performance Indicators (KPI) (Release 18). Clause 6.7 defines energy consumption and energy efficiency KPIs for 5G Virtualized Network Functions (VNF), network slices, etc.

- TS 28.552 (Management and orchestration; 5G performance measurements), whose clause 5.1.1.19 defines Power, Energy and Environment (PEE) measurements for 5G Physical Network Functions (PNF)

- TS 28.622 (Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)), which defines (non-exclusive list) manageable PEE parameters for 5G Network Functions, performance measurement job (enabling an authorized consumer to collect energy consumption / energy efficiency measurements and KPIs from OAM), etc.

- TS 28.532 (Management and orchestration; Generic management services), which defines APIs that can be used by authorized consumer(s) to e.g. collect energy consumption, energy efficiency measurements and KPIs, and PEE parameters

- TR 28.813: Study on new aspects of Energy Efficiency (EE) for 5G (Release 17)

- TR 28.880: Study on energy efficiency and energy saving aspects of 5G networks and services (Release 19)

- TR 28.913: Study on new aspects of Energy Efficiency (EE) for 5G phase 2 (Release 18)]

### 4.2.1	Introduction

Several standards setting organisations broadly active in the areas of broadcasting and telecommunications are considering energy efficiency and the reduction of climate impact. Likewise, several industry fora are active in this area. This section documents some of the efforts underway, and references standards and reports currently available.

### 4.2.2	3GPP

#### 4.2.2.1	Introduction

#### 4.2.2.2	Collection and exposure of energy consumption information at OAM

##### 4.2.2.1.1	Introduction

This clause summarizes TS 28.554 [28554] as it relates to the evaluation and collection of energy consumption information by the Operations, Administration and Maintenance (OAM) capability of the 5G System, as specified in 3GPP Release 18 by SA WG5.

##### 4.2.2.1.2	Collection of network energy information by OAM

Clause 6.7.3 of TS 28.554 [28554] defines the Energy Consumption KPI of a Physical Node. The network energy information that can be collected by the OAM capability includes that listed in table 4.2.2.1.2-1.

Table 4.2.2.1.2-1: Network energy information collected by OAM

| KPI category | Description | Reference | Clause |
| --- | --- | --- | --- |
| Energy Consumption (EC) information | Energy Consumption of a gNodeB | TS 28.554 [28554] | 6.7.3.4.2 |
|  | Energy Consumption of the NG-RAN |  | 6.7.3.4.1 |
|  | Energy Consumption of the 5GC |  | 6.7.3.2.1 |
|  | Energy Consumption of a 5G Network Function |  | 6.7.3.1.1 |
|  | Estimated Energy Consumption of a Virtualized Network Function |  | 6.7.3.1.2 |
|  | Energy Consumption of a network slice |  | 6.7.3.3 |
|  | Energy Consumption of a Physical Network Function (PNF) as well as other Power, Energy, Environmental (PEE) measurements | TS 28.552 [28552] | 5.1.1.19.2 |
| Energy Efficiency KPIs | Energy Efficiency of the NG-RAN data | TS 28.554 [28554] | 6.7.1 |
|  | Energy Efficiency of the 5GC |  | 6.7.4.1 |
|  | Energy Efficiency of a network slice |  | 6.7.2 |

In the case of a Virtualized Network Function (VNF) hosted on a physical node, the energy consumption of the VNF is estimated as a portion of the total energy consumption of the physical node on which the VNF is executing, based on its relative virtual CPU usage, virtual memory usage, virtual disk usage and I/O traffic (all metrics collected from ETSI MANO) as defined in clause 6.3.1.2 of TS 28.554 [28554].

##### 4.2.2.1.3	Exposure of network energy information by OAM

Network energy information may be collected by OAM and exposed (other mechanisms may exist) as defined by the Service-Based Management Architecture (SBMA) in TS 28.533 [28533]. Any authorised consumer willing to collect such measurements or KPIs is first required by [28533] to create an instance of a performance metrics production job (i.e., an instance of the PerfMetricJob information element – see clause 4.3.31 of TS 28.622 [28622]) by invoking the createMOI operation of the Provisioning Management Service (MnS) (see clause 11.1.1.1 of TS 28.532 [28532]).

The consumer is required by [28533] to specify:

-	Which measurement(s) or KPI(s) it wishes to be collected, e.g. the energy consumption of a 5G NF, etc.

-	Which network entities (represented by managed objects) it wishes the information to be collected from (e.g., SMF x, UPF y, etc.).

-	The granularity period (expressed in seconds) it wishes the measurements or KPIs to be reported over.

-	The reporting method, mainly:

-	File-based reporting: Performance data is accumulated for a certain time before it is reported; the data is delivered as a file; the file content encoding is either XML or ASN.1.

-	Stream-based reporting: The producer sends the performance data to the consumer as when they are ready. The volume of the performance data reported is expected to be small, and the granularity period is expected to be short. The stream content encoding technique is either GPB or ASN.1.

Depending on the selected reporting method, the consumer collects the measurements or KPIs as follows:

-	In the case of file-based reporting, the producer sends the notification NotifyFileReady to subscribed consumer(s) when a new file becomes available on the producer for subsequent download by consumer(s).

-	In the case of stream-based reporting, the producer sends units of streaming data to the consumer by invoking the reportStreamData operation.

#### 4.2.2.3		Collection and exposure of energy consumption information at NF

TR 23.700-66 [23700-66] studies and identifies potential enhancements to the 5G System (e.g., including network energy-related information exposure, enhancement for subscription and policy control to enable energy efficiency as a service criterion) to improve energy efficiency and to support energy saving in the network.

Three different key issues have been identified in that study:

-	KI#1: Network energy related information exposure

-	KI#2: Subscription and policy control to support energy efficiency and energy saving as service criteria

-	KI#3: 5GS enhancements for network energy saving and efficiency

KI#2 is not in scope of this study. The conclusions of KI#1 and KI#3 in clause 8 of TR 23.700-66 [23700-66] and the normative work following will be used for collection and exposure of Energy Consumption information at Network Functions (NFs) and are summarised as follows:

1.	A new network functionality will be defined to collect and calculate energy-related information and expose it to authorised consumers subject to operator's policy:

-	If the authorised consumer is a 5GC Network Function, the information exposure granularities that can be configured in this policy will include per application, per UE, per-UE-per-QoS Flow, per PDU session.

-	If the authorised consumer is an Application Function, the information exposure granularities that can be configured in this policy will include: per UE, per UE per application, per PDU session.

NOTE:	Whether the new network functionality can be deployed as a standalone Network Function or co-located with other Network Functions is for further study outside the scope of the present document.

2.	The energy-related information that can be exposed according to the above exposure granularities will include:

-	Energy Consumption information as defined in TS 28.310 [28310].

-	Renewable energy information defined as energy from renewable non-fossil sources. For example (but not limited to) wind, solar, aerothermal, geothermal, hydrothermal.

3.	A consumer of energy-related information (i.e., 5GC NF or AF) may request different modes of exposure (e.g. periodic reporting or threshold-based reporting) as part of its subscription request.

4.	The new network functionality supporting the calculation of the Energy Consumption information includes the following aspects:

- OAM: provides the NF/Node-level Energy Consumption information at the gNB(s) and UPF(s) serving the UE.

- OAM: provides the overall data volume of the gNodeB.

- The information of a) and b) received from OAM could be used by the new network functionality for all the UEs served by the NF/Node.

- UPF: provides the overall data volume of the UPF.

- UPF: provides the data volume for the QoS Flow or the Service Data Flow (SDF).

- When the gNodeB and/or the (I-)UPF(s) which are serving the UE change, the serving gNodeB ID and UPF ID will be sent to the new network functionality through AMF/SMF.

5.	The new network functionality determines the end-to-end energy consumption based on energy consumption per the granularities above at the serving Network Function (i.e. NG-RAN and UPF).

6.	In Release 19, only the energy-related information of user plane communication (not control plane signalling) is supported.

7.	Enhancements to NF discovery and (re-)selection procedures based on energy-related information:

- 	The NF profile may be extended (e.g. by including the new energy-related information or by reusing existing NF profile parameters) to allow an operator to influence NF discovery and selection based on its energy strategy.

- 	NF discovery and (re-) selection will be enhanced to consider the energy-related information from the NF profiles and/or discovery requests from the NF consumer.

8.	Enhancements to existing operations and procedures for energy saving and energy efficiency:

-	The User Plane path of a PDU session may be adjusted so that it consumes less energy.

The recommendations of the present document focusing on media services will need to be aligned with the conclusions in clause 8 of TR 23.700-66 [23700-66] impacting Application Functions used for media services.

#### 4.2.2.4	UE data collection, reporting and event exposure

##### 4.2.2.4.1	UE data collection, reporting and event exposure architecture

A generic architecture for the collection and reporting of UE data is defined in TS 26.531 [26531] and the corresponding APIs are specified in TS 26.532 [26532] but UE energy consumption has not been considered in these specifications up to and including in Release 18. Subject to study in the present document, a potential solution would be to expand the scope of these specifications to support the collection and exposure of energy consumption information at UE.

The main principle of the reference architecture defined in clause 4 of TS 26.531 [26531] and reproduced in figure 4.2.2.4-1 below is the addition of an intermediary Application Function named the Data Collection AF which is used collected UE data reports from data collection clients and Application Servers, and to synthesise from those reports a set of events which are exposed to event consumer subscriber, such as the Network Data Analytics Function (NWDAF) or an Event Consumer AF deployed in the Application Service Provider.

Figure 4.2.2.4.1-1: Reference architecture for data collection and reporting

Data collection and reporting functionality in the Data Collection AF is provisioned at reference point R1 by a Provisioning AF of the Application Service Provider that may be deployed either inside or outside the trusted domain. The purpose of the Data Collection AF is to receive UE data reports from one of three possible sources:

1.	Directly from the UE. In this case the Direct Data Collection Client is responsible for collecting relevant data in the UE (typically from a UE Application using a suitable API at reference point R7) and for sending data reports to the Data Collection AF via reference point R2.

2.	Indirectly from the UE. In this case, an Application Service Provider collects data from UE Applications privately via reference point R8 and employ an Indirect Data Collection Client subfunction to then send data reports to the Data Collection AF via reference point R3.

3.	From an Application Server that has been used to deliver media to/from a UE. Application Server instances (AS) inside or outside the trusted domain may also collect data and report it to the Data Collection AF via reference point R4.

The Data Collection AF aggregates and filters UE data that is reported to it. The processed UE data is exposed by the Data Collection AF to the NWDAF in the form of data reporting event notifications via reference point R5. Certain UE data may also be exposed in the form of data reporting events by the Data Collection AF to an Event Consumer AF residing in the Application Service Provider via reference point R6.

When they are deployed in different trust domains, the interactions between the system actors of the UE data collection, reporting and event exposure architecture may be mediated through the NEF, as illustrated by various collaboration scenarios defined in annex A of TS 26.531 [26531].

##### 4.2.2.4.2	UE data collection, reporting and event exposure for 5G Media Streaming

The instantiation of the UE data collection, reporting and event exposure architecture in the 5GMS System is defined in clause 4.7 of TS 26.501 [26501] and the reference architecture for this instantiation is reproduced in figure 4.2.2.4-2.

Figure 4.2.2.4.2-1: Data collection and reporting architecture instantiation for 5G Media Streaming

Three existing 5GMS reference points are reused in this instantiation: M1 (for provisioning UE data collection, reporting and event exposure), M5 (for media session handling) and M6 (for interacting with the Media Session Handler).

-	The Provisioning AF of the Application Service Provider is not instantiated in the 5GMS architecture. Data collection and reporting is instead provisioned using the procedures using M1 defined in TS 26.501 [26501].

-	The Data Collection AF for 5G Media Streaming is instantiated in the 5GMS AF.

-	The Direct Data Collection Client for 5G Media Streaming is instantiated in the Media Session Handler. This takes logical responsibility for the UE data collection activities of the Metrics Collection & Reporting and Consumption Collection & Reporting subfunctions and the subsequent reporting of this UE data via reference point M5. It also takes logical responsibility for the logging of ANBR-based Network Assistance invocations by the Network Assistance subfunction and their subsequent reporting to the Data Collection AF instantiated in the 5GMS AF via reference point R2.

NOTE:	The use of M5 instead of R2 is due to the definition of some procedures before the definition of this instantiation. M5 was already used for QoE metrics and consumption reports, or invocations of the downlink dynamic policy procedures and invocations of the AF-based downlink Network Assistance procedures. Thus, for these procedures, M5 is used instead of R2.

-	The Indirect Data Collection Client is not instantiated in the 5GMS architecture. Indirect reporting of UE data is outside the scope of 5G Media Streaming. Thus, R8 is not instantiated in the 5GMS architecture.

-	The role of the AS in the abstract reference architecture is played by 5GMS AS. (This may be deployed as a trusted AS within the 5G System or deployed externally.)

-	The Event Consumer AF is instantiated in the 5GMS Application Provider as a consumer of 5G Media Streaming events from the Data Collection AF.

-	Reference point R7 is not instantiated in the 5GMS architecture. Configuration of 5GMS-related data reporting in the Media Session Handler by the 5GMS-Aware Application is managed through the existing media session handling client API at reference point M6.

##### 4.2.2.4.3		QoE Measurement Collection (QMC) functionality

TS 38.300 [38300] defines the QoE Measurement Collection (QMC) feature which enables collection of application layer measurements from the UE. QMC is supported for the following service types in NR cells:

-	QoE Measurement Collection for DASH streaming services in TS 26.247 [26247];

-	QoE Measurement Collection for MTSI services in TS 26.114 [26114];

-	QoE Measurement Collection for VR services in TS 26.118 [26118].

The QMC feature also supports collection of QoE measurements for any of the supported service types carried by the MBS communication service defined in TS 23.247 [23247], namely:

-	MBS broadcast;

-	MBS multicast.

More details of QMC control and configuration can be found in TS 28.405 [28405]. A potential solution would be to reuse and expand the QMC functionality to support the reporting of energy consumption information by the UE.

##### 4.2.2.4.4	Potential use of UE data collection, reporting and event exposure architecture for collection and exposure of energy consumption information

[Editor’s note: Data collection and reporting architecture seems a good basis for collection and exposure of energy consumption information at UE, but a series of questions arises in connection with reaching this goal:

1.	What energy-related information will be provided by the UE? Is this information related only to the communication part or media part?

2.	Which UE function will be the source of the energy consumption information (e.g., per-application energy usage data collected by a system-level service running on the UE, or contributed by individual applications running on the UE reporting their own energy usage data)?

3.	How should UE energy consumption data be reported by a UE to the 5G System?

a)	Reuse the existing 5GMS consumption reporting mechanism by extending relevant data type(s) specified in TS 26.510 [26510] to report UE energy consumption data?

b)	Reuse the existing 5GMS QoE metrics reporting mechanism and reporting format by extending the QM10 XML schema schema specified in clause 10.2 of TS 26.247[26247] to report UE energy consumption data?

c)	Reuse the existing 5GMS QoE metrics reporting mechanism but specify a brand new QoE metrics reporting scheme in TS 26.510 [26510] to report UE energy consumption data?

d)	Specify a new generic UE data domain in TS 26.531 [26531] and a JSON-based reporting format in TS 26.532 [26532] to report UE energy consumption data to the generic Data Collection AF?

4.	In the context of options 3(c) and 3(d), can commonly supported metrics and client data reporting format and protocols like CTA-WAVE CMCD [CTA-5005-A] be used to cover the required scope?

NOTE:	The parallel study of future reporting mechanisms and formats in TR 26.804 is relevant in the context of question 4.

5.	Which reference points should be used to report UE energy consumption data to the Data Collection AF?

a)	If using the generic UE data collection, reporting and event exposure architecture defined in TS 26.531 [26531], data could be reported through reference points R2, R3 and/or R4.

b)	If using the 5GMS instantiation of the generic UE data collection, reporting and event exposure architecture defined in clause 4.7 of TS 26.501 [26501], the Indirect Data Collection Client is not available in Release 18, so only R2 (or M5) and R4 are available for data reporting. Are there use cases requiring reference point R3? Is it relevant to be able to use several interfaces?

6.	Would it be useful to expose energy-related information of the network to the Media Session Handler to help it optimize its media session in an energy-efficient way?]

### 4.2.3	Other Standards Development Organisations

#### 4.2.3.1	ITU-T

Within the International Telecommunications Union, the T-sector includes Study Group 5 "Environment and Circular Economy" (SG5). Part of its mandate is to define and develop “methodologies for evaluating ICT effects on climate change and publishing guidelines for using ICTs in an eco-friendly way. Under its environmental mandate SG5 is also responsible for studying design methodologies to reduce ICT’s and e-waste’s adverse environmental effects, for example, through recycling of ICT facilities and equipment.”

Among its activities, ITU-T Study Group 5 is developing technical reports, supplements and international standards for the environmental requirements of 5G [L.1210..L.Sup43]. Further, the L.1400 series of reports and recommendations present methodologies and guidelines for the assessment of the greenhouse gas emissions and energy consumption of the ICT sector. For example, ITU-T L.1450 presents a methodology for the assessment of the impact of telecommunications systems [L.1450]. It was used in an assessment of the electricity usage and greenhouse gas emissions of the ICT sector [ICT].

#### 4.2.3.2	ITU-R

The remit of ITU-R Study Group 6 (SG6) is programme production and interchange. Its Working Party 6A (WP 6A) has published ITU-R Report BT.2385 "Reducing the environmental impact of terrestrial broadcasting systems" [BT2385]. Working Party 6C (WP 6C) has a rapporteur group which has produced the following documents:

-	ITU-R Opinion 104, "Advice for sustainability strategies incorporating carbon offsetting policies" [OP104]

-	ITU-R Report BT.2521, "Practical examples of actions to realize energy aware broadcasting" [BT2521]. This report is based on a webinar held in March 2022.

- 	ITU-R Report BT.2540, "Display energy reduction through image signal processing" [BT2540]. This document describes techniques for producing, transmitting and using metadata which enables display devices to use less energy.

#### 4.2.3.3	MPEG

The ISO/IEC JTC 1/SC 29 committee Coding of audio, picture, multimedia and hypermedia information has published the ISO/IEC 23001-11:2023 (Green MPEG) standard [23001-11]. The various components of the standard define methods for the reduction of the power consumption of decoders and of displays. A further component defines a method for the selection of energy-efficient media. A final method allows for quality recovery after low-power encoding. The standard is currently in revision, and it is to be extended to enable the carriage of metadata to more efficiently reduce the power requirements of display devices receiving the content with the metadata.

#### 4.2.3.4	DVB

DVB has carried out a study mission to assess the potential for developing energy-efficient video transmission systems. This work has resulted in the creation of a new CM-EE (Energy Efficiency) working group in its Commercial Module. It has also published a report on the topic available as Blue Book S100 "Study Mission report on Energy Aware service Delivery and Consumption" [S100].

#### 4.2.3.5	ATSC

ATSC's "Planning Team 9 – Sustainability in Media and Data Delivery Services (PT9) will study the benefits of broadcast data delivery as relates to sustainable energy usage in a world increasingly dependent on data delivery. The team will consider linear and file-based media delivery as well as linear and file-based data delivery. PT9 will report the results of this work to the Board. If technical work in ATSC is recommended, PT9 will further document rationale for the work and ideally also document possible architectural approaches and requirements, such as interoperability with existing networks, which would accommodate the identified use cases. PT9 does not draft standards or recommended practices; it may draft New Project Proposals and/or Planning Team Reports. PT9 reports to the ATSC Board of Directors and participation is open to all ATSC members." [PT9].

### 4.2.4	Industry Fora

#### 4.2.4.1	Greening of Streaming

Greening of Streaming is a member association investigating energy efficiency in the context of media streaming applications [GoS]. One of the challenges the group is aiming to address is that of accurately measuring the energy expenditure of streaming services, given that currently the available data is sparse and not very precise. It further intends to define best practices.

#### 4.2.4.2	DIMPACT

“DIMPACT is a collaborative initiative between leading media, entertainment and technology companies and world-class researchers [DIMPACT].” The group is convened by Carnstone Partners Ltd, and research and technical expertise is provided by researchers from the University of Bristol. It has currently over 20 members. The group has developed a tool to measure the emissions of serving digital media and entertainment products. This tool is available as a web application and is able to estimate emissions originating from video streaming, online banner advertising, digital publishing, and audio streaming. The DIMPACT website makes available several publications explaining their methodology [DTMeth] and defining principles for streaming and digital media carbon footprinting [DTPaper].

#### 4.2.4.3	Ultra HD Forum

The Sustainability Working Group of the Ultra HD Forum is investigating energy efficiency opportunities throughout content distribution, from content encoding through distribution and display. One result has been a regular series of public demonstrations directed to sustainability at major broadcast conferences (specifically IBC and NAB) where the Ultra HD Forum regularly has a booth [UHDF]. Some of these demonstrate the degree of influence the consumer can have on the energy consumption by the display.

### 4.2.5	Greenhouse Gas Protocol

The Greenhouse Gas Protocol [GGPweb] is an organisation that “provides standards, guidance, tools and training for business and government to measure and manage climate-warming emissions.” The first edition of their reporting standard was published in 2001, establishing a reporting framework for businesses. Relative to a given company, the concept of “scopes” is introduced, which delineate direct and indirect emission source, and are used for accounting and reporting purposes. The reporting principally involves the six greenhouse gasses that are defined in the Kyoto protocol: carbon dioxide (CO2), methane (CH4), nitrous oxide (N2O), hydrofluorocarbons (HFCs), perfluorocarbons (PFCs) and sulphurhexafluoride (SF6). Scopes 1 (sources owned or controlled by a company giving rise to direct greenhouse gas) and 2 (the electricity purchased and consumed by a company gives rise to greenhouse gas emissions) are defined in [GGPacc], and Scope 3 (All other indirect emissions) is defined in [GGPsc3].

NOTE:	3GPP SA5 is also considering the Greenhouse Gas Protocol as part of TR 28.913 “Study on new aspects of Energy Efficiency (EE) for 5G phase 2” [28913]

#### 4.2.5.1	Scope 1

Sources owned or controlled by a company give rise to direct greenhouse gas emissions. The activities undertaken by a company that give rise to scope 1 emissions include the generation of electricity, heat, or steam; physical or chemical processing; transportation of materials, products, and waste; fugitive emissions.

#### 4.2.5.2	Scope 2

The electricity purchased and consumed by a company gives rise to greenhouse gas emissions. Scope 2 emissions occur at the facility where the electricity is generated, rather than where the electricity is consumed. For the reporting company, these emissions are therefore counted as one form of indirect emissions. As purchased electricity is for many companies one of the largest sources of greenhouse gas emissions, it also offers a significant potential for reductions, either by investing in energy efficient technologies, by energy conservation, or by switching to less greenhouse gas intensive sources of electricity.

#### 4.2.5.3	Scope 3

All other indirect emissions can be reported under scope 3. The emissions reported in this category are the consequence of the activities of a company, but they come from sources not owned or controlled by this company. These indirect emissions arise elsewhere in the corporate value chain of a given company, both upstream and downstream. Upstream emissions are indirect emissions relating to purchased or acquired goods and services. Downstream emissions relate to sold goods and services. Scope 3 indirect emissions are categorised into 15 distinct categories, of which the upstream categories are:

- Purchased goods and services. Extraction, production, and transportation of goods and services purchased or acquired by the reporting company in the reporting year, not otherwise included in categories 2 to 8.

- Capital goods. Extraction, production, and transportation of capital goods, purchased or acquired by the reporting company in the reporting year.

- Fuel- and energy-related activities (not included in scope 1 or scope 2). Extraction, production, and transportation of fuels and energy purchased or acquired by the reporting company in the reporting year.

- Upstream transportation and distribution. Transportation and distribution of purchased products or services in the reporting year, including inbound and outbound logistics; transportation between a company’s own facilities.

- Waste generated in operations. Disposal and treatment of waste generated by the reporting company’s operations in the reporting year.

- Business travel. Transportation of employees for business-related activities in the reporting year in vehicles not owned or operated by the reporting company.

- Employee commuting. Transportation of employees between their homes and their worksites during the reporting year, in vehicles not owned or operated by the reporting company.

- Upstream leased assets. Operation of assets leased by the reporting company in the reporting year.

The downstream categories are:

- Downstream transportation and distribution. Transportation and distribution of products sold by the reporting company in the reporting year between the reporting company’s operations and the end consumer, if not paid for by the reporting company (in vehicles and facilities not owned by the reporting company).

- Processing of sold products. Processing of intermediate products sold in the reporting year by the downstream companies.

- Use of sold products. End use of goods and services sold by the reporting company in the reporting year.

- End-of-life treatment of sold products. Waste disposal and treatment of products sold by the reporting company (in the reporting year) at the end of their life.

- Downstream leased assets. Operation of assets owned by the reporting company and leased by other companies in the reporting year.

- Franchises. Operation of franchise in the reporting year.

Investments. Operation of investments (including equity and debt investments and project finance) in the reporting year.

# 5	Use cases

## 5.1	Baseline use cases defined by SA1

Use cases regarding enhancements to Energy Efficiency of 5G network and application service enabler aspects are listed in TR 22.882 [22282]. Five of them have been identified as media-related and therefore fall within the scope of this study:

-	Use case 5.5 on service energy monitoring by an Application Server: The Application Service Provider cares about energy consumption in the Data Network as a result of the service provided by an Application Server to UEs. This could be for one or more of the following three reasons:

-	The Application Service Provider needs to demonstrate that it is reducing energy consumption;

-	The service has an associated energy cost, and the Application Service Provider wants to reduce it;

-	The Application Service Provider recognises that there are policies that limit energy use and controls the overall use of the service to operate within those constraints.

-	Use case 5.6 on on supporting service-level energy efficiency analysis for verticals: An Application Service Provider is running three different enterprise applications over two network slices. It proposes exposure of data volume and energy consumption of different Network Functions participating in the delivery of the service for different time periods at the request of the Application Service Provider. The Application Service Provider may use existing 3GPP procedures to infer Network Slice energy consumption and the number of PDU sessions per network slice.

- 	Use case 5.8 on Application service Energy Efficiency (AEE) monitoring: The energy consumed by an application service at the device side as well as at the network side is monitored and predicted by the 5G System and is exposed as a monitoring event to the Application Service Provider to allow an application layer action. In the context of media delivery, this action could be for example trigerring multicast/broadcast delivery for a given service area and time of the day.

-	Use case 5.9 on renewable energy consumption information exposure: Mobile Network Operators need to understand and track the proportion of energy consumed in their networks that is sourced from renewable sources, which can be made available to customers and authorized third parties.

-	Use case 5.10 on supporting carbon-aware communication service: The Mobile Network Operator provides to end users an estimate of the carbon emissions for the services consumed, for example the equivalent carbon dioxide emissions corresponding to the data consumed by a user during a particular billing cycle.

-	Use case 5.14 on reducing GHG footprint of Application Services: By considering the temporal and spatial information of sustainable energy source and availability, the possibility of reduction of the greenhouse gas footprint for application services is explored. Rather than optimising compute tasks for highest throughput or lowest latency, those tasks having flexibility in both when and where they are executed (e.g., some AI/ML training or video processing) are routed to a computing node using the (most) sustainable energy sources at that moment.

Media-related requirements associated with these use cases are addressed in the following Key Issues, complemented by requirements associated with the findings identified in clause 4.

## 5.2	Refinement of relevant use cases defined by 3GPP SA1 in 3GPP SA4 context

## 5.3	Additional use cases defined by 3GPP SA4

Use cases defined by SA1 on energy monitoring or energy consumption information exposure are not yet taken into consideration in 26.XXX series specifications but similar use cases have already been addressed. As explained in clause 4.2.2.4, some mechanisms like UE data collection, reporting and event exposure have already been defined. For consistency between specifications, supporting the collection and exposure of UE energy consumption information will require expansion of these existing mechanisms or else the use of mechanisms widely deployed in the market. This expansion will have to take into consideration indicators requested by regulators.

In France, the “Chaize Act” on reinforcing regulation of the digital sector by Arcep, strengthens Arcep’s powers by giving it the ability to collect environmental data not only from electronic communications operators, but also from online communication service providers, data centre operators, consumer device manufacturers, network equipment suppliers and operating system providers. Arcep (France’s Regulatory Authority for Electronic Communications, Postal Affairs and Press Distribution) has been collecting indicators since 2020 from France’s four largest telecoms operators, to be able to track the evolution of their environmental footprint, and relays this information through the publication of its annual “Achieving digital sustainability” survey [ARCEP]. The fourth edition of the survey, which Arcep will be publishing in early 2025, will incorporate data for monitoring the environmental footprint of a new category of player, namely mobile network equipment suppliers. This work has been complemented by ARCOM (the French Regulatory Authority for Audiovisual and Digital Communication) in its recommendation n° 2023-02 about consumer information on energy consumption and greenhouse gas emissions equivalents of data consumption related to the use of television services, on-demand audiovisual media services and video sharing platform services.

In addition to collecting energy consumption information from UEs and exposing it to event consumers, ARCOM encourages collection and exposure of energy consumption information to UEs could help to address the energy efficiency issue. This would be used to inform users about the environmental impact of consuming audiovisual content, but this information could also be used by UEs to optimise energy efficiency associated with media consumption.

ARCOM also encourages service providers to offer access to video quality parameter settings, allowing an easy way for end users to choose a simple “energy efficiency” mode. Instead of always being in a “best video performances according to network conditions regardless energy consumption” mode, having data on the QoE and energy consumption could enable a second mode to be offered to users representing reasonably good video performance with good energy efficiency. Having this information, instead of having a manual selection of video bit rates or SDR/HDR modes, the 5GMS Client could automatically select the best compromise to offer this additional mode to the users.

Regulators like ARCOM also encourage TV services, VOD services, video sharing plaforms and other actors in the sector to put in place a common methodology for calculating the environmental impact of audiovisual uses. This work will have to be follow as the study item plans to study the feasibility of having implementation-independent metrics and a framework to evaluate the energy usage/savings of multimedia standards features and proposals.

# 6	Key issues

[Editor’s note: Description of key issues and their potential requirements]

## 6.1	Key Issue #1: Information exposure

### 6.1.1 	Description

As described in the use cases summarised in clause 5.1, energy-related information needs to be collected, estimated, and exposed by the 5G System. This information is not only necessary for internal network optimisation, but it will also benefit service adaptation by third-party Application Service Providers. With the consent of Mobile Network Operators, it is relevant to expose energy-related information (e.g., energy consumption information, energy efficiency information, renewable energy and carbon emission information) to authorised consumers. Network energy information exposure has already been studied in TR 23.700-66 [23700-66]. The purpose of this Key Issue is to extend this work, taking into consideration the media context (e.g., the 5G Media Streaming System according to TS 26.501 [26501], 5G Multicast–Broadcast User Services according to TS 26.502 [26502], the Real-time Media Communication according to TS 26.506 [26506]) including UE-related energy information exposure. As explained in clause 4.2.2, a UE data collection and reporting architecture already exist in the 5G System. But energy-related information is missing.

In this context, the subsequent analysis by this Key Issue should consider:

1.	How should UE energy-related information be reported by a UE to the 5G System

2.	Which reference points should be used to report UE energy-related information to the Data Collection AF

3.	Would it be useful to expose energy-related information of the network to the Media Session Handler to help it optimize its media session in an energy-efficient way

[Editor’s note: Additional questions will probably to be added when other 4.2 clauses will be completed]

This analysis will need to take into consideration existing work done in 3GPP but also other market trends. Application Service Providers are often reluctant to deploy solutions specific to mobile networks. Proposing technologies already supported in their services or technologies that are agnostic to the network, or which operate passively without the active involvement or knowledge of applications, are more likely adopted.

### 6.1.2	Potential requirements

Subclause 6.4 in [22882] contains the consolidated requirements extracted from use cases, related to information exposure related with this Key Issue:

[CPR 6.4-1] Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to expose information on energy consumption forserving this 3rd party. [CPR 6.4-2] Subject to operator’s policy, the 5G system shall support a means to expose energy consumption to authorized third parties for services, including energy consumption information related to the condition of energy credit limit (e.g. when the energy consumption is reaching the energy credit limit). [CPR 6.4-3] Subject to operator policy, the 5G system shall provide means for the trusted 3rd party, to configure which network performance statistic information (e.g. the data rate, packet delay and packet loss) for the communication service provided to the 3rd party, needs to be exposed along with the information on energy consumption for serving this 3rd party. [CPR 6.4-4] Based on operator policy and agreement with 3rd party, the 5G system shall be able to expose energy consumption information and prediction on energy consumption of the 5G network per application service to the 3rd party. [CPR 6.4-5] Subject to operator’s policy and agreement with 3rd party, the 5G system shall support a mechanism for the 3rd party to provide current or predicted energy consumption information over a specific period of time.

Additional potential requirements identified from related work:

[PR 1-1] Where possible, it is required to reuse existing mechanisms (e.g., UE data collection and reporting architecture as in TS 26.531 [26531]) and information for exposure of energy-related information. [PR 1-2] It is required to reuse commonly supported client data reporting formats for energy-related information exposure when possible. [PR 1-3] It is required that the 5GMS Client is able to obtain energy-related information from the UE, allowing it to optimise the media delivery sessions it is handling in an energy-efficient manner.

[Editor’s note: Additional Potential Requirements will probably to be added when other 4.2 clauses will be completed]

## 6.2	Key Issue #2: Monitoring and measurement

### 6.2.1 	Description

To facilitate efficient energy use and energy saving for media services, first, measurements are useful. Environmental impact of the consumption of media services depends various actors: device manufacturer, networks, datacentres, service providers but also users through their usage behaviour.

Requirements of monitoring and measurement related to energy consumption and efficiency have already been defined in TR 22.882 [22882] but those requirements are only related to energy consumption monitoring of 5G network or 5G system. Network energy information measurement and monitoring have already been studied in TR 23.700-66 [23700-66]. The purpose of this Key Issue is to extend this work, taking into consideration the media context (e.g., the 5G Media Streaming System according to TS 26.501 [26501], 5G Multicast–Broadcast User Services according to TS 26.502 [26502], the Real-time Media Communication according to TS 26.506 [26506]) including UE-related energy information measurement and monitoring. The more efficient way to address energy saving would be to consider the end-to-end chain, seeking to understand how decisions made on one end of the chain impact power consumption on other ends of the workflow. As explained in section 4.2.2, UE data collection has been defined to monitor QoE in the 5G system. But energy-related information is missing.

NOTE 1:	It is presumed that the user has granted consent for its UE data to be collected, reported and subsequently exposed by means outside the scope of the study or following normative work.

NOTE 2:	The collection, reporting and exposure of location-based UE data is expected to comply with regional regulatory requirements and may be further limited by MNO policy.

In this context, the subsequent analysis by this Key Issue will consider:

- Which UE energy-related information will be collected to measure, correlate, and optimize energy usage across the entire streaming distribution chain?

- Which UE entity is appropriate to measure this UE energy-related information?

This issue is even more important for advanced media services such as XR services, Split Rendered media services, etc. which are expected to incur substantial energy consumption both at the device and network levels, presenting significant challenges for operators and service providers.

For instance, when a Mobile Network Operator (MNO) deploys a communication service to fulfil application service requirements, such as those of a gaming application, it is crucial for the customer – whether an Application Service Provider (ASP) or an industry vertical – to ensure that the application service reduces energy consumption for both end users and the data network. Failure to do so could necessitate short-notice application layer adaptations within the Application Service Provider's domain. This may result in adjustments being made to service levels in response to anticipated high energy consumption in specific service areas or during peak hours that adversely affect the Quality of Experience for service users.

In a practical scenario, an ASP intends to deploy a gaming service within a designated service area, served by an MNO's 5G network. Various service levels may exist, each associated with specific Key Performance Indicators (KPIs), such as automation levels or video quality targets. To ensure energy efficiency the ASP monitors the energy efficiency of its application service across specified service levels. Monitoring of application energy consumption may occur periodically or may be event-triggered, depending on the ASP's requirements, which are typically outlined in the Service Level Agreement (SLA). The MNO and ASP may agree on a certain energy efficiency target for the application service and optionally for given service levels.

This analysis will need to take into consideration existing work done in 3GPP but also other market trends. Application Service Providers are often reluctant to deploy solutions specific to mobile networks. Proposing technologies already supported in their services or technologies that are agnostic to the network, or which operate passively without the active involvement or knowledge of applications, are more likely adopted.

### 6.2.2	Potential requirements

Subclause 6.3 in [22882] contains the consolidated requirements extracted from use cases, related to monitoring and measurement related with this Key Issue:

[CPR 6.3-1] Subject to operator's policy, the 5G network shall support energy consumption monitoring at per network slice and per subscriber granularity. NOTE 1: Energy consumption monitoring as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirement does not imply that some form of 'real time' monitoring is required. The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services. [CPR 6.3-2] Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to monitor energy consumption for serving this 3rd party, independently from NG-RAN deployment scenarios. NOTE2: The granularity of energy consumption measurement could vary according to different situations, for example, when several services share a same network slice, etc. NOTE 3: The energy consumption information can be related to the network resources of network slice, NPNs, etc. [CPR 6.3-3] Subject to operator policy and regulatory requirements, the 5G system shall be able to monitor the energy consumption for serving the 3rd party, together with the network performance statistic information for the services provided by that network, through same update rate e.g. hourly or daily. NOTE 4: The network performance statistic information could be the data rate, packet delay and packet loss, etc.

Based on TS 28.310 [28310] and other related work in 3GPP, the following potential requirements need to be considered in this Key Issue:

[PR 2-1]	Where possible, it is required to reuse existing mechanisms (e.g., UE data collection and reporting architecture as in TS 26.531 [26531]) and information for measurement and monitoring of energy-related information.

[PR 2-2]	It is required to reuse commonly supported client data metrics for energy-related information measurement and monitoring when possible.

[PR 2-3]	Based on the collected and/or predicted energy efficiency information exposed from the 5G System, the Application Service Provider is able to adapt the application service parameters based on the 5GS feedback. Monitoring of application energy consumption may occur periodically or may be event-triggered, depending on the ASP's requirements, which are typically outlined in the Service Level Agreement (SLA). In order to perform such operations, the Application Service Provider requires relevant APIs to be exposed by participating entities related to the usage of media applications in order to expose Energy efficiency related information. For example, in the case of the 5G Media Streaming collaboration described in clause A.2 of TS 26.501 [26501], this depends on the relationship between 5GMS Application Provider, 5GMSd-Aware Application, 5GMSd Client, 5GMSd AF, and 5GMSd AS.

## 6.3	Key Issue #3: Evaluation framework

### 6.3.1 	Description

The complexity of the media delivery chain raises technical challenges in terms of environmental impact measurement and accountability. There is no explicitly and uniformly defined methodology to evaluate the energy usage/savings of multimedia standards features and proposals which leads to significant variation in the results of different studies. Regulators and legislatures like the European Parliament have already provided more precise guidelines, targeting companies or services, to assess cases of unfair environmental advertising [EmpCo].

A characterisation framework to evaluate the energy usage/savings of multimedia standards features and proposals for relevant 3GPP scenarios is also needed to justify claims. The framework presented in this Key Issue is based on the normative work (e.g., granularities or energy-related information) recommended by Key Issues #1 and #2 of the present document, and includes a set of relevant scenarios and anchors. It is not dependent on a particular implementation of the feature or other parts of the system and allows an energy-efficient implementation to be compared against an implementation not relying on the proposed standards feature. The commonly supported external estimation frameworks mentioned in clauses 4.2.3 and 4.2.4 are to be reviewed and agreed by SA4 before using them to develop the evaluation framework in this Key Issue.

### 6.3.2	Potential requirements

[PR 3-1]	Based on the APIs, metrics, and mechanisms available in the 5G System, it is required that a framework is defined to evaluate the energy usage/savings of multimedia standards features and proposals for relevant 3GPP scenarios on which anchors are defined.

[PR 3-2] 	It is required that standardized approaches to estimate the energy usage/savings that have been reviewed and agreed by 3GPP SA4 are used in the evaluation framework.

# 7	Potential Solutions

[Editor’s note: Description of potential solutions]

## 7.1 	Mapping of Solutions to Key issues

## 7.X	Solution #<X>: <Solution Title>

### 7.X.1	Key issue mapping

### 7.X.2	Functional Description

### 7.X.3	Procedures

### 7.X.4	Impacts on existing services, entities and interfaces

# 8	Conclusions and proposed next steps

[Editor’s note: This clause will list conclusions that have been agreed in the study.]

[Editor’s note: Potential TR Conclusions proposed by Qualcomm for KI#3:

We believe that developing an evaluation framework is going to take considerable work and more study to agree on a robust implementation-independent methodology for evaluating proposed standards features. Thus far, the potential requirements in this section are not sufficient to recommend normative work in this area.  When developing conclusions, it would be more appropriate to recommend that more study is needed to develop a good evaluation framework.]

Annex <A>:
<Informative annex title for a Technical Report>

Informative annexes in Technical Reports do not use "(informative") in the title, since all annexes in TRs are informative. Use style "Heading 9" in TRs.

Annex <B> (informative):
Change history

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

This is the last annex for TS/TSs which details the change history using the following table.
This table is to be used for recording progress during the WG drafting process till TSG approval of this TS/TR.
For TRs under change control, use one line per approved Change Request
Date: use format YYYY-MM
CR: four digits, leading zeros as necessary
Rev: blank, or number (max two digits)
Cat: use one of the letters A, B, C, D, F
Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover
New vers: use format [n]n.[n]n.[n]n

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-03 | SA4#127-bis-e | S4-240565 |  |  |  | TR skeleton | 0.0.1 |
| 2024-04 | SA4#127-bis-e | S4-240816 |  |  |  | Addition of related work (S4-240803) | 0.1.0 |
| 2024-05 | SA4#128 | S4-240875 |  |  |  | Addition of Collection and exposure of energy consumption information by OAM (S4aI240057) | 0.1.1 |
| 2024-05 | SA4#128 | S4-241243 |  |  |  | Addition of Description of the existing collection and exposure of energy consumption information at UE (S4-241212), Context information on greenhouse gas reporting laws, protocols and framework (S4-241218), Network energy use (S4-241220), UE energy consumption information reporting (S4-241224), KI#1 (S4-241228) | 0.2.0 |
| 2024-08 | SA4#129-e | S4-241409 |  |  |  | Addition of Key Issue #2: Monitoring and measurement (S4aI240112) | 0.2.1 |
| 2024-08 | SA4#129-e | S4-241736 |  |  |  | Addition of Additional use cases defined by SA4 (S4-241718), related work in ITU-R (S4-241431), energy-related information amendment (S4-241432), Description of the existing collection and exposure of energy consumption information at NF (S4-241722) and KI#3 (S4-241732). | 0.3.0 |
| 2024-10 | SA4-e (AH) MBS SWG post 129e | S4aI240172 |  |  |  | Addition of Correction on collection and exposure at NF (S4aI240173), Complements on KI#2 (S4aI240142). | 0.3.1 |