| 3GPP TR 28.828 V1.3.0 (2023-03) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on charging aspects for enhanced support of non-public networks (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Background	9

4.1	General	9

4.2	Non-public networks functionality and architecture	10

4.2.1	Stand-alone Non-Public Network (SNPN)	10

4.2.2	Public Network Integrated NPN (PNI-NPN)	11

4.3	Charging modes	11

5	Charging scenarios and key issues for SNPN	12

5.1	Topic 1: Converged charging for data connectivity in SNPN	12

5.1.1	Use cases	12

5.1.1.1	Use case #1a: End users access the SNPN service directly	12

5.1.1.2	Use case #1b: End users access the SNPN service via PLMN	12

5.1.1.3	Use case #1c: End users access the PLMN service via SNPN	12

5.1.2	Potential charging requirements	12

5.1.3	Key issues	13

5.1.4	Possible solutions	13

5.1.4.1	Solution #1: Data connectivity in SNPN Charging per UE directly	13

5.1.4.1.1	General	13

5.1.4.1.2	Reference architecture	13

5.1.4.1.3	Message flows	13

5.1.4.2	Solution #2: Data connectivity in SNPN Charging per UE (access the SNPN via PLMN)	14

5.1.4.2.1	General	14

5.1.4.2.2	Reference architecture	14

5.1.4.2.3	Message flows	14

5.1.4.3	Solution #3: Data connectivity in SNPN Charging per PLMN (access the PLMN via SNPN)	14

5.1.4.3.1	General	14

5.1.4.3.2	Reference architecture	14

5.1.4.3.3	Message flows	15

5.1.5	Evaluation	15

5.1.6	Conclusion	15

5.2	Topic 2: Converged charging for access connection in SNPN	16

5.2.1	Use cases	16

5.2.1.1	Use case #2a: Network access control	16

5.2.1.2	Use case #2b: Converged charging for number of UEs	16

5.2.1.3	Use case #2c: Converged charging for number of PDU sessions	16

5.2.2	Potential charging requirements	16

5.2.3	Key issues	17

5.2.4	Possible solutions	17

5.2.4.1	Solution #1: End user charging for SNPN network usage of access	17

5.2.4.1.1	General	17

5.2.4.1.2	General Registration – PEC charging	17

5.2.4.1.3	General Registration – IEC	17

5.2.4.1.4	General Registration – ECUR	17

5.2.4.1.5	Deregistration – PEC charging	17

5.2.4.2	Solution #2: Converged charging for number of UEs	18

5.2.4.2.1	General	18

5.2.4.2.2	Architecture description	18

5.2.4.2.3	Procedures description	18

5.2.4.3	Solution #3: Converged charging for number of PDU sessions	19

5.2.4.3.1	General	19

5.2.4.3.2	Architecture description	19

5.2.4.3.3	Procedures description	19

5.2.4.4	Solution #4: Converged charging for number of UEs or PDU sessions using separate CCS	21

5.2.4.4.1	General	21

5.2.4.4.2	Architecture description	21

5.2.4.4.3	Procedures description	22

5.2.5	Evaluation	22

5.2.5.1	Evaluation of possible solutions for end user charging for SNPN network usage of access	22

5.2.5.2	Evaluation of possible solutions for converged charging for number of UEs	22

5.2.5.3	Evaluation of possible solutions for converged charging for number of PDU sessions	22

5.2.6	Conclusion	23

5.3	Topic 3: Converged charging for data connectivity in PLMN	23

5.3.1	Use cases	23

5.3.1.1	Use case #3a: End users access the PLMN service via SNPN	23

5.3.1.2	Use case #3b: End users access the SNPN service via PLMN	24

5.3.2	Potential charging requirements	24

5.3.3	Key issues	24

5.3.4	Possible solutions	24

5.3.4.1	Solution #1: Data connectivity in PLMN Charging per UE (access the PLMN via SNPN)	24

5.3.4.1.1	General	24

5.3.4.1.2	Reference architecture	24

5.3.4.1.3	Message flows	25

5.3.4.2	Solution #2: Data connectivity in PLMN Charging per SNPN (access the SNPN via PLMN)	25

5.3.4.2.1	General	25

5.3.4.2.2	Reference architecture	25

5.3.4.2.3	Message flows	26

5.3.5	Evaluation	26

5.3.6	Conclusion	26

6	Charging scenarios and key issues for PNI-NPN	27

6.1	Topic 1: Converged charging for access connection in PNI-NPN	27

6.1.1	Use cases	27

6.1.1.1	Use case #1a: Network access control	27

6.1.1.2	Use case #1b: Converged charging for number of UEs	27

6.1.1.3	Use case #1c: Converged charging for number of PDU sessions	27

6.1.2	Potential charging requirements	27

6.1.3	Key issues	28

6.1.4	Possible solutions	28

6.1.4.1	Solution #1: End user charging for PNI-NPN network usage of access	28

6.1.4.1.1	General	28

6.1.4.1.2	General Registration – PEC charging	29

6.1.4.1.3	General Registration – IEC	31

6.1.4.1.4	General Registration – ECUR	32

6.1.4.1.5	Deregistration – PEC charging	33

6.1.4.2	Solution #2: Converged charging for number of UEs	34

6.1.4.2.1	General	34

6.1.4.2.2	Architecture description	34

6.1.4.2.3	Procedures description	35

6.1.4.3	Solution #3: Converged charging for number of PDU sessions	36

6.1.4.3.1	General	36

6.1.4.3.2	Architecture description	36

6.1.4.3.3	Procedures description	36

6.1.4.4	Solution #4: Converged charging for number of UEs or PDU sessions using separate CCS	37

6.1.4.4.1	General	37

6.1.4.4.2	Architecture description	38

6.1.4.4.3	Procedures description	38

6.1.5	Evaluation	41

6.1.5.1	Evaluation of possible solutions for end user charging for PNI-NPN network usage of access	41

6.1.5.2	Evaluation of possible solutions for converged charging for number of UEs	41

6.1.5.3	Evaluation of possible solutions for converged charging for number of PDU sessions	41

6.1.6	Conclusion	41

6.2	Topic 2: Converged charging for data connectivity in PNI-NPN	42

6.2.1	Use cases	42

6.2.1.1	Use case #1a: End user charging for PNI-NPN data connectivity	42

6.2.2	Potential charging requirements	42

6.2.3	Key issues	42

6.2.4	Possible solutions	42

6.2.4.1	Solution #1: End user charging for PNI-NPN data connectivity	42

6.2.4.1.1	General	42

6.2.4.1.2	Reference architecture	43

6.2.4.1.3	Message flows	43

6.2.5	Evaluation	43

6.2.6	Conclusion	43

7	Conclusions and recommendations	44

Annex A:	Change history	45

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document studies the charging aspects for enhanced support of non-public networks on the TS 23.501 [2], TS 23.502 [3], TS 23.503 [4], TS 23.228 [5] and TS 23.167 [6] incorporating conclusions from 3GPP TR 23.734 [7] and TR 23.700-07 [8].

The following is studied:

-	Charging scenarios and potential charging requirements for Non-Public Networks (i.e. Stand-alone Non-Public Network (SNPN) or Public Network Integrated NPN (PNI-NPN)).

-	Possible charging solutions for Non-Public Networks with potential impact on charging architecture, charging functions and charging procedures.

-	End user charging and Inter-Provider charging by considering the different business roles.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[6]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[7]	3GPP TR 23.734: "Study on enhancement of 5G System (5GS) for vertical and Local Area Network (LAN) services".

[8]	3GPP TR 23.700-07: "Study on enhanced support of Non-Public Networks (NPN)".

[9]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[10]	3GPP TS 28.557: "Management of Non-Public Networks (NPN); Stage 1 and stage 2".

[11]	3GPP TS 32.256: "5G connection and mobility domain charging; stage 2".

[12]	3GPP TS 32.255: 5G data connectivity domain charging; Stage 2".

[13]	3GPP TR 32.847: "Study on Charging Aspects for Network Slicing Phase 2".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

Void.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CAG	Closed Access Group

CSC 	Communication Service Customer

CSP 	Communication Service Provider

DNN	Data Network Name

N3IWF	Non-3GPP InterWorking Function

NID	Network identifier

NPN-OP	NPN Operator

NPN-SC	NPN Service Customer

NPN-SP	NPN Service Provider

PNI-NPN	Public Network Integrated NPN

SNPN	Stand-alone Non-Public Network

4	Background

4.1	General

In SA1, Non-Public Networks service requirements are documented in TS 22.261 [9]. In SA2, there are two study items and two work items which are related to Non-Public Networks. The corresponding studies are documented in TR 23.734 [7] and TR 23.700-07 [8], respectively and the normative work is documented in TS 23.501 [2], TS 23.502 [3], TS 23.503 [4], TS 23.228 [5] and TS 23.167 [6].

A Non-Public Network (NPN) is a 5GS deployed for non-public use, see TS 22.261 [9]. As described in TS 23.501 [2], an NPN is either:

-	a Stand-alone Non-Public Network (SNPN), i.e. operated by an NPN operator and not relying on network functions provided by a PLMN; or

-	a Public Network Integrated NPN (PNI-NPN), i.e. a non-public network deployed with the support of a PLMN.

Functionality and architecture of Stand-alone NPN are described in clause 4.2.1 and functionality and architecture of Public Network Integrated NPNs are described in clause 4.2.2.

4.2	Non-public networks functionality and architecture

4.2.1	Stand-alone Non-Public Network (SNPN)

SNPN 5GS deployments are based on the architecture depicted in clause 4.2.3 TS 23.501 [2], the architecture for 5GC with untrusted non-3GPP access (Figure 4.2.1-1) for access to SNPN services via a PLMN (and vice versa).

Figure 4.2.1-1: Non-roaming architecture for 5G Core Network with untrusted non-3GPP access

To access SNPN services, a UE that has successfully registered with a PLMN over 3GPP access may perform another registration via the PLMN User Plane with an SNPN (using the credentials of that SNPN). In order to obtain access to Non-Public Network services when the UE is camping in NG-RAN of a PLMN, the UE obtains IP connectivity, discovers and establishes connectivity to an N3IWF in the Stand-alone Non-Public Network. The architecture for access to SNPN services via PLMN is depicted in Figure 4.2.1-2.

Figure 4.2.1-2: Access to Stand-alone Non-Public Network services via PLMN

To access PLMN services, a UE in SNPN access mode that has successfully registered with an SNPN may perform another registration via the SNPN User Plane with a PLMN (using the credentials of that PLMN). In order to obtain access to PLMN services when the UE is camping in NG-RAN of Stand-alone Non-Public Network, the UE obtains IP connectivity, discovers and establishes connectivity to an N3IWF in the PLMN. The architecture for access to PLMN services via SNPN is depicted in Figure 4.2.1-3.

Figure 4.2.1-3: Access to PLMN services via Stand-alone Non-Public Network

4.2.2	Public Network Integrated NPN (PNI-NPN)

Public Network Integrated NPNs are NPNs made available via PLMNs e.g. by means of dedicated DNNs, or by one (or more) Network Slice instances allocated for the NPN. When a PNI-NPN is made available via a PLMN, then the UE needs to have a subscription for the PLMN in order to access PNI-NPN.

A Closed Access Group (CAG) identifies a group of subscribers who are permitted to access one or more CAG cells associated to the CAG. CAG is used for the PNI-NPNs to prevent UE(s), which are not allowed to access the NPN via the associated cell(s), from automatically selecting and accessing the associated CAG cell(s).

4.3	Charging modes

As per TS 23.501 [2], in the context of Non-Public Networks, responsibilities regarding operations need to be clearly defined and assigned to roles. NPN charging can be built upon the roles related to NPN management specified in the clause 4.2 of TS 28.557 [10] without the consideration on the network equipment and infrastructure. Non-public networks in 5G environment involves the services or capabilities provided by multiple service providers in the form of following business roles:

- 	NPN Operator (NPN-OP): a Network Operator (NOP) is limited to 5G networks for non-public use, i.e. NPNs.

- 	NPN Service Provider (NPN-SP): a Communication Service Provider (CSP) which provides communication services for non-public use, e.g. vertical, tenant, NOP.

- 	NPN Service Customer (NPN-SC): a Communication Service Customer (CSC) which consumes communication services for non-public use, e.g. end user, vertical, tenant.

Considering the different business models such as B2B, B2C, and B2B2X, charging modes for non-public networks include (but not limited to):

-	End user charging;

-	Inter-provider charging.

Depending on the scenarios an organisation can play one or several roles simultaneously, and apply charging models based on corresponding business relationship, e.g.:

- 	Charging models for NPN-SP to charge NPN-SC for using NPN services.

-	 Charging models for NPN-OP to charge NPN-SP for using their networks and related network services.

- 	When NPN-OP and NPN-SP role are provided by the same enterprise, to charge NPN-SC role for using their networks and NPN services.

In deployments, there could be business scenarios where one or more of the business roles are supported by a single enterprise. The present document does not impose any restrictions to the possible deployment scenarios.

5	Charging scenarios and key issues for SNPN

5.1	Topic 1: Converged charging for data connectivity in SNPN

5.1.1	Use cases

5.1.1.1	Use case #1a: End users access the SNPN service directly

SNPN 5GS deployments are based on the 5G system architecture depicted in clause 4.2.3 of TS 23.501 [2].

Charged Party: An End User (UE) who accessed SNPN services.

Charging Party: SNPN operator who charges the End user for data connectivity in SNPN.

The potential charging requirements and key issues for this UC are: REQ-CH_SNPN_DC-01 and Key Issue #1a.

5.1.1.2	Use case #1b: End users access the SNPN service via PLMN

SNPN 5GS deployments are based on the 5G system architecture depicted in clause 4.2.3 of TS 23.501 [2], the architecture for 5GC with untrusted non-3GPP access for access to SNPN services via a PLMN (and vice versa).

Charged Party: An End User (UE) has a subscription with the PLMN and SNPN which allows access of the SNPN service via PLMN.

Charging Party:

-	SNPN operator charges the End user for data connectivity in SNPN.

The potential charging requirements and key issues for this UC are: REQ-CH_SNPN_DC-01 and Key Issue #1a.

5.1.1.3	Use case #1c: End users access the PLMN service via SNPN

SNPN 5GS deployments are based on the 5G system architecture depicted in clause 4.2.3 of TS 23.501 [2], the architecture for access to PLMN services via SNPN is depicted in Figure 4.2.1-3. To access PLMN services, a UE in SNPN access mode establishes the data connectivity to an N3IWF in the PLMN.

Charged Party: The PLMN has a agreement with the SNPN which allows end users accessing the PLMN service via SNPN.

Charging Party:

-	SNPN operator charges the PLMN for data connectivity to or from PLMN via SNPN.

The potential charging requirements and key issues for this UC are: REQ-CH_SNPN_DC-02 and Key Issue #1b.

5.1.2	Potential charging requirements

REQ-CH_SNPN_DC-01: The SNPN should support collecting charging information of data connectivity charging in SNPN per UE for SNPN services.

REQ-CH_SNPN_DC-02: The SNPN should support collecting charging information of data connectivity charging via SNPN per PLMN for PLMN services.

5.1.3	Key issues

The following key issues are identified:

-	Key Issue #1a: the charging information collection and reporting in SNPN for data connectivity per UE.

-	Key Issue #1b: the charging information collection and reporting in SNPN for data connectivity per PLMN.

5.1.4	Possible solutions

5.1.4.1	Solution #1: Data connectivity in SNPN Charging per UE directly

5.1.4.1.1	General

This solution #1 which relying on 5G data connectivity converged charging architecture defined in TS 32.255 [12], with the extension of SNPN charging per UE, addresses the Key Issue #1a.

5.1.4.1.2	Reference architecture

Based on figure 4.2.1-3, the CHF in the SNPN is added. The UE is charged by CHF in the SNPN for 5G data connectivity in SNPN.

Figure 5.1.4.1.2-1: Data connectivity of SNPN Charging per UE directly in SBI representation

5.1.4.1.3	Message flows

The message flows are the same with the 5G data connectivity per PDU session charging specified in the TS 32.255 [12].

SMF in the SNPN reports the Charging Data Request contains SNPN ID (PLMN ID and NID identifying an SNPN) to the CHF in the SNPN.

5.1.4.2	Solution #2: Data connectivity in SNPN Charging per UE (access the SNPN via PLMN)

5.1.4.2.1	General

This solution #2 which relying on 5G data connectivity converged charging architecture defined in TS 32.255 [12], with Charging UE by SNPN, addresses the Key Issue #1a for access SNPN via PLMN.

5.1.4.2.2	Reference architecture

Based on figure 4.2.1-2 the CHF in the SNPN is added. The UE is charged by CHF in the SNPN for data connectivity via PLMN.

Figure 5.1.4.2.1-1: Data connectivity of SNPN Charging (access the SNPN via PLMN)

5.1.4.2.3	Message flows

For access the SNPN services, the message flows for the PDU session establishment, PDU session modification, PDU session release of 5G data connectivity charging would be the same as in clauses 5.2.2.13.2.1, 5.2.2.13.3.1 and 5.2.2.13.4.1 of TS 32.255 [12].

The SMF in SNPN reports the Charging Data Request contains access type ("Non-3GPP access") to the CHF belongs to the same SNPN, using PLMN ID and NID to identify the SNPN.

5.1.4.3	Solution #3: Data connectivity in SNPN Charging per PLMN (access the PLMN via SNPN)

5.1.4.3.1	General

This solution #3 which relying on 5G data connectivity converged charging architecture defined in TS 32.255 [12], with the extension of SNPN charging per PLMN for accessing the PLMN service, addresses the Key Issue #1b, covering the REQ-CH_SNPN_DC-02.

5.1.4.3.2	Reference architecture

Based on figure 4.2.1-3, the CHF in the SNPN is added. The PLMN is charged by CHF in the SNPN for 5G data connectivity to support of accessing the PLMN service.

Figure 5.1.4.3.2-1: Data connectivity in SNPN Charging (access the PLMN via SNPN)

5.1.4.3.3	Message flows

For access the PLMN services via SNPN, the message flows for the PDU session establishment, PDU session modification, PDU session release of 5G data connectivity charging would be the same as in clauses 5.2.2.13.2.1, 5.2.2.13.3.1 and 5.2.2.13.4.1 of TS 32.255 [12].

The CHF in SNPN charges the volume usage per PLMN based on the accumulation of usage for UEs belonging to the same PLMN. In the solution, the SMF reports the charging Data request contains the PLMN ID.

5.1.5	Evaluation

Solution #1 and solution #2 solve the key issue #1a:

-	Solution #1 focus on the access SNPN service directly for end user charging based on PDU session charging via SMF in SNPN.

-	Solution #2 focus on the access the SNPN service via PLMN for end user charging based on PDU session charging via SMF.

-	The UE is charged by CHF in the SNPN for 5G data connectivity to support of accessing the SNPN service via PLMN.

-	The UE is charged by CHF in the PLMN for 5G data connectivity to support of accessing PLMN.

Solution #3 solves the key issue 1b for the interworking case:

-	Solution #3 focus on the access the PLMN service via SNPN for inter-provider charging.

-	The solution 3 solves the data connectivity in SNPN Charging per PLMN (access the PLMN via SNPN) for inter-provider charging, which aggregates usage for UEs belonging to the same PLMN.

-	For inter-provider charging, the PLMN ID is required and this relies on that the SNPN can provide the PLMN identifier.

5.1.6	Conclusion

The Solution #1, Solution #2 and Solution #3 are selected to the normative work.

5.2	Topic 2: Converged charging for access connection in SNPN

5.2.1	Use cases

5.2.1.1	Use case #2a: Network access control

The combination of a PLMN ID and Network Identifier (NID) identifies an SNPN. If a UE performs the registration or service request procedure in an SNPN identified by a PLMN ID and a NID and there is no subscription for the UE, then AMF rejects the UE with a cause code to prevent the UE from automatically selecting and registering with the same SNPN as described in TS 23.501 [2].

-	The NPN-OP provides the SNPN to NPN-SP (e.g. vertical industry customer), and NPN-SP provides NPN services to NPN-SC. It is important for NPN-OP to collect usage for SNPN resources associated to access connection for the purpose of charging or statistics.

The potential charging requirements for this UC are: REQ-eNPN_CH_SNPN_AC-01, REQ-eNPN_CH_SNPN_AC-02.

5.2.1.2	Use case #2b: Converged charging for number of UEs

The maximum number of UEs, corresponding to the maximum number of UEs associated with Network Identifier, represents the maximum number of UEs using the SNPN network simultaneously.

For the SNPN, it is important for NPN-OP to collect number of UEs per Network Identifier of vertical industry customers for the purpose of charging or statistics. In addition, NPN-OP can perform tiered charging for different number of UEs threshold.

The potential charging requirements for this UC are: REQ-eNPN_CH_SNPN_AC-03, REQ-eNPN_CH_SNPN_AC-04.

5.2.1.3	Use case #2c: Converged charging for number of PDU sessions

The maximum number of PDU sessions, corresponding to the maximum number of PDU sessions associated with Network Identifier, represents the maximum number of PDU sessions using the SNPN network simultaneously.

For the SNPN, it is important for NPN-OP to collect number of PDU sessions per Network Identifier of vertical industry customers for the purpose of charging or statistics. In addition, NPN-OP can perform tiered charging for different number of PDU sessions threshold.

The potential charging requirements for this UC are: REQ-eNPN_CH_SNPN_AC-03, REQ-eNPN_CH_SNPN_AC-05.

5.2.2	Potential charging requirements

REQ-eNPN_CH_SNPN_AC-01: The 5G system should support converged charging per UE for SNPN network access and usage.

REQ-eNPN_CH_SNPN_AC-02: The 5G system should support converged charging per vertical industry consumer for SNPN network access and usage.

REQ-eNPN_CH_SNPN_AC-03: The 5G system should support converged charging per Network Identifier for SNPN network access and usage.

REQ-eNPN_CH_SNPN_AC-04: The 5G system should support converged charging per Network Identifier for SNPN network access and usage, based on "number of UEs".

REQ-eNPN_CH_SNPN_AC-05: The 5G system should support converged charging per Network Identifier for SNPN network access and usage, based on "number of PDU sessions".

5.2.3	Key issues

This key issue is for investigating how to support converged charging for access connection in SNPN considering REQ-eNPN_CH_SNPN_AC-01, REQ-eNPN_CH_SNPN_AC-02, REQ-eNPN_CH_SNPN_AC-03, REQ-eNPN_CH_SNPN_AC-04 and REQ-eNPN_CH_SNPN_AC-05. This investigation covers the following:

-	Key Issue #2a: Identification of the charging information for converged charging for SNPN network access and usage;

-	Key Issue #2b: Identification of the main interactions with the NFs to obtain the charging information;

-	Key Issue #2c: The charging mechanism supporting converged inter-provider charging for SNPN network access and usage.

5.2.4	Possible solutions

5.2.4.1	Solution #1: End user charging for SNPN network usage of access

5.2.4.1.1	General

This solution #1 which relying on 5G connection and mobility converged charging architecture defined in TS 32.256 [11], with the extension of including Network Identifier for SNPN network access in registration management charging per UE, addresses the Key Issue #2a and Key Issue #2b.

In this solution, AMF collects charging information including Network Identifier per UE registration to the 5GS, and the charging information for end user usage of access in SNPN network reported by AMF is obtained by CHF.

The subclause below describes registration management charging. The following scenarios specified in TS 32.256 [11] are supported:

- 	PEC;

-	IEC;

-	ECUR.

For deregistration procedures, only PEC scenario is supported.

5.2.4.1.2	General Registration – PEC charging

This solution is the same as clause 6.1.4.1.2 with:

-	"CAG Identifier(s)" replaced by "Network Identifier".

5.2.4.1.3	General Registration – IEC

This solution is the same as clause 6.1.4.1.3 with:

-	"CAG Identifier(s)" replaced by "Network Identifier".

5.2.4.1.4	General Registration – ECUR

This solution is the same as clause 6.1.4.1.4 with:

-	"CAG Identifier(s)" replaced by "Network Identifier".

5.2.4.1.5	Deregistration – PEC charging

This solution is the same as clause 6.1.4.1.5 with:

-	"CAG Identifier(s)" replaced by "Network Identifier".

5.2.4.2	Solution #2: Converged charging for number of UEs

5.2.4.2.1	General

This solution #2, charges vertical industry customer when "Number of UEs" threshold for the Network Identifier is reached or "maximum number of UEs" for the Network Identifier is reached, which addresses the Key Issue #2a, Key Issue #2b and Key Issue #2c.

5.2.4.2.2	Architecture description

A new NF NPNACF (Non-Public Network Admission Control Function) is introduced to the solution. The converged charging architecture for NPNACF (CTF) based solution as depicted in figure 5.2.4.2.2-1 is proposed for converged charging for number of UEs per Network Identifier.

Figure 5.2.4.2.2-1: NPNACF (CTF) converged charging architecture

5.2.4.2.3	Procedures description

The figure 5.2.4.2.3-1 describes the high-level charging procedure for NPNACF (CTF) based converged charging.

Figure 5.2.4.2.3-1: NPNACF (CTF) based converged charging-IEC

0ch. CHF obtains the "maximum number of UEs" per Network Identifier.

The NPNACF is configured per Network Identifier, with "maximum number of UEs" and a set of "Number of UEs" intermediate thresholds (downwards and upwards triggers) under the "maximum number of UEs".

Registration procedure per TS 32.256 [11] clause 5.2.2.2.3 steps 1~16.

3. The AMF sends NumOfUEsUpdate_Request to NPNACF. The NPNACF checks availability and updates Number of UEs per Network Identifier.

3ch-a: For a Network Identifier one "Number of UEs" threshold is reached or "maximum number of UEs" is reached.

3ch-b: The NPNACF sends Charging Data Request to CHF for reporting number of UEs for the Network Identifier.

3ch-c. Account and rating control based on number of UEs for the Network Identifier.

3ch-d. CHF provides response to NPNACF by sending Charging Data Response.

4. The NPNACF sends NumOfUEsUpdate_Response to AMF.

5. Registration procedure per TS 32.256 [11], clause 5.2.2.2.3 steps 17~24.

NOTE :	The support for controlling number of UEs per Network Identifier is dependent on SA WG2.

5.2.4.3	Solution #3: Converged charging for number of PDU sessions

5.2.4.3.1	General

This solution #3, charges vertical industry customer when "Number of PDU sessions" threshold for the Network Identifier is reached or "maximum number of PDU sessions" for the Network Identifier is reached, which addresses the Key Issue #2a, Key Issue #2b and Key Issue #2c.

5.2.4.3.2	Architecture description

A new NF NPNACF (Non-Public Network Admission Control Function) is introduced to the solution. The converged charging architecture for NPNACF (CTF) based solution as depicted in figure 5.2.4.2.2-1 is proposed for converged charging for number of PDU sessions per Network Identifier.

5.2.4.3.3	Procedures description

The figure 5.2.4.3.3-1 describes the high-level charging procedure for NPNACF (CTF) based converged charging.

Figure 5.2.4.3.3-1: NPNACF (CTF) based converged charging-IEC

0ch. CHF obtains the "maximum number of PDU sessions" per Network Identifier.

1. The NPNACF is configured per Network Identifier, with "maximum number of PDU sessions" and a set of "Number of PDU sessions" intermediate thresholds (downwards and upwards triggers) under the "maximum number of PDU sessions".

2. PDU session establishment procedure per TS 32.255 [12], clause 5.2.2.2.2 steps 1~9.

3. The SMF sends NumOfPDUsUpdate_Request to NPNACF. The NPNACF checks availability and updates Number of PDU sessions per Network Identifier.

3ch-a: For a Network Identifier one "Number of PDU sessions" threshold is reached or "maximum number of PDU sessions" is reached.

3ch-b: The NPNACF sends Charging Data Request to CHF for reporting number of PDU sessions for the Network Identifier.

3ch-c. Account and rating control based on number of PDU sessions for the Network Identifier.

3ch-d. CHF provides response to NPNACF by sending Charging Data Response.

4. The NPNACF sends NumOfPDUsUpdate_Response to SMF.

5. PDU session establishment procedure per TS 32.255 [12] clause 5.2.2.2.2 steps 10~14.

NOTE :	The support for controlling PDU sessions per Network Identifier is dependent on SA WG2.

5.2.4.4	Solution #4: Converged charging for number of UEs or PDU sessions using separate CCS

5.2.4.4.1	General

This solution addresses the key issues #2a, 2b and 2c covering potential requirements REQ-eNPN_CH_SNPN_AC-03, REQ-eNPN_CH_SNPN_AC-04 and REQ-eNPN_CH_SNPN_AC-05 based on distributed charging architecture split between two Converged Charging Systems (CCS) and exchanges between them.

Two types of subscribers are considered in the context of SNPN: the NPN-SC, and the individual end-user "UE" served under the NPN. Each type of subscriber is handled under a dedicated CCS: "NPN CCS" and "UE CCS" respectively.

UE CCS of the PLMN performs charging functionalities for individual UEs, and includes in particular:

-	CHF exposing Nchf services associated to individual UEs.

-	ABMF/RF hosting individual UE account balance and subscribed services, as well as the rates for services.

NPN CCS of the PLMN performs charging functionalities for NPN-SC, and includes in particular:

-	CHF exposing Nchf services associated to individual UEs.

-	ABMF/RF hosting individual NPN-SC account balance and subscribed services, as well as the rates for services.

How the account balance and subscribed services are provisioned (for both NPN-SC and individual UE) is unspecified and expected to be covered at BSS and OSS level.

A new reference point (ChX) is defined between CHF in NPN CCS and the CHF in UE CCS enabling association between individual UE charging and NPN-SC charging. This association is achieved by UE CCS responsible for individual UE charging consuming this new charging service exposed by the NPN-SC CCS.

The new reference (ChX) may use the Nchf_ConvergedCharging service with extensions or use a completely new Nchf service.

For UE:

-	the 5G data connectivity charging from SMF is specified in 3GPP TS 32.255 [12]

-	the 5G connection and mobility charging from AMF is specified in 3GPP TS 32.256 [11]

and uses Nchf_ConvergedCharging service exposed by CHF in UE CCS.

5.2.4.4.2	Architecture description

Figure 5.2.4.4.2-1: Distributed charging architecture in reference point representation

Figure 5.2.4.4.2-2: Distributed charging architecture in service based representation

5.2.4.4.3	Procedures description

This would be the same as for the figure 6.1.4.4.3-1 for the number of PDU sessions and 6.1.4.4.3-2 for the number of UEs with the possibility of using Network Identifier and PLMN Id to identify the SNPN.

5.2.5	Evaluation

5.2.5.1	Evaluation of possible solutions for end user charging for SNPN network usage of access

Solution #1 has proposed end user charging for SNPN network usage of access, enabling the AMF to collect charging information which including Network Identifier per UE registration to the 5GS, and the charging information for end user usage of access in SNPN network reported by AMF is obtained by CHF.

5.2.5.2	Evaluation of possible solutions for converged charging for number of UEs

Solution #2 and #4 propose solutions on converged charging for number of UEs. The following table lists the characteristics of the two solutions.

Table 5.2.5.2-1: Comparison between Solution #2 and #4

|  | NF controlling number of UEs per PNI-NPN | NF acts as a CTF |
| --- | --- | --- |
| Solution #2: NPNACF (CTF) based converged charging for number of UEs | NPNACF (New NF) | NPNACF |
| Solution #4: Separate CCS based converged charging for number of UEs | CHF (New NPN CCS) | AMF |

The principles of solution #2 are inline with the NSACF charging of Network Slicing in TR 32.847 [13], and the identification of the SNPN can be based on e.g. Network Identifier.

The principles of solution #4, are inline the solution for network slice charging based on 5G data connectivity in TS 32.255 [12] annex D.

5.2.5.3	Evaluation of possible solutions for converged charging for number of PDU sessions

Solution #3 and #4 propose solutions on converged charging for number of PDU sessions. The following table lists the characteristics of the two solutions.

Table 5.2.5.3-1: Comparison between Solution #3 and #4

|  | NF controlling number of PDU sessions per NPI-NPN | NF acts as a CTF |
| --- | --- | --- |
| Solution #3: NPNACF (CTF) based converged charging for number of PDU sessions | NPNACF (New NF) | NPNACF |
| Solution #4: Separate CCS based converged charging for number of PDU sessions | CHF (New NPN CCS) | SMF |

The principles of solution #3 are inline with the NSACF charging of Network Slicing in TR 32.847 [13], and the identification of the SNPN can be based on e.g. Network Identifier.

The principles of solution #4, are inline the solution for network slice charging based on 5G data connectivity in TS 32.255 [12] annex D.

5.2.6	Conclusion

Based on the evaluation in clause 5.2.5.1, solution #1 is the only solution for end user charging for SNPN network usage of access. The solution #1 is selected to the normative work.

Based on the evaluation in clause 5.2.5.2, solution #2 needs support for controlling number of UEs for SNPN per Network Identifier which is not currently supported by SA WG2. Solution #4 needs support a new interface between CHF in NPN CCS and CHF in UE CCS which is not currently supported, and it also has some impact on PDU session establishment procedure in SMF, and registration procedure in AMF, which require converged charging. It is concluded that using converged charging mechanism for simultaneous number of UEs per network slice to support converged charging for number of UEs per SNPN in normative work of release 18. It can solve a unique S-NSSAI is allocated for one SNPN.

Based on the evaluation in clause 5.2.5.3, solution #3 needs support for controlling PDU sessions for SNPN per Network Identifier which is not currently supported by SA WG2. Solution #4 needs support a new interface between CHF in NPN CCS and CHF in UE CCS which is not currently supported, and it also has some impact on PDU session establishment procedure in SMF, and registration procedure in AMF, which require converged charging. It is concluded that using converged charging mechanism for simultaneous number of PDU sessions per network slice to support converged charging for number of PDU sessions per SNPN in normative work of release 18. It can solve a unique S-NSSAI is allocated for one SNPN.

5.3	Topic 3: Converged charging for data connectivity in PLMN

5.3.1	Use cases

5.3.1.1	Use case #3a: End users access the PLMN service via SNPN

SNPN 5GS deployments are based on the 5G system architecture depicted in clause 4.2.3 of TS 23.501 [2], the architecture for access to PLMN services via SNPN is depicted in Figure 4.2.1-3. To access PLMN services, a UE in SNPN access mode establishes the data connectivity to an N3IWF in the PLMN.

Charged Party: The End User (UE) has a subscription with the SNPN and PLMN which allows access of the PLMN service.

Charging Party:

-	PLMN operator charges the 5G data connectivity for the End user.

The potential charging requirements for this UC are: REQ-CH_PLMN_DC-01 and Key Issue #3a.

5.3.1.2	Use case #3b: End users access the SNPN service via PLMN

SNPN 5GS deployments are based on the 5G system architecture depicted in clause 4.2.3 of TS 23.501 [2], the architecture for 5GC with untrusted non-3GPP access for access to SNPN services via a PLMN.

Charged Party: The SNPN has a agreement with the PLMN which allows End users accessing the SNPN service via PLMN.

Charging Party:

-	PLMN charges the 5G data connectivity for the SNPN.

The potential charging requirements and key issues for this UC are: REQ-CH_PLMN_DC-02 and key issue #3b.

5.3.2	Potential charging requirements

REQ-CH_PLMN_DC-01: The 5G system should support collecting charging information based on the 5G data connectivity usage per UE.

REQ-CH_PLMN_DC-02: The 5G system should support collecting charging information based on the 5G data connectivity usage per SNPN.

5.3.3	Key issues

The following key issues are identified:

-	Key Issue #3a: the charging information collection and reporting in 5G system for 5G data connectivity per UE.

-	Key Issue #3b: the charging information collection and reporting in 5G system for 5G data connectivity for PLMN per SNPN.

5.3.4	Possible solutions

5.3.4.1	Solution #1: Data connectivity in PLMN Charging per UE (access the PLMN via SNPN)

5.3.4.1.1	General

This solution #1 which relying on 5G data connectivity converged charging architecture defined in TS 32.255 [12], with the extension of PLMN Charging via SNPN access per UE, addresses the Key Issue #3a.

5.3.4.1.2	Reference architecture

Based on figure 4.2.1-3, the CHF in the PLMN is added. The UE is charged by CHF in the PLMN for 5G data connectivity of PLMN services.

Figure 5.3.4.1.2-1: Data connectivity in PLMN Charging per UE (access the PLMN via SNPN)

5.3.4.1.3	Message flows

For access the PLMN services, the message flows for the PDU session establishment, PDU session modification, PDU session release of 5G data connectivity charging would be the same as in clauses 5.2.2.13.2.1, 5.2.2.13.3.1 and 5.2.2.13.4.1 of TS 32.255 [12].

The SMF in the PLMN reports the Charging Data Request contains RAT type specifics to SNPN

5.3.4.2	Solution #2: Data connectivity in PLMN Charging per SNPN (access the SNPN via PLMN)

5.3.4.2.1	General

This solution #2 which relying on 5G data connectivity converged charging architecture defined in TS 32.255 [12], with the charging for the usage in the PLMN per SNPN by PLMN, addresses the Key Issue #3b.

5.3.4.2.2	Reference architecture

Based on figure 4.2.1-2, the CHF in the PLMN is added. The SNPN is charged by CHF in the PLMN for data connectivity.

Figure 5.3.4.2.2-1: Data connectivity in PLMN Charging per SNPN (access the SNPN via PLMN)

5.3.4.2.3	Message flows

For access the SNPN services, the message flows for the PDU session establishment, PDU session modification, PDU session release of 5G data connectivity charging would be the same as in clauses 5.2.2.13.2.1, 5.2.2.13.3.1 and 5.2.2.13.4.1 of TS 32.255 [12].

The NWu (for NPN) between the UE and the N3IWF in the SNPN for establishing a secure tunnel between UE and the N3IWF over the PLMN. The SMF in PLMN can distinguish the usage packet transferred to N3IWF in SNPN, and sends the volume to the CHF in the PLMN belongs to same N3IWF address.

The CHF in PLMN charges the volume usage per SNPN based on the accumulation of usage reporting per UE, may with the same N3IWF address.

5.3.5	Evaluation

The Solution #1 solves the key issue #3a:

-	solves the data connectivity in PLMN (access the PLMN via SNPN) for End user charging, with a RAT type specific to a SNPN and with a SNPN ID (PLMN ID and NID identifying SNPN), if available.

-	For end user charging, the SNPN ID is not necessary. Without the SNPN ID, there is no impact on the Solution #1.

The Solution #2 solves the key issue #3b:

-	solves the data connectivity in PLMN (access the SNPN via PLMN) for inter-provider charging, with the volume usage per SNPN based on the accumulation of usage reporting per UE, with the same N3IWF address to indicate the SNPN.

5.3.6	Conclusion

Solution #1 and Solution #2 are selected to the normative work.

6	Charging scenarios and key issues for PNI-NPN

6.1	Topic 1: Converged charging for access connection in PNI-NPN

6.1.1	Use cases

6.1.1.1	Use case #1a: Network access control

Relying on PLMN network, PNI-NPN provides logically independent 5G non-public network for vertical industry customers through network slice instances/ dedicated DNNs, and supporting of CAG feature. CAG is used for access control to prevent unauthorized UEs from trying to access a PNI-NPN.

The NPN-OP provides the PNI-NPN to NPN-SP (e.g. vertical industry customer), and NPN-SP provides NPN services to NPN-SC. It is important for NPN-OP to collect usage for PNI-NPN resources associated to access connection for the purpose of charging or statistics.

The potential charging requirements for this UC are: REQ-eNPN_CH_PNI-NPN_AC-01, REQ-eNPN_CH_PNI-NPN_AC-02.

6.1.1.2	Use case #1b: Converged charging for number of UEs

For vertical industry consumers, some enterprise UEs may be restricted to only access the non-public network of the enterprise, even if PLMN coverage is available in the same geographic area. To use Closed Access Group, enterprise UE may subscribe the CAG information, indicating CAG Identifiers the UE is allowed to access.

The maximum number of UEs, corresponding to the maximum number of UEs associated with CAG Identifier, represents the maximum number of UEs using the PNI-NPN network simultaneously in restricted area. If the enterprise has one or more CAG Identifiers, the maximum number of UEs can be calculated per CAG Identifier.

For the PNI-NPN with CAG scenarios, it is important for NPN-OP to collect number of UEs per CAG Identifier of vertical industry customers for the purpose of charging or statistics. In addition, NPN-OP can perform tiered charging for different number of UEs threshold.

The potential charging requirements for this UC are: REQ-eNPN_CH_PNI-NPN_AC-03, REQ-eNPN_CH_PNI-NPN_AC-04.

6.1.1.3	Use case #1c: Converged charging for number of PDU sessions

The maximum number of PDU sessions, corresponding to the maximum number of PDU sessions associated with CAG Identifier, represents the maximum number of PDU sessions supported by the PNI-NPN network in restricted area. If the enterprise has one or more CAG Identifiers, the maximum number of PDU sessions can be calculated per CAG Identifier.

For the PNI-NPN with CAG scenarios, it is important for NPN-OP to collect number of PDU sessions per CAG Identifier of vertical industry customers for the purpose of charging or statistics. In addition, NPN-OP can perform tiered charging for different number of PDU sessions threshold.

The potential charging requirements for this UC are: REQ-eNPN_CH_PNI-NPN_AC-03, REQ-eNPN_CH_PNI-NPN_AC-05.

6.1.2	Potential charging requirements

REQ-eNPN_CH_PNI-NPN_AC-01: The 5G system should support converged charging per UE for PNI-NPN network access and usage.

REQ-eNPN_CH_PNI-NPN_AC-02: The 5G system should support converged charging per vertical industry consumer for PNI-NPN network access and usage.

REQ-eNPN_CH_PNI-NPN_AC-03: The 5G system should support converged charging per CAG Identifier for PNI-NPN network access and usage.

REQ-eNPN_CH_PNI-NPN_AC-04: The 5G system should support converged charging per CAG Identifier for PNI-NPN network access and usage, based on "number of UEs".

REQ-eNPN_CH_PNI-NPN_AC-05: The 5G system should support converged charging per CAG Identifier for PNI-NPN network access and usage, based on "number of PDU sessions".

6.1.3	Key issues

This key issue is for investigating how to support converged charging for access connection in PNI-NPN considering REQ-eNPN_CH_PNI-NPN_AC-01, REQ-eNPN_CH_PNI-NPN_AC-02, REQ-eNPN_CH_PNI-NPN_AC-03, REQ-eNPN_CH_PNI-NPN_AC-04 and REQ-eNPN_CH_PNI-NPN_AC-05. This investigation covers the following:

-	Key Issue #2a: Identification of the charging information for converged charging for PNI-NPN network access and usage;

-	Key Issue #2b: Identification of the main interactions with the NFs to obtain the charging information;

-	Key Issue #2c: The charging mechanism supporting converged inter-provider charging for PNI-NPN network access and usage.

6.1.4	Possible solutions

6.1.4.1	Solution #1: End user charging for PNI-NPN network usage of access

6.1.4.1.1	General

The CAG information of the UE is included in the subscription as part of the Mobility Restrictions as defined in clause 5.30.3 of TS 23.501 [2]:

-	an Allowed CAG list i.e. a list of CAG Identifiers the UE is allowed to access; and

-	optionally, a CAG-only indication whether the UE is only allowed to access 5GS via CAG cells.

This solution #1 which relying on 5G connection and mobility converged charging architecture defined in TS 32.256 [11], with the extension of including CAG Identifier(s) for PNI-NPN network access in registration management charging per UE, addresses the Key Issue #2a and Key Issue #2b.

Figure 6.1.4.1.1-1: 5G connection and mobility converged charging architecture 
defined in TS 32.256 [11]

In this solution, AMF collects charging information including CAG Identifier(s) per UE registration to the 5GS, and the charging information for end user usage of access in PNI-NPN network reported by AMF is obtained by CHF.

The subclause below describes registration management charging. The following scenarios specified in TS 32.256 [11] are supported:

- 	PEC;

-	IEC;

-	ECUR.

For deregistration procedures, only PEC scenario is supported.

6.1.4.1.2	General Registration – PEC charging

The following figure 6.1.4.1.2-1 describes a Registration charging message flow in PEC charging applicable to:

-	Initial Registration in 5GS;

-	Mobility Registration Update;

-	Periodic Registration Update; or

-	Emergency Registration.

Figure 6.1.4.1.2-1: Registration – PEC charging

1-14c.	Registration procedure initiated by UE.

10: The new AMF notifies the old AMF that the registration of the UE in the new AMF is completed.

14d: If necessary, the old AMF is requested by UDM to delete Registration Management contexts and PDU Sessions.

14dCh: The old AMF interacts with the CHF as per deregistration procedure described in figure 6.1.4.1.5-1.

21.	The AMF verifies whether UE access is allowed by Mobility Restrictions as defined in TS 23.501 [2]. Upon successful procedure, Registration Accept sent to the UE.

21ch-a. The AMF sends Charging Data Request [Event] including CAG Identifier(s) to CHF for the UE successful registration.

21ch-b. The CHF creates the CDR for this registration.

21ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the AMF.

6.1.4.1.3	General Registration – IEC

The following figure 6.1.4.1.3-1 describes a Registration charging message flow in IEC, based on the clause 6.1.4.1.2 Registration description:

Figure 6.1.4.1.3-1: Registration – IEC

1-14c.	Registration procedure initiated by UE.

15-16: AM policy association procedure.

16ch-a. The AMF sends Charging Data Request [Event] including CAG Identifier(s) to CHF for the UE registration to be granted authorization, rated and accounted.

16ch-b. Account and Rating control by the CHF for the registration and CDR creation.

16ch-c. The CHF acknowledges by sending Charging Data Response [Event] and the AMF, granting authorization to the registration.

17 to 24. Same steps as in Figure 6.1.4.1.2-1.

6.1.4.1.4	General Registration – ECUR

The following figure 6.1.4.1.4-1 describes a Registration charging message flow in ECUR, based on the clause 6.1.4.1.2 Registration description:

Figure 6.1.4.1.4-1: Registration – ECUR

1-14c.	Registration procedure initiated by UE.

15-16: AM policy association procedure.

16ch-a. The AMF sends Charging Data Request [Initial] including CAG Identifier(s) to CHF for the UE registration to be granted authorization.

16ch-b. Account and Rating control by the CHF for the registration and open a CDR.

16ch-c. The CHF acknowledges by sending Charging Data Response [Initial] and the AMF, granting authorization to the registration.

17 to 21. Same steps as in Figure 6.1.4.1.2-1.

21ch-a. The AMF sends Charging Data Request [Termination] including CAG Identifier(s) to CHF for the UE successful registration.

21ch-b. The CHF closes the CDR for this registration.

21ch-c. The CHF acknowledges by sending Charging Data Response [Termination] to the AMF.

6.1.4.1.5	Deregistration – PEC charging

The Deregistration message flows are applicable to:

-	UE-initiated deregistration

-	Network-initiated Deregistration.

The following figure 6.1.4.1.5-1 describes a UE-initiated deregistration in PEC charging, based on figure 5.2.2.2.8.1 of TS 32.256 [11] description:

Figure 6.1.4.1.5-1: UE-initiated Deregistration – PEC charging

1.	Deregistration procedure initiated by UE.

1ch-a. The AMF sends Charging Data Request [Event] including CAG Identifier(s) to CHF for the UE deregistration.

1ch-b. The CHF creates the CDR for this deregistration.

1ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the AMF.

7: The AMF sends NAS message Deregistration Accept to UE. In case of Deregistration type is switch-off, the Deregistration Accept message is not sent.

The following figure 6.1.4.1.5-2 describes a Network-initiated Deregistration in PEC charging, based on figure 5.2.2.2.8.2 of TS 32.256 [11] description:

Figure 6.1.4.1.5-2: Network-initiated Deregistration – PEC charging

1.	AMF receives Nudm_UECM_DeregistrationNotification from UDM.

2.	Deregistration Request sent towards the UE. In case of Implicit Deregistration, the Deregistration Request message is not sent.

2ch-a. The AMF sends Charging Data Request [Event] including CAG Identifier(s) to CHF for the UE successful deregistration.

2ch-b. The CHF creates the CDR for this deregistration.

2ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the AMF.

[3-8]. Deregistration procedure steps.

6.1.4.2	Solution #2: Converged charging for number of UEs

6.1.4.2.1	General

This solution #2, charges vertical industry customer when "Number of UEs" threshold for the CAG Identifier is reached or "maximum number of UEs" for the CAG Identifier is reached, which addresses the Key Issue #2a, Key Issue #2b and Key Issue #2c.

6.1.4.2.2	Architecture description

A new NF NPNACF (Non-Public Network Admission Control Function) is introduced to the solution. The converged charging architecture for NPNACF (CTF) based solution as depicted in figure 6.1.4.2.2-1 is proposed for converged charging for number of UEs per CAG Identifier.

Figure 6.1.4.2.2-1: NPNACF (CTF) converged charging architecture

6.1.4.2.3	Procedures description

The figure 6.1.4.2.3-1 describes the high-level charging procedure for NPNACF (CTF) based converged charging.

Figure 6.1.4.2.3-1: NPNACF (CTF) based converged charging

0ch. CHF obtains the "maximum number of UEs" per CAG Identifier.

1. The NPNACF is configured per CAG Identifier, with "maximum number of UEs" and a set of "Number of UEs" intermediate thresholds (downwards and upwards triggers) under the "maximum number of UEs".

2. Number of UEs per CAG Identifier availability check and update procedure.

2ch-a: For a CAG Identifier one "Number of UEs" threshold is reached or "maximum number of UEs" is reached.

2ch-b: The NPNACF sends Charging Data Request to CHF for reporting number of UEs for the CAG Identifier.

2ch-c. Account and rating control based on number of UEs for the CAG Identifier.

2ch-d. CHF provides response to NPNACF by sending Charging Data Response.

NOTE :	The support for controlling number of UEs per CAG Identifier is dependent on SA WG2.

6.1.4.3	Solution #3: Converged charging for number of PDU sessions

6.1.4.3.1	General

This solution #3, charges vertical industry customer when "Number of PDU sessions" threshold for the CAG Identifier is reached or "maximum number of PDU sessions" for the CAG Identifier is reached, which addresses the Key Issue #2a, Key Issue #2b and Key Issue #2c.

6.1.4.3.2	Architecture description

A new NF NPNACF (Non-Public Network Admission Control Function) is introduced to the solution. The converged charging architecture for NPNACF (CTF) based solution as depicted in figure 6.1.4.2.2-1 is proposed for converged charging for number of PDU sessions per CAG Identifier.

6.1.4.3.3	Procedures description

The figure 6.1.4.3.3-1 describes the high-level charging procedure for NPNACF (CTF) based converged charging.

Figure 6.1.4.3.3-1: NPNACF (CTF) based converged charging-IEC

0ch. CHF obtains the "maximum number of PDU sessions" per CAG Identifier.

1. The NPNACF is configured per CAG Identifier, with "maximum number of PDU sessions" and a set of "Number of PDU sessions" intermediate thresholds (downwards and upwards triggers) under the "maximum number of PDU sessions".

2. PDU session establishment procedure per TS 32.255 [12] clause 5.2.2.2.2 steps 1~9.

3. The SMF sends NumOfPDUsUpdate_Request to NPNACF. The NPNACF checks availability and updates Number of PDU sessions per CAG Identifier.

3ch-a: For a CAG Identifier one "Number of PDU sessions" threshold is reached or "maximum number of PDU sessions" is reached.

3ch-b: The NPNACF sends Charging Data Request to CHF for reporting number of PDU sessions for the CAG Identifier.

3ch-c. Account and rating control based on number of PDU sessions for the CAG Identifier.

3ch-d. CHF provides response to NPNACF by sending Charging Data Response.

4. The NPNACF sends NumOfPDUsUpdate_Response to SMF.

5. PDU session establishment procedure per TS 32.255 [12] clause 5.2.2.2.2 steps 10~14.

NOTE :	The support for controlling PDU sessions per CAG Identifier is dependent on SA WG2.

6.1.4.4	Solution #4: Converged charging for number of UEs or PDU sessions using separate CCS

6.1.4.4.1	General

This solution addresses the key issues #2a, 2b and 2c covering potential requirements REQ-eNPN_CH_PNI-NPN_AC-03 and REQ-eNPN_CH_PNI-NPN_AC-03 based on distributed charging architecture split between two Converged Charging Systems (CCS) and exchanges between them.

Two types of subscribers are considered in the context of NPN: the NPN-SC, and the individual end-user "UE" served under the NPN. Each type of subscriber is handled under a dedicated CCS: "NPN CCS" and "UE CCS" respectively.

UE CCS performs charging functionalities for individual UEs, and includes in particular:

-	CHF exposing Nchf services associated to individual UEs.

-	ABMF/RF hosting individual UE account balance and subscribed services, as well as the rates for services.

NPN CCS performs charging functionalities for NPN-SC, and includes in particular:

-	CHF exposing Nchf services associated to individual NPN-SCs.

-	ABMF/RF hosting individual NPN-SC account balance and subscribed services, as well as the rates for services.

How the account balance and subscribed services are provisioned (for both NPN-SC and individual UE) is unspecified and expected to be covered at BSS and OSS level.

A new reference point (ChX) is defined between CHF in NPN CCS and the CHF in UE CCS enabling association between individual UE charging and NPN-SC charging. This association is achieved by UE CCS responsible for individual UE charging consuming this new charging service exposed by the NPN-SC CCS.

The new reference (ChX) may use the Nchf_ConvergedCharging service with extensions or use a completely new Nchf service.

For UE:

-	the 5G data connectivity charging from SMF is specified in 3GPP TS 32.255 [12];

-	the 5G connection and mobility charging from AMF is specified in 3GPP TS 32.256 [11];

and uses Nchf_ConvergedCharging service exposed by CHF in UE CCS.

6.1.4.4.2	Architecture description

Figure 6.1.4.4.2-1: Distributed charging architecture in reference point representation

Figure 6.1.4.4.2-2: Distributed charging architecture in service based representation

6.1.4.4.3	Procedures description

The figure 6.1.4.4.3-1 describes the high-level charging procedure for UE PDU session converged charging influenced by NPN converged charging using distributed charging architecture.

Figure 6.1.4.4.3-1: UE PDU session converged Charging influenced by NPN converged charging

1.	Steps according to 3GPP TS 32.255 [12] clause 5.2.2.2.2 for individual UE charging covered by UE CCS.

9ch1-a.	Charging data request [Initial]: SMF (CTF) sends the request to the UE CHF for the service to be granted authorization to start, and to reserve the number of units if required.

9ch1-b.	Account, Rating, Reservation control: UE CCS rates the requests either based on the number of units requested or on internal unit determination, if possible, the NPN CCS performs the corresponding reservation of funds on the UE's account balance.

9ch1-c. 	Determine NPN CHF: UE CCS determines that the UE PDU session charging needs input from NPN charging, the determination can for example be based on CAG identifier.

9ch2-a.	Charging service request [Initial]: UE CCS (acting like a CTF) sends the request to the NPN CHF for the service to be granted authorization to start, and to reserve the number of units if required.

9ch2-b.	Account, Rating, Reservation control: NPN CCS rates the requests either based on the number of units requested or on internal unit determination, if possible, the NPN CCS performs the corresponding reservation of funds on the NPN-SC's account balance. Examples of type of units could be "number of PDU sessions", the number of units would always be one if there is a one-to-one relationship between charging data request and charging service request i.e., (the trigger for charging service request is reception of a charging data request in the UE CCS).

9ch2-c.	Open CDR: based on policies, the NPN CHF opens a CDR for the NPN-SC related to the service.

9ch2-d.	Charging service response [Initial]: The NPN CHF grants authorization to UE CHF for the service to start, with the reserved number of units e.g., "number of PDU sessions".

9ch1-d.	Open CDR: based on policies, the UE CHF opens a CDR for the UE related to the service.

9ch2-e.	Charging data response [Initial]: The UE CHF grants authorization to SMF (CTF) for the service to start, with the reserved number of units e.g., "number of bytes".

2.	PDU session establishment procedure per TS 32.255 [12] clause 5.2.2.2.2 from step 10.

The figure 6.1.4.4.3-2 describes the high-level charging procedure for UE registration converged charging influenced by NPN converged charging using distributed charging architecture.

Figure 6.1.4.4.3-2: UE Registration converged charging influenced by NPN converged charging

1.	Steps according to TS 32.256 [11] clause 5.2.2.2.4 for individual UE charging covered by UE CCS.

16ch1-a.	Charging data request [Initial]: AMF (CTF) sends the request to the UE CHF for the service to be granted authorization to start, and to reserve the number of units if required.

16ch1-b.	Account, Rating, Reservation control: UE CCS rates the requests, if possible, the NPN CCS performs the corresponding reservation of funds on the UE's account balance.

16ch1-c. 	Determine NPN CHF: UE CCS determines that the UE registration charging needs input from NPN charging, the determination can for example be based on CAG identifier.

16ch2-a.	Charging service request [Initial]: UE CCS (acting like a CTF) sends the request to the NPN CHF for the service to be granted authorization to start, and to reserve the number of units if required.

16ch2-b.	Account, Rating, Reservation control: NPN CCS rates the requests, if possible, the NPN CCS performs the corresponding reservation of funds on the NPN-SC's account balance. Examples of type of units could be "number of UE registrations", the trigger for charging service request could be reception of a charging data request in the UE CCS.

16ch2-c.	Open CDR: based on policies, the NPN CHF opens a CDR for the NPN-SC related to the service.

16ch2-d.	Charging service response [Initial]: The NPN CHF grants authorization to UE CHF for the registration to continue.

16ch1-d.	Open CDR: based on policies, the UE CHF opens a CDR for the UE related to the service.

16ch2-e.	Charging data response [Initial]: The UE CHF grants authorization to AMF (CTF) for the registration to continue.

2.	Registration procedure per TS 32.256 [11] clause 5.2.2.2.4 from step 17.

6.1.5	Evaluation

6.1.5.1	Evaluation of possible solutions for end user charging for PNI-NPN network usage of access

Solution #1 has proposed end user charging for PNI-NPN network usage of access, enabling the AMF to collect charging information which including CAG Identifier(s) per UE registration to the 5GS, and the charging information for end user usage of access in PNI-NPN network reported by AMF is obtained by CHF.

6.1.5.2	Evaluation of possible solutions for converged charging for number of UEs

Solution #2 and #4 propose solutions on converged charging for number of UEs. The following table lists the characteristics of the two solutions.

Table 6.1.5.2-1: Comparison between Solution #2 and #4

|  | NF controlling number of UEs per PNI-NPN | NF acts as a CTF |
| --- | --- | --- |
| Solution #2: NPNACF (CTF) based converged charging for number of UEs | NPNACF (New NF) | NPNACF |
| Solution #4: Separate CCS based converged charging for number of UEs | CHF (New NPN CCS) | AMF |

The principles of solution #2 are inline with the NSACF charging of Network Slicing in TR 32.847 [13], and the identification of the PNI-NPN can be based on e.g. CAG Identifier.

The principles of solution #4, are inline the solution for network slice charging based on 5G data connectivity in TS 32.255 [12] annex D.

6.1.5.3	Evaluation of possible solutions for converged charging for number of PDU sessions

Solution #3 and #4 propose solutions on converged charging for number of PDU sessions. The following table lists the characteristics of the two solutions.

Table 6.1.5.3-1: Comparison between Solution #3 and #4

|  | NF controlling number of PDU sessions per NPI-NPN | NF acts as a CTF |
| --- | --- | --- |
| Solution #3: NPNACF (CTF) based converged charging for number of PDU sessions | NPNACF (New NF) | NPNACF |
| Solution #4: Separate CCS based converged charging for number of PDU sessions | CHF (New NPN CCS) | SMF |

The principles of solution #3 are inline with the NSACF charging of Network Slicing in TR 32.847 [13], and the identification of the PNI-NPN can be based on e.g. CAG Identifier.

The principles of solution #4, are inline the solution for network slice charging based on 5G data connectivity in TS 32.255 [12] annex D.

6.1.6	Conclusion

Based on the evaluation in clause 6.1.5.1, solution #1 is the only solution for end user charging for PNI-NPN network usage of access. The solution #1 is selected to the normative work.

Based on the evaluation in clause 6.1.5.2, solution #2 needs support for controlling number of UEs for PNI-NPN per CAG Identifier which is not currently supported by SA WG2. Solution #4 needs support a new interface between CHF in NPN CCS and CHF in UE CCS which is not currently supported, and it also has some impact on PDU session establishment procedure in SMF, and registration procedure in AMF, which require converged charging. It is concluded that using converged charging mechanism for simultaneous number of UEs per network slice to support converged charging for number of UEs per PNI-NPN in normative work of release 18. It can solve a unique S-NSSAI is allocated for one PNI-NPN. Multiple PNI-NPNs share the same S-NSSAI is not supported in this release.

Based on the evaluation in clause 6.1.5.3, solution #3 needs support for controlling number of PDU sessions for PNI-NPN per CAG Identifier which is not currently supported by SA WG2. Solution #4 needs support a new interface between CHF in NPN CCS and CHF in UE CCS which is not currently supported, and it also has some impact on PDU session establishment procedure in SMF, and registration procedure in AMF, which require converged charging. It is concluded that using converged charging mechanism for simultaneous number of PDU sessions per network slice to support converged charging for number of PDU sessions per PNI-NPN in normative work of release 18. It can solve a unique S-NSSAI is allocated for one PNI-NPN. Multiple PNI-NPNs share the same S-NSSAI is not supported in this release.

6.2	Topic 2: Converged charging for data connectivity in PNI-NPN

6.2.1	Use cases

6.2.1.1	Use case #1a: End user charging for PNI-NPN data connectivity

It is important for NPN-OP to collect usage for PNI-NPN data connectivity for the purpose of charging or statistics.

The potential charging requirements for this UC are: REQ-eNPN_CH_PNI-NPN_DC-01, REQ-eNPN_CH_PNI-NPN_DC-02.

6.2.2	Potential charging requirements

REQ-eNPN_CH_PNI-NPN_DC-01: The 5G system should support converged charging per UE for PNI-NPN data connectivity.

REQ-eNPN_CH_PNI-NPN_DC-02: The 5G system should support converged charging per vertical industry consumer for PNI-NPN data connectivity.

REQ-eNPN_CH_PNI-NPN_DC-03: The 5G system should support converged charging per CAG Identifier for PNI-NPN data connectivity.

6.2.3	Key issues

This key issue is for investigating how to support converged charging for data connectivity in PNI-NPN considering REQ-eNPN_CH_PNI-NPN_DC-01, REQ-eNPN_CH_PNI-NPN_DC-02, and REQ-eNPN_CH_PNI-NPN_DC-03. This investigation covers the following:

-	Key Issue #2c: Identification of the charging information collection and reporting for converged charging for PNI-NPN data connectivity.

6.2.4	Possible solutions

6.2.4.1	Solution #1: End user charging for PNI-NPN data connectivity

6.2.4.1.1	General

This solution #1 which relying on 5G data connectivity converged charging architecture defined in TS 32.255 [12], with the extension of including CAG Identifier(s) for PNI-NPN network data connectivity charging per UE, addresses the Key Issue #2c.

6.2.4.1.2	Reference architecture

In this solution, SMF collects charging information including CAG Identifier(s) per UE, and the charging information for end user charging for PNI-NPN data connectivity reported by SMF is obtained by CHF.

Figure 6.2.4.1.2-1: 5G data connectivity converged charging architecture 
defined in TS 32.255 [12]

6.2.4.1.3	Message flows

The message flows are the same with the 5G data connectivity per PDU session charging specified in the TS 32.255 [12].

SMF reports the Charging Data Request contains CAG Identifier(s) to the CHF in the PNI-NPN.

6.2.5	Evaluation

Solution #1 has proposed end user charging for PNI-NPN data connectivity, enabling the SMF to collect charging information which including CAG Identifier(s) per UE, and the charging information for end user charging for PNI-NPN data connectivity reported by SMF is obtained by CHF. Solution #1 needs support for SMF to collect CAG Identifier(s) and needs support by CT WG4. Solution #1 can solve a unique CAG Identifier(s) is allocated for one PNI-NPN, but it does not support multiple PNI-NPNs sharing the same CAG Identifier(s).

In addition to CAG Identifier(s), the charging information collected by SMF in solution #1 can also be S-NSSAI or dedicated DNNs, or dedicated DNN and S-NSSAI combination to identify PNI-NPN.

6.2.6	Conclusion

Based on the evaluation in clause 6.2.5, solution #1 is the only solution for end user charging for PNI-NPN data connectivity. The solution #1 based on S-NSSAI, dedicated DNNs, or a combination to identify PNI-NPN is selected to the normative work, any other attribute is dependent on support from SMF.

7	Conclusions and recommendations

The functionality, architecture, and charging modes for Non-public networks as the background are described in the present technical report. In addition, use cases, potential charging requirements, key issues are identified and possible solutions for SNPN and PNI-NPN are developed and evaluated. The present document made conclusions on the following aspects:

-	Converged charging for data connectivity in SNPN, in clause 5.1.6;

-	Converged charging for access connection in SNPN, in clause 5.2.6;

-	Converged charging for data connectivity in PLMN, in clause 5.3.6;

-	Converged charging for access connection in PNI-NPN, in clause 6.1.6;

-	Converged charging for data connectivity in PNI-NPN, in clause 6.2.6.

To support converged charging for Non-public networks in normative work, it is recommended to specify the requirements, architectures and solutions for the aspects mentioned above based on the conclusions of the corresponding solutions documented in clauses 5.1.6, 5.2.6, 5.3.6, 6.1.6 and 6.2.6 of the present document.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | SA5#142e | S5-222381 |  |  |  | Initial skeleton | 0.0.0 |
| 2022-04 | SA5#142e | S5-222395 S5-222392 S5-222394 |  |  |  | Skeleton update Add scope Add reference | 0.1.0 |
| 2022-05 | SA5#143e | S5-223082 S5-223083 S5-223709 S5-223706 S5-223708 S5-223710 |  |  |  | Introduction of general background Introduction of non-public networks functionality and architecture Introduction of charging modes Add the use case for SNPN Add charging scenarios and key issues for SNPN on converged charging for access connection Add charging scenarios and key issues for PNI-NPN on converged charging for access connection | 0.2.0 |
| 2022-07 | SA5#144e | S5-224187 S5-224188 S5-224189 S5-224480 |  |  |  | Addition of solution for end user charging for PNI-NPN network access and usage Adding charging scenarios and key issues for PNI-NPN in topic 1 Addition of solution for converged charging for number of UEs The New Topic about access to PLMN services via SNPN | 0.3.0 |
| 2022-08 | SA5#145e | S5-225652 S5-225764 S5-225188 S5-225765 S5-225190 S5-225766 S5-225647 S5-225653 S5-225438 S5-225439 |  |  |  | Add end user charging solution to SNPN network access and usage Introduction of use case and solution on number of UEs New charging scenarios and key issues for PNI-NPN topic 1 Adding solution on number of PDU sessions for PNI-NPN New charging scenarios and key issues for SNPN topic 2 Adding solution on number of PDU sessions for SNPN Addition of converged charging using separate CCS Add the solution for SNPN access the PLMN services Correction on the use cases description in the Topic 1 Correction on the use cases description in the Topic 3 | 0.4.0 |
| 2022-09 | SA#97e | SP-220844 |  |  |  | EditHelp review, presented for information | 1.0.0 |
| 2022-11 | SA5#146 | S5-226706 S5-226707 S5-226708 S5-226709 |  |  |  | Add evaluation and conclusion for PNI-NPN topic 1 Addition of solution in clause 5.2 using separate CCS Add the solution for key issue #1c Add the solution for accessing the SNPN service via PLMN | 1.1.0 |
| 2023-01 | SA5#146Bis-e | S5-231255 S5-231268 S5-231269 S5-231033 S5-231034 S5-231035 |  |  |  | Correction on the SNPN Topic 1 and SNPN Topic 3 Evaluation and Conclusion for SNPN Topic 1 Evaluation and Conclusion for SNPN Topic 3 Add evaluation and conclusion for SNPN topic 2 Update PNI-NPN topic 1 for consistency Add Conclusions and recommendations | 1.2.0 |
| 2023-03 | SA5#147 | S5-232823 S5-232824 S5-232826 S5-232827 S5-232828 S5-232829 S5-232825 S5-232830 S5-232831 S5-232832 S5-232833 |  |  |  | Rel-18 pCR 28.828 Correction on solution #2 for PNI-NPN topic 1 Rel-18 pCR 28.828 Solve Editor's Note in PNI-NPN topic 1 Rel-18 pCR 28.828 Correction on conclusion for PNI-NPN topic 1 Rel-18 pCR 28.828 Correction on the solution 3 in the Topic 1 Rel-18 pCR 28.828 Remove editor's note for the Topic 1 Rel-18 pCR 28.828 Solve Editor's Note in SNPN topic 2 Rel-18 pCR 28.828 Correction on conclusion for SNPN topic 2 Rel-18 pCR 28.828 Correction on the solution 2 in the Topic 3 Rel-18 pCR 28.828 Remove editor's note for the Topic 3 Rel-18 pCR 28.828 Add converged charging for data connectivity in PNI-NPN Rel-18 pCR 28.828 Update conclusions and recommendations | 1.3.0 |