| 3GPP TR 28.839 V1.0.0 (2023-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on charging aspects for time sensitive networking (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Background	9

4.1	General	9

4.2	Business Roles for the TSN Charging	10

5	Charging scenarios and potential requirements	11

5.1	Exposure of Time Synchronization	11

5.1.1	General	11

5.1.2	Use Case	11

5.1.2.1	Use Case #1.1 Exposure of Time Synchronization Service	11

5.1.3	Potential charging requirements	11

5.1.4	Key issues	11

5.2	Enablers for Time Sensitive Communications and Time Synchronization	12

5.2.1	General	12

5.2.2	Use Case	12

5.2.2.1	Use Case #2.1 Enablers for Time Sensitive Communications and Time Synchronization	12

5.2.3	Potential charging requirements	13

5.2.4	Key issues	13

5.3	Time Sensitive Communication	13

5.3.1	General	13

5.3.2	Use Case	13

5.3.2.1	Use Case #3.1 Time Sensitive Communication via TSN 5GS Bridge	13

5.3.3	Potential charging requirements	14

5.3.4	Key issues	14

5.4	5GS Bridge Management and Configuration Charging	14

5.4.1	General	14

5.4.2	Use Case	14

5.4.2.1	Use Case #4.1 TSN 5GS Bridge management charging	14

5.4.2.2	Use Case #4.2 TSN 5GS Bridge configuration charging	15

5.4.3	Potential charging requirements	15

5.4.4	Key issues	15

6	Key issues	16

6.1	Key issue #x: <key issue name>	16

6.2	Key issues and solutions mapping	16

7	Solutions for Key issues	16

7.1	 Solution #1: Exposure of Time Synchronization Service via NEF	16

7.1.1	General	16

7.1.2	Architecture description	17

7.1.3	Flow description	17

7.2	 Solution #2: 5GS Bridge management and configuration charging via SMF	18

7.2.1	General	18

7.2.2	Architecture description	18

7.2.3	Flow description	18

7.3	 Solution #3: 5GS Bridge management and configuration charging	21

7.3.1	General	21

7.3.2	Architecture description	21

7.3.3	Flow description	21

7.4	 Solution #4: Time Sensitive Communications charging from TSN station to TSN station	22

7.4.1	General	22

7.4.2	Architecture description	23

7.4.3	Flow description	23

7.4.3.1	TSN Bridge/End station to TSN Bridge/End station of the Time Sensitive Communication	23

7.5	 Solution #5: Time Sensitive Communications charging	25

7.5.1	General	25

7.5.2	Architecture description	26

7.5.3	Flow description	26

7.5.3.1	TSN Bridge/End station to the TSN System of the Time Sensitive Communication	26

7.6	 Solution #6: Enablers for TSC and TS Charging	27

7.6.1	General	27

7.6.2	Architecture description	28

7.6.3	Flow description	28

7.6.3.1	TSCTSF directly interact with the AF	28

7.7	 Solution #7: Exposure of Time Synchronization Service with trusted AF	29

7.7.1	General	29

7.7.2	Architecture description	29

7.7.3	Flow description	29

7.y	 Solution Evaluation	31

7.z	Solution Conclusions	31

8	Conclusions and recommendations	31

Annex <A>: Change history	32

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

Should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

Can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

Will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies the charging aspects for the support of Time Sensitive Networking based on requirements from TS 22.261 [2] and TS 22.104 [7] of 5G system and vertical domains, the architecture, functional and protocol to enable Time Sensitive Communication and Time Synchronization, described in the TS 23.501 [3], TS 23.502 [4], TS 23.503 [5], TR 23.734 [6] and TS 24.519 [8].

The following are studied:

-	possible charging scenarios and requirements for 5G System integration as a bridge in TSN network;

-	possible charging scenarios and requirements for the Time Sensitive Communications and Time Synchronization service;

-	possible new charging aspects for the following key items;

-	QoS requirements for TSN (e.g. support for TSC QoS Flows, QoS mapping from TSN QoS information to 5GS QoS profile);

-	Edge computing scenario support in TSN Transport (e.g. DS-TT and NW-TT);

-	charging enhancement (e.g. new triggers) due to policy control for TSN traffic which may impact the charging;

-	potential charging solutions for the above charging scenarios and requirements and taking in account TSN application requirements (e.g. instance survival time).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TR 23.734: "Study on enhancement of 5G System (5GS) for vertical and Local Area Network (LAN) services".

[7]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[8]	3GPP TS 24.519: "5G System (5GS); Time-Sensitive Networking (TSN) Application Function (AF) to Device-Side TSN Translator (DS-TT) and Network-Side TSN Translator (NW-TT) protocol aspects; Stage 3."

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

survival time: the time that an application consuming a communication service may continue without an anticipated message. as defined in TS 22.261 [2].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

N84	Reference point between TSCTSF and NEF.

N85	Reference point between TSCTSF and NEF.

N86:	Reference point between TSCTSF and AF.

N87:	Reference point between TSCTSF and UDR.

Ntsctsf		Service-based interface exhibited by TSCTSF.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GS	5G System

ASTI	Access Stratum TIme

CNC	Centralized Network Configuration

DS-TT	Device-Side TSN Translator

MNO	Mobile Network Operator

NW-TT	Network-Side TSN Translator

TS	Time Synchronization

TSC	Time Sensitive Communications

TSCTSF	Time Sensitive Communication and Time Synchronization function

TSN	Time-Sensitive Networking

TSN AF	Time Sensitive Networking Application Function

TSN-SC	TSN Service Customer

TSN-SP	TSN Service Provider

# 4	Background

## 4.1	General

As per the requirements described in the TS 22.261 [2] and TS 22.104 [7] about clock synchronization, time-sensitive communication, the corresponding description to support of integration with TSN and enablers for Time Sensitive Communications and Time Synchronization are specified TS 23.501[3] clause 5.27 and clause 5.28.

-	5GS acts as a Layer 2 Ethernet Bridge. When integrated with IEEE TSN network, 5GS functions acts as one or more TSN Bridges of the TSN network. The configuration of the 5GS Bridge can be provided by TSN AF and CNC.

-	5G System features that can be used independently or in combination to enable time-sensitive communication and time synchronization.

The figure 4.1.1 describes the architecture for 5GS appearing as TSN bridge, specified in the clause in 4.4.8.2 TS 23.501[3] and the figure 4.1.2 describes the architecture for AF requested support of Time Sensitive Communication and Time Synchronization, specified in the clause 4.4.8.3 TS 23.501[3].

Figure 4.1-1: System architecture view with 5GS appearing as TSN bridge

Figure 4.1-2: Architecture to enable Time Sensitive Communication and Time Synchronization services

## 4.2	Business Roles for the TSN Charging

5G system as the 5GS Bridge to support the integration with TSN, the TSN charging can be built upon the business roles as following, related to 5G Bridge management, TSN traffic transmission over the PDU Session:

- 	Mobile Network Operator (MNO): provides 5GS system as the 5GS Bridge.

- 	TSN Service Provider (TSN-SP): provides communication services for TSN Bridge use, e.g. enterprise.

- 	TSN Service Customer (TSN-SC): consumes communication services for TSN Bridge use, e.g. end user, enterprise.

5G System features that can be used independently or in combination to enable time-sensitive communication and time synchronization, the TSN charging can be built upon the business roles as following, related to the time synchronization configuration and capability, including the exposure of the time sensitive service, time-sensitive communication.

- 	Mobile Network Operator (MNO): provides the 5GS system features.

- 	TS Service Provider (TS-SP): provides communication services for time-sensitive communication and time synchronization, e.g. enterprise.

- 	TS Service Customer (TS-SC): consumes communication services for time-sensitive communication and time synchronization, e.g. end user, enterprise.

In deployments, there could be business scenarios where one or more of the business roles are supported by a single enterprise. The present document does not impose any restrictions to the possible deployment scenarios.

# 5	Charging scenarios and potential requirements

## 5.1	Exposure of Time Synchronization

### 5.1.1	General

5G system supports time synchronization service that can be activated and deactivated by AF. In order to enable Time Sensitive Communication AF requested time sensitive communication and time synchronization services, The TSCTSF/NEF exposes 5GS capability to support Time Synchronization service. The TSCTSF (directly or via NEF) exposes the 5GS and/or UE availability and capabilities for synchronization service to the AF as described in TS 23.502 [4].

Exposure of time synchronization comprises the following capabilities:

-	The AF may learn 5GS and/or UE availability and capabilities for time synchronization service.

-	The AF controls activation and deactivation of the time synchronization service for the target UE(s).

### 5.1.2	Use Case

#### 5.1.2.1	Use Case #1.1 Exposure of Time Synchronization Service

TS-SC actives the time synchronization service via AF and the AF provides request to the NEF and the NEF forwards the parameters to the TSCTSF. The AF trusted by the operator provides these parameters to the TSCTSF directly.

TS-SC de-actives the time synchronization service via AF and the AF provides request to the NEF and the NEF forwards the parameters to the TSCTSF or directly to TSCTSF.

The 5GS provides the requested 5GS and/or UE availability and capabilities for TS-SC.

The MNO may charge the TS-SC based on the exposure events for Time Synchronization Service.

The corresponding potential charging requirements and key issues: REQ-CH_ETS-01, REQ-CH_ ETS -02, and the Key Issue #1a, Key Issue #1b.

### 5.1.3	Potential charging requirements

REQ-CH_ETS-01: The 5GS charging system should support the Exposure of Time Synchronization charging per 5GS bridge.

REQ-CH_ETS-02: The 5GS charging system should support the Exposure of Time Synchronization charging per TSN.

### 5.1.4	Key issues

The following key issues are identified:

-	Key Issue #1a: Identify the charging scenarios and network function which can be support the Exposure of Time Synchronization charging.

-	Key Issue #1b: Identify the charging information which can be support the Exposure of Time Synchronization charging.

## 5.2	Enablers for Time Sensitive Communications and Time Synchronization

### 5.2.1	General

5G System features can be used independently or in combination to enable time-sensitive communication and time synchronization as specified in the TS 23.501[3], including the following:

-	Delay-critical GBR;

-	A hold and forward mechanism to schedule traffic;

-	TSC Assistance Information: describes TSC flow traffic characteristics

-	Time Synchronization: describes how 5GS can operate as a PTP Relay

### 5.2.2	Use Case

#### 5.2.2.1	Use Case #2.1 Enablers for Time Sensitive Communications and Time Synchronization

If the AF is considered to be trusted by the operator, the AF could interact directly with TSCTSF, using the same service as the NEF for Time Sensitive Communications and Time Synchronization. The TSCTSF (directly or via NEF) exposes the 5GS and/or UE availability and capabilities for synchronization service to the AF as described in clause 4.15.9.2 of TS 23.502 [4].

For supporting time synchronization service requested by TS-SC, the 5GS is configured to operate in one or multiple PTP instances, including the support for derivation of Uu time synchronization error budget.

For supporting transmission gate schedules requested by TS-SC, the 5GS Bridge is required to provide Bridge Delays for each port pair and traffic class of the 5GS bridge to an TSN system

For supporting the TSC traffic characteristics requested by TS-SC, TSC QoS Flows use a Delay-critical GBR resource type and TSC Assistance Information.

The MNO may charge the TS-SC based on the enabler event for Time Synchronization Service.

The corresponding potential charging requirements and key issues: REQ-CH_ETS-01, and the Key Issue #2a, Key Issue #2b.

### 5.2.3	Potential charging requirements

REQ-CH_ENTS-01: The 5GS charging system should support the Exposure of Time Synchronization charging per TSN.

### 5.2.4	Key issues

The following key issues are identified:

-	Key Issue #2a: Identify the charging scenarios and network function which can be support the enabler of Time Synchronization charging.

-	Key Issue #2b: Identify the charging information which can be support the enabler of Time Synchronization charging.

## 5.3	Time Sensitive Communication

### 5.3.1	General

The Time Sensitive Communication related features are supported based on AF request are described in TS 23.501[3]. For each 5GS Bridge of a TSN network, the port on NW-TT support the connectivity to the TSN network, the ports on DS-TT side are associated to the PDU Session providing connectivity to the TSN network. The TSN End station/TSN Bridge support the Time Sensitive Communication over the 5GS bridge, via the DS-TT, UE, UPF, NW-TT based on the PDU session.

The granularity of the 5GS TSN bridge is per UPF for each network instance or DNN/S-NSSAI.

### 5.3.2	Use Case

#### 5.3.2.1	Use Case #3.1 Time Sensitive Communication via TSN 5GS Bridge

During the PDU session, Time sensitive communication between the TSN End stations via DS-TT and NW-TT.

If the TSN AF determines that the TSN stream is for UE-Network communication. The UE1/DS-TT sends the traffic data from TSN stations via UPF/NW-TT to TSN network for the Time Sensitive Communication over TSN 5GS Bridge.

If the TSN AF determines that the TSN stream is for UE-UE communication (i.e. ingress and egress ports are in DS-TTs), the TSN AF divides the stream into one uplink stream and one or more downlink streams and provides the streams on AF Session basis to the PCF(s). The SMF applies local switching in order to enable UPF locally forward uplink stream from one PDU session as downlink stream in another PDU session.

The MNO may charge the UE based on the traffic data volume.

The MNO may charge the TSN based on the traffic data volume.

The corresponding potential charging requirements and key issues: REQ-CH_TSC-01, REQ-CH_TSC-02, REQ-CH_TSC-03 and the Key Issue #3a, Key Issue #3b.

### 5.3.3	Potential charging requirements

REQ-CH_TSC-01: The 5GS charging system should support the data connectivity charging for Time Sensitive Communication per 5GS bridge.

REQ-CH_TSC-02: The 5GS charging system should support the data connectivity charging for Time Sensitive Communication per TSN.

REQ-CH_TSC-03: The 5GS charging system should support the data connectivity charging for Time Sensitive Communication per UE based on the data volume.

### 5.3.4	Key issues

The following key issues are identified:

-	Key Issue #3a: Identify the charging scenarios and network function which can be support the Time Sensitive Communication charging.

-	Key Issue #3b: Identify the charging information which can be support the Time Sensitive Communication charging.

## 5.4	5GS Bridge Management and Configuration Charging

### 5.4.1	General

To support of integration with TSN, the 5GS acts as a Layer 2 Ethernet Bridge. The TSN AF in 5GS is responsible to receive the bridge information of 5GS Bridge from 5GS, as well as register or update this information to the TSN CNC. The bridge information of 5GS Bridge is used by the TSN network (via TSN CNC) to make appropriate management and configuration for the 5GS Bridge.

### 5.4.2	Use Case

#### 5.4.2.1	Use Case #4.1 TSN 5GS Bridge management charging

When the TSN Service Customer (TSN-SC) initiates the TSN services, the 5GS reports the 5GS Bridge information and receiving the configuration via TSN AF to the TSN network, including but not limited to the following aspects specified in the TS 23.501[3].

-	5GS Bridge information;

-	Capabilities of 5GS Bridge;

-	Topology of 5GS Bridge;

-	Traffic classes and their priorities per port;

-	Stream Parameters in order to support PSFP.

The MNO can charge TSN-SP for the interaction between the 5GS and TSN for the 5GS bridge management.

The corresponding potential charging requirements and key issues: REQ-CH_BC-01 and the Key Issue #4a, Key Issue #4b.

#### 5.4.2.2	Use Case #4.2 TSN 5GS Bridge configuration charging

Based on the agreement between the TSN Service Provider (TSN-SP) and MNO, the pre-configuration of 5GS bridge may be deployed in the PCF or UPF.

When the TSN Service Customer (TSN-SC) initiates the TSN services, the TSN network can configure the 5GS Bridge via TSN AF or TSCTSF, including but not limited to the following aspects specified in the TS 23.501[3].

-	Bridge ID of 5GS Bridge;

-	Configuration information of scheduled traffic on ports of DS-TT and NW-TT;

-	Chassis ID of 5GS Bridge;

-	Traffic forwarding information;

-	Configuration information per stream.

The MNO can charge TSN-SP for the interaction between the 5GS and TSN for the 5GS bridge configuration during PDU session.

The corresponding potential charging requirements and key issues: REQ-CH_BC-02and the Key Issue #4c, Key Issue #4d.

### 5.4.3	Potential charging requirements

REQ-CH_BC-01: The 5GS charging system should support the 5GS bridge management charging per 5GS bridge.

REQ-CH_BC-02: The 5GS charging system should support the 5GS bridge configuration charging per 5GS bridge.

### 5.4.4	Key issues

The following key issues are identified:

-	Key Issue #4a: Identify the charging scenarios and network function which can be support the 5GS bridge management charging.

-	Key Issue #4b: Identify the charging information which can be support the 5GS bridge management charging.

-	Key Issue #4c: Identify the charging scenarios and network function which can be support the 5GS bridge configuration charging.

-	Key Issue #4d: Identify the charging information which can be support the 5GS bridge configuration charging.

# 6	Key issues

## 6.1	Key issue #x: <key issue name>

## 6.2	Key issues and solutions mapping

# 7	Solutions for Key issues

## 7.1		Solution #1: Exposure of Time Synchronization Service via NEF

### 7.1.1	General

The solution #1, addresses the Key Issue #1a and #1b, covering the REQ-CH_ETS-02.

Based on the clause 4.15.9.2 Exposure of UE availability for Time Synchronization service, specified in the TS 23.502 [4], the AF subscribes to notifications and to explicitly cancel a previous subscription for UE availability for time synchronization service, via the following NEF services:

-	Nnef_TimeSynchronization create or update time synchronization configuration as well as to activate and deactivate the time synchronization service for which the NEF uses service operations provided by TSCTSF. Described in TS 23.501 [3] clause 5.27.1.8.

-	Nnef_ASTI service to control the 5G access stratum time distribution configuration, for which the NEF uses service operations provided by TSCTSF. Described in TS 23.501 [3] clause 5.27.1.8

-	Nnef_AFsessionWithQoS to reserve resources for an AF session. Described in TS 23.503 [20] clause 6.1.3.22

### 7.1.2	Architecture description

Figure 7.1.2-1: NEF converged charging architecture

### 7.1.3	Flow description

The figure 7.1.3-1 describes the high-level charging procedure for Time Synchronization service based on NEF.

Figure 7.1.3-1: AF Request via NEF using PEC

1.	NEF receives a Time Synchronization Service Request from an AF, including a list of UE identities (GPSIs) or Groups of UEs identified by an External Group Identifier that further define the subset of the target UEs and AF-Service-Identifier, or DNN and optionally S-NSSAI which used to indicate the TS-SC.

2.	NEF performs the actions needed to fulfil the Time Synchronization Service Request.

3.	If authorized, the NEF continues the Time Synchronization Service Request processing and sends the Time Synchronization Service Response to the AF.

3ch-a. The NEF sends Charging Data Request [Event] to CHF for the received Time Synchronization Service Request.

3ch-b. The CHF creates a CDR for this Time Synchronization Service Request.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the NEF.

Editor’s note: the subscriber id is ffs.

## 7.2		Solution #2: 5GS Bridge management and configuration charging via SMF

### 7.2.1	General

The solution #2, addresses the Key Issue #4a and #4b, covering the REQ-CH_BC-01.

Based on the clause 4.3.2 and clause 4.3.3 Non-roaming and Roaming with Local Breakout for UE requested PDU Session Establishment and PDU modification, specified in the TS 23.502 [4], the SMF supports 5GS Bridge management and configuration.

### 7.2.2	Architecture description

Figure 7.2.2-1: SMF converged charging architecture

### 7.2.3	Flow description

The figure 7.2.3-1 describes the high-level charging procedure for 5GS bridge information reporting based on SMF during the PDU session establishment.

Figure 7.2.3-1: 5GS Bridge management and configuration charging via SMF during the PDU session establishment

1-9.	PDU session establishment charging, based on figure 5.2.2.2.2-1 TS 32.255 [X] description. During SM Policy Association, the PCF may provide policy control request trigger for 5GS Bridge Information as defined in clause 6.1.3.5 of TS 23.503 [5]. The SMF may perform a Session Management Policy Modification procedure to report some event to the PCF that has previously subscribed.

9ch-a. The SMF sends Charging Data Request [Initial] to CHF for start of the PDU session.

9ch-b. The CHF open a CDR for this PDU session.

9ch-c. The CHF acknowledges by sending Charging Data Response [Initial] to the SMF. The CHF may provide charging related trigger for 5GS Bridge Information reporting. e.g. enable/disable the trigger or change the category.

10-20.	PDU session establishment, figure 4.3.2.2.1.1 TS 23.502 [4] description.

20ch-a. When the trigger for 5GS Bridge information available is armed, the SMF sends Charging Data Request [Update] to CHF, in order to support the integrate with IEEE TSN, if the 5GS Bridget information available is immediate report, with 5GS Bridge information (e.g. 5GS user-plane Node ID, port number of the DS-TT port, MAC address of the DS-TT Ethernet port for Ethernet PDU Session type, UE IP address for IP PDU Session type and UE-DS-TT Residence Time (if available) as provided by the UE).

20ch-b. The CHF updates the CDR for 5GS Bridget information available.

20ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the SMF.

The figure 7.2.3-2 describes the high-level charging procedure for 5GS bridge management and configuration charging based on SMF during the PDU session modification.

Figure 7.2.3-2: 5GS Bridge management and configuration charging during the PDU session modification

- PDU session establishment as described in the TS 23.502 [4] clause 4.3.2.2 UE Requested PDU Session Establishment.

- SMF establish the charging session for start of the PDU session.

- SMF receive the configuration of 5GS bridge from CNC.

- The PDU session may be modified because the configuration request from CNC.

4ch-a. The SMF sends Charging Data Request [Update] to CHF for PDU session modification, with the trigger QoS change.

4ch-b. The CHF update a CDR for the configuration request from CNC.

4ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the SMF.

- SMF reports the 5GS bridge information to the CNC.

5ch-a. The SMF sends Charging Data Request [Update] to CHF for the reporting of the configuration.

5ch-b. The CHF updates a CDR.

5ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the SMF.

## 7.3		Solution #3: 5GS Bridge management and configuration charging

### 7.3.1	General

The solution #3, addresses the Key Issue #4a, Key Issue #4b, Key Issue #4c and #4d, covering the REQ-CH_BC-01 and REQ-CH_BC-02.

Based on the clause 6.2.32 in the TS 23.501 [3], the TSN AF supports control plane translator functionality for the integration of the 5GS with a TSN network, this involves e.g.:

-	5GS Bridge management.

-	Port and bridge management information exchange with DS-TT or NW-TT.

-	Interactions with the CNC for 5GS Bridge configuration and reporting.

-	determining the TSC Assistance Container and TSN QoS information by mapping TSN Stream(s).

TSN AF can support the 5GS Bridge management and configuration charging via the interactions with the CNC.

### 7.3.2	Architecture description

Figure 7.3.2-1: TSN AF converged charging architecture

### 7.3.3	Flow description

The figure 7.3.3-1 describes the high-level charging procedure for 5GS bridge management and configuration charging based on TSN AF interaction with the PCF and CNC. In this case, the TSN AF, PCF and CHF belong to the same MNO.

Figure 7.3.3-1: 5GS bridge management and configuration charging via TSN AF

- TSN AF receives the 5GS bridge management and configuration from CNC. The capabilities of each port on UE/DS-TT side and UPF/NW-TT side are integrated as part of the configuration of the 5GS Bridge and are notified to TSN AF and delivered to CNC for TSN bridge registration and modification.

- TSN AF forwards the 5GS bridge management and configuration to PCF.

2ch-a. The TSN AF sends Charging Data Request [Event] to CHF for the request from the CNC about the 5GS bridge and configuration.

2ch-b. The CHF creates a CDR for the request from CNC.

2ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the TSN AF.

- PCF sends the 5GS bridge management and configuration response to the TSN AF.

- TSN AF forwards the 5GS bridge management and configuration response to the CNC.

Editor’s Note: The different solution for having the CHF and TNS AF belonging to different enterprises is ffs.

## 7.4		Solution #4: Time Sensitive Communications charging from TSN station to TSN station

### 7.4.1	General

The solution #4, addresses the Key Issue #3a and #3b, covering the REQ-CH_TSC-01, REQ-CH_TSC-02 and REQ-CH_TSC-03.

Based on the clause 5.28.1 in the TS 23.501[3], 5.28.1 5GS bridge management, there is only one PDU Session per DS-TT port for a given UPF. All PDU Sessions which connect to the same TSN network via a specific UPF are grouped into a single 5GS bridge.

For time sensitive communications, the following options are present in the figure 7.4.1-1:

-	TSN Bridge/End station to TSN Bridge/End station: based on the PDU session 1B, UE1-DS_TT interact with the UPF-B/NW-TT. based on the PDU session 2B, UE2-DS-TT interact with the UPF-B/NW-TT. The UPF-B/NW-TT is responsible for the traffic communication between TSN Bridget/End stations. The SMF applies local switching in order to enable UPF locally forward uplink stream from one PDU session as downlink stream in another PDU session.

Figure 7.4.1-1: TSC for TSN Station to TSN Station

### 7.4.2	Architecture description

Figure 7.4.2-1: SMF converged charging architecture

### 7.4.3	Flow description

The traffic data of Time sensitive communication is transferred on the 5GS Bridge. The granularity of the 5GS TSN bridge is per UPF for each network instance or DNN/S-NSSAI. The bridge ID of the 5GS TSN bridge is bound to the UPF ID of the UPF.

#### 7.4.3.1	TSN Bridge/End station to TSN Bridge/End station of the Time Sensitive Communication

The figure 7.4.3.1-1 describes the high-level charging procedure for TSN Bridge/End station to TSN Bridge/End station of the Time Sensitive Communication based on SMF during the PDU session.

Figure 7.4.3.1-1: TSC for TSN Bridge/End station to TSN Bridge/End station

  - UE 1 and UE 2 performs the Time Sensitive Communications on the same 5GS bridge.

4. 	UPF sends the usage reporting for the Time sensitive communications.

5ch-a.	The SMF sends Charging Data Request [Initial] to the UE CHF for the TSN stream delivery with the 5GS Bridge id and related 5GS bridge information.

5ch-b. The UE CHF creates a CDR.

5ch-c. The UE CHF acknowledges by sending Charging Data Response [Initial] to the SMF.

6ch-a.	The UE CHF sends Charging Data Request [Initial] to the TSN CHF for the TSN stream delivery with the 5GS Bridge id and related 5GS bridge information.

6ch-b. The TSN CHF creates a CDR for the start of the TSN stream.

6ch-c. The TSN CHF acknowledges by sending Charging Data Response [Initial] to the UE CHF.

For Time Sensitive Communications service, the message flows for the PDU session establishment, PDU session modification, PDU session release of 5G data connectivity charging would be the same as in clauses 5.2.2.13.2.1, 5.2.2.13.3.1 and 5.2.2.13.4.1 of TS 32.255 [X].

TSN CCS or billing domain aggregates the volume usage per 5GS bridge via 5GS bridge Id for TSC based on the accumulation of usage reporting via the 5GS bridge.

TSN CCS or billing domain aggregates the volume usage per TSN via UPF ID for TSC based on the accumulation of usage reporting per 5GS bridge of TSN.

Editor’s note: The trigger from UE CHF for TSN CHF is ffs.

## 7.5		Solution #5: Time Sensitive Communications charging

### 7.5.1	General

The solution #5, addresses the Key Issue #3a and #3b, covering the REQ-CH_TSC-01, REQ-CH_TSC-02 and REQ-CH_TSC-03.

Based on the clause 5.28.1 in the TS 23.501 [3], 5.28.15GS bridge management, there is only one PDU Session per DS-TT port for a given UPF. All PDU Sessions which connect to the same TSN network via a specific UPF are grouped into a single 5GS bridge.

For time sensitive communications, the following options are present in the figure 7.5.1-1:

-	TSN Bridge/End station to the TSN System: based on the PDU session 1A, the UE1-DS_TT and TSN system transfers the TSN stream via the UPF-A/NW-TT on the 5GS Bridge A.

Figure 7.5.1-1: TSC for TSN Station to TSN system

### 7.5.2	Architecture description

Figure 7.5.2-1: SMF converged charging architecture

### 7.5.3	Flow description

The traffic data of Time sensitive communication is transferred on the 5GS Bridge. The granularity of the 5GS TSN bridge is per UPF for each network instance or DNN/S-NSSAI. The bridge ID of the 5GS TSN bridge is bound to the UPF ID of the UPF.

#### 7.5.3.1	TSN Bridge/End station to the TSN System of the Time Sensitive Communication

The figure 7.5.3.1-1 describes the high-level charging procedure for TSN Bridge/End station to the TSN System of the Time Sensitive Communication based on SMF during the PDU session.

Figure 7.5.3.1-1: TSC for TSN Bridge/End station to TSN System

1.	SMF collects the usage of the TSN stream and reports the charging data information per UE per PDU session from UPF.

2ch-a.	The SMF sends Charging Data Request [Initial, Requested Quota] to the UE CHF for the TSN stream delivery with the 5GS Bridge id and related 5GS bridge information.

2ch-b. The UE CHF creates a CDR.

2ch-c. The UE CHF acknowledges by sending Charging Data Response [Initial, Requested Quota] to the SMF.

3ch-a. The UE CCS sends Charging Data Request [Initial] to TSN CHF for the request for the TSC on the 5GS bridge.

3ch-b. The TSN CHF creates a CDR for the request for TSC.

3ch-c. The TSN CHF acknowledges by sending Charging Data Response [Initial] to the UE CHF.

For Time Sensitive Communications service, the message flows for the PDU session establishment, PDU session modification, PDU session release of 5G data connectivity charging would be the same as in clauses 5.2.2.13.2.1, 5.2.2.13.3.1 and 5.2.2.13.4.1 of TS 32.255 [X].

The UE CCS charges the volume usage per UE based on the charging information reporting for the UE.

TSN CCS or billing domain aggregates the volume usage per 5GS bridge via 5GS bridge Id for TSC based on the accumulation of usage reporting per UE via the 5GS bridge.

TSN CCS or billing domain aggregates the volume usage per TSN via UPF ID for TSC based on the accumulation of usage reporting per UE via the 5GS bridge of TSN.

Editor’s note: The quota management for TSN charging (e.g. B2B) is ffs.

## 7.6		Solution #6: Enablers for TSC and TS Charging

#### 7.6.1	General

The solution #6, addresses the Key Issue #2a and #2b, covering the REQ-CH_ETS-01.

Based on the clause 5.2.27 TSCTSF Services in TS 23.502 [4], TSCTSF Services and Service Operations are present in the following table 7.6-1.

The NF consumer AF invokes the TSCTSF service directly or via NEF based on the whether the AF is considered to be trusted by the operator or not.

Table 7.6-1: NF Services provided by the TSCTSF

| Service Name | Operation Semantics | Example Consumer(s) |
| --- | --- | --- |
| Ntsctsf_TimeSynchronization | Request/Response Subscribe/Notify | AF, NEF |
| Ntsctsf_ASTI | Request/Response | AF, NEF |
| Ntsctsf_QoSandTSCAssistance | Request/Response Subscribe/Notify | AF, NEF |

For the Time Synchronization service charging, if the NEF charging described in the clause 7.1 solution #1 is supported, TSCTSF(CTF) charging is not required. If the AF is trusted party, interacting with TSCTSF without NEF, the TSCTSF(CTF) charging may be supported.

#### 7.6.2	Architecture description

Figure 7.6.2-1: TSCTSF converged charging architecture - CEF

#### 7.6.3	Flow description

#### 7.6.3.1	TSCTSF directly interact with the AF

If the AF is considered to be trusted by the operator, the TSCTSF sends the Ntsctsf_TimeSynchronization message directly to the AF. The figure 7.6.3.1-1 describes the high-level charging procedure for enablers for TSC and TS Charging based on TSCTSF.

Figure 7.6.3.1-1: 5GS Bridge management charging via SMF

1.	TSCTSF receives a Time Synchronization Service Request/subscribe message from an AF, including a list of UE identities (GPSIs) or Groups of UEs identified by an External Group Identifier that further define the subset of the target UEs and AF-Service-Identifier, or DNN and optionally S-NSSAI which used to indicate the TS-SC.

2.	TSCTSF performs the actions needed to fulfil the Time Synchronization Service Request/subscribe and If authorized, the TSCTSF sends the Time Synchronization Service Response/notify message.

2ch-a. The TSCTSF sends Charging Data Request [Event] to CHF for the received Time Synchronization Service Request/subscribe.

2ch-b. The CHF creates a CDR for this Time Synchronization Service Request/subscribe.

2ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the TSCTSF.

## 7.7		Solution #7: Exposure of Time Synchronization Service with trusted AF

### 7.7.1	General

The solution #7, addresses the Key Issue #1a and #1b, covering the REQ-CH_ETS-02.

The TS 23.501 [3] clause 4.4.8.3 states that if the AF is considered to be trusted by the operator, the AF could interact directly with TSCTSF. The TS 23.502 [4] clause 5.2.27, describes the TSCTSF services that either the AF or NEF can consume:

-	Ntsctsf_TimeSynchronization_: create update and delete time synchronization configuration and to activate and deactivate the time synchronization service, further described in TS 23.501 [3] clause 5.27.1.8.

-	Ntsctsf_ASTI_: control the 5G access stratum time distribution configuration, further described in TS 23.501 [3] clause 5.27.1.8.

-	Ntsctsf_QoSandTSCAssistance_: specific QoS and provide assistance for handling traffic characterized by TSC QoS parameters, further described in TS 23.503 [20] clause 6.1.3.22.

### 7.7.2	Architecture description

Figure 7.7.2-1: AF converged charging architecture

### 7.7.3	Flow description

The following figure 7.7.3-1 describes the AF session with required QoS update procedure, based on figure 4.15.6.6-1 TS 23.502 [4] description:

Figure 7.7.3-1: AF Request directly to TSCTSF using PEC

1 to 4b.	Start and determination that TSCTSF needs to be invoked, implying that steps 3a, 3b, 4a, 4b, 5, 6a, 7a, 7b, and 8 from figure 4.15.6.6-1 TS 23.502 [4] are applicable.

4ch-a.	The AF sends Charging Data Request [Event] to CHF for the received Ntsctsf_QoSandTSCAssistance_Create response.

4ch-b.	The CHF creates a CDR.

4ch-c.	The CHF acknowledges by sending Charging Data Response [Event] to the AF.

6a to 6b.	From figure 4.15.6.6a-1 TS 23.502 [4]

6ch-a.	The AF sends Charging Data Request [Event] to CHF for the received Ntsctsf_QoSandTSCAssistance_Notify.

6ch-b.	The CHF creates a CDR.

6ch-c.	The CHF acknowledges by sending Charging Data Response [Event] to the AF.

Editor’s note: the subscriber id is ffs.

## 7.y		Solution Evaluation

## 7.z	Solution Conclusions

# 8	Conclusions and recommendations

Annex <A>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | SA5#145e | S5-225756 |  |  |  | Initial skeleton | 0.0.0 |
| 2022-08 | SA5#145e | S5-225757 |  |  |  | Update of the Scope | 0.1.0 |
| 2022-08 | SA5#145e | S5-225492 |  |  |  | Update of the Reference | 0.1.0 |
| 2022-11 | SA5#146 | S5-226504 |  |  |  | Update of the Skeleton | 0.2.0 |
| 2022-11 | SA5#146 | S5-226505 |  |  |  | Update of the Terms, Symbols and Abbreviations | 0.2.0 |
| 2022-11 | SA5#146 | S5-226506 |  |  |  | Add the Background of TSN architecture | 0.2.0 |
| 2022-11 | SA5#146 | S5-226710 |  |  |  | Add the Business Roles | 0.2.0 |
| 2022-12 |  |  |  |  |  | Identical version, no changes (MCC) | 0.3.0 |
| 2022-12 |  |  |  |  |  | Correction of version on cover page and history table | 0.3.1 |
| 2023-01 | SA5#146bis-e | S5-231076 |  |  |  | Add Use Case for Exposure of Time Synchronization | 0.4.0 |
| 2023-03 | SA5#147 | S5-232834 |  |  |  | Add Use Case for Enablers for Time Sensitive Communications | 0.5.0 |
| 2023-03 | SA5#147 | S5-232835 |  |  |  | Add Use Case for Time Sensitive Communication | 0.5.0 |
| 2023-03 | SA5#147 | S5-232836 |  |  |  | Add Use Case for TSN 5GS bridge Charging | 0.5.0 |
| 2023-03 | SA5#147 | S5-232837 |  |  |  | Add solution for the Network Exposure Charging | 0.5.0 |
| 2023-04 | SA5#148e | S5-233430 |  |  |  | Clarification on the solution the Network Exposure Charging | 0.6.0 |
| 2023-04 | SA5#148e | S5-233703 |  |  |  | Addition of the solution for the 5GS bridge management | 0.6.0 |
| 2023-04 | SA5#148e | S5-233704 |  |  |  | Addition of the solution for the 5GS bridge management and configuration | 0.6.0 |
| 2023-04 | SA5#148e | S5-233706 |  |  |  | Addition of the solution for the TSC from TSN station to TSN station | 0.6.0 |
| 2023-04 | SA5#148e | S5-233707 |  |  |  | Addition of the solution for the TSC from TSN station to TSN system | 0.6.0 |
| 2023-05 | SA5#149 | S5-234627 S5-234628 S5-234629 S5-234630 |  |  |  | Adding missing abbreviations and references Correcting exposure solution using NEF Addition of the solution for the Enablers for TSC and TS Charging Adding solution using charging from trusted AF and PEC | 0.7.0 |
| 2023-06 | SA#100 | SP-230639 |  |  |  | Presented for information | 1.0.0 |