| 3GPP TS 29.548 V0.4.0 (2023-10) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Service Enabler Architecture Layer for Verticals (SEAL); SEAL Data Delivery (SEALDD) Server Services; Stage 3 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	10

2	References	11

3	Definitions, symbols and abbreviations	12

3.1	Definitions	12

3.2	Symbols	12

3.3	Abbreviations	12

4	Overview	13

5	Services offered by the SEALDD Server	14

5.1	Introduction	14

5.2	SDD_Transmission Service	15

5.2.1	Service Description	15

5.2.2	Service Operations	15

5.2.2.1	Introduction	15

5.2.2.2	SDD_Transmission_Request	15

5.2.2.2.1	General	15

5.2.2.2.2	SEALDD Transmission Request	15

5.2.2.3	SDD_Transmission_ConnStatusSubscribe	16

5.2.2.3.1	General	16

5.2.2.3.2	SEALDD Connection Status Subscription Creation	16

5.2.2.3.3	SEALDD Connection Status Subscription Update	17

5.2.2.3.4	SEALDD Connection Status Subscription Deletion	17

5.2.2.4	SDD_Transmission_ConnStatusNotify	17

5.2.2.4.1	General	17

5.2.2.4.2	Connection Status Event Notification	17

5.3	SDD_DataStorage Service	18

5.3.1	Service Description	18

5.3.2	Service Operations	18

5.3.2.1	Introduction	18

5.4	SDD_DDContext Service	19

5.4.1	Service Description	19

5.4.2	Service Operations	19

5.4.2.1	Introduction	19

5.4.2.2	SDD_DDContext_Push	19

5.4.2.2.1	General	19

5.4.2.2.2	DD Context Push	19

5.4.2.3	SDD_DDContext_Pull	19

5.4.2.3.1	General	19

5.4.2.3.2	DD Context Pull	20

5.5	SDD_TransmissionQualityMeasurement	21

5.5.1	Service Description	21

5.5.2	Service Operations	21

5.5.2.1	Introduction	21

5.5.2.2	SDD_TransmissionQualityMeasurement_Subscribe	21

5.5.2.2.1	General	21

5.5.2.2.2	Transmission Quality Measurement Subscription Creation	21

5.5.2.2.3	Transmission Quality Measurement Subscription Update	22

5.5.2.2.4	Transmission Quality Measurement Subscription Deletion	23

5.5.2.3	SDD_TransmissionQualityMeasurement_Notify	23

5.5.2.3.1	General	23

5.5.2.3.2	Transmission Quality Measurement Notification	24

5.5.2.4	SDD_TransmissionQualityMeasurement_Request	25

5.5.2.4.1	General	25

5.5.2.4.2	Transmission Quality Measurement Retrieval	25

5.6	SDD_PolicyConfiguration	26

5.6.1	Service Description	26

5.6.2	Service Operations	26

5.6.2.1	Introduction	26

5.6.2.2	SDD_PolicyConfiguration_Create	26

5.6.2.2.1	General	26

5.6.2.2.2	Policy Configuration Creation	26

5.6.2.3	SDD_PolicyConfiguration_Update	27

5.6.2.3.1	General	27

5.6.2.3.2	Policy Configuration Update	27

5.6.2.4	SDD_PolicyConfiguration_Delete	28

5.6.2.4.1	General	28

5.6.2.4.2	Policy Configuration Deletion	28

6	API Definitions	29

6.1	SDD_Transmission Service API	29

6.1.1	Introduction	29

6.1.2	Usage of HTTP	29

6.1.3	Resources	29

6.1.3.1	Overview	29

6.1.3.2	Resource: Connection Status Subscriptions	30

6.1.3.2.1	Description	30

6.1.3.2.2	Resource Definition	30

6.1.3.2.3	Resource Standard Methods	30

6.1.3.2.3.1	POST	30

6.1.3.2.4	Resource Custom Operations	31

6.1.3.3	Resource: <resource 2>	31

6.1.4	Custom Operations without associated resources	31

6.1.4.1	Overview	31

6.1.4.2	Operation: RequestTrans	32

6.1.4.2.1	Description	32

6.1.4.2.2	Operation Definition	32

6.1.5	Notifications	33

6.1.5.1	General	33

6.1.5.2	Connection Status Notification	33

6.1.5.2.1	Description	33

6.1.5.2.2	Target URI	33

6.1.5.2.3	Standard Methods	33

6.1.6	Data Model	34

6.1.6.1	General	34

6.1.6.2	Structured data types	35

6.1.6.2.1	Introduction	35

6.1.6.2.2	Type: TransReq	36

6.1.6.2.3	Type: TransResp	36

6.1.6.2.4	Type: ConnInfo	36

6.1.6.2.5	Type: QosInfo	37

6.1.6.2.6	Type: ValServBdw	37

6.1.6.2.7	Type: ValUsersBdw	37

6.1.6.2.8	Type: ConnStatusSubsc	37

6.1.6.2.9	Type: ConnStatusNotif	38

6.1.6.2.10	Type: ConnStatusReport	38

6.1.6.2.11	Type: ConnEstabData	38

6.1.6.3	Simple data types and enumerations	38

6.1.6.3.1	Introduction	38

6.1.6.3.2	Simple data types	38

6.1.6.3.3	Enumeration: ConnectStatEvent	38

6.1.6.3.4	Enumeration: TransType	39

6.1.6.4	Data types describing alternative data types or combinations of data types	39

6.1.6.5	Binary data	39

6.1.6.5.1	Binary Data Types	39

6.1.7	Error Handling	39

6.1.7.1	General	39

6.1.7.2	Protocol Errors	39

6.1.7.3	Application Errors	39

6.1.8	Feature negotiation	40

6.1.9	Security	40

6.2	SDD_DataStorage Service API	41

6.3	SDD_DDContext Service API	42

6.3.1	Introduction	42

6.3.2	Usage of HTTP	42

6.3.3	Resources	42

6.3.3.1	Overview	42

6.3.3.2	Resource: DD Context Instances	43

6.3.3.2.1	Description	43

6.3.3.2.2	Resource Definition	43

6.3.3.2.3	Resource Standard Methods	43

6.3.3.2.3.1	POST	43

6.3.3.2.3.2	GET	44

6.3.3.2.4	 Resource Custom Operations	45

6.3.4	Custom Operations without associated resources	45

6.3.5	Notifications	45

6.3.6	Data Model	46

6.3.6.1	General	46

6.3.6.2	Structured data types	46

6.3.6.2.1	Introduction	46

6.3.6.2.2	Type: DdContext	46

6.3.6.2.3	Type: TranspLayerContext	47

6.3.6.2.4	Type: DdContextPushReq	47

6.3.6.2.5	Type: DdContextResp	47

6.3.6.3	Simple data types and enumerations	47

6.3.6.3.1	Introduction	47

6.3.6.3.2	Simple data types	47

6.3.6.4	Data types describing alternative data types or combinations of data types	47

6.3.6.5	Binary data	48

6.3.6.5.1	Binary Data Types	48

6.3.7	Error Handling	48

6.3.7.1	General	48

6.3.7.2	Protocol Errors	48

6.3.7.3	Application Errors	48

6.3.8	Feature negotiation	48

6.1.9	Security	48

6.4	SDD_TransmissionQualityMeasurement API	49

6.4.1	Introduction	49

6.4.2	Usage of HTTP	49

6.4.3	Resources	49

6.4.3.1	Overview	49

6.4.3.2	Resource: Transmission Quality Measurement Subscriptions	50

6.4.3.2.1	Description	50

6.4.3.2.2	Resource Definition	50

6.4.3.2.3	Resource Standard Methods	50

6.4.3.2.3.1	POST	50

6.4.3.2.4	Resource Custom Operations	51

6.4.3.3	Resource: Individual Transmission Quality Measurement Subscription	51

6.4.3.3.1	Description	51

6.4.3.3.2	Resource Definition	51

6.4.3.3.3	Resource Standard Methods	52

6.4.3.3.3.1	GET	52

6.4.3.3.3.2	PUT	53

6.4.3.3.3.3	PATCH	54

6.4.3.3.3.4	DELETE	55

6.4.3.3.4	Resource Custom Operations	55

6.4.3.4	Resource: Historical Transmission Quality Measurement Reports	56

6.4.3.4.1	Description	56

6.4.3.4.2	Resource Definition	56

6.4.3.4.3	Resource Standard Methods	56

6.4.3.4.3.1	GET	56

6.4.3.4.4	Resource Custom Operations	57

6.4.4	Custom Operations without associated resources	57

6.4.5	Notifications	57

6.4.5.1	General	57

6.4.5.2	Transmission Quality Measurement Notification	58

6.4.5.2.1	Description	58

6.4.5.2.2	Target URI	58

6.4.5.2.3	Standard Methods	58

6.4.5.2.3.1	POST	58

6.4.6	Data Model	59

6.4.6.1	General	59

6.4.6.2	Structured data types	61

6.4.6.2.1	Introduction	61

6.4.6.2.2	Type: TransQualMeasSubsc	61

6.4.6.2.3	Type: TransQualMeasReq	62

6.4.6.2.4	Type: TransQualMeasSubscPatch	62

6.4.6.2.5	Type: TransQualMeasNotif	62

6.4.6.2.6	Type: TransQualMeasReport	63

6.4.6.2.7	Type: TransQualMeasCriteria	63

6.4.6.2.8	Type: TransQualMeasData	64

6.4.6.2.9	Type: ValTargetUeId	67

6.4.6.2.10	Type: HistTransQualMeasReports	67

6.4.6.3	Simple data types and enumerations	67

6.4.6.3.1	Introduction	67

6.4.6.3.2	Simple data types	67

6.4.6.3.3	Enumeration: MeasurementId	68

6.4.6.3.4	Enumeration: RepGranularity	68

6.4.6.4	Data types describing alternative data types or combinations of data types	68

6.4.6.5	Binary data	68

6.4.6.5.1	Binary Data Types	68

6.4.7	Error Handling	68

6.4.7.1	General	68

6.4.7.2	Protocol Errors	69

6.4.7.3	Application Errors	69

6.4.8	Feature negotiation	69

6.4.9	Security	69

6.5	SDD_PolicyConfiguration API	70

6.5.1	Introduction	70

6.5.2	Usage of HTTP	70

6.5.3	Resources	70

6.5.3.1	Overview	70

6.5.3.2	Resource: Policy Configurations	71

6.5.3.2.1	Description	71

6.5.3.2.2	Resource Definition	71

6.5.3.2.3	Resource Standard Methods	71

6.5.3.2.3.1	POST	71

6.5.3.2.4	Resource Custom Operations	72

6.5.3.3	Resource: Individual Policy Configuration	72

6.5.3.3.1	Description	72

6.5.3.3.2	Resource Definition	72

6.5.3.3.3	Resource Standard Methods	72

6.5.3.3.3.1	GET	72

6.5.3.3.3.2	PUT	73

6.5.3.3.3.3	PATCH	74

6.5.3.3.3.4	DELETE	75

6.5.3.3.4	Resource Custom Operations	76

6.5.4	Custom Operations without associated resources	76

6.5.5	Notifications	76

6.5.6	Data Model	76

6.5.6.1	General	76

6.5.6.2	Structured data types	77

6.5.6.2.1	Introduction	77

6.5.6.2.2	Type: PolicyConfig	77

6.5.6.2.3	Type: PolicyConfigPatch	78

6.5.6.2.4	Type: SealddPolicy	78

6.5.6.3	Simple data types and enumerations	78

6.5.6.3.1	Introduction	78

6.5.6.3.2	Simple data types	78

6.5.6.3.3	Enumeration: QualGuarPolicy	78

6.5.6.3.4	Enumeration: BdwCtrlPolicy	79

6.5.6.4	Data types describing alternative data types or combinations of data types	79

6.5.6.5	Binary data	79

6.5.6.5.1	Binary Data Types	79

6.5.7	Error Handling	79

6.5.7.1	General	79

6.5.7.2	Protocol Errors	80

6.5.7.3	Application Errors	80

6.5.8	Feature negotiation	80

6.5.9	Security	80

7	Using Common API Framework	81

Annex A (normative): OpenAPI specification	82

A.1	General	82

A.2	SDD_Transmission API	83

A.3	SDD_DDContext API	88

A.4	SDD_DataStorage API	91

A.5	SDD_TransmissionQualityMeasurement API	92

A.6	SDD_PolicyConfiguration API	101

Annex B (informative): Withdrawn API versions	106

B.1	General	106

B.2	SDD_Transmission API	106

B.3	SDD_DDContext API	106

B.4	SDD_DataStorage API	106

B.5	SDD_TransmissionQualityMeasurement API	106

B.6	SDD_PolicyConfiguration API	106

Annex C (informative): Change history	107

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the SEAL Data Delivery (SEALDD) Server Services, for enabling the support of SEAL Data Delivery (SEALDD) services for vertical applications. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the SEALDD Server.

The stage 2 application layer architecture, functional requirements, procedures and information flows necessary for enabling SEAL Data Delivery (SEALDD) are specified in 3GPP TS 23.433 [7].

The common protocol and interface aspects for API definition are specified in clause 5.2 of 3GPP TS 29.122 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[3]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[4]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[5]	3GPP TR 21.900: "Technical Specification Group working methods".

[6]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[7]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[8]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[9]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[10]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[11]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[12]	IETF RFC 9113: "HTTP/2".

[13]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[14]	IETF RFC 7807: "Problem Details for HTTP APIs".

[15]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3".

[16]	3GPP TS 29.508: "5G System; Session Management Event Exposure Service; Stage 3".

[17]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3".

[18]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purpose of the present document, the terms and definitions specified in clause 3.1 of 3GPP TS 23.433 [7] and clause 3.1 of 3GPP TS 29.549 [15] also apply, including the ones referencing other specifications.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

E2E	End to End

SEAL	Service Enabler Architecture Layer for Verticals

SEALDD	SEAL Data Delivery

VAL	Vertical Application Layer

# 4	Overview

The SEAL Data Delivery (SEALDD) Server forms part of the SEAL Enabler Layer defined in 3GPP TS 23.434 [6] and aims to ensure the efficient use and deployment of storage and delivery capabilities for the application content/data of vertical applications. The SEALDD Server supports for this purpose, among other functionalities defined in 3GPP TS 23.433 [7], the following functionalities:

-	SEALDD application layer support functions to VAL Servers over the SEALDD-S reference point, i.e.:

-	SEALDD regular connection establishment management, including signalling transmission and regular data transmission;

-	SEALDD enabled E2E redundant transmission management;

-	SEALDD Server discovery and selection management;

-	SEALDD enabled data storage management;

-	SEALDD enabled data transmission quality measurement management;

-	SEALDD enabled bandwidth control for different VAL users; and

-	SEALDD enabled data transmission quality guarantee management.

-	SEALDD application layer support functions to other SEALDD Servers over the SEALDD-E reference point, i.e.:

-	SEALDD Server relocation management.

Figure 4-1 shows the reference model of the SEALDD Enabler Layer, with a focus on the SEALDD Server:

Figure 4-1: SEALDD Enabler Layer functional model

# 5	Services offered by the SEALDD Server

## 5.1	Introduction

The SEALDD Server provides the following services:

-	SDD_Transmission

-	SDD_DataStorage

-	SDD_DDContext

-	SDD_TransmissionQualityMeasurement

-	SDD_PolicyConfiguration

NOTE:	The stage 2 Sdd_RegularTransmission and Sdd_URLLCTransmission APIs defined in 3GPP TS 23.433 [7] are defined via the SDD_Transmission API.

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | API Name | Annex |
| --- | --- | --- | --- | --- | --- |
| SDD_Transmission | 6.1 | SEALDD Data Transmission | TS29548_SDD_Transmission.yaml | sdd-trans | A.2 |
| SDD_DataStorage | 6.2 | SEALDD Data Storage | TS29548_SDD_DataStorage.yaml | sdd-ds | A.3 |
| SDD_DDContext | 6.3 | SEALDD Server Relocation Service | TS29548_SDD_DDContext.yaml | sdd-ddc | A.4 |
| SDD_TransmissionQualityMeasurement | 6.4 | Data Transmission Quality Measurement Service | TS29548_SDD_TransmissionQualityMeasurement.yaml | sdd-tqm | A.5 |
| SDD_PolicyConfiguration | 6.5 | SEALDD Policy Configuration | TS29548_SDD_PolicyConfiguration.yaml | sdd-pc | A.6 |

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (i.e. SEALDD Server) takes the role of the SCEF and the service consumer (e.g., VAL Server or another SEALDD Server) takes the role of the SCS/AS.

## 5.2	SDD_Transmission Service

### 5.2.1	Service Description

The SDD_Transmission service exposed by the SEALDD Server enables a service consumer (e.g. VAL Server) to:

-	request SEALDD enabled Regular or URLLC application data transmission; and

-	subscribe to / receive notifications on SEALDD connection status event(s).

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operations defined for the SDD_Transmission service are shown in table 5.5.2.1-1.

Table 5.5.2.1-1: SDD_Transmission Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| SDD_Transmission_Request | This service operation enables a service consumer to request SEALDD enabled regular or URLLC application data transmission. | e.g. VAL Server |
| SDD_Transmission_ConnStatusSubscribe | This service operation enables a service consumer to request the creation of a subscription to SEALDD connection status event(s) reporting. | e.g. VAL Server |
| SDD_Transmission_ConnStatusNotify | This service operation enables a service consumer to receive SEALDD connection status event(s) notifications. | SEALDD Server |

#### 5.2.2.2	SDD_Transmission_Request

##### 5.2.2.2.1	General

This service operation is used by a service consumer (e.g. VAL Server) to request SEALDD enabled Regular or URLLC application data transmission to the SEALDD Server.

The following procedures are supported by the "SDD_Transmission_Request" service operation:

-	SEALDD Transmission Request.

##### 5.2.2.2.2	SEALDD Transmission Request

Figure 5.2.2.2.2-1 depicts a scenario where a service consumer (e.g. VAL Server) sends a request to the SEALDD Server to request SEALDD enabled Regular or URLLC application data transmission (see also clauses 9.2 and 9.3 of 3GPP°TS°23.433°[7]).

Figure 5.2.2.2.2-1: Procedure for SEALDD Transmission Request

1.	In order to request SEALDD enabled Regular or URLLC application data transmission, the service consumer (e.g. VAL Server) shall send an HTTP POST request to the SEALDD Server targeting the URI of the corresponding custom operation (i.e., "RequestTrans"), with the request body including the TransReq data structure. The "{transType}" URI variable path segment shall be set to either:

-	"regular", when Regular application data transmission is requested; or

-	"urllc", when URLLC application data transmission is requested.

2a.	Upon success, the SEALDD Server shall respond with an HTTP "200 OK" status code with the response body containing the TransResp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.7.

#### 5.2.2.3	SDD_Transmission_ConnStatusSubscribe

##### 5.2.2.3.1	General

This service operation is used by a service consumer (e.g., VAL Server) to request to create a subscription to SEALDD connection status event(s) reporting.

The following procedures are supported by the "SDD_Transmission_ConnStatusSubscribe" service operation:

-	SEALDD Connection Status Subscription Creation.

-	SEALDD Connection Status Subscription Update.

-	SEALDD Connection Status Subscription Deletion.

##### 5.2.2.3.2	SEALDD Connection Status Subscription Creation

Figure 5.2.2.3.2-1 depicts a scenario where a service consumer (e.g., VAL Server) sends a request to the SEALDD Server to request the creation of a subscription to SEALDD Connection Status event(s) reporting (see also clauses 9.2 and 9.3 of 3GPP°TS°23.433°[7]).

Figure 5.2.2.3.2-1: Procedure for SEALDD Connection Status Subscription Creation

1.	In order to subscribe to SEALDD connection status event(s) reporting, the service consumer (e.g. VAL Server) shall send an HTTP POST request message to the SEALDD Server targeting the URI of the "Connection Status Subscriptions" collection resource with the request body including the ConnStatusSubsc data structure defined in clause 6.1.6.2.8.

2a.	Upon success, the SEALDD Server shall respond with a "201 Created" status code with the response body including a representation of the created "Individual Connection Status Subscription" resource within the ConnStatusSubsc data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.7.

##### 5.2.2.3.3	SEALDD Connection Status Subscription Update

Editor's Note:	Whether this service operation is needed or not and its definition are FFS.

##### 5.2.2.3.4	SEALDD Connection Status Subscription Deletion

Editor's Note:	Whether this service operation is needed or not and its definition are FFS.

#### 5.2.2.4	SDD_Transmission_ConnStatusNotify

##### 5.2.2.4.1	General

This service operation is used by a SEALDD Server to notify a previously subscribed service consumer (e.g., VAL Server) on:

-	SEALDD connection status event(s).

The following procedures are supported by the "SDD_Transmission_ConnStatusNotify" service operation:

-	Connection Status Notification.

##### 5.2.2.4.2	Connection Status Event Notification

Figure 5.2.2.4.2-1 depicts a scenario where the SEALDD Server sends a request to notify a previously subscribed service consumer (e.g. VAL Server) SEALDD connection status event(s) (see also clauses 9.2 and 9.3 of 3GPP°TS°23.433°[7]).

Figure 5.2.2.4.2-1: Procedure for Connection Status Event Notification

1.	In order to notify a previously subscribed service consumer (e.g. VAL Server) on SEALDD Connection Status event(s), the SEALDD Server shall send an HTTP POST request message to the service consumer (e.g. VAL server) with the request URI set to "{notifUri}", where the "notifUri" variable set to the value received from the service consumer (e.g., VAL Server) during the creation of the corresponding SEALDD Connection Status Subscription using the procedures defined in clause 5.2.2.3, and the request body including the ConnStatusNotif data structure.

2a.	Upon success, the service consumer (e.g. VAL Server) shall respond to the SEALDD Server with a "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.7.

## 5.3	SDD_DataStorage Service

### 5.3.1	Service Description

The SDD_DataStorage service exposed by the SEALDD Server enables a service consumer (e.g. VAL Server) to:

-	create/update/delete/query a SEALDD Data Storage;

-	create/update/delete a SEALDD Data Storage delivery subscription; and

-	request for one-time SEALDD Data Storage delivery.

### 5.3.2	Service Operations

#### 5.3.2.1	Introduction

The service operations defined for the SDD_DataStorage service are shown in table 5.3.2.1-1.

Table 5.3.2.1-1: SDD_DataStorage Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| SDD_DataStorage_Create | This service operation enables a service consumer to request the creation of a SEALDD Data Storage. | e.g. VAL Server |
| SDD_DataStorage_Manage | This service operation enables a service consumer to request the update/modification/deletion of an existing SEALDD Data Storage. | e.g. VAL Server |
| SDD_DataStorage_Query | This service operation enables a service consumer to request the deletion of an existing SEALDD Data Storage. | e.g. VAL Server |
| SDD_DataStorage_DelSubscribe | This service operation enables a service consumer to request the creation/update/deletion of a SEALDD Data Storage delivery subscription. | e.g. VAL Server |
| SDD_DataStorage_DelNotify | This service operation enables a service consumer to receive SEALDD Data Storage delivery notifications. | e.g. VAL Server |
| SDD_DataStorage_DelRequest | This service operation enables a service consumer to request for one-time SEALDD Data Storage delivery. | e.g. VAL Server |

## 5.4	SDD_DDContext Service

### 5.4.1	Service Description

The SDD_DDContext service exposed by the SEALDD Server via the SEALDD-E reference point enables a service consumer (e.g., SEALDD Server) to:

-	push the DD context to the new SEALDD Server; and

-	pull the DD context from the old SEALDD Server.

### 5.4.2	Service Operations

#### 5.4.2.1	Introduction

The service operations defined for the SDD_DDContext API are shown in the table 5.4.2.1-1.

Table 5.4.2.1-1: Operations of the SDD_DDContext API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| SDD_DDContext_Push | This service operation is used by the old SEALDD Server to push the DD context to the new SEALDD Server. | e.g., SEALDD Server |
| SDD_DDContext_Pull | This service operation is used by the new SEALDD Server to pull the DD context from the old SEALDD Server. | e.g., SEALDD Server |

#### 5.4.2.2	SDD_DDContext_Push

##### 5.4.2.2.1	General

This service operation is used by a service consumer (e.g., old SEALDD Server) to push the DD Context to the SEALDD Server (e.g., new SEALDD Server).

The following procedures are supported by the "SDD_DDContext_Push" service operation:

-	DD Context Push.

##### 5.4.2.2.2	DD Context Push

Editor's note: The procedure description for the SDD_DDContext_Push is FFS.

1.	In order to push the DD context to the SEALDD Server (e.g., new SEALDD Server), the service consumer (e.g., old SEALDD Server) shall send an HTTP POST request targeting the URI of the "DD Context Instances" collection resource, with the request body including the DdContextPushReq data structure.

2a.	Upon success, the SEALDD Server shall respond with an HTTP "200 OK" status code with the response body containing the DdContextPushResp data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.4.7.

#### 5.4.2.3	SDD_DDContext_Pull

##### 5.4.2.3.1	General

This service operation is used by a service consumer (e.g., new SEALDD Server) to pull the DD context from the SEALDD Server (e.g., old SEALDD Server).

The following procedures are supported by the "SDD_DDContext_Push" service operation:

-	DD Context Pull.

##### 5.4.2.3.2	DD Context Pull

Editor's note: The procedure description for the SDD_DDContext_Pull is FFS.

1.	In order to pull the DD context from the SEALDD Server (e.g., old SEALDD Server), the service consumer (e.g., new SEALDD Server) shall send an HTTP GET request message targeting the URI of the "DD Context Instances" collection resource.

2a.	Upon success, the SEALDD Server shall respond with an HTTP "200 OK" status code with the response body containing the DdContextPushReq data structure.

NOTE:	The content of the DdContextPushReq in step 2a is determined by the SEALDD Server based on the service consumer's identity.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP GET response body, as specified in clause 6.4.7.

## 5.5	SDD_TransmissionQualityMeasurement

### 5.5.1	Service Description

The SDD_TransmissionQualityMeasurement service exposed by the SEALDD Server enables a service consumer (e.g. VAL Server) to:

-	create/update/delete a Transmission Quality Measurement Subscription;

-	receive Transmission Quality Measurement notifications; and

-	request historical Transmission Quality Measurement reports.

### 5.5.2	Service Operations

#### 5.5.2.1	Introduction

The service operations defined for the SDD_TransmissionQualityMeasurement service are shown in table 5.5.2.1-1.

Table 5.5.2.1-1: SDD_TransmissionQualityMeasurement Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| SDD_TransmissionQualityMeasurement_Subscribe | This service operation enables a service consumer to create/update/delete a Transmission Quality Measurement Subscription. | e.g. VAL Server |
| SDD_TransmissionQualityMeasurement_Notify | This service operation enables a service consumer to receive Transmission Quality Measurement notifications. | SEALDD Server |
| SDD_TransmissionQualityMeasurement_Request | This service operation enables a service consumer to request historical Transmission Quality Measurement reports. | e.g. VAL Server, SEALDD Server, NSCE Server |

#### 5.5.2.2	SDD_TransmissionQualityMeasurement_Subscribe

##### 5.5.2.2.1	General

This service operation is used by a service consumer (e.g. VAL Server, SEALDD Server, NSCE Server) to request the creation/update/deletion of a Transmission Quality Measurement Subscription at the SEALDD Server.

The following procedures are supported by the "SDD_TransmissionQualityMeasurement_Subscribe" service operation:

-	Transmission Quality Measurement Subscription Creation.

-	Transmission Quality Measurement Subscription Update.

-	Transmission Quality Measurement Subscription Deletion.

##### 5.5.2.2.2	Transmission Quality Measurement Subscription Creation

Figure 5.5.2.2.2-1 depicts a scenario where a a service consumer (e.g. VAL Server) sends a request to the SEALDD Server to request the creation of a Transmission Quality Measurement Subscription (see also clause 9.7 of 3GPP°TS°23.433°[7]).

Figure 5.5.2.2.2-1: Procedure for Transmission Quality Measurement Subscription Creation

1.	In order to subscribe to transmission quality measurement reporting, the service consumer (e.g. VAL Server) shall send an HTTP POST request to the SEALDD Server targeting the URI of the "Transmission Quality Measurement Subscriptions" collection resource, with the request body including the TransQualMeasSubsc data structure.

2a.	Upon success, the SEALDD Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual Transmission Quality Measurement Subscription" resource within the TransQualMeasSubsc data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.4.7.

##### 5.5.2.2.3	Transmission Quality Measurement Subscription Update

Figure 5.5.2.2.3-1 depicts a scenario where a service consumer (e.g. VAL Server) sends a request to the SEALDD Server to request the update of an existing Transmission Quality Measurement Subscription (see also clause 9.7 of 3GPP°TS°23.433°[7]).

Figure 5.5.2.2.3-1: Procedure for Transmission Quality Measurement Subscription Update

1.	In order to update an existing transmission quality measurement subscription, the service consumer (e.g. VAL Server) shall send an HTTP PUT/PATCH request to the SEALDD Server, targeting the URI of the corresponding "Individual Transmission Quality Measurement Subscription" resource, with the request body including either:

-	the updated representation of the resource within the TransQualMeasSubsc data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the TransQualMeasSubscPatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative Service Consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the SEALDD Server shall update the targeted "Individual Transmission Quality Measurement Subscription" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Transmission Quality Measurement Subscription" resource within the TransQualMeasSubsc data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.4.7.

Editor's Note:	The definition of this procedure is FFS and pending stage 2 feedback.

##### 5.5.2.2.4	Transmission Quality Measurement Subscription Deletion

Figure 5.5.2.2.4-1 depicts a scenario where a service consumer (e.g. VAL Server) sends a request to the SEALDD Server to delete an existing Transmission Quality Measurement Subscription (see also clause 9.7 of 3GPP°TS°23.433°[7]).

Figure 5.5.2.2.4-1: Procedure for Transmission Quality Measurement Subscription Deletion

1.	In order to request the deletion of an existing transmission quality measurement subscription, the service consumer (e.g. VAL Server) shall send an HTTP DELETE request to the SEALDD Server targeting the corresponding "Individual Transmission Quality Measurement Subscription" resource.

NOTE:	An alternative Service Consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the SEALDD Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.4.7.

Editor's Note:	The definition of this procedure is FFS and pending stage 2 feedback.

#### 5.5.2.3	SDD_TransmissionQualityMeasurement_Notify

##### 5.5.2.3.1	General

This service operation is used by a SEALDD Server to notify a previously subscribed service consumer (e.g. VAL Server) on:

-	transmission Quality Measurement report(s).

The following procedures are supported by the "SDD_TransmissionQualityMeasurement_Notify" service operation:

-	Transmission Quality Measurement Notification.

##### 5.5.2.3.2	Transmission Quality Measurement Notification

Figure 5.5.2.3.2-1 depicts a scenario where the SEALDD Server sends a request to notify a previously subscribed service consumer (e.g. VAL Server) on Transmission Quality Measurement report(s) (see also clause 9.7 of 3GPP°TS°23.433°[7]).

Figure 5.5.2.3.2-1: Procedure for Transmission Quality Measurement Notification

1.	In order to notify a previously subscribed service consumer (e.g. VAL Server) on Transmission Quality Measurement report(s), the SEALDD Server shall send an HTTP POST request to the service consumer (e.g. VAL Server) with the request URI set to "{notifUri}", where the "notifUri" is set to the value received from the service consumer (e.g. VAL Server) during the creation/update of the corresponding Transmission Quality Measurement Subscription using the procedures defined in clause 5.5.2.2, and the request body including the TransQualMeasNotif data structure.

2a.	Upon success, the service consumer (e.g. VAL Server) shall respond to the SEALDD Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.4.7.

#### 5.5.2.4	SDD_TransmissionQualityMeasurement_Request

##### 5.5.2.4.1	General

This service operation is used by a service consumer (e.g. VAL Server, SEALDD Server, NSCE Server) to retrieve Historical Transmission Quality Measurement Report(s) from the SEALDD Server.

The following procedures are supported by the "SDD_TransmissionQualityMeasurement_Request" service operation:

-	Transmission Quality Measurement Retrieval.

##### 5.5.2.4.2	Transmission Quality Measurement Retrieval

Figure 5.5.2.4.2-1 depicts a scenario where a a service consumer (e.g. VAL Server, SEALDD Server, NSCE Server) sends a request to the SEALDD Server to retrieve Historical Transmission Quality Measurement Report(s) (see also clause 9.7 of 3GPP°TS°23.433°[7]).

Figure 5.5.2.4.2-1: Procedure for Transmission Quality Measurement Retrieval

1.	In order to retrieve Historical Transmission Quality Measurement Report(s), the service consumer (e.g. VAL Server, SEALDD Server, NSCE Server) shall send an HTTP GET request to the SEALDD Server targeting the URI of the "Historical Transmission Quality Measurement Reports" collection resource, with the request URI containing query parameters to filter the responses from the SEALDD Server.

2a.	Upon success, the SEALDD Server shall respond with an HTTP "200 OK" status code with the response body containing the requested Historical Transmission Quality Measurement Report(s).

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP GET response body, as specified in clause 6.4.7.

## 5.6	SDD_PolicyConfiguration

### 5.6.1	Service Description

The SDD_PolicyConfiguration service exposed by the SEALDD Server enables a service consumer (e.g. VAL Server) to:

-	create/update/delete a SEALDD Policy Configuration.

### 5.6.2	Service Operations

#### 5.6.2.1	Introduction

The service operations defined for the SDD_PolicyConfiguration service are shown in table 5.6.2.1-1.

Table 5.6.2.1-1: SDD_PolicyConfiguration Service Operations

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| SDD_PolicyConfiguration_Create | This service operation enables a service consumer to request the creation of a SEALDD Policy Configuration. | e.g., VAL Server |
| SDD_PolicyConfiguration_Update | This service operation enables a service consumer to request the update of an existing SEALDD Policy Configuration. | e.g., VAL Server |
| SDD_PolicyConfiguration_Delete | This service operation enables a service consumer to request the deletion of an existing SEALDD Policy Configuration. | e.g., VAL Server |

#### 5.6.2.2	SDD_PolicyConfiguration_Create

##### 5.6.2.2.1	General

This service operation is used by a service consumer (e.g., VAL Server) to request the creation of a SEALDD Policy Configuration at the SEALDD Server.

The following procedures are supported by the "SDD_PolicyConfiguration_Create" service operation:

-	Policy Configuration Creation.

##### 5.6.2.2.2	Policy Configuration Creation

Figure 5.6.2.2.2-1 depicts a scenario where a a service consumer (e.g., VAL Server) sends a request to the SEALDD Server to request the creation of a SEALDD Policy Configuration (see also clause 9.10 of 3GPP°TS°23.433°[7]).

Figure 5.6.2.2.2-1: Procedure for Policy Configuration Creation

1.	In order to create a new SEALDD Policy Configuration, the service consumer (e.g., VAL Server) shall send an HTTP POST request to the SEALDD Server targeting the URI of the "Policy Configurations" collection resource, with the request body including the PolicyConfig data structure.

2a.	Upon success, the SEALDD Server shall respond with an HTTP "201 Created" status code with the response body containing a representation of the created "Individual Policy Configuration" resource within the PolicyConfig data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.5.7.

#### 5.6.2.3	SDD_PolicyConfiguration_Update

##### 5.6.2.3.1	General

This service operation is used by a service consumer (e.g., VAL Server) to request the update of an existing SEALDD Policy Configuration at the SEALDD Server.

The following procedures are supported by the "SDD_PolicyConfiguration_Update" service operation:

-	Policy Configuration Update.

##### 5.6.2.3.2	Policy Configuration Update

Figure 5.6.2.3.2-1 depicts a scenario where a service consumer (e.g., VAL Server) sends a request to the SEALDD Server to request the update of an existing SEALDD Policy Configuration (see also clause 9.10 of 3GPP°TS°23.433°[7]).

Figure 5.6.2.3.2-1: Procedure for Policy Configuration Update

1.	In order to update an existing SEALDD Policy Configuration, the service consumer (e.g., VAL Server) shall send an HTTP PUT/PATCH request to the SEALDD Server, targeting the URI of the corresponding "Individual Policy Configuration" resource, with the request body including either:

-	the updated representation of the resource within the PolicyConfig data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource within the PolicyConfigPatch data structure, in case the HTTP PATCH method is used.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the SEALDD Server shall update the targeted "Individual Policy Configuration" resource accordingly and respond with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual Policy Configuration" resource within the PolicyConfig data structure; or

-	an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP PUT/PATCH response body, as specified in clause 6.5.7.

#### 5.6.2.4	SDD_PolicyConfiguration_Delete

##### 5.6.2.4.1	General

This service operation is used by a service consumer (e.g., VAL Server) to request the deletion of an existing SEALDD Policy Configuration at the SEALDD Server.

The following procedures are supported by the "SDD_PolicyConfiguration_Delete" service operation:

-	Policy Configuration Deletion.

##### 5.6.2.4.2	Policy Configuration Deletion

Figure 5.6.2.4.2-1 depicts a scenario where a service consumer (e.g., VAL Server) sends a request to the SEALDD Server to request the deletion of an existing SEALDD Policy Configuration (see also clause 9.10 of 3GPP°TS°23.433°[7]).

Figure 5.6.2.4.2-1: Procedure for Policy Configuration Deletion

1.	In order to request the deletion of an existing SEALDD Policy Configuration, the service consumer (e.g., VAL Server) shall send an HTTP DELETE request to the SEALDD Server targeting the corresponding "Individual Policy Configuration" resource.

NOTE:	An alternative service consumer (i.e. other than the one that requested the creation of the targeted resource) can initiate this request.

2a.	Upon success, the SEALDD Server shall respond with an HTTP "204 No Content" status code.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP DELETE response body, as specified in clause 6.5.7.

# 6	API Definitions

## 6.1	SDD_Transmission Service API

### 6.1.1	Introduction

The SDD_Transmission shall use the SDD_Transmission API.

The API URI of the SDD_Transmission API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [15], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

-	The <apiName> shall be "sdd-trans".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (i.e. SEALDD Server) takes the role of the SCEF and the service consumer (e.g., VAL Server) takes the role of the SCS/AS.

### 6.1.2	Usage of HTTP

The provisions of clause 6.3 of 3GPP TS 29.549 [15] shall apply for the SDD_Transmission API.

### 6.1.3	Resources

#### 6.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.1.3.1-1 depicts the resource URIs structure for the SDD_Transmission API.

Figure 6.1.3.1-1: Resource URI structure of the SDD_Transmission API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Connection Status Subscriptions | /subscriptions | POST | Request the creation of a new Connection Status Subscription. |

Editor's Note: Whether the individual resource should be defined with the update/modify and delete operations on it is FFS.

#### 6.1.3.2	Resource: Connection Status Subscriptions

##### 6.1.3.2.1	Description

This resource represents the collection of Connection Status Subscriptions managed by the SEALDD Server.

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-trans/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	POST

This method enables a service consumer (e.g., VAL Server) to request the creation of a new Connection Status Subscription at the SEALDD Server.

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ConnectStatSubsc | M | 1 | Represents the DD connection status subscription request. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ConnectStatSubsc | M | 1 | 201 Created | Successful case. The requested Connection Status Subscription resource is successfully created and a representation of the created "Individual Connection Status Subscription" resource is returned in the response body. |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Editor's Note: The error cases of this API are FFS.

Table 6.1.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/sdd-trans/<apiVersion>/subscriptions{subscriptionId} |

##### 6.1.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.1.3.3	Resource: <resource 2>

And so on if there are more than two resources supported by the service. Same structure as in clause 6.1.3.2.

### 6.1.4	Custom Operations without associated resources

#### 6.1.4.1	Overview

The structure of the custom operation URIs of the SDD_Transmission API is shown in Figure 6.1.4.1-1.

Figure 6.1.4.1-1: Custom operation URI structure of the SDD_Transmission API

Table 6.1.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the SDD_Transmission API.

Table 6.1.4.1-1: Custom operations without associated resources

| Custom operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| RequestTrans | /{transType}/request-trans | POST | Enables a VAL Server to request the SEALDD enabled regular or URLLC application data transmission service. |

The custom operations shall support the URI variables defined in table 6.1.4.1-2.

Table 6.1.4.1-2: URI variables for this custom operation

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1. |
| transType | TransType | Represents the requested transmission type (i.e., Regular transmission or URLLC transmission). |

#### 6.1.4.2	Operation: RequestTrans

##### 6.1.4.2.1	Description

The custom operation enables a VAL Server to request the SEALDD enabled regular or URLLC application data transmission service to the SEALDD Server.

##### 6.1.4.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in tables 6.1.4.2.2-1 and 6.1.4.2.2-2.

Table 6.1.4.2.2-1: Data structures supported by the <e.g. POST> Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TransReq | M | 1 | Contains the parameters to request the SEALDD enabled regular or URLLC application data transmission service. |

Table 6.1.4.2.2-2: Data structures supported by the <e.g. POST> Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TransResp | M | 1 | 200 OK | The SEALDD enabled regular or URLLC application data transmission service request was successfully received and processed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2] |
| NOTE: The manadatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI located in an alternative SEALDD Server. |

Table 6.1.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative target URI located in an alternative SEALDD Server. |

### 6.1.5	Notifications

#### 6.1.5.1	General

Notifications shall comply to clause 6.6 of 3GPP TS 29.549 [15].

Table 6.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Connection Status Notification | {notifUri} | POST | This service operation enables a SEALDD Server to notify a previously subscribed service consumer (e.g. VAL Server) on SEALDD connection status event(s). |

#### 6.1.5.2	Connection Status Notification

##### 6.1.5.2.1	Description

The Connection Status Notification is used by the SDD Server to notify a previously subscribed service consumer (e.g., VAL Server) on SEALDD connection status event(s).

##### 6.1.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| notifUri | String formatted as URI with the Callback Uri |

##### 6.1.5.2.3	Standard Methods

6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-2.

Table 6.1.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ConnectStatNotif | M | 1 | Represents the SEALDD connection status notification. |

Table 6.1.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Connection Status Notification is successfully received. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer (e.g. VAL Server) where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer (e.g. VAL Server) where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Table 6.1.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative service consumer (e.g. VAL Server) towards which the notification should be redirected. |

Table 6.1.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative service consumer (e.g. VAL Server) towards which the notification should be redirected. |

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the SDD_Transmission API.

Table 6.1.6.1-1: SDD_Transmission API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| ConnEstabData | 6.1.6.2.11 | Represents SEALDD connection status establishment data. |  |
| ConnStatusEvent | 6.1.6.3.3 | Represents Connection Status Events. |  |
| ConnInfo | 6.1.6.2.4 | Represents SEALDD Data transmission connection information. |  |
| ConnStatusNotif | 6.1.6.2.9 | Represents a Connection Status Event Notification. |  |
| ConnStatusReport | 6.1.6.2.10 | Represents a Connection Status Event report. |  |
| ConnStatusSubsc | 6.1.6.2.8 | Represents a Connection Status Event Subscription. |  |
| QoSInfo | 6.1.6.2.5 | Represents SEALDD related QoS requirements. |  |
| TransReq | 6.1.6.2.2 | Represents the parameters to request the SEALDD enabled regular or URLLC application data transmission service. |  |
| TransResp | 6.1.6.2.3 | Represents a SEALDD enabled regular or URLLC application data transmission service response. |  |
| TransType | 6.1.6.3.4 | Represents the requested transmission type (i.e., Regular transmission or URLLC transmission). |  |
| ValServBdw | 6.1.6.2.6 | Represents VAL Server related bandwidth information. |  |
| ValUsersBdw | 6.1.6.2.7 | Represents VAL users related bandwidth information. |  |

Table 6.1.6.1-2 specifies data types re-used by the SDD_Transmission API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SDD_Transmission API.

Table 6.1.6.1-2: SDD_Transmission API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Bandwidth | 3GPP TS 29.122 [2] | Represents a bandwidth. |  |
| DurationSec | 3GPP TS 29.122 [2] | Represents a time duration in seconds. |  |
| Ipv4Addr | 3GPP TS 29.122 [2] | Represents an IPv4 address. |  |
| Ipv4Addr | 3GPP TS 29.122 [2] | Represents an IPv6 address. |  |
| Port | 3GPP TS 29.122 [2] | Represents an IP port. |  |
| SupportedFeatures | 3GPP TS 29.571 [18] | Represents the supported features and used to negotiate the supported optional features. |  |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: TransReq

Table 6.1.6.2.2-1: Definition of type TransReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServerId | string | M | 1 | Contains the identifier if the VAL Server that is sending the request. |  |
| valServiceId | string | O | 0..1 | Contains the identifier of the VAL service. |  |
| valTargetUeId | ValTargetUeId | O | 0..1 | Contains the identifier if the target VAL UE. |  |
| valServerConnInfo | ConnInfo | M | 1 | Contains VAL Server's side SEALDD-S Data transmission connection information, i.e., address/port and/or URI via which the VAL Server desires to receive the application traffic from the SEALDD Server. |  |
| qosInfo | QosInfo | O | 0..1 | Contains the requested QoS requirements for the application data transmission. |  |
| valServerBdw | ValServBdw | O | 0..1 | Contains the total UL/DL bandwidth limit of the VAL Server. |  |
| valUsersBdw | ValUsersBdw | O | 0..1 | Contains the UL/DL bandwidth limits for VAL users. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.8. This attribute shall be provided if feature negotiation shall take place. |  |

##### 6.1.6.2.3	Type: TransResp

Table 6.1.6.2.3-1: Definition of type TransResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ddServerConnInfo | ConnInfo | C | 0..1 | Contains SEALDD Server's side SEALDD-S Data transmission connection information, i.e., address/port and/or URI via which the SEALDD Server desires to receive the application traffic from the VAL Server. This attribute shall be provided, if needed. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.1.8. This attribute shall be provided if feature negotiation shall take place. |  |

##### 6.1.6.2.4	Type: ConnInfo

Table 6.1.6.2.4-1: Definition of type ConnInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ipv4Addr | Ipv4Addr | C | 0..1 | Contains the IPv4 address. |  |
| ipv6Addr | Ipv6Addr | C | 0..1 | Contains the IPv6 address. |  |
| port | Port | O | 0..1 | Contains the port information. This attribute may be present only when either the "ipv4Addr" attribute or the "ipv6Addr" attribute is present. |  |
| uri | Uri | C | 0..1 | Contains a target URI. |  |
| NOTE: These attributes are mutually exclusive. Either one of them shall be present. |  |  |  |  |  |

##### 6.1.6.2.5	Type: QosInfo

Table 6.1.6.2.5-1: Definition of type QosInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

Editor's Note:	The definition and content of the QosInfo data structure is FFS.

##### 6.1.6.2.6	Type: ValServBdw

Table 6.1.6.2.6-1: Definition of type ValServBdw

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| totalUlBdw | Bandwidth | M | 1 | Contains the total UL bandwidth of the VAL Server. |  |
| totalDlBdw | Bandwidth | M | 1 | Contains the total DL bandwidth of the VAL Server. |  |

##### 6.1.6.2.7	Type: ValUsersBdw

Table 6.1.6.2.7-1: Definition of type ValUserBdw

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| minUlBdw | Bandwidth | M | 1 | Contains the minimum UL bandwidth for VAL users. |  |
| minDlBdw | Bandwidth | M | 1 | Contains the minimum DL bandwidth for VAL users. |  |
| maxUlBdw | Bandwidth | M | 1 | Contains the maximum UL bandwidth for VAL users. |  |
| maxDlBdw | Bandwidth | M | 1 | Contains the maximum DL bandwidth for VAL users. |  |

##### 6.1.6.2.8	Type: ConnStatusSubsc

Table 6.1.6.2.8-1: Definition of type ConnStatusSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| events | array(ConnStatusEvent) | M | 1..N | Represents the list of the subscribed event(s). |  |
| valServiceId | string | O | 0..1 | Represents the identity of the VAL service. |  |
| valTgtUe | ValTargetUe | O | 0..1 | Represents the target VAL UE or VAL user. |  |
| sContext | FFS | M | 1 | Represents the SEALDD-S data transmission connection information. |  |
| notifUri | Uri | M | 1 | It indicates the URI where the notification should be delivered to. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This parameter shall be provided if the feature negotiation needs to take place. |  |

Editor's Note: The data type of the "sContext" attribute is FFS and to be aligned with the implementation of the Sdd_RegularTransmission API.

Editor's Note: The encoding of the "valTgtUe" attribute is FFS.

##### 6.1.6.2.9	Type: ConnStatusNotif

Table 6.1.6.2.9-1: Definition of type ConnStatusNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reports | array(ConnStatusReport) | M | 1..N | Represents the connection status event report(s). |  |

##### 6.1.6.2.10	Type: ConnStatusReport

Table 6.1.6.2.10-1: Definition of type ConnStatusReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| event | ConnStatusEvent | M | 1 | Represents the connection status event. |  |
| valTgtUe | ValTargetUeId | O | 0..1 | Represents the target VAL UE or VAL user. |  |
| conEstStat | ConnectEstabData | C | 0..1 | Represents the SEALDD connection establishment data. This attribute shall be present only if the "event" attribute is set to "ESTABLISHED". |  |

Editor's Note: The encoding of the "valTgtUe" attribute is FFS.

##### 6.1.6.2.11	Type: ConnEstabData

Table 6.1.6.2.11-1: Definition of type ConnEstabData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sContext | FFS | M | 1 | Represents the SEALDD-S data transmission connection information. |  |
| comLifetime | DurationSec | O | 0..1 | Represents the SEALDD communication lifetime. |  |

Editor's Note: The data type of the "sContext" attribute is FFS and to be aligned with the implementation of the Sdd_RegularTransmission API.

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.1.6.3.3	Enumeration: ConnectStatEvent

The enumeration ConnectStatEvent represents a Connection Status Event. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration ConnectStatEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ESTABLISHED | Indicates that the SEALDD connection is established. |  |
| RELEASED | Indicates that the SEALDD connection is released. |  |

##### 6.1.6.3.4	Enumeration: TransType

The enumeration TransType represents the requested transmission type. It shall comply with the provisions defined in table 6.1.6.3.4-1.

Table 6.1.6.3.4-1: Enumeration TransType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| regular | Indicates that the SEALDD connection is established. |  |
| urllc | Indicates that the SEALDD connection is released. |  |
| NOTE: The enumeration values defined in this table shall use the "lower-with-hyphen" naming convention, as defined in clause 5.2.4.1 of 3GPP TS 29.122 [2], as they are used as a URI path segment. They shall not follow the "UPPER_WITH_UNDERSCORE" convention for enumerations as defined in clause 5.2.9.10 of 3GPP TS 29.122 [2]. |  |  |

#### 6.1.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 6.1.6.5	Binary data

##### 6.1.6.5.1	Binary Data Types

Table 6.1.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the SDD_Transmission API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [15].

In addition, the requirements in the following clauses are applicable for the SDD_Transmission API.

#### 6.1.7.2	Protocol Errors

No specific procedures for the SDD_Transmission API are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the SDD_Transmission API are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the SDD_Transmission API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [15].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.1.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [15] shall apply for the SDD_Transmission API.

## 6.2	SDD_DataStorage Service API

And so on if there are more than two services supported by the NF. Same structure as in clause 6.1.

## 6.3	SDD_DDContext Service API

### 6.3.1	Introduction

The SDD_DDContext service shall use the SDD_DDContext API.

The API URI of the SDD_DDContext Service API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure as defined in clause 6.5 of 3GPP TS 29.549 [15], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

-	The <apiName> shall be "sdd-ddc".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.3, the service producer (i.e. SEALDD Server) takes the role of the SCEF and the service consumer (e.g., VAL Server or another SEALDD Server) takes the role of the SCS/AS.

### 6.3.2	Usage of HTTP

The provisions of clause 6.3 of 3GPP TS 29.549 [15] shall apply for the SDD_DDContext API.

### 6.3.3	Resources

#### 6.3.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.3.3.1-1 depicts the resource URIs structure for the SDD_DDContext API.

Figure 6.3.3.1-1: Resource URI structure of the SDD_DDContext API

Table 6.3.3.1-1 provides an overview of the resources and applicable HTTP methods for the SDD_DDContext API.

Table 6.3.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| DD Context Instances | /contexts | POST | Push the DD context to the SEALDD Server. |
|  |  | GET | Pull the DD context from the SEALDD Server. |

#### 6.3.3.2	Resource: DD Context Instances

##### 6.3.3.2.1	Description

The DD Context Instances resource represents all the DD Context Instances managed by the SEALDD Server.

##### 6.3.3.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-ddc/<apiVersion>/contexts

This resource shall support the resource URI variables defined in the table 6.3.3.2.2-1.

Table 6.3.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 of 3GPP TS 29.549 [15]. |

##### 6.3.3.2.3	Resource Standard Methods

###### 6.3.3.2.3.1	POST

This operation enables to push the DD context to the SEALDD Server.

This method shall support the URI query parameters specified in table 6.3.3.2.3.1-1.

Table 6.3.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.3.3.2.3.1-2 and the response data structures and response codes specified in table 6.3.3.2.3.1-3.

Table 6.3.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DdContextPushReq | M | 1 | Represents the DD context pushed to the SEALDD Server. |

Table 6.3.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DdContextResp | M | 1 | 200 OK | Successful case. The DD context is successfully pushed to the SEALDD Server and the related information is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

Editor's Note: The error cases of the SDD_DDContext API are FFS.

Table 6.3.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

Table 6.3.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

###### 6.3.3.2.3.2	GET

This operation enables to pull the DD Context from the SEALDD Server.

This method shall support the URI query parameters specified in table 6.3.3.2.3.2-1.

Table 6.3.3.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supp-feats | SupportedFeatures | O | 0..1 | To filter irrelevant responses related to unsupported features. |

This method shall support the request data structures specified in table 6.3.3.2.3.2-2 and the response data structures and response codes specified in table 6.3.3.2.3.2-3.

Table 6.3.3.2.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.3.3.2.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DdContextResp | M | 1 | 200 OK | Successful case. The requested DD context is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Editor's Note: The application errors of the SDD_DDContext API are FFS.

Table 6.3.3.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

Table 6.3.3.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

##### 6.3.3.2.4		Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 6.3.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 6.3.5	Notifications

There are no notifications defined for this API in this release of the specification.

### 6.3.6	Data Model

#### 6.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.3.6.1-1 specifies the data types defined for the SDD_DDContext API.

Table 6.3.6.1-1: SDD_DDContext API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DdContext | 6.3.6.2.2 | Represents the DD context. |  |
| DdContextPushReq | 6.3.6.2.4 | Represents the DD context relocation push request. |  |
| DdContextResp | 6.3.6.2.5 | Represents the DD context relocation push response. |  |
| TranspLayerContext | 6.3.6.2.3 | Represents the transport layer context. |  |

Table 6.3.6.1-2 specifies data types re-used by the SDD_DDContext API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SDD_DDContext API.

Table 6.3.6.1-2: SDD_DDContext API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| RouteInformation | 3GPP TS 29.571 [18] | Represents the end point information. |  |
| SupportedFeatures | 3GPP TS 29.571 [18] | Represents the supported features and used to negotiate the supported optional features. |  |
| TransportProtocol | 3GPP TS 29.558 [17] | Represents the transport layer protocol. |  |

#### 6.3.6.2	Structured data types

##### 6.3.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

##### 6.3.6.2.2	Type: DdContext

Table 6.3.6.2.2-1: Definition of type DdContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| uuContext | FFS | M | 1 | Represents the context related to the SEALDD-Uu connection. |  |
| sContext | FFS | M | 1 | Represents the context related to the SEALDD-S connection. |  |
| trLayerContext | TranspLayerContext | O | 0..1 | Represents the transport layer context. |  |

Editor's Note: The data type of the "uuContext" attribute is FFS and to be aligned with the implementation of the Sdd_RegularTransmission API.

Editor's Note: The data type of the "sContext" attribute is FFS and to be aligned with the implementation of the Sdd_RegularTransmission API.

##### 6.3.6.2.3	Type: TranspLayerContext

Table 6.3.6.2.3-1: Definition of type TranspLayerContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| transProtoc | TransportProtocol | M | 1 | Represents the transport layer context for the SEALDD-Uu user plane communication. |  |

##### 6.3.6.2.4	Type: DdContextPushReq

Table 6.3.6.2.4-1: Definition of type DdContextPushReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ddContext | DdContext | M | 1 | Represents the DD context information. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

##### 6.3.6.2.5	Type: DdContextResp

Table 6.3.6.2.5-1: Definition of type DdContextResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| endPoint | RouteInformation | C | 0..1 | Represents the endpoint information for new SEALDD-Uu user plane communication. This attribute shall be present only in a response to a DD context push request. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Represents the supported features. This attribute shall be provided when feature negotiation needs to take place. |  |

#### 6.3.6.3	Simple data types and enumerations

##### 6.3.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.3.6.3.2	Simple data types

The simple data types defined in table 6.3.6.3.2-1shall be supported.

Table 6.3.6.3.2-1: Simple data types

| Type name | Description |
| --- | --- |

#### 6.3.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 6.3.6.5	Binary data

##### 6.3.6.5.1	Binary Data Types

Table 6.3.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 6.3.7	Error Handling

#### 6.3.7.1	General

For the SDD_DDContext API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [15].

In addition, the requirements in the following clauses are applicable for the SDD_DDContext API.

#### 6.3.7.2	Protocol Errors

No specific protocol errors for the SDD_DDContext API are specified.

#### 6.3.7.3	Application Errors

The application errors defined for the SDD_DDContext API are listed in table 6.3.7.3-1.

Table 6.3.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 6.3.8	Feature negotiation

The optional features listed in table 6.3.8-1 are defined for the SDD_TransmissionQualityMeasurement API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [15].

Table 6.3.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.3.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [15] shall apply for the SDD_DDContext API.

## 6.4	SDD_TransmissionQualityMeasurement API

### 6.4.1	Introduction

The SDD_TransmissionQualityMeasurement service shall use the SDD_TransmissionQualityMeasurement API.

The API URI of the SDD_TransmissionQualityMeasurement Service API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [15], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

-	The <apiName> shall be "sdd-tqm".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.4, the service producer (i.e. SEALDD Server) takes the role of the SCEF and the service consumer (e.g., VAL Server or another SEALDD Server) takes the role of the SCS/AS.

### 6.4.2	Usage of HTTP

The provisions of clause 6.3 of 3GPP TS 29.549 [15] shall apply for the SDD_TransmissionQualityMeasurement API.

### 6.4.3	Resources

#### 6.4.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.4.3.1-1 depicts the resource URIs structure for the SDD_TransmissionQualityMeasurement API.

Figure 6.4.3.1-1: Resource URIs structure of the SDD_TransmissionQualityMeasurement API

Table 6.4.3.1-1 provides an overview of the resources and applicable HTTP methods for the SDD_TransmissionQualityMeasurement API.

Table 6.4.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Transmission Quality Measurement Subscriptions | /subscriptions | POST | Request the creation of a Transmission Quality Measurement Subscription. |
| Individual Transmission Quality Measurement Subscription | /subscriptions/{subscriptionId} | GET | Retrieve an existing "Individual Transmission Quality Measurement Subscription". |
|  |  | PUT | Request the update of an existing "Individual Transmission Quality Measurement Subscription". |
|  |  | PATCH | Request the modification of an existing "Individual Transmission Quality Measurement Subscription". |
|  |  | DELETE | Request the deletion of an existing "Individual Transmission Quality Measurement Subscription". |
| Historical Transmission Quality Measurement Reports | /reports | GET | Retrieve Historical Transmission Quality Measurement Report(s). |

#### 6.4.3.2	Resource: Transmission Quality Measurement Subscriptions

##### 6.4.3.2.1	Description

This resource represents the collection of Transmission Quality Measurement Subscriptions managed by the SEALDD Server.

##### 6.4.3.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-tqm/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 6.4.3.2.2-1.

Table 6.4.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 of 3GPP TS 29.549 [15]. |

##### 6.4.3.2.3	Resource Standard Methods

###### 6.4.3.2.3.1	POST

The HTTP POST method allows a service consumer (e.g. VAL Server, other SEALDD Server, NSCE Server) to request the creation of a Transmission Quality Measurement Subscription at the SEALDD Server.

This method shall support the URI query parameters specified in table 6.4.3.2.3.1-1.

Table 6.4.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.2.3.1-2 and the response data structures and response codes specified in table 6.4.3.2.3.1-3.

Table 6.4.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TransQualMeasSubsc | M | 1 | Represents the parameters to request the creation of a Transmission Quality Measurement Subscription. |

Table 6.4.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TransQualMeasSubsc | M | 1 | 201 Created | Successful case. The Transmission Quality Measurement Subscription is successfully created and a representation of the created "Individual Transmission Quality Measurement Subscription" resource shall be returned. An HTTP "Location" header that contains the resource URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/sdd-tqm/<apiVersion>/subscriptions/{subscriptionId} |

##### 6.4.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.4.3.3	Resource: Individual Transmission Quality Measurement Subscription

##### 6.4.3.3.1	Description

This resource represents a Transmission Quality Measurement Subscription managed by the SEALDD Server.

##### 6.4.3.3.2	Resource Definition

Resource URI: {apiRoot}/sdd-tqm/<apiVersion>/subscriptions/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.4.3.3.2-1.

Table 6.4.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 of 3GPP TS 29.549 [15]. |
| subscriptionId | string | Represents the identifier of the "Individual Transmission Quality Measurement Subscription" resource. |

##### 6.4.3.3.3	Resource Standard Methods

###### 6.4.3.3.3.1	GET

The HTTP GET method allows a service consumer (e.g. VAL Server) to retrieve an existing "Individual Transmission Quality Measurement Subscription" resource at the SEALDD Server.

This method shall support the URI query parameters specified in table 6.4.3.3.3.1-1.

Table 6.4.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.1-2 and the response data structures and response codes specified in table 6.4.3.3.3.1-3.

Table 6.4.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.4.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TransQualMeasSubsc | M | 1 | 200 OK | Successful case. The requested "Individual Transmission Quality Measurement Subscription" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

Table 6.4.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

###### 6.4.3.3.3.2	PUT

The HTTP PUT method allows a service consumer (e.g. VAL Server) to request the update of an existing "Individual Transmission Quality Measurement Subscription" resource at the SEALDD Server.

This method shall support the URI query parameters specified in table 6.4.3.3.3.2-1.

Table 6.4.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.2-2 and the response data structures and response codes specified in table 6.4.3.3.3.2-3.

Table 6.4.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TransQualMeasSubsc | M | 1 | Represents the updated representation of the "Individual Transmission Quality Measurement Subscription" resource. |

Table 6.4.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TransQualMeasSubsc | M | 1 | 200 OK | Successful case. The "Individual Transmission Quality Measurement Subscription" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Transmission Quality Measurement Subscription" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

Table 6.4.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

###### 6.4.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer (e.g. VAL Server) to request the modification of an existing "Individual Transmission Quality Measurement Subscription" resource at the SEALDD Server.

This method shall support the URI query parameters specified in table 6.4.3.3.3.3-1.

Table 6.4.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.3-2 and the response data structures and response codes specified in table 6.4.3.3.3.3-3.

Table 6.4.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TransQualMeasSubscPatch | M | 1 | Represents the parameters to request the modification of the "Individual Transmission Quality Measurement Subscription" resource. |

Table 6.4.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| TransQualMeasSubsc | M | 1 | 200 OK | Successful case. The "Individual Transmission Quality Measurement Subscription" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Transmission Quality Measurement Subscription" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

Table 6.4.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

###### 6.4.3.3.3.4	DELETE

The HTTP DELETE method allows a service consumer (e.g. VAL Server) to request the deletion of an existing "Individual Transmission Quality Measurement Subscription" resource at the SEALDD Server.

This method shall support the URI query parameters specified in table 6.4.3.3.3.4-1.

Table 6.4.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.4.3.3.3.4-2 and the response data structures and response codes specified in table 6.4.3.3.3.4-3.

Table 6.4.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.4.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Transmission Quality Measurement Subscription" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

Table 6.4.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

##### 6.4.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.4.3.4	Resource: Historical Transmission Quality Measurement Reports

##### 6.4.3.4.1	Description

This resource represents the collection of Historical Transmission Quality Measurement Reports managed by the SEALDD Server.

##### 6.4.3.4.2	Resource Definition

Resource URI: {apiRoot}/sdd-tqm/<apiVersion>/reports

This resource shall support the resource URI variables defined in table 6.4.3.4.2-1.

Table 6.4.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 of 3GPP TS 29.549 [15]. |

##### 6.4.3.4.3	Resource Standard Methods

###### 6.4.3.4.3.1	GET

The HTTP GET method allows a service consumer (e.g. VAL Server, SEALDD Server, NSCE Server) to retrieve Historical Transmission Quality Measurement Report(s) from the SEALDD Server.

This method shall support the URI query parameters specified in table 6.4.3.4.3.1-1.

Table 6.4.3.4.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| app-traffic-ids | array(string) | M | 1 | Contains the identifier(s) of the targeted application traffic. This can be in the form of e.g. VAL Service ID, VAL Server ID. |  |
| val-group-id | string | O | 0..1 | Contains the identity of the VAL group to which the request is related. (NOTE) |  |
| val-ue-ids-list | array(ValTargetUeId) | O | 1..N | Contains the list of the identifier(s) of the VAL UE(s) to which the request is related. (NOTE) |  |
| all-val-ues | boolean | O | 0..1 | Indicates that the request is related to all VAL UE(s) of the application traffic identified by the application traffic identifier(s) provided within the "appTrafficIds" attribute. (NOTE) |  |
| supp-feat | SupportedFeatures | O | 0..1 | Contains the list of supported features. This attribute shall be present when feature negotiation needs to take place. |  |
| NOTE: These query parameters are mutually exclusive. Either one of them may be present. |  |  |  |  |  |

Editor's Note:	Whether the VAL UE Identity can also take the form of a UE address as specified in Table 9.7.3.1-1 of 3GPP TS 23.433 is FFS.

This method shall support the request data structures specified in table 6.4.3.4.3.1-2 and the response data structures and response codes specified in table 6.4.3.4.3.1-3.

Table 6.4.3.4.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.4.3.4.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| HistTransQualMeasReports | M | 1 | 200 OK | Successful case. The requested Historical Transmission Quality Measurement Report(s) shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.3.4.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

Table 6.4.3.4.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

##### 6.4.3.4.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 6.4.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 6.4.5	Notifications

#### 6.4.5.1	General

Notifications shall comply to clause 6.6 of 3GPP TS 29.549 [15].

Table 6.4.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Transmission Quality Measurement Notification | {notifUri} | POST | This service operation enables a SEALDD Server to notify a previously subscribed service consumer (e.g. VAL Server) on Transmission Quality Measurement reports. |

#### 6.4.5.2	Transmission Quality Measurement Notification

##### 6.4.5.2.1	Description

The Transmission Quality Measurement Notification is used by a SEALDD Server to notify a previously subscribed service consumer on Transmission Quality Measurement reports.

##### 6.4.5.2.2	Target URI

The Callback URI "{notifUri}" shall be used with the callback URI variables defined in table 6.4.5.2.2-1.

Table 6.4.5.2.2-1: Callback URI variables

| Name | Data type | Definition |
| --- | --- | --- |
| notifUri | Uri | String formatted as a URI containing the Callback URI. The notification URI is provided as part of the corresponding Transmission Quality Measurement Subscription creation/update/modification request as defined in clause 6.4.3. |

##### 6.4.5.2.3	Standard Methods

###### 6.4.5.2.3.1	POST

This method shall support the request data structures specified in table 6.4.5.2.3.1-1 and the response data structures and response codes specified in table 6.4.5.2.3.1-2.

Table 6.4.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TransQualMeasNotif | M | 1 | Represents a Transmission Quality Measurement notification. |

Table 6.4.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The Transmission Quality Measurement notification is successfully received and acknowledged. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI representing the end point of an alternative service consumer where the notification should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.4.5.2.3.1-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

Table 6.4.5.2.3.1-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI representing the end point of an alternative service consumer towards which the notification should be redirected. |

### 6.4.6	Data Model

#### 6.4.6.1	General

This clause specifies the application data model supported by the API.

Table 6.4.6.1-1 specifies the data types defined for the SDD_TransmissionQualityMeasurement API.

Table 6.4.6.1-1: SDD_TransmissionQualityMeasurement API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| HistTransQualMeasReports | 6.4.6.2.10 | Represents Historical Transmission Quality Measurement Report(s). |  |
| MeasurementId | 6.4.6.3.3 | Represents the transmission quality measurement type. |  |
| RepGranularity | 6.4.6.3.4 | Represents the reporting granularity. |  |
| TransQualMeasCriteria | 6.4.6.2.7 | Represents the transmission quality measurement reporting criteria. |  |
| TransQualMeasData | 6.4.6.2.8 | Represents the transmission quality measurement data. |  |
| TransQualMeasNotif | 6.4.6.2.5 | Represents a Transmission Quality Measurement notification. |  |
| TransQualMeasReport | 6.4.6.2.6 | Represents a Transmission Quality Measurement report. |  |
| TransQualMeasReq | 6.4.6.2.3 | Represents Transmission Quality Measurement reporting requirements. |  |
| TransQualMeasSubsc | 6.4.6.2.2 | Represents a Transmission Quality Measurement subscription. |  |
| TransQualMeasSubscPatch | 6.4.6.2.4 | Represents the requested modifications to a Transmission Quality Measurement subscription. |  |
| ValTargetUeId | 6.4.6.2.9 | Represents the identifier of a targeted VAL UE. |  |

Table 6.4.6.1-2 specifies data types re-used by the SDD_TransmissionQualityMeasurement API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SDD_TransmissionQualityMeasurement API.

Table 6.4.6.1-2: SDD_TransmissionQualityMeasurement API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| BitRate | 3GPP TS 29.571 [18] | Represents a bit rate. |  |
| BitRateRm | 3GPP TS 29.571 [18] | Represents a bit rate, but with the OpenAPI "nullable: true" property. |  |
| DateTime | 3GPP TS 29.122 [2] | Represents a date and a time. |  |
| DateTimeRo | 3GPP TS 29.122 [2] | Represents a date and a time with the read-only property. |  |
| DurationSec | 3GPP TS 29.122 [2] | Represents a time duration in seconds. |  |
| Ipv4Addr | 3GPP TS 29.122 [2] | Represents an IPv4 address. |  |
| Ipv6Addr | 3GPP TS 29.122 [2] | Represents an IPv6 address. |  |
| NotificationMethod | 3GPP TS 29.508 [16] | Represents the reporting type. |  |
| PacketLossRate | 3GPP TS 29.571 [18] | Represents the packet loss rate. |  |
| PacketLossRateRm | 3GPP TS 29.571 [18] | Represents the packet loss rate, but with the OpenAPI "nullable: true" property. |  |
| Port | 3GPP TS 29.122 [2] | Represents an IP port. |  |
| SupportedFeatures | 3GPP TS 29.571 [18] | Used to negotiate the applicability of the optional features. |  |
| TimeWindow | 3GPP TS 29.122 [2] | Represents a time window. |  |
| Uint32 | 3GPP TS 29.571 [18] | Represents an unsigned 32-bit integer. |  |
| Uint32Rm | 3GPP TS 29.571 [18] | Represents an unsigned 32-bit integer, but with the OpenAPI "nullable: true" property. |  |
| Uinteger | 3GPP TS 29.571 [18] | Represents an unsigned integer. |  |
| UintegerRm | 3GPP TS 29.571 [18] | Represents an unsigned integer, but with the OpenAPI "nullable: true" property. |  |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |
| ValidityConditions | 3GPP TS 29.549 [15] | Represents temporal and/or spatial validity conditions. |  |

#### 6.4.6.2	Structured data types

##### 6.4.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

##### 6.4.6.2.2	Type: TransQualMeasSubsc

Table 6.4.6.2.2-1: Definition of type TransQualMeasSubsc

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appTrafficIds | array(string) | M | 1..N | Contains the identifier(s) of the targeted application traffic. This can be in the form of e.g. VAL Service ID, VAL Server ID. |  |
| valGroupId | string | C | 0..1 | Contains the identity of the VAL group to which the subscription is related. (NOTE) |  |
| valUeIdsList | array(ValTargetUeId) | C | 1..N | Contains the list of the identifier(s) of the VAL UE(s) to which the subscription is related. (NOTE) |  |
| allValUesInd | boolean | C | 0..1 | Indicates that the subscription is related to all VAL UE(s) of the application traffic identified by the application traffic identifier(s) provided within the "appTrafficIds" attribute. (NOTE) |  |
| measConds | array(ValidityConditions) | O | 1..N | Contains the set of temporal and/or spatial measurement conditions of the subscription. |  |
| reqs | map(TransQualMeasReq) | M | 1..N | Contains the transmission quality measurement reporting requirements of the subscription. The key of the map shall be any unique string encoded value. |  |
| subsExpTime | DateTimeRo | O | 0..1 | Indicates the time at which the Transmission Quality Measurement subscription shall expire. This attribute may only be present in Transmission Quality Measurement subscription creation/update responses. If this attribute is absent, this means that the Transmission Quality Measurement subscription shall not expire, until explicitly deleted by the service consumer (e.g. VAL Server). |  |
| notifUri | Uri | M | 1 | Contains the URI via which notifications shall be delivered. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.4.8. This attribute shall be provided if feature negotiation shall take place. |  |
| NOTE: These attributes are mutually exclusive. Either one of them shall be present. |  |  |  |  |  |

Editor's Note:	Whether the VAL UE Identity can also take the form of a UE address as specified in Table 9.7.3.1-1 of 3GPP TS 23.433 is FFS.

##### 6.4.6.2.3	Type: TransQualMeasReq

Table 6.4.6.2.3-1: Definition of type TransQualMeasReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| measId | array(MeasurementId) | M | 1..N | Contains the requested transmission quality measurement(s). |  |
| repType | NotificationMethod | O | 0..1 | Contains the requested reporting type (e.g. periodic reporting). The default value is "PERIODIC" (i.e., periodic reporting), if this attribute is not present. |  |
| repPeriodicity | DurationSec | O | 0..1 | Contains the reporting periodicity. This attribute shall be present only if the "repType" attribute is not present, or is present and set to the value "PERIODIC". |  |
| repGranularity | RepGranularity | O | 0..1 | Contains the reporting granularity, i.e., whether the requested transmission quality measurement reporting is per individual VAL UE, VAL group or all VAL UE(s) granularity. |  |
| measWindow | TimeWindow | O | 0..1 | Contains the measurement window for the requested transmission quality measurements. |  |
| measExpTime | DateTime | O | 0..1 | Contains the measurement expiration time. |  |
| repCriteria | TransQualMeasCriteria | O | 0..1 | Contains the transmission quality measurement reporting criteria. Only the criteria related to the subscribed measurements within the "measId" attribute shall be present within this attribute. |  |

##### 6.4.6.2.4	Type: TransQualMeasSubscPatch

Table 6.4.6.2.4-1: Definition of type TransQualMeasSubscPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| measConds | array(ValidityConditions) | O | 1..N | Contains the updated set of temporal and/or spatial measurement conditions of the subscription. |  |
| reqs | map(TransQualMeasReq) | O | 1..N | Contains the updated transmission quality measurement reporting requirements of the subscription. The key of the map shall be any unique string encoded value and shall be set to the same value as the one provided during the creation of the transmission quality measurement subscription. |  |
| notifUri | Uri | O | 0..1 | Contains the updated URI via which the notifications shall be delivered. |  |

##### 6.4.6.2.5	Type: TransQualMeasNotif

Table 6.4.6.2.5-1: Definition of type TransQualMeasNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reports | array(TransQualMeasReport) | M | 1..N | Contains the transmission quality measurement report(s). |  |

##### 6.4.6.2.6	Type: TransQualMeasReport

Table 6.4.6.2.6-1: Definition of type TransQualMeasReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| measId | array(MeasurementId) | M | 1..N | Contains the reported transmission quality measurement(s). |  |
| valUeIds | array(ValTargetUeId) | C | 1..N | Contains the list of the identifier(s) of the VAL UE(s) to which the transmission quality measurement report is related. This attribute may not be present only if the corresponding subscription is for a single VAL UE. Otherwise, this attribute shall be present. |  |
| measData | TransQualMeasData | O | 0..1 | Contains the reported transmission quality measurement data. |  |

##### 6.4.6.2.7	Type: TransQualMeasCriteria

Table 6.4.6.2.7-1: Definition of type TransQualMeasCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| minLatency | UintegerRm | O | 0..1 | Contains the requested minimum latency (expressed in milliseconds) for transmission quality measurement reporting. (NOTE) |  |
| maxLatency | UintegerRm | O | 0..1 | Contains the requested maximum latency (expressed in milliseconds) for transmission quality measurement reporting. (NOTE) |  |
| minBitRate | BitRateRm | O | 0..1 | Contains the requested minimum bit rate for transmission quality measurement reporting. (NOTE) |  |
| maxBitRate | BitRateRm | O | 0..1 | Contains the requested maximum bit rate for transmission quality measurement reporting. (NOTE) |  |
| minPackLossRate | PacketLossRateRm | O | 0..1 | Contains the requested minimum packet loss rate for transmission quality measurement reporting. (NOTE) |  |
| maxPackLossRate | PacketLossRateRm | O | 0..1 | Contains the requested maximum packet loss rate for transmission quality measurement reporting. (NOTE) |  |
| minJitter | Uint32Rm | O | 0..1 | Contains the requested minimum jitter (expressed in nanoseconds) for transmission quality measurement reporting. (NOTE) |  |
| maxJitter | Uint32Rm | O | 0..1 | Contains the requested maximum jitter (expressed in nanoseconds) for transmission quality measurement reporting. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

##### 6.4.6.2.8	Type: TransQualMeasData

Table 6.4.6.2.8-1: Definition of type TransQualMeasData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| minLatency | Uinteger | O | 0..1 | Contains the measured minimum latency (expressed in milliseconds). (NOTE) |  |
| maxLatency | Uinteger | O | 0..1 | Contains the measured maximum latency (expressed in milliseconds). (NOTE) |  |
| avgLatency | Uinteger | O | 0..1 | Contains the measured average latency (expressed in milliseconds). (NOTE) |  |
| stdDevLatency | Uinteger | O | 0..1 | Contains the standard deviation (expressed in milliseconds) for the measured latency. This attribute may be present only if the "minLatency", "maxLatency" and/or "avgLatency" attributes is/are present. |  |
| kPercLatency | Uinteger | O | 0..1 | Contains the kPercentile (expressed in milliseconds) for the measured latency. This attribute may be present only if the "minLatency", "maxLatency" and/or "avgLatency" attributes is/are present. |  |
| minBitRate | BitRate | O | 0..1 | Contains the measured minimum bit rate. (NOTE) |  |
| maxBitRate | BitRate | O | 0..1 | Contains the measured maximum bit rate. (NOTE) |  |
| avgBitRate | BitRate | O | 0..1 | Contains the measured average bit rate. (NOTE) |  |
| stdDevBitRate | BitRate | O | 0..1 | Contains the standard deviation for the measured bit rate. This attribute may be present only if the "minBitRate", "maxBitRate" and/or "avgBitRate" attributes is/are present. |  |
| kPercBitRate | BitRate | O | 0..1 | Contains the kPercentile for the measured bit rate. This attribute may be present only if the "minBitRate", "maxBitRate" and/or "avgBitRate" attributes is/are present. |  |
| minPackLossRate | PacketLossRate | O | 0..1 | Contains the measured minimum packet loss rate. (NOTE) |  |
| maxPackLossRate | PacketLossRate | O | 0..1 | Contains the measured maximum packet loss rate. (NOTE) |  |
| avgPackLossRate | PacketLossRate | O | 0..1 | Contains the measured average packet loss rate. (NOTE) |  |
| stdDevPackLossRate | PacketLossRate | O | 0..1 | Contains the standard deviation for the measured packet loss rate. This attribute may be present only if the "minPackLossRate", "maxPackLossRate" and/or "avgPackLossRate" attributes is/are present. |  |
| kPercPackLossRate | PacketLossRate | O | 0..1 | Contains the kPercentile for the measured packet loss rate. This attribute may be present only if the "minPackLossRate", "maxPackLossRate" and/or "avgPackLossRate" attributes is/are present. |  |
| minJitter | Uint32 | O | 0..1 | Contains the measured minimum jitter (expressed in nanoseconds). (NOTE) |  |
| maxJitter | Uint32 | O | 0..1 | Contains the measured maximum jitter (expressed in nanoseconds). (NOTE) |  |
| avgJitter | Uint32 | O | 0..1 | Contains the measured average jitter (expressed in nanoseconds). (NOTE) |  |
| stdDevJitter | Uint32 | O | 0..1 | Contains the standard deviation (expressed in nanoseconds) for the measured Jitter. This attribute may be present only if the "minJitter", "maxJitter" and/or "avgJitter" attributes is/are present. |  |
| kPercJitter | Uint32 | O | 0..1 | Contains the kPercentile (expressed in nanoseconds) for the measured Jitter. This attribute may be present only if the "minJitter", "maxJitter" and/or "avgJitter" attributes is/are present. |  |
| measPeriod | DurationSec | O | 0..1 | Contains the measurement period of the reported measurements. |  |
| timestamp | DateTime | O | 0..1 | Contains the timestamp of the reported measurements. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

##### 6.4.6.2.9	Type: ValTargetUeId

Table 6.4.6.2.9-1: Definition of type ValTargetUeId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valUserId | string | C | 0..1 | Contains the unique identifier of the targeted VAL user. (NOTE) |  |
| valUeId | string | C | 0..1 | Contains the unique identifier of the targeted VAL UE. (NOTE) |  |
| valUeIpv4Addr | Ipv4Addr | C | 0..1 | Contains the IPv4 address of the targeted VAL UE. (NOTE) |  |
| valUeIpv6Addr | Ipv6Addr | C | 0..1 | Contains the IPv6 address of the targeted VAL UE. (NOTE) |  |
| port | Port | O | 0..1 | Contains the port information of the targeted VAL UE. This attribute may only be present if either the "valUeIpv4Addr" attribute or the "valUeIpv6Addr" attribute is present. |  |
| NOTE: These attributes are mutually exclusive. Either one of them shall be present. |  |  |  |  |  |

##### 6.4.6.2.10	Type: HistTransQualMeasReports

Table 6.4.6.2.10-1: Definition of type HistTransQualMeasReports

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| reports | array(TransQualMeasReport) | M | 0..N | Contains the Historical Transmission Quality Measurement Report(s). If there are no Historical Transmission Quality Measurement Report(s) fulfilling the request, an empty array shall be returned within this attribute. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.4.8. This attribute shall be provided if feature negotiation is taking place. |  |

#### 6.4.6.3	Simple data types and enumerations

##### 6.4.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.4.6.3.2	Simple data types

The simple data types defined in table 6.4.6.3.2-1 shall be supported.

Table 6.4.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.4.6.3.3	Enumeration: MeasurementId

The enumeration MeasurementId represents the requested measurement identifier. It shall comply with the provisions defined in table 6.4.6.3.3-1.

Table 6.4.6.3.3-1: Enumeration MeasurementId

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LATENCY | Indicates that the requested/reported measurement is the latency. |  |
| BITRATE | Indicates that the requested/reported measurement is the bit rate. |  |
| PACKET_LOSS_RATE | Indicates that the requested/reported measurement is the packet loss rate. |  |
| JITTER | Indicates that the requested/reported measurement is the jitter. |  |

##### 6.4.6.3.4	Enumeration: RepGranularity

The enumeration RepGranularity represents the requested reporting granularity. It shall comply with the provisions defined in table 6.4.6.3.4-1.

Table 6.4.6.3.4-1: Enumeration RepGranularity

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| INDIVIDUAL_VAL_UE | Indicates that the requested granularity is individual VAL UE. |  |
| VAL_GROUP | Indicates that the requested granularity is VAL Group. |  |
| ALL_UES | Indicates that the requested granularity is All UE(s). |  |

#### 6.4.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 6.4.6.5	Binary data

##### 6.4.6.5.1	Binary Data Types

Table 6.4.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 6.4.7	Error Handling

#### 6.4.7.1	General

For the SDD_TransmissionQualityMeasurement API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [15].

In addition, the requirements in the following clauses are applicable for the SDD_TransmissionQualityMeasurement API.

#### 6.4.7.2	Protocol Errors

No specific protocol errors for the SDD_TransmissionQualityMeasurement API are specified.

#### 6.4.7.3	Application Errors

The application errors defined for the SDD_TransmissionQualityMeasurement API are listed in Table 6.4.7.3-1.

Table 6.4.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 6.4.8	Feature negotiation

The optional features listed in table 6.4.8-1 are defined for the SDD_TransmissionQualityMeasurement API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [15].

Table 6.4.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.4.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [15] shall apply for the SDD_TransmissionQualityMeasurement API.

## 6.5	SDD_PolicyConfiguration API

### 6.5.1	Introduction

The SDD_PolicyConfiguration service shall use the SDD_PolicyConfiguration API.

The API URI of the SDD_PolicyConfiguration Service API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [15], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

-	The <apiName> shall be "sdd-pc".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5 of 3GPP TS 29.549 [15].

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 6.5, the service producer (i.e. SEALDD Server) takes the role of the SCEF and the service consumer (e.g., VAL Server) takes the role of the SCS/AS.

### 6.5.2	Usage of HTTP

The provisions of clause 6.3 of 3GPP TS 29.549 [15] shall apply for the SDD_PolicyConfiguration API.

### 6.5.3	Resources

#### 6.5.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.5.3.1-1 depicts the resource URIs structure for the SDD_PolicyConfiguration API.

Figure 6.5.3.1-1: Resource URIs structure of the SDD_PolicyConfiguration API

Table 6.5.3.1-1 provides an overview of the resources and applicable HTTP methods for the SDD_PolicyConfiguration API.

Table 6.5.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Policy Configurations | /configurations | POST | Request the creation of a Policy Configuration. |
| Individual Policy Configuration | /configurations/{configId} | GET | Retrieve an existing "Individual Policy Configuration" resource. |
|  |  | PUT | Request the update of an existing "Individual Policy Configuration" resource. |
|  |  | PATCH | Request the modification of an existing "Individual Policy Configuration" resource. |
|  |  | DELETE | Request the deletion of an existing "Individual Policy Configuration" resource. |

#### 6.5.3.2	Resource: Policy Configurations

##### 6.5.3.2.1	Description

This resource represents the collection of SEALDD Policy Configurations managed by the SEALDD Server.

##### 6.5.3.2.2	Resource Definition

Resource URI: {apiRoot}/sdd-pc/<apiVersion>/configurations

This resource shall support the resource URI variables defined in table 6.5.3.2.2-1.

Table 6.5.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5.1. |

##### 6.5.3.2.3	Resource Standard Methods

###### 6.5.3.2.3.1	POST

The HTTP POST method allows a service consumer (e.g., VAL Server) to request the creation of a Policy Configuration at the SEALDD Server.

This method shall support the URI query parameters specified in table 6.5.3.2.3.1-1.

Table 6.5.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.5.3.2.3.1-2 and the response data structures and response codes specified in table 6.5.3.2.3.1-3.

Table 6.5.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolicyConfig | M | 1 | Represents the parameters to request the creation of a Policy Configuration. |

Table 6.5.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolicyConfig | M | 1 | 201 Created | Successful case. The Policy Configuration is successfully created and a representation of the created "Individual Policy Configuration" resource shall be returned. An HTTP "Location" header that contains the resource URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.5.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/sdd-pc/<apiVersion>/configurations/{configId} |

##### 6.5.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.5.3.3	Resource: Individual Policy Configuration

##### 6.5.3.3.1	Description

This resource represents a SEALDD Policy Configuration managed by the SEALDD Server.

##### 6.5.3.3.2	Resource Definition

Resource URI: {apiRoot}/sdd-pc/<apiVersion>/configurations/{configId}

This resource shall support the resource URI variables defined in table 6.5.3.3.2-1.

Table 6.5.3.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5.1. |
| configId | string | Represents the identifier of the "Individual Policy Configuration" resource. |

##### 6.5.3.3.3	Resource Standard Methods

###### 6.5.3.3.3.1	GET

The HTTP GET method allows a service consumer (e.g., VAL Server) to retrieve an existing "Individual Policy Configuration" resource at the SEALDD Server.

This method shall support the URI query parameters specified in table 6.5.3.3.3.1-1.

Table 6.5.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.5.3.3.3.1-2 and the response data structures and response codes specified in table 6.5.3.3.3.1-3.

Table 6.5.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.5.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolicyConfig | M | 1 | 200 OK | Successful case. The requested "Individual Policy Configuration" resource shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.5.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

Table 6.5.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

###### 6.5.3.3.3.2	PUT

The HTTP PUT method allows a service consumer (e.g., VAL Server) to request the update of an existing "Individual Policy Configuration" resource at the SEALDD Server.

This method shall support the URI query parameters specified in table 6.5.3.3.3.2-1.

Table 6.5.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.5.3.3.3.2-2 and the response data structures and response codes specified in table 6.5.3.3.3.2-3.

Table 6.5.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolicyConfig | M | 1 | Represents the updated representation of the "Individual Policy Configuration" resource. |

Table 6.5.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolicyConfig | M | 1 | 200 OK | Successful case. The "Individual Policy Configuration" resource is successfully updated and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Policy Configuration" resource is successfully updated and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP PUT method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.5.3.3.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

Table 6.5.3.3.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

###### 6.5.3.3.3.3	PATCH

The HTTP PATCH method allows a service consumer (e.g., VAL Server) to request the modification of an existing "Individual Policy Configuration" resource at the SEALDD Server.

This method shall support the URI query parameters specified in table 6.5.3.3.3.3-1.

Table 6.5.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.5.3.3.3.3-2 and the response data structures and response codes specified in table 6.5.3.3.3.3-3.

Table 6.5.3.3.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolicyConfigPatch | M | 1 | Represents the parameters to request the modification of the "Individual Policy Configuration" resource. |

Table 6.5.3.3.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolicyConfig | M | 1 | 200 OK | Successful case. The "Individual Policy Configuration" resource is successfully modified and a representation of the updated resource shall be returned in the response body. |
| n/a |  |  | 204 No Content | Successful case. The "Individual Policy Configuration" resource is successfully modified and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP PATCH method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.5.3.3.3.3-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

Table 6.5.3.3.3.3-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

###### 6.5.3.3.3.4	DELETE

The HTTP DELETE method allows a service consumer (e.g., VAL Server) to request the deletion of an existing "Individual Policy Configuration" resource at the SEALDD Server.

This method shall support the URI query parameters specified in table 6.5.3.3.3.4-1.

Table 6.5.3.3.3.4-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.5.3.3.3.4-2 and the response data structures and response codes specified in table 6.5.3.3.3.4-3.

Table 6.5.3.3.3.4-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.5.3.3.3.4-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The "Individual Policy Configuration" resource is successfully deleted. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SEALDD Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status code for the HTTP DELETE method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.5.3.3.3.4-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

Table 6.5.3.3.3.4-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative SEALDD Server. |

##### 6.5.3.3.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

### 6.5.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

### 6.5.5	Notifications

There are no notifications defined for this API in this release of the specification.

### 6.5.6	Data Model

#### 6.5.6.1	General

This clause specifies the application data model supported by the API.

Table 6.5.6.1-1 specifies the data types defined for the SDD_PolicyConfiguration API.

Table 6.5.6.1-1: SDD_PolicyConfiguration API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| BdwCtrlPolicy | 6.5.6.3.4 | Represents the bandwidth control policy. |  |
| QualGuarPolicy | 6.5.6.3.3 | Represents the quality guarantee policy. |  |
| PolicyConfig | 6.5.6.2.2 | Represents a SEALDD Policy Configuration. |  |
| PolicyConfigPatch | 6.5.6.2.3 | Represents the parameters to request the modification of a SEALDD Policy Configuration. |  |
| SealddPolicy | 6.5.6.2.4 | Represents a SEALDD Policy. |  |

Table 6.5.6.1-2 specifies data types re-used by the SDD_PolicyConfiguration API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SDD_PolicyConfiguration API.

Table 6.5.6.1-2: SDD_PolicyConfiguration API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTimeRo | 3GPP TS 29.571 [18] | Represents a date and a time with the "read-only" property. |  |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |
| ValTargetUeId | Clause 6.4.6.2.9 | Represents the identifier of a targeted VAL UE. |  |
| SupportedFeatures | 3GPP TS 29.571 [18] | Used to negotiate the applicability of the optional features. |  |

#### 6.5.6.2	Structured data types

##### 6.5.6.2.1	Introduction

This clause defines the data structures to be used in resource representations.

##### 6.5.6.2.2	Type: PolicyConfig

Table 6.5.6.2.2-1: Definition of type PolicyConfig

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appTrafficIds | array(string) | M | 1 | Contains the identifier(s) of the targeted application traffic. This can be in the form of e.g. VAL Service ID, VAL Server ID. |  |
| valUeId | ValTargetUeId | O | 0..1 | Contains the identifier of the VAL UE to which the SEALDD policy configuration is related. |  |
| sealddPol | SealddPolicy | M | 1 | Represents the SEALDD policy that is to be configured. |  |
| expTime | DateTimeRo | O | 0..1 | Contains the expiration time of the policy configuration. This attribute may only be present in the Policy Configuration creation/update responses. If this attribute is absent, this means that the Policy Configuration shall not expire, until explicitly deleted by the service consumer (e.g. VAL Server). |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported features among the ones defined in clause 6.5.8. This attribute shall be provided if feature negotiation shall take place. |  |

Editor's Note:	The encoding of the "valUeId" attribute is FFS.

##### 6.5.6.2.3	Type: PolicyConfigPatch

Table 6.5.6.2.3-1: Definition of type PolicyConfigPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sealddPol | SealddPolicy | O | 0..1 | Represents the updated SEALDD policy. |  |

##### 6.5.6.2.4	Type: SealddPolicy

Table 6.5.6.2.4-1: Definition of type SealddPolicy

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| qualGuarSets | array(QualGuarPolicy) | O | 1..N | Contains the quality guarantee policy. This attribute shall not contain any combination of the "ESTABLISH_REDUNDANT_TRANS_PATH", "REESTABLISH_TRANS_PATH", "SWITCH_TO_BACKUP_TRANS_PATH" actions. (NOTE) |  |
| bdwCtrlSets | array(BdwCtrlPolicy) | O | 1..N | Contains the bandwidth control policy. This attribute shall not contain at the same time both the "REALLOCATE_DL" and "NOT_REALLOCATE_DL" actions nor both the "REALLOCATE_UL" and "NOT_REALLOCATE_UL" actions. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

#### 6.5.6.3	Simple data types and enumerations

##### 6.5.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.5.6.3.2	Simple data types

The simple data types defined in table 6.5.6.3.2-1 shall be supported.

Table 6.5.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.5.6.3.3	Enumeration: QualGuarPolicy

The enumeration QualGuarPolicy represents the quality guarantee policy. It shall comply with the provisions defined in table 6.5.6.3.3-1.

Table 6.5.6.3.3-1: Enumeration QualGuarPolicy

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| ESTABLISH_REDUNDANT_TRANS_PATH | Indicates that the quality guarantee action to be performed is to establish a redundant transmission path. |  |
| REESTABLISH_TRANS_PATH | Indicates that the quality guarantee action to be performed re- establish the transmission path. |  |
| SWITCH_TO_BACKUP_TRANS_PATH | Indicates that the quality guarantee action to be performed is to switch to the backup transmission path. |  |
| CHANGE_SEALDD_SERVER | Indicates that the quality guarantee action to be performed is to change the connected SEALDD Server. |  |

##### 6.5.6.3.4	Enumeration: BdwCtrlPolicy

The enumeration BdwCtrlPolicy represents the the bandwidth control policy. It shall comply with the provisions defined in table 6.5.6.3.4-1.

Table 6.5.6.3.4-1: Enumeration BdwCtrlPolicy

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| REALLOCATE_DL | Indicates that the bandwidth control action to be performed is to reallocate the bandwidth limit between different VAL users for DL traffic. |  |
| REALLOCATE_UL | Indicates that the bandwidth control action to be performed is to reallocate the bandwidth limit between different VAL users for UL traffic. |  |
| NOT_REALLOCATE_DL | Indicates that the bandwidth control action to be performed is to not reallocate the bandwidth limit between different VAL users for DL traffic. |  |
| NOT_REALLOCATE_UL | Indicates that the bandwidth control action to be performed is to not reallocate the bandwidth limit between different VAL users for UL traffic. |  |

#### 6.5.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

#### 6.5.6.5	Binary data

##### 6.5.6.5.1	Binary Data Types

Table 6.5.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |

### 6.5.7	Error Handling

#### 6.5.7.1	General

For the SDD_PolicyConfiguration API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [15].

In addition, the requirements in the following clauses are applicable for the SDD_PolicyConfiguration API.

#### 6.5.7.2	Protocol Errors

No specific protocol errors for the SDD_PolicyConfiguration API are specified.

#### 6.5.7.3	Application Errors

The application errors defined for the SDD_PolicyConfiguration API are listed in Table 6.5.7.3-1.

Table 6.5.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

### 6.5.8	Feature negotiation

The optional features listed in table 6.5.8-1 are defined for the SDD_PolicyConfiguration API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [15].

Table 6.5.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.5.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [15] shall apply for the SDD_PolicyConfiguration API.

# 7	Using Common API Framework

The provisions of clause 8 of 3GPP TS 29.549 [15] shall apply for the SEALDD Server APIs defined in this specification.

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [5] and clause 5B of 3GPP TR 21.900 [7]).

# A.2	SDD_Transmission API

openapi: 3.0.0

info:

title: SEALDD Server Data Transmission Service

version: 1.0.0-alpha.2

description: |

SEALDD Server Data Transmission Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.548 V0.4.0; Service Enabler Architecture Layer for Verticals (SEAL);

SEAL Data Delivery (SEALDD) Server Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.548/

servers:

- url: '{apiRoot}/sdd-trans/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/{transType}/request-trans:

parameters:

- name: transType

in: path

description: >

Represents the requested transmission type.

required: true

schema:

$ref: '#/components/schemas/TransType'

post:

summary: Request SEALDD enabled Regular or URLLC Data Transmission.

operationId: RequestTrans

tags:

- Request SEALDD Data Transmission

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TransReq'

responses:

'200':

description: >

OK. The SEALDD enabled Regular or URLLC application data transmission service request

was successfully received and processed.

content:

application/json:

schema:

$ref: '#/components/schemas/TransResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

summary: Create a new Connection Status Subscription.

operationId: SubscribeConnStatus

tags:

- Connection Status Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ConnStatusSubsc'

responses:

'201':

description: >

Successful case. The requested Connection Status Subscription resource is successfully

created and a representation of the created"Individual Connection Status Subscription"

resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ConnStatusSubsc'

headers:

Location:

description: >

Contains the URI of the newly created connection status subscription resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

ConnStatusNotif:

'{$request.body#/notifUri}':

post:

summary: Notify a previously subscribed service consumer (e.g. VAL Server) on SEALDD connection status event(s).

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ConnStatusNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

TransReq:

description: >

Represents the parameters to request the SEALDD enabled Regular or URLLC application data

transmission service.

type: object

properties:

valServerId:

type: string

valServiceId:

type: string

valTargetUeId:

$ref: 'TS29548_SDD_TransmissionQualityMeasurement.yaml#/components/schemas/ValTargetUeId'

valServerConnInfo:

$ref: '#/components/schemas/ConnInfo'

valServerBdw:

$ref: '#/components/schemas/ValServBdw'

valUsersBdw:

$ref: '#/components/schemas/ValUsersBdw'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- valServerId

- valServerConnInfo

TransResp:

description: >

Represents a SEALDD enabled Regular or URLLC application data transmission service response.

type: object

properties:

ddServerConnInfo:

$ref: '#/components/schemas/ConnInfo'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

ConnInfo:

description: >

Represents SEALDD Data transmission connection information.

type: object

properties:

ipv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

port:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

uri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

oneOf:

- required: [ipv4Addr]

- required: [ipv6Addr]

- required: [uri]

ValServBdw:

description: >

Represents VAL Server related bandwidth information.

type: object

properties:

totalUlBdw:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Bandwidth'

totalDlBdw:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Bandwidth'

required:

- totalUlBdw

- totalDlBdw

ValUsersBdw:

description: >

Represents VAL users related bandwidth information.

type: object

properties:

minUlBdw:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Bandwidth'

minDlBdw:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Bandwidth'

maxUlBdw:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Bandwidth'

maxDlBdw:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Bandwidth'

required:

- minUlBdw

- minDlBdw

- maxUlBdw

- maxDlBdw

ConnStatusSubsc:

description: >

Represents a Connection Status Event Subscription.

type: object

properties:

events:

type: array

items:

$ref: '#/components/schemas/ConnStatusEvent'

minItems: 1

description: >

Represents the list of the subscribed event(s).

valServiceId:

type: string

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- events

- notifUri

ConnStatusNotif:

description: >

Represents a Connection Status Event Notification.

type: object

properties:

event:

$ref: '#/components/schemas/ConnStatusEvent'

valServiceId:

type: string

valTgtUe:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

conEstStat:

$ref: '#/components/schemas/ConnEstabData'

required:

- event

ConnEstabData:

description: >

Represents SEALDD connection status establishment data.

type: object

properties:

comLifetime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

# Simple data types and Enumerations

TransType:

anyOf:

- type: string

enum:

- regular

- urllc

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the requested transmission type.

Possible values are:

- regular: Indicates that the requested transmission type is Regular transmission.

- urllc: Indicates that the requested transmission type is URLLC transmission.

ConnStatusEvent:

anyOf:

- type: string

enum:

- ESTABLISHED

- RELEASED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents Connection Status Events.

Possible values are:

- ESTABLISHED: Indicates that the SEALDD connection is established.

- RELEASED: Indicates that the SEALDD connection is released.

# A.3	SDD_DDContext API

openapi: 3.0.0

info:

title: SEALDD Server Data Delivery Context Relocation

version: 1.0.0-alpha.2

description: |

SEALDD Server Data Delivery Context Relocation.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.548 V0.4.0 Service Enabler Architecture Layer for Verticals (SEAL);

SEAL Data Delivery (SEALDD) Server Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.548/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/sdd-ddc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/contexts:

get:

summary: Pull the DD context from the SEALDD Server.

operationId: PullDdContext

tags:

- DD Context Instances (Collection)

parameters:

- name: supp-feats

in: query

description: Contains the list of supported features.

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Successful case. The requested DD context is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/DdContextResp'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Push the DD context to the SEALDD Server.

tags:

- DD Context Instances (Collection)

requestBody:

description: >

Represents the DD context to be pushed to the

SEALDD Server.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DdContextPushReq'

responses:

'201':

description: >

Created. Successful case. The DD context is successfully pushed to the SEALDD Server and

the related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/DdContextResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

DdContext:

description: Represents the DD context.

type: object

properties:

trLayerContext:

$ref: '#/components/schemas/TranspLayerContext'

TranspLayerContext:

description: Represents the transport layer context.

type: object

properties:

transProtoc:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/TransportProtocol'

required:

- transProtoc

DdContextPushReq:

description: Represents the DD context relocation push request.

type: object

properties:

ddContext:

$ref: '#/components/schemas/DdContext'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

DdContextResp:

description: Represents the DD context relocation push response.

type: object

properties:

endPoint:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RouteInformation'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

# A.4	SDD_DataStorage API

<Service 3> API definition.

# A.5	SDD_TransmissionQualityMeasurement API

openapi: 3.0.0

info:

title: SEALDD Server Data Transmission Quality Measurement Service

version: 1.0.0-alpha.3

description: |

SEALDD Server Data Transmission Quality Measurement Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.548 V0.4.0; Service Enabler Architecture Layer for Verticals (SEAL);

SEAL Data Delivery (SEALDD) Server Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.548/

servers:

- url: '{apiRoot}/sdd-tqm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

post:

summary: Request the creation of a Transmission Quality Measurement Subscription.

operationId: CreateTransQualMeasSubsc

tags:

- Transmission Quality Measurement Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TransQualMeasSubsc'

responses:

'201':

description: >

Created. The Transmission Quality Measurement Subscription is successfully created

and a representation of the created Individual Transmission Quality Measurement

Subscription resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TransQualMeasSubsc'

headers:

Location:

description: >

Contains the URI of the created Individual Transmission Quality Measurement

Subscription resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

TransQualMeasNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TransQualMeasNotif'

responses:

'204':

description: >

No Content. The Transmission Quality Measurement notification is successfully

received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of the Individual Transmission Quality Measurement Subscription

resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Transmission Quality Measurement Subscription resource.

operationId: GetIndTransQualMeasSubsc

tags:

- Individual Transmission Quality Measurement Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual Transmission Quality Measurement Subscription resource

shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/TransQualMeasSubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Transmission Quality Measurement Subscription resource.

operationId: UpdateIndTransQualMeasSubsc

tags:

- Individual Transmission Quality Measurement Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TransQualMeasSubsc'

responses:

'200':

description: >

OK. The Individual Transmission Quality Measurement Subscription resource is

successfully updated and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/TransQualMeasSubsc'

'204':

description: >

No Content. The Individual Transmission Quality Measurement Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Transmission Quality Measurement Subscription resource.

operationId: ModifyIndTransQualMeasSubsc

tags:

- Individual Transmission Quality Measurement Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/TransQualMeasSubscPatch'

responses:

'200':

description: >

OK. The Individual Transmission Quality Measurement Subscription resource is

successfully modified and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/TransQualMeasSubsc'

'204':

description: >

No Content. The Individual Transmission Quality Measurement Subscription resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Transmission Quality Measurement Subscription resource.

operationId: DeleteIndTransQualMeasSubsc

tags:

- Individual Transmission Quality Measurement Subscription (Document)

responses:

'204':

description: >

No Content. The Individual Transmission Quality Measurement Subscription resource is

successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/reports:

get:

summary: Retrieve Historical Transmission Quality Measurement Report(s).

operationId: GetHistTransQualMeasReports

tags:

- Historical Transmission Quality Measurement Reports (Collection)

parameters:

- name: app-traffic-ids

in: query

required: true

schema:

type: array

items:

type: string

minItems: 1

- name: val-group-id

in: query

required: false

schema:

type: string

- name: val-ue-ids-list

in: query

required: false

schema:

type: array

items:

$ref: '#/components/schemas/ValTargetUeId'

minItems: 1

- name: all-val-ues

in: query

required: false

schema:

type: boolean

default: false

- name: supp-feat

in: query

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The requested Historical Transmission Quality Measurement Report(s) shall be

returned.

content:

application/json:

schema:

$ref: '#/components/schemas/HistTransQualMeasReports'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

TransQualMeasSubsc:

description: >

Represents a Transmission Quality Measurement subscription.

type: object

properties:

appTrafficIds:

type: array

items:

type: string

minItems: 1

valGroupId:

type: string

valUeIdsList:

type: array

items:

$ref: '#/components/schemas/ValTargetUeId'

minItems: 1

allValUesInd:

type: boolean

default: false

measConds:

type: array

items:

$ref: 'TS29549_SS_Events.yaml#/components/schemas/ValidityConditions'

minItems: 1

reqs:

type: object

additionalProperties:

$ref: '#/components/schemas/TransQualMeasReq'

minProperties: 1

nullable: true

description: >

Represents the transmission quality measurement reporting requirements of the

subscription.

The key of the map shall be any unique string encoded value.

subsExpTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTimeRo'

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- appTrafficIds

- reqs

- notifUri

oneOf:

- required: [valGroupId]

- required: [valUeIdsList]

- required: [allValUesInd]

TransQualMeasReq:

description: >

Represents Transmission Quality Measurement requirements.

type: object

properties:

measId:

type: array

items:

$ref: '#/components/schemas/MeasurementId'

minItems: 1

repType:

$ref: 'TS29508_Nsmf_EventExposure.yaml#/components/schemas/NotificationMethod'

repPeriodicity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

repGranularity:

$ref: '#/components/schemas/RepGranularity'

measWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

measExpTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- measId

TransQualMeasSubscPatch:

description: >

Represents the requested modifications to a Transmission Quality Measurement subscription.

type: object

properties:

measConds:

type: array

items:

$ref: 'TS29549_SS_Events.yaml#/components/schemas/ValidityConditions'

minItems: 1

reqs:

type: object

additionalProperties:

$ref: '#/components/schemas/TransQualMeasReq'

minProperties: 1

nullable: true

description: >

Represents the transmission quality measurement reporting requirements of the

subscription.

The key of the map shall be any unique string encoded value.

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

TransQualMeasNotif:

description: >

Represents a Transmission Quality Measurement notification.

type: object

properties:

reports:

type: array

items:

$ref: '#/components/schemas/TransQualMeasReport'

minItems: 1

required:

- reports

TransQualMeasReport:

description: >

Represents a Transmission Quality Measurement report.

type: object

properties:

measId:

type: array

items:

$ref: '#/components/schemas/MeasurementId'

minItems: 1

valUeIds:

type: array

items:

$ref: '#/components/schemas/ValTargetUeId'

minItems: 1

required:

- measId

TransQualMeasCriteria:

description: >

Represents a Transmission Quality Measurement report.

type: object

properties:

minLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UintegerRm'

maxLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UintegerRm'

minBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRateRm'

maxBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRateRm'

minPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRateRm'

maxPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRateRm'

minJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32Rm'

maxJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32Rm'

anyOf:

- required: [minLatency]

- required: [maxLatency]

- required: [minBitRate]

- required: [maxBitRate]

- required: [minPackLossRate]

- required: [maxPackLossRate]

- required: [minJitter]

- required: [maxJitter]

TransQualMeasData:

description: >

Represents a Transmission Quality Measurement report.

type: object

properties:

minLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

maxLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avgLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

stdDevLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

kPercLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

avgBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

stdDevBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

kPercBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

minPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

maxPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

avgPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

stdDevPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

kPercPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

minJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

maxJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

avgJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

stdDevJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

kPercJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

anyOf:

- required: [minLatency]

- required: [maxLatency]

- required: [avgLatency]

- required: [minBitRate]

- required: [maxBitRate]

- required: [avgBitRate]

- required: [minPackLossRate]

- required: [maxPackLossRate]

- required: [avgPackLossRate]

- required: [minJitter]

- required: [maxJitter]

- required: [avgJitter]

ValTargetUeId:

description: Represents the identifier of a targeted VAL UE.

type: object

properties:

valUserId:

type: string

valUeId:

type: string

valUeIpv4Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

valUeIpv6Addr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

port:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

oneOf:

- required: [valUserId]

- required: [valUeId]

- required: [valUeIpv4Addr]

- required: [valUeIpv6Addr]

HistTransQualMeasReports:

description: >

Represents Historical Transmission Quality Measurement Report(s).

type: object

properties:

reports:

type: array

items:

$ref: '#/components/schemas/TransQualMeasReport'

minItems: 0

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- reports

MeasurementId:

anyOf:

- type: string

enum:

- LATENCY

- BITRATE

- PACKET_LOSS_RATE

- JITTER

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the transmission quality measurement type.

Possible values are:

- LATENCY: Indicates that the requested/reported measurement is the latency.

- BITRATE: Indicates that the requested/reported measurement is the bit rate.

- PACKET_LOSS_RATE: Indicates that the requested/reported measurement is the packet loss

rate.

- JITTER: Indicates that the requested/reported measurement is the jitter.

RepGranularity:

anyOf:

- type: string

enum:

- INDIVIDUAL_VAL_UE

- VAL_GROUP

- ALL_UES

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the reporting granularity.

Possible values are:

- INDIVIDUAL_VAL_UE: Indicates that the requested granularity is individual VAL UE.

- VAL_GROUP: Indicates that the requested granularity is VAL Group.

- ALL_UES: Indicates that the requested granularity is all UE(s).

# A.6	SDD_PolicyConfiguration API

openapi: 3.0.0

info:

title: SEALDD Server Policy Configuration Service

version: 1.0.0-alpha.1

description: |

SEALDD Server Policy Configuration Service.

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.548 V0.4.0; Service Enabler Architecture Layer for Verticals (SEAL);

SEAL Data Delivery (SEALDD) Server Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.548/

servers:

- url: '{apiRoot}/sdd-pc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/configurations:

post:

summary: Request the creation of a Policy Configuration.

operationId: CreatePolicyConfig

tags:

- Policy Configurations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

responses:

'201':

description: >

Created. The Policy Configuration is successfully created and a representation of

the created Individual Policy Configuration resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

headers:

Location:

description: >

Contains the URI of the created Individual Policy Configuration resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/configurations/{configId}:

parameters:

- name: configId

in: path

description: >

Represents the identifier of the Individual Policy Configuration resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Policy Configuration resource.

operationId: GetIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

responses:

'200':

description: >

OK. The requested Individual Policy Configuration resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Policy Configuration resource.

operationId: UpdateIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

responses:

'200':

description: >

OK. The Individual Policy Configuration resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully updated

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Policy Configuration resource.

operationId: ModifyIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/PolicyConfigPatch'

responses:

'200':

description: >

OK. The Individual Policy Configuration resource is successfully modified and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully modified and

no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Policy Configuration resource.

operationId: DeleteIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

responses:

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

PolicyConfig:

description: >

Represents a SEALDD Policy Configuration.

type: object

properties:

appTrafficIds:

type: array

items:

type: string

minItems: 1

valUeId:

$ref: 'TS29548_SDD_TransmissionQualityMeasurement.yaml#/components/schemas/ValTargetUeId'

sealddPol:

$ref: '#/components/schemas/SealddPolicy'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTimeRo'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- appTrafficIds

- sealddPol

PolicyConfigPatch:

description: >

Represents the requested modifications to a SEALDD Policy Configuration.

type: object

properties:

sealddPol:

$ref: '#/components/schemas/SealddPolicy'

SealddPolicy:

description: >

Represents a SEALDD Policy.

type: object

properties:

qualGuarSets:

type: array

items:

$ref: '#/components/schemas/QualGuarPolicy'

minItems: 1

bdwCtrlSets:

type: array

items:

$ref: '#/components/schemas/BdwCtrlPolicy'

minItems: 1

anyOf:

- required: [qualGuarantee]

- required: [bdwControl]

QualGuarPolicy:

anyOf:

- type: string

enum:

- ESTABLISH_REDUNDANT_TRANS_PATH

- REESTABLISH_TRANS_PATH

- SWITCH_TO_BACKUP_TRANS_PATH

- CHANGE_SEALDD_SERVER

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the quality guarantee policy.

Possible values are:

- ESTABLISH_REDUNDANT_TRANS_PATH: Indicates that the quality guarantee action to be

performed is to establish a redundant transmission path.

- REESTABLISH_TRANS_PATH: Indicates that the quality guarantee action to be performed

reestablish the transmission path.

- SWITCH_TO_BACKUP_TRANS_PATH: Indicates that the quality guarantee action to be performed

is to switch to the backup transmission path.

- CHANGE_SEALDD_SERVER: Indicates that the quality guarantee action to be performed is to

change the connected SEALDD Server.

BdwCtrlPolicy:

anyOf:

- type: string

enum:

- REALLOCATE_DL

- REALLOCATE_UL

- NOT_REALLOCATE_DL

- NOT_REALLOCATE_UL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the bandwidth control policy.

Possible values are:

- REALLOCATE_DL: Indicates that the bandwidth control action to be performed is to

reallocate the bandwidth limit between different VAL users for DL traffic.

- REALLOCATE_UL: Indicates that the bandwidth control action to be performed is to

reallocate the bandwidth limit between different VAL users for UL traffic.

- NOT_REALLOCATE_DL: Indicates that the bandwidth control action to be performed is to

not reallocate the bandwidth limit between different VAL users for DL traffic.

- NOT_REALLOCATE_UL: Indicates that the bandwidth control action to be performed is to

not reallocate the bandwidth limit between different VAL users for UL traffic.

Annex B (informative):
Withdrawn API versions

# B.1	General

This Annex lists withdrawn API versions of the APIs defined in the present specification. 3GPP TS 29.501 [5] clause 4.3.1.6 describes the withdrawal of API versions.

# B.2	SDD_Transmission API

The API versions listed in table B.2-1 are withdrawn for the SDD_Transmission API.

Table B.2-1: Withdrawn API versions of the SDD_Transmission service

| API version number | Remarks |
| --- | --- |

# B.3	SDD_DDContext API

And so on if there are more than two services supported by the NF.

# B.4	SDD_DataStorage API

<Service 3> API.

# B.5	SDD_TransmissionQualityMeasurement API

The API versions listed in table B.5-1 are withdrawn for the SDD_TransmissionQualityMeasurement API.

Table B.5-1: Withdrawn API versions of the SDD_TransmissionQualityMeasurement service

| API version number | Remarks |
| --- | --- |

# B.6	SDD_PolicyConfiguration API

<Service 3> API.

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-03 | CT3#126 | C3-230474 | - | - | - | The skeleton for the new SEADD Server Services TS | 0.0.0 |
| 2023-04 | CT3#127-e | C3-231509 | - | - | - | Inclusion of C3-231212, C3-231568, C3-231214 and C3-231569. | 0.1.0 |
| 2023-06 | CT3#128 | C3-232590 | - | - | - | Inclusion of C3-232513, C3-232524, C3-232525, C3-232526 and C3-232527. | 0.2.0 |
| 2023-09 | CT3#129 | C3-233737 | - | - | - | Inclusion of C3-233655, C3-233656, C3-233657, C3-233658, C3-233659, C3-233660, C3-233715, C3-233716, C3-233717, C3-233718 and C3-233719. | 0.3.0 |
| 2023-10 | CT3#130 | C3-234660 | - | - | - | Inclusion of C3-234127, C3-234128, C3-234129, C3-234499, C3-234500, C3-234666 and C3-234670. | 0.4.0 |