| 3GPP TR 33.749 V0.3.0 (2024-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC) phase 3 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Overview	8

5	Key issues	8

5.1	General	8

5.2	Key issues related with 5G System Enhancements for Edge Computing	8

5.2.1	Key Issue #1.1: Security aspects related to enhancements of EAS and local UPF (re)selection.	8

5.2.1.1	Key issue details	8

5.2.1.2	Security threats	9

5.2.1.3	Potential security requirements	9

5.2.X	Key Issue #X: <Key Issue Name>	9

5.2.X.1	Key issue details	9

5.2.X.2	Threats	9

5.2.X.3	Potential security requirements	9

5.3	Key issues related with enhanced architecture for enabling Edge Applications	9

5.3.1	Key Issue #2.1: Secure retrieval of 5G system UE Ids and privacy related information.	9

5.3.1.1	Key issue details	9

5.3.1.2	Security threats	10

5.3.1.2.1	Threats posed by a malicious EEC	10

5.3.1.2.2	Threats posed by malicious EAS/EES outside of the operator domain	10

5.3.1.3	Potential security requirements	10

5.3.1.3.1	Verification of the user information provided by the EEC	10

5.3.1.3.2	Protection of the UE privacy	10

5.3.X	Key Issue #X: <Key Issue Name>	10

5.3.X.1	Key issue details	10

5.3.X.2	Security threats	10

5.3.X.3	Potential security requirements	10

6	Solutions	10

6.1	Solution #1: Usage of existing public IP address to verify EEC provided IP address	11

6.1.1	Solution overview	11

6.1.2	Solution details	12

6.1.3	Solution evaluation	12

6.2	Solution #2: Authorizing AF to retrieve UE ID	13

6.2.1	Solution overview	13

6.2.2	Solution details	13

6.2.3	Evaluation	14

6.3	Solution #3: Secure retrieval of 5G UE Id and privacy related information	14

6.3.1	Introduction	14

6.3.2	Solution details	14

6.3.3	Evaluation	16

6.4	Solution #4: Secure retrieval of 5G UE privacy related information based on AKMA	16

6.4.1	Introduction	16

6.4.2	Solution details	16

6.4.3	Evaluation	17

6.5	Solution #5: EEC proviced information verification	17

6.5.1	Introduction	17

6.5.2	Details	17

6.5.3	Evaluation	18

6.6	Solution #6: UE ID token	18

6.6.1	Solution overview	18

6.6.2	Solution details	18

6.6.3	Solution evaluation	19

6.7	Solution #7: Verification of EEC provided IP address	19

6.7.1	Solution overview	19

6.7.2	Solution details	20

6.7.3	Solution evaluation	20

6.8	Solution #8: Verification of EEC provided IP address using access token	21

6.8.1	Solution overview	21

6.8.2	Solution details	21

6.8.3	Solution evaluation	22

6.Y	Solution #Y: <Solution Name>	22

6.Y.1	Introduction	22

6.Y.2	Solution details	22

6.Y.3	Evaluation	22

7	Conclusions	22

Annex <X> (informative): Change history	23

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document studies the security enhancements on the support for Edge Computing in the 5G Core network defined in TR 23.700-49 [2], and enhanced architecture for enabling Edge Applications defined in TS 23.558 [3]. Specifically, the present document focuses on the following:

- Study the security aspects on the enhancements for EAS (re)discovery and UPF (re)selection with reducing impact on central 5GC NFs, enhancement of EAS and local UPF (re)selection,  and EC Traffic Routing between local part of DN and central part of DN the Edge Hosting Environment information management.

- Study the security on the enhancements to Edge Enabler layer (EEL) to support additional scenarios for edge services.

- Study the authorization between EESes for both Application Context Relocation (ACR) and Edge Node Sharing (ENS) scenarios.

- Study the secure retrieval of 5G system UE Ids and privacy related information in the EDGE.

The study is based on the work done in the 3GPP TS 33.558 [4], 3GPP TR 33.839 [5], 3GPP TR 33.739 [6].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-49: "Study on Enhancement of support for Edge Computing in 5G Core network - Phase 3".

[3]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[4]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications; Stage 2".

[5]	3GPP TR 33.839: "Study on security aspects of enhancement of support for edge computing in the 5G Core ".

[6]	3GPP TR 33.739: "Study on security enhancement of support for edge computing phase 2".

[7]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

[8]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[9]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Overview

The present document studies the security enhancements on the support for Edge Computing in the 5G Core network defined in 3GPP TS 23.548 [7], and application architecture for enabling Edge Applications defined in 3GPP TS 23.558 [3]. For the architecture and procedure of EC supported in 5GC, refer to 3GPP TS 23.548 [7]. For more details on enabling Edge Applications, it is proposed to refer to 3GPP TS 23.558 [3].

5	Key issues

Editor’s Note: This clause contains all the key issues identified during the study.

5.1	General

Clause 5 describes the security key issues related with 5G System Enhancements for Edge Computing of 3GPP TR 23.700-49 [2] in clause 5.2, and Enhanced Architecture for Enabling Edge Applications of 3GPP TS 23.558 [3] in clause 5.3.

5.2	Key issues related with 5G System Enhancements for Edge Computing

5.2.1	Key Issue #1.1: Security aspects related to enhancements of EAS and local UPF (re)selection.

5.2.1.1	Key issue details

Several enhancements in the procedures for EAS and local UPF (re)selection have been proposed in TR 23.700-49 [2]. It has been concluded that N6 delay measurement is a factor of decision in those procedures. The N6 delay measurement between the termination points, i.e., UPF and EAS, is to be performed using existing IETF standard protocols (e.g., ICMP, TWAMP, OWAMP, etc.). The 5G network and the EDGE negotiate and exchange information about the termination points, protocols and parameters to be used, and then the SMF (“controller”) configures the UPF(s) to conduct the measurements as “senders” to the network devices/compute nodes/EAS(s) acting as “reflectors/receiver”. The UPF(s) will report the results to SMF.

The IETF RFCs corresponding to the IP Performanace Metrics (IPPM) defined protocols, such as RFC 4656, RFC 5357, RFC 8762, etc., include security considerations to be taken into account when those protocols are in use. RFC 2681 [yy] refers to two types of security concerns in these metrics: the potential harm caused by measurements, and the potential harm to the measurements.

The security risks associated with each protocol need to be tackled in the context of EAS and local UPF (re)selection, assuming that EDGE servers, such as the EAS, are placed outside the operator domain.

5.2.1.2	Security threats

Denial of Service: Potential attackers can exploit the capabilities of protocols like ICMP to overwhelm targeted servers like EAS (e.g., ping flood, smurf attacks, etc.).

Spoofing and Theft of service: Without authentication between, the termination points, the client may fake the source address of other device and use the measurement services produced by the server/reflector/receiver.

Tampering: The measurement traffic can be compromised by for example a man-in-the-middle entity, which can inject fake measurement traffic, or give to that a different priority, etc. Consequently, the selection of the server (UPF or EAS) could be biased by the attacker to their interest, leading in the worst-case scenario to the selection of a UPF or EAS already compromised and/or under their control.

5.2.1.3	Potential security requirements

Editor’s Note: The security mechanisms of the protocols to be used in N6 delay measurement are ffs.

5.2.X	Key Issue #X: <Key Issue Name>

5.2.X.1	Key issue details

5.2.X.2	Threats

5.2.X.3	Potential security requirements

5.3	Key issues related with enhanced architecture for enabling Edge Applications

5.3.1	Key Issue #2.1: Secure retrieval of 5G system UE Ids and privacy related information.

5.3.1.1	Key issue details

This Key issue addresses the security and privacy aspects related to the retrieval of 5G system UE Ids and privacy related information (e.g., UE location) by an Edge Application Server (EAS), Edge Enabler Server (EES) and/or Edge Enabler Client (EEC).

Clause 8.6.5 of 3GPP TS 23.558 [3] defines UE identifier API which is used by an EAS or EEC to obtain the identifier of the UE if the EAS or EEC does not have it (e.g. it has not already cached). This identifier, called UE ID (could be the GPSI or the EEL-generated Edge UE ID, defined in clause 7.2.6 of 3GPP TS 23.558 [3]), is used by the EAS to invoke capability APIs specific to UEs over EDGE-3 and/or EDGE-7 depending on the UE ID type.

The EES uses user information (e.g. IP address) received in the UE Identifier API invocation and obtains the UE identifier by interacting with NEF as specified in clause 4.15.10 of 3GPP TS 23.502 [8]. The EES may utilize the Nnef_UEId_Get service (clause 4.15.10 of 3GPP TS 23.502 [8]) providing the user information provided by the EEC. Without proper security mechanisms in place, Nnef_UEId API services can be abused, so that UE Id may be disclosed to un-authorized entities, enabling them for example to track UEs.

Since user information may be used to determine the 5G system UE Id and consequently privacy related information (e.g., identity, location, etc.), it is needed to ensure that this user information being used in the APIs (UE Identifier API and Nnef_UEId ) is trusted, and that the AFs (EAS, EES, ECS) and EEC are authorized to use this user information as parameter(s) in their API invocations.

Following the security principle of sharing information on a need-to-know principle, it should be analysed whether and how (i.e. under which circumstances) EAS needs to know the 5G UE Id when requesting a service on the UE’s behalf.

The related security and privacy aspects in the use of the UE Identifier API and Nnef_UEId API concern the information provided by the EEC, as well as the behaviour of EDGE Application Functions (AFs), namely EAS and EES. The security threats and corresponding requirements have been split to cover both aspects.

5.3.1.2	Security threats

5.3.1.2.1	Threats posed by a malicious EEC

If the User information provided by the EEC is not verified and the EEC is not authorized to use them, a malicious or compromised EEC or a malicious API consumer can try to execute spoofing attacks to learn identifiers of other UEs.

5.3.1.2.2	Threats posed by malicious EAS/EES outside of the operator domain

If the User information provided by the EEC do not ensure the privacy of the UE, EAS/EES can abuse the UE Identifier API and/or Nnef_UEId APIs to break UE privacy (e.g., UE identity, location, etc.).

Editor’s note: Whether the abuse of the UE Identifier API and/or Nnef_UEId APIs can be used to know the network topology is ffs.

5.3.1.3	Potential security requirements

5.3.1.3.1	Verification of the user information provided by the EEC

5G system should support a mechanism to verify the user information provided by EEC.

5.3.1.3.2	Protection of the UE privacy

5G system should verify that the EDGE application functions are authorized to retrieve the 5G system UE Id  and that the procedures of calling UE identifier API and Nnef_UEId API do not compromise the privacy related information.

5.3.X	Key Issue #X: <Key Issue Name>

5.3.X.1	Key issue details

5.3.X.2	Security threats

5.3.X.3	Potential security requirements

6	Solutions

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |
| --- | --- | --- |
|  | KI#1.1 | KI #2.1 |
| Solution #1: Usage of existing public IP address to verify EEC provided IP address |  | X |
| Solution #2: Authorizing AF to retrieve UE ID |  | X |
| Solution #3: Secure retrieval of 5G UE Id and privacy related information |  | X |
| Solution #4: Secure retrieval of 5G UE privacy related information based on AKMA |  | X |
| Solution #5: EEC proviced information verification |  | X |
| Solution #6: UE ID token |  | X |
| Solution #7: Verification of EEC provided IP address |  | X |
| Solution #8: Verification of EEC provided IP address using access token |  | X |

6.1	Solution #1: Usage of existing public IP address to verify EEC provided IP address

6.1.1	Solution overview

This solution is for the key issue # 2.1 on secure retrieval of 5G system UE Ids and privacy related information.

In NAT case, to differentiate different UEs using the same public IP address, the UPF will allocate a unique port number for each UE and the UPF stores the mapping between private IP address, public IP address and port number, the IP information knowed in each node in NAT case is showed in the following figure 6.1.1-1.

Figure 6.1.1-1: NAT procedure

Therefore, to verify the EEC provided IP address, this solution proposes to use the mapping between private IP address, public IP address and port number in the UPF to verify the mapping EEC provided IP address, EES obtained public IP address and port number based on the source IP address and port number of received UE ID API message.This solution can be applied in the case where NAT is embedded in the UPF and no matter which NAT mode is used by UPF.

Editor’s notes: it is FFS for the potential risk of exposing the private and public IP addresses of EEC to an entity as EES.

6.1.2	Solution details

The procedure of the solution is presented in figure 6.1.2-1 and steps are explained in detail below.

Figure 6.1.2-1 EEC provided IP address verification via existing public IP address and port number

Step 0.	If NAT is used in the UPF, the UPF stores the mapping between private IP address, public IP address, port number.

Step 1.	The EEC in the UE sends UE Identifier API request with the private IP address#1 to the EES as clause 8.6.5 of 3GPP TS 23.558 [3].

Step 2.	EES obtains the UE public IP address#2, port number based on the sourse IP address and source port number of the IP data from UPF which include the UE Identifier API request.

Step 3.	EES requests to retrieve UE ID via the Nnef_UEId_Get service operation. The request message includes private IP address #1, public IP address#2, port number associated with the public IP address#2.

Steps 4-5.	NEF obtains the private IP address#3 from UPF via existing procedure specified in clause 4.15.10 (from step 3 to step 6) of 3GPP TS 23.502 [8].

Step 6.	NEF verify if the private IP address#3 is equal to EEC provided IP address#1, if verification is successful, the NEF continues to obtain the AF specific GPSI via step 7 and step 8.

Steps 7-8.	NEF obtains AF specific GPSI via existing procedure specified in clause 4.15.10 (from step 7 to step 10) of 3GPP TS 23.502 [8].

Steps 9-10.	NEF sends Nnef_UEId_Get response with AF specific GPSI to EES, and the EES send UE Identifier API request with AF specific GPSI to EEC.

6.1.3	Solution evaluation

This solution addresses the requirements of KI#2.1 on the user information verification.

The EEC provided IP address, public IP address and port number are all provided to NEF, then the NEF retrieves the private IP address corresponding to the public IP address and port number via the exising procedure and performs the EEC provided IP address verification. There is no additional impact to other network functions.

The verification may not be required when the UPF has the mapping table between public IP address and private IP address.

The solution assumes that NAT is embedded in the UPF, and the NAT mapping table can be exposed to EDGE application.

The solution is limited to the verification of the provided private IP address by the EEC to the EES, and used by the EES to invoke NEF API.

6.2	Solution #2: Authorizing AF to retrieve UE ID

6.2.1	Solution overview

This solution is for KI#1 on the secure retrieval of 5G system UE IDs. Since the AF may request for the UE ID without UE authorization, the solution proposes to use fresh hash value with count, which is a changeable value shared between the UE and the 5GC, to authorize the AF to retrieve the UE ID. It can also prevent replay attacks from the malicious entity (e.g., malicious UE or AF).

6.2.2	Solution details

This solution supports the 5GC to verify whether the AF is authorized by the UE to retrieve the UE ID based on the IP address, and verify whether the parameters used for verification is replayed by the adversary. Specifically, when the AF needs to retrieve the UE ID, it first applies for a fresh hash value from the UE. After that, the UE generates a hash value based on the count, SUPI and IP address, and then sends the generated hash value to the AF, which uses the hash value to request for the UE ID from the NEF. The NEF then requests BSF to provide information of the UE (i.e., SUPI). Before providing the SUPI of UE, the BSF shall also generate a hash' based on the shared count, SUPI and IP address, and then compare the hash' value with the received hash value. If the hash value and hash' value is the same, the BSF can provide the SUPI. Otherwise, the BSF will provide a failure indication to the NEF.

Figure 6.2.2-1: Authorization of AF and verification of AF provided IP address

Step 1.	When the AF needs to retrieve the UE ID, it sends a UEID hash request to the UE.

Step 2.	The UE generates a hash value based on the SUPI, IP address, and count. After that, the UE increases the count by 1, which is used for the next hash value generation procedure. Note that the count can be configured to start from 0 in each PDU session. The hash value is calculated based on value that is known to both the EEC and BSF.  Example values that can be used as input ot the hash calculation include the UE’s SUPI, public IP Address, private IP Address and port number, DNN of the PDU Session, and S-NSSAI of the PDU Session.

Editor’s Note: How to avoid the brute force type of attack for the hash value is FFS.

Editor’s Note: How to synchronize the counter between the UE and network is FFS.

NOTE: A malicious attacker could pre-calculate all potential hash values and thus from the hash value derive the UE’s SUPI (while it should only learn the AF specific UE identifier via the NEF)Step 3.	The UE sends the generated hash value to the AF.

Step 4.	The AF invokes the Nnef_UEId_Get request with the parameter of hash value and IP address to get the UE ID.

Steps 5-8.	This solution reuses Steps 3-6 in Clause 4.15.10, TS 23.502 [8].

Step 9.	When the NEF invokes the Nbsf_Management_Discovery service operation, the NEF provides the hash value and IP address to the BSF.

Step 10.	The BSF determines the SUPI and count based on the IP address. The BSF then generates the hash' value, and then compares the hash' value with the received hash value. The BSF will then increase the count by 1.

Step 11.	If the hash' value is equal to the received hash value, the BSF will respond to the NEF with a SUPI. Otherwise, the BSF will respond to the NEF with an indication that request is denied with the reason that the IP address is not associated with the UE.

Steps 12-14.	This solution reuse Steps 9-11 in Clause 4.15.10, TS 23.502 [8].

6.2.3	Evaluation

TBD

6.3	Solution #3: Secure retrieval of 5G UE Id and privacy related information

6.3.1	Introduction

This solution addresses the security requirements exposed in key issue #1 related to the secure retrieval of 5G system UE Id and privacy related information, i.e., the verification of the user information provided by the EEC, and the protection of the UE privacy.

The main design principle of the solution is based on the utilization of a temporary identifier (ID) as UE identity to fetch the Application Function (AF) specific UE Id from Nnef_UEId service. The temporary ID is generated by the 5G Core (e.g., UDM), it includes a randomly generated value, and it is associated to the PDU session (eventually to the AF ID).

6.3.2	Solution details

Figure 6.Y.2-1 illustrates the procedure to fetch the 5G system UE Id and privacy related information.

Figure 6.Y.2-1: Procedure to fetch the 5G system UE Id and privacy related information

1. PDU session establishment / modification request from the UE to the 5G system.

2. The 5G Core (e.g., UDM) generates (per UE) an internal temporary ID (with a ramdomly generated part) for PDU session.

NOTE: The UE could request multiple temporary IDs during the same PDU session (PDU Session modification). The UE can use this to invalidate previous IDs which might have been compromised and/or at fix intervals.

Editor’s Note: the details on which NF generates the temporary ID is ffs.

Editor’s Note: the details of the temporary ID are ffs.

3. The 5G Core returns the internal temporary ID to the UE as part of the NAS PDU session establishment/modification response.

Editor’s Note: how the EEC is authorized to get the assigned temp ID is ffs.

4. The UE (EEC) uses the allocated temporary ID inside the Edge application signaling in the communication with the application function AF (e.g., EAS).

5. The AF uses the received temporary ID to invoke the Nnef_UEId API intended to fetch the 5G system UE identifier, i.e., AF specific UE Id, as per the procedure described in TS 23.502 figure 4.15.10-1 [8].

6. The NEF verifies the temporary ID, authorizes the request coming from the AF and accordingly retrieves from the 5G Core the AF specific UE Id corresponding to the temporary ID.

Editor’s Note: how the NEF fetches the temporary ID from the NF which generates it to make the verification is ffs.

7. The NEF replies to the AF with the corresponding AF specific UE Id.

8-9 (Optional). Assuming the application logic requires to fetch the UE location, a location request is sent to the NEF using the AF specific UE Id, and the corresponding information is provided by the 5G system through the NEF.

10. The AF responds to the UE (EEC).

6.3.3	Evaluation

The solution requires the generation of a new temporary ID identifier by the 5G Core, that needs to be used in the invocation of Nnef_UEId API services.

The solution avoids the mapping of potentially long-term and easily inferred identifiers, with the 5GS UE Id, to block malicious applications from performing UE tracking type of attacks.

Editor’s Note: The evaluation should state the applicability of the solution, i.e. what is the solution solving and what are the assumptions.

Editor’s Note: Further evaluation is to be defined.

6.4	Solution #4: Secure retrieval of 5G UE privacy related information based on AKMA

6.4.1	Introduction

This solution addresses the security requirements exposed in key issue #1 related to the secure retrieval of 5G system UE privacy related information, i.e., the verification of the user information provided by the EEC, and the protection of the UE privacy.

The main design principle of the solution is based on AKMA feature, more specifically on the A-KID, an application specific ID derived from the long term symmetric pre-shared key K between the UE and the network. The EDGE application server (EAS) and/or enabler server (EES), use the AKMA protocol to retrieve the 5G UE privacy related information.

The solution assumes that AF (EES/EAS) communicates with 5GC through NEF.

6.4.2	Solution details

Figure 6.4.2-1 illustrates the procedure to fetch the 5G system UE privacy related information.

Figure 6.4.2-1: Procedure to fetch the 5G system UE Id and privacy related information

Preliminary steps:

- Primary authentication

- AKMA Key derivation (A-KID) as per procedure in TS 33.535 [9].

- Creation of AKMA context in AAnF (SUPI, KAKMA, A-KID) as per procedure in [9].

1. The application client in the UE (EEC) requests a service to the AF, i.e., EDGE application server (EAS, EES), including the A-KID (AKMA Key Identifier) in the request.

2-5. AKMA procedure intended to provide KAF to the AF from the AAnF via NEF as specified in clause 6.3 of [9].

NOTE: AF specific UE Id does not need to be part of the of the incoming request (step 2), since the NEF will provide the GPSI (external ID) to AF.

6. The AF requires to know privacy information of the UE available in the 5G Core, such as the the location of UE, and makes the request to the 5G Core via NEF, using the given GPSI as UE identifier.

7. The NEF checks the GPSI and if the request is authorized, the corresponding information, in this example location of the UE, is provided to the AF.

8. The AF responds to the UE (EEC).

6.4.3	Evaluation

The main impact of the solution is that it requires the support of AKMA feature in the 5G system.

The 5G UE Id is not required to be fetched by the AF, since 5G Core through AKMA feature provides already the authentication service, and the trust between UE, 5G Core and AF.

Editor’s Note: The evaluation should state the applicability of the solution, i.e. what is the solution solving and what are the assumptions.

Editor’t Note: Further evaluation is to be defined.

6.5	Solution #5: EEC proviced information verification

6.5.1	Introduction

This solution addresses key issue #1: "5G system should support a mechanism to verify the user information provided by EEC".

Clause 8.6.5 of 3GPP TS 23.558 [3] defines UE identifier API which is used by an EAS or EEC to obtain the identifier of the UE if the EAS or EEC does not have it (e.g. it has not already cached).

When EEC invokes the API, if available, this IE contains both UE’s private IPv6 address (due to the existence of NAT66) and UE’s private IPv4 address. According to TS 33.558, EEC authentication is not mandatory, there might be cases that EEC is not authenticated. When EEC is not authenticated, there is risk that the information provided by EEC is not owned by this EEC, leading to the result that the UE indentifier API is abused and other UE information is leaked.

This solution proposed to add the varification method when EEC authentication is not performed.

6.5.2	Details

Preassumption: the authenticated EEC will not provide untrusted user information.

Following the procedure in TS 23.558 on EEC requesting UE identifier, it shall provide the security credentials. It is proposed in this solution, that the security credential in the UE Identifier API request can be a certificate to EES to verify, when the EEC doesn’t support any other authentication methods defined in TS 33.558.

Step 1. EEC sends the UE Identifier API request, including the User Information (i.e.UE address), EEC signs the User Information using its private key. EEC also includes the Cert information as the security credential field as defined in TS 23.558[3].

Step 2. EES verifies the certificate. If the verification is successful, EES verifies the signature of the User information using the public key.

Step 3-6. Following the TS 23.558 clause 8.6.5.2 step 2-4, EES retrieves UE ID fro NEF and sends it back to EEC.

Editor’s Note: how to address the certificate management issue is FFS.

NOTE: the trustworthiness of EEC provided information is relying on PKI.

6.5.3	Evaluation

TBD

Editor’s note: The evaluation should state the applicability of the solution, i.e. what is the solution solving and what are the assumptions.

6.6	Solution #6: UE ID token

6.6.1	Solution overview

This solution proposes usage of UE ID token to address the security requirements key issue #2.1. With the usage of UE ID token, there will be no need to share private IP address with EEC and other entities in the EC architecture.

6.6.2	Solution details

High-level overview of the solution is presented in figure 6.6.2-1 and steps are explained in detail below.

Figure 6.6.2-1: UE ID token based solution

Step 1:	The EEC obtains a UE ID token from the UE ID server which is located inside the operator domain. UE ID server can identify the UE by using the private IP address of the PDU session.

Editor’s note: Further details, such as how the EEC authenticates with the UE ID server of the operator, are FFS.

Step 2:	The EEC sends the token to the EES to learn edge specific UE ID.

Step 3:	The EES invokes the NEF IP by sending the UE ID token.

Step 4:	The NEF interacts with the UE ID server to identify the UE.

Step 5:	The NEF sends the UE ID to the EES.

Step 6:	The EES sends the response to the EEC.

Editor’s note: Clarification on which architecture the solution is based on is FFS.

6.6.3	Solution evaluation

Editor’s note: Evaluation is FFS.

Editor’s note: The evaluation should state the applicability of the solution, i.e. what is the solution solving and what are the assumptions.

6.7	Solution #7: Verification of EEC provided IP address

6.7.1	Solution overview

This solution addresses the security requirement of key issue#2.1 on EEC provided IP address verification.

This solution proposes to use a PDU session ID for verifying the authenticity of assigned IP address.

6.7.2	Solution details

Figure 6.7.2-1: EEC provided IP address verification

Step 1: The UE/EEC invokes UE Identifier API exposed by the EES. The request includes the CN assigned private IP address of the UE and the PDU session ID.

Step 2: On receiving the UE Identifier API request, the EES invokes Nnef_UEId_Get service operation for translating the UE's Private IP address to its UE ID. EES includes the received IP Address and PDU Session ID in the request message.

NOTE:	Solution assumes with PDU session ID the NEF fetches the supi to be sent in step 3.

Step 3: The NEF invokes the Nudm_SDM_Get request and sends the corresponding SUPI in the request.

Step 4: In Nudm_SDM_Get response the UDM sends the AF specific UE identifier and the PDU session ID(s) of the SUPI.

Step 5: The NEF verifies whether the received PDU Session ID from EES is present in the list of PDU session ID(s) from the UDM.

Step 6-7: If the verification is successful, then the NEF provides the AF specific UE Identifier to the EES in Nnef_UEId_Ger response. Then the EES forwards the AF specific UE Identifier to the UE/EEC.

Editor’s Note: Whether there is a privacy on usage of PDU session ID and whether usage of PDU session ID as secret information is enough to prevent spoofing attacks are FFS.

6.7.3	Solution evaluation

Editor’s note: The evaluation should state the applicability of the solution, i.e. what is the solution solving and what are the assumptions.

6.8	Solution #8: Verification of EEC provided IP address using access token

6.8.1	Solution overview

This solution addresses the security requirement of key issue#2.1 on EEC provided IP address verification.

This solution proposes to use an access token for mapping with the assigned IP address. This access token is provided by the ECS as specified in 33.558 [4].

6.8.2	Solution details

Figure 6.8.2-1: EEC provided IP address verification

Step 1: The UE/EEC invokes UE Identifier API exposed by the EES. The request includes the CN assigned private IP address of the UE and the access token.

Step 2: On receiving the UE Identifier API request, the EES verifies the access token and then invokes Nnef_UEId_Get service operation for translating the UE's Private IP address to its UE ID. EES includes the received IP Address in the request message.

Step 3: The NEF invokes the Nudm_SDM_Get request and sends the corresponding SUPI in the request.

Step 4-5: In Nudm_SDM_Get response the UDM sends the AF specific UE identifier and UE IDs of the received SUPI. The NEF provides the AF specific UE Identifier and UE IDs to the EES in Nnef_UEId_Get response.

Step 6: The EES then verifies whether the UE ID received in the access token from EEC and from the NEF are the same.

Step 7: If the verification is successful, then the EES provides the AF specific UE Identifier to the EEC in UE Identifier API response.

Editor’s Note: How NEF know the AF ID to be able to provide the AF specific ID is FFS.

Editor’s Note: Whether the procedure is necessary if the EEC has the UE ID in the token is FFS.

6.8.3	Solution evaluation

Editor’s note: The evaluation should state the applicability of the solution, i.e. what is the solution solving and what are the assumptions.

6.Y	Solution #Y: <Solution Name>

6.Y.1	Introduction

Editor’s Note: Each solution should list the key issues being addressed.

6.Y.2	Solution details

6.Y.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

7	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-04 | SA3#115 Adhoc-e | S3-241217 |  |  |  | Skeleton of TR33.749 | 0.0.0 |
| 2024-04 | SA3#115 Adhoc-e | S3-241569 |  |  |  | Included changes from S3-241564 and S3-241216 | 0.1.0 |
| 2024-05 | SA3#116 | S3-242604 |  |  |  | Included changes from S3-242567 and S3-242661 | 0.2.0 |
| 2024-08 | SA3#117 | S3-243721 |  |  |  | Included changes from S3-243627, S3-243628, S3-243629, S3-243630, S3-243631, S3-243632, S3-243633, S3-243634, S3-243635 and S3-243665 | 0.3.0 |