| 3GPP TS 36.133 V17.6.0 (2022-06) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Error! Cannot open file referenced on page 1

Foreword	92

1	Scope	94

2	References	94

3	Definitions, symbols and abbreviations	96

3.1	Definitions	96

3.2	Symbols	98

3.3	Abbreviations	99

3.4	Test tolerances	102

3.5	Additional notation	102

3.5.1	Groups of bands	102

3.6	General	104

3.6.1	Applicability of requirements in this specification version	104

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	110

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	112

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	113

3.6.2	Applicability of requirements for EN-DC operation	114

3.6.3	Applicability of requirements for NE-DC operation	115

3.6.4	Applicability of requirements for NGEN-DC operation	116

4	E-UTRAN RRC_IDLE state mobility	116

4.1	Cell Selection	116

4.2	Cell Re-selection	116

4.2.1	Introduction	116

4.2.2	Requirements	117

4.2.2.1	Measurement and evaluation of serving cell	118

4.2.2.2	Void	119

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	119

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	121

4.2.2.5	Measurements of inter-RAT cells	123

4.2.2.5.1	Measurements of UTRAN FDD cells	123

4.2.2.5.2	Measurements of UTRAN TDD cells	125

4.2.2.5.3	Measurements of GSM cells	126

4.2.2.5.4	Measurements of HRPD cells	127

4.2.2.5.5	Measurements of cdma2000 1X	128

4.2.2.5.6	Measurements of NR cells	129

4.2.2.5.7	Measurements of NR cells subject to CCA	131

4.2.2.6	Evaluation of cell re-selection criteria	133

4.2.2.7	Maximum interruption in paging reception	133

4.2.2.8	void	134

4.2.2.9	UE measurement capability	134

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	134

4.2.2.10	Reselection to CSG cells	135

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	135

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	136

4.2.2.11	Void	136

4.2.2.12	Void	136

4.2.2.13	Void	136

4.3	Minimization of Drive Tests (MDT)	136

4.3.1	Introduction	137

4.3.2	Measurements	137

4.3.2.1	Requirements	137

4.3.3	Relative Time Stamp Accuracy	137

4.3.3.1	Requirements	137

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	137

4.3.4.1	 Requirements	137

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	138

4.3.5.1	Requirements for timeSinceFailure	138

4.4	MBSFN Measurements	138

4.4.1	Introduction	138

4.4.2	MBSFN RSRP measurements	138

4.4.3	MBSFN RSRQ measurements	138

4.4.4	MCH BLER measurements	139

4.5	Proximity-based Services	139

4.5.1	Introduction	139

4.5.2	Requirements	139

4.5.2.1	Interruptions with ProSe Direct Discovery	139

4.5.2.2	Interruptions with ProSe Direct Communication	139

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	139

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	140

4.6	Cell Selection and Re-selection Requirements for UE category NB1	140

4.6.1	Cell Selection	140

4.6.2	Cell Re-selection	140

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	141

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	142

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	143

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	144

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	146

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	146

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	148

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	149

4.6.2.7	Maximum interruption in paging reception in normal coverage	151

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	151

4.6.2.8	UE measurement capability	151

4.6.2.9	WUS receptions for NB1	151

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	152

4.6.3.1	Introduction	152

4.6.3.2	Requirements on UE synchronization for transmission using PUR	152

4.6.3.3	Requirements on TA validation for transmission using PUR	152

4.7	Cell Selection and Re-selection Requirements for UE category M1	153

4.7.1	Cell Selection	153

4.7.2	Cell Re-selection	153

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	153

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	153

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	154

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	155

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	157

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	158

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	159

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	159

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	159

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	160

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	161

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	164

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	165

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	166

4.7.2.3	WUS receptions for UE category M1	166

4.7.3	Channel quality report for UE Category M1 in idle mode	166

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	167

4.7.4.1	Introduction	167

4.7.4.2	Requirements on UE synchronization for transmission using PUR	167

4.7.4.3	Requirements on TA validation for transmission using PUR	167

4.8	Idle State Positioning Measurement Requirements for UE category NB1	168

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	168

4.8.1.1	RSTD Measurement Reporting Delay	170

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	170

4.8.2.1	RSTD Measurement Reporting Delay	172

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	172

4.8.3.1	RSTD Measurement Reporting Delay	173

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	174

4.8.4.1	RSTD Measurement Reporting Delay	175

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	176

4.8.5.1	Measurement Reporting Delay	177

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	177

4.8.6.1	Measurement Reporting Delay	178

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	179

4.8.7.1	Measurement Reporting Delay	180

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	180

4.8.8.1	Measurement Reporting Delay	181

4.9	Idle Mode CA Measurement	182

4.9.1	Introduction	182

4.9.2	Requirement	182

4.9.2.1	Detected cell requirement during state transition and Idle mode	182

4.9.2.2	Measurements of inter-frequency CA candidate cells	183

4.9.2.3	Measurements on serving cell	183

4A	E-UTRAN RRC_INACTIVE state mobility	184

4A.1	Cell Re-selection	184

4A.1.1	Introduction	185

4A.1.2	Requirements	185

4A.1.2.1	UE measurement capability	185

4A.1.2.2	Measurement and evaluation of serving cell	185

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	185

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	185

4A.1.2.5	Evaluation of cell re-selection criteria	185

4A.1.2.6	Maximum interruption in paging reception	185

4A.1.2.7	Measurements of inter-RAT NR cells	185

4A.2	Requirements for UE Category M1	185

4A.2.1	Introduction	185

4A.2.2	Cell Selection	185

4A.2.3	Cell Reselection	185

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	185

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	185

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	186

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	186

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	187

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	187

4A.2.4	Channel quality report for UE Category M1 in idle mode	188

5	E-UTRAN RRC_CONNECTED state mobility	188

5.1	E-UTRAN Handover	189

5.1.1	Introduction	189

5.1.2	Requirements	189

5.1.2.1	E-UTRAN FDD – FDD	189

5.1.2.1.1	Handover delay	189

5.1.2.1.2	Interruption time	189

5.1.2.2	E-UTRAN FDD – TDD	191

5.1.2.2.1	(Void)	191

5.1.2.2.2	(Void)	191

5.1.2.3	E-UTRAN TDD – FDD	191

5.1.2.3.1	(Void)	191

5.1.2.3.2	(Void)	191

5.1.2.4	E-UTRAN TDD – TDD	191

5.1.2.4.1	Handover delay	191

5.1.2.4.2	Interruption time	191

5.1.2.5	E-UTRAN HD–FDD	193

5.1.2.5.1	Handover delay	193

5.1.2.5.2	Interruption time	193

5.1.2.6	E-UTRAN FDD – FDD conditional handover	194

5.1.2.6.1	Handover delay	194

5.1.2.6.2	Measurement time	195

5.1.2.6.3	Preparation time	195

5.1.2.6.4	Interruption time	195

5.1.2.7	E-UTRAN FDD – TDD conditional handover	196

5.1.2.8	E-UTRAN TDD – FDD conditional handover	196

5.1.2.9	E-UTRAN TDD – TDD conditional handover	196

5.2	Void	196

5.3	Handover to other RATs	196

5.3.1	E-UTRAN - UTRAN FDD Handover	196

5.3.1.1	Introduction	196

5.3.1.1.1	Handover delay	196

5.3.1.1.2	Interruption time	196

5.3.2	E-UTRAN - UTRAN TDD Handover	197

5.3.2.1	Introduction	197

5.3.2.2	Requirements	197

5.3.2.2.1	Handover delay	197

5.3.2.2.2	Interruption time	197

5.3.3	E-UTRAN - GSM Handover	198

5.3.3.1	Introduction	198

5.3.3.2	Requirements	198

5.3.3.2.1	Handover delay	198

5.3.3.2.2	Interruption time	198

5.3.4	E-UTRAN - NR FR1 Handover	199

5.3.4.1	Introduction	199

5.3.4.2	Handover delay	199

5.3.4.3	Interruption time	199

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	199

5.3.4A.1	Introduction	199

5.3.4A.2	Handover delay	200

5.3.4A.3	Interruption time	200

5.3.5	E-UTRAN - NR FR2 Handover	201

5.3.5.1	Introduction	201

5.3.5.2	Handover delay	201

5.3.5.3	Interruption time	201

5.4	Handover to Non-3GPP RATs	202

5.4.1	E-UTRAN – HRPD Handover	202

5.4.1.1	Introduction	202

5.4.1.1.1	Handover delay	202

5.4.1.1.2	Interruption time	202

5.4.2	E-UTRAN – cdma2000 1X Handover	203

5.4.2.1	Introduction	203

5.4.2.1.1	Handover delay	203

5.4.2.1.2	Interruption time	203

5.5	E-UTRAN Handover for Cat-M1 UEs	203

5.5.1	Introduction	203

5.5.2	Requirements in CEModeA	203

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	203

5.5.2.1.1	Handover delay	204

5.5.2.1.2	Interruption time	204

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	204

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	204

5.5.2.3.1	Void	205

5.5.2.3.2	Void	205

5.5.3	Requirements in CEModeB	205

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	205

5.5.3.1.1	Handover delay	205

5.5.3.1.2	Interruption time	205

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	206

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	206

5.6	Void	206

5.7	E-UTRAN DAPS Handover	206

5.7.1	Introduction	206

5.7.2	Requirements	206

5.7.2.1	E-UTRAN FDD – FDD	206

5.7.2.1.1	DAPS Handover delay	206

5.7.2.1.2	Interruption time	207

5.7.2.2	E-UTRAN FDD – TDD	207

5.7.2.3	E-UTRAN TDD – FDD	208

5.7.2.4	E-UTRAN TDD – TDD	208

5.8	EN-DC Handover with PSCell	208

5.8.1	Introduction	208

5.8.1.1	Handover with PSCell Interruption time	208

5.8.1.2	Handover with PSCell - NR PSCell Changing Delay requirements	208

5.9	EN-DC Handover with PSCell using CCA	209

5.9.1	Introduction	209

5.9.1.1	Handover with PSCell – E-UTRA HO Interruption time	210

5.9.1.2	Handover with PSCell - NR PSCell Change Delay requirements	210

6	RRC Connection Mobility Control	211

6.1	RRC Re-establishment	211

6.1.1	Introduction	211

6.1.2	Requirements	211

6.1.2.1	UE Re-establishment delay requirement	211

6.2	Random Access	212

6.2.1	Introduction	212

6.2.2	Requirements	212

6.2.2.1	Contention based random access	212

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	212

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	212

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	212

6.2.2.1.4	Void	212

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	212

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	213

6.2.2.2	Non-Contention based random access	213

6.2.2.2.1	Correct behaviour when receiving Random Access Response	213

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	213

6.2.3	Requirements for Cat-M1 UEs	213

6.3	RRC Connection Release with Redirection	213

6.3.1	Introduction	213

6.3.2	Requirements	213

6.3.2.1	RRC connection release with redirection to UTRAN FDD	213

6.3.2.2	RRC connection release with redirection to GERAN	214

6.3.2.3	RRC connection release with redirection to UTRAN TDD	214

6.3.2.4	RRC connection release with redirection to NR	215

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	215

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	217

6.4.1	Introduction	217

6.4.2	Requirements	217

6.5	RRC Re-establishment for NB-IoT UEs	217

6.5.1	Introduction	217

6.5.2	Requirements	217

6.5.2.1	UE Re-establishment delay requirement in normal coverage	217

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	218

6.6	Random Access for UE category NB1	218

6.6.1	Introduction	218

6.6.2	Requirements	219

6.6.2.1	Correct behaviour when receiving Random Access Response reception	219

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	219

6.6.2.3	Correct behaviour when receiving a NACK on msg3	219

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	219

6.6.2.5	Correct behaviour when contention Resolution timer expires	219

6.6.2.6	MSG3-based channel quality report for UE Category NB1	219

6.6.3	Requirements for NPRACH configuration	220

6.7	RRC Re-establishment for Cat-M1 UEs	220

6.7.1	Introduction	220

6.7.2	Requirements	220

6.7.2.1	UE Re-establishment delay requirement for CEModeA	220

6.7.2.2	UE Re-establishment delay requirement for CEModeB	221

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	221

6.8.1	Introduction	221

6.8.2	Requirements	222

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	222

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	222

6.9.1	Introduction	222

6.9.2	Requirements	222

7	Timing and signalling characteristics	223

7.1	UE transmit timing	223

7.1.1	Introduction	223

7.1.2	Requirements	224

7.2	UE timer accuracy	225

7.2.1	Introduction	225

7.2.2	Requirements	225

7.3	Timing Advance	225

7.3.1	Introduction	225

7.3.2	Requirements	225

7.3.2.1	Timing Advance adjustment delay	225

7.3.2.2	Timing Advance adjustment accuracy	226

7.4	Cell phase synchronization accuracy (TDD)	226

7.4.1	Definition	226

7.4.2	Minimum requirements	226

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	227

7.5.1	Introduction	227

7.5.2	eNodeB Synchronization Requirements	227

7.5.2.1	Synchronized E-UTRAN	227

7.5.2.2	Non-Synchronized E-UTRAN	227

7.6	Radio Link Monitoring	227

7.6.1	Introduction	227

7.6.2	Requirements	229

7.6.2.1	Minimum requirement when no DRX is used	229

7.6.2.2	Minimum requirement when DRX is used	229

7.6.2.3	Minimum requirement at transitions	231

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	231

7.6.2.5	Minimum requirement under IDC Interference	231

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	231

7.7.1	Introduction	231

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	232

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	233

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	234

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	235

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	236

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	237

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	238

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	238

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	238

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	239

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	240

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	241

7.7.14	SCell Activation Delay Requirement for Dormant SCell	241

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	243

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	243

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	245

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	245

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	247

7.8	Interruptions with Carrier Aggregation	249

7.8.1	Introduction	249

7.8.2	Requirements	249

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	249

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	249

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	249

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	250

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	250

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	250

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	250

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	250

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	251

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	252

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	252

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	252

7.8.2.13	Interruptions at SRS carrier based switching	253

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	254

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	254

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	254

7.8.2.17	Interruptions during CQI measurement on dormant SCell	254

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	255

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	255

7.8.2.20	Interruptions at SCell hibernation	255

7.8.2.21	Interruptions at direct SCell activation and hibernation	255

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	256

7.9.1	Introduction	256

7.9.2	Minimum Requirements for Interband Carrier Aggregation	256

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	256

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	257

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	257

7.10.1	Introduction	257

7.10.2	Requirements	257

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	257

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	257

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	258

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	258

7.11	Radio Link Monitoring for UE Category 0	259

7.11.1	Introduction	259

7.11.2	Requirements for FD-FDD and TDD	260

7.11.2.1	Minimum requirement when no DRX is used	260

7.11.2.2	Minimum requirement when DRX is used	261

7.11.2.3	Minimum requirement at transitions	261

7.11.3	Requirements for HD-FDD	262

7.11.3.1	Minimum requirement when no DRX is used	262

7.11.3.2	Minimum requirement when DRX is used	262

7.11.3.3	Minimum requirement at transitions	263

7.12	Interruptions with Dual Connectivity	263

7.12.1	Introduction	263

7.12.2	Requirements	263

7.12.2.1	Interruptions at PSCell addition/release	263

7.12.2.2	Interruptions at transitions between active and non-active during DRX	263

7.12.2.3	Interruptions at transitions from non-DRX to DRX	264

7.12.2.4	Interruptions at SCell addition/release	264

7.12.2.5	Interruptions at SCell activation/deactivation	264

7.12.2.6	Interruptions during measurements on SCC	265

7.12.2.7	Interruptions at SRS carrier based switching	265

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	266

7.13.1	Definition	266

7.13.2	Minimum requirements	266

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	266

7.14.1	Introduction	266

7.14.2	PSCell Addition Delay Requirement	266

7.14.3	PSCell Release Delay Requirement	267

7.15	Maximum Receive Timing Difference in Dual Connectivity	267

7.15.1	Introduction	267

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	267

7.16	Proximity-based Services	267

7.16.1	Introduction	267

7.16.2	Requirements	267

7.16.2.1	ProSe UE transmission timing	267

7.16.2.1.1	Serving cell or PCell as timing reference	268

7.16.2.1.2	SCell or non-serving cell as timing reference	268

7.16.3	Interruptions with ProSe	268

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	268

7.16.3.2	Interruptions at ProSe Direct Communication configuration	269

7.16.3.3	Interruptions during ProSe Direct Discovery	269

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	269

7.16.3.5	Interruptions during ProSe Direct Communication	270

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	270

7.16.4.1	Measurement and evaluation of selected cell	270

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	270

7.16.5	Selection / Reselection of ProSe relay UE	271

7.16.6	ProSe operation under deactivated SCell	271

7.17	Maximum Transmission Timing Difference in Dual Connectivity	271

7.17.1	Introduction	271

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	272

7.18.1	Introduction	272

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	272

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	272

7.19	Radio Link Monitoring for UE Category M1	272

7.19.1	Introduction	272

7.19.2	Requirements for FD-FDD and TDD CE mode A	273

7.19.2.1	Minimum requirement when no DRX is used	274

7.19.2.2	Minimum requirement when DRX is used	275

7.19.2.3	Minimum requirement at transitions	276

7.19.3	Requirements for HD-FDD with CE mode A	276

7.19.3.1	Minimum requirement when no DRX is used	277

7.19.3.2	Minimum requirement when DRX is used	277

7.19.3.3	Minimum requirement at transitions	278

7.19.4	Requirements for FD-FDD and TDD with CE mode B	278

7.19.4.1	Minimum requirement when no DRX is used	280

7.19.4.2	Minimum requirement when DRX is used	281

7.19.4.3	Minimum requirement at transitions	282

7.19.5	Requirements for HD-FDD with CE mode B	282

7.19.5.1	Minimum requirement when no DRX is used	282

7.19.5.2	Minimum requirement when DRX is used	283

7.19.5.3	Minimum requirement at transitions	284

7.20	UE transmit timing for NB-IoT	284

7.20.1	Introduction	284

7.20.2	Requirements	284

7.21	 UE timer accuracy for NB-IoT	285

7.21.1	Introduction	285

7.21.2	Requirements	285

7.22	Timing Advance for NB-IoT	285

7.22.1	Introduction	285

7.22.2	Requirements	285

7.22.2.1	Timing Advance adjustment delay	285

7.22.2.2	Timing Advance adjustment accuracy	285

7.23	Radio Link Monitoring for Category NB1 UE	285

7.23.1	Introduction	285

7.23.2	Requirements for Category NB1 UE	286

7.23.2.1	Minimum requirement when no DRX is used	286

7.23.2.2	Minimum requirement when DRX is used	287

7.23.2.3	Minimum requirement at transitions	287

7.24	UE transmit timing for Category M1	287

7.24.1	Introduction	287

7.24.2	Requirements	288

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	288

7.25.1	Definition	288

7.25.2	Minimum requirements	289

7.26	UE transmit timing for Category M2	289

7.26.1	Introduction	289

7.26.2	Requirements	289

7.27	UE timer accuracy for category M1	289

7.27.1	Introduction	289

7.27.2	Requirements	289

7.28	Timing Advance for Category M1	290

7.28.1	Introduction	290

7.28.2	Requirements	290

7.29	Interruptions requirements with FeMBMS	290

7.29.1	Introduction	290

7.29.2	Requirements	290

7.30	Numerology switching delay requirements with FeMBMS	290

7.30.1	Introduction	290

7.30.2	Requirements	290

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	290

7.31.1	Introduction	290

7.31.2	NR PSCell Addition Delay Requirement	291

7.31.3	NR PSCell Release Delay Requirement	291

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	292

7.31A.1	Introduction	292

7.31A.2	NR PSCell Addition Delay Requirement	292

7.31A.3	NR PSCell Release Delay Requirement	293

7.32	Interruptions with EN-DC	293

7.32.1	Introduction	293

7.32.2	Requirements	294

7.32.2.1	Interruptions at PSCell addition/release	294

7.32.2.2	Interruptions at transitions between active and non-active during DRX	294

7.32.2.3	Interruptions at transitions from non-DRX to DRX	294

7.32.2.4	Interruptions at SCell addition/release	294

7.32.2.5	Interruptions at SCell activation/deactivation	294

7.32.2.6	Interruptions during measurements on SCC	295

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	295

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	295

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	295

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	295

7.32.2.7	Interruptions at active BWP switching	296

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	296

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	297

7.32.2.10	Interruptions at SCell hibernation	297

7.32.2.11	Interruptions at direct SCell activation and hibernation	297

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	297

7.32.2.13	Interruptions at NR SRS carrier based switching	297

7.32.2.14	Interruptions at NR SCell dormancy	298

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	298

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	299

7.32.2.15	Interruption during NR measurement with autonomous gaps	299

7.32.2.16	Interruptions at SRS carrier based switching	299

7.32.2.17	Interruptions at SCG activation/deactivation	299

7.32.2.18	Interruptions due to NR SRS antenna port switching	300

7.32.2.19	Interruptions at fast SCell activation/deactivation	300

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	301

7.33.1	Introduction	301

7.33.2	Requirements	301

7.34	Void	301

7.35	Interruptions with SFTD measurements	301

7.35.1	Introduction	301

7.35.2	Requirements	301

7.36	Interruptions with NE-DC	302

7.36.1	Introduction	302

7.36.2	Requirements	302

7.36.2.1	Interruptions at transitions between active and non-active during DRX	302

7.36.2.2	Interruptions at transitions from non-DRX to DRX	302

7.36.2.3	Interruptions at SCell addition/release	302

7.36.2.4	Interruptions at SCell activation/deactivation	303

7.36.2.5	Interruptions during measurements on SCC	303

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	303

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	303

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	303

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	303

7.36.2.6	Interruptions at active BWP switching	304

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	305

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	305

7.36.2.9	Interruptions at SCell hibernation	305

7.36.2.10	Interruptions at direct SCell activation and hibernation	305

7.36.2.11	Interruptions at NR SRS carrier based switching	305

7.36.2.12	Interruptions at NR SCell dormancy	306

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	306

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	306

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	306

7.36.2.14	Interruption during NR measurement with autonomous gaps	307

7.36.2.15	Interruptions at SRS carrier based switching	307

7.36.2.16	Interruptions due to NR SRS antenna port switching	308

7.37	Interruptions during NR measurement with autonomous gaps	308

7.37.1	Introduction	308

7.37.2	Requirements	309

7.38 SCG Activation and Deactivation Delay	309

7.38.1	Introduction	309

7.38.2	SCG Activation Delay Requirement	309

7.38.3	SCG Deactivation Delay Requirement	310

8	UE Measurements Procedures in RRC_CONNECTED State	310

8.1	General Measurement Requirements	310

8.1.1	Introduction	310

8.1.2	Requirements	311

8.1.2.1	UE measurement capability	311

8.1.2.1.1	Monitoring of multiple layers using gaps	319

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	320

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	321

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	323

8.1.2.1.1d	Monitoring of multiple layers using gaps (RedCap)	324

8.1.2.1.2	Network controlled small gap	324

8.1.2.2	E-UTRAN intra frequency measurements	326

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	326

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	331

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	335

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	336

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	337

8.1.2.3	E-UTRAN inter frequency measurements	337

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	338

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	343

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	350

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	350

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	350

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	351

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	353

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	354

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	355

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	362

8.1.2.4	Inter RAT measurements	362

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	362

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	367

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	367

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	371

8.1.2.4.5	E-UTRAN FDD – GSM measurements	371

8.1.2.4.6	E-UTRAN TDD – GSM measurements	376

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	376

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	378

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	378

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	378

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	379

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	379

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	379

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	379

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	381

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	381

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	381

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	381

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	382

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	382

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	384

8.1.2.4.21	E-UTRAN FDD – NR measurements	384

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	388

8.1.2.4.22	E-UTRAN TDD – NR measurements	392

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	392

8.1.2.4.23	Void	392

8.1.2.4.24	Void	392

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	392

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	394

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	394

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	395

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	395

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	395

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	397

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	399

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	401

8.1.2.6.5	Void	402

8.1.2.6.6	Void	402

8.1.2.6.7	Void	402

8.1.2.6.8	Void	402

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	402

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	403

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	404

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	406

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	408

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	410

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	411

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	413

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	415

8.1.2.7	E-UTRAN E-CID Measurements	417

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	417

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	418

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	419

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	420

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	420

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	421

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	423

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	426

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	430

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	433

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	433

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	434

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	434

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	435

8.1.2.10	Void	435

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	435

8.2.1	Introduction	435

8.2.2	Requirements	435

8.3	Measurements for E-UTRA carrier aggregation	440

8.3.1	Introduction	440

8.3.2	Measurements of the primary component carrier	440

8.3.3	Measurements of a secondary component carrier	440

8.3.3.1	Measurements of a secondary component carrier with active SCell	440

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	441

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	441

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	442

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	444

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	444

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	444

8.4.1	Introduction	444

8.4.2	Measurements on the primary component carrier	445

8.4.3	Measurements on a secondary component carrier	446

8.4.4	Measurements on both primary component carrier and a secondary component carrier	446

8.4.5	Measurements on different secondary component carriers	447

8.5	Measurements for UE category 0	448

8.5.1	Introduction	448

8.5.2 	Requirements	449

8.5.2.1 	E-UTRAN intra frequency measurements	449

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	449

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	452

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	454

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	458

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	458

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	459

8.6	Discovery signal measurements	460

8.6.1	Introduction	460

8.6.2	Requirements for CRS based discovery signal measurements	460

8.6.2.1	E-UTRAN intra frequency measurements	460

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	460

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	463

8.6.2.2	E-UTRAN inter frequency measurements	465

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	466

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	468

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	471

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	471

8.6.3	Requirements for CSI-RS based discovery signal measurements	471

8.6.3.1	E-UTRAN intra frequency measurements	471

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	472

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	474

8.6.3.2	E-UTRAN inter frequency measurements	476

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	477

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	479

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	482

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	482

8.7	Discovery signal measurements for E-UTRA carrier aggregation	482

8.7.1	Introduction	482

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	483

8.7.2.1	Measurements of the primary component carrier	483

8.7.2.2	Measurements of a secondary component carrier	483

8.7.2.3	Measurements of a secondary component carrier with active SCell	483

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	483

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	483

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	484

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	486

8.7.3.1	Measurements of the primary component carrier	486

8.7.3.2	Measurements of a secondary component carrier	486

8.7.3.3	Measurements of a secondary component carrier with active SCell	486

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	486

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	486

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	488

8.8	Measurements for E-UTRA dual connectivity	489

8.8.1	Introduction	489

8.8.2	Intra-frequency measurements requirements on PCell	489

8.8.3	Intra-frequency measurements requirements on PSCell	490

8.8.4	Inter-frequency and inter-RAT measurement requirements	490

8.8.5	Intra-frequency measurements with autonomous gaps	490

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	490

8.8.5.2	ECGI reporting delay	491

8.8.6	Inter-frequency measurements with autonomous gaps	491

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	491

8.8.6.2	ECGI reporting delay	492

8.8.7	SSTD Measurements	492

8.8.7.1	Introduction	492

8.8.7.2	SSTD Measurement requirements	492

8.8.7.3	SSTD Measurement Reporting Delay	493

8.8.8	Intra-frequency measurements requirements on SCell	493

8.9	MBSFN Measurements	493

8.9.1	Introduction	493

8.9.2	MBSFN RSRP Measurements	493

8.9.3	MBSFN RSRQ Measurements	494

8.9.4	MCH BLER Measurements	494

8.10	Proximity-based Services	494

8.10.1	Introduction	494

8.10.2	Requirements	494

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	494

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	495

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	495

8.11.1	Introduction	495

8.11.2	CRS based discovery signal measurements	496

8.11.2.1	E-UTRAN intra-frequency measurements	496

8.11.2.1.1	Requirements	496

8.11.2.1.1.1	Requirements when no DRX is used	496

8.11.2.1.1.1.1	Measurement Reporting Requirements	498

8.11.2.1.1.2	Requirements when DRX is used	499

8.11.2.1.1.2.1	Measurement Reporting Requirements	501

8.11.2.2	E-UTRAN inter-frequency measurements	502

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	502

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	506

8.11.3	CSI-RS based discovery signal measurements	506

8.11.3.1	E-UTRAN intra-frequency measurements	506

8.11.3.1.1	Requirements	506

8.11.3.1.1.1	Requirements when no DRX is used	506

8.11.3.1.1.1.1	Measurement Reporting Requirements	508

8.11.3.1.1.2	Requirements when DRX is used	508

8.11.3.1.1.2.1	Measurement Reporting Requirements	509

8.11.3.2	E-UTRAN inter-frequency measurements	510

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	510

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	514

8.11.4	RSSI measurements	514

8.11.4.1	E-UTRAN intra-frequency measurements	514

8.11.4.2	E-UTRAN inter-frequency measurements	514

8.11.5	Channel occupancy measurements	515

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	515

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	515

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	515

8.12.1	Introduction	515

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	515

8.12.2.1	Introduction	515

8.12.2.2	Measurements of a secondary component carrier	515

8.12.2.3	Measurements of a secondary component carrier with active SCell	515

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	516

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	516

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	518

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	521

8.12.3.1	Introduction	521

8.12.3.2	Measurements of a secondary component carrier	521

8.12.3.3	Measurements of a secondary component carrier with active SCell	521

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	521

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	521

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	523

8.13	Measurements for UE Category M1	525

8.13.1	Introduction	525

8.13.2	Requirements for UE category M1 with CE mode A	525

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	526

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	526

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	531

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	533

8.13.2.2	Void	537

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	537

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	537

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	540

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	542

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	543

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	543

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	545

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	547

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	548

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	548

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	548

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	549

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	549

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	549

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	550

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	550

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	551

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	552

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	552

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	553

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	557

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	559

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	563

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	563

8.13.3	Requirements for UE category M1 with CE mode B	564

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	564

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	565

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	569

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	572

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	577

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	578

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	578

8.13.3.2	Void	579

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	579

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	579

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	582

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	584

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	585

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	585

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	585

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	586

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	586

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	586

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	587

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	587

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	587

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	592

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	594

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	598

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	599

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	599

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	601

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	603

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	604

8.14	Measurements for UE category NB1	604

8.14.1	Introduction	604

8.14.2	NB-IoT intra frequency measurements under normal coverage	605

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	605

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	605

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	605

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	605

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	605

8.14.4	Connected mode channel quality report for UE Category NB1	605

8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM	606

8.14.6	NB-IoT neighbour cell measurements	606

8.14.6.1	Introduction	606

8.14.6.2	Requirements	606

8.14.6.3	Intra-frequency neighbour cell measurements	607

8.14.6.4	Inter-frequency neighbour cell measurements	607

8.15	Void	608

8.16	Measurements for UE Category M2	608

8.16.1	Introduction	608

8.16.2	Requirements for UE category M2 with CE mode A	609

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	609

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	609

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	610

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	610

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	611

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	611

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	611

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	611

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	614

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	617

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	617

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	617

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	619

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	622

8.16.3	Requirements for UE category M2 with CE mode B	622

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	622

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	623

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	625

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	628

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	628

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	628

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	630

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	633

8.17	Measurements for E-UTRA – NR Dual Connectivity	633

8.17.1	Introduction	633

8.17.1.1	Measurement Gap Sharing	633

8.17.1A	Intrafrequency Measurements	634

8.17.2	SFTD Measurements	634

8.17.2.1	Introduction	634

8.17.2.2	SFTD Measurement requirements	634

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	635

8.17.2.3	SFTD Measurement Reporting Delay	636

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	637

8.17.3.1	Introduction	637

8.17.3.2	E-UTRAN FDD inter frequency measurements	637

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	637

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	639

8.17.3.3	E-UTRAN TDD inter frequency measurements	641

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	641

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	642

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	644

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	644

8.17.4.1.1	NR Inter-RAT cell identification	645

8.17.4.1.2	NR Inter-RAT measurement	646

8.17.4.1.3	NR Inter-RAT measurement reporting	647

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	648

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	648

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	648

8.17.4A.1.1	NR Inter-RAT cell identification	649

8.17.4A.1.2	NR Inter-RAT measurement	650

8.17.4A.1.3	NR Inter-RAT measurement reporting	651

8.17.4A.1.4	NR inter-RAT RSSI measurements	651

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	652

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	652

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	652

8.17.5.1	Introduction	652

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	653

8.17.5.2.1	Identification of a new UTRA FDD cell	653

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	653

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	653

8.17.5.2.4	Periodic Reporting	654

8.17.5.2.5	Event Triggered Reporting	654

8.17.5.2.6	Event-triggered Periodic Reporting	654

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	654

8.17.5.3.1	Periodic Reporting	656

8.17.5.3.2	Event Triggered Reporting	656

8.17.5.3.3	Event-triggered Periodic Reporting	656

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	656

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	657

8.17.7.1	Introduction	657

8.17.7.2	Identification of a new UTRA FDD cell for SON	657

8.17.7.2.1	Requirements when no DRX is used	657

8.17.7.2.2	Requirements when DRX is used	657

8.17.7.2.3	Reporting Delay	658

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	659

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	659

8.17.9.1	Introduction	659

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	659

8.17.9.2.1	Identification of a new UTRA FDD cell	659

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	659

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	660

8.17.9.2.4	Periodic Reporting	660

8.17.9.2.5	Event Triggered Reporting	660

8.17.9.2.6	Event-triggered Periodic Reporting	661

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	661

8.17.9.3.1	Periodic Reporting	662

8.17.9.3.2	Event Triggered Reporting	662

8.17.9.3.3	Event-triggered Periodic Reporting	662

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	663

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	663

8.17.11.1	Introduction	663

8.17.11.2	Identification of a new UTRA TDD cell for SON	663

8.17.11.2.1	Requirements when no DRX is used	663

8.17.11.2.2	Requirements when DRX is used	663

8.17.11.2.3	Reporting Delay	664

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	664

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	665

8.17.13.1	Introduction	665

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	665

8.17.13.2.1	GSM carrier RSSI	665

8.17.13.2.2	BSIC verification	665

8.17.13.2.3	Enhanced BSIC verification	667

8.17.13.2.4	Periodic Reporting	667

8.17.13.2.5	Event Triggered Reporting	667

8.17.13.2.6	Event-triggered Periodic Reporting	668

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	668

8.17.13.3.1	GSM carrier RSSI	668

8.17.13.3.2	BSIC verification	668

8.17.13.3.3	Periodic Reporting	670

8.17.13.3.4	Event Triggered Reporting	670

8.17.13.3.5	Event-triggered Periodic Reporting	670

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	670

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	670

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	670

8.17.15.1.1	RSTD Measurement Reporting Delay	671

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	671

8.17.15.2.1	RSTD Measurement Reporting Delay	671

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	671

8.17.15.3.1	RSTD Measurement Reporting Delay	672

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	672

8.17.15.4.1	RSTD Measurement Reporting Delay	672

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	673

8.17.16.1	Introduction	673

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	673

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	673

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	673

8.17.17.1	Introduction	673

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	673

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	673

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	673

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	673

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	673

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	673

8.18	Measurements for non-BL/CE UE	673

8.18.1	Introduction	673

8.18.2	Requirements for non-BL/CE UE with CE Mode B	674

8.18.2.1	E-UTRAN intra frequency measurements	674

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	674

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	674

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	675

8.19	Measurements for NR – E-UTRA Dual Connectivity	676

8.19.1	Introduction	676

8.19.2	Intra-frequency Measurements	676

8.19.3	Inter-frequency Measurements	676

8.19.4	Void	676

8.19.5	Intra-frequency E-CID Measurements	676

8.19.6	Intra-frequency measurements with autonomous gaps	677

8.19.6.1	Introduction	677

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	677

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	677

8.19.7	Inter-frequency measurements with autonomous gaps	677

8.19.7.1	Introduction	677

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	677

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	677

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	677

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	677

8.20	Inter-RAT NR Measurements for RedCap UE	677

8.20.1	Introduction	677

8.20.2	E-UTRAN FDD – NR measurements	677

8.20.2.1	Identification of a new NR cell	677

8.20.2.2	Periodic Reporting	681

8.20.2.3	Event Triggered Reporting	681

8.20.2.4	Event-triggered Periodic Reporting	681

8.20.3	E-UTRAN TDD – NR measurements	681

9	Measurements performance requirements for UE	682

9.1	E-UTRAN measurements	682

9.1.1	Introduction	682

9.1.2	Intra-frequency RSRP Accuracy Requirements	682

9.1.2.1	Absolute RSRP Accuracy	682

9.1.2.2	Relative Accuracy of RSRP	683

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	684

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	684

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	685

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	686

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	687

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	688

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	689

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	689

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	689

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	690

9.1.2B.1	Introduction	690

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	690

9.1.3	Inter-frequency RSRP Accuracy Requirements	691

9.1.3.1	Absolute RSRP Accuracy	691

9.1.3.2	Relative Accuracy of RSRP	692

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	693

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	693

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	694

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	694

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	695

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	696

9.1.3B.1	Introduction	696

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	696

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	696

9.1.4	RSRP Measurement Report Mapping	697

9.1.5	Intra-frequency RSRQ Accuracy Requirements	697

9.1.5.1	Absolute RSRQ Accuracy	697

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	698

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	699

9.1.5.4	Absolute WB-RSRQ Accuracy	700

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	701

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	702

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	702

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	702

9.1.5B.1	Introduction	702

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	702

9.1.6	Inter-frequency RSRQ Accuracy Requirements	703

9.1.6.1	Absolute RSRQ Accuracy	703

9.1.6.2	Relative Accuracy of RSRQ	704

9.1.6.3	Absolute WB-RSRQ Accuracy	705

9.1.6.4	Relative WB-RSRQ Accuracy	705

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	706

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	707

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	707

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	707

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	708

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	709

9.1.6B.1	Introduction	709

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	709

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	709

9.1.7	RSRQ Measurement Report Mapping	710

9.1.8	Power Headroom	710

9.1.8.1	Period	711

9.1.8.2	Reporting Delay	711

9.1.8.3	Void	711

9.1.8.4	Report Mapping	711

9.1.9	UE Rx – Tx time difference	711

9.1.9.1	Measurement Requirement	711

9.1.9.2	Measurement Report mapping	712

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	713

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	714

9.1.10	Reference Signal Time Difference (RSTD)	715

9.1.10.1	Intra-Frequency Accuracy Requirement	715

9.1.10.2	Inter-Frequency Accuracy Requirement	716

9.1.10.3	RSTD Measurement Report Mapping	718

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	718

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	719

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	720

9.1.11	Carrier aggregation measurement accuracy	721

9.1.11.1	Primary component carrier accuracy requirement	722

9.1.11.2	Secondary component carrier accuracy requirement	722

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	722

9.1.11.4	Secondary component carrier relative accuracy requirement	722

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	722

9.1.13	Measurement accuracy for UE category 0	723

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	723

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	723

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	724

9.1.14	Accuracy requirements for Discovery Signal Measurements	725

9.1.14.1	Introduction	725

9.1.14.2	RSRP measurements in discovery signal occasions	725

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	725

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	725

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	725

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	726

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	727

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	727

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	727

9.1.14.3.3	CSI-RSRP measurement report mapping	728

9.1.14.4	RSRQ measurements in discovery signal occasions	728

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	728

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	729

9.1.15.1.1	Primary component carrier accuracy requirement	729

9.1.15.1.2	Secondary component carrier accuracy requirement	729

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	729

9.1.15.1.4	Secondary component carrier relative accuracy requirement	729

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	729

9.1.15.2.1	Primary component carrier accuracy requirement	729

9.1.15.2.2	Secondary component carrier accuracy requirement	729

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	729

9.1.15.2.4	Secondary component carrier relative accuracy requirement	729

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	730

9.1.17	RS-SINR Measurements	730

9.1.17.1	Measurement Report Mapping	730

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	731

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	731

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	731

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	731

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	732

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	733

9.1.18.1	Introduction	733

9.1.18.2	RSRP measurements	733

9.1.18.2.1	RSRP measurement report mapping	733

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	733

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	734

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	734

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	735

9.1.18.3	RSRQ measurements	735

9.1.18.3.1	RSRQ measurement report mapping	735

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	735

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	736

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	736

9.1.18.4	CSI-RSRP measurements	737

9.1.18.4.1	CSI-RSRP measurement report mapping	737

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	737

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	737

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	738

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	738

9.1.18.5	RSSI measurements	739

9.1.18.5.1	RSSI measurement report mapping	739

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	739

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	740

9.1.18.6	Channel occupancy measurements	740

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	740

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	740

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	741

9.1.19.1	Introduction	741

9.1.19.2	Accuracy requirements for measurements on SCC	741

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	741

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	741

9.1.20	SFN and Subframe Time Difference (SSTD)	742

9.1.20.1	SSTD Accuracy Requirement	742

9.1.20.2	SSTD Measurement Report Mapping	742

9.1.21	Measurement accuracy for UE category M1	743

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	743

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	745

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	746

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	748

9.1.21.5	RSRP Measurement Report Mapping	749

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	749

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	750

9.1.21.8	RSRQ Measurement Report Mapping	751

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	751

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	752

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	753

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	754

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	755

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	756

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	757

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	758

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	759

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	760

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	761

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	762

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	765

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	768

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	768

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	771

9.1.22	Measurement accuracy for UE Category NB1	772

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	772

9.1.22.2	Void	773

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	773

9.1.22.4	Void	774

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	774

9.1.22.6	Void	775

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	775

9.1.22.8	Void	776

9.1.22.9	NRSRP Measurement Report Mapping	776

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	776

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	777

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	778

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	779

9.1.22.14	NRSRQ Measurement Report Mapping	780

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	781

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	781

9.1.22.17	Channel quality reporting for UE Category NB2 with 16-QAM	782

9.1.23	Power Headroom for UE Category NB1	782

9.1.23.1	Period	783

9.1.23.2	Reporting Delay	783

9.1.23.3	Report Mapping for UE Category NB1	783

9.1.23.3.1	Void	784

9.1.23.3.2	Void	784

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	784

9.1.24	Void	785

9.1.25	Measurement accuracy for UE category M2	785

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	785

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	786

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	787

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	788

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	790

9.1.26	Measurement Accuracy for non-BL CE UE	791

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	792

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	794

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	794

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	796

9.1.26.5	RSRP Measurement Report Mapping	797

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	797

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	797

9.1.26.8	RSRQ Measurement Report Mapping	798

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	798

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	799

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	800

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	801

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	802

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	802

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	802

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	803

9.1.27	SFN and frame Timing Difference (SFTD)	804

9.1.27.1	SFTD Accuracy Requirement	804

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	806

9.1.28.1	SFTD Accuracy Requirement under CCA	806

9.2	UTRAN FDD Measurements	807

9.2.1	UTRAN FDD CPICH RSCP	807

9.2.2	Void	808

9.2.3	UTRAN FDD CPICH Ec/No	808

9.3	UTRAN TDD Measurements	808

9.3.1	UTRAN TDD P-CCPCH RSCP	809

9.3.2	Void	809

9.3.3	Void	809

9.4	GSM Measurements	809

9.4.1	GSM carrier RSSI	809

9.5	CDMA2000 1x RTT Measurements	809

9.5.1	CDMA2000 1x RTT Pilot Strength	809

9.6	PCMAX,c	810

9.6.1	Report Mapping	810

9.6.2 	Estimation Period	810

9.6.3	Reporting Delay	810

9.7	IEEE802.11 Measurements	810

9.7.1	WLAN RSSI	810

9.7.2	WLAN RSSI Measurement Report Mapping	810

9.8	MBSFN Measurements	811

9.8.1	Introduction	811

9.8.2	MBSFN RSRP	811

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	811

9.8.2.2	MBSFN RSRP measurement report mapping	812

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	812

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	813

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	813

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	813

9.8.3	MBSFN RSRQ	814

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	814

9.8.3.2	MBSFN RSRQ measurement report mapping	814

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	815

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	815

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	815

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	816

9.8.4	MCH BLER	816

9.8.4.1	Measurement report mapping for MCH BLER	816

9.8.4.2	Measurement report mapping for MCH Block Number	817

9.9	ProSe Measurements	818

9.9.1	Introduction	818

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	818

9.9.2.1	Absolute S-RSRP Accuracy	818

9.9.2.2	Relative Accuracy of S-RSRP	819

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	820

9.9.3.1	Absolute SD-RSRP Accuracy	820

9.9.3.2	Relative Accuracy of SD-RSRP	820

9.10	V2X Measurements	821

9.10.1	Introduction	821

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	821

9.10.2.1	Absolute S-RSRP Accuracy	821

9.10.2.2	Relative Accuracy of S-RSRP	822

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	822

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	822

9.10.4	S-RSSI Measurement Accuracy Requirements	823

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	823

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	823

9.11	NR Measurements	824

9.11.1	NR SS-RSRP Measurements	824

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	824

9.11.2	NR SS-RSRQ Measurements	825

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	825

9.11.3	NR SS-SINR Measurements	825

9.11.4	NR SS-RSRP Measurements under CCA	826

9.11.5	NR SS-RSRQ Measurements under CCA	826

9.11.6	NR SS-SINR Measurements under CCA	826

9.11.7	NR RSSI Measurements under CCA	826

9.11.8	NR Channel Occupancy Measurements under CCA	827

10	Measurements Performance Requirements for E-UTRAN	827

10.1	Received Interference Power	827

10.1.1	Absolute accuracy requirement	827

10.1.2	Relative accuracy requirement	827

10.1.3	Received Interference Power measurement report mapping	828

10.2	Angle of Arrival (AOA)	828

10.2.1	Range/mapping	828

10.3	Timing Advance (TADV)	828

10.3.1	Report mapping	828

11	ProSe Requirements in Any Cell Selection state	829

11.1	Introduction	829

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	829

11.2.1	Introduction	829

11.2.2	ProSe UE transmission timing	829

11.3	Initiation/Cease of SLSS Transmissions	830

11.3.1	Introduction	830

11.3.2	Requirements	830

11.4	Measurements for ProSe in Any Cell Selection State	830

11.4.1	Introduction	830

11.4.2	Requirements	830

11.4.2.1	E-UTRA FDD	830

11.4.2.2	E-UTRA TDD	831

11.5	Selection / Reselection of ProSe Synchronization Reference	831

11.5.1	Introduction	831

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	831

11.5.2.1	Introduction	831

11.5.2.2	Requirements	831

11.6	Void	832

11.7	Selection / Reselection of ProSe relay UE	832

11.7.1	Introduction	832

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	832

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	832

12.1	Introduction	832

12.2	Transmit Timing	833

12.2.1	GNSS as timing reference	833

12.3	Interruption	833

12.4	Reliability of GNSS signal	833

13	V2X Requirements	833

13.1	Introduction	833

13.2	UE Transmit Timing	834

13.2.1	Introduction	834

13.2.2	GNSS as synchronization reference source	834

13.2.3	Serving cell/PCell as synchronization reference source	834

13.2.4	SyncRef UE as synchronization reference source	834

13.3	Initiation/Cease of SLSS Transmissions	834

13.3.1	Introduction	834

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	834

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	835

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	835

13.4	Selection / Reselection of V2X Synchronization Reference Source	836

13.5	Autonomous Resource Selection/Reselection measurements	837

13.5.1	Introduction	837

13.5.2	PSSCH-RSRP measurements	837

13.5.3	S-RSSI measurements	837

13.6	Congestion Control measurements	837

13.7	Interruption	837

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	837

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	837

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	838

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	838

13.8	Reliability of GNSS signal	838

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	839

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	839

Annex A (normative): Test Cases	818

A.1	Purpose of annex	818

A.2	Requirement classification for statistical testing	818

A.2.1	Types of requirements in TS 36.133	818

A.2.1.1	Time and delay requirements on UE higher layer actions	818

A.2.1.2	Measurements of power levels, relative powers and time	818

A.2.1.3	Implementation requirements	819

A.2.1.4	Physical layer timing requirements	819

A.3	RRM test configurations	820

A.3.1	Reference Measurement Channels	820

A.3.1.1	PDSCH	820

A.3.1.1.1	FDD	820

A.3.1.1.2	TDD	825

A.3.1.1.3	FDD for UE category 0	828

A.3.1.1.4	HD-FDD for UE category 0	829

A.3.1.1.5	TDD for UE category 0	830

A.3.1.1.6	Frame Structure 3	831

A.3.1.2	PCFICH/PDCCH/PHICH	832

A.3.1.2.1	FDD	832

A.3.1.2.2	TDD	832

A.3.1.2.3	HD-FDD for UE category 0	833

A.3.1.2.4	FS 3	833

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	833

A.3.1.3.1	FDD in CEModeA	834

A.3.1.3.2	HD-FDD in CEModeA	834

A.3.1.3.3	TDD in CEModeA	835

A.3.1.3.4	FDD in CEModeB	835

A.3.1.3.5	HD-FDD in CEModeB	836

A.3.1.3.6	TDD in CEModeB	836

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	837

A.3.1.4.1	FDD in CEModeA	837

A.3.1.4.2	HD-FDD in CEModeA	838

A.3.1.4.3	TDD in CEModeA	839

A.3.1.4.4	FDD in CEModeB	840

A.3.1.4.5	 HD-FDD in CEModeB	841

A.3.1.4.6	TDD in CEModeB	841

A.3.1.5	NPDSCH Reference Channel for UE category NB1	842

A.3.1.5.1	HD-FDD in-band operation	842

A.3.1.5.2	Void	843

A.3.1.5.3	HD-FDD standalone operation	843

A.3.1.5.4	Void	844

A.3.1.5.5	HD-FDD guard band operation	844

A.3.1.5.6	Void	845

A.3.1.5.7	TDD in-band operation	845

A.3.1.5.8	TDD standalone operation	845

A.3.1.5.9	TDD guard band operation	846

A.3.1.6	NPDCCH Reference Channel for UE category NB1	846

A.3.1.6.1	In-band operation	846

A.3.1.6.2	Void	847

A.3.1.6.3	Standalone operation	847

A.3.1.6.4	Void	847

A.3.1.6.5	Guard band operation	847

A.3.1.6.6	Void	848

A.3.2	OFDMA Channel Noise Generator (OCNG)	848

A.3.2.1	OCNG Patterns for FDD	848

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	849

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	849

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	850

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	850

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	851

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	852

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	852

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	852

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	853

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	854

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	854

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	855

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	855

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	856

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	856

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	857

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	857

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	858

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	858

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	859

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	859

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	860

A.3.2.2	OCNG Patterns for TDD	860

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	861

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	861

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	862

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	862

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	863

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	864

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	865

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	866

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	866

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	867

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	867

A.3.2.3	OCNG Patterns for Narrowband IoT	868

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	869

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	870

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	870

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	871

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	872

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	873

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	874

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	875

A.3.2.4	OCNG Patterns for V2X sidelink	876

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	876

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	877

A.3.3	Reference DRX Configurations	877

A.3.4	ABS Transmission Configurations	878

A.3.4.1	Non-MBSFN ABS Transmission Configurations	878

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	878

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	878

A.3.4.2	MBSFN ABS Transmission Configurations	879

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	879

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	880

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	880

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	880

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	881

A.3.6.1	Introduction	881

A.3.7	Test Cases with Different Channel Bandwidths	881

A.3.7.1	Introduction	881

A.3.7.2	Principle of testing	881

A.3.8	Antenna Configuration	881

A.3.8.1	Antenna connection for 4 Rx capable UEs	881

A.3.8.1.1 Introduction	881

A.3.8.1.2 Principle of testing	881

A.3.8.1.2.1 Single carrier tests	881

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	882

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	883

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	883

A.3.8.2	Antenna connection for 8 Rx capable UEs	883

A.3.8.2.1	Introduction	883

A.3.8.2.2	Principle of testing	883

A.3.8.2.2.1	Single carrier tests	883

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	884

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	884

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	884

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	884

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	884

A.3.9.1	Introduction	884

A.3.9.2	Principle of testing	884

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	885

A.3.10.1	Introduction	885

A.3.10.2	Principle of testing	885

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	885

A.3.11.1	Introduction	885

A.3.11.2	Principle of Testing	885

A.3.12	Proximity-based Services	885

A.3.12.1	Introduction	885

A.3.12.2	Reference DRX configurations for ProSe tests	885

A.3.12.3	Test Cases with Different Channel Bandwidths	886

A.3.12.3.1	Introduction	886

A.3.12.3.2	Principle of testing	886

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	886

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	893

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	896

A.3.12.6.1	FDD	896

A.3.12.7	Reference measurement channels for ProSe Direct Communication	896

A.3.12.7.1	FDD	896

A.3.12.8	ProSe Receive Traffic Generator	897

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	897

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	897

A.3.13	Time Offset between Cells	898

A.3.13.1	Introduction	898

A.3.13.2	Definition	898

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	898

A.3.14.1	Introduction	898

A.3.14.2	Principle of testing	898

A.3.15	Dual connectivity test cases with different combination of duplex mode	898

A.3.15.1	Introduction	898

A.3.15.2	Principle of testing	898

A.3.16	Reference PRACH Configurations	899

A.3.17	Listen before talk model	899

A.3.17.1	Introduction	899

A.3.17.2	Definition	899

A.3.18	Reference NPRACH Configurations	900

A.3.19	Dual connectivity test cases with different bandwidth combinations	901

A.3.19.1	Introduction	901

A.3.19.2	Principle of testing	901

A.3.20	Category M1 UE Test Cases	902

A.3.20.1	Introduction	902

A.3.20.2	Principle of Cat-M1 UE Testing	902

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	903

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	904

A.3.21.1	Introduction	904

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	904

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	905

A.3.22	Category 1bis UE Test Cases	906

A.3.22.1	Introduction	906

A.3.22.2	Principle of Category 1bis UE Testing	906

A.3.23	Category NB2 UE Test Cases	910

A.3.23.1	Introduction	910

A.3.23.2	Principle of Category NB2 UE Testing	910

A.3.24	V2X sidelink communication	913

A.3.24.1	Introduction	913

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	914

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	918

A.3.25	Category M2 UE Test Cases	919

A.3.25.1	Introduction	919

A.3.25.2	Principle of Cat-M2 UE Testing	919

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	920

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	921

A.3.26.1	Introduction	921

A.3.26.2	Principle of testing	921

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	921

A.3.27.1	Introduction	921

A.3.27.2	Principle of INACTIVE cell re-selection Testing	921

A.4	E-UTRAN RRC_IDLE state	921

A.4.2	Cell Re-Selection	921

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	921

A.4.2.1.1	Test Purpose and Environment	921

A.4.2.1.2	Test Requirements	923

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	924

A.4.2.2.1	Test Purpose and Environment	924

A.4.2.2.2	Test Requirements	925

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	926

A.4.2.3.1	Test Purpose and Environment	926

A.4.2.3.2	Test Requirements	927

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	928

A.4.2.4.1	Test Purpose and Environment	928

A.4.2.4.2	Test Requirements	929

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	930

A.4.2.5.1	Test Purpose and Environment	930

A.4.2.5.2	Test Requirements	931

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	932

A.4.2.6.1	Test Purpose and Environment	932

A.4.2.6.2	Test Requirements	933

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	934

A.4.2.7.1	Test Purpose and Environment	934

A.4.2.7.2	Test Requirements	935

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	936

A.4.2.8.1	Test Purpose and Environment	936

A.4.2.8.2	Test Requirements	937

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	938

A.4.2.9.1	Test Purpose and Environment	938

A.4.2.9.2	Test Requirements	938

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	938

A.4.2.10.1	Test Purpose and Environment	938

A.4.2.10.2	Test Requirements	942

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	942

A.4.2.11.1	Test Purpose and Environment	942

A.4.2.11.2	Test Requirements	946

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	946

A.4.2.12.1	Test Purpose and Environment	946

A.4.2.12.2	Test Requirements	948

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	949

A.4.2.13.1	Test Purpose and Environment	949

A.4.2.13.2	Test Requirements	950

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	951

A.4.2.14.1	Test Purpose and Environment	951

A.4.2.14.2	Test Requirements	952

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	953

A.4.2.15.1	Test Purpose and Environment	953

A.4.2.15.2	Test Requirements	954

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	955

A.4.2.16.1	Test Purpose and Environment	955

A.4.2.16.2	Test Requirements	956

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	957

A.4.2.17.1	Test Purpose and Environment	957

A.4.2.17.2	Test Requirements	958

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	959

A.4.2.18.1	Test Purpose and Environment	959

A.4.2.18.2	Test Requirements	961

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	962

A.4.2.19.1	Test Purpose and Environment	962

A.4.2.19.2	Test Requirements	964

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	965

A.4.2.20.1	Test Purpose and Environment	965

A.4.2.20.2	Test Requirements	966

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	967

A.4.2.21.1	Test Purpose and Environment	967

A.4.2.21.2	Test Requirements	968

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	969

A.4.2.22.1	Test Purpose and Environment	969

A.4.2.22.2	Test Requirements	970

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	971

A.4.2.23.1	Test Purpose and Environment	971

A.4.2.23.2	Test Requirements	972

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	973

A.4.2.24.1	Test Purpose and Environment	973

A.4.2.24.2	Test Requirements	976

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	977

A.4.2.25.1	Test Purpose and Environment	977

A.4.2.25.2	Test Requirements	978

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	979

A.4.2.26.1	Test Purpose and Environment	979

A.4.2.26.2	Test Requirements	980

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	981

A.4.2.27.1	Test Purpose and Environment	981

A.4.2.27.2	Test Requirements	982

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	983

A.4.2.28.1	Test Purpose and Environment	983

A.4.2.28.2	Test Requirements	984

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	985

A.4.2.29.1	Test Purpose and Environment	985

A.4.2.29.2	Test Requirements	986

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	987

A.4.2.30.1	Test Purpose and Environment	987

A.4.2.30.2	Test Requirements	988

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	989

A.4.2.31.1	Test Purpose and Environment	989

A.4.2.31.2	Test Requirements	990

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	991

A.4.2.32.1	Test Purpose and Environment	991

A.4.2.32.2	Test Requirements	992

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	993

A.4.2.33.1	Test Purpose and Environment	993

A.4.2.33.2	Test Requirements	994

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	995

A.4.2.34.1	Test Purpose and Environment	995

A.4.2.34.2	Test Requirements	996

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	997

A.4.2.35.1	Test Purpose and Environment	997

A.4.2.35.2	Test Requirements	999

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1000

A.4.2.36.1	Test Purpose and Environment	1000

A.4.2.36.2	Test Requirements	1002

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1003

A.4.2.37.1	Test Purpose and Environment	1003

A.4.2.37.2	Test Requirements	1005

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1006

A.4.2.38.1	Test Purpose and Environment	1006

A.4.2.38.2	Test Requirements	1008

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1009

A.4.2.39.1	Test Purpose and Environment	1009

A.4.2.39.2	Test Requirements	1010

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1011

A.4.2.40.1	Test Purpose and Environment	1011

A.4.2.40.2	Test Requirements	1012

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1013

A.4.2.41.1	Test Purpose and Environment	1013

A.4.2.41.2	Test Requirements	1016

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1017

A.4.2.42.1	Test Purpose and Environment	1017

A.4.2.42.2	Test Requirements	1019

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1020

A.4.2.43.1	Test Purpose and Environment	1020

A.4.2.43.2	Test Requirements	1022

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1023

A.4.2.44.1	Test Purpose and Environment	1023

A.4.2.44.2	Test Requirements	1025

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1026

A.4.2.45.1	Test Purpose and Environment	1026

A.4.2.45.2	Test Requirements	1028

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1029

A.4.2.46.1	Test Purpose and Environment	1029

A.4.2.46.2	Test Requirements	1031

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1032

A.4.2.47.1	Test Purpose and Environment	1032

A.4.2.47.2	Test Requirements	1034

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1035

A.4.2.48.1	Test Purpose and Environment	1035

A.4.2.48.2	Test Requirements	1036

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1037

A.4.2.49.1	Test Purpose and Environment	1037

A.4.2.49.2	Test Requirements	1039

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1040

A.4.2.50.1	Test Purpose and Environment	1040

A.4.2.50.2	Test Requirements	1042

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1043

A.4.2.51.1	Test Purpose and Environment	1043

A.4.2.51.2	Test Requirements	1045

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1046

A.4.2.52.1	Test Purpose and Environment	1046

A.4.2.52.2	Test Requirements	1048

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1049

A.4.2.53.1	Test Purpose and Environment	1049

A.4.2.53.2	Test Requirements	1050

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1051

A.4.2.54.1	Test Purpose and Environment	1051

A.4.2.54.2	Test Requirements	1053

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1054

A.4.2.55.1	Test Purpose and Environment	1054

A.4.2.55.2	Test Requirements	1056

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1057

A.4.2.56.1	Test Purpose and Environment	1057

A.4.2.56.2	Test Requirements	1058

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1059

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1059

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1059

A.4.3.1.1.1	Test Purpose and Environment	1059

A.4.3.1.1.2	Test Requirements	1061

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1061

A.4.3.1.2.1	Test Purpose and Environment	1061

A.4.3.1.2.2	Test Requirements	1064

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1064

A.4.3.1.3.1	Test Purpose and Environment	1064

A.4.3.1.3.2	Test Requirements	1067

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1067

A.4.3.1.4.1	Test Purpose and Environment	1067

A.4.3.1.4.2	Test Requirements	1068

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1068

A.4.3.1.5.1	Test Purpose and Environment	1068

A.4.3.1.5.2	Test Requirements	1071

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1072

A.4.3.2.1	Test Purpose and Environment	1072

A.4.3.2.1.1	Void	1072

A.4.3.2.1.2	1.28Mcps TDD option	1072

A.4.3.2.1.3	Void	1074

A.4.3.2.2	Test Requirements	1074

A.4.3.2.2.1	1.28Mcps TDD option	1074

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1074

A.4.3.2A.1	Test Purpose and Environment	1074

A.4.3.2A.2	Test Requirements	1078

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1078

A.4.3.3.1	Test Purpose and Environment	1078

A.4.3.3.2	Test Requirements	1081

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1081

A.4.3.3A.1	Test Purpose and Environment	1081

A.4.3.3A.2	Test Requirements	1086

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1087

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1087

A.4.3.4.1.1	Test Purpose and Environment	1087

A.4.3.4.1.2	Test Requirements	1089

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1089

A.4.3.4.2.1	Test Purpose and Environment	1089

A.4.3.4.2.2	Test Requirements	1091

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1091

A.4.3.4.3.1	Test Purpose and Environment	1091

A.4.3.4.3.2	Test Requirements	1094

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1094

A.4.3.4.4.1	Test Purpose and Environment	1094

A.4.3.4.4.2	Test Requirements	1098

A.4.4	E-UTRAN to GSM Cell Re-Selection	1098

A.4.4.1	E-UTRAN FDD – GSM:	1098

A.4.4.1.1	Test Purpose and Environment	1098

A.4.4.1.2	Test Requirements	1100

A.4.4.2	E-UTRAN TDD – GSM:	1100

A.4.4.2.1	Test Purpose and Environment	1100

A.4.4.2.2	Test Requirements	1102

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1103

A.4.5.1	E-UTRAN FDD – HRPD	1103

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1103

A.4.5.1.1.1	Test Purpose and Environment	1103

A.4.5.1.1.2	Test Requirements	1105

A.4.5.2	E-UTRAN TDD – HRPD	1105

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1105

A.4.5.2.1.1	Test Purpose and Environment	1105

A.4.5.2.1.2	Test Requirements	1108

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1108

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1108

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1108

A.4.6.1.1.1	Test Purpose and Environment	1108

A.4.6.1.1.2	Test Requirements	1111

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1111

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1111

A.4.6.2.1.1	Test Purpose and Environment	1111

A.4.6.2.1.2	Test Requirements	1114

A.4.7	Idle State Positioning Measurement for UE category NB1	1114

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1114

A.4.7.1.1	Test Purpose and Environment	1114

A.4.7.1.2	Test Requirements	1119

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1120

A.4.7.2.1	Test Purpose and Environment	1120

A.4.7.4.2	Test Requirements	1124

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1125

A.4.7.3.1	Test Purpose and Environment	1125

A.4.7.3.2	Test Requirements	1129

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1130

A.4.7.4.1	Test Purpose and Environment	1130

A.4.7.4.2	Test Requirements	1134

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1135

A.5.1	E-UTRAN Handover	1135

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1135

A.5.1.1.1	Test Purpose and Environment	1135

A.5.1.1.2	Test Requirements	1137

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1137

A.5.1.2.1	Test Purpose and Environment	1137

A.5.1.2.2	Test Requirements	1139

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1139

A.5.1.3.1	Test Purpose and Environment	1139

A.5.1.3.2	Test Requirements	1141

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1141

A.5.1.4.1	Test Purpose and Environment	1141

A.5.1.4.2	Test Requirements	1143

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1143

A.5.1.5.1	Test Purpose and Environment	1143

A.5.1.5.2	Test Requirements	1145

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1145

A.5.1.6.1	Test Purpose and Environment	1145

A.5.1.6.2	Test Requirements	1147

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1147

A.5.1.7.1	Test Purpose and Environment	1147

A.5.1.7.2	Test Requirements	1150

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1150

A.5.1.8.1	Test Purpose and Environment	1150

A.5.1.8.2	Test Requirements	1153

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1153

A.5.1.9.1	Test Purpose and Environment	1153

A.5.1.9.2	Test Requirements	1154

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1154

A.5.1.10.1	Test Purpose and Environment	1154

A.5.1.10.2	Test Requirements	1156

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1156

A.5.1.11.1	Test Purpose and Environment	1156

A.5.1.11.2	Test Requirements	1158

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1158

A.5.1.12.1	Test Purpose and Environment	1158

A.5.1.12.2	Test Requirements	1160

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1160

A.5.1.13.1	Test Purpose and Environment	1160

A.5.1.13.2	Test Requirements	1162

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1163

A.5.1.14.1	Test Purpose and Environment	1163

A.5.1.14.2	Test Requirements	1164

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1165

A.5.1.15.1	Test Purpose and Environment	1165

A.5.1.15.2	Test Requirements	1166

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1167

A.5.1.16.1	Test Purpose and Environment	1167

A.5.1.16.2	Test Requirements	1168

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1169

A.5.1.17.1	Test Purpose and Environment	1169

A.5.1.17.2	Test Requirements	1170

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1171

A.5.1.18.1	Test Purpose and Environment	1171

A.5.1.18.2	Test Requirements	1172

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1173

A.5.1.19.1	Test Purpose and Environment	1173

A.5.1.19.2	Test Requirements	1174

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1175

A.5.1.20.1	Test Purpose and Environment	1175

A.5.1.20.2	Test Requirements	1176

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1177

A.5.1.21.1	Test Purpose and Environment	1177

A.5.1.21.2	Test Requirements	1178

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1178

A.5.1.22.1	Test Purpose and Environment	1178

A.5.1.22.2	Test Requirements	1180

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1180

A.5.1.23.1	Test Purpose and Environment	1180

A.5.1.23.2	Test Requirements	1182

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1182

A.5.1.24.1	Test Purpose and Environment	1182

A.5.1.24.2	Test Requirements	1184

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1184

A.5.1.25.1	Test Purpose and Environment	1184

A.5.1.25.2	Test Requirements	1186

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1187

A.5.1.26.1	Test Purpose and Environment	1187

A.5.1.26.2	Test Requirements	1188

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1189

A.5.1.27.1	Test Purpose and Environment	1189

A.5.1.27.2	Test Requirements	1190

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1191

A.5.1.28.1	Test Purpose and Environment	1191

A.5.1.28.2	Test Requirements	1192

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1193

A.5.1.29.1	Test Purpose and Environment	1193

A.5.1.29.2	Test Requirements	1194

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1195

A.5.1.30.1	Test Purpose and Environment	1195

A.5.1.30.2	Test Requirements	1196

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1197

A.5.1.31.1	Test Purpose and Environment	1197

A.5.1.31.2	Test Requirements	1198

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1199

A.5.1.32.1	Test Purpose and Environment	1199

A.5.1.32.2	Test Requirements	1200

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1201

A.5.1.33.1	Test Purpose and Environment	1201

A.5.1.13.2	Test Requirements	1202

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1203

A.5.1.34.1	Test Purpose and Environment	1203

A.5.1.34.2	Test Requirements	1204

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1205

A.5.1.35.1	Test Purpose and Environment	1205

A.5.1.35.2	Test Requirements	1206

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1207

A.5.1.36.1	Test Purpose and Environment	1207

A.5.1.36.2	Test Requirements	1208

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1209

A.5.1.37.1	Test Purpose and Environment	1209

A.5.1.37.2	Test Requirements	1210

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1211

A.5.1.38.1	Test Purpose and Environment	1211

A.5.1.38.2	Test Requirements	1212

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1213

A.5.1.39.1	Test Purpose and Environment	1213

A.5.1.39.2	Test Requirements	1214

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1215

A.5.1.40.1	Test Purpose and Environment	1215

A.5.1.40.2	Test Requirements	1216

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1217

A.5.1.41.1	Test Purpose and Environment	1217

A.5.1.41.2	Test Requirements	1219

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1220

A.5.1.42.1	Test Purpose and Environment	1220

A.5.1.42.2	Test Requirements	1221

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1222

A.5.1.43.1	Test Purpose and Environment	1222

A.5.1.43.2	Test Requirements	1223

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1224

A.5.1.44.1	Test Purpose and Environment	1224

A.5.1.44.2	Test Requirements	1225

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1226

A.5.1.45.1	Test Purpose and Environment	1226

A.5.1.45.1	Test Requirements	1227

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1228

A.5.1.46.1	Test Purpose and Environment	1228

A.5.1.46.2	Test Requirements	1229

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1230

A.5.1.47.1	Test Purpose and Environment	1230

A.5.1.47.2	Test Requirements	1231

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1232

A.5.1.48.1	Test Purpose and Environment	1232

A.5.1.48.2	Test Requirements	1233

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1234

A.5.1.49.1	Test Purpose and Environment	1234

A.5.1.49.2	Test Requirements	1235

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1236

A.5.1.50.1	Test Purpose and Environment	1236

A.5.1.50.2	Test Requirements	1237

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1238

A.5.1.51.1	Test Purpose and Environment	1238

A.5.1.51.2	Test Requirements	1240

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1240

A.5.152.1	Test Purpose and Environment	1240

A.5.1.52.2	Test Requirements	1243

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1244

A.5.1.53.1	Test Purpose and Environment	1244

A.5.1.53.2	Test Requirements	1246

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1247

A.5.1.54.1	Test Purpose and Environment	1247

A.5.1.54.2	Test Requirements	1249

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1250

A.5.1.55.1	Test Purpose and Environment	1250

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1254

A.5.1.56.1	Test Purpose and Environment	1254

A.5.1.56.2	Test Requirements	1257

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1258

A.5.1.57.1	Test Purpose and Environment	1258

A.5.1.57.2	Test Requirements	1260

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1260

A.5.1.58.1	Test Purpose and Environment	1260

A.5.1.58.2	Test Requirements	1263

A.5.2	E-UTRAN Handover to other RATs	1263

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1263

A.5.2.1.1	Test Purpose and Environment	1263

A.5.2.1.2	Test Requirements	1265

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1266

A.5.2.2.1	Test Purpose and Environment	1266

A.5.2.2.2	Test Requirements	1269

A.5.2.3 E-UTRAN FDD- GSM Handover	1269

A.5.2.3.1	Test Purpose and Environment	1269

A.5.2.3.2	Test Requirements	1270

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1271

A.5.2.4.1	Test Purpose and Environment	1271

A.5.2.4.1.1	Void	1271

A.5.2.4.1.2	1.28 Mcps TDD option	1271

A.5.2.4.1.3	Void	1273

A.5.2.4.2	Test Requirements	1273

A.5.2.4.2.1	Void	1273

A.5.2.4.2.2	1.28 Mcps TDD option	1273

A.5.2.4.2.3	Void	1273

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1273

A.5.2.5.1	Test Purpose and Environment	1273

A.5.2.5.1.3	Void	1276

A.5.2.5.2	Test Requirements	1276

A.5.2.5.2.1	Void	1276

A.5.2.5.2.2	1.28 Mcps TDD option	1276

A.5.2.5.2.3	Void	1276

A.5.2.6	E-UTRAN TDD - GSM Handover	1276

A.5.2.6.1	Test Purpose and Environment	1276

A.5.2.6.2	Test Requirements	1278

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1279

A.5.2.7.1	Test Purpose and Environment	1279

A.5.2.7.2	Test Requirements	1281

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1281

A.5.2.8.1	Test Purpose and Environment	1281

A.5.2.8.2	Test Requirements	1282

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1283

A.5.2.9.1	Test Purpose and Environment	1283

A.5.2.9.2	Test Requirements	1284

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1285

A.5.2.10.1	Test Purpose and Environment	1285

A.5.2.10.2	Test Requirements	1287

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1287

A.5.2.10A.1	Test Purpose and Environment	1287

A.5.2.10A.2	Test Requirements	1290

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1290

A.5.2.10B.1	Test Purpose and Environment	1290

A.5.2.10B.2	Test Requirements	1293

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1293

A.5.2.11.1	Test Purpose and Environment	1293

A.5.2.11.2	Test Requirements	1293

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1294

A.5.3.1	E-UTRAN FDD – HRPD Handover	1294

A.5.3.1.1	Test Purpose and Environment	1294

A.5.3.1.2	Test Requirements	1296

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1296

A.5.3.2.1	Test Purpose and Environment	1296

A.5.3.2.2	Test Requirements	1299

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1299

A.5.3.3.1	Test Purpose and Environment	1299

A.5.3.3.2	Test Requirements	1302

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1302

A.5.3.4.1	Test Purpose and Environment	1302

A.5.3.4.2	Test Requirements	1303

A.5.3.5	E-UTRAN TDD – HRPD Handover	1304

A.5.3.5.1	Test Purpose and Environment	1304

A.5.3.5.2	Test Requirements	1306

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1307

A.5.3.6.1	Test Purpose and Environment	1307

A.5.3.6.2	Test Requirements	1310

A.6	RRC Connection Control	1310

A.6.1	RRC Re-establishment	1310

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1310

A.6.1.1.1	Test Purpose and Environment	1310

A.6.1.1.2	Test Requirements	1311

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1312

A.6.1.2.1	Test Purpose and Environment	1312

A.6.1.2.2	Test Requirements	1313

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1314

A.6.1.3.1	Test Purpose and Environment	1314

A.6.1.3.2	Test Requirements	1315

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1316

A.6.1.4.1	Test Purpose and Environment	1316

A.6.1.4.2	Test Requirements	1317

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1318

A.6.1.5.1	Test Purpose and Environment	1318

A.6.1.5.2	Test Requirements	1318

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1318

A.6.1.6.1	Test Purpose and Environment	1318

A.6.1.6.2	Test Requirements	1320

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1321

A.6.1.7.1	Test Purpose and Environment	1321

A.6.1.7.2	Test Requirements	1322

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1323

A.6.1.8.1	Test Purpose and Environment	1323

A.6.1.8.2	Test Requirements	1324

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1325

A.6.1.9.1	Test Purpose and Environment	1325

A.6.1.9.2	Test Requirements	1326

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1327

A.6.1.10.1	Test Purpose and Environment	1327

A.6.1.10.2	Test Requirements	1328

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1329

A.6.1.11.1	Test Purpose and Environment	1329

A.6.1.11.2	Test Requirements	1330

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1331

A.6.1.12.1	Test Purpose and Environment	1331

A.6.1.12.2	Test Requirements	1332

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1333

A.6.1.13.1	Test Purpose and Environment	1333

A.6.1.13.2	Test Requirements	1334

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1335

A.6.1.14.1	Test Purpose and Environment	1335

A.6.1.14.2	Test Requirements	1336

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1337

A.6.1.15.1	Test Purpose and Environment	1337

A.6.1.15.2	Test Requirements	1339

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1340

A.6.1.16.1	Test Purpose and Environment	1340

A.6.1.16.2	Test Requirements	1342

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1343

A.6.1.17.1	Test Purpose and Environment	1343

A.6.1.17.2	Test Requirements	1344

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1345

A.6.1.18.1	Test Purpose and Environment	1345

A.6.1.18.2	Test Requirements	1346

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1347

A.6.1.19.1	Test Purpose and Environment	1347

A.6.1.19.2	Test Requirements	1348

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1349

A.6.1.20.1	Test Purpose and Environment	1349

A.6.1.20.2	Test Requirements	1350

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1351

A.6.1.21.1	Test Purpose and Environment	1351

A.6.1.21.2	Test Requirements	1352

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1353

A.6.1.22.1	Test Purpose and Environment	1353

A.6.1.22.2	Test Requirements	1354

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1355

A.6.1.23.1	Test Purpose and Environment	1355

A.6.1.23.2	Test Requirements	1357

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1358

A.6.1.24.1	Test Purpose and Environment	1358

A.6.1.24.2	Test Requirements	1360

A.6.2	Random Access	1361

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1361

A.6.2.1.1	Test Purpose and Environment	1361

A.6.2.1.2	Test Requirements	1362

A.6.2.1.2.1	Random Access Response Reception	1362

A.6.2.1.2.2	No Random Access Response Reception	1362

A.6.2.1.2.3	Receiving a NACK on msg3	1362

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1363

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1363

A.6.2.1.2.6	Contention Resolution Timer expiry	1363

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1363

A.6.2.2.1	Test Purpose and Environment	1363

A.6.2.2.2	Test Requirements	1364

A.6.2.2.2.1	Random Access Response Reception	1365

A.6.2.2.2.2	No Random Access Response Reception	1365

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1365

A.6.2.3.1	Test Purpose and Environment	1365

A.6.2.3.2	Test Requirements	1367

A.6.2.3.2.1	Random Access Response Reception	1367

A.6.2.3.2.2	No Random Access Response reception	1367

A.6.2.3.2.3	Receiving a NACK on msg3	1367

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1368

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1368

A.6.2.3.2.6	Contention Resolution Timer expiry	1368

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1368

A.6.2.4.1	Test Purpose and Environment	1368

A.6.2.4.2	Test Requirements	1370

A.6.2.4.2.1	Random Access Response Reception	1370

A.6.2.4.2.2	No Random Access Response Reception	1370

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1370

A.6.2.5.1	Test Purpose and Environment	1370

A.6.2.5.2	Test Requirements	1371

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1371

A.6.2.6.1	Test Purpose and Environment	1371

A.6.2.6.2	Test Requirements	1371

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1372

A.6.2.7.1	Test Purpose and Environment	1372

A.6.2.7.2	Test Requirements	1374

A.6.2.7.2.1	Random Access Response Reception	1374

A.6.2.7.2.2	No Random Access Response Reception	1374

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1374

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1375

A.6.2.8.1	Test Purpose and Environment	1375

A.6.2.8.2	Test Requirements	1377

A.6.2.8.2.1	Random Access Response Reception	1377

A.6.2.8.2.2	No Random Access Response Reception	1377

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1377

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1378

A.6.2.9.1	Test Purpose and Environment	1378

A.6.2.9.2	Test Requirements	1381

A.6.2.9.2.1	Random Access Response Reception	1381

A.6.2.9.2.2	No Random Access Response Reception	1382

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1382

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1383

A.6.2.10.1	Test Purpose and Environment	1383

A.6.2.10.2	Test Requirements	1385

A.6.2.10.2.1	Random Access Response Reception	1385

A.6.2.10.2.2	No Random Access Response Reception	1386

A.6.2.10.2.3	Receiving a NACK on msg3	1386

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1386

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1386

A.6.2.10.2.6	Contention Resolution Timer expiry	1386

A.6.2.10.2.7	PRACH Resource Selection	1387

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1387

A.6.2.11.1	Test Purpose and Environment	1387

A.6.2.11.2	Test Requirements	1389

A.6.2.11.2.1	Random Access Response Reception	1389

A.6.2.11.2.2	No Random Access Response Reception	1390

A.6.2.11.2.3	Receiving a NACK on msg3	1390

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1390

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1390

A.6.2.11.2.6	Contention Resolution Timer expiry	1390

A.6.2.11.2.7	PRACH Resource Selection	1391

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1391

A.6.2.12.1	Test Purpose and Environment	1391

A.6.2.12.2	Test Requirements	1393

A.6.2.12.2.1	Random Access Response Reception	1393

A.6.2.12.2.2	No Random Access Response Reception	1394

A.6.2.12.2.3	Receiving a NACK on msg3	1394

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1394

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1394

A.6.2.12.2.6	Contention Resolution Timer expiry	1394

A.6.2.12.2.7	PRACH Resource Selection	1395

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1395

A.6.2.13.1	Test Purpose and Environment	1395

A.6.2.13.2	Test Requirements	1397

A.6.2.13.2.1	Random Access Response Reception	1397

A.6.2.13.2.2	No Random Access Response Reception	1398

A.6.2.13.2.3	Receiving a NACK on msg3	1398

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1398

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1398

A.6.2.13.2.6	Contention Resolution Timer expiry	1398

A.6.2.13.2.7	PRACH Resource Selection	1399

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1399

A.6.2.14.1	Test Purpose and Environment	1399

A.6.2.14.2	Test Requirements	1401

A.6.2.14.2.1	Random Access Response Reception	1401

A.6.2.14.2.2	No Random Access Response Reception	1402

A.6.2.14.2.3	Receiving a NACK on msg3	1402

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1402

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1402

A.6.2.14.2.6	Contention Resolution Timer expiry	1402

A.6.2.14.2.7	PRACH Resource Selection	1403

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1403

A.6.2.15.1	Test Purpose and Environment	1403

A.6.2.15.2	Test Requirements	1405

A.6.2.15.2.1	Random Access Response Reception	1405

A.6.2.15.2.2	No Random Access Response Reception	1406

A.6.2.15.2.3	Receiving a NACK on msg3	1406

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1406

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1406

A.6.2.15.2.6	Contention Resolution Timer expiry	1406

A.6.2.15.2.7	PRACH Resource Selection	1407

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1407

A.6.2.16.1	Test Purpose and Environment	1407

A.6.2.16.2	Test Requirements	1410

A.6.2.16.2.1	Random Access Response Reception	1410

A.6.2.16.2.2	No Random Access Response Reception	1411

A.6.2.16.2.3	Receiving a NACK on msg3	1411

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1411

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1411

A.6.2.16.2.6	Contention Resolution Timer expiry	1411

A.6.2.16.2.7	NPRACH Resource Selection	1411

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1412

A.6.2.17.1	Test Purpose and Environment	1412

A.6.2.17.2	Test Requirements	1415

A.6.2.17.2.1	Random Access Response Reception	1415

A.6.2.17.2.2	No Random Access Response Reception	1416

A.6.2.17.2.3	Receiving a NACK on msg3	1416

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1416

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1416

A.6.2.17.2.6	Contention Resolution Timer expiry	1416

A.6.2.17.2.7	NPRACH Resource Selection	1416

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1417

A.6.2.18.1	Test Purpose and Environment	1417

A.6.2.18.2	Test Requirements	1420

A.6.2.18.2.1	Random Access Response Reception	1420

A.6.2.18.2.2	No Random Access Response Reception	1421

A.6.2.18.2.3	Receiving a NACK on msg3	1421

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1421

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1421

A.6.2.18.2.6	Contention Resolution Timer expiry	1421

A.6.2.18.2.7	NPRACH Resource Selection	1421

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1422

A.6.2.19.1	Test Purpose and Environment	1422

A.6.2.19.2	Test Requirements	1424

A.6.2.19.2.1	Random Access Response Reception	1424

A.6.2.19.2.2	No Random Access Response Reception	1425

A.6.2.19.2.3	Receiving a NACK on msg3	1425

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1425

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1425

A.6.2.19.2.6	Contention Resolution Timer expiry	1425

A.6.2.19.2.7	NPRACH Resource Selection	1425

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1426

A.6.2.20.1	Test Purpose and Environment	1426

A.6.2.20.2	Test Requirements	1428

A.6.2.20.2.1	Random Access Response Reception	1428

A.6.2.20.2.2	No Random Access Response Reception	1429

A.6.2.20.2.3	Receiving a NACK on msg3	1429

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1429

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1429

A.6.2.20.2.6	Contention Resolution Timer expiry	1429

A.6.2.20.2.7	NPRACH Resource Selection	1429

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1430

A.6.2.21.1	Test Purpose and Environment	1430

A.6.2.21.2	Test Requirements	1432

A.6.2.21.2.1	Random Access Response Reception	1432

A.6.2.21.2.2	No Random Access Response Reception	1433

A.6.2.21.2.3	Receiving a NACK on msg3	1433

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1433

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1433

A.6.2.21.2.6	Contention Resolution Timer expiry	1433

A.6.2.21.2.7	NPRACH Resource Selection	1433

A.6.3	RRC Connection Release with Redirection	1434

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1434

A.6.3.1.1	Test Purpose and Environment	1434

A.6.3.1.2	Test Requirements	1436

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1436

A.6.3.2.1	Test Purpose and Environment	1436

A.6.3.2.2	Test Requirements	1438

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1438

A.6.3.3.1	Test Purpose and Environment	1438

A.6.3.3.2	Test Requirements	1439

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1440

A.6.3.4.1	Test Purpose and Environment	1440

A.6.3.4.2	Test Requirements	1441

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1442

A.6.3.5.1	Test Purpose and Environment	1442

A.6.3.5.2	Test Requirements	1444

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1444

A.6.3.6.1	Test Purpose and Environment	1444

A. 6.3.6.2	Test Requirements	1447

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1447

A.6.3.7.1	Test Purpose and Environment	1447

A.6.3.7.2	Test Requirements	1450

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1450

A.6.3.8.1	Test Purpose and Environment	1450

A.6.3.8.2	Test Requirements	1453

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1453

A.6.3.9.1	Test Purpose and Environment	1453

A.6.3.9.2	Test Requirements	1455

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1455

A.6.3.10.1	Test Purpose and Environment	1455

A.6.3.10.2	Test Requirements	1457

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1457

A.6.3.11.1	Test Purpose and Environment	1457

A.6.3.11.2	Test Requirements	1459

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1459

A.6.3.12.1	Test Purpose and Environment	1459

A.6.3.12.2	Test Requirements	1462

A.7	Timing and Signalling Characteristics	1443

A.7.1	UE Transmit Timing	1443

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1443

A.7.1.1.1	Test Purpose and Environment	1443

A.7.1.1.2	Test Requirements	1444

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1445

A.7.1.2.1	Test Purpose and Environment	1445

A.7.1.2.2	Test Requirements	1447

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1448

A.7.1.3.1	Test Purpose and Environment	1448

A.7.1.3.2	Test Requirements	1450

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1451

A.7.1.4.1	Test Purpose and Environment	1451

A.7.1.4.2	Test Requirements	1452

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1453

A.7.1.4A.1	Test Purpose and Environment	1453

A.7.1.4A.2	Test Requirements	1453

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1453

A.7.1.5.1	Test Purpose and Environment	1453

A.7.1.5.2	Test Requirements	1454

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1454

A.7.1.6.1	Test Purpose and Environment	1454

A.7.1.6.2	Test Requirements	1456

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1457

A.7.1.7.1	Test Purpose and Environment	1457

A.7.1.7.2	Test Requirements	1458

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1459

A.7.1.7A.1	Test Purpose and Environment	1459

A.7.1.7A.2	Test Requirements	1459

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1459

A.7.1.7B.1	Test Purpose and Environment	1459

A.7.1.7B.2	Test Requirements	1459

A.7.1.8	Void	1460

A.7.1.8.1	Void	1460

A.7.1.8.2	Void	1460

A.7.1.9	Void	1460

A.7.1.9.1	Void	1460

A.7.1.9.2	Void	1460

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1460

A.7.1.10.1	Test Purpose and Environment	1460

A.7.1.10.2	Test Requirements	1462

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1462

A.7.1.11.1	Test Purpose and Environment	1462

A.7.1.11.2	Test Requirements	1464

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1464

A.7.1.12.1	Test Purpose and Environment	1464

A.7.1.12.2	Test Requirements	1466

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1466

A.7.1.13.1	Test Purpose and Environment	1466

A.7.1.13.2	Test Requirements	1468

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1469

A.7.1.14.1	Test Purpose and Environment	1469

A.7.1.14.2	Test Requirements	1470

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1470

A.7.1.15.1	Test Purpose and Environment	1470

A.7.1.15.2	Test Requirements	1471

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1472

A.7.1.16.1	Test Purpose and Environment	1472

A.7.1.16.2	Test Requirements	1473

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1474

A.7.1.17.1	Test Purpose and Environment	1474

A.7.1.17.2	Test Requirements	1476

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1477

A.7.1.18.1	Test Purpose and Environment	1477

A.7.1.18.2	Test Requirements	1479

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1480

A.7.1.19.1	Test Purpose and Environment	1480

A.7.1.19.2	Test Requirements	1481

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1481

A.7.1.20.1	Test Purpose and Environment	1481

A.7.1.20.2	Test Requirements	1483

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1483

A.7.1.21.1	Test Purpose and Environment	1483

A.7.1.21.2	Test Requirements	1485

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1485

A.7.1.22.1	Test Purpose and Environment	1485

A.7.1.22.2	Test Requirements	1487

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1487

A.7.1.23.1	Test Purpose and Environment	1487

A.7.1.23.2	Test Requirements	1489

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1489

A.7.1.24.1	Test Purpose and Environment	1489

A.7.1.24.2	Test Requirements	1490

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1491

A.7.1.25.1	Test Purpose and Environment	1491

A.7.1.25.2	Test Requirements	1492

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1493

A.7.1.26.1	Test Purpose and Environment	1493

A.7.1.26.2	Test Requirements	1494

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1495

A.7.1.27.1	Test Purpose and Environment	1495

A.7.1.27.2	Test Requirements	1497

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1497

A.7.1.28.1	Test Purpose and Environment	1497

A.7.1.28.2	Test Requirements	1499

A.7.2	UE Timing Advance	1500

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1500

A.7.2.1.1	Test Purpose and Environment	1500

A.7.2.1.2	Test Requirements	1501

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1502

A.7.2.2.1	Test Purpose and Environment	1502

A.7.2.2.2	Test Requirements	1504

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1504

A.7.2.3.1	Test Purpose and Environment	1504

A.7.2.3.2	Test Requirements	1504

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1504

A.7.2.4.1	Test Purpose and Environment	1504

A.7.2.4.2	Test Requirements	1507

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1507

A.7.2.5.1	Test Purpose and Environment	1507

A.7.2.5.2	Test Requirements	1509

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1509

A.7.2.5A.1	Test Purpose and Environment	1509

A.7.2.5A.2	Test Requirements	1510

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1510

A.7.2.5B.1	Test Purpose and Environment	1510

A.7.2.5B.2	Test Requirements	1510

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1510

A.7.2.6.1	Test Purpose and Environment	1510

A.7.2.6.2	Test Requirements	1513

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1513

A.7.2.7.1	Test Purpose and Environment	1513

A.7.2.7.2	Test Requirements	1515

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1515

A.7.2.8.1	Test Purpose and Environment	1515

A.7.2.8.2	Test Requirements	1517

A.7.2.9.2	Test Requirements	1518

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1519

A.7.2.10.1	Test Purpose and Environment	1519

A.7.2.10.2	Test Requirements	1520

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1520

A.7.2.11.1	Test Purpose and Environment	1520

A.7.2.11.2	Test Requirements	1522

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1522

A.7.2.12.1	Test Purpose and Environment	1522

A.7.2.12.2	Test Requirements	1524

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1525

A.7.2.13.1	Test Purpose and Environment	1525

A.7.2.13.2	Test Requirements	1526

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1527

A.7.2.14.1	Test Purpose and Environment	1527

A.7.2.14.2	Test Requirements	1529

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1529

A.7.2.15.1	Test Purpose and Environment	1529

A.7.2.15.2	Test Requirements	1531

A.7.3	Radio Link Monitoring	1531

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1531

A.7.3.1.1	Test Purpose and Environment	1531

A.7.3.1.2	Test Requirements	1535

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1535

A.7.3.2.1	Test Purpose and Environment	1535

A.7.3.2.2	Test Requirements	1539

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1539

A.7.3.3.1	Test Purpose and Environment	1539

A.7.3.3.2	Test Requirements	1543

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1543

A.7.3.4.1	Test Purpose and Environment	1543

A.7.3.4.2	Test Requirements	1547

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1547

A.7.3.5.1	Test Purpose and Environment	1547

A.7.3.5.2	Test Requirements	1550

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1551

A.7.3.6.1	Test Purpose and Environment	1551

A.7.3.6.2	Test Requirements	1554

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1554

A.7.3.7.1	Test Purpose and Environment	1554

A.7.3.7.2	Test Requirements	1557

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1557

A.7.3.8.1	Test Purpose and Environment	1557

A.7.3.8.2	Test Requirements	1560

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1560

A.7.3.9.2	Test Requirements	1563

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1563

A.7.3.10.1	Test Purpose and Environment	1563

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1567

A.7.3.11.1	Test Purpose and Environment	1567

A.7.3.11.2	Test Requirements	1572

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1572

A.7.3.12.1	Test Purpose and Environment	1572

A.7.3.12.2	Test Requirements	1577

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1577

A.7.3.13.1	Test Purpose and Environment	1577

A.7.3.13.2	Test Requirements	1580

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1580

A.7.3.14.1	Test Purpose and Environment	1580

A.7.3.14.2	Test Requirements	1583

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1583

A.7.3.15.1	Test Purpose and Environment	1583

A.7.3.15.2	Test Requirements	1587

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1587

A.7.3.16.1	Test Purpose and Environment	1587

A.7.3.16.2	Test Requirements	1592

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1592

A.7.3.17.1	Test Purpose and Environment	1592

A.7.3.17.2	Test Requirements	1596

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1596

A.7.3.18.1	Test Purpose and Environment	1596

A.7.3.18.2	Test Requirements	1600

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1600

A.7.3.19.1	Test Purpose and Environment	1600

A.7.3.19.2	Test Requirements	1605

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1605

A.7.3.20.1	Test Purpose and Environment	1605

A.7.3.20.2	Test Requirements	1610

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1610

A.7.3.21.1	Test Purpose and Environment	1610

A.7.3.21.2	Test Requirements	1615

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1615

A.7.3.22.1	Test Purpose and Environment	1615

A.7.3.22.2	Test Requirements	1620

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1620

A.7.3.23.1	Test Purpose and Environment	1620

A.7.3.23.2	Test Requirements	1621

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1621

A.7.3.24.1	Test Purpose and Environment	1621

A.7.3.24.2	Test Requirements	1622

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1622

A.7.3.25.1	Test Purpose and Environment	1622

A.7.3.25.2	Test Requirements	1623

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1623

A.7.3.26.1	Test Purpose and Environment	1623

A.7.3.26.2	Test Requirements	1626

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1626

A.7.3.27.1	Test Purpose and Environment	1626

A.7.3.27.2	Test Requirements	1629

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1629

A.7.3.28.1	Test Purpose and Environment	1629

A.7.3.28.2	Test Requirements	1632

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1632

A.7.3.29.1	Test Purpose and Environment	1632

A.7.3.29.2	Test Requirements	1635

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1635

A.7.3.30.1	Test Purpose and Environment	1635

A.7.3.30.2	Test Requirements	1638

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1638

A.7.3.31.1	Test Purpose and Environment	1638

A.7.3.31.2	Test Requirements	1641

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1641

A.7.3.32.1	Test Purpose and Environment	1641

A.7.3.32.2	Test Requirements	1644

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1644

A.7.3.33.1	Test Purpose and Environment	1644

A.7.3.33.2	Test Requirements	1647

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1647

A.7.3.34.1	Test Purpose and Environment	1647

A.7.3.34.2	Test Requirements	1650

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1650

A.7.3.35.1	Test Purpose and Environment	1650

A.7.3.35.2	Test Requirements	1653

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1653

A.7.3.36.1	Test Purpose and Environment	1653

A.7.3.36.2	Test Requirements	1656

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1656

A.7.3.37.1	Test Purpose and Environment	1656

A.7.3.37.2	Test Requirements	1659

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1659

A.7.3.38.1	Test Purpose and Environment	1659

A.7.3.38.2	Test Requirements	1662

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1662

A.7.3.39.1	Test Purpose and Environment	1662

A.7.3.39.2	Test Requirements	1665

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1666

A.7.3.40.1	Test Purpose and Environment	1666

A.7.3.40.2	Test Requirements	1668

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1669

A.7.3.41.1	Test Purpose and Environment	1669

A.7.3.41.2	Test Requirements	1672

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1672

A.7.3.42.1	Test Purpose and Environment	1672

A.7.3.42.2	Test Requirements	1675

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1675

A.7.3.43.1	Test Purpose and Environment	1675

A.7.3.43.2	Test Requirements	1678

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1678

A.7.3.44.1	Test Purpose and Environment	1678

A.7.3.44.2	Test Requirements	1681

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1682

A.7.3.45.1	Test Purpose and Environment	1682

A.7.3.45.2	Test Requirements	1684

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1685

A.7.3.46.1	Test Purpose and Environment	1685

A.7.3.46.2	Test Requirements	1688

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1688

A.7.3.47.1	Test Purpose and Environment	1688

A.7.3.47.2	Test Requirements	1691

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1691

A.7.3.48.1	Test Purpose and Environment	1691

A.7.3.48.2	Test Requirements	1694

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1694

A.7.3.49.1	Test Purpose and Environment	1694

A.7.3.49.2	Test Requirements	1697

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1697

A.7.3.50.1	Test Purpose and Environment	1697

A.7.3.50.2	Test Requirements	1700

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1700

A.7.3.51.1	Test Purpose and Environment	1700

A.7.3.51.2	Test Requirements	1703

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1703

A.7.3.52.1	Test Purpose and Environment	1703

A.7.3.52.2	Test Requirements	1706

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1706

A.7.3.53.1	Test Purpose and Environment	1706

A.7.3.53.2	Test Requirements	1709

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1709

A.7.3.54.1	Test Purpose and Environment	1709

A.7.3.54.2	Test Requirements	1712

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1712

A.7.3.55.1	Test Purpose and Environment	1712

A.7.3.55.2	Test Requirements	1715

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1715

A.7.3.56.1	Test Purpose and Environment	1715

A.7.3.56.2	Test Requirements	1718

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1718

A.7.3.57.1	Test Purpose and Environment	1718

A.7.3.57.2	Test Requirements	1721

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1721

A.7.3.58.1	Test Purpose and Environment	1721

A.7.3.58.2	Test Requirements	1724

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1724

A.7.3.59.1	Test Purpose and Environment	1724

A.7.3.59.2	Test Requirements	1727

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1727

A.7.3.60.1	Test Purpose and Environment	1727

A.7.3.60.2	Test Requirements	1731

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1731

A.7.3.61.1	Test Purpose and Environment	1731

A.7.3.61.2	Test Requirements	1735

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1735

A.7.3.62.1	Test Purpose and Environment	1735

A.7.3.62.2	Test Requirements	1739

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1739

A.7.3.63.1	Test Purpose and Environment	1739

A.7.3.63.2	Test Requirements	1743

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1743

A.7.3.64.1	Test Purpose and Environment	1743

A.7.3.64.2	Test Requirements	1746

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1747

A.7.3.65.1	Test Purpose and Environment	1747

A.7.3.65.2	Test Requirements	1750

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1751

A.7.3.66.1	Test Purpose and Environment	1751

A.7.3.66.2	Test Requirements	1753

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1754

A.7.3.67.1	Test Purpose and Environment	1754

A.7.3.67.2	Test Requirements	1757

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1758

A.7.3.68.1	Test Purpose and Environment	1758

A.7.3.68.2	Test Requirements	1760

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1760

A.7.3.69.1	Test Purpose and Environment	1760

A.7.3.69.2	Test Requirements	1762

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1762

A.7.3.70.1	Test Purpose and Environment	1762

A.7.3.70.2	Test Requirements	1764

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1764

A.7.3.71.1	Test Purpose and Environment	1764

A.7.3.71.2	Test Requirements	1767

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1767

A.7.3.72.1	Test Purpose and Environment	1767

A.7.3.72.2	Test Requirements	1769

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1769

A.7.3.73.1	Test Purpose and Environment	1769

A.7.3.73.2	Test Requirements	1771

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1771

A.7.3.74.1	Test Purpose and Environment	1771

A.7.3.74.2	Test Requirements	1774

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1774

A.7.3.75.1	Test Purpose and Environment	1774

A.7.3.75.2	Test Requirements	1776

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1777

A.7.3.76.1	Test Purpose and Environment	1777

A.7.3.76.2	Test Requirements	1780

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1780

A.7.3.77.1	Test Purpose and Environment	1780

A.7.3.77.2	Test Requirements	1783

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1783

A.7.3.78.1	Test Purpose and Environment	1783

A.7.3.78.2	Test Requirements	1786

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1786

A.7.3.79.1	Test Purpose and Environment	1786

A.7.3.79.2	Test Requirements	1789

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1789

A.7.3.80.1	Test Purpose and Environment	1789

A.7.3.80.2	Test Requirements	1792

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1792

A.7.3.81.1	Test Purpose and Environment	1792

A.7.3.81.2	Test Requirements	1795

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1795

A.7.3.82.1	Test Purpose and Environment	1795

A.7.3.82.2	Test Requirements	1798

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1798

A.7.3.83.1	Test Purpose and Environment	1798

A.7.3.83.2	Test Requirements	1799

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1800

A.7.3.84.1	Test Purpose and Environment	1800

A.7.3.84.2	Test Requirements	1802

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1802

A.7.3.85.1	Test Purpose and Environment	1802

A.7.3.85.2	Test Requirements	1803

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1804

A.7.3.86.1	Test Purpose and Environment	1804

A.7.3.86.2	Test Requirements	1806

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1806

A.7.3.87.1	Test Purpose and Environment	1806

A.7.3.87.2	Test Requirements	1808

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1809

A.7.3.88.1	Test Purpose and Environment	1809

A.7.3.88.2	Test Requirements	1813

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1813

A.7.3.89.1	Test Purpose and Environment	1813

A.7.3.89.2	Test Requirements	1817

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1817

A.7.3.90.1	Test Purpose and Environment	1817

A.7.3.90.2	Test Requirements	1821

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1821

A.7.3.91.1	Test Purpose and Environment	1821

A.7.3.91.2	Test Requirements	1825

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1825

A.7.3.92.1	Test Purpose and Environment	1825

A.7.3.92.2	Test Requirements	1828

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1829

A.7.3.93.1	Test Purpose and Environment	1829

A.7.3.93.2	Test Requirements	1832

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1833

A.7.3.94.1	Test Purpose and Environment	1833

A.7.3.94.2	Test Requirements	1835

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1836

A.7.3.95.1	Test Purpose and Environment	1836

A.7.3.95.2	Test Requirements	1839

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1839

A.7.3.96.1	Test Purpose and Environment	1839

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1841

A.7.3.97.1	Test Purpose and Environment	1841

A.7.3.97.2	Test Requirements	1844

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1844

A.7.3.98.1	Test Purpose and Environment	1844

A.7.3.98.2	Test Requirements	1847

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1847

A.7.3.99.1	Test Purpose and Environment	1847

A.7.3.99.2	Test Requirements	1850

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1850

A.7.3.100.1	Test Purpose and Environment	1850

A.7.3.100.2	Test Requirements	1852

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1852

A.7.3.101.1	Test Purpose and Environment	1852

A.7.3.101.2	Test Requirements	1854

A.7.4	Interruption for Dual Connectivity	1854

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1854

A.7.4.1.1	Test Purpose and Environment	1854

A.7.4.1.2	Test Requirements	1857

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1857

A.7.4.2.1	Test Purpose and Environment	1857

A.7.4.2.2	Test Requirements	1859

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1859

A.7.4.3.1	Test Purpose and Environment	1859

A.7.4.3.2	Test Requirements	1861

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1861

A.7.4.4.1	Test Purpose and Environment	1861

A.7.4.4.2	Test Requirements	1864

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1864

A.7.4.5.1	Test Purpose and Environment	1864

A.7.4.5.2	Test Requirements	1866

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1866

A.7.4.6.1	Test Purpose and Environment	1866

A.7.4.6.2	Test Requirements	1868

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1868

A.7.4.7.1	Test Purpose and Environment	1868

A.7.4.7.2	Test Requirements	1871

A.7.5	Proximity-based Services	1871

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1871

A.7.5.1.1	Test Purpose and Environment	1871

A.7.5.1.2	Test Requirements	1872

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1873

A.7.5.2.1	Test Purpose and Environment	1873

A.7.5.1.2	Test Requirements	1873

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1874

A.7.5.3.1	Test Purpose and Environment	1874

A.7.5.3.2	Test Requirements	1875

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1876

A.7.5.4.1	Test Purpose and Environment	1876

A.7.5.4.2	Test Requirements	1877

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1878

A.7.5.5.1	Test Purpose and Environment	1878

A.7.5.5.2	Test Requirements	1880

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1881

A.7.5.6.1	Test Purpose and Environment	1881

A.7.5.6.2	Test Requirements	1882

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1883

A.7.5.7.1	Test Purpose and Environment	1883

A.7.5.7.2	Test Requirements	1885

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1885

A.7.5.8.1	Test Purpose and Environment	1885

A.7.5.8.2	Test Requirements	1887

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1888

A.7.5.9.1	Test Purpose and Environment	1888

A.7.5.9.2	Test Requirements	1890

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1891

A.7.5.10.1	Test Purpose and Environment	1891

A.7.5.10.2	Test Requirements	1893

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1894

A.7.5.11.1	Test Purpose and Environment	1894

A.7.5.11.2	Test Requirements	1896

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1896

A.7.5.12.1	Test Purpose and Environment	1896

A.7.5.12.2	Test Requirements	1900

A.7.6	Interruption for carrier aggregation	1901

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1901

A.7.6.1.1	Test Purpose and Environment	1901

A.7.6.1.2	Test Requirements	1904

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1904

A.7.6.2.1	Test Purpose and Environment	1904

A.7.6.2.2	Test Requirements	1907

A.8	UE Measurements Procedures	1907

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1907

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1907

A.8.1.1.1	Test Purpose and Environment	1907

A.8.1.1.2	Test Requirements	1909

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1910

A.8.1.2.1	Test Purpose and Environment	1910

A.8.1.2.2	Test Requirements	1911

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1912

A.8.1.3.1	Test Purpose and Environment	1912

A.8.1.3.2	Test Requirements	1914

A.8.1.4	Void	1914

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1914

A.8.1.5.1	Test Purpose and Environment	1914

A.8.1.5.2	Test Requirements	1916

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1916

A.8.1.6.1	Test Purpose and Environment	1916

A.8.1.6.2	Test Requirements	1918

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1918

A.8.1.7.1	Test Purpose and Environment	1918

A.8.1.7.2	Test Requirements	1920

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1921

A.8.1.8.1	Test Purpose and Environment	1921

A.8.1.8.2	Test Requirements	1923

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1924

A.8.1.9.1	Test Purpose and Environment	1924

A.8.1.9.2	Test Requirements	1924

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1924

A.8.1.10.1	Test Purpose and Environment	1924

A.8.1.10.2	Test Requirements	1925

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1925

A.8.1.11.1	Test Purpose and Environment	1925

A.8.1.11.2	Test Requirements	1927

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1928

A.8.1.12.1	Test Purpose and Environment	1928

A.8.1.12.2	Test Requirements	1929

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1930

A.8.1.13.1	Test Purpose and Environment	1930

A.8.1.13.2	Test Requirements	1932

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1932

A.8.1.14.1	Test Purpose and Environment	1932

A.8.1.14.2	Test Requirements	1934

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1935

A.8.1.15.1	Test Purpose and Environment	1935

A.8.1.15.2	Test Requirements	1936

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1937

A.8.1.16.1	Test Purpose and Environment	1937

A.8.1.16.2	Test Requirements	1939

A.8.1.17	Void	1939

A.8.1.18	Void	1939

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1939

A.8.1.19.1	Test Purpose and Environment	1939

A.8.1.19.2	Test Requirements	1941

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1942

A.8.1.20.1	Test Purpose and Environment	1942

A.8.1.20.2	Test Requirements	1944

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1944

A.8.1.21.1	Test Purpose and Environment	1944

A.8.1.21.2	Test Requirements	1946

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1947

A.8.1.22.1	Test Purpose and Environment	1947

A.8.1.22.2	Test Requirements	1949

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1949

A.8.1.23.1	Test Purpose and Environment	1949

A.8.1.23.2	Test Requirements	1950

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1951

A.8.1.24.1	Test Purpose and Environment	1951

A.8.1.24.2	Test Requirements	1953

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1953

A.8.1.25.1	Test Purpose and Environment	1953

A.8.1.25.2	Test Requirements	1955

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1955

A.8.1.26.1	Test Purpose and Environment	1955

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1958

A.8.1.27.1	Test Purpose and Environment	1958

A.8.1.27.2	Test Requirements	1960

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1960

A.8.1.28.1	Test Purpose and Environment	1960

A.8.1.28.2	Test Requirements	1962

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1962

A.8.1.29.1	Test Purpose and Environment	1962

A.8.1.29.2	Test Requirements	1964

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1965

A.8.1.30.1	Test Purpose and Environment	1965

A.8.1.30.2	Test Requirements	1967

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1967

A.8.1.31.1	Test Purpose and Environment	1967

A.8.1.31.2	Test Requirements	1969

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1970

A.8.1.32.1	Test Purpose and Environment	1970

A.8.1.32.2	Test Requirements	1971

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1972

A.8.1.33.1	Test Purpose and Environment	1972

A.8.1.33.2	Test Requirements	1973

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1974

A.8.1.34.1	Test Purpose and Environment	1974

A.8.1.34.2	Test Requirements	1975

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1976

A.8.1.35.1	Test Purpose and Environment	1976

A.8.1.35.2	Test Requirements	1977

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1978

A.8.1.36.1	Test Purpose and Environment	1978

A.8.1.36.2	Test Requirements	1979

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1980

A.8.1.37.1	Test Purpose and Environment	1980

A.8.1.37.2	Test Requirements	1982

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1982

A.8.1.38.1	Test Purpose and Environment	1982

A.8.1.38.2	Test Requirements	1983

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1984

A.8.1.39.1	Test Purpose and Environment	1984

A.8.1.39.2	Test Requirements	1986

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1986

A.8.1.40.1	Test Purpose and Environment	1986

A.8.1.40.2	Test Requirements	1988

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1988

A.8.1.41.1	Test Purpose and Environment	1988

A.8.1.41.2	Test Requirement	1990

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1990

A.8.1.42.1	Test Purpose and Environment	1990

A.8.1.42.2	Test Requirement	1991

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	1991

A.8.1.43.1	Test Purpose and Environment	1991

A.8.1.43.2	Test Requirements	1994

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1994

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1994

A.8.2.1.1	Test Purpose and Environment	1994

A.8.2.1.2	Test Requirements	1996

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1997

A.8.2.2.1	Test Purpose and Environment	1997

A.8.2.2.2	Test Requirements	1999

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1999

A.8.2.3.1	Test Purpose and Environment	1999

A.8.2.3.2	Test Requirements	2001

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2002

A.8.2.4.1	Test Purpose and Environment	2002

A.8.2.4.2	Test Requirements	2004

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2004

A.8.2.5.1	Test Purpose and Environment	2004

A.8.2.5.2	Test Requirements	2006

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2007

A.8.2.6.1	Test Purpose and Environment	2007

A.8.2.6.2	Test Requirements	2010

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2011

A.8.2.7.1	Test Purpose and Environment	2011

A.8.2.7.2	Test Requirements	2012

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2013

A.8.2.8.1	Test Purpose and Environment	2013

A.8.2.8.2	Test Requirements	2015

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2015

A.8.2.9.1	Test Purpose and Environment	2015

A.8.2.9.2	Test Requirements	2017

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2018

A.8.2.10.1	Test Purpose and Environment	2018

A.8.2.10.2	Test Requirements	2020

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2020

A.8.2.11.1	Test Purpose and Environment	2020

A.8.2.11.2	Test Requirements	2022

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2022

A.8.2.12.1	Test Purpose and Environment	2022

A.8.2.12.2	Test Requirements	2024

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2025

A.8.2.13.1	Test Purpose and Environment	2025

A.8.2.13.2	Test Requirements	2027

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2027

A.8.2.14.1	Test Purpose and Environment	2027

A.8.2.14.2	Test Requirement	2029

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2029

A.8.2.15.1	Test Purpose and Environment	2029

A.8.2.15.2	Test Requirements	2031

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2031

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2031

A.8.3.1.1	Test Purpose and Environment	2031

A.8.3.1.2	Test Requirements	2033

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2033

A.8.3.2.1	Test Purpose and Environment	2033

A.8.3.2.2	Test Requirements	2036

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2036

A.8.3.3.1	Test Purpose and Environment	2036

A.8.3.3.2	Test Requirements	2039

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2039

A.8.3.4.1	Test Purpose and Environment	2039

A.8.3.4.2	Test Requirements	2041

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2041

A.8.3.5.2	Test Requirements	2043

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2043

A.8.3.6.1	Test Purpose and Environment	2043

A.8.3.6.2	Test Requirements	2045

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2046

A.8.3.7.1	Test Purpose and Environment	2046

A.8.3.7.2	Test Requirements	2048

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2049

A.8.3.8.1	Test Purpose and Environment	2049

A.8.3.8.2	Test Requirements	2052

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2052

A.8.3.9.1	Test Purpose and Environment	2052

A.8.3.9.2	Test Requirements	2056

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2057

A.8.3.10.1	Test Purpose and Environment	2057

A.8.3.10.2	Test Requirements	2058

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2058

A.8.3.11.1	Test Purpose and Environment	2058

A.8.3.11.2	Test Requirement	2060

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2061

A.8.3.12.1	Test Purpose and Environment	2061

A.8.3.12.2	Test Requirement	2062

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2063

A.8.3.13.1	Test Purpose and Environment	2063

A.8.3.13.2	Test Requirement	2064

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2065

A.8.3.14.1	Test Purpose and Environment	2065

A.8.3.14.2	Test Requirement	2066

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2067

A.8.3.15.1	Test Purpose and Environment	2067

A.8.3.15.2	Test Requirement	2068

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2069

A.8.3.16.1	Test Purpose and Environment	2069

A.8.3.16.2	Test Requirement	2071

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2071

A.8.3.17.1	Test Purpose and Environment	2071

A.8.3.17.2	Test Requirement	2074

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2074

A.8.3.18.1	Test Purpose and Environment	2074

A.8.3.18.2	Test Requirement	2077

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2077

A.8.3.19.1	Test Purpose and Environment	2077

A.8.3.19.2	Test Requirement	2080

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2080

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2080

A.8.4.1.1	Test Purpose and Environment	2080

A.8.4.1.2	Test Requirements	2082

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2082

A.8.4.2.1	Test Purpose and Environment	2082

A.8.4.2.2	Test Requirements	2085

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2085

A.8.4.3.1	Test Purpose and Environment	2085

A.8.4.3.2	Test Requirements	2087

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2088

A.8.4.4.1	Test Purpose and Environment	2088

A.8.4.4.2	Test Requirements	2090

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2091

A.8.4.5.1	Test Purpose and Environment	2091

A.8.4.5.2	Test Requirements	2093

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2093

A.8.4.6.1	Test Purpose and Environment	2093

A.8.4.6.2	Test Requirements	2094

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2094

A.8.4.7.1	Test Purpose and Environment	2094

A.8.4.7.2	Test Requirements	2098

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2098

A.8.4.8.1	Test Purpose and Environment	2098

A.8.4.8.2	Test Requirements	2101

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2101

A.8.4.9.1	Test Purpose and Environment	2101

A.8.4.9.2	Test Requirements	2106

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2106

A.8.4.10.1	Test Purpose and Environment	2106

A.8.4.10.2	Test Requirements	2108

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2108

A.8.4.11.1	Test Purpose and Environment	2108

A.8.4.11.2	Test Requirement	2110

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2110

A.8.4.12.1	Test Purpose and Environment	2110

A.8.4.12.2	Test Requirement	2112

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2113

A.8.4.13.1	Test Purpose and Environment	2113

A.8.4.13.2	Test Requirement	2114

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2115

A.8.4.14.1	Test Purpose and Environment	2115

A.8.4.14.2	Test Requirement	2117

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2117

A.8.4.15.1	Test Purpose and Environment	2117

A.8.4.15.2	Test Requirement	2120

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2120

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2120

A.8.5.1.1	Test Purpose and Environment	2120

A.8.5.1.2	Test Requirements	2122

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2122

A.8.5.2.1	Test Purpose and Environment	2122

A.8.5.2.2	Test Requirements	2124

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2124

A.8.5.3.1	Test Purpose and Environment	2124

A.8.5.3.2	Test Requirements	2127

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2127

A.8.5.4.1	Test Purpose and Environment	2127

A.8.5.4.2	Test Requirements	2129

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2129

A.8.5.5.1	Test Purpose and Environment	2129

A.8.5.5.2	Test Requirements	2132

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2132

A.8.5.6.1	Test Purpose and Environment	2132

A.8.5.6.2	Test Requirements	2133

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2134

A.8.5.7.1	Test Purpose and Environment	2134

A.8.5.7.2	Test Requirements	2134

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2134

A.8.5.8.1	Test Purpose and Environment	2134

A.8.5.8.2	Test Requirements	2137

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2137

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2137

A.8.6.1.1	Test Purpose and Environment	2137

A.8.6.1.2	Test Requirements	2139

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2139

A.8.6.2.1	Test Purpose and Environment	2139

A.8.6.2.2	Test Requirements	2142

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2142

A.8.6.3.1	Test Purpose and Environment	2142

A.8.6.3.2	Test Requirements	2145

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2145

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2145

A.8.7.1.1	Test Purpose and Environment	2145

A.8.7.1.1.1	Void	2145

A.8.7.1.1.2	1.28 Mcps TDD option	2145

A.8.7.1.1.3	Void	2147

A.8.7.1.2	Test Requirements	2147

A.8.7.1.2.1	Void	2147

A.8.7.1.2.2	1.28 Mcps TDD option	2147

A.8.7.1.2.3	Void	2147

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2147

A.8.7.2.1	Test Purpose and Environment	2147

A.8.7.2.2	Test Requirements	2150

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2151

A.8.7.3.1	Test Purpose and Environment	2151

A.8.7.3.2	Test Parameters	2151

A.8.7.3.3	Test Requirements	2152

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2153

A.8.7.4.1	Test Purpose and Environment	2153

A.8.7.4.2	Test Requirements	2155

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2155

A.8.7.5.1	Test Purpose and Environment	2155

A.8.7.5.2	Test Requirements	2157

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2157

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2157

A.8.7A.1.1	Test Purpose and Environment	2157

A.8.7A.1.2	Test Requirements	2160

A.8.8	E-UTRAN FDD – GSM Measurements	2160

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2160

A.8.8.1.1	Test Purpose and Environment	2160

A.8.8.1.2	Test Requirements	2162

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2162

A.8.8.2.1	Test Purpose and Environment	2162

A.8.8.2.2	Test Requirements	2164

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2165

A.8.8.3.1	Test Purpose and Environment	2165

A.8.8.3.2	Test Requirements	2166

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2167

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2167

A.8.9.1.1	Test Purpose and Environment	2167

A.8.9.1.2	Test Requirements	2168

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2169

A.8.9.2.1	Test Purpose and Environment	2169

A.8.9.2.2	Test Requirements	2171

A.8.10	E-UTRAN TDD – GSM Measurements	2171

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2171

A.8.10.1.1	Test Purpose and Environment	2171

A.8.10.1.2	Test Requirements	2172

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2173

A.8.10.2.1	Test Purpose and Environment	2173

A.8.10.2.2	Test Requirements	2175

A.8.11	Monitoring of Multiple Layers	2175

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2175

A.8.11.1.1	Test Purpose and Environment	2175

A.8.11.1.2	Test Requirements	2177

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2178

A.8.11.2.1	Test Purpose and Environment	2178

A.8.11.2.2	Test Requirements	2179

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2180

A.8.11.3.1	Test Purpose and Environment	2180

A.8.11.3.2	Test Requirements	2182

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2182

A.8.11.4.1	Test Purpose and Environment	2182

A.8.11.4.2	Test Requirements	2185

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2185

A.8.11.5.1	Test Purpose and Environment	2185

A.8.11.5.2	Test Requirements	2187

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2188

A.8.11.6.1	Test Purpose and Environment	2188

A.8.11.6.2	Test Requirements	2190

A.8.12	RSTD Intra-frequency Measurements	2191

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2191

A.8.12.1.1	Test Purpose and Environment	2191

A.8.12.1.2	Test Requirements	2195

A.8.12.1.2A	Test Requirements for UE Category 1bis	2195

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2195

A.8.12.2.1	Test Purpose and Environment	2195

A.8.12.2.2	Test Requirements	2200

A.8.12.2.2A	Test Requirements for UE Category 1bis	2200

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2200

A.8.12.3.1	Test Purpose and Environment	2200

A.8.12.3.2	Test Requirements	2206

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2206

A.8.12.4.1	Test Purpose and Environment	2206

A.8.12.4.2	Test Requirements	2211

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2211

A.8.12.5.1	Test Purpose and Environment	2211

A.8.12.5.2	Test Requirements	2216

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2216

A.8.12.6.1	Test Purpose and Environment	2216

A.8.12.6.2	Test Requirements	2221

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2221

A.8.12.7.1	Test Purpose and Environment	2221

A.8.12.7.2	Test Requirements	2226

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2226

A.8.12.8.1	Test Purpose and Environment	2226

A.8.12.8.2	Test Requirements	2231

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2231

A.8.12.9.1	Test Purpose and Environment	2231

A.8.12.9.2	Test Requirements	2236

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2236

A.8.12.10.1	Test Purpose and Environment	2236

A.8.12.10.2	Test Requirements	2241

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2241

A.8.12.11.1	Test Purpose and Environment	2241

A.8.12.11.2	Test Requirements	2246

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2246

A.8.12.12.1	Test Purpose and Environment	2246

A.8.12.12.2	Test Requirements	2251

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2251

A.8.12.13.1	Test Purpose and Environment	2251

A.8.12.13.2	Test Requirements	2256

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2256

A.8.12.14.1	Test Purpose and Environment	2256

A.8.12.14.2	Test Requirements	2261

A.8.13	RSTD Inter-frequency Measurements	2261

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2261

A.8.13.1.1	Test Purpose and Environment	2261

A.8.13.1.2	Test Requirements	2266

A.8.13.1.2A	Test Requirements for UE Category 1bis	2266

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2266

A.8.13.2.1	Test Purpose and Environment	2266

A.8.13.2.2	Test Requirements	2272

A.8.13.2.2A	Test Requirements for UE Category 1bis	2272

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2272

A.8.13.3.1	Test Purpose and Environment	2272

A.8.13.3.2	Test Requirements	2279

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2279

A.8.13.4.1	Test Purpose and Environment	2279

A.8.13.4.2	Test Requirements	2285

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2285

A.8.13.5.1	Test Purpose and Environment	2285

A.8.13.5.2	Test Requirements	2291

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2291

A.8.13.6.1	Test Purpose and Environment	2291

A.8.13.6.2	Test Requirements	2297

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2297

A.8.13.7.1	Test Purpose and Environment	2297

A.8.13.7.2	Test Requirements	2303

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2303

A.8.13.8.1	Test Purpose and Environment	2303

A.8.13.8.2	Test Requirements	2309

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2309

A.8.13.9.1	Test Purpose and Environment	2309

A.8.13.9.2	Test Requirements	2315

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2315

A.8.13.10.1	Test Purpose and Environment	2315

A.8.13.10.2	Test Requirements	2321

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2321

A.8.13.11.1	Test Purpose and Environment	2321

A.8.13.11.2	Test Requirements	2327

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2327

A.8.13.12.1	Test Purpose and Environment	2327

A.8.13.12.2	Test Requirements	2333

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2333

A.8.13.13.1	Test Purpose and Environment	2333

A.8.13.13.2	Test Requirements	2339

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2339

A.8.13.14.1	Test Purpose and Environment	2339

A.8.13.14.2	Test Requirements	2345

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2345

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2345

A.8.14.1.1	Test Purpose and Environment	2345

A.8.14.1.2	Test Requirements	2347

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2347

A.8.14.2.1	Test Purpose and Environment	2347

A.8.14.2.2	Test Requirements	2350

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2350

A.8.14.3.1	Test Purpose and Environment	2350

A.8.14.3.2	Test Requirements	2352

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2353

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2353

A.8.15.1.1	Test Purpose and Environment	2353

A.8.15.1.2	Test Requirements	2354

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2354

A.8.15.2.1	Test Purpose and Environment	2354

A.8.15.2.2	Test Requirements	2357

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2357

A.8.15.3.1	Test Purpose and Environment	2357

A.8.15.3.2	Test Requirements	2359

A.8.16	E-UTRAN Carrier Aggregation Measurements	2360

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2360

A.8.16.1.1	Test Purpose and Environment	2360

A.8.16.1.2	Test Requirements	2362

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2362

A.8.16.2.1	Test Purpose and Environment	2362

A.8.16.2.2	Test Requirements	2364

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2364

A.8.16.3.1	Test Purpose and Environment	2364

A.8.16.3.2	Test Requirements	2366

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2367

A.8.16.3A.1	Test Purpose and Environment	2367

A.8.16.3A.2	Test Requirements	2368

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2369

A.8.16.4.1	Test Purpose and Environment	2369

A.8.16.4.2	Test Requirements	2370

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2371

A.8.16.4A.1	Test Purpose and Environment	2371

A.8.16.4A.2	Test Requirements	2373

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2374

A.8.16.5.1	Test Purpose and Environment	2374

A.8.16.5.2	Test Requirements	2374

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2374

A.8.16.6.1	Test Purpose and Environment	2374

A.8.16.6.2	Test Requirements	2375

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2375

A.8.16.7.1	 Test Purpose and Environment	2375

A.8.16.7.2	 Test Requirements	2376

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2376

A.8.16.8.1	 Test Purpose and Environment	2376

A.8.16.8.2	 Test Requirements	2377

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2377

A.8.16.9.1	Test Purpose and Environment	2377

A.8.16.9.2	Test Requirements	2378

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2378

A.8.16.10.1	Test Purpose and Environment	2378

A.8.16.10.2	Test Requirements	2379

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2379

A.8.16.11.1	 Test Purpose and Environment	2379

A.8.16.11.2	 Test Requirements	2380

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2380

A.8.16.12.1	 Test Purpose and Environment	2380

A.8.16.12.2	 Test Requirements	2380

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2380

A.8.16.13.1	Test Purpose and Environment	2380

A.8.16.13.2	Test Requirements	2381

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2381

A.8.16.14.1	Test Purpose and Environment	2381

A.8.16.14.2	Test Requirements	2382

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2382

A.8.16.15.1	 Test Purpose and Environment	2382

A.8.16.7.2	 Test Requirements	2382

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2382

A.8.16.16.1	 Test Purpose and Environment	2382

A.8.16.16.2	 Test Requirements	2383

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2383

A.8.16.17.1	Test Purpose and Environment	2383

A.8.16.17.2	Test Requirements	2385

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2386

A.8.16.17A.1	Test Purpose and Environment	2386

A.8.16.17A.2	Test Requirements	2386

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2386

A.8.16.17B.1	Test Purpose and Environment	2386

A.8.16.17B.2	Test Requirements	2387

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2387

A.8.16.17C.1	Test Purpose and Environment	2387

A.8.16.17C.2	Test Requirements	2387

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2387

A.8.16.18.1	Test Purpose and Environment	2387

A.8.16.18.2	Test Requirements	2389

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2390

A.8.16.18A.1	Test Purpose and Environment	2390

A.8.16.18A.2	Test Requirements	2390

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2390

A.8.16.18B.1	Test Purpose and Environment	2390

A.8.16.18B.2	Test Requirements	2391

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2391

A.8.16.18C.1	Test Purpose and Environment	2391

A.8.16.18C.2	Test Requirements	2391

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2391

A.8.16.18D.1	Test Purpose and Environment	2391

A.8.16.18D.2	Test Requirements	2392

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2392

A.8.16.19.1	Test Purpose and Environment	2392

A.8.16.19.2	Test Requirements	2394

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2394

A.8.16.19A.1	Test Purpose and Environment	2394

A.8.16.19A.2	Test Requirements	2395

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2395

A.8.16.19B.1	Test Purpose and Environment	2395

A.8.16.19B.2	Test Requirements	2395

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2395

A.8.16.19C.1	Test Purpose and Environment	2395

A.8.16.19C.2	Test Requirements	2396

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2396

A.8.16.20.1	Test Purpose and Environment	2396

A.8.16.20.2	Test Requirements	2398

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2399

A.8.16.20A.1	Test Purpose and Environment	2399

A.8.16.20A.2	Test Requirements	2399

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2399

A.8.16.20B.1	Test Purpose and Environment	2399

A.8.16.20B.2	Test Requirements	2400

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2400

A.8.16.20C.1	Test Purpose and Environment	2400

A.8.16.20C.2	Test Requirements	2400

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2400

A.8.16.20D.1	Test Purpose and Environment	2400

A.8.16.20D.2	Test Requirements	2401

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2401

A.8.16.21.1	Test Purpose and Environment	2401

A.8.16.21.2	Test Requirements	2403

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2403

A.8.16.22.1	Test Purpose and Environment	2403

A.8.16.22.2	Test Requirements	2405

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2405

A.8.16.23.1	Test Purpose and Environment	2405

A.8.16.23.2	Test Requirements	2408

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2408

A.8.16.24.1	Test Purpose and Environment	2408

A.8.16.24.2	Test Requirements	2411

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2411

A.8.16.25.1	Test Purpose and Environment	2411

A.8.16.25.2	Test Requirements	2414

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2414

A.8.16.26.1	Test Purpose and Environment	2414

A.8.16.26.2	Test Requirements	2417

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2417

A.8.16.27.1	Test Purpose and Environment	2417

A.8.16.27.2	Test Requirements	2422

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2422

A.8.16.28.1	Test Purpose and Environment	2422

A.8.16.28.2	Test Requirements	2427

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2427

A.8.16.29.1	Test Purpose and Environment	2427

A.8.16.29.2	Test Requirements	2432

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2432

A.8.16.30.1	Test Purpose and Environment	2432

A.8.16.30.2	Test Requirements	2437

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2437

A.8.16.31.1	Test Purpose and Environment	2437

A.8.16.31.2	Test Requirements	2442

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2442

A.8.16.32.1	Test Purpose and Environment	2442

A.8.16.32.2	Test Requirements	2447

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2447

A.8.16.33.1	Test Purpose and Environment	2447

A.8.16.33.2	Test Requirements	2452

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2452

A.8.16.34.1	Test Purpose and Environment	2452

A.8.16.34.2	Test Requirements	2457

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2457

A.8.16.35.1	Test Purpose and Environment	2457

A.8.16.35.2	Test Requirements	2459

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2460

A.8.16.36.1	Test Purpose and Environment	2460

A.8.16.36.2	Test Requirements	2462

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2463

A.8.16.37.1	Test Purpose and Environment	2463

A.8.16.37.2	Test Requirements	2465

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2466

A.8.16.38.1	Test Purpose and Environment	2466

A.8.16.38.2	Test Requirements	2468

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2469

A.8.16.39.1	Test Purpose and Environment	2469

A.8.16.39.2	Test Requirements	2472

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2472

A.8.16.40.1	Test Purpose and Environment	2472

A.8.16.40.2	Test Requirements	2475

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2475

A.8.16.41.1	Test Purpose and Environment	2475

A.8.16.41.2	Test Requirements	2477

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2478

A.8.16.42.1	Test Purpose and Environment	2478

A.8.16.42.2	Test Requirements	2480

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2481

A.8.16.43.1	Test Purpose and Environment	2481

A.8.16.43.2	Test Requirements	2484

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2484

A.8.16.44.1	Test Purpose and Environment	2484

A.8.16.44.2	Test Requirements	2487

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2487

A.8.16.45.1	Test Purpose and Environment	2487

A.8.16.45.2	Test Requirements	2490

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2490

A.8.16.46.1	Test Purpose and Environment	2490

A.8.16.46.2	Test Requirements	2492

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2493

A.8.16.47.1	Test Purpose and Environment	2493

A.8.16.47.2	Test Requirements	2495

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2496

A.8.16.48.1	Test Purpose and Environment	2496

A.8.16.48.2	Test Requirements	2498

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2499

A.8.16.49.1	Test Purpose and Environment	2499

A.8.16.49.2	Test Requirements	2502

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2502

A.8.16.50.1	Test Purpose and Environment	2502

A.8.16.50.2	Test Requirements	2504

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2505

A.8.16.51.1	Test Purpose and Environment	2505

A.8.16.51.2	Test Requirements	2510

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2510

A.8.16.52.1	Test Purpose and Environment	2510

A.8.16.52.2	Test Requirements	2515

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2515

A.8.16.53.1	Test Purpose and Environment	2515

A.8.16.53.2	Test Requirements	2519

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2519

A.8.16.54.1	Test Purpose and Environment	2519

A.8.16.54.2	Test Requirements	2523

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2523

A.8.16.55.1	Test Purpose and Environment	2523

A.8.16.55.2	Test Requirements	2529

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2529

A.8.16.56.1	Test Purpose and Environment	2529

A.8.16.56.2	Test Requirements	2535

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2535

A.8.16.57.1	Test Purpose and Environment	2535

A.8.16.57.2	Test Requirements	2537

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2538

A.8.16.58.1	Test Purpose and Environment	2538

A.8.16.58.2	Test Requirements	2540

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2541

A.8.16.59.1	Test Purpose and Environment	2541

A.8.16.59.2	Test Requirements	2545

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2546

A.8.16.60.1	Test Purpose and Environment	2546

A.8.16.60.2	Test Requirements	2549

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2550

A.8.16.61.1	Test Purpose and Environment	2550

A.8.16.61.2	Test Requirements	2553

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2553

A.8.16.62.1	Test Purpose and Environment	2553

A.8.16.62.2	Test Requirements	2555

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2556

A.8.16.63.1	Test Purpose and Environment	2556

A.8.16.63.2	Test Requirements	2559

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2560

A.8.16.64.1	Test Purpose and Environment	2560

A.8.16.64.2	Test Requirements	2564

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2564

A.8.16.65.1	Test Purpose and Environment	2564

A.8.16.65.2	Test Requirements	2569

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2569

A.8.16.66.1	Test Purpose and Environment	2569

A.8.16.66.2	Test Requirements	2574

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2574

A.8.16.67.1	Test Purpose and Environment	2574

A.8.16.67.2	Test Requirements	2579

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2579

A.8.16.68.1	Test Purpose and Environment	2579

A.8.16.68.2	Test Requirements	2584

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2584

A.8.16.69.1	Test Purpose and Environment	2584

A.8.16.69.2	Test Requirements	2588

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2588

A.8.16.70.1	Test Purpose and Environment	2588

A.8.16.70.2	Test Requirements	2592

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2592

A.8.16.71.1	Test Purpose and Environment	2592

A.8.16.71.2	Test Requirements	2597

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2597

A.8.16.72.1	Test Purpose and Environment	2597

A.8.16.72.2	Test Requirements	2602

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2602

A.8.16.73.1	Test Purpose and Environment	2602

A.8.16.73.2	Test Requirements	2608

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2608

A.8.16.74.1	Test Purpose and Environment	2608

A.8.16.74.2	Test Requirements	2614

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2614

A.8.16.75.1	Test Purpose and Environment	2614

A.8.16.75.2	Test Requirements	2620

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2620

A.8.16.76.1	Test Purpose and Environment	2620

A.8.16.76.2	Test Requirements	2626

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2626

A.8.16.77.1	Test Purpose and Environment	2626

A.8.16.77.2	Test Requirements	2630

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2630

A.8.16.78.1	Test Purpose and Environment	2630

A.8.16.78.2	Test Requirements	2632

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2633

A.8.16.79.1	Test Purpose and Environment	2633

A.8.16.79.2	Test Requirements	2640

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2640

A.8.16.80.1	Test Purpose and Environment	2640

A.8.16.80.2	Test Requirements	2647

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2647

A.8.16.81.1	Test Purpose and Environment	2647

A.8.16.81.2	Test Requirements	2653

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2653

A.8.16.82.1	Test Purpose and Environment	2653

A.8.16.82.2	Test Requirements	2659

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2659

A.8.16.83.1	Test Purpose and Environment	2659

A.8.16.83.2	Test Requirements	2664

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2664

A.8.16.84.1	Test Purpose and Environment	2664

A.8.16.84.2	Test Requirements	2670

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2670

A.8.16.85.1	Test Purpose and Environment	2670

A.8.16.85.2	Test Requirements	2674

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2674

A.8.16.86.1	Test Purpose and Environment	2674

A.8.16.86.2	Test Requirements	2677

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2677

A.8.16.87.1	Test Purpose and Environment	2677

A.8.16.87.2	Test Requirements	2683

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2683

A.8.16.88.1	Test Purpose and Environment	2683

A.8.16.88.2	Test Requirements	2691

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2691

A.8.16.89.1	Test Purpose and Environment	2691

A.8.16.89.2	Test Requirements	2698

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2698

A.8.16.90.1	Test Purpose and Environment	2698

A.8.16.90.2	Test Requirements	2705

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2705

A.8.16.91.1	Test Purpose and Environment	2705

A.8.16.91.2	Test Requirements	2711

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2711

A.8.16.92.1	Test Purpose and Environment	2711

A.8.16.92.2	Test Requirements	2719

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2719

A.8.16.93.1	Test Purpose and Environment	2719

A.8.16.93.2	Test Requirements	2726

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2726

A.8.16.94.1	Test Purpose and Environment	2726

A.8.16.94.2	Test Requirements	2733

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2733

A.8.16.95.1	Test Purpose and Environment	2733

A.8.16.95.2	Test Requirements	2741

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2741

A.8.16.96.1	Test Purpose and Environment	2741

A.8.16.96.2	Test Requirements	2750

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2750

A.8.16.97.1	Test Purpose and Environment	2750

A.8.16.97.2	Test Requirements	2757

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2757

A.8.16.98.1	Test Purpose and Environment	2757

A.8.16.98.2	Test Requirements	2765

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2765

A.8.16.99.1	Test Purpose and Environment	2765

A.8.16.99.2	Test Requirements	2775

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2775

A.8.16.100.1	Test Purpose and Environment	2775

A.8.16.100.2	Test Requirements	2785

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2785

A.8.16.101.1	Test Purpose and Environment	2785

A.8.16.101.2	Test Requirements	2795

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2795

A.8.16.102.1	Test Purpose and Environment	2795

A.8.16.102.2	Test Requirements	2806

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2806

A.8.16.103.1	Test Purpose and Environment	2806

A.8.16.103.2	Test Requirements	2811

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2812

A.8.16.104.1	Test Purpose and Environment	2812

A.8.16.104.2	Test Requirements	2816

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2817

A.8.16.105.1	Test Purpose and Environment	2817

A.8.16.105.2	Test Requirements	2821

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2822

A.8.16.106.1	Test Purpose and Environment	2822

A.8.16.106.2	Test Requirements	2826

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2826

A.8.16.107.1	Test Purpose and Environment	2826

A.8.16.107.2	Test Requirements	2828

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2829

A.8.16.108.1	Test Purpose and Environment	2829

A.8.16.108.2	Test Requirements	2831

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2832

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2832

A.8.17.1.1	Test Purpose and Environment	2832

A.8.17.1.2	Test Requirements	2838

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2838

A.8.17.2.1	Test Purpose and Environment	2838

A.8.17.2.2	Test Requirements	2845

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2845

A.8.17.3.1	Test Purpose and Environment	2845

A.8.17.3.2	Test Requirements	2846

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2846

A.8.17.4.1	Test Purpose and Environment	2846

A.8.17.4.2	Test Requirements	2847

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2847

A.8.17.5.1	Test Purpose and Environment	2847

A.8.17.5.2	Test Requirements	2848

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2848

A.8.17.6.1	Test Purpose and Environment	2848

A.8.17.6.2	Test Requirements	2849

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2850

A.8.17.7.1	Test Purpose and Environment	2850

A.8.17.7.2	Test Requirements	2850

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2851

A.8.17.8.1	Test Purpose and Environment	2851

A.8.17.8.2	Test Requirements	2851

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2852

A.8.17.9.1	Test Purpose and Environment	2852

A.8.17.9.2	Test Requirements	2853

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2853

A.8.17.10.1	Test Purpose and Environment	2853

A.8.17.10.2	Test Requirements	2860

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2861

A.8.17.11.1	Test Purpose and Environment	2861

A.8.17.11.2	Test Requirements	2867

A.8.18	E-UTRAN TDD – HRPD Measurements	2868

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2868

A.8.18.1.1	Test Purpose and Environment	2868

A.8.18.1.2	Test Requirements	2870

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2870

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2870

A.8.19.1.1	Test Purpose and Environment	2870

A.8.19.1.2	Test Requirements	2871

A.8.20	Inter-frequency/RAT Measurements in CA mode	2872

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2872

A.8.20.1.1	Test Purpose and Environment	2872

A.8.20.1.2	Test Requirements	2873

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2873

A.8.20.2.1	Test Purpose and Environment	2874

A.8.20.2.2	Test Requirements	2875

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2876

A.8.20.2A.1	Test Purpose and Environment	2876

A.8.20.2A.2	Test Requirements	2876

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2876

A.8.20.2B.1	Test Purpose and Environment	2876

A.8.20.2B.2	Test Requirements	2879

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2879

A.8.20.3.1	Test Purpose and Environment	2879

A.8.20.3.2	Test Requirements	2881

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2881

A.8.20.4.1	Test Purpose and Environment	2881

A.8.20.4.1.1	1.28 Mcps TDD option	2881

A.8.20.4.2	Test Requirements	2883

A.8.20.4.2.1	1.28 Mcps TDD option	2883

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2883

A.8.20.4A.1	Test Purpose and Environment	2883

A.8.20.4A.1.1	1.28 Mcps TDD option	2883

A.8.20.4A.2	Test Requirements	2883

A.8.20.4A.2.1	1.28 Mcps TDD option	2883

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2884

A.8.20.4B.1	Test Purpose and Environment	2884

A.8.20.4B.1.1	1.28 Mcps TDD option	2884

A.8.20.4B.2	Test Requirements	2886

A.8.20.4B.2.1	1.28 Mcps TDD option	2886

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2886

A.8.21.1	Test Purpose and Environment	2886

A.8.21.2 Test Requirements	2890

A.8.22	E-UTRAN Discovery Signal Measurements	2890

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2890

A.8.22.1.1	Test Purpose and Environment	2890

A.8.22.1.2	Test Requirements	2893

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2893

A.8.22.2.1	Test Purpose and Environment	2893

A.8.22.2.2	Test Requirements	2896

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2896

A.8.22.3.1	Test Purpose and Environment	2896

A.8.22.3.2	Test Requirements	2899

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2899

A.8.22.4.1	Test Purpose and Environment	2899

A.8.22.4.2	Test Requirements	2902

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2902

A.8.22.5.1	Test Purpose and Environment	2902

A.8.22.5.2	Test Requirements	2906

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2906

A.8.22.6.1	Test Purpose and Environment	2906

A.8.22.6.2	Test Requirements	2910

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2910

A.8.22.7.1	Test Purpose and Environment	2910

A.8.22.7.2	Test Requirements	2914

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2914

A.8.22.8.1	Test Purpose and Environment	2914

A.8.22.8.2	Test Requirements	2918

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2918

A.8.22.9.1	Test Purpose and Environment	2918

A.8.22.9.2	Test Requirements	2920

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2921

A.8.22.10.1	Test Purpose and Environment	2921

A.8.22.10.2	Test Requirements	2923

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2924

A.8.22.11.1	Test Purpose and Environment	2924

A.8.22.11.2	Test Requirements	2927

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2927

A.8.22.12.1	Test Purpose and Environment	2927

A.8.22.12.2	Test Requirements	2931

A.8.23	E-UTRAN Dual Connectivity Measurements	2931

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2931

A.8.23.1.1	Test Purpose and Environment	2931

A.8.23.1.2	Test Requirements	2934

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2934

A.8.23.2.1	Test Purpose and Environment	2934

A.8.23.2.2	Test Requirements	2937

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2937

A.8.23.3.1	Test Purpose and Environment	2937

A.8.23.3.2	Test Requirements	2940

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2940

A.8.23.4.1	Test Purpose and Environment	2940

A.8.23.4.2	Test Requirements	2943

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2943

A.8.23.5.1	Test Purpose and Environment	2943

A.8.23.5.2	Test Requirements	2946

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2946

A.8.23.6.1	Test Purpose and Environment	2946

A.8.23.6.2	Test Requirements	2949

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2949

A.8.23.7.1	Test Purpose and Environment	2949

A.8.23.7.2	Test Requirements	2951

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2952

A.8.23.8.1	Test Purpose and Environment	2952

A.8.23.8.2	Test Requirements	2954

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2955

A.8.23.9.1	Test Purpose and Environment	2955

A.8.23.9.2	Test Requirements	2958

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2958

A.8.23.10.1	Test Purpose and Environment	2958

A.8.23.10.2	Test Requirements	2961

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2961

A.8.23.11.1	Test Purpose and Environment	2961

A.8.23.11.2	Test Requirements	2964

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2964

A.8.23.12.1	Test Purpose and Environment	2964

A.8.23.12.2	Test Requirements	2967

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2967

A.8.23.13.1	Test Purpose and Environment	2967

A.8.23.13.2	Test Requirements	2970

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2970

A.8.23.14.1	Test Purpose and Environment	2970

A.8.23.14.2	Test Requirements	2973

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2973

A.8.23.15.1	Test Purpose and Environment	2973

A.8.23.15.2	Test Requirements	2976

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2976

A.8.23.16.1	Test Purpose and Environment	2976

A.8.23.16.2	Test Requirements	2977

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2978

A.8.23.17.1	Test Purpose and Environment	2978

A.8.23.17.2	Test Requirements	2980

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2980

A.8.23.18.1	Test Purpose and Environment	2980

A.8.23.18.2	Test Requirements	2981

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2982

A.8.23.19.1	Test Purpose and Environment	2982

A.8.23.19.2	Test Requirements	2983

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2984

A.8.23.20.1	Test Purpose and Environment	2984

A.8.23.20.2	Test Requirements	2985

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2986

A.8.23.21.1	Test Purpose and Environment	2986

A.8.23.21.2	Test Requirements	2987

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2988

A.8.23.22.1	Test Purpose and Environment	2988

A.8.23.22.2	Test Requirements	2989

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2990

A.8.23.23.1	Test Purpose and Environment	2990

A.8.23.23.2	Test Requirements	2991

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2992

A.8.23.24.1	Test Purpose and Environment	2992

A.8.23.24.2	Test Requirements	2994

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2995

A.8.23.25.1	Test Purpose and Environment	2995

A.8.23.25.2	Test Requirements	2997

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2998

A.8.23.26.1	Test Purpose and Environment	2998

A.8.23.26.2	Test Requirements	3000

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3001

A.8.23.27.1	Test Purpose and Environment	3001

A.8.23.27.2	Test Requirements	3004

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3004

A.8.23.28.1	Test Purpose and Environment	3004

A.8.23.28.2	Test Requirements	3008

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3008

A.8.23.29.1	Test Purpose and Environment	3008

A.8.23.29.2	Test Requirements	3012

A.8.24	Proximity-based Services	3012

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3012

A.8.24.1.1	Test Purpose and Environment	3012

A.8.24.1.2	Test Requirements	3013

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3014

A.8.24.2.1	Test Purpose and Environment	3014

A.8.24.2.2	Test Requirements	3015

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3015

A.8.24.3.1	Test Purpose and Environment	3016

A.8.24.3.2	Test Requirements	3017

A.8.25	E-UTRAN-WLAN Measurements	3018

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3018

A.8.25.1.1	Test Purpose and Environment	3018

A.8.25.1.2	Test Requirements	3020

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3020

A.8.25.2.1	Test Purpose and Environment	3020

A.8.25.2.2	Test Requirements	3022

A.8.26 Frame Structure 3 (FS3)	3022

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3022

A.8.26.1.1	Test Purpose and Environment	3022

A.8.26.1.2	Test Requirements	3024

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3025

A.8.26.2.1	Test Purpose and Environment	3025

A.8.26.2.2	Test Requirements	3027

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3028

A.8.26.3.1	Test Purpose and Environment	3028

A.8.26.3.2	Test Requirements	3031

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3031

A.8.26.3A.1	Test Purpose and Environment	3031

A.8.26.3A.2	Test Requirements	3034

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3034

A.8.26.4.1	Test Purpose and Environment	3034

A.8.26.4.2	Test Requirements	3038

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3038

A.8.26.4A.1	Test Purpose and Environment	3038

A.8.26.4A.2	Test Requirements	3042

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3042

A.8.26.5.1	Test Purpose and Environment	3042

A.8.26.5.2	Test Requirements	3045

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3045

A.8.26.5A.1	Test Purpose and Environment	3045

A.8.26.5A.2	Test Requirements	3049

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3049

A.8.26.6.1	Test Purpose and Environment	3049

A.8.26.6.2	Test Requirements	3053

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3053

A.8.26.6A.1	Test Purpose and Environment	3053

A.8.26.6A.2	Test Requirements	3057

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3057

A.8.26.7.1	Test Purpose and Environment	3057

A.8.26.7.2	Test Requirements	3060

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3060

A.8.26.8.1	Test Purpose and Environment	3060

A.8.26.8.2	Test Requirements	3064

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3064

A.8.26.9.1	Test Purpose and Environment	3064

A.8.26.9.2	Test Requirements	3067

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3067

A.8.26.10.1	Test Purpose and Environment	3067

A.8.26.10.2	Test Requirements	3070