| 3GPP TS 38.133 V17.0.0 (2020-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	39

1	Scope	41

2	References	41

3	Definitions, symbols and abbreviations	42

3.1	Definitions	42

3.2	Symbols	43

3.3	Abbreviations	44

3.4	Test tolerances	46

3.5	Frequency bands grouping	46

3.5.1	Introduction	46

3.5.2	NR operating bands in FR1	47

3.5.3	NR operating bands in FR2	47

3.6	Applicability of requirements in this specification version	48

3.6.1	RRC connected state requirements in DRX	48

3.6.2	Number of serving carriers	49

3.6.2.1	Number of serving carriers for SA	49

3.6.2.2	Number of serving carriers for EN-DC	49

3.6.2.3	Number of serving carriers for NE-DC	49

3.6.2.4	Number of serving carriers for NR-DC	49

3.6.3	Applicability for intra-band FR2	50

3.6.4	Applicability for FR2 UE power classes	50

3.6.5	Applicability for SDL bands	50

3.6.6	Applicability of requirements for NGEN-DC operation	50

3.6.7	Applicability of QCL	50

3.6.9	Applicability of requirements for scheduling availability	50

3.6.10	Applicability of requirements for measurement restrictions	51

4	SA: RRC_IDLE state mobility	51

4.1	Cell Selection	51

4.2	Cell Re-selection	51

4.2.1	Introduction	51

4.2.2	Requirements	51

4.2.2.1	UE measurement capability	51

4.2.2.2	Measurement and evaluation of serving cell	52

4.2.2.3	Measurements of intra-frequency NR cells	52

4.2.2.4	Measurements of inter-frequency NR cells	53

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	55

4.2.2.6	Maximum interruption in paging reception	56

4.2.2.7	General requirements	57

4.2.2.8	Minimum requirement at transitions	57

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	58

4.2.2.9.1	Introduction	58

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	58

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	58

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	59

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	59

4.2.2.10.1	Introduction	59

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	59

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	60

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	61

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	61

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	61

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	62

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	63

4.2A	Cell Re-selection when subject to CCA	63

4.2A.1	Introduction	63

4.2A.2	Requirements	64

4.2A.2.1	UE measurement capability	64

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	64

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	65

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	66

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	68

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	68

4.2A.2.7	General requirements	68

4.3	Minimization of Drive Tests (MDT)	69

4.3.1	Introduction	69

4.3.2	Measurement Requirements	69

4.3.3	Requirements for Relative Time Stamp Accuracy	69

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	70

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	70

4.4	Idle Mode CA/DC Measurements	70

4.4.1	Introduction	70

4.4.2	Measurement Requirements	70

4.4.2.1	Detected cell requirement during state transition and Idle mode	70

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	71

4.4.2.3	Measurements on serving cell	72

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	72

5	SA: RRC_INACTIVE state mobility	74

5.1	Cell Re-selection	74

5.1.1	Introduction	74

5.1.2	Requirements	74

5.1.2.1	UE measurement capability	74

5.1.2.2	Measurement and evaluation of serving cell	74

5.1.2.3	Measurements of intra-frequency NR cells	74

5.1.2.4	Measurements of inter-frequency NR cells	74

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	74

5.1.2.6	Maximum interruption in paging reception	74

5.1.2.7	General requirements	74

5.1A	Cell Re-selection with CCA	74

5.1A.1	Introduction	74

5.1A.2	Requirements	75

5.1A.2.1	UE measurement capability	75

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	75

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	75

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	75

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	75

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	75

5.1A.2.7	General requirements	75

5.2	Void	75

5.3	Minimization of Drive Tests (MDT)	75

5.3.1	Introduction	75

5.3.2	Measurement Requirements	76

5.3.3	Requirements for Relative Time Stamp Accuracy	76

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	76

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	76

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	76

5.4	Idle Mode CA/DC Measurements	76

5.4.1	Introduction	76

5.4.2	Measurement Requirements	76

5.4.2.1	Detected cell requirement during state transition and Idle mode	77

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	77

5.4.2.3	Measurements on serving cell	77

6	RRC_CONNECTED state mobility	77

6.1	Handover	77

6.1.1	NR Handover	77

6.1.1.1	Introduction	77

6.1.1.2	NR FR1 - NR FR1 Handover	77

6.1.1.2.1	Handover delay	77

6.1.1.2.2	Interruption time	77

6.1.1.3	NR FR2- NR FR1 Handover	78

6.1.1.3.1	Handover delay	78

6.1.1.3.2	Interruption time	78

6.1.1.4	NR FR2- NR FR2 Handover	79

6.1.1.4.1	Handover delay	79

6.1.1.4.2	Interruption time	79

6.1.1.5	NR FR1- NR FR2 Handover	80

6.1.1.5.1	Handover delay	80

6.1.1.5.2	Interruption time	80

6.1.2	NR Handover to other RATs	81

6.1.2.1	NR – E-UTRAN Handover	81

6.1.2.1.1	Introduction	81

6.1.2.1.2	Handover delay	81

6.1.2.1.3	Interruption time	81

6.1.2.2	NR – UTRAN Handover	81

6.1.2.2.1	Introduction	81

6.1.2.2.2	Handover delay	81

6.1.2.2.3	Interruption time	82

6.1.3	NR DAPS Handover	82

6.1.3.1	Introduction	82

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	83

6.1.3.2.1	DAPS handover delay	83

6.1.3.2.2	Interruption time	84

6.1.3.3	NR FR2- NR FR1 DAPS Handover	85

6.1.3.3.1	DAPS handover delay	85

6.1.3.3.2	Interruption time	86

6.1.3.4	NR FR1- NR FR2 DAPS Handover	86

6.1.3.4.1	DAPS handover delay	87

6.1.3.4.2	Interruption time	87

6.1.4	NR Conditional Handover	87

6.1.4.1	Introduction	87

6.1.4.2	NR FR1 – NR FR1 conditional handover	88

6.1.4.3	NR FR2 – NR FR1 conditional handover	89

6.1.4.4	NR FR2 – NR FR2 conditional handover	89

6.1.4.4.1	Handover delay	89

6.1.4.4.2	Measurement time	89

6.1.4.4.3	Preparation time	90

6.1.4.4.4	Interruption time	90

6.1.4.5	NR FR1 – NR FR2 conditional handover	90

6.1A	Void	91

6.1A.1	Void	91

6.1A.1.1	Void	91

6.1A.1.2	Void	91

6.1A.1.2.1	Void	91

6.1A.1.2.2	Void	91

6.1B	Handover to target cell using CCA	91

6.1B.1	NR Handover	91

6.1B.1.1	Introduction	91

6.1B.1.2	NR FR1 - NR FR1 Handover	91

6.1B.1.2.1	Handover delay	91

6.1B.1.2.2	Interruption time	91

6.2	RRC Connection Mobility Control	92

6.2.1	SA: RRC Re-establishment	92

6.2.1.1	Introduction	92

6.2.1.2	Requirements	92

6.2.1.2.1	UE Re-establishment delay requirement	93

6.2.1A	RRC Re-establishment with CCA	94

6.2.1A.1	Introduction	94

6.2.1A.2	Requirements	94

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	95

6.2.2	Random access	96

6.2.2.1	Introduction	96

6.2.2.2	Requirements for 4-step RA type	96

6.2.2.2.1	Contention based random access	97

6.2.2.2.2	Non-Contention based random access	97

6.2.2.2.3	UE behaviour when configured with supplementary UL	98

6.2.2.3	Requirements for 2-step RA type	98

6.2.2.3.1	Contention based random access	99

6.2.2.3.2	Non-Contention based random access	100

6.2.2.3.3	UE behaviour when configured with supplementary UL	100

6.2.3	SA: RRC Connection Release with Redirection	100

6.2.3.1	Introduction	100

6.2.3.2	Requirements	101

6.2.3.2.1	RRC connection release with redirection to NR	101

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	101

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	102

7	Timing	103

7.1	UE transmit timing	103

7.1.1	Introduction	103

7.1.2	Requirements	103

7.1.2.1	Gradual timing adjustment	104

7.1.2.2	Void	105

7.2	UE timer accuracy	105

7.2.1	Introduction	105

7.2.2	Requirements	105

7.3	Timing advance	106

7.3.1	Introduction	106

7.3.2	Requirements	106

7.3.2.1	Timing Advance adjustment delay	106

7.3.2.2	Timing Advance adjustment accuracy	106

7.4	Cell phase synchronization accuracy	106

7.4.1	Definition	106

7.4.2	Minimum requirements	106

7.5	Maximum Transmission Timing Difference	106

7.5.1	Introduction	106

7.5.2	Minimum Requirements for inter-band EN-DC	107

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	107

7.5.3	Minimum Requirements for intra-band EN-DC	107

7.5.4	Minimum Requirements for NR Carrier Aggregation	108

7.5.5	Minimum Requirements for inter-band NE-DC	108

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	109

7.5.6	Minimum Requirements for inter-band NR DC	109

7.6	Maximum Receive Timing Difference	109

7.6.1	Introduction	109

7.6.2	Minimum Requirements for inter-band EN-DC	110

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	110

7.6.3	Minimum Requirements for intra-band EN-DC	110

7.6.4	Minimum Requirements for NR Carrier Aggregation	111

7.6.5	Minimum Requirements for inter-band NE-DC	111

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	112

7.6.6	Minimum Requirements for inter-band NR DC	112

7.7	deriveSSB-IndexFromCell tolerance	113

7.7.1	Minimum requirements	113

7.8	Void	113

8	Signalling characteristics	113

8.1	Radio Link Monitoring	113

8.1.1	Introduction	113

8.1.2	Requirements for SSB based radio link monitoring	114

8.1.2.1	Introduction	114

8.1.2.2	Minimum requirement	115

8.1.2.3	Measurement restrictions for SSB based RLM	117

8.1.3	Requirements for CSI-RS based radio link monitoring	117

8.1.3.1	Introduction	117

8.1.3.2	Minimum requirement	118

8.1.3.3	Measurement restrictions for CSI-RS based RLM	120

8.1.4	Minimum requirement at transitions	121

8.1.5	Minimum requirement for UE turning off the transmitter	121

8.1.6	Minimum requirement for L1 indication	121

8.1.7	Scheduling availability of UE during radio link monitoring	122

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	122

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	122

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	122

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	123

8.1A	Radio Link Monitoring with CCA on Target Frequency	123

8.1A.1	Introduction	123

8.1A.2	Requirements for SSB Based Radio Link Monitoring	124

8.1A.2.1	Introduction	124

8.1A.2.2	Minimum Requirement	125

8.1A.3	Minimum requirement at transitions	126

8.1A.4	Minimum requirement for UE turning off the transmitter	126

8.1A.5	Minimum requirement for L1 indication	127

8.1A.6	Scheduling availability of UE during radio link monitoring	127

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	127

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	127

8.2	Interruption	127

8.2.1	EN-DC Interruption	127

8.2.1.1	Introduction	127

8.2.1.2	Requirements	128

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	128

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	128

8.2.1.2.3	Interruptions at SCell addition/release	129

8.2.1.2.4	Interruptions at SCell activation/deactivation	130

8.2.1.2.5	Interruptions during measurements on SCC	131

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	132

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	133

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	134

8.2.1.2.9	Interruptions at SCell hibernation	134

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	134

8.2.1.2.11	Interruptions due to UE-specific CBW change	135

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	135

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	136

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	137

8.2.1.2.15	Interruptions due to SCell dormancy	137

8.2.1.2.16	 Interruptions when identifying CGI of an NR cell with autonomous gaps	138

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	139

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	139

8.2.2.1	Introduction	139

8.2.2.2	Requirements	140

8.2.2.2.1	Interruptions at SCell addition/release	140

8.2.2.2.2	Interruptions at SCell activation/deactivation	141

8.2.2.2.3	Interruptions during measurements on deactivated SCC	142

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	142

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	142

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	144

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	145

8.2.2.2.8	Interruptions due to UE-specific CBW change	145

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	145

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	146

8.2.2.2.11	Interruptions at direct SCell activation	147

8.2.2.2.12	Interruptions due to SCell dormancy	147

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	147

8.2.2.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	148

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	148

8.2.3	NE-DC Interruptions	149

8.2.3.1	Introduction	149

8.2.3.2	Requirements	149

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	149

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	150

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	150

8.2.3.2.4	Interruptions at SCell activation/deactivation	151

8.2.3.2.5	Interruptions during measurements on SCC	152

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	153

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	154

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	154

8.2.3.2.9	Interruptions at SCell hibernation	154

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	155

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	155

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	156

8.2.3.2.13	Interruptions due to SCell dormancy	157

8.2.3.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	157

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	158

8.2.4	NR-DC: Interruptions	159

8.2.4.1	Introduction	159

8.2.4.2	Requirements	159

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	159

8.2.4.2.2	Interruptions at SCell activation/deactivation	160

8.2.4.2.3	Interruptions during measurements on SCC	161

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	161

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	162

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	162

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	162

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	163

8.2.4.2.9	Interruptions at NR SRS carrier based switching	163

8.2.4.2.10	Interruptions at direct SCell activation	164

8.2.4.2.11	 Interruptions when identifying CGI of an NR cell with autonomous gaps	164

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	165

8.2.4.2A	Interruptions due to SCell dormancy	166

8.2.4.2A.1	Interruptions due to SCell dormancy switch	166

8.2.4.2A.2	Interruptions due to CQI measurements during SCell dormancy	166

8.2.4.2A.3	Interruptions due to RRM measurements during SCell dormancy	166

8.3	SCell Activation and Deactivation Delay	166

8.3.1	Introduction	166

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	166

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	170

8.3.4	Direct SCell Activation at SCell addition	170

8.3.5	Direct SCell Activation at Handover	171

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	172

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	175

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	175

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	176

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	178

8.3A.1	Introduction	178

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	178

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	181

8.4	UE UL carrier RRC reconfiguration delay	181

8.4.1	Introduction	181

8.4.2	UE UL carrier configuration delay requirement	181

8.4.3	UE UL carrier deconfiguration delay requirement	181

8.5	Link Recovery Procedures	181

8.5.1	Introduction	181

8.5.2	Requirements for SSB based beam failure detection	182

8.5.2.1	Introduction	182

8.5.2.2	Minimum requirement	183

8.5.2.3	Measurement restriction for SSB based beam failure detection	184

8.5.3	Requirements for CSI-RS based beam failure detection	185

8.5.3.1	Introduction	185

8.5.3.2	Minimum requirement	185

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	187

8.5.4	Minimum requirement for L1 indication	188

8.5.5	Requirements for SSB based candidate beam detection	188

8.5.5.1	Introduction	188

8.5.5.2	Minimum requirement	188

8.5.5.3	Measurement restriction for SSB based candidate beam detection	190

8.5.6	Requirements for CSI-RS based candidate beam detection	191

8.5.6.1	Introduction	191

8.5.6.2	Minimum requirement	191

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	193

8.5.7	Scheduling availability of UE during beam failure detection	193

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	194

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	194

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	194

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	195

8.5.8	Scheduling availability of UE during candidate beam detection	195

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	195

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	195

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	195

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	196

8.5.9	Requirements for Link Recovery with Link Recovery Request (LRR)	196

8.5.9.1	Introduction	196

8.5.9.2	Requirement	196

8.5A	Link Recovery Procedures when CCA is used on target frequency	196

8.5A.1	Introduction	196

8.5A.2	Requirements for SSB based beam failure detection	197

8.5A.2.1	Introduction	197

8.5A.2.2	Minimum requirement	197

8.5A.2.3	Measurement restriction for SSB based beam failure detection	198

8.5A.4	Minimum requirement for L1 indication	198

8.5A.5	Requirements for SSB based candidate beam detection	199

8.5A.5.1	Introduction	199

8.5A.5.2	Minimum requirement	199

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	199

8.5A.7	Scheduling availability of UE during beam failure detection	200

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	200

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	200

8.5A.8	Scheduling availability of UE during candidate beam detection	200

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	200

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	200

8.6	Active BWP switch delay	200

8.6.1	Introduction	200

8.6.2	DCI and timer based BWP switch delay on a single CC	200

8.6.2A	DCI based BWP switch delay on multiple CCs	202

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	202

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	203

8.6.2B	Timer based BWP switch delay on multiple CCs	203

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	203

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	204

8.6.3	RRC based BWP switch delay on a single CC	204

8.6.3A	RRC based BWP switch delay on multiple CCs	205

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	205

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	205

8.6.4	BWP switch delay on Consistent UL LBT recovery	206

8.7	Void	206

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	206

8.8.1	Introduction	206

8.8.2	E-UTRAN PSCell Addition Delay Requirement	206

8.8.3	E-UTRAN PSCell Release Delay Requirement	207

8.9	NR-DC: PSCell Addition and Release Delay	207

8.9.1	Introduction	207

8.9.2	PSCell Addition Delay Requirement	207

8.9.3	PSCell Release Delay Requirement	208

8.10	Active TCI state switching delay	208

8.10.4	DCI based TCI state switch delay	209

8.10.5	RRC based TCI state switch delay	210

8.10.6	Active TCI state list update delay	210

8.10A	Active TCI state switching delay with CCA	210

8.10A.1	Introduction	210

8.10A.2	Known conditions for TCI state	210

8.10A.3	MAC-CE based TCI state switch delay	211

8.10A.4	DCI based TCI state switch delay	211

8.10A.5	RRC based TCI state switch delay	212

8.10A.6	Active TCI state list update delay	212

8.11	PSCell Change	212

8.11A	void	213

8.11B	Conditional PSCell Change	213

8.11B.1	Introduction	213

8.11B.2	Conditoinal PSCell Change delay	213

8.11B.2.1	Measurement time	213

8.12	Uplink spatial relation switch delay	214

8.12.1	Introduction	214

8.12.2	Known conditions for spatial relation when associated with DL-RS	214

8.12.3	MAC-CE based spatial relation switch delay	214

8.12.4	DCI based spatial relation switch delay	215

8.12.5	RRC based spatial relation switch delay	215

8.13	UE-specific CBW change	215

8.13.1	Introduction	215

8.13.2	UE-specific CBW change delay	215

8.14	Pathloss reference signal switching delay	215

8.14.1	Introduction	215

8.14.2	Known conditions for pathloss reference signal	215

8.14.3	MAC-CE based pathloss reference signal switch delay	216

9	Measurement Procedure	216

9.1	General measurement requirement	216

9.1.1	Introduction	216

9.1.2	Measurement gap	217

9.1.2.1	EN-DC: Measurement Gap Sharing	227

9.1.2.1a	SA: Measurement Gap Sharing	228

9.1.2.1b	NE-DC: Measurement Gap Sharing	229

9.1.2.1c	NR-DC: Measurement Gap Sharing	230

9.1.3	UE Measurement capability	230

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	230

9.1.3.1a	SA: Monitoring of multiple layers using gaps	231

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	231

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	232

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	232

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	233

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	234

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	235

9.1A.3.2	Void	236

9.1.3A	UE Measurement capability under operation mode with CCA	236

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	236

9.1.3A.1A	SA: Monitoring of multiple layers using gaps under CCA	236

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	236

9.1A.3.2a	Void9.1.3A.2A SA: Maximum allowed layers for multiple monitoring under CCA	236

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	237

9.1.4.1	Introduction	237

9.1.4.2	Requirements	237

9.1.5	Carrier-specific scaling factor	241

9.1.5.1	Monitoring of multiple layers outside gaps	241

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	243

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	243

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	244

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	245

9.1.5.2	Monitoring of multiple layers within gaps	246

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB and CSI-RS-based L3 measurements performed within gaps	247

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB and CSI-RS-based L3 measurements performed within gaps	248

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	249

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	251

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	252

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	253

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	253

9.1.6	Minimum requirement at transitions	253

9.2	NR intra-frequency measurements	253

9.2.1	Introduction	253

9.2.2	Requirements applicability	254

9.2.3	Number of cells and number of SSB	254

9.2.3.1	Requirements for FR1	254

9.2.3.2	Requirements for FR2	254

9.2.4	Measurement Reporting Requirements	255

9.2.4.1	Periodic Reporting	255

9.2.4.2	Event-triggered Periodic Reporting	255

9.2.4.3	Event Triggered Reporting	255

9.2.5	Intrafrequency measurements without measurement gaps	255

9.2.5.1	Intrafrequency cell identification	255

9.2.5.2	Measurement period	258

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	259

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	259

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	259

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	260

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	260

9.2.5.4	SFTD Measurements between PCell and PSCell	261

9.2.5.4.1	Introduction	261

9.2.5.4.2	SFTD Measurement delay	261

9.2.5.4.3	SFTD Measurement Reporting Delay	261

9.2.6	Intra-frequency measurements with measurement gaps	262

9.2.6.1	Void	262

9.2.6.2	Intra-frequency cell identification	262

9.2.6.3	Intrafrequency Measurement Period	263

9.2A	NR intra-frequency measurements with CCA	264

9.2A.1	Introduction	264

9.2A.2	Requirements applicability	265

9.2A.3	Number of cells and number of SSB	265

9.2A.4	Measurement Reporting Requirements	265

9.2A.5	Intra-frequency measurements without measurement gaps	266

9.2A.5.2	Measurement period	268

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	269

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	269

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	269

9.2A.6	Intra-frequency measurements with measurement gaps	270

9.2A.6.1	Intra-frequency cell identification	270

9.2A.6.2	Intra-frequency Measurement Period	271

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	271

9.2A.7.1	Intra-frequency RSSI measurements	271

9.2A.7.2	Intra-frequency Channel occupancy measurements	272

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	273

9.3	NR inter-frequency measurements	273

9.3.1	Introduction	273

9.3.2	Requirements applicability	274

9.3.2.1	Void	274

9.3.2.2	Void	274

9.3.3	Number of cells and number of SSB	274

9.3.3.1	Requirements for FR1	274

9.3.3.2	Requirements for FR2	275

9.3.4	Inter-frequency measurement with measurement gaps	275

9.3.4.1	Void	276

9.3.4.2	Void	276

9.3.5	Inter-frequency measurements	276

9.3.5.1	Void	277

9.3.5.2	Void	277

9.3.5.3	Void	277

9.3.6	Inter-frequency measurements reporting requirements	277

9.3.6.1	Periodic Reporting	277

9.3.6.2	Event-triggered Periodic Reporting	277

9.3.6.3	Event-triggered Reporting	277

9.3.7	Void	278

9.3.8	Inter-frequency SFTD measurement requirements	278

9.3.8.1	Introduction	278

9.3.8.2	SFTD Measurement delay	278

9.3.8.3	SFTD Measurement reporting delay	279

9.3.9	Inter frequency measurements without measurement gaps	279

9.3.9.1	Inter frequency Cell identification	279

9.3.9.2	Measurement period	281

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	281

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	281

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	282

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	282

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	283

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	283

9.3A.1	Introduction	283

9.3A.2	Requirements applicability	283

9.3A.3	Number of cells and number of SSB	284

9.3A.3.1	Requirements	284

9.3A.4	Inter-frequency cell identification	284

9.3A.5	Inter-frequency measurements	285

9.3A.6	NR Inter-frequency measurements reporting requirements	285

9.3A.6.1	Periodic Reporting	285

9.3A.6.2	Event-triggered Periodic Reporting	286

9.3A.6.3	Event-triggered Reporting	286

9.3A.8	Inter-frequency RSSI measurements	286

9.3A.9	Inter-frequency channel occupancy measurements	287

9.4	Inter-RAT measurements	287

9.4.1	Introduction	287

9.4.2	NR − E-UTRAN FDD measurements	288

9.4.2.1	Introduction	288

9.4.2.2	Requirements when no DRX is used	289

9.4.2.3	Requirements when DRX is used	289

9.4.2.4	Measurement reporting requirements	290

9.4.2.4.1	Periodic Reporting	290

9.4.2.4.2	Event-Triggered Periodic Reporting	290

9.4.2.4.3	Event-Triggered Reporting	290

9.4.3	NR − E-UTRAN TDD measurements	291

9.4.3.1	Introduction	291

9.4.3.2	Requirements when no DRX is used	291

9.4.3.3	Requirements when DRX is used	292

9.4.3.4	Measurement reporting requirements	293

9.4.3.4.1	Periodic Reporting	293

9.4.3.4.2	Event-Triggered Periodic Reporting	293

9.4.3.4.3	Event-Triggered Reporting	294

9.4.4	Inter-RAT RSTD measurements	294

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	294

9.4.4.1.1	Introduction	294

9.4.4.1.2	Requirements	295

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	298

9.4.4.2.1	Introduction	298

9.4.4.2.2	Requirements	298

9.4.5	Inter-RAT E-CID measurements	302

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	302

9.4.5.1.1	Introduction	302

9.4.5.1.2	Requirements	302

9.4.5.1.3	Measurement Reporting Delay	302

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	302

9.4.5.2.1	Introduction	302

9.4.5.2.2	Requirements	302

9.4.5.2.3	Measurement Reporting Delay	302

9.4.6	NR − UTRAN FDD measurements	302

9.4.6.1	Introduction	302

9.4.6.2	Requirements when no DRX is used	303

9.4.6.3	Requirements when DRX is used	304

9.4.7	NR – E-UTRAN measurements with autonomous gaps	306

9.4.7.1	Reporting delay due to E-UTRAN CGI reading with autonomous gaps	306

9.5	L1-RSRP measurements for Reporting	306

9.5.1	Introduction	306

9.5.2	Requirements applicability	306

9.5.3	Measurement Reporting Requirements	307

9.5.3.1	Periodic Reporting	307

9.5.3.2	Semi-Persistent Reporting	307

9.5.3.3	Aperiodic Reporting	307

9.5.4	L1-RSRP measurement requirements	308

9.5.4.1	SSB based L1-RSRP Reporting	308

9.5.4.2	CSI-RS based L1-RSRP Reporting	309

9.5.4A	L1-RSRP measurement requirements (with CCA serving cell)	312

9.5.4A.1	SSB based L1-RSRP Reporting	312

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	313

9.5.5.1	Measurement restriction for SSB based L1-RSRP	313

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	313

9.5.6	Scheduling availability of UE during L1-RSRP measurement	314

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	314

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	314

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	315

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	315

9.6	NE-DC: Measurements	316

9.6.1	Introduction	316

9.6.2	SFTD Measurements	316

9.6.2.1	Introduction	316

9.6.2.2	SFTD Measurement requirements	316

9.7	Cross Link Interference measurements	317

9.7.1	Introduction	317

9.7.2	SRS-RSRP measurements	317

9.7.2.1	Introduction	317

9.7.2.2	Requirements applicability	317

9.7.2.3	Measurement Reporting Requirements	317

9.7.2.3.1	Periodic Reporting	317

9.7.2.3.2	Event-triggered Periodic Reporting	318

9.7.2.3.3	Event Triggered Reporting	318

9.7.2.4	Measurement capability	318

9.7.2.5	SRS-RSRP measurement period	318

9.7.3	CLI-RSSI measurements	318

9.7.3.1	Introduction	318

9.7.3.2	Requirements applicability	318

9.7.3.3	Measurement Reporting Requirements	319

9.7.3.3.1	Periodic Reporting	319

9.7.3.3.2	Event-triggered Periodic Reporting	319

9.7.3.3.3	Event Triggered Reporting	319

9.7.3.4	Measurement capability	319

9.7.3.5	CLI-RSSI measurement period	319

9.7.4	Scheduling availability of UE during CLI measurements	320

9.7.4.1	Scheduling availability of UE performing measurement on FR1	320

9.7.4.2	Scheduling availability of UE performing measurement on FR2	320

9.8	L1-SINR measurements for Reporting	321

9.8.1	Introduction	321

9.8.2	Requirements applicability	321

9.8.3	Measurement Reporting Requirements	321

9.8.3.1	Periodic Reporting	322

9.8.3.2	Semi-Persistent Reporting	322

9.8.4	L1-SINR measurement requirements	322

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	322

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	325

9.8.5	Measurement restriction for L1-SINR measurement	327

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	327

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	328

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	328

9.8.6	Scheduling availability of UE during L1-SINR measurement	329

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	329

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	329

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	330

9.9	 NR measurements for positioning	330

9.9.1	Introduction	330

9.9.2	RSTD measurements	330

9.9.2.1	Introduction	330

9.9.2.2	Requirements Applicability	331

9.9.2.3	Measurement Capability	331

9.9.2.4	Measurement Reporting Requirements	331

9.9.2.4.1	Void	331

9.9.2.4.2	Void	331

9.9.2.4.3	Void	331

9.9.2.5	Measurements Period Requirements	331

9.9.2.6	Void	332

9.9.3	PRS-RSRP measurements	332

9.9.3.1	Introduction	332

9.9.3.2	Requirements applicability	333

9.9.3.3	Measurement Capability	333

9.9.3.4	Measurement Reporting Requirements	333

9.9.3.5	Measurement Period Requirements	333

9.9.4	UE Rx-Tx time difference measurements	334

9.9.4.1 Introduction	334

9.9.4.2 Requirements Applicability	335

9.9.4.3 Measurement Capability	335

9.9.4.4 Measurement Reporting Requirements	335

9.9.4.5 Measurement Period Requirements	335

Editors Note: The applicability of requirements if the time duration of a DL PRS resource exceeds the UE capability N.9.9.5	NR E-CID measurements	336

9.9.5.1	Introduction	336

9.9.5.2	Measurement Requirements	336

9.9.5.2.1	Intra-frequency Measurement Requirements	336

9.9.5.2.2	Inter-frequency Measurement Requirements	337

9.9.5.2.3	Measurement Reporting Delay	337

9.10	CSI-RS based L3 measurements	337

9.10.1	Introduction	337

9.10.2	CSI-RS based intra-frequency measurements	338

9.10.2.1	Introduction	338

9.10.2.2	Requirements applicability	338

9.10.2.3	Number of cells and number of CSI-RS	339

9.10.2.3.1	Requirements for FR1	339

9.10.2.3.2	Requirements for FR2	339

9.10.2.4	Measurement Reporting Requirements	339

9.10.2.4.1	Periodic Reporting	339

9.10.2.4.2	Event-triggered Periodic Reporting	339

9.10.2.4.3	Event Triggered Reporting	339

9.10.2.5	Intra-frequency measurements without measurement gaps	340

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	341

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	341

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	341

9.10.3	CSI-RS based Inter-frequency measurements	341

9.10.3.1	Introduction	341

9.10.3.2	Requirements applicability	341

9.10.3.3	Number of cells and number of CSI-RS resources	342

9.10.3.3.1	Requirements for FR1	342

9.10.3.3.2	Requirements for FR2	342

9.10.3.4	Measurements reporting requirements	342

9.10.3.4.1	Periodic Reporting	342

9.10.3.4.2	Event-triggered Periodic Reporting	342

9.10.3.4.3	Event-triggered Reporting	343

9.10.3.5	Inter frequency measurements with measurement gaps	343

9.11	NR measurements with autonomous gaps	344

9.11.1	Introduction	344

9.11.2	CGI identification of an NR cell with autonomous gaps	344

9.11.3	CGI reporting delay	345

10	Measurement Performance requirements	345

10.1	NR measurements	345

10.1.1	Introduction	345

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	346

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	346

10.1.2.1.1	Absolute SS-RSRP Accuracy	346

10.1.2.1.2	Relative SS-RSRP Accuracy	347

10.1.2.2	Void	347

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	347

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	347

10.1.3.1.1	Absolute SS-RSRP Accuracy	347

10.1.3.1.2	Relative SS-RSRP Accuracy	348

10.1.3.2	Void	349

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	349

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	349

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	349

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	349

10.1.4.2	Void	350

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	350

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	350

10.1.5.1.1	Absolute SS-RSRP Accuracy	350

10.1.5.1.2	Relative SS-RSRP Accuracy	351

10.1.5.2	Void	352

10.1.6 RSRP Measurement Report Mapping	352

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	354

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	354

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	354

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	355

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	355

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	355

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	356

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	356

10.1.9.1.1	Aboslute Accuracy of SS-RSRQ in FR1	356

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	357

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	358

10.1.11	RSRQ report mapping	360

10.1.12	Intra-frequency SINR accuracy requirements for FR1	360

10.1.13	Intra-frequency SINR accuracy requirements for FR2	361

10.1.14	Inter-frequency SINR accuracy requirements for FR1	362

10.1.15	Inter-frequency SINR accuracy requirements for FR2	364

10.1.16	SINR report mapping	366

10.1.17	Power Headroom	367

10.1.18	PCMAX,c,f	368

10.1.19	L1-RSRP accuracy requirements for FR1	368

10.1.20	L1-RSRP accuracy requirements for FR2	372

10.1.21	SFTD accuracy requirements	375

10.1.22	CLI measurement accuracy requirements	379

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	383

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	383

10.1.23	RSTD Measurements	385

10.1.23.1	Introduction	385

10.1.23.2	Measurement Accuracy Requirements	386

10.1.23.3	Report mapping	386

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	386

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	388

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	390

10.1.24	PRS-RSRP Measurements	392

10.1.24.1	Introduction	392

10.1.24.2	Measurement Accuracy Requirements	392

10.1.24.3	Report mapping	392

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	392

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	393

10.1.25	UE Rx-Tx Time Difference Measurements	395

10.1.25.1	Introduction	395

10.1.25.2	Measurement Accuracy Requirements	396

10.1.25.3	Report mapping	396

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	396

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	398

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	399

10.1.26	FR2 P-MPR report	401

10.1.26.1	Report mapping	401

10.2	E-UTRAN measurements	401

10.2.1	Introduction	401

10.2.2	E-UTRAN RSRP measurements	402

10.2.3	E-UTRAN RSRQ measurements	402

10.2.4	E-UTRAN RSTD measurements	402

10.2.5	E-UTRAN RS-SINR measurements	402

10.3	UTRAN FDD Measurements	403

10.3.1	UTRAN FDD CPICH RSCP	403

10.3.2	UTRAN FDD CPICH Ec/No	404

11	Void	404

12	V2X Requirements	404

12.1	Introduction	404

12.2	UE Transmit Timing	404

12.2.1	Introduction	404

12.2.2	GNSS as synchronization reference source	404

12.2.3	NR Cell as synchronization reference source	405

12.2.4	E-URTAN Cell as synchronization reference source	405

12.2.5	SyncRef UE as synchronization reference source	406

12.3	Initiation/Cease of SLSS Transmissions	406

12.3.1	Introduction	406

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	406

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	407

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	408

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	408

12.4	Selection / Reselection of V2X Synchronization Reference Source	408

12.5	L1 SL-RSRP measurements	409

12.5.1	Introduction	409

12.5.2	SL-RSRP measurements	409

12.6	Congestion Control measurements	410

12.7	Interruption	410

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	410

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	410

12.8	Reliability of GNSS signal	412

12.9	Scheduling availability	412

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	412

13	Measurement Performance Requirements for NR gNB	413

13.1	UL-RTOA	413

13.1.1	Report mapping	413

13.2	gNB Rx-Tx time difference	415

13.2.1	Report mapping	415

13.3	UL SRS RSRP measurement	417

13.3.1	Report mapping	417

13.4	AoA/ZoA	418

13.4.1	Report mapping	418

Annex A (normative):	Test Cases	408

A.1	Purpose of annex	408

A.2	Requirement classification for statistical testing	408

A.2.1	Types of requirements in TS 38.133	408

A.2.1.1	Time and delay requirements on UE higher layer actions	408

A.2.1.2	Measurements of power levels, relative powers and time	408

A.2.1.3	Implementation requirements	409

A.2.1.4	Physical layer timing requirements	409

A.3	RRM test configurations	410

A.3.1	Reference measurement channels	410

A.3.1.1	PDSCH	410

A.3.1.1.1	FDD	410

A.3.1.1.2	TDD	411

A.3.1.2	CORESET for RMSI scheduling	414

A.3.1.2.1	FDD	414

A.3.1.2.2	TDD	415

A.3.1.3	CORESET for RMC scheduling	418

A.3.1.3.1	FDD	418

A.3.1.3.2	TDD	419

A.3.1.4	TDD UL/DL configuration	422

A.3.2	OFDMA channel noise generator (OCNG)	423

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	423

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	423

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	424

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as PDSCH RMC	424

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	425

A.3.2.2	Void	425

A.3.3	Reference DRX configurations	425

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	425

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	426

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	426

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	426

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	427

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	427

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	427

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	428

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	428

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms	428

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	429

A.3.4	Test Cases with Different Channel Bandwidths	429

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	429

A.3.4.1.1	Introduction	429

A.3.4.1.2	Principle of testing	429

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	429

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	429

A.3.5.1.1	Introduction	429

A.3.5.1.2	Principle of Testing	429

A.3.6	Antenna configurations	430

A.3.6.1	Antenna configurations for FR1	430

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	430

A.3.6.1.1.1	Introduction	430

A.3.6.1.1.2	Principle of testing	430

A.3.6.2	Antenna configurations for FR2	432

A.3.7	EN-DC test setup	432

A.3.7.1	Introduction	432

A.3.7.2	E-UTRAN Serving Cell Parameters	432

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	432

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	434

A.3.7A	NR FR1-FR2 test setup	435

A.3.8	PRACH configurations	435

A.3.8.1	Introduction	435

A.3.8.2	PRACH configurations in FR1	435

A.3.8.2.1	FR1 PRACH configuration 1	435

A.3.8.2.2	FR1 PRACH configuration 2	436

A.3.8.2.3	FR1 PRACH configuration 3	437

A.3.8.2.4	FR1 PRACH configuration 4	438

A.3.8.3	PRACH configurations in FR2	439

A.3.8.3.1	FR2 PRACH configuration 1	439

A.3.8.3.2	FR2 PRACH configuration 2	440

A.3.8.3.3	FR2 PRACH configuration 3	441

A.3.8.3.4	FR2 PRACH configuration 4	442

A.3.9	BWP configurations	443

A.3.9.1	Introduction	443

A.3.9.2	Downlink BWP configurations	444

A.3.9.2.1	Initial BWP	444

A.3.9.2.2	Dedicated BWP	444

A.3.9.3	Uplink BWP configurations	444

A.3.9.3.1	Initial BWP	444

A.3.9.3.2	Dedicated BWP	445

A.3.10	SSB Configurations	445

A.3.10.1	SSB Configurations for FR1	445

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	445

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	447

A.3.11	SMTC Configurations	452

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	452

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	452

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	452

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	453

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	453

A.3.12	Test Cases with Different CC Configurations	453

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	453

A.3.12.1.1	Introduction	453

A.3.12.1.2	Principle of testing	453

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	453

A.3.12.2.1	Introduction	453

A.3.12.2.2	Principle of testing	453

A.3.13	Test Cases in SA and EN-DC Operations	454

A.3.13.1	Introduction	454

A.3.13.2	Principle of Testing	454

A.3.14	CSI-RS configurations	454

A.3.14.1	FDD	454

A.3.14.2	TDD	456

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	458

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	458

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	459

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	459

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	459

A.3.15.3	Setup 3: 2 AoAs	459

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	459

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	459

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	460

A.3.16	TCI State Configuration	460

A.3.16.1	Introduction	460

A.3.16.2	TCI states	460

A.3.17	Configurations of CSI-RS for tracking	461

A.3.17.1	Configuration of CSI-RS for tracking for FR1	461

A.3.17.1.1	FDD	461

A.3.17.1.2	TDD	462

A.3.17.2	Configuration of CSI-RS for tracking for FR2	463

A.3.17.2.1	TDD	463

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	464

A.3.18.1	Introduction	464

A.3.18.2	PRACH Power Measurement	464

A.3.19	Test applicability for DAPS handover	464

A.3.19.1	Introduction	464

A.3.19.2	Principle of testing	464

A.3.20	MsgA configurations	464

A.3.20.1	Introduction	464

A.3.20.2	MsgA configurations in FR1	465

A.3.20.2.1	FR1 MsgA configuration 1	465

A.3.20.2.2	FR1 MsgA configuration 2	466

A.3.20.3	MsgA configurations in FR2	468

A.3.20.3.1	FR2 MsgA configuration 1	468

A.3.20.3.2	FR2 MsgA configuration 2	469

A.4	EN-DC tests with all NR cells in FR1	471

A.4.1	Void	471

A.4.2	Void	471

A.4.3	RRC_CONNECTED state mobility	471

A.4.3.1	Void	471

A.4.3.2	RRC Connection Mobility Control	471

A.4.3.2.1	Void	471

A.4.3.2.2	Random Access	471

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	471

A.4.3.2.2.2	Non-contention based random access test in FR1 for PSCell in EN-DC	474

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	478

A.4.3.2.2.4	Non-contention based random access test for 2-step RA type in FR1 for PSCell in EN-DC	481

A.4.3.2.3	Void	485

A.4.4	Timing	485

A.4.4.1	UE transmit timing	485

A.4.4.1.1	NR UE Transmit Timing Test for FR1	485

A.4.4.1.1.1	Test Purpose and environment	485

A.4.4.1.1.2	Test requirements	489

A.4.4.2	UE timer accuracy	489

A.4.4.3	Timing advance	489

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	489

A.4.4.3.1.1	Test Purpose and Environment	489

A.4.4.3.1.2	Test Parameters	489

A.4.4.3.1.3	Test Requirements	493

A.4.5	Signaling characteristics	493

A.4.5.1	Radio link Monitoring	493

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	494

A.4.5.1.1.1	Test Purpose and Environment	494

A.4.5.1.1.2	Test Requirements	498

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	498

A.4.5.1.2.1	Test Purpose and Environment	498

A.4.5.1.2.2	Test Requirements	504

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	504

A.4.5.1.3.1	Test Purpose and Environment	504

A.4.5.1.3.2	Test Requirements	510

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	510

A.4.5.1.4.1	Test Purpose and Environment	510

A.4.5.1.4.2	Test Requirements	516

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	516

A.4.5.1.5.1	Test Purpose and Environment	516

A.4.5.1.5.2	Test Requirements	521

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	521

A.4.5.1.6.1	Test Purpose and Environment	521

A.4.5.1.6.2	Test Requirements	527

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	527

A.4.5.1.7.1	Test Purpose and Environment	527

A.4.5.1.7.2	Test Requirements	532

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	532

A.4.5.1.8.1	Test Purpose and Environment	532

A.4.5.1.8.2	Test Requirements	538

A.4.5.2	Interruption	538

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	538

A.4.5.2.1.1	Test Purpose and Environment	538

A.4.5.2.1.2	Test Requirements	542

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	542

A.4.5.2.2.1	Test Purpose and Environment	542

A.4.5.2.2.2	Test Requirements	546

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	546

A.4.5.2.3.1	Test Purpose and Environment	546

A.4.5.2.3.2	Test Requirements	550

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	550

A.4.5.2.4.1	Test Purpose and Environment	550

A.4.5.2.4.2	Test Requirements	554

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	554

A.4.5.2.5.1	Test Purpose and Environment	554

A.4.5.2.5.2	Test Requirements	558

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	558

A.4.5.2.6.1	Test Purpose and Environment	558

A.4.5.2.6.2	Test Requirements	562

A.4.5.2.7	Void	562

A.4.5.3	SCell Activation and Deactivation Delay	562

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	562

A.4.5.3.1.1	Test Purpose and Environment	562

A.4.5.3.1.2	Test Requirements	567

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 320 ms SCell measurement cycle	568

A.4.5.3.2.1	Test Purpose and Environment	568

A.4.5.3.2.2	Test Requirements	568

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	568

A.4.5.3.3.1	Test Purpose and Environment	568

A.4.5.3.3.2	Test Requirements	569

A.4.5.4	UE UL carrier RRC reconfiguration Delay	569

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	569

A.4.5.4.1.1 Test Purpose and Environment	569

A.4.5.4.1.2	Test Requirements	577

A.4.5.5	Beam Failure Detection and Link recovery procedures	577

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	577

A.4.5.5.1.1	Test Purpose and Environment	577

A.4.5.5.1.2	Test Requirements	583

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	583

A.4.5.5.2.1	Test Purpose and Environment	583

A.4.5.5.2.2	Test Requirements	589

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	589

A.4.5.5.3.1	Test Purpose and Environment	589

A.4.5.5.3.2	Test Requirements	595

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	595

A.4.5.5.4.1	Test Purpose and Environment	595

A.4.5.5.4.2	Test Requirements	601

A.4.5.6	Active BWP switch	601

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	601

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	601

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	607

A.4.5.6.2	RRC-based Active BWP Switch	611

A.4.5.7	PSCell addition and release delay	616

A.4.5.7.1	Addition and Release Delay of known NR PSCell	616

A.4.5.7.1.1	Test purpose and environment	616

A.4.5.7.1.2	Test Requirements	620

A.4.5.8	DL Interruptions at switching between two uplink carriers	620

A.4.5.8.1	Test Purpose and Environment	620

A.4.5.8.2	Test Requirements	624

A.4.6	Measurement procedure	624

A.4.6.1	Intra-frequency Measurements	624

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	624

A.4.6.1.1.1	Test purpose and Environment	624

A.4.6.1.1.2	Test parameters	624

A.4.6.1.1.3	Test Requirements	628

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	628

A.4.6.1.2.1	Test purpose and Environment	628

A.4.6.1.2.2	Test parameters	628

A.4.6.1.2.2	Test Requirements	632

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	632

A.4.6.1.3.1	Test purpose and Environment	632

A.4.6.1.3.2	Test parameters	632

A.4.6.1.3.3	Test Requirements	636

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	636

A.4.6.1.4.1	Test purpose and Environment	636

A.4.6.1.4.2	Test parameters	636

A.4.6.1.4.3	Test Requirements	640

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	640

A.4.6.1.5.1	Test purpose and Environment	640

A.4.6.1.5.2	Test parameters	640

A.4.6.1.5.3	Test Requirements	642

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	643

A.4.6.1.6.1	Test purpose and Environment	643

A.4.6.1.6.2	Test parameters	643

A.4.6.1.6.3	Test Requirements	645

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	646

A.4.6.1.7.1	Test purpose and Environment	646

A.4.6.1.7.2	Test parameters	646

A.4.6.1.7.3	Test Requirements	650

A.4.6.2	Inter-frequency Measurements	650

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	650

A.4.6.2.1.1	Test Purpose and Environment	650

A.4.6.2.1.2	Test Requirements	654

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	654

A.4.6.2.2.1	Test Purpose and Environment	654

A.4.6.2.2.2	Test Requirements	658

A.4.6.2.3	Void	659

A.4.6.2.4	Void	659

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	659

A.4.6.2.5.1	Test Purpose and Environment	659

A.4.6.2.5.2	Test Requirements	663

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	663

A.4.6.2.6.1	Test Purpose and Environment	663

A.4.6.2.6.2	Test Requirements	668

A.4.6.2.7	Void	669

A.4.6.2.8	Void	669

A.4.6.3	Void	669

A.4.6.4	L1-RSRP measurement for beam reporting	669

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	669

A.4.6.4.1.1	Test Purpose and Environment	669

A.4.6.4.1.2	Test parameters	669

A.4.6.4.1.3	Test Requirements	672

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	672

A.4.6.4.2.1	Test Purpose and Environment	672

A.4.6.4.2.2	Test parameters	673

A.4.6.4.2.3	Test Requirements	676

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	676

A.4.6.4.3.1	Test Purpose and Environment	676

A.4.6.4.3.2	Test parameters	677

A.4.6.4.3.3	Test Requirements	680

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	680

A.4.6.4.4.1	Test Purpose and Environment	680

A.4.6.4.4.2	Test parameters	681

A.4.6.4.4.3	Test Requirements	683

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	683

A.4.6.4.5.1	Test Purpose and Environment	683

A.4.6.4.5.2	Test parameters	684

A.4.6.4.2.3	Test Requirements	687

A.4.6.5	CLI measurements	687

A.4.6.5.1	SRS-RSRP measurement with non-DRX	687

A.4.6.5.1.1	Test Purpose and Environment	687

A.4.6.5.1.2	Test Parameters	688

A.4.6.5.1.3	Test Requirements	691

A.4.6.5.2	CLI-RSSI measurement with non-DRX	691

A.4.6.5.2.1	Test Purpose and Environment	691

A.4.6.5.2.2	Test Parameters	692

A.4.6.5.2.3	Test Requirements	693

A.4.7	Measurement Performance requirements	694

A.4.7.1	SS-RSRP	694

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	694

A.4.7.1.1.1	Test Purpose and Environment	694

A.4.7.1.1.2	Test parameters	694

A.4.7.1.1.3	Test Requirements	698

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	698

A.4.7.1.2.1	Test Purpose and Environment	698

A.4.7.1.2.2	Test parameters	699

A.4.7.1.2.3	Test Requirements	703

A.4.7.1.3	Void	703

A.4.7.2	SS-RSRQ	703

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	703

A.4.7.2.1.1	Test Purpose and Environment	703

A.4.7.2.1.2	Test Parameters	704

A.4.7.2.1.3	Test Requirements	709

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	709

A.4.7.2.2.1	Test Purpose and Environment	709

A.4.7.2.2.2	Test Parameters	709

A.4.7.2.2.3	Test Requirements	714

A.4.7.3	SS-SINR	714

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	714

A.4.7.3.1.1	Test Purpose and Environment	714

A.4.7.3.1.2	Test Parameters	714

A.4.7.3.1.3	Test Requirements	718

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	718

A.4.7.3.2.1	Test Purpose and Environment	718

A.4.7.3.2.2	Test Parameters	718

A.4.7.3.2.3	Test Requirements	723

A.4.7.4	L1-RSRP measurement for beam reporting	723

A.4.7.4.1	SSB based L1-RSRP measurement	723

A.4.7.4.1.1	Test Purpose and Environment	723

A.4.7.4.1.2	Test parameters	724

A.4.7.4.1.3	Test Requirements	727

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	727

A.4.7.4.2.1	Test Purpose and Environment	727

A.4.7.4.2.2	Test parameters	728

A.4.7.4.2.3	Test Requirements	731

A.4.7.5	SFTD accuracy	731

A.4.7.5.1	SFTD accuracy	731

A.4.7.5.1.1	Test Purpose and Environment	731

A.4.7.5.1.2	Test Parameters	731

A.4.7.5.1.3	Test Requirements	736

A.4.7.5.2	Void	736

A.4.7.5.3	Void	736

A.4.7.6	CLI measurements	736

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	736

A.4.7.6.1.1	Test Purpose and Environment	736

A.4.7.6.1.2	Test parameters	736

A.4.7.6.1.3	Test Requirements	742

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	742

A.4.7.6.2.1	Test Purpose and Environment	742

A.4.7.6.2.2	Test parameters	743

A.4.7.6.2.3	Test Requirements	746

A.4.8	Void	746

A.5	EN-DC tests with one or more NR cells in FR2	747

A.5.1	Void	747

A.5.2	Void	747

A.5.3	RRC_CONNECTED state mobility	747

A.5.3.1	Void	747

A.5.3.2	RRC Connection Mobility Control	747

A.5.3.2.1	Void	747

A.5.3.2.2	Random Access	747

A.5.3.2.2.1	Contention based random access test in FR2 for PSCell/SCell in EN-DC	747

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	751

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	755

A.5.3.2.2.4	2-step RA type SSB based non-contention based random access test in FR2 for PSCell/SCell in EN-DC	759

A.5.3.2.3	Void	763

A.5.4	Timing	763

A.5.4.1	UE transmit timing	763

A.5.4.1.1	NR UE Transmit Timing Test for FR2	763

A.5.4.1.1.1	Test Purpose and environment	763

A.5.4.1.1.2	Test requirements	766

A.5.4.2	UE timer accuracy	767

A.5.4.3	Timing advance	767

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	767

A.5.4.3.1.1 Test Purpose and Environment	767

A.5.4.3.1.2 Test Parameters	767

A.5.4.3.1.3	Test Requirements	770

A.5.5	Signaling characteristics	770

A.5.5.1	Radio link Monitoring	770

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	770

A.5.5.1.1.1	Test Purpose and Environment	770

A.5.5.1.1.2	Test Requirements	773

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	774

A.5.5.1.2.1	Test Purpose and Environment	774

A.5.5.1.2.2	Test Requirements	777

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	778

A.5.5.1.3.1	Test Purpose and Environment	778

A.5.5.1.3.2	Test Requirements	781

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	781

A.5.5.1.4.1	Test Purpose and Environment	781

A.5.5.1.4.2	Test Requirements	786

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	786

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	792

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	797

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	802

A.5.5.1.8.2	Test Requirements	808

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	808

A.5.5.1.9.1	Test Purpose and Environment	808

A.5.5.1.9.2	Test Requirements	810

A.5.5.2	Interruption	810

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	810

A.5.5.2.1.1	Test Purpose and Environment	810

A.5.5.2.1.2	Test Requirements	813

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	813

A.5.5.2.2.1	Test Purpose and Environment	813

A.5.5.2.2.2	Test Requirements	816

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	816

A.5.5.2.3.1	Test Purpose and Environment	816

A.5.5.2.3.2	Test Requirements	820

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	820

A.5.5.2.4.1	Test Purpose and Environment	820

A.5.5.2.4.2	Test Requirements	823

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	824

A.5.5.2.5.1	Test Purpose and Environment	824

A.5.5.2.5.2	Test Requirements	827

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	828

A.5.5.2.6.1	Test Purpose and Environment	828

A.5.5.2.6.2	Test Requirements	830

A.5.5.3	SCell Activation and Deactivation Delay	831

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	831

A.5.5.3.1.1	Test Purpose and Environment	831

A.5.5.3.1.2	Test Requirements	833

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	833

A.5.5.3.2.1	Test Purpose and Environment	833

A.5.5.3.2.2	Test Requirements	836

A.5.5.3.3	Void	836

A.5.5.3.4	Void	836

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	836

A.5.5.3.5.1	Test Purpose and Environment	836

A.5.5.3.5.2	Test Requirements	840

A.5.5.4	Void	841

A.5.5.5	Beam Failure Detection and Link recovery procedures	841

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	841

A.5.5.5.1.1	Test Purpose and Environment	841

A.5.5.5.1.2	Test Requirements	847

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	847

A.5.5.5.2.1	Test Purpose and Environment	847

A.5.5.5.2.2	Test Requirements	853

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	853

A.5.5.5.3.1	Test Purpose and Environment	853

A.5.5.5.3.2	Test Requirements	859

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	859

A.5.5.5.4.1	Test Purpose and Environment	859

A.5.5.5.4.2	Test Requirements	865

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	865

A.5.5.5.5.1	Test Purpose and Environment	865

A.5.5.5.5.2	Test Requirements	871

A.5.5.6	Active BWP switch	871

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	871

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	871

A.5.5.6.1.1.1	Test Purpose and Environment	871

A.5.5.6.1.1.2	Test Requirements	875

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 DL active BWP switch with FR2 SCell in non-DRX in synchronous EN-DC	876

A.5.5.6.2	RRC-based Active BWP Switch	880

A.5.5.7	PSCell addition and release delay	884

A.5.5.7.1	Addition and Release Delay of NR PSCell	884

A.5.5.7.1.1	Test purpose and environment	884

A.5.5.7.1.2	Test Requirements	887

A.5.5.8	Active TCI state switch delay	888

A.5.5.8.1	MAC-CE based active TCI state switch	888

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	888

A.5.5.8.1.1.1	Test Purpose and Environment	888

A.5.5.8.1.1.2	Test Requirements	890

A.5.5.8.2	RRC based active TCI state switch	890

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	890

A.5.5.8.2.1.1	Test Purpose and Environment	890

A.5.5.8.2.1.2	Test Requirements	893

A.5.6	Measurement procedure	893

A.5.6.1	Intra-frequency Measurements	893

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	893

A.5.6.1.1.1	Test purpose and Environment	893

A.5.6.1.1.2	Test Requirements	896

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	896

A.5.6.1.2.1	Test purpose and Environment	896

A.5.6.1.2.2	Test Requirements	899

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	900

A.5.6.1.3.1	Test purpose and Environment	900

A.5.6.1.3.2	Test Requirements	903

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	903

A.5.6.1.4.1	Test purpose and Environment	903

A.5.6.1.4.2	Test Requirements	907

A.5.6.2	Inter-frequency Measurements	908

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	908

A.5.6.2.1.1	Test Purpose and Environment	908

A.5.6.2.1.2	Test Requirements	911

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	911

A.5.6.2.2.1	Test Purpose and Environment	911

A.5.6.2.2.2	Test Requirements	915

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	915

A.5.6.2.3.1	Test Purpose and Environment	915

A.5.6.2.3.2	Test Requirements	919

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	919

A.5.6.2.4.1	Test Purpose and Environment	919

A.5.6.2.4.2	Test Requirements	922

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	922

A.5.6.2.5.1	Test Purpose and Environment	922

A.5.6.2.5.2	Test Requirements	927

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	928

A.5.6.2.6.1	Test Purpose and Environment	928

A.5.6.2.6.2	Test Requirements	931

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	931

A.5.6.2.7.1	Test Purpose and Environment	931

A.5.6.2.7.2	Test Requirements	935

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	935

A.5.6.2.8.1	Test Purpose and Environment	935

A.5.6.2.8.2	Test Requirements	939

A.5.6.3	L1-RSRP measurement for beam reporting	940

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	940

A.5.6.3.1.1	Test Purpose and Environment	940

A.5.6.3.1.2	Test parameters	940

A.5.6.3.1.3	Test Requirements	942

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	942

A.5.6.3.2.1	Test Purpose and Environment	942

A.5.6.3.2.2	Test parameters	943

A.5.6.3.2.3	Test Requirements	945

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	946

A.5.6.3.3.1	Test Purpose and Environment	946

A.5.6.3.3.2	Test parameters	946

A.5.6.3.3.3	Test Requirements	948

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	949

A.5.6.3.4.1	Test Purpose and Environment	949

A.5.6.3.4.2	Test parameters	949

A.5.6.3.4.3	Test Requirements	951

A.5.6.4	CLI measurements	952

A.5.6.4.1	SRS-RSRP measurement with DRX	952

A.5.6.4.1.1	Test Purpose and Environment	952

A.5.6.4.1.2	Test Parameters	952

A.5.6.4.1.3	Test Requirements	954

A.5.6.4.2	CLI-RSSI measurement with DRX	954

A.5.6.4.2.1	Test Purpose and Environment	954

A.5.6.4.2.2	Test Parameters	955

A.5.6.4.2.3	Test Requirements	957

A.5.7	Measurement Performance requirements	957

A.5.7.1	SS-RSRP	957

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	957

A.5.7.1.1.1	Test Purpose and Environment	957

A.5.7.1.1.2	Test parameters	957

A.5.7.1.1.3	Test Requirements	959

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	960

A.5.7.1.2.1	Test Purpose and Environment	960

A.5.7.1.2.2	Test parameters	960

A.5.7.1.2.3	Test Requirements	964

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	965

A.5.7.1.3.1	Test Purpose and Environment	965

A.5.7.1.3.2	Test parameters	965

A.5.7.1.3.3	Test Requirements	968

A.5.7.2	SS-RSRQ	968

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	968

A.5.7.2.1.1	Test Purpose and Environment	968

A.5.7.2.1.2	Test Parameters	969

A.5.7.2.1.3	Test Requirements	973

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	973

A.5.7.2.2.1	Test Purpose and Environment	973

A.5.7.2.2.2	Test Parameters	973

A.5.7.2.2.3	Test Requirements	976

A.5.7.3	SS-SINR	976

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	976

A.5.7.3.1.1	Test Purpose and Environment	976

A.5.7.3.1.2	Test Parameters	976

A.5.7.3.1.3	Test Requirements	979

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	979

A.5.7.3.2.1	Test Purpose and Environment	979

A.5.7.3.2.2	Test Parameters	979

A.5.7.3.2.3	Test Requirements	981

A.5.7.4	L1-RSRP measurement for beam reporting	982

A.5.7.4.1	SSB based L1-RSRP measurement	982

A.5.7.4.1.1	Test Purpose and Environment	982

A.5.7.4.1.2	Test parameters	982

A.5.7.4.1.3	Test Requirements	984

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	985

A.5.7.4.2.1	Test Purpose and Environment	985

A.5.7.4.2.2	Test parameters	985

A.5.7.4.2.3	Test Requirements	987

A.5.7.5	CLI measurements	988

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	988

A.5.7.5.1.1	Test Purpose and Environment	988

A.5.7.5.1.2	Test parameters	988

A.5.7.5.1.3	Test Requirements	991

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	992

A.5.7.5.2.1	Test Purpose and Environment	992

A.5.7.5.2.2	Test parameters	992

A.5.7.5.2.3	Test Requirements	994

A.5.8	Void	995

A.6	NR standalone tests with all NR cells in FR1	959

A.6.1	SA: RRC_IDLE state mobility	959

A.6.1.1	Cell re-selection to NR	959

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	959

A.6.1.1.1.1	Test Purpose and Environment	959

A.6.1.1.1.2	Test Parameters	959

A.6.1.1.1.3	Test Requirements	963

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	963

A.6.1.1.2.1	Test Purpose and Environment	963

A.6.1.1.2.2	Test Parameters	963

A.6.1.1.2.3	Test Requirements	967

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	967

A.6.1.1.3.1	Test Purpose and Environment	967

A.6.1.1.3.2	Test Parameters	967

A.6.1.1.3.3	Test Requirements	972

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	972

A.6.1.1.4.1	Test Purpose and Environment	972

A.6.1.1.4.2	Test Parameters	972

A.6.1.1.4.3	Test Requirements	976

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	976

A.6.1.1.5.1	Test Purpose and Environment	976

A.6.1.1.5.2	Test Parameters	976

A.6.1.1.5.3	Test Requirements	980

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	980

A.6.1.1.6.1	Test Purpose and Environment	980

A.6.1.1.6.2	Test Parameters	981

A.6.1.1.6.3	Test Requirements	985

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	986

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	986

A.6.1.2.1.1	Test Purpose and Environment	986

A.6.1.2.1.2	Test Parameters	986

A.6.1.2.1.3	Test Requirements	989

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	990

A.6.1.2.2.1	Test Purpose and Environment	990

A.6.1.2.2.2	Test Parameters	990

A.6.1.2.2.3	Test Requirements	993

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	994

A.6.1.2.3.1	Test Purpose and Environment	994

A.6.1.2.3.2	Test Parameters	994

A.6.1.2.3.3	Test Requirements	997

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	998

A.6.1.2.4.1	Test Purpose and Environment	998

A.6.1.2.4.2	Test Parameters	998

A.6.1.2.4.3	Test Requirements	1002

A.6.2	SA: RRC_INACTIVE state mobility	1005

A.6.3	RRC_CONNECTED state mobility	1005

A.6.3.1	Handover	1005

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1005

A.6.3.1.1.1	Test Purpose and Environment	1005

A.6.3.1.1.2	Test Parameters	1006

A.6.3.1.1.3 Test Requirements	1009

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1009

A.6.3.1.2.1	Test Purpose and Environment	1009

A.6.3.1.2.2	Test Parameters	1009

A.6.3.1.2.3	Test Requirements	1013

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1013

A.6.3.1.3.1	Test Purpose and Environment	1013

A.6.3.1.3.2	Test Parameters	1013

A.6.3.1.3.3 Test Requirements	1017

A.6.3.1.4	 SA NR - E-UTRAN handover	1017

A.6.3.1.4.1	Test Purpose and Environment	1017

A.6.3.1.4.2	Test Requirements	1023

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1023

A.6.3.1.5.1	Test Purpose and Environment	1023

A.6.3.1.5.2	Test Requirements	1029

A.6.3.1.6	 SA NR - UTRAN FDD handover	1029

A.6.3.1.6.1	Test Purpose and Environment	1029

A.6.3.1.6.2	Test Requirements	1033

A.6.3.1.7	Intra-frequency handover in FR1; synchronous scenario	1033

A.6.3.1.7.1	Test Purpose and Environment	1033

A.6.3.1.7.2	Test Parameters	1033

A.6.3.1.7.3 Test Requirements	1037

A.6.3.1.8	Intra-frequency handover in FR1; asynchronous scenario	1038

A.6.3.1.8.1	Test Purpose and Environment	1038

A.6.3.1.8.2	Test Parameters	1038

A.6.3.1.8.3 Test Requirements	1042

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1043

A.6.3.1.9.1	Test Purpose and Environment	1043

A.6.3.1.9.2	Test Parameters	1043

A.6.3.1.9.3 Test Requirements	1047

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1047

A.6.3.1.10.1	Test Purpose and Environment	1047

A.6.3.1.10.2	Test Parameters	1047

A.6.3.1.10.3 Test Requirements	1051

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1051

A.6.3.1.11.1	Test Purpose and Environment	1051

A.6.3.1.11.2	Test Parameters	1051

A.6.3.1.11.3 Test Requirements	1058

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1058

A.6.3.1.12.1	Test Purpose and Environment	1058

A.6.3.1.12.2	Test Parameters	1058

A.6.3.1.12.3 Test Requirements	1065

A.6.3.2	RRC Connection Mobility Control	1065

A.6.3.2.1	SA: RRC Re-establishment	1065

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1065

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1069

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1073

A.6.3.2.2	Random Access	1076

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1076

A.6.3.2.2.2	4-step RA type non-Contention based random access test in FR1 for NR standalone	1080

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1085

A.6.3.2.2.4	2-step RA type SSB based non-contention based test in FR1 for NR standalone	1089

A.6.3.2.3	SA: RRC Connection Release with Redirection	1093

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1093

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1097

A.6.3.3 Conditional handover	1104

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1104

A.6.3.3.1.1	Test Purpose and Environment	1104

A.6.3.3.1.2	Test Parameters	1104

A.6.3.3.1.3 Test Requirements	1108

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1108

A.6.3.3.2.1	Test Purpose and Environment	1108

A.6.3.3.2.2	Test Parameters	1108

A.6.3.3.2.3 Test Requirements	1112

A.6.4	Timing	1112

A.6.4.1	UE transmit timing	1112

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1112

A.6.4.1.1.1	Test Purpose and environment	1112

A.6.4.1.1.2	Test requirements	1116

A.6.4.2	UE timer accuracy	1116

A.6.4.3	Timing advance	1116

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1116

A.6.4.3.1.1	Test Purpose and Environment	1116

A.6.4.3.1.2	Test Parameters	1116

A.6.4.3.1.3	Test Requirements	1120

A.6.5	Signalling characteristics	1120

A.6.5.1	Radio link Monitoring	1120

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1121

A.6.5.1.1.1	Test Purpose and Environment	1121

A.6.5.1.1.2	Test Requirements	1126

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1126

A.6.5.1.2.1	Test Purpose and Environment	1126

A.6.5.1.2.2	Test Requirements	1132

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1132

A.6.5.1.3.1	Test Purpose and Environment	1132

A.6.5.1.3.2	Test Requirements	1138

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1138

A.6.5.1.4.1	Test Purpose and Environment	1138

A.6.5.1.4.2	Test Requirements	1144

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1144

A.6.5.1.5.1	Test Purpose and Environment	1144

A.6.5.1.5.2	Test Requirements	1149

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1149

A.6.5.1.6.1	Test Purpose and Environment	1149

A.6.5.1.6.2	Test Requirements	1155

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1155

A.6.5.1.7.1	Test Purpose and Environment	1155

A.6.5.1.7.2	Test Requirements	1159

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1159

A.6.5.1.8.1	Test Purpose and Environment	1159

A.6.5.1.8.2	Test Requirements	1165

A.6.5.2	Interruption	1165

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1165

A.6.5.2.1.2	Test Requirements	1169

A.6.5.3	SCell Activation and Deactivation Delay	1169

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1169

A.6.5.3.1.1	Test Purpose and Environment	1169

A.6.5.3.1.2	Test Requirements	1174

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 320ms SCell measurement cycle	1174

A.6.5.3.2.1	Test Purpose and Environment	1174

A.6.5.3.2.2	Test Requirements	1175

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1175

A.6.5.3.3.1	Test Purpose and Environment	1175

A.6.5.3.3.2	Test Requirements	1175

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1176

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1176

A.6.5.4.1.1	Test Purpose and Environment	1176

A.6.5.4.1.2	Test Requirements	1185

A.6.5.4.2	Void	1185

A.6.5.5	Beam Failure Detection and Link recovery procedures	1185

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1185

A.6.5.5.1.1	Test Purpose and Environment	1185

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1191

A.6.5.5.2.1	Test Purpose and Environment	1191

A.6.5.5.2.2	Test Requirements	1198

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1198

A.6.5.5.3.1	Test Purpose and Environment	1198

A.6.5.5.3.2	Test Requirements	1204

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1204

A.6.5.5.4.1	Test Purpose and Environment	1204

A.6.5.5.4.2	Test Requirements	1210

A.6.5.6	Active BWP switch	1210

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	1210

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	1210

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	1215

A.6.5.6.2	RRC-based Active BWP Switch	1220

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	1220

A.6.5.7	DL interruptions at switching between two uplink carriers	1224

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	1224

A.6.5.7.1.1	Test Purpose and Environment	1224

A.6.5.7.1.2	Test Requirements	1228

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	1228

A.6.5.7.2.1	Test Purpose and Environment	1228

A.6.5.7.2.2	Test Requirements	1232

A.6.6	Measurement procedure	1232

A.6.6.1	Intra-frequency Measurements	1232

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	1232

A.6.6.1.1.1	Test purpose and Environment	1232

A.6.6.1.1.2	Test parameters	1232

A.6.6.1.1.3	Test Requirements	1236

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	1236

A.6.6.1.2.1	Test purpose and Environment	1236

A.6.6.1.2.2	Test parameters	1236

A.6.6.1.2.3	Test Requirements	1240

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	1240

A.6.6.1.3.1	Test purpose and Environment	1240

A.6.6.1.3.2	Test parameters	1240

A.6.6.1.3.3	Test Requirements	1244

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	1244

A.6.6.1.4.1	Test purpose and Environment	1244

A.6.6.1.4.2	Test parameters	1244

A.6.6.1.4.3	Test Requirements	1248

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	1248

A.6.6.1.5.1	Test purpose and Environment	1248

A.6.6.1.5.2	Test parameters	1248

A.6.6.1.5.3	Test Requirements	1250

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	1251

A.6.6.1.6.1	Test purpose and Environment	1251

A.6.6.1.6.2	Test parameters	1251

A.6.6.1.6.3	Test Requirements	1252

A.6.6.2	Inter-frequency Measurements	1253

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1253

A.6.6.2.1.1	Test Purpose and Environment	1253

A.6.6.2.1.2	Test Requirements	1257

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1257

A.6.6.2.2.1	Test Purpose and Environment	1257

A.6.6.2.2.2	Test Requirements	1261

A.6.6.2.3	Void	1262

A.6.6.2.4	Void	1262

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1262

A.6.6.2.5.1	Test Purpose and Environment	1262

A.6.6.2.5.2	Test Requirements	1266

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1266

A.6.6.2.6.1	Test Purpose and Environment	1266

A.6.6.2.6.2	Test Requirements	1270

A.6.6.2.7	Void	1271

A.6.6.2.8	Void	1271

A.6.6.3	Inter-RAT Measurements	1271

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	1271

A.6.6.3.1.1	Test Purpose and Environment	1271

A.6.6.3.1.2	Test Requirements	1277

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	1277

A.6.6.3.2.1	Test Purpose and Environment	1277

A.6.6.3.2.2	Test Requirements	1284

A.6.6.4	L1-RSRP measurement for beam reporting	1284

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1284

A.6.6.4.1.1	Test Purpose and Environment	1284

A.6.6.4.1.2	Test parameters	1284

A.6.6.4.1.3	Test Requirements	1287

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	1287

A.6.6.4.2.1	Test Purpose and Environment	1287

A.6.6.4.2.2	Test parameters	1288

A.6.6.4.2.3	Test Requirements	1291

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1291

A.6.6.4.3.1	Test Purpose and Environment	1291

A.6.6.4.3.2	Test parameters	1292

A.6.6.4.3.3	Test Requirements	1295

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1295

A.6.6.4.4.1	Test Purpose and Environment	1295

A.6.6.4.4.2	Test parameters	1296

A.6.6.4.4.3	Test Requirements	1299

A.6.6.5	1299

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	1299

A.6.6.5.1.1	Test Purpose and Environment	1299

A.6.6.5.1.2	Test Requirements	1303

A.6.6.6	CLI measurements	1303

A.6.6.6.1	SRS-RSRP measurement with DRX	1303

A.6.6.6.1.1	Test Purpose and Environment	1303

A.6.6.6.1.2	Test Parameters	1304

A.6.6.6.1.3	Test Requirements	1307

A.6.6.6.2	CLI-RSSI measurement with DRX	1307

A.6.6.6.2.1	Test Purpose and Environment	1307

A.6.6.6.2.2	Test Parameters	1308

A.6.6.6.2.3	Test Requirements	1310

A.6.7	Measurement Performance requirements	1310

A.6.7.1	SS-RSRP	1310

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1310

A.6.7.1.1.1	Test Purpose and Environment	1310

A.6.7.1.1.2	Test parameters	1310

A.6.7.1.1.3	Test Requirements	1315

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1315

A.6.7.1.2.1	Test Purpose and Environment	1315

A.6.7.1.2.2	Test parameters	1315

A.6.7.1.2.3	Test Requirements	1319

A.6.7.1.3	Void	1319

A.6.7.2	SS-RSRQ	1319

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1319

A.6.7.2.1.1	Test Purpose and Environment	1319

A.6.7.2.1.2	Test Parameters	1320

A.6.7.2.1.3	Test Requirements	1325

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1325

A.6.7.2.2.1	Test Purpose and Environment	1325

A.6.7.2.2.2	Test Parameters	1325

A.6.7.2.2.3	Test Requirements	1330

A.6.7.3	SS-SINR	1330

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1330

A.6.7.3.1.1	Test Purpose and Environment	1330

A.6.7.3.1.2	Test Parameters	1330

A.6.7.3.1.3	Test Requirements	1335

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1335

A.6.7.3.2.1	Test Purpose and Environment	1335

A.6.7.3.2.2	Test Parameters	1335

A.6.7.3.2.3	Test Requirements	1340

A.6.7.4	L1-RSRP measurement for beam reporting	1341

A.6.7.4.1	SSB based L1-RSRP measurement	1341

A.6.7.4.1.1	Test Purpose and Environment	1341

A.6.7.4.1.2	Test parameters	1341

A.6.7.4.1.3	Test Requirements	1345

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1345

A.6.7.4.2.1	Test Purpose and Environment	1345

A.6.7.4.2.2	Test parameters	1346

A.6.7.4.2.3	Test Requirements	1350

A.6.7.5	E-UTRAN RSRP	1350

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1350

A.6.7.5.1.1	Test Purpose and Environment	1350

A.6.7.5.1.2	Test parameters	1351

A.6.7.5.1.3	Test Requirements	1355

A.6.7.6	E-UTRAN RSRQ	1356

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1356

A.6.7.6.1.1	Test Purpose and Environment	1356

A.6.7.6.1.2	Test parameters	1356

A.6.7.6.1.3	Test Requirements	1360

A.6.7.7	E-UTRAN RS-SINR	1361

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1361

A.6.7.7.1.1	Test Purpose and Environment	1361

A.6.7.7.1.2	Test parameters	1361

A.6.7.7.1.3	Test Requirements	1365

A.6.7.8	CLI measurements	1366

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	1366

A.6.7.8.1.1	Test Purpose and Environment	1366

A.6.7.8.1.2	Test parameters	1366

A.6.7.8.1.3	Test Requirements	1372

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	1372

A.6.7.8.2.1	Test Purpose and Environment	1372

A.6.7.8.2.2	Test parameters	1373

A.6.7.8.2.3	Test Requirements	1376

A.7	NR standalone tests with one or more NR cells in FR2	1377

A.7.1	SA: RRC_IDLE state mobility	1377

A.7.1.1	Cell re-selection to NR	1377

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	1377

A.7.1.1.1.1	Test Purpose and Environment	1377

A.7.1.1.1.2	Test Parameters	1377

A.7.1.1.1.3	Test Requirements	1381

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	1381

A.7.1.1.2.1	Test Purpose and Environment	1381

A.7.1.1.2.2	Test Parameters	1381

A.7.1.1.2.3	Test Requirements	1385

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1385

A.7.1.1.3.1	Test Purpose and Environment	1385

A.7.1.1.3.2	Test Parameters	1385

A.7.1.1.3.3	Test Requirements	1388

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1388

A.7.1.1.4.1	Test Purpose and Environment	1388

A.7.1.1.4.2	Test Parameters	1388

A.7.1.1.4.3	Test Requirements	1391

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1391

A.7.1.1.5.1	Test Purpose and Environment	1391

A.7.1.1.5.2	Test Parameters	1391

A.7.1.1.5.3	Test Requirements	1394

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1394

A.7.1.1.6.1	Test Purpose and Environment	1394

A.7.1.1.6.2	Test Parameters	1394

A.7.1.1.6.3	Test Requirements	1397

A.7.2	SA: RRC_INACTIVE state mobility	1397

A.7.3	RRC_CONNECTED state mobility	1397

A.7.3.1	Handover	1397

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	1397

A.7.3.1.1.1	Test Purpose and Environment	1397

A.7.3.1.1.2	Test Parameters	1397

A.7.3.1.1.3	Test Requirements	1401

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	1401

A.7.3.1.2.1	Test Purpose and Environment	1401

A.7.3.1.2.2	Test Parameters	1401

A.7.3.1.2.3	Test Requirements	1404

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	1404

A.7.3.1.3.1	Test Purpose and Environment	1404

A.7.3.1.3.2	Test Parameters	1404

A.7.3.1.3.3 Test Requirements	1407

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	1407

A.7.3.1.4.1	Test Purpose and Environment	1407

A.7.3.1.4.2	Test Parameters	1407

A.7.3.1.4.3 Test Requirements	1414

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	1414

A.7.3.1.5.1	Test Purpose and Environment	1414

A.7.3.1.5.2	Test Parameters	1414

A.7.3.1.5.3 Test Requirements	1420

A.7.3.2	RRC Connection Mobility Control	1420

A.7.3.2.1	SA: RRC Re-establishment	1420

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	1420

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	1423

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	1426

A.7.3.2.1.3.1	Test Purpose and Environment	1426

A.7.3.2.1.3.2	Test Requirements	1428

A.7.3.2.2	Random Access	1429

A.7.3.2.2.1	Contention based random access test in FR2 for NR Standalone	1429

A.7.3.2.2.2	Non-contention based random access test in FR2 for NR Standalone	1433

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	1437

A.7.3.2.2.4	Non-contention based random access test for 2-step RA type in FR2 for NR Standalone	1440

A.7.3.2.3	SA: RRC Connection Release with Redirection	1443

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	1443

A.7.3.3 Conditional Handover	1447

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	1447

A.7.3.3.1.1	Test Purpose and Environment	1447

A.7.3.3.1.2	Test Parameters	1447

A.7.3.3.1.2.3	Test Requirements	1450

A.7.3.3.2	Inter-frequency handover from FR2 to FR2; unknown target cell	1450

A.7.3.3.2.1	Test Purpose and Environment	1450

A.7.3.3.2.2	Test Parameters	1450

A.7.3.3.2.3 Test Requirements	1452

A.7.4	Timing	1452

A.7.4.1	UE transmit timing	1452

A.7.4.1.1	NR UE Transmit Timing Test for FR2	1452

A.7.4.1.1.1	Test Purpose and environment	1452

A.7.4.1.1.2	Test requirements	1455

A.7.4.2	UE timer accuracy	1456

A.7.4.3	Timing advance	1456

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	1456

A.7.4.3.1.1	Test Purpose and Environment	1456

A.7.4.3.1.2	Test Parameters	1456

A.7.4.3.1.3 Test Requirements	1459

A.7.5	Signaling characteristics	1460

A.7.5.1	Radio link Monitoring	1460

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1460

A.7.5.1.1.1	Test Purpose and Environment	1460

A.7.5.1.1.2	Test Requirements	1463

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1464

A.7.5.1.2.1	Test Purpose and Environment	1464

A.7.5.1.2.2	Test Requirements	1468

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1469

A.7.5.1.3.1	Test Purpose and Environment	1469

A.7.5.1.3.2	Test Requirements	1472

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1472

A.7.5.1.4.1	Test Purpose and Environment	1472

A.7.5.1.4.2	Test Requirements	1477

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1477

A.7.5.1.5.1	Test Purpose and Environment	1477

A.7.5.1.5.2	Test Requirements	1481

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1481

A.7.5.1.6.1	Test Purpose and Environment	1481

A.7.5.1.6.2	Test Requirements	1485

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1485

A.7.5.1.7.1	Test Purpose and Environment	1485

A.7.5.1.7.2	Test Requirements	1488

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1488

A.7.5.1.8.1	Test Purpose and Environment	1488

A.7.5.1.8.2	Test Requirements	1492

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	1492

A.7.5.1.9.1	Test Purpose and Environment	1492

A.7.5.1.9.2	Test Requirements	1493

A.7.5.2	Interruption	1494

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	1494

A.7.5.3	SCell Activation and Deactivation Delay	1497

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	1497

A.7.5.3.1.1	Test Purpose and Environment	1497

A.7.5.3.1.2	Test Requirements	1499

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	1499

A.7.5.3.2.1	Test Purpose and Environment	1499

A.7.5.3.2.1	Test Purpose and Environment	1499

A.7.5.3.2.2	Test Requirements	1504

A.7.5.4	Void	1505

A.7.5.5	Beam Failure Detection and Link recovery procedures	1505

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1505

A.7.5.5.1.1	Test Purpose and Environment	1505

A.7.5.5.1.2	Test Requirements	1510

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	1510

A.7.5.5.2.1	Test Purpose and Environment	1510

A.7.5.5.2.2	Test Requirements	1515

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1515

A.7.5.5.3.1	Test Purpose and Environment	1515

A.7.5.5.3.2	Test Requirements	1521

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	1521

A.7.5.5.4.1	Test Purpose and Environment	1521

A.7.5.5.4.2	Test Requirements	1527

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1527

A.7.5.5.5.1	Test Purpose and Environment	1527

A.7.5.5.5.2	Test Requirements	1532

A.7.5.6	Active BWP switch	1532

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	1532

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of PCell with non-DRX in SA	1532

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of PCell with non-DRX in SA	1537

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	1542

A.7.5.6.1.3.1	Test Purpose and Environment	1542

A.7.5.6.1.3.2	Test Requirements	1545

A.7.5.6.2	RRC-based Active BWP Switch	1545

A.7.5.7	PSCell addition and release delay	1549

A.7.5.7.1	Addition and Release Delay of known NR PSCell	1549

A.7.5.7.1.1	Test Purpose and Environment	1549

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	1553

A.7.5.7.2.1	Test Purpose and Environment	1553

A.7.5.7.2.2	Test Requirements	1557

A.7.5.8	Active TCI state switch delay	1557

A.7.5.8.1	MAC-CE based active TCI state switch	1557

A.7.5.8.2	RRC based active TCI state switch	1559

A.7.6	Measurement procedure	1562

A.7.6.1	Intra-frequency Measurements	1562

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	1562

A.7.6.1.1.1	Test purpose and Environment	1562

A.7.6.1.1.2	Test Requirements	1565

A.7.6.1.2	SA event triggered reporting test without gap under DRX	1565

A.7.6.1.2.1	Test purpose and Environment	1565

A.7.6.1.2.2	Test Requirements	1568

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	1569

A.7.6.1.3.1	Test purpose and Environment	1569

A.7.6.1.3.2	Test Requirements	1571

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	1571

A.7.6.1.4.1	Test purpose and Environment	1571

A.7.6.1.4.2	Test Requirements	1574

A.7.6.2	Inter-frequency Measurements	1575

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	1575

A.7.6.2.1.1	Test Purpose and Environment	1575

A.7.6.2.1.2	Test Requirements	1578

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1578

A.7.6.2.2.1	Test Purpose and Environment	1578

A.7.6.2.2.2	Test Requirements	1582

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1582

A.7.6.2.3.1	Test Purpose and Environment	1582

A.7.6.2.3.2	Test Requirements	1586

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1586

A.7.6.2.4.1	Test Purpose and Environment	1586

A.7.6.2.4.2	Test Requirements	1590

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1590

A.7.6.2.5.1	Test Purpose and Environment	1590

A.7.6.2.5.2	Test Requirements	1594

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1595

A.7.6.2.6.1	Test Purpose and Environment	1595

A.7.6.2.6.2	Test Requirements	1599

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1600

A.7.6.2.7.1	Test Purpose and Environment	1600

A.7.6.2.7.2	Test Requirements	1604

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1605

A.7.6.2.8.1	Test Purpose and Environment	1605

A.7.6.2.8.2	Test Requirements	1609

A.7.6.3	L1-RSRP measurement for beam reporting	1610

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1610

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1610

A.7.6.3.1.1	Test Purpose and Environment	1610

A.7.6.3.1.2	Test parameters	1610

A.7.6.3.1.3	Test Requirements	1612

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1612

A.7.6.3.2.1	Test Purpose and Environment	1612

A.7.6.3.2.2	Test parameters	1613

A.7.6.3.2.3	Test Requirements	1615

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1615

A.7.6.3.3.1	Test Purpose and Environment	1615

A.7.6.3.3.2	Test parameters	1616

A.7.6.3.3.3	Test Requirements	1618

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1619

A.7.6.3.4.1	Test Purpose and Environment	1619

A.7.6.3.4.2	Test parameters	1619

A.7.6.3.3.3	Test Requirements	1621

A.7.6.4	CLI measurements	1622

A.7.6.4.1	SRS-RSRP measurement with non-DRX	1622

A.7.6.4.1.1	Test Purpose and Environment	1622

A.7.6.4.1.2	Test Parameters	1622

A.7.6.4.1.3	Test Requirements	1624

A.7.6.4.2	CLI-RSSI measurement with non-DRX	1624

A.7.6.4.2.1	Test Purpose and Environment	1624

A.7.6.4.2.2	Test Parameters	1625

A.7.6.4.2.3	Test Requirements	1626

A.7.7	Measurement Performance requirements	1627

A.7.7.1	SS-RSRP	1627

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1627

A.7.7.1.1.1	Test Purpose and Environment	1627

A.7.7.1.1.2	Test parameters	1627

A.7.7.1.1.3	Test Requirements	1631

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1631

A.7.7.1.2.1	Test Purpose and Environment	1631

A.7.7.1.2.2	Test parameters	1632

A.7.7.1.2.3	Test Requirements	1635

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1636

A.7.7.1.3.1	Test Purpose and Environment	1636

A.7.7.1.3.2	Test parameters	1636

A.7.7.1.3.3	Test Requirements	1639

A.7.7.2	SS-RSRQ	1640

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1640

A.7.7.2.1.1	Test Purpose and Environment	1640

A.7.7.2.1.2	Test Parameters	1640

A.7.7.2.1.3	Test Requirements	1642

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1642

A.7.7.2.2.1	Test Purpose and Environment	1642

A.7.7.2.2.2	Test Parameters	1642

A.7.7.2.2.3	Test Requirements	1644

A.7.7.3	SS-SINR	1644

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1644

A.7.7.3.1.1	Test Purpose and Environment	1644

A.7.7.3.1.2	Test Parameters	1644

A.7.7.3.1.3	Test Requirements	1647

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1647

A.7.7.3.2.1	Test Purpose and Environment	1647

A.7.7.3.2.2	Test Parameters	1647

A.7.7.3.2.3	Test Requirements	1649

A.7.7.4	L1-RSRP measurement for beam reporting	1649

A.7.7.4.1	SSB based L1-RSRP measurement	1649

A.7.7.4.1.1	Test Purpose and Environment	1649

A.7.7.4.1.2	Test parameters	1650

A.7.7.4.1.3	Test Requirements	1652

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1653

A.7.7.4.2.1	Test Purpose and Environment	1653

A.7.7.4.2.2	Test parameters	1653

A.7.7.4.2.3	Test Requirements	1655

A.7.7.5	CLI measurements	1656

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	1656

A.7.7.5.1.1	Test Purpose and Environment	1656

A.7.7.5.1.2	Test parameters	1656

A.7.7.5.1.3	Test Requirements	1659

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	1660

A.7.7.5.2.1	Test Purpose and Environment	1660

A.7.7.5.2.2	Test parameters	1660

A.7.7.5.2.3	Test Requirements	1662

A.8	E-UTRA standalone tests for NR RRM	1663

A.8.1	Void	1663

A.8.2	RRC_IDLE state mobility	1663

A.8.2.1	Inter-RAT NR Cell re-selection	1663

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	1663

A.8.2.1.1.1	Test Purpose and Environment	1663

A.8.2.1.1.2	Test Requirements	1668

A.8.3	RRC_CONNECTED state mobility	1669

A.8.3.1	Handover	1669

A.8.3.1.1	E-UTRAN - NR handover in FR1	1669

A.8.3.1.1.1	Test Purpose and Environment	1669

A.8.3.1.1.2	Test Requirements	1674

A.8.4	Measurement procedure	1674

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	1674

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	1674

A.8.4.1.1.1	Test Purpose and Environment	1674

A.8.4.1.1.2	Test Requirements	1676

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	1677

A.8.4.1.2.1	Test Purpose and Environment	1677

A.8.4.1.2.2	Test Requirements	1678

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	1678

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1678

A.8.4.2.1.1	Test Purpose and Environment	1678

A.8.4.2.1.2	Test Requirements	1683

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1683

A.8.4.2.2.1	Test Purpose and Environment	1683

A.8.4.2.2.2	Test Requirements	1687

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1687

A.8.4.2.3.1	Test Purpose and Environment	1687

A.8.4.2.3.2	Test Requirements	1692

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1692

A.8.4.2.4.1	Test Purpose and Environment	1692

A.8.4.2.4.2	Test Requirements	1696

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	1696

A.8.4.2.5.1	Test Purpose and Environment	1696

A.8.4.2.5.2	Test Requirements	1698

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	1699

A.8.4.2.6.1	Test Purpose and Environment	1699

A.8.4.2.6.2	Test Requirements	1701

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	1702

A.8.4.2.7.1	Test Purpose and Environment	1702

A.8.4.2.7.2	Test Requirements	1704

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	1705

A.8.4.2.8.1	Test Purpose and Environment	1705

A.8.4.2.8.2	Test Requirements	1707

A.8.5	Measurement performance	1708

A.8.5.1	SFTD accuracy	1708

A.8.5.1.1	SFTD accuracy	1708

A.8.5.1.1.1	Test Purpose	1708

A.8.5.1.1.2	Test Environment	1708

A.8.5.1.1.3	Test Requirements	1714

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	1714

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	1714

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	1719

A.8.5.2.1.2.1	Test Purpose and Environment	1719

A.8.5.2.1.2.2	Test Parameters	1719

A.8.5.2.1.2.3	Test Requirements	1721

A.8.5.2.2	SS-RSRQ	1721

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	1721

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	1726

A.8.5.2.2.2.1	Test Purpose and Environment	1726

A.8.5.2.2.2.2	Test Parameters	1726

A.8.5.2.2.2.3	Test Requirements	1728

A.8.5.2.3	SS-SINR	1728

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	1728

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	1732

A.8.5.2.3.2.1	Test Purpose and Environment	1732

A.8.5.2.3.2.2	Test Parameters	1732

A.8.5.2.3.2.3	Test Requirements	1734

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	1584

B.1	Conditions for NR RRC_IDLE state mobility	1584

B.1.1	Introduction	1584

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1584

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1585

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	1585

B.2.1	Introduction	1585

B.2.1.1	General	1585

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	1586

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	1586

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	1586

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	1587

B.2.1.4	Gain to SS-RSRP measurement point for FR1	1587

B.2.1.5	Gain to SS-RSRP measurement point for FR2	1587

B.2.1.5.1	Gain to SS-RSRP measurement point for Rx Beam Peak angle of arrival	1587

B.2.2	Conditions for NR intra-frequency measurements	1588

B.2.3	Conditions for NR inter-frequency measurements	1589

B.2.4	Conditions for NR L1-RSRP reporting	1590

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1590

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1591

B.2.5	Conditions for RRC connection release with redirection to NR	1592

B.2.6	Void	1593

B.2.6.1	Void	1593

B.2.6.2	Void	1594

B.3	RRM Requirements Exceptions	1595

B.3.1	Introduction	1595

B.3.2	Receiver sensitivity relaxation for CA	1595

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1595

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1595

B.3.2.2.1	Inter-band carrier aggregation	1595

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1595

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1596

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1596

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1596

B.3.2.4.1	Intra-band contiguous carrier aggregation	1596

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1596

B.3.3	Receiver sensitivity relaxation for DC	1596

B.3.3.1	Receiver sensitivity relaxation for EN-DC	1596

B.3.3.2	Receiver sensitivity relaxation for NE-DC	1596

B.3.4	Receiver sensitivity relaxation for SUL	1596

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1596

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1597

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1597

Annex C (informative):	Change history	1598