| 3GPP TR 38.786 V0.2.0 (2023-05) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR Sidelink evolution; User Equipment (UE) radio transmission and reception; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Background	9

5	Operating bands and channel arrangement for SL evolution	10

5.1	Operating bands	10

5.1.1	Operating bands for single carrier operation in unlicensed band	10

5.1.2	Operating band combinations for inter-band con-current operation	10

5.1.3	Operating band combinations for NR SL CA operation	10

5.2	Channel bandwidth	11

5.2.1	Channel bandwidth for single carrier operation in unlicensed band	11

5.2.2	Channel bandwidth for inter-band con-current operation	11

5.2.3	Channel bandwidth for NR SL CA operation	11

5.3	Channel arrangement	12

6	Transmitter characteristics for NR SL evolution	12

6.1	Tx requirements for NR SL single carrier operation in unlicensed band	12

6.1.1	Maximum output power for NR SL-U	12

6.1.2	UE maximum output power reduction for NR SL-U	12

6.1.2.1	MPR for SL-U operation in single CC	14

6.1.3	UE additional maximum output power reduction for NR SL-U	14

6.1.3.1	A-MPR for SL-U with NS_28	15

6.1.3.1.1	A-MPR for simultaneous PSSCH/PSCCH transmission	15

6.1.3.1.2	A-MPR for S-SSB transmission	15

6.1.3.1.3	A-MPR for PSFCH transmission	15

6.1.3.2	A-MPR for SL-U with NS_29	15

6.1.3.2.1	A-MPR for simultaneous PSSCH/PSCCH transmission	15

6.1.3.2.2	A-MPR for S-SSB transmission	15

6.1.3.2.3	A-MPR for PSFCH transmission	15

6.1.3.3	A-MPR for SL-U with NS_30	15

6.1.3.3.1	A-MPR for simultaneous PSSCH/PSCCH transmission	15

6.1.3.3.2	A-MPR for S-SSB transmission	15

6.1.3.3.3	A-MPR for PSFCH transmission	15

6.1.3.4	A-MPR for SL-U with NS_31	15

6.1.3.4.1	A-MPR for simultaneous PSSCH/PSCCH transmission	15

6.1.3.4.2	A-MPR for S-SSB transmission	15

6.1.3.4.3	A-MPR for PSFCH transmission	15

6.1.3.5	A-MPR for SL-U with NS_53	16

6.1.3.5.1	A-MPR for simultaneous PSSCH/PSCCH transmission	16

6.1.3.5.2	A-MPR for S-SSB transmission	16

6.1.3.5.3	A-MPR for PSFCH transmission	16

6.1.3.6	A-MPR for SL-U with NS_54	16

6.1.3.6.1	A-MPR for simultaneous PSSCH/PSCCH transmission	16

6.1.3.6.2	A-MPR for S-SSB transmission	16

6.1.3.6.3	A-MPR for PSFCH transmission	16

6.1.3.7	A-MPR for SL-U with NS_58	16

6.1.3.7.1	A-MPR for simultaneous PSSCH/PSCCH transmission	16

6.1.3.7.2	A-MPR for S-SSB transmission	16

6.1.3.7.3	A-MPR for PSFCH transmission	16

6.1.3.8	A-MPR for SL-U with NS_59	16

6.1.3.8.1	A-MPR for simultaneous PSSCH/PSCCH transmission	16

6.1.3.8.2	A-MPR for S-SSB transmission	16

6.1.3.8.3	A-MPR for PSFCH transmission	16

6.1.3.9	A-MPR for SL-U with NS_60	16

6.1.3.9.1	A-MPR for simultaneous PSSCH/PSCCH transmission	16

6.1.3.9.2	A-MPR for S-SSB transmission	16

6.1.3.9.3	A-MPR for PSFCH transmission	16

6.1.3.10	A-MPR for SL-U with NS_61	17

6.1.3.10.1	A-MPR for simultaneous PSSCH/PSCCH transmission	17

6.1.3.10.2	A-MPR for S-SSB transmission	17

6.1.3.10.3	A-MPR for PSFCH transmission	17

6.1.4	Configured transmitted power for NR SL-U	17

6.1.5	Minimum output power for NR SL-U	17

6.1.6	Transmit OFF power for NR SL-U	17

6.1.7	ON/OFF time mask for NR SL-U	17

6.1.8	Power control for NR SL-U	18

6.1.9	Transmit signal quality for NR SL-U	18

6.1.10	Spectrum emission mask for NR SL-U	19

6.1.11	ACLR requirements for NR SL-U	19

6.1.12	Spurious emissions for NR SL-U	20

6.1.13	Spurious emission band UE co-existence for NR SL-U	20

6.1.14	Transmit intermodulation for NR SL-U	20

6.2	Tx requirements for inter-band con-current operation	20

6.2.1	Maximum output power for inter-band con-current operation	20

6.2.2	UE maximum output power reduction for inter-band con-current operation	20

6.2.3	UE additional maximum output power reduction for inter-band con-current operation	20

6.2.4	Configured transmitted power for inter-band con-current operation	21

6.2.5	Minimum output power for inter-band con-current operation	21

6.2.6	Transmit OFF power for inter-band con-current operation	21

6.2.7	ON/OFF time mask for inter-band con-current operation	21

6.2.8	Power control for inter-band con-current operation	21

6.2.9	Transmit signal quality for inter-band con-current operation	21

6.2.10	Spectrum emission mask for inter-band con-current operation	21

6.2.11	ACLR requirements for inter-band con-current operation	22

6.2.12	Spurious emissions for inter-band con-current operation	22

6.2.13	Spurious emission band UE co-existence for inter-band con-current operation	22

6.2.14	Transmit intermodulation for inter-band con-current operation	22

6.3	Tx requirements for NR SL CA operation	22

6.3.1	Maximum output power for NR SL CA operation	22

6.3.2	UE maximum output power reduction for NR SL CA operation	23

6.3.3	UE additional maximum output power reduction for NR SL CA operation	23

6.3.4	Configured transmitted power for NR SL CA operation	24

6.3.5	Minimum output power for NR SL CA operation	24

6.3.6	Transmit OFF power for NR SL CA operation	24

6.3.7	ON/OFF time mask for NR SL CA operation	24

6.3.8	Power control for NR SL CA operation	24

6.3.9	Transmit signal quality for NR SL CA operation	24

6.3.10	Spectrum emission mask for NR SL CA operation	24

6.3.11	ACLR requirements for NR SL CA operation	24

6.3.12	Spurious emissions for NR SL CA operation	24

6.3.13	Spurious emission band UE co-existence for NR SL CA operation	24

6.3.14	Transmit intermodulation for NR SL CA operation	24

7	Receiver characteristics for NR SL evolution	24

7.1	Rx requirements for NR SL single carrier operation in unlicensed bands	24

7.1.1	Reference sensitivity power level for NR SL-U	24

7.1.2	Maximum input level for NR SL-U	26

7.1.3	Adjacent Channel Selectivity for NR SL-U	26

7.1.4	Blocking characteristics for NR SL-U	26

7.1.5	Spurious response for NR SL-U	28

7.1.6	Intermodulation characteristics for NR SL-U	28

7.2	Rx requirements for inter-band con-current operation	28

7.2.1	Reference sensitivity power level for inter-band con-current operation	28

7.2.2	Maximum input level for inter-band con-current operation	28

7.2.3	Adjacent Channel Selectivity for inter-band con-current operation	28

7.2.4	Blocking characteristics for inter-band con-current operation	28

7.2.5	Spurious response for inter-band con-current operation	28

7.2.6	Intermodulation characteristics for inter-band con-current operation	28

7.3	Rx requirements for NR SL CA operation	29

7.3.1	Reference sensitivity power level for NR SL CA operation	29

7.3.2	Maximum input level for NR SL CA operation	29

7.3.3	Adjacent Channel Selectivity for NR SL CA operation	29

7.3.4	Blocking characteristics for NR SL CA operation	29

7.3.5	Spurious response for NR SL CA operation	29

7.3.6	Intermodulation characteristics for NR SL CA operation	29

8	Co-channel coexistence between LTE Sidelink and NR Sidelink	29

Annex A: Change history	29

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for NR sidelink evolution services in Rel-18. The purpose is to specify radio solutions that are necessary for NR to support sidelink services working on unlicensed spectrums.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: "Guideline on WI/SI for new Operating Bands".

[3]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AGC	Automatic Gain Control

A-MPR	Additional Maximum Power Reduction

BLER	BLock Error Rate

BS	Base Station

CBW	Channel Bandwidth

CDF	Cumulative Distribution Function

CP-OFDM	Cyclic Prefix-OFDM

DMRS	Demodulation Reference Signal

EIRP	Equivalent Isotropically Radiated Power

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplexing

FR1	Frequency Range 1

ITS	Intelligent Transportation System

LTE	Long Term Evolution

MPR	Maximum Power Reduction

NF	Noise Figure

NR	New Radio

OLPC	Open Loop Power Control

PC	Power Control

PRB	Physical Resource Block

ProSe	Proximity-based Services

PSCCH	Physical Sidelink Control CHannel

PSSCH	Physical Sidelink Shared CHannel

REFSENS	Reference Sensitivity

RF	Radio Frequency

SCS	Sub-Carrier Spacing

SINR	Signal to Interference plus Noise Ratio

SL	Sidelink

SL-U	Sidelink at unlicensed band

SNR	Signal-to-Noise Ratio

TDD	Time Division Duplex

TDM	Time Division Multiplexing

UE	User Equipment

UL	Uplink

V2V	Vehicle to Vehicle

V2X	Vehicle to Everything

# 4	Background

In Rel-16, sidelink communication was developed in RAN mainly to support advanced V2X applications. In Rel-17, SA2 studied and standardized Proximity based service including public safety and commercial related service. As part of Rel-17, power saving solutions (e.g., partial sensing, DRX) and inter-UE coordination have been developed in RAN1 and RAN2 to improve power consumption for battery limited terminals and reliability of sidelink transmissions.

Although NR sidelink was initially developed for V2X applications, there is growing interest in the industry to expand the applicability of NR sidelink to commercial use cases. For commercial sidelink applications, two key requirements have been identified:

-	Increased sidelink data rate

-	Support of new carrier frequencies for sidelink

Increased sidelink data rate is motivated by applications such as sensor information (video) sharing between vehicles with high degree of driving automation. Commercial use cases could require data rates in excess of what is possible in Rel-17. Increased data rate can be achieved with the support of sidelink carrier aggregation and sidelink over unlicensed spectrum. Furthermore, by enhancing the FR2 sidelink operation, increased data rate can be more efficiently supported on FR2. While the support of new carrier frequencies and larger bandwidths would also allow to improve its data rate, the main benefit would come from making sidelink more applicable for a wider range of applications. More specifically, with the support of unlicensed spectrum and the enhancement in FR2, sidelink will be in a better position to be implemented in commercial devices since utilization of the ITS band is limited to ITS safety related applications.

Another aspect to consider is the V2X deployment scenario where both LTE V2X and NR V2X devices are to coexist in the same frequency channel. For the two different types of devices to coexist while using a common carrier frequency, it is important that there is mechanism to efficiently utilize resource allocation by the two technologies without negatively impacting the operation of each technology. This requirement was also mentioned as part of the input from 5G Automotive Association to the Rel-18 RAN Workshop.

# 5	Operating bands and channel arrangement for SL evolution

## 5.1	Operating bands

### 5.1.1	Operating bands for single carrier operation in unlicensed band

NR Sidelink is designed to operate in the unlicensed operating bands in FR1 defined in Table 5.1.1-1.

Table 5.1.1-1. NR SL-U operating bands in FR1

| NR SL-U operating band | Sidelink (SL) Transmission operating band | Sidelink (SL) Reception operating band | Duplex Mode | Interface |
| --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |  |
| n461 | 5150 MHz – 5925 MHz | 5150 MHz – 5925 MHz | HD | PC5 |
| n961,2 | 5925 MHz – 7125 MHz | 5925 MHz – 7125 MHz | HD | PC5 |
| n1021 | 5925 MHz – 6425 MHz | 5925 MHz – 6425 MHz | HD | PC5 |
| [NOTE 1: Client devices are prohibited from connecting directly to another client device in the frequency range 5850 – 5895 MHz and 5925 – 7125 MHz in USA, and in the frequency range 5925 – 7125 MHz in Canada per regulatory rules. Direct connection between client devices in the shared spectrum band is subject to region-specific regulatory rules. NOTE 2: For band n96 in USA, SL-U operation is excluded for vehicular UEs.] |  |  |  |  |

### 5.1.2	Operating band combinations for inter-band con-current operation

It has been agreed to use  n78@licensed band + n46@unlicensed band as the example band combination for Uu@Licensed and SL@Un-licensed.

The below inter-band con-current operation has been agreed not to be supported in Rel-18.

-	Uu@Un-licensed and SL@Un-licensed

-	Uu@Un-licensed and SL@licensed

-	Intra-band SL@Un-licensed

NR SL operation is designed to operate concurrently with NR uplink/downlink on the operating band combinations listed in Table 5.1.2-1.

Table 5.1.2-1 Band combinations of inter-band con-current operating bands

| NR SL inter-band con-current operating Band | NR Band | Interface |
| --- | --- | --- |
| SL_n78-n46 | n78 | Uu |
|  | n46 | PC5 |

### 5.1.3	Operating band combinations for NR SL CA operation

NR SL CA operation is designed to operate in the operating bands in FR1 defined in Table 5.1.3.

Table 5.1.3-1: Intra-band contiguous CA operating bands for SL CA in FR1

| NR SL CA Band | NR Band | Interface |
| --- | --- | --- |
| SL_n47 | n47 | PC5 |

## 5.2	Channel bandwidth

### 5.2.1	Channel bandwidth for single carrier operation in unlicensed band

Channel bandwidths defined for NR SL-U operating bands are in Table 5.2.1-1.

Table 5.2.1-1 NR SL-U channel bandwidth

|  |  | SL-U band /channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Operating Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n46 | 15 |  | 20 |  | 40 |  |  |  |  |  |
|  | 30 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
|  | 60 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
| n96 | 15 |  | 20 |  | 40 |  |  |  |  |  |
|  | 30 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
|  | 60 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
| n102 | 15 |  | 20 |  | 40 |  |  |  |  |  |
|  | 30 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
|  | 60 |  | 20 |  | 40 |  | 60 | 80 |  | 1001 |
| NOTE 1: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |

Support for 10MHz channel bandwidth is not defined for NR SL-U single carrier operation in bands n46, n96 and n102.

### 5.2.2	Channel bandwidth for inter-band con-current operation

It has been agreed to apply each channel bandwidth that is supported in a band for Uu and a band for SL-U.

For NR SL inter-band con-current operation in FR1, the NR channel bandwidths and SL channel bandwidths for each operating band are specified in Table 5.2.2-1.

Table 5.2.2-1: Inter-band con-current operation configurations

| NR SL inter-band con-current operating configuration | NR Band | Interface | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| SL_n78A-n46A | n78 | Uu | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  | n46 | PC5 | 20, 40, 60, 80, 100 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1 in TS38.101-1. |  |  |  |  |

### 5.2.3	Channel bandwidth for NR SL CA operation

For NR SL CA operation, the SL CA channel bandwidths for each operating band is specified in Table 5.2.3-1.

Table 5.2.3-1: Intra-band contiguous CA operating bands for SL CA in FR1

| Sidelink CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sidelink CA configuration | Sidelink CA configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| SL_n47B | SL_n47B | [10] | [20,30] |  |  | 70 | 0 |
|  |  | [20] | [20,30] |  |  |  |  |
|  |  | 30 | [30],40 |  |  |  |  |

## 5.3	Channel arrangement

### 5.3.1 Channel spacing

The general requirements in clause 5.4.1 of TS 38.101 are applied for NR SL-U.

### 5.3.2	Channel raster

The general requirements in clause 5.4.2 of TS 38.101 are applied for SL-U.

NR-ARFCN and channel raster requirements in 5.4.2.1 of TS 38.101 are applied for NR SL-U with following exception:

-	7.5kHz frequency raster shift, which can be used in NR V2X in band n47 is not defined for NR SL-U operation in bands n46, n96, n102.

-	Channel raster entries for each operating band requirements in clause 5.4.2.3 of TS 38.101 are applied for NR SL-U with following exception: Channel raster points for 10MHz CBW in band n46 as defined in Table 5.4.2.3-2 are not applicable for NR SL-U.

The mapping between the RF reference frequency on the channel raster and the corresponding resource element is given in Table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both Tx and Rx for SL. The mapping must apply to at least one numerology supported by the UE.

Table 5.4.2.2-1: Channel raster to resource element mapping

|  | NRBmod2 = 0 | NRBmod2 = 1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block number |  |  |

### 5.3.3	Synchronization raster

# 6	Transmitter characteristics for NR SL evolution

## 6.1	Tx requirements for NR SL single carrier operation in unlicensed band

### 6.1.1	Maximum output power for NR SL-U

### 6.1.2	UE maximum output power reduction for NR SL-U

The following assumption can serve as a starting point for MPR simulation assumptions that are considered to derive MPR requirements for SL-U operation.

-	Basic RF parameters for MPR of single carrier SL-U UE

-	Waveforms: CP-OFDM for SL-U

-	Supported CBW: 20/40/60/80/100MHz.

-	Modulation: QPSK/16-QAM/64-QAM/256-QAM

-	SCS: 15/30/60kHz

-	C-IMD: 45dBc or 60dBc

-	EVM and impairments

| Modulation | SystemEVM | PA only | Image | PA+image |
| --- | --- | --- | --- | --- |
| QPSK | 17.5% | 10% | 28dB | 10.0% |
| 16QAM | 12.5% | 8% | 28dB | 8.0% |
| 64QAM | 8% | 4% | 28dB | 5.65% |
| 256QAM | 3.5% | 1.8% | 34dB | 2.69% |

-	IBE and impairment exceptions are same as NR-U

-	Reuse IBE mask from TS38.10101 Table 6.4F.2.3-1

-	Image/carrier exceptions position and specific test according to interlaces.

-	ACLR: 27dBc for both PC3/PC5 UE with NR MBW

-	SEM according to 38.101-1 clause 6.5F.2.2

-	PA calibration & PA configuration:

-	PC3 (both single PA and dual PAs with 20+20): 1dB MPR and DFT-s-OFDM QPSK with 100RB0 20MHz waveform with 30dB ACLR

-	PC5 (single PA): 1dB MPR and DFT-s-OFDM QPSK 100RB3 20MHz waveform with 27dB ACLR

-	MPR table format can be reused the MPR format of NR-U according to each transmitted channel e.g.PSSCH/PSCCH transmission, PSFCH transmission and PSBCH transmission.

-	The MPR will be applied to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel.

-	Follow the RB interlaced allocation based on Table 6.2F.2-1 and Table 6.2F.2-2 in TS38.101-1. Also refer the interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. FFS on the inter-laced RB allocation for PSFCH, S-SSB by RAN1

For SL-U in single CC operation, the simultaneous transmission of PSCCH and PSSCH in the same sub-frame is supported and the following constraints in Table 6.1.2-1 can be assumed.

Table 6.1.2-1: SL-U operation’s MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes | Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,125,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216,220,225,228,230,240,250,252,255,260,264,270 The Allowed LCRB will be decided by the supported SCS. |
| Regarding PSCCH / PSSCH multiplexing |  |
| PSCCH size | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH | 0dB |

For simultaneous transmission of PSFCH transmission for SL-U operation, RAN4 assumed as follow

Table 6.1.2-2: SL-U UE’s MPR simulation assumptions for PSFCH transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSSCH | QPSK |
| PSFCH | ZC sequence |
| Structure of Slot | Baseline is follow RAN1 agreements |
| RB allocation | - 1 RB per user - All users have the same power per RB - Total power of all users equals 23 or 26dBm - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For multiple RB-sets, RAN4 only considers contiguous RB sets. - Both Non-contiguous PSFCH RB allocation and contiguous PSFCH allocation are allowed MPR will be derived by non-contiguous PSFCH RB allocation (N>1) - At least, the worst cases with possible RBstart and Ngap need to be checked. ( Ngap = RBend – RBstart ) For example: The worst case N gap is (106-1 =105*15kHz*12=) 18.9MHz for 20MHz, 15kHz SCS - IMD problem by dual PSFCH in SEM/SE region shall be considered to derive MPR level according to all supporting CBW and SCS. - N (Number of users) is up to 5 and RBs except for RBstart and RBend can be inserted between RBstart and RBend randomly. - Assumption of N in RAN4 is only for MPR simulation purpose, the final number is up to RAN1 decision. |

For S-SSB transmission for SL-U operation, RAN4 assumed as follow

Table 6.1.2-3e: SL-U UE’s MPR simulation assumptions for S-SSB transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSBCH | QPSK |
| S-PSS | M-sequence |
| S-SSS | Golden-sequence |
| S-SSB structure |  |
| RB allocation | - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB set, the 11RB will be repeated N time in a RB set. For the multiple RB-sets, UE may transmit S-SSB repetition in the contiguous multiple RB sets. RBstart: All the possible cases LCRB: 11 RB |

#### 6.1.2.1	MPR for SL-U operation in single CC

<Editor Note> Detail MPR requirements will be added

### 6.1.3	UE additional maximum output power reduction for NR SL-U

Based on the NR-U agreement for the additional spectrum emission mask in clause 6.5F.2.3 in TS38.101-1, RAN4 do not consider the additional spectrum emission mask.

For the additional spurious emission requirements for SL-U, RAN4 will consider the NR-U additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1.

To derive A-MPR requirements for SL-U operation in single CC, RAN4 will assume the basic simulation parameters and assumptions in section 6.1.2 and the additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1 will be considered.

#### 6.1.3.1	A-MPR for SL-U with NS_28

##### 6.1.3.1.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.1.2	A-MPR for S-SSB transmission

##### 6.1.3.1.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.2	A-MPR for SL-U with NS_29

##### 6.1.3.2.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.2.2	A-MPR for S-SSB transmission

##### 6.1.3.2.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.3	A-MPR for SL-U with NS_30

##### 6.1.3.3.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.3.2	A-MPR for S-SSB transmission

##### 6.1.3.3.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.4	A-MPR for SL-U with NS_31

##### 6.1.3.4.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.4.2	A-MPR for S-SSB transmission

##### 6.1.3.4.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.5	A-MPR for SL-U with NS_53

##### 6.1.3.5.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.5.2	A-MPR for S-SSB transmission

##### 6.1.3.5.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.6	A-MPR for SL-U with NS_54

##### 6.1.3.6.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.6.2	A-MPR for S-SSB transmission

##### 6.1.3.6.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.7	A-MPR for SL-U with NS_58

##### 6.1.3.7.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.7.2	A-MPR for S-SSB transmission

##### 6.1.3.7.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.8	A-MPR for SL-U with NS_59

##### 6.1.3.8.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.8.2	A-MPR for S-SSB transmission

##### 6.1.3.8.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.9	A-MPR for SL-U with NS_60

##### 6.1.3.9.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.9.2	A-MPR for S-SSB transmission

##### 6.1.3.9.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.10	A-MPR for SL-U with NS_61

##### 6.1.3.10.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.10.2	A-MPR for S-SSB transmission

##### 6.1.3.10.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

### 6.1.4	Configured transmitted power for NR SL-U

The configured transmitted power for NR-U is defined in clause 6.2F.4 of TS 38.101-1 and the normal NR requirement in clause 6.2.4 of TS 38.101-1 apply. For NR V2X, PCMAX is defined for PSSCH\PSCCH, S-SSB and PSFCH in clause 6.2E.4 specifically.

It has been agreed to reuse the NR V2X Configured transmitted power requirements in 38.101-1 clause 6.2E.4 for NR SL-U.

### 6.1.5	Minimum output power for NR SL-U

The Minimum output power of NR-U is defined in clause 6.3F.1 of TS 38.101-1 and  the normal NR requirement apply, which is the same for NR V2X.

It has been agreed to reuse the NR single CC minimum requirements in 38.101-1 clause 6.3.1 for NR SL-U.

### 6.1.6	Transmit OFF power for NR SL-U

The Transmit OFF power of NR-U is defined in clause 6.3F.2 of TS 38.101-1 and  the normal NR requirement apply, which is the same for NR V2X.

It has been agreed to reuse the NR single CC minimum requirements in 38.101-1 clause 6.3.2 for NR SL-U.

### 6.1.7	ON/OFF time mask for NR SL-U

Definition of ON/OFF time mask can be divided into two parts. First part defines the transition from OFF to ON and the second part transition from ON to OFF. For the ON to OFF part, it was agreed that 10us transient period is located inside the guard period. For the OFF to ON part at the beginning of the transmission, whether to put the whole 15 us transient period inside the CP-E or to reuse the NR-U mask that only put 10us inside the CP-E was discussed and following this discussion, and information from RAN1 that 16us or 25us LBT time preceding the CP-E already includes 5us ramp-up time, it was agreed to reuse the NR-U approach where first 5us of the OFF tot ON transient period is located before the CP-E.

ON/OFF masks for NR SL-U PSSCH and PSCCH and S-SSB that are in line with the above agreements are shown in figures below.

Guard

Period

N+1 Slot (incl. transmission gap)

CP

-

E

N

+2

Slot

5us

10us

10us

Transient Period

Transient Period

End of OFF

Power Requirement

N

0

Slot

Start of N

+1

power

End of N

+1

power

Start of OFF power

requirement

Figure 6.1.7-1 General ON/OFF time mask for SL-U PSSCH and PSCCH

Figure 6.1.7-2 ON/OFF time mask for SL-U S-SSB

### 6.1.8	Power control for NR SL-U

The power control of NR-U is defined in clause 6.3F.4 of TS 38.101-1 and  the power control of NR V2X is defined in clause 6.3E.4. Since only absolute power control requirement is defined for NR V2X, it has been agreed that only absolute power control requirement is defined for NR SL-U.

During the NR-U discussion it was agreed that absolute requirement applies at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 40 ms. Gap duration of 40ms is larger than 20ms, which is defined in generic requirements in Clause 6.3.4.2. of TS 38.101-1. This extension of gap duration is in line with eLAA requirement in 36.101 and is defined to maintain power control accuracy over longer periods of no transmission due to the uncertainty introduced by LBT. Therefore NR SL-U can reuse NR-U absolute power control requirement as defined in Clause 6.3F.4.2.

### 6.1.9	Transmit signal quality for NR SL-U

6.1.9.1	Frequency Error

The Frequency error for  NR-U is defined in clause 6.4F.1 of TS 38.101-1 and the normal NR requirement in clause 6.4.1 of TS 38.101-1 apply. For NR V2X, frequency error is defined in clause 6.4E,1 as ±0.1 PPM accuracy observed over a period of 1 ms against GNSS.”

It has been agreed to reuse the NR V2X Frequency error requirements in 38.101-1 clause 6.4E.1 for NR SL-U.

6.1.9.2	EVM

The EVM of NR-U is defined in clause 6.4F.2 of TS 38.101-1 and the normal NR requirement in clause 6.4.2.1 of TS 38.101-1 apply. For NR V2X, EVM is defined in clause 6.4E.2.2  with other than the Pi/2-BPSK modulation, which is not supported for NR V2X.

It has been agreed to reuse the NR V2X EVM requirements in 38.101-1 clause 6.4E.2.2 for NR SL-U.

-	For NR V2X the EVM requirements in clause 6.4.2.1 are valid for PSCCH, PSSCH and PSBCH physical channels with other than pi/2-BPSK modulation, which is not supported for NR V2X, and excluding the guard symbol at the end of transmitted slots

6.1.9.3 	Carrier Leakage

The Carrier Leakage of NR-U is defined in clause 6.4.2.2 of TS 38.101-1 and  the normal NR requirement apply, which is the same for NR V2X.

It has been agreed to reuse the NR single CC carrier leakage requirements in 38.101-1 clause 6.4.2.2 for NR SL-U.

6.1.9.4	Equalizer spectrum flatness

The Equalizer spectrum flatness of NR-U is referred to clause 6.4.2.4 of TS 38.101-1 and the normal NR requirement apply, which is the same for NR V2X.

It has been agreed to reuse the NR single CC Equalizer spectrum flatness requirements in 38.101-1 clause 6.4.2.4 for NR SL-U.

6.1.9.5	In-band Emission

The In-band Emission of NR-U is defined in clause 6.4F.2.3 of TS 38.101-1. For NR V2X, In-band Emission is defined in clause 6.4E,2.4  where normal NR requirement of clause 6.4.2.3 apply.

It has been agreed to reuse the NR-U In-band emission requirements in 38.101-1 Table 6.4F.2.3-1 for NR SL-U. Minimum requirements for in-band emissions for NR SL-U PSCCH, PSSCH and PSBCH transmissions over the measurement interval that takes into account guard period at the end of the transmit slot.

### 6.1.10	Spectrum emission mask for NR SL-U

6.1.10.1	Occupied Bandwidth

The Occupied Bandwidth of NR-U is defined in clause 6.5.1 of TS 38.101-1 and the normal NR requirement apply, which is the same for NR V2X.

It has been agreed to reuse the NR single CC Occupied Bandwidth requirements in 38.101-1 clause 6.5.1 for NR SL-U.

6.1.10.2	SEM

The SEM of NR-U is defined in clause 6.5F.2.2 of TS 38.101-1. For NR V2X, In-band Emission is defined in clause 6.5E,2.2  where normal NR requirement of clause 6.5.2.2 apply.

It has been agreed to reuse the NR-U SEM requirements in 38.101-1 Table 6.4F.2.3-1 for NR SL-U.

6.1.10.3	A-SEM

Additional spectrum emission mask requirements are not defined for NR-U.

It is agreed not to define A-SEM requirements for NR SL-U when operating in bands n46, n96 and n102.

### 6.1.11	ACLR requirements for NR SL-U

The ACLR of NR-U is defined in clause 6.5F.2.4 of TS 38.101-1. For NR V2X, ACLR is defined in clause 6.5E.2.4  where normal NR requirement of clause 6.5.2.4 apply. The ACLR requirement was optimized for operation in un-licensed band during the development of the NR-U requirements.

It has been agreed to reuse the NR-U ACLR requirements in 38.101-1 clause 6.5F.2.4 for NR SL-U.

### 6.1.12	Spurious emissions for NR SL-U

The general spurious emission of NR-U is defined in clause 6.5.3.1 of TS 38.101-1 and the normal NR requirement apply, which is the same for NR V2X.

It has been agreed to reuse the NR single CC general spurious emission requirements in 38.101-1 clause 6.5.3.1 for NR SL-U.

The additional spurious emission of NR-U is defined in clause 6.5F.3.3 of TS 38.101-1 with different NS values for different bands.

It has been agreed to reuse the NR-U single CC minimum requirements in TS 38.101-1 clause 6.5F.3.3 for NR SL-U.

### 6.1.13	Spurious emission band UE co-existence for NR SL-U

UE coexistence requirements are not defined for NR-U.

It has been agreed not to define UE coexistence spurious emission requirements for NR SL-U when operating in bands n46, n96 and n102 in case of non-concurrent operation.

### 6.1.14	Transmit intermodulation for NR SL-U

The Transmit intermodulation of NR-U is defined in clause 6.5.4 of TS 38.101-1 and the normal NR requirement apply, which is the same for NR V2X.

It has been agreed to reuse the NR single CC Transmit intermodulation requirements in 38.101-1 clause 6.5.4 for NR SL-U.

## 6.2	Tx requirements for inter-band con-current operation

It has been agreed to consider the functionality and requirements of single carrier SL-U operation as a basis of SL-U  and the functionality and requirements of single carrier Uu operation as a basis of Uu in con-current operation.

### 6.2.1	Maximum output power for inter-band con-current operation

It has been agreed to target the following power classes.

-	Power class 3 Uu@Licensed + Power class 5 SL@Un-licensed

-	Power class 3 Uu@Licensed + Power class 3 SL@Un-licensed can be considered as 2nd priority if Power class 3 SL is agreed in a single carrier at unlicensed band

For Power class 2 for Uu@Licensed for inter-band concurrent operation, it is not considered in Rel-18.

### 6.2.2	UE maximum output power reduction for inter-band con-current operation

It has been agreed to refer to the existing requirement for NR Uu and refer to the SL-U requirement if specified.

For the inter-band con-current operation, the allowed maximum power reduction (MPR) for the maximum output power shall be applied per each component carrier. The MPR requirements in TS 38.101-1 clause 6.2.2 apply for NR Uu operation in licensed band, and the MPR requirements in clause X apply for NR sidelink operation in unlicensed band.

### 6.2.3	UE additional maximum output power reduction for inter-band con-current operation

It has been agreed to refer to the existing requirement for NR Uu and refer to the SL-U requirement if specified.

For the inter-band con-current operation, the allowed additional maximum power reduction (A-MPR) for the maximum output power shall be applied per each component carrier. The A-MPR requirements in clause 6.2.3 apply for NR Uu operation in licensed band, and the A-MPR requirements in clause X apply for NR sidelink operation in unlicensed band.

### 6.2.4	Configured transmitted power for inter-band con-current operation

It has been agreed to

-	consider the existing requirement for NR V2X con-current operation (6.2E.4.2 in TS38.101-1)  as starting point

-	reuse the existing delta Tib of CA_n46-n78 for inter-band concurrent operation in Rel-18

### 6.2.5	Minimum output power for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

For the inter-band con-current operation, the requirements specified in TS 38.101-1 clause 6.3.1 shall apply for NR Uu operation in licensed band and the requirements specified in clause X shall apply for NR sidelink operation in unlicensed band.

### 6.2.6	Transmit OFF power for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

For the inter-band con-current operation, the requirements specified in TS 38.101-1 clause 6.3.2 shall apply for NR Uu operation in licensed band and the requirements specified in clause X shall apply for NR sidelink operation in unlicensed band

### 6.2.7	ON/OFF time mask for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

### 6.2.8	Power control for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

For the inter-band con-current operation, the requirements specified in TS 38.101-1 clause 6.3.4 shall apply for NR Uu operation in licensed band and the requirements specified in clause X shall apply for NR sidelink operation in unlicensed band

### 6.2.9	Transmit signal quality for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

### 6.2.10	Spectrum emission mask for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

For the inter-band con-current operation, the general/additional SEM requirements specified in TS 38.101-1 clause 6.5.2 shall apply for NR Uu operation in licensed band and the general/additional SEM requirements specified in clause X shall apply for NR sidelink operation in unlicensed band.

### 6.2.11	ACLR requirements for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

For the inter-band con-current operation, the ACLR requirement specified in clause TS 38.101-1 6.5.2.4 shall apply for NR Uu operation in licensed band and the ACLR requirement specified in clause X shall apply for NR sidelink operation in unlicensed band.

### 6.2.12	Spurious emissions for inter-band con-current operation

The general spurious emission is not needed for inter-band con-current operation as the NR SL only defines spurious emission for non-current case.

### 6.2.13	Spurious emission band UE co-existence for inter-band con-current operation

It has been agreed to reuse the inter band CA_n46-n78 co-existence requirement for con-current operation with Uu@78 + SL@n46.

### 6.2.14	Transmit intermodulation for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

For the inter-band con-current operation, the requirements specified in TS 38.101-1 clause 6.5.4 shall apply for NR Uu operation in licensed band and the requirements specified in clause X shall apply for NR sidelink operation in unlicensed band.

## 6.3	Tx requirements for NR SL CA operation

### 6.3.1	Maximum output power for NR SL CA operation

For the intra-band SL CA operation, the following NR SL CA UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub frame (1ms).

Table 6.3.1-1: NR SL CA UE Power Class

| NR SL CA band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n47B |  |  |  |  | 23 | +2/-33 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: For intra-band SL CA UE, the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 3: 3 refers to the transmission bandwidths (Figure 5.6-1 in TS38.101-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

### 6.3.2	UE maximum output power reduction for NR SL CA operation

For basic parameters, reuse the simulation assumptions in TR38.785 (Rel-17 enhanced NR sidelink). Other constraints for PSCCH/PSSCH/PSFCH/S-SSB can be assumed based on current RAN1’s agreement.

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 23dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 25dBc |
| IQ image | 25dBc |
| CIM3 | FFS: 45dBc or, 45dBc and 60dBc |
| PA calibration | PA calibrated to deliver [31dBc] ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

6.3.2.1 MPR for NR SL CA operation

<Editor Note> Detail MPR requirements will be added.

### 6.3.3	UE additional maximum output power reduction for NR SL CA operation

For the additional emission limits in Europe in ITS spectrum, RAN4 already defined the regulatory requirements in TS38.101-1. The ETSI regulation shall be considered to derive the A-MPR requirements for NR SL CA UE. For the US related regulatory requirements will be studied after final FCC announcement for the additional emission limits in US.

To derive A-MPR requirements for SL CA operation in ITS spectrum, RAN4 will assume the basic simulation parameters and assumptions in section 6.3.2 and the additional spectrum emission mask in Table 6.5E.2.3.1-1 and additional spurious emission requirements in Table 6.5E.3.4.2-1 and Table 6.5E.3.4.2-2  in TS38.101-1 will be considered when NS_33 is configured from the pre-configured radio parameters in the geometrical region or indicated in the cell.

6.3.3.1 A-MPR for NR SL CA operation

<Editor Note> Detail A-MPR requirements will be added.

### 6.3.4	Configured transmitted power for NR SL CA operation

### 6.3.5	Minimum output power for NR SL CA operation

### 6.3.6	Transmit OFF power for NR SL CA operation

### 6.3.7	ON/OFF time mask for NR SL CA operation

### 6.3.8	Power control for NR SL CA operation

### 6.3.9	Transmit signal quality for NR SL CA operation

### 6.3.10	Spectrum emission mask for NR SL CA operation

### 6.3.11	ACLR requirements for NR SL CA operation

### 6.3.12	Spurious emissions for NR SL CA operation

### 6.3.13	Spurious emission band UE co-existence for NR SL CA operation

### 6.3.14	Transmit intermodulation for NR SL CA operation

# 7	Receiver characteristics for NR SL evolution

## 7.1	Rx requirements for NR SL single carrier operation in unlicensed bands

### 7.1.1	Reference sensitivity power level for NR SL-U

Background information for NR V2X reference sensitivity requirement has been captured in the TR 38.886 as captured below and can be reused for NR SL-U

The reference sensitivity power level REFSENS is the minimum mean power applied to the UE antenna connector at which the throughput shall meet or exceed 95% of the maximum throughput of the reference measurement channels.

The V2X UE REFSENS is defined by the following equation:

REFSENSV2X=kTB + SNRV2X +10log10(LCRB*SCS*12/RX_BW) +( NFV2X+ IM) – Diversity gain

Where

-	kTB: Thermal noise level is [-174dBm(kT) + 10*log10(RX BW)]dBm.

-	NF: Noise figure. 13 dB is used for LAA and can be reused for NR V2X requirements. Assumed NF is 9dB < 3GHz, NF is 10dB>= 3GHz (e.g B42, n77, n78, n79…) at licensed bands at FR1.

-	IM: 2.5 dB is assumed. When the number of RB size equal to or less than 24RBs, 0.5dB additional relaxation is allowed.

-	Target SNR: -0.5 dB

-	Diversity gain: 3dB

For NR-U, the following equations in table below are used to calculate the REFSENS requirement and it can be further re-used in NR SL-U in bands n46, n96 and n102, while taking account that for NR-U target SNR of -1dB was used and therefore the REFSENS values in table below are 0.5dB higher than with NR SL-U target SNR of 0.5dB.

Table 7.1.1-1 REFSENS for NR-U

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | -89.7 + 10log10(NRB/106) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -89.9 + 10log10(NRB/51) |  |
|  | 60 | 20, 40, 60, 80, 100 | -90.1 + 10log10(NRB/24) |  |
| n96, n102 | 15 | 20, 40 | -89.2 + 10log10(NRB/106) | HD |
|  | 30 | 20, 40, 60, 80, 100 | -89.4 + 10log10(NRB/51) |  |
|  | 60 | 20, 40, 60, 80, 100 | -89.6 + 10log10(NRB/24) |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the RB configuration for SL-U REFSENS as shown in table 7.1.1-1. |  |  |  |  |

For the NR-U REFESENS requirement, the RB configuration for larger than 20MHz is considered as wideband operation and hence all intra-cell guard bands between sub-bands should be filled. In this case, the full RB allocation of NR is used to define the reference sensitivity requirement. However, for V2X, specific RB allocation using the sub-channels based on the RAN2 specification TS 38.331 is used as captured below:

Following the TS 38.331 definitions Table 7.1.1-2 shows the RB configuration for NR SL-U reference sensitivity requirement.

Table 7.1.1-2 NR SL-U RB configurations for REFSENS

|  |  | NR band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR SL-U Band | SCS kHz | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Duplex Mode |
| n46 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | [135] |  |
| n96 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | [135] |  |
| n102 | 15 | 105 | 216 |  |  |  | HD |
|  | 30 | 50 | 105 | 160 | 216 | 270 |  |
|  | 60 | 24 | 50 | 75 | 105 | [135] |  |

With the agreed SNR and RB configuration as above, the REFNSENS requirement for NR SL-U is  shown in Table 7.1.1-3.

Table 7.1.1-3 REFSENS for SL-U

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n46 | 15 | 20, 40 | [-89.2 + 10log10(NRB/105)] | HD |
|  | 30 | 20, 40, 60, 80, 100 | [-89.4 + 10log10(NRB/50)] |  |
|  | 60 | 20, 40, 60, 80, 100 | [-89.6 + 10log10(NRB/24)] |  |
| n96, n102 | 15 | 20, 40 | [-88.7 + 10log10(NRB/105)] | HD |
|  | 30 | 20, 40, 60, 80, 100 | [-88.9 + 10log10(NRB/50)] |  |
|  | 60 | 20, 40, 60, 80, 100 | [-89.1 + 10log10(NRB/24)] |  |
| NOTE 1: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the RB configuration for NR SL-U REFSENS as shown in table 7.1.1-2 |  |  |  |  |

### 7.1.2	Maximum input level for NR SL-U

The Maximum input level for NR-U is defined in clause 7.4 of TS 38.101-1. For NR V2X, Maximum input level is defined in clause 7.4E.1. The NRV2X Sidelink requirement only covers up to 40MHz and the minimum requirement is the same as NR-U.

It has been agreed to reuse the NRU Maximum input level requirements in TS 38.101-1 clause 7.4 for NR SL-U.

### 7.1.3	Adjacent Channel Selectivity for NR SL-U

For the ACS requirement, the ACS for NR-U was defined for bands above 3.3GHz considering the unlicensed bands of n46, n96, n102 are all high bands. However, the requirement format is different from NR single CC which can be found below, i.e. the ACS is adjusted according to the CBW while the NR single CC is fixed as 33dB for bands above 3.3GHz. The ACS requirement has been extensively discussed in NR-U WI with various views considering the difference of ACLR, ACIR impacts, and comparison with WIFI system, and coexistence with LAA, and the final value is a compromised outcome.

ACS Requirement for NR-U

Furthermore, for NR normal requirement, there are two test cases for ACS as case 1 and case 2. During the NR-U discussion, the case 2 was considered not necessary for operation in unlicensed bands. It has been agreed to reuse NR-U ACS requirement for NR SL-U using the NR SL-U RMC, as discussed in general part.

### 7.1.4	Blocking characteristics for NR SL-U

7.1.4.1	In Band Blocking

The in band blocking requirement for NR V2X is captured in clause 7.6E.2.1 and for NR-U it is captured in clause 7.F.2.1. The transmission power is the same for 20 and 40MHz and the difference is the bandwidth of interferer. The 10MHz as smallest CBW for NR V2X is used as BWinterferer and corresponding offset frequency is decided. In NR SL-U, 10MHz is not supported and 20MHz is supported as the smallest CBW for single CC.

In this case, the NR-U IBB can be reused for NR SL-U.

7.1.4.2	Out of Band Blocking

The NR sidelink OBB requirement is reused from LTE sidelink and hence different power level for the transmission power based on different bandwidth. For NR-U OBB requirement, it is based on the NR normal requirement since there is no Wi-Fi corresponding requirement. Further modification has applied for range 3 considering the shallower frequency response of the band filter at high frequencies. In this case, the NR-U requirement can be reused for NR SL-U.

7.1.4.3	Narrow Band Blocking

Narrow band blocking was introduced in NR single CC to mimic the potential interference coming from narrow band systems, but such system is not existing in the targeting unlicensed spectrum and therefore the Narrow Band Blocking requirements are not defined for NR-U.

It has been agreed not to define Narrow Band Blocking requirements for NR SL-U.

### 7.1.5	Spurious response for NR SL-U

The NR sidelink spurious response requirement is reused from LTE sidelink and hence different power level for the transmission power based on different bandwidth is used.For NR SL-U operation in unlicensed bands, the  NR-U requirement can be reused.

### 7.1.6	Intermodulation characteristics for NR SL-U

The NR sidelink Intermodulation requirement is reused from LTE sidelink and hence different power level for the transmission power based on different bandwidth is used.For NR SL-U operation in unlicensed bands,  the  NR-U requirement can be reused.

## 7.2	Rx requirements for inter-band con-current operation

### 7.2.1	Reference sensitivity power level for inter-band con-current operation

It has been agreed to

-	reuse the existing CA_n46-n78 MSD requirements for MSD of the inter-band concurrent operation, if power class 5 was assumed for the aggressor band n46 in the existing MSD requirement

-	reuse the existing delta Rib of CA_n46-n78 for inter-band concurrent operation in Rel-18

### 7.2.2	Maximum input level for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

### 7.2.3	Adjacent Channel Selectivity for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

### 7.2.4	Blocking characteristics for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

### 7.2.5	Spurious response for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

### 7.2.6	Intermodulation characteristics for inter-band con-current operation

It has been agreed to reuse NR uu and SL-U requirements for each band if an issue is not identified.

## 7.3	Rx requirements for NR SL CA operation

### 7.3.1	Reference sensitivity power level for NR SL CA operation

### 7.3.2	Maximum input level for NR SL CA operation

### 7.3.3	Adjacent Channel Selectivity for NR SL CA operation

### 7.3.4	Blocking characteristics for NR SL CA operation

### 7.3.5	Spurious response for NR SL CA operation

### 7.3.6	Intermodulation characteristics for NR SL CA operation

# 8	Co-channel coexistence between LTE Sidelink and NR Sidelink

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-02 | RAN4#106 |  |  |  |  | Initial Skeleton | 0.0.1 |
| 2023-04 | RAN4#106bis-e | R4-2306634 |  |  |  | TP to TR on the SL-e TX requirement | 0.1.0 |
| 2023-04 | RAN4#106bis-e | R4-2306635 |  |  |  | TP to TR on on the SL-e RX requirement | 0.1.0 |
| 2023-05 | RAN4#107 | R4-2310378 |  |  |  | TP for TR 38.786 on the MPR/A-MPR simulation assumptions | 0.2.0 |
| 2023-05 | RAN4#107 | R4-2310307 |  |  |  | TP on system parameters for SL unlicensed operation for single CC | 0.2.0 |
| 2023-05 | RAN4#107 | R4-2310308 |  |  |  | on the SL-e TX requirement | 0.2.0 |
| 2023-05 | RAN4#107 | R4-2310309 |  |  |  | on the SL-e RX requirement | 0.2.0 |
| 2023-05 | RAN4#107 | R4-2310311 |  |  |  | TP to TR on con-current operation on Uu and sidelink | 0.2.0 |
| 2023-05 | RAN4#107 | R4-2310312 |  |  |  | TP for TR 38.786 on the updated TR structure for NR SL CA operation | 0.2.0 |