3GPP TR 38.828 V16.0.0 (2019-06)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Cross Link Interference (CLI) handling and

Remote Interference Management (RIM) for NR;

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	8

2	References	8

3	Definitions, symbols and abbreviations	9

3.1	Definitions	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Background	9

4.1	General	9

4.2	WID description	10

4.2.1	Justification	10

4.2.2	Objective	11

4.3	Dynamic TDD adjacent interference scenarios	12

4.3.1	General	12

4.3.2	Interference scenarios that occur for both synchronized and unsynchronized TDD (including CLI)	12

4.3.3	Additional interference scenarios that occur for unsynchronized TDD (including dynamic TDD and CLI)	13

4.4	BS-BS interference for zero grid shift scenarios	13

5	Co-existence analysis	14

5.1	Scenarios	14

5.1.1	FR1	14

5.1.2	FR2	15

5.2	Simulation assumptions	16

5.2.1	FR1	16

5.2.1.1	Network layout model	16

5.2.1.1.1	Urban macro	16

5.2.1.1.2	Indoor	17

5.2.1.2	ACIR	18

5.2.1.3	UE distribution	18

5.2.1.4	Other simulation parameters	18

5.2.1.5	Antenna configuration	19

5.2.1.5.1	Urban macro scenario	19

5.2.1.5.2	Indoor scenario	19

5.2.1.5.3	UE antenna element pattern	19

5.2.2	FR2	20

5.2.2.1	Network layout model	20

5.2.2.1.1	Urban macro	20

5.2.2.1.2	Dense urban	20

5.2.2.1.3	Indoor	21

5.2.2.2	ACLR and ACS	21

5.2.2.3	UE distribution	22

5.2.2.3.1	Urban Macro (Macro-to-Macro)	22

5.2.2.3.2	Dense Urban (Micro-to-Micro)	22

5.2.2.3.3	Indoor-to-Indoor and Indoor-to-Macro	22

5.2.2.4	Other simulation parameters	22

5.2.2.5	Antenna configuration	23

5.2.2.5.1	Urban macro scenario	23

5.2.2.5.2	Dense urban scenario	24

5.2.2.5.3	Indoor scenario	24

5.2.2.5.4	UE antenna element pattern	25

5.2.3	Common assumptions	25

5.2.3.1	Propagation model	25

5.2.3.2	LOS model	26

5.2.3.3	O-to-I penetration loss	27

5.2.3.4	Transmission power control model	28

5.2.3.5	Received signal power model	28

5.2.3.6	Evaluation metric	29

5.2.3.7	Antenna modelling	29

5.2.3.8	Simulation description	29

5.3	Simulation results	30

5.3.1	General	30

5.3.2	Simulation limitations	30

5.3.3	FR1	31

5.3.3.1	Scenario 1: 4 GHz Macro→ Macro (DL)	31

5.3.3.1.1	Results	31

5.3.3.2	Scenario 2: 4 GHz Macro→ Macro (UL)	32

5.3.3.2.1	Results	32

5.3.3.3	Scenario 3: 4 GHz Macro → Indoor (DL)	32

5.3.3.3.1	Results	32

5.3.3.4	Scenario 4: 4 GHz Macro → Indoor (UL)	33

5.3.3.4.1	Results	33

5.3.3.5	Scenario 5: 4 GHz Indoor → Macro (DL)	33

5.3.3.5.1	Results	33

5.3.3.6	Scenario 6: 4 GHz Indoor → Macro (UL)	34

5.3.3.6.1	Results	34

5.3.3.7	Scenario 7: 4 GHz Indoor → Indoor (DL)	34

5.3.3.7.1	Results	34

5.3.3.8	Scenario 8: 4 GHz Indoor → Indoor (UL)	35

5.3.3.8.1	Results	35

5.3.4	FR2	36

5.3.4.1	Scenario 9: 30 GHz Macro → Macro (DL)	36

5.3.4.1.1	Results	36

5.3.4.2	Scenario 10: 30 GHz Macro → Macro (UL)	36

5.3.4.2.1	Results	36

5.3.4.3	Scenario 11: 30 GHz Micro → Micro (DL)	37

5.3.4.3.1	Results	37

5.3.4.4	Scenario 12: 30 GHz Micro → Micro (UL)	37

5.3.4.4.1	Results	37

5.3.4.5	Scenario 13: 30 GHz Indoor → Macro (DL)	37

5.3.4.5.1	Results	37

5.3.4.6	Scenario 14: 30 GHz Indoor → Macro (UL)	38

5.3.4.6.1	Results	38

5.3.4.7	Scenario 15: 30 GHz Indoor → Indoor (DL)	38

5.3.4.7.1	Results	38

5.3.4.8 Scenario 16: 30 GHz Indoor → Indoor (UL)	39

5.3.4.8.1	Results	39

6	Summary and recommendations	39

6.1	Zero grid shift	39

6.2	Summary of results for 100% grid shift	39

6.2.1	FR1	39

6.2.1.1	Macro-to-Macro scenario	39

6.2.1.2	Indoor-to-Macro scenario	40

6.2.1.3	Indoor-to-Indoor scenario	40

6.2.1.4	Macro-to-Indoor scenario	40

6.2.2	FR2	40

6.2.2.1	Macro-to-Macro scenario	40

6.2.2.2	Indoor-to-Macro scenario	40

6.2.2.3	Indoor-to-Indoor scenario	40

6.2.2.4	Micro-to-Micro scenario	40

6.3	Recommendations	41

6.3.1	FR1	41

6.3.1.1	Macro-to-Macro scenario	41

6.3.1.2	Indoor scenarios (Indoor-to-Macro and Indoor-to-Indoor)	41

6.3.2	FR2	41

6.3.2.1	Macro-to-Macro scenario	41

6.3.2.2	Indoor scenarios (Indoor-to-Macro and Indoor-to-Indoor)	41

6.3.2.3	Micro-to-Micro scenario	41

Annex A:	 Detailed simulation results for non-zero grid shift	42

A.1	FR1	42

A.1.1	Scenario 1: 4GHz Macro → Macro (DL)	42

A.1.1.1	Ericsson	42

A.1.1.1.1	100% utilization	42

A.1.1.1.2	50% utilization	44

A.1.1.1.3	10% utilization	45

A.1.1.2	Huawei	45

A.1.1.3	LGE	46

A.1.1.4	Qualcomm	46

A.1.2	Scenario 2: 4GHz Macro → Macro (UL)	47

A.1.2.1	Ericsson	47

A.1.2.1.1	100% grid shift	47

A.1.2.1.1.1	100% utilization	47

A.1.2.1.1.2	50% utilization	48

A.1.2.1.1.3	10% utilization	48

A.1.2.1.2	Other grid shifts	48

A.1.2.2	Huawei	49

A.1.2.3	LGE	50

A.1.3	Scenario 3: 4GHz Macro → Indoor (UL)	50

A.1.3.1	Huawei	50

A.1.3.2	Nokia	51

A.1.3.2.1	Full buffer	51

A.1.3.2.2	FTP3 with 10% load	52

A.1.4	Scenario 4: 4GHz Macro → Indoor (DL)	53

A.1.4.1	Huawei	53

A.1.4.2	Nokia	54

A.1.4.2.1	Full buffer	54

A.1.4.2.2	FTP3 with 10% load	55

A.1.4.3	Qualcomm	55

A.1.5	Scenario 5: 4GHz Indoor → Macro (UL)	56

A.1.5.1	Huawei	56

A.1.5.2	LGE	56

A.1.5.3	Nokia	58

A.1.5.3.1	Full buffer	58

A.1.5.3.2	FTP3 with 10% load	59

A.1.6	Scenario 6: 4GHz Indoor → Macro (DL)	60

A.1.6.1	Huawei	60

A.1.6.2	LGE	60

A.1.6.3	Nokia	62

A.1.6.3.1	Full buffer	62

A.1.6.3.2	FTP3 with 10% load	63

A.1.7	Scenario 7: 4GHz Indoor → Indoor (UL)	64

A.1.7.1	Ericsson	64

A.1.7.1.1	100% utilization	64

A.1.7.1.2	0% utilization	65

A.1.7.1.3	10% utilization	65

A.1.7.2	Huawei	65

A.1.7.3	LGE	66

A.1.7.4	Nokia	67

A.1.7.4.1	Full buffer	67

A.1.7.4.2	FTP3 with 10% load	68

A.1.8	Scenario 8: 4GHz Indoor → Indoor (DL)	69

A.1.8.1	Ericsson	69

A.1.8.1.1	100% utilization	69

A.1.8.1.2	50% utilization	70

A.1.8.1.3	10% utilization	71

A.1.8.2	Huawei	71

A.1.8.3	LGE	72

A.1.8.4	Nokia	73

A.1.8.4.1	Full buffer	73

A.1.8.4.2	FTP3 with 10% load	74

A.1.8.5	Qualcomm	74

A.2	FR2	75

A.2.1	Scenario 9: 30 GHz Macro → Macro (DL)	75

A.2.1.1	Ericsson	75

A.2.1.2	Huawei	75

A.2.1.3	LGE	76

A.2.1.4	Nokia	76

A.2.1.4.1	Full buffer	76

A.2.1.4.2	FTP3 with 10% load	77

A.2.1.5	Qualcomm	77

A.2.2	Scenario 10: 30 GHz Macro → Macro (UL)	78

A.2.2.1	Ericsson	78

A.2.2.1.1	100% grid shift	78

A.2.2.1.1.1	100% Utilization	78

A.2.2.1.1.2	50% Utilization	78

A.2.2.1.1.3	10% Utilization	79

A.2.2.1.2	50% grid shift	79

A.2.2.1.2.1	100% Utilization	79

A.2.2.1.2.2	50% Utilization	79

A.2.2.1.2.3	10% Utilization	80

A.2.2.2	Huawei	80

A.2.2.3	LGE	81

A.2.3	Scenario 11: 30 GHz Micro → Micro (UL)	81

A.2.3.1	Ericsson	81

A.2.3.2	Huawei	82

A.2.4	Scenario 12: 30 GHz Micro → Micro (DL)	82

A.2.4.1	Ericsson	82

A.2.4.2	Huawei	83

A.2.4.3	Qualcomm	83

A.2.5	Scenario 13: 30 GHz Indoor → Macro (UL)	84

A.2.5.1	Huawei	84

A.2.5.2	LGE	84

A.2.6	Scenario 14: 30 GHz Indoor → Macro (DL)	85

A.2.6.1	Huawei	85

A.2.6.2	LGE	85

A.2.7	Scenario 15: 30 GHz Indoor → Indoor (UL)	86

A.2.7.1	Ericsson	86

A.2.7.2	Huawei	87

A.2.7.3	LGE	87

A.2.7.4	Nokia	88

A.2.7.4.1	Full buffer	88

A.2.7.4.2	FTP3 with 10% load	89

A.2.8	Scenario 16: 30 GHz Indoor → Indoor (DL)	90

A.2.8.1	Ericsson	90

A.2.8.2	Huawei	90

A.2.8.3	LGE	91

A.2.8.4	Nokia	92

A.2.8.4.1	Full buffer	92

A.2.8.4.2	FTP3 with 10% load	93

A.2.8.5	Qualcomm	93

Annex B:	 Detailed analysis for zero grid shift	94

Annex C:	 Change history	95

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document provides the results of co-existence evaluation for CLI and RIM for NR.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-182864: "Revised WID on Cross link Interference (CLI) handling and Remote Interference Management (RIM) for NR".

[3]	3GPP TR 38.802: "Study on new radio access technology Physical layer aspects".

[4]	3GPP TR 38.866: "Study on remote interference management for NR".

[5]	3GPP TR 38.803: “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”

[6]	3GPP TR 36.828: “Further enhancements to LTE Time Division Duplex (TDD) for Downlink-Uplink (DL-UL) interference management and traffic adaptation”

[7]	3GPP TS 38.101-1: “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[8]	3GPP TR 36.873: “Study on 3D channel model for LTE”

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Downlink: in general the direction from a Network to a UE.

Macro cells: “Macro cells” are outdoor cells with a large cell radius.

Micro cells: “Micro cells” are small cells.

Throughput: A parameter describing service speed. The number of data bits successfully transferred in one direction between specified reference points per unit time (source: ITU-T I.113).

Uplink: An “uplink” is a unidirectional radio link for the transmission of signals from a UE to a base station, from a Mobile Station to a mobile base station or from a mobile base station to a base station.

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Power Ratio

ACS	Adjacent Channel Selectivity

BS	Base Station

CLI	Cross Link Interference

DL	Downlink

eNB	E-UTRAN Node B

FDD	Frequency Division Duplex

gNB	Next generation Node B

NR	New Radio

RIM	Remote Interference Management

SINR	Signal-to-Interference Noise Ratio

TDD	Time division duplex(ing)

TDM	Time division multiplex(ing)

UE	User Equipment

UL	Uplink

# 4	Background

## 4.1	General

RAN4 requirements designed to ensure co-existence between different networks operating on adjacent carriers in the same band (i.e. ACLR, ACS) were developed in release 15 under the assumption of synchronized TDD. Interference between adjacent carriers is mitigated as long as all networks apply uplink and downlink at the same occasions.

Dynamic TDD describes a mode of operation in which a network adapts the DL/UL subframe pattern according to traffic conditions. If different nodes in the same network apply DL and UL at different times, then interference between different UEs and different BS occurs. RAN1 has specified measurements to enable co-channel Cross Link Interference (CLI) mitigation within the same network.

Dynamic TDD also causes interferers between networks on adjacent channels. Unlike the co-channel case, interference between adjacent channel networks cannot be coordinated. Instead, the interference is mitigated by transmitter and receiver selectivity (ACLR and ACS) as analogue filtering is not generally feasible within an operating band.

As part of the CLI WI, RAN4 has been tasked to investigate the adjacent channel co-existence effects arising when CLI, or more generically dynamic TDD is operated in an aggressor network. The scope of the investigation is to target no or very minimal impact on RF requirements, and so the investigation examines the throughput losses experienced in a victim network assuming that all base stations and UEs conform to the release 15 requirements.

This report captures a description of the adjacent channel interference effects that arise with dynamic TDD as well as a simulation investigation of adjacent channel interference in a number of different deployment scenarios.

## 4.2	WID description

### 4.2.1	Justification

As captured in TR38.802, NR aims to support duplexing flexibility in both paired and unpaired spectrum.

“NR supports paired and unpaired spectrum and strives to maximize commonality between the technical solutions, allowing FDD operation on a paired spectrum, different transmission directions in either part of a paired spectrum, TDD operation on an unpaired spectrum where the transmission direction of time resources is not dynamically changed, and TDD operation on an unpaired spectrum where the transmission direction of most time resources can be dynamically changing. DL and UL transmission directions at least for data can be dynamically assigned on a per-slot basis at least in a TDM manner. It is noted that transmission directions include all of downlink, uplink, sidelink, and backhaul link. NR supports at least semi-statically assigned DL/UL transmission direction as gNB operation, i.e., the assigned DL/UL transmission direction can be signaled to UE by higher layer signaling.”

Rel-14 NR study showed that duplexing flexibility with cross-link interference mitigation shows better user throughput compared to static UL/DL operation or dynamic UL/DL operation without interference mitigation in indoor hotspot (4GHz and 30GHz) and urban macro scenarios (4GHz and 2GHz). The mitigation techniques include coordinated scheduling scheduling/beamforming, power control, link adaptation, hybrid dynamic/static UL/DL resource assignment.

During Rel-15 NR WI, enablers for basic support of cross-link interference mitigation schemes to support duplexing flexibility for paired and unpaired spectrum were discussed but could not specified as the work has been deprioritized. Mainly, it was agreed to introduce UE-to-UE measurement for CLI, and TRP-to-TRP measurement/coordination techniques were discussed. Rel-15 NR specification supports mechanisms to allow dynamic DL/UL assignments. Yet, any cross-link interference mitigation techniques and coexistence requirements have not been specified and thus the use of dynamic DL/UL assignment operation is considerably restricted.

As observed during study, dynamic DL/UL resource assignments would be more beneficial in indoor hotspot and urban macro scenarios where gNB TX power is rather limited, and ISD is small. During the study, dynamic DL/UL assignment operations offers performance gain in high frequencies such as 4 GHz and 30 GHz. In such frequencies, advanced MIMO techniques can be utilized to mitigate cross-link interference which has not been fully evaluated.

Furthermore, semi-static and/or dynamic DL/UL resource assignments should also consider coexistence issues particularly among different operators where tight coordination are challenged. Coexistence mechanisms among different operators in co-channel and adjacent channels are essential regardless whether each cell operates in semi-static or dynamic DL/UL assignment mechanism. For efficient coexistence, not only coexistence requirements need to be understood but also advanced mechanisms to mitigate interference such as TRP-to-TRP measurement and adaptation based on measurements should be considered.

In commercial TD-LTE network with macro deployment scenario, it is observed in relatively large scale of eNBs that IoT (amounts to above -105dBm, even to -90dBm in some extreme cases) in these eNBs intermittently deteriorated to severely impact the network coverage and connection successful rate. Jointed with several approaches, e.g., IoT statistics from the eNBs in some regions over the forecast of troposphere bending (http://www.dxinfocentre.com/tropo.html) , as well as the symptoms varying with the artificially constructed transmission patterns, it is identified that this kind of IoT degradation is caused by the downlink signal of remote eNB (furthest as 300km away with observed record) as long as the atmospheric conditions favourable for producing troposphere bending of radio waves are available. To mitigate the impact of this kind of remote base station interference as it intermittently happens, but not to scarify the network resources all the time, in the field TD-LTE network, some adaptive mechanisms are introduced, where abnormal IoT enhancement will trigger the victim eNB to transmit a specific signal in a window, each eNB that detected the specific signal in a window will identify itself as the contributor of the deteriorated IoT in some eNB(s), and then it will reconfigure the GP or some other parameters to reduce its weight to the interference. In this framework, the impact of troposphere bending is mitigated to some extension, however, some disadvantages are also apparent in the proprietary implementation, e.g., rely on some static mechanism for detection signal transmission and detection  due to lack inter-vendor inter-eNB coordination, decision making is per individual eNB based implementation, etc.

In NR deployment on lower TDD frequency, the impact of the troposphere bending will continue existing if no special mechanisms are introduced. Though the design of the frame structure in NR has already considered much more flexible GP to leave larger room for avoiding the remote interference, it is necessary to study mechanisms for identifying when or how long will the long enough GP be configured, as well as corresponding gNB’s behaviour and inter-gNB’s coordination procedure.

In the RIM SI, the frameworks for mechanisms for gNBs to start and terminate the transmission/detection of reference signal(s), the functionalities and requirements of the corresponding RS(s) as well as the design of the RS(s), and the backhaul-based coordination mechanisms among gNBs have been studied, where the outcome and conclusions of the SI was captured in TR38.866. It is recommended to specify RIM RS(s) to support identifying remote interference related information, it is also recommended to specify the inter-set RIM backhaul signalling via the core network for backhaul-based solution.

### 4.2.2	Objective

In the WID on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR , the detailed objectives for cross-link interference mitigation to support flexible resource adaptation for unpaired NR cells are:

-	Specify cross-link interference measurements and reporting at a UE (e.g., CLI-RSSI and/or CLI-RSRP) [RAN1, RAN2, RAN4]

-	Perform coexistence study to identify conditions of coexistence among different operators in adjacent channels [RAN4]

-	Target no or very minimal impact on RF requirement

NOTE:	Measurement and coordination mechanisms should be applicable to IAB nodes.

The detailed objectives for remote-interference management are:

-	Specify RIM RS resource and configurations, including [RAN1]

-	A basic RIM-RS resource

-	Configuration of RIM-RS and distinguishable RIM RS-1/2 resources, including sequence type, time and frequency transmission pattern

-	Determine gNB set identification information through detection of RIM-RS(s) by implicit or explicit indication. Determine further information that can be carried by the RIM-RS, such as “Ducting phenomenon exists”, “Enough mitigation” & “Not enough mitigation”,   [RAN1, SA5]

-	Specify the inter-set RIM backhaul signalling via the core network to convey the messages of “RIM-RS detected” and “RIM-RS disappeared [RAN3]

-	Identify corresponding OAM functions to support RIM operation [RAN1, RAN3].

This technical report focuses on evaluating co-existence of CLI among different operators in adjacent channels and RAN4 targets no or very minimal impact on RF requirement.

## 4.3	Dynamic TDD adjacent interference scenarios

### 4.3.1	General

This section provides a description of the interference scenarios that occur between different TDD networks within the same operating band. Section 4.3.2 describes interference scenarios that occur during periods of time in which both networks transmit DL or both UL. These interference scenarios occur in both synchronized and unsynchronized networks. Section 4.3.3 describes additional interference scenarios that arise in unsynchronized networks when one network is transmitting downlink whilst another transmits uplink.

### 4.3.2	Interference scenarios that occur for both synchronized and unsynchronized TDD (including CLI)

In synchronized TDD networks, there are two aspects of co-existence; DL-DL and UL-UL. For DL-DL co-existence, UEs in a victim network are impacted by nearby BS from an aggressor network during DL subframes.

Figure 4.3.2-1: DL-DL inter-operator interference scenario

The impact of the aggressor BS may be twofold; adjacent channel leakage from the aggressor BS may raise the interference floor at the victim UE. Alternatively, the wanted power from the aggressor BS may leak into the wanted carrier due to the UE adjacent channel selectivity. Thus, BS ACLR and UE ACS are the key RF parameters relating to DL-DL co-existence.

UL-UL co-existence refers to degradation of a BS receiver due to the impact of nearby aggressor network UEs during UL subframes.

Figure 4.3.2-2: UL-UL inter-operator interference scenario

The impact of the UEs may be twofold; adjacent channel leakage from the aggressor UEs may degrade the BS interference floor. Alternatively, the wanted power from aggressor UEs may leak into the BS receive carrier due to the adjacent channel selectivity of the BS. Thus, UE ACLR and BS ACS are the key RF parameters relating to UL-UL coexistence.

When operating dynamic TDD or CLI, BS to UE and UE to BS interference of this type can occur during subframes that are aligned in the same direction for both operators.

### 4.3.3	Additional interference scenarios that occur for unsynchronized TDD (including dynamic TDD and CLI)

Dynamic TDD in at least one network implies additional scenarios for adjacent channel interference. When both networks transmit in the same direction, then the DL-DL and UL-UL scenarios described in section 4.3.2 still occur. However, there will be additional instances of DL (aggressor) – UL (victim) and UL (aggressor) – DL (victim).

CLI techniques may be used within an individual network to avoid interference due to dynamic TDD. However, between adjacent frequency networks, it must be assumed that the interference is not coordinated and hence the interference is only mitigated by suppression of adjacent channel and interference and receiver selectivity (ACLR/ACS).

In the DL (aggressor) – UL (victim) scenario, an aggressor BS is transmitting whilst BS in a receiver network are receiving.

Figure 4.3.3-1: DL-UL adjacent channel interference scenario

For co-located BS, the interference from the aggressor will cause blocking, as described in section 4.4. For non-co-located BS, the extent of degradation will depend on the inter-site distance and the antenna gains of the two BS.

In the UL (aggressor) – DL (victim) scenario, an aggressor UE is transmitting whilst close to a victim UE.

Figure 4.3.3-2: UL-DL adjacent channel interference scenario

The statistical co-existence will depend on UE TX power, separation, traffic etc. and needs to be explored with system simulation.

## 4.4	BS-BS interference for zero grid shift scenarios

When dynamic TDD is operated by at least two operators that are co-located at the same site and transmit in the same band, then BS-BS interference will occur. If the victim is receiving and the aggressor transmitting, then the victim will suffer interference. Conversely, if the aggressor is receiving and the victim transmitting, then the aggressor will suffer the interference.

It was concluded that if dynamic TDD is operated at co-located base stations, during subframes in which the victim BS is receiving uplink whilst an aggressor BS is transmitting downlink the victim BS receiver would fail due to interference from the aggressor. No simulation analysis is required for the co-located base station scenario, since the uplink receiver blocking will occur in all subframes for which one BS receives whilst the other transmits. Details of the analysis leading to this conclusion are provided in Annex B.

# 5	Co-existence analysis

## 5.1	Scenarios

### 5.1.1	FR1

Table 5.1.1-1 summarizes agreed simulation scenarios for FR1 (4 GHz).

Table 5.1.1-1: Summary of simulation scenarios for FR1 (4 GHz)

| Scenario No. | Deployment Scenario (Aggressor -> Victim) | Aggressor baseline | Aggressor in CLI | Victim |
| --- | --- | --- | --- | --- |
| 1 | Macro → Macro | NR, 100 MHz, DL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, UL100% | NR, 100 MHz, DL |
| 2 |  | NR, 100 MHz, UL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, DL100% | NR, 100 MHz, UL |
| 3 | Macro → Indoor | NR, 100 MHz, DL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, UL100% | NR, 100 MHz, DL |
| 4 |  | NR, 100 MHz, UL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, DL100% | NR, 100 MHz, UL |
| 5 | Indoor → Macro | NR, 100 MHz, DL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, UL100% | NR, 100 MHz, DL |
| 6 |  | NR, 100 MHz, UL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, DL100% | NR, 100 MHz, UL |
| 7 | Indoor → Indoor | NR, 100 MHz, DL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, UL100% | NR, 100 MHz, DL |
| 8 |  | NR, 100 MHz, UL | 1. NR, 100 MHz, DL50%+UL50% 2. NR, 100MHz, DL100% | NR, 100 MHz, UL |

### 5.1.2	FR2

Table 5.1.2-1 summarizes agreed simulation scenarios for FR2 (30 GHz).

Table 5.1.2-1: Summary of simulation scenarios for FR2 (30 GHz)

| Scenario No. | Deployment Scenario (Aggressor -> Victim) | Aggressor baseline | Aggressor in CLI | Victim |
| --- | --- | --- | --- | --- |
| 9 | Macro → Macro | NR, 200 MHz, DL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, UL100% | NR, 200 MHz, DL |
| 10 |  | NR, 200 MHz, UL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, DL100% | NR, 200 MHz, UL |
| 11 | Micro → Micro | NR, 200 MHz, DL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, UL100% | NR, 200 MHz, DL |
| 12 |  | NR, 200 MHz, UL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, DL100% | NR, 200 MHz, UL |
| 13 | Indoor → Macro | NR, 200 MHz, DL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, UL100% | NR, 200 MHz, DL |
| 14 |  | NR, 200 MHz, UL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, DL100% | NR, 200 MHz, UL |
| 15 | Indoor → Indoor | NR, 200 MHz, DL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, UL100% | NR, 200 MHz, DL |
| 16 |  | NR, 200 MHz, UL | 1. NR, 200 MHz, DL50%+UL50% 2. NR, 200MHz, DL100% | NR, 200 MHz, UL |

## 5.2	Simulation assumptions

### 5.2.1	FR1

#### 5.2.1.1	Network layout model

##### 5.2.1.1.1	Urban macro

Details on urban macro network layout model are listed in Table 5.2.1.1.1-1.

Table 5.2.1.1.1-1: Single operator layout for urban macro in FR1 (4GHz)

| Layout | Single layer with 19 hexagonal cell with wrap around |
| --- | --- |
| Inter-BS distance | 500m |
| Carrier frequency | 4 GHz |
| Path-loss model | - Macro(Aggressor) → Macro(Victim) - Macro-to-UE: UMa [5] - Macro-to-Macro: UMa (h_UE=25m) [5] - UE-to-UE: Outdoor UE – Outdoor UE [6] + penetration loss [5] |
| BS Tx power | 49 dBm |
| UE Tx power | 23dBm |
| BS antenna configurations | (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ Note 1,2 |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 5dBi (assuming antenna 1.8dB loss) |
| BS receiver noise figure | 5dB |
| UE antenna configuration | Omni |
| UE antenna height | hUT=3(nfl-1)+1.5 nfl for outdoor UEs: 1 nfl for indoor UEs: nfl~uniform(1,Nfl) where Nfl = 1 |
| UE antenna gain | 0 dBi |
| UE receiver noise figure | 9dB |
| Multi operators layout | uncoordinated operation (100% Grid Shift) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. |  |

##### 5.2.1.1.2	Indoor

Details on indoor network layout model are listed in Table 5.2.1.1.2-1.

Table 5.2.1.1.2-1: Single operator layout for Indoor scenarios in FR1 (4GHz)

| Layouts | 1. Indoor-to-Indoor : 6 BSs per 120m x 50m 2. Indoor-to-Macro : the number of Indoor per macro cell (drop randomly) = 1 |
| --- | --- |
| Inter-BS distance | Indoor-to-Indoor: 20m |
|  | The minimum distance between Macro to Indoor: [35]m |
| Minimum BS-UE (2D) distance | Indoor-to-Indoor: 0m |
| Minimum UE-UE (2D) distance | Indoor-to-Indoor: 1m~3m |
| Carrier frequency | 4GHz |
| BS TX power | 24 dBm |
| UE TX power | 23 dBm |
| Path-loss model | - Indoor (Aggressor) → Macro (Victim): - BS-to-BS: InH-office [5] + penetration loss [5] - BS-to-UE: InH-office [5] + penetration loss [5] - UE-to-UE: Outdoor UE – Outdoor UE [6] + penetration loss [5] - Indoor (Aggressor) → Indoor (Victim) - BS-to-BS: InH-office [5] - BS-to-UE: InH-office [5] - UE-to-UE: InH-office [5] |
| BS antenna | FR1 BS antenna element pattern for Indoor scenario from subclause 5.2.1.5.1 / ceiling |
| BS antenna height: | 3m |
| UE antenna | Omni |
| UE antenna height | 1.5m |
| Antenna gain of UE | 0 dBi |
| Cell selection criteria | Cell selection is based on RSRP |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| UE power control | Power control as defined in Section 5.2.3.4 |
| Multi operators layout | uncoordinated operation (100% Grid Shift) |

#### 5.2.1.2	ACIR

Table 5.2.1.2-1: ACIR for FR1

| Parameter | Assumption/Value |
| --- | --- |
| ACIR BS-BS | 43dB |
| ACIR BS-UE | 33dB |
| ACIR UE-BS | 30dB |
| ACIR UE-UE | 28dB |

#### 5.2.1.3	UE distribution

Table 5.2.1.3-1: UE distribution for FR1

| Scenarios | UE distribution |
| --- | --- |
| Indoor-to-Indoor | Indoor -> Indoor = 1 user per Transmission Reception Point; 100% indoor |
| Macro-to-Indoor | Indoor <-> macro = 1 user per Transmission Reception Point; Indoor has 100% indoor UE. Macro victim has 50% indoor UE and 50% outdoor. Indoor <-> macro = Aggressor: 1 user per Transmission Reception Point, 100% indoor. Victim: 1 user per Transmission Reception Point, 100% outdoor |
| Urban Macro (Macro-to-Macro) | 20% indoor and 80% outdoor |

#### 5.2.1.4	Other simulation parameters

Table 5.2.1.4-1: Other simulation parameters for FR1

| Parameters | Indoor | Urban macro |
| --- | --- | --- |
| Channel bandwidth | 100MHz | 100MHz |
| Scheduled channel bandwidth per UE (DL) | 100MHz | 100MHz |
| Scheduled channel bandwidth per UE (UL) | 100MHz | 100MHz |
| Traffic model | Low (RU 10%) and Full buffer | Low (RU 10%) and Full buffer |
| DL power control | NO | NO |
| UL power control | YES | YES |
| BS max TX power in dBm TRP (Total Radiated Power) | 24 dBm | 49 dBm |
| UE max TX power in dBm | 23 dBm | 23 dBm |
| UE min TX power in dBm | -33 dBm (100 MHz CBW) [7] | -33 dBm (100 MHz CBW) [7] |
| BS Noise figure in dB | 5 dB | 5 dB |
| UE Noise figure in dB | 9 dB | 9 dB |
| Handover margin | 3 dB (Same as FR2) | 3 dB (Same as FR2) |

#### 5.2.1.5	Antenna configuration

##### 5.2.1.5.1	Urban macro scenario

Table 5.2.1.5.1-1: FR1 BS antenna element pattern for Urban Macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=65°,SLAV=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=25dB |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi (assuming 1.8dB loss) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree horizontal element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results. |  |

##### 5.2.1.5.2	Indoor scenario

Table 5.2.1.5.2-1: FR1 BS antenna element pattern for Indoor scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3.5 dBi (assuming 1.8dB loss) |

##### 5.2.1.5.3	UE antenna element pattern

The UE antenna element is assumed to be omnidirectional with 0dBi gain.

### 5.2.2	FR2

#### 5.2.2.1	Network layout model

##### 5.2.2.1.1	Urban macro

Table 5.2.2.1.1-1: Single operator layout for urban macro in FR2 (30 GHz)

| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| --- | --- |
| Inter-site distance | 200m |
| BS antenna height | 25 m |
| Path-loss model | - Macro (Aggressor) – Macro (Victim) - Macro-to-Macro: UMa (h_UE=25m) [5] - Macro-to-UE(V): Uma + penetration loss[5] - UE-to-UE: UMi (h_BS=1.5m ~ 22.5m) + penetration loss [5] |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |
| Multi operators layout | uncoordinated operation (100% Grid Shift) |

##### 5.2.2.1.2	Dense urban

Table 5.2.2.1.2-1: Single operator layout for Dense urban in FR2 (30 GHz)

| Parameters | Values | Remark |
| --- | --- | --- |
| Network layout | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |
| Radius of UE dropping within a micro cell | < 28.9 m |  |
| BS antenna height | 10 m |  |
| Channel model | Micro (A) – Micro (V) [5] - Micro-to-Micro: UMi (h_UE=10m) - Micro-to-Micro UE: UMi + penetration loss - Micro (UE)-to-Micro (UE): UMi (h_BS=1.5m ~ 22.5m) + penetration loss between UEs |  |
| Shadowing correlation | Between cite: 0.5 |  |
| Multi operator layout | Cluster circle is coordinated | Note 2 |
| Minimum distance between micro BSs in different operator | 10 m |  |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD = 200m is assumed. Note 2: Macro cell is collocated. Micro BS itself is randomly dropped. |  |  |

##### 5.2.2.1.3	Indoor

Table 5.2.2.1.3-1: Single operator layout for Indoor scenarios in FR2 (30 GHz)

| Parameters | Values | Remark |
| --- | --- | --- |
| Network layout | Indoor-to-Indoor : Total 12 BSs (operator A: 6 BSs & operator B: 6 BSs) 120m x 50m Indoor-to-macro: Indoors are placed at different locations |  |
| Inter-site distance | Indoor – Indoor = 20m |  |
|  | The minimum distance between Macro to Indoor: [35]m |  |
| BS antenna height | 3 m | ceiling |
| Path-loss model | Indoor(Aggressor) → Indoor(Victim) - BS-to-BS: InH-office [5] - BS-to-UE: InH-office [5] - UE-to-UE: InH-office [5] Indoor (Agressor) → Macro (Victim) - BS-to-BS: InH-office (h_UE=3m) + penetration loss [5] - BS-to-UE: InH-office (h_UE=3m) + penetration loss [5] - UE-to-UE: InH-office (h_BS=1.5m) + penetration loss [5] |  |
| Shadowing correlation | N/A |  |
| Multi operators layout for indoor | Uncoordinated operation (100%) |  |

#### 5.2.2.2	ACLR and ACS

Table 5.2.2.2-1: ACLR and ACS for FR2

| Parameter | Assumption/Value |
| --- | --- |
| BS ACLR | 28 dB |
| UE ACLR | 17 dB |
| BS ACS | 23.5 dB |
| UE ACS | 23 dB |

#### 5.2.2.3	UE distribution

##### 5.2.2.3.1	Urban Macro (Macro-to-Macro)

Table 5.2.2.3.1-1: UE distribution for Urban Macro case in FR2

| UE location | Outdoor/indoor | Outdoor and indoor |
| --- | --- | --- |
|  | Indoor UE ratio | 0% |
|  | LOS/NLOS | LOS and NLOS |
|  | UE antenna height | 1.5 m ≦ hUT ≦ 22.5 m |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 35 m |

##### 5.2.2.3.2	Dense Urban (Micro-to-Micro)

Table 5.2.2.3.2-1: UE distribution for Dense Urban case in FR2

| UE location | Outdoor/indoor | Outdoor and indoor |
| --- | --- | --- |
|  | Indoor UE ratio | 80 % |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |
|  | LOS/NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 [8] |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 3m |

##### 5.2.2.3.3	Indoor-to-Indoor and Indoor-to-Macro

Table 5.2.2.3.3-1: UE distribution for Indoor cases in FR2

| Scenarios | UE distribution |
| --- | --- |
| Indoor-to-Indoor | Indoor -> Indoor = 1 user per Transmission Reception Point; 100% indoor |
| Macro-to-Indoor | Indoor <-> macro = Aggressor: 1 user per Transmission Reception Point, 100% indoor. Victim: 1 user per Transmission Reception Point, 100% outdoor |

#### 5.2.2.4	Other simulation parameters

Table 5.2.2.4-1: Other simulation parameters

| Parameters | Indoor | Urban macro | Dense urban |
| --- | --- | --- | --- |
| Channel bandwidth | 200MHz | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (DL) | 200MHz | 200MHz | 200MHz |
| Scheduled channel bandwidth per UE (UL) | 200MHz | 200MHz | 200MHz |
| Traffic model | Low (RU 10%) and Full buffer | Low (RU 10%) and Full buffer | Low (RU 10%) and Full buffer |
| DL power control | NO | NO | NO |
| UL power control | YES | YES | YES |
| BS max TX power in dBm TRP (Total Radiated Power) | 23dBm | 43dBm | 33dBm |
| UE Peak EIRP in dBm | 22.4 dBm | 22.4 dBm | 22.4 dBm |
| UE min TX power in dBm | -40dBm | -40dBm | -40dBm |
| BS Noise figure in dB | 10 (note 1) | 10 (note 1) | 10 (note 1) |
| UE Noise figure in dB | 10 (note 1) | 10 (note 1) | 10 (note 1) |
| Handover margin | 3dB | 3dB | 3dB |
| Note 1: ITU WP5D response |  |  |  |

#### 5.2.2.5	Antenna configuration

BS Antenna modelling [subcaluse 5.2.3.2 in TR 38.803 [5]]

##### 5.2.2.5.1	Urban macro scenario

Table 5.2.2.5.1-1: FR2 BS antenna modelling for urban macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| (Mg, Ng, M, N, P) | For 30 GHz: (1, 1, 8, 16, 2) Note1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results. |  |

##### 5.2.2.5.2	Dense urban scenario

Table 5.2.2.5.2-1: FR2 BS antenna element pattern for dense urban scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| BS antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) Note 1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results |  |

##### 5.2.2.5.3	Indoor scenario

Table 5.2.2.5.3-1: FR2 BS antenna element pattern for indoor scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 2dB loss) |
| (Mg, Ng, M, N, P) note | For 30 GHz: (1, 1, 4, 8, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 90 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation |  |

##### 5.2.2.5.4	UE antenna element pattern

Table 5.2.2.5.4-1: FR2 UE antenna element pattern

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 5dBi directivity and 2dB loss) |
| BS antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) Note 1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees Note 3 Fixed elevation: 90 degrees |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note3: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. Note 4: A 90 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation |  |

### 5.2.3	Common assumptions

#### 5.2.3.1	Propagation model

The Path loss model is summarized in Table 5.2.3.1-1. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 5.2.3.1-1.

Table 5.2.3.1-1: Path-loss models

| Scenario | Pathloss [dB], fc is in GHz and d is in meters (6) | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- |
| UMa LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 25 m |
| UMa NLOS |  | σSF =6 | 10 m < d2D < 5 000 m 1.5 m ≦ hUT ≦ 22.5 m hBS = 25 m Explanations: see note 3 |
| UMi - Street Canyon LOS |  | σSF=4.0 σSF=4.0 | 10m < d2D < d'BP 1) d'BP < d2D <5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m |
| UMi – Street Canyon NLOS |  | σSF=7.82 | 10 m < d2D < 5000m 1.5m ≦ hUT≦ 22.5m hBS = 10 m Explanations: see note 4 |
| InH - Office LOS |  | σSF=3.0 | 1<d3D<100m |
| InH - Office NLOS |  | σSF=8.03 | 1<d3D<86m |
| Note 1: d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. In UMi scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – 1.0 m, h'UT = hUT–1.0 m, where hBS and hUT are the actual antenna heights, and the effective environment height is assumed to be equal to 1.0 m. In UMa scenario the effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and the effective environment height hE is a function of the link between a BS and a UT. In the event that the link is determined to be LOS, hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. Note 2: The applicable frequency range of the PL formula in this table is 0.8 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified format and and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |

#### 5.2.3.2	LOS model

The Line-Of-Sight (LOS) probabilities are given in Table 5.2.3.2-1.

Table 5.2.3.2-1: LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMi – Street canyon | Outdoor users: Indoor users: Use d2D-out in the formula above instead of d2D |
| UMa | Outdoor users: where and Indoor users: Use d2D-out in the formula above instead of d2D |
| Indoor – Open office |  |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for Uma |  |

#### 5.2.3.3	O-to-I penetration loss

The Path loss incorporating O-to-I building penetration loss is modelled as in the following:

PL = PLb + PLtw + PLin + N(0, σP2)

where PLb is the basic outdoor path loss given in Section 5.1.2.2.1. PLtw is the building penetration loss through the external wall, PLin is the inside loss dependent on the depth into the building, and σP  is the standard deviation for the penetration loss.

PLtw is characterized as:

is an additional loss is added to the external wall loss to account for non-perpendicular incidence;

, is the penetration loss of material i, example values of which can be found in Table 5.2.3.3-1.

pi is proportion of i-th materials, where ; and

N is the number of materials.

Table 5.2.3.3-1: Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass | Lglass=2+0.2⋅f |
| IRR glass | LIRRglass=23+0.3⋅f |
| Concrete | Lconcrete=5+4⋅f |
| Wood | Lwood=4.85+0.12⋅f |
| Note: f is in GHz |  |

Table 5.2.3.3-2 gives PLtw, PLin and σP  for two O-to-I penetration loss models. The O-to-I penetration is UT-specifically generated, and is added to the SF realization in the log domain.

Table 5.2.3.3-2: O-to-I penetration loss model

|  | Path loss through external wall: PLtw [dB] | Indoor loss: PLin [dB] | Standard deviation: σP [dB] |
| --- | --- | --- | --- |
| Low-loss model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lconcrete10 | 0.5d2D-in | 4.4 |
| High-loss model | 5-10log100.7⋅10-LIRRglass10+0.3⋅10-Lconcrete10 | 0.5d2D-in | 6.5 |

d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for RMa, UMa and UMi-Street Canyon. d2D-in shall be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

The composition of low and high loss is a simulation parameter that should be determined by the user of the channel models, and is dependent on the use of metal-coated glass in buildings and the deployment scenarios. Such use is expected to differ in different markets and regions of the world and also may increase over years to new regulations and energy saving initiatives. Furthermore, the use of such high-loss glass currently appears to be more predominant in commercial buildings than in residential buildings in some regions of the world.

The pathloss incorporating O-to-I car penetration loss is modelled as in the following:

PL = PLb + N(μ, σP2)

where PLb is the basic outdoor path loss given in Section 7.4.1. μ = 9, and σP = 5. Optionally, for metallized car windows, μ = 20 can be used. The O-to-I car penetration loss models are applicable for at least 0.6-60 GHz.

#### 5.2.3.4	Transmission power control model

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied with following parameters.

-	CLx-ile = 88 + 10*log10(200/X), where X is UL transmission BW (MHz)

-	γ = 1

#### 5.2.3.5	Received signal power model

The following model is applied.

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

RX_PWR is the received power.

TX_PWR is the transmitted power.

G_TX is the transmitter antenna gain (directional array gain).

G_RX is the receiver antenna gain (directional array gain).

#### 5.2.3.6	Evaluation metric

Table 5.2.3.6-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

#### 5.2.3.7	Antenna modelling

Note the above gives the correct antenna array radiation pattern, however the correct gain is only achieved if the element pattern  is selected for the exact element spacing. For other element spacing’s, the element pattern  must be separately calculated such that it is correct for the element spacing (dg,H and dg,V). If  is not linked to the element spacing then the calculated absolute gain may diverge from the correct value in a manner that varies as the beam is steered.

The correct composite array radiation pattern directivity(D) is given by:

,

The composite array radiation pattern gain can then be calculated as:

Where L is the Loss associated with the antenna. This is currently included in the estimate for element gain , and is 1.8dB.

#### 5.2.3.8	Simulation description

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

- UEs are distributed randomly across the network.

2.	UE associations: UEs are associated to base station based on coupling loss.

- Associations are made assuming a single element at both UE and BS.

3.	Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

4.	(Optional) Throughput is computed in the victim systems without considering ACI as below:

- ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

5.	Throughput is computed considering ACI as below:

- ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

NOTE:	Simulation results should be in the form of the throughput with adjacent network with DTDD and without DTDD and the relative difference between the two can be compared at 50% and 5% points.

## 5.3	Simulation results

### 5.3.1	General

This sub-clause captures the co-existence simulation results for scenarios in Table 5.1.1-1 and Table 5.1.1-2. The following cases are considered as follows:

-	Case 1: (Baseline) the transmission directions of aggressor and victim are set as all DL.

-	Case 2: The transmission direction of aggressor is randomly set as DL and UL with a 50% probability.

-	Case 3: The transmission direction of aggressor is set as UL, which is opposite to the victim’s transmission direction.

### 5.3.2	Simulation limitations

Some limitations may have the potential to influence the impact to neighbor operator networks in the co-existence simulation. The limitations are not necessarily specific to the CLI simulations. Some of these aspects may be as follows:

-	The simulations currently schedule a single UE at a time. For FR2 with beamforming, this makes most sense, but for FR1 and sectorized base station antennas, multi-user frequency domain scheduling could reveal more interference cases. There may be many UEs transmitting in uplink simultaneously resulting in a higher total radiated UL power. This could change the UL-to-DL and also UL-to-UL ACI interference environment.

-	Lack of co-channel interference mitigation schemes in the indoor scenarios. For any adjacent channel UL transmission to exceed the co-channel interference level, the interfering UE would need to be extremely close to the victim UE. Different geometries and ICIC mechanisms could improve the co-channel performance, thereby making adjacent channel interference more visible.

-	In addition to the base station geometry, the UE geometry may also have an impact on the results. The UEs are dropped in the simulations uniformly in the specified simulation area, whereas in reality users would sometimes group e.g. in a meeting room.

The RF performance parameters used in the simulation are according to RAN4 minimum requirements at the specified test points. For example, when it comes to UE performance:

-	The TX leakage on the adjacent channel (ACLR) depends on the actual resource allocation, with the specified minimum requirement typically occurring only with a fully populated channel.

-	ACLR also depends on transmit power, typically improving when less than maximum output power is configured.

### 5.3.3	FR1

#### 5.3.3.1	Scenario 1: 4 GHz Macro→ Macro (DL)

##### 5.3.3.1.1	Results

Table 5.3.3.1-1: SINR and throughput degradation for Macro aggressor Macro victim

| Source | Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | UL | 50DL/50UL | UL |
| Huawei (1905522) | 5% | -0.47 | -0.13 | -10.13 | -2.83 |
|  | 50% | -0.12 | -0.37 | -0.57 | -1.75 |
|  | 95% | -0.74 | -0.21 | 0.00 | 0.00 |
| LGE (1907601) | 5% | - | 2.5 | - | 31.33 |
|  | 50% | - | 0.04 | - | 0.2 |
|  | 95% | - | 0.2 | - | 0.47 |
| Ericsson (1906097) | 5% | <1 | <1 | 3.1 | 3.1 |
|  | 50% | <1 | <1 | 1.2 | 1.2 |
|  | 95% | <1 | <1 | 1.1 | 2.1 |
| Nokia (1907604) | 5% | -0.7 | -1.5 | -7.9 | -31.1 |
|  | 50% | -0.3 | -0.7 | -3 | -9 |
|  | 95% | -0.1 | -0.4 | 1.7 | -1.1 |
| Qualcomm (1906703) | 5% | - | -0.70 | - | -8.44 |
|  | 50% | - | -0.38 | - | -1.78 |
|  | 95% | - | -0.27 | - | 0 |

Table 5.3.3.1.1-2: SINR and throughput degradation for Macro aggressor Macro victim (low traffic)

| Source | Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | UL | 50DL/50UL | UL |
| LGE (1907601) | 5% | - | 0.58 |  | 7.61 |
|  | 50% | - | 0.07 |  | 0.33 |
|  | 95% | - | 0.55 |  | 1.33 |
| Ericsson (1906097) | 5% | <1 | <1 | 4 | 4 |
|  | 50% | <1 | <1 | 1.5 | 2 |
|  | 95% | <1 | <1 | <1 | <1 |

#### 5.3.3.2	Scenario 2: 4 GHz Macro→ Macro (UL)

##### 5.3.3.2.1	Results

Table 5.3.3.2.1-1: SINR and throughput degradation for Macro aggressor Macro victim

| Source | Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | DL | 50DL/50UL | DL |
| Huawei (1905522) | 5% | 0.27 | 0.76 | 4.98 | 13.90 |
|  | 50% | 0.28 | 0.52 | 1.70 | 3.19 |
|  | 95% | -0.02 | -0.03 | -0.11 | -0.15 |
| LGE (1907601) | 5% | - | 2.63 | - | 44.99 |
|  | 50% | - | 0.46 | - | 3.13 |
|  | 95% | - | 0.05 | - | 0.25 |
| Ericsson (1906097) (NOTE 1) | 5% | <1 | <1 | <1 | -1.3 |
|  | 50% | <1 | <1 | -1.5 | -3 |
|  | 95% | <1 | <1 | <1 | -1.1 |
| Nokia (1907604) | 5% | 0.1 | 0.8 | -3.1 | 2 |
|  | 50% | 0.1 | 0.4 | 2.2 | 2.8 |
|  | 95% | 0.1 | 0.3 | 0.4 | 0.1 |
| NOTE 1: Further Ericsson results in this scenario with reduced cell size and grid shift indicated throughput losses |  |  |  |  |  |

Table 5.3.3.2.1-2: SINR and throughput degradation for Macro aggressor Macro victim (low traffic)

| Source | Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | DL | 50DL/50UL | DL |
| LGE (1907601) | 5% | - | 1.08 | - | 21.72 |
|  | 50% | - | 0.08 | - | 0.52 |
|  | 95% | - | 0 | - | -0.02 |
| Ericsson (1906097) | 5% | <1 | <1 | -3.3 | -3.4 |
|  | 50% | <1 | <1 | <1 | -1 |
|  | 95% | <1 | <1 | <1 | <1 |

#### 5.3.3.3	Scenario 3: 4 GHz Macro → Indoor (DL)

##### 5.3.3.3.1	Results

Table 5.3.3.3.1-1: SINR and throughput degradation for Macro aggressor Indoor victim

| Source | Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | UL | 50DL/50UL | UL |
| Huawei (1905522) | 5% | -0.06 | -0.02 | -1.25 | -0.42 |
|  | 50% | 0.04 | -0.02 | 0.56 | -0.40 |
|  | 95% | 0.1 | -0.09 | 0.81 | -0.74 |
| Nokia (1907604) | 5% | -0.1 | -0.1 | 9 | 7.1 |
|  | 50% | -0.1 | 0.1 | 0.9 | 1.2 |
|  | 95% | 0.2 | 1.5 | 1.3 | 1.7 |
| Qualcomm (1906703) | 5% | - | 0 | - | 0 |
|  | 50% | - | 0.07 | - | 0.76 |
|  | 95% | - | 1.02 | - | 3.45 |

#### 5.3.3.4	Scenario 4: 4 GHz Macro → Indoor (UL)

##### 5.3.3.4.1	Results

Table 5.3.3.4.1-1: SINR and throughput degradation for Macro aggressor Indoor victim

| Source | Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | DL | 50DL/50UL | DL |
| Huawei (1905522) | 5% | -0.06 | 0.01 | -1.01 | 0.34 |
|  | 50% | -0.04 | 0.03 | -0.64 | 0.33 |
|  | 95% | 0.18 | 0.28 | 1.55 | 2.43 |
| Nokia (1907604) | 5% | -0.6 | -2.2 | -0.9 | -19 |
|  | 50% | -1 | -2 | -4.2 | -9.5 |
|  | 95% | -0.6 | -2.1 | -0.1 | -11.1 |

#### 5.3.3.5	Scenario 5: 4 GHz Indoor → Macro (DL)

##### 5.3.3.5.1	Results

Table 5.3.3.5.1-1: SINR and throughput degradation for Indoor aggressor Macro victim

| Source | Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | UL | 50DL/50UL | UL |
| Huawei (1905522) | 5% | 0.04 | 0.09 | 0.71 | 1.52 |
|  | 50% | -0.42 | 0 | -2.90 | -0.02 |
|  | 95% | -0.03 | -0.11 | -0.12 | -0.38 |
| LGE (1907601) | 5% | - | -0.73 | - | -14.31 |
|  | 50% | - | 0.06 | - | 0.32 |
|  | 95% | - | 0.81 | - | 2.07 |
| Nokia (1907604) | 5% | 5.5 | -0.1 | 38.2 | 31.8 |
|  | 50% | 1.2 | -0.4 | 7.4 | -5.9 |
|  | 95% | 0.7 | -0.6 | 4 | 4.9 |

Table 5.3.3.5.1-2: SINR and throughput degradation for Indoor aggressor Macro victim (24 dBm TX power of a local BS )

| Source | Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | UL | 50DL/50UL | UL |
| LGE (1907601) | 5% | - | -0.83 | - | -16.3 |
|  | 50% | - | -0.13 | - | -0.69 |
|  | 95% | - | 0.84 | - | 2.14 |

#### 5.3.3.6	Scenario 6: 4 GHz Indoor → Macro (UL)

##### 5.3.3.6.1	Results

Table 5.3.3.6.1-1: SINR and throughput degradation for Indoor aggressor Macro victim

| Source | Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | DL | 50DL/50UL | DL |
| Huawei (1905522) | 5% | -0.07 | -0.27 | -1.33 | -5.13 |
|  | 50% | 0.2 | -0.03 | 1.07 | -0.18 |
|  | 95% | 0 | -0.01 | 0.00 | 0.00 |
| LGE (1907601) | 5% | - | -2.31 | - | -70.18 |
|  | 50% | - | -1.44 | - | -14.28 |
|  | 95% | - | -0.05 | - | -0.22 |
| Nokia (1907604) | 5% | 0 | 0 | 7.2 | 20.9 |
|  | 50% | 0.1 | 0 | 17.1 | 15 |
|  | 95% | 0.1 | 0.1 | -0.8 | 0 |

Table 5.3.3.6.1-2: SINR and throughput degradation for Indoor aggressor Macro victim (24 dBm TX power of a local BS )

| Source | Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | DL | 50DL/50UL | DL |
| LGE (1907601) | 5% | - | -2.54 | - | -79.15 |
|  | 50% | - | -1.24 | - | -12.03 |
|  | 95% | - | -0.04 | - | -0.2 |

#### 5.3.3.7	Scenario 7: 4 GHz Indoor → Indoor (DL)

##### 5.3.3.7.1	Results

Table 5.3.3.7.1-1: SINR and throughput degradation for Indoor aggressor Indoor victim

| Source | Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | UL | 50DL/50UL | UL |
| Huawei (1905522) | 5% | 0.13 | 0 | 2.51 | -0.11 |
|  | 50% | 0.03 | -0.01 | 0.51 | -0.10 |
|  | 95% | 0.06 | 0.01 | 0.50 | 0.06 |
| LGE (1907601) | 5% | - | -0.08 | - | -1.4 |
|  | 50% | - | -0.11 | - | -1.16 |
|  | 95% | - | -0.43 | - | -1.49 |
| Ericsson (1906099) | 5% | <1 | <1 | <1 | <1 |
|  | 50% | <1 | <1 | <1 | <1 |
|  | 95% | <1 | <1 | <1 | <1 |
| Nokia (1907604) | 5% | 0.2 | 0 | 0.6 | -0.5 |
|  | 50% | -0.2 | -0.5 | -0.8 | -0.8 |
|  | 95% | -0.2 | -2.8 | -1.8 | -1.2 |

Table 5.3.3.7.1-2: SINR and throughput degradation for Indoor aggressor Indoor victim 
(24 dBm TX power of a local BS )

| Source | Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | UL | 50DL/50UL | UL |
| LGE (1907601) | 5% | - | -0.02 | - | -0.33 |
|  | 50% | - | -0.02 | - | -0.2 |
|  | 95% | - | 0.75 | - | 2.62 |
| Ericsson (1906099) | 5% | <1 | <1 | <1 | <1 |
|  | 50% | <1 | <1 | <1 | <1 |
|  | 95% | <1 | <1 | <1 | <1 |
| Qualcomm (1906703) | 5% | - | -0.01 | - | -0.17 |
|  | 50% | - | -0.07 | - | -0.71 |
|  | 95% | - | 1.15 | - | 0.26 |

#### 5.3.3.8	Scenario 8: 4 GHz Indoor → Indoor (UL)

##### 5.3.3.8.1	Results

Table 5.3.3.8.1-1: SINR and throughput degradation for Indoor aggressor Indoor victim (30 dBm TX power of a local BS )

| Source | Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | DL | 50DL/50UL | DL |
| Huawei (1905522) | 5% | 0.06 | 0.01 | 0.95 | 0.20 |
|  | 50% | 0.19 | 0.03 | 2.94 | 0.53 |
|  | 95% | -0.43 | 0.12 | -3.90 | 1.08 |
| LGE (1907601) | 5% | - | -0.04 | - | -0.76 |
|  | 50% | - | -0.19 | - | -2.06 |
|  | 95% | - | -0.28 | - | -1.39 |
| Ericsson (1906099) | 5% | <1 | <1 | -5.5 | -8.1 |
|  | 50% | <1 | <1 | -4.1 | -8.1 |
|  | 95% | -1 | -1.1 | -3 | -7 |
| Nokia (1907604) | 5% | -0.2 | 0.3 | -8.3 | -3.2 |
|  | 50% | 0.5 | 0.8 | -1.9 | -2.8 |
|  | 95% | 0.3 | 0.3 | 3.5 | 1.5 |

Table 5.3.3.8.1-2: SINR and throughput degradation for Indoor aggressor Indoor victim (24 dBm TX power of a local BS )

| Source | Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | DL | 50DL/50UL | DL |
| LGE (1907601) | 5% | - | -0.05 | - | -0.97 |
|  | 50% | - | -0.24 | - | -2.64 |
|  | 95% | - | -0.52 | - | -2.63 |
| Ericsson (1906099) | 5% | <1 | <1 | -1 | -1 |
|  | 50% | <1 | <1 | <1 | -1 |
|  | 95% | <1 | <1 | 1 | <1 |

### 5.3.4	FR2

#### 5.3.4.1	Scenario 9: 30 GHz Macro → Macro (DL)

#### 5.3.4.1.1	Results

Table 5.3.4.1.1-1: SINR and throughput degradation for Macro aggressor Macro victim

| Source | Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | UL | 50DL/50UL | UL |
| Huawei (1905523) | 5% | -0.36 | -0.4 | -3.75 | -4.07 |
|  | 50% | 0 | -0.68 | 0.00 | 0.00 |
|  | 95% | -0.22 | -0.82 | 0.00 | 0.00 |
| LGE (1907062) | 5% | - | 5.5 | - | 37.08 |
|  | 50% | - | 2.9 | - | 9.5 |
|  | 95% | - | 2.19 | - | 4.96 |
| Ericsson (196098) | 5% | <1 | <1 | <1 | <1 |
|  | 50% | <1 | <1 | <1 | <1 |
|  | 95% | <1 | <1 | <1 | <1 |
| Qualcomm (1906703) | 5% | - | -0.38 | - | -1.95 |
|  | 50% | - | -0.22 | - | 0 |
|  | 95% | - | -0.32 | - | 0 |
| Nokia (1907604) | 5% | -0.7 | -1.5 | -16.8 | -35.0 |
|  | 50% | -0.3 | -0.7 | -1.2 | -7.0 |
|  | 95% | -0.1 | -0.4 | 0.7 | -1.2 |

#### 5.3.4.2	Scenario 10: 30 GHz Macro → Macro (UL)

##### 5.3.4.2.1	Results

Table 5.3.4.2.1-1: SINR and throughput degradation for Macro aggressor Macro victim

| Source | Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | DL | 50DL/50UL | DL |
| Huawei (1905523) | 5% | 1.209 | 0.4108 | 1.04 | 0.66 |
|  | 50% | 0.0604 | 0.126 | 0.37 | 0.78 |
|  | 95% | 0.0011 | 0.0023 | 0.01 | 0.02 |
| LGE (1907062) | 5% | - | 0.56 | - | 10.04 |
|  | 50% | - | 0.79 | - | 5.18 |
|  | 95% | - | 0.01 | - | 0.08 |
| Ericsson (196098) (NOTE 1) | 5% | 0.35 | 0.64 | <1 | <1 |
|  | 50% | 0.74 | 1.55 | 8 | 14.5 |
|  | 95% | 0.48 | 1.04 | 4.11 | 8.53 |
| Nokia (1907604) | 5% | 0.2 | 0.9 | -0.7 | 4.2 |
|  | 50% | 0.1 | 0.5 | 0.4 | 0.5 |
|  | 95% | 0.1 | 0.2 | 1.9 | -0.9 |
| NOTE 1: Further Ericsson results in this scenario with reduced cell size and grid shift indicated throughput losses |  |  |  |  |  |

#### 5.3.4.3	Scenario 11: 30 GHz Micro → Micro (DL)

##### 5.3.4.3.1	Results

Table 5.3.4.3.1-1: SINR and throughput degradation for Micro aggressor Micro victim

| Source | Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | UL | 50DL/50UL | UL |
| Huawei (1905523) | 5% | -0.17 | 0.03 | -3.50 | 0.50 |
|  | 50% | -0.38 | -0.26 | -1.81 | -1.24 |
|  | 95% | -1.04 | -0.88 | 0.00 | 0.00 |
| Ericsson (1906101) | 5% | <1 | <1 | <1 | <1 |
|  | 50% | <1 | <1 | <1 | <1 |
|  | 95% | <1 | <1 | <1 | <1 |
| Qualcomm (1906703) | 5% | - | 0.06 | - | 0.33 |
|  | 50% | - | -0.44 | - | 0 |
|  | 95% | - | -0.37 | - | 0 |

#### 5.3.4.4	Scenario 12: 30 GHz Micro → Micro (UL)

##### 5.3.4.4.1	Results

Table 5.3.4.4.1-1: SINR and throughput degradation for Micro aggressor Micro victim

| Source | Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | DL | 50DL/50UL | DL |
| Huawei (1905523) | 5% | 1.387 | 4.8167 | NA | NA |
|  | 50% | 1.3956 | 3.5008 | 10.24 | 25.23 |
|  | 95% | 0.0024 | 0.0068 | 0.02 | 0.04 |
| Ericsson (1906101) | 5% | <1 | <1 | <1 | <1 |
|  | 50% | <1 | <1 | <1 | <1 |
|  | 95% | <1 | <1 | <1 | <1 |

#### 5.3.4.5	Scenario 13: 30 GHz Indoor → Macro (DL)

##### 5.3.4.5.1	Results

Table 5.3.4.5.1-1: SINR and throughput degradation for Micro aggressor Micro victim

| Source | Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | UL | 50DL/50UL | UL |
| Huawei (1905523) | 5% | -0.46 | 0.05 | -2.36 | 0.25 |
|  | 50% | -0.32 | 0.01 | 0.00 | 0.00 |
|  | 95% | -0.25 | 0.04 | 0.00 | 0.00 |
| LGE (1907062) | 5% | - | 0.14 | - | 0.92 |
|  | 50% | - | 0.25 | - | 0.81 |
|  | 95% | - | 0.29 | - | 0.64 |

#### 5.3.4.6	Scenario 14: 30 GHz Indoor → Macro (UL)

##### 5.3.4.6.1	Results

Table 5.3.4.6.1-1: SINR and throughput degradation for Indoor aggressor Macro victim

| Source | Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | DL | 50DL/50UL | DL |
| Huawei (1905523) | 5% | -0.51 | -0.31 | -4.28 | -2.64 |
|  | 50% | -0.01 | -0.02 | -0.07 | -0.12 |
|  | 95% | 0 | 0 | 0.00 | 0.00 |
| LGE (1907062) | 5% | - | -1.03 | - | -20.33 |
|  | 50% | - | -0.19 | - | -1.23 |
|  | 95% | - | 0 | - | -0.03 |

#### 5.3.4.7	Scenario 15: 30 GHz Indoor → Indoor (DL)

##### 5.3.4.7.1	Results

Table 5.3.4.7.1-1: SINR and throughput degradation for Indoor aggressor Indoor victim

| Source | Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | UL | 50DL/50UL | UL |
| Huawei (1905523) | 5% | -2.44 | 0.61 | NA | NA |
|  | 50% | 0.02 | 0.84 | 0.06 | 5.22 |
|  | 95% | -0.82 | 1 | 0.00 | 0.00 |
| LGE (1907062) | 5% | - | 1.3 | - | 19.72 |
|  | 50% | - | 0.46 | - | 4.7 |
|  | 95% | - | 0.17 | - | 1.01 |
| Ericsson (1906100) | 5% | <1 | <1 | <1 | <1 |
|  | 50% | <1 | <1 | <1 | <1 |
|  | 95% | <1 | <1 | <1 | <1 |
| Nokia (1907604) | 5% | 0.1 | 0.3 | 3.7 | 26.7 |
|  | 50% | -0.1 | 0.4 | 6.3 | 9.2 |
|  | 95% | -0.1 | -0.1 | -7.5 | 0.4 |
| Qualcomm (1906703) | 5% | - | 0.01 | - | 0.08 |
|  | 50% | - | -0.06 | - | -0.21 |
|  | 95% | - | 0.10 | - | 0 |

Table 5.3.4.7.1-2: SINR and throughput degradation for Indoor aggressor Indoor victim (low traffic)

| Source | Observation Point | Victim DL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | UL | 50DL/50UL | UL |
| LGE (1907062) | 5% | - | 0.16 | - | 2.61 |
|  | 50% | - | -0.02 | - | 0.19 |
|  | 95% | - | -0.09 | - | 0.55 |

#### 5.3.4.8 Scenario 16: 30 GHz Indoor → Indoor (UL)

##### 5.3.4.8.1	Results

Table 5.3.4.8.1-1: SINR and throughput degradation for Indoor aggressor Indoor victim

| Source | Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | DL | 50DL/50UL | DL |
| Huawei (1905523) | 5% | -1.5532 | 0.3189 | NA | NA |
|  | 50% | -0.2514 | 0.0858 | -2.19 | 0.75 |
|  | 95% | -0.0048 | -0.0026 | -0.03 | -0.02 |
| LGE (1907062) | 5% | - | -0.13 | - | -2.17 |
|  | 50% | - | -0.11 | - | -1.1 |
|  | 95% | - | -0.09 | - | -0.55 |
| Ericsson (1906100) | 5% | 1,1 | 2,6 | 12 | 19 |
|  | 50% | 1,6 | 4,1 | 17 | 32 |
|  | 95% | 1,1 | 2,5 | 11 | 23 |
| Nokia (1907604) | 5% | 1.7 | 3 | 8.8 | 49.4 |
|  | 50% | -0.1 | 0.8 | -0.7 | 9.9 |
|  | 95% | 0 | -0.5 | 1.5 | -1.1 |

Table 5.3.4.8.1-2: SINR and throughput degradation for Indoor aggressor Indoor victim (low traffic)

| Source | Observation Point | Victim UL |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SNR degradation (dB) |  | Throughput degradation (%) |  |
|  |  | 50DL/50UL | DL | 50DL/50UL | DL |
| LGE (1907062) | 5% | - | -0.12 | - | -1.98 |
|  | 50% | - | -0.08 | - | -0.78 |
|  | 95% | - | -0.04 | - | -0.27 |

# 6	Summary and recommendations

## 6.1	Zero grid shift

As discussed in Annex B, for zero grid shift, RX blocking (and hence zero UL throughput) occurs at the victim base station during subframes in which a co-located aggressor BS transmits in all scenarios.

## 6.2	Summary of results for 100% grid shift

### 6.2.1	FR1

For the evaluated adjacent channel for FR1 scenarios, the following observations have been made for Cross link interference (CLI) based on different traffic conditions (full buffer and low traffic mode) and different BS Tx power levels without any RF requirement change or interference mitigation:

#### 6.2.1.1	Macro-to-Macro scenario

-	For BS to BS interference, performance degradation was observed in adjacent channel.

-	No performance degradation relating to ACLR/ACS due to UE to UE interference was observed in adjacent channel.

#### 6.2.1.2	Indoor-to-Macro scenario

-	For BS to BS interference and UE to UE interference, no performance degradation relating to ACLR/ACS was observed in adjacent channel.

#### 6.2.1.3	Indoor-to-Indoor scenario

-	For BS to BS interference, no performance degradation was observed in adjacent channel based on both full buffer and low traffic mode provided that the BS and UE have similar power.

-	When higher BS transmission power is assumed, some performance degradation was observed for BS to BS interference.

-	For UE to UE interference, no performance degradation relating to ACLR/ACS was observed in adjacent channel based on both full buffer and low traffic mode.

#### 6.2.1.4	Macro-to-Indoor scenario

-	For BS to BS interference, no performance degradation was observed in adjacent channel.

-	For UE to UE interference, performance degradation was observed in adjacent channel by some companies.

It is noted that there may be a possibility of UE blocking occurring but this was not investigated as the studies in this WI are focused on ACLR/ACS co-existence impact.

### 6.2.2	FR2

For the evaluated adjacent channel for FR2 scenarios, the following observations have been made for Cross link interference (CLI) based on different traffic conditions (full buffer and low traffic mode) and different BS Tx power levels without any RF requirement change or interference mitigation:

#### 6.2.2.1	Macro-to-Macro scenario

-	For BS to BS interference, some performance degradation was observed in adjacent channel with 100% grid shift. The extent of the observed degradation varied from minor to significant between companies. The degradations increase with lower grid shift and decrease with lower output power.

-	For UE to UE interference, no performance degradation relating to ACLR/ACS was observed in adjacent channel.

#### 6.2.2.2	Indoor-to-Macro scenario

-	For BS to BS interference and UE to UE interference, no performance degradation was observed in adjacent channel.

#### 6.2.2.3	Indoor-to-Indoor scenario

-	For BS to BS interference, some results showed no performance degradation in the adjacent channel, and other results showed performance degradation. The performance degradation has some dependency on how power control is operated.

-	For UE to UE interference, no performance degradation relating to ACLR/ACS was observed in adjacent channel based on both full buffer and low traffic mode.

#### 6.2.2.4	Micro-to-Micro scenario

-	Simulations were performed by 3 companies the results for BS-BS interference were contradictory as summarized below:

-	no performance degradation relating to ACLR/ACS was observed.

-	Significant BS to BS interference was observed relating to ACLR/ACS observed.

-	All results showed that there was no UE to UE interference relating to ACLR/ACS.

-	Some deployments in which the micro is close to the victim may cause losses in a victim network, but this is lost in the statistics. Operators may need to take care about the distance between micros in close range.

It is noted that there may be a possibility of UE blocking occurring but this was not investigated as the studies in this WI are focused on ACLR/ACS co-existence impact.

## 6.3	Recommendations

### 6.3.1	FR1

#### 6.3.1.1	Macro-to-Macro scenario

-	Performance degradation was observed from the BS-to-BS interference for macro-macro scenario, which suggests that dynamic TDD should not be operated in such scenarios.

#### 6.3.1.2	Indoor scenarios (Indoor-to-Macro and Indoor-to-Indoor)

-	Performance degradations were not observed from operating dynamic TDD between an indoor network and a macro network and vice versa if there is sufficient isolation between them. No significant impact from operating dynamic TDD for the indoor scenario was observed as long as the BS and UE powers are similar and the operators co-ordinate so that basestation positions are offset. If higher BS power is assumed, some throughput degradation in the indoor scenario was observed due to BS to BS interference. The observations imply that dynamic TDD can be used in indoors as long as care is taken.

### 6.3.2	FR2

#### 6.3.2.1	Macro-to-Macro scenario

-	Some performance degradation was observed from the BS-to-BS interference for macro-macro scenario. The differences in the simulation results imply that operating dynamic TDD in this scenario without impact to neighbor network may be deployment dependent and requires at least careful planning and collaboration between operators to avoid performance impact.

#### 6.3.2.2	Indoor scenarios (Indoor-to-Macro and Indoor-to-Indoor)

-	Performance degradations were not observed from operating dynamic TDD between an indoor network and a macro network if there is sufficient isolation between them. Results suggested that to avoid degradation, careful layout and parameterization are necessary for indoor to indoor scenario. Overall, the observations imply that dynamic TDD can be used indoors as long as care is taken.

#### 6.3.2.3	Micro-to-Micro scenario

-	For micro to micro, the differences in the simulation results imply that to avoid BS to BS interference, operators may need to consider the proximity of micro BS in the same area. Overall, the observations imply that dynamic TDD can be used in certain micro deployments as long as care is taken.

Annex A: 
Detailed simulation results for non-zero grid shift

# A.1	FR1

## A.1.1	Scenario 1: 4GHz Macro → Macro (DL)

### A.1.1.1	Ericsson

#### A.1.1.1.1	100% utilization

Figure A.1.1.1.1-1: DL SINR and throughput CDFs with 100% grid shift, 500m ISD

Figure A.1.1.1.1-2: DL throughput CDFs with 50% grid shift, 500m ISD

Figure A.1.1.1.1-3: DL SINR and throughput CDFs with 10% grid shift, 500m ISD

#### A.1.1.1.2	50% utilization

Figure A.1.1.1.2-1: DL SINR and throughput CDFs with 100% grid shift, 500m ISD

#### A.1.1.1.3	10% utilization

Figure A.1.1.1.3-1: DL SINR CDF with 100% grid shift, 500m ISD.

### A.1.1.2	Huawei

Figure A.1.1.2-1: CDF of DL SINR and throughput from Huawei

### A.1.1.3	LGE

Figure A.1.1.3-1: Macro-to-Macro SINR result (victim: DL)

### A.1.1.4	Qualcomm

Figure A.1.1.4-1: Comparison of SINR and throughput performance with ACI in UMa-to-UMa scenario

## A.1.2	Scenario 2: 4GHz Macro → Macro (UL)

### A.1.2.1	Ericsson

#### A.1.2.1.1	100% grid shift

##### A.1.2.1.1.1	100% utilization

Figure A.1.2.1.1.1-1: UL SINR and throughput CDFs with 100% grid shift, 500m ISD.

Figure A.1.2.1.1.1-2: UL SINR and throughput CDFs with 100% grid shift, 200m ISD

##### A.1.2.1.1.2	50% utilization

Figure A.1.2.1.1.2-1: UL SINR and throughput CDFs with 100% grid shift, 500m ISD

##### A.1.2.1.1.3	10% utilization

Figure A.1.2.1.1.3-1: UL SINR and throughput CDFs with 100% grid shift, 500m ISD.

#### A.1.2.1.2	Other grid shifts

Figure A.1.2.1.2-1: UL SINR and throughput CDFs with 50% grid shift, 500m ISD.

Figure A.1.2.1.2-2: UL SINR and throughput CDFs with 10% grid shift, 500m ISD.

### A.1.2.2	Huawei

Figure A.1.2.2-1: CDF of UL SINR and throughput from Huawei

### A.1.2.3	LGE

Figure A.1.2.3-1: Macro-to-Macro SINR result (victim: UL)

## A.1.3	Scenario 3: 4GHz Macro → Indoor (UL)

### A.1.3.1	Huawei

Figure A.1.3.1-1: CDF of UL SINR and throughput from Huawei

### A.1.3.2	Nokia

#### A.1.3.2.1	Full buffer

Figure A.1.3.2.1-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Indoor UL victim (full buffer)

#### A.1.3.2.2	FTP3 with 10% load

Figure A.1.3.2.2-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Indoor UL victim, 10% traffic

## A.1.4	Scenario 4: 4GHz Macro → Indoor (DL)

### A.1.4.1	Huawei

Figure A.1.4.1-1: CDF of DL SINR and throughput from Huawei

### A.1.4.2	Nokia

#### A.1.4.2.1	Full buffer

Figure A.1.4.2.1-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Indoor DL victim, full buffer traffic

#### A.1.4.2.2	FTP3 with 10% load

Figure A.1.4.2.2-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Indoor DL victim, 10% traffic

### A.1.4.3	Qualcomm

Figure A.1.4.3-1: Comparison of SINR and throughput performance with ACI in UMa-to-InH scenario

## A.1.5	Scenario 5: 4GHz Indoor → Macro (UL)

### A.1.5.1	Huawei

Figure A.1.5.1-1: CDF of UL SINR and throughput from Huawei

### A.1.5.2	LGE

Figure A.1.5.2-1: Indoor-to-Macro (victim: UL) SINR result (24 dBm Tx power)

Figure A.1.5.2-2: Indoor-to-Macro (victim: UL) SINR result (30 dBm Tx power)

### A.1.5.3	Nokia

#### A.1.5.3.1	Full buffer

Figure A.1.5.3.1-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Macro UL victim, full buffer traffic

#### A.1.5.3.2	FTP3 with 10% load

Figure A.1.5.3.2-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Macro UL victim, 10% traffic

## A.1.6	Scenario 6: 4GHz Indoor → Macro (DL)

### A.1.6.1	Huawei

Figure A.1.6.1-1: CDF of DL SINR and throughput from Huawei

### A.1.6.2	LGE

Figure A.1.6.2-1: Indoor-to-Macro (victim: DL) SINR result (24 dBm Tx power)

Figure A.1.6.2-2: Indoor-to-Macro (victim: DL) SINR result (30 dBm Tx power)

### A.1.6.3	Nokia

#### A.1.6.3.1	Full buffer

Figure A.1.6.3.1-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Macro DL victim, full buffer traffic

#### A.1.6.3.2	FTP3 with 10% load

Figure A.1.6.3.2-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Macro DL victim, 10% traffic

## A.1.7	Scenario 7: 4GHz Indoor → Indoor (UL)

### A.1.7.1	Ericsson

#### A.1.7.1.1	100% utilization

Figure A.1.7.1.1-1: CDF for the SINR and throughput for the UL victim (24 dBm BS transmission power)

Figure A.1.7.1.1-2: CDF for the SINR and throughput for the UL victim (30 dBm BS transmission power)

#### A.1.7.1.2	0% utilization

Figure A.1.7.1.2-1: CDF for the SINR and throughput for the UL victim

#### A.1.7.1.3	10% utilization

Figure A.1.7.1.3-1: CDF for the SINR and corresponding throughput for the UL victim.

### A.1.7.2	Huawei

Figure A.1.7.2-1: CDF of UL SINR and throughput from Huawei

### A.1.7.3	LGE

Figure A.1.7.3-1: Indoor-to-Indoor (victim: UL) SINR result (24 dBm Tx power)

Figure A.1.7.3-2: Indoor-to-Indoor (victim: UL) SINR result (30 dBm Tx power)

### A.1.7.4	Nokia

#### A.1.7.4.1	Full buffer

Figure A.1.7.4.1-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor UL victim, full buffer traffic

#### A.1.7.4.2	FTP3 with 10% load

Figure A.1.7.4.2-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor UL victim, 10% traffic

## A.1.8	Scenario 8: 4GHz Indoor → Indoor (DL)

### A.1.8.1	Ericsson

#### A.1.8.1.1	100% utilization

Figure A.1.8.1.1-1: CDFs for the SINR and throughput for the DL victim with linear and logarithmic scale 
(24 dBm BS transmission power)

Figure A.1.8.1.1-2: CDFs for the SINR and throughput for the DL victim with linear and logarithmic scale 
(30 dBm BS transmission power)

#### A.1.8.1.2	50% utilization

Figure A.1.8.1.2-1: CDF for the SINR and corresponding throughput for the DL victim.

#### A.1.8.1.3	10% utilization

Figure A.1.8.1.3-1: CDF for the SINR and corresponding throughput for the DL victim.

### A.1.8.2	Huawei

Figure A.1.8.2-1: CDF of DL SINR and throughput from Huawei

### A.1.8.3	LGE

Figure A.1.8.3-1: Indoor-to-Indoor (victim: DL) SINR result (24 dBm Tx power)

Figure A.1.8.3-2: Indoor-to-Indoor (victim: DL) SINR result (30 dBm Tx power)

### A.1.8.4	Nokia

#### A.1.8.4.1	Full buffer

Figure A.1.8.4.1-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor DL victim, full buffer traffic

#### A.1.8.4.2	FTP3 with 10% load

Figure A.1.8.4.2-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor DL victim, 10% traffic

### A.1.8.5	Qualcomm

Figure A.1.8.5-1: Comparison of SINR and throughput performance with ACI in InH-to-InH scenario (24 dBm Tx power)

# A.2	FR2

## A.2.1	Scenario 9: 30 GHz Macro → Macro (DL)

### A.2.1.1	Ericsson

Figure A.2.1.1-1: Downlink SINR and Throughput CDFs with 100% grid shift

### A.2.1.2	Huawei

Figure A.2.1.2-1: CDF of DL SINR and throughput from Huawei

### A.2.1.3	LGE

Figure A.2.1.3-1: Macro-to-Macro SINR result (victim: DL)

### A.2.1.4	Nokia

#### A.2.1.4.1	Full buffer

Figure A.2.1.4.1-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Macro DL victim, full buffer traffic

#### A.2.1.4.2	FTP3 with 10% load

Figure A.2.1.4.2-1: SINR (top) and throughput (bottom) degradation for Macro aggressor Macro DL victim, 10% traffic

### A.2.1.5	Qualcomm

Figure A.2.1.5-1: Comparison of SINR and throughput performance with ACI in UMa-to-UMa scenario

## A.2.2	Scenario 10: 30 GHz Macro → Macro (UL)

### A.2.2.1	Ericsson

#### A.2.2.1.1	100% grid shift

##### A.2.2.1.1.1	100% Utilization

Figure A.2.2.1.1.1-1: Uplink SINR and Throughput CDFs with 100% grid shift, 100% utilization

##### A.2.2.1.1.2	50% Utilization

Figure A.2.2.1.1.2-1: Uplink SINR and Throughput CDFs with 100% grid shift, 50% utilization

##### A.2.2.1.1.3	10% Utilization

Figure A.2.2.1.1.3-1: Uplink SINR and Throughput CDFs with 100% grid shift, 10% utilization

#### A.2.2.1.2	50% grid shift

##### A.2.2.1.2.1	100% Utilization

Figure A.2.2.1.2.1-1: Uplink SINR and Throughput CDFs with 50% grid shift, 100% utilization

##### A.2.2.1.2.2	50% Utilization

Figure A.2.2.1.2.2-1: Uplink SINR and Throughput CDFs with 50% grid shift, 50% utilization

##### A.2.2.1.2.3	10% Utilization

Figure A.2.2.1.2.3-1: Uplink SINR and Throughput CDFs with 50% grid shift, 10% utilization

### A.2.2.2	Huawei

Figure A.2.2.2-1: CDF of UL SINR and throughput from Huawei

### A.2.2.3	LGE

Figure A.2.2.3-1: Macro-to-Macro SINR result (victim: UL)

## A.2.3	Scenario 11: 30 GHz Micro → Micro (UL)

### A.2.3.1	Ericsson

Figure A.2.3.1-1: CDF for the SINR and throughput for the UL victim.

### A.2.3.2	Huawei

Figure A.2.3.2-1: CDF of UL SINR and throughput from Huawei

## A.2.4	Scenario 12: 30 GHz Micro → Micro (DL)

### A.2.4.1	Ericsson

Figure A.2.4.1-1: CDF for the SINR and throughput for the DL victim.

### A.2.4.2	Huawei

Figure A.2.4.2-1: CDF of DL SINR and throughput from Huawei

### A.2.4.3	Qualcomm

Figure A.2.4.3-1: Comparison of SINR and throughput performance with ACI in UMi-to-UMi scenario

## A.2.5	Scenario 13: 30 GHz Indoor → Macro (UL)

### A.2.5.1	Huawei

Figure A.2.5.1-1: CDF of UL SINR and throughput from Huawei

### A.2.5.2	LGE

Figure A.2.5.2-1: Indoor-to-Macro SINR result (victim: UL)

## A.2.6	Scenario 14: 30 GHz Indoor → Macro (DL)

### A.2.6.1	Huawei

Figure A.2.6.1-1: CDF of DL SINR and throughput from Huawei

### A.2.6.2	LGE

Figure A.2.6.2-1: Indoor-to-Macro SINR result (victim: DL)

## A.2.7	Scenario 15: 30 GHz Indoor → Indoor (UL)

### A.2.7.1	Ericsson

Figure A.2.7.1-1: CDF for the SINR and throughput for the UL victim.

Figure A.2.7.1-2: CDF for the SINR for the UL victim, without UL power control.

### A.2.7.2	Huawei

Figure A.2.7.2-1: CDF of UL SINR and throughput from Huawei

### A.2.7.3	LGE

Figure A.2.7.3-1: Indoor-to-Indoor (victim: UL) SINR result

### A.2.7.4	Nokia

#### A.2.7.4.1	Full buffer

Figure A.2.7.4.1-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor UL victim, full buffer traffic

#### A.2.7.4.2	FTP3 with 10% load

Figure A.2.7.4.2-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor UL victim, 10% traffic

## A.2.8	Scenario 16: 30 GHz Indoor → Indoor (DL)

### A.2.8.1	Ericsson

Figure A.2.8.1-1: CDF for the SINR and throughput for the DL victim.

### A.2.8.2	Huawei

Figure A.2.8.2-1: CDF of DL SINR and throughput from Huawei

### A.2.8.3	LGE

Figure A.2.8.3-1: Indoor-to-Indoor (victim: DL) SINR result

### A.2.8.4	Nokia

#### A.2.8.4.1	Full buffer

Figure A.2.8.4.1-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor DL victim, full buffer traffic

#### A.2.8.4.2	FTP3 with 10% load

Figure A.2.8.4.2-1: SINR (top) and throughput (bottom) degradation for Indoor aggressor Indoor DL victim, 10% traffic

### A.2.8.5	Qualcomm

Figure A.2.8.5-1: Comparison of SINR and throughput performance with ACI in InH-to-InH scenario

Annex B: 
Detailed analysis for zero grid shift

This annex provides analysis of the inter-BS interference effects when there is zero grid shift (i.e. co-located aggressor and victim base stations). The interference between the base stations may be calculated analytically by considering the aggressor TX power, and the isolation between co-located base stations.

FR1:

For FR1, a typical assumption for the isolation between co-located base stations is 30dB. The 30dB assumption is the basis of the transmitter intermodulation and co-location blocking requirements for the FR1 specifications. A typical TRP transmit power as assumed for this study is 24-49dB, dependent on scenario (see section 5.3.1).

The power arriving into the receiver of a co-located victim is the aggressor TX power – isolation = 24 to 49dBm – 30dB = -6 to 19dBm. The FR1 receiver blocking requirement is -43dBm, so the interference from the aggressor to the victim will block the RF receiver and prevent uplink reception at the victim.

FR2:

For FR2, during the development of the 38.104 RF requirements in RAN4, around 50 to 70dB isolation between co-located BS was assumed. The total radiated power assumed for an FR2 BS in this study is 23 to 33 dBm, depending on scenario (see section 5.3.2). The range of interference power levels arriving at the receiver of a co-located receiver would be equal to TX power – isolation = (23 to 33dBm) – (50 to 70dB) = -17 to -47dBm. The FR2 blocking requirement is equal to the reference sensitivity + 33dB. The highest FR2 blocking level is -50dBm (for other sensitivity levels, the FR2 blocking requirement will be lower). Thus, power levels in the range arising for co-location scenarios will lead to receiver blocking in all circumstances.

Annex C: 
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-04 | RAN4 #90BIS | R4-1905090 |  |  |  | TR skeleton for Co-existence study of Cross-Link Interference (CLI) | 0.0.1 |
|  |  |  |  |  |  | Approved TPs in RAN4#90BIS R4-1905091, “TP to TR 38.828: Background” R4-1905210, “TP on CLI scenarios and system level simulation assumption” R4-1905093, “TP to TR 38.828: Inter-Operator interference mechanisms for unsynchronized TDD” R4-1905094, “TP to TR 38.828: Inter operator interference for co-located BS” |  |
| 2019-05 | RAN4#91 | R4-1906044 |  |  |  | TR 38.828 v0.1.0 for Co-existence study of Cross-Link Interference (CLI) | 0.1.0 |
|  |  |  |  |  |  | Approved TPs in RAN4#91 R4-1907605, “TP on summary and recommendations of co-existence evaluation of CLI” R4-1907603, “TP on simulation results and AnnexA_Detailed simulation results for non zero grid shift” R4-1907600, “TP to TR 38.828: Addition of missing antenna configurations to simulation assumptions” R4-1907599, “TP to TR 38.828 – clean up” |  |
| 2019-05 | RAN4#91 | R4-1906094 |  |  |  | TR 38.828 v0.2.0 for Co-existence study of Cross-Link Interference (CLI) | 0.2.0 |
| 2019-06 | RAN#84 | RP-191357 |  |  |  | TR 38.828 1.0.0 is submitted for 1 step approval | 1.0.0 |
| 2019-06 | RAN#84 |  |  |  |  | Approved by plenary – Rel-15 spec under change control | 16.0.0 |