| 3GPP TR 22.850 V0.1.0 (2024-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on 3GPP AI/ML Consistency Alignment (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms and abbreviations	9

3.1	Terms	9

3.2	Abbreviations	9

4	Background	9

5	AI/ML related activities in all Working Groups	10

5.1 	General	10

5.2	AI/ML related activities in TSG SA & CT Working Groups	14

5.2.1	 AI/ML related terminology	14

5.2.1.1	 TSG SA WG2	14

5.2.1.2	 TSG SA WG5	15

5.2.1.3	 TSG SA WG6	15

5.2.2	AI/ML related activities	16

5.2.2.1	Rel-18 SA1 WID - AI/ML model transfer in 5GS (AIML_MT)	16

5.2.2.1.1	Description	16

5.2.2.1.2	Activities summary	16

5.2.2.2	Rel-19 SA1 SID - AI/ML Model Transfer Phase 2 (FS_AIML_MT_Ph2)	17

5.2.2.2.1	Description	17

5.2.2.2.2	Activities summary	17

5.2.2.3	Rel-19 SA1 WID - AI/ML Model Transfer Phase 2 (AIML_MT_Ph2)	17

5.2.2.3.1	Description	17

5.2.2.3.2	Activities summary	17

5.2.2.4	Rel-18 SA2 WID - Enablers for Network Automation for 5G – phase 3 (eNA_Ph3)	17

5.2.2.4.1	Description	17

5.2.2.4.2	Activities summary	18

5.2.2.5	Rel-18 SA2 WID - System Support for AI/ML-based Services (AIMLsys)	18

5.2.2.5.1	Description	18

5.2.2.5.2	Activities summary	18

5.2.2.6	Rel-19 SA2 SID - Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) (FS_AIML_CN)	18

5.2.2.6.1	Description	18

5.2.2.6.2	Activities summary	19

5.2.2.7	Rel-19 SA2 WID - Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) (AIML_CN)	19

5.2.2.7.1	Description	19

5.2.2.7.2	Activities summary	20

5.2.2.8	Rel-18 SA3 WID - Security aspects of enablers for Network Automation for 5G - phase 3 (eNA_SEC_PH3)	20

5.2.2.8.1	Description	20

5.2.2.8.2	Activities summary	20

5.2.2.9	Rel-19 SA3 SID - Security aspects of Core Network Enhanced Support for AIML (FS_AIML_CN_SEC)	20

5.2.2.9.1	Description	20

5.2.2.9.2	Activities summary	20

5.2.2.10	Rel-19 SA4 SID - Artificial Intelligence (AI) and Machine Learning (ML) for Media (FS_AI4Media)	21

5.2.2.10.1	Description	21

5.2.2.10.2	Activities summary	21

5.2.2.11	Rel-18 SA5 WID - AI/ML management (AIML_MGT)	21

5.2.2.11.1	Description	21

5.2.2.11.2	Activities summary	22

5.2.2.12	Rel-19 SA5 SID - AI/ML management - phase 2 (FS_AIML_MGT_Ph2)	22

5.2.2.12.1	Description	22

5.2.2.12.2	Activities summary	22

5.2.2.13	Rel-19 SA6 SID - Application layer support for AI/ML services (FS_AIMLAPP)	23

5.2.2.13.1	Description	23

5.2.2.13.2	Activities summary	23

5.2.2.14	Rel-19 SA6 WID - Application enablement for AI/ML services (AIML_App)	23

5.2.2.14.1	Description	23

5.2.2.14.2	Activities summary	24

5.2.2.15	Rel-19 CT4 WID - Protocol for AI Data Collection from UPF (FS_PAIDC-UPF)	24

5.2.2.15.1	Description	24

5.2.2.15.2	Activities summary	24

5.3	AI/ML related activities in TSG RAN Working Groups	24

5.3.1	 AI/ML related terminology	24

5.3.1.1	 TSG RAN WG1	24

5.3.1.2	 TSG RAN WG3	26

5.3.2	AI/ML related activities	26

5.3.2.1	Rel-19 RAN1/RAN4 WID - Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (NR_AIML_air)	26

5.3.2.1.1	Description	26

5.3.2.1.2	Activities summary	28

5.3.2.2	Rel-19 RAN2 SID - AIML for mobility in NR (FS_NR_AIML_Mob)	28

5.3.2.2.1	Description	28

5.3.2.2.2	Activities summary	29

5.3.2.3	Rel-18 RAN3 WID - Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (NR_AIML_NGRAN-Core)	29

5.3.2.3.1	Description	29

5.3.2.3.2	Activities summary	29

5.3.2.4	Rel-19 RAN3 SID - Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (FS_NR_AIML_NGRAN_enh)	30

5.3.2.4.1	Description	30

5.3.2.4.2	Activities summary	30

6	Analysis on AI/ML across 3GPP	30

6.1 General	30

6.2	AI/ML related terminology	30

6.2.0	Analysis on terminology consistency	30

6.2.1	 Analysis on ML model	30

6.2.1.2	 Analysis on ML model training	31

6.2.1.3	Analysis on ML model re-training	31

6.2.1.4	Analysis on ML model testing	32

6.2.1.5	 Analysis on ML model inference	32

6.2.1.6	Analysis on ML model activation & ML model de-activation	32

6.2.1.7	Analysis on ML model lifecycle	33

6.2.1.8	 Analysis on ML model lifecycle management	33

6.2.2	Analysis on Horizontal Federated Learning	33

6.2.3	Analysis on Vertical Federated Learning	33

6.2.4	Analysis on Decision vs Prediction vs Output	34

6.2.5	Analysis on ML vs AI vs AI/ML	34

6.3	AI/ML related features	34

6.3.X	Analysis on feature alignment #X <alignment title>	34

7	Overall Evaluation	34

8	Conclusions	34

Annex <A> (informative):	35

A.1	ML model life cycle management (LCM)	35

A.1.1	Observations and analyses: AI/ML LCM	36

A.2	ML model lifecycle management capabilities	36

A.2.1	Observations and analyses: ML model lifecycle management capabilities	37

A.3	AI/ML functionalities management scenarios	38

A.3.1	 Observations and analyses: AI/ML functionalities management  scenarios	40

Annex <B> (informative): Change history	40

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

This study will investigate ongoing AI/ML work in TSG CT, TSG RAN and TSG SA Working Groups and identify instances of any potential misalignment and/or inconsistencies and provide information on any potential outcome to the respective WGs to resolve any issues with appropriate SA-level co-ordination as necessary.

The study is led by TSG SA in close collaboration and inputs from TSG CT and TSG RAN.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 21.918: “Summary of Rel-18 Work Items”

[3]		3GPP TR 38.843: “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”.

[4]		3GPP TR 23.700-84: “Study on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) (Release 19)”.

[5]		3GPP TR 22.874 “Study on traffic characteristics and performance requirements for AI/ML model transfer in 5GS (Release 18)”.

[6]	3GPP TS 22.261, “Service requirements for the 5G system (Release 19)”.

[7]		3GPP TR 23.700-82: “Study on application layer support for AI/ML services (Release 19)”.

[8]	3GPP TS 23.288: “Architecture enhancements for 5G System (5GS) to support network data analytics services (Release 18)”.

[9]		3GPP TS 28.105: “Artificial Intelligence / Machine Learning (AI/ML) management (Release 18)”.

[10]		3GPP TR 38.843: “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”.

[11]	3GPP TS 38.300: “NR and NG-RAN Overall Description; Stage 2 (Release 18)”.

[12]		3GPP TR 26.927: “Study on Artificial Intelligence and Machine Learning in 5G media services (Release 19)”

[13]	3GPP TS 38.401: “Architecture description”

[14]	3GPP TS 38.420: “Xn general aspects and principles”

[15]	3GPP TS 38.423: “Xn application protocol (XnAP)”

[16]	3GPP TS 28.105: “Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management”

[17]	3GPP TR 37.817: “Study on enhancement for Data Collection for NR and EN-DC”

[18]	3GPP TR 28.908: “Study on Artificial Intelligence/Machine Learning (AI/ ML) management”

[19]	3GPP TR 28.858; AI/ML management – phase 2

[20]	3GPP TR 29.889: “Study on Protocol for AI Data Collection from UPF(Release 19)”.

3	Definitions of terms and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Background

In Rel-18 and Rel-19, most working groups in TSG SA, CT and RAN have already performed SIs and/or have WIs relating to the AI/ML topic. These activities address different usage scenarios and associated specific use cases exploiting AI/ML for the operation of the 3GPP System ranging from radio interface operations (e.g. beam management, positioning), NG-RAN operations (e.g. energy saving, load balancing), to network management & orchestration, media services, and application enablement aspects.

With the complexity of the 3GPP systems and its operations and that of AI/ML, it is vital that the use of AI/ML in the operation of the 3GPP system (incl. related AI/ML model LCM) for any given use case be bound to specific principles, guidelines, design criteria, and requirements to safeguard the operation of the 3GPP System. This includes the capability to, e.g. fallback to non-AI/ML operation (i.e., not relying on inference process) whenever necessary not to negatively affect the NW and E2E performance.

This requires, as a minimum, the introduction of a common set of definitions to prevent any inconsistencies in the definition and use of AI/ML LCM across 3GPP WGs, to identify any misalignments/inconsistencies, and to communicate such inconsistencies to WGs for better alignment within 3GPP across different AI/ML related initiatives.

NOTE: AI/ML models and associated algorithms are certainly implementation specific and therefore out of scope of this study.

Editor's Note: The study item does not impact ongoing studies and normative work for AI/ML across all SA/RAN/CT WGs for Rel-19.

5	AI/ML related activities in all Working Groups

5.1 	General

This clause will investigate and identify AI/ML related activities of all working groups of Rel-18 features and Rel-19 studies and work items in TSG CT, TSG RAN and TSG SA Working Groups, as per the list in table 5.1-1.

Table 5.1-1: List of Rel-18 and Rel-19 AI/ML activities in 3GPP working groups

| Unique ID | Release | Title | Acronym | Approved SID/WID | TRs/TSs |
| --- | --- | --- | --- | --- | --- |
| SA1 |  |  |  |  |  |
| 920030 | Rel-18 | Stage 1 of AMMT | AIML_MT | SP-220440 | TS 22.261 [6] |
| 950008 | Rel-19 | Study on AI/ML Model Transfer Phase2 | FS_AIML_MT_Ph2 | SP-220439 | TR 22.876 [x] |
| 1000030 | Rel-19 | AI/ML Model Transfer Phase 2 | AIML_MT_Ph2 | SP-230514 | TS 22.261 [6] |
| SA2 |  |  |  |  |  |
| 980019 | Rel-18 | System Support for AI/ML-based Services | AIMLsys | SP-231278 | TS 23.501 [x] TS 23.502 [x] TS 23.503 [x] TS 23.288 [8] |
| 980020 | Rel-18 | Enablers for Network Automation for 5G – phase 3 | eNA_Ph3 | SP-230110 | TS 23.501 [x] TS 23.502 [x] TS 23.503 [x] TS 23.288 [8] |
| 1020068 | Rel-19 | Study on Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) | FS_AIML_CN | SP-231800 | TR 23.700-84 [4] |
| 1040033 | Rel-19 | Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) | AIML_CN | SP-240991 | TS 23.288 [8] TS 23.501 [x] TS 23.502 [x] TS 23.503 [x] TS 23.273 [x] |
| SA3 |  |  |  |  |  |
| 990042 | Rel-18 | Security aspects of enablers for Network Automation for 5G - phase 3 | eNA_Ph3_SEC Rel-18 | SP-230155 | TS 33.501 [x] |
| 1030035 | Rel-19 | Study on security aspects of Core Network Enhanced Support for AIML | FS_AIML_CN_SEC | SP-240509 | TS 33.784 [x] |
| SA4 |  |  |  |  |  |
| 950011 | Rel-19 (started in Rel-18) | Feasibility Study on Artificial Intelligence (AI) and Machine Learning (ML) for Media | FS_AI4Media | SP-220328 | TR 26.927 [12] TR 26.847 [x] |
| SA5 |  |  |  |  |  |
| 990119 | Rel-18 | AI/ML management | AIML_MGT | SP-230335 | TS 28.105 [9] TS 28.552 [x] TS 32.425 [x] TS 28.554 [x] TS 28.541 [x] |
| 1020023 | Rel-18 | NEF Charging enhancement to support AI/ML in 5GS | AIMLsysNEF_CH | SP-231706 | TS 32.254 [x] TS 32.291 [x] |
| 1020007 | Rel-19 | Study on AI/ML management - phase 2 | FS_AIML_MGT_Ph2 | SP-240965 | TR 28.858 [19] |
| SA6 |  |  |  |  |  |
| 970036 | Rel-18 | Application Data Analytics Enablement Service | ADAES | SP-230275 | TS 23.436 [x] TS 23.434 [x] TS 23.558 [x] |
| 1010005 | Rel-19 | Study on application layer support for AI/ML services | FS_AIMLAPP | SP-231182 | TR 23.700-82 [7] |
| 1040075 | Rel-19 | Application enablement for AI/ML services | AIML_App | SP-241008 | TS 23.482 [x] TS 23.436 [x] TS 23.434 [x] TS 23.558 [x] |
| CT3 |  |  |  |  |  |
| 990008 | Rel-18 | CT3 aspects of System Support for AI/ML-based Services | AIMLsys | CP-230329 | TS 29.122 [x] TS 29.513 [x] TS 29.514 [x] TS 29.517 [x] TS 29.591 [x] TS 29.519 [x] TS 29.520 [x] TS 29.521 [x] TS 29.522 [x] |
| 990010 | Rel-18 | CT3 aspects of Enablers for Network Automation for 5G - phase 3 | eNA_Ph3 | CP-240079 | TS 29.508 [x] TS 29.122 [x] TS 29.513 [x] TS 29.517 [x] TS 29.519 [x] TS 29.520 [x] TS 29.522 [x] TS 29.523 [x] TS 29.525 [x] TS 29.551 [x] TS 29.552 [x] TS 29.574 [x] TS 29.575 [x] TS 29.576 [x] TS 29.591 [x] |
| CT4 |  |  |  |  |  |
| 990008 | Rel-18 | CT4 aspects of AIML | Rel-18 CT4 aspects of AI/ML | CP-230329 | TS 29.503 [x] TS 29.504 [x] TS 29.505 [x] TS 29.510 [x] TS 29.564 [x] |
| 990010 | Rel-18 | CT4 aspects of Enablers for Network Automation for 5G - phase 3 | eNA_Ph3 | CP-240079 | TS 29.503 [x] TS 29.510 [x] TS 29.518 [x] TS 29.536 [x] TS 29.564 [x] TS 29.571 [x] |
| 1040005 | Rel-19 | Study on Protocol for AI Data Collection from UPF | FS_PAIDC_UPF | CP-241025 | TR 29.889[20] |
| RAN1 |  |  |  |  |  |
| 1021093 | Rel-19 | Core part: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface | NR_AIML_airAir | RP-240774 | TS 38.843 [3] TS 38.300 [11] TS 38.214 [x] TS 38.215 [x] TS 38.331 [x] TS 38.305 [x] TS 37.355 [x] TS 38.455 [x] TS 38.133 [x] TS 38.133 [x] |
| RAN2 |  |  |  |  |  |
| 1020084 | Rel-19 | Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR | FS_NR_AIML_Mob | RP-240082 | TR 38.744 [x] |
| RAN3 |  |  |  |  |  |
| 941010 | Rel-18 | Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN | NR_AIML_NGRAN-Core | RP-233441 | TS 38.300 [11] TS 38.401 [13] TS 38.420 [14] TS 38.423 [15] |
| 1020083 | Rel-19 | Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN | FS_NR_AIML_NGRAN_enh | RP-240323 | TR 38.743 [x] |
| RAN4 |  |  |  |  |  |
| 1022093 | Rel-19 | Perf. part: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface | NR_AIML_air-Perf | RP-240774 | TS 38.133 [x] |

Editors Note: further review of this table is needed to ensure completeness and identify relationships between different activities.

5.2	AI/ML related activities in TSG SA & CT Working Groups

5.2.1		AI/ML related terminology

5.2.1.1		TSG SA WG2

The following definitions are provided in clause 3 of 3GPP TS 23.288 [8]:

- Analytics Accuracy Information: Represent a performance measure of an analytics ID provided by an NWDAF containing AnLF, which is composed of the number of correct predictions of the analytics ID out of all predictions and the corresponding number of samples.

- ML Model Accuracy Information: Represent a performance measure of a ML Model provided by an NWDAF containing MTLF, which is composed of the number of correct predictions by the ML Model out of all predictions and the corresponding number of samples.

- Analytics Feedback Information: Indicates that the consumer NF has taken action(s) influenced by the previously provided analytics, which may or may not affect the ground truth data.

The following definitions are provided in clause 5 of 3GPP TS 23.288 [8]:

- Analytics logical function (AnLF): A logical function in NWDAF, which performs inference, derives analytics information (i.e. derives statistics and/or predictions based on Analytics Consumer request) and exposes analytics service i.e. Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo.

- Model Training logical function (MTLF): A logical function in NWDAF, which trains Machine Learning (ML) models and exposes new training services (e.g. providing trained ML Model) as defined in clause 7.5 and clause 7.6.

The following definitions are provided in clause 3 of 3GPP TR 23.700-84 [4]:

- Horizontal Federated Learning (HFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs).

- Vertical Federated Learning (VFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs).

- Label: A label is the training objective in supervised machine learning.

- VFL Server: An NWDAF or AF that integrates local training results, computes gradient information or loss information and send them to VFL client(s) for the local ML model update in VFL training process. It also coordinates the VFL training process by discovering and selecting VFL clients. In VFL inference process, The VFL server aggregates local inference results from VFL clients to generate the final VFL inference result and sends the final VFL inference result to the consumer. Only one VFL server may exist for each VFL process.

- VFL Client: An NWDAF or AF that holds the local dataset and performs local training and inference as asked by VFL Server. There can be multiple VFL Clients in VFL training and inference.

The following definitions are provided in clause 6.15 of 3GPP TR 23.700-84 [4]:

- VFL active participant: A VFL function that owns part of an ML model for an analytic ID and knows the labels for the ML model. The active participant is the main function for training an ML model for an analytic ID.

- VFL passive participant: A VFL function that owns part of an ML model for an analytic ID but does not know the labels of the ML model but is able to collect local data for one or more features.

5.2.1.2		TSG SA WG5

The following definitions are provided in clause 3 of 3GPP TS 28.105 [9]:

- ML model: a manageable representation of an ML model algorithm.

  - NOTE 1: an ML model algorithm is a mathematical algorithm through which running a set of input data can generate a set of inference output.

  - NOTE 2: ML model algorithm is proprietary and not in scope for standardization and therefore not treated in this specification.

  - NOTE 3: ML model may include metadata. Metadata may include e.g. information related to the trained model, and applicable runtime context.

- ML model training: a process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model.

- ML model initial training: a process of training an initial version of an ML model.

- ML model re-training: a process of training a previous version of an ML model and generate a new version.

  - NOTE 4: a new version of a trained ML model supports the same type of inference as the previous version of the ML model, i.e., the data type of inference input and data type of inference output remain unchanged between the two versions of the ML model, but parameter values might be different for the re-trained model.

- ML model joint training: a process of training a group of ML models.

- ML training function: a logical function with ML model training capabilities.

- AI/ML inference function: a logical function that employs an ML model to conduct inference.

- ML model testing: a process of testing an ML model using testing data.

- ML testing function: a logical function with ML model testing capabilities.

- AI/ML inference: a process of running a set of input data through a trained ML model to produce set of output data, such as predictions.

  - NOTE 5: the inference represents the process to realize the AI capabilities by utilizing a trained ML model and other AI enablers if needed, hence the AI/ML prefix is used when referring to inference as compared to training and testing.

- AI/ML inference function: a logical function that employs trained ML model(s) to conduct inference.

- AI/ML inference emulation: running the inference process to evaluate the performance of an ML model in an emulation environment before deploying it into the target environment.

- ML model deployment: a process of making a trained ML model available for use in the target environment.

5.2.1.3		TSG SA WG6

The following definitions are provided in clause 3 of 3GPP TR 23.700-82 [7]:

- ML model: According to 3GPP TS 28.105 [9], mathematical algorithm that can be "trained" by data and human expert input as examples to replicate a decision an expert would make when provided that same information.

- ML model lifecycle: The lifecycle of an ML model includes data collection, data processing, model training, model verification, model, instantiation and deployment, model monitoring and termination of ML model components.

- ML model training: According to 3GPP TS 28.105 [9], ML model training includes capabilities of an ML training function or service to take data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss.

- ML model inference: According to 3GPP TS 28.105 [9], ML model training includes capabilities of an ML model inference function that employs an ML model and/or AI decision entity to conduct inference.

- AI/ML enablement: an application enablement framework consisting of one or more AI/ML enabler capabilities based on implementation. Such function can be deployed as (or within) an enablement layer server (e.g. SEAL or ADAES) or client.

- AI/ML client: an application layer entity (also referred as ML client) which is an AI/ML endpoint and performs client-side operations (e.g. related to the ML model lifecycle). Such AI/ML client can be a VAL client or AIML enabler client and may be configured e.g. to provide ML model training and inference locally e.g. at the VAL UE side.

- AI/ML server: an application layer entity which is an AI/ML endpoint and performs server-side operations (e.g. related to the ML model lifecycle). Such AI/ML server can be a VAL server or AIML enabler server.

- FL member: An FL member or participant is an entity which has a role in the FL process. An FL member can be an FL client performing ML model training, or an FL server performing aggregation/collaboration for the FL process. The FL member in this study is assumed to be either a functionality at the VAL UE or at the network/server side (AIML enablement server, VAL server).

- FL client: An FL member which locally trains the ML model as requested by the FL server.

- FL server: An FL member which generates global ML model by aggregating local model information from FL clients.

5.2.2	AI/ML related activities

5.2.2.1	Rel-18 SA1 WID - AI/ML model transfer in 5GS (AIML_MT)

5.2.2.1.1	Description

Editor's Note:	Description clause will be further clarified whether to be based on WI summaries or WID objectives.

The objective of this work item is to specify performance requirements (for end-to-end latency, experienced data rate, communication service availability) and service requirements (for AI/ML QoS management, AI/ML model /data distribution/transfer, network performance and resource utilization monitoring/prediction) for 5GS to support the following AI/ML operations for various applications (e.g. image/speech recognition, media editing/enhancements, robot control, automotive):

-	AI/ML operation splitting between AI/ML endpoints

-	AI/ML model/data distribution and sharing over 5G system

-	Distributed/Federated Learning over 5G system

5.2.2.1.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.2.2.2	Rel-19 SA1 SID - AI/ML Model Transfer Phase 2 (FS_AIML_MT_Ph2)

5.2.2.2.1	Description

The objective of this study is to explore new use cases and potential service and performance requirements to support efficient AI/ML operations using direct device connections. This includes:

-	Distributed AI training and inference based on direct device connections, such as traffic KPIs, various QoS, and functional requirements for sidelink transmission.

-	Considerations for charging and security aspects.

5.2.2.2.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.2.2.3	Rel-19 SA1 WID - AI/ML Model Transfer Phase 2 (AIML_MT_Ph2)

5.2.2.3.1	Description

The objective of this work item is to specify KPI and functional requirements for 5GS to support the AIML data transfer by leveraging direct device connection under 5G network control. These objectives were derived based on outcome of Rel-19 study in SA1 that relates to how the 5GS supports the transmissions of AI/ML-based services over the application layer. The study addressed use cases and potential performance requirements for 5G system support of application layer Artificial Intelligence (AI)/Machine Learning (ML) model distribution and transfer (download, upload, updates, etc.), and identified traffic characteristics of AI/ML model distribution, transfer and training for various applications, e.g. video/speech recognition, robot control, automotive, other verticals.

5.2.2.3.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.2.2.4	Rel-18 SA2 WID - Enablers for Network Automation for 5G – phase 3 (eNA_Ph3)

5.2.2.4.1	Description

The objective of this work item is to further enhance NWDAF, based on what has been specified in the previous releases to allow 5GS to support network automation. This work item focuses on architecture enhancement, new scenarios and the necessary inputs and outputs to the NWDAF based on the conclusions of the study in Rel-18. The work focuses on 10 key aspects as follows:

-	Improve correctness of NWDAF analytics

-	NWDAF-assisted application detection

-	Data and analytics exchange in roaming case

-	Enhancements on Data collection and Storage

-	Enhancements on trained ML Model sharing

-	NWDAF-assisted URSP

-	Enhancements on QoS Sustainability analytics

-	Supporting Federated Learning in 5GC

-	Enhancement of NWDAF with finer granularity of location information

-	Interactions with MDAS/MDAF

5.2.2.4.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.2.2.5	Rel-18 SA2 WID - System Support for AI/ML-based Services (AIMLsys)

5.2.2.5.1	Description

This work item implements the conclusions of the Rel-18 study on the 5GS architectural and functional extensions to enable 5GS to assist the Application AI/ML operations. The normative text will be defined based on the agreed conclusions on 6 key issues, including unfinished discussions on proposals on how to define the aspects that are left to be finalized during normative work, and ensuring consistency with other 5GS features. The agreed conclusions focus on the following aspects:

-	Monitoring of network resource utilization to support the Application AI/ML operations

-	Expsoure of 5GC information to authorized 3rd party for Application AI/ML operations

-	Enhancement of external parameter provisioning in 5GC to assist the Application AI/ML operations

-	Enhancement in 5GC to enable Application AI/ML traffic transport

-	Enhancement of QoS and Policy control to support Application AI/ML data transport over 5GS

-	5GS assistance to federated learning operation

5.2.2.5.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.2.2.6	Rel-19 SA2 SID - Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) (FS_AIML_CN)

5.2.2.6.1	Description

The aim of this study is to investigate and identify potential architectural and system-level enhancements to support AI/ML enhancements. Specifically, the objectives include:

- AI/ML Cross-Domain Coordination Aspects: Investigate enhancements to support AI-enabled RAN based on the conclusions of the RAN study in [a]. This task will discuss whether and how to support cross-domain (i.e., UE, RAN, 5GC, OAM, and AF) collaborative AI/ML mechanisms for the aspects described below.

  - Enhancements to LCS for AI/ML-Based Positioning: Examine whether and how to consider enhancements to LCS to support AI/ML-based positioning.

- Collaborative AI/ML Operations for Vertical Federated Learning (VFL): Determine potential enhancements needed to enable the 5G system to assist in collaborative AI/ML operations involving 5GC/NWDAF and/or AF for “Vertical Federated Learning (VFL).” This work will be based solely on and limited to the scope of justified use cases.

- Enhancements to Support NWDAF-Assisted Policy Control and Address Network Abnormal Behavior:

  - Investigate additional support needed to enhance 5GC NF operations (i.e., policy control and QoS) assisted by NWDAF. This task will first identify specific use cases to define the appropriate scope. It will analyze the impacts on NWDAF (e.g., the need to understand specific NF functionality) and the compatibility of new solutions with existing analytics to determine the necessity and benefits of new solutions.

  - Study the prediction, detection, prevention, and mitigation of network abnormal behaviors, such as signaling storms, with the assistance of NWDAF.

5.2.2.6.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.2.2.7	Rel-19 SA2 WID - Core Network Enhanced Support for Artificial Intelligence (AI)/Machine Learning (ML) (AIML_CN)

5.2.2.7.1	Description

The objective of this work item is to specify the following enhancements to 5GS as per the conclusions reached within the Rel-19 study for the following aspects:

- Enhancements to LCS to Support Direct AI/ML-Based Positioning

- LMF Enhancements: The LMF will be enhanced to perform location calculations based on an ML model. The triggers for data collection and model training within the LMF will be implementation-specific.

- MTLF and LMF Enhancements: Both the MTLF and the LMF will be enhanced to facilitate ML model training for AI/ML-based positioning.

- Procedure Development: Related procedures for data collection will be developed in coordination with RAN WGs.

- 5GC Support for Vertical Federated Learning

  - 5GC Enhancements: The 5GC will be enhanced to support vertical federated learning (VFL), a technique that does not involve exchanging or sharing local datasets or ML models, in the following scenarios:

-	VFL among NWDAFs within a single PLMN.

-	VFL between NWDAF(s) within a single PLMN and AF(s). NWDAF-Assisted Policy Control and QoS Enhancement

- NWDAF-Assisted Policy Control and QoS Enhancement

  - Assistance Information: Based on PCF requests, the NWDAF may provide assistance information to the PCF to aid in the determination and modification of QoS parameters.

- NWDAF Enhancements to Support Network Abnormal Behaviours Mitigation and Prevention

  - Signalling Storm Mitigation: The NWDAF will support signalling storm mitigation and prevention by providing analytics related to the detection and prediction of signalling storms caused by massive signalling from UEs and/or NFs.

5.2.2.7.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.2.2.8	Rel-18 SA3 WID - Security aspects of enablers for Network Automation for 5G - phase 3 (eNA_SEC_PH3)

5.2.2.8.1	Description

The main objective of this work is to produce normative specification based on the conclusions from Rel-18 study.  More specifically, the following objectives are expected to be specified:

-	Protection of data and analytics exchange in roaming case

-	Security for AI/ML model storage and sharing

-	Authorization of selection of participant NWDAF instances in the Federated Learning group

5.2.2.8.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.2.2.9	Rel-19 SA3 SID - Security aspects of Core Network Enhanced Support for AIML (FS_AIML_CN_SEC)

5.2.2.9.1	Description

The objectives of this study are the following:

- Security Aspects on Enhancements to LCS: Study security aspects on enhancements to LCS to support AI/ML-based positioning, considering the conclusions in [a] and [b].

- Security Aspects of Cross-Domain Vertical Federated Learning (VFL):

  - Authorization of VFL Group Members: Examine the authorization of members of the VFL group.

  - Security Aspects of Enhancements on SA2 Architecture: Investigate security aspects of enhancements on SA2 architecture to support VFL.

5.2.2.9.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.2.2.10	Rel-19 SA4 SID - Artificial Intelligence (AI) and Machine Learning (ML) for Media (FS_AI4Media)

5.2.2.10.1	Description

The primary objective of this study item is to identify relevant interoperability requirements and implementation constraints of AI/ML in 5G media services. The specific objectives include:

- Use Cases for Media-Based AI/ML Scenarios: List and describe the use cases for media-based AI/ML scenarios, based on those defined in [c].

- Media Service Architecture and Service Flows: Describe the media service architecture and relevant service flows for the scenarios. Identify the impacts on the architecture for each use case, including any potential gaps with existing 5G media service architectures. Also, describe the model operation configurations for each use case, including split AI/ML operations, and identify where certain AI/ML operations occur.

- Data Formats and Protocols: Identify and document the available data formats and suitable protocols for the exchange of different data components of various AI/ML models, such as model data, metadata, media data, and intermediate data necessary for such model operation configurations. Investigate the data traffic characteristics of these data components for delivery over the 5G system, including any needs and potentials for data rate reduction.

- Key Performance Indicators (KPIs): Identify and study key performance indicators for such scenarios, based on the initial considerations in [d]. Emphasize the use cases, model operation configurations, and data components identified in earlier objectives, focusing on objective performance metrics considering the identified KPIs.

- Normative Work and Collaboration: Identify potential areas for normative work as the next phase. Communicate and align with SA2 and other potential 3GPP working groups on relevant aspects related to the study.

5.2.2.10.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.2.2.11	Rel-18 SA5 WID - AI/ML management (AIML_MGT)

5.2.2.11.1	Description

The objective of this work is to specify the AI/ML management capabilities, including use cases, requirements and solutions for each phase of the AI/ML operational workflow for managing the AI/ML capabilities in 5GS (i.e., management and orchestration, 5GC and NG-RAN), including:

- Management capabilities for ML training phase, which includes control of producer-initiated ML training, data management for ML training, performance evaluation for ML training, ML entity validation, ML context management, ML entity capability discovery and ML entity testing

- Management capabilities for ML deployment phase, including management of ML entity loading

- Management capabilities for AI/ML inference phase.

To describe the deployment scenarios of the AI/ML management capabilities, with consideration of alignment with other relevant 3GPP WGs (e.g., RAN3, SA2) and ETSI ISG ZSM.

5.2.2.11.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.2.2.12	Rel-19 SA5 SID - AI/ML management - phase 2 (FS_AIML_MGT_Ph2)

5.2.2.12.1	Description

The objectives of this study item include:

- Continuation of AI/ML Studies: Continue the study on AI/ML emulation, AI/ML inference coordination, and ML knowledge transfer that are leftover from Rel-18.

- Management Aspects of AI/ML Functionalities Defined by Other 3GPP WGs:

  - AI/ML Model Transfer in 5GS: Study the management aspects (LCM, CM, and PM) of AI/ML model transfer in 5GS, as defined in SA1.

  - 5GS Support for AI/ML-Based Services: Investigate the management aspects of 5GS support for AI/ML-based services, as defined in SA2.

  - Support for AI/ML Services at Application Enablement Layer: Examine the management aspects of support for AI/ML services at the application enablement layer, as defined in SA6.

- Management Aspects of AI/ML Functionalities Defined by 3GPP SA5 WG:

  - Management Data Analytics (MDA) Phase 3: Study the management aspects (LCM, CM, and PM) of AI/ML functionalities defined by 3GPP SA5 WG, including MDA phase 3.

- AI/ML Management and Operation Capabilities: Investigate the AI/ML management and operation capabilities to support different types of AI/ML technologies needed for AI/ML in 5GS, such as Federated Learning, Reinforcement Learning, Online and Offline Training, Distributed Learning, and Generative AI.

- Sustainability Aspects of AI/ML:

  - Energy Consumption/Efficiency Impacts: Evaluate the energy consumption and efficiency impacts associated with AI/ML solutions for all operational phases (training, emulation, deployment, inference).

- Trustworthiness Aspects Related to AI/ML Functionalities in 5GS:

  - Concept of Trustworthiness: Further study the concept of trustworthiness for AI/ML in the context of OAM.

  - Data for Trustworthiness Indicators: Identify and analyze data (e.g., measurements, events) to support the calculation of trustworthiness indicators.

5.2.2.12.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.2.2.13	Rel-19 SA6 SID - Application layer support for AI/ML services (FS_AIMLAPP)

5.2.2.13.1	Description

The objective of this study is to enable support for AI/ML services at the application enablement layer. This includes the following:

- Analysis of Rel-18 and Rel-19 Requirements: Analyze the requirements in [d] related to AI/ML model distribution, transfer, and training. Identify key issues and develop corresponding architectural requirements at the application enablement layer, along with potential enhancements to the application layer architecture.

- Architectural and Functional Implications: Study the architectural and functional implications on existing SA6 application enablers (e.g., ADAES, other SEAL services, EDGEAPP) for supporting AI/ML lifecycle operations. This includes operations such as data collection, data preparation, training, inference, and federated learning for ML models used in ADAE layer analytics.

- Potential Solutions and APIs: Identify potential solutions, including information flows and developer-friendly application enablement APIs, to satisfy the architectural requirements and enhancements identified in the previous points.

- Impact on Deployments and Business Models: Investigate the possible impacts of application layer support for AI/ML services on different deployments and business models.

5.2.2.13.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.2.2.14	Rel-19 SA6 WID - Application enablement for AI/ML services (AIML_App)

5.2.2.14.1	Description

The objectives of this work include the following:

Develop Stage 2 normative technical specification for AIML enablement service as a new SEAL service, based on the key issues, architecture, solutions, and conclusions captured in [e]. The Stage 2 normative technical specification will include the following aspects:

-	Architecture requirements, deployment models and application architecture for AIML service enablement over 3GPP networks.

-	Procedures, information flows and APIs supporting concluded solutions related to AIML enablement capabilities for AI/ML, FL (e.g., Vertical FL among VAL UEs, Horizontal FL), Transfer Learning. Such capabilities include:

- Support AIML client management (e.g., registration, discovery) and selection.

- Support AIML service lifecycle management aspects (e.g., training, inference, data management).

- Support AIML operation split and ML model distribution operations.

- Support AIML operations in edge / distributed deployments.

-	Procedures information flows and APIs supporting concluded solutions for application layer support capabilities related to new ADAE analytics services. Such new analytics services include:

- DN Energy Analytics.

- Analytics for supporting FL.

Identify potential enhancements to other enablement frameworks (e.g. SEAL, EGDEAPP and CAPIF) based on the specified solutions for the above objectives.

5.2.2.14.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

### 5.2.2.15	Rel-19 CT4 WID - Protocol for AI Data Collection from UPF (FS_PAIDC-UPF)

5.2.2.15.1	Description

In Rel-18, the UPF offers services to the NEF, AF, SMF, NWDAF, DCCF, MFAF via the Nupf service based interface for data collecting in AI/ML related activities. In Rel-19, CT4 is studying “Protocol for AI Data Collection from UPF”, which aims at studying UPF data Collection for AI/ML and whether alternative protocols, or enhancements to the existing SBI protocol, are needed to optimize the AI/ML data collection while ensuring secure, scalable and reliable data transfers across the core network identifies.

5.2.2.15.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.3	AI/ML related activities in TSG RAN Working Groups

5.3.1		AI/ML related terminology

5.3.1.1		TSG RAN WG1

The following definitions are provided in clause 3 of 3GPP TR 38.843 [3]:

- AI/ML-enabled Feature: refers to a Feature where AI/ML may be used.

- AI/ML Model: A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs.

- AI/ML model delivery: A generic term referring to delivery of an AI/ML model from one entity to another entity in any manner. Note: An entity could mean a network node/function (e.g., gNB, LMF, etc.), UE, proprietary server, etc.

- AI/ML model ID:  A logical AI/ML model is identified by a Model ID. The Model ID, if needed, can be used in a Functionality (defined in functionality-based LCM) for LCM operations.

- AI/ML model Inference:  A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs.

- AI/ML model testing: A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model.

- AI/ML model training: A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference.

- AI/ML model transfer: Delivery of an AI/ML model over the air interface in a manner that is not transparent to 3GPP signalling, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model.

- AI/ML model validation: A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training.

- Data collection: A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference.

- Federated learning / federated training: A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples.

- Functionality identification: A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE. Note: Information regarding the AI/ML functionality may be shared during functionality identification. Where AI/ML functionality resides depends on the specific use cases and sub use cases.

- Management instruction: Information needed to ensure proper inference operation. This information may include selection/(de)activation/switching of AI/ML models or AI/ML functionalities, fallback to non-AI/ML operation, etc.

- Model activation: enable an AI/ML model for a specific AI/ML-enabled feature.

- Model deactivation: disable an AI/ML model for a specific AI/ML-enabled feature.

- Model download: Model transfer from the network to UE.

- Model identification: A process/method of identifying an AI/ML model for the common understanding between the NW and the UE. Note: The process/method of model identification may or may not be applicable. Note: Information regarding the AI/ML model may be shared during model identification.

- Model monitoring: A procedure that monitors the inference performance of the AI/ML model.

- Model parameter update: Process of updating the model parameters of a model.

- Model selection: The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation.

- Model switching: Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific AI/ML-enabled feature.

- Model update: Process of updating the model parameters and/or model structure of a model.

- Model upload: Model transfer from UE to the network.

- Network-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the network.

- Offline field data: The data collected from field and used for offline training of the AI/ML model.

- Offline training: An AI/ML training process where the model is trained based on collected dataset, and where the trained model is later used or delivered for inference. Note: This definition only serves as a guidance. There may be cases that may not exactly conform to this definition but could still be categorized as offline training by commonly accepted conventions.

- Online field data: The data collected from field and used for online training of the AI/ML model.

- Online training: An AI/ML training process where the model being used for inference) is (typically continuously) trained in (near) real-time with the arrival of new training samples.

- Reinforcement Learning (RL): A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a.  reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with.

- Semi-supervised learning: A process of training a model with a mix of labelled data and unlabelled data.

- Supervised learning: A process of training a model from input and its corresponding labels.

- Test encoder/decoder for TE: AI/ML model for UE encoder/gNB decoder implemented by TE.

- Two-sided (AI/ML) model: A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa.

- UE-side (AI/ML) model: An AI/ML Model whose inference is performed entirely at the UE.

- Unsupervised learning: A process of training a model without labelled data.

- Proprietary-format models: ML models of vendor-/device-specific proprietary format, from 3GPP perspective. They are not mutually recognizable across vendors and hide model design information from other vendors when shared. Note: An example is a device-specific binary executable format.

- Open-format models: ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective. They are mutually recognizable between vendors and do not hide model design information from other vendors when shared.

5.3.1.2		TSG RAN WG3

The following definitions are provided in clause 16.20 of 3GPP TS 38.300 [11]:

- AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [9].

- AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [9].

5.3.2	AI/ML related activities

5.3.2.1	Rel-19 RAN1/RAN4 WID - Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (NR_AIML_air)

5.3.2.1.1	Description

Editor's Note:	Description clause will be further clarified whether to be based on WI summaries or WID objectives.

The objective of this work is to provide specification support for the following aspects:

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]:

  - Specify necessary RAN4 core requirements for the above two use cases.

  - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24):

- CSI feedback enhancement [RAN1]:

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

  - For CSI prediction (UE-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the [a] (e.g., cell/site specific model could be considered to improve performance gain).

- Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1]

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]:

  - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection

  - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (clause 7.2.1.3.2 of [a]) study along with the implications and limitations of each of the methods

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

- Testability and interoperability [RAN4]:

  - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least:

    - Relation to legacy requirements

    - Performance monitoring and LCM aspects considering use-case specifics

    - Generalization aspects

    - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.)

    - UE processing capability and limitations

    - Post-deployment validation due to model change/drift

  - RAN5 aspects related to testability and interoperability to be addressed on a request basis

- For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models

  - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases

  - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

5.3.2.1.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.3.2.2	Rel-19 RAN2 SID - AIML for mobility in NR (FS_NR_AIML_Mob)

5.3.2.2.1	Description

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. The investigation is to evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

5.3.2.2.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.3.2.3	Rel-18 RAN3 WID - Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (NR_AIML_NGRAN-Core)

5.3.2.3.1	Description

The objective of this work is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization.

Support of AI/ML for NG-RAN, as a RAN internal function, is used to facilitate Artificial Intelligence (AI) and Machine Learning (ML) techniques in NG-RAN. The objective of AI/ML for NG-RAN is to improve network performance and user experience, through analysing the data collected and autonomously processed by the NG-RAN, which can yield further insights, e.g., for Network Energy Saving, Load Balancing, Mobility Optimization.

Support of AI/ML in NG-RAN requires inputs from neighbour NG-RAN nodes (e.g., predicted information, feedback information, measurements) and/or UEs (e.g., measurement results).

Signalling procedures used for the exchange of information to support AI/ML in NG-RAN are use case and data type agnostic, which means that the intended usage of the data exchanged via these procedures (e.g., input, output, feedback) is not indicated. The collection and reporting of information are configured through the Data Collection Reporting Initiation procedure, while the actual reporting is performed through the Data Collection Reporting procedure.

Support of AI/ML in NG-RAN does not apply to ng-eNB.

For the deployment of AI/ML in NG-RAN, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the NG-RAN node.

-	AI/ML Model Training and AI/ML Model Inference are both located in the NG-RAN node.

5.3.2.3.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

5.3.2.4	Rel-19 RAN3 SID - Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (FS_NR_AIML_NGRAN_enh)

5.3.2.4.1	Description

The objective of this study is to further investigate new AI/ML based use cases and identify enhancements to support AI/ML functionality, and further discussions on the Rel-18 leftovers. The detailed objectives of the study are listed as follows:

- Study two new AI/ML based use cases, i.e., Network Slicing and CCO, with existing NG-RAN interfaces and architecture (including non-split architecture and split architecture).

- Rel-18 leftovers as candidates for normative work, based on the Rel-18 principles, as follows:

  - Mobility optimization for NR-DC

  - Split architecture support for Rel-18 use cases based on the conclusions from Rel-18 WI

  - Energy Saving enhancements, e.g., Energy Cost Prediction

  - Continuous MDT collection targeting the same UE across RRC states

  - Multi-hop UE trajectory across gNBs.

5.3.2.4.2	Activities summary

Editor's note:	This clause describes high-level AI/ML activities e.g. LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring. Sub-clause(s) may be added to capture details.

6	Analysis on AI/ML across 3GPP

6.1 General

This clause will identify any potential misalignments and inconsistencies for AI/ML across 3GPP, based on Clause 5.

NOTE 1:  Any RAN related aspects are subject to early coordination and feedback from TSG RAN.

6.2	AI/ML related terminology

6.2.0	Analysis on terminology consistency

This clause identifies any potential misalignments and inconsistencies for AI/ML terminology across 3GPP, based on Clause 5.

6.2.1		Analysis on ML model

The term ‘ML model’ has been defined differently by SA5, SA6 and RAN1, as illustrated in Table 1.

Editor’s note: Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

| TSG (TS/TR) | ML model |
| --- | --- |
| 3GPP SA5 TS 28.105 [9] | A manageable representation of an ML model algorithm. NOTE 1: an ML model algorithm is a mathematical algorithm through which running a set of input data can generate a set of inference output. NOTE 2: ML model algorithm is proprietary and not in scope for standardization and therefore not treated in this specification. NOTE 3: ML model may include metadata. Metadata may include e.g. information related to the trained model, and applicable runtime context. |
| 3GPP SA6 TR 23.700-82 [7] | According to 3GPP TS 28.105 [9], mathematical algorithm that can be "trained" by data and human expert input as examples to replicate a decision an expert would make when provided that same information. |
| 3GPP RAN1 TR 38.843 [3] | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |

Table 1: Definition of ML model as defined across 3GPP WGs.

6.2.1.2		Analysis on ML model training

The term ‘ML model training’ has been defined differently by SA5, SA6 and RAN1, as illustrated in Table 2. RAN3 follows the definition of SA5.

Editor’s note: Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

| TSG (TS/TR) | ML model training |
| --- | --- |
| 3GPP SA5 TS 28.105 [9] | A process performed by an ML training function to take training data, run it through an ML model algorithm, derive the associated loss and adjust the parameterization of that ML model iteratively based on the computed loss and generate the trained ML model. |
| 3GPP SA6 TR 23.700-82 [7] | According to 3GPP TS 28.105 [9], ML model training includes capabilities of an ML training function or service to take data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss. |
| 3GPP RAN1 TR 38.843 [3] | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference. |
| 3GPP RAN3 TS 38.300 [11] | AI/ML Model Training follows the definition of the "ML model training" as specified in clause 3.1 of TS 28.105 [9]. |

Table 2: Definition of ML model training as defined across 3GPP WGs.

6.2.1.3	Analysis on ML model re-training

The term ‘ML model re-training’ has been defined differently by SA5 and RAN1, as illustrated in Table 3. RAN1 introduces two new terms, i.e., ML model parameter update and ML model update, which is nothing but ML model re-training.

Editor’s note: Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

| TSG (TS/TR) | ML model re-training / ML model parameter update / ML model update |
| --- | --- |
| 3GPP SA5 TS 28.105 [9] | ML model re-training: A process of training a previous version of an ML model and generate a new version. |
| 3GPP RAN1 TR 38.843 [3] | ML model parameter update: A process of updating the model parameters of a model. Model update: A process of updating the model parameters and/or model structure of a model |

Table 3: Definition of ML model re-training / ML model parameter update as defined across 3GPP WGs.

6.2.1.4	Analysis on ML model testing

The term ‘ML model testing’ has been defined differently by SA5 and RAN1, as illustrated in Table 4.

Editor’s note: Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

| TSG (TS/TR) | ML model testing |
| --- | --- |
| 3GPP SA5 TS 28.105 [9] | A process of testing an ML model using testing data. |
| 3GPP RAN1 TR 38.843 [3] | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |

Table 4: Definition of ML model testing as defined across 3GPP WGs.

6.2.1.5		Analysis on ML model inference

The term ‘ML model inference’ has been defined differently by SA5, SA6 and RAN1, as illustrated in Table 5. RAN3 follows the definition of SA5.

Editor’s note: Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

| TSG (TS/TR) | ML model inference |
| --- | --- |
| 3GPP SA5 TS 28.105 [9] | A process of running a set of input data through a trained ML model to produce set of output data, such as predictions. |
| 3GPP SA6 TR 23.700-82 [7] | According to 3GPP TS 28.105 [9], ML model training includes capabilities of an ML model inference function that employs an ML model and/or AI decision entity to conduct inference. |
| 3GPP RAN1 TR 38.843 [3] | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs. |
| 3GPP RAN3 TS 38.300 [11] | AI/ML Model Inference follows the definition of the "AI/ML inference" as defined in clause 3.1 of TS 28.105 [9]. |

Table 5: Definition of ML model inference as defined across 3GPP WGs.

6.2.1.6	Analysis on ML model activation & ML model de-activation

The term ‘ML model activation’ and ‘ML model deactivation’ have been defined by RAN1, as illustrated in Table 6. SA5 mentions the terms ML activation and ML deactivation several times in 3GPP TS 28.105 [9] but does not provide a definition.

Editor’s note: Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

| TSG (TS/TR) | ML model inference |
| --- | --- |
| 3GPP RAN1 TR 38.843 [3] | ML Model activation: enable an AI/ML model for a specific AI/ML-enabled feature. ML Model deactivation: disable an AI/ML model for a specific AI/ML-enabled feature. |

Table 6: Definition of ML model inference as defined across 3GPP WGs.

6.2.1.7	Analysis on ML model lifecycle

The term ‘ML model lifecycle’ has been defined by SA6, as illustrated in Table 7. However, 3GPP SA2 TS 23.288 [8] 3GPP SA2 TR 23.700-84 [4], 3GPP SA4 TR 26.927 [12], 3GPP SA5 TS 28.105 [9], 3GPP SA6 TR 23.700-82 [7], 3GPP RAN1 TR 38.843 [3] and RAN3 also mentions one or more phases of ML model life cycle without providing a clear definition of ML model lifecycle.

Editor’s note: Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

| TSG (TS/TR) | ML model lifecycle |
| --- | --- |
| 3GPP SA6 TR 23.700-82 [7] | The lifecycle of an ML model includes data collection, data processing, model training, model verification, model, instantiation and deployment, model monitoring and termination of ML model components. |

Table 7: Definition of ML model lifecycle as defined across 3GPP WGs.

6.2.1.8		Analysis on ML model lifecycle management

SA5 describes the ML model lifecycle in clause 4a.0 of 3GPP TS 28.105[9], and ML model lifecycle management capabilities for ML model training, ML model testing, ML inference emulation, ML model deployment and ML inference in clause 6.1 of 3GPP TS 28.105 [9].

Editor’s note: Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

6.2.2	Analysis on Horizontal Federated Learning

The term ‘Horizontal Federated Learning’ has been defined in SA2 and RAN1 defines ‘Federated Learning’, as illustrated in Table 9.

Editor’s note: Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

| TSG (TS/TR) | ML model inference |
| --- | --- |
| 3GPP SA2 TR 23.700-84 [4] | Horizontal Federated Learning: A federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples (e.g. UE IDs). |
| 3GPP RAN1 TR 38.843 [3] | Federated Learning: A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |

Table 9: Definition of Horizontal Federated Learning as defined across 3GPP WGs.

6.2.3	Analysis on Vertical Federated Learning

The term ‘Vertical Federated Learning’ has been defined in SA2 and RAN1 defines ‘Federated Learning’, as illustrated in Table 10.

Editor’s note: Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

| TSG (TS/TR) | ML model inference |
| --- | --- |
| 3GPP SA2 TR 23.700-84 [4] | Vertical Federated Learning: A federated learning technique without exchanging/sharing local data set, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs). |
| 3GPP RAN1 TR 38.843 [3] | Federated Learning: A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |

Table 10: Definition of Vertical Federated Learning as defined across 3GPP WGs.

6.2.4	Analysis on Decision vs Prediction vs Output

RAN1 and RAN3 only uses “prediction” in all corresponding ML related TRs/TSs. SA2 uses “output” in all corresponding TRs/TSs where output may include both statistics and predictions. SA5 uses “decision” in all corresponding TRs/TSs with few occurrences of “prediction”.

Editor’s note: Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

6.2.5	Analysis on ML vs AI vs AI/ML

RAN1, RAN2, RAN3 and SA1 only uses “AI/ML” in all corresponding ML related TRs/TSs. SA2 uses a mix of “ML” and “AI/ML” in all corresponding ML related TRs/TSs. SA3, SA4 and SA6 uses a mix of “AI/ML”, “AI” and “ML” in all corresponding ML related TRs/TSs. SA5 uses “ML” for training/testing/emulation and “AI/ML” for inference in all corresponding ML related TRs/TSs.

Editor’s note: Further analysis may be needed, e.g. to determine whether a unified definition can be derived.

6.3	AI/ML related features

6.3.X	Analysis on feature alignment #X <alignment title>

Editor's Note:	This clause describes AI/ML related terminology features misalignments, including cross-domain (UE, RAN, core network, media, OAM, and application enablement) aspects. Examples of areas to be investigated are LCM for AI/ML, data collection/storage/exposure, model training/delivery/ (de)-activation/inference emulation, inference/storage/exposure, performance evaluation and accuracy monitoring.

7	Overall Evaluation

Editor's Note:	This clause will provide a general evaluation of potential terminology inconsistency #X and potential feature misalignment #X

8	Conclusions

Editor's Note:	This clause will provide information on any potential outcome from clause 5, clause 6 and clause 7 to the respective WGs (according to their Terms of Reference (ToR)) to resolve any issues with appropriate SA-level co-ordination as necessary

Annex <A> (informative):

Editor's Note:	it is for FFS, which of these parts from SA5 (SP-241234) will be moved/incorporated to clause 6

A.1	ML model life cycle management (LCM)

Rel-18 specification [x2] addressed the AI/ML LCM management capabilities (including wide range of use cases, corresponding requirements (stage 1) and solutions (stage 2 NRMs & stage 3 OpenAPIs) for the ML model, including ML model training (which also includes validation), testing, AI/ML inference emulation, deployment and AI/ML inference steps of the lifecycle as shown below for managing the entire lifecycle of the ML model:

Start of quoted text (from TS 28.105)

## 4a.0	ML model lifecycle

AI/ML techniques are widely used in 5GS (including 5GC, NG-RAN, and management system), the generic AI/ML operational workflow in the lifecycle of an ML model, is depicted in Figure 4a.0-1.

Figure 4a.0-1: ML model lifecycle

The ML model lifecycle includes training, testing, emulation, deployment, and inference. These steps are briefly described below:

-	ML model training: training, including initial training and re-training, of an ML model or a group of ML models. It also includes validation of the ML model to evaluate the performance when the ML model performs on the training data and validation data. If the validation result does not meet the expectation (e.g., the variance is not acceptable), the ML model needs to be re-trained.

-	ML model testing: testing of a validated ML model to evaluate the performance of the trained ML model when it performs on testing data. If the testing result meets the expectations, the ML model may proceed to the next step If the testing result does not meet the expectations, the ML model needs to be re-trained.

-	AI/ML inference emulation: running an ML model for inference in an emulation environment. The purpose is to evaluate the inference performance of the ML model in the emulation environment prior to applying it to the target network or system.

NOTE:	The AI/ML inference emulation is considered optional and can be skipped in the AI/ML operational workflow.

-	ML model deployment: ML model deployment includes the ML model loading process (a.k.a. a sequence of atomic actions) to make a trained ML model available for use at the target AI/ML inference function.

ML model deployment may not be needed in some cases, for example when the training function and inference function are co-located.

-	AI/ML inference: performing inference using a trained ML model by the AI/ML inference function. The AI/ML inference may also trigger model re-training or update based on e.g., performance monitoring and evaluation.

End of quoted text

A.1.1	Observations and analyses: AI/ML LCM

- The AI/ML workflow defined by SA5 [x2] represents a general framework encapsulating the various life cycle management (LCM) operations for ML model (i.e., model training, testing, emulation, deployment, and inference).

- The AI/ML LCM capabilities defined by SA5 for each of the operational steps are generic for managing of 3GPP system including the Management and orchestration, CN and RAN domains.

- It is important to recognise that “domain-specific” ML model life cycle related tasks can be developed for the specific domains by the relevant 3GPP WGs, e.g., the RAN WGs can specify data collection within the RAN domain needed to train the UE-side, network-side, or the two-sided UE/network ML models, and specific LCM operations for UE-side model over air-interface.

- While ML model and AI/ML inference function life cycle can be specified by the relevant 3GPP WG for the specific domain (i.e., RAN, CN or Management & Orchestration), the “management aspects” of life cycle (i.e., life cycle management) remains to be primarily a “management task” that falls within the responsibility of SA5.

- The ML models and the associated “Life Cycle” can be a use case and/or domain specific, the management of the Life Cycle (i.e., LCM) is a higher layer task which is typically a role of the OAM that encompasses the process of e.g., the governance, automation, and operational practices applied to the entire AI/ML lifecycle. It is therefore imperative to distinguish the difference between Life Cycle and Life Cycle Management.

- Where feasible, the ML model LCM workflow and associated management capabilities specified by SA5 in TS 28.105 [9] could be considered by 3GPP for the currently ongoing and future relevant specification development. The 3GPP WG(s) should potentially provide AI/ML LCM-related requirements, if any, to SA5 to avoid duplication and contention of effort.

NOTE: SA5 Rel-18 specification in TS 28.105 on ML model LCM and the associated management capabilities [x2] does not address the UE-side and UE/Network-side Model LCM.

A.2	ML model lifecycle management capabilities

Each step in the ML model lifecycle. i.e., the ML model training, ML model testing, AI/ML emulation, ML model deployment and AI/ML inference correspond to number of dedicated management capabilities. The specified capabilities are developed based on corresponding use cases and requirements. The management capabilities specified by SA5 [x2] are highlighted below:

Start of quoted text (From TS 28.105)

## 6.1	ML model lifecycle management capabilities

Each operational step in the ML model lifecycle (see clause 4a.0.1) is supported by one or more AI/ML management capabilities as listed below.

Management capabilities for ML model training

-	ML model training management: allowing the MnS consumer to request the ML model training, consume and control the producer-initiated training, and manage the ML model training/re-training process. The training management capability may include training performance management and setting a policy for the producer-initiated ML model training.

-	ML model training capability also includes validation to evaluate the performance of the ML model when performing on the validation data, and to identify the variance of the performance on the training and validation data. If the variance is not acceptable, the ML model would need to be re-trained before being made available for the next step in the operational workflow (e.g., ML model testing).

Management capabilities for ML testing

-	ML model testing management:  allowing the MnS consumer to request the ML model testing, and to receive the testing results for a trained ML model. It may also include capabilities for selecting the specific performance metrics to be used or reported by the ML testing function. MnS consumer may also be allowed to trigger ML model re-training based on the ML model testing performance results.

Management capabilities for AI/ML inference emulation:

- AI/ML inference emulation: a capability allowing an MnS consumer to request an ML inference emulation for a specific ML model or models (after the training, validation, and testing) to evaluate the inference performance in an emulation environment prior to applying it to the target network or system.

Management capabilities for ML model entity deployment:

-	ML entity loading management: allowing the MnS consumer to trigger, control and/or monitor the ML model loading process.

Management capabilities for AI/ML inference:

-	AI/ML inference management: allowing an MnS consumer to control the inference, i.e., activate/deactivate the inference function and/or ML model/models, configure the allowed ranges of the inference output parameters. The capabilities also allow the MnS consumer to monitor and evaluate the inference performance and when needed trigger an update of an ML model or an AI/ML inference function.

The use cases and corresponding requirements for AI/ML management capabilities are specified in the following clauses.

End of quoted text

A.2.1	Observations and analyses: ML model lifecycle management capabilities

- ML model lifecycle management (LCM) capabilities are crucial for the effective deployment, operation, and optimization of AI/ML-enabled features and capabilities in both the NG-RAN and 5GC. These capabilities ensure that AI/ML models are not only developed and trained correctly but also tested, deployed, evaluated, and operated efficiently in the network environment.

- The management capabilities outlined in TS 28.105 offer a structured approach to managing the various steps of the ML model lifecycle. This structured approach is applicable to AI/ML-enabled features and capabilities in NG-RAN, 5GC, and management system, ensuring consistency and reliability in the deployment and operation of AI/ML technologies for different domains.

- The AI/ML LCM management capabilities are foundational for integrating advanced AI/ML features into 5G networks. By ensuring that ML models are effectively managed from the training step through to inference, these capabilities provide robust and reliable AI/ML-driven network enhancements.

- The AI/ML LCM workflow and associated management capabilities specified by SA5 in [x2] should be considered as the baseline for the AI/ML E2E framework for the 3GPP. These capabilities provide a comprehensive foundation for ensuring that AI/ML models and related processes are consistently managed across all steps of their lifecycle, promoting seamless integration and operation for all domain within the 5G system.

A.3	AI/ML functionalities management scenarios

The Rel-18 specification [x2] also documented AI/ML functionalities management scenarios in relation with managed AI/ML features which describe the possible locations of ML training function and AI/ML inference function involving the various 3GPP system domains.

Start of quoted text (From TS 28.105)

## 4a.2	AI/ML functionalities management scenarios (relation with managed AI/ML features)

The ML training function and/or AI/ML inference function can be located in the RAN domain MnS consumer (e.g. cross-domain management system) or the domain-specific management system (i.e. a management function for RAN or CN), or Network Function.

For MDA, the ML training function can be located inside or outside the MDAF. The AI/ML inference function is in the MDAF.

For NWDAF, the ML training function can be located in the MTLF of the NWDAF or the management system, the AI/ML inference function is in the AnLF.

For RAN, the ML training function and AI/ML inference function can both be located in the gNB, or the ML training function can be located in the management system and AI/ML inference function is located in the gNB.

Therefore, there might exist several location scenarios for ML training function and AI/ML inference function.

Scenario 1:

The ML training function and AI/ML inference function are both located in the 3GPP management system (e.g. RAN domain management function). For instance, for RAN domain-specific MDA, the ML training function and AI/ML inference functions for MDA can be located in the RAN domain-specific MDAF. As depicted in figure 4a.2-1.

Figure 4a.2-1: Management for RAN domain specific MDAF

Similarly, for CN domain-specific MDA the ML training function and AI/ML inference function can be located in CN domain-specific MDAF.

Scenario 2:

For RAN AI/ML capabilities the ML training function is located in the 3GPP RAN domain-specific management function while the AI/ML inference function is located in gNB. See figure 4a.2-2.

Figure 4a.2-2: Management where the ML model training is located in RAN domain management function and AI/ML inference is located in gNB

Scenario 3:

The ML training function and AI/ML inference function are both located in the gNB. See figure 4a.2-3.

Figure 4a.2-3: Management where the ML model training and AI/ML inference are both located in gNB

Scenario 4:

For NWDAF, the ML training function and AI/ML inference function are both located in the NWDAF. See figure 4a.2-4.

Figure 4a.2-4: Management where the ML model training and AI/ML inference are both located in CN

End of quoted text

A.3.1	 Observations and analyses: AI/ML functionalities management 	scenarios

- The functional arrangement scenarios defined by SA5 specifications demonstrate that different part of the ML model life cycle can be managed depending on the use case.

- The functional arrangements represent management deployment scenarios where for example ML model training related tasks can either be a domain specific or as a cooperative multi-domain task involving for example RAN and management & orchestration or CN and management & orchestration (OAM) domains.

- The LCM workflow defined by SA5 serves as a management framework to accommodate and enable all the possible functional arrangement scenarios within or cross-domains in the 3GPP system.

- The functional arrangement scenarios, coupled with the ML model LCM as defined by SA5 in TS 28.105 [9], can be considered in the ongoing and any future 3GPP relevant specification development.

Annex <B> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-09 | TSG SA#105 | SP-241367 | - | - | - | Proposed skeleton agreed for FS_AIML_CAL at TSG SA#105 | 0.0.0 |
| 2024-09 | TSG SA#105 | - |  |  |  | Implementing following approved papers: SP-241407, SP-241395, SP-241408, SP-241397, SP-241409, SP-241410. | 0.1.0 |