| 3GPP TR 22.870 V0.1.0 (2024-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group TSG SA; Study on 6G Use Cases and Service Requirements; Stage 1 (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	10

Introduction	11

1	Scope	12

2	References	12

3	Definitions of terms, symbols and abbreviations	14

3.1	Terms	14

3.2	Symbols	14

3.3	Abbreviations	14

4	Overview	14

5	System and Operational Aspects	14

5.1	Network Security for 6G	14

5.1.1	Description	15

5.1.2	Potential New Requirements	15

5.2	Use case on quantum-resistant security	15

5.2.1	Description	15

5.2.2	Pre-conditions	16

5.2.3	Service Flows	16

5.2.4	Post-conditions	16

5.2.5	Existing features partly or fully covering the use case functionality	16

5.2.6	Potential New Requirements needed to support the use case	16

5.3	Use case on UE selecting a Base Station after assessing its legitimacy	17

5.3.1	Description	17

5.3.2	Pre-conditions	17

5.3.3	Service Flows	17

5.3.4	Post-conditions	17

5.3.5	Existing features partly or fully covering the use case functionality	17

5.3.6	Potential New Requirements needed to support the use case	17

5.4	Use case on Fixed Wireless Access (FWA)	17

5.4.1	Description	17

5.4.2	Potential New Requirements needed to support the use case	18

5.5	Use case on end-to-end energy efficiency improvement for the network and UE	18

5.5.1	Description	18

5.5.2	Pre-conditions	19

5.5.3	Service Flows	20

5.5.4	Post-conditions	20

5.5.5	Existing features partly or fully covering the use case functionality	20

5.5.6	Potential New Requirements needed to support the use case	20

6	AI	21

6.1	Use Case	21

7	Integrated Sensing and Communication	21

7.0	General	21

7.1	Use Case on coordination of search and rescue missions in large disaster areas	21

7.1.1	Description	21

7.1.2	Pre-conditions	21

7.1.3	Service Flows	21

7.1.4	Post-conditions	22

7.1.5	Existing features partly or fully covering the use case functionality	22

7.1.6	Potential New Requirements needed to support the use case	22

7.2	Use Case on safety assistance for vulnerable pedestrians	22

7.2.1	Description	22

7.2.2	Pre-conditions	23

7.2.3	Service Flows	23

7.2.4	Post-conditions	23

7.2.5	Existing features partly or fully covering the use case functionality	23

7.2.6	Potential New Requirements needed to support the use case	23

7.3	Use case for high-resolution topographical maps	24

7.3.1	Description	24

7.3.2	Pre-conditions	24

7.3.3	Service Flows	24

7.3.4	Post-conditions	25

7.3.5	Existing features partly or fully covering the use case functionality	25

7.3.6	Potential New Requirements needed to support the use case	25

7.4	Use case on low-altitude UAV supervision	26

7.4.1	Description	26

7.4.2	Pre-conditions	28

7.4.3	Service Flows	28

7.4.4	Post-conditions	29

7.4.5	Existing features partly or fully covering the use case functionality	29

7.4.6	Potential New Requirements needed to support the use case	29

7.5	Use Case on Environment Object Reconstruction	29

7.5.1	Description	29

7.5.2	Pre-conditions	30

7.5.3	Service Flows	30

7.5.4	Post-conditions	32

7.5.5	Existing features partly or fully covering the use case functionality	32

7.5.6	Description	32

7.6	Use Case on road digitalization	32

7.6.1	Description	32

7.6.2	Pre-conditions	32

7.6.3	Service Flows	33

7.6.4	Post-conditions	34

7.6.5	Existing features partly or fully covering the use cast functionality	34

7.6.6	Potential New Requirements needed to support the use case	34

8	Ubiquitous Connectivity	34

8.1	Use case on Ubiquitous and Resilient Network	34

8.1.1	Description	34

8.1.2	Pre-conditions	35

8.1.3	Service Flows	36

8.1.4	Post-conditions	36

8.1.5	Existing features partly or fully covering the use case functionality	36

8.1.6	Potential New Requirements needed to support the use case	37

8.2	Use case on enhanced user experience with sparse LEO satellite deployment	37

8.2.1	Description	37

8.2.2	Pre-conditions	37

8.2.3	Service Flows	37

8.2.4	Post-conditions	38

8.2.5	Existing features partly or fully covering the use case functionality	38

8.2.6	Potential New Requirements needed to support the use case	38

8.3	Use case on “service continuity for wearable mobile devices”	38

8.3.1	Description	38

8.3.2	Pre-conditions	39

8.3.3	Service Flows	39

8.3.4	Post-conditions	39

8.3.5	Existing features partly or fully covering the use case functionality	39

8.3.6	Potential New Requirements needed to support the use case	40

8.4	Use case on resilient positioning in satellite networks	40

8.4.1	Description	40

8.4.2	Pre-conditions	40

8.4.3	Service Flows	41

8.4.4	Post-conditions	41

8.4.5	Existing features partly or fully covering the use case functionality	41

8.4.6	Potential New Requirements needed to support the use case	42

8.5	Use case on “Disaster relief”	42

8.5.1	Description	42

8.5.2	Pre-conditions	43

8.5.3	Service Flows	43

8.5.4	Post-conditions	43

8.5.5	Existing features partly or fully covering the use case functionality	43

8.5.6	Potential New Requirements needed to support the use case	45

8.6	Use case on “low-energy positioning in satellite networks”	45

8.6.1	Description	45

8.6.2	Pre-conditions	45

8.6.3	Service Flows	45

8.6.4	Post-conditions	45

8.6.5	Existing features partly or fully covering the use case functionality	46

8.6.6	Potential New Requirements needed to support the use case	46

8.7	Use case on Global mobile video	46

8.7.1	Description	46

8.7.2	Pre-conditions	47

8.7.3	Service Flows	47

8.7.4	Post-conditions	47

8.7.5	Existing features partly or fully covering the use case functionality	47

8.7.6	Potential New Requirements needed to support the use case	47

9	Immersive Communication	48

9.1	Use case on Immersive Gaming	48

9.1.1	Description	48

9.1.2	Pre-conditions	50

9.1.3	Service Flows	50

9.1.4	Post-conditions	51

9.1.5	Existing features partly or fully covering the use case functionality	51

9.1.6	Potential New Requirements needed to support the use case	51

9.2	Use case on multi-media services with deterministic experience via collaborative processing among UE-network-cloud	51

9.2.1	Description	51

9.2.2	Pre-conditions	53

9.2.3	Service Flows	53

9.2.4	Post-conditions	54

9.2.5	Existing features partly or fully covering the use case functionality	54

9.2.6	Potential New Requirements needed to support the use case	54

9.3	XR rendering offload support	55

9.3.1	Description	55

9.3.2	Pre-conditions	55

9.3.3	Service Flows	55

9.3.4	Post-conditions	55

9.3.5	Existing features partly or fully covering the use cases functionality	55

9.3.6	Potential New Requirements needed to support the use case	55

9.4	Use case on Seamless Immersive Reality in Education	56

9.4.1	Description	56

9.4.2	Pre-conditions	57

9.4.3	Service Flows	57

9.4.4	Post-conditions	57

9.4.5	Existing features partly or fully covering the use case functionality	58

9.4.6	Potential New Requirements needed to support the use case	58

10	Massive Communication	59

10.1	Use case 1	59

11	Further Use Cases on Industry and Verticals	59

11.1	Use case on communication on board of UAM aircrafts	59

11.1.1	Description	59

11.1.2	Pre-conditions	60

11.1.3	Service Flows	60

11.1.4	Post-conditions	60

11.1.5	Existing features partly or fully covering the use case functionality	60

11.1.6	Potential New Requirements needed to support the use case	60

W	Other Use Cases	61

W. 1	Use case on coordinating computing and communication for XR rendering	61

W.1.1	Description	61

W.1.2	Pre-conditions	62

W.1.3	Service Flows	62

W.1.4	Post-conditions	63

W.1.5	Existing features partly or fully covering the use case functionality	63

W.1.6	Potential New Requirements needed to support the use case	63

W.2	Use case on Optimizing 6G Infrastructure Utilization via Resource Exposure in 6G	63

W.2.1 	Description	63

W.2.2	Pre-conditions	64

W.2.3	Service Flows	64

W.2.4	Post-conditions	65

W.2.5	Existing features partly or fully covering the use case functionality	65

W.2.6	Potential new requirements needed to support the use case	65

X	Other Considerations	65

Y	Consolidated Potential Requirements	65

Z	Conclusion and Recommendations	65

Annex A: Additional Use Cases	66

A.1	Use Case #X	66

A.2	Use Case #Y	66

A.3	Use Case #Z	66

Annex <X>: Change history	67

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[5.1X]	QIAN LI et all, “6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies” in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

[6.5x]	GSMA, "Going green: benchmarking the energy efficiency of mobile networks (Second edition)", Feb 2023.

[6.5y] 	3GPP TR 32.972: " Study on system and functional aspects of energy efficiency in 5G networks".

[6.5z]	3GPP TR 23.700-66

[7x]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[7.2x]	5GAA: "5GAA MRP input to 3GPP Stage 1 workshop on IMT 2030 Use Cases".

[7.2y]	5GAA: Technical report “Evolution of vehicular communication systems beyond 5G”

[7.3x]	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication",

[7.3y]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1"

[7.3z]	SAE J3016 202104 Levels of Driving Automation.

[7.4x]	TR 22.837, Feasibility Study on Integrated Sensing and Communication.

[7.4y]	G. Liu, L. Ma, Y. Xue, et al, “SensCAP: A Systematic Sensing Capability Performance Metric for 6G ISAC,” IEEE Internet of Things Journal, vol. 11, no. 18, pp. 29438–29454, 2024.

[7.4z]	Yasin, Jawad N., et al. "Unmanned aerial vehicles (uavs): Collision avoidance systems and approaches." IEEE access 8 (2020): 105139-105155.

[7.5r1]	Igal Bilik, Oren Longman, Shahar Villeval, and Joseph Tabrikian, “The Rise of Radar for Autonomous Vehicles”, IEEE signal processing magazine, Volume 36, Issue 5, September 2019.

[7.5r2] 	https://www.geospatialworld.net/prime/vodafone-country-scale-digital-twins-engineer-networks.

[7.5r3] 	https://knowledge-base.inspire.ec.europa.eu/publications/technical-guidance-implementation-inspire-dataset-and-service-metadata-based-isots-191392007_en.

[7,6x]	3GPP TS 22.261: "Service requirements for the 5G system".

[9.1X] 	 Wang et.al, “Effect of Frame Rate on User Experience, Performance, and Simulator Sickness in Virtual Reality”, IEEE Transaction on Visualization and Computer Graphics. 2023 Feb 22:PP. doi: 10.1109/TVCG.2023.3247057

[9.2x1] 	https://en.wikipedia.org/wiki/Adreno

[9.2x2] 	https://www.labtekno.com/snapdragon-8-gen-2-vs-apple-a16-bionic/

[9.2x3] 	Algorithms and tools for GPU-accelerated Computer Generated Holography | Ewireless Research Group (https://www.ewireless.eng.ed.ac.uk/projects)

[9.2x4] 	https://phlearn.com/tutorial/how-many-exposures-hdr/

[9.2x5]	https://photographylife.com/compressed-vs-uncompressed-vs-lossless-compressed-raw

[9.2x6]	https://www.dotphoton.com/products/jetraw-core

[9.2x7] 	https://www.lambdatest.com/learning-hub/web-performance-testing

[9.2x8] 	Static 3D triangle mesh compression overview, Marcos Aviles, Grupo de Tratamiento de Imágenes, Universidad Politécnica de Madrid, Spain, 15th IEEE International Conference on Image Processing

[9.2x9] 	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[8.4X]	EUSPA, “Report on Aviation and Drones – User Needs and Requirements”, 2023.

[8.6x]	3GPP TR 36.763: “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”.

[9.0x]	Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

[9.0y]	3GPP TS 22.261: " Service requirements for the 5G system; Stage 1".

[9.0z]	3GPP TS 22.156: "Mobile Metaverse Services".

[9.3X] 	ITU-R Recommendation M.2160-0 (11/2023): "Framework and overall objectives of the future development of IMT for 2030 and beyond" (https://www.itu.int/rec/R-REC-M.2160-0-202311-I/en).

[9.3Y] 	3GPP TS 23.502: "Procedures for the 5G System".

[9.4x]	Hexa-X-II, "Deliverable D1.2: 6G Use Cases and Requirements", December 2023.

[11.1a]	https://www.easa.europa.eu/en/what-is-uam.

[11.1b]	https://www.autoflight.com/en/air/.

[11,1c]	"Low altitude economy development white paper (2.0) All digital solutions", International digital economy academy (IDEA), 2023.

[11.1d]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP; Stage 1".

11.1e]	https://www.unmannedairspace.info/latest-news-and-information/shenzhen-reaches-record-number-of-drone-operations-with-600000-flights-in-2023/

[11.1f]	"C-V2X Use Cases and Service Level Requirements Volume II", 5GAA White Paper, 2023.

[11.1g]	https://www.flyeye.io/drone-acronym-daa/.

[W.2x]	Report ITU-R M.2516-0, "Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond ". (https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-M.2516-2022-PDF-E.pdf)

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Overview

Editor's Note: This part will cover the high-level value and principle of 6G system.

Editor's note: To introduce 6G with some generic justification and structure of the TR.

Overview text…

Editor's Note: for all the following clauses: if it is unclear which clause a proposed use case relates to (e.g. if the use case is linked to more than one clause), the use case ought to be proposed in the clause that closely relates to the main idea of the use case.

# 5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, sustainability and energy efficiency, device diversity, support of legacy services

## 5.1	Network Security for 6G

### 5.1.1	Description

It is expected that cellular networks become “more secure” with each subsequent generation.  Societies are becoming more connected, increasing reliance on mobile networks to provide crucial connectivity for all aspects of daily life. Networks are becoming more complex, with 6G also seeming to be a convergence of disparate technologies (e.g., information, operation, communication), complicating the ecosystem with their differences in approaches, threats, disciplines, and capabilities.

It is incumbent on the mobile communication ecosystem to enhance security and privacy, embracing new security paradigms, techniques, and leveraging evolving security technologies.

Moreover, it is expected that cloud deployments will play an even more important role in 6G as it is today for 5G. Network elements will be deployed in private, public, and hybrid clouds and not all network elements of one operator’s network will be deployed in the same cloud or data centre. Network elements of one operator might also be instantiated in another operator’s cloud environment.

This requires adapting the security mechanisms, with network elements being able to make trust/security related decisions self-sufficiently based on the owners’ policy and without constant enquiries to a central authority in the home or visited network.

Furthermore, it is important that trust and security is applied on the appropriate layer, i.e. not limited to transport layer.

Despite the additional dimension of multi-stakeholder cloud environments, it is important to decrease the administrative and operational burden for establishing security within and between 3GPP networks. As we can see today many operators struggle with the bilateral administration of security to enable interconnection and roaming with other operators.

Therefore, a decentralized, yet common approach for exchanging security keys and authentication/authorization credentials is needed.

### 5.1.2	Potential New Requirements

[PR-5.x-1] The 6G network shall provide security mechanisms for secure access to and communication with network elements.

[PR-5.x-2] The 6G network shall support establishment of secure communication between network elements while protecting network related information (e.g. network element identities, topology) from disclosure to unauthorized parties.

## 5.2	Use case on quantum-resistant security

### 5.2.1	Description

According to some public predications, CRQC (cryptographically relevant quantum computer) may appear in 2030s.  However, quantum computing technology poses significant threats to some classical cryptography, especially to widely-used cryptographic algorithms (including both symmetric and asymmetric cryptography) that secure much of today's data, rendering electronic secrets vulnerable to discovery. Hence, the migration to quantum-resistant algorithms shall start as soon as possible.

The following figure lists the general application scenarios of cryptographic algorithms in 5G system.

Figure 5.2.1-1 Application scenario of cryptographic algorithms for 5G system

### 5.2.2	Pre-conditions

Alice bought a new cell phone with quantum-resistant capability and started to use the cell phone to connect to mobile networks. The network also turns on the quantum-resistant capability in order to protect users from quantum attacks. The quantum attacks as shown in Figure 5.2.2-1 will be blocked by the quantum-resistant approaches.

Figure 5.2.2-1 An example of quantum attacks to the UE and the mobile network

### 5.2.3	Service Flows

Assume that an attacker has the CRQC capability, the attacker can crack the traditional cryptographic algorithms based on the CRQC function and launch attacks to the network.

1.  Alice turns on the UE. The UE attaches to a base station, registers to the mobile networks, and establishes PDU session. Alice’s UE initiates some services and starts to transfer data between her phone and the network through the mobile network.

2.  An attacker is doing some malicious behaviours like eavesdropping and deciphering the traffic using CRQC, blocking and inserting malicious messages, trying to break and forge some NFs, etc.

3.  As the UE privacy, communication for different elements (UE\NF) and service authorization are protected by quantum-resistant approaches, the attack initiated by the attacker fails.

### 5.2.4	Post-conditions

Thanks to the quantum-resistant approaches adopted by the UE and network, the interactions between UE and network, between network elements, and between different PLMNs is protected from attackers with CRQC.

### 5.2.5	Existing features partly or fully covering the use case functionality

Quantum threats used to be studied in Release 16 TR 33.841 for 5G systems.

3GPP imported 256-bit Confidentiality and Integrity Algorithms for the Air Interface in TS 35.246, TS 35.247, TS 35.248 during Release 19. These 256-bit algorithms can be used to protect the RRC/NAS/UP traffic of the air interface from quantum attack. But how to use these algorithms (e.g., algorithm negotiation, key derivation) in 3GPP has not specified yet. And quantum-resistance is not considered in other scenarios like the identity protection, the NDS-IP security, the NF-to-NF authentication and communication protection, the NF service authorization, inter-PLMN protection, etc.

Editor’s note:  references are needed for the TR and TSs listed above.

### 5.2.6	Potential New Requirements needed to support the use case

[PR-5.2.6-1]	The 6G system shall provide security protection for communication against the potential attacks posed by quantum computing.

## 5.3	Use case on UE selecting a Base Station after assessing its legitimacy

### 5.3.1	Description

The majority of the efforts in the mobile communication system security has been focused on delivering secure communication once connected, whereas security related to bootstrapping those connections are relatively less explored. A UE can interact with a Base Station without assessing its legitimacy. This opens way for diverse attacks. False Base Station (FBS) attacks are one such example and its potential to cause active and passive impacts are an alarming concern, worldwide. Addressing fundamental security issues is important, and can entail a system level reappraisal of risk and expectations. This is most feasible to undertake at the beginning of a new mobile generation. Attacks mounted by a FBS is a major security flaw in the existing communication system, There is an important opportunity to address this concern in the next generation wireless systems.

In this use case, it is assumed that by the increasing the level of security between the radio network entities and the UE during the initial network selection procedure, it will be possible to prevent false Base Station at earlier stages of communication establishment. The sooner that security threats are addressed in the process of interaction between components of the system, the more risks can be reduced to the 3GPP system.

### 5.3.2	Pre-conditions

User A subscribes to the communication services of Operator A.

Operator A deploys an efficient Base Station authenticity verification mechanism such that the UE can verify the authenticity of the Base Station.

### 5.3.3	Service Flows

1.	Operator A’s network provides necessary security parameters to verify the authenticity Base Station.

2.	User A turns on the mobile phone.

3.	User A’s mobile phone verifies the authenticity of any Base Station.

4.	User A's mobile phone camps on the authentic Base Station of the Operator A’s network and starts the 6G registration procedure.

### 5.3.4	Post-conditions

User A always camps and accesses the services from authentic Base Stations of Operator A.

### 5.3.5	Existing features partly or fully covering the use case functionality

There are no existing features partly or fully covering the use case functionality.

### 5.3.6	Potential New Requirements needed to support the use case

[PR-5.3.6-1]	Subject to operator policy and regulatory requirements, the 6G system shall support a means for a UE to be able to distinguish a false base station from an authentic base station.

## 5.4	Use case on Fixed Wireless Access (FWA)

### 5.4.1	Description

Fixed Wireless Access (FWA) in 5G NR has transformed the Home Internet business and has become a significant revenue generator for mobile network operators (MNO).  It is expected that FWA services will continue to grow in 6G. While FWA has been supported since LTE, 5G’s speed and flexibility has enabled it to emerge as a transformative and successful 5G service. FWA has become a key strategy in expanding high-speed broadband services to previously unserved or underserved areas (i.e., a tool to bridge the Digital Divide), for in-building services, and enterprise (i.e., private network) deployments. First agreed 6G Use Case

Some initial deployments of FWA used “fallow capacity” in markets where eMBB usage was not in high demand, but the underlying network protocols, procedures, and resource management schemes of the macro networks are designed for eMBB services and Smartphone devices that require mobility support.  FWA devices have a very different mobility profile, traffic usage pattern, and are able to take advantage of higher output power improving their in-building link performance.

FWA should provide a quantitative improvement in service by using 6G services and capabilities (e.g., AI/ML).

### 5.4.2	Potential New Requirements needed to support the use case

[PR-5.4-1] The 6G system shall provide optimized network capabilities for FWA (e.g., support stationary devices).

[PR-5.4-2] The 6G system shall support FWA in relevant bands taking into consideration the regulatory requirements for each specific band.

[PR-5.4-3] The 6G system shall enable the means to provide awareness of user service characteristics (e.g., data rate, latency) to support the RAN and CN in making real-time resource allocation for FWA.

[PR-5.4-4] The 6G system shall provide FWA a comparable level of security and privacy protection to other 6G services.

[PR-5.4-5] The 6G system shall support efficient FWA connectivity.

Editor’s Note: this requirement is FFS.

## 5.5	Use case on end-to-end energy efficiency improvement for the network and UE

### 5.5.1	Description

In the ITU-R Recommendation “Framework and overall objectives of the future development of IMT for 2030 and beyond”, sustainability is considered as the design principles commonly applicable to all usage scenarios. Supporting end-to-end energy efficiency is an important design target for the sustainability of 6G system.

Editor’s note:  reference is needed for the ITU document referenced above.

Regards to end-to-end energy efficiency of 6G system, UE energy efficiency is an important part. Power consumption of modem (including both baseband and RF) has contributed more and more to total UE power consumption due to large number of power consuming features supported, e.g., AI and computing as well as the strong demand of high data volume transmission. Energy efficiency design for UE could further extend UE battery life while guarantee certain service quality. In the 6G system, it is expected that the network could assist the UE to improve energy efficiency as one of the services, e.g., UE could subscribe energy efficiency service to realize power saving based on network assistance. The UE could also assist the network to improve the system energy efficiency to further reduce operation cost for mobile operators. So the UE and network could mutually benefit from the coordination for energy efficiency improvement.

Considering the above, the energy efficiency and energy saving should consider the end-to-end performance including both the network and UE. Considering the sustainability target of the 6G system, it should greatly improve the end-to-end energy efficiency compared to the 5G system.

Figure 5.5.1-1: Supporting end-to-end energy efficiency and energy saving

Energy efficiency shall be a quantifiable metric of sustainability and is important for the success of IMT-2030 technology. The energy efficiency KPI should be defined to ensure that IMT-2030 is designed in an energy efficient way. From the operator’s point of view, the KPI helps the operator to decide how much the network energy efficiency is improved by applying a specific energy-saving technology. Then, based on the quantified KPI, the network could provide energy efficiency improvement services for the subscribers. From the subscriber’s point of view, the KPI clearly shows that the energy efficiency improvement services are important for them to reduce the power consumption for the data transmission, so they are more willing to subscribe the services from the network. This would also be a new business model for the operators. In general, the energy efficiency KPI is important and helpful to both the operators and subscribers.

Energy efficiency should be defined taking into account both communication performance metric and energy consumption. According to GSMA study [6.5x] and 3GPP TR 32.972 [6.5y], several measurements means for energy efficiency are proposed by considering the energy consumption and the data transmission. Communication related performance (e.g. data volume, latency, data rate, etc.) of data transmission should be considered for the energy efficiency measurements. For example, energy saving by reducing the data rate can’t always satisfy the subscribers’ requirements. It is also important for the network to realize energy saving with guaranteed QoS.

Regards to end-to-end energy efficiency, the energy related information collection from radio access network, e.g., at per UE, or per application granularity, should also be considered. Radio conditions would affect the energy consumption of the UE for the same amount of data volume transmission. For example, the UE location in the Cell affects the energy consumption, e.g., the UE located in cell edge consumes more energy compared to the one located in cell centre for the same amount of data volume transmission.

Considering the energy efficiency at the UE, it is closely related with the user experience. Based on different technologies in the 6G network to improve the energy efficiency, the operator may offer different energy efficiency improvement services to the subscribers. Based on UE’s indication, the network may apply certain power saving technology to improve energy efficiency at the UE to satisfy the user’s requirement on energy efficiency. This energy efficiency improvement service for the subscribers is also a new business model for the operators in 6G system. The new business model is also the driven for the 6G system deployment. The energy efficiency improvement service would further benefit the operators and subscribers to achieve the energy efficiency target in the 6G system.

### 5.5.2	Pre-conditions

The operator has offered a series of energy efficiency improvement services in the 6G network for the UE to save energy and extend the battery life. Different energy efficiency improvement services may be provided to the subscribers at different prices or incentives. The subscribers may order certain energy efficiency improvement services.

For example, the operator A may offer the following two kinds of energy efficiency improvement services:

-	Energy efficiency improvement service A: Improving the energy efficiency by reducing UE energy consumption with maintaining the service experience (e.g., low latency, high throughput).

-	Energy efficiency improvement service B: Improving the energy efficiency by reducing UE energy consumption with essential service access, e.g., voice call only. The essential service list is based on the agreement between the subscriber and operator.

Bob buys a pair of AR glasses and wants to wear it to access AR content during the trip. Normally, the battery of his AR glasses can last for two hours. Since it’s not possible to find the power source during the trip for the AR glasses, he orders the Energy efficiency improvement service A for the glasses from the operator.

Bob also has a cell phone. Sometimes his cell phone is about to run out of battery, but he does not want to miss some important calls, or still wants to access certain applications (e.g. navigation map application). He prefers the network to provide essential network access for services (e.g. voice call) for his cell phone in order to improve the energy efficiency and extend the battery life. So he orders the Energy efficiency improvement service B with providing his preference on the essential service list for his cell phone.

### 5.5.3	Service Flows

When visiting the pyramids, Bob wears his AR glasses to access specific AR tour guide information. Usually, the battery of his AR glasses can last for two hours. Considering that the tour might last for four hours, in order to save power for this AR glasses, Bob indicates to the network to apply Energy efficiency improvement service A to minimize the power consumption of the AR glasses while ensuring the specific QoS. Based on the indication, the network applies Energy efficiency improvement service A for Bob’s AR glasses.

After the long trip, Bob needs to take the bus to go back home. It takes nearly one hour. He finds that the battery of his cell phone is below 10% and he can’t charge his cell phone on the bus. So he indicates to the network to apply Energy efficiency improvement service B for his cell phone. Based on the indication, the network applies Energy efficiency improvement service B for Bob’s cell phone. The network only provides limited network access to the essential services (e.g. voice call) as predefined by Bob in order to reduce the UE energy consumption.

### 5.5.4	Post-conditions

During the visit to the pyramids, Bob is able to watch AR tour guide information with his AR glasses with good service experience. The AR glasses is used for four hours with good service experience due to the Energy efficiency improvement service A.

During the trip back home, due to Energy efficiency improvement service B, although the battery of his cell phone is below 10%, Bob’s cell phone could still access the basic services (e.g. voice call) for nearly one hour.

### 5.5.5	Existing features partly or fully covering the use case functionality

The energy efficiency requirements for 5G system have been defined in clause 6.15 and 6.15a of TS 22.261 [7.6x], including the general requirements, requirements for energy related information as a service criteria, requirements for energy states, requirements for monitoring and measurement, requirements for information exposure, requirements for network actions leveraging energy efficiency as a service criteria.

The study on Energy Efficiency as service criteria Ph 2 is under study in SA1 R20. The related use cases and requirements to the 5G system are introduced in TR 22.283 [6.5y].

Some of the procedures and network functions proposed in 3GPP TR 23.700-66 [6.5z] can be used to realize part of the 5G requirements and service flows described in TS 22.261[7,6x] above.

The 6G system requirements on the energy efficiency and energy saving are based on the existing requirements in the 5G.

Editor’s note:  Reference to TR 23.700-66 is needed.

### 5.5.6	Potential New Requirements needed to support the use case

[PR-5.5.6-1] The 6G system shall improve energy efficiency of 6G system compared to 5G system.

# 6	AI

Editor's Note:  this clause will contain AI-related use cases.

## 6.1	Use Case

# 7	Integrated Sensing and Communication

Editor's Note: "Integrated Sensing and Communication" facilitates new applications and services that require sensing capabilities. It offers wide area multi-dimensional sensing that provides spatial information about unconnected objects as well as connected devices and their movements and surroundings.

## 7.0	General

Integrated Sensing and Communication (ISAC) facilitates new applications and services that require sensing capabilities. The service includes offering wide area multi-dimensional sensing that provides spatial information about unconnected objects as well as connected devices and their movements and surroundings.

Normative service requirements for ISAC have been specified in [7x].

## 7.1	Use Case on coordination of search and rescue missions in large disaster areas

### 7.1.1	Description

In large disaster areas, a high degree of coordination for the search and rescue operations is essential. In such situations, sensing can play an important role in providing helpful information to the PPDR authorities and first responders by providing an integrated platform for real-time monitoring and coordination which will support the efficient allocation of resources and the facilitation of decision-making in challenging environments.

Collecting data from a disaster area(s) involves the use of special equipment and devices supporting sensing to capture real-time information about the affected area. Also, the base stations in the disaster areas, or temporarily deployable and tactical base stations, can be used for providing the required sensing services.

The collected environmental and devices sensing data can be used to generate real-time maps for the affected aeras allowing to prioritize and to direct the rescue efforts more efficiently. Furthermore, these maps can be used to monitor the evacuation processes, as well as the situations at the evacuation centres (e.g. to detect overcrowding at an evacuation centre).

### 7.1.2	Pre-conditions

A) After the earthquake struck, a wide disaster area emerged.

Base stations can provide sensing services even after a disaster, or new base stations can be installed to ensure providing the required capacity and/or coverage over areas where TN is down.

Rescue teams equipped and using special equipment and devices supporting sensing.

B) people are expected to gather at evacuation centres (rescue points).

The data on people flow is needed at evacuation centres to distribute disaster relief supplies.

### 7.1.3	Service Flows

A) Coordination of search and rescue:

- Rescue teams start searching using sensing devices.

- Sensing technology is used to analyse the environment, structure, etc. of the disaster areas and generate real-time maps.

- The generated maps will reflect severely affected areas, searched areas, and unsearched areas.

- The rescue command centre can use the real-time map information to optimize the dispatch of rescue teams.

B) People evacuation, and gathering at designated evacuation centres:

- By using sensing technologies, data about people flow can be monitored and provided to rescue teams.

- The rescue teams will provide the necessary relief supplies based on the level of crowding at the evacuation centres and distribute them accordingly.

- In the event of overcrowding at an evacuation centre, additional facilities will be installed (e.g. portable toilets, drinking water points) or a secondary evacuation centre will be set up to guide people there.

### 7.1.4	Post-conditions

A) The Rescue Command Centre ensures the optimal allocation of resources for more efficient rescue operations.

B) The real-time monitoring of crowds helps in providing the required supplies. In the case of overcrowding, appropriate measures can be taken.

### 7.1.5	Existing features partly or fully covering the use case functionality

None.

### 7.1.6	Potential New Requirements needed to support the use case

[PR-7.1.6-1]	Based on operator policy, regional and/or national regulations, the 6G network in the ETWS Notification Area shall provide secure mechanisms of collecting the sensing results with a specified level of accuracy that can be used to generate real-time maps

Editor's Note: 	Definition of KPIs for the above requirements is FFS.

## 7.2	Use Case on safety assistance for vulnerable pedestrians

### 7.2.1	Description

In a presentation by 5GAA at the 6G Use Case Workshop [7.2x], a positioning accuracy of 1 meter was identified as necessary for Vulnerable Road Users (VRUs). Additionally, their technical report [7.2y] noted that VRUs might carry various devices to enhance pedestrian safety. GNSS-based positioning alone is insufficient due to issues such as slow convergence, multipath in urban jungles, and susceptibility to jamming or spoofing. Therefore, rapid sensing through communication with roadside base stations and other infrastructure is essential. For instance, if a pedestrian begins moving toward a crosswalk at a red light, it is crucial to quickly detect this movement and send a warning to the user equipment (UE) they carry. This situation may occur even with typical pedestrians; those looking at their smartphones while waiting for the light often mistakenly interpret movements around them as an indication that the signal has changed and start walking.  Furthermore, assessing whether a pedestrian can fully cross before the light turns red requires an instantaneous, quantitative understanding of their walking speed. With this data, it becomes possible to determine, based on the road width and remaining green-light time, whether the pedestrian can cross safely.

While speed detection can be achieved through side imaging, using radio waves with high rectilinearity, such as mm-wave, to detect variations of the propagation time or to measure the Doppler shift creates a robust system that does not demand extensive computing power. Although pedestrians are the primary VRU group, the aging population has led to increasingly varied walking speeds, making it impractical to assume a single, typical walking speed. Real-time measurement in each instance is essential. Besides, in the case of wheelchair users braking can be applied more quickly and reliably without human intervention. However, applying the optimal braking force to prevent forward pitching requires knowledge of the speed before braking, making it particularly important to accurately capture low-speed movements.

### 7.2.2	Pre-conditions

Good partnership and cooperation are established between the road supervision department and Mobile Operator A in City B. At the request of the supervision department, suitable base stations or small cells equipped with directional antennas are selected and installed near crossings. This setup enables Mobile Operator A to continuously detect the presence of VRUs who have subscribed to safety assistance services.

### 7.2.3	Service Flows

1. Bill, who lives in City B, is now 75 years old. Although he remains active, he can no longer conceal the decline in his mobility. At his family’s suggestion, he has signed up for the safety assistance service option with Mobile Operator A.

2. One summer day, after enjoying a conversation with an old friend at his favourite bar, Bill started walking home. However, his thoughts were so absorbed in replaying the conversation that he attempted to cross the street at a red light without realizing it.

3. At that moment, a warning sound emitted from his mobile phone, stopping him in his tracks. A young person standing next to him, who happened to be looking at their smartphone while waiting for the light, also noticed the warning and helped him stop.

4. Just as Bill was thanking the young person, the light changed to green without him realizing. The young person quickly crossed the street, and Bill started crossing as well, but his phone emitted another warning sound.

5. This second alert made Bill realize that, at his current walking speed, he wouldn’t be able to make it across during the current green light. He decided to wait until the light turned green again. As soon as it did, he started crossing, and this time there was no warning sound, allowing him to cross safely.

### 7.2.4	Post-conditions

Thanks to the capability to detect and evaluate the mobility of vulnerable pedestrians and to alert them via their mobile devices, the safety level of highly vulnerable individuals is enhanced. Network-based sensing provides rapid, accurate, and robust results, which can also support autonomous vehicle driving by detecting and reporting the movement of these pedestrians.

### 7.2.5	Existing features partly or fully covering the use case functionality

GNSS can estimate a pedestrian's position with an accuracy of approximately several meters while they are walking.

By taking GNSS positioning measurements at multiple locations over several seconds or ten seconds, an approximate speed can be calculated.

Positioning technology using carrier phase for centimetre-level accuracy is currently being discussed for indoor applications

### 7.2.6	Potential New Requirements needed to support the use case

[PR-7.2.6-1]	The 6G system shall be able to support the following KPIs.

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Safety Assistance for vulnerable pedestrians | Outdoor (Crossing) | [95] | [1] | N/A | [0.5] | N/A | [0.2] | [0.5] x N/A | ≤[500] | ≤[0.1] | ≤[5] | ≤[5] |

## 7.3	Use case for high-resolution topographical maps

### 7.3.1	Description

Some 5G Sensing Use Cases in [7.3x] and the associated functional requirements and performance requirements in [7.3y], capture the use of mobile network sensing measurements for services such as topographical maps, through the exposure of the sensing results to an application server connected to the mobile network. The application server may combine this sensing information with other non-3GPP sensing data e.g. LiDAR, camera data, and/or with other non-3GPP data e.g. localization, synchronisation data, to improve the detail of the application server output by enhancing the output data/product beyond that obtainable from the mobile network sensing measurement data alone.

Enhancements in 6G mobile network sensing will provide for enhanced sensing performance for example in terms of measurement accuracy, measurement latency, measurement confidence level and measurement efficiency through enhanced wave form design, enhanced integration of 6G mobile network access node connectivity and coordinated resource utilisation across triggered sensing transmitters.

6G mobile network sensing in the RAN will be provided by sensing transmitters and receivers which may be capable of enhanced sensing operation.

This use case is distinguished from previous use cases in [7.3x] not only by the 6G mobile network sensing using more advanced and reliable sensing measurements but also due to sensing transmitters and receivers being able to provide enhanced sensing measurements with higher QoS and performing to higher KPIs that enable the application server to provide enhanced or higher-resolution topographical maps.

### 7.3.2	Pre-conditions

An application server capable of producing topological maps from mobile network sensing measurements and data and other non-mobile network sensing data is connected to a mobile network.

The mobile network supports the application server through the acquisition of sensing measurements and data from its connected 6G capable sensing transmitters and receivers, through the authorisation of the application server and the identification of suitable sensing transmitters and receivers capable of supporting the application servers request for high accuracy sensing assistance data. The application server request may include an indication of a required sensing measurement accuracy in order to ensure production of high-resolution topological maps.

### 7.3.3	Service Flows

An application server used for the production of topographical maps, activates a service request for mobile network to provide sensing measurement data of an enhanced QoS, in accordance with the required measurement accuracy, to support the production of high-resolution topological map for a specific area.

The mobile network receives the server request and identifies for a specified area the authorised and authenticated mobile network sensing transmitters and receivers capable of providing 6G sensing measurement data to support the enhanced QoS in accordance with the application server request.

The mobile network configures the identified 6G sensing measurement transmitters and receivers for the enhanced sensing measurement and with the sensing mode e.g. monostatic, bi-static or multi-static, in order to obtain the sensing measurements in accordance with the requested service enhanced QoS. The 6G sensing measurement data is collected and transferred via the mobile network to the application server, for the production of high-resolution maps for the specific area and for a time period as specified.

### 7.3.4	Post-conditions

The application server processes the mobile network sensing data produced from the selected mobile network sensing transmitters and receivers, to produce high resolution topological map.

In one use of the high resolution topographical map, the map is forwarded to vehicles capable of high levels of autonomous driving capability e.g. L3-L5 autonomy [7.3z], where higher levels of topological map accuracy are required.

Production of high resolution topographical maps can be used for many other services e.g., civil engineering, for outdoor pursuits, assist emergency services, road traffic control, hazard prevention, etc.

### 7.3.5	Existing features partly or fully covering the use case functionality

Combining non-3GPP generated High Definition (HD) sensing measurements with mobile network measurements e.g. [7.3x] use case 5.28, may provide some limited ability to produce topological maps. However, the  resolution, repeatability and complexity in achieving and maintaining such a solution is unreliable. In particular it addresses challenges with the reliability and repeatability in configuring, synchronising and processing data from both the mobile network sensing results with sensing results from other disparately connected HD sensors of varying sensing types, in order to produce reliable results whilst minimising impacts on service quality and latency.

### 7.3.6	Potential New Requirements needed to support the use case

New functional requirements are:

New Performance requirements are:

[PR 7.3.6-1] The 6G system shall be able to support the following KPIs.

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| High topology mapping | Outdoor | N/A | [0.10] | [0.10] | [0.12] | [ - ] | [0.4] | [≤ 0.6] | [50] | [≤ 0.2] | [≤10] | [<1] |
| NOTE 1: KPIs derived from FFS (reference to be confirmed) |  |  |  |  |  |  |  |  |  |  |  |  |

## 7.4	Use case on low-altitude UAV supervision

### 7.4.1	Description

With the development of UAV technologies, light and small civilian UAVs have played a great role in aerial photography, agriculture, mapping and other fields. Various commercial UAV applications are now becoming a reality. These UAVs typically operate at low altitudes and may produce a series of safety control problems, e.g., UAV illegal intrusion and UAV collision. Thus, how to realize the low-altitude UAV supervision is important and challenging in 6G.

In some scenario, these UAVs may follow carefully planned routes that ensure efficient, regulated, and safe operation in designated airspace.  To perform tasks like package delivery, surveillance, or environmental monitoring, commercial UAVs operate based on pre-determined flight paths that dictate their altitude, speed, and direction. For instance, a UAV delivering goods will follow a direct route from the dispatch location to the recipient, while a UAV assigned to environmental monitoring will travel from its station to a specific target area for data collection. Route design and optimization are crucial for safe and efficient UAV operations. These flight routes which are approved by UAV operators, prioritize the shortest flight path, avoid restricted airspace, and ensure safe distances from obstacles such as buildings, trees, or other UAVs. Following a strict route minimizes the risk of accidents and enhances the reliability of UAV services.

Although commercial UAVs are equipped with sensors to assist with real-time navigation, these sensors can be affected by environmental conditions like lighting, weather, or geographical obstructions. Such limitations can impair a UAV’s ability to accurately determine its position, altitude, or velocity, which may lead to deviations from the approved flight path. Furthermore, for services such as good delivery, where a number of UAVs are involved in a given area, UAV collision might happen due to sensor limitations and lead to safety issues.

The existing UAV tracking technologies, such as ground-based radar systems and dedicated surveillance equipment, provide route monitoring. However, the widespread deployment of these systems faces challenges due to high installation and maintenance costs and limited availability of suitable installation sites.

The existing UAV tracking technologies, such as ground-based radar systems and dedicated surveillance equipment, provide route monitoring. However, the widespread deployment of these systems faces challenges due to high installation and maintenance costs and limited availability of suitable installation sites.

As illustrated in Figure x.1.1-1[7.4x], user equipment (UE) connected to 6G Radio Access Network (RAN) entities can be configured to support sensing operations. This configuration enhances sensing coverage, provides additional positioning reference points for sensing measurements, and improves the accuracy and reliability of sensing results. These improvements are due to the higher density of UEs compared to base stations, which increases the likelihood that some UEs are positioned closer to the UAV than the 6G RAN entities (for example, with a UAV located between two 6G RAN entities and a UE located directly beneath the UAV). Additionally, certain UEs may be placed in reflection directions that provide a larger radar cross section (RCS) for the UAV, taking into account the UAV's RCS variations in different incident/reflection angles.

Editor’s note:  Insert correct figure number referenced above.

The 6G sensing processing unit can gather sensing data from one or multiple network infrastructures. Upon request, the 6G network operator can provide UAV flight trajectory tracking services to trusted third-party applications, such as UAV service operators, regulatory agencies, Uncrewed Aerial System Traffic Management (UTM) systems, UAV itself, etc.

Figure 7.4.1-1: Low-altitude UAV trajectory tracing by 6G system

Thus, how to realize the low-altitude UAV supervision (e.g. UAV intrusion detection, UAV trajectory tracking) is important and challenging in 6G.

Sensing is an efficient technology for object detection by means of 6G radio signals, e.g., monitoring UAV illegal flying in a specific area. 6G network could provide sensing service by collecting sensing data, transmitting sensing data, processing sensing data, storing sensing data and support sensing result exposure to the third application platform. These sensing data has the following characteristics:

-	These data may not necessarily belong to a specific UE while these data may be produced by the relationship between the network and the physical environment.

-	These data may be produced by a UE or a base station in order to complete a specific task which needs multi-dimensional cooperation.

-	These data may have relationship with time and space which needs efficient on-demand transmission, storage and collection.

-	These data may be collected from non-3GPP sensing sources which needs unified management in 6G network.

Facing these new data characteristics, the 6G network capability of data processing needs to be extended based on the 5G network including multi-source heterogeneous data collection, efficient and guaranteed large-scale data transmission, efficient data processing within the network, and unified data storage to support multi-node cooperative sensing and multi-node information convergence.

In the low-altitude UAV supervision scenario, the 6G network could be used for sensing the UAV intrusion such as UAV illegal flying in a restricted area including government and company region. In this scenario, the network security and data security need to be guaranteed due to the privacy issue. Furthermore, the historical data may be used to identify an illegal UAV. In addition to the communication property of the detected UAV itself, the data management in this low-altitude sensing scenario must be operated within the network.

In a word, 6G network will break through the "pipeline" capability and go towards to a new type of information service network by realizing diversified data collection, transmission, processing, storage and exposure within the core network.

### 7.4.2	Pre-conditions

In this use case, a UAV Operator/UTM provides package delivery services within an area covered by a 6G network.

Network operator NN provides 6G sensing service for UAV flight assistance service, including illegal UAV intrusion, UAV flight trajectory tracing, UAV collision prediction and etc.  NN can make use of wireless base station to sense the airspace within their coverage area and report the sensing information (including tracked UAV and the environment around the UAV) to the USS (Uncrewed Aerial System Service Supplier)/UTM (Uncrewed Aerial System Traffic Management).

The Company MM uses the USS/UMT to supervise the low-altitude UAVs and manage potential illegal intrusion into the restricted areas. MM has proved its restricted area information to the USS/UMT.

The USS/UMT uses 6G sensing service provided by the 6G network operator NN to detect potential UAV illegal intrusion and UAV collision prediction.

The UAV Operator/UTM provides specific details to the 6G network operator NN, including the characteristics of the UAV that will be tracked, along with details about the time and location for flight tracing. This information includes regulated flight paths as well as potential areas where the UAV might temporarily deviate from its route.

The 6G network operator NN can realize 6G data collection, 6G data transmission, 6G data processing, 6G data storage within the network, and providing sensing results to the USS/UMT/UAV.

### 7.4.3	Service Flows

For illegal UAV intrusion:

1.	The Company MM requests 6G sensing service for illegal UAV intrusion in the restricted area from the USS/UMT.

2.	The USS/UMT transmits the request to the 6G Network operator NN.

3.	The 6G Network operator NN selects the base stations located in the restricted area to collect and process initial sensing data by collaborative sensing. The selected 6G base station constantly collects sensing data of the location of UAVs near the restricted area and sends the sensing data to the 6G core network with a defined frequency to obtain the sensing result (i.e., the distance between the UAV and the border or motion trail).

4.	The 6G Network aggregates, forwards and transmits the data generated by the base stations, and processes the data to obtain the sensing results.

5.	The 6G Network exposes the sensing result to the USS/UMIT. The USS/UMIT could trigger to send warning messages to the UAV or intercept the illegal UAV directly based on the sensing results.

For UAV flight trajectory tracing:

1.	When the scheduled time for tracking begins, the 6G network operator activates the UAV trajectory tracing service within the designated area until the tracking session ends. The UAV operator then launches UAV#1, which takes off from the delivery source and heads toward the destination, following a pre-set flight path.

2.	Using radio sensing, a network of 6G base stations and connected devices (UEs) detect UAV#1 and continuously gather data on its position and movement, such as distance, velocity and angle. These metrics, also known as 3GPP sensing data, are sent to a 6G processing unit for real-time analysis.

3.	During the flight, if UAV#1 leaves the coverage range of one base station and enters a new coverage zone, the 6G system could let the old base station stop radio sensing, and switch to new base station for sensing UAV#1 until it is out of coverage. This transition is based on the UAV’s estimated position and velocity, which the 6G processing unit calculates. The network can automatically adjust the sensing operations at base stations depending on this data or based on a pre-defined time frame. In certain cases, sensing handover may be triggered to maintain continuous coverage. For instance, if the current base station's connection weakens or if another nearby base station can offer better coverage for UAV#1, the system proactively shifts the sensing function to this new station to ensure uninterrupted tracking.

4.	The 6G processing unit can aggregates sensing data from multiple sources, including RANs and UEs, to estimate UAV#1’s location and velocity. Similar approach can also be applied to UAV#2 - UAV#N, in the case there are multiple UAVs providing service in the area. This real-time information is then transmitted to the UAV operator and/or UTM, who monitors the UAV’s trajectory.

5.	If UAV#1 - UAV#N deviate from prescribed route, the UAV operator and/or UTM receives alerts, allowing them to take corrective action and redirect the UAV as necessary.

### 7.4.4	Post-conditions

For illegal UAV intrusion: The illegal UAVs are away from the restricted area. Potential privacy risks are avoided. Thanks to the wide-area and constant sensing capability of the 6G base station, and the efficient data transmission, processing and storage by the 6G core network, the safety supervision of the low-altitude space of Company ‘MM’ is improved.

For UAV flight trajectory tracing: UAV#1 follows the tracked flight route to deliver the package to its destination, or any off-route movements are detected.

For UAV collision prediction: UAV#1 flies efficiently to its destination.

### 7.4.5	Existing features partly or fully covering the use case functionality

In TS 22.137 [7.3y], there are requirements specified for 5G system on integration of sensing and communication.

### 7.4.6	Potential New Requirements needed to support the use case

[PR-7.4.6-1] Subject to operator policy, the 6G network shall enable the base station to send sensing measurement data to the core network, and enable the core network to aggregate, collect, process, and store sensing measurement data from base stations.

[PR-7.4.6-2] The 6G system should support energy-efficient sensing operations.

[PR-7.4.6-3] The 6G system should provide mechanisms to ensure sensing service with a given sensing system capacity.

NOTE:    The term ‘sensing system capacity’ is the maximum number of targets that can be detected per unit area given sensing quality of service (QoS) requirements per target, which include localization accuracy and sensing service latency [7.4y].

## 7.5	Use Case on Environment Object Reconstruction

### 7.5.1	Description

Environmental object reconstruction offers significant potential with great societal and business impact, representing opportunities across wide range of sectors, from smart factories, homes and transportation to components of/entire smart cities and countries. The 3GPP ISAC provides a non-invasive dual-functionality of communication and sensing data collection of environmental features for reconstruction. This sensing data collection is expected to unlock new services and support enhanced performance, deployment utilization, energy, and spectral efficiencies due to the nature of ISAC and also its wide-area coverage and radio-signal availability. For static environmental objects, such as outdoor building/city construction and indoor machinery/wall, 3GPP wireless sensing is effective to provide a wealth of low-cost environmental information for both wide-areas and certain areas. Additionally, for monitoring dynamic targets of interest, such as vehicles, this is essential for enhancing environmental perception, especially in scenarios where non-line-of-sight (NLOS) conditions or poor visibility may limit traditional sensing methods.

The use case of environmental object reconstruction in 3GPP demands finer characterization of surrounding targeted objects based on wireless sensing signal to facilitate industrial innovation. Some applications include:

- Smart Transportation: The impact of autonomous driving is expected to be significant in terms of safety and comfort, and potentially high efficiencies with respect to traffic, logistics, energy, etc. Such application requires advanced knowledge of moving target detection and its trajectory with detection-to-track association [7.5r1], within a multi-object tracking context, including perception of micro-features of the surrounding objects such as micro-Doppler effects, precise classification, and accurate dimensions/orientations, etc. Rough localization of these objects is insufficient for ensuring transport safety and public confidence. Therefore, environmental object reconstruction by the 3GPP wireless sensing is to provide assistance for interpreting the complex traffic scenario and making rapid decisions.

- Smart City: a smart city can demand survey/digital twinning for a component/system of an entire city, which has motivated a number of smart city initiatives [7.5r3] to capture dynamic nature of society. Initially based on cameras, these digital twin models, can be greatly enhanced by environmental object reconstruction enabled by 3GPP wireless sensing

- Smart Home: there are increasingly interests for innovative application of smart home, e.g. for fall detection, provided with better sensing privacy protection and reliability. Environmental object reconstruction at home is expected for finer sensing information of home objects in order to improve service stability for better consumer experience.

Typical environmental objects to be reconstructed can include the following:

Table 7.5.1-1: Representative dimensions of an environment object

| Object Type | Dimensions |
| --- | --- |
| Building | Approximately ~400m x ~200m x ~20m (L x W x H), static |
| Vehicle | Truck: 13mx2.6mx3m (LxWxH), up to 140km/h |

The 3GPP ISAC is expected to offer significantly higher precision and resolution in sensing, and more detailed characteristics of an environment object in the spatial domain.

### 7.5.2	Pre-conditions

Map provider A is a third-party service provider which can render and virtualize detected/tracked surrounding environmental objects within its application, offer real-time 3D virtualization of objects (e.g., via 3D glasses), and display alerts for object-related warnings and information. Examples of alerts include “a car is approaching from the left street corner in 3 seconds".

Good partnership and cooperation are established between Map provider A and Mobile operator B in City C. Requested by Map provider A for sensing service, suitable sensing transmitter and/or sensing receiver  deployed in City #C are selected by Mobile Operator B to constantly sense environment objects of City #C including building and vehicle. The sensing signal emitted from sensing transmitter arrives at an environment object whose micro-objects will reflect/diffract the signal to be detected by selected sensing receivers.

NOTE:	For the ease of elaboration, base station or UE is acting as sensing transmitter and/or sensing receiver. Other sensing modes are not excluded and can be useful for environmental object reconstruction.

Charlie is a subscriber of Mobile operator B, and also a subscriber of Map provider A for real-time 3D virtualisation of his surrounding objects. Charlie would like to navigate unfamiliar city streets while wearing 3D glasses, which display surrounding objects in real-time, for his interest and safety.

### 7.5.3	Service Flows

Figure 7.5.3-1

Step 1: Charlie is a tourist, who is driving a car to enjoy the view around her in City #C. He would like to navigate unfamiliar city streets while wearing 3D glasses, which display surrounding objects in real-time, for her interest and safety. Map provider A initiates a sensing request to Mobile Operator B for the latest information of environment reconstruction in the vicinity of Charlie, who is the subscriber of both Map provider A and Operator B. The sensing objects, required by Map provider A, include vehicles, city architecture, etc, around Charlie up to a certain range.

Step 2: Mobile Operator B selects and configures the sensing transmitter and sensing receiver, and any applicable non-3GPP sensors, to facilitate the sensing operations. Both 3GPP and non-3GPP sensing data will be collected, aggregated, and processed by Operator B’s network in accordance with environment reconstruction requirements.

Step 3: Based on environment reconstruction requirements, operator B is expected to preform further sensing operations dedicated to each individual object, to provide fine characteristic, in order to determine object type/dimension, monitor micro-motions like car turning, etc. Corresponding sensing operations may require cooperative and dedicated sensing resource allocations across 3GPP system for Charlie. Thereafter sensing results of each object are delivered to Map provider A.

Step 4: The sensing results generated by Operator B, with more detailed characteristics of environment objects, are exposed to Map Provider A. This data enables Map Provider A to update Charlie's localized map, reflecting changes such as ongoing construction of university campus, shapes/dimensions of targets, and real-time tracking of movement direction/speed etc.

Step 5: Charlie receives a real-time hazard (e.g. a car is approaching from the left street corner in 3 seconds) display from Map Provider A virtualized in his glass, tailored to Charlie's trajectory. This information is transmitted through Operator B’s network, adhering to pre-defined latency requirements.

Step 6: Luckily Charlie has a chance to visit City# C again. Mobile Operator B keep detecting and tracking ongoing renovation by sensing for local university campus including new classroom buildings. During his visit, such renovation is completed roughly. By partnering with Mobile Operator B, Map Provider A can integrate and reconstruct the buildings into the real-time 3D map. Charlie decides to pay a visit because of his interest and unique building design.

### 7.5.4	Post-conditions

User Charlie can safely travel in a new city with enjoyable information associated with his surroundings and necessary travel advice.

### 7.5.5	Existing features partly or fully covering the use case functionality

3GPP TR 22.837 has described use cases to monitor micro doppler effect by ISAC caused by chest rise/fall during sleeping when detecting the doppler. The sensing results are represented by human respiration rate.

Editor’s note:  reference is needed for the TR referenced above.

In this use case, 3GPP ISAC is expected to detect and track more comprehensive characteristics of individual environmental object, e.g. for a building, vehicle, robot, etc, with sufficient and accurate sensing information per object type.

### 7.5.6	Description

[PR-7.5.6-1] The 6G system shall be able to provide sensing service with higher resolution and accuracy as described in the following table.

Table 7.5.6-1

| Object Type | Localisation accuracy (Vertical/Horizontal) | Velocity accuracy (Vertical/Horizontal) | Missed detection | False Alarm | Refreshing rate |
| --- | --- | --- | --- | --- | --- |
| Building | [1~10m] | NA | FFS | FFS | FFS |
| Vehicle | [0.5~1m] | [0.5~5 m/s] | FFS | FFS | FFS |

Editor’s Note:	 the KPI values for this use case is FFS.

## 7.6	Use Case on road digitalization

### 7.6.1	Description

Many transportation and urban applications require real-time and citywide traffic flow estimation, which is the basis for transportation planning and traffic control. Estimated traffic flow is generally represented by the number of cyclists/vehicles/pedestrians passing a reference location per unit of time and can be virtualized by a third-party application.

In order to enable real-time navigation for automatic driving or traffic flow monitoring, it is necessary to define the upper limit of sensing object detection/tracking required by a 3GPP sensing service, since any 3GPP based sensing detection/tracking coexists with communication services and is not cost-free for operators by sharing. 3GPP system shall allocate limited but sufficient spectrum resources, power, base stations and/or road side units and etc for sensing operations. Such kind of necessity can be represented by sensing target density

For example, the number of vehicles that needs to be simultaneously detected and tracked at a crossroad may be up to 1000 cars per [km2], including all stationary and moving vehicles occupying that cross-road temporally:

NOTE 1: it is assumed that the number of lanes for a major crossroad could be 8 for each direction with 3.5 meter lane width, and that the number of vehicles per lane for the purpose of tracking is 5.

### 7.6.2	Pre-conditions

Good partnership and cooperation are established between Traffic Department A and Mobile Operator B. Traffic Department A subscribes the 3GPP wireless sensing service from Operator B for the real-time road digitalization.

In order to monitor traffic volume constantly, Mobile Operator #B have deployed and activated base stations and road side units around traffic intensive areas, such as crossroad, urban road and highway, in order to provide a wide sensing coverage and capability to detect and track all moving objects, including vehicles, bicycles and pedestrians effectively.

NOTE:	For the ease of elaboration, base station is acting as sensing transmitter and/or sensing receiver in this case. Other sensing modes can also be feasible and useful as well.

### 7.6.3	Service Flows

Figure 7.6.3-1

Figure 7.6.3-2

1.	Traffic Department A receives a lot of complaints from local residents about heavy traffic at crossroads and highways due to ongoing city construction or bad route planning.

2.	Traffic Department A requests Mobile Operator B to detect and track at least [100] sensing objects simultaneously including cyclists, pedestrians and vehicles at crossroads shown in Figure 7.6.3-1, and at least [50] sensing objects simultaneously including high-speed vehicles at overpasses shown in Figure 7.6.3-2. According to preferred sensing target density, base stations and road side units deployed by Operator B along these areas constantly monitor the road situation by appropriate sensing operations according to these requests.

3.	Mobile Operator B collects 3GPP sensing data from base stations and road side units, and non-3GPP sensing data from sensors (e.g. radar, cameras) if available. Then Mobile Operator B provides Traffic Department A with real-time traffic flow information, including vehicle volume of different directions within a unit time (e.g. every 10 minutes, every half hour), average queuing time for a vehicle/pedestrian as the crossroad, etc.

4.	For more demanding traffic monitoring, Traffic Department A requests Mobile Operator B to detect and track more objects simultaneously at given crossroads and overpasses during rush hours. Mobile Operator B adapts to the request by updating the configuration parameters of sensing operations.

5.	Based on sensing results exposed by Mobile Operator B for road digitalization, Traffic Department A figures out solutions to improve the utilization of public roads at real-time by finer spatial/temporal traffic control and traffic divergence.

### 7.6.4	Post-conditions

Thanks to the network-wide coverage, the bird’s-eye-view of base station, road digitalization is enabled by capturing real-time information of the road environment.

### 7.6.5	Existing features partly or fully covering the use cast functionality

The capability to provide secure means for a trusted third-party to request the sensing target density object tracking is not covered.

### 7.6.6	Potential New Requirements needed to support the use case

[PR-7.6.6-1]	Subject to regulation and operator’s policy, 6G system shall efficiently support sensing target density requested by the third party for a given sensing service.

Editor’s Note:	The definition of sensing target density and whether communication quality will be taken into account in the above PR is FFS.

# 8	Ubiquitous Connectivity

Editor's Note: "Ubiquitous Connectivity" is intended to enhance connectivity to bridge the digital divide and enhance user experience, to address presently uncovered or scarcely covered areas, particularly rural, remote and sparsely populated areas, and indoor connectivity

## 8.1	Use case on Ubiquitous and Resilient Network

### 8.1.1	Description

The Ubiquitous Network use case is focused on delivering Mobile Broadband connectivity to every user across the globe, eliminating any “white zones”. This includes ensuring network access in all locations, such as remote areas, regions with challenging geographical conditions (e.g., mountains, forests), airspace for aerial operations (e.g., drone operations), as well as the open ocean.

This ubiquitous network will be realized by using both Terrestrial Networks (TN) and Non-Terrestrial Networks (NTN) (including satellites, High-Altitude Platform Stations (HAPS), air-to-ground networks, and unmanned aerial vehicles (UAVs))  in a transparent manner for the end user, fulfilling the required Quality of Service (QoS) criteria, such as bit rates and latency, to support daily needs.

While most users in these areas may not experience services with the highest performance levels (e.g., no Ultra-Reliable Low-Latency Communication (URLLC) or the most immersive experiences), they will be able to access a wide range of services with dependable connectivity, including high-quality voice or video streaming.

The Ubiquitous Network will ensure that every person on Earth can access the Internet and its most common applications, while also enabling the provision of new services, such as digital health services. Remote medical consultations will become feasible in areas without adequate medical infrastructure. Educational institutions, such as schools, could also leverage more advanced applications for remote virtual education.

Finally, the supporting of TN with NTN ensures that the network can deliver guaranteed connectivity under all conditions, maintaining a minimum level of service even during a crisis. This network resilience will be critical for emergency services in challenging weather conditions, such as storms (where network elements might be compromised), or during natural disasters like floods, earthquakes, tsunamis, human-induced catastrophes (e.g., conflicts), or other events that currently cause network outages.

Table 8.1.1-1: Potential sustainability impacts of the use case

|  | Sustainability Handprints (benefits) | Sustainability Footprints (costs) |
| --- | --- | --- |
| Environmental | Preventing travel-related emissions as a result of remote access to services Enabling collection of environmental data for Earth monitoring Access to precise status information can enable precision farming practices (agriculture, aquaculture) to reduce the use of fertilizers, pesticides, and fresh water | Increased material and energy use to build this ecosystem (e.g., devices, base stations, satellites) Increased land use (e.g., networks, data centres) could potentially impact existing habitat, and thus, biodiversity Potential increase of e-waste on land, oceans and space if not handled properly (e.g., collection, 100% biodegradable) |
| Social | Providing everyone access to the digital ecosystem Providing access to digital services to all (e.g., entertainment, education, transactions, voting, etc.) à Bridging the digital divide Contribute to making networks reliable (perception), everywhere Enhanced trustworthiness of digital services (availability and accountability) Delivering increased resilience in networks, crucial in unexpected events (e.g., natural disasters) Enhanced healthcare and education to reach new areas Increases food yield from enhanced agricultural management New job opportunities | Potential digital divide for people with functional variation / ageing population / IT literacy if all services are meant to be handled digitally Potential risk to privacy if all services are meant to be handled digitally Potential mental health problem due to possibility of always being connected and being traceable |
| Economic | Improved economic resilience from wider availability of connectivity for different services Efficiency improvements from the reuse of resources Economic benefits (profitability) to society/country from combining ubiquitous network with other use cases (e.g. in public services) | Profitability challenge from the network investment: to make digital services available to everyone, prices must be “affordable” which risks the economic/financial return of this use case. Resilience challenges from the maintenance of network |

### 8.1.2	Pre-conditions

Alex is living in a mountainous area with challenging access conditions. He has registered UEs (e.g., smartphone, tablet, computer) for communication and the registered to the network can provide the wide coverage with TN and also NTN.

### 8.1.3	Service Flows

1.       Alex is located in a mountainous area with difficult access conditions.

2.       The network ensures the continuous connectivity between the terrestrial and non-terrestrial networks (TN and NTN).

3.       Alex connects to the consultation using a compatible device (smartphone, tablet, computer), with sufficient video quality to enable the doctor to diagnose common health problems.

4.       Throughput and latency requirements are less stringent than for URLLC communications, enabling smooth consultation even under variable network conditions.

5.       Advanced security mechanisms are in place to protect data integrity, confidentiality, and availability across the network.

Figure 8.1.3-1: Ubiquitous Network: "Connectivity at Remote Locations”

### 8.1.4	Post-conditions

Thanks to the ubiquitous network, Alex can now benefit from a wide range of services provided by the network. Alex can engage in virtual consultations with doctors. The video quality is sufficient for diagnosing numerous common health issues.

### 8.1.5	Existing features partly or fully covering the use case functionality

The following functionalities, described in the 3GPP TS 22.261 “Service requirements for the 5G system” v19.7.0, can be included in the list of requirements necessary to provide this use case:

- Network Slicing (Clause 6.1.2),

- Multiple Access Technologies (Clause 6.3.2),

- Priority, QoS, and Policy Control (Clause 6.7.2).

- Connectivity Models (Clause 6.9.2),

- Extreme Long Range Coverage in Low Density Areas (Clause 6.17.2)

Editor’s note:  referenced is needed for TS referenced above.

### 8.1.6	Potential New Requirements needed to support the use case

KPIs

Editor's note: KPIs are FFS and need to be further justified.

Table 8.1.6-1: Ubiquitous Network KPIs

| End-to-end latency [ms] | Reliability [%] | User experienced data rate [Mb/s] | Connection density [devices/m2] | Coverage [%] | Mobility [km/h] | Location accuracy [m] |
| --- | --- | --- | --- | --- | --- | --- |
| [10-100] | [99.9 –99.999] | [0.1 –25] Downlink/ [2] Uplink | [0.1] | - Up to [10-15] km range (cell radius) for TN - [99.9]% earth human environment on earth area coverage with integrated networks | up to [120] | Low (≈ [10]) |

## 8.2	Use case on enhanced user experience with sparse LEO satellite deployment

### 8.2.1	Description

Different from terrestrial cellular and GEO deployment, the LEO satellite has a moving coverage which requires a new deployment thinking. A satellite operator may have a launch plan for a large size LEO constellation. However, the deployment of complete LEO constellation is a long-term task which needs several years or even longer. The deployment is both time consuming and costly. According to the target service area, a satellite operator may choose the sequence of the deployment of satellites, which helps to provide the service to the user of target service area as early as possible.  The satellite operator can start the commercialization of the constellation once the number of the satellites is sufficient instead of waiting until the whole constellation has been deployed completely. Even with the un-completely deployed constellation, the satellite operator should guarantee the uninterrupted service for the users. To achieve this, each satellite of the constellation should provide larger coverage compared with the case that the LEO satellites are densely deployed.

### 8.2.2	Pre-conditions

Operator A is a satellite operator with launch plans for a large-size constellation of LEO satellite.  Alice has subscribed the UE direct to the satellite service of Operator A and her smartphone is capable to directly connect with Operator A’s satellites.

### 8.2.3	Service Flows

- Operator A plans to deploy a large-size LEO constellation step by step.

- Before the deployment of the large-size LEO constellation is complete, Operator A decides to provide connectivity service to users when the sparse constellation with sufficient satellites (e.g., several hundreds) for the target service area has been constructed. Due to the sparse deployments, each satellite needs to support sufficiently long-rang coverage.

- Alice is going on a hiking in a mountain area with her smartphone. She noticed that there is no terrestrial network coverage and her smartphone has switched to the Operator A’s satellite network. When she is on the way, she picks up a call from her mother. She talks with her mother for serveral minutes. During the call, there is no interruption since the satellites provide the continuous coverage.

- After walking for a long time, Alice takes a break, meanwhile she browses the web to search some information on the hiking route using her smartphone. There is also no interruption during the internet access. After the break, Alice continues the hiking.

### 8.2.4	Post-conditions

Thanks to the support of service using LEO satellites access with spares satellite deployment, Operator A can start the commercial usage of the constellation even though the constellation is sparse, while the users can have a good experience of the satellite communication in the constellation’s early deployment stage.

### 8.2.5	Existing features partly or fully covering the use case functionality

Clause 6.46 of TS 22.261 defines the requirements for 5G systems with satellite access. Clause 7.4 of TS 22.261 defines the KPIs for 5G systems with satellite access. This use case proposes new requirements for satellite access in case of sparse LEO satellite constellation.

Editor’s note:  referenced is needed for the TS above.

### 8.2.6	Potential New Requirements needed to support the use case

[PR-8.2.6-1]: The 6G system with satellite access shall optimize the coverage per LEO satellite.

## 8.3	Use case on “service continuity for wearable mobile devices”

### 8.3.1	Description

Reelika has just started a famous ultra-trail “Grand Raid des Pyrénées” in the Pyrénées mountains between France and Spain. She is equipped with wearable mobile devices including a wrist watch, a forehead mounted camera and a smart phone in order to share in real time her position, her health conditions and possibly voice message or short videos with her friends Guillaume and Didier in Paris but also with her coach Loïc waiting for her at a check point in the middle of the race in a remote area without mobile access coverage. Reelika expects that her personal data (e.g., position, health conditions, etc.) are protected.

As she starts, her mobile device is first served by the mobile access but then when climbing the first mountain, she reaches the edge of the mobile access coverage. Throughout the race, her wearable devices are transferred between mobile and satellite access whenever the mobile access coverage disappears.

Editor’s note:  consistency of terms – mobile access coverage = TN access?

Her friends don’t perceive the transition between both access technologies (i.e. no packet loss when non real time service and no interruption for real time service). However, they may perceive that the quality of the image/voice is adapted to varying available bandwidth and latency.

The race organisers as well as all her friends including Loïc, keep receiving regularly the position and health conditions of Reelika. At some point Loïc detects that Reelika is getting very tired and cannot progress. He decides to send her a short message to cheer her up. Reelika wearable mobile device receives the text message and convert it to voice. Reelika listens to the message and thanks him with a video message.

During the night, Reelika initiates an emergency call. Reelika reports to the race organisers that she is with a runner who injured himself while accidentally falling in a pit. The race organisers detect that Reelika may have crossed the Spanish border and launches a request for reliable positioning service in order to determine whether the French or the Spanish public safety organisation should intervene. Reelika’s reported position is confirmed to be in Spain and hence the Spanish public safety organisation is alerted who takes over the Reelika call. The on-going emergency call is extended to a Spanish first responder who fortunately is not far from the accident. He is able to reach the spot within a quarter of an hour, thanks to complementary guidance from Reelika.

Editor’s note:  consistency of terms – emergency call to third party and multi-party emergency call in Service Flow clause?

Reelika is able to continue her race leaving the injured runner in good hands.

As Loïc is monitoring the progress of Reelika, it starts raining and Loïc decides to enter the large shelter where the runners will find some assistance. The rain is so dense that the race organisers decide in coordination with the public safety to alert the runners and all the followers along the track about possible slippage of terrain. An alert message is broadcasted to the area with the intent to reach the maximum number of persons, including the ones in indoor conditions. With this alert message, public safety requests all runners to report their position every 10 minutes during the next two hours. Loïc (indoor), Reelika and all race participants/followers in the area receive this alert message.

At a certain point in the race, Reelika sets off on a bad track, due to the de-tagging of local activists. The monitoring system of the race organizer quickly detects that she is going off the planned track. She is immediately alerted with an alert message that triggers an alarm sound in her wrist watch. She consults in her wearable device where she lost the planned track and how she can rejoin it.

The race organizers inform the race participants behind Reelika about the virtual track to rejoin the planned track as well as in field member of the race organization to correct the track. This prevents late runners to lose time with this track issue and no one need to be sent after lost runners.

Fortunately, Reelika can successfully complete her race without further technical incident and all her friends join in a video conference to celebrate with her.

### 8.3.2	Pre-conditions

Reelika’s wearable mobile devices are served by the mobile access

### 8.3.3	Service Flows

From Reelika (mobile or satellite access) to the race organizers (mobile access), to Guillaume/Didier (mobile access) and to Loïc (satellite access): messaging (including reported position), non-real time and real time services

From Reelika (mobile or satellite access) to Loïc (satellite access): messaging services

From Loïc (satellite access) to Reelika (mobile or satellite access) messaging services

From Reelika (satellite access) to public safety headquarter (mobile access): emergency call (including reported position)

From race organiser (mobile access) to Reelika (satellite access): reliable positioning service (network based)

Between Reelika (satellite access), Spanish public safety headquarters (satellite access) and 1st responder (satellite access): multi party emergency call (including reported positions of both Reelika and 1st responder)

From public safety headquarter (mobile access) to Reelika (satellite access) and Loïc (satellite access, indoor): public warning

From Reelika (satellite access) to in field first responder (satellite access) and to public safety headquarter (mobile access): reported position

From race organisers to Reelika (satellite access) and following runners (satellite access): alert about off track and guidance to re-join the planned track

Between Reelika (mobile access), Guillaume/Didier in France (mobile access) and Loïc (satellite access): real time services (video conference).

### 8.3.4	Post-conditions

Reelika’s wearable mobile device is served by the mobile access

### 8.3.5	Existing features partly or fully covering the use case functionality

In TS 22.261 one can read:

Editor’s note:  referenced needed for TS above.

Clause 6.46.3	Service continuity

For a 5G system with satellite access, the following requirements apply:

- A 5G system with satellite access shall support service continuity between 5G terrestrial access network and 5G satellite access networks owned by the same operator or owned by different operators having an agreement.

- Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support service continuity (with minimum service interruption) for a UE engaged in an active communication, when the UE changes from a direct network connection via 5G terrestrial access to an indirect network connection via a relay UE (using satellite access) and vice-versa.

Clause 6.41.2.9 	Regulatory Services

A hosting network using the 5G system shall be able to support regulatory services (e.g., PWS, LI, and emergency calls), based on regional/national regulatory requirements.

- In the above, three aspects have not been clearly specified:

- Service continuity with min interruption time between satellite and terrestrial (mobile) access

- The density of UE supported by satellite access for SMS

- The support of PWS via satellite for UE also in light indoor conditions

### 8.3.6	Potential New Requirements needed to support the use case

[PR-8.3.6-1]: The 6G system shall be able to ensure service continuity with minimum interruption for UE during the transition between terrestrial and satellite access and vice versa.

[PR-8.3.6-2]: The 6G system via its satellite access, shall support SMS delivery to a high density of UE i.e. up to [1000] UE per km2.

[PR-8.3.6-3]: Subject to regulatory requirements, the 6G system using satellite access, shall be able to support PWS for broadcasting warning notifications to UE in adverse propagation conditions e.g. light indoor conditions, dense forest.

## 8.4	Use case on resilient positioning in satellite networks

### 8.4.1	Description

Positioning, Navigation and Timing (PNT) is fundamental for a wide range of sectors, such as transportation, critical infrastructure (e.g. 6G system) and emergency services. Nowadays, GNSS is the backbone technology to fulfil these PNT needs. However, the dependency of these sectors on GNSS certainly results in major threats and risks, in case of unavailability or disruption of GNSS, with severe safety, legal and economic impacts. The resilient positioning use case is based on the demand to withstand and recover rapidly from PNT disruptions anywhere.

In addition, the 6G system may assume the need to obtain the UE location with a valid GNSS solution for the operation of 6G NTN solutions. In such situation, the degradation or unavailability of GNSS positioning, such as due to jamming, spoofing or obstructions, may lead to a degradation or loss of the 6G NTN communication capability, as it may currently occur in 5G NTN operations. For 6G NTN, there may be alternative communication approaches, but if these approaches are limited or not successful, a 6G integrated communication and positioning system can provide a GNSS-independent positioning solution for 6G NTN communications.

The satellite network operator of the 6G system is able to provide the resilient positioning service, by ensuring the availability of 3GPP positioning technologies under the service area, and independently of the availability of non-3GPP technologies (e.g. GNSS). This resilient positioning service can then be exploited by any kind of 6G user (e.g. aircrafts and drones [8.4X]) that may experience GNSS degraded service.

### 8.4.2	Pre-conditions

The UE has satellite network coverage (e.g. LEO constellation) in the 6G system.

The UE can be unregistered or registered (e.g. in idle or connected mode) to the satellite network.

The UE can be under the coverage of multiple or multi-orbit satellite networks (e.g. LEO and GEO constellations) deployed by one or more operators and have subscription(s) for accessing these satellite networks.

The environment of use is outdoor static or moving with coverage provided by the satellite network(s).

The UE is equipped with a native 6G communication and positioning function. The positioning function relies on 3GPP technologies delivered by the satellite network, to avoid any dependence on the availability of non-3GPP technologies.

### 8.4.3	Service Flows

The 3GPP positioning service provided by the satellite network can be initiated by a third party or by the end-user.

This 3GPP positioning service can determine the UE location with a certain resilient accuracy level.

When triggering the 3GPP positioning service, if the UE fails to obtain the location data within a time limit or the location data is not accurate enough, the UE can select different satellite networks or improve the accuracy of location data combining multiple satellite networks.

The UE location is used to provide the resilient positioning service.

The UE can report its UE location and positioning accuracy level.

### 8.4.4	Post-conditions

The user successfully obtains the resilient positioning service.

### 8.4.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 clause 6.27.1 includes the following description:

Editor’s note:  referenced needed for TS above and throughout this clause.

5G positioning services aims to support verticals and applications with positioning accuracies better than 10 meters, thus more accurate than the ones of TS 22.071 [24] for LCS. High accuracy positioning is characterized by ambitious system requirements for positioning accuracy in many verticals and applications, including regulatory needs.

3GPP TS 22.261 clause 6.27.2 includes the following positioning requirements:

The 5G system shall provide different 5G positioning services, supported by different single and hybrid positioning methods to supply absolute and relative positioning.

NOTE 2: 	hybrid positioning methods include both the combination of 3GPP positioning technologies and the combination of 3GPP positioning technologies with non-3GPP positioning technologies such as, GNSS (e.g. Beidou, Galileo, GPS, Glonass), Network-based Assisted GNSS and High-Accuracy GNSS, Terrestrial Beacon Systems, dead-reckoning sensors (e.g. IMU, barometer), WLAN/Bluetooth-based positioning.

The 5G system shall support mechanisms to determine the UE’s position-related data for period when the UE is outside the coverage of 3GPP RAT-dependent positioning technologies but within the 5G positioning service area (e.g. within the coverage of satellite access).

However, these requirements do not ensure resilience of the 3GPP positioning technologies in satellite networks, independently of the availability of non-3GPP technologies (e.g. GNSS).

3GPP TS 22.261 clause 7.3.2.1 includes the following performance requirements:

The 5G system shall support the combination of 3GPP and non-3GPP positioning technologies to achieve performances of the 5G positioning services better than those achieved using only 3GPP positioning technologies.

NOTE 1:	For instance, the combination of 3GPP positioning technologies with non-3GPP positioning technologies such as GNSS (e.g. Beidou, Galileo, GLONASS, and GPS), Terrestrial Beacon Systems (TBS), sensors (e.g. barometer, IMU), WLAN/Bluetooth-based positioning, can support the improvement of accuracy, positioning service availability, reliability and/or confidence level, the reduction of positioning service latency, the increase of the update rate of the position-related data, increase the coverage (service area).

NOTE 2:	The combination can vary over time to optimise the performances, and can be the combination of multiple positioning technologies at the same epoch and/or the combination of multiple positioning technologies at different epochs.

3GPP TS 22.261 clause 7.3.2.2 includes the following requirements for horizontal and vertical positioning service levels:

The 5G system shall be able to provide positioning services with the performances requirements reported in Table 7.3.2.2-1.

However, these performance requirements are not fully independent of non-3GPP positioning technologies, since a combination of 3GPP and non-3GPP positioning technologies can be used to fulfil these performance requirements.

### 8.4.6	Potential New Requirements needed to support the use case

[PR-8.4.5-1]	The 6G system with satellite access shall be able to provide positioning service with 3GPP technologies, independently of non-3GPP positioning technologies (e.g. GNSS) with the following KPIs:

Table 8.4.6-1: Performance requirements for satellite-based positioning services

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |
| Airplane landing | [10] m | [10] m | [99%] | [1] s | Outdoor | Up to [350] km/h | Airplane mounted |
| UAV positioning | [1] m | [1] m | [99%] | [1] s | Outdoor | Up to [160] km/h | UAV mounted |

Editor’s Note:	Performance requirements on determining the UE location subject to regulatory requirements and operators’ policies are FFS.

## 8.5	Use case on “Disaster relief”

### 8.5.1	Description

The national meteorological centre has detected the upcoming of an outstanding storm with heavy rains which are likely going to cause a major flood in the Valencia region. The alert is propagated to the public safety organisations as well as to the population.

All first responders are equipped with a set of wearable devices (i.e. handheld, bodycam, vital signs monitoring sensors, …) and some with drones with regular and thermal (infrared) cameras.

The storm and floods created power supply cuts in the region that resulted in the loss of terrestrial cellular network connectivity. Thanks to the available satellite access, the national public safety organisation is guiding first responders that are already on site (most are volunteers among the population). Additional first responder teams are deployed each with their all-terrain vehicles or amphibious vehicles in the harbour. The remaining bandwidth of the satellite access can be used by the population to exchange messages.

Quickly, it is decided to ask assistance from public safety and disaster relief organisations from neighbouring countries.

Local access networks are mounted on each land vehicle with enabled satellite connectivity. Each local access network can be used for communication between the team members, with their headquarters (area, regional, national) as well as with other responder teams. Even civilians could exploit this connectivity if sufficient remaining bandwidth is available. When the wind burst decayed in power drones are used by team members to assess the disaster, report to headquarter(s) and rescue the population.

As the flood took place in the coastal area, casualties are being spread to the sea and therefore, public safety organisation are deploying boats each equipped with an on-board local access network and satellite connectivity.

Responders or drones may have to move beyond the coverage of a local access network. In such cases continuity of service is ensured through smooth transition to the satellite access. Several local access networks can be directly connected via satellite to ease the coordination between the teams. User equipment belonging to national first responders can seamlessly communicate with user equipment appertaining to responders of the neighbouring countries.

During the recovery phase, an HIBS (base station on board an HAPS) can be launched to increase the available capacity over the area before the terrestrial base stations are repaired.

### 8.5.2	Pre-conditions

The base stations of the terrestrial network have been flooded which created a major failure. The terrestrial network is down and “terrestrial” mobile service is not available in the area.

### 8.5.3	Service Flows

Given that the terrestrial network is down, all UEs will look for alternative available network in the area, that is the satellite access and later the HAPS based access network.

Satellite or HAPS based access network can be used to support

- Public warning service to all population (including 1st responder’s volunteers) in the impacted area (also in adverse propagation conditions such as light indoor)

- Non real time and real time services to pedestrian or drone mounted UE

- Backhaul connectivity to vehicle/boat mounted local access point

- Connectivity (without usage of satellite feeder link) between two local access points

The satellite or HAPS based access bandwidth can be pre-empted for public safety organisations but the remaining bandwidth if available may be used by the population only for messaging.

Vehicle/boat mounted Local access points can be used

- To serve pedestrian or drone mounted UE

- To support UE to UE connectivity

### 8.5.4	Post-conditions

Terrestrial network is restored and operational again

### 8.5.5	Existing features partly or fully covering the use case functionality

TS22.261 already includes the following relevant requirements:

Editor’s note:  referenced needed for TS above.

Clause 6.46.7	Satellite and Relay UEs:

- A 5G system with satellite access shall be able to support relay UEs with satellite access.

Clause 6.46.9	UE-Satellite-UE communication:

- Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support UE-Satellite-UE communication regardless of whether the feeder link is available or not.

- Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide QoS control of a UE-Satellite-UE communication.

- Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support different types of UE-Satellite-UE communication (e.g. voice, messaging, broadband, unicast, multicast, broadcast).

Clause 7.4.2	Requirements

A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency.

NOTE 1:	 5 ms network latency is assumed and added to satellite one-way delay.

A 5G system providing service with satellite access shall be able to support MEO based satellite access with up to 95 ms end-to-end latency.

NOTE 2:	 5 ms network latency is assumed and added to satellite one-way delay.

A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 35 ms end-to-end latency.

NOTE 3:	 5 ms network latency is assumed and added to satellite one-way delay.

A 5G system shall support negotiation on quality of service taking into account latency penalty to optimise the QoE for UE.

The 5G system with satellite access shall support high uplink data rates for 5G satellite UEs.

The 5G system with satellite access shall support high downlink data rates for 5G satellite UEs.

The 5G system with satellite access shall support communication service availabilities of at least 99,99%.

Table 7.4.2-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pedestrian (note 2) | [1] Mbit/s | [100] kbit/s | 1,5 Mbit/s/km2 | 150 kbit/s/km2 | [100]/km2 | [1,5] % | Pedestrian | Handheld |
| Public safety | [3,5] Mbit/s | [3,5] Mbit/s | TBD | TBD | TBD | N/A | 100 km/h | Handheld |
| Vehicular connectivity (note 3) | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | 50 % | Up to 250 km/h | Vehicle mounted |
| Airplanes connectivity (note 4) | 360 Mbit/s/ plane | 180 Mbit/s/ plane | TBD | TBD | TBD | N/A | Up to 1000 km/h | Airplane mounted |
| Stationary | 50 Mbit/s | 25 Mbit/s | TBD | TBD | TBD | N/A | Stationary | Building mounted |
| Video surveillance (note 4a) | [0,5] Mbit/s | [3] Mbit/s | TBD | TBD | TBD | N/A | Up to 120km/h or stationary (note 4b) | Vehicle mounted or fixed installation |
| Narrowband IoT connectivity | [2] kbit/s | [10] kbit/s | 8 kbit/s/km2 | 40 kbit/s/km2 | [400]/km2 | [1] % | [Up to 100 km/h] | IoT |
| Note 1: Area capacity is averaged over a satellite beam. Note 2: Data rates based on Extreme long-range coverage target values in clause 6.17.2. User density based on rural area in Table 7.1-1. Note 3: Based on Table 7.1-1 Note 4: Based on an assumption of 120 users per plane 15/7.5 Mbit/s data rate and 20 % activity factor per user Note 4a: Refer to video surveillance data transmitted (in UL) from a UE on the ground (e.g. picture or video from a camera) using satellite NG-RAN to connect to 5GC, and video surveillance-related configuration or control data sent (in DL) to the UE/device. 0.5 Mbit/s for DL experienced data rate is based on MAVLINK protocol that is widely used for UAV control. 3 Mbit/s for UL experienced data rate is based on the assumed sum from 2.5 Mbit/s for video streaming and 0.5 Mbit/s for data transmission. Note 4b: Up to 120km/h applies to vehicle mounted while stationary applies to fixed installation. Note 5: All the values in this table are targeted values and not strict requirements. Note 6: Performance requirements for all the values in this table should be analyzed independently for each scenario. |  |  |  |  |  |  |  |  |

=> What is not yet covered in 5G using satellite access are:

- Other SWAP (Size, Weight and Power) constrained moving platforms (e.g. flying drone, small vessels mounted) compared to vehicle.

- Relay UE-Satellite-Relay UE communication.

### 8.5.6	Potential New Requirements needed to support the use case

[PR-8.5.6-1]: The 6G system using satellite access, shall be able to provide eMBB service to UE mounted on SWAP (Size, Weight and Power) constrained moving platforms (e.g. flying drone, small vessels mounted).

## 8.6	Use case on “low-energy positioning in satellite networks”

### 8.6.1	Description

This use case corresponds to a capability for 6G with satellite access to provide low-energy positioning for low-power and low-complexity devices (e.g. IoT and low-power devices) when served by a satellite network.

### 8.6.2	Pre-conditions

A battery-powered IoT or low-power device has been deployed in an environment where it obtains its initial position and synchronization from non-3GPP methods (e.g., GNSS) and / or terrestrial networks. A combination of non-3GPP technologies (e.g., GNSS) and 3GPP technologies, terrestrial and non-terrestrial, can be configured if compatible with energy consumption requirements. Integrating GNSS into IoT devices operating over satellite networks significantly impacts battery consumption, especially in battery-powered UEs requiring frequent positioning updates. Mitigating the power consumption due to GNSS would therefore be beneficial. The extent of this impact varies based on GNSS start modes (hot, warm, or cold), power consumption of GNSS modules, reporting intervals, and device mobility. According to the analysis carried out in [8.6x], GNSS position fixes can reduce battery life from approximately 5% up to 45% in cases of frequent position fixes.

### 8.6.3	Service Flows

At some point, the IoT or low-power device may lose its original positioning sources due to one of the following reasons:

- GNSS-originated position: GNSS signal may be lost due to interference, or obstruction, or intentionally disabled to augment its autonomy.

- Terrestrial network-originated position: the IoT or low-power device may have moved beyond terrestrial network coverage, such as in the case of asset tracking sensor installed on a container to be transport by sea, rail, or road.

In this context, the IoT or low-power device, served by the satellite access, is expected to know an approximate position from prior positioning. This requires the IoT or low-power device to refresh a more accurate position.

Therefore, the IoT or low-power UE initiates the 3GPP positioning method over satellite to estimate its current location.

The 6G system broadcasts to the UE via its satellite access at least the following necessary information, including but not limited to:

- Network assistance data: Satellite ephemeris and additional assistance data to improve accuracy (e.g., ionospheric models to correct the atmospheric delay errors, etc.)

- Reference signal for time of arrival measurements.

### 8.6.4	Post-conditions

The IoT or low-power device successfully estimates/updates its position based on information provided by the 6G system via the satellite access.

### 8.6.5	Existing features partly or fully covering the use case functionality

Ranging signals and positioning methods for IoT and low-power applications are defined in TS 36.211 and TS 38.211 and TS 36.305 and TS 38.305, respectively, but only for terrestrial networks. They need to be adapted to satellite access context.

Editor’s note:  references needed for TSs above.

### 8.6.6	Potential New Requirements needed to support the use case

[PR-8.6.6-1]: The 6G system shall be able to provide a mechanism for UEs with satellite access to determine its position using 3GPP positioning technology only.

Editor’s note: The target horizontal accuracy and availability of this 3GPP positioning technology in the area covered by the satellite access is FFS

## 8.7	Use case on Global mobile video

### 8.7.1	Description

Many places and people are currently underserved when it comes to Mobile Broadband (MBB) services. From the user’s and society’s point of view a lot is gained already with a basic internet connection, since many internet services can be delivered with a fairly moderate bitrate, and the most important needs would be met already with a low activity factor. Therefore, the problem is mainly related to providing remote service coverage for basic MBB. Still, such basic services may be the basis of sensitive systems (e.g. related to health or surveillance) and therefore uninterrupted and resilient operation is important.

The use case Global mobile video is about provisioning access to basic broadband services, exemplified by the capability to make a video call, at remote places on earth where people live or work, using handheld-type of 3GPP devices. A full global area coverage (e.g. 99.9%) needs to be provided through NTN access. However, TN needs to handle more populated areas, for instance through very large cells, such that the total traffic to be handled by NTN is not too high.

Based on population density this can be divided into remote and deep rural scenarios.

Remote scenario
Covering virtually all people on Earth, and virtually all areas including oceans. In most cases very sparsely populated areas (approx. <1person/km2). The expectation is that this scenario is covered mainly by NTN access. A lower rate, activity factor, and area traffic can be supported in these areas. The Global Video service might be delivered with reduced quality compared to the Deep Rural scenario.

Deep Rural scenario 
A deep rural scenario represents sparsely populated areas (approx. 1-10person/km2). The expectation is that this scenario is mainly covered by TN access, from large macro cells to very large “boomer” cells, an in addition have NTN coverage to fill gaps between TN cells. A higher rate, activity factor, and area traffic can be supported in these areas.

Key value impact analysis

Energy resources: Increased energy consumption due to the buildout of networks.

Material resources: The material need increases when building out networks with new 6G sites and satellites. Material resource depletion increases as materials sent into space is considered not recyclable.

Emissions: Internet access provides possibility for various activities (e.g., bank, hospital, work) leading to less travel needs and thereby reduced emissions. Emissions related to producing and operating new equipment can be somewhat mitigated.

Biodiversity and land use: New sites, especially high tower 6G sites, increases the land use for ICT.

Education: Internet access enable access to remote educational material.

Health: Global internet could enable using remote healthcare/first aid leading to better healthcare access in rural areas.

Inclusion and Equality: Providing the possibility to internet access facilitating banking, healthcare and education services is positive from an inclusion point of view. However, important to not leave anyone behind and thereby risk widening the gap.

Trustworthiness: Increased resilience for ICT services could be provided by satellites included in the UC as back up in unforeseeable situations.

Work and income: This use case could enable the possibility of running businesses from everywhere.

Infrastructure: Increasing the access to internet is the main goal of the use case.

### 8.7.2	Pre-conditions

UE capable of connecting to NTN for access. For high accessibility this should be achieved with mainstream UE’s.

### 8.7.3	Service Flows

- User initiates or receives video call to handheld device to/from another user.

- If TN access available the traffic is supported by TN, otherwise NTN access is used.

- The call is sustained uninterrupted, also under mobility, until either user terminates call.

### 8.7.4	Post-conditions

The user is able to use uninterrupted internet services, such as video communication, regardless of position and for an extended period.

### 8.7.5	Existing features partly or fully covering the use case functionality

- 5G Advanced specifies a Non-Terrestrial Network component.

- 5G Terrestrial network.

### 8.7.6	Potential New Requirements needed to support the use case

[PR-8.7.6-1]	The 6G system shall support service continuity within NTN, and between TN and NTN.

[PR-8.7.6-2]	Following KPI’s should be achieved for the global mobile video service.

| KPIs per scenario | Requirement |
| --- | --- |
| Remote area (<1person/km2) | User experienced rate ([5Mbps], DL), ([1Mbps], UL), (reduced video quality acceptable) Area coverage ([99.9%]) Area traffic [50kbps/km2] (DL, corresponding to an activity factor of 2% with an average user density of 0.5 person/km2) |
| Deep Rural area (1-10 person/km2) | User experienced rate ([10Mbps], DL), ([2Mbps], UL) (standard video quality) Area coverage ([99.9%]) Area traffic [1Mbps/km2] (DL, corresponding to an activity factor of 4% with an average user density of 5 person/km2 equally shared by two operators) |

Editor's note: the values can be further discussed.

# 9	Immersive Communication

Editor's Note "Immersive communication" require support for mixed traffic of video, audio, haptic and other environment data in a reliable and synchronous manner, combining low latency and high data rates.

9.0	General

Immersive Communication has been described as follows by ITU-R: "This usage scenario extends the enhanced Mobile Broadband (eMBB) of IMT-2020 and covers use cases which provide a rich and interactive video (immersive) experience to users, including the interactions with machine interfaces." [9.0x].

Normative requirements for mobile metaverse services, which include immersive communication, have been specified in [9.0y] and [9.0z]. Use cases considered in this clause of the present document will consider the gap with what they proposed and existing stage 1 specifications.

## 9.1	Use case on Immersive Gaming

### 9.1.1	Description

With the growing adoption of extended reality in the 5G era, it is expected that 5G-Advanced and 6G will scale immersive experiences for the enterprise and consumer mass market. In 6G, users of immersive technologies are expected to work, play, and interact seamlessly in both the physical and virtual worlds.These immersive experiences would be enabled by leveraging advanced extended reality (XR) and multimedia features such as user-interaction via Avatar-Holographic conferencing, spatial collaboration on Hi-fidelity 3D objects, high resolution immersive 2D-3D Cloud Gaming, high resolution 360o 2D-3D content streaming and Multi-Modal AI-intermediated User Experiences.

User-interaction via Avatar or Holographic conferencing:

User-interaction in immersive experiences can be via Avatar or Holographic conferencing. This feature allows participants to engage in virtual meetings where they appear as lifelike avatars or holograms, creating a sense of physical presence despite being miles apart. By leveraging advanced motion capture and real-time rendering, users can express emotions, gestures, and body language, making interactions more natural and engaging. This form of conferencing not only enhances communication and collaboration but also breaks down geographical barriers, enabling a more inclusive and dynamic way of connecting with others.

Spatial collaboration on Hi-fidelity 3D objects:

Spatial collaboration on high-fidelity 3D objects enables multiple participants regardless of their location to work together in immersive environments. The feature enables multiple users to interact with detailed 3D models in a shared virtual space, allowing for real-time modifications and discussions. By providing a highly realistic and interactive representation of objects, users can explore every angle, scale, and detail as if they were physically present. This level of collaboration enriches immersive experiences, and is  particularly beneficial in fields such as design, engineering, and architecture, where precision and detail are crucial. It enhances creativity, improves communication, and accelerates decision-making processes, making it an invaluable tool for modern collaborative efforts.

High resolution immersive 2D or 3D Cloud Gaming:

High-resolution immersive 2D or 3D cloud gaming with rich backgrounds is revolutionizing the gaming industry by delivering stunning visual experiences directly from the cloud. This technology allows gamers to access and play graphically intensive games on various devices (e.g. handheld gaming device, Hi-resolution television or monitor, Head mounted displays) without the need for high-end hardware. By leveraging powerful cloud servers, games can render intricate details and complex environments in real-time, providing seamless transitions between 2D and 3D perspectives. The rich backgrounds enhance the immersive experience, making players feel as if they are truly part of the game world. This advancement not only democratizes access to high-quality gaming but also paves the way for more interactive and visually captivating game designs, pushing the boundaries of what is possible in digital entertainment.

High resolution 360o 2D or 3D content streaming:

High-resolution 360° 2D or 3D content streaming is transforming how we experience digital media by offering an unparalleled level of immersion and detail. This feature allows users to stream content that can be viewed from any angle, providing a fully immersive experience whether in 2D or 3D. By leveraging advanced compression algorithms, powerful cloud infrastructure, high-resolution 360° streaming ensures smooth playback and high-quality visuals, even on devices with varying capabilities. This innovation is particularly impactful in fields such as virtual tourism, education, and entertainment, where it enables users to explore environments and scenarios as if they were physically present.

Multi-Modal AI-intermediated User Experiences:

Multi-modal AI-intermediated user experiences are redefining how we interact with technology by integrating various forms of input and output to create seamless, intuitive interactions. This approach leverages AI to interpret and respond to multiple modes of user interaction, such as voice, text, gestures, and even facial expressions and video streams representing user context. By combining these inputs, AI can provide more accurate and context-aware responses, enhancing the overall user experience across all forms of 2D and 3D content consumption discussed above. Users can control actions through a combination of voice commands and gestures, with the AI understanding and executing tasks based on the context, presenting the results via hi-fidelity 2D or 3D assets.

These extended reality and multimedia features and 6G capabilities such as enhanced communication, sensing, compute and AI would enable significant improvement in immersive experiences across many fields including education, healthcare, entertainment, and enterprise collaboration, bringing us closely to vision of seamless merging together of the physical and virtual worlds.

In this contribution, we will focus on immersive gaming experiences. Enhanced Immersive gaming enables single or multiple participants in various location(s) to interact in real time with digital representation of object(s) and their surrounding environments, which are generated with hi-fidelity 3D assets and digital representation of human using avatars and/or holograms. These digital representations of object(s), environment and humans are created leveraging inputs such as visual, audio or tactile features and/or location information generated from sensors (such as cameras, RF sensors and haptic sensors) embedded in the vicinity/environment or on the participants. In addition, the interactions between entities in the immersive gaming experiences can be made more accurate, intuitive and seamless using cloud gaming and multi-modal AI-intermediated techniques.

Key Value Impact Analysis:

Economic Growth:

The demand for immersive experiences drives technological innovation in many areas including communication, extended Reality, compute and AI, with applications beyond gaming. Overall, immersive gaming contributes to industry growth, creating jobs and fostering economic activity while providing a rich and multifaceted experience.

Inclusion:

Immersive gaming offers numerous benefits, especially, social benefits such as fostering teamwork, emotional connection and a sense of community and camaraderie, irrespective of the physical distance of separation. The use case discussed below aims by no means to exclude people but instead, the goal is to include as many participants in this gaming experience as possible irrespective of their separation distance. In the case, all the participants (i.e. players, cheerleaders and spectators) are localized, further consideration is required on whether this technology is needed. Also, significant consideration needs to be made to ensure that the technologies involved simulate seamless immersive gaming as much as possible to ensure that the experience of the participants is not negatively impacted or burdensome.

### 9.1.2	Pre-conditions

The University of Nee’s basketball team is playing the last game of the tournament against a visiting team from University of Zee. This is a home game for the University of Nee so its players would be participating in person at the Nee city basketball gymnasium. However, the team from the University of Zee are flying in from another country, but due to bad weather, two players from the University of Zee, the team’s MVP,  David and another player Steve flights and would have to join the game virtually from location A, respectively.

As a result, the basketball players involved in this game are in two different locations, Nee city basketball gymnasium and location A. The immersive services at these locations are provided by Operators NeCom and ZeCom, respectively. Each location is outfitted with equipment (e.g. camera, sensors and edge computing resources) to support the immersive experience. Each player wears immersive gears including HMD/AR glasses and motion capturing devices such as haptics gloves, smart watches and compute puck/phones.

There are also dance routines performed by twelve cheerleaders physically present at the Nee city basketball gymnasium and three virtual cheerleaders at location B with the immersive experience in that location provided by Operator TeCom. Each cheerleader would also wear HMD/AR glasses and haptics gloves.

Also viewing the game are spectators (approximately one hundred people), some of whom are in-person at the Nee city basketball gymnasium and many more watching remotely. Leveraging high resolution 360o 2D/3D content streaming techniques, the in-person spectators view the game using HMD/AR glasses while the remote spectators can watch the game using the HMD, tablet, or even their phones.

### 9.1.3	Service Flows

When the game starts, all the immersive participants at the Nee City basketball gymnasium can view their surroundings, including in-person players, the cheerleaders and spectators through their immersive devices. In addition, virtual players and cheerleaders represented as avatars/holograms with accompanying movements, gestures and even facial expressions are rendered into their appropriate display devices. From the perspective of the virtual players and cheerleaders, they see themselves as participating in activities (i.e. playing basketball or dancing) in the Nee City basketball gymnasium and with background features showing highly realistic 3D digital representation of the objects (such as the court and baskets), players, cheerleaders and spectators that are present at the location.

For a truly immersive experience involving between local and virtual players, the ball is virtually created and interacted with by both in-person and remote players using gloves outfitted with haptic sensors. The movements and impacts on the ball from the players are communicated to all viewers in “real-time”.  During halftime, dance routines are performed by cheerleaders twelve of whom are local, while the remaining three are participating from location B. The movements, locations and gestures of the cheerleaders are combined virtually and displayed to the all the viewers.

In addition, the spectators can customize their viewing perspectives, such as the desirable viewing angle, location or resolution.  Spectators interact with their immersive applications using voice commands and gestures, with the AI understanding and executing tasks based on the context, presenting the results via hi-fidelity 2D or 3D assets.

The immersive experience of players, cheerleaders and spectators are enabled by cellular traffic flows that can be classified into the following categories:

- Compute flows: Traffic flows that are related to immersion and AI related compute that might be split between the compute processing available across cellular connections.

- Conversational and game state flows: Traffic flows that are related to audio, video, avatar/hologram-based interaction between users. Also, traffic flows that are related to information representing the global immersion state across all users and objects.

- Streaming flows: Traffic flows that are related to slower time scale streaming of information to certain users, e.g., spectators.

### 9.1.4	Post-conditions

With the advancements in immersive gaming technologies and 6G capabilities, local and remote players, cheerleaders and spectators can seamlessly participate and watch an immersive basketball game.

### 9.1.5	Existing features partly or fully covering the use case functionality

Existing immersive features from the 3GPP SA1 Mobile Metaverse Services Study and Work document in TS 22.856 and TS 22.156 can be used to enable some of the features in this use case.

Editor’s note:  referenced needed for TSs above.

### 9.1.6	Potential New Requirements needed to support the use case

Table-9.1.6-1: Key Performance Indicator (KPI) for Immersive Gaming

| Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | Area Traffic capacity | Message size (byte) | Transfer Interval | Positioning accuracy | UE Speed | Service Area |
| Compute flows: [5ms - 20 ms] Conversational and game state flows: [50 - 100ms] Streaming flows: [200 - 300ms] | Player/Cheerleader DL: [640 Mbps] (Note 1) UL: [100Mbps] (Note 2) Spectator: DL: [320 Mbps] (Note 3) UL: [10Mbps] (Note 4) | [99.9 – 99.99%] | N/A | N/A | N/A | [≤10 cm] | Stationary, pedestrian | [38 m x 15 m] (Note 5) |
| Note 1: This data rate is estimated assuming - 2 eye buffers, 8K video resolution, refresh rate of 120 fps, compression ratio of 300 and 8 bits per color. It is important to note that the data rates may change under different assumptions. Data Rate = Video resolution*(Bits per color*3)*Refresh rate* # of eye buffer/(compression ratio). The frame rate of 120 fps is assumed as it has been shown that such high frame rate help reduce the probability of simulator sickness [9.1x]. Note 2: Significantly higher compared to 5G to enable sensor sharing for split computation. Note 3: DL data rate for spectator is assumed to be 2D 8k video Note 4: UL tracking for spectator not as intensive as the player/cheerleader Note 5: Average basketball court is 28m x 15m (adding 5m on both sides for the spectator seating) to make a 38 m x 15 m basketball gymnasium. |  |  |  |  |  |  |  |  |

Editor’s Note: Whether 8K video resolution per eye is required for immersive gaming is FFS.

## 9.2	Use case on multi-media services with deterministic experience via collaborative processing among UE-network-cloud

### 9.2.1	Description

Usually, the limited image rendering capability of mobile devices poses challenges for services that require high-quality image processing capability like high order super-resolution and de-noising algorithm, as well as high-quality rendering capability like ray tracing effect for gaming. As shown in Figure 9.2.1-1, the GPU computing power for image rendering increasing in last ten years and the predication up to 2030 for mobile phone platform as well as the computing power required for advanced services has been summarized as below based on [9.2x1] [9.2x2] [9.2x3]:

Figure 9.2.1-1: increasing GPU computing power for mobile platform

It can be observed that with the failure of Moore's law, the growth of mobile platform computing power gradually slows down. Offloading rendering task to network/cloud is a promising trend to support advanced multi-media services in the future. However, the latency for offloading rendering task to network/cloud is quite difficult to be always guaranteed in anytime/anywhere due to the varied radio channel condition and network load/coverage. UE-Network-Cloud synergized multi-media operation is a collaboration mode which allows mobile device to dynamically offload part of rendering task to the cloud to improve the user experience based on communication link status and maintains deterministic rendering task processing latency by local backup processing in case communication link is not good as shown in Figure 9.2.1-2. There’s an expectation for 3GPP systems to consider such dynamic rendering offload framework to adapt to the synergized multi-media service processing trend.

Figure 9.2.1-2: UE-Network-Cloud synergized multi-media operation framework

The rendering task split function is a function provided by the UE operating system (OS) to map the rendering task request from the APP to local or remote rendering resources. Upon a rendering task is requested to be executed, the task split function can select the local and/or the remote rendering resources based on the relevant factors, such as required processing quality, radio link status, estimated latency, to ensure the deterministic user experiences for the requested multi-media service. The typical services which can be benefited from UE-Network-Cloud synergized multi-media operation include photo enhancement and ray tracing for gaming and so on.

The UE-Network-Cloud synergized operation raises several new issues, as follows:

-	uplink data rate: The rendering task offloading operation usually requires high uplink data rate to achieve higher service availability, considering the metadata to be uploaded for the rendering task is usually large with tight latency, e.g. 5Mbits/50ms, 150Mbits/1.5s (see detail in service flow part). However, the current network typically cannot guarantee sufficient uplink data rate especially at cell edge.

-	Real time communication link capability awareness: The rendering task split function might not know the exact communication link capability such as data rate, latency, and UE power consumption for data transmission, therefore it is hard for the UE to make the split decision.

-	Computing task level PDB requirement guarantee: The uplink metadata burst for a rendering task offloading arrives in a random way, and the latency requirement is intended for the whole data burst (i.e. the metadata of the task) rather than an individual packet. As shown in Figure 9.2.1-3, the current GBR QoS flow framework guarantees the data rate in a fixed averaging window and fulfils latency in a per packet basis which is suitable for the periodic traffic. For the burst data carrying the metadata for rendering tasks, the latency and data rate should be guaranteed toward the whole data set rather than a specific packet. Therefore, the fixed averaging window as for periodic traffic is not suitable for bursty traffic. There is no suitable QoS type to guarantee the deterministic transmission latency for unpredictable uplink busty data traffic.

Figure 9.2.1-3: Burst latency requirement

### 9.2.2	Pre-conditions

1. Rendering resources are deployed in the cloud.

2. The UE operation system supports to map the rendering task from the APP to the local (in the mobile phone) and/or the remote (in the cloud) rendering resources.

### 9.2.3	Service Flows

Synergized photo enhancement case:

1.	User A is in a tourism attraction, taking photos with his mobile phone. Once he clicks the photo button, the phone senor produces raw data of the photo, including up to 6 frames (4K per frame) of raw files spanning 300ms for HDR reconstruction. Raw files retain the original uncompressed image data from the camera sensor, which is usually larger than compressed image format such as JPEG and PNG, and has unique advantages for imaging processing in practice.

NOTE 1: 2 to 10 frames are needed for HDR reconstruction based on [9.2x4]; 6 frames are assumed in this use case.

2.	Upon reception of the rendering task request from camera APP for photo enhancement, the rendering task split function inside the UE OS determines whether or not to upload the raw data to the cloud for more powerful post-processing to enhance the photo quality, for which some factors would be considered such as potential delay and UE power consumption for data transmission.

3.	If the rendering task split function decides to upload the photo to the cloud, the raw data for the photo is compressed to around 150Mb (based on the assumption of 6 frames × 4K raw data and compressed ratio assumption in [9.2x5] [9.2x6]) and uploaded to the cloud within around 1.5s and downloaded the processed photo from the cloud. The UE can reduce the number of frames for uploading based on communication link capability. Therefore, the required uplink data rate in radio layer would be 150M/1.5s=100Mbps.

NOTE 2: E2E latency of 3s is assumed based on users' patience statistics as shown in [9.2x7], where 1.5s is allocated for uploading to the cloud while 1.5s for processing in the cloud and downloading to the UE.

4.	If the rendering task split function decides not to upload the photo to the cloud, the local post-processing will be performed.

5.	When the user A views the photos, the downloaded photo or the local enhanced photo can be shown to the user.

Synergized gaming enhancement case:

1.	User A starts a gaming, the gaming APP produces the scene metadata, including 3D objects, lighting data, and materials, user actions etc. The typical data size after compression can be 5-20Mb (assuming middle to high complexity 3D model including 0.35 to 2 million vertexes (200 bits per vertex) 3D models and compression ratio assumption for 3D model in [9.2x8]).

2.	The gaming APP submits the metadata to rendering task split function inside the UE OS for rendering, the rendering task split function can seek to utilize the more powerful computing in the cloud for higher quality such as ray tracing effect. The rendering task split function determines how to split the metadata to the cloud by considering some factors such as potential delay and UE power consumption for data transmission. For example, the UE selects to offload the slow changed scene background rendering to the cloud while performing rendering for fast moving objects using local rendering resources.

3.	The UE uploads the metadata allocated for the cloud rendering within 50ms [9.2x9] and download the rendering result.

4.	The UE combines the local rendered result and cloud rendered result to the user A.

5.	Once the radio condition goes worse, the full local rendering can be activated to maintain user experience.

### 9.2.4	Post-conditions

User A continues enjoying high quality photo taking and high-quality gaming experience.

### 9.2.5	Existing features partly or fully covering the use case functionality

The split rendering architectures defined in TR 26.928 and TS 26.565, and the corresponding QoS requirements in TS 22.261, allow a UE to offload rendering requirement to the cloud for XR rendering.  These architectures assume that the metadata including 3D models of the gaming and APP logic is pre-installed in the cloud, therefore only user pose information is needed to be transferred in the uplink and there is no dynamic offload decision on whether to render a specific scene in the cloud or UE local resources (i.e. fully rely on the rendering result in the cloud). Therefore, the uplink requirement and dynamic offload decision requirement are not covered by the existing split rendering use cases and part of the downlink requirements on rendering result delivery are covered by the existing split rendering use cases.

It is worth noticing that Edge Enabler Client (EEC) is defined in TS 23.558, which provides supporting functions needed for Application Clients. There is some similarity between EEC and the “rendering task split function” described in this use case.

Editor’s note:  references needed for TR/TSs above.

### 9.2.6	Potential New Requirements needed to support the use case

[PR-9.2.6-1] The 3GPP system shall support burst type QoS with the KPI requirements summarized below：

| Use cases | Burst size (note 1) | Max Allowed latency for a burst (note 2) | Service bit rate: user-experienced data rate | UE speed | Service Area |
| --- | --- | --- | --- | --- | --- |
| Synergized photo enhancement | UL: [150Mbits] | UL: [1500ms] | UL: [100Mbit/s] | Stationary or Pedestrian | Countrywide |
| Synergized gaming enhancement | UL: [5-20Mbits] | UL: [50ms] | UL: [100–400Mbit/s] | Stationary or Pedestrian | Countrywide |
| NOTE 1: assuming 6 x 4K raw pictures and compression ratio in [9.2x6] for photo enhancement. Assuming 3D models including 0.35 to 2 million vertexes and compression ratio assumption for 3D model in [9.2x8] for gaming enhancement. NOTE 2: 1500ms is derived from the E2E latency of 3s (based on users' patience statistics as shown in [9.2x7]) and 1.5s for processing in the cloud and downloading. 50ms uplink latency is derived from [9.2x9]. |  |  |  |  |  |

## 9.3	XR rendering offload support

### 9.3.1	Description

There is a growing demand that people can use diverse types of devices other than smartphones, which connect to mobile network system [9.3X]. Then, in 6G, a number of devices around us are expected to be connected to 6G system. With the trend, wearable devices are expected to be more popular devices for people, but even with such devices like XR devices, users would like to experience immersive applications which require much computing capability for processing application data. However, due to the limited computing capability, user experience could be affected. Therefore, there will be strong needs for such devices to be able to offload application data processing to the edge/cloud server.

In current computing technology, we can offload application data processing to edge/cloud server with completely separated way from mobile network system. However, from user experience point of view, 6G system shall support network and/or device control based on computing offload use of user devices.

This use case aims to provide a service scenario when a user wants a computing offload service supported by the network and requirements for 6G system.

### 9.3.2	Pre-conditions

John's AR glasses can be connected to a mobile network operator's network.

John subscribes to a cloud rendering service to offload the processing functions of the terminal.

NOTE: The cloud rendering service is provided by a function outside the 3GPP system.

### 9.3.3	Service Flows

- John is now walking outside. He wants to communicate with one of his friends by using holographic interaction application which is adequately coordinated for his AR glasses.

- However, the glass is lightweight, and it will take a lot of time to render the hologram image, so he decided to use the cloud rendering service he contracted to improve the experience.

- The network obtains information from the cloud rendering service and the interaction app that John uses, such as the communication related requirements (e.g., required latency) and the computing related requirements (e.g., processing capacity, energy consumption).

- Based on this information, the network controls QoS and provides guidance to cloud rendering service and application on optimal offloading (e.g., placement of compute processing nodes).

### 9.3.4	Post-conditions

The network was able to understand the computing requirements of John's apps and the offload requirements of the cloud rendering service, and set up QoS for John's device, allowing him to use the XR offload service comfortably.

### 9.3.5	Existing features partly or fully covering the use cases functionality

Solution for QoS modification based on communication requirements from application is specified in clause 4.15.6.6 of TS 23.502 [9.3Y].

### 9.3.6	Potential New Requirements needed to support the use case

[PR-9.3.6-1] 	Subject to privacy considerations and regulatory requirements, the 6G network shall be able to enable compute offloading to operator managed data network for third party applications.

## 9.4	Use case on Seamless Immersive Reality in Education

### 9.4.1	Description

Immersive reality combines immersive telepresence and immersive collaboration. Immersive telepresence allows remote participants to appear as physically present in the very same environment as co-located participants. The remote participants perceive the environment and other participants as if they were there in person, thanks to stimulation of their visual, aural, and haptic senses. Immersive collaboration provides a new way of interaction with other people and objects.

Immersive reality technology opens up new opportunities in the education sector, where students are able to collaborate, discuss, and learn in an immersive setting with remote and co-located participants. Participants interact with each other in a more natural way in an immersive reality environment, and participants are also able to interact more naturally with virtual or digital representations of real objects.

The immersive classroom may be local (where all students are physically co-located and learn with virtual objects), hybrid (with both physically co-located as well as remote participants), or fully immersive (where both students and instructors are virtually present).

Figure 9.4.1-1: Hybrid Immersive Classroom Scenario

For example, consider a hybrid immersive chemistry classroom, exploring and assembling 3D models of complex molecules, shown in Figure 9.4-1. The “orange-handed” student and the teacher represent participants remote to the physical classroom while the “red-handed” and the “blue-handed” students represent participants physically present in the classroom. The equipment local to the physical classroom, e.g. cameras, and wearables worn by the students, senses the real environment, communicates the actions and the outcome of the collaborative molecule assembling, and communicates to establish immersive co-presence of the remote and physically present participants.

Further justification and background for the KPIs for this use case is found in the following table [9.4x]:

Table 9.4.1-1: Justification and clarification of KPIs

| KPI | Target Range | Justification |
| --- | --- | --- |
| User-experienced data rate [Mb/s] | < 250 | For DL, but also for UL |
| Area traffic capacity [Mb/s/m2] | < 250 < 20 | Indoor: per floor in a multi-story building Wide area: for immersive experience “on the go” |
| Mobility | seamless HO | Pedestrian for participants in the classroom |
| End-to-end latency [ms] | < 10 < 50 < 150 | Split rendering Voice Collaboration |
| Reliability [%] | 99.9– 99.999 | Depending on data stream |
| Positioning accuracy [cm] | ≤ 10 horizontal ≤ 10 vertical | Positioning of AR glasses and sensors. Network-assisted positioning can be used for e.g. sensors, but high positioning accuracy for AR typically requires device-based sensors and sensor fusion. |

Key Value impact analysis

Material resources: Increased electronic waste from the disposal of devices and network equipment. Increased material consumption from producing the hardware components and expanding network infrastructure including raw material extraction, manufacturing processes, and transportation

Emissions: Reduction of emissions by a reduction in physical travel for work or education.

Inclusion and Equality: Enhances educational opportunities of the population regardless of their location, and improves the efficacy of the education process, both for the students and for the teachers. However, there is a potential digital inequality depending on connectivity access, information technologies (IT) literacy and economic status.

Trustworthiness: Preserved/uncompromised privacy is key for the enablement of this service. However, there are still potential risks for privacy intrusion associated to localisation and positioning data.

### 9.4.2	Pre-conditions

The students in the physical classroom are each using an AR device and have joined the classroom application.

Both AR devices are very simple and light so that the students can use them for long periods of time, which means that the AR devices have limited battery and limited computing capabilities.

The physical classroom has a local server with available computing resources.

### 9.4.3	Service Flows

The teacher, who is located remotely, starts the classroom lesson and pulls up a 3D model of a complex molecule.The AR devices of the students in the physical classroom uses the local server to perform the necessary computation tasks to enable their AR devices to display the same 3D model of the complex molecule (e.g. shadows and lighting aspects of the molecule).

The teacher asks the students in the classroom and the remote student to work together to disassemble the molecule and use the components to construct a new one.

A combination of the information from the wearables of the students and the cameras present in the physical classroom sense the environment, e.g. the students' hand gestures and movements.

The AR devices of the students in the physical classroom uses the local server to perform the necessary computation tasks (e.g. via an AIML model) to anticipate their hand gesture and movements and translate them into specific actions on the 3D molecule model.

### 9.4.4	Post-conditions

N/A

### 9.4.5	Existing features partly or fully covering the use case functionality

- Sensing: immersive experience requires the human sensory system to receive realistic stimuli from a mixed or virtual reality. Some scenarios may use integrated sensing and communication (ISAC) or may apply sensor fusion of network and sensor data of connected sensors. Sensing has been covered in Stage-1 in TS 22.137.

- Positioning: this use case requires accurate positioning for a seamless immersive experience. Positioning requirements are defined in TS 22.261. However, the KPIs from this use case need to be enhanced with greater accuracy compared with the 5G KPIs.

- Media synchronisation: low E2E latency, in combination with the synchronisation of different media for each participant, as well as synchronisation of inter-participant media are needed to ensure a coherent and realistic user experience. This has been covered by TS 22.156 (e.g. [R-5.1.1-002] and [R-5.1.1-003]).

- Digital immersive mapping as a service: to assist seamless immersive reality, digital immersive mapping is provided as a service by the network. This has been covered by TS 22.156 (i.e, clause 5.2.1. Localized mobile metaverse service).

- Service continuity: service at a minimum level to provide a sufficient and for the end user comprehensible and satisfactory QoE across diverse locations ranging from local wireless networks to the wide area network. This has been covered by TS 22.156.

- Distributed Federated Learning: distributed federated learning involving multiple UEs is discussed in TS 22.261. What is not discussed in federated inference, where a AM/ML model includes information from multiple cameras/wearables/sensors for inference.

### 9.4.6	Potential New Requirements needed to support the use case

Requirements

[PR-9.4.6-1]	Subject to operator policy, the operator managed data network (e.g. core network or edge compute domain) shall enable the support of computing tasks to render 6DoF video and spatial audio

{PR-9.4.6-2]	Subject to operator policy, the operator managed data network (e.g. core network or edge compute domain) shall enable the offloading of computing tasks to render 6DoF video and spatial audio to a 3rd party.

[PR-9.4.2-3]	Subject to operator policy, the operator managed data network (e.g. core network or edge compute domain) shall support hosting of e.g. an AI/ML model in the edge compute domain based on latency, transport load or data privacy requirements.

[PR-9.4.2-4]	Subject to operator policy and privacy considerations, the 6G system shall support means to coordinate data flows from UEs (e.g. cameras/wearables/sensors), associated with a specific place (e,g, a classroom, meeting room, office), to enable e.g. an AI/ML model to improve the user’s perception of and interaction with an immersive scene, e.g. to more accurately predict scene changes or object movements within that specific place.

Editor’s Note: terminology and definition of ‘place’ is FFS

Editor’s Note: clarifications on privacy considerations are FFS

Editor’s Note: Operator managed data network is FFS

KPIs

| Scenario | User-experienced data rate [Mb/s] | Area traffic capacity [Mb/s/m2] |  | End-to-end latency [ms] |  |  | Positioning accuracy [cm] |  | Mobility |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Indoor | Outdoor (Wide Area) | Split rendering | Voice | Collaboration | Horizontal | Vertical |  |
| Seamless Immersive Reality in Education | [< 250] | [< 250] | [< 20] | [< 10] | [< 50] | [< 150] | [≤ 10] | [≤ 10] | Pedestrian |

# 10	Massive Communication

Editor's Note: "Massive communication" include e.g. LPWA

## 10.1	Use case 1

# 11	Further Use Cases on Industry and Verticals

Editor's Note: This clause considers various capabilities and use cases needed to support the specific needs of different vertical markets. This includes e.g. Hyper Reliable and Low-Latency Communication, digital twin and other industrial communications use cases. This is not an exhaustive list

## 11.1	Use case on communication on board of UAM aircrafts

### 11.1.1	Description

Urban air mobility (UAM) is a new safe, secure and more sustainable air transportation system for passengers and cargo in urban environments, enabled by new technologies and integrated into multimodal transportation systems. The transportation is performed by electric vertical take-off and landing (eVTOL) aircrafts, remotely piloted or with a pilot onboard [11.1a]. In February 2024 an eVTOL named as "PROSPERITY" [11,1b] that can contain 5 passengers conducted a Shenzhen-Zhuhai test flight, which was a cross-sea and cross-city route. KT showed, at the MWC2024, their UAM Skypath solution, which can provide 5G service for UAM aircrafts flying at 300 – 600 meters altitude.

Compared with the common UAVs (Uncrewed Aerial Aircraft), the UAM aircrafts have the following major differences:

- Large size and heavy weight, higher AGL (above ground level)

Low-altitude airspace usually refers to the airspace with a vertical distance of less than 1000m from the ground, and it can be extended to less than 3000m according to the characteristics and actual needs of different regions [11.1c]. The AGL of UAM aircrafts can be up to 1000m, while the AGL of small UAVs is less than 300m. Communication for the UAM aircrafts with higher AGL need to be considered.

- Higher reliability and safety requirement with human beings onboard

UAM aircrafts share some common low altitude airspace. As predicted by Professor Shen, there will be about 100,000 UAVs flying in Shenzhen's sky at the same time in the future [11.1e]. Considering the area of Shenzhen is 1997 km2, there will be about 50 UAVs flying in 1 km2 airspace. To ensure high reliability and safety, the UAM aircrafts must be aware of the object information that near its flight trajectory.

To guarantee the safety of aircrafts, Detect and Avoid (DAA) technology is widely used in UAVs by using a combination of sensors, cameras, and radar to continuously monitor the UAV’s surroundings [11.1g]. These sensors detect obstacles, other aircraft, and potential hazards in the flight path. The system then processes this information in real-time and adjusts the UAV’s flight path to avoid collisions. However, the capabilities of the sensors on UAVs are not able to sense the blockage far away. Since the UAM aircrafts carry human beings on board, UAV’s DAA system is not adequate to ensure the safety of passengers onboard. By also utilizing the 3GPP sensing service, the safety of UAM aircrafts flying in the common airspace can be guaranteed, as well as the running efficiency of UAM aircraft can be improved so as to transport more passengers or goods.

Typically, the message size for sensing one object could be 1kbyte [11.1f], including information of size/position/speed/direction. Around 25 objects (25kbyte) per frame (20ms) need to be sensed for an aircraft. The reliability requirement for UAM aircrafts is about 99.9%.

- In addition, with passengers onboard the human communication on board of UAM has to be considered.

### 11.1.2	Pre-conditions

The UAM aircrafts are connected to the 6G network, which provides the following services:

- Sensing services to the UAM aircrafts;

- Communication services to the passengers onboard.

### 11.1.3	Service Flows

1.	May and Fei plan visit City B from City A by UAM, and the aircraft can contain up to 4 passengers.

2.	To guarantee the safety of the passengers onboard, the UAM aircraft requests the sensing service from the 6G network. The base station(s) along the flight path will sense the environment information especially other aircrafts within its interesting area, e.g., the interesting area is an airspace with the size of 1 square kilometre and height from 0 to 1000 meters.

3.	As part of the sensing service, the 6G network sends the sensing and/or the warning information to the UAM aircraft.

4.	Upon receipt of the above information, the UAM aircraft further process the information to identify collision threat. If there are some blockages threatening flight safety, the UAM aircraft will perform collision avoidance in advance.

5.	Meanwhile passengers onboard either watch HD video or surf the Internet using their smartphone. The view is very beautiful during the flight, May is very happy to share the scenery with her friends through 4K real-time video on social media by smartphone, e.g., WeChat. Fei is not interested in the scenery during the flight, and he is enjoying a football match live broadcast. The core network sends data of football game with 4K or 8K live broadcast to base station, then Fei can receive the football game data by his smartphone.

### 11.1.4	Post-conditions

Thanks to the sensing services and communication services provided by the 6G, May and Fei had a great onboard experience and safely arrived at the destination via the UAM aircraft.

### 11.1.5	Existing features partly or fully covering the use case functionality

From TS 22.125 [11.1d] “Uncrewed Aerial System (UAS) support in 3GPP”, KPIs for services provided to the UAV applications are defined, especially Table 7.1-1 KPIs for services provided to the UAV applications and Table 7.2-1 KPIs for command and control of UAV operation.

However, the message size of UAV command and control is less than <10kbyte. Latency and reliability requirement are not sufficient for UAM aircrafts. In addition, the AGL for UAV is not high enough for UAM aircrafts.

### 11.1.6	Potential New Requirements needed to support the use case

[PR11.1.6-1] The 6G System shall support the transmission of sensing result; information to the UAM aircrafts with the following KPI requirements.

|  | Typical transmission interval | Altitude AGL | Typical message Size | End to end Latency | Reliability |
| --- | --- | --- | --- | --- | --- |
| Sensing result to a UAM aircraft | [20 ms] | up to 1000 m | [25 kbyte] | [20 ms] | [99.9%] |
| NOTE: Typically, the message size for sensing one object is 1kbyte [x6]. It is assumed that around 25 objects (25kbyte) per frame (20ms) are sensed for an aircraft. The reliability requirement for UAM aircrafts is about 99.9%. |  |  |  |  |  |

[PR- 11.1.6-2] The 6G System shall support services provided to the UAM applications with the following KPI requirements.

| Services | Data rate | End to end Latency | Altitude AGL | service area |
| --- | --- | --- | --- | --- |
| 8K video live broadcast | 100Mbps UAM aircraft originated (note) | 200 ms (note) | up to 1000 m | Urban, scenic area |
|  | 600Kbps UAM aircraft terminated (note) | 20 ms (note) | up to 1000 m |  |
| Video streaming | 4 Mbps for 720p video 9 Mbps for 1080p video UAM aircraft originated (note) | 100 ms (note) | up to 1000 m | Urban, rural area |
|  | 100 Mbps for 8K video UAM aircraft originated (note) | 100 ms (note) | up to 1000 m |  |
| NOTE: These values are aligned with the KPIs for services provided to the UAV applications in TS 22.261, table 7.1-1. |  |  |  |  |

# W	Other Use Cases

Editor's Note: If there are any use cases that does not fall into any sections in this TR, these use cases could be placed in this clause, subject to further discussion. How this clause will be used is FFS.

## W. 1	Use case on coordinating computing and communication for XR rendering

### W.1.1	Description

The exploration of ultimate user experience in consumer products, and the digital and intelligent transformation in the industrial fields have promoted the widely use of computing-intensive applications such as AR/XR, cyber-physical systems, industry robots and etc. However, the balance between computing capabilities and the size and cost of the devices raises a big challenge for the deployment of the entire applications. The ITU-R report [W.1x] points out several emerging technologies are being envisioned to address the challenges. One trend is to process data at the network edge close to the data source for real-time response, low data transport costs and energy efficiency by edge computing technologies, while another trend is to scale out device computing capability beyond its physical limitations by splitting computing workload over reachable computing resources. With the development of each technology trend, the independent control, management and orchestration of communication and the computing resources has been identified to bring out significant negative impact on the performance of the end-to-end solution.

The 6G system is expected to change the situation by coordinating the communication service and computing resource utilization in the stage of system architecture design. In this way, the 6G network can make the ubiquitous single-point computing resource connected and participate in the scheduling of various types of computing resources such as UE, MEC server, cloud server. For example, the 6G network can select appropriate computing resource based on service characteristics to achieve optimal resource usage. On the other hand, the data transmission for the computation can be more flexible and efficient with additional information (e.g. workload, available capability) about the selected computing resources.

The real-time rendering of 3D scenes in large-scale XR application is a typical case of computing-intensive application, which requires large amount of calculation to handle the complicated model and texture mapping. Sometime limited to the capability of hardware, it’s impossible for a single device to render the scene individually, and the distribution of the computing workload to other computing capable nodes can solve the issue. Sometimes the duplicated rendering of the same XR scene which is being requested by multiple users can be avoided if choose to execute the rendering in the cloud server. However, the XR application can have little information on the status of computing resources so that it’s difficult for the planned data transmission and computation to work together efficiently.

This use case illustrates how the 6G system enables better user experience of split XR rendering via the coordination of computing resources and communication resources.

Figure W.1.1-1: Coordinating Computing and Communication for XR rendering

### W.1.2	Pre-conditions

Several computing resource nodes (e.g. MEC servers, cloud computing center) for XR services are deployed distributedly in different locations and have been enrolled in 6G core network. MEC server A, B and C are provisioned as a part of rendering pipelines to execute render engine, engine adaption, rendering acceleration for the rendering task. Cloud computing center is capable of all the functions for XR rendering.

XR Application Platform provides the render services to various XR applications.

Task Management Server is responsible for task analyse, sub-task planning, graphic composition and etc.

The UE supporting XR application is the subscriber of the 6G network. It has registered to 6G network and been authorized for the computing coordination service.

### W.1.3	Service Flows

- Regarding XR application’s need, UE sends a request of XR rendering to XR Application Platform.

- XR application platform discomposes UE’s request to different tasks and dispatches the render task to Task Management Server through IP network.

- Task Management Server analyzes the render task and plan the computing task based on the capability of computing resources. Then it sends the requests of computing coordination service and XR communication service to 6G core network, including the information of computing coordination service QoS(e.g. requested computing capabilities, response time) and communication service QoS (e.g. latency).

- 6G core network selects MEC server B and cloud computing center based on the requested QoS of computing coordination service, and the status information of all enrolled computing resources, and then feedback Task Management Server about the coordination result (e.g. selected computing resources). Meanwhile, 6G core network establishes the communication paths between UE and MEC server B and cloud computing center to transmit data for the rendering based on the requested QoS of communication service.

- Task Management Server distributes the necessary data and associated information to all the selected computing resources via IP network.

- MEC server B and cloud computing center executes individual rendering sub-tasks, sync-up with Task Management Server about the progress of its sub-tasks and sync-up the status information (e.g. computing workload, congestion status) with 6G core network.

- The 6G core network detects the overload of MEC server B, and informs Task Management Server to replace MEC server B with MEC server C.

- Task Management Server collects the rendering graphics from MEC server B, MEC server C and Cloud center after the sub-tasks are finished and composes the XR image for XR Application Platform. Then XR Application Platform responds the rendered XR image to UE.

### W.1.4	Post-conditions

The render task is efficiently completed with the collaboration of several edge computing servers under the coordination of 6G core network.

The rendered XR image is successfully provided to UE as expected.

### W.1.5	Existing features partly or fully covering the use case functionality

None

### W.1.6	Potential New Requirements needed to support the use case

[PR-W.1.6-1] The 6G network shall support mechanisms to manage computing resources (i.e. edge server(s) or cloud server(s)). e.g. enroll computing resources, maintain the information of computing resources and etc.

Editor’s note: It’s FFS to refine the wording of examples.

[PR-W.1.6-2] The 6G network shall support mechanisms to collect the status information (e.g. computing workload, congestion information, available capability, power consumption) of trusted computing resources (i.e. edge server(s) or cloud server(s)) on-demand or periodically.

[PR-W.1.6-3] The 6G network shall provide mechanisms to expose to trusted 3rd party the information (e.g. computing capability, the location, allowed service types, status, power consumption) of computing resources (i.e. edge server(s) or cloud server(s)) .

[PR-W.1.6-4] Subject to operator’s policy, application needs or both, the 6G network shall support the selection of computing resource(s) (i.e. edge server(s) or cloud server(s)) based on e.g. requested computing capabilities, and support routing of data traffic between a UE and the selected computing resource(s) based on required communication service QoS.

Editor’s note: It is FFS to clarify the terminology regarding computing resource and computing node in the requirements.

## W.2	Use case on Optimizing 6G Infrastructure Utilization via Resource Exposure in 6G

### W.2.1 	Description

With the advent of advanced technologies such as Generative Artificial Intelligence (Gen AI), Large Language Models (LLMs), and AI-accelerated xPUs, wireless network operators are now equipped with substantial computational and storage infrastructures that often remain underutilized. These computation resources may be hosted at edge sites in a distributed manner. It is in the strategic interest of these operators to expose these dormant resources to third-party entities, allowing them to leverage the computational power and AI capabilities when required. In exchange, operators can monetize these assets by charging nominal fees for compute cycles, AI processing, and storage utilization by third parties.

This approach not only generates additional revenue streams for network operators but also ensures the optimal utilization of existing infrastructure. Consequently, it justifies the need for new requirements in 6G networks to expose and manage network resources effectively. By incorporating standardized interfaces, robust security protocols, and advanced resource management frameworks, 6G can facilitate efficient and secure access to these resources. This transformation would position 6G networks as comprehensive platforms for distributed computing and AI services, aligning with the evolving demands of the technology landscape and provide resource as a service. This unique network-aware resource exposure capability is enabled by the 6G system’s centralized policy framework.

The 6G system will be built on the cloud, optimized for ubiquitous computing, and natively provide computing services (e.g., in forms of infrastructure services, platform services, and software services). [W.2X]

Figure W.2.1-1: Illustration of network resource usage and exposure to 3rd party

NOTE:	The figure above is just an illustration to depict where computation resources could be deployed e.g. at an edge site.

### W.2.2	Pre-conditions

Network Operator A is equipped with significant compute, storage, and AI processing resources. Some of these resources are used by NFs or OAM etc. However, resource utilization varies based on network load, and during periods of low demand—often at certain times of day or night—up to 40% of these resources remain idle. These idle resources represent untapped potential, lying unused with no opportunity for additional utilization.

Meanwhile, Operator A’s customer, a gaming company called X, experiences high demand for computational resources during nighttime hours when many gamers are online on both computers and mobile devices. As games become increasingly complex and distributed, companies like X are integrating LLMs into their platforms to enhance gameplay. Offloading some of this processing to external infrastructure would be beneficial, particularly if X could access these idle resources at a competitive rate. Being an existing customer of Operator A, company X could potentially secure these resources at favourable rates.

### W.2.3	Service Flows

1	A third-party requests access to specific GPU or AI resources via an API provided by Operator X

2	The 6G network verifies the request against defined policies and authorizes access if conditions are met.

3	The 6G network checks with the policy framework, which in turns may confirm with orchestration system if the resources are available, how many of them are used for NF and if the load is expected to increase based on history of network usage. Upon approval, the 6G system allocates the requested resources and establishes secure access for the third-party. This unique network-aware resource exposure capability is enabled by the 6G system’s centralized policy framework.

4	The operator could also add its own rules and policy to manage the resource allocation with the help of policy framework.

5	The system continuously monitors resource usage and records data for charging and reporting purposes.

6	The 3rd party is also allowed to analyse the usage in real time.

Editor’s note:  Should Operator X in 1 above be Operator A?

### W.2.4	Post-conditions

1	Third-party entity completes its tasks using the allocated resources.

2	The 6G network releases the resources and updates the availability status in real-time

3	Usage data is logged, and relevant charges are generated and sent for billing.

4	Policy function is in control of entire operation.

### W.2.5	Existing features partly or fully covering the use case functionality

< Highlight existing features in the existing set of normative specifications that partly or fully cover this use case.>

### W.2.6	Potential new requirements needed to support the use case

[PR W.2.6-1] Based on operator policy, the 6G network shall provide suitable APIs to allow authorized third parties to retrieve availability information about computational resources (e.g., storage, AI processing units, xPUs information etc.) inside operator managed data network (e.g., similar to Service Hosting Environment, Edge Compute domain) and to utilize these computational resources for running workloads on demand.

[PR W.2.6-2] The 6G network shall support charging for the usage of network resources by third parties.

[PR W.2.6-3] The 6G network shall support detailed monitoring and reporting of computational resource usage in the operator managed data network (e.g., similar to Service Hosting Environment, Edge Compute domain).

# X	Other Considerations

Editor's Note: e.g., charging, etc. if any

# Y	Consolidated Potential Requirements

Editor's Note: It needs to be decided whether / how consolidation would be performed.

# Z	Conclusion and Recommendations

Annex A:
Additional Use Cases

Editor's Note: If there are any use cases that does not fall into any sections in this TR, these use cases could be placed in the Annex, subject to further discussion. How the Annex will be used is FFS.

# A.1	Use Case #X

# A.2	Use Case #Y

# A.3	Use Case #Z

Annex <X>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | SA1#107 |  |  |  |  | First version | 0.0.0 |
| 2024-11 | SA1#108 | S1-244670, S1-244819, S1-244823, S1-244838, S1-244843, S1-244847, S1-244868, S1-244869, S1-244870, S1-244875, S1-344876, S1-244895, S1-244896, S1-244897, S1-244898. S1-244900, S1-244901, S1-244902, S1-244903, S1-244905, S1-244911, S1-344913, S1-244914, S1-244915, S1-244918, S1-244919, S1-244921 |  |  |  | Output of approved pCRs from SA1 #108 | 0.1.0 |