| 3GPP TS 23.369 V0.3.0 (2025-04) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Architecture support for Ambient power-enabled Internet of Things; Stage 2 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms and abbreviations	7

3.1	Terms	7

3.2	Abbreviations	7

4	Architecture model and concepts	8

4.1	General concept	8

4.2	Architecture	8

4.2.1	General	8

4.2.2	Architecture for NG-RAN connectivity	8

4.2.2.1	General	8

4.2.2.2	Direct connectivity between AIOTF and NG-RAN	10

4.2.2.3	Indirect connectivity between AIOTF and NG-RAN via an AMF	10

4.3	Reference points	11

4.4	Service-based interfaces	12

4.5	Functional Entities	12

4.5.1	AIoT Device	12

4.5.2	AIoT Readers	12

4.5.3	AIOTF	12

4.5.4	NEF	13

4.5.5	AF	13

4.5.6	NRF	13

4.5.7	AMF	13

4.5.8	UDR	13

4.5.9	ADM	13

4.6	Protocol Stacks	14

4.6.1	General	14

4.6.2	Protocol Stack between AIoT Device and AF	14

4.6.2.1	General	14

4.6.2.2	Protocol Stack between AF and AIoT Device for NG-RAN Direct Connectivity	15

4.6.2.3	Protocol Stack between AF and AIoT Device for NG-RAN Indirect Connectivity	15

5	High level functionality and features	15

5.1	General	15

5.2	AIoT Services	16

5.2.1	AIoT Inventory service	16

5.2.2	AIoT Command service	16

5.3	Discovery and Selection of AIoT node(s)	16

5.3.1	AIOTF Discovery and Selection	16

5.3.2	ADM Discovery and Selection	16

5.3.3	NG-RAN Node and RAN Reader Selection	17

5.3.4	AMF Discovery and Selection	17

5.4	Assistance information provided to NG-RAN node	18

5.5	AIoT Device Profile Management	18

5.6	AF authorization for the AIoT Services	18

5.7	Identifiers	19

5.7.1	General	19

5.7.2	AIoT Device Permanent Identifier	19

5.8	Filtering Information	20

6	AIoT Procedures	21

6.1	General	21

6.2	AIoT Service Procedures	21

6.2.1	General	21

6.2.2	Inventory Procedure	21

6.2.3	Command Procedure	24

6.2.4	Procedures between AIOTF and NG-RAN for Indirect Connectivity	26

7	Network Functions Services	26

7.1	General	26

7.2	AIOTF services	26

7.2.1	General	26

7.2.2	Naiotf_AIoT_Inventory service operation	27

7.2.3	Naiotf_AIoT_Command service operation	27

7.2.4	Naiotf_AIoT_Notify service operation	28

7.3	AMF services	28

7.3.1	General	28

7.3.2	Namf_AIoT_MessageDelivery service operation	28

7.3.3	Namf_AIoT_Notify service operation	29

7.4	NEF services	29

7.4.1	General	29

7.4.2	Nnef_AIoT_Inventory service operation	29

7.4.3	Nnef_AIoT_Command service operation	30

7.4.4	Nnef_AIoT_Notify service operation	30

7.5	ADM services	30

7.5.1	General	30

7.5.2	Nadm_DM_Query service operation	31

7.5.3	Nadm_DM_Update service operation	31

Annex A (informative): Change history	32

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document specifies architectural enhancements to the 5G system to support Ambient power-enabled Internet of Things, complying to the requirements in TS 22.369 [2] applicable to the AIoT Device types, traffic types, use cases and connectivity topologies defined in TS 38.300 [5].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for Ambient power-enabled IoT".

[3]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[6]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[7]	GS1 TDS Release 2.1: "EPC Tag Data Standard".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	3GPP TS 33.369: "Security aspects of ambient IoT services in 5G".

3	Definitions of terms and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AIoT Device: An Ambient IoT device is an IoT device powered by energy harvesting, with limited energy storage capability.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ADM	AIoT Data Management

AIoT	Ambient IoT

AIOTF	Ambient IoT Function

EPC	Electronic Product Code

4	Architecture model and concepts

4.1	General concept

AIoT is a service that can be provided by the 5GS system to support Ambient power-enabled IoT devices that are powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other suitable power source.

The 5GS System architecture for AIoT include the following functions and procedures for:

-	AIoT Device identification;

-	AIoT Device inventory;

-	Providing to, and obtaining from, an AIoT Device application data.

-	Disabling AIoT Devices.

4.2	Architecture

4.2.1	General

The 5GS System architecture for AIoT includes core network functions, different AIoT Reader architectures and AIoT Devices. The different AIoT Reader architectures allow for different deployment options. The following AIoT Reader architectures are defined:

-	NG- RAN (which supports AIoT Reader), which includes either a direct connectivity between NG-RAN and the AIOTF or an indirect connectivity between NG-RAN and the AIOTF via an AMF.

The NG-RAN in this specification refers to the gNB which supports AIoT related functionalities, as specified in TS 38.300 [5]. The gNB may only support communication with AIoT Devices.

The architecture for Network Exposure Function, using reference point representation, defined in clause 4.2.3 of TS 23.501 [3] is applicable for AIoT, with a southbound interface from the NEF to AIOTF.

4.2.2	Architecture for NG-RAN connectivity

4.2.2.1	General

5GS system architecture for AIoT supports the following connectivities to access an NG-RAN:

-	Direct Connectivity: AIOTF communicates with NG-RAN directly.

-	Indirect Connectivity via AMF: NG-RAN and the AIOTF communicate indirectly via an AMF.

Figure 4.2.2.1-1 depicts the complete non-roaming architecture showing the overall 5GS architecture for support of AIoT, including both the Indirect Connectivity and Direct Connectivity.

Figure 4.2.2.1-1: Non-roaming AIoT System Architecture

NOTE 1:	For the sake of clarity and to depict the complete reference point architecture, the AMF, AIOT2 and N2 as depicted using dashed lines, as all deployments might not use them.

Figure 4.2.2.1-2 depicts the complete non-roaming AIoT system architecture, using the reference point representation.

Figure 4.2.2.1-2: Non-roaming AIoT System Architecture (RAN Readers) in reference point representation

NOTE 2:	For the sake of clarity of the point-to-point diagrams, the AF and NRF have not been depicted. However, all depicted Network Functions can interact with the NRF as necessary.

NOTE 3:	For clarity, the UDR and its connections with ADM, are not depicted in the point-to-point and service-based architecture diagrams. For more information on the ADM data storage architectures refer to clause 4.5.8.

NOTE 4:	For the sake of clarity and to depict the complete reference point architecture, the AMF, AIOT3, N2 and AIOT2 as depicted using dashed lines, as all deployments might not use them.

The architectures in the following clauses showing parts of the overall AIoT architecture specific to each connectivity option:

-	Direct Connectivity: the AIOTF uses AIOT2 to access NG-RAN, and is described in clause 4.2.2.2.

-	Indirect Connectivity via AMF: the AIOTF uses an AMF which uses N2 to access NG-RAN, and is described in clause 4.2.2.3.

NOTE 5:	A deployment that only uses, e.g. the Direct Connectivity only does not need to deploy the NFs, reference points and service-based interfaces associated with the Indirect Connectivity, and vice-versa.

4.2.2.2	Direct connectivity between AIOTF and NG-RAN

In the Direct Connectivity architecture, the AIOTF uses AIOT2 to communicate directly with NG-RAN.

Figure 4.2.2.2-1 depicts the AIoT architecture, using the service-based interfaces, showing only the parts of the AIoT architecture for an AIOTF connecting to NG-RAN directly. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-1 remain unchanged.

Figure 4.2.2.2-1: NG- RAN - AIOT Direct connectivity Architecture

Figure 4.2.2.2-2 depicts the AIoT architecture, using the reference point representation, showing only the parts of the AIoT architecture for an AIOTF access NG-RAN. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-2 remain unchanged.

Figure 4.2.2.2-2: NG-RAN - AIOT Direct connectivity Architecture in reference point representation

4.2.2.3	Indirect connectivity between AIOTF and NG-RAN via an AMF

Figure 4.2.2.3-1 depicts the AIoT architecture, using the service-based interfaces showing only the parts of the AIoT architecture for an AIOTF connects indirectly to NG-RAN via an AMF. The remaining parts of the AIoT Architecture shown in Figure 4.2.2.1-1 remain unchanged.

Figure 4.2.2.3-1: NG-RAN - AIOT Indirect connectivity Architecture

Figure 4.2.2.3-2 depicts the AIoT architecture, using the reference point representation showing only the parts of the AIoT architecture for an AIOTF connects to NG-RAN via an AMF. The remaining parts of the AIoT architecture shown in Figure 4.2.2.1-2 remain unchanged.

Figure 4.2.2.3-2: NG-RAN - AIOT Indirect connectivity Architecture in reference point representation

4.3	Reference points

The AIoT Architecture contains the following reference points:

AIOT1:	Reference point between the AIoT Device and the AIOTF.

AIOT2:	Reference point between the NG-RAN and the AIOTF.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

AIOT3:	Reference point between the AIOTF and the AMF.

AIOT4:	Reference point between the AIOTF and the NEF.

AIOT5:	Reference point between the AIOTF and the NRF.

AIOT6:	Reference point between the AIOTF and the ADM.

AIOT7:	Reference point between the ADM and the UDR.

AIOT8:	Reference point between the ADM and the NEF.

In addition to the relevant reference points defined in TS 23.501 [3], in the case of AIoT, these reference points are as follows:

N2:	Reference point between the NG-RAN and the AMF.

N33:	Reference point between NEF and AF.

4.4	Service-based interfaces

Naiotf:	Service-based interface exhibited by the AIOTF.

Nadm:	Service-based interface exhibited by the ADM.

In addition to the relevant services defined in TS 23.501 [3] the following service-based interfaces are enhanced for AIoT in this specification:

Namf:	Service-based interface exhibited by AMF.

Nnef:	Service-based interface exhibited by NEF.

Naf:	Service-based interface exhibited by AF.

Nnrf:	Service-based interface exhibited by NRF.

4.5	Functional Entities

4.5.1	AIoT Device

Editor’s note:	The definition of AIoT Device needs to align with the definition at RAN WGs.

4.5.2	AIoT Readers

Editor’s note:	NG-RAN and AIoT Reader will be aligned later with RAN WGs. It is FFS whether to specify the relationship between NG-RAN and AIoT Reader and which term to use in this TS.

4.5.3	AIOTF

The AIOTF supports the following functions:

-	Termination of AIoT NAS protocol with AIoT Device.

-	Connectivity with an NG-RAN via a direct interface reference point or via an AMF.

-	Support of AIoT service operations towards the AIoT Devices(s):

-	Providing an interface to the AF (or via NEF) for AIoT services and authorizing the trusted AF's AIoT service operation request.

-	Triggering the NG-RAN to perform AIoT service operations towards the AIoT Device(s), and optionally determining and providing assistance information to the NG-RAN .

-	Report the service operation results to AF (or via NEF) based on the local configuration or the AF request.

-	NG-RAN selection and optionally a list of RAN Reader selection for AIoT service operations.

-	AMF selection based on target area information when AIOTF connects the NG-RAN indirectly via an AMF.

-	Correlation ID allocation corresponding to the AF service operation request.

-	Retrieving AIoT device profile data from ADM.

-	Retrieving AF subscription data from ADM.

-	Optionally AIoT Device context management.

-	Perform aggregation of the AIoT service responses, determine and provide assistant for the AIoT aggregation in NG-RAN.

NOTE:	Authentication of AIoT Devices related to AIOTF is defined in TS 33.369 [9].

4.5.4	NEF

In addition to the functions defined in TS 23.501 [3], the NEF performs the following functions:

-	Providing a service exposure API to AFs of 3rd party for AIoT services.

-	Interacting with AF of 3rd party and AIOTF.

-	Selection of AIOTF for AIoT services.

-	Authorization of the untrusted AF’s AIoT operation request.

4.5.5	AF

The AF performs the following functions to support AIoT services:

-	Interacting with NEF for AIoT related service exposure for 3rd party .

-	Interacting with AIOTF for AIoT related service exposure for trusted AF.

4.5.6	NRF

In addition to the functions defined in TS 23.501 [3], the NRF performs the following functions:

-	Support of new NF types AIOTF and their corresponding NF profiles. The NF profile includes the AIOTF ID or IP address, NF type and the serving area information of the AIOTF.

-	Support of AIOTF discovery based on parameters such as target area information.

4.5.7	AMF

The AMF performs the following functions when the NG-RAN and the AIOTF communicate indirectly via an AMF:

-	Relaying signalling for AIoT service between AIoT-RAN and AIOTF.

-	Providing transparent transport for AIoT NAS messages between AIoT Device and AIOTF.

-	Providing transport for NG-RAN Node’s serving area information between NG-RAN and AIOTF.

Editor’s note:	Further functions on AMF is FFS.

4.5.8	UDR

In addition to the functions defined in TS 23.501 [3], the UDR may support the following functions:

-	Storage of AIoT device profile data.

4.5.9	ADM

The ADM supports the following functions:

-	Management of AIoT device profile data.

4.6	Protocol Stacks

4.6.1	General

This clause specifies the protocol stacks between entities used for AIoT. The protocol stacks include the following:

-	Protocol stacks between AIoT Device and AFs, including the protocol stacks among AIoT Device, NG-RAN, AMF, AIOTF, NEF and AFs.

Editor's note:	The protocol stacks between RAN and 5GC in the following clauses need to align with RAN specifications.

4.6.2	Protocol Stack between AIoT Device and AF

4.6.2.1	General

The general protocol stack between an AIoT Device, NG-RAN, AIOTF and AF is shown in Figure 4.6.2.1-1.

Legend:

-	AIoT NAS: The NAS protocol between AIoT Device and AIOTF.

-	AIoT AS: It is between the AIoT Device and the NG-RAN is specified in TS 38.300 [5].

-	AIoT Data: It is the application data exchanged between the AIoT Device and AF.

Figure 4.6.1-1: Protocol Stack Between AIoT Device and AF

The NG-RAN may communicate to AIOTF via different connectivities, see clause 4.2.2.1. The NG-RAN protocol stack remains the same and is transparent to the different connectivities. The protocol stacks and routing of information between an NG-RAN and the AIOTF is defined in other clauses depending upon the architecture used:

-	Direct Connectivity: When the NG-RAN communicates with AIOTF directly, the protocol stack is specified in clause 4.6.2.2.

-	Indirect Connectivity: When the NG-RAN communicates with AIOTF indirectly via an AMF, the protocol stack is specified in clause 4.6.2.3.

The AIoT NAS protocol supports the inventory response and command related signalling.

4.6.2.2	Protocol Stack between AF and AIoT Device for NG-RAN Direct Connectivity

Legend:

-	AIoT Reader Control: It is between the AIOTF and NG-RAN.

Figure 4.6.2.2-1: Protocol Stack between AF and AIoT Device for Direct Connectivity option

The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the NG-RAN transparently.

4.6.2.3	Protocol Stack between AF and AIoT Device for NG-RAN Indirect Connectivity

Figure 4.6.3.1-2: Protocol Stack between AF and AIoT Device for Indirect Connectivity via AMF option

In this Protocol Stack, the AMF is additionally involved and its functionality is as defined in clause 4.5.7. The AIoT NAS messages between the AIoT device and the AIOTF are transferred via the NG-RAN and AMF transparently.

Editor's note:	Whether AIoT Reader Control is transported by NGAP or is part of the NGAP protocol will be updated based on RAN WG3 decision.

Editor's note:	The reference to AIoT AS Layer protocol will be updated based on RAN WG1 or RAN WG2 decision.

5	High level functionality and features

5.1	General

5.2	AIoT Services

The following AIoT Services are supported:

-	Inventory service;

-	Command service.

5.2.1	AIoT Inventory service

AIoT Inventory service is used to discover the AIoT devices, i.e. to obtain the AIoT device identifiers.

5.2.2	AIoT Command service

Three types of Command service operations are supported: Read, Write and Disable.

An AF uses the Command service Read operation to retrieve information from a specific AIoT device.

An AF uses the Command service Write operation to configure information into a specific AIoT device.

An AF uses the Command service Disable operation to permanently disable the capability of the AIoT device(s) to transmit RF signals

5.3	Discovery and Selection of AIoT node(s)

5.3.1	AIOTF Discovery and Selection

The AIOTF discovery and selection functionality is supported by the NEF and AF to determine an AIOTF to handle AIoT Services.

When the NEF or AF performs AIOTF discovery and selection for an AIoT service operation request, the following applies:

-	The NEF or AF shall utilize the NRF to discover AIOTF instance(s) unless AIOTF information is available by other means, e.g. locally configured in the NEF or AF. The NEF or AF provides to the NRF the Target Area information, when trying to discover AIOTF instance(s). The NRF provides AIOTF instance(s) to the NEF or AF.

NOTE:	The local configuration in the NEF can be used for example to select a specific AIOTF instance for the request from a given AF.

-	The NEF or the AF selects the AIOTF instance(s) based on the available AIOTF instance(s) (obtained from the NRF or locally configured)

When AIoT Device ID information indicates individual AIoT device(s), the NEF may select AIOTF(s) by taking into account the last known AIOTF instance(s) for those device(s) from ADM.

5.3.2	ADM Discovery and Selection

The ADM discovery and selection function is supported by the AIOTF to select an ADM instance to retrieve the device profile data or update the last known AIOTF for the AIoT device. The AIOTF may also discover and select an ADM to retrieve AF authorization data. Similarly, the NEF uses the ADM discovery and selection function to select an ADM to obtain the last known AIOTF for the AIoT device.

When the ADM discovery is performed, the AIOTF or the NEF utilizes the NRF to discover the ADM instance(s) unless the ADM information is available by other means, e.g., locally configured. The AIOTF or the NEF selects an ADM instance based on the available ADM instances (obtained from the NRF or locally configured).

One or more of the following factors may be considered for the ADM discovery and selection:

-	The AIoT device permanent ID or domain information.

-	The AF ID.

Editor’s Note: 	The further investigation of factors for the ADM selection will be needed and other factors are FFS.

5.3.3	NG-RAN Node and RAN Reader Selection

The AIOTF selects NG-RAN node(s) and optionally RAN readers.

The AIOTF obtains the NG-RAN information (supported Area, RAN reader ID list, and, optionally, the location of each served RAN reader) via OAM. The AIOTF receives an AIoT service request including the Target Area information from the NEF or trusted AF. Based on the received Target Area information and the NG-RAN information configured by OAM, the AIOTF selects the NG-RAN node(s) and optionally RAN reader(s).

NOTE 1:	The Target Area information can span the supported Area of multiple NG-RAN node(s) or can be a subset of the supported Area of a NG-RAN node.

The AIOTF sends the AIoT service request to the selected NG-RAN node(s), optionally including the RAN Area information derived from mapping the Target Area information to the supported Area and/or RAN reader ID list, if selected, to assist RAN reader selection by NG-RAN node, either directly or through the selected AMF. If multiple NG-RAN nodes are selected, the AIOTF sends the AIoT service request to each selected NG-RAN node along with its corresponding RAN Area information and/or the RAN reader ID list, if selected

If the AIOTF does not provide the RAN Area information or the RAN reader ID list in the AIoT service request to the NG-RAN node, then the NG-RAN node may use all available RAN readers.

Editor’s note:	NG-RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Editor’s note:	The AIOTF or AMF configuration of NG-RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Editor’s note:	It is FFS how NG-RAN transfers and updates its information to the AIOTF in indirect connectivity.

NOTE 2:	RAN reader ID is not exposed to the AF.

If an AIoT service request includes AIoT device Identifier(s), the AIOTF may consider the last known serving RAN reader(s) from the AIoT device context to determine targeted RAN reader(s) directly.

NOTE 3:	From the selected NG-RAN node, the AIOTF receives the Inventory Report including the RAN reader ID that represents the AIoT device’s location at reader ID granularity. The AIOTF uses the RAN reader ID to update the last known serving RAN reader information in the local AIoT device context.

5.3.4	AMF Discovery and Selection

For indirect Connectivity via AMF (see clause 4.2), AMF discovery and selection functionality is implemented in AIOTF.

When the AIOTF performs AMF discovery and selection in order to forward AIoT service operation messages the NG-RAN via AMF, the following applies:

-	After the NG-RAN selection for the AIoT service operation (see clause 5.3.3), the AIOTF selects the AMF that has association with the selected NG-RAN nodes in order to forward the AIoT service operation messages.

Editor’s note: 	It is FFS whether and how the procedure is performed between AMF and AIOTF in order to provide the NG-RAN ID of NG-RAN from the AMF to the AIOTF.

5.4	Assistance information provided to NG-RAN node

The AIOTF provides the following assistance information to the NG-RAN together with the service operation requests:

a)	AIoT service type (e.g. Inventory, Command) .

b)	Optionally, approximate number of AIoT devices based on AF request.

c)	Optionally, approximate D2R message size based on AF request.

d)	Optionally, time interval as AIoT aggregation assistance information.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	It is FFS where detailed description on time interval as AIoT aggregation assistance information will be captured, i.e. in this clause or in another clause.

If not provided by the AF, bullets b) and c) in the above assistance information provided by the AIOTF may be based on local configuration based on SLA between the AIoT service provider represented by an AF and the operator.

If the AF has provided a time interval, then the AIOTF should set bullet d) to NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval.

The assistance information is used by the NG-RAN for performing service operations, e.g. radio resource allocation by using bullets b) and c), AIoT Device responses aggregation by using bullet d).

5.5	AIoT Device Profile Management

The ADM may hold operator’s subscription data for the AIoT Device used in the network. If the AIoT Device is managed by the network, then the profile data for an AIoT Device is required in the network, otherwise the corresponding profile data (e.g. AIoT Device Permanent ID or credentials) is stored external to the network.

The AIoT Device Permanent ID is used by the AIOTF together with local configuration, 3rd party related context to locate the entity which stores the profile data of an AIoT Device.

In case the AIoT Device is managed by the network, the AIOTF checks whether the AIoT Device Permanent ID from AIoT Device has the profile data in the network and retrieves the profile data. The profile data for AIoT Device is different with UE subscription data as defined in TS 23.502 [4] clause 5.2.3, it is stored in the ADM  network entity that exclusively supports management of AIoT Device’s profile data. The AIoT Device Permanent ID is the primary key for AIoT device profile data in the ADM.

The table 5.5-1 below describes information storage structures for AIoT device profile data.

Table 5.5-1: AIoT Device Profile Data

| Field | Description |
| --- | --- |
| AIoT Device Permanent ID | Uniquely identifies the AIoT Device. |
| Last known AIOTF information | Indicate the last known AIOTF that serves the AIoT device, or unknown |

NOTE:	Security materials and security mechanism involving ADM are specified in TS 33.369 [9].

5.6	AF authorization for the AIoT Services

The information needed to support the authorization of the AF for performing the AIoT service is stored as the authorization data for 3rd party AF in the ADM, or locally configured in the AIOTF.

Table 5.6-1 below describes items stored as AF authorization data for the AIoT.

Table 5.6-1: AF Authorization Data for AIoT

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed area | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
| Allowed service operations | Indicate the allowed service operation (s) for the indicated AF, e.g. inventory, read, write, permanent disable. |
| Allowed target AIoT Devices | Indicate the allowed AIoT Device(s) for the indicated AF. The information indicating the allowed target AIoT Devices is a list of the permanent AIoT Device ID (see clause 5.7) or the filtering information (see clause 5.8). |

The authorization of the AF for the AIoT includes two parts:

-	NEF performs AIoT AF request authorization based on the service level agreement (SLA) between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501 [8].

-	AIOTF may perform authorization of AIoT service requested by the AF, using the AF authorization data retrieved from n the ADM or configured locally as described in above Table 5.6-1. When ADM is used, the AIOTF also subscribes to changes of AF authorization data in the ADM for synchronization.

5.7	Identifiers

5.7.1	General

5.7.2	AIoT Device Permanent Identifier

In order to support the AIoT feature in 5G System, a globally unique AIoT Device Permanent Identifier shall be allocated to each AIoT Device. An AIoT Device Permanent Identifier is assigned either by an operator or by a third party. The AIoT Device Permanent Identifier is used to identify an AIoT Device and locate the entity where the AIoT Device related information is stored.

NOTE 1:	How to configure an AIoT Device with the AIoT Device Permanent Identifier is out of scope of this specification.

The AIoT Device Permanent Identifier includes the following components:

-	The ID Type, including:

-	Information indicating whether a PLMN ID is included.

-	Information indicating whether a NID is included.

-	Information indicating whether a third party identifier is included.

-	Identification Information Type, indicating whether the Identification Information contains an  EPC or unstructured information.

-	The Domain Information includes none, one or more of the following:

-	A PLMN Identifier (i.e., MCC and MNC) as specified in TS 23.003 [6] when the information in the ID type indicates it is included

-	A Network Identifier (NID) as specified in TS 23.003 [6] when the information in the ID type indicates it is included.

-	A third party identifier used to identify a third party when the information in the ID type indicates it is included.

-	The Identification Information is used to distinguish different AIoT Devices within the scope identified by Domain Information (if available) and can contain either:

-	An EPC, as defined in clause 14 of [7];

-	Unstructured information, where the contents is defined by the allocator.

Figure 5.7.2-1: AIoT Device Permanent Identifier Structure

An operator allocated AIoT Device Permanent Identifier should include the identifier of the network for the operator. The identifier of the network is present as either a PLMN Identifier, NID or both in the AIoT Device Permanent Identifier.

A third party allocated AIoT Device Permanent Identifier may include none of the following information or include any combination of at least one kind of the following information: a PLMN Identifier, NID or the third party identifier.

NOTE 3:	The length of ID Type, PLMN Identifier (if present), NID (if present) and the third party identifier (if present) components is fixed. The length of the Identification Information is variable. The details are specified in TS 23.003 [6] and TS 29.xxx [xx].

Editor’s note:	The reference in NOTE 3 needs to be updated, when the appropriate stage 3 document is identified.

Editor’s Note:	Whether the Domain Information can be empty needs to be clarified.

The following lengths are supported for the Identification Information in an AIoT Device Permanent Identifier: 96 bits, and 128 bits.

NOTE 4:	The encoding for the length of the Identification Information enables additional shorter or longer fixed lengths to be supported in the future.

5.8	Filtering Information

The filtering information is used to identify or filter multiple AIoT Devices and is constructed by one or multiple components of the AIoT Device Permanent Identifier as defined in clause 5.7.

The filtering information includes a list of filtering element information. The filtering element information includes:

-	Information indicating a which component of the AIoT Device Permanent Identifier that is used to match the bitstring.

-	A bitstring which is used to compare with the component.

Each bitstring is corresponding to one component of the AIoT Device Permanent Identifier.

NOTE:	The bitstring match can start at any position in the corresponding component.

Editor's note:	Whether and how to secure the filtering information is up to SA3.

To determine whether an AIoT Device Permanent Identifier matches the filtering information, it is compared with every filtering element information within filtering information by comparing the bitstring in a filtering element information with the indicated component of its AIoT Device Permanent Identifier. If all the compared bitstrings match the AIoT Device Permanent Identifier then an AIoT Device Permanent Identifier matches the filtering information. If an AIoT Device Permanent Identifier does not contain an indicated component then it does not match the filtering information.

6	AIoT Procedures

6.1	General

Editor’s Note: The contents of this general clause is FFS and is expected to describe there are service procedures which are reader type/routing agnostic and there are procedures which relate to the transports to the readers.

6.2	AIoT Service Procedures

6.2.1	General

Clause 6.2.2 provides the procedure for AIoT Inventory. Clause 6.2.3 provides the procedure for AIoT Command.

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires additional details, alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Connectivity and Indirect Connectivity options in the procedures.

6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [External Target Area information], information about the target AIoT Device(s), [Approximate number of AIoT Devices])  service operation request to the NEF.

The External Target Area information is specified in clause 5.3.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The approximate number of AIoT Devices, if provided, is used to determine the number of AIoT Devices expected to respond to this inventory service operation, which is sent by AIOTF to the NG-RAN in the assistance information for NG-RAN in step 7 for proper radio resource allocation.

Editor's note:	The parameters for the inventory service operation need further definition.

2.	The NEF may further authorize the AF request as specified in clause 5.6.

The NEF determines the Target Area information, and selects one or multiple AIOTF(s) to handle the request as specified in clause 5.3.1.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF(s).

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF may perform authorization as specified in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a Correlation ID corresponding to this AF service operation request.

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier.

AIOTF performs Reader Selection, see clause 5.3.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Identification Information to be included in the paging message, and assistance information to the selected NG-RAN.

Editor's note:	Whether the Inventory Request sent to NG-RAN includes indication about whether there will be a follow up command or not needs to be determined.

8.	The NG-RAN sends an Inventory Response to the AIOTF with the Correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message from the AIOTF, the RAN Reader(s) will execute the inventory operation. The RAN Reader(s) broadcast the paging message that includes the AIoT Identification Information.

The AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8.

If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10.	NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s). The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s).

NOTE:	When to erase the stored mapping between the Reader ID and AIoT device ID(s) is up to implementation and local configuration.

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	Optionally, if the NG-RAN detects that no more AIoT Devices will respond to the inventory procedure, the NG-RAN informs the AIOTF that the procedure is complete.

Editor’s Note:	The details how interactions between NG-RAN and AIOTF to indicate no further Inventory Reports will be sent and completion of the procedure need to be aligned with RAN.

13.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the Naiotf_AIoT_Notify message including a list of AIoT Device Permanent Identifier (s). The AIOTF may send multiple reports. The AIOTF in the final Naiotf_AIoT_Notify message indicates it is the last report for this operation. If multiple AIOTFs are involved in the procedure, the NEF may receive the AIoT_Notify from multiple AIOTFs.

14.	When receiving the Naiotf_AIoT_Notify message from AIOTF, the NEF informs the AF of the outcome of the AIoT_Inventory request by sending the Nnef_AIoT_Notify message(s) including the AIoT Device Permanent Identifier(s). The NEF in the final Nnef_AIoT_Notify message indicates that it is the last report for this operation.

6.2.3	Command Procedure

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Connectivity and Indirect Connectivity options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command (in case of untrusted AF) Request (AF ID, Command Type, AIoT Device identification information, [External Target Area information],  [Approximate number of AIoT Devices], [Approximate D2R message size], AIoT data) message to NEF.

Editor's note:	It is FFS whether and how to structure the AIoT data if the Command Type is Read, Write or Disable.

The AIoT Device identification information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

2.	The NEF selects the AIOTF(s) as described in clause 5.3.1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code and step 6 is performed before ending the procedure.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, AIoT Device identification information, [Target area information], , [Approximate number of AIoT Devices], [Approximate D2R message size], AIoT data) message to the selected AIOTF.

4.	The AIOTF receives the AIoT command operation request and checks the parameters included in the request. The AIOTF performs NG- RAN and optionally RAN Reader selection as specified in clause 5.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

The AIOTF determines assistance information as described in clause 5.4.

The AIOTF performs AF authorization for AIoT command operation as described in clause 5.6.

The AIOTF performs AMF selection as described in clause 5.3.4.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 11, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information.

If none of successful Inventory response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in Step 12.

8.	For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID],  NAS Command Request, [Approximate D2R message size]) to the NG-RAN directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data.

Editor's note:	What parameter(s) are used in the Command Request to enable NG-RAN node to target a specific AIoT Device requires coordination with RAN3.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

9.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device.

10.	The AIoT Device sends the AS D2R message (NAS Command Response) to the NG-RAN. The NAS Command Response message may include the AIoT data.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

11.	The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response) to the AIOTF directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4.

12.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Naiotf_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

13.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

6.2.4	Procedures between AIOTF and NG-RAN for Indirect Connectivity

An AIOTF and NG-RAN can use an indirect interface via an AMF as described in clause 4.2.2.4. The additional steps used for indirect interface between AIOTF and NG-RAN are shown in Figure 6.2.4-1.

Figure 6.2.4-1: Procedure for AIOTF and NG-RAN for indirect connectivity via an AMF

1.	The AIOTF sends Namf_AIoT_MessageDelivery message (AIoT Transfer Container, NG-RAN ID, AIOTF ID, Message Type for AIoT Transfer Container) to the AMF. The AIoT Transfer Container may be Inventory Request Transfer or Command Request Transfer.

2.	The AMF sends an NGAP message (AIOTF ID, AIoT Transfer Container) to the target NG-RAN.

3.	The NG-RAN sends an NGAP message (AIOTF ID, AIoT Transfer Container) to an AMF. AIoT Transfer Container may be Inventory Response Transfer, Inventory Report Transfer, Inventory Failure Transfer, Command Response Transfer or Command Failure Transfer.

4.	AMF sends the Namf_AIoT_Notify message (AIoT Transfer Container) to the AIOTF.

7	Network Functions Services

7.1	General

7.2	AIOTF services

7.2.1	General

The AIOTF supports to expose AIoT services towards the AF or the NEF as described in Table 7.2.1-1.

Table 7.2.1-1: NF services provided by the AIOTF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naiotf_AIoT | Inventory | Request/Response | NEF, AF |
|  | Command | Request/Response | NEF, AF |
|  | Notify | Subscribe/Notify | NEF, AF |

Editor's note:	It is FFS how to support the service between AMF and AIOTF in order to report and update the information provided by the RAN for indirect connectivity.

Editor's note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

7.2.2	Naiotf_AIoT_Inventory service operation

Service operation name: Naiotf_AIoT_Inventory.

Description: The NF consumer requests an inventory operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the inventory operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information for multiple AIoT Devices.

Inputs, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

7.2.3	Naiotf_AIoT_Command service operation

Service operation name: Naiotf_AIoT_Command.

Description: The NF consumer requests a command operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the command operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information for multiple AIoT Devices.

4)	Command Type: Read, Write or Permenant Disable.

5)	Command specific parameters:

Editor’s note:	The AIoT Data specific for the Read/Write/Disable command is FFS.

Editor’s note:	The Command specific parameters for Read/Write/Disable command is FFS.

Inputs, Optional:

1)	Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

-	Approximate message size from the AIoT Device for Read Operation.

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also be provided by the AF for the other command operations.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

7.2.4	Naiotf_AIoT_Notify service operation

Service operation name: Naiotf_AIoT_Notify

Description: The AIOTF uses this service operation to notify the results or status of the service operation towards the NF consumers. If the NF consumer invokes the Naiotf_AIoT_Inventory, or Naiotf_AIoT_Command service operation, the NF consumer implicitly subscribes to the results of the requested service operation.

Inputs, Required:

1)	Common report information: Transaction ID, List of AIoT Device ID or Failure Cause in case of Failure).

2)	Read command specific report information: Information obtained from each target AIoT Device.

Editor’s note:	It is FFS whether that fail to discovery any AIoT device is normal operation result or failure case.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Editor’s note:	It is FFS how to make AF be aware of the report is finished in case there are multiple notify for the same AIoT operation.

7.3	AMF services

7.3.1	General

AMF supports to expose AIoT services towards the AIOTF as described in Table 7.3.1-1. The Namf_AIoT AMF service is used when NG-RAN and the AIoTF communicate indirectly via an AMF.

Table 7.3.1-1: NF services provided by the AMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Namf_AIoT | MessageDelivery | Request/Response | AIOTF |
|  | Notify | Subscribe/Notify | AIOTF |

7.3.2	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: The NF consumer requests to send AIoT data (i.e. AIoT inventory/command messages) towards NG-RAN or AIoT devices.

Inputs, Required:

1)	AIoT messages to deliver to NG-RAN.

2)	NG-RAN ID.

Outputs, Required: AIoT messages received from NG-RAN, Result indication (Success or Failure), Failure Cause in case of Failure.

Editor’s note:	It is FFS whether the Correlation identifier or AIOTF ID is included in the Namf_AIoT_MessageDelivery.

7.3.3	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: The NF consumer requests to receive AIoT data from NG-RAN or AIoT devices. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the AIoT data from NG-RAN or AIoT devices.

Inputs, Required:

1)	AIoT messages received from NG-RAN.

Input, Optional: None.

Outputs, Required: Operation execution result indication.

Output, Optional: None.

Editor's note:	AIoT messages is FFS and coordination with RAN3 is needed.

Editor's note:	It is FFS how to make AIOT be aware of the report is finished in case there are multiple notify for the same operation.

Editor's note:	It is FFS whether the Correlation identifier is included in the Namf_AIoT_Notify.

7.4	NEF services

7.4.1	General

In addition to those defined in clause 7.2.8 of TS 23.501 [3] and clause 5.2.6 of TS 23.502 [4], table 7.4.1-1 illustrates additional NEF services to support AIoT.

Table 7.4.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoT | Inventory | Request/Response | AF |
|  | Command | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

Editor’s note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

7.4.2	Nnef_AIoT_Inventory service operation

Service operation name: Nnef_AIoT_Inventory

Description: The consumer requests to perform an inventory operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the inventory operation.

Input, Optional: Approximate number of AIoT Devices.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.3	Nnef_AIoT_Command service operation

Service operation name: Nnef_AIoT_Command

Description: The consumer requests to perform a command operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the command operation, Command type (Read, Write, or Disable).

Editor’s note:	The AIoT Data specific for the Read/Write/Disable command is FFS.

Input, Optional: Approximate number of AIoT Devices, Approximate message size from the AIoT Device for Read command type.

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also apply for the other command types.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.4	Nnef_AIoT_Notify service operation

Service operation name: Nnef_AIoT_Notify

Description: The consumer receives notification of the status or results of the requested service operation. If the consumer invokes the Nnef_AIoT_Inventory, or Nnef_AIoT_Command service operation, the consumer implicitly subscribes to the results of the requested service operation.

Input, Required: AF Transaction ID, a list of AIoT Device ID(s) or Failure Cause in case of Failure, Information obtained from each target AIoT Device.

Input, Optional: None.

Output, Required: Result indication.

Output, Optional: None.

Editor’s note:	The NEF AIoT service operation definition needs to be revisited to align with AIOTF services.

7.5	ADM services

7.5.1	General

The following table shows the Nadm_DM Service and Service Operations.

Table 7.5.1-1: NF services provided by ADM

| NF service name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadm_DM | Query | Request/Response | AIOTF, NEF |
|  | Update | Request/Response | AIOTF |

7.5.2	Nadm_DM_Query service operation

Service operation name: Nadm_DM_Query

Description: NF service consumer may request the AIoT device profile data or the AF authorization data from the ADM.

Inputs, Required: AIoT Device Permanent ID, or AF ID.

Input, Optional: None.

Outputs, Required: the AIoT device profile data or the AF authorization data.

Output, Optional: None.

Editor's note:	Whether AF authorization data is stored in ADM is FFS.

7.5.3	Nadm_DM_Update service operation

Service operation name: Nadm_DM_Update

Description: NF service consumer may update the AIoT device profile data in the ADM.

Inputs, Required: AIoT Device Permanent ID, updated AIoT device profile data.

Input, Optional: None.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

Output, Optional: None.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-01 | SA2#166 AH-e | S2-2501256 | - | - | - | Proposed skeleton agreed at SA2#166AH-e | 0.0.0 |
| 2025-01 | SA2#166 AH-e | - | - | - | - | Inclusion of documents approved in SA2#166AH-e: S2-2501253, S2-2501254, S2-2501255, S2-2501257, S2-2501258. | 0.1.0 |
| 2025-02 | SA2#167 | - | - | - | - | Inclusion of documents approved in SA2#167: S2-2502368, S2-2502555, S2-2502564, S2-2502578, S2-2502579, S2-2502581, S2-2502582, S2-2502585, S2-2502586, S2-2502588, S2-2502589, S2-2502590 | 0.2.0 |
| 2025-04 | SA2#168 | - | - | - | - | Inclusion of documents approved in SA2#168: S2-2504012, S2-2504013, S2-2504063, S2-2504275, S2-2504280, S2-2504281, S2-2504283, S2-2504286, S2-2504287, S2-2504289, S2-2504290, S2-2504291, S2-2504292, S2-2504318, S2-2504319, S2-2504320, S2-2504321 | 0.3.0 |