| 3GPP TS 23.392 V0.1.0 (2024-07) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Application enablement aspects for MMTel; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview	7

5.	Architectural requirements	7

6.	Architecture	7

6.1	General	7

6.2	MMTel Enabler architecture	7

6.3	Functional entities	8

6.3.1	General	8

6.3.2	MMTel Enabler Server	9

6.3.3	MMTel Enabler Client	9

6.3.4	Application Server	9

6.3.5	Controlling Application Server	10

6.3.6	Application Client	10

6.4	Reference points	10

6.4.1	General	10

6.4.2	MMTel-1 (between the MMTel Enabler client and the MMTel Enabler server)	10

6.4.3	MMTel-2 (between the MMTel Enabler server and the Application Server)	10

6.4.4	MMTel-3 (between the MMTel Enabler server and the Controlling Application Server)	10

6.4.5	MMTel-4 (between the MMTel Enabler client and the DCMTSI client)	10

6.4.6	App-1 (between the Application Client and the Application Server)	11

7.	Identities	11

8.	Procedures and information flows	11

9.	Deployment Guideline	11

Annex A: Change history	11

S6#62-Ad Hoc-e

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document specifies the application layer architecture and procedures to support the MMTel service, and also specifies the service enabler layer capability exposure procedures and related APIs for enablement aspects of MMTel service

This work takes into consideration the existing stage 1 and stage 2 work within 3GPP related to MMTel service specified in 3GPP TS 22.261 [2], 3GPP TS 23.228 [3] and 3GPP TS 26.114 [4].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[4]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction"

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

MMTel service: An MTSI service as specified in 3GPP TS 26.114 [4] with or without Data channel support.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 26.114 [4] apply:

DCMTSI client

DCMTSI client in terminal

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.228 [3] apply:

Application data channel

Bootstrap data channel

Data channel application

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

DC	Data Channel

DCMF	Data Channel Media Function

DCMTSI	Data Channel Multimedia Telephony Service for IMS

IMS	IP Multimedia Core Network Subsystem

OMA	Open Mobile Alliance

SIP	Session Initiation Protocol

# 4	Overview

Editor's note:	This clause will describe the functionality overview for MMTelAPP service..

.

# 5	Architectural requirements

Editor's Note: This clause provides the all architectural requirements identified.

# 6	Architecture

Editor's Note: This clause provides the architecture of MMTelAPP Enabler.

## 6.1	General

This clause specifies the architecture of MMTel Enabler.

## 6.2	MMTel Enabler architecture

Figure 6.2-1 shows the MMTel Enabler architecture.

The MMTel Enabler Server is hosted within the PLMN operator network which supports the MMTel services capabilities exposure.

Figure 6.2.1-1: MMTel Enabler architecture

The MMTel functional entities on the UE and the server sides are MMTel Enabler client and MMTel Enabler server respectively.

The MMTel Enabler client communicates with the MMTel Enabler server over the MMTel-1 reference point to support the application layer interactions needed in MMTel service.

NOTE	The interaction between MMTel Enabler Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

The MMTel Enabler client also communicates with the DCMTSI client over the MMTel-4 reference point to downloading behavior on DCMTSI Client based on the application policy provided by the MMTel Enabler Server. The DCMTSI client is specified in 3GPP TS 26.114 [4].

The Application Server communicates with the MMTel Enabler server over the MMTel-2 reference point to invoke the MMTel service related capabilities provided by MMTel Enabler server,

The Controlling Application Server communicates with the MMTel Enabler server over the MMTel-3 reference point to manage the information needed in MMTel service.

The Application Server communicates with the Application Client on the UE over the App-1 reference point to provide the Data Channel Application specific logic.

## 6.3	Functional entities

### 6.3.1	General

The functional entities for MMTel Enabler are described in the following subclauses.

Editor's note:	 whether more functionalities of the MMTel Enabler client and the MMTel Enabler server are needed is FFS.

### 6.3.2	MMTel Enabler Server

The MMTel Enabler Server provides the server side functionalities corresponding to the MMTel service.

The MMTel Enabler Server supports:

-	the Application Server and Controlling Application Server to discover, to authenticate and to get authorization the MMTel/MMTel service APIs provided by the MMTel Enabler server;

-	the interaction with Controlling Application Server to configure the data channel application profile;

-	the integration of the multiple network layer capabilities provided by the underlying network, including the 3GPP core network and IMS into one service enabler layer API. The MMTel Enabler Server exposes these service enabler layer APIs to the Application Server and Controlling Application Server instead of exposes the network layer capabilities directly. The capabilities provided by the underlying network include:

a)	the APIs provided by DCSF via DC3/N33 reference point;

b)	the APIs provided by DCSF via DC4 reference point; and

c)	the APIs provided by MF/MRF via MDC3/MDC4 reference points;

-	the communication with the corresponding functions in the underlying network on behalf of the Application Server and Controlling Application Server when the capabilities provided by the underlying network are needed to be invoked;

-	the interaction with MMTel Enabler client for exchanging service configuration related information, e.g. the data channel application profile, needed in MMTel service,

-	providing MMTel/MMTel specific value added services which can be used by the Application Server in the mmtel communication between UE and Application Server;

-	the interaction with SEAL functionalities to use the corresponding SEAL services in MMTel service, e.g. to use  SEAL notification management service to update of data channel application profiles to UE;

-	the storage of the application layer information associated with the MMTel services;

-	the media function to support the media handling/translation, e.g. acts as a media gateway of WebRTC or if supported media type of application is different with the UE; and

-	the HTTP Proxy to support the establishes a Bootstrap data channel and application data channel, e.g. if the address type of application is different with the UE, e.g. IPv4 and IPv6, etc.

Editor's note:	Whether other functionalities, e.g. function to handle MMTel service related Signalling, are needed in the MMTel Enabler Server is FFS.

### 6.3.3	MMTel Enabler Client

The MMTel Enabler Client provide the client side functionality needed in MMTel service.

The MMTel Enabler Client interact with DCMTSI Client in the same UE to control the downloading behavior on DCMTSI Client based on the data channel application profiles provided by the MMTel Enabler Server.

NOTE 1:	the DCMTSI client is responsible for creating bootstrap channel and actually downloading the data channel applications.

NOTE 2	The interaction between MMTel Enabler Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

### 6.3.4	Application Server

The Application Server communicates with the MMTel Enabler server over the MMTel-2 reference point to invoke the MMTel service related capabilities to communicate with the UE, e.g. establishes a MMTel session with or without IMS data channel to the UE, or establishes an application data channel to the UE. It may also communicate with Application Client to provide Application specific logic.

NOTE:	The Application Server needs to support MMTel-2 reference point, but the detailed functionalities of the Application Server is out of scope of the present document.

### 6.3.5	Controlling Application Server

The Controlling Application Server is a specific Application Server that only used by the MMTel service provider, i.,e. MNO, to manage the service configuration related information, e.g. data channel application profiles, needed in MMTel service. It communicates with the MMTel Enabler server over the MMTel-3 reference point to invoke the MMTel service related capabilities and does not interacts with Application Client on the UE directly.

NOTE:	The functionalities of the Controlling Application Server is out of scope of the present document.

Editor's note:	 Whether the controlling AS functionalities can be provided by OAM or not is FFS and may require coordination with SA5.

### 6.3.6	Application Client

The Application Client is an entity in the application layer to implement and perform the application service logic for its own service.

NOTE:	The Application Client can be the data channel application client. The functionalities of the Application Client is out of scope of the present document.

## 6.4	Reference points

### 6.4.1	General

The reference points for MMTel Enabler are described in the following subclauses.

Editor's note:	 The functions of the reference points are FFS and to be updated per solution.

### 6.4.2	MMTel-1 (between the MMTel Enabler client and the MMTel Enabler server)

The MMTel-1 reference point supports the interactions between a MMTel Enabler client and the corresponding MMTel Enabler server. This reference point supports the exchange of service configuration related information needed in MMTel service.

### 6.4.3	MMTel-2 (between the MMTel Enabler server and the Application Server)

The interactions between the Application Server and the underlying network are provided by the MMTel Enabler server via MMTel-2 reference point to ensure the security access of the underlying network. This reference point supports the Application Server to discover MMTel service APIs, to authenticate, to get authorization and to communicate with the MMTel service.

### 6.4.4	MMTel-3 (between the MMTel Enabler server and the Controlling Application Server)

The interactions between the MMTel Enabler server and the Controlling Application Server are referred to MMTel-3 reference point. This reference point supports the Controlling Application Server to manage the service configuration related  information needed in MMTel service.

### 6.4.5	MMTel-4 (between the MMTel Enabler client and the DCMTSI client)

The interaction between MMTel Enabler Client and DCMTSI Client.are referred to MMTel-4 reference point. This reference point is internal interaction in the dialler application on the UE and implementation specific.

NOTE:	The functionalities of the MMTel-4 reference point is out of scope of the present document.

### 6.4.6	App-1 (between the Application Client and the Application Server)

The App-1reference point supports the communication between  Application Client and Application Server to provide the Data Channel Application specific logic.

NOTE:	The functionalities of the App-1 reference point is out of scope of the present document.

# 7	Identities

Editor's Note: This clause provides the all general identities for MMTelAPP Enabler.

# 8	Procedures and information flows

Editor's Note: This clause provides the all procedures and information flows for MMTelAPP Enabler.

# 9	Deployment Guideline

Editor's note:	This clause captures deployment guidelines for MMTelAPP Enabler if needed.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-07 | SA6#62-Ad Hoc-e |  | - | - | - | Proposed skeleton approved at S6#62-Ad Hoc-e | 0.0.0 |
| 2023-07 | SA6#62-Ad Hoc-e |  |  |  |  | Implemented pCRs approved in S6#62-Ad Hoc-e: S6a240100, S6a240101, S6a240102, S6a240103, S6a240282 Editorial changes by the rapporteur | 0.1.0 |