| 3GPP TS 23.392 V1.1.0 (2025-02) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Application enablement aspects for MMTel; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Overview	9

5	Architectural requirements	10

5.1	General	10

5.2	Data Channel Application management requirements	10

5.2.1	Description	10

5.2.2	Requirements	10

5.3	MMTel service enablement requirements	10

5.3.1	Description	10

5.3.2	Requirements	10

6	Architecture	10

6.1	General	10

6.2	MMTel Enabler architecture	11

6.2.1	General	11

6.2.2	MMTel Enabler Server architecture model supporting IMS Session Management	12

6.3	Functional entities	13

6.3.1	General	13

6.3.2	MMTel Enabler Server	13

6.3.3	MMTel Enabler Client	13

6.3.4	Application Server	14

6.3.5	Controlling Application Server	14

6.3.6	Application Client	14

6.3.7	SEAL Client	14

6.3.8	SEAL server	14

6.4	Reference points	14

6.4.1	General	14

6.4.2	MMTel-1 (between the MMTel Enabler Client and the MMTel Enabler Server)	14

6.4.3	MMTel-2 (between the MMTel Enabler Server and the Application Server)	14

6.4.4	MMTel-3 (between the MMTel Enabler Server and the Controlling Application Server)	15

6.4.5	MMTel-4 (between the MMTel Enabler Client and the DCMTSI client)	15

6.4.6	App-1 (between the Application Client and the Application Server)	15

6.4.7	SEAL-C	15

6.4.8	SEAL-S	15

6.4.9	SEAL-UU	15

7	Identities	15

7.1	General	15

7.2	Application ID (APPID)	15

8	Procedures and information flows	16

8.1	General	16

8.2	DC application and profile management	16

8.2.1	General	16

8.2.2	DC application and profile configuration	16

8.2.2.1	General	16

8.2.2.2	Procedure	16

8.2.2.3	Information flows	17

8.2.2.3.1	DC application and profile configuration request	17

8.2.2.3.2	DC application and profile configuration response	19

8.2.3	DC application and profile update	19

8.2.3.1	General	19

8.2.3.2	Procedure	19

8.2.3.3	Information flows	20

8.2.3.3.1	DC application and profile update request	20

8.2.3.3.2	DC application and profile update response	22

8.2.4	DC application deletion	22

8.2.4.1	General	22

8.2.4.2	Procedure	22

8.2.4.3	Information flows	23

8.2.4.3.1	DC Application delete request	23

8.2.4.3.2	DC Application delete response	23

8.2.5	Getting DC application profile information	23

8.2.5.1	General	23

8.2.5.2	Procedure	23

8.2.5.3	Information flows	24

8.2.5.3.1	Getting DC application profile information request	24

8.2.5.3.2	Getting DC application profile information response	24

8.3	Control the downloading of data channel applications	25

8.3.1	General	25

8.3.2	DC application profiles downloading on UE	25

8.3.2.1	General	25

8.3.2.2	Procedure	25

8.3.2.3	Information flows	26

8.3.2.3.1	Get Root application request	26

8.3.2.3.2	Get Root application response	26

8.3.2.3.3	Get DC application profile List request	27

8.3.2.3.4	Get DC application profile List response	27

8.3.3	DC application profiles updating on UE	30

8.3.3.2	Procedure	30

8.3.3.2.1	The MMTel Enabler Client request DC application profiles based on notification	30

8.3.3.2.2	The MMTel Enabler Server notifies the DC application profile to the MMTel Enabler Client	31

8.3.3.3	Information flows	32

8.3.3.3.1	Root application and profile update notification	32

8.3.3.3.2	Root application notification	32

8.4	Application calling enablement	32

8.4.1	General	32

8.4.2	Application calling service API with Data Channel capability	32

8.4.2.1	General	32

8.4.2.2	Procedure	32

8.4.2.2.1	Establishing an Application Call with Data Channel capability between non-IMS application and DCMTSI client	32

8.4.2.3	Information flows	33

8.4.2.3.1	Application Call establishment request	33

8.4.2.3.2	Application Call establishment response	33

8.4.3	Third-Party Call service API with Data Channel capability	34

8.4.3.1	General	34

8.4.3.2	Procedure	34

8.4.3.2.1	Establishing a Third-Party Call with Data Channel capability	34

8.4.3.3	Information flows	35

8.4.3.3.1	Third-Party Call with DC capability establishment request	35

8.4.3.3.2	Third-Party Call with DC capability establishment response	35

8.4.4	Establishing an Application Call with Data Channel capability (A2P)	36

8.4.4.1	Procedure	36

8.4.4.2	Information flows	38

8.4.4.2.1	Application Add Data Chanel media to UE1 session (A2P) request	38

8.4.4.2.2	Application Add Data Chanel media to UE1 session (A2P) response	38

8.4.4.2.3	Notify Application of DCH media info (A2P) request	38

8.4.4.2.4	Notify Application of DCH media info (A2P) response	39

8.4.5	Application calling service API with Data Channel capability	39

8.4.5.1	Procedure	39

8.4.5.1.1	Establishing an Application Call with Data Channel capability (P2A)	39

8.4.5.2	Information flows	40

8.4.5.2.1	Notify DCH Application of MDC2 media (P2A) request	40

8.5	Multiple call control handling coordination among different Application Servers	41

8.5.1	General	41

8.5.2	Procedure	41

8.5.3	Information flows	42

8.5.3.1	call control result notification	42

9	Deployment Guideline	42

9.1	General	42

9.2	MMTel Enabler server deployed in PLMN operator domain	42

Annex A: Change history	44

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document specifies the application layer architecture and procedures to support the MMTel service, and also specifies the service enabler layer capability exposure procedures and related APIs for enablement aspects of MMTel service

This work takes into consideration the existing stage 1 and stage 2 work within 3GPP related to MMTel service specified in 3GPP TS 22.261 [2], 3GPP TS 23.228 [3] and 3GPP TS 26.114 [4].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[4]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction"

[5]	GSMA NG.134: "IMS Data Channel, Version 1.0".

[6]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL)".

[7]	OMA-TS-REST_NetAPI_ThirdPartyCall-V1_0-20130212-C: "RESTful Network API for Third Party Call".

[8]	OMA-AD-IMS-V1_0-20050809-A: "Utilization of IMS capabilities Architecture".

[9]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[10]	IETF RFC 7989 (October 2016): "End-to-End Session Identification in IP-Based Multimedia Communication Networks".

[11]	OMA-TS-REST_NetAPI_Common-V1_0-20180116-A: "Common definitions for RESTful Network APIs".

[12] 	3GPP TS 23.502: " Procedures for the 5G System (5GS); Stage 2".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

MMTel service: An MTSI service as specified in 3GPP TS 26.114 [4] with or without Data channel support.

Root application: A root page of of the Data Channel Application that the subscriber could select and use, as specified in GSMA NG.134 [5].

For the purposes of the present document, the following terms and definitions given in 3GPP TS 26.114 [4] apply:

DCMTSI client

DCMTSI client in terminal

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.228 [3] apply:

Application data channel

Bootstrap data channel

Data channel application

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

DC	Data Channel

DCMTSI	Data Channel Multimedia Telephony Service for IMS

IMS	IP Multimedia Core Network Subsystem

OMA	Open Mobile Alliance

SIP	Session Initiation Protocol

# 4	Overview

The MMTel Enabler provides the Application enablement for MMTel service, mainly including the following aspects:

1	DC Application management: providing the management capabilities of Data Channel Applications and data channel application profile to PLMN operators which provide MMTel service with IMS Data Channel

2	DC Application downloading control: providing the capabilities to control the downloading of Data Channel applications on UE to PLMN operators which provide MMTel service with IMS Data Channel. The downloading is controlled by the data channel application profile downloaded to the UE.

3	usage of MMTel service: providing capabilities to application providers/Vertical service provider to use MMTel service including:

a.	call between application and DCMTSI client:

b.	Third-Party Call;

c.	Person-to-Application (P2A) Application Data Channel; and

d.	Application-to-Person (A2P) Application Data Channel.

The DC Application management is specified in clause 8.2, DC Application downloading control is specified in clause 8.3, and usage of MMTel service is specified in clause 8.4.

# 5	Architectural requirements

## 5.1	General

This clause specifies architectural requirements for MMTel Enabler.

## 5.2	Data Channel Application management requirements

### 5.2.1	Description

This clause specifies the Data Channel Application management requirements for MMTel Enabler.

### 5.2.2	Requirements

[AR-5.2.2-a] The MMTel Enabler shall provide capability for MMTel service provider to upload, update and delete the Data Channel applications on DCAR via MMTel Enabler .

[AR-5.2.2-b] The MMTel Enabler shall provide capability for MMTel service provider to configure, update, get and deleted the Data Channel application profile on DCAR via MMTel Enabler server.

[AR-5.2.2-c] The MMTel Enabler shall provide capability for Data Channel application provider to get the Data Channel application profile information.

[AR-5.2.2-d] The MMTel Enabler shall provide capability to control the downloading of data channel applications on UE by providing data channel application profile to UE.

## 5.3	MMTel service enablement requirements

### 5.3.1	Description

This clause specifies the MMTel service enablement requirements for MMTel Enabler.

### 5.3.2	Requirements

[AR-5.3.2-a] The MMTel Enabler shall provide capability for application without IMS capabilities to establish, modify or terminate a call with an Application supporting MMTel service.

[AR-5.3.2-b] The MMTel Enabler shall provide capability for application providers/Vertical service provider to use MMTel service including establish, modify or terminate a:

a.	call between application and DCMTSI client:

b.	Third-Party Call;

c.	Person-to-Application (P2A) Application Data Channel; and

d.	Application-to-Person (A2P) Application Data Channel

# 6	Architecture

## 6.1	General

This clause specifies the architecture of MMTel Enabler.

## 6.2	MMTel Enabler architecture

### 6.2.1	General

Figure 6.2-1 shows the MMTel Enabler architecture.

In this release, the MMTel Enabler Server is hosted within the PLMN operator network which supports the MMTel services capabilities exposure.

Figure 6.2.1-1: MMTel Enabler architecture

The MMTel functional entities on the UE and the server sides are MMTel Enabler Client and MMTel Enabler Server respectively.

The MMTel Enabler Client communicates with the MMTel Enabler Server over the MMTel-1 reference point to support the application layer interactions needed in MMTel service.

NOTE	The interaction between MMTel Enabler Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

The MMTel Enabler Client also communicates with the DCMTSI client over the MMTel-4 reference point to downloading behavior on DCMTSI Client based on the application policy provided by the MMTel Enabler Server. The DCMTSI client is specified in 3GPP TS 26.114 [4].

The Application Server communicates with the MMTel Enabler Server over the MMTel-2 reference point to invoke the MMTel service related capabilities provided by MMTel Enabler Server,

The Controlling Application Server communicates with the MMTel Enabler Server over the MMTel-3 reference point to manage the information needed in MMTel service.

The Application Server communicates with the Application Client on the UE over the App-1 reference point to provide the Data Channel Application specific logic.

The MMTel Enabler Client(s) interacts with SEAL Clients over the SEAL-C reference point specified for each SEAL service. The MMTel Enabler Server(s) interacts with SEAL Servers over the SEAL-S reference point specified for each SEAL service. The interaction between a SEAL Client and the corresponding SEAL Server is supported by SEAL-UU reference point specified for each SEAL service as specified in 3GPP TS 23.434 [6].

### 6.2.2	MMTel Enabler Server architecture model supporting IMS Session Management

The MMTel Enabler Server provides MMTel services capabilities including IMS Session Management capabilities and Data Channel related capabilities to Application Server.

NOTE 1:	The MMTel Enabler Server architecture model supporting IMS Session Management will be updated after the IMS Session management capabilities provided by NEF is specified by SA2.

Figure 6.2.2-2: Option2: MMTel Enabler Server supports OMA OSE

Figure 6.2.2-1 shows the relationship between MMTel Enabler Server and IMS. The MMTel Enabler Server supports OMA Enabler, and interacts with IMS by using either:

a.	IMS elements interfaces specified in.3GPP TS 23.228 [3] and OMAIMS [8]; or

b.	implementation specified interfaces.

NOTE 2:	The detailed interaction between MMTel Enabler Server and IMS is out of scope of the present document.

NOTE 3:	The architecture to support the IMS Session Management will be updated if new capabilities are provided by IMS.

The MMTel Enabler Server interacts with IMS via N33/DC4/MDC2/MDC3 reference points to utilize the Data Channel related capabilities provided by IMS as specified in 3GPP TS 23.228 [3].

## 6.3	Functional entities

### 6.3.1	General

The functional entities for MMTel Enabler are described in the following subclauses.

### 6.3.2	MMTel Enabler Server

The MMTel Enabler Server provides the server side functionalities corresponding to the MMTel service.

The MMTel Enabler Server supports:

-	the Application Server and Controlling Application Server to discover, to authenticate and to get authorization the MMTel/MMTel service APIs provided by the MMTel Enabler Server;

-	the interaction with Controlling Application Server to configure the DC application profile;

-	the integration of the multiple network layer capabilities provided by the underlying network, including the 3GPP core network and IMS into one service enabler layer API. The MMTel Enabler Server exposes these service enabler layer APIs to the Application Server and Controlling Application Server instead of exposes the network layer capabilities directly. The capabilities provided by the underlying network include:

a)	the APIs provided by DCSF via DC3/N33 reference point;

b)	the APIs provided by DCSF via DC4 reference point; and

c)	the APIs provided by MF/MRF via MDC3/MDC4 reference points;

NOTE: in this release, the MMTel Enabler Server interacts with IMS via IMS elements interfaces specified in 3GPP TS 23.228 [3] or implementation specified interfaces;

-	the communication with the corresponding functions in the underlying network on behalf of the Application Server and Controlling Application Server when the capabilities provided by the underlying network are needed to be invoked;

-	the interaction with MMTel Enabler Client for exchanging service configuration related information, e.g. the DC application profile, needed in MMTel service,

-	providing MMTel/MMTel specific value added services which can be used by the Application Server in the mmtel communication between UE and Application Server;

-	the interaction with SEAL functionalities to use the corresponding SEAL services in MMTel service, e.g. to use SEAL notification management service to update of DC application profiles to UE;

-	the storage of the application layer information associated with the MMTel services;

-	the media function to support the media handling/translation, e.g. acts as a media gateway of WebRTC or if supported media type of application is different with the UE;

-	the HTTP Proxy to support the establishes a Bootstrap data channel and application data channel, e.g. if the address type of application is different with the UE, e.g. IPv4 and IPv6, etc;

-	providing call control handling consolidation when receiving call control requests from multiple Application Servers, e.g. to merge same control commands, to reject call control requests from certain Application Server with low priority when conflict happens or to decide the call control request execution sequence, etc; and

-	acting as CAPIF API provider domain functions as specified in 3GPP TS 23.222 [9] and interacts with CAPIF core function via CAPIF-3/3e/4/4e/5/5e reference points.

### 6.3.3	MMTel Enabler Client

The MMTel Enabler Client provide the client side functionality needed in MMTel service.

The MMTel Enabler Client interact with DCMTSI Client in the same UE to control the downloading behavior on DCMTSI Client based on the DC application profiles provided by the MMTel Enabler Server.

NOTE 1:	the DCMTSI client is responsible for creating bootstrap channel and actually downloading the data channel applications.

NOTE 2	The interaction between MMTel Enabler Client and DCMTSI Client.is internal interaction in the dialler application on the UE and implementation specific.

### 6.3.4	Application Server

The Application Server communicates with the MMTel Enabler Server over the MMTel-2 reference point to invoke the MMTel service related capabilities to communicate with the UE, e.g. establishes a MMTel session with or without IMS data channel to the UE, or establishes an application data channel to the UE. It may also communicate with Application Client to provide Application specific logic.

NOTE:	The Application Server needs to support MMTel-2 reference point, but the detailed functionalities of the Application Server is out of scope of the present document.

### 6.3.5	Controlling Application Server

The Controlling Application Server is a specific Application Server that only used by the MMTel service provider, i.,e. MNO, to manage the service configuration related information, e.g. DC application profiles, needed in MMTel service. It communicates with the MMTel Enabler Server over the MMTel-3 reference point to invoke the MMTel service related capabilities and does not interacts with Application Client on the UE directly.

NOTE:	The functionalities of the Controlling Application Server is out of scope of the present document.

### 6.3.6	Application Client

The Application Client is an entity in the application layer to implement and perform the application service logic for its own service.

NOTE:	The Application Client can be the data channel application client. The functionalities of the Application Client is out of scope of the present document.

### 6.3.7	SEAL Client

The following SEAL Clients for MMTel enablement service are supported:

-	Notification management client as specified in clause 17 of TS 23.434 [6];

### 6.3.8	SEAL server

The following SEAL servers for MMTel enablement service are supported:

-	Notification management server as specified in clause 17 of TS 23.434 [6];

## 6.4	Reference points

### 6.4.1	General

The reference points for MMTel Enabler are described in the following subclauses.

### 6.4.2	MMTel-1 (between the MMTel Enabler Client and the MMTel Enabler Server)

The MMTel-1 reference point supports the interactions between a MMTel Enabler Client and the corresponding MMTel Enabler Server. This reference point supports:

-	the MMTel Enabler Client to download the DC application profile from MMTel Enabler Server;

-	the MMTel Enabler Server notifies the MMTel Enabler Client in the UE to send application request messages to the MMTel Enabler Server to request the profiles of data channel applications; and

-	the MMTel Enabler Server sends the profiles to the UE directly after the UE subscribes to the profile notification

### 6.4.3	MMTel-2 (between the MMTel Enabler Server and the Application Server)

The interactions between the Application Server and the underlying network are provided by the MMTel Enabler Server via MMTel-2 reference point to ensure the security access of the underlying network. This reference point supports:

-	the Application Server to discover MMTel service APIs, to authenticate, to get authorization and to communicate with the MMTel service; and

-	to provide the MMTel calling API with Data Channel capability to the Applications to use the MMTel service.

### 6.4.4	MMTel-3 (between the MMTel Enabler Server and the Controlling Application Server)

The interactions between the MMTel Enabler Server and the Controlling Application Server are referred to MMTel-3 reference point. This reference point supports:

-	the Controlling Application Server to manage the service configuration related information needed in MMTel service; and

-	the Controlling Application Server to get the DC application profile information from the MMTel Enabler Server.

### 6.4.5	MMTel-4 (between the MMTel Enabler Client and the DCMTSI client)

The interaction between MMTel Enabler Client and DCMTSI Client. are referred to MMTel-4 reference point. This reference point is internal interaction in the dialler application on the UE and implementation specific.

NOTE:	The functionalities of the MMTel-4 reference point is out of scope of the present document.

### 6.4.6	App-1 (between the Application Client and the Application Server)

The App-1reference point supports the communication between Application Client and Application Server to provide the Data Channel Application specific logic.

NOTE:	The functionalities of the App-1 reference point is out of scope of the present document.

### 6.4.7	SEAL-C

The following SEAL-C reference points for MMTel enablement service are supported:

-	NM-C reference point for notification management as specified in 3GPP TS 23.434 [6];

### 6.4.8	SEAL-S

The following SEAL-S reference points for MMTel enablement service are supported:

-	NM-S reference point for notification management as specified in 3GPP TS 23.434 [6];

### 6.4.9	SEAL-UU

The following SEAL-UU reference points for MMTel enablement service are supported:

-	NM-UU reference point for notification management as specified in 3GPP TS 23.434 [6];

# 7	Identities

## 7.1	General

The following clauses list identities that are used in this technical specification.

## 7.2	Application ID (APPID)

The APPID is a globally unique value that identifies a Data Channel Application. It is assigned by the MMTel Enabler Server. The value of this IE is also used to set the "req-app-id" parameter in "a=3gpp-req-app" media-level SDP attribute as specified in 3GPP TS 26.114 [4] when the application requests application data channel(s).

# 8	Procedures and information flows

## 8.1	General

This clause provides procedures and information flows necessary for application enablement aspects for MMTel including information elements used in the procedures.

## 8.2	DC application and profile management

### 8.2.1	General

The following clauses specify procedures and information flows for DC application and profile management.

### 8.2.2	DC application and profile configuration

#### 8.2.2.1	General

DC application and profile configuration enables a Controlling Application Server to configure the DC application and profile with the MMTel Enabler Server.

#### 8.2.2.2	Procedure

Figure 8.2.2.2-1 illustrates the DC application and profile configuration procedure.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and configured with the necessary credentials to enable authenticating one another.

3.	Both DC application and profile have been uploaded to Controlling Application Server.

Figure 8.2.2.2-1: DC application and profile configuration

1.	The Controlling Application Server sends a DC application and profile configuration request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.2.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server initiates authentication procedures with the Requester Identity. If the registration is successful, the MMTel Enabler Server allocates an APPID and creates the DC application profile for each DC Application included in the DC application and profile configuration request, and maintain the mapping of APPID and DC Application Profile.

3.	The MMTel Enabler Server configures the DC application and profile on DCAR.

4.	The MMTel Enabler Server sends a DC application and profile configuration response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.2.3.2.

Editor's note:	 The detailed interactions between the MMTel Enabler Server and DCAR are FFS.

#### 8.2.2.3	Information flows

##### 8.2.2.3.1	DC application and profile configuration request

Table 8.2.2.3.1-1 describes information elements for the DC application and profile configuration request from the Controlling Application Server to the MMTel Enabler Server.

Table 8.2.2.3.1-1: Information elements in DC application and profile configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Number of DC Application included | M | Indicates total number of DC Application included in this request |
| DC Application list | M | A list of DC application and configuration parameters included in this request. Each element in this list contains a Data Channel Application package and corresponding configuration parameters. The detailed information elements of DC application are listed in Table 8.2.2.3.1-2. |

Table 8.2.2.3.1-2: Information elements in DC application list

| Information element | Status | Description |
| --- | --- | --- |
| Application Identifier | M | Identifier of the Data Channel Application assigned by the Application Provider |
| Application Name (see NOTE 1) | O | Name of the Data Channel Application |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application |
| Application Icon Url | O | Icon url of the Data Channel Application |
| Application Version (see NOTE 1) | O | The newest version of the application. |
| Application Validity | O | The latest validity of the application. |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: Precall only:the Data Channel Application is allowed to be used before the MMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. Incall:the Data Channel Application is allowed to be used after the MMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. Precall+Incall: the Data Channel Application is allowed to be used during the entire Precall and incall. |
| Autoload (see NOTE 3) | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch (see NOTE 3) | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. |
| If Work Without Peer DC (see NOTE 2) | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Supported Scenario (see NOTE 2) | O | Indicates supported media type required for this Data Channel Application. The values of this IE include: Voice call only: this Data Channel Application can be used if and only if the corresponding call is a voice call. Video call only: this Data Channel Application can be used if and only if the corresponding call is a video call. Voice and video call: this Data Channel Application can be used in both voice call and video call. |
| Condition (see NOTE 2) | O | Indicates whether this Data Channel Application can be used if under this condition. The values of this IE include but not limited: Service area: the application is allowed to be used in a certain area. Time period: the application is allowed to be used for a certain period of time. |
| 3gpp Qos Hint (see NOTE 2) | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| PersonalDataCollection (see NOTE 4) | O | Indicates whether this Data Channel Application will collect the user personal data (along with related consent opt-in/opt-out procedures). |
| PersonalDataCollectionInfoURL (see NOTE 4) | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected, their processing their protection etc. |
| Application Package (see NOTE 1) | O | The Code package of the Data Channel Application. |
| NOTE 1: This IE shall be present when when configuring the DC application. NOTE 2: This IE shall be present when when configuring the DC application profile. NOTE 3: These IEs can be applied only if the specific service using this data channel application is agreed to be used by the user based on the subscription of this specific service, or explicitly agreed to be applied by the user on the UE. The relationship with PersonalDataCollection should be considered, which is out of 3GPP scope. NOTE 4: These IEs are mandatory when regulatory requirements request that the user has to be informed if the personal data are collected by the application and how they are treated. |  |  |

##### 8.2.2.3.2	DC application and profile configuration response

Table 8.2.2.3.2-1 describes information elements for the DC Application and profile configuration response from the MMTel Enabler Server.

Table 8.2.2.3.2-1: Information elements for DC application and profile configuration response

| Information element | Status | Description |
| --- | --- | --- |
| DC application and profile configuration response list | M | A list of DC application and profile configuration response. This list contains a response for each DC Application. The detailed information is listed in Table 8.2.2.3.2-2. |

Table 8.2.2.3.2-2: Information elements in DC application and profile configuration response list

| Information element | Status | Description |
| --- | --- | --- |
| Application Identifier | M | Identifier of the Data Channel Application assigned by the Application Provider. |
| Successful response | O | Indicates the success of the configuration of this DC application and profile |
| > APPID | M | Identifier of the Data Channel Application assigned by the MMTel Enabler Server |
| Failure response | O | Indicates the failure of the configuration of this DC application and profile |
| > Reason | O | Indicates the reason for failure. |

### 8.2.3	DC application and profile update

#### 8.2.3.1	General

DC application and profile update enables a Controlling Application Server to update the DC application and profile with the MMTel Enabler Server.

#### 8.2.3.2	Procedure

Figure 8.2.3.2-1 illustrates the DC application and profile update procedure.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and configured with the necessary credentials to enable authenticating one another.

Figure 8.2.3.2-1: DC application and profile update

1.	The Controlling Application Server sends a DC application and profile update request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.3.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server initiates authentication procedures with the Requester Identity. If the registration is successful, the MMTel Enabler Server updates the DC Application or DC application profile for each DC Application based on information provided in the request.

3.	The MMTel Enabler Server updates the DC application and profile to DCAR.

4.	The MMTel Enabler Server sends a DC application and profile update response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.3.3.2.

Editor's note:	 The detailed interactions between the MMTel Enabler Server and DCAR are FFS.

#### 8.2.3.3	Information flows

##### 8.2.3.3.1	DC application and profile update request

Table 8.2.3.3.1-1 describes information elements for the DC application and profile update request from the Controlling Application Server to the MMTel Enabler Server.

Table 8.2.3.3.1-1: Information elements in DC application and profile update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Number of DC Application included | M | Indicates total number of DC Application included in this request |
| DC Application list | M | A list of DC application and configuration parameters included in this request. Each element in this list contains a Data Channel Application package and corresponding configuration parameters. The detailed information elements of DC application and profile are listed in Table 8.2.2.3.1-2. |

Table 8.2.3.3.1-2: Information elements in DC application list

| Information element | Status | Description |
| --- | --- | --- |
| APPID | M | Identifier of the Data Channel Application |
| Application Name | O | Name of the Data Channel Application |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application |
| Application Icon Url | O | Icon url of the Data Channel Application |
| Application Version | O | The newest version of the application. |
| Application Validity | O | The latest validity of the application |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: Precall only:the Data Channel Application is allowed to be used before the MMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. Incall:the Data Channel Application is allowed to be used after the MMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. Precall+Incall: the Data Channel Application is allowed to be used during the entire Precall and incall. |
| Autoload | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. |
| If Work Without Peer DC | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Supported Scenario | O | Indicates supported media type required for this Data Channel Application. The values of this IE include: Voice call only: this Data Channel Application can be used if and only if the corresponding call is a voice call. Video call only: this Data Channel Application can be used if and only if the corresponding call is a video call. Voice and video call: this Data Channel Application can be used in both voice call and video call. |
| Condition | O | Indicates whether this Data Channel Application can be used if under this condition. The values of this IE include but not limited: Service area: the application is allowed to be used in a certain area. Time period: the application is allowed to be used for a certain period of time. |
| 3gpp Qos Hint | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| PersonalDataCollection | O | Indicates whether this Data Channel Application will collect the user personal data (along with related consent opt-in/opt-out procedures). |
| PersonalDataCollectionInfoURL | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected, their processing their protection etc. |
| Application Package (see NOTE 2) | O | The Code package of the Data Channel Application. |
| NOTE 1: At least one of these optional IEs is present in the update request. A special case. When the application is updated, the request must carry the application package and application version. NOTE 2: The Application Package also support incremental updates. |  |  |

##### 8.2.3.3.2	DC application and profile update response

Table 8.2.3.3.2-1 describes information elements for the DC application and profile update response from the MMTel Enabler Server.

Table 8.2.3.3.2-1: Information elements for DC application and profile update response

| Information element | Status | Description |
| --- | --- | --- |
| DC application and profile update response list | M | A list of application and profile update response. This list contains a response for each DC application and profile. The detailed information is listed in Table 8.2.3.3.2-2. |

Table 8.2.3.3.2-2: Information elements in DC application and profile update response list

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates the DC application and profile update resquest was successful. |
| Failure response | O | Indicates that the DC application and profile update resquest failed |
| > Cause | O | Indicates the cause of DC application and profile update resquest failure. |

### 8.2.4	DC application deletion

#### 8.2.4.1	General

DC application deletion enables a Controlling Application Server to delete DC application with the MMTel Enabler Server.

#### 8.2.4.2	Procedure

Figure 8.2.4.2-1 illustrates the DC application deletion procedure.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and configured with the necessary credentials to enable authenticating one another.

Figure 8.2.4.2-1: DC application deletion

1.	The Controlling Application Server sends a DC Application delete request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.4.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server initiates authentication procedures with the Requester Identity. If the registration is successful, the MMTel Enabler Server requests the DCAR to delete the DC application and profile (DC applications and profile need to be deleted at the same time) and delete the mapping of APPID and DC application profile.

3.	The MMTel Enabler Server deletes the DC application and profile on DCAR.

4.	The MMTel Enabler Server sends a DC Application delete response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.4.3.2.

Editor's note:	 The detailed interactions between the MMTel Enabler Server and DCAR are FFS.

#### 8.2.4.3	Information flows

##### 8.2.4.3.1	DC Application delete request

Table 8.2.4.3.1-1 describes information elements for the DC Application delete request from the Controlling Application Server to the MMTel Enabler Server.

Table 8.2.4.3.1-1: Information elements in DC Application delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Number of DC Application included | M | Indicates total number of DC Application included in this request |
| List of APPID | M | List of identifiers of the DC application to be deleted. |

##### 8.2.4.3.2	DC Application delete response

Table 8.2.4.3.2-1 describes information elements for the DC Application delete response from the MMTel Enabler Server.

Table 8.2.4.3.2-1: Information elements for DC Application delete response

| Information element | Status | Description |
| --- | --- | --- |
| DC Application delete response list | M | A list of Application deletion response. The detailed information is listed in Table 8.2.4.3.2-2. |

Table 8.2.4.3.2-2: Information elements in DC Application delete response list

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates the deletion of the DC Application was successful |
| Failure response | O | Indicates the deletion of the DC Application was failed |
| > Cause | O | Indicates the cause of failure |

### 8.2.5	Getting DC application profile information

#### 8.2.5.1	General

Getting DC application profile information enables a Controlling Application Server to get the DC application profile information from the MMTel Enabler Server.

#### 8.2.5.2	Procedure

Figure 8.2.5.2-1 illustrates the procedure of getting DC application profile information.

Pre-conditions:

1.	The Controlling Application Server has connected to the serving network successfully.

2.	Both the Controlling Application Server and MMTel Enabler Server have established a secured connection and configured with the necessary credentials to enable authenticating one another.

Figure 8.2.5.2-1: Getting DC application profile information

1.	The Controlling Application Server sends a DC application and profile information retrieval request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.2.5.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server initiates authentication procedures with the Requester Identity. If the registration is successful, the MMTel Enabler Server gets the DC application Profile information based on the APPID included in the Getting DC application profile information request.

3.	The MMTel Enabler Server sends a Getting DC application profile information response to the Controlling Application Server. The response message includes information elements as specified in clause 8.2.5.3.2.

#### 8.2.5.3	Information flows

##### 8.2.5.3.1	Getting DC application profile information request

Table 8.2.5.3.1-1 describes information elements for the Getting DC application profile information request from the Controlling Application Server to the MMTel Enabler Server.

Table 8.2.5.3.1-1: Information elements in Getting DC application profile information request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the DC Application provider (VAL service provider) performing the request. |
| Security credentials | O | Security information required by the MMTel Enabler Server. |
| Number of DC Application included | M | Indicates total number of DC Application included in this request |
| APPID list | M | A list of APPID for the request. |

##### 8.2.5.3.2	Getting DC application profile information response

Table 8.2.5.3.2-1 describes information elements for the Getting DC application profile information response from the MMTel Enabler Server.

Table 8.2.5.3.2-1: Information elements in Getting DC application profile information response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the Getting DC application profile information request was successful. |
| > List of DC application profile information | O | List of DC application profile information as defined in Table 8.2.2.3.1-2. |
| Failure response | O | Indicates that the Getting DC application profile information request failed. |
| > Cause | O | Indicates the cause of Getting DC application profile information request failure. |

## 8.3	Control the downloading of data channel applications

### 8.3.1	General

The following clauses specify procedures and information flows for Controlling of the data channel applications downloading.

### 8.3.2	DC application profiles downloading on UE

#### 8.3.2.1	General

Application profiles downloading enables a MMTel Enabler Client to download the DC application profile from MMTel Enabler Server and use it to control the downloading behavior on DCMTSI Client in the same UE. This procedure is a detailed HTTP interaction procedure in the bootstrap data channel which established as per Annex AC.7.1

#### 8.3.2.2	Procedure

Figure 8.3.2.2-1 illustrates the DC application profiles downloading procedure.

Pre-conditions:

1.	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.3.2.2-1: MMTel Enabler Client in the UE requests the DC application profile list

1.	The MMTel Enabler Client in the UE sends a Get Root application request to MMTel Enabler Server to get the Root application. The request message includes information elements as specified in Table 8.3.2.3.1-1. The application type is used by MMTel Enabler Server to suggest DC applications for download.

NOTE 1:	In a implementation, the DCSF may support the DC application profile functionality of MMTel Enabler Server function, the DCSF may also interact with the MMTel Enabler Server to support this procedure, i.e. The MMTel Enabler Client sends a request to the DCSF, which then sends the request to the MMTel Enabler Server. The response in the opposite direction is also sent to the MMTel Enabler Client through the DCSF.

NOTE 2:	The interaction between DCSF and MMTel Enabler Server is implementation specific in this release.

2.	The MMTel Enabler Server checks whether the Root application version included in the Get Root application request is the newest version of the Root application. If the Root application version IE is not included, the Root application version is not considered as newest.

3.	The MMTel Enabler Server sends a Get Root application response to the MMTel Enabler Client in the UE. The response message includes information elements as specified in Table 8.3.2.3.2-1.

If Root application is locally stored in the MMTel Enabler Client, validity of the application will be updated to the value received in Root application validity IE.

NOTE 3:	When service provider wants the UE to delete the stored Root application, the response message includes negative value in the Root application validity IE.

NOTE 4:	The root application can also be implementation specific.

4.	The MMTel Enabler Client in the UE send a Get DC application profile List request to MMTel Enabler Server to get the DC application profile list available for this user. The request message includes information elements as specified in Table 8.3.2.3.3-1.

5.	The MMTel Enabler Server sends a Get DC application profile List response to the MMTel Enabler Client in the UE. The response message includes information elements as specified in Table 8.3.2.3.4-1. The response message includes list of Data Channel Application profile.

The MMTel Enabler Client locally stores the downloaded DC application profiles and displays the related application information from the DC application profile on the Root application. Users can select and download corresponding DC applications based on the profile information on the Root application. The MMTel Enabler Client may stores the downloaded DC Application which may be obsoleted in the condition of, e.g. the validity is expired, update available for application, etc.

#### 8.3.2.3	Information flows

##### 8.3.2.3.1	Get Root application request

Table 8.3.2.3.1-1 describes information elements for the Get Root application request from the MMTel Enabler Client to the MMTel Enabler Server.

Table 8.3.2.3.1-1: Information elements in Get Root application request

| Information element | Status | Description |
| --- | --- | --- |
| UE Information (see NOTE) | M | The UE related information required to identify the Root application (e.g. device type, device vendor, etc) |
| Root application version | O | The newest version of the Root application stored in the MMTel Enabler Client in the UE. This IE presents if Root application is locally stored in the MMTel Enabler Client. |
| NOTE: The MMTel service provider can provide the MMTel Enabler Client with different Root application based on this IE. |  |  |

##### 8.3.2.3.2	Get Root application response

Table 8.3.2.3.2-1 describes information elements for the Get Root application response from the MMTel Enabler Server to the MMTel Enabler Client.

Table 8.3.2.3.2-1: Information elements for Get Root application response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of Get Root application request. |
| Update needed | M | Indicates whether the Root application is needed to be updated by the MMTel Enabler Client in the UE |
| Root application version | O | The newest version of the Root application. This IE presents only if the Root application is needed to be downloaded or updated by the MMTel Enabler Client in the UE. |
| Root application validity | O | The validity of the Root application. This IE presents only if the Root application is needed to be downloaded or updated by the MMTel Enabler Client in the UE. |
| Root application (see NOTE) | O | The Root application provided by the MMTel service provider, e.g. layout, and/or home page etc of the Data Channel Application list. This IE presents if the update of the Root application in the UE is needed, i.e. the Root application version included in the Get Root application request is not equal to the newest version on the MMTel Enabler Server. |
| NOTE: The detailed information of this IE is implementation specific and out of scope of the present document. |  |  |

##### 8.3.2.3.3	Get DC application profile List request

Table 8.3.2.3.3-1 describes information elements for the Get DC application profile List request from the MMTel Enabler Client to the MMTel Enabler Server.

Table 8.3.2.1-3: Information elements in Get DC application profile List request

| Information element | Status | Description |
| --- | --- | --- |
| MMTel Enabler Client version (see NOTE) | M | The version of MMTel Enabler Client |
| application type | O | type of application (e.g. emergency, conference, XR, etc.) |
| Begin index | O | The starting index of the first Data Channel Application required in the Data Channel Application list. This IE presents if the requested Data Channel Application is not the first one in the Data Channel Application list, e.g. when the Data Channel Application list is needed to be shown in multiple pages, this IE is needed to be included when request the pages other than the first page. The default value of this IE is 0 if this IE is absent. |
| Number of application profile required | O | The total number of DC application profiles required in this request |
| NOTE: The MMTel service provider can provide the MMTel Enabler Client with different DC application profile list based on this IE. |  |  |

##### 8.3.2.3.4	Get DC application profile List response

Table 8.3.2.3.4-1 describes information elements for the Get DC application profile List response from the MMTel Enabler Server to the MMTel Enabler Client.

Table 8.3.2.3.4-1: Information elements in Get DC application profile List response

| Information element | Status | Description |
| --- | --- | --- |
| DC Application profile list | M | A list of DC application profiles available for this user. Each element in this list contains a DC application profile of this Data Channel Application. The detailed information elements of DC application profile are listed in Table 8.3.2.1-5. |

Table 8.3.2.3.4-2: Information elements in DC application profile

| Information element | Status | Description |
| --- | --- | --- |
| APPID | M | Identifier of the Data Channel Application |
| Application Name | O | Name of the Data Channel Application |
| Service Type | O | Service type of the Data Channel Application, used to help the user understand the type of services provided by the application |
| Application Icon | O | Icon of the Data Channel Application |
| Application Version | O | The newest version of the application. |
| Application Validity | O | The latest validity of the application. |
| Application Loading Phase | O | Indicates when this Data Channel Application is allowed to be used. The values of this IE include: Precall only:the Data Channel Application is allowed to be used before the MMTel call session is established, i.e. after the 18x response is sent/received and before the 200 OK of the initial SIP INVITE request is sent/received. Incall:the Data Channel Application is allowed to be used after the MMTel call session is established, i.e. after the 200 OK of the initial SIP INVITE request is sent/received. |
| Autoload (see NOTE 1) | O | Indicates whether this Data Channel Application is needed to be load to the UE automatically. |
| Autolaunch (see NOTE 1) | O | Indicates whether this Data Channel Application is needed to be downloaded to the UE and run automatically. |
| If Work Without Peer DC | O | Indicates whether this Data Channel Application can be used if Data Channel is not supported by the other party of the call. |
| Supported Scenario | O | Indicates supported media type required for this Data Channel Application. The values of this IE include: Voice call only: this Data Channel Application can be used if and only if the corresponding call is a voice call. Video call only: this Data Channel Application can be used if and only if the corresponding call is a video call. Voice and video call: this Data Channel Application can be used in both voice call and video call. |
| Condition | O | Indicates whether this Data Channel Application can be used if under this condition. The values of this IE include but not limited: Service area: the application is allowed to be used in a certain area. Time period: the application is allowed to be used for a certain period of time. |
| 3gpp Qos Hint | O | Indicates the QoS requirement of this Data Channel Application which is used to set the value of “a=3gpp-qos-hint” attribute in SDP for the data channel(s) used by the application. |
| PersonalDataCollection (see NOTE 2) | O | Indicates whether this Data Channel Application will collect the user personal data. |
| PersonalDataCollectionInfoURL (see NOTE 2) | O | URL to retrieve the description of the purpose of mandatory and optional personal data to be collected (along with related consent opt-in/opt-out procedures), their processing their protection etc. |
| NOTE 1: These IEs can be applied only if the specific service using this data channel application is agreed to be used by the user based on the subscription of this specific service, or explicitly agreed to be applied by the user on the UE. NOTE 2: These IEs are mandatory when regulatory requirements request that the user has to be informed if the personal data are collected by the application and how they are treated. |  |  |

### 8.3.3	DC application profiles updating on UE

#### 8.3.3.1	General

There are two modes in this procedure:

the MMTel Enabler Server notifies the MMTel Enabler Client in the UE to send application request messages to the MMTel Enabler Server to request the profiles of data channel applications;

the MMTel Enabler Server sends the profiles to the UE directly after the UE subscribes to the profile notification.

The two detailed procedures of DC application profiles updating on UE and the related information flow are specified in clause 8.3.3.2 and 8.3.3.3.

NOTE :	The updating procedures go over the top of DCSF. The detailed procedures to update data channel applications to the UE are out of scope of 3GPP TS 23.228 [3].

#### 8.3.3.2	Procedure

##### 8.3.3.2.1	The MMTel Enabler Client request DC application profiles based on notification

The MMTel Enabler Server can notify the MMTel Enabler Client in the UE to send application request messages to the MMTel Enabler Server to request the DC application profiles, the detailed procedure is illustrated in Figure 8.3.3.2.1-1.

NOTE:	The MMTel Enabler Server can instruct the MMTel Enabler Client to request root application and DC application profiles as required. e.g. root application version update, offline of a data channel application, etc.

Pre-condition:

1.	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.3.3.2.1-1: MMTel Enabler Client request DC application profiles based on notification

1.	The notification channel has been established between notification management client and notification management Server as specified clause 17.3.3 of TS 23.434 [6].

2.	The MMTel Enabler Client subscribes to the MMTel Enabler Server for receiving notifications.

3.	The MMTel Enabler Server sends a Root application and profile update notification to notify the MMTel Enabler Client in the UE to get the Root application or DC application profile. The Root application and profile update notification includes information elements as specified in Table 8.3.3.3.1-1.

The step 4 to step 8 are same as steps 1 through 5 in clause 8.3.2.2.

##### 8.3.3.2.2	The MMTel Enabler Server notifies the DC application profile to the MMTel Enabler Client

The MMTel Enabler Server notifies the DC application profiles to the MMTel Enabler Client is illustrated in Figure 8.3.3.2.2-1.

Pre-condition:

1.	The bootstrap data channels have been established as specified in 3GPP TS 23.228 [3].

Figure 8.3.3.2.2-1: MMTel Enabler Server notifies the DC application profile list to UE

1.	The notification channel has been established between notification management client and notification management server as specified clause 17.3.3 of TS 23.434 [6].

2.	The MMTel Enabler Client subscribes to the MMTel Enabler Server for receiving notifications.

3.	The MMTel Enabler Server checks the provision of Root application and DC application profile list is needed, e.g. the UE has IMS DC capability, an application version or application profile update is available, etc.

NOTE:	The detail additional checklists in this step are out of scope of the present document.

4.	The MMTel Enabler Server sends a Root application notification message to the MMTel Enabler Client in the UE. The notification message includes information elements as specified in Table 8.3.3.3.2-1.

The MMTel Enabler Server sends a data channel DC application profile List notification to the MMTel Enabler Client in the UE. The notification message includes information elements are as same as the elements in Table 8.3.2.3.4-1.

#### 8.3.3.3	Information flows

##### 8.3.3.3.1	Root application and profile update notification

Table 8.3.3.3.1-1 describes information elements for the Root application and profile update notification from the MMTel Enabler Server to the MMTel Enabler Client.

Table 8.3.2.3.1-1: Information elements in Root application and profile update notification

| Information element | Status | Description |
| --- | --- | --- |
| Urgent | O | Indicates whether the operation is urgent to UE, i.e. whether the Root application and profile need to be updated immediately. E.g. the update of Root application and profile is urgent if the available DC Application for the user is changed |

##### 8.3.3.3.2	Root application notification

Table 8.3.3.3.2-1 describes information elements for the Root application notification from the MMTel Enabler Server to the MMTel Enabler Client.

Table 8.3.3.3.2-1: Information elements for Root application notification

| Information element | Status | Description |
| --- | --- | --- |
| Update needed | M | Indicates whether the Root application is needed to be updated by the MMTel Enabler Client in the UE |
| Root application version | O | The newest version of the Root application. This IE presents only if the Root application is needed to be updated by the MMTel Enabler Client in the UE. |
| Root application (see NOTE) | O | The Root application provided by the MMTel service provider, e.g. layout, and/or home page etc. of the Data Channel Application list. This IE presents if the update of the Root application in the UE is needed, i.e. the Root application version included in the Get Root application request is not equal to the newest version on the MMTel Enabler Server. |
| NOTE: The detailed information of this IE is implementation specific and out of scope of the present document. |  |  |

## 8.4	Application calling enablement

### 8.4.1	General

The following clauses specify procedures and information flows for Application calling enablement.

### 8.4.2	Application calling service API with Data Channel capability

#### 8.4.2.1	General

The MMTel Enabler Server exposes the Application calling service API with Data Channel capability to the Applications in order to support establishing a call between Application and DCMTSI client.

#### 8.4.2.2	Procedure

##### 8.4.2.2.1	Establishing an Application Call with Data Channel capability between non-IMS application and DCMTSI client

Figure 8.4.2.2.1-1 illustrates the operation to establish an Application Call with Data Channel capability between non-IMS application and DCMTSI client.

Pre-conditions:

1.	The Application is authorized to use Application calling service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [3].

3.	The MMTel Enabler Server is authorized to use OMA ThirdPartyCall API as specified in OMA-TS-REST_NetAPI_ThirdPartyCall [7].

Figure 8.4.2.2.1-1: Establishing a Application Call with Data Channel capability between non-IMS applications and DCMTSI client

1.	The Application sends an Application Call with DC capability establishment request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.4.1.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server can utilize the OMA Third Party Call API [7] to establish a Call session with audio and/or video media as specified in OMA-TS-REST_NetAPI_ThirdPartyCall and obtain the result of the Third-Party Call establishment. The MMTel Enabler Server can also obtains the IMS Session ID associated with the Third Party Call as specified in RFC 7989 [7989], and handles the mapping between the OMA call session ID and the IMS session ID.

NOTE 1:	The OMA call session ID attribute in the OMA Third Party Call API is generated by the entity implementing the OMA APIs, which is different from the IMS session Id.

3,	The MMTel Enabler Server utilizes the DC update capability of the IMS core network to add DC media to the existing Application Call Session as specified in Annex AC of 3GPP TS 23.228 [3]. The IMS core network returns the update result.

3.	The MMTel Enabler Server sends an Application Call establishment response to the Application. The response message includes information elements as specified in clause 8.4.1.3.2.

NOTE 2:	In this release, the MMTel Enabler Server is deployed in the PLMN operator domain.

Editor's note:	The flow will need to be updated once SA2 work is done to specify NEF exposure of IMS session management (Nnef_ImsSessionManagement).

#### 8.4.2.3	Information flows

##### 8.4.2.3.1	Application Call establishment request

The information elements of Application Call establishment request are as same as the elements in Table 8.4.3.3.1-1.

##### 8.4.2.3.2	Application Call establishment response

The information elements of Application Call establishment response are as same as the elements in Table 8.4.3.3.2-1.

### 8.4.3	Third-Party Call service API with Data Channel capability

#### 8.4.3.1	General

The MMTel Enabler Server exposes the Third-Party Call service API with Data Channel capability to the Vertical service provider in order to support establishing a call between two UEs.

#### 8.4.3.2	Procedure

##### 8.4.3.2.1	Establishing a Third-Party Call with Data Channel capability

Figure 8.4.3.2.1-1 illustrates the operation to establish a Third-Party Call with Data Channel capability.

Pre-conditions:

1.	The Vertical service provider is authorized to use Third-Party Call service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use the IMS DC capability APIs as defined in 3GPP TS 23.228 [3].

3.	The MMTel Enabler Server is authorized to use OMA THIRD PARTY CALL API as specified in OMA-TS-REST_NetAPI_ThirdPartyCall [7].

Figure 8.4.3.2.1-1: Establishing a Third-Party Call with Data Channel capability

1.	The Vertical service provider sends a Third-Party Call with DC capability establishment request to the MMTel Enabler Server. The request message includes information elements as specified in clause 8.4.3.3.1.

2.	Upon receiving the request, the MMTel Enabler Server validates if the requester is authorized for the request, If the requester is authorized then the MMTel Enabler Server utilizes the OMA Third Party Call API [7] to establish a Third-Party Call session with audio and/or video media as specified in OMA-TS-REST_NetAPI_ThirdPartyCall and obtain the result of the Third Party Call establishment. The MMTel Enabler Server obtains IMS Session ID associated with the Third Party Call as specified in RFC 7989 [10],the OMA API call Session ID and handles the mapping between OMA call session ID and the IMS session ID

NOTE :	The OMA call session ID attribute in the OMA Third Party Call API is generated by the entity implementing the OMA APIs, which is different from the IMS session Id.

3.	the MMTel Enabler Server utilizes the DC update capability of the IMS core network to add DC media to the existing Third-Party Call Session as specified in 3GPP TS 23.228 [3], and the IMS core network returns the update result.

4.	The MMTel Enabler Server sends a Third-Party Call with DC capability establishment response to the Vertical service provider. The response message includes information elements as specified in clause 8.4.3.3.2.

NOTE 2:	In this release, the MMTel Enabler Server is deployed in the PLMN operator domain.

Editor's note:	The flow will need to be updated once SA2 work is done to specify NEF exposure of IMS session management (Nnef_ImsSessionManagement).

#### 8.4.3.3	Information flows

##### 8.4.3.3.1	Third-Party Call with DC capability establishment request

Table 8.4.3.3.1-1 describes information elements for the Third-Party Call with DC capability establishment request from the Application Server to the MMTel Enabler Server.

Table 8.3.2.3.1-1: Third-Party Call with DC capability establishment request

| Information element | Status | Description | Related OMA API Element |
| --- | --- | --- | --- |
| Originating ID (see NOTE) | M | The identifier of the caller. | The first element in the participant list, which considered to denote the “A-Party” specified in OMA Third Party Call API [7] |
| Terminating ID (see NOTE) | M | The identifier of the callee. | The second element in the participant list, which considered to denote the “B-Party” specified in OMA Third Party Call API [7] |
| Media information | M | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of mmtel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. | The mediaInfo of the CallSessionInformation specified in OMA Third Party Call API [7] |
| DC media information | O | Identifier of whether DC media is expected to be used in this call | N/A |
| Application Profile requested | O | The DC application profile expected to be used in this call | N/A |
| Notification information | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. | The callbackReference of the CallSessionInformation specified in OMA Third Party Call API [7] |
| NOTE: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain,. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. |  |  |  |

##### 8.4.3.3.2	Third-Party Call with DC capability establishment response

Table 8.4.3.3.2-1 describes information elements for the Third-Party Call with DC capability establishment response from the Application Server to the MMTel Enabler Server.

Table 8.3.2.3.2-1: Third-Party Call with DC capability establishment response

| Information element | Status | Description | Related OMA API Element |
| --- | --- | --- | --- |
| Call establishment result | M | Indication if the Call establishment is success or failure. | The HTTP response codes of the created call session as specified in OMA REST_NetAPI_Common [11] |
| Session ID (see NOTE 1) | O | The identifier of the call session. | The callSessionId as specified in OMA Third Party Call API [7] |
| Failure Cause (see NOTE 2) | O | The reason for failure | The Policy Exception and Service Exception fault as specified in OMA Third Party Call API [7] |
| Originating ID (see NOTE 3) | M | The identifier of the caller. | The first element in the participant list, which considered to denote the “A-Party” specified in OMA Third Party Call API [7] |
| Terminating ID (see NOTE 3) | M | The identifier of the callee. | The second element in the participant list, which considered to denote the “B-Party” specified in OMA Third Party Call API [7] |
| Media information | M | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of mmtel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. | The mediaInfo of the CallSessionInformation specified in OMA Third Party Call API [7] |
| DC media information | O | Identifier of whether DC media is expected to be used in this call | N/A |
| Application Profile requested | O | The DC application profile expected to be used in this call | N/A |
| NOTE 1: This IE shall only be present when the Call establishment result is Success. NOTE 2: This IE shall only be present when the Call establishment result is Failure. NOTE 3: The Originating ID and Terminating ID may or may not be MSISDN based identifier, i.e. a private identifier in the Application domain,. If Originating ID or Terminating ID is not MSISDN based identifier, the MMTel Enabler Server will translate it into a routable ID in the IMS domain and record the mapping relationship between the Originating ID/Terminating ID and the routable ID. |  |  |  |

### 8.4.4	Establishing an Application Call with Data Channel capability (A2P)

#### 8.4.4.1	Procedure

Figure 8.4.4-1 illustrates the operation to establish an Application Call with Data Channel capability between the Application and the MMTel Enabler Server in order to add an A2P Data Channel to an existing IMS session of the user on UE1 in alignment with the A2P flow specified in 3GPP TS 23.228 [3].

Pre-conditions:

1.	The Application can use DCH (supports the media set information for DCH) and is authorized to use the service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [3].

3.	The MMTel Enabler Server is authorized to use the NEF exposed IMS session management capabilities as specified in 3GPP TS 23.502 [12].

4.	The MMTel Enabler Server is subscribed to the AEF (NEF) for IMS session events information e.g., media changes such as requested for the specific DCH Application, for a list of users.

NOTE:	How the IMS session ID is provided by the NEF exposure of IMS events in 3GPP TS 23.502 [12] is out of scope of this specification.

5.	The DCH Application Server is subscribed to the MMTel Enabler Server for events related to the DCH application.

6.	An IMS MMTel session is ongoing for User (UE1) with another user who requests the DCH Application to provide some DCH media to the user on UE1.

Figure 8.4.4-1: Establishing a Application Call with Data Channel capability

1.	The user sends a request, via the 5GC and IMS, to add DCH media for a specific DCH Application. The NEF notifies the MMTel Enabler Server of the new IMS session event for the for the media changes related to the DCH Application. The request includes parameters such as the session ID, MDC2 endpoint, A2P type.

2.	The MMTel Enabler Server requests an IMS session update to the NEF, providing the Session ID and MDC2 endpoint address. The NEF propagates the request to the IMS core which sets the media operation. The NEF sends the response to the MMTel Enabler to the IMS session Update request.

3.	The MMTel Enabler Server sends the response to the DCH Application Server including the Session ID for the Add media request.

4.	Once the IMS system completed the media preparation for the IMS session, the NEF sends an IMS session management notification request to the MMTel Enabler Server with MDC2 resource information for the Session ID. The DCH Application Server responds (acknowledges) the MMTel Enabler Server notification.

5.	The MMTel Enabler Server notifies the DCH Application Server of the MDC2 resource information for the Session ID.

6.	The MMTel Enabler Server acknowledges the NEF notification.

Post-condition: The UE1 upon getting the updated IMS session information for the DCH media proceeds to download the DCH Application.

#### 8.4.4.2	Information flows

##### 8.4.4.2.1	Application Add Data Chanel media to UE1 session (A2P) request

Table 8.4.4.2.1-1: Application Add Data Chanel media to UE1 session (A2P) request

| Information element | Status | Description |
| --- | --- | --- |
| Data Channel APPID | M | The identifier of the DCH application |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Media resource information | O | The media information to be passed via the Data Chanel. It includes information described in 3GPP TS 23.502 [12] clause 5.2.6.40. |
| Application Profile requested | O | The DC application profile expected to be used in this call. |
| Notification address | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. |

##### 8.4.4.2.2	Application Add Data Chanel media to UE1 session (A2P) response

Table 8.4.4.2.2-1: Application Add Data Chanel media (A2P) response

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Successful response | O | Indicates the Notificaiton of Application of DCH media info (A2P) request was successful |
| Failure response | O | Indicates the Notificaiton of Application of DCH media info (A2P) request has failed |
| > Cause | O | Indicates the cause of request failure |

##### 8.4.4.2.3	Notify Application of DCH media info (A2P) request

Table 8.4.4.2.3-1: Notify Application of DCH media info (A2P) request

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Notification address | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. |
| Media resource information | O | The media information to be passed via the Data Chanel. It includes information described in 3GPP TS 23.502 [12] clause 5.2.6.40. |
| Application Profile requested | O | The DC application profile expected to be used in this call. |

##### 8.4.4.2.4	Notify Application of DCH media info (A2P) response

Table 8.4.4.2.4-1: Notify Application of DCH media info (A2P) response

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Successful response | O | Indicates the Notificaiton of Application of DCH media info (A2P) request was successful |
| Failure response | O | Indicates the Notificaiton of Application of DCH media info (A2P) request has failed |
| > Cause | O | Indicates the cause of request failure |

### 8.4.5	Application calling service API with Data Channel capability

#### 8.4.5.1	Procedure

##### 8.4.5.1.1	Establishing an Application Call with Data Channel capability (P2A)

Figure 8.4.5.1.1-1 illustrates the operation to establish an Application Call with Data Channel capability between a user (on UE1) and a Data Chanel (DCH) Application via the MMTel Enabler Server.

Pre-conditions:

1.	The Application can use DCH (supports the media set information for DCH) and is authorized to use the service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [3].

3.	The MMTel Enabler Server is authorized to use the NEF exposed IMS session management capabilities as specified in [8].

4. The MMTel Enabler Server is subscribed to the AEF (NEF) for IMS session events information for the specific DCH Application (using the DCH Application Called party ID).

NOTE:	How the IMS session ID is provided by the NEF exposure of IMS events in 3GPP TS 23.502 [12] is out of scope of this specification.

5.	The DCH Application Server is subscribed to the MMTel Enabler Server for events related to the DCH application.

6.	An IMS MMTel session is ongoing for User (UE).

Figure 8.4.5.1.1-1: Establishing a Application Call with Data Channel capability (P2A)

1.	The user (on UE1) sends a request, via the 5GC and IMS, to add DCH media for a specific DCH Application.

2.	The P2A session establishment is handled via 5GC and IMS system.

3.	NEF notifies the MMTel Enabler Server of the new IMS session event for the DCH Application.

4.	The MMTel Enabler Server notifies the DCH Application Server of the new request for DCH media.

5.	The DCH Application Server accepts the request and provides the MDC2 media resource requested.

6.	The MMTel Enabler Server utilizes the IMS session update capability exposed by the NEF [8] to request addition of the DCH media to the existing IMS MMTel Call Session.

7.	The NEF further interacts with the IMS system to request the DCH media addition to the ongoing MMTel session and IMS further handles this as specified in Annex AC of 3GPP TS 23.228 [3].

Post-condition: The Application data channel is activated.

#### 8.4.5.2	Information flows

##### 8.4.5.2.1	Notify DCH Application of MDC2 media (P2A) request

Table 8.4.5.2.1-1: Notify DCH Application of MDC2 media (P2A) request

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Media resource information | O | The MDC2 media information requested by the UE1 to be sent over the Data Chanel. It includes information described in 3GPP TS 23.502 [12] clause 5.2.6.40. |

##### 8.4.5.2.2	Notify DCH Application of MDC2 media (P2A) response

Table 8.4.5.2.2-1: Notify DCH Application of MDC2 media (P2A) response

| Information element | Status | Description |
| --- | --- | --- |
| Session ID | M | The session identifier of the session on which MMTel Enabler Server must add the new media. |
| Successful response | O | Indicates the Notificaiton of Application of DCH media info (A2P) request was successful |
| Failure response | O | Indicates the Notificaiton of Application of DCH media info (A2P) request has failed |
| > Cause | O | Indicates the cause of request failure |

## 8.5	Multiple call control handling coordination among different Application Servers

### 8.5.1	General

When receiving call control requests from multiple Application Servers, the MMTel Enabler Server provides call control handling consolidation and coordination.

### 8.5.2	Procedure

Figure 8.5.2-1 illustrates the procedure of multiple call control handling coordination among different Application Servers.

Pre-conditions:

1.	The Application Servers and MMTel Enabler Server have been configured with the necessary credentials to enable authenticating one another.

2.	The call control handling policy (e.g. the priority of each Application Server) is pre-configured in MMTel Enabler Server.

3.	Application Servers have completed call event subscription to MMTel Enabler Server and the MMTel Enabler Server subscribes the call event from DCSF/IMS core.

4.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in 3GPP TS 23.228 [3].

Figure 8.5.2-1: Multiple call control handling coordination among different Application Servers

1.	DCSF/IMS Core node sends call event notification, i,e, Nimsas_SessionEventControl_Notify as specifed in 3GPP TS 23.228 [3], to MMTel Enabler Server (e.g. SessionEstablishmentRequestEvent, MediaChangeRequestEvent., etc.).

2.	The MMTel Enabler Server checks which Application Server has subscribed this call event notification.

3.	The MMTel Enabler Server sends Call event notification to Application Server(s) which subscribed this call event notification. The Call event notification request includes information elements as specified in Annex AA.2.4.2 of 3GPP TS 23.228 [3].

4-5.Application Servers receive call event notification and sends Call control request (e.g. create new call session, terminate call session or call control, etc.) based on its own service logic. The Call control request includes information elements as specified in Annex AA.2.4.3 or Annex AA.2.4.4 of 3GPP TS 23.228 [3].

6.	The MMTel Enabler Server receives call control requests from different Application Servers.It checks whether these call control requests are contradictory and makes call control handling decision based on service policy and service priority (e.g. if no conflict happens, to decide call control request execution sequence; if conflict happens, to deny call control request from certain AS with lower priority, etc.).

7-8.The MMTel Enabler Server sends call control responses to each Application Server based on the call control handling decision including that whether the call control request is accepted and the failure reason if the call control request is not accepted. The call control response includes information elements as specified in Annex AA.2.4.3 or Annex AA.2.4.4 of 3GPP TS 23.228 [3].

9.	The MMTel Enabler Server sends call control requests decided in step 6 from Application Servers (or permitted Application Servers in conflict scenario) to DCSF/IMS core network by using Nimsas_MediaControl Service or Nimsas_ImsSessionManagement Service as specifed in 3GPP TS 23.228 [3].The DCSF/IMS core network sends a corresponding response (e.g, whether the call is established successfully or not) as specifed in 3GPP TS 23.228 [3] to the MMTel Enabler Server.

10. The MMTel Enabler Server sends the call control result notification request to the permitted Application Servers in conflict scenario decided in step 6. The call control result notification includes information elements as specified in clause 8.5.3.1. The Application Server sends the corresponding control result notification response to the MMTel Enabler Server.

### 8.5.3	Information flows

#### 8.5.3.1	Call control result notification

Table 8.5.3.1-1 describes information elements for call control result notification request from the MMTel Enabler Server to the Application server.

Table 8.5.3.1-1: Call control result notification request

| Information element | Status | Description |
| --- | --- | --- |
| Final result of the call control request | M | Indicates the final result of the call control request provided by the IMS |
| Media resource information set | O | If media resource information set is provided by IMS, include the media resource information set in this notification |

# 9	Deployment Guideline

## 9.1	General

This clause describes deployments of the functional model specified in clause 6.

## 9.2	MMTel Enabler server deployed in PLMN operator domain

The MMTel Enabler server is recommended to be deployed in the PLMN operator domain. Figure 9.2-1 illustrates the MMTel Enabler server deployed in PLMN operator domain.

Figure 9.2-1: MMTel Enabler server deployed in PLMN operator domain

The Application server can be deployed in PLMN operator domain or Application service provider domain and interacts with MMTel Enabler server via MMTel-2 reference point and MMTel-2e reference point respectively.The Application server deployed in PLMN operator domain, e.g. Application server 2 can only interact with the MMTel Enabler server deployed in the same PLMN operator domain, i.e. MMTel Enabler server 2. The Application server deployed in Application service provider domain, e.g. Application server 1, can interact with MMTel Enabler servers deployed in different PLMN operator domains, i.e. MMTel Enabler server 1 and MMTel Enabler server 2, based on business agreement between PLMN operator and Application service provider. In this case, the MMTel-2 reference point becomes MMTel-2e reference point.

The Controlling Application server can only be deployed in PLMN operator domain and interacts with MMTel Enabler server via MMTel-3 reference point. The Controlling Application server can only interact with the MMTel Enabler server deployed in the same PLMN operator domain.

NOTE:	In this release, the MMTel Enabler Server is deployed in the PLMN operator domain.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-07 | SA6#62-Ad Hoc-e |  | - | - | - | Proposed skeleton approved at S6#62-Ad Hoc-e | 0.0.0 |
| 2023-07 | SA6#62-Ad Hoc-e |  |  |  |  | Implemented pCRs approved in S6#62-Ad Hoc-e: S6a240100, S6a240101, S6a240102, S6a240103, S6a240282 Editorial changes by the rapporteur | 0.1.0 |
| 2023-10 | SA6#63 |  |  |  |  | Implemented pCRs approved in S6#63: S6-244639, S6-244640, S6-244641, S6-244642, S6-244643, S6-244644 Editorial changes by the rapporteur | 0.2.0 |
| 2023-11 | SA6#64 |  |  |  |  | Implemented pCRs approved in S6#64: S6-245625, S6-245626, S6-245627 Editorial changes by the rapporteur | 0.3.0 |
| 2024-12 | SA#106 | SP-241697 |  |  |  | Submitted to SA#106 for information | 1.0.0 |
| 2025-02 | SA6#65 |  |  |  |  | Implemented pCRs approved in S6#65: S6-250072, S6-250074, S6-250077, S6-250078, S6-250079, S6-250150, S6-250372, S6-250373, S6-250374, S6-250375, S6-250376, S6-250379, S6-250380, S6-250381, S6-250382, S6-250383, S6-250384, S6-250549, S6-250550 Editorial changes by the rapporteur | 1.1.0 |