| 3GPP TR 23.700-23 V1.2.0 (2024-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Application enabler for XR Services; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPPOrganizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPPonly. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope		8

2	References		8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms		9

3.2	Symbols		9

3.3	Abbreviations		9

4	Key issues		9

4.1	Key issue #1: KPI measurement and exposure	9

4.2	Key issue #2: E2E Multi-Modal Communication Flows	10

4.3	Key Issue #3: Coordination between direct UE connection and network based connection for AR/VR services		10

4.4	Key issue #4: Application enablement layer capabilities usage to supporting the XR services	11

4.5	Key issue #5: Support the tethered UE	11

4.6	Key issue #6: XR application server selection enhancement	12

4.7	Key issue #7: E2E KPI optimization for XR service	12

4.8	Key issue #8: PDU set handling for XR traffic	13

5	Application enablement architecture requirements	14

5.1	General requirements	14

5.2	Support for direct UE connection	14

5.2.1	Description	14

5.2.2	Requirements	14

5.3	Multi-modal flows alignment and monitoring requirements	14

5.3.1	Description	14

5.3.2	Requirements	14

5.4	Tethered device discovery and monitoring requirements	14

5.4.1	Description	14

5.4.2	Requirements	15

6	Application enablement architecture for XR services	15

6.1	Application enablement architecture	15

6.1.1	General		15

6.1.2	Option1: Application enablement architecture for XRAPP function	15

6.1.3	Option2: Application enablement architecture based on existing application enablement capabilities	16

6.1.4	Option3: CAPIF architecture to support the service discovery and invocation for XR application	19

7	Solutions		20

7.0	Mapping of solutions to key issues	20

7.1	Solution #1: SEALDD Policy Configuration for multi-modal flows	20

7.1.1	Architecture Impacts	20

7.1.2	Solution description	20

7.1.3	Information flows	21

7.1.4	Solution evaluation	21

7.2	Solution #2: Policy based Multi-modal SEALDD flow establishment	21

7.2.1	Architecture Impacts	21

7.2.2	Solution description	21

7.2.3	Solution evaluation	23

7.3	Solution #3: Support of Multi-Modal traffic indication in SEALDD layer	23

7.3.1	Architecture Impacts	23

7.3.2	Solution description	23

7.3.3	Solution evaluation	27

7.4	Solution #4: Support of QoS measurement for Multi-Modal traffic in SEALDD layer	28

7.4.1	Architecture Impacts	28

7.4.2	Solution description	28

7.4.3	Solution evaluation	37

7.5	Solution #5: Multi-flow synchronization for multi-modal XR application	37

7.5.1	Architecture Impacts	37

7.5.2	Solution description	37

7.5.3	Solution evaluation	39

7.6	Solution #6: Multi-modal flows alignment and monitoring	39

7.6.1	Architecture Impacts	39

7.6.2	Solution description	39

7.6.3	Solution evaluation	42

7.7	Solution #7: Support multi-modal service in SEALDD	43

7.7.1	Architecture Impacts	43

7.7.2	Solution description	43

7.7.3	Solution evaluation	49

7.8	Solution #8: EAS instantiation enhancement to satisfy E2E KPI requirements for XR application	50

7.8.1	Architecture Impacts	50

7.8.2	Solution description	50

7.8.3	Solution evaluation	52

7.9	Solution #9: Support performance analytics for tethered UEs	52

7.9.1	Architecture Impacts	52

7.9.2	Solution description	52

7.9.3	Solution evaluation	54

7.10	Solution #10: EDN selection based on predicted network connectivity	54

7.10.1	Architecture Impacts	54

7.10.2	Solution description	54

7.10.3	Solution evaluation	55

7.11	Solution #11: Support the tethered UE based on PINAPP	55

7.11.1	Architecture Impacts	55

7.11.2	Solution description	58

7.11.3	Solution evaluation	61

7.12	Solution #12: XR application server deployment enhancement	61

7.12.1	Architecture Impacts	61

7.12.2	Solution description	61

7.12.3	Solution evaluation	63

7.13	Solution #13: XR EAS selection based on N6 delay measurement	64

7.13.1	Architecture Impacts	64

7.13.2	Solution description	64

7.13.3	Solution evaluation	65

7.14	Solution #14: Support UE to UE communication in different modes via SEALDD	66

7.14.1	Architecture Impacts	66

7.14.2	Solution description	66

7.14.3	Information flows	68

7.14.4	Solution evaluation	68

7.15	Solution #15: Application enablement layer capabilities usage to support the user group level QoS guarantee		69

7.15.1	Architecture Impacts	69

7.15.2	Solution description	69

7.15.3	Solution evaluation	70

7.16	Solution #16: XR Application enablement client capability information procedure	70

7.16.1	Architecture Impacts	70

7.16.2	Solution description	71

7.x	Solution #x: <title>	74

7.x.1	Architecture Impacts	74

7.x.2	Solution description	74

7.x.3	Solution evaluation	74

8	Business Relationships	74

8.1	General		74

9	Overall evaluation	74

9.1	General		74

9.2	Architecture evaluation	75

9.3	Solution evaluation	76

9.3.1	Key Issue #1 solutions evaluation	76

9.3.2	Key issue #2 solutions evaluation	76

9.3.3	Key issue #3 solutions evaluation	77

9.3.4	Key Issue #4 solutions evaluation	77

9.3.5	Key Issue #5 solutions evaluation	78

9.3.6	Key Issue #6 solutions evaluation	78

9.3.7	Key Issue #7 solutions evaluation	78

9.3.8	Key issue #8 solutions evaluation	78

10	Conclusions		79

10.1	General conclusions	79

10.2	Conclusions of key issue #1	79

10.3	Conclusions of key issue #2	79

10.4	Conclusions of key issue #3	79

10.5	Conclusions of key issue #4	80

10.6	Conclusions of key issue #5	80

10.7	Conclusions of key issue #6	80

10.8	Conclusions of key issue #7	80

10.9	Conclusions of key issue #8	80

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

XR (Extended Reality) refers to a set of general services which leverage High Data Rate Low Latency (HDRLL), AR/VR, and tactile/multi-modality communication.

SA1 defines the requirement for XR (Extended Reality) relevant Services. In Release-18, SA1 defined the requirement for tactile and multi-modal communication service in the 3GPP TS 22.261 clause 6.43, including the support to provide policy(ies) for flows associated with an application, and apply 3rd party provided policy(ies).

XRM in SA2 studies the key issues, solutions and conclusions on the support of advanced media services, e.g. High Data Rate Low Latency (HDRLL) services, AR/VR/XR services, and tactile/multi-modality communication services. Also, some network information is going to be exposed, including the congestion level information, Data rate, delay difference and round trip delay of QoS flow.

Considering the XR requires high bandwidth and low latency requirement, it is important to explore how to ensure transmission quality from the application enabling layer, based on the capabilities of the 5GC and the policies or application information provided by AF(s).

1	Scope

The present document studies application enabling layer architecture requirements, key issues, and solution recommendations to support XR services.

The study is based on the requirements defined in 3GPP TS 22.261 [2] and is dependent on the 5GC architecture supporting XRM services as specified in 3GPP TS 23.501 [3], 23.502 [4], and 23.503 [5].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	3GPP TS23.433: "Service Enabler Architecture Layer for Verticals (SEAL);Data Delivery enabler for vertical applications; Stage 2".

[7]	3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices".

[8]	3GPP TR 26.806: "Study on Tethering AR Glasses – Architectures, QoS and Media Aspects".

[9]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[10]	3GPP TR 22.847: "Study on supporting tactile and multi-modality".

[11]	3GPP TS 23.558:"Architecture for enabling Edge Applications".

[12]	3GPP TS 23.434:"Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[13]	3GPP TR 22.856:"Feasibility Study on Localized Mobile Metaverse Services".

[14]	IETF RFC 6184: "RTP Payload Format for H.264 Video".

[15]	IETF RFC 7798: "RTP Payload Format for High Efficiency Video Coding (HEVC) ".

[16]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[17]	3GPP TS28.538: "Management and orchestration; Edge Computing Management (ECM)".

[18]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".

[19]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[20]	3GPP TS 28.541: "5G Network Resource Model (NRM)".

[21]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[22]	What are the most common performance issues in VR video streaming? https://www.linkedin.com/advice/0/what-most-common-performance-issues-vr-video-streaming-azquc

[23] 	The Science Behind Optimized Video Delivery | Coconut©, https://www.coconut.co/articles/optimizing-video-delivery

[24] 	Zhang, Wenxiao , et al. "DeepVista: 16K Panoramic Cinema on Your Mobile Device." WWW '21: The Web Conference 2021 2021.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Key issues

4.1	Key issue #1: KPI measurement and exposure

According to clause 6.43 of 3GPP TS 22.261 [2], synchronization between different media components is crucial for immersive multi-modal VR applications. This becomes particularly important when the synchronization threshold between two or more modalities is lower than the latency Key Performance Indicator (KPI) for the application, emphasizing the significance of minimizing delay differences. Otherwise, the arrived flows need to wait for other highly related flows that do not arrived yet. The SA2 provides the 5GS Packet Delay Variation monitoring to expose the variation of packet delay measured between UE and PSA UPF, which is only the single flow delay jitter. SA2 also defines the PDU Set Delay Budget (PSDB), which is the maximum delay that a PDU Set may experience for the transfer between the UE and the N6 termination point at the UPF. Application enabling layer could help to provide the end-to-end measurement of the delay difference among those associated flows.

Another example is the duration of the burst. When the duration of the burst is long or the burst are too frequent, those indicators can suggest that the burst traffic may exceed the transmission capacity, leading to the viewer to detect frame dropping. Therefore, the duration of the burst, or, the frequency of burst may become an indicator that VAL may be interested in when providing XR services. The enabling layer can monitor the traffic to get the predicted duration of the burst based on the statistic, which can be used to do the traffic shaping.

To sum up, there are multiple indicators that may impact the performance of XR services, therefore this key issue will study:

-	Identify what KPI the application enabling layer could provide that impact the QoS of the XR services.

NOTE: Coordination with SA4 and reuse of SA2 capabilities are needed.

-	How the KPI could be measured at the application enabling layer, and the enhancement of the SEALDD services.

4.2	Key issue #2: E2E Multi-Modal Communication Flows

Many XR use cases will require E2E multi-modal communication flows between application clients and application servers. SA1 has defined requirements for tactile and multi-modal communication service in 3GPP TS 22.261 clause 6.43, including the support to provide policy(ies) for E2E multi-modal communication flows associated with applications.

Using information provided by application clients and servers (e.g., policies), the application enablement layer, in coordination with the 5G CN, can support functionality to assist in managing E2E multi-modal communication flows between application clients and application servers.

This key issue will study:

-	Whether and how to support the E2E multi-modal communication flows between application clients and application servers within the application enablement layer?

-	Whether and how to support the interaction between the application enablement layer and 5G CN to manage E2E multi-modal communication flows between application clients and application servers?

-	Whether and how SEALDD may be enhanced to assist in managing E2E multi-modal communication flows between application clients and application servers involved in the same application service (e.g., support for multi-modal aware SEALDD flow management and policies)?

NOTE 1: This key issue should not impact 3GPP Core Network.

NOTE 2: SA6 should coordinate with SA4 on E2E multi-modal communication to ensure SA6 defined features are compatible with the needs of SA4. SA4 work will be reused for this KI as much as possible.

4.3	Key Issue #3: Coordination between direct UE connection and network based connection for AR/VR services

In clause 7.6.1 of 3GPP TS 22.261 [2], the gaming or training service data can be exchanged between two 5G connected AR/VR devices. Communication over a direct link between the UEs can improve performance and service experience. Communication over 5G network and direct wireless connection can be used in parallel between AR/VR devices for redundancy or for different kinds of multi-modal call flows. Based on user consent, operator policy and trusted 3rd party request, the 5G network shall support a means to authorize specific UEs to transmit data (e.g. AR/VR service data) via direct device connection in a certain location and time or when direct device connection cannot fulfil the required QoS, communication over 5G network can provide redundant connection.

It is required to study the following:

-	How the application enablement layer can be enhanced to support usage of direct communication between 5G connected AR/VR devices.

-	Whether and how the application enablement layer can be enhanced to make coordination (e.g., the connection redundancy) between network based communication and direct communication between AR/VR devices for AR/VR services communication.

4.4	Key issue #4: Application enablement layer capabilities usage to supporting the XR services

The XRApp aims to help XR service providers in delivering immersive experience which is enabled by communicating and synchronizing data streams such that multiple participants have a consistent immersive impression.

From SA6 perspective, in addition to the SEALDD, which could provide the data delivery services, more application enablement layer capabilities could be utilized.

Taking Online AR Gaming as an example, this gaming involves two types of users. Player UEs receive streams of the live game from the game server and send control signals back to the server, and spectators who can join the live game, receive the stream from various perspectives, and potentially interact with players through cheering or game reward mechanisms. To support online AR gaming, the following capabilities could be leveraged (with potential enhancements):

-	Data delivery for guaranteed multi-modal flow transmission.

-	Network Slice Capability Enablement function for Provisioning differentiated network slices for players and spectators;

-	Location management function to facilitate geofencing for the designated game arena in AR gaming.

-	AI/ML may be required for Image & Object Recognition.

Currently, the XRApp is identified to be deployed as functionality component of the SEALDD, but there is a gap between the current architecture and the above requirements. So it is proposed to study the following open issues:

-	Whether and how the identified application enablement capabilities could be utilized to jointly support the XR service.

-	Whether and how the application enablement architecture could support the XR service taking advantage of the identified application enablement capabilities.

4.5	Key issue #5: Support the tethered UE

Different from other types of AR UE, the end-to-end path for the tethered UE includes one more wireless/wireline tethering link between AR Glasses and the tethering 5G Phone. In order to fulfill the end-to-end QoS requirements for the AR glass session, the consumer needs to acquire the tethering link status via measurement and analysis, and takes it into account when determining the QoS for the 5G system link. With the tethering link status, the application enablement layer may communicate with AF for dynamic QoS policy adjustment accordingly.

The 5G WireLess Tethered AR UE is introduced in TR26.998 [7], and is further studied in TR 26.806[8]. they provides the Segment-by-segment delay measurement solution based on the ICMP, and End-to-end delay measurement based on RTP. The End-to-end delay measurement relies on tethered device to do the RTP based measurement. While it successfully identifies non-5G delays, distinguishing delays from the tethered link and those after N6 remains challenging. The Segment-by-segment delay measurement utilizes the UPF and tethered device for ICMP based measurement, however how UPF retrieves the RTT between the UPF and the application server and further exposes the latency results to the AF are not supported in SA2 in current release. And all the measurement results are sent to the AF, indicating that SA4 does not play a role in optimizing latency based on the measurements.

The above study provides good foundation for this KI. This KI can further study how to identify tethered links and optimize transmission.

It is proposed to study the open issues:

-	Whether and how could the application enabling layer identify traffic flows from the tethered devices behind the UE.

-	Whether and how could the application enabling layer support the acquisition of tethering link status via measurement and analytic and provide a transporting optimization, considering different type of the tethered UE (standalong tethered device, or wire/wireless tethered device).

-	Whether and how the PINAPP could be utilized to support the tethered UE.

4.6	Key issue #6: XR application server selection enhancement

As for XR service, current XR application server selection mechanism using EDGEAPP may not be able to satisfy the strict latency requirement. And considering that XR application server may involve multi-model communication flow, to support multi-model communication flow synchronization, it is better to improve service experience to provide ensure the operation on the multi-model synchronization.

However, due to one DNAI may be mapped to multiple UPFs in different EDNs, thus only using DNAI to and application server may not be enough.

Consider the following case (as showed in figure 4.6.1):

The UE is in the overlapping area of EDN#1 and EDN#2, when performing service provisioning, the UE may anchor to UPF#2 however the UE may connect to the EES#1 for EAS information. Then the UE may connect to EAS#1 discovered from EES#1.

The issue may happen that the user plane path is not optimal, which may not be able to satisfy the XR application requirement.

Figure 4.6.1: Scenario on XR application server selection

This key issue includes the following aspects:

-	Whether and how enablement layer can support XR application server to further reduce service latency?

-	Whether and how enablement layer can support XR application server selection?

NOTE: For this Key issue, the coordinate with SA2 may be required.

4.7	Key issue #7: E2E KPI optimization for XR service

In real-time XR service, E2E KPI (e.g. low latency) is critical to provide immersive experience for XR users. Typically, the overall processing of XR service consists of several parts, such as media rendering, media transmission over 5G network, etc. The XR application servers may have different capabilities to perform the intensive processing, such as media rendering, which results in differentiated processing delay. In other aspects, the traffic flow between XR application client and XR application may have the different transmission quality due to the network handling. The E2E performance of XR service is impacted not only by data transmission, but also by data processing.

Therefore, further study the E2E KPI optimization mechanism considering the processing delay of XR application server, to satisfy real-time XR service is required. For example, as the enable layer, how to discover or initiate the appropriate XR application server and/or how to interact with 5GS is critical to satisfy the E2E performance of XR service.

This key issue includes the following aspects:

-	How to support the E2E KPI optimization to meet real-time XR service requirements, taking the processing delay of XR application server into account, e.g. considering the potential enhancements to EDGEAPP in XR application server discovery or instantiation.

-	What available information from 5G network can be used by enable layer to optimize the real-time XR service requirements.

4.8	Key issue #8: PDU set handling for XR traffic

SA2 and SA4 introduced PDU set based handling to support XR traffic. As mentioned in 3GPP TS 23.501 [3] and 3GPP TS 26.522 [9], for multi-modal flows transported in different QoS flow, the importance of PDU set can be used to help RAN to discard PDUs belonging to the same PDU set across flows, when needed. The PDU set identification and corresponding set-based policy enforcement are performed in CN and RAN, and inclusion of PDU set information is done in application layer.

For XR DL data, PSA UPF can mark PDU set information in GTP based on received protocol extension, e.g., RTP extension. Then during AN congestion, 5G-RAN and/or UPF will perform specific packet handling (e.g. drop) for PDUs in a set and also for PDU sets in the same SDF based on PDU set information in GTP.

For XR UL data, UE lower layer can mark PDU set information in AN-protocol based on received protocol extension, e.g., RTP extension. Then during AN congestion, 5G-RAN will perform packet handling for PDUs in a set and also for PDU sets in the same SDF based on PDU set information in AN-protocol.

SEALDD was specified in 3GPP since Rel-18, the user plane protocol stack over 5GS is depicted in Figure 4.8-1. The Application content including SEALDD layer information and VAL payload can be transferred between the SEALDD client in the UE and the SEALDD server in the data network.

Figure 4.8-1: User plane protocol stack for SEALDD traffic transmission over 5GS

NOTE:	PDU layer corresponds to the PDU carried between the UE and the DN over the PDU Session. E.g. UDP/IP, TCP/IP, QUIC/UDP/IP.

Since the inclusion of PDU set information is done in application layer, the application enablement layer, in coordination with 5G CN, can also facilitate PDU set related handling for a single flow or multiple flows in XR traffic.

This key issue will study:

-	Whether and how SEALDD or other application enabler may be enhanced to facilitate XR communication flows between application clients and application servers involved in the same application service in relation to PDU set (e.g., support for PDU set handling, QoS measurement)?

5	Application enablement architecture requirements

5.1	General requirements

The general architecture requirements specified in clause 6.1 to 6.4 of 3GPP TS 23.433 [6] are applicable for XRApp service.

5.2	Support for direct UE connection

5.2.1	Description

This clause specifies the requirements related to support for coordination between direct UE connection and network based connection.

5.2.2	Requirements

[AR-5.2.2-a] The architecture shall support a mechanism to authorize specific UEs to transmit XR data via direct device connection.

[AR-5.2.2-b] The architecture shall support a mechanism for monitoring the availability and QoS of direct link between UEs for XR service data transmission.

[AR-5.2.2-c] The architecture shall be capable of reporting the availability and QoS of direct link for XR service data transmission.

[AR-5.2.2-d] The architecture shall be capable of making network based connections take over the XR data transmission when direct device connection cannot fulfil the required QoS or vice versa.

5.3	Multi-modal flows alignment and monitoring requirements

5.3.1	Description

This subclause specifies the requirements for multi-modal flows alignment and monitoring service.

5.3.2	Requirements

[AR-5.3.2-a] The XRApp shall provide a mechanism to enable multi-modal flows alignment by the authorized VAL server.

[AR-5.3.2-b] The XRApp shall provide a mechanism to enable multi-modal flows alignment monitoring by the authorized users or VAL server.

5.4	Tethered device discovery and monitoring requirements

5.4.1	Description

This subclause specifies the requirements to support the tethered device.

5.4.2	Requirements

[AR-5.4.2-a] The XRApp shall provide a mechanism to discover the tethered device for the XR service.

[AR-5.4.2-b] The XRApp shall provide a mechanism to monitor the tethered device by measuring the tethering link for the XR service.

6	Application enablement architecture for XR services

6.1	Application enablement architecture

6.1.1	General

This clause provides the overall architecture description.

6.1.2	Option1: Application enablement architecture for XRAPP function

When the XRApp function is a SEAL layer function, it could be deployment implementation by integrating the related enabler function (e.g. SEALDD server, NSCE server) to support XR application.

The XRApp function provides the service to VAL server over XRApp-S. The XRApp function interact with XRApp client over XRApp-UU. The XRApp client communicates with the XRApp client on the other device (i.e., tethered device) over the XRApp-UUe.

Figure 6.1.2-1: overview of XRApp function

6.1.2.1	Functional Elements

6.1.2.1.1	Server-side XRApp Functionality

The XRApp functional entity provides for the XR service enablement.

6.1.2.1.2	Client-side XRApp Functionality

The XRApp client functional entity acts as the application client for enabling the XR related service.

6.1.3	Option2: Application enablement architecture based on existing application enablement capabilities

Instead of defining an additional independent architecture, XR related services could be supported by enhancing relevant existing functions(e.g., SEALDD).

6.1.3.1	Application enablement architecture based on SEALDD

To facilitate the specific optimization for XR application provided by 5G network, the application enablement architecture for the XRApp service is based on the generic functional model specified in3GPPTS23.434[4] and 3GPPTS23.433 [6].

The XRApp can be deployed as functionality component of the SEALDD, then the architecture for SEALDD is applicable to XRApp.

Figure 6.1.3.1-1:Application enablement architecture based on SEALDD

6.1.3.2	Mapping of multi-modal flows to reference points

Figure 6.3.1-1 illustrates an example mapping of SEALDD multi-modal flows to the reference points of the application enablement architecture.

Multi-modal services are based on several data flows related to each other and subject to application coordination. The data flows can transfer different types of data (for example audio, video, positioning, haptic data) and may come from different sources(e.g. a single UE, a single device or multiple devices connected to the single UE, or multiple UEs). (e.g. 3GPP TR 23.501[3] clause 5.37.2.

To support E2E multi-modal communication flows between multiple VAL clients and servers (e.g. 3GPP TR 22.847[10] clause 4.2), a SEALDD server and clients may support multi-modal service capabilities such as establishing and managing SEALDD multi-modal flows. Via SEALDD multi-modal flows, the SEALDD server and clients perform multi-modal traffic transfer and management processing (e.g., E2E synchronization of application traffic having multi-modal dependencies with one another).

Figure 6.1.3.2-1: Example SEALDD multi-modal flows mapping to reference points.

Figure 6.1.3.2-1 illustrates XR services using two different XR Servers (A and B). The services provided are used in one case by a single integrated VAL client, VAL client 3, on UE Y. The services provided are also used by UE X with VAL XR client 2 on UE X and VAL XR client 1 on a device tethered to UE X (see 3GPP TR 26.806[8] clause 4.4).

6.1.3.3	Application enablement architecture based on SEALDD and PINAPP to support the Tethered device

Based on types of tethered device, there are two type of Application enablement architecture based on SEALDD and PINAPP to support the Tethered device.

For the tethered XR device, where XR application client is deployed on the tethered UE device, the application enablement layer architecture is shown in the figure 6.1.3.3-1. The SEALDD client on the 3GPP UE, acting as PIN client, could get tethered device information from the PEMC using the PIN-3 interface. Then the SEALDD client could interact with XR client on the tethered device to do the tethered link measurement. Some enhancement may be needed to support the interaction between the SEALDD client and XR client on different device over SEALDD-C.

Figure 6.1.3.3-1. Application enablement architecture based on SEALDD and PINAPP without SEALDD client in the tethering device

For the Tethered Display XR device, where the XR application is deployed on the 3GPP UE and only the XR runtime is on the tethered UE,  the application enablement layer architecture is shown in the figure 6.1.3.3-2. The SEALDD client on the 3GPP UE, acting as PIN client, could get tethered device information from the PEMC using the PIN-3 interface. The SEALDD-UUc interface needs to be defined to support the interaction between the SEALDD client on the tethered UE and SEALDD client on the 3GPP UE.

Figure 6.1.3.3-2. Application enablement architecture based on SEALDD and PINAPP with SEALDD client in the tethering device

6.1.3.4	Reference Points

In addition to the reference points defined in SEALDD and PINAPP, the following reference points need to be supported.

6.1.3.4.1	SEALDD-C

SEALDD-C reference point enablers the interaction between the VAL client on the tethered device and SEALDD client on the 3GPP device.

6.1.3.4.1	SEALDD-UUc

SEALDD-UUc reference point enablers the interaction between the SEALDD client on the tethered device and SEALDD client on the 3GPP device.

6.1.4	Option3: CAPIF architecture to support the service discovery and invocation for XR application

The border AEF in existing CAPIF architecture in 3GPP TS 23.222 [18] can be utilized to support the XR service by invoking the northbound API exposed by the application enablement (e.g. SEAL, EDGEAPP) or the northbound API exposed by 5GS, as described in Figure 6.1.4-1.

Figure 6.1.4-1: Reuse CAPIF architecture to support the service discovery and invocation for XR application

The XR application acts as API invoker performs the API discovery and receives the details of the service API (i.e. border AEF).  Then the XR application performs service API invocation according to the border AEF address. The border AEF acts as the entry endpoint to route service API request to multiple AEFs (e.g. EDGEAPP service, SEAL service) based on the routing information obtained from CCF.

NOTE 1:  The detailed services of enablement layer need to be invoked depends on the specific scenario and use case for XR application.

NOTE 2: The EDGEAPP service in this part refers to the service discovery and invocation in EDGE-3 interface between EAS and EES.

NOTE 3: The border AEF in this part is the entry point to hide the AEF topology, it is not mandatory depends on the specific deployment for service exposure.

7	Solutions

7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  | X |  |  |  |  |  |  |
| Sol #2 |  | X |  |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |  |  |
| Sol #4 | X |  |  |  | X |  |  | X |
| Sol #5 |  | X |  |  |  |  |  |  |
| Sol #6 | X |  |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  |  | X |
| Sol #8 |  |  |  |  |  |  | X |  |
| Sol #9 |  |  |  |  | X |  |  |  |
| Sol #10 |  |  |  |  |  | X |  |  |
| Sol #11 |  |  |  |  | X |  |  |  |
| Sol #12 |  |  |  |  |  | X |  |  |
| Sol #13 |  |  |  |  |  | X |  |  |
| Sol #14 |  |  | X |  |  |  |  |  |
| Sol #15 |  |  |  | X |  |  |  |  |
| Sol #16 |  |  |  | X |  |  |  |  |

7.1	Solution #1: SEALDD Policy Configuration for multi-modal flows

7.1.1	Architecture Impacts

This solution adds support for multi-modal SEALDD policy configuration functionality by enhancing the existing SEALDD policy configuration functionality supported in clause 9.10 of 3GPP TS 23.433[6]. There are no impacts to the SEALDD architecture.

7.1.2	Solution description

This solution is targeting KI #2: E2E Multi-Modal Communication Flows and specifies enhancements to the SEALDD policy configuration information flows. In this procedure, the VAL server is a specific server for configuring multi-modal SEALDD policies and is different from VAL servers used for VAL application processing.

The SEALDD policy configuration procedure detailed in 3GPP TS23.433[6] clause 9.10.2.1 is reused with the enhanced information flows below

NOTE 1:	Corresponding enhancements to multi-modal SEALDD policy update and delete procedures will be completed during the normative phase.

7.1.3	Information flows

7.1.3.1	SEALDD policy configuration request for multi-modal flows

Table 7.1.3.1-1 describes the new information elements to be added to the SEALDD policy configuration request in 3GPP TS 23.434[6] Table 9.10.3.1-1 in order to support multi-modal flows.

Table 7.1.3.1-1: SEALDD policy configuration request IEs for multi-modal flows

| Information element | Status | Description |
| --- | --- | --- |
| Multi-modal VAL flows list | M | List of individual VAL flows applicable to this multi-modal SEALDD policy. |
| > Application traffic identifiers | M | Identifier of the application traffic for this individual VAL flow (e.g. VAL server ID, VAL service ID) |
| > VAL UE identity | O | Identifier of the VAL UE for this individual VAL flow |
| > SEALDD policy | M | SEALDD policy associated with this individual VAL flow as defined in Table 9.10.3.1-1 of 3GPP TS 23.433 |
| Multi-modal SEALDD policy | M | Multi-modal SEALDD policy applicable to Multi-modal VAL flows list. This IE is used for complementing other SEALDD measurement policies for multi-modal flows |
| > Burst policy | O (See NOTE 1) | Policy used by the SEALDD layer to determine multi-modal flow configuration |
| > Synchronization policy | O (See NOTE 1) | Policy used by the SEALDD layer to perform multi-modal synchronization operations. |
| > Policy expiration time | O | Indicates the validity period of the policy |
| NOTE 1: At least one of these multi-modal SEALDD policies shall be present in the request |  |  |

NOTE 2:	The burst policy and the synchronization policy, and their use, are to be harmonized with KI#1 solutions.

Editor's Note:	How to harmonize the burst related parameters considering burst policy in DD policy is FFS.

7.1.4	Solution evaluation

This solution addresses KI #2: E2E Multi-Modal Communication Flows by proposing enhancements to the SEALDD policy configuration information elements. There are no impacts to the SEALDD architecture.

7.2	Solution #2: Policy based Multi-modal SEALDD flow establishment

7.2.1	Architecture Impacts

This solution describes policy-based multi-modal SEALDD flow establishment functionality.

7.2.2	Solution description

This solution is targeting KI #2: E2E Multi-Modal Communication Flows and specifies a policy based multi-modal SEALDD flow establishment procedure.

NOTE 1:	In the normative phase, this solution is to be harmonized with the corresponding solution for multi-modal application data delivery initiated by VAL server, as well as with clause 9.2.2.3 of 3GPP TS 23.433.

NOTE 2:	A corresponding procedure for deletion of a multi-modal SEALDD flow is to be defined during the normative phase.

7.2.2.1	Multi-modal SEALDD flow establishment based on policy

Figure 7.2.2.1 illustrates the procedure for a SEALDD server to establish a multi-modal SEALDD flow based on a configured multi-modal SEALDD policy. The multi-modal SEALDD flow is established by the SEALDD server before multi-modal application communication between VAL clients and VAL servers starts.

Pre-conditions:

1.	The SEALDD server has been configured with a multi-modal SEALDD policy.

Figure 7.2.2.1-1: Policy based multi-modal SEALDD flow establishment

1.	The VAL servers subscribe to SEALDD event exposure for connection status using the procedure defined in 3GPP TS 23.433[6] clause 9.2.2.6.

2.	Based on the configured multi-modal SEALDD policy information, the SEALDD server determines the applicable VAL flows and triggers the establishment of required individual SEALDD connections as described in 3GPP TS 23.433 clause 9.2.2.3. Next, the SEALDD server triggers the establishment of a multi-modal SEALDD flow and associates the individual SEALDD connections with it. To maintain this association, the SEALDD server assigns a multi-modal SEALDD flow identifier and associates it with the identifiers of each individual SEALDD connection.

3.	During establishment of the multi-modal SEALDD flow, the SEALDD server interacts with the 3GPP CN to configure multi-modal QoS flow parameters based on the multi-modal SEALDD policy information. The SEALDD server utilizes the NEF/PCF/NRM/EES service for multi-modal QoS adjustment. Specifically, the SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [4] and 3GPP TS 23.503 [5], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service API and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [4] and 3GPP TS 23.503 [5], if the SEALDD server is connected to the PCF via NEF. SEALDD relies also upon the EES Session with QoS API as specified in 3GPP TS 23.558 [11] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [12].

4.	The SEALDD server allocates an IP address(s) and port(s) for sending and receiving packet over the SEAL-S reference point. The SEALDD server then sends SEALDD connection establishment notification with VAL service ID, the IP address and port to each of the VAL servers.

5.	The SEALDD server allocates an IP address and port for sending and receiving packet over the SEAL-UU reference point. The SEALDD server then sends a regular data transmission connection establishment request to the SEALDD client with SEALDD flow ID, VAL service ID, the IP address and port and receives a response. UE IP address (and port) can be included by the SEALDD client in the response or sent in a separate update message by SEALDD client if a different UE IP address is to be used in SEALDD connection user plane.

NOTE 1:	Step 4 and step 5 can be done in parallel.

NOTE 2:	Step 5 can be sent via PDU session (if exist) or via application triggering (if no PDU session exists).

6.	The SEALDD client further notifies VAL clients about the SEALDD connections being established.

Upon receiving multi-modal application traffic from VAL clients (not shown in the figure), the SEALDD client sends it to SEALDD server in SEALDD traffic. The SEALDD server identifies multi-modal application traffic based on the VAL service ID and further sends the multi-modal application traffic to the VAL servers. The downlink multi-modal application traffic sent from VAL server to VAL client is processed similarly.

Editor's Note: It is FFS whether this procedure applies to multiple XR VAL clients on the same SEAL-UU connection.

7.2.3	Solution evaluation

This solution enables end-to-end multi-modal connection between VAL client and VAL server for application data delivery, with the connection initiation being based on policy, and it is well-aligned with the existing functionality in clause 9.2.2.3 of 3GPP TS 23.433. The solution has minimal architectural impacts.

7.3	Solution #3: Support of Multi-Modal traffic indication in SEALDD layer

7.3.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [6].

7.3.2	Solution description

7.3.2.1	General

This solution addresses the improvement of the SEALDD layer to support Multi-Modal service.

The solution improves the SEALDD layer defined in 3GPP TS 23.433, i.e.:

1.	The multiple flows can be requested in one SEALDD enabled regular data transmission connection establishment.

2.	The SEALDD server uses 5GC capabilities to configure XR traffic flows.

In this solution, the following definitions are used:

1.	Multi-Modal Service Identifier is an identifier of the multi-modal service as defined in clause 6.1.3.27.3 of 3GPP TS 23.503 [5].

2.	Multi-Modal Service Requirements is requirements for the multi-modal traffic handling in 5GC as defined in clause 6.1.3.27.3 of 3GPP TS 23.503 [5].

In this solution, the following assumptions are used:

1.	The VAL service identified by the VAL Service ID serves only one XR application. Thus, the VAL Service ID functionally equals to the Multi-Modal Service Identifier and may be generated by the SEALDD server based on VAL service ID and used by the SEALDD server for negotiation with 5GC via N33/N5 interfaces for the XR traffic handling.

2.	The VAL server(s) within one VAL Service may provide different functional parts of the XR application, e.g., VAL Server A performs video flow(s) processing, VAL Server B performs audio flow(s) processing, and/or VAL Server C performs both video and audio flows processing. Thus, the Multi-Modal Service Requirements for the XR flows may be determined and generated by the SEALDD server based on VAL service ID and/or VAL server ID for negotiation with 5GC via N33/N5 interfaces for the XR traffic handling.

NOTE:	The Multi Model Service Identifier is not used in flow correlation.

7.3.2.2	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.433 [6] can be enhanced (highlighted in bold italics) as follows.

NOTE:	SEALDD data transmission procedures with single SEALDD connection are described as typical example below.

9.2.2.2 SEALDD enabled regular data transmission connection establishment procedure Figure 9.2.2.2-1 illustrates the procedure for establishing regular SEALDD data transmission connection. Pre-condition: - The VAL server can discover and select the SEALDD server by CAPIF functions. Figure 9.2.2.2-1: SEALDD enabled regular data transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for application traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_RegularTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, list of requested flows, including SEALDD-S Data transmission connection information of the VAL server side, and optionally, the QoS information for the application traffic, e.g. QoS requirements. 2. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates address/port of the SEALDD server to receive the packets from the VAL server for application data transfer as SEALDD-S data transmission connection information of the SEALDD server side for each requested flow provided in step 1. The SEALDD server allocates a specific address or port used for SEALDD traffic transfer with the specific UE for the VAL server and responds with a SEALDD service response (including SEALDD-S data transmission connection information of the SEALDD server side) for each requested flow provided in step 1. The VAL server and SEALDD server can use SEALDD-S data transmission connection information to establish the data transmission connection between VAL server and SEALDD server for application data transfer. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1, as defined in clause 5.2.6.9 and in clause 5.2.5.3 of 3GPP TS 23.502 [6]. The AF request includes the application traffic descriptor containing the address or ports allocated by SEALDD server, and the QoS information for application traffic, and may include Multi-Modal Service ID together with Multi-modal Service Requirements. The QoS information may be determined by SEALDD server according to VAL service ID for different service type of application traffic if the QoS information is not provided by VAL server. The Multi-Modal Service ID may be determined by the SEALDD server according to VAL service ID. The Multi-modal Service Requirements may be determined by the SEALDD server according to VAL service ID and/or VAL server ID. The SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service APIs and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QoS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4]. NOTE 1: The SEALDD-S data transmission connection information of the SEALDD server side is optional to respond to the VAL server, if the SEALDD server uses the downlink pull mode to obtain the data/content from the address provided by the VAL server in step 1, and uses the uplink push mode to send the data/content to the address provided by VAL server. 3. Data transmission sessions information is provisioned to the VAL client by the VAL server via application signalling. NOTE 2: The application signalling may be transmitted via direct application layer connection or via the SEALDD layer. 4. The VAL client sends a SEALDD service request to SEALDD client. The VAL client receives a SEALDD service response to the SEALDD client. The response indicates that whether the SEALDD service request is successful or not. 5. The VAL/SEALDD client discover and select the proper SEALDD server for the VAL application, as described in clause 9.4.3. After this step, the VAL server is discovered and selected along with the associated SEALDD server, the SEALDD client can get the SEALDD server's address. 6. The SEALDD client allocates a SEALDD flow ID mapping to the identifiers of the application traffic. The SEALDD client sends Sdd_RegularTransmissionConnection_Establish request to SEALDD server with the SEALDD client ID, the SEALDD flow ID, the SEALDD traffic descriptor of the SEALDD client side (the address/port of the SEALDD client for receiving the downlink SEALDD traffic), VAL server ID, VAL service ID. The request message also contains the selected VAL server endpoint information and UEID. The SEALDD server retrieves the location information of the VAL UE or SEALDD client from SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12 and verifies with the Geofence policy configured by the VAL server to allow or restrict the data connection establishment. If the location information is allowed as per the configured geofence policy then the SEALDD server allows the data connection establishment, otherwise SEALDD server returns a failed result e.g. performs connection reject. NOTE 3: The SEALDD server can use or update the association between SEALDD-UU connection and SEALDD-S connection that associated with UE ID, VAL service ID, VAL server endpoint, which is used to correlate the SEALDD traffic and the VAL application traffic. NOTE 4: The SEALDD flow ID is used by the SEALDD client and SEALDD server to identify different VAL application traffic of the same SEALDD client. The SEALDD flow ID may be same with the identifiers of the application traffic or new simplified IDs allocated by SEALDD. 7. The SEALDD server responds to the SEALDD client with the SEALDD traffic descriptor of SEALDD server side (e.g. address/port allocated in step 2, transport layer protocol) mapping to the application traffic. 8. If the connection between VAL server and SEALDD server is not established in step 2, the SEALDD server establishes connection with VAL server for the VAL client to transmit application traffic mapping to the SEALDD traffic according to the SEALDD-S information negotiated in step 1-2. 9. The SEALDD client uses the SEALDD traffic descriptor of SEALDD server side for SEALDD connection establishment. After this step, the SEALDD client and SEALDD server both get the whole SEALDD traffic descriptor (including the UE's address/port and SEALDD server's address/port for the SEALDD traffic transmission). 10. If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-9 for each requested flow provided in step 1. NOTE 5: The way for the multiple flows configuration between SEALDD client and SEALDD server (i.e., re-use/modify Sdd_RegularTransmissionConnection_Establish procedure or use other new introduced procedure) will be decided in the normative stage. After the negotiation and establishment of the connections, the SEALDD client gets the mapping information between application traffic and SEALDD flow ID. The SEALDD server gets the mapping information between the SEALDD flow ID and the SEALDD-S connection. Upon receiving application traffic from VAL client, the SEALDD maps it to SEALDD traffic with SEALDD traffic descriptors as negotiated with SEALDD server in step 6 and step 7. The SEALDD traffic is sent to the SEALDD server. The SEALDD server maps the SEALDD traffic to the application traffic according to the stored SEALDD traffic descriptor, SEALDD client ID and SEALDD flow ID. The SEALDD server sends the recovered application traffic to the address provided by VAL server in step 1, via the connection established in step 2 or 8 according to the mapping information. The downlink application traffic sent from VAL server to VAL client is processed similarly. The SEALDD server receives any UE location change notification using SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12, then the SEALDD server performs the data delivery in alignment with the geofence policy. If the UE is in the forbidden location or not allowed for the given VAL service to send/receive data as per the Geofence policy, then the SEALDD server performs action like releases the connection and informs the VAL server that UE is not reachable because in a forbidden location using connection event status procedure. If UE enters the allowed location area then the SEALDD server initiates the connection establishment using the procedure defined in clause 9.2.2.3. 9.2.3.1 SEALDD enabled regular transmission request Table 9.2.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the regular application transmission service. Table 9.2.3.1-1: SEALDD enabled Regular transmission request Information element; Status; Description. VAL server ID; M; Identity of the VAL server. VAL service ID; O; Identity of the VAL service. UE identity; O; Identifier of specific UE, e.g. UE ID. List of requested flows; M; List of the requested multi-modal flows. >SEALDD-S Data transmission connection information; M; Address/port and/or URL of the VAL server to receive the application packets from the SEALDD server. > QoS information; O; QoS information provided by VAL server. >VAL server’s total bandwidth limit; O (See NOTE); The total bandwidth limit of VAL server, including UL/DL. >VAL users’ bandwidth limit; O (See NOTE); The bandwidth limits (i.e. minimum bandwidth requirement and maximum bandwidth limit) for VAL users, including UL/DL. NOTE: These IEs are used for the SEALDD enabled bandwidth control for different VAL users.. 9.2.3.2 SEALDD enabled regular transmission response Table 9.2.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the regular application transmission. Table 9.2.3.2-1: SEALDD enabled regular transmission response Information element; Status; Description. Result; M; Success or failure.. List of SEALDD-S information Data transmission connection information; O; List of Address/port and/or URL of the SEALDD server to receive the packets from the VAL server for application traffic transfer (NOTE 2). Cause; O See NOTE; Indicates the reason for the failure, e.g. SEALDD policy mismatch.. NOTE 1: The IE is only present if the Result is failure. NOTE 2: The order of elements in this list matches the order of elements in the List of requested flows IE defined in table 9.2.3.1-1.

7.3.3	Solution evaluation

This solution addresses KI#2 for the following aspects:

-	Whether and how to support the interaction between the application enablement layer and 5G CN to manage E2E multi-modal communication flows between application clients and application servers?

-	Whether and how SEALDD may be enhanced to assist in managing E2E multi-modal communication flows between application clients and application servers involved in the same application service (e.g., support for multi-modal aware SEALDD flow management and policies)?

7.4	Solution #4: Support of QoS measurement for Multi-Modal traffic in SEALDD layer

7.4.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [6].

7.4.2	Solution description

The current QoS measurement in SEALDD layer supports the QoS measurements with VAL UE, VAL Group, or VAL UE identity list. These three options of the monitoring granularity are not able to fulfil the XR application scenario due to the XR application scenario requires the QoS measurement granularity per Multi-Modal flow and each VAL UE may have two or more active Multi-Modal flows at the same time.

The following definitions are used in the solution, i.e.:

1.	Traffic descriptor is an endpoint (e.g., Address/Port or URL) at the SEALDD server for the SEALDD traffic. The traffic descriptor uniquely identifies the flow and can be used for the flow identification in the QoS measurement request.

2.	Crossflow QoS measurement is a QoS measurement that considers: XR traffic route (e.g. {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]}), and possible DL flows synchronization overhead.

NOTE:	The synchronization overhead includes latency, bitrate, etc.

The XR scenario requires finer granularity for the QoS measurements, crossflow QoS measurement and the mapping between QoS measurements and XR metrics, i.e.:

1.	The current QoS measurements in SEALDD layer supports end-to-end QoS measurements only; thus, an additional granularity for QoS measurements like UL/DL needs to be added for XR scenarios.

2.	The traffic descriptor (Address/port or URL at the SEALDD server) is proposed to use for the Multi-Modal flow identification.

3.	The crossflow QoS measurement provides added value for the XR service, for example, Motion to Photon can be estimated as crossflow RTT measurement (e.g. {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]}). The detailed definition Motion-to-Photon is provided in clause 7.6.1 in 3GPP TS 22.856[13].

The solution supports the QoS measurement when the endpoint for the XR traffic is located at the tethered device, i.e.:

-	Assumption: the tethered device may not have the direct connection to the SEALDD server.

-	The flow traffic descriptor (e.g., Address/port or URL) at the SEALDD server is used to identify the end-to-end traffic flow including tethered device.

-	The QoS measurement in SEALDD layer takes under consideration the characteristics of the link between SEALDD client and the tethered device.

-	The SEALDD client at tethering UE may utilize PINAPP service to discover the tethered device.

The solution improves the SEALDD layer defined in 3GPP TS 23.433, i.e.:

1.	The traffic descriptor(s) and crossflow measurement information are included into the SEALDD enabled data transmission quality measurement subscription request in order to signal the multi-modal service QoS request in SEALDD layer;

2.	Finer granularity of QoS measurement identifiers, i.e., UL/DL/E2E. Example: UL jitter.

3.	Improvement of the SEALDD Client for the crossflow RTT measurement for multiple devices in XR scenario, e.g., wireless controllers and headset interaction.

4.	Improvement of the SEALDD Client for tethered UE QoS measurement.

7.4.2.2	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.433 [6] can be enhanced (highlighted in bold italics) as follows.

NOTE:	SEALDD data transmission procedures with single SEALDD connection are described as typical example below.

9.7.2.1 Data transmission quality measurement Figure 9.7.2-1 illustrate the procedure for SEALDD enabled data transmission quality measurement. The SEALDD client and SEALDD server is enhanced to carry out the data transmission quality measurement. Pre-conditions: 1. The SEALDD server and SEALDD client are synchronized to the time source provided by 5GS as specified in 3GPP TS 23.501 [5]. 2. The VAL server discovers and selects the SEALDD server by CAPIF functions. Figure 9.7.2.1-1: SEALDD enabled data transmission quality measurement procedure 1. An on-going regular data transmission connection is established according to clause 9.2.2.2. 2. The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), requirement of transmission quality measurement (e.g. latency, jitter, bitrate, packet loss rate) and measurement target UE (e.g. a single UE, a group of UEs or all UEs), flow(s)traffic descriptor(s), and may also include reporting criteria, reporting frequency, spatial condition and temporal condition. NOTE: The spatial and/or temporal condition can be used by SEALDD server to apply when and where the measurement is performed. For instance, the measurement is expected to be done for a group of VAL UEs with a scheduled route (from city A to city B via highway A2 and A3), from 9:00 a.m. to 11:00 a.m. on Tuesday and from 1:00 p.m. to 5:00 p.m. on Thursday. 3. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends a response to the VAL server with the subscription ID, expiration time. 4. If the transmission quality measurement requirement list provided by VAL server in step 2, indicates that the latency is needed to be measured, the SEALDD server initiates the DL packet delay measurement. The SEALDD server encapsulates the DL monitoring packet (i.e. DL SEALDD packet with SEALDD DL monitoring header and VAL traffic as payload, or dummy DL SEALDD packet generated for data transmission quality monitoring) with local time T1 when the SEALDD server sends out the DL monitoring packets. The SEALDD server considers the spatial and/or temporal conditions when starting/resuming the transmission quality measurement. If the conditions are not satisfied, the SEALDD server stops/suspends the transmission quality measurement. NOTE 1: For other metrics in transmission quality measurement requirement list (e.g. bitrate), the transmission quality result can be obtained by performance detection on the SEALDD server within a period of time. 5. The SEALDD client receives the DL monitoring packet, and records the local time T2. If the enpoint for the SEALDD traffic is located at the tethered device, the SEALDD client in 5G UE measures the QoS between the SEALDD client in 5G UE and the SEALDD client (i.e. for architecture that has the SEALDD client on the tethered device) or the VAL client (i.e. for architecture that without the SEALDD client on the tethered device) in tethered device, e.g., the SEALDD client may measure the tethered link delay using the ICMP ping protocol as defined in IETF RFC792 [RFC792]). NOTE 2: Only RTT can be measured for the link between SEALDD client and the tethered device. NOTE 3: The figure will be updated to add another SEALDD client in tethered device in normative phase if the architecture option#1 is used. NOTE 4: The tethered link measurement between SEALDD client and VAL client is based on assumption that there is business agreement between the VAL provider and the SEALDD provider, which request the VAL client to respond to ICMP Ping packet. 6. Similarly, the SEALDD client encapsulates the UL monitoring packet (i.e. UL SEALDD packet with SEALDD UL monitoring header and VAL traffic as payload, or dummy UL SEALDD packet generated for data transmission quality monitoring) with local time T2 recorded in step 5 and local time T3 when the SEALDD client sends out the UL monitoring packet. If the endpoint for the VAL traffic is located at the tethered device, the measurement delay in the tethered link (step 5) is considered in T2 and T3 timestamps, e.g., T2 timestamp includes the half of the measurement delay and T3 timestamp excludes the half of the measurement delay. NOTE 5: With ICMP ping over tethered link, it is assumed that the UL packet delay and the DL packet delay are the same between the tethering UE and the tethered device. 7. The SEALDD server records the local time T4 when the SEALDD server receives the UL monitoring packet and calculates the latency with T1, T2, T3, T4. The SEALDD server can also calculate the bitrate, jitter and packet loss rate over a certain period over a specific SEALDD connection by recording the status of the SEALDD packets carrying VAL traffic or dummy SEALDD packets generated for transmission quality measurement reports. The SEAL DD server also evaluates the reporting criteria if present in the SEALDD transmission quality measurement subscription request in order to generate the transmission quality measurement report. 8. The SEALDD server reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, packet loss rate) to the VAL server via the notification message. When a VAL group ID or a list of VAL UE IDs or all VAL UEs indication is received in step 2, step 4 to step 7 is repeated for VAL UEs in the group/list or for all VAL UEs. The SEALDD server maps the VAL UE group ID to a list of VAL UE IDs if a VAL group ID is received. The SEALDD server identifies SEALDD connections corresponding to the desired VAL UE(s) or flow(s)traffic descriptor(s) to trigger measurement. And depending on the reporting requirement for multiple UEs, the SEALDD server calculates the needed report for the VAL server. When the VAL server decides to update or unsubscribe transmission quality measurement subscription after performing step 2 and step 3, the VAL server can send data transmission quality measurement subscription update request and data transmission quality measurement unsubscribe request to SEALDD server, as specified in Table 9.7.3.9-1 and Table 9.7.3.11-1, respectively. 9.7.2.2 Data transmission quality query Figure 9.7.2.2-1 illustrates the procedure for SEALDD enabled data transmission quality query. This procedure is used to obtain the historical transmission quality result already measured as described in clause 9.7.2.1. Pre-conditions: 1. The SEALDD server performs the data transmission quality measurement procedure, as described in clause 9.7.2.1. Figure 9.7.2.2-1: SEALDD enabled data transmission quality query procedure 1. The consumers (e.g. VAL server, SEALDD server, NSCE server, ADAE server) can send a SEALDD transmission quality query request to the SEALDD server to obtain the transmission quality measurement result. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), VAL UE ID or VAL UE group ID, flow(s)traffic descriptor(s), or crossflow measurement information. 2. The SEALDD server responds with the transmission quality measurement result (e.g. packet delay, bitrate, packet loss rate). 9.7.2.3 Data transmission quality measurement reported by SEALDD client Figure 9.7.2.3-1 illustrate the procedure for SEALDD enabled data transmission quality measurement for VAL traffic. The SEALDD client receives transmission quality measurement requirement, decides to start VAL data transmission monitoring and generates measurement reports. Figure 9.7.2.3-1: VAL data transmission quality measurement reported by SEALDD client 1. An on-going regular data transmission connection is established according to clause 9.2.2.2. The transmission quality measurement can be triggered by VAL server or VAL client, which is described in step 2 to step 5 and step 6, correspondingly. The request may include the crossflow measurement information (e.g. {[traffic descriptor 1, UL]; [traffic descriptor 2/DL]}) associated in the same multi-modal service for crossflow RTT measurement. 2. The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), requirement of transmission quality measurement (e.g. latency, jitter, bitrate) and measurement target UE (e.g. a single UE, a group of UEs or all UEs), and may also include reporting criteria, reporting frequency, spatial condition and temporal condition. NOTE 1: The spatial and/or temporal condition can be used by SEALDD client to apply when and where the measurement is performed. For instance, the measurement is expected to be done for a group of VAL UEs with a scheduled route (from city A to city B via highway A2 and A3), from 9:00 a.m. to 11:00 a.m. on Tuesday and from 1:00 p.m. to 5:00 p.m. on Thursday. 3. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server responds to the VAL server. 4-5. The SEALDD server sends a SEALDD transmission quality measurement subscription request to the SEALDD client and the SEALDD client responds to the SEALDD server. The SEALDD client, based on the received service quality guarantee policy including thresholds and action, can take corrective action as described in clause 9.7.2.3. If crossflow RTT measurement requirement is received from the VAL server, the SEALDD server identifies the SEALDD connections for the involved VAL UEs and sends to the SEALDD client(s) with crossflow RTT measurement requirement including corresponding UL/DL flow information. 6. The VAL client triggers the SEALDD transmission quality measurement procedure to the SEALDD client, in order to collect the measurement report information. 7. After SEALDD client determines to start measurement process, upon UL packet arrival, the SEALDD client initiates the UL packet delay measurement. The SEALDD client encapsulates the UL monitoring packet (i.e. UL SEALDD packet with SEALDD UL monitoring header and VAL traffic as payload for VAL data transmission quality monitoring) with local time T1 when the SEALDD client sends out the UL monitoring packet. The SEALDD client considers the spatial and/or temporal conditions when starting/resuming the transmission quality measurement. If the conditions are not satisfied, the SEALDD client stops/suspends the transmission quality measurement. For crossflow RTT measurement, the SEALDD client received UL flow information starts recording local time T1 when sending out the 1st UL packet (or the 1st UL packet in a PDU set) matching the UL flow information to the SEALDD server. The T1 is sent in the encapsulated UL monitoring packet to the SEALDD server. 8. The SEALDD server receives the UL monitoring packet, and records the local time T2. 9. Similarly, the SEALDD server encapsulates the DL monitoring packet (i.e. DL SEALDD packet with SEALDD DL monitoring header and VAL traffic as payload, or dummy UL SEALDD packet generated for data transmission quality monitoring in case there is no DL VAL traffic for DL packet delay monitoring) with local time T2 recorded in step 8 and local time T3 when the SEALDD server sends out the DL monitoring packet. NOTE 2: When the SEALDD server sends the dummy UL packet as monitoring response to the SEALDD client depends on SEALDD server implementation. For crossflow RTT measurement, when the 1st DL packet (or the last DL packet in a PDU set) matching the received DL flow information is to be sent, the SEALDD server encapsulates the DL monitoring packet with previously received T1 and sends the DL monitoring packet to the SEALDD client. 10. The SEALDD client records the local time T4 when the SEALDD client receives the DL monitoring packet and calculates the latency with T1, T2, T3, T4. The SEALDD client can also calculate the bitrate and jitter over a certain period over a specific SEALDD connection by recording the status of the SEALDD monitoring packets. The SEALDD client also evaluates the reporting criteria if present in the SEALDD transmission quality measurement subscription request in order to generate the transmission quality measurement report. For crossflow RTT measurement, if T1 is received in the DL packet, the SEALDD client records local time T2 and calculates RTT based on T1 and T2. Depending on which entity triggers the data transmission quality measurement, step 11 and step 12 corresponds to step 2 to step 5, step 13 corresponds to step 6. 11-12. The SEALDD client reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, crossflow QoS measurements) to the VAL server via the SEALDD server. 13. The SEALDD client reports the data transmission quality measurement results to the VAL client. NOTE 3: The crossflow RTT measurement calculated by the SEALDD client in step 10 can be used as an estimation of the lower bound for the Motion-to-Photon latency in the XR scenarios. When a VAL group ID or a list of VAL UE IDs or all VAL UEs indication is received in step 2, step 4 to step 11 is repeated for VAL UEs in the group/list or for all VAL UEs. The SEALDD server maps the VAL UE group ID to a list of VAL UE IDs if a VAL group ID is received. The SEALDD server identifies SEALDD connections corresponding to the desired VAL UE(s) to trigger measurement. And depending on the reporting requirement for multiple UEs, the SEALDD server collects and aggregates the needed report for the VAL server. 9.7.3.1 SEALDD enabled data transmission quality measurement subscription request Table 9.7.3.1-1 describes the information flow from the VAL server to the SEALDD server for subscribing to the data transmission measurement service. Table 9.7.3.1-1: SEALDD transmission quality measurement subscription request Information element; Status; Description. Application traffic identifiers; M; Identify of the application traffic (e.g. VAL server ID, VAL service ID). VAL UE identity; O (See NOTE); Identifier of the VAL UE for which measurements need to be provided, e.g. UE ID. VAL UE group ID; O (See NOTE); Identifier of a specific VAL UE group, as defined in clause 7.5 of 3GPP TS 23.434 [4].. VAL UE identity list; O (See NOTE); Identifies a list of VAL UEs, e.g. the list of UE ID.. All VAL UEs Indication; O (See NOTE); Indicates all VAL UEs of the application identified by application traffic identifiers.. Flow(s)traffic descriptor(s); O (See NOTE); Indicates the flow(s) traffic descriptor(s) (Address/port or URL) for the measurement.. Crossflow measurement information; O (see NOTE); Represents the crossflow measurement information, e.g., list of pairs traffic descriptor with traffic direction (UL or DL). Example of the crossflow measurement information: {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]}.. Measurement conditions; O; Indicates the temporal and/or spatial conditions.. Transmission quality measurement requirements list; M; The measurement requirement information. > Measurement ID; M; Measurement identifiers, e.g. UL/DL/E2E latency, UL/DL bitrate, UL/DL/E2E packet loss rate, UL/DL/E2E jitter, and crossflows identifiers (e.g., crossflows latency, crossflows bitrate, crossflows packet loss rate, crossflows jitter).. > Reporting frequency; O; The reporting frequency of measurement results (e.g. periodic reporting). If not present, it implies periodic reporting.. > Reporting periodicity; O; If the reporting frequency is periodic, the reporting periodicity shall be provided. For multiple UEs, it is recommended to give sufficient time to allow report aggregation.. > Reporting granularity; O; The reporting granularity indicates whether the measurement report is for individual VAL UE or for VAL UE group or for all VAL UEs, if VAL UE group or all VAL UEs is the measurement target.. > Measurement period window; O; Indicates the measurement period window for transmission quality measurements. > Measurement expiration time; O; Indicates the measurement expiration time. > Reporting criteria; O; Indicates the criteria for reporting measurement results, e.g. if the latency or bitrate reaches below or above a certain value. It also includes a unique identifier for each criteria of more than one criteria is specified.. NOTE: One of them shall be present as the measurement target UE or traffic descriptor.. 9.7.3.3 SEALDD enabled data transmission quality measurement notification Table 9.7.3.3-1 describes the information flow from the SEALDD server to the VAL server for notifying the transmission quality measurement reports. Table 9.7.3.3-1: SEALDD transmission quality measurement notification Information element; Status; Description. Subscription ID; M; Subscription identifier corresponding to the subscription.. Transmission quality measurement reports list; M; The generated transmission quality results in SEALDD server, as specified in Table 9.7.3.3-2.. Table 9.7.3.3-2 describes the information elements for the transmission quality measurement reports list, provided by the SEALDD server after performing transmission quality measurement. Table 9.7.3.3-2: SEALDD transmission quality measurement reports list Information element; Status; Description. > Measurement ID; M; Measurement identifiers (e.g. UL/DL/E2E latency, UL/DL bitrate, UL/DL/E2E packet loss rate, UL/DL/E2E jitter) and crossflows identifiers (e.g., crossflows latency, crossflows bitrate, crossflows packet loss rate, crossflows jitter). > VAL UE ID(s) or flow(s)traffic descriptor(s); O (see NOTE); It indicates the VAL UE(s) under SEALDD measurement. For a single VAL UE or flow(s)traffic descriptor(s), it can be omitted and the associated measurement values are for the single VAL UE. For multiple VAL UEs or flow(s)traffic descriptor(s) with reporting granularity set to individual UE or flow traffic descriptor, the associated measurement values are for individual VAL UE or flow traffic descriptor as indicated in this IE. For multiple VAL UEs with reporting granularity set to VAL UE group/list or all VAL UEs, the associated measurement values are aggregation for all VAL UEs or the VAL UE group/list and this IE includes the measured VAL UEs.. > Crossflow measurement information; O (see NOTE); Represents the crossflow measurement information, e.g., list of pairs traffic descriptor with traffic direction (UL or DL). Example of the crossflow measurement information: {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]}.. > Average measurement value; O; The average measurement value of measurement results. > Minimum measurement value; O; The minimum measurement value of measurement results. > maximum measurement value; O; The maximum measurement value of measurement results. > Standard deviation measurement value; O; Standard deviation measurement value of measurement results. > k-Percentile measurement value; O; Indicates the k-percentile measurement value of measurement results. > Measurement period; O; Indicates the measurement period. > Timestamp; O; Indicates the timestamp of measurement results. NOTE: These IEs are mutually exclusive.. 9.7.3.4 SEALDD enabled data transmission quality query request Table 9.7.3.4-1 describes the information flow from the other consumers (e.g. SEALDD server, NSCE server, ADAE server) to the SEALDD server for querying the data transmission quality measurement result. Table 9.7.3.4-1: SEALDD transmission quality query request Information element; Status; Description. Application traffic identifiers; M; Identify of the application traffic (e.g. VAL server ID, VAL service ID). VAL UE ID(s) or flow(s) traffic descriptor(s); O (see NOTE); Identifier of VAL UE(s) or flow (s) traffic descriptor(s) need to be queried, e.g. single VAL UE, multiple VAL UEs, flow(s) traffic descriptor(s), or VAL UE group. Crossflow measurement information; O (see NOTE); Represents the crossflow measurement information, e.g., list of pairs traffic descriptor and traffic direction (UL or DL). Example of the crossflow measurement information: {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]}.. NOTE: These IEs are mutually exclusive..

7.4.3	Solution evaluation

This solution addresses KI#1 to support:

-	what KPI the application enabling layer could provide that impact the QoS of the XR services.

-	how the KPI could be measured at the application enabling layer, and the enhancement of the SEALDD services.

This solution also addresses KI#8 to support:

-	QoS measurement for PDU set on crossflow RTT.

This solution addresses KI#5 to support:

-	Whether and how could the application enabling layer support the acquisition of tethering link status via measurement and analytic and provide a transporting optimization, considering different type of the tethered device (standalong tethered device, or wire/wireless tethered device).

7.5	Solution #5: Multi-flow synchronization for multi-modal XR application

7.5.1	Architecture Impacts

This solution is presented based on the existing SEALDD architecture in 3GPP TS 23.433 [6].

7.5.2	Solution description

7.5.2.1	General

This solution addresses key issue #2 on E2E multi-modal communication flow, to solve how the SEALDD layer to manage the multiple association flows for multi-modal XR service to satisfy the multi-flow synchronization.

By performing the transmission quality measurements for multi-modal application, the SEALDD server can determine the service flow that needs to be adjusted and the corresponding required QoS information based on the transmission quality result and the synchronization threshold.

7.5.2.2	Procedure

The following figure provides a high-level procedure that the SEALDD server determines which flow(s) is needed to be adjusted for multi-modal application, and interacts with 5GC with the determined QoS information to adjust the transmission quality to satisfy multi-flow synchronization.

Pre-conditions:

1.	The VAL server discovers and selects the SEALDD server by CAPIF functions.

2.	The SEALDD server has been provisioned to multi-modal XR policy including the synchronization threshold for multi-modal application, as specified in 3GPP TS 22.261 [2].

Figure 7.5.2.2-1: SEALDD enabled multi-flow synchronization procedure

1.	The VAL server sends a SEALDD Transmission request for multi-modal XR to the SEALDD server. The request includes the identifier of the application traffic (e.g. VAL service ID, VAL server ID), the SEALDD-S connection information for multiple flows from VAL server side (e.g. address/port for audio flow, address/port for tactile flow, etc).

2.	The SEALDD server determines to enable data transmission and synchronization for the multi-modal application (e.g. VAL service ID, VAL server ID). The SEALDD server allocates the SEALDD-S connection information for multiple flows from SEALDD server side, and maintains the multi-flow association information for SEALDD-S interface (i.e., SEALDD-S connection information from VAL server side, and/or SEALDD server side), and sends a response to the VAL server.

NOTE 1:	The detailed message/API name for SEALDD transmission request for multi-modal XR will be determined in the normative phase.

3.	The VAL client sends a SEALDD service request to SEALDD client.

4.	The VAL/SEALDD client discovers and selects the proper SEALDD server for the VAL application.

5.	The SEALDD client sends SEALDD Transmission Connection_Establish request for multi-modal XR to SEALDD server with the SEALDD client ID, VAL server ID, VAL service ID, and the SEALDD traffic descriptor of the SEALDD client side.

6.	According to the determination in step 2, if the data transmission and synchronization is enabled for the specified application identifier (e.g. VAL service ID, VAL server ID), the SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-Uu flow) to the SEALDD client. The SEALDD server maintains the multi-flow association information for SEALDD-Uu interface (i.e., SEALDD traffic descriptor from SEALDD client side, and/or SEALDD server side).

7.	The SEALDD server performs data transmission quality measurement in SEALDD-Uu interface based on the mapping information for multiple flow association information between SEALDD-S interface and SEALDD-Uu interface. Upon receiving the packets from multiple associated flows in SEALDD-S interface as descripted in step 2, the SEALDD server performs the packet encapsulation with sending timestamp information in the corresponding SEALDD-Uu interface, and can calculate the transmission delay measurement result of multiple associated flows after obtaining the receiving timestamp from the SEALDD client.

8.	Based on the calculated transmission delay for multiple associated flow over SEALDD-Uu interface in step 7, and the synchronization threshold for multi-modal application as described in pre-condition, the SEALDD server can determine the service flow(s) (i.e. address/port for SEALDD-Uu flow) that needs to be adjusted among the multiple associated flows in SEALDD-Uu interface, and the corresponding required QoS information (i.e. transmission delay).

9.	The SEALDD server sends the AF request to 5GC via N33/N5 with the SEALDD traffic descriptor of the adjusted flow(s) (i.e. address/port for the adjusted SEALDD-Uu flow) and the corresponding required QoS information determined in step 8, by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of 3GPP TS 23.502 [4]. The SEALDD traffic descriptor of the adjusted flow(s) contains the address or port in SEALDD server side, and/or SEALDD client side.

NOTE 2:	This procedure is applicable for both downlink and uplink synchronization of multi-modal flow. For downlink and/or uplink, the step 8 is determined according to the measured downlink and/or uplink transmission delay in step 7.

After requesting the transmission quality optimization on 5G network with the required QoS for the adjusted flow(s), the multi-flow synchronization of multi-modal application can be satisfied.

7.5.3	Solution evaluation

This solution focuses on the muti-flow synchronization for multi-modal XR application, where the SEALDD layer is enhanced to establish the SEALDD connection, and further determine the required QoS information for multi-modal flow(s), it can solve the three open issues in key issue #2.

7.6	Solution #6: Multi-modal flows alignment and monitoring

7.6.1	Architecture Impacts

This solution requires a mechanism for data storage to cache the traffic flows.

7.6.2	Solution description

7.6.2.1	General

Synchronization between different multi-modal XR application components is crucial. In other words, the delay difference among the associated flows should be small. Otherwise, the arrived flows need to wait for other associated flows that do not arrived yet.

To avoid the downlink traffic flow delay difference introduced from (R)AN and Internet, the SEALDD client could do the multi-modal traffic flow alignment, by steering associated multi-modal traffic flows to be transferred to application client at specific time.

As shown in figure 7.6.2.1-1, flow#1 and flow#2 are associated flows. They are sent out from VAL server at the same time, while flow#1 arrives at SEALDD client earlier than flow#2 due to different routing path. To align those associated flows, the SEALDD client holds and buffers the flow#1 for a while. Until all the associated flows arrived(i.e., flow#2), the SEALDD client sends out those associated flows to application client at the same time.

Figure 7.6.2.1-1: DL traffic flow alignment

7.6.2.2	Procedure of flow alignment

Figure 7.6.2.2-1illustrates the procedures of multi-modal flows alignment.

Precondition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

-	The connection has been established among the UE, SEALDD server and VAL server.

-	The SEALDD server has been provisioned to multi-modal XR policy including the synchronization threshold for multi-modal application, as specified in 3GPP TS 22.261 [2].

SEALDD client

5GC

SEALDD server

1. Multi-modal flows coordination request

VAL server

5. Flows coordination

4. Multi-modal flows coordination response

Connection established

2. Multi-modal flows coordination request

3. Multi-modal flows coordination response

Application client

SEALDD client

5GC

SEALDD server

1. Multi-modal flows coordination request

VAL server

5. Flows coordination

4. Multi-modal flows coordination response

Connection established

2. Multi-modal flows coordination request

3. Multi-modal flows coordination response

Application client

SEALDD client

5GC

SEALDD server

1. Multi-modal flows coordination request

VAL server

5. Flows coordination

4. Multi-modal flows coordination response

Connection established

2. Multi-modal flows coordination request

3. Multi-modal flows coordination response

Application client

Figure 7.6.2.2-1: Multi-modal flows alignment

1.	VAL server sends the multi-modal flows alignment request with VAL service ID, UE ID(s), flows descriptor,   Multi-modal Service ID, flows transmission requirement. Flows transmission requirement including the delay requirement, number of associated flows, maximum acceptable duration for traffic flow alignment. Maximum acceptable time duration for traffic flow alignment is used to limit the maximum waiting time for the associated flow.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends the multi-modal flows alignment request to the SEALDD client identified by the UE ID(s), along with the information received in step1, and the flows alignment assistance information (e.g. timestamp in the RTP header, RTCP).

If the maximum acceptable duration for traffic flow alignment is not provided, then the SEALDD server would determine the maximum acceptable duration for traffic flow alignment based on VAL service ID and/or flows transmission requirement. The server may translate the flows descriptor into SEALDD flow ID.

NOTE 1:	The flow alignment assistance information can be obtained by SEALDD server based on SEALDD policy.

NOTE 2: The SEALDD client performs the caching and transmission to align multi-modal flows based on the flow alignments assistance information and maximum acceptable time duration.

Editor's note: Whether and how to make use of existing CN functionality (e.g. PDU set related feature) is FFS.

NOTE 3: The NTP timestamp and RTP timestamp in RTCP sender report (SR) can be used to to identify the associated packet among muti-modal flow, and further be used to perform alignment in SEALDD client.

3.	The SEALDD client sends the multi-modal flows alignment response to the SEALDD server.

4.	The SEALDD server sends the multi-modal flows alignment response to the VAL server indicating the result of multi-modal flows alignment.

5.	When the traffic flow arrived, the SEALDD client initiate the multi-modal flows alignment based on the received information in step 1. The flows need to be aligned is detected by the VAL service ID, flows descriptor, and flow alignment assistance information.

If number of associated flows is provided, the number of associated flows is used to determine whether all the associated flows arrived. If the flow alignment assistance information is provided, the SEALDD client can identify the associated packets (e.g. with the same RTP timestamp) in the multi-modal flows. After all associated packet in multi-modal flows arrived, the SEALDD client sends the associated packets to the application client. If the maximum acceptable time duration is provided, once the maximum acceptable time is reached, the SEALDD client will no longer waits for the associated packets in multi-modal flows even if the associated packet is not arrived yet. The maximum acceptable time may be determined by the SEALDD server based on the transmission quality result for multi-modal flow and the synchronization threshold.

NOTE 4:	Step3 and step5 can be executed in parallel.

7.6.2.3	Procedure of flow alignment monitoring

Figure 7.6.2.3-1illustrates the procedure of multi-modal flows alignment monitoring.

Precondition:

-	The multi-modal flows alignment has been requested and triggered.

SEALDD client

5GC

SEALDD server

1. Multi-modal flows coordination monitoring request

VAL server

5. Flows coordination monitoring

7. Multi-modal flows coordination monitoring notification

Multi-modal flows alignment

2. Multi-modal flows coordination monitoring request

4. Multi-modal flows coordination monitoring response

6. Multi-modal flows coordination monitoring notification

3. Multi-modal flows coordination monitoring response

Figure 7.6.2.3-1: Traffic flow alignment monitoring

1.	VAL server sends the multi-modal flows alignment monitoring request. VAL service ID,UE ID, flows description, monitoring period are provided in the request.

2.	Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends the multi-modal flows alignment monitoring request to the SEALDD client.

3.	The SEALDD client sends the multi-modal flows alignment monitoring response to the SEALDD server.

4.	The SEALDD server sends the multi-modal flows alignment monitoring response to the VAL server, indicating whether the multi-modal flows alignment monitoring request is successful.

5.	The SEALDD client calculates the end-to-end delay difference based on the arrival time and timestamp(e.g., timestamp in the RTP header) among the flows.

Based on the flows description, delay difference for traffic flows identified needs to be calculated. The arrival time of first and last detected flow, T1 and T2, are recorded, and the delay difference is calculated based on T1 and T2.

6-7.SEALDD client sends the multi-modal flows alignment monitoring result to the SEALDD server, then to the VAL server.

7.6.3	Solution evaluation

This solution addresses Key issue #1and key issue #2: KPI measurement and exposure, and multi-modal communication. It proposes multi-modal flows alignment and monitoring to optimize the delay difference of multi-modal flows.

For multi-modal flows alignment, VAL server sends multi-modal flows alignment request to SEALDD client via SEALDD server, including VAL service ID, UE ID(s), flows description, flows transmission requirement and flows alignment assistance information. The SEALDD client utilizes these information to identifies multi-modal flows and perform the alignment.

For multi-modal flows monitoring, VAL server sends multi-modal flows alignment monitoring request to SEALDD client via SEALDD server, including VAL service ID, UE ID, flows description and monitoring period. The SEALDD client utilizes these information to identifies multi-modal flows and perform the monitoring.

7.7	Solution #7: Support multi-modal service in SEALDD

7.7.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [6].

7.7.2	Solution description

7.7.2.1	General

This solution addresses the improvement of the SEALDD layer to support Multi-Modal service.

In order to simplify VAL handling, the SEALDD layer can perform packetilization. Figure 7.7.2.1-2 illustrates packetilization in SEALDD layer and information flows:

Figure 7.7.2.1-2: SEALDD packetilization for VAL media data

The VAL can send media data (e.g. H.264 raw data in the form of [NALU][NALU]…[NALU]) to the SEALDD layer. After SEALDD packetilization using RTP (e.g. for H.264 as in IETF RFC 6184 [14] or H.265 as in IETF RFC 7798 [15]), NAL units can be segmented (if size is too large for line transmission) or aggregated (for small NAL units) by SEALDD, SEALDD also updates NALU header. Between the SEALDD client and SEALDD server, the SEALDD payload transferred via SEALDD connection (e.g. SEALDD/UDP/IP) includes RTP/RTCP/RTSP data.

NOTE:	The media data between SEALDD server and VAL server can be transferred via websocket which is implementation specific.

7.7.2.2	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.433 [6] can be enhanced (highlighted in bold italics) as follows.

NOTE:	SEALDD data transmission procedures with single SEALDD connection are described as typical example below.

9.2.2.2 SEALDD enabled regular data transmission connection establishment procedure Figure 9.2.2.2-1 illustrate the procedure for establishing regular SEALDD data transmission connection. Pre-condition: - The VAL server can discover and select the SEALDD server by CAPIF functions. Figure 9.2.2.2-1: SEALDD enabled regular data transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for application traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_RegularTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements. 2. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, SEALDD server allocates address/port of the SEALDD server to receive the packets from the VAL server for application data transfer as SEALDD-S data transmission connection information of the SEALDD server side. The SEALDD server allocates a specific address or port used for SEALDD traffic transfer with the specific UE for the VAL server and responds with a SEALDD service response (including SEALDD-S data transmission connection information of the SEALDD server side). The VAL server and SEALDD server can use SEALDD-S data transmission connection information to establish the data transmission connection between VAL server and SEALDD server for application data transfer. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5, as defined in clause 5.2.6.9 and in clause 5.2.5.3 of 3GPP TS 23.502 [6]. The AF request includes the application traffic descriptor containing the address or ports allocated by SEALDD server, and the QoS information for application traffic. The QoS information may be determined by SEALDD server according to VAL service ID for different service type of application traffic if the QoS information is not provided by VAL server. The SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service APIs and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QoS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4]. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. NOTE 1: The SEALDD-S data transmission connection information of the SEALDD server side is optional to respond to the VAL server, if the SEALDD server uses the downlink pull mode to obtain the data/content from the address provided by the VAL server in step 1, and uses the uplink push mode to send the data/content to the address provided by VAL server. 3. Data transmission session information is provisioned to the VAL client by the VAL server via application signalling. NOTE 2: The application signalling may be transmitted via direct application layer connection or via the SEALDD layer. 4. The VAL client sends a SEALDD service request to SEALDD client. The VAL client receives a SEALDD service response to the SEALDD client. The response indicates that whether the SEALDD service request is successful or not. 5. The VAL/SEALDD client discover and select the proper SEALDD server for the VAL application, as described in clause 9.4.3. After this step, the VAL server is discovered and selected along with the associated SEALDD server, the SEALDD client can get the SEALDD server's address. 6. The SEALDD client allocates a SEALDD flow ID mapping to the identifiers of the application traffic. The SEALDD client sends Sdd_RegularTransmissionConnection_Establish request to SEALDD server with the SEALDD client ID, the SEALDD flow ID, the SEALDD traffic descriptor of the SEALDD client side (the address/port of the SEALDD client for receiving the downlink SEALDD traffic), VAL server ID, VAL service ID. The request message also contains the selected VAL server endpoint information and UEID. The SEALDD server retrieves the location information of the VAL UE or SEALDD client from SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12 and verifies with the Geofence policy configured by the VAL server to allow or restrict the data connection establishment. If the location information is allowed as per the configured geofence policy then the SEALDD server allows the data connection establishment, otherwise SEALDD server returns a failed result e.g. performs connection reject. NOTE 3: The SEALDD server can use or update the association between SEALDD-UU connection and SEALDD-S connection that associated with UE ID, VAL service ID, VAL server endpoint, which is used to correlate the SEALDD traffic and the VAL application traffic. NOTE 4: The SEALDD flow ID is used by the SEALDD client and SEALDD server to identify different VAL application traffic of the same SEALDD client. The SEALDD flow ID may be same with the identifiers of the application traffic or new simplified IDs allocated by SEALDD. 7. The SEALDD server responds to the SEALDD client with the SEALDD traffic descriptor of SEALDD server side (e.g. address/port allocated in step 2, transport layer protocol) mapping to the application traffic. The SEALDD server may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the SEALDD regular transmission connection establishment response. 8. If the connection between VAL server and SEALDD server is not established in step 2, the SEALDD server establishes connection with VAL server for the VAL client to transmit application traffic mapping to the SEALDD traffic according to the SEALDD-S information negotiated in step 1-2. 9. The SEALDD client uses the SEALDD traffic descriptor of SEALDD server side for SEALDD connection establishment. After this step, the SEALDD client and SEALDD server both get the whole SEALDD traffic descriptor (including the UE's address/port and SEALDD server's address/port for the SEALDD traffic transmission). After the negotiation and establishment of the connections, the SEALDD client gets the mapping information between application traffic and SEALDD flow ID. The SEALDD server gets the mapping information between the SEALDD flow ID and the SEALDD-S connection. Upon receiving application traffic from VAL client, the SEALDD maps it to SEALDD traffic with SEALDD traffic descriptors as negotiated with SEALDD server in step 6 and step 7. The SEALDD traffic is sent to the SEALDD server. The SEALDD server maps the SEALDD traffic to the application traffic according to the stored SEALDD traffic descriptor, SEALDD client ID and SEALDD flow ID. The SEALDD server sends the recovered application traffic to the address provided by VAL server in step 1, via the connection established in step 2 or 8 according to the mapping information. The downlink application traffic sent from VAL server to VAL client is processed similarly. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP). The SEALDD server receives any UE location change notification using SEAL LM services defined in 3GPP TS 23.434[4] clause 9.3.12, then the SEALDD server performs the data delivery in alignment with the geofence policy. If the UE is in the forbidden location or not allowed for the given VAL service to send/receive data as per the Geofence policy, then the SEALDD server performs action like releases the connection and informs the VAL server that UE is not reachable because in a forbidden location using connection event status procedure. If UE enters the allowed location area then the SEALDD server initiates the connection establishment using the procedure defined in clause 9.2.2.3. 9.2.2.3 SEALDD enabled regular data transmission connection establishment based on policy The SEALDD servers has Data Delivery (DD) policy being provisioned. Before the application communication between VAL client and VAL server starts, the DD policy is enforced by the SEALDD server to establish the SEALDD connection. Pre-conditions: 1. The SEALDD server has DD policies available. Figure 9.2.2.3-1: Policy enforced by SEALDD server for connectivity 1. The VAL server subscribes to SEALDD event exposure for connection status using the procedure defined in clause 9.2.2.6. 2. When the time for data transmission is about to start, the SEALDD server enforces the policy to trigger regular data transmission connection establishment. If spatial condition for UE is provided, the SEALDD server also ensures the UE’s location requirement is satisfied when establishing regular data transmission connection (e.g. by using NEF service for monitoring UE location or SEAL location service for UE entering area of interest). 3. If there is a special routing requirement for SEALDD user plane traffic (e.g. running on a specific slice and DNN), the SEALDD server interacts with 3GPP CN to provision service specific parameters with NEF as described in 3GPP TS 23.502 [6], clause 4.15.6.10 and clause 4.15.6.7. If there are QoS requirements in the DD policy, the SEALDD server also applies QoS to ensure the quality for SEALDD traffic by utilizing NEF/PCF/NRM/EES service for QoS adjustment. Specifically, the SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service API and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QoS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4]. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. If the DD policy specifies failure detection report, the SEALDD server may subscribe to CN analytics (e.g. DN performance analytics) from NEF/NWDAF and further notify data delivery status of application traffic to VAL client (via SEALDD client) and VAL server based on analytics result. 4. The SEALDD server allocates an IP address and port for sending and receiving packet over SEAL-S reference point, then SEALDD server sends SEALDD connection establishment notification to the VAL server with VAL service ID, the IP address and port. 5-6. The SEALDD server allocates an IP address and port for sending and receiving packet over SEAL-Uu reference point, then SEALDD server sends regular data transmission connection establishment request to the SEALDD client with SEALDD flow ID, VAL service ID, the IP address and port. The SEALDD server may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the SEALDD regular transmission connection establishment request. The request is responded by the SEALDD client. UE IP address (and port) may be included by the SEALDD client in the response or sent in a separate update message by SEALDD client if a different UE IP address is to be used in SEALDD connection user plane. NOTE 1: Step 4 and step 5 can be done in parallel. NOTE 2: Step 5 can be sent via PDU session (if exist) or via application triggering (if no PDU session exists). 7. The SEALDD client further notifies the VAL client about the SEALDD connection being established. Upon receiving application traffic from VAL client (not shown in the figure), the SEALDD client sends it to SEALDD server in SEALDD traffic. The SEALDD server identifies application traffic based on the VAL service ID and further sends the application traffic to VAL server. The downlink application traffic sent from VAL server to VAL client is processed similarly. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP). 9.2.3.1 SEALDD enabled regular transmission request Table 9.2.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the regular application transmission service. Table 9.2.3.1-1: SEALDD enabled Regular transmission request Information element; Status; Description. VAL server ID; M; Identity of the VAL server. VAL service ID; O; Identity of the VAL service. UE identity; O; Identifier of specific UE, e.g. UE ID. SEALDD-S Data transmission connection information; M; Address/port and/or URL of the VAL server to receive the application packets from the SEALDD server. QoS information; O; QoS information provided by VAL server. VAL server’s total bandwidth limit; O (See NOTE); The total bandwidth limit of VAL server, including UL/DL. VAL users’ bandwidth limit; O (See NOTE); The bandwidth limits (i.e. minimum bandwidth requirement and maximum bandwidth limit) for VAL users, including UL/DL. Protocol description; O; The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP.. NOTE: These IEs are used for the SEALDD enabled bandwidth control for different VAL users.. 9.2.3.7 SEALDD connection status subscription request Table 9.2.3.7-1 describes the information flow from the VAL server to SEALDD server to subscribe to SEALDD connection status information. Table 9.2.3.7-1: SEALDD connection status subscription request Information element; Status; Description. VAL server ID; M; Identity of the VAL server. Event ID list; M; Identifies a list of events such as establishment, release, congestion report.. VAL service ID; O; Identity of the VAL service. Identity; O; Identifier of VAL UE or VAL user.. SEALDD-S Data transmission connection information; M; Address/port of the VAL server to send/receive the application packets to/from the SEALDD server.. Immediate reporting flag; O; Indicates the immediate reporting of connection status notification. SEALDD client connection status check periodicity; O; Indicates the frequency to perform SEALDD client connection status check. Protocol description; O; The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP.. 9.2.3.3 SEALDD regular transmission connection establishment request Table 9.2.3.3-1 describes the information flow from the SEALDD client to the SEALDD server or from the SEALDD server to the SEALDD client for requesting the regular SEALDD connection establishment. Table 9.2.3.3-1: SEALDD regular transmission connection establishment request Information element; Status; Description. Requestor ID; M; Identity of the requestor (SEALDD client or SEALDD server).. SEALDD flow ID; M (See NOTE 1); Identity of the SEALDD flow.. VAL server ID; O; Identity of the VAL server, applicable for SEALDD client side initiated request.. VAL service ID; O; Identity of the VAL service. Selected VAL server endpoint; M; Endpoint of the selected VAL server. SEALDD traffic descriptor; O; SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD client side (for client side initiated request) or the SEALDD server side (for server side initiated request) used to establish SEALDD connection.. VAL UE/user identity; O; The VAL user ID of the VAL user or VAL UE ID.. SEALDD communication lifetime; O; Identifies the DD communication lifetime, applicable for SEALDD server side initiated request.. Capability for BAT and periodicity adaptation; O (See NOTE 2); Indicates BAT and periodicity adaptation capability for SEALDD client in client side initiated request.. Transmission assistance info; O (See NOTE 2); Indicates transmission assistance information for uplink SEALDD traffic in client side initiated request. It includes BAT, BAT window, periodicity, and periodicity range.. L4S feedback capability; O (See NOTE 3); Identifies the L4S feedback capability (i.e. ECN identification, L4S feedback) for client side initiated request. Protocol description; O; The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP.. NOTE 1: The SEALDD flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. NOTE 2: Only one of these IEs may be present in the message. NOTE 3: This IE is used for the SEALDD enabled congestion control for VAL applications, as specified in clause 9.8.2.2.. 9.2.3.4 SEALDD regular transmission connection establishment response Table 9.2.3.4-1 describes the information flow from the SEALDD server to the SEALDD client or from the SEALDD client to the SEALDD server for responding to the regular SEALDD connection establishment. Table 9.2.3.4-1: SEALDD regular transmission connection establishment response Information element; Status; Description. Result; M; Indicates the success or failure of establishing the SEALDD connection.. SEALDD traffic descriptor; O; SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD server side (for client side initiated request) or the SEALDD client side (for server side initiated request) used to establish SEALDD connection.. Pending timer; O (See NOTE 1); The pending timer to trigger the re-connection from SEALDD client when bandwidth limit check is failed.. Suggested traffic transmission bandwidth; O (See NOTE 1); The suggested traffic transmission bandwidth used by SEALDD client or SEALDD server to perform bandwidth control for VAL users, including UL/DL.. Cause; O (See NOTE 2); Indicates the reason for the failure, e.g. SEALDD policy mismatch.. Capability for BAT and periodicity adaptation; O (See NOTE 3); Indicates BAT and periodicity adaptation capability for SEALDD client in server side initiated request.. Capability for BAT and periodicity adaptation; O (See NOTE 3); Indicates BAT and periodicity adaptation capability for SEALDD client in server side initiated request.. Protocol description; O; The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP.. NOTE 1: These IEs are used for the SEALDD enabled bandwidth control for different VAL users, applicable for client side initiated request. NOTE 2: This IE is only present if the Result is failure NOTE 3: Only one of these IEs may be present in the message..

7.7.3	Solution evaluation

This solution addresses KI#8 to support:

-	using SEALDD to facilitate XR traffic between application clients and application servers regarding XR traffic transmission with PDU set inclusion.

7.8	Solution #8: EAS instantiation enhancement to satisfy E2E KPI requirements for XR application

7.8.1	Architecture Impacts

This solution is presented based on the existing EDGEAPP architecture in 3GPP TS 23.558 [11].

7.8.2	Solution description

7.8.2.1	General

This solution proposes that the enablement layer (i.e. EES) can determine the required computing ability requirements of EAS instantiation and/or the required QoS information. By interacting with management system and 5GC, the E2E KPI application requirements of XR application can be guaranteed.

In this solution, the EES gives the initial determination about required computing ability and/or the required QoS to satisfy the AC service KPI requirements (i.e. the E2E KPI requirements for XR application). The EES may update the computing ability and/or the QoS information after step 6 for further EAS instantiation, such as according to the real-time monitoring/report about computing quality and transmission quality for the instantiated EAS.

NOTE 1:	The E2E KPI requirements mentioned in this paper, indicates the latency requirements of XR application/AC (i.e. the response time in AC service KPI).

7.8.2.2	Procedure

The following figure provides the procedure of EAS instantiation enhancement to satisfy E2E KPI requirement (e.g. low latency) for XR application. The EES determines the required computing ability and/or the required QoS information based on the AC service KPI, the available computing capability (i.e., maximum computing capability) and the available transmission capability for the requested EAS.

Pre-conditions:

1.	EAS registers to EES according to EAS Registration procedure in clause 8.4.3 of 3GPP TS 23.558 [11].

2.	EES registers to ECS according to EES Registration procedure in clause 8.4.4 of 3GPP TS 23.558 [11].

3.	The ECSP management system provides the information about instantiable EASs (e.g. instantiable EAS ID, the available computing capability) to the EES.

Figure 7.8.2.2-1: EAS instantiation enhancement procedure

1.	The EEC performs service provisioning procedure, as described in clause 8.3.3 of 3GPP TS 23.558 [11].

2.	The EEC sends the EAS discovery request to the EES, the request includes the AC profile with the required EAS ID, and the AC service KPI (i.e. the E2E KPI requirements for XR application).

3.	The EAS discovery request triggers the EES to verify whether the instantiation of the EAS indicated in the request is required, as specified in clause 8.12.1 of 3GPP TS 23.558 [11]. If EAS instantiation is required, the EES can determine the required computing ability based on the AC service KPI, the available computing capability and the available transmission capability for the requested EAS.

NOTE 2:	The available computing capability (i.e., maximum computing capability) is provided to EES by the ECSP management system as described in pre-condition, which can be used to as the maximum requested computing ability.

NOTE 3:	The EES can obtain the available transmission capability from NWDAF, by utilizing the DN performance analytic in clause 6.14 of 3GPP TS 23.288 [16], which can be used to as the maximum requested QoS information (i.e. transmission quality).

4.	If the EES determines that EAS instantiation is needed, the EES can request the ECSP management system to perform the EAS instantiation with the required EAS ID, and the EAS requirements including the required computing ability determined in step 3, as specified in clause 7.1.2.1 of 3GPP TS 28.538 [17]. Upon reception of the EAS instantiation request, the ECSP management system verifies EAS instantiation requirements and performs EAS instantiation as specified in 3GPP TS 28.538 [17]. After the EAS instantiation is completed, the EES can be informed by ECSP management system with the result of EAS instantiation including the EAS endpoint information.

NOTE 4:	In step4, the EES sends a report for a need of the EAS instantiation to the ECSP management system to consider instantiating the requested EAS by invoking an MnS API of the ECSP management system as described in clause 8.12 of 3GPP TS 23.558 [11]. The ECSP management system has the final decision about EAS instantiation.

5.	After the EAS instantiation is completed, the EES will receive the EAS profile information via the EAS registration procedure.

6.	The EES can send the AF request to 5GC via N33/N5 with the application descriptor (i.e. the EAS endpoint information after instantiated by ECSP management system in step 4 or step 5), and the required QoS information determined in step 3, by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of 3GPP TS 23.502 [4].

NOTE 5:	In step 6, the EES may update the required QoS information based on receiving the EAS profile about the detailed EAS service KPI, if step 5 happens.

7.	The EES sends the EAS discovery response to EEC with the EAS endpoint information.

NOTE 6:	If the EAS instantiation is failed in step 4, the step 5 can be skipped, and the EES responds in EAS discovery response with an appropriate failure cause. The XR application client/AC can access the instantiated EAS with the required computing ability and the required QoS information, the E2E KPI application requirements of XR application can be guaranteed.

7.8.3	Solution evaluation

This solution addresses the all open issues in key issue #7, the EDGEAPP as enablement layer is enhanced for EAS instantiation to optimize the E2E KPI performance for XR service. The EES triggers the ECSP management system to instantiate the EAS by using the MnS API as in clause 8.12 of 3GPP TS 23.558. This solution will be revisited if there are some legacy issues or update about MnS API usage in 3GPP TS 23.558.

7.9	Solution #9: Support performance analytics for tethered UEs

7.9.1	Architecture Impacts

This solution requires the following architecture enhancements:

-	ADAES needs to introduce a new service for collecting measurements, monitoring and providing analytics for tethered UEs.

-	XRApp server (at SEALDD server or a separate XR enabler) acts as a consumer of ADAES to consume analytics on the VAL session performance related to the tethered link.

The solution introduces a new ADAES analytics event on “tethered VAL connectivity performance”. The tethered VAL connectivity performance can be defined as the application session performance corresponding to either only the “tethered link” (tethered UE and host UE) or the end to end VAL performance including the tethered link (as extension of VAL session performance analytics as in 3GPP TS 23.436 clause 8.2.3).

7.9.2	Solution description

The solution provides a mechanism for analyzing the delay for application layer segments within an end-to-end XR application service segment, and in particular the link between the XRApp Client (e.g., SEALDD client) at the Tethered Device, and the 3GPP UE. Based on this analysis, the mechanism includes the derivation of application layer statistics or predictions for the segment of interest towards the consumer (e.g., SEALDD/VAL server or client).

The procedure associated with the ADAE analytics for tethered UE and application connectivity performance is described below as depicted in Figure 7.9.2-1.

Pre-conditions:

1.	A connection between an ADAEC and ADAES is established.

2.	A connection between tethered VAL UE and host VAL UE is established.

Figure 7.9.2-1: Tethered VAL connectivity performance analytics procedure

1.	The consumer of the ADAES analytics service, e.g., the XRApp server at SEALDD or VAL layer, sends a subscription request to ADAES and provides the analytics event ID e.g., "tethered VAL connectivity performance", the target tethered VAL UE ID or group of tethered UE IDs, the VAL session/service IDs associated with the tethered VAL UE ID, the time validity and area of the request etc.

2.	The ADAES sends a subscription response as an ACK to the consumer.

3.	The ADAES maps the analytics ID (i.e., tethered VAL UE connectivity performance) to a list of data collection event identifiers, and optionally a list of data producer IDs. Such mapping may be preconfigured by the ADAES itself based on ASP preferred event types, or VAL type, or alternatively may be preconfigured by an MNO by means of the OAM.

4.	The ADAES sends a tethered VAL UE connectivity performance data request to the determined tethered ADAEC with the analytics event ID and the configuration of the reporting required (e.g., periodic, based on maximum delay threshold etc.). Such request also includes additional application QoS attributes to be analyzed additionally based on other metrics (tethered link delay, E2E delay, 5GS PER, 5GS PDB/PSDB etc.).

An application session then starts between the tethered VAL UE and the VAL server.

The next steps may happen asynchronously and in parallel given the 5GC reporting of events for the configured data collection.

5a.	The ADAEC starts collecting data from the tethered VAL UE based on the request. The collection and reporting can be similar to ADAE procedure on VAL session performance analytics (as in step 8 of clause 8.2.3 in 3GPP TS 23.436). As enhancement of existing data collection, ADAEC may collect various events related to tethered link delay, e.g., TetheredLinkDelayExperience. This data can thus be about the delay measurements, throughput (e.g., max WiFi capacity), QoE measurements, etc.

5b.	The ADAEC detects/predicts an application QoS change. Such a change can be for example a change in the QoS attributes requirements by means of detecting an event where the maximum latency threshold (e.g., over the tethered link or E2E) accepted by the ASP is exceeded, whereas the detection mechanism is performed over a given time horizon based on the analytics subscription request.

6.	The ADAES sends a tethered VAL UE connectivity performance data response to the ADAES, including the detected/predicted application QoS change.

7a.	The ADAES can optionally further collect data about the DN performance analytics by means of NWDAF events consumption including service experience analytics etc. This type of analytics will provide statistics/predictions on the user plane performance over a specific application server instance and can be used to predict possible service experience downgrade/deviation that could be used as input to step 7b to predict an application QoS change.

7b.	The ADAES predicts an application QoS change. Such a change can be for example a change in the QoS attributes requirements by means of predicting (based on collected metrics and detection in step 5b) an event where the maximum latency threshold (e.g., over the tethered link, over the DN link, or over the E2E connectivity) accepted by the ASP is likely to be exceeded over a given time horizon based on the analytics subscription request.

NOTE:	How the DN performance analytics together with the ADAEC measurements are used together to trigger an event related to application QoS change is up to implementation.

8.	The ADAES sends the derived analytics notification to the consumer (e.g., VAL server or any other authorized NF/AF consumer).

7.9.3	Solution evaluation

This solution provides a mechanism for monitoring performance of a tethered UE session with the server and providing analytics on the performance attributes (e.g. RTT). This solution requires enhancement in ADAES to support such new capability. Also, some steps of this solution may be impact depending on whether the XRApp server and client are deployed in SEALDD or VAL layer.

The benefit for such analytics is that the XRApp (e.g. at VAL server) consuming ADAE tethered connectivity performance analytics can adapt its behavior (e.g., application layer configuration adaptation, source coding rate reduction, video encoding configuration changes in terms of FPS, Q-parameter, number of eye buffers, or similarly, audio encoding configuration change to a lower encoding quality, AS/EAS load balancing etc.), or request SEAL NRM / 5GC for adaption of the QoS treatment based on the ASP application requirements.

This solution is feasible and doesn’t depend on other 3GPP features.

7.10	Solution #10: EDN selection based on predicted network connectivity

7.10.1	Architecture Impacts

This solution is based on architecture of EDGEAPP as described in 3GPP TS 23.558 [11].

7.10.2	Solution description

7.10.2.1	General

In service provisioning stage, the UE network connectivity towards EDN is not yet established. This solution improves service provisioning procedure in EDGEAPP, the ECS can obtain the predicted Application session's network connectivity then the ECS can proceed with identifying EES and corresponding EDN for the EEC in the UE.

7.10.2.2	Impact to EDGEAPP

Figure 7.10.2.2-1: Service provisioning – Request/Response

The procedure is similar to clause 8.3.3.2.2 of 3GPP TS 23.558 [11] with the following clarifications:

-	In step 2, the ECS obtains predicted application session connectivity information (e.g. DNAI, N6 routing info) for UE traffic towards EDN. Then the ECS further identifies the EES(s) based on the obtained DNAI and EES registered DNAI, considering which EES endpoint is topologically close to the obtained N6 routing information. Alternatively, if EES has registered EES location (e.g. civic address, GPS coordinates, data center ID)into the ECS and the ECS can map the predicted application session connectivity information to EES location, then the ECS can identify the EES(s) according to the predicted application session connectivity information, ECS local configuration and EES location in EES registration information.

Editor's Note:	It is FFS how to obtain the predicted application session connectivity information for UE traffic towards EDN and whether application session connectivity information can include UPF instance information, which depends on SA2 for session info exposure.

7.10.2.4	Information flows

Editor's note:	This clause provides the corresponding APIs and information flows for supporting the solution.

7.10.3	Solution evaluation

This solution addresses KI#6 to support XR application server selection to further reduce service latency in a scenario depicted in figure 4.6.1. Function related to how to obtain predicted application session connectivity information needs further evaluation.

7.11	Solution #11: Support the tethered UE based on PINAPP

7.11.1	Architecture Impacts

As discussed in TR 26.806 [8], there are three types of tethered UE, Tethered Standalone AR Glasses, Tethered Display AR Glasses, and Tethered AR Glasses with 5G Relay. To focus more on the interaction with applications, the architecture of the application enablement layer can be divided into the following two types based on the deployment of the XR application. One is the tethered XR device, where XR application client is deployed on the tethered XR device, i.e., Tethered Standalone AR Glasses, and Tethered AR Glasses with 5G Relay. The other is the Tethered Display XR device, who deploys the XR application on the 5G client.

Usually the tethered XR device is none-3GPP device so it should connecting to the 5G UE to access the 3GPP network. Considering the PEGC is a UE with subscription data related to PIN within the 5GS, so the 5G UE could be the  PEGC and PEMC, the tethered XR device can act as a PINE.

Then the proposed application layer architecture for the tethered XR device is shown in Figure 7.11.1-1. The XR application client runs on the tethered XR device. The tethered XR device is tethered to a 3GPP UE and potentially uses the PINAPP capabilities to support the XR application, e.g., 5G UE acting as the PEGC and PEMC could be enhanced to provide the measurement service.

Figure 7.11.1-1. Application enablement architecture based on PINAPP to support Tethered XR device

SEALDD could utilize PINAPP’s PIN management capability to do the tethered device discovery, then to do the tethering link measurement of the tethered XR device. Some enhancement is needed to support the interaction between the SEALDD client and XR client on different device over SEALDD-C. As shown in the figure, the SEALDD client on the 3GPP UE, acting as PIN client, could get tethered device information from the PEMC using the PIN-3 interface.

Figure 7.11.1-2. Application enablement architecture based on SEALDD and PINAPP to support Tethered XR device

For the Tethered Display device, the proposed application layer architecture is shown in Figure 7.11.1-2. The 3GPP UE includes the XR application and functions. The Tethered Display device is connected to the 3GPP UE for displaying. The PIN client is installed on the Tethered Display device to measure and manage the media transmission.

Figure 7.11.1-3. Application enablement architecture based on PINAPP to support Tethered display device

SEALDD could utilize PINAPP’s PIN management capability to do the tethered device discovery, then to do the tethering link measurement, with some architecture enhancement. As shown in the figure, the SEALDD client on the 3GPP UE, acting as PIN client, could get tethered device information from the PEMC using the PIN-3 interface. The SEALDD-UUc interface needs to be defined to support the interaction between the SEALDD client on the tethered UE and SEALDD client on the 3GPP UE.

Figure 7.11.1-4. Application enablement architecture based on SEALDD and PINAPP to support Tethered display device

7.11.2	Solution description

With the above architecture discussion, there are two ways to do the tethering link measurement. One is SEALDD and VAL coordination measurement based on PIN, the other is the SEALDD measurement based on PIN. The SEALDD and VAL coordination measurement is based on assumption that there is business agreement between the VAL provider and the SEALDD provider, which request the VAL client to respond to ICMP Ping packet. The SEALDD layer based measurement is based on the SEALDD client installed on tethered and 3GPP devices.

7.11.2.1	SEALDD and VAL coordination measurement based on PIN

Figure 7.11.2.1-1 illustrates the procedure of SEALDD and VAL coordination measurement based on PIN.

Pre-conditions:

1.	The UE or PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE or PINE has already been registered in PIN server;

3.	The PEMC on the 3GPP device has already get the tethered device information from PIN server;

4.	There is business agreement between the VAL provider and the SEALDD provider, which request the VAL client to respond to ICMP Ping packet.

Figure 7.11.2.1-1: SEALDD and VAL coordination measurement based on PIN procedure

1.	The PIN is successfully created and in use. The 3GPP UE acting as PEGC and PEMC, the Tethered XR devices acting as a PINE. During the PIN creation, the detail information of the tethered device has been provide to PEMC on 3GPP UE from PINAPP server as defined in clause 8.5.2, 3GPP TS 23.542[19].

2.	The regular data transmission connection is established, with the information received in step1.

The connection between the SEALDD server and 3GPP UE is established as defined in clause 9.2.2.2, 3GPP TS 23.433.

3.	The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. Optionally, the request contains the tethered device information. If authorization is successful, the SEALDD server sends a response to the VAL server with the subscription ID, expiration time.

4.	The SEALDD transmission quality measurement as defined in clause 9.7.2.1, 3GPP TS 23.433.

5.	The SEALDD client gets the information of the tethered device from the PEMC over PIN-1, e.g., the MAC address, PINE Address, Port number.

Editor's note: The specific service and API supporting the PINE discovery needs to be defined in PINAPP.

6.	The SEALDD client on the 3GPP UE interacts with XR client on the tethered UE to do the measurement based on ICMP ping protocol.

7.	The SEALDD client sends the report to the SEALDD server.

8.	The SEALDD server reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, packet loss rate) to the VAL server via the notification message.

7.11.2.2	SEALDD measurement based on PIN

Figure 7.11.2.2-1 illustrates the procedure of SEALDD measurement based on PIN.

Pre-conditions:

1.	The UE or PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE or PINE has already been registered in PIN server;

3.	The PEMC on the 3GPP device has already get the tethered device information from PIN server;

Figure 7.11.2.2-1: SEALDD measurement based on PIN procedure

1.	The PIN is successfully created and in use. The 3GPP UE acting as PEGC and PEMC, the Tethered XR devices acting as a PINE. During the PIN creation, the detail information of the tethered device has been provide to PEMC on 3GPP UE from PINAPP server as defined in clause 8.5.2, 3GPP TS 23.542[19].

2.	The regular data transmission connection is established, with the information received in step1.

The connection between the SEALDD server and 3GPP UE is established as defined in clause 9.2.2.2, 3GPP TS 23.433.

Optionally, the SEALDD client gets the tethered device information form PEMC(e.g., the MAC address, PINE Address, Port number.) and sends the direct  data transmission connection request to the tethered UE using the information form PEMC to establish a connection between the 3GPP UE and tethered device as defined in 7.11.2.3.

3.	The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. Optionally, the request contains the tethered device information. If authorization is successful, the SEALDD server sends a response to the VAL server with the subscription ID, expiration time.

4.	The SEALDD transmission quality measurement as defined in clause 9.7.2.1, 3GPP TS 23.433.

5.	The SEALDD client gets the information of the tethered device from the PEMC over PIN-1, e.g., the MAC address, PINE Address, Port number.

Editor's note: The specific service and API supporting the PINE discovery needs to be defined in PINAPP.

6.	The SEALDD client on the 3GPP UE interacts with SEALDD client on the tethered UE to do the measurement based on ICMP ping protocol, or using monitoring packet in established tethering link SEALDD connection.

7.	The SEALDD client sends the report to the SEALDD server.

8.	The SEALDD server reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, packet loss rate) to the VAL server via the notification message.

7.11.2.3	Connection establishment between the SEALDD client on 3GPP UE and tethered device

Depicted in figure 7.11.2.3-1 is the procedure for establishment of direct data transmission connection between the SEALDD client on 3GPP UE and tethered device.

Figure 7.11.2.3-1: Establishment of Direct Connection between the SEALDD client on 3GPP UE and tethered device

1.	SEALDD client on 3GPP UE sends a Direct Connection Request to SEALDD client on tethered UE. This message includes the UE identity, SEALDD-UUc Data transmission connection information(e.g., address/port allocated).

2.	SEALDD client on Tethered UE initiates the procedure for mutual authentication. The successful completion of the authentication procedure completes the establishment of the Direct Connection.

7.11.3	Solution evaluation

This solution addresses KI#5 by to utilizing PINAPP to do the tethered device discovery, the SEALDD to do the traffic transmission and measurement (including tethered device link). For SEALDD and VAL coordination measurement, the SEALDD client on the 3GPP UE interacts with XR client on the tethered UE to do the measurement based on ICMP ping protocol. For SEALDD measurement, the SEALDD client on the 3GPP UE interacts with SEALDD client on the tethered UE to do the measurement based on ICMP ping protocol, or using monitoring packet in established tethering link SEALDD connection.

7.12	Solution #12: XR application server deployment enhancement

7.12.1	Architecture Impacts

This solution is presented based on the existing EDGEAPP architecture in 3GPP TS 23.558 [11].

7.12.2	Solution description

7.12.2.1	General

This solution proposes that the enabler layer determines the EAS (e.g. XR application) which can provide service with minimum N6 latency, using such mechanism, the E2E KPI requirement of XR application can be satisfied.

In this solution, to avoid the UPF and EAS are not close to each other (e.g. not in the same data center) and the N6 delay still contains the delay of UPF – APP server involving the data transmission via router (s), the EES can determine the EAS based on the EAS related UPF information and the UE serving UPF information. In this way, the data transmission on N6 can be optimized since the data transmission does not need to be forwarded by additional router(s), and the shorter N6 delay can be provided.

7.12.2.2	Procedure

Pre-condition:

1.	The EEC obtains the ECS address information.

Procedure:

1.	The EEC sends the service provisioning request message to the ECS as described in 3GPP TS 23.558 clause 8.3.3.2.2.

2.	Upon receiving the request message from EEC, the ECS determines the EES as described in 3GPP TS 23.558 clause 8.3.3.2.2.

3.	The ECS sends the service provisioning response message to the EEC, the response message contains the identified EES information.

4.	The EEC send the EAS discovery request message to the EES as described in 3GPP TS 23.558 clause 8.5.2.2

5.	Upon receiving the request message from EEC, the EES determines the EAS (i.e. XR EAS) as described in 3GPP TS 23.558 clause 8.5.2.2. Furthermore,  the EES can determine the EAS (i.e. XR EAS) based on the EAS UPF information (e.g. UPF endpoint) in EAS profile and UE serving UPF information which can be obtained from 5GC.

6.	The EES sends the EAS discovery response message to the EEC, the response message contains the identified EAS (i.e. XR EAS) information. Alternatively, the EES may based on UE IP address information (i.e., NATed IP from local PAS), and select the EAS (i.e. XR EAS) where its IP address has a close affinity to the local UPF.

NOTE:	The EES and EAS can be pre-configured with relevant UPF information via OAM.

Editor's note: whether the solution is to be included in Normative work is subject to coordination with SA2 and it is FFS.

Editor's note: How to obtain the UE serving UPF information is FFS

7.12.2.4	Enhancement to EAS profile

Table 8.2.4-1: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| EAS UPF information | O | The UPF information (e.g. UPF endpoint) related to the EAS |
| List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| > Bundle ID (NOTE 3) | O | Bundle ID as described in clause 7.2.10. |
| > List of EAS IDs (NOTE 2, NOTE 3) | O | List of the EAS IDs of the EASs to be invoked by the EAS for an EAS driven association of EASs. |
| > Bundle type | M | Type of the EAS bundle as described in clause 7.2.10 |
| > Main EASID | O | Indicate which EAS in a bundle takes the main EAS service role. |
| > EAS bundle requirements | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. |
| Allowed MNO information (NOTE 4) | O | Information of the allowed operator (e.g. MNO name, PLMN ID) from which its subscriber can consume the EAS services |
| EAS Type | O | The category or type of EAS (e.g. V2X, UAV, application enabler) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS synchronization support | O | Indicates if the EAS supports content synchronization between EASs. |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE indicates which ACR scenarios are supported by the EAS, also indicates the EAS ability (e.g. EAS bundle information) of handling bundled EAS ACR. |
| EAS Transport layer service continuity support | O | This IE indicates the EAS service continuity support for seamless transport layer (e.g. TCP/TLS/QUIC) relocation |
| General context holding time duration (NOTE 1) | O | The time duration that the EAS holds the context before the AC connects to the EAS in case of ACR for service continuity planning. It is an indication of the time the EAS holds the application context for a UE to move to its service area after receiving an ACR notification from the EES following an ACR request from the EEC. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |
| NOTE 1: Since the EASID of the EAS identifies the type of the application (e.g. SA6Video, SA6Game etc) as described in clause 7.2.4, "General context holding time duration" determined by EAS can depend on the EASID (type of the application). NOTE 2: This IE may be provided when only bundle ID is provided, and the bundle type indicates the proxy bundle. NOTE 3: At least one of the IEs shall be present if EAS bundle information is provided. NOTE 4: For edge node sharing scenario, in order to restrict the access to the subscriber of the partner operator, this IE should only include MNO information of the leading operator. |  |  |

Editor's note: How the UPF information in the EAS profile is synchronized with UPF status in 5GC is FFS

7.12.3	Solution evaluation

Solution#12 provide feasible solution for further minimum N6 latency for XR application to address Key issue#6. The EAS discovery procedure is enhance to select the EAS with minimum N6 latency.

This solution has dependency on SA2 thus further evaluation is required based on the progress on SA2.

7.13	Solution #13: XR EAS selection based on N6 delay measurement

7.13.1	Architecture Impacts

This solution is based on architecture of EDGEAPP as described in 3GPP TS 23.558 [11].

7.13.2	Solution description

7.13.2.1	General

In EAS discovery procedure, the EES sends N6 delay measurement request to 5GC and obtains the N6 delay of each pair of discovered XR EAS and UPF for the DNAI of the XR EAS if SA2 can expose N6 delay information to EES.

Editor's note: whether the N6 delay request indicator is needed for EAS discovery or selection is FFS.

Editor's note: whether and how the N6 delay measurement could utilized in EAS registration procedure is FFS.

7.13.2.2	Procedure

Pre-condition:

1.	The EEC obtains the ECS address information.

Procedure:

1.	Service provisioning procedure is performed as described in 3GPP TS 23.558 clause 8.3.3.2.2.

2.	The EEC send the EAS discovery request message to the EES as described in 3GPP TS 23.558 clause 8.5.2.2.

3.	Upon receiving the request message from EEC, the EES discovers the candidate XR EAS(s) as described in 3GPP TS 23.558 clause 8.5.2.2.

4.	The EES sends N6 delay measurement request to the 5GC, including: an indication of considering N6 delay, IP address(es) of candidate EAS(s), DNAI(s) of the candidate EAS(s), the measurement protocol(s) and optionally the protocol-specific configuration parameters.

Editor's note:	The exact information element and service of N6 delay measurement is FFS and depend on SA2 working progress.

5.	The 5GC performs N6 delay measurement for each pair of candidate XR EAS and UPF associated with requested DNAI(s).

6.	The 5GC sends N6 delay measurement response to the EES, including the N6 delay(s) related to each candidate XR EAS.

7.	The EES sends the EAS discovery response message to the EEC as described in 3GPP TS 23.558 clause 8.5.2.2.

7.13.3	Solution evaluation

This solution addresses KI#6 to support XR EAS selection based on the measurement of N6 delay of XR EAS(s) and UPF(s) performed by 5GC.

Editor's note: Whether the solution can be concluded and whether the solution can be in normative phase depends on whether SA2 expose N6 delay information to AF.

7.14	Solution #14: Support UE to UE communication in different modes via SEALDD

7.14.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [6] with additional inter-SEALDD client reference point.

Figure 7.14.1-1: SEALDD architecture with on-network and off-network

VAL UE 1 and VAL UE 2 have on-network connectivity, and both VAL UEs can directly communication with each other via off-network connectivity (i.e. SEALDD-PC5) or via on-nework connectivity (i.e. SEALDD).

7.14.2	Solution description

7.14.2.1	General

When two VAL UEs need to communicate with each other for exchanging gaming/interactive data in XR service, they can use servers in the DN as data relay or communicate with each other directly.

NOTE 1:	This solution is only applicable for the XR transmission scenario without involvement of VAL server. As an example, XR gaming application client can support LAN-type of multi-player game mode without involvement of XR gaming server to process run-time game data.In order to improve service experience (esp. for reducing service latency), SEALDD server monitors distance of UEs involved in the XR communication. When SEALDD server finds that the two UEs are within direct communication range, the SEALDD server instructs the SEALDD clients to switch to direct communication mode. Such mode switch decision needs to take E2E communication performance into consideration.

If the UE-to-UE application performance analytics result from ADAE shows the quality in off-network mode will be degraded and the quality in on-network mode is estimated to be good, or vice versa, the mode switch is done in SEALDD layer.

NOTE 2:	For the same XR application service, all XR traffic flows are transmitted via either off-network or on-network between UEs.

7.14.2.2	SEALDD enabled UE-to-UE communication based on policy

The SEALDD servers has Data Delivery (DD) policy being provisioned for UE-to-UE communication. The DD policy is enforced by the SEALDD server to switch the SEALDD connection for UE-to-UE communication (either direct or indirect).

Pre-conditions:

- The SEALDD server has DD policies available.

- The SEALDD clients in UE1 and UE2 has discovered the same SEALDD server.

Figure 7.14.2.2-1: Policy enforced by SEALDD server for connectivity between two UEs

1a.	The UE-to-UE communication is established between UE 1 and UE 2 via SEALDD client.

1b.	The SEALDD client in UE1/2 informs the SEALDD server about the establishment of direct SEALDD communication.

2.	Based on SEALDD UE-to-UE policy, the SEALDD server requests 3GPP CN (NWDAF/NEF) for UE relative proximity analytics for UE1 and UE2, requests 3GPP CN (GMLC/NEF) for SL/Ranging service exposure about the relative locations or distances and directions related to the UE1 and UE2, and requests ADAES for UE-to-UE application performance analytics as described in 3GPP TS 23.436 clause 8.4, requests NWDAF/NEF for QoS Sustainability Analytics and Service Experience Analytics for UEs and/or requests PCF/NEF for QoS monitoring.

NOTE 1:	If SA2 provided services can support UE-to-UE communication analytics will be further re-visited in the normative work. Whether ADAES supports off-network communication analytics will be further worked in normative stage.

NOTE 2:  For the switch between direct mode and indirect mode, it is FFS whether UE can trigger such an action.

3.	According to the SEALDD UE-to-UE policy, if relative proximity analytics result shows that both UEs are not in vicinity to each other (distance between UEs > proximity threshold in UE-to-UE policy), and/or the QoS/QoE for UE-to-UE direct communication analytics result is no so good (e.g. PLR in analytics report > PLR threshold in UE-to-UE policy), the SEALDD continues with step 4 for indirect communication preparation; otherwise, the SEALDD keeps monitoring (as described in step 2) for the two UEs.

4.	After establishment of SEALDD connections for UE1 and UE2, the SEALDD traffic carrying XR flows are processed in the SEALDD server. The SEALDD server forwards traffic between UE1 and UE2, and may buffer data burst from originating UE and send it to destination UE smoothly.

5.	If later on, both UEs are in proximity which is capable of direct communication and the QoS/QoE analytics result from ADAES for UE-to-UE direct communication shows good quality, the SEALDD server helps to establish direct SEALDD communication by informing the SEALDD client of any of the two UEs with the information of its peer SEALDD client (of UE1 or UE2). Then the SEALDD client 1 or 2 establishes SEALDD connection towards the peer SEALDD client via SEALDD-PC5 and the SEALDD traffic carrying XR flows are exchanged directly between UE1 and UE2 via SEALDD-PC5.

Based on the monitoring and analytics result (as requested in step 2), and SEALDD UE-to-UE policy:

-	if the current UE1-UE2 communication mode is direct and SEALDD server decides to switch to indirect communication mode, the SEALDD server establishes SEALDD connections with SEALDD client 1 and SEALDD client 2 as described in step 4, then triggers direct SEALDD communication release towards the SEALDD client which received establishment request in step 5 and the SEALDD client further releases the direct SEALDD connection towards its peer SEALDD client via SEALDD-PC5.

-	if the current UE1-UE2 communication mode is indirect and SEALDD server decides to switch to direct communication mode, the SEALDD server triggers direct SEALDD communication setup as described in step 5 and releases SEALDD connections towards SEALDD client 1 and SEALDD client 2.

7.14.3	Information flows

Table 7.14.3-1 describes the new information elements to be added to the SEALDD policy configuration request in 3GPP TS 23.433 [6] Table 9.10.3.1-1 in order to support UE-to-UE communication flows and policy.

Table 7.14.3.1-1: SEALDD policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| XR traffic flows | M | Identify traffic flows for XR application. |
| > Application traffic identifiers | M | Identify of the application traffic (e.g. VAL server ID, VAL service ID) |
| > identity list | M | List of identifiers of the VAL UEs or VAL users for which SEALDD policy applies |
| SEALDD policy | M | The SEALDD policy associated with application traffic identifiers, VAL UE identity |
| > UE-to-UE policy | O | Specifies UE-to-UE direct communication policy including proximity threshold, QoS threshold (e.g. bitrate, latency and jitter, PLR, PER) or QoE threshold (e.g. MOS as specified in ITU-T P.1203.3 [xx]) for entering direct communication mode, and proximity threshold, QoS threshold or QoE threshold for leaving direct communication mode. |

NOTE:	Remaining information flows (e.g. inform connection establishment in step 1b, trigger direct SEALDD communication setup in step 5) to support the procedure can be done during normative work.

7.14.4	Solution evaluation

This solution addresses KI#3 to support:

-	How the application enablement layer can be enhanced to support usage of direct communication between 5G connected AR/VR devices.

-	Whether and how the application enablement layer can be enhanced to make coordination (e.g., the connection redundancy) between network-based communication and direct communication between AR/VR devices for AR/VR services communication.

The SEALDD layer facilitates the communication between UEs, the SEALDD server supervises the UE-to-UE distance and their communication quality to coordinate on-network communication and off-network communication.

7.15	Solution #15: Application enablement layer capabilities usage to support the user group level QoS guarantee

7.15.1	Architecture Impacts

No architecture impact identified.

7.15.2	Solution description

Figure 7.15.2-1 illustrates the procedure to support the user group level QoS guarantee. The user group level QoS guarantee provides different QoS for different users in single XR service.

Pre-conditions:

1.	The SEALDD server acts as NSaaS provider, and is authorized to get network slice related information and  capabilities in Network Slice ServiceProfile (3GPP TS 28.541[20]) from NSCE server as defined in 3GPP TS 23.435[21].

Figure 7.15.2-1: procedure of supporting the user group level QoS guarantee

- The VAL server sends a user group level QoS guarantee request to SEALDD server. The request includes Service Description, QoS guarantee requirement list, and corresponding different target UE(s). When the All other VAL UEs Indication is indicated, the corresponding requirements apply to all other VAL UEs of the application identified by VAL server ID and/or VAL service ID, except for those already indicated (if any). The QoS guarantee requirement list could include the Delay requirement, Delay Variation requirement, Bandwidth requirement, Number of UE requirement, Coverage Area requirement, Other requirement(s), Delay Variation requirement.

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Target UE(s) | M | Identifier of the VAL UE(s) |
| >VAL UE identity | O (See NOTE) | Identifier of the VAL UE, e.g. UE ID |
| >VAL UE group ID | O (See NOTE) | Identifier of a specific VAL UE group, as defined in clause 7.5 of 3GPP TS 23.434 [4] |
| >VAL UE identity list | O (See NOTE) | Identifies a list of VAL UEs, e.g. the list of UE ID |
| >All other VAL UEs Indication | O (See NOTE) | Indicates all other VAL UEs of the application identified by VAL server ID and/or VAL service ID, except for those already indicated (if any) |
| Slice identifier | O | The slice identifier (S-NSSAI) the VAL server consumed |
| QoS guarantee requirement list | M | The QoS guarantee requirement information |
| > Delay requirement | O | Maximum delay accepted by the target UE(s) |
| > Delay Variation requirement | O | Maximum Delay Variation accepted by the target UE(s) |
| > Bandwidth requirement | O | Minimum bandwidth accepted by the target UE(s) |
| > Number of UE requirement | O | Number of UE requirement |
| > Coverage Area requirement | O | Coverage Area requirement |
| > Other requirement(s) | O | In addition to the requirements above, other service requirements, e.g., isolation requirement, PDU Set QoS Parameters, etc.. |
| > Valid time | O | The valid time for the requirement |
| NOTE: One of them shall be present as the measurement target UE. |  |  |

2a.	Based on the received QoS guarantee requirement list and network slice related information received from NSCE, the SEALDD server determine whether the slice related communication service lifecycle management is needed. If the performance of network slice identified by the provided Slice identifier is not satisfying, the SEALDD server could trigger slice related communication service Reconfiguration.

If the network slice related information of network slice identified by the provided Slice identifier is not received from NSCE before, the SEALDD server can get the related information by invoking the Network Slice Information delivery as defined in 3GPP TS 23.435 [21], clause 9.17.

Before triggering the network slice management service, to determine the target UE of network slice(s), the SEALDD server acting as AF, may receive a UE location report or a monitoring event report from 5GC (assuming that SEALDD server has subscribed to consume 5GC services like LCS or NEF monitoring events related to UE actual location, or UE mobility analytics from NWDAF) or Location service from SEAL.

2b.	The SEALDD server translates the QoS guarantee requirement into individual QoS parameters, and initiates AF session with required QoS as defined in 3GPP TS 23.502 [4], clause 4.15.6.6a, or Multi-member AF session with required QoS as defined in 3GPP TS 23.502 [4], clause 4.16.5.3, or QoS/resource management capability provided by SEAL NRM as defined in 3GPP TS 23.434 [12], clause 14.3.5.2.

2c.	The SEALDD server determines whether the SEALDD enabled bandwidth control is needed. If needed(e.g., multiple user groups have different bandwidth requirements), the SEALDD server may initiate the SEALDD enabled bandwidth control for different VAL users as defined in 3GPP TS23.433 [6], clause 9.8.2.

3.	The SEALDD server sends the user group level QoS guarantee response to the VAL server indicating the result of the request.

7.15.3	Solution evaluation

This solution addresses KI#4, the user group level QoS guarantee is provided based on the interaction with NSCE server and 5GC.

7.16	Solution #16: XR Application enablement client capability information procedure

7.16.1	Architecture Impacts

This solution describes XR Application enablement client capability information procedure.

7.16.2	Solution description

This solution is targeting Key issue #4: Application enablement layer capabilities usage to support the XR services. The SEALDD server and client support XR App functionality. There is a heterogeneity in the VR device ecosystem. As per the [22], the device constraints plays a big role in optimizing the VR data delivery. Because different devices have different capabilities and limitations in terms of processing power, memory, battery life, display resolution, and rendering quality. For example, standalone VR headsets, such as Oculus Quest, have lower hardware specifications than PC-based VR headsets, such as HTC Vive. This means that they may not be able to handle high-quality VR videos smoothly and may require lower resolutions, bitrates, or frame rates. To optimize VR video streaming for different devices, it is important to consider the capabilities. It is also mentioned in [22] that for example, a high-resolution video that displays beautifully on a 4K TV may struggle to play smoothly on a low-end smartphone due to limited processing power. Similarly, a video encoded with a high bitrate could experience buffering on devices with slower network connections. In [24] provides an example that to stream a 16K video at 30 frames per second (FPS) requires a raw bitrate of ∼48 Gbps; even after encoding, its bitrate could be around 300 Mbps. It is important to understand the capability(like resolution, FPS, etc) to better serve the user.

Therefore this solution introduces xr application client capabilities to enable the sealdd layer to optimize the XR traffic data delivery.

9.8.2.1 SEALDD enabled bandwidth control for different VAL users The SEALDD layer can provide the differentiated data delivery service with different bandwidth experience for VAL users, where the VAL server can provide the bandwidth limit (i.e., minimum bandwidth requirement and maximum bandwidth limit) for VAL users. Figure 9.8.2.1-1 illustrates the procedure for bandwidth control for different VAL users. Pre-conditions: 1. The VAL server has discovered and selected the SEALDD server by CAPIF functions as specified in clause 9.4.2. 2. The SEALDD server has subscribed to 5GC for QoS monitoring of the specific UE related to the VAL user, as defined in clause 5.2.6.9 in 3GPP TS 23.502 [6]. 3. The SEALDD policy (i.e, the bandwidth control policy) has been configured in SEALDD server, as described in clause 9.10. 4. The SEALDD client can obtain the XR device capability information. Figure 9.8.2.1-1: SEALDD enabled bandwidth control transmission procedure 1. The VAL server sends a Sdd_regularTransmission request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), the VAL server’s total bandwidth limit and the bandwidth limits (i.e. minimum bandwidth requirement and maximum bandwidth limit) for VAL users. 2. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends a response to the VAL server. 3. The VAL client sends a SEALDD service request to SEALDD client. 4. The VAL/SEALDD client discover and select the proper SEALDD server for the VAL application. After this step, the VAL server is discovered and selected along with the associated SEALDD server, the SEALDD client can get the SEALDD server's address. 5. The SEALDD client sends Sdd_RegularTransmissionConnection_Establish request to SEALDD server with the SEALDD client ID, the VAL user or UE identity, XR application device capability information as per the Table 9.2.3.3-1. 6. The SEALDD server performs bandwidth limit check according to the VAL user’s bandwidth limit, the current SEALDD traffic delivery status, the VAL server’s total bandwidth limit and/or the related UE’s current network status (i.e. via QoS monitoring report from the 5GC). If the available bandwidth (i.e. the remaining bandwidth that can be used for the VAL user without exceeding the VAL server’s total bandwidth limit) cannot meet the VAL user’s minimum bandwidth requirement, the SEALDD server will reject the SEALDD client’s connection establishment request. 7. When the available bandwidth can meet the VAL user’s requirement, the SEALDD client can establish the SEALDD connection with the SEALDD server. The SEALDD server can calculate the suggested traffic transmission bandwidth to the SEALDD client according to the VAL user’s bandwidth limit and the related UE’s current network status (i.e. via QoS monitoring report or ECN marking for L4S report from the 5GC) and XR application device capability information. NOTE 1: whether the XR device capability can be obtained by SEALDD client, and what the detailed XR device capability can be used by SEALDD server to perform bandwidth control are needed to further discuss. NOTE 2: Whether the XR device capability is obtained and maintained by VAL layer is enough to support the bandwidth control are needed to further discuss. 8. If the bandwidth limit check is failed (i.e., the available bandwidth cannot meet the VAL user’s minimum bandwidth requirement) in step 6, the SEALDD server can send Sdd_RegularTransmissionConnection_Establish response with the failed result (i.e., reject the connection establishment) and the pending timer to trigger the re-connection from SEALDD client. If the bandwidth limit check is successful (i.e., the available bandwidth can meet the VAL user’s requirement) in step 6, the SEALDD server can send Sdd_RegularTransmissionConnection_Establish response with the successful result and/or the suggested traffic transmission bandwidth. NOTE: The SEALDD server can re-allocate the available bandwidth resource to different VAL users according to the configured bandwidth control policy, as described in clause 9.10. If the connection establishment is rejected, the SEALDD client can re-establish SEALDD connection by performing steps 5-8, when the pending timer is expired. For the uplink application traffic, the SEALDD client can buffer or drop some packets when the uplink traffic from VAL client exceeds the suggested traffic transmission bandwidth. Similarly, for the downlink application traffic, the SEALDD server can buffer or drop some packet when the downlink traffic from VAL server exceeds the suggested traffic transmission bandwidth. 9.2.3.3 SEALDD regular transmission connection establishment request Table 9.2.3.3-1 describes the information flow from the SEALDD client to the SEALDD server or from the SEALDD server to the SEALDD client for requesting the regular SEALDD connection establishment. Table 9.2.3.3-1: SEALDD regular transmission connection establishment request Information element; Status; Description. Requestor ID; M; Identity of the requestor (SEALDD client or SEALDD server).. SEALDD-UU flow ID; M (See NOTE 1); Identity of the SEALDD-UU flow.. VAL server ID; O; Identity of the VAL server, applicable for SEALDD client side initiated request.. VAL service ID; O; Identity of the VAL service. Selected VAL server endpoint; M; Endpoint of the selected VAL server. SEALDD traffic descriptor; O; SEALDD traffic descriptor (e.g. address, port, URL, transport layer protocol) of the SEALDD client side (for client side initiated request) or the SEALDD server side (for server side initiated request) used to establish SEALDD connection.. Identity; O; The VAL user ID of the VAL user or VAL UE ID.. SEALDD communication lifetime; O; Identifies the DD communication lifetime, applicable for SEALDD server side initiated request.. Capability for BAT and periodicity adaptation; O (See NOTE 2); Indicates BAT and periodicity adaptation capability for SEALDD client in client side initiated request.. Transmission assistance info; O (See NOTE 2); Indicates transmission assistance information for uplink SEALDD traffic in client side initiated request. It includes BAT, BAT window, periodicity, and periodicity range.. L4S feedback capability; O (See NOTE 3); Identifies the L4S feedback capability (i.e. ECN identification, L4S feedback) for client side initiated request. XR Application device capability information; O (See NOTE 4); Indicates XR Application device capability information. NOTE 1: The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and data transmission session. NOTE 2: If provided, only one of these IEs shall be present in the message. NOTE 3: This IE is used for the SEALDD enabled congestion control for VAL applications, as specified in clause 9.8.2.2. NOTE 4: This IE is used for the SEALDD enabled XR traffic data delivery..

7.x	Solution #x: <title>

Editor's Note:	Provide a suitable title for the solution.

7.x.1	Architecture Impacts

Editor's note:	This clause provides the architecture impacts of the solution.

7.x.2	Solution description

Editor's Note:	This clause will describe the solution. Each solution should clearly describe which of the key issues it covers and how.

7.x.3	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

8	Business Relationships

8.1	General

The business relationships outlined in 3GPP TS 23.434 and 3GPP TS 23.433[6] remain applicable, with the following modifications:

-	the XR service provider acting as the VAL service provider;

-	the XRAPP service provider acting as the SEAL/SEALDD service provider.

9	Overall evaluation

9.1	General

All the key issues and solutions specified in this technical report are listed in Table 9.1-1. It includes the mapping of the key issues to the solutions. It also lists the dependencies on other enablement framework.

Table 9.1-1: Key issue and solutions

| Key issues | Solution | Dependency on other application enablement capabilities | Dependency on other working groups |
| --- | --- | --- | --- |
| #1: KPI measurement and exposure | #4: Support of QoS measurement for Multi-Modal traffic in SEALDD layer | SEALDD | None |
|  | #5: Multi-flow synchronization for multi-modal XR application | SEALDD | None |
|  | #6: Multimodal flows alignment and monitoring | SEALDD | None |
| #2: E2E Multi-Modal communication flows | #1: SEALDD Policy Configuration for multi-modal flows | SEALDD | None |
|  | #2: Policy based Multi-modal SEALDD flow establishment | SEALDD | None |
|  | #3: Support of Multi-Modal traffic indication in SEALDD layer | SEALDD | None |
|  | #5: Multi-flow synchronization for multi-modal XR application | SEALDD | None |
|  | #6: Multimodal flows alignment and monitoring | SEALDD | None |
| #3: Coordination between direct UE connection and network based connection; | Solution #X: Support UE to UE communication in different modes via SEALDD | SEALDD | None |
| #4: application enablement layer capabilities usage to supporting the XR services; | Solution #Y: Application enablement layer capabilities usage to supporting the XR services | SEALDD, SEAL | None |
| #5: support the tethered UE; | #4: Support of QoS measurement for Multi-Modal traffic in SEALDD layer | SEALDD | None |
|  | #9: Support performance analytics for tethered UEs | ADAES | None |
|  | #11: Support the tethered UE based on PINAPP | SEALDD, PINAPP | None |
| #6: XR application server selection enhancement; | #10: EDN selection based on predicted network connectivity | EDGEAPP | SA2 |
|  | #12: XR application server deployment enhancement | EDGEAPP | SA2 |
|  | #13: XR EAS selection based on N6 delay measurement | EDGEAPP | SA2 |
| #7: E2E KPI optimization for XR service; | #8: EAS instantiation enhancement to satisfyE2E KPI requirements for XRapp | EDGEAPP | None |
| #8: PDU set handling for XR traffic. | #7: Support multi-modal service in SEALDD | SEALDD | None |
|  | #4: Support of QoS measurement for Multi-Modal traffic in SEALDD layer | SEALDD | None |

Editor's Note:	This table will be updated accordingly based on the working progress.

9.2	Architecture evaluation

As discussed in clause 9.1, most solutions have dependencies on other application enablement capabilities.

-	7 solutions are based on SEALDD capability and Potential SEALDD architecture enhancement is needed (based on conclusion of KI#5);

-	3 solutions has impact on the EDGEAPP, no architecture impact;

-	1 solution is dependent on PINAPP, no architecture impact;

-	1 solution is dependent on ADAES, no architecture impact.

With those architecture analyses, two architecture options are proposed in clause 6.1. Option#1defined in clause 6.1.2 proposes an Application enablement architecture for XRAPP function. Option#2 defined in clause 6.1.3 proposes an Application enablement architecture based on existing application enablement capabilities, where XR related services are supported by enhancing relevant existing functions. Option#3 proposes to reuse CAPIF architecture to support the SEAL service discovery and invocation for XR application.

For architecture defined in clause 6.1.2, XRApp services could be exposed via a single function.

For architecture defined in clause 6.1.3, it would be easier for other Verticals to utilize XR related services because they are more like general capabilities.

It is recommended to take option #2 and option #3 with some potential enhancements as the basic architecture to support the XR service.

9.3	Solution evaluation

9.3.1	Key Issue #1 solutions evaluation

Key issue #1 studies which KPI could be measured and exposed to VAL. There are three solutions addressing the related issues. Solution #4, solution #5 and Solution #6 identify different KPIs using different identification.

Regarding the KPI to be measured:

-	Solution #4 improves the SEALDD client for the crossflow RTT measurement for multiple devices in XR scenario, e.g., wireless controllers and headset interaction. The crossflow QoS measurement provides finer granularity for the QoS measurements considering XR traffic route, which could be bi-direction flows. For example, Motion to Photon can be estimated as crossflow RTT measurement (e.g. {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]})

-	Solution #5 proposes to enhance the SEALDD server to perform data transmission quality measurement by including the timestamp information in the SEALDD server encapsulated packets. The transmission delay of multiple associated flows can be calculated after receiving packets with timestamp from the SEALDD client.

-	Solution #6 proposes to align the associated flows of multi-modal XR application to optimize the delay difference. Delay difference is the difference in the time it takes for two or more associated flows to arrive at their destination, which is the single direction.

Regarding the target flow identification, :

-	Solution #4 uses traffic descriptor with traffic direction, which is an endpoint (e.g., Address/Port or URL) at the SEALDD server for the SEALDD traffic. The traffic descriptor uniquely identifies the flow and can be used for the flow identification in the QoS measurement request.

-	Solution #6 uses flows description, which could be Multi-modal Service ID.

The above two solutions use different information element to do the target flow identification, even though the used traffic descriptor and flows description are somehow functionally equivalent. it is recommended that the detailed information element and terminology used should be aligned.

9.3.2	Key issue #2 solutions evaluation

The open issue of KI#2 include:

-	Whether and how to support the E2E multi-modal communication flows between application clients and application servers within the application enablement layer?

-	Whether and how to support the interaction between the application enablement layer and 5G CN to manage E2E multi-modal communication flows between application clients and application servers?

-	Whether and how SEALDD may be enhanced to assist in managing E2E multi-modal communication flows between application clients and application servers involved in the same application service (e.g., support for multi-modal aware SEALDD flow management and policies)?

Based on the approved solutions, it can be classified into four aspects as following:

Aspect#1: the SEALDD policy configuration for multi-modal flows:

Solution#1 is proposed to configure the related SEALDD policy (i.e. synchronization policy) to the SEALDD server. Solution #1 is the only one solution for this aspect.

Aspect#2: the SEALDD connection establishment for multi-modal application.

There are three solutions (i.e. solution#2, solution#3, and solution#5) for the negotiation on the SEALDD-Uu connection information.

In solution#5, the one control plane interaction for SEALDD connection is sufficient to negotiate the multiple SEALDD flow for multi-modal application(s). The multiple control plane interactions (i.e. repeating SEALDD-Uu connection establishment in solution #3, and establishing the individual SEALDD connection and the remaining SEALDD connections in solution#2)will cause the extra complexities, for example, for the SEALDD connection update for multiple flow, the multiple control plane interaction in SEALDD-Uu is needed.

Aspect#3:the multi-modal flow synchronization.

There are two solutions (i.e. solution#5 and solution#6) for this aspect, and they are complementary.

Solution #6 is applicable for multi-modal flow alignment in downlink direction, there is no interaction with 5GC. Solution #5 is applicable for both downlink and uplink synchronization of multi-modal flow, by requesting the 5G network with the required QoS for the adjusted flow(s). Compared with solution#6, solution #5 can additionally optimize the E2E transmission quality for multi-modal application by requesting the QoS optimization to 5G network.

Aspect#4: the QoS information determination for multi-modal requirements

There are two solutions (i.e. solution#3 and solution#5) for this aspect, and they are complementary.

Solution#3 determines the initial or static QoS requirements by SEALDD server according to VAL service ID and/or VAL server ID. The QoS requirements for multi-modal flow can be updated by SEALDD server based on the transmission quality result and the synchronization threshold, as descripted in solution #5.

9.3.3	Key issue #3 solutions evaluation

Key issue#3 studies how to make coordination between network-based communication(on-network communication) and direct communication (off-network communication) between AR/VR devices for AR/VR services communication. Solution #X proposes to coordinate on-network communication and off-network communication by switching the SEALDD connection for UE-to-UE communication (either direct or indirect) based on the monitoring of UE-to-UE distance and their communication quality.

9.3.4	Key Issue #4 solutions evaluation

The open issue of KI#4 include:

-	Whether and how the identified application enablement capabilities could be utilized to jointly support the XR service.

-	Whether and how the application enablement architecture could support the XR service taking advantage of the identified application enablement capabilities.

Solution #X proposes the user group level QoS guarantee based on the interaction with NSCE server and 5GC. The network slice related services provided by NSCE server and Setting up an AF session with required QoS service provided by 5GC, or QoS/resource management capability provided by SEAL NRM could be utilized.

As discussed in clause 6.1.4, architecture option3, the existing SEAL layer could utilize CAPIF to support the service discovery and invocation for XR application.

9.3.5	Key Issue #5 solutions evaluation

Key issue #5 studies how to identify and measure the traffic flows from the tethered devices behind the UE. There are three solutions addressing the related issues: solution #4, solution #9 and solution #11.

-	Solution #4 proposes that the SEALDD client on the tethering device to measures the QoS between the SEALDD client and tethered device.

-	Solution #9 proposes that ADAEC to collect the data from the tethered VAL UE(SEALDD client or VAL client), and analyze the delay for end-to-end XR application service, and the tethered link.

-	Solution #11 proposes to utilize PINAPP to do the tethered device discovery, and utilize SEALDD to measure the delay. The detailed architecture impact is analyzed to support the tethered device.

Regarding the tethered device identification, Solution #4 and solution #11 proposes to utilize PINAPP to do the tethered device discovery and management.

Regarding the tethered link measurement, solution #11 and solution#4 propose to enhance the SEALDD to do the measurement. Solution #11 focuses on how the PINAPP and SEALDD collaborate to do the tethered link measurement, while solution #4 elaborates the detail of measurement implemented by SEALDD. Solution #9 focuses on how the analysis implemented based on collected data.

9.3.6	Key Issue #6 solutions evaluation

Key issue #6 studies the XR EAS selection enhancement for reducing service latency. There are three solutions addressing the related issues: solution #10, solution #12 and solution #13.

-	Solution #10 proposes that the ECS may obtain predicted application session connectivity information if application session connectivity information can be exposed by 5GC, and the Service provisioning may be enhanced considering the closeness of the EES endpoint and the predicted application session connectivity information.

-	Solution #12 proposes that the EES may based on UE IP address information (i.e., NATed IP from local PAS) select the EAS (i.e. XR EAS) where its IP address has a close affinity to the local UPF.

-	Solution #13 proposes to utilize N6 delay measurement mechanism to enhance the EAS discovery if the N6 delay information can be exposed by 5GC. After candidate EAS(s) discovery is performed, the EES may send request to 5GC for N6 delay measurement for each pair of candidate XR EAS and UPF related to DNAI of the XR EAS if the N6 delay information can be exposed by 5GC.

9.3.7	Key Issue #7 solutions evaluation

The open issue of KI#7 includes:

-	How to support the E2E KPI optimization to meet real-time XR service requirements, taking the processing delay of XR application server into account, e.g. considering the potential enhancements to EDGEAPP in XR application server discovery or instantiation.

-	What available information from 5G network can be used by enable layer to optimize the real-time XR service requirements.

Solution #8 is the only solution for KI#7, the enablement layer (i.e. EES) can determine the required computing ability requirements of EAS instantiation and/or the required QoS information. By interacting with management system (as per the mechanism specified by SA5) and 5GC, the E2E KPI application requirements of XR application can be optimized.

Editor's note: The conclusion and overall evaluation of Key Issue #7 may be updated accordingly if there are other solutions added.

9.3.8	Key issue #8 solutions evaluation

The open issue of KI#8 include:

-	Whether and how SEALDD or other application enabler may be enhanced to facilitate XR communication flows between application clients and application servers involved in the same application service in relation to PDU set (e.g., support for PDU set handling, QoS measurement)?

In the existing solutions, solution#4 addresses QoS measurement for XR flows including PDU set on crossflow RTT. The SEALDD measurement function is enhanced to identify the 1st UL or last DL packet in a PDU set, e.g. for SEALDD client triggered measurement, the SEALDD client needs to identify the last DL packet in a PDU set to complete the measurement for a DL video frame. The crossflow QoS measurement provides added value for the XR service, for example, Motion to Photon can be estimated as crossflow RTT measurement (e.g. {[traffic descriptor 1 for joystick input, UL]; [traffic descriptor 2 for video frame, DL]}).

Editor's note: Whether identifying packet in PDU set is necessary in crossflow RTT measurement is FFS.

In the existing solutions, solution#7 addresses PDU set handling support. The SEALDD layer is enhanced with RTP packetilization and PDU set inclusion.

10	Conclusions

10.1	General conclusions

XR related transmission services are supported based on existing SEALDD architecture with the enhancement identified in clause 6.1.3. CAPIF architecture is utilized to facilitate the SEAL service discovery and invocation for XR application. Specific service enhancements are concluded in the following clauses.

10.2	Conclusions of key issue #1

Solution #4, solution#5 and solution #6 can be modified and adopted into normative work with the following principles:

-	Solution #4 can be used to measure and expose crossflow QoS measurement.

-	Both solution#5 and #6 can be used to measure the transmission delay, while solution #6 can also be used to measure and expose delay difference.

-	For the target flow identification, traffic descriptor could be used to uniquely identifies the flow while the exact information element needs to be defined during normative work.

10.3	Conclusions of key issue #2

Solution#1, solution#2, solution #3, solution #5and solution #6 can be considered for normative work (with modifications as needed) with the following principles:

-	Solution #1 can be used to configure the SEALDD policy to SEALDD server to support multi-modal XR application.

-	For the negotiation on the SEALDD-Uu connection information for multi-modal XR application, the one control plane interaction between SEALDD client and SEALDD server can be utilized, as descripted in solution #1.

-	For the QoS information in multi-modal requirements, solution#3 determines the initial or static QoS requirements by SEALDD server, and can be further updated based on the transmission quality result and the synchronization threshold, as descripted in solution #5.

-	Solution #6 is applicable for multi-modal flow alignment in downlink direction, there is no interaction with 5GC. Solution #5 is applicable for both downlink and uplink synchronization of multi-modal flow, by requesting the 5G network with the required QoS for the adjusted flow(s). These two solutions can be merged/harmonized for the multi-modal flow alignment/synchronization.

NOTE:	The detailed message/API name for the SEALDD transmission in SEALDD-S interface and the SEALDD transmission establishment SEALDD-Uu interface will be determined in the normative phase.

10.4	Conclusions of key issue #3

Solution#X can be considered into normative work to support the coordination between network-based communication (on-network communication) and direct communication (off-network communication).

NOTE 1:	If SA2 provided services can support UE-to-UE communication analytics will be further re-visited in the normative work. Whether ADAES supports off-network communication analytics will be further worked in normative stage.

NOTE 2:  More work in the normative phase is needed to determine whether the decision to switch between direct mode and indirect mode is made by SEALDD client or SEALDD server.

10.5	Conclusions of key issue #4

For the aspect of service discovery and invocation for XR application in key issue #4, the existing CAPIF mechanism can be utilized to support the XR service by invoking the northbound API exposed by the application enablement (e.g. SEAL, EDGEAPP) or the northbound API exposed by 5GS.

The NSCE provided network slice related services and 5GC provided AF session with required QoS services, or SEAL NRM provided QoS/resource management capability could be utilized to optimize the QoS of XR application.

10.6	Conclusions of key issue #5

It is agreed to adopted the following principles in the normative work:

-	The PINAPP is utilized to do the tethered device discovery and management.

-	The SEALDD is enhanced to do the tethered link measurement.

-	The ADAES is enhanced  to support performance analytics for tethered devices.

10.7	Conclusions of key issue #6

Editor's note:	This clause will provide conclusions of key issue #6.

10.8	Conclusions of key issue #7

Solution#8 can be considered into normative work to support the E2E KPI optimization for XR service.

10.9	Conclusions of key issue #8

Solution#4 can be considered for normative work with the following principles:

-	Solution #4 can be used to measure and expose crossflow QoS measurement for XR flows including PDU set.

-	For the target flow identification, traffic descriptor could be used to uniquely identifies the flow while the exact information element needs to be defined during normative work.

Solution #7 can be considered for PDU set handling support for normative work.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-11 | SA6#58 |  |  |  |  | TR skeleton | 0.0.0 |
| 2023-12 | SA6#58 |  |  |  |  | TR Skeleton agreed in SA6#58: S6-234018, S6-234042, S6-234020, S6-234087, S6-234107 Editorial changes by the rapporteur | 0.1.0 |
| 2024-03 | SA6#59 |  |  |  |  | PCR agreed in SA6#59: S6-240150, S6-240679, S6-240683, S6-240708. | 0.2.0 |
| 2024-04 | SA6#60 |  |  |  |  | PCR agreed in SA6#60:S6-241618, S6-241490, S6-241491, S6-241654, S6-241495, S6-241640, S6-241497, S6-241498, S6-241499, S6-241500, S6-241563, S6-241564, S6-241565, S6-241494. Editorial changes by the rapporteur | 0.3.0 |
| 2024-05 | SA6#61 |  |  |  |  | PCR agreed in SA6#61: S6-242198, S6-242529, S6-242531, S6-242534, S6-242540, S6-242543, S6-242544, S6-242545, S6-242669, S6-242686, S6-242689. Editorial changes by the rapporteur | 0.4.0 |
| 2024-05 | SA6#61 |  |  |  |  | PCR agreed in SA6#61: S6-242751, S6-242753, S6-242754. Editorial changes by the rapporteur | 0.5.0 |
| 2024-06 | SA#104 | SP-240737 |  |  |  | Submitted to SA#104 for information | 1.0.0 |
| 2024-07 | SA6#62 Ad-Hoc |  |  |  |  | PCR agreed in SA6#62 Ad-Hoc: S6a240374, S6a240247. S6a240352, S6a240353, S6a240354, S6a240271, S6a240375, S6a240376, S6a240377, S6a240238, S6a240355, S6a240163, S6a240073, S6a240251, S6a240356, S6a240091. Editorial changes by the rapporteur. | 1.1.0 |
| 2024-08 | SA6#62 |  |  |  |  | PCR agreed in SA6#62: S6-243544, S6-243545, S6-243546, S6-243547, S6-243027, S6-243747, S6-243651, S6-243553, S6-243555, S6-243556, S6-243174, S6-243558. Editorial changes by the rapporteur. | 1.2.0 |