| 3GPP TR 28.858 V0.2.0 (2024-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Artificial Intelligence / Machine Learning (AI/ML) management Phase 2 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	11

Introduction	12

1	Scope	13

2	References	13

3	Definitions of terms, symbols and abbreviations	13

3.1	Terms	13

3.2	Symbols	13

3.3	Abbreviations	13

4	Concepts and overview	14

4.1	Overview	14

4.2	Energy consumption of AI/ML	14

5	Management capabilities for AI/ML lifecycle	15

5.1	ML model training	15

5.1.1	ML-Knowledge-based Transfer Learning	15

5.1.1.1	Description	16

5.1.1.2	Use cases	17

5.1.1.2.1	Discovering sharable Knowledge	17

5.1.1.2.2	Knowledge sharing and transfer learning	17

5.1.1.3	Potential requirements	21

5.1.1.4	Possible solutions	22

5.1.1.5	Evaluation	23

5.1.2 	ML pre-training	23

5.1.2.1 	Description	23

5.1.2.2	Use cases	23

5.1.2.2.1	 Consumer requested ML pre-training	23

5.1.2.3	Potential requirements	23

5.1.2.5	Evaluation	24

5.1.3	ML Fine-tuning	24

5.1.3.1 	Description	24

5.1.3.2	Use cases	24

5.1.3.2.1	 ML fine-tuning for a pre-trained ML model	24

5.1.3.3	Potential requirements	24

5.1.3.4	Possible solutions	24

5.1.3.5	Evaluation	25

5.1.4	ML model training for multiple contexts	25

5.1.4.1	Description	25

5.1.4.2	Use cases	25

5.1.4.2.1	ML model training for multiple contexts	25

5.1.4.3	Potenital Requirements	25

5.1.4.4	Possible solutions	25

5.1.4.5	Evaluation	26

5.1.5	ML training data statistics	26

5.1.5.1	Description	26

5.1.5.2	Use Cases	26

5.1.5.2.1	 Training data statistical properties for ML training	26

5.1.5.3	Potential requirements	26

5.1.5.4	Possible solutions	26

5.1.5.5	Evaluation	27

5.1.6	ML model confidence	27

5.1.6.1	Description	27

5.1.6.2	 Use Cases	27

5.1.6.2.1	 Model Confidence Threshold in ML Training	27

5.1.6.3	Potential requirements	27

5.1.6.4	Possible solutions	27

5.1.6.5	Evaluation	28

5.1.7	Management of Reinforcement Learning	28

5.1.7.1	Description	28

5.1.7.2	Use cases	28

5.1.7.2.1	Exploration in Reinforcement Learning	28

5.1.7.3	Potential Requirements	28

5.1.7.4	Possible solutions	29

5.1.7.5	Evaluation	29

5.1.8 	Sustainable AI/ML	29

5.1.8.1 	Description	29

5.1.8.2	Use cases	29

5.1.8.2.1	AI/ML energy consumption evaluation and reporting for ML model training	29

5.1.8.3	Potential Requirements	29

5.1.8.4	Possible solutions	30

5.1.8.5	Evalaution	30

5.1.9	ML model distributed training	30

5.1.9.1	Description	30

5.1.9.2	Use cases	30

5.1.9.2.1	ML model distributed training	30

5.1.9.3	Potential requirements	30

5.1.9.4	Possible solutions	30

5.1.9.5	Evaluation	30

5.1.10	Management of Federated Learning	31

5.1.10.1	Description	31

5.1.10.2	Use cases	31

5.1.10.2.1	Management of different roles in Federated Learning	31

5.1.10.3	Potential requirements	32

5.1.10.4	Possible solutions	32

5.1.10.5	Evaluation	32

5.2	ML model testing	32

5.3	AI/ML inference emulation	32

5.3.1	ML inference emulation	32

5.3.1.1	Description	32

5.3.1.2	Use cases	32

5.3.1.2.1	AI/ML inference emulation	32

5.3.1.2.2	Managing ML inference emulation	33

5.3.1.3	Potential requirements	33

5.3.1.4	Possible solutions	34

5.3.1.5	Evaluation	35

5.3.2	ML emulation environment selection	35

5.3.2.1	Description	35

5.3.2.2	Use cases	35

5.3.2.2.1	ML emulation environment selection	35

5.3.2.3	Potential requirements	36

5.3.2.4	Possible solutions	36

5.3.2.5	Evaluation	36

5.4	AI/ML deployment	36

5.4.1	Enhance the ML model loading use case	36

5.4.2 	Managing ML Model Transfer in RAN	36

5.1.2.1 	Description	36

5.4.2.2	Use cases	36

5.4.2.2.1	Map ML Model Delivery in RAN to ML Model Loading in SA5	36

5.4.2.3	Potential Requirements	37

5.5	AI/ML inference	37

5.5.1	Coordination between the ML capabilities	37

5.5.1.1	Description	37

5.5.1.2	Use cases	37

5.5.1.2.1	 Alignment of the ML capability between 5GC/RAN and 3GPP management system	37

5.5.1.3	Potential requirements	37

5.5.1.4	Possible solutions	38

5.5.1.4.1	Possible solution #1	38

5.5.1.4.2	Evaluation	38

5.5.2 	Sustainable AI/ML	38

5.5.2.1 	Description	38

5.5.2.2	Use cases	39

5.5.2.2.1	AI/ML energy consumption evaluation and reporting for AI/ML inference	39

5.5.2.3	Potential requirements	39

5.5.2.4	Possible solutions	39

5.5.2.5	Evaluation	39

5.5.3	ML remedial action management	39

5.5.3.1	Description	39

5.5.3.2	Use cases	39

5.5.3.2.1	ML remedial actions due to performance degradation and energy consumption	39

5.5.3.3	Potential requirements	40

5.5.3.4	Possible solutions	40

5.5.3.5	Evaluation	40

5.5.4	Managing ML models in use in a live network	40

5.5.4.1	Description	40

5.5.4.2	Use Cases	40

5.5.4.2.1	Handling of underperforming ML trained models in live networks	40

5.5.4.2.2	 Performance monitoring of Network Functions with ML trained models in live networks	40

5.5.4.3	Potential requirements	40

5.5.4.4	Possible solutions	40

5.5.4.5	Evaluation	40

6	Conclusions and recommendations	40

Annex <A>: Change history	42

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document studies the enhanced Artificial Intelligence / Machine Learning (AI/ML) management capabilities and services building on the work of Rel-18, as well as new management aspects related to AI/ML functionalities across various 3GPP working groups within 5GS. It focuses particularly on the management of AI/ML-enabled features utilized in management and orchestration, 5GC, and NG-RAN. Furthermore, the study also investigates the AI/ML management capabilities supporting the emerging AI/ML technologies (e.g., Federated Learning and Generative AI) and the sustainability and trustworthiness of AI/ML in 5G system.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.105: Artificial Intelligence / Machine Learning (AI/ML) management".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 28.541: " Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Concepts and overview

## 4.1	Overview

AI/ML techniques have gained significant interests and advancements across various industries, including telecommunications, where they are now being applied to enhance mobile networks. While AI/ML technologies have matured considerably, certain aspects continue to evolve, and new techniques frequently emerge to complement existing methods.

AI/ML techniques can be characterized from several perspectives:

- Learning Methods: AI/ML employs various learning methods such as supervised, semi-supervised, unsupervised, and reinforcement learning. Each method is suited to specific inference categories (e.g., prediction) and requires distinct types of training data.

- Learning Complexity: AI/ML techniques vary in complexity, ranging from basic Machine Learning to more advanced e.g., Deep Learning with neural networks.

- Learning Architecture: Depending on the topology and location of learning tasks, AI/ML can be categorized into Generative AI, centralized, distributed, and federated learning.

- Learning Continuity: AI/ML can be implemented as online learning, offline learning or continual learning, depending on whether the learning process is static or ongoing.

In the 5GS, AI/ML capabilities are used across various domains, including management and orchestration, 5GC and NG-RAN. The performance of AI/ML inference function within 5GS depends on how well-trained ML models for accurate and efficient inference.

To effectively deploy AI/ML capabilities in 5GS, it is crucial to manage the ML models and AI/ML inference functions according to the specific characteristics and requirements of the use cases they support.

The study investigates enhancements of AI/ML management capabilities by building on the work from Rel-18 and exploring new management aspects across various 3GPP working groups. Additional advanced use cases are investigated, covering AI/ML management and operational capabilities to support different types of AI/ML technologies required for AI/ML in the 5G system (e.g. Federated Learning, Reinforcement Learning, online/offline learning, Generative AI and Distributed Learning):

Some primary aspects of AI/ML sustainability are studied, including the evaluation of energy and resource consumption and efficiency impacts associated with AI/ML features across all operational phases (training, emulation, deployment, inference).

The present TR also investigates management aspects (LCM CM and PM) of additional AI/ML functionalities defined by 3GPP SA WGs and RAN WGs.

## 4.2	Energy consumption of AI/ML

The energy consumption of AI/ML is to be considered according to various aspects, including:

- Multiplicative aspect: This depends on the phase of the AI/ML model lifecycle being considered (e.g., ML model training, ML model testing, AI/ML inference emulation, ML model deployment, AI/ML inference). For instance, AI/ML inference has a repetitive nature, whereas ML model training may be performed only once (though some re-training may be necessary). This multiplicative aspect can be measured by the number of (inference) cycles in each phase.

- Dataset volume: ML model training typically requires large sets of data to be collected, stored and processed, in contrast to AI/ML inference, which generally handles smaller data volumes. Dataset volumes can be estimated by the number of entries in the dataset (e.g. the number of gNBs) multiplied by the number of parameters that characterize each entry (e.g. the number of parameters collected per gNB). Dataset volumes are considerably larger during the ML model training phase than during the AI/ML inference phase.

- Computational complexity of each cycle: This can be measured by the number of operations (e.g. floating-point operations) performed during each cycle. The computational complexity may vary depending on the ML model’s complexity and software efficiency.

- Performance of the hardware platform: The performance during each phase of the AI/ML model lifecycle depends on the type of hardware used (e.g. GPUs vs. CPUs), as well as hardware’s performance characteristics and overall system design.

In contrast with AI/ML energy consumption, AI/ML may also help save energy, e.g. by optimizing the usage of network resources and consequently enabling network energy savings.

# 5	Management capabilities for AI/ML lifecycle

## 5.1	ML model training

### 5.1.1	ML-Knowledge-based Transfer Learning

Editor’s Note: The content in this clause may need to be aligned with the latest version of TS28.105 [2].

#### 5.1.1.1	Description

It is known that existing ML capability can be leveraged in producing or improving new or other ML capability. Specifically, using transfer learning knowledge contained in one or more ML models may be transferred to another ML model. Transfer learning relies on task and domain similarity to deduce whether some parts of a deployed ML model can be reused in another domain / task with some modifications.  aspects of transfer learning that are appropriate in multi-vendor environments need to be supported in network management systems. However, ML models are not to  be multi-vendor objects, i.e. it will not be possible to transfer an ML model from one function to another. Instead, the knowledge contained in the model, called ML knowledge, should be transferred instead of transferring the ML model itself.

ML knowledge represents the information (e.g., experience that indicates the recommended outputs given set of input data) gained by the MLModel through learning. This information can be in the form of (but not limited to) statistics (e.g., in a distribution) or a summary (e.g. in a table).

As an example, the knowledge contained in an ML model deployed to perform mobility optimization by day can be leveraged to produce a new ML model to perform mobility optimization by night. As such and as illustrated by figure 5.1.1.1-1, the network or its management system needs to have the required management services for ML Transfer Learning (MLKLT), where ML Transfer Learning refers to means to allow and support the usage and fulfilment of transfer learning between any two ML models.

Figure 5.1.1.1-1: ML Knowledge Transfer Learning (MLKLT) flow between the source MLKLT
(which is the model with the pre-trained ML model), the peer MLKLT
(which is the model that shall train a new ML model) and the MLKLT MnS consumer
(which may be the operator or another management function that wishes to trigger or control MLKLT)

#### 5.1.1.2	Use cases

##### 5.1.1.2.1	Discovering sharable Knowledge

For the transfer learning, it is expected that the source ML Knowledge Transfer Learning MnS producer shares its knowledge with the target ML Training function, either simply as single knowledge transfer instance or through an interactive transfer learning process. The concept of knowledge here represents any experiences or information gathered by the MLModel in the ML Knowledge Transfer Learning MnS producer through training, inference, updates, or testing. This information or experiences can be in the form of - but not limited to - data statistics or other features of the underlying ML model. It may also be the output of an MLModel. The 3GPP management systems should provide means for an MnS consumer to discover this potentially shareable knowledge as well as means for the provider of MLKLT to share the knowledge with the MnS consumer.

##### 5.1.1.2.2	Knowledge sharing and transfer learning

The transfer learning may be triggered by a MnS consumer either to fulfil the learning for itself or for it to be accomplished through another ML Training function. The model containing the knowledge may be an independent managed entity (the ML model). Alternatively, the ML model may also be an entity that is not independently managed but is an attribute of a managed ML model or ML function in which case MLKLT does not involve sharing the ML model or parts thereof but may imply implementing the means and services to enable the sharing of knowledge contained within the ML model or ML-enabled function. The 3GPP management system should provide means and the related services needed to realize the ML transfer learning process.

Specifically, the 3GPP management system should provide means for an MnS consumer to request and receive sharable knowledge as well as means for the provider of MLKLT to share the knowledge with the MnS consumer or any stated target ML Training function. Similarly, the 3GPP management system should provide means for an MnS consumer to manage and control the MLKLT process and the related requests associated with transfer learning between two ML models or between the two ML models and a shared knowledge repository.

The two use cases should address the three scenarios represented by figures 5.1.1.2.2-1 to 5.1.1.2.2-4. Note that, the use case and requirements here focus on the required management capabilities. The implementation of the knowledge transfer learning processes are implementation details that are out of the scope of the present document.

Figure 5.1.1.2.2-1: Scenario 1 - Interactions for ML-Knowledge Transfer Learning (MLKLT) to
support training at the ML knowledge Transfer MnS consumer -
the ML knowledge Transfer MnS consumer obtains the ML knowledge
which it then uses for training the new ML model based on knowledge received
from the MLKLT source MnS producer

Figure 5.1.1.2.2-2: Scenario 2 - interactions for ML-Knowledge Transfer Learning (MLKTL) to
support training at the ML knowledge transfer MnS consumer triggered by the MLKTL Source -
the ML Transfer Learning MnS consumer acting as the MLKTLSource
(the source of the ML knowledge) triggers the training at the ML knowledge Transfer MnS consumer by providing the ML knowledge to be used for the training,
the ML Transfer Learning MnS consumer then undertakes the training

Figure 5.1.1.2.2-3: Scenario 3 - interactions for ML-Knowledge Transfer Learning (MLKLT) to
support training at the Peer ML knowledge Transfer MnS producer who is different from the
ML knowledge Transfer MnS consumer - the ML knowledge Transfer MnS consumer triggers
training at the MLKLT peer MnS producer. The MLKLT MnS consumer then obtains the
ML knowledge from the MLKLT source MnS producer and then uses the knowledge for training
the new ML model based on knowledge received from the MLKLT source MnS producer

Figure 5.1.1.2.2-4: Scenario 4 - interactions for ML-Knowledge Transfer Learning (MLKLT) to
support training at the Source ML knowledge Transfer MnS producer -
the ML knowledge Transfer MnS consumer triggers training at the MLKLT source MnS producer.
The MLKLT MnS consumer then obtains the ML knowledge from the MLKLT source MnS producer
and then uses the knowledge for training the new ML model based on knowledge received from
the MLKLT source MnS producer

#### 5.1.1.3	Potential requirements

REQ-MLKLT-1: The 3GPP management system should have a capability enabling an authorized MnS consumer to discover the available shared knowledge from a given MLKLT MnS producer according to a stated set of criteria.

REQ-MLKLT-2: The 3GPP management system should have a capability enabling an authorized MnS consumer to request a MLKLT MnS producer to provide some or all the knowledge available for sharing according to some stated criteria.

REQ-MLKLT-3: The 3GPP management system should have a capability for a MLKLT MnS producer to report to an authorized MnS consumer on the available shared knowledge according to a ReportingCriteria specified in a request for information on available Knowledge.

REQ-MLKLT-4: The 3GPP management system should have a capability enabling an authorized MnS consumer to request a MLKLT MnS producer to trigger and execute a transfer learning instance to a specified ML model or ML-enabled function.

REQ-MLKLT-5: The 3GPP management system should have a capability for an authorized MnS consumer (e.g. an operator or the function/entity that generated the request for available Knowledge or for information thereon) to manage the request for knowledge or its information and subsequent process, e.g. to suspend, re-activate or cancel the MLKnowledgeRequest; or to adjust the description of the desired knowledge.

REQ-MLKLT-6: The 3GPP management system should have a capability for an authorized MnS consumer (e.g. an operator or the function/entity that generated the request for MLKLT) to manage or control a specific MLKLTJob, e.g. to start, suspend or restart the MLKLTJob; or to adjust the transfer learning conditions or characteristics i.e. Modify MLKLTJob attributes.

Note: the MLKLTJob represents the process of knowledge-based trasfer learning.

REQ-MLKLT-7: The 3GPP management system should have a capability enabling an MLModel to register available knowledge to a shared knowledge repository, e.g. through a MLKnowledgeRegistration process.

REQ-MLKLT-8: The 3GPP management system should have a capability enabling KnowledgeRepo to act as the MLKLT MnS Producer to enable an authorized MnS consumer to request the shared knowledge repository to provide information on the available knowledge according to some given criteria.

REQ-MLKLT-9: The 3GPP management system should have a capability enabling KnowledgeRepo to act as the MLKLT MnS Producer to enable an authorized MnS consumer to request the KnowledgeRepo to provide some or all the knowledge available for sharing according to some given criteria.

REQ-MLKLT-10: The 3GPP management system should have a capability enabling KnowledgeRepo to act as the MLKLT MnS Producer to enable an authorized MnS consumer (e.g. an operator or the function/entity that generated the MLKnowledgeRequest) to manage the request, e.g. to suspend, re-activate or cancel the MLKnowledgeRequest ; or to adjust the description of the desired knowledge.

REQ-MLKLT-11: The 3GPP management system should have a capability enabling KnowledgeRepo to act as the MLKLT MnS Producer to enable an authorized MnS consumer (e.g. an operator) to manage or control a specific MLKLTJob, e.g. to start, suspend or restart the MLKLTJob; or to adjust the transfer learning conditions or characteristics.

#### 5.1.1.4	Possible solutions

Discovering sharable Knowledge

To discover sharable knowledge:

-	The MnS consumer may send a request to the MLKLT MnS producer to provide information on the available sharable knowledge. In other words, the MLKLT MnS producer receives a request to report on the available sharable knowledge.

-	The request may be generic or may state a set of criteria which the knowledge should fulfil.

-	The request may be referred to as MLKnowledgeInfoRequest.

-	The MLKnowledgeInfoRequest must have informational description (Metadata description) of the task and domain related to the required knowledge or given a network problem.

-	An ML model or a function containing an ML model may register its available knowledge to a shared knowledge repository, e.g. through a MLKnowledgeRegistration process.

-	The MLKnowledgeRegistration must contain informational description (Metadata description) of the task and domain related to the registered knowledge or suitable network problem.

Knowledge sharing and transfer learning

To share knowledge:

-	Introduce an IOC for an ML Knowledge request. The MnS consumer may send a request to the MLKLT MnS producer to share a specific kind of knowledge. i.e. the MLKLT MnS producer receives a request to provide sharable knowledge, The request may be referred to as MLKnowledgeRequest.

-	Introduce an IOC for an ML transfer learning process or job which is instantiated for any request for transfer learning or ML knowledge transfer. The MLKLT MnS producer instantiates a ML transfer learning process. The process may be referred to as MLKLTJob.

-	The MLKLTJob is responsible for adapting the required knowledge into a shareable format with the MLKLT consumer.

-	MLKLTJob may be a continuous process where knowledge is shared with the MLKLT consumer frequently to account for updates in the knowledge.

NOTE:	It may also be the case that the consumer directly instantiates the MLKLTJob without a separate request.

#### 5.1.1.5	Evaluation

The solution described in clause 5.1.1.4 proposes simple information objects that can enable ML functions to exchange their knowledge to be used towards transfer learning but in a way that enables the vendor specific aspects of the ML models not to be exposed. Therefore, the solution described in clause 5.1.1.4 is a feasible solution to be developed further in the normative specifications.

### 5.1.2 	ML pre-training

#### 5.1.2.1 	Description

A pre-training is an ML model training for training an ML model with domain related dataset that covers more than one type of inference. For example, an ML model could be pre-trained with the dataset from SLS analysis capability group covering type of inference including ServiceExperienceAnalysis, NetworkSliceThroughputAnalysis, NetworkSliceTrafficAnalysis, NetworkSliceLoadAnalysis and E2ElatencyAnalysis [see TS 28.104 [3].

The pre-training is not intended to support a specific aIMLInferenceName but rather focuses on commonality among  number of use cases.

The ML model training process defined in TS 28.105 [2] covers pre-training.

#### 5.1.2.2	Use cases

##### 5.1.2.2.1		Consumer requested ML pre-training

The pre-training type follows the common procedure of ML model training, however the information in training request is different.

#### 5.1.2.3	Potential requirements

REQ-ML_ TRAIN -MLPT-1: The ML training MnS producer shall have a capability to enable an authorized consumer to request a pre-training of an ML model.

Editor note: It is for further discussion if the same requirements in existing TS 28.105 [2] Table 6.2a.1.3-1 related to training function also applies for pre-training.

#### 5.1.2.4	Possible solutions

##### 5.1.2.4.1	Possible solution #1

The exsting MLTrainingRequest IOC and MLModel IOC can be enhanced to support pre-traning.

Enhancement on MLTraingRequest IOC:  Since the exsting MLTrainingRequest IOC cannot support consumer requested ML pre-training, it is proposed to introduce following attributes into MLTrainingRequest IOC:

- PreTrainingIndication, indicates that the request model is a Pre-training Model. Besides, change the support qualifier of “aIMLInferenceName” to CM, where the condition is that the ML Model requested to train is not a pre-training ML Model.

- PotentialInferenceScope, indicates a set of types of inference, which may include a list of supported aIMLInferenceName.

Enhancement on MLModel IOC:  Since the pre-training is not aimed to support a specific inference capability but focuses on commonality in use cases, it’s proposed to introduce following attributes:

- PreTrainingModelIndication, indicates the ML Model is a Pre-training Model. Besides, the support qualifier of “aIMLInferenceName” should be changed to CM, which means this attribute should be present when the MLModel MOI represents a Pre-training Model.

- PotentialInferenceScope. Definition is documented above.

#### 5.1.2.5	Evaluation

### 5.1.3	ML Fine-tuning

#### 5.1.3.1 	Description

ML fine-tuning refers to training an already trained ML model (e.g. a pre-trained ML model) with a changed scope.

Re-training of an ML Model is not based on a specific type of inference, while fine-tuning of a pre-trained model means that the ML model will be trained with focus on some specific types of inference.

ML Model training process defined in TS 28.105 covers ML fine-tuning.

#### 5.1.3.2	Use cases

##### 5.1.3.2.1		ML fine-tuning for a pre-trained ML model

An ML model can be trained by dataset that covers more than one type of inference, which the pre-trained model could cover. The pre-trained model can be fine-tuned for specific types of inference by the producer.

NOTE: It is for FFS whether fine-tuning process could be applied for the same producer who pre-trained the model or for another producer.

#### 5.1.3.3	Potential requirements

REQ-ML_ TRAIN-MLFT-1: The ML training MnS producer shall have a capability to enable an authorized consumer to request the fine-tuning of a pre-trained ML model.

Editor’s note: It is for further discussion if the same requirements in existing TS 28.105 Table 6.2a.1.3-1 [2]  related to training function also applies for fine-tuning.

#### 5.1.3.4	Possible solutions

The exsting MLModel IOC and MLTrainingRequest IOC can be enhanced to support fine-tuning.

Enhancement Aspects on MLTrainingRequest IOC:  Since the exsting MLTrainingRequest IOC cannot support consumer requested ML fine-tuning, both following enhancements are possible.

- FineTuningIndication, indicates the consumer requested ML fine-tuning. The support qualifier should be CM, which means this attribute should be present when the MLTrainingRequest MOI represents the request for ML Model Fine-tuning. Besides, change the support qualifier of “aIMLInferenceName” to CM, which means that this attribute should be present when the MLTrainingRequest MOI does not represent the request for ML Model Fine-tuning.

Enhancement Aspects on MLModel IOC:  Since fine-tuning is aiming at a specific inference type for a pre-trained model, the AIML capabilities would be narrowed  which has the best performance. It is proposed to introduce following attributes:

- FinetuningModelIndication, indicates that the ML Model is a Finetuning Model. Besides, change the support qualifier of “aIMLInferenceName” to CM, where the condition is that the ML Model requested to train is not a pre-training ML Model.

- PotentialInferenceScope, indicates a set of types of inference, which may include a list of supported aIMLInferenceName.

#### 5.1.3.5	Evaluation

### 5.1.4	ML model training for multiple contexts

#### 5.1.4.1	Description

ML models can be trained for similar contexts allowing for efficient management of these ML models. If an ML model that is trained for a particular context needs to be updated, it can be realized using an ML model that was trained for different context as the baseline.

#### 5.1.4.2	Use cases

##### 5.1.4.2.1	ML model training for multiple contexts

Although the ML model may provide an AI/ML inference service for multiple scenarios, there are similarities in the contexts where ML models operate and perform AI/ML inferences. For e.g., two ML model instances for the same inference type in urban or rural areas would have significant overlap in their contexts. The context similarity can be leveraged in forming a cluster of ML models, where ML model instances in the cluster are either trained from the same previously trained ML model or from an ML model previously trained for another similar context as the baseline. The training of an ML model for multiple contexts allows for efficiency by cluster training rather than individually training each one of them. ML training needs to support the capability to train the cluster of ML models from the same pre-trained ML model or from an ML model previously trained for another similar context as the baseline. As input to the training, the different contexts and clustering criteria needed to distinguish the ML model instances may be provided by the MnS consumer.

In the case of degradation of ML models, updating of ML models is expected to be triggered. For ML models created through cluster training, the retraining of a degraded ML model could be triggered to start from another member of the cluster, i.e. start from an ML model with another context to create a new ML model with the desired context.

#### 5.1.4.3	Potenital Requirements

REQ-ML_CLUSTER-TRAIN-1: The MLT MnS producer should have a capability for an authorized MnS consumer to request training, update or loading of a cluster of ML models associated to a set of multiple contexts from a previously trained ML model.

#### 5.1.4.4	Possible solutions

The  MLTrainingRequest IOC has to be extended with:

- a flag indicating that a cluster of ML models has to be trained.

- criteria to form the cluster, e.g., clusteringCriteria. This may indicate context similarities, e.g., similar geographical location, training data similarities etc.

- the DN of the ML instance to be used as the baseline to train another ML model with a related context.

- the expectedRunTimeContext can be used to capture the context of new ML models that should be trained.

Note: The solution for updating or loading a cluster of ML models associated to a set of multiple contexts from a previously trained ML model is FFS.

#### 5.1.4.5	Evaluation

### 5.1.5	ML training data statistics

#### 5.1.5.1	Description

During ML training, it is important to ensure that training data is as uniform and representative as possible, and outliers are handled appropriately during the data preprocessing so as to train robust ML models that do not require frequent re-training.

#### 5.1.5.2	Use Cases

##### 5.1.5.2.1		Training data statistical properties for ML training

Non-uniform distribution of training data can significantly degrade the performance of trained ML models. The ML model may learn and reflect this non-uniformity, rather than the true underlying patterns in the data. This could lead to inaccurate predictions when the ML model is deployed for inference. Furthermore, an ML model trained on non-uniform data may not generalize well to new unseen data, as the training data might not accurately represent the full range of possible inputs the ML model may encounter during inference. This can limit the usefulness of the ML model.

Similarly, outliers in training data can significantly degrade the performance of trained ML model. ML models learn to make predictions based on the patterns they identify in the training data. Outliers can skew these patterns and lead to an ML model that is biased towards these extreme values, rather than accurately reflecting the majority of the data.

Therefore, it is crucial to ensure that training data is as uniform and representative as possible, and outliers are handled appropriately during the data preprocessing so as to train robust ML models that do not require frequent re-training.

#### 5.1.5.3	Potential requirements

REQ-DATA-STAT-1: The 3GPP management system should enable an authorized consumer to provide information on the training dataset distribution that the ML Training MnS producer should take into account for training an ML model.

REQ-DATA-STAT-2: The 3GPP management system should enable an authorized consumer to provide information on the usage of outliers in the training dataset that the ML Training MnS producer should take into account for training an ML model.

#### 5.1.5.4	Possible solutions

1) Introduce a new attribute of type, e.g., datasetStatisticalProperties, in the MLTrainingRequest IoC requested by an authorized MnS Consumer to the ML Training MnS Producer. This information influences the training data selection mechanisms to be used for training an ML model, assuring that the trained ML model reflects the statistical characteristics of the data that are relevant for the aIMLinferenceName. The proposed datasetStatisticalProperties datatype may include the following two attributes:

- UniformlyDistributedTrainingData - It indicates the need for using training data that are uniformly distributed according to the different aspects of the aIMLinferenceName. The attribute is of type Boolean. For e.g., in case of coverage problem analytics, data samples for gNBs belonging to each geographical location area needs to be uniformly distributed for the ML model to correctly determine in which geographical location areas the coverage problem occurred during inference.

- TrainingDataWithOrWithoutOutliers – It indicates that the training data samples should consider or disregard data samples that are at the extreme boundaries of the value range. The attribute is of type Boolean. For e.g., in case of coverage problem analytics, if there are only say few training data samples belonging to a particular geographical location, they may be considered as outliers w.r.t. the other training data samples..

#### 5.1.5.5	Evaluation

The solution described in clause 5.1.5.4 proposes the addition of two new attributes to the MLTrainingRequest IoC to enable the MnS consumer to indicate the training data statistical properties allowed to be used by the MnS producer for training an ML model. Therefore, the solution described in clause 5.1.5.4 is a feasible solution to be developed further in the normative specifications.

### 5.1.6	ML model confidence

#### 5.1.6.1	Description

Confidence is a measure of the probability that a prediction made by the ML model is correct. It is often expressed as a percentage, with 100% being absolute certainty and 0% being absolute uncertainty. Confidence score can be useful in prediction-making processes, where a certain level of confidence might be required before taking action based on the ML model’s prediction. However, it is important to note that a high confidence score does not guarantee the the ML model’s prediction is correct, but it only indicates the ML model belief in its prediction. In simpler terms, confidence is a measure of the ML model’s certainty about its prediction. It is not a measure of the ML model’s accuracy or correctness. An ML model can be very confident about a prediction and still be wrong. Conversely, an ML model can be less confident about a prediction and still be correct.

#### 5.1.6.2		Use Cases

##### 5.1.6.2.1		Model Confidence Threshold in ML Training

In the ML training phase, the ML training function may support the capability to evaluate the confidence value of the trained ML model. The ML model confidence value indicates the average confidence value (in unit of percentage) that the ML model would perform for data with the same distribution as training data. Essentially, this is a measure of degree of the convergence of the trained ML model. Therefore, the MnS consumer may prefer to indicate the confidence threshold value that the trained ML model should satisfy before the ML model is deployed for inference.

#### 5.1.6.3	Potential requirements

REQ-MODEL_CONF-1: The AI/ML training MnS producer should have a capability allowing an authorized MnS consumer to indicate the confidence threshold that the AI/ML MnS producer should meet on the trained ML model.

#### 5.1.6.4	Possible solutions

Introduce a new attribute, e.g., modelConfidenceIndication, in the MLTrainingRequest IoC requested by an authorized MnS consumer to the ML Training MnS Producer. This attribute indicates to the ML Training MnS Producer the minimum average confidence value (in unit of percentage) that the ML model should meet on the data with the same distribution as training data. If the minimum average confidence value cannot be satisfied, then the ML Training MnS Producer should retrain the ML model. The attribute is of type real with allowed values {0…100}.

#### 5.1.6.5	Evaluation

### 5.1.7	Management of Reinforcement Learning

#### 5.1.7.1	Description

Reinforcement Learning (RL) is a type of machine learning where an agent learns to make decisions by taking actions in an environment to achieve a goal. The agent learns from the consequences of its actions, rather than from being explicitly taught and it selects its actions based on its past experiences (exploitation) and also by new choices (exploration), which is essentially a trial-and-error approach. In RL, an agent interacts with the environment in discrete time steps. At each time step, the agent receives the current state of the environment and selects an action. The environment, in turn, provides a reward and the new state. As illustrated in Figure 5.1.6.1, suppose RL is applied for coverage problem analysis use case, the RL agent actions are used to optimize the coverage problem and the RL environment can be the simulation environment. The actions can be changes to the values of network adjustable parameters (e.g. change the transmission power of the NR sector carrier, see TS 28.104 [3]). The states can be the network PMs/KPIs like RSRP distribution etc. The reward can be the score of an RL performance metric to evaluate the PMs/KPIs. The goal of the agent is to learn a policy, which tells it what action to take under what circumstances, that maximizes the sum of rewards. The main advantage of this approach is in the ability to automatically adapt to the characteristics of the environment, making it suitable approach for handling dynamic environments such as mobile networks.

Figure 5.1.6.1: Reinforcement Learning in Domain Management Function

#### 5.1.7.2	Use cases

##### 5.1.7.2.1	Exploration in Reinforcement Learning

Reinforcement Learning (RL) has the ability to learn and adapt itself to dynamic environments and thus finds the near optimal solution to the problem. This makes the RL-based approaches very interesting for applications in the mobile networks. However, the potential negative impact to the mobile network caused by RL is still the main drawback. In particular, during the exploration step performing trials and learning from errors may have an impact on the operational network and may result in unsafe operations causing network performance degradations. Therefore, the exploration step in RL needs to be under a controlled environment that are not supposed to violate system performance requirements. If the RL agent behaves in an unexpected manner, there needs to be a set of fall-back actions in place, e.g., to switch from RL-based solution to non-RL-based solution.

For RL management, the MnS consumer can query the ML Training MnS producer to discover if RL is supported. When RL is supported, a consumer may want to provide a scope (e.g., geographical area, time window) that can aid the producer to select/create the environment when performing RL. In the event RL is supported, the consumer may also want to state their preference for environment type for RL during training i.e. simulated environment or real network.

NOTE:	Support for both environment types can be considered optional in the RL training.

#### 5.1.7.3	Potential Requirements

REQ-RL_MGMT-01: The ML training MnS producer should have a capability allowing an authorized MnS consumer to query if RL training is supported.

REQ-RL_MGMT-02: The ML training MnS producer should have a capability allowing an authorized MnS consumer to specify the preferred RL environment type.

REQ-RL_MGMT-03: The ML training MnS producer should have a capability to allow an authorized MnS consumer to specify the preferred RL environment scope

#### 5.1.7.4	Possible solutions

### 5.1.7.5	Evaluation

### 5.1.8 	Sustainable AI/ML

#### 5.1.8.1 	Description

Sustainable AI/ML aims to reduce energy usage in the ML model training step. It is important to investigate effective methods for evaluating AI/ML energy consumption and efficiency for ML model training which will aid in decreasing energy consumption, enhancing energy efficiency, and promoting sustainable development of AI/ML in 5G systems.

#### 5.1.8.2	Use cases

##### 5.1.8.2.1	AI/ML energy consumption evaluation and reporting for ML model training

AI/ML energy consumption is a significant concern, especially as ML models become more complex and data-intensive. The ML model training is typically the most energy-intensive part since it involves processing large datasets and performing numerous calculations to adjust the ML model parameters.

The MnS consumer may have concern on the energy consumption for ML model training and the MnS Producer should report the related information. MnS Consumer may ask the MnS Producer to report the energy consumption of training each ML model. To satisfy such query request from the MnS consumer, methods should be investigated to measure AIML energy consumption. Then, MnS Consumer can use the energy consumption information and compare with the network performance gain due to ML model to evaluate the total benefit of using the Ml model.

Regarding the various AI/ML energy consumption aspects outlined in clause 4.X, ML model training may be characterized by several factors including:

- Number of cycles, a low number of cycles generally contributes to reduced energy consumption,

- Dataset size, large datasets generally contribute to  increased energy consumption due to intensive data processing,

- Number of floating-point operations, complex ML model contributes to increased energy consumption,

- Deployment on high-end hardware platform, which can either increase energy consumption due to

their substantial power requirements, or reduce it by offering higher energy efficiency through

optimized performance.

#### 5.1.8.3	Potential Requirements

REQ-AIML_TRAIN_ENERGY_MGT-01: The MLT MnS producer should have a capability to allow an authorized MnS consumer to query energy consumption pertaining to training each ML model.

REQ-AIML_TRAIN_ENERGY_MGT-02: The MLT MnS producer should have a capability to report the energy consumption pertaining to training each ML model to an authorized consumer.

#### 5.1.8.4	Possible solutions

The IoC MLTrainingReport needs to be enhanced with a new attribute related to energy consumption for training each ML model, e.g., modelEnergyConsumptionTraining. This attribute is of type real and indicates the energy consumption value pertaining to the ML model training, e.g., “KWh”.

Editor’s Note: The method to determine energy consumption for ML model training is FFS. If energy consumption for ML model training cannot be determined, the proposed solution in 5.1.8.4 becomes redundant.

#### 5.1.8.5	Evalaution

### 5.1.9	ML model distributed training

#### 5.1.9.1	Description

Distributed training is a model training paradigm that involves spreading training workload across multiple training functions, to accelerate the training process and/or reduce the required computational resources. Distributed training can be used for traditional machine learning models, as well as for large models.

In 5GS, the ML training function may be located within the management system or in the NF (e.g. gNB or NWDAF), i.e. the worker node for training. Each node has different computing resources and storage capacity based on physical infrastructure such as CPU/GPU/DPU, memory, storage, and network bandwidth. In order to obtain load balance between nodes and maximize the efficiency of resource utilization, splitting up the training may be necessary and involving multiple training functions according to the actual situation of nodes may be needed. Thus, aspects of distributed training need to be supported in the management systems.

#### 5.1.9.2	Use cases

##### 5.1.9.2.1	ML model distributed training

In 5GS, the MnS consumer may require to speed up the training process under certain conditions (e.g. the size of the model may be too large for a single training function).

When receiving an ML training request, the MLT MnS producer may evaluate whether distributed training is needed according to the training requirements provided by the ML training consumer, and it is up to the MLT MnS producer  to determine appropriate training function(s) which need to participate in the ML model training. Collaboration and mutual agreement may require between distributed ML training functions.

NOTE 1:	How to split the ML model and synchronize the parameters in different training function depends on the distributed algorithm which are proprietary and not in scope for standardization.

NOTE 2: The data exchange between different training functions should be in the security tunnel with appropriate authentication and authorization mechanisms.

#### 5.1.9.3	Potential requirements

REQ-ML_DIST-TRNG-01:	The ML training MnS producer should have a capability allowing the authorized consumer to provide distributed training requirements to the MnS Producer.

#### 5.1.9.4	Possible solutions

TBD

#### 5.1.9.5	Evaluation

TBD

#### 5.1.10	Management of Federated Learning

#### 5.1.10.1	Description

Federated Learning (FL) is a distributed machine learning approach that allows multiple FL clients to collaboratively train an ML model on local datasets contained in each FL client without explicitly exchanging data samples.

FL is supported by a group of FL clients and FL server wherein FL client keeps the data localized and private, and trains the ML model directly on the local nodes (client) where the data is generated or stored.

Federated learning can be categorized into two main types: Horizontal federated learning (HFL) and Vertical federated learning (VFL), based on the nature of the data distribution and the way the model training is orchestrated among participants. For HFL, the process typically includes FL Client discovery and selection, and ML model distribution and aggregation.

-

Figure 5.1.10.1-1: ML model distribution and aggregation for HFL

NOTE: 	A prior agreement needs to exist between the FL server and the FL clients to exchange ML models.

#### 5.1.10.2	Use cases

##### 5.1.10.2.1	Management of different roles in Federated Learning

For FL, an ML model is collaboratively trained by a group of Fl clients (e.g., MTLF in NWDAFs) including one acting as FL server and the others acting as FL clients. Federated Learning training allows multiple FL clients to collaboratively train an ML model on local datasets, it means that the local training of each FL client needs to start and complete almost at the same time to ensure the performance of ML model aggregation performing in FL server.

For managing the FL, the ML training MnS consumer needs to know the FL clients and FL server involved in the FL, so that the consumer understands the impact of each one of them and can manage it correspondingly.

When receiving an ML Training request, the ML Training MnS Producer should evaluate whether FL process needs to be started according to the training requirements (e.g., minimal number of FL Clients, minimal number of total iterations, minimal number of data samples and available time of the FL Clients) provided by the ML training consumer. Based on the received requirements, the FL server may select appropriate FL Clients.

To evaluate the performance of FL, the consumer needs to know the performance of the final global ML model running on the local training data set of participating FL clients. For instance, if an FL server cannot generate a global ML model with satisfied performance for the FL clients, the consumer may interact with the MnS ML training producer to optimize the FL for future training, e.g., updating the criteria for selecting FL clients.

In addition, the consumer needs to get the information about the contribution of each FL client to the FL process, for instance, number of iterations the FL client participated in the FL, number of data examples the FL client used, training duration the FL Client performed.

#### 5.1.10.3	Potential requirements

REQ-FL_MGMT-01: The ML training MnS producer should have a capability allowing an authorized consumer to discover the FL roles (FL server or FL client) in Federated Learning.

REQ-FL_MGMT-02: The ML training MnS producer should have a capability allowing an authorized consumer to provide FL training requirements to the MnS Producer.

REQ-FL_MGMT-03: The ML training MnS producer should have a capability allowing an authorized consumer to provide requirements for selecting FL clients in Federated Learning to the MnS Producer.

REQ-FL_MGMT-04: The ML training MnS producer should have a capability allowing an authorized consumer to get the performance of ML model on each participating FL client.

REQ-FL_MGMT-05: The ML training MnS producer should have a capability to report the information about the contribution of each FL client to the FL process to MnS consumer.

#### 5.1.10.4	Possible solutions

#### 5.1.10.5	Evaluation

## 5.2	ML model testing

## 5.3	AI/ML inference emulation

### 5.3.1	ML inference emulation

Editor’s Note: The content in this clause may need to be aligned with the latest version of TS28.105 [2].

#### 5.3.1.1	Description

A trained ML model can be used for inference within the stated scope e.g. on a managed function or in a management function. Accordingly, there may be an AI/ML inference MnS producer that is responsible for executing the inference.

#### 5.3.1.2	Use cases

##### 5.3.1.2.1	AI/ML inference emulation

After an ML model is trained, validation is done to ensure the training process is completed successfully. Typically, validation is done by preserving part of the training data set and using it after training to check whether the ML model has been trained correctly or not. However, even after the ML model is validated during development, inference emulation is necessary to check if the ML model containing the ML model is working correctly under certain runtime context or using certain inference emulation data set. In principle, the two operations are similar on a functional level, where both of them check the ML performance against given context or data to ensure the ML functionality is functioning correctly. But inference emulation involves interaction with third parties, e.g. the operators who use the ML model or third-party systems that may rely on the results computed by the ML model. For these reasons, it is necessary to support inference emulation, specifically to support means:

-	For a given MnS consumer to request for a specific AI/ML capability to be executed in ML inference emulator environment.

-	For a given MnS consumer to request a specific ML inference emulator to execute a given AI/ML capability.

-	For a managed function to act as a ML inference emulator and execute AI/ML capabilities in a controlled way.

The network or its management system needs to have the capabilities and provide the services needed to enable the MnS consumer to request inference emulation and receive feedback on the inference emulation of a specific ML model or of an application or function that contains an ML model.

##### 5.3.1.2.2	Managing ML inference emulation

The 3GPP management system may have resources for multiple emulation environments to be used depending on need. These may include simulation environments, a digital twin of the network, a test network or the real network under curtain constrained conditions, e.g. for a selected set of UEs. The multiple emulation environments may represent different levels of trust that the operator or management system has in the ML model or AI/ML inference functions. Correspondingly, 3GPP management system needs to have means and method for Orchestrating the inference emulation i.e. say called the inference emulation orchestrator or inference emulation function. Accordingly:

-	the emulation progression process involves choosing the right type and instance of an emulation environment to which an ML model, AI/ML inference function or the action thereof may be tested depending on the needs of the function to be tested and the available emulation environments and their resources;

-	the emulation process may also involve executing the ML model, AI/ML inference function or its action on the real network but in a controlled fashion, e.g. only within certain hours or only on cells with a particular kind of load or only on cells in a particular area or in limited subscriber groups.

Relatedly, the actions taken by the inference emulation function may include:

-	Controlling the allowed parameter space/ranges of the parameters optimized by the ML model or AI/ML inference function depending on the emulation environment to which the ML model, AI/ML inference function or the actions are being executed.

-	Adjusting the parameter space in consideration of the observed behaviour of the ML  model or AI/ML inference function.

-	Deploying the actions of the ML model or AI/ML inference function on a selected emulation environment or on the real network.

-	Blocking the ML model or AI/ML inference function from being used on the network.

#### 5.3.1.3	Potential requirements

REQ-AI/ML_EMUL-1: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to query the available emulation environment(s).

REQ-AI/ML_EMUL-2: The MnS producer for AI/ML inference emulation should have a capability to inform an authorized MnS consumer of the available emulation environment(s).

REQ-AI/ML_EMUL-3: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to request an ML inference emulation for a specific ML model or models.

REQ-AI/ML_EMUL-4: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to request for ML Inference Emulation for a specific ML model using specified data or data with specifically stated characteristics and inference emulation features.

REQ-AI/ML_EMUL-5: The MnS producer for AI/ML inference emulation should have a capability to inform authorized MnS consumer about the status of the emulation of an ML model under emulation.

REQ-AI/ML_EMUL-6: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer (e.g. an operator) to manage or control a specific ML inference emulation process, e.g. to start, suspend or restart the inference emulation; or to adjust the inference emulation conditions or characteristics.

REQ-AI/ML_EMUL-7: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to request reporting, and receive reports on the progress and outcome of an emulation process.

REQ-AI/ML_EMUL-8: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to configure an ML model or AI/ML inference function supporting with the level of trust that expresses the degree to which the ML model or AI/ML inference function or the different action thereof have been confirmed as trusted.

REQ-AI/ML_EMUL-9: The MnS producer for AI/ML inference emulation should have a capability to graduate an ML model, AI/ML inference function or the different action thereof through different levels of trust each expressing a different degree to which the ML model, AI/ML inference function or action has been confirmed as trusted.

#### 5.3.1.4	Possible solutions

1)	Introduce an IOC with the properties of the ML inference emulation function. This may be termed as an MLInferenceEmulationFunction to be name-contained in either a Subnetwork, a ManagedFunction or a ManagementFunction. The MLInferenceEmulationFunction may be a separate function or may be name-contained in an inference function.

This IOC contains attributes including the following:

-	list of the ML models which can be emulated and possibly in which emulation environments;

-	indication of progression of the MLModel or AI/ML inference function that is under emulation to indicate the degree to which the inference has been emulated and trusted;

-	different characteristics for which different emulations may be supported;

-	a hierarchy for different emulation environments, e.g. an emulator that uses only a test network vs. an emulator that activates the actions in the real network at specified time such as the maintenance window.

2)	Introduce an IOC representing an available emulation environment, e.g. a new IOC named as AvailableEmulationEnvironment, or EmulationSubNetwork, or the existing SubNetwork IOC with an attribute indicating it is a subnetwork used for emulation.

The instance of this IOC is created by the MnS producer to allow the consumer to query, or be informed of, the information of the available emulation environments. This IOC contains the following properties (e.g. the subordinated IOC or attributes):

-	inference functions or ML models under emulation.

3)	Introduce an IOC for the request for ML inference emulation which shall capture the consumer's requirements for inference emulation. This may be named as an MLInferenceEmulationRequest to be name-contained by the MLInferenceEmulationFunction.

The MLInferenceEmulationRequest shall be associated with at least 1 MLModel for which the inference emulation is being executed.

This IOC contains attributes including the following:

-	identifier of the ML models requested for emulation;

-	identifier of the selected emulation environment;

-	time window for the emulation.

Each MLInferenceEmulationRequest may have a RequestStatus field that is used to track the status of the specific MLInferenceEmulationRequest or the associated MLInferenceEmulationProcess. The RequestStatus is an enumeration with the possible values as: "Pending" if no action has been taken for the request; "Triggered" when an MLInferenceEmulationProcess has been instantiated; "Suspended" when the request or its job has been suspended by MnS consumer or producer and "Served" when the job has run to completion.

4)	Introduce an IOC for the process of ML inference emulation from the objects for inference emulation shall be instantiated. This may be named as an MLInferenceEmulationProcess to be name-contained by the MLInferenceEmulationfunction.

The MLInferenceEmulationProcess shall be associated with at least one MLModel for which the inference emulation is being executed.

This IOC contains the following attributes:

-	progress indicator;

-	identifier of the corresponding emulation request.

5)	Introduce a report on ML inference emulation, which may provide reporting on ML emulation for one or more MLInferenceEmulationRequests or MLInferenceEmulationProcesses. This report may be equivalent to the inference report, and:

-	the ML inference emulation report may indicate the emulation environments in which the model has been emulated and passed;

-	the ML inference emulation report may also include the specific performance metrics for that emulation environments.

May also introduce the IOCs and datatypes for request, process and report on ML Inference and then or update these IOCs with the features of the ML Inference Emulation as introduced above.

6)	The IOCs, attributes and performance measurements for the solutions of performance evaluation for AI/ML inference for inference phase, as described in clause 5.2.6.4, which can be reused for monitoring the inference performance of the ML  models during the emulation.

7)	The IOCs and attributes for configuration management of 5G system, as defined in 3GPP TS 28.541 [20], which can be reused for configuring the emulation environment.

#### 5.3.1.5	Evaluation

The solution described in clause 5.3.1.4 reuses the existing provisioning MnS operations and notifications in combination with extensions of the NRM. Requests for inference emulation for a given ML model may be instantiated using provisioning management service implemented via CRUD (Create, Read, Update, Delete) operations on the request objects. The solution provides the flexibility to allow any function that can execute n ML model to be the MnS producer for ML inference emulation, e.g. an inference function or a generic sandbox function.

Therefore, the solution described in clause 5.3.1.4 is a feasible solution to be developed further in the normative specifications.

### 5.3.2	ML emulation environment selection

#### 5.3.2.1	Description

#### 5.3.2.2	Use cases

##### 5.3.2.2.1	ML emulation environment selection

Although a ML model is well trained, the performance of the ML model in the production network is difficult to determine and guarantee because the training environment and production network are not exactly the same. If a trained or tested ML model is directly applied to the production network, it may have a negative impact on the production network.

ML emulation means that the ML model is applied in an emulation environment to check whether the performance of ML model meets the expected inference characteristics. The MnS inference consumer may want to receive results of ML emulation to decide whether to use the ML model.

The management system should have the capability to enable an MnS consumer to select the emulation environment, as well as provide the configuration information related to the emulation environment. The configuration information may include determining the simulation scope, simulation time, etc.

#### 5.3.2.3	Potential requirements

REQ-EMUL-x: The MnS producer for AI/ML inference emulation should have a capability enabling an authorized MnS consumer to select the emulation environment.

#### 5.3.2.4	Possible solutions

#### 5.3.2.5	Evaluation

## 5.4	AI/ML deployment

### 5.4.1	Enhance the ML model loading use case

#### 5.4.1.1	Description

In TS 28.105 [2], the existing use cases (Consumer requested ML model loading in clause 6.4.1.2.1 and Control of producer-initiated ML model loading in clause 6.4.1.2.2) are described to support loading one ML model.

However, the consumer may want to load more than one ML model. The MnS producer can have a capability allowing an authorized consumer to request to load and manage the loading process for one or more ML model(s).

#### 5.4.1.2	Use cases

#### 5.4.1.3	Potential requirements

REQ-ML_LOAD-1: The MnS producer for ML model loading should have a capability allowing an authorized consumer to request to trigger loading of one or more ML model(s).

#### 5.4.1.4	Possible solutions

Enhance the NRM fragment for ML model loading by changing the muiltiplicity from “1” to “*” for the relationships.

- MLModelLoadingRequest to MLmodel

- MLModelLoadingProcess to MLmodel

### 5.4.2 	Managing ML Model Transfer in RAN

#### 5.1.2.1 	Description

ML Model Transfer Use Case in TR 38.843 refers to the delivery of an AI/ML model over the air interface. ML Model Delivery refers to delivery of an AI/ML model from one entity to another entity. Interaction over air interface in not the  scope of SA5 and what is called delivery in RAN corresponds to ML Model Loading in SA5.

#### 5.4.2.2	Use cases

##### 5.4.2.2.1	Map ML Model Delivery in RAN to ML Model Loading in SA5

The solution 4b in TR 38.843 states that OAM can transfer/deliver AI/ML model(s) to UE. The exsting solution for ML Model Loading service in TS 28.105 [2] can be used by RAN to load the model to gNB(s).

##### 5.4.2.3	Potential Requirements

REQ-Loading_MGT-01: The 3GPP Management System should have a capability to support ML Model Loading.

## 5.5	AI/ML inference

### 5.5.1	Coordination between the ML capabilities

Editor’s Note: The content in this clause may need to be aligned with the latest version of TS28.105

#### 5.5.1.1	Description

For ML in 5GC or RAN, the ML capabilities in 5GC or RAN may be needed to coordinate with 3GPP management analytics and possibly other aspects in order to improve the overall performance.

Typically, due to the type of the collected data used for model training/inference for 5GC or RAN, the performance of a model in 5GC or RAN may be biased in some respects. On the other hand, the 3GPP management system collects data of longer range and from a wide scope of RAN nodes/5GC, which consequently implies that the predictions calculated by the management system will be unbiased towards the overall RAN nodes/5GC. However, 3GPP management system predictions may lack insight of patterns related to specific node behaviour or finer granularity of time. Hence with coordination or alignment of the ML capability among 5GC/RAN and 3GPP management system, the overall performance may be improved.

To enable the coordination between the ML capabilities, the configuration (e.g. a triggering condition, i.e. when a result is needed from the RAN analytics to MDA) may be needed. On the other hand, the result of the coordination may be communicated towards the consumer(s) and hence there is a need to enhance the reporting in order to capture the deviation of predictions (and/or the related context) so the consumer can gain a better understanding regarding the coordination of ML capabilities.

#### 5.5.1.2	Use cases

##### 5.5.1.2.1		Alignment of the ML capability between 5GC/RAN and 3GPP management system

Generally, the typical data from 5GS data is measurements (PM, KPI), which may be modeled as "time series data" and the analytics of "time series data" is normally to learn the seasonality, trend, etc., patterns. Different types of seasonality patterns exist, e.g. daily, weekly, monthly, seasonally, annually, etc. A RAN node collects finer granularity data for short duration. The finer seasonality pattern will be well captured in a timely manner, while the 3GPP management system with longer range of data (which is more aggregated) will more accurately capture the higher level of seasonality patterns. Hence, combing the analytics results from RAN, 5G core and 3GPP management system or between RAN and 5G core may improve the accuracy for overall predictions.

On another matter, the data collected from one RAN or 5G core node would tend to be biased for that specific RAN node or NF, which implies that the prediction with the data learned and inferred will tend to be biased for that specific node. On the other hand, the 3GPP management system collects data of longer range and from a large amount of different RAN nodes or NFs, which consequently implies that the prediction will be unbiased towards the overall RAN nodes or 5G core area. Hence combining the results from both the RAN or 5G core and 3GPP management system, or between NWDAF and RAN may also improve the overall predictions accuracy (and mitigate the bias).

#### 5.5.1.3	Potential requirements

REQ-AIML_COORD-01: 3GPP management system should have the capability to allow an authorized consumer to request a coordination of different  ML capabilities, e.g.,  between MDAS and RAN function, or MDAF and NWDAF, or among MDAF(s) or among ML Model(s).

REQ-AIML_COORD-02: 3GPP management system should have the capability to report the result of coordination of different ML capabilities.

#### 5.5.1.4	Possible solutions

##### 5.5.1.4.1	Possible solution #1

1)	Introduce the information Elements (e.g. instance of IOC or a dataType) for interaction between ML MnS producer and consumer (e.g. the RAN analytics or NWDAF, or entity consuming the RAN analytics or NWDAF) to support coordination of the ML capability between 5GC/RAN and 3GPP management system: MLCapabilityCoordinationRequest and a response informant element MLCapabilityCoordinationResponse. This information element may represent the triggering configuration (or a triggering policy) for predictions coordination from two ML capabilities. This information Element may allow an MOI (or MOI using this information element) to be created on the ML (inference) MnS Producer and may contain the following attributes:

-	The analytics deviation indicator, such as a threshold (determining that the prediction calculated by a data analytics function exceeds a configurable certain value, corresponding to the prediction available at a different data analytics function, by a "threshold").

-	The requested analytics (analytics type name, list of analytics values or output, time intervals, confidence degree, etc.).

-	The target objects, e.g. gNBs, and the related characteristics.

-	Area of interest, geographical area or TA.

2)	MLCapabilityCoordinationResponse- this information element may represent the response indicating the analytics or data obtained according to the MLCapabilityCoordinationRequest. This information Element may be created by the ML MnS (inference) producer towards the MnS consumer and includes output analytics which can be statistics or predictions. The MLCapabilityCoordinationResponse and MLCapabilityCoordinationRequest may also be data attribute in analytic request and response or analytics report.

Figure 5.5.1.4.1-1: Interaction between ML MnS producer and consumer
to support coordination of the ML capability

##### 5.5.1.4.2	Evaluation

The solution described in clause 5.5.1.4.1 proposes simple information elements and procedure that may enable MnS Producer to trigger configuration to be used for analytic coordination. This NRM based solution reuses the existing provisioning MnS Operations and notifications for control and reporting. Therefore, the solution described in clause 5.5.1.4.1 is a feasible solution to be developed further in the normative specifications.

### 5.5.2 	Sustainable AI/ML

#### 5.5.2.1 	Description

AI/ML techniques have been integrated into mobile networks enabling the learning of network states and provide informed recommendations on the most effective actions based on these learnings. This approach significantly enhances the efficiency in managing a wide array of network use cases. These include, but are not limited to, mobility management, load balancing, and resource optimization. The expectation is that the application of AI/ML techniques will lead to an improvement in overall network performance due to the ability of these techniques to provide more accurate predictions and more effective decision-making processes. However, it is important to note that while the use of AI/ML techniques can lead to significant improvements in network performance, these benefits may come at the cost of increased energy consumption. AI/ML-based solutions, due to their complex computational requirements, may consume additional energy compared to traditional, non-AI/ML solutions.

Sustainable AI/ML aims to reduce energy usage in the AI/ML inference step. It is important to investigate effective methods for evaluating AI/ML energy consumption and efficiency for AI/ML inference which will aid in decreasing energy consumption, enhancing energy efficiency, and promoting sustainable development of AI/ML in 5G systems.

#### 5.5.2.2	Use cases

##### 5.5.2.2.1	AI/ML energy consumption evaluation and reporting for AI/ML inference

AI/ML energy consumption is a significant concern, especially as ML models become more complex and data-intensive. The ML model once trained is deployed for inference which consumes a considerable amount of energy. Each inference made by the ML model requires computational resources, and these resources in turn require energy to operate. With potentially thousands of inferences being made, the total energy consumption during inference can be substantial.

The MnS consumer may have concern on the energy consumption for AI/ML inference and the MnS Producer should report the related information. MnS Consumer may ask the MnS Producer to report the energy consumption for inference. To satisfy such query request from the MnS consumer, methods should be investigated to measure AIML energy consumption during the inference. Then, MnS Consumer can use the energy consumption information and compare with the network performance gain due to ML model to evaluate the total benefit of using the ML model.

#### 5.5.2.3	Potential requirements

REQ-AIML_INF_ENERGY_MGT-01: The AI/ML inference MnS producer should have a capability to allow an authorized MnS consumer to query energy consumption pertaining to generating inferences.

REQ-AIML_INF_ENERGY_MGT-02: The AI/ML inference MnS producer should have a capability to report the energy consumption pertaining to generating inferences to an authorized consumer.

#### 5.5.2.4	Possible solutions

The inferenceOutputs attribute (of type inferenceOutput) defined in the AIMLInferenceReport IoC needs to be enhanced with a new attribute related to energy consumption for AI/ML inference, e.g., inferenceEnergyConsumption. This attribute is of type real and indicates the energy consumption value pertaining to generating AI/ML inference, e.g., “KWh”.

Editor’s Note: The method to determine energy consumption for AI/ML inference is FFS. If energy consumption for AI/ML inference cannot be determined, the proposed solution in 5.5.2.4 becomes redundant.

#### 5.5.2.5	Evaluation

### 5.5.3	ML remedial action management

#### 5.5.3.1	Description

Trained ML models are deployed in the network where they will perform inference. The MnS Consumer may activate the ML inference by providing the scope (e.g. time window, geographical area) in which the ML inference should be activated or a policy for activation. The assumption is that the activation of ML inference will improve the network performance. However, it is difficult to assess the benefits and to quantify such benefits of applying ML inference in a given context of operational network, before using it.

#### 5.5.3.2	Use cases

##### 5.5.3.2.1	ML remedial actions due to performance degradation and energy consumption

By monitoring the network performance, the AI/ML inference MnS Consumer may identify the degradation of network performance or assess the benefits of employing ML model in the network, e.g. improvements in network performance when activating AI/ML inference. Based on the observations the AI/ML inference MnS Consumer may decide to perform certain remedial actions in order to mitigate undesired effect of the activated AI/ML inference. This may include the request to deactivate the AI/ML inference completely or change/reduce the scope of the AI/ML inference activation (e.g., limiting to only certain cells or geographical areas). Additionally, such remedial actions may also be triggered by the AI/ML inference MnS Consumer due to the increased energy consumption for AI/ML inference without achieving significant performance gains.

#### 5.5.3.3	Potential requirements

REQ-FB-1:  The 3GPP management system shall have a capability to enable an authorized MnS consumer to request change and reduction of the scope of AI/ML inference activation as a remedial action.

#### 5.5.3.4	Possible solutions

#### 5.5.3.5	Evaluation

### 5.5.4	Managing ML models in use in a live network

#### 5.5.4.1	Description

#### 5.5.4.2	Use Cases

##### 5.5.4.2.1	Handling of underperforming ML trained models in live networks

Actions may need to be taken by a network operator once an ML trained model has been identified that is contributing towards non-optimal running of the network. These actions may involve for example, without service interruptions, reverting to running of the network without ML based optimizations or replacing current ML model with an earlier model one that was performing better.

##### 5.5.4.2.2		Performance monitoring of Network Functions with ML trained models in live networks

Several trained ML models maybe in use in an operator network with each one of them influencing network performance. The Network Functions with these ML models need to be monitored  to ensure network is running optimally with these models in use. KPIs for evaluating runtime performance of Network Functions using ML models should be provided for this purpose as part of the model and these KPIs would need to be collectable by the network operator.

#### 5.5.4.3	Potential requirements

REQ-DATA-ACT-1: The 3GPP management system should have a capability enabling an authorized consumer to identify an ML Model that caused performance measurement and/or KPI degradation.

REQ-DATA-ACT-2: The 3GPP management system should have a capability to recommend remedial actions to address performance measurement and/or KPI degradation caused by an ML model.

REQ-DATA-PERF-1: The 3GPP management system should be able to collect, from the network, performance data pertaining to Network Functions actively utilizing ML models.

REQ-DATA-PERF-2: The 3GPP management system should support performance KPIs that can be monitored for performance assurance purpose.

#### 5.5.4.4	Possible solutions

#### 5.5.4.5	Evaluation

# 6	Conclusions and recommendations

For the development of Rel-19 normative specifications, it is recommended to

- specify information models for enabling knowledge-based transfer learning according to the solution in clause 5.1.1.4.

- specify information models for coordination of AI/ML Inference as described in the solution in clause 5.5.1.4.

- specify information models for orchestrating AI/ML Inference as described in the solution in clause 5.5.1.4

- specify information models for managing the progression of inference emulation according to the solution in clause 5.3.1.4.

Annex <A>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | SA5#156 | S5-243521 |  |  |  | Initial skeleton | 0.0.0 |
| 2024-08 | SA5#154 | S5-242150 |  |  |  | Rel-19 pCR TR28.9xx R19 Study usecases for continuation from 28.908 | 0.1.0 |
| 2024-08 | SA5#155 | S5-243096 |  |  |  | Rel-19 Add use case of ML pre-training | 0.1.0 |
| 2024-08 | SA5#155 | S5-243118 |  |  |  | Rel-19 pCR TR28.9xx Clusters for Joint Training of ML models | 0.1.0 |
| 2024-08 | SA5#155 | S5-243115 |  |  |  | Rel-19 pCR TR28.9xx Training Data Statistical Properties | 0.1.0 |
| 2024-08 | SA5#155 | S5-243116 |  |  |  | Rel-19 pCR TR 28.9xx Confidence Threshold | 0.1.0 |
| 2024-08 | SA5#155 | S5-243314 |  |  |  | Rel-19 pCR TR28.9xx Management of Reinforcement Learning | 0.1.0 |
| 2024-08 | SA5#155 | S5-243434 |  |  |  | Rel-19 pCR TR 28.9xx AI/ML energy consumption and energy efficiency | 0.1.0 |
| 2024-08 | SA5#156 | S5-244592 |  |  |  | pCR TR 28.858 add scope | 0.2.0 |
| 2024-08 | SA5#156 | S5-244593 |  |  |  | pCR TR 28.858 add overview and list of references | 0.2.0 |
| 2024-08 | SA5#156 | S5-244596 |  |  |  | Rel-19 pCR TR28.858 Add solution for ML model training for multiple contexts | 0.2.0 |
| 2024-08 | SA5#156 | S5-244605 |  |  |  | Rel-19 pCR TR 28.858 Add use case, requirements and solution for AIML inference energy consumption | 0.2.0 |
| 2024-08 | SA5#156 | S5-244604 |  |  |  | Rel-19 pCR TR 28.858 Add requirements and solution for ML model training energy consumption | 0.2.0 |
| 2024-08 | SA5#156 | S5-244594 |  |  |  | Rel-19 pCR TR28.858 Add use case and requirements for ML fallback management | 0.2.0 |
| 2024-08 | SA5#156 | S5-244595 |  |  |  | Rel-19 pCR TR 28.858 Add evaluation for training data statistical properties | 0.2.0 |
| 2024-08 | SA5#156 | S5-244619 |  |  |  | Rel-19 pCR TR 28.858 Add use case on ML entity distributed training | 0.2.0 |
| 2024-08 | SA5#156 | S5-244826 |  |  |  | Rel-19 pCR TR 28.858 Add solution for ML model confidence threshold | 0.2.0 |
| 2024-08 | SA5#156 | S5-244615 |  |  |  | Rel-19 pCR TR 28.858 Add Potential Solution for Fine-tuning | 0.2.0 |
| 2024-08 | SA5#156 | S5-244609 |  |  |  | Rel-19 pCR TR 28.858 Add use case for ML Fine-tuning | 0.2.0 |
| 2024-08 | SA5#156 | S5-244616 |  |  |  | Rel-19 pCR TR 28.858 Add Potential Solution for Pre-training | 0.2.0 |
| 2024-08 | SA5#156 | S5-244634 |  |  |  | Rel-19 pCR TR 28.858 add use cases, requirements for reinforcement learning management | 0.2.0 |
| 2024-08 | SA5#156 | S5-244599 |  |  |  | Rel-19 pCR TR 28.858 Enhance the ML model loading use case and solution | 0.2.0 |
| 2024-08 | SA5#156 | S5-244637 |  |  |  | pCR TR 28.858 Update the UC and requirements for pre-training | 0.2.0 |
| 2024-08 | SA5#156 | S5-244817 |  |  |  | pCR TR 28.858 Add use case for management of Federated Learning | 0.2.0 |
| 2024-08 | SA5#156 | S5-244607 |  |  |  | pCR TR 28.858 Add Use Case and Requirements for ML Model Transfer | 0.2.0 |
| 2024-08 | SA5#156 | S5-244606 |  |  |  | pCR TR 28.858 Sustainability of AI-ML | 0.2.0 |
| 2024-08 | SA5#156 | S5-244601 |  |  |  | Rel-19 pCR TR28.858 Add use case of AI emulation | 0.2.0 |
| 2024-08 | SA5#156 | S5-244618 |  |  |  | Rel-19 pCR Network Management actions in response to an underperforming ML algorithm used in a live network | 0.2.0 |
| 2024-08 | SA5#156 | S5-244617 |  |  |  | Collection of runtime performance KPIs for ML models | 0.2.0 |
| 2024-08 | SA5#156 | S5-244600 |  |  |  | Rel-19 pCR TR28.858 Conclude R19 continuation use cases | 0.2.0 |
| 2024-08 | SA5#156 | S5-244598 |  |  |  | Rel-19 pCR TR28.858 Update R19 continuation use cases | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |
| 2024-08 | SA5#156 |  |  |  |  |  | 0.2.0 |