| 3GPP TR 28.858 V1.0.0 (2024-12) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Artificial Intelligence / Machine Learning (AI/ML) management Phase 2 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	7

Introduction	8

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	10

3.3	Abbreviations	11

4	Concepts and overview	11

4.1	Overview	11

4.2	Energy consumption of AI/ML	12

4.3	Management of AI/ML capabilities for RAN and 5GC	12

4.3.1	Management of ML model training and AI/ML inference function for RAN	12

4.3.1.1	Managing NG-RAN AI/ML based Coverage and Capacity Optimization	12

4.3.1.2	Managing NG-RAN AI/ML-based Network Slicing	12

4.3.2	Management of ML model training and AI/ML inference function for 5GC	13

4.4	AI/ML trustworthiness	13

5	Management capabilities for AI/ML lifecycle	13

5.1	ML model training	13

5.1.1	ML-Knowledge-based Transfer Learning	13

5.1.1.1	Description	13

5.1.1.2	Use cases	14

5.1.1.2.1	Discovering sharable Knowledge	14

5.1.1.2.2	Knowledge sharing and transfer learning	14

5.1.1.3	Potential requirements	15

5.1.1.4	Possible solutions	16

5.1.1.5	Evaluation	17

5.1.2	ML pre-training	17

5.1.2.1	Description	17

5.1.2.2	Use cases	17

5.1.2.2.1	Consumer requested ML pre-training	17

5.1.2.3	Potential requirements	17

5.1.2.4	Possible solutions	17

5.1.2.4.1	Possible solution #1	17

5.1.2.4.2	Possible solution #2	18

5.1.2.5	Evaluation	18

5.1.3	ML Fine-tuning	18

5.1.3.1	Description	18

5.1.3.2	Use cases	18

5.1.3.2.1	 ML fine-tuning for a pre-trained ML model	18

5.1.3.3	Potential requirements	19

5.1.3.4	Possible solution	19

5.1.3.4.1	Possible solution #1	19

5.1.3.4.2	Possible solutions #2	19

5.1.3.5	Evaluation	19

5.1.4	ML model training for multiple contexts	20

5.1.4.1	Description	20

5.1.4.2	Use cases	20

5.1.4.2.1	ML model training for multiple contexts	20

5.1.4.3	Potential Requirements	20

5.1.4.4	Possible solutions	20

5.1.4.5	Evaluation	21

5.1.5	ML training data statistics	21

5.1.5.1	Description	21

5.1.5.2	Use Cases	21

5.1.5.2.1	 Training data statistical properties for ML training	21

5.1.5.3	Potential requirements	21

5.1.5.4	Possible solutions	21

5.1.5.5	Evaluation	22

5.1.6	ML model confidence	22

5.1.6.1	Description	22

5.1.6.2	Use Cases	22

5.1.6.2.1	Model Confidence Threshold in ML Training	22

5.1.6.3	Potential requirements	22

5.1.6.4	Possible solutions	22

5.1.6.5	Evaluation	22

5.1.7	Management of Reinforcement Learning	23

5.1.7.1	Description	23

5.1.7.2	Use cases	23

5.1.7.2.1	Exploration in Reinforcement Learning	23

5.1.7.2.2	Training Conflict in Reinforcement Learning	24

5.1.7.3	Potential Requirements	24

5.1.7.4	Possible solutions	24

5.1.7.4.1	Possible solution #1: Exploration in Reinforcement Learning	24

5.1.7.4.2	Possible solution #2: Training Conflict in Reinforcement Learning	25

5.1.7.5	Evaluation	25

5.1.7.5.1	Exploration in Reinforcement Learning	25

5.1.7.5.2	Training Conflict in Reinforcement Learning	25

5.1.8	Sustainable AI/ML for ML training	25

5.1.8.1	Description	25

5.1.8.2	Use cases	26

5.1.8.2.1	AI/ML energy consumption evaluation and reporting for ML model training	26

5.1.8.3	Potential Requirements	26

5.1.8.4	Possible solutions	26

5.1.8.4.1	Possible solution #1	26

5.1.8.4.2	Possible solution #2	26

5.1.8.5	Evaluation	26

5.1.9	ML model distributed training	27

5.1.9.1	Description	27

5.1.9.2	Use cases	27

5.1.9.2.1	ML model distributed training	27

5.1.9.3	Potential requirements	27

5.1.9.4	Possible solutions	27

5.1.9.4.1	ML model distributed training	27

5.1.9.5	Evaluation	28

5.1.10	Management of Federated Learning	28

5.1.10.1	Description	28

5.1.10.2	Use cases	28

5.1.10.2.1	Management of different roles in Federated Learning	28

5.1.10.3	Potential requirements	29

5.1.10.4	Possible solutions	29

5.1.10.5	Evaluation	30

5.1.11	ML Authentication	30

5.1.11.1	Description	30

5.1.11.2	Potential Requirements	30

5.1.11.3	Possible Solution	30

5.1.11.4	Evaluation	31

5.1.12	AI/ML prediction latency	31

5.1.12.1	Description	31

5.1.12.2	Use cases	31

5.1.12.2.1	AI/ML prediction latency during ML model training	31

5.1.12.3	Potential requirements	31

5.1.12.4	Possible solutions	31

5.1.12.5	Evaluation	31

5.2	AI/ML inference emulation	32

5.2.1	ML inference emulation	32

5.2.1.1	Description	32

5.2.1.2	Use cases	32

5.2.1.2.1	AI/ML inference emulation	32

5.2.1.2.2	Managing ML inference emulation	32

5.2.1.3	Potential requirements	33

5.2.1.4	Possible solutions	33

5.2.1.5	Evaluation	35

5.2.2	ML inference emulation environment selection	35

5.2.2.1	Description	35

5.2.2.2	Use cases	35

5.2.2.2.1	ML inference emulation environment selection	35

5.2.2.3	Potential requirements	35

5.2.2.4	Possible solutions	35

5.2.2.5	Evaluation	35

5.3	AI/ML deployment	36

5.3.1	Enhance the ML model loading use case	36

5.3.1.1	Description	36

5.3.1.2	Use cases	36

5.3.1.3	Potential requirements	36

5.3.1.4	Possible solutions	36

5.3.2 	Managing ML Model Transfer/delivery	36

5.3.2.1 	Description	36

5.3.2.2	Use cases	36

5.3.2.2.1	Relation of ML model delivery in RAN to ML model loading in SA5	36

5.3.2.3	Potential Requirements	37

5.3.2.4	 Possible solutions	37

5.3.2.5	 Evaluation	37

5.4	AI/ML inference	37

5.4.1	Coordination between the ML capabilities	37

5.4.1.1	Description	37

5.4.1.2	Use cases	37

5.4.1.2.1	Alignment of the ML capability between 5GC/RAN and 3GPP management system	37

5.4.1.3	Potential requirements	38

5.4.1.4	Possible solutions	38

5.4.1.4.1	Possible solution #1	38

5.4.1.5	Evaluation	39

5.4.2 	Sustainable AI/ML for AI/ML inference	39

5.4.2.1 	Description	39

5.4.2.2	Use cases	39

5.4.2.2.1	AI/ML energy consumption evaluation and reporting for AI/ML inference	39

5.4.2.3	Potential requirements	39

5.4.2.4	Possible solutions	39

5.4.2.4.1	Possible solution #1	39

5.4.2.4.2	Possible solution #2	40

5.4.2.5	Evaluation	40

5.4.3	ML remedial action management	40

5.4.3.1	Description	40

5.4.3.2	Use cases	40

5.4.3.2.1	ML remedial actions due to performance degradation and energy consumption	40

5.4.3.3	Potential requirements	40

5.4.3.4	Possible solutions	40

5.4.3.5	Evaluation	41

5.4.4	Managing ML models in use in a live network	41

5.4.4.1	Description	41

5.4.4.2	Use Cases	41

5.4.4.2.1	Handling of underperforming ML trained models in live networks	41

5.4.4.2.2	Performance monitoring of Network Functions with ML trained models in live networks	41

5.4.4.3	Potential requirements	41

5.4.4.4	Possible solutions	41

5.4.4.5	Evaluation	42

5.4.5	AI/ML prediction latency	42

5.4.5.1	 Description	42

5.4.5.2	Use cases	43

5.4.5.2.1	AI/ML prediction latency during inference	43

5.4.5.3	Potential requirements	43

5.4.5.4	Possible solutions	43

5.4.5.5	Evaluation	43

5.4.6	ML explainability	43

5.4.6.1	 Description	43

5.4.6.2	Use cases	43

5.4.6.2.1	Local explanation in AI/ML inference	43

5.4.6.3	Potential requirements	44

5.4.6.4	Possible solutions	44

5.4.6.5	Evaluation	44

6	Conclusions and recommendations	44

Annex <A>: 	Change history	46

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document studies the enhanced Artificial Intelligence / Machine Learning (AI/ML) management capabilities and services building on the work of Rel-18, as well as new management aspects related to AI/ML functionalities across various 3GPP working groups within 5GS. It focuses particularly on the management of AI/ML-enabled features utilized in management and orchestration, 5GC, and NG-RAN. Furthermore, the study also investigates the AI/ML management capabilities supporting the emerging AI/ML technologies (e.g. Federated Learning, Reinforcement Learning, Pre-training, Fine-tuning and Distributed training) and the sustainability and trustworthiness of AI/ML in 5G system.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.105: Artificial Intelligence / Machine Learning (AI/ML) management".

[3]	3GPP TR 23.700-84: "Study on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML)".

[4]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[5]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[6]	3GPP TR 38.843: "Study on Artificial Intelligence (AI)/Machine Learning (ML)". 
for NR air interface.

[7]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[8]	3GPP TS 38.300: "NR and NG-RAN Overall Description;".

[9]	3GPP TR 38.743: "Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN ".

[10]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support; network data analytics services".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], TS 28.105 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and TS 28.105 [2].

Distributed training: an ML training approach that distributes the training workload across multiple ML training functions.

Federated Learning: a distributed machine learning approach where the ML model is trained collaboratively by multiple ML training functions. This includes multiple FL clients, which perform training on local data, and one FL server, which aggregates model updates from the clients iteratively without exchanging data samples.

Fine-tuning: the process of training a pre-trained ML model with a changed or narrowed scope.

FL client: a training function that trains an ML model on local data and share only the model outcome with the FL server/FL Client, preserving data privacy.

FL Server: a function that aggregates the ML model outcomes from FL clients to produce a global ML model.

Horizontal Federated Learning (HFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different FL clients for local model training have the same feature space for different samples.

ML Knowledge-based Transfer Learning: a technique where the knowledge gained from training of one or more ML models is applied or adapted to improve or develop another ML model.

Pre-training: the process of training an ML model on a dataset not specific to any type of inference.

Reinforcement Learning: a machine learning approach where an RL agent learns to make decisions by interacting with the environment and taking actions to maximize cumulative rewards (see NOTE 1).

NOTE 1: 	The examples of rewards could be improved resource allocation, reduced latency, or enhanced user experience.

Vertical Federated Learning (VFL): a federated learning technique without exchanging/sharing local data set, wherein the local data set in different VFL participant for local model training have different feature spaces for the same samples.

NOTE 1:	The definitions of HFL and VFL referenced above are sourced from TR 23.700-84 [3]. This definition might be updated or modified in normative work.

NOTE 2:	The definition of VFL is included for completeness and may need to be revisited with considerations of concrete use cases where VFL may be used in 5GS.

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

FL	Federated Learning

HFL	Horizontal Federated Learning

MLKTL	ML Knowledge Transfer Learning

RL	Reinforcement Learning

VFL	Vertical Federated Learning

# 4	Concepts and overview

## 4.1	Overview

AI/ML techniques have gained significant interests and advancements across various industries, including telecommunications, where they are now being applied to enhance mobile networks. While AI/ML technologies have matured considerably, certain aspects continue to evolve, and new techniques frequently emerge to complement existing methods.

AI/ML techniques can be characterized from several perspectives:

-	Learning Methods: AI/ML employs various learning methods such as supervised, semi-supervised, unsupervised, and reinforcement learning. Each method is suited to specific inference categories (e.g. prediction) and requires distinct types of training data.

-	Learning Complexity: AI/ML techniques vary in complexity, ranging from basic Machine Learning to more advanced e.g. Deep Learning with neural networks.

-	Learning Architecture: Depending on the topology and location of learning tasks, AI/ML can be categorized into Generative AI, centralized, distributed, and federated learning.

-	Learning Continuity: AI/ML can be implemented as online learning, offline learning or continual learning, depending on whether the learning process is static or ongoing.

In the 5GS, AI/ML capabilities are used across various domains, including management and orchestration, 5GC and NG-RAN. The performance of AI/ML inference function within 5GS depends on how well-trained ML models for accurate and efficient inference.

To effectively deploy AI/ML capabilities in 5GS, it is crucial to manage the ML models and AI/ML inference functions according to the specific characteristics and requirements of the use cases they support.

The study investigates enhancements of AI/ML management capabilities by building on the work from Rel-18 and exploring new management aspects across various 3GPP working groups. Additional advanced use cases are investigated, covering AI/ML management and operational capabilities to support different types of AI/ML technologies required for AI/ML in the 5G system (e.g. Federated Learning, Reinforcement Learning, online/offline learning, Generative AI and Distributed Learning).

Some primary aspects of AI/ML sustainability are studied, including the evaluation of energy and resource consumption and efficiency impacts associated with AI/ML features across all operational steps (training, emulation, deployment, inference).

The present TR also investigates management aspects (LCM CM and PM) of additional AI/ML functionalities defined by 3GPP SA WGs and RAN WGs.

## 4.2	Energy consumption of AI/ML

The energy consumption of AI/ML is to be considered according to various aspects, including:

-	Multiplicative aspect: This depends on the step of the AI/ML model lifecycle being considered (e.g. ML model training, ML model testing, AI/ML inference emulation, ML model deployment, AI/ML inference). For instance, AI/ML inference has a repetitive nature, whereas ML model training may be performed only once (though some re-training may be necessary). This multiplicative aspect can be measured by the number of (inference) cycles in each step.

-	Dataset volume: ML model training typically requires large sets of data to be collected, stored and processed, in contrast to AI/ML inference, which generally handles smaller data volumes. Dataset volumes can be estimated by the number of entries in the dataset (e.g. the number of gNBs) multiplied by the number of parameters that characterize each entry (e.g. the number of parameters collected per gNB). Dataset volumes are considerably larger during the ML model training step than during the AI/ML inference step.

-	Computational complexity of each cycle: This can be measured by the number of operations (e.g. floating-point operations) performed during each cycle. The computational complexity may vary depending on the ML model's complexity and software efficiency.

-	Performance of the hardware platform: The performance during each step of the AI/ML model lifecycle depends on the type of hardware used (e.g. GPUs vs. CPUs), as well as hardware's performance characteristics and overall system design.

In contrast with AI/ML energy consumption, AI/ML may also help save energy, e.g. by optimizing the usage of network resources and consequently enabling network energy savings.

## 4.3	Management of AI/ML capabilities for RAN and 5GC

### 4.3.1	Management of ML model training and AI/ML inference function for RAN

#### 4.3.1.1	Managing NG-RAN AI/ML based Coverage and Capacity Optimization

In TS 28.105 [2], the existing use cases (AI/ML inference capability configuration management in clause 6.5.4) are described to support managing NG-RAN AI/ML-based related use case defined in TS 38.300 [8].

TR 38.743 [9] defines the use case of AI/ML based distributed Coverage and Capacity Optimization. The MnS producer for AI/ML inference management needs to provide a capability for configuration of the AI/ML based distributed Coverage and Capacity Optimization inference function.

An NG-RAN AI/ML-based distributed Coverage and Capacity Optimization capability may use one or more ML models or AI/ML Inference Functions to derive CCO recommendations.

The MnS consumer monitors the network performance and determines whether to, and when to activate or deactivate an AI/ML inference functions related to an AI/ML-based Coverage and Capacity Optimization function. The activation and deactivation actions for AI/ML inference functions related to an AI/ML-based Coverage and Capacity Optimization conducted by the MnS producer may also be triggered by some defined policies provided by the consumer.

#### 4.3.1.2	Managing NG-RAN AI/ML-based Network Slicing

In TS 28.105 [2], the existing use cases (AI/ML inference capability configuration management in clause 6.5.4) are described to support managing NG-RAN AI/ML-based related use case defined in TS 38.300 [8].

TR 38.743 [9] defines the use case of AI/ML based distributed Network Slicing. The MnS producer for AI/ML inference management needs to provide a capability for configuration of the AI/ML based distributed Network Slicing inference function.

An NG-RAN AI/ML-based distributed Network Slicing function may use one or more ML models or AI/ML inference functions to derive slice optimization recommendations.

The MnS consumer monitors the network performance and determines whether to, and when to activate or deactivate an AI/ML inference function related to an AI/ML-based Network Slicing function. The activation and deactivation actions for AI/ML inference function related to an AI/ML-based Network Slicing conducted by the MnS producer may also be triggered by defined policies provided by the consumer.

### 4.3.2	Management of ML model training and AI/ML inference function for 5GC

In TS 28.105 [2], the existing AI/ML functionalities management scenarios (relation with managed AI/ML features) (clause 4a.2) have captured the case where the ML model training and AI/ML inference functions are both located in 5GC.

In case of managing ML model training for NWDAF [10], an NWDAF may use one or more ML models to derive analytics results, the MnS consumer needs to manage the ML model training (e.g. federated learning) supported by the ML training function (i.e., MTLF) in the NWDAF.

In case of managing AI/ML inference for NWDAF, for monitoring whether AI/ML capability is enabled, the MnS consumer can check the value of activation status of an AnLF within NWDAF in case of AI/ML capability is supported by AnLF.

## 4.4	AI/ML trustworthiness

The present document focuses on the following aspects of AI/ML trustworthiness:

-	Explainability

Other aspects of AI/ML trustworthiness have not been studied in the present document.

# 5	Management capabilities for AI/ML lifecycle

## 5.1	ML model training

### 5.1.1	ML-Knowledge-based Transfer Learning

#### 5.1.1.1	Description

It is known that existing ML capability can be leveraged in producing new or improving other ML capability. Specifically, transfer learning allows knowledge contained in one or more ML models to be transferred to another ML model. Transfer learning relies on task and domain similarity to determine whether parts of a deployed ML model can be reused in another domain / task with some modifications.

In multi-vendor environments, aspects of transfer learning need to be supported in network management systems. However, as ML models are not expected to be multi-vendor objects, i.e. an ML model cannot be transferred directly from one function to another, the knowledge contained within the model, referred to as ML knowledge, should be transferred instead of the ML model itself.

ML knowledge represents the information gained through the training of an ML Model (e.g. experience that indicates the recommended outputs for a given set of input data). This information can be of various forms such as statistics (e.g. a distribution) or summaries (e.g. tables).

For example, knowledge contained in an ML model deployed to perform mobility optimization during the day can be leveraged to produce a new ML model to perform mobility optimization at night. As illustrated in Figure 5.1.1.1-1, the network or its management system needs to have the required management services to support ML Knowledge-based Transfer Learning (MLKTL). ML Knowledge-based Transfer Learning refers to the capability of enabling and managing the transfer learning between any two ML models or training functions.

Figure 5.1.1.1-1: ML Knowledge-based Transfer Learning (MLKTL) flow between the source MLKTL
(which is the MnS producer with the pre-trained ML model), the peer MLKTL
(which is the MnS producer that shall train a new ML model) and the MLKTL MnS consumer
(which may be the operator or another management function that triggers or controls MLKTL)

NOTE 1: 	The services between the source and peer for the transfer knowledge and triggering of learning is implementation specific and is not in scope of the current technical report.

NOTE 2: 	Transfer learning (including knowledge-based transfer learning) can be sensitive from a data exposure and privacy aspect. Knowledge-based transfer learning will not be triggered for non-authorized consumers, but the authorization of the consumers is not in scope of the current technical report.

#### 5.1.1.2	Use cases

##### 5.1.1.2.1	Discovering sharable Knowledge

For transfer learning, it is expected that the source ML Knowledge-based Transfer Learning (MLKTL) MnS producer shares its knowledge with the target ML training function, either through a single instance of knowledge transfer or through an interactive transfer learning process. The ML knowledge here represents any experiences or information gathered by the ML model within the MLKTL MnS producer through training, inference, updates, or testing. This knowledge can be in the form of data statistics, features of the underlying ML model, or the output of the ML model. The 3GPP management systems should provide mechanisms for an MnS consumer to discover this potentially shareable knowledge as well as the means for the MLKTL provider to share that knowledge with the MnS consumer.

##### 5.1.1.2.2	Knowledge sharing and transfer learning

Transfer learning may be triggered by an MnS consumer, either to fulfil its own learning needs or delegate the process to another ML training function. The model containing the knowledge may be an independent managed entity or it could be an attribute of a managed ML model or ML training function. In the latter case, MLKTL does not involve sharing the ML model itself or parts of it but would focus on enabling the sharing of the knowledge contained within the ML model or ML-enabled function.

The 3GPP management system should provide mechanisms and services needed to realize the ML Knowledge-based transfer learning process. Specifically, the 3GPP management system should enable an MnS consumer to request and receive sharable knowledge as well as means for the producer of MLKTL to share the knowledge with the MnS consumer or a designated target ML training function. Similarly, the 3GPP management system should support MnS consumers in managing and controlling the MLKTL process, including handling requests associated with knowledge transfer learning between two ML models directly or via a shared knowledge repository.

The two use cases should address the four scenarios illustrated below.

It should be noted that, the use cases and requirements here focus on the required management capabilities the implementation of the knowledge transfer learning processes are implementation details that are out of the scope of the present document.

Scenario 1: Interactions for ML-Knowledge-based Transfer Learning (MLKTL) to support training at the ML knowledge Transfer MnS consumer as peer - ML knowledge-based Transfer Learning MnS consumer obtains the ML knowledge which it then uses for training the new ML model based on knowledge received from the MLKTL source MnS producer.

Scenario 2: Interactions for ML-Knowledge-based Transfer Learning (MLKTL) to support training at the peer ML Knowledge-based Transfer Learning MnS producer triggered by the MLKTL Source - the ML Knowledge-based Transfer Learning MnS consumer acting as the MLKTL Source (the source of the ML knowledge) triggers the training at the ML knowledge-based Transfer MnS producer by providing the ML knowledge to be used for the training, the ML Knowledge-based Transfer Learning MnS consumer then undertakes the training.

Scenario 3: Interactions for ML-Knowledge-based Transfer Learning (MLKTL) to support training at the Peer ML knowledge-based Transfer Learning MnS producer who is different from the ML knowledge-based Transfer Learning MnS consumer - the ML knowledge-based Transfer Learning MnS consumer triggers training at the MLKTL peer MnS producer. The MLKTL MnS consumer then obtains the ML knowledge from the MLKTL source MnS producer and then uses the knowledge for training the new ML model based on knowledge received from the MLKTL source MnS producer.

Scenario 4: Interactions for ML-Knowledge-based Transfer Learning (MLKTL) to support training at the Source ML knowledge Transfer MnS producer - the ML knowledge-based Transfer Learning MnS consumer triggers training at the MLKTL source MnS producer. The MLKTL MnS consumer then takes its available ML knowledge-based and uses the knowledge for training the new ML model based.

#### 5.1.1.3	Potential requirements

REQ-MLKTL-1: The 3GPP management system should have a capability to enable an authorized MnS consumer to discover available shared knowledge from a given MLKTL MnS producer according to a stated set of criteria.

REQ-MLKTL-2: The 3GPP management system should have a capability to enable an authorized MnS consumer to request a MLKTL MnS producer to provide some or all the available knowledge for sharing according to specified criteria.

REQ-MLKTL-3: The 3GPP management system should have a capability for an MLKTL MnS producer to report to an authorized MnS consumer on the available shared knowledge according to a Reporting Criteria specified in a request for information about the available Knowledge.

REQ-MLKTL-4: The 3GPP management system should have a capability enabling an authorized MnS consumer to request a MLKTL MnS producer to initiate and execute a transfer learning instance to a specified ML model or ML-enabled function.

REQ-MLKTL-5: The 3GPP management system should have a capability to enable an authorized MnS consumer (e.g. an operator or the function/entity that generated the request for available Knowledge) to manage or control the knowledge request or its information and subsequent process, e.g. to suspend, re-activate or cancel the ML Knowledge Request; or to adjust the description of the desired knowledge.

REQ-MLKTL-6: The 3GPP management system should have a capability to enable an authorized MnS consumer (e.g. an operator or the function/entity that generated the request for MLKTL) to manage or control a specific MLKTL Job, e.g. to start, suspend or restart the MLKTL Job; or to adjust the transfer learning conditions or characteristics (i.e. Modify MLKTL Job attributes).

NOTE: 	The MLKTL Job represents the process of knowledge-based transfer learning.

REQ-MLKTL-7: The 3GPP management system should have a capability to enable an ML model to register available knowledge to a shared knowledge repository, e.g. through a ML Knowledge Registration process.

REQ-MLKTL-8: The 3GPP management system should have a capability to enable a Knowledge Repository to act as the MLKTL MnS producer, allowing an authorized MnS consumer to request the shared knowledge repository to provide information on the available knowledge according to specified criteria.

REQ-MLKTL-9: The 3GPP management system should have a capability to enable Knowledge Repository to act as the MLKTL MnS producer allowing an authorized MnS consumer to request the Knowledge Repository to provide some or all the knowledge available for sharing based on specific criteria.

REQ-MLKTL-10: The 3GPP management system should have a capability to enable Knowledge Repository to act as the MLKTL MnS producer allowing an authorized MnS consumer (e.g. an operator or the function/entity that generated the ML Knowledge Request) to manage the request, e.g. to suspend, re-activate or cancel the ML Knowledge Request ; or to adjust the description of the desired knowledge.

REQ-MLKTL-11: The 3GPP management system should have a capability to enable Knowledge Repository to act as the MLKTL MnS producer allowing an authorized MnS consumer (e.g. an operator) to manage or control a specific MLKTL Job, e.g. to start, suspend or restart the MLKTL Job; or to adjust the transfer learning conditions or characteristics.

#### 5.1.1.4	Possible solutions

Discovering sharable Knowledge

To discover sharable knowledge:

-	The MnS consumer may send a request to the MLKTL MnS producer to provide information on the available  sharable knowledge. In other words, the MLKTL MnS producer receives a request to report on the available sharable knowledge.

-	The request may be generic or may state a set of criteria which the knowledge should fulfil.

-	The request may be referred to as MLKnowledgeInfoRequest.

-	The MLKnowledgeInfoRequest must have informational description (Metadata description) of the task and domain related to the required knowledge or given a network problem.

-	An ML model or a function containing an ML model may register its available knowledge to a shared knowledge repository, e.g. through a MLKnowledgeRegistration process.

-	The MLKnowledgeRegistration must contain informational description (Metadata description) of the task and domain related to the registered knowledge or suitable network problem.

Knowledge sharing and transfer learning

To share knowledge:

-	Introduce an IOC for an ML Knowledge request. The MnS consumer may send a request to the MLKTL MnS producer to share a specific kind of knowledge. i.e. the MLKTL MnS producer receives a request to provide sharable knowledge, The request may be referred to as MLKnowledgeRequest.

-	Introduce an IOC for an ML Knowledge-based transfer learning process or job which is instantiated for any request for transfer learning or ML knowledge-based transfer. The MLKTL MnS producer instantiates a MLknowledge-based transfer learning process. The process may be referred to as MLKTLJob.

-	The MLKTLJob is responsible for adapting the required knowledge into a shareable format with the MLKTL consumer.

-	MLKTLJob may be a continuous process where knowledge is shared with the MLKTL consumer frequently to account for updates in the knowledge.

NOTE: 	It may also be the case that the consumer directly instantiates the MLKTLJob without a separate request.

#### 5.1.1.5	Evaluation

The use case has provided requirements and solutions for knowledge-based transfer learning as a subset of transfer learning and a means for achieving the same outcomes as transfer learning (i.e. reusing an existing model to create a new model) but without need to exchange ML models.

The solution described in clause 5.1.1.4 proposes information objects that can enable ML functions to exchange their knowledge to for transfer learning, while ensuring that vendor-specific aspects of the ML models are not exposed. Therefore, the solution described in clause 5.1.1.4 represents a feasible approach prior to further developments and elaboration in the normative specifications.

NOTE 1: 	The normative solution shall not include specification of services between the source and peer for the transfer knowledge and triggering of learning.

NOTE 2: 	Transfer learning (including knowledge-based transfer learning) can be sensitive from a data exposure and privacy aspect. The normative work should describe interaction for authenticated and authorized consumers to ensure that knowledge-based transfer learning will not be triggered for non-authenticated or non-authorized consumers. The authentication and authorization process is not in the scope of this particular use case and is referred to existing procedures.

### 5.1.2	ML pre-training

#### 5.1.2.1	Description

Pre-training refers to the process of training an ML Model using a dataset that supports multiple types of inference. For example, an ML model could be pre-trained using dataset from SLS analysis capability group covering type of inference including ServiceExperienceAnalysis, NetworkSliceThroughputAnalysis, NetworkSliceTrafficAnalysis, NetworkSliceLoadAnalysis and E2ElatencyAnalysis (see TS 28.104 [4]).

The pre-training is not intended to support a specific aIMLInferenceName but rather leveraging on commonality among number of use cases.

The ML model training process defined in TS 28.105 [2] also covers pre-training.

#### 5.1.2.2	Use cases

##### 5.1.2.2.1	Consumer requested ML pre-training

The pre-training type follows the standard procedure of ML model training, however the information provided in training request differs.

#### 5.1.2.3	Potential requirements

REQ-ML_ TRAIN -MLPT-1: The ML training MnS producer shall have a capability to enable an authorized consumer to request a pre-training of an ML model.

NOTE: 	It is for further discussion if the same requirements in clause 6.2b.3 "Requirements for ML model training" in TS 28.105 [2] related to training function also applies for pre-training.

#### 5.1.2.4	Possible solutions

##### 5.1.2.4.1	Possible solution #1

The existing MLTrainingRequest IOC and MLModel IOC can be enhanced to support pre-training.

Enhancement on MLTraingRequest IOC: Since the existing MLTrainingRequest IOC cannot support consumer requested ML pre-training, it is proposed to introduce following attributes into MLTrainingRequest IOC:

-	preTrainingIndication, indicates that the request model is a Pre-training Model. Besides, change the support qualifier of "aIMLInferenceName" to CM, where the condition is that the ML Model requested to train is not a pre-training ML Model.

-	potentialInferenceScope, indicates a set of types of inference, which may include a list of supported aIMLInferenceName.

Enhancement on MLModel IOC: Since the pre-training is not aimed to support a specific inference capability but focuses on commonality in use cases, it is proposed to introduce following attributes:

-	preTrainingModelIndication, indicates the ML Model is a Pre-training Model. Besides, the support qualifier of "aIMLInferenceName" should be changed to CM, which means this attribute should be present when the MLModel MOI represents a Pre-training Model.

-	potentialInferenceScope. Definition is documented above.

##### 5.1.2.4.2	Possible solution #2

The existing MLTrainingRequest IOC and MLModel IOC can be enhanced to support pre-training.

Enhancement on MLTraingRequest IOC: Since the existing MLTrainingRequest IOC cannot support consumer requested ML pre-training, it is proposed to introduce following attributes into MLTrainingRequest IOC:

-	mLTrainingType, indicates the type of training, e.g. initial training, re-training, pre-training, etc.

- 	potentialInferenceScope, indicates the inference capabilities. It can be an NG/RAN capability, MDA capability or any other extension.

Enhancement on MLModel IOC:

-	potentialInferenceScope. Definition is documented above.

The existing ML training procedures in TS 28.105 [2] already include all the necessary mechanisms for initial training and re-training an ML model. The existing MLTrainingRequest IOC can be utilized to support pre-training by reusing the defined attribute MLTrainingType, and PotentialInferenceScope which are of type string and allows for one or more values.

#### 5.1.2.5	Evaluation

Potential Solution #2 offers a simpler, more efficient, and flexible approach to pre-training models, IOCs and mechanisms in TS 28.105 [2]. It minimizes complexity, aligns with current standards making it the preferred solution over Solution #1.

### 5.1.3	ML Fine-tuning

#### 5.1.3.1	Description

ML fine-tuning refers to the process of training an already trained ML model (e.g. a pre-trained ML model) with a changed scope.

In contrast, re-training of an ML Model is not based on a specific type of inference, while fine-tuning of a pre-trained model means that the ML model will be trained with focus on some specific types of inference.

The ML Model training process defined in TS 28.105 [2] includes provisions for fine-tuning pre-trained models to narrow down the scope of inference.

#### 5.1.3.2	Use cases

##### 5.1.3.2.1		ML fine-tuning for a pre-trained ML model

A pre-trained ML model may be initially trained on a dataset that supports various types of inference. However, fine-tuning allows the model to narrow down the scope.

NOTE:	 It is for further discussion whether fine-tuning process could be applied for the same producer who pre-trained the model or for another producer.

#### 5.1.3.3	Potential requirements

REQ-ML_ TRAIN-MLFT-1: The ML training MnS producer shall have a capability to enable an authorized consumer to request the fine-tuning of a pre-trained ML model.

NOTE:	 It is for further discussion if the same requirements in clause 6.2b.3 "Requirements for ML model training" in TS 28.105 [2] related to training function also applies for fine-tuning.

#### 5.1.3.4	Possible solution

##### 5.1.3.4.1	Possible solution #1

The existing MLModel IOC and MLTrainingRequest IOC can be enhanced to support fine-tuning.

Enhancement Aspects on MLTrainingRequest IOC: Since the existing MLTrainingRequest IOC cannot support consumer requested ML fine-tuning, both following enhancements are possible.

-	FineTuningIndication, indicates the consumer requested ML fine-tuning. The support qualifier should be CM, which means this attribute should be present when the MLTrainingRequest MOI represents the request for ML Model Fine-tuning. Besides, change the support qualifier of "aIMLInferenceName" to CM, which means that this attribute should be present when the MLTrainingRequest MOI does not represent the request for ML Model Fine-tuning.

-	PotentialInferenceScope, indicates a set of types of inference, which may include a list of supported aIMLInferenceName.

Enhancement Aspects on MLModel IOC: Since fine-tuning is aiming at a specific inference type for a pre-trained model, the AIML capabilities would be narrowed which has the best performance. It is proposed to introduce following attributes:

-	FinetuningModelIndication, indicates that the ML Model is a Finetuning Model. Besides, change the support qualifier of "aIMLInferenceName" to CM, where the condition is that the ML Model requested to train is not a pre-training ML Model.

-	PotentialInferenceScope, indicates a set of types of inference, which may include a list of supported aIMLInferenceName.

##### 5.1.3.4.2	Possible solutions #2

The existing ML training procedures in TS 28.105 [2] already provide all the necessary mechanisms for initial training and re-training of an ML model. The existing MLTrainingRequest IOC may be used to support fine-tuning, similarly to how it is used for the re-training procedure.

The key distinction between retraining and fine-tuning is that retraining results in a new version of the same ML model, while fine-tuning produces results in a new ML model, which has a different mlModelId, MLModelRef, and aIMLInferenceName than the ML model that was requested to be fine-tuned.

To enable fine-tuning within the current framework the following enhancements are need:

Enhancement on MLTrainingRequest IOC:

-	mLTrainingType, indicates the type of training, e.g. pre-training, fine-tunning, etc. (see clause [1] 5.1.2.4).

-	change the AIMLInferenceName from 1 to 1..*

#### 5.1.3.5	Evaluation

Possible solution #1 described in clause 5.1.3.4.1 proposes to add "FineTuningIndication" and "PotentialInferenceScope" to the MLTrainingRequest IOC, and add "FinetuningModelIndication" and "PotentialInferenceScope" to the MLModel IOC. Only simple enhancement is introduced to support fine-tuning, which is feasible to be developed further in the normative specifications.

Compared to possible solution 1#, possible solution 2# in clause 5.1.3.4.2 offers a simpler and flexible approach to fine-tuning an ML model by reusing existing attributes, IOCs and mechanisms in TS 28.105 [2]. It also aligns with the use case for pre-training in clause  5.1.2.4.2 of [2] by introducing the generic attribute mLTrainingType to indicate the type of training requested by the consumer. By changing AIMOInferenceName from 1 to * to indicate the capability of a fine-tuning model.

Further discussions are needed in normative work in regards of modelling to represent the capability of a fine-tuning ML model.

### 5.1.4	ML model training for multiple contexts

#### 5.1.4.1	Description

ML models can be trained for similar contexts allowing for efficient management of these ML models. If an ML model that is trained for a particular context needs to be updated, it can be realized using an ML model that was trained for different context as the baseline.

#### 5.1.4.2	Use cases

##### 5.1.4.2.1	ML model training for multiple contexts

Although the ML model may provide an AI/ML inference service for multiple scenarios, there are similarities in the contexts where ML models operate and perform AI/ML inferences. For e.g. two ML model instances for the same inference type in urban or rural areas would have significant overlap in their contexts. The context similarity can be leveraged in forming a cluster of ML models, where ML model instances in the cluster are either trained from the same previously trained ML model or from an ML model previously trained for another similar context as the baseline. The training of an ML model for multiple contexts allows for efficiency by cluster training rather than individually training each one of them. ML training needs to support the capability to train the cluster of ML models from the same baseline ML model or from an ML model previously trained for another similar context as the baseline. As input to the training, the clustering criteria needed to distinguish the ML model instances may be provided by the MnS consumer.

In the case of degradation of ML models, updating of ML models is expected to be triggered. For ML models created through cluster training, the retraining of a degraded ML model could be triggered to start from another member of the cluster, i.e. start from an ML model with another context to create a new ML model with the desired context.

#### 5.1.4.3	Potential Requirements

REQ-ML_CLUSTER-TRAIN-1: The MLT MnS producer should have a capability for an authorized MnS consumer to request training of a cluster of ML models associated to a set of multiple contexts from a previously trained ML model.

#### 5.1.4.4	Possible solutions

The MLTrainingRequest IOC has to be extended with:

-	a flag indicating that a cluster of ML models has to be trained.

-	a clustering criteria indicating which ML models can form the cluster. This may indicate context similarities, e.g. similar geographical location, training data similarities etc. Take traffic congestion analysis as example, two ML model instances for one specific inference type during peak and off-peak hours would have significant geographical similarity in their contexts, these two models can be placed in one cluster. The DN of the ML instance to be used as the baseline to train another ML model with a related context.

-	the expectedRunTimeContext can be used to capture the context of new ML models that should be trained.

#### 5.1.4.5	Evaluation

The solution described in clause 5.1.4.4 proposes the addition of new attributes to the MLTrainingRequest IoC to enable the MnS consumer to request training of a cluster of ML models associated to a set of multiple contexts from a previously trained ML model. Therefore, the solution described in clause 5.1.4.4 is a feasible solution to be developed further in the normative specifications.

The feasibility of using mLModelCoordinationGroup related information elements, defined in TS 28.105 [2], to implement solution #2 requires further investigation in the normative work.

### 5.1.5	ML training data statistics

#### 5.1.5.1	Description

During ML training, it is important to ensure that training data is as uniform and representative as possible, and outliers are handled appropriately during the data pre-processing so as to train robust ML models that do not require frequent re-training. The MnS consumer may request the ML training producer to train the ML model with specific training data statistical properties.

#### 5.1.5.2	Use Cases

##### 5.1.5.2.1		Training data statistical properties for ML training

Non-uniform distribution of training data can significantly degrade the performance of trained ML models. The ML model may learn and reflect this non-uniformity, rather than the true underlying patterns in the data. This could lead to inaccurate predictions when the ML model is deployed for inference. Furthermore, an ML model trained on non-uniform data may not generalize well to new unseen data, as the training data might not accurately represent the full range of possible inputs the ML model may encounter during inference. This can limit the usefulness of the ML model.

Similarly, outliers in training data can significantly degrade the performance of trained ML model. ML models learn to make predictions based on the patterns they identify in the training data. Outliers can skew these patterns and lead to an ML model that is biased towards these extreme values, rather than accurately reflecting the majority of the data.

Therefore, it is crucial to ensure that training data is as uniform and representative as possible, and outliers are handled appropriately during the data pre-processing so as to train robust ML models that do not require frequent re-training.

#### 5.1.5.3	Potential requirements

REQ-DATA-STAT-1: The 3GPP management system should enable an authorized consumer to provide information on the training dataset distribution that the ML Training MnS producer should take into account for training an ML model.

REQ-DATA-STAT-2: The 3GPP management system should enable an authorized consumer to provide information on the usage of outliers in the training dataset that the ML Training MnS producer should take into account for training an ML model.

#### 5.1.5.4	Possible solutions

1) Introduce a new attribute of type, e.g. datasetStatisticalProperties, in the MLTrainingRequest IoC requested by an authorized MnS Consumer to the ML Training MnS Producer. This information influences the training data selection mechanisms to be used for training an ML model, assuring that the trained ML model reflects the statistical characteristics of the data that are relevant for the aIMLinferenceName.

The proposed datasetStatisticalProperties datatype may include the following two attributes:

UniformlyDistributedTrainingData: It indicates the need for using training data that are uniformly distributed according to the different aspects of the aIMLinferenceName. The type of this attribute is Boolean. For e.g. in case of coverage problem analytics, training data samples for gNBs belonging to each geographical location area needs to be uniformly distributed for the ML model to correctly determine in which geographical location areas the coverage problem occurred during inference. When non-uniform distribution data samples are used to train the ML model and deployed to the coverage analysis inference function, it could lead to inaccurate predictions for the coverage problems.

TrainingDataWithOrWithoutOutliers: It indicates that the training data samples should consider or disregard data samples that are at the extreme boundaries of the value range. The type of this attribute is Boolean. For e.g. in case of coverage problem analytics, if there are only few training data samples belonging to a particular geographical location, they may be considered as outliers with respect to the other training data samples.

#### 5.1.5.5	Evaluation

The solution described in clause 5.1.5.4 proposes the addition of two new attributes to the MLTrainingRequest IoC to enable the MnS consumer to indicate the training data statistical properties allowed to be used by the MnS producer for training an ML model. Therefore, the solution described in clause 5.1.5.4 is a feasible solution to be developed further in the normative specifications.

### 5.1.6	ML model confidence

#### 5.1.6.1	Description

Confidence is a measure of the probability that a prediction made by the ML model is correct. It is often expressed as a percentage, with 100 % being absolute certainty and 0 % being absolute uncertainty. Confidence score can be useful in prediction-making processes, where a certain level of confidence might be required before taking action based on the ML model's prediction. However, it is important to note that a high confidence score does not guarantee the ML model's prediction is correct, but it only indicates the ML model belief in its prediction. In simpler terms, confidence is a measure of the ML model's certainty about its prediction. It is not a measure of the ML model's accuracy or correctness. An ML model can be very confident about a prediction and still be wrong. Conversely, an ML model can be less confident about a prediction and still be correct.

#### 5.1.6.2	Use Cases

##### 5.1.6.2.1	Model Confidence Threshold in ML Training

In the ML training step, the ML training function may support the capability to evaluate the confidence value of the trained ML model. The ML model confidence value indicates the average confidence value (in unit of percentage) that the ML model would perform for data with the same distribution as training data. Essentially, this is a measure of degree of the convergence of the trained ML model. Therefore, the MnS consumer may prefer to indicate the confidence threshold value that the trained ML model should satisfy before the ML model is deployed for inference.

#### 5.1.6.3	Potential requirements

REQ-MODEL_CONF-1: The AI/ML training MnS producer should have a capability allowing an authorized MnS consumer to indicate the minimum average confidence value that the AI/ML MnS producer should meet on the trained ML model.

#### 5.1.6.4	Possible solutions

Introduce a new attribute, e.g. modelConfidenceIndication, in the MLTrainingRequest IoC requested by an authorized MnS consumer to the ML Training MnS Producer. This attribute indicates to the ML Training MnS Producer the minimum average confidence value (in unit of percentage) that the ML model should meet on the data with the same distribution as training data. The attribute is of type real with allowed values {0…100}.

#### 5.1.6.5	Evaluation

The solution described in clause 5.1.6.4 proposes the addition of a new attribute to the MLTrainingRequest IoC to enable the MnS consumer to indicate the minimum average confidence value that the MnS producer should meet for training an ML model. Therefore, the solution described in clause 5.1.6.4 is a feasible solution to be developed further in the normative specifications.

The feasibility of using existing information elements in TS 28.105 [2] to implement the solution requires further investigation in the normative work.

### 5.1.7	Management of Reinforcement Learning

#### 5.1.7.1	Description

Reinforcement Learning (RL) is a type of machine learning where an agent learns to make decisions by taking actions in an environment to achieve a goal. The agent learns from the consequences of its actions, rather than from being explicitly taught and it selects its actions based on its past experiences (exploitation) and also by new choices (exploration), which is essentially a trial-and-error approach. In RL process, an agent interacts with the environment in discrete time steps. At each time step, the RL agent receives the current state of the environment and selects actions to be taken. The environment provides the new state in turn and the RL agent will receive the reward based on the state of the environment. For example, RL is applied for coverage problem analysis use case, the RL agent actions are used to optimize the coverage problem, and the RL environment can be the simulation environment. The actions can be changes to the values of network adjustable parameters (e.g. change the transmission power of the NR sector carrier, see TS 28.104 [4]). The states can be the network PMs/KPIs like RSRP distribution etc. The reward can be the score of an RL performance metric to evaluate the PMs/KPIs. The goal of the agent is to learn a policy, which tells it what action to take under what circumstances, that maximizes the sum of rewards. The main advantage of this approach is in the ability to automatically adapt to the characteristics of the environment, making it suitable approach for handling dynamic environments such as mobile networks.

Figure 5.1.7.1-1: Example diagram of Reinforcement Learning

#### 5.1.7.2	Use cases

##### 5.1.7.2.1	Exploration in Reinforcement Learning

Reinforcement Learning (RL) has the ability to learn and adapt itself to dynamic environments and thus finds the near optimal solution to the problem. This makes the RL-based approaches very interesting for applications in the mobile networks. However, the potential negative impact to the mobile network caused by RL is still the main drawback. In particular, during the exploration step performing trials and learning from errors may have an impact on the operational network and may result in unsafe operations causing network performance degradations. Therefore, the exploration step in RL needs to be under a controlled environment or a stable training configuration (higher exploration rate, i.e., more exploration rounds, always results in a higher training efficiency with a more severe network performance degradation) that is not supposed to violate system performance requirements. If the RL agent behaves in an unexpected manner, there needs to be a set of fall-back actions in place, e.g. to switch from RL-based solution to non-RL-based solution, to fall back to last discrete time step, and to terminate the RL process.

For RL management, the MnS consumer can query the ML Training MnS producer to discover if RL is supported. When RL is supported, a consumer may want to provide a scope (e.g. geographical area, time window) that can aid the producer to select/create the environment when performing RL. The environment scope may include information of the entities where the ML model is involved acting as RL agent. In the event RL is supported, the consumer may also want to state their preference for environment type for RL during training i.e. simulated environment or real network. When the real network is preferred by the MnS consumer, the consumer can provide network performance requirements (e.g. lower bound threshold, acceptable range, maximum performance deterioration Rate, etc.) of performing ML training of RL, so as to make the MnS producer adapt the training configurations to meet the network performance requirements. Furthermore, the consumer can provide its concerned performance metrics to guide the producer to set reward/state.

NOTE:	 Support for both environment types can be considered optional in the RL training.

##### 5.1.7.2.2	Training Conflict in Reinforcement Learning

The training process of RL is realized by the actions with their impacts to the RL environment. In the online training, if there are multiple RL training processes (of multiple ML models for different AI/ML inference functions) sharing a same RL environment, simultaneously, they will interfere with each other, which may cause the training conflict. To be specific, if multiple ML training processes of RL have conflicts, their agents may make actions at the same time, then the state of the RL environment will be affected by these actions. This kind of training error will result in the performance loss of the trained ML models, even cause the training process difficult to converge.

For example, for the training processes whose related inference functions are load balancing optimization (LBO) and mobility robustness optimization (MRO), their RL environment states both include the cell load while the actions from the two RL agents both include the Cell Individual Offset (CIO) adjustment to optimize the handover decisions. If these two RL training processes are processed at the same time with a same environment for an MnS producer, the loads of the adjacent cells will be affected by both training processes, thus causing the training conflict.

For control the conflict of reinforcement learning, the MnS consumer should know whether there are conflicts during the RL training. The producer should determine the conflict and provide the training conflict indication in RL training to authorized MnS consumer. The MnS consumer may specify the conflict resolution requirements to producer. For example, the consumer can cancel/suspend some training processes or configure conflict resolution requirements in advance.

#### 5.1.7.3	Potential Requirements

REQ-RL_MGMT-01: The ML training MnS producer should have a capability allowing an authorized MnS consumer to query if RL training is supported.

REQ-RL_MGMT-02: The ML training MnS producer should have a capability allowing an authorized MnS consumer to specify the preferred RL environment type.

REQ-RL_MGMT-03: The ML training MnS producer should have a capability to allow an authorized MnS consumer to specify the preferred RL environment scope.

REQ-RL_MGMT-04: The ML training MnS producer should have a capability to allow an authorized MnS consumer to specify RL configuration scope for exploration in reinforcement learning.

REQ-RL_MGMT-05: The ML training MnS producer should have a capability allowing an authorized MnS consumer to provide network performance requirements of performing RL training.

REQ-RL_MGMT-06: The ML training MnS producer should have a capability to provide the training conflict indication during RL online training to authorized MnS consumer.

REQ-RL_MGMT-07: The ML training MnS producer should have a capability allowing an authorized MnS consumer to specify the conflict resolution requirements during RL online training.

#### 5.1.7.4	Possible solutions

##### 5.1.7.4.1	Possible solution #1: Exploration in Reinforcement Learning

This solution proposes to enhance the existing MLTrainingRequest IOC and MLTrainingReport IOC to allow the RL consumer to specify RL requirement including environment selection and performance requirements.

Enhancement on MLTrainingFunction IOC:

Extend the existing IOC MLTrainingFunction with an attribute whose datatype is the supported learning technology including RL training, indicating the supported RL environment type (e.g. RL on simulation environments, RL on the real network) of the MnS producer.

Enhancement on MLTrainingRequest IOC: Introduce a new <<datatype>> "RLRequirement" to the MLTrainingRequest IOC, which may include following attributes:

-	RLEnviromentType, represents required RL environment type. The allowed values may be "Online", "Offline" representing real-network and simulation network respectively.

-	RLEnviromentScope, represents RL environment scope, which may be a RL geographical area and/or network node(s), time window and required network load. The scope dose not only include the entities directly involved in RL process, but also includes other entities impacted by RL agent actions.

-	RLConfigurationScope, e.g. the range of actions that the agent is allowed to take.

-	RLPerformanceRequirements, represents the attribute of the network performance requirements for online ML training, which indicates the tolerable network performance degradation (e.g. minimum or maximum performance threshold, acceptable range or maximum performance deterioration ratio). When the network performance is within the range, the RL training process can be continued. Otherwise, fall back actions can be determined by the producer. This information can be used to decide the appropriate rewards and state settings.

##### 5.1.7.4.2	Possible solution #2: Training Conflict in Reinforcement Learning

The existing MLTrainingProcess IOC or MLTrainingReport should be enhanced to support the management of reinforcement training conflict.

When training process detect the conflicts, it suspends the process and generate a conflict information.

Option 1: extend MLTrainingProcess IOC with this conflict information. The producer writes the conflict information and notify to the consumer.

Option 2: extend MLTrainingReport IOC with this conflict information. The producer writes the conflict information into the training report and notify the report to the consumer.

The MnS consumer can get this attribute from multiple MLTrainingProcess MOIs or MLTrainingReport MOI then the consumer can determine the conflict training processes. The MnS consumer can modify the existing cancelProcess or suspendProcess attribute to manage the training processes, so as to avoid training conflicts.

#### 5.1.7.5	Evaluation

##### 5.1.7.5.1	Exploration in Reinforcement Learning

The solution described in clause 5.1.7.4.1 proposes enhancements to several IoCs to facilitate the management of reinforcement learning:

-	adding support for RL in the MLTrainingFunction IOC; and

-	enhance the MLTrainingRequest IOC to enable the MnS consumer to indicate the preferred RL environment type, RL environment scope, RL configuration scope and RL performance requirements to the MnS producer.

Therefore, the solution described in clause 5.1.7.4.1 is feasible to be developed further in the normative specifications.

##### 5.1.7.5.2	Training Conflict in Reinforcement Learning

The solution described in clause 5.1.7.4.1 proposes to enhance with the conflict information to the MLTrainingProcess or MLTrainingReport IOC to indicate to support the management of reinforcement training conflict. Conflicts are discovered during training and are therefore most appropriate to be resolved during training. Therefore, the option1 solution described in clause 5.1.7.4.1 is a feasible solution to be developed further in the normative specifications.

### 5.1.8	Sustainable AI/ML for ML training

#### 5.1.8.1	Description

Sustainable AI/ML aims to reduce energy usage in the ML model training step. It is important to investigate effective methods for evaluating AI/ML energy consumption and efficiency for ML model training which will aid in decreasing energy consumption, enhancing energy efficiency, and promoting sustainable development of AI/ML in 5G systems.

#### 5.1.8.2	Use cases

##### 5.1.8.2.1	AI/ML energy consumption evaluation and reporting for ML model training

AI/ML energy consumption is a significant concern, especially as ML models become more complex and data intensive. The ML model training is typically the most energy-intensive part since it involves processing large datasets and performing numerous calculations to adjust the ML model parameters.

The MnS consumer may have concern on the energy consumption for ML model training and the MnS Producer should report the related information. MnS Consumer may ask the MnS Producer to report the energy consumption of training each ML model. To satisfy such query request from the MnS consumer, methods should be investigated to measure AIML energy consumption. Then, MnS Consumer can use the energy consumption information and compare with the network performance gain due to ML model to evaluate the total benefit of using the ML model. Alternatively, the MnS consumer may ask the MnS Producer to report the number of floating-point operations for training each ML model.

Regarding the various AI/ML energy consumption aspects outlined in clause 4.2, ML model training may be characterized by several factors including:

-	Number of cycles, a low number of cycles generally contributes to reduced energy consumption.

-	Dataset size, large datasets generally contribute to increased energy consumption due to intensive data processing.

-	Number of floating-point operations, complex ML model contributes to increased energy consumption.

-	Deployment on high-end hardware platform, which can either increase energy consumption due to their substantial power requirements, or reduce it by offering higher energy efficiency through optimized performance.

#### 5.1.8.3	Potential Requirements

REQ-AIML_TRAIN_ENERGY_MGT-01: The MLT MnS producer should have a capability to allow an authorized MnS consumer to query the number of floating-point operations pertaining to training each ML model.

REQ-AIML_TRAIN_ENERGY_MGT-02: The MLT MnS producer should have a capability to report the number of floating-point operations pertaining to training each ML model to an authorized consumer.

#### 5.1.8.4	Possible solutions

##### 5.1.8.4.1	Possible solution #1

The IoC MLTrainingReport needs to be enhanced with a new attribute related to energy consumption for training each ML model, e.g. modelEnergyConsumptionTraining. This attribute is of type real and indicates the energy consumption value pertaining to the ML model training, e.g. kilowatt-hour "kWh".

NOTE:	 The method to determine energy consumption for ML model training is for further discussions. If energy consumption for ML model training cannot be determined, the proposed possible solution #1 in 5.1.8.4.1 becomes redundant.

##### 5.1.8.4.2	Possible solution #2

The IoC MLTrainingReport needs to be enhanced with a new attribute, e.g. flopsTraining, to indicate the number of floating-point operations needed to train an ML model. This attribute is of type integer.

#### 5.1.8.5	Evaluation

Solution 1 in clause 5.1.8.4.1 proposes adding an attribute to the IOC MLTrainingReport that indicates the energy consumption for training an ML model. There is also a note that the method to determine energy consumption is for further discussions and if this cannot be determined the solution becomes redundant. The method for determining energy consumption has not been determined as part of the present document, therefore no recommendations are made for this solution to be directly used in normative phase.

Solution 2 described in clause 5.1.8.4.2 proposes the addition of a new attribute to the MLTrainingReport IoC to enable the MnS producer to report the number of floating-point operations needed to train an ML model which is a feasible solution to be developed further in the normative specifications.

### 5.1.9	ML model distributed training

#### 5.1.9.1	Description

Distributed training is a model training paradigm that involves spreading training workload across multiple training functions, to accelerate the training process and/or reduce the required computational resources. Distributed training can be used for traditional machine learning models, as well as for large models.

In 5GS, the ML training function may be located within the management system or in the NF (e.g. gNB or NWDAF), i.e. the worker node for training. Each node has different computing resources and storage capacity based on physical infrastructure such as CPU/GPU/DPU, memory, storage, and network bandwidth. In order to obtain load balance between nodes and maximize the efficiency of resource utilization, splitting up the training may be necessary and involving multiple training functions according to the actual situation of nodes may be needed. Thus, aspects of distributed training need to be supported in the management systems.

#### 5.1.9.2	Use cases

##### 5.1.9.2.1	ML model distributed training

In 5GS, the MnS consumer may require speeding up the training process under certain conditions (e.g. the size of the model may be too large for a single training function).

When receiving an ML training request, the MLT MnS producer may evaluate whether distributed training is needed according to the training requirements provided by the ML training consumer, and it is up to the MLT MnS producer to determine appropriate training function(s) which need to participate in the ML model training. Collaboration and mutual agreement may require between distributed ML training functions.

The actions of ML model distributed training may involve for example, splitting the training of an ML model across many ML training functions, each responsible for computing a portion of the model's operations.

NOTE 1:	How to split the ML model and synchronize the parameters in different training function depends on the distributed algorithm which are proprietary and not in scope for standardization.

NOTE 2: The data exchange between different training functions should be in the security tunnel with appropriate authentication and authorization mechanisms.

#### 5.1.9.3	Potential requirements

REQ-ML_DIST-TRNG-01:	The ML training MnS producer should have a capability allowing the authorized consumer to provide distributed training requirements to the MnS Producer.

#### 5.1.9.4	Possible solutions

##### 5.1.9.4.1	ML model distributed training

This solution uses the instances of following IOCs for interaction between ML Training MnS producer and consumer to support the management of ML model distributed training.

-	New attribute name "isDistributedTraningSupported" for MLTrainingFunction IOC (see TS 28.105 [2]) to indicate whether the ML Training function could support ML model distributed training.

-	Introduce a new datatype for MLTrainingRequest IOC to capture the requirement from the consumer which may contain the following attributes:

-	Attributes which indicate the requirement for distributed training, e.g. total training time, computing resources etc.

-	"suggestedTrainingNode" list which indicate suggested training nodes.

NOTE:	 The suggested NRM fragment structure may need to be refactored and aligned with other solutions in normative phase.

#### 5.1.9.5	Evaluation

The solution described in clause 5.1.9.4 proposes the addition of new attributes and new datatype to the MLTrainingRequest IOC to enable the MnS consumer train the ML model in a distributed way. The solution minimizes complexity and aligns with current standards. Therefore, the solution described in clause 5.1.9.4 is a feasible solution to be developed further in the normative specifications.

### 5.1.10	Management of Federated Learning

#### 5.1.10.1	Description

Federated Learning (FL) is a distributed machine learning approach that allows multiple FL clients to collaboratively train an ML model on local datasets contained in each FL client without explicitly exchanging data samples.

FL is supported by a group of FL clients and FL server wherein FL client keeps the data localized and private and trains the ML model directly on the local nodes (client) where the data is generated or stored.

Federated learning can be categorized into two main types: Horizontal federated learning (HFL) and Vertical Federated Learning (VFL), based on the nature of the data distribution and the way the model training is orchestrated among participants. For HFL, the process typically includes FL Client discovery and selection, and ML model distribution and aggregation.

Figure 5.1.10.1-1: ML model distribution and aggregation for HFL

NOTE:	 A prior agreement needs to exist between the FL server and the FL clients to exchange ML models.

#### 5.1.10.2	Use cases

##### 5.1.10.2.1	Management of different roles in Federated Learning

For FL, an ML model is collaboratively trained by a group of FL clients (e.g. MTLF in NWDAFs) including one acting as FL server and the others acting as FL clients. Federated Learning training allows multiple FL clients to collaboratively train an ML model on local datasets, it means that the local training of each FL client needs to start and complete almost at the same time to ensure the performance of ML model aggregation performing in FL server.

For managing the FL, the ML training MnS consumer needs to know the FL clients and FL server involved in the FL, so that the consumer understands the impact of each one of them and can manage it correspondingly.

When receiving an ML Training request, the ML Training MnS Producer should evaluate whether FL process needs to be started according to the training requirements (e.g. minimal number of FL Clients, minimal number of total iterations, minimal number of data samples and available time of the FL Clients) provided by the ML training consumer. Based on the received requirements, the FL server may select appropriate FL Clients.

To evaluate the performance of FL, the consumer needs to know the performance of the final global ML model running on the local training data set of participating FL clients. For instance, if an FL server cannot generate a global ML model with satisfied performance for the FL clients, the consumer may interact with the MnS ML training producer to optimize the FL for future training, e.g. updating the criteria for selecting FL clients.

In addition, the consumer needs to get the information about the contribution of each FL client to the FL process, for instance, number of iterations the FL client participated in the FL, number of data examples the FL client used, training duration the FL Client performed.

Maintaining a Federation Learning process in FL execution phase is helpful to make the FL training efficient, which may include reselection, addition, or removal of FL Client(s). The consumer needs to get the information of candidate FL clients and contribution of each FL client to maintain the FL process.

#### 5.1.10.3	Potential requirements

REQ-FL_MGMT-01: The ML training MnS producer should have a capability allowing an authorized consumer to discover the FL roles (FL server or FL client) in Federated Learning.

REQ-FL_MGMT-02: The ML training MnS producer should have a capability allowing an authorized consumer to provide FL training requirements to the MnS Producer.

REQ-FL_MGMT-03: The ML training MnS producer should have a capability allowing an authorized consumer to provide requirements for selecting FL clients in Federated Learning to the MnS Producer.

REQ-FL_MGMT-04: The ML training MnS producer should have a capability allowing an authorized consumer to get the performance of ML model on each participating FL client.

REQ-FL_MGMT-05: The ML training MnS producer should have a capability to report the information about the contribution of each FL client to the FL process to MnS consumer.

REQ-FL_MGMT-06: The ML training MnS producer should have a capability to report the candidate FL Clients for the FL process.

REQ-FL_MGMT-07: The ML training MnS producer should have a capability allowing an authorized consumer to reselect, add, or remove the FL Clients in the FL Process according to the candidate FL clients.

#### 5.1.10.4	Possible solutions

This solution uses the instances of following IOCs for interaction between ML Training MnS producer and consumer to support the management of Federated Learning.

1) The IOC MLTrainingFunction (see TS 28.105 [2]) extended with the following attributes:

-	the supported role for FL, which indicates the FL role (FL server or FL client) for each type of ML model (identified by ML model Id) for which the ML Training function participates into the ML training.

2) A new IOC containing the FL requirements under MLTrainingFunction (see TS 28.105 [2]). This IOC contains the following attributes:

-	FL client selection requirements for each type of ML model (identified by ML model Id) for which the ML training function acts as the FL server. The requirements could be minimal available data samples (e.g. the number of available data samples), minimal available time (e.g. the time window that the FL client is available for training), geographical location, minimal training performance score of the interim local ML model running on the local training data samples on the FL client.

The IOC is illustrated by IOC XYZ in figure 5.1.10.4-1 below.

Figure 5.1.10.4-1: Example of NRM fragment for FL requirements

3) The IOC MLTrainingReport (see TS 28.105 [2]) extended with the following attributes:

-	FL result, including the information about the contribution of each participating FL client to the FL process, (e.g. number of training iterations each FL client has participated in), and the performance score of the final global ML model running on the local training data set on each FL client.

-	candidateFLClientID, indicates the DN of the candidate FL client.

NOTE:	 The criterion to calculate the contribution of each FL Client is to be further discussed in the normative phase.

#### 5.1.10.5	Evaluation

The solution described in clause 5.1.10.4 proposes to enhance the MLTrainingFunction and MLTrainingReport IOC to support the management of different roles in Federated Learning. However, only management of HFL is supported in the possible solution and it may be further enhanced to support VFL in the future release. The introduced solution aligns with the current standards, which is feasible to be developed further in the normative specifications.

### 5.1.11	ML Authentication

#### 5.1.11.1	Description

Authentication is a fundamental requirement for ensuring the integrity and security of the ML model training process. It is imperative that each participating entity, whether operating within a centralized, distributed, or federated learning environment, is authenticated is using a secure and reliable method. This practice serves to prevent unauthenticated access and guarantees that only verified entities will participate in the ML training process, thereby maintaining integrity, accuracy and reliability of the overall ML model.

As part of the training process, each participating entity should authenticate. The ML training function is responsible for validating these authentications to ensure that all entities participating on the ML Training process are verified.

The existing authentication mechanisms specified in TS 28.533 [7], clause 4.9.1, provide the necessary framework for ML authentication. These outline robust authentication services, including the use of certificates, tokens, or mutual authentication protocols, ensuring that only authenticated entities are permitted to contribute to the training process.

#### 5.1.11.2	Potential Requirements

REQ-ML_AUTH-01: The ML training MnS producer should support the capability to authenticate all participating entities involved in a training process.

#### 5.1.11.3	Possible Solution

The authentication mechanisms specified in TS 28.533 [7], clause 4.9.1, provide the necessary framework for ML authentication in centralized, distributed, and federated learning environments. These mechanisms include secure methods such as certificates, tokens, and mutual authentication protocols, ensuring that only authorized entities participate in the ML model training process.

#### 5.1.11.4	Evaluation

The solution in clause 5.1.11.3 is feasible, offering a simple and efficient approach by reusing the existing authentication mechanisms from TS 28.533 [7]. This approach minimizes complexity and requires no additional development in the normative specifications. Any more advanced solutions, if necessary, would require further investigation. Bottom of Form

### 5.1.12	AI/ML prediction latency

#### 5.1.12.1	Description

AI/ML prediction latency refers to the time it takes for an ML model to process an input and produce an output. This is a critical metric for real-time AI/ML enabled use cases where quick responses are essential. It is important to have the awareness of AI/ML prediction latency as well as to allow consumer to indicate the requirements regarding the AI/ML prediction latency.

#### 5.1.12.2	Use cases

##### 5.1.12.2.1	AI/ML prediction latency during ML model training

Depending on the use case, the AI/ML training MnS consumer specifies ML model performance requirements that need to be considered during the training process by the ML training MnS producer. For use cases with stringent time constraints, these requirements may include specific targets for the latency of AI/ML prediction computations.

To meet these latency requirements, the ML training MnS producer can employ various optimization techniques during model training (e.g. quantization, pruning).

By carefully selecting and applying optimization techniques, the ML training MnS producer can ensure that the trained ML model meets the specified latency requirements, thereby enabling real-time performance in critical use cases.

#### 5.1.12.3	Potential requirements

REQ-AI/ML_Train_Latency-1: The 3GPP management system should have a capability allowing an authorized MnS consumer to provide the accepted AI/ML prediction latency requirements.

#### 5.1.12.4	Possible solutions

Introduce a new attribute, e.g. expectedPredictionLatency, in the MLTrainingRequest IoC requested by an authorized MnS consumer to the ML training MnS producer. This attribute is an upperbound on the duration that it should take for an ML model prediction. To meet these latency requirements, the ML training MnS producer can employ various optimization techniques during model training (e.g. quantization, pruning) which is internal to the ML training MnS producer.

#### 5.1.12.5	Evaluation

The solution described in clause 5.1.12.4 proposes the addition of new attributes to the MLTrainingRequest IoC to enable the MnS consumer to request AI/ML prediction latency from the ML training MnS producer. Therefore, the solution described in clause 5.1.12.4 is a feasible solution to be developed further in the normative specifications.

## 5.2	AI/ML inference emulation

### 5.2.1	ML inference emulation

#### 5.2.1.1	Description

A trained ML model can be used for inference within the stated scope e.g. on a managed function or in a management function. Accordingly, there may be an AI/ML inference MnS producer that is responsible for executing the inference process.

#### 5.2.1.2	Use cases

##### 5.2.1.2.1	AI/ML inference emulation

After an ML model is trained, validation is done to ensure the training process is completed successfully. Typically, validation is done by preserving part of the training data set and using it after training to check whether the ML model has been trained correctly. However, even after the ML model is validated during development, inference emulation is necessary to check if the ML model is working correctly under certain runtime context or using certain inference emulation data set.

In principle, the two operations are similar on a functional level, where both of them check the ML performance against given context or data to ensure the ML functionality is performing correctly. However, inference emulation involves interactions with third parties, e.g. the operators who use the ML model or third-party systems that may rely on the results computed by the ML model.

For these reasons, it is necessary to support inference emulation, specifically to support means:

-	Enabling a given MnS consumer to request for a specific AI/ML capability to be executed in ML inference emulator environment.

-	For a given MnS consumer to request a specific ML inference emulator to execute a given AI/ML capability.

-	For a managed function to act as a ML inference emulator and execute AI/ML capabilities in a controlled way.

The network or its management system needs to have the capabilities and provide the services needed to enable the MnS consumer to request inference emulation and receive feedback on the inference emulation of a specific ML model or of an application or function that contains an ML model.

##### 5.2.1.2.2	Managing ML inference emulation

The 3GPP management system may have resources for multiple emulation environments to be used depending on need. These may include simulation environments, a digital twin of the network, a test network or even the real network under curtain constrained conditions, e.g. for a selected set of UEs. The different emulation environments may represent varying levels of trust that the operator or management system has in the ML model or AI/ML inference functions. Accordingly, 3GPP management system needs to have means and method for orchestrating the inference emulation which could be an inference emulation orchestrator or an inference emulation function.

Key aspects of the orchestration may include:

-	the emulation progression process involves selecting the appropriate type and instance of an emulation environment to test an ML model, AI/ML inference function or its actions, based on the testing needs and the available emulation environments and their resources

-	the emulation process may also involve executing the ML model, AI/ML inference function or its actions on the real network but in a controlled manner, e.g. only within certain hours or only on cells with a particular type of load or only on cells in a particular geographical area or in limited subscriber groups.

Accordingly, the actions performed by the inference emulation function may include:

-	Controlling the allowed parameter space/ranges of the parameters optimized by the ML model or AI/ML inference function depending on the emulation environment to which the ML model, AI/ML inference function or the actions are being executed.

-	Adjusting the parameter space taking into consideration the observed behaviour of the ML model or AI/ML inference function.

-	Deploying the actions of the ML model or AI/ML inference function on a selected emulation environment or the real network.

-	Blocking the ML model or AI/ML inference function from being deployed on the network.

#### 5.2.1.3	Potential requirements

REQ-AI/ML_EMUL-1: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to query the available emulation environment(s).

REQ-AI/ML_EMUL-2: The MnS producer for AI/ML inference emulation should have a capability to inform an authorized MnS consumer of the available emulation environment(s).

REQ-AI/ML_EMUL-3: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to request an ML inference emulation for a specific ML model or models.

REQ-AI/ML_EMUL-4: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to request for ML inference emulation for a specific ML model using specified data or data with specifically stated characteristics and inference emulation features.

REQ-AI/ML_EMUL-5: The MnS producer for AI/ML inference emulation should have a capability to inform authorized MnS consumer about the status of the emulation of an ML model under emulation.

REQ-AI/ML_EMUL-6: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer (e.g. an operator) to manage or control a specific ML inference emulation process, e.g. to start, suspend or restart the inference emulation; or to adjust the inference emulation conditions or characteristics.

REQ-AI/ML_EMUL-7: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to request reporting and receive reports on the progress and outcome of an emulation process.

REQ-AI/ML_EMUL-8: The MnS producer for AI/ML inference emulation should have a capability to allow an authorized MnS consumer to configure an ML model or AI/ML inference function supporting with a level of trust that expresses the degree to which the ML model or AI/ML inference function or its actions have been verified as trusted.

REQ-AI/ML_EMUL-9: The MnS producer for AI/ML inference emulation should have a capability to graduate an ML model, AI/ML inference function, or its actions through different levels of trust, each indicating the degree to which the ML model, AI/ML inference function or actions have been verified as trusted.

#### 5.2.1.4	Possible solutions

1) Introduce an IOC with the properties of the ML inference emulation function. This may be termed as an MLInferenceEmulationFunction to be name-contained in either a Subnetwork, a ManagedFunction or a ManagementFunction. The MLInferenceEmulationFunction may be a separate function or may be name-contained in an inference function.

This IOC contains attributes including the following:

-	list of the ML models which can be emulated and possibly in which emulation environments

-	indication of progression of the MLModel or AI/ML inference function that is under emulation to indicate the degree to which the inference has been emulated and trusted,

-	different characteristics for which different emulations may be supported,

-	a hierarchy for different emulation environments, e.g. an emulator that uses only a test network vs. an emulator that activates the actions in the real network at specified time such as the maintenance window.

2) Introduce an IOC representing an available emulation environment, e.g. a new IOC named as AvailableEmulationEnvironment, or EmulationSubNetwork, or the existing SubNetwork IOC with an attribute indicating it is a subnetwork used for emulation.

The instance of this IOC is created by the MnS producer to allow the consumer to query, or be informed of, the information of the available emulation environments. This IOC contains the following properties (e.g. the subordinated IOC or attributes):

-	inference functions or ML models under emulation.

3) Introduce an IOC for the request for ML inference emulation which shall capture the consumer's requirements for inference emulation. This may be named as an MLInferenceEmulationRequest to be name-contained by the MLInferenceEmulationFunction.

The MLInferenceEmulationRequest shall be associated with at least 1 MLModel for which the inference emulation is being executed.

This IOC contains attributes including the following:

-	identifier of the ML models requested for emulation,

-	identifier of the selected emulation environment,

-	time window for the emulation.

Each MLInferenceEmulationRequest may have a RequestStatus field that is used to track the status of the specific MLInferenceEmulationRequest or the associated MLInferenceEmulationProcess. The RequestStatus is an enumeration with the possible values as: "Pending" if no action has been taken for the request; "Triggered" when an MLInferenceEmulationProcess has been instantiated; "Suspended" when the request or its job has been suspended by MnS consumer or producer and "Served" when the job has run to completion.

4) Introduce an IOC for the process of ML inference emulation from the objects for inference emulation shall be instantiated. This may be named as an MLInferenceEmulationProcess to be name-contained by the MLInferenceEmulationfunction.

The MLInferenceEmulationProcess shall be associated with at least one MLModel for which the inference emulation is being executed.

This IOC contains the following attributes:

-	progress indicator,

-	identifier of the corresponding emulation request.

5) Introduce a report on ML inference emulation, which may provide reporting on ML emulation for one or more MLInferenceEmulationRequests or MLInferenceEmulationProcesses. This report may be equivalent to the inference report, and:

-	the ML inference emulation report may indicate the emulation environments in which the model has been emulated and passed

-	the ML inference emulation report may also include the specific performance metrics for that emulation environments.

May also introduce the IOCs and datatypes for request, process and report on ML Inference and then or update these IOCs with the features of the ML Inference Emulation as introduced above.

6) The IOCs, attributes and performance measurements for the solutions of performance evaluation for AI/ML inference for inference step, which can be reused for monitoring the inference performance of the ML models during the emulation.

7) The IOCs and attributes for configuration management of 5G system, as defined in 3GPP TS 28.541 [5], which can be reused for configuring the emulation environment.

#### 5.2.1.5	Evaluation

The solution described in clause 5.2.1.4 reuses the existing provisioning MnS operations and notifications in combination with extensions of the NRM. Requests for inference emulation for a given ML model may be instantiated using provisioning management service implemented via CRUD (Create, Read, Update, Delete) operations on the request objects. The solution provides the flexibility to allow any function that can execute n ML model to be the MnS producer for ML inference emulation, e.g. an inference function or a generic sandbox function.

Therefore, the solution described in clause 5.2.1.4 is a feasible solution to be developed further in the normative specifications.

### 5.2.2	ML inference emulation environment selection

#### 5.2.2.1	Description

Void.

#### 5.2.2.2	Use cases

##### 5.2.2.2.1	ML inference emulation environment selection

Although an ML model may be well-trained, its performance in the production network can be difficult to predict and guarantee because the training environment and production network are not identical. If a trained or tested ML model is directly applied to the production network, it may negatively impact the production network.

ML emulation involves applying the ML model in an emulation environment to verify whether its performance meets the expected inference characteristics. The MnS inference consumer may want to receive results of ML emulation to decide whether the ML model is suitable for deployment in the production network.

The management system should have the capability to enable an MnS consumer to select the appropriate emulation environment and provide the necessary configuration properties related to that environment. The configuration information may include defining the scope of the simulation, the simulation time, and other relevant parameters.

#### 5.2.2.3	Potential requirements

REQ-EMUL-1: The MnS producer for AI/ML inference emulation should have a capability enabling an authorized MnS consumer to select the emulation environment.

#### 5.2.2.4	Possible solutions

Introduce an attribute in the MLInferenceEmulationFunction IOC, or an IOC under MLInferenceEmulationFunction IOC to provide the available emulation environments that the ML emulation function supports. With this information, the ML inference emulation MnS consumer will be able to select the emulation environment to emulate the AI/ML inference for the specified ML model(s).

This new attribute or IOC indicates the list of available environments, and each environment is a defined by a structure including a name or id, description, configuration properties, and the status.

#### 5.2.2.5	Evaluation

The solution described in clause 5.2.2.4 reuses the existing provisioning MnS operations and proposes an attribute or IOC indicating the list of available environments. Therefore, the solution described in clause 5.2.2.4 is a feasible solution to be developed further in the normative specifications.

## 5.3	AI/ML deployment

### 5.3.1	Enhance the ML model loading use case

#### 5.3.1.1	Description

In TS 28.105 [2], the existing use cases (Consumer requested ML model loading in clause 6.4.1.2.1 and Control of producer-initiated ML model loading in clause 6.4.1.2.2) are described to support loading one ML model.

However, the consumer may want to load more than one ML model. The MnS producer can have a capability allowing an authorized consumer to request to load and manage the loading process for one or more ML model(s).

#### 5.3.1.2	Use cases

#### 5.3.1.3	Potential requirements

REQ-ML_LOAD-1: The MnS producer for ML model loading should have a capability allowing an authorized consumer to request to trigger loading of one or more ML model(s).

#### 5.3.1.4	Possible solutions

Enhance the NRM fragment for ML model loading by changing the multiplicity from "1" to "*" for the relationships.

-	MLModelLoadingRequest to MLmodel

-	MLModelLoadingProcess to MLmodel

### 5.3.2 	Managing ML Model Transfer/delivery

#### 5.3.2.1 	Description

The ML Model transfer/delivery clause 7.2.1.4 in TR 38.843 [6] addresses number of potential solutions for model delivery/transfer to the UE, including the following solution:

"Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE."

A note is also included in clause 7.2.1.4 of TR 38.843 [5] for solution 4b:

"Note: For Solution 4b, RAN2 discussed the following two solutions but did not study or analyse their feasibility:
- OAM can transfer/deliver AI/ML models to UE via OAM→RAN→UE, where Control Plane (CP) signalling is used for RAN→UE.
- OAM can transfer/deliver AI/ML models to UE via OAM→UE, e.g. via IP tunnel."

According to RAN, AI/ML model transfer refers to the delivery of an AI/ML model over the air interface while AI/ML model delivery refers to delivery of an AI/ML model from one entity to another.

NOTE:	 The specific details of the solution 4b identified by RAN as documented in TR 38.843 [6] as one of the potential solutions for model delivery to UE is yet to be investigated by SA5.

#### 5.3.2.2	Use cases

##### 5.3.2.2.1	Relation of ML model delivery in RAN to ML model loading in SA5

When Control Plane signalling is used for RAN→UE. AI/ML model delivery in solution 4b corresponds to the "OAM→RAN→UE scenario" in TR 38.843 [6]. As the interaction over air interface between gNB and UE is outside the scope of SA5, what is referred to as "delivery" in RAN corresponds to ML Model Loading in SA5 for which the existing solutions for ML model loading in TS 28.105 [2] could be reused for the model transfer process between OAM and the gNB.

#### 5.3.2.3	Potential Requirements

The requirements documented in TS 28.105 [2], clause 6.4.1.3 are applicable for AI/ML model delivery to gNB. Further clarifications into clause 6.4.1.3 should be further discussed and decided during the normative phase.

#### 5.3.2.4	Possible solutions

Clarification on the relation between ML Model Delivery and ML Model Loading is needed. Add the following sentences to elaborate the relation of between ML Model Delivery in RAN and ML Model Loading in clause 6.4.1 of TS 28.105 [2].

"ML Model Delivery in RAN refers to the delivery of an AI/ML model from one entity to another entity. Interaction over air interface in not the scope of SA5 and what is called ML model delivery in RAN corresponds to ML Model Loading in SA5."

#### 5.3.2.5	Evaluation

The solution described in clause 5.3.2.4 proposes to clarify the relation between ML Model Delivery and ML Model Loading. However, further enhancements to ML model loading capabilities defined by SA5 may still be needed in the option of OAM transferring/delivering AI/ML models to UE via OAM→RAN→UE, where Control Plane (CP) signalling is used for RAN→UE and should be investigated in the normative work. Furthermore, the option of OAM transferring/delivering AI/ML models to UE via OAM →UE, e.g. via IP tunnel is FFS and needs to be investigated in SA5 once the requirements are clear from RAN working groups.

## 5.4	AI/ML inference

### 5.4.1	Coordination between the ML capabilities

#### 5.4.1.1	Description

For ML in 5GC or RAN, the ML capabilities in 5GC or RAN may be needed to coordinate with 3GPP management analytics and possibly other aspects in order to improve the overall performance.

Typically, due to the type of the collected data used for model training/inference for 5GC or RAN, the performance of a model in 5GC or RAN may be biased in some respects. On the other hand, the 3GPP management system collects data of longer range and from a wide scope of RAN nodes/5GC, which consequently implies that the predictions calculated by the management system will be unbiased towards the overall RAN nodes/5GC. However, 3GPP management system predictions may lack insight of patterns related to specific node behaviour or finer granularity of time. Hence with coordination or alignment of the ML capability among 5GC/RAN and 3GPP management system, the overall performance may be improved.

To enable the coordination between the ML capabilities, the configuration (e.g. a triggering condition, i.e. when a result is needed from the RAN analytics to MDA) may be needed. On the other hand, the result of the coordination may be communicated towards the consumer(s) and hence there is a need to enhance the reporting in order to capture the deviation of predictions (and/or the related context) so the consumer can gain a better understanding regarding the coordination of ML capabilities.

#### 5.4.1.2	Use cases

##### 5.4.1.2.1	Alignment of the ML capability between 5GC/RAN and 3GPP management system

Generally, the typical data from 5GS data is measurements (PM, KPI), which may be modelled as "time series data" and the analytics of "time series data" is normally to learn the seasonality, trend, etc., patterns. Different types of seasonality patterns exist, e.g. daily, weekly, monthly, seasonally, annually, etc. A RAN node collects finer granularity data for short duration. The finer seasonality pattern will be well captured in a timely manner, while the 3GPP management system with longer range of data (which is more aggregated) will more accurately capture the higher level of seasonality patterns. Hence, combing the analytics results from RAN, 5G core and 3GPP management system or between RAN and 5G core may improve the accuracy for overall predictions.

On another matter, the data collected from one RAN or 5G core node would tend to be biased for that specific RAN node or NF, which implies that the prediction with the data learned and inferred will tend to be biased for that specific node. On the other hand, the 3GPP management system collects data of longer range and from a large amount of different RAN nodes or NFs, which consequently implies that the prediction will be unbiased towards the overall RAN nodes or 5G core area. Hence combining the results from both the RAN or 5G core and 3GPP management system, or between NWDAF and RAN may also improve the overall predictions accuracy (and mitigate the bias).

To be noted, when coordinating with management ML capability, more than one ML capability may be involved. From a single ML capability perspective, one ML model may support one (or a set of) specific type of inference(s) capability. One ML inference function may employ two or more ML models to perform more sophisticated inference(s) instead of developing a new specific ML model. The involved ML models may work together in a coordinated manner, for which a coordination of ML capabilities would be needed.

#### 5.4.1.3	Potential requirements

REQ-AIML_COORD-01: 3GPP management system should have the capability to allow an authorized consumer to request a coordination of different ML capabilities, e.g. between MDAS and RAN function, or MDAF and NWDAF, or among MDAF(s) or among ML Model(s).

REQ-AIML_COORD-02: 3GPP management system should have the capability to report the result of coordination of different ML capabilities.

#### 5.4.1.4	Possible solutions

##### 5.4.1.4.1	Possible solution #1

1) Introduce the information Elements (e.g. instance of IOC or a dataType) for interaction between ML MnS producer and consumer (e.g. RAN analytics or NWDAF, or entity consuming the RAN analytics or NWDAF) to support coordination of the ML capability between 5GC/RAN and 3GPP management system: MLCapabilityCoordinationRequest and a response informant element MLCapabilityCoordinationResponse. This information element may represent the triggering configuration (or a triggering policy) for predictions coordination from two ML capabilities. This information Element may allow an MOI (or MOI using this information element) to be created on the ML (inference) MnS Producer and may contain the following attributes:

-	The analytics deviation indicator, such as a threshold (determining that the prediction calculated by a data analytics function exceeds a configurable certain value, corresponding to the prediction available at a different data analytics function, by a "threshold").

-	The requested analytics (analytics type name, list of analytics values or output, time intervals, confidence degree, etc.).

-	The target objects, e.g. gNBs, and the related characteristics.

-	Area of interest, geographical area or TA.

2) MLCapabilityCoordinationResponse- this information element may represent the response indicating the analytics or data obtained according to the MLCapabilityCoordinationRequest. This information Element may be created by the ML MnS (inference) producer towards the MnS consumer and includes output analytics which can be statistics or predictions. The MLCapabilityCoordinationResponse and MLCapabilityCoordinationRequest may also be data attribute in analytic request and response or analytics report.

Figure 5.4.1.4.1-1: Interaction between ML MnS producer and consumer
to support coordination of the ML capability

#### 5.4.1.5	Evaluation

The solution described in clause 5.4.1.4.1 proposes simple information elements and procedure that may enable MnS Producer to trigger configuration to be used for analytic coordination. This NRM based solution reuses the existing provisioning MnS Operations and notifications for control and reporting. Therefore, the solution described in clause 5.4.1.4.1 is a feasible solution to be developed further in the normative specifications.

### 5.4.2 	Sustainable AI/ML for AI/ML inference

#### 5.4.2.1 	Description

AI/ML techniques have been integrated into mobile networks enabling the learning of network states and provide informed recommendations on the most effective actions based on these learnings. This approach significantly enhances the efficiency in managing a wide array of network use cases. These include, but are not limited to, mobility management, load balancing, and resource optimization. The expectation is that the application of AI/ML techniques will lead to an improvement in overall network performance due to the ability of these techniques to provide more accurate predictions and more effective decision-making processes. However, it is important to note that while the use of AI/ML techniques can lead to significant improvements in network performance, these benefits may come at the cost of increased energy consumption. AI/ML-based solutions, due to their complex computational requirements, may consume additional energy compared to traditional, non-AI/ML solutions.

Sustainable AI/ML aims to reduce energy usage in the AI/ML inference step. It is important to investigate effective methods for evaluating AI/ML energy consumption and efficiency for AI/ML inference which will aid in decreasing energy consumption, enhancing energy efficiency, and promoting sustainable development of AI/ML in 5G systems.

#### 5.4.2.2	Use cases

##### 5.4.2.2.1	AI/ML energy consumption evaluation and reporting for AI/ML inference

AI/ML energy consumption is a significant concern, especially as ML models become more complex and data intensive. The ML model once trained is deployed for inference which consumes a considerable amount of energy. Each inference made by the ML model requires computational resources, and these resources in turn require energy to operate. With potentially thousands of inferences being made, the total energy consumption during inference can be substantial. Alternatively, the MnS consumer may ask the MnS Producer to report the number of floating-point operations for generating AI/ML inferences.

The MnS consumer may have concern on the energy consumption for AI/ML inference and the MnS Producer should report the related information. MnS Consumer may ask the MnS Producer to report the energy consumption for inference. To satisfy such query request from the MnS consumer, methods should be investigated to measure AIML energy consumption during the inference. Then, MnS Consumer can use the energy consumption information and compare with the network performance gain due to ML model to evaluate the total benefit of using the ML model.

#### 5.4.2.3	Potential requirements

REQ-AIML_INF_ENERGY_MGT-01: The AI/ML inference MnS producer should have a capability to allow an authorized MnS consumer to query floating-point operations pertaining to generating inferences.

REQ-AIML_INF_ENERGY_MGT-02: The AI/ML inference MnS producer should have a capability to report the floating-point operations pertaining to generating inferences to an authorized consumer.

#### 5.4.2.4	Possible solutions

##### 5.4.2.4.1	Possible solution #1

The inferenceOutputs attribute (of type inferenceOutput) defined in the AIMLInferenceReport IoC needs to be enhanced with a new attribute related to energy consumption for AI/ML inference, e.g. inferenceEnergyConsumption. This attribute is of type real and indicates the energy consumption value pertaining to generating AI/ML inference, e.g. kilowatt-hour "kWh".

NOTE:	 The method to determine energy consumption for AI/ML inference is for further discussion. If energy consumption for AI/ML inference cannot be determined, the proposed possible solution #1 in 5.4.2.4.1 becomes redundant.

##### 5.4.2.4.2	Possible solution #2

The IoC MLTrainingReport needs to be enhanced with a new attribute, e.g. flopsInference, to indicate the number of floating-point operations needed to generate AI/ML inferences. This attribute is of type integer.

#### 5.4.2.5	Evaluation

Solution 1 in clause 5.4.2.4.1 proposes adding an attribute to the IOC AIMLInferenceReport that indicates the energy consumption for generating an AI/ML inference. There is also a note that the method to determine energy consumption is for further discussion and if this cannot be determined the solution becomes redundant. The method for determining energy consumption has not been determined as part of the present document, therefore no recommendations are made for this solution to be directly used in normative phase.

Solution 2 described in clause 5.4.2.4.2, proposes the addition of a new attribute to the AIMLInferenceReport IoC to enable the MnS producer to report the number of floating-point operations needed to generate AI/ML inference which is a feasible solution to be developed further in the normative specifications.

### 5.4.3	ML remedial action management

#### 5.4.3.1	Description

Trained ML models are deployed in the network where they will perform inference. The MnS Consumer may activate the ML inference by providing the scope (e.g. time window, geographical area) in which the ML inference should be activated or a policy for activation. The assumption is that the activation of ML inference will improve the network performance. However, it is difficult to assess the benefits and to quantify such benefits of applying ML inference in a given context of operational network, before using it.

#### 5.4.3.2	Use cases

##### 5.4.3.2.1	ML remedial actions due to performance degradation and energy consumption

By monitoring the network performance, the AI/ML inference MnS Consumer may identify the degradation of network performance or assess the benefits of employing ML model in the network, e.g. improvements in network performance when activating AI/ML inference. Based on the observations the AI/ML inference MnS Consumer may decide to perform certain remedial actions in order to mitigate undesired effect of the activated AI/ML inference. This may include the request to deactivate the AI/ML inference completely, change/reduce the scope of the AI/ML inference activation (e.g. limiting to only certain cells or geographical areas) or request for retraining the ML model. Additionally, such remedial actions may also be triggered by the AI/ML inference MnS Consumer due to the increased energy consumption for AI/ML inference without achieving significant performance gains.

#### 5.4.3.3	Potential requirements

REQ-FB-1: The 3GPP management system shall have a capability to enable an authorized MnS consumer to request change and reduction of the scope of AI/ML inference activation as a remedial action.

#### 5.4.3.4	Possible solutions

The description for the attributes AIMLManagementPolicy.managedActivationScope, AIMLInferenceFunction.managedActivationScope, ManagedActivationScope.dNList, ManagedActivationScope.timeWindow and ManagedActivationScope.geoPolygon defined in [2] has to be elaborated to enable the MnS Consumer to reduce the scope of AI/ML inference activation as a remedial action in order to counteract observed undesirable effects of AI/ML inference. The scope of AI/ML de-activation may include the following:

-	Geographical area

-	Time window

-	List of DNs of managed elements

The scope of AI/ML de-activation may be given as a part of de-activation policies defined by the MnS Consumer instructing the MnS Producer on how to automatically take remedial actions by changing or reducing the scope of AI/ML inference activation. For complete AI/ML deactivation, the ManagedActivationScope.dNList, ManagedActivationScope.dNList and ManagedActivationScope.geoPolygon should be an empty list.

#### 5.4.3.5	Evaluation

The solution described in clause 5.4.3.4 proposes simple enhancements to information elements and attribute descriptions of AIMLManagementPolicy.managedActivationScope, AIMLInferenceFunction.managedActivationScope, ManagedActivationScope.dNList, ManagedActivationScope.timeWindow and ManagedActivationScope.geoPolygon for AI/ML deactivation as a remedial action. Therefore, the solution described in clause 5.4.3.4 is a feasible solution to be developed further in the normative specifications.

### 5.4.4	Managing ML models in use in a live network

#### 5.4.4.1	Description

Void.

#### 5.4.4.2	Use Cases

##### 5.4.4.2.1	Handling of underperforming ML trained models in live networks

Actions may need to be taken by a network operator once an ML trained model has been identified that is contributing towards non-optimal running of the network. These actions may involve for example, without service interruptions, reverting to running of the network without ML based optimizations or replacing current ML model with an earlier model one that was performing better.

##### 5.4.4.2.2	Performance monitoring of Network Functions with ML trained models in live networks

Several trained ML models maybe in use in an operator network with each one of them influencing network performance. The Network Functions with these ML models need to be monitored to ensure network is running optimally with these models in use. KPIs for evaluating runtime performance of Network Functions using ML models should be provided for this purpose as part of the model and these KPIs would need to be collectable by the network operator.

#### 5.4.4.3	Potential requirements

REQ-DATA-ACT-1: The 3GPP management system should have a capability enabling an authorized consumer to identify an ML Model that caused performance measurement and/or KPI degradation.

REQ-DATA-ACT-2: The 3GPP management system should have a capability to recommend remedial actions to address performance measurement and/or KPI degradation caused by an ML model.

REQ-DATA-PERF-1: The 3GPP management system should be able to collect, from the network, performance data pertaining to Network Functions actively utilizing ML models.

REQ-DATA-PERF-2: The 3GPP management system should support performance KPIs that can be monitored for performance assurance purpose.

#### 5.4.4.4	Possible solutions

The solution requires providing the additional information in the AI/ML Inference Report. This information will specify the potential negative network impacts of the execution of the inference output result. This will include impacted network scope (e.g. identifier of the network function, geographical location, impact time etc.) and the affected performance measurements/KPI. This information can then enable an authorized consumer to a) take an informed decision about executing the inference output result b) identify the ML models that is/are causing a specific performance degradation in the network at some future point of time. The consumer can then decide to either deactivate the inference or update the inference function properties to mitigate (i.e., stop and then take actions to resolve) the performance degradation.

NOTE 1: 	The proposed solution is specific for the cases where the value of the attribute aIMLInferenceName indicates the values of the MDA type (see 3GPP TS 28.104 [4]).

The required information will be added as part of InferenceReport as defined in 3GPP TS 28.105 [2].

-	AffectedScope: This will specify the scope of affect the inference output may have. This may include:

-	Identifier of the network functions that may be affected by the output result of the inference function. This will be in form of a DN.

-	A Geographical location indicating that all the network function in that location may be affected by the inference output result.

-	A time duration at which the network function(s) may be affected.

-	Existing attribute ManagedActivationScope may be used for this purpose.

-	Affected Performance Data: This will identify the potential performance data that may be affected in a non-optimal way due to the recommendations/configurations provided as part of inference output result.

-	PMMetrics: This will identify the performance data or the KPI that may be affected. This will be the name of PM and KPI as defined in 3GPP TS 28.552 and 28.554 respectively.

-	ExpectedPMValues: This will specify the potential non-optimal value of the performance data.

NOTE 2: 	How these predictions will be ascertained need to be considered in the normative phase.

NOTE 3:	 The solution where the value of the attribute aIMLInferenceName does not indicate the values of the MDA type is for further discussion.

NOTE 4:	 The need to have use case specific effected performance data is to be done as part of normative work..

Considering that multiple ML models may be concurrently deployed for generating inferences in the network, more than one ML model may simultaneously impact the same PM(s)KPI(s). Thus, it may be challenging for an authorized consumer to hold an ML model accountable for PM/KPI improvement (or drop). In this case, the solution assumes that multiple ML model will be held responsible for PM/KPI improvement (or drop).

#### 5.4.4.5	Evaluation

The solution described in clause 5.4.4.4 proposes information objects that can enable identification of the ML model(s) that potentially be causing the performance degradation. Enabling consumer to either deactivate the inference or update the inference function properties to mitigate the performance degradation.

The solution suggests to deactivate all potentially responsible models. However, it may be possible that not all reported models are responsible for the performance degradation. Determining the actual impact of ML models on PM(s)/KPI(s) needs to be further investigated during the normative phase.

### 5.4.5	AI/ML prediction latency

#### 5.4.5.1	Description

AI/ML prediction latency refers to the time it takes for an ML model to process an input and produce an output. This is a critical metric for real-time AI/ML enabled use cases where quick responses are essential. It is important to have the awareness of AI/ML prediction latency as well as to allow consumer to indicate the requirements regarding the AI/ML prediction latency.

#### 5.4.5.2	Use cases

##### 5.4.5.2.1	AI/ML prediction latency during inference

AI/ML prediction latency is a critical factor in use cases with stringent time constraints. During inference, the latency of AI/ML prediction is influenced by several factors, including the hardware platform executing the inference, techniques such as ML model quantization and pruning, and the overall performance of the ML model. To ensure that the AI/ML prediction latency meets the requirements specified by an authorized MnS consumer, it is essential to monitor the achieved AI/ML prediction latency by recording the triggering and reception time of inference once the ML model is loaded and activated in the AI/ML inference function and report the metric to the AI/ML inference MnS producer.

#### 5.4.5.3	Potential requirements

REQ-AI/ML_INF_Latency-1: The 3GPP management system should have a capability to report to an authorized MnS consumer the achieved AI/ML prediction latency.

#### 5.4.5.4	Possible solutions

The inferenceOutputs attribute (of type inferenceOutput) defined in the AIMLInferenceReport IoC needs to be enhanced with new attributes related to AI/ML prediction latency, e.g. achievedPredictionLatency. This attribute is an indication on the average duration it took for ML model predictions on the inference data.

#### 5.4.5.5	Evaluation

The solution described in clause 5.4.5.4 proposes the addition of new attributes to the AIMLInferenceReport IoC to enable the MnS producer to report the achieved AI/ML prediction latency to the AI/ML inference MnS producer. Therefore, the solution described in clause 5.4.5.4 is a feasible solution to be developed further in the normative specifications.

### 5.4.6	ML explainability

#### 5.4.6.1		Description

Explainable ML refers to a process that enables the consumers (e.g. human operator) to understand and trust the outputs provided by ML models. In essence, explainable ML is about making the decision-making of ML comprehensible to its consumers. On a broad level, explainable ML can be categorized into two types:

-	Local explanation: The aim is to explain individual outputs provided by an ML model, i.e., it focuses on explaining why a specific output was generated by the ML model for a particular input data sample.

-	Global explanation: The aim is to explain the whole ML model behaviour, i.e., it focuses on explaining how the ML model works in general across several or all possible input samples.

#### 5.4.6.2	Use cases

##### 5.4.6.2.1	Local explanation in AI/ML inference

Local explanation is an additional information that the MnS consumer can use to analyse the ML model's potential impact in terms of network performance gain and make informed decisions regarding deactivation of AI/ML inference or fallback to previous version of the ML model. The ML Training Request IOC initiated by the MnS consumer does not specify requirements for local explainability support in ML model training. It is essential for the MnS consumer to indicate this need so that the MnS producer can preprocess the training data or employ suitable techniques to generate local explanations.

Once the ML model is trained to generate both outputs and corresponding explanations, it is deployed for inference. The generated local explanations by the AI/ML inference MnS producer may need to be reported to the AI/ML inference MnS consumer. These local explanations provide additional information that the MnS consumer can use to analyse the ML model's potential impact on network performance. This analysis helps in making informed decisions regarding the deactivation of AI/ML inference or fallback to a previous version of the ML model. By considering network performance PM/KPIs, model performance, and local explanations, the MnS consumer can effectively identify the ML model contributing to network performance degradation when several ML models are active in the network.

#### 5.4.6.3	Potential requirements

REQ-ML-EXP-1: The 3GPP management system should provide the capability for an authorized consumer to indicate the support for local explanations in ML model training.

REQ-ML-EXP-2: The 3GPP management system should provide the capability for an authorized consumer to receive from the AI/ML inference MnS producer the generated local explanations for inference.

#### 5.4.6.4	Possible solutions

Introduce a new attribute, e.g. localExplanationIndicator, in the MLTrainingRequest IoC requested by an authorized MnS Consumer to the ML Training MnS Producer. When the attribute is set to True, ML Training MnS Producer should train the ML model such that it can provide local explanations for the aIMLInferenceName. The attribute is of type Boolean.

Introduce a new attribute inferenceLocalExplanation in the inferenceOutputs attribute (which is of type inferenceOutput) in the AIMLInferenceReport IoC. This information indicates the local explanation for the generated inference corresponding to a particular inferenceOutputId. The attribute is of type string.

#### 5.4.6.5	Evaluation

The solution described in clause 5.4.6.4 proposes the addition of a new attribute to the MLTrainingRequest IoC to enable the MnS consumer to indicate the need for the trained ML model to provide local explanations. As a precondition, the authorized consumer needs to discover if the MLTrainingFunction supports the capability to train an ML model to generate local explanations before indicating the need for the trained ML model to provide local explanations.

Furthermore, the solution proposes the addition of a new attribute to the AIMLInferenceReport IoC to enable the MnS producer to provide local explanations corresponding to the generated inferences. Further consideration, e.g. identifying the suitable local explanations for each ML model, achieved interoperability needs further investigation in the normative phase. Therefore, the solution described in clause 5.4.6.4 is a feasible solution to be developed further in the normative specifications.

# 6	Conclusions and recommendations

The present technical report describes the enhanced AI/ML management capabilities and services, based on TS 28.105, for the 3GPP 5GS, including the management and orchestration systems, 5GC, and NG-RAN, where AI/ML features or capabilities are employed. The enhanced management capabilities and services are developed based on a wide range of use cases, along with corresponding potential requirements and possible solutions, as described in Clause 5.

Clause 4 introduces key concepts and relevant terminologies foundational to understanding AI/ML management and orchestration capabilities in the 5GS. It defines terms that extend existing Rel-18 definitions as well as highlighting core principles and concepts that support AI/ML lifecycle management capabilities. Clause 5 then elaborates on the specific management capabilities needed for the AI/ML lifecycle, covering various aspects of ML model training, AI/ML inference emulation, ML model deployment, and AI/ML inference.

Moving into the normative specification development phase, it is recommended to specify AI/ML management capabilities aligned with the relevant use cases, requirements, and solutions for AI/ML lifecycle management steps. These capabilities should address specific functionalities defined by 3GPP, including:

-	NG-RAN AIML-based Coverage and Capacity Optimization, and NG-RAN AIML-based Network Slicing defined by RAN3,

-	Model delivery/transfer as defined by RAN1/2,

-	ML model training and AI/ML inference functions for 5GC as defined by SA2, and

-	MDA (Management Data Analytics) as defined by SA5.

Furthermore, detailed capabilities should support:

ML model training

-	ML-Knowledge-based Transfer Learning

-	ML pre-training

-	ML Fine-tuning

-	ML model training for multiple contexts

-	ML training data statistics

-	ML model confidence

-	Management of Reinforcement Learning

-	Sustainable AI/ML for ML training

-	ML model Distributed training

-	Management of Federated Learning

-	ML authentication

-	AI/ML prediction latency

-	ML explainability

AI/ML inference emulation

-	ML inference emulation

-	ML inference emulation environment selection

AI/ML deployment

-	Enhance the ML model loading use case

-	Managing ML model transfer/delivery

AI/ML inference

-	Coordination between and orchestration of the ML capabilities

-	Sustainable AI/ML for AIML inference

-	ML remedial action management

-	Managing ML models in use in a live network

-	AI/ML prediction latency

-	ML explainability

Annex <A>:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | SA5#156 | S5-243521 |  |  |  | Initial skeleton | 0.0.0 |
| 2024-08 | SA5#154 | S5-242150 |  |  |  | Rel-19 pCR TR28.9xx R19 Study usecases for continuation from 28.908 | 0.1.0 |
| 2024-08 | SA5#155 | S5-243096 |  |  |  | Rel-19 Add use case of ML pre-training | 0.1.0 |
| 2024-08 | SA5#155 | S5-243118 |  |  |  | Rel-19 pCR TR28.9xx Clusters for Joint Training of ML models | 0.1.0 |
| 2024-08 | SA5#155 | S5-243115 |  |  |  | Rel-19 pCR TR28.9xx Training Data Statistical Properties | 0.1.0 |
| 2024-08 | SA5#155 | S5-243116 |  |  |  | Rel-19 pCR TR 28.9xx Confidence Threshold | 0.1.0 |
| 2024-08 | SA5#155 | S5-243314 |  |  |  | Rel-19 pCR TR28.9xx Management of Reinforcement Learning | 0.1.0 |
| 2024-08 | SA5#155 | S5-243434 |  |  |  | Rel-19 pCR TR 28.9xx AI/ML energy consumption and energy efficiency | 0.1.0 |
| 2024-08 | SA5#156 | S5-244592 |  |  |  | pCR TR 28.858 add scope | 0.2.0 |
| 2024-08 | SA5#156 | S5-244593 |  |  |  | pCR TR 28.858 add overview and list of references | 0.2.0 |
| 2024-08 | SA5#156 | S5-244596 |  |  |  | Rel-19 pCR TR28.858 Add solution for ML model training for multiple contexts | 0.2.0 |
| 2024-08 | SA5#156 | S5-244605 |  |  |  | Rel-19 pCR TR 28.858 Add use case, requirements and solution for AIML inference energy consumption | 0.2.0 |
| 2024-08 | SA5#156 | S5-244604 |  |  |  | Rel-19 pCR TR 28.858 Add requirements and solution for ML model training energy consumption | 0.2.0 |
| 2024-08 | SA5#156 | S5-244594 |  |  |  | Rel-19 pCR TR28.858 Add use case and requirements for ML fallback management | 0.2.0 |
| 2024-08 | SA5#156 | S5-244595 |  |  |  | Rel-19 pCR TR 28.858 Add evaluation for training data statistical properties | 0.2.0 |
| 2024-08 | SA5#156 | S5-244619 |  |  |  | Rel-19 pCR TR 28.858 Add use case on ML entity distributed training | 0.2.0 |
| 2024-08 | SA5#156 | S5-244826 |  |  |  | Rel-19 pCR TR 28.858 Add solution for ML model confidence threshold | 0.2.0 |
| 2024-08 | SA5#156 | S5-244615 |  |  |  | Rel-19 pCR TR 28.858 Add Potential Solution for Fine-tuning | 0.2.0 |
| 2024-08 | SA5#156 | S5-244609 |  |  |  | Rel-19 pCR TR 28.858 Add use case for ML Fine-tuning | 0.2.0 |
| 2024-08 | SA5#156 | S5-244616 |  |  |  | Rel-19 pCR TR 28.858 Add Potential Solution for Pre-training | 0.2.0 |
| 2024-08 | SA5#156 | S5-244634 |  |  |  | Rel-19 pCR TR 28.858 add use cases, requirements for reinforcement learning management | 0.2.0 |
| 2024-08 | SA5#156 | S5-244599 |  |  |  | Rel-19 pCR TR 28.858 Enhance the ML model loading use case and solution | 0.2.0 |
| 2024-08 | SA5#156 | S5-244637 |  |  |  | pCR TR 28.858 Update the UC and requirements for pre-training | 0.2.0 |
| 2024-08 | SA5#156 | S5-244817 |  |  |  | pCR TR 28.858 Add use case for management of Federated Learning | 0.2.0 |
| 2024-08 | SA5#156 | S5-244607 |  |  |  | pCR TR 28.858 Add Use Case and Requirements for ML Model Transfer | 0.2.0 |
| 2024-08 | SA5#156 | S5-244606 |  |  |  | pCR TR 28.858 Sustainability of AI-ML | 0.2.0 |
| 2024-08 | SA5#156 | S5-244601 |  |  |  | Rel-19 pCR TR28.858 Add use case of AI emulation | 0.2.0 |
| 2024-08 | SA5#156 | S5-244618 |  |  |  | Rel-19 pCR Network Management actions in response to an underperforming ML algorithm used in a live network | 0.2.0 |
| 2024-08 | SA5#156 | S5-244617 |  |  |  | Collection of runtime performance KPIs for ML models | 0.2.0 |
| 2024-08 | SA5#156 | S5-244600 |  |  |  | Rel-19 pCR TR28.858 Conclude R19 continuation use cases | 0.2.0 |
| 2024-08 | SA5#156 | S5-244598 |  |  |  | Rel-19 pCR TR28.858 Update R19 continuation use cases | 0.2.0 |
| 2024-10 | SA5#157 | S5-245857 |  |  |  | pCR TR 28.858 clarifications and corrections into AI/ML inference emulation | 0.3.0 |
| 2024-10 | SA5#157 | S5-245859 |  |  |  | pCR TR 28.858 add reference to TS 28.105 for Rel-19 terms and definitions | 0.3.0 |
| 2024-10 | SA5#157 | S5-245990 |  |  |  | Rel-19 pCR TR 28.858 add solution for AI emulation | 0.3.0 |
| 2024-10 | SA5#157 | S5-245991 |  |  |  | pCR 28.858 Updates to Coordination between the ML capabilities | 0.3.0 |
| 2024-10 | SA5#157 | S5-245993 |  |  |  | Rel-19 pCR TR 28.858 Update the use case of ML training data statistics | 0.3.0 |
| 2024-10 | SA5#157 | S5-245994 |  |  |  | Rel-19 pCR TR 28.858 improve ML-knowledge-based transfer learning use case | 0.3.0 |
| 2024-10 | SA5#157 | S5-245995 |  |  |  | pCR TR 28.858 clarifications and corrections into ML-Knowledge-based Transfer Learning | 0.3.0 |
| 2024-10 | SA5#157 | S5-245996 |  |  |  | Rel-19 pCR 28.858 Solution for managing ML Model in live network | 0.3.0 |
| 2024-10 | SA5#157 | S5-245998 |  |  |  | pCR TR 28.858 clarifications into ML model transfer use case | 0.3.0 |
| 2024-10 | SA5#157 | S5-246002 |  |  |  | Rel-19 pCR TR 28.858 Update use case on ML entity distributed training | 0.3.0 |
| 2024-10 | SA5#157 | S5-246003 |  |  |  | Rel-19 pCR TR 28.858 Add possible solutions on ML entity distributed training management | 0.3.0 |
| 2024-10 | SA5#157 | S5-246004 |  |  |  | Rel-19 pCR TR 28.858 Update Description and Add New Requirements to Management of Federated Learning Use Case | 0.3.0 |
| 2024-10 | SA5#157 | S5-246248 |  |  |  | pCR TR 28.858 Add possible solution for management of Federated Learning | 0.3.0 |
| 2024-10 | SA5#157 | S5-246251 |  |  |  | Rel-19 pCR TR 28.858 Add Evaluation and Conclusion for Pre-training | 0.3.0 |
| 2024-10 | SA5#157 | S5-246253 |  |  |  | pCR 28.858 Correct use of specification | 0.3.0 |
| 2024-10 | SA5#157 | S5-246254 |  |  |  | Rel-19 pCR TR 28.858 Update the Description of Management of Reinforcement Learning Use Case | 0.3.0 |
| 2024-10 | SA5#157 | S5-246255 |  |  |  | Rel-19 pCR TR 28.858 Add New Requirements and Possible Solution to the Reinforcement Learning Use Case | 0.3.0 |
| 2024-10 | SA5#157 | S5-246256 |  |  |  | Rel-19 pCR TR 28.858 add possible solutions for conflict management of reinforcement learning | 0.3.0 |
| 2024-10 | SA5#157 | S5-246257 |  |  |  | Rel-19 pCR TR 28.858 add use cases, requirements for conflict management of reinforcement learning | 0.3.0 |
| 2024-10 | SA5#157 | S5-246258 |  |  |  | PCR TR 28.858 Adding ML Authentication UseCase | 0.3.0 |
| 2024-10 | SA5#157 | S5-246259 |  |  |  | Rel-19 pCR TR 28.858 Update the use case of ML model training for multiple contexts | 0.3.0 |
| 2024-10 | SA5#157 | S5-246260 |  |  |  | Rel-19 pCR TR 28.858 Add evaluation for ML model training for multiple contexts | 0.3.0 |
| 2024-10 | SA5#157 | S5-246261 |  |  |  | Rel-19 pCR TR 28.858 Add evaluation for ML model confidence | 0.3.0 |
| 2024-10 | SA5#157 | S5-246262 |  |  |  | Rel-19 pCR TR 28.858 Add solution for ML remedial action management | 0.3.0 |
| 2024-10 | SA5#157 | S5-246263 |  |  |  | Rel-19 pCR TR 28.858 AIML prediction latency | 0.3.0 |
| 2024-10 | SA5#157 | S5-246264 |  |  |  | pCR TR 28.858 add terms and abbreviations | 0.3.0 |
| 2024-10 | SA5#157 | S5-246266 |  |  |  | Rel-19 pCR TR 28.858 ML explainability | 0.3.0 |
| 2024-10 | SA5#157 | S5-246284 |  |  |  | Rel-19 pCR TR 28.858 Add solution for ML inference aspects of sustainable AIML | 0.3.0 |
| 2024-10 | SA5#157 | S5-246285 |  |  |  | Rel-19 pCR TR 28.858 Add solution for ML training aspects of sustainable AIML | 0.3.0 |
| 2024-11 | SA5#158 | S5-247061 |  |  |  | pCR TR 28.858 add conclusion and recommendations | 0.4.0 |
| 2024-11 | SA5#158 | S5-247062 |  |  |  | pCR TR 28.858 rapporteur corrections and cleanup | 0.4.0 |
| 2024-11 | SA5#158 | S5-247063 |  |  |  | pCR TR 28.858 add clarifications and corrections to the Scope | 0.4.0 |
| 2024-11 | SA5#158 | S5-247064 |  |  |  | pCR TR 28.858 add clarifications and corrections to the Terms | 0.4.0 |
| 2024-11 | SA5#158 | S5-247065 |  |  |  | PCR TR 28.858 Correct Fine-Tunning term definition | 0.4.0 |
| 2024-11 | SA5#158 | S5-247066 |  |  |  | Rel 19 pCR TR 28.858 ML-Knowledge-based Transfer Learning Evaluation | 0.4.0 |
| 2024-11 | SA5#158 | S5-247067 |  |  |  | Rel-19 pCR 28.858 Solution enhancement and evaluation for ML model in live network | 0.4.0 |
| 2024-11 | SA5#158 | S5-247068 |  |  |  | Rel-19 pCR TR 28.858 Clarifications to the description and use case for ML model transfer or delivery | 0.4.0 |
| 2024-11 | SA5#158 | S5-247069 |  |  |  | Rel-19 pCR TR 28.858 Add solution and evaluation for AIML prediction latency | 0.4.0 |
| 2024-11 | SA5#158 | S5-247331 |  |  |  | PCR TR 28.858 Correct Model Confidence Evaluation | 0.4.0 |
| 2024-11 | SA5#158 | S5-246643 |  |  |  | Rel-19 pCR TR 28.858 Add evaluation for ML model distributed training | 0.4.0 |
| 2024-11 | SA5#158 | S5-247086 |  |  |  | R19 pCR TR 28.858 Add Evaluation on Management of Federated Learning | 0.4.0 |
| 2024-11 | SA5#158 | S5-247087 |  |  |  | pCR TR 28.858 add clarifications on ML pre-training use case | 0.4.0 |
| 2024-11 | SA5#158 | S5-247329 |  |  |  | PCR TR 28.858 Correct model training for multiple contexts | 0.4.0 |
| 2024-11 | SA5#158 | S5-247089 |  |  |  | Rel-19 pCR TR 28.858 Enhance solution and evaluation for management of RL | 0.4.0 |
| 2024-11 | SA5#158 | S5-247091 |  |  |  | Rel-19 pCR TR 28.858 add evaluation and conclusions for conflict management of reinforcement learning | 0.4.0 |
| 2024-11 | SA5#158 | S5-247330 |  |  |  | R19 pCR TR 28.858 Update the Reinforcement Learning Use Case | 0.4.0 |
| 2024-11 | SA5#158 | S5-247315 |  |  |  | Rel-19 pCR TR 28.858 Add evaluation for remedial action management | 0.4.0 |
| 2024-11 | SA5#158 | S5-247094 |  |  |  | Rel-19 pCR TR 28.858 Add solution and evaluation for ML explainability in inference | 0.4.0 |
| 2024-11 | SA5#158 | S5-247098 |  |  |  | Rel-19 pCR TR 28.858 enhance the overview to support the NG-RAN and 5GC | 0.4.0 |
| 2024-11 | SA5#158 | S5-247092 |  |  |  | Rel-19 pCR TR 28.858 Add evaluation for sustainable AIML | 0.4.0 |
| 2024-12 |  |  |  |  |  | EditHelp's cleanup | 0.4.1 |
| 2024-12 | SA#106 | SP-241604 |  |  |  | Presentation to SA for Information and Approval | 1.0.0 |