| 3GPP TR 29.866 V1.0.0 (2024-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Study on IMS Disaster Prevention and Restoration Enhancement (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Baseline	9

5	Key Issues	9

5.1	Key Issue #1: Continue service for registered users in case of HSS/UDM failure or overload	9

5.1.1	Description of the use case	9

5.1.2	Key issue definition	9

5.2	Key Issue #2: Handle all PCRF/PCF instances failure scenario with minimum impact to the registered IMS users	10

5.2.1	Description of the use case	10

5.2.2	Key issue definition	10

5.3	Key Issue #3: Speed up the restoration procedure of the terminating session when EPC/5GC NF fails	10

5.3.1	Description of the use case	10

5.3.2	Key issue definition	10

5.4	Key Issue #4: Preventing the IMS disaster in the HSS overload scenario	11

5.4.1	Description	11

5.4.2	Key issue definition	11

6	Solutions	11

6.0	Mapping of Solutions to Key Issues	11

6.1	Solution#1: Solution for optimizing Re-registration process in case of HSS/UDM failure or overload	11

6.1.1	Description	11

6.1.1.1	General	11

6.1.1.2	(Re)-registration procedure for routing without I-CSCF	12

6.1.1.3	(Re)-registration procedure for routing with I-CSCF	13

6.1.2	Impacts on services, entities and interfaces	13

6.1.2.1	Routing without I-CSCF	13

6.1.2.2	Routing with I-CSCF	14

6.1.3	Pros	14

6.1.4	Cons	14

6.2	Solution#2: Preventing the IMS disaster in the HSS overload scenario	14

6.2.1	Description	14

6.2.1.1	General	14

6.2.1.2	IMS initial registration of reducing HSS load	14

6.2.1.3	IMS third-party registration of reducing HSS load	16

6.2.2	Impacts on services, entities and interfaces	16

6.2.3	Pros	17

6.2.4	Cons	17

6.3	Solution#3: Solution for maintaining the IMS registration and call services uninterruptible in case of HSS/UDM failure	17

6.3.1	Description	17

6.3.1.1	General	17

6.3.1.2	Storage of S-CSCF Address	17

6.3.1.3	IMS initial registration procedure in case of HSS failure	18

6.3.1.4	IMS re-registration procedure in case of HSS failure	20

6.3.1.5	IMS MT procedure in case of HSS failure	20

6.3.2	Impacts on services, entities and interfaces	20

6.3.3	Pros	21

6.3.4	Cons	21

6.4	Solution #4: Solution for HSS/UDM bypass	21

6.4.1	Description	21

6.4.1.1	General	21

6.4.1.2	IMS deregistration procedure of HSS/UDM bypass	22

6.4.1.3	IMS initial registration procedure of HSS/UDM bypass	22

6.4.1.4	IMS re-registration procedure of HSS/UDM bypass	23

6.4.1.5	MT procedure of HSS/UDM bypass	23

6.4.2	Impacts on services, entities and interfaces	24

6.4.3	Pros	24

6.4.4	Cons	24

6.5	Solution #5: Solution for PCRF/PCF bypass	24

6.5.1	Description	24

6.5.1.1	General	24

6.5.1.2	Default IMS bearer establishment procedure of PCRF/PCF bypass	24

6.5.1.3	IMS MO procedure of PCRF/PCF bypass	25

6.5.1.4	IMS MT procedure of PCRF/PCF bypass	26

6.5.1.5	Dedicated IMS bearer termination procedure of PCRF/PCF bypass	26

6.5.2	Impacts on services, entities and interfaces	27

6.5.3	Pros	27

6.5.4	Cons	27

6.6	Solution #6: Solution for speeding up the restoration procedure of the terminating session when EPC/5GC NF fails	27

6.6.1	Description	27

6.6.1.1	General	27

6.6.1.2	Quick restoration procedure of IMS terminating session	28

6.6.2	Impacts on services, entities and interfaces	29

6.6.3	Pros	29

6.6.4	Cons	29

6.7	Solution#7: Enhance the P-CSCF restoration in the case of 5GC/EPC UP failure	29

6.7.1	Description	29

6.7.1.1	Gerenal	29

6.7.1.2	CN user plane failure detected	30

6.7.2	Impacts on services, entities and interfaces	31

6.7.3	Pros	31

6.7.4	Cons	31

6.8	Solution#8: Enhance the P-CSCF restoration in the case of 5GC/EPC failure	31

6.8.1	Description	31

6.8.1.1	Gerenal	31

6.8.1.2	PCRF/PCF detecting PGW-C/SMF failure	31

6.8.2	Impacts on services, entities and interfaces	33

6.8.3	Pros	33

6.8.4	Cons	33

7	Evaluations	33

7.1	Evaluation of Solutions for Key Issue#1	33

7.2	Evaluation of Solutions for Key Issue#2	34

7.3	Evaluation of Solutions for Key Issue#3	34

7.4	Evaluation of Solutions for Key Issue#4	34

8	Conclusions	35

8.1	Conclusion of Solutions for Key Issue#1	35

8.2	Conclusion of Solutions for Key Issue#2	35

8.3	Conclusion of Solutions for Key Issue#3	35

8.4	Conclusion of Solutions for Key Issue#4	35

Annex A (informative): Change history	36

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This Technical Report aims to study potential enhancements on IMS Disaster Prevention and Restoration, to increase the stability of the IMS system.

# 1	Scope

The present document identifies IMS restoration mechanisms as specified in 3GPP TS 23.380 [2] and studies potential optimization solutions. The document will focus on the following aspects:

-	Create/enhance corresponding the restoration mechanism to support the normal use of voice, video and SMS services in the IMS network. Specifically, it adopts the context data of registered online users stored by HSS/UDM, S-CSCF, P-CSCF and MMTEL to bypass the involved faulty network functions or links. That supports the following procedures:

-	Re-registration procedure for registered online users;

-	Mobile terminated procedure for registered online users;

-	EPS fallback procedure for registered online users;

-	Initial registration procedure for registered online users after deregistration.

-	Create/enhance corresponding network disaster-prevention mechanisms, including:

-	UE specific registration analysis mechanism, to prevent registration with missing parameter or incorrect parameters repeatedly.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.380: "IMS Restoration Procedures".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[4]	3GPP TS 23.007: "Restoration procedures".

[5]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx interfaces; Signalling flows and message contents ".

[6]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[7]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[8]	IETF RFC 7683: "Diameter Overload Indication Conveyance".

[9]	3GPP TS 33.203: "Technical Specification Group Services and System Aspects; 3G security; Access security for IP-based services".

[10]	3GPP TS 23.203: "Policy and charging control architecture".

[11]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[12]	IETF RFC 6733: "Diameter Base Protocol".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core

5QI	5G QoS Identifier

AAA	Authorization-Authentication-Answer

AAR	Authorization-Authentication-Request

AMF	Access Management Function

AS	Application Server

AV	Authentication Vector

AVP	Attribute-Value Pair

CSCF	Call Server Control Function

EPC	Evolved Packet Core

HSS	Home Subscriber Server

I-CSCF	Interrogating-CSCF

iFC	initial Filter Criteria

IMS	IP Multimedia Subsystem

IMS-AGW	IMS Access Gateway

MAA	Multimedia-Auth-Answer

MAR	Multimedia-Auth-Request

MME	Mobility Management Entity

MMTEL	Multimedia Telephony

P-CSCF	Proxy-CSCF

P-GW	PDN Gateway

PCC	Policy and Charging Control

PCF	Policy Control Function

PCRF	Policy and Charging Rules Function

PFCP	Packet Forwarding Control Protocol

RAR	Re-Auth-Request

RR	RTCP Receiver Report

RTCP	Real-Time Control Protocol

QCI	QoS Class Identifier

S-CSCF	Serving CSCF

SAA	Server-Assignment-Answer

SAR	Server-Assignment-Request

SMF	Session Management Function

UDA	User Data Answer

UDM	Unified Data Management

UDR	User Data Request

UPF	User Plane Function

# 4	Baseline

3GPP TS 23.380 [2] has specified the procedures required in IMS to handle a S-CSCF or a P-CSCF service interruption scenario with minimum impact to the service to the end user. But IMS Restoration Procedures covering service interruption of other network elements (e.g. UDM, DRA, etc.) are not defined. So it is proposed to create/enhance corresponding restoration mechanism to network elements in the IMS network, to enhance the stability of IMS system.

Besides this, UE registration with missing parameter or incorrect parameters repeatedly may also lead to HSS/UDM overload, and thus make troubles to IMS. So it is proposed to create/enhance UE specific registration analysis for these abnormal behaviours and help to prevent IMS disasters.

# 5	Key Issues

This clause describes the issues that needs to be considered for IMS Disaster Prevention and Restoration Enhancement.

## 5.1	Key Issue #1: Continue service for registered users in case of HSS/UDM failure or overload

### 5.1.1	Description of the use case

3GPP TS 23.380 [2] has already specified a couple of IMS restoration procedures for P-CSCF/S-CSCF failure scenario. However, the HSS/UDM failure or overload scenario has not been considered. In addition, P-CSCF/S-CSCF restoration procedures via HSS increases the HSS load which may further cause HSS overload.

HSS/UDM stores authentication data and subscription data of IMS subscribers. When one HSS/UDM instance fails, other HSS/UDM instances in the same set could provide services to I-CSCF/S-CSCF and IMS AS. In this case, there is no impact on IMS services. But once all the HSS/UDM instances in the set are unavailable, most IMS procedures (e.g. IMS (re-)registration, IMS deregistration, mobile terminating session, AS originating session, supplementary services such as IP SMS, etc.) are impacted which results the service failure. Similar situation happens to HSS/UDM overload scenario, since at that time the HSS/UDM is hard to handle the related procedures.

Even if HSS/UDM failure or overload, operators have strong requirement to continue IMS service for those users already registered to IMS, so that to minimize the service interruption due to HSS/UDM failure or overload. How to enhance existing procedure to support the operator requirement should be considered.

### 5.1.2	Key issue definition

This key issue will study the following aspects:

-	in the case of all HSS/UDM instances failure or overload, how to continue IMS service for registered users as much as possible;

-	which procedures can be enhanced to continue IMS service for registered users, and how to enhance the procedures.

## 5.2	Key Issue #2: Handle all PCRF/PCF instances failure scenario with minimum impact to the registered IMS users

### 5.2.1	Description of the use case

The reliability of PS domain is the prerequisite of the availability of IMS voice, video and SMS service. In 4G network, CSFB solution could be used to provide voice call service for subscribers when EPC network failure causes that VoLTE is unavailable. But CSFB solution could not be used anymore after 2/3G sunset. Therefore, it’s important to study how to minimize the impact to IMS services when some network function in EPC/5GC network fails, especially when all network functions of this kind are faulty.

PCRF/PCF provides QoS guarantee to IMS services, which are defined in 3GPP TS 23.503 [7]. When one PCRF/PCF instance fails, the other PCRF/PCF instances in the set could provide services to P-CSCF and there is no impact on IMS services. But once all the PCRF/PCF instances in the set are unavailable, the dedicated bearers of voice/video media could not be established which causes that all the newly initiated calls fail. This key issue will study the solution addressing this case.

### 5.2.2	Key issue definition

This key issue will study the following aspects:

-	in the case of all PCRF/PCF instances failure or overload, how to continue IMS service for registered users as much as possible;

-	which procedures can be enhanced to continue IMS service for registered users, and how to enhance the procedures.

## 5.3	Key Issue #3: Speed up the restoration procedure of the terminating session when EPC/5GC NF fails

### 5.3.1	Description of the use case

In most of EPC/5GC NF failure scenarios, the UE could invoke re-attach and re-registration procedures quickly if

-	the UE initiates a new IMS session and some SIP error code is sent by the IMS network; or

-	the EPC/5GC network detects the failure and kicks off the related UEs to re-attach.

Then the UE’s service will be restored soon.

But in some cases, the re-attach and re-registration procedures will not be invoked immediately. In that case, the UE’s terminating session will be unavailable until its IMS registration is expired and it invokes the re-registration procedure. If the UE’s registration expiration time is 6000 seconds, the unavailable time is 50 minutes in the worst case. It’s important to find a solution to speed up the restoration procedure of the terminating session when EPC/5GC NF fails.

### 5.3.2	Key issue definition

This key issue will study the following aspects:

-	Specify the mechanism that speeds up the restoration procedure of the terminating session when EPC/5GC NF fails.

NOTE:	The mechanism to speed up the restoration procedure should not impact the UE and introduce overload of EPC/5GC.

## 5.4	Key Issue #4: Preventing the IMS disaster in the HSS overload scenario

### 5.4.1	Description

3GPP TS 23.380 [2] has already specified a couple of IMS restoration procedures for the P-CSCF/S-CSCF failure scenario. However, how to prevent the IMS disaster has not been considered in the HSS overload scenario.

In addition to overload control mechanisms in IETF RFC 7683 [8], the following flow-reduction mechanisms for overload control are defined. If many UEs initiate the initial REGISTER request to the IMS network, a large number of signalling interactions with HSS will accelerate the occurrence of signalling storm. Thus, it is necessary to consider the case in which the signalling interactions between NFs and HSS can be simplified to reduce the HSS load. For example, if the IP address assigned to the UE by the EPC/5GC changes when the UE undergoes a location move, the UE will initiate an initial REGISTER request to the IMS network. At the same time, the used S-CSCF is the same as the last used S-CSCF. Therefore, the stored user data in the S-CSCF can be reused to complete the initial registration procedure, further reducing unnecessary signalling interaction with HSS.

### 5.4.2	Key issue definition

This key issue will study the following aspects:

-	How to reduce the load of HSS to prevent IMS disaster when the HSS overloads. For example, by reducing signal flow via Cx and Sh interfaces.

NOTE:	This solution of the KI is only for the HSS overload scenario, which does not impact the existing procedures for normal cases.

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| 1 | X |  |  |  |
| 2 |  |  |  | X |
| 3 | X |  |  |  |
| 4 | X |  |  |  |
| 5 |  | X |  |  |
| 6 |  |  | X |  |
| 7 |  |  | X |  |
| 8 |  |  | X |  |

## 6.1	Solution#1: Solution for optimizing Re-registration process in case of HSS/UDM failure or overload

### 6.1.1	Description

#### 6.1.1.1	General

The solution is to address the Key Issue #1: Continue service for registered users in case of HSS/UDM failure or overload.

3GPP TS 23.380 [2] has already specified a couple of IMS restoration procedures for P-CSCF/S-CSCF failure scenario. However, the HSS/UDM failure or overload scenario has not been considered. In addition, the existing P-CSCF/S-CSCF restoration procedures via HSS/UDM increases the HSS/UDM load which may further cause HSS/UDM overload.

In the case of HSS/UDM failure, most IMS procedures (e.g. IMS (re-)registration, IMS deregistration, mobile terminating session, AS originating session, supplementary services such as IP SMS, etc.) are impacted which results in the service failure.

This solution considers to continue IMS service for registered UE, in the following procedures:

-	(re)-registration procedure for routing without I-CSCF;

-	(re)-registration procedure for routing with I-CSCF;

Selection of routing without I-CSCF or routing with I-CSCF in the scenario of HSS/UDM failure or overload is based on operation policy.

#### 6.1.1.2	(Re)-registration procedure for routing without I-CSCF

The below Figure 6.1.1.2-1 illustrates the re-registration procedure for routing without I-CSCF when HSS/UDM failure or overload is detected.

Figure 6.1.1.2-1: Re-registration procedure for routing without I-CSCF

1.	Prior to the expiration of a user's registration, the user sends a re-registration message.

2.	The P-CSCF assesses the user's registration status. If the user is already registered, it forwards the re-registration message directly to the S-CSCF.

3-4. Authentication and registration timer update process are consistent with procedures outlined in Annex A of 3GPP TS 29.228 [5] and more details are described in 3GPP TS 24.229 [6] and 3GPP TS 23.228 [3].

5-6. According to the strategy in 3GPP TS 23.228 [3], step 5-6 are optional as an optimization. If the S-CSCF chooses to interact with HSS/UDM, when the HSS/UDM has no response or returns an appropriate error code, the S-CSCF shall skip the interactions with HSS/UDM and return 200 OK to the UE.

7-8. The S-CSCF returns 200 OK to the UE.

#### 6.1.1.3	(Re)-registration procedure for routing with I-CSCF

The below Figure 6.1.1.3-1 illustrates the Re-registration procedure for routing with I-CSCF when HSS/UDM failure or overload is detected.

Figure 6.1.1.3-1: Re-registration procedure for routing with I-CSCF

1.	Prior to the expiration of a user's registration, the user sends a re-registration message.

2.	The P-CSCF assesses the user's registration status. If the user is already registered, the P-CSCF adds the address of S-CSCF to a new header field and forwards the re-registration message to the I-CSCF.

3.	The I-CSCF identifies the new header field within the message. If it presents, I-CSCF forwards the re-registration message to the S-CSCF address obtained from the new header field.

4-5.	Authentication and registration timer update process are consistent with procedures outlined in Annex A of 3GPP TS 29.228 [5] and more details are described in 3GPP TS 24.229 [6] and 3GPP TS 23.228 [3].

6-7.	According to the strategy in 3GPP TS 23.228 [3], step 6-7 are optional as an optimization. If the S-CSCF chooses to interact with HSS/UDM, when the HSS/UDM has no response or returns an appropriate error code, the S-CSCF shall skip the interactions with HSS and return 200 OK to the UE.

8-10.	The S-CSCF returns 200 OK to the UE.

### 6.1.2	Impacts on services, entities and interfaces

If HSS/UDM failure or overload is detected, some enhancements are needed for the optimization.

#### 6.1.2.1	Routing without I-CSCF

The requirements for the modification of P-CSCF:

-	Add functionality for directly routing re-registration messages to S-CSCF.

The requirements for the modification of S-CSCF:

-	Bypass the HSS/UDM interaction during the re-registration procedure.

#### 6.1.2.2	Routing with I-CSCF

The requirements for the modification of P-CSCF:

-	Including the address of S-CSCF into a new header field.

The requirements for the modification of I-CSCF:

-	Identifying the new header field, if included, forwarding the re-registration message to the S-CSCF address obtained from the new header field;

The requirements for the modification of S-CSCF:

-	Bypass the HSS/UDM interaction during the re-registration procedure.

### 6.1.3	Pros

This solution provides different efficient ways to bypass the HSS/UDM if HSS/UDM failure or overload is detected.

### 6.1.4	Cons

The cons of the solution:

-	Enhancements to P-CSCF, I-CSCF and S-CSCF are required.

-	The method of routing without I-CSCF deviates from the signaling flows specified in 3GPP TS 23.228 [3], so this approach needs to be discussed in Stage 2.

## 6.2	Solution#2: Preventing the IMS disaster in the HSS overload scenario

### 6.2.1	Description

#### 6.2.1.1	General

This solution is proposed to address Key Issue #2 "Preventing the IMS disaster in the HSS overload scenario".

If the response failure rate from HSS arrives at the thresholds based on the indicators analysis by internal or external mechanisms, or the HSS identifies that it is in an overload situation by implementing specific means, such as the level of usage of internal resources (CPU, memory), the access to external resources, etc., as described in 3GPP TS 29.228 [5] Annex H and IETF RFC 7683 [8], or other internal mechanisms of S-CSCF, the S-CSCF will reduce the requests traffic to the HSS. The HSS as a reporting node determines the specific contents of OC-OLR AVP in overload reports and sends OC-OLR AVPs to I/S-CSCF as reacting nodes. The specific thresholds of HSS overload are defined based on the operator's policy.

This solution considers reducing the Diameter signalling interactions between S-CSCF/AS and HSS using stored available AVs and user profile, which contains two procedures as follows and discusses how to decrease HSS load in these procedures:

-	IMS initial registration procedure.

-	IMS third-party registration procedure.

#### 6.2.1.2	IMS initial registration of reducing HSS load

Figure 6.2.1.2-1 below illustrates the IMS initial registration procedure in the case of the presence of user data in the S-CSCF.

Figure 6.2.1.2-1: Initial registration procedure under the presence of user data in the S-CSCF

1-5.	The UE initiates an initial REGISTER request to the P-CSCF, the procedures between P-CSCF, I-CSCF, S-CSCF, and HSS, referring to procedures in 3GPP TS 29.228 [5] Annex A and more details as described in 3GPP TS 24.229 [6] and 3GPP TS 23.228 [3].

6.	The S-CSCF receives a REGISTER request with the tag of "integrity-protected=no", which means that the registration type is initial REGISTER described in 3GPP TS 33.203 [9]. When the S-CSCF detects the HSS overload, the S-CSCF shall initiate the initial registration optimized mechanism to check whether any available AV and user profiles are stored.

7.	If there are stored available AVs, the S-CSCF shall skip steps 7-8. If there are no stored available AVs, the S-CSCF shall send MAR request to obtain AVs to HSS.

8.	The HSS shall generate AVs, including RAND, XRES, CK, IK, and AUTN, then send them to the S-CSCF.

9.	The S-CSCF selects one group of AVs, and sends the 401 (Unauthorized) response to the initial REGISTER request to the I-CSCF.

10-16.	These steps as described in 3GPP TS 24.229 [6].

17.	The S-CSCF sends the SAR message to the HSS.

18.	The HSS shall send the SAA response to the S-CSCF.

-	If the S-CSCF receives the Result-Code in the SAA response as DIAMETER_UNABLE_TO_DELIVER (3002) or DIAMETER_UNABLE_TO_COMPLY (5012), or time-out of SAA response, the S-CSCF shall ignore the error response and send 200 OK response to the I-CSCF. Meanwhile, the S-CSCF shall set the UPDATE RESTORATION INFO flag as PENDING while next receives a REGISTER request, the S-CSCF resends the SAR request to the HSS.

19.	The S-CSCF shall return the 200 OK response to the I-CSCF.

20-21.	The I-CSCF sends 200 OK response to the P-CSCF and the P-CSCF sends 200 OK response to the UE.

#### 6.2.1.3	IMS third-party registration of reducing HSS load

Upon successful completion of the initial registration procedure, the S-CSCF shall send a third-party REGISTER request to the MMTel AS and IP-SM-GW according to the stored context of the UE. The diameter signalling interaction between MMTel AS/ IP-SM-GW and HSS can be further reduced in case of HSS overload. Figure 6.2.1.3-1 below illustrates the third-party registration procedure under the presence of user data in MMTel AS/IP-SM-GW.

Figure 6.2.1.3-1: Third-party registration procedure under the presence of user data in MMTel AS/IP-SM-GW

1.	S-CSCF sends a third-party REGISTER request to the MMTel AS. UDR and SNR requests needn't be sent to obtain user data due to the presence of user data in the MMTel AS.

2.	MMTel sends the 200 OK response to the S-CSCF.

3.	S-CSCF sends a third-party REGISTER request to the IP-SM-GW. UDR and SNR requests also needn't be sent to obtain UE's data due to the presence of user data in the IP-SM-GW.

4.	IP-SM-GW sends the 200 OK response to the S-CSCF.

### 6.2.2	Impacts on services, entities and interfaces

Compared to procedures defined in 3GPP TS 23.228 [3], some enhancements of the S-CSCF are needed for the optimization.

1.	Determining whether stored available AVs present while the S-CSCF receives an initial REGISTER request.

2.	Optimizing the handling mechanism of SAR/SAA messages while the S-CSCF receives an initial REGISTER request during the registration procedure.

3.	If the S-CSCF receives the Result-Code in the SAA response as DIAMETER_UNABLE_TO_DELIVER (3002) or DIAMETER_UNABLE_TO_COMPLY (5012), or time-out of SAA response, the S-CSCF shall ignore the error response and send 200 OK response to the I-CSCF. Meanwhile, the S-CSCF shall set the UPDATE RESTORATION INFO flag as PENDING while next receives a REGISTER request, the S-CSCF resends the SAR request to the HSS.

4.	The third-party REGISTER request is sent to the same AS from the S-CSCF as the process of normal use by the S-CSCF the last used AS according to UE's iFCs, which can further simplify the Diameter signalling (Sh) between AS and HSS.

### 6.2.3	Pros

The pros of the solution:

-	The solution reduces MAR/MAA signalling over Cx interface, and UDR/UDA and SNR/SNA signalling over Sh interface the UE-initiated initial registration procedure in case of HSS overload with stored user data. Especially for Sh interface signalling, which significantly reduces the signalling amplification factor of the Diameter signalling network.

-	The solution can further decrease the probability of signalling storm and accelerate failure restoration for the IMS network.

### 6.2.4	Cons

The cons of the solution:

-	Enhancements to S-CSCF are required.

## 6.3	Solution#3: Solution for maintaining the IMS registration and call services uninterruptible in case of HSS/UDM failure

### 6.3.1	Description

#### 6.3.1.1	General

This solution is proposed to address Key Issue #1 "Continue service for registered users in case of HSS/UDM failure or overload".

This solution considers to continue IMS service for registered UE, in the following procedures:

-	IMS initial registration procedure;

-	IMS re-registration procedure;

-	IMS MT procedure.

During the IMS (re-)registration and MT services procedures, I-CSCF will retrieve the S-CSCF address in the terminating network from the HSS of the terminating UE. To ensure successful S-CSCF address retrieval in case of HSS/UDM failure, the S-CSCF address is stored in a new AS during the registration process.

#### 6.3.1.2	Storage of S-CSCF Address

The below Figure 6.3.1.2-1 illustrates the storage of S-CSCF address during the IMS registration procedure.

Figure 6.3.1.2-1: Storage of S-CSCF address in initial registration procedure

1-2.	UE has registered to the IMS network.

3-4.	S-CSCF sends the third-party registration request to a new AS based on the iFC information. Upon a successful registration, the AS will store the S-CSCF address, IMSI, MSISDN for the UE.

NOTE:	The new AS is unique and its address is pre-configured in the I-CSCF, dedicated for HSS bypass in case of HSS failure..

#### 6.3.1.3	IMS initial registration procedure in case of HSS failure

Although the UE has registered to the IMS network, it still has the possibility to initiate an initial registration request in some cases. For example, the exchange of P-CSCF, PDN connection re-established, etc. At the same time, the used S-CSCF is the same as the last used S-CSCF and the new AS has restored the UE registration-related data (e.g. S-CSCF address, IMSI, MSISDN, etc.). Therefore, the UE data on the AS can be fully used for the completion of the initial registration procedure for the UE. The below Figure 6.3.1.3-1 illustrates the initial registration procedure of HSS bypass.

NOTE:	If a UE does not have an allocated S-CSCF, this solution for IMS initial registration does not work.

Figure 6.3.1.3-1: IMS initial registration procedure of HSS bypass

1-2.	The UE initiates an initial REGISTER request to the P-CSCF and the P-CSCF forwards the initial REGISTER request to the I-CSCF.

3-4.	I-CSCF fails to send a UAR or doesn’t receive a corresponding response from HSS, then it forwards the REGISTER request to the AS based on the pre-configured address of the AS.

5a.	AS supports forwarding the REGISTER request to the S-CSCF according to the UE registration-related data (e.g. S-CSCF address, IMSI, MSISDN, etc.). The UE data is obtained directly or indirectly from the ISC interface (S-CSCF-AS).

5b-5c.	AS also supports returning the 305 message with the S-CSCF address to the I-CSCF. Then the I-CSCF sends the REGISTER to the S-CSCF.

6a-6b.	During the IMS initial registration procedure, the HSS will send an ordered array of authentication vectors to the S-CSCF as described in clause 6.1.1 of 3GPP TS 33.203 [9]. Therefore, when the used S-CSCF is the same as the last used S-CSCF and existing available AVs, the S-CSCF selects one group of available AVs and returns the 401 (Unauthorized) response to the initial REGISTER request to the AS. Then the AS forwards the 401 (Unauthorized) response to the I-CSCF.

6c.	The S-CSCF selects one group of available AVs and returns the 401 (Unauthorized) response to the initial REGISTER request to the I-CSCF.

7-10.	These steps are the same as the procedure of solution 2 described in clause 6.2.1.2.

11-13.	These steps are the same as the 3 to 6 steps above.

14-16.	The S-CSCF shall return the 200 OK response to the UE by the AS/I-CSCF and the P-CSCF.

#### 6.3.1.4	IMS re-registration procedure in case of HSS failure

According to 3GPP TS 24.229 [6], upon the initial registration being successful, the P-CSCF will store the address of the S-CSCF serving the specific subscriber. The re-registration procedures are then the same as in solution 1. Either the procedure with I-CSCF described in clause 6.1.1.2 or the procedure without I-CSCF described in clause 6.1.1.3 may be used.

#### 6.3.1.5	IMS MT procedure in case of HSS failure

The below Figure 6.3.1.5-1 illustrates the MT procedure of HSS bypass.

Figure 6.3.1.5-1: IMS MT procedure of HSS bypass

1.	I-CSCF in the terminating network receives an incoming INVITE request.

2-3.	I-CSCF fails to send an LIR or doesn’t receive a corresponding response from HSS, then it forwards the INVITE request to the AS based on the pre-configured address of the AS.

4a.	AS supports forwarding the INVITE request to the S-CSCF according to the UE registration-related data (e.g. S-CSCF address, IMSI, MSISDN, etc.). The UE data is obtained directly or indirectly from the ISC interface (S-CSCF-AS).

4b-4c.	AS also supports returning the 305 message with the S-CSCF address to the I-CSCF. Then the I-CSCF sends the INVITE to the S-CSCF.

5-6.	S-CSCF forwards the INVITE request to UE B through the P-CSCF.

7.	18X/200 OK/ACK procedure proceeds.

### 6.3.2	Impacts on services, entities and interfaces

The requirement for I-CSCF:

-	The capability to forward the REGISTER and INVITE request to the AS in order to obtain the S-CSCF address, IMSI, and MSISDN for serving the subscriber.

The requirement for the AS:

-	The capability to forward the REGISTER and INVITE request to the S-CSCF serving the subscriber based on the UE data obtained directly or indirectly from the ISC interface.

or

-	The capability to return the 305 message with the S-CSCF address to the I-CSCF based on the UE data obtained directly or indirectly from the ISC interface.

### 6.3.3	Pros

The pros for both Option A and Option B:

-	This solution provides an efficient way to bypass the failed HSS and associated paths.

-	This solution can bypass the HSS to keep the IMS (re-)registration and MT procedures uninterruptible when all the HSS instances fail. It supplements and improves solution 1.

The pros only for Option A:

-	The AS can directly route REGISTER and INVITE messages to the S-CSCF of the UE.

The pros only for Option B:

-	The AS first needs to return 305 (Use Proxy) with the S-CSCF address, and then the I-CSCF routes REGISTER and INVITE messages to the S-CSCF.

### 6.3.4	Cons

The cons for both Option A and Option B:

-	Enhancements to S-CSCF to configure the iFC of the new AS in the third-party registration, which will increase the overhead of registration signalling.

-	Enhancements to I-CSCF to forward REGISTER and INVITE requests to the AS.

-	The address of the AS needs to be preconfigured for I-CSCF in this solution.

-	Restoration of the new AS is needed to be considered.

The cons only for Option A:

-	Enhancements to the AS to forward REGISTER and INVITE requests to the S-CSCF serving the subscriber.

The cons only for Option B:

-	Enhancements to the AS to return 305 (Use Proxy) message to the I-CSCF.

## 6.4	Solution #4: Solution for HSS/UDM bypass

### 6.4.1	Description

#### 6.4.1.1	General

This solution addresses Key Issue #1 "Continue service for registered users in case of HSS/UDM failure or overload".

Solution 1 has provided a solution addressing Key Issue #1 too. But it only considers the case that the P-CSCF has the address of the S-CSCF serving the subscriber when it receives a (re-)REGISTER request. This solution considers the case that the P-CSCF does not have the address of the S-CSCF address serving the subscriber. The typical scenario is that a UE attaches to a new P-GW for roaming or mobility.

This solution contains four procedures as follows and discusses how to bypass HSS/UDM in these procedures:

-	IMS deregistration procedure;

-	IMS initial registration procedure;

-	IMS re-registration procedure;

-	IMS terminating session procedure.

The MO procedure need not be considered for it has no interaction with the HSS/UDM. That is, there is no impact to the originating session procedure of VoLTE/VoNR when all the HSS/UDM instances fail.

#### 6.4.1.2	IMS deregistration procedure of HSS/UDM bypass

According to the IMS de-registration procedure specified in clause 5.3.1 of 3GPP TS 23.228 [3], the S-CSCF shall remove all subscription information related to this specific Public User Identity.

To address the abnormal cases such as HSS/UDM failure or overload discussed in Key Issue #1, it’s recommended that the S-CSCF delays removing all subscription information related to this specific Public User Identity in the de-registration procedure. This is a prerequisite for subsequent processes to be implemented. The delay time depends on the operator's policy.

#### 6.4.1.3	IMS initial registration procedure of HSS/UDM bypass

Figure 6.4.1.3-1 below illustrates the IMS initial registration procedure in the case of all the HSS/UDM instances failure.

Figure 6.4.1.3-1: IMS initial registration procedure of HSS/UDM bypass

1-3.	UE A initiates an initial REGISTER request to the P-CSCF and the P-CSCF sends the initial REGISTER request to the I-CSCF. The I-CSCF detects that all the HSS/UDM instances in the network fail.

4-5.	Then the I-CSCF sends OPTIONS requests to all the S-CSCFs in the network to query which has the user data of UE A. The OPTIONS requests contain the identity of UE A.

6-7.	The S-CSCF-2 which has the subscription information of UE A will returns a 200 OK response to the I-CSCF, including the address of itself in Contact header field, other S-CSCFs will return a 404 Not Found response to the I-CSCF.

NOTE:	The I-CSCF can use some mechanism to improve the query efficiency of OPTIONS. For example, it can use the same S-CSCF selection algorithm used after it gets the S-CSCF capability set from the HSS/UDM, select the first S-CSCF it queries, or it sends out the OPTOINS requests in batches, that is, it sends the next batch of OPTIONS requests out only when all it receives are 404 Not Found responses and the S-CSCFs in the same pool with it are queried in the first batch if the pool interworking is used.

8.	Then the I-CSCF routes the initial REGISTER request to the S-CSCF-2's address carried in the received 200 OK response.

9.	The S-CSCF-2 returns a 401 Unauthorized response to the I-CSCF, including the address of itself in Service-Route header field.

10-11.	The 401 Unauthorized response will be returned to UE A via the P-CSCF. Then the P-CSCF knows the address of the S-CSCF-2 serving UE A.

Then UE A sends an initial REGISTER request to the P-CSCF and the procedure is same with the procedure of solution 1. Both the procedure with I-CSCF described in clause 6.1.1.2 and the procedure without I-CSCF described in clause 6.1.1.3 can be used here.

#### 6.4.1.4	IMS re-registration procedure of HSS/UDM bypass

According to 3GPP TS 24.229 [6], the P-CSCF will store the address of the S-CSCF serving the specific subscriber after the initial registration is successful. Then the P-CSCF can route the re-REGISTER request to the corresponding S-CSCF directly or it can carry the address of the corresponding S-CSCF in some header field of the re-REGISTER request and forward the re-REGISTER request to the I-CSCF, then the I-CSCF can routes the re-REGISTER request to the corresponding S-CSCF based on the address added by the P-CSCF.

#### 6.4.1.5	MT procedure of HSS/UDM bypass

Figure 6.4.1.5-1 below illustrates the MT procedure in the case of all the HSS/UDM instances failure.

Figure 6.4.1.5-1: IMS MT procedure of HSS/UDM bypass

1-2.	The I-CSCF in the terminating network receives an incoming INVITE request and it detects that that all the HSS/UDM instances fail.

3-4.	Then the I-CSCF sends OPTIONS requests to all the S-CSCFs in the network to query which has the user data of UE B. The OPTIONS requests contain the identity of UE B.

5-6.	The S-CSCF-2 which has the subscription information of UE B will returns a 200 OK response to the I-CSCF, including the address of itself in Contact header field, other S-CSCFs will return a 404 Not Found response to the I-CSCF.

NOTE:	The I-CSCF can use some mechanism to improve the query efficiency of OPTIONS. For example, it can use the same S-CSCF selection algorithm used after it gets the S-CSCF capability set from the HSS/UDM, select the first S-CSCF it queries, or it sends out the OPTOINS requests in batches, that is, it sends the next batch of OPTIONS requests out only when all it receives are 404 Not Found responses, and the S-CSCFs in the same pool with it are queried in the first batch if the pool interworking is used.

7.	Then the I-CSCF routes the incoming INVITE request to the S-CSCF-2's address carried in the 200 OK response it received.

8-9.	The S-CSCF sends the INVITE request to UE B via the P-CSCF.

10.	18X/200 OK/ACK procedure proceeds.

### 6.4.2	Impacts on services, entities and interfaces

The requirements for I-CSCF:

-	Query the subscriber's serving S-CSCF from S-CSCF through OPTIONS/200 OK interaction when detecting the HSS/UDM failure.

The requirements for S-CSCF:

-	Delay removing all subscription information related to this specific Public User Identity in the de-registration procedure.

-	Return the address of itself in the 200 OK response of OPTIONS request from I-CSCF if it has the subscription information of the specific subscriber.

### 6.4.3	Pros

This solution can bypass the HSS/UDM to continue the IMS (re-)registration and the IMS terminating session procedure when all the HSS/UDM instances fail, even though P-CSCF does not have the address of the S-CSCF serving the subscriber. It supplements and improves solution 1.

### 6.4.4	Cons

Enhancements to I-CSCF and S-CSCF are required.

Additional signaling overhead is introduced.

## 6.5	Solution #5: Solution for PCRF/PCF bypass

### 6.5.1	Description

#### 6.5.1.1	General

This solution addresses Key Issue #2 "Handle all PCRF/PCF instances failure scenario with minimum impact to the registered IMS users". It covers the following procedures which need to interact with PCRF/PCF and discusses how to bypass PCRF/PCF in these procedures:

-	Default IMS bearer establishment procedure for IMS signaling;

-	IMS originating session procedure in which the dedicated IMS bearer(s) need to be established for IMS media of the originating UE;

-	IMS terminating session procedure in which the dedicated IMS bearer(s) need to be established for IMS media of the terminating UE.

-	IMS session release procedure in which the dedicated IMS bearer(s) need to be terminated for both the originating UE and the terminating UE.

#### 6.5.1.2	Default IMS bearer establishment procedure of PCRF/PCF bypass

Before an IMS subscriber initiates a registration on an IMS network, a default IMS bearer must be established. Figure 6.5.1.2-1 below illustrates the default IMS bearer establishment procedure in the case of all the PCRF/PCF instances failure.

Figure 6.5.1.2-1: Default IMS bearer establishment procedure of PCRF/PCF bypass

1-2.	UE A sends a PDN Connectivity Request message to the MME/AMF to request a PDN connection. The MME/AMF allocates a bearer ID and sends a Create Session Request message to the PGW-C/SMF.

3.	The PGW-C/SMF detects that all the PCRF/PCF instances in the network fail, then it uses local PCC policy to establish a default IMS bearer (QCI/5QI=5) for IMS signaling.

4.	The subsequent procedure of the default IMS bearer establishment proceeds.

5.	After the default IMS bearer is established, UE A initiates the initial registration procedure which specified in 3GPP TS 23.228 [3].

#### 6.5.1.3	IMS MO procedure of PCRF/PCF bypass

Figure 6.5.1.3-1 below illustrates the IMS MO procedure in the case of all the PCRF/PCF instances failure.

Figure 6.5.1.3-1: IMS MO procedure of PCRF/PCF bypass

1-3.	UE A initiates an initial INVITE request to the P-CSCF to establish an IMS session. The P-CSCF routes the INVITE request to the I-/S-CSCF. Then the I-/S-CSCF returns a 183 response to the P-CSCF.

4-7.	The P-CSCF detects that all the PCRF/PCF instances fail, and starts the PCRF/PCF bypass procedure by sending an enhanced H.248 MOD request to the IMS-AGW. After receiving the enhanced H.248 MOD request, the IMS-AGW sends RTCP RR (Receiver Report) packets to UE A and returns an H.248 MOD response to the P-CSCF.

8-9.	The PGW-U/UPF detects the RTCP RR packets and finds there is no dedicated IMS bearer which has been established, then it starts the PCRF/PCF bypass procedure by sending a PFCP Session Report Request to the PGW-C/SMF.

10-12.	The PGW-C/SMF uses local PCC policy to establish the dedicated IMS bearer for IMS media and returns a PFCP Session Report Response to the PGW-U/UPF. The subsequent procedure of the dedicated IMS bearer establishment proceeds.

13-14.	The P-CSCF sends the 183 response to UE A and the subsequent procedure of IMS originating session proceeds.

#### 6.5.1.4	IMS MT procedure of PCRF/PCF bypass

Figure 6.5.1.4-1 below illustrates the IMS terminating session procedure in the case of all the PCRF/PCF instances failure.

Figure 6.5.1.4-1: IMS MT procedure of PCRF/PCF bypass

1-4.	The I-/S-CSCF in the terminating network receives an incoming INVITE request and routes it to UE B through the P-CSCF. UE B sends a 183 response to the P-CSCF.

5-8.	The P-CSCF detects that all the PCRF/PCF instances fail, and starts the PCRF/PCF bypass procedure by sending an enhanced H.248 MOD request to the IMS-AGW. After receiving the enhanced H.248 MOD request, the IMS-AGW sends RTCP RR (Receiver Report) packets to UE B and returns an H.248 MOD response to the P-CSCF.

9-10.	The PGW-U/UPF detects the RTCP RR packets and finds there is no dedicated IMS bearer which has been established, then it starts the PCRF/PCF bypass procedure by sending a PFCP Session Report Request to the PGW-C/SMF.

11-12.	The PGW-C/SMF uses local PCC policy to establish the dedicated IMS bearer for IMS media and returns a PFCP Session Report Response to the PGW-U/UPF. The subsequent procedure of the dedicated IMS bearer establishment proceeds.

13-15.	The P-CSCF sends the 183 response to the originating network through the I-/S-CSCF and the subsequent procedure of IMS terminating session proceeds.

#### 6.5.1.5	Dedicated IMS bearer termination procedure of PCRF/PCF bypass

When releasing the established IMS session, the normal dedicated IMS bearer termination procedure cannot be performed in the case of all PCRF/PCF instances failure. The PGW-U/UPF need to detect the traffic over the dedicated IMS bearers. When it finds that there is no traffic over some dedicated IMS bearer for longer than the timer which is configured by the operator, the PGW-C/SMF initiate bearer termination procedure for the dedicated IMS bearer.

### 6.5.2	Impacts on services, entities and interfaces

The requirements for P-CSCF:

-	Support to detect that all the PCRF/PCF instances fail and start the PCRF/PCF bypass procedure by sending an enhanced H.248 MOD request to the IMS-AGW.

The requirements for IMS-AGW:

-	Support RTCP Receiver Report packets to UE when receiving an enhanced H.248 MOD request.

The requirements for PGW-U/UPF:

-	Support to detect RTCP RR packets and start the PCRF/PCF bypass procedure by sending a Session Report Request to the PGW-C/SMF when there is no dedicated IMS bearer which has been established.

-	Support to detect the traffic over the dedicated IMS bearers and initiate bearer termination procedure for the dedicated IMS bearer when it finds that there is no traffic over some dedicated IMS bearer for longer than the timer.

The requirements for PGW-C/SMF:

-	Support to bypass PCRF/PCF by using local PCC policy to establish the default IMS bearer for IMS signaling when detecting that all the HSS/UDM instances fail.

-	Support to bypass PCRF/PCF by using local PCC policy to establish the dedicated IMS bearer for IMS media when receiving a Session Report Request from the PGW-U/UPF.

### 6.5.3	Pros

This solution can bypass the PCRF/PCF to continue the default IMS bearer establishment, the IMS originating session and the IMS terminating session procedure when all the PCRF/PCF instances fail.

### 6.5.4	Cons

-	This solution introduces the extra requirements described in clause 6.5.2 to the PGW-U/UPF, PGW-C/SMF, P-CSCF and IMS-AGW when all the PCRF/PCF instances fail and it will increase the system overhead of these NFs.

-	This solution does not support scenarios where there is a need to interact with the PCRF/PCF, e.g., for Network Provided Location Information as specified in 3GPP TS 23.203 [10] and 3GPP TS 23.503 [7], or to get the roaming status of the UE, or to modify the QoS parameters of the media session, or build the new dedicated bearer(s) after the media re-negotiation.

## 6.6	Solution #6: Solution for speeding up the restoration procedure of the terminating session when EPC/5GC NF fails

### 6.6.1	Description

#### 6.6.1.1	General

This solution addresses Key Issue #3 "Speed up the restoration procedure of the terminating session when EPC/5GC NF fails".

In most of EPC/5GC NF failure scenarios, the UE could invoke re-attach and re-registration procedures quickly after the failure is restored if

-	the UE initiates a new IMS session and some SIP error code is returned by the IMS network; or

-	the EPC/5GC network detects the failure and kicks off the related UEs to re-attach.

Then the UE’s service will be restored soon. In addition, the UE’s terminating session can be available if the operator’s network supports CS Fallback.

But when the operator’s network does not support CS Fallback, the re-attach and re-registration procedures will not be invoked immediately, the UE's terminating session will be unavailable until its IMS registration is expired and it invokes the re-registration procedure. If the UE’s registration expiration time is 6000 seconds, the unavailable time is 50 minutes in the worst case, as specified in clause 5.1.1.4.1 of 3GPP TS 24.229 [6].

The solution can avoid this situation which makes use of the HSS based P-CSCF Restoration procedure specified in 3GPP TS 23.380 [2] when the P-GW failure is detected by the P-CSCF serving the terminating UE.

#### 6.6.1.2	Quick restoration procedure of IMS terminating session

Figure 6.6.1.2-1 illustrates the quick restoration procedure of IMS terminating session in the EPC network below. It can also be used in the 5GC network and the only difference is that the Diameter messages in Figure 6.6.1.2-1 need to be replaced with the corresponding SBI messages.

Figure 6.6.1.2-1: Quick restoration procedure of IMS terminating session

1-3.	The I-/S-CSCF in the terminating network receives an incoming INVITE request and routes it to the P-CSCF serving UE B. Then the P-CSCF routes the INVITE request to UE B.

4-5.	The P-CSCF does not receive any response from UE B after some time out, so the P-CSCF determines that the SIP signalling has already attempted the corresponding re-transmissions or retrials. Therefore, the P-CSCF sends a Rx Authorization-Authentication-Request (AAR) with the Specific-Action AVP set as a new value indicating that it's a test to check whether the P-GW is normal to the PCRF to check whether there is a failure in EPC network.

6-7.	If the PCRF has identified that the P-GW is reachable, then it sends a Gx Re-Auth-Request (RAR) to the P-GW according to the received AAR.

8-9.	If the P-GW finds that there is no PDU session context of User B, it returns an Rx Re-Auth-Answer (RAA) with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) specified in IETF RFC 6733 [12] to the PCRF.

10.	The PCRF returns an Rx Authorization-Authentication-Answer (AAA) with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) specified in IETF RFC 6733 [12] or the Rx Experimental-Result-Code AVP set to IP-CAN_SESSION_NOT_AVAILABLE (5065) specified in 3GPP TS 29.214 [11] to the P-CSCF.

11-12.	The P-CSCF sends a 503 Service Unavailable response with the P-CSCF Restoration indication to the I-/S-CSCF. Then the I-/S-CSCF starts the HSS based P-CSCF Restoration procedure, as specified in clause 5.4.3.1 of 3GPP TS 23.380 [2].

During the HSS based P-CSCF Restoration procedure, re-attach and IMS registration procedure of UE B will be invoked quickly. Then UE B will receive the next incoming session for it has re-connected to the EPC network successfully.

### 6.6.2	Impacts on services, entities and interfaces

The requirements for P-CSCF:

-	Support to send an Rx Authorization-Authentication-Request (AAR) with the Specific-Action AVP set as a new value indicating that it's a test to check whether the P-GW is normal when timeout.

-	Support to send a 503 Service Unavailable response with the P-CSCF Restoration indication to the I-/S-CSCF after receiving the Rx AAA with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) or the Rx Experimental-Result-Code AVP set to IP-CAN_SESSION_NOT_AVAILABLE (5065).

The requirements for S-CSCF:

-	Support to start the HSS based P-CSCF Restoration procedure specified in 3GPP TS 23.380 [2] after receiving a 503 Service Unavailable response with the P-CSCF Restoration indication.

The requirements for PCRF:

-	Support to send a Gx Re-Auth-Request (RAR) to the P-GW after receiving an Rx AAR with the Specific-Action AVP set as ACCESS_NETWORK_INFO_REPORT (12) from the P-CSCF.

-	Support to return a Rx AAA with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) or the Rx Experimental-Result-Code AVP set to IP-CAN_SESSION_NOT_AVAILABLE (5065) to the P-CSCF when a response with the Result-Code AVP set to DIAMETER_UNKNOWN_SESSION_ID (5002) is returned by the P-GW.

The requirement for P-GW:

-	Support to return a Gx RAA with the specific error code to the PCRF when the P-GW is not reachable.

### 6.6.3	Pros

This solution can speed up the restoration procedure of the terminating session when the P-GW fails and utilizes the existing HSS based P-CSCF Restoration procedure specified in 3GPP TS 23.380 [2].

### 6.6.4	Cons

This solution introduces the extra requirements described in clause 6.6.2 to the P-GW, P-CSCF and PCF when the terminating UE does not respond to the initial INVITE request and it will increase the system overhead of these NFs.

## 6.7	Solution#7: Enhance the P-CSCF restoration in the case of 5GC/EPC UP failure

### 6.7.1	Description

#### 6.7.1.1	Gerenal

The solution is to address the Key Issue #3: "Speed up the restoration procedure of the terminating session when EPC/5GC NF fails".

When the PGW-U or the UPF fails, the current IMS session will fail. On development of network, the 3G network will be taken out of the network and CS fallback will not be supported. Thus the UE will not be informed of such failure. The UE will remain unavailable until it initiates any IMS procedure, i.e. re-registration, or originating IMS session.

In order to inform the UE of CN node failure immediately, the HSS based P-CSCF Restoration procedure specified in 3GPP TS 23.380 [2] can be re-used with some enhancement of notifying CN user plane (e.g. PGW-U/UPF) failure.

#### 6.7.1.2	CN user plane failure detected

The below Figure 6.7.1.2-1 illustrates ICMP message to notify CN user plane (e.g. PGW-U/UPF) failure.

Figure 6.7.1.2-1: CN user plane failure detected during the IMS terminating session

1-3.	The S-CSCS in the originating network sends SIP INVITE to the I/S-CSCF in the terminating network. The I/S-CSCF routes the INVITE request to the P-CSCF in the terminating network.

4.	The P-CSCF in terminating network sends SIP INVITE as the DL data to the PGW-U/UPF.

5-6.	If the PGW-U/UPF cannot forward any DL data including SIP INVITE to the UE because of the UP plane failure. Based on the operator’s policy, the UPF detects the delivery failure of IMS related messages. The UPF sends the ICMP Destination Unreachable message to the P-CSCF.

NOTE 1:	The PGW-U/UPF may detect IMS related messages based on Deep Packet Inspection (DPI) mechanism of data detection. IMS service is one of the most important services for operators. Thus operators may configure some reserved IP addresses and ports to transfer IMS service. The PGW-U/UPF may identify IMS related messages based on these reserved address and port.7.	On receiving ICMP Destination Unreachable message, P-CSCF decides to send a response with an error code (i.e. the UE registration data is not present) to the S-CSCF or not send any response. The S-CSCF shall identify whether the called UE's terminating P-CSCF is not able to process this request, based on received error codes (i.e. the UE registration data is not present) or no response. Then the HSS based P-CSCF restoration as specified in clause 5.4.2.1 in TS23.380 is invoked.

NOTE 2:	There may be different cases of DL data forwarding failure, e.g. RAN node failure, UE unreachable, CN congestion, and etc. The PGW-U/UPF cannot differentiate these cases. Although the P-CSCF restoration is invoked, the restoration may not be performed successfully because the signaling may not be transferred to the UE e.g. in the case of RAN failure or CN congestion. The procedure should not increase the congestion situation when CN congestion happens.

### 6.7.2	Impacts on services, entities and interfaces

The enhancements for PGW-U/UPF:

-	Send an ICMP Destination Unreachable message if the IMS related messages cannot be transferred to the UE.

The enhancements for P-CSCF:

-	Consider the CN failure cases indicated by PGW-U/UPF as P-CSCF failure and trigger the HSS based P-CSCF restoration.

### 6.7.3	Pros

This solution can speed up the restoration procedure of the terminating session when the CN UP (i.e. P-GW-U or UPF) fails and utilizes the existing HSS based P-CSCF Restoration procedure specified in 3GPP TS 23.380 [2].

This solution re-uses the ICMP message to report routing error.

### 6.7.4	Cons

This solution enhances implementation logic on the P-CSCF and PGW-U/UPF.

## 6.8	Solution#8: Enhance the P-CSCF restoration in the case of 5GC/EPC failure

### 6.8.1	Description

#### 6.8.1.1	Gerenal

The solution is to address the Key Issue #3: "Speed up the restoration procedure of the terminating session when EPC/5GC NF fails".

When PGW-C/SMF or PGW-U/UPF fails, the current IMS session will fail. On development of network, the 3G network will be taken out of the network and CS fallback will not be supported. Thus the UE will not be informed of such failure. The UE will remain unavailable until it initiates any IMS procedure, i.e. re-registration, or originating IMS session.

In order to inform the UE of CN node failure immediately, the HSS based P-CSCF Restoration procedure specified in 3GPP TS 23.380 [2] can be re-used with some enhancement of notifying CN node (e.g. PGW-C/SMF) failure.

#### 6.8.1.2	PCRF/PCF detecting PGW-C/SMF failure

The below Figure 6.8.1.2-1 illustrates procedure that the PCRF/PCF detects and notifies the PGW-C/SMF failure.

Figure 6.8.1.2-1: PCRF/PCF detecting PGW-C/SMF failure during the IMS terminating session

1-3.	The S-CSCF in the originating network sends SIP INVITE to the I/S-CSCF in the terminating network. The I/S-CSCF routes the INVITE request to the P-CSCF in the terminating network.

4-5.	When the P-CSCF (acting as the AF) determines that session related media requires PCC supervision, or the access network information (i.e. user location and/or user timezone information) is required, the P-CSCF in terminating network sends a Rx Authorization-Authentication-Request (AAR) to the PCRF as specified in TS 29.214 and the PCRF returns a Rx Authorization-Authentication-Answer (AAA) to the P-CSCF.

6-7.	The PCRF/PCF decides to detect the availability of CN nodes. PCRF sends a Gx Re-Auth-Request (RAR) to the PGW-C/SMF for availability detection and receives no response if the PGW-C/SMF fails. If the PGW-C/SMF detects some errors in user plane, it may send a response with service unavailability indication.

If PCF and SMF are utilized, the Npcf_SMPolicyControl_UpdateNotify is invoked. When the entire SMF set fails, each response returned from the SMF in the SMF set indicates the service unavailability or there is no response received.

NOTE:	The PCRF/PCF decides to detect the availability of CN nodes based on the operator’s policy configuration. Frequent CN node failure detection shall be avoided. For example, the PCRF/PCF is configured to count the number of Rx sessions associated with the same PGW-C/SMF or the PCRF/PCF triggers the detection periodically.

8.	The CN nodes failure is detected in above step, PCRF/PCF sends Rx Abort-Session-Request (ASR) to the P-CSCF including the service unavailable indication. The PCRF/PCF may store the failure status of the CN failure related to the same PGW-C or the same SMF set locally for future usage e.g. to avoid triggering Gx RAR message in each time. If any request is received from the PGW-C or any SMF in the SMF set, the PCRF/PCF will update the status of the PGW-C or the SMF set to available.

9.	Based on this indication, P-CSCF decides to send a response with an error code (i.e. the UE registration data is not present) to the S-CSCF or not send any response. The S-CSCF shall identify whether the called UE's terminating P-CSCF is not able to process this request, based on received error codes (i.e. the UE registration data is not present) or no response. Then the HSS based P-CSCF restoration as specified in clause 5.4.2.1 in TS23.380 is invoked.

### 6.8.2	Impacts on services, entities and interfaces

The enhancements for PCRF/PCF:

-	Decide to initiate the Gx RAR to the PGW-C or invoke Npcf_SMPolicyControl_UpdateNotify to the SMF in the SMF set based on operator’s policy on receiving Rx AAR from P-CSCF.

-	Send the Rx ASR to indicate CN node failure, if PGW-C/SMF or PGW-U/UPF failure is detected.

-	Store the CN nodes failure status for future usage e.g. to avoid triggering Gx RAR message in each time.

The enhancements for PGW-C/SMF:

-	Indicate UP node failure in the Gx RAA or Npcf_SMPolicyControl_UpdateNotify response.

The enhancements for P-CSCF:

-	Consider the CN failure cases indicated by the PCRF/PCF as P-CSCF failure and trigger the HSS based P-CSCF restoration on receiving the service unavailable indication from the PCRF/PCF.

### 6.8.3	Pros

This solution can speed up the restoration procedure of the terminating session when the CN nodes (i.e. PGW-C/SMF, PGW-U/UPF) fails and utilizes the existing HSS based P-CSCF Restoration procedure specified in 3GPP TS 23.380 [2].

This solution enables the operator to configure the detection frequencies to avoid frequent detection to the PGW-C/SMF.

### 6.8.4	Cons

This solution extends the messages exchanged between the PCRF/PCF and the P-CSCF, and between the PCRF/PCF and the PGW-C/SMF.

This solution requires operator to configure the proper detection period. It is difficult to make balance between heavy load triggered by the detection and detecting failure in time. Too frequent detection which may bring more load to the CN should be avoided. However, the long periodic may cause the failure cannot be detected in time.

# 7	Evaluations

This clause describes evaluations of the solutions.

## 7.1	Evaluation of Solutions for Key Issue#1

A group of Solutions have been proposed for Key Issue#1 to address the IMS (re-)registration and MT services interruption for UEs due to HSS failure or overload.

Solution #1 ensures successful IMS re-registration of a registered UE in the IMS network when HSS fails or overloads. This prevents the failure of IMS re-registration from causing the UE to initiate an initial registration and accelerate the formation of the signalling storm.

In the solution for the re-registration process without I-CSCF, P-CSCF forwards the REGISTER request directly to the S-CSCF to enable re-registration bypassing the HSS while minimally altering the existing network functionality.

The solution of the re-registration procedure for routing with I-CSCF adds the S-CSCF address stored in P-CSCF to a new header field of the REGISTER request. Then, the I-CSCF forwards the REGISTER request to the S-CSCF bypassing the HSS by parsing the S-CSCF address of the new header field.

Solution#3 proposes a new AS to guarantee the IMS initial registration and MT services continue for UEs registered to the IMS network in case of HSS failure or overload, which supplements Solution #1. UE registration-related data (e.g. S-CSCF address, IMSI, MSISDN, etc.) of the new AS are synchronously derived from the ISC interface (S-CSCF-AS) directly or indirectly. The new AS not only supports directly forwarding the IMS REGISTER/INVITE requests to the S-CSCF for Option A, but also supports the return of the 305 (Use Proxy) with the S-CSCF address to the I-CSCF, which then sends the IMS REGISTER/INVITE requests to the S-CSCF serving UEs for Option B. Option A and Option B don't require any modification of existing network functions. There are fewer signalling interactions for REGISTER/INVITE procedures using Option A. This increases the flexibility of new AS applications. The solution supports IMS (re-)registration and MT services from roaming or non-roaming UEs. But Solution#3 introduces a new AS which needs to work even in the normal scenario. The recovery mechanism of the new AS needs to be considered.

Solution#4 proposes to utilize the OPTIONS mechanism to find the S-CSCF serving the subscriber for bypassing HSS, requiring the S-CSCF to delay removing all subscription information related to this specific Public User Identity in the de-registration procedure. However, Solution#4 introduces a new CSCF interaction mechanism, the significant signalling processing overhead of the I/S-CSCF, the overhead of S-CSCF delay removing all subscription data, and the increased SIP session latency (especially for calls) need to be considered.

If HSS fails, the re-attach or re-registration procedures of the UE in the PS domain cannot be completed. Therefore, both Solution#3 and Solution#4 cannot support the IMS initial registration procedure for deregistered UEs in the scenario of KI#1.

## 7.2	Evaluation of Solutions for Key Issue#2

Solution #5 is the only solution addressing the Key Issue #2: Handle all PCRF/PCF instances failure scenario with minimum impact to the registered IMS users.

Solution #5 proposes a mechanism to bypass the PCRF/PCF to continue the default IMS bearer establishment, the IMS originating session and the IMS terminating session procedure when all the PCRF/PCF instances fail. It introduces the extra requirements to the PGW-U/UPF, PGW-C/SMF, P-CSCF and IMS-AGW and will increase the system overhead of these NFs. It only continues the QoS procedure when all the PCRF/PCF instances fail, but does not support other service scenarios such as Network Provided Location Information, which needs interaction with the PCRF/PCF.

## 7.3	Evaluation of Solutions for Key Issue#3

Solution #6 is a solution addressing the Key Issue #3: Speed up the restoration procedure of the terminating session when EPC/5GC NF fails.

Solution #6 proposes a check mechanism over Rx and Gx interface in the EPC network for the P-CSCF to distinguish the terminating session failure due to the P-GW abnormal and the P-CSCF restores this kind of failure through the HSS based P-CSCF Restoration procedure specified in 3GPP TS 23.380 [2]. It introduces the extra requirements to the P-GW, P-CSCF and PCRF.

## 7.4	Evaluation of Solutions for Key Issue#4

The signalling traffic to the HSS needs to be reduced when the S-CSCF or HSS identifies that the HSS is in an overload situation by indicators analysis means described in 3GPP TS 29.228 [5] Annex H and IETF RFC 7683 [8] or other internal mechanisms of S-CSCF.

Solution #2 proposes to reduce the burden on the HSS during the IMS initial registration process and the IMS third-party registration process by utilizing available AVs and user profile stored on the S-CSCF in case of HSS overload. This helps reduce the HSS load further decrease the probability of signalling storm and accelerate failure restoration for the IMS network while making minimal changes to the network function. As this would provide benefit to the operation of each network operator, introducing the capabilities for Solution #2 should be considered.

# 8	Conclusions

This clause describes conclusions.

## 8.1	Conclusion of Solutions for Key Issue#1

## 8.2	Conclusion of Solutions for Key Issue#2

Based on the evaluation in clause 7.2, it is concluded to use Solutions #5 for normative specification work.

## 8.3	Conclusion of Solutions for Key Issue#3

## 8.4	Conclusion of Solutions for Key Issue#4

According to the evaluation for KI#4, reducing the Diameter signalling interactions between S-CSCF/AS and HSS using stored available AVs should be defined and the following conclusions are agreed:

1.	Updates of the IMS registration process in the HSS overload scenario are needed, including:

-	Procedures for IMS initial registration and IMS third-party registration.

2.	Updates of network function enhancements for the IMS registration process in the HSS overload scenario are needed, including:

-	Updates of S-CSCF in 3GPP TS 23.380 [2], and collaborate with SA2 for updating 3GPP TS 23.228 [3] and 3GPP TS 24.229 [6].

It is concluded that solution #2 is selected for normative specification work.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-10 | CT4#118 | C4-234587 | - | - | - | Implementation of C4-234059, C4-234525, C4-234526, C4-234527, C4-234529, C4-234569, C4-234570, C4-234571 and C4-234577 in CT4#118 | 0.1.0 |
| 2023-11 | CT4#119 | C4-235658 |  |  |  | Implementation of C4-235574 and C4-235575 in CT4#119 | 0.2.0 |
| 2024-03 | CT4#121 | C4-240850 |  |  |  | Implementation of C4-240732 and C4-240782 in CT4#121 | 0.3.0 |
| 2024-04 | CT4#122 | C4-241404 |  |  |  | Implementation of C4-241467, C4-241401, C4-241538 and C4-241545 in CT4#122 | 0.4.0 |
| 2024-05 | CT4#123 | C4-242488 |  |  |  | Implementation of C4-242172, C4-242403, C4-242404, C4-242407, C4-242408, C4-242409, C4-242481, C4-242483 and C4-242493 in CT4#123 | 0.5.0 |
| 2024-06 | CT#104 | CP-241240 |  |  |  | Presented for information | 1.0.0 |