| 3GPP TS 31.117 V0.1.0 (2024-08) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Mobile Equipment (ME) conformance test specification; Universal Subscriber Identity Module Application Toolkit (USAT) application behavioural test specification for non-removable Universal Subscriber Identity Module (nrUSIM) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	9

Introduction	10

1	Scope	11

2	References	11

3	Definitions of terms, symbols and abbreviations	12

3.1	Terms	12

3.2	Symbols	13

3.3	Abbreviations	13

3.4	Mobile station definition and configurations	13

3.5	Coding Conventions	13

3.6	Applicability	14

3.6.1	Applicability of the present document	14

3.6.2	Applicability of the individual tests	14

3.6.3	Declaration of options specific for testing of terminals with non-removable USIM	14

3.6.4	Applicability to user equipment	15

3.6.5	Supported additional explicit verification methods	15

3.7	Table of optional features	16

3.8	Applicability table	16

4	Test environment	46

4.0	General Test purpose	46

4.1.1	General test environment	46

4.1.2	Example - test environment for contents verification	47

4.1.3	Example - test environment for seamless testing	47

4.1.4	Example – test environment for test toolkit events based testing	48

4.2	Requirements to the EUT and the test environment	49

4.2.1	General Requirements	49

4.2.2	Requirements to the UE (EUT) – supported interfaces	49

4.2.3	Supported RATs	49

4.2.4	Initial and final procedure steps	50

4.3	Suitability assessment	50

4.4	Definition of nrUICC values and System Simulator parameters for USAT testing	50

4.4.1	Introduction	50

4.4.1.1	Installation, provisioning or modification methods for EFs and DFs	50

4.4.1.2	GSMA TS.48 Version and usage	50

4.4.2	Definition of default values for USAT testing	50

4.4.2.1	Applications on the default nrUICC	50

4.4.2.2	Definition of USIM default values	50

4.4.2.3	Definition of DF_TELECOM default values	55

4.4.3	Definition of nrUICC values and System Simulator parameters for USAT testing - E-UTRAN/EPC	56

4.4.3.1	Applications on the E-UTRAN/EPC nrUICC	56

4.4.3.2	Definition of E-UTRAN/EPC USIM values	56

4.4.3.3	Definition of E-UTRAN/EPC ISIM values	56

4.4.3.4	Definition of E-UTRAN System Simulator parameters	59

4.4.4	Definition of nrUICC values and System Simulator parameters for USAT testing - NG-RAN	60

4.4.4.1	Applications of the NG-RAN nrUICC	60

4.4.4.2	Definition of NG-RAN USIM	60

4.4.4.3	Definition of NG-RAN USIM supporting Rel-17 features	61

4.4.4.4	Definition of NG-RAN USIM supporting CAG	61

4.4.4.5	Definition of NG-RAN ISIM values	62

4.4.4.6	Definition of NG-RAN System Simulator parameters	63

5	Testing methodology in general	63

5.1	Testing of optional functions and procedures	63

5.2	Test interfaces and facilities	63

5.3	Information to be provided by the apparatus supplier	63

6	Implicit testing	63

7	Measurement uncertainty	63

8	Format of tests	64

9	Generic call set up procedures	64

10	USIM Application Toolkit (USAT) testing on an ME with non-removable UICC	64

10.1	Introduction	64

10.2	General Test purpose	64

10.3	Initialization of USAT functionality on an ME with non-removable UICC	64

10.3.1	Verification of the USAT support on an ME with non-removable UICC (Profile Download)	64

10.3.1.1	Definition and applicability	64

10.3.1.2	Conformance requirement	64

10.3.1.3	Test purpose	64

10.3.1.4	Method of test	64

10.3.1.4.1	Initial conditions	64

10.3.1.4.2	Procedure	65

10.3.1.5	Test requirement	65

10.3.2	Contents of the TERMINAL PROFILE command	65

10.3.2.1	Definition and applicability	65

10.3.2.2	Conformance requirement	65

10.3.2.3	Test purpose	65

10.3.2.4	Method of test	66

10.3.2.4.1	Initial conditions	66

10.3.2.4.2	Procedure	66

10.3.2.5	Test requirement	66

10.3.2.6	Acceptance criteria	66

10.3.3	Servicing of proactive UICC commands	66

10.3.3.1	Definition and applicability	66

10.3.3.2	Conformance requirement	66

10.3.3.3	Test purpose	66

10.3.3.4	Method of test	66

10.3.3.4.1	Initial conditions	66

10.3.3.4.2	Procedure	67

10.3.3.5	Test requirement	67

10.3.3.6	Acceptance criteria	67

10.4	Proactive UICC commands	67

10.4.1	DISPLAY TEXT	67

10.4.1.1	DISPLAY TEXT (Normal)	67

10.4.1.2	DISPLAY TEXT (Support of "No response from user")	67

10.4.1.3	DISPLAY TEXT (Display of extension text)	67

10.4.1.4	DISPLAY TEXT (Sustained text)	67

10.4.1.5	DISPLAY TEXT (Display of icons)	68

10.4.1.6	DISPLAY TEXT (UCS2 display in Cyrillic)	68

10.4.1.7	DISPLAY TEXT (Variable Time out)	68

10.4.1.8	DISPLAY TEXT (Support of Text Attribute)	68

10.4.1.9	DISPLAY TEXT (UCS2 display in Chinese)	68

10.4.1.10	DISPLAY TEXT (UCS2 display in Katakana)	68

10.4.2	GET INKEY	69

10.4.2.1	GET INKEY (Normal)	69

10.4.2.2	GET INKEY (No response from User)	69

10.4.2.3	GET INKEY (UCS2 display in Cyrillic)	69

10.4.2.4	GET INKEY (UCS2 entry in Cyrillic)	69

10.4.2.5	GET INKEY ("Yes/No" Response)	69

10.4.2.6	GET INKEY (Display of icons)	69

10.4.2.7	GET INKEY (Help Information)	69

10.4.2.8	GET INKEY (Variable Time out)	70

10.4.2.9	GET INKEY (Support of Text Attribute)	70

10.4.2.10	GET INKEY (UCS2 display in Chinese)	70

10.4.2.11	GET INKEY (UCS2 entry in Chinese)	70

10.4.2.12	GET INKEY (UCS2 display in Katakana)	70

10.4.2.13	GET INKEY (UCS2 entry in Katakana)	70

10.4.3	GET INPUT	71

10.4.3.1	GET INPUT (Normal)	71

10.4.3.2	GET INPUT (No response from User)	71

10.4.3.3	GET INPUT (UCS2 display in Cyrillic)	71

10.4.3.4	GET INPUT (UCS2 entry in Cyrillic)	71

10.4.3.5	GET INPUT (Default text)	71

10.4.3.6	GET INPUT (Display of icons)	71

10.4.3.7	GET INPUT (Help Information)	71

10.4.3.8	GET INPUT (Support of Text Attribute)	72

10.4.3.9	GET INPUT (UCS2 display in Chinese)	72

10.4.3.11	GET INPUT (UCS2 display in Katakana)	72

10.4.3.12	GET INPUT (UCS2 entry in Katakana)	72

10.4.4	MORE TIME	72

10.4.5	PLAY TONE	73

10.4.5.2	PLAY TONE (UCS2 display in Cyrillic)	73

10.4.5.3	PLAY TONE (Display of icons)	73

10.4.5.4	PLAY TONE (Support of Text Attribute)	73

10.4.5.5	PLAY TONE (UCS2 display in Chinese)	73

10.4.5.6	PLAY TONE (UCS2 display in Katakana)	73

10.4.6	POLL INTERVAL	73

10.4.7	REFRESH	73

10.4.7.1	REFRESH (Normal)	73

10.4.7.2	REFRESH (IMSI changing procedure)	74

10.4.7.3	REFRESH (Steering of roaming)	74

10.4.7.4	REFRESH (AID)	74

10.4.7.5	REFRESH (IMSI changing procedure, E-UTRAN)	74

10.4.7.6	REFRESH (IMSI changing procedure, NG-RAN)	74

10.4.7.7	REFRESH (SUPI_NAI changing procedure, NG-RAN)	74

10.4.8	SET UP MENU and ENVELOPE MENU SELECTION	75

10.4.8.1	SET UP MENU (Normal) and ENVELOPE MENU SELECTION	75

10.4.8.2	SET UP MENU (Help request support) and ENVELOPE MENU SELECTION	75

10.4.8.3	SET UP MENU (Help request support) and ENVELOPE MENU SELECTION	75

10.4.8.4	SET UP MENU (Display of icons) and ENVELOPE MENU SELECTION	75

10.4.8.5	SET UP MENU (Soft Keys support) and ENVELOPE MENU SELECTION	75

10.4.8.6	SET UP MENU (Support of Text Attribute) and ENVELOPE MENU SELECTION	75

10.4.8.7	SET UP MENU (UCS2 display in Cyrillic) and ENVELOPE MENU SELECTION	76

10.4.8.8	SET UP MENU (UCS2 display in Chinese) and ENVELOPE MENU SELECTION	76

10.4.8.9	SET UP MENU (UCS2 display in Katakana) and ENVELOPE MENU SELECTION	76

10.4.9	SELECT ITEM	76

10.4.9.1	SELECT ITEM (Mandatory features for ME supporting SELECT ITEM)	76

10.4.9.2	SELECT ITEM (Next action support)	76

10.4.9.3	SELECT ITEM (Default item support)	76

10.4.9.4	SELECT ITEM ((Help request support)	76

10.4.9.5	SELECT ITEM (Icons support)	77

10.4.9.6	SELECT ITEM (Presentation style)	77

10.4.9.7	SELECT ITEM (Soft keys support)	77

10.4.9.8	SELECT ITEM (Support of "No response from user")	77

10.4.9.9	SELECT ITEM (Support of Text Attribute)	77

10.4.9.10	SELECT ITEM (UCS2 display in Cyrillic)	77

10.4.9.11	SELECT ITEM (UCS2 display in Chinese)	77

10.4.9.12	SELECT ITEM (UCS2 display in Katakana)	78

10.4.10	SEND SHORT MESSAGE	78

10.4.10.1	SEND SHORT MESSAGE (Normal)	78

10.4.10.2	SEND SHORT MESSAGE (UCS2 display in Cyrillic)	78

10.4.10.3	SEND SHORT MESSAGE (Icon support)	78

10.4.10.4	SEND SHORT MESSAGE (Support of Text Attribute)	78

10.4.10.5	SEND SHORT MESSAGE (UCS2 display in Chinese)	78

10.4.10.6	SEND SHORT MESSAGE (UCS2 display in Katakana)	78

10.4.10.7	SEND SHORT MESSAGE (IMS)	78

10.4.10.8	SEND SHORT MESSAGE (Over SGs in E-UTRAN)	78

10.4.11	SEND SS	78

10.4.12	SEND USSD	79

10.4.13	SET UP CALL	79

10.4.14	POLLING OFF	79

10.4.15	PROVIDE LOCAL INFORMATION	79

10.4.16	SET UP EVENT LIST	79

10.4.17	PERFORM CARD APDU	79

10.4.17.1	PERFORM CARD APDU (Normal)	79

10.4.17.2	PERFORM CARD APDU (Detachable card reader)	79

10.4.18	POWER OFF CARD	80

10.4.18.1	POWER OFF CARD (Normal)	80

10.4.18.2	POWER OFF CARD (Detachable card reader)	80

10.4.19	POWER ON CARD	80

10.4.19.1	POWER ON CARD (Normal)	80

10.4.19.2	POWER ON CARD (Detachable card reader)	80

10.4.20	GET READER STATUS	80

10.4.20.1	GET READER STATUS (Normal)	80

10.4.20.2	GET READER STATUS (Detachable card reader)	80

10.4.21	TIMER MANAGEMENT and ENVELOPE TIMER EXPIRATION	81

10.4.21.1	TIMER MANAGEMENT (Normal)	81

10.4.21.2	ENVELOPE TIMER EXPIRATION (Normal)	81

10.4.22	SET UP IDLE MODE TEXT	81

10.4.23	RUN AT COMMAND	81

10.4.23.1	RUN AT COMMAND (Normal)	81

10.4.23.2	RUN AT COMMAND (Icon support)	81

10.4.23.3	RUN AT COMMAND (Support of Text Attribute)	81

10.4.23.4	RUN AT COMMAND (UCS2 display in Cyrillic)	81

10.4.23.5	RUN AT COMMAND (UCS2 display in Chinese)	82

10.4.23.6	RUN AT COMMAND (UCS2 display in Katakana	82

10.4.24	SEND DTMF	82

10.4.25	LANGUAGE NOTIFICATION	82

10.4.26	LAUNCH BROWSER	82

10.4.26.1	LAUNCH BROWSER (No session already launched)	82

10.4.26.2	LAUNCH BROWSER (Interaction with current session)	82

10.4.26.3	LAUNCH BROWSER (UCS2 display in Cyrillic)	82

10.4.26.4	LAUNCH BROWSER (Icon Support)	82

10.4.26.5	LAUNCH BROWSER (Support of Text Attribute)	82

10.4.26.6	LAUNCH BROWSER (UCS2 Display in Chinese)	82

10.4.26.7	LAUNCH BROWSER (UCS2 Display in Katakana)	82

10.4.26.8	LAUNCH BROWSER (NG-RAN bearer)	83

10.4.27	OPEN CHANNEL	83

10.4.27.1	Void	83

10.4.26.2	OPEN CHANNEL (Related to GPRS)	83

10.4.26.3	OPEN CHANNEL (Default Bearer)	83

10.4.26.4	OPEN CHANNEL (Local Bearer)	83

10.4.26.5	OPEN CHANNEL (GPRS, Support of Text Attribute)	83

10.4.27.6	OPEN CHANNEL (Related to E-UTRAN)	83

10.4.27.7	OPEN CHANNEL (UICC Access to IMS)	83

10.4.27.8	OPEN CHANNEL (related to NG-RAN)	83

10.4.28	CLOSE CHANNEL	84

10.4.28.1	CLOSE CHANNEL (Normal)	84

10.4.28.2	CLOSE CHANNEL (support of Text Attribute)	84

10.4.28.3	CLOSE CHANNEL (E-UTRAN/EPC)	84

10.4.28.4	CLOSE CHANNEL (NG-RAN)	84

10.4.29	RECEIVE DATA	84

10.4.29.1	RECEIVE DATA (Normal)	84

10.4.29.2	RECEIVE DATA (Support of Text Attribute)	84

10.4.30	SEND DATA	84

10.4.30.1	SEND DATA (Normal)	84

10.4.30.2	SEND DATA (Support of Text Attribute)	85

10.4.30.3	SEND DATA(E-UTRAN)	85

10.4.30.4	SEND DATA(NG-RAN)	85

10.4.31	GET CHANNEL STATUS	85

10.5	Data Download to UICC	85

10.5.1	SMS-PP Data Download	85

10.5.2	Cell Broadcast Data Download	85

10.5.3	SMS-PP Data Download over IMS	85

10.5.4	SMS-PP Data Download over SGs in E-UTRAN	85

10.6	CALL CONTROL BY USIM	86

10.6.1	Procedure for Mobile Originated calls	86

10.6.2	Procedure for Supplementary (SS) Services	86

10.6.3	Interaction with Fixed Dialling Number (FDN)	86

10.6.4	Support of Barred Dialling Number (BDN) service	86

10.6.5	Barred Dialling Number (BDN) service handling for terminals not supporting BDN	86

10.7	EVENT DOWNLOAD	86

10.7.1	MT Call Event	86

10.7.2	Call Connected Event	86

10.7.3	Call Disconnected Event	86

10.7.4	Location Status Event	86

10.7.4.1	Location Status Event (Normal)	86

10.7.5	User Activity Event	86

10.7.5.1	User Activity Event (Normal)	86

10.7.6	Idle Screen Available Event	87

10.7.7	Card Reader Status Event	87

10.7.7.1	Card Reader Status (Normal)	87

10.7.7.2	Card Reader Status (Detachable card reader)	87

10.7.8	Language Selection Event	87

10.7.8.1	Language Selection Event (Normal)	87

10.7.9	Browser Termination Event	87

10.7.10	Data Available Event	87

10.7.10.1	Data Available Event (Normal)	87

10.7.11	Channel Status event	87

10.7.12	Access Technology Change event	88

10.7.13	Display parameter changed event	88

10.7.14	Local Connection event	88

10.7.15	Network search mode change event	88

10.7.16	Browsing status event	88

10.7.17	Network Rejection event	88

10.7.18	CSG Cell Selection event	88

10.7.19	IMS registration event	88

10.7.20	Incoming IMS data event	89

10.7.20.1	Incoming IMS data (Normal)	89

10.7.21	Data Connection Status Change event	89

10.7.22	CAG Cell Selection event	89

10.7.22.1	CAG Cell Selection (Normal)	89

10.8	MO SHORT MESSAGE CONTROL BY USIM	89

10.9	Handling of command number	89

10.10	CALL CONTROL on EPS PDN Connection	89

10.11	Call Control on PDP Context Activation	90

10.11.1	Procedure for Mobile Originated calls	90

10.12	Change eCall mode	90

10.13	CALL CONTROL on PDU Session Establishment for NG-RAN	90

10.13.1	Procedure for Mobile Originated calls	90

10.14	ENVELOPE SMS-PP Data Download on NAS messages	90

10.14.1	Routing Indicator Data update via DL NAS TRANSPORT messages	90

10.14.2	Steering of Roaming via DL NAS TRANSPORT message	90

10.14.3	Steering of Roaming via REGISTRATION ACCEPT message	91

Annex A (informative):	Examples of Test-nrUICC	92

A.0	General information	92

A.1	Test EF structure - 1	92

A.1.1	EFSTK (STK)	92

A.1.2	EFSETSTK (SETSTK)	93

A.1.3	EFCC (CALL CONTROL)	93

A.1.4	EFEVENTLIST (EVENT LIST)	93

A.2	Test EF structure - 2	94

A.2.1	EFTC_IN (TC_IN)	94

A.2.2	EFTC_OUT (TC_OUT)	95

A.2.3	EFEF_RESET (EF_RESET)	95

Annex B (normative):	Details of terminal profile support	96

Annex C (informative):	Change history	97

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document defines application behavioural tests for USIM Application Toolkit functionality on an ME with a non-removable UICC/USIM where no access to the physical UICC-Terminal interface can be granted when interacting with a 3GPP network.

The aim of the present document is to ensure the correct behaviour of a UE interfacing with a 3GPP network whilst it is proved that USIM Application Toolkit specific data and functionality is used wherever indicated, similar to application tests defined in TS 31.124 [2] but without direct access to the UICC-Terminal interface.

The present document does not define any aspects related to the administrative management phase of the UICC. Any internal technical realisation of either the UICC or the terminal is only specified where these are important for the verification of specific behaviour. Application specific details for applications residing on an UICC are specified in the respective application specific documents.

# 1	Scope

The present document describes behavioural tests for the USIM Application Toolkit implemented in Mobile Equipment (ME) or Mobile Stations (MS) supporting a non-removable UICC only within the 3GPP digital cellular telecommunications system, in compliance with the relevant requirements, and in accordance with the relevant guidance given in ISO/IEC 9646-7 [19] and ETSI ETS 300 406 [20].

This document shall provide alternative verification and testing approaches for test cases initially defined in TS 31.124 [2] or ETSI TS 102 384 respectively. To guarantee the highest possible reliability, the verification of existing requirements shall be done using methods defined by ETSI or 3GPP.

A 3GPP ME may support functionality that is not required by 3GPP, but the requirements to do so are outside of the scope of 3GPP. Thus, the present document does not contain tests or references to ETSI TS 102 384 [3] tests for features which are out of scope of 3GPP.

Due to possible limitations in profile handling and updating it is not intended to use the test cases defined within this document with MEs implemented in accordance with Rel-12 or earlier.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 31.124: " Mobile Equipment (ME) conformance test specification; Universal Subscriber Identity Module Application Toolkit (USAT) conformance test specification".

[3]	ETSI TS 102 384: "Smart cards; UICC-Terminal interface; Card Application Toolkit (CAT) conformance specification".

[4]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[5]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing".

[6]	3GPP TS 34.108: "Common test environments for User Equipment (UE) conformance testing".

[7]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[8]	ISO/IEC 9646-7: "Information technology - Open Systems Interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[9]	ETSI TS 102 221: "UICC-Terminal interface; Physical and logical characteristics".

[10]	GSMA TS.48: "Generic eUICC Test Profile for Device Testing".

[11]	ETSI TS 103 666-1: "Smart Secure Platform (SSP); Part 1: General characteristics".

[12]	ETSI TS 103 666-2: "Smart Secure Platform (SSP); Part 2: Integrated SSP (iSSP) characteristics".

[13]	ETSI TS 103 666-3: "Smart Secure Platform (SSP); Part 3: Embedded SSP (eSSP) Type 1 characteristics".

[14]	802.11-2016: "IEEE Standard for Information technology—Telecommunications and information exchange between systems Local and metropolitan area networks—Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[15]	ETSI TS 102 225: "Secured packet structure for UICC based applications".

[16]	3GPP TS 27.007: "AT command set for User Equipment (UE) ".

[17]	3GPP TS 31.130: "(U)SIM Application Programming Interface (API); (U)SIM API for Java™Card".

[18]	3GPP TS 22.011: "Service accessibility".

[19]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[20]	3GPP TS 21.111: "USIM and IC card requirements".

[21]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[22]	Trusted Connectivity Alliance: "eUICC Profile Package: Interoperable Format Technical Specification".

[23]	ETSI TS 102 241: "UICC Application Programming Interface (UICC API) for Java Card™".

[24]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[25]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[26]	3GPP TS 31.111: "Universal Subscriber Identity Module (USIM) Application Toolkit (USAT)"

[27]	ETSI TS 102 223: "Smart Cards; Card Application Toolkit (CAT)"

[28]	ETSI TS 102 671: "Machine to Machine UICC; Physical and logical characteristics"

[29]	GSMA SGP.22: "RSP Technical specification"

[30]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3"

[31]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol specification"

[32]	3GPP TS 31.101: "UICC-terminal interface; Physical and logical characteristics"

[33]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing".

[34]	ISO/IEC 10646-1: "Information technology - Universal Multiple Octet Coded Character Set (UCS) – Part 1: Architecture and Basic Multilingual Plane".

[35]	ISO/IEC 10646-2: "Information technology - Universal Multiple Octet Coded Character Set (UCS) – Part 2: Supplementary Planes".

[36]	3GPP TS 22.001: "Principles of circuit telecommunication services supported by a Public Land Mobile Network (PLMN)"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], TS 31.124 [2] and the following apply:

E-UTRAN:	term used for E-UTRAN in WB-S1 mode.

NAA:	term used in ETSI TS 102 384 [3] refers to the USIM application.

NB-IoT:	term used for E-UTRAN in NB-S1 mode.

nrUICC:	non-removable Universal Integrated Circuit Card. The non-removable card hosting the nrUSIM application embedded or integrated into a ME.

nrUSIM:	non-removable Universal Subscriber Identity Module, i.e. a USIM application or equivalent functionality embedded or integrated into a ME.

Terminal:	term used in ETSI TS 102 384 [3], refers to the Mobile Equipment (ME).

TT:	Test Tool: collective term for requirements fulfilling but not clearly specified test environment.

UICC:	term used in ETSI TS 102 384 [3] refers to the USIM card.

A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

bx		Bit x of byte (leftmost bit is MSB)

Bn		Byte No. n

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] [1], TS 31.124 [2] and the following apply:

CR	Conformance Requirement

EUT	Equipment Under Test

SA	Suitability Assessment

TT	Test Tool

USAT	USIM Application Toolkit

An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

## 3.4	Mobile station definition and configurations

The mobile station definition and configurations specified in TS 34.108 [6] and TS 36.508 [35] shall apply, unless otherwise specified in the present clause.

## 3.5	Coding Conventions

For the purposes of the present document, the following coding conventions apply:

All lengths are presented in bytes, unless otherwise stated. Each byte B is represented by eight bits b8 to b1, where b8 is the most significant bit (MSB) and b1 is the least significant bit (LSB). In each representation, the leftmost bit is the MSB.

In the UICC, all bytes specified as RFU shall be set to '00' and all bits specifies as RFU shall be set to '0'. If the  USIM application exists on a UICC or is built on a generic telecommunications card, then other values may apply for the non-USIM applications. The values will be defined in the appropriate specifications for such cards and applications. These bytes and bits shall not be interpreted by a ME in a 3GPP session.

The coding of all data objects in the present document is according to ETSI TS 102 221 [9]. All data objects are BER-TLV except if otherwise defined.

## 3.6	Applicability

### 3.6.1	Applicability of the present document

The present document applies to user equipment that supports the USIM Application Toolkit optional feature where the UICC-terminal interface is not accessible and communication on the UICC-terminal interface cannot be traced.

### 3.6.2	Applicability of the individual tests

Table B.1 lists the optional, conditional, or mandatory features for which the supplier of the implementation states the support. As pre-condition the supplier of the implementation shall state the support of possible options in accordance with table A.1 of TS 31.124 [2], clause 3.3. ME default configuration in accordance with table A.2 of TS 31.124 [2], clause 5.2 and declare what testing options are supported in table A.3 of the present document.

The "Release XY ME" columns shows the status of the entries as follows:

The following notations, defined in ISO/IEC 9646-7 [7], are used for the status column:

M	mandatory – the capability is required to be supported.

O	optional – the capability may be supported or not.

N/A	not applicable – in the given context, it is impossible to use the capability.

X	prohibited (excluded) – there is a requirement not to use this capability in the given context.

O.i	qualified optional – for mutually exclusive or selectable options from a set. "i" is an integer which identifies a unique group of related optional items and the logic of their selection which is defined immediately following the table.

Ci	conditional – the requirement on the capability ("M", "O", "X" or "N/A") depends on the support of other optional or conditional items. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF … THEN (IF … THEN … ELSE…) ELSE …" shall be used to avoid ambiguities.

The "Additional test case execution recommendation" column shows the status of the entries as follows:

A	applicable - the test is applicable according to the corresponding entry in the "Rxx ME" column

R	redundant – the test has to be considered as redundant when the corresponding E-UTRAN/EPC related test of the present document has been validated and successfully executed. In that case the requirement may be verified by means of the E-UTRAN/EPC functionality only.

AERi	Additional test case Execution Recommendation – with respect to the above listed definitions of ("A") and ("R") the test is applicable ("A") or redundant ("R") depending on the support of other optional or conditional items. "i" is an integer identifying a unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." shall be used to avoid ambiguities.

References to items

For each possible item answer (answer in the support column) there exists a unique reference, used, for example, in the conditional expressions. It is defined as the table identifier, followed by a solidus character "/", followed by the item number in the table. If there is more than one support column in a table, the columns shall be discriminated by letters (a, b, etc.), respectively.

EXAMPLE:	A.1/4 is the reference to the answer of item 4 in table A.1.

### 3.6.3	Declaration of options specific for testing of terminals with non-removable USIM

To identify a suitable test environment the support of some specific features is required. The supplier of the implementation shall state the support of possible options in table A.3.

Table A.3: Declaration of supported testing options

| Item | Option | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- |
| 1 | Support of UTRAN access | O |  | O_UTRAN |
| 2 | Support of E-UTRAN access | O |  | O_E-UTRAN |
| 3 | Support of NB-IoT access only | O |  | O_NB-IoT |
| 4 | Support of 5G Core Network | O |  | O_5GC |
| 5 | Support of New Radio access | O |  | O_5G_NR |
| 6 | Support of RSP(SGP.22) | C003 |  | O_RSP22 |
| 7 | Support of AT+CSIM | O |  | O_AT+CSIM |
| 8 | ME supports non-removable UICC only (see NOTE 1) | O |  | O_NON-REMOVABLE_UICC_ONLY |
| 9 | Support of UICC and USIM API for Java Card (see NOTE 2) | O |  | O_JAVA_CARD_API |
| 10 | Support of USAT functionality (see NOTE 3) | O |  | O_USAT |
| NOTE1: | ‘ME supports non-removable UICC only’ means that access to the physical card interface as defined in ETSI TS 102 221 is not available |  |  |  |
| NOTE 2: | The UE shall claim to support the Java Card API if test relevant functions as defined in the examples in Annex A, clauses A.1 and A.2 are supported. |  |  |  |
| NOTE 3: | The support of the USAT as defined here requires the support of the UICC API defined in ETSI TS 102 241 [23] and the USIM API defined in TS 31.130 [17] |  |  |  |

### 3.6.4	Applicability to user equipment

The applicability to user equipment supporting the non-removable USIM is specified in table B.1, regardless of references to complete tests, test purposes, conformance requirements or test methods from TS 31.124 [2] the applicability of the individual test cases is defined within the present document.

To execute tests defined in the present document, methods to trigger proactive commands from the non-removable USIM are required. The verification of conformance requirements can be done by using and (explicit verification methods only. The support of explicit verification methods by the EUT (UE) has to be declared in accordance with table A.4 (see clause 3.6.5). Test sequence specific declarations of methods required to be supported are listed in the Applicability table – Table B.1.

If tests require verification of proactive commands, the TT should be enabled to trigger required proactive commands using a test toolkit applet loaded on to the non-removable UICC.

Network dependent test cases on UEs supporting NB-IoT shall be verified by accessing the NB System Simulator (NB-SS).

### 3.6.5	Supported additional explicit verification methods

To execute tests defined in the present document, methods to trigger proactive commands from the non-removable USIM are required. The verification of conformance requirements can be done by using and (explicit verification methods only. The support of explicit verification methods by the EUT (UE) has to be declared in accordance with table A.4 (see clause 3.6.5). Test sequence specific declarations of methods required to be supported are listed in the Applicability table – Table B.1.

If tests require verification of proactive commands, TT shall be able to trigger required proactive commands using a test toolkit applet loaded on to the non-removable UICC.

Network dependent test cases on UEs supporting NB-IoT shall be verified by accessing the NB System Simulator (NB-SS).

Table A.4: Test Options Declaration

| Item | Option | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- |
| 1 | Support of Toolkit Test Events (see note) | O |  | O_Toolkit_Test_Events |
| 2 | Support of seamless test APDU logging via Baseband (see note) | O |  | O_Seamless_APDU_Logging |
| 3 | Interface for file contents verification | O |  | O_File_Contents_Verification |
| NOTE: The support of the SSP Test Tool Interface (ETSI TS 103 834) is handled like the support of seamless test APDU logging via Baseband or for Test Toolkit Event based testing. Specific information may be added to test cases where needed. |  |  |  |  |

For details on these options see clauses 4.1.3, to 4.1.6 of the present document.

## 3.7	Table of optional features

Support of several features is optional or release dependent for the user equipment. However, if a UE states conformance with a specific 3GPP release, it is mandatory for the UE to support all mandatory functions of that release, as stated in table A.1.

The supplier of the implementation shall state the support of possible options in table A.1 as defined in TS 31.124 [2].

## 3.8	Applicability table

Table B.1: Applicability of tests

| Test# | Seq. | Title | from Rel. | to Rel. (see note) | Appl. | Terminal Profile | NW Dep. | Exec. parameter | Support |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10.3.1 | PROFILE DOWNLOAD |  |  |  |  |  |  |  |  |
|  | 1 | PROFILE DOWNLOAD | Rel-13 |  | M | E.1/1 |  |  |  |
| 10.3.2 | Contents of the TERMINAL PROFILE command |  |  |  |  |  |  |  |  |
|  | 1 | Contents of the TERMINAL PROFILE command | Rel-13 |  | M | E.1/1 |  |  |  |
| 10.3.3 | Servicing of Proactive UICC Commands |  |  |  |  |  |  |  |  |
|  | 1 | Servicing of Proactive UICC Commands | Rel-13 |  | M |  |  |  |  |
| 10.4 | Proactive UICC commands |  |  |  |  |  |  |  |  |
| 10.4.1 | DISPLAY TEXT |  |  |  |  |  |  |  |  |
| 10.4.1.1 | DISPLAY TEXT (Normal) |  |  |  |  |  |  |  |  |
|  | 1.1 | Unpacked | Rel-13 |  | C177 | E.1/17 AND E.1/110 |  |  |  |
|  | 1.2 | Screen busy | Rel-13 |  | C177 | E.1/17 AND E.1/110 |  |  |  |
|  | 1.3 | High priority | Rel-13 |  | C177 | E.1/17 AND E.1/110 |  |  |  |
|  | 1.4 | Packed | Rel-13 |  | C177 | E.1/17 AND E.1/110 |  |  |  |
|  | 1.5 | Clear after delay | Rel-13 |  | C177 | E.1/17 AND E.1/110 |  |  |  |
|  | 1.6 | Long text up to 160 bytes | Rel-13 |  | C177 | E.1/17 AND E.1/110 |  |  |  |
|  | 1.7 | Backwards move in USIM session | Rel-13 |  | C177 AND C178 | E.1/17 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.8 | Session terminated by user | Rel-13 |  | C177 AND C178 | E.1/17 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.9 | Command not understood by ME | Rel-13 |  | C177 | E.1/17 AND E.1/110 |  |  |  |
| 10.4.1.2 | DISPLAY TEXT (Support of "No response from user") |  |  |  |  |  |  |  |  |
|  | 2.1 | No response from user | Rel-13 |  | C120 AND C177 AND C178 | E.1/17 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.1.3 | DISPLAY TEXT (Display of extension text) |  |  |  |  |  |  |  |  |
|  | 3.1 | Extension Text | Rel-13 |  | C177 | E.1/17 AND E.1/16 AND E.1/110 |  |  |  |
| 10.4.1.4 | DISPLAY TEXT (Sustained text) |  |  |  |  |  |  |  |  |
|  | 4.1 | Sustained text; unpacked data 8 bits | Rel-13 |  | C177 | E.1/17 AND E.1/65 AND E.1/110 |  |  |  |
|  | 4.2 | Sustained text; clear message after delay | Rel-13 |  | C177 | E.1/17 AND E.1/65 AND E.1/110 |  |  |  |
|  | 4.3 | Sustained text; wait for user MMI to clear | Rel-13 |  | C177 AND C178 | E.1/17 AND E.1/65 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.1.5 | DISPLAY TEXT (Display of icons) |  |  |  |  |  |  |  |  |
|  | 5.1 | Display of basic icon; self-explanatory | Rel-13 |  | C108 AND C177 | E.1/17 AND E.1/110 |  |  |  |
|  | 5.2 | Display of colour icon | Rel-13 |  | C171 AND C177 | E.1/17 AND E.1/110 |  |  |  |
|  | 5.3 | Display of basic icon - not self-explanatory | Rel-13 |  | C108 AND C177 | E.1/17 AND E.1/110 |  |  |  |
| 10.4.1.6 | DISPLAY TEXT (UCS2 display in Cyrillic) |  |  |  |  |  |  |  |  |
|  | 6.1 | UCS2 display in Cyrillic | Rel-13 |  | C118 AND C177 | E.1/17 AND E.1/15 AND E.1/110 |  |  |  |
| 10.4.1.7 | DISPLAY TEXT (Variable Time out) |  |  |  |  |  |  |  |  |
|  | 7.1 | Variable Timeout | Rel-13 |  | C126 AND C177 AND C178 | E.1/17 AND E.1/137 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.1.8 | DISPLAY TEXT (Support of Text Attribute) |  |  |  |  |  |  |  |  |
|  | 8.1 | Text attribute – left alignment | Rel-13 |  | C153 AND C177 | E.1/17 AND E.1/124 AND E.1/217 AND E.1/110 |  |  |  |
|  | 8.2 | Text attribute – center alignment | Rel-13 |  | C154 AND C177 | E.1/17 AND E.1/124 AND E.1/218 AND E.1/110 |  |  |  |
|  | 8.3 | Text attribute – right alignment | Rel-13 |  | C155 AND C177 | E.1/17 AND E.1/124 AND E.1/219 AND E.1/110 |  |  |  |
|  | 8.4 | Text attribute – large font size | Rel-13 |  | C157 AND C156 AND C177 | E.1/17 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 |  |  |  |
|  | 8.5 | Text attribute – small font size | Rel-13 |  | C158 AND C156 AND C177 | E.1/17 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 |  |  |  |
|  | 8.6 | Text attribute – bold on | Rel-13 |  | C160 AND C159 AND C177 | E.1/17 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 |  |  |  |
|  | 8.7 | Text attribute – italic on | Rel-13 |  | C161 AND C159 AND C177 | E.1/17 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 |  |  |  |
|  | 8.8 | Text attribute – underlined on | Rel-13 |  | C162 AND C159 AND C177 | E.1/17 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 |  |  |  |
|  | 8.9 | Text attribute – strikethrough on | Rel-13 |  | C163 AND C159 AND C177 | E.1/17 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 |  |  |  |
|  | 8.10 | Text attribute – foreground and background colours | Rel-13 |  | C164 AND C165 AND C177 | E.1/17 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 |  |  |  |
| 10.4.1.9 | DISPLAY TEXT (UCS2 display in Chinese) |  |  |  |  |  |  |  |  |
|  | 9.1 | UCS2 display in Chinese | Rel-13 |  | C143 AND C177 | E.1/17 AND E.1/15 AND E.1/110 |  |  |  |
| 10.4.1.10 | DISPLAY TEXT (UCS2 display in Katakana) |  |  |  |  |  |  |  |  |
|  | 10.1 | UCS2 display in Katakana | Rel-13 |  | C145 AND C177 | E.1/17 AND E.1/15 AND E.1/110 |  |  |  |
| 10.4.2 | GET INKEY |  |  |  |  |  |  |  |  |
| 10.4.2.1 | GET INKEY (Normal) |  |  |  |  |  |  |  |  |
|  | 1.1 | Prompt unpacked | Rel-13 |  | C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.2 | Prompt packed | Rel-13 |  | C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.3 | Backwards move in UICC session | Rel-13 |  | C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.4 | Session terminated by user | Rel-13 |  | C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.5 | SMS alphabet | Rel-13 |  | C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.6 | Long text up to 160 bytes | Rel-13 |  | C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.2.2 | GET INKEY (No response from User) |  |  |  |  |  |  |  |  |
|  | 2.1 | No response from user | Rel-13 |  | C120 AND C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.2.3 | GET INKEY (UCS2 display in Cyrillic) |  |  |  |  |  |  |  |  |
|  | 3.1 | UCS2 display in Cyrillic | Rel-13 |  | C118 AND C177 AND C178 | E.1/18 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
|  | 3.2 | UCS2 display; Long text up to 70 chars in Cyrillic | Rel-13 |  | C118 AND C177 AND C178 | E.1/18 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.2.4 | GET INKEY (UCS2 entry in Cyrillic) |  |  |  |  |  |  |  |  |
|  | 4.1 | UCS2 entry in Cyrillic | Rel-13 |  | C105 AND C177 AND C178 | E.1/18 AND E.1/14 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.2.5 | GET INKEY ("Yes/No" Response) |  |  |  |  |  |  |  |  |
|  | 5.1 | "Yes/No" response | Rel-13 |  | C177 AND C178 | E.1/18 AND E.1/60 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.2.6 | GET INKEY (Display of icons) |  |  |  |  |  |  |  |  |
|  | 6.1 | Basic icon - self-explanatory | Rel-13 |  | C108 AND C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.2 | Basic icon - non self-explanatory | Rel-13 |  | C108 AND C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.3 | Colour icon - self-explanatory | Rel-13 |  | C171 AND C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.4 | Colour icon - non self-explanatory | Rel-13 |  | C171 AND C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.2.7 | GET INKEY (Help Information) |  |  |  |  |  |  |  |  |
|  | 7.1 | Help information | Rel-13 |  | C107 AND C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.2.8 | GET INKEY (Variable Time out) |  |  |  |  |  |  |  |  |
|  | 8.1 | Variable Timeout | Rel-13 |  | C126 AND C177 AND C178 | E.1/18 AND E.1/140 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.2.9 | GET INKEY (Support of Text Attribute) |  |  |  |  |  |  |  |  |
|  | 9.1 | Text attribute – left alignment | Rel-13 |  | C153 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/217 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.2 | Text attribute – center alignment | Rel-13 |  | C154 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/218 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.3 | Text attribute – right alignment | Rel-13 |  | C155 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/219 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.4 | Text attribute – large font size | Rel-13 |  | C157 AND C156 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.5 | Text attribute – small font size | Rel-13 |  | C158 AND C156 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.6 | Text attribute – bold on | Rel-13 |  | C160 AND C159 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.7 | Text attribute – italic on | Rel-13 |  | C161 AND C159 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.8 | Text attribute – underlined on | Rel-13 |  | C162 AND C159 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.9 | Text attribute – strikethrough on | Rel-13 |  | C163 AND C159 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.10 | Text attribute – foreground and background colours | Rel-13 |  | C164 AND C165 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.2.10 | GET INKEY (UCS2 display in Chinese) |  |  |  |  |  |  |  |  |
|  | 10.1 | UCS2 display in Chinese | Rel-13 |  | C143 AND C177 AND C178 | E.1/18 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
|  | 10.2 | UCS2 display in Chinese; Long text up to 70 chars | Rel-13 |  | C143 AND C177 AND C178 | E.1/18 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.2.11 | GET INKEY (UCS2 entry in Chinese) |  |  |  |  |  |  |  |  |
|  | 11.1 | UCS2 entry in Chinese | Rel-13 |  | C142 AND C177 AND C178 | E.1/18 AND E.1/14 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.2.12 | GET INKEY (UCS2 display in Katakana) |  |  |  |  |  |  |  |  |
|  | 12.1 | UCS2 display in Katakana | Rel-13 |  | C145 AND C177 AND C178 | E.1/18 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
|  | 12.2 | UCS2 display in Katakana; Long text up to 70 chars | Rel-13 |  | C145 AND C177 AND C178 | E.1/18 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.2.13 | GET INKEY (UCS2 entry in Katakana) |  |  |  |  |  |  |  |  |
|  | 13.1 | UCS2 entry in Katakana | Rel-13 |  | C144 AND C177 AND C178 | E.1/18 AND E.1/14 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.3 | GET INPUT |  |  |  |  |  |  |  |  |
| 10.4.3.1 | GET INPUT (Normal) |  |  |  |  |  |  |  |  |
|  | 1.1 | Digits only | Rel-13 |  | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.2 | Input packed | Rel-13 |  | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.3 | SMS alphabet | Rel-13 |  | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.4 | Hidden input | Rel-13 |  | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.5 | Min / max acceptable length | Rel-13 |  | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.6 | Backwards move in UICC session | Rel-13 |  | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.7 | Session terminated by user | Rel-13 |  | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.8 | Prompt text up to 160 bytes | Rel-13 |  | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.9 | SMS default alphabet; ME to echo text; packing not required | Rel-13 |  | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.10 | Null length for the text string | Rel-13 |  | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.3.2 | GET INPUT (No response from User) |  |  |  |  |  |  |  |  |
|  | 2.1 | No response from user | Rel-13 |  | C120 AND C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.3.3 | GET INPUT (UCS2 display in Cyrillic) |  |  |  |  |  |  |  |  |
|  | 3.1 | Text sting coding in UCS2 in Cyrillic | Rel-13 |  | C118 AND C177 AND C178 | E.1/19 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
|  | 3.2 | Max length for the text string coding in UCS2 in Cyrillic | Rel-13 |  | C118 AND C177 AND C178 | E.1/19 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.3.4 | GET INPUT (UCS2 entry in Cyrillic) |  |  |  |  |  |  |  |  |
|  | 4.1 | Character set from UCS2 alphabet in Cyrillic | Rel-13 |  | C105 AND C177 AND C178 | E.1/19 AND E.1/14 AND E.1/110 AND E.1/111 |  |  |  |
|  | 4.2 | Character set from UCS2 alphabet in Cyrillic; max length | Rel-13 |  | C105 AND C177 AND C178 | E.1/19 AND E.1/14 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.3.5 | GET INPUT (Default text) |  |  |  |  |  |  |  |  |
|  | 5.1 | Default text for the input | Rel-13 |  | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
|  | 5.2 | Default text for the input; max length | Rel-13 |  | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.3.6 | GET INPUT (Display of icons) |  |  |  |  |  |  |  |  |
|  | 6.1 | Basic icon; self-explanatory | Rel-13 |  | C108 AND C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.2 | Basic icon; non self-explanatory | Rel-13 |  | C108 AND C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.3 | Colour icon; self-explanatory | Rel-13 |  | C171 AND C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.4 | Colour icon; non self-explanatory | Rel-13 |  | C171 AND C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.3.7 | GET INPUT (Help Information) |  |  |  |  |  |  |  |  |
|  | 7.1 | 8 bit data Message; help information available | Rel-13 |  | C107 AND C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.3.8 | GET INPUT (Support of Text Attribute) |  |  |  |  |  |  |  |  |
|  | 8.1 | Text attribute– left alignment | Rel-13 |  | C153 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/217 AND E.1/110 AND E.1/111 |  |  |  |
|  | 8.2 | Text attribute – center alignment | Rel-13 |  | C154 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/218 AND E.1/110 AND E.1/111 |  |  |  |
|  | 8.3 | Text attribute – right alignment | Rel-13 |  | C155 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/219 AND E.1/110 AND E.1/111 |  |  |  |
|  | 8.4 | Text attribute – large font size | Rel-13 |  | C157 AND C156 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 AND E.1/111 |  |  |  |
|  | 8.5 | Text attribute – small font size | Rel-13 |  | C158 AND C156 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 AND E.1/111 |  |  |  |
|  | 8.6 | Text attribute – bold on | Rel-13 |  | C160 AND C159 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 AND E.1/111 |  |  |  |
|  | 8.7 | Text attribute – italic on | Rel-13 |  | C161 AND C159 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 AND E.1/111 |  |  |  |
|  | 8.8 | Text attribute – underlined on | Rel-13 |  | C162 AND C159 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 AND E.1/111 |  |  |  |
|  | 8.9 | Text attribute – strikethrough on | Rel-13 |  | C163 AND C159 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 AND E.1/111 |  |  |  |
|  | 8.10 | Text attribute – foreground and background colours | Rel-13 |  | C164 AND C159 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.3.9 | GET INPUT (UCS2 display in Chinese) |  |  |  |  |  |  |  |  |
|  | 9.1 | Text string - UCS2 coding in Chinese | Rel-13 |  | C143 AND C177 AND C178 | E.1/19 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.2 | Text string - max length UCS2 coding in Chinese | Rel-13 |  | C143 AND C177 AND C178 | E.1/19 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.3.10 | GET INPUT (UCS2 entry in Chinese) |  |  |  |  |  |  |  |  |
|  | 10.1 | Character set from UCS2; Chinese characters | Rel-13 |  | C142 AND C177 AND C178 | E.1/19 AND E.1/14 AND E.1/110 AND E.1/111 |  |  |  |
|  | 10.2 | Character set from UCS2; max length; Chinese characters | Rel-13 |  | C142 AND C177 AND C178 | E.1/19 AND E.1/14 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.3.11 | GET INPUT (UCS2 display in Katakana) |  |  |  |  |  |  |  |  |
|  | 11.1 | Text string - UCS2 coding in Katakana | Rel-13 |  | C145 AND C177 AND C178 | E.1/19 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
|  | 11.2 | Text string - max length UCS2 coding in Katakana | Rel-13 |  | C145 AND C177 AND C178 | E.1/19 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.3.12 | GET INPUT (UCS2 entry in Katakana) |  |  |  |  |  |  |  |  |
|  | 12.1 | Character set from UCS2; Katakana | Rel-13 |  | C144 AND C177 AND C178 | E.1/19 AND E.1/14 AND E.1/110 AND E.1/111 |  |  |  |
|  | 12.2 | Character set from UCS2; max length; Katakana | Rel-13 |  | C144 AND C177 AND C178 | E.1/19 AND E.1/14 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.4 | MORE TIME |  |  |  |  |  |  |  |  |
|  | 1.1 | MORE TIME | Rel-13 |  | M | E.1/20 |  |  |  |
| 10.4.5 | PLAY TONE |  |  |  |  |  |  |  |  |
| 10.4.5.2 | PLAY TONE (UCS2 display in Cyrillic) |  |  |  |  |  |  |  |  |
|  | 2.1 | UCS2 display in Cyrillic | Rel-13 |  | C118 AND C179 | E.1/21 AND E.1/15 AND E.1/110 |  | TCEP001 |  |
| 10.4.5.3 | PLAY TONE (Display of icons) |  |  |  |  |  |  |  |  |
|  | 3.1 | Basic icon; self-explanatory | Rel-13 |  | C108 AND C179 | E.1/21 AND E.1/110 |  | TCEP001 |  |
|  | 3.2 | Basic icon; non self-explanatory | Rel-13 |  | C108 AND C179 | E.1/21 AND E.1/110 |  | TCEP001 |  |
|  | 3.3 | Colour icon; self-explanatory | Rel-13 |  | C171 AND C179 | E.1/21 AND E.1/110 |  | TCEP001 |  |
|  | 3.4 | Colour icon; non self-explanatory | Rel-13 |  | C171 AND C179 | E.1/21 AND E.1/110 |  | TCEP001 |  |
| 10.4.5.4 | PLAY TONE (Support of Text Attribute) |  |  |  |  |  |  |  |  |
|  | 4.1 | Text attribute – left alignment | Rel-13 |  | C153 AND C179 | E.1/21 AND E.1/124 AND E.1/218 AND E.1/110 |  | TCEP001 |  |
|  | 4.2 | Text attribute – center alignment | Rel-13 |  | C154 AND C179 | E.1/21 AND E.1/124 AND E.1/218 AND E.1/110 |  | TCEP001 |  |
|  | 4.3 | Text attribute – right alignment | Rel-13 |  | C155 AND C179 | E.1/21 AND E.1/124 AND E.1/219 AND E.1/110 |  | TCEP001 |  |
|  | 4.4 | Text attribute – large font size | Rel-13 |  | C157 AND C156 AND C179 | E.1/21 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 |  | TCEP001 |  |
|  | 4.5 | Text attribute – small font size | Rel-13 |  | C158 AND C156 AND C179 | E.1/21 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 |  | TCEP001 |  |
|  | 4.6 | Text attribute – bold on | Rel-13 |  | C160 AND C159 AND C179 | E.1/21 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 |  | TCEP001 |  |
|  | 4.7 | Text attribute – italic on | Rel-13 |  | C161 AND C159 AND C179 | E.1/21 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 |  | TCEP001 |  |
|  | 4.8 | Text attribute – underlined on | Rel-13 |  | C162 AND C159 AND C179 | E.1/21 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 |  | TCEP001 |  |
|  | 4.9 | Text attribute – strikethrough on | Rel-13 |  | C163 AND C159 AND C179 | E.1/21 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 |  | TCEP001 |  |
|  | 4.10 | Text attribute– foreground and background colours | Rel-13 |  | C164 AND C165 AND C179 | E.1/21 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 |  | TCEP001 |  |
| 10.4.5.5 | PLAY TONE (UCS2 display in Chinese) |  |  |  |  |  |  |  |  |
|  | 5.1 | UCS2 display in Chinese | Rel-13 |  | C143 AND C179 | E.1/21 AND E.1/15 AND E.1/110 |  | TCEP001 |  |
| 10.4.5.6 | PLAY TONE (UCS2 display in Katakana) |  |  |  |  |  |  |  |  |
|  | 6.1 | UCS2 display in Katakana | Rel-13 |  | C145 AND C179 | E.1/21 AND E.1/15 AND E.1/110 |  | TCEP001 |  |
| 10.4.6 | POLL INTERVAL |  |  |  |  |  |  |  |  |
|  | 1.1 |  | Rel-13 |  | M | E.1/221.1 |  |  |  |
| 10.4.7 | REFRESH |  |  |  |  |  |  |  |  |
| 10.4.7.1 | REFRESH (Normal) |  |  |  |  |  |  |  |  |
|  | 1.3 | USIM initialization and file change notification of ADN | Rel-13 |  | C177 AND C178 | E.1/24 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.5 | UICC reset | Rel-13 |  | M | E.1/24 |  |  |  |
| 10.4.7.3 | REFRESH (Steering of roaming) |  |  |  |  |  |  |  |  |
|  | 3.3 | Steering of roaming; E-UTRAN | Rel-13 |  | C222 | E.1/24 AND E.1/135 AND E.1/236 | E-USS OR NB-SS |  |  |
|  | 3.4 | Steering of roaming; NG-RAN | Rel-16 |  | C231 | E.1/24 AND E.1/236 | NG-SS only |  |  |
| 10.4.7.4 | REFRESH (AID) |  |  |  |  |  |  |  |  |
|  | 4.1 | Refresh with AID; E-UTRAN | Rel-13 |  | C202 | E.1/24 | E-USS only |  |  |
| 10.4.7.5 | REFRESH (IMSI changing procedure, E-UTRAN) |  |  |  |  |  |  |  |  |
|  | 5.1 | UICC Reset for IMSI Changing procedure; E-UTRAN | Rel-13 |  | C222 | E.1/24 | E-USS OR NB-SS |  |  |
|  | 5.2 | 3G Session Reset for IMSI Changing procedure; E-UTRAN | Rel-13 |  | C222 | E.1/24 | E-USS OR NB-SS |  |  |
| 10.4.7.6 | REFRESH (IMSI changing procedure, NG-RAN ) |  |  |  |  |  |  |  |  |
|  | 6.1 | UICC Reset for IMSI Changing procedure; NG-RAN | Rel-16 |  | C231 | E.1/24 OR (E.1/24 AND E.1/256) | NG-SS only |  |  |
|  | 6.2 | 3G Session Reset for IMSI Changing procedure; NG-RAN | Rel-16 |  | C231 | E.1/24 OR (E.1/24 AND E.1/256) | NG-SS only |  |  |
| 10.4.7.7 | REFRESH (SUPI_NAI changing procedure, NG-RAN) |  |  |  |  |  |  |  |  |
|  | 7.1 | UICC Reset for SUPI_NAI Changing procedure; NG-RAN | Rel-16 |  | C231 AND C233 | E.1/24 | NG-SS only |  |  |
|  | 7.2 | 3G Session Reset for SUPI_NAI Changing procedure; NG-RAN | Rel-16 |  | C231 AND C233 | E.1/24 | NG-SS only |  |  |
| 10.4.8 | SET UP MENU and ENVELOPE MENU SELECTION |  |  |  |  |  |  |  |  |
| 10.4.8.1 | SET UP MENU (Normal) and ENVELOPE MENU SELECTION |  |  |  |  |  |  |  |  |
|  | 1.1 | Set up; menu selection; replace and remove menu | Rel-13 |  | C177 AND C178 | E.1/30 AND E.1/4 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.2 | Large menu | Rel-13 |  | C177 AND C178 | E.1/30 AND E.1/4 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.8.2 | SET UP MENU (Help request support) and ENVELOPE MENU SELECTION |  |  |  |  |  |  |  |  |
|  | 2.1 | Help information | Rel-13 |  | C107 AND C177 AND C178 | E.1/30 AND E.1/4 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.8.3 | SET UP MENU (Help request support) and ENVELOPE MENU SELECTION |  |  |  |  |  |  |  |  |
|  | 3.1 | Next action indicator | Rel-13 |  | C177 AND C178 | E.1/30 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.8.4 | SET UP MENU (Display of icons) and ENVELOPE MENU SELECTION |  |  |  |  |  |  |  |  |
|  | 4.1 | Basic icon; not self-explanatory | Rel-13 |  | C172 AND C177 AND C178 | E.1/30 AND E.1/110 AND E.1/111 |  |  |  |
|  | 4.2 | Basic icon; self-explanatory | Rel-13 |  | C172 AND C177 AND C178 | E.1/30 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.8.5 | SET UP MENU (Soft keys support) and ENVELOPE MENU SELECTION |  |  |  |  |  |  |  |  |
|  | 5.1 | Soft key preferred | Rel-13 |  | C112 AND C177 AND C178 | E.1/30 AND E.1/74 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.8.6 | SET UP MENU (Support of Text Attribute) and ENVELOPE MENU SELECTION |  |  |  |  |  |  |  |  |
|  | 6.1 | Text attribute – left alignment | Rel-13 |  | C153 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/217 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.2 | Text attribute – center alignment | Rel-13 |  | C154 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/218 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.3 | Text attribute – right alignment | Rel-13 |  | C155 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/219 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.4 | Text attribute – large font size | Rel-13 |  | C157 AND C156 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.5 | Text attribute – small font size | Rel-13 |  | C158 AND C156 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.6 | Text attribute – bold on | Rel-13 |  | C160 AND C159 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.7 | Text attribute – italic on | Rel-13 |  | C161 AND C159 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.8 | Text attribute – underlined on | Rel-13 |  | C162 AND C159 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.9 | Text attribute – strikethrough on | Rel-13 |  | C163 AND C159 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.10 | Text attribute – foreground and background colours | Rel-13 |  | C164 AND C165 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.8.7 | SET UP MENU (UCS2 display in Cyrillic) and ENVELOPE MENU SELECTION |  |  |  |  |  |  |  |  |
|  | 7.1 | UCS2 display in Cyrillic | Rel-13 |  | C118 AND C177 AND C178 | E.1/39 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.8.8 | SET UP MENU (UCS2 display in Chinese) and ENVELOPE MENU SELECTION |  |  |  |  |  |  |  |  |
|  | 8.1 | UCS2 display in Chinese | Rel-13 |  | C143 AND C177 AND C178 | E.1/39 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.8.9 | SET UP MENU (UCS2 display in Katakana) and ENVELOPE MENU SELECTION |  |  |  |  |  |  |  |  |
|  | 9.1 | UCS2 display in Katakana | Rel-13 |  | C145 AND C177 AND C178 | E.1/39 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.9 | SELECT ITEM |  |  |  |  |  |  |  |  |
| 10.4.9.1 | SELECT ITEM (Mandatory features for ME supporting SELECT ITEM) |  |  |  |  |  |  |  |  |
|  | 1.1 | Mandatory features | Rel-13 |  | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.2 | Large menu | Rel-13 |  | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.3 | Call option | Rel-13 |  | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.4 | Backward move | Rel-13 |  | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.5 | "Y" successful | Rel-13 |  | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
|  | 1.6 | Large menu | Rel-13 |  | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.9.2 | SELECT ITEM (Next action support) |  |  |  |  |  |  |  |  |
|  | 2.1 | Next action indicator | Rel-13 |  | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.9.3 | SELECT ITEM (Default item support) |  |  |  |  |  |  |  |  |
|  | 3.1 | Default selected | Rel-13 |  | C177 AND C178 AND C194 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.9.4 | SELECT ITEM ((Help request support) |  |  |  |  |  |  |  |  |
|  | 4.1 | Help request | Rel-13 |  | C107 AND C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.9.5 | SELECT ITEM (Icons support) |  |  |  |  |  |  |  |  |
|  | 5.1 | Basic icon; not self-explanatory | Rel-13 |  | C172 AND C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
|  | 5.2 | Basic icon; self-explanatory | Rel-13 |  | C172 AND C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.9.6 | SELECT ITEM (Presentation style) |  |  |  |  |  |  |  |  |
|  | 6.1 | Presentation as a choice of navigation options | Rel-13 |  | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
|  | 6.2 | Presentation as a choice of data values | Rel-13 |  | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.9.7 | SELECT ITEM (Soft keys support) |  |  |  |  |  |  |  |  |
|  | 7.1 | Soft keys | Rel-13 |  | C112 AND C177 AND C178 | E.1/25 AND E.1/73 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.9.8 | SELECT ITEM (Support of "No response from user") |  |  |  |  |  |  |  |  |
|  | 8.1 | No Response from user | Rel-13 |  | C120 AND C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.9.9 | SELECT ITEM (Support of Text Attribute) |  |  |  |  |  |  |  |  |
|  | 9.1 | Text attribute – left alignment | Rel-13 |  | C153 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/217 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.2 | Text attribute – center alignment | Rel-13 |  | C154 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/218 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.3 | Text attribute – right alignment | Rel-13 |  | C155 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/219 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.4 | Text attribute – large font size | Rel-13 |  | C157 AND C156 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.5 | Text attribute – small font size | Rel-13 |  | C158 AND C156 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.6 | Text attribute – bold on | Rel-13 |  | C160 AND C159 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.7 | Text attribute – italic on | Rel-13 |  | C161 AND C159 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.8 | Text attribute – underline on | Rel-13 |  | C162 AND C159 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.9 | Text attribute – strikethrough on | Rel-13 |  | C163 AND C159 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 AND E.1/111 |  |  |  |
|  | 9.10 | Text attribute – foreground and background colours | Rel-13 |  | C164 AND C165 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.9.10 | SELECT ITEM (UCS2 display in Cyrillic) |  |  |  |  |  |  |  |  |
|  | 10.1 | UCS2 in Cyrillic characters; 0x80 UCS2 coding | Rel-13 |  | C118 AND C177 AND C178 | E.1/39 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
|  | 10.2 | UCS2 in Cyrillic characters; 0x81 UCS2 coding | Rel-13 |  | C118 AND C177 AND C178 | E.1/39 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
|  | 10.3 | UCS2 in Cyrillic characters; 0x82 UCS2 coding | Rel-13 |  | C118 AND C177 AND C178 | E.1/39 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.9.11 | SELECT ITEM (UCS2 display in Chinese) |  |  |  |  |  |  |  |  |
|  | 11.1 | UCS2 display in Chinese | Rel-13 |  | C143 AND C177 AND C178 | E.1/25 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.9.12 | SELECT ITEM (UCS2 display in Katakana) |  |  |  |  |  |  |  |  |
|  | 12.1 | UCS2 in Katakana characters; 0x80 UCS2 coding | Rel-13 |  | C145 AND C177 AND C178 | E.1/25 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
|  | 12.2 | UCS2 in Katakana characters; 0x81 UCS2 coding | Rel-13 |  | C145 AND C177 AND C178 | E.1/25 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
|  | 12.3 | UCS2 in Katakana characters; 0x82 UCS2 coding | Rel-13 |  | C145 AND C177 AND C178 | E.1/25 AND E.1/15 AND E.1/110 AND E.1/111 |  |  |  |
| 10.4.10 | SEND SHORT MESSAGE |  |  |  |  |  |  |  |  |
| 10.4.10.7 | SEND SHORT MESSAGE (IMS) |  |  |  |  |  |  |  |  |
|  | 7.1 | SMS-over-IP; E-UTRAN | Rel-13 |  | C196 | E.1/26 AND E.1/110 | E-USS only | TCEP001 |  |
| 10.4.10.8 | SEND SHORT MESSAGE (Over SGs in E-UTRAN). |  |  |  |  |  |  |  |  |
|  | 8.1 | Send Short Message over SGs; E-UTRAN | Rel-13 |  | C220 | E.1/26 AND E.1/110 | E-USS OR NB-SS | TCEP001 |  |
| 10.4.11 | SEND SS |  |  |  |  |  |  |  |  |
| 10.4.12 | SEND USSD |  |  |  |  |  |  |  |  |
| 10.4.13 | SET UP CALL |  |  |  |  |  |  |  |  |
| 10.4.14 | POLLING OFF |  |  |  |  |  |  |  |  |
|  | 1.2 | POLLING OFF; E-UTRAN | Rel-13 |  | C222 | E.1/23 | E-USS OR NB-SS |  |  |
| 10.4.15 | PROVIDE LOCAL INFORMATION |  |  |  |  |  |  |  |  |
|  | 1.2 | IMEI | Rel-13 |  | M | E.1/31 |  |  |  |
|  | 1.4 | Date; time and time zone | Rel-13 |  | M | E.1/59 |  |  |  |
|  | 1.5 | Language setting | Rel-13 |  | C217 | E.1/68 |  |  |  |
|  | 1.9 | IMEISV | Rel-13 |  | M | E.1/143 |  |  |  |
|  | 1.10 | Network Search Mode | Rel-13 |  | M | E.1/144 | E-USS |  |  |
|  | 1.11 | Charge State of the Battery | Rel-13 |  | C139 | E.1/170 |  |  |  |
|  | 1.14 | Access Technology; E-UTRAN | Rel-13 |  | C222 | E.1/72 | E-USS OR NB-SS |  |  |
|  | 1.15 | E-UTRAN Intra-Frequency Measurements | Rel-13 |  | C190 | E.1/183 | E-USS only |  |  |
|  | 1.16 | E-UTRAN Inter-Frequency Measurements | Rel-13 |  | C190 | E.1/183 | E-USS only |  |  |
|  | 1.17 | E-UTRAN Local Info (MCC; MNC; TAC & E-UTRAN Cell ID) | Rel-13 |  | C222 | E.1/31 AND E.1/135 | E-USS OR NB-SS |  |  |
|  | 1.18 | Discovery of surrounding CSG cells | Rel-13 |  | C195 | E.1/242 | E-USS only |  |  |
|  | 1.22 | NG-RAN Local Info (MCC; MNC; TAC & NG-RAN Cell ID) | Rel-15 |  | C231 | E.1/31 | NG-SS only |  |  |
|  | 1.23 | Access Technology; NG-RAN | Rel-15 |  | C231 | E.1/72 | NG-SS only |  |  |
|  | 1.24 | Slices Information | Rel-16 |  | C231 | E.1/284 | NG-SS only |  |  |
|  | 1.25 | Slices Information; no served Slice | Rel-16 |  | C231 | E.1/284 | NG-SS only |  |  |
|  | 1.26 | Slices Information; several served Slices | Rel-16 |  | C231 | E.1/284 | NG-SS only |  |  |
|  | 1.27 | NG-RAN Timing advance | Rel-16 |  | C231 | E.1/305 | NG-SS only |  |  |
|  | 1.28 | PROVIDE LOCAL INFORMATION; NG-RAN Intra-Frequency Measurements | Rel-16 |  | C231 | E.1/305 | NG-SS only |  |  |
|  | 1.29 | PROVIDE LOCAL INFORMATION; NG-RAN Inter-Frequency Measurements | Rel-16 |  | C231 | E.1/305 | NG-SS only |  |  |
|  | 1.30 | PROVIDE LOCAL INFORMATION; CAG information list | Rel-17 |  | C235 | E.1/287 | NG-SS only |  |  |
|  | 1.27 | NG-RAN Timing advance | Rel-16 |  | C231 | E.1/305 | NG-SS only |  |  |
|  | 1.28 | PROVIDE LOCAL INFORMATION; NG-RAN Intra-Frequency Measurements | Rel-16 |  | C231 | E.1/305 | NG-SS only |  |  |
|  | 1.29 | PROVIDE LOCAL INFORMATION; NG-RAN Inter-Frequency Measurements | Rel-16 |  | C231 | E.1/305 | NG-SS only |  |  |
|  | 1.30 | PROVIDE LOCAL INFORMATION; CAG information list | Rel-17 |  | C235 | E.1/287 | NG-SS only |  |  |
| 10.4.16 | SET UP EVENT LIST |  |  |  |  |  |  |  |  |
| 10.4.17 | PERFORM CARD APDU |  |  |  |  |  |  |  |  |
| 10.4.17.1 | PERFORM CARD APDU (Normal) |  |  |  |  |  |  |  |  |
|  | 1.1 | Additional card inserted; Select MF and Get Response | Rel-13 |  | C109 | E.1/51 |  |  |  |
|  | 1.2 | Additional card inserted; Select DF GSM; Select EFPLMN; Update Binary; Read Binary on EFPLMN | Rel-13 |  | C109 | E.1/51 |  |  |  |
|  | 1.3 | Additional card inserted; card powered off | Rel-13 |  | C109 | E.1/51 |  |  |  |
|  | 1.4 | No card inserted; card powered off | Rel-13 |  | C109 | E.1/51 |  |  |  |
|  | 1.5 | Invalid card reader identifier | Rel-13 |  | C109 | E.1/51 |  |  |  |
| 10.4.17.2 | PERFORM CARD APDU (Detachable card reader) |  |  |  |  |  |  |  |  |
|  | 2.1 | Detachable reader | Rel-13 |  | C116 | E.1/51 |  |  |  |
| 10.4.18 | POWER OFF CARD |  |  |  |  |  |  |  |  |
| 10.4.18.1 | POWER OFF CARD (Normal) |  |  |  |  |  |  |  |  |
|  | 1.1 | Additional card inserted | Rel-13 |  | C109 | E.1/50 |  |  |  |
| 10.4.18.2 | POWER OFF CARD (Detachable card reader) |  |  |  |  |  |  |  |  |
|  | 2.1 | Detachable card reader | Rel-13 |  | C116 | E.1/50 |  |  |  |
| 10.4.19 | POWER ON CARD |  |  |  |  |  |  |  |  |
| 10.4.19.1 | POWER ON CARD (Normal) |  |  |  |  |  |  |  |  |
|  | 1.1 | Additional card inserted | Rel-13 |  | C109 | E.1/49 |  |  |  |
| 10.4.19.2 | POWER ON CARD (Detachable card reader) |  |  |  |  |  |  |  |  |
|  | 2.1 | Detachable card reader | Rel-13 |  | C116 | E.1/49 |  |  |  |
| 10.4.20 | GET READER STATUS |  |  |  |  |  |  |  |  |
| 10.4.20.1 | GET READER STATUS (Normal) |  |  |  |  |  |  |  |  |
|  | 1.1 | Additional card inserted; card powered | Rel-13 |  | C109 | E.1/52 |  |  |  |
|  | 1.2 | Additional card inserted; card not powered | Rel-13 |  | C109 | E.1/52 |  |  |  |
| 10.4.20.2 | GET READER STATUS (Detachable card reader) |  |  |  |  |  |  |  |  |
|  | 2.1 | Detachable card reader | Rel-13 |  | C116 | E.1/52 |  |  |  |
| 10.4.21 | TIMER MANAGEMENT and ENVELOPE TIMER EXPIRATION |  |  |  |  |  |  |  |  |
| 10.4.21.1 | TIMER MANAGEMENT (Normal) |  |  |  |  |  |  |  |  |
|  | 1.1 | Start timer 1 several times; get the current value of the timer and deactivate the timer successfully | Rel-13 |  | M | E.1/57 AND E.1/58 |  |  |  |
|  | 1.2 | Start timer 2 several times; get the current value of the timer and deactivate the timer successfully | Rel-13 |  | M | E.1/57 AND E.1/58 |  |  |  |
|  | 1.3 | Start timer 8 several times; get the current value of the timer and deactivate the timer successfully | Rel-13 |  | M | E.1/57 AND E.1/58 |  |  |  |
|  | 1.4 | Try to get the current value of a timer which is not started: action in contradiction with the current timer state | Rel-13 |  | M | E.1/57 AND E.1/58 |  |  |  |
|  | 1.5 | Try to deactivate a timer which is not started: action in contradiction with the current timer state | Rel-13 |  | M | E.1/57 AND E.1/58 |  |  |  |
|  | 1.6 | Start 8 timers successfully | Rel-13 |  | M | E.1/57 AND E.1/58 |  |  |  |
| 10.4.21.2 | ENVELOPE TIMER EXPIRATION (Normal) |  |  |  |  |  |  |  |  |
|  | 2.1 | Pending proactive UICC command | Rel-13 |  | M | E.1/6 AND E.1/57 |  |  |  |
|  | 2.2 | USIM application toolkit busy | Rel-13 |  | M | E.1/6 AND E.1/57 AND E.1/20 |  |  |  |
| 10.4.22 | SET UP IDLE MODE TEXT |  |  |  |  |  |  |  |  |
| 10.4.23 | RUN AT COMMAND |  |  |  |  |  |  |  |  |
| 10.4.23.1 | RUN AT COMMAND (Normal) |  |  |  |  |  |  |  |  |
|  | 1.1 | No alpha identifier presented | Rel-13 |  | C110 | E.1/62 |  |  |  |
|  | 1.2 | Null data alpha identifier presented | Rel-13 |  | C110 | E.1/62 |  |  |  |
|  | 1.3 | Alpha identifier presented | Rel-13 |  | C110 AND C177 | E.1/62 AND E.1/110 |  |  |  |
| 10.4.23.2 | RUN AT COMMAND (Icon support) |  |  |  |  |  |  |  |  |
|  | 2.1 | Basic icon; self-explanatory | Rel-13 |  | C114 AND C177 | E.1/62 AND E.1/110 |  |  |  |
|  | 2.2 | Colour icon; self-explanatory | Rel-13 |  | C173 AND C177 | E.1/62 AND E.1/110 |  |  |  |
|  | 2.3 | Basic icon; non self-explanatory | Rel-13 |  | C114 AND C177 | E.1/62 AND E.1/110 |  |  |  |
|  | 2.4 | Colour icon; non self-explanatory | Rel-13 |  | C173 AND C177 | E.1/62 AND E.1/110 |  |  |  |
|  | 2.5 | Basic icon non self-explanatory; no alpha identifier presented | Rel-13 |  | C110 AND C153 AND C177 | E.1/62 AND E.1/110 |  |  |  |
| 10.4.23.3 | RUN AT COMMAND (Support of Text Attribute) |  |  |  |  |  |  |  |  |
|  | 3.1 | Text attribute – left alignment | Rel-13 |  | C110 AND C154 AND C177 | E.1/62 AND E.1/124 AND E.1/217 AND E.1/110 |  |  |  |
|  | 3.2 | Text attribute – center alignment | Rel-13 |  | C110 AND C155 AND C177 | E.1/62 AND E.1/124 AND E.1/218 AND E.1/110 |  |  |  |
|  | 3.3 | Text attribute – right alignment | Rel-13 |  | C110 AND C157 AND C156 AND C177 | E.1/62 AND E.1/124 AND E.1/219 AND E.1/110 |  |  |  |
|  | 3.4 | Text attribute – large font size | Rel-13 |  | C110 AND C158 AND C156 AND C177 | E.1/62 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 |  |  |  |
|  | 3.5 | Text attribute – small font size | Rel-13 |  | C110 AND C160 AND C159 AND C177 | E.1/62 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 |  |  |  |
|  | 3.6 | Text attribute – bold on | Rel-13 |  | C110 AND C161 AND C159 AND C177 | E.1/62 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 |  |  |  |
|  | 3.7 | Text attribute – italic on | Rel-13 |  | C110 AND C162 AND C159 AND C177 | E.1/62 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 |  |  |  |
|  | 3.8 | Text attribute – underline on | Rel-13 |  | C110 AND C163 AND C159 AND C177 | E.1/62 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 |  |  |  |
|  | 3.9 | Text attribute – strikethrough on | Rel-13 |  | C110 AND C164 AND C165 AND C177 | E.1/62 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 |  |  |  |
|  | 3.10 | Text attribute – foreground and background colours | Rel-13 |  | C110 AND C164 AND C165 AND C177 | E.1/62 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 |  |  |  |
| 10.4.23.4 | RUN AT COMMAND (UCS2 display in Cyrillic) |  |  |  |  |  |  |  |  |
|  | 4.1 | UCS2 display in Cyrillic | Rel-13 |  | C149 AND C177 | E.1/62 AND E.1/15 AND E.1/110 |  |  |  |
| 10.4.23.5 | RUN AT COMMAND (UCS2 display in Chinese) |  |  |  |  |  |  |  |  |
|  | 5.1 | UCS2 display in Chinese | Rel-13 |  | C150 AND C177 | E.1/62 AND E.1/15 AND E.1/110 |  |  |  |
| 10.4.23.6 | RUN AT COMMAND (UCS2 display in Katakana |  |  |  |  |  |  |  |  |
|  | 6.1 | UCS2 display in Katakana | Rel-13 |  | C151 AND C177 | E.1/62 AND E.1/15 AND E.1/110 |  |  |  |
| 10.4.24 | SEND DTMF |  |  |  |  |  |  |  |  |
| 10.4.25 | LANGUAGE NOTIFICATION |  |  |  |  |  |  |  |  |
|  | 1.1 | Specific language notification | Rel-13 |  | C181 AND C218 | E.1/70 |  |  |  |
|  | 1.2 | Non-specific language notification | Rel-13 |  | C181 AND C218 | E.1/70 |  |  |  |
| 10.4.26 | LAUNCH BROWSER |  |  |  |  |  |  |  |  |
| 10.4.26.8 | LAUNCH BROWSER (NG-RAN bearer) |  |  |  |  |  |  |  |  |
|  | 8.1 | Only NG-RAN bearer specified and gateway proxy identity | Rel-16 |  | C111 AND C231 | E.1/71 AND E.1/98 AND E.1/110 AND E.1/111 | NG-SS only |  |  |
|  | 8.2 | Trigger LAUNCH BROWSER by CALL CONTROL | Rel-16 |  | C111 AND C231 | E.1/71 AND E.1/110 AND E.1/111 | NG-SS only |  |  |
|  | 8.3 | Trigger LAUNCH BROWSER by MT Call event | Rel-16 |  | C111 AND C231 | E.1/71 AND E.1/110 AND E.1/111 | NG-SS only |  |  |
|  | 8.4 | Trigger LAUNCH BROWSER during mobile originated call | Rel-16 |  | C111 AND C231 | E.1/71 AND E.1/110 AND E.1/111 | NG-SS only |  |  |
|  | 8.5 | Trigger LAUNCH BROWSER during mobile terminated call | Rel-16 |  | C111 AND C231 | E.1/71 AND E.1/110 AND E.1/111 | NG-SS only |  |  |
| 10.4.27 | OPEN CHANNEL |  |  |  |  |  |  |  |  |
| 10.4.27.6 | OPEN CHANNEL (related to E-UTRAN) |  |  |  |  |  |  |  |  |
|  | 6.1 | Immediate link establishment; E-UTRAN; bearer type '02' | Rel-13 |  | C224 | E.1/89 AND E.1/135 | E-USS OR NB-SS |  |  |
|  | 6.2 | Immediate link establishment; E-UTRAN; bearer type '0B' | Rel-13 |  | C182 | E.1/89 AND E.1/135 | E-USS only |  |  |
|  | 6.3 | E-UTRAN; bearer type '02'; immediate link establishment with Network Access Name; with alpha identifier | Rel-13 |  | C224 | E.1/89 AND E.1/110 AND E.1/111 AND E.1/135 | E-USS OR NB-SS |  |  |
|  | 6.4 | E-UTRAN bearer type '03'; immediate link establishment with alpha identifier; user did not accept the proactive command | Rel-13 |  | C182 AND C177 | E.1/89 AND E.1/110 AND E.1/111 AND E.1/135 | E-USS OR NB-SS |  |  |
|  | 6.5 | E-UTRAN; bearer type '03' – default EPS bearer; immediate link establishment | Rel-13 |  | C182 | E.1/89 AND E.1/135 | E-USS OR NB-SS |  |  |
|  | 6.6 | BIP is not a 3GPP PS data off exempt service | Rel-14 |  | C228 | E.1/2 AND E.1/89 AND E.1/135 | E-USS |  |  |
|  | 6.7 | BIP is a 3GPP PS data off exempt service | Rel-14 |  | C228 | E.1/2 AND E.1/89 AND E.1/135 | E-USS |  |  |
|  | 6.8 | Maximum number of open channel requests | Rel-14 |  |  | E.1/89 AND E.1/135 | E-USS OR NB-SS |  |  |
| 10.4.27.7 | OPEN CHANNEL (UICC Access to IMS) |  |  |  |  |  |  |  |  |
|  | 7.1 | OPEN CHANNEL for IMS; IARI list stored on the USIM | Rel-13 |  | C207 | E.1/33 AND E.1/89 AND E.1/247 AND E.1/249 | E-USS |  |  |
| 10.4.27.8 | OPEN CHANNEL (related to NG-RAN) |  |  |  |  |  |  |  |  |
|  | 8.1 | NG-RAN bearer type '03' – default PDU Session; immediate link establishment | Rel-15 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | 8.2 | NG-RAN bearer type '0C'; immediate link establishment | Rel-15 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | 8.3 | NG-RAN bearer type '0C'; after receiving policy update for URSP from network | Rel-15 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | 8.4 | NG-RAN bearer type '0C'; PDU Session is already available for the same DNN | Rel-15 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | 8.5 | NG-RAN bearer type '02' – default PDU Session; immediate link establishment | Rel-15 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | 8.6 | NG-RAN bearer type '0B' – default PDU Session; immediate link establishment | Rel-15 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
| 10.4.28 | CLOSE CHANNEL |  |  |  |  |  |  |  |  |
| 10.4.28.3 | CLOSE CHANNEL (E-UTRAN/EPC) |  |  |  |  |  |  |  |  |
|  | 3.1 | Default EPS bearer | Rel-13 |  | C224 | E.1/89 AND E.1/90 | E-USS OR NB-SS |  |  |
|  | 3.2 | EPS bearer with APN different from default APN | Rel-13 |  | C224 | E.1/89 AND E.1/90 | E-USS OR NB-SS | TCEP001, TCEP002 |  |
|  | 3.3 | Command qualifier set to 1 | Rel-13 |  | C230 | E.1/89 AND E.1/90 | E-USS OR NB-SS |  |  |
| 10.4.28.4 | CLOSE CHANNEL (NG-RAN) |  |  |  |  |  |  |  |  |
|  | 4.1 | NG-RAN; bearer type '03' – default PDU Session | Rel-15 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | 4.2 | NG-RAN; bearer type '0C' | Rel-15 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
| 10.4.29 | RECEIVE DATA |  |  |  |  |  |  |  |  |
|  | 1.2 | Already opened channel – E-UTRAN; APN different from default | Rel-13 |  | C182 | E.1/89 AND E.1/91 AND E.1/92 | E-USS OR NB-SS |  |  |
|  | 1.3 | Length of received data exceeds the buffer size. | Rel-15 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | 1.4 | Receive 65535 Bytes of data | Rel-16 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | 1.5 | Send refresh after receiving data. | Rel-16 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | 1.7 | 2 consecutive RECEIVE DATA | Rel-16 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
| 10.4.30 | SEND DATA |  |  |  |  |  |  |  |  |
| 10.4.30.3 | SEND DATA(E-UTRAN) |  |  |  |  |  |  |  |  |
|  | 3.1 | Immediate mode – E-UTRAN; Default EPS bearer | Rel-13 |  | C223 | E.1/89 AND E.1/92 | E-USS OR NB-SS |  |  |
|  | 3.2 | Store mode – E-UTRAN; APN different from default APN | Rel-13 |  | C224 | E.1/89 AND E.1/92 | E-USS OR NB-SS |  |  |
| 10.4.30.4 | SEND DATA(NG-RAN) |  |  |  |  |  |  |  |  |
|  | 4.1 | NG-RAN; bearer type '03' – Default PDU Session; immediate mode | Rel-16 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | 4.2 | NG-RAN; bearer type '0C'; Store mode | Rel-16 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
| 10.4.31 | GET CHANNEL STATUS |  |  |  |  |  |  |  |  |
|  | 1.4 | EPS bearer with APN different from default APN | Rel-13 |  | C224 | E.1/89 AND E.1/93 | E-USS OR NB-SS |  |  |
|  | 1.5 | EPS bearer with APN different from default APN; after a link dropped | Rel-13 |  | C224 | E.1/89 AND E.1/93 | E-USS OR NB-SS |  |  |
|  | 1.6 | After a link dropped during receiving data | Rel-15 |  | C232 | E.1/89 AND E.1/281 | NG-SS only |  |  |
| 10.5 | Data Download to UICC |  |  |  |  |  |  |  |  |
| 10.5.1 | SMS-PP Data Download |  |  |  |  |  |  |  |  |
| 10.5.2 | Cell Broadcast Data Download |  |  |  |  |  |  |  |  |
| 10.5.3 | SMS-PP Data Download over IMS |  |  |  |  |  |  |  |  |
|  | 3.1 | SMS-PP Data Download over IMS; E-UTRAN | Rel-13 |  | C198 | E.1/2 | E-USS only | TCEP001 |  |
| 10.5.4 | SMS-PP Data Download over SGs in E-UTRAN |  |  |  |  |  |  |  |  |
|  | 4.1 | SMS-PP Data Download over SGs; E-UTRAN | Rel-8 |  | C205 | E.1/2 | E-USS OR NB-SS | TCEP001 |  |
| 10.6 | CALL CONTROL BY USIM |  |  |  |  |  |  |  |  |
| 10.6.1 | Procedure for Mobile Originated calls |  |  |  |  |  |  |  |  |
| 10.6.2 | Procedure for Supplementary (SS) Services |  |  |  |  |  |  |  |  |
| 10.6.3 | Interaction with Fixed Dialling Number (FDN) |  |  |  |  |  |  |  |  |
| 10.6.4 | Support of Barred Dialling Number (BDN) service |  |  |  |  |  |  |  |  |
| 10.6.5 | Barred Dialling Number (BDN) service handling for terminals not supporting BDN |  |  |  |  |  |  |  |  |
| 10.7 | EVENT DOWNLOAD |  |  |  |  |  |  |  |  |
| 10.7.1 | MT Call Event |  |  |  |  |  |  |  |  |
| 10.7.2 | Call Connected Event |  |  |  |  |  |  |  |  |
| 10.7.3 | Call Disconnected Event |  |  |  |  |  |  |  |  |
| 10.7.4 | Location Status Event |  |  |  |  |  |  |  |  |
| 10.7.4.1 | Location Status Event (Normal) |  |  |  |  |  |  |  |  |
|  | 1.2 | E-UTRAN | Rel-13 |  | C222 | E.1/37 AND E.1/33 AND E.1/135 | E-USS OR NB-SS |  |  |
|  | 1.3 | NG-RAN | Rel-15 |  | C231 | E.1/37 AND E.1/33 | NG-SS only |  |  |
| 10.7.5 | User Activity Event |  |  |  |  |  |  |  |  |
| 10.7.5.1 | User Activity Event (Normal) |  |  |  |  |  |  |  |  |
|  | 1.1 | user activity event | Rel-13 |  | C178 | E.1/38 AND E.1/33 AND E.1/111 |  |  |  |
| 10.7.6 | Idle Screen Available Event |  |  |  |  |  |  |  |  |
| 10.7.7 | Card Reader Status Event |  |  |  |  |  |  |  |  |
| 10.7.7.1 | Card Reader Status (Normal) |  |  |  |  |  |  |  |  |
|  | 1.1 | Card reader status normal | Rel-13 |  | C109 | E.1/40 AND E.1/33 |  |  |  |
| 10.7.7.2 | Card Reader Status (Detachable card reader) |  |  |  |  |  |  |  |  |
|  | 2.1 | Detachable card reader | Rel-13 |  | C116 | E.1/40 AND E.1/33 |  |  |  |
| 10.7.8 | Language Selection Event |  |  |  |  |  |  |  |  |
| 10.7.8.1 | Language Selection Event (Normal) |  |  |  |  |  |  |  |  |
|  | 1.1 | Language selection event | Rel-13 |  | C177 AND C178 AND C181 AND C216 | E.1/41 AND E.1/33 AND E.1/110 AND E.1/111 |  |  |  |
| 10.7.9 | Browser Termination Event |  |  |  |  |  |  |  |  |
| 10.7.10 | Data Available Event |  |  |  |  |  |  |  |  |
| 10.7.10.1 | Data Available Event (Normal) |  |  |  |  |  |  |  |  |
|  | 1.2 | Data available event | Rel-13 |  | C223 | E.1/43 AND E.1/89 AND E.1/92 AND E.1/33 | E-USS OR NB-SS |  |  |
|  | 1.3 | Data available; PSM by SUSPEND UICC | Rel-13 | Rel-13 |  | E.1/43 AND E.1/89 AND E.1/92 AND E.1/33 | E-USS OR NB-SS | TCEP003 |  |
|  |  |  | Rel-14 |  | C225 |  |  |  |  |
|  | 1.4 | Data available event | Rel-13 | Rel-13 |  | E.1/43 AND E.1/89 AND E.1/92 AND E.1/33 | E-USS OR NB-SS | TCEP003 |  |
|  |  |  | Rel-14 |  | C226 |  |  |  |  |
|  | 1.5 | Data available event | Rel-13 | Rel-13 |  | E.1/43 AND E.1/89 AND E.1/92 AND E.1/33 | E-USS OR NB-SS | TCEP004 |  |
|  |  |  | Rel-14 |  | C227 |  |  |  |  |
| 10.7.11 | Channel Status event |  |  |  |  |  |  |  |  |
|  | 1.2 | Channel status event | Rel-13 |  | C223 | E.1/44 AND E.1/89 AND E.1/33 | E-USS OR NB-SS |  |  |
| 10.7.12 | Access Technology Change event |  |  |  |  |  |  |  |  |
|  | 1.4 | Single access technology; NG-RAN | Rel-15 |  | C231 | E.1/45 AND E.1/33 | NG-SS only |  |  |
| 10.7.13 | Display parameter changed event |  |  |  |  |  |  |  |  |
| 10.7.14 | Local Connection event |  |  |  |  |  |  |  |  |
| 10.7.15 | Network search mode change event |  |  |  |  |  |  |  |  |
|  | 1.1 | Network search mode change event | Rel-13 |  | M | E.1/48 AND E.1/33 |  |  |  |
| 10.7.16 | Browsing status event |  |  |  |  |  |  |  |  |
| 10.7.17 | Network Rejection event |  |  |  |  |  |  |  |  |
|  | 1.1 | ATTACH REJECT | Rel-13 |  | C190 | E.1/33 AND E.1/197 | E-USS OR NB-SS |  |  |
|  | 1.2 | TRACKING AREA UPDATE REJECT | Rel-13 |  | C190 | E.1/33 AND E.1/197 | E-USS OR NB-SS |  |  |
|  | 1.3 | REGISTRATION REJECT-Initial Registration | Rel-15 |  | C231 | E.1/33 AND E.1/197 | NG-SS only |  |  |
|  | 1.4 | REGISTRATION REJECT- Mobility Registration updating | Rel-15 |  | C231 | E.1/33 AND E.1/197 | NG-SS only |  |  |
| 10.7.18 | CSG Cell Selection event |  |  |  |  |  |  |  |  |
|  | 1.1 | CSG cell Selection | Rel-13 |  | C200 | E.1/201 | E-USS only |  |  |
| 10.7.19 | IMS registration event |  |  |  |  |  |  |  |  |
|  |  | IMS registration event (Refer to 10.4.27.7 AND 27.22.7.20) | Rel-13 |  |  |  |  |  |  |
| 10.7.20 | Incoming IMS data event |  |  |  |  |  |  |  |  |
|  | 1.1 | IMS Registration and Data available event; IARI list stored on the ISIM | Rel-10 |  | C208 | E.1/33 AND E.1/43 AND E.1/89 AND E.1/91 AND E.1/246 AND E.1.247 AND E.1/249 | E-USS |  |  |
| 10.7.21 | Data Connection Status Change event |  |  |  |  |  |  |  |  |
|  | 1.1 | E-UTRAN; Activate PDN and Deactivate PDN | Rel-14 |  | C229 | E.1/275 | E-USS OR NB-SS |  |  |
|  | 1.2 | NG-RAN; Activate PDU and Deactivate PDU | Rel-17 |  | C232 | E.1/275 | NG-SS |  |  |
| 10.7.22 | CAG Cell Selection event |  |  |  |  |  |  |  |  |
|  | 1.1 | EVENT DOWNLOAD – CAG Cell Selection | Rel-17 |  | C235 | E.1/287 | NG-SS only |  |  |
| 10.8 | MO SHORT MESSAGE CONTROL BY USIM |  |  |  |  |  |  |  |  |
|  | 1.10 | Over SG in E-UTRAN; with Proactive command; allowed; no modification | Rel-13 |  | C220 | E.1/12 AND E.1/26 AND E.1/110 | E-USS OR NB-SS | TCEP001 |  |
|  | 1.11 | Over SG in E-UTRAN; with user SMS; allowed; no modification | Rel-13 |  | C220 | E.1/12 | E-USS OR NB-SS |  |  |
|  | 1.12 | Over SG in E-UTRAN; with Proactive command; Not allowed | Rel-13 |  | C220 | E.1/12 AND E.1/26 AND E.1/110 | E-USS OR NB-SS | TCEP001 |  |
|  | 1.13 | Over SG in E-UTRAN; with user SMS; Not allowed | Rel-13 |  | C220 | E.1/12 | E-USS OR NB-SS |  |  |
|  | 1.14 | Over SG in E-UTRAN; with Proactive command; Allowed with modifications' | Rel-13 |  | C220 | E.1/12 AND E.1/26 AND E.1/110 | E-USS OR NB-SS | TCEP001 |  |
|  | 1.15 | Over SG in E-UTRAN; with user SMS; Allowed with modifications | Rel-13 |  | C220 | E.1/12 | E-USS OR NB-SS |  |  |
|  | 1.16 | Over SG in E-UTRAN; with Proactive command; USIM responds with '90 00'; Allowed; no modification | Rel-13 |  | C220 | E.1/12 AND E.1/26 AND E.1/110 | E-USS OR NB-SS | TCEP001 |  |
|  | 1.17 | Over SG in E-UTRAN; Send Short Message attempt by user; USIM responds with '90 00'; Allowed; no modification | Rel-13 |  | C220 | E.1/12 | E-USS OR NB-SS |  |  |
| 10.9 | Handling of command number |  |  |  |  |  |  |  |  |
|  | 1.1 | DISPLAY TEXT normal priority | Rel-13 |  | C177 | E.1/17 AND E.1/110 |  |  |  |
| 10.10 | CALL CONTROL on EPS PDN Connection |  |  |  |  |  |  |  |  |
|  | 1.1 | E-UTRAN – default PDN connection activation; allowed without modification | Rel-13 | Rel-13 |  | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | E-USS OR NB-SS |  |  |
|  |  |  | Rel-14 |  | C222 |  |  |  |  |
|  | 1.2 | E-UTRAN – default PDN connection activation; not allowed | Rel-13 | Rel-13 |  | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | E-USS OR NB-SS |  |  |
|  |  |  | Rel-14 |  | C222 |  |  |  |  |
|  | 1.3 | E-UTRAN – default PDN connection activation; allowed with modification | Rel-13 | Rel-13 |  | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | E-USS OR NB-SS |  |  |
|  |  |  | Rel-14 |  | C222 |  |  |  |  |
|  | 1.4 | E-UTRAN – PDN connection triggered by user; UICC sends 90 00 | Rel-13 | Rel-13 |  | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | E-USS only |  |  |
|  |  |  | Rel-14 |  | C190 |  |  |  |  |
|  | 1.5 | E-UTRAN – PDN connection triggered by user; UICC sends 93 00 | Rel-13 | Rel-13 |  | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | E-USS only |  |  |
|  |  |  | Rel-14 |  | C190 |  |  |  |  |
|  | 1.6 | E-UTRAN – PDN connection triggered by user; allowed with modification | Rel-13 | Rel-13 |  | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | E-USS only |  |  |
|  |  |  | Rel-14 |  | C190 |  |  |  |  |
|  | 1.7 | PDN connection activation from OPEN CHANNEL command | Rel-13 | Rel-13 |  | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | E-USS only |  |  |
|  |  |  | Rel-14 |  | C182 |  |  |  |  |
| 10.11 | Call Control on PDP Context Activation |  |  |  |  |  |  |  |  |
| 10.12 | Change eCall mode |  |  |  |  |  |  |  |  |
|  | 1.1 | REFRESH after change eCall mode; disable FDN in EFEST; E-UTRAN | Rel-13 | Rel-13 |  | E.1/24 AND E.1/2 | E-USS only |  |  |
|  |  |  | Rel-14 |  | C190 |  |  |  |  |
|  | 1.2 | REFRESH after change eCall mode; enable FDN in EFEST; E-UTRAN | Rel-13 | Rel-13 |  | E.1/24 AND E.1/2 | E-USS only |  |  |
|  |  |  | Rel-14 |  | C190 |  |  |  |  |
|  | 1.3 | REFRESH after changing eCall mode; disable FDN in EFEST; IMS Emergency Services in E-UTRAN | Rel-14 |  | C202 | E.1/24 AND E.1/2 | E-USS only |  |  |
| 10.13 | CALL CONTROL on PDU Session Establishment for NG-RAN |  |  |  |  |  |  |  |  |
|  | 1.1 | Allowed without modification; PDU Session establishment triggered by User | Rel-15 |  | C231 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | NG-SS only |  |  |
|  | 1.2 | Not allowed | Rel-15 |  | C231 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | NG-SS only |  |  |
|  | 1.3 | Triggered by user; UICC sends 90 00 | Rel-15 |  | C231 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | NG-SS only |  |  |
|  | 1.4 | Triggered by user; UICC sends 93 00 | Rel-15 |  | C231 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | NG-SS only |  |  |
|  | 1.5 | Triggered by user; allowed with modification of SM PDU DN request container | Rel-15 |  | C234 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | NG-SS only |  |  |
|  | 1.6 | Triggered by user; allowed with modification of ePCO | Rel-15 |  | C231 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | NG-SS only |  |  |
|  | 1.7 | Triggered by OPEN CHANNEL | Rel-15 |  | C232 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | NG-SS only |  |  |
| 10.14 | ENVELOPE SMS-PP Data Download on NAS messages |  |  |  |  |  |  |  |  |
| 10.14.1 | Routing Indicator Data update via DL NAS TRANSPORT messages |  |  |  |  |  |  |  |  |
|  | 1.1 | "acknowledgement not requested" and "re-registration not requested" | Rel-15 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
|  | 1.2 | "acknowledgement not requested" and "re-registration requested" | Rel-15 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
|  | 1.3 | "acknowledgement requested" and "re-registration requested" | Rel-15 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
|  | 1.4 | "acknowledgement requested" and "re-registration not requested" | Rel-15 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
| 10.14.2 | Steering of Roaming via DL NAS TRANSPORT message |  |  |  |  |  |  |  |  |
|  | 2.1 | REFRESH command [Steering of Roaming] | Rel-15 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
|  | 2.3 | "Acknowledgement requested" and REFRESH command [Steering of Roaming] | Rel-15 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
|  | 2.4 | Long message in several ENVELOPE commands with REFRESH command [Steering of Roaming] | Rel-16 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
| 10.14.3 | Steering of Roaming via REGISTRATION ACCEPT message |  |  |  |  |  |  |  |  |
|  | 3.1 | REFRESH command [Steering of Roaming] | Rel-15 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
|  | 3.3 | Long message with REFRESH command [Steering of Roaming] | Rel-15 |  | C231 | E.1/24 AND E.1/2 | NG-SS only |  |  |
| NOTE: Blank entries indicate the latest valid release at the time of publication of this specification |  |  |  |  |  |  |  |  |  |

| C101 | IF A.1/1 THEN M ELSE N/A | -- O_Cap_Conf |
| --- | --- | --- |
| C102 | void |  |
| C103 | void |  |
| C104 | IF A.1/2 THEN M ELSE N/A | -- O_Sust_text |
| C105 | IF A.1/3 AND A.1/41 THEN M ELSE N/A | -- O_Ucs2_Entry AND O_UCS2_Cyrillic |
| C106 | IF A.1/4 THEN M ELSE N/A | -- O_Ext_Str |
| C107 | IF A.1/5 THEN M ELSE N/A | -- O_Help |
| C108 | IF A.1/6 THEN O.1 ELSE N/A | -- O_Icons |
| C109 | IF A.1/7 THEN M ELSE N/A | -- O_Dual_Slot |
| C110 | IF A.1/9 AND A.1/46 THEN M ELSE N/A | -- O_Run_At AND O_+CIMI |
| C111 | IF (A.1/10 OR E.1/71) THEN M ELSE N/A | -- O_LB |
| C112 | IF A.1/11 THEN M ELSE N/A | -- O_Soft_key |
| C113 | void |  |
| C114 | IF C110 AND C108 THEN O.1 ELSE N/A | -- O_Run_At AND O_+CIMI AND O_Icons |
| C115 | IF C111 AND C108 THEN M ELSE N/A | -- O_LB AND O_Icons |
| C116 | IF A.1/7 AND A.1/8 THEN M ELSE N/A | -- O_Dual_Slot AND O_Detach_Rdr |
| C117 | void |  |
| C118 | IF A.1/15 AND A.1/41 THEN M ELSE N/A | -- O_Ucs2_Disp AND O_UCS2_Cyrillic |
| C119 | IF A.1/19 THEN M ELSE N/A | -- O_Redial |
| C120 | IF A.1/20 THEN M ELSE N/A | -- O_D_NoResp |
| C121 | IF A.1/21 AND A.1/17 THEN M ELSE N/A | -- O_BIP_GPRS AND O_UDP |
| C122 | IF C111 AND A.1/16 THEN M ELSE N/A | -- O_LB AND O_GPRS |
| C123 | void |  |
| C124 | IF A.1/22; test x.A M ELSE x.B M (where x is the expected sequence number value) | -- O_CP_Subaddr |
| C125 | IF A.1/23 THEN M ELSE N/A | -- O_Imm_Resp |
| C126 | IF A.1/24 THEN M ELSE N/A | -- O_Duration |
| C127 | void |  |
| C128 | void |  |
| C129 | void |  |
| C130 | void |  |
| C131 | void |  |
| C132 | IF A.1/27 THEN M ELSE N/A | -- O_BIP_Local |
| C133 | void |  |
| C134 | IF A.1/38 THEN M ELSE N/A | -- O_MMS |
| C135 | void |  |
| C136 | void |  |
| C137 | void |  |
| C138 | void |  |
| C139 | IF A.1/35 THEN M ELSE N/A | -- O_Batt |
| C140 | IF A.1/39 THEN M ELSE N/A | -- O_UC_Before_EnvCC |
| C141 | IF A.1/40 THEN M ELSE N/A | -- O_UC_After_EnvCC |
| C142 | IF A.1/3 AND A.1/42 THEN M ELSE N/A | -- O_UCS2_Entry AND O_UCS2_Chinese |
| C143 | IF A.1/15 AND A.1/42 THEN M ELSE N/A | -- O_UCS2_Disp AND O_UCS2_Chinese |
| C144 | IF A.1/3 AND A.1/43 THEN M ELSE N/A | -- O_UCS2_Entry AND O_UCS2_Katakana |
| C145 | IF A.1/15 AND A.1/43 THEN M ELSE N/A | -- O_UCS2_Disp AND O_UCS2_Katakana |
| C146 | IF A.1/45 THEN M ELSE N/A | -- O_FDN |
| C147 | IF A.1/44 THEN M ELSE N/A | -- O_BDN |
| C148 | IF A.1/9 AND A.1/47 THEN M ELSE N/A | -- O_Run_At AND O_+CGMI |
| C149 | IF C148 AND C118 THEN M ELSE N/A | -- O_Run_At AND O_+CGMI AND O_Ucs2_Disp AND O_Ucs2_Cyrillic |
| C150 | IF C148 AND C143 THEN M ELSE N/A | -- O_Run_At AND O_+CGMI AND O_Ucs2_Disp AND O_Ucs2_Chinese |
| C151 | IF C148 AND C145 THEN M ELSE N/A | -- O_Run_At AND O_+CGMI AND O_Ucs2_Disp AND O_Ucs2_Katakana |
| C152 | IF C121 AND A.1/49 THEN M ELSE N/A | -- O_BIP_GPRS AND O_UDP AND O_BUFFER_SIZE |
| C153 | IF A.1/50 THEN M ELSE N/A | -- O_TAT_AL |
| C154 | IF A.1/51 THEN M ELSE N/A | -- O_TAT_AC |
| C155 | IF A.1/52 THEN M ELSE N/A | -- O_TAT_AR |
| C156 | IF A.1/53 THEN M ELSE N/A | -- O_TAT_FSN |
| C157 | IF A.1/54 THEN M ELSE N/A | -- O_TAT_FSL |
| C158 | IF A.1/55 THEN M ELSE N/A | -- O_TAT_FSS |
| C159 | IF A.1/56 THEN M ELSE N/A | -- O_TAT_SN |
| C160 | IF A.1/57 THEN M ELSE N/A | -- O_TAT_SB |
| C161 | IF A.1/58 THEN M ELSE N/A | -- O_TAT_SI |
| C162 | IF A.1/59 THEN M ELSE N/A | -- O_TAT_SU |
| C163 | IF A.1/60 THEN M ELSE N/A | -- O_TAT_SS |
| C164 | IF A.1/61 THEN M ELSE N/A | -- O_TAT_STFC |
| C165 | IF A.1/62 THEN M ELSE N/A | -- O_TAT_STFB |
| C166 | IF A.1/63 THEN test step option n.A M ELSE test step option n.B M | -- O_longFTN |
| C167 | IF A.1/64 THEN M ELSE N/A | -- O_GERAN |
| C168 | IF A.1/65 THEN M ELSE N/A | -- O_Global_PB |
| C169 | IF (C121 AND A.1/68 THEN test x.A M ELSE IF (C121 AND NOT A.1/68) test x.B M ELSE N/A | -- (O_BIP_GPRS AND O_UDP AND O_User_Confirm_Before_PDP_Context_Request) OR (O_BIP_GPRS AND O_UDP AND NOT O_User_Confirm_Before_PDP_Context_Request) |
| C170 | IF A.1/69 THEN M ELSE N/A | -- O_Serv_SS_HOLD |
| C171 | IF A.1/6 THEN O.2 ELSE N/A | -- O_Icons |
| C172 | IF A.1/6 THEN O.4 ELSE N/A | -- O_Icons |
| C173 | IF C110 AND A.1/6 THEN O.2 ELSE N/A | -- O_Run_At AND O_+CIMI AND O_Icons |
| C174 | IF A.1/78 AND A.1/79 THEN M ELSE N/A | -- O_AddInfo_SS AND O_Serv_SS_CFU |
| C175 | IF A.1/78 AND A.1/80 THEN M ELSE N/A | -- O_AddInfo_SS AND O_Serv_SS_CLIR |
| C176 | IF A.1/44 THEN N/A ELSE M | -- O_BDN |
| C177 | IF A.1/84 THEN M ELSE N/A | -- O_No_Type_ND |
| C178 | IF A.1/85 THEN M ELSE N/A | -- O_No_Type_NK |
| C179 | IF A.1/86 THEN M ELSE N/A | -- O_No_Type_NA |
| C180 | IF A.1/87 THEN M ELSE N/A | -- O_No_Type_NS |
| C181 | IF A.1/88 THEN M ELSE N/A | -- O_No_Type_NL |
| C182 | IF A.1/18 AND (A.1/132 OR A.1/133) THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD) |
| C183 | IF (NOT A.1/135) AND (A.1/64 OR A.1/134) THEN M ELSE N/A | -- (NOT O_EUTRAN_NO_UTRAN_NO_GERAN) AND (O_GERAN OR O_UTRAN) |
| C184 | IF A.1/134 THEN M ELSE N/A | -- O_UTRAN |
| C185 | IF A.1/6 AND A.1/111 THEN M ELSE N/A | -- O_Icons AND O_Icon_Rec1_Send_SS |
| C186 | IF A.1/6 AND A.1/115 THEN M ELSE N/A | -- O_Icons AND O_Icon_Rec2_Send_USSD |
| C187 | IF A.1/6 AND A.1/114 THEN M ELSE N/A | -- O_Icons AND O_Icon_Rec1_Send_USSD |
| C188 | IF A.1/6 AND A.1/120 THEN M ELSE N/A | -- O_Icons AND O_Icon_Rec1_Set_Up_Idle_Mode_Text |
| C189 | IF C110 AND A.1/6 AND A.1/123 THEN M ELSE N/A | -- O_Run_At AND O_+CIMI AND O_Icons AND O_Icon_Rec1_Run_AT_Cmd |
| C190 | IF (A.1/139 OR A.1/140) THEN M ELSE N/A | -- pc_eTDD OR pc_eFDD |
| C191 | IF A.1/21 AND A.1/18 THEN M ELSE N/A | -- O_BIP_GPRS AND O_TCP |
| C192 | IF A.1/21 AND A.1/18 AND A.1/72 THEN M ELSE N/A | -- O_BIP_GPRS AND O_TCP AND O_TCP_UICC_ServerMode |
| C193 | IF (A.1/10 OR (E.1/71 AND E.1/42)) AND A.1/193 THEN M ELSE N/A | -- O_LB AND O_Browser_Termination |
| C194 | IF A.1/138 THEN M ELSE N/A | -- O_Select_Item_Default_Item |
| C195 | IF A.1/137 THEN M ELSE N/A | -- O_CSG_Cell_Discovery |
| C196 | IF A.1/142 AND (A.1/139 OR A.1/140) THEN M ELSE N/A | -- pc_MO_SM-over-IMS AND (pc_eFDD OR pc_eTDD) |
| C197 | IF A.1/142 AND A.1/134 AND A.1/194 THEN M ELSE N/A | -- pc_MO_SM-over-IMS AND O_UTRAN AND O_IMS_UTRAN |
| C198 | IF A.1/141 AND (A.1/139 OR A.1/140) THEN M ELSE N/A | -- pc_SM-over-IP-receiver AND (pc_eFDD OR pc_eTDD) |
| C199 | IF A.1/141 AND A.1/134 AND A.1/194 THEN M ELSE N/A | -- pc_SM-over-IP-receiver AND O_UTRAN AND O_IMS_UTRAN |
| C200 | IF A.1/136 THEN M ELSE N/A | -- O_Event_CSG_Cell_Selection |
| C201 | IF A.1/64 AND A.1/149 THEN M ELSE N/A | -- O_GERAN AND O_SMS-CB_Data_Download |
| C202 | IF (A.1/139 OR A.1/140) AND A.1/150 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD) AND O_IMS |
| C203 | IF A.1/134 AND A.1/150 THEN M ELSE N/A | -- O_UTRAN AND O_IMS |
| C204 | IF A.1/151 THEN N/A ELSE M | -- O_PS_OPMODE |
| C205 | IF (A.1/139 OR A.1/140) AND A.1/152 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD) AND O_SMS_SGs_MT |
| C206 | IF (A.1/139 OR A.1/140) AND A.1/153 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD) AND O_SMS_SGs_MO |
| C207 | IF A.1/147 AND A.1/148 AND A.1/150 THEN M ELSE O | -- O_Event_IMS_Registration AND O_UICC_ACCESS_IMS AND O_IMS |
| C208 | IF A.1/146 AND A.1/147 AND A.1/148 AND A.1/150 THEN M ELSE N/A | -- O_Event_Incoming_IMS_Data AND O_Event_IMS_Registration AND O_UICC_ACCESS_IMS AND O_IMS |
| C209 | IF (A.1/157 OR A.1/159) THEN M ELSE N/A | -- (pc_SMS_CS_MO OR pc_SMS_PS_MO) |
| C210 | IF (NOT A.1/135) AND (A.1/64 OR A.1/134) AND (A.1/157 OR A.1/159) THEN M ELSE N/A | -- (NOT (O_EUTRAN_NO_UTRAN_NO_GERAN) AND (O_GERAN OR O_UTRAN)) AND (pc_SMS_CS_MO OR pc_SMS_PS_MO) |
| C211 | IF (A.1/156 OR A.1/158) THEN M ELSE N/A | -- (pc_SMS_CS_MT OR pc_SMS_PS_MT) |
| C212 | IF (NOT A.1/135) AND (A.1/64 OR A.1/134) AND (A.1/156 OR A.1/158) THEN M ELSE N/A | -- (NOT (O_EUTRAN_NO_UTRAN_NO_GERAN) AND (O_GERAN OR O_UTRAN)) AND (pc_SMS_CS_MT OR pc_SMS_PS_MT) |
| C213 | IF (NOT A.1/160) THEN M ELSE N/A | -- NOT O_Rej_Launch_Browser_withDefURL |
| C214 | IF A.1/160 THEN M ELSE N/A | -- O_Rej_Launch_Browser_withDefURL |
| C215 | IF A.1/16 THEN M ELSE N/A | -- O_GPRS |
| C216 | IF A.1/161 THEN M ELSE N/A | -- O_Lang_Select |
| C217 | IF A.1/162 THEN M ELSE N/A | -- O_Provide_Local_LS |
| C218 | IF A.1/163 THEN M ELSE N/A | -- O_Lang_Notif |
| C219 | IF A.1/164 THEN M ELSE N/A | -- O_Refresh_AlphaIdentifier |
| C220 | IF (A.1/139 OR A.1/140 OR A.1/173) AND A.1/153 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD OR pc_NB) AND O_SMS_SGs_MO |
| C221 | IF (A.1/139 OR A.1/140 OR A.1/173) AND A.1/152 THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD OR pc_NB) AND O_SMS_SGs_MT |
| C222 | IF (A.1/139 OR A.1/140 OR A.1/173) THEN M ELSE N/A | -- pc_eTDD OR pc_eFDD OR pc_NB |
| C223 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) |
| C224 | IF A.1/18 AND A.1/178 AND (A.1/132 OR A.1/133 OR A.1/177) THEN M ELSE N/A | -- O_TCP AND pc_Multiple_PDN AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) |
| C225 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) AND A.1/182 THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) AND O_PSM_SUSPEND_UICC |
| C226 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) AND A.1/181 THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) AND O_PSM_DEAC_UICC |
| C227 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) AND A.1/183 THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) AND O_eDRX_SUSPEND_UICC |
| C228 | IF (A.1/132 OR A.1/133) AND A.1/152 AND A.1/184 THEN M ELSE N/A | -- (pc_BIP_eFDD OR pc_BIP_eTDD) AND O_SMS_SGs_MT AND O_PS_Data_Off |
| C229 | IF (A.1/132 OR A.1/133 OR A.1/177) THEN M ELSE N/A | -- pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB |
| C230 | A.1/17 AND A.1/178 AND (A.1/132 OR A.1/133 OR A.1/177) THEN M ELSE N/A | -- O_UDP AND pc_Multiple_PDN AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) |
| C231 | IF A.1/187 THEN M ELSE N/A | -- pc_NG_RAN |
| C232 | IF (A.1/187 AND A.1/188) THEN M ELSE N/A | -- pc_NG_RAN AND pc_BIP_NG_RAN |
| C233 | IF (A.1/191) THEN M ELSE N/A | --O_SUPI_NAI |
| C234 | IF A.1/187 AND A.1/195 THEN M ELSE N/A | -- pc_NG_RAN AND O_Set_DN_Specific_ID |
| C235 | IF (A.1/187 AND A.1/196) THEN M ELSE N/A | -- pc_NG_RAN AND pc_CAG |

| O.1 | IF A.1/zz tests x.yA M ELSE tests x.yB M (where zz corresponds to the option relating to the command being tested (e.g. A.1/90 if Display Text supports icons as defined in record 1 of EF(IMG)) and x.y is the expected sequence number value) |
| --- | --- |
| O.2 | IF A.1/zz tests x.yA M ELSE tests x.yB M (where zz corresponds to the option relating to the command being tested (e.g. A.1/91 if Display Text supports icons as defined in record 2 of EF(IMG)) and x.y is the expected sequence number value) |
| O.3 | void |
| O.4 | IF A.1/zz AND A.1/ww tests x.yA M ELSE tests x.yB M (where zz and ww correspond to the option relating to the command being tested (e.g. A.1/90 if Display Text supports icons as defined in record 1 of EF(IMG) and A.1.92 if Display Text supports icons as defined in record 5 of EF(IMG) ) and x.y is the expected sequence number value) |

| TCEP001 | IF NOT A.1/84 THEN during the test execution; the display or the non-display of any alpha identifier; text string or icon shall be treated as successfully verified. |
| --- | --- |
| TCEP002 | IF NOT A.1/85 THEN the terminal may open the channel without explicit confirmation by the user. |
| TCEP003 | If A.1/181 and/or A.1/182 is supported; in addition to the test case initial conditions; any specific information or particular UE configurations required to ensure that the UE performs UICC deactivation/suspension in PSM shall be provided by the UE manufacturer |
| TCEP004 | If A.1/183 is supported; in addition to the test case initial conditions; any specific information or particular UE configurations required to ensure that the UE suspends the UICC in eDRX shall be provided by the UE manufacturer |

| AER001 | IF ((A.1/21 AND A.1/17) AND ((A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(10.4.27.6; Seq. 6.1) ELSE A | -- (O_BIP_GPRS AND O_UDP) AND (pc_BIP_eFDD OR pc_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |
| --- | --- | --- |
| AER002 | IF ((A.1/132 OR A.1/133 OR A.1/173) AND (A.1/134 OR A.1/64))) THEN R(27.22.7.4 Seq. 1.2) ELSE A | -- (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_NB) AND (O_GERAN OR O_UTRAN) |
| AER003 | IF ((A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(10.4.15 Seq. 1.17) ELSE A | -- (pc_BIP_eFDD OR pc_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |
| AER004 | IF ((A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(10.4.15 Seq. 1.14) ELSE A | -- (pc_BIP_eFDD OR pc_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |
| AER005 | IF ((A.1/21 AND A.1/17) AND ((A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(10.4.27.6; Seq. 6.4) ELSE A | -- (O_BIP_GPRS AND O_UDP) AND (pc_BIP_eFDD OR pc_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |
| AER006 | IF ((A.1/21 AND A.1/17) AND ((A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(10.4.27.6; Seq. 6.3) ELSE A | -- (O_BIP_GPRS AND O_UDP) AND (pc_BIP_eFDD OR pc_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |
| AER007 | IF ((A.1/21 AND A.1/17) AND ((A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(10.4.27.6; Seq. 6.5) ELSE A | -- (O_BIP_GPRS AND O_UDP) AND (pc_BIP_eFDD OR pc_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |
| AER008 | IF ((A.1/21 AND A.1/17) AND ((A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(10.4.29; Seq. 1.2) ELSE A | -- (O_BIP_GPRS AND O_UDP) AND (pc_BIP_eFDD OR pc_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |

# 4	Test environment

## 4.0	General Test purpose

Testing of functional conformance to USIM Application Toolkit commands, including proactive UICC commands when implemented on a nrUSIM.

All facilities given by the TERMINAL PROFILE as supported, for which tests exist in the present document, shall be tested. Many of the proactive UICC commands include an alpha identifier data object. This is intended to be a short one- or two-word identifier for the ME to optionally display on the screen along with any other indications, at the same time as the ME performs the UICC command.

NOTE:	The sequence of USIM Application Toolkit commands is specific to the Toolkit Application being executed within the nrUSIM, hence sequential testing of commands is not possible. The testing will therefore have to be performed on a command-by-command basis controlled by the TT.

## 4.1	Test environment description

### 4.1.1	General test environment

Without having the UICC-Terminal interface accessible a direct verification of APDU/data timing and contents is not possible. Thus, alternative implementations and methods will be used to provide sufficient confidence in the result obtained. The present document will not specify an authoritative test environment. The following figure shows a test environment that allows the verification of test results for UEs with an integrated and not removable UICC/USIM (nrUSIM).

Without having the UICC-Terminal interface accessible, a direct verification of APDU or data contents is not possible. The present document shall provide a test environment and test methods that allow the verification of test results for UEs with an integrated and not removable UICC/USIM (nrUSIM).

Figure 4.1: General test environment

Figure 4.1 gives an overview on how a test case shall be executed.

Based on the identified test purpose and the related conformance requirements an appropriate test sequence is defined. The test itself can be split into three phases:

-	In the preparation phase the initial set-up for the test case is performed. Test specific data is transferred to the nrUSIM. E.g. by provisioning a test specific profile, provisioning test specific proactive command data etc.

-	In the execution phase the test procedure is performed. It has to be ensured that all steps defined in the test procedure are executed and that they are executed in order. The data generated during this execution is stored in the TT and/or test EFs in the file system within the nrUICC.

-	In the verification phase the data, procedures and processes identified and stored during execution are checked against given conformance requirements. A final verification of specific EF/DF contents might be required. The exchange of required data has to be ensured, even though the required data transfer is not necessarily listed in the test procedure.

### 4.1.2	Example - test environment for contents verification

Contents verification within the scope of the present document describes a procedure that allows the TT to compare contents of EFs/DFs available on the EUT with expected values defined in the present document.

Example:

-	During preparation phase specific content for the EFFPLMN is updated in the nrUSIM;

-	During execution of the test the FPLMN value is modified;

-	As the FPLMN value stored in EFFPLMN is not transferred to the TT during test execution;

-	The TT performs a read procedure on EFFPLMN at test case end (not necessarily part of the test procedure);

-	The TT compares the value read with the expected value stored in the TT.

The contents verification method described here is not applicable when a verification of EF/DF contents needs to be performed at a time other than the preparation or the verification phase.

NOTE 1:	It is not expected that a reading procedure on EF contents can be performed whilst the test procedure is executed.

The test environment needed to perform contents verification is identical to the test environment shown for implicit testing.

NOTE 2:	A test toolkit applet can be used for testing USAT specific test cases in the present document and EF content verification method is not applicable for verifying USAT specific requirements.

### 4.1.3	Example - test environment for seamless testing

Seamless testing within the scope of the present document relies on monitoring of data transfer between ME and nrUSIM traced by a software interface between nrUSIM and baseband implemented by the ME vendor.

The logged communication can be transferred to the TT and will be used to determine if conformance requirements are met.

NOTE:	A test toolkit applet can be used for testing USAT specific test cases in the present document and TT shall be able to trigger proactive commands using the applet if required by the test. Refer to Annex A.1 for examples of test EFs required for the Applet.

Figure 4.2: Test environment for seamless testing

### 4.1.4	Example – test environment for test toolkit events based testing

Test toolkit events based testing within the scope of the present document is applicable for UEs supporting the required USIM application toolkit functionality. A toolkit applet is installed onto the nrUSIM, capable of handling test events internal to the card runtime environment and the applets to monitor APDUs received at the nrUICC.

NOTE 1:	The same test toolkit applet can be used for testing USAT specific test cases in the present document and TT shall be able to trigger proactive commands using the applet if required by the test. Refer to Annex A.2 for examples of test EFs required for the Applet.

The logged events can be stored in a test EF during the test execution phase and transferred to the TT during the verification phase to determine if conformance requirements are met.

NOTE 2:	A test toolkit applet can be used for testing USAT specific test cases in the present document and EF content verification method is not applicable for verifying USAT specific requirements.

Figure 4.4: Test environment for test toolkit events based testing

Please see the required "Test configuration state" in Annex N and the "Examples of test configuration usage" in Annex O of ETSI TS 102 221 [8]

## 4.2	Requirements to the EUT and the test environment

### 4.2.1	General Requirements

All tests defined in the subsequent clauses apply to UEs operating an nrUSIM/nrUICC, what implies that the UICC-Terminal interface cannot be exposed to a TT. Thus, direct tracing and explicit verification of APDUs or data sent via the UICC-Terminal interface is not possible. As explicit verification of APDUs, data or file contents is needed to verify a conformance requirement the implementation and applicability has to be declared (see also clause 3.7.1).

The tests are not applicable for UEs providing a UICC-Terminal interface in accordance with interface form factors defined in ETSI TS 102 221 [9] or ETSI TS 102 671 [29].

The following sequence of tests confirms:

a)	the usage of the test specific UICC/USIM data;

b)	the correct interpretation of data read from the USIM (Universal Subscriber Identification Module) by the ME;

c)	the correct writing of data to the USIM by the ME;

d)	the initiation of appropriate procedures by the ME;

e)	the correct execution of functions

All tests apply to the USIM application on the UICC or an equivalent application implemented in accordance with the ETSI SSP specifications ETSI TS 103 666-1 [10], ETSI TS 103 666-2 [11] and ETSI TS 103 666-3 [12]

For the proactive command test cases in clause 10 and its subclauses:

-	a test toolkit applet shall be used for verifying test case specific proactive commands and TT shall be able to trigger proactive commands using the applet as required by the test,

and

-	one or more test specific EFs may be required to program data required for the proactive commands.

### 4.2.2	Requirements to the UE (EUT) – supported interfaces

The EUT has to support interfaces and administration methods to allow the TT or the User to set the initial conditions defined for the test cases. Where the EUT may be solely the UE or the UE connected to an interface device.

NOTE:	The connection of the UE and an interface device is set up similar to what is described in the Companion Device scenario in GSMA SGP.22 [30], but not limited to remote SIM provisioning functionality.

Suggested interfaces:

-	Wi-Fi (IEEE 802.11-2016 [13])

-	USB (USB-IF, USB 2.0 or higher)

The UE has to support at least one of the following data transfer methods:

-	RFM and OTA via ETSI TS 102 225 [14] and ETSI TS 102 226 [15]

-	AT commands as defined in TS 27.007 [16]

-	JavaTMCard as defined in TS 31.130 [17]

Even though the availability of at least one of the suggested interfaces and one of the suggested data transfer methods is recommended, interfaces and methods for UICC/USIM preparation for testing purposes are to be provided by the UE vendor and are out of scope of the present document.

### 4.2.3	Supported RATs

UEs tested in accordance with the present document shall support any 3GPP defined RATs. Test cases defined in the present document may contain RAT or RAN specific conformance requirements and methods of test. The applicability of the individual test cases can be determined by using Table B.1.

Tests that would require 2G network access (GERAN) an 3G network access (UTRAN) are out of scope of the present document.

### 4.2.4	Initial and final procedure steps

Initial and final procedure steps are out of scope of the current specification to not explicitly exclude methods that might be used to get a UE set up at test case start or to have it 'cleaned up' at the end of a test (if required).

The testing person has to ensure that the UE has installed test specific the UICC and USIM data in accordance with the definitions of the particular test (see note). The UE has to be accessible and is brought into a specific reception mode if required by the test case. It has to be ensured that interfaces that are used to verify file data or a specific functionality do not interfere or block the operation of the test procedure as defined within this specification.

For verification purposes an interaction with the UE is allowed even after ending the defined test procedure. If such interaction takes place it has to be reported to the TT. A verification of conformance requirements that is based on data or information that is generated during this post-procedure interaction has to be identifiable as a post process but can be used for the result generation.

NOTE:	For all EFs, DFs and ADFs building the UICC and USIM where no data is explicitly defined in the test or by reference, an appropriate test value can be used (e.g.: values from GSMA TS.48 [9]).

## 4.3	Suitability assessment

Verification by "implicit" methods is not applicable for the USAT conformance requirements defined in the present document. Either the seamless testing method (A.4/2) defined in clause 4.1.3 or the test events-based method (A.4/1) defined in clause 4.1.4 depending on the device and nrUICC capabilities shall be used for verifying conformance requirements defined in the present document.

## 4.4	Definition of nrUICC values and System Simulator parameters for USAT testing

### 4.4.1	Introduction

#### 4.4.1.1	Installation, provisioning or modification methods for EFs and DFs

Installation, provisioning or modification methods for EFs and DFs defined in the UICCs/USIMs used for testing or in the test cases are out of scope of the present document. Respective methods have to be provided by the UE vendor.

#### 4.4.1.2	GSMA TS.48 Version and usage

Unless stated differently in the test description or a specific UICC definition, GSMA TS.48 eSIM GTP v5.0 is used.

The usage of file values defined in GSMA TS.48 [9] does not imply that remote SIM provisioning as defined in GSMA or profiles as defined by the TCA have to be supported by the nrUSIM.

### 4.4.2	Definition of default values for USAT testing

#### 4.4.2.1	Applications on the default nrUICC

The default configuration of the nrUICC shall host at least one USIM application.

#### 4.4.2.2	Definition of USIM default values

The USIM application shall be configured as defined in GSMA TS.48 [9] with the following exceptions to file definitions:

EFUST (USIM Service Table)

Logically:

| Service n°1: | Local Phone Book | available |
| --- | --- | --- |
| Service n°2: | Fixed Dialling Numbers (FDN) | available |
| Service n°6: | Barred Dialling Numbers (BDN) | available |
| Service n°10: | Short Message Storage (SMS) | available |
| Service n°11: | Short Message Status Reports (SMSR) | available |
| Service n°12: | Short Message Service Parameters (SMSP) | available |
| Service n°15: | Cell Broadcast Message Identifier | available |
| Service n°17: | Group Identifier Level 1 | not available |
| Service n°18: | Group Identifier Level 2 | not available |
| Service n°20: | User controlled PLMN selector with Access Technology | available |
| Service n°22: | Image (IMG) | available |
| Service n°27: | GSM Access | available |
| Service n°28: | Data download via SMS-PP | available |
| Service n°29: | Data download via SMS-CB | available |
| Service n°30: | Call Control by USIM | not available |
| Service n°31: | MO-SMS Control by USIM | not available |
| Service n°32: | RUN AT COMMAND command | available |
| Service n°33: | shall be set to '1' | available |
| Service n°34: | Enabled Services Table | available |
| Service n°85 | EPS Mobility Management Information | available |
| Service n°86 | Allowed CSG Lists and corresponding indications | not available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| binary | xx1x xx11 | x1xx 111x | xx1x 1x00 | 1001 11xx | xxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 |  |  |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx01 xxxx |  |  |  |  |  |

The coding of EFUST shall conform with the capabilities of the USIM used.

EFEST (Enabled Services Table)

Logically:

| Service n°1: | Fixed Dialling Numbers (FDN) |
| --- | --- |
| Service n°2: | Barred Dialling Numbers (BDN) |
| Service n°3: | APN Control List (ACL) |

Coding:

| Byte | B1 |
| --- | --- |
| Hex | 00 |

EFIMSI (International Mobile Subscriber Identity)

Logically:

Length:	8 bytes

IMSI:	001 01 0123456789

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 08 | 09 | 10 | 10 | 10 | 32 | 54 | 76 | 98 |

EFAD (Administrative Data)

Logically:

Type approval operations

OFM to be deactivated by the Terminal

MNC:	2 digit

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 80 | 00 | 00 | 02 |

EFLOCI (Location Information)

Logically:

LAI-MCC:	001

LAI-MNC:	01

LAI-LAC:	0001

TMSI:	"FF .. FF"

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | FF | FF | FF | FF | 00 | F1 | 10 | 00 | 01 | FF | 00 |

EFEPSLOCI (EPS Information)

Logically:

GUTI:				0010100010266341122

Last visited registered TAI:	001/01/0001

EPS update status:		not updated

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 0B | F6 | 00 | F1 | 10 | 00 | 01 | 02 | 66 | 43 | 11 | 22 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |  |  |  |  |  |  |
|  | 00 | F1 | 10 | 00 | 01 | 01 |  |  |  |  |  |  |

EFEPSNSC (EPS NAS Security Context)

Logically:

Key Set Identifier KSIASME:		'07' (no key available)

ASME Key (KSIASME):			32 byte key, any value

Uplink NAS count:			'00'

Downlink NAS count:			'00'

Identifiers of selected NAS integrity
and encryption algorithm:		'FF'

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | .. | .. | B39 | B40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 34 | 80 | 01 | 07 | 81 | 20 | xx | .. | .. | xx | 82 |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 | B50 | B51 | B52 |
|  | 04 | 00 | 00 | 00 | 00 | 83 | 04 | 00 | 00 | 00 | 00 | 84 |
|  | B53 | B54 |  |  |  |  |  |  |  |  |  |  |
|  | 01 | FF |  |  |  |  |  |  |  |  |  |  |

EFCBMI (Cell Broadcast Message Identifier)

Logically:

Cell Broadcast Message Identifier 1:	'03 E7'

Coding:

| Byte | B1 | B2 | B3 | .. | Bx |
| --- | --- | --- | --- | --- | --- |
| Hex | 03 | E7 | FF | .. | FF |

EFCBMID (Cell Broadcast Message Identifier for Data Download)

Logically:

Cell Broadcast Message Identifier 1:	'10 01'

Coding:

| Byte | B1 | B2 | B3 | .. | Bx |
| --- | --- | --- | --- | --- | --- |
| Hex | 10 | 01 | FF | .. | FF |

EFFDN (Fixed Dialling Numbers)

Logically:

Record 1:

Length of alp ha identifier:	6 characters;

Alpha identifier:		"FDN111";

Length of BCD number:	"03";

TON and NPI:			Telephony and unknown;

Dialled number:		123;

CCI:				None;

Ext2:				None.

Coding for record 1:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 46 | 44 | 4E | 31 | 31 | 31 | 03 | 81 | 21 | F3 | FF | FF |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | FF | FF | FF | FF | FF | FF | FF | FF |

Record 2:

Length of alpha identifier:	6 characters;

Alpha identifier:		"FDN222";

Length of BCD number:	"03";

TON and NPI:			Telephony and Unknown;

Dialled number:		9876;

CCI:				None;

Ext2:				None.

Coding for record 2:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 46 | 44 | 4E | 32 | 32 | 32 | 03 | 81 | 89 | 67 | FF | FF |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | FF | FF | FF | FF | FF | FF | FF | FF |

Record 3:

Length of alpha identifier:	6 characters;

Alpha identifier:		"FDN333";

Length of BCD number:	"0B";

TON and NPI:			Telephony and International;

Dialled number:		+12345678901234567890;

CCI:				None;

Ext2:				None.

Coding for record 3:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 46 | 44 | 4E | 33 | 33 | 33 | 0B | 91 | 21 | 43 | 65 | 87 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 09 | 21 | 43 | 65 | 87 | 09 | FF | FF |

EFBDN (Barred Dialling Numbers)

Logically:

Record 1:

Length of alpha identifier:		6 characters;

Alpha identifier:			"BDN111";

Length of BCD number:		"06";

TON and NPI:				Telephony and International;

Dialled number:			+1357924680;

CCI:					None;

Ext4:					None;

Comprehension method pointer:	None.

Coding for record 1:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 44 | 4E | 31 | 31 | 31 | 06 | 91 | 31 | 75 | 29 | 64 |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 |
|  | 08 | FF | FF | FF | FF | FF | FF | FF | FF |

Record 2:

Length of alpha identifier:		6 characters;

Alpha identifier:			"BDN222";

Length of BCD number:		"03";

TON and NPI:				Telephony and Unknown;

Dialled number:			122;

CCI:					None;

Ext4:					None;

Comprehension method pointer:	None.

Coding for record 2:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 44 | 4E | 32 | 32 | 32 | 04 | 81 | 21 | F2 | FF | FF |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF |

Record 3:

Length of alpha identifier:		6 characters;

Alpha identifier:			"BDN333";

Length of BCD number:		"03";

TON and NPI:				Telephony and Unknown;

Dialled number:			112;

CCI:					None;

Ext4:					None;

Comprehension method pointer:	None.

Coding for record 3:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 44 | 4E | 33 | 33 | 33 | 03 | 81 | 11 | F2 | FF | FF |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF |

EFECC (Emergency Call Codes)

Logically:

Emergency call code:			"122";

Emergency call code alpha identifier:	"TEST";

Emergency call Service Category:		RFU

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 21 | F2 | FF | 54 | 45 | 53 | 54 | 00 |

EFSMSS (SMS Status)

Logically:

Last used TP-MR set to"00".

Memory capacity available (flag unset b1="1").

Coding:

| Byte | B1 | B2 |
| --- | --- | --- |
| Hex | 00 | FF |

EFSMSP (Short message service parameters)

Logically:

Record 1:

Record length:			28 bytes

Parameter Indicators:

TP-Destination Address:	Parameter absent

TS-Service Centre Address:	Parameter present

TP-Protocol Identifier:		Parameter absent

TP-Data Coding Scheme:	Parameter absent

TP-Validity Period:		Parameter absent

TS-Service Centre Address:

TON:				International Number

NPI:				"ISDN / telephone numbering plan"

Dialled number string:		"112233445566778"

Coding for record 1:

| Byte | B1 | B2 | B3 | .. | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | FD | FF | FF | .. | FF | 09 | 91 | 11 | 22 | 33 | 44 | 55 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 |
|  | 66 | 77 | F8 | FF | FF | FF | FF | FF |

For the display of icon: See ETSI TS 102 384 [3] clause 27.22.1B.

NOTE:	For test sequences executed in accordance with the present specification, the listed values replace the configurations defined in TS 31.124 [2], clause 27.22.2A and 27.22.2B.1.

#### 4.4.2.3	Definition of DF_TELECOM default values

EFPSISMSC (Public Service Identity of the SM-SC)

1 record only.

Logically:

Record 1:

Public Service Identity of the SM-SC:		tel:+112233445566778

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 14 | 74 | 65 | 6C | 3A | 2B | 31 | 31 | 32 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | … | Bxx |
|  | 37 | 38 | FF | FF | FF | FF | FF | FF | … | FF |

### 4.4.3	Definition of nrUICC values and System Simulator parameters for USAT testing - E-UTRAN/EPC

#### 4.4.3.1	Applications on the E-UTRAN/EPC nrUICC

The E-UTRAN/EPC nrUICC shall host at least one USIM application as defined in clause 4.4.3.2. 
If IMS access is required the nrUICC shall host an ISIM as defined in clause 4.4.3.3 in addition.

#### 4.4.3.2	Definition of E-UTRAN/EPC USIM values

The values of the E-UTRAN/EPC USIM are identical to the values defined for the default USIM in clause 4.4.2.2 of the present document.

NOTE:	For test sequences executed in accordance with the present specification, the listed values may replace the configurations defined in TS 31.124 [2], clause 27.22.2B.1.

#### 4.4.3.3	Definition of E-UTRAN/EPC ISIM values

The E-UTRAN/EPC ISIM shall allow IMS access and shall be configured with the following values:

EFAD (Administrative Data)

Logically:	Type approval operations

Coding:

| Byte | B1 | B2 | B3 |
| --- | --- | --- | --- |
| Hex | 80 | 00 | 00 |

EFIST (ISIM Service Table)

Logically:

| Service n°1: | P-CSCF address | available |
| --- | --- | --- |
| Service n°2 | Generic Bootstrapping Architecture (GBA) | not available |
| Service n°3 | HTTP Digest | not available |
| Service n°4 | GBA-based Local Key Establishment Mechanism | not available |
| Service n°5 | Support of P-CSCF discovery for IMS Local Break Out | not available |
| Service n°6 | Short Message Storage (SMS) | available |
| Service n°7 | Short Message Status Reports (SMSR) | available |
| Service n°8 | Support for SM-over-IP including data download via SMS-PP as defined in TS 31.111 [31] | available |

Coding:

| Byte | B1 |
| --- | --- |
| binary | 1110 0001 |

EFIMPI (IMS private user identity)

Logically:	001010123456789@test.3gpp.com

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 1D | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 74 | 65 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 73 | 74 | 2E | 33 | 67 | 70 | 70 | 2E | 63 | 6F |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 6D | FF | FF | FF | FF | FF | FF | FF | FF | FF |

EFDOMAIN (Home Network Domain Name)

Logically:	test.3gpp.com

Coding;

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 0D | 74 | 65 | 73 | 74 | 2E | 33 | 67 | 70 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 70 | 2E | 63 | 6F | 6D | FF | FF | FF | FF | FF |

EFIMPU (IMS public user identity)

Record 1:

Logically:	sip:001010123456789@ims.mnc246.mcc081.3gppnetwork.org

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 35 | 73 | 69 | 70 | 3A | 30 | 30 | 31 | 30 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 31 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 39 | 40 | 69 | 6D | 73 | 2E | 6D | 6E | 63 | 32 |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 34 | 36 | 2E | 6D | 63 | 63 | 30 | 38 | 31 | 2E |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 | B50 |
|  | 33 | 67 | 70 | 70 | 6E | 65 | 74 | 77 | 6F | 72 |
|  | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 | B60 |
|  | 6B | 2E | 6F | 72 | 67 | FF | FF | FF | FF | FF |

Record 2:

Logically:	sip:+11234567890@test.3gpp.com

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 1E | 73 | 69 | 70 | 3A | 2B | 31 | 31 | 32 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 30 | 40 | 74 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 65 | 73 | 74 | 2E | 33 | 67 | 70 | 70 | 2E | 63 |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 6F | 6D | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 | B50 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 | B60 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

Record 3:

Logically:	tel:+11234567890

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 10 | 74 | 65 | 6C | 3A | 2B | 31 | 31 | 32 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 30 | FF | FF |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 | B50 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 | B60 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

EFP-CSCF (P-CSCF ADDRESS)

Logically:

Address Type:		FQDN

P-CSCF Address:	pcscf1.anyims.test.3gpp.com

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | 1C | 00 | 70 | 63 | 73 | 63 | 66 | 31 | 2E |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 61 | 6E | 79 | 69 | 6D | 73 | 2E | 74 | 65 | 73 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 74 | 2E | 33 | 67 | 70 | 70 | 2E | 63 | 6F | 6D |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

NOTE:	This EF does not apply for 3GPP and shall not be used by a terminal using a 3GPP access network or a 3GPP Interworking WLAN.

EFSMS (Short Message Service)

At least 10 records.

All records shall be empty.

Logically:	Status byte set to empty.

Coding Record 1-x (x ≥10):

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 | … | B176 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex: | 00 | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF | … | FF |

EFSMSR (Short message status reports)

This EF shall contain as many records as EFSMS.

All records shall be empty.

Logically:	Status byte set to empty.

Coding Record 1-x (x ≥10):

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 00 | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

EFSMSP (Short message service parameters)

Logically:

Record 1:

Record length:			28 bytes

Parameter Indicators:

TP-Destination Address:		Parameter absent

TS-Service Centre Address:	Parameter present

TP-Protocol Identifier:		Parameter absent

TP-Data Coding Scheme:	Parameter absent

TP-Validity Period:		Parameter absent

TS-Service Centre Address:

TON:				International Number

NPI:				"ISDN / telephone numbering plan"

Dialled number string:		"112233445566778"

Coding:

| Byte | B1 | B2 | B3 | ... | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | FD | FF | FF | ... | FF | 09 | 91 | 11 | 22 | 33 | 44 | 55 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 |  |  |  |  |
|  | 66 | 77 | F8 | FF | FF | FF | FF | FF |  |  |  |  |

All other records shall be empty.

EFSMSS (SMS Status)

Logically:

Last used TP-MR set to"00".

Memory capacity available (flag unset b1="1").

Coding:

| Byte | B1 | B2 |
| --- | --- | --- |
| Hex | FD | FF |

NOTE:	For test sequences executed in accordance with the present specification, the formerly listed values replace the is configuration defined in TS 31.124 [2], clause 27.22.2C.3

#### 4.4.3.4	Definition of E-UTRAN System Simulator parameters

The default E-UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) =	001;

-	Mobile Network Code (MNC) =	01;

-	Tracking Area Code (TAC) =	0001;

-	Cell Identity value =		0001.

The default EPS bearer context is defined in of TS 36.508 [5], clause 6.6.1 "Reference default EPS bearer context #1".

The default PDP type shall be "IP".

### 4.4.4	Definition of nrUICC values and System Simulator parameters for USAT testing - NG-RAN

#### 4.4.4.1	Applications of the NG-RAN nrUICC

The NG-RAN nrUICC shall host at least one USIM application as defined in clause 4.4.4.2, 4.4.4.3 or 4.4.4.4.
If the nrUICC configuration shall allow IMS access it shall host an ISIM as defined in clause 4.4.4.5 in addition

#### 4.4.4.2	Definition of NG-RAN USIM

The NG-RAN USIM application shall be configured as the default USIM in clause 4.4.2.2 of the present document with the following exceptions or additions:

EFUST (USIM Service Table)

Logically:

Settings for services n°1 to n°85 in EFUST are set as defined in clause 4.4.2.2. The following changes and additions apply:

| Service n°86 | Allowed CSG Lists and corresponding indications | available |
| --- | --- | --- |
| Service n°122 | 5GS Mobility Management Information | available |
| Service n°123 | 5G Security Parameters | available |
| Service n°124 | Subscription identifier privacy support | available |
| Service n°125 | SUCI calculation by the USIM | not available |

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| binary | xx1x xx11 | x1xx 111x | xx1x 1x00 | 1001 11xx | xxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 |  | B16 |  |  |  |
|  | xxxx xxxx | xxxx xxxx | xx01 xxxx | ... | xxx0 111x |  |  |  |

The coding of EFUST shall conform with the capabilities of the USIM used.

EF5GS3GPPLOCI (5GS 3GPP location information)

Logically:

5G-GUTI:			FF FF FF FF FF FF FF FF FF FF FF FF FF

TAI:			246 081 000000

5GS update status:	5U2 NOT UPDATED

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | FF | 42 | 16 | 80 | 00 | 00 | 00 | 01 |

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

Logically:

Protection Scheme Identifier List data object

Protection Scheme Identifier 1 – null

Key Index 1: 0

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 |
| --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | 02 | 00 | 00 | A1 | 00 |

EFRouting_Indicator (Routing Indicator EF)

Logically:

Routing Indicator: 17

Coding:

| Byte | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 71 | FF | 00 | 00 |

EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context EF)

Logically:

5GS NAS Security Context:

ngKSI:				00

KAMF:				32 bytes, value not checked

Uplink NAS count:		any value

Downlink NAS count:		any value

Identifiers of selected NAS:	any value

integrity and encryption algorithms:

Identifiers of selected EPS NAS:	any value

integrity and encryption algorithms

for use after mobility to EPS

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | Bx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | XX | 80 | 01 | 00 | 81 | xx | xx | … | xx |

NOTE:	For test sequences executed in accordance with the present specification, the formerly listed values replace the is configuration defined in TS 31.124 [2], clause 27.22.2D.1

#### 4.4.4.3	Definition of NG-RAN USIM supporting Rel-17 features

The NG-RAN USIM application supporting Rel-17 features shall be configured as the NG-RAN USIM in clause 4.4.4.2 of the present document with the following exceptions or additions:

EFUST (USIM Service Table)

Logically:

| Service n°1 to n°146 |  | defined | see coding |
| --- | --- | --- | --- |
| Service n°147 to n°152 |  | not defined | not available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 | B18 | B19 |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 111x | xxxx xxxx | xxxx xxxx | 0000 00xx |

NOTE:	For test sequences executed in accordance with the present specification, the formerly listed values replace the is configuration defined in TS 31.124 [2], clause 27.22.2D.3

#### 4.4.4.4	Definition of NG-RAN USIM supporting CAG

The NG-RAN USIM application supporting CAG features shall be configured as the NG-RAN USIM in clause 4.4.4.2 of the present document with the following exceptions or additions:

EFUST (USIM Service Table)

Logically:

| Service n°1 to n°136 |  | defined | see coding |
| --- | --- | --- | --- |
| Service n°137: |  | Preconfigured CAG information list | available |
| Service n°138 to 146 |  | defined | see coding |
| Service n°147 to n°152 |  | not defined | not available |

Coding:

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x | xxxx xxxx | xxxx 1x00 | xxxx x1xx | xxxx xx11 | xxxx xxxx | xxxx xxxx | xxxx xxxx |
|  | B9 | B10 | B11 | … | B16 | B17 | B18 | B19 |
|  | xxxx xxxx | xxxx xxxx | xx11 xxxx | ... | xxx0 111x | xxxx xxxx | xxxx xxx1 | 0000 00xx |

EFCAG (Pre-configured CAG information list)

Logically:

PLMN: 			244 083 (MCC MNC)

CAG only: 			1

Range indication: 	1

CAG-ID range: 		00 00 00 01 – 00 00 00 07

PLMN: 			244 084 (MCC MNC)

CAG only: 			1

Range indication:		1

CAG-ID range:		00 00 00 01 – 00 00 00 07

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 00 | 1A | 0C | 42 | 34 | 80 | 03 | 00 | 00 | 00 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 01 | 00 | 00 | 00 | 07 | 0C | 42 | 44 | 80 | 03 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 |  |  |
|  | 00 | 00 | 00 | 01 | 00 | 00 | 00 | 07 |  |  |

EFCAG (Pre-configured CAG information list)

Logically:

PLMN: 			244 083 (MCC MNC)

CAG only:		1

Range indication:		1

CAG-ID range:		00 00 00 01 – 00 00 00 07

PLMN:			244 084 (MCC MNC)

CAG only: 		1

Range indication: 	1

CAG-ID range:		00 00 00 01 – 00 00 00 07

Coding:

| Byte | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 00 | 1A | 0C | 42 | 34 | 80 | 03 | 00 | 00 | 00 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 01 | 00 | 00 | 00 | 07 | 0C | 42 | 44 | 80 | 03 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 |  |  |
|  | 00 | 00 | 00 | 01 | 00 | 00 | 00 | 07 |  |  |

NOTE:	For test sequences executed in accordance with the present specification, the formerly listed values replace the is configuration defined in TS 31.124 [2], clause 27.22.2D.4

#### 4.4.4.5	Definition of NG-RAN ISIM values

The NG-RAN ISIM shall contain an ISIM configuration as defined in clause 4.4.3.3 of the present document.

NOTE:	For test sequences executed in accordance with the present specification, the formerly listed values replace the is configuration defined in TS 31.124 [2], clause 27.22.2E.2

#### 4.4.4.6	Definition of NG-RAN System Simulator parameters

The default NG-RAN parameters of the system simulator are:

-	Mobile Country Code (MCC) =	001;

-	Mobile Network Code (MNC) =	01;

-	Tracking Area Code (TAC) =	000001;

-	Cell Identity value =		0001 (36 bits).

# 5	Testing methodology in general

When possible, the present document refers to ETSI TS 102 384 [3] to describe generic aspects of application toolkit tests

## 5.1	Testing of optional functions and procedures

Any function or procedure which is optional, as indicated in the present document, may be subject to a conformance test

if it is implemented in the ME.

## 5.2	Test interfaces and facilities

The UICC and NG-SS/NB-SS/E-USS interfaces provide the main test interfaces for the purpose of performing conformance tests.

The tests which require a network simulator shall be carried out with using a Next Generation System Simulator when accessing a NG-RAN, a NB System Simulator when accessing an E-UTRAN in NB-S1 mode, an Evolved Universal System Simulator when accessing an E-UTRAN in WB-S1 mode.

## 5.3	Information to be provided by the apparatus supplier

The information to be provided by the apparatus supplier specified in TS 38.508-1 [39], TS 36.523-2 [34], TS 36.508 [33], TS 34.108 [12] and TS 51.010-1 [23] shall apply, unless otherwise specified in the present clause.

In addition, the apparatus supplier shall provide the information with respect to the Supported Option table A.1 and to ME's default configuration table A.2.

# 6	Implicit testing

For some 3GPP features conformance is not verified explicitly in the present document. This does not imply that correct functioning of these features is not essential, but that these are implicitly tested to a sufficient degree in other tests.

It should be noted that for these features some aspects must be explicitly tested, e.g. the ability to switch between 1.8 V and 3 V operation.

Some UICC features will be explicitly tested as result of other tests. These should be identified for the following reason:

- To identify the areas of overlap and thus provide a more efficient testing.

# 7	Measurement uncertainty

The measured value relating to the corresponding limit shall be used to determine whether or not a terminal equipment meets the requirement. (ETR 028, annex B).

This process is often referred to as "shared risk".

# 8	Format of tests

The Format of tests defined in TS 31.124 [2], clause 8 applies.

# 9	Generic call set up procedures

The generic call set up procedures as defined in TS 31.124 [2], clause 9 apply.

# 10	USIM Application Toolkit (USAT) testing on an ME with non-removable UICC

## 10.1	Introduction

The introduction provided in TS 31.124 [2], clause 27.0 applies.

## 10.2	General Test purpose

The General Test purpose defined in TS 31.124 [2], clause 27.22.1A applies.

27.22.2A	 - 2E 	VoidNOTE:	See clauses from 4.4.2 to 4.4.6 of present document.

## 10.3	Initialization of USAT functionality on an ME with non-removable UICC

### 10.3.1	Verification of the USAT support on an ME with non-removable UICC (Profile Download)

#### 10.3.1.1	Definition and applicability

See clause 3.6.2 of the present document.

#### 10.3.1.2	Conformance requirement

The ME shall support the PROFILE DOWNLOAD command as defined in:

-	TS 31.111 [27] clause 5.2.

#### 10.3.1.3	Test purpose

To verify that the ME sends a TERMINAL PROFILE command in accordance with the above requirements.

#### 10.3.1.4	Method of test

##### 10.3.1.4.1	Initial conditions

The ME is connected to the TT. The nrUICC in the ME is configured as defined in clause 4.4.2 of the present document and hosts a USIM with elementary files coded as defined in clause 4.4.2.2.

##### 10.3.1.4.2	Procedure

Expected Sequence 1 (PROFILE DOWNLOAD)

| Step | Direction | Message / Action | Information |
| --- | --- | --- | --- |
| 1 | USER ME | Power on ME | [UICC Activation] |
| 2 | ME UICC | Select EFPL |  |
| 3 | UICC ME | Read EFPL |  |
| 4 | ME UICC | TERMINAL PROFILE 1.1 | PROFILE DOWNLOAD |
| 5 | UICC ME | NORMAL ENDING OF COMMAND 1.1 |  |
| 6 | ME UICC | Select USIM Application |  |

TERMINAL PROFILE: 1.1

Logically:

Coding:

| APDU | CLA=80 | INS=10 | P1=00 | P2=00 | P3=XX |
| --- | --- | --- | --- | --- | --- |

| DATA IN: | YY | ZZ | .. |
| --- | --- | --- | --- |

With XX representing the length of the following DATA IN depending on the USIM Toolkit commands supported by the ME, and with YY, ZZ, .. representing here the bytes of the TERMINAL PROFILE data, as specified in TS 31.111  [27], clause 5.2.

NORMAL ENDING OF COMMAND: 1.1

Logically:

Coding:

| SW1=90 | SW2=00 |
| --- | --- |

#### 10.3.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.

### 10.3.2	Contents of the TERMINAL PROFILE command

#### 10.3.2.1	Definition and applicability

See table E.1 in annex B of TS 31.124 [2].

#### 10.3.2.2	Conformance requirement

CR 1	The ME shall support the PROFILE DOWNLOAD command as defined in:

-	TS 31.111 [27] clause 5.2.

#### 10.3.2.3	Test purpose

The purpose of this test is to:

1.	verify that the TERMINAL PROFILE indicates that Profile Download facility is supported.

2.	record which USIM Application Toolkit facilities are supported by the ME, to determine which subsequent tests are required.

#### 10.3.2.4	Method of test

##### 10.3.2.4.1	Initial conditions

The ME is connected to the TT. The nrUICC in the ME is configured as defined in clause 4.4.2 of the present document and hosts a USIM with elementary files coded as defined in clause 4.4.2.2.

##### 10.3.2.4.2	Procedure

| Step | Direction | Action | Information |
| --- | --- | --- | --- |
| 1 | USER > ME | Power on ME | [UICC Activation] |
| 2 | ME > TT | Send TERMINAL PROFILE | The TT shall record the content of the TERMINAL PROFILE |
| 3 | UICC > ME | Return SW1/SW2: '90 00' |  |
| 4 | USER > ME | Power off ME |  |

#### 10.3.2.5	Test requirement

The ME shall operate in the manner defined in the test procedure.

#### 10.3.2.6	Acceptance criteria

CR 1 is met if a TERMINAL PROFILE command with bit 1 of the first byte set to 1 (facility supported by ME) is sent and if the TERMINAL PROFILE information "support" recorded is in accordance with the "Status" column as defined in table E.1 for the corresponding ME USIM Toolkit Release and Options

NOTE:	Support of features defined only in releases later than currently tested release shall be ignored.

### 10.3.3	Servicing of proactive UICC commands

#### 10.3.3.1	Definition and applicability

See clause 3.6.2 of the present document.

#### 10.3.3.2	Conformance requirement

CR 1	On detection of a pending USIM Application Toolkit command from the UICC the ME shall perform the FETCH command to retrieve the proactive UICC command. The result of the executed command shall be transmitted from the ME to the UICC within a TERMINAL RESPONSE command.

-	TS 31.111 [27] clause 6.3.

NOTE:	The MORE TIME proactive command is used in this test. The ME shall have knowledge of this command but may not support this USIM Application Toolkit facility.

#### 10.3.3.3	Test purpose

To verify that the ME uses the FETCH command to obtain the proactive UICC command, after detection of a pending proactive UICC command. The pending proactive UICC command is indicated by the response parameters '91 xx' from the UICC.

To verify that the ME transmits the result of execution of the proactive UICC command to the UICC in the TERMINAL RESPONSE command.

#### 10.3.3.4	Method of test

##### 10.3.3.4.1	Initial conditions

The ME is connected to the TT. The nrUICC in the ME is configured as defined in clause 4.4.2 of the present document and hosts a USIM with elementary files coded as defined in clause 4.4.2.2.

A process is established to make the nrUSIM indicate that a proactive UICC command is pending. The nrUSIM is configured to send the Proactive command 'MORE TIME'.

A test toolkit applet shall be used for generating proactive UICC commands specific to the test case and TT shall be able to trigger the MORE TIME proactive commands using the applet. One or more test specific EFs may be required to program data required for the proactive command.

Depending on the capabilities supported by device and nrUICC, APDUs of FETCH and TERMINAL RESPONSE commands shall be verified using either of the methods defined in clauses 4.1.3 and 4.1.4 of the present document (A.4/2 and A.4/1).

##### 10.3.3.4.2	Procedure

| Step | Direction | MESSAGE / Action | Information / Comments |
| --- | --- | --- | --- |
| 1 | USER > ME | Power on ME | [UICC Activation] |
| 2 | ME | Execute PROFILE DOWNLOAD |  |
| 3 | nrUSIM > ME | Return SW1/SW2: '91 0B' | The nrUSIM indicates that a Proactive UICC Command is pending |
| 4 | ME > nrUSIM | Send FETCH command |  |
| 5 | nrUSIM > ME | Return Proactive UICC Command 2.1: MORE TIME |  |
| 6 | USER > ME | Power off ME |  |

#### 10.3.3.5	Test requirement

The ME shall operate in the manner defined in the test procedure.

#### 10.3.3.6	Acceptance criteria

CR 1 is met if the ME sends a FETCH command in step 4) and it can be verified via A.4/x method that the TERMINAL RESPONSE command with command number "01", type of command "02" and command qualifier "00".

## 10.4	Proactive UICC commands

### 10.4.1	DISPLAY TEXT

#### 10.4.1.1	DISPLAY TEXT (Normal)

For test sequences 1.1 to 1.9 the test descriptions from TS 31.124 [2], clause 27.22.4.1.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.1.2	DISPLAY TEXT (Support of "No response from user")

For test sequence 2.1 the test descriptions from TS 31.124 [2], clause 27.22.4.1.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.1.3	DISPLAY TEXT (Display of extension text)

For test sequence 3.1 the test descriptions from TS 31.124 [2], clause 27.22.4.1.3 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.1.4	DISPLAY TEXT (Sustained text)

For test sequences 4.1 to 4.3 the test descriptions from TS 31.124 [2], clause 27.22.4.1.4 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.1.5	DISPLAY TEXT (Display of icons)

For test sequences 5.1 to 5.3 the test descriptions from TS 31.124 [2], clause 27.22.4.1.5 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.1.6	DISPLAY TEXT (UCS2 display in Cyrillic)

For test sequence 6.1 the test descriptions from TS 31.124 [2], clause 27.22.4.1.6 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.1.7	DISPLAY TEXT (Variable Time out)

For test sequence 7.1 the test descriptions from TS 31.124 [2], clause 27.22.4.1.7 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.1.8	DISPLAY TEXT (Support of Text Attribute)

For test sequences 8.1 to 8.10 the test descriptions from TS 31.124 [2], clause 27.22.4.1.8 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.1.9	DISPLAY TEXT (UCS2 display in Chinese)

For test sequence 9.1 the test descriptions from TS 31.124 [2], clause 27.22.4.1.9 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.1.10	DISPLAY TEXT (UCS2 display in Katakana)

For test sequence 10.1 the test descriptions from TS 31.124 [2], clause 27.22.4.1.10 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.2	GET INKEY

#### 10.4.2.1	GET INKEY (Normal)

For test sequences 1.1 to 1.6 the test descriptions from TS 31.124 [2], clause 27.22.4.2.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.2.2	GET INKEY (No response from User)

For test sequence 2.1 the test descriptions from TS 31.124 [2], clause 27.22.4.2.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.2.3	GET INKEY (UCS2 display in Cyrillic)

For test sequence 3.1 and 3.2 the test descriptions from TS 31.124 [2], clause 27.22.4.2.3 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.2.4	GET INKEY (UCS2 entry in Cyrillic)

For test sequence 4.1 the test descriptions from TS 31.124 [2], clause 27.22.4.2.4 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.2.5	GET INKEY ("Yes/No" Response)

For test sequence 5.1 the test descriptions from TS 31.124 [2], clause 27.22.4.2.5 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2,

#### 10.4.2.6	GET INKEY (Display of icons)

For test sequences 6.1 to 6.4 the test descriptions from TS 31.124 [2], clause 27.22.4.2.6 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2,

#### 10.4.2.7	GET INKEY (Help Information)

For test sequence 7.1 the test descriptions from TS 31.124 [2], clause 27.22.4.2.7 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.2.8	GET INKEY (Variable Time out)

For test sequence 8.1 the test descriptions from TS 31.124 [2], clause 27.22.4.2.8 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.2.9	GET INKEY (Support of Text Attribute)

For test sequence 9.1 to 9.10 the test descriptions from TS 31.124 [2], clause 27.22.4.2.9 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.2.10	GET INKEY (UCS2 display in Chinese)

For test sequences 10.1 and 10.2 the test descriptions from TS 31.124 [2], clause 27.22.4.2.10 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.2.11	GET INKEY (UCS2 entry in Chinese)

For test sequence 11.1 the test descriptions from TS 31.124 [2], clause 27.22.4.2.11 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.2.12	GET INKEY (UCS2 display in Katakana)

For test sequences 12.1 and 12.2 the test descriptions from TS 31.124 [2], clause 27.22.4.2.12 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.2.13	GET INKEY (UCS2 entry in Katakana)

For test sequence 13.1 the test descriptions from TS 31.124 [2], clause 27.22.4.2.13 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.3	GET INPUT

#### 10.4.3.1	GET INPUT (Normal)

For test sequences 1.1 to 1.10 the test descriptions from TS 31.124 [2], clause 27.22.4.3.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.3.2	GET INPUT (No response from User)

For test sequence 2.1 the test descriptions from TS 31.124 [2], clause 27.22.4.3.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.3.3	GET INPUT (UCS2 display in Cyrillic)

For test sequences 3.1 and 3.2 the test descriptions from TS 31.124 [2], clause 27.22.4.3.3 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.3.4	GET INPUT (UCS2 entry in Cyrillic)

For test sequences 4.1 and 4.2 the test descriptions from TS 31.124 [2], clause 27.22.4.3.4 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.3.5	GET INPUT (Default text)

For test sequences 5.1 and 5.2 the test descriptions from TS 31.124 [2], clause 27.22.4.3.5 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.3.6	GET INPUT (Display of icons)

For test sequences 6.1 to 6.4 the test descriptions from TS 31.124 [2], clause 27.22.4.3.6 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.3.7	GET INPUT (Help Information)

For test sequence 7.1 the test descriptions from TS 31.124 [2], clause 27.22.4.3.7 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.3.8	GET INPUT (Support of Text Attribute)

For test sequences 8.1 to 8.10 the test descriptions from TS 31.124 [2], clause 27.22.4.3.8 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.3.9	GET INPUT (UCS2 display in Chinese)

For test sequences 9.1 and 9.2 the test descriptions from TS 31.124 [2], clause 27.22.4.3.9 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

10.4.3.10	GET INPUT (UCS2 entry in Chinese)

For test sequences 10.1 and 10.2 the test descriptions from TS 31.124 [2], clause 27.22.4.3.10 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.3.11	GET INPUT (UCS2 display in Katakana)

For test sequences 11.1 and 11.2 the test descriptions from TS 31.124 [2], clause 27.22.4.3.11 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.3.12	GET INPUT (UCS2 entry in Katakana)

For test sequences 12.1 and 12.2 the test descriptions from TS 31.124 [2], clause 27.22.4.3.12 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.4	MORE TIME

For test sequence 1.1 the test descriptions from TS 31.124 [2], clause 27.22.4.4 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.5	PLAY TONE

#### 10.4.5.2	PLAY TONE (UCS2 display in Cyrillic)

For test sequence 2.1 the test descriptions from TS 31.124 [2], clause 27.22.4.5.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.5.3	PLAY TONE (Display of icons)

For test sequences 3.1 and 3.4 the test descriptions from TS 31.124 [2], clause 27.22.4.5.3 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.5.4	PLAY TONE (Support of Text Attribute)

For test sequences 4.1 to 4.10 the test descriptions from TS 31.124 [2], clause 27.22.4.5.4 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.5.5	PLAY TONE (UCS2 display in Chinese)

For test sequence 5.1 the test descriptions from TS 31.124 [2], clause 27.22.4.5.5 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.5.6	PLAY TONE (UCS2 display in Katakana)

For test sequence 6.1 the test descriptions from TS 31.124 [2], clause 27.22.4.5.6 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.6	POLL INTERVAL

For test sequence 1.1 the test descriptions from TS 31.124 [2], clause 27.22.4.6 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.7	REFRESH

#### 10.4.7.1	REFRESH (Normal)

For test sequences 1.3 and 1.5 the test descriptions from TS 31.124 [2], clause 27.22.4.7.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

-	If not available already, the nrUICC is configured to provide a global phonebook to execute test sequence 1.3.

Test sequences 1.1, 1.2, 1.4, 1.6 and 1.7 are not applicable to MEs operating a nrUICC.

#### 10.4.7.2	REFRESH (IMSI changing procedure)

FFS

#### 10.4.7.3	REFRESH (Steering of roaming)

For test sequences 3.3 and 3.4 the test descriptions from TS 31.124 [2], clause 27.22.4.7.3 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	For test sequences 3.3 the nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2 plus the additional parameters provided for sequence 3.3 in TS 31.124 [2], clause 27.22.4.7.3.4.1.

-	For test sequences 3.4 the USIM is configured with values for USIM Application Toolkit testing as defined in clause 4.4.5 of the present document plus the additional parameters provided for sequence 3.4 in TS 31.124 [2], clause 27.22.4.7.3.4.1.

Test sequences 3.1 and 3.2 are not applicable to MEs operating a nrUICC.

#### 10.4.7.4	REFRESH (AID)

For test sequence 4.1 the test descriptions from TS 31.124 [2], clause 27.22.4.7.4 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2. and an ISIM as defined in clause 4.4.3.3.

#### 10.4.7.5	REFRESH (IMSI changing procedure, E-UTRAN)

For test sequences 5.1 and 5.2 the test descriptions from TS 31.124 [2], clause 27.22.4.7.5 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2.

#### 10.4.7.6	REFRESH (IMSI changing procedure, NG-RAN)

For test sequences 6.1 and 6.2 the test descriptions from TS 31.124 [2], clause 27.22.4.1.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2.

#### 10.4.7.7	REFRESH (SUPI_NAI changing procedure, NG-RAN)

For test sequences 7.1 and 7.2 the test descriptions from TS 31.124 [2], clause 27.22.4.7.7 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2 plus the exceptions defined in clause 27.22.4.7.7.4.1 of TS 31.124 [2].

-	The NG-RAN simulator of the TT is configured with the parameters provided in clause 27.22.4.7.7.4.1 of TS 31.124 [2].

### 10.4.8	SET UP MENU and ENVELOPE MENU SELECTION

#### 10.4.8.1	SET UP MENU (Normal) and ENVELOPE MENU SELECTION

For test sequences 1.1 and 1.2 the test descriptions from TS 31.124 [2], clause 27.22.4.8.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.8.2	SET UP MENU (Help request support) and ENVELOPE MENU SELECTION

For test sequence 2.1 the test descriptions from TS 31.124 [2], clause 27.22.4.8.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.8.3	SET UP MENU (Help request support) and ENVELOPE MENU SELECTION

For test sequence 3.1 the test descriptions from TS 31.124 [2], clause 27.22.4.8.3 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.8.4	SET UP MENU (Display of icons) and ENVELOPE MENU SELECTION

For test sequences 4.1 and 4.2 the test descriptions from TS 31.124 [2], clause 27.22.4.8.4 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.8.5	SET UP MENU (Soft Keys support) and ENVELOPE MENU SELECTION

For test sequence 5.1 the test descriptions from TS 31.124 [2], clause 27.22.4.8.5 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.8.6	SET UP MENU (Support of Text Attribute) and ENVELOPE MENU SELECTION

For test sequences 6.1 to 6.10 the test descriptions from TS 31.124 [2], clause 27.22.4.8.6 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.8.7	SET UP MENU (UCS2 display in Cyrillic) and ENVELOPE MENU SELECTION

For test sequence 7.1 the test descriptions from TS 31.124 [2], clause 27.22.4.8.7 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.8.8	SET UP MENU (UCS2 display in Chinese) and ENVELOPE MENU SELECTION

For test sequence 8.1 the test descriptions from TS 31.124 [2], clause 27.22.4.8.8 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.8.9	SET UP MENU (UCS2 display in Katakana) and ENVELOPE MENU SELECTION

For test sequence 9.1 the test descriptions from TS 31.124 [2], clause 27.22.4.8.9 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.9	SELECT ITEM

#### 10.4.9.1	SELECT ITEM (Mandatory features for ME supporting SELECT ITEM)

For test sequence 1.1 the test descriptions from TS 31.124 [2], clause 27.22.4.9.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.9.2	SELECT ITEM (Next action support)

For test sequence 2.1 the test descriptions from TS 31.124 [2], clause 27.22.4.9.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.9.3	SELECT ITEM (Default item support)

For test sequence 3.1 the test descriptions from TS 31.124 [2], clause 27.22.4.9.3 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.9.4	SELECT ITEM ((Help request support)

For test sequence 4.1 the test descriptions from TS 31.124 [2], clause 27.22.4.9.4 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.9.5	SELECT ITEM (Icons support)

For test sequences 5.1 and 5.2 the test descriptions from TS 31.124 [2], clause 27.22.4.9.5 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.9.6	SELECT ITEM (Presentation style)

For test sequences 6.1 and 6.2 the test descriptions from TS 31.124 [2], clause 27.22.4.9.6 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.9.7	SELECT ITEM (Soft keys support)

For test sequence 7.1 the test descriptions from TS 31.124 [2], clause 27.22.4.9.7 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.9.8	SELECT ITEM (Support of "No response from user")

For test sequence 8.1 the test descriptions from TS 31.124 [2], clause 27.22.4.9.8 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.9.9	SELECT ITEM (Support of Text Attribute)

For test sequences 9.1 to 9.10 the test descriptions from TS 31.124 [2], clause 27.22.4.9.9 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.9.10	SELECT ITEM (UCS2 display in Cyrillic)

For test sequences 10.1 to 10.3 the test descriptions from TS 31.124 [2], clause 27.22.4.9.10 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.9.11	SELECT ITEM (UCS2 display in Chinese)

For test sequence 11.1 the test descriptions from TS 31.124 [2], clause 27.22.4.9.11 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.9.12	SELECT ITEM (UCS2 display in Katakana)

For test sequences 12.1 to 12.3 the test descriptions from TS 31.124 [2], clause 27.22.4.9.12 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.10	SEND SHORT MESSAGE

#### 10.4.10.1	SEND SHORT MESSAGE (Normal)

FFS

#### 10.4.10.2	SEND SHORT MESSAGE (UCS2 display in Cyrillic)

FFS

#### 10.4.10.3	SEND SHORT MESSAGE (Icon support)

FFS

#### 10.4.10.4	SEND SHORT MESSAGE (Support of Text Attribute)

FFS

#### 10.4.10.5	SEND SHORT MESSAGE (UCS2 display in Chinese)

FFS

#### 10.4.10.6	SEND SHORT MESSAGE (UCS2 display in Katakana)

FFS

#### 10.4.10.7	SEND SHORT MESSAGE (IMS)

For test sequence 7.1 the test descriptions from TS 31.124 [2], clause 27.22.4.1.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2 and an ISIM as defined in clause 4.4.3.3.

Test sequence 7.2 is not applicable to MEs operating a nrUICC.

#### 10.4.10.8	SEND SHORT MESSAGE (Over SGs in E-UTRAN)

For test sequence 8.1 the test descriptions from TS 31.124 [2], clause 27.22.4.1.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2.

### 10.4.11	SEND SS

FFS

### 10.4.12	SEND USSD

FFS

### 10.4.13	SET UP CALL

FFS

### 10.4.14	POLLING OFF

For test sequence 1.2 the test descriptions from TS 31.124 [2], clause 27.22.4.1.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2.

Test sequence 1.1 is not applicable to MEs operating a nrUICC.

### 10.4.15	PROVIDE LOCAL INFORMATION

For test sequences 1.2, 1.4, 1.5, 1.9 to 1.11, 1.14 to 1.18 and 1.22 to 1.30 the test descriptions from TS 31.124 [2], clause 27.22.4.1.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	For test sequences 1.2, 1.4, 1.5, 1.9 to 1.11 and 1.14 to 1.18 the nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2.

-	For test sequences 1.22 to 1.30 the nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2.

Test sequences 1.1, 1.3, 1.6, 1.7, 1.12 and 1.13 are not applicable to MEs operating a nrUICC.

Test sequence 1.8 is voided and test sequences 1.19 to 1.21 are "TBD" in TS 31.124 [2]. For editorial purposes the numbering in the present specification is kept identical.

### 10.4.16	SET UP EVENT LIST

FFS

### 10.4.17	PERFORM CARD APDU

#### 10.4.17.1	PERFORM CARD APDU (Normal)

For test sequences 1.1 and 1.3 to 1.5 the test descriptions from TS 31.124 [2], clause 27.22.4.17.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

Test sequence 1.2 is not applicable to MEs operating a nrUICC.

#### 10.4.17.2	PERFORM CARD APDU (Detachable card reader)

For test sequence 2.1 the test descriptions from TS 31.124 [2], clause 27.22.4.17.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.18	POWER OFF CARD

#### 10.4.18.1	POWER OFF CARD (Normal)

For test sequence 1.1 the test descriptions from TS 31.124 [2], clause 27.22.4.18.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.18.2	POWER OFF CARD (Detachable card reader)

For test sequence 2.1 the test descriptions from TS 31.124 [2], clause 27.22.4.18.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.19	POWER ON CARD

#### 10.4.19.1	POWER ON CARD (Normal)

For test sequence 1.1 the test descriptions from TS 31.124 [2], clause 27.22.4.19.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.19.2	POWER ON CARD (Detachable card reader)

For test sequence 2.1 the test descriptions from TS 31.124 [2], clause 27.22.4.19.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.20	GET READER STATUS

#### 10.4.20.1	GET READER STATUS (Normal)

For test sequences 1.1 and 1.2 the test descriptions from TS 31.124 [2], clause 27.22.4.18.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.20.2	GET READER STATUS (Detachable card reader)

For test sequence 2.1 the test descriptions from TS 31.124 [2], clause 27.22.4.20.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.21	TIMER MANAGEMENT and ENVELOPE TIMER EXPIRATION

#### 10.4.21.1	TIMER MANAGEMENT (Normal)

For test sequences 1.1 to 1.6 the test descriptions from TS 31.124 [2], clause 27.22.4.21.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.21.2	ENVELOPE TIMER EXPIRATION (Normal)

For test sequences 2.1 and 2.2 the test descriptions from TS 31.124 [2], clause 27.22.4.21.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.22	SET UP IDLE MODE TEXT

Test sequences for SET UP IDLE MODE TEXT currently defined in TS 31.124 [2] are not applicable for MEs operating a nrUICC. For editorial purposes the clause and the clause numbering are kept.

### 10.4.23	RUN AT COMMAND

#### 10.4.23.1	RUN AT COMMAND (Normal)

For test sequences 1.1 to 1.3 the test descriptions from TS 31.124 [2], clause 27.22.4.23.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.23.2	RUN AT COMMAND (Icon support)

For test sequences 2.1 to 2.5 the test descriptions from TS 31.124 [2], clause 27.22.4.23.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

10.4.23.3	RUN AT COMMAND (Support of Text Attribute)

For test sequences 3.1 to 3.10 the test descriptions from TS 31.124 [2], clause 27.22.4.23.3 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.23.4	RUN AT COMMAND (UCS2 display in Cyrillic)

For test sequence 4.1 the test descriptions from TS 31.124 [2], clause 27.22.4.23.4 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.23.5	RUN AT COMMAND (UCS2 display in Chinese)

For test sequence 5.1 the test descriptions from TS 31.124 [2], clause 27.22.4.23.5 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.23.6	RUN AT COMMAND (UCS2 display in Katakana

For test sequence 6.1 the test descriptions from TS 31.124 [2], clause 27.22.4.23.6 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.24	SEND DTMF

FFS

### 10.4.25	LANGUAGE NOTIFICATION

For test sequences 1.1 and 1.2 the test descriptions from TS 31.124 [2], clause 27.22.4.25 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.4.26	LAUNCH BROWSER

#### 10.4.26.1	LAUNCH BROWSER (No session already launched)

FFS

#### 10.4.26.2	LAUNCH BROWSER (Interaction with current session)

FFS

#### 10.4.26.3	LAUNCH BROWSER (UCS2 display in Cyrillic)

FFS

#### 10.4.26.4	LAUNCH BROWSER (Icon Support)

FFS

#### 10.4.26.5	LAUNCH BROWSER (Support of Text Attribute)

FFS

#### 10.4.26.6	LAUNCH BROWSER (UCS2 Display in Chinese)

FFS

#### 10.4.26.7	LAUNCH BROWSER (UCS2 Display in Katakana)

FFS

#### 10.4.26.8	LAUNCH BROWSER (NG-RAN bearer)

For test sequences 8.1 to 8.5 the test descriptions from TS 31.124 [2], clause 27.22.4.20.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	For test sequences 8.1 the nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2.

-	For test sequences 8.2 to 8.5 the nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2 and an ISIM as defined in clause 4.4.4.5.

-	Additional parameters provided in TS 31.124 [2], clause 27.22.4.26.8.4.1 apply to the appropriate configuration.

### 10.4.27	OPEN CHANNEL

#### 10.4.27.1	Void

Clause kept for editorial reasons.

#### 10.4.26.2	OPEN CHANNEL (Related to GPRS)

FFS

#### 10.4.26.3	OPEN CHANNEL (Default Bearer)

FFS

#### 10.4.26.4	OPEN CHANNEL (Local Bearer)

FFS

#### 10.4.26.5	OPEN CHANNEL (GPRS, Support of Text Attribute)

FFS

#### 10.4.27.6	OPEN CHANNEL (Related to E-UTRAN)

For test sequences 6.1 to 6.8 the test descriptions from TS 31.124 [2], clause 27.22.4.27.6 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.27.7	OPEN CHANNEL (UICC Access to IMS)

For test sequence 7.1 the test descriptions from TS 31.124 [2], clause 27.22.4.27.7 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2 and an ISIM as defined in clause 4.4.3.3 plus the additional parameters provided for sequence 7.1 in TS 31.124 [2], clause 27.22.4.27.7.4.1.

#### 10.4.27.8	OPEN CHANNEL (related to NG-RAN)

For test sequences 8.1 to 8.6 the test descriptions from TS 31.124 [2], clause 27.22.4.27.8 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2 plus the additional parameters provided in TS 31.124 [2], clause 27.22.4.27.8.4.1.

### 10.4.28	CLOSE CHANNEL

#### 10.4.28.1	CLOSE CHANNEL (Normal)

FFS

#### 10.4.28.2	CLOSE CHANNEL (support of Text Attribute)

FFS

#### 10.4.28.3	CLOSE CHANNEL (E-UTRAN/EPC)

For test sequences 3.1 to 3.3 the test descriptions from TS 31.124 [2], clause 27.22.4.28.3 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2 plus the additional parameters provided in TS 31.124 [2], clause 27.22.4.28.3.4.1.

#### 10.4.28.4	CLOSE CHANNEL (NG-RAN)

For test sequences 4.1 and 4.2 the test descriptions from TS 31.124 [2], clause 27.22.4.28.4 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2 plus the additional parameters provided in TS 31.124 [2], clause 27.22.4.28.4.4.1.

### 10.4.29	RECEIVE DATA

#### 10.4.29.1	RECEIVE DATA (Normal)

For test sequences 1.2 to 1.5 and 1.7 the test descriptions from TS 31.124 [2], clause 27.22.4.29.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	For test sequences 1.2 the nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2.

-	For test sequences 1.3 to 1.5 and 1.7 the nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2.

-	Additional parameters provided in TS 31.124 [2], clause 27.22.4.29.1.4.1 apply to the appropriate configuration.

Test sequences 1.1 and 1.6 are not applicable to MEs operating a nrUICC.

#### 10.4.29.2	RECEIVE DATA (Support of Text Attribute)

FFS

### 10.4.30	SEND DATA

#### 10.4.30.1	SEND DATA (Normal)

FFS

#### 10.4.30.2	SEND DATA (Support of Text Attribute)

FFS

#### 10.4.30.3	SEND DATA(E-UTRAN)

For test sequences 3.1 and 3.2 the test descriptions from TS 31.124 [2], clause 27.22.4.30.3 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.4.30.4	SEND DATA(NG-RAN)

For test sequences 4.1 and 4.2 the test descriptions from TS 31.124 [2], clause 27.22.4.30.4 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2 plus the additional parameters provided in TS 31.124 [2], clause 27.22.4.30.4.4.1.

### 10.4.31	GET CHANNEL STATUS

For test sequences 1.4 to 1.6 the test descriptions from TS 31.124 [2], clause 27.22.4.31 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	For test sequences 1.4 and 1.5 the nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2.

-	For test sequence 1.6 the nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2.

Test sequences 1.1 to 1.3 are not applicable to MEs operating a nrUICC.

## 10.5	Data Download to UICC

### 10.5.1	SMS-PP Data Download

FFS

### 10.5.2	Cell Broadcast Data Download

FFS

### 10.5.3	SMS-PP Data Download over IMS

For test sequence 3.1 the test descriptions from TS 31.124 [2], clause 27.22.5.3 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2 and an ISIM as defined in clause 4.4.3.3.

Test sequence 3.2 is not applicable to MEs operating a nrUICC.

### 10.5.4	SMS-PP Data Download over SGs in E-UTRAN

For test sequence 4.1 the test descriptions from TS 31.124 [2], clause 27.22.5.4 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

## 10.6	CALL CONTROL BY USIM

### 10.6.1	Procedure for Mobile Originated calls

FFS

### 10.6.2	Procedure for Supplementary (SS) Services

FFS

### 10.6.3	Interaction with Fixed Dialling Number (FDN)

FFS

### 10.6.4	Support of Barred Dialling Number (BDN) service

FFS

### 10.6.5	Barred Dialling Number (BDN) service handling for terminals not supporting BDN

FFS

## 10.7	EVENT DOWNLOAD

### 10.7.1	MT Call Event

FFS

### 10.7.2	Call Connected Event

FFS

### 10.7.3	Call Disconnected Event

FFS

### 10.7.4	Location Status Event

#### 10.7.4.1	Location Status Event (Normal)

For test sequences 1.2 and 1.3 the test descriptions from TS 31.124 [2], clause 27.22.7.4 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

Test sequence 1.1 is not applicable to MEs operating a nrUICC.

### 10.7.5	User Activity Event

#### 10.7.5.1	User Activity Event (Normal)

For test sequence 1.1 the test descriptions from TS 31.124 [2], clause 27.22.7.5.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.7.6	Idle Screen Available Event

FFS

### 10.7.7	Card Reader Status Event

#### 10.7.7.1	Card Reader Status (Normal)

For test sequence 1.1 the test descriptions from TS 31.124 [2], clause 27.22.7.7.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

#### 10.7.7.2	Card Reader Status (Detachable card reader)

For test sequence 2.1 the test descriptions from TS 31.124 [2], clause 27.22.7.7.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.7.8	Language Selection Event

#### 10.7.8.1	Language Selection Event (Normal)

For test sequence 1.1 the test descriptions from TS 31.124 [2], clause 27.22.7.8.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.7.9	Browser Termination Event

FFS

### 10.7.10	Data Available Event

#### 10.7.10.1	Data Available Event (Normal)

For test sequences 1.2 to 1.5 the test descriptions from TS 31.124 [2], clause 27.22.7.10.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2 plus the additional parameters provided for sequence 1.2 to 1.5 in TS 31.124 [2], clause 27.22.7.10.4.1.

Test sequence 1.1 is not applicable to MEs operating a nrUICC.

### 10.7.11	Channel Status event

For test sequence 1.2 the test descriptions from TS 31.124 [2], clause 27.22.7.11 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

Test sequence 1.1 is not applicable to MEs operating a nrUICC.

### 10.7.12	Access Technology Change event

For test sequence 1.4 the test descriptions from TS 31.124 [2], clause 27.22.7.12 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2 plus the additional parameters provided for sequence 1.4 in TS 31.124 [2], clause 27.22.7.12.4.1.

Test sequences 1.1 to 1.3 are not applicable to MEs operating a nrUICC.

### 10.7.13	Display parameter changed event

FFS

### 10.7.14	Local Connection event

FFS

### 10.7.15	Network search mode change event

For test sequence 1.1 the test descriptions from TS 31.124 [2], clause 27.22.7.15 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

### 10.7.16	Browsing status event

FFS

### 10.7.17	Network Rejection event

For test sequences 1.1 to 1.4 the test descriptions from TS 31.124 [2], clause 27.22.7.17 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2 or an nrUICC as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2 plus the additional parameters provided in TS 31.124 [2], clause 27.22.7.17.4.1.

### 10.7.18	CSG Cell Selection event

For test sequence 1.1 the test descriptions from TS 31.124 [2], clause 27.22.7.18 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2 plus the additional parameters provided in TS 31.124 [2], clause 27.22.7.18.4.1.

### 10.7.19	IMS registration event

The IMS registration event is tested in 10.4.27.7.1 and 10.7.20

### 10.7.20	Incoming IMS data event

#### 10.7.20.1	Incoming IMS data (Normal)

For test sequences 1.1 and 1.2 the test descriptions from TS 31.124 [2], clause 27.22.7.20 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2 plus the additional parameters provided in TS 31.124 [2], clause 27.22.7.20.1.4.1.

### 10.7.21	Data Connection Status Change event

For test sequence 1.1 the test descriptions from TS 31.124 [2], clause 27.22.7.21 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2 plus the additional parameters provided in TS 31.124 [2], clause 27.22.7.21.1.4.1.

### 10.7.22	CAG Cell Selection event

#### 10.7.22.1	CAG Cell Selection (Normal)

For test sequence 1.1 the test descriptions from TS 31.124 [2], clause 27.22.7.22 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.4 plus the additional parameters provided in TS 31.124 [2], clause 27.22.7.22.1.4.1.

## 10.8	MO SHORT MESSAGE CONTROL BY USIM

For test sequences 1.10 to 1.17 the test descriptions from TS 31.124 [2], clause 27.22.8 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2 plus the additional parameters provided for sequences 1.10 to 1.17 in TS 31.124 [2], clause 27.22.8.4.1.

Test sequences 1.1 to 1.8 are not applicable to MEs operating a nrUICC. Sequence 1.9 is voided in TS 31.124 [2], the numbering is kept identical for better readability.

## 10.9	Handling of command number

For test sequence 1.1 the test descriptions from TS 31.124 [2], clause 27.22.9 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2.

## 10.10	CALL CONTROL on EPS PDN Connection

For test sequence 1.1 to 1.7 the test descriptions from TS 31.124 [2], clause 27.22.10 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.3 of the present document, hosting a USIM as defined in clause 4.4.3.2 plus the additional parameters provided for sequences 1.1 to 1.7 in TS 31.124 [2], clause 27.22.10.1.4.1.

## 10.11	Call Control on PDP Context Activation

### 10.11.1	Procedure for Mobile Originated calls

FFS

## 10.12	Change eCall mode

For test sequences 1.1 to 1.3 the test descriptions from TS 31.124 [2], clause 27.22.10.12 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with default values for USIM Application Toolkit testing as defined in clause 4.4.2 of the present document, hosting a USIM as defined in clause 4.4.2.2 plus the additional parameters provided for the appropriate sequences 1.1 to 1.3 in TS 31.124 [2], clause 27.22.12.1.4.

Test sequences 1.4 and 1.5 are not applicable to MEs operating a nrUICC.

## 10.13	CALL CONTROL on PDU Session Establishment for NG-RAN

### 10.13.1	Procedure for Mobile Originated calls

For test sequences 1.1 to 1.7 the test descriptions from TS 31.124 [2], clause 27.22.10.13.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2 plus the additional parameters provided for the appropriate sequences 1.1 to 1.7 in TS 31.124 [2], clause 27.22.13.1.4.

## 10.14	ENVELOPE SMS-PP Data Download on NAS messages

### 10.14.1	Routing Indicator Data update via DL NAS TRANSPORT messages

For test sequences 1.1 to 1.4 the test descriptions from TS 31.124 [2], clause 27.22.14.1 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2 plus the additional parameters provided for the appropriate sequences 1.1 to 1.4 in TS 31.124 [2], clause 27.22.14.1.4.

### 10.14.2	Steering of Roaming via DL NAS TRANSPORT message

For test sequences 2.1, 2.3 and 2.4 the test descriptions from TS 31.124 [2], clause 27.22.14.2 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2 plus the additional parameters provided for the appropriate sequences 2.1, 2.3 and 2.4 in TS 31.124 [2], clause 27.22.14.2.4.

Test sequence 2.2 is voided in TS 31.124 [2], the numbering is kept identical for better readability.

### 10.14.3	Steering of Roaming via REGISTRATION ACCEPT message

For test sequences 3.1 and 3.2 the test descriptions from TS 31.124 [2], clause 27.22.14.3 apply with the following exceptions:

-	The General Requirements as defined in clause 4.2.1 apply

-	The nrUICC is configured with values for USIM Application Toolkit testing as defined in clause 4.4.4 of the present document, hosting a USIM as defined in clause 4.4.4.2 plus the additional parameters provided for the appropriate sequences 3.1 and 3.2 in TS 31.124 [2], clause 27.22.14.3.4.1.

Test sequence 3.2 is voided in TS 31.124 [2], the numbering is kept identical for better readability.

Annex A (informative):
Examples of Test-nrUICC

# A.0	General information

The details provided for the Test-SIM (TestSIM) in TS 31.124 [2], Annex A apply to the Test-nrUICC used within the present test specification. Possible exceptions and additions are shown in the following clauses of Annex A.

# A.1	Test EF structure - 1

DFTEST shall be present at the Master File (MF) level to be used for testing purposes.

The general structure of DFTEST and associated EF is defined below:

|  |  |  |  |  |  |  |  |  |  |  | MF |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | '3F00' |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | EFDIR |  |  | EFPL |  |  | EFARR |  |  | EFICCID |  |  | EFUMPC |  |  |
|  |  |  |  |  |  |  |  | '2F00' |  |  | '2F05' |  |  | '2F06' |  |  | '2FE2' |  |  | '2F08' |  |  |
|  |  | DFTEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | '7F0A' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | EFSTK |  |  | EFSETSTK |  |  | EFCC |  |  | EFEVENTLIST |  |  |  |  |  |
|  |  |  |  |  |  |  |  | '2F0A' |  |  | '2F0B' |  |  | '2F0C' |  |  | '2F0D' |  |  |  |  |  |

The information in EFSTK, EFSETSTK, EFCC, EFEVENTLIST shall be used. A Generic STK Applet that utilizes the test data configured in these new EFs to trigger the specific commands from the UICC shall be used.

## A.1.1	EFSTK (STK)

| Identifier: '2F0A' |  | Structure: linear fixed |  |  | Mandatory |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 255 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS DEACTIVATE ALWAYS ACTIVATE ALWAYS |  |  |  |  |  |  |  |
| Record | Description |  | Default Value |  |  | M/O | Length |
| 1 | Test Data - Record 1 |  | FF FF .. FF FF |  |  | M | 255 |
| .. | .. |  | .. |  |  | M | 255 |
| 40 | Test Data - Record 40 |  | FF FF .. FF FF |  |  | M | 255 |

Individual record can hold the tester configured data, specific to the Proactive command that needs to be issued by the terminal referenced by EFSETSTK, for details on the structure and coding of the Proactive command refer to TS 31.111 [27], ETSI TS 102 223 [28].

## A.1.2	EFSETSTK (SETSTK)

| Identifier: '2F0B' |  | Structure: transparent |  |  | Mandatory |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| File size: 255 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS DEACTIVATE ALWAYS ACTIVATE ALWAYS |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1-255 | Test Data |  | 00 |  |  | M | 255 |

Test Data is configured with the specific record number of EFSTK. When configured, it will imply to trigger the respective STK command at specified record number of EFSTK.

For example, if EFSTK is configured with '0x01', the Applet will issue the command configured at record 01 of EFSTK. Once the command is issued successfully, the applet will clear this EFSETSTK to the default value.

If EFSTK is configured as '0x80', the Applet will issue the command configured at all records of EFSTK. This is helpful for tests where multiple Proactive commands are required to be issued in sequence (from record 1 to n of EFSTK) after receiving the Terminal Response from the prior command.

## A.1.3	EFCC (CALL CONTROL)

| Identifier: '2F0C' |  | Structure: transparent |  |  | Mandatory |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 255 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS DEACTIVATE ALWAYS ACTIVATE ALWAYS |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1-255 | Test Data |  | 00..00 |  |  | M | 255 |

Test Data is configured with the Call control response (as defined in ETSI TS 102 223 [28], TS 31.111 [27]) to process the Call Control Envelope commands received from terminal based on the Call Control service configured in UST.

For Example:

- '00 00' implies allowed not modified.

- '01 00' implies not allowed.

- '02 LL…XX' implies allowed with modifications. (LL denotes the length of the TLV).

## A.1.4	EFEVENTLIST (EVENT LIST)

| Identifier: '2F0D' |  | Structure: transparent |  |  | Mandatory |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: X bytes (1 ≤ X ≤ 255) |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS DEACTIVATE ALWAYS ACTIVATE ALWAYS |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| X | Test Data |  | FF..FF |  |  | M | X |

The Test data is of format <length><event1><event2>...<event n>... FF FF, where the length indicates the number of configured events. Supported events are defined in ETSI TS 102 223 [28], TS 31.111 [27].

For verification purposes the data persists even after the card reset. During test initialization this data needs to be cleared by the tester by reverting the contents to default values (FF..FF).

# A.2	Test EF structure - 2

EFTC_IN and EFTC_OUT files are used during test case execution, and it shall be updated fresh for each test case. EFEF_RESET file is used for saving EF content required to bring back the TS.48 profile to original profile content before initializing test specific data.

This EF structure may be suitable for the test environment defined in clause 4.1.4.

|  |  |  |  |  |  | MF |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | '3F00' |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | EFDIR |  |  | EFPL |  |  | EFARR |  |  | EFICCID |  |  |  | EFUMPC |  |
|  |  |  | '2F00' |  |  | '2F05' |  |  | '2F06' |  |  | '2FE2' |  |  |  | '2F08' |  |
|  |  |  | EFEF_RESET |  |  | EFTC_IN |  |  | EFTC_OUT |  |
|  |  |  | '2FF7' |  |  | '2FF8' |  |  | '2FF9' |  |

## A.2.1	EFTC_IN (TC_IN)

This file contains data required for a test applet to handle test case execution. Content and format of the data may be changed depending on the Applet implementation.

| Identifier: '2FF8' |  | Structure: transparent |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 255 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS DEACTIVATE ALWAYS ACTIVATE ALWAYS |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1-255 | Test Data |  | FF..FF |  |  | M | 255 |

The EF can include data related to,

- Test step control

  - TT can update this data to direct test applet to move to next step in the test case.

- Toolkit command data

  - This can be a sequence of TLVs related to proactive commands or event registrations or call control response data or any other action required for the test case.

  - Different Tags can be defined for this purpose.

Example:

| Bytes | Description | M/O | Length |
| --- | --- | --- | --- |
| 1 | Test step control | M | 1 byte |
| 2 | Tag 1: Wait for new step tag | M | 1 byte |
| 3 | Tag 1 length | M | 1 byte |
| 4 to 4+X | Tag 1 data | M | X bytes |
| 5+X to 6+X | Tag 2: Proactive command tag | O | 1 byte |
| 7+X to 8+X | Tag 2 length | O | 1 byte |
| 9+X to 9+X+Y | Tag 2 data | O | Y bytes |

## A.2.2	EFTC_OUT (TC_OUT)

This file contains data received from the terminal and it can be read by the TT to verify if meeting the acceptance criteria of a test case. Additionally, few leading bytes can be used for tracking EFTC_IN and EFTC_OUT current offsets if required by the applet. Content and format of the data may be changed depending on the Applet implementation.

| Identifier: '2FF9' |  | Structure: transparent |  |  | Optional |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 255 bytes |  |  |  | Update activity: low |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS DEACTIVATE ALWAYS ACTIVATE ALWAYS |  |  |  |  |  |  |  |
| Bytes | Description |  | Default Value |  |  | M/O | Length |
| 1-255 | Test Data |  | FF..FF |  |  | M | 255 |

The EF can include data related to,

- Test EFs offset control,

  - TT can update initial offsets during TC initialization.

  - TT can track and update test EF offsets as applet executes test steps.

- Toolkit command response data received from the terminal,

  - This can be a sequence of TLVs related to proactive command responses (TERMIAL RESPONSE) or envelop commands (EVENTs).

  - Different Tags can be used for this purpose.

Example:

| Bytes | Description | M/O | Length |
| --- | --- | --- | --- |
| 1-2 | Test EFs offset control bytes | M | 2 bytes |
| 3-4 | Length | M | 2 bytes |
| 5 | Tag 1: Proactive command 1 tag (for TERMINAL RESPONSE 1) | O | 1 byte |
| 6 | Tag 1 length | O | 1 byte |
| 7 to 7+X | Tag 1 data | O | X bytes |
| 8+X to 9+X | Tag 2: Proactive command 2 tag (for TERMINAL RESPONSE 2) | O | 1 byte |
| 10+X to 11+X | Tag 2 length | O | 1 byte |
| 12+X to 12+X+Y | Tag 2 data | O | Y bytes |

## A.2.3	EFEF_RESET (EF_RESET)

This file contains data required for updating EFs in the TS.48 profile to bring back to the original content of the profile during test case initialization. TT can request applet through test control byte to read the content from this EF and update required EFs in the profile. Content and format of the data may be changed depending on the Applet implementation. Content and format of the data may be changed depending on the Applet implementation.

| Identifier: '6F3B' |  | Structure: linear fixed |  |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Record length: 128 bytes |  |  | Update activity: low |  |  |  |  |
| File size: 128n, (n=25) |  |  |  |  |  |  |  |
| Access Conditions: READ ALWAYS UPDATE ALWAYS DEACTIVATE ALWAYS ACTIVATE ALWAYS |  |  |  |  |  |  |  |
| Record | Description |  |  | Default Value | M/O |  | Length |
| 1 | EF content read control data |  |  | FF FF .. FF FF | M |  | X bytes |
| 2 | Sequence of EF content data - 1 |  |  | FF FF .. FF FF | M |  | 128 bytes |
| .. | .. |  |  | .. | O |  | 128 bytes |
| 25 | Sequence of EF content data - 24 |  |  | FF FF .. FF FF | O |  | 128 bytes |

The EF can include data related to,

- EF content read control data,

  - TT can update this record with an instruction on what records to be read to update the EFs in TS.48 profile.

- Sequence of EF content data,

  - This can be a sequence of TLVs related to EF content in the TS.48 profile.

  - Each sequence can include data related to more than one EF in the TS.48 profile.

  - Applet can choose a format to include number of ‘FF’ padding or unused bytes instead of updating with a sequence of ‘FF’ bytes to save the size of data in a record.

Example format:

| Record number | Data format |
| --- | --- |
| 1 | <instruction to read all none empty records>, <FEFRESH flag> OR <read from start record>, <read end record>, <REFRESH flag> |
| 2 | < EF id 1>,<0 (for Transparent EF this is 0)>,<no. of FF>,<length of data 1>,<data 1>, < EF id 2>,<Record# 1>,<no. of FF>,<length of actual data 2.1>,<data 2.1>, < EF id 2>,<Record# 2>,<no. of FF>,<length of actual data 2.2>,<data 2.2>, … |
| 3 | < EF id 5>,<0 (Transparent EF)>,<no. of FF>,<length of data 5>,<data 5>, < EF id 6>,<1 (Record# 1)>,<no. of FF>,<length of actual data 6.1>,<data 6.1>, < EF id 7>,<0 (Transparent E)>,<no. of FF>,<length of actual data 7>,<data 7>, … |

Example data:

| Record number | Data format |
| --- | --- |
| 1 | FD,1 OR 2 ,3, 0 |
| 2 | 6FC901000401020304… |
| 3 | 6FCE050701096FD2006103030303… |

Annex B (normative):
Details of terminal profile support

The details of the terminal profile provided in Table E.1 in TS 31.124 [2], Annex B identically apply to the present specification.

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New |
| 2024-08 | CT6#119b | C6-240459 |  |  |  | Agreed version after CT6#119bis | 0.1.0 |

Annex <B>:
<Informative annex title for a Technical Report>

Informative annexes in Technical Reports do not use "(informative") in the title, since all annexes in TRs are informative. Use style "Heading 9" in TRs.