| 3GPP TR 38.744 V0.0.4 (2024-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Abbreviations	6

4	AI/ML mobility use cases	7

4.1 General	7

4.2	RRM measurement prediction	7

4.3	Measurement event prediction	7

4.4	RLF prediction	7

5	Evaluations	8

5.1	Common evaluation methodology, metrics and assumptions	8

5.2	RRM measurement prediction	10

5.2.1	Evaluation methodology, metrics and assumptions	10

5.2.2	Evaluation results	12

5.3	Measurement event prediction	12

5.3.1	Evaluation methodology, metrics and assumptions	12

5.3.2	Evaluation results	12

5.4	RLF prediction	12

5.4.1	Evaluation methodology, metrics and assumptions	12

5.4.2	Evaluation results	12

6	Potential specification impact	12

6.1	LCM, protocol and procedure aspects	12

6.1.1	Common aspects	12

6.1.2	RRM measurement prediction	12

6.1.3	Measurement event prediction	12

6.1.4	RLF/HOF prediction	12

6.2	Interoperability, testability, and RRM requirements	12

7	Conclusion	13

Annex <A> (informative): <Informative annex for a Technical Specification>	13

A.1	Simulation template table	13

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document …

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[3]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[4]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz"

[5]	3GPP TR 38.843: “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”

[6]	3GPP TS 38.300: “NR and NG-RAN Overall description; Stage-2”

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

HOF:	Handover failure

MRRS	Measurement reduction rate in spatial domain

MRRT	Measurement reduction rate in temporal domain

OW	Observation window

PW	Prediction window

RLF:	Radio link failure

4	AI/ML mobility use cases

4.1 General

The use cases in this study focus on RRC_CONNECTED mode and cover RRM measurement prediction, measurement event prediction and RLF/HOF prediction for PCell change procedure in standalone NR scenario. The study of the use cases is driven mainly by two study goals. The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements. The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

Editor Note 1: This section intends to capture the study goals, and description of use cases.

Editor Note 2: RAN2 may discuss handover performance after evaluation scenario(s) with good measurements prediction accuracy is found.

4.2	RRM measurement prediction

3 sub-use cases are considered for cell-level RRM measurement prediction:

Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

3 sub-use cases are considered for beam-level RRM measurement prediction:

Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated

Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

Editor Note 1: L3 filtered beam level prediction sub-use cases are lower priority and should focus on FR2 intra-frequency temporal domain case A

4.3	Measurement event prediction

Editor Note 1: The measurement event refers to measurement events A1-A5 defined in clause 5.5.4 in 38.331. Measurement event A3 is taken as starting point, other events are FFS.

Editor Note 2: At least indirect measurement event prediction will be studied. And direct measurement event prediction is also allowed. The definition of direct and indirect measurement event prediction is FFS.

Editor Note 3: Study on measurement event prediction can be started after some further progress on RRM measurement prediction has been made.

4.4	RLF prediction

The study focuses on RLF detected upon T310 expiry in PCell [2].

RLF can be predicted directly based on actual measurement result(s) e.g. SINR of PCell. In indirect RLF prediction, based on actual measurement result(s) e.g. SINR of PCell measurement result(s) is predicted at first, based on which RLF is derived. The RLF prediction yields probability of RLF happening within a time window or at time instance at least for direct RLF prediction.

Editor Note 1: FFS on definition of time instance.

Editor Note 2: HOF prediction is down prioritized.

5	Evaluations

5.1	Common evaluation methodology, metrics and assumptions

Synthesized datasets based on channel model and deployment [4] are used for evaluation. Field data can be used optionally. In principle once a set of simulation parameters and assumptions are settled, it should also be used for the baseline case (i.e. without AI/ML model), model training (e.g. data set generation), model validation, model test and inference operation [5] etc. Between training and test data set, different random seeds are used at least for channel modelling and UE trajectory.

No traffic model is simulated in this study.

Both sliding L1/L3 filtering and non-sliding L1/L3 filtering options can be used for evaluation.

Figure 5.1-1 Sliding L1/L3 filtering

Figure 5.1-2: non-sliding L1/L3 filtering

In sliding L1/L3 filtering, filtered L1 or L3 measurement result are generated every sample period. In non-sliding L1/L3 filtering, filtered L1 or L3 measurement result are generated every measurement period.

In both L1/L3 filtering options, the filtered L1 measurement result is obtained based on the rawL1 measurement results corresponding to reference point A in Figure 9.2.4-1[6] within one measurement period. The filtered L3 measurement result is obtained as specified in section 5.5.3.2 [2]

Simulation assumptions collected in the table 5.1-1 are for FR1 and FR2:

Table 5.1-1 Simulation assumptions of FR1 and FR2

| Parameters | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz, 15/30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; Inter-frequency correlation model is optional. without UErotation,Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and optional LOSsoft; without UErotation,Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 20MHz | 80MHz |
| UE speed | 30,60,90 km/h for study targeting measurement reduction 60,90,120 km/h for study targeting HO performance improvement | 30,60,90 km/h for study targeting measurement reduction 60,90,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ 1,2 or 4 TX beams are assumed. | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ 8,16 or 32 TX beams are assumed |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) 1RX beam is assumed | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) 4RX beams are assumed |
| UE Antenna radiation pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 44dBm | 40 dBm (baseline) Other values (e.g., 34 dBm) not precluded |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | companies report one of the spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 | companies report one of the spatial consistency procedures: - Procedure A in TR38.901 - Procedure B in TR38.901 |
| UE trajectory model | 3 options in 38.843 section 6.3.1 | 3 options in 38.843 section 6.3.1 |
| UE trajectory boundary processing model | Companies report which of the following models they used: wrap-around model, circle-bouncing model, boundary-terminated model | Companies report which of the following models they used: wrap-around model, circle-bouncing model, boundary-terminated model |
| Sampling period | 40ms | 80ms |

Editor Note 1: This section intends to capture evaluation metrics, methodology and simulation assumptions common for all use cases

Editor Note 2: AI/ML model generalization could be studied after sufficient evaluation of different use cases.

Editor Note 3: No explicit result calibration (e.g., as in TR 36.839) is expected. Evaluation KPI(s) with detailed evaluation descriptions can be reported for cross-checking purposes.

5.2	RRM measurement prediction

5.2.1	Evaluation methodology, metrics and assumptions

Measurement prediction accuracy for cell-level RRM measurement prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases.

Measurement reduction rate for intra-frequency scenario is defined in the temporal domain (called MRRT) by assuming same length of measurement time instances and spatial domain respectively(called MRRS):

MRRT = skipped measurement time instances / total measurement time instances

MRRS = skipped beams to be measured/ total beams to be measured

In intra-frequency temporal domain case A, continuous measurement results in PW are predicted by continuous historical measurement result(s) in OW. Then OW and PW slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option), where measurement result(s) are actually measured before sliding. One example is illustrated in Figure 5.2.1-1:

Figure 5.2.1-1 Example of intra-frequency temporal domain case A

Intra-frequency intra-cell temporal domain case A prediction is evaluated for the 2nd study goal for both FR1 and FR2 scenario.

In intra-frequency temporal domain case B, measurement results in PW are predicted by historical measurement result(s) in OW. Then OW and PW slide forward with either sampling period(s) (with sliding L1/L3 filtering option) or measurement period(s) (with non-sliding L1/L3 filtering option) and measurement result(s) in previous PW is/are skipped during window sliding. Example 1 and example 2 are illustrated in Figure 5.2.1-2 and Figure 5.2.1-3 respectively, between which example 2 is recommended as baseline for evaluation.

Note: The historical measurement results in OW are at least actual measurement results. And Companies are free to report if they use predicted measurement results in OW as input of AI/ML model.

Figure 5.2.1-2 Example 1 of intra-frequency temporal domain case B

Figure 5.2.1-3 Example 2 of intra-frequency temporal domain case B

Intra-frequency intra-cell temporal domain case B prediction is evaluated for 1st study goal by predicting a sub set of measurement instances in temporal domain of the same cell for both FR1 and FR2 scenario. MRRT(s) should be aligned among companies without defining detail TDM pattern. Both case A and case B are applicable for all RRM sub-use cases and focus on at least pure temporal domain.

Intra-frequency intra-cell spatial domain prediction is evaluated for the 1st study goal by measuring a sub set of configured SSB as input to the model to derive L3 filtered cell-level measurements for every time instance of the same cell. It is only evaluated for FR2 intra-frequency scenario and is applicable for RRM sub-use case 1 and 3. MRRS(s) should be aligned among companies without defining detailed pattern.

For both Intra-frequency prediction inter-cell prediction and FR1 to FR1 inter-frequency inter-cell prediction, no measurement is reduced in both temporal and spatial domain for cell to be measured. For FR1 to FR1 inter-frequency inter-cell prediction, focus on the case where cell to be measured and cell to be predicted are located in the same sector of either serving site or same neighbouring site. If inter-frequency correlation model is assumed, section 7.6.5 in [4] is taken as baseline for inter-frequency correlation model. FR1 to FR1 inter-frequency inter-cell prediction is applicable for all RRM sub-use cases.

Intra-frequency inter-cell prediction refers to neighbouring cell prediction based on measurements of either co-located or non-collocated serving cell or neighbouring cell.

The prioritization among evaluation scenarios is captured in table 5.2.1-1

Table 5.2.1-1 prioritization of evaluation scenarios

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Following RRC parameters are assumed for RRM measurement prediction:

Table 5.2.1-2

| L3 filtering parameter | value |
| --- | --- |
| FR1 FilterCoefficient | 4 |
| FR2 FilterCoefficient | 4 |

Table 5.2.1-3

| Measurement period | value |
| --- | --- |
| FR1 to FR1 intra-frequency without gap | 200ms |
| FR1 to FR1 inter-frequency with gap | 200ms |
| FR2 to FR2 intra-frequency without gap | 400ms |

Table 5.2.1-4

| Consolidation parameter | value |
| --- | --- |
| nrofSS-BlocksToAverage for FR1 | 1 |
| nrofSS-BlocksToAverage for FR2 | 3 |
| absThreshSS-BlocksConsolidation for FR1 | -110dbm |
| absThreshSS-BlocksConsolidation for FR2 | -110dbm |

Editor Note 1: This section intends to capture RRM measurement prediction specific metrics, methodology and assumptions

Editor Note 2: RAN2 will start the evaluation with measurements prediction accuracy and model complexity.

Editor Note 3: The table 5.2.1-1 is captured for study purpose. It may be removed in the final TR.

Editor Note 4: Intra-frequency inter-cell prediction will not be evaluated at least in early stage.

Editor Note 5: Apart from FilterCoefficient, which is for both cell level and beam level RRM measurement prediction sub use cases, the rest parameters are for cell level RRM measurement prediction only so far.

5.2.2	Evaluation results

5.3	Measurement event prediction

5.3.1	Evaluation methodology, metrics and assumptions

Editor Note: This section intends to capture measurement event prediction specific metrics, methodology and assumptions

5.3.2	Evaluation results

5.4	RLF prediction

5.4.1	Evaluation methodology, metrics and assumptions

Editor Note 1: This section intends to capture RLF prediction specific metrics, methodology and assumptions

Editor Note 2: FR2 study will be prioritized for RLF prediction.

5.4.2	Evaluation results

6	Potential specification impact

6.1	LCM, protocol and procedure aspects

Editor Note: Discussion on mobility specific LCM, protocol and procedures are captured in this section.

Editor Note: This SID will reuse the common framework of LCM captured in sections 7.2.1 and 7.3.2 of 38.843 and the agreement concluded under WID NR_AIML_air-Core in principle. Anything mobility specific will be captured here.

6.1.1	Common aspects

Editor Note: Specification impacts common to all use cases are captured here

6.1.2	RRM measurement prediction

Editor Note: RRM measurement prediction specific part is captured here

6.1.3	Measurement event prediction

Editor Note: The measurement event prediction specific part is captured here

6.1.4	RLF/HOF prediction

Editor Note: RLF/HOF prediction specific part is captured here

6.2	Interoperability, testability, and RRM requirements

Editor Note: This section intends to capture the spec impact on testability, interoperability, and RRM requirements and performance [RAN4] based on SID.

7	Conclusion

Annex <A> (informative):
<Informative annex for a Technical Specification>

Informative annexes may appear in both Technical Specifications and Technical Reports. Use style "Heading 8" for use in TSs.

Informative annexes shall not contain requirements for the implementation of the Technical Specification.

A.1	Simulation template table

| Report parameters |  | Company A | …… |
| --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) |  |  |
|  | UE speed (30,60,90,120 Km/h) |  |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) |  |  |
|  | Inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) |  |  |
|  | Whether LOSsoft is modeled or not |  |  |
|  | spatial consistency option (A or B) |  |  |
|  | Number of TX beams |  |  |
|  | Number of RX beams |  |  |
|  | Measurement reduction rate(50%~80%Note2) |  |  |
|  | OW(Note3) |  |  |
|  | PW (Note3) |  |  |
|  | Any other parameters (Note 4) |  |  |
| Data Size (Number of Samples) | Training/validity |  |  |
|  | Testing |  |  |
| AI/ML model input/output | Model input (Note 5) |  |  |
|  | Model output(Note 6) |  |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) |  |  |
|  | Model complexity in a number of parameters(M) |  |  |
|  | Model complexity in model size (e.g. Mbyte) |  |  |
|  | Computational complexity [FLOPs] |  |  |
| Metrics | Average L3 cell-level RSRP difference (dBm) |  |  |
|  | Other optional KPIs (e.g., L1 beam-level RSRP difference,) |  |  |

Table A.1-1

Note1: Only applicable for FR1 to FR1 inter-frequency prediction.

Note2: Only applicable for intra-frequency prediction, either temporal domain case B or spatial domain. For FR1 to FR1 inter-frequency prediction, it is fixed i.e. no measurement will be performed on the frequency carrier to be predicted

Note3: For intra-frequency temporal domain case A,the ratio between OW and PW is at least limited to the value range {5,4,3,2,1,1/2,1/3,1/4,1/5}. And the length of OW and PW should be multiple times of sampling period or measurement period of FR1 or FR2 respectively. For intra-frequency temporal domain case B, the value range of OW is {40ms~2000ms} and the value range of PW is {40ms~800ms}.

Note4: This could be any other parameter e.g., BS antenna configuration, UE antenna configuration, BS TX power etc.

Note5: Apart from input of RRM sub-use case 1,2,3, other input information e.g. L1 filtering for L1 beam measurement, UE location , information of input cells are captured here too

Note6: Apart from output of RRM sub-use case 1,2,3, other output e.g. information of output cells is captured here too

Annex <B> (informative):
Change history

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

This is the last annex for TS/TSs which details the change history using the following table.
This table is to be used for recording progress during the WG drafting process till TSG approval of this TS/TR.
For TRs under change control, use one line per approved Change Request
Date: use format YYYY-MM
CR: four digits, leading zeros as necessary
Rev: blank, or number (max two digits)
Cat: use one of the letters A, B, C, D, F
Subject/Comment: for TSs under change control, include full text of the subject field of the Change Request cover
New vers: use format [n]n.[n]n.[n]n

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-06-11 | RAN2#126 | R2-2406096 |  |  |  | Endorsed skeleton | 0.02 |
| 2024-8-21 | RAN2#127 | R2-2406309 |  |  |  | Endorsed text proposal | 0.03 |