| 3GPP TR 38.746 V0.2.0 (2024-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; High power UE (power class 1.5 or 2) for NR inter-band Carrier Aggregation (CA) / Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s); (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	9

3.3	Abbreviations	9

4	Background	9

4.1	TR Maintenance	10

5	High Power UE CA of 2 bands DL and 1 or 2 bands UL	10

5.x	CA_nX-nY	10

5.x.1	Configurations	10

5.x.2	Maximum output power	10

5.x.3	REFSENS requirements	11

5.x.3.1	Power class 2 case a	11

5.x.3.2	Power class 2 case b	11

5.x.3.3	Power class 2 case c	11

5.x.3.4	Power class 1.5 case a	11

5.x.3.5	Power class 1.5 case b	11

5.x.3.6	Power class 1.5 case c	11

5.x.3.7	Power class 1.5 case d	11

5.x.3.8	Power class 1.5 case e	11

5.x.3.9	Power class 1.5 for single uplink nX	11

5.x.3.10	Power class 1.5 for single uplink nY	11

5.x.4	∆TIB and ∆RIB values	11

5.1	CA_n1-n78	11

5.1.1	Operating bands for CA	11

5.1.2	Maximum output power for inter-band CA	11

5.1.3	MSD scenario studies	12

5.1.4	REFSENS requirements	12

5.1.4.1	REFSENS requirements for total power class 2	12

5.1.4.2	REFSENS requirements for total power class 1.5	12

5.2	CA_n3-n78	13

5.2.1	Operating bands for CA	13

5.2.2	Maximum output power for inter-band CA	13

5.2.3	MSD scenario studies	13

5.2.4	REFSENS requirements	14

5.2.4.1	REFSENS requirements for total power class 2	14

5.2.4.2	REFSENS requirements for total power class 1.5	14

5.3	CA_n7-n78	15

5.3.1	Operating bands for CA	15

5.3.2	Maximum output power for inter-band CA	15

5.3.3	MSD scenario studies	15

5.3.4	REFSENS requirements	16

5.3.4.1	REFSENS requirements for total power class 2	16

5.3.4.2	REFSENS requirements for total power class 1.5	16

5.4	CA_n28-n78	16

5.4.1	Operating bands for CA	16

5.4.2	Maximum output power for inter-band CA	17

5.4.3	MSD scenario studies	17

5.4.4	REFSENS requirements	17

5.4.4.1	REFSENS requirements for total power class 2	17

5.4.4.2	REFSENS requirements for total power class 1.5	18

5.5	CA_n1-n77	18

5.5.1	Operating bands for CA	18

5.5.2	Maximum output power for inter-band CA	18

5.5.3	MSD scenario studies	18

5.5.4	REFSENS requirements	19

5.5.4.1	REFSENS requirements for total power class 2	19

5.6	CA_n3-n77	19

5.6.1	Operating bands for CA	19

5.6.2	Maximum output power for inter-band CA	19

5.6.3	MSD scenario studies	19

5.6.4	REFSENS requirements	20

5.6.4.1	REFSENS requirements for total power class 2	20

5.7	CA_n28-n77	20

5.7.1	Operating bands for CA	20

5.7.2	Maximum output power for inter-band CA	20

5.7.3	MSD scenario studies	20

5.7.4	REFSENS requirements	21

5.7.4.1	REFSENS requirements for total power class 2	21

5.8	CA_n26-n78	21

5.8.1	Operating bands for CA	21

5.8.2	Maximum output power for inter-band CA	21

5.8.3	MSD scenario studies	21

5.8.4	REFSENS requirements	22

5.8.4.1	REFSENS requirements for total power class 2	22

5.8.4.2	REFSENS requirements for total power class 1.5	22

6	High Power UE CA of 3 bands DL and 1 or 2 bands UL	22

6.x	CA_nX-nY-nZ	22

6.x.1	Configurations	22

6.x.2	Maximum output power	23

6.x.3	REFSENS requirements	23

6.x.3.1	Power class 2 case a	23

6.x.4	∆TIB and ∆RIB values	23

6.1	CA_n1-n18-n41	23

6.1.1	Configurations	23

6.1.2	Maximum output power	24

6.1.3	REFSENS requirements	24

6.1.3.1	Power class 2 case a, b	24

6.1.4	∆TIB and ∆RIB values	24

6.2	CA_n3-n18-n41	25

6.2.1	Configurations	25

6.2.2	Maximum output power	25

6.2.3	REFSENS requirements	25

6.2.3.1	Power class 2 case a, b	25

6.2.4	∆TIB and ∆RIB values	26

6.3	CA_n18-n28-n41	26

6.3.1	Configurations	26

6.3.2	Maximum output power	26

6.3.3	REFSENS requirements	26

6.3.3.1	Power class 2 case a, b	26

6.3.4	∆TIB and ∆RIB values	27

6.4	CA_n1-n18-n77	27

6.4.1	Configurations	27

6.4.2	Maximum output power	27

6.4.3	REFSENS requirements	27

6.4.3.1	Power class 2 case a, b	27

6.4.4	∆TIB and ∆RIB values	28

6.5	CA_n1-n26-n78	29

6.5.1	Configurations	29

6.5.2	Maximum output power	29

6.5.3	REFSENS requirements	30

6.5.3.1	Power class 2 case a, b	30

6.5.4	∆TIB and ∆RIB values	30

6.6	CA_n3-n26-n78	31

6.6.1	Configurations	31

6.6.2	Maximum output power	33

6.6.3	REFSENS requirements	33

6.6.3.1	Power class 2 case a, b	33

6.6.4	∆TIB and ∆RIB values	33

7	High Power UE CA with SUL	33

7.x	CA_nW-nX_SUL_nY-nZ	33

7.x.1	Configurations	33

7.x.2	Maximum output power	34

7.x.3	REFSENS requirements	34

7.x.4	∆TIB and ∆RIB values	34

Annex A:  Valid UL configurations	35

Annex B (informative): Change history	35

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is one of four technical reports for High power UE (power class 1.5 or 2) for NR Inter-band Carrier Aggregation (CA)/Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s) under Rel-19 timeframe. The objective is to gather the relevant background information and studies in order to complete the band-combination specific requirements for the newly requested band combinations for power class 2 and power class 1.5 UE. Table 1-1 lists the band combinations covered by this TR with three power class cases for uplink.

Table 1-1 High power UE (power class 1.5 or 2) inter-band CA/DC and SUL band combinations

| # | Band combination List | Power class cases for uplink |
| --- | --- | --- |
| 1 | High power UE (power class 1.5 or 2) for NR Inter-band Carrier Aggregation (CA)/Dual connectivity (DC) with/without Supplementary uplink (SUL) with high power on TDD band(s) Note: Including PC3 FDD/TDD+ PC3 TDD case | PC2 inter-band CA/DC: 1UL(TDD): PC2 on TDD band 2UL (TDD+TDD): PC3 or PC2 on TDD band 2UL (FDD+TDD): PC3 on FDD band, PC3 or PC2 on TDD band |
|  |  | PC1.5 inter-band CA/DC: 1UL(TDD): PC 1.5 on TDD band 2UL (TDD+TDD): PC3, PC2 or PC1.5 on TDD band 2UL (FDD+TDD): PC3 on FDD band, PC1.5 on TDD band |
|  |  | PC2 SUL band combinations with or without CA: SUL: PC3 or PC2 on SUL band NUL(TDD): PC2 on TDD band PC1.5 SUL band combinations with or without CA: SUL: PC3 or PC2 on SUL band NUL(TDD): PC1.5 on TDD band NUL = Normal Uplink in contrast to SUL. |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-241679: “Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)”.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, for serving cell c.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

BS				Base Station

BCS				Bandwidth Combination Set

CA				Carrier Aggregation

CC				Component Carriers

DC				Dual Connectivity

DL				Downlink

FDD				Frequency Division Duplex

MPR 				Allowed maximum power reduction

MSD				Maximum Sensitivity Degradation

REFSENS				Reference Sensitivity power level

SCS				Subcarrier spacing

SUL 				Supplementary uplink

TDD				Time Division Duplex

UE				User Equipment

UL				Uplink

# 4	Background

At 3GPP RAN#104 meeting, a basket Work Item on “High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary uplink (SUL)” [2] was approved for Rel-19. There are four objectives of the core part of the basket work item. The objectives covered by this TR are as follows:

Specify the band-combination specific RF requirements for all band combinations cases captured in table 1-1. The band combinations are defined in the attached excel file of this WI.

The requirements that need to analyse and specify include

-	Maximum output power, Tx power tolerance and A-MPR (Additional Maximum Power Reduction) requirements if needed

-	Analysing combinations that have self-desensitization, applicable ∆TIB, c and ∆RIB, c and reference sensitivity exceptions including MSD test cases.

-	Other additional impact to the requirements due to the high power on UL, if necessary

The present document is a technical report for this basket Work Item.

## 4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band/band combination to ensure that the TPs related to the band/band combination have been implemented.

# 5	High Power UE CA of 2 bands DL and 1 or 2 bands UL

## 5.x	CA_nX-nY

### 5.x.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5A.3.1-1x unless additional clarification>

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nX-nY | nX8,9, nY8,9, CA_nX-nY8,9 | nX |  |  |
|  |  | nY |  |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

### 5.x.2	Maximum output power

<Editor’s note: In table 5.x.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 5.x.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

Table 5.x.2-2 UE Power Class 1.5 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 1.5 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 29dBm | 26dBm | 26dBm |
|  | Case b | 29dBm | 23dBm | 29dBm |
|  | Case c | 29dBm | 29dBm | 23dBm |
|  | Case d | 29dBm | 26dBm | 29dBm |
|  | Case e | 29dBm | 29dBm | 26dBm |

### 5.x.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. >

#### 5.x.3.1	Power class 2 case a

#### 5.x.3.2	Power class 2 case b

#### 5.x.3.3	Power class 2 case c

#### 5.x.3.4	Power class 1.5 case a

#### 5.x.3.5	Power class 1.5 case b

#### 5.x.3.6	Power class 1.5 case c

#### 5.x.3.7	Power class 1.5 case d

#### 5.x.3.8	Power class 1.5 case e

#### 5.x.3.9	Power class 1.5 for single uplink nX

#### 5.x.3.10	Power class 1.5 for single uplink nY

### 5.x.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

## 5.1	CA_n1-n78

### 5.1.1	Operating bands for CA

Table 5.x.1-1:  CA band combination of band n1+n78

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n1-n78 | n1, n78 |

### 5.1.2	Maximum output power for inter-band CA

Table 5.1.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.1.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.1.3	MSD scenario studies

Table 5.13-1 and 5.1.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n1-n78.

Table 5.1.3-1: MSD scenarios for CA_n1-n78 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | N/A | N/A | N/A |
| IMD4 | 3 * n1 – n78 | n1 | Yes |

Table 5.1.3-2: MSD scenarios for CA_n1-n78 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | N/A | N/A | N/A |
| IMD4 | 3 * f n1 – f n78 | n1 | No |

### 5.1.4	REFSENS requirements

#### 5.1.4.1	REFSENS requirements for total power class 2

The 2Tx MSD due to IMD4 has already been specified, which is applicable for 3Tx.

#### 5.1.4.2	REFSENS requirements for total power class 1.5

- The MSD due to IMD4 for PC1.5 3Tx are specified in the following table. The IMD4 MSD values for CA_n1-n78 and CA_n2-n77 are the same. Therefore, the CA_n1-n78 PC1.5 MSD values are reused from CA_n2-n77 PC1.5 MSD values.

Table 5.1.4.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78 | n1 |  | 1950 | 5 | 25 | 2140 | 26.4 | FDD | IMD4 |
|  | n78 | 3710 |  | 10 | 50 | 3710 | N/A | TDD | N/A |

-	Cross band MSD is defined for CA_n2-n77 therefore it is also needed for CA_n1-n78 since band n1 is even closer than band n2. Same MSD value as cor CA_n2-n77 are used.

Table 5.1.4.2-2: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n1 | 3350 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 | 2.7 | >ACLR2 |

## 5.2	CA_n3-n78

### 5.2.1	Operating bands for CA

Table 5.2.1-1:  CA band combination of band n3+n78

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n3-n78 | n3, n78 |

### 5.2.2	Maximum output power for inter-band CA

Table 5.2.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.2.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.2.3	MSD scenario studies

Table 5.x.3-1 and 5.x.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n3-n78.

Table 5.2.3-1: MSD scenarios for CA_n3-n78 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n78 (UL1/DL2) | n3 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD2 | n78 – n3 | n3 | Yes |
| IMD4 | 3 * n3 – n78 | n3 | Yes |

Table 5.2.3-2: MSD scenarios for CA_n3-n78 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n78 (UL1/DL2) | n3 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD2 | f n78 – f n3 | n3 | No |
| IMD4 | 3 * f n3 – f n78 | n3 | No |

### 5.2.4	REFSENS requirements

#### 5.2.4.1	REFSENS requirements for total power class 2

-	The 2Tx MSD due to harmonic mixing has already been specified, which is applicable for 3Tx.

-	The 2Tx MSD due to IMD2 has already been specified, which is applicable for 3Tx.

-	The 2Tx MSD due to IMD4 has already been specified, which is applicable for 3Tx.

#### 5.2.4.2	REFSENS requirements for total power class 1.5

-	The 2Tx MSD due to harmonic mixing has already been specified, which is applicable for 3Tx.

-	The MSD due to IMD2 and IMD4 for PC1.5 3Tx are specified in the following table. The IMD2 and IMD4 MSD values for CA_n3-n78 and CA_n2-n77 are the same. Therefore, the CA_n3-n78 PC1.5 MSD values are reused from CA_n2-n77 PC1.5 MSD values.

Table 5.2.4.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n78 | n3 |  | 1750 | 5 | 25 | 1845 | 35.2 | FDD | IMD2 |
|  | n78 |  | 3595 | 10 | 50 | 3595 | N/A | TDD | N/A |
|  | n3 | 1770 |  | 5 | 25 | 1865 | 26.4 | FDD | IMD4 |
|  | n78 | 3445 |  | 10 | 50 | 3445 | N/A | TDD | N/A |

Harmonic mixing defined in Table 5.x.4.2-2 below.

Table 5.2.4.2-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC1.5 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n3 | 5 | 15 | 25 (RBstart=0) | 5 | 11.1 | NOTE 4 | UL1/DL2 |
| n78 | n3 | 40 | 15 | 216 (RBstart=0) | 40 | 4 | NOTE 4 | UL1/DL2 |

## 5.3	CA_n7-n78

### 5.3.1	Operating bands for CA

Table 5.3.1-1:  CA band combination of band n7+n78

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n7-n78 | n7, n78 |

### 5.3.2	Maximum output power for inter-band CA

Table 5.3.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.3.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.3.3	MSD scenario studies

Table 5.x.3-1 and 5.x.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_ n7-n78.

Table 5.3.3-1: MSD scenarios for CA_n7-n78 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | n78 | n7 | Yes |
| IMD | N/A | N/A | N/A |

Table 5.3.3-2: MSD scenarios for CA_n7-n78 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | n78 | n7 | Yes |
| IMD | N/A | N/A | N/A |

### 5.3.4	REFSENS requirements

#### 5.3.4.1	REFSENS requirements for total power class 2

-	The 2Tx MSD due to cross band isolation has already been specified, which is applicable for 3Tx.

-	The 2Tx MSD due to IMD4 has already been specified, which is applicable for 3Tx.

#### 5.3.4.2	REFSENS requirements for total power class 1.5

-	The 2Tx MSD due to cross band isolation is specified below with same MSD value as used for CA_n7-n78.

Table 5.3.4.2-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC1.5 aggressor NR single UL band for DL NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n7 | 3350 | 100 | 30 | 270 (RBstart=0) | 2687.5 | 5 | 9.0 | >ACLR2 |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: Void. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

## 5.4	CA_n28-n78

### 5.4.1	Operating bands for CA

Table 5.4.1-1:  CA band combination of band n28+n78

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n28-n78 | n28, n78 |

### 5.4.2	Maximum output power for inter-band CA

Table 5.4.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.4.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.4.3	MSD scenario studies

Table 5.x.3-1 and 5.x.3-2 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n28-n78.

Table 5.4.3-1: MSD scenarios for CA_n28-n78 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n78 (UL1/DL5) | n28 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD5 | 4 * n28 – n78 | n28 | No |

Table 5.4.3-2: MSD scenarios for CA_n28-n78 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n78 (UL1/DL5) | n28 | No |
| Cross band leakage | N/A | N/A | N/A |
| IMD5 | 4 * f n28 – f n78 | n28 | No |

### 5.4.4	REFSENS requirements

#### 5.4.4.1	REFSENS requirements for total power class 2

- The MSD due to IMD5 for PC2 is specified in the following table.

Table 5.4.4.1-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n78 | n28 |  | 705.5 | 5 | 25 | 760.5 | 19.2 | FDD | IMD5 |
|  | n78 | 3582.5 |  | 10 | 50 | 3582.5 | N/A | TDD | N/A |

#### 5.4.4.2	REFSENS requirements for total power class 1.5

- The MSD due to IMD5 for PC1.5 3Tx are specified in the following table. The IMD5 MSD values for CA_n28-n78 and CA_n5-n77 are the same. Therefore, the CA_n28-n78 PC1.5 MSD values are reused from CA_n5-n77 PC1.5 MSD values.

Table 5.4.4.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n78 | n28 |  | 705.5 | 5 | 25 | 760.5 | 24.3 | FDD | IMD5 |
|  | n78 | 3582.5 |  | 10 | 50 | 3582.5 | N/A | TDD | N/A |

## 5.5	CA_n1-n77

### 5.5.1	Operating bands for CA

Table 5.5.1-1:  CA band combination of band n1+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n1-n77 | n1, n77 |

### 5.5.2	Maximum output power for inter-band CA

Table 5.5.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n77A |  |  | 267 | +2/-3 | 23 | +2/-3 |
| NOTE 7: The UE supports PC3 with UL MIMO in FDD band and PC3 or PC2 in TDD band. |  |  |  |  |  |  |

### 5.5.3	MSD scenario studies

For inter-band UL CA_n1A-n77A, IMD2 and IMD4 fall into n1 DL, and the corresponding MSD values are already specified in Table 7.3A.5-1a of latest version of TS38.101-1[4].

Table 5.x.3-1 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n1-n77.

Table 5.5.3-1: MSD scenarios for CA_n1-n77 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | N/A | N/A | N/A |
| Cross band leakage | N/A | N/A | N/A |
| IMD2 | 1*f_n77-1*f_n1 | n1 DL | Yes |
| IMD4 | 3*f_n1-1*f_n77 | n1 DL | Yes |

### 5.5.4	REFSENS requirements

#### 5.5.4.1	REFSENS requirements for total power class 2

-	The 2Tx MSD due to IMD2 has already been specified, which is applicable for 3Tx.

-	The 2Tx MSD due to IMD4 has already been specified, which is applicable for 3Tx.

## 5.6	CA_n3-n77

### 5.6.1	Operating bands for CA

Table 5.6.1-1:  CA band combination of band n3+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n3-n77 | n3, n77 |

### 5.6.2	Maximum output power for inter-band CA

Table 5.6.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n3A-n77A |  |  | 267 | +2/-3 | 23 | +2/-3 |
| NOTE 7: The UE supports PC3 with UL MIMO in FDD band and PC3 or PC2 in TDD band. |  |  |  |  |  |  |

### 5.6.3	MSD scenario studies

The 2nd order harmonic mixing falls into Rx frequencies of n3, and the corresponding MSD values are already specified in Table 7.3A.4.4a-1 of latest version of TS38.101-1[4].

For inter-band UL CA_n3A-n77A, IMD2, IMD4 and IMD5 of dual UL falls into n3 DL, and the corresponding MSD values are already specified in Table 7.3A.5-1a of latest version of TS38.101-1[4].

Table 5.x.3-1 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n3-n77.

Table 5.6.3-1: MSD scenarios for CA_n3-n77 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n77(UL1/DL2) | n3 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD2 | 1*f_n77 - 1*f_n3 | n3 DL | Yes |
| IMD4 | 3*f_n3 – 1*f_n77 | n3 DL | Yes |

### 5.6.4	REFSENS requirements

#### 5.6.4.1	REFSENS requirements for total power class 2

-	The 2Tx MSD due to harmonic mixing has already been specified, which is applicable for 3Tx.

-	The 2Tx MSD due to IMD2 has already been specified, which is applicable for 3Tx.

-	The 2Tx MSD due to IMD4 has already been specified, which is applicable for 3Tx.

## 5.7	CA_n28-n77

### 5.7.1	Operating bands for CA

Table 5.7.1-1:  CA band combination of band n28+n77

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n28-n77 | n28, n77 |

### 5.7.2	Maximum output power for inter-band CA

Table 5.7.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n77A |  |  | 267 | +2/-3 | 23 | +2/-3 |
| NOTE 7: The UE supports PC3 with UL MIMO in FDD band and PC3 or PC2 in TDD band. |  |  |  |  |  |  |

### 5.7.3	MSD scenario studies

The 5th order harmonic mixing falls into Rx frequencies of n28, and the corresponding MSD values are already specified in Table 7.3A.4.4a-1 of latest version of TS38.101-1[4].

For inter-band UL CA_n28A-n77A, IMD5 of dual UL falls into n28 DL, and the corresponding MSD values are already specified in Table 7.3A.5-1a of latest version of TS38.101-1[4].

Table 5.x.3-1 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_n28-n77.

Table 5.7.3-1: MSD scenarios for CA_n28-n77 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n77(UL1/DL5) | n28 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD5 | 1*f_n28 - 4*f_n77 | n28 DL | Yes |

### 5.7.4	REFSENS requirements

#### 5.7.4.1	REFSENS requirements for total power class 2

-	The 2Tx MSD due to harmonic mixing has already been specified, which is applicable for 3Tx.

-	The 2Tx MSD due to IMD5 has already been specified, which is applicable for 3Tx.

## 5.8	CA_n26-n78

### 5.8.1	Operating bands for CA

Table 5.8.1-1:  CA band combination of band n26+n78

| NR CA Band combination | NR Band |
| --- | --- |
| CA_n26-n78 | n26, n78 |

### 5.8.2	Maximum output power for inter-band CA

Table 5.8.2-1: UE Power Class for 3Tx inter-band CA with UL MIMO

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n26A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. |  |  |  |  |  |  |

Table 5.8.2-2: UE Power Class for 3Tx inter-band CA with TxD

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n26A-n78A | 293 | +2/-3 | 262 | +2/-3 | 23 | +2/-3 |
| NOTE 1: (Void in this TR) NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. |  |  |  |  |  |  |

### 5.8.3	MSD scenario studies

Table 5.x.3-1 and 5.x.3-2 summarizes frequency ranges where harmonic mixing, cross band leakage, IMD interferences occur for CA_n26-n78.

Table 5.8.3-1: MSD scenarios for CA_n26-n78 with total power class 2

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n78 (UL1/DL4) | n26 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD4 | f n78 – 3 * f n26 | n26 | Yes |

Table 5.8.3-2: MSD scenarios for CA_n26-n78 with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | n78 (UL1/DL4) | n26 | Yes |
| Cross band leakage | N/A | N/A | N/A |
| IMD4 | f n78 – 3 * f n26 | n26 | No |

### 5.8.4	REFSENS requirements

#### 5.8.4.1	REFSENS requirements for total power class 2

-	The MSD due to harmonic mixing PC2 is already specified in 38.101-1.

-	The MSD due to IMD4 for PC2 is already specified in 38.101-1.

#### 5.8.4.2	REFSENS requirements for total power class 1.5

-	The MSD due to harmonic mixing PC1.5 is already specified in 38.101-1.

-	The MSD due to IMD5 for PC1.5 3Tx are specified in the following table. MSD value is reused from CA_n5-n77,

Table 5.8.4.2-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC1.5 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band |  | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n26-n78 | n26 |  | 836.5 | 5 | 25 | 881.5 | 25.5 | FDD | IMD4 |
|  | n78 | 3391 |  | 10 | 50 | 3391 | N/A | TDD | N/A |

# 6	High Power UE CA of 3 bands DL and 1 or 2 bands UL

## 6.x	CA_nX-nY-nZ

### 6.x.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5A.3.2-1x unless additional clarification>

Table 6.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nX-nY-nZ | nX7,9, nY7,9, CA_nX-nY7,9 | nX |  |  |
|  |  | nY |  |  |
|  |  | nZ |  |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.x.2	Maximum output power

<Editor’s note: In table 6.x.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 6.x.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

Table 6.x.2-2 UE Power Class 1.5 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 1.5 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_nX-nY | Case a | 29dBm | 26dBm | 26dBm |
|  | Case b | 29dBm | 23dBm | 29dBm |
|  | Case c | 29dBm | 29dBm | 23dBm |
|  | Case d | 29dBm | 26dBm | 29dBm |
|  | Case e | 29dBm | 29dBm | 26dBm |

### 6.x.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. The requirements in this TR are intended to be power class 2 cases based, however how to address in the spec will be further discussed. >

#### 6.x.3.1	Power class 2 case a

### 6.x.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

## 6.1	CA_n1-n18-n41

### 6.1.1	Configurations

Table 6.1.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n18A-n41A | CA_n1A-n41A7 CA_n18A-n41A7 | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.1.2	Maximum output power

Table 6.1.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n18-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.1.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.1.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL CA_n1A-n41A falls into n18 DL. The MSD exception is defined as below.

Table 6.1.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n18-n41 | n1 | 1960 | 5 | 25 | 2150 | N/A | FDD | N/A |
|  | n41 | 2505 | 10 | 50 | 2505 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 12.4 | FDD | IMD5 |

### 6.1.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.2	CA_n3-n18-n41

### 6.2.1	Configurations

Table 6.2.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n18A-n41A | CA_n3A-n41A7 CA_n18A-n41A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.2.2	Maximum output power

Table 6.2.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n18-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.2.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.2.3.1	Power class 2 case a, b

Based on calculation, IMD2 and IMD3 of dual UL CA_n3A-n41A fall into n18 DL, and IMD2 of dual UL CA_n18A-n41A falls in to n3 DL. The MSD exceptions are defined as follows.

Table 6.2.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n18-n41 | n18 | N/A | 5 | N/A | 865 | 34.9 | FDD | IMD2 |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | FDD | N/A |
|  | n41 | 2630 | 10 | 50 | 2630 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 875 | 28 | FDD | IMD3 |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n41 | 2670 | 5 | 25 | 2670 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1850 | 34.8 | FDD | IMD2 |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | TDD | N/A |
|  | n18 | 820 | 5 | 25 | 865 | N/A | FDD | N/A |

### 6.2.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.3	CA_n18-n28-n41

### 6.3.1	Configurations

Table 6.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n28A-n41A | CA_n18A-n41A7 CA_n28A-n41A7 | n18 | 5, 10, 15 | 0 |
|  |  | n28 | 5, 10 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.3.2	Maximum output power

Table 6.3.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n18-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n28-n41 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.3.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.3.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL CA_n18A-n41A falls into n28 DL. The MSD exception is defined as below.

Table 6.3.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n28-n41 | n18 | 825 | 5 | 25 | 870 | N/A | FDD | N/A |
|  | n41 | 2505 | 10 | 50 | 2505 | N/A | TDD | N/A |
|  | n28 | N/A | 5 | N/A | 795 | 16.9 | FDD | IMD5 |

### 6.3.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.4	CA_n1-n18-n77

### 6.4.1	Configurations

Table 6.4.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n18A-n77A CA_n1A-n18A-n77(2A) | CA_n1A-n77A7 CA_n18A-n77A7 | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n18 | 5, 10, 15 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | CA_n77(2A)_BCS1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination. |  |  |  |  |

### 6.4.2	Maximum output power

Table 6.4.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n18-n77 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.4.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.4.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL CA_n1A-n77A falls into n18 DL, IMD3 of dual UL CA_n18A-n77A falls into n1 DL, the MSD exceptions are defined as below

Table 6.4.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n18-n77 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
| CA_n1-n18-n77(2A) | n77 | 3390 | 10 | 50 | 3390 | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 18.6 | FDD | IMD5 |
|  | n1 | N/A | 5 | N/A | 2120 | 25.4 | FDD | IMD3 |
|  | n18 | 825 | 5 | 25 | 870 | N/A | FDD | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | TDD | N/A |

### 6.4.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

R4-2414295-298

## 6.5	CA_n1-n26-n78

### 6.5.1	Configurations

Table 6.5.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n26A-n78A | CA_n1A-n26A CA_n1A-n78A7 CA_n26A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n26A-n78C | CA_n78C CA_n1A-n26A CA_n1A-n78A7 CA_n26A-n78A7 | n1 | 5, 10, 15, 20 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n1A-n26(2A)-n78A | CA_n1A-n26A CA_n1A-n78A7 CA_n26A-n78A7 CA_n26(2A) | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n1A-n26A-n78(2A) | CA_n1A-n26A CA_n1A-n78A7 CA_n26A-n78A7 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n26(2A)-n78(2A) | CA_n1A-n26A CA_n1A-n78A7 CA_n26A-n78A7 CA_n26(2A) | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n1A-n26(2A)-n78C | CA_n26(2A) CA_n78C CA_n1A-n26A CA_n1A-n78A7 CA_n26A-n78A7 | n1 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.5.2	Maximum output power

Table 6.5.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n1-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n26-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.5.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.5.3.1	Power class 2 case a, b

Based on calculation, IMD5 of dual UL CA_n1A-n78A falls into n26 DL.

Based on calculation, IMD3 of dual UL CA_n26A-n78A falls into n1 DL.

The MSD exception is defined as below.

Table 6.5.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n26-n78 | n1 | N/A | 5 | N/A | 2122 | 27.0 | FDD | IMD3 |
|  | n26 | 829 | 5 | 25 | 874 | N/A | FDD | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | TDD | N/A |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n26 | N/A | 5 | N/A | 885 | 15.3 | FDD | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | TDD | N/A |

### 6.5.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

## 6.6	CA_n3-n26-n78

### 6.6.1	Configurations

Table 6.6.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| CA_n3A-n26A-n78A | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n26 | 5, 10, 15, 20 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n26A-n78C | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n78C | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n3A-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n26(2A) | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3A-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n26(2A) | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3A-n26(2A)-n78C | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n26(2A) CA_n78C | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| CA_n3B-n26A-n78A | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n26A-n78(2A) | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n26A-n78C | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n78C | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n78 | CA_n78B_BCS0 |  |
| CA_n3B-n26(2A)-n78A | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n26(2A) | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n3B-n26(2A)-n78(2A) | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n26(2A) | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
| CA_n3B-n26(2A)-n78C | CA_n3A-n26A CA_n3A-n78A7 CA_n26A-n78A7 CA_n26(2A) CA_n78C | n3 | CA_n3B_BCS0 | 0 |
|  |  | n26 | CA_n26(2A)_BCS0 |  |
|  |  | n78 | CA_n78C_BCS0 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 6.6.2	Maximum output power

Table 6.6.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n3-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
| CA_n26-n78 | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

### 6.6.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 6.6.3.1	Power class 2 case a, b

Based on calculation, no IMD of dual UL CA_n3A-n78A falls into n26 DL.

Based on calculation, IMD3 of dual UL CA_n26A-n78A falls into n3 DL.

The MSD exception is defined as below.

Table 6.6.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC2 CA

| CA_n3-n26-n78 | n3 | N/A | 5 | N/A | 1862 | 24.6 | FDD | IMD3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n26 | 839 | 5 | 25 | 884 | N/A | FDD | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |

### 6.6.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 CA.

# 7	High Power UE CA with SUL

## 7.x	CA_nW-nX_SUL_nY-nZ

### 7.x.1	Configurations

<Editor’s note: the CA configurations and bandwidth combinations sets should be kept same as defined in 38.101-1 Table 5.5C-4 unless additional clarification>

Table 7.x.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink configuration | NR Band | Channel Bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nW-nX_SUL_nY-nZ | SUL_nY-nZ | nW |  |  |
|  |  | nX |  |  |
|  |  | nY |  |  |
|  |  | nZ |  |  |

### 7.x.2	Maximum output power

### 7.x.3	REFSENS requirements

<Editor’s note: This agenda will capture the Reference sensitivity exceptions or MSD requirements due to higher power for  CA carrier, please use the same table format as in 38101-1. >

### 7.x.4	∆TIB and ∆RIB values

<Editor’s note: If no change by comparing to the values for power class 3 CA, this section will be void.>

Annex A: 
Valid UL configurations

For High Power UE CA of 2 or 3 bands DL with 1 or 2 bands UL, the following UL configurations are applicable, subject to the general requirements:

-	One band UL with one CC: nXA, nYA

-	One band UL with two CC: CA_nXB, CA_nXC, CA_nX(2A), CA_nYB, CA_nYC, CA_nY(2A)

-	wo band UL with one CC per band: CA_nXA-nYA

-	two band UL with two CC in one band: CA_nXB-nYA, CA_nXC-nYA, CA_nXA-nYB, CA_nXA-nYC

The following three UL cluster cases are not supported:

-	CA_nX(2A)-nYA, CA_nXA-nY(2A)

Combinations with four UL CCs are not supported.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | RAN4 #112 | R4-2412268 |  |  |  | TR skeleton | 0.1.0 |
| 2024-10 | RAN4 #112-bis | R4-2414979 |  |  |  |  | 0.2.0 |