| 3GPP TR 28.878 V1.1.0 (2024-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Management of Network Sharing Phase3; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Concepts and background	7

4.1	Indirect Network Sharing	7

5	Use Cases	7

5.1	Use Case#1: Service-based management architecture and access requirements for MOCN	7

5.1.1	Description	7

5.1.2	Potential requirements	7

5.1.3	Potential solutions	8

5.1.3.1	Potential solution #1: Enhance the existing service-based management architecture and management workflow for MOCN	8

5.1.4	Evaluation of potential solutions	8

5.2	Use Case#2: Trace job and collection requirements for POPs	8

5.2.1	Description	8

5.2.2	Potential requirements	9

5.2.3	Potential solutions	9

5.2.4	Evaluation of potential solutions	9

5.3	Use Case#3: S-RAN management of Indirect Network Sharing	9

5.3.1	Description	9

5.3.2	Potential requirements	9

5.3.3	Potential solutions	9

5.3.4	Evaluation of potential solutions	10

5.4	Use Case#4: RAN configuration management support for Indirect Network Sharing	10

5.4.1	Description	10

5.4.2	Potential requirements	10

5.4.3	Potential solutions	10

5.4.4	Evaluation of potential solutions	10

5.5	Use Case#5: CN configuration management support for Indirect Network Sharing	11

5.5.1	Description	11

5.5.2	Potential requirements	11

5.5.3	Potential solutions	11

5.5.4	Evaluation of potential solutions	11

5.6	Use Case#6: performance management support for Indirect Network Sharing	11

5.6.1	Description	11

5.6.2	Potential requirements	12

5.6.3	Potential solutions	12

5.6.4	Evaluation of potential solutions	12

6	Conclusions and recommendations	12

6.1	Use Case#1: Service-based management architecture and access requirements for MOCN	12

6.2	Use Case#2: Trace job and collection requirements for POPs	13

6.3	Use Case#3: S-RAN management of Indirect Network Sharing	13

6.4	Use Case#4: RAN configuration management support for Indirect Network Sharing	13

6.5	Use Case#5: CN configuration management support for Indirect Network Sharing	13

6.6	Use Case#6: performance management support for Indirect Network Sharing	13

Annex A:	Change history	15

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document study the management of network sharing including Trace job and collection requirements for POPs, some issues related Rel-18, such as enhancements for SBMA based MOCN scenario, scenario related to access rights on CRUD operation of MOCN management data, and potential enhancements for Indirect Network Sharing scenarios.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TR 22.851: "Study on Network Sharing Aspects".

[4]	3GPP TS 32.130: "Telecommunication management; Network sharing; Concepts and requirements".

[5]	3GPP TR 28.835: "Study on management aspects of 5G Multiple Operator Core Network (MOCN) network sharing phase 2".

[6]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

[7]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[8]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[9]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[10]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1]. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

Void.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

GWCN	Gateway Core Network

MOCN	Multiple Operator Core Network

4	Concepts and background

Network sharing is a crucial approach to decrease CAPEX of 5G constructions for operators. As stated in 3GPP TS 22.261 [2] the increased density of access nodes needed to meet future performance objectives pose considerable challenges in deployment and acquiring spectrum and antenna locations. RAN sharing is seen as a technical solution to these issues. Plenty of previous normative works have been completed during Rel-17 and Rel-18.

During Rel-18, SA5 has specified new scenarios for MOCN Network sharing, SBMA deployment examples for MOCN management architectures and related NRM enhancements in 3GPP TS 32.130 [4]. Some issues for existing MOCN, including Trace job and collection requirements for POPs, etc., are supposed to be further studied.

During Rel-19, Indirect Network Sharing has been introduced in 3GPP TS 22.261 [2]. Potential management enhancements are supposed to be studied.

4.1	Indirect Network Sharing

According to the definition in clause 6.21.2.2 of 3GPP TS 22.261 [2], Indirect Network Sharing is a type of NG-RAN Sharing in which the communication between the Shared NG-RAN and the Participating Operator's core network is routed through the Hosting NG-RAN Operator's core network. In RAN Sharing operations, NG-RAN resources can be used by multiple network operators. Indirect Network Sharing is one of the possible sharing methods.

5	Use Cases

5.1	Use Case#1: Service-based management architecture and access requirements for MOCN

5.1.1	Description

In 3GPP TR 28.835 [5], there are some remaining issues that need to continue to study, including Issue #4: Service-based management architecture for MOCN and Issue #5: Use of MSAC for MOCN sharing scenarios.

The Service Based Management Architecture (SBMA) defined in 3GPP TS 28.533 [7] need to be provided for MOCN networking sharing. But the potential solution is an example of the service-based management architecture for MOCN, and the comprehensive service-based management architecture needs further study.

The management system of MOP should have the capability to allow authorized POP users to access, read and download data of configuration, performance metrics (including performance measurement and KPI) and alarms. Therefore, some potential solutions of users' access and relevant system user permissions for each POP need to be studied.

5.1.2	Potential requirements

REQ-NetShare_Config-1: The MnS producer of MOP should support MnS consumer to request to access the MOP's management system.

REQ-NetShare_Config-2: The MnS producer of MOP should allow or forbid MnS consumer to obtain configuration, performance and alarm data.

REQ-NetShare_Config-3: The MnS producer of MOP should support roles for each POP as MnS consumers.

5.1.3	Potential solutions

5.1.3.1	Potential solution #1: Enhance the existing service-based management architecture and management workflow for MOCN

This solution is proposed to enhance service-based management architecture for MOCN defined in 3GPP TS 32.130 [4].

Figure 5.1.3.1-1

5.1.4	Evaluation of potential solutions

Only one potential solution is identified, which is feasible.

5.2	Use Case#2: Trace job and collection requirements for POPs

5.2.1	Description

For 5G MOCN Networking Sharing, different POPs require different data for personalized operation and maintenance services, including collecting different trace and performance data requirements.

EXAMPLE:	The different trace data or performance data with collection time window, collection requirements, etc., should be reported for each POP.

The existing specifications do not provide some descriptions of trace job, collection flows and interface details for NG-RAN MOCN network sharing scenario. Therefore, some potential solutions of trace job and collection requirements for POPs need to be studied.

5.2.2	Potential requirements

REQ-NetShare_TraceConfig-1: The 3GPP management system of the MOP should have the capability to support operator-specific management data collection jobs.

5.2.3	Potential solutions

Following are the enhancements to support the requirements above based on existing ManagementDataCollection in 3GPP TS 28.622 [6].

Enhancement Aspect 1: Add attribute "jobid" to ManagementDataCollection to identify the operator-specific management data collection job.

Enhancement Aspect 2: Add a description of how to identify PLMN by using PLMNId as the value of jobid.

NOTE:	PLMNInfoList and PLMNId is defined in 3GPP TS 28.541 [8].

Enhancement Aspect 3: Add attributes related to role "TraceJobRef" and "PerfMetricJobRef" of ManagementDataCollection. The "TraceJobRef" and "PerfMetricJobRef" represents the DN of TraceJob IOC and PerfMetricJob IOC.

Following aspects needs to be added or enhanced in 3GPP TS 32.130 [4]:

-	Add the description of how to use ManagementDataCollection to support operator-specific management data collection job.

5.2.4	Evaluation of potential solutions

Only one potential solution is identified, which is feasible.

5.3	Use Case#3: S-RAN management of Indirect Network Sharing

5.3.1	Description

According to 3GPP TS 22.261 [2], Indirect Network Sharing is defined as follow:

"a type of NG-RAN Sharing in which the communication between the Shared NG-RAN and the Participating Operator's core network is routed through the Hosting NG-RAN Operator's core network".

Therefore, Indirect Network Sharing is proposed to share NG-RAN indirectly through core network. Regarding 5G MOCN, NG-RAN is shared directly by the connection to each POP's core network. According to the definition of S-RAN in 3GPP TS 32.130 [4], shared NG-RAN in the Indirect Network Sharing scenario might be identified as S-RAN. As a result, each POP in the Indirect Network Sharing scenario could have requirements to manage S-RAN. The hosting operator, as MOP in the Indirect Network Sharing scenario, should have capabilities to allow authorized consumers to manage S-RAN.

5.3.2	Potential requirements

REQ-NetShare_Req-1: The 3GPP management system of the MOP shall support a capability allowing its authorized consumer to manage S-RAN in the Indirect Network Sharing scenario.

5.3.3	Potential solutions

Following aspects needs to be added or enhanced in management workflows in 3GPP TS 32.130 [4]:

- Add descriptions to support for management of Indirect Network Sharing as follows:

REQ-NS_GEN-CON-2: The 3GPP management system of the MOP shall support a capability allowing its authorized consumer to manage S-RAN according to any of the following scenarios:

i)	Multi-Operator Core Network.

ii)	Gateway Core Network.

iii)	Indirect Network Sharing.

- Add the management architecture diagram for Indirect Network Sharing.

5.3.4	Evaluation of potential solutions

Only one potential solution is identified, which is feasible.5.4	Use Case#4: RAN configuration management support for Indirect Network Sharing

5.4.1	Description

According to 3GPP TS 23.501 [10], Indirect Network Sharing is supported as follow:

"For Indirect Network Sharing, the shared RAN is broadcasting multiple PLMN IDs, including the PLMN ID which represents the hosting operator and the PLMN IDs which represent participating operators.".

To achieve the management of RAN sharing for Indirect Network Sharing, multiple PLMN IDs with same Cell identity broadcast and multiple Cell identity broadcast are supposed to be supported configuring.

5.4.2	Potential requirements

REQ-NetShare-RAN-Cfg_Req-1: The 3GPP management system of the MOP should have the capability to configure PLMNId individually for each POP.

REQ-NetShare-RAN-Cfg_Req-2: The 3GPP management system of the MOP should have the capability to configure PLMN-IdentityInfo (including PLMNId, NR Cell Identity, TAC) individually for each POP.

5.4.3	Potential solutions

Following are the enhancements to support the requirements above based on existing OperatorDU and NROperatorCellDU in 3GPP TS 28.541 [8].

Enhancement Aspect:

Update OperatorDU and NROperatorCellDU definitions to describe  Indirect Network Sharing scenario.

Following aspects needs to be enhanced in 3GPP TS 32.130 [4]:

-	Add descriptions of potential requirements on RAN configurations for Indirect Network Sharing.

-	Add a new workflow that describes MOP uses OperatorDU and NROperatorCellDU to support Indirect Network Sharing.

5.4.4	Evaluation of potential solutions

Only one potential solution is identified, which is feasible.

5.5	Use Case#5: CN configuration management support for Indirect Network Sharing

5.5.1	Description

According to 3GPP TS 23.501 [10], Indirect Network Sharing is supported as follow:

"The communication between the shared RAN and the core network of the participating operator is routed through the core network of the hosting operator that connects to the shared RAN.".

To achieve the management of Indirect Network Sharing, first communication between the shared RAN and the core network of each POP is supposed to support configuring, which means that the MOP should support configuring communication routing to POPs and POP should support configuring being routed from the MOP. Second, AMF of the MOP should allow to configure PLMN ID for each POP, and each POP should allow to configure the inter-plmn discovery.

5.5.2	Potential requirements

REQ-NetShare-CN-Cfg_Req-1: The 3GPP management system of the MOP should have the capability to configure communication to route through the core network for each POP.

REQ-NetShare-CN-Cfg_Req-2: The 3GPP management system of each POP should have the capability to configure communication to route through the core network for the MOP.

REQ-NetShare-CN-Cfg_Req-3: The 3GPP management system of the MOP should have the capability to configure PLMNID of serving AMF individually for each POP.

5.5.3	Potential solutions

Following aspects needs to be added or enhanced in 3GPP TS 32.130 [4]:

-	Add descriptions of potential requirements on CN configurations for Indirect Network Sharing.

-	Add a new workflow as follows:

This clause describes the workflows for the management of the Core Networks in Indirect network sharing scenario.

In this workflow, the core network of MOP (identified by PLMN#1) has connection to the core network of POP(POP B identified by PLMN#2). The MnS consumer belong to their own operators.

The MnS of  MOP requests MnS producer to configure "PLMNInfoList" and "interPlmnFQDN" of AMFFunction. The MnS of POP requests MnS producer to configure "interPlmnFQDN" of AMFFunction.

5.5.4	Evaluation of potential solutions

Only one potential solution is identified, which is feasible.

5.6	Use Case#6: performance management support for Indirect Network Sharing

5.6.1	Description

According to 3GPP TS 22.261 [2], Indirect Network Sharing is defined as follow:

"a type of NG-RAN Sharing in which the communication between the Shared NG-RAN and the Participating Operator's core network is routed through the Hosting NG-RAN Operator's core network.".

According to 3GPP TS 23.501 [10], Indirect Network Sharing is supported as follow:

"The communication between the shared RAN and the core network of the participating operator is routed through the core network of the hosting operator that connects to the shared RAN.".

Therefore, performance measurement of Indirect Network Sharing includes RAN and CN parts and both parts require the performance measurement in the PLMN granularity.

5.6.2	Potential requirements

REQ-NetShare-RAN-Perf_Req-1: The 3GPP management system of the MOP should have the capability to collect and report some measurements from RAN in PLMN granularity for each POP.

REQ-NetShare-CN-Perf_Req-2: The 3GPP management system of the MOP should have the capability to collect and report some measurements from NFs of CN in PLMN granularity for each POP.

5.6.3	Potential solutions

Following are the enhancements to support the requirements above in 3GPP TS 28.552 [9].

Enhancement Aspect: add PLMN granularity support for the performance measurement of related NFs in the Indirect Network Sharing Scenario (e.g. AMF, SMF) of the core network, including:

-	Mean number of registered subscribers

-	Maximum number of registered subscribers.

-	Number of initial registration requests.

-	Number of PDU sessions (Mean).

-	Number of PDU sessions (Maximum).

-	Number of PDU session creation requests.

-	Number of successful PDU session creations.

-	Number of failed PDU session creations.

Following aspects needs to be added or enhanced in 3GPP TS 32.130 [4]:

-	Add descriptions of potential requirements performance management support for Indirect Network Sharing.

5.6.4	Evaluation of potential solutions

Only one potential solution is identified, which is feasible.

6	Conclusions and recommendations

6.1	Use Case #1: Service-based management architecture and access requirements for MOCN

This use case identifies that there are some remaining issues that need to continue to study in TR 28.835[5], including Issue #4: Service-based management architecture for MOCN and Issue #5: Use of MSAC for MOCN sharing scenarios. In order to address these issues, the potential solution enhances the existing service-based management architecture for MOCN. Detailed description about this solution is in clause 5.1.3.

It is recommended to make some enhancement on service-based management architecture for MOCN according to this solution in the future normative work.

6.2	Use Case #2: Trace job and collection requirements for POPs

This use case identifies that different POPs require different data for personalized operation and maintenance services, including collecting different trace and performance data requirements. In order to address this issue, the potential solution proposes some enhancements based on existing ManagementDataCollection to support operator-specific management data collection jobs. Detailed description about this solution is in clause 5.2.3.

6.3	Use Case #3: S-RAN management of Indirect Network Sharing

This use case identifies that the hosting operator, as MOP in the Indirect Network Sharing scenario, should have capabilities to allow authorized consumers to manage S-RAN.. In order to address this issue, the potential solution proposes some enhancements of existing descirpitions. Detailed description about this solution is in clause 5.3.3.

It is recommended to add descriptions to support for management of Indirect Network Sharing in TS 32.130[4]. According to this solution in the future normative work to satisfy the requirements for S-RAN management of Indirect Network Sharing.

6.4	Use Case #4: RAN configuration management support for Indirect Network Sharing

This use case identifies that to achieve the management of RAN sharing for Indirect Network Sharing, multiple PLMN IDs with same Cell identity broadcast and multiple Cell identity broadcast are supposed to be supported configuring. In order to address this issue, the potential solution proposes some enhancements based on existing existing OperatorDU and NROperatorCellDU. Detailed description about this solution is in clause 5.4.3.

It is recommended to update OperatorDU and NROperatorCellDU definitions in TS 28.541[8] and add descriptions in TS 32.130[4]. According to this solution in the future normative work is to satisfy the requirements for RAN configuration management support for Indirect Network Sharing.

6.5	Use Case #5: CN configuration management support for Indirect Network Sharing

This use case identifies that Core Network configuration enhancements for Indirect Network Sharing are supposed to be supported. In order to address this issue, the potential solution proposes some enhancements based on existing PLMNInfoList and interPlmnFQDN. Detailed description about this solution is in clause 5.5.3.

It is recommended to add descrptions and management workflows in TS 32.130[4] according to this solution in the future normative work to satisfy the requirements for CN configuration management support for Indirect Network Sharing.

6.6	Use Case #6: performance management support for Indirect Network Sharing

This use case identifies that to observe the performance of different operators, similar with other network sharing scenarios, the performance measurement in PLMN granularity is supposed to be supported. In order to address this issue, the potential solution proposes some enhancements based on existing performance measurement in related NFs. Detailed description about this solution is in clause 5.2.3.

It is recommended to make some enhancement on existing performance measurement in TS 28.552[9] according to this solution in the future normative work to satisfy the requirements for performance management support for Indirect Network Sharing.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-04 | SA5#154 |  |  |  |  | Initial skeleton | 0.0.0 |
| 2024-04 | SA5#154 | S5-242132 |  |  |  | pCR TR 28.878 Add scope | 0.1.0 |
| 2024-04 | SA5#154 | S5-242133 |  |  |  | pCR TR 28.878 Add structure | 0.1.0 |
| 2024-04 | SA5#154 | S5-242134 |  |  |  | pCR TR 28.878 Concepts and overview | 0.1.0 |
| 2024-04 | SA5#154 | S5-242135 |  |  |  | pCR TR 28.878 Add use case, requirements for Service-based management architecture and access requirements for MOCN | 0.1.0 |
| 2024-04 | SA5#154 | S5-242136 |  |  |  | pCR TR 28.878 Add use case, requirements for Trace job and collection requirements for POPs | 0.1.0 |
| 2024-05 | SA5#155 | S5-243327 |  |  |  | pCR TR 28.878 Update Use Case#1 Service-based management architecture and access requirements for MOCN | 0.2.0 |
| 2024-05 | SA5#155 | S5-243327 |  |  |  | pCR TR 28.878 Add concepts and background of Indirect Network Sharing | 0.2.0 |
| 2024-05 | SA5#155 | S5-243328 |  |  |  | pCR TR 28.878 Add new use case and requirements on S-RAN management of Indirect Network Sharing | 0.2.0 |
| 2004-08 | SA5#156 | S5-244854 |  |  |  | pCR TR 28.878 Add potential solution for Service-based management architecture and access requirements for MOCN | 0.3.0 |
| 2004-08 | SA5#156 | S5-244855 |  |  |  | pCR TR 28.878 Add potential solution for S-RAN management of Indirect Network Sharing | 0.3.0 |
| 2004-08 | SA5#156 | S5-244856 |  |  |  | pCR TR 28.878 Add potential solution for Trace job and collection requirements for POPs | 0.3.0 |
| 2004-08 | SA5#156 | S5-244857 |  |  |  | pCR TR 28.878 Add use case, requirements and solution on performance management support for Indirect Network Sharing | 0.3.0 |
| 2004-08 | SA5#156 | S5-244858 |  |  |  | pCR TR 28.878 Add use case, requirements and solution on CN configuration management support for Indirect Network Sharing | 0.3.0 |
| 2004-08 | SA5#156 | S5-244859 |  |  |  | pCR TR 28.878 Add use case, requirements and solution on RAN configuration management support for Indirect Network Sharing | 0.3.0 |
| 2024-09 | SA#105 | SP-241130 |  |  |  | Presented at SA#105 for Information | 1.0.0 |
| 2024-09 |  |  |  |  |  | After editHelp cleanup | 1.0.1 |
| 2024-10 | SA5#157 | S5‑245789 |  |  |  | pCR TR 28.878 Add evaluation, conclusion and recommendation for Use Case #1 | 1.1.0 |
| 2024-10 | SA5#157 | S5‑245790 |  |  |  | pCR TR 28.878 Add evaluation, conclusion and recommendation for Use Case #2 | 1.1.0 |
| 2024-10 | SA5#157 | S5‑245796 |  |  |  | pCR TR 28.878 Add evaluation, conclusion and recommendation for Use Case #6 | 1.1.0 |
| 2024-10 | SA5#157 | S5‑246108 |  |  |  | pCR TR 28.878 Add evaluation, conclusion and recommendation for Use Case #3 | 1.1.0 |
| 2024-10 | SA5#157 | S5‑246109 |  |  |  | pCR TR 28.878 Add evaluation, conclusion and recommendation for Use Case #4 | 1.1.0 |
| 2024-10 | SA5#157 | S5‑246110 |  |  |  | pCR TR 28.878 Add evaluation, conclusion and recommendation for Use Case #5 | 1.1.0 |
| 2024-10 | SA5#157 | S5‑246112 |  |  |  | pCR TR28.878 Editorial corrections | 1.1.0 |