3GPP TS 29.522 V19.1.0 (2024-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System; Network Exposure Function Northbound APIs;

Stage 3

(Release 19)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	37

1	Scope	38

2	References	38

3	Definitions and abbreviations	41

3.1	Definitions	41

3.2	Abbreviations	41

4	NEF Northbound Interface	42

4.1	Overview	42

4.2	Reference model	45

4.3	Functional elements	46

4.3.1	NEF	46

4.3.2	AF	46

4.4	Procedures over NEF Northbound Interface	46

4.4.1	Introduction	46

4.4.2	Procedures for Monitoring	46

4.4.3	Procedures for Device Triggering	56

4.4.4	Procedures for resource management of Background Data Transfer	56

4.4.5	Procedures for CP Parameters Provisioning	57

4.4.6	Procedures for PFD Management	58

4.4.7	Procedures for Traffic Influence	59

4.4.7.1	General	59

4.4.7.2	AF request identified by UE address	59

4.4.7.3	AF request not identified by UE address	60

4.4.7.4	Handling of Event Notifications	61

4.4.7.4.1	Handling of UP path management event notification	61

4.4.7.4.2	Handling of Traffic routing requirements enforcement outcome event notification	61

4.4.7.5	Processing AF requests to influence traffic routing for HR-SBO session	62

4.4.7.6	Handling of payload headers event notification	63

4.4.8	Procedures for changing the chargeable party at session set up or during the session	63

4.4.9	Procedures for AF required QoS	64

4.4.9.1	General	64

4.4.9.2	Procedures for AF setting up an AF session with required QoS for target UE identified by UE address or for target list of UEs identified by list of UE addresses	64

4.4.9.2.0	General	64

4.4.9.2.1	Procedure for QoS handling and QoS monitoring for list of UEs	65

4.4.9.2.2	Subscriptions to QoS handling and QoS Monitoring	66

4.4.9.2.3	Provisioning of TSCAI input Information and QoS related data	70

4.4.9.2.4	Subscription to the access type change and plmn change	71

4.4.9.2.5	Provisioning of ToS traffic class	72

4.4.9.2.6	Provisioning of multi-modal services	72

4.4.9.2.7	Provisioning of Round-Trip latency requirements	73

4.4.9.2.8	Subscription to the report of L4S support information	74

4.4.9.2.9	Provisioning of PDU Set handling	74

4.4.9.2.10	Provisioning of traffic information for UE power saving management	74

4.4.9.2.11	Provisioning of the QoS timing information	74

4.4.9.2.12	Provisioning of Dynamic ChangingTraffic Characteristics	75

4.4.9.2.13	Provisioning of the Multiplexed Media Information	75

4.4.9.3	Procedures for AF requested QoS for a target UE or group of UE(s) not identified by UE address(es)	75

4.4.10	Procedures for MSISDN-less Mobile Originated SMS	75

4.4.11	Procedures for Network Configuration Parameters Provisioning	76

4.4.12	Procedures for Non-IP data delivery	77

4.4.12.1	General	77

4.4.12.2	NIDD configuration Triggered by the NEF	77

4.4.12.3	NIDD configuration triggered by the AF and NIDD delivery	77

4.4.13	Procedures for RACS Parameter Provisioning	78

4.4.14	Procedures for analytics information exposure	78

4.4.14.1	Subscription/unsubscription to notification of analytics information	78

4.4.14.2	Fetch analytics information	80

4.4.15	Procedures for 5G LAN Parameter Provisioning	81

4.4.15.1	General	81

4.4.15.2	Creation of a new subscription for 5G LAN parameter provisioning	81

4.4.15.3	Modification of an existing subscription for 5G LAN parameter provisioning	81

4.4.15.4	Deletion of an existing subscription for 5G LAN parameter provisioning	82

4.4.15.5	5G LAN parameter provisioning event notification	82

4.4.16	Procedures for applying BDT policy	82

4.4.17	Procedures for Enhanced Coverage Restriction Control	83

4.4.18	Procedures for IPTV Configuration	83

4.4.20	Procedures for service specific parameter provisioning	84

4.4.21	Procedures for ACS configuration parameter provisioning	88

4.4.22	Procedures for Mobile Originated Location Request	88

4.4.22.1	General	88

4.4.22.2	Location Update Notification triggered by UE	89

4.4.23	Procedures for AKMA	89

4.4.23.1	General	89

4.4.23.2	AKMA Application Key Request	89

4.4.23.3	AKMA Service Disablement Notification	89

4.4.24	Procedures for Time Synchronization Exposure	90

4.4.24.0	General	90

4.4.24.1	Subscription and unsubscription to notification of Time Synchronization Capabilites	90

4.4.24.2	Time Synchronization Exposure Configuration	91

4.4.24.3	Management of 5G access stratum time distribution	92

4.4.25	Procedures for ECS address Provisioning	94

4.4.26	Procedures for AM Policy Authorization	94

4.4.26.1	General	94

4.4.26.2	Creation of a new Individual Application AM Context	95

4.4.26.3	Modification of an existing individual Application AM Context	95

4.4.26.4	Deletion of an existing individual Application AM Context	96

4.4.26.5	Create or modify subscription to notification of AM policy event	96

4.4.26.6	Unsubscription to notification of AM policy event	96

4.4.26.7	Notification of AM policy event	97

4.4.27	Procedures for AF triggered Access and Mobility Influence	97

4.4.27.1	General	97

4.4.27.2	Create the AM Influence Subscription	97

4.4.27.3	Modifiy the AM Influence Subscription	97

4.4.27.4	Delete the AM Influence Subscription	98

4.4.27.5	Notification of service area coverage outcome events	98

4.4.28	Procedures for Northbound EAS Deployment Information management	98

4.4.28.1	General	98

4.4.28.2	Creation of a new Individual EAS Deployment information resource	98

4.4.28.3	Modification of an existing individual EAS Deployment Information resource	99

4.4.28.4	Deletion of an existing individual EAS Deployment Information resource	99

4.4.28.5	Deletion of EAS Deplyoment Information based on given criteria	100

4.4.29	Procedures for MBS Management	100

4.4.29.1	General	100

4.4.29.2	Procedures for MBS TMGI management	100

4.4.29.2.1	General	100

4.4.29.2.2	Procedure for MBS TMGI(s) allocation or MBS TMGI(s) expiry time refresh	100

4.4.29.2.3	Procedure for MBS TMGI(s) deallocation	101

4.4.29.2.4	Procedure for MBS TMGI(s) timer expiry notification	102

4.4.29.3	Procedures for MBS session management	102

4.4.29.3.1	General	102

4.4.29.3.2	Procedure for MBS session creation	102

4.4.29.3.3	Procedure for MBS session update	105

4.4.29.3.4	Procedure for MBS session deletion	106

4.4.29.3.5	Procedure for MBS session status subscription	106

4.4.29.3.6	Procedure for MBS session status unsubscription	106

4.4.29.3.7	Procedure for MBS session status notification	106

4.4.29.4	Procedures for MBS Parameters Provisioning	107

4.4.29.4.1	General	107

4.4.29.4.2	Procedure for multicast MBS Session Authorization information provisioning	107

4.4.29.4.3	Procedure for multicast MBS Session Assistance information provisioning	109

4.4.29.5	Procedures for MBS User Service management	109

4.4.29.5.1	General	109

4.4.29.5.2	Procedure for MBS User Service creation	109

4.4.29.5.3	Procedure for MBS User Service retrieval	110

4.4.29.5.4	Procedure for MBS User Service update/modification	110

4.4.29.5.5	Procedure for MBS User Service deletion	110

4.4.29.6	Procedures for MBS User Data Ingest Session management	111

4.4.29.6.1	General	111

4.4.29.6.2	Procedure for MBS User Data Ingest Session creation	111

4.4.29.6.3	Procedure for MBS User Data Ingest Session retrieval	111

4.4.29.6.4	Procedure for MBS User Data Ingest Session update/modification	112

4.4.29.6.5	Procedure for MBS User Data Ingest Session deletion	112

4.4.29.6.6	Procedure for MBS User Data Ingest Session Status Subscription	113

4.4.29.6.7	Procedure for MBS User Data Ingest Session Status update/modification	113

4.4.29.6.8	Procedure for MBS User Data Ingest Session Status Unsubscription	114

4.4.29.6.9	Procedure for MBS User Data Ingest Session Status Notification	114

4.4.29.7	Procedures for MBS Group Message Delivery Management	114

4.4.29.7.1	General	114

4.4.29.7.2	Procedure for MBS Group Message Delivery Creation	114

4.4.29.7.3	Procedure for MBS Group Message Delivery Update	115

4.4.29.7.4	Procedure for MBS Group Message Delivery Deletion	115

4.4.29.7.5	Procedure for MBS Group Message Delivery Status Notification	116

4.4.30	Procedures for Data Reporting	116

4.4.30.1	General	116

4.4.30.2	Procedure for Data Reporting Session Management	116

4.4.30.3	Procedure for Data Report	117

4.4.31	Procedures for Data Reporting Provisioning	117

4.4.31.1	General	117

4.4.31.2	Procedure for Data Reporting Provisioning Session Management	117

4.4.31.3	Procedure for Data Reporting Configuration management	118

4.4.32	Procedures for UE ID retrieval	118

4.4.32.1	General	118

4.4.32.2	Retrieve AF specific UE ID service operation	119

4.4.32.3	Retrieve UE ID in the form of MSISDN	120

4.4.32.4	UE ID Mapping Information Provisioning	121

4.4.33	Procedures for Media Streaming Event Exposure	122

4.4.33.1	General	122

4.4.33.2	Procedure for Media Streaming Event Exposure Subscription Creation	122

4.4.33.3	Procedure for Media Streaming Event Exposure Subscription Update	122

4.4.33.4	Procedure for Media Streaming Event Exposure Unsubscription	123

4.4.33.5	Procedure for Media Streaming Event Exposure Notification	123

4.4.34	Procedures for DNAI Mapping	124

4.4.34.1	General	124

4.4.34.2	Creation of a new subscription for DNAI Mapping	124

4.4.34.3	Deletion of an existing individual DNAI Mapping subscription	124

4.4.34.4	Notification for updated DNAI Mapping information	124

4.4.35	Procedures for negotiation of planned data transfer with QoS requirements	125

4.4.36	Procedures for Member UE Selection Assistance	126

4.4.36.1	General	126

4.4.36.2	Procedure for Member UE Selection Assistance Subscription Creation	126

4.4.36.3	Procedure for Member UE Selection Assistance Subscription Update	126

4.4.36.4	Procedure for Member UE Selection Assistance Subscription Unsubscription	126

4.4.36.5	Procedure for Member UE Selection Assistance Notification	127

4.4.37	Procedures for Group Parameters Provisioning	127

4.4.37.1	General	127

4.4.37.2	Procedures for DNN and S-NSSAI specific Group Parameters Provisioning	127

4.4.38	Procedures for Network Slice Parameters Provisioning	128

4.4.38.1	General	128

4.4.38.2	Procedures for Network Slice Usage Control Parameters Provisioning	129

4.4.39	Procedures for UE Address Retrieval	130

4.4.39.1	General	130

4.4.39.2	Procedures for UE Address Retrieval	130

4.4.39.3	Creation of a new subscription to obtain UE Address	130

4.4.39.4	Deletion of an existing individual UE Address subscription	131

4.4.39.5	Notification for UE Address information	131

4.4.40	Procedures for ECS Address Configuration Information provisioning in roaming	131

4.4.40.1	General	131

4.4.40.2	Creation of new ECS Address Configuration Information Set	131

4.4.40.3	Modification of existing ECS Address Configuration Information Set	132

4.4.40.4	Deletion of existing Individual ECS Address Configuration Information Set	132

4.4.40.5	Deletion of ECS Address Configuration Information based on given criteria	133

4.4.41.1	General	133

4.4.41.2	Procedures for RSLPPI Parameters Provisioning	133

4.4.42.1	General	134

4.4.42.2	Procedures for Static IP Address Parameters Provisioning	134

4.4.43.1	General	135

4.4.43.2	Creation of a new UAV Flight Assistance Configuration	135

4.4.43.3	Update of an existing individual UAV Flight Assistance Configuration	136

4.4.43.4	Deletion of an existing individual UAV Flight Assistance Configuration	136

4.4.43.5	Notification for UAV Flight Assistance information	136

4.4.44	Procedures for CAG Information Parameters Provisioning	136

4.4.44.1	General	136

4.4.45	Procedures for UAV Flight information Retrieval	137

4.4.45.1	General	137

4.4.45.2	Procedures for UAV Flight information Retrieval	138

4.4.46	Procedures for IMS Session Management	138

4.4.46.1	General	138

4.4.46.2	Creation a new IMS session	138

4.4.46.3	Modification of an existing IMS session	138

4.4.46.3	Release an existing IMS session	139

4.4.47	Procedures for IMS Event Exposure Services	139

4.4.47.1	General	139

4.4.47.2	Creation of IMS Event Exposure Service Subscription	139

4.4.47.3	Procedure for IMS Event Exposure Service Subscription Update	139

4.4.47.4	Unsubscription of IMS Event Exposure Service	140

4.4.47.5	Notification of IMS Event(s)	140

5	NEF Northbound APIs	140

5.1	Introduction	140

5.2	Information applicable to several APIs	143

5.3	Reused APIs	143

5.4	TrafficInfluence API	144

5.4.0	Introduction	144

5.4.1	Resources	145

5.4.1.1	Overview	145

5.4.1.2	Resource: Traffic Influence Subscription	145

5.4.1.2.1	Introduction	145

5.4.1.2.2	Resource Definition	145

5.4.1.2.3	Resource Methods	146

5.4.1.2.3.1	General	146

5.4.1.2.3.2	GET	146

5.4.1.2.3.3	POST	147

5.4.1.3	Resource: Individual Traffic Influence Subscription	147

5.4.1.3.1	Introduction	147

5.4.1.3.2	Resource Definition	147

5.4.1.3.3	Resource Methods	148

5.4.1.3.3.1	General	148

5.4.1.3.3.2	GET	148

5.4.1.3.3.3	PUT	149

5.4.1.3.3.4	PATCH	149

5.4.1.3.3.5	DELETE	150

5.4.1A	Custom Operations without associated resources	151

5.4.2	Notifications	151

5.4.2.1	Introduction	151

5.4.2.2	Event Notification	152

5.4.2.2.1	Description	152

5.4.2.2.2	Target URI	152

5.4.2.2.3	Operation Definition	152

5.4.2.3	Acknowledgement of event notification	153

5.4.2.3.1	Description	153

5.4.2.3.2	Target URI	153

5.4.2.3.3	Operation Definition	154

5.4.2.3.3.1	Notification via HTTP POST	154

5.4.2.4	Event Notification of Payload Headers	154

5.4.2.4.1	Description	154

5.4.2.4.2	Target URI	154

5.4.2.4.3	Operation Definition	155

5.4.3	Data Model	156

5.4.3.1	General	156

5.4.3.2	Reused data types	156

5.4.3.3	Structured data types	157

5.4.3.3.1	Introduction	157

5.4.3.3.2	Type: TrafficInfluSub	157

5.4.3.3.3	Type: TrafficInfluSubPatch	164

5.4.3.3.4	Type: EventNotification	169

5.4.3.3.5	Type: AfResultInfo	172

5.4.3.3.6	Type AfAckInfo	172

5.4.3.3.7	Type TrafficDataSet	172

5.4.3.3.8	Type TrafficDataSetRm	173

5.4.3.3.9	Type: HeaderHandlingReport	173

5.4.3.4	Simple data types and enumerations	173

5.4.3.4.1	Introduction	173

5.4.3.4.2	Simple data types	173

5.4.3.4.3	Enumeration: SubscribedEvent	173

5.4.3.4.4	Enumeration: AfResultStatus	174

5.4.4	Used Features	174

5.4.5	Error handling	175

5.4.5.1	General	175

5.4.5.2	Protocol Errors	176

5.4.5.3	Application Errors	176

5.5	NiddConfigurationTrigger API	176

5.5.0	Introduction	176

5.5.1	Resources	176

5.5.1A	Custom Operations without associated resources	176

5.5.2	Notifications	176

5.5.2.1	Introduction	176

5.5.2.2	Event Notification	177

5.5.2.3	Operation Definition	177

5.5.2.3.1	Notification via HTTP POST	177

5.5.2.3.2	Notification via Websocket	178

5.5.3	Data Model	178

5.5.3.1	General	178

5.5.3.2	Reused data types	178

5.5.3.3	Structured data types	178

5.5.3.3.1	Introduction	178

5.5.3.3.2	Type: NiddConfigurationTrigger	178

5.5.3.3.3	Type: NiddConfigurationTriggerReply	178

5.5.3.4	Simple data types and enumerations	179

5.5.3.4.1	Introduction	179

5.5.3.4.2	Simple data types	179

5.5.4	Used Features	179

5.5.5	Error handling	179

5.5.5.1	General	179

5.5.5.2	Protocol Errors	179

5.5.5.3	Application Errors	179

5.6	AnalyticsExposure API	180

5.6.0	Introduction	180

5.6.1	Resources	180

5.6.1.1	Overview	180

5.6.1.2	Resource: Analytics Exposure Subscriptions	181

5.6.1.2.1	Introduction	181

5.6.1.2.2	Resource Definition	181

5.6.1.2.3	Resource Methods	181

5.6.1.2.3.1	General	181

5.6.1.2.3.2	GET	181

5.6.1.2.3.3	POST	182

5.6.1.3	Resource: Individual Analytics Exposure Subscription	183

5.6.1.3.1	Introduction	183

5.6.1.3.2	Resource Definition	183

5.6.1.3.3	Resource Methods	183

5.6.1.3.3.1	General	183

5.6.1.3.3.2	GET	183

5.6.1.3.3.3	PUT	184

5.6.1.3.3.4	DELETE	185

5.6.1A	Custom Operations without associated resources	186

5.6.1A.1	Overview	186

5.6.1A.2	Operation: fetch	187

5.6.1A.2.1	Description	187

5.6.1A.2.2	Operation Definition	187

5.6.2	Notifications	188

5.6.2.1	Introduction	188

5.6.2.2	Event Notification	188

5.6.2.3	Operation Definition	188

5.6.2.3.1	Notification via HTTP POST	188

5.6.2.3.2	Notification via Websocket	189

5.6.3	Data Model	189

5.6.3.1	General	189

5.6.3.2	Reused data types	190

5.6.3.3	Structured data types	194

5.6.3.3.1	Introduction	194

5.6.3.3.2	Type: AnalyticsExposureSubsc	195

5.6.3.3.3	Type: AnalyticsEventNotification	196

5.6.3.3.4	Type: AnalyticsEventNotif	197

5.6.3.3.5	Type: AnalyticsEventSubsc	201

5.6.3.3.6	Type: AnalyticsEventFilterSubsc	202

5.6.3.3.7	Type TargetUeId	208

5.6.3.3.8	Void	209

5.6.3.3.9	Type UeMobilityExposure	209

5.6.3.3.10	Type UeLocationInfo	210

5.6.3.3.11	Void	210

5.6.3.3.12	Type: AnalyticsRequest	210

5.6.3.3.13	Type AnalyticsEventFilter	211

5.6.3.3.14	Type AnalyticsData	216

5.6.3.3.15	Type AbnormalExposure	219

5.6.3.3.16	Type CongestInfo	220

5.6.3.3.17	Type CongestionAnalytics	220

5.6.3.3.18	Type QosSustainabilityExposure	221

5.6.3.3.19	Type NetworkPerfExposure	222

5.6.3.3.20	Type AnalyticsFailureEventInfo	222

5.6.3.3.21	Type WlanPerformInfo	223

5.6.3.4	Simple data types and enumerations	223

5.6.3.4.1	Introduction	223

5.6.3.4.2	Simple data types	223

5.6.3.4.3	Enumeration: AnalyticsEvent	223

5.6.3.4.4	Enumeration: AnalyticsFailureCode	224

5.6.4	Used Features	225

5.6.5	Error handling	229

5.6.5.1	General	229

5.6.5.2	Protocol Errors	229

5.6.5.3	Application Errors	229

5.7	5GLANParameterProvision API	229

5.7.0	Introduction	229

5.7.1	Resources	230

5.7.1.1	Overview	230

5.7.1.2	Resource: 5GLAN Parameters Provision Subscriptions	230

5.7.1.2.1	Introduction	230

5.7.1.2.2	Resource Definition	230

5.7.1.2.3	Resource Methods	231

5.7.1.2.3.1	General	231

5.7.1.2.3.2	GET	231

5.7.1.2.3.3	POST	232

5.7.1.3	Resource: Individual 5GLAN Parameters Provision Subscription	232

5.7.1.3.1	Introduction	232

5.7.1.3.2	Resource Definition	232

5.7.1.3.3	Resource Methods	233

5.7.1.3.3.1	General	233

5.7.1.3.3.2	GET	233

5.7.1.3.3.3	PUT	234

5.7.1.3.3.4	DELETE	234

5.7.1.3.3.5	PATCH	235

5.7.1A	Custom Operations without associated resources	236

5.7.1B	Notifications	236

5.7.1B.1	General	236

5.7.1B.2	5G LAN Parameter Provisioning Event Notification	237

5.7.1B.2.1	Description	237

5.7.1B.2.2	Target URI	237

5.7.1B.2.3	Operation Definition	237

5.7.1B.2.3.1	Notification via HTTP POST	237

5.7.1B.2.3.2	Void	238

5.7.2	Data Model	238

5.7.2.1	General	238

5.7.2.2	Reused data types	238

5.7.2.3	Structured data types	239

5.7.2.3.1	Introduction	239

5.7.2.3.2	Type: 5GLanParametersProvision	240

5.7.2.3.3	Type: 5GLanParameters	240

5.7.2.3.4	Type: AppDescriptor	243

5.7.2.3.5	Type: 5GLanParametersProvisionPatch	244

5.7.2.3.6	Type: 5GLanParametersPatch	244

5.7.2.3.7	Type: AppDescriptorRm	245

5.7.2.3.8	Void	245

5.7.2.3.9	Type: CpParams	245

5.7.2.3.10	Type: NpConfigParams	246

5.7.2.3.11	Type: LpiParams	246

5.7.2.3.12	Type: AcsParams	246

5.7.2.3.13	Type: ECSAddrParams	246

5.7.2.3.14	Type: DnnSnssaiParams	247

5.7.2.3.15	Type: 5GLanParamProvNotif	247

5.7.2.3.16	Type: NpConfigNotif	247

5.7.2.3.17	Type: MaxGrpDataRateInfo	247

5.7.2.4	Simple data types and enumerations	248

5.7.2.4.1	Introduction	248

5.7.2.4.2	Simple data types	248

5.7.2.4.3	Enumeration: AaaUsage	248

5.7.3	Used Features	248

5.7.4	Error handling	249

5.7.4.1	General	249

5.7.4.2	Protocol Errors	249

5.7.4.3	Application Errors	249

5.8	ApplyingBdtPolicy API	249

5.8.0	Introduction	249

5.8.1	Resources	249

5.8.1.1	Overview	249

5.8.1.2	Resource: Applied BDT Policy Subscriptions	250

5.8.1.2.1	Introduction	250

5.8.1.2.2	Resource Definition	250

5.8.1.2.3	Resource Methods	251

5.8.1.2.3.1	General	251

5.8.1.2.3.2	GET	251

5.8.1.2.3.3	POST	252

5.8.1.3	Resource: Individual Applied BDT Policy Subscription	252

5.8.1.3.1	Introduction	252

5.8.1.3.2	Resource Definition	252

5.8.1.3.3	Resource Methods	252

5.8.1.3.3.1	General	252

5.8.1.3.3.2	GET	253

5.8.1.3.3.3	PATCH	253

5.8.1.3.3.4	DELETE	254

5.8.1A	Custom Operations without associated resources	255

5.8.2	Notifications	255

5.8.3	Data Model	255

5.8.3.1	General	255

5.8.3.2	Reused data types	256

5.8.3.3	Structured data types	256

5.8.3.3.1	Introduction	256

5.8.3.3.2	Type: AppliedBdtPolicy	256

5.8.3.3.3	Type: AppliedBdtPolicyPatch	256

5.8.3.4	Simple data types and enumerations	257

5.8.3.4.1	Introduction	257

5.8.3.4.2	Simple data types	257

5.8.4	Used Features	257

5.8.5	Error handling	257

5.8.5.1	General	257

5.8.5.2	Protocol Errors	257

5.8.5.3	Application Errors	257

5.9	IPTVConfiguration API	258

5.9.0	Introduction	258

5.9.1	Resources	258

5.9.1.1	Overview	258

5.9.1.2	Resource: IPTV Configurations	259

5.9.1.2.1	Introduction	259

5.9.1.2.2	Resource Definition	259

5.9.1.2.3	Resource Methods	259

5.9.1.2.3.1	General	259

5.9.1.2.3.2	GET	259

5.9.1.2.3.3	POST	260

5.9.1.3	Resource: Individual IPTV Configuration	261

5.9.1.3.1	Introduction	261

5.9.1.3.2	Resource Definition	261

5.9.1.3.3	Resource Methods	261

5.9.1.3.3.1	General	261

5.9.1.3.3.2	GET	261

5.9.1.3.3.3	PUT	262

5.9.1.3.3.4	DELETE	263

5.9.1.3.3.5	PATCH	264

5.9.1A	Custom Operations without associated resources	265

5.9.1B	Notifications	265

5.9.2	Data Model	265

5.9.2.1	General	265

5.9.2.2	Reused data types	265

5.9.2.3	Structured data types	265

5.9.2.3.1	Introduction	265

5.9.2.3.2	Type: IptvConfigData	266

5.9.2.3.3	Type: MulticastAccessControl	266

5.9.2.3.4	Type: IptvConfigDataPatch	267

5.9.2.4	Simple data types and enumerations	267

5.9.2.4.1	Introduction	267

5.9.2.4.2	Simple data types	267

5.9.2.4.3	Enumeration: AccessRightStatus	267

5.9.3	Used Features	267

5.9.4	Error handling	267

5.9.4.1	General	267

5.9.4.2	Protocol Errors	268

5.9.4.3	Application Errors	268

5.10	LpiParameterProvision API	268

5.10.0	Introduction	268

5.10.1	Resources	268

5.10.1.1	Overview	268

5.10.1.2	Resource: LPI Parameters Provisionings	269

5.10.1.2.1	Introduction	269

5.10.1.2.2	Resource Definition	269

5.10.1.2.3	Resource Methods	269

5.10.1.2.3.1	General	269

5.10.1.2.3.2	GET	269

5.10.1.2.3.3	POST	270

5.10.1.3	Resource: Individual LPI Parameters Provisioning	271

5.10.1.3.1	Introduction	271

5.10.1.3.2	Resource Definition	271

5.10.1.3.3	Resource Methods	271

5.10.1.3.3.1	General	271

5.10.1.3.3.2	GET	271

5.10.1.3.3.3	PUT	272

5.10.1.3.3.3A	PATCH	273

5.10.1.3.3.4	DELETE	274

5.10.1A	Custom Operations without associated resources	275

5.10.1B	Notifications	275

5.10.2	Data Model	275

5.10.2.1	General	275

5.10.2.2	Reused data types	275

5.10.2.3	Structured data types	276

5.10.2.3.1	Introduction	276

5.10.2.3.2	Type: LpiParametersProvision	276

5.10.2.3.3	Type: LpiParametersProvisionPatch	277

5.10.2.4	Simple data types and enumerations	277

5.10.2.4.1	Introduction	277

5.10.2.4.2	Simple data types	277

5.10.3	Used Features	277

5.10.4	Error handling	277

5.10.4.1	General	277

5.10.4.2	Protocol Errors	277

5.10.4.3	Application Errors	278

5.11	ServiceParameter API	278

5.11.0	Introduction	278

5.11.1	Resources	278

5.11.1.1	Overview	278

5.11.1.2	Resource: Service Parameter Subscriptions	279

5.11.1.2.1	Introduction	279

5.11.1.2.2	Resource Definition	279

5.11.1.2.3	Resource Methods	279

5.11.1.2.3.1	General	279

5.11.1.2.3.2	GET	279

5.11.1.2.3.3	POST	281

5.11.1.3	Resource: Individual Service Parameter Subscription	281

5.11.1.3.1	Introduction	281

5.11.1.3.2	Resource Definition	281

5.11.1.3.3	Resource Methods	281

5.11.1.3.3.1	General	281

5.11.1.3.3.2	GET	282

5.11.1.3.3.3	PUT	282

5.11.1.3.3.4	DELETE	283

5.11.1.3.3.5	PATCH	284

5.11.1A	Notifications	285

5.11.1A.1	Introduction	285

5.11.1A.2	AF Notifications	285

5.11.1A.2.1	Description	285

5.11.1A.2.2	Target URI	285

5.11.1A.3	Operation Definition	286

5.11.1A.3.1	Notification via HTTP POST	286

5.11.1A.3.2	Notification via Websocket	287

5.11.1B	Custom Operations without associated resources	287

5.11.2	Data Model	287

5.11.2.1	General	287

5.11.2.2	Reused data types	291

5.11.2.3	Structured data types	291

5.11.2.3.1	Introduction	291

5.11.2.3.2	Type: ServiceParameterData	292

5.11.2.3.3	Type: ServiceParameterDataPatch	296

5.11.2.3.4	Type: UrspRuleRequest	298

5.11.2.3.5	Type: RouteSelectionParameterSet	299

5.11.2.3.6	Type: AfNotification	299

5.11.2.3.7	Type: EventInfo	300

5.11.2.3.8	Type: TrafficDescriptorComponents	301

5.11.2.3.9	Type: NetworkDescription	303

5.11.2.3.10	Void	303

5.11.2.3.11	Void	303

5.11.2.3.12	Type: Non3gppDeviceInformation	303

5.11.2.4	Simple data types and enumerations	303

5.11.2.4.1	Introduction	303

5.11.2.4.2	Simple data types	304

5.11.2.4.3	Enumeration: Event	307

5.11.2.4.4	Enumeration: AuthorizationResult	308

5.11.2.4.5	Enumeration: Failure	308

5.11.2.4.6	Enumeration: ConnectionCapabilities	308

5.11.3	Used Features	309

5.11.4	Error handling	311

5.11.4.1	General	311

5.11.4.2	Protocol Errors	311

5.11.4.3	Application Errors	311

5.12	ACSParameterProvision API	311

5.12.0	Introduction	311

5.12.1	Resources	311

5.12.1.1	Overview	311

5.12.1.2	Resource: ACS Configuration Subscriptions	312

5.12.1.2.1	Introduction	312

5.12.1.2.2	Resource Definition	312

5.12.1.2.3	Resource Methods	313

5.12.1.2.3.1	General	313

5.12.1.2.3.2	GET	313

5.12.1.2.3.3	POST	314

5.12.1.3	Resource: Individual ACS Configuration Subscription	314

5.12.1.3.1	Introduction	314

5.12.1.3.2	Resource Definition	314

5.12.1.3.3	Resource Methods	314

5.12.1.3.3.1	General	314

5.12.1.3.3.2	GET	315

5.12.1.3.3.3	PUT	315

5.12.1.3.3.3A	PATCH	316

5.12.1.3.3.4	DELETE	317

5.12.1A	Custom Operations without associated resources	318

5.12.1B	Notifications	318

5.12.2	Data Model	318

5.12.2.1	General	318

5.12.2.2	Reused data types	319

5.12.2.3	Structured data types	319

5.12.2.3.1	Introduction	319

5.12.2.3.2	Type: AcsConfigurationData	319

5.12.2.3.3	Type: AcsConfigurationDataPatch	320

5.12.2.4	Simple data types and enumerations	320

5.12.2.4.1	Introduction	320

5.12.2.4.2	Simple data types	320

5.12.3	Used Features	320

5.12.4	Error handling	320

5.12.4.1	General	320

5.12.4.2	Protocol Errors	320

5.12.4.3	Application Errors	320

5.13	MoLcsNotify API	321

5.13.0	Introduction	321

5.13.1	Resources	321

5.13.1A	Custom Operations without associated resources	321

5.13.2	Notifications	321

5.13.2.1	Introduction	321

5.13.2.2	Event Notification	321

5.13.2.3	Operation Definition	322

5.13.2.3.1	Notification via HTTP POST	322

5.13.3	Data Model	322

5.13.3.1	General	322

5.13.3.2	Reused data types	322

5.13.3.3	Structured data types	323

5.13.3.3.1	Introduction	323

5.13.3.3.2	Type: LocUpdateData	323

5.13.3.3.3	Type: LocUpdateDataReply	323

5.13.3.4	Simple data types and enumerations	323

5.13.3.4.1	Introduction	323

5.13.3.4.2	Simple data types	324

5.13.4	Used Features	324

5.13.5	Error handling	324

5.13.5.1	General	324

5.13.5.2	Protocol Errors	324

5.13.5.3	Application Errors	324

5.14	AKMA API	324

5.14.1	Introduction	324

5.14.2	Resources	325

5.14.3	Custom Operations without associated resources	325

5.14.3.1	Overview	325

5.14.3.2	Operation: Retrieve	325

5.14.3.2.1	Description	325

5.14.3.2.2	Operation Definition	325

5.14.4	Notifications	326

5.14.4.1	General	326

5.14.4.2	AKMA Service Disablement Notification	326

5.14.4.2.1	Description	326

5.14.4.2.2	Target URI	327

5.14.4.2.3	Operation Definition	327

5.14.5	Data Model	328

5.14.5.1	General	328

5.14.5.2	Reused data types	328

5.14.5.3	Structured data types	328

5.14.5.3.1	Introduction	328

5.14.5.3.2	Type: AkmaAfKeyRequest	329

5.14.5.3.3	Type: AkmaAfKeyData	329

5.14.5.3.4	Type: ServiceDisableNotif	329

5.14.5.4	Simple data types and enumerations	330

5.14.5.4.1	Introduction	330

5.14.5.4.2	Simple data types	330

5.14.6	Used Features	330

5.14.7	Error handling	330

5.14.7.1	General	330

5.14.7.2	Protocol Errors	331

5.14.7.3	Application Errors	331

5.15	TimeSyncExposure API	331

5.15.0	Introduction	331

5.15.1	Resources	331

5.15.1.1	Overview	331

5.15.1.2	Resource: Time Synchronization Exposure Subscriptions	332

5.15.1.2.1	Introduction	332

5.15.1.2.2	Resource Definition	333

5.15.1.2.3	Resource Methods	333

5.15.1.2.3.1	General	333

5.15.1.2.3.2	GET	333

5.15.1.2.3.3	POST	334

5.15.1.3	Resource: Individual Time Synchronization Exposure Subscription	335

5.15.1.3.1	Introduction	335

5.15.1.3.2	Resource Definition	335

5.15.1.3.3	Resource Methods	335

5.15.1.3.3.1	General	335

5.15.1.3.3.2	GET	335

5.15.1.3.3.3	PUT	336

5.15.1.3.3.4	DELETE	337

5.15.1.4	Resource: Time Synchronization Exposure Configurations	338

5.15.1.4.1	Introduction	338

5.15.1.4.2	Resource Definition	338

5.15.1.4.3	Resource Methods	339

5.15.1.4.3.1	General	339

5.15.1.4.3.2	GET	339

5.15.1.4.3.3	POST	340

5.15.1.5	Resource: Individual Time Synchronization Exposure Configuration	341

5.15.1.5.1	Introduction	341

5.15.1.5.2	Resource Definition	341

5.15.1.5.3	Resource Methods	341

5.15.1.5.3.1	General	341

5.15.1.5.3.2	GET	341

5.15.1.5.3.3	PUT	342

5.15.1.5.3.4	DELETE	343

5.15.2	Custom Operations without associated resources	344

5.15.3	Notifications	344

5.15.3.1	Introduction	344

5.15.3.2	Time Synchronization Capability Notification	345

5.15.3.2.1	Description	345

5.15.3.2.2	Callback URI	345

5.15.3.2.3	Operation Definition	345

5.15.3.2.3.1	Notification via HTTP POST	345

5.15.3.2.3.2	Notification via Websocket	346

5.15.3.3	Time Synchronization Configuration Notification	346

5.15.3.3.1	Description	346

5.15.3.3.2	Callback URI	346

5.15.3.3.3	Operation Definition	346

5.15.3.3.3.1	Notification via HTTP POST	346

5.15.3.3.3.2	Void	347

5.15.4	Data Model	347

5.15.4.1	General	347

5.15.4.2	Reused data types	348

5.15.4.3	Structured data types	349

5.15.4.3.1	Introduction	349

5.15.4.3.2	Type: TimeSyncExposureSubsc	350

5.15.4.3.3	Type: TimeSyncCapability	352

5.15.4.3.4	Void	353

5.15.4.3.5	Void	353

5.15.4.3.6	Type: TimeSyncExposureConfig	353

5.15.4.3.7	Type: TimeSyncExposureSubsNotif	355

5.15.4.3.8	Type SubsEventNotification	355

5.15.4.3.9	Type: TimeSyncExposureConfigNotif	355

5.15.4.3.10	Type: EventFilter	355

5.15.4.3.11	Type: PtpCapabilitiesPerUe	356

5.15.4.3.12	Type: PtpInstance	356

5.15.4.3.13	Void	357

5.15.4.3.14	Void	357

5.15.4.3.15	Void	357

5.15.4.3.16	Void	357

5.15.4.3.17	Type: StateOfConfiguration	357

5.15.4.3.18	Type: ConfigForPort	358

5.15.4.3.19	Type: StateOfDstt	360

5.15.4.3.20	Void	361

5.15.4.3.21	Void	361

5.15.4.3.22	Type: AvailReport	361

5.15.4.4	Simple data types and enumerations	361

5.15.4.4.1	Introduction	361

5.15.4.4.2	Simple data types	361

5.15.4.4.3	Void	361

5.15.4.4.4	Enumeration: Protocol	361

5.15.4.4.5	Enumeration: GmCapable	361

5.15.4.4.6	Enumeration: SubscribedEvent	362

5.15.4.4.7	Enumeration: InstanceType	362

5.15.4.4.8	Enumeration: AsTimeResource	362

5.15.4.4.9	Enumeration: AcceptanceCriteriaResultIndication	362

5.15.4.4.10	Enumeration: AvailStatus	363

5.15.4.5	Data types describing alternative data types or combinations of data types	363

5.15.4.5.1	Type: ReportedCabability	363

5.15.5	Used Features	363

5.15.6	Error handling	363

5.15.6.1	General	363

5.15.6.2	Protocol Errors	363

5.15.6.3	Application Errors	364

5.16	EcsAddressProvision API	364

5.16.0	Introduction	364

5.16.1	Resources	364

5.16.1.1	Overview	364

5.16.1.2	Resource: ECS Address Provision Configurations	365

5.16.1.2.1	Introduction	365

5.16.1.2.2	Resource Definition	365

5.16.1.2.3	Resource Methods	365

5.16.1.2.3.1	General	365

5.16.1.2.3.2	GET	365

5.16.1.2.3.3	POST	366

5.16.1.3	Resource: Individual ECS Address Provision Configuration	367

5.16.1.3.1	Introduction	367

5.16.1.3.2	Resource Definition	367

5.16.1.3.3	Resource Methods	367

5.16.1.3.3.1	General	367

5.16.1.3.3.2	GET	367

5.16.1.3.3.3	PUT	368

5.16.1.3.3.4	DELETE	369

5.16.1A	Custom Operations without associated resources	370

5.16.1B	Notifications	370

5.16.2	Data Model	370

5.16.2.1	General	370

5.16.2.2	Reused data types	370

5.16.2.3	Structured data types	371

5.16.2.3.1	Introduction	371

5.16.2.3.2	Type: EcsAddressProvision	371

5.16.2.4	Simple data types and enumerations	371

5.16.2.4.1	Introduction	371

5.16.2.4.2	Simple data types	372

5.16.3	Used Features	372

5.16.4	Error handling	372

5.16.4.1	General	372

5.16.4.2	Protocol Errors	372

5.16.4.3	Application Errors	372

5.17	AMPolicyAuthorization API	372

5.17.0	Introduction	372

5.17.1	Resources	373

5.17.1.1	Overview	373

5.17.1.2	Resource: Application AM Contexts	374

5.17.1.2.1	Introduction	374

5.17.1.2.2	Resource Definition	374

5.17.1.2.3	Resource Methods	374

5.17.1.2.3.1	General	374

5.17.1.2.3.2	POST	374

5.17.1.3	Resource: Individual Application AM Context	375

5.17.1.3.1	Introduction	375

5.17.1.3.2	Resource Definition	375

5.17.1.3.3	Resource Methods	375

5.17.1.3.3.1	General	375

5.17.1.3.3.2	GET	375

5.17.1.3.3.3	PATCH	376

5.17.1.3.3.4	DELETE	378

5.17.1.4	Resource: AM Policy Events Subscription	379

5.17.1.4.1	Introduction	379

5.17.1.4.2	Resource Definition	379

5.17.1.4.3	Resource Methods	379

5.17.1.4.3.1	General	379

5.17.1.4.3.2	PUT	379

5.17.1.4.3.3	DELETE	381

5.17.1A	Custom Operations without associated resources	381

5.17.2	Notifications	382

5.17.2.1	Introduction	382

5.17.2.2	AM Event Notification	382

5.17.2.2.1	Description	382

5.17.2.2.2	Callback URI	382

5.17.2.2.3	Operation Definition	382

5.17.2.2.3.1	Notification via HTTP POST	382

5.17.2.2.3.2	Notification via Websocket	383

5.17.3	Data Model	383

5.17.3.1	General	383

5.17.3.2	Reused data types	384

5.17.3.3	Structured data types	384

5.17.3.3.1	Introduction	384

5.17.3.3.2	Type: AppAmContextExpData	385

5.17.3.3.3	Type: AppAmContextExpUpdateData	386

5.17.3.3.4	Type: GeographicalArea	386

5.17.3.4	Simple data types and enumerations	386

5.17.3.4.1	Introduction	386

5.17.3.4.2	Simple data types	386

5.17.3.5	Data types describing alternative data types or combinations of data types	386

5.17.3.5.1	Type: AppAmContextExpRespData	386

5.17.4	Used Features	387

5.17.5	Error handling	387

5.17.5.1	General	387

5.17.5.2	Protocol Errors	387

5.17.5.3	Application Errors	387

5.18	AMInfluence API	387

5.18.0	Introduction	387

5.18.1	Resources	388

5.18.1.1	Overview	388

5.18.1.2	Resource: AM Influence Subscription	388

5.18.1.2.1	Introduction	388

5.18.1.2.2	Resource Definition	389

5.18.1.2.3	Resource Methods	389

5.18.1.2.3.1	General	389

5.18.1.2.3.2	GET	389

5.18.1.2.3.3	POST	390

5.18.1.3	Resource: Individual AM Influence Subscription	390

5.18.1.3.1	Introduction	390

5.18.1.3.2	Resource Definition	390

5.18.1.3.3	Resource Methods	391

5.18.1.3.3.1	General	391

5.18.1.3.3.2	GET	391

5.18.1.3.3.3	PUT	392

5.18.1.3.3.4	PATCH	392

5.18.1.3.3.5	DELETE	393

5.18.1A	Custom Operations without associated resources	394

5.18.2	Notifications	394

5.18.2.1	Introduction	394

5.18.2.2	Event Notification	395

5.18.2.2.1	Description	395

5.18.2.2.2	Target URI	395

5.18.2.2.3	Operation Definition	395

5.18.3	Data Model	396

5.18.3.1	General	396

5.18.3.2	Reused data types	396

5.18.3.3	Structured data types	397

5.18.3.3.1	Introduction	397

5.18.3.3.2	Type: AmInfluSub	397

5.18.3.3.3	Type: AmInfluSubPatch	400

5.18.3.3.4	Type: AmInfluEventNotif	401

5.18.3.3.5	Type: DnnSnssaiInformation	402

5.18.3.4	Simple data types and enumerations	402

5.18.3.4.1	Introduction	402

5.18.3.4.2	Simple data types	402

5.18.3.4.3	Enumeration: AmInfluEvent	402

5.18.4	Used Features	402

5.18.5	Error handling	403

5.18.5.1	General	403

5.18.5.2	Protocol Errors	403

5.18.5.3	Application Errors	403

5.19	MBSTMGI API	403

5.19.1	Introduction	403

5.19.2	Resources	403

5.19.3	Custom Operations without associated resources	403

5.19.3.1	Overview	403

5.19.3.2	Operation: Allocate	404

5.19.3.2.1	Description	404

5.19.3.2.2	Operation Definition	404

5.19.3.3	Operation: Deallocate	405

5.19.3.3.1	Description	405

5.19.3.3.2	Operation Definition	405

5.19.4	Notifications	406

5.19.4.1	General	406

5.19.4.2	Notification of Allocated MBS TMGI(s) Timer Expiry	406

5.19.4.2.1	Description	406

5.19.4.2.2	Target URI	406

5.19.4.2.3	Operation Definition	407

5.19.4.2.3.1	Notification via HTTP POST	407

5.19.4.2.3.2	Notification via Websocket	407

5.19.5	Data Model	408

5.19.5.1	General	408

5.19.5.2	Structured data types	408

5.19.5.2.1	Introduction	408

5.19.5.2.2	Type: TmgiAllocRequest	409

5.19.5.2.3	Type: TmgiAllocResponse	410

5.19.5.2.4	Type: TmgiDeallocRequest	410

5.19.5.2.5	Type: ExpiryNotif	410

5.19.5.2.6	Type: ReducedMbsServArea	411

5.19.5.3	Simple data types and enumerations	411

5.19.5.3.1	Introduction	411

5.19.5.3.2	Simple data types	411

5.19.5.4	Data types describing alternative data types or combinations of data types	412

5.19.5.4.1	Type: ProblemDetailsTmgiAlloc	412

5.19.6	Used Features	412

5.19.7	Error handling	412

5.19.7.1	General	412

5.19.7.2	Protocol Errors	412

5.19.7.3	Application Errors	412

5.20	MBSSession API	413

5.20.1	Introduction	413

5.20.2	Resources	413

5.20.2.1	Overview	413

5.20.2.2	Resource: MBS sessions	415

5.20.2.2.1	Introduction	415

5.20.2.2.2	Resource Definition	415

5.20.2.2.3	Resource Methods	416

5.20.2.2.3.1	POST	416

5.20.2.2.4	Resource Custom Operations	416

5.20.2.3	Resource: Individual MBS Session	416

5.20.2.3.1	Introduction	416

5.20.2.3.2	Resource Definition	416

5.20.2.3.3	Resource Standard Methods	417

5.20.2.3.3.1	PATCH	417

5.20.2.3.3.3	DELETE	418

5.20.2.3.4	Resource Custom Operations	419

5.20.2.4	Resource: MBS Session Subscriptions	419

5.20.2.4.1	Introduction	419

5.20.2.4.2	Resource Definition	419

5.20.2.4.3	Resource Methods	419

5.20.2.4.3.1	GET	419

5.20.2.4.3.2	POST	420

5.20.2.4.4	Resource Custom Operations	421

5.20.2.5	Resource: Individual MBS Session Subscription	421

5.20.2.5.1	Introduction	421

5.20.2.5.2	Resource Definition	421

5.20.2.5.3	Resource Methods	421

5.20.2.5.3.1	GET	421

5.20.2.5.3.2	DELETE	422

5.20.2.5.4	Resource Custom Operations	423

5.20.2.6	Resource: MBS Parameters Provisionings	423

5.20.2.6.1	Introduction	423

5.20.2.6.2	Resource Definition	423

5.20.2.6.3	Resource Methods	424

5.20.2.6.3.1	GET	424

5.20.2.6.3.2	POST	425

5.20.2.6.4	Resource Custom Operations	425

5.20.2.7	Resource: Individual MBS Parameters Provisioning	425

5.20.2.7.1	Introduction	425

5.20.2.7.2	Resource Definition	426

5.20.2.7.3	Resource Methods	426

5.20.2.7.3.1	GET	426

5.20.2.7.3.2	PUT	427

5.20.2.7.3.3	PATCH	428

5.20.2.7.3.4	DELETE	429

5.20.2.7.4	Resource Custom Operations	430

5.20.3	Custom Operations without associated resources	430

5.20.4	Notifications	430

5.20.4.1	General	430

5.20.4.2	MBS Session Status Notification	430

5.20.4.2.1	Description	430

5.20.4.2.2	Target URI	430

5.20.4.2.3	Operation Definition	431

5.20.4.2.3.1	Notification via HTTP POST	431

5.20.5	Data Model	431

5.20.5.1	General	431

5.20.5.2	Structured data types	432

5.20.5.2.1	Introduction	432

5.20.5.2.2	Type: MbsSessionCreateReq	433

5.20.5.2.3	Type: MbsSessionCreateRsp	433

5.20.5.2.4	Type: MbsSessionSubsc	433

5.20.5.2.5	Type: MbsSessionStatusNotif	433

5.20.5.2.6	Type: MbsPpData	434

5.20.5.2.7	Type: MbsSessAuthData	434

5.20.5.2.8	Type: MbsPpDataPatch	435

5.20.5.2.9	Type: MbsSessAssistInfo	435

5.20.5.2.10	Type: MbsSessionUpdateResp	435

5.20.5.3	Simple data types and enumerations	435

5.20.5.3.1	Introduction	435

5.20.5.3.2	Simple data types	436

5.20.6	Used Features	436

5.20.7	Error handling	436

5.20.7.1	General	436

5.20.7.2	Protocol Errors	436

5.20.7.3	Application Errors	436

5.21	EASDeployment API	437

5.21.0	Introduction	437

5.21.1	Resources	437

5.21.1.1	Overview	437

5.21.1.2	Resource: EAS Deployment Information	438

5.21.1.2.1	Introduction	438

5.21.1.2.2	Resource Definition	438

5.21.1.2.3	Resource Methods	438

5.21.1.2.3.1	General	438

5.21.1.2.3.2	GET	439

5.21.1.2.3.3	POST	439

5.21.1.3	Resource: Individual EAS Deployment Information	440

5.21.1.3.1	Introduction	440

5.21.1.3.2	Resource Definition	440

5.21.1.3.3	Resource Methods	440

5.21.1.3.3.1	General	440

5.21.1.3.3.2	GET	440

5.21.1.3.3.3	PUT	441

5.21.1.3.3.4	DELETE	442

5.21.2	Custom Operations without associated resources	443

5.21.2.1	Overview	443

5.21.2.2	Operation: remove-edis	444

5.21.2.2.1	Description	444

5.21.2.2.2	Operation Definition	444

5.21.3	Notifications	444

5.21.4	Data Model	444

5.21.4.1	General	444

5.21.4.2	Reused data types	445

5.21.4.3	Structured data types	445

5.21.4.3.1	Introduction	445

5.21.4.3.2	Type: EasDeployInfo	446

5.21.4.3.3	Type: DnaiInformation	446

5.21.4.3.4	Type: DnsServerIdentifier	447

5.21.4.3.5	Type: EdiDeleteCriteria	447

5.21.4.4	Simple data types and enumerations	448

5.21.4.4.1	Introduction	448

5.21.4.4.2	Simple data types	448

5.21.5	Used Features	448

5.21.6	Error handling	448

5.21.6.1	General	448

5.21.6.2	Protocol Errors	449

5.21.6.3	Application Errors	449

5.22	ASTI API	449

5.22.0	Introduction	449

5.22.1	Resources	449

5.22.1.1	Overview	449

5.22.1.2	Resource: ASTI Configurations	450

5.22.1.2.1	Introduction	450

5.22.1.2.2	Resource Definition	450

5.22.1.2.3	Resource Methods	451

5.22.1.2.3.1	General	451

5.22.1.2.3.2	GET	451

5.22.1.2.3.3	POST	452

5.22.1.2.4	Resource Custom Operations	452

5.22.1.2.4.1	Overview	452

5.22.1.2.4.2	Operation: retrieve	452

5.22.1.2.4.2.1	Description	452

5.22.1.2.4.2.2	Operation Definition	453

5.22.1.3	Resource: Individual ASTI Configuration	453

5.22.1.3.1	Introduction	453

5.22.1.3.2	Resource Definition	453

5.22.1.3.3	Resource Methods	453

5.22.1.3.3.1	General	453

5.22.1.3.3.2	GET	453

5.22.1.3.3.3	PUT	454

5.22.1.3.3.4	DELETE	455

5.22.2	Custom Operations without associated resources	456

5.22.3	Notifications	456

5.22.3.1	General	456

5.22.3.2	ASTI Notification	456

5.22.3.2.1	Description	456

5.22.3.2.2	Target URI	456

5.22.3.2.3	Operation Definition	457

5.22.3.2.3.1	Notification via HTTP POST	457

5.22.3.2.3.2	Void	458

5.22.4	Data Model	458

5.22.4.1	General	458

5.22.4.2	Reused data types	458

5.22.4.3	Structured data types	458

5.22.4.3.1	Introduction	458

5.22.4.3.2	Type: AccessTimeDistributionData	459

5.22.4.3.3	Type: StatusRequestData	459

5.22.4.3.4	Type: StatusResponseData	460

5.22.4.3.5	Type: ActiveUe	460

5.22.4.3.6	Type AstiConfigNotification	460

5.22.4.3.7	Type AstiConfigStateNotification	460

5.22.4.4	Simple data types and enumerations	460

5.22.4.4.1	Introduction	460

5.22.4.4.2	Simple data types	461

5.22.5	Used Features	461

5.22.6	Error handling	461

5.22.6.1	General	461

5.22.6.2	Protocol Errors	461

5.22.6.3	Application Errors	461

5.23	DataReporting API	462

5.23.1	Introduction	462

5.23.2	Resources	462

5.23.2.1	Overview	462

5.23.2.2	Resource: Data Reporting Sessions	463

5.23.2.2.1	Introduction	463

5.23.2.2.2	Resource definition	463

5.23.2.2.3	Resource Methods	463

5.23.2.2.3.1	POST	463

5.23.2.3	Resource: Individual Data Reporting Session	464

5.23.2.3.1	Introduction	464

5.23.2.3.2	Resource Definition	464

5.23.2.3.3	Resource standard methods	464

5.23.2.3.3.1	GET	464

5.23.2.3.3.2	PUT	465

5.23.2.3.3.3	DELETE	467

5.23.2.3.4	Resource custom operations	468

5.23.3	Custom Operations without associated resources	469

5.23.4	Notifications	469

5.23.5	Data Model	469

5.23.3.1	General	469

5.23.6	Used Features	469

5.23.7	Error handling	469

5.23.7.1	General	469

5.23.7.2	Protocol Errors	469

5.23.7.3	Application Errors	469

5.24	DataReportingProvisioning API	470

5.24.1	Introduction	470

5.24.2	Resources	470

5.24.2.1	Overview	470

5.24.2.2	Resource: Data Reporting Provisioning Sessions	472

5.24.2.2.1	Introduction	472

5.24.2.2.2	Resource definition	473

5.24.2.2.3	Resource Methods	473

5.24.2.2.3.1	POST	473

5.24.2.3	Resource: Individual Data Reporting Provisioning Session	474

5.24.2.3.1	Introduction	474

5.24.2.3.2	Resource Definition	474

5.24.2.3.3	Resource standard methods	474

5.24.2.3.3.1	GET	474

5.24.2.3.3.2	Void	475

5.24.2.3.3.3	DELETE	475

5.24.2.4	Resource: Data Reporting Configurations	476

5.24.2.4.1	Introduction	476

5.24.2.4.2	Resource definition	477

5.24.2.4.3	Resource Methods	477

5.24.2.4.3.1	POST	477

5.24.2.5	Resource: Individual Data Reporting Configuration	479

5.24.2.5.1	Introduction	479

5.24.2.5.2	Resource Definition	479

5.24.2.5.3	Resource standard methods	479

5.24.2.5.3.2	GET	479

5.24.2.5.3.3	PUT	480

5.24.2.5.3.3A	PATCH	482

5.24.2.5.3.4	DELETE	483

5.24.3	Custom Operations without associated resources	483

5.24.4	Notifications	484

5.24.5	Data Model	484

5.24.5.1	General	484

5.24.6	Used Features	484

5.24.7	Error handling	484

5.24.7.1	General	484

5.24.7.2	Protocol Errors	484

5.24.7.3	Application Errors	485

5.25	UEId API	485

5.25.1	Introduction	485

5.25.2	Resources	485

5.25.2.1	Overview	485

5.25.2.2	Resource: UE ID Mapping Information Provisionings	486

5.25.2.2.1	Introduction	486

5.25.2.2.2	Resource Definition	486

5.25.2.2.3	Resource Methods	486

5.25.2.2.3.1	GET	486

5.25.2.2.3.2	POST	487

5.25.2.2.4	Resource Custom Operations	488

5.25.2.3	Resource: Individual UE ID Mapping Provisioning	488

5.25.2.3.1	Introduction	488

5.25.2.3.2	Resource Definition	488

5.25.2.3.3	Resource Methods	489

5.25.2.3.3.1	GET	489

5.25.2.3.3.2	PUT	490

5.25.2.3.3.3	PATCH	491

5.25.2.3.3.4	DELETE	492

5.25.3	Custom Operations without associated resources	493

5.25.3.1	Overview	493

5.25.3.2	Operation: Retrieve	494

5.25.3.2.1	Description	494

5.25.3.2.2	Operation Definition	494

5.25.3.2.3	Void	495

5.25.3.3	Operation: GetMsisdn	495

5.25.3.3.1	Description	495

5.25.3.3.2	Operation Definition	495

5.25.4	Notifications	496

5.25.5	Data Model	496

5.25.5.1	General	496

5.25.5.2	Structured data types	497

5.25.5.2.1	Introduction	497

5.25.5.2.2	Type: UeIdReq	498

5.25.5.2.3	Type: UeIdInfo	498

5.25.5.2.4	Type: MsisdnReq	499

5.25.5.2.5	Type: MsisdnInfo	499

5.25.5.2.6	Type: UeIdMappingInfo	500

5.25.5.2.7	Type: RangSlUeIdMappInfo	500

5.25.5.2.8	Type: UeIdMappingInfoPatch	500

5.25.5.3	Simple data types and enumerations	500

5.25.5.3.1	Introduction	500

5.25.5.3.2	Simple data types	500

5.25.6	Used Features	501

5.25.7	Error handling	501

5.25.7.1	General	501

5.25.7.2	Protocol Errors	501

5.25.7.3	Application Errors	501

5.25.8	Security	501

5.26	MBSUserService API	501

5.26.1	Introduction	501

5.26.2	Resources	502

5.26.2.1	Overview	502

5.26.2.2	Resource: MBS User Services	503

5.26.2.2.1	Introduction	503

5.26.2.2.2	Resource Definition	503

5.26.2.2.3	Resource Standard Methods	503

5.26.2.2.3.1	GET	503

5.26.2.2.3.2	POST	504

5.26.2.2.4	Resource Custom Operations	504

5.26.2.3	Resource: Individual MBS User Service	505

5.26.2.3.1	Introduction	505

5.26.2.3.2	Resource Definition	505

5.26.2.3.3	Resource Standard Methods	505

5.26.2.3.3.1	GET	505

5.26.2.3.3.2	PUT	506

5.26.2.3.3.3	PATCH	507

5.26.2.3.3.4	DELETE	508

5.26.2.3.4	Resource Custom Operations	509

5.26.3	Custom Operations without associated resources	509

5.26.4	Notifications	509

5.26.5	Data Model	509

5.26.5.1	General	509

5.26.5.2	Structured data types	510

5.26.5.2.1	Introduction	510

5.26.5.3	Simple data types and enumerations	510

5.26.5.3.1	Introduction	510

5.26.5.3.2	Simple data types	510

5.26.6	Used Features	510

5.26.7	Error handling	510

5.26.7.1	General	510

5.26.7.2	Protocol Errors	511

5.26.7.3	Application Errors	511

5.27	MBSUserDataIngestSession API	511

5.27.1	Introduction	511

5.27.2	Resources	511

5.27.2.1	Overview	511

5.27.2.2	Resource: MBS User Data Ingest Sessions	513

5.27.2.2.1	Introduction	513

5.27.2.2.2	Resource Definition	513

5.27.2.2.3	Resource Standard Methods	513

5.27.2.2.3.1	GET	513

5.27.2.2.3.2	POST	514

5.27.2.2.4	Resource Custom Operations	515

5.27.2.3	Resource: Individual MBS User Data Ingest Session	515

5.27.2.3.1	Introduction	515

5.27.2.3.2	Resource Definition	515

5.27.2.3.3	Resource Standard Methods	515

5.27.2.3.3.1	GET	515

5.27.2.3.3.2	PUT	516

5.27.2.3.3.3	PATCH	517

5.27.2.3.3.4	DELETE	519

5.27.2.3.4	Resource Custom Operations	519

5.27.2.4	Resource: MBS User Data Ingest Session Status Subscriptions	520

5.27.2.4.1	Introduction	520

5.27.2.4.2	Resource Definition	520

5.27.2.4.3	Resource Standard Methods	520

5.27.2.4.3.1	GET	520

5.27.2.4.3.2	POST	521

5.27.2.4.4	Resource Custom Operations	522

5.27.2.5	Resource: Individual MBS User Data Ingest Session Status Subscription	522

5.27.2.5.1	Introduction	522

5.27.2.5.2	Resource Definition	522

5.27.2.5.3	Resource Standard Methods	522

5.27.2.5.3.1	GET	522

5.27.2.5.3.2	PUT	523

5.27.2.5.3.3	PATCH	524

5.27.2.5.3.4	DELETE	525

5.27.2.5.4	Resource Custom Operations	526

5.27.3	Custom Operations without associated resources	526

5.27.4	Notifications	526

5.27.4.1	General	526

5.27.4.2	MBS User Data Ingest Session Status Change Notification	527

5.27.4.2.1	Description	527

5.27.4.2.2	Target URI	527

5.27.4.2.3	Operation Definition	527

5.27.4.2.3.1	Notification via HTTP POST	527

5.27.4.2.3.2	Void	528

5.27.5	Data Model	528

5.27.5.1	General	528

5.27.5.2	Structured data types	529

5.27.5.2.1	Introduction	529

5.27.5.3	Simple data types and enumerations	529

5.27.5.3.1	Introduction	529

5.27.5.3.2	Simple data types	529

5.27.6	Used Features	529

5.27.7	Error handling	530

5.27.7.1	General	530

5.27.7.2	Protocol Errors	530

5.27.7.3	Application Errors	530

5.28	MSEventExposure API	531

5.28.1	Introduction	531

5.28.2	Resources	532

5.28.2.1	Overview	532

5.28.2.2	Resource: Media Streaming Event Exposure Subscriptions	532

5.28.2.2.1	Introduction	532

5.28.2.2.2	Resource Definition	533

5.28.2.2.3	Resource Standard Methods	533

5.28.2.2.3.1	GET	533

5.28.2.2.3.2	POST	534

5.28.2.2.4	Resource Custom Operations	534

5.28.2.3	Resource: Individual Media Streaming Event Exposure Subscription	534

5.28.2.3.1	Introduction	534

5.28.2.3.2	Resource Definition	535

5.28.2.3.3	Resource Standard Methods	535

5.28.2.3.3.1	GET	535

5.28.2.3.3.2	PUT	536

5.28.2.3.3.3	DELETE	537

5.28.2.3.4	Resource Custom Operations	538

5.28.3	Custom Operations without associated resources	538

5.28.4	Notifications	538

5.28.4.1	General	538

5.28.4.2	Media Streaming Event Exposure Notification	538

5.28.4.2.1	Description	538

5.28.4.2.2	Target URI	538

5.28.4.2.3	Operation Definition	538

5.28.4.2.3.1	Notification via HTTP POST	538

5.28.4.2.3.2	Void	539

5.28.5	Data Model	539

5.28.5.1	General	539

5.28.5.2	Structured data types	540

5.28.5.2.1	Introduction	540

5.28.5.3	Simple data types and enumerations	540

5.28.5.3.1	Introduction	540

5.28.5.3.2	Simple data types	540

5.28.6	Used Features	540

5.28.7	Error handling	540

5.28.7.1	General	540

5.28.7.2	Protocol Errors	541

5.28.7.3	Application Errors	541

5.29	MBSGroupMsgDelivery API	541

5.29.1	Introduction	541

5.29.2	Resources	541

5.29.2.1	Overview	541

5.29.2.2	Resource: MBS Group Message Deliveries	542

5.29.2.2.1	Introduction	542

5.29.2.2.2	Resource Definition	542

5.29.2.2.3	Resource Standard Methods	543

5.29.2.2.3.1	GET	543

5.29.2.2.3.2	POST	544

5.29.2.2.4	Resource Custom Operations	544

5.29.2.3	Resource: Individual MBS Group Message Delivery	544

5.29.2.3.1	Introduction	544

5.29.2.3.2	Resource Definition	544

5.29.2.3.3	Resource Standard Methods	545

5.29.2.3.3.1	GET	545

5.29.2.3.3.2	PATCH	546

5.29.2.3.3.3	DELETE	547

5.29.2.3.4	Resource Custom Operations	548

5.29.3	Custom Operations without associated resources	548

5.29.4	Notifications	548

5.29.4.1	General	548

5.29.4.2	MBS Group Message Delivery Notification	548

5.29.4.2.1	Description	548

5.29.4.2.2	Target URI	548

5.29.4.2.3	Operation Definition	549

5.29.5	Data Model	549

5.29.5.1	General	549

5.29.5.2	Structured data types	550

5.29.5.2.1	Introduction	550

5.29.5.2.2	Void	551

5.29.5.2.3	Type: MbsGroupMsgDel	551

5.29.5.2.4	Void	553

5.29.5.2.5	Type: MbsGroupMsgDelPatch	553

5.29.5.2.6	Type: MbsGroupMsgDelStatusNotif	553

5.29.5.3	Simple data types and enumerations	553

5.29.5.3.1	Introduction	553

5.29.5.3.2	Simple data types	553

5.29.5.4	Data types describing alternative data types or combinations of data types	554

5.29.5.4.1	Type: MbsServArea	554

5.29.6	Used Features	554

5.29.7	Error handling	554

5.29.7.1	General	554

5.29.7.2	Protocol Errors	554

5.29.7.3	Application Errors	554

5.30	DNAIMapping API	554

5.30.2	Resources	555

5.30.2.1	Overview	555

5.30.2.2	Resource: DNAI Mapping Subscriptions	556

5.30.2.2.1	Introduction	556

5.30.2.2.2	Resource Definition	556

5.30.2.2.3	Resource Standard Methods	556

5.30.2.2.3.1	Void	556

5.30.2.2.3.2	GET	556

5.30.2.2.3.3	POST	557

5.30.2.2.4	Resource Custom Operations	558

5.30.2.3	Resource: Individual DNAI Mapping Subscription	558

5.30.2.3.1	Introduction	558

5.30.2.3.2	Resource Definition	558

5.30.2.3.3	Resource Standard Methods	558

5.30.2.3.3.1	Void	558

5.30.2.3.3.2	GET	558

5.30.2.3.3.3	DELETE	559

5.30.2.3.4	Resource Custom Operations	560

5.30.4	Notifications	561

5.30.4.1	General	561

5.30.4.2	DNAI Mapping Information Update Notification	561

5.30.4.2.1	Description	561

5.30.4.2.2	Target URI	561

5.30.4.2.3	Operation Definition	561

5.30.4.2.3.1	Notification via HTTP POST	561

5.30.4.2.3.2	Void	562

5.30.5	Data Model	562

5.30.5.1	General	562

5.30.5.2	Structured data types	563

5.30.5.2.1	Introduction	563

5.30.5.2.2	Type: DnaiMapSub	564

5.30.5.2.3	Type: DnaiMapUpdateNotif	565

5.30.5.2.4	Void	565

5.30.5.3	Simple data types and enumerations	565

5.30.5.3.1	Introduction	565

5.30.5.3.2	Simple data types	565

5.30.5.4	Data types describing alternative data types or combinations of data types	565

5.30.6	Used Features	565

5.30.7	Error handling	566

5.30.7.1	General	566

5.30.7.2	Protocol Errors	566

5.30.7.3	Application Errors	566

5.31	PdtqPolicyNegotiation API	566

5.31.0	Introduction	566

5.31.1	Resources	566

5.31.1.1	Overview	566

5.31.1.2	Resource: PDTQ Policy Subscriptions	567

5.31.1.2.1	Introduction	567

5.31.1.2.2	Resource Definition	567

5.31.1.2.3	Resource Standard Methods	568

5.31.1.2.3.1	Void	568

5.31.1.2.3.2	GET	568

5.31.1.2.3.3	POST	569

5.31.1.2.4	Resource Custom Operations	569

5.31.1.3	Resource: Individual PDTQ Policy Subscription	569

5.31.1.3.1	Introduction	569

5.31.1.3.2	Resource Definition	570

5.31.1.3.3	Resource Standard Methods	570

5.31.1.3.3.1	Void	570

5.31.1.3.3.2	GET	570

5.31.1.3.3.3	PATCH	571

5.31.1.3.3.4	DELETE	572

5.31.1.3.4	Resource Custom Operations	573

5.31.1A	Custom Operations without associated resources	573

5.31.2	Notifications	573

5.31.2.1	General	573

5.31.2.2	PDTQ Warning Notification	574

5.31.2.2.1	Description	574

5.31.2.2.2	Target URI	574

5.31.2.2.3	Operation Definition	574

5.31.2.2.3.1	Notification via HTTP POST	574

5.31.3	Data Model	575

5.31.3.1	General	575

5.31.3.2	Void	575

5.31.3.3	Structured data types	575

5.31.3.3.1	Introduction	575

5.31.3.3.2	Type: Pdtq	576

5.31.3.3.3	Type: PdtqPatch	578

5.31.3.3.4	Type: PdtqNotification	578

5.31.3.4	Simple data types and enumerations	578

5.31.3.4.1	Introduction	578

5.31.3.4.2	Simple data types	578

5.31.3.4	Data types describing alternative data types or combinations of data types	579

5.31.4	Used Features	579

5.31.5	Error handling	579

5.31.5.1	General	579

5.31.5.2	Protocol Errors	579

5.31.5.3	Application Errors	579

5.32	MemberUESelectionAssistance API	579

5.32.1	Introduction	579

5.32.2	Resources	580

5.32.2.1	Overview	580

5.32.2.2	Resource: Member UE Selection Assistance Subscriptions	581

5.32.2.2.1	Introduction	581

5.32.2.2.2	Resource definition	581

5.32.2.2.3	Resource Standard Methods	582

5.32.2.2.3.1	POST	582

5.32.2.2.3.2	GET	582

5.32.2.2.4	Resource Custom Operations	583

5.32.2.3	Resource: Individual Member UE Selection Assistance Subscription	583

5.32.2.3.1	Introduction	583

5.32.2.3.2	Resource Definition	584

5.32.2.3.3	Resource Standard Methods	584

5.32.2.3.3.1	GET	584

5.32.2.3.3.2	PUT	585

5.32.2.3.3.3	PATCH	586

5.32.2.3.3.4	DELETE	587

5.32.2.3.4	Resource Custom Operations	588

5.32.3	Custom Operations without associated resources	588

5.32.4	Notifications	588

5.32.5	Data Model	590

5.32.5.1	General	590

5.32.5.2	Structured data types	592

5.32.5.2.1	Introduction	592

5.32.5.2.2	Type: MemUeSelectAssistSubsc	593

5.32.5.2.3	Type: MemUeSeletAssistNotif	595

5.32.5.2.4	Type: QoSFilterCriteria	596

5.32.5.2.5	Type: AccessRatTypeFilterCriteria	596

5.32.5.2.6	Type: E2ETransTimeFilterCriteria	597

5.32.5.2.7	Type: UeLocFilterCriteria	597

5.32.5.2.8	Type: UeHisLocFilterCriteria	598

5.32.5.2.9	Type: UeDirectionFilterCriteria	598

5.32.5.2.10	Type: UeDistanceFilterCriteria	598

5.32.5.2.11	Type: ServiceExpFilterCriteria	599

5.32.5.2.12	Type: DnnFilterCriteria	599

5.32.5.2.13	Type: CandiUeListInfo	600

5.32.5.2.14	Type: MemUeSeletReport	600

5.32.5.2.15	Type: MemUeSelectAssistSubscPatch	601

5.32.5.2.16	Type: CandUeInfo	602

5.32.5.3	Simple data types and enumerations	602

5.32.5.3.1	Introduction	602

5.32.5.3.2	Simple data types	602

5.32.5.3.3	Enumeration: FilterCriterionType	603

5.32.5.4	Data types describing alternative data types or combinations of data types	603

5.32.6	Used Features	603

5.32.7	Error handling	603

5.32.7.1	General	603

5.32.7.2	Protocol Errors	603

5.32.7.3	Application Errors	603

5.33	GroupParametersProvisioning API	604

5.33.1	Introduction	604

5.33.2	Resources	604

5.33.2.1	Overview	604

5.33.2.2	Resource: Group Parameters Provisionings	605

5.33.2.2.1	Introduction	605

5.33.2.2.2	Resource Definition	605

5.33.2.2.3	Resource Methods	605

5.33.2.2.3.1	GET	605

5.33.2.2.3.2	POST	606

5.33.2.2.4	Resource Custom Operations	607

5.33.2.3	Resource: Individual Group Parameters Provisioning	607

5.33.2.3.1	Introduction	607

5.33.2.3.2	Resource Definition	607

5.33.2.3.3	Resource Methods	608

5.33.2.3.3.1	GET	608

5.33.2.3.3.2	PUT	609

5.33.2.3.3.3	PATCH	610

5.33.2.3.3.4	DELETE	611

5.33.2.3.4	Resource Custom Operations	612

5.33.3	Custom Operations without associated resources	612

5.33.4	Notifications	612

5.33.5	Data Model	612

5.33.5.1	General	612

5.33.5.2	Structured data types	613

5.33.5.2.1	Introduction	613

5.33.5.2.2	Type: GrpPpData	613

5.33.5.2.3	Type: GrpPpDataPatch	613

5.33.5.2.4	Type: DnnSnssaiGrpData	614

5.33.5.2.5	Type: AfReqDefaultQoS	614

5.33.5.2.6	Type: LadnServArea	614

5.33.5.3	Simple data types and enumerations	614

5.33.5.3.1	Introduction	614

5.33.5.3.2	Simple data types	615

5.33.5.4	Data types describing alternative data types or combinations of data types	615

5.33.6	Used Features	615

5.33.7	Error handling	615

5.33.7.1	General	615

5.33.7.2	Protocol Errors	615

5.33.7.3	Application Errors	615

5.34	SliceParamProvision API	615

5.34.1	Introduction	615

5.34.2	Resources	616

5.34.2.1	Overview	616

5.34.2.2	Resource: Slice Parameters Provisionings	617

5.34.2.2.1	Introduction	617

5.34.2.2.2	Resource Definition	617

5.34.2.2.3	Resource Methods	617

5.34.2.2.3.1	GET	617

5.34.2.2.3.2	POST	618

5.34.2.2.4	Resource Custom Operations	619

5.34.2.3	Resource: Individual Slice Parameters Provisioning	619

5.34.2.3.1	Introduction	619

5.34.2.3.2	Resource Definition	619

5.34.2.3.3	Resource Methods	620

5.34.2.3.3.1	GET	620

5.34.2.3.3.2	PUT	621

5.34.2.3.3.3	PATCH	622

5.34.2.3.3.4	DELETE	623

5.34.2.3.4	Resource Custom Operations	624

5.34.3	Custom Operations without associated resources	624

5.34.4	Notifications	624

5.34.5	Data Model	624

5.34.5.1	General	624

5.34.5.2	Structured data types	625

5.34.5.2.1	Introduction	625

5.34.5.2.2	Type: SlicePpData	625

5.34.5.2.3	Type: SlicePpDataPatch	625

5.34.5.3	Simple data types and enumerations	625

5.34.5.3.1	Introduction	625

5.34.5.3.2	Simple data types	625

5.34.6	Used Features	626

5.34.7	Error handling	626

5.34.7.1	General	626

5.34.7.2	Protocol Errors	626

5.34.7.3	Application Errors	626

5.35	UeAddress API	626

5.35.1	Introduction	626

5.35.2	Resources	627

5.35.2.1	Overview	627

5.35.2.2	Resource: UE Address Subscriptions	627

5.35.2.2.1	Introduction	627

5.35.2.2.2	Resource Definition	627

5.35.2.2.3	Resource Methods	628

5.35.2.2.3.1	General	628

5.35.2.2.3.2	GET	628

5.35.2.2.3.3	POST	629

5.35.2.3	Resource: Individual UE Address Subscription	629

5.35.2.3.1	Introduction	629

5.35.2.3.2	Resource Definition	629

5.35.2.3.3	Resource Methods	630

5.35.2.3.3.1	General	630

5.35.2.3.3.2	GET	630

5.35.2.3.3.3	DELETE	631

5.35.3	Custom Operations without associated resources	632

5.35.3.1	Overview	632

5.35.3.2	Operation: Retrieve	632

5.35.3.2.1	Description	632

5.35.3.2.2	Operation Definition	632

5.35.4	Notifications	633

5.35.4.1	General	633

5.35.4.2	UE Address Notification	633

5.35.4.2.1	Description	633

5.35.4.2.2	Target URI	634

5.35.4.2.3	Operation Definition	634

5.35.4.2.3.1	Notification via HTTP POST	634

5.35.5	Data Model	635

5.35.5.1	General	635

5.35.5.2	Structured data types	635

5.35.5.2.1	Introduction	635

5.35.5.2.2	Type: UeAddressReq	635

5.35.5.2.3	Type: UeAddressInfo	636

5.35.2.2.4	Type: UeAddrSubsc	636

5.35.2.2.5	Type: UeAddrInfo	637

5.35.2.2.6	Type: UeAddrNotif	637

5.35.2.2.7	Type: UeNatAddrInfo	637

5.35.5.3	Simple data types and enumerations	637

5.35.5.3.1	Introduction	637

5.35.5.3.2	Simple data types	637

5.35.5.4	Data types describing alternative data types or combinations of data types	638

5.35.6	Used Features	638

5.35.7	Error handling	638

5.35.7.1	General	638

5.35.7.2	Protocol Errors	638

5.35.7.3	Application Errors	638

5.36	ECSAddress API	638

5.36.1	Introduction	638

5.36.2	Resources	639

5.36.2.1	Overview	639

5.36.2.2	Resource: ECS Address Configuration Information Sets	640

5.36.2.2.1	Introduction	640

5.36.2.2.2	Resource Definition	640

5.36.2.2.3	Resource Standard Methods	640

5.36.2.2.3.1	Void	640

5.36.2.2.3.2	GET	640

5.36.2.2.3.3	POST	641

5.36.2.2.4	Resource Custom Operations	642

5.36.2.3	Resource: Individual ECS Address Configuration Information Set	642

5.36.2.3.1	Introduction	642

5.36.2.3.2	Resource Definition	642

5.36.2.3.3	Resource Standard Methods	643

5.36.2.3.3.1	Void	643

5.36.2.3.3.2	GET	643

5.36.2.3.3.3	PUT	644

5.36.2.3.3.3A	PATCH	645

5.36.2.3.3.4	DELETE	646

5.36.2.3.4	Resource Custom Operations	647

5.36.3	Custom Operations without associated resources	647

5.36.3.1	Overview	647

5.36.3.2	Operation: RemoveEcsAddr	648

5.36.3.2.1	Description	648

5.36.3.2.2	Operation Definition	648

5.36.4	Notifications	649

5.36.5	Data Model	649

5.36.5.1	General	649

5.36.5.2	Void	650

5.36.5.3	Structured data types	650

5.36.5.3.1	Introduction	650

5.36.5.3.2	Type: EcsAddrInfo	650

5.36.5.3.3	Type: EcsAddrDeleteCriteria	651

5.36.5.3.4	Type: EcsAddrInfoPatch	651

5.36.5.4	Simple data types and enumerations	651

5.36.5.4.1	Introduction	651

5.36.5.4.2	Simple data types	652

5.36.5.4	Data types describing alternative data types or combinations of data types	652

5.36.6	Used Features	652

5.36.7	Error handling	652

5.36.7.1	General	652

5.36.7.2	Protocol Errors	652

5.36.7.3	Application Errors	652

5.37	RSLPPIParametersProvisioning API	652

5.37.1	Introduction	652

5.37.2	Resources	653

5.37.2.1	Overview	653

5.37.2.2	Resource: RSLPPI Parameters Provisionings	654

5.37.2.2.1	Introduction	654

5.37.2.2.2	Resource Definition	654

5.37.2.2.3	Resource Methods	654

5.37.2.2.3.1	GET	654

5.37.2.2.3.2	POST	655

5.37.2.2.4	Resource Custom Operations	656

5.37.2.3	Resource: Individual RSLPPI Parameters Provisioning	656

5.37.2.3.1	Introduction	656

5.37.2.3.2	Resource Definition	656

5.37.2.3.3	Resource Methods	657

5.37.2.3.3.1	GET	657

5.37.2.3.3.2	PUT	658

5.37.2.3.3.3	PATCH	659

5.37.2.3.3.4	DELETE	660

5.37.2.3.4	Resource Custom Operations	661

5.37.3	Custom Operations without associated resources	661

5.37.4	Notifications	661

5.37.5	Data Model	661

5.37.5.1	General	661

5.37.5.2	Structured data types	662

5.37.5.2.1	Introduction	662

5.37.5.2.2	Type: RslppiData	662

5.37.5.2.3	Type: RslppiPpData	662

5.37.5.2.4	Type: RslppiPpDataPatch	662

5.37.5.3	Simple data types and enumerations	662

5.37.5.3.1	Introduction	662

5.37.5.3.2	Simple data types	663

5.37.5.4	Data types describing alternative data types or combinations of data types	663

5.37.6	Used Features	663

5.37.7	Error handling	663

5.37.7.1	General	663

5.37.7.2	Protocol Errors	663

5.37.7.3	Application Errors	663

5.38	AddressingParamProvision API	663

5.38.1	Introduction	663

5.38.2	Resources	664

5.38.2.1	Overview	664

5.38.2.2	Resource: Addressing Parameters Provisionings	665

5.38.2.2.1	Introduction	665

5.38.2.2.2	Resource Definition	665

5.38.2.2.3	Resource Methods	665

5.38.2.2.3.1	GET	665

5.38.2.2.3.2	POST	666

5.38.2.2.4	Resource Custom Operations	667

5.38.2.3	Resource: Individual Addressing Parameters Provisioning	667

5.38.2.3.1	Introduction	667

5.38.2.3.2	Resource Definition	667

5.38.2.3.3	Resource Methods	668

5.38.2.3.3.1	GET	668

5.38.2.3.3.2	PUT	669

5.38.2.3.3.3	PATCH	670

5.38.2.3.3.4	DELETE	671

5.38.2.3.4	Resource Custom Operations	672

5.38.3	Custom Operations without associated resources	672

5.38.4	Notifications	672

5.38.5	Data Model	672

5.38.5.1	General	672

5.38.5.2	Structured data types	673

5.38.5.2.1	Introduction	673

5.38.5.2.2	Type: AddrPpData	673

5.38.5.2.3	Type: StaticIpAddrParams	673

5.38.5.2.4	Type: AddrPpDataPatch	673

5.38.5.3	Simple data types and enumerations	673

5.38.5.3.1	Introduction	673

5.38.5.3.2	Simple data types	673

5.38.5.4	Data types describing alternative data types or combinations of data types	674

5.38.6	Used Features	674

5.38.7	Error handling	674

5.38.7.1	General	674

5.38.7.2	Protocol Errors	674

5.38.7.3	Application Errors	674

5.39	UAVFlightAssistance API	674

5.39.1	Introduction	674

5.39.2	Resources	675

5.39.2.2	Resource: UAV Flight Assistance Configurations	676

5.39.2.2.1	Introduction	676

5.39.2.2.2	Resource definition	676

5.39.2.2.3	Resource Methods	676

5.39.2.2.3.1	POST	676

5.39.2.3	Resource: Individual UAV Flight Assistance Configuration	677

5.39.2.3.1	Introduction	677

5.39.2.3.2	Resource Definition	677

5.39.2.3.3	Resource standard methods	677

5.39.2.3.3.1	PUT	677

5.39.2.3.3.2	DELETE	678

5.39.3	Custom Operations without associated resources	679

5.39.4	Notifications	679

5.39.4.1	General	679

5.39.5	Data Model	681

5.39.5.1	General	681

5.39.5.2	Structured data types	681

5.39.5.2.1	Introduction	681

5.39.5.2.2	Type: PlannedFlightPaths	681

5.39.5.2.3	Type: SingleFlightPath	682

5.39.5.2.4	Type: FlightPathInfo	682

5.39.5.2.5	Type: UAVFlightAssistNotif	682

5.39.5.2.6	Type: NotifyResponse	682

5.39.5.3	Simple data types and enumerations	682

5.39.5.3.1	Introduction	682

5.39.5.3.2	Simple data types	683

5.39.5.3.3	Enumeration: UavStatus	683

5.39.5.3.4	Enumeration: RequestPurpose	683

5.39.6	Used Features	683

5.39.7	Error handling	683

5.39.7.1	General	683

5.39.7.2	Protocol Errors	683

5.39.7.3	Application Errors	683

5.40	CagInfoParamProvision API	684

5.40.1	Introduction	684

5.40.2	Resources	684

5.40.2.1	Overview	684

5.40.2.2	Resource: CAG Information Parameters Provisionings	685

5.40.2.2.1	Introduction	685

5.40.2.2.2	Resource Definition	685

5.40.2.2.3	Resource Methods	685

5.40.2.2.3.1	GET	685

5.40.2.2.3.2	POST	686

5.40.2.2.4	Resource Custom Operations	687

5.40.2.3	Resource: Individual CAG Information Parameters Provisioning	687

5.40.2.3.1	Introduction	687

5.40.2.3.2	Resource Definition	687

5.40.2.3.3	Resource Methods	688

5.40.2.3.3.1	GET	688

5.40.2.3.3.2	PUT	689

5.40.2.3.3.3	PATCH	690

5.40.2.3.3.4	DELETE	691

5.40.2.3.4	Resource Custom Operations	692

5.40.3	Custom Operations without associated resources	692

5.40.4	Notifications	692

5.40.5	Data Model	692

5.40.5.1	General	692

5.40.5.2	Structured data types	693

5.40.5.2.1	Introduction	693

5.40.5.2.2	Type: CagInfoPpData	693

5.40.5.2.3	Type: CagInfoPpDataPatch	693

5.38.5.2.4	Type: CagInformation	693

5.40.5.3	Simple data types and enumerations	693

5.40.5.3.1	Introduction	693

5.40.5.3.2	Simple data types	693

5.40.6	Used Features	694

5.40.7	Error handling	694

5.40.7.1	General	694

5.40.7.2	Protocol Errors	694

5.40.7.3	Application Errors	694

5.41	RetrieveInfoUAVFlight API	694

5.41.1	Introduction	694

5.41.2	Resources	695

5.41.3	Custom Operations without associated resources	695

5.41.3.1	Overview	695

5.41.3.2	Operation: Retrieve	695

5.41.3.2.1	Description	695

5.41.3.2.2	Operation Definition	696

5.41.4	Notifications	696

5.41.5	Data Model	696

5.41.5.1	General	696

5.41.5.2	Structured data types	697

5.41.5.2.1	Introduction	697

5.41.5.2.2	Type: RetrieveUAVFlightReq	697

5.41.5.2.3	CandidateFlightPathInfo	698

5.41.5.2.4	Type: RetrievedUAVFlightInfo	698

5.41.5.3	Simple data types and enumerations	698

5.41.5.3.1	Introduction	698

5.41.5.3.2	Simple data types	698

5.41.5.3.3	Enumeration: RetrievePurpose	698

5.41.5.4	Data types describing alternative data types or combinations of data types	698

5.41.6	Used Features	699

5.41.7	Error handling	699

5.41.7.1	General	699

5.41.7.2	Protocol Errors	699

5.41.7.3	Application Errors	699

5.42	ImsSessionManagement API	699

5.42.1	Introduction	699

5.42.2	Resources	700

5.42.2.1	Overview	700

5.42.2.2	Resource: IMS Sessions	700

5.42.2.2.1	Description	700

5.42.2.2.2	Resource Definition	700

5.42.2.2.3	Resource Standard Methods	701

5.42.2.3	Resource: Individual IMS Session	701

5.42.2.3.1	Description	701

5.42.2.3.2	Resource Definition	701

5.42.2.3.3	Resource Standard Methods	702

5.42.3	Custom Operations without associated resources	704

5.42.4	Notifications	704

5.42.5	Data Model	704

5.42.5.1	General	704

5.42.6	Used Features	704

5.42.7	Error Handling	705

5.42.7.1	General	705

5.42.7.2	Protocol Errors	705

5.42.7.3	Application Errors	705

5.43	ImsEventExposure API	705

5.43.1	Introduction	705

5.43.2	Resources	705

5.43.2.1	Overview	705

5.43.2.2	Resource: IMS EE Subscriptions	706

5.43.2.2.1	Introduction	706

5.43.2.2.2	Resource definition	707

5.43.2.2.3	Resource Standard Methods	707

5.43.2.2.3.1	POST	707

5.43.2.2.3.2	GET	707

5.43.2.2.4	Resource Custom Operations	708

5.43.2.3	Resource: Individual IMS EE Subscription	709

5.43.2.3.1	Introduction	709

5.43.2.3.2	Resource Definition	709

5.43.2.3.3	Resource Standard Methods	709

5.43.2.3.3.1	GET	709

5.43.2.3.3.2	PUT	710

5.43.2.3.3.3	PATCH	711

5.43.2.3.3.4	DELETE	712

5.43.2.3.4	Resource Custom Operations	713

5.43.3	Custom Operations without associated resources	713

5.43.4	Notifications	713

5.43.5	Data Model	715

5.43.5.1	General	715

5.43.5.2	Structured data types	715

5.43.5.2.1	Introduction	715

5.43.5.2.2	Type: IMSEESubsc	716

5.43.5.2.3	Type: IMSEENotif	716

5.43.5.3	Simple data types and enumerations	716

5.43.5.3.1	Introduction	716

5.43.5.3.2	Simple data types	716

5.43.5.4	Data types describing alternative data types or combinations of data types	717

5.43.6	Used Features	717

5.43.7	Error handling	717

5.43.7.1	General	717

5.43.7.2	Protocol Errors	717

5.43.7.3	Application Errors	717

6	Security	717

7	Using Common API Framework	718

7.1	General	718

7.2	Security	718

Annex A (normative):	OpenAPI representation for NEF Northbound APIs	719

A.1	General	719

A.2	TrafficInfluence API	719

A.3	NiddConfigurationTrigger API	731

A.4	AnalyticsExposure API	733

A.5	5GLANParameterProvision API	750

A.6	ApplyingBdtPolicy API	759

A.7	IPTVConfiguration API	763

A.8	LpiParameterProvision API	769

A.9	ServiceParameter API	774

A.10	ACSParameterProvision API	788

A.11	MoLcsNotify API	793

A.12	AKMA API	794

A.13	TimeSyncExposure API	797

A.14	EcsAddressProvision API	812

A.15	AMPolicyAuthorization API	816

A.16	AMInfluence API	823

A.17	MBSTMGI API	829

A.18	MBSSession API	833

A.19	EASDeployment API	843

A.20	ASTI API	849

A.21	DataReporting API	856

A.22	DataReportingProvisioning API	859

A.23	UEId API	865

A.24	MBSUserService API	872

A.25	MBSUserDataIngestSession API	876

A.26	MSEventExposure API	885

A.27	MBSGroupMsgDelivery API	890

A.28	DNAIMapping API	895

A.29	PDTQPolicyNegotiation API	899

A.30	MemberUESelectionAssistance API	904

A.31	GroupParametersProvisioning API	915

A.32	SliceParamProvision API	921

A.33	UEAddress API	925

A.34	ECSAddress API	930

A.35	RSLPPIParametersProvisioning API	936

A.36	AddressingParamProvision API	941

A.37	UAVFlightAssistance API	946

A.38	CagInfoParamProvision API	951

A.39	RetrieveInfoUAVFlight API	956

A.40	ImsSessionManagement API	958

A.41	ImsEventExposure API	958

Annex B (informative):	Change history	965