3GPP TSG-SA3 Meeting #120	S3-250349

# Athens, GR, 17 -21 February 2025

Source:	NCSC, MITRE-FFRDC, OTD-US, NSA

Title:	Initial Trust in the NF for ACME account creation

Document for:	Approval

Agenda item:	4.18

Spec:	-

Version:	-

Work Item:	ACME_SBA

Comments

This TDoc suggests a mechanism for establishing initial trust. The text is intended to be include in the draftCR created for the ACME_SBA WID.

In this proposed change [XX] is the ACME RFC reference inherited from the draftCR.

The ACME mechanism for establishing initial trust via an external authorizing entity when creating a new ACME account is External Account Binding. In this instance the external authorizer is the OAM software. The mechanism is comparable with the IAK mechanism of RFC 4210 form CMPv2 adopted in TS 33.310 using OAM generated symmetric keys issued to the NF to create a MAC, and shared out-of-band with the CA.

Proposed Changes

* * * First Change * * * *

## YY.2 	Initial trust

### YY.2.1 Overview

Initial trust in the NF by the CA should be established prior to or during the account creation stage. This prevents unauthorized entities from creating accounts and attempting to request certificates. Failure to do this could lead to resource exhaustion and to increasing the attack surface for obtaining mis-issued certificates.

The mechanism for establishing initial trust uses external account binding as specified in section 7.3.4 of [XX]. The operator’s OAM software provisions a symmetric MAC key to each NF it creates and securely communicates this to the CA along with the NF Instance IdThe details of this secure channel are outside the scope of this document. This is functionally equivalent to the IAK scheme in clause 10.

Note:	Only symmetric MAC authentication is supported in [XX]. This requires the secure communication of information for each NF created. It may be desirable to develop alternative authentication mechanisms; a certificate-based signing scheme would remove the need to maintain a secure communications channel and reduce the OAM to CA communication to the exchange of root certificates at the configuration stage of the CA or OAM.

### YY.2.2 Details

To enforce the use of external account binding, the ACME server should ensure the "externalAccountBinding" field is defined in the "directory" resource and it is given the value of "true".

When the OAM creates a new NF, initial trust is established as follows

- The OAM creates a unique HS256 MAC key for the NF

- The OAM communicates this key in base64-url encoded form and the NF Instance Id to the ACME server

- The ACME server uses the NF Instance Id as the key identifier for the MAC key

- The OAM provisions the MAC key to the NF

- The NF sends a "newAccount" message using the external account binding mechanism in section 7.3.4 of [XX] where the “signature” field contains the MAC value computed with the MAC key and the "kid" field contains the NF Instance Id

Note 1: The ACME server can use the NF Instance Id to limit the number of accounts associated to the NF Instance.

Note 2: The OAM does not communicate the NF Type in this step. It is part of the claim attested to in the Token Authority.

* * * End of Changes * * * *