ETSI STC SMG3 WPA GPRS Ad-Hoc	Tdoc N1-99407

Oslo, Norway

18-21 May, 1999

| CHANGE REQUEST No : |  |  |  |  | A118r1 |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 03.60 |  | Version: |  |  | 6.3.1 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  |  |  |
| list SMG plenary meeting no. here |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Motorola | Date: | 14 May, 1999 |
| --- | --- | --- | --- |

| Subject: | Information storage |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | X |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
|  |  |  |  | UMTS |  |
| marked with an X) |  |  |  |  |  |

| Reason for change: | 03.60 v6.3.1 refers to the SGSN information storage field "GGSN Address". This field was removed and replaced by "GGSN Address In Use". The reference to the removed field is proposed deleted. The "GGSN Address In Use" field is adequately covered in e.g., subclause 13.2 and Annex A. Furthermore, Access Point Name was missing in some of the tables in the Information Storage clause. The tables have also been amended to reflect that the APN in use may not be the same as the APN requested or subscribed. |
| --- | --- |

| Clauses affected: | 9.2, 13.2, 13.3, and 13.4. |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 9.2	PDP Context Activation, Modification, and Deactivation Functions

These functions are only meaningful at the NSS level and in the MS, and do not directly involve the BSS. An MS in STANDBY or READY state can initiate these functions at any time to activate or deactivate a PDP context in the MS, the SGSN, and the GGSN. A GGSN may request the activation of a PDP context to a GPRS-attached subscriber. A GGSN may initiate the deactivation of a PDP context.

Upon receiving an Activate PDP Context Request message, the SGSN shall initiate procedures to set up PDP contexts.

Upon receiving a Deactivate PDP Context Request message, the SGSN shall initiate procedures to deactivate PDP contexts.

An MS does not have to receive the (De-)Activate PDP Context Accept message before issuing another (De-)Activate PDP Context Request. However, only one request can be outstanding for every NSAPI.

## 13.2	SGSN

SGSN maintains MM context and PDP context information for MSs in STANDBY and READY states. Table 1 shows the context fields for one MS.

Table 1: SGSN MM and PDP Contexts

| Field | Description |
| --- | --- |
| IMSI | IMSI is the main reference key. |
| MM State | Mobility management state, IDLE, STANDBY, or READY. |
| P-TMSI | Packet Temporary Mobile Subscriber Identity. |
| IMEI | International Mobile Equipment Identity |
| P-TMSI Signature | A signature used for identification checking purposes. |
| Routeing Area | Current routeing area. |
| Cell Identity | Current cell in READY state, last known cell in STANDBY or IDLE state. |
| Cell Identity Age | Time elapsed since the last LLC PDU was received from the MS at the SGSN. |
| VLR Number | The VLR number of the MSC/VLR currently serving this MS. |
| New SGSN Address | The IP address of the new SGSN where buffered and not sent N-PDUs should be forwarded to. |
| Authentication Triplets | Authentication and ciphering parameters. |
| Kc | Currently used ciphering key. |
| CKSN | Ciphering key sequence number of Kc. |
| Ciphering algorithm | Selected ciphering algorithm. |
| Radio Access Classmark | MS radio access capabilities. |
| SGSN Classmark | MS network capabilities. |
| DRX Parameters | Discontinuous reception parameters. |
| MNRG | Indicates whether activity from the MS shall be reported to the HLR. |
| NGAF | Indicates whether activity from the MS shall be reported to the MSC/VLR. |
| PPF | Indicates whether paging for GPRS and non-GPRS services can be initiated. |
| MSISDN | The basic MSISDN of the MS. |
| SMS Parameters | SMS-related parameters, e.g., operator-determined barring. |
| Recovery | Indicates if HLR or VLR is performing database recovery. |
| Radio Priority SMS | The RLC/MAC radio priority level for uplink SMS transmission. |
| Each MM context contains zero or more of the following PDP contexts: |  |
| PDP Context Identifier | Index of the PDP context. |
| PDP Type | PDP type, e.g., X.25 or IP. |
| PDP Address | PDP address, e.g., an X.121 address. |
| NSAPI | Network layer Service Access Point Identifier. |
| TI | Transaction Identifier. |
| PDP State | Packet data protocol state, INACTIVE or ACTIVE. |
| APN Subscribed | The APN received from the HLR. |
| APN in Use | The APN currently used. |
| GGSN Address in Use | The IP address of the GGSN currently used. |
| VPLMN Address Allowed | Specifies whether the MS is allowed to use the APN in the domain of the HPLMN only, or additionally the APN in the domain of the VPLMN. |
| QoS Profile Subscribed | The quality of service profile subscribed. |
| QoS Profile Requested | The quality of service profile requested. |
| QoS Profile Negotiated | The quality of service profile negotiated. |
| Radio Priority | The RLC/MAC radio priority level for uplink user data transmission. |
| SND | GTP sequence number of the next downlink N-PDU to be sent to the MS. |
| SNU | GTP sequence number of the next uplink N-PDU to be sent to the GGSN. |
| Charging Id | Charging identifier, identifies charging records generated by SGSN and GGSN. |
| Reordering Required | Specifies whether the SGSN shall reorder N-PDUs before delivering the N-PDUs to the MS. |

In case of anonymous access the SGSN maintains the MM context and PDP context information for MSs in READY state. Table 2 shows the context fields for one MS.

Table 2: SGSN MM and PDP Contexts for Anonymous Access

| Field | Description |
| --- | --- |
| A-TLLI | Auxiliary Temporary Logical Link Identity. |
| AA-TID | Anonymous Access Tunnel Identifier. |
| Routeing Area | Current routeing area. |
| Cell Identity | Current cell. |
| PDP Type | PDP type, e.g., X.25 or IP. |
| PDP Address | PDP address, e.g., an X.121 address. |
| NSAPI | Network layer Service Access Point Identifier. |
| TI | Transaction Identifier. |
| APN in Use | The APN currently used. |
| QoS Profile Negotiated | The quality of service profile negotiated. |
| QoS Profile Requested | The quality of service profile requested. |
| GGSN Address in Use | The IP address of the GGSN currently used. |
| Radio Priority | The RLC/MAC radio priority level for uplink user data transmission. |
| SND | GTP sequence number of the next downlink N-PDU to be sent to the MS. |
| SNU | GTP sequence number of the next uplink N-PDU to be sent to the GGSN. |
| Charging Id | Charging identifier, identifies charging records generated by SGSN and GGSN. |
| Reordering Required | Specifies whether the SGSN shall reorder N-PDUs before delivering the N-PDUs to the MS. |

## 13.3	GGSN

GGSN maintains activated PDP contexts. Table 3 shows the PDP context fields for one PDP Address.

Table 3: GGSN PDP Context

| Field | Description |
| --- | --- |
| IMSI | International Mobile Subscriber Identity. |
| NSAPI | Network layer Service Access Point Identifier. |
| PDP Type | PDP type, e.g., X.25 or IP. |
| PDP Address | PDP address, e.g., an X.121 address. |
| Dynamic Address | Indicates whether PDP Address is static or dynamic. |
| QoS Profile Negotiated | The quality of service profile negotiated. |
| SGSN Address | The IP address of the SGSN currently serving this MS. |
| APN in Use | The APN Network Identifier currently used. |
| MNRG | Indicates whether the MS is marked as not reachable for GPRS at the HLR. |
| Recovery | Indicates if the SGSN is performing database recovery. |
| SND | GTP sequence number of the next downlink N-PDU to be sent to the MS. |
| SNU | GTP sequence number of the next uplink N-PDU to be received from the SGSN. |
| Charging Id | Charging identifier, identifies charging records generated by SGSN and GGSN. |
| Reordering Required | Specifies whether the GGSN shall reorder N-PDUs received from the SGSN. |

If a PDP context is enabled for network-requested PDP context activation, then IMSI, PDP Type, PDP Address, SGSN Address and MNRG contain valid information also when the PDP context is inactive and when the MS is GPRS-detached.

In case of anonymous access the GGSN maintains activated PDP contexts. Table 4 shows the PDP context fields for one MS.

Table 4: GGSN PDP Context for Anonymous Access

| Field | Description |
| --- | --- |
| AA-TID | Anonymous Access Tunnel Identifier. |
| PDP Type | PDP type, e.g., X.25 or IP. |
| PDP Address | PDP address, e.g., an X.121 address. |
| QoS Profile Negotiated | The quality of service profile negotiated. |
| SGSN Address | The IP address of the SGSN serving this MS. |
| APN in Use | The APN Network Identifier currently used. |
| SND | GTP sequence number of the next downlink N-PDU to be sent to the MS. |
| SNU | GTP sequence number of the next uplink N-PDU to be sent to the GGSN. |
| Charging Id | Charging identifier, identifies charging records generated by SGSN and GGSN. |
| Reordering Required | Specifies whether the GGSN shall reorder N-PDUs received from the SGSN. |

A GGSN that supports anonymous access shall have a list of server addresses that are allowed to be accessed by anonymous MSs. The method to maintain the list of the servers is outside the scope of the present document.

## 13.4	MS

Each GPRS MS maintains MM and PDP context information in IDLE, STANDBY and READY states. The information may be contained in the MS and the TE. Table 5 shows the MS context fields.

Table 5: MS MM and PDP Contexts

| Field | SIM | Description |
| --- | --- | --- |
| IMSI | X | International Mobile Subscriber Identity. |
| MM State |  | Mobility management state, IDLE, STANDBY, or READY. |
| P-TMSI | X | Packet Temporary Mobile Subscriber Identity. |
| P-TMSI Signature | X | A signature used for identification checking purposes. |
| Routeing Area | X | Current routeing area. |
| Cell Identity |  | Current cell. |
| Kc | X | Currently used ciphering key. |
| CKSN | X | Ciphering key sequence number of Kc. |
| Ciphering algorithm |  | Selected ciphering algorithm. |
| Classmark |  | MS classmark. |
| DRX Parameters |  | Discontinuous reception parameters. |
| Radio Priority SMS |  | The RLC/MAC radio priority level for uplink SMS transmission. |
| Each MM context contains zero or more of the following PDP contexts: |  |  |
| PDP Type |  | PDP type, e.g., X.25 or IP. |
| PDP Address |  | PDP address, e.g., an X.121 address. |
| PDP State |  | Packet data protocol state, INACTIVE or ACTIVE. |
| Dynamic Address Allowed |  | Specifies whether the MS is allowed to use a dynamic address. |
| NSAPI |  | Network layer Service Access Point Identifier. |
| TI |  | Transaction Identifier. |
| Access Point Name |  | The APN requested. |
| QoS Profile Requested |  | The quality of service profile requested. |
| QoS Profile Negotiated |  | The quality of service profile negotiated. |
| Radio Priority |  | The RLC/MAC radio priority level for uplink user data transmission. |

The information marked with an "X" in Table 5:

-	shall be stored in the SIM if the connected SIM is GPRS-aware; and

-	may be stored in the ME after GPRS detach if the connected SIM is not GPRS-aware.

If the SIM is GPRS-aware, then the IMSI, P-TMSI, P-TMSI Signature, Routeing Area, Kc, and CKSN stored in the SIM shall be used when accessing the GPRS services.

If the SIM is not GPRS-aware, then the P-TMSI, P-TMSI Signature, Routeing Area, Kc, and CKSN stored in the ME shall be used if and only if the IMSI stored in the SIM is identical to the IMSI image maintained in the ME. If the IMSI stored in the SIM is different from the IMSI image in the ME, then the IMSI image in the ME shall not be used, and the MS shall identify itself with the IMSI stored in the SIM when performing a GPRS attach. IMSI, P-TMSI, P-TMSI Signature, Routeing Area, Kc, and CKSN may be stored in the ME after the GPRS attach has been successfully performed.

For anonymous access each GPRS MS maintains MM and PDP context information in READY state. The information may be contained in the ME and the TE. Table 6 shows the MS context fields.

Table 6: MS MM and PDP Contexts for Anonymous Access

| Field | Description |
| --- | --- |
| A-TLLI | Auxiliary Temporary Logical Link Identity. |
| Routeing Area | Current routeing area. |
| Cell Identity | Current cell. |
| PDP Type | PDP type, e.g., X.25 or IP. |
| PDP Address | PDP address, e.g., an X.121 address. |
| NSAPI | Network layer Service Access Point Identifier. |
| TI | Transaction Identifier. |
| Access Point Name | The APN requested. |
| QoS Profile Requested | The quality of service profile requested. |
| QoS Profile Negotiated | The quality of service profile negotiated. |
| Radio Priority | The RLC/MAC radio priority level for uplink user data transmission. |