ETSI SMG3 WPA GPRS ad-hoc	Tdoc SMG3 N1-99421

Oslo, Norway

May 18-21, 1999

| CHANGE REQUEST No : |  |  |  |  |  | Axxx | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 04.08 |  | Version |  | 6.3.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Ericsson | Date: | 14-05-1999 |
| --- | --- | --- | --- |

| Subject: | Correction to the handling of subscribed QoS values. |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | X |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | This CR modifies the QoS IE to be a type 4 IE in order to allow future, backword compatible extensions of the IE. In addition the validity of the values ‘subscribed QoS’ in the QoS IE is clarified – these values are valid in the MS to network direction only. Unused point codes are sent to reserved. The indication of the length parameter is added to the Format field in the QoS IE in the SM messages in sections 9.5.1, 9.5.2, 9.5.6, 9.5.10, 9.5.11. |
| --- | --- |

| Clauses affected: | 10.5.6.5, 9.5.1, 9.5.2, 9.5.6, 9.5.10, 9.5.11 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: | Section 10.5 from GSM 04.08 and section 11.2.2 from GSM 04.07 are attached to this CR. |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

*** First modification ***

#### 10.5.6.5	Quality of service

The purpose of the quality of service information element is to specify the QoS parameters for a PDP context.

The quality of service is a type 4  information element with a length of 5 octets .

The quality of service information element is coded as shown in figure 10.5.138/GSM 04.08 and table 10.5.156/GSM 04.08.

|  | 8 7 6 5 4 3 2 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Quality of service IEI |  |  |  |  | octet 1 |
|  | Length of quality of service IE |  |  |  |  | octet 2 |
|  | 0 0 spare | Delay class |  |  | Reliability class | octet 3 |
|  | Peak throughput |  |  | 0 spare | Precedence class | octet 4 |
|  | 0 0 0 spare |  | Mean throughput |  |  | octet 5 |

Figure 10.5.138/GSM 04.08: Quality of service information element

Table 10.5.156/GSM 04.08: Quality of service information element

Reliability class, octet 3 (see GSM 03.60)
Bits
3 2 1
In MS to network direction:
0 0 0	Subscribed reliability class
In network to MS direction:
0 0 0	Reserved
In MS to network direction and in network to MS direction :
0 0 1	Acknowledged GTP, LLC, and RLC; Protected data
0 1 0	Unacknowledged GTP; Acknowledged LLC and RLC, Protected data
0 1 1	Unacknowledged GTP and LLC; Acknowledged RLC, Protected data
1 0 0	Unacknowledged GTP, LLC, and RLC, Protected data
1 0 1	Unacknowledged GTP, LLC, and RLC, Unprotected data
1 1 1        Reserved

All other values are interpreted as Unacknowledged GTP and LLC; Acknowledged RLC, Protected data  in this version of the protocol.

Delay class, octet 3 (see GSM 02.60 and GSM 03.60)
Bits
6 5 4
In MS to network direction:
0 0 0	Subscribed delay class
In network to MS direction:
0 0 0	Reserved
In MS to network direction and in network to MS direction :
0 0 1	Delay class 1
0 1 0	Delay class 2
0 1 1	Delay class 3
1 0 0	Delay class 4 (best effort)
1 1 1        Reserved
 
All other values are interpreted as Delay class 4 (best effort) in this version 
of the protocol.

Bit 7 and 8 of octet 3 are spare and shall be coded all 0.

Precedence class, octet 4 (see GSM 03.60)
Bits
3 2 1
In MS to network direction:
0 0 0	Subscribed precedence
In network to MS direction:
0 0 0	Reserved
In MS to network direction and in network to MS direction :
0 0 1	High priority
0 1 0	Normal priority
0 1 1	Low priority
1 1 1        Reserved

All other values are interpreted as Normal priority  in this version of the protocol.

Bit 4 of octet 4 is spare and shall be coded as 0.

Peak throughput, octet 4
Bits
8 7 6 5
In MS to network direction:
0 0 0 0	Subscribed peak throughput
In network to MS direction:
0 0 0 0	Reserved
In MS to network direction and in network to MS direction :
0 0 0 1	Up to 1 000 octet/s
0 0 1 0	Up to 2 000 octet/s
0 0 1 1	Up to 4 000 octet/s
0 1 0 0	Up to 8 000 octet/s
0 1 0 1	Up to 16 000 octet/s
0 1 1 0	Up to 32 000 octet/s
0 1 1 1	Up to 64 000 octet/s
1 0 0 0	Up to 128 000 octet/s
1 0 0 1	Up to 256 000 octet/s
1 1 1 1 Reserved

All other values are interpreted as Up to 1 000 octet/s  in this 
version of the protocol.

Mean throughput, octet 5
Bits
5 4 3 2 1
In MS to network direction:
0 0 0 0 0		Subscribed mean throughput
In network to MS direction:
0 0 0 0 0		Reserved
In MS to network direction and in network to MS direction :
0 0 0 0 1		100 octet/h
0 0 0 1 0		200 octet/h
0 0 0 1 1		500 octet/h
0 0 1 0 0		1 000 octet/h
0 0 1 0 1		2 000 octet/h
0 0 1 1 0		5 000 octet/h
0 0 1 1 1		10 000 octet/h
0 1 0 0 0		20 000 octet/h
0 1 0 0 1		50 000 octet/h
0 1 0 1 0		100 000 octet/h
0 1 0 1 1		200 000 octet/h
0 1 1 0 0		500 000 octet/h
0 1 1 0 1		1 000 000 octet/h
0 1 1 1 0		2 000 000 octet/h
0 1 1 1 1		5 000 000 octet/h
1 0 0 0 0		10 000 000 octet/h
1 0 0 0 1		20 000 000 octet/h
1 0 0 1 0		50 000 000 octet/h
1 1 1 1 0		Reserved
1 1 1 1 1		Best effort

The value Best effort indicates that throughput shall be made available to the MS on a per need and availability basis.

All other values are interpreted as Best effort  in this 
version of the protocol.

Bits 8 to 6 of octet 5 are spare and shall be coded all 0.

*** Next modification ***

### 9.5.1	Activate PDP context request

This message is sent by the MS to the network to request activation of a PDP context.	
See table 9.5.1/GSM 04.08.

Message type:	activate PDP context request

Significance:		global

Direction:			MS to network

Table 9.5.1/GSM 04.08: Activate PDP context request message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 10.2 | M | V | 1/2 |
|  | Transaction identifier | Transaction identifier 10.3.2 | M | V | 1/2 |
|  | Activate PDP context request message identity | Message type 10.4 | M | V | 1 |
|  | NSAPI | Network service access point identifier 10.5.6.2 | M | V | 1 |
|  | Requested LLC SAPI | LLC service access point identifier 10.5.6.9 | M | V | 1 |
|  | Requested QoS | Quality of service 10.5.6.5 | M | LV | 4 |
|  | Requested PDP address | Packet data protocol address 10.5.6.4 | M | LV | 3 - 19 |
| 28 | Access point name | Access point name 10.5.6.1 | O | TLV | 3 - 255 |
| 27 | Protocol configuration options | Protocol configuration options 10.5.6.3 | O | TLV | 3 - 255 |

*** Next modification ***

### 9.5.2	Activate PDP context accept

This message is sent by the network to the MS to acknowledge activation of a PDP context.	
See table 9.5.2/GSM 04.08.

Message type:	activate PDP context accept

Significance:		global

Direction:			network to MS

Table 9.5.2/GSM 04.08: Activate PDP context Accept message content

| IEI |  | Information Element |  | Type/Reference |  | Presence |  | Format |  | Length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator |  | Protocol discriminator 10.2 |  | M |  | V |  | 1/2 |  |
|  |  | Transaction identifier |  | Transaction identifier 10.3.2 |  | M |  | V |  | 1/2 |
|  |  | Activate PDP context accept message identity |  | Message type 10.4 |  | M |  | V |  | 1 |
|  |  | Negotiated LLC SAPI |  | LLC service access point identifier 10.5.6.9 |  | M |  | V |  | 1 |
|  |  | Negotiated QoS |  | Quality of service 10.5.6.5 |  | M |  | LV |  | 4 |
|  |  | Radio priority |  | Radio priority 10.5.7.2 |  | M |  | V |  | 1/2 |
|  |  | Spare half octet |  | Spare half octet 10.5.1.8 |  | M |  | V |  | 1/2 |
|  |  | PDP address |  | Packet data protocol address 10.5.6.4 |  | O |  | TLV |  | 4 -20 |
| 27 |  | Protocol configuration options |  | Protocol configuration options 10.5.6.3 |  | O |  | TLV |  | 3 -255 |

*** Next modification ***

### 9.5.6	Modify PDP context request

This message is sent by the network to the MS to request modification of an active PDP context. See table 9.5.6/GSM 04.08.

Message type:	modify PDP context request

Significance:		global

Direction:			network to MS

Table 9.5.6/GSM 04.08: modify PDP context request message content

| IEI |  | Information Element |  | Type/Reference |  | Presence |  | Format |  | Length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protocol discriminator |  | Protocol discriminator 10.2 |  | M |  | V |  | 1/2 |
|  |  | Transaction identifier |  | Transaction identifier 10.3.2 |  | M |  | V |  | 1/2 |
|  |  | Modify PDP context request message identity |  | Message type 10.4 |  | M |  | V |  | 1 |
|  |  | Radio priority |  | Radio priority 10.5.7.2 |  | M |  | V |  | 1/2 |
|  |  | Spare half octet |  | Spare half octet 10.5.1.8 |  | M |  | V |  | 1/2 |
|  | Requested LLC SAPI |  | LLC service access point identifier 10.5.6.9 |  | M |  | V |  | 1 |  |
|  | New QoS |  | Quality of service 10.5.6.5 |  | M |  | LV |  | 4 |  |

*** Next modification ***

### 9.5.10	Activate AA PDP context request

This message is sent by the MS to the network to initiate activation of an AA PDP context.	
See table 9.5.10/GSM 04.08.

Message type:	Activate AA PDP context Request

Significance:		global

Direction:			MS to network

Table 9.5.10/GSM 04.08: Activate AA PDP context request message content

| IEI |  |  | Information Element |  |  | Type/Reference |  | Presence |  |  | Format |  | Length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Protocol discriminator |  |  | Protocol discriminator 10.2 |  | M |  |  | V |  | 1/2 |
|  |  |  | Transaction identifier |  |  | Transaction identifier 10.3.2 |  | M |  |  | V |  | 1/2 |
|  |  |  | Activate AA PDP context request message identity |  |  | Message type 10.4 |  | M |  |  | V |  | 1 |
|  |  |  | NSAPI |  |  | Network service access point identifier 10.5.6.2 |  | M |  |  | V |  | 1 |
|  |  |  | Requested LLC SAPI |  |  | LLC service access point identifier 10.5.6.9 |  | M |  |  | V |  | 1 |
|  |  |  | Requested QoS |  |  | Quality of service 10.5.6. 5 |  | M |  |  | LV |  | 4 |
|  |  | Requested packet data protocol address |  |  | Packet data protocol address 10.5.6.4 |  |  |  | M |  | LV |  | 3 - 18 |
| 28 |  | Access point name |  |  | Access point name 10.5.6.1 |  |  |  | M |  | LV |  | 3 - 255 |
| 27 |  | Protocol configuration options |  |  | Protocol configuration options 10.5.6.3 |  |  |  | M |  | LV |  | 3 - 255 |
| 29 | Requested AA-READY timer value |  |  | GPRS Timer 10.5.7.3 |  |  | O |  |  | TV |  | 2 |  |

*** Next modification ***

### 9.5.11	Activate AA PDP context accept

This message is sent by the network to the MS to acknowledge the activation of an AA PDP context. See table 9.5.11/GSM 04.08.

Message type:	Activate AA PDP context Accept

Significance:		global

Direction:			network to MS

Table 9.5.11/GSM 04.08: Activate AA PDP context Accept message content

| IEI |  | Information Element |  | Type/Reference |  | Presence |  | Format |  | Length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protocol discriminator |  | Protocol discriminator 10.2 |  | M |  | V |  | 1/2 |
|  |  | Transaction identifier |  | Transaction identifier 10.3.2 |  | M |  | V |  | 1/2 |
|  |  | Activate AA PDP context accept message identity |  | Message type 10.4 |  | M |  | V |  | 1 |
|  |  | Negotiated LLC SAPI |  | LLC service access point identifier 10.5.6.9 |  | M |  | V |  | 1 |
|  |  | Negotiated QoS |  | Quality of service 10.5.6. 5 |  | M |  | LV |  | 4 |
|  | P-TMSI |  | Mobile station identity 10.5.7.1 |  | M |  | LV |  | 6 |  |
|  |  | Packet data protocol address |  | Packet data protocol address 10.5.6.4 |  | M |  | LV |  | 3 - 18 |
|  |  | Radio priority |  | Radio priority 10.5.7.2 |  | M |  | V |  | 1/2 |
|  |  | Spare half octet |  | Spare half octet 10.5.1.8 |  | M |  | V |  | 1/2 |
|  | SM cause |  | SM cause 10.5.6.6 |  | M |  | V |  | 1 |  |
| 27 |  | Protocol configuration options |  | Protocol configuration options 10.5.6.3 |  | O |  | TLV |  | 3 - 255 |
| 29 | Negotiated AA-Ready timer value |  | GPRS Timer 10.5.7.3 |  | O |  | TV |  | 2 |  |

### /***************************** 04.08 version 6.3.0 *********************************/

### 10.5	Other information elements

The different formats (V, LV, T, TV, TLV) and the four categories of information elements (type 1, 2, 3, and 4) are defined in GSM 04.07.

The first octet of an information element in the non-imperative part contains the IEI of the information element. If this octet does not correspond to an IEI known in the message (see GSM 04.07), the receiver shall assume that the information element is:

-	if bit 8 of the first octet of the IE has the value 1, the IE is of type 1 or 2, i.e. that it is an information element of one octet length; or

-	if bit 8 of the first octet of the IE has the value 0, the IE is of type 4, i.e. that the next octet is the length indicator indicating the length of the remaining of the information element. If in this case bits 5, 6, and 7 of the first octet of the IE also have the value 0, the IE is encoded as "comprehension required".

NOTE:	The handling of messages containing unknown IEs encoded as "comprehension required" is specified in section 8.

This rule allows the receiver to jump over unknown information elements and to analyse any following information elements.

The information elements which are common for at least two of the three protocols Radio Resources management, Mobility Management and Call Control, are listed in section 10.5.1.

The information elements for the protocols Radio Resources management, Mobility Management and Call Control are listed in sections 10.5.2, 10.5.3 and 10.5.4 respectively. Default information element identifiers are listed in annex K.

NOTE:	Different information elements may have the same default information element identifier if they belong to different protocols.

The descriptions of the information element types in sections 10.5.1, 10.5.2, 10.5.3, and 10.5.4 are organized in alphabetical order of the IE types. Each IE type is described in one subsection.

The subsection may have an introduction:

-	possibly explaining the purpose of the IE;

-	possibly describing whether the IE belongs to type 1, 2, 3, 4 or 5;

-	possibly indicating the length that the information element has if it is either type 5 or if it is used in format TV (type 1 and 3) or TLV (type 4).

A figure of the subsection defines the structure of the IE indicating:

-	possibly the position and length of the IEI. (However it depends on the message in which the IE occurs whether the IE contains an IEI.);

-	the fields the IE value part is composed of;

-	possibly the position and length of the length indicator. (However it depends on the IE type whether the IE contains a length indicator or not.);

-	possibly octet numbers of the octets that compose the IE (see clause a) below).

Finally, the subsection contains tables defining the structure and value range of the fields that compose the IE value part. The order of appearance for information elements in a message is defined in section 9.

The order of the information elements within the imperative part of messages has been chosen so that information elements with 1/2 octet of content (type 1) go together in succession. The first type 1 information element occupies bits 1 to 4 of octet N, the second bits 5 to 8 of octet N, the third bits 1 to 4 of octet N + 1 etc. If the number of type 1 information elements is odd then bits 5 to 8 of the last octet occupied by these information elements contains a spare half octet IE in format V.

Where the description of information elements in this Technical Specification contains bits defined to be "spare bits", these bits shall set to the indicated value (0 or 1) by the sending side, and their value shall be ignored by the receiving side. With few exceptions, spare bits are indicated as being set to "0" in GSM 04.08.

The following rules apply for the coding of type 4 information elements:

a)	The octet number of an octet (which is defined in the figure of a subsection) consists of a positive integer, possibly of an additional letter, and possibly of an additional asterisk, see clause f). The positive integer identifies one octet or a group of octets.

b)	Each octet group is a self contained entity. The internal structure of an octet group may be defined in alternative ways.

c)	An octet group is formed by using some extension mechanism. The preferred extension mechanism is to extend an octet (N) through the next octet(s) (Na, Nb, etc.) by using bit 8 in each octet as an extension bit.

The bit value "0" indicates that the octet group continues through to the next octet. The bit value "1" indicates that this octet is the last octet of the group. If one octet (Nb) is present, the preceding octets (N and Na) shall also be present.

In the format descriptions appearing in section 10.5.1 to 10.5.4, bit 8 is marked "0/1 ext" if another octet follows. Bit 8 is marked "1 ext" if this is the last octet in the extension domain.

Additional octets may be defined in later versions of the protocols ("1 ext" changed to "0/1 ext") and equipments shall be prepared to receive such additional octets; the contents of these octets shall be ignored. However the length indicated in sections 9 and 10 only takes into account this version of the protocols.

d)	In addition to the extension mechanism defined above, an octet (N) may be extended through the next octet(s) (N+1, N+2 etc.) by indications in bits 7-1 (of octet N).

e)	The mechanisms in c) and d) may be combined.

f)	Optional octets are marked with asterisks (*).

### /***************************** 04.07 version 6.2.1 *********************************/

### 11.2.2	Description methods for IE structure

Standard IEs can be further structured in parts called fields. Two description methods are recommended and described hereafter.

#### 11.2.2.1	Tables

According to this description method, the IE is presented in its maximum format, i.e., T, TV or TLV, in a picture representing the bits in a table, each line representing an octet. Bits appear in the occidental order, i.e., from left of the page to right of the page, and from top of the page to bottom of the page.

Boxes so delimited contains typically the field name, possibly an indication of which bits in the field are in the box, and possibly a value (e.g., for spare bits).

A specific method can be used in the IE description to describe a branching structure, i.e., a structure variable according to the value of particular fields in the IE. This design is unusual outside type 4 IEs, and as, a design rule, should be used only in type 4 IEs.

a)	The octet number of an octet within the IE is defined typically in the table. It consists of a positive integer, possibly of an additional letter, and possibly of an additional asterisk, see clause f). The positive integer identifies one octet or a group of octets.

b)	Each octet group is a self contained entity. The internal structure of an octet group may be defined in alternative ways.

c)	An octet group is formed by using some extension mechanism. The preferred extension mechanism is to extend an octet (N) through the next octet(s) (Na, Nb, etc.) by using bit 8 in each octet as an extension bit.

The bit value "0" indicates that the octet group continues through to the next octet. The bit value "1" indicates that this octet is the last octet of the group. If one octet (Nb) is present, the preceding octets (N and Na) shall also be present.

In the format descriptions appearing in section 10.5.1 to 10.5.4, bit 8 is marked "0/1 ext" if another octet follows. Bit 8 is marked "1 ext" if this is the last octet in the extension domain.

Additional octets may be defined in later versions of the protocols ("1 ext" changed to "0/1 ext") and equipments shall be prepared to receive such additional octets; the contents of these octets shall be ignored. However the length indicated in sections 9 and 10 only takes into account this version of the protocols.

d)	In addition to the extension mechanism defined above, an octet (N) may be extended through the next octet(s) (N+1, N+2 etc.) by indications in bits 7-1 (of octet N).

e)	The mechanisms in c) and d) may be combined.

f)	Optional octets are marked with asterisks (*). As a design rule, the presence of absence of an optional octet should be determinable from information in the IE and preceding the optional octet. Care should be taken not to introduce ambiguities with optional octets.

##### 11.2.2.1.1	Compact notation

The compact notation described in Annex B can be used to describe the value part of a standard IE. This method is recommended for complex structures, or for a branching structure not respecting octet boundaries.