ETSI SMG3 WPA GPRS ad-hoc	Tdoc N1-99471

Oslo, Norway								 			(Revision of N1-99421)

May 18-21, 1999

| CHANGE REQUEST No : |  |  |  |  |  | A515r4 | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 04.08 |  | Version |  | 6.3.0 |  |  |  |
| Submitted to SMG | #29 |  | for approval |  | X | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network | X |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | GPRS |
| --- | --- |

| Source: | Ericsson | Date: | 21-05-1999 |
| --- | --- | --- | --- |

| Subject: | Correction to the handling of subscribed QoS values. |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 | X |
| and one release | C Functional modification of feature |  |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | This CR modifies the QoS IE to be a type 4 IE in order to allow future, backword compatible extensions of the IE. In addition the validity of the values ‘subscribed QoS’ in the QoS IE is clarified – these values are valid in the MS to network direction only. Unused point codes are sent to reserved. The indication of the length parameter is added to the Format field in the QoS IE in the SM messages in sections 9.5.1, 9.5.2, 9.5.6, 9.5.10, 9.5.11. |
| --- | --- |

| Clauses affected: | 10.5.6.5, 9.5.1, 9.5.2, 9.5.6, 9.5.10, 9.5.11 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: | Section 10.5 from GSM 04.08 and section 11.2.2 from GSM 04.07 are attached to this CR. |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

*** First modification ***

#### 10.5.6.5	Quality of service

The purpose of the quality of service information element is to specify the QoS parameters for a PDP context.

The quality of service is a type 4  information element with a length of 5 octets .

The quality of service information element is coded as shown in figure 10.5.138/GSM 04.08 and table 10.5.156/GSM 04.08.

|  | 8 7 6 5 4 3 2 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Quality of service IEI |  |  |  |  | octet 1 |
|  | Length of quality of service IE |  |  |  |  | octet 2 |
|  | 0 0 spare | Delay class |  |  | Reliability class | octet 3 |
|  | Peak throughput |  |  | 0 spare | Precedence class | octet 4 |
|  | 0 0 0 spare |  | Mean throughput |  |  | octet 5 |

Figure 10.5.138/GSM 04.08: Quality of service information element

Table 10.5.156/GSM 04.08: Quality of service information element

Reliability class, octet 3 (see GSM 03.60)
Bits
3 2 1
In MS to network direction:
0 0 0	Subscribed reliability class
In network to MS direction:
0 0 0	Reserved
In MS to network direction and in network to MS direction :
0 0 1	Acknowledged GTP, LLC, and RLC; Protected data
0 1 0	Unacknowledged GTP; Acknowledged LLC and RLC, Protected data
0 1 1	Unacknowledged GTP and LLC; Acknowledged RLC, Protected data
1 0 0	Unacknowledged GTP, LLC, and RLC, Protected data
1 0 1	Unacknowledged GTP, LLC, and RLC, Unprotected data
1 1 1        Reserved

All other values are interpreted as Unacknowledged GTP and LLC; Acknowledged RLC, Protected data  in this version of the protocol.

Delay class, octet 3 (see GSM 02.60 and GSM 03.60)
Bits
6 5 4
In MS to network direction:
0 0 0	Subscribed delay class
In network to MS direction:
0 0 0	Reserved
In MS to network direction and in network to MS direction :
0 0 1	Delay class 1
0 1 0	Delay class 2
0 1 1	Delay class 3
1 0 0	Delay class 4 (best effort)
1 1 1        Reserved
 
All other values are interpreted as Delay class 4 (best effort) in this version 
of the protocol.

Bit 7 and 8 of octet 3 are spare and shall be coded all 0.

Precedence class, octet 4 (see GSM 03.60)
Bits
3 2 1
In MS to network direction:
0 0 0	Subscribed precedence
In network to MS direction:
0 0 0	Reserved
In MS to network direction and in network to MS direction :
0 0 1	High priority
0 1 0	Normal priority
0 1 1	Low priority
1 1 1        Reserved

All other values are interpreted as Normal priority  in this version of the protocol.

Bit 4 of octet 4 is spare and shall be coded as 0.

Peak throughput, octet 4
Bits
8 7 6 5
In MS to network direction:
0 0 0 0	Subscribed peak throughput
In network to MS direction:
0 0 0 0	Reserved
In MS to network direction and in network to MS direction :
0 0 0 1	Up to 1 000 octet/s
0 0 1 0	Up to 2 000 octet/s
0 0 1 1	Up to 4 000 octet/s
0 1 0 0	Up to 8 000 octet/s
0 1 0 1	Up to 16 000 octet/s
0 1 1 0	Up to 32 000 octet/s
0 1 1 1	Up to 64 000 octet/s
1 0 0 0	Up to 128 000 octet/s
1 0 0 1	Up to 256 000 octet/s
1 1 1 1 Reserved

All other values are interpreted as Up to 1 000 octet/s  in this 
version of the protocol.

Mean throughput, octet 5
Bits
5 4 3 2 1
In MS to network direction:
0 0 0 0 0		Subscribed mean throughput
In network to MS direction:
0 0 0 0 0		Reserved
In MS to network direction and in network to MS direction :
0 0 0 0 1		100 octet/h
0 0 0 1 0		200 octet/h
0 0 0 1 1		500 octet/h
0 0 1 0 0		1 000 octet/h
0 0 1 0 1		2 000 octet/h
0 0 1 1 0		5 000 octet/h
0 0 1 1 1		10 000 octet/h
0 1 0 0 0		20 000 octet/h
0 1 0 0 1		50 000 octet/h
0 1 0 1 0		100 000 octet/h
0 1 0 1 1		200 000 octet/h
0 1 1 0 0		500 000 octet/h
0 1 1 0 1		1 000 000 octet/h
0 1 1 1 0		2 000 000 octet/h
0 1 1 1 1		5 000 000 octet/h
1 0 0 0 0		10 000 000 octet/h
1 0 0 0 1		20 000 000 octet/h
1 0 0 1 0		50 000 000 octet/h
1 1 1 1 0		Reserved
1 1 1 1 1		Best effort

The value Best effort indicates that throughput shall be made available to the MS on a per need and availability basis.

All other values are interpreted as Best effort  in this 
version of the protocol.

Bits 8 to 6 of octet 5 are spare and shall be coded all 0.

*** Next modification ***

### 9.5.1	Activate PDP context request

This message is sent by the MS to the network to request activation of a PDP context.	
See table 9.5.1/GSM 04.08.

Message type:	activate PDP context request

Significance:		global

Direction:			MS to network

Table 9.5.1/GSM 04.08: Activate PDP context request message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator | Protocol discriminator 10.2 | M | V | 1/2 |
|  | Transaction identifier | Transaction identifier 10.3.2 | M | V | 1/2 |
|  | Activate PDP context request message identity | Message type 10.4 | M | V | 1 |
|  | NSAPI | Network service access point identifier 10.5.6.2 | M | V | 1 |
|  | Requested LLC SAPI | LLC service access point identifier 10.5.6.9 | M | V | 1 |
|  | Requested QoS | Quality of service 10.5.6.5 | M | LV | 4 |
|  | Requested PDP address | Packet data protocol address 10.5.6.4 | M | LV | 3 - 19 |
| 28 | Access point name | Access point name 10.5.6.1 | O | TLV | 3 - 255 |
| 27 | Protocol configuration options | Protocol configuration options 10.5.6.3 | O | TLV | 3 - 255 |

*** Next modification ***

### 9.5.2	Activate PDP context accept

This message is sent by the network to the MS to acknowledge activation of a PDP context.	
See table 9.5.2/GSM 04.08.

Message type:	activate PDP context accept

Significance:		global

Direction:			network to MS

Table 9.5.2/GSM 04.08: Activate PDP context Accept message content

| IEI |  | Information Element |  | Type/Reference |  | Presence |  | Format |  | Length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protocol discriminator |  | Protocol discriminator 10.2 |  | M |  | V |  | 1/2 |  |
|  |  | Transaction identifier |  | Transaction identifier 10.3.2 |  | M |  | V |  | 1/2 |
|  |  | Activate PDP context accept message identity |  | Message type 10.4 |  | M |  | V |  | 1 |
|  |  | Negotiated LLC SAPI |  | LLC service access point identifier 10.5.6.9 |  | M |  | V |  | 1 |
|  |  | Negotiated QoS |  | Quality of service 10.5.6.5 |  | M |  | LV |  | 4 |
|  |  | Radio priority |  | Radio priority 10.5.7.2 |  | M |  | V |  | 1/2 |
|  |  | Spare half octet |  | Spare half octet 10.5.1.8 |  | M |  | V |  | 1/2 |
|  |  | PDP address |  | Packet data protocol address 10.5.6.4 |  | O |  | TLV |  | 4 -20 |
| 27 |  | Protocol configuration options |  | Protocol configuration options 10.5.6.3 |  | O |  | TLV |  | 3 -255 |

*** Next modification ***

### 9.5.6	Modify PDP context request

This message is sent by the network to the MS to request modification of an active PDP context. See table 9.5.6/GSM 04.08.

Message type:	modify PDP context request

Significance:		global

Direction:			network to MS

Table 9.5.6/GSM 04.08: modify PDP context request message content

| IEI |  | Information Element |  | Type/Reference |  | Presence |  | Format |  | Length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protocol discriminator |  | Protocol discriminator 10.2 |  | M |  | V |  | 1/2 |
|  |  | Transaction identifier |  | Transaction identifier 10.3.2 |  | M |  | V |  | 1/2 |
|  |  | Modify PDP context request message identity |  | Message type 10.4 |  | M |  | V |  | 1 |
|  |  | Radio priority |  | Radio priority 10.5.7.2 |  | M |  | V |  | 1/2 |
|  |  | Spare half octet |  | Spare half octet 10.5.1.8 |  | M |  | V |  | 1/2 |
|  | Requested LLC SAPI |  | LLC service access point identifier 10.5.6.9 |  | M |  | V |  | 1 |  |
|  | New QoS |  | Quality of service 10.5.6.5 |  | M |  | LV |  | 4 |  |

*** Next modification ***

### 9.5.10	Activate AA PDP context request

This message is sent by the MS to the network to initiate activation of an AA PDP context.	
See table 9.5.10/GSM 04.08.

Message type:	Activate AA PDP context Request

Significance:		global

Direction:			MS to network

Table 9.5.10/GSM 04.08: Activate AA PDP context request message content

| IEI |  |  | Information Element |  |  | Type/Reference |  | Presence |  |  | Format |  | Length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Protocol discriminator |  |  | Protocol discriminator 10.2 |  | M |  |  | V |  | 1/2 |
|  |  |  | Transaction identifier |  |  | Transaction identifier 10.3.2 |  | M |  |  | V |  | 1/2 |
|  |  |  | Activate AA PDP context request message identity |  |  | Message type 10.4 |  | M |  |  | V |  | 1 |
|  |  |  | NSAPI |  |  | Network service access point identifier 10.5.6.2 |  | M |  |  | V |  | 1 |
|  |  |  | Requested LLC SAPI |  |  | LLC service access point identifier 10.5.6.9 |  | M |  |  | V |  | 1 |
|  |  |  | Requested QoS |  |  | Quality of service 10.5.6. 5 |  | M |  |  | LV |  | 4 |
|  |  | Requested packet data protocol address |  |  | Packet data protocol address 10.5.6.4 |  |  |  | M |  | LV |  | 3 - 18 |
| 28 |  | Access point name |  |  | Access point name 10.5.6.1 |  |  |  | M |  | LV |  | 3 - 255 |
| 27 |  | Protocol configuration options |  |  | Protocol configuration options 10.5.6.3 |  |  |  | M |  | LV |  | 3 - 255 |
| 29 | Requested AA-READY timer value |  |  | GPRS Timer 10.5.7.3 |  |  | O |  |  | TV |  | 2 |  |

*** Next modification ***

### 9.5.11	Activate AA PDP context accept

This message is sent by the network to the MS to acknowledge the activation of an AA PDP context. See table 9.5.11/GSM 04.08.

Message type:	Activate AA PDP context Accept

Significance:		global

Direction:			network to MS

Table 9.5.11/GSM 04.08: Activate AA PDP context Accept message content

| IEI |  | Information Element |  | Type/Reference |  | Presence |  | Format |  | Length |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protocol discriminator |  | Protocol discriminator 10.2 |  | M |  | V |  | 1/2 |
|  |  | Transaction identifier |  | Transaction identifier 10.3.2 |  | M |  | V |  | 1/2 |
|  |  | Activate AA PDP context accept message identity |  | Message type 10.4 |  | M |  | V |  | 1 |
|  |  | Negotiated LLC SAPI |  | LLC service access point identifier 10.5.6.9 |  | M |  | V |  | 1 |
|  |  | Negotiated QoS |  | Quality of service 10.5.6. 5 |  | M |  | LV |  | 4 |
|  | P-TMSI |  | Mobile station identity 10.5.7.1 |  | M |  | LV |  | 6 |  |
|  |  | Packet data protocol address |  | Packet data protocol address 10.5.6.4 |  | M |  | LV |  | 3 - 18 |
|  |  | Radio priority |  | Radio priority 10.5.7.2 |  | M |  | V |  | 1/2 |
|  |  | Spare half octet |  | Spare half octet 10.5.1.8 |  | M |  | V |  | 1/2 |
|  | SM cause |  | SM cause 10.5.6.6 |  | M |  | V |  | 1 |  |
| 27 |  | Protocol configuration options |  | Protocol configuration options 10.5.6.3 |  | O |  | TLV |  | 3 - 255 |
| 29 | Negotiated AA-Ready timer value |  | GPRS Timer 10.5.7.3 |  | O |  | TV |  | 2 |  |